| 3GPP TR 38.852 V17.3.0 (2023-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Regulatory background	8

4.1	EC Decision 2021/1703	8

4.2	ECC Decision(20)02	8

4.3	Technical conditions	8

4.3.1	Base stations using wideband technologies	8

4.3.2	Terminals other than cab-radios using wideband technologies	9

5	Frequency band arrangement	9

6	NR system parameters	9

7	RF requirements	10

7.1	BS specific requirements	10

7.1.1	General	10

7.1.2	Transmitter characteristics	10

7.1.2.1	BS maximum output power	10

7.1.2.2	Unwanted emissions	11

7.1.2.2.1	General	11

7.1.2.2.2	Tx spurious emissions	11

7.1.3	Receiver characteristics	11

7.1.3.2	Rx blocking	11

7.2	UE specific requirements	11

7.2.1	Transmitter characteristics	11

7.2.2	Receiver characteristics	12

8	Performance requirements	14

9	Deployment aspects	14

10	Conclusion	14

Annex A (informative): Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report deals with the use of the Rail Mobile Radio spectrum in the 1900MHz frequency band, which was assigned by the ECC Decision (20)02 [1] for the use by the railways in Europe.The purpose is to gather the relevant background information and studies in order to address all the necessary precautions to make the unpaired spectrum of 1900-1910MHz usable for 5G NR.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ECC Decision (20)02: "Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) ".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	RP‑211496	Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR), WID

[4]	3GPP TS 38.104: " NR; Base Station (BS) radio transmission and reception".

[5]	European Commission implementing Decision (EU) 2021/1730 on the harmonised use of the paired frequency bands 874,4-880,0 MHz and 919,4-925,0 MHz and of the unpaired frequency band 1 900-1 910 MHz for Railway Mobile Radio, 28 September 2021.

[6]	ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWChannel	BS channel bandwidth

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

PREFSENS	Conducted Reference Sensitivity power level

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS	Advanced Antenna Systems

ACLR	Adjacent Channel Leakage Ratio

ARFCN	Absolute Radio Frequency Channel Number

BS	Base Station

BW	Bandwidth

CEPT	Conférence Européenne des administrations des Postes et Télécommunications

EC	European Commission

ECC	Electronic Communications Committee

EIRP	Effective Isotropic Radiated Power

GSCN	Global Synchronization Channel Number

HST	High Speed Train

MFCN	Mobile/Fixed Communications Networks

NR	New Radio

REFSENS	Reference Sensitivity

RF	Radio Frequency

RMR	Rail Mobile Radio

SCS	Sub-Carrier Spacing

SS	Synchronization Signal

TDD	Time Division Duplex

# 4	Regulatory background

## 4.1	EC Decision 2021/1703

On September 28th 2021, the European Commission Decision 2021/1703 [5] establishes the harmonised conditions for the availability and efficient use of radio spectrum for the Railway Mobile Radio (RMR) in the band 1900 – 1910 MHz. By 1 January 2022, Member States shall designate and make available on a non-exclusive basis the unpaired frequency band 1900 – 1910 MHz for Railway Mobile Radio, in accordance with the technical conditions listed in subclause 4.3.

## 4.2	ECC Decision(20)02

The ECC Decision (20)02 [1] addresses the designation of the unpaired frequency band 1900-1910 MHz to be used for Railway Mobile Radio (RMR) on a CEPT wide basis.

By this Decision, the CEPT administrations shall designate the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) on a non-exclusive basis, according to the technical conditions listed in subclause 4.3.

This Decision entered into force on November 20th, 2020.

## 4.3	Technical conditions

### 4.3.1	Base stations using wideband technologies

Only non AAS Base Stations are considered.

Table 4.3.1-1: General in-block requirement mandatory for uncoordinated deployment

| RMR channel BW | Maximum EIRP |
| --- | --- |
| 10 MHz | 65 dBm/10 MHz (NOTE) |
| NOTE: In case an administration wishes to allow higher e.i.r.p., coordination or other mitigation measures are required. |  |

Table 4.3.1-2: Baseline requirement

| Frequency Range | EIRP limit |
| --- | --- |
| 1920 - 1980 MHz | -43 dBm/5 MHz |

Table 4.3.1-3: Requirements on wideband RMR BS receiver characteristics

| Parameter | Value |
| --- | --- |
| Level of the wanted signal | RefSens + 3 dB |
| Maximum 5 MHz LTE interfering signal in 1805 - 1880 MHz | -20 dBm |
| NOTE 1: The antenna connector of the BS receiver is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. NOTE 2: These requirements cover both blocking and third-order intermodulation. |  |

### 4.3.2	Terminals other than cab-radios using wideband technologies

Table 4.3.2-1: Terminals and cab-radio requirements using wideband technologies

| Maximum output power | 23 dBm |
| --- | --- |
| ACLR | 30dB minimum |
| Other | Uplink power control is mandatory and shall be activated. |

# 5	Frequency band arrangement

The new RMR band is within the range of FR1 and is proposed as TDD band (Table 5-1).

Table 5-1: New NR RMR band in FR1

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD |

# 6	NR system parameters

The following system parameters are defined for RMR 1900MHz band:

Table 6-1: RMR 1900 - Channel bandwidth

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 | [5] | 10 |  |  |  |  |  |  |  |  |  |  |  |
| n101 | 30 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6-2: RMR 1900 - applicable NR-ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n101 | 100 | 380000 – <20> – 382000 | 380000 – <20> – 382000 |

Table 6-3: RMR 1900 – applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n101 | -15 kHz | Case A | 4754 – <1> – 4768 |
|  | 30 kHz | Case C | 4760 – <1> – 4764 |

# 7	RF requirements

## 7.1	BS specific requirements

### 7.1.1	General

According to ECC Decision (20)02 [1], the BS RF work for RMR1900 is limited to the Wide Area Base Stations,  determined by the use along railway lines and their corresponding characteristics defined by 3GPP TS 38.104 [4].

In accordance to ECC Decision 20(02) [1], the related studies were done for the non-AAS BS architectures,  defined in 3GPP TS 38.104 [4]. Therefore the RAN4 requirements derivation was limited to the BS type 1-C requirements, only.

As the EIRP limits were defined in the ECC Decision (20)02 [1], consideration of non-AAS BS architecture required to convert those EIRP limits into the conducted requirements assuming a maximum antenna gain that can vary depending on the used antenna and related antenna gain as well as the potential losses, e.g., feeder. In the present consideration an antenna gain of 18 dBi and 4 dB losses in accordance to ECC Report 318 [6] resulting to a maximum gain value of GmaxRMR1900 = 14 dBi. Other assumptions may lead to other maximum gain values and conducted requirements. Also, one antenna connector was assumed. If the BS has more than one antenna connector, the limits shall be re-calculated considering the intended number of antenna connectors serving a cell.

### 7.1.2	Transmitter characteristics

#### 7.1.2.1	BS maximum output power

Based on ECC Decision (20)02 [1], the BS maximum output power for BS operating in band n101 applicable for [uncoordinated deployment], should not exceed the Prated,c,AC derived based on table 4.3.1-1 EIRP limits, with the assumption of a maximum value of the BS GmaxRMR1900 , and assuming one antenna connector,as:

(65 – GmaxRMR1900) dBm / 10 MHz = 51 dBm / 10 MHz,

(62 – GantRMR1900) dBm / 5 MHz = 48 dBm / 5 MHz,

where GmaxRMR1900 is the maximum gain (antenna gain and losses) of the RMR 1900 BS consideration.

Note:	5 MHz channel bandwidth support was not considered in ECC Decision (20)02 [1] and was added on request of RMR operators. The above limit for 5 MHz channel bandwidth was then specified based on the limit for 10 MHz, keeping constant PSD.

#### 7.1.2.2	Unwanted emissions

##### 7.1.2.2.1	General

Only Category B emission requirements are applicable to NR operation in RMR1900 band.

##### 7.1.2.2.2	Tx spurious emissions

Additional Tx spurious emission requirements are considered for band n101 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], as summarized in table 7.1.2.2.2-1. The requirement is derived based on table 4.3.1-5, with the assumption of maximum gain value of the BS GmaxRMR1900:

Table 7.1.2.2.2-1: Additional Tx spurious emissions requirement derivation for n101

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 1920 MHz – 1980 MHz | - 43 - GmaxRMR1900 = -57 dBm (NOTE) | 5 MHz |
| NOTE: Assuming a 14dBi maximum gain (antenna gain and losses) for GmaxRMR1900, and assuming one antenna connector. |  |  |

### 7.1.3	Receiver characteristics

RAN4 will not specify requirements related to the CEPT assumptions for BS enhanced selectivity operating adjacent to band n1 and deployed close by the corresponding railway infrastructure.

#### 7.1.3.2	Rx blocking

Rx blocking requirements applicable for band n101 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], are summarized in table 7.1.3.2-1. Interfering signal characteristic is 5 MHz LTE signal, as specified in ECC Decision (20)02) [1].

Table 7.1.3.2-1: Rx blocking requirement derivation for n101

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Center Frequency of Interfering Signal (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| [5], 10 | PREFSENS + 3 dB | Wide Area BS: -20 | 1807.5 MHz to 1877.5 MHz | 5 MHz LTE signal |

## 7.2	UE specific requirements

### 7.2.1	Transmitter characteristics

The UE requirements for RMR 1900 are summarized in the upcoming tables applicable for 3GPP TS 38.101-1 [2].

Table 7.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n101 |  |  |  |  |  |  | 23 | ±2 |

Table 7.2.1-2: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n101 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 41, 42, 43, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |

### 7.2.2	Receiver characteristics

The following 3GPP TS 38.101-1 [2] receiver characteristics changes are expected due to introduction of 1900MHz RMR band:

Table 7.2.2-1: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |

Table 7.2.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n100 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.2.2-3: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28,n34, n38,n39, n40, n41, n483, n50, n51, n53, n65, n66, n67, n70, n74, n75, n76, n85, n91, n92, n93, n94, n101 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |

Table 7.2.2-4: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, | Pinterferer | dBm | -44 | -30 | -15 |
| n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n485, n50, n51, n536, n65, n66, n67, n70, n71, n74, n75, n76, n85, n91, n92, n93, n94, n101 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

Table 7.2.2-5: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n74, n75, n76, n85, n101 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |

# 8	Performance requirements

The High Speed Train (HST) BS demodulation requirements are to be transparently applied for the NR operation in the RMR1900 operating band.

# 9	Deployment aspects

In general, network deployment aspects are out of scope of the RAN4 work. For the derivation of the BS RF requirements it was assumed that the RMR and MFCN base stations are not co-located.

Co-location of a MFCN BS and a RMR BS requires coordination among the involved parties.

In accordance with ECC Decision (20)02 Decides 4 [1], in case of implementation of coordination procedures or other mitigation measures for band n101 deployments, it is possible to deploy higher E.I.R.P. than stated in the technical conditions in ECC Decision (20)02 Annex 3 [1].

For uncoordinated deployment the technical conditions stated in ECC Decision (20)02 Annex 3 [1] apply for the in-block requirements, i.e. output power limit applies for band n101 operation.

The use of spectrum and its related conditions, e.g., E.I.R.P, in accordance to ECC Decision (20)02 Part B [1], are in the responsibility of national regulation and coordination among involved parties.

# 10	Conclusion

The study dealt with how the 1900-1910MHz spectrum block (RMR1900) can be used for rail communication using 5G NR applicable in Europe. The subject-areas 5G NR system, BS RF and UE RF were examined in detail.

General:	The focus was on the use of RMR1900 under uncoordinated conditions, assuming that RMR BS operates independently of other operators e.g. MFCN. Co-location with other operators and resulting coordination is subject to national regulation and will be not further detailed by 3GPP RAN4.

System Parameter:	The RMR1900 spectrum block can be used by either a 10MHz CBW or a 5MHz CBW. The corresponding system parameters are to be transferred to 3GPP TS 38.101-1 [2] and 3GPP TS 38.104 [4].

BS RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver. The agreed  parameters are to be transferred to 3GPP TS 38.104 [4]. Furthermore, the coexistence with other 3GPP radio technologies has been included. These are to be captured in the corresponding normative 3GPP technical specifications.

UE RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly, and the necessary conclusions were derived for the use of Power Class 3 (23dBm). The agreed parameters in the present TR are to be transferred  to 3GPP TS 38.101-1 [2].

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-08 | RAN4#100-e |  |  |  |  | Initial baseline | 0.0.1 |
| 2021-11 | RAN4#101e | R4-2114886 |  |  |  | TP to 38.852 on 1900MHz RMR RAN4 system parameters | 0.1.0 |
| 2022-01 | RAN4#101-bis-e | R4-2120042 |  |  |  | TP to 38.852 on 1900MHz RMR RAN4 system parameters | 0.2.0 |
| 2022-01 | RAN4#101-bis-e | R4-2120044 |  |  |  | TP to 38.852 on 1900MHz RMR RAN4 UE RF requirements | 0.2.0 |
| 2022-01 | RAN4#101-bis-e | R4-2120681 |  |  |  | TPs to TR 38.853: BS RF related agreements for RMR1900 | 0.2.0 |
| 2022-03 | RAN4#102-e | R4-2201328 |  |  |  | RMR 900MHz band – TP to TR 38.852 | 0.2.0 |
| 2022-03 | RAN4#102-e | R4-2202243 |  |  |  | Proposed changes for clause 7.1 in 3GPP TR 38.852 | 0.2.0 |
| 2022-03 | RAN4#102-e | R4-2202244 |  |  |  | Proposed changes for clause 7.2 in 3GPP TR 38.852 | 0.2.0 |
| 2022-03 | RAN4#102-e | R4-2203052 |  |  |  | Proposed changes for clause 9 in 3GPP TR 38.852 | 0.2.0 |
| 2022-03 | RAN4#102-e | R4-2203058 |  |  |  | TP to TR 38.852: BS RF requirements | 0.2.0 |
| 2022-04 | RAN4#102-e | R4-2207272 |  |  |  | TP BS RF conducted requirements for n101 | 0.3.0 |
| 2022-04 | RAN4#102-e | R4-2206279 |  |  |  | TP 1900MHz RMR band – conclusion | 0.3.0 |
| 2022-05 | RAN4#103-e | R4-2210882 |  |  |  | RMR1900 MHz band – TP to TR 38.852 | 0.3.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-06 | RAN#96 |  |  |  |  | Approved by plenary – Rel-17 spec under change control | 17.0.0 |
| 2023-03 | RAN#99 | RP-230517 | 0001 |  | F | Correction to TR 38.852 | 17.1.0 |
| 2023-06 | RAN#100 | RP-231339 | 0002 | 1 | F | CR to TR 38.852 - clarification on the limits specified from ECC EIRP limits for band n101 | 17.2.0 |
| 2023-12 | RAN#102 | RP-233350 | 0006 | 1 | F | Correction to TR 38.852 | 17.3.0 |