3GPP TS 38.521-3 V18.1.0 (2023-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;
User Equipment (UE) conformance specification;
Radio transmission and reception;
Part 3: Range 1 and Range 2 Interworking operation
with other radios

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Keywords

3GPP, New Radio

Contents

Foreword	17

1	Scope	18

2	References	18

3	Definitions, symbols and abbreviations	19

3.1	Definitions	19

3.2	Symbols	19

3.3	Abbreviations	20

4	General	22

4.1	Relationship between minimum requirements and test requirements	22

4.2	Applicability of minimum requirements	22

4.3	Specification suffix information	23

4.4	Test points analysis	24

4.5	Applicability and test coverage rules	24

4.5.0	General	24

4.5.1	Test coverage across 5G NR connectivity options	25

4.6	E-UTRA configuration for EN-DC FR1 tests applying the E-UTRA anchor-agnostic approach	25

4.7	E-UTRA configuration for EN-DC FR2 tests applying the E-UTRA anchor-agnostic approach	27

4.8	E-UTRA configuration for EN-DC FR1 tests not applying the E-UTRA anchor-agnostic approach	29

5	Operating bands and channel arrangement	29

5.1	General	29

5.2	Operating bands	30

5.2A	Operating bands for CA	30

5.2A.1	Inter-band CA between FR1 and FR2	30

5.2B	Operating bands for DC	30

5.2B.1	General	30

5.2B.2 to 5.2B.7	Void	30

5.2E	Operating bands for V2X	30

5.2E.1	Intra-band V2X bands	30

5.2E.2	Inter-band V2X bands	30

5.3	UE Channel bandwidth	31

5.3A	UE Channel bandwidth for CA	31

5.3A.1	Inter-band CA between FR1 and FR2	31

5.3B	UE Channel bandwidth for DC	31

5.3B.0	General	31

5.3B.1	Intra-band EN-DC in FR1	32

5.3B.1.1	General	32

5.3B.1.2	BCS for Intra-band contiguous EN-DC	32

5.3B.1.3	BCS for Intra-band non-contiguous EN-DC	32

5.3E	UE Channel bandwidth for V2X	33

5.3E.0	General	33

5.3E.1	Intra-band contiguous V2X in FR1	33

5.3E.2	Intra-band non-contiguous V2X in FR1	34

5.3E.3	Inter-band V2X in FR1	34

5.4	Void	34

5.4A	Channel arrangement for CA	34

5.4B	Channel arrangement for DC	34

5.4B.0	General	34

5.4B.1	Channel spacing for intra-band EN-DC carriers	34

5.5	Configuration	35

5.5A	Configuration for CA	35

5.5A.1	Inter-band CA configurations between FR1 and FR2	35

5.5B	Configuration for DC	37

5.5B.1	General	37

5.5B.2	Intra-band contiguous EN-DC	37

5.5B.3	Intra-band non-contiguous EN-DC	38

5.5B.4	Inter-band EN-DC within FR1	38

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)	39

5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)	44

5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)	51

5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)	56

5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)	58

5.5B.4a	Inter-band NE-DC within FR1	59

5.5B.4a.1	Inter-band NE-DC configurations within FR1 (two bands)	59

5.5B.5	Inter-band EN-DC including FR2	59

5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)	60

5.5B.5.2	Inter-band EN-DC configurations including FR2 (three bands)	63

5.5B.5.3	Inter-band EN-DC configurations including FR2 (four bands)	69

5.5B.5.4	Inter-band EN-DC configurations including FR2 (five bands)	73

5.5B.5.5	Void	73

5.5B.6	Inter-band EN-DC including FR1 and FR2	73

5.5B.6.1	Void	74

5.5B.6.2	Inter-band EN-DC configurations including FR1 and FR2 (three bands)	74

5.5B.6.3	Inter-band EN-DC configurations including FR1 and FR2 (four bands)	74

5.5B.6.4	Inter-band EN-DC configurations including FR1 and FR2 (five bands)	74

5.5B.6.5	Inter-band EN-DC configurations including FR1 and FR2 (six bands)	74

5.5B.7	Inter-band NR-DC between FR1 and FR2	74

5.5B.7.0	General	74

5.5B.7.1	Inter-band NR-DC configurations between FR1 and FR2 (two bands)	74

5.5E	Configuration for V2X operation	74

5.5E.1	General	74

5.5E.2	Intra-band contiguous V2X operation in FR1	75

5.5E.3	Intra-band non-contiguous V2X operation in FR1	75

5.5E.4	Inter-band V2X operation in FR1	75

5.5E.4.1	Inter-band V2X configurations within FR1 (two bands)	75

6	Transmitter characteristics	75

6.1	General	75

6.2	Transmitter power	77

6.2A	Transmitter power for CA without EN-DC	77

6.2A.1	UE maximum output power for CA	77

6.2A.1.1	UE maximum output power for inter-band NR CA between FR 1 and FR 2 without EN-DC	77

6.2A.2	UE maximum output power reduction for CA	77

6.2A.2.1	UE maximum output power reduction for inter-band NR CA between FR 1 and FR 2 without EN-DC	77

6.2A.3	UE additional maximum output power reduction for CA	77

6.2A.3.1	UE additional maximum output power reduction for inter-band NR CA between FR 1 and FR 2 without EN-DC	77

6.2A.4	Configured output power level for CA	78

6.2A.4.1	Configured output power level for inter-band NR CA between FR 1 and FR 2 without EN-DC	78

6.2A.4.2	ΔTIB,c for CA	78

6.2A.4.2.1	ΔTIB,c for inter-band CA between FR 1 and FR 2	78

6.2B	Transmitter power for DC	78

6.2B.1	UE Maximum Output Power for DC	78

6.2B.1.1	UE Maximum Output Power for Intra-Band Contiguous EN-DC	78

6.2B.1.2	UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC	83

6.2B.1.3	UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC)	87

6.2B.1.3_1	UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC)	110

6.2B.1.3a	UE Maximum Output Power for Inter-Band NE-DC within FR1	115

6.2B.1.4	UE Maximum Output Power for Inter-Band EN-DC including FR2	116

6.2B.1.4.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TRP	116

6.2B.1.4.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage	117

6.2B.1.4_1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (>1 NR CC)	118

6.2B.1.4_1.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs)	118

6.2B.1.4_1.1.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - EIRP and TRP	118

6.2B.1.4_1.1.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - Spherical Coverage	119

6.2B.1.4_1.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs)	120

6.2B.1.4_1.2.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - EIRP and TRP	120

6.2B.1.4_1.2.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - Spherical Coverage	121

6.2B.1.4_1.3	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs)	122

6.2B.1.4_1.3.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - EIRP and TRP	122

6.2B.1.4_1.3.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) – Spherical Coverage	123

6.2B.1.4_1.4	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs)	124

6.2B.1.4_1.4.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) - EIRP and TRP	124

6.2B.1.4_1.4.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) – Spherical Coverage	124

6.2B.1.4_1.5	UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs)	124

6.2B.1.4_1.5.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) - EIRP and TRP	124

6.2B.1.4_1.5.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) – Spherical Coverage	125

6.2B.1.4_1.6	UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs)	125

6.2B.1.4_1.6.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) - EIRP and TRP	125

6.2B.1.4_1.6.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) – Spherical Coverage	125

6.2B.1.4_1.7	UE Maximum Output Power for Inter-Band EN-DC including FR2 (8 NR CCs)	125

6.2B.1.4_1.7.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (8 NR CCs) - EIRP and TRP	125

6.2B.1.4_1.7.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (8 NR CCs) – Spherical Coverage	126

6.2B.1.4D	UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO - EIRP and TRP	126

6.2B.1.4D.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO	126

6.2B.1.4D.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO- Spherical coverage	127

6.2B.1.5	UE Maximum Output Power for Inter-Band EN-DC including both FR1 and FR2	128

6.2B.1.5D	UE Maximum Output Power for Inter-Band EN-DC including both FR1 and FR2 for UL MIMO	128

6.2B.1.6	UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps	128

6.2B.2	UE Maximum Output Power reduction for EN-DC	129

6.2B.2.0	General	129

6.2B.2.1	UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC	129

6.2B.2.2	UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC	142

6.2B.2.3	UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 (1 NR CC)	152

6.2B.2.3a	UE Maximum Output Power reduction for Inter-Band NE-DC within FR1 (1 NR CC)	153

6.2B.2.4	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC)	153

6.2B.2.4a	UE Maximum Output Power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC)	154

6.2B.2.4D	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC)	155

6.2B.2.4_1	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (>1 NR CC)	156

6.2B.2.4_1.1	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (2 NR CCs)	156

6.2B.2.4_1.2	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (3 NR CCs)	157

6.2B.2.4_1.3	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (4 NR CCs)	158

6.2B.2.5	UE Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2	159

6.2B.2.5D	UE Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2 for UL MIMO	159

6.2B.3	UE additional maximum output power reduction for EN-DC	159

6.2B.3.1	UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC	159

6.2B.3.2	UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC	184

6.2B.3.3	UE Additional Maximum Output Power reduction for Inter-Band EN-DC within FR1 (1 NR CC)	202

6.2B.3.4	UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC)	203

6.2B.3.4D	UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) for UL MIMO	205

6.2B.3.5	UE Additional Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2	206

6.2B.3.5D	UE Additional Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2 for UL MIMO	206

6.2B.4	Configured Output Power for EN-DC	206

6.2B.4.1	Configured Output Power Level for EN-DC	206

6.2B.4.1.0	Minimum Conformance Requirements	206

6.2B.4.1.1	Configured Output Power Level for Intra-Band Contiguous EN-DC	216

6.2B.4.1.2	Configured Output Power for Intra-Band Non-Contiguous EN-DC	221

6.2B.4.1.3	Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC)	226

6.2B.4.1.3_1	Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC)	233

6.2B.4.1.4	Configured Output Power for Inter-Band EN-DC including FR2 (1 NR CC)	238

6.2B.4.1.4_1	Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC)	239

6.2B.4.1.5	Configured Output Power for Inter-Band EN-DC including both FR1 and FR2	240

6.2B.4.2	ΔTIB,c for EN-DC	240

6.2B.4.2.1	Intra-Band Contiguous EN-DC	240

6.2B.4.2.2	Intra-Band non-Contiguous EN-DC	240

6.2B.4.2.3	Inter-Band EN-DC within FR1	241

6.2B.4.2.3a	Inter-band NE-DC within FR1	260

6.2B.4.2.4	Inter-band EN-DC including FR2	260

6.2B.4.2.5	Inter-band EN-DC including both FR1 and FR2	261

6.2B.5	Configured Output Power for NR-DC	261

6.2B.5.1	Configured Output power Level	261

6.2B.5.1.1	Configured Output Power Level for Inter-band NR-DC between FR1 and FR2	261

6.2E	Transmitter power for V2X in FR1	262

6.2E.1	UE Maximum Output Power for V2X	262

6.2E.1.0	Minimum conformance requirements	262

6.2E.1.1	UE maximum output power for Intra-band contiguous V2X	263

6.2E.1.2	UE maximum output power for Intra-band non-contiguous V2X	263

6.2E.1.3	UE maximum output power for Inter-band V2X	263

6.2E.1.3.1	UE maximum output power for Inter-band V2X with E-UTRA Uu and NR Sidelink	263

6.2E.1.3.2	UE maximum output power reduction for Inter-band V2X with NR Uu and E-UTRA V2X	265

6.2E.2	UE maximum output power reduction for V2X	267

6.2E.2.0	Minimum conformance requirements	267

6.2E.2.1	UE maximum output power reduction for Intra-band V2X	267

6.2E.2.2	UE maximum output power reduction for Inter-band V2X	267

6.2E.2.2.1	UE maximum output power reduction for Inter-band V2X with E-UTRA Uu and NR Sidelink	267

6.2E.2.2.2	UE maximum output power reduction for Inter-band V2X with NR Uu and E-UTRA V2X	269

6.3	Output power dynamics	271

6.3A	Output Power Dynamics for CA without EN-DC	271

6.3A.1	UE Output Power Dynamics for CA	271

6.3A.1.1	UE Output Power Dynamics for NR CA between FR 1 and FR 2 without EN-DC	271

6.3B	Output power dynamics for DC	272

6.3B.1	Minimum Output Power for EN-DC	272

6.3B.1.1	Minimum Output Power for intra-band contiguous EN-DC	272

6.3B.1.2	Minimum output power for intra-band non-contiguous EN-DC	273

6.3B.1.3	Minimum output power for inter-band EN-DC within FR1 (1 NR CC)	274

6.3B.1.4	Minimum Output Power for EN-DC Interband including FR2 (1 NR CC)	275

6.3B.1.4_1	Minimum output power for inter-band EN-DC including FR2 (>1 NR CC)	275

6.3B.1.4_1.1	Minimum output power for inter-band EN-DC including FR2 (2 NR CCs)	275

6.3B.1.4_1.2	Minimum output power for inter-band EN-DC including FR2 (3 NR CCs)	276

6.3B.1.4_1.3	Minimum output power for inter-band EN-DC including FR2 (4 NR CCs)	277

6.3B.1.4_1.4	Minimum output power for inter-band EN-DC including FR2 (5 NR CCs)	278

6.3B.1.4_1.5	Minimum output power for inter-band EN-DC including FR2 (6 NR CCs)	279

6.3B.1.4_1.6	Minimum output power for inter-band EN-DC including FR2 (7 NR CCs)	280

6.3B.1.4_1.7	Minimum output power for inter-band EN-DC including FR2 (8 NR CCs)	281

6.3B.1.4D	Minimum output power for inter-band EN-DC including FR2 for UL MIMO	282

6.3B.2	Transmit OFF Power for EN-DC	283

6.3B.2.1	Transmit OFF Power for intra-band contiguous EN-DC	283

6.3B.2.2	Transmit OFF Power for intra-band non-contiguous EN-DC	283

6.3B.2.3	Transmit OFF Power for inter-band EN-DC within FR1 (1 NR CC)	284

6.3B.2.4	Transmit OFF Power for inter-band EN-DC including FR2 (1 NR CC)	284

6.3B.2.4_1	Void	285

6.3B.2.4D	Transmit OFF Power for inter-band EN-DC including FR2 for UL-MIMO	285

6.3B.3	Transmit ON/OFF time mask/PUCCH time mask for EN-DC	286

6.3B.3.1	Transmit ON/OFF time mask for intra-band contiguous EN-DC	286

6.3B.3.2	Transmit ON/OFF time mask for intra-band non-contiguous EN-DC	287

6.3B.3.3	Transmit ON/OFF time mask for inter-band EN-DC within FR1 (1 NR CC)	288

6.3B.3.4	Transmit ON/OFF time mask for inter-band EN-DC including FR2 (1 NR CC)	288

6.3B.3.4_1	Transmit ON/OFF time mask for Inter-Band EN-DC including FR2 (>1 NR CC)	289

6.3B.3.4_1.1	Transmit ON/OFF time mask for Inter-Band EN-DC including FR2 (2 NR CCs)	289

6.3B.3.4D	Transmit ON/OFF time mask for inter-band EN-DC including FR2 for UL-MIMO	290

6.3B.3_1	E-UTRA and NR switching time mask for switching between two uplink carriers	291

6.3B.3_1.1	E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC	291

6.3B.4	PRACH time mask for EN-DC	294

6.3B.4.1	PRACH time mask for intra-band contiguous EN-DC	294

6.3B.4.2	PRACH Time Mask for intra-band non-contiguous EN-DC	296

6.3B.4.3	PRACH Time Mask for inter-band EN-DC within FR1 (1 NR CC)	297

6.3B.4.4	PRACH Time Mask for inter-band EN-DC including FR2 (1 NR CC)	298

6.3B.5	Output power dynamics for EN-DC with UL sharing from UE perspective	298

6.3B.5.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective	298

6.3B.6	Output power dynamics for intra-band EN-DC without dual PA capability	299

6.3B.7	Output power dynamics for intra-band EN-DC with dual PA capability	299

6.3B.8	Power control for EN-DC	299

6.3B.8.1	Absolute power tolerance for EN-DC	299

6.3B.8.1.1	Absolute power tolerance for intra-band contiguous EN-DC	299

6.3B.8.1.2	Absolute power tolerance for intra-band non-contiguous EN-DC	300

6.3B.8.1.3	Absolute power tolerance for inter-band EN-DC within FR1 (1 NR CC)	300

6.3B.8.1.4	Absolute power tolerance for inter-band EN-DC including FR2 (1 NR CC)	301

6.3B.8.2	Relative power tolerance for EN-DC	302

6.3B.8.2.1	Relative power tolerance for intra-band contiguous EN-DC	302

6.3B.8.2.2	Relative power tolerance for intra-band non-contiguous EN-DC	303

6.3B.8.2.3	Relative power tolerance for inter-band EN-DC within FR1 (1 NR CC)	304

6.3B.8.2.4	Relative power tolerance for inter-band EN-DC including FR2 (1 NR CC)	304

6.3B.8.3	Aggregate power tolerance for EN-DC	305

6.3B.8.3.1	Aggregate power tolerance for intra-band contiguous EN-DC	305

6.3B.8.3.2	Aggregate power tolerance for intra-band non-contiguous EN-DC	306

6.3B.8.3.3	Aggregate power tolerance for inter-band EN-DC within FR1 (1 NR CC)	307

6.3B.8.3.4	Aggregate power tolerance for inter-band EN-DC including FR2 (1 NR CC)	307

6.3E	Output power dynamics for V2X	308

6.3E.0	Minimum conformance requirements	308

6.3E.1	Output power dynamics for intra-band V2X operation	309

6.3E.2	Output power dynamics for inter-band V2X operation	309

6.4	Transmit signal quality	309

6.4A	Transmit Signal Quality for inter-band NR CA between FR and FR2 without EN-DC	309

6.4A.1	Frequency error for inter-band NR CA between FR 1 and FR 2 without EN-DC	309

6.4A.2	Transmit Modulation Quality for inter-band NR CA without EN-DC	310

6.4A.2.1	Error Vector Magnitude for inter-band NR CA between FR 1 and FR 2 without EN-DC	310

6.4A.2.2	Carrier Leakage for inter-band NR CA between FR 1 and FR 2 without EN-DC	310

6.4A.2.3	In-band Emissions for inter-band NR CA between FR 1 and FR 2 without EN-DC	310

6.4A.2.4	EVM Equalizer Spectral Flatness for inter-band NR CA between FR 1 and FR 2 without EN-DC	311

6.4B	Transmit Signal Quality for DC	311

6.4B.1	Frequency error	311

6.4B.1.1	Frequency error for Intra-band contiguous EN-DC	311

6.4B.1.2	Frequency error for Intra-band non-contiguous EN-DC	312

6.4B.1.3	Frequency error for Inter-band EN-DC within FR1 (1 NR CC)	313

6.4B.1.3a	Frequency Error for inter-band NE-DC within FR1	314

6.4B.1.4	Frequency Error for inter-band EN-DC including FR2 (1 NR CC)	314

6.4B.1.4_1	Frequency Error for Inter-band EN-DC including FR2 (>1 NR CC)	315

6.4B.1.4_1.1	Frequency Error for Inter-band EN-DC including FR2 (2 NR CCs)	315

6.4B.1.4_1.2	Frequency Error for Inter-band EN-DC including FR2 (3 NR CCs)	316

6.4B.1.4_1.3	Frequency Error for Inter-band EN-DC including FR2 (4 NR CCs)	317

6.4B.1.4D	Frequency error for inter-band EN-DC including FR2 for UL-MIMO	318

6.4B.2	Transmit Modulation Quality for DC	319

6.4B.2.1	Transmit Modulation Quality for intra-band contiguous EN-DC	319

6.4B.2.1.1	Error Vector Magnitude for intra-band contiguous EN-DC	319

6.4B.2.1.2	Carrier Leakage for intra-band contiguous EN-DC	320

6.4B.2.1.3	In-band Emissions for intra-band contiguous EN-DC	321

6.4B.2.1.4	EVM Equalizer Flatness for intra-band contiguous EN-DC	326

6.4B.2.2	Transmit Modulation Quality for intra-band non-contiguous EN-DC	327

6.4B.2.2.1	Error Vector Magnitude for intra-band non-contiguous EN-DC	327

6.4B.2.2.2	Carrier Leakage for intra-band non-contiguous EN-DC	329

6.4B.2.2.3	In-band Emissions for intra-band non-contiguous EN-DC	330

6.4B.2.2.4	EVM Equalizer Flatness for intra-band non-contiguous EN-DC	331

6.4B.2.3	Transmit Modulation Quality for inter-band EN-DC within FR1	332

6.4B.2.3.1	Error Vector Magnitude for inter-band EN-DC within FR1 (1 NR CC)	332

6.4B.2.3.2	Carrier Leakage for inter-band EN-DC within FR1 (1 NR CC)	333

6.4B.2.3.3	In-band Emissions for inter-band EN-DC within FR1 (1 NR CC)	334

6.4B.2.3.4	EVM Equalizer Flatness for inter-band EN-DC within FR1 (1 NR CC)	334

6.4B.2.3a	Transmit Modulation Quality for inter-band NE-DC within FR1	335

6.4B.2.3a.1	Error Vector Magnitude for inter-band NE-DC within FR1	335

6.4B.2.3a.2	Carrier Leakage for inter-band NE-DC within FR1	335

6.4B.2.3a.3	In-band Emissions for inter-band NE-DC within FR1	335

6.4B.2.3a.4	EVM Equalizer Flatness for inter-band NE-DC within FR1 (1 NR CC)	335

6.4B.2.4	Transmit Modulation Quality for inter-band EN-DC including FR2	336

6.4B.2.4.1	Error Vector Magnitude for inter-band EN-DC including FR2 (1 NR CC)	336

6.4B.2.4.1a	Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC)	337

6.4B.2.4.1_1	Error Vector Magnitude for inter-band EN-DC including FR2 (>1 NR CC)	338

6.4B.2.4.1_1.1	Error Vector Magnitude for inter-band EN-DC including FR2 (2 NR CCs)	338

6.4B.2.4.1_1.2	Error Vector Magnitude for inter-band EN-DC including FR2 (3 NR CCs)	339

6.4B.2.4.1_1.3	Error Vector Magnitude for inter-band EN-DC including FR2 (4 NR CCs)	339

6.4B.2.4.1_1.4	Error Vector Magnitude for inter-band EN-DC including FR2 (5 NR CCs)	340

6.4B.2.4.1_1.5	Error Vector Magnitude for inter-band EN-DC including FR2 (6 NR CCs)	341

6.4B.2.4.1_1.6	Error Vector Magnitude for inter-band EN-DC including FR2 (7 NR CCs)	342

6.4B.2.4.1_1.7	Error Vector Magnitude for inter-band EN-DC including FR2 (8 NR CCs)	343

6.4B.2.4.1D	Error Vector Magnitude for inter-band EN-DC including FR2 for UL MIMO	344

6.4B.2.4.2	Carrier Leakage for inter-band EN-DC including FR2 (1 NR CC)	345

6.4B.2.4.2D	Carrier Leakage for inter-band EN-DC including FR2 for UL MIMO	346

6.4B.2.4.2_1	Carrier Leakage for inter-band EN-DC including FR2 (>1 NR CC)	347

6.4B.2.4.2_1.1	Carrier Leakage for inter-band EN-DC including FR2 (2 NR CCs)	347

6.4B.2.4.2_1.2	Carrier Leakage for inter-band EN-DC including FR2 (3 NR CCs)	348

6.4B.2.4.2_1.3	Carrier Leakage for inter-band EN-DC including FR2 (4 NR CCs)	349

6.4B.2.4.2_1.4	Carrier Leakage for inter-band EN-DC including FR2 (5 NR CCs)	349

6.4B.2.4.2_1.5	Carrier Leakage for inter-band EN-DC including FR2 (6 NR CCs)	350

6.4B.2.4.2_1.6	Carrier Leakage for inter-band EN-DC including FR2 (7 NR CCs)	351

6.4B.2.4.2_1.7	Carrier Leakage for inter-band EN-DC including FR2 (8 NR CCs)	352

6.4B.2.4.3	In-band Emissions for inter-band EN-DC including FR2 (1 NR CC)	353

6.4B.2.4.3D	In-band Emissions for inter-band EN-DC including FR2 for UL MIMO	354

6.4B.2.4.3_1	In-band Emissions for inter-band EN-DC including FR2 (>1 NR CC)	355

6.4B.2.4.3_1.1	In-band Emissions for inter-band EN-DC including FR2 (2 NR CCs)	355

6.4B.2.4.3_1.2	In-band Emissions for inter-band EN-DC including FR2 (3 NR CCs)	356

6.4B.2.4.3_1.3	In-band Emissions for inter-band EN-DC including FR2 (4 NR CCs)	356

6.4B.2.4.3_1.4	In-band Emissions for inter-band EN-DC including FR2 (5 NR CCs)	357

6.4B.2.4.3_1.5	In-band Emissions for inter-band EN-DC including FR2 (6 NR CCs)	358

6.4B.2.4.3_1.6	In-band Emissions for inter-band EN-DC including FR2 (7 NR CCs)	359

6.4B.2.4.3_1.7	In-band Emissions for inter-band EN-DC including FR2 (8 NR CCs)	360

6.4B.2.4.4	EVM Equalizer Flatness for inter-band EN-DC including FR2 (1 NR CC)	361

6.4B.2.4.4D	EVM Equalizer Flatness for inter-band EN-DC including FR2 for UL MIMO	362

6.4B.2.4.5	EVM spectral flatness for pi/2 BPSK modulation for inter-band EN-DC including FR2 (1 NR CC)	363

6.4E	Transmit signal quality for V2X operation in FR1	364

6.4E.1	Frequency error for V2X	364

6.4E.1.0	Minimum conformance requirements	364

6.4E.1.1	Frequency error for Intra-band V2X	364

6.4E.1.2	Frequency error for Inter-band V2X	364

6.4E.2	Transmit modulation quality for V2X	364

6.4E.2.1	Error Vector Magnitude for V2X	364

6.4E.2.1.1	Error Vector Magnitude for intra-band V2X	365

6.4E.2.1.2	Error Vector Magnitude for Inter-band V2X	365

6.4E.2.2	Carrier leakage for V2X	365

6.4E.2.2.1	Carrier leakage for intra-band V2X	365

6.4E.2.3	In-band emissions for V2X	365

6.4E.2.3.1	In-band emissions for intra-band V2X	366

6.5	Output RF spectrum emissions	366

6.5A	Output RF spectrum emissions for CA	366

6.5A.1	Occupied bandwidth for CA without EN-DC	366

6.5A.2	Out-of-band emissions for CA without EN-DC	366

6.5A.2.1	Spectrum emissions mask for CA without EN-DC	366

6.5A.2.2	Additional Spectrum emissions mask for CA without EN-DC	366

6.5A.2.3	Adjacent channel leakage ratio for CA without EN-DC	367

6.5A.3	Spurious emissions for CA without EN-DC	367

6.5A.3.1	Inter-band CA between FR1 and FR2	367

6.5B	Output RF spectrum emissions for DC	367

6.5B.1	Occupied bandwidth for EN-DC	367

6.5B.1.1	Occupied bandwidth for Intra-Band Contiguous EN-DC	367

6.5B.1.2	Occupied bandwidth for Intra-Band Non-Contiguous EN-DC	370

6.5B.1.3	Occupied bandwidth for Inter-Band EN-DC within FR1 (1 NR CC)	371

6.5B.1.4	Occupied bandwidth for Inter-Band EN-DC including FR2 (1 NR CC)	371

6.5B.1.4_1	Occupied bandwidth for Inter-band EN-DC including FR2 (>1 NR CC)	372

6.5B.1.4_1.1	Occupied bandwidth for Inter-band EN-DC including FR2 (2 NR CCs)	372

6.5B.1.4_1.2	Occupied bandwidth for Inter-band EN-DC including FR2 (3 NR CCs)	373

6.5B.1.4_1.3	Occupied bandwidth for Inter-band EN-DC including FR2 (4 NR CCs)	374

6.5B.1.4D	Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO	375

6.5B.2	Out-of-band emissions for EN-DC	376

6.5B.2.1	Out-of-band emissions for Intra-band contiguous EN-DC	376

6.5B.2.1.1	Spectrum emissions mask for intra-band contiguous EN-DC	376

6.5B.2.1.2	Additional spectrum emissions mask for intra-band contiguous EN-DC	384

6.5B.2.1.3	Adjacent channel leakage ratio for intra-band contiguous EN-DC	389

6.5B.2.2	Out-of-band emissions for Intra-band non-contiguous EN-DC	391

6.5B.2.2.1	Spectrum emissions mask for intra-band non-contiguous EN-DC	391

6.5B.2.2.2	Additional Spectrum emissions mask for intra-band non-contiguous EN-DC	393

6.5B.2.2.3	Adjacent channel leakage ratio for intra-band non-contiguous EN-DC	395

6.5B.2.3	Out-of-band emissions for Inter-band EN-DC within FR1	397

6.5B.2.3.1	Spectrum emissions mask for Inter-band EN-DC within FR1 (1 NR CC)	397

6.5B.2.3.2	Additional Spectrum emissions mask for Inter-band EN-DC within FR1 (1 NR CC)	397

6.5B.2.3.3	Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC)	398

6.5B.2.3.3.1	NR - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC)	398

6.5B.2.3.3.2	UTRA - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC)	399

6.5B.2.3a	Out-of-band emissions for Inter-band NE-DC within FR1	400

6.5B.2.3a.1	Spectrum emissions mask for Inter-band NE-DC within FR1	400

6.5B.2.3a.2	Additional Spectrum emissions mask for Inter-band NE-DC within FR1	400

6.5B.2.3a.3	Adjacent channel leakage ratio for inter-band NE-DC within FR1	400

6.5B.2.4	Out-of-band emissions for Inter-band EN-DC including FR2	400

6.5B.2.4.1	Spectrum emissions mask for Inter-band EN-DC including FR2 (1 NR CC)	400

6.5B.2.4.1a	Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC)	401

6.5B.2.4.1_1	Spectrum emissions mask for Inter-band EN-DC including FR2 (>1 NR CC)	402

6.5B.2.4.1_1.1	Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs)	402

6.5B.2.4.1_1.2	Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs)	403

6.5B.2.4.1_1.3	Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs)	404

6.5B.2.4.1D	Spectrum emissions mask for inter-band EN-DC including FR2 for UL MIMO	405

6.5B.2.4.3	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (1 NR CC)	405

6.5B.2.4.3_1	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (>1 NR CC)	406

6.5B.2.4.3_1.1	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (2 NR CCs)	406

6.5B.2.4.3_1.2	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 NR CCs)	407

6.5B.2.4.3_1.3	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 NR CCs)	408

6.5B.2.4.3_1.4	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 NR CCs)	409

6.5B.2.4.3_1.5	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (6 NR CCs)	410

6.5B.2.4.3_1.6	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (7 NR CCs)	411

6.5B.2.4.3_1.7	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (8 NR CCs)	412

6.5B.2.4D.3	Adjacent channel leakage ratio for inter-band EN-DC including FR2 for UL MIMO	413

6.5B.3	Spurious emissions for EN-DC	414

6.5B.3.1	Spurious Emissions for intra-band contiguous EN-DC	414

6.5B.3.1.1	General spurious emissions for intra-band contiguous EN-DC	414

6.5B.3.1.2	Spurious emission band UE co-existence for intra-band contiguous EN-DC	416

6.5B.3.2	Spurious Emissions for intra-band non-contiguous EN-DC	419

6.5B.3.2.1	General spurious emissions for Intra-band non-contiguous EN-DC	419

6.5B.3.2.2	Spurious emission band UE co-existence for intra-band non-contiguous EN-DC	421

6.5B.3.3	Spurious Emissions for Inter-band EN-DC within FR1	424

6.5B.3.3.1	General spurious emissions for Inter-band EN-DC within FR1	424

6.5B.3.3.2	Spurious emission band UE co-existence for Inter-band within FR1	440

6.5B.3.3a	Inter-band NE-DC within FR1	507

6.5B.3.3a.1	General Spurious Emissions for Inter-band NE-DC within FR1	507

6.5B.3.3a.2	Spurious emission band UE co-existence for Inter-band NE-DC within FR1	507

6.5B.3.4	Spurious Emissions for Inter-band including FR2	507

6.5B.3.4.1	General Spurious Emissions for Inter-band including FR2 (1 NR CC)	507

6.5B.3.4.1a	General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)	508

6.5B.3.4.1_1	General Spurious emissions for Inter-band EN-DC including FR2 (>1 NR CC)	509

6.5B.3.4.1_1.1	General Spurious emissions for Inter-band EN-DC including FR2 (2 NR CCs)	509

6.5B.3.4.1_1.2	General Spurious emissions for Inter-band EN-DC including FR2 (3 NR CCs)	510

6.5B.3.4.1_1.3	General Spurious emissions for Inter-band EN-DC including FR2 (4 NR CCs)	511

6.5B.3.4.1_1.4	General Spurious emissions for Inter-band EN-DC including FR2 (5 NR CCs)	512

6.5B.3.4.1_1.5	General Spurious emissions for Inter-band EN-DC including FR2 (6 NR CCs)	513

6.5B.3.4.1_1.6	General Spurious emissions for Inter-band EN-DC including FR2 (7 NR CCs)	514

6.5B.3.4.1_1.7	General Spurious emissions for Inter-band EN-DC including FR2 (8 NR CCs)	515

6.5B.3.4.1D	General Spurious Emissions for inter-band EN-DC including FR2 for UL MIMO	516

6.5B.3.4.2	Spurious emission band UE co-existence for Inter-band including FR2 (1 NR CC)	516

6.5B.3.4.2a	Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC)	517

6.5B.3.4.2_1	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (>1 NR CC)	518

6.5B.3.4.2_1.1	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2 NR CCs)	518

6.5B.3.4.2_1.2	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3 NR CCs)	519

6.5B.3.4.2_1.3	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4 NR CCs)	520

6.5B.3.4.2_1.4	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (5 NR CCs)	521

6.5B.3.4.2_1.5	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (6 NR CCs)	522

6.5B.3.4.2_1.6	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (6 NR CCs)	523

6.5B.3.4.2_1.7	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (8 NR CCs)	524

6.5B.3.4.2D	Spurious emission band UE co-existence for inter-band EN-DC including FR2 for UL MIMO	525

6.5B.3.5	Spurious emissions for Inter-band including FR1 and FR2	526

6.5B.3.5.1	General Spurious Emissions for Inter-band including FR1 and FR2	526

6.5B.3.5.2	Spurious emission band UE co-existence for Inter-band including FR1 and FR2	526

6.5B.4	Additional Spurious Emissions for EN-DC	527

6.5B.4.1	Additional Spurious Emissions for Intra-band contiguous EN-DC	527

6.5B.4.2	Additional Spurious Emissions for Intra-band non-contiguous EN-DC	530

6.5B.4.3	Additional Spurious Emissions for Inter-band EN-DC within FR1 (1 NR CC)	531

6.5B.4.4	Additional Spurious Emissions for Inter-band including FR2 (1 NR CC)	532

6.5B.4.4a	Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)	532

6.5B.4.4_1	Additional Spurious Emissions for Inter-band including FR2 (>1 NR CC)	533

6.5B.4.4_1.1	Additional Spurious Emissions for Inter-band including FR2 (2 NR CC)	533

6.5B.4.4_1.2	Additional Spurious Emissions for Inter-band including FR2 (3 NR CC)	534

6.5B.4.4_1.3	Additional Spurious Emissions for Inter-band including FR2 (4 NR CC)	535

6.5B.5	Transmit intermodulation	536

6.5B.5.1	Transmit intermodulation for Intra-band contiguous EN-DC	536

6.5B.5.2	Transmit intermodulation for Intra-band non-contiguous EN-DC	536

6.5B.5.3	Transmit intermodulation for Inter-band EN-DC within FR1 (1 NR CC)	537

6.5B.5.3a	Transmit Intermodulation for Inter-band NE-DC within FR1	537

6.5B.5.4	Transmit intermodulation for Inter-band EN-DC including FR2	538

6.5B.5.5	Transmit intermodulation for Inter-band EN-DC including both FR1 and FR2	538

6.5E	Output RF spectrum emissions for V2X operation in FR1	538

6.5E.1	Occupied bandwidth for V2X	538

6.5E.1.0	Minimum conformance requirements	538

6.5E.1.1	Occupied bandwidth for intra-band V2X	538

6.5E.1.2	Occupied bandwidth for inter-band V2X	538

6.5E.2	Out of band emission for V2X	538

6.5E.2.0	Minimum conformance requirements	538

6.5E.2.1	Out of band emission for intra-band V2X	539

6.5E.2.2	Out of band emission for inter-band V2X	539

6.5E.3	Spurious emissions for V2X	539

6.5E.3.1	General spurious emissions for V2X	539

6.5E.3.1.1	General spurious emissions for intra-band V2X	539

6.5E.3.1.2	General spurious emissions for inter-band V2X	540

6.5E.4	Transmit intermodulation for V2X	540

6.5E.4.0	Minimum conformance requirements	540

6.5E.4.1	Transmit intermodulation for intra-band V2X	540

6.5E.4.2	Transmit intermodulation for inter-band V2X	540

6.6B	Beam Correspondence for EN-DC	541

6.6B.4	Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP	541

6.6B.5	Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP	542

7	Receiver characteristics	543

7.1	General	543

7.2	Void	544

7.3	Void	544

7.3A	Reference sensitivity for CA without EN-DC	544

7.3A.1	General	544

7.3A.2	Reference sensitivity power level for CA without EN-DC	544

7.3A.3	ΔRIB,c for CA without EN-DC	544

7.3A.3.1	ΔRIB,c for Inter-band CA between FR1 and FR2 without EN-DC	545

7.3A.4	Void	545

7.3B	Reference sensitivity level for DC	545

7.3B.1	General	545

7.3B.2	Reference sensitivity for EN-DC	545

7.3B.2.0	Minimum Conformance Requirements of Reference sensitivity for EN-DC	545

7.3B.2.0.1	Intra-band contiguous EN-DC	545

7.3B.2.0.2	Intra-band non-contiguous EN-DC	546

7.3B.2.0.3	Inter-band EN-DC within FR1	546

7.3B.2.0.3a	Inter-band NE-DC within FR1	574

7.3B.2.0.4	Inter-band EN-DC including FR2	574

7.3B.2.0.5	Inter-band EN-DC including both FR1 and FR2	574

7.3B.2.1	Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs)	575

7.3B.2.2	Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs)	580

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)	580

7.3B.2.3_1	Reference sensitivity for EN-DC within FR1 (>2 CCs)	615

7.3B.2.3_1.1	Reference sensitivity for EN-DC within FR1 (3 CCs)	615

7.3B.2.3_1.2	Reference sensitivity for EN-DC within FR1 (4 CCs)	650

7.3B.2.3_1.3	Reference sensitivity for EN-DC within FR1 (5 CCs)	655

7.3B.2.3_1.4	Reference sensitivity for EN-DC within FR1 (6 CCs)	658

7.3B.2.4	Reference sensitivity for Inter-band EN-DC including FR2 (1 NR CC)	658

7.3B.2.4_1	Reference sensitivity for Inter-band EN-DC including FR2 (>1 NR CC)	659

7.3B.2.4_1.1	Reference sensitivity for Inter-band EN-DC including FR2 (2 NR CCs)	659

7.3B.2.4_1.2	Reference sensitivity for Inter-band EN-DC including FR2 (3 NR CCs)	660

7.3B.2.4_1.3	Reference sensitivity for Inter-band EN-DC including FR2 (4 NR CCs)	661

7.3B.2.4_1.4	Reference sensitivity for Inter-band EN-DC including FR2 (5 NR CCs)	661

7.3B.2.4_1.5	Reference sensitivity for Inter-band EN-DC including FR2 (6 NR CCs)	662

7.3B.2.4_1.6	Reference sensitivity for Inter-band EN-DC including FR2 (7 NR CCs)	663

7.3B.2.4_1.7	Reference sensitivity for Inter-band EN-DC including FR2 (8 NR CCs)	663

7.3B.2.4D	Reference sensitivity for inter-band EN-DC including FR2 for UL MIMO	664

7.3B.2.5	Reference sensitivity for Inter-band EN-DC including FR1 and FR2 (3 CCs)	664

7.3B.2.5D	Reference sensitivity for inter-band EN-DC including FR1 and FR2 for UL MIMO	664

7.3B.2.6	Void	665

7.3B.3	ΔRIB,c ΔRIBNC for EN-DC	665

7.3B.3.0	General	665

7.3B.3.1	Reference sensitivity ΔRIB,c for Intra-band Contiguous EN-DC	665

7.3B.3.2	Reference sensitivity ΔRIB,c for Intra-band non-contiguous EN-DC	665

7.3B.3.3	ΔRIB,c for Inter-band EN-DC within FR1	666

7.3B.3.3.1	ΔRIB,c for EN-DC in two bands	666

7.3B.3.3.2	ΔRIB,c for EN-DC in three bands	669

7.3B.3.3.3	ΔRIB,c for EN-DC in four bands	674

7.3B.3.3.4	ΔRIB,c for EN-DC in five bands	678

7.3B.3.3.5	ΔRIB,c for EN-DC six bands	680

7.3B.3.3a	ΔRIB,c for Inter-band NE-DC within FR1	680

7.3B.3.4	Reference sensitivity for ΔRIB,c Inter-band EN-DC including FR2	680

7.3B.3.5	Reference sensitivity for ΔRIB,c Inter-band EN-DC including both FR1 and FR2	680

7.3B.4	EIS Spherical Coverage for Inter-band EN-DC including FR2 (1 NR CC)	680

7.4	Void	681

7.4A	Maximum Input Level for CA	681

7.4B	Maximum Input Level for DC	681

7.4B.0	Minimum conformance requirement	681

7.4B.0.1	Intra-band contiguous EN-DC	681

7.4B.0.2	Intra-band non-contiguous EN-DC	682

7.4B.0.3	Inter-band EN-DC within FR1	682

7.4B.0.4	Inter-band EN-DC including FR2	682

7.4B.0.5	Inter-band EN-DC including both FR1 and FR2	682

7.4B.1	Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs)	682

7.4B.2	Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs)	685

7.4B.3	Maximum Input Level for Inter-band EN-DC within FR1 (1 NR CC)	687

7.4B.3_1	Maximum Input Level for EN-DC within FR1 (>2 CCs)	688

7.4B.3_1.1	Maximum Input Level for EN-DC within FR1 (3 CCs)	688

7.4B.3_1.2	Maximum Input Level for EN-DC within FR1 (4 CCs)	688

7.4B.3_1.3	Maximum Input Level for EN-DC within FR1 (5 CCs)	689

7.4B.3_1.4	Maximum Input Level for EN-DC within FR1 (6 CCs)	690

7.4B.3a	Maximum Input Level for inter-band NE-DC within FR1 (2 CCs)	691

7.4B.4	Maximum Input Level for inter-band EN-DC including FR2 (1 NR CC)	691

7.4B.4_1	Maximum Input Level for inter-band EN-DC including FR2 (>1 NR CC)	692

7.4B.4_1.1	Maximum Input Level for Inter-Band EN-DC including FR2 (2 NR CCs)	692

7.4B.4_1.2	Maximum Input Level for Inter-Band EN-DC including FR2 (3 NR CCs)	693

7.4B.4_1.3	Maximum Input Level for Inter-Band EN-DC including FR2 (4 NR CCs)	694

7.4B.4_1.4	Maximum Input Level for Inter-Band EN-DC including FR2 (5 NR CCs)	695

7.4B.4D	Maximum Input Level for inter-band EN-DC including FR2 for UL MIMO	696

7.4B.5	Maximum Input Level for inter-band EN-DC including both FR1 and FR2	696

7.4B.5D	Maximum Input Level for inter-band EN-DC including FR1 and FR2 for UL MIMO	696

7.5	Void	697

7.5A	Adjacent Channel Selectivity for CA	697

7.5B	Adjacent channel selectivity for DC	697

7.5B.0	Minimum Conformance Requirements	697

7.5B.0.1	Intra-band contiguous EN-DC	697

7.5B.0.2	Intra-band non-contiguous EN-DC	698

7.5B.0.3	Inter-band EN-DC within FR1	698

7.5B.0.3a	Inter-band NE-DC within FR1	698

7.5B.0.4	Inter-band EN-DC including FR2	698

7.5B.0.4a	Inter-band NE-DC including FR2	698

7.5B.0.5	Inter-band EN-DC including both FR1 and FR2	698

7.5B.1	Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs)	699

7.5B.2	Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs)	702

7.5B.3	Adjacent Channel Selectivity for inter-band EN-DC within FR1 (1 NR CC)	703

7.5B.3_1	Adjacent Channel Selectivity for EN-DC within FR1 (>2 CCs)	703

7.5B.3_1.1	Adjacent Channel Selectivity for EN-DC within FR1 (2 NR CCs)	704

7.5B.3_1.2	Adjacent Channel Selectivity for EN-DC within FR1 (3 NR CCs)	704

7.5B.3_1.3	Adjacent Channel Selectivity for EN-DC within FR1 (4 NR CCs)	705

7.5B.3_1.4	Adjacent Channel Selectivity for EN-DC within FR1 (5 NR CCs)	706

7.5B.3a	Adjacent Channel Selectivity for inter-band NE-DC within FR1 (2 CCs)	707

7.5B.4	Adjacent Channel Selectivity for inter-band EN-DC including FR2 (1 NR CC)	707

7.5B.4_1	Adjacent Channel Selectivity for inter-band EN-DC including FR2 (>1 NR CC)	708

7.5B.4_1.1	Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2 NR CCs)	708

7.5B.4_1.2	Void	710

7.5B.4_1.3	Void	710

7.5B.4_1.4	Void	710

7.5B.4D	Adjacent Channel Selectivity for inter-band EN-DC including FR2 for UL MIMO	710

7.5B.5	Adjacent Channel Selectivity for inter-band EN-DC including both FR1 and FR2	710

7.5B.5D	Adjacent Channel Selectivity for inter-band EN-DC including FR1 and FR2 for UL MIMO	710

7.6	Void	710

7.6A	Blocking characteristics for CA	710

7.6B	Blocking characteristics for DC	711

7.6B.1	General	711

7.6B.2	Inband blocking for DC	711

7.6B.2.0	Minimum Conformance Requirements	711

7.6B.2.0.1	Intra-band contiguous EN-DC	711

7.6B.2.0.2	Intra-band non-contiguous EN-DC	711

7.6B.2.0.3	Inter-band EN-DC within FR1	712

7.6B.2.0.3a	Inter-band NE-DC within FR1	712

7.6B.2.0.4	Inter-band EN-DC including FR2	712

7.6B.2.0.5	Inter-band EN-DC including both FR1 and FR2	712

7.6B.2.1	Inband blocking for intra-band contiguous EN-DC (2 CCs)	712

7.6B.2.2	Inband blocking for intra-band non-contiguous EN-DC (2 CCs)	715

7.6B.2.3	Inband blocking for inter-band EN-DC within FR1 (1 NR CC)	716

7.6B.2.3_1	Inband blocking for EN-DC within FR1 (>2 CCs)	717

7.6B.2.3_1.1	Inband blocking for EN-DC within FR1 (3 CCs)	717

7.6B.2.3_1.2	Inband blocking for EN-DC within FR1 (4 CCs)	720

7.6B.2.3_1.3	Inband blocking for EN-DC within FR1 (5 CCs)	723

7.6B.2.3_1.4	Void	724

7.6B.2.3a	In-band blocking for inter-band NE-DC within FR1 (2 CCs)	724

7.6B.2.4	Inband blocking for inter-band EN-DC including FR2 (1 NR CC)	724

7.6B.2.4_1	Inband blocking for inter-band EN-DC including FR2 (>1 NR CC)	725

7.6B.2.4_1.1	Inband blocking for inter-band EN-DC including FR2 (2 NR CCs)	725

7.6B.2.4_1.2	Void	725

7.6B.2.4_1.3	Void	725

7.6B.2.4_1.4	Void	725

7.6B.2.4D	Inband blocking for inter-band EN-DC including FR2 for UL MIMO	726

7.6B.2.5	Inband blocking for inter-band EN-DC including both FR1 and FR2	726

7.6B.2.5D	Inband blocking for inter-band EN-DC including FR1 and FR2 for UL MIMO	726

7.6B.3	Out-of-band blocking for DC	726

7.6B.3.0	Minimum Conformance Requirements	726

7.6B.3.0.1	Intra-band contiguous EN-DC	726

7.6B.3.0.2	Intra-band non-contiguous EN-DC	727

7.6B.3.0.3	Inter-band EN-DC within FR1	727

7.6B.3.0.3a	Inter-band NE-DC within FR1	728

7.6B.3.0.4	Inter-band EN-DC including FR2	729

7.6B.3.0.5	Inter-band EN-DC including both FR1 and FR2	729

7.6B.3.1	Out-of-band blocking for intra-band contiguous EN-DC (2 CCs)	729

7.6B.3.2	Out-of-band blocking for intra-band non-contiguous EN-DC (2 CCs)	732

7.6B.3.3	Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs)	733

7.6B.3.3_1	Out-of-band blocking for EN-DC within FR1 (>2 CCs)	741

7.6B.3.3_1.1	Out-of-band blocking for EN-DC within FR1 (3 CCs)	741

7.6B.3.3_1.2	Out-of-band blocking for EN-DC within FR1 (4 CCs)	744

7.6B.4	Narrow band blocking for DC	748

7.6B.4.0	Minimum Conformance Requirements	748

7.6B.4.0.1	Intra-band contiguous EN-DC	748

7.6B.4.0.2	Intra-band non-contiguous EN-DC	749

7.6B.4.0.3	Inter-band EN-DC within FR1	749

7.6B.4.0.3a	Inter-band NE-DC within FR1	749

7.6B.4.0.4	Inter-band EN-DC including FR2	749

7.6B.4.0.5	Inter-band EN-DC including both FR1 and FR2	749

7.6B.4.1	Narrow band blocking for intra-band contiguous EN-DC (2 CCs)	750

7.6B.4.2	Narrow band blocking for intra-band non-contiguous EN-DC (2 CCs)	753

7.6B.4.3	Narrow band blocking for inter-band EN-DC within FR1 (1 NR CC)	754

7.6B.4.3_1	Narrow band blocking for EN-DC within FR1 (>2 CCs)	755

7.6B.4.3_1.1	Narrow band blocking for EN-DC within FR1 (3 CCs)	755

7.6B.4.3_1.2	Narrow band blocking for EN-DC within FR1 (4 CCs)	758

7.6B.4.3_1.3	Narrow band blocking for EN-DC within FR1 (5 CCs)	761

7.6B.4.3_1.4	Void	762

7.6B.4.3a	Narrow band blocking for inter-band NE-DC within FR1 (2 CCs)	762

7.6E	Blocking characteristics for V2X in FR1	762

7.6E.0	Minimum conformance requirements	762

7.6E.1	In-band blocking for V2X operation	762

7.6E.2	Out-of-band blocking for V2X operation	762

7.7	Void	763

7.7A	Spurious response for CA	763

7.7B	Spurious response for DC	763

7.7B.0	Minimum Conformance Requirements	763

7.7B.0.1	Intra-band contiguous EN-DC	763

7.7B.0.2	Intra-band non-contiguous EN-DC	763

7.7B.0.3	Inter-band EN-DC within FR1	764

7.7B.0.3a	Inter-band NE-DC within FR1	764

7.7B.0.4	Inter-band EN-DC including FR2	764

7.7B.0.5	Inter-band EN-DC including both FR1 and FR2	764

7.7B.1	Spurious Response for intra-band contiguous EN-DC (2 CCs)	764

7.7B.2	Spurious Response for intra-band non-contiguous EN-DC (2 CCs)	766

7.7B.3	Spurious Response for inter-band EN-DC within FR1 (2 CCs)	767

7.7B.3_1	Spurious Response for EN-DC within FR1 (>2 CCs)	771

7.7B.3_1.1	Spurious Response for EN-DC within FR1 (3 CCs)	771

7.7B.3_1.2	Spurious Response for EN-DC within FR1 (4 CCs)	773

7.7E	Spurious response for V2X in FR1	775

7.7E.0	Minimum conformance requirements	775

7.7E.1	Spurious response for V2X operation	776

7.8	Void	776

7.8B	Intermodulation characteristics for DC	776

7.8B.1	General	776

7.8B.2	Wide band Intermodulation	776

7.8B.2.0	Minimum Conformance Requirements	776

7.8B.2.0.1	Intra-band contiguous EN-DC	776

7.8B.2.0.2	Intra-band non-contiguous EN-DC	776

7.8B.2.0.3	Inter-band EN-DC within FR1	777

7.8B.2.0.4	Inter-band EN-DC including FR2	777

7.8B.2.0.5	Inter-band EN-DC including both FR1 and FR2	777

7.8B.2.1	Wideband Intermodulation for intra-band contiguous EN-DC (2 CCs)	777

7.8B.2.2	Wideband Intermodulation for intra-band non-contiguous EN-DC (2 CCs)	780

7.8B.2.3	Wideband Intermodulation for inter-band EN-DC within FR1 (2 CCs)	781

7.8B.2.3_1	Wideband Intermodulation for EN-DC within FR1 (>2 CCs)	782

7.8B.2.3_1.1	Wideband Intermodulation for EN-DC within FR1 (3 CCs)	782

7.8B.2.3_1.2	Wideband Intermodulation for EN-DC within FR1 (4 CCs)	785

7.8B.2.3_1.3	Wideband Intermodulation for EN-DC within FR1 (5 CCs)	788

7.8B.2.3_1.4	Wideband Intermodulation for EN-DC within FR1 (6 CCs)	790

7.8B.2.3a	Wideband Intermodulation for inter-band NE-DC within FR1 (2 CCs)	790

7.8B.2.4 to 7.8B.2.9	Void	791

7.8E	Intermodulation characteristics for V2X operation in FR1	791

7.8E.1	Wide band Intermodulation for V2X	791

7.8E.1.0	Minimum conformance requirements	791

7.8E.1.1	Wide band Intermodulation for V2X operation	791

7.9	Void	791

7.9B	Spurious emissions for DC	791

7.9B.0	Minimum Conformance Requirements	791

7.9B.0.1	Intra-band contiguous EN-DC	791

7.9B.0.2	Intra-band non-contiguous EN-DC	791

7.9B.0.3	Inter-band EN-DC within FR1	792

7.9B.0.4	Inter-band EN-DC including FR2	792

7.9B.0.5	Inter-band EN-DC including both FR1 and FR2	792

7.9B.1	Spurious Emissions for intra-band contiguous EN-DC (2 CCs)	792

7.9B.2	Spurious Emissions for intra-band non-contiguous EN-DC (2 CCs)	793

7.9B.3	Spurious Emissions for inter-band EN-DC within FR1 (1 NR CC)	794

7.9B.3_1	Spurious Emissions for EN-DC within FR1 (>2 CCs)	795

7.9B.3_1.1	Spurious Emissions for EN-DC within FR1 (3 CCs)	795

7.9B.3a	Spurious Emissions for inter-band NE-DC within FR1 (2 CCs)	795

7.9B.4	Spurious Emissions for inter-band EN-DC including FR2 (1 NR CC)	796

7.9B.5	Spurious Emissions for inter-band EN-DC including both FR1 and FR2 (3 CCs)	797

Annex A (normative): Measurement Channels	798

A.1	General	798

A.2	UL reference measurement channels for E-UTRA TDD Config 2	798

A.2.1	General	798

A.2.2	Reference measurement channels for E-UTRA	799

A.2.2.1	Full RB allocation	799

A.2.2.1.1	QPSK	799

A.2.2.1.2	16-QAM	799

A.2.2.1.3	64-QAM	800

A.2.2.1.4	256 QAM	801

A.2.2.2	Partial RB allocation	802

A.2.2.2.1	QPSK	802

A.2.2.2.2	16-QAM	804

A.2.2.2.3	64-QAM	807

A.2.2.2.4	256 QAM	810

A.3	DL reference measurement channels for E-UTRA	811

A.3.1	General	811

Annex B (normative): Propagation Conditions	816

Annex C (normative): Downlink Physical Channels	817

Annex D (normative): Characteristics of the Interfering Signal	818

Annex E (normative): Global In-Channel Tx Test	818

Annex F (informative): Measurement uncertainties and Test Tolerances	819

F.1	Acceptable uncertainty of Test System (normative)	819

F.1.0	General	819

F.1.0.1	Conducted Measurements	819

F.1.0.2	Radiated Measurements	819

F.1.1	Measurement of test environments	819

F.1.1.1	Conducted Measurements	819

F.1.1.2	Radiated Measurements	819

F.1.2	Measurement of transmitter	820

F.1.3	Measurement of receiver	831

F.2	Interpretation of measurement results (normative)	835

F.3	Test Tolerance and Derivation of Test Requirements (informative)	835

F.3.1	Measurement of test environments	835

F.3.2	Measurement of transmitter	836

F.3.3	Measurement of receiver	846

F.4	Uplink power window	849

F.4.1	Introduction	849

F.4.2	Setting the power window above a requirement	850

F.4.2.1	NR FR1	850

F.4.2.2	NR FR2	851

F.4.2.3	E-UTRA	852

F.4.3	Setting the power window below a requirement	853

F.4.3.1	NR FR1	853

F.4.3.2	NR FR2	854

F.4.3.3	E-UTRA	855

F.4.4	Setting the power window centred on a target value	856

F.4.4.1	NR FR1	856

F.4.4.2	NR FR2	857

F.4.4.3	E-UTRA	858

Annex G (normative): Uplink Physical Channels	860

Annex H (normative): Statistical Testing	861

H.1	General	861

H.2	Statistical testing of receiver characteristics	861

H.2.1	General	861

H.2.2	Mapping throughput to error ratio	861

H.2.3	Design of the test	862

H.2.4	Numerical definition of the pass fail limits	863

H.2.5	Pass fail decision rules	864

Annex I (normative): Void	865

Annex J (normative): Test applicability per permitted test method	866

Annex K (normative): EIRP, TRP, and EIS measurement procedures	867

Annex L (normative): Void	868

Annex M (normative): Dual uplink interferer	869

Annex N (normative): Modified MPR behaviour	870

N.1	Indication of modified MPR behaviour	870

Annex O (informative): Change history	871

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

The present document is part 3 of a multi-part Technical Specification (TS) covering the New Radio (NR) User Equipment (UE) conformance specification, which is divided in the following parts:

-	TS 38.521-1 [8]: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone" (the present document).

-	TS 38.521-2 [9]: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

-	TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

-	TS 38.521-4 [22]: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance".

-	TS 38.522 [14]: "NR; User Equipment (UE) conformance specification; Applicability of RF and RRM test cases".

-	TS 38.533 [23]: "NR; User Equipment (UE) conformance specification; Radio resource management (RRM)".

# 1	Scope

The present document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain RF characteristics for carrier aggregation between Range 1 and Range 2 and additional requirements for EN-DC, NE-DC and NGEN-DC.

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the "definition and applicability" part of the test.

For example only Release 15 and later UE declared to support 5G-NR shall be tested for this functionality. In the event that for some tests different conditions apply for different releases, this is indicated within the text of the test itself.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[6]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[7]	3GPP TR 38.905: "NR; Derivation of test points for radio transmission and reception conformance test cases".

[8]	3GPP TS 38.521-1:" User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone.

[9]	3GPP TS 38.521-2:"NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

[10]	3GPP TS 36.521-1:"Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance testing"

[11]	3GPP TS 36.508:"Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[12]	3GPP TS 36.133:"Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[13]	3GPP TS 36.211: "E-UTRA; Physical channels and modulation".

[14]	3GPP TS 38.522: "NR; User Equipment (UE) conformance specification; Applicability of radio transmission, radio reception and radio resource management test cases".

[15]	Void.

[16]	3GPP TS 38.306: "NR: User Equipment (UE) radio access capabilities".

[17]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[18]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[19]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[20]	3GPP TS 36.213: "E-UTRA Physical layer procedures".

[21]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[22]	3GPP TS 38.521-4: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance".

[23]	3GPP TS 38.533: "NR; User Equipment (UE) conformance specification; Radio resource management (RRM)".

[24]	3GPP TS 36.214: "E-UTRA; Physical layer; Measurements".

[25]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for CA or DC operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for CA or DC operation, for serving cell c

BWE-UTRA_Channel	Channel bandwidth of E-UTRA carrier

BWE-UTRA_Channel_CA	Channel bandwidth of E-UTRA sub-block which is composed of intra-band contiguous CA E-UTRA carriers

BWNR_Channel	Channel bandwidth of NR carrier

BWNR_Channel_CA	Channel bandwidth of NR sub-block which is composed of intra-band contiguous CA NR carriers

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EN-DCACLR	The ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW

E-UTRAACLR	E-UTRA ACLR

FC	RF reference frequency for the carrier centre on the channel raster

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FInterferer	Frequency of the interferer

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the NR out of band emission and spurious emission domains

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth
NRB_agg=1jNRBj*2μj  for carrier 1 to j, where μ is defined in TS 38.211 [13]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks
NRB,cj=NRBj*2μj  for carrier j, where μ is defined in TS 38.211 [13]

PCMAX	The configured maximum UE output power

PEMAX	Maximum allowed UE output power signalled by higher layers

PInterferer	Modulated mean power of the interferer

PPowerClass	The nominal UE power (i.e. no tolerance)

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart	Indicates the lowest RB index of transmitted resource blocks

Wgap	The sub-block gap between the two sub-blocks

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth Combination Set

BPSK	Binary Phase Shift Keying

BW	Bandwidth

CA	Carrier Aggregation

CC	Component Carrier

CG	Carrier Group

CP-OFDM	Cyclic Prefix-OFDM

CQI	Channel quality indicator

CW	Continuous Wave

DC	Dual Connectivity

DCI	Downlink Control Information

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

DTX	Discontinuous Transmission

EIRP	Equivalent Isotropically Radiated Power

EIS	Effective Isotropic Sensitivity

EN-DC	E-UTRA/NR DC

E-UTRA	Evolved UTRA

EVM	Error Vector Magnitude

FDM	Frequency Division Multiplexing

FFT	Fast Fourier Transformation

FR	Frequency Range

ENBW	The aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block

HARQ	Hybrid automatic repeat request

IDFT	Inverse Discrete Fourier Transformation

ITS	Intelligent Transportation System

ITU-R	Radio communication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MCG	Master Cell Group

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MU	Measurement Uncertainty

MR-DC	Multi-Radio Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NR/5GC	NR connected to 5GC

NS	Network Signalling

NSA	Non-Standalone, a mode of operation where operation of a radio is assisted with another radio

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal frequency division multiplex

OOB	Out-of-band

OOBE	Out-of-band emission

OTA	Over The Air

PBCH	Physical broadcast channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

P-MPR	Power Management Maximum Power Reduction

PRACH	Physical random-access channel

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

QAM	Quadrature Amplitude Modulation

QPSK	Quadrature Phase Shift Keying

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

RMC	Reference Measurement Channel

RNTI	Radio Network Temporary Identifier

Rx	Receiver

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SL	Sidelink

SRS	Sounding Reference Symbol

SS	Synchronization Symbol / System Simulator

SUL	Supplementary uplink

TDM	Time Division Multiplex

TPC	Transmit Power Control

TRP	Total Radiated Power

TT	Test Tolerance

Tx	Transmitter

UE	User Equipment

UL	Uplink

UL MIMO	Up Link Multiple Antenna transmission

ULSUP	Uplink sharing from UE perspective

V2X	Vehicle to Everything

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

TS 38.101-3 [4] is interwork specification for NR UE, covering RF characteristics and minimum performance requirements. Conformance to TS 38.101-3 [4] is demonstrated by fulfilling the test requirements specified in the present document.

The Minimum Requirements given in TS 38.101-3 [4] make no allowance for measurement uncertainty (MU). The present document defines test tolerances (TT). These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in TS 38.101-3 [4] to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by various levels of "Shared Risk" principle as described below.

a)	Core specification value is not relaxed by any relaxation value (TT=0). For each single measurement, the probability of a borderline good UE being judged as FAIL equals the probability of a borderline bad UE being judged as PASS.

-	Test tolerances equal to 0 (TT=0) are considered in this specification.

b)	Core specification value is relaxed by a relaxation value (TT>0). For each single measurement, the probability of a borderline bad UE being judged as PASS is greater than the probability of a borderline good UE being judged as FAIL.

-	Test tolerances lower than measurement uncertainty and greater than 0 (0 < TT < MU) are considered in this specification.

-	Test tolerances high up to measurement uncertainty (TT = MU) are considered in this specification which is also known as "Never fail a good DUT" principle.

c)	Core specification value is tightened by a stringent value (TT<0). For each single measurement, the probability of a borderline good UE being judged as FAIL is greater than the probability of a borderline bad UE being judged as PASS.

-	Test tolerances lower than 0 (TT<0) are not considered in this specification.

The "Never fail a good DUT" and the "Shared Risk" principles are defined in Recommendation ITU-R M.1545 [21].

## 4.2	Applicability of minimum requirements

a)	In TS 38.101-3 [4] the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty, it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal.

d)	Terminal that supports EN-DC or NE-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [5] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in TS 38.101-3 [4].

e)	All the requirements for intra-band contiguous and non-contiguous EN-DC or NE-DC apply under the assumption of the same uplink-downlink and special subframe configurations in the E-UTRA and slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the NR for the EN-DC or NE-DC, a time offset between the two RATs configurations may be required.

f)	For EN-DC or NE-DC combinations with CA configurations for E-UTRA and/or NR, all the requirements for E-UTRA and/or NR all the requirements for E-UTRA and/or NR intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the PSCell and SCells for NR and the same uplink-downlink and special subframe configurations in PCell and SCells for E-UTRA.

A terminal which supports an EN-DC or NE-DC configuration shall support:

-	If any subsets of the EN-DC or NE-DC configuration do not specify its own bandwidth combination sets in 5.3B, then the terminal shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets it signals the support for in NR CA configuration part of E-UTRA – NR DC.

-	Else if one of the subsets of the EN-DC or NE-DC configuration specify its own bandwidth combination sets in 5.3B, then the terminal shall support a product set of channel bandwidth for each band specified by E-UTRA bandwidth combination sets, NR bandwidth combination sets, and EN-DC or NE-DC bandwidth combination sets it signals the support.

A terminal which supports an inter-band EN-DC or NE-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC or NE-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

A terminal which supports NE-DC configurations shall meet the minimum requirements for corresponding EN-DC configuration, unless otherwise specified.

For CA or DC configurations, which include FR2 intra-band CA combinations with multiple FR2 subblocks, where at least one of the subblocks is contiguous CA combination.

-	if the field partialFR2-FallbackRX-Req is not present, the UE shall meet all applicable UE RF requirements for the highest order CA configuration and all associated fallback CA configurations;

-	if the field partialFR2-FallbackRX-Req is present, for each FR2 intra-band CA configuration with multiple sub-blocks that the UE indicates support for explicitly in UE capability signalling: the in-gap UE RF requirements in clauses 7.5A, 7.5B, 7.6A, 7.6B apply as the equivalent requirements for the associated fallback FR2 intra-band CA configurations with the same number of sub-blocks, where at least one of the sub-blocks consists of a contiguous CA configuration. The UE shall meet all applicable UE RF requirements for fallback CA configurations with a lesser number of sub-blocks;

-	regardless of the field partialFR2-FallbackRX-Req, the UE shall meet all DL out-of-gap requirements for all lower order fallback CA configurations.

Terminal that supports inter-band NR-DC between FR1 and FR2 configuration shall meet the requirements for corresponding CA configuration (suffix A), unless otherwise specified.

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) between FR1 and FR2 |
| B | Dual-Connectivity (DC) with and without SUL including UL sharing from UE perspective, inter-band NR DC between FR1 and FR2 |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |

## 4.4	Test points analysis

The information on test point analysis and test point selection including number of test points for each test case is shown in TR 38.905 [7] clause 4.3.

## 4.5	Applicability and test coverage rules

## 4.5.0	General

(1)	The applicability and test coverage rules for Non-Standalone (NSA) only capable devices shall include the following:

a)	For each NR band in a device; test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)	Test all the EN-DC or NE-DC FR2 non-exception test requirements in this specification with test procedures which refer appropriately back to TS 38.521-2 [9] for each NR band. Test only one EN-DC or NE-DC combination per FR2 band for each EN-DC or NE-DC configuration as defined in clause 5.5B of TS 38.101-3 [4] using LTE anchor agnostic approach.

c)	Test all the EN-DC or NE-DC FR1 non-exception test requirements in this specification with test procedures which refer appropriately back to TS 38.521-1 [8] for each NR band. Test only one EN-DC or NE-DC combination per FR1 band for each EN-DC or NE-DC configuration as defined in clause 5.5B of 38.101-3 [4] using LTE anchor agnostic approach.

(2)	The applicability and test coverage rules for Standalone (SA) and NSA capable devices shall include the following:

a)	For each NR band in a device, test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)	Test all the Standalone FR2 test requirements as per test procedures in TS 38.521-2 [9] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR2 test requirements for that NR band and need not be retested. If Standalone FR2 cannot be tested (due to test case not being complete), then test in EN-DC or NE-DC mode following (1) b) above.

c)	Test all the Standalone FR1 test requirements as per test procedures in TS 38.521-1 [8] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR1 test requirements for that NR band and need not be retested. If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), then test in EN-DC or NE-DC mode following (1)c) above.

### 4.5.1	Test coverage across 5G NR connectivity options

The test cases in this specification cover both NR/5GC (including FR1+FR2 CA or FR1+FR2 NR-DC) as well as EN-DC, NE-DC and NGEN-DC testing. Below shall be the understanding with respect to coverage across 5G NR connectivity options:

1)	Unless otherwise stated within the test case, it shall be understood that test requirements are agnostic of the EN-DC, NE-DC and NGEN-DC connectivity options configured within the test. The test coverage across the EN-DC, NE-DC and NGEN-DC connectivity options shall be considered fulfilled by executing the test case in one of these connectivity options. In the case of non-exception requirements, test coverage of NE-DC is fulfilled by execution of NR/5GC connectivity option test cases.

2)	EN-DC is the default connectivity option used for EN-DC, NE-DC and NGEN-DC test cases.

3)	If a UE does not support EN-DC, any other supported connectivity option of NE-DC or NGEN-DC can be configured to execute the test. The leverage rule detailed in (1) would apply.

Table 4.5.1-1: Void

Table 4.5.1-2: Void

## 4.6	E-UTRA configuration for EN-DC FR1 tests applying the E-UTRA anchor-agnostic approach

This clause applies to EN-DC test cases where E-UTRA anchor needs to be configured as per the anchor-agnostic approach outlined in clauses 6.1 and 7.1 of TS 38.101-3 [4].The LTE anchor-agnostic approach is defined as measurements on the NR carrier under conditions where the LTE anchor resources do not interfere with NR operation. The configuration defined in this clause ensures establishment of such conditions.

For baseline configuration, the E-UTRA carrier will be configured for each test case in clauses 6 and 7 as defined in the equivalent standalone E-UTRA test in TS 36.521-1 [10]. However, the below exceptions defined in Table 4.6-1, 4.6-2, 4.6-3, 4.6-4 and 4.6-5 are applied to ensure that the E-UTRA anchor resources do not interfere with NR operation.

For EN-DC within FR1 band combinations with multiple E-UTRA component carriers, it is sufficient to configure any one E-UTRA carrier from the carrier group whenever it is determined that anchor agnostic approach can be applied. Unless otherwise stated, the number of component carriers (CCs) listed in the test case titles of Clause 6 and clause 7 shall refer to the number of component carriers configured within the test case.

Table 4.6-1: E-UTRA configuration for EN-DC FR1 tests applying anchor agnostic approach

| Parameter | Value | Comments |
| --- | --- | --- |
| Test Frequency during and after connection setup | Mid (See Table 4.6-2) | As defined in TS 36.508 for the LTE band under test |
| Bandwidth during and after connection setup | 5 MHz (See Table 4.6-2) | Supported by all LTE bands. |
| DL signal levels during connection setup | RS EPRE -85.0 dBm/15kHz | DL physical channels as defined in Annex C0, C.1, C.2 and Annex C.3 of TS 36.521-1 [10]. TS 36.521-1 [10] annex C.0 defines the default DL power level of RS EPRE to be -85dBm/15kHz. |
| UL Signal levels during connection setup | PUSCH Power | Attained by enabling open loop power control and setting up UL signal levels according to Annex H.0, H.2 and H.3 of TS 36.521-1 [10]. |
| DL/UL RMC after connection setup | 0 RB allocation on both DL and UL (see Table 4.6-2) | Once the LTE link is established, then LTE Tx can be restricted by configuring 0 RB allocation on DL and UL. TimeAlignmentTimerDedicated IE to be set to infinity to ensure UE doesn't look for TA adjustments (See Table 4.6-5) |
| CQI Reports and SRS after connection setup | Disabled (See Table 4.6-3 and 4.6-4) | Disable periodic and aperiodic CQI reports to ensure none of these transmissions occur on the LTE uplink. Since LTE transmissions could easily exceed spurious emissions limits, tests that are intended to measure RF parameters on the NR should simply avoid LTE transmit altogether. |
| Number of OFDM symbols for PDCCH | 3 | The PCFICH carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe, as specified in TS 36.211 [8] clause 6.7 |

Table 4.6-2: E-UTRA Test Configuration Table

| E-UTRA Test Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency | Downlink |  | Uplink |  |
|  |  | Modulation | RB allocation | Modulation | RB allocation |
| 5 MHz2 | MidRange1 | N/A | 0 | N/A | 0 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [11] clause 4.3.1. NOTE 2: For EN-DC Intra-band tests that need to apply E-UTRA anchor agnostic approach, refer to and pick applicable E-UTRA channel bandwidth from clause 5.3B.1 and indicate within test case if it is different than 5 MHz. |  |  |  |  |  |

Table 4.6-3: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table 4.6-4: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-ConfigDedicated | Not present |  | RBC |
| } |  |  |  |

Table 4.6-5: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

## 4.7	E-UTRA configuration for EN-DC FR2 tests applying the E-UTRA anchor-agnostic approach

This clause applies to EN-DC test cases where E-UTRA anchor needs to be configured as per the anchor-agnostic approach outlined in clauses 6.1 and 7.1 of TS 38.101-3 [4].The LTE anchor-agnostic approach is defined as measurements on the NR carrier under conditions where the LTE anchor resources do not interfere with NR operation. The configuration defined in this clause ensures establishment of such conditions.

For baseline configuration, the E-UTRA carrier will be configured for each test case in clauses 6 and 7 as defined in the equivalent standalone E-UTRA test in TS 36.521-1 [10]. However, the below exceptions defined in Table 4.7-1 to 4.7-7 are applied to ensure that the E-UTRA anchor resources do not interfere with NR operation.

Since the E-UTRA link is always a functional link when testing EN-DC including FR2 band combinations, it is sufficient to configure any one E-UTRA carrier from the carrier group, irrespective of the number of E-UTRA carriers in the EN-DC combination under test. Unless otherwise stated, the number of component carriers (CCs) listed in the test case titles of Clause 6 and clause 7 shall refer to the number of component carriers configured within the test case.

Table 4.7-1: E-UTRA configuration for EN-DC FR2 tests applying anchor agnostic approach

| Parameter | Value | Comments |
| --- | --- | --- |
| Test Frequency during and after connection setup | Mid (See Table 4.7-2) | As defined in TS 36.508 for the LTE band under test |
| Bandwidth during and after connection setup | 5 MHz (See Table 4.7-2) | Supported by all LTE bands. |
| DL signal levels | See table 4.7-3 | DL physical channels as defined in Annex C0, C.1, C.2 and Annex C.3 of TS 36.521-1 [10]. |
| UL Signal levels for connection setup and UBF transmission | PUSCH Power | Attained by enabling open loop power control and setting up UL signal levels according to Annex H.0, H.2 and H.3 of TS 36.521-1 [10] with the exception for power control message exception defined in Table 4.7-5 |
| DL/UL RMC after connection setup except for UBF transmission | 0 RB allocation on both DL and UL (see Table 4.7-2) | Once the LTE link is established, then LTE Tx can be restricted by configuring 0 RB allocation on DL and UL. TimeAlignmentTimerDedicated IE to be set to infinity to ensure UE doesn't look for TA adjustments (See Table 4.7-7) |
| CQI Reports and SRS after connection setup | Disabled (See Table 4.7-4 and 4.7-6) | Disable periodic and aperiodic CQI reports to ensure none of these transmissions occur on the LTE uplink. Since LTE transmissions could easily exceed spurious emissions limits, tests that are intended to measure RF parametrics on the NR should simply avoid LTE transmit altogether. |
| Number of OFDM symbols for PDCCH | 3 | The PCFICH carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe, as specified in TS 36.211 [8] clause 6.7 |

Table 4.7-2: E-UTRA Test Configuration Table

| E-UTRA Test Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency | Downlink |  | Uplink |  |
|  |  | Modulation | RB allocation | Modulation | RB allocation |
| 5 MHz2 | MidRange1 | N/A | 0 | N/A | 0 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [11] clause 4.3.1 NOTE 2: For EN-DC Intra-band tests that need to apply E-UTRA anchor agnostic approach, refer to and pick applicable E-UTRA channel bandwidth from clause 5.3B.1 and indicate within test case if it is different than 5 MHz. |  |  |  |  |  |

Table 4.7-3: Default Downlink power levels for E-UTRA anchor

|  | Unit | Band Group | Channel Bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5MHz | 10MHz | 15 MHz | 20 MHz |
| RS EPRE | dBm/15kHz | FDD_A, TDD_A | N/A | N/A | ≥ -120.0 | N/A | N/A | N/A |
|  |  | FDD_B1, TDD_B1 | N/A | N/A | ≥ -119.5 | N/A | N/A | N/A |
|  |  | FDD_C, TDD_C | N/A | N/A | ≥ -119.0 | N/A | N/A | N/A |
|  |  | FDD_D, TDD_D | N/A | N/A | ≥ -118.5 | N/A | N/A | N/A |
|  |  | FDD_E, TDD_E | N/A | N/A | ≥ -118.0 | N/A | N/A | N/A |
|  |  | FDD_G, TDD_G | N/A | N/A | ≥ -117.0 | N/A | N/A | N/A |
|  |  | FDD_H, TDD_H | N/A | N/A | ≥ -116.5 | N/A | N/A | N/A |
|  |  | FDD_N, TDD_N | N/A | N/A | ≥ -113.5 | N/A | N/A | N/A |
| NOTE 1: The power level is specified at RSRP reference point as defined in TS 36.214 [24] NOTE 2: E-UTRA Band groups are defined in TS 36.133 [12] clause 3.5.1. |  |  |  |  |  |  |  |  |

Table 4.7-4: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table 4.7-5: UplinkPowerControlCommon-DEFAULT : Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, UplinkPowerControlCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalPUSCH | -60 (-60 dBm) | To attain maximum power from the DUT |  |
| } |  |  |  |

Table 4.7-6: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-ConfigDedicated | Not present |  | RBC |
| } |  |  |  |

Table 4.7-7: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

## 4.8	E-UTRA configuration for EN-DC FR1 tests not applying the E-UTRA anchor-agnostic approach

Unless otherwise stated, the following message exception defined in tables 4.8-1, 4.8-2 and 4.8-3 apply for EN-DC FR1 tests not applying the E-UTRA anchor-agnostic approach.

Table 4.8-1: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table 4.8-2: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-ConfigDedicated | Not present |  | RBC |
| } |  |  |  |

Table 4.8-3: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specifications are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The present specification covers band combinations including

-	at least one FR1 operating band and one FR2 operating band for carrier aggregation and dual connectivity operations;

-	at least one E-UTRA operating band for dual connectivity operations.

## 5.2	Operating bands

NR is designed to operate in FR1 operating bands defined in TS 38.101-1 [2] and FR2 operating bands defined in TS 38.101-2 [3]. E-UTRA is designed to operate in operating bands defined in TS 36.101 [4].

## 5.2A	Operating bands for CA

### 5.2A.1	Inter-band CA between FR1 and FR2

NR carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1. The band combinations include at least one FR1 operating band and one FR2 operating band.

If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

Table 5.2A.1-1: Band combinations for inter-band NR CA between FR1 and FR2

Editor’s note: No band combinations for inter-band NR CA between FR1 and FR2 specified due to the testability issues that combined testing of NR FR1 in conducted mode and NR FR2 in radiated mode.

## 5.2B	Operating bands for DC

### 5.2B.1	General

The operating bands are specified in clause 5.5B for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured.

### 5.2B.2 to 5.2B.7	Void

## 5.2E	Operating bands for V2X

### 5.2E.1	Intra-band V2X bands

NR V2X operation is designed to operate with E-UTRA sidelink in TDM mode on the operating bands combinations listed in Table 5.2E.1-1.

Table 5.2E.1-1: Intra-band V2X operating bands

| E-UTRA V2X-NR V2X Band combination | E-UTRA or NR Band | Interface |
| --- | --- | --- |
| V2X_47_n471 | 47 | PC5 |
|  | n47 | PC5 |
| NOTE 1: Only single switched SL is supported. |  |  |

### 5.2E.2	Inter-band V2X bands

NR V2X operation is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.2E.2-1.

Table 5.2E.2-1: Inter-band con-current V2X operating bands

| E-UTRA-NR V2X Band Combination | E-UTRA or NR Band | Interface |
| --- | --- | --- |
| V2X_20_n38 | 20 | Uu |
|  | n38 | PC5 |
| V2X_n71_47 | 47 | PC5 |
|  | n71 | Uu |
| V2X_n71_(n) 471 | 47 | PC5 |
|  | n47 | PC5 |
|  | n71 | Uu |
| NOTE 1: Only single switched SL in ITS band is supported. |  |  |

## 5.3	UE Channel bandwidth

## 5.3A	UE Channel bandwidth for CA

### 5.3A.1	Inter-band CA between FR1 and FR2

For inter-band NR CA between FR1 and FR2, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class as specified in clause 5.3A.5 of TS 38.101-1 [2] and clause 5.3A.4 of TS 38.101-2 [3] independently.

## 5.3B	UE Channel bandwidth for DC

### 5.3B.0	General

For intra-band contiguous EN-DC, the aggregated channel bandwidth is sum of the individual NR and E-UTRA channel bandwidths assuming nominal EN-DC channel with 0 kHz offset spacing as specified in clause 5.4.

ENBW = BWNR_Channel + BWE-UTRA_Channel

In the case where the NR sub-block and/or the E-UTRA sub-block itself is composed of intra-band contiguous CA carriers, the EN-DC aggregated channel bandwidth is the sum of the aggregated channel bandwidths of the NR and E-UTRA sub-blocks assuming nominal EN-DC channel spacing between the NR sub-block and E-UTRA sub-block.

ENBW = BWNR_Channel_CA + BWE-UTRA_Channel_CA

Intra-band contiguous EN-DC configurations are defined using intra-band contiguous EN-DC bandwidth class notation DC_(n)Xyz where the first EN-DC bandwidth class letter indicates the number of contiguous E-UTRA carriers and the second EN-DC bandwidth class letter indicates the number of contiguous NR carriers for the EN-DC combination of E-UTRA Band X and NR Band nX. Applicable contiguous intraband EN-DC bandwidth classes are listed in Table 5.3B.0-1

Table 5.3B.0-1: Intra-band contiguous EN-DC bandwidth classes

| Intra-band contiguous EN-DC bandwidth class | Number of contiguous CC |  |
| --- | --- | --- |
|  | E-UTRA | NR |
| AA | 1 | 1 |
| AB | 1 | 2 |
| CA | 2 | 1 |
| DA | 3 | 1 |

### 5.3B.1	Intra-band EN-DC in FR1

#### 5.3B.1.1	General

The requirements for intra-band EN-DC in this specification are defined for EN-DC configurations with associated bandwidth combination sets.

For each EN-DC configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

#### 5.3B.1.2	BCS for Intra-band contiguous EN-DC

For intra-band contiguous EN-DC, an EN-DC configuration is consisting of an E-UTRA band and a corresponding NR band having the same frequency range which supports an intra-band contiguous EN-DC bandwidth class.

Bandwidth combination sets for intra-band contiguous EN-DC are specified in Table 5.3B.1.2-1. The EN-DC configurations and bandwidth combination sets in Table 5.3B.1.2-1 also apply to higher order EN-DC combinations that include inter-band and intra-band EN-DC on the downlink and inter-band EN-DC on the uplink. If no BCS is reported in the UE capabilities for an intra-band combination the default is that the UE supports BCS0.

Table 5.3B.1.2-1: EN-DC configurations and bandwidth combination sets defined for intra-band contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)41AA | DC_(n)41AA | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
| DC_(n)41CA | DC_(n)41AA | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
| DC_(n)71AA | DC_(n)71AA | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: The channel bandwidths for E-UTRA or NR carrier should be at least supported in one of the BCS indicated in E-UTRA bandwidth combination sets or NR bandwidth combination sets if reported. |  |  |  |  |  |  |

#### 5.3B.1.3	BCS for Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC, an EN-DC configuration is consisting of an E-UTRA band and a corresponding NR band having the same frequency range which supports E-UTRA and NR carriers, where E-UTRA configuration is indicated by using E-UTRA CA bandwidth class as defined in TS 36.101 [5] and NR configuration is indicated by using NR CA bandwidth class as defined in TS 38.101-1 [2].

Requirements for intra-band non-contiguous EN-DC are defined for the EN-DC configurations and bandwidth combination sets specified in Table 5.3B.1.3-1. The EN-DC configurations and bandwidth combination sets in Table 5.3B.1.3-1 also apply to higher order EN-DC combinations that include inter-band and intra-band EN-DC on the downlink and inter-band EN-DC on the uplink.  If no BCS is reported in the UE capabilities for an intra-band combination the default is that the UE supports BCS0.

Table 5.3B.1.3-1: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_41A_n41A | DC_41A_n41A | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
| DC_41C_n41A | DC_41A_n41A | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
| DC_41D_n41A | DC_41A_n41A | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
| NOTE 1: Only single switched UL is supported in Rel-15. |  |  |  |  |  |  |

## 5.3E	UE Channel bandwidth for V2X

### 5.3E.0	General

The requirements specified in clause 5.3B are applicable to NR V2X UE.

### 5.3E.1	Intra-band contiguous V2X in FR1

For intra-band contiguous E-UTRA NR V2X UE, an EN-DC bandwidth class in Table 5.3B.0-1 are considered to specify the V2X transmission/reception configurations.

Bandwidth combination sets and V2X transmission/reception configurations for intra-band contiguous V2X UE are specified in Table 5.3E.1-1.

Table 5.3E.1-1: E-UTRA-NR V2X configurations and bandwidth combination sets for intra-band contiguous V2X UE

| V2X configuration | SL transmission band | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| V2X_(n)47AA | E-UTRA Band 47 or NR band n47 | 10 | 10,20,30,40 | 60 | 0 |
|  |  | 20 | 10,20,30,40 |  |  |

### 5.3E.2	Intra-band non-contiguous V2X in FR1

For intra-band non-contiguous E-UTRA NR V2X UE, an EN-DC bandwidth class in Table 5.3B.0-1 are considered to specify the V2X transmission/reception configurations.

Bandwidth combination sets and SL transmission/reception configurations for intra-band non-contiguous V2X are specified in Table 5.3E.2-1.

Table 5.3E.2-1: E-UTRA-NR V2X configurations and bandwidth combination sets for intra-band non-contiguous V2X UE

| V2X configuration | SL transmission band | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| V2X_47A_n47A | E-UTRA Band 47 or NR band n47 | 10 | 10,20,30,40 | 60 | 0 |
|  |  | 20 | 10,20,30,40 |  |  |

### 5.3E.3	Inter-band V2X in FR1

For inter-band E-UTRA NR V2X UE, the each channel bandwidth for inter-band V2X operations in FR1 is specified in TS 36.101 [4] and TS 38.101-1 [2], respectively.

## 5.4	Void

## 5.4A	Channel arrangement for CA

The channel arrangement for CA operations in FR1 and FR2 as specified in TS 38.101-1 [2] and TS 38.101-2 [3], respectively.

## 5.4B	Channel arrangement for DC

### 5.4B.0	General

The channel arrangement for intra-band EN-DC operations in FR1 is specified in TS 36.101 [5] and TS 38.101-1 [2], respectively.

### 5.4B.1	Channel spacing for intra-band EN-DC carriers

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between E-UTRA carrier and an adjacent NR carrier for intra-band contiguous EN-DC is defined as following:

-	For NR operating bands with 100 kHz channel raster,

Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2

-	For NR operating bands with 15 kHz channel raster,

Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-5kHz, 0kHz, 5kHz} for ∆FRaster equals to 15 kHz

Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-10kHz, 0kHz, 10kHz} for ∆FRaster equals to 30 kHz

where BWE-UTRA_Channel and BWNR_Channel are the channel bandwidths of the E-UTRA and NR carriers, ∆FRaster is the  band dependent channel raster granularity defined in TS38.101-1[2]. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

For intra-band non-contiguous EN-DC the channel spacing between E-UTRA and NR carriers shall be larger than the nominal channel spacing defined in this clause.

## 5.5	Configuration

## 5.5A	Configuration for CA

### 5.5A.1	Inter-band CA configurations between FR1 and FR2

Table 5.5A.1-1: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

Editor’s note: No Inter-band CA configurations between FR1 and FR2 (two bands) specified due to the testability issues that combined testing of NR FR1 in conducted mode and NR FR2 in radiated mode.

## 5.5B	Configuration for DC

### 5.5B.1	General

The operating bands and bandwidth classes are specified for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured. The EN-DC, NE-DC or NGEN-DC band combinations include at least one E-UTRA operating band.

For EN-DC or NE-DC configurations indicated by column "Single Uplink allowed" (e.g., problematic band combinations as defined in TS 38.306 [16]) in tables in this clause the UE may indicate capability of not supporting simultaneous dual and triple uplink operation due to possible intermodulation interference to its own primary downlink channel bandwidth of PCell or PSCell if the intermodulation order is 2 or if the intermodulation order is 3 for the combinations when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz.

In the case for EN-DC or NE-DC configurations listed in tables in this clause for which the intermodulation products caused by the dual and triple uplink operation fall into the receive band but do not interfere with its own primary downlink channel bandwidth of PCell or PSCell as defined in Annex I the UE is mandated to operate in dual and triple uplink mode. Single Uplink is also allowed for certain band combinations where intermodulation or reverse intermodulation products could create difficulty for meeting emission requirements.

For EN-DC combinations of order 3 or higher, "Single Uplink allowed" UL configurations captured in Table 5.5B.2-1, Table 5.5B.3-1, and Table 5.5B.4-1 apply.

If multiple UL DC configurations are listed for multiple DL DC configurations, valid uplink configurations are such that uplink does not have more carriers than downlink.

Non-contiguous resource allocation and almost contiguous allocation are not applicable for E-UTRA or NR carrier part of intra-band EN-DC configuration.

If the mandatory simultaneous Rx/Tx capability applies for a lower order DC configuration, when the applicable lower order DC configuration is a band pair in a higher order DC configuration, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order DC configuration.

For a higher order EN-DC band combination of which DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset, the frequency range in band n28/28 is restricted for the higher order band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.

For NR inter-band dual connectivity specified in 5.5B.7, the corresponding NR CA configurations in 5.5A.1, i.e., dual uplink inter-band carrier aggregation between FR1 and FR2 with uplink assigned to two NR bands, are applicable to Dual Connectivity.

NOTE 1:	Requirements for the dual connectivity configurations are defined in the clause corresponding NR uplink CA between FR1 and FR2 configurations, unless otherwise specified.

### 5.5B.2	Intra-band contiguous EN-DC

Supported channel bandwidths for E-UTRA operating bands are defined in TS 36.521-1 [10] and for NR operating bands in TS 38.521-1 [8].

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)41AA5 DC_(n)41CA5 | DC_(n)41AA | Yes3 |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. NOTE 5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |

### 5.5B.3	Intra-band non-contiguous EN-DC

Supported channel bandwidths for E-UTRA operating bands are defined in TS 36.521-1 [10] and for NR operating bands in TS 38.521-1 [8].

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_41A_n41A3 DC_41C_n41A3 DC_41D_n41A3 | DC_41A_n41A | Yes4 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported in Rel-15 NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Reserve. NOTE 7: Single UL allowed due to potential emission issues and self-interference. |  |  |

### 5.5B.4	Inter-band EN-DC within FR1

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.521-1 [10] and for NR operating bands and CA configurations in TS 38.521-1 [8] and present document.

#### 5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A | DC_1A_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A | DC_1A_n7A | No |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n41A7 | DC_1A_n41A | No |  |
| DC_1A_n77A7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7 | DC_1A_n78A | No | No |
| DC_1A-1A_n78A | DC_1A_n78A | No |  |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No | No |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A-2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n41A DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n77A | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n77(2A) | DC_2A_n77A | DC_2_n77 |  |
| DC_2A-2A_n77A21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77(2A) | DC_2A_n77A | DC_2_n77 |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n1A | DC_3A_n1A | DC_3_n1 |  |
| DC_3A_n5A | DC_3A_n5A | DC_3_n5 |  |
| DC_3A_n7A | DC_3A_n7A | No |  |
| DC_3A_n28A | DC_3A_n28A | No |  |
| DC_3A_n41A7 | DC_3A_n41A | DC_3_n41 | No |
| DC_3A_n77A7 | DC_3A_n77A | DC_3_n77 | No |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A | DC_3_n78 | No |
| DC_3A_n79A7 DC_3A_n79C7 | DC_3A_n79A | No | No |
| DC_5A_n2A | DC_5A_n2A | No |  |
| DC_5A_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A_n77A | DC_5A_n77A | No |  |
| DC_5A_n77(2A) | DC_5A_n77A | No |  |
| DC_5A_n78A7 DC_5A_n78C7 | DC_5A_n78A | No | No |
| DC_7A_n1A | DC_7A_n1A | No |  |
| DC_7A_n3A | DC_7A_n3A | No |  |
| DC_7A_n5A | DC_7A_n5A | DC_7_n5 |  |
| DC_7A_n28A | DC_7A_n28A | No |  |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |  |
| DC_7A_n71A | DC_7A_n71A | No |  |
| DC_7A_n77A7 | DC_7A_n77A | No |  |
| DC_7A_n78A7 DC_7C_n78A7 | DC_7A_n78A | No |  |
| DC_7A-7A_n78A7, 21 | DC_7A_n78A21 | No |  |
| DC_8A_n1A | DC_8A_n1A | No |  |
| DC_8A_n3A | DC_8A_n3A | No |  |
| DC_8A_n20A | DC_8A_n20A | Yes |  |
| DC_8A_n41A7 | DC_8A_n41A | No | No |
| DC_8A_n77A7 | DC_8A_n77A | No | No |
| DC_8A_n78A7 | DC_8A_n78A | No | No |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |  |
| DC_11A_n77A7 | DC_11A_n77A | No | No |
| DC_11A_n78A7 | DC_11A_n78A | No | No |
| DC_11A_n79A7 | DC_11A_n79A | No |  |
| DC_12A_n2A | DC_12A_n2A | No |  |
| DC_12A_n5A | DC_12A_n5A | No |  |
| DC_12A_n66A | DC_12A_n66A | No |  |
| DC_12A_n77A | DC_12A_n77A | DC_12_n77 |  |
| DC_12A_n77(2A) | DC_12A_n77A | DC_12_n77 |  |
| DC_12A_n78A | DC_12A_n78A | DC_12_n78 |  |
| DC_13A_n2A | DC_13A_n2A | No |  |
| DC_13A_n66A | DC_13A_n66A | No |  |
| DC_13A_n77A | DC_13A_n77A | No |  |
| DC_14A_n2A | DC_14A_n2A | No |  |
| DC_14A_n66A | DC_14A_n66A | No |  |
| DC_14A_n77A | DC_14A_n77A | No |  |
| DC_14A_n77(2A) | DC_14A_n77A | No |  |
| DC_18A_n77A7 | DC_18A_n77A | No | No |
| DC_18A_n78A7 | DC_18A_n78A | No | No |
| DC_18A_n79A7 | DC_18A_n79A | No |  |
| DC_19A_n1A | DC_19A_n1A | No |  |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |  |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No | No |
| DC_19A_n79A7 | DC_19A_n79A | No | No |
| DC_20A_n1A | DC_20A_n1A | No |  |
| DC_20A_n3A | DC_20A_n3A | No |  |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |
| DC_20A_n28A8,11,13 | DC_20A_n28A | No |  |
| DC_20A_n78A7 | DC_20A_n78A | No |  |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |  |
| DC_21A_n1A | DC_21A_n1A | No |  |
| DC_21A_n28A17 | DC_21A_n28A | DC_21_n28 |  |
| DC_21A_n77A7 | DC_21A_n77A | No |  |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No | No |
| DC_25A_n41A | DC_25A_n41A | No |  |
| DC_26A_n41A | DC_26A_n41A | No |  |
| DC_26A_n77A7 | DC_26A_n77A | No |  |
| DC_26A_n78A7 | DC_26A_n78A | No |  |
| DC_26A_n79A7 | DC_26A_n79A | No |  |
| DC_28A_n3A | DC_28A_n3A | No |  |
| DC_28A_n5A | DC_28A_n5A | No |  |
| DC_28A_n7A | DC_28A_n7A | No |  |
| DC_28A_n77A7 | DC_28A_n77A | No | No |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A | No | No |
| DC_28A_n79A7 | DC_28A_n79A | No |  |
| DC_30A_n2A | DC_30A_n2A | No |  |
| DC_30A_n5A | DC_30A_n5A | No |  |
| DC_30A_n66A | DC_30A_n66A | No |  |
| DC_30A_n77A | DC_30A_n77A | No |  |
| DC_30A_n77(2A) | DC_30A_n77A | No |  |
| DC_38A_n78A7 | DC_38A_n78A | No |  |
| DC_39A_n41A3 DC_39C_n41A3 | DC_39A_n41A DC_39C_n41A | No | No |
| DC_39A_n79A7 | DC_39A_n79A | No | No |
| DC_40A_n1A | DC_40A_n1A | No |  |
| DC_40A_n41A3 | DC_40A_n41A | No |  |
| DC_40A_n78A DC_40C_n78A | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n79A7,12 DC_40C_n79A7,12 | DC_40A_n79A | No | No |
| DC_41A_n28A7 | DC_41A_n28A | No |  |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A | No |  |
| DC_41A_n78A DC_41C_n78A | DC_41A_n78A | No |  |
| DC_41A_n79A6,7 | DC_41A_n79A | No | No |
| DC_42A_n77A3,4,9,11 DC_42C_n77A3,4,9,11 DC_42D_n77A3,4,9,11 | N/A | N/A |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42E_n78A3,4,9,11 | N/A | N/A |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42E_n79A9,15 | N/A | N/A |  |
| DC_48A_n46A | DC_48A_n46A | No |  |
| DC_48A_n5A | DC_48A_n5A | No |  |
| DC_48A_n66A | DC_48A_n66A | No |  |
| DC_66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |  |
| DC_66A_n41A | DC_66A_n41A | No |  |
| DC_66A_n71A | DC_66A_n71A | No |  |
| DC_66A_n77A | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A_n77(2A) | DC_66A_n77A | DC_66_n77 |  |
| DC_66A-66A_n77A21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77(2A) | DC_66A_n77A | DC_66_n77 |  |
| DC_66A_n78A | DC_66A_n78A | No |  |
| DC_71A_n2A | DC_71A_n2A | No |  |
| DC_71A_n66A | DC_71A_n66A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured PCell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addition to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. NOTE 9: The combination is not used alone as fallback mode of other band combinations in which UL in Band 42 is not used. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 of 38.101-3 [4] apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [25] and clause 7.32.2.12 of 36.133 [12] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: Reserved. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1. NOTE 18: Reserved. NOTE 19: Reserved. NOTE 20: Reserved. NOTE 21: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. |  |  |  |

#### 5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A_n41A5 | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A_n77A5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5DC_1A-3C_n78A5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3C_n78(2A)5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n79A5 | DC_1A_n79A DC_3A_n79A |
| DC_1A-5A_n78A5 DC_1A-5A_n78C5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |
| DC_1A-7A_n3A | DC_1A_n3A DC_7A_n3A |
| DC_1A-7A_n28A5 | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A_n78A5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n78(2A)5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-19A_n77A5 | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A_n78A5 DC_1A-19A_n78C5 | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A_n79A5 DC_1A-19A_n79C5 | DC_1A_n79A DC_19A_n79A |
| DC_1A-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A6,11,12 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n77A5 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A_n78A5 DC_1A-21A_n78C5 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A_n79A5 DC_1A-21A_n79C5 | DC_1A_n79A DC_21A_n79A |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-41A_n28A5 DC_1A-41C_n28A5 | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-41A_n41A | DC_1A_n41A |
| DC_1A-41A_n77A | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n78A | DC_1A_n78A DC_41A_n78A |
| DC_1A-42A_n77A10,11 DC_1A-42C_n77A10,11 DC_1A-42D_n77A10,11 | DC_1A_n77A |
| DC_1A-42A_n78A10,11 DC_1A-42A_n78C10,11 DC_1A-42C_n78A10,11 DC_1A-42C_n78C10,11 DC_1A-42D_n78A10,11 DC_1A-42E_n78A10,11 | DC_1A_n78A |
| DC_1A-42A_n79A DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42E_n79A | DC_1A_n79A |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_2A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-13A_n77A | DC_2A_n77A DC_13A_n77A |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A_n77A | DC_2A_n77A DC_66A_n77A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_2A_n5A-n77A14 | DC_2A_n5A DC_2A_n77A14 |
| DC_3A_n1A-n78A5 | DC_3A_n1A DC_3A_n78A |
| DC_3A_n1A-n79A5 | DC_3A_n1A DC_3A_n79A |
| DC_3A-5A_n78A5 DC_3A-5A_n78C5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-7A_n5A | DC_3A_n5A DC_7A_n5A |
| DC_3A-7A_n28A | DC_3A_n28A DC_7A_n28A |
| DC_3A-7A_n78A5 DC_3C-7A_n78A5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n78A5 | DC_3A_n78A DC_8A_n78A |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-8A_n78(2A) | DC_3A_n78A DC_8A_n78A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-19A_n77A5 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A_n78A5 DC_3A-19A_n78C5 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A_n79A5 DC_3A-19A_n79C5 | DC_3A_n79A DC_19A_n79A |
| DC_3A-20A_n1A | DC_3A_n1A DC_20A_n1A |
| DC_3A-20A_n28A5,6,11,12 | DC_3A_n28A DC_20A_n28A |
| DC_3A-20A_n78A5 | DC_3A_n78A DC_20A_n78A |
| DC_3A-21A_n77A5 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A_n78A5 DC_3A-21A_n78C5 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A_n79A5 DC_3A-21A_n79C5 | DC_3A_n79A DC_21A_n79A |
| DC_3A-28A_n78A5 | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5 | DC_3A_n28A DC_3A_n78A |
| DC_3A-40A_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A-41A_n28A5 DC_3A-41C_n28A5 | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A | DC_3A_n41A |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n77(2A) | DC_3A_n77A DC_41A_n77A |
| DC_3A-42A_n77A10,11 DC_3A-42C_n77A10,11 DC_3A-42D_n77A10,11 | DC_3A_n77A |
| DC_3A-42A_n78A10,11 DC_3A-42A_n78C10,11 DC_3A-42C_n78A10,11 DC_3A-42C_n78C10,11 DC_3A-42D_n78A10,11 DC_3A-42E_n78A10,11 | DC_3A_n78A |
| DC_3A-42A_n79A DC_3A-42A_n79C DC_3A-42C_n79A DC_3A-42C_n79C DC_3A-42D_n79A DC_3A-42E_n79A | DC_3A_n79A |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_3A_n78A-n79A | DC_3A_n78A DC_3A_n79A |
| DC_5A-7A_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_7A_n5A-n78A DC_7C_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |
| DC_7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A-20A_n1A | DC_7A_n1A DC_20A_n1A |
| DC_7A-20A_n3A | DC_7A_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6,11,12 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-28A_n3A | DC_7A_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 | DC_7A_n5A DC_28A_n5A |
| DC_7A-28A_n78A5 | DC_7A_n78A DC_28A_n78A |
| DC_7A_n28A-n78A5 DC_7C_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_13A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A_n77A | DC_13A_n77A DC_66A_n77A |
| DC_13A_n2A-n77A14 | DC_13A_n2A DC_13A_n77A14 |
| DC_18A-41A_n3A | DC_18A_n3A DC_41A_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_19A_n1A-n78A5 | DC_19A_n1A DC_19A_n78A |
| DC_19A_n1A-n79A5 | DC_19A_n1A DC_19A_n79A |
| DC_19A-21A_n77A5 | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A_n78A5 DC_19A-21A_n78C5 | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A_n79A5 DC_19A-21A_n79C5 | DC_19A_n79A DC_21A_n79A |
| DC_19A-42A_n77ADC_19A-42C_n77A | DC_19A_n77A |
| DC_19A-42A_n78A DC_19A-42C_n78A DC_19A-42A_n78C DC_19A-42C_n78C | DC_19A_n78A |
| DC_19A-42A_n79A DC_19A-42C_n79A DC_19A-42A_n79C DC_19A-42C_n79C | DC_19A_n79A |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_20A_n28A-n78A5,6,11,12 | DC_20A_n28A DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_21A_n1A-n78A5 | DC_21A_n1A DC_21A_n78A |
| DC_21A_n1A-n79A5 | DC_21A_n1A DC_21A_n79A |
| DC_21A-42A_n77A10,11 DC_21A-42C_n77A10,11 | DC_21A_n77A |
| DC_21A-42A_n78A10,11 DC_21A-42A_n78C10,11 DC_21A-42C_n78A10,11 DC_21A-42C_n78C10,11 | DC_21A_n78A |
| DC_21A-42A_n79A DC_21A-42A_n79C DC_21A-42C_n79A DC_21A-42C_n79C | DC_21A_n79A |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A-42A_n78A10,11 | DC_28A_n78A |
| DC_41A-42A_n78A10,11 | DC_41A_n78A |
| DC_66A_n2A-n77A14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A_n5A-n77A14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A-(n)71AA | DC_66A_n71A DC_(n)71AA |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported. NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured PCell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 7: Void. NOTE 8: Reserved. NOTE 9: Reserved. NOTE 10: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 12: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 13: Reserved. NOTE 14: Minimum requirements for PC2 are applicable for this uUplink EN-DC configuration in this downlink/uplink to EN-DC configuration. |  |

#### 5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A |
| DC_1A-3A-7A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-8A_n78A2 | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A-19A_n77A2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n78A2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n79A2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n28A3,7,8 | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-21A_n77A2 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A_n78A2 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A_n79A2 | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-3A-28A_n78A2 | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A |
| DC_1A-3A-42A_n78A6,7 DC_1A-3A-42C_n78A6,7DC_1A-3A-42D_n78A6,7 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-42A_n79A DC_1A-3A-42C_n79ADC_1A-3A-42D_n79A | DC_1A_n79A DC_3A_n79A |
| DC_1A-7A-20A_n28A3,7,8 | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A_n78A2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-28A_n78A | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A |
| DC_1A-19A-21A_n77A | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n78A2 DC_1A-19A-21A_n78C2 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n79A DC_1A-19A-21A_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-19A-42A_n77A6,7 DC_1A-19A-42C_n77A6,7 | DC_19A_n77A |
| DC_1A-19A-42A_n78A DC_1A-19A-42A_n78C DC_1A-19A-42C_n78A DC_1A-19A-42C_n78C | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A-42A_n79A DC_1A-19A-42A_n79C DC_1A-19A-42C_n79A DC_1A-19A-42C_n79C | DC_1A_n79A DC_19A_n79A |
| DC_1A-20A_n28A-n78A2,3,7,8 | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-21A-42A_n77A6,7 DC_1A-21A-42C_n77A6,7 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A-42A_n78A6,7 DC_1A-21A-42A_n78C6,7 DC_1A-21A-42C_n78A6,7 DC_1A-21A-42C_n78C6,7 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A-42A_n79A DC_1A-21A-42A_n79C DC_1A-21A-42C_n79A DC_1A-21A-42C_n79C | DC_1A_n79A DC_21A_n79A |
| DC_2A-7A-13A_n66A DC_2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7C-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-13A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-14A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-66A_n5A-n77A9 | DC_2A_n5A DC_2A_n77A9 DC_66A_n5A DC_66A_n77A9 |
| DC_2A-66A-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA |
| DC_3A-7A-20A_n8A | DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_3A-7A-20A_n28A3,7,8 | DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_3A-7A-20A_n78A2 | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-28A_n78A2 | DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A_n28A-n78A2 | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A |
| DC_3A-19A-21A_n78A2 DC_3A-19A-21A_n78C2 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A_n79A2 DC_3A-19A-21A_n79C2 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n77A6,7 DC_3A-19A-42C_n77A6,7 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A-42A_n78A6,7 DC_3A-19A-42A_n78C6,7 DC_3A-19A-42C_n78A6,7 DC_3A-19A-42C_n78C6,7 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A-42A_n79A2 DC_3A-19A-42A_n79C2 DC_3A-19A-42C_n79A2 DC_3A-19A-42C_n79C2 | DC_3A_n79A DC_19A_n79A |
| DC_3A-20A_n28A-n78A2,3,7,8 | DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-21A-42A_n77A6,7 DC_3A-21A-42C_n77A6,7 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A-42A_n78A6,7 DC_3A-21A-42A_n78C6,7 DC_3A-21A-42C_n78A6,7 DC_3A-21A-42C_n78C6,7 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A-42A_n79A DC_3A-21A-42A_n79C DC_3A-21A-42C_n79A DC_3A-21A-42C_n79C | DC_3A_n79A DC_21A_n79A |
| DC_7A-20A_n28A-n78A2,3,7,8 | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_13A-66A_n2A-n77A9 | DC_13A_n2A DC_13A_n77A9 DC_66A_n2A DC_66A_n77A9 |
| DC_19A-21A-42A_n77A6,7 DC_19A-21A-42C_n77A6,7 | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A-42A_n78A6,7 DC_19A-21A-42A_n78C6,7 DC_19A-21A-42C_n78A6,7 DC_19A-21A-42C_n78C6,7 | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A-42A_n79A DC_19A-21A-42A_n79C DC_19A-21A-42C_n79A DC_19A-21A-42C_n79C | DC_19A_n79A DC_21A_n79A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: UL carrier shall be supported in Band 2 or band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 9: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. |  |

#### 5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A-20A_n28A3,6,7 | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-3A-7A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A |
| DC_1A-3A-7A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-19A-42A_n78A5,6 DC_1A-3A-19A-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A-42A_n79A DC_1A-3A-19A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n28A-n78A2,3,6,7 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-21A-42A_n78A5,6 DC_1A-3A-21A-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A-42A_n79A DC_1A-3A-21A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-7A-20A_n28A-n78A2,3,6,7 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-19A-21A-42A_n78A5,6 DC_1A-19A-21A-42A_n78C5,6 DC_1A-19A-21A-42C_n78A5,6 DC_1A-19A-21A-42C_n78C5,6 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A-42A_n79A DC_1A-19A-21A-42A_n79C DC_1A-19A-21A-42C_n79A DC_1A-19A-21A-42C_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-7A-20A_n28A-n78A2,3,6,7 | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-19A-21A-42A_n78A5,6 DC_3A-19A-21A-42C_n78A5,6 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42A_n79A5,6 DC_3A-19A-21A-42C_n79A5,6 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 4: Reserved. NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

#### 5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A-20A_n28A-n78A2,3,5,6 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Reserved NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

### 5.5B.4a	Inter-band NE-DC within FR1

#### 5.5B.4a.1	Inter-band NE-DC configurations within FR1 (two bands)

Table 5.5B.4a.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_n28A_3A DC_n28A_3C | DC_n28A_3A | No |
| DC_n28A_39A DC_n28A_39C | DC_n28A_39A | No |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |  |

### 5.5B.5	Inter-band EN-DC including FR2

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.521-1 [10] and for NR operating bands and CA configurations in TS 38.521-1 [8], TS 38.521-2 [9] and present document.

#### 5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A | DC_1A_n257A |
| DC_2A_n257A | DC_2A_n257A |
| DC_2A_n257(2A) | DC_2A_n257A |
| DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2C_n260A | DC_2A_n260A DC_2A_n260G DC_2A_n260H |
| DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261M | DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A_n261(2A) DC_2A_n261(3A) | DC_2A_n261A |
| DC_2A_n261(2G) DC_2A_n261(2H) | DC_2A_n261A DC_2A_n261G DC_2A_n261H |
| DC_2A_n261(A-G) DC_2A_n261(A-2G) DC_2A_n261(A-H) DC_2A_n261(A-I) DC_2A_n261(G-H) DC_2A_n261(G-I) DC_2A_n261(H-I) DC_2A_n261(2A-G) DC_2A_n261(2A-I) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A_n261(A-G-H) DC_2A_n261(A-G-I) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_3A_n257G DC_3A_n257H DC_3A_n257I | DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260ODC_5B_n260A | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260O DC_5A_n260P DC_5A_n260Q |
| DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261M | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_5A_n261(2A) DC_5A_n261(3A) DC_5A_n261(4A) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_8A_n257A DC_8A_n257D DC_8A_n257E DC_8A_n257F DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K DC_8A_n257L DC_8A_n257M | DC_8A_n257A |
| DC_13A_n257A | DC_13A_n257A |
| DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260M | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260O |
| DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I |
| DC_18A_n257A DC_18A_n257I | DC_18A_n257A DC_18A_n257I |
| DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_20A_n257A | DC_20A_n257A |
| DC_21A_n257A DC_21A_n257G | DC_21A_n257A DC_21A_n257G |
| DC_66A_n260A DC_66A_n260D DC_66A_n260E DC_66A_n260F DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A-66A_n260A DC_66A-66A_n260G DC_66A-66A_n260H DC_66A-66A_n260I DC_66A-66A_n260J DC_66A-66A_n260K DC_66A-66A_n260L DC_66A-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I |
| DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A_n261(2G) DC_66A_n261(2H) | DC_66A_n261A DC_66A_n261G DC_66A_n261H |
| DC_66A_n261(A-2G) DC_66A_n261(2A-G) DC_66A_n261(2A-H) DC_66A_n261(2A-I) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A_n261(A-G-H) DC_66A_n261(A-G-I) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A-66A_n260G DC_66A-66A_n260H DC_66A-66A_n260I DC_66A-66A_n260J DC_66A-66A_n260K DC_66A-66A_n260L DC_66A-66A_n260M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations |  |

#### 5.5B.5.2	Inter-band EN-DC configurations including FR2 (three bands)

Table 5.5B.5.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n257A2 DC_1A-3A_n257G DC_1A-3A_n257H DC_1A-3A_n257I | DC_1A_n257A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_1A-18A_n257A2 DC_1A-18A_n257I | DC_1A_n257A DC_1A_n257I DC_18A_n257A DC_18A_n257I |
| DC_1A-19A_n257A2 DC_1A-19A_n257G DC_1A-19A_n257H DC_1A-19A_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n257A |
| DC_1A-21A_n257A2 DC_1A-21A_n257G DC_1A-21A_n257H DC_1A-21A_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-41A_n257A DC_1A-41A_n257I | DC_1A_n257A DC_1A_n257I DC_41A_n257A DC_41A_n257I |
| DC_1A-42A_n257A DC_1A-42A_n257G DC_1A-42A_n257H DC_1A-42A_n257I DC_1A-42C_n257A DC_1A-42D_n257A DC_1A-42D_n257G DC_1A-42D_n257H DC_1A-42D_n257I DC_1A-42E_n257A DC_1A-42E_n257G DC_1A-42E_n257H DC_1A-42E_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A |
| DC_2A-5A_n257A2 | DC_2A_n257A DC_5A_n257A |
| DC_2A-5A_n260A DC_2A-5A_n260G DC_2A-5A_n260H DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M | DC_2A_n260A DC_5A_n260A |
| DC_2A-5A_n261A DC_2A-5A_n261G DC_2A-5A_n261H DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M | DC_2A_n261A DC_5A_n261A DC_2A_n261G DC_5A_n261G DC_2A_n261H DC_5A_n261H DC_2A_n261I DC_5A_n261I |
| DC_2A-13A_n260A2 | DC_2A_n260A DC_13A_n260A |
| DC_2A-13A_n261A DC_2A-13A_n261G DC_2A-13A_n261H DC_2A-13A_n261I DC_2A-13A_n261J DC_2A-13A_n261K DC_2A-13A_n261L DC_2A-13A_n261M | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H |
| DC_2A-13A_n261(2A) DC_2A-13A_n261(2G) DC_2A-13A_n261(2H) DC_2A-13A_n261(3A) | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H |
| DC_2A-13A_n261(A-G) DC_2A-13A_n261(A-2G) DC_2A-13A_n261(A-H) DC_2A-13A_n261(A-I) DC_2A-13A_n261(G-H) DC_2A-13A_n261(G-I) DC_2A-13A_n261(H-I) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(2A-I) | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H |
| DC_2A-13A_n261(A-G-H) DC_2A-13A_n261(A-G-I) | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H |
| DC_2A-14A_n260A DC_2A-14A_n260G DC_2A-14A_n260H DC_2A-14A_n260I | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I |
| DC_2A-2A-14A_n260A DC_2A-2A-14A_n260G DC_2A-2A-14A_n260H DC_2A-2A-14A_n260I | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I |
| DC_2A-30A_n260A DC_2A-30A_n260G DC_2A-30A_n260H DC_2A-30A_n260I DC_2A-30A_n260J DC_2A-30A_n260K DC_2A-30A_n260L DC_2A-30A_n260M | DC_2A_n260A DC_30A_n260A |
| DC_2A-66A_n257A2 | DC_2A_n257A DC_66A_n257A |
| DC_2A-66A_n260A DC_2A-66A_n260G DC_2A-66A_n260H DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I |
| DC_3A-18A_n257A DC_3A-18A_n257I | DC_3A_n257A DC_3A_n257I DC_18A_n257A DC_18A_n257I |
| DC_3A-19A_n257A2 DC_3A-19A_n257D2 DC_3A-19A_n257G DC_3A-19A_n257H DC_3A-19A_n257I | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n257A DC_19A_n257D |
| DC_3A-21A_n257A2 DC_3A-21A_n257D2 DC_3A-21A_n257G DC_3A-21A_n257H DC_3A-21A_n257I | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n257A DC_21A_n257D |
| DC_3A-41A_n257A DC_3A-41A_n257I | DC_3A_n257A DC_3A_n257I DC_41A_n257A DC_41A_n257I |
| DC_3A-42A_n257A2 DC_3A-42A_n257D2 DC_3A-42A_n257G DC_3A-42A_n257H DC_3A-42A_n257I DC_3A-42C_n257A2 DC_3A-42C_n257D2 DC_3A-42C_n257G DC_3A-42C_n257H DC_3A-42C_n257I DC_3A-42D_n257A2 DC_3A-42D_n257G DC_3A-42D_n257H DC_3A-42D_n257I DC_3A-42E_n257A2 DC_3A-42E_n257G DC_3A-42E_n257H DC_3A-42E_n257I | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257D |
| DC_5A-7A_n257A2 | DC_5A_n257A DC_7A_n257A |
| DC_5A-30A_n260A DC_5A-30A_n260G DC_5A-30A_n260H DC_5A-30A_n260I DC_5A-30A_n260J DC_5A-30A_n260K DC_5A-30A_n260L DC_5A-30A_n260M | DC_5A_n260A DC_30A_n260A |
| DC_5A-66A_n257A | DC_5A_n257A DC_66A_n257A |
| DC_5A-66A_n260A DC_5A-66A_n260G DC_5A-66A_n260H DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M | DC_5A_n260A DC_66A_n260A |
| DC_12A-30A_n260A DC_12A-30A_n260G DC_12A-30A_n260H DC_12A-30A_n260I DC_12A-30A_n260J DC_12A-30A_n260K DC_12A-30A_n260L DC_12A-30A_n260M | DC_12A_n260A DC_30A_n260A |
| DC_12A-66A_n260A DC_12A-66A_n260G DC_12A-66A_n260H DC_12A-66A_n260I DC_12A-66A_n260J DC_12A-66A_n260K DC_12A-66A_n260L DC_12A-66A_n260M | DC_12A_n260A DC_66A_n260A |
| DC_13A-66A_n260A2 | DC_13A_n260A DC_66A_n260A |
| DC_14A-30A_n260A DC_14A-30A_n260G DC_14A-30A_n260H DC_14A-30A_n260I | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I |
| DC_14A-66A_n260A DC_14A-66A_n260G DC_14A-66A_n260H DC_14A-66A_n260I | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I |
| DC_14A-66A-66A_n260A DC_14A-66A-66A_n260G DC_14A-66A-66A_n260H DC_14A-66A-66A_n260I | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I |
| DC_19A-21A_n257A2 DC_19A-21A_n257D2 DC_19A-21A_n257G DC_19A-21A_n257H DC_19A-21A_n257I | DC_19A_n257A DC_19A_n257D DC_21A_n257A DC_21A_n257D DC_21A_n257G |
| DC_19A-42A_n257A2 DC_19A-42A_n257D2 DC_19A-42A_n257G2 DC_19A-42A_n257H2 DC_19A-42A_n257I2 DC_19A-42C_n257A2 DC_19A-42C_n257G2 DC_19A-42C_n257H2 DC_19A-42C_n257I2 | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-42A_n257A2 DC_21A-42A_n257D2 DC_21A-42A_n257G DC_21A-42A_n257H DC_21A-42A_n257I DC_21A-42C_n257A2 DC_21A-42C_n257G DC_21A-42C_n257H DC_21A-42C_n257I | DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257D |
| DC_28A-42A_n257A2 | DC_28A_n257A DC_42A_n257A |
| DC_30A-66A_n260A DC_30A-66A_n260G DC_30A-66A_n260H DC_30A-66A_n260I DC_30A-66A_n260J DC_30A-66A_n260K DC_30A-66A_n260L DC_30A-66A_n260M | DC_30A_n260A DC_66A_n260A |
| DC_41A-42A_n257A | DC_41A_n257A DC_42A_n257A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. |  |

#### 5.5B.5.3	Inter-band EN-DC configurations including FR2 (four bands)

Table 5.5B.5.3-1: Inter-band EN-DC configurations including FR2 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-18A_n257A DC_1A-3A-18A_n257I | DC_1A_n257A DC_1A_n257I DC_3A_n257A DC_3A_n257I DC_18A_n257A DC_18A_n257I |
| DC_1A-3A-19A_n257A2 DC_1A-3A-19A_n257G DC_1A-3A-19A_n257H DC_1A-3A-19A_n257I | DC_1A_n257A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n257A |
| DC_1A-3A-21A_n257A2 DC_1A-3A-21A_n257G DC_1A-3A-21A_n257H DC_1A-3A-21A_n257I | DC_1A_n257A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n257A |
| DC_1A-3A-41A_n257A DC_1A-3A-41A_n257I | DC_1A_n257A DC_1A_n257I DC_3A_n257A DC_3A_n257I DC_41A_n257A DC_41A_n257I |
| DC_1A-3A-42A_n257A DC_1A-3A-42A_n257G DC_1A-3A-42A_n257H DC_1A-3A-42A_n257I DC_1A-3A-42C_n257A DC_1A-3A-42C_n257G DC_1A-3A-42C_n257H DC_1A-3A-42C_n257I DC_1A-3A-42D_n257G DC_1A-3A-42D_n257H DC_1A-3A-42D_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_1A-19A-21A_n257G DC_1A-19A-21A_n257H DC_1A-19A-21A_n257I | DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-19A-42A_n257A DC_1A-19A-42A_n257G DC_1A-19A-42A_n257H DC_1A-19A-42A_n257I DC_1A-19A-42C_n257A DC_1A-19A-42C_n257G DC_1A-19A-42C_n257H DC_1A-19A-42C_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n257A DC_42A_n257A |
| DC_1A-21A-42A_n257A DC_1A-21A-42A_n257G DC_1A-21A-42A_n257H DC_1A-21A-42A_n257I DC_1A-21A-42C_n257A DC_1A-21A-42C_n257G DC_1A-21A-42C_n257H DC_1A-21A-42C_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257D |
| DC_2A-14A-30A_n260A DC_2A-14A-30A_n260G DC_2A-14A-30A_n260H DC_2A-14A-30A_n260I | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I |
| DC_2A-14A-66A_n260A DC_2A-14A-66A_n260G DC_2A-14A-66A_n260H DC_2A-14A-66A_n260I | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I |
| DC_2A-14A-66A-66A_n260A DC_2A-14A-66A-66A_n260G DC_2A-14A-66A-66A_n260H DC_2A-14A-66A-66A_n260I | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I |
| DC_3A-19A-21A_n257A2 | DC_3A_n257A DC_19A_n257A DC_21A_n257A |
| DC_3A-19A-42A_n257A DC_3A-19A-42A_n257G DC_3A-19A-42A_n257H DC_3A-19A-42A_n257I DC_3A-19A-42C_n257A DC_3A-19A-42C_n257D DC_3A-19A-42C_n257G DC_3A-19A-42C_n257H DC_3A-19A-42C_n257I | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_3A-21A-42A_n257A DC_3A-21A-42A_n257G DC_3A-21A-42A_n257H DC_3A-21A-42A_n257I DC_3A-21A-42C_n257A DC_3A-21A-42C_n257D DC_3A-21A-42C_n257G DC_3A-21A-42C_n257H DC_3A-21A-42C_n257I | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_14A-30A-66A_n260A DC_14A-30A-66A_n260G DC_14A-30A-66A_n260H DC_14A-30A-66A_n260I | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I |
| DC_14A-30A-66A-66A_n260A DC_14A-30A-66A-66A_n260G DC_14A-30A-66A-66A_n260H DC_14A-30A-66A-66A_n260I | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I |
| DC_19A-21A-42A_n257A2 DC_19A-21A-42A_n257G2 DC_19A-21A-42A_n257H2 DC_19A-21A-42A_n257I2 DC_19A-21A-42C_n257A2 DC_19A-21A-42C_n257D2 DC_19A-21A-42C_n257G2 DC_19A-21A-42C_n257H2 DC_19A-21A-42C_n257I2 | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. |  |

#### 5.5B.5.4	Inter-band EN-DC configurations including FR2 (five bands)

Table 5.5B.5.4-1: Inter-band EN-DC configurations including FR2 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-19A-42A_n257A DC_1A-3A-19A-42A_n257G DC_1A-3A-19A-42A_n257H DC_1A-3A-19A-42A_n257I DC_1A-3A-19A-42C_n257A DC_1A-3A-19A-42C_n257G DC_1A-3A-19A-42C_n257H DC_1A-3A-19A-42C_n257I | DC_1A_n257A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n257A DC_42A_n257A |
| DC_1A-3A-21A-42A_n257A DC_1A-3A-21A-42C_n257A DC_1A-3A-21A-42C_n257G DC_1A-3A-21A-42C_n257H DC_1A-3A-21A-42C_n257I | DC_1A_n257A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_21A_n257A DC_42A_n257A |
| DC_1A-19A-21A-42A_n257A DC_1A-19A-21A-42A_n257G DC_1A-19A-21A-42A_n257H DC_1A-19A-21A-42A_n257I DC_1A-19A-21A-42C_n257A DC_1A-19A-21A-42C_n257G DC_1A-19A-21A-42C_n257H DC_1A-19A-21A-42C_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n257A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A |
| DC_2A-14A-30A-66A_n260A DC_2A-14A-30A-66A_n260G DC_2A-14A-30A-66A_n260H DC_2A-14A-30A-66A_n260I | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |

#### 5.5B.5.5	Void

### 5.5B.6	Inter-band EN-DC including FR1 and FR2

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.521-1 [10] and for NR operating bands and CA configurations in TS 38.521-1 [8], TS 38.521-2 [9] and present document.

#### 5.5B.6.1	Void

#### 5.5B.6.2	Inter-band EN-DC configurations including FR1 and FR2 (three bands)

Table 5.5B.6.2-1: Inter-band EN-DC configurations including FR1 and FR2 (three bands)

Editor’s note:	No Inter-band EN-DC configurations including FR1 and FR2 (three bands) specified due to the testability issues that combined testing of NR FR1 in conducted mode and NR FR2 in radiated mode.

#### 5.5B.6.3	Inter-band EN-DC configurations including FR1 and FR2 (four bands)

Table 5.5B.6.3-1: Inter-band EN-DC configurations including FR1 and FR2 (four bands)

Editor’s note: No Inter-band EN-DC configurations including FR1 and FR2 (four bands) specified due to the testability issues that combined testing of NR FR1 in conducted mode and NR FR2 in radiated mode.

#### 5.5B.6.4	Inter-band EN-DC configurations including FR1 and FR2 (five bands)

Table 5.5B.6.4-1: Inter-band EN-DC configurations including FR1 and FR2 (five bands)

Editor’s note: No Inter-band EN-DC configurations including FR1 and FR2 (five bands) specified due to the testability issues that combined testing of NR FR1 in conducted mode and NR FR2 in radiated mode.

#### 5.5B.6.5	Inter-band EN-DC configurations including FR1 and FR2 (six bands)

Table 5.5B.6.5-1: Inter-band EN-DC configurations including FR1 and FR2 (six bands)

Editor’s note: No Inter-band EN-DC configurations including FR1 and FR2 (six bands) specified due to the testability issues that combined testing of NR FR1 in conducted mode and NR FR2 in radiated mode.

### 5.5B.7	Inter-band NR-DC between FR1 and FR2

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.521-1 [10] and for NR operating bands and CA configurations in TS 38.521-1 [8], TS 38.521-2 [9] and present document.

#### 5.5B.7.0	General

The configurations and bandwidth combination sets for the FR1-FR2 NR-DC combinations in the following sub-section are defined in the tables for FR1-FR2 carrier aggregation in section 5.5A.1.

#### 5.5B.7.1	Inter-band NR-DC configurations between FR1 and FR2 (two bands)

Table 5.5B.7-1: Inter-band NR-DC configurations between FR1 and FR2 (two bands)

Editor’s note: No Inter-band NR-DC configurations between FR1 and FR2 (two bands) specified due to the testability issues that combined testing of NR FR1 in conducted mode and NR FR2 in radiated mode.

## 5.5E	Configuration for V2X operation

### 5.5E.1	General

The operating bands and bandwidth classes are specified for V2X operation.

### 5.5E.2	Intra-band contiguous V2X operation in FR1

Table 5.5E.2-1: Intra-band contiguous V2X configurations

| V2X configuration | SL transmission |
| --- | --- |
| V2X_(n)47AA | E-UTRA Band 47 or NR band n47 |
| NOTE 1: Only single switched SL is supported. |  |

### 5.5E.3	Intra-band non-contiguous V2X operation in FR1

Table 5.5E.3-1: Intra-band non-contiguous V2X configurations

| V2X configuration | SL transmission |
| --- | --- |
| V2X_47A_n47A | E-UTRA Band 47 or NR band n47 |
| NOTE 1: Only single switched SL is supported. |  |

### 5.5E.4	Inter-band V2X operation in FR1

#### 5.5E.4.1	Inter-band V2X configurations within FR1 (two bands)

Table 5.5E.4.1-1: Inter-band V2X configurations

| V2X configuration | V2X transmission configuration |
| --- | --- |
| V2X_20A_n38A | V2X_20A_n38A |
| V2X_n71A_47A | V2X_n71A_47A |
| NOTE 1: V2X transmission configurations are the configurations supported by the present release of specifications. |  |

# 6	Transmitter characteristics

## 6.1	General

Editor's note:	Test configurations/environments that require new spherical scan shall be included in test procedure clause and identifying such scenarios is currently FFS and owned by RAN5.

Unless otherwise stated the transmitter, characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with E-UTRA connecting to the network by OTA without calibration.

For NR FR2 Tx test cases the identified beam peak direction can be stored and reused for a device under test in various configurations/environments for the full duration of device testing as long as beam peak direction is the same.

Unless otherwise stated, requirements for NR transmitter written in TS 38.521-1 [8] and TS 38.521-2 [9] apply and are assumed anchor agnostic. Requirements are verified under conditions where anchor resources do not interfere NR operation. For Rel-15, unless otherwise stated, if UE indicates IE maxNumberSRS-Ports-PerResource = n2 in NR standalone operation mode, the said UE shall meet the NR requirements for either power class 2 or power class 3 in EN-DC within FR1 if UE indicates IE maxNumberSRS-Ports-PerResource = n1 for EN-DC on this NR band. For Rel-16 and forward, if UE indicates IE powerClassNRPart-r16 as defined in TS 38.331 [18] in EN-DC, UE shall meet NR requirements according to this power class.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

For conformance testing involving FR2 test cases in this specification, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

Uplink RB allocations for NR FR1 carrier given in TS 38.521-1 [8] Table 6.1-1 are used throughout this clause, unless otherwise stated by the test case.

Uplink RB allocations for NR FR2 carrier given in TS 38.521-2 [9] Table 6.1-1 are used throughout this clause, unless otherwise stated by the test case.

Uplink RB allocations for E-UTRA carrier and E-UTRA intra-band contiguous CA (2CC) given in Table 6.1-1 and Table 6.1-2 respectively are used throughout this clause, unless otherwise stated by the test case.

Table 6.1-1: Common uplink configuration for E-UTRA carrier

| Channel Bandwidth | RB allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Full_Allocation | Partial_Allocation | 1RB_Left | 1RB_Right |
| 1.4MHz | 6@0 | 5@0 | 1@0 | 1@5 |
| 3MHz | 15@0 | 4@0 | 1@0 | 1@14 |
| 5MHz | 25@0 | 8@0 | 1@0 | 1@24 |
| 10MHz | 50@0 | 12@0 | 1@0 | 1@49 |
| 15MHz | 75@0 | 16@0 | 1@0 | 1@74 |
| 20MHz | 100@0 | 18@0 | 1@0 | 1@99 |
| NOTE: Partial_Allocation corresponds to the test points with 0dB MPRsingle,E-UTRA for QPSK modulation type included in TS 36.521-1 Table 6.2.2.4.1-1. |  |  |  |  |

Table 6.1-2 Common uplink RB allocation for E-UTRA intra-band contiguous CA (2CC)

| Smallest Component Carrier Transmission Bandwidth | PCC & SCC RB allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Partial_Allocation |  | 1RB_Left |  | 1RB_Right |  |
|  | PCC | SCC | PCC | SCC | PCC | SCC |
| 5MHz | P_8@0 | S_0@0 | P_1@0 | S_0@0 | P_1@RBmax | S_0@0 |
| 10MHz | P_12@0 | S_0@0 | P_1@0 | S_0@0 | P_1@RBmax | S_0@0 |
| 15MHz | P_16@0 | S_0@0 | P_1@0 | S_0@0 | P_1@RBmax | S_0@0 |
| 20MHz | P_18@0 | S_0@0 | P_1@0 | S_0@0 | P_1@RBmax | S_0@0 |
| NOTE: Partial_Allocation corresponds to the test points with 0dB MPRE-UTRA, CA for QPSK modulation type included in TS 36.521-1 Table 6.2.2.4.1-1. |  |  |  |  |  |  |

## 6.2	Transmitter power

## 6.2A	Transmitter power for CA without EN-DC

### 6.2A.1	UE maximum output power for CA

#### 6.2A.1.1	UE maximum output power for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.2A.1.1.1	Test purpose

Same test purpose as in clause 6.2.1 in TS 38.521-1 [8] for NR/5GC FR1 carrier(s) and clause 6.2.1 in TS 38.521-2 [9] for NR/5GC FR2 carrier(s).

6.2A.1.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR/5GC FR1 conducted mode with NR/5GC FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for maximum output power apply and are tested in TS 38.521-1 [8] clauses 6.2 and 6.2A and TS 38.521-2 [9] clauses 6.2 and 6.2A.

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.1	UE maximum output power reduction for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.2A.2.1.1	Test purpose

Same test purpose as in clause 6.2.2 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.2.2 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.2A.2.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The SA requirements for additional spectrum emissions mask apply and are tested in TS 38.521-1 [8] clauses 6.2 and 6.2A and TS 38.521-2 [9] clauses 6.2 and 6.2A.

### 6.2A.3	UE additional maximum output power reduction for CA

#### 6.2A.3.1	UE additional maximum output power reduction for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.2A.3.1.1	Test purpose

Same test purpose as in clause 6.2.3.1 in TS 38.521-1 [8] for NR FR1 carrier and clause 6.2.3.1 in TS 38.521-2 [9] for NR FR2 carrier.

6.2A.3.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The single carrier requirements for Additional Maximum Output Power apply and are tested as part of clause 6.2.3 in TS 38.521-1 [8] for NR FR1 carrier and clause 6.2.3 in TS 38.521-2 [9] for NR FR2 carrier.

### 6.2A.4	Configured output power level for CA

#### 6.2A.4.1	Configured output power level for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.2A.4.1.1	Test purpose

Same test purpose as in clause 6.2.4 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.2.4 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.2A.4.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for configured output power level apply and are tested in TS 38.521-1 [8] clauses 6.2 and 6.2A and TS 38.521-2 [9] clauses 6.2 and 6.2A.

#### 6.2A.4.2	ΔTIB,c for CA

##### 6.2A.4.2.1	ΔTIB,c for inter-band CA between FR 1 and FR 2

For the UE which supports inter-band NR CA configuration, ΔTIB,c in Table 6.2A.4.2.1-1 applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2A.4.2.1-1: Void

## 6.2B	Transmitter power for DC

### 6.2B.1	UE Maximum Output Power for DC

#### 6.2B.1.1	UE Maximum Output Power for Intra-Band Contiguous EN-DC

Editor's note:

-	For overlapping transmission there is no test point satisfying 0dB MPR according to RAN4 specification.

6.2B.1.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC operation on FR1.

6.2B.1.1.3	Minimum conformance requirements

The following UE Power Classes define the total maximum output power for any transmission bandwidth(s) of the CG(s) configured.

The maximum output power is measured as the total maximum output power across the UE antenna connector(s). The period of measurement shall be at least one sub frame.

Table 6.2B.1.1.3-1: Maximum output power for EN-DC (continuous sub-blocks)

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA |  |  | 23 | +2/-3 |
| DC_(n)41AA | 26 | +2/-31 | 23 | +2/-31 |
| NOTE 1: An uplink EN-DC configuration in which the band has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: Power Class 3 is the default power class unless otherwise stated. |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [17] is provided and set to the maximum output power of the default power class or lower;

-	apply all requirements for the default power class, and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in subclause 6.2B.4;

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.

LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.1.1.4	Test description

6.2B.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2.All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2, and are shown in table 6.2B.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C, clause C.2 and in TS 38.521-1 [8] Annex C, clause C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.1.1.4.1-1: Test configuration table for Intra-Band Contiguous EN-DC

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause 4.3.1 |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in TS 38.508-1 [5] clause 4.3.1 |  | Highest NRB_agg |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  | Highest (NOTE 3) |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A | QPSK | Partial_Allocation | N/A | N/A |
| 2 | N/A | QPSK | 1RB_Left | N/A | N/A |
| 3 | N/A | N/A | N/A | DFT-s-OFDM QPSK | Inner Full |
| 4 | N/A | N/A | N/A | DFT-s-OFDM QPSK | Inner_1RB_LEFT |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 3: For DC_(n)71AA, only NR SCS of 15 kHz is tested. |  |  |  |  |  |

Table 6.2B.1.1.4.1-2: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] clause A.3.1.1 for SS diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C, clause C.0 and TS 38.521-1 [8] Annex C, clause C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B, clause B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 6.2B.1.1.4.1-1on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

3.	For a UE supporting dynamic power sharing, measure the mean power over all component carriers. For a UE not supporting dynamic power sharing, measure the power of each component carrier individually. Ffor the tested EN-DC configuration, the requirements described in clause 6.2B.2.1.5 shall be met. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	For UEs supporting Power Class 2, repeat steps 1~3 on the applicable bands with message exception defined in Table 6.2B.1.1.4.3-5.

6.2B.1.1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.2B.1.1.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [6], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2B.1.1.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE |
|  | 26 |  | Power Class 2 UE |

Table 6.2B.1.1.4.3-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 3 UE |
|  | 26 |  | Power Class 2 UE |

Table 6.2B.1.1.4.3-3: RRCConnectionReconfiguration: tdm-PatternConfig if operating on FDD band

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 ::= CHOICE{ |  |  | Test IDs 7-15 |
| setup :: = SEQUENCE { |  | Apply if operating on FDD band for a UE NOT indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE according to TS 38.213 [x] clause 7.6.1 |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2B.1.1.4.3-4: SystemInfomationBlockType1: tdd-Config if operating on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.1.1.4.3-5: RRCConnectionReconfiguration: p-MaxUE-FR1-r15 (step 4 in 6.2B.1.1.4.2)

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 2 UE |
| } |  |  |  |

6.2B.1.1.5	Test requirements

The maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the DC UE Power Class and tolerance in Table 6.2B.1.1.5-1 for power class 3 UE and Table 6.2B.1.1.5-2 for power class 2 UE. The corresponding requirements is specified in Table 6.2.2.5-1, 6.2.2_1.5-1 in TS 36.521-1 [10] or Table 6.2.1.5-1, 6.2.1.5-2 in TS 38.521-1 [8].

The maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed in Table 6.2B.1.1.5-1.

Table 6.2B.1.1.5-1: Maximum output power for EN-DC (continuous sub-blocks) for power class 3

| DC configuration | Power class2 | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA |  |  | 23 | +2+TT/-3-TT |
| DC_(n)41AA |  |  | 23 | +2+TT/-31+TT |
| NOTE 1: It is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.1.5-3. |  |  |  |  |

Table 6.2B.1.1.5-2: Maximum output power for EN-DC (continuous sub-blocks) for Power Class 2 for Rel-15

| DC configuration | Carrier | Power class2 | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_(n)41AA | NR carrier | 26 | +2+TT/-31-TT | UE indicates PC2 on NR band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-31-TT | UE indicates PC3 on NR band | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-31-TT | UE indicates PC2 ON E-UTRA band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-31-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| NOTE 1: It is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.1.5-3 |  |  |  |  |  |

Table 6.2B.1.1.5-2a: Maximum output power for EN-DC (continuous sub-blocks) for Power Class 2 for Rel-16 and forward

| DC configuration | Carrier | Power class2 | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_(n)41AA | NR carrier | 26 | +2+TT/-31-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-31-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-31-TT | UE indicates PC2 ON E-UTRA band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-31-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| NOTE 1: It is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.1.5-3. |  |  |  |  |  |

Table 6.2B.1.1.5-3: Test Tolerance (Maximum Output Power for Intra-Band Contiguous EN-DC)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| ENBW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < ENBW ≤ 100MHz | 1.0 dB | 1.0 dB |

#### 6.2B.1.2	UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC

Editor's note:

-	For overlapping transmission there is no test point satisfying 0dB MPR according to RAN4 specification.

6.2B.1.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC operation on FR1.

6.2B.1.2.3	Minimum conformance requirements

The following UE Power Classes define the total maximum output power for any transmission bandwidth(s) of the CG(s) configured.

The maximum output power is measured as the total maximum output power across the UE antenna connector(s). The period of measurement shall be at least one sub frame.

Table 6.2B.1.2.3-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_2A_n2A4 |  |  | 23 | +2/-3 |
| DC_41A_n41A | 26 | +2/-31 | 23 | +2/-31 |
| DC_66A_n66A4 |  |  | 23 | +2/-3 |
| NOTE 1: An uplink EN-DC configuration in which the band has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: Only single switched UL is supported in Rel-15. NOTE 3: Power Class 3 is the default power class unless otherwise stated. NOTE 4: Only single switched UL is supported. |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [17] is provided and set to the maximum output power of the default power class or lower:

-	apply all requirements for the default power class, and set the configured transmitted power as specified in subclause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in subclause 6.2B.4.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.

LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.1.2.4	Test description

6.2B.1.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in Table 5.3B.1.3-1, channel bandwidths and sub-carrier spacings for the NR cell are specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in Table 5.3B.1.3-1, and are shown in table 6.2B.1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C, clause C.2 and in TS 38.521-1 [8] Annex C, clause C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.1.2.4.1-1: Test configuration table for intra-band non-contiguous EN-DC

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause 4.3.1 |  | Minimum Wgap, Maximum Wgap |  |  |  |
| Test EN-DC bandwidth combination as specified in TS 38.508-1 [5] clause 4.3.1 |  | Highest NRB_agg |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  | Highest |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A | QPSK | Partial_Allocation | N/A | N/A |
| 2 | N/A | QPSK | 1RB_Left | N/A | N/A |
| 3 | N/A | N/A | N/A | DFT-s-OFDM QPSK | Inner Full |
| 4 | N/A | N/A | N/A | DFT-s-OFDM QPSK | Inner_1RB_LEFT |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] A.3.1.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.2.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.1.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 6.2B.1.2.4.1-1on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level for Power class 3.

3.	Measure the mean transmitted power over all EN-DC component carriers in the EN-DC, which shall meet the requirements described in table 6.2B.1.2.5-1 and the period of the measurement shall be at least the continuous duration of one active sub-frame.

4.	For UEs supporting Power Class 2, repeat steps 1~3 on the applicable bands with message exception defined in Table 6.2B.1.1.4.3-5.

6.2B.1.2.4.3	Message contents

Same message contents as specified in 6.2B.1.1.4.3.

6.2B.1.2.5	Test requirements

The maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the DC UE Power Class and tolerance in Table 6.2B.1.2.5-1 for power class 3 UE and Table 6.2B.1.2.5-2 for power class 2 UE. The corresponding requirements is specified in table 6.2.2.5-1, 6.2.2_1.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1, 6.2.1.5-2 in TS 38.521-1 [8].

The maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed in Table 6.2B.1.2.5-1.

Table 6.2B.1.2.5-1: Maximum output power for EN-DC (non-continuous sub-blocks) for power class 3

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_2A_n2A4 |  |  | 23 | +2+TT/-3-TT |
| DC_41A_n41A |  |  | 23 | +2+TT/-31-TT |
| DC_66A_n66A4 |  |  | 23 | +2+TT/-3-TT |
| NOTE 1: It is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: Only single switched UL is supported in Rel-15. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.2.5-3 NOTE 4: Only single switched UL is supported. |  |  |  |  |

Table 6.2B.1.2.5-2: Maximum output power for EN-DC (non-continuous sub-blocks) for power class 2 for Rel-15

| DC configuration | Carrier | Power class2 | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_41A_n41A | NR carrier | 26 | +2+TT/-31-TT | UE indicates PC2 on NR band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-21-TT | UE indicates PC3 on NR band | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-31-TT | UE indicates PC3 on E-UTRA carrier of this DC_Configutation | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-21-TT | UE indicates PC3 on E-UTRA carrier of this DC_Configutation | UE meets power class 3 requirements |
| NOTE 1: It is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.1.5-3 |  |  |  |  |  |

Table 6.2B.1.2.5-2a: Maximum output power for EN-DC (non-continuous sub-blocks) for power class 2 for Rel-16 and forward

| DC configuration | Carrier | Power class2 | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_41A_n41A | NR carrier | 26 | +2+TT/-31-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-21-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-21-TT | UE indicates PC2 ON E-UTRA carrier of this DC_Configutation | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-21-TT | UE indicates PC3 on E-UTRA carrier of this DC_Configutation | UE meets power class 3 requirements |
| NOTE 1: It is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.2.5-3 |  |  |  |  |  |

Table 6.2B.1.2.5-3: Test Tolerance

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

#### 6.2B.1.3	UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC)

6.2B.1.3.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC with 1 E-UTRA CC and 1 NR CC operating on FR1.

6.2B.1.3.3	Minimum conformance requirements

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2/-3 |
| DC_1A_n5A |  |  | 23 | +2/-3 |
| DC_1A_n7A |  |  | 23 | +2/-3 |
| DC_1A_n8A |  |  | 23 | +2/-3 |
| DC_1A_n28A |  |  | 23 | +2/-3 |
| DC_1A_n41A |  |  | 23 | +2/-3 |
| DC_1A_n77A |  |  | 23 | +2/-3 |
| DC_1A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_1A_n79A | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n5A |  |  | 23 | +2/-3 |
| DC_2A_n41A |  |  | 23 | +2/-3 |
| DC_2A_n66A |  |  | 23 | +2/-3 |
| DC_2A_n71A |  |  | 23 | +2/-3 |
| DC_2A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n78A |  |  | 23 | +2/-3 |
| DC_3A_n1A |  |  | 23 | +2/-3 |
| DC_3A_n5A |  |  | 23 | +2/-3 |
| DC_3A_n7A |  |  | 23 | +2/-3 |
| DC_3A_n8A |  |  | 23 | +2/-3 |
| DC_3A_n28A |  |  | 23 | +2/-3 |
| DC_3A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n77A |  |  | 23 | +2/-3 |
| DC_3A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n79A | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n2A |  |  | 23 | +2/-3 |
| DC_5A_n40A |  |  | 23 | +2/-3 |
| DC_5A_n66A |  |  | 23 | +2/-3 |
| DC_5A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A |  |  | 23 | +2/-3 |
| DC_7A_n1A |  |  | 23 | +2/-3 |
| DC_7A_n3A |  |  | 23 | +2/-3 |
| DC_7A_n5A |  |  | 23 | +2/-3 |
| DC_7A_n8A |  |  | 23 | +2/-3 |
| DC_7A_n28A |  |  | 23 | +2/-3 |
| DC_7A_n66A |  |  | 23 | +2/-3 |
| DC_7A_n78A |  |  | 23 | +2/-3 |
| DC_8A_n1A |  |  | 23 | +2/-3 |
| DC_8A_n3A |  |  | 23 | +2/-3 |
| DC_8A_n20A |  |  | 23 | +2/-3 |
| DC_8A_n28A |  |  | 23 | +2/-3 |
| DC_8A_n41A |  |  | 23 | +2/-3 |
| DC_8A_n77A |  |  | 23 | +2/-3 |
| DC_8A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_11A_n77A |  |  | 23 | +2/-3 |
| DC_11A_n78A |  |  | 23 | +2/-3 |
| DC_11A_n79A |  |  | 23 | +2/-3 |
| DC_12A_n2A |  |  | 23 | +2/-3 |
| DC_12A_n5A |  |  | 23 | +2/-3 |
| DC_12A_n66A |  |  | 23 | +2/-3 |
| DC_12A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_12A_n78A |  |  | 23 | +2/-3 |
| DC_13A_n2A |  |  | 23 | +2/-3 |
| DC_13A_n66A |  |  | 23 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_14A_n2A |  |  | 23 | +2/-3 |
| DC_14A_n66A |  |  | 23 | +2/-3 |
| DC_14A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_18A_n77A |  |  | 23 | +2/-3 |
| DC_18A_n78A |  |  | 23 | +2/-3 |
| DC_19A_n1A |  |  | 23 | +2/-3 |
| DC_18A_n79A |  |  | 23 | +2/-3 |
| DC_19A_n77A |  |  | 23 | +2/-3 |
| DC_19A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_19A_n79A | 266 | +2/-3 | 23 | +2/-3 |
| DC_20A_n1A |  |  | 23 | +2/-3 |
| DC_20A_n3A |  |  | 23 | +2/-3 |
| DC_20A_n7A |  |  | 23 | +2/-3 |
| DC_20A_n8A |  |  | 23 | +2/-3 |
| DC_20A_n28A |  |  | 23 | +2/-3 |
| DC_20A_n78A |  |  | 23 | +2/-3 |
| DC_21A_n1A |  |  | 23 | +2/-3 |
| DC_21A_n28A |  |  | 23 | +2/-3 |
| DC_21A_n77A |  |  | 23 | +2/-3 |
| DC_21A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_21A_n79A | 266 | +2/-3 | 23 | +2/-3 |
| DC_25A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n77A |  |  | 23 | +2/-3 |
| DC_26A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n3A |  |  | 23 | +2/-3 |
| DC_28A_n5A |  |  | 23 | +2/-3 |
| DC_28A_n7A |  |  | 23 | +2/-3 |
| DC_28A_n77A |  |  | 23 | +2/-3 |
| DC_28A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_28A_n79A |  |  | 23 | +2/-3 |
| DC_30A_n2A |  |  | 23 | +2/-3 |
| DC_30A_n5A |  |  | 23 | +2/-3 |
| DC_30A_n66A |  |  | 23 | +2/-3 |
| DC_30A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_39A_n41A | 265 | +2/-3 | 23 | +2/-2 |
| DC_39A_n79A | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n1A |  |  | 23 | +2/-3 |
| DC_40A_n41A |  |  | 23 | +2/-3 |
| DC_40A_n78A |  |  | 23 | +2/-3 |
| DC_40A_n79A |  |  | 23 | +2/-3 |
| DC_41A_n28A |  |  | 23 | +2/-3 |
| DC_41A_n77A |  |  | 23 | +2/-3 |
| DC_41A_n78A |  |  | 23 | +2/-3 |
| DC_41A_n79A | 265 | +2/-3 | 23 | +2/-3 |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n5A |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  | 23 | +2/-3 |
| DC_66A_n2A |  |  | 23 | +2/-3 |
| DC_66A_n5A |  |  | 23 | +2/-3 |
| DC_66A_n41A |  |  | 23 | +2/-3 |
| DC_66A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n78A |  |  | 23 | +2/-3 |
| DC_71A_n2A |  |  | 23 | +2/-3 |
| DC_71A_n66A |  |  | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 7: Only single switched UL is supported. NOTE 8: The UE that supports a PC2 uplink EN-DC configuration with single carrier for each individual band and a composite of supporting PC3 within a TDD or FDD band and PC2 within a second TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

If a UE supports a different power class than the default UE power class for an EN-DC band combination and NR TDD inter-band EN-DC combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified clause 6.2B.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in clause 6.2B.4.

If a UE supports a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

- if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of E-UTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of E-UTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4;

else

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.

LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.1.3.4	Test description

6.2B.1.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, DC configuration specified in clause 5.5B.4 and test channel bandwidths specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1, and sub-carrier spacing based on NR operating bands specified in TS 38.521-1 [8] clause 5.3. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in table 6.2B.1.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C, clause C.2 and in TS 38.521-1 [8] Annex C, clause C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.1.3.4.1-1: Test configuration table

| Default Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 and TS 36.508 [6] |  |  |  |  | Low range for E-UTRA CC1 and NR CC1, Mid range for E-UTRA CC1 and NR CC1, High range for E-UTRA CC1 and NR CC1 (NOTE 4) |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [6] clause 4.3.1 and TS 38.508-1 clause 4.3.1 |  |  |  |  | 5MHz for E-UTRA CC1 and Lowest for NR CC1, Highest for E-UTRA CC1 and Highest for NR CC1 |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | Test Freq | E-UTRA BW | NR BW | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  |  |  |  | E-UTRA Cell |  | NR Cell |  |
|  |  |  |  |  | Modulation | RB allocation (NOTE 1) | Modulation (NOTE 3) | RB allocation (NOTE 2) |
| 15 | High | Default | Default | N/A | QPSK | 1RB_Right | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Right |
| 25 | Low | Default | Default |  | QPSK | 1RB_Left | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |
| 35 | Default | Default | Default |  | QPSK | Partial_Allocation | DFT-s-OFDM PI/2 BPSK | Inner_Full |
| 45 | High | Default | Default |  | QPSK | 1RB_Right | DFT-s-OFDM QPSK | Inner_1RB_Right |
| 55 | Low | Default | Default |  | QPSK | 1RB_Left | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 65 | Default | Default | Default |  | QPSK | Partial_Allocation | DFT-s-OFDM QPSK | Inner_Full |
| 7 | High | 5MHz, Highest | Lowest |  | QPSK | 1RB_Right | N/A | N/A |
| 8 | Low | 5MHz, Highest | Lowest |  | QPSK | 1RB_Left | N/A | N/A |
| 9 | Default | 5MHz, Highest | Lowest |  | QPSK | Partial_Allocation | N/A | N/A |
| 10 | High | 5MHz | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Right |
| 11 | Low | 5MHz | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |
| 12 | Default | 5MHz | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM PI/2 BPSK | Inner_Full |
| 13 | High | 5MHz | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM QPSK | Inner_1RB_Right |
| 14 | Low | 5MHz | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 15 | Default | 5MHz | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 3: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in NR FR1. NOTE 4: For NR band n28, the Highest test channel bandwidth is replaced by 20MHz due to MPR is always larger than 0dB for 30MHz bandwidth. NOTE 5: Only applicable to UEs not supporting UE capability singleUL-Transmission. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] clause A.3.1.1 for SS and clause A.3.2.1 for UE.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set according to Table 6.2B.1.3.3-1.

5.	Propagation conditions are set according to TS 36.521-1 [10] and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.3.4.3.

7.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.1.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 6.2B.1.3.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. For an E-UTRA FDD and NR TDD EN-DC band combination, if UE supports PC2, the percentage of EUTRA uplink symbols transmitted in each radio frame shall be less than 40%.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

3.	For test ID 1~6 measure the sum of mean transmitted power over all EN-DC component carriers in the EN-DC, which shall meet the requirements described in table 6.2B.1.3.5-1 and the period of the measurement shall be at least the continuous duration of one active sub-frame. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

For test ID 7~15 measure the mean transmitted power over E-UTRA carrier or NR carrier, which shall meet the requirements described in table 6.2.2.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1 in TS 38.521-1 [8] respectively. The period of the measurement shall be at least the continuous duration of one active sub-frame.

4.	For UEs supporting Power Class 2, repeat steps 1~3 on the applicable bands with message exception defined in Table 6.2B.1.3.4.3-5.

6.2B.1.3.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions:

Table 6.2B.1.3.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [6], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2B.1.3.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| p-NR-FR1 | 20 | For simultaneous E-UTRA and NR transmission | Power Class 3 UE Test IDs 1-6 |
|  | 23 | For Test IDs 7~9 NR carrier is configured but not measured. | Power Class 3 UE Test IDs 7-15 |
|  | 23 | For simultaneous E-UTRA and NR transmission | Power Class 2 UE Test IDs 1-6 |
|  | 26 | For Test IDs 7~9 NR carrier is configured but not measured. | Power Class 2 UE Test IDs 7-15 |
|  | 26 | For simultaneous E-UTRA and NR transmission | Power Class 2 UE Test IDs 1-6 if UE indicates support of higherPowerLimit-r17 |
| } |  |  |  |

Table 6.2B.1.3.4.3-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 20 | For simultaneous E-UTRA and NR transmission | Power Class 3 UE Test IDs 1-6 |
|  | 23 | For Test IDs 10~15 E-UTRA carrier is configured but not measured. | Power Class 3 UE Test IDs 7-15 |
|  | 23 | For simultaneous E-UTRA and NR transmission | Power Class 2 UE Test IDs 1-6 |
|  | 26 | For Test IDs 10~15 E-UTRA carrier is configured but not measured. | Power Class 2 UE Test IDs 7-15 |

Table 6.2B.1.3.4.3-3: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE does not support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  | Test IDs 7-15 |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2B.1.3.4.3-4: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.1.3.4.3-5: RRCConnectionReconfiguration: p-MaxUE-FR1-r15 (step 4 in 6.2B.1.3.4.2)

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 2 UE |
| } |  |  |  |

6.2B.1.3.5	Test requirements

For test ID 1~6 the maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the UE Power Class and tolerance in Table 6.2B.1.3.5-1.

For test ID 7~15 the maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the UE Power Class and tolerance in table 6.2.2.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1 in TS 38.521-1 [8] for E-UTRA carier and NR carrier respectively for Power class 3, and in Table 6.2B.1.3.5-2a for Power class 2.

For test ID 1~6 the maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed by Power Class 3 and tolerance in Table 6.2B.1.3.5-1.

For test ID 7~15 the maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed by Power Class 3 and tolerance in table 6.2.2.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1 in TS 38.521-1 [8] for E-UTRA carier and NR carrier respectively.

Table 6.2B.1.3.5-1: Maximum output power for inter-band EN-DC (two bands), for overlapping UL transmission

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n7A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n78A | 268, 27.89 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_1A_n79A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_2A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n71A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n77A | 268, 27.89 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_2A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n7A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n8A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n41A | 268, 27.89 | +2+TT/-3-TT3 | 23 | +2+TT/-3-TT |
| DC_3A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n78A | 268, 27.89 | +2+TT/-3-TT3 | 23 | +2 +TT/-3-TT |
| DC_3A_n79A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_5A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_5A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_5A_n77A | 268, 27.89 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_5A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n1A |  |  |  | +2 +TT/-3-TT |
| DC_8A_n3A |  |  |  | +2 +TT/-3-TT |
| DC_8A_n20A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n78A | 268, 27.89 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_11A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_11A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_11A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n77A | 268, 27.89 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_14A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_14A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_14A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n78A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_19A_n79A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_20A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n7A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n8A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n78A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_21A_n79A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_25A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n7A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n78A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_28A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_30A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_30A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_30A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_30A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_39A_n41A | 26, 27.89 | +2/-31 | 23 | +2 +TT/-3-TT |
| DC_39A_n79A | 26, 27.89 | +2/-3 | 23 | +2 +TT/-3-TT |
| DC_40A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_40A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_40A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_40A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_41A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_41A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_41A_n79A | 26, 27.89 | +2/-31 | 23 | +2 +TT/-3-TT |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_48A_n46A |  |  | 23 | +2 +TT/-3-TT |
| DC_48A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n71A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n77A | 268, 27.89 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_66A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_71A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_71A_n66A |  |  | 23 | +2 +TT/-3-TT |
| NOTE 1: TT applies to output power in each UL carrier with E-UTRA UL transmission not overlapping with NR UL transmission in time, and its value is the same as TT of standalone E-UTRA or NR transmission. For detailed values refer to Table 6.2B.1.3.5-4. NOTE 2: TT applies to overall output power with E-UTRA UL transmission overlapping with NR UL transmission in time, and its value is the maximum TT among all E-UTRA and NR UL carriers. For detailed values refer to Table 6.2B.1.3.5-3. NOTE 3: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 in TS 38.521-1 [8] or NOTE 2 in Table 6.2.2.3-1 in TS 36.521-1 [21] is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. NOTE 5: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 6: Power Class 3 is the default power class unless otherwise stated. NOTE 7: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE supports PC3 within E-UTRA cell group and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. NOTE 9: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band and indicates support of higherPowerLimit-r17 capability. |  |  |  |  |

Table 6.2B.1.3.5-2: Void

Table 6.2B.1.3.5-2a: Maximum output power for inter-band EN-DC (two bands), for non-overlapping UL transmission for power class 2

| EN-DC configuration | Carrier | Power class 2 (dBm) | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_1A_n79A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_2A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_3A_n41A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-2-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_3A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_3A_n79A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_5A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_8A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_12A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_13A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_14A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_19A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_19A_n79A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_21A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_21A_n79A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_28A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_30A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_39A_n41A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-2-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_39A_n79A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_41A_n79A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-2-TT | UE indicates PC2 on E-UTRA band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_66A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| NOTE 1: TT applies to output power in each UL carrier with E-UTRA UL transmission not overlapping with NR UL transmission in time, and its value is the same as TT of standalone E-UTRA or NR transmission. For detailed values refer to Table 6.2B.1.3.5-4. NOTE 2: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 in TS 38.521-1 [8] or NOTE 2 in Table 6.2.2.3-1 in TS 36.521-1 [21] is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |

Table 6.2B.1.3.5-3: Test Tolerance for UE maximum output power (Overlapping UL transmission)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.1.3.5-4: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

#### 6.2B.1.3_1	UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC)

6.2B.1.3_1.1	Test purpose

Same test purpose as in clause 6.2B.1.3.1

6.2B.1.3_1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC operating on FR1.

6.2B.1.3_1.3	Minimum conformance requirements

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3_1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_7C_n78A |  |  | 23 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signalled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.

LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.1.3_1.4	Test description

6.2B.1.3_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, DC configuration specified in clause 5.5B.4 and test channel bandwidths specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1, and sub-carrier spacing based on NR operating bands specified in TS 38.521-1 [8] clause 5.3. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in table 6.2B.1.3_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C, clause C.2 and in TS 38.521-1 [8] Annex C, clause C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.1.3_1.4.1-1: Test configuration table

| Default Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 and TS 36.508 [6] |  |  |  |  | Low range for MCG and SCG, High range for MCG and SCG |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [6] clause 4.3.1 and TS 38.508-1 clause 4.3.1 |  |  |  |  | Lowest NRB_agg for E-UTRA CCs and Lowest for NR CC1, Highest NRB_agg for E-UTRA CCs and Highest for NR CC1 |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | Test Freq | E-UTRA BW | NR BW | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  |  |  |  | MCG – EUTRA PCC&SCC |  | SCG - NR |  |
|  |  |  |  |  | Modulation | RB allocation (NOTE 1) | Modulation (NOTE 3) | RB allocation (NOTE 2) |
| 1 | High | Default | Default | N/A | QPSK | 1RB_Right | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Right |
| 2 | Low | Default | Default |  | QPSK | 1RB_Left | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |
| 3 | Default | Default | Default |  | QPSK | Partial_Allocation | DFT-s-OFDM PI/2 BPSK | Inner_Full |
| 4 | High | Default | Default |  | QPSK | 1RB_Right | DFT-s-OFDM QPSK | Inner_1RB_Right |
| 5 | Low | Default | Default |  | QPSK | 1RB_Left | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 6 | Default | Default | Default |  | QPSK | Partial_Allocation | DFT-s-OFDM QPSK | Inner_Full |
| 7 | High | Lowest NRB_agg, Highest NRB_agg | Lowest |  | QPSK | 1RB_Right | N/A | N/A |
| 8 | Low | Lowest NRB_agg, Highest NRB_agg | Lowest |  | QPSK | 1RB_Left | N/A | N/A |
| 9 | Default | Lowest NRB_agg, Highest NRB_agg | Lowest |  | QPSK | Partial_Allocation | N/A | N/A |
| 10 | High | Lowest NRB_agg | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Right |
| 11 | Low | Lowest NRB_agg | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |
| 12 | Default | Lowest NRB_agg | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM PI/2 BPSK | Inner_Full |
| 13 | High | Lowest NRB_agg | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM QPSK | Inner_1RB_Right |
| 14 | Low | Lowest NRB_agg | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 15 | Default | Lowest NRB_agg | Lowest, Highest |  | N/A | N/A | DFT-s-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2 in current specification. For 1RB_Left allocation, the PCC is located on the lower frequency while SCC is located on the higher frequency. For 1RB_Right allocation, the PCC is located on the higher frequency while SCC is located on the lower frequency. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 3: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in NR FR1. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] clause A.3.1.1 for SS and clause A.3.2.1 for UE.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG PCC and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set according to Table 6.2B.1.3_1.3-1.

5.	Propagation conditions are set for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.3_1.4.3.

7.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.1.3_1.4.2	Test procedure

1.	Configure E-UTRA SCC according to TS 36.521-1 [10] Annex C.0, C.1 and Annex C.3.0 for all E-UTRA downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [11] clause 5.2A.4. Message contents are defined in clause 6.2B.1.3_1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321, clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [12], clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2B.1.3_1.4.1-1 on E-UTRA PCC and SCC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

6.	For test ID 1~6 measure the sum of mean transmitted power over all EN-DC component carriers in the EN-DC, which shall meet the requirements described in table 6.2B.1.3_1.5-1 and the period of the measurement shall be at least the continuous duration of one active sub-frame.

For test ID 7~9 measure the sum of mean transmitted power over all E-UTRA component carriers, which shall meet the requirements described in Table 6.2.2A.1.5-1 in TS 36.521-1 [10]. The period of the measurement shall be at least the continuous duration of one active sub-frame.

For test ID 10~15 measure the mean transmitted power over NR carrier, which shall meet the requirements described in Table 6.2.1.5-1 in TS 38.521-1 [8] respectively. The period of the measurement shall be at least the continuous duration of one active sub-frame.

6.2B.1.3_1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions:

Table 6.2B.1.3_1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [6], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2B.1.3_1.4.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| p-NR-FR1 | 20 | For simultaneous E-UTRA and NR transmission | Power Class 3 UE Test IDs 1-6 |
|  | 23 | For Test IDs 7~9 NR carrier is configured but not measured. | Power Class 3 UE Test IDs 7-15 |
| } |  |  |  |

Table 6.2B.1.3_1.4.3-3: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 20 | For simultaneous E-UTRA and NR transmission | Power Class 3 UE Test IDs 1-6 |
|  | 23 | For Test IDs 10~15 E-UTRA carrier is configured but not measured. | Power Class 3 UE Test IDs 7-15 |

Table 6.2B.1.3_1.4.3-4: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE does not support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  | Test IDs 7-15 |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2B.1.3_1.4.3-5: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.2B.1.3_1.5	Test requirements

For test ID 1~6 the maximum output power for the DC configuration, derived in step 6 shall be within the range prescribed by the UE Power Class and tolerance in Table 6.2B.1.3_1.5-1.

For test ID 7~15 the maximum output power for the DC configuration, derived in step 6 shall be within the range prescribed by the UE Power Class and tolerance in Table 6.2.2A.1.5-1 in TS 36.521-1 [10] or Table 6.2.1.5-1 in TS 38.521-1 [8] for E-UTRA CG and NR CG respectively for Power class 3.

Table 6.2B.1.3_1.5-1: Maximum output power for inter-band EN-DC (two bands), for overlapping UL transmission

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_7C_n78A |  |  | 23 | +2+TT/-3-TT |
| NOTE 1: TT applies to output power in each UL carrier with E-UTRA UL transmission not overlapping with NR UL transmission in time, and its value is the same as TT of standalone E-UTRA or NR transmission. For detailed values refer to Table 6.2B.1.3_1.5-2. NOTE 2: TT applies to overall output power with E-UTRA UL transmission overlapping with NR UL transmission in time, and its value is the maximum TT among all E-UTRA and NR UL carriers. For detailed values refer to Table 6.2B.1.3_1.5-2. NOTE 3: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. NOTE 5: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 6: Power Class 3 is the default power class unless otherwise stated. NOTE 7: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signalled separately by the UE. NOTE 8: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |  |  |

Table 6.2B.1.3_1.5-2: Test Tolerance for UE maximum output power (Overlapping UL transmission)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

#### 6.2B.1.3a	UE Maximum Output Power for Inter-Band NE-DC within FR1

6.2B.1.3a.1	Test purpose

Same test purpose as in clause 6.2B.1.3.1.

6.2B.1.3a.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band NE-DC operating on FR1.

6.2B.1.3a.3	Minimum conformance requirements

For inter-band NE-DC of E-UTRA and NR in FR1, the following UE power classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3a.3-1: Maximum output power for inter-band NE-DC (two bands)

| NE-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_3A DC_n28A_3C | 23 | +2/-3 |
| DC_n28A_39A | 23 | +2/-3 |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.3a.

6.2B.1.3a.4	Test description

6.2B.1.3a.4.1	Initial condition

Same initial condition as in clause 6.2B.1.3.4.1 with the following exception:

Step 6 of Initial conditions will be updated as below:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NE-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.3a.4.3.

6.2B.1.3a.4.2	Test procedure

Same test procedure as in clause 6.2B.1.3.4.2

6.2B.1.3a.4.3	Message contents

Same message contents as in clause 6.2B.1.3.4.3.

6.2B.1.3a.5	Test requirements

For test ID 1~6 the maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the UE Power Class and tolerance in Table 6.2B.1.3a.5-1 for NE-DC.

Table 6.2B.1.3a.5-1: Maximum output power for inter-band NE-DC (two bands), for overlapping UL transmission

| NE-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_3A DC_n28A_3C | 23 | +2+TT/-3-TT |
| DC_n28A_39A | 23 | +2+TT/-3-TT |

Same Test Tolerance as in Table 6.2B.1.3.5-3.

#### 6.2B.1.4	UE Maximum Output Power for Inter-Band EN-DC including FR2

##### 6.2B.1.4.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2.1.1 in TS 38.521-2 is incomplete for power class 2 and 4.

6.2B.1.4.1.1	Test purpose

Same test purpose as in clause 6.2.1.1.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.1.4.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.2B.1.4.1.3	Minimum conformance requirements

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.2 and 6.2.2A of TS 36.101 [10] and for NR single carrier and CA operation specified in subclause 6.2.1, 6.2A.1, and 6.2D.1 of TS 38.101-2 [9] apply.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.4.

6.2B.1.4.1.4	Test description

Same test description as in clause 6.2.1.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.2.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4.1.5	Test requirement

Same test requirement as in clause 6.2.1.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.2B.1.4.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 2 and 4.

6.2B.1.4.2.1	Test purpose

Same test purpose as in clause 6.2.1.2.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.1.4.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.2B.1.4.2.3	Minimum conformance requirements

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.2 and 6.2.2A of TS 36.101 [10] and for NR single carrier and CA operation specified in subclause 6.2.1, 6.2A.1, and 6.2D.1 of TS 38.101-2 [9] apply.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.4.

6.2B.1.4.2.4	Test description

Same test description as in clause 6.2.1.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.2.1.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of initial conditions as in clause 6.2.1.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2.1.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4.2.5	Test requirement

Same test requirement as in clause 6.2.1.2.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.2B.1.4_1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (>1 NR CC)

##### 6.2B.1.4_1.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs)

###### 6.2B.1.4_1.1.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.1.1 in TS 38.521-2 [9] is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.1.1 in TS 38.521-2 [9] is incomplete for power class 1, 2 and 4.

-	Test Procedures for EIRP beam peak Extreme Conditions are FFS.

6.2B.1.4_1.1.1.1	Test purpose

Same test purpose as in clause 6.2A.1.1.1.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.1.4_1.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.2B.1.4_1.1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2A.1.1.1.3 in TS 38.521-2 [9].

6.2B.1.4_1.1.1.4	Test description

6.2B.1.4_1.1.1.4.1	Initial condition

Same test description as in clause 6.2A.1.1.1.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.1.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.1.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.1.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4_1.1.1.5	Test Requirements

Same test requirement as in clause 6.2A.1.1.1.5 in TS 38.521-2 [9] for the NR carriers.

###### 6.2B.1.4_1.1.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - Spherical Coverage

Editor's note: The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.2.1 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.2.1 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

6.2B.1.4_1.1.2.1	Test purpose

Same test purpose as in clause 6.2A.1.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.1.4_1.1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2NR UL CCs.

6.2B.1.4_1.1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2A.1.2.1.3 in TS 38.521-2 [9].

6.2B.1.4_1.1.2.4	Test description

6.2B.1.4_1.1.2.4.1	Initial condition

Same test description as in clause 6.2A.1.2.1.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.1.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.1.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.1.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4_1.1.2.5	Test Requirements

Same test requirement as in clause 6.2A.1.2.1.5 in TS 38.521-2 [9] for the NR carriers.

##### 6.2B.1.4_1.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs)

###### 6.2B.1.4_1.2.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.1.2 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.1.2 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

-	Test Procedures for EIRP beam peak Extreme Conditions are FFS.

6.2B.1.4_1.2.1.1	Test purpose

Same test purpose as in clause 6.2A.1.1.2.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3NR UL CCs.

6.2B.1.4_1.2.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2A.1.1.2.3 in TS 38.521-2 [9].

6.2B.1.4_1.2.1.4	Test description

6.2B.1.4_1.2.1.4.1	Initial condition

Same test description as in clause 6.2A.1.1.2.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.1.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.1.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.1.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4_1.2.1.5	Test Requirements

Same test requirement as in clause 6.2A.1.1.2.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.2B.1.4_1.2.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - Spherical Coverage

Editor’s note: The following aspects are either missing or not yet determined:

- The referred test case 6.2A.1.2.2 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

- The referred test case 6.2A.1.2.2 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

6.2B.1.4_1.2.2.1	Test purpose

Same test purpose as in clause 6.2A.1.2.2.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.2B.1.4_1.2.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2A.1.2.2.3 in TS 38.521-2 [9].

6.2B.1.4_1.2.2.4	Test description

6.2B.1.4_1.2.2.4.1	Initial condition

Same test description as in clause 6.2A.1.2.2.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.1.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.1.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.1.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4_1.2.2.5	Test Requirements

Same test requirement as in clause 6.2A.1.2.2.5 in TS 38.521-2 [9] for the NR carriers.

##### 6.2B.1.4_1.3	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs)

###### 6.2B.1.4_1.3.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.1.3 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.1.3 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

-	Test Procedures for EIRP beam peak Extreme Conditions are FFS.

6.2B.1.4_1.3.1.1	Test purpose

Same test purpose as in clause 6.2A.1.1.3.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4NR UL CCs.

6.2B.1.4_1.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2A.1.1.3.3 in TS 38.521-2 [9].

6.2B.1.4_1.3.1.4	Test description

6.2B.1.4_1.3.1.4.1	Initial condition

Same test description as in clause 6.2A.1.1.3.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.1.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.1.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.1.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4_1.3.1.5	Test Requirements

Same test requirement as in clause 6.2A.1.1.3.5 in TS 38.521-2 [9] for the NR carriers.

###### 6.2B.1.4_1.3.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) – Spherical Coverage

Editor’s note: The following aspects are either missing or not yet determined:

- The referred test case 6.2A.1.2.3 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

- The referred test case 6.2A.1.2.3 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

6.2B.1.4_1.3.2.1	Test purpose

Same test purpose as in clause 6.2A.1.2.3.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4NR UL CCs.

6.2B.1.4_1.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2A.1.2.3.3 in TS 38.521-2[9].

6.2B.1.4_1.3.2.4	Test description

6.2B.1.4_1.3.2.4.1	Initial condition

Same test description as in clause 6.2A.1.2.3.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.1.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.1.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.1.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4_1.3.2.5	Test Requirements

Same test requirement as in clause 6.2A.1.2.3.5 in TS 38.521-2 [9] for the NR carriers.

##### 6.2B.1.4_1.4	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs)

###### 6.2B.1.4_1.4.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) - EIRP and TRP

6.2B.1.4_1.4.1.1	Test purpose

Same test purpose as in clause 6.2A.1.1.4.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.4.1.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in RAN4 for FR2 NR carrier.

No test case details are specified

6.2B.1.4_1.4.1.5	Test Requirements

Same test requirement as in clause 6.2A.1.1.4.5 in TS 38.521-2 [9] for the NR carriers.

###### 6.2B.1.4_1.4.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) – Spherical Coverage

6.2B.1.4_1.4.2.1	Test purpose

Same test purpose as in clause 6.2A.1.2.4.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.4.2.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in RAN4 for FR2 NR carrier.

No test case details are specified.

6.2B.1.4_1.4.2.5	Test Requirements

Same test requirement as in clause 6.2A.1.2.4.5 in TS 38.521-2 [9] for the NR carriers.

##### 6.2B.1.4_1.5	UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs)

###### 6.2B.1.4_1.5.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) - EIRP and TRP

6.2B.1.4_1.5.1.1	Test purpose

Same test purpose as in clause 6.2A.1.1.5.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.5.1.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in RAN4 for FR2 NR carrier.

No test case details are specified.

###### 6.2B.1.4_1.5.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) – Spherical Coverage

6.2B.1.4_1.5.2.1	Test purpose

Same test purpose as in clause 6.2A.1.2.5.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.5.2.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in RAN4 for FR2 NR carrier.

No test case details are specified.

##### 6.2B.1.4_1.6	UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs)

###### 6.2B.1.4_1.6.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) - EIRP and TRP

6.2B.1.4_1.6.1.1	Test purpose

Same test purpose as in clause 6.2A.1.1.6.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.6.1.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in RAN4 for FR2 NR carrier.

No test case details are specified.

###### 6.2B.1.4_1.6.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) – Spherical Coverage

6.2B.1.4_1.6.2.1	Test purpose

Same test purpose as in clause 6.2A.1.2.6.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.6.2.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in RAN4 for FR2 NR carrier.

No test case details are specified.

##### 6.2B.1.4_1.7	UE Maximum Output Power for Inter-Band EN-DC including FR2 (8 NR CCs)

###### 6.2B.1.4_1.7.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (8 NR CCs) - EIRP and TRP

6.2B.1.4_1.7.1.1	Test purpose

Same test purpose as in clause 6.2A.1.1.7.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.7.1.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in RAN4 for FR2 NR carrier.

No test case details are specified.

###### 6.2B.1.4_1.7.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (8 NR CCs) – Spherical Coverage

6.2B.1.4_1.7.2.1	Test purpose

Same test purpose as in clause 6.2A.1.2.7.1 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.7.2.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in RAN4 for FR2 NR carrier.

No test case details are specified.

#### 6.2B.1.4D	UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO - EIRP and TRP

##### 6.2B.1.4D.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2D.1.1 in TS 38.521-2 [9] is incomplete for ETC.

6.2B.1.4D.1.1	Test purpose

Same test purpose as in clause 6.2D.1.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.1.4D.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with UL MIMO.

6.2B.1.4D.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2D.1.1 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.2B.1.4.

6.2B.1.4D.1.4	Test description

Same test description as in clause 6.2D.1.1 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.2D.1.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2D.1.1 in TS 38.521-2 [9] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.6.

6.2B.1.4D.1.5	Test requirement

Same test requirement as in clause 6.2D.1.5 of TS 38.521-2 [9] for the NR carrier.

##### 6.2B.1.4D.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO- Spherical coverage

6.2B.1.4D.2.1	Test purpose

Same test purpose as in clause 6.2D.1.2 in TS 38.521-2 [9] for the NR carrier.

6.2B.1.4D.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with UL MIMO.

6.2B.1.4D.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2D.1.2 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.2B.1.4.

6.2B.1.4D.2.4	Test description

Same test description as in clause 6.2D.1.2 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.2D.1.2 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2D.1.2 in TS 38.521-2 [9] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.4D.2.5	Test requirement

Same test requirement as in clause 6.2D.1.2 of TS 38.521-2 [9] for the NR carrier.

#### 6.2B.1.5	UE Maximum Output Power for Inter-Band EN-DC including both FR1 and FR2

6.2B.1.5.1	Test purpose

Same test purpose as in clause 6.2.1.1 in TS 38.521-1 [8] for NR FR1 carrier and 6.2.1.1 in TS 38.521-2 [9] for NR FR2 carrier.

6.2B.1.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for maximum output power apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 6.2B.

#### 6.2B.1.5D	UE Maximum Output Power for Inter-Band EN-DC including both FR1 and FR2 for UL MIMO

6.2B.1.5D.1	Test purpose

Same test purpose as in clause 6.2.1.1 in TS 38.521-1 [8] for NR FR1 carrier and 6.2.1.1 in TS 38.521-2 [9] for NR FR2 carrier.

6.2B.1.5D.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for maximum output power apply and are tested as part of the EN-DC within FR1 as in clause 6.2.1 in TS 38.521-1 [8] and EN-DC within FR2 as in clause 6.2.1 in TS 38.521-2 [9].

### 6.2B.1.6	UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps

Editor's note:	This clause is incomplete. The associated standalone test 6.2.5 in TS 38.521-2 [9] is incomplete as the following aspects are either missing of not yet determined:

-	The test case is incomplete for band n259.

-	Initial conditions are pending analysis for PC1, PC5, PC6 and PC7.

-	Impact of polarization on the measurements is still FFS.

-	MU and TT and structure of the Test Requirements Section are pending further analysis

6.2B.1.6.1	Test purpose

Same test purpose as in clause 6.2.5 in TS 38.521-2 [9] for the NR carrier.

6.2B.1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC along with support for UL-gaps

6.2B.1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2.5 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.1.6.

6.2B.1.6.4	Test description

Same test description as in clause 6.2.5 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.2.5 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of initial conditions as in clause 6.2.5 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2.5 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.1.6.5	Test requirements

Same test requirement as in clause 6.2.5 in TS 38.521-2 [9] for the NR carrier.

### 6.2B.2	UE Maximum Output Power reduction for EN-DC

#### 6.2B.2.0	General

The UE maximum output power reduction (MPR) specified in this clause is applicable for UEs configured with EN-DC when NS_01 is indicated in the MCG and the SCG. The MPR applies subject to indication in the field modifiedMPRbehavior for the SCG [2].

#### 6.2B.2.1	UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC

6.2B.2.1.1	Test purpose

Same test purpose as in clause 6.2.2.1 in TS 38.521-1 [8] for the NR carrier.

6.2B.2.1.2	Test applicability

This test case applies to all types of E-UTRA power class 3 and power class 2 UE release 15 and forward, supporting intra-band contiguous EN-DC.

NOTE:	Test execution is not necessary if clause 6.5B.2.1.3 ACLR is executed since MPR requirement is verified in this test case.

6.2B.2.1.3	Minimum conformance requirements

6.2B.2.1.3.1	General

When the UE is configured for intra-band contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this clause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [5]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPRENDC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRENDC )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [5]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPRENDC )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPRENDC )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [5]

MPRENDC is defined in Clause 6.2B.2.1.3.2.

6.2B.2.1.3.2	MPR for power class 3 and power class 2

MPR in this subclause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with ENDC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in subclause 6.2.4 of TS 36.101 [5] and 6.2.2 of TS 38.101-1 [2] apply. For a UE supporting dynamic power sharing for DC_(n)71AA for which dual simultaneous uplink transmissions are mandatory and A-MPR defined in subclause 6.2B.3.1.1 is applied as MPR. The allowed maximum output power reduction for IM3 related emissions applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA =		15;		0 ≤ B < 0.5

10;		0.5 ≤ B < 1.0

8;		1.0 ≤ B < 2.0

6;		2.0 < B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.1.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR. LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

No exception requirements for NR or E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that supports dynamic power sharing. LTE anchor agnostic approach is not applied for this case based on the test point analysis in TS 38.905 [7].

6.2B.2.1.4	Test description

6.2B.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on NR operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.4B.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521.1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.2.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1. |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1. |  |  |  |  | Low range, High range |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1. |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1. |  |  |  |  | Lowest, Highest (NOTE 10) |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (NOTE 1) | Power config (NOTE 8) |
| 1 | Default | Default | Default | N/A | 16QAM | Outer_Full | DFT-s-OFDM Pi/2 BPSK | Outer_Full | B |
| 2 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | B |
| 3 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 4 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | B |
| 6 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 8 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 9 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 10 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 11 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 12 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 13 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 14 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 15 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 16QAM | Outer_Full | B |
| 16 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | Edge_1RB_Right | B |
| 17 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | N/A | A |
| 18 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Right | A |
| 19 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | Edge_1RB_Left | B |
| 20 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Left | A |
| 21 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | N/A | A |
| 22 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 64QAM | Outer_Full | B |
| 23 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 64QAM | Edge_1RB_Right | B |
| 24 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 64QAM | Edge_1RB_Left | B |
| 25 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 256QAM | Outer_Full | B |
| 26 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 256QAM | Edge_1RB_Right | B |
| 27 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 256QAM | Edge_1RB_Left | B |
| 28 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM QPSK | Outer_Full | B |
| 29 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right | B |
| 30 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | N/A | A |
| 31 (Note 3) | High |  |  |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Right | A |
| 32 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Left | B |
| 33 (Note 4) | Low |  |  |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Left | A |
| 34 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | N/A | A |
| 35 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 16QAM | Outer_Full | B |
| 36 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | Edge_1RB_Right | B |
| 37 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | N/A | A |
| 38 (Note 3) | High |  |  |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Right | A |
| 39 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | Edge_1RB_Left | B |
| 40 (Note 4) | Low |  |  |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Left | A |
| 41 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | N/A | A |
| 42 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 64QAM | Outer_Full | B |
| 43 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 64QAM | Edge_1RB_Right | B |
| 44 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 64QAM | Edge_1RB_Left | B |
| 45 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 46 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 47 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 48 (Note 4) | Default |  |  |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Edge_Full_Right is defined as 2 RBs allocated at the right edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1 NOTE 7: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. NOTE 8: Power config as specified in Table 6.2B.2.1.4.3-3 and Table 6.2B.2.1.4.3-4 for PC3 UE or Table 6.2B.2.1.4.3-5 and Table 6.2B.2.1.4.3-6 for PC2 UE. NOTE 9: Test IDs with simultaneous E-UTRA and NR UL transmission don’t apply to DC_(n)71AA for a UE supporting dynamic power sharing (A-MPR is applied as MPR and covered by 6.2B.3.1.1). NOTE 10: For DC_(n)71AA, only NR SCS of 15 kHz is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.2.1.4.3.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2B.2.1.4.1-1 on E-UTRA CC and NR CC respectively. For test points configured with message in Table 6.2B.2.1.4.3-7, NR SS only schedules UL RMC on NR slots that does not overlap with E-UTRA uplink subframe. Since the UL has no payload and no loopback data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command starting in this step for the UE to reach PUMAX level.

3.	For a UE supporting dynamic power sharing, measure the mean power over all component carriers. For a UE not supporting dynamic power sharing, measure the power of each component carrier individually. For the tested EN-DC configuration, the requirements described in clause 6.2B.2.1.5 shall be met. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	For UEs supporting Power Class 2, repeat steps 1~3 for Test ID 2 and 4 in Table 6.2B.2.1.4.1-1 on the applicable bands with message exception defined in Table 6.2B.2.1.4.3-7.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2B.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 with the following exceptions:

Table 6.2B.2.1.4.3-1: AdditionalSpectrumEmission for MCG

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 (NS_01) |  |  |

Table 6.2B.2.1.4.3-2: AdditionalSpectrumEmission for SCG

| Derivation Path: 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 (NS_01) |  |  |

Table 6.2B.2.1.4.3-3: PhysicalCellGroupConfig for PC3

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power config A (NOTE 1) |
|  | 20 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.2.1.4.3-4: RRCConnectionReconfiguration: nr-Config-r15 for PC3

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power config A (NOTE 1) |
|  | 20 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.2.1.4.3-5: PhysicalCellGroupConfig for PC2

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 26 |  | Power config A (NOTE 1) |
|  | 23 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.2.1.4.3-6: RRCConnectionReconfiguration: nr-Config-r15 for PC2

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 26 |  | Power config A (NOTE 1) |
|  | 23 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

6.2B.2.1.4.3-7: RRCConnectionReconfiguration: p-MaxUE-FR1-r15 (step 4 in 6.2B.2.1.4.2)

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 2 UE |
| } |  |  |  |

Table 6.2B.2.1.4.3-8: RRCConnectionReconfiguration: tdm-PatternConfig if operating on FDD band

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 ::= CHOICE{ |  |  | Power config A (NOTE 1) |
| setup :: = SEQUENCE { |  | Apply if operating on FDD band for a UE NOT indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE according to TS 38.213 [x] clause 7.6.1 |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. |  |  |  |

6.2B.2.1.4.3-9: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.2B.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the UE Power Class, the nominal maximum output power and tolerance in Table 6.2B.2.1.5-1 ~ Table 6.2B.2.1.5-6a.

The maximum output power, derived in step 4 shall be within the range prescribed by Power Class 3, the nominal maximum output power and tolerance in  Table 6.2B.2.1.5-1 ~ Table 6.2B.2.1.5-3a.

Table 6.2B.2.1.5-1: UE Power Class 3 test requirements, UE supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 3.0 | 20.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 14.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15 | 11.0 | 12.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 6.0 - TT |
|  | 30, 60 | 6.0 | 17.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 12.0 - TT |
| 48 (NOTE 3) | 15 | 7.0 | 16.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 11.0 - TT |
|  | 30, 60 | 5.0 | 18.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 13.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-2: UE Power Class 3 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | N/A | 1.0 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 3.5 | 19.5 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 17.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 1 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 2 | 21.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 19.0 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3 | 20.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 18.0 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-3: UE Power Class 3 E-UTRA carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MCG MPRc (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 6.0 | 17.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 12.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 14.0 | 9.0 | 20.0 | 6.0 | 2.5 | 22.5 + TT | 3.0 - TT |
| 48 (NOTE 3) | 15, 30, 60 | 9.0 | 14.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 9.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-3a: UE Power Class 3 NR carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | SCG MPR’c (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 6.0 | 17.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 12.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15 | 14.0 | 9.0 | 20.0 | 6.0 | 2.5 | 22.5 + TT | 3.0 - TT |
|  | 30, 60 | 9.0 | 14.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 9.0- TT |
| 48 (NOTE 3) | 15, 30 | 9.0 | 14.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 9.0- TT |
|  | 60 | 7.0 | 16.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 11.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-4: UE Power Class 2 test requirements, UE supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 3.0 | 23.0 | 26.0 | 3.0 | 2.0 | 28.0 + TT | 20.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15 | 11.0 | 15.0 | 26.0 | 6.0 | 2.0 | 28.0 + TT | 9.0 - TT |
|  | 30, 60 | 6.0 | 20.0 | 26.0 | 6.0 | 2.0 | 28.0 + TT | 14.0 - TT |
| 48 (NOTE 3) | 15 | 7.0 | 19.0 | 26.0 | 5.0 | 2.0 | 28.0 + TT | 14.0 - TT |
|  | 30, 60 | 5.0 | 21.0 | 26.0 | 5.0 | 2.0 | 28.0 + TT | 16.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-5: UE Power Class 2 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC2 on NR band Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16))

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | N/A | 1.0 | 25.0 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 23.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-5a: UE Power Class 2 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16))

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | N/A | 1.0 | 25.0 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 23.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 0.5 | 22.5 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 1.0 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 2.0 | 21.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 19.0 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3.0 | 20.0 | 23.0 | 2.5 | 2.0 | 25.0 + TT | 17.5 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-6: UE Power Class 2 E-UTRA carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MCG MPRc (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 6.0 | 20.0 | 23.0 | 2.5 | 2.0 | 25.0 + TT | 17.5 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 14.0 | 12.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 6.0 - TT |
| 48 (NOTE 3) | 15, 30, 60 | 9.0 | 17.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 12.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-6a: UE Power Class 2 NR carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | SCG MPR’c (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 6.0 | 20.0 | 23.0 | 2.5 | 2.0 | 25.0 + TT | 17.5 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15 | 14.0 | 12.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 6.0 - TT |
|  | 30, 60 | 9.0 | 17.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 12.0 - TT |
| 48 (NOTE 3) | 15, 30 | 9.0 | 17.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 12.0 - TT |
|  | 60 | 7.0 | 19.0 | 23.0 | 3.5 | 2.0 | 25.0 + TT | 15.5 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-7: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| ENBW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < ENBW ≤ 100MHz | 1.0 dB | 1.0 dB |

#### 6.2B.2.2	UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC

6.2B.2.2.1	Test purpose

Same test purpose as in clause 6.2B.2.1.1.

6.2B.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

NOTE:	Test execution is not necessary if clause 6.5B.2.2.3 ACLR is executed since MPR requirement is verified in this test case.

6.2B.2.2.3	Minimum conformance requirements

6.2B.2.2.3.1	General

When the UE is configured for intra-band non-contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this subclause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [4]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPRENDC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRENDC )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPRENDC )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPRENDC )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

MPRENDC is defined in Clause 6.2B.2.2.3.2.

6.2B.2.2.3.2	MPR for power class 3 and power class 2

MPR in this subclause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with ENDC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in subclause 6.2.4 of TS 36.101 [5] and 6.2.3 of TS 38.101-1 [2] apply. The allowed maximum output power reduction for IM3 related emissions applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA =	18	;	0 ≤ B < 1.0

17	;	1.0 ≤ B < 2.0

16	;	2.0 ≤ B < 5.0

15	;	5.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B= (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000.000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12 * SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.2.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA overlap in time with NR. LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

No exception requirements for NR or E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR. LTE anchor agnostic approach is not applied for this case based on the test point analysis in TS 38.905 [7].

6.2B.2.2.4	Test description

6.2B.2.2.4.1	Initial conditions

Same initial conditions as described in clause 6.2B.2.1.4.1 for both E-UTRA and NR carriers with the following exception:

-	Instead of Table 6.2B.2.1.4.1-1 --> use Table 6.2B.2.2.4.1-1.

Table 6.2B.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1. |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508 [7] clause 4.3.1 for different DC bandwidth classes |  |  |  |  | Low with maxWgap (NR low – E-UTRA high); High with maxWgap (E-UTRA low – NR high) |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.3-1. |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1. |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  |  |  | Modulation | RB allocation (Note 3) | Modulation | RB allocation (NOTE 1) | Power config (NOTE 6) |
| 1 | Default | Default | Default | N/A | QPSK | Outer_Full | DFT-s-OFDM Pi/2 BPSK | Outer_Full | B |
| 2 | High |  |  |  | QPSK | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | B |
| 3 | High |  |  |  | QPSK | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 4 | High |  |  |  | QPSK | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | A |
| 5 | Low |  |  |  | QPSK | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | B |
| 6 | Low |  |  |  | QPSK | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | A |
| 7 | Low |  |  |  | QPSK | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 8 | Default |  |  |  | QPSK | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 9 | High |  |  |  | QPSK | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 10 | High |  |  |  | QPSK | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 11 | High |  |  |  | QPSK | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 12 | Low |  |  |  | QPSK | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 13 | Low |  |  |  | QPSK | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 14 | Low |  |  |  | QPSK | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 15 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 16QAM | Outer_Full | B |
| 16 | High |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | Edge_1RB_Right | B |
| 17 | High |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | N/A | A |
| 18 | High |  |  |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Right | A |
| 19 | Low |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | Edge_1RB_Left | B |
| 20 | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Left | A |
| 21 | Low |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | N/A | A |
| 22 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 64QAM | Outer_Full | B |
| 23 | High |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 64QAM | Edge_1RB_Right | B |
| 24 | Low |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 64QAM | Edge_1RB_Left | B |
| 2 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 256QAM | Outer_Full | B |
| 26 | High |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 256QAM | Edge_1RB_Right | B |
| 27 | Low |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 256QAM | Edge_1RB_Left | B |
| 28 | Default |  |  |  | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full | B |
| 29 | High |  |  |  | QPSK | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right | B |
| 30 | High |  |  |  | QPSK | Outer_1RB_Left | CP-OFDM QPSK | N/A | A |
| 31 | High |  |  |  | QPSK | N/A | CP-OFDM QPSK | Edge_1RB_Right | A |
| 32 | Low |  |  |  | QPSK | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Left | B |
| 33 | Low |  |  |  | QPSK | N/A | CP-OFDM QPSK | Edge_1RB_Left | A |
| 34 | Low |  |  |  | QPSK | Outer_1RB_Right | CP-OFDM QPSK | N/A | A |
| 3 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 16QAM | Outer_Full | B |
| 36 | High |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | Edge_1RB_Right | B |
| 37 | High |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | N/A | A |
| 38 | High |  |  |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Right | A |
| 39 | Low |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | Edge_1RB_Left | B |
| 40 | Low |  |  |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Left | A |
| 41 | Low |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | N/A | A |
| 42 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 64QAM | Outer_Full | B |
| 43 | High |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 64QAM | Edge_1RB_Right | B |
| 44 | Low |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 64QAM | Edge_1RB_Left | B |
| 45 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 46 | High |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 47 | Low |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 48 | Low |  |  |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Edge_Full_Right is defined as 2 RBs allocated at the right edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 4: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1 NOTE 5: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. NOTE 6: Power config as specified in Table 6.2B.2.1.4.3-3 and Table 6.2B.2.1.4.3-4 for PC3 UE or Table 6.2B.2.1.4.3-5 and Table 6.2B.2.1.4.3-6 for PC2 UE. |  |  |  |  |  |  |  |  |  |

6.2B.2.2.4.2	Test procedure

Same test procedure as described in clause 6.2B.2.1.4.2.

6.2B.2.2.4.3	Message contents

Same message contents as in clause 6.2B.2.1.4.3.

6.2B.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.2.2.5-1 ~ Table 6.2B.2.2.5-8a.

Table 6.2B.2.2.5-1: UE Power Class 3 test requirements, UE supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 12.0 | 11.0 | 23.0 | 6.0 | 4.0 | 27.0 + TT | 5.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15,30, 60 | 15.0 | 8.0 | 23.0 | 7.0 | 4.0 | 27.0 + TT | 1.0 - TT |
| 48 (NOTE 3) | 15 | 14.0 | 9.0 | 23.0 | 7.0 | 4.0 | 27.0 + TT | 2.0 - TT |
|  | 30, 60 | 13.0 | 10.0 | 23.0 | 7.0 | 4.0 | 27.0 + TT | 3.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-2: UE Power Class 3 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | 15, 30, 60 | 1.0 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 3.5 | 19.5 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 17.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 1 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 2 | 21.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 19.0 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3 | 20.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 18.0 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-3: UE Power Class 3 E-UTRA carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MCG MPRc (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 15.0 | 8.0 | 20.0 | 6.0 | 2.5 | 22.5 + TT | 2.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 17.0 | 6.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | -1.0 - TT |
| 48 (NOTE 3) | 15, 30, 60 | 17.0 | 6.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | -1.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-3a: UE Power Class 3 NR carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | SCG MPR’c (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 15.0 | 8.0 | 20.0 | 6.0 | 2.5 | 22.5 + TT | 2.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 17.0 | 6.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | -1.0 - TT |
| 48 (NOTE 3) | 15, 30 | 17.0 | 6.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | -1.0 - TT |
|  | 60 | 16.0 | 7.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-4: Void

Table 6.2B.2.2.5-5: UE Power Class 2 test requirements, UE supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 12.0 | 14.0 | 26.0 | 6.0 | 4.0 | 30.0 + TT | 8.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15,30, 60 | 15.0 | 11.0 | 26.0 | 6.0 | 4.0 | 30.0 + TT | 5.0 - TT |
| 48 (NOTE 3) | 15 | 14.0 | 12.0 | 26.0 | 6.0 | 4.0 | 30.0 + TT | 6.0 - TT |
|  | 30, 60 | 13.0 | 13.0 | 26.0 | 6.0 | 4.0 | 30.0 + TT | 7.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-6: UE Power Class 2 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16))

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | 15, 30, 60 | 1.0 | 25.0 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 23.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-6a: UE Power Class 2 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16))

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | 15, 30, 60 | 1.0 | 25.0 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 23.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 0.5 | 22.5 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 1.0 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 2.0 | 21.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 19.0 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3.0 | 20.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 17.5 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-7: UE Power Class 2 E-UTRA carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MCG MPRc (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 15.0 | 11.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 5.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 17.0 | 9.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 3.0 - TT |
| 48 (NOTE 3) | 15, 30, 60 | 17.0 | 9.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 3.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-7a: UE Power Class 2 NR carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | SCG MPR’c (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 15.0 | 11.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 5.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 17.0 | 9.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 3.0 – TT |
| 48 (NOTE 3) | 15, 30 | 17.0 | 9.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 3.0 – TT |
|  | 60 | 16.0 | 10.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 4.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-8: Void

Table 6.2B.2.2.5-8a: Void

Table 6.2B.2.2.5-9: Test Tolerance for UE maximum output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.2.2.5-9a: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

#### 6.2B.2.3	UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 (1 NR CC)

6.2B.2.3.1	Test purpose

Same test purpose as in clause 6.2.2.1 in TS 38.521-1 [8] for the NR carrier.

6.2B.2.3.2	Test applicability

The requirements of this test apply to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

NOTE:	Test execution is not necessary if clause 6.5B.2.3.3 ACLR is executed since MPR requirement is verified in this test cases.

6.2B.2.3.3	Minimum conformance requirements

For inter-band EN-DC between E-UTRA and NR FR1, UE maximum output power reduction specified in TS 36.101 [5] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

6.2B.2.3.4	Test description

Same test description as in clause 6.2.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.2.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.2.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.2.2.4.2 in TS 38.521-1 [8].

6.2B.2.3.5	Test requirement

For PC3 UE, same test requirement as in clause 6.2.2.5 in TS 38.521-1 [8].

For PC2 UE (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16), same test requirement for PC2 UE as in clause 6.2.2.5 in TS 38.521-1 [8]. For PC2 UE (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16)), same test requirement for PC3 UE as in clause 6.2.2.5 in TS 38.521-1 [8].

#### 6.2B.2.3a	UE Maximum Output Power reduction for Inter-Band NE-DC within FR1 (1 NR CC)

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier in this test case are tested in 6.2.2 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.2.3 and 6.2.3A of TS 36.521-1 [10]. Neither NR carrier nor LTE carrier(s) needs to be tested again.

#### 6.2B.2.4	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC)

Editor's note:	Following aspects are missing or under discussion

-	The referred test case 6.2.2 in TS 38.521-2 [9] is incomplete for PC2 and PC4.

6.2B.2.4.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified maximum output power with MPR and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.2.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.2B.2.4.3	Minimum conformance requirements

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101[5] and for NR single carrier and CA operation specified in clauses 6.2.2, 6.2A.2, and 6.2D.2 of TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.4.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.2B.2.4.4	Test description

6.2B.2.4.4.1	Initial conditions

Same test description as in clause 6.2.2.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.7-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C, clauses C.0, C.1 and C.3.0, and uplink signals according to Annex H, clauses H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.7-1.

Step 6 of Initial conditions as in clause 6.2.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

Same test procedure as in clause 6.2.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.2.4.5	Test requirement

Same test requirement as in clause 6.2.2.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.2B.2.4a	UE Maximum Output Power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC)

6.2B.2.4a.1	Test purpose

Same test purpose as in clause 6.2.2_1 in TS 38.521-2 [9] for the NR carrier.

6.2B.2.4a.2	Test applicability

This test case applies to:

-All types of E-UTRA PC3 UE release 15 supporting inter-band EN-DC including FR2 with 1 NR UL CC and modifiedMPRbehaviour bit 0 capability (according to Annex P.1 in TS38.521-2 [9]).

- All types of E-UTRA PC3 UE release 16 and forward supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.2B.2.4a.3	Minimum conformance requirements

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101[5] and for NR single carrier and CA operation specified in clauses 6.2.2, 6.2A.2, and 6.2D.2 of TS 38.101-2 [3] apply.

Same minimum conformance requirements as in clause 6.2.2_1.3 in TS 38.521-2 [9] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.4.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.2B.2.4a.4	Test description

Same test description as in clause 6.2.2_1.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.2.2_1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C, clauses C.0, C.1 and C.3.0, and uplink signals according to Annex H, clauses H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.2.2_1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

Same test procedure as in clause 6.2.2_1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.2.4a.5	Test requirement

Same test requirement as in clause 6.2.2_1.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.2B.2.4D	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC)

Editor's note:	Following aspects are missing or under discussion

-	The referred test case 6.2D.2 in TS 38.521-2 [9] is incomplete.

6.2B.2.4D.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified maximum output power with MPR and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.2.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC with UL MIMO.

6.2B.2.4D.3	Minimum conformance requirements

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101[5] and for NR single carrier and CA operation specified in clauses 6.2.2, 6.2A.2, and 6.2D.2 of TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.4.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

6.2B.2.4D.4	Test description

6.2B.2.4D.4.1	Initial conditions

Same test description as in clause 6.2D.2.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.2D.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C, clauses C.0, C.1 and C.3.0, and uplink signals according to Annex H, clauses H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.2D.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

Same test procedure as in clause 6.2D.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.2.4.5	Test requirement

Same test requirement as in clause 6.2D.2.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.2B.2.4_1	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (>1 NR CC)

##### 6.2B.2.4_1.1	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (2 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.2.1 in TS 38.521-2 [9] is incomplete for aggregated BW > 400MHz and intra-band non-contiguous CA.

-	The referred test case 6.2A.2.1 in TS 38.521-2 [9] is incomplete for power class 1, 2 and 4.

6.2B.2.4_1.1.1	Test purpose

Same test purpose as in clause 6.2.2.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.2.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.2B.2.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2B.2.4.3.

6.2B.2.4_1.1.4	Test description

6.2B.2.4_1.1.4.1	Initial condition

Same test description as in clause 6.2A.2.1.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.2.4_1.1.5	Test Requirements

Same test requirement as in clause 6.2A.2.1.5 in TS 38.521-2 [9] for the NR carriers.

##### 6.2B.2.4_1.2	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (3 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.2.2 in TS 38.521-2 [9] is incomplete for aggregated BW > 400MHz and intra-band non-contiguous CA.

-	The referred test case 6.2A.2.2 in TS 38.521-2 [9] is incomplete for power class 1, 2 and 4.

6.2B.2.4_1.2.1	Test purpose

Same test purpose as in clause 6.2.2.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.2.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.2B.2.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2B.2.4.3.

6.2B.2.4_1.2.4	Test description

6.2B.2.4_1.2.4.1	Initial condition

Same test description as in clause 6.2A.2.2.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.2.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.2.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.2.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.2.4_1.2.5	Test Requirements

Same test requirement as in clause 6.2A.2.2.5 in TS 38.521-2 [9] for the NR carriers.

##### 6.2B.2.4_1.3	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (4 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.2.3 in TS 38.521-2 [9] is incomplete for aggregated BW > 400MHz and intra-band non-contiguous CA.

-	The referred test case 6.2A.2.3 in TS 38.521-2 [9] is incomplete for power class 1, 2 and 4.

6.2B.2.4_1.3.1	Test purpose

Same test purpose as in clause 6.2.2.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.2.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.2B.2.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2B.2.4.3.

6.2B.2.4_1.3.4	Test description

6.2B.2.4_1.3.4.1	Initial condition

Same test description as in clause 6.2A.2.3.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.2A.2.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2A.2.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.2A.2.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.2.4_1.3.5	Test Requirements

Same test requirement as in clause 6.2A.2.3.5 in TS 38.521-2 [9] for the NR carriers.

#### 6.2B.2.5	UE Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2

6.2B.2.5.1	Test purpose

Same test purpose as in clause 6.2B.2.3.1 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.2B.2.4.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.2B.2.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for maximum output power apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 6.2B.

#### 6.2B.2.5D	UE Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2 for UL MIMO

6.2B.2.5D.1	Test purpose

Same test purpose as in clause 6.2D.2.1 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.2D.2.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.2B.2.5D.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for maximum output power apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 6.2B.

### 6.2B.3	UE additional maximum output power reduction for EN-DC

#### 6.2B.3.1	UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC

Editor's note:

-	Test requirements for non-overlapping transmission of non-DPS UE need further investigation

6.2B.3.1.1	Test purpose

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2B.1.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2B.3.1.2 	Test applicability

The requirements of this test apply in test case 6.5B.2.1.2 Additional spectrum emission mask for network signalled values NS_04 and NS_35 to all types of E-UTRA power class 3 and power class 2 UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.2B.3.1.3	Minimum conformance requirements

For intra-band contiguous EN-DC band combinations with additional requirements the allowed A-MPR is specified in table 6.2B.3.1.3-1 for UEs configured with EN-DC and combinations of network signalling values indicated in the E-UTRA and NR cell groups.

Unless otherwise stated the A-MPR specified in clause 6.2B.3.1 for intra-band contiguous EN-DC configurations is the total power reduction allowed including MPR.

Table 6.2B.3.1.3-1: Additional maximum power reduction for Intra-band contiguous EN-DC

| DC configuration | Requirement (subclause) | E-UTRA network signalling value | NR network signalling value | A-MPR (clause) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA | 6.5B.2.1.2.3.1 | NS_35 | NS_35 | 6.2B.3.1.3.13 |
| DC_(n)41AA1 | 6.5B.2.1.2.3.2 | NS_01 or NS_04 | NS_04 | 6.2B.3.1.3.24 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The additional emission requirement is indicated when the combination of network signalling values in the two CGs is set (only for UEs configured with EN-DC). NOTE 3: The A-MPR is applied as MPR if NS_35 is not signalled. NOTE 4: Void. |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.3.1.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA overlap in time with NR.LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that doesn't support dynamic power sharing. LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

No exception requirements for NR or E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that supports dynamic power sharing. LTE anchor agnostic approach is not applied for this case.

6.2B.3.1.3.1	A-MPR for DC_(n)71AA

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with TS 36.101 [5]

-	for the SCG, A-MPR'c = [A-MPRDC]

-	for the total configured transmission power, A-MPRtot = A-MPRDC

with A-MPRDC as defined in this subclause.

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = A-MPRE-UTRA

-	for the SCG,

A-MPR'c = A-MPRNR

with A-MPRE-UTRA and A-MPRNR as defined in this subclause.

For DC_(n)71AA with configured with network signaling values as per Table 6.2B.3.1.0-1 the allowed A-MPR is defined by

-	for UE indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE

A-MPRDC = CEIL{ MA,DC (A), 0.5}

where A-MPRDC is the total power reduction allowed (dB),

-	for OFDM:

MA,DC =	11.00 - 11.67*A;		0.00 < A ≤ 0.30

8.10 - 2.00*A;					0.30 < A ≤ 0.80

6.50;								0.80 < A ≤ 1.00

-	for DFT-S-OFDM:

MA,DC =	11.00 - 13.33*A;		0.00 < A ≤ 0.30

8.00 - 3.33*A;					0.30 < A ≤ 0.60

6.00;								0.60 < A ≤ 1.00

where:

with LCRB, E-UTRA and NRB, E-UTRA the number of allocated PRB and transmission bandwidth for MCG, LCRB,NR and NRB,NR the number of allocated PRB and transmission bandwidth for SCG with SCS = 15 kHz.

-	for UE not indicating support of dynamicPowerSharing

A- MPRE-UTRA = CEIL{ MA, E-UTRA, 0.5}

A-MPRNR = CEIL{ MA,NR, 0.5}

where A-MPR is the total power reduction allowed per CG with

MA,E-UTRA=MA,DC(AE-UTRA,wc)-1-E-UTRAMA,NR=MA,DC(ANR,wc)-1-NRAE-UTRA,wc=LCRB,E-UTRA+1NRB,E-UTRA+NRB,NRANR,wc=1+LCRB,NRNRB,E-UTRA+NRB,NR∆E-UTRA=10 log10NRB,E-UTRANRB,E-UTRA+NRB,NR∆NR=10 log10NRB,NRNRB,E-UTRA+NRB,NR	MA,E-UTRA=MA,DC(AE-UTRA,wc)-1-E-UTRA

MA,NR=MA,DC(ANR,wc)-1-NR

AE-UTRA,wc=LCRB,E-UTRA+1NRB,E-UTRA+NRB,NR

ANR,wc=1+LCRB,NRNRB,E-UTRA+NRB,NR

a??E-UTRA=10 log10NRB,E-UTRANRB,E-UTRA+NRB,NR

a??NR=10 log10NRB,NRNRB,E-UTRA+NRB,NR

Where LCRB,NR and NRB,NR the number of allocated PRB and transmission bandwidth for SCG with SCS = 15 kHz.

6.2B.3.1.3.2	A-MPR for NS_04

6.2B.3.1.3.2.0	General

When the UE is configured for B41/n41 intra-band contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this clause. The A-MPR for EN-DC defined in this clause is used instead of MPR defined in 6.2B.2.2, not additively, so EN-DC MPR = 0 when NS_04 is signaled. For UEs scheduled with single uplink transmission, AMPR in subclause 6.2.4 of [5] and 6.2.3 of [2] apply.

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with 36.101 [5]

-	for the SCG,

A-MPR'c = A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3)

-	for the total configured transmission power,

A-MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - A-MPRE-UTRA)/10) + 10^((PPowerClass,NR - A-MPRNR)/10))

where

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA + MPRsingle,E-UTRA, A-MPRIM3 )

with

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 36.101 [5]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [5]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = MAX( A-MPRsingle, E-UTRA + MPRsingle,E-UTRA, A-MPRIM3 )

-	for the SCG,

A-MPR'c = MAX( A-MPRsingle,NR, A-MPRIM3 )

where

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 36.101 [5]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [5]

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If FIM3,low_block,low < 2490.5 MHz

Channel Configuration Case B. A-MPRIM3 defined in clause 6.2B.3.1.3.2.2.

Else

Channel Configuration Case A. A-MPRIM3 defined in clause 6.2B.3.1.3.2.1.

where

-	FIM3,low_block,low = (2 * Flow_channel,low_edge) – Fhigh_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

Where the transmission bandwidth configuration for NR is the maximum frequency span covering all the configured SCSSpecificCarrier for scenarios that carrier bandwidths with different SCS can be fully overlapped.

6.2B.3.1.3.2.1	A-MPRIM3 for NS_04 to meet -13 dBm / 1MHz for 26dBm UE power

A-MPR in this subclause is relative to 26 dBm for a power class 2 Cell Group. The same A-MPR is used relative to 23 dBm for a power class 3 Cell Group. For the UE is configured with channel configurations Case A or Case C (defined in Clause 6.2B.3.2.3.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 15 ; 0 ≤ B < 0.5

10 ; 0.5 ≤ B < 1.0

8 ; 1.0 ≤ B < 2.0

6 ; 2.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR =15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA =15 kHz is assumed in calculation of B

and MA is reduced by 1 dB for B < 2.0.

6.2B.3.1.3.2.2	A-MPR for NS_04 to meet -25 dBm / 1MHz for 26 dBm UE power

A-MPR in this subclause is relative to 26 dBm for a power class 2 Cell Group. The same A-MPR is used relative to 23 dBm for a power class 3 Cell Group. For the UE is configured with channel configurations Case B or Case D (defined in clause 6.2B.3.2.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 15 ; 0 ≤ B < 1.0

14 ; 1.0 ≤ B < 2.0

13 ; 2.0 ≤ B < 5.0

12 ; 5.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR =15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA =15 kHz is assumed in calculation of B

and MA is reduced by 1 dB.

6.2B.3.1.4	Test description

6.2B.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on NR operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.2B.3.1.4.1-1 through 6.2B.3.1. 4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.3.1.4.1-0: Void

Table 6.2B.3.1.4.1-1: Test configuration table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  | Low range, High range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  | Lowest, Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (Note 1) | Power config (Note 8) |
| 1 | Default | N/A | 16QAM | Outer_Full | DFT-s-OFDM Pi/2 BPSK | Outer_Full | B |
| 2 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | B |
| 3 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 4 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | B |
| 6 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 8 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 9 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 10 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 11 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 12 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 13 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 14 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 15 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 16QAM | Outer_Full | B |
| 16 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | Edge_1RB_Right | B |
| 17 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | N/A | A |
| 18 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Right | A |
| 19 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | Edge_1RB_Left | B |
| 20 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Left | A |
| 21 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | N/A | A |
| 22 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 64QAM | Outer_Full | B |
| 23 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 64QAM | Edge_1RB_Right | B |
| 24 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 64QAM | Edge_1RB_Left | B |
| 25 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 256QAM | Outer_Full | B |
| 26 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 256QAM | Edge_1RB_Right | B |
| 27 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 256QAM | Edge_1RB_Left | B |
| 28 | Default |  | 16QAM | Outer_Full | CP-OFDM QPSK | Outer_Full | B |
| 29 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right | B |
| 30 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | N/A | A |
| 31 (Note 3) | High |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Right | A |
| 32 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Left | B |
| 33 (Note 4) | Low |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Left | A |
| 34 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | N/A | A |
| 35 | Default |  | 16QAM | Outer_Full | CP-OFDM 16QAM | Outer_Full | B |
| 36 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | Edge_1RB_Right | B |
| 37 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | N/A | A |
| 38 (Note 3) | High |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Right | A |
| 39 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | Edge_1RB_Left | B |
| 40 (Note 4) | Low |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Left | A |
| 41 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | N/A | A |
| 42 | Default |  | 16QAM | Outer_Full | CP-OFDM 64QAM | Outer_Full | B |
| 43 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 64QAM | Edge_1RB_Right | B |
| 44 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 64QAM | Edge_1RB_Left | B |
| 45 | Default |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 46 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 47 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 48 (Note 4) | Default |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Edge_Full_Right is defined as 2 RBs allocated at the right edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 7: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. NOTE 8: Power config as specified in Table 6.2B.3.1.4.3-1-1 to 6.2B.3.1.4.3-2 (PC3) or Table 6.2B.3.1.4.3-3 to 6.2B.3.1.4.3-4 (PC2). |  |  |  |  |  |  |  |

Table 6.2B.3.1.4.1-2: NR test configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  | Low range, High range (Note 7) |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  | Lowest, Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (Note 1) | Power config (Note 8) |
| 1 | Default | N/A | 16QAM | Outer_Full | DFT-s-OFDM Pi/2 BPSK | Outer_Full | B |
| 2 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | B |
| 3 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 4 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM PiI/2 BPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | B |
| 6 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 8 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 9 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 10 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 11 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 12 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 13 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 14 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 15 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 16QAM | Outer_Full | B |
| 16 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | Edge_1RB_Right | B |
| 17 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | N/A | A |
| 18 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Right | A |
| 19 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | Edge_1RB_Left | B |
| 20 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Left | A |
| 21 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | N/A | A |
| 22 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 64QAM | Outer_Full | B |
| 23 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 64QAM | Edge_1RB_Right | B |
| 24 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 64QAM | Edge_1RB_Left | B |
| 25 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 256QAM | Outer_Full | B |
| 26 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 256QAM | Edge_1RB_Right | B |
| 27 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 256QAM | Edge_1RB_Left | B |
| 28 | Default |  | 16QAM | Outer_Full | CP-OFDM QPSK | Outer_Full | B |
| 29 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right | B |
| 30 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | N/A | A |
| 31 (Note 3) | High |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Right | A |
| 32 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Left | B |
| 33 (Note 4) | Low |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Left | A |
| 34 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | N/A | A |
| 35 | Default |  | 16QAM | Outer_Full | CP-OFDM 16QAM | Outer_Full | B |
| 36 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | Edge_1RB_Right | B |
| 37 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | N/A | A |
| 38 (Note 3) | High |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Right | A |
| 39 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | Edge_1RB_Left | B |
| 40 (Note 4) | Low |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Left | A |
| 41 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | N/A | A |
| 42 | Default |  | 16QAM | Outer_Full | CP-OFDM 64QAM | Outer_Full | B |
| 43 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 64QAM | Edge_1RB_Right | B |
| 44 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 64QAM | Edge_1RB_Left | B |
| 45 | Default |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 46 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 47 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 48 (Note 4) | Default |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Edge_Full_Right is defined as 2 RBs allocated at the right edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 7: Additional IM3 test frequencies may apply. NOTE 8: Power config as specified in Table 6.2B.3.1.4.3-1 to 6.2B.3.1.4.3-2 (PC3) or Table 6.2B.3.1.4.3-3 to 6.2B.3.1.4.3-4 (PC2). NOTE 9: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. |  |  |  |  |  |  |  |

Table 6.2B.3.1.4.1-3: Additional IM3 Test Frequencies for NS_04 intra-band contiguous EN-DC

| Additional Initial Condition |  |
| --- | --- |
| Additional IM3 Test Frequencies | if SCS 15 kHz, 15 kHz NR raster, and NR ChBw 40 MHz or 50 MHz then IM3 test frequencies as specified in Table 6.2B.3.1.4.1-4. if SCS 30 kHz, 30 kHz NR raster, and NR ChBw 40 MHz, 50 MHz or 60 MHz then IM3 test frequencies as specified in Table 6.2B.3.1.4.1-5. if SCS 60 kHz, 15 kHz NR raster, and NR ChBw 40 MHz, 50 MHz or 60 MHz then IM3 test frequencies as specified in Table 6.2B.3.1.4.1-6. |

Table 6.2B.3.1.4.1-4: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 15 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2600.400 | 40694 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2570.400 | 514080 | 2550.96 | 510192 | 0 | 15 | 6384 | 514080 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2620.400 | 40894 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2585.400 | 517080 | 2561.1 | 512220 | 0 | 15 | 6408 | 517080 |

Table 6.2B.3.1.4.1-5: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 30 kHz, 30 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2600.400 | 40694 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2570.400 | 514080 | 2550.96 | 510192 | 0 | 15 | 6384 | 514080 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2620.400 | 40894 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2585.400 | 517080 | 2561.1 | 512220 | 0 | 15 | 6408 | 517080 |
| E-UTRA: 20MHz + NR: 60MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2640.500 | 41095 | - | - | - | - | - | - |
|  | NR CC1 | 60 | 162 | Downlink & Uplink | IM3 | 2600.500 | 520100 | 2571.34 | 514268 | 0 | 30 | 6438 | 520100 |

Table 6.2B.3.1.4.1-6: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 60 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2600.400 | 40694 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 51 | Downlink & Uplink | IM3 | 2570.400 | 514080 | 2552.04 | 510408 | 0 | 15 | 6387 | 514080 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2620.400 | 40894 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 65 | Downlink & Uplink | IM3 | 2585.400 | 517080 | 2562 | 512400 | 0 | 15 | 6411 | 517080 |
| E-UTRA: 20MHz + NR: 60MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2640.500 | 41095 | - | - | - | - | - | - |
|  | NR CC1 | 60 | 79 | Downlink & Uplink | IM3 | 2600.500 | 520100 | 2572.06 | 514412 | 0 | 15 | 6435 | 520100 |

Editor's note:	The following lines belong at the end of clause 6.2B.3.1.4.1. As new tables are added to this clause, these lines should always follow the tables.

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for E-UTRA the cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG link respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.1.4.3.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 6.2B.3.1.4.1-1 or 6.2B.3.1.4.1-2 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers for the EN-DC configuration, which shall meet the requirements described in table 6.2B.3.1.5.1-1 through to 6.2B.3.1.5.2-6. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms). For TDD, only slots consisting of only UL symbols are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2B.3.1.4.1-1 or 6.2B.3.1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1, with the following exceptions.

Table 6.2B.3.1.4.3-1: RRCConnectionReconfiguration: nr-Config-r15 for PC3

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power config A (NOTE 1) |
|  | 20 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.3.1.4.3-2: PhysicalCellGroupConfig for PC3

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power config A (NOTE 1) |
|  | 20 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.3.1.4.3-3: RRCConnectionReconfiguration: nr-Config-r15 for PC2

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 26 |  | Power config A (NOTE 1) |
|  | 23 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.3.1.4.3-4: PhysicalCellGroupConfig for PC2

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 26 |  | Power config A (NOTE 1) |
|  | 23 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

6.2B.3.1.4.3-5: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.3.1.4.3-6: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE does not support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  | Power config A (NOTE 1) |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. |  |  |  |

6.2B.3.1.4.3.1	Message contents exceptions (network signalled value "NS_04")

Message contents are according to TS 38.508-1 [6] clause 4.6.1 with the following exceptions for NS_04:

Table 6.2B.3.1.4.3.1-1: AdditionalSpectrumEmission for MCG and "NS_04"

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_04) |  |  |

Table 6.2B.3.1.4.3.1-2: AdditionalSpectrumEmission for SCG and "NS_04"

| Derivation Path: TS 38.508-1 [6] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_04) |  |  |

6.2B.3.1.4.3.2	Message contents exceptions (network signalled value "NS_35")

Message contents are according to TS 38.508-1 [6] clause 4.6.1 with the following exceptions for NS_35:

Table 6.2B.3.1.4.3.2-1: AdditionalSpectrumEmission for MCG and "NS_35"

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_35) |  |  |

Table 6.2B.3.1.4.3.2-2: AdditionalSpectrumEmission for for SCG "NS_35"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_35) |  |  |

6.2B.3.1.5	Test requirement

Table: 6.2B.3.1.5-1: Test Tolerance for UE maximum output power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| ENBW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < ENBW ≤ 100MHz | 1.0 dB | 1.0 dB |

6.2B.3.1.5.1	Test requirement for network signalled value "NS_35"

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in table 6.2B.3.1.5.1-1. The allowed A-MPR values specified in table 6.2B.3.1.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.1.1.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2B.1.1.3-1 apply.

Table 6.2B.3.1.5.1-1: UE Power Class test requirements for network signalled value "NS_35"for UEs not supporting dynamic power sharing

| Test ID | Test freq. range | E-UTRA BW | NR BW | Modulation | PPowerClass (dBm) | ΔPPowerClass (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 6.0 | 0 | 17.0 | 5 | +2/-3 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 6.0 | 0 | 17.0 | 5 | +2/-3 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 6.0 | 0 | 17.0 | 5 | +2/-3 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 6.0 | 0 | 17.0 | 5 | +2/-3 | 25+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 10.5 | 0 | 12.5 | 6 | +2/-3 | 25+TT | 6.5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 10.5 | 0 | 12.5 | 6 | +2/-3 | 25+TT | 6.5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 3, 10, 17 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 12.5 | 0 | 10.5 | 6 | +2/-3 | 25+TT | 4.5-TT |
| 3, 14, 17 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 4, 11, 18 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 12.5 | 0 | 10.5 | 12.5 | +2/-3 | 25+TT | 4.5-TT |
| 4, 11, 18 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 16.0 | 0 | 7.0 | 16.0 | +2/-3 | 25+TT | 0-TT |
| 6, 13, 20 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 6, 13, 20 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 7, 14, 21 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 7, 14, 21 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 28, 35, 42, 45 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 28, 35, 42, 45 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 28, 35, 42, 45 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 28, 35, 42, 45 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 29, 35, 42, 45 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6.5-TT |
| 29, 35, 42, 45 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 29, 35, 42, 45 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6.5-TT |
| 29, 35, 42, 45 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 30, 34, 37, 41 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 30, 34, 37, 41 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 31, 33, 38, 40 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 31, 33, 38, 40 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |

Table 6.2B.3.1.5.1-1A: UE Power Class test requirements for network signalled value "NS_35" for UEs supporting dynamic power sharing

| Test ID | Test freq. range | E-UTRA BW | NR BW | Modulation | PPowerClass (dBm) | ΔPPowerClass (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25 | Low | 5 | 5 | E-UTRA | 23 | 0 | 8.5 | 0 | 14.5 | 5 | +2/-3 | 25+TT | 9.5-TT |
| 1, 8, 15, 22, 25 | Low | 5 | 5 | NR | 23 | 0 | 8.5 | 0 | 14.5 | 5 | +2/-3 | 25+TT | 9.5-TT |
| 1, 8, 15, 22, 25 | Low | 5 | 15 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 1, 8, 15, 22, 25 | Low | 5 | 15 | NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 1, 8, 15, 22, 25 | High | 5 | 5 | E-UTRA | 23 | 0 | 8.5 | 0 | 14.5 | 5 | +2/-3 | 25+TT | 9.5-TT |
| 1, 8, 15, 22, 25 | High | 5 | 5 | NR | 23 | 0 | 8.5 | 0 | 14.5 | 5 | +2/-3 | 25+TT | 9.5-TT |
| 1, 8, 15, 22, 25 | High | 15 | 5 | E-UTRA | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 1, 8, 15, 22, 25 | High | 15 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 5 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 15 | NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 5 | 5 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 15 | 5 | E-UTRA | 23 | 0 | 11.5 | 0 | 11.5 | 6 | +2/-3 | 25+TT | 5.5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 3, 10, 17 | Low | 5 | 5 | E-UTRA/ | 23 | 0 | 12.5 | 0 | 10.5 | 6 | +2/-3 | 25+TT | 4.5-TT |
| 3, 10, 17 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 4, 11, 18 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 4, 11, 18 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 6, 17, 20 | Low | 5 | 5 | E-UTRA/ | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 6, 17, 20 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.5 | 0 | 6.5 | 7 | +2/-3 | 25+TT | -0.5-TT |
| 7, 14, 21 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 7, 14, 21 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 28, 35, 42, 45 | Low | 5 | 5 | E-UTRA | 23 | 0 | 9.5 | 0 | 13.5 | 5 | +2/-3 | 25+TT | 8.5-TT |
| 28, 35, 42, 45 | Low | 5 | 5 | NR | 23 | 0 | 9.5 | 0 | 13.5 | 5 | +2/-3 | 25+TT | 8.5-TT |
| 28, 35, 42, 45 | Low | 5 | 15 | E-UTRA | 23 | 0 | 13.5 | 0 | 9.5 | 6 | +2/-3 | 25+TT | 3.5-TT |
| 28, 35, 42, 45 | Low | 5 | 15 | NR | 23 | 0 | 7.0 | 0 | 16.0 | 5 | +2/-3 | 25+TT | 11-TT |
| 28, 35, 42, 45 | High | 5 | 5 | E-UTRA | 23 | 0 | 9.5 | 0 | 13.5 | 5 | +2/-3 | 25+TT | 8.5-TT |
| 28, 35, 42, 45 | High | 5 | 5 | NR | 23 | 0 | 9.5 | 0 | 13.5 | 5 | +2/-3 | 25+TT | 8.5-TT |
| 28, 35, 42, 45 | High | 15 | 5 | E-UTRA | 23 | 0 | 7.0 | 0 | 16.0 | 5 | +2/-3 | 25+TT | 11-TT |
| 28, 35, 42, 45 | High | 15 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | Low | 5 | 5 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | Low | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 29, 36, 43, 46 | Low | 5 | 15 | NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 29, 36, 43, 46 | High | 5 | 5 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | High | 15 | 5 | E-UTRA | 23 | 0 | 11.5 | 0 | 11.5 | 6 | +2/-3 | 25+TT | 0-TT |
| 29, 36, 43, 46 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 5.5-TT |
| 30, 37 | Low | 5 | 5 | E-UTRA/ | 23 | 0 | 13.0 | 0 | 10.0 | 6 |  | 25+TT | 4-TT |
| 30, 37 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.0 | 0 | 7.0 | 7 |  | 25+TT | 0-TT |
| 31, 38 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 |  | 25+TT | 4-TT |
| 31, 38 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 |  | 25+TT | 0-TT |
| 34, 41 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 |  | 25+TT | 4-TT |
| 34, 41 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 |  | 25+TT | 0-TT |
| 33, 40 | Low | 5 | 5 | E-UTRA/ | 23 | 0 | 13.0 | 0 | 10.0 | 6 |  | 25+TT | 4-TT |
| 33, 40 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.5 | 0 | 6.5 | 7 |  | 25+TT | -0.5-TT |
| NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2B.3.1.5.2	Test requirement for network signalled value "NS_04"

Table 6.2B.3.1.5.2-1: UE Power Class 3 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 3 | 20 | 23 | 6 | 2 | 25+TT | 14-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 9 | 14 | 23 | 6 | 2 | 25+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 11 | 12 | 23 | 6 | 2 | 25+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 6 | 17 | 23 | 5 | 2 | 25+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 11 | 12 | 23 | 6 | 2 | 25+TT | 6-TT |
| 48 (Note 3, 4) | 15 | 7 | 16 | 23 | 5 | 2 | 25+TT | 11-TT |
| 48 (Note 3, 4) | 30, 60 | 12 | 11 | 23 | 6 | 2 | 25+TT | 5-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 7 | 16 | 23 | 5 | 2 | 25+TT | 11-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-2: UE Power Class 3 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission not overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | MPR (dB) | A-MPR (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (Note 1) | 15, 30, 60 | 0 | 3 | 20 | 23 | 4 | 2 | 25+TT | 16-TT |
| 4 (Note 2) | 15, 30, 60 | 0.5 | 0 | 22.5 | 23 | 2 | 2 | 25+TT | 20.5-TT |
| 6 (Note 2) | 15, 30, 60 | 0 | 3.5 | 19.5 | 23 | 3.5 | 2 | 25+TT | 16-TT |
| 11 (Note 2) | 15, 30, 60 | 1 | 0 | 22 | 23 | 2 | 2 | 25+TT | 20-TT |
| 13 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 (Note 2) | 15, 30, 60 | 2 | 0 | 21 | 23 | 2 | 2 | 25+TT | 19-TT |
| 20 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5-TT |
| 33 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12-TT |
| 38 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5-TT |
| 40 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12-TT |
| NOTE 1: Test configuration IDs without transmission overlap, 1RB E-UTRA allocation, A-MPR requirements in TS 36.101 [5] apply NOTE 2: Test configuration IDs without transmission overlap, 1RB NR allocation, A-MPR requirements in TS 38.101-1 [2] apply. NOTE 3: Void. NOTE 4: Void. NOTE 5: Void. NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-3: UE Power Class 3 test requirements for NS_04, not supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 6 | 17 | 23 | 5 | 2 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 12 | 11 | 23 | 6 | 2 | 25+TT | 5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 14 | 9 | 23 | 7 | 2 | 25+TT | 2-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 9 | 14 | 23 | 6 | 2 | 25+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 14 | 9 | 23 | 7 | 2 | 25+TT | 2-TT |
| 48 (Note 3, 4) | 15 | 10 | 13 | 23 | 6 | 2 | 25+TT | 7-TT |
| 48 (Note 3, 4) | 30, 60 | 15 | 8 | 23 | 7 | 2 | 25+TT | 1-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 10 | 13 | 23 | 6 | 2 | 25+TT | 7-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-4: UE Power Class 2 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 3 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 9 | 17 | 26 | 5 | 2 | 28+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 11 | 15 | 26 | 6 | 2 | 28+TT | 9-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 6 | 20 | 26 | 6 | 2 | 28+TT | 14-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 11 | 15 | 26 | 6 | 2 | 28+TT | 9-TT |
| 48 (Note 3, 4) | 15 | 7 | 19 | 26 | 5 | 2 | 28+TT | 14-TT |
| 48 (Note 3, 4) | 30, 60 | 12 | 14 | 26 | 6 | 2 | 28+TT | 8-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 7 | 19 | 26 | 5 | 2 | 28+TT | 14-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-5: UE Power Class 2 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16))

| Test ID | Test SCS (kHz) | MPR (dB) | A-MPR (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (Note 1) | 15, 30, 60 | 0 | 3 | 23 | 26 | 2 | 2 | 28+TT | 21-TT |
| 4 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 6 (Note 2) | 15, 30, 60 | 0 | 5.5 | 20.5 | 26 | 2.5 | 2 | 28+TT | 18-TT |
| 11 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 13 (Note 2) | 15, 30, 60 | 0 | 6 | 20 | 26 | 2.5 | 2 | 28+TT | 17.5-TT |
| 18 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 20 (Note 2) | 15, 30, 60 | 0 | 6 | 20 | 26 | 2.5 | 2 | 28+TT | 17.5-TT |
| 31 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 33 (Note 2) | 15, 30, 60 | 0 | 7.5 | 18.5 | 26 | 4 | 2 | 28+TT | 14.5-TT |
| 38 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 40 (Note 2) | 15, 30, 60 | 0 | 7.5 | 18.5 | 26 | 4 | 2 | 28+TT | 14.5-TT |
| NOTE 1: Test configuration IDs without transmission overlap, 1RB E-UTRA allocation, A-MPR requirements in TS 36.101 [5] apply. NOTE 2: Test configuration IDs without transmission overlap, 1RB NR allocation, A-MPR requirements in TS 38.101-1 [2] apply. NOTE 3: Void. NOTE 4: Void. NOTE 5: Void. NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-5a: UE Power Class 2 test requirements for NS_04, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16))

| Test ID | Test SCS (kHz) | MPR (dB) | A-MPR (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (Note 1) | 15, 30, 60 | 0 | 3 | 23 | 26 | 2 | 2 | 28+TT | 21+TT |
| 4 (Note 2) | 15, 30, 60 | 0.5 | 0 | 22.5 | 23 | 2 | 2 | 25+TT | 20.5+TT |
| 6 (Note 2) | 15, 30, 60 | 0 | 3.5 | 19.5 | 23 | 3.5 | 2 | 25+TT | 16+TT |
| 11 (Note 2) | 15, 30, 60 | 1 | 0 | 22 | 23 | 2 | 2 | 25+TT | 20+TT |
| 13 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5+TT |
| 18 (Note 2) | 15, 30, 60 | 2 | 0 | 21 | 23 | 2 | 2 | 25+TT | 19+TT |
| 20 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5+TT |
| 31 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5+TT |
| 33 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12.5+TT |
| 38 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5+TT |
| 40 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12.5+TT |
| NOTE 1: Test configuration IDs without transmission overlap, 1RB E-UTRA allocation, A-MPR requirements in TS 36.101 [5] apply. NOTE 2: Test configuration IDs without transmission overlap, 1RB NR allocation, A-MPR requirements in TS 38.101- [2] apply. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-6: UE Power Class 2 test requirements for NS_04, not supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 6 | 20 | 26 | 6 | 2 | 28+TT | 14-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 12 | 14 | 26 | 6 | 2 | 28+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 14 | 12 | 26 | 6 | 2 | 28+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 9 | 17 | 26 | 5 | 2 | 28+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 14 | 12 | 26 | 6 | 2 | 28+TT | 6-TT |
| 48 (Note 3, 4) | 15 | 10 | 16 | 26 | 5 | 2 | 28+TT | 11-TT |
| 48 (Note 3, 4) | 30, 60 | 15 | 11 | 26 | 6 | 2 | 28+TT | 5-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 10 | 16 | 26 | 5 | 2 | 28+TT | 11-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

#### 6.2B.3.2	UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC

Editor's note:	This test case is incomplete. The following aspects are either missing or not yet determined:

-	Test frequencies for the Minimum WGAP

-	Test requirements for non-overlapping transmission of non-DPS UE need further investigation

6.2B.3.2.1	Test purpose

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2B.1.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2B.3.2.2	Test applicability

The requirements of this test apply in test case 6.5B.2.2.2 Additional spectrum emission mask for network signalled values NS_04 to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.2B.3.2.3	Minimum conformance requirements

For intra-band non-contiguous EN-DC band combinations with additional requirements the A-MPR allowed are specified in table 6.2B.3.2.3-1 for UEs configured with EN-DC and combinations of network signalling values indicated in the E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR specified in subclause6.2B.3.2 for intra-band non-contiguous EN-DC configurations is the total power reduction allowed including MPR. For UEs scheduled with single uplink transmission, AMPR in subclause 6.2.4 of [4] and 6.2.3 of [2] apply.

Table 6.2B.3.2.3-1: Allowed power reduction for intra-band non-contiguous EN-DC

| DC configuration | Requirement (clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (clause) |
| --- | --- | --- | --- | --- |
| DC_41A_n41A1 | 6.6.3.3.19 and 6.6.2.2.2 of TS 36.101 [5] and 6.5.2.3.2 and 6.5.3.3.1 of TS 38.101-1 [2] | NS_01 or NS_04 | NS_04 | 6.2B.3.2.3.1 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The requirement applies when the combination of network signalling values in the two CGs is set (only for UEs configured with EN-DC). |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.3.2.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA overlap in time with NR.LTE anchor agnostic approach is not applied for this case and referred to as sub-test 1. E-UTRA test point analysis is included and E-UTRA measurements are performed.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that doesn't support dynamic power sharing. LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

No exception requirements for NR or E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that supports dynamic power sharing. LTE anchor agnostic approach is not applied for this case.

6.2B.3.2.3.1	A-MPR for NS_04

When the UE is configured for B41/n41 intra-band non-contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this clause. The A-MPR for EN-DC defined in this clause is used instead of MPR defined in 6.2B.2.2, not additively, so EN-DC MPR=0 when NS_04 is signalled.

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with TS 36.101 [5]

-	for the SCG,

A-MPR'c = A-MPRNR = MAX( A-MPRsingle,NR, A-MPREN-DC)

-	for the total configured transmission power,

A-MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - A-MPRE-UTRA)/10) + 10^((PPowerClass,NR - A-MPRNR)/10))

where

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA + MPRsingle,E-UTRA, A-MPREN-DC )

A-MPREN-DC = MAX(A-MPRIM3, A-MPRACLRoverlap )

with

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 38.101-3 [4]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 38.101-3 [4]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = MAX( A-MPRsingle, E-UTRA + MPRsingle,E-UTRA, A-MPRIM3, A-MPRACLRoverlap)

-	for the SCG,

A-MPR'c = MAX( A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap)

where

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 36.101 [5]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [5]

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If AND( FIM3,low_block,high < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

Channel Configuration Case C. A-MPRIM3 defined in Clause 6.2B.3.1.3.2.1

Else

Channel Configuration Case D. A-MPRIM3 defined in Clause 6.2B.3.1.3.2.2

where

-	FIM3,low_block,high = (2 * Flow_channel,high_edge ) – Fhigh_channel,low_edge

-	FIM3,high_block,low = (2 * Fhigh_channel,low_edge) – Flow_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Flow_channel,high_edge is the uppermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,low_edge is the lowermost frequency of upper transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.6.2.2.2 in [5] and Subclause 6.5.2.3.2 in [2] respectively.

Where the transmission bandwidth configuration for NR is the maximum frequency span covering all the configured SCSSpecificCarrier for scenarios that carrier bandwidths with different SCS can be fully overlapped.

The UE determines the value of A-MPRACLRoverlap as specified in Table 6.2B.3.2.3.1-1:

Table 6.2B.3.2.3.1-1: A-MPRACLRoverlap

| Wgap | A-MPRACLRoverlap |
| --- | --- |
| < BWchannel,E-UTRA + BWchannel,NR | 4 dB |
| ≥ BWchannel,E-UTRA + BWchannel,NR | 0 dB |
| NOTE 1: Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge |  |

6.2B.3.2.4	Test description

6.2B.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.5B.3-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in test configuration table 6.2B.3.1.4.1-1 through 6.2B.3.1.4.1-2 with additional IM3 test frequencies for NS_04 in 6.2B.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in Annex TS 36.521-1 [10] Annex C and in Annex C2 for LTE link and NR link respectively.

Table 6.2B.3.2.4.1-0: E-UTRA test configuration table

| E-UTRA Test Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency (Note 1) | Downlink | Uplink |  |
|  |  | N/A for A-MPR testing | Modulation | RB allocation |
| 20 MHz | Low range and High range (Note 2) |  | QPSK | 100 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [6] clause 4.3.1 NOTE 2: NR carrier shall be the outermost carrier during test. |  |  |  |  |

Table 6.2B.3.2.4.1-1: Additional IM3 Test Frquencies for NS_04 intra-band non-contiguous EN-DC

| Additional Initial Condition |  |
| --- | --- |
| Additional IM3 Test Frquencies | if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 15 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-2. if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 30 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-3. if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 60 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-4. |

Table 6.2B.3.2.4.1-2: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 15 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2528.86 | 505772 | 0 | 15 | 6327 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2657.26 | 531452 | 0 | 15 | 6648 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2665.000 | 533000 | 2640.7 | 528140 | 0 | 15 | 6606 | 533000 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519 | 503800 | 0 | 15 | 6303 | 508660 |

Table 6.2B.3.2.4.1-3: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 30 kHz, 30 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 106 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2529.22 | 505844 | 0 | 30 | 6333 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 106 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2657.62 | 531524 | 0 | 30 | 6654 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 133 | Downlink & Uplink | IM3 | 2665.000 | 533000 | 2641.06 | 528212 | 0 | 30 | 6612 | 533000 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 133 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519.36 | 503872 | 0 | 30 | 6309 | 508660 |

Table 6.2B.3.2.4.1-4: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 60 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 51 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2529.94 | 505988 | 0 | 15 | 6330 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 51 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2658.34 | 531668 | 0 | 15 | 6651 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 65 | Downlink & Uplink | IM3 | 2665.000 | 535340 | 2653.3 | 530660 | 0 | 15 | 6639 | 535340 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 65 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519.9 | 503980 | 0 | 15 | 6306 | 508660 |

Editor's note:	The following lines belong at the end of clause 6.2B.3.2.4.1. As new tables are added to this clause, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for LTE link and NR link respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.2.4.3.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.3.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 6.2B.3.2.4.1-1 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers for the EN-DC configuration, which shall meet the requirements described in table 6.2B.3.2.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms).

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2B.3.1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1, with the following exceptions.

Table 6.2B.3.2.4.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Apply if run test points with E-UTRA UL transmission not overlapping with NR UL transmission in time for PC3 UE |
|  | 20 |  | Apply if run test points with E-UTRA UL transmission overlapping with NR UL transmission in time for PC3 UE, and UE doesn’t support dynamic power sharing. |
| P-Max | 23 |  | Apply if run test points with E-UTRA UL transmission not overlapping with NR UL transmission in time for PC3 UE |
|  | 20 |  | Apply if run test points with E-UTRA UL transmission overlapping with NR UL transmission in time for PC3 UE, and UE doesn’t support dynamic power sharing. |

6.2B.3.2.4.3-2: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.3.2.4.3-3: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE does not support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  | Apply if run test points with E-UTRA UL transmission not overlapping with NR UL transmission in time for PC3 UE |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. |  |  |  |

6.2B.3.2.4.3.1	Message contents exceptions (network signalled value "NS_04")

For "NS_04" see A-MPR test case in table 6.2B.3.1.4.3.1-1 and table 6.2B.3.1.4.3.1-2.

6.2B.3.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.3.2.5-0 to Table 6.2B.3.2.5-2a. The allowed A-MPR values specified in Table 6.2B.3.2.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.1.1.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2B.1.1.3-1 apply.

Table 6.2B.3.2.5-0: Test Tolerance for UE maximum output power (LTE, NR TX separately)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| LTE | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.3.2.5-1: UE Power Class 3 test requirements for network signalled value "NS_04"

| Test ID | Modulation | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | A-MPREN-DC (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 1 | 0 | 2 | 0 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 2 | 0 | 2 | 0 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 25+TT | 6-TT (4.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 1 | 0 | - | Note 5 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 2 | 0 | - | Note 5 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 25+TT | 6-TT (4.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 3 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 4 | 0 | 1 | Note 6 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 25+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 3 | 0 | - | Note 5 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 4 | 0 | - | Note 5 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 25+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 2 | 0 | - | Note 5 | 11 | 11 | 0 (1.5) | 9 (7.5) | 6 (6) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 | E-UTRA Note 1 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37 and 41 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 4 | NR | 0 | 0.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 25+TT | 20.5-TT (19-TT) |
| 6 | NR | 0 | 0 | Note 9 | 3.5 | 3.5 | 0 (1.5) | 19.5 (18) | 3.5 (4) | 2 (3.5) | 25+TT | 16-TT (14-TT) |
| 11 | NR | 0 | 1 | Note 9 | 0 | 0 | 0 (1.5) | 22 (20.5) | 2 (2.5) | 2 (3.5) | 25+TT | 20-TT (18-TT) |
| 13 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 18 | NR | 0 | 2 | Note 9 | 0 | 0 | 0 (1.5) | 21 (19.5) | 2 (3.5) | 2 (3.5) | 25+TT | 19-TT (16-TT) |
| 20 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 31 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 33 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| 38 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 40 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| NOTE 1: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 2: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 3: When NR SCS = 15kHz. NOTE 4: When NR SCS = 30 kHz or 60 kHz. NOTE 5: NR A-MPR values for NS_04 are defined in Table 6.2.3.3.2-1. NOTE 6: E-UTRA A-MPR= 3 dB for 1 RB and fc < 2517.5 MHz, otherwise 0 dB. NOTE 7: ΔTC,c = 1.5 dB for transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, otherwise 0 dB. NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.2.5-0. NOTE 9: Apply for UE supporting dynamic power sharing. NR A-MPR values for NS_04 are defined in TS 38.101-1 [2]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.2.5-2: UE Power Class 2 test requirements for network signalled value "NS_04" (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16))

| Test ID | Modulation | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | A-MPREN-DC (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 1 | 3 | 2 | 0 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 2 | 3 | 2 | 0 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 1 | 3 | - | Note 5 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 3 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 4 | 3 | 1 | Note 6 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 2 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 3 | 3 | - | Note 5 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 4 | 3 | - | Note 5 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 9 (7.5) | 6 (6) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 | E-UTRA Note 1 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37 and 41 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 4 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 6 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 20.5 (19) | 2.5 (3.5) | 2 (3.5) | 28+TT | 18-TT (15.5-TT) |
| 11 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 13 | NR | 0 | 0 | Note 9 | 6 | 6 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 28+TT | 17.5-TT (14.5-TT) |
| 18 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 20 | NR | 0 | 0 | Note 9 | 6 | 6 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 28+TT | 17.5-TT (14.5-TT) |
| 31 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 33 | NR | 0 | 0 | Note 9 | 7.5 | 7.5 | 0 (1.5) | 18.5 (17) | 4 (5) | 2 (3.5) | 28+TT | 14.5-TT (12-TT) |
| 38 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 40 | NR | 0 | 0 | Note 9 | 7.5 | 7.5 | 0 (1.5) | 18.5 (17) | 4 (5) | 2 (3.5) | 28+TT | 14.5-TT (12-TT) |
| NOTE 1: When FIM3,low_block,low ≥ 2490.5 MHz (Case A). NOTE 2: When FIM3,low_block,low < 2490.5 MHz (Case B). NOTE 3: When NR SCS = 15kHz. NOTE 4: When NR SCS = 30 kHz or 60 kHz. NOTE 5: NR A-MPR values for NS_04 are defined in Table 6.2.3.3.2-1. NOTE 6: E-UTRA A-MPR= 3 dB for 1 RB and fc < 2517.5 MHz, otherwise 0 dB. NOTE 7: ΔTC,c = 1.5 dB for transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, otherwise 0 dB. NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.2.5-0. NOTE 9: Apply for UE supporting dynamic power sharing. NR A-MPR values for NS_04 are defined in TS 38.101-1 [24]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.2.5-2a: UE Power Class 2 test requirements for network signalled value "NS_04" (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16))

| Test ID | Modulation | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | A-MPREN-DC (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 1 | 3 | 2 | 0 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 2 | 3 | 2 | 0 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 1 | 3 | - | Note 5 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 3 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 4 | 3 | 1 | Note 6 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 2 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 3 | 3 | - | Note 5 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 4 | 3 | - | Note 5 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 9 (7.5) | 6 (6) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 | E-UTRA Note 1 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37 and 41 | E-UTRA Note 2 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 4 | NR | 0 | 0.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 25+TT | 20.5-TT (19-TT) |
| 6 | NR | 0 | 0 | Note 9 | 3.5 | 3.5 | 0 (1.5) | 19.5 (18) | 3.5 (4) | 2 (3.5) | 25+TT | 16-TT (14-TT) |
| 11 | NR | 0 | 1 | Note 9 | 0 | 0 | 0 (1.5) | 22 (20.5) | 2 (2.5) | 2 (3.5) | 25+TT | 20-TT (18-TT) |
| 13 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 18 | NR | 0 | 2 | Note 9 | 0 | 0 | 0 (1.5) | 21 (19.5) | 2 (3.5) | 2 (3.5) | 25+TT | 19-TT (16-TT) |
| 20 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 31 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 33 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| 38 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 40 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| NOTE 1: When FIM3,low_block,low ≥ 2490.5 MHz (Case A). NOTE 2: When FIM3,low_block,low < 2490.5 MHz (Case B). NOTE 3: When NR SCS = 15kHz. NOTE 4: When NR SCS = 30 kHz or 60 kHz. NOTE 5: NR A-MPR values for NS_04 are defined in Table 6.2.3.3.2-1. NOTE 6: E-UTRA A-MPR= 3 dB for 1 RB and fc < 2517.5 MHz, otherwise 0 dB. NOTE 7: ΔTC,c = 1.5 dB for transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, otherwise 0 dB. NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.2.5-0. NOTE 9: Apply for UE supporting dynamic power sharing. NR A-MPR values for NS_04 are defined in TS 38.101-1 [2]. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.2B.3.3	UE Additional Maximum Output Power reduction for Inter-Band EN-DC within FR1 (1 NR CC)

6.2B.3.3.1	Test purpose

Same test purpose as in clause 6.2.3.1 in TS 38.521-1 [8] for the NR carrier.

6.2B.3.3.2	Test applicability

The requirements of this test apply to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

NOTE:	Test execution is not necessary if clause 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 are executed since A-MPR requirements are verified in these test cases.

6.2B.3.3.3	Minimum conformance requirements

For inter-band EN-DC between E-UTRA and NR FR1, UE additional maximum output power reduction specified in TS 36.101 [5] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.3.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied

6.2B.3.3.4	Test description

Same test description as in clause 6.2.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.2.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 6.2B.3.3.4-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 6.2B.3.3.4-1.

Step 6 of Initial conditions as in clause 6.2.3.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.Same test procedure as in clause 6.2.3.4.2 in TS 38.521-1 [8].

6.2B.3.3.5	Test requirement

For PC3 UE, same test requirement as in clause 6.2.2.5 in TS 38.521-1 [8].

For PC2 UE (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)), same test requirement for PC2 UE as in clause 6.2.3.5 in TS 38.521-1 [8]. For PC2 UE (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16)), same test requirement for PC3 UE as in clause 6.2.3.5 in TS 38.521-1 [8].

#### 6.2B.3.4	UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC)

6.2B.3.4.1	Test purpose

Same test purpose as in clause 6.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.3.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.2B.3.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.2B.3.4.4	Test description

6.2B.3.4.4.1	Initial conditions

Same test description as in clause 6.2.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.2.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

6.2B.3.4.4.2	Test procedure

Same test procedure as in clause 6.2.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.3.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 with the following exceptions for each network signalled value.

1.	Information element AdditionalSpectrumEmission for NR can be set in nr-SecondaryCellGroupConfig according to TS 38.331 [15]. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.4.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_202) | for band n257 |  |
| AdditionalSpectrumEmission | 2 (NS_202) | for band n258 |  |
| AdditionalSpectrumEmission | 3 (NS_203) | for band n258 |  |

6.2B.3.4.5	Test requirement

Same test requirement as in clause 6.2.3.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.2B.3.4D	UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.2D.3 in TS 38.521-2 [9] is incomplete

6.2B.3.4D.1	Test purpose

Same test purpose as in clause 6.2D.3.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.3.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC with UL MIMO.

6.2B.3.4D.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2D.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

6.2B.3.4D.4	Test description

6.2B.3.4D.4.1	Initial conditions

Same test description as in clause 6.2D.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.2D.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2D.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

6.2B.3.4D.4.2	Test procedure

Same test procedure as in clause 6.2D.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.3.4D.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO, with the following exceptions for each network signalled value.

1.	Information element AdditionalSpectrumEmission for NR can be set in nr-SecondaryCellGroupConfig according to TS 38.331 [15]. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4D.4.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_202) | for band n257 |  |
| AdditionalSpectrumEmission | 2 (NS_202) | for band n258 |  |
| AdditionalSpectrumEmission | 3 (NS_203) | for band n258 |  |

6.2B.3.4D.5	Test requirement

Same test requirement as in clause 6.2D.3.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.2B.3.5	UE Additional Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2

6.2B.3.5.1	Test purpose

Same test purpose as in clause 6.2.3.1 in TS 38.521-1 [8] for NR FR1 carrier and clause 6.2.3.1 in TS 38.521-2 [9] for NR FR2 carrier.

6.2B.3.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for Additional Maximum Output Power apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 6.2B.3.

#### 6.2B.3.5D	UE Additional Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2 for UL MIMO

6.2B.3.5D.1	Test purpose

Same test purpose as in clause 6.2D.3.1 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.2D.3.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.2B.3.5D.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for maximum output power apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 6.2B.

### 6.2B.4	Configured Output Power for EN-DC

#### 6.2B.4.1	Configured Output Power Level for EN-DC

##### 6.2B.4.1.0	Minimum Conformance Requirements

6.2B.4.1.0.1	Configured output power level

6.2B.4.1.0.1.1	Intra-band contiguous EN-DC

The following requirements apply for one component carrier per CG configured for synchronous DC.

For intra-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for EN-DC operation PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [19].

The configured maximum output power PCMAX_E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_E-UTRA,c (p) ≤ PCMAX_E-UTRA,c (p) ≤  PCMAX H_E-UTRA,c (p)

where PCMAX_L_E-UTRA,c and PCMAX H_E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [5] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_E-UTRA,c = MIN {MIN(PEMAX,c , PEMAX, EN-DC, PLTE) – tC_E-UTRA, c, (PPowerClass,EN-DC – ΔPPowerClass,EN-DC), (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_E-UTRA, c + TProSe, P-MPRc)}

PCMAX H_E-UTRA,c = MIN {PEMAX,c, PEMAX, EN-DC , PLTE, PPowerClass, EN-DC, PPowerClass – ΔPPowerClass}

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [17];

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [17] which is the same as PLTE in TS 38.213 [19];

-	∆tC_EUTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 of TS 36.101 [5] applies; ∆tC_EUTRA, c = 0 dB otherwise;

and whenever NS_01 is not indicated within CG 1:

-	for a UE indicating support of dynamicPowerSharing, the MPRc and the A-MPRc are determined in accordance with the DCI of serving cell c of the CG 1 and the specification in clause 6.2.4 of TS 36.101 [5];

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with clause 6.2B.3.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

and whenever NS_01 is indicated in CG 1:

-	for a UE indicating support of dynamicPowerSharing, the MPRc is determined in accordance with the DCI of serving cell c of the CG 1 and the specification in clause 6.2.4 of TS 36.101 [5];

-	for a UE not indicating support of dynamicPowerSharing, the MPRc is determined in accordance with clause 6.2B.2.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

The configured maximum output power PCMAX_NR,c (q) in physical channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_NR,c and PCMAX H_NR,c are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified  as follows:

PCMAX_L,f,c,,NR = MIN {MIN(PEMAX,c , PEMAX, EN-DC, PNR) - TC_NR, c, (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC, PNR , PPowerClass, EN-DC, PPowerClass – ΔPPowerClass }

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [17];

-	PLTE signalled by RRC as p-MaxEUTRA-r15 in TS 36.331 [17]

-	PNR is the value given by the field  p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in  [18] and signalled by RRC;

-	ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c specified in clause 6.2B.4.2.1 for EN-DC, the individual Power Class defined in table 6.2B.1.1 and any other additional power reductions parameters specified in clauses 6.2B.2 and 6.2B.3 for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

PPowerClass, EN-DC is defined in clause 6.2B.1.1 for intra-band contiguous EN-DC;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2]; in case IE [powerClassNRPart] as defined in TS 38.331 [18] is indicated, PPowerClass,NR should use that value instead.

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [5];-

ΔPPowerClass,EN-DC is 3 dB for a power class 2 capable EN-DC UE when  LTE UL/DL configuration is 0 or 6; or LTE UL/DL configuration is 1 and special subframe configuration is 0 or 5; ΔPPowerClass,EN-DC = 3 dB when the IE p-maxUE-FR1 as defined in TS 36.331 [17] is provided and set to the maximum output power of the default power class or lower; ΔPPowerClass,EN-DC is 6 dB for a power class 1.5 capable EN-DC UE when the LTE UL duty cycle is greater than max(50%, maxUplinkDutyCycle); ΔPPowerClass,EN-DC is 3 dB for a power class 1.5 capable EN-DC UE when the LTE UL duty cycle is between max(50%, maxUplinkDutyCycle) and max(25%, maxUplinkDutyCycle/2); otherwise ΔPPowerClass,EN-DC = 0 dB;

and whenever NS_01 is not indicated within CG 2:

-	for a UE indicating support of dynamicPowerSharing, A-MPRc = A-MPR'c with A-MPR'c determined in accordance with clause 6.2B.3.1 and MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, A-MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with clause 6.2B.3.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

and whenever NS_01 is indicated in CG 2.

-	for a UE indicating support of dynamicPowerSharing, MPRc = MPR'c with MPR'c determined in accordance with clause 6.2B.2.1 and A-MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the MPRc is determined in accordance with clause 6.2B.2.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

If the EN-DC UE is not supporting dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [5] and TS 38.101-1 [2] respectively apply with the modifications specified above.

If the UE does not support dynamic power sharing,

PTotalEN-DC = MIN { PEMAX, EN-DC , PPowerClass, EN-DC - ΔPPowerClass,EN-DC } + 0.3 dB

For UEs indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE the UE can configure the total maximum transmission power PTotalEN-DC within the range

PEN-DC,tot_L ≤ PTotalEN-DC ≤  PEN-DC,tot_H

where

PEN-DC,tot_L (p,q) = MIN{ PPowerClass,EN-DC - ΔPPowerClass,EN-DC – MAX{MPRtot, A-MPRtot}, PEMAX,EN-DC}

PEN-DC,tot_H (p,q) = MIN{PPowerClass,EN-DC, PEMAX,EN-DC }

for sub-frame p on CG 1 overlapping with physical channel q on CG 2 and with MPRtot and A-MPRtot in accordance with 6.2B.2.1 and clause 6.2B.3.1, respectively.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,f,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

For UEs indicating support of dynamicPowerSharing, the measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_L) and THIGH(PCMAX_H) for applicable values of PCMAX_L and PCMAX_L specified in Table 6.2B.4.1.0.1.1-2.

When an UL subframe transmission p from E-UTRA overlap with a physical channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.0.1.1-1 when same or different subframes and physical channel durations are used in aggregated carriers. PPowerClass ,EN-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.0.1.1-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | E-UTRA Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_EN-DC_H (p,q) , PCMAX_EN-DC_H (p,q+1), … , PCMAX_EN-DC_H (p,q+n) }

where PCMAX_EN-DC_H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical channel overlapping with LTE subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_EN-DC_L (p,q) , PCMAX_EN-DC_L (p,q+1), … , PCMAX_EN-DC_L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical channel overlapping with E-UTRA subframe p,

With

PCMAX_EN-DC_H(p,q) = MIN {10 log10 [pCMAX H_E-UTRA,c (p) + pCMAX H,f,c,NR c(q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

a= 10 log10 [pCMAX_E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PEN-DC,tot_L

b= 10 log10 [pCMAX_E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PEN-DC,tot_L

If a= FALSE and the configured transmission power spectral density between the MCG and SCG differs by less than 6 dB

PCMAX_EN-DC_L(p,q) = MIN {10 log10 [pCMAX L_E-UTRA,c (p) + pCMAX L,f,c,,NR c(q)], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

ELSE If (a=TRUE) AND (b=FALSE) and the configured transmission power spectral density between the MCG and SCG differs by less than 6 dB

PCMAX_EN-DC_L(p,q) = MIN {10 log10 [pCMAX L_E-UTRA,c (p) + pCMAX L,f,c,,NR c(q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

ELSE If b= TRUE or the transmission power after power scaling spectral density between the MCG and SCG differs by more than 6 dB

PCMAX_EN-DC_L(p,q) = MIN {10 log10 [pCMAX L_E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

where

-	pCMAX H_E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H_NR,c (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L_E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L_NR,c(q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.1.3-1 for intra-band EN-DC;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6] dB

-	pCMAX  E-UTRA,c (p) is the linear value of PCMAX  E-UTRA,c (p), the real configured max power for E-UTRA

-	pCMAX,f,c  NR (q) is the linear value of PCMAX,f,c  NR (q), the real configured max power of NR

Table 6.2B.4.1.0.1.1-2: PCMAX tolerance for Dual Connectivity E-UTRANR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

If the UE supports dynamic power sharing, and when LTE and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions and unless otherwise stated.

10log(pCMAX L,f,c,,NR(q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,,NR(q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,,NR (q)) + THIGH (10log(pCMAX H, f,c,,NR (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4.1.0.1.1-2.

If the UE supports dynamic power sharing, the measured maximum output power in subframe p on CG 1, pUMAX,c,E-UTRA,  shall meet the requirements in clause 6.2.5 in TS 36.101 [5] with the limits PCMAX_L,c and PCMAX_H,c replaced by PCMAX_L_E-UTRA,c and PCMAX_H_E- UTRA,c as specified above, respectively.

If the configured transmission power spectral density between the MCG and SCG differs by more than 6 dB, then

PUMAX,f,c,NR (q) ≤ 10log(pCMAX H, f,c,,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

6.2B.4.1.0.1.2	Intra-band non-contiguous EN-DC

The following requirements apply for one component carrier per CG configured for synchronous DC. The CG(s) are indexed by j = 1 for MCG and j = 2 for SCG.

The configured maximum output power PCMAX_E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set in accordance with clause 6.2B.4.1.0.1.1 but where

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc determined in accordance with clause 6.2B.3.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

whenever NS_01 is not indicated within CG 1 while

-	for a UE not indicating support of dynamicPowerSharing, the MPRc determined in accordance with clause 6.2B.2.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

whenever NS_01 is indicated in CG 1.

The configured maximum output power PCMAX_NR,c (q) in physical channel q for the configured NR carrier shall be set in accordance with clause 6.2B.4.1.0.1.1 but where

-	for a UE indicating support of dynamicPowerSharing, A-MPRc = A-MPR'c with A-MPR'c determined in accordance with clause 6.2B.3.2 and MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, A-MPRc is determined in accordance with [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with clause 6.2B.3.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

For UEs indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE, the UE can configure the total transmission power in accordance with clause 6.2B.4.1.0.1.1 but with Ppowerclass,EN-DC the EN-DC power class of the intra-band non-contiguous band combination configured and A-MPR determined in accordance with clause 6.2B.3.2.

whenever NS_01 is not indicated in CG 2 while

-	for a UE indicating support of dynamicPowerSharing, MPRc = MPR'c with MPR'c determined in accordance with clause 6.2B.2.2 and A-MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the MPRc is determined in accordance with clause 6.2B.2.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

whenever NS_01 is indicated in CG 2.

For UEs indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE, the UE can configure the total transmission power in accordance with clause 6.2B.4.1.1 but with Ppowerclass,EN-DC the EN-DC power class of the intra-band non-contiguous band combination configured and A-MPR determined in accordance with clause 6.2B.3.2.

The total maximum output power PUMAX over both CGs is measured in accordance with clause 6.2B.4.1.0.1.1 and shall be within the limits specified in clause 6.2B.4.1.0.1.1 but with parameters applicable for the non-contiguous band combination configured.

The maximum output power levels pUMAX,c,E-UTRA and pUMAX,f,c,NR for the CGs are measured in accordance with clause 6.2B.4.1.0.1.1 and shall be within the limits specified in clause 6.2B.4.1.0.1.1 but with parameters applicable for the non-contiguous band combination configured.

6.2B.4.1.0.1.3	Inter-band EN-DC within FR1

For inter-band dual connectivity with one uplink serving cell or more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG and one uplink serving cell on the NR CG or more than one uplink serving cells configured for intra-band UL CA, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission  power for EN-DC operation, PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [19]. For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, the PCMAX applies to the entire E-UTRA CG. For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the NR CG, the PCMAX applies to the entire NR CG.

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell and E-UTRA Pcell is FDD, the UE is not expected to be configured with more than one serving cells in the uplink.

The configured maximum output power PCMAX_E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_E-UTRA,c (p) ≤  PCMAX_E-UTRA,c (p) ≤  PCMAX H_E-UTRA,c (p)

where PCMAX_L_E-UTRA,c and PCMAX H_E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [5] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_E-UTRA,c = MIN { PEMAX, EN-DC , (PPowerClass, EN-DC – ΔPPowerClass, EN-DC ), MIN(PEMAX,c , PLTE) – tC_E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + tC _E-UTRA, c + TProSe, P-MPRc)}

PCMAX H_E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass, EN-DC ), PLTE, PPowerClass, E-UTRA – ΔPPowerClass, E-UTRA}

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for the E-UTRA CG as specified in TS 36.101 [5] clause 6.2.5A modified by PLTE as follows:

PCMAX_L_ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c  - TC , (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ), PLTE, PPowerClass,EN-DC }

PCMAX H _ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c , PPowerClass,E-UTRA, PLTE, PPowerClass,EN-DC}

The configured maximum output power PCMAX_NR,c (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_NR,c and PCMAX H_NR,c are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified as follows:

PCMAX_L,f,c,,NR = MIN { PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass, EN-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass, NR – ΔPPowerClass, NR) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass ), PNR , PPowerClass, NR – ΔPPowerClass, NR }

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the NR CG, PCMAX_L,f,c, NR and PCMAX_H,f,c, NR are the limits for the NR CG as specified in [2] subclause 6.2A.4 modified by PNR as follows:

PCMAX_L,f,c,NR  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA, PPowerClass,NR – MAX(MPR + A-MPR + ΔTIB,c + T_NR ,C + TRxSRS, P-MPR ), PNR, PPowerClass,EN-DC }

PCMAX_H,f,c,NR  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,NR, PNR, PPowerClass,EN-DC}

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [17];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PPowerClass refers to the maximum output power of the E-UTRA intra-band CA power class given in Table 6.2.2A-1 of TS 36.101 [5];

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [17];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in clause 6.2.3A and clause 6.2.4A of TS 36.101 [5] respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX_ E-UTRA,c is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers within the E-UTRA CG.

-	If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG, MPRc and A-MPRc are determined by subclause 6.2.2 of [2]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined TS 38.331 [18];

-	Δtc_E-UTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] applies for a serving cell c, otherwise TC_E-UTRA,c = 0 dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0 dB;  TC_NR,C is the highest value TC_NR,C among all serving cells c if more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3 for inter-band EN-DC; if the UE indicates higherPowerLimit-r17 and ΔPPowerClass,EN-DC = 0, PPowerClass,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB;

-	∆PPowerClass,EN-DC = 3 dB for a power class 2 capable EN-DC UE when requirements of default power class had been applied as specified in sub-clause 6.2B.1; otherwise ∆PPowerClass,EN-DC = 0 dB;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2]; in case IE [powerClassNRPart] as defined in TS 38.331 [18] is indicated, PPowerClass,NR should use that value instead.

-	ΔPPowerClass,NR is 3 dB or 0 dB according to clause 6.2.4 of TS 38.101-1 [2] for a UE that supports power class 2 in the NR band of the EN-DC combination as defined in clause 6.2.1 of TS 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [5];

- 	ΔPPowerClass,E-UTRA is 3 dB or 0 dB according to clause 6.2.5 of TS 36.101 [5] for a UE that supports power class 2 in the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of TS 36.101 [5];

-	ΔTIB,c specified in clause 6.2B.4.2.3 for EN-DC, the individual Power Class defined in table 6.2B.1.3 and any other additional power reductions parameters specified in clauses 6.2B.2 and 6.2B.3for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	∆TRxSRS is the highest value among all serving cells c.

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

PTotalEN-DC = 10log10(PtotalEN-DC) with PTotalEN-DC the configured maximum transmission power for EN-DC operation as specified in clause 7.6 of TS 38.213 [19].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC – ΔPPowerClass, EN-DC }

If the UE does not support dynamic power sharing,

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC  – ΔPPowerClass, EN-DC } + 0.3 dB

If the EN-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [5] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalEN-DC applies.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_E-UTRA,c and PCMAX_NR,c for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for EN-DC operation, PTotalEN-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.0.1.3-2When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.0.1.3-1 when same or different subframe and physical-channel durations are used in aggregated carriers. The lesser of PPowerClass ,EN-DC and PEMAX,EN-DC shall not be exceeded by the UE during any evaluation period of time where PPowerClass ,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB if the UE indicates higherPowerLimit-r17.

Table 6.2B.4.1.0.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | E-UTRA Subframe on all aggregated cells of E-UTRA | Min(Tno_hopping, Physical Channel Length) on all aggregated cells of NR |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_EN-DC_H (p,q) , PCMAX_EN-DC_H (p,q+1), … , PCMAX_EN-DC_H (p,q+n) }

where PCMAX_EN-DC_H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_EN-DC_L (p,q) , PCMAX_EN-DC_L (p,q+1), … , PCMAX_EN-DC_L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p,

With

PCMAX_EN-DC_H(p,q) = MIN {10 log10 [pCMAX H_E-UTRA,c (p) + pCMAX H,f,c,NR c(q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

a= 10 log10 [pCMAX_E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalEN-DC

b= 10 log10 [pCMAX_E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PTotalEN-DC

If a= FALSE

PCMAX_EN-DC_L(p,q) = MIN {10 log10 [pCMAX L_E-UTRA,c (p) + pCMAX L,f,c,,NR c(q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If (a=TRUE) AND (b=FALSE)

PCMAX_EN-DC_L(p,q) = MIN {10 log10 [pCMAX L_E-UTRA,c (p) + pCMAX L,f,c,,NR c(q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If b= TRUE

PCMAX_EN-DC_L(p,q) = MIN {10 log10 [pCMAX L_E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC}

where

-	pCMAX H_E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H_NR,c (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L_E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L_NR,c(q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3-1 for inter-band EN-DC; if the UE indicates higherPowerLimit-r17, PPowerClass,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6]

-	pCMAX_E-UTRA,c (p) is the linear value of PCMAX_E-UTRA,c (p), the configured max power for E-UTRA. If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PCMAX_ E-UTRA,c (p) will be replaced by PCMAX(p) which is the configured maximum power for the entire E-UTRA CG.

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the configured max power of NR, If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG,  PCMAX_ NR,c (q) will be replaced by PCMAX(q) which is the configured maximum power for the entire NR CG.

Table 6.2B.4.1.0.1.3-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When E-UTRA and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions.

10log(pCMAX L,f,c,,NR c(q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,,NR c(q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,,NR c (q)) + THIGH (10log(pCMAX H, f,c,,NR c (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4.1.0.1.3-2.

6.2B.4.1.0.1.4	Inter-band EN-DC including FR2

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, with NR configured in FR2, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2.

The UE maximum configured power PCMAX,c(i), on E-UTRA for the subframe i shall be set according to clause 6.2.5 from TS 36.101 [5]. Applicable inter-band ΔTIB,c parameters shall be used according to the clauses 6.2B.4.2.4 or 6.2B.4.2.5.

The UE maximum configured power PCMAX,c(j), on NR for the slot j shall be set according to clause 6.2.4 from TS 38.101-2 [3].

For the configured power measurements TS 36.101 [5] clause 6.2.5 and TS 38.101-2 [3] clause 6.2.4 are applicable.

6.2B.4.1.0.1.5	Inter-band EN-DC including both FR1 and FR2

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, with both CGs configured in FR1, the requirements specified in clause 6.2B.4.1.0.1.3 apply.

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, with NR configured in FR2, the requirements specified in clause 6.2B.4.1.0.1.4 apply.

For inter-band dual connectivity with one uplink serving cell in first CG on E-UTRA and two uplink serving cells in second CG on NR FR1 and NR FR2 respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) , i = 1,2,3 with i=1 for E-UTRA, i=2 for NR FR1 and i=3 for NR FR2.

–	For serving cell on FR2, the requirements specified in clause 6.2.4 in TS 38.101-2 [3] apply to the UE maximum configured power PCMAX,c(3),3 and the measured maximum configured power.

–	For remaining inter-band dual connectivity involving CG1 and CG2, the requirements specified in clause 6.2B.4.1.0.1.3 apply.

##### 6.2B.4.1.1	Configured Output Power Level for Intra-Band Contiguous EN-DC

6.2B.4.1.1.1	Test purpose

To verify the UE does not exceed the power bounds defined by PCMAX_L and PCMAX_H

6.2B.4.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.2B.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.4.1.0.1.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.4.1.1.4	Test description

6.2B.4.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.2B.4.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.4.1.1.4.1-1: Test configurations table for intra-band contiguous EN-DC

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| NR and E-UTRA Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 |  |  | Mid range |  |  |  |  |
| Test EN-DC bandwidth combination as specified in TS 38.508-1 [5] clause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Highest |  |  |  |  |
| NR/E-UTRA Test Parameters for UE supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1f | N/A | QPSK | Full_Allocation | {-13, 7, 12, 15, 19, 23} for PC3 UE {-13, 7, 12, 15, 21, 26} for PC2 UE | CP-OFDM QPSK | Outer_Full | {-13, 7, 12, 15, 18, 23} for PC3 UE {-13, 7, 12, 15, 21, 26} for PC2 UE |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner Full | {-10, 10, 15} |
| NR/E-UTRA Test Parameters for UE not supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1d | N/A | QPSK | Full_Allocation | {-10, 10, 15, 23} for PC3 UE {-10, 10, 15, 26} for PC2 UE | CP-OFDM QPSK | Outer_Full | {-10, 10, 15, 23} for PC3 UE {-10, 10, 15, 26} for PC2 UE |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner Full | {-10, 10, 15} |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 3: The suffix in Test ID identifies the configured power level being tested from the PLTE and PNR lists |  |  |  |  |  |  |  |

Table 6.2B.4.1.1.4.1-2: Void

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.4.1.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.4.1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according table 6.2B.4.1.1.4.1-1 on E-UTRA CC and NR CC respectively. For test points configured with message in Table 6.2B.4.1.1.4.3-1, NR SS only schedules UL RMC on NR slots that does not overlap with E-UTRA uplink subframe. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

3.	Measure the mean transmitted power over E-UTRA component carrier and NR component carrier respectively, or/and measure the sum of mean transmitted power over E-UTRA and NR component carriers according to Table 6.2B.4.1.1.5-1 and Table 6.2B.4.1.1.5-2. The period of the measurement shall be at least the continuous duration of one active sub-frame. For TDD, only slots consisting of only UL symbols are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2B.4.1.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.4.1.1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.2B.4.1.1.4.3-1: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE doesn’t support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  | Test ID 1d |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2B.4.1.1.4.3-2: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.4.1.1.4.3-3: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | Defined as PLTE in Table 6.2B.4.1.1.4.1-1 |  |  |

Table 6.2B.4.1.1.4.3-4: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| p-NR-FR1 | Defined as PNR in Table 6.2B.4.1.1.4.1-1 |  |  |
| } |  |  |  |

Table 6.2B.4.1.1.4.3-5: RRCConnectionReconfiguration: p-MaxUE-FR1-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE |
|  | 26 |  | Power Class 2 UE |
| } |  |  |  |

6.2B.4.1.1.5	Test requirement

For UE supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.1.5-1.

Table 6.2B.4.1.1.5-1: PCMAX configured UE output power for UE supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | Not measured | Not measured | -10dBm ± (7+TT) |
| Test ID 1b | Not measured | Not measured | 10dBm ± (7+TT) |
| Test ID 1c | Not measured | Not measured | 15 ± (6+TT) |
| Test ID 1d | Not measured | Not measured | 18 ± (5+TT) |
| Test ID 1e | Not measured | 18 + (5+TT) / 11 – (6+TT) for PC3 UE | 21.54dBm + (3+TT) / 19.64 - (5+TT) for PC3 UE |
|  |  | 21 + (3+TT) / 14 - (6+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE repoting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) | 24 + (2+TT) / 21.8 - (5+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) |
|  |  | 21 + (3+TT) / 11 - (6+TT) for PC2 UE (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16)) | 24 + (2+TT) / 21.4 - (5+TT) for PC2 UE (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16)) |
| Test ID 1f | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE | N/A | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE |
|  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE |  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1of TS 36.521-1 [10] for PC2 UE |
| Test ID 2a | -10dBm ± (7+TT) | N/A- | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A- | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A- | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.1.5-3 |  |  |  |

For UE not supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.1.5-2.

Table 6.2B.4.1.1.5-2: PCMAX configured UE output power for UE not supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) | Not measured |
| Test ID 1b | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) | Not measured |
| Test ID 1c | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) | Not measured |
| Test ID 1d | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE | N/A | Not measured |
|  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE |  |  |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.1.5-3 |  |  |  |

Table 6.2B.4.1.1.5-3: Test Tolerance (UE configured UE output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| ENBW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < ENBW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

##### 6.2B.4.1.2	Configured Output Power for Intra-Band Non-Contiguous EN-DC

6.2B.4.1.2.1	Test purpose

To verify the UE does not exceed the power bounds defined by PCMAX_L and PCMAX_H.

6.2B.4.1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.2B.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.4.1.0.1.2.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.4.1.2.4	Test description

6.2B.4.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.2B.4.1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.4.1.2.4.1-1: Test configurations table for intra-band non-contiguous EN-DC

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| NR and E-UTRA Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 |  |  | Maximum Wgap |  |  |  |  |
| Test EN-DC bandwidth combination as specified in TS 38.508-1 [5] clause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Highest (NOTE 4) |  |  |  |  |
| NR/E-UTRA Test Parameters for UE supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1d | N/A | QPSK | Full_Allocation | {-13, 7, 10, 23} for PC3 UE {-13, 7, 12, 26} for PC2 UE | CP-OFDM QPSK | Outer_Full | {-13, 7, 10, 23} for PC3 UE {-13, 7, 13, 26} for PC2 UE |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner Full | {-10, 10, 15} |
| NR/E-UTRA Test Parameters for UE not supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1b | N/A | QPSK | Full_Allocation | {-10, 23} for PC3 UE {-10, 26} for PC2 UE | CP-OFDM QPSK | Outer_Full | {-10, 23} for PC3 UE {-10, 26} for PC2 UE |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner Full | {-10, 10, 15} |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 3: The suffix in Test ID identifies the configured power level being tested from the PLTE and PNR lists. NOTE 4: For DC_(n)71AA, only NR SCS of 15 kHz is tested. |  |  |  |  |  |  |  |

Table 6.2B.4.1.2.4.1-2: Void

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.4.1.2.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.4.1.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according table 6.2B.4.1.2.4.1-1 on E-UTRA CC and NR CC respectively. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

3.	Measure the mean transmitted power over E-UTRA component carrier and NR component carrier respectively, or/and measure the sum of mean transmitted power over E-UTRA and NR component carriers according to Table 6.2B.4.1.2.5-1 and Table 6.2B.4.1.2.5-2. The period of the measurement shall be at least the continuous duration of one active sub-frame. For TDD, only slots consisting of only UL symbols are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2B.4.1.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.4.1.2.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.2B.4.1.2.4.3-1: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE doesn’t support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2B.4.1.2.4.3-2: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.4.1.2.4.3-3: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | Defined as PLTE in Table 6.2B.4.1.2.4.1-1 |  |  |

Table 6.2B.4.1.2.4.3-4: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| p-NR-FR1 | Defined as PNR in Table 6.2B.4.1.2.4.1-1 |  |  |
| } |  |  |  |

Table 6.2B.4.1.2.4.3-5: RRCConnectionReconfiguration: p-MaxUE-FR1-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nonCriticalExtension ::= SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE |
|  | 26 |  | Power Class 2 UE |
| } |  |  |  |

6.2B.4.1.2.5	Test requirement

For UE supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.2.5-1.

Table 6.2B.4.1.2.5-1: PCMAX configured UE output power for UE supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | Not measured | Not measured | -10dBm ± (7+TT) |
| Test ID 1b | Not measured | Not measured | 10dBm ± (7+TT) |
| Test ID 1c | Not measured | 10 + (7+TT) / 2 – (7+TT) for PC3 UE | 13.0 + (6+TT) / 10.64 – (7+TT) for PC3 UE |
|  |  | 13 + (6+TT) / 5 - (7+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) | 15.5 + (6+TT) / 12.8 - (6+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) |
|  |  | 13 + (6+TT) / 2 - (7+TT) for PC2 UE (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16)) | 15.5 + (6+TT) / 12.4 - (6+TT) for PC2 UE (Rel-15 and forward UE indicates PC3 on NR band, Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16)) |
| Test ID 1d | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE | N/A | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE |
|  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE |  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.1.5-3 |  |  |  |

For UE not supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.1.5-2.

Table 6.2B.4.1.2.5-2: PCMAX configured UE output power for UE not supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) | Not measured |
| Test ID 1b | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE | N/A | Not measured |
|  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE |  |  |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.1.5-3 |  |  |  |

Table 6.2B.4.1.2.5-3: Test Tolerance for UE configured UE output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.4.1.2.5-4: Test Tolerance for UE configured UE output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

##### 6.2B.4.1.3	Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC)

6.2B.4.1.3.1	Test purpose

To verify the UE does not exceed the power bounds defined by PCMAX and PCMAX_H.

6.2B.4.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC with 1 E-UTRA CC and 1 NR CC within FR1.

6.2B.4.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.4.1.0.1.3.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.4.1.3.4	Test description

6.2B.4.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.2B.4.1.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.4.1.3.4.1-1: Test configurations table for inter-band EN-DC

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause 4.3.1 |  |  | Mid range (NOTE 4) |  |  |  |  |
| Test EN-DC bandwidth combination as specified in TS 38.508-1 [5] clause 4.3.1 |  |  | 5MHz for E-UTRA CC1 and Lowest for NR CC1, Highest for E-UTRA CC1 and Highest for NR CC1 |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Highest |  |  |  |  |
| NR/E-UTRA Test Parameters for UE supporting DPS (NOTE 6) |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1h | N/A | QPSK | Partial_Allocation | {-13, 12, 14, 17, 18, 19, 20, 23} for PC3 UE {-13, 12, 14, 17, 18, 19, 237, 265} for PC2 UE | DFT-s-OFDM QPSK | Inner_Full | {-13, 12, 14, 17, 18, 19, 23, 23} for PC3 UE {-13, 12, 14, 17, 18, 19, 267, 265 } for PC2 UE |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner_Full | {-10, 10, 15} |
| NR/E-UTRA Test Parameters for UE not supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1d | N/A | QPSK | Partial_Allocation | {-10, 10, 15, 23} for PC3 UE {-10, 10, 15, 26} for PC2 UE | CP-OFDM QPSK | Inner_Full | {-10, 10, 15, 23} for PC3 UE {-10, 10, 15, 26} for PC2 UE |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner_Full | {-10, 10, 15} |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 3: The suffix in Test ID identifies the configured power level being tested from the PLTE and PNR lists NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequency. NOTE 5: Only applicable to UEs indicating PC2 on E-UTRA band. NOTE 6: For PC2 UE, Test ID 1g and 1h are not required if UE indicates support of higherPowerLimit-r17.NOTE 7: The test point is only applicable to Rel-15 UEs supporting PC2 on NR band. Test point is applicable for UEs Rel-16 and forward, supporting PC2 on both NR band and NR part power class, or supporting PC2 on NR band and not supporting NR part power class. The capability of NR part power class is indicated in TS 38.508-2 Table A.4.3.2B.2.3.1-3a. NOTE 7: For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency. |  |  |  |  |  |  |  |

Table 6.2B.4.1.3.4.1-2: Void

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to Table 6.2B.4.1.3.4.1-1.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.4.1.3.4.3.

7.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.4.1.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according table 6.2B.4.1.3.4.1-1 on E-UTRA CC and NR CC respectively. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

3.	Measure the mean transmitted power over E-UTRA component carrier and NR component carrier respectively, or/and measure the sum of mean transmitted power over E-UTRA and NR component carriers according to Table 6.2B.4.1.3.5-1 and Table 6.2B.4.1.3.5-2. The period of the measurement shall be at least the continuous duration of one active sub-frame. For TDD, only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2B.4.1.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.4.1.3.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.2B.4.1.3.4.3-1: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE doesn’t support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2B.4.1.3.4.3-2: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.4.1.3.4.3-3: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | Defined as PLTE in Table 6.2B.4.1.3.4.1-1 |  |  |

Table 6.2B.4.1.3.4.3-4: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| p-NR-FR1 | Defined as PNR in Table 6.2B.4.1.3.4.1-1 |  |  |
| } |  |  |  |

Table 6.2B.4.1.3.4.3-5: RRCConnectionReconfiguration: p-MaxUE-FR1-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE |
|  | 26 |  | Power Class 2 UE |
| } |  |  |  |

6.2B.4.1.3.5	Test requirement

For UE supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.3.5-1.

Table 6.2B.4.1.3.5-1: PCMAX configured UE output power for UE supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | Not measured | Not measured | -10dBm ± (7+TT) |
| Test ID 1b | Not measured | Not measured | 15dBm ± (6+TT) |
| Test ID 1c | Not measured | Not measured | 17 ± (5+TT) |
| Test ID 1d | Not measured | Not measured | 20 + (4+TT)/ - (6+TT) |
| Test ID 1e | Not measured | Not measured | 21 + (3+TT)/ - (5+TT) |
| Test ID 1f | Not measured | Not measured | 22+ (2+TT)/ - (5+TT) |
| Test ID 1g | Not measured | 23 + (2+TT) / 17 – (5+TT) for PC3 UE | 23 + (2+TT) / 21.8 - (5+TT) for PC3 UE |
|  |  | 26 + (2+TT) / 20 - (6+TT) for PC2 UE | 26 + (2+TT) / 24.8 - (3+TT) for PC2 UE |
| Test ID 1h | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE. | N/A | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE |
|  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE (For a power class 2 capable UE operating on LTE band) |  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE (For a power class 2 capable UE operating on LTE band) |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.3.5-3 and Table 6.2B.4.1.3.5-4. |  |  |  |

For UE not supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.3.5-2.

Table 6.2B.4.1.3.5-2: PCMAX configured UE output power for UE not supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) | Not measured |
| Test ID 1b | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) | Not measured |
| Test ID 1c | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) | Not measured |
| Test ID 1d | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE | N/A | Not measured |
|  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE (For a power class 2 capable UE operating on LTE band) |  |  |
|  | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC2 UE (For a power class 3 UE operating on LTE band) |  |  |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.3.5-3 and Table 6.2B.4.1.3.5-4. |  |  |  |

Table 6.2B.4.1.3.5-3: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.4.1.3.5-4: Test Tolerance for UE maximum output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in 6.2B.4.2 applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

##### 6.2B.4.1.3_1	Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC)

6.2B.4.1.3_1.1	Test purpose

Same test purpose as in clause 6.2B.4.1.3.1

6.2B.4.1.3_1.2	Test applicability

This test case applies to all types of E-UTRA UE release 16 and forward, supporting inter-band EN-DC with 2 E-UTRA CCs and 1 NR CC FR1.

6.2B.4.1.3_1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.4.1.0.1.3.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.4.1.3_1.4	Test description

6.2B.4.1.3_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.2B.4.1.3_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.4.1.3_1.4.1-1: Test configurations table for inter-band EN-DC

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause 4.3.1 |  |  | Mid range for MCG and SCG |  |  |  |  |
| Test EN-DC bandwidth combination as specified in TS 38.508-1 [5] clause 4.3.1 |  |  | Lowest NRB_agg for E-UTRA CCs and Lowest for NR CC1, Highest NRB_agg for E-UTRA CCs and Highest for NR CC1 |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Highest |  |  |  |  |
| NR/E-UTRA Test Parameters for UE supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | MCG – EUTRA PCC&SCC |  |  | SCG - NR |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1h | N/A | QPSK | Partial_Allocation | {-13, 12, 14, 17, 18, 19, 20, 23} | DFT-s-OFDM QPSK | Inner_Full | {-13, 12, 14, 17, 18, 19, 23, 23} |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner Full | {-10, 10, 15} |
| NR/E-UTRA Test Parameters for UE not supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | MCG – EUTRA PCC&SCC |  |  | SCG - NR |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1d | N/A | QPSK | Partial_Allocation | {-10, 10, 15, 23} | DFT-s-OFDM QPSK | Inner_Full | {-10, 10, 15, 23} |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner_Full | {-10, 10, 15} |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-2 in current specification. NOTE 3: The suffix in Test ID identifies the configured power level being tested from the PLTE and PNR lists. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG PCC and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set according to Table 6.2B.4.1.3_1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.4.1.3_1.4.3.

7.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.4.1.3_1.4.2	Test procedure

1.	Configure E-UTRA SCC according to TS 36.521-1 [10] Annex C.0, C.1, and Annex C.3.0 for all E-UTRA downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [11] clause 5.2A.4. Message contents are defined in clause 6.2B.4.1.3_1.4.3.

3.	SS activates SCC by sending the MAC-CE according to TS 36.321 clauses 5.13 and 6.1.3.8. Wait for at least 2 seconds as per TS 36.133  [12] clause 8.3.3.2.

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according Table 6.2B.4.1.3_1.4.1-1 on E-UTRA PCC and SCC and NR CC respectively. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

6.	Measure the mean transmitted power over E-UTRA CG and NR CG respectively, or/and measure the sum of mean transmitted power over E-UTRA CG and NR CG according to Table 6.2B.4.1.3_1.5-1 and Table 6.2B.4.1.3_1.5-2. The period of the measurement shall be at least the continuous duration of one active sub-frame. For TDD, only slots consisting of only UL symbols are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2B.4.1.3_1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.4.1.3_1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.2B.4.1.3_1.4.3-1: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE doesn’t support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2B.4.1.3_1.4.3-2: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.4.1.3_1.4.3-3: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | Defined as PLTE in Table 6.2B.4.1.3_1.4.1-1 |  |  |

Table 6.2B.4.1.3_1.4.3-4: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| p-NR-FR1 | Defined as PNR in Table 6.2B.4.1.3_1.4.1-1 |  |  |
| } |  |  |  |

Table 6.2B.4.1.3_1.4.3-5: RRCConnectionReconfiguration: p-MaxUE-FR1-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE |
|  | 26 |  | Power Class 2 UE |
| } |  |  |  |

6.2B.4.1.3_1.5	Test requirement

For UE supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.3_1.5-1.

Table 6.2B.4.1.3_1.5-1: PCMAX configured UE output power for UE supporting DPS for PC3 UE

|  | E-UTRA CG | NR CG | Total power measured over all E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | Not measured | Not measured | -10 dBm ± (7+TT) |
| Test ID 1b | Not measured | Not measured | 15 dBm ± (6+TT) |
| Test ID 1c | Not measured | Not measured | 17 dBm ± (5+TT) |
| Test ID 1d | Not measured | Not measured | 20 dBm + (4+TT)/ - (6+TT) |
| Test ID 1e | Not measured | Not measured | 21 dBm + (3+TT)/ - (5+TT) |
| Test ID 1f | Not measured | Not measured | 22 dBm + (2+TT)/ - (5+TT) |
| Test ID 1g | Not measured | 23 dBm + (2+TT) / 17 dBm – (5+TT) | 23 dBm + (2+TT) / 21.8 dBm - (5+TT) |
| Test ID 1h | Maximum output power with reduction as defined in Table 6.2.3A.1.5-1 of TS 36.521-1 [10] for PC3 UE | N/A | Maximum output power with reduction as defined in Table 6.2.3A.1.5-1 of TS 36.521-1 [10] for PC3 UE |
|  | Maximum output power with reduction as defined in Table 6.2.3A.1_3.5-1 of TS 36.521-1 [10] for PC2 UE |  | Maximum output power with reduction as defined in Table 6.2.3A.1_3.5-1 of TS 36.521-1 [10] for PC2 UE |
| Test ID 2a | -10 dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10 dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15 dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10 dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10 dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15 dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.3_1.5-3 and Table 6.2B.4.1.3_1.5-4. |  |  |  |

For UE not supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.3_1.5-2.

Table 6.2B.4.1.3_1.5-2: PCMAX configured UE output power for UE not supporting DPS

|  | E-UTRA CG | NR CG | Total power measured over all E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) | Not measured |
| Test ID 1b | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) | Not measured |
| Test ID 1c | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) | Not measured |
| Test ID 1d | Maximum output power with reduction as defined in Table 6.2.3A.1.5-1 of TS 36.521-1 [10] for PC3 UE | N/A | Not measured |
|  | Maximum output power with reduction as defined in Table 6.2.3A.1_3.5-1 of TS 36.521-1 [10] for PC2 UE |  |  |
| Test ID 2a | -10 dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10 dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15 dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10 dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10 dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15 dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2A-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.3_1.5-3 and Table 6.2B.4.1.3_1.5-4. |  |  |  |

Table 6.2B.4.1.3_1.5-3: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.4.1.3_1.5-4: Test Tolerance for UE maximum output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in 6.2B.4.2 applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

##### 6.2B.4.1.4	Configured Output Power for Inter-Band EN-DC including FR2 (1 NR CC)

6.2B.4.1.4.1	Test purpose

Same test purpose as in clause 6.2.4.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.4.1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

The requirements of this test for NR carrier are covered in test cases 6.2B.1.4 Maximum output power for Inter-Band EN-DC including FR2, 6.2B.2.4 Maximum output power reduction for Inter-Band EN-DC including FR2 and 6.2B.3.4 UE maximum output power with additional requirements for Inter-Band EN-DC including FR2 to all types of NR UE release 15 and forward.

6.2B.4.1.4.3	Minimum conformance requirements

UE configured output power requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.5 and 6.2.5A of [10] and for NR single carrier and CA operation specified in subclause 6.2.4, 6.2A.4 and 6.2D.4 of [9] apply.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.4.1.4.

6.2B.4.1.4.4	Test description

This test is covered by clause 6.2B.1.4 Maximum output power for Inter-Band EN-DC including FR2, 6.2B.2.4 Maximum output power reduction for Inter-Band EN-DC including FR2 and 6.2B.3.4 UE maximum output power with additional requirements for Inter-Band EN-DC including FR2.

6.2B.4.1.4.5	Test requirement

This test is covered by clause 6.2B.1.4 Maximum output power for Inter-Band EN-DC including FR2, 6.2B.2.4 Maximum output power reduction for Inter-Band EN-DC including FR2 and 6.2B.3.4 UE maximum output power with additional requirements for Inter-Band EN-DC including FR2.

##### 6.2B.4.1.4_1	Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC)

Editor’s note: The following aspects are either missing or not yet determined:

-	The referred test case 6.2.4_1 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

-	The referred test case 6.2.4_1 in TS 38.521-2 is incomplete for band n259.

6.2B.4.1.4_1.1	Test purpose

Same test purpose as in clause 6.2.4_1.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.4.1.4_1.2	Test applicability

This test case applies to all types of E-UTRA UE release 16 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC and supporting mpr-PowerBoost-FR2-r16 UE capability.

6.2B.4.1.4_1.3	Minimum conformance requirements

UE configured output power requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.5 and 6.2.5A of [10] and for NR single carrier specified in subclause 6.2.4_1 of [9] apply.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.4.1.4.

6.2B.4.1.4_1.4	Test description

Same test description as in clause 6.2.4_1.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.2.4_1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C, clauses C.0, C.1 and C.3.0, and uplink signals according to Annex H, clauses H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.2.4_1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

Same test procedure as in clause 6.2.4_1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.4.1.4_1.5	Test requirement

Same test requirement as in clause 6.2.4_1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.2B.4.1.5	Configured Output Power for Inter-Band EN-DC including both FR1 and FR2

6.2B.4.1.5.1	Test purpose

Same test purpose as in clause 6.2.4.1 in TS 38.521-1 [8] for NR FR1 carrier and 6.2.4.1 in TS 38.521-2 [9] for NR FR2 carrier.

6.2B.4.1.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for maximum output power apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 6.2B.

#### 6.2B.4.2	ΔTIB,c for EN-DC

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in Tables below applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

Unless ΔTIB,c  is specified for the NE-DC configuration, the specified ΔTIB,c  for the EN-DC configuration including same bands as the corresponding NE-DC configuration is applicable for the NE-DC configuration.

##### 6.2B.4.2.1	Intra-Band Contiguous EN-DC

ΔTIB,c is not applicable for intra-band contiguous EN-DC.

##### 6.2B.4.2.2	Intra-Band non-Contiguous EN-DC

ΔTIB,c is not applicable for intra-band non-contiguous EN-DC.

##### 6.2B.4.2.3	Inter-Band EN-DC within FR1

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1_n3 | 1 | 0.3 |
|  | n3 | 0.3 |
| DC_1_n5 | 1 | 0.3 |
|  | n5 | 0.3 |
| DC_1_n8 | 1 | 0.3 |
|  | n8 | 0.3 |
| DC_1_n28 | 1 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n41 | 1 | 0.5 |
|  | n41 | 0.5 |
| DC_1_n77 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_2_n5 | 2 | 0.3 |
|  | n5 | 0.3 |
| DC_2_n41 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_2_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n71 | 2 | 0.3 |
|  | n71 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 2 | 0.6 |
|  | n77 | 0.8 |
| DC_2_n78 | 2 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n1 | 3 | 0.3 |
|  | n1 | 0.3 |
| DC_3_n5 | 3 | 0.3 |
|  | n5 | 0.3 |
| DC_3_n7 | 3 | 0.5 |
|  | n7 | 0.5 |
| DC_3_n8 | 3 | 0.3 |
|  | n8 | 0.3 |
| DC_3_n28 | 3 | 0.3 |
|  | n28 | 0.3 |
| DC_3_n41 | 3 | 0.5 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| DC_3_n77 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_5_n2, DC_5-5_n2 | 5 | 0.3 |
|  | n2 | 0.3 |
| DC_5_n66 | 5 | 0.3 |
|  | n66 | 0.3 |
| DC_5_n77 | 5 | 0.6 |
|  | n77 | 0.8 |
| DC_5_n78 | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n1 | 7 | 0.6 |
|  | n1 | 0.5 |
| DC_7_n3 | 7 | 0.5 |
|  | n3 | 0.5 |
| DC_7_n5 | 7 | 0.3 |
|  | n5 | 0.3 |
| DC_7_n8 | 7 | 0.3 |
|  | n8 | 0.6 |
| DC_7_n28 | 7 | 0.3 |
|  | n28 | 0.3 |
| DC_7_n66, DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_7_n78, DC_7-7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_8_n1 | 8 | 0.3 |
|  | n1 | 0.3 |
| DC_8_n3 | 8 | 0.3 |
|  | n3 | 0.3 |
| DC_8_n20 | 8 | 0.4 |
|  | n20 | 0.4 |
| DC_8_n28 | 8 | 0.6 |
|  | n28 | 0.5 |
| DC_8_n41 | 8 | 0.3 |
|  | n41 | 0.3 |
| DC_8_n77 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_8_n78 | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_11_n77 | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_11_n78 | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_12_n2 | 12 | 0.3 |
|  | n2 | 0.3 |
| DC_12_n5 | 12 | 0.4 |
|  | n5 | 0.8 |
| DC_12_n66 | 12 | 0.8 |
|  | n66 | 0.3 |
| DC_12_n78 | 12 | 0.5 |
|  | n78 | 0.8 |
| DC_13_n2 | 13 | 0.3 |
|  | n2 | 0.3 |
| DC_13_n66 | 13 | 0.3 |
|  | n66 | 0.3 |
| DC_13_n77 | 13 | 0.5 |
|  | n77 | 0.8 |
| DC_14_n2 | 14 | 0.3 |
|  | n2 | 0.3 |
| DC_14_n66 | 14 | 0.3 |
|  | n66 | 0.3 |
| DC_18_n77 | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_18_n78 | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_19_n77 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78 | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n1 | 20 | 0.3 |
|  | n1 | 0.3 |
| DC_20_n3 | 20 | 0.3 |
|  | n3 | 0.3 |
| DC_20_n7 | 20 | 0.3 |
|  | n7 | 0.3 |
| DC_20_n8 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28 | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_20_n78 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21_n77 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78 | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_25_n41 | 25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_26_n41 | 26 | 0.3 |
|  | n41 | 0.3 |
| DC_26_n77 | 26 | 0.3 |
|  | n77 | 0.8 |
| DC_26_n78 | 26 | 0.3 |
|  | n78 | 0.8 |
| DC_28_n5 | 28 | 0.5 |
|  | n5 | 0.5 |
| DC_28_n77 | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_28_n78 | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_30_n5 | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.8 |
| DC_38_n78 | n78 | 0.5 |
| DC_39_n41 | 39 | 0.5 |
|  | n41 | 0.5 |
| DC_39_n79 | 39 | 0.3 |
|  | n79 | 0.8 |
| DC_40_n1 | n1 | 0.5 |
|  | 40 | 0.5 |
| DC_40_n415 | 40 | 0.5 |
|  | n41 | 0.5 |
| DC_40_n78 | n78 | 0.56 |
| DC_40_n79 | 40 | 0.3 |
|  | n79 | 0.8 |
| DC_41_n28 | 41 | 0.3 |
|  | n28 | 0.3 |
| DC_41_n77 | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_41_n78 | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_41_n79 | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_48_n5 | 48 | 0.3 |
|  | n5 | 0.3 |
| DC_48_n46 | 48 | 0.8 |
| DC_48_n66 | 48 | 0.8 |
|  | n66 | 0.6 |
| DC_66_n2 | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_66_n41 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_66_n71 | 66 | 0.3 |
|  | n71 | 0.3 |
| DC_66_n77 DC_66-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |
| DC_66_n78 | 66 | 0.6 |
|  | n78 | 0.8 |
| DC_71_n2 | 71 | 0.3 |
|  | n2 | 0.3 |
| DC_71_n66 | 71 | 0.3 |
|  | N66 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. NOTE 5: Applicable for UE supporting inter-band EN-DC without simultaneous Rx/Tx. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.3.2-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
| DC_1-3_n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n41 | 0.33 |
| DC_1-3_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
| DC_1-5_n78 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |
| DC_1-7_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n3 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.3 |
| DC_1-8_n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-18_n77 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n3 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |
| DC_1-20_n28 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-20_n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-21_n77 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |
| DC_1-21_n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_1-21_n79 | 1 | 0.3 |
|  | 21 | 0.3 |
| DC_1-28_n3 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |
| DC_1-28_n78 DC_1_n28-n78 | 1 | 0.3 |
|  | 28 or n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | n28 | 0.3 |
| DC_1-41_n28 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
| DC_1-41_n41 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
| DC_1-41_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-41_n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-42_n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-42_n78 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-42_n79 | 1 | 0.3 |
|  | 42 | 0.8 |
| DC_1_n77-n79 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78-n79 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_2-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |
|  | 2 | 0.6 |
| DC_2_n5-n77 | n5 | 0.3 |
|  | n77 | 0.8 |
|  | 2 | 0.6 |
| DC_2-13_n77 | 13 | 0.5 |
|  | n77 | 0.8 |
| DC_2-14_n2 | 2 | 0.3 |
|  | 14 | 0.3 |
|  | n2 | 0.3 |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | n66 | 0.5 |
| DC_2-30_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_2-66_n5 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_2-66_n41 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_2-66_n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |
|  | 2 | 0.6 |
| DC_2-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |
| DC_2-(n)71 | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_3-5_n78 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.3 |
| DC_3-7_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_3-18_n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_3-18_n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_3-19_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_3-19_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_3-19_n79 | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_3-20_n1 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | n1 | 0.3 |
| DC_3-20_n28 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_3-20_n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-21_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-21_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-21_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_3-28_n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_3_n28-n78 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| DC_3-40_n1 | 3 | 0.5 |
|  | 40 | 0.5 |
|  | n1 | 0.5 |
| DC_3-41_n28 | 3 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n28 | 0.3 |
| DC_3-41_n41 | 3 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| DC_3-41_n77 | 3 | 0.6 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n77 | 0.8 |
| DC_3-42_n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-42_n78 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-42_n79 | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_3_n77-n79 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78-n79 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_5-7_n78 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_5-30_n66 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_7-20_n28 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_7-20_n1 | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.5 |
| DC_7-20_n3 | 7 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.5 |
| DC_7-20_n78 | 7 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_7-28_n3 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n3 | 0.5 |
| DC_7-28_n78 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_7_n28-n78 | 7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
|  | 13 | 0.3 |
| DC_13_n2-n77 | n2 | 0.6 |
|  | n77 | 0.8 |
|  | 13 | 0.3 |
| DC_13-66-n2 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | 13 | 0.5 |
| DC_13-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |
| DC_14-66_n2 DC_14-66-66_n2 | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_14-66_n66 | 14 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_18-41_n3 | 18 | 0.3 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n3 | 0.5 |
| DC_18-41_n77 | 18 | 0.3 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n77 | 0.8 |
| DC_18-41_n78 | 18 | 0.3 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n78 | 0.8 |
| DC_19-21_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_19-21_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_19-21_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
| DC_19-42_n77 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-42_n78 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-42_n79 | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_19_n77-n79 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78-n79 | 19 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_20_n28-n78 | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_20_n76-n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_21-42_n77 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-42_n78 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-42_n79 | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_21_n77-n79 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78-n79 | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_28_n7-n78 | 28 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |
| DC_28-42_n78 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_41-42_n78 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | 66 | 0.6 |
| DC_66_n2-n77 | n2 | 0.6 |
|  | n77 | 0.8 |
|  | 66 | 0.6 |
| DC_66_n5-n77 | n5 | 0.3 |
|  | n77 | 0.8 |
| DC_66-(n)71 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-7_n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |
| DC_1-3-8_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-19_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-19_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_1-3-20_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-20_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-21_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_1-3-21_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_1-3-21_n79 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_1-3-28_n78 DC_1-3_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 or n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_1-7-20_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7-20_n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | n78 | 0.8 |
| DC_1-7-28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7_n28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-19-42_n77 | 1 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-19-42_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-19-42_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-20_n28-n78 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-21-42_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-21-42_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-21-42_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_2-7-13_n66 DC_2-7-7-13_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |
| DC_2-7-66_n66 DC_2-7-7-66_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 |  |
| DC_2-7-66_n78 DC_2-7-7-66_n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |
|  | 2 | 0.5 |
| DC_2-13-66_n77 | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n77 | 0.8 |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-14-66_n66 DC_2-2-14-66_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | 2 | 0.6 |
| DC_2-66_n5-n77 | 66 | 0.6 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |
| DC_2-66-(n)71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_3-7-20_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n28 | 0.5 |
| DC_3-7-20_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-7-28_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-19-21_n78 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-19-21_n79 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_3-19-42_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-19-42_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-19-42_n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_3-20_n28-n78 | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-21-42_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-21-42_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-21-42_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_7-20_n28-n78 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
|  | 13 | 0.3 |
| DC_13-66_n2-n77 | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |
| DC_19-21-42_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-21-42_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-21-42_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |

6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-7-20_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.6 |
| DC_1-3-7-28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-7_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-19-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-19-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-3-20_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-21-42_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.6 |
| DC_1-3-21-42_n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_1-7-20_n28-n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_3-7-20_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

6.2B.4.2.3.5	ΔTIB,c for EN-DC six bands

Table 6.2B.4.2.3.5-1: ΔTIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

##### 6.2B.4.2.3a	Inter-band NE-DC within FR1

Unless ΔTIB,c  is specified in this clause, the value of ΔTIB,c  for the correspondingly specified EN-DC configuration in clause 6.2B.4.2.3 is applicable.

Table 6.2B.4.2.3a-1: ΔTIB,c due to NE-DC(two bands)

| Inter-band NE-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_n28_39 | n28 | 0.3 |
|  | 39 | 0.3 |

##### 6.2B.4.2.4	Inter-band EN-DC including FR2

6.2B.4.2.4.1	ΔTIB,c for EN-DC two bands

Unless otherwise stated, ΔTIB,c for E-UTRA and NR FR2 bands of inter-band EN-DC combinations defined in table 5.5B.5.1-1 is set to zero.

Table 6.2B.4.2.4.1-1: Void

6.2B.4.2.4.2	ΔTIB,c for EN-DC three bands

Unless otherwise stated, ΔTIB,c for NR FR2 bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.2-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the NR FR2 bands.

Table 6.2B.4.2.4.2-1: Void

6.2B.4.2.4.3	ΔTIB,c for EN-DC four bands

Unless otherwise stated, ΔTIB,c for NR FR2 bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.3-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the NR FR2 bands.

Table 6.2B.4.2.4.3-1: Void

6.2B.4.2.4.4	ΔTIB,c for EN-DC five bands

Unless otherwise stated, ΔTIB,c for NR FR2 bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.4-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the NR FR2 bands.

Table 6.2B.4.2.4.4-1: Void

6.2B.4.2.4.5	Void

##### 6.2B.4.2.5	Inter-band EN-DC including both FR1 and FR2

6.2B.4.2.5.1	ΔTIB,c for EN-DC three bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.2-1, ΔTIB,c for constituent NR FR2 bands is set to zero, and ΔTIB,c for constituent E-UTRA and NR FR1 bands is the same as those for the corresponding inter band EN-DC configuration without the NR FR2 bands specified in 6.2B.4.2.3.

Table 6.2B.4.2.5.1-1: Void

6.2B.4.2.5.2	ΔTIB,c for EN-DC four bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.3-1, ΔTIB,c for constituent NR FR2 bands is set to zero, and ΔTIB,c for constituent E-UTRA and NR FR1 bands is the same as those for the corresponding inter band EN-DC configuration without the NR FR2 bands specified in 6.2B.4.2.3.

6.2B.4.2.5.3	ΔTIB,c for EN-DC five bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.4-1, ΔTIB,c for constituent NR FR2 bands is set to zero, and ΔTIB,c for constituent E-UTRA and NR FR1 bands is the same as those for the corresponding inter band EN-DC configuration without the NR FR2 bands specified in 6.2B.4.2.3.

6.2B.4.2.5.4	ΔTIB,c for EN-DC six bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.5-1, ΔTIB,c for constituent NR FR2 bands is set to zero, and ΔTIB,c for constituent E-UTRA and NR FR1 bands is the same as those for the corresponding inter band EN-DC configuration without the NR FR2 bands specified in 6.2B.4.2.3.

### 6.2B.5	Configured Output Power for NR-DC

#### 6.2B.5.1	Configured Output power Level

##### 6.2B.5.1.1	Configured Output Power Level for Inter-band NR-DC between FR1 and FR2

6.2B.5.1.1.1	Test purpose

Same test purpose as in clause 6.2.4.1 in TS 38.521-1 [8] for NR FR1 carrier and 6.2.4.1 in TS 38.521-2 [9] for NR FR2 carrier.

6.2B.5.1.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for configured output power apply and are tested as part of the standalone NR within FR1 in clause 6.2.4 in TS 38.521-1 [8] and standalone NR within FR2 in clause 6.2.4 in TS 38.521-2 [9].

## 6.2E	Transmitter power for V2X in FR1

### 6.2E.1	UE Maximum Output Power for V2X

#### 6.2E.1.0	Minimum conformance requirements

6.2E.1.0.1	UE maximum output power for Intra-band contiguous V2X

For intra-band contiguous V2X operating UE, the allowed UE maximum output power shall be applied in Table 6.2.2-1 [5] for E-UTRA SL transmission or applied in Table 6.2.1-1 [2] for NR SL transmission, respectively.

Table 6.2E.1.0.1-1: Maximum output power for V2X combination (continuous sub-blocks)

| V2X configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| V2X_(n)47AA |  |  | 23 | +2/-31 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported |  |  |  |  |

6.2E.1.0.2	UE maximum output power for Intra-band non-contiguous V2X

For intra-band non-contiguous V2X operating UE, the allowed UE maximum output power shall be applied in Table 6.2.2-1 [5] for E-UTRA SL transmission or applied in Table 6.2.1-1 [2] for NR SL transmission, respectively.

Table 6.2E.1.0.2-1: Maximum output power for V2X combination (non-contiguous sub-blocks)

| V2X configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| V2X_47A_n47A |  |  | 23 | +2/-31 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported |  |  |  |  |

6.2E.1.0.3	UE maximum output power for Inter-band V2X

For the inter-band con-current NR V2X operation, the maximum output power is specified in Table 6.2E.1.0.3-1. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.0.3-1: Con-current V2X UE Power Class

| V2X con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_20A_n38A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_n71A_47A |  |  |  |  | 23 | +2/-34 |  |  |
| NOTE 1: The con-current band combinations is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2E.1.

#### 6.2E.1.1	UE maximum output power for Intra-band contiguous V2X

6.2E.1.1.1	Test purpose

To verify that UE’s transmit power doesn’t exceed the range prescribed by the specified power value.

6.2E.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.0.1. The requirements in this test case can be well covered in clause 6.2.2G of TS 36.521-1 [10] and clause 6.2E.1 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.2E.1.2	UE maximum output power for Intra-band non-contiguous V2X

6.2E.1.1.1	Test purpose

To verify that UE’s transmit power doesn’t exceed the range prescribed by the specified power value.

6.2E.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.0.2. The requirements in this test case can be well covered in clause 6.2.2G of TS 36.521-1 [10] and clause 6.2E.1 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.2E.1.3	UE maximum output power for Inter-band V2X

##### 6.2E.1.3.1	UE maximum output power for Inter-band V2X with E-UTRA Uu and NR Sidelink

Editor's note:

-	Connection diagram is FFS

-	Generic test procedure is FFS

-	Message exception is FFS

-	Test requirements are FFS

6.2E.1.3.1.1	Test purpose

Same test purpose as in clause 6.2E.1.1.1 in TS 38.521-1 [8].

6.2E.1.3.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward supporting concurrent operation between E-UTRA Uu and NR sidelink.

6.2E.1.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.1.0.3.

6.2E.1.3.1.4	Test description

6.2E.1.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on NR operating bands specified in clause 5.3E.3, channel bandwidths and sub-carrier spacings for the NR sidelink carrier specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each E-UTRA-NR V2X band combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.1.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 of TS 36.521-1 [10] for E-UTRA Uu carrier. The details of sidelink reference measurement channels (RMCs) are specified in A.7 of TS 38.521-1 [8] for NR V2X carrier. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521.1 [10] Annex C.2 for E-UTRA Uu carrier.

Table 6.2E.1.3.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1. |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [11] subclause 4.3.1 for E-UTRA carrier and TS 38.508-1 [6] clause 4.3.1 for NR carrier. |  |  | Low range for E-UTRA Uu carrier and NR sidelink carrier High range for E-UTRA Uu carrier and NR sidelink carrier |  |  |
| Test inter-band V2X bandwidth combination as specified in clause 5.3E.3. |  |  | Lowest for E-UTRA Uu carrier and NR sidelink carrier Highest for E-UTRA Uu carrier and NR sidelink carrier |  |  |
| Test SCS for the NR carrier as specified in TS 38.521-1 [8] Table 5.3.5-1. |  |  | 15kHz |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | E-UTRA Uu carrier |  |  | NR sidelink Carrier |  |
|  | Downlink Configuration | Uplink Configuration |  | Modulation | PSCCH and PSSCH RB allocation (Note 2) |
|  |  | Modulation | RB allocation (Note 1) |  |  |
| 1 | N/A | QPSK | Partial_Allocation | CP-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1E-1 in TS 38.521-1 [8]. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, FFS for TE diagram and clause FFS for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] clause 4.10. Message content exceptions are defined in clause 6.2E.1.3.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, and uplink signals according to TS 36.521-1 [10] Annex H for the E-UTRA cell.

4.	The E-UTRA UL Reference Measurement channels and the V2X Reference Measurement Channel are set according to Table 6.2E.1.3.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in State FFS according to TS 36.508 [7] clause FFS.

6.2E.1.3.1.4.2	Test procedure

1.	The SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the E-UTRA UL RMC according to Table 6.2E.1.3.1.4.1-1. The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the E-UTRA UL and NR sidelink RMCs.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level. Configure the UE to transmit PSCCH and PSSCH with the PUMAX level of each test points.

3.	Measure the mean power over all component carriers in the inter-band con-current V2X configuration. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2E.1.3.1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6 for E-UTRA Uu carrier and TS 38.508-1 [6] clause 4.6.3 for NR sidelink carrier with the following exceptions:

FFS

6.2E.1.3.1.5	Test requirement

FFS

##### 6.2E.1.3.2	UE maximum output power reduction for Inter-band V2X with NR Uu and E-UTRA V2X

Editor's note:

-	Connection diagram is FFS

-	Generic test procedure is FFS

-	Message exception is FFS

-	Test requirements are FFS

6.2E.1.3.2.1	Test purpose

Same test purpose as in clause 6.2E.1.1.1 in TS 38.521-1 [8].

6.2E.1.3.2.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting concurrent operation between NR Uu and E-UTRA V2X.

6.2E.1.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.1.0.3.

6.2E.1.3.2.4	Test description

6.2E.1.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on NR operating bands specified in clause 5.3E.3, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA V2X carrier are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each E-UTRA-NR V2X band combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.1.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 of TS 38.521-1 [8] for NR Uu carrier. The details of sidelink reference measurement channels (RMCs) are specified in Annex A.6 of TS 36.521-1 [10] for E-UTRA V2X carrier. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C.2 for NR Uu carrier.

Table 6.2E.1.3.2.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1. |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [11] subclause 4.3.1 for E-UTRA carrier and TS 38.508-1 [6] clause 4.3.1 for NR carrier. |  |  | Low range for NR Uu carrier and E-UTRA sidelink carrier High range for NR Uu carrier and E-UTR sidelink carrier |  |  |
| Test inter-band V2X bandwidth combination as specified in clause 5.3E.3. |  |  | Lowest for NR Uu carrier and E-UTRA sidelink carrier Highest for NR Uu carrier and E-UTRA sidelink carrier |  |  |
| Test SCS for the NR carrier as specified in TS 38.521-1 [8] Table 5.3.5-1. |  |  | 15kHz |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | NR Uu Carrier |  |  | E-UTRA V2X Carrier |  |
|  | Downlink Configuration | Uplink Configuration |  | Modulation | PSSCH RB allocation |
|  |  | Modulation | RB allocation (Note 1) |  |  |
| 1 | N/A | DFT-s-OFDM QPSK | Inner Full | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1 in TS 38.521-1 [8]. |  |  |  |  |  |

- Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, FFS for TE diagram and clause FFS for UE diagram.

2.	The parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3. The parameter settings for the E-UTRA V2X sidelink transmission are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.2E.1.3.2.4.3.

3.	Downlink signals are initially set up according to TS 38.521-1 [8] Annex C.0, and uplink signals according to TS 38.521-1 [8] Annex G for the NR cell.

4.	The NR UL Reference Measurement channels and the E-UTRA V2X Reference Measurement Channel are set according to Table 6.2E.1.3.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

8.	Ensure the UE is in state FFS according to TS 38.508-1 [6] clause FFS.

6.2E.1.3.2.4.2	Test procedure

1.	The SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the NR UL RMC according to Table 6.2E.1.3.2.4.1-1 on NR CC. The UE starts to perform the E-UTRA V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UL has no payload and no loopback data to send, the UE transmits uplink MAC padding bits on the NR UL and E-UTRA V2X sidelink RMC.

2.	Send continuously uplink power control "up" commands to the UE until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command starting in this step for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers in the inter-band con-current V2X configuration. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms). For TDD slots with transient periods are not under test.

6.2E.1.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 for NR Uu carrier and TS 36.508 [11] clause 4.6 for E-UTRA V2X sidelink carrier with the following exceptions:

FFS

6.2E.1.3.2.5	Test requirement

FFS

### 6.2E.2	UE maximum output power reduction for V2X

#### 6.2E.2.0	Minimum conformance requirements

6.2E.2.0.1	UE maximum output power reduction for Intra-band V2X

For intra-band V2X operating UE, maximum output power reduction specified in clause 6.2.3G [5] and in clause 6.2E.2 [2] apply, respectively.

6.2E.2.0.2	UE maximum output power reduction for Inter-band V2X

For the inter-band con-current NR V2X operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The MPR requirements in subclause 6.2.3 of TS 36.101 [5] apply for E-UTRA Uu operation in licensed band, and the MPR requirements in subclause 6.2E.2 of TS 38.101-1 [2] apply for NR sidelink operation. The MPR requirements in subclause 6.2.3G of TS 36.101 [5] apply for E-UTRA V2X operation, and the MPR requirements in subclause 6.2.2 of TS 38.101-1 [2] apply for NR Uu operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2E.2.

#### 6.2E.2.1	UE maximum output power reduction for Intra-band V2X

6.2E.2.1.1	Test purpose

To verify that the backoff of UE’s transmit power doesn’t exceed the range prescribed by the specified MPR value and tolerance specified in clause of configured output power.

6.2E.2.1.2	Test applicability

This test applies to all types of UE release 16 and forward, supporting E-UTRA-NR intra-band contiguous V2X configuration or E-UTRA-NR intra-band non-contiguous V2X configuration.

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.1 and 6.2E.1.2. The requirements in this test case can be well covered in clause 6.2.3G of TS 36.521-1 [10] and clause 6.2E.2 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.2E.2.2	UE maximum output power reduction for Inter-band V2X

##### 6.2E.2.2.1	UE maximum output power reduction for Inter-band V2X with E-UTRA Uu and NR Sidelink

Editor's note:

-	Connection diagram is FFS

-	Generic test procedure is FFS

-	Message exception is FFS

-	Test requirements are FFS

6.2E.2.2.1.1	Test purpose

Same test purpose as in clause 6.2E.2.1.1 in TS 38.521-1 [8].

6.2E.2.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward supporting concurrent operation between E-UTRA Uu and NR sidelink.

6.2E.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.2.

6.2E.2.2.1.4	Test description

6.2E.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on NR operating bands specified in clause 5.3E.3, channel bandwidths and sub-carrier spacings for the NR sidelink carrier specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each E-UTRA-NR V2X band combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 of TS 36.521-1 [10] for E-UTRA Uu carrier. The details of sidelink reference measurement channels (RMCs) are specified in A.7 of TS 38.521-1 [8] for NR V2X carrier. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521.1 [10] Annex C.2 for E-UTRA Uu carrier.

Table 6.2E.2.2.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1. |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [11] subclause 4.3.1 for E-UTRA carrier and TS 38.508-1 [6] clause 4.3.1 for NR carrier. |  |  | Low range for E-UTRA Uu carrier and NR sidelink carrier High range for E-UTRA Uu carrier and NR sidelink carrier |  |  |
| Test inter-band V2X bandwidth combination as specified in clause 5.3E.3. |  |  | Lowest for E-UTRA Uu carrier and NR sidelink carrier Highest for E-UTRA Uu carrier and NR sidelink carrier |  |  |
| Test SCS for the NR carrier as specified in TS 38.521-1 [8] Table 5.3.5-1. |  |  | 15kHz |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | E-UTRA Uu carrier |  |  | NR sidelink Carrier |  |
|  | Downlink Configuration | Uplink Configuration |  | Modulation | PSCCH and PSSCH RB allocation (Note 2) |
|  |  | Modulation | RB allocation (Note 1) |  |  |
| 1 | N/A | QPSK | Partial_Allocation | CP-OFDM 16QAM | Inner_Full |
| 2 |  | QPSK | Full_Allocation | CP-OFDM 16QAM | Outer_Full |
| 3 |  | 16QAM | Partial_Allocation | CP-OFDM 64QAM | Outer_Full |
| 4 |  | 16QAM | Full_Allocation | CP-OFDM 256QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1E-1 in TS 38.521-1 [8]. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, FFS for TE diagram and clause FFS for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] clause 4.10. Message content exceptions are defined in clause 6.2E.1.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, and uplink signals according to TS 36.521-1 [10] Annex H for the E-UTRA cell.

4.	The E-UTRA UL Reference Measurement channels and the V2X Reference Measurement Channel are set according to Table 6.2E.2.2.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in State FFS according to TS 36.508 [7] clause FFS. Message content exceptions are defined in clause 6.2E.2.2.1.4.3.

6.2E.2.2.1.4.2	Test procedure

1.	The SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the E-UTRA UL RMC according to Table 6.2E.2.2.1.4.1-1. The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the E-UTRA UL and NR sidelink RMCs.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level. Configure the UE to transmit PSCCH and PSSCH with the PUMAX level of each test points.

3.	Measure the mean power over all component carriers in the inter-band con-current V2X configuration. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.2E.2.2.1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6 for E-UTRA Uu carrier and TS 38.508-1 [6] clause 4.6.3 for NR sidelink carrier with the following exceptions:

FFS

6.2E.2.2.1.5	Test requirement

FFS

##### 6.2E.2.2.2	UE maximum output power reduction for Inter-band V2X with NR Uu and E-UTRA V2X

Editor's note:

-	Connection diagram is FFS

-	Generic test procedure is FFS

-	Message exception is FFS

-	Test requirements are FFS

6.2E.2.2.2.1	Test purpose

Same test purpose as in clause 6.2E.2.1.1 in TS 38.521-1 [8].

6.2E.2.2.2.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting concurrent operation between NR Uu and E-UTRA V2X.

6.2E.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.2.

6.2E.2.2.2.4	Test description

6.2E.2.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on NR operating bands specified in clause 5.3E.3, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA V2X carrier are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each E-UTRA-NR V2X band combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.2.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 of TS 38.521-1 [8] for NR Uu carrier. The details of sidelink reference measurement channels (RMCs) are specified in Annex A.6 of TS 36.521-1 [10] for E-UTRA V2X carrier. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C.2 for NR Uu carrier.

Table 6.2E.2.2.2.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1. |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [11] subclause 4.3.1 for E-UTRA carrier and TS 38.508-1 [6] clause 4.3.1 for NR carrier. |  |  | Low range for NR Uu carrier and E-UTRA sidelink carrier High range for NR Uu carrier and E-UTRA sidelink carrier |  |  |
| Test inter-band V2X bandwidth combination as specified in clause 5.3E.3. |  |  | Lowest for NR Uu carrier and E-UTRA sidelink carrier Highest for NR Uu carrier and E-UTRA sidelink carrier |  |  |
| Test SCS for the NR carrier as specified in TS 38.521-1 [8] Table 5.3.5-1. |  |  | 15kHz |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | NR Uu Carrier |  |  | E-UTRA V2X Carrier |  |
|  | Downlink Configuration | Uplink Configuration |  | Modulation | PSSCH RB allocation |
|  |  | Modulation | RB allocation (Note 1) |  |  |
| 1 | N/A | DFT-s-OFDM QPSK | Inner_Full | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 2 |  | DFT-s-OFDM QPSK | Outer_Full | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 3 |  | DFT-s-OFDM 16QAM | Inner_Full | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 4 |  | DFT-s-OFDM 16QAM | Outer_Full | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 5 |  | DFT-s-OFDM 64QAM | Outer_Full | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 6 |  | DFT-s-OFDM 256QAM | Outer_Full | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 7 |  | CP-OFDM QPSK | Inner_Full | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 8 |  | CP-OFDM 16PSK | Inner_Full | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 9 |  | CP-OFDM 16PSK | Outer_Full | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 10 |  | CP-OFDM 64PSK | Outer_Full | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| 11 |  | CP-OFDM 256PSK | Outer_Full | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1 in TS 38.521-1 [8]. |  |  |  |  |  |

- Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, FFS for TE diagram and clause FFS for UE diagram.

2.	The parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3. The parameter settings for the E-UTRA V2X sidelink transmission are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.2E.1.1.4.3.

3.	Downlink signals are initially set up according to TS 38.521-1 [8] Annex C.0, and uplink signals according to TS 38.521-1 [8] Annex G for the NR cell.

4.	The NR UL Reference Measurement channels and the E-UTRA V2X Reference Measurement Channel are set according to Table 6.2E.2.2.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

8.	Ensure the UE is in state FFS according to TS 38.508-1 [6] clause FFS. Message contents are defined in clause 6.2E.2.2.2.4.3.

6.2E.2.2.2.4.2	Test procedure

1.	The SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the NR UL RMC according to Table 6.2E.2.2.2.4.1-1 on NR CC. The UE starts to perform the E-UTRA V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UL has no payload and no loopback data to send, the UE transmits uplink MAC padding bits on the NR UL and E-UTRA V2X sidelink RMC.

2.	Send continuously uplink power control "up" commands to the UE until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command starting in this step for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers in the inter-band con-current V2X configuration. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms). For TDD slots with transient periods are not under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2E.2.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2E.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 for NR Uu carrier and TS 36.508 [11] clause 4.6 for E-UTRA V2X sidelink carrier with the following exceptions:

FFS

6.2E.2.2.2.5	Test requirement

FFS

## 6.3	Output power dynamics

## 6.3A	Output Power Dynamics for CA without EN-DC

### 6.3A.1	UE Output Power Dynamics for CA

#### 6.3A.1.1	UE Output Power Dynamics for NR CA between FR 1 and FR 2 without EN-DC

6.3A.1.1.1	Test purpose

Same test purpose as in clause 6.3 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.3 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.3A.1.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for maximum output power apply and are tested in TS 38.521-1 [8] clauses 6.3 and 6.2A and TS 38.521-2 [9] clauses 6.3 and 6.3A.

## 6.3B	Output power dynamics for DC

### 6.3B.1	Minimum Output Power for EN-DC

#### 6.3B.1.1	Minimum Output Power for intra-band contiguous EN-DC

6.3B.1.1.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.3B.1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-1 [8] for the NR carrier.

Same minimum conformance requirements as in clause 6.3.2.3 in TS 36.521-1 [10] for E-UTRA carrier.

For intra-band contiguous EN-DC operation in FR1, minimum output power requirements specified in clause 6.3.1 of TS 38.101-1 [2] and clause 6.3.2 of TS 36.101 [5] shall only apply when the power of all NR and E-UTRA carriers are set to minimum value.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.1.1.4	Test description

Same test descriptions as in clause 6.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 6.3B.1.1.4-1: Test configuration table

| E-UTRA Test Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency | Downlink | Uplink |  |
|  |  | N/A for min output power test | Modulation | RB allocation |
| 5 MHz | MidRange |  | QPSK | 25 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |

For Initial conditions as in clause 6.3.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 6.3B.1.1.4-1.

3.1.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The UL Reference Measurement channels are set according to Table 6.3B.1.1.4-1.

Step 6 of Initial conditions as in clause 6.3.1.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.3.1.4.2 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1.	For E-UTRA component, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to table 6.3B.1.1.4-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.1.	For E-UTRA component, send continuously uplink power control "down" commands in every uplink scheduling information to the UE.

6.3B.1.1.5	Test requirements

Same test requirement as in clause 6.3.1.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.1.2	Minimum output power for intra-band non-contiguous EN-DC

6.3B.1.2.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.3B.1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.

6.3B.1.2.4	Test description

Same test descriptions as in clause 6.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 6.3B.1.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | Low with maxWGap, High with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.3-1 across bandwidth combination sets supported by the UE | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the lowest NRB_SCG and highest NRB_SCG are tested for Lowest NRB_agg, and Highest NRB_agg, respectively. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 6.3B.1.2.4-1.

For Initial conditions as in clause 6.3.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 6.3B.1.2.4-1.

3.1.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C, clauses C.0, C.1 and C.3.0, and uplink signals according to Annex H, clauses H.1 and H.3.0.

4.1.	The UL Reference Measurement channels are set according to Table 6.3B.1.2.4-1.

Step 6 of Initial conditions as in clause 6.3.1.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.3B.1.2.5	Test requirements

Same test requirement as in clause 6.3.1.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.1.3	Minimum output power for inter-band EN-DC within FR1 (1 NR CC)

6.3B.1.3.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

6.3B.1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.1.3.4	Test description

Same test descriptions as in clause 6.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.1.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3B.1.3.5	Test requirements

Same test requirement as in clause 6.3.1.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.1.4	Minimum Output Power for EN-DC Interband including FR2 (1 NR CC)

Editor's note:	The following aspects of the clause are for future consideration:

-	Testing of extreme conditions for FR2 is FFS.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 2 and 4.

6.3B.1.4.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.3B.1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.1.

6.3B.1.4.4	Test description

Same test description as in clause 6.3.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.3.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of initial conditions as in clause 6.3.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4.5	Test requirements

Same test requirement as in clause 6.3.1.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.3B.1.4_1	Minimum output power for inter-band EN-DC including FR2 (>1 NR CC)

##### 6.3B.1.4_1.1	Minimum output power for inter-band EN-DC including FR2 (2 NR CCs)

Editor's note: The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.1.1 in TS 38.521-2 [9] is incomplete for NR aggregated channel bandwidth wider than 400MHz and power classes 1, 2, 4.

6.3B.1.4_1.1.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.3B.1.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.

6.3B.1.4_1.1.4	Test description

6.3B.1.4_1.1.4.1	Initial condition

Same test description as in clause 6.3A.1.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3A.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3A.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3A.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4_1.1.5	Test Requirements

Same test requirement as in clause 6.3A.1.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.3B.1.4_1.2	Minimum output power for inter-band EN-DC including FR2 (3 NR CCs)

Editor's note: The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.1.2 in TS 38.521-2 [9] is incomplete for NR aggregated channel bandwidth wider than 400MHz and power classes 1, 2, 4.

6.3B.1.4_1.2.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.3B.1.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.

6.3B.1.4_1.2.4	Test description

6.3B.1.4_1.2.4.1	Initial condition

Same test description as in clause 6.3A.1.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3A.1.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3A.1.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3A.1.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4_1.2.5	Test Requirements

Same test requirement as in clause 6.3A.1.2.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.3B.1.4_1.3	Minimum output power for inter-band EN-DC including FR2 (4 NR CCs)

Editor's note: The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.1.3 in TS 38.521-2 [9] is incomplete for NR aggregated channel bandwidth wider than 400MHz and power classes 1, 2, 4.

6.3B.1.4_1.3.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.3B.1.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.

6.3B.1.4_1.3.4	Test description

6.3B.1.4_1.3.4.1	Initial condition

Same test description as in clause 6.3A.1.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3A.1.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3A.1.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3A.1.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4_1.3.5	Test Requirements

Same test requirement as in clause 6.3A.1.3.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.3B.1.4_1.4	Minimum output power for inter-band EN-DC including FR2 (5 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.1.3 in TS 38.521-2 [9] is incomplete for NR aggregated channel bandwidth wider than 400MHz and power classes 1, 2, 4.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.3B.1.4_1.4.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 NR UL CCs.

6.3B.1.4_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.

6.3B.1.4_1.4.4	Test description

6.3B.1.4_1.4.4.1	Initial condition

Same test description as in clause 6.3A.1.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3A.1.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3A.1.4.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3A.1.4.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4_1.4.5	Test Requirements

Same test requirement as in clause 6.3A.1.4.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.3B.1.4_1.5	Minimum output power for inter-band EN-DC including FR2 (6 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.1.3 in TS 38.521-2 [9] is incomplete for NR aggregated channel bandwidth wider than 400MHz and power classes 1, 2, 4.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.3B.1.4_1.5.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 6 NR UL CCs.

6.3B.1.4_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.

6.3B.1.4_1.5.4	Test description

6.3B.1.4_1.5.4.1	Initial condition

Same test description as in clause 6.3A.1.5.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3A.1.5.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3A.1.5.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3A.1.5.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4_1.5.5	Test Requirements

Same test requirement as in clause 6.3A.1.5.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.3B.1.4_1.6	Minimum output power for inter-band EN-DC including FR2 (7 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.1.3 in TS 38.521-2 [9] is incomplete for NR aggregated channel bandwidth wider than 400MHz and power classes 1, 2, 4.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.3B.1.4_1.6.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 7 NR UL CCs.

6.3B.1.4_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.

6.3B.1.4_1.6.4	Test description

6.3B.1.4_1.6.4.1	Initial condition

Same test description as in clause 6.3A.1.6.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3A.1.6.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3A.1.6.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3A.1.6.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4_1.6.5	Test Requirements

Same test requirement as in clause 6.3A.1.6.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.3B.1.4_1.7	Minimum output power for inter-band EN-DC including FR2 (8 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.1.3 in TS 38.521-2 [9] is incomplete for NR aggregated channel bandwidth wider than 400MHz and power classes 1, 2, 4.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.3B.1.4_1.7.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 8 NR UL CCs.

6.3B.1.4_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.

6.3B.1.4_1.7.4	Test description

6.3B.1.4_1.7.4.1	Initial condition

Same test description as in clause 6.3A.1.7.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3A.1.7.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3A.1.7.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3A.1.7.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4_1.7.5	Test Requirements

Same test requirement as in clause 6.3A.1.7.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.3B.1.4D	Minimum output power for inter-band EN-DC including FR2 for UL MIMO

6.3B.1.4D.1	Test purpose

Same test purpose as in clause 6.3D.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.1.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC FR2.

6.3B.1.4D.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3D.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.3B.

6.3B.1.4D.4	Test Description

Same test description as in clause 6.3D.1.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 6.3D.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3D.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same Test procedure as in clause 6.3D.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.1.4D.5	Test Requirement

Same test requirement as specified in clause 6.3D.1.5 of TS 38.521-2 [9] for the NR carrier(s).

### 6.3B.2	Transmit OFF Power for EN-DC

#### 6.3B.2.1	Transmit OFF Power for intra-band contiguous EN-DC

6.3B.2.1.1	Test purpose

Same test purpose as in clause 6.3.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.2.1.2	Test applicability

The requirements of this test apply in Clause 6.3B.3 Tx ON/OFF time mask/PUCCH time mask to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.3B.2.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.2.1.4	Test description

This test is covered by Clause 6.3B.3 Tx ON/OFF time mask/PUCCH time mask for EN-DC.

6.3B.2.1.5	Test requirements

Same test requirement as in clause 6.3.2.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.2.2	Transmit OFF Power for intra-band non-contiguous EN-DC

6.3B.2.2.1	Test purpose

Same test purpose as in clause 6.3.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.2.2.2	Test applicability

The requirements of this test apply in clause 6.3B.3 Tx ON/OFF time mask/PUCCH time mask to all types of E-UTRA UE Release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.3B.2.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.2.2.4	Test description

This test is covered by Clause 6.3B.3 Tx ON/OFF time mask/PUCCH time mask for EN-DC.

6.3B.2.2.5	Test requirements

Same test requirement as in clause 6.3.2.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.2.3	Transmit OFF Power for inter-band EN-DC within FR1 (1 NR CC)

6.3B.2.3.1	Test purpose

Same test purpose as in clause 6.3.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.2.3.2	Test applicability

The requirements of this test apply in Clause 6.3B.3.3 Tx ON/OFF time mask/PUCCH time mask to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

6.3B.2.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.2.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.2.3.4	Test description

This test is covered by Clause 6.3B.3 Tx ON/OFF time mask/PUCCH time mask for EN-DC.

6.3B.2.3.5	Test requirements

Same test requirement as in clause 6.3.2.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.2.4	Transmit OFF Power for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	This test case is complete for Band n257. Following aspects are either missing or not yet determined:

- The referred test case 6.3.2 in TS 38.521-2 [9] is incomplete for other than band n257.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1 FR2b, 2 and 4.

6.3B.2.4.1	Test purpose

Same test purpose as in clause 6.3.2.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.2.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.3B.2.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.2.4.4	Test description

Same test description as in clause 6.3.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.2.4.5	Test requirements

Same test requirement as in clause 6.3.2.5 in TS 38.521-2 [8] for the NR carrier.

#### 6.3B.2.4_1	Void

### 6.3B.2.4D	Transmit OFF Power for inter-band EN-DC including FR2 for UL-MIMO

6.3B.2.4D.1	Test purpose

Same test purpose as in clause 6.3D.2.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.2.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 for UL-MIMO.

6.3B.2.4D.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3D.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.3B.

6.3B.2.4D.4	Test Description

Same test description as in clause 6.3D.2.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 6.3D.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3D.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same Test procedure as in clause 6.3D.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.2.4D.5	Test Requirement

Same test requirement as specified in clause 6.3D.2.5 of TS 38.521-2 [9] for the NR carrier(s).

### 6.3B.3	Transmit ON/OFF time mask/PUCCH time mask for EN-DC

#### 6.3B.3.1	Transmit ON/OFF time mask for intra-band contiguous EN-DC

6.3B.3.1.1	Test purpose

Same test purpose as in clause 6.3.3.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.3.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.3B.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.2.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.3.1.4	Test description

Same test descriptions as in clause 6.3.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.3.3.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA Downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.3.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.3.3.2.4.2 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3B.3.1.5	Test requirements

Same test requirement as in clause 6.3.3.2.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.3.2	Transmit ON/OFF time mask for intra-band non-contiguous EN-DC

6.3B.3.2.1	Test purpose

Same test purpose as in clause 6.3.3.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.3.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.3B.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.2.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.3.2.4	Test description

Same test descriptions as in clause 6.3.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.3.3.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA Downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.3.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.3.3.2.4.2 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3B.3.2.5	Test requirements

Same test requirement as in clause 6.3.3.2.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.3.3	Transmit ON/OFF time mask for inter-band EN-DC within FR1 (1 NR CC)

6.3B.3.3.1	Test purpose

Same test purpose as in clause 6.3.3.2.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.3.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

6.3B.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.2.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

6.3B.3.3.4	Test description

Same test descriptions as in clause 6.3.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.3.3.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA Downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.3.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3.3.2.4.2 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3B.3.3.5	Test requirements

Same test requirement as in clause 6.3.3.2.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.3B.3.4	Transmit ON/OFF time mask for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.3.3.2 in TS 38.521-2 [9] is incomplete for power class 1, 2 and 4.

-	The referred test case 6.3.3.2 in TS 38.521-2 [9] is incomplete for band n259.

6.3B.3.4.1	Test purpose

Same test purpose as in clause 6.3.3.2.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.3.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC FR2 with 1 NR UL CC.

6.3B.3.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.3B.

6.3B.3.4.4	Test Description

Same test description as in clause 6.3.3.2.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 6.3.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.3.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same Test procedure as in clause 6.3.3.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.3.4.5	Test Requirement

Same test requirement as specified in clause 6.3.3.2.5 of TS 38.521-2 [9] for the NR carrier(s).

#### 6.3B.3.4_1	Transmit ON/OFF time mask for Inter-Band EN-DC including FR2 (>1 NR CC)

##### 6.3B.3.4_1.1	Transmit ON/OFF time mask for Inter-Band EN-DC including FR2 (2 NR CCs)

Editor's note:	This test case is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.3.1.1 in TS 38.521-2 is incomplete on test requirements.

-	Measurement Uncertainty and Test Tolerances are FFS.

-	The method of setting UE transmitted power is FFS.

-	TP analysis is FFS.

-	Applicability of Beam peak of single UL is FFS.

-	The UPLF test mode is applicable to UEs Release 16 and forward. This test case is incomplete for Release 15 	until UE PHR method is used to prevent SCell drop.

6.3B.3.4_1.1.1	Test purpose

Same test purpose as in clause 6.3A.3.1.1.1 in TS 38.521-2 [9] for the NR carriers.

6.3B.3.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.3B.3.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3A.3.1.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.3B.

6.3B.3.4_1.1.4	Test description

Same test description as in clause 6.3A.3.1.1.4 in TS 38.521-2 [9] for the NR carriers with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.3A.3.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3A.3.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3A.3.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7 of TS 36.508 [11].

6.3B.3.4_1.1.5	Test Requirements

Same test requirement as in clause 6.3A.3.1.1.5 in TS 38.521-2 [9] for the NR carriers.

### 6.3B.3.4D	Transmit ON/OFF time mask for inter-band EN-DC including FR2 for UL-MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.3D.3 in TS 38.521-2 [9] is incomplete

6.3B.3.4D.1	Test purpose

Same test purpose as in clause 6.3D.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.3.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC FR2.

6.3B.3.4D.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3D.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.3B.

6.3B.3.4D.4	Test Description

Same test description as in clause 6.3D.3.1.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 6.3D.3.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3D.3.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same Test procedure as in clause 6.3D.3.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.3.4D.5	Test Requirement

Same test requirement as specified in TS 38.521-2 [9] clause 6.3D.3.1.5 for the NR carrier(s).

### 6.3B.3_1	E-UTRA and NR switching time mask for switching between two uplink carriers

#### 6.3B.3_1.1	E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The test clause number needs further discussion.

6.3B.3_1.1.1	Test purpose

To verify that the E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC meets the requirements given in 6.3B.4.1 in TS 38.101-3 [4].

The E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC defines the transient period(s) and the switching period allowed between two uplink carriers for an uplink band pair of an inter-band EN-DC configuration when the capability uplinkTxSwitchingPeriod is present.

6.3B.3_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 16 and forward, supporting 2UL inter-band EN-DC and dynamic UL Tx switching.

6.3B.3_1.1.3	Minimum conformance requirements

In addition to the requirements in 6.3B.0 and the maximum output power requirement specified in Table 6.2B.1.3-1 with inter-band EN-DC (two bands), the switching time mask specified in this sub-clause is applicable for an uplink band pair of an inter-band EN-DC configuration without SUL band when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 [14], where E-UTRA UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3B.4.1-1 are only located in NR carrier, and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3B.4.1-1: Time mask for switching between E-UTRA UL carrier and NR UL carrier, where the switching period is located in NR carrier

The requirements apply for the case of co-located and synchronized network deployment with the max receiving timing difference of 3us between the two carriers.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3B.4.1 for the NR and the E-UTRA carrier.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied.

6.3B.3_1.1.4	Test description

6.3B.3_1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2, and are shown in table 6.3B.3_1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C, clause C.2 and in TS 38.521-1 [8] Annex C, clause C.2 for E-UTRA CG and NR CG respectively.

Table 6.3B.3_1.1.4.1-1: Test configuration table for Inter-band EN-DC Uplink switching

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  | Normal |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause 4.3.1 |  | Mid range for E-UTRA CC1 and NR CC1 (NOTE 3) |  |  |  |
| Test EN-DC bandwidth combination as specified in TS 38.508-1 [5] clause 4.3.1 |  | Highest for E-UTRA CC1 and NR CC1 |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  | Highest |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A | QPSK | Outer Full | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.2 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement Channel is set according to Table 6.3A.3.1_1.4.1-1 for E-UTRA CG and NR CG respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7. 	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.3A.3.1_1.4.3 in TS 38.521-1 [8] for the NR CG.

6.3B.3_1.1.4.2	Test procedure

Carrier 1 and Carrier 2 refers to the E-UTRA component and the NR carrier respectively

1.	Configure Carrier 2 according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure Carrier 2 as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.1_1.4.3 in TS 38.521-1 [8] for the NR Carrier 2.

3.	For Carrier 1, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.3B.3_1.1.4.1-1 on E-UTRA subframe n-1, where E-UTRA subframe n is an uplink slot for Carrier 2 and n-1 ≥ 20 when SCS=15 kHz (n-1 ≥ 40 when SCS=30 kHz, n-1 ≥ 80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

4	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

5.	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3B.3_1.1.4.1-1 on Carrier 2 on slot n (n ≥1) and slot m, with both slot n and slot m being uplink slots for Carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in TS 38.508-1 [5] subclause 4.3.6.1.1.2.

6.	Measure the output power of UE PUSCH transmission for Carrier 1 during the E-UTRA subframe n-1, for Carrier 1 excluding a transient period of 20 µs in the end of the E-UTRA subframe n-1.

7.	Measure the sum of output power of UE PUSCH transmission on Carrier 2 over all antenna connectors during slot n and slot m excluding a switching period X and a transient period of 10 µs in the beginning of slot n and in the end of slot m. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

8.	For Carrier 1, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.3B.3_1.1.4.1-1 on E-UTRA subframe m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

9.	Measure the output power of UE PUSCH transmission for Carrier 1 during slot m+1 excluding a transient period of 20 µs in the beginning of slot m+1.

6.3B.3_1.1.5	Test requirements

The requirements for the power of Carrier 1 measured in step 6, step 9 of the test procedure, and the power of Carrier 2 measured in step 7 shall not exceed the values specified in Table 6.3B.3_1.1.5-1 and Table 6.3B.3_1.1.5-2, respectively.

Table 6.3B.3_1.1.5-1: E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC (On power on the E-UTRA Carrier)

|  | Measured output power |
| --- | --- |
| Transmit ON power | For each E-UTRA band tested, same as the requirements in Table 6.2.3.5-1 of TS 36.521-1 [10] as approporate. |

Table 6.3B.3_1.1.5-2: E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC (On power on the NR Carrier)

|  | Measured output power |
| --- | --- |
| Transmit ON power | For each NR band tested, same as Test ID 9 in Table 6.2.2.5-1 to Table 6.2.2.5-3 of TS 38.521-1 [8] as appropriate |
| NOTE1: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2.2.5-5 of TS 38.521-1 [8] |  |

### 6.3B.4	PRACH time mask for EN-DC

#### 6.3B.4.1	PRACH time mask for intra-band contiguous EN-DC

6.3B.4.1.1	Test purpose

Same test purpose as in clause 6.3.3.4.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.4.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.3B.4.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.4.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.4.1.4	Test description

Same test description as in clause 6.3.3.4.4 in TS 38.521-1 [8] with the following exception:

Table 6.3B.4.1.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different DC bandwidth classes. | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 6.3B.4.1.4-1.

For Initial conditions as in clause 6.3.3.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 5 of Initial conditions as in clause 6.3.3.4.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

6.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

For Message contents as in clause 6.3.3.4.4.3 in TS 38.521-1 [8], the following exception:

Table 6.3B.4.1.4-2: RACH-ConfigGeneric: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 14 | Unpaired Spectrum | PRACH Format 0 |
|  | 128 | Unpaired Spectrum | PRACH Format A3 |

6.3B.4.1.5	Test requirements

Same test requirement as in clause 6.3.3.4.5 in TS 38.521-1 [8].

#### 6.3B.4.2	PRACH Time Mask for intra-band non-contiguous EN-DC

6.3B.4.2.1	Test purpose

Same test purpose as in clause 6.3.3.4.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.4.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.3B.4.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.4.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.4.2.4	Test description

Same test description as in clause 6.3.3.4.4 in TS 38.521-1 [8] with the following exception:

Table 6.3B.4.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 36TS 38.508-1 [76] clause 4.3.1 for different EN-DC bandwidth classes | MaxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.3-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 6.3B.4.2.4-1.

For Initial conditions as in clause 6.3.3.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 5 of Initial conditions as in clause 6.3.3.4.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

6.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

For Message contents as in clause 6.3.3.4.4.3 in TS 38.521-1 [8], the following exception:

Table 6.3B.4.2.4-2: RACH-ConfigGeneric: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 14 | Unpaired Spectrum | PRACH Format 0 |
|  | 128 | Unpaired Spectrum | PRACH Format A3 |

6.3B.4.2.5	Test requirements

Same test requirement as in clause 6.3.3.4.5 in TS 38.521-1 [8].

#### 6.3B.4.3	PRACH Time Mask for inter-band EN-DC within FR1 (1 NR CC)

6.3B.4.3.1	Test purpose

Same test purpose as in clause 6.3.3.4.1 in TS 38.521-1 [8] for the NR carrier.

6.3B.4.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

6.3B.4.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.4.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.4.3.4	Test description

Same test description as in clause 6.3.3.4.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.For Initial conditions as in clause 6.3.3.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 5 of Initial conditions as in clause 6.3.3.4.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

6.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test requirement as in clause 6.3.3.4.5 in TS 38.521-1 [8].

### 6.3B.4.4	PRACH Time Mask for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.3.3.4 in TS 38.521-2 [9] is incomplete

6.3B.4.4.1	Test purpose

Same test purpose as in clause 6.3.3.4.1 in TS 38.521-2 [9] for the NR carrier.

6.3B.4.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 .

6.3B.4.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.3.4.3 in TS 38.521-2 [9] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

6.3B.4.4.4	Test description

Same test description as in clause 6.3.3.4.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 6.3.3.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 5 of Initial conditions as in clause 6.3.3.4.4.1 in TS 38.521-2 [9] is replaced by the following two steps:

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

6.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.4.4.5	Test requirement

Same test requirement as specified in clause 6.3.3.4.5 of TS 38.521-2 [9] for the NR carrier(s).

### 6.3B.5	Output power dynamics for EN-DC with UL sharing from UE perspective

#### 6.3B.5.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective

No test case details are specified. Current test procedures for time masks are based on power measurement in relatively long period compared with transient period. For time masks between 2 active time slots with different power level, the test procedure can't provide enough resolution to identify non-conformant UEs. Therefore the minimum requirement is not testable.

### 6.3B.6	Output power dynamics for intra-band EN-DC without dual PA capability

No test case details are specified. Current test procedures for time masks are based on power measurement in relatively long period compared with transient period. For time masks between 2 active time slots with different power level, the test procedure can't provide enough resolution to identify non-conformant UEs. Therefore the minimum requirement is not testable.

### 6.3B.7	Output power dynamics for intra-band EN-DC with dual PA capability

No test case details are specified. Current test procedures for time masks are based on power measurement in relatively long period compared with transient period. For time masks between 2 active time slots with different power level, the test procedure can't provide enough resolution to identify non-conformant UEs. Therefore the minimum requirement is not testable.

### 6.3B.8	Power control for EN-DC

#### 6.3B.8.1	Absolute power tolerance for EN-DC

##### 6.3B.8.1.1	Absolute power tolerance for intra-band contiguous EN-DC

6.3B.8.1.1.1	Test purpose

Same test purpose as in clause 6.3.4.2.1 in TS 38.521-1 [8] for the NR FR1 carrier(s),

6.3B.8.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.3B.8.1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.2.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.1.1.4	Test description

Same test description as in clause 6.3.4.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.2.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.2.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.3B.8.1.1.5	Test Requirement

Same test requirement as in clause 6.3.4.2.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.1.2	Absolute power tolerance for intra-band non-contiguous EN-DC

6.3B.8.1.2.1	Test purpose

Same test purpose as in clause 6.3.4.2.1 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.3B.8.1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.2.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.1.2.4	Test description

Same test description as in clause 6.3.4.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.2.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.2.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.3B.8.1.2.5	Test Requirement

Same test requirement as in clause 6.3.4.2.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.1.3	Absolute power tolerance for inter-band EN-DC within FR1 (1 NR CC)

6.3B.8.1.3.1	Test purpose

Same test purpose as in clause 6.3.4.2.1 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

6.3B.8.1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.2.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.1.3.4	Test description

Same test description as in clause 6.3.4.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.2.4.1-1 will be updated as below.

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3B.8.1.3.5	Test Requirement

Same test requirement as in clause 6.3.4.2.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.1.4	Absolute power tolerance for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.3.4.2 in TS 38.521-2 [9] is incomplete for extreme conditions.

-	The referred test case 6.3.4.2 in TS 38.521-2 [9] is incomplete for PC 1, 2, and 4.

6.3B.8.1.4.1	Test purpose

Same test purpose as in clause 6.3.4.2.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.3B.8.1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.3B.8.1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.1.4.4	Test description

Same test description as in clause 6.3.4.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.2.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.3.4.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.2.4.1 in TS 38.521-2 [9] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.7.

6.3B.8.1.4.5	Test Requirement

Same test requirement as in clause 6.3.4.2.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.3B.8.2	Relative power tolerance for EN-DC

##### 6.3B.8.2.1	Relative power tolerance for intra-band contiguous EN-DC

6.3B.8.2.1.1	Test purpose

Same test purpose as in clause 6.3.4.3.1 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.3B.8.2.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.3.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.2.1.4	Test description

Same test description as in clause 6.3.4.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.3.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.3B.8.2.1.5	Test Requirement

Same test requirement as in clause 6.3.4.3.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.2.2	Relative power tolerance for intra-band non-contiguous EN-DC

6.3B.8.2.2.1	Test purpose

Same test purpose as in clause 6.3.4.3.1 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.3B.8.2.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.3.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.2.2.4	Test description

Same test description as in clause 6.3.4.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.3.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.3B.8.2.2.5	Test Requirement

Same test requirement as in clause 6.3.4.3.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.2.3	Relative power tolerance for inter-band EN-DC within FR1 (1 NR CC)

6.3B.8.2.3.1	Test purpose

Same test purpose as in clause 6.3.4.3.1 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.2.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

6.3B.8.2.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.3.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.2.3.4	Test description

Same test description as in clause 6.3.4.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.3.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3B.8.2.3.5	Test Requirement

Same test requirement as in clause 6.3.4.3.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.2.4	Relative power tolerance for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.3.4.3 in TS 38.521-2 [9] is incomplete

6.3B.8.2.4.1	Test purpose

Same test purpose as in clause 6.3.4.3.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.3B.8.2.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.3B.8.2.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.2.4.4	Test description

Same test description as in clause 6.3.4.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.3.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-2 [9] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.7.

6.3B.8.2.4.5	Test Requirement

Same test requirement as in clause 6.3.4.3.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.3B.8.3	Aggregate power tolerance for EN-DC

##### 6.3B.8.3.1	Aggregate power tolerance for intra-band contiguous EN-DC

6.3B.8.3.1.1	Test purpose

Same test purpose as in clause 6.3.4.4.1 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.3B.8.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.4.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.3.1.4	Test description

Same test description as in clause 6.3.4.4.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.4.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.3.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.3B.8.3.1.5	Test Requirement

Same test requirement as in clause 6.3.4.4.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.3.2	Aggregate power tolerance for intra-band non-contiguous EN-DC

6.3B.8.3.2.1	Test purpose

Same test purpose as in clause 6.3.4.4.1 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.3B.8.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.4.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.3.2.4	Test description

Same test description as in clause 6.3.4.4.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.4.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.4.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.3B.8.3.2.5	Test Requirement

Same test requirement as in clause 6.3.4.4.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.3.3	Aggregate power tolerance for inter-band EN-DC within FR1 (1 NR CC)

6.3B.8.3.3.1	Test purpose

Same test purpose as in clause 6.3.4.4.1 in TS 38.521-1 [8] for the NR FR1 carrier(s).

6.3B.8.3.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

6.3B.8.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.4.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.3.3.4	Test description

Same test description as in clause 6.3.4.4.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.4.4.1-1 will be updated as below.

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.3.4.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.4.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.3B.8.3.3.5	Test Requirement

Same test requirement as in clause 6.3.4.4.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.3B.8.3.4	Aggregate power tolerance for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.3.4.4 in TS 38.521-2 [9] is incomplete for PC 1, 2, and 4.

6.3B.8.3.4.1	Test purpose

Same test purpose as in clause 6.3.4.4.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.3B.8.3.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.3B.8.3.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.4.4.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.3B.8.3.4.4	Test description

Same test description as in clause 6.3.4.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

Notes defined in Table 6.3.4.4.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.3.4.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.3.4.4.4.1 in TS 38.521-2 [9] is replaced by the following steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.3B.8.3.4.5	Test Requirement

Same test requirement as in clause 6.3.4.4.5 in TS 38.521-2 [9] for the NR carrier.

## 6.3E	Output power dynamics for V2X

### 6.3E.0	Minimum conformance requirements

6.3E.0.1	General

The E-UTRA SL and NR SL switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

6.3E.0.2	Output power dynamics for intra-band V2X operation

For intra-band V2X operation bands specified in subclause 5.3E.1 and 5.3E.2, the SL switching time masks in Figure 6.3E.2-1 shall apply.

The switching time shall be located on the RAT of lower priority when NR SL and LTE SL have different priorities based on priority information specified in TS 38.213. It is up to UE implementation when NR SL and LTE SL have the same priority based on priority information specified in TS 38.213.

Figure 6.3E.0.2-1: Time mask for switching between NR SL and E-UTRA SL

6.3E.0.3	Output power dynamics for inter-band V2X con-current operation

For inter-band con-current NR V2X operation, the output power dynamics requirement shall be applied per each component carrier. The output dynamic requirements specified in clause 6.3 of TS 36.101 [5] apply for E-UTRA UL transmission and the requirements specified in clause 6.3E of TS 38.101-1 [2] apply for NR SL transmission. The output dynamic requirements specified in clause 6.3.2G, 6.3.3G, 6.3.4G of TS 36.101 [5] apply for E-UTRA SL transmission and the requirements specified in clause 6.3 of TS 38.101-1 [2] apply for NR UL transmission.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.3E.

### 6.3E.1	Output power dynamics for intra-band V2X operation

SL switching time mask requirements are specified for intra-band V2X operation, giving criteria on how the switching period position is decided based on priority information. No test is needed for this time mask requirement.

### 6.3E.2	Output power dynamics for inter-band V2X operation

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.3.2, 6.3.3, 6.3.4 and 6.3.5 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.3E.1.1, [6.3E.2.2, 6.3E.3.2.1, 6.3E.3.3.1 and 6.3E.3.4.1] of TS 38.521-1 [8] and don’t need to be tested again.

For inter-band V2X configuration with NU Uu and E-UTRA V2X sidelink, the requirements in this test case can be well covered in clause 6.3.1, 6.3.2, 6.3.3 and 6.3.4 of TS 38.521-1 [8] for NR Uu carrier and clause 6.3.2G.1, 6.3.3G.1, 6.3.4G.1 and 6.3.4G.4 of TS 36.521-1 [10] and don’t need to be tested again.

## 6.4	Transmit signal quality

### 6.4A	Transmit Signal Quality for inter-band NR CA between FR and FR2 without EN-DC

#### 6.4A.1	Frequency error for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.4A.1.1	Test purpose

Same test purpose as in clause 6.4.1 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.4.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.4A.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for frequency error apply and are tested in TS 38.521-1 [8] clauses 6.4 and 6.4A and TS 38.521-2 [9] clauses 6.4 and 6.4A.

### 6.4A.2	Transmit Modulation Quality for inter-band NR CA without EN-DC

#### 6.4A.2.1	Error Vector Magnitude for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.4A.2.1.1	Test purpose

Same test purpose as in clause 6.4.2.1 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.4.2.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.4A.2.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for error vector magnitude apply and are tested in TS 38.521-1 [8] clauses 6.4 and 6.4A and TS 38.521-2 [9] clauses 6.4 and 6.4A.

#### 6.4A.2.2	Carrier Leakage for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.4A.2.2.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.4.2.2 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.4A.2.2.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for carrier leakage apply and are tested in TS 38.521-1 [8] clauses 6.4 and 6.4A and TS 38.521-2 [9] clauses 6.4 and 6.4A.

#### 6.4A.2.3	In-band Emissions for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.4A.2.3.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.4.2.3 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.4A.2.3.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for in-band emissions apply and are tested in TS 38.521-1 [8] clauses 6.4 and 6.4A and TS 38.521-2 [9] clauses 6.4 and 6.4A.

#### 6.4A.2.4	EVM Equalizer Spectral Flatness for inter-band NR CA between FR 1 and FR 2 without EN-DC

6.4A.2.4.1	Test purpose

Same test purpose as in clause 6.4.2.4 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.4.2.4 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.4A.2.4.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for EVM equalizer spectral flatness apply and are tested in TS 38.521-1 [8] clauses 6.4 and 6.4A and TS 38.521-2 [9] clauses 6.4 and 6.4A.

## 6.4B	Transmit Signal Quality for DC

### 6.4B.1	Frequency error

#### 6.4B.1.1	Frequency error for Intra-band contiguous EN-DC

6.4B.1.1.1	Test purpose

Same test purpose as in clause 6.4.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.4B.1.1.3	Minimum conformance requirements

For intra-band contiguous EN-DC, the requirement shall apply on each component carrier as defined in clause 6.5.1 in TS 38.101-3 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.4B.1.1.

6.4B.1.1.4	Test description

Same test description as in clause 6.4.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Notes defined in Table 6.4.1.4.1-1 will be updated as below.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.4B.1.1.5	Test Requirement

The 10 frequency error Δf results must fulfil the test requirement defined in clause 6.4.1.5 TS 38.521-1 [8].

#### 6.4B.1.2	Frequency error for Intra-band non-contiguous EN-DC

6.4B.1.2.1	Test purpose

Same test purpose as in clause 6.4.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.4B.1.2.3	Minimum conformance requirements

For intra-band non-contiguous EN-DC, the requirement shall apply on each component carrier as defined in clause 6.5.1 in TS 38.101-3 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.4B.1.2.

6.4B.1.2.4	Test description

Same test description as in clause 6.4.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.3, all of these configurations shall be tested with applicable test parameters for each intra-band non-contiguous EN-DC configuration specified in clause 5.3B.1.3, and are shown in table 6.4B.1.2.4-1.

Table 6.4B.1.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  | MinWgap , MaxWgap |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 |  |  | Highest NRB_agg (NOTE3) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation |  | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |  |  |  |  |

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1 with the exception that E-UTRA test frequency are specified in Table 7.4B.2.4.1-1 based on intra-band non-contiguous EN-DC configuration specified in clause 5.3B.1.3.

For Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Steps 4 and 6 of Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8] is replaced by:

4.	The DL and UL Reference Measurement frequencies are set according to Table 6.4B.1.2.4-1.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.4B.1.2.5	Test Requirement

The 10 frequency error Δf results must fulfil the test requirement defined in TS 38.521-1 [8], clause 6.4.1.5.

#### 6.4B.1.3	Frequency error for Inter-band EN-DC within FR1 (1 NR CC)

6.4B.1.3.1	Test purpose

Same test purpose as in clause 6.4.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.4B.1.3.3	Minimum conformance requirements

For inter-band EN-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [5] and in clause 6.4.1 in TS 38.101-1 [2], respectively, with all component carriers active. If multiple component carriers are assigned to one E-UTRA band, the requirements in clauses 6.5.1A in TS 36.101 [5] apply for those component carriers, and if multiple component carriers are assigned to one NR band, the requirements in clauses 6.4A.1 in TS 38.101-1 [2] apply for those component carriers.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.4B.1.3.

6.4B.1.3.4	Test description

Same test description as in clause 6.4.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4B.1.3.5	Test Requirement

The 10 frequency error Δf results must fulfil the test requirement defined in 6.4.1.5 TS 38.521-1 [8].

#### 6.4B.1.3a	Frequency Error for inter-band NE-DC within FR1

No exception requirements applicable to E-UTRA or LTE.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 6.4.1 and 6.4A.1 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.5.1 and 6.5.1A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

#### 6.4B.1.4	Frequency Error for inter-band EN-DC including FR2 (1 NR CC)

6.4B.1.4.1	Test purpose

Same test purpose as in clause 6.4.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.1.4.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band EN-DC including FR2 with 1 NR UL CC.

6.4B.1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.

6.4B.1.4.4	Test description

Same test description as in clause 6.4.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.4.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.1.4.5	Test requirements

Same test requirement as in clause 6.4.1.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.4B.1.4_1	Frequency Error for Inter-band EN-DC including FR2 (>1 NR CC)

##### 6.4B.1.4_1.1	Frequency Error for Inter-band EN-DC including FR2 (2 NR CCs)

6.4B.1.4_1.1.1	Test purpose

Same test purpose as in clause 6.4.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.1.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.4B.1.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.1.4.

6.4B.1.4_1.1.4	Test description

6.4B.1.4_1.1.4.1	Initial condition

Same test description as in clause 6.4A.1.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.1.4_1.1.5	Test Requirements

Same test requirement as in clause 6.4A.1.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.1.4_1.2	Frequency Error for Inter-band EN-DC including FR2 (3 NR CCs)

6.4B.1.4_1.2.1	Test purpose

Same test purpose as in clause 6.4.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.1.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.4B.1.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.1.4.

6.4B.1.4_1.2.4	Test description

6.4B.1.4_1.2.4.1	Initial condition

Same test description as in clause 6.4A.1.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.1.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.1.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.1.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.1.4_1.2.5	Test Requirements

Same test requirement as in clause 6.4A.1.2.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.1.4_1.3	Frequency Error for Inter-band EN-DC including FR2 (4 NR CCs)

6.4B.1.4_1.3.1	Test purpose

Same test purpose as in clause 6.4.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.1.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.4B.1.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.1.4.

6.4B.1.4_1.3.4	Test description

6.4B.1.4_1.3.4.1	Initial condition

Same test description as in clause 6.4A.1.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.1.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.1.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.1.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.1.4_1.3.5	Test Requirements

Same test requirement as in clause 6.4A.1.3.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.1.4D	Frequency error for inter-band EN-DC including FR2 for UL-MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4D.1 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS and Annex F needs to be updated.

6.4B.1.4D.1	Test purpose

Same test purpose as in clause 6.4D.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.1.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 and UL MIMO.

6.4B.1.4D.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], 38.101-1 [2] and 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.1.4.

Same minimum conformance requirements as in clause 6.4D.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.1.4D.4	Test description

6.4B.1.4D.4.1	Initial conditions

Same test description as in clause 6.4D.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4D.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4D.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4D.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.1.4D.5	Test requirement

Same test requirement as in clause 6.4D.1.5 in TS 38.521-2 [9] for the NR carrier.

### 6.4B.2	Transmit Modulation Quality for DC

#### 6.4B.2.1	Transmit Modulation Quality for intra-band contiguous EN-DC

##### 6.4B.2.1.1	Error Vector Magnitude for intra-band contiguous EN-DC

6.4B.2.1.1.1	Test purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further equalised using the channel estimates subjected to the EVM equaliser spectrum flatness requirement specified in clause 6.4B.2.1.4.3. For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and the duration of PUCCH/PUSCH channel, or one hop, if frequency hopping is enabled for PUCCH and PUSCH in the time domain. The EVM measurement interval is reduced by any symbols that contains an allowable power transient as defined in clause 6.3.3.3 of TS 38.521-1 [8].

6.4B.2.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band EN-DC.

6.4B.2.1.1.3	Minimum conformance requirements

For the intra-band contiguous EN-DC with one component carrier per CG the EVM requirement applies with PRB allocation in one of the CG and the other CG unallocated.

The EVM requirements for each CG are according to clause 6.5.2 of TS 36.101 [5] for the MCG and clause 6.4.2 of TS 38.101-1 [2] for the SCG with EN-DC configured.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.1.1.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.1.1.4	Test description

Same test description as in clause 6.4.2.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Table 6.4B.2.1.1.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 6.4B.2.1.1.4-1.

For Initial conditions as in clause 6.4.2.1.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.1.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.4.2.1.4.2 in TS 38.521-1 [8].

For Message contents as in clause 6.4.2.1.4.3 in TS 38.521-1 [8], the following exception:

Table 6.4B.2.1.1.4-2: RACH-ConfigGeneric: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 14 | Unpaired Spectrum | PRACH Format 0 AND delay is given to SCG in Step 8 of Initial conditions |
|  | 12 | Unpaired Spectrum | PRACH Format 0 AND delay is given to MCG in Step 8 of Initial conditions |

6.4B.2.1.1.5	Test requirements

Same test requirement as in clause 6.4.2.1.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.4B.2.1.2	Carrier Leakage for intra-band contiguous EN-DC

6.4B.2.1.2.1	Test purpose

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency or centre frequency of aggregated transmission bandwidth configuration. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal. Carrier leakage interferes with the centre sub carriers of the UE under test (if allocated), especially, when their amplitude is small. The measurement interval is defined over one slot in the time domain.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

6.4B.2.1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band EN-DC.

6.4B.2.1.2.3	Minimum conformance requirements

The carrier leakage requirements for each CG are according to clause 6.5.2 of TS 36.101 [5] for the MCG and clause 6.4.2 of TS 38.101-1 [2] for the SCG with EN-DC configured.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.1.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.1.2.4	Test description

Same test description as in clause 6.4.2.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Table 6.4B.2.1.2.4-1: Test Configuration

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 6.4B.2.1.2.4-1For Initial conditions as in clause 6.4.2.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.2.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.4B.2.1.2.5	Test requirements

Same test requirement as in clause 6.4.2.2.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.4B.2.1.3	In-band Emissions for intra-band contiguous EN-DC

6.4B.2.1.3.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks.

The in-band emission is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is defined over one slot in the time domain, however, the minimum requirement applies when the in-band emission measurement is averaged over 10 sub-frames. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one or more symbols, accordingly.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of in-band emissions.

6.4B.2.1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.4B.2.1.3.3	Minimum conformance requirements

For the MCG the in-band emission requirements in Table 6.5.2A.3.1-1 and Table 6.5.2A.3.1-2 in TS 36.101 [5] apply within the aggregated transmission bandwidth configuration of the EN-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth  within the MCG at the edge of the said aggregated transmission bandwidth configuration.

For the SCG the in-band emission requirements in Table 6.5.2A.3.1-1 and Table 6.5.2A.3.1-2 in TS 36.101 [5] apply within the aggregated transmission bandwidth configuration of the EN-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth  within the SCG at the edge of the aggregated transmission bandwidth configuration.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.1.3

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.4B.2.1.3.4	Test description

6.4B.2.1.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on NR operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.4B.2.1.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521.1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.4B.2.1.3.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 2) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  | Lowest |  |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |
|  |  | Modulation |  | RB allocation (NOTE 5) | Modulation | RB allocation (NOTE 1) |
| 1 (NOTE3) | N/A for In-band emission test | QPSK |  | 0 | DFT-s-OFDM QPSK | Inner_1RB_Right |
| 2 (NOTE 4) |  | QPSK |  | 0 | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 3 (NOTE3) |  | QPSK |  | 0 | CP-OFDM QPSK | Inner_1RB_Right |
| 4 (NOTE 4) |  | QPSK |  | 0 | CP-OFDM QPSK | Inner_1RB_Left |
| 5 (NOTE3) |  | QPSK |  | Outer_1RB_Left | DFT-s-OFDM QPSK | 0 |
| 6 (NOTE 4) |  | QPSK |  | Outer_1RB_Right | DFT-s-OFDM QPSK | 0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7. 	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.4B.2.1.3.4.3.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.4B.2.1.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4B.2.1.3.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	For NR CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level 0dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

3.	Measure In-band emission on NR CC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on E-UTRA CC. For TDD slots with transient periods are not under test.

4.	For NR CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level -30dBm, where MU and Uplink power control window size are defined above.

5.	Measure In-band emission on NR CC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on E-UTRA CC. For TDD slots with transient periods are not under test

6.	For NR CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level -40dBm, where MU and Uplink power control window size are defined above.

7.	Measure In-band emission on NR CC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on E-UTRA CC. For TDD slots with transient periods are not under test.

8.	For E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level 0dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

9.	Measure In-band emission on E-UTRA CC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on NR CC. For TDD slots with transient periods are not under test.

10.	For E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level -30dBm, where MU and Uplink power control window size are defined above.

11.	Measure In-band emission on E-UTRA CC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on NR CC. For TDD slots with transient periods are not under test

12.	For E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level -40dBm, where MU and Uplink power control window size are defined above.

13.	Measure In-band emission on E-UTRA CC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on NR CC. For TDD slots with transient periods are not under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.4B.2.1.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE 2:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

Table 6.4B.2.1.3.4.2-1: Void

Table 6.4B.2.1.3.4.2-2: Void

6.4B.2.1.3.4.3	Message contents

Message contents are according to TS 36.508-1 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1.

6.4B.2.1.3.5	Test requirements

Each of the [20] In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.4B.2.1.3.5-1.

Table 6.4B.2.1.3.5-1: Test requirements for in-band emissions (allocated component carrier)

| Parameter | Unit | Limit |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | + TT |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 |  | Exception for IQ image (NOTE 3) |
| Carrier leakage | dBc | 25 + TT | Output power > 0 dBm | Exception for Carrier frequency (NOTE 4) |
|  |  | 20 + TT | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | 10 + TT | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. The limit is evaluated in each non-allocated RB. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs NOTE 3: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. The measurement bandwidth is 1 RB. NOTE 4: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in the non-allocated RB to the measured total power in all allocated RBs. NOTE 5: is the Transmission Bandwidth (see Figure 5.6-1) not exceeding NOTE 6: is the Transmission Bandwidth Configuration (see Figure 5.6-1) of the component carrier with RBs allocated. NOTE 7: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 8: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). NOTE 9: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. NOTE 10: Test tolerance TT = 0.8 dB. |  |  |  |  |

The in-band emissions results, measured with the spectral test shall not exceed the corresponding values in Table 6.4B.2.1.3.5-2.

Table 6.4B.2.1.3.5-2: Test requirements for in-band emissions (not allocated component carrier)

| Para-meter | Unit | Meas BW NOTE 1 | Limit |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB (180KHz rectangular) |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non-allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB (180KHz rectangular) | -25 + TT NOTE 2 |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of the contiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| Carrier leakage | dBc | BW of 1 RB (180KHz rectangular) | NOTE 3 |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -25 + TT | Output power > 0 dBm |  |  |
|  |  |  | -20 + TT | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -10 + TT | -40 dBm Output power < -30 dBm |  |  |
| NOTE 1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. NOTE 2: Exceptions to the general limit is are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. NOTE 3: Two Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs NOTE 4: NOTES 1, 5, 6, 7, 8, 9 from Table 6.5.2A.3.1-1 apply for Table 6.5.2A.3.1-2 as well. NOTE 5: for measured non-allocated RB in the non-allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 6: Test tolerance TT = 0.8 dB. |  |  |  |  |  |  |

##### 6.4B.2.1.4	EVM Equalizer Flatness for intra-band contiguous EN-DC

6.4B.2.1.4.1	Test purpose

Same test purpose as in clause 6.4.2.4 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.4B.2.1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.4.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.1.4.4	Test description

Same test description as in clause 6.4.2.4.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Table 6.4B.2.1.4.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4.2.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.4.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.4.2.4.4.2 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4B.2.1.4.5	Test requirement

Same test requirement as in clause 6.4.2.4.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.4B.2.2	Transmit Modulation Quality for intra-band non-contiguous EN-DC

##### 6.4B.2.2.1	Error Vector Magnitude for intra-band non-contiguous EN-DC

6.4B.2.2.1.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.4B.2.2.1.3	Minimum conformance requirements

For the intra-band non-contiguous EN-DC with one component carrier per CG the EVM requirement applies with PRB allocation in one of the CG and the other CG unallocated.

The EVM requirements for each CG are according to clause 6.5.2.1 of TS 36.101 [5] for the MCG and clause 6.4.2.1.3 of TS 38.521-1 [8] for the SCG with EN-DC configured.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.2.1.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.2.1.4	Test description

Same test description as in clause 6.4.2.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.For Initial conditions as in clause 6.4.2.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.1.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.4.2.1.4.2 in TS 38.521-1 [8].

For Message contents as in clause 6.4.2.1.4.3 in TS 38.521-1 [8], the following exception:

Table 6.4B.2.2.1.4-1: RACH-ConfigGeneric: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 14 | Unpaired Spectrum | PRACH Format 0 AND delay is given to SCG in Step 8 of Initial conditions |
|  | 12 | Unpaired Spectrum | PRACH Format 0 AND delay is given to MCG in Step 8 of Initial conditions |

6.4B.2.2.1.5	Test requirement

Same test requirement as in clause 6.4.2.1.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.4B.2.2.2	Carrier Leakage for intra-band non-contiguous EN-DC

6.4B.2.2.2.1	Test purpose

Same test purpose as in clause 6.4.2.2.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.4B.2.2.2.3	Minimum conformance requirements

The carrier leakage requirements for each CG are according to clause 6.5.2.2 of TS 36.101 [5] for the MCG and clause 6.4.2.2.3 of TS 38.521-1 [8] for the SCG with EN-DC configured and PRB allocation only in the CG being measured.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.2.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.2.2.4	Test description

Same test description as in clause 6.4.2.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4.2.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.2.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.4B.2.2.2.5	Test requirement

Same test requirement as in clause 6.4.2.2.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.4B.2.2.3	In-band Emissions for intra-band non-contiguous EN-DC

6.4B.2.2.3.1	Test purpose

Same test purpose as in clause 6.4.2.3.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.2.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.4B.2.2.3.3	Minimum conformance requirements

For the MCG the in-band emission requirements in Table 6.5.2A.3.1-1 and Table 6.5.2A.3.1-2 in TS 36.101 [5] apply within the transmission bandwidth configuration of the MCG with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth  within the MCG at the edge of the transmission bandwidth configuration.

For the SCG the in-band emission requirements in Table 6.4.2.3-1 TS 38.101-1 [2]] apply within the transmission bandwidth configuration of the SCG with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth  within the SCG at the edge of the transmission bandwidth configuration.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.2.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.2.3.4	Test description

Same test description as in clause 6.4.2.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Table 6.4B.2.2.3.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 | Low range, Mid range, High range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.3-1 | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested for Lowest NRB_agg and Highest NRB_agg. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 6.4B.2.2.3.4-1.

For Initial conditions as in clause 6.4.2.3.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.3.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.4B.2.2.3.5	Test requirements

Same test requirement as in clause 6.4.2.3.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.4B.2.2.4	EVM Equalizer Flatness for intra-band non-contiguous EN-DC

6.4B.2.2.4.1	Test purpose

Same test purpose as in clause 6.4.2.4 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.2.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.4B.2.2.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.4.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.2.4.4	Test description

Same test description as in clause 6.4.2.4.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

Table 6.4B.2.2.4.4-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  | MaxWGap |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 |  | Highest NRB_agg (NOTE 1, 2) |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for EVM equalizer flatness testing | Modulation | NR RB allocation (NOTE 3) |
| 1 |  | DFT-s-OFDM QPSK | Outer Full |
| 2 |  | CP-OFDM QPSK | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1 of 38.521-1 [8]. NOTE 2: Lowest and highest allowed NR channel BW as specified in Table 5.3B.1.3-1. If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1-1 of 38.521-1 [8]. |  |  |  |

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4.2.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.4.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.4.2.4.4.2 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4B.2.2.4.5	Test requirement

Same test requirement as in clause 6.4.2.4.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.4B.2.3	Transmit Modulation Quality for inter-band EN-DC within FR1

##### 6.4B.2.3.1	Error Vector Magnitude for inter-band EN-DC within FR1 (1 NR CC)

6.4B.2.3.1.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.4B.2.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.3.1.4	Test description

Same test description as in clause 6.4.2.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.For Initial conditions as in clause 6.4.2.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.1.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.4.2.1.4.2 in TS 38.521-1 [8].

6.4B.2.3.1.5	Test requirement

Same test requirement as in clause 6.4.2.1.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.4B.2.3.2	Carrier Leakage for inter-band EN-DC within FR1 (1 NR CC)

6.4B.2.3.2.1	Test purpose

Same test purpose as in clause 6.4.2.2.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.4B.2.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.3.2.4	Test description

Same test description as in clause 6.4.2.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4.2.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4B.2.3.2.5	Test requirement

Same test requirement as in clause 6.4.2.2.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.4B.2.3.3	In-band Emissions for inter-band EN-DC within FR1 (1 NR CC)

6.4B.2.3.3.1	Test purpose

Same test purpose as in clause 6.4.2.3.1 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.3.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.4B.2.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.3.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.3.3.4	Test description

Same test description as in clause 6.4.2.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.For Initial conditions as in clause 6.4.2.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.3.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.4.2.3.4.2 in TS 38.521-1 [8].

6.4B.2.3.3.5	Test requirement

Same test requirement as in clause 6.4.2.3.5 in TS 38.521-1 [8] for the NR carrier.

##### 6.4B.2.3.4	EVM Equalizer Flatness for inter-band EN-DC within FR1 (1 NR CC)

6.4B.2.3.4.1	Test purpose

Same test purpose as in clause 6.4.2.4 in TS 38.521-1 [8] for the NR carrier.

6.4B.2.3.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.4B.2.3.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.4.3 in TS 38.521-1 [8] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.3.4.4	Test description

6.4B.2.3.4.4.1	Initial conditions

Same test description as in clause 6.4.2.4.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4.2.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.4.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4.2.4.4.2 in TS 38.521-1 [8] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4B.2.3.4.5	Test requirement

Same test requirement as in clause 6.4.2.4.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.4B.2.3a	Transmit Modulation Quality for inter-band NE-DC within FR1

##### 6.4B.2.3a.1	Error Vector Magnitude for inter-band NE-DC within FR1

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested covered in 6.4.2.1 and 6.4A.2.1 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.5.2.1 and 6.5.2A.1 of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

##### 6.4B.2.3a.2	Carrier Leakage for inter-band NE-DC within FR1

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 6.4.2.2 and 6.4A.2.2 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.5.2.2 and 6.5.2A.2 of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

##### 6.4B.2.3a.3	In-band Emissions for inter-band NE-DC within FR1

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 6.4.2.3 and 6.4A.2.3 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.5.2 and 6.5.2A.3 of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

##### 6.4B.2.3a.4	EVM Equalizer Flatness for inter-band NE-DC within FR1 (1 NR CC)

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier in this test case are tested in 6.4.2.4 and 6.4.2.5 of TS 38.521-1 [8], and the requirements for LTE carrier in this test case are tested in 6.5.2.4 of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

#### 6.4B.2.4	Transmit Modulation Quality for inter-band EN-DC including FR2

##### 6.4B.2.4.1	Error Vector Magnitude for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined in the referred test case 6.4.2.1 in TS 38.521-2 [9]:

-	Measurement Uncertainty and Test Tolerance are FFS except for PUSCH, PC3 in FR2a and FR2b, and PC1 in FR2a.

6.4B.2.4.1.1	Test purpose

Same test purpose as in clause 6.4.2.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.4B.2.4.1.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], 38.101-1 [2] and 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

6.4B.2.4.1.4	Test description

6.4B.2.4.1.4.1	Initial conditions

Same test description as in clause 6.4.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4.2.1.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1.5	Test requirement

Same test requirement as in clause 6.4.2.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.1a	Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined in the referred test case 6.4.2.1_1 in TS 38.521-2 [9]:

-	Measurement Uncertainty and Test Tolerance are FFS except for PUSCH, PC3 in FR2a and FR2b.

6.4B.2.4.1a.1	Test purpose

Same test purpose as in clause 6.4.2.1_1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1a.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.4B.2.4.1a.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], 38.101-1 [2] and 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4.2.1_1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

6.4B.2.4.1a.4	Test description

Same test description as in clause 6.4.2.1_1.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4.2.1_1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.1_1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4.2.1_1.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1_2.5	Test requirement

Same test requirement as in clause 6.4.2.1_1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.1_1	Error Vector Magnitude for inter-band EN-DC including FR2 (>1 NR CC)

###### 6.4B.2.4.1_1.1	Error Vector Magnitude for inter-band EN-DC including FR2 (2 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.1.1 in TS 38.521-2 [9] is incomplete.

-	Test configuration table is FFS.

-	This test is incomplete due to lack of RRC framework for LO position retrieval

6.4B.2.4.1_1.1.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.4B.2.4.1_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.4B.2.4.1_1.1.4	Test description

6.4B.2.4.1_1.1.4.1	Initial condition

Same test description as in clause 6.4A.2.1.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1_1.1.5	Test Requirements

Same test requirement as in clause 6.4A.2.1.1.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.1_1.2	Error Vector Magnitude for inter-band EN-DC including FR2 (3 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.1.2 in TS 38.521-2 [9] is incomplete.

-	Test configuration table is FFS.

6.4B.2.4.1_1.2.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.4B.2.4.1_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.4B.2.4.1_1.2.4	Test description

6.4B.2.4.1_1.2.4.1	Initial condition

Same test description as in clause 6.4A.2.1.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.1.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.1.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.1.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1_1.2.5	Test Requirements

Same test requirement as in clause 6.4A.2.1.2.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.1_1.3	Error Vector Magnitude for inter-band EN-DC including FR2 (4 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.1.3 in TS 38.521-2 [9] is incomplete.

-	Test configuration table is FFS.

6.4B.2.4.1_1.3.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.4B.2.4.1_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.4B.2.4.1_1.3.4	Test description

6.4B.2.4.1_1.3.4.1	Initial condition

Same test description as in clause 6.4A.2.1.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.1.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.1.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.1.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1_1.3.5	Test Requirements

Same test requirement as in clause 6.4A.2.1.3.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.1_1.4	Error Vector Magnitude for inter-band EN-DC including FR2 (5 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.1_1.4.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 NR UL CCs.

6.4B.2.4.1_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.4B.2.4.1_1.4.4	Test description

6.4B.2.4.1_1.4.4.1	Initial condition

Same test description as in clause 6.4A.2.1.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.1.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.1.4.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.1.4.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1_1.4.5	Test Requirements

Same test requirement as in clause 6.4A.2.1.4.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.1_1.5	Error Vector Magnitude for inter-band EN-DC including FR2 (6 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.1_1.5.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 6 NR UL CCs.

6.4B.2.4.1_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.4B.2.4.1_1.5.4	Test description

6.4B.2.4.1_1.5.4.1	Initial condition

Same test description as in clause 6.4A.2.1.5.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.1.5.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.1.5.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.1.5.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1_1.5.5	Test Requirements

Same test requirement as in clause 6.4A.2.1.5.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.1_1.6	Error Vector Magnitude for inter-band EN-DC including FR2 (7 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.1_1.6.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 7 NR UL CCs.

6.4B.2.4.1_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.4B.2.4.1_1.6.4	Test description

6.4B.2.4.1_1.6.4.1	Initial condition

Same test description as in clause 6.4A.2.1.6.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.1.6.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.1.6.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.1.6.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1_1.6.5	Test Requirements

Same test requirement as in clause 6.4A.2.1.6.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.1_1.7	Error Vector Magnitude for inter-band EN-DC including FR2 (8 NR CCs)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.1_1.7.1	Test purpose

Same test purpose as in clause 6.4.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 8 NR UL CCs.

6.4B.2.4.1_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.4B.2.4.1_1.7.4	Test description

6.4B.2.4.1_1.7.4.1	Initial condition

Same test description as in clause 6.4A.2.1.7.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.1.7.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.1.7.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.1.7.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1_1.7.5	Test Requirements

Same test requirement as in clause 6.4A.2.1.7.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.1D	Error Vector Magnitude for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4D.2.1 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS and Annex F needs to be updated.

6.4B.2.4.1D.1	Test purpose

Same test purpose as in clause 6.4D.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.1D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 and UL MIMO.

6.4B.2.4.1D.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], 38.101-1 [2] and 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4D.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.4.1D.4	Test description

6.4B.2.4.1D.4.1	Initial conditions

Same test description as in clause 6.4D.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4D.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4D.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4D.2.1.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.1D.5	Test requirement

Same test requirement as in clause 6.4D.2.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.2	Carrier Leakage for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainty and Test Tolerance are FFS for PC1, PC2, and PC4.

-	The test case is incomplete for band n259.

6.4B.2.4.2.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.4B.2.4.2.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], 38.101-1 [2] and 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4.2.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied

6.4B.2.4.2.4	Test description

6.4B.2.4.2.4.1	Initial conditions

Same test description as in clause 6.4.2.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4.2.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4.2.2.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2.5	Test requirement

Same test requirement as in clause 6.4.2.2.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.2D	Carrier Leakage for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4D.2.2 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS and Annex F needs to be updated.

6.4B.2.4.2D.1	Test purpose

Same test purpose as in clause 6.4D.2.2.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 and UL MIMO.

6.4B.2.4.2D.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], 38.101-1 [2] and 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4D.2.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied

6.4B.2.4.2D.4	Test description

6.4B.2.4.2D.4.1	Initial conditions

Same test description as in clause 6.4D.2.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4D.2.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4D.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4D.2.2.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2D.5	Test requirement

Same test requirement as in clause 6.4D.2.2.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.2_1	Carrier Leakage for inter-band EN-DC including FR2 (>1 NR CC)

###### 6.4B.2.4.2_1.1	Carrier Leakage for inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.2.1 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

-	This test is incomplete due to lack of RRC framework for LO position retrieval.

6.4B.2.4.2_1.1.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR CCs.

6.4B.2.4.2_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.2.3.

6.4B.2.4.2_1.1.4	Test Description

6.4B.2.4.2_1.1.4.1	Initial conditions

Same test description as in clause 6.4A.2.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2_1.1.5	Test Requirement

Same test requirement as in clause 6.4A.2.2.1.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.2_1.2	Carrier Leakage for inter-band EN-DC including FR2 (3 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.2.2 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.2_1.2.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR CCs.

6.4B.2.4.2_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.2.3.

6.4B.2.4.2_1.2.4	Test Description

6.4B.2.4.2_1.2.4.1	Initial conditions

Same test description as in clause 6.4A.2.2.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.2.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.2.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.2.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2_1.2.5	Test Requirement

Same test requirement as in clause 6.4A.2.2.2.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.2_1.3	Carrier Leakage for inter-band EN-DC including FR2 (4 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.2.3 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.2_1.3.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.4B.2.4.2_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.2.3.

6.4B.2.4.2_1.3.4	Test Description

6.4B.2.4.2_1.3.4.1	Initial conditions

Same test description as in clause 6.4A.2.2.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.2.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.2.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.2.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2_1.3.5	Test Requirement

Same test requirement as in clause 6.4A.2.2.3.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.2_1.4	Carrier Leakage for inter-band EN-DC including FR2 (5 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.2.4 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.2_1.4.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 NR UL CCs.

6.4B.2.4.2_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.2.3.

6.4B.2.4.2_1.4.4	Test Description

6.4B.2.4.2_1.4.4.1	Initial conditions

Same test description as in clause 6.4A.2.2.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.2.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.2.4.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.2.4.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2_1.4.5	Test Requirement

Same test requirement as in clause 6.4A.2.2.4.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.2_1.5	Carrier Leakage for inter-band EN-DC including FR2 (6 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.2.5 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.2_1.5.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 6 NR UL CCs.

6.4B.2.4.2_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.2.3.

6.4B.2.4.2_1.5.4	Test Description

6.4B.2.4.2_1.5.4.1	Initial conditions

Same test description as in clause 6.4A.2.2.5.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.2.5.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.2.5.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.2.5.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2_1.5.5	Test Requirement

Same test requirement as in clause 6.4A.2.2.5.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.2_1.6	Carrier Leakage for inter-band EN-DC including FR2 (7 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.2.6 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.2_1.6.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 7 NR UL CCs.

6.4B.2.4.2_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.2.3.

6.4B.2.4.2_1.6.4	Test Description

6.4B.2.4.2_1.6.4.1	Initial conditions

Same test description as in clause 6.4A.2.2.6.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.2.6.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.2.6.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.2.6.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2_1.6.5	Test Requirement

Same test requirement as in clause 6.4A.2.2.6.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.2_1.7	Carrier Leakage for inter-band EN-DC including FR2 (8 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.2.7 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.2_1.7.1	Test purpose

Same test purpose as in clause 6.4.2.2 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.2_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 8 NR UL CCs.

6.4B.2.4.2_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.2.3.

6.4B.2.4.2_1.7.4	Test Description

6.4B.2.4.2_1.7.4.1	Initial conditions

Same test description as in clause 6.4A.2.2.7.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.4A.2.2.7.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.2.7.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.2.7.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.2_1.7.5	Test Requirement

Same test requirement as in clause 6.4A.2.2.7.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.3	In-band Emissions for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4.2.3 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

-	38.101-2 [3] clause 6.3.4.3: Relative power tolerances are in square brackets.

6.4B.2.4.3.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.4B.2.4.3.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], TS 38.101-1 [2] and TS 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.4.3.4	Test description

6.4B.2.4.3.4.1	Initial conditions

Same test description as in clause 6.4.2.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4.2.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4.2.3.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3.5	Test requirement

Same test requirement as in clause 6.4.2.3.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.3D	In-band Emissions for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4D.2.3 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS and Annex F needs to be updated.

6.4B.2.4.3D.1	Test purpose

Same test purpose as in clause 6.4D.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 and UL MIMO.

6.4B.2.4.3D.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], TS 38.101-1 [2] and TS 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4D.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.4.3D.4	Test description

6.4B.2.4.3D.4.1	Initial conditions

Same test description as in clause 6.4D.2.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4D.2.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4D.2.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4D.2.3.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3D.5	Test requirement

Same test requirement as in clause 6.4D.2.3.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.3_1	In-band Emissions for inter-band EN-DC including FR2 (>1 NR CC)

###### 6.4B.2.4.3_1.1	In-band Emissions for inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.3.1 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

-	This test is incomplete due to lack of RRC framework for LO position retrieval.

6.4B.2.4.3_1.1.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2NR UL CCs.

6.4B.2.4.3_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.3.3.

6.4B.2.4.3_1.1.4	Test description

6.4B.2.4.3_1.1.4.1	Initial conditions

Same test description as in clause 6.4A.2.3.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4A.2.3.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.3.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.3.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3_1.1.5	Test requirement

Same test requirement as in clause 6.4A.2.3.1.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.3_1.2	In-band Emissions for inter-band EN-DC including FR2 (3 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.3.2 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.3_1.2.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3NR UL CCs.

6.4B.2.4.3_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.3.3.

6.4B.2.4.3_1.2.4	Test description

6.4B.2.4.3_1.2.4.1	Initial conditions

Same test description as in clause 6.4A.2.3.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4A.2.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.3.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.3.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3_1.2.5	Test requirement

Same test requirement as in clause 6.4A.2.3.2.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.3_1.3	In-band Emissions for inter-band EN-DC including FR2 (4 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.3.3 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.3_1.3.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.4B.2.4.3_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.3.3.

6.4B.2.4.3_1.3.4	Test description

6.4B.2.4.3_1.3.4.1	Initial conditions

Same test description as in clause 6.4A.2.3.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4A.2.3.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.3.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.3.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3_1.3.5	Test requirement

Same test requirement as in clause 6.4A.2.3.3.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.3_1.4	In-band Emissions for inter-band EN-DC including FR2 (5 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.3.4 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.3_1.4.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 NR UL CCs.

6.4B.2.4.3_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.3.3.

6.4B.2.4.3_1.4.4	Test description

6.4B.2.4.3_1.4.4.1	Initial conditions

Same test description as in clause 6.4A.2.3.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4A.2.3.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.3.4.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.3.4.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3_1.4.5	Test requirement

Same test requirement as in clause 6.4A.2.3.4.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.3_1.5	In-band Emissions for inter-band EN-DC including FR2 (6 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.3.5 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.3_1.5.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 6 NR UL CCs.

6.4B.2.4.3_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.3.5.

6.4B.2.4.3_1.5.4	Test description

6.4B.2.4.3_1.5.4.1	Initial conditions

Same test description as in clause 6.4A.2.3.5.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4A.2.3.5.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.3.5.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.3.5.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3_1.5.5	Test requirement

Same test requirement as in clause 6.4A.2.3.5.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.3_1.6	In-band Emissions for inter-band EN-DC including FR2 (7 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.3.6 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.3_1.6.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 7 NR UL CCs.

6.4B.2.4.3_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.3.3.

6.4B.2.4.3_1.6.4	Test description

6.4B.2.4.3_1.6.4.1	Initial conditions

Same test description as in clause 6.4A.2.3.6.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4A.2.3.6.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.3.6.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.3.6.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3_1.6.5	Test requirement

Same test requirement as in clause 6.4A.2.3.6.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.4B.2.4.3_1.7	In-band Emissions for inter-band EN-DC including FR2 (8 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4A.2.3.7 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.3_1.7.1	Test purpose

Same test purpose as in clause 6.4.2.3 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.3_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 8 NR UL CCs.

6.4B.2.4.3_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4B.2.4.3.3.

6.4B.2.4.3_1.7.4	Test description

6.4B.2.4.3_1.7.4.1	Initial conditions

Same test description as in clause 6.4A.2.3.7.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4A.2.3.7.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4A.2.3.7.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4A.2.3.7.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.3_1.7.5	Test requirement

Same test requirement as in clause 6.4A.2.3.7.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.4	EVM Equalizer Flatness for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4.2.4 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

6.4B.2.4.4.1	Test purpose

Same test purpose as in clause 6.4.2.4 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with1 NR UL CC.

6.4B.2.4.4.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], TS 38.101-1 [2] and TS 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4.2.4.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.4.4.4	Test description

6.4B.2.4.4.4.1	Initial conditions

Same test description as in clause 6.4.2.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4.2.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4.2.4.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.4.5	Test requirement

Same test requirement as in clause 6.4.2.4.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.4D	EVM Equalizer Flatness for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4D.2.4 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS and Annex F needs to be updated.

6.4B.2.4.4D.1	Test purpose

Same test purpose as in clause 6.4D.2.4.1 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 and UL MIMO.

6.4B.2.4.4D.3	Minimum conformance requirements

For inter-band EN-DC including FR2 or both FR1 and FR2, transmit modulation quality for EN-DC operations in FR1 and FR2 as specified in TS 36.101 [5], TS 38.101-1 [2] and TS 38.101-2 [3] apply for E-UTRA, NR FR1 and NR FR2 respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4D.2.4.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.4.4D.4	Test description

6.4B.2.4.4D.4.1	Initial conditions

Same test description as in clause 6.4D.2.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4D.2.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4D.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4D.2.4.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.4D.5	Test requirement

Same test requirement as in clause 6.4D.2.4.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.5	EVM spectral flatness for pi/2 BPSK modulation for inter-band EN-DC including FR2 (1 NR CC)

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The referred test case 6.4.2.5 in TS 38.521-2 [9] is incomplete.

6.4B.2.4.5.1	Test purpose

Same test purpose as in clause 6.4.2.5 in TS 38.521-2 [9] for the NR carrier.

6.4B.2.4.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with1 NR UL CC.

6.4B.2.4.5.3	Minimum conformance requirements

Transmit modulation quality requirement for E-UTRA single carrier and CA operation specified in clauses 6.5.2 and 6.5.2A of TS 36.101 [4] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.4.2, 6.4A.2 and 6.4D.2 of TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

Same minimum conformance requirements as in clause 6.4.2.5.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.4B.2.4.5.4	Test description

6.4B.2.4.5.4.1	Initial conditions

Same test description as in clause 6.4.2.5.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.4.2.5.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.2.4.5.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.4.2.5.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.4B.2.4.5.5	Test requirement

Same test requirement as in clause 6.4.2.5.5 in TS 38.521-2 [9] for the NR carrier.

## 6.4E	Transmit signal quality for V2X operation in FR1

### 6.4E.1	Frequency error for V2X

#### 6.4E.1.0	Minimum conformance requirements

For intra-band V2X operating UE, the requirement shall apply on each component carrier as defined in clause 6.5.1G in TS 36.101 [5] and in clause 6.4E.1 in TS 38.101-1 [2], respectively.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.4.1 of TS 36.101 [5] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.4E.1 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4E.1.

#### 6.4E.1.1	Frequency error for Intra-band V2X

6.4E.1.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4E.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.4E.1.0. The requirements in this test case can be well covered in clause 6.5.1G of TS 36.521-1 [10] and clause 6.4E.1 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.4E.1.2	Frequency error for Inter-band V2X

6.4E.1.2.1	Test purpose

Same test purpose as in clause 6.4E.1 in TS 38.521-1 [8].

6.4E.1.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.5.1 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.4E.1.1 of TS 38.521-1 [8] and don’t need to be tested again.

### 6.4E.2	Transmit modulation quality for V2X

#### 6.4E.2.1	Error Vector Magnitude for V2X

6.4E.2.1.0	Minimum conformance requirements

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.1 in TS 36.101 [5] and in clause 6.4E.2.1 in TS 38.101-1 [2], respectively.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.5.2 of TS 36.101 [5] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.4E.2.1 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4E.2.1.1.

##### 6.4E.2.1.1	Error Vector Magnitude for intra-band V2X

6.4E.2.1.1.1	Test purpose

Same test purpose as in clause 6.4E.2 in TS 38.521-1 [8].

6.4E.2.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.4E.2.1.0. The requirements in this test case can be well covered in clause 6.5.2.1G of TS 36.521-1 [10] and clause 6.4E.2.1 of TS 38.521-1 [8] and don’t need to be tested again.

##### 6.4E.2.1.2	Error Vector Magnitude for Inter-band V2X

6.4E.2.1.2.1	Test purpose

Same test purpose as in clause 6.4E.2 in TS 38.521-1 [8].

6.4E.2.1.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.5.2.1 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.4E.2.2.1 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.4E.2.2	Carrier leakage for V2X

6.4E.2.2.0	Minimum conformance requirements

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.2 in TS 36.101 [5] and in clause 6.4E.2.2 in TS 38.101-1 [2], respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4E.2.2.2.

##### 6.4E.2.2.1	Carrier leakage for intra-band V2X

6.4E.2.2.1.1	Test purpose

Same test purpose as in clause 6.4E.2.3.1 in TS 38.521-1 [8].

6.4E.2.2.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.0. The requirements in this test case can be well covered in clause 6.5.2.2G of TS 36.521-1 [10] and clause 6.4E.2.3 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.4E.2.3	In-band emissions for V2X

6.4E.2.3.0	Minimum conformance requirements

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.3 in TS 36.101 [5] and in clause 6.4E.2.3 in TS 38.101-1 [2], respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4E.2.2.3.

##### 6.4E.2.3.1	In-band emissions for intra-band V2X

6.4E.2.3.1.1	Test purpose

Same test purpose as in clause 6.4E.2.4.1 in TS 38.521-1 [8].

6.4E.2.3.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.0. The requirements in this test case can be well covered in clause 6.5.2.3G of TS 36.521-1 [10] and clause 6.4E.2.4 of TS 38.521-1 [8] and don’t need to be tested again.

## 6.5	Output RF spectrum emissions

## 6.5A	Output RF spectrum emissions for CA

### 6.5A.1	Occupied bandwidth for CA without EN-DC

6.5A.1.1	Test purpose

Same test purpose as in clause 6.5.1 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.5.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.5A.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for occupied bandwidth apply and are tested in TS 38.521-1 [8] clauses 6.5 and 6.5A and TS 38.521-2 [9] clauses 6.5 and 6.5A.

### 6.5A.2	Out-of-band emissions for CA without EN-DC

#### 6.5A.2.1	Spectrum emissions mask for CA without EN-DC

6.5A.2.1.1	Test purpose

Same test purpose as in clause 6.5.2.2 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.5.2.1 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.5A.2.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for additional spectrum emissions mask apply and are tested in TS 38.521-1 [8] clauses 6.5 and 6.5A and TS 38.521-2 [9] clauses 6.5 and 6.5A.

#### 6.5A.2.2	Additional Spectrum emissions mask for CA without EN-DC

6.5A.2.2.1	Test purpose

Same test purpose as in clause 6.5.2.3 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.5.2.2 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.5A.2.2.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for additional spectrum emissions mask apply and are tested in TS 38.521-1 [8] clauses 6.5 and 6.5A and TS 38.521-2 [9] clauses 6.5 and 6.5A.

#### 6.5A.2.3	Adjacent channel leakage ratio for CA without EN-DC

No test case details specified as there are no exception requirements applicable to NR FR1 or NR FR2 as per TS 38.101-3 [4], clause 6.5A.2. The NR/5GC requirement for ACLR applies and is tested in TS 38.521-1 [8] and TS 38.521-2 [9] for FR1 and FR2 respectively.

### 6.5A.3	Spurious emissions for CA without EN-DC

#### 6.5A.3.1	Inter-band CA between FR1 and FR2

6.5A.3.1.1	Test purpose

Same test purpose as in clause 6.5.3 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.5.3 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.5A.3.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for spurious emission for inter-band CA between FR1 and FR2 and UE co-existence requirements apply for each component carrier and are tested in TS 38.521-1 [8] clauses 6.5 and 6.5A and TS 38.521-2 [9] clauses 6.5 and 6.5A.

## 6.5B	Output RF spectrum emissions for DC

### 6.5B.1	Occupied bandwidth for EN-DC

#### 6.5B.1.1	Occupied bandwidth for Intra-Band Contiguous EN-DC

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

- measurement uncertainty for ENBW > 100 MHz is FFS.

6.5B.1.1.1	Test purpose

To verify that the UE occupied bandwidth for intra-band contiguous EN-DC for all transmission bandwidth configurations supported by the UE are less than their specific limits.

6.5B.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.5B.1.1.3	Minimum conformance requirements

For intra-band contiguous EN-DC, the occupied bandwidth is a measure of the bandwidth containing the 99% of the total integrated power of the transmitted spectrum. The OBW shall be less than the aggregated channel bandwidth for EN-DC, denoted as EN-BW in clause 5.3B.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.5B.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.5B.1.1.4	Test description

6.5B.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.5B.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.1.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  | All |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  | Lowest SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for OBW testing. | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 6.5B.1.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

3.	Measure the power spectrum distribution over all EN-DC component carriers in the EN-DC within two times or more range over the requirement for Occupied Bandwidth specification for intra-band contiguous EN-DC centring on the current carrier frequency in the EN-DC configuration. The characteristics of the filter shall be approximately Gaussian (typical spectrum analyser filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is at least 1ms over consecutive active uplink slots.

4.	Calculate the total power within the range of all frequencies measured in step 3 and save this value as "Total power".

5.	Sum up the power upward from the lower boundary of the measured frequency range in step 3 and seek the limit frequency point by which this sum becomes 0.5% of "Total power" and save this point as "Lower Frequency".

6.	Sum up the power downward from the upper boundary of the measured frequency range in step 3 and seek the limit frequency point by which this sum becomes 0.5% of "Total power" and save this point as "Upper Frequency".

7.	Calculate the difference "Upper Frequency" – "Lower Frequency" = "Occupied Bandwidth" between the two limit frequencies obtained in step 5 and step 6.

6.5B.1.1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.1.1.4.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.1.1.4.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.1.1.4.3-4: SystemInfomationBlockType1: tdd-Config if operating on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.5B.1.1.5	Test requirements

The measured Occupied Bandwidth shall not exceed values of aggregated channel bandwidth as defined in clause 5.3B.1.2 for intra-band contiguous EN-DC.

#### 6.5B.1.2	Occupied bandwidth for Intra-Band Non-Contiguous EN-DC

6.5B.1.2.1	Test purpose

Same test purpose as in clause 6.5.1.1 in TS 38.521-1 [8] for the NR carrier.

6.5B.1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.1.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.1.2.4	Test description

Same test description as in clause 6.5.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 6.5B.1.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1.4.3 for cases with single E-UTRA carrier | Low with maxWgap (NR low – E-UTRA high) |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] subclause 4.3.1.4.3 | All for NR; Lowest for E-UTRA |

For Initial conditions as in clause 6.5.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.1.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.5.1.4.2 in TS 38.521-1 [8].

6.5B.1.2.5	Test requirement

Same test requirement as in clause 6.5.1.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.5B.1.3	Occupied bandwidth for Inter-Band EN-DC within FR1 (1 NR CC)

6.5B.1.3.1	Test purpose

Same test purpose as in clause 6.5.1.1 in TS 38.521-1 [8] for the NR carrier.

6.5B.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.5B.1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.1.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.1.3.4	Test description

Same test description as in clause 6.5.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 6.5.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.1.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.1.3.5	Test requirement

Same test requirement as in clause 6.5.1.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.5B.1.4	Occupied bandwidth for Inter-Band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5.1 in TS 38.521-2 [9] is incomplete for some scenarios (band, bandwidth, power class etc…) as indicated in its editor’s note.

6.5B.1.4.1	Test purpose

Same test purpose as in clause 6.5.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.1.4.4	Test description

6.5B.1.4.4.1	Initial conditions

Same test description as in clause 6.5.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.5.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.1.4.5	Test requirement

Same test requirement as in clause 6.5.1.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.5B.1.4_1	Occupied bandwidth for Inter-band EN-DC including FR2 (>1 NR CC)

Editor's note: Test applicability, Test Description in below sub-clauses requires updates to clarify number of E-UTRA carriers that will be configured during the test that will be limited to only 1 E-UTRA CC

##### 6.5B.1.4_1.1	Occupied bandwidth for Inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.1.1 in TS 38.521-2 [9] is incomplete for some scenarios (band, aggregated bandwidth, CA configuration, power class etc…) as indicated in its editor’s note.

6.5B.1.4_1.1.1	Test purpose

Same test purpose as in clause 6.5.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.1.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.1.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.1.

6.5B.1.4_1.1.4	Test description

6.5B.1.4_1.1.4.1	Initial condition

Same test description as in clause 6.5A.1.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.1.4_1.1.5	Test Requirements

Same test requirement as in clause 6.5A.1.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.1.4_1.2	Occupied bandwidth for Inter-band EN-DC including FR2 (3 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.1.2 in TS 38.521-2 [9] is incomplete for some scenarios (band, aggregated bandwidth, CA configuration, power class etc…) as indicated in its editor’s note.

6.5B.1.4_1.2.1	Test purpose

Same test purpose as in clause 6.5.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.1.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.5B.1.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.1.

6.5B.1.4_1.2.4	Test description

6.5B.1.4_1.2.4.1	Initial condition

Same test description as in clause 6.5A.1.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.1.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.1.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.1.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.1.4_1.2.5	Test Requirements

Same test requirement as in clause 6.5A.1.2.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.1.4_1.3	Occupied bandwidth for Inter-band EN-DC including FR2 (4 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.1.1 in TS 38.521-2 [9] is incomplete for some scenarios (band, aggregated bandwidth, CA configuration, power class etc…) as indicated in its editor’s note.

6.5B.1.4_1.3.1	Test purpose

Same test purpose as in clause 6.5.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.1.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.5B.1.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.1.

6.5B.1.4_1.3.4	Test description

6.5B.1.4_1.3.4.1	Initial condition

Same test description as in clause 6.5A.1.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.1.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.1.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.1.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.1.4_1.3.5	Test Requirements

Same test requirement as in clause 6.5A.1.3.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.5B.1.4D	Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5D.1 in TS 38.521-2 [9] is incomplete

6.5B.1.4D.1	Test purpose

Same test purpose as in clause 6.5D.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.1.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 UL CCs.

6.5B.1.4D.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5D.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.1.4D.4	Test description

6.5B.1.4D.4.1	Initial conditions

Same test description as in clause 6.5D.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.5D.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5D.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5D.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.1.4D.5	Test requirement

Same test requirement as in clause 6.5D.1.5 in TS 38.521-2 [9] for the NR carrier.

### 6.5B.2	Out-of-band emissions for EN-DC

#### 6.5B.2.1	Out-of-band emissions for Intra-band contiguous EN-DC

##### 6.5B.2.1.1	Spectrum emissions mask for intra-band contiguous EN-DC

6.5B.2.1.1.1	Test purpose

To verify that the power of any UE emissions shall not exceed specified level for the specified aggregated bandwidth for the EN-DC intra-band contiguous.

6.5B.2.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.5B.2.1.1.3	Minimum conformance requirements

The general spectrum emission for intra-band contiguous EN-DC is specified in Table 6.5B.2.1.1.3-1.

Table 6.5B.2.1.1.3-1: General spectrum emission mask for intra-band contiguous EN-DC

| ΔfOOB (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0 – 1 | Max(Round(10*log(0.15/ENBW)),-24) | 30 kHz |
| ± 1 – 5 | -10 | 1 MHz |
| ± 5 – ENBW | -13 | 1 MHz |
| ± ENBW – (ENBW+5) | -25 | 1 MHz |
| NOTE: ENBW refers to the aggregated channel bandwidth in MHz as defined in clause 5.3B. |  |  |

The normative reference for this measurement is TS 38.101-3 [4] clause 6.5B.2.1.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.5B.2.1.1.4	Test description

6.5B.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.5B.2.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.2.1.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  | Low range, High range |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  | Lowest, Highest (NOTE 10) |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (NOTE 1) | Power config (NOTE 8) |
| 1 | Default | Default | Default | N/A | 16QAM | Outer_Full | DFT-s-OFDM Pi/2 BPSK | Outer_Full | B |
| 2 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | B |
| 3 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 4 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | B |
| 6 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 8 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 9 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 10 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 11 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 12 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 13 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 14 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 15 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 16QAM | Outer_Full | B |
| 16 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | Edge_1RB_Right | B |
| 17 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | N/A | A |
| 18 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Right | A |
| 19 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | Edge_1RB_Left | B |
| 20 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Left | A |
| 21 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | N/A | A |
| 22 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 64QAM | Outer_Full | B |
| 23 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 64QAM | Edge_1RB_Right | B |
| 24 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 64QAM | Edge_1RB_Left | B |
| 25 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 256QAM | Outer_Full | B |
| 26 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 256QAM | Edge_1RB_Right | B |
| 27 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 256QAM | Edge_1RB_Left | B |
| 28 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM QPSK | Outer_Full | B |
| 29 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right | B |
| 30 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | N/A | A |
| 31 (Note 3) | High |  |  |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Right | A |
| 32 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Left | B |
| 33 (Note 4) | Low |  |  |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Left | A |
| 34 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | N/A | A |
| 35 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 16QAM | Outer_Full | B |
| 36 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | Edge_1RB_Right | B |
| 37 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | N/A | A |
| 38 (Note 3) | High |  |  |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Right | A |
| 39 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | Edge_1RB_Left | B |
| 40 (Note 4) | Low |  |  |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Left | A |
| 41 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | N/A | A |
| 42 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 64QAM | Outer_Full | B |
| 43 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 64QAM | Edge_1RB_Right | B |
| 44 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 64QAM | Edge_1RB_Left | B |
| 45 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 46 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 47 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 48 (Note 4) | Default |  |  |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. Edge_Full_Right is defined as 2 RBs allocated at the right edge of the E-UTRA component. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1 NOTE 7: Power config as specified in Table 6.5B.2.1.2.4.3-3 (PC3) or 6.5B.2.1.2.4.3-4 (PC2). NOTE 8: All test points in this table must also exist in table 6.2B.2.1.4.1-1 (MPR). NOTE 9: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. NOTE 10: For DC_(n)71AA, only NR SCS of 15 kHz is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.2.1.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.2.1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.1.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

3.	For a UE supporting dynamic power sharing, measure the mean power over all component carriers. For a UE not supporting dynamic power sharing, measure the power of each component carrier individually. The measure transmitted power shall meet the requirements in clause 6.2B.2.1.5. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.2.1.1.5-1 and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.5B.2.1.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5B.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 with the following exceptions:

Table 6.5B.2.1.1.4.3-1: AdditionalSpectrumEmission for MCG

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 (NS_01) |  |  |

Table 6.5B.2.1.1.4.3-2: AdditionalSpectrumEmission for SCG

| Derivation Path: 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 (NS_01) |  |  |

Table 6.5B.2.1.1.4.3-3: PhysicalCellGroupConfig for PC3

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power config A (NOTE 1) |
|  | 20 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.1.4.3-4: RRCConnectionReconfiguration: nr-Config-r15 for PC3

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power config A (NOTE 1) |
|  | 20 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.1.4.3-5: PhysicalCellGroupConfig for PC2

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 26 |  | Power config A (NOTE 1) |
|  | 23 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.1.4.3-6: RRCConnectionReconfiguration: nr-Config-r15 for PC2

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 26 |  | Power config A (NOTE 1) |
|  | 23 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.1.4.3-7: RRCConnectionReconfiguration: tdm-PatternConfig if operating on FDD band

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 ::= CHOICE{ |  |  | Power config A (NOTE 1) |
| setup :: = SEQUENCE { |  | Apply if operating on FDD band for a UE NOT indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE according to TS 38.213 [x] clause 7.6.1 |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.1.4.3-8: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.5B.2.1.1.5	Test requirements

The power of any UE emissions shall fulfil requirements in Table 6.5B.2.1.1.5-1.

Table 6.5B.2.1.1.5-1: General spectrum emission mask for intra-band contiguous EN-DC

| ΔfOOB (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0 - 1 | Max(Round(10*log(0.15/ENBW)),-24) | 30 kHz |
| ± 1 - 5 | -10 + TT | 1 MHz |
| ± 5 - ENBW | -13 + TT | 1 MHz |
| ± ENBW – (ENBW+5) | -25 + TT | 1 MHz |
| NOTE: ENBW refers to the aggregated channel bandwidth in MHz as defined in clause 5.3B. |  |  |

Table 6.5B.2.1.1.5-2: Test Tolerance (Spectrum Emission Mask)

| f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| 1.5 dB | 1.8 dB | 1.8 dB |

##### 6.5B.2.1.2	Additional spectrum emissions mask for intra-band contiguous EN-DC

6.5B.2.1.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5B.2.1.2.2	Test applicability

This test case applies to all types of E-UTRA power class 3 and power class 2 UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.5B.2.1.2.3	Minimum conformance requirements

6.5B.2.1.2.3.1	Minimum requirement for network signalled value "NS_35"

For contiguous intra-band EN-DC configuration of DC_(n)71AA when NS_35 is indicated for the UE the requirements in table 6.5B.2.1.2.3-1 apply in the frequency ranges immediately adjacent and outside the aggregation of the said sub-blocks

When NS_35 is indicated in the MCG and NS_35 is indicated in the SCG the requirements in table 6.5B.2.1.2.3.1-1 apply in the frequency ranges immediately adjacent and outside the aggregated sub-blocks of the EN-DC configuration for DC_(n)71AA.

Table 6.5B.2.1.2.3.1-1: Additional requirements

|  | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW-0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW +5 MHz | ENBW + 0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.1.2.1.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA overlap in time with NR.LTE and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that doesn't support dynamic power sharing. LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

No exception requirements for NR or E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that supports dynamic power sharing. LTE anchor agnostic approach is not applied for this case.

6.5B.2.1.2.3.2	Minimum requirement for network signalled value "NS_04"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The Band 41/n41 SEM transition point from -13 dBm/MHz to -25 dBm/MHz is based on the emission bandwidth. The emission bandwidth is defined as the width of the signal between two points, one below the carrier centre frequency and one above the carrier centre frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power. Since the 26 dB emission bandwidth is implementation dependent, the transmission bandwidths occupied by RBs is used for the SEM. The emission bandwidth for LTE carriers is document in TS 36.101 [5], and the emission bandwidth for NR carriers is documented in TS 38.101-1 [2]. The total emission bandwidth for contiguous intra-band EN-DC is the sum of the emission bandwidth for each CC plus the guard band between contiguous CCs.

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.2.1.2.3.2-1.

Table 6.5B.2.1.2.3.2-1: n41 SEM with NS_04

|  | Spectrum emission limit (dBm)/ measurement bandwidth for each channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | > 50 MHz | Measurement bandwidth |
| ± 0 - 1 | -18 | -20 | -21 | -24 | -25 |  | 30 kHz |
| ± 1 - 5 | -10 |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 |  |  |  |  |  |  |
| NOTE 1: X is defined as the sum of the emission bandwidth of the component carriers plus the guard band between contiguous CCs. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.1.2.2.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.5B.2.1.2.4	Test description

6.5B.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in test configuration table 6.2B.3.1.4.1-1 through 6.2B.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.2.1.2.4.1-0: E-UTRA test configuration table

| E-UTRA Test Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency (Note 1) | Downlink | Uplink |  |
|  |  | N/A for A-MPR testing. | Modulation | RB allocation |
| 20 MHz | Low range and High range (Note 2) |  | QPSK | 100 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [6] clause 4.3.1 NOTE 2: NR carrier shall be the outermost carrier during test. |  |  |  |  |

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.2.1 for SS diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for E-UTRA CG link and NR CG link respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG link and NR CG link respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG , Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.2.1.2.4.3.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.2.1.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 6.2B.3.1.4.1-1 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers for the EN-DC configuration, which shall meet the requirements described in table 6.5B.2.1.2.5.1-1 through to 6.5B.2.1.2.5.2-1. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms).

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5B.2.1.2.5-1 through to 6.5B.2.1.2.5.2-1 and using a rms detector. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active time slots.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2B.3.1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5B.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1, with the following exceptions.

Table 6.5B.2.1.2.4.3-1: PhysicalCellGroupConfig for PC3

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power config A (NOTE 1) |
|  | 20 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.2.4.3-1: RRCConnectionReconfiguration: nr-Config-r15 for PC3

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power config A (NOTE 1) |
|  | 20 |  | Power config B (NOTE 2) |
| P-Max | 23 |  |  |
|  | 20 |  |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.2.4.3-3: PhysicalCellGroupConfig for PC2

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 26 |  | Power config A (NOTE 1) |
|  | 23 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.2.4.3-4: RRCConnectionReconfiguration: nr-Config-r15 for PC2

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 26 |  | Power config A (NOTE 1) |
|  | 23 |  | Power config B (NOTE 2) |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.5B.2.1.2.4.3-5: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.5B.2.1.2.4.3.1	Message contents exceptions for network signalled value "NS_35"

For "NS_35" see A-MPR test case in table 6.2B.3.1.4.3.2-1 and table 6.2B.3.1.4.3.2-2.

6.5B.2.1.2.4.3.2	Message contents exceptions for network signalled value "NS_04"

For "NS_04" see A-MPR test case in table 6.2B.3.1.4.3.1-1 and table 6.2B.3.1.4.3.1-2.

6.5B.2.1.2.5	Test requirement

6.5B.2.1.2.5-1: Test Tolerance (Additional Spectrum Emission Mask)

| f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| 1.5 dB | 1.8 dB | 1.8 dB |

6.5B.2.1.2.5.1	Test requirement for network signalled value "NS_35"

When "NS_35" is indicated in the cell measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in table 6.2B.3.1.5.1-1, and the power of any UE shall not exceed the described values in table 6.5B.2.1.2.5.1-1. The requirements in the table apply in the frequency ranges immediately adjacent and outside the aggregation of the sub-blocks.

Table 6.5B.2.1.2.5.1-1: Additional requirements for "NS_35"

| ΔfOOB | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13+TT | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13+TT | 100 kHz |
| ENBW f < ENBW +5 MHz | ENBW + 0.5 MHz f_offset < ENBW + 4.5 MHz | -25+TT | 1 MHz |
| NOTE: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

6.5B.2.1.2.5.2	Test requirement for network signalled value "NS_04"

When "NS_04" is indicated in the cell measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in tables 6.2B.3.1.5.2-1, and the power of any UE shall not exceed the described values in table 6.5B.2.1.2.5.2-1. The requirements in the table apply in the frequency ranges immediately adjacent and outside the aggregation of the sub-blocks.

Table 6.5B.2.1.2.5.2-1: Additional requirements for n41 SEM with NS_04

|  | Spectrum emission limit (dBm)/ measurement bandwidth for each channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | > 50 MHz | Measurement bandwidth |
| ± 0 - 1 | -18+TT | -20+TT | -21+TT | -24+TT | -25+TT |  | 30 kHz |
| ± 1 - 5 | -10+TT |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13+TT |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25+TT |  |  |  |  |  |  |
| NOTE 1: X is defined as the sum of the emission bandwidth of the component carriers plus the guard band between contiguous CCs. |  |  |  |  |  |  |  |

##### 6.5B.2.1.3	Adjacent channel leakage ratio for intra-band contiguous EN-DC

6.5B.2.1.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage Power Ratio (ACLR).

6.5B.2.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.5B.2.1.3.3	Minimum conformance requirements

For EN-DC operation with an E-UTRA sub-block immediately adjacent to an NR sub-block, the ACLR is defined as the ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW at nominal channel spacing. The UE shall meet the ACLR minimum requirement EN-DCACLR specified in Table 6.5B.2.1.3.3-1 with ENBW the sum of the sub-block bandwidths.

The assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in 6.5B.2.1.3.3-1.

Table 6.5B.2.1.3.3-1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR for PC3 | dBc | 30 |
| EN-DCACLR for PC2 | dBc | 31 |
| Measurement bandwidth of EN-DC channel |  | 1.00*ENBW |
| Measurement bandwidth of adjacent channel |  | 0.95*ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz as defined in clause 5.3B. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.1.3.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.5B.2.1.3.4	Test description

6.5B.2.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in test configuration tables defined in section 6.2B.2.1.4.1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.2.1.3.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.2.1.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to test configuration tables defined in section 6.2B.2.1.4.1 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements in clause 6.2B.2.1.5 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the filtered mean power of the transmitted signal centred on the aggregated sub-block ENBW with a measurement filter of bandwidth according to test configuration tables defined in section 6.2B.2.1.4.1 and using a rms detector. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots For TDD, only slots consisting of only UL symbols are under test.

5.	Measure the filtered mean power of the first adjacent channel on both lower and upper side of the assigned NR + E-UTRA channel, respectively with a frequency offset and measurement filter of bandwidth according to test configuration tables defined in section 6.2B.2.1.4.1 and using a rms detector.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper side respectively.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables defined in section6.2B.2.1.4.1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5B.2.1.3.4.3	Message contents

Same message contents as in clause 6.2B.2.1.4.3.

6.5B.2.1.3.5	Test requirement

The measured adjacent channel power ratio, derived in step 6, shall be higher than the limits in Table 6.5B.2.1.3.5-1.

Table 6.5B.2.1.3.5-1: ACLR requirement for intra-band EN-DC (contiguous sub-blocks)

|  | Power class 2 | Power class 3 |
| --- | --- | --- |
| NR ACLR | 31 - TT dBc | 30 - TT dBc |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5B.2.1.3.5-2. |  |  |

Table 6.5B.2.1.3.5-2: Test Tolerance

|  | f ≤ 4.0GHz | 4.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 100MHz | 0.8 dB | 1.0 dB |

#### 6.5B.2.2	Out-of-band emissions for Intra-band non-contiguous EN-DC

##### 6.5B.2.2.1	Spectrum emissions mask for intra-band non-contiguous EN-DC

6.5B.2.2.1.1	Test purpose

To verify that the power of any UE emissions shall not exceed specified level for the specified channel bandwidth.

6.5B.2.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.2.2.1.3	Minimum conformance requirements

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.2.1.

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask and therefore LTE anchor agnostic approach is not applied.

6.5B.2.2.1.4	Test description

6.5B.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in TS 38.508-1 [6] clause 4.3.1.4.3, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in TS 38.508-1 [6] clause 4.3.1.4.3 and are shown in Table 6.5B.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.2.2.1.4.1-1: Test Configuration Table

| Initial Conditions and Test Parameters |  |
| --- | --- |
| Same as defined in Table 6.2B.2.2.4.1-1 with the following exceptions. |  |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 | Normal |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2. The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.2.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.2.2.1.4.2	Test Procedure

1.	For NR carrier, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	For E-UTRA carrier, SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command starting in this step for the UE to reach PUMAX level.

4.	For a UE supporting dynamic power sharing, measure the mean power over all component carriers. For a UE not supporting dynamic power sharing, measure the power of each component carrier individually. The measure transmitted power shall meet the requirements in clause 6.2B.2.2.5. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms). For TDD slots with transient periods are not under test.

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to clause 6.5B.2.2.1.5 and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency.

NOTE 1: When switching to DFT-s-OFDM waveform, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5B.2.2.1.4.3	Message Content

Same message contents as in clause 6.2B.2.2.4.3.

6.5B.2.2.1.5	Test requirement

-For NR carrier frequency masks not overlapping with E-UTRA carrier frequency masks, the power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1 defined in TS 38.521-1 [8].

-For E-UTRA carrier frequency masks not overlapping with NR carrier frequency masks, the power of any UE emission shall fulfil requirements in Table 6.6.2.1.5-1 or 6.6.2.1.5-2 in TS 36.521-1 [10], as applicable.

-For NR carrier frequency masks overlapping with E-UTRA carrier frequency masks, the most relaxed limit is used between requirements in Table 6.5.2.2.5-1 defined in TS 38.521-1 [8] and Table 6.6.2.1.5-1 or 6.6.2.1.5-2 in TS 36.521-1 [10], as applicable.

##### 6.5B.2.2.2	Additional Spectrum emissions mask for intra-band non-contiguous EN-DC

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	For Wgap < NR ΔfOOB + E-UTRA ΔfOOB, test description and test requirements are FFS.

6.5B.2.2.2.1	Test purpose

Same test purpose as in clause 6.5.2.3 in TS 38.521-1 [8] for the NR carrier.

6.5B.2.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.2.2.2.3	Minimum conformance requirements

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.2.2.

No exception requirements applicable to NR or E-UTRA when Wgap > NR ΔfOOB + E-UTRA ΔfOOB. LTE anchor agnostic approach is applied when Wgap > NR ΔfOOB + E-UTRA ΔfOOB.

Exception requirements for both NR and E-UTRA are defined for this test when Wgap < NR ΔfOOB + E-UTRA ΔfOOB and therefore LTE anchor agnostic approach is not applied when Wgap < NR ΔfOOB + E-UTRA ΔfOOB.

6.5B.2.2.2.4	Test description

For Wgap > NR ΔfOOB + E-UTRA ΔfOOB:

Same test description as in clause 6.5.2.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5.2.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.2.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.5.2.3.4.2 in TS 38.521-1 [8] with the following steps exception:

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

For Wgap < NR ΔfOOB + E-UTRA ΔfOOB:

FFS.

6.5B.2.2.2.4.3	Message Content

Message contents are according to TS 36.508-1 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.2.2.2.4.3-1: RRCConnectionReconfiguration: nr-Config-r15 when Wgap < NR ΔfOOB + E-UTRA ΔfOOB

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.2.2.2.4.3-2: PhysicalCellGroupConfig when Wgap < NR ΔfOOB + E-UTRA ΔfOOB

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.2.2.2.4.3-3: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band when Wgap < NR ΔfOOB + E-UTRA ΔfOOB

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.5B.2.2.2.5	Test requirement

For Wgap > NR ΔfOOB + E-UTRA ΔfOOB:

Power of any UE emission shall fulfil requirements in Table 6.5.2.3.5-1 defined in TS 38.521-1 [8] for the NR carrier.

For Wgap < NR ΔfOOB + E-UTRA ΔfOOB:

FFS.

##### 6.5B.2.2.3	Adjacent channel leakage ratio for intra-band non-contiguous EN-DC

6.5B.2.2.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5B.2.2.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.2.2.3.3	Minimum conformance requirements

For intra-band non-contiguous EN-DC, the EN-DC Adjacent Channel Leakage power Ratio (EN-DCACLR) is the ratio of the sum of the filtered mean powers centred on the assigned E-UTRA and NR sub-block frequencies to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. In case the sub-block gap bandwidth Wgap is smaller than an E-UTRA or NR sub-block bandwidth, no EN-DCACLR requirement is set for the corresponding sub-block for the gap. The assigned EN-DC sub-block power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in TS 36.101 [5] for the E-UTRA sub-block, and TS 38.101-1 [2] for the NR sub-block. If the measured adjacent channel power is greater than –50dBm then the EN-DCACLR shall be higher than the value specified in for E-UTRAACLR and NRACLR.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.2.3.

Adjacent Channel Leakage power Ratio must be measured for both NR and E-UTRA and therefore LTE anchor agnostic approach is not applied.

6.5B.2.2.3.4	Test description

6.5B.2.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in TS 38.508-1 [6] clause 4.3.1.4.3, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in TS 38.508-1 [6] clause 4.3.1.4.3 and are shown in Table 6.5B.2.2.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.2.2.3.4.1-1: Test Configuration Table

Initial Conditions and Test Parameters

Same as defined in Table 6.2B.2.2.4.1-1

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2. The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.2.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.2.2.3.4.2	Test Procedure

1.	For NR carrier, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.3.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	For E-UTRA carrier, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously power control “up” commands to the UE for NR and E-UTRA until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements in clause 6.2B.2.2.5 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

5.	Measure the rectangular filtered mean power for the assigned NR channel using a rms detector.

6.	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR channel using a rms detector, respectively. Skip the measurement within the Wgap in case Wgap < NR sub-block bandwidth.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

8.	Measure the rectangular filtered mean power for E-UTRA channel using a rms detector.

9.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively. Skip the measurement within the Wgap in case Wgap < E-UTRA sub-block bandwidth.

10.	Calculate the ratios of the power between the values measured in step 7 over step 8 for lower and upper E-UTRAACLR, respectively.

NOTE 1:	When switching to DFT-s-OFDM waveform, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5B.2.2.3.4.3	Message Content

Same message contents as in clause 6.2B.2.2.4.3.

6.5B.2.2.3.5	Test requirement

For NR ACLR, if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR shall be higher than the limits in Table 6.5.2.4.1.5-2 defined in clause 6.5.2.4.1.5 in TS 38.521-1 [8] for the NR carrier.

For E-UTRA ACLR, if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRA ACLR shall be higher than the limits in Table 6.6.2.3.5.1-1 defined in clause 6.6.2.3.5 in TS 36.521-1 [10] for the E-UTRA carrier.

#### 6.5B.2.3	Out-of-band emissions for Inter-band EN-DC within FR1

##### 6.5B.2.3.1	Spectrum emissions mask for Inter-band EN-DC within FR1 (1 NR CC)

6.5B.2.3.1.1	Test purpose

Same test purpose as in clause 6.5.2.2 in TS 38.521-1 [8] for the NR carrier.

6.5B.2.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.5B.2.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.2.3.1.4	Test description

Same test description as in clause 6.5.2.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.For Initial conditions as in clause 6.5.2.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.2.3.1.5	Test requirement

Power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1 defined in TS 38.521-1 [8] for the NR carrier.6.5B.2.3.2.

##### 6.5B.2.3.2	Additional Spectrum emissions mask for Inter-band EN-DC within FR1 (1 NR CC)

6.5B.2.3.2.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-1 [8] for the NR carrier.

6.5B.2.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.5B.2.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.2.3.2.4	Test description

Same test description as in clause 6.5.2.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5.2.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.3.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.2.3.2.5	Test requirement

Power of any UE emission shall fulfil requirements in applicable table from Table 6.5.2.3.5-1 to Table 6.5.2.3.5.2-1 defined in TS 38.521-1 [8] for the NR carrier.

##### 6.5B.2.3.3	Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC)

###### 6.5B.2.3.3.1	NR - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC)

6.5B.2.3.3.1.1	Test purpose

Same test purpose as in clause 6.5.2.4.1.1 in TS 38.521-1 [8].

6.5B.2.3.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.5B.2.3.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.4.1.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied

6.5B.2.3.3.1.4	Test description

Same test description as in clause 6.5.2.4.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.5.2.4.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.4.1.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.2.3.3.1.5	Test requirement

Same test requirement as in clause 6.5.2.4.1.5 in TS 38.521-1 [8].

###### 6.5B.2.3.3.2	UTRA - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC)

6.5B.2.3.3.2.1	Test purpose

Same test purpose as in clause 6.5.2.4.2.1 in TS 38.521-1 [8].

6.5B.2.3.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.5B.2.3.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.4.2.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied

6.5B.2.3.3.2.4	Test description

Same test description as in clause 6.5.2.4.2.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.5.2.4.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.4.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.2.3.3.2.5	Test requirement

Same test requirement as in clause 6.5.2.4.2.5 in TS 38.521-1 [8].

#### 6.5B.2.3a	Out-of-band emissions for Inter-band NE-DC within FR1

##### 6.5B.2.3a.1	Spectrum emissions mask for Inter-band NE-DC within FR1

No exception requirements applicable to NR or LTE.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 6.5.2.2 and 6.5A.2.2 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.6.2.1 and 6.6.2.1A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

##### 6.5B.2.3a.2	Additional Spectrum emissions mask for Inter-band NE-DC within FR1

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 6.5.2.3 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.6.2.2 and 6.6.2.2A  of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

##### 6.5B.2.3a.3	Adjacent channel leakage ratio for inter-band NE-DC within FR1

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 6.5.2.4 and 6.5A.2.4 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.6.2.3 and 6.6.2.3A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

#### 6.5B.2.4	Out-of-band emissions for Inter-band EN-DC including FR2

##### 6.5B.2.4.1	Spectrum emissions mask for Inter-band EN-DC including FR2 (1 NR CC)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1 FR2b, 2, and 4.

6.5B.2.4.1.1	Test purpose

Same test purpose as in clause 6.5.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.2.4.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.2.4.1.4	Test description

6.5B.2.4.1.4.1	Initial conditions

Same test description as in clause 6.5.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.5.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.1.5	Test requirement

Same test requirement as in clause 6.5.2.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.2.4.1a	Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC)

Editor’s note: The following aspects are either missing or not yet determined:

-	The referred test case 6.5.2.1_1 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

6.5B.2.4.1a.1	Test purpose

Same test purpose as in clause 6.5.2.1_1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.1a.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.2.4.1a.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1_1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.2.4.1a.4	Test description

Same test description as in clause 6.5.2.1_1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.5.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2.4_1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.2.1_1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.1a.5	Test requirement

Same test requirement as in clause 6.5.2.1_1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.2.4.1_1	Spectrum emissions mask for Inter-band EN-DC including FR2 (>1 NR CC)

Editor's note: Test applicability, Test Description in below sub-clauses requires updates to clarify number of E-UTRA carriers that will be configured during the test

###### 6.5B.2.4.1_1.1	Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.1.1 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

6.5B.2.4.1_1.1.1	Test purpose

Same test purpose as in clause 6.5.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.1_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.2.4.1_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.1.4.

6.5B.2.4.1_1.1.4	Test description

6.5B.2.4.1_1.1.4.1	Initial condition

Same test description as in clause 6.5A.2.1.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.1_1.1.5	Test Requirements

Same test requirement as in clause 6.5A.2.1.1.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.2.4.1_1.2	Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.1.2 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

6.5B.2.4.1_1.2.1	Test purpose

Same test purpose as in clause 6.5.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.1_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.5B.2.4.1_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.1.4.

6.5B.2.4.1_1.2.4	Test description

6.5B.2.4.1_1.2.4.1	Initial condition

Same test description as in clause 6.5A.2.1.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.1.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.1.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.1.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.1_1.2.5	Test Requirements

Same test requirement as in clause 6.5A.2.1.2.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.2.4.1_1.3	Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.1.3 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

6.5B.2.4.1_1.3.1	Test purpose

Same test purpose as in clause 6.5.2.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.1_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.5B.2.4.1_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.1.4.

6.5B.2.4.1_1.3.4	Test description

6.5B.2.4.1_1.3.4.1	Initial condition

Same test description as in clause 6.5A.2.1.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.1.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.1.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.1.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.1_1.3.5	Test Requirements

Same test requirement as in clause 6.5A.2.1.3.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.2.4.1D	Spectrum emissions mask for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5D.2.1 in TS 38.521-2 [9] is incomplete

6.5B.2.4.1D.1	Test purpose

Same test purpose as in clause 6.5D.2.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.1D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC FR2.

6.5B.2.4.1D.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5D.2.1 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.2B.2.4.

6.5B.2.4.1D.4	Test description

Same test description as in clause [6.5D.2.1] in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause [6.5D.2.1] in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

[Step 6] of Initial conditions as in clause [6.5D.2.1] in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

6.5B.2.4.1D.5	Test Requirement

Same test requirement as in 	clause 6.5D.2.1.5 of TS 38.521-2 [9] for the NR carrier.

##### 6.5B.2.4.3	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5.2.3 in TS 38.521-2 [9] is incomplete for PC2 and PC4.

-	The referred test case 6.5.2.3 in TS 38.521-2 [9] is incomplete for aggregated BW > 400MHz.

6.5B.2.4.3.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.2.4.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.2.4.3.4	Test description

6.5B.2.4.3.4.1	Initial conditions

Same test description as in clause 6.5.2.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.5.2.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.2.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.3.5	Test requirement

Same test requirement as in clause 6.5.2.3.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.2.4.3_1	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (>1 NR CC)

Editor's note: Test applicability, Test Description in below sub-clauses requires updates to clarify number of E-UTRA carriers that will be configured during the test

###### 6.5B.2.4.3_1.1	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.2.1 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

6.5B.2.4.3_1.1.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.3_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.2.4.3_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.5B.2.4.3_1.1.4	Test description

6.5B.2.4.3_1.1.4.1	Initial condition

Same test description as in clause 6.5A.2.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.3_1.1.5	Test Requirements

Same test requirement as in clause 6.5A.2.2.1.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.2.4.3_1.2	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.2.2 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

6.5B.2.4.3_1.2.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.3_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.5B.2.4.3_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.5B.2.4.3_1.2.4	Test description

6.5B.2.4.3_1.2.4.1	Initial condition

Same test description as in clause 6.5A.2.2.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.2.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.2.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.2.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.3_1.2.5	Test Requirements

Same test requirement as in clause 6.5A.2.2.2.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.2.4.3_1.3	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.2.3 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

6.5B.2.4.3_1.3.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.3_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.5B.2.4.3_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.4.

6.5B.2.4.3_1.3.4	Test description

6.5B.2.4.3_1.3.4.1	Initial condition

Same test description as in clause 6.5A.2.2.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.2.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.2.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.2.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.3_1.3.5	Test Requirements

Same test requirement as in clause 6.5A.2.2.3.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.2.4.3_1.4	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 NR CCs)

Editor's note:	The test case is incomplete. 	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.2.4 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

-	Measurement Uncertainties and Test Tolerances is incomplete.

6.5B.2.4.3_1.4.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.3_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 NR UL CCs.

6.5B.2.4.3_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.4.

6.5B.2.4.3_1.4.4	Test description

6.5B.2.4.3_1.4.4.1	Initial condition

Same test description as in clause 6.5A.2.2.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.3_1.4.5	Test Requirements

Same test requirement as in clause 6.5A.2.2.4.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.2.4.3_1.5	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (6 NR CCs)

Editor's note:	The test case is incomplete.	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.2.4 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

-	Measurement Uncertainties and Test Tolerances is incomplete.

6.5B.2.4.3_1.5.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.3_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 6 NR UL CCs.

6.5B.2.4.3_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.4.

6.5B.2.4.3_1.5.4	Test description

6.5B.2.4.3_1.5.4.1	Initial condition

Same test description as in clause 6.5A.2.2.5.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.3_1.5.5	Test Requirements

Same test requirement as in clause 6.5A.2.2.5.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.2.4.3_1.6	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (7 NR CCs)

Editor's note:	The test case is incomplete.	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.2.4 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

-	Measurement Uncertainties and Test Tolerances is incomplete.

6.5B.2.4.3_1.6.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.3_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 7 NR UL CCs.

6.5B.2.4.3_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.4.

6.5B.2.4.3_1.6.4	Test description

6.5B.2.4.3_1.6.4.1	Initial condition

Same test description as in clause 6.5A.2.2.6.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.3_1.6.5	Test Requirements

Same test requirement as in clause 6.5A.2.2.6.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.2.4.3_1.7	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (8 NR CCs)

Editor's note:	The test case is incomplete.	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.2.2.4 in TS 38.521-2 [9] is incomplete for intra-band contiguous CA supporting aggregated BW > 400MHz are TBD.

-	Measurement Uncertainties and Test Tolerances is incomplete.

6.5B.2.4.3_1.7.1	Test purpose

Same test purpose as in clause 6.5.2.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.3_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 8 NR UL CCs.

6.5B.2.4.3_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.4.

6.5B.2.4.3_1.7.4	Test description

6.5B.2.4.3_1.7.4.1	Initial condition

Same test description as in clause 6.5A.2.2.7.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.2.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.2.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.3_1.7.5	Test Requirements

Same test requirement as in clause 6.5A.2.2.7.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.2.4D.3	Adjacent channel leakage ratio for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5D.2.2 in TS 38.521-2 [9] is incomplete

6.5B.2.4D.3.1	Test purpose

Same test purpose as in clause 6.5D.2.2 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4D.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC FR2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.2.4D.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5D.2.2 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.2B.2.4.

6.5B.2.4D.3.4	Test description

Same test description as in clause 6.5D.2.2 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5D.2.2 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

[Step 6] of Initial conditions as in clause 6.5D.2.2 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

Same test procedure as in clause 6.5D.2.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4D.3.5	Test Requirement

Same test requirement as in clause 6.5D.2.2 of TS 38.521-2 [9] for the NR carrier.

### 6.5B.3	Spurious emissions for EN-DC

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions. The spurious emission limits are specified in terms of general requirements in line with SM.329 [3] and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5B.3.1	Spurious Emissions for intra-band contiguous EN-DC

##### 6.5B.3.1.1	General spurious emissions for intra-band contiguous EN-DC

6.5B.3.1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5B.3.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.5B.3.1.1.3	Minimum conformance requirements

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [5] and clause 6.5.3.1 of TS 38.101-1 [2] apply beyond any frequencies for which the out-of-band emissions requirements in clause 6.5B.2.1 of TS 38.101-3 [4] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.1.1.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.3.1.1.4	Test description

Same test description as in clause 6.5.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 6.5B.3.1.1.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | Low range, Mid range, High range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested for Lowest NRB_agg and Highest NRB_agg, respectively. |  |

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths are specified in Table 4.6-1 except for the parameters specified in Table 6.5B.3.1.1.4-1.

For Initial conditions as in clause 6.5.3.1.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 6.5B.3.1.1.4-1.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

4.1.	The UL Reference Measurement channels are set according to Table 6.5B.3.1.1.4-1.

Step 6 of Initial conditions as in clause 6.5.3.1.4 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.5.3.1.4 in TS 38.521-1 [8].

6.5B.3.1.1.5	Test Requirement

The measured average power of spurious emission, derived in step 5, shall not exceed the described value in Table 6.5B.3.1.1.5-1.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) from the edge of the channel bandwidth shown in Table 6.5.3.1.3-1 of TS 38.521-1 [8] for NR carrier, and Table 6.6.3.1.3-1 of TS 36.521-1[10] for E-UTRA carrier.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5B.3.1.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz. NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz. NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 36.101 [5] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |

##### 6.5B.3.1.2	Spurious emission band UE co-existence for intra-band contiguous EN-DC

6.5B.3.1.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions for band UE co-existence for intra-band contiguous EN-DC.

6.5B.3.1.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward supporting intra-band contiguous EN-DC.

6.5B.3.1.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified EN-DC configurations for coexistence with protected bands.

The requirements in Tables 6.5B.3.1.2.3-1 and 6.5B.3.1.2.3-1a apply on each component carrier with all component carriers are active.

Table 6.5B.3.1.2.3-1: Requirements for intra band contiguous EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_(n)71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 NR Band n77 5 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 3 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| DC_(n)41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 66, 70, 71, 73, 74 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than F OOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 5: Only applies to NR UE release 16 and forward supporting intra-band contiguous EN-DC. |  |  |  |  |  |  |  |

Table 6.5B.3.1.2.3-1a: Requirements for intra band contiguous EN-DC in Rel-18

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_(n)71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 54 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 NR Band n77 5 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 3 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| DC_(n)41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 54 66, 70, 71, 73, 74 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than F OOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 5: Only applies to NR UE release 16 and forward supporting intra-band contiguous EN-DC. |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.1.2.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included, and E-UTRA measurements are performed.

6.5B.3.1.2.4	Test description

6.5B.3.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.2, and the channel bandwidth combination for E-UTRA and NR component carriers shall follow the value specified in Table 5.3B.1.2-1. All these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 6.5B.3.1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.3.1.2.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  | Low range, High range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  | Lowest and Highest NRB_agg (NOTE 3) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  | Lowest SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Spurious emission. | QPSK | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right |
| 2 |  | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same aggregated channel BW, only the combination with the highest NR BW is tested. NOTE 4: The test configuration applies to intra-band contiguous EN-DC indicating support of dual simultaneous UL as defined in clause 5.5B.2. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	E-UTRA downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

4.	NR downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG, respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG, respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.3.1.2.4.3.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.3.1.2.4.2	Test Procedure

1.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.1.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.1.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Both NR and E-UTRA SS send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.3.1.2.3-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5B.3.1.2.3-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

6.5B.3.1.2.4.3	Message Contents

Message contents are according to TS 36.508 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions:

Table 6.5B.3.1.2.4.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.3.1.2.4.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.3.1.2.4.3-3: SystemInfomationBlockType1: tdd-Config if operating on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.5B.3.1.2.5	Test Requirement

Test requirements for Spurious Emissions UE Co-existence for intra-band contiguous EN-DC are the same as the minimum requirements described in clause 6.5B.3.1.2.3 minimum requirements and are not repeated in this clause.

The requirements for the UE are release specific and can be found in Tables 6.5B.3.1.2.3-1 to 6.5B.3.1.2.3-1a. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined.

#### 6.5B.3.2	Spurious Emissions for intra-band non-contiguous EN-DC

##### 6.5B.3.2.1	General spurious emissions for Intra-band non-contiguous EN-DC

6.5B.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5B.3.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward supporting intra-band non-contiguous EN-DC.

6.5B.3.2.1.3	Minimum conformance requirements

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.521-1 [10] and clause 6.5.3.1 of TS 38.521-1 [8] apply beyond any frequencies for which the out-of-band emissions requirements in clause 6.5B.2.2 of TS 38.101-3 [4] apply. If for some frequency an individual CC spurious emission requirement overlaps with the general spectrum emission mask or the bandwidth of another CC then it does not apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.2.1.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.3.2.1.4	Test description

Same test description as in clause 6.5.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 6.5B.3.2.1.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | Low with maxWgap High with maxWgap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested for Lowest NRB_agg and Highest NRB_agg, respectively. |  |

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths are specified in Table 4.6-1 except for the parameters specified in Table 6.5B.3.2.1.4-1.

For Initial conditions as in clause 6.5.3.1.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 6.5B.3.2.1.4-1.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Same test procedure as in clause 6.5.3.1.4.2 in TS 38.521-1 [8].

6.5B.3.2.1.5	Test Requirement

Same test requirement as in clause 6.5B.3.1.1.5.

##### 6.5B.3.2.2	Spurious emission band UE co-existence for intra-band non-contiguous EN-DC

6.5B.3.2.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions for band UE co-existence for intra-band non-contiguous EN-DC.

6.5B.3.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward supporting intra-band non-contiguous EN-DC.

6.5B.3.2.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified EN-DC configurations for co-existence with protected bands.

The requirements in Tables 6.5B.3.2.2.3-1 and  6.5B.3.2.2.3-1a apply with all component carriers are active.

Table 6.5B.3.2.2.3-1: Requirements for intra-band non-contiguous EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| DC_41A_n41A | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 27, 28, 29, 34, 39, 42, 44, 45, 48, 50, 51, 66, 70, 71, 73, 74 NR Band n77, n78 and n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 30, 40 | FDL_low | - | FDL_high | -40 | 1 |  |
| … |  |  |  |  |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th, or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

Table 6.5B.3.2.2.3-1a: Requirements for intra-band non-contiguous EN-DC in Rel-18

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| DC_41A_n41A | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 27, 28, 29, 34, 39, 42, 44, 45, 48, 50, 51, 54, 66, 70, 71, 73, 74 NR Band n77, n78 and n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 30, 40 | FDL_low | - | FDL_high | -40 | 1 |  |
| … |  |  |  |  |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th, or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for EUTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.2.2.

Exception requirements are applicable for NR but not for E-UTRA within this test. LTE anchor agnostic approach is not applied. E-UTRA configuration is included.

6.5B.3.2.2.4	Test description

6.5B.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.3, and the channel bandwidth combination for E-UTRA and NR component carriers shall follow the value specified in Table 5.3B.1.3-1. All these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.3 and are shown in Table 6.5B.3.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.3.2.2.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  | Low with maxWgap High with maxWgap |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.3-1 |  | Lowest and Highest NRB_agg (NOTE 3) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  | Lowest SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Spurious emission. | QPSK | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right |
| 2 |  | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same aggregated channel BW, only the combination with the highest NR BW is tested. NOTE 4: The test configuration applies to intra-band non-contiguous EN-DC indicating support of dual simultaneous UL as defined in clause 5.5B.3. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for SS diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

4.	NR downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B and TS 38.521-1 [8] Annex B for E-UTRA link and NR link respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.3.2.2.4.3.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.3.2.2.4.2	Test Procedure

1.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.2.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.2.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for both NR and E-UTRA carriers until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.3.2.2.3-1. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

6.5B.3.2.2.4.3	Message Contents

Message contents are according to TS 36.508 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.3.2.2.4.3-1: RRCConnectionReconfiguration: nr-Config-r15.

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.3.2.2.4.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.3.2.2.4.3-3: SystemInfomationBlockType1: tdd-Config if operating on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.5B.3.2.2.5	Test Requirement

Test requirements for Spurious Emissions UE Co-existence for intra-band non-contiguous EN-DC are the same as the minimum requirements described in clause 6.5B.3.2.2.3 and are not repeated in this clause.

The requirements for the UE are release specific and can be found in Tables 6.5B.3.2.2.3-1 to 6.5B.3.2.2.3-1a. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined.

#### 6.5B.3.3	Spurious Emissions for Inter-band EN-DC within FR1

##### 6.5B.3.3.1	General spurious emissions for Inter-band EN-DC within FR1

6.5B.3.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5B.3.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward supporting inter-band EN-DC.

6.5B.3.3.1.3	Minimum conformance requirements

The general spurious emissions requirements specified in subclause 6.6.3.1 of TS 36.101 [5], subclause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. For the case of inter-band EN-DC with a single carrier per cell group, the general spurious emissions requirements also apply with both downlink carrier and both uplink carriers active. Limits on configured maximum output power for the uplink according to subclause 6.2B.4 apply.

NOTE:	The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band.  Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.3.1. Exception requirements applicable for both NR and LTE, therefore LTE anchor agnostic approach is not applied.

6.5B.3.3.1.4	Test description

6.5B.3.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, DC configuration specified in Table 5.5B.4.1-1 and test channel bandwidths specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1, and sub-carrier spacing based on NR operating bands specified in TS 38.521-1 [8] clause 5.3. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 6.5B.3.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.3.3.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  | 5 MHz for E-UTRA CC and Lowest for NR CC Highest for E-UTRA CC and Highest for NR CC |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  | Lowest SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Spurious emission. | QPSK | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Left |
| 2 |  | QPSK | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Right |
| 3 |  | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 3: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 4: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B.4. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for SS diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

4.	NR downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B and TS 38.521-1 [8] Annex B for E-UTRA link and NR link respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.3.3.1.4.3.

8.	For both E-UTRA and NR UL uplink carriers active when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.3.3.1.4.2	Test procedure

1.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for both NR and E-UTRA carriers until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.3.3.1.5-1(for EN-DC configurations listed in Table 6.5B.3.3.1.5-1) or Table 6.5.3.1.5-1 in TS 38.521-1 [8] (for EN-DC configurations not listed in Table 6.5B.3.3.1.5-1). The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

5.	For UE operating on EN-DC configuration with Band n41, redo the test for frequency range 1 GHz  f < 12.75 GHz with the message content in step 7 of initial conditions with exceptions defined in Table 6.5B.3.3.1.4.3-4.

In addition to test configuration and test procedure above, EN-DC only capable UEs and UEs not supporting NR/5GC mode in the tested band need to be tested according to LTE anchor agnostic below for EN-DC configurations listed in Table 6.5B.3.3.1.5-1.

Same test description as in clause 6.5.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.3.3.1.4.3	Message Contents

Message contents are according to TS 36.508 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.3.3.1.4.3-1: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | Sa2 |  |  |
| specialSubframePatterns | Ssp7 |  |  |
| } |  |  |  |

Table 6.5B3.3.1.4.3-1a: Void

Table 6.5B.3.3.1.4.3-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.3.3.1.4.3-3: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Exception for step 5 in test procedure:

Table 6.5B.3.3.1.4.3-4: Message contents

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.5B.3.3.1.5	Test Requirement

For EN-DC configurations listed in Table 6.5B.3.3.1.5-1, the corresponding test requirements of the same table apply and are required to be tested with both LTE and NR carriers active.

For EN-DC configurations not listed in Table 6.5B.3.3.1.5-1, the test requirements in Table 6.5.3.1.5-1 in TS 38.521-1[8] shall apply and are required to be tested with both LTE and NR carriers active.

For EN-DC only capable devices, for EN-DC configurations listed in Table 6.5B.3.3.1.5-1 test requirements for NR carrier are the same as Table 6.5.3.1.5-1 in TS 38.521-1 [8] and are tested with anchor agnostic approach.

Table 6.5B.3.3.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| Test requirements for DC_1A_n3A configuration |  |  |  |
| 135 MHz ≤ f ≤ 270 MHz | -36 dBm | 100 kHz |  |
| 1440 MHz ≤ f ≤ 1650 MHz 2055 MHz ≤ f ≤ 2250 MHz 3630 MHz ≤ f ≤ 3765 MHz 5340 MHz ≤ f ≤ 5745 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_1A_n5A configuration |  |  |  |
| 222 MHz ≤ f ≤ 332 MHz | -36 dBm | 100 kHz |  |
| 1071 MHz ≤ f ≤ 1156 MHz 2744 MHz ≤ f ≤ 2829 MHz 2991 MHz ≤ f ≤ 3136 MHz 3568 MHz ≤ f ≤ 3678 MHz 4664 MHz ≤ f ≤ 4809 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_1A_n7A configuration |  |  |  |
| 520 MHz ≤ f ≤ 650 MHz | -36 dBm | 100 kHz |  |
| 1270 MHz ≤ f ≤ 1460 MHz 3020 MHz ≤ f ≤ 3220 MHz 4420 MHz ≤ f ≤ 4550 MHz 6340 MHz ≤ f ≤ 6530 MHz 6920 MHz ≤ f ≤ 7120 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_1A_n8A configuration |  |  |  |
| 90 MHz ≤ f ≤ 220 MHz | -36 dBm | 100 kHz |  |
| 1005 MHz ≤ f ≤ 1100 MHz 2800 MHz ≤ f ≤ 2895 MHz 2925 MHz ≤ f ≤ 3080 MHz 3680 MHz ≤ f ≤ 3810 MHz 4720 MHz ≤ f ≤ 4875 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_1A_n28A configuration |  |  |  |
| 424 MHz ≤ f ≤ 574 MHz | -36 dBm | 100 kHz |  |
| 1172 MHz ≤ f ≤ 1277 MHz 2623 MHz ≤ f ≤ 2728 MHz 3092 MHz ≤ f ≤ 3257 MHz 3326 MHz ≤ f ≤ 3476 MHz 4543 MHz ≤ f ≤ 4708 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_1A_n41A configuration |  |  |  |
| 516 MHz ≤ f ≤ 770 MHz | -36 dBm | 100 kHz |  |
| 1150 MHz ≤ f ≤ 1464 MHz 3012 MHz ≤ f ≤ 3460 MHz 4416 MHz ≤ f ≤ 4670 MHz 6336 MHz ≤ f ≤ 6650 MHz 6912 MHz ≤ f ≤ 7360 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_1A_n77A Configuration |  |  |  |
| 360 MHz ≤ f ≤ 660 MHz | -36 dBm | 100 kHz |  |
| 1320 MHz ≤ f ≤ 2280 MHz 4620 MHz ≤ f ≤ 6480 MHz 7140 MHz ≤ f ≤ 8160 MHz 8520 MHz ≤ f ≤ 10380 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_1A_n78A Configuration |  |  |  |
| 40 MHz ≤ f ≤ 660 MHz | -36 dBm | 100 kHz |  |
| 1320 MHz ≤ f ≤ 1880 MHz 4620 MHz ≤ f ≤ 5780 MHz 7140 MHz ≤ f ≤ 7760 MHz 8520 MHz ≤ f ≤ 9580 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_1A_n79A Configuration |  |  |  |
| 440 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1160 MHz 2420 MHz ≤ f ≤ 3080 MHz 6320 MHz ≤ f ≤ 8080 MHz 8240 MHz ≤ f ≤ 8960 MHz 10720 MHz ≤ f ≤ 11980 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_2A_n5A Configuration |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz | -36 dBm | 100 kHz |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_2A_n41A Configuration |  |  |  |
| 586 MHz ≤ f ≤ 840 MHz | -36 dBm | 100 kHz |  |
| 1010 MHz ≤ f ≤ 1324 MHz 3082 MHz ≤ f ≤ 3530 MHz 4346 MHz ≤ f ≤ 4600 MHz 6196 MHz ≤ f ≤ 6510 MHz 6842 MHz ≤ f ≤ 7290 MHz | -30 dBm | 1 MHz | 2 |
|  | -25 dBm | 1 MHz | 1 |
| Test requirements for DC_2A_n71A Configuration |  |  |  |
| 454 MHz ≤ f ≤ 584 MHz | -36 dBm | 100 kHz |  |
| 1152 MHz ≤ f ≤ 1247 MHz 2513 MHz ≤ f ≤ 2608 MHz 3002 MHz ≤ f ≤ 3157 MHz 3176 MHz ≤ f ≤ 3306 MHz 4363 MHz ≤ f ≤ 4518 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_2A_n77A Configuration |  |  |  |
| 500 MHz ≤ f ≤ 520 MHz | -36 dBm | 100 kHz |  |
| 1390 MHz ≤ f ≤ 2350 MHz 4690 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8020 MHz 8450 MHz ≤ f ≤ 10310 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n1A Configuration |  |  |  |
| 135 MHz ≤ f ≤ 270 MHz | -36 dBm | 100 kHz |  |
| 1440 MHz ≤ f ≤ 1650 MHz 2055 MHz ≤ f ≤ 2250 MHz 3630 MHz ≤ f ≤ 3765 MHz 5340 MHz ≤ f ≤ 5745 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n5A Configuration |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 137 MHz 861 MHz ≤ f ≤ 961 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz ≤ f ≤ 2746 MHz 3358 MHz ≤ f ≤ 3483 MHz 4244 MHz ≤ f ≤ 4419 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n7A Configuration |  |  |  |
| 715 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1070 MHz 3215 MHz ≤ f ≤ 3430 MHz 4210 MHz ≤ f ≤ 4355 MHz 5920 MHz ≤ f ≤ 6140 MHz 6710 MHz ≤ f ≤ 6925 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n8A Configuration |  |  |  |
| 25 MHz ≤ f ≤ 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 120 MHz 795 MHz ≤ f ≤ 905 MHz | -36 dBm | 100 kHz |  |
| 2505 MHz ≤ f ≤ 2700 MHz 3470 MHz ≤ f ≤ 3615 MHz 4300 MHz ≤ f ≤ 4485 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n28A Configuration |  |  |  |
| 214 MHz ≤ f ≤ 379 MHz 962 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1082 MHz 2413 MHz ≤ f ≤ 2533 MHz 2672 MHz ≤ f ≤ 2867 MHz 3116 MHz ≤ f ≤ 3281 MHz 4123 MHz ≤ f ≤ 4318 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n41A Configuration |  |  |  |
| 711 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000MHz ≤ f ≤ 1074 MHz 3207MHz ≤ f ≤ 3670 MHz 4206MHz ≤ f ≤ 4475 MHz 5916MHz ≤ f ≤ 6260 MHz 6702MHz ≤ f ≤ 7165 MHz | -30 dBm | 1 MHz | 2 |
|  | -25 dBm | 1 MHz | 1 |
| Test requirements for DC_3A_n77A Configuration |  |  |  |
| 270 MHz ≤ f ≤ 780 MHz | -36 dBm | 100 kHz |  |
| 1515 MHz ≤ f ≤ 2490 MHz 4815 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7770 MHz 8310 MHz ≤ f ≤ 10185 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n78A Configuration |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz | -36 dBm | 100 kHz |  |
| 1515 MHz ≤ f ≤ 2090 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_3A_n79A Configuration |  |  |  |
| 830 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1580 MHz 2615 MHz ≤ f ≤ 3290 MHz 6110 MHz ≤ f ≤ 6785 MHz 7015 MHz ≤ f ≤ 8570 MHz 10510 MHz ≤ f ≤ 11785 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_5A_n2A Configuration |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz | -36 dBm | 100 kHz |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_5A_n66A Configuration |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 132 MHz 861 MHz ≤ f ≤ 956 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz ≤ f ≤ 2736 MHz 3358 MHz ≤ f ≤ 3478 MHz 4244 MHz ≤ f ≤ 4409 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_5A_n77A Configuration |  |  |  |
| 1602 MHz ≤ f ≤ 3376 MHz 4124 MHz ≤ f ≤ 9249 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_5A_n78A Configuration |  |  |  |
| 1602 MHz ≤ f ≤ 2152 MHz 2451 MHz ≤ f ≤ 2976 MHz 4124 MHz ≤ f ≤ 4649 MHz 4948 MHz ≤ f ≤ 5498 MHz 5751 MHz ≤ f ≤ 6776 MHz 7424 MHz ≤ f ≤ 8449 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_7A_n1A Configuration |  |  |  |
| 520 MHz ≤ f ≤ 650 MHz | -36 dBm | 100 kHz |  |
| 1270 MHz ≤ f ≤ 1460 MHz 3020 MHz ≤ f ≤ 3220 MHz 4420 MHz ≤ f ≤ 4550 MHz 6340 MHz ≤ f ≤ 6530 MHz 6920 MHz ≤ f ≤ 7120 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_7A_n3A Configuration |  |  |  |
| 715 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1070 MHz 3215 MHz ≤ f ≤ 3430 MHz 4210 MHz ≤ f ≤ 4355 MHz 5920 MHz ≤ f ≤ 6140 MHz 6710 MHz ≤ f ≤ 6925 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_7A_n5A Configuration |  |  |  |
| 802 MHz ≤ f ≤ 922 MHz | -36 dBm | 100 kHz |  |
| 1651 MHz ≤ f ≤ 1746 MHz 3324 MHz ≤ f ≤ 3419 MHz 4148 MHz ≤ f ≤ 4316 MHz 5824 MHz ≤ f ≤ 5989 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_7A_n8A Configuration |  |  |  |
| 670 MHz ≤ f ≤ 810 MHz | -36 dBm | 100 kHz |  |
| 1585 MHz ≤ f ≤ 1690 MHz 3380 MHz ≤ f ≤ 3485 MHz 4085 MHz ≤ f ≤ 4400 MHz 5880 MHz ≤ f ≤ 6055 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_7A_n28A Configuration |  |  |  |
| 1004 MHz ≤ f ≤ 1164 MHz 1752 MHz ≤ f ≤ 1867 MHz 3203 MHz ≤ f ≤ 3318 MHz 3906 MHz ≤ f ≤ 4066 MHz 4252 MHz ≤ f ≤ 4437 MHz 5703 MHz ≤ f ≤ 5888 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_7A_n78A Configuration |  |  |  |
| 730 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1840 MHz 4030 MHz ≤ f ≤ 5100 MHz 5800 MHz ≤ f ≤ 6370 MHz 8300 MHz ≤ f ≤ 8940 MHz 9100 MHz ≤ f ≤ 10170 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_8A_n1A Configuration |  |  |  |
| 90 MHz ≤ f ≤ 220 MHz | -36 dBm | 100 kHz |  |
| 1005 MHz ≤ f ≤ 1100 MHz 2800 MHz ≤ f ≤ 2895 MHz 2925 MHz ≤ f ≤ 3080 MHz 3680 MHz ≤ f ≤ 3810 MHz 4720 MHz ≤ f ≤ 4875 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_8A_n3A Configuration |  |  |  |
| 25 MHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 120 MHz 795 MHz ≤ f ≤ 905 MHz | -36 dBm | 100 kHz |  |
| 2505 MHz ≤ f ≤ 2700 MHz 3470 MHz ≤ f ≤ 3615 MHz 4300 MHz ≤ f ≤ 4485 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_8A_n20A Configuration |  |  |  |
| 18 MHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 83 MHz 749 MHz ≤ f ≤ 844 MHz 898 MHz ≤ f ≤ 998 MHz | -36 dBm | 100 kHz |  |
| 1712 MHz ≤ f ≤ 1777 MHz 2544 MHz ≤ f ≤ 2692 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_8A_n28A Configuration |  |  |  |
| 132 MHz ≤ f ≤ 212 MHz 491 MHz ≤ f ≤ 616 MHz | -36 dBm | 100 kHz |  |
| 1012 MHz ≤ f ≤ 1127 MHz 1583 MHz ≤ f ≤ 1663 MHz 2286 MHz ≤ f ≤ 2411 MHz 2463 MHz ≤ f ≤ 2578 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_8A_n41A Configuration |  |  |  |
| 666 MHz ≤ f ≤ 930 MHz | -36 dBm | 100 kHz |  |
| 1581MHz ≤ f ≤ 1810 MHz 3376MHz ≤ f ≤ 3605 MHz 4077MHz ≤ f ≤ 4520 MHz 5872MHz ≤ f ≤ 6295 MHz | -30 dBm | 1 MHz | 2 |
|  | -25 dBm | 1 MHz | 1 |
| Test requirements for DC_8A_n77A Configuration |  |  |  |
| 1470 MHz ≤ f ≤ 3320 MHz 4180 MHz ≤ f ≤ 9315 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_8A_n78A Configuration |  |  |  |
| 1470 MHz ≤ f ≤ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 4180 MHz ≤ f ≤ 4715 MHz 5060 MHz ≤ f ≤ 5630 MHz 5685 MHz ≤ f ≤ 6720 MHz 7480 MHz ≤ f ≤ 8515 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_11A_n77A Configuration |  |  |  |
| 404.2 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1344.2 MHz 1852.1 MHz ≤ f ≤ 2772.1 MHz 4727.9 MHz ≤ f ≤ 7095.8 MHz 8027.9 MHz ≤ f ≤ 9847.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_11A_n78A Configuration |  |  |  |
| 404.2 MHz ≤ f ≤ 944.2 MHz | -36 dBm | 100 kHz |  |
| 1852.1 MHz ≤ f ≤ 2372.1 MHz 4727.9 MHz ≤ f ≤ 6695.8 MHz 8027.9 MHz ≤ f ≤ 9047.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_11A_n79A Configuration |  |  |  |
| 1504.2 MHz ≤ f ≤ 2144.2 MHz 2952.1 MHz ≤ f ≤ 3572.1 MHz 5827.9 MHz ≤ f ≤ 6447.9 MHz 7255.8 MHz ≤ f ≤ 8572.1 MHz 10227.9 MHz ≤ f ≤ 11447.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_12A_n2A Configuration |  |  |  |
| 418 MHz ≤ f ≤ 512 MHz | -36 dBm | 100 kHz |  |
| 1134 MHz ≤ f ≤ 1211 MHz 2549 MHz ≤ f ≤ 2626 MHz 2984 MHz ≤ f ≤ 3121 MHz 3248 MHz ≤ f ≤ 3342 MHz 4399 MHz ≤ f ≤ 4536 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_12A_n66A Configuration |  |  |  |
| 278 MHz ≤ f ≤ 382 MHz 994 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1081 MHz 2409 MHz ≤ f ≤ 2496 MHz 2704 MHz ≤ f ≤ 2861 MHz 3108 MHz ≤ f ≤ 3212 MHz 4119 MHz ≤ f ≤ 4276 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_12A_n77A Configuration |  |  |  |
| 1868 MHz ≤ f ≤ 3501 MHz 3999 MHz ≤ f ≤ 5632 MHz 5884 MHz ≤ f ≤ 9116 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_12A_n78A Configuration |  |  |  |
| 1868 MHz ≤ f ≤ 2402 MHz 2584 MHz ≤ f ≤ 3101 MHz 3999 MHz ≤ f ≤ 4516 MHz 4698 MHz ≤ f ≤ 5232 MHz 5884 MHz ≤ f ≤ 6901 MHz 7299 MHz ≤ f ≤ 8316 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_13A_n2A Configuration |  |  |  |
| 276 MHz ≤ f ≤ 356 MHz | -36 dBm | 100 kHz |  |
| 1063 MHz ≤ f ≤ 1133 MHz 2627 MHz ≤ f ≤ 2697 MHz 2913 MHz ≤ f ≤ 3043 MHz 3404 MHz ≤ f ≤ 3484 MHz 4477 MHz ≤ f ≤ 4607 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_13A_n66A Configuration |  |  |  |
| 136 MHz ≤ f ≤ 226 MHz 923 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1003 MHz 2487 MHz ≤ f ≤ 2567 MHz 2633 MHz ≤ f ≤ 2783 MHz 3264 MHz ≤ f ≤ 3354 MHz 4197 MHz ≤ f ≤ 4347 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_13A_n77A Configuration |  |  |  |
| 1726 MHz ≤ f ≤ 3423 MHz 4077 MHz ≤ f ≤ 5774 MHz 5813 MHz ≤ f ≤ 9187 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_14A_n2A Configuration |  |  |  |
| 254 MHz ≤ f ≤ 334 MHz | -36 dBm | 100 kHz |  |
| 1052 MHz ≤ f ≤ 1122 MHz 2638 MHz ≤ f ≤ 2708 MHz 2902 MHz ≤ f ≤ 3032 MHz 3426 MHz ≤ f ≤ 3506 MHz 4488 MHz ≤ f ≤ 4618 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_14A_n66A Configuration |  |  |  |
| 114 MHz ≤ f ≤ 204 MHz 912 MHz ≤ f ≤ 992 MHz | -36 dBm | 100 kHz |  |
| 2498 MHz ≤ f ≤ 2578 MHz 2622 MHz ≤ f ≤ 2772 MHz 3286 MHz ≤ f ≤ 3376 MHz 4208 MHz ≤ f ≤ 4358 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_14A_n77A Configuration |  |  |  |
| 1704 MHz ≤ f ≤ 3412 MHz 4088 MHz ≤ f ≤ 5796 MHz 5808 MHz ≤ f ≤ 9198 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_18A_n77A Configuration |  |  |  |
| 1640 MHz ≤ f ≤ 3385 MHz 4115 MHz ≤ f ≤ 5860 MHz 5770 MHz ≤ f ≤ 9230 MHz | -30dBm | 1MHz |  |
| Test requirements for DC_18A_n78A Configuration |  |  |  |
| 1640 MHz ≤ f ≤ 2170 MHz 2470 MHz ≤ f ≤ 2985 MHz 4115 MHz ≤ f ≤ 4630 MHz 4930 MHz ≤ f ≤ 5460 MHz 5770 MHz ≤ f ≤ 6785 MHz 7415 MHz ≤ f ≤ 8430 MHz | -30dBm | 1MHz |  |
| Test requirements for DC_19A_n1A Configuration |  |  |  |
| 230 MHz <= f <= 320 MHz | -36 dBm | 100 kHz |  |
| 1075 MHz <= f <= 1150 MHz 2750 MHz <= f <= 2825 MHz 2995 MHz <= f <= 3130 MHz 3580 MHz <= f <= 3670 MHz 4670 MHz <= f <= 4805 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_19A_n77A Configuration |  |  |  |
| 1610 MHz ≤ f ≤ 3370 MHz 4130 MHz ≤ f ≤ 5890 MHz 5755 MHz ≤ f ≤ 9245 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_19A_n78A Configuration |  |  |  |
| 1610 MHz ≤ f ≤ 2140 MHz 2455 MHz ≤ f ≤ 2970 MHz 4130 MHz ≤ f ≤ 4645 MHz 4960 MHz ≤ f ≤ 5490 MHz 5755 MHz ≤ f ≤ 6770 MHz 7430 MHz ≤ f ≤ 8445 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_19A_n79A Configuration |  |  |  |
| 2710 MHz ≤ f ≤ 3340 MHz 3555 MHz ≤ f ≤ 4170 MHz 5230 MHz ≤ f ≤ 5845 MHz 6060 MHz ≤ f ≤ 6690 MHz 7955 MHz ≤ f ≤ 9170 MHz 9630 MHz ≤ f ≤ 10845 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_20A_n1A Configuration |  |  |  |
| 196 MHz ≤ f ≤ 316 MHz | -36 dBm | 100 kHz |  |
| 1058 MHz ≤ f ≤ 1148 MHz 2752 MHz ≤ f ≤ 2842 MHz 2978 MHz ≤ f ≤ 3128 MHz 3584 MHz ≤ f ≤ 3704 MHz 4672 MHz ≤ f ≤ 4822 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_20A_n3A Configuration |  |  |  |
| 14 MHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 121 MHz 848 MHz ≤ f ≤ 953 MHz | -36 dBm | 100 kHz |  |
| 2542 MHz ≤ f ≤ 2738 MHz 3374 MHz ≤ f ≤ 3509 MHz 4252 MHz ≤ f ≤ 4432 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_20A_n7A Configuration |  |  |  |
| 776 MHz ≤ f ≤ 906 MHz | -36 dBm | 100 kHz |  |
| 1638 MHz ≤ f ≤ 1738 MHz 3332 MHz ≤ f ≤ 3432 MHz 4138 MHz ≤ f ≤ 4308 MHz 5832 MHz ≤ f ≤ 6002 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_20A_n8A Configuration |  |  |  |
| 18 MHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 83 MHz 749 MHz ≤ f ≤ 844 MHz 898 MHz ≤ f ≤ 998 MHz | -36 dBm | 100 kHz |  |
| 1712 MHz ≤ f ≤ 1777 MHz 2544 MHz ≤ f ≤ 2692 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_20A_n28A Configuration |  |  |  |
| 84 MHz ≤ f ≤ 159 MHz 544 MHz ≤ f ≤ 664 MHz 916 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1021 MHz 1535 MHz ≤ f ≤ 1610 MHz 2238 MHz ≤ f ≤ 2358 MHz 2367 MHz ≤ f ≤ 2472 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_20A_n78A Configuration |  |  |  |
| 1576 MHz ≤ f ≤ 2136 MHz 2438 MHz ≤ f ≤ 2968 MHz 4132 MHz ≤ f ≤ 4662 MHz 4964 MHz ≤ f ≤ 5524 MHz 5738 MHz ≤ f ≤ 6768 MHz 7432 MHz ≤ f ≤ 8462 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_21A_n1A Configuration |  |  |  |
| 457.1 MHz ≤ f ≤ 532.1 MHz 915.8 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 2377.1 MHz ≤ f ≤ 2512.1 MHz 3367.9 MHz ≤ f ≤ 3442.9 MHz 4815.8 MHz ≤ f ≤ 4905.8 MHz 5287.9 MHz ≤ f ≤ 5422.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_21A_n28A Configuration |  |  |  |
| 6.9 MHz ≤ f ≤ 28.1 MHz | -36 dBm | 10 kHz |  |
| 709.9 MHz ≤ f ≤ 734.9 MHz | -36 dBm | 100 kHz |  |
| 2157.8 MHz ≤ f ≤ 2200.9 MHz 2903.9 MHz ≤ f ≤ 2938.9 MHz 3623.8 MHz ≤ f ≤ 3663.8 MHz | -30 dBm | 1 MHz | 4 |
| Test requirements for DC_21A_n77A Configuration |  |  |  |
| 374.2 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1304.2 MHz 1837.1 MHz ≤ f ≤ 2752.1 MHz 4747.9 MHz ≤ f ≤ 7125.8 MHz 8047.9 MHz ≤ f ≤ 9862.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_21A_n78A Configuration |  |  |  |
| 374.2 MHz ≤ f ≤ 904.2 MHz | -36 dBm | 100 kHz |  |
| 1837.1 MHz ≤ f ≤ 2352.1 MHz 4747.9 MHz ≤ f ≤ 6152.1 MHz 6195.8 MHz ≤ f ≤ 6725.8 MHz 8047.9 MHz ≤ f ≤ 9062.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_21A_n79A Configuration |  |  |  |
| 1474.2 MHz ≤ f ≤ 2104.2 MHz 2937.1 MHz ≤ f ≤ 3552.1 MHz 5847.9 MHz ≤ f ≤ 6462.9 MHz 7295.8 MHz ≤ f ≤ 8552.1 MHz 10247.9 MHz ≤ f ≤ 11462.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_25A_n41A Configuration |  |  |  |
| 581 MHz ≤ f ≤ 840 MHz | -36 dBm | 100 kHz |  |
| 1010 MHz ≤ f ≤ 1334 MHz 3077 MHz ≤ f ≤ 3530 MHz 4346 MHz ≤ f ≤ 4605 MHz 6196 MHz ≤ f ≤ 6520 MHz 6842 MHz ≤ f ≤ 7295 MHz | -30 dBm | 1 MHz | 2 |
|  | -25 dBm | 1 MHz | 1 |
| Test requirements for DC_26A_n41A Configuration |  |  |  |
| 798 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1062 MHz 1647 MHz ≤ f ≤ 1876 MHz 3310 MHz ≤ f ≤ 3539 MHz 4124 MHz ≤ f ≤ 4566 MHz 5806 MHz ≤ f ≤ 6229 MHz | -30 dBm | 1 MHz | 2 |
|  | -25 dBm | 1 MHz | 1 |
| Test requirements for DC_26A_n77A Configuration |  |  |  |
| 1602 MHz ≤ f ≤ 3386 MHz 4114 MHz ≤ f ≤ 5898 MHz 5751 MHz ≤ f ≤ 9249 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_26A_n78A Configuration |  |  |  |
| 1602 MHz ≤ f ≤ 2172 MHz 2451 MHz ≤ f ≤ 2986 MHz 4114 MHz ≤ f ≤ 4649 MHz 4928 MHz ≤ f ≤ 5498 MHz 5751 MHz ≤ f ≤ 6786 MHz 7414 MHz ≤ f ≤ 8449 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_26A_n79A Configuration |  |  |  |
| 2702 MHz ≤ f ≤ 3372 MHz 3551 MHz ≤ f ≤ 4186 MHz 5214 MHz ≤ f ≤ 5849 MHz 6028 MHz ≤ f ≤ 6698 MHz 7951 MHz ≤ f ≤ 9186 MHz 9614 MHz ≤ f ≤ 10849 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_28A_n3A Configuration |  |  |  |
| 214 MHz ≤ f ≤ 379 MHz 962 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1082 MHz 2413 MHz ≤ f ≤ 2533 MHz 2672 MHz ≤ f ≤ 2867 MHz 3116 MHz ≤ f ≤ 3281 MHz 4123 MHz ≤ f ≤ 4318 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_28A_n5A Configuration |  |  |  |
| 76 MHz ≤ f ≤ 146 MHz 557 MHz ≤ f ≤ 672 MHz 900 MHz ≤ f ≤ 995 MHz | -36 dBm | 100 kHz |  |
| 1527 MHz ≤ f ≤ 1597 MHz 2230 MHz ≤ f ≤ 2345 MHz 2351 MHz ≤ f ≤ 2446 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_28A_n7A Configuration |  |  |  |
| 1004 MHz ≤ f ≤ 1164 MHz 1752 MHz ≤ f ≤ 1867 MHz 3203 MHz ≤ f ≤ 3318 MHz 3906 MHz ≤ f ≤ 4066 MHz 4252 MHz ≤ f ≤ 4437 MHz 5703 MHz ≤ f ≤ 5888 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_28A_n77A Configuration |  |  |  |
| 1804 MHz ≤ f ≤ 3497 MHz 4003 MHz ≤ f ≤ 5696 MHz 5852 MHz ≤ f ≤ 9148 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_28A_n78A Configuration |  |  |  |
| 1804 MHz ≤ f ≤ 2394 MHz 2552 MHz ≤ f ≤ 3097 MHz 4003 MHz ≤ f ≤ 4548 MHz 4706 MHz ≤ f ≤ 5296 MHz 5852 MHz ≤ f ≤ 6897 MHz 7303 MHz ≤ f ≤ 8348 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_28A_n79A Configuration |  |  |  |
| 2904 MHz ≤ f ≤ 3594 MHz 3652 MHz ≤ f ≤ 4297 MHz 5103 MHz ≤ f ≤ 5748 MHz 5806 MHz ≤ f ≤ 6496 MHz 8052 MHz ≤ f ≤ 9297 MHz 9503 MHz ≤ f ≤ 10748 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_30A_n2A Configuration |  |  |  |
| 395 MHz ≤ f < 465 MHz | -36 dBm | 100 kHz |  |
| 1385 MHz ≤ f ≤ 1515 MHz 2700 MHz ≤ f ≤ 2780 MHz 4155 MHz ≤ f ≤ 4225 MHz 6005 MHz ≤ f ≤ 6135 MHz 6460 MHz ≤ f ≤ 6540 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_30A_n5A Configuration |  |  |  |
| 607 MHz ≤ f ≤ 667 MHz | -36 dBm | 100 kHz |  |
| 1456 MHz ≤ f ≤ 1491 MHz 3129 MHz ≤ f ≤ 3164 MHz 3761 MHz ≤ f ≤ 3806 MHz 3953 MHz ≤ f ≤ 4013 MHz 5434 MHz ≤ f ≤ 5479 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_30A_n77A Configuration |  |  |  |
| 410 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1895 MHz 4285 MHz ≤ f ≤ 5615 MHz 7910 MHz ≤ f ≤ 8830 MHz 8905 MHz ≤ f ≤ 10715 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_39A_n41A Configuration |  |  |  |
| 576 MHz ≤ f < 810 MHz | -36 dBm | 100 kHz |  |
| 1070MHz ≤ f ≤ 1344 MHz 3072MHz ≤ f ≤ 3500 MHz 4376MHz ≤ f ≤ 4610 MHz 6256MHz ≤ f ≤ 6530 MHz 6872MHz ≤ f ≤ 7300 MHz | -30 dBm | 1 MHz | 2 |
|  | -25 dBm | 1 MHz | 1 |
| Test requirements for DC_39A_n79A Configuration |  |  |  |
| 560 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1240 MHz 2480 MHz ≤ f ≤ 3120 MHz 6280 MHz ≤ f ≤ 8120 MHz 8160 MHz ≤ f ≤ 8840 MHz 10680 MHz ≤ f ≤ 11920 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_40A_n1A Configuration |  |  |  |
| 320 MHz ≤ f ≤ 480 MHz | -36 dBm | 100 kHz |  |
| 1440 MHz ≤ f ≤ 1660 MHz 2620 MHz ≤ f ≤ 2880 MHz 4220 MHz ≤ f ≤ 4380 MHz 6140 MHz ≤ f ≤ 6360 MHz 6520 MHz ≤ f ≤ 6780 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_40A_n41A Configuration |  |  |  |
| 96 MHz ≤ f < 390 MHz | -36 dBm | 100 kHz |  |
| 1910 MHz ≤ f ≤ 2304 MHz 2592 MHz ≤ f ≤ 3080 MHz 4796 MHz ≤ f ≤ 5090 MHz 7096 MHz ≤ f ≤ 7780 MHz | -30 dBm | 1 MHz | 2 |
|  | -25 dBm | 1 MHz | 1 |
| Test requirements for DC_40A_n78A Configuration |  |  |  |
| 800 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1500 MHz 4200 MHz ≤ f ≤ 5300 MHz 5600 MHz ≤ f ≤ 6200 MHz 7900 MHz ≤ f ≤ 8600 MHz 8900 MHz ≤ f ≤ 10000 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_40A_n79A Configuration |  |  |  |
| 2000 MHz ≤ f ≤ 2700 MHz 6400 MHz ≤ f ≤ 7700 MHz 9000 MHz ≤ f ≤ 9800 MHz 11100 MHz ≤ f ≤ 12400 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_41A_n28A Configuration |  |  |  |
| 1000 MHz ≤ f ≤ 1284 MHz 1748 MHz ≤ f ≤ 1987 MHz 3199 MHz ≤ f ≤ 3438 MHz 3902 MHz ≤ f ≤ 4186 MHz 4244 MHz ≤ f ≤ 4677 MHz 5695 MHz ≤ f ≤ 6128 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_41A_n77A Configuration |  |  |  |
| 610 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 2080 MHz 3910 MHz ≤ f ≤ 6890 MHz 8292 MHz ≤ f ≤ 11090 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_41A_n78A Configuration |  |  |  |
| 610 MHz ≤ f ≤ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 2080 MHz 3910 MHz ≤ f ≤ 5104 MHz 5796 MHz ≤ f ≤ 6490 MHz 8292 MHz ≤ f ≤ 10290 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_41A_n79A Configuration |  |  |  |
| 8 MHz ≤ f ≤ 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 980 MHz | -36 dBm | 100 kHz |  |
| 1710 MHz ≤ f ≤ 2504 MHz 6110 MHz ≤ f ≤ 7690 MHz 9392 MHz ≤ f ≤ 10380 MHz 11296 MHz ≤ f ≤ 12690 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_42A_n77A Configuration |  |  |  |
| 300 MHz ≤ f ≤ 800 MHz | -36 dBm | 100 kHz |  |
| 2600 MHz ≤ f ≤ 5000 MHz 6700 MHz ≤ f ≤ 7800 MHz 10000 MHz ≤ f ≤ 12000 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_48A_n5A Configuration |  |  |  |
| 1852 MHz ≤ f ≤ 2052 MHz 2701 MHz ≤ f ≤ 2876 MHz 4374 MHz ≤ f ≤ 4549 MHz 5198 MHz ≤ f ≤ 5398 MHz 6251 MHz ≤ f ≤ 6576 MHz 7924 MHz ≤ f ≤ 8249 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_48A_n66A Configuration |  |  |  |
| 10 MHz ≤ f < 30 MHz | -36 dBm | 10 KHz |  |
| 30 MHz ≤ f ≤ 280 MHz | -36 dBm | 100 KHz |  |
| 1770 MHz ≤ f ≤ 1990 MHz 5260 MHz ≤ f ≤ 5690 MHz 6970 MHz ≤ f ≤ 7260 MHz 8810 MHz ≤ f ≤ 9180 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_66A_n5A Configuration |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f ≤ 132 MHz 861 MHz ≤ f ≤ 956 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz ≤ f ≤ 2736 MHz 3358 MHz ≤ f ≤ 3478 MHz 4244 MHz ≤ f ≤ 4409 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_66A_n41A Configuration |  |  |  |
| 716 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1064 MHz 3212 MHz ≤ f ≤ 3670 MHz 4206 MHz ≤ f ≤ 4470 MHz 5916 MHz ≤ f ≤ 6250 MHz 6702 MHz ≤ f ≤ 7160 MHz | -30 dBm | 1 MHz | 2 |
|  | -25 dBm | 1 MHz | 1 |
| Test requirements for DC_66A_n71A Configuration |  |  |  |
| 314 MHz ≤ f ≤ 454 MHz | -36 dBm | 100 kHz |  |
| 1102 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_66A_n77A Configuration |  |  |  |
| 260MHz ≤ f ≤ 780 MHz | -36 dBm | 100 kHz |  |
| 1520 MHz ≤ f ≤ 2490MHz 5010 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7760 MHz 8310 MHz ≤ f ≤ 10180 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_66A_n78A Configuration |  |  |  |
| 260 MHz ≤ f ≤ 380 MHz | -36 dBm | 100 kHz |  |
| 1520 MHz ≤ f ≤ 2090 MHz 4820 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7360 MHz 8310 MHz ≤ f ≤ 9380 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_71A_n2A Configuration |  |  |  |
| 454 MHz ≤ f ≤ 584 MHz | -36 dBm | 100 kHz |  |
| 1152 MHz ≤ f ≤ 1247 MHz 2513 MHz ≤ f ≤ 2608 MHz 3002 MHz ≤ f ≤ 3157 MHz 3176 MHz ≤ f ≤ 3306 MHz 4363 MHz ≤ f ≤ 4518 MHz | -30 dBm | 1 MHz |  |
| Test requirements for DC_71A_n66A Configuration |  |  |  |
| 314 MHz ≤ f ≤ 454 MHz | -36 dBm | 100 kHz |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz | -30 dBm | 1 MHz |  |
| NOTE 1: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 36.101 [5] when NS_04 is signalled. NOTE 2: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |

##### 6.5B.3.3.2	Spurious emission band UE co-existence for Inter-band within FR1

Editor's note:	The default and additional test configuration is analysed based on the assumption that only intermodulation products need to be tested. The band combinations without TP analysis in TR 38.905 are incomplete.

6.5B.3.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions for band UE co-existence for inter-band EN-DC.

6.5B.3.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward supporting inter-band EN-DC.

6.5B.3.3.2.3	Minimum conformance requirements

This clause specifies the additional requirements for the specified uplink EN-DC, for coexistence with protected bands with the single CC uplink assigned to E-UTRA and NR bands for the specified uplink carrier aggregation configurations in Table 6.5B.3.3.2-1. The requirements in Tables 6.5B.3.3.2.3-1 to 6.5B.3.3.2.3-4 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the requirements in Tables 6.5B.3.3.2.3-1 to 6.5B.3.3.2.3-4 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5B.3.3.2.3-1 to 6.5B.3.3.2.3-4 and in 6.5.3.2 of [2] and clause 6.6.3.2 of [4] would be considered to be verified by the measurements verifying the uplink single carrier UE to UE co-existence requirements.

Table 6.5B.3.3.2.3-1: Spurious emission band UE co-existence limits Rel-15

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band n3, n34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_2_n5 | E-UTRA Bands 4, 5, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 41, 43, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n66 | E-UTRA Bands 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Bands 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band n71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2_n78 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n7 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n8 | E-UTRA Band 1, 11, 20, 21, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67,68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n28 | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_3_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2,7 |
| DC_7_n28 | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 42, 43, 50, 65, 66, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_8_n77 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n78 | E-UTRA Band 1, 20, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_11_n77 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n79 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_12_n66 | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 4, 48, 50, 51, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_19_n77 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20_n28 | E-UTRA Band 3, 7, 8, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n78 | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | FDL_low |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_21_n77 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n79 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_25_n41 | E-UTRA band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 66, 70,71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA/NR Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_26_n41 | E-UTRA/NR Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n78 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 |  | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n79 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n77 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_30_n5 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_38_n78 | N/A |  |  |  |  |  |  |
| DC_39_n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| DC_41_n77 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n78 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 40, 42, 44, 45, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_42_n77 | N/A |  |  |  |  |  |  |
| DC_42_n78 | N/A |  |  |  |  |  |  |
| DC_42_n79 | N/A |  |  |  |  |  |  |
| DC_66_n5 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n71 | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7,22, 25, 41, 42, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_66_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 of TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: Void NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.3-2: Spurious emission band UE co-existence limits Rel-16

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n3 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n7 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_1_n8 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n77 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n78 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n79 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_3_n1 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_3_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n7 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n28 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_3_n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_7_n1 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7,16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7,16 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n3 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n5 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 14 |
| DC_7_n8 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n28 | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n66 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n78 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_8_n1 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_8_n3 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
| DC_8_n20 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_8_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9, 12 |
| DC_8_n41 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_8_n77 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n78 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_11_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_12_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_13_n2 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n66 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 803 | -35 | 0.00625 | 5 |
| DC_14_n2 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_14_n66 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_19_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20_n1 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_20_n3 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_21_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n41 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n78 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n79 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n3 | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| DC_28_n7 | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_28_n77 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_28_n79 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_39_n41 | Frequency range | 1805 | - | 1855 | -40 | 1 | 5 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |
| DC_39_n79 | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| DC_40_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_40_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n77 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n78 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_48_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.3-3: Spurious emission band UE co-existence limits Rel-17

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n3 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 67 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n7 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_1_n8 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n41 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 8, 20 |
| DC_1_n77 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n78 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n79 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_2_n7 | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n1 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_3_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| rrrrrrrrrrrrr | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n8 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n28 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_3_n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n7 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 14 |
| DC_5_n30 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_7_n1 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7,16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7,16 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n2 | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n3 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n5 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 14 |
| DC_7_n8 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n28 | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n66 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n71 | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n77 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n78 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n80 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_8_n1 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_8_n3 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3.12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5. 12 |
| DC_8_n20 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_8_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9, 12 |
| DC_8_n41 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_8_n77 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n78 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_11_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_12_n7 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_12_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_13_n2 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n5 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n7 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_13_n48 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n66 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 803 | -35 | 0.00625 | 5 |
| DC_13_n71 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n77 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_14_n2 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_14_n30 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_14_n77 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_14_n66 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_18_n3 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_18_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n1 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20_n1 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_20_n3 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_20A_91A_ULSUP-TDM, DC_20A_92A_ULSUP-TDM | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_21_n1 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| DC_21_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n41 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n78 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n79 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n3 | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| DC_28_n7 | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n77 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n79 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_39_n40 | Frequency range | 1805 |  | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 5, 7, 18 |
| DC_39_n41 | Frequency range | 1805 | - | 1855 | -40 | 1 | 5 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |
| DC_39_n79 | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| DC_40_n1 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_40_n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_40_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_40_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_41_n77 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_42_n1 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_42_n3 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_48_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_66_n7 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_66_n38 | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 22 |

Table 6.5B.3.3.2.3-4: Spurious emission band UE co-existence limits Rel-18

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n3 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 67 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n7 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_1_n8 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n41 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 8, 20 |
| DC_1_n77 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n78 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n79 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_2_n7 | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n1 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_3_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n7 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n8 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n28 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_3_n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n7 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 14 |
| DC_5_n30 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_7_n1 | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7,16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7,16 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n2 | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n3 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n5 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 14 |
| DC_7_n8 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n28 | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n66 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n71 | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n77 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n78 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n80 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_8_n1 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_8_n3 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3.12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5. 12 |
| DC_8_n20 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_8_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9, 12 |
| DC_8_n41 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_8_n77 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n78 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_11_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_12_n7 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_12_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_13_n2 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n5 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n7 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_13_n48 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n66 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 803 | -35 | 0.00625 | 5 |
| DC_13_n71 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n77 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_14_n2 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_14_n30 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_14_n77 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_14_n66 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_18_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_18_n3 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_18_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n1 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20_n1 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_20_n3 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_20A_91A_ULSUP-TDM, DC_20A_92A_ULSUP-TDM | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_21_n1 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| DC_21_n77 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n79 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n41 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n78 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n79 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n3 | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| DC_28_n7 | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n77 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n79 | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_39_n40 | Frequency range | 1805 |  | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 5, 7, 18 |
| DC_39_n41 | Frequency range | 1805 | - | 1855 | -40 | 1 | 5 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |
| DC_39_n79 | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| DC_40_n1 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_40_n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_40_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_40_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n28 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_41_n77 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_42_n1 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_42_n3 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_48_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_66_n7 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_66_n38 | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 22 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [4] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 19: Void NOTE 20: Void. NOTE 21: Void NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

NOTE:	To simplify the above Tables, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.3.2.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.5B.3.3.2.4	Test description

6.5B.3.3.2.4.1	Initial conditions

Same initial conditions as described in clause 6.5B.3.1.2.4.1 with the following exceptions:

-	Instead of Table 6.5B.3.1.2.4.1-1 --> use Table 6.5B.3.3.2.4.1-1.

Table 6.5B.3.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | DC Configuration / NRB_agg |  |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 1,2) |  |  |  |
|  | DC Configuration |  |  |  | E-UTRA Ch BW/NRB | NR Ch BW/NRB |  | CC MOD E-UTRA/NR |  | E-UTRA & NR RB allocation |  |  | CC MOD E-UTRA/NR | E-UTRA & NR allocations (LCRB @ RBstart) |  |
|  | E-UTRA |  | NR |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low | nY | Low | Highest Ch BW /Highest NRB | Highest Ch BW /Highest NRB |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | X | High | nY | High | Highest Ch BW /Highest NRB | Highest Ch BW /Highest NRB |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@RBmax | 1@RBmax |
| Test Setting for DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | n3 | Low | 10/50 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 1 | High | n3 | Mid | 10/50 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| Test Setting for DC_1A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 1 | High | n5 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| Test Setting for DC_1A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | High | n7 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 2 | 1 | High | n7 | Mid | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@14 |
| Test Setting for DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | n8 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_1A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | n28 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 1 | Low | n28 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 1 | High | n28 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| Test Setting for DC_1A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@250 |
| 2 | 1 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 3 | 1 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 4 | 1 | High | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@247 |
| 5 | 1 | Hiqh | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 6 | 1 | High | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@85 |
| 7 | 1 | High | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| Test Setting for DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | High | n77 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| 2 | 1 | Low | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 3 | 1 | Low | n77 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 4 | 1 | Low | n77 | NOTE 18 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 1 | Low | n77 | NOTE 18 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 1 | Low | n77 | NOTE 18 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 7 | 1 | Low | n77 | NOTE 18 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 8 | 1 | Low | n77 | NOTE 18 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | High | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 2 | 1 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 3 | 1 | Low | n78 | Note 8 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_1A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | n79 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@172 |
| 2 | 1 | High | n79 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 3 | 1 | High | n79 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 4 | 1 | Low | n79 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@201 |
| 5 | 1 | High | n79 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| Test Setting for DC_2A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 2 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@30 |
| 3 | 2 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@62 |
| 4 | 2 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@209 |
| 5 | 2 | High | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 6 | 2 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_2A_n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | High | n48 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@88 |
| Test Setting for DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | n66 | Low | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 2 | High | n66 | High | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@215 |
| 3 | 2 | High | n66 | Low | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 4 | 2 | High | n66 | Low | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 2 | High | n66 | High | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@215 |
| Test Setting for DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | n71 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | n77 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 2 | High | n77 | Note 27 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@200 |
| 3 | 2 | HIgh | n77 | Note 27 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 4 | 2 | Low | n77 | Note 27 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@200 |
| Test Setting for DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 2 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 3 | 2 | High | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 4 | 2 | Low | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| Test Settings for DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | n1 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 3 | Low | n1 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| Test Setting for DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@77 |
| 2 | 3 | High | n5 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 3 | Low | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 4 | 3 | Low | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 3 | High | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| Test Setting for DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | n7 | High | 20/100 | 10/52 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@47 | 1@51 |
| 2 | 3 | Mid | n7 | High | 20/100 | 10/52 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@51 |
| 3 | 3 | Low | n7 | Low | 20/100 | 10/52 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@51 |
| 4 | 3 | Mid | n7 | Low | 20/100 | 10/52 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 5 | 3 | Low | n7 | High | 20/100 | 10/52 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@51 |
| Test Setting for DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | n8 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 3 | High | n8 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 3 | Low | n8 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| Test Settings for DC_3A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | n28 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 3 | Low | n28 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| 3 | 3 | High | n28 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| Test Settings for DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 3 | High | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 3 | 3 | Mid | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 4 | 3 | Note 4 | n41 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| Test Setting for DC_3A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | n77 | NOTE 19 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 3 | Low | n77 | NOTE 19 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 3 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 4 | 3 | Low | n77 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@75 |
| 5 | 3 | Low | n77 | NOTE 19 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 3 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 7 | 3 | High | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| Test Setting for DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | High | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 2 | 3 | High | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 3 | 3 | Low | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| Test Settings for DC_3A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | High | n79 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 2 | 3 | High | n79 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 3 | 3 | Low | n79 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 4 | 3 | High | n79 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@134 |
| 5 | 3 | Low | n79 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@66 |
| Test Settings for DC_5A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 | Low | n2 | Low | 10/50 | 20/106 |  | QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 5 | High | n2 | High | 10/50 | 20/106 |  | QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@105 |
| Test Settings for DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 | High | n66 | Mid | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| 2 | 5 | High | n66 | High | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@215 |
| 3 | 5 | Low | n66 | Low | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@215 |
| 4 | 5 | High | n66 | Low | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| Test Setting for DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 | High | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| 2 | 5 | Low | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 3 | 5 | Low | n77 | Note 28 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 4 | 5 | Low | n77 | Note 28 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 5 | Low | n77 | Note 28 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 5 | Low | n77 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 7 | 5 | High | n77 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 8 | 5 | High | n77 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@140 |
| Test Setting for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 | Low | n78 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 5 | Low | n78 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 3 | 5 | Low | n78 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 4 | 5 | High | n78 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@29 |
| 5 | 5 | Low | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| Test Setting for DC_7A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Low | n1 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 7 | Low | n1 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| 3 | 7 | Mid | n1 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@22 | 1@105 |
| Test Setting for DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | High | n3 | Low | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@77 |
| 2 | 7 | Low | n3 | Low | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@59 |
| 3 | 7 | High | n3 | High | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@86 |
| 4 | 7 | High | n3 | Low | 10/50 | 5/25 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| 5 | 7 | High | n3 | Low | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 6 | 7 | Low | n3 | Mid | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@66 |
| Test Setting for DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | High | n5 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 2 | 7 | Low | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_7A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | High | n8 | Mid | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 2 | 7 | High | n8 | Mid | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 7 | Low | n8 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_7A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | High | n28 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| Test Setting for DC_7A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Low | n66 | High | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@215 |
| 2 | 7 | Low | n66 | High | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 7 | Mid | n66 | High | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@50 | 1@215 |
| 4 | 7 | Low | n66 | High | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@215 |
| 5 | 7 | Mid | n66 | Low | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 6 | 7 | High | n66 | Low | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| Test Setting for DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 7 | Mid | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 3 | 7 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@196 |
| 4 | 7 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 5 | 7 | High | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@77 | 1@0 |
| 6 | 7 | Mid | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 7 | 7 | High | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 8 | 7 | High | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 9 | 7 | High | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 10 | 7 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@165 |
| Test Settings for DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Low | n1 | Low | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Settings for DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | High | n3 | Low | 10/50 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| 2 | 8 | Low | n3 | Low | 10/50 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@159 |
| 3 | 8 | High | n3 | High | 10/50 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@159 |
| Test Settings for DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | High | n20 | Low | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@12 |
| 2 | 8 | High | n20 | High | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@105 |
| 3 | 8 | Low | n20 | Low | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Settings for DC_8A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | High | n28 | High | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@105 |
| 2 | 8 | Low | n28 | Low | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| Test Settings for DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Low | n41 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 8 | Low | n41 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 3 | 8 | Low | n41 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 4 | 8 | High | n41 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 5 | 8 | Low | n41 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@155 |
| Test Settings for DC_8A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Low | n77 | Note 9 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 8 | High | n77 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@37 |
| 3 | 8 | High | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@97 |
| 4 | 8 | Low | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@211 |
| 5 | 8 | Low | n77 | Note 9 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 8 | High | n77 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 7 | 8 | High | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| 8 | 8 | Low | n77 | Note 9 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 9 | 8 | High | n77 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@85 |
| 10 | 8 | Low | n77 | Note 9 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| Test Settings for DC_8A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | High | n78 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 2 | 8 | High | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 8 | Low | n78 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 4 | 8 | High | n78 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 8 | High | n78 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| 6 | 8 | Mid | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@25 | 1@0 |
| Test Settings for DC_11A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 11 | Low | n77 | Note 10 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 11 | Low | n77 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 11 | Low | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@63 |
| 4 | 11 | Low | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@104 |
| 5 | 11 | Low | n77 | Note 10 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 11 | Low | n77 | Note 10 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 7 | 11 | Low | n77 | Note 10 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 8 | 11 | Low | n77 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@75 |
| 9 | 11 | Low | n77 | Note 10 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 10 | 11 | Low | n77 | Note 10 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Settings for DC_11A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 11 | Low | n78 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@184 |
| 2 | 11 | Low | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 11 | Low | n78 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@63 |
| 4 | 11 | Low | n78 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@104 |
| 5 | 11 | Low | n78 | Note 11 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 11 | Low | n78 | Note 11 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 7 | 11 | Low | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@75 |
| Test Settings for DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 11 | Low | n79 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 11 | Low | n79 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@132 |
| 3 | 11 | High | n79 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@30 |
| 4 | 11 | Low | n79 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| Test Setting for DC_12A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 | HIGH | n2 | HIGH | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@105 |
| Test Setting for DC_12A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 | Low | n66 | Low | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 12 | High | n66 | High | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@215 |
| 3 | 12 | Low | n66 | High | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@215 |
| Test Setting for DC_12A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 | Low | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 12 | High | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 3 | 12 | Low | n78 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@136 |
| Test Setting for DC_13A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 13 | Low | n2 | Low | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 13 | Mid | n77 | Note 29 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 2 | 13 | Mid | n77 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@0 |
| 3 | 13 | Mid | n77 | Note 29 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@210 |
| 4 | 13 | Low | n77 | Note 29 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 13 | Low | n77 | Note 29 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 13 | High | n77 | HIGH | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 7 | 13 | Low | n77 | LOW | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_14A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 | Low | n2 | Low | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_14A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 | Low | n66 | Low | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 14 | High | n66 | High | 10/50 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@215 |
| Test Setting for DC_19A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | Low | n1 | Low | 15/75 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_19A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | Low | n77 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 19 | Low | n77 | Note 26 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 19 | High | n77 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@0 |
| 4 | 19 | Low | n77 | Note 26 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 19 | Low | n77 | Note 26 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 19 | Low | n77 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 7 | 19 | Low | n77 | Note 26 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_19A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | High | n78 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@272 |
| 2 | 19 | Low | n78 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 19 | High | n78 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@0 |
| 4 | 19 | Low | n78 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@165 |
| 5 | 19 | Low | n78 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 6 | 19 | Low | n78 | Note 17 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | Low | n79 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_20A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | n1 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 20 | High | n1 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| Test Setting for DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | n3 | Low | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 20 | High | n3 | High | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@159 |
| 3 | 20 | Low | n3 | High | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@159 |
| Test Setting for DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | High | n7 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 2 | 20 | High | n7 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 20 | Low | n7 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 4 | 20 | High | n7 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | n8 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 20 | High | n8 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 20 | Low | n8 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| Test Setting for DC_20A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | n28 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| Test Setting for DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 20 | Low | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 20 | Low | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@205 |
| 4 | 20 | High | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@104 |
| 5 | 20 | Low | n78 | Note 24 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_21A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Low | n1 | Low | 15/75 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 21 | High | n1 | High | 15/75 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@105 |
| 3 | 21 | Low | n1 | Low | 15/75 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| Test Setting for DC_21A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Low | n28 | Low | 15/75 | 5/25 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_21A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Low | n77 | Note 21 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 21 | Low | n77 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 21 | Low | n77 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@174 |
| 4 | 21 | Low | n77 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@215 |
| 5 | 21 | High | n77 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@0 |
| 6 | 21 | Low | n77 | Note 21 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 7 | 21 | Low | n77 | Note 21 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 8 | 21 | Low | n77 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@130 |
| 9 | 21 | Low | n77 | Note 21 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 10 | 21 | High | n77 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@0 |
| Test Setting for DC_21A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Low | n78 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 21 | Low | n78 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 21 | High | n78 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@272 |
| 4 | 21 | Low | n78 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@215 |
| 5 | 21 | High | n78 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@0 |
| 6 | 21 | Low | n78 | Note 20 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 7 | 21 | Low | n78 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@130 |
| Test Setting for DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Low | n79 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 21 | Low | n79 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 21 | Low | n79 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@38 |
| 4 | 21 | Low | n79 | Note 22 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_25A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 25 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 25 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@30 |
| 3 | 25 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@62 |
| 4 | 25 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@209 |
| 5 | 25 | High | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 6 | 25 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_25A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 25 | Low | n77 | Note 34 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@200 |
| Test Setting for DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 | High | n41 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@272 |
| 2 | 26 | Low | n41 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@12 |
| 3 | 26 | Low | n41 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@41 |
| 4 | 26 | High | n41 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@0 |
| 5 | 26 | Low | n41 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@232 |
| 6 | 26 | High | n41 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@25 |
| Test Setting for DC_26A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 | Low | n77 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 26 | Low | n77 | NOTE 25 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 26 | High | n77 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@34 |
| 4 | 26 | Low | n77 | NOTE 25 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 26 | Low | n77 | NOTE 25 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 26 | Low | n77 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@201 |
| 7 | 26 | Low | n77 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 8 | 26 | Low | n77 | NOTE 25 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 | Low | n78 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 26 | Low | n78 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 26 | High | n78 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@74 | 1@34 |
| 4 | 26 | Low | n78 | Mid | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@78 |
| 5 | 26 | Low | n78 | Low | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@201 |
| 6 | 26 | Low | n78 | Note 13 | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A (Note 14) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Setting for DC_28A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | Low | n3 | Low | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 28 | High | n3 | High | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@159 |
| 3 | 28 | High | n3 | Low | 20/100 | 30/160 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| Test Setting for DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | High | n5 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 2 | 28 | Low | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 28 | High | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| Test Setting for DC_28A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | High | n7 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| Test Setting for DC_28A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | High | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 2 | 28 | Low | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@158 |
| 3 | 28 | High | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@130 |
| 4 | 28 | Low | n77 | Note 23 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 28 | Low | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 28 | Low | n77 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 7 | 28 | Low | n77 | Note 23 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_28A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | Low | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 28 | High | n78 | High | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 3 | 28 | Mid | n78 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@272 |
| 4 | 28 | High | n78 | Low | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@190 |
| 5 | 28 | High | n78 | Mid | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_28A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | High | n79 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| Test Setting for DC_30A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 30 | High | n5 | High | 10/50 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@105 |
| Test Setting for DC_30A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 30 | High | n77 | Note 35 | 10/50 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@100 |
| Test Settings for DC_39A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 39 | High | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| Test Settings for DC_39A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 39 | Low | n79 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@146 |
| 2 | 39 | Low | n79 | Note 5 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Settings for DC_40A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 40 | Low | n1 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 40 | High | n1 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 40 | High | n1 | Mid | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@56 |
| 4 | 40 | High | n1 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@77 |
| 5 | 40 | High | n1 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@39 |
| Test Settings for DC_40A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 40 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 40 | High | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 3 | 40 | Mid | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@50 | 1@272 |
| 4 | 40 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| Test Settings for DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 40 | Low | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@79 |
| 2 | 40 | High | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@110 |
| 3 | 40 | Low | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 4 | 40 | Low | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@234 |
| 5 | 40 | High | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@29 |
| 6 | 40 | Low | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@50 |
| 7 | 40 | Low | n78 | High | 15/75 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 8 | 40 | High | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 9 | 40 | Low | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@110 |
| 10 | 40 | Low | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@155 |
| 11 | 40 | High | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@183 |
| 12 | 40 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| Test Settings for DC_40A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 40 | Low | n79 | Low | 20/100 | 50/270 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@269 |
| 2 | 40 | High | n79 | Low | 20/100 | 50/270 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 40 | High | n79 | High | 20/100 | 50/270 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@269 |
| Test Settings for DC_41A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | High | n28 | Low | 20/100 | 15/79 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 41 | Note 33 | n28 | High | 5/25 | 15/79 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@78 |
| 3 | 41 | Mid | n28 | Low | 10/50 | 15/79 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@49 | 1@78 |
| Test Settings for DC_41A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | Mid | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 41 | Low | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@185 |
| 3 | 41 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@57 |
| 4 | 41 | Low | n77 | Note 15 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 41 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 6 | 41 | High | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@201 |
| 7 | 41 | Mid | n77 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@46 |
| 8 | 41 | Low | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@214 |
| 9 | 41 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@29 |
| 10 | 41 | High | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 11 | 41 | Mid | n77 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 12 | 41 | Low | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 13 | 41 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 14 | 41 | Mid | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 15 | 41 | High | n77 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@161 |
| 16 | 41 | High | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@122 |
| 17 | 41 | Mid | n77 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@27 |
| 18 | 41 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@240 |
| 19 | 41 | Mid | n77 | Note 15 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@125 |
| Test Settings for DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | High | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 2 | 41 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@185 |
| 3 | 41 | High | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 4 | 41 | Low | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 5 | 41 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@214 |
| 6 | 41 | Low | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@69 |
| 7 | 41 | Low | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 8 | 41 | High | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@161 |
| 9 | 41 | Mid | n78 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 10 | 41 | Low | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@98 |
| Test Settings for DC_41A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | Low | n79 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 41 | High | n79 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 3 | 41 | High | n79 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@188 |
| 4 | 41 | High | n79 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@93 |
| 5 | 41 | Mid | n79 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 6 | 41 | High | n79 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@151 |
| 7 | 41 | High | n79 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 8 | 41 | Low | n79 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Setting for DC_42A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A (Note 14) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_48A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 48 | High | n5 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| 2 | 48 | High | n5 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 48 | Low | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Settings for DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 48 | High | n66 | Low | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Test Settings for DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low | n2 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 66 | High | n2 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 66 | Low | n2 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| 4 | 66 | Low | n2 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@105 |
| 5 | 66 | Low | n2 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@46 |
| Test Settings for DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low | n5 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 2 | 66 | High | n5 | High | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@105 |
| 3 | 66 | Low | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 4 | 66 | High | n5 | Low | 20/100 | 20/106 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| Test Settings for DC_66A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 2 | 66 | Low | n41 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| 3 | 66 | High | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@59 |
| 4 | 66 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@248 |
| 5 | 66 | High | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@89 |
| 6 | 66 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@218 |
| 7 | 66 | High | n41 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| 8 | 66 | Low | n41 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@186 |
| 9 | 66 | Low | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@203 |
| 10 | 66 | High | n41 | Low | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@0 |
| Test Settings for DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A (Note 14) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Setting for DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | High | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 2 | 66 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@110 |
| 3 | 66 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@166 |
| 4 | 66 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@220 |
| 5 | 66 | High | n77 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@100 |
| 6 | 66 | High | n77 | Note 30 | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| 7 | 66 | High | n77 | HIgh | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@50 |
| 8 | 66 | Low | n77 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@196 |
| 9 | 66 | Low | n77 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@47 |
| Test Settings for DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@143 |
| 2 | 66 | High | n78 | High | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@99 | 1@272 |
| 3 | 66 | Low | n78 | Mid | 20/100 | 100/273 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@272 |
| Test Settings for DC_71A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 71 | HIGH | n2 | HIGH | 20/100 | 20/106 | CP-OFDM QPSK |  | NA |  | CP-OFDM QPSK |  |  | 1@99 | 1@105 |
| Test Settings for DC_71A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 71 | Low | n66 | Low | 20/100 | 40/216 |  | QPSK/CP-OFDM QPSK |  | NA |  | QPSK / CP-OFDM QPSK |  | 1@0 | 1@0 |
| Note 1: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 2: X, nY correspond to the different bands in the DC Configuration. E.g. for DC_1A_n3A, X=1, nY=n3. Note 3: Void. Note 4: Test Point ID 4 for DC_3A_n41A have the centre carrier frequency of 1773 MHz in Band 3 (EARFCN=19830). Note 5: Test Point ID 4 for DC_39A_n79A has the centre carrier frequency of 4649.96 MHz in Band 79 (NR ARFCN=709998). Note 6: Void. Note 7: Void. Note 8: Test Point ID 3 for DC_1A_n78A has the centre carrier frequency of 3473.43 MHz in Band 78 (NR ARFCN=631562). Note 9: Test Point ID 1 for DC_8A_n77A has the centre carrier frequency of 3950.13 MHz in Band n77 (NR ARFCN=663342). Test Point ID 5 has the centre carrier frequency of 3524.55 MHz in Band n77 (NR ARFCN=634970). Test Point ID 8 has the centre carrier frequency of 3584.55 MHz in Band n77 (NR ARFCN=638970). Test Point ID 10 has centre carrier frequency of 3500 MHz in Band n77 (NR ARFCN=633333). Note 10: Test Point ID 1 for DC_11A_n77A has the centre carrier frequency of 3617.43 MHz in Band n77 (NR ARFCN=641162). Test Point ID 5 has the centre carrier frequency of 3686.43 MHz in Band n77 (NR ARFCN=645762). Test Point ID 6 has the centre carrier frequency of 3494.94 MHz in Band n77 (NR ARFCN=632996). Test Point ID 7 has the centre carrier frequency of 3858.42 MHz in Band n77 (NR ARFCN=657228). Test Point ID 9 has the centre carrier frequency of 4037.43 MHz in Band n77 (NR ARFCN=669162). Test Point ID 10 has the centre carrier frequency of 4097.43 MHz in Band n77 (NR ARFCN=673162). Note 11: Test Point ID 5 for DC_11A_n78A has the centre carrier frequency of 3686.43 MHz in Band n78 (NR ARFCN=645762). Test Point ID 6 has the centre carrier frequency of 3494.94 MHz in Band n78 (NR ARFCN=632996). Note 12: Void. Note 13: Test Point ID 6 for DC_26A_n78A has the centre carrier frequency of 3483.78 MHz in Band n78 (NR ARFCN=632252). Note 14: Test case not applicable for the EN-DC configuration as no IM products occurs in the protected bands. For EN-DC only capable devices, the test requirements as in clause 6.5.3.2.5 in TS 38.521-1 [8] is needed. Note 15: Test Point ID 4 for DC_41A_n77A has the centre carrier frequency of 3488.55 MHz in Band n77 (NR ARFCN=632570). Test Point ID 19 has the centre carrier frequency of 3500 MHz in Band n77 (NR ARFCN=633333). Note 16: Void. Note 17: Test Point ID 6 for DC_19A_n78A has the centre carrier frequency of 3499.8 MHz in Band n78 (NR ARFCN=633320). Note 18: Test Point ID 4 for DC_1A_n77A has the centre carrier frequency of 3472.95 MHz in Band n77 (NR ARFCN=631530). Test Point ID 5 has the centre carrier frequency of 3987.03 MHz in Band n77 (NR ARFCN=665802). Test Point ID 6 has the centre carrier frequency of 3857.04 MHz in Band n77 (NR ARFCN=657136). Test Point ID 7 has the centre carrier frequency of 3874.53 MHz in Band n77 (NR ARFCN=658302). Test Point ID 8 has the centre carrier frequency of 3887.04 MHz in Band n77 (NR ARFCN=659136). Note 19: Test Point ID 1 for DC_3A_n77A has the centre carrier frequency of 3900.03 MHz in Band n77 (NR ARFCN=660002). Test Point ID 2 has the centre carrier frequency of 3602.55 MHz in Band n77 (NR ARFCN=640170). Test Point ID 5 has the centre carrier frequency of 3660.15 MHz in Band n77 (NR ARFCN=644010). Note 20: Test Point ID 6 for DC_21A_n78A has the centre carrier frequency of 3515.19 MHz in Band n78 (NR ARFCN=634346). Note 21: Test Point ID 1 for DC_21A_n77A has the centre carrier frequency of 3637.68 MHz in Band n77 (NR ARFCN=642512). Test Point ID 6 has the centre carrier frequency of 3515.19 MHz in Band n77 (NR ARFCN=634346). Test Point ID 7 has the centre carrier frequency of 3898.92 MHz in Band n77 (NR ARFCN=659928). Test Point ID 9 has the centre carrier frequency of 4057.68 MHz in Band n77 (NR ARFCN=670512). Note 22: Test Point ID 4 for DC_21A_n79A has the centre carrier frequency of 4846.53 MHz in Band n79 (NR ARFCN=723102). Note 23: Test Point ID 4 for DC_28A_n77A has the centre carrier frequency of 3474,63 MHz in Band n77 (NR ARFCN=631642). Test Point ID 7 for have the centre carrier frequency of 3597,12 MHz in Band n77 (NR ARFCN=639808). Note 24: Test Point ID 5 for DC_20A_n78A has the centre carrier frequency of 3477.03 MHz in Band n78 (NR ARFCN=631802). Note 25: Test Point ID 2 for DC_26A_n77A has the centre carrier frequency of 3521.13 MHz in Band n77 (NR ARFCN=634742). Test Point ID 4 has the centre carrier frequency of 3578.73 MHz in Band n77 (NR ARFCN=638582). Test Point ID 5 has the centre carrier frequency of 4028.64 MHz in Band n77 (NR ARFCN=668576). Test Point ID 8 has the centre carrier frequency of 3483.78 MHz in Band n77 (NR ARFCN=632252). Note 26: Test Point ID 2 for DC_19A_n77A has the centre carrier frequency of 3553.14 MHz in Band n77 (NR ARFCN=636876). Test Point ID 4 has the centre carrier frequency of 4060.62 MHz in Band n77 (NR ARFCN=670708). Test Point ID 5 has the centre carrier frequency of 3610.74 MHz in Band n77 (NR ARFCN=640716). Test Point ID 7 has the centre carrier frequency of 3499.8 MHz in Band n77 (NR ARFCN=633320). Note 27: Test Point ID 2 for DC_2A_n77A has the centre carrier frequency of 3900 MHz in Band n77 (NR ARFCN=660000). Test Point ID 3 has the centre carrier frequency of 4000 MHz in Band n77 (NR ARFCN=666667). Test Point ID 4 has the centre frequency of 3500 MHz in Band n77 (NR ARFCN=633333). Note 28: Test Point ID 3 for DC_5A_n77A has the centre carrier frequency of 4000 MHz in Band n77 (NR ARFCN=666667). Test Point ID 4 has the centre carrier frequency of 3600 MHz in Band n77 (NR ARFCN=640000). Test Point ID 5 has the centre carrier frequency of 3658.14 MHz in Band n77 (NR ARFCN=643876). Note 29: Test Point ID 1 for DC_13A_n77A has the centre carrier frequency of 3500 MHz in Band n77 (NR ARFCN=633333). Test Point ID 3 has the centre carrier frequency of 3880 MHz in Band n77 (NR ARFCN=658668). Test Point ID 4 has the centre carrier frequency of 3611.64 MHz in Band n77 (NR ARFCN=640776). Test Point ID 5 has the centre carrier frequency of 3504.24 MHz in Band n77 (NR ARFCN=633616). Note 30: Test Point ID 6 for DC_66A_n77A has the centre carrier frequency of 3900 MHz in Band n77 (NR ARFCN=660000). Note 31: Test Point ID 2 for DC_18A_n77A have the centre carrier frequency of 3441,39 MHz in Band n77 (NR ARFCN=649400). Test Point ID 3 and 5 for DC_18A_n77A have the centre carrier frequency of 3602,55 MHz in Band n77 (NR ARFCN=640170). Test Point ID 4 for DC_18A_n77A have the centre carrier frequency of 4049,76 MHz in Band n77 (NR ARFCN=669984). Note 32: Test Point ID 2 for DC_18A_n78A have the centre carrier frequency of 3441,39 MHz in Band n78 (NR ARFCN=629400). Test Point ID 3 for DC_18A_n78A have the centre carrier frequency of 3602,55 MHz in Band n78 (NR ARFCN=640170). Note 33: Test Point ID 2 for DC_41A_n28 A and Band 41 is defined as a carrier with CBW=5MHz and centre frequency at 2660.1MHz (NR ARFCN=532020). Note 34: Test Point ID 1 for DC_25A_n77A has the centre frequency of 3500 MHz in Band n77 (NR ARFCN=633333) Note 35: Test Point ID 1 for DC_30A_n77A has the centre carrier frequency of 4000 MHz in Band n77 (NR ARFCN=666667). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Additional step 9 when both bands are TDD:

8.	For both E-UTRA and NR UL uplink carriers active when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.3.3.2.4.2	Test Procedure

Same test procedure as described in clause 6.5B.3.1.2.4.2 with the following exceptions:

Instead of Table 6.5B.3.1.2.3-1 --> use Table 6.5B.3.3.2.5-1.

In addition to test configurations above, EN-DC only capable UEs and UEs not supporting NR/5GC mode in the tested band needs to be tested according to LTE anchor agnostic approach below.

Same test description as in clause 6.5.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.3.3.2.4.3	Message Contents

Message contents are according to TS 36.508 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.3.3.2.4.3-1: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | Sa2 |  |  |
| specialSubframePatterns | Ssp7 |  |  |
| } |  |  |  |

Table 6.5B3.3.2.4.3-1a: Void

Table 6.5B.3.3.2.4.3-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.3.3.2.4.3-3: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

6.5B.3.3.2.5	Test Requirement

Test requirements for Spurious Emissions UE Co-existence when both LTE and NR carriers are required to be active are release specific and can be found in Table 6.5B.3.3.2.5-1 If the UE support a band combination from a later release, the requirements for this band combination are taken from the table of earliest release where requirements for this band combination are defined.

Test requirements for EN-DC only capable devices when using LTE anchor agnostic approach can be found in clause 6.5.3.2.5 in TS 38.521-1 [8].

For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the test requirements in Table 6.5B.3.3.2.5-1 can be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.2.5-1: Requirements for inter-band within FR1 according to UE NR release

| EN-DC Configuration | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n28 | Rel-15 | E-UTRA Band 7, 31, 41, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-15 Rel-16 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Rel-17 Rel-18 | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
| DC_1_n77 | Rel-15 | E-UTRA Band 1, 3, 11, 21, 34, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  |  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  |  | Frequency range | 1915 | - | 1920 | 1.6 | 5 | 5, 7, 8 |
| DC_1_n78 | Rel-15 | E-UTRA Band 3, 11, 21, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
| DC_1_n79 | Rel-15 | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_2_n5 | Rel-15 | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n66 | Rel-15 | E-UTRA Band 4, 10, 24, 50, 66, 70, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  |  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n71 | Rel-15 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n78 | Rel-15 | E-UTRA Band 24, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n7 | Rel-15 | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n8 | Rel-15 | E-UTRA Band 20, 28, 38, 69 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 7, 22, 41, 42, 43 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n28 | Rel-15 | NR band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 7, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_3_n77 | Rel-15 | E-UTRA Band 1, 3, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 | Rel-15 | E-UTRA Band 3, 34, 39 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 | Rel-15 | E-UTRA Band 5, 8, 11, 18, 19, 21, 41, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_5_n66 | Rel-15 | E-UTRA Band 5, 6, 7, 8, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  |  | E-UTRA Band 41, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5_n77 | Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n78 | Rel-15 | E-UTRA Band 1, 2, 3, 4, 7, 25, 34, 38, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2, 7 |
| DC_7_n28 | Rel-15 | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_7_n78 | Rel-15 | E-UTRA Band 3, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 50, 51, 67, 68, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_8_n3 | Rel-16 Rel-17 Rel-18 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
| DC_8_n20 | Rel-16 Rel-17 Rel-18 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_8_n28 | Rel-16 Rel-17 Rel-18 | Frequency range | 491 | - | 616 | -42 | 8 | 5, 17 |
| DC_8_n41 | Rel-16 Rel-17 Rel-18 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
| DC_8_n77 | Rel-15 | E-UTRA Band 1, 32, 33, 34, 38, 39, 40,50, 65, 69, 74, 75 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n78 | Rel-15 | E-UTRA band 34, 39, 40, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_11_n77 | Rel-15 r | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | Rel-15 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_11_n79 | Rel-15 | E-UTRA Band 1, 3, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_12_n66 | Rel-15 | E-UTRA Band 41,53 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_12_n78 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_19_n77 | Rel-15 | E-UTRA Band 1, 3, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Rel-17 Rel-18 | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | Rel-15 | E-UTRA Band 1, 3, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Rel-17 Rel-18 | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | Rel-15 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_21_n77 | Rel-15 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | Rel-15 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_21_n79 | Rel-15 | E-UTRA Band 3, 21, 34, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_25_n41 | Rel-15 | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_26_n41 | Rel-15 | E-UTRA Band 3, 5, 18, 19, 26, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  |  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | Rel-15 | E-UTRA Band 1, 3, 34, 39, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n5 | Rel-16 | Frequency range | 557 | - | 672 | -42 | 8 | 5, 17 |
|  | Rel-17 Rel-18 | Frequency range | 557 | - | 672 | -26.2 | 6 | 14 |
|  |  | Frequency range | 662 | - | 672 | -26.2 | 6 | 5 |
| DC_26_n78 | Rel-16 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Rel-17 Rel-18 | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n77 | Rel-15 | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 | Rel-15 | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_28_n79 | Rel-15 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_30_n5 | Rel-15 | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_39_n41 | Rel-15 | E-UTRA Band 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_39_n79 | Rel-15 | E-UTRA Band 8, 28, 41, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_40_n41 | Rel-15 | Bands 1, 34, 39, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |
| DC_41_n28 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 1 | 3, 9 |
| DC_41_n77 | Rel-15 | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n78 | Rel-15 | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | Rel-15 | E-UTRA Band 1, 3, 5, 8, 18, 19, 26, 28, 34, 40, 44, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_48_n5 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_66_n5 | Rel-15 | E-UTRA Band 5, 6, 7, 8, 26, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n78 | Rel-15 | E-UTRA Band 3, 34, 39 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 of TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 – 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink. NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB: B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.5-2: Void

Table 6.5B.3.3.2.5-3: Void

#### 6.5B.3.3a	Inter-band NE-DC within FR1

##### 6.5B.3.3a.1	General Spurious Emissions for Inter-band NE-DC within FR1

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 6.5.3.1 and 6.5A.3.1 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.6.3.1 and 6.6.3.1A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

##### 6.5B.3.3a.2	Spurious emission band UE co-existence for Inter-band NE-DC within FR1

FFS

#### 6.5B.3.4	Spurious Emissions for Inter-band including FR2

Editor's note:	This clause is complete for Band n257, n258, n260 and n261. The following aspects are either missing or not yet determined:

-	The referred test case 6.5.3 in TS 38.521-2 [9] is incomplete for frequency above 80 GHz.

##### 6.5B.3.4.1	General Spurious Emissions for Inter-band including FR2 (1 NR CC)

Editor’s Note:

-	This clause is complete for Band n257, n258, n260 and n261 and for power class 1 and 3.

-		For n259, measurement uncertainty is TBD in the range 80 GHz < f <= 87 GHz. Up to 80GHz, for n259 and PC3 this clause is complete.

6.5B.3.4.1.1	Test purpose

Same test purpose as in clause 6.5.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.3.4.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1.4	Test description

6.5B.3.4.1.4.1	Initial conditions

Same test description as in clause 6.5.3.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.3.1.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1.5	Test requirement

Same test requirement as in clause 6.5.3.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.3.4.1a	General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)

Editor’s note: This clause is complete for Band n257, n258, n260 and n261 for PC1 and PC3. The following aspects of the clause are for future consideration:

-	For n259, measurement uncertainty is TBD in the range 80 GHz < f <= 87 GHz. Up to 80GHz, for n259 and PC3 this clause is complete.

6.5B.3.4.1a.1	Test purpose

Same test purpose as in clause 6.5.3.1_1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1a.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.3.4.1a.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.1_1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1a.4	Test description

Same test description as in clause 6.5.3.1_1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2.4_1.4 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.3.1.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1a.5	Test requirement

Same test requirement as in clause 6.5.3.1_1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.3.4.1_1	General Spurious emissions for Inter-band EN-DC including FR2 (>1 NR CC)

###### 6.5B.3.4.1_1.1	General Spurious emissions for Inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.1.1 in TS 38.521-2 [9] is incomplete for:

▪ Bands other than n257, n258, n260 and n261

▪ Power classes 1, 2 and 4

6.5B.3.4.1_1.1.1	Test purpose

Same test purpose as in clause 6.5.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.3.4.1_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.3.4.

6.5B.3.4.1_1.1.4	Test description

6.5B.3.4.1_1.1.4.1	Initial condition

Same test description as in clause 6.5A.3.1.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1_1.1.5	Test Requirements

Same test requirement as in clause 6.5A.3.1.1.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.1_1.2	General Spurious emissions for Inter-band EN-DC including FR2 (3 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.1.2 in TS 38.521-2 [9] is incomplete for:

▪ Bands other than n257, n258, n260 and n261

▪ Power classes 1, 2 and 4

6.5B.3.4.1_1.2.1	Test purpose

Same test purpose as in clause 6.5.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.5B.3.4.1_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.3.4.

6.5B.3.4.1_1.2.4	Test description

6.5B.3.4.1_1.2.4.1	Initial condition

Same test description as in clause 6.5A.3.1.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.1.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.1.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.1.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1_1.2.5	Test Requirements

Same test requirement as in clause 6.5A.3.1.2.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.1_1.3	General Spurious emissions for Inter-band EN-DC including FR2 (4 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.1.3 in TS 38.521-2 [9] is incomplete for:

▪ Bands other than n257, n258, n260 and n261

▪ Power classes 1, 2 and 4

6.5B.3.4.1_1.3.1	Test purpose

Same test purpose as in clause 6.5.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.5B.3.4.1_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.3.4.

6.5B.3.4.1_1.3.4	Test description

6.5B.3.4.1_1.3.4.1	Initial condition

Same test description as in clause 6.5A.3.1.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.1.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.1.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.1.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1_1.3.5	Test Requirements

Same test requirement as in clause 6.5A.3.1.3.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.1_1.4	General Spurious emissions for Inter-band EN-DC including FR2 (5 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.1.4 in TS 38.521-2 [9] is incomplete for:

▪ Bands other than n257, n258, n260 and n261

▪ Power classes 1, 2 and 4

-	Measurement Uncertainties and Test Tolerances is incomplete.

6.5B.3.4.1_1.4.1	Test purpose

Same test purpose as in clause 6.5.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 NR UL CCs.

6.5B.3.4.1_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1_1.4.4	Test description

6.5B.3.4.1_1.4.4.1	Initial condition

Same test description as in clause 6.5A.3.1.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1_1.4.5	Test Requirements

Same test requirement as in clause 6.5A.3.1.4.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.1_1.5	General Spurious emissions for Inter-band EN-DC including FR2 (6 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.1.5 in TS 38.521-2 [9] is incomplete for:

▪ Bands other than n257, n258, n260 and n261

▪ Power classes 1, 2 and 4

-	Measurement Uncertainties and Test Tolerances is incomplete.

6.5B.3.4.1_1.5.1	Test purpose

Same test purpose as in clause 6.5.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 6 NR UL CCs.

6.5B.3.4.1_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1_1.5.4	Test description

6.5B.3.4.1_1.5.4.1	Initial condition

Same test description as in clause 6.5A.3.1.5.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1_1.5.5	Test Requirements

Same test requirement as in clause 6.5A.3.1.5.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.1_1.6	General Spurious emissions for Inter-band EN-DC including FR2 (7 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.1.6 in TS 38.521-2 [9] is incomplete for:

▪ Bands other than n257, n258, n260 and n261

▪ Power classes 1, 2 and 4

-	Measurement Uncertainties and Test Tolerances is incomplete.

6.5B.3.4.1_1.6.1	Test purpose

Same test purpose as in clause 6.5.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 7 NR UL CCs.

6.5B.3.4.1_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1_1.6.4	Test description

6.5B.3.4.1_1.6.4.1	Initial condition

Same test description as in clause 6.5A.3.1.6.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1_1.6.5	Test Requirements

Same test requirement as in clause 6.5A.3.1.6.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.1_1.7	General Spurious emissions for Inter-band EN-DC including FR2 (8 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.1.7 in TS 38.521-2 [9] is incomplete for:

▪ Bands other than n257, n258, n260 and n261

▪ Power classes 1, 2 and 4

-	Measurement Uncertainties and Test Tolerances is incomplete.

6.5B.3.4.1_1.7.1	Test purpose

Same test purpose as in clause 6.5.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 8 NR UL CCs.

6.5B.3.4.1_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1_1.7.4	Test description

6.5B.3.4.1_1.7.4.1	Initial condition

Same test description as in clause 6.5A.3.1.7.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.1.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.1.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1_1.7.5	Test Requirements

Same test requirement as in clause 6.5A.3.1.7.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.3.4.1D	General Spurious Emissions for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5D.3 in TS 38.521-2 [9] is incomplete

6.5B.3.4.1D.1	Test purpose

Same test purpose as in clause 6.5D.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC FR2.

6.5B.3.4.1D.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5D.3.1 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1D.4	Test Description

Same test description as in clause 6.5D.3.1 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 6.5D.3.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5D.3.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1D.5	Test Requirement

Same test requirement as specified in clause 6.5D.3.1.5 of TS 38.521-2 [9] for the NR carrier(s).

##### 6.5B.3.4.2	Spurious emission band UE co-existence for Inter-band including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5.3.2 in TS 38.521-2 [9] is incomplete for PC2 and PC4.

6.5B.3.4.2.1	Test purpose

Same test purpose as in clause 6.5.3.2.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.3.4.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.1.

6.5B.3.4.2.4	Test description

Same Test description as in clause 6.5.3.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.3.2.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2.5	Test requirement

Same Test requirement as in clause 6.5.3.2.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.3.4.2a	Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5.3.2_1 in TS 38.521-2 [9] is incomplete for PC2 and PC4.

6.5B.3.4.2a.1	Test purpose

Same test purpose as in clause 6.5.3.2_1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2a.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.3.4.2a.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.2_1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.1.

6.5B.3.4.2a.4	Test description

Same Test description as in clause 6.5.3.2_1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2.4_1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.3.2_1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2a.5	Test requirement

Same Test requirement as in clause 6.5.3.2_1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.3.4.2_1	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (>1 NR CC)

###### 6.5B.3.4.2_1.1	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.2.1 in TS 38.521-2 [9] is incomplete for:

▪ Testability and relaxation of the requirement for Bands other than n257, n258, n260 and n261

▪ MU and TT for Power classes 1, 2 and 4

6.5B.3.4.2_1.1.1	Test purpose

Same test purpose as in clause 6.5.3.2 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.3.4.2_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.2 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.3.2.

6.5B.3.4.2_1.1.4	Test description

6.5B.3.4.2_1.1.4.1	Initial condition

Same test description as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2_1.1.5	Test Requirements

Same test requirement as in clause 6.5A.3.2.1.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.2_1.2	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.2.2 in TS 38.521-2 [9] is incomplete for:

▪ Testability and relaxation of the requirement for Bands other than n257, n258, n260 and n261

▪ MU and TT for Power classes 1, 2 and 4

6.5B.3.4.2_1.2.1	Test purpose

Same test purpose as in clause 6.5.3.2 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.5B.3.4.2_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.3.4.

6.5B.3.4.2_1.2.4	Test description

6.5B.3.4.2_1.2.4.1	Initial condition

Same test description as in clause 6.5A.3.2.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.2.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.2.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.2.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2_1.2.5	Test Requirements

Same test requirement as in clause 6.5A.3.2.2.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.2_1.3	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.2.3 in TS 38.521-2 [9] is incomplete for:

▪ Testability and relaxation of the requirement for Bands other than n257, n258, n260 and n261

▪ MU and TT for Power classes 1, 2 and 4

6.5B.3.4.2_1.3.1	Test purpose

Same test purpose as in clause 6.5.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR UL CCs.

6.5B.3.4.2_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.3.4.

6.5B.3.4.2_1.3.4	Test description

6.5B.3.4.2_1.3.4.1	Initial condition

Same test description as in clause 6.5A.3.2.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.2.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.2.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.2.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2_1.3.5	Test Requirements

Same test requirement as in clause 6.5A.3.2.3.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.2_1.4	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (5 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.2.4 in TS 38.521-2 [9] is incomplete for:

-	Testability and relaxation of the requirement for Bands other than n257, n258, n260 and n261

-	MU and TT for Power classes 1, 2 and 4

-	Annex F MU, TT tables need to add new rows to include these test cases

6.5B.3.4.2_1.4.1	Test purpose

Same test purpose as in clause 6.5.3.2 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 NR UL CCs.

6.5B.3.4.2_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.2.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.1.

6.5B.3.4.2_1.4.4	Test description

6.5B.3.4.2_1.4.4.1	Initial condition

Same test description as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2_1.4.5	Test Requirements

Same test requirement as in clause 6.5A.3.2.4.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.2_1.5	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (6 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.2.5 in TS 38.521-2 [9] is incomplete for:

-	Testability and relaxation of the requirement for Bands other than n257, n258, n260 and n261

-	MU and TT for Power classes 1, 2 and 4

-	Annex F MU, TT tables need to add new rows to include these test cases

6.5B.3.4.2_1.5.1	Test purpose

Same test purpose as in clause 6.5.3.2 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 6 NR UL CCs.

6.5B.3.4.2_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.2.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.1.

6.5B.3.4.2_1.5.4	Test description

6.5B.3.4.2_1.5.4.1	Initial condition

Same test description as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2_1.5.5	Test Requirements

Same test requirement as in clause 6.5A.3.2.5.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.2_1.6	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (6 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.2.6 in TS 38.521-2 [9] is incomplete for:

-	Testability and relaxation of the requirement for Bands other than n257, n258, n260 and n261

-	MU and TT for Power classes 1, 2 and 4

-	Annex F MU, TT tables need to add new rows to include these test cases

6.5B.3.4.2_1.6.1	Test purpose

Same test purpose as in clause 6.5.3.2 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 7 NR UL CCs.

6.5B.3.4.2_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.2.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.1.

6.5B.3.4.2_1.6.4	Test description

6.5B.3.4.2_1.6.4.1	Initial condition

Same test description as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2_1.6.5	Test Requirements

Same test requirement as in clause 6.5A.3.2.6.5 in TS 38.521-2 [9] for the NR carrier.

###### 6.5B.3.4.2_1.7	Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (8 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5A.3.2.7 in TS 38.521-2 [9] is incomplete for:

-	Testability and relaxation of the requirement for Bands other than n257, n258, n260 and n261

-	MU and TT for Power classes 1, 2 and 4

-	Annex F MU, TT tables need to add new rows to include these test cases

6.5B.3.4.2_1.7.1	Test purpose

Same test purpose as in clause 6.5.3.2 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 8 NR UL CCs.

6.5B.3.4.2_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.2.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.1.

6.5B.3.4.2_1.7.4	Test description

6.5B.3.4.2_1.7.4.1	Initial condition

Same test description as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.2.1.4 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.2_1.7.5	Test Requirements

Same test requirement as in clause 6.5A.3.2.7.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.3.4.2D	Spurious emission band UE co-existence for inter-band EN-DC including FR2 for UL MIMO

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.5D.3 in TS 38.521-2 [9] is incomplete

6.5B.3.4.2D.1	Test purpose

Same test purpose as in clause 6.5D.3.2 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.2D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC FR2.

6.5B.3.4.2D.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5D.3.2 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1D.4	Test Description

Same test description as in clause 6.5D.3.2 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 6.5D.3.2 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5D.3.2 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1D.5	Test Requirement

Same test requirement as specified in clause 6.5D.3.2.5 of TS 38.521-2 [9] for the NR carrier(s).

#### 6.5B.3.5	Spurious emissions for Inter-band including FR1 and FR2

##### 6.5B.3.5.1	General Spurious Emissions for Inter-band including FR1 and FR2

6.5B.3.5.1.1	Test purpose

Same test purpose as in clause 6.5.3 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.5.3 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.5B.3.5.1.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for spurious emissions apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 6.5B.3.

##### 6.5B.3.5.2	Spurious emission band UE co-existence for Inter-band including FR1 and FR2

6.5B.3.5.2.1	Test purpose

Same test purpose as in clause 6.5.3 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 6.5.3 in TS 38.521-2 [9] for NR FR2 carrier(s).

6.5B.3.5.2.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for spurious emissions apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 6.5B.3.

### 6.5B.4	Additional Spurious Emissions for EN-DC

#### 6.5B.4.1	Additional Spurious Emissions for Intra-band contiguous EN-DC

6.5B.4.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5B.4.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward supporting intra-band contiguous EN-DC.

6.5B.4.1.3	Minimum conformance requirements

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5B.4.1.3.1	Minimum requirement (network signalled value "NS_04")

When "NS 04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.1.3.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 of TS 38.521-1 [8] from the edge of the channel bandwidth.

Table 6.5B.4.1.3.1-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2495 ≤ f < 2496 | -13 | 1% of Channel BW for contiguous BW up to 100 MHz, 1 MHz for contiguous BW > 100 MHz |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.1.1.

Exception requirements for both NR and E-UTRA are defined for this test. LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included, and E-UTRA measurements are performed.

6.5B.4.1.4	Test description

6.5B.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on EN-DC operating bands specified in 5.3B.1.2. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing are shown in Table 6.5B.4.1.4.1-1 for both E-UTRA and NR. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 for NR. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C and in TS 38.521-1 [8] Annex C2 for LTE link and NR link respectively.

Table 6.5B.4.1.4.1-1: EN-DC test configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  | Low range, High range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (Note 1) | Power config (Note 6) |
| 1 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 2 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 3 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 4 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 6 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 8 | Default |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 9 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 10 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 11 (Note 4) | Default |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 6: Power config as specified in Table 6.2B.3.1.4.3-1 to 6.2B.3.1.4.3-2 (PC3) or Table 6.2B.3.1.4.3-3 to 6.2B.3.1.4.3-4 (PC2). NOTE 7: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for SS diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	E-UTRA downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

5.	NR downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 of TS 38.521-1 [8].

6.	The UL Reference Measurement channels are set according to Table 6.5B.4.1.4.1-1.

7.	NR propagation conditions are set according to B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.4.1.4.3.

9.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.4.1.4.2	Test Procedure

1.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.4.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.4.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for both NR and E-UTRA carriers until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	Measure the mean power of each component carriers for the EN-DC configuration, which shall meet the requirements described in 6.2B.3.1.5.2 depending on NS-values. The period of the measurement shall be at least the continuous duration of one sub-frame.

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.4.1.3.1-1. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

6.5B.4.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [6] clause 4.6 with the following exceptions for each network signalled value.

6.5B.4.1.4.3.1	Message contents exceptions for network signalled value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.5B.4.1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [6] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

Table 6.5B.4.1.4.3.1-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.4.1.4.3.1-3: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

6.5B.4.1.5	Test Requirement

Test requirements for additional spurious emissions for intra-band contiguous EN-DC are the same as the minimum requirements described in clause 6.5B.4.1.3 and are not repeated in this clause.

#### 6.5B.4.2	Additional Spurious Emissions for Intra-band non-contiguous EN-DC

6.5B.4.2.1	Test purpose

Same minimum conformance requirements as in clause 6.5B.4.1.1.

6.5B.4.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.4.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5B.4.1.3.

6.5B.4.2.4	Test description

6.5B.4.2.4.1	Initial conditions

Same initial conditions as described in clause 6.5B.4.1.4.1 for both E-UTRA and NR carriers with the following exception:

1.	For each EN-DC combination specified in Table 5.3B.1.3-1, channel spacing between NR and E-UTRA is specified according to clause 5.4B.1.

2.	Set up the NR and E-UTRA test frequencies so that NR carrier is located at the lower frequency side as specified in Table 5.3B.1.3-1. Repeat each testing with E-UTRA carrier frequency is located at the lower side as specified in Table 5.3B.1.3-1.

3.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.4.2.4.2	Test Procedure

Same test procedure as described in clause 6.5B.4.1.4.2.

6.5B.4.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [6] clause 4.6 with the following exceptions for each network signalled value.

6.5B.4.2.4.3.1 	Message contents exceptions for network signalled value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.5B.4.2.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [6] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.5B.4.2.5	Test Requirement

Test requirements for Spurious Emissions for intra-band non-contiguous EN-DC are the same as the minimum requirements described in 6.5B.4.2.3 and are not repeated in this clause.

#### 6.5B.4.3	Additional Spurious Emissions for Inter-band EN-DC within FR1 (1 NR CC)

6.5B.4.3.1	Test purpose

Same test purpose as in clause 6.5.3.3.1 in TS 38.521-1 [8] for the NR carrier.

6.5B.4.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward supporting inter-band EN-DC within FR1 with 1 NR UL.

6.5B.4.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.4.3.4	Test description

Same test description as in clause 6.5.3.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5.3.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and 	propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.3.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.5.3.3.4.2 in TS 38.521-1 [8].

6.5B.4.3.5	Test Requirement

Same test requirement as in clause 6.5.3.3.5 in TS 38.521-1 [8] for the NR carrier.

#### 6.5B.4.4	Additional Spurious Emissions for Inter-band including FR2 (1 NR CC)

Editor's note:	This clause is complete for Band n257 and n258 and power class 3. The following aspects are either missing or not yet determined:

-	The referred test case 6.5.3.3 in TS 38.521-2 [9] is incomplete for frequency above 80 GHz.

6.5B.4.4.1	Test purpose

Same test purpose as in clause 6.5.3.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.4.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4.4	Test description

Same test description as in clause 6.5.3.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.5.3.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.3.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4.5	Test requirement

Same test requirement as in clause 6.5.3.3.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.4.4a	Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)

Editor's note:	This clause is complete for Band n257 and n258 and PC3. The following aspects are either missing or not yet determined:

-	The referred test case 6.5.3.3_1 in TS 38.521-2 [9] is incomplete for frequency above 80 GHz.

6.5B.4.4a.1	Test purpose

Same test purpose as in clause 6.5.3.3_1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4a.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.4.4a.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.3_1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4a.4	Test description

Same test description as in clause 6.5.3.3_1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.5.3.3_1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2.4_1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.3.3_1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4a.5	Test requirement

Same test requirement as in clause 6.5.3.3_1.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.5B.4.4_1	Additional Spurious Emissions for Inter-band including FR2 (>1 NR CC)

##### 6.5B.4.4_1.1	Additional Spurious Emissions for Inter-band including FR2 (2 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Test procedure only includes the testing of smartphone and is FFS for laptop and FWA.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5B.4.4_1.1.1	Test purpose

Same test purpose as in clause 6.5A.3.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.4.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4_1.1.4	Test description

6.5B.4.4_1.1.4.1	Initial condition

Same test description as in clause 6.5A.3.3.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.3.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.3.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.3.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4_1.1.5	Test Requirements

Same test requirement as in clause 6.5A.3.3.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.4.4_1.2	Additional Spurious Emissions for Inter-band including FR2 (3 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Test procedure only includes the testing of smartphone and is FFS for laptop and FWA.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5B.4.4_1.2.1	Test purpose

Same test purpose as in clause 6.5A.3.3.2.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.5B.4.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4_1.2.4	Test description

6.5B.4.4_1.2.4.1	Initial condition

Same test description as in clause 6.5A.3.3.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.3.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.3.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4_1.2.5	Test Requirements

Same test requirement as in clause 6.5A.3.3.2.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.4.4_1.3	Additional Spurious Emissions for Inter-band including FR2 (4 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Test procedure only includes the testing of smartphone and is FFS for laptop and FWA.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5B.4.4_1.3.1	Test purpose

Same test purpose as in clause 6.5A.3.3.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.4.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4_1.3.4	Test description

6.5B.4.4_1.3.4.1	Initial condition

Same test description as in clause 6.5A.3.3.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.3.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.3.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.3.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4_1.3.5	Test Requirements

Same test requirement as in clause 6.5A.3.3.3.5 in TS 38.521-2 [9] for the NR carrier.

### 6.5B.5	Transmit intermodulation

### 6.5B.5.1	Transmit intermodulation for Intra-band contiguous EN-DC

No test case details are specified. No transmit intermodulation requirements are applied for intra band contiguous EN DC unless otherwise stated as in clause TS 38.101-3 [4] clause 6.5B.5.1.

#### 6.5B.5.2	Transmit intermodulation for Intra-band non-contiguous EN-DC

No test case details are specified. No transmit intermodulation requirements are applied for intra band contiguous EN DC unless otherwise stated as in clause TS 38.101-3 [4] clause 6.5B.5.2.

#### 6.5B.5.3	Transmit intermodulation for Inter-band EN-DC within FR1 (1 NR CC)

6.5B.5.3.1	Test purpose

Same test purpose as in clause 6.5.4 in TS 38.521-1 [8] for the NR carrier.

6.5B.5.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL.

6.5B.5.3.3	Minimum conformance requirements

The transmit intermodulation requirement specified in clauses 6.7.1 and 6.7.1A of TS 36.101 [5] and clauses 6.5.4 and 6.5A.4 of TS 38.101-1 [2] apply for each component carrier in E-UTRA bands and NR bands, respectively.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this measurement is TS 38.101-3 [4] clause 6.5B.5.3.

6.5B.5.3.4	Test description

Same test description as in clause 6.5.4.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.4.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.5.4.4.2 in TS 38.521-1 [8].

6.5B.5.3.5	Test Requirement

The ratio derived in step 6 and 8, shall not exceed the described value in Table 6.5.4.5-1 defined in TS 38.521-1 [8].

#### 6.5B.5.3a	Transmit Intermodulation for Inter-band NE-DC within FR1

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 6.5.4 and 6.5A.4 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 6.7 and 6.7A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

#### 6.5B.5.4	Transmit intermodulation for Inter-band EN-DC including FR2

#### 6.5B.5.5	Transmit intermodulation for Inter-band EN-DC including both FR1 and FR2

## 6.5E	Output RF spectrum emissions for V2X operation in FR1

### 6.5E.1	Occupied bandwidth for V2X

#### 6.5E.1.0	Minimum conformance requirements

For intra-band V2X, the occupied bandwidth specified in clause 6.6.1G in TS 36.101 [5] and specified in clause 6.5E.1 in TS 38.101-1 [2] apply for each frequency range respectively.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.6.1 of TS 36.101 [5] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.5E.1 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.1.

#### 6.5E.1.1	Occupied bandwidth for intra-band V2X

6.5E.1.1.1	Test purpose

Same test purpose as in clause 6.5E.1 in TS 38.521-1 [8].

6.5E.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.5E.1.0. The requirements in this test case can be well covered in clause 6.6.1G of TS 36.521-1 [10] and clause 6.5E.1 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.5E.1.2	Occupied bandwidth for inter-band V2X

6.5E.1.2.1	Test purpose

Same test purpose as in clause 6.5E.1 in TS 38.521-1 [8].

6.5E.1.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.6.1 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.5E.1 of TS 38.521-1 [8] and don’t need to be tested again.

### 6.5E.2	Out of band emission for V2X

#### 6.5E.2.0	Minimum conformance requirements

For intra-band V2X, out-of-band emissions specified in clause 6.6.2G in TS 36.101 [5] and specified in clause 6.5E.2 in TS 38.101-1 [2] apply for each frequency range respectively.

For the inter-band con-current NR V2X operation, the general SEM/additional SEM requirements and ACLR specified in subclause 6.6.2 of TS 36.101 [5] shall apply for the E-UTRA uplink in licensed band and the general SEM/additional SEM and ACLR requirements specified in subclause 6.5E.2 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.2.

#### 6.5E.2.1	Out of band emission for intra-band V2X

6.5E.2.1.1	Test purpose

Same test purpose as in clause 6.5E.2 in TS 38.521-1 [8].

6.5E.2.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.5E.2.0. The requirements in this test case can be well covered in clause 6.6.2G of TS 36.521-1 [10] and clause 6.5E.2 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.5E.2.2	Out of band emission for inter-band V2X

6.5E.2.2.1	Test purpose

Same test purpose as in clause 6.5E.2 in TS 38.521-1 [8].

6.5E.2.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.6.2 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.5E.2 of TS 38.521-1 [8] and don’t need to be tested again.

### 6.5E.3	Spurious emissions for V2X

#### 6.5E.3.1	General spurious emissions for V2X

6.5E.3.1.0	Minimum conformance requirements

For intra-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [5] and clause 6.5E.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

For inter-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [5] and clause 6.5E.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.3.

##### 6.5E.3.1.1	General spurious emissions for intra-band V2X

6.5E.3.1.1.1	Test purpose

Same test purpose as in clause 6.5E.3.1 in TS 38.521-1 [8].

6.5E.3.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.5E.3.1.0. The requirements in this test case can be well covered in clause 6.6.3.1 of TS 36.521-1 [10] and clause 6.5E.3.1 of TS 38.521-1 [8] and don’t need to be tested again.

##### 6.5E.3.1.2	General spurious emissions for inter-band V2X

6.5E.3.1.2.1	Test purpose

Same test purpose as in clause 6.5E.3.1 in TS 38.521-1 [8].

6.5E.3.1.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.6.3.1 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.5E.3.1 of TS 38.521-1 [8] and don’t need to be tested again.

### 6.5E.4	Transmit intermodulation for V2X

#### 6.5E.4.0	Minimum conformance requirements

For intra-band V2X, transmit intermodulation requirements specified in clause 6.7G.1 of TS 36.101 [5] and clause 6.5E.4 of TS 38.101-1 [2] apply for each frequency range respectively.

For inter-band con-current NR V2X operation, the requirements specified in subclause 6.7.1 of TS 36.101 [5] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.5E.4 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.4.

#### 6.5E.4.1	Transmit intermodulation for intra-band V2X

6.5E.4.1.1	Test purpose

Same test purpose as in clause 6.5E.4.1 in TS 38.521-1 [8].

6.5E.4.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.5E.4.1. The requirements in this test case can be well covered in clause 6.7G.1 of TS 36.521-1 [10] and clause 6.5E.4.1 of TS 38.521-1 [8] and don’t need to be tested again.

#### 6.5E.4.2	Transmit intermodulation for inter-band V2X

6.5E.4.2.1	Test purpose

Same test purpose as in clause 6.5E.4.2 in TS 38.521-1 [8].

6.5E.4.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.7.1 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.5E.4.2 of TS 38.521-1 [8] and don’t need to be tested again.

## 6.6B	Beam Correspondence for EN-DC

### 6.6B.4	Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP

Editor's note:	The following aspects are either missing of not yet determined:

-	The associated standalone test 6.6.1 in TS 38.521-2 [9] is incomplete for band n259.

6.6B.4.1	Test purpose

Same test purpose as in clause 6.6.1.1 in TS 38.521-2 [9] for the NR carrier.

6.6B.4.2	Test applicability

This test case applies to all types of E-UTRA-NR Power Class 3 UE release 15, supporting inter-band EN-DC including FR2 with 1 NR UL CC that do not support beam correspondence without UL beam sweeping.

This test case applies to all types of E-UTRA-NR Power Class 3 UE release 16 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC that do not support SSB-based or CSI-RS based enhanced beam correspondence and do not support beam correspondence without UL beam sweeping.

6.6B.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.6B.4.

6.6B.4.4	Test description

Same test description as in clause 6.6.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.6.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of initial conditions as in clause 6.3.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.6B.4.5	Test requirements

Same test requirement as in clause 6.6.1.5 in TS 38.521-2 [9] for the NR carrier.

### 6.6B.5	Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP

Editor's note:	This clause is incomplete. The following aspects are either missing of not yet determined:

-	The associated standalone test 6.6.2 in TS 38.521-2 [9] is incomplete.

6.6B.5.1	Test purpose

Same test purpose as in clause 6.6.2.1 in TS 38.521-2 [9] for the NR carrier.

6.6B.5.2	Test applicability

This test case applies to all types of E-UTRA-NR Power Class 3 UE release 16 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC and support either CSI-RS or SSB based beam correspondence and do not support beam correspondence without UL beam sweeping.

6.6B.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.6B.5.

6.6B.5.4	Test Procedure

Same test description as in clause 6.6.2.5 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 6.6.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of initial conditions as in clause 6.3.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.3.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.6B.5.5	Test Requirements

Same test requirement as in clause 6.6.2.5 in TS 38.521-2 [9] for the NR carrier.

# 7	Receiver characteristics

## 7.1	General

Editor's note:	Test configurations/environments that require new spherical scan shall be included in test procedure clause and identifying such scenarios is currently FFS and owned by RAN5.

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with E-UTRA connecting to the network by OTA without calibration.

For NR FR2 Rx test cases the identified beam peak direction can be stored and reused for a device under test in various configurations/environments for the full duration of device testing as long as beam peak direction is the same.

The requirements defined in this clause are the extra requirements compared with the single carrier requirements defined in TS 38.521-1 [8] and TS 38.521-2 [9].

Unless otherwise stated, the UL and DL reference measurement channels are the same with the configurations specified in TS 38.521-1 [8] and TS 38.521-2 [9].

Unless otherwise stated, requirements for NR receiver written in TS 38.521-1 [8] and TS 38.521-2 [9] apply and are assumed anchor agnostic. Requirements are verified under conditions where anchor resources do not interfere NR operation.

For intra-band EN-DC, the output power is configured as follows:

-	One E-UTRA uplink carrier with the output power set to 29dB below PCMAX_L,c and the NR band whose downlink is being tested has its uplink carrier output power set to 4dB below PCMAX_L,f,c.

-	One NR uplink carrier with the output power set to  29dB below PCMAX_L,f,c and the E-UTRA band whose downlink is being tested has its uplink carrier output power set to4 dB below PCMAX_L,c.

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks, an in-gap test refers to the case when the interfering signal is located at a negative offset with respect to the assigned lowest channel frequency of the highest sub-block and located at a positive offset with respect to the assigned highest channel frequency of the lowest sub-block.

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks, an out-of-gap test refers to the case when the interfering signal(s) is (are) located at a positive offset with respect to the assigned channel frequency of the highest carrier frequency or located at a negative offset with respect to the assigned channel frequency of the lowest carrier frequency.

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing adjacent channel selectivity requirements, in-band blocking requirements (for each case), and narrow band blocking requirements apply for in-gap tests only if the corresponding interferer frequency offsets with respect to the two measured carriers satisfy the following condition in relation to the sub-block gap size Wgap for at least one of the E-UTRA or NR sub-blocks, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset)| – BWChannel

For the E-UTRA sub-block, the FInterferer (offset), for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier as specified in clause 7.5.1, clause 7.6.1 and clause 7.6.3 for the respective requirement in TS 36.521 [10] and BWChannel. FInterferer (offset) for the E-UTRA sub-block with two or more contiguous component carriers is the interference frequency offset with respect to the carrier adjacent to the gap is specified in clause 7.5.1A, 7.6.1A and 7.6.3A in TS 36.521 [10].

For the NR sub-block, the FInterferer (offset), for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier as specified in clause 7.5, clause 7.6.2 and clause 7.6.4 for the respective requirement in TS 38.521-1 [8] and BWChannel..

The interferer frequency offsets for adjacent channel selectivity, each in-band blocking case and narrow- band blocking shall be tested separately with a single in-gap interferer at a time.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

For conformance testing involving FR2 test cases in this specification, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

## 7.2	Void

## 7.3	Void

## 7.3A	Reference sensitivity for CA without EN-DC

### 7.3A.1	General

For NR CA operation NR single carrier and CA operation of REFSENS requirements defined in TS 38.101-1 [2] and TS 38.101-2 [3] apply to all downlink bands part of NR CA configurations listed in Table 5.2A.1-1 unless sensitivity degradation is allowed as defined in clause 7.3A in TS 38.101-3 [4].

A UE which supports inter-band NR CA configuration is allowed to apply each sensitivity degradation for FR1 specified in clause 7.3A.2 TS 38.101-1 [2] and for FR2 specified in clause 7.3A.2 of TS 38.101-2 [3] independently.

### 7.3A.2	Reference sensitivity power level for CA without EN-DC

7.3A.2.1	Test purpose

Same test purpose as in clause 7.3.2 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 7.3.2 in TS 38.521-2 [9] for NR FR2 carrier(s).

7.3A.2.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for receiver sensitivity power level apply and are tested in TS 38.521-1 [8] clauses 7.3 and 7.3A and TS 38.521-2 [9] clauses 7.3 and 7.3A.

### 7.3A.3	ΔRIB,c for CA without EN-DC

For the UE which supports inter-band NR CA configuration, the minimum requirement for reference sensitivity in clause 7.3.2 in TS 38.101-1 [2] and clause 7.3.2, 7.3A.2in TS 38.101-2 [3] shall be increased by the amount given in ΔRIB,c in Tables below. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1GHz, the applicable additional ∆RIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in TS 38.101-1 [2] and 7.3A, 7.3B in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in TS 38.101-1 [2] and 7.3A, 7.3B in TS 38.101-3 [4] for the applicable operating bands.

#### 7.3A.3.1	ΔRIB,c for Inter-band CA between FR1 and FR2 without EN-DC

Unless otherwise stated, ΔRIB,c for NR FR1 band and FR2 band of inter-band CA defined in table 5.5A.1-1 is set to zero.

### 7.3A.4	Void

## 7.3B	Reference sensitivity level for DC

### 7.3B.1	General

For EN-DC, E-UTRA and NR single carrier, CA, and MIMO operation of REFSENS requirements defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 36.101 [5] apply to all downlink bands of EN-DC configurations listed in clause 5.5B, unless sensitivity degradation exception is allowed in this clause of this specification, clause 7.3 in TS 38.101-1 [2], clause 7.3 in TS 38.101-2 [3] or clause 7.3 in TS 36.101 [5]. Allowed exceptions specified in this clause of the specification, clause 7.3 in TS 38.101-1 [2], clause 7.3 in TS 38.101-2 [3] or clause 7.3 in TS 36.101 [4] also apply to any higher order EN-DC configuration combination containing one of the band combinations that exception is allowed for. Reference sensitivity exceptions are specified by applying maximum sensitivity degradation (MSD) into applicable REFSENS requirement. EN-DC REFSENS requirements shall be met for NR uplink transmissions using QPSK DFT-s-OFDM waveforms as defined in clause 7.3.2 in TS 38.101-1 [2]. Unless otherwise specified UL allocation uses the lowest SCS allowable for a given channel BW. Limits on configured maximum output power for the uplink according to subclause 6.2B.4 shall apply.

In case of interband EN-DC the receiver REFSENS requirements in this clause do not apply for 1.4 and 3 MHz E-UTRA carriers. For the case of inter-band EN-DC with a single carrier per cell group and multi-carrier per cell group, in addition to the E-UTRA and NR single carrier, CA, and MIMO operation of REFSENS requirements defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [5], the REFSENS requirements specified therein also apply with both downlink carriers and both uplink carriers active unless sensitivity exceptions are allowed in this clause of this specification, clause 7.3 in TS 38.101-1 [2] or clause 7.3 in TS 36.101 [5].

For reference sensitivity exception test points where the specified carrier frequency does not correspond to a valid NR-ARFCN, the closest NR-ARFCN as specified in TS 38.101-1 [2] clause 5.4.2 applies.

For operations with 4 Rx antenna ports in an E-UTRA band or an NR band, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.1-1a of TS 36.101[5] for the E-UTRA band or in Table 7.3.2-2 of TS 38.101-1 [2] for the NR band when MSD > 0.

NOTE:	For inter-band EN-DC, the reference sensitivity requirement with both uplink carriers active is allowed to be verified for only a single inter-band EN-DC configuration per NR band.

### 7.3B.2	Reference sensitivity for EN-DC

#### 7.3B.2.0	Minimum Conformance Requirements of Reference sensitivity for EN-DC

##### 7.3B.2.0.1	Intra-band contiguous EN-DC

For intra-band contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels. The reference sensitivity requirements apply with all uplink carriers and all downlink carriers active for EN-DC configuration and Uplink EN-DC configuration listed in Table 5.5B.2-1 and Table 5.5B.3-1, as supported by the UE. For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements apply with single uplink transmission. The downlink carrier(s) from the cell group with uplink shall be configured closer to the uplink operating band than any of the downlink carriers from the cell group without uplink.

Sensitivity degradation is allowed for Intra-band contiguous EN-DC configurations listed in Table 7.3B.2.0.1-1, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3B.2.0.1-1 and E-UTRA and NR single carrier requirements do not apply.

Table 7.3B.2.0.1-1: Reference sensitivity (MSD) for intra-band contiguous EN-DC

| EN-DC configuration/channel allocations/MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 |  |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 | FDD |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 |  |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 | FDD |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [5], with additional limits on configured maximum output power for the uplink according to subclause 6.2B.4. |  |  |  |  |  |  |  |

##### 7.3B.2.0.2	Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels.

For DC_3A_n3A intra-band non-contiguous EN-DC combination, only single switched UL is supported in rel.15, therefore, no MSD is specified and E-UTRA and NR single carrier requirements apply.

##### 7.3B.2.0.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

7.3B.2.0.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.0.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3B.2.0.3.1-2.

Table 7.3B.2.0.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1, 3 | n772, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 2 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 3 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 5 | n786,7 |  | 10.5 | 8.9 | 7.8 |  |  | 5.4 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| 5 | n776, 7, 17 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
|  | n774, 5, 17 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 8 | n418,9 |  | 13 | 11.3 | 10.1 |  | 8.3 | 7.0 | 6.1 | 5.5 |  | 4.3 | 3.9 | 3.5 |
| 8 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| 8 | n794,5 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| 12 | n668,9,10 | 10 | 7.5 | 6.2 | 5.5 |  |  | 2.4 |  |  |  |  |  |  |
| 12 | n775 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 12 | n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 13 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 14 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 18, 19 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 28 | n512,13 | 27.8 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n513 | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 20 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n28 | 2116 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |  |
| 26 | n418,9 |  | 10.3 | 8.4 | 7.4 |  |  | 5 | 4.3 | 3.9 |  | 3.1 | 2.7 |  |
| 26 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n28 | 18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
| 71 | n211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
|  | n212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
| n71 | 211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
|  | 212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
| n71 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |  |
| 66 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n66 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_66_n78, and DC_25_n77. NOTE 14: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). NOTE 15: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 16: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n77 | 15 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| 2 | n77 | 15 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| 2 | n78 | 15 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| 3 | n77 | 15 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| 3 | n78 | 15 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| 5 | n77 | 15 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 5 | n78 | 15 | 8 | 16 | 25 | 25 |  |  | 25 |  |  |  |  |  |  |
| 8 | n41 | 15 |  | 16 | 25 | 25 |  | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 8 | n77 n78 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 8 | n79 | 15 |  |  |  |  |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| 12 | n66 | 15 | 8 | 16 | 20 | 20 |  |  | 20 |  |  |  |  |  |  |
| 12 | n77 | 15 |  | 10 | 15 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| 12 | n78 | 15 |  | 10 | 15 | 20 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 13 | n77 | 15 |  | 15 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| 14 | n77 | 15 |  | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| 18 | n77, n78 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 19 | n77, n78 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| 20 | n77 n78 | 15 |  | 16 | 25 | 251, 252 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 26 | n41 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 |  |  |  |  |  |
| 26 | n77 n78 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n28 | 1 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 28 | n51 | 15 | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n77 n78 | 15 |  | 10 | -15 | 20 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 66 | n77 | 15 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 66 | n78 | 15 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n66 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |
| 71 | n2 | 15 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |  |
| n71 | 2 | 15 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |  |
| n71 | 7 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1 [2] applies. NOTE 2: Void. NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 6: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-3: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n46 | 482,4 | 22.6 | 19.5 | 17.8 | 16.6 |  |  |  |  |  |  |  |
| NOTE 1: FFS NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE 3: FFS NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-4: n46 Reference sensitivity measurement exclusion region in MHz

FFS

Table 7.3B.2.0.3.1-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC paring with n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n46 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |

7.3B.2.0.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) are specified in Table 7.3B.2.0.3.2-1 with uplink configuration of the aggressor band (high) specified in Table 7.3B.2.0.3.2-2.

Table 7.3B.2.0.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n2 | 714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | 6.1 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 122 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 132 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 142 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n79 | 114 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and in MHz and fULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: Void. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-1a: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 2 | 9.1 | 8.0 | 7.0 | 6.7 |  |  |  |  |  |  |  |
| n77 | 121 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n77 | 131 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n77 | 141 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n79 | 191 | 32.5 | 29.5 | 27.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 42.3 | 39.3 | 37.5 |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and in MHz and fULHBfULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n2 | 71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n77 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 12 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 13 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 14 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n78 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n79 | 11 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1[2]. NOTE 4: Unless otherwise stated, the UL resource blocks allocation is applied at the centre of the channel bandwidth. The note applies to the entire table. NOTE 5: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.0.3.3	Void

7.3B.2.0.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same EN-DC configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3B.2.0.3.4-1, Table 7.3B.2.0.3.4-1a with uplink configuration of the aggressor band specified in Table 7.3B.2.0.3.4-2.

Table 7.3B.2.0.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band /MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dBm) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n15 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |
| 15 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | [1.4] |  |  |  |  |  |
| 1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |
| 1 | n41 |  | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | [0.7] | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| n5 | 28 | 4.5 | 3 | 2.2 | 0.3 |  |  |  |  |  |  |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |
| n41 | 1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |
| n41 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n41 | 3 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n41 | 25 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n77 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| n41 | 661 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |
| 41 | n77 |  | 8.3 | 8.3 | 8.3 |  |  | 6.3 | 5.3 | 4.5 | 4.0 | 3.9 | 3.8 |
| 3 | n51 | 6.4 |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 |  |  | 8.3 |  |  |  |  |  |
| n78 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| n78 | 38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |
| n78 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| n78 | 46 |  |  |  | 7 |  |  |  |  |  |  |  |  |
| 41 | n78 |  | 8.3 | 8.3 | 8.3 |  |  | 6.3 | 5.3 | 4.5 | 4.0 | 3.9 | 3.8 |
| n79 | 424 | 2.8 | 2.8 | 2.8 | 2.8 |  |  |  |  |  |  |  |  |
| 48 | n46 | - | - | - | 7 | - | - | 5.7 | - | 5.1 | 4.7 | - | - |
| n46 | 48 | 13.3 | 10.4 | 8.8 | 7.8 | - | - | 7.8 | 7 | 6.5 | 5.7 | 5.4 | 5.1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified. NOTE 5: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 6: The requirements only apply for UEs supporting inter-band DC_42_n79 ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.4-1a: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| n41 | 3 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n1 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | [25] |  |  |  |  |  |
| 1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |
| 1 | n41 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 3 | n41 | 15 |  | 50 | 50 | 50 |  | [50] | 50 | 50 | 50 | 50 | 50 | 50 |
| n5 | 28 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |
| n40 | 1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |
| n41 | 1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |
| n41 | 2 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n41 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n41 | 25 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n77 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n41 | 66 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |
| 41 | n77 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| 3 | n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 15 | 25 | 25 | 25 | 25 |  |  | 25 |  |  |  |  |  |
| n78 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n78 | 38 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n78 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n78 | 46 | 30 |  |  |  | 270 |  |  |  |  |  |  |  |  |
| 41 | n78 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| n79 | 42 | 30 | 2704 | 2704 | 2704 | 2704 |  |  |  |  |  |  |  |  |
| 48 | n46 | 15 |  |  |  | 216 |  |  | 216 |  | 216 | 216 |  |  |
| n46 | 48 | 30 | 216 | 216 | 216 | 216 |  |  | 216 | 216 | 216 | 216 | 216 | 216 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 2: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 3: When the maximum UL RB allocation "LCRB" value is less than the maximum transmission bandwidth configuration “NRB” defined in Table 5.3.2-1 in 38.101-1 [2] for the specified UL band SCS, the UL band should be configured using the lowest CBW that is compatible with the maximum specified LCRB value. NOTE 4: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. NOTE 5: The requirements only apply for UEs supporting inter-band ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.0.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

7.3B.2.0.3.5.0	General

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex M  the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [5] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [5] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [5], with parameters specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1A_n8A | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
| DC_1A_SUL_n77A- |  |  |  |  |  |  |  |
| n84A | n77, n78 | 4090 | 10 | 50 | 4090 | N/A | N/A |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
| DC_1A-SUL_n77A-n84A |  |  |  |  |  |  |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A | n77, n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
| DC_2A_n77A DC_2A_n77(2A) DC_2A-2A_n77A | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
| DC_2A-2A_n77(2A) |  |  |  |  |  |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 8.0 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_2A_n78A | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
| DC_3_n5 | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_3C_n7A | n7 | 2535 | 10 | 50 | 2655 | 10.25 | IMD4 |
| DC_3A_n8A | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
| DC_3C_n41A | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | IMD4 |
| DC_3A_SUL_n41A-n80A, | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
| DC_3C_SUL_n41A-n80A | n41 | 2657.5 | 10 | 52 | 2657.5 | N/A | N/A |
| DC_3A_n77A | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
| DC_3A_SUL_n77A-n80A DC_3A_n78A |  |  |  |  |  |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
| DC_3A_SUL_n77A-n80A, DC_3A_n78A |  |  |  |  |  |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_5A_n77A8 DC_5A_n77(2A)8 | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | 5.5 | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n66A 4DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A_n77A | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_8A_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
|  | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
| DC_8A_n3A | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n20A | n20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | n20 | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  | 8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD33 |
| DC_8A_n41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_SUL_n41A- | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
| n81A | n41 | 2685 | 10 | 52 | 2685 | N/A | N/A |
| DC_8A_n77A DC_8A_n78A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
| DC_8A-n79C, DC_8A-SUL_n78A-n81A | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | H4 |
| DC_12A_n77A DC_12A_n77(2A) | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_12_n78 | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_13A_n77A | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 |
|  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |
| DC_14A_n77A DC_14A_n77(2A) | 14 | 795.5 | 5 | 15 | 765.5 | 5.5 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_20A_n3A | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_8A_n79A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
| DC_8A-SUL_n79A-n81A | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_18A_n77A DC_18A_n78A | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_12A_n77A | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_19A_n78A | 19 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20_n7 | 20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 21 | IMD3 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | IMD3 |
| DC_20A_n77A DC_20A_n78A, | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
| DC_20A-SUL_n78A-n82A | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_20A_n77A | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD54 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_21A_n28A7 | 21 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | IMD5 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD3 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD 4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_25A_n77A DC_25A-25A_n77A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 8 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_26A_n77A, | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
| DC_26A_n78A | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28A_n77A, | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_28A_n78A, DC_28A-SUL_n78A-n83A | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_30A_n77A DC_30A_n77(2A) | 30 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_48A_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_66A_n2A | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
| DC_66A_n25A | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) DC_66A-66A-66A_n77A | 66 | 1775 | 5 | 25 | 2175 | 31.0 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 5.0 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2130 | 5.0 | IMD5 |
|  | n78 | 3660 | 5 | 50 | 3660 | N/A | N/A |
| DC_71A_n66A | 71 | 675 | 5 | 25 | 629 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Void NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_2A_n77A DC_2A-2A_n77A DC_2A_n77C DC_2A-2A_n77C | 2 | 1855 | 5 | 25 | 1935 | 32.10 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 19.10 | IMD41 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_5A_n77A3 DC_5A_n77C3 | 5 | 844 | 5 | 25 | 889 | 18.60 | IMD41 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_12A_n77A | 12 | 702 | 5 | 20 | 732 | 11.7 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_13A_n77A DC_13A_n77C | 13 | 782 | 5 | 20 | 751 | 15.37 | IMD5 |
|  | n77 | 3879 | 10 | 50 | 3879 | N/A | N/A |
| DC_14A_n77A | 14 | 795.5 | 5 | 15 | 765.5 | 11.7 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 33.4 | IMD3 |
|  | n79 | 4420.5 | 10 | 50 | 4420.5 | N/A | N/A |
| DC_30A_n77A | 30 | 2310 | 5 | 25 | 2355 | 17.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | 1775 | 5 | 25 | 2175 | 34.33 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 11.27 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: Void. NOTE 3: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 4: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

7.3B.2.0.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.0.3.5.2-0: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 |  | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |  |

Editor’s Note:

- In 38.101-3 (V18.3.0) Table 7.3B.2.3.5.2-1 (MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)), ENDC band combination DC_2A-13A_n77A, and DC_2A-66A_n77A have UL/DL bandwidth 5 MHz for n77, while there is no bandwidth definition of 5MHz for n77 in 38.101-1 (V18.3.0) Table 5.3.5-1 (Channel bandwidths for each NR band). Therefore in following Table 7.3B.2.0.3.5.2-1 corresponding cases, n77 UL/DL bandwidth of 10 MHz and UL CLRB of 50 are used instead.

Table 7.3B.2.0.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-7A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-7A_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | IMD3 |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | N/A |
| DC_1A-18A_n77A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_1A-18A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2154.6 | 3.6 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n78 | 3647 | 10 | 50 | 3647 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | N/A | 5 | N/A | 1500 | 31.5 | IMD2 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | IMD5 |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | IMD5 |
| DC_1A-41A_n78A | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1955 | 5 | 25 | 2145 | 8.7 | IMD4 |
|  | 41 | 2507.5 | 10 | 50 | 2507.5 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_2A_n5A-n77A | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3540 | 16.0 | IMD3 |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | 12 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 2 | N/A | 5 | N/A | 1944 | 16.0 | IMD3 |
| DC_2A-13A_n77A | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n77 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 7.2 | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 22.6 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |
|  | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | N/A | 5 | N/A | 2115 | 29.2 | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | N/A | 5 | N/A | 2140 | 10.4 | IMD4 |
| DC_2A-66A_n77A | n77 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | M/A | N/A |
|  | 66 | N/A | 5 | N/A | 2175 | 4.0 | IMD5 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
|  | 2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_3A-5A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 5 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD2X |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
| DC_3A-7A_n28A DC_3C-7A_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
| DC_3A-7A_n78A DC_3C-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-18A_n77A DC_3A-18A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 18 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-19A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | 840 | 5 | 25 | 885 | [18.5] | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | 3 | 1733 | 5 | 25 | 1828 | 9.4 | IMD4 |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
| DC_3A-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1862.5 | 30.8 | IMD2 |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | N/A | N/A |
|  | n78 | 3322 | 10 | 50 | 3322 | N/A | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 |
|  | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A-41A_n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |
| DC_3A-SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD34 |
| DC_7A-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |
|  | 20 | 855 | 5 | 25 | 814 | N/A | N/A |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 836 | 5 | 25 | 795 | 17.4 | IMD3 |
|  | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD3 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 7 | 2504 | 5 | 25 | 2624 | 18.8 | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
| DC_7A-20A_n28A | 20 | 842 | 5 | 25 | 801 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | 7 | 2520 | 10 | 50 | 2640 | 5.9 | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
|  | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_7A-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 780 | 8.3 | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3421 | N/A | N/A |
|  | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3421 | N/A | N/A |
|  | 7 | 2570 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 768 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3421 | N/A | N/A |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1896 | 5 | 25 | 1976 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3460 | 17.3 | IMD3 |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 16.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
| DC_13A-66A_n2A | 66 | N/A | 5 | N/A | 2156 | 7..2 | IMD4 |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
| DC_13A-66A_n77A | 66 | N/A | 5 | N/A | 2156 | 17.1 | IMD3 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | 16.0 | IMD3 |
|  | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_28A_n7A-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_28A-42A_n79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |
| DC_20A_n28A-n78A DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |
|  | n2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |
| DC_66A_n2A-n77A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_66A_n5A-n77A | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3460 | 16.6 | IMD39 |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 3: This band is subject to IMD3 also which MSD is not specified. NOTE 4: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 5 to NOTE 8: FFS NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

7.3B.2.0.3.5.3	Void

##### 7.3B.2.0.3a	Inter-band NE-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band. This subclause addresses directly only NE-DC configurations that don't have a corresponding specified EN-DC configuration or specific NE-DC exceptions.

7.3B.2.0.3a.1	Reference sensitivity exceptions due to UL harmonic interference for NE-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same NE-DC configuration. For the NE-DC configurations that have an EN-DC defined configuration, the reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3B.2.3.1-2 are applicable.

##### 7.3B.2.0.4	Inter-band EN-DC including FR2

7.3B.2.0.4.1	Void

##### 7.3B.2.0.5	Inter-band EN-DC including both FR1 and FR2

7.3B.2.0.5.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC including both FR1 and FR2

For inter-band EN-DC of E-UTRA and NR in both FR1 and FR2, the UE is allowed to apply each sensitivity degradation for EN-DC in FR1 specified in clause 7.3B.2.3 TS 38.101-3 and for EN-DC including FR2 specified in clause 7.3B.2.3 of TS 38.101-3 independently.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.3B.2.

#### 7.3B.2.1	Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs)

Editor's note:	MSD test point selection is based on core spec requirement and shall be added to TP analysis to TS 38.905 [7].

7.3B.2.1.1	Test purpose

To verify the ability of UE that support intra-band contiguous EN-DC configurations to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise. A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.3B.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.1.4	Test description

7.3B.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B, and the channel bandwidth combination for E-UTRA and NR component carriers shall follow the value specified in Table 5.3B.1.2-1. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2, and are shown in Table 7.3B.2.1.4.1-1 for DC_(n)71AA and Table 7.3B.2.1.4.1-2 for intra-band contiguous EN-DC other than DC_(n)71AA.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.3B.2.1.4.1-1: Test configurations table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  | Specified below |  |  |  |  |  |  |  |
| E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  | Specified below |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  | Specified below |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Lowest |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3 in TS 36.521-1 [10], dependent on PCC Band |  |  |  |  |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  | Uplink Configuration |  |  |  |  |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |  |  | NR Modulation |  | NR RB allocation |  | E-UTRA Modulation |  | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB |  |  | DFT-s-OFDM QPSK |  |  |  | QPSK |  |  |
| Test Point configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_(n)71AA for dual UL UE |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | E-UTRA/NR band | FC (UL) (MHz) NUL |  | UL Channel BW (MHz) |  |  | UL allocation (LCRB) |  | FC (DL) (MHz) NDL |  | Duplex mode |  |
| 1 | 71 | 665.5MHz, EUL= 133147 |  | 5 |  |  | 5@19 |  | 619.5 MHz EDL = 68611 |  | FDD |  |
|  | n71 | 675.5 NUL= 135100 |  | 15 |  |  | 15@0 |  | 629.5 NDL = 125900 |  |  |  |
| 2 | 71 | 670.5 EUL= 133197 |  | 15 |  |  | 15@59 |  | 624.5 EDL = 68661 |  |  |  |
|  | n71 | 680.5 NUL= 136100 |  | 5 |  |  | 5@0 |  | 634.5 NDL = 126900 |  |  |  |
| 3 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@39 |  | 622 EDL = 68636 |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 10 |  |  | 10@0 |  | 632 NDL = 126400 |  |  |  |
| 4 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@0 |  | 622 EDL = 68636 |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 10 |  |  | 10@41 |  | 632 NDL = 126400 |  |  |  |
| 5 | 71 | 665.5 EUL= 133147 |  | 5 |  |  | 5@19 |  | 619.5 EDL = 68611 |  | FDD |  |
|  | n71 | 675.5 NUL= 135100 |  | 153 |  |  | 15@0 |  | 6323 NUL= 126400 |  |  |  |
| 6 | 71 | 670.5 EUL= 133197 |  | 15 |  |  | 15@59 |  | 624.5 EDL = 68661 |  |  |  |
|  | n71 | 680.5 NUL= 136100 |  | 53 |  |  | 5@0 |  | 6373 NUL= 127400 |  |  |  |
| 7 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@39 |  | 622 EDL = 68636 |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 103 |  |  | 10@0 |  | 634.53 NUL= 126900 |  |  |  |
| 8 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@0 |  | 622 EDL = 68636 |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 103 |  |  | 10@41 |  | 634.53 NUL= 126900 |  |  |  |
| Test Settings for DC_(n)71AA for single UL UE |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 71 | default |  | Highest |  |  | 0 |  | default |  | FDD |  |
|  | n71 | default |  | Highest |  |  | REFSENS_NR |  | default |  |  |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 4: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 5: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.1.4.1-2: Test configurations table for EN-DC configurations not affected by Reference sensitivity exceptions, EN-DC configuration other than DC_(n)71AA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  | Low range, High range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3 in TS 36.521-1 [10], dependent on PCC Band |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_(n)XAA Configuration (Intra-band contiguous EN-DC) – Note 2 |  |  |  |  |  |  |  |  |
| 1 (Note 4) | X | default |  |  | nX | default |  |  |
|  | QPSK | QPSK | Highest NRB_agg(Note 6) | All RBs / REFSENS_LTE | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest NRB_agg(Note 6) | All RBs / REFSENS_NR |
| 2 (Note 5) | X | default |  |  | nX | default |  |  |
|  | N/A | QPSK | Highest NRB | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| Note 1: REFSENS_NR and REFSENS_LTE refer to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3 of TS 38.521-1 [8] and Table 7.3.4.1-1 of TS 36.521 [10] for NR and E-UTRA CC respectively. Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 4: Test point for UE supporting dual UL. Note 5: Test point for UE supporting single UL. Note 6: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-URA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels for NR are set according to Table 7.3B.2.1.4.1-1 and Table 7.3B.2.1.4.1-2.

7.	The DL and UL Reference Measurement channels for E-UTRA are set according to Table 7.3B.2.1.4.1-1 and Table 7.3B.2.1.4.1-2.

8.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8].

9.	E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

10.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.3B.2.1.4.3.

11.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.3B.2.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.1.4.1-1 and Table 7.3B.2.1.4.1-2 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.1-1 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	For test points in Table 7.3B.2.1.4.1-1, set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3B.2.1.5-1. For test points in Table 7.3B.2.1.4.1-2, set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.3-1 for NR band and TS 36.521-1 [10] Table 7.3.3-1 for E-UTRA band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

Message contents exceptions are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value.

7.3B.2.1.5	Test requirement

For intra-band contiguous EN-DC configurations DC_(n)71AA with dual UL, the throughput of each CG shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Table 7.3B.2.1.5-1.

Reference sensitivity test requirements for EN-DC configurations other than DC_(n)71AA with dual UL UE, are specified in Table 7.3.5-1 in TS 36.521-1 [10] for the LTE CC, and Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.1.5-1: Reference sensitivity for intra-band DC_(n)71AA with dual UL

| EN-DC | E-UTRA/ NR band | SCS (kHz) | Channel BW (MHz) | Ref sensitivity (dBm) | Note | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)71AA | 71 | N/A | 5 | -96.5 | Note 1 | FDD |
|  | n71 | 15 | 15 | -89.8 +TT |  |  |
|  | 71 | N/A | 15 | -91.3 | Note 2 |  |
|  | n71 | 15 | 5 | -95.6 +TT |  |  |
|  | 71 | N/A | 10 | -93.5 | Note 3 |  |
|  | n71 | 15 | 10 | -92.3 +TT |  |  |
|  | 71 | N/A | 10 | -76.3 | Note 4 |  |
|  | n71 | 15 | 10 | -64.6 +TT |  |  |
|  | 71 | N/A | 5 | -96.5 | Note 5 |  |
|  | n71 | 15 | 159 | -89.1 +TT |  |  |
|  | 71 | N/A | 15 | -91.3 | Note 6 |  |
|  | n71 | 15 | 59 | -95.0 +TT |  |  |
|  | 71 | N/A | 10 | -93.5 | Note 7 |  |
|  | n71 | 15 | 109 | -91.5 +TT |  |  |
|  | 71 | N/A | 10 | -76.3 | Note 8 |  |
|  | n71 | 15 | 109 | -64.9 +TT |  |  |
| NOTE 1: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 1. NOTE 2: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 2. NOTE 3: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 3. NOTE 4: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 4. NOTE 5: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 5. NOTE 6: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 6. NOTE 7: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 7. NOTE 8: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 8. NOTE 9: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. |  |  |  |  |  |  |

Table 7.3B.2.1.5-2: Test Tolerance (TT) for NR RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

#### 7.3B.2.2	Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs)

Editor's note:	No test case details specified due to no confirmed industry interest for intra-band non-contiguous EN-DC combinations (removed at RAN5 #97).

#### 7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

For exception requirements applicable for both NR and LTE, LTE anchor agnostic approach is not applied.

For non-exception requirements applicable to NR, LTE anchor agnostic approach is applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Void

Table 7.3B.2.3.4.2.1-1: Void

Table 7.3B.2.3.4.2.1-2_1: Void

Table 7.3B.2.3.4.2.1-2_2: Void

Table 7.3B.2.3.4.2.1-2_3: Void

Table 7.3B.2.3.4.2.1-2_4: Void

Table 7.3B.2.3.4.2.1-2_5: Void

Table 7.3B.2.3.4.2.1-2_6: Void

Table 7.3B.2.3.4.2.1-2_7: Void

Table 7.3B.2.3.4.2.1-2_8: Void

Table 7.3B.2.3.4.2.1-2_9: Void

Table 7.3B.2.3.4.2.1-2_11: Void

Table 7.3B.2.3.4.2.1-2_12: Void

Table 7.3B.2.3.4.2.1-2_13: Void

Table 7.3B.2.3.4.2.1-2_14: Void

Table 7.3B.2.3.4.2.1-2_15: Void

Table 7.3B.2.3.4.2.1-2_16: Void

Table 7.3B.2.3.4.2.1-2_17: Void

Table 7.3B.2.3.4.2.1-2_18: Void

Table 7.3B.2.3.4.2.1-2_19: Void

Table 7.3B.2.3.4.2.1-2_20: Void

Table 7.3B.2.3.4.2.1-2_21: Void

Table 7.3B.2.3.4.2.1-2_22: Void

Table 7.3B.2.3.4.2.1-2_23: Void

Table 7.3B.2.3.4.2.1-4: Void

Table 7.3B.2.3.4.2.1-4a: Void

Table 7.3B.2.3.4.2.1-4b: Void

Table 7.3B.2.3.4.2.1-4c: Void

Table 7.3B.2.3.4.2.1-4d: Void

Table 7.3B.2.3.4.2.1-4e: Void

Table 7.3B.2.3.4.2.1-4f: Void

Table 7.3B.2.3.4.2.1-4g: Void

Table 7.3B.2.3.4.2.1-4h: Void

Table 7.3B.2.3.4.2.1-4i: Void

Table 7.3B.2.3.4.2.1-4j: Void

Table 7.3B.2.3.4.2.1-4k: Void

Table 7.3B.2.3.4.2.1-4l: Void

Table 7.3B.2.3.4.2.1-4m: Void

Table 7.3B.2.3.4.2.1-4n: Void

Table 7.3B.2.3.4.2.1-5: Void

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  |  |  |  | NRB |  |  |  |  | Band |  |  | Range |  | NRB |  |  |  |
|  | UL MOD | DL MOD |  |  |  |  | CH BW |  |  | DLalloc / UL alloc |  | UL MOD |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 | Mid |  |  |  |  |  |  |  |  |  | n3 |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 24,10 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1760 / DL 1855 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 36 | 1 | Low |  |  |  |  |  |  |  |  |  | n3 |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1965 / DL 2155 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 887.5 / DL 932.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_1A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 713.3 / DL 768.3 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 28 | 1 | UL 1970 / DL 2160 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 708 / DL 763 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| Test Settings for a DC_1A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 1 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 26 | 1 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3900 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3870 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4090.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3709.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3709.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 2 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_LTE |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 2 | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2175 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 2 | UL 1883.3 /DL1963.3 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2150 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | DL 1980 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 665.5 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 23 | 2 | DL1980 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 673 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 35 | 2 | UL 1881 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3690.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 3 8 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3850.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 45 | 2 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3920.01 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 4a 9 | 2 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 2 | UL 1855/DL 1935 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3790.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 64 | 2 | UL 1900/DL 1980 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 74 | 2 | UL 1885/DL 1965 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3810 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1870 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3740.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 2 | UL 1885 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3739.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 2 | UL 1855/ DL 1935 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3790.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 2 | UL 1885/ DL 1965 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3690 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 | Mid |  |  |  |  |  |  |  |  |  | n1 |  |  | Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 26 | 3 | High |  |  |  |  |  |  |  |  |  | n1 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 34 | 3 | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  | n1 |  |  | UL 1950 / DL 2140 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1771 / DL 1866 |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 838 / DL 883 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 3 | UL 1721 / DL 1816 |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 838 / DL 883 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1730 / DL 1825 |  |  |  |  |  |  |  |  |  | n7 |  |  | UL 2535 / DL 2655 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1755 / DL 1850 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 900 / DL 945 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 3 | UL 1747.5 / DL 1842.5 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 897.5 / DL 942.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 26 | 3 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 34, 10 | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2657.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_3A_n77A and DC_3A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 3 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3525 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 35 | 3 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3685.005 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 39 | 3 | Low |  |  |  |  |  |  |  |  |  | n78 |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs/0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 44 | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3574.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 54 | 3 | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3435 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 5 | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1721/ DL 2121 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 837.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 5 | UL 829 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4145.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 38 | 5 | UL Mid (836.5 MHz) |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 44, 13 | 5 | UL 844/DL 889 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3421.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 54, 13 | 5 | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4177.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 5 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3560.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 5 | UL 844/DL 889 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3421.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1825 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 10 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2667 |  |  |  |  |  |  |  |  |  | n5 |  |  | DL 879 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 10 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2130 |  |  |  |  |  |
|  | QPSK |  |  |  |  |  | 10 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 7 | DL 2672 |  |  |  |  |  |  |  |  |  | n71 |  |  | UL/ 668 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 28 | 7 | DL 2630 |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 668 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| Test Settings for a DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2660 |  |  |  |  |  |  |  |  |  | N77 |  |  | DL 3870 |  |  |  |  |  |
|  | QPSK |  |  |  |  |  | 5 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 | High |  |  |  |  |  |  |  |  |  | n78 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 932.5 |  |  |  |  |  |  |  |  |  | n1 |  |  | DL 2155 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 945 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1850 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 | DL 942.5 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1842.5 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | N/A |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  | n20 |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  | n20 |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | UL 885 / DL 942.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2655 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 60 MHz | All RBs / 0 |  |  |
| 28 | 8 | UL 885 / DL 942.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2546 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 60 MHz | All RBs / 0 |  |  |
| 34 | 8 | UL 882.5 / DL 927.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2685MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_8A_n77A and DC_8A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3590.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 2,8 | 8 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3520.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 8 | UL 897.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3634.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 | Mid |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4457.7 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 11 | Mid |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4512.69 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_12A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | UL 710 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1730 /DL 2130 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 28 | 12 | UL 710 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1770 /DL 2170 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_12A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3537.495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3351 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 35 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3687.495 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 49 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3750 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 12 | UL 702/DL 732 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3540 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_12A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | UL 707/DL 737 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3535 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 12 | UL 707/DL 737 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3750 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34,10 | 12 | UL 710/DL 740 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3580 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 | UL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3909.99 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 13 | UL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 35 | 13 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3755 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 49 | 13 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3850 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 13 | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3891.495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_14A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3964.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 18 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.1 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 25 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3814.995 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.01 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 14 | UL 795.5/DL 765.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3947.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_18A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 18 | UL 820 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4100 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 28 | 18 | UL 820 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4000 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_19A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 19 | UL 835 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4175 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 28 | 19 | UL 835 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4100 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_19A_n79A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 | DL 884 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4420 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 19 | DL 884 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4445.025 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1775 / DL 1870 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 20 | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1735 / DL 1830 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  | n7 |  |  | UL 2512 / DL 2632 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 20 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3387.99 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 20 | UL 850 / DL 809 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3358.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_21A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 21 | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 713 / DL 780.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_21A_n79A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4510.2 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4800 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 21 | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4420.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_25A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 10 MHz |  |  | All RBs / 0 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 26 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | N/A |  |  |  | QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2524.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 50 MHz |  |  | All RBs / 0 |
| 2,8 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2572 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | QPSK |  | 50 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 839 |  |  |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2562 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 45 | 26 | DL 886.6 |  |  |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2660 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_LTE |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3366 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 2,8 | 26 | High |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3446.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 836.5 |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3391.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 | High |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4432.5 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 26 | High |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4470.51 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 | High |  |  |  |  |  |  |  |  | n5 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 28 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  | UL/DL 3540 |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 28 | 28 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 34 | 28 | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  | n77/n78 |  | UL/DL 3582.495 |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |
| 45 | 28 | Low |  |  |  |  |  |  |  |  |  | n77 |  | UL/DL 3815.01 |  |  |  |  |  |  |
|  | N/A | QPSK |  |  | 10 MHz |  |  | All RBs / 0 |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_30A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 30 | Low |  |  |  |  |  |  |  |  | n66 |  |  |  | High |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_30A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 30 | UL 2310 |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3487.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_38A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 38 | DL High |  |  |  |  |  |  |  |  | n78 |  |  |  | UL Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3525 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 2,9 | 40 | Low |  |  |  |  |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 | Low |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3750 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 | High |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 41 | High |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 48 |  |  | UL/DL 3555 |  |  |  |  |  |  | n66 |  |  |  | UL 1760 MHZ |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 23 | 48 |  |  | UL/DL 3565 |  |  |  |  |  |  | n66 |  |  |  | UL 1770 |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 38 | 48 |  |  | UL/DL 3600 |  |  |  |  |  |  | n66 |  |  |  | UL1760 |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 44 | 48 |  |  | UL/DL 3630 |  |  |  |  |  |  | n66 |  |  |  | UL 1715 / DL 2115 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 | 66 |  |  | UL1775/ DL2175 |  |  |  |  |  |  | n2 |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 24, 10 | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  | n2 |  |  |  | UL 1883.3 / DL 1963.3 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  | n5 |  |  |  | UL 838/ DL 883 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  | n25 |  |  |  | UL 1855/ DL 1935 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 24 | 66 |  |  | UL 1712.5/ DL 2112.5 |  |  |  |  |  |  | n25 |  |  |  | UL 1912.5/ DL 1992.5 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 34 | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  | n25 |  |  |  | UL 1883.3/ DL 1963.3 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 66 |  |  | High |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  | n71 |  |  |  | UL 675/ DL 629 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 66 |  | UL 1720 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3440.01 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 23 | 66 |  | UL 1720 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3410 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 38 | 66 |  | UL Mid |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 44 | 66 |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3949.995 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 54 | 66 |  | UL 1760 / DL 2160 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 66 |  | UL 1755 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3510 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 23 | 66 |  | UL 1755 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3480 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 34 | 66 |  | UL 1730/ DL2130 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3660 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for DC_71A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 71 |  | UL 665.5 |  |  |  |  |  |  |  | n2 |  |  |  | DL 1987.5 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5MHz |  | All RBs / REFSENS_ENDC_1 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / 0 |
| 23 | 71 |  | UL 673 |  |  |  |  |  |  |  | n2 |  |  |  | DL1980 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 35 | 71 |  | Low |  |  |  |  |  |  |  | n2 |  |  |  | UL 1881 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 49 | 71 |  | High |  |  |  |  |  |  |  | n2 |  |  |  | UL 1881 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for DC_71A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 71 |  | UL 675/ DL 629 |  |  |  |  |  |  |  | n66 |  |  |  | UL1750/ DL 2150 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE 15: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2B.2.3.1-2 of TS 38.508-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in Tables 7.3B.2.3.5-1 to 7.3B.2.3.5-4. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

Non-exception requirements are tested according to LTE anchor agnostic below.

Same test description as in clause 7.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.3.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.3.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with NOTE 4 in Table 7.3B.2.3.4.2.1-6, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

For non- exception requirements (LTE anchor agnostic approach) of all EN-DC configurations, test requirements for NR carrier are the same as 7.3.2.5 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n772 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n772, 13 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n782 | 15 |  | -71.9+TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
| 2 | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.9 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | --95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| 3 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  |  |  | -74.1 +TT15 | -74.1 +TT15 | -74.0 +TT15 |  |  | -73.9 +TT15 | -74.0 +TT15 |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  |  |  | -74.4 +TT15 | -74.2 +TT15 | -74.2 +TT15 |  |  | -74.0 +TT15 | -74.1 +TT15 | --74.1 +TT15 | -74.0 +TT15 | -74.0 +TT15 | -74.0 +TT15 |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  |  |  |  | -74.8 +TT15 | -74.5 +TT15 | -74.4 +TT15 |  |  | -74.2 +TT15 | -74.2 +TT15 | -74.2 +TT15 | -74.1 +TT15 | -74.1 +TT15 | -74.1 +TT15 |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -96.9 +TT15 | -95.4 +TT15 | -94.6 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.2 +TT15 | -95.5 +TT15 | -94.8 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.6 +TT15 | -95.8 +TT15 | -95.0 +TT15 |  |  |  |  |  |  |  |  |
| 5 | n78 | 15 |  | -84.2 +TT | -84.4 +TT |  |  |  | -84.2 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -84.3 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.3 +TT | -84.5 +TT | -84.4 +TT | -84.3 +TT | -84.0 +TT | -84.2 +TT |
|  |  | 60 |  | -84.5 +TT | -84.6 +TT | -84.5 +TT |  |  | -84.5 +TT | -84.6 +TT | -84.5 +TT | -84.4 +TT | -84.1 +TT | -84.3 +TT |
| 8 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -84.0 +TT | -83.9 +TT |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -84.3 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 8 | n41 | 15 |  | -81.8 +TT | -81.7 +TT | -81.7 +TT |  |  | -81.6 +TT | -81.5 +TT |  |  |  |  |
|  |  | 30 |  | -82.1 +TT | -81.8 +TT | -81.9 +TT |  |  | -81.7 +TT | -81.6 +TT | -81.4 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
|  |  | 60 |  | -82.5 +TT | -82.1 +TT | -81.1 +TT |  |  | -81.9 +TT | -81.7 +TT | -81.6 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
| 12 | n66 | 15 | -89.5 +TT | -88.8 +TT | -88.3 +TT | -87.8 +TT |  |  | -87.7 +TT |  |  |  |  |  |
|  |  | 30 |  | -89.1 +TT | -88.4 +TT | -88.0 +TT |  |  | -87.8 +TT |  |  |  |  |  |
|  |  | 60 |  | -89.5 +TT | -88.7 +TT | -88.2 +TT |  |  | -87.9 +TT |  |  |  |  |  |
| 12 | n774, 5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 12 | n78 | 15 |  | -85.4 +TT | -85.1 +TT | -84.9 +TT | -84.6+TT | -84.4+TT | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  | -85.7 +TT | -85.2 +TT | -85.1 +TT | -84.7+TT | -84.5+TT | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  | -86.1 +TT | -85.5 +TT | -85.3 +TT | -84.9+TT | -84.6+TT | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| 13 | n774, 5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 14 | n774, 5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 18， 19 | n774,5 n784,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 21 | n28 | 15 | -96.0 +TT | -93.0 +TT | -91.0 +TT | -88.3 +TT |  | -76.0 +TT |  |  |  |  |  |  |
|  |  | 30 |  | -93.1 +TT | -91.1 +TT | -89.5 +TT |  | -76.1 +TT |  |  |  |  |  |  |
| 28 | n774,5 n784,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 20 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n41 | 15 |  | -84.5 +TT | -84.6 +TT | -84.4 +TT |  |  | -83.6 +TT | -83.3 +TT | 3.9 +TT | 3.1 +TT | 2.7 +TT |  |
|  |  | 30 |  | -84.8 +TT | -84.7 +TT | -84.6 +TT |  |  | -83.7 +TT | -83.4 +TT | -83.0 +TT | -82.5 +TT | -82.4 +TT |  |
|  |  | 60 |  | -85.2 +TT | -85.0 +TT | -84.8 +TT |  |  | -83.9 +TT | -83.5 +TT | -83.2 +TT | -82.5 +TT | -82.4 +TT |  |
| 26 | n776,7 n786,7 | 15 |  | -85.0 +TT | -84.9 +TT | -84.7 +TT |  |  | -83.6 +TT | -84.6 +TT |  |  |  |  |
|  |  | 30 |  | -85.3 +TT | -85.0+TT | -84.9 +TT |  |  | -83.7 +TT | -84.7 +TT | -84.7 +TT | -84.6 +TT | -84.6 +TT | -84.6 +TT |
|  |  | 60 |  | -85.7 +TT | --85.3 +TT | --85.1 +TT |  |  | -83.9 +TT | -84.8 +TT | -84.8 +TT | -84.7 +TT | -84.7 +TT | -84.7 +TT |
| 26 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -85.6 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -85.7 +TT | -84.5 +TT |
| n28 | 18,9,10 | N/A | -89.1 | -88.7 | -88.3 | -88.0 |  |  |  |  |  |  |  |  |
| 71 | n211 | 15 | -93.4 +TT | -93.8 +TT | -92.3 +TT | -91.2 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.1 +TT | -92.4 +TT | -91.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.5 +TT | -92.7 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |
| 71 | n212 | 15 | -96.3 +TT | -93.8 +TT | -92.3 +TT | -91.2 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.1 +TT | -92.4 +TT | -91.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.5 +TT | -92.7 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |
| n71 | 211 | N/A | -92.7 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 212 | N/A | -95.6 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 7 | N/A | -82.7 | -82.6 | -82.4 | -82.3 |  |  |  |  |  |  |  |  |
| 66 | n772, 13 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 66 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2+TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 | N/A | -70.7 | -70.6 | -70.8 | -70.8 |  |  |  |  |  |  |  |  |
| n66 | 48,3 | N/A | -96.1 | -93.6 | -92.3 | -91.6 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 15: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 16: Void NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for PC3 EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n2 | 71 | N/A | -69.7 | -69.9 | -70.1 | -71.2 |  |  |  |  |  |  |  |
| n41 | 264 | N/A | -72.5 | -69.5 | -69.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | N/A | -91.2 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.910 | -9210 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n77 | 13 | N/A |  | -65.3 | -65.3 |  |  |  |  |  |  |  |  |
| n77 | 12 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 14 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 282 | N/A | -69.8 | -69.8 | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -90.6 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.310 | -92.010 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 214 | N/A | -60.0 | -60.0 | -60.0 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-2a: Reference sensitivity due to receiver harmonic mixing for PC2 EN-DC in NR FR11

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 2 | N/A | -88.2 | -86.3 | -85.5 | -84.6 |  |  |  |  |  |  |  |
|  |  |  | -90.910 | -8910 | -88.210 | -87.310 |  |  |  |  |  |  |  |
| n77 | 132 | N/A |  | -62.3 | -62.3 |  |  |  |  |  |  |  |  |
| n77 | 12 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 14 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n79 | 192 | N/A | -66.8 | -66.8 | -66.8 |  |  |  |  |  |  |  |  |
| n79 | 214 | N/A | -57.0 | -57.0 | -57.0 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-3 with uplink configuration specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.5-3: Reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n1 | 3 | N/A | -93.3 | -91 | -89.5 | -88.5 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | -94 +TT | -91.6 +TT | -90.1 +TT | -89.1 +TT | -88.1 +TT | -87.4 +TT | -86.2 +TT |  |  |  |  |  |  |
| 1 | n41 | 30 |  | -89.0 +TT | -87.0 +TT | -85.9 +TT |  | -83.9 +TT | -82.6 +TT | -81.6 +TT | -80.8 +TT |  | -79.5 +TT | -79.0 +TT | -78.6 +TT |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
|  |  |  |  | -97.1 +TT4 | -95.1 +TT4 | -94 +TT4 |  | -92.0 +TT4 | -90.7 +TT4 | -89.7 +TT4 | -88.9 +TT4 |  | -87.6 +TT4 | -87.1 +TT4 | -86.7 +TT4 |
| n5 | 28 | 15 | -93.3 | -91.8 | -90.8 | -90 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 15 | -90.2 | -87.2 | -85.4 | -84.2 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | N/A | 3 | n41 | 30 | 3 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | N/A | -95.2 | -92.2 | -90.4 | -89.2 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| n41 | 66 | N/A | -95.3 | -92.3 | -90.5 | -89.3 |  |  |  |  |  |  |  |  |  |
| 41 | n773 | 30 |  | -87.3 +TT | -85.3 +TT | -84.1 +TT |  |  | -82.9 +TT | -82.9 +TT | -82.9 +TT |  | -82.1 +TT | -81.7 +TT | -81.3 +TT |
| 30 | n66 | 15 | -91.2 +TT | -88.0 +TT | -86.2 +TT | -85.0 +TT |  |  |  | -81.8 +TT |  |  |  |  |  |
| n78 | 71 | N/A | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 30 |  | -87.8 +TT | -85.8 +TT | -84.6 +TT |  |  | -83.4 +TT | -83.4 +TT | -83.4 +TT |  | -82.6 +TT | -82.2 +TT | -81.8 +TT |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-3a: Reference sensitivity exceptions due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87.0 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
|  |  |  |  | -97.1 +TT3 | -95.1 +TT3 | -94 +TT3 |  | -92.0 +TT3 | -90.7 +TT3 | -89.7 +TT3 | -88.9 +TT3 |  | -87.6 +TT3 | -87.1 +TT3 | -86.7 +TT3 |
| n41 | 3 | N/A | -94.0 | -91.0 | -89.2 | -88.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | -96.73 | -93.73 | -91.93 | -90.73 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-4 with uplink configuration specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n8A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n77A. DC_2A_n77(2A) | 2 | N/A | -71.3 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -89.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n78A | 2 | N/A | -71.37 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.37 | - | - | - | - | N/A | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| DC_3A_n8A | 3 | N/A | - | REFSENS | - | - | - | N/A | FDD |
|  | n8 | 15 | -88.3 | - | - | - | - | IMD43 | FDD |
|  | 3 | N/A | - | -86.9 | - | - | - | IMD5 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  |  |  | -90.87 | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  |  |  | -88.37 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n66A | 5 | N/A | -67.3 | - | - | - | - | IMD23 | FDD |
|  | n66 |  | REFSENS | - | - | - | - | N/A | FDD |
| DC_5A_n77A8e | 5 | N/A | -89 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 5 | N/A | -91.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n20A | n20 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 8 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_12A_n77A DC_12A_n77(2A) | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_14A_n77A DC 14A n77(2A) | 14 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20_n7 | 20 | N/A | -84.3 | - | - | - | - | IMD3 | FDD |
|  | n7 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20_n8 | 20 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
|  | n8 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_30A_n77A DC_30A_n77(2A) | 30 | N/A | -92.8 | - | - | - | - | IMD3 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) | 66 | N/A | -67.8 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_71A_n66A | 71 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n66 | 15 | -94.5 +TT | - | - | - | - | IMD4 | FDD |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-5: Reference sensitivity exceptions due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm)) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order) | Duplex mode |
| DC_1A_n78A | 1 | N/A | REFSENS-17.8 | - | - | - | - | IMD4 |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n77A | 2 | N/A | -65.2 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A |  |  | -65.157 |  |  |  |  |  |  |
| DC_2A_n77C | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A-2A_n77C | 2 | N/A | -78.2 | - | - | - | - | IMD49 | FDD |
|  |  |  | -78.157 |  |  |  |  |  |  |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n41A | 3 | N/A | -77.9 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.67 | - | - | - | - |  |  |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -64.4 | - | - | - | - | IMD2 | FDD |
|  |  |  | -67.17 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -77.8 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.57 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n77A8 | 5 | N/A | -78.7 | - | - | - | - | IMD49 | FDD |
| DC_5A_n77C8 | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12A_n77A | 12 | N/A | -84.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -80.93 | - | - | - | - | IMD5 | FDD |
| DC_13A_n77C | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_14A_n77A | 14 | N/A | -84.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n79A | 21 | N/A | -65.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | REFSENS |  | - | - | N/A | TDD |
| DC_30A_n77A | 30 | N/A | -81.4 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | N/A | -64.47 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -87.53 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

#### 7.3B.2.3_1	Reference sensitivity for EN-DC within FR1 (>2 CCs)

##### 7.3B.2.3_1.1	Reference sensitivity for EN-DC within FR1 (3 CCs)

Editor's note:	The test requirements section of this test case contains some EN-DC configurations that have no test points defined meaning these cannot be tested.

Editor's note:	For EN-DC configurations with >1NR CC, test points in Table 7.3B.2.3_1.1.4.1-0 only need to be tested if the configuration is a max NR CC of the UE, where detailed execution rules are FFS.

7.3B.2.3_1.1.1	Test purpose

To verify the ability of UE that support EN-DC configurations to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when no CA exceptions are allowed and single carrier requirements apply whenever possible. A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B.2.3_1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 3CC EN-DC.

7.3B.2.3_1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.3_1.1.4	Test description

7.3B.2.3_1.1.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.1.4.1-0 and 7.3B.2.3_1.1.4.1-1.

Table 7.3B.2.3_1.1.4.1-0: Test Configuration Table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | DC_XA_nYA-nZA Mid range for MCG and SCG DC_XA_nYC: Mid range for XA Low range, High Range for nXC DC_(n)XCA: Low range, High Range DC_XA_nY(2A): TBD |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB" columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCC1 – EUTRA/NR |  |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range |  | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc/UL alloc | UL MOD | DL MOD |  | UL/DL Ch BW | DLalloc/ ULalloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc/UL alloc |
| Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | X | default |  |  |  | nX | default |  |  |
|  | QPSK | QPSK | Highest | All RBs / REFSENS_LTE | N/A | QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA-XA_nXA/ DC_XA_(n)XAA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid |  |  | nY | default |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| 2 | X | Mid |  |  | nY | default |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | DFT-s-OFDM QPSK | CP-OFDM QPSK |  | Highest | All RBs / REFSENS_NR | N/A | CP-OFDM QPSK | Highest | All RBs / 0 |
| Note 1: LTE anchor agnostic configuration Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: LTE anchor agnostic configuration with LTE CA where LTE CA fallback to 1CC is sufficient to test Note 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 5: For UE supporting multiple EN-DC configurations with the same NR CA combination, only one EN-DC configuration is tested. Note 6: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  | EN-DC configurations containing the following band combinations: 20-n78: Mid in band 20 and Mid in band 78. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB" and "E-UTRA NRB" columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  |  |  |  |  |  |  |  |  | CG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  | NRB |  |  |  | Band |  | Range |  |  |  | NRB |  |  |  |  |  |  |  | Band |  |  | Range |  | NRB |  |  |  |
|  | UL MOD | DL MOD |  | CH BW |  | DLalloc/UL alloc |  | UL MOD |  | DL MOD |  |  |  | UL/DL Ch BW |  |  |  |  |  | DLalloc/ ULalloc |  | UL MOD |  |  | DL MOD |  | UL/DL Ch BW |  | DLalloc/UL alloc |  |
| Default Test Settings for a DC_XA_nYC, DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_1A-3A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1975 / DL 2165 MHz |  |  |  |  |  | 3 |  | DL 1818.5 MHz |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs |  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / All RBs |  |  |
| 2 | 3 | UL 1780 / DL 1875 MHz |  |  |  |  |  | 1 |  | DL 2139 MHz |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs |  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / 0 |  | DFT-s- OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / All RBs |  |  |
| Test Settings for DC_1A-7A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1935 / DL 2125 MHz |  |  |  |  |  | 7 |  | DL 2653 MHz |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 718 / DL 773 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs |  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-3A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  | 3 |  | DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3757.5 MHz |  |  |  |  |  |
|  | QPSK |  | QPSK |  | 5 MHz | All RBs / All RBs |  | N/A |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  | 3 |  | DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3980 MHz |  |  |  |  |  |
|  | QPSK |  | QPSK |  | 5 MHz | All RBs / All RBs |  | N/A |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 1 |  | DL 2140 MHz |  |  |  |  | 3 |  | UL 1775 MHz / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3915 MHz |  |  |  |  |  |
|  | N/A |  | QPSK |  | 5 MHz | All RBs / 0 |  | QPSK |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  | All RBs / All RBs |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-3A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | 3 |  |  | DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3757.5 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | DL 2125 MHz |  |  |  |  | 3 |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3725 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-3A_n79A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2140 MHz |  |  |  |  | 3 |  |  | UL 1750 / DL 1845 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4860 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-5A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2122 MHz |  |  |  |  | 5 |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3780 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | UL 1975 / DL 2165 MHz |  |  |  |  | 5 |  |  | DL 885 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3405 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1977.5 / DL 2167.5 MHz |  |  |  |  | 7 |  |  | DL 2627.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3305 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | DL 2140 MHz |  |  |  |  | 7 |  |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3580 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 10MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-18A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2120 MHz |  |  |  | 18 |  |  | UL 825 / DL 870 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3770 MHz |  |  |  |  |  |  |
|  | N/A |  | QPSK | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  | All RBs / All RBs |
| Test Settings for DC_1A-19A_n79A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  | 19 |  |  | DL 882.5 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4782.5 MHz |  |  |  |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  | All RBs / All RBs |
| 2 | 1 |  | DL 2140 MHz |  |  |  | 19 |  |  | UL 837.5 / DL 882.5 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4652.5 MHz |  |  |  |  |  |  |
|  | N/A |  | QPSK | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  | All RBs / All RBs |
| Test Settings for DC_1A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1925/ DL 2115 MHz |  |  |  |  | 20 |  |  | DL 805 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2120 MHz |  |  |  |  | 20 |  |  | UL 835 / DL 794 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3790 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3330 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-21A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2154.6 MHz |  |  |  |  | 21 |  |  | UL1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3605 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | DL 2154.6 MHz |  |  |  |  | 21 |  |  | UL1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3647 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | 21 |  |  | DL 1500 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3450 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 4 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | 21 |  |  | DL 1500 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3675 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-28A_n3A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2139 MHz |  |  |  |  | 28 |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A_n28A-n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | n78 |  |  | 3416 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 733 / DL 788 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / All RBs |  |
| 2 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | n28 |  |  | DL 790 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3320 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-28A_n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | UL 740 / DL 795 MHz |  |  |  |  | 1 |  |  | DL 2150 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3630 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | UL 1970 / DL 2160 |  |  |  |  | 28 |  |  | DL 794 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3352 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-41A_n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1970 / DL 2160 MHz |  |  |  |  | 41 |  |  | 2510 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3400 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | DL 2140 MHz |  |  |  |  | 41 |  |  | 2640 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3710 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 1 | UL 1930 / DL 2120 MHz |  |  |  |  | 41 |  |  | 2510 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 4150 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-41A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1935 / DL 2125 MHz |  |  |  |  | 41 |  |  | 2653 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 718 / DL 773 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-42A_n79A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1977.5 / DL 2167.5 MHz |  |  |  |  | 42 |  |  | DL 3490 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A_n5A-n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | UL 1880 / DL 1960 MHz |  |  |  |  | n5 |  |  | UL 830 / DL 875 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3540 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| Test Settings for DC_2A-13A_n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | DL 1944 MHz |  |  |  |  | 13 |  |  | UL 783 / DL 752 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3510 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-66A_n5A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | UL 1900 / DL 1980 MHz |  |  |  |  | 66 |  |  | UL 1740 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 830 / DL 875 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-66A_n41A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | UL 1860 / DL 1940 MHz |  |  |  |  | 66 |  |  | UL 1715 / DL 2115 MHz |  |  |  |  |  |  |  |  |  |  | n41 |  |  | 2685 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-66A_n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | UL 1855 / DL 1935 MHz |  |  |  |  | 66 |  |  | DL 2115 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3970 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 2 | UL 1880 / DL 1960 MHz |  |  |  |  | 66 |  |  | DL 2140 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3500 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 2 | UL 1860 / DL 1940 MHz |  |  |  |  | 66 |  |  | DL 2175 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3915 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 4 | 2 | DL 1960 MHz |  |  |  |  | 66 |  |  | UL 1760 / DL 2160 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3720 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-7A_n5A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1780/ DL 1875 MHz |  |  |  |  | 7 |  |  | DL 2625 MHz |  |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 845 / DL 890 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-7A_n8A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1780/ DL 1875 MHz |  |  |  |  | 7 |  |  | DL 2670 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 890 / DL 935 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-7A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1712.5 / DL 1807.5 MHz |  |  |  |  | 7 |  | DL 2682 MHz |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz | All RBs / All RBs | N/A |  | QPSK |  |  | 10 MHz |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | UL 2543 / DL 2663 MHz |  |  |  |  | 3 |  | DL 1832.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz | All RBs / All RBs | N/A |  | QPSK |  |  | 5 MHz |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1820 MHz |  |  |  |  | 7 |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3310 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 3 | DL 1820 MHz |  |  |  |  | 7 |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3475 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-8A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1820 MHz |  |  |  |  | 8 |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3640 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-19A_n79A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1775 / DL 1870 MHz |  |  |  |  | 19 |  |  | DL 885 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4435 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| 2 | 3 | DL 1877.5 MHz |  |  |  |  | 19 |  |  | UL 842.5 / DL887.5 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4420 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1860 MHz |  |  |  |  | 20 |  |  | UL 840 MHz / DL 799 |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 900 / DL 945 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1828 MHz |  |  |  |  | 20 |  |  | UL 852 / DL 811 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1820 MHz |  |  |  |  | 20 |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3510 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-21A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1767.5 / DL 1862.5 MHz |  |  |  |  | 21 |  |  | DL 1507.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3795 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 3 | DL 1862.5 |  |  |  |  | 21 |  |  | UL 1459.5 / DL 1507.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3322 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-21A_n79A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1869.2 MHz |  |  |  |  | 21 |  |  | UL 1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4770 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | UL 740 / DL 760 MHz |  |  |  |  | 3 |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3350 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1750/ DL 1845 MHz |  |  |  |  | n78 |  |  | 3764 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-40A_n1A Configuration – Note 3, 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1735 / DL 1830 MHz |  |  |  |  | 40 |  |  | DL 2380 MHz |  |  |  |  |  |  |  |  |  |  | n1A |  |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-41A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1832.5 MHz |  |  |  |  | 41 |  |  | 2543 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 3 | UL 1780 / DL 1875 MHz |  |  |  |  | 41 |  |  | 2518 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 738 / DL 793 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 3 | 3 | UL 1715 / DL 1810 MHz |  |  |  |  | 41 |  |  | 2687 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-41A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1720 / DL 1815 MHz |  |  |  |  | 41 |  |  | 2640 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3900 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 3 | DL 1840 MHz |  |  |  |  | 41 |  |  | 2620 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3400 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_5A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 | UL 844 / DL 889 MHz |  |  |  |  | 7 |  |  | DL 2645 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3489 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 5 | DL 879 MHz |  |  |  |  | 7 |  |  | UL 2550 / DL 2670 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3429 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 5 | DL 875 MHz |  |  |  |  | 7 |  |  | UL 2525 / DL 2645 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3350 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-8A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2530 / DL 2650 MHz |  |  |  |  | 8 |  |  | DL 940 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1735 / DL 1830 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2670 MHz |  |  |  |  | 8 |  |  | UL 890 / DL 935 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n1A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2510 / DL 2630 MHz |  |  |  |  | 20 |  |  | 800 MHz |  |  |  |  |  |  |  |  |  |  | n1 |  |  | UL 1940 / DL 2130 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2543 / DL 2663 MHz |  |  |  |  | 20 |  |  | DL 806 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1737 / DL 1832 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2630 MHz |  |  |  |  | 20 |  |  | UL 855 / DL 896 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2565 / DL 2685 MHz |  |  |  |  | 20 |  |  | DL 795 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 885 / DL 930 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2640 MHz |  |  |  |  | 20 |  |  | UL 840 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 900 / DL 945 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 3 | 7 | DL 2624 MHz |  |  |  |  | 20 |  |  | UL 857 / DL 816 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | UL 842/ DL 801 MHz |  |  |  |  | 7 |  |  | DL 2640 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2560 / DL 2680 MHz |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3370 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 7 | UL 2560 / DL 2680 MHz |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3435 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 7 | DL 2675 MHz |  |  |  |  | 20 |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3520 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-28A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2543 / DL 2663 MHz |  |  |  |  | 28 |  |  | DL 796 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | 1842 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2685 MHz |  |  |  |  | 28 |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | 1810 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-28A_n5A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2540 / DL 2660 MHz |  |  |  |  | 28 |  |  | DL 776 MHz |  |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2630 MHz |  |  |  |  | 28 |  |  | UL 730 / DL 785 MHz |  |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 840 / DL 874 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2567.5 / DL 2687.5 MHz |  |  |  |  | 28 |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3350 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 7 | UL 2567.5 / DL 2687.5 MHz |  |  |  |  | 28 |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3460 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 7 | DL 2650 MHz |  |  |  |  | 28 |  |  | UL 740 / DL 795 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3390 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2565/ DL 2685 MHz |  |  |  |  | n78 |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | UL 2565/ DL 2685 MHz |  |  |  |  | n28 |  |  | DL 800 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3365 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-14A_n66A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | DL 1954 MHz |  |  |  |  | 14 |  |  | UL 793 / DL 763 MHz |  |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1770 / DL 2170 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_13A_n2A-n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 13 | UL 782 / DL 751 MHz |  |  |  |  | n2 |  |  | UL 1896 / DL 1976 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3460 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| 2 | 13 | UL 782 / DL 751 MHz |  |  |  |  | n2 |  |  | DL 1960 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3524 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_13A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 13 | UL 782 / DL 751 MHz |  |  |  |  | 66 |  |  | DL 2156 MHz |  |  |  |  |  |  |  |  |  |  | n2 |  |  | UL 1860 / DL 1940 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_13A-66A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 13 | UL 782 / DL 751 MHz |  |  |  |  | 66 |  |  | DL 2156 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3720 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_14A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 | DL 763 MHz |  |  |  |  | 66 |  |  | UL 1762 / DL 2162 MHz |  |  |  |  |  |  |  |  |  |  | n2 |  |  | UL 1874 / DL 1954 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_18A-41A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 18 | DL 865 MHz |  |  |  |  | 41 |  |  | 2640 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3527.5 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10MHz |  | All RBs / All RBs |  |
| Test Settings for DC_18A-41A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 18 | DL 865 MHz |  |  |  |  | 41 |  |  | 2640 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3527.5 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10MHz |  | All RBs / All RBs |  |
| Test Settings for DC_19A-21A_n79A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | UL 837.5 / DL 882.2 MHz |  |  |  |  | 21 |  |  | DL 1500 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4850 |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40MHz |  | All RBs / All RBs |  |
| Test Settings for DC_20A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | UL 857 / DL 816 MHz |  |  |  |  | n78 |  |  | 3314 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 20 | UL 837 / DL 796 MHz |  |  |  |  | n28 |  |  | UL 744 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3310 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 5MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10MHz |  | All RBs / All RBs |  |
| Test Settings for DC_20A_n28A-n75A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Mid |  |  |  |  | n28 |  |  | High |  |  |  |  |  |  |  |  |  |  | n75 |  |  | DL 1476MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 20MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 | N/A |  |  | CP-OFDM QPSK |  |  | 20MHz |  | All RBs / N/A |  |
| 2Note 8 | 20 | Mid |  |  |  |  | n28 |  |  | Low |  |  |  |  |  |  |  |  |  |  | n75 |  |  | High |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 20MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 | N/A |  |  | CP-OFDM QPSK |  |  | 20MHz |  | All RBs / N/A |  |
| Test Settings for DC_28A_n7A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | UL 745 / DL 8005 MHz |  |  |  |  | n78 |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  | n7 |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 28 | UL 745 / DL 8005 MHz |  |  |  |  | n7 |  |  | DL 2650 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3390 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_66A_n2A-n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | UL 1760 / DL 2160 MHz |  |  |  |  | n2 |  |  | DL 1960 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3720 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_66A_n5A-n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | UL 1770 / DL 2170 MHz |  |  |  |  | n5 |  |  | UL 845 / DL 890 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3460 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| Note 1: Void Note 2: Void Note 3: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. Note 4: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 6: Test only applicable for to UEs not supporting UE capability singleUL-Transmissionn for the correspondent uplink configuration. Note 7: NR CA fallback to 1CC is sufficient to test, unless both NR cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 8: Test ID with UL harmonic exception avoided. Note 9: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 10: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.1.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2.

7.3B.2.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with Note 3 in Table 7.3B.2.3_1.1.4.1-1, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3_1.1.5	Test requirement

Reference sensitivity test requirements for EN-DC configurations affected by 3 band 2UL intermodulation interference, are specified in Table 7.3B.2.3_1.1.5-1 and Table 7.3B.2.3_1.1.5-2 with uplink configuration specified in Table 7.3B.2.3_1.1.4.1-1.

Reference sensitivity test requirements for test points in Table 7.3B.2.3_1.1.4.1-1, are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.3_1.1.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | Test ID | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 3 | N/A | -92.3 | - | - | - |  | IMD5 |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 1 | N/A | -88.3 | - | - | - |  | IMD4 |  |
| DC_1A-7A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 7 | N/A | - | -64.3 | - | - |  | IMD2 |  |
| DC_1A-3A_n77A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -64.8 | - | - | - |  | IMD2 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -87.8 | - | - | - |  | IMD4 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | -68.3 | - | - | - | FDD | IMD2 |  |
|  |  | 3 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1 | N/A | -91.0 | - | - | - | FDD | IMD4 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -65.1 | - | - | - |  | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | -96.5 | - | - | - | FDD | IMD5 |  |
|  |  | 3 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-3A_n79A | 1 | 1 | N/A | -95.7 | - | - | - | FDD | IMD5 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_1A-5A_n78A | 1 | 1 | N/A | -91.0 | - | - | - | FDD | IMD4 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | -81.2 | - | - | - | FDD | IMD3 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -94.2 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-7A_n78A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 7 | N/A | -88.2 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -90.6 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | - | REFSENS | - |  | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-18A_n77A | 1 | 1 | N/A | -82.9 | - | - | - | FDD | IMD3 |  |
|  |  | 18 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-19A_n79A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -81.0 | - | - | - | FDD | IMD3 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 2 | 1 | N/A | -91.2 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_1A-20A_n8A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -87.8 | - | - | - | FDD | IMD4 |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-20A_n78A | 1 | 1 | N/A | -79.0 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-20A_n78A | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -93.3 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| ee | 1 | 1 | N/A | -68.7 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -95.7 | - | - | - | FDD | IMD5 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -67.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -96.4 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-28A_n3A | 1 | 28 | N/A | REFSENS |  | - | - | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS |  | - | - | FDD | N/A |  |
|  |  | 1 | N/A | -88.3 |  | - | - | FDD | IMD4 |  |
| DC_1A_n28A-n78A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -79.1 | - | - | TDD | IMD3 |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n28 | 15 | -94.5 | - | - | - | FDD | IMD5 |  |
| DC_1A-28A_n78A | 1 | 1 | N/A | -83.6 | - | - | - | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -93.6 | - | - |  | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-41A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | - | -64.3 | - | - | TDD | IMD2 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-41A_n77A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -86.3 | - | - | - | TDD | IMD4 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -90.0 | - | - | - | FDD | IMD4 |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -93.7 | - | - | - | TDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-42A_n79A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  |  | 42 | N/A | -93.2 | - | - | - | TDD | IMD5 |  |
|  | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A_n5A-n77A |  | n5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | -78.3 | - | - | TDD | IMD3 |  |
|  | 1 | 2 | N/A | -81.3 | - | - | - | FDD | IMD3 |  |
| DC_2A-13A_n77A |  | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_2A-14A_n66A | 1 | 2 | N/A | -90.1 | - | - | - | FDD | IMD4 |  |
|  |  | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n66 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A-66A_n5A | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -91.6 | - | - | - | FDD | IMD4 |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A-66A_n41A | 1 | 2 | N/A | -74.4 | - | - | - | FDD | IMD4 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n41 | 15 | REFSENS | - | - | - | TDD | N/A |  |
|  | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -69.6 | - | - | - | FDD | IMD2 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -88.4 | - | - | - | FDD | IMD4 |  |
| DC_2A-66A_n77A |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -94.8 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 2 | N/A | -65.2 | - | - | - | FDD | IMD2 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-5A_n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -70.3 | - | - | - | FDD | IMD2 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 3 | N/A | -88.3 | - | - | - | FDD | IMD4 |  |
|  |  |  |  | [TBD] |  |  | 10.75 |  |  |  |
| DC_3A-7A_n5A | 1 | 3 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 7 | N/A | - | - 64.3 | - | - | FDD | IMD2XX |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_3A-7A_n28A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 7 | N/A | - | -77.4 | - | - |  | IMD3 |  |
|  | 2 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 3 | N/A | -70.3 | - | - | - |  | IMD2 |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A | 1 | 3 | N/A | -78.7 | - | - | - | FDD | IMD3 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -87.7 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-8A_n78A | 1 | 3 | N/A | -79.8 | - | - | - | FDD | IMD3 |  |
|  |  | 8 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-19A_n79A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -80.8 | - | - | - | FDD | IMD3 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 2 | 3 | N/A | -96.1 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_3A-20A_n28A | 1 | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -86.9 | - | - | - | FDD | IMD4 |  |
| DC_3A-20A_n78A | 1 | 3 | N/A | -79.0 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-21A_n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -90.5 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -65.5 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-21A_n79A | 1 | 3 | N/A | -78.5 | - | - | - | FDD | IMD3 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_3A_n28A-n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -90.3 | - | - | TDD | IMD5 |  |
| DC_3A-28A_n78A | 1 | 3 | N/A | -79 | - | - | - | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-40A_n1A | 1 | n1 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 40 | N/A | -91.3 | - | - | - | TDD | IMD5 |  |
| DC_3A-41A_n28A | 1 | 3 | N/A | -70.3 | - | - | - | FDD | IMD2 |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n28 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -69.9 | - | - | - | TDD | IMD2 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | TDD | N/A |  |
|  | 3 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -81.4 | - | - | - | TDD | IMD3 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | TDD | N/A |  |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | -92 | - | - | - | TDD | IMD5 |  |
|  | 2 | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 3 | N/A | -79.9 | - | - | - | FDD | IMD3 |  |
| DC_5A-7A_n78A | 1 | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 7 | N/A | -67.2 | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 5 | N/A | -67.1 | - | - | - | FDD | IMD2 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 5 | N/A | -94.0 | - | - | - | FDD | IMD5 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A-8A_n3A | 1 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 8 | N/A | -78.3 | - | - | - | FDD | IMD3 |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | - | -65.3 | - | - | FDD | IMD2+ IMD3YY |  |
|  |  | 8 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n1A | 1 | 7 |  |  | REFSENS |  |  | FDD | N/A |  |
|  |  | 20 |  |  | -88.8 |  |  | FDD | IMD5 |  |
|  |  | n1 |  | REFSENS |  |  |  | FDD | N/A |  |
| DC_7A-20A_n3A | 1 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 20 | N/A | - | -82.8 | - | - | FDD | IMD2 |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | - | -68.3 | - | - | FDD | IMD2 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n3 | 15 | - | REFSENS | - | - | FDD | N/A |  |
| DC_7A-20A_n8A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -78.9 | - | - | - | FDD | IMD3 |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | -76.2 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 3 | 7 | N/A | -78.5 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n28A | 1 | 7 | N/A | - | -91.4 | - | - | FDD | IMD5 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -65.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -93.3 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 7 | N/A | -66.5 | - | - | - | FDD | IMD2 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A-28A_n3A | 1 | 7 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | 28 | N/A | -77.8 |  |  |  | FDD | IMD2 |  |
|  |  | n3 | 15 | REFSENS |  |  |  | FDD | N/A |  |
|  | 2 | 7 | N/A | -79.3 |  |  |  | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS |  |  |  | FDD | N/A |  |
| DC_7A-28A_n5A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -93.4 | - | - | - | FDD | IMD5 |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | -91.9 | - | - | - | FDD | IMD5 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-28A_n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -89.5 | - | - | - |  | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -94.8 | - | - | - |  | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 7 | N/A | -66.8 | - | - | - | FDD | IMD2 |  |
|  |  | 28 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A_n28A-n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -65.1 | - | - | TDD | IMD2 |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | -69.0 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A_n2A-n77A |  | n77 | 15 | - | -77 | - | - | TDD | IMD3 |  |
|  | 2 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | -81 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A-66A_n2A |  | 66 | N/A | -91.3 | - | - | - | FDD | IMD4 |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A-66A_n77A |  | 66 | N/A | -81.4 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | FDD | N/A |  |
| DC_18A-41A_n77A DC_18A-41C_n77A | 1 | 18 | N/A | -95.9 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 1 | 18 | N/A | -95.9 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
| DC_19A-21A_n79A | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -95.5 | - | - | - | FDD | IMD5 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_20A_n28A-n78A | 1 | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -86.1 | - | - | TDD | IMD4 |  |
|  | 2 | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n28 | 15 | -89.1+TT | - | - | - | FDD | IMD4 |  |
| DC_28A_n7A-n78A | 1 | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n7 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -65.1 | - | - | TDD | IMD2 |  |
|  | 2 | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n7 | 15 | -66.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_14A-66A_n2A | 1 | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -91.2 | - | - | - | FDD | IMD4 |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 1 | n2 | 15 | -65.2 | - | - | - | FDD | IMD2 |  |
| DC_66A_n2A-n77A |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_66A_n5A-n77A |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | -77.7 | - | - | TDD | IMD39 |  |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 6: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the Band 46. The reference sensitivity should only be verified when this is not the case (the requirements for Band 46 specified in the CA_7A-46A in clause 7.3.1 of TS 36.101 [5] apply). NOTE 7: This band is subject to IMD3 also which MSD is not specified. NOTE 8: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| NOTE 1: Both of the transmitters shall be set min (+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |  |  |

Test tolerance is the same as given in Table 7.3B.2.3.5-2.

##### 7.3B.2.3_1.2	Reference sensitivity for EN-DC within FR1 (4 CCs)

7.3B.2.3_1.2.1	Test purpose

Same as in clause 7.3B.2.3.1.

7.3B.2.3_1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 CCs EN-DC.

7.3B.2.3_1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

For EN-DC combinations with no exception requirements applicable to NR or E-UTRA, LTE anchor agnostic approach is applied.

For EN-DC combinations with exceptional requirements, LTE anchor agnostic approach is not applied.

7.3B.2.3_1.2.4	Test description

7.3B.2.3_1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for E-UTRA consist of the test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1, with the exception that the E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration under test.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5B.2-1. The 4CC EN-DC configurations listed in table 7.3B.2.3_1.2.4.1-0 shall not be tested according to TR 38.905 [7] test point analysis. The other 4CC configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-1 for NR band. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C2.

Table 7.3B.2.3_1.2.4.1-0: 4CC EN-DC configurations that shall not be tested

| EN-DC type | E-UTRA CA | NR CA | Notation |
| --- | --- | --- | --- |
| Intra-band non-contiguous EN-DC (1 band) | Yes (cont) | No | DC_XD_nXA |
|  | Yes (non-cont) | No | Note 4 |
| Inter-band EN-DC | Yes (all types) | No | DC_XD_nYA, DC_XA-YC_nZA, DC_XA-XA-YA_nZA, DC_XA-YA-ZA_nRA |
|  | Yes (all types) | Yes (cont) | DC_XC_nYC, DC_XA-XA_nYC, DC_XA-YA_nYC |
|  |  | Yes (non-cont) | DC_XC_nY(2A), DC_XA-YA_nY(2A) |
|  |  | Yes (inter) | DC_XC_nYA-nZA, DC_XA-XA_nYA-nZA, DC_XA-YA_nZA-nRA |
| Inter-band + Intra-band contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA_(n)ZAA, DC_XC_(n)YAA |
|  | Yes | Yes | DC_(n)XCA-nYA |
| Intra-band non contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA-ZA_nZA |
|  | Yes | Yes | DC_XA-YA_nYA-nZA |
| NOTE 1: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 2: The table apply to all band numbers, i.e. all values of X, Y and Z defined in TS38.101-3 NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: No such config in TS 38.101-3 [7] V16.5.0x |  |  |  |

Table 7.3B.2.3_1.2.4.1-1: Test Configuration Table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | DC_(n)XDA: Low range, High Range DC_XA_nYA-nZC: Mid range for X and nY Low range, High Range for nY DC_XA_nY(2A)-nZA: Mid range for X, nY, nZ |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB" columns |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_(n)XDA Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 (Note 6) | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS_LTE | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nX |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| 2 (Note 7) | PCC(M) | X |  | Highest NRB | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nX |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| Default Test Settings for a DC_XA_nY(2A)-nZA Configuration – Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | nY |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | nZ |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nY |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | nY |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | nY |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nZ |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| Default Test Settings for a DC_XA_nYA-nZC Configuration– Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | nZ |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | nZ |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nY |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | nZ |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | nY |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nZ |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: LTE anchor agnostic configuration NOTE 5: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases NOTE 6: Test point for UE supporting dual UL NOTE 7: Test point for UE supporting single UL NOTE 8: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 9: UE supporting multiple EN-DC configurations with the same NR CA combination, only one EN-DC configuration is tested. NOTE 10: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |

For 4CC EN-DC configurations affected by exceptions, exception testing can be covered by 2CC or 3CC fallback configurations. No exception test points are needed in this test case.

Table 7.3B.2.3_1.2.4.1-2: Void

Table 7.3B.2.3_1.2.4.1-3: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.3B.2.3_1.2.4.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.3B.2.3_1.2.4.2	Test procedure

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3_1.2.4.2.1-1 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3_1.2.4.2.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to NR carrier to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

7.3B.2.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

7.3B.2.3_1.2.5	Test Requirement

Reference sensitivity test requirements for EN-DC configurations are specified in Table 7.3.5-1 in TS 36.521-1 [10] for the LTE CC, and Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.3_1.2.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

FFS

Table 7.3B.2.3_1.2.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

FFS

##### 7.3B.2.3_1.3	Reference sensitivity for EN-DC within FR1 (5 CCs)

7.3B.2.3_1.3.1	Test purpose

Same as in clause 7.3B.2.3.1.

7.3B.2.3_1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5 CCs inter-band EN-DC.

7.3B.2.3_1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.3_1.3.4	Test description

7.3B.2.3_1.3.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.3.4.1-1 and Table 7.3B.2.3_1.3.4.1-2

Table 7.3B.2.3_1.3.4.1-1: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (four bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB" column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA_ZC_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC3(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.3.4.1-2: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (five bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB " column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA-ZA-SA_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC3(M) | S | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different E-UTRA bands i.e. X != Y != Z, R corresponds to NR band. NOTE 3: For inter-band EN-DC 4CCs configuration with four bands, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1, testing for 4CC with four band can be skipped. |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.3.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2.

7.3B.2.3_1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.

7.3B.2.3_1.3.5	Test requirement

For inter-band EN-DC configurations, the throughput of each CG shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Tables Table 7.3.2.5-1 in TS 38.521-1 [8] and parameters specified Tables 7.3.2.4.1-1, Tables 7.3.2.4.1-2 and Tables 7.3.2.4.1-3 in TS 38.521-1 [8] for NR band.

Each EN-DC combination defined in Table 5.5B.4.3-1 shall be tested in anchor-agnostic mode as described in this clause. If a test point is overlapped with uplink EN-DC combo MSD test points, the requirement with MSD shall apply.

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable for two, three, four and five bands operation.

Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3_1.3.5-1 for four bands and Table 7.3B.2.3_1.3.5-2 for five bands with uplink configuration specified in Table 7.3B.2.3_1.3.4.1-1 for four bands and 7.3B.2.3_1.3.4.1-2 for five bands, respectively.

For a given inter-band EN-DC 5CCs configuration defined in Table 5.5B.4.3-1 and Table 5.5B.4.4-1, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1 and Table 5.5B.4.4-1, the test requirement for default fallback uplink EN-DC configurations defined in Table 5.5B.4.3-1 applies.

Table 7.3B.2.3_1.3.5-1 Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (four bands)

TBD

Table 7.3B.2.3_1.3.5-2 Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (five bands)

TBD

##### 7.3B.2.3_1.4	Reference sensitivity for EN-DC within FR1 (6 CCs)

TBD

#### 7.3B.2.4	Reference sensitivity for Inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 2 and 4.

7.3B.2.4.1	Test purpose

Same test purpose as in clause 7.3.2.1 in TS 38.521-2 [9] for the NR carrier.

7.3B.2.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR DL CC.

7.3B.2.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.0.4 TS 3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.3B.2.4.

7.3B.2.4.4	Test description

7.3B.2.4.4.1	Initial conditions

Same test description as in clause 7.3.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 7.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.3.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 7.3.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.3B.2.4.5	Test requirement

Same test requirement as in clause 7.3.2.5 in TS 38.521-2 [9] for the NR carrier.

#### 7.3B.2.4_1	Reference sensitivity for Inter-band EN-DC including FR2 (>1 NR CC)

##### 7.3B.2.4_1.1	Reference sensitivity for Inter-band EN-DC including FR2 (2 NR CCs)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.1.1	Test purpose

Same test purpose as in clause 7.3B.2.4.1.

7.3B.2.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 2 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.1.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.1.4	Test description

For inter-band EN-DC including FR2 UE configured as 2 NR DL CCs and 1 LTE DL CC, the test description of 2DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.1.4 in TS 38.521-2 [9] with the exceptions described below.

7.3B.2.4_1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.5.1, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-2 [9] clause 5.3A and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2 or clause 5.4.2A. All valid configurations shall be tested with applicable test parameters for inter-band EN-DC including FR2 configuration specified in clause 5.5B.5, and the configuration for NR carrier are shown in TS 38.521-2 [9] Table 7.3A.2.1.4.1-1.

For Initial conditions as in clause 7.3A.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are configured as per clause 4.7 with parameters set according to Table 4.7-1 and propagation conditions set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.3A.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.3B.2.4_1.1.4.2	Test Procedure

Same test procedure as in clause 7.3A.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.3B.2.4_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1.

7.3B.2.4_1.1.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.2	Reference sensitivity for Inter-band EN-DC including FR2 (3 NR CCs)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.2.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 3 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.2.4	Test description

For inter-band EN-DC including FR2 UE configured as 3 NR DL CCs and 1 LTE DL CC, the test description of 3DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.2.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.2.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.3	Reference sensitivity for Inter-band EN-DC including FR2 (4 NR CCs)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.3.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 4 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.3.4	Test description

For inter-band EN-DC including FR2 UE configured as 4 NR DL CCs and 1 LTE DL CC, the test description of 4DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.3.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.3.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.4	Reference sensitivity for Inter-band EN-DC including FR2 (5 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.4.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 5 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.4.4	Test description

For inter-band EN-DC including FR2 configured as 5 NR DL CCs and 1LTE DL CC, the test description of 5DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.4.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.4.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.5	Reference sensitivity for Inter-band EN-DC including FR2 (6 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.5.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 6 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.5.4	Test description

For inter-band EN-DC including FR2 configured as 6 NR DL CCs and 1LTE DL CC, the test description of 6DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.5.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.5.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.6	Reference sensitivity for Inter-band EN-DC including FR2 (7 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.6.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 7 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.6.4	Test description

For inter-band EN-DC including FR2 configured as 7 NR DL CCs and 1LTE DL CC, the test description of 7DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.6.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.6.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.7	Reference sensitivity for Inter-band EN-DC including FR2 (8 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.7.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 8 NR DL CCs in intra-band contiguous configuration.

7.3B.2.4_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.7.4	Test description

For inter-band EN-DC including FR2 configured as 8 NR DL CCs and 1LTE DL CC, the test description of 8DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.7.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.7.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

### 7.3B.2.4D	Reference sensitivity for inter-band EN-DC including FR2 for UL MIMO

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach can be applied and only NR carriers need to be tested.

No test case details are specified. Given UE’s Rx performance would not be impacted by the Tx configuration in NR FR2 TDD bands, the requirements in this test case can be well covered in 7.3B.2.4 and 7.3B.2.4_1 and don’t need to be tested again.

#### 7.3B.2.5	Reference sensitivity for Inter-band EN-DC including FR1 and FR2 (3 CCs)

7.3B.2.5.1	Test purpose

Same test purpose as in 7.3B.2 in TS 38.521-1 [8] for NR FR1 carrier(s) and 7.3.2 in TS 38.521-2 [9] for NR FR2 carrier(s).

7.3B.2.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for reference sensitivity apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 7.3B.

#### 7.3B.2.5D	Reference sensitivity for inter-band EN-DC including FR1 and FR2 for UL MIMO

7.3B.2.5D.1	Test purpose

Same test purpose as in clause 7.3D in TS 38.521-1 [8] for NR FR1 carrier and 7.3D in TS 38.521-2 [9] for NR FR2 carrier.

7.3B.2.5D.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for reference sensitivity apply and are tested as part of the EN-DC within FR1 as in clause 7.3 in TS 38.521-1 [8] and EN-DC within FR2 as in clause 7.3 in TS 38.521-2 [9].

#### 7.3B.2.6	Void

### 7.3B.3	ΔRIB,c ΔRIBNC for EN-DC

#### 7.3B.3.0	General

For the UE which supports inter-band EN-DC or NE-DC configuration, the minimum requirement for reference sensitivity in Table 7.3.1-1 and Table 7.3.1-1a in TS.36101 [5], clause 7.3.2, 7.3A.2, 7.3C.2 in TS 38.101-1 [2] and clause 7.3.2, 7.3A.2 in TS 38.101-2 [3] shall be increased by the amount given in ΔRIB,c ΔRIBNC in Tables below where unless otherwise stated, the same ΔRIB,c, ΔRIBNC are applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔRIB,c or ΔRIBNC is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in TS 38.101-1 [2] and 7.3A, 7.3B in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied.

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in TS 38.101-1 [2] and 7.3A, 7.3B in TS 38.101-3 [4] for the applicable operating bands.

Unless ΔRIB,c is specified for the NE-DC configuration, the specified ΔRIB,c for the EN-DC configuration including same bands as the corresponding NE-DC configuration is applicable for the NE-DC configuration.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.3B.3.

#### 7.3B.3.1	Reference sensitivity ΔRIB,c for Intra-band Contiguous EN-DC

FFS

#### 7.3B.3.2	Reference sensitivity ΔRIB,c for Intra-band non-contiguous EN-DC

Table 7.3B.3.2-1: Intra-band non-contiguous EN-DC with one uplink configuration on E-UTRA for reference sensitivity

| DC configuration | Aggregated channel bandwidth (LTE+NR) |  | Wgap / (MHz) | UL E-UTRA allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA | NR |  |  |  |  |

#### 7.3B.3.3	ΔRIB,c for Inter-band EN-DC within FR1

##### 7.3B.3.3.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.3.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | n28 | 0.2 |
| DC_1_n41 | 1 | 0 |
|  | n41 | 0 |
| DC_1_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78 | n78 | 0.5 |
| DC_2_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 2 | 0.2 |
|  | n77 | 0.5 |
| DC_2_n78 | 2 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n41 | n41 | 03 |
|  |  | 0.54 |
| DC_3_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5_n78 | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_7_n8 | n8 | 0.2 |
| DC_7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_7_n71 | n71 | 0.2 |
| DC_7_n77 | n77 | 0.5 |
| DC_7_n78, DC_7-7_n78 | n78 | 0.5 |
| DC_8_n77 | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_11_n77 | n77 | 0.5 |
| DC_11_n78 | n78 | 0.5 |
| DC_12_n5 | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_12_n66 | 12 | 0.5 |
| DC_12_n77 | 12 | 0.2 |
|  | n77 | 0.5 |
| DC_12_n78 | 12 | 0.2 |
|  | n78 | 0.5 |
| DC_14_n77 | 14 | 0.2 |
|  | n77 | 0.5 |
| DC_18_n77 | n77 | 0.5 |
| DC_19_n77 | n77 | 0.5 |
| DC_19_n78 | n78 | 0.5 |
| DC_20_n78 | n78 | 0.5 |
| DC_21_n77 | n77 | 0.5 |
| DC_21_n78 | n78 | 0.5 |
| DC_25_n41 | n41 | 01 |
|  |  | 0.52 |
| DC_26_n77 | n77 | 0.5 |
| DC_26_n78 | n78 | 0.5 |
| DC_28_n51 | n51 | 0.2 |
| DC_28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_30_n77 | n77 | 0.5 |
| DC_38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_39-n41 | 39 | 0.2 |
|  | n41 | 0.2 |
| DC_39_n79 | n79 | 0.5 |
| DC_40_n78 | 40 | 0.45 |
|  | n78 | 0.55 |
| DC_40_n79 | n79 | 0.5 |
| DC_41_n28 | 41 | 0 |
|  | n28 | 0 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_48_n66 | 48 | 0.5 |
|  | n66 | 0.2 |
| DC_66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_66_n25 | 66 | 0.3 |
|  | n25 | 0.3 |
| DC_66_n41 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_66_n77 DC_66-66_n77 | 66 | 0.2 |
|  | n77 | 0.5 |
| DC_66_n78 | 66 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 3: Applicable for the frequency range of 2515-2690 MHz. NOTE 4: Applicable for the frequency range of 2496-2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

##### 7.3B.3.3.2	ΔRIB,c for EN-DC in three bands

Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | n28 | 0.2 |
| DC_1-3_n41 | n41 | 03 |
| DC_1-3_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n3-n78 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3_n79 | n79 | 0.5 |
| DC_1-5_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n28 | n28 | 0.2 |
| DC_1-7_n78 | 1 | 0.2 |
|  | 7 or n7 | 0.2 |
|  | n78 | 0.5 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_1-8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-18_n77 | n77 | 0.5 |
| DC_1-19_n77 | n77 | 0.5 |
| DC_1-19_n78 | n78 | 0.5 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-20_n78 | n78 | 0.5 |
| DC_1-21_n77 | n77 | 0.5 |
| DC_1-21_n78 | 1 | 0.2 |
|  | n78 | 0.5 |
| DC_1-28_n78 DC_1_n28-n78 | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
| DC_1-41_n28 | n28 | 0.2 |
| DC_1-41_n41 | 1 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |
| DC_1-41_n77 | n77 | 0.5 |
| DC_1-41_n78 | n78 | 0.5 |
| DC_1-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-42_n78 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-42_n79 | 42 | 0.5 |
| DC_1_n77-n79 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78-n79 | n78 | 0.5 |
| DC_2-5_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0.2 |
| DC_2_n5-n77 | 2 | 0.2 |
|  | n77 | 0.5 |
| DC_2-13_n66 DC_2-2-13_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | 2 | 0.2 |
| DC_2-13_n77 | 13 | 0.2 |
|  | n77 | 0.5 |
| DC_2-29_n260 | 2 | 0 |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.4 |
|  | 30 | 0.5 |
| DC_2-30_n66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_2-66_n5 DC_2A-2A-66A_n5A DC_2-66-66_n5 DC_2A-2A-66A-66A_n5A | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_2-66_n41 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_2-66_n71B | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 2 | 0.3 |
| DC_2-66_n77 | 66 | 0.3 |
|  | n77 | 0.5 |
| DC_2-66_n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |
| DC_3_n1-n77 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n1-n78 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_3-5_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7_n8 | n8 | 0.2 |
| DC_3-7_n78 DC_3-7-7_n78 | 3 | 0.2 |
|  | 7 or n7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8_n28 | 8 | 0.2 |
|  | n28 | 0.1 |
| DC_3-8_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-18_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-18_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-19_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20_n28 | 20 | 0.1 |
|  | n28 | 0.1 |
| DC_3-20_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 28 | 0.1 |
|  | n5 | 0.1 |
|  | n41 | 01/0.52 |
| DC_3-28_n78 DC_3_n28-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-41_n28 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0 |
| DC_3-41_n41 | 41 | 03 |
|  |  | 0.54 |
|  | n41 | 03 |
|  |  | 0.54 |
| DC_3-41_n77 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n77 | 0.5 |
| DC_3-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-42_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5-7_n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.52 |
| DC_5-30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_7_n1-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_7-8_n3 | 8 | 0.2 |
| DC_7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_7-20_n78 | n78 | 0.5 |
| DC_7-28_n78 | n78 | 0.5 |
| DC_7_n28-n78 | n78 | 0.5 |
| DC_13_n2-n77 | n2 | 0.2 |
|  | n77 | 0.5 |
| DC_13-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | 13 | 0.3 |
| DC_13-66_n77 | 66 | 0.3 |
|  | n77 | 0.5 |
| DC_14-66_n2 DC_14-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_18-41_n77 | n77 | 0.5 |
| DC_18-41_n78 | n78 | 0.5 |
| DC_19-21_n77 | n77 | 0.5 |
| DC_19-21_n78 | n78 | 0.5 |
| DC_19-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-42_n79 | 42 | 0.5 |
| DC_19_n77-n79 | n77 | 0.5 |
| DC_19_n78-n79 | n78 | 0.5 |
| DC_20_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_20-38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_20_n76-n78 | n78 | 0.5 |
| DC_21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-42_n79 | 42 | 0.5 |
| DC_21_n77-n79 | n77 | 0.5 |
| DC_21_n78-n79 | n78 | 0.5 |
| DC_28_n7-n78 | n78 | 0.5 |
| DC_28-41_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.5 |
| DC_28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_66_n5 | 66 | 0.4 |
|  | n5 | 0.5 |
| DC_41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
|  | 66 | 0.3 |
| DC_66_n2-n77 | n2 | 0.3 |
|  | n77 | 0.5 |
| DC_66_n5-n77 | 66 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

##### 7.3B.3.3.3	ΔRIB,c for EN-DC in four bands

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7_n28 | n28 | 0.2 |
| DC_1-3-7_n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 or n7 | 0.3 |
|  | n78 | 0.5 |
| DC_1-3-8_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-19_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-20_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-21_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-21_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-28_n78 DC_1-3_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3_n77-n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78-n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3_SUL_n78-n80 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-7-20_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-28_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-19-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-19-42_n79 | 42 | 0.5 |
| DC_1-19_n77-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.5 |
| DC_1-19_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.5 |
| DC_1-20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-21-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-21-42_n79 | 42 | 0.5 |
| DC_1-21_n77-n79 | n77 | 0.5 |
| DC_1-21_n78-n79 | n78 | 0.5 |
| DC_1-42_n77-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-42_n78-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_2-7-13_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_2-7-66_n66 DC_2-7-7-66_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 |  |
| DC_2-7-66_n78 DC_2-7-7-66_n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |
|  | 2 | 0.3 |
| DC_2-13-66_n77 | 66 | 0.3 |
|  | n77 | 0.5 |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_2-14-66_n66 DC_2-2-14-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_2-30-66_n5 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | 2 | 0.3 |
| DC_2-66_n5-n77 | 66 | 0.3 |
|  | n77 | 0.5 |
| DC_2-66-(n)71 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_3-7_n1-n78 | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.5 |
| DC_3-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.1 |
| DC_3-7-20_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-28_n78 DC_3-7_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-19-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-19-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-19-42_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-19-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3-19_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20_n28-n78 | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21_n77-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21_n78-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-41-42_n77 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-41-42_n78 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-41-42_n79 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
| DC_3-42_n77-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-42_n78-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_7-20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_12-30-66_n260 | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | 66 | 0.2 |
| DC_13-66_n2-n77 | n2 | 0.2 |
|  | n77 | 0.5 |
| DC_19-21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-21-42_n79 | 42 | 0.5 |
| DC_19-21_n77-n79 | n77 | 0.5 |
| DC_19-21_n78-n79 | n78 | 0.5 |
| DC_19-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515 MHz. |  |  |

##### 7.3B.3.3.4	ΔRIB,c for EN-DC in five bands

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-7-20_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7-28_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-19-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-20_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-21-42_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.2 |
| DC_1-3-21-42_n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n79 | 0.0 |
| DC_1-7-20_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n79 | 42 | 0.5 |
| DC_1-19-42_n77-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-21-42_n77-n79 | 1 | 0.2 |
|  | 21 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-42_n78-n79 | 21 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-7-20_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_3-19-21-42_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-19-21-42_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_19-21-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-21-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

##### 7.3B.3.3.5	ΔRIB,c for EN-DC six bands

Table 7.3B.3.3.5-1: ΔRIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

#### 7.3B.3.3a	ΔRIB,c for Inter-band NE-DC within FR1

Unless ΔRIB,c is specified in this clause, the value of ΔRIB,c for the correspondingly specified EN-DC configuration in clause 7.3B.3.3 is applicable.

#### 7.3B.3.4	Reference sensitivity for ΔRIB,c Inter-band EN-DC including FR2

The ΔRIB,c for NR FR2 band of inter-band CA defined in tables 5.5B.5.1-1 to 5.5B.5.5-1 is set to zero.

#### 7.3B.3.5	Reference sensitivity for ΔRIB,c Inter-band EN-DC including both FR1 and FR2

The ΔRIB,c for NR FR2 band of inter-band CA defined in tables 5.5B.6.2-1 to 5.5B.6.5-1 is set to zero and ΔRIB,c for constituent E-UTRA and NR FR1 bands is the same as those for the corresponding inter band EN-DC configuration without the NR FR2 bands specified in 7.3B.3.3.

### 7.3B.4	EIS Spherical Coverage for Inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 2 and 4.

7.3B.4.1	Test purpose

Same test purpose as in clause 7.3.4.1 in TS 38.521-2 [9] for the NR carrier.

7.3B.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR DL CC.

7.3B.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3.4.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-2 [9] clause 7.3.4.

7.3B.4.4	Test description

7.3B.4.4.1	Initial conditions

Same test description as in clause 7.3.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 7.3.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.3.4.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 7.3.4.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.3B.4.5	Test requirement

Same test requirement as in clause 7.3.4.5 in TS 38.521-2 [9] for the NR carrier.

## 7.4	Void

## 7.4A	Maximum Input Level for CA

7.4A.1	Test purpose

Same test purpose as in clause 7.4 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 7.4 in TS 38.521-2 [9] for NR FR2 carrier(s).

7.4A.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for maximum input level apply and are tested in TS 38.521-1 [8] clause 7.4 and 7.4A and TS 38.521-2 [9] clauses 7.4 and 7.4A.

## 7.4B	Maximum Input Level for DC

### 7.4B.0	Minimum conformance requirement

#### 7.4B.0.1	Intra-band contiguous EN-DC

Intra-band contiguous EN-DC maximum input level requirement and parameters are defined in Table 7.4B.0.1-1.

Table 7.4B.0.1-1: Maximum Input

| Power in Largest CC, E-UTRA or NR, dBm | X1 |
| --- | --- |
| Power in each other CC, dBm | X1 – 10*log10(NxSCSx/NySCSy) |
| NOTE 1: Power in Largest E-UTRA or NR bandwidth CC, listed in Table 7.4-1 [2] NOTE 2: Nx, SCSx is the number of RB's and Sub carrier spacing in the largest carrier bandwidth and could be LTE or NR carrier NOTE 3: Ny, SCSy is the number of RB's in any other carrier. NOTE 4: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L as defined in clause 6.2B.4. NOTE 5: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 [5] with PCMAX_L as defined in clause 6.2B.4. |  |

#### 7.4B.0.2	Intra-band non-contiguous EN-DC

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.4.1 for single carrier operation and in clause 7.4.1A for CA in TS 36.101 [5].

For the NR sub-block, the requirement is defined in clause 7.4 in TS 38.101-1 [2].

#### 7.4B.0.3	Inter-band EN-DC within FR1

Maximum input level requirement for E-UTRA single carrier and CA operation specified in clauses 7.4.1 and 7.4.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.4 and 7.4A of TS 38.101-1 [2] apply.

#### 7.4B.0.4	Inter-band EN-DC including FR2

Maximum input level requirement for E-UTRA single carrier and CA operation specified in clauses 7.4.1 and 7.4.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.4 and 7.4A of TS 38.101-2 [3] apply.

#### 7.4B.0.5	Inter-band EN-DC including both FR1 and FR2

Maximum input level requirement for E-UTRA single carrier and CA operation specified in clauses 7.4.1 and 7.4.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.4 and 7.4A of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.4B.

### 7.4B.1	Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs)

7.4B.1.1	Test purpose

Maximum input level for intra-band contiguous EN-DC tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB or a gNB.

7.4B.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.4B.1.3	Minimum conformance requirements

Refer to Clause 7.4B.0.1 for the intra-band contiguous EN-DC maximum input level requirement.

Exception requirements are defined for this test, therefore LTE agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

The normative reference for this requirement is TS 38.101-3 [4] Clause 7.4B.1.

7.4B.1.4	Test Description

7.4B.1.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in table 5.5B.2-1, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each intra-band contiguous EN-DC configuration specified in clause 5.3B.1.2, and are shown in table 7.4B.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A.2 for E-UTRA RMC for FDD , and TS 38.521-1 [8] Annex A.2 for NR RMC Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.4B.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 5) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  | Lowest |  |  |  |
| Test Parameters for Intra-band Contiguous EN-DC Configuration |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  | Uplink Configuration |  |  |  |  |
| Test ID | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation |  | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| 1 | CP-OFDM 64QAM | Full RB (NOTE 1) | CP-OFDM 64QAM | Full RB | DFT-s-OFDM QPSK |  | NOTE 2 | QPSK | NOTE 3 |
| 2 | CP-OFDM 256QAM | Full RB (NOTE 1) | CP-OFDM 256QAM | Full RB | DFT-s-OFDM QPSK |  | NOTE 2 | QPSK | NOTE 3 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Same RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 3: Same RB allocation shall be used per the E-UTRA band and channel BW as specified in Table 7.3.3-2 of TS 36.521 [10]. NOTE 4: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE5: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.1 for SS diagram and A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1,C.2,C3.1 and TS 38.521-1 [8] Annex C.0,C.1,C.2,C3.1for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H.0,H.1,H.2,H.3.1 and TS 38.521-1 [8] Annex G.0,G.1,G.2,G.3.1 for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.4B.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.4B.1.4.2	Test Procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 7.4B.1.4.1 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.4B.1.4.1-1 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	SS sets the Downlink signal level for the E-UTRA CC and NR CC to the value defined in Table 7.4B.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.4B.1.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Measure the average throughput for each component carrier for duration sufficient to achieve statistical significance according to Annex H.2 in TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

Table 7.4B.1.4.2-1: Void

7.4B.1.4.3	Message Contents

Message contents are according to TS 36.508-1 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.4B.1.5	Test Requirement

The throughput measurement of each CC derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels with parameters specified in Table 7.4B.1.5-1

Table 7.4B.1.5-1: Maximum input level requirement for each CC

| Channel bandwidth of Largest BW CC | Rx Parameter |  |  |  |
| --- | --- | --- | --- | --- |
|  | Power in the Largest CC | Power in the other CC | Power in the Largest CC | Power in the other CC |
| 5 MHz | -252-TT | -252 -10*log10(NxSCSx/NySCSy) -TT | -273-TT | -273-10*log10(NxSCSx/NySCSy) -TT |
| 10 MHz | -252-TT | -252 -10*log10(NxSCSx/NySCSy) -TT | -273-TT | -273-10*log10(NxSCSx/NySCSy) -TT |
| 15 MHz | -252-TT | -252 -10*log10(NxSCSx/NySCSy) -TT | -273-TT | -273-10*log10(NxSCSx/NySCSy) -TT |
| 20 MHz | -252-TT | -252 -10*log10(NxSCSx/NySCSy) -TT | -273-TT | -273-10*log10(NxSCSx/NySCSy) -TT |
| 25 MHz | -242-TT | -242 -10*log10(NxSCSx/NySCSy) -TT | -263-TT | -263-10*log10(NxSCSx/NySCSy) -TT |
| 30 MHz | -232-TT | -232 -10*log10(NxSCSx/NySCSy) -TT | -253-TT | -253-10*log10(NxSCSx/NySCSy) -TT |
| 40 MHz | -222-TT | -222 -10*log10(NxSCSx/NySCSy) -TT | -243-TT | -243-10*log10(NxSCSx/NySCSy) -TT |
| 50 MHz | -212-TT | -212 -10*log10(NxSCSx/NySCSy) -TT | -233-TT | -233-10*log10(NxSCSx/NySCSy) -TT |
| 60 MHz | -202-TT | -202 -10*log10(NxSCSx/NySCSy) -TT | -223-TT | -223-10*log10(NxSCSx/NySCSy) -TT |
| 80 MHz | -202-TT | -202 -10*log10(NxSCSx/NySCSy) -TT | -223-TT | -223-10*log10(NxSCSx/NySCSy) -TT |
| 90 MHz | -202-TT | -202 -10*log10(NxSCSx/NySCSy) -TT | -223-TT | -223-10*log10(NxSCSx/NySCSy) -TT |
| 100 MHz | -202-TT | -202 -10*log10(NxSCSx/NySCSy) -TT | -223-TT | -223-10*log10(NxSCSx/NySCSy) -TT |
| NOTE 1: Nx, SCSx is the number of RB's and Sub carrier spacing in the largest carrier bandwidth and could be LTE or NR carrier. NOTE 2: Reference measurement channel refers to Clauses A.3.2.3 or A.3.3.3 in TS 38.521-1 [8] for 64-QAM NR Carrier, and to Tables A.3.2-3, A.3.2-4 for 64QAM in TS 36.521-1 [10] for E-UTRA Carrier. NOTE 3: Reference measurement channel refers to Clauses A.3.2.4 or A.3.3.4 in TS 38.521-1 [8] for 256QAM NR Carrier, and Tables A.3.2-5, A.3.2-6 in TS 36.521-1 [10] for 256QAM E-UTRA Carrier. NOTE 4: Ny, SCSy is the number of RB's in any other carrier NOTE 5: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L as defined in clause 6.2B.4. NOTE 6: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 38.101-3 [4] with PCMAX_L as defined in clause 6.2B.4 for single carrier. NOTE 7: TT for each frequency is specified in Table 7.4B.1.5-2 |  |  |  |  |

Table 7.4B.1.5-2: Test Tolerance (Maximum input level)

| f ≤ 3.0GHz | 3.0GHz < f ≤6.0GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

### 7.4B.2	Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs)

7.4B.2.1	Test purpose

Maximum input level for intra-band non-contiguous EN-DC tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB or a gNB.

7.4B.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC in FR1 with 2 DL CCs.

7.4B.2.3	Minimum conformance requirements

Refer to Clause 7.4B.0.2 for the intra-band non-contiguous EN-DC maximum input level requirement.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.4B.2, and TS 38.101-1 [2] clause 7.4, and TS 38.101-2 [3] clauses 7.4 and 7.4A

No exception requirements for E-UTRA CG and NR CG, LTE agnostic approach applies.

7.4B.2.4	Test Description

Same test description as in clause 7.4.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions in the initial test configuration.

-	The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.3, all of these configurations shall be tested with applicable test parameters for each intra-band non-contiguous EN-DC configuration specified in clause 5.3B.1.3, and are shown in table 7.4B.2.4.1-1.

Table 7.4B.2.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | MaxWGap for intra-band non-contiguous EN-DC |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.3-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Lowest, Mid, Highest of Channel BW for NR CC |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  | Lowest |  |  |  |
| Test Parameters for EN-DC Configuration |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  | Uplink Configuration |  |  |  |  |
| Test ID | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation |  | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 2 | NOTE 2 | DFT-s-OFDM QPSK |  | NOTE 1 | NOTE 2 | NOTE 2 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 2 | NOTE 2 | DFT-s-OFDM QPSK |  | NOTE 1 | NOTE 2 | NOTE 2 |
| NOTE 1: Same RB allocation shall be used per modulation as specified in Table 7.4.4.1-1 of TS 38.521-1 [8]. NOTE 2: Modulation and RB allocation for E-UTRA CC refers to the Table 4.6-2. NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1 with the exception that

-	E-UTRA test frequency are specified in Table 7.4B.2.4.1-1 based on intra-band non-contiguous EN-DC configuration specified in clause 5.3B.1.3.

For Initial conditions as in clause 7.4.4.1 in TS 38.521-1 [8] with the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8] is replaced by the following three steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.4B.2.5	Test Requirement

Same test requirement as in clause 7.4.5 in TS 38.521-1 [8] for NR carrier.

### 7.4B.3	Maximum Input Level for Inter-band EN-DC within FR1 (1 NR CC)

7.4B.3.1	Test purpose

Same test purpose as in clause 7.4.1 in TS 38.521-1 [8] for the NR carrier.

7.4B.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR DL CC.

7.4B.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.4.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.4B.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.4B.3.4	Test Description

Same test description as in clause 7.4.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.4.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.4B.3.5	Test Requirement

Same test requirement as in clause 7.4.5 in TS 38.521-1 [8] for the NR carrier.

### 7.4B.3_1	Maximum Input Level for EN-DC within FR1 (>2 CCs)

#### 7.4B.3_1.1	Maximum Input Level for EN-DC within FR1 (3 CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	Only inter-band EN-DC and intra-band non-contiguous EN-DC within FR1 are considered. Testing of intra-band contiguous EN-DC is FFS.

7.4B.3_1.1.1	Test purpose

Same test purpose as in clause 7.4B.3.1.

7.4B.3_1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band or intra-band non-contiguous EN-DC within FR1 with 2 NR DL CCs and one or more E-UTRA DL CC(s).

7.4B.3_1.1.3	Minimum conformance requirements

Refer to Clause 7.4B.0.2 for the intra-band non-contiguous EN-DC and Clause 7.4B.0.3 for the inter-band EN-DC within FR1 maximum input level requirement.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.4B.3_1.1.4	Test Description

Same test description as in clause 7.4A.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.4A.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.4A.1.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.4B.3_1.1.5	Test Requirement

Same test requirement as specified in TS 38.521-1 [8] clause 7.4A.1.5 for the NR carrier(s).

#### 7.4B.3_1.2	Maximum Input Level for EN-DC within FR1 (4 CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	Only inter-band EN-DC and intra-band non-contiguous EN-DC within FR1 are considered. Testing of intra-band contiguous EN-DC is FFS.

7.4B.3_1.2.1	Test purpose

Same test purpose as in clause 7.4B.3.1.

7.4B.3_1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band or intra-band non-contiguous EN-DC within FR1 with 3 NR DL CCs and one or more E-UTRA DL CC(s).

7.4B.3_1.2.3	Minimum conformance requirements

Refer to Clause 7.4B.0.2 for the intra-band non-contiguous EN-DC and 7.4B.0.3 for the inter-band EN-DC within FR1 maximum input level requirement.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.4B.3_1.2.4	Test Description

Same test description as in clause 7.4A.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.4A.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.4A.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.4B.3_1.2.5	Test Requirement

Same test requirement as specified in TS 38.521-1 [8] clause 7.4A.2.5 for the NR carriers.

#### 7.4B.3_1.3	Maximum Input Level for EN-DC within FR1 (5 CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	Only inter-band EN-DC and intra-band non-contiguous EN-DC within FR1 are considered. Testing of intra-band contiguous EN-DC is FFS.

7.4B.3_1.3.1	Test purpose

Same test purpose as in clause 7.4B.3.1.

7.4B.3_1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band or intra-band non-contiguous EN-DC within FR1 with 4 NR DL CCs and one or more E-UTRA DL CC(s).

7.4B.3_1.3.3	Minimum conformance requirements

Refer to Clause 7.4B.0.2 for the intra-band non-contiguous EN-DC and Clause 7.4B.0.3 for the inter-band EN-DC within FR1 maximum input level requirement.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.4B.3_1.3.4	Test Description

Same test description as in clause 7.4A.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.4A.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.4A.3.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.4B.3_1.3.5	Test Requirement

Same test requirement as specified in TS 38.521-1 [8] clause 7.4A.3.5 for the NR carrier(s).

#### 7.4B.3_1.4	Maximum Input Level for EN-DC within FR1 (6 CCs)

Editor's note:	The test case in this clause is incomplete, the following aspects are either missing or not yet determined:

-	Only inter-band EN-DC and intra-band non-contiguous EN-DC within FR1 are considered. Testing of intra-band contiguous EN-DC is FFS.

-	The referred test case 7.4A.4 in TS 38.521-1 [8] is incomplete.

7.4B.3_1.4.1	Test purpose

Same test purpose as in clause 7.4B.3.1.

7.4B.3_1.4.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band or intra-band non-contiguous EN-DC within FR1 with 5 NR DL CCs and one or more E-UTRA DL CC(s).

7.4B.3_1.4.3	Minimum conformance requirements

Refer to Clause 7.4B.0.2 for the intra-band non-contiguous EN-DC and Clause 7.4B.0.3 for the inter-band EN-DC within FR1 maximum input level requirement.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.4B.3_1.4.4	Test Description

Same test description as in clause [7.4A.4.4] in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause [7.4A.4.4.1] in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause [7.4A.4.4.1] in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.4B.3_1.4.5	Test Requirement

Same test requirement as specified in TS 38.521-1 [8] clause [7.4A.4.5] for the NR carrier(s).

### 7.4B.3a	Maximum Input Level for inter-band NE-DC within FR1 (2 CCs)

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 7.4 and 7.4A of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 7.4 and 7.4A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

### 7.4B.4	Maximum Input Level for inter-band EN-DC including FR2 (1 NR CC)

Editor’s note: The following aspects are either missing or not yet determined:

-	The referred test case 7.4 in TS 38.521-2 is incomplete.

7.4B.4.1	Test purpose

Same test purpose as in clause 7.4 in TS 38.521-2 [9] for the NR carrier.

7.4B.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR DL CC.

The minimum conformance requirements of NR FR2 carrier in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing.

NOTE:	As a result TC 7.4B.4 has not been included in the test case applicability table 4.1.3-1, TS 38.522. This does not preclude the test from being used for R&D or other purposes if deemed useful to all types of NR UE release 15 and forward.

7.4B.4.3	Minimum conformance requirements

Refer to Clause 7.4B.0.4 for the inter-band EN-DC including FR2 maximum input level requirement.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.4B.4.4	Test description

Same test description as in clause 7.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 7.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS36.521-1 [10].

Step 6 of Initial conditions as in clause 7.4.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 7.4.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.4B.4.5	Test requirement

Same test requirement as in clause 7.4.5 in TS 38.521-2 [9] for the NR carrier.

### 7.4B.4_1	Maximum Input Level for inter-band EN-DC including FR2 (>1 NR CC)

#### 7.4B.4_1.1	Maximum Input Level for Inter-Band EN-DC including FR2 (2 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The referred test case 7.4A.1 in TS 38.521-2 is incomplete.

7.4B.4_1.1.1	Test purpose

Same test purpose as in clause 7.4 in TS 38.521-2 [9] for the NR carrier.

7.4B.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR DL CCs.

The minimum conformance requirements of NR FR2 carrier in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing.

NOTE:	As a result TC 7.4B.4_1.1 has not been included in the test case applicability table 4.1.3-1, TS 38.522. This does not preclude the test from being used for R&D or other purposes if deemed useful to all types of NR UE release 15 and forward.

7.4B.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.4B.4.3.

7.4B.4_1.1.4	Test description

Same test description as in 7.4A.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in 7.4A.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS36.521-1 [10].

Step 6 of Initial conditions as in 7.4A.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in 7.4A.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.4B.4_1.1.5	Test Requirements

Same test requirement as in 7.4A.1.5 in TS 38.521-2 [9] for the NR carrier(s).

#### 7.4B.4_1.2	Maximum Input Level for Inter-Band EN-DC including FR2 (3 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The referred test case 7.4A.1 and 7.4A.2 in TS 38.521-2 is incomplete.

7.4B.4_1.2.1	Test purpose

Same test purpose as in clause 7.4 in TS 38.521-2 [9] for the NR carrier.

7.4B.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR DL CCs.

The minimum conformance requirements of NR FR2 carrier in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing.

NOTE:	As a result TC 7.4B.4_1.2 has not been included in the test case applicability table 4.1.3-1, TS 38.522. This does not preclude the test from being used for R&D or other purposes if deemed useful to all types of NR UE release 15 and forward.

7.4B.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.4B.4.3.

7.4B.4_1.2.4	Test description

Same test description as in 7.4A.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 7.4A.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS36.521-1 [10].

Step 6 of Initial conditions as in 7.4A.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in 7.4A.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.4B.4_1.2.5	Test Requirements

Same test requirement as in clause 7.4A.2.5 in TS 38.521-2 [9] for the NR carrier(s).

#### 7.4B.4_1.3	Maximum Input Level for Inter-Band EN-DC including FR2 (4 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The referred test case 7.4A.1, 7.4A.2 and 7.4A.3 in TS 38.521-2 is incomplete.

7.4B.4_1.3.1	Test purpose

Same test purpose as in clause 7.4 in TS 38.521-2 [9] for the NR carrier.

7.4B.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 NR DL CCs.

The minimum conformance requirements of NR FR2 carrier in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing.

NOTE:	As a result TC 7.4B.4_1.3 has not been included in the test case applicability table 4.1.3-1, TS 38.522. This does not preclude the test from being used for R&D or other purposes if deemed useful to all types of NR UE release 15 and forward.

7.4B.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.4B.4.3.

7.4B.4_1.3.4	Test description

Same test description as in 7.4A.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in 7.4A.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS36.521-1 [10].

Step 6 of Initial conditions as in 7.4A.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in 7.4A.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.4B.4_1.2.5	Test Requirements

Same test requirement as in 7.4A.3.5 in TS 38.521-2 [9] for the NR carrier(s).

#### 7.4B.4_1.4	Maximum Input Level for Inter-Band EN-DC including FR2 (5 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The referred test case 7.4A.4 in TS 38.521-2 is incomplete.

7.4B.4_1.4.1	Test purpose

Same test purpose as in clause 7.4 in TS 38.521-2 [9] for the NR carrier.

7.4B.4_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 NR DL CCs.

The minimum conformance requirements of NR FR2 carrier in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing.

NOTE:	As a result TC 7.4B.4_1.4 has not been included in the test case applicability table 4.1.3-1, TS 38.522. This does not preclude the test from being used for R&D or other purposes if deemed useful to all types of NR UE release 15 and forward.

7.4B.4_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.4B.4.3.

7.4B.4_1.4.4	Test description

Same test description as in 7.4A.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in 7.4A.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS36.521-1 [10].

Step 6 of Initial conditions as in clause 7.4A.4.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 7.4A.4.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.4B.4_1.4.5	Test Requirements

Same test requirement as in 7.4A.4.5 in TS 38.521-2 [9] for the NR carrier(s).

### 7.4B.4D	Maximum Input Level for inter-band EN-DC including FR2 for UL MIMO

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach can be applied and only NR carriers need to be tested.

No test case details are specified. Given UE’s Rx performance would not be impacted by the Tx configuration in NR FR2 TDD bands, the requirements in this test case can be well covered in 7.4B.4 and 7.4B.4_1 and don’t need to be tested again.

### 7.4B.5	Maximum Input Level for inter-band EN-DC including both FR1 and FR2

7.4B.5.1	Test purpose

Same test purpose as in clause 7.4.1 in TS 38.521-1 [8] for NR FR1 carrier and 7.4.1 in TS 38.521-2 [9] for NR FR2 carrier.

7.4B.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for maximum input level apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 7.4B.

### 7.4B.5D	Maximum Input Level for inter-band EN-DC including FR1 and FR2 for UL MIMO

7.4B.5D.1	Test purpose

Same test purpose as in clause 7.4D.1 in TS 38.521-1 [8] for NR FR1 carrier and 7.4D.1 in TS 38.521-2 [9] for NR FR2 carrier.

7.4B.5D.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for maximum input level apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 7.4B.

## 7.5	Void

## 7.5A	Adjacent Channel Selectivity for CA

7.5A.0	Test purpose

Same test purpose as in clauses 7.5 and 7.5A in TS 38.521-1 [8] for NR FR1 carrier(s) and clauses 7.5 and 7.5A in TS 38.521-2 [9] for NR FR2 carrier(s).

7.5A.1	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for adjacent channel selectivity apply and are tested in clauses 7.5 and 7.5A in TS 38.521-1 [8] and clauses 7.5 and 7.5A in TS 38.521-2 [9].

## 7.5B	Adjacent channel selectivity for DC

### 7.5B.0	Minimum Conformance Requirements

#### 7.5B.0.1	Intra-band contiguous EN-DC

Intra-band contiguous EN-DC ACS requirement and parameters are defined for test case 1 in Table 7.5B.0.1-1 and for test case 2 in Table 7.5B.0.1-2.

Table 7.5B.0.1-1: ACS test case 1

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| ACS, dB | X1 | 19.2 | 18.5 | 17.9 |
| Pinterferer, dBm | PI 2 | Aggregated power + 17.7 dB | Aggregated power + 17 dB | Aggregated power + 16.4dB |
| Pw in Transmission BW configuration, per CC, dBm | REFSENS +14dB |  |  |  |
| NOTE 1: X is ACS level at the specified EN-DC aggregated Bandwidth from Table 7.5.1A-1 in TS 36.101 [5] NOTE 2: PI is from Table 7.5.1A-2 in TS 36.101 [5] NOTE 3: Jammer BW and offset is from Table 7.5.1A-2 in TS 36.101 [5] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 4: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4 from TS 38.101-3 [4]. NOTE 5: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 from TS 38.101-3 [4] for single carrier. |  |  |  |  |

Table 7.5B.0.1-2: ACS test case 2

| EN-DC Aggregated Bandwidth, ENBW, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | -42.7 +10log10(NRB,c/ NRB_agg) | -42 +10log10(NRB,c/NRB_agg) | -41.4 +10log10(NRB,c/NRB_agg) |
| Pinterferer, dBm | -25 |  |  |  |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.5.1A-3 in TS 36.101 [5] NOTE 2: Jammer BW and offset is from Table 7.5.1A-3 in TS 36.101 [5] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 3: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4 from TS 38.101-3 [4]. NOTE 4: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 from TS 38.101-3 [4] for single carrier. |  |  |  |  |

#### 7.5B.0.2	Intra-band non-contiguous EN-DC

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.5.1 for single carrier operation and in clause 7.5.1A for CA in TS 36.101 [5].

For the NR sub-block, the requirement is defined in clause 7.5 in TS 38.101-1 [2].

The blocker configuration is defined in the general clause 7.1 in TS 38.101-3 [4].

#### 7.5B.0.3	Inter-band EN-DC within FR1

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.5 and 7.5A of TS 38.101-1 [2] apply.

#### 7.5B.0.3a	Inter-band NE-DC within FR1

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in subclauses 7.5.1 and 7.5.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in subclauses 7.5 and 7.5A of TS 38.101-1 [2] apply.

#### 7.5B.0.4	Inter-band EN-DC including FR2

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.5 and 7.5A of TS 38.101-2 [3] apply.

#### 7.5B.0.4a	Inter-band NE-DC including FR2

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.5 and 7.5A of TS 38.101-2 [3] apply.

#### 7.5B.0.5	Inter-band EN-DC including both FR1 and FR2

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.5 and 7.5A of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.5B.1, 7.5B.2, 7.5B.3, 7.5B.3a, 7.5B.4, 7.5B.4a, and 7.5B.5.

### 7.5B.1	Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs)

7.5B.1.1	Test purpose

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR and E-UTRA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

7.5B.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.5B.1.3	Minimum conformance requirements

Refer to Clause 7.5B.0.1 for the intra-band contiguous EN-DC in FR1.

Exception requirements are applicable for NR, therefore LTE anchor agnostic approach is not applied. E-UTRA test points are defined and measurements performed over the aggregated EN-DC bandwidth.

7.5B.1.4	Test description

7.5B.1.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in table 5.5B.2-1, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2.All of these configurations shall be tested with applicable test parameters for each intra-band contiguous EN-DC configuration specified in clause 5.5B.2, and are shown in Table 7.5B.1.4.1-1.

In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements.

The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A.2 for E-UTRA RMC for FDD , TS 38.521-1 [8] Annex A.2 for NR UL RMC and TS 38.521-1 [8] Annex A.3 for NR DL RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.5B.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Lowest supported SCS |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| Test ID | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_NR | QPSK | REFSENS_LTE |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS_NR and REFSENS_LTE refer to Uplink configuration in Table 7.3.2.4.1-1 in TS 38.521-1 [8] and Table 7.3.4.1-1 in TS 36.521 [10] for NR and E-UTRA CC respectively. NOTE 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in A.3.1.1 for SS diagram and A.3.2 for UE diagram in TS 38.508-1 [6].

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.5B.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.5B.1.4.2	Test Procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.5B.1.4.1-1 on the E-UTRA CC and NR CC, respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.5B.1.4.1-1 on the E-UTRA CC and NR CC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.5B.0.1-1 (Case 1). For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.5B.0.1-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Set the Interferer signal level to the value as defined in Table 7.5B.0.1-1 (Case 1) and frequency below the aggregated component carriers, using a modulated interferer bandwidth as defined in Annex D.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2 in TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

6.	Repeat steps from 3 to 5, using an interfering signal above the aggregated component carriers in Case 1 at step 4.

7.	Set the Downlink signal level to the value as defined in Table 7.5B.0.1-2 (Case 2). For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.5B.0.1-2 for at least the duration of the Throughput measurement, where MU and Uplink power control window size are defined above.

8.	Set the Interferer signal level to the value as defined in Table 7.5B.0.1-2 (Case 2) and frequency below the aggregated component carriers, using a modulated interferer bandwidth as defined in Annex D.

9.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2 in TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

10.	Repeat steps from 7 to 9, using an interfering signal above the aggregated component carriers in Case 2 at step 8.

11.	Repeat for applicable channel bandwidths and operating band combinations in both Case 1 and Case 2.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.5B.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Message contents exceptions are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value..

7.5B.1.5	Test requirement

The throughput measurement of each CG derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively under the conditions specified in Table 7.5B.0.1-1, and also under the conditions specified in Table 7.5B.0.1-2.

### 7.5B.2	Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs)

7.5B.2.1	Test purpose

Same test purpose as clause 7.5B.1.1

7.5B.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC in FR1 with 2 DL CCs.

7.5B.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.5B.0.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.5B.2.4	Test description

Same test description as in subclause 7.5.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 7.5B.2.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different DC bandwidth classes | Low with maxWgap, High with maxWgap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.3-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 7.5B.2.4.1-1.

For initial conditions as in clause 7.5.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Step 6 of Initial conditions as in clause 7.5.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Step 3 of Test procedure as in clause 7.6.2.4.2 in TS 38.521-1 [8] shall treat the in-gap tests as below: For intra-band non-contiguous EN-DC of two sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing requirements apply for in-gap tests only if the corresponding interferer frequency offsets satisfy the following condition in relation to the sub-block gap size Wgap, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset)| – BWChannel

Same test procedure as specified in clause 7.5.4.2 in TS 38.521-1 [8].

7.5B.2.5	Test requirement

Same test requirement as specified in TS 38.521-1 [8] Clause 7.5.5.

### 7.5B.3	Adjacent Channel Selectivity for inter-band EN-DC within FR1 (1 NR CC)

7.5B.3.1	Test purpose

Same test purpose as in clause 7.5B.1.1.

7.5B.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR DL CC.

7.5B.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.5B.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.5B.3.4	Test description

Same test description as in clause 7.5.4.2 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.5.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.5.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.5.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.5B.3.5	Test requirement

Same test requirement as specified in TS 38.521-1 [8] Clause 7.5.5.

### 7.5B.3_1	Adjacent Channel Selectivity for EN-DC within FR1 (>2 CCs)

Editor's note: The following aspects are either missing or not yet determined:

-	The ACS for EN-DC within FR1 (2 NR CCs) test cases below cover only inter-band and intra-band non-contiguous EN-DC. Testing of intra-band contiguous EN-DC is FFS.

#### 7.5B.3_1.1	Adjacent Channel Selectivity for EN-DC within FR1 (2 NR CCs)

7.5B.3_1.1.1	Test purpose

Same test purpose as in clause 7.5B.1.1.

7.5B.3_1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous or inter-band EN-DC within FR1 with 2 NR DL CCs.

7.5B.3_1.1.3	Minimum conformance requirements

The minimum conformance requirements for intra-band non-contiguous or inter-band EN-DC within FR1 are defined in clause 7.5B.0.2 and 7.5B.0.3 respectively.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.5B.3_1.1.4	Test description

Same test description as in clause 7.5A.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.5A.1.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.5.4.1 or 7.5A.1.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.5A.1.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.5B.3_1.1.5	Test requirement

Same test requirement as specified in TS 38.521-1 [8] Clause 7.5A.1.5 for the NR carrier(s).

#### 7.5B.3_1.2	Adjacent Channel Selectivity for EN-DC within FR1 (3 NR CCs)

Editor's note: The following aspects are either missing or not yet determined:

-	The ACS for EN-DC within FR1 (3 NR CCs) test cases below cover only inter-band and intra-band non-contiguous EN-DC. Testing of intra-band contiguous EN-DC is FFS.

7.5B.3_1.2.1	Test purpose

Same test purpose as in clause 7.5B.1.1.

7.5B.3_1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous or inter-band EN-DC within FR1 with 3 NR DL CCs.

7.5B.3_1.2.3	Minimum conformance requirements

The minimum conformance requirements for intra-band non-contiguous and inter-band EN-DC within FR1 are defined in clause 7.5B.0.2 and 7.5B.0.3 respectively.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.5B.3_1.2.4	Test description

Same test description as in clause 7.5A.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.5A.2.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.5A.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.5A.2.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.5B.3_1.2.5	Test requirement

Same test requirement as specified in TS 38.521-1 [8] Clause 7.5A.2.5 for the NR carrier(s).

#### 7.5B.3_1.3	Adjacent Channel Selectivity for EN-DC within FR1 (4 NR CCs)

Editor's note: The following aspects are either missing or not yet determined:

-	The ACS for EN-DC within FR1 (4 NR CCs) test cases below cover only inter-band and intra-band non-contiguous EN-DC. Testing of intra-band contiguous EN-DC is FFS.

7.5B.3_1.3.1	Test purpose

Same test purpose as in clause 7.5B.1.1.

7.5B.3_1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous or inter-band EN-DC within FR1 with 4 NR DL CCs.

7.5B.3_1.3.3	Minimum conformance requirements

The minimum conformance requirements for intra-band non-contiguous and inter-band EN-DC within FR1 are defined in clause 7.5B.0.2 and 7.5B.0.3 respectively.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.5B.3_1.3.4	Test description

Same test description as in clause 7.5A.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.5A.3.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.5A.3.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.5A.3.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.5B.3_1.3.5	Test requirement

Same test requirement as specified in TS 38.521-1 [8] Clause 7.5A.3.5 for the NR carriers.

#### 7.5B.3_1.4	Adjacent Channel Selectivity for EN-DC within FR1 (5 NR CCs)

Editor's note: The test cases in this clause are incomplete. The following aspects are either missing or not yet determined:

- The ACS for EN-DC within FR1 (5 NR CCs) test cases below cover only inter-band and intra-band non-contiguous EN-DC. Testing of intra-band contiguous EN-DC is FFS.

- The referred clauses [7.5A.4.4], [7.5A.4.4.1] and [7.5A.4.5] have not been defined in TS 38.521-1 [8] yet.

7.5B.3_1.4.1	Test purpose

Same test purpose as in clause 7.5B.1.1.

7.5B.3_1.4.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous or inter-band EN-DC within FR1 with 5 NR DL CCs.

7.5B.3_1.4.3	Minimum conformance requirements

The minimum conformance requirements for intra-band non-contiguous and inter-band EN-DC within FR1 are defined in clause 7.5B.0.2 and 7.5B.0.3 respectively.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.5B.3_1.4.4	Test description

Same test description as in clause [7.5A.4.4] in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause [7.5A.4.4.1] in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause [7.5A.4.4.1] in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause [7.5A.4.4.1] in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.5B.3_1.4.5	Test requirement

Same test requirement as specified in TS 38.521-1 [8] Clause [7.5A.4.5] for the NR carriers.

### 7.5B.3a	Adjacent Channel Selectivity for inter-band NE-DC within FR1 (2 CCs)

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 7.5 and 7.5A of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 7.5 and 7.5A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

### 7.5B.4	Adjacent Channel Selectivity for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	Following aspects are either missing or not yet determined:

-	Measurement Uncertainty is FFS for power classes other than 1, 3 and 5.

-	The minimum conformance requirements for Case 2 in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed.

7.5B.4.1	Test purpose

Same test purpose as in clause 7.5B.1.1.

7.5B.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR DL CC.

7.5B.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.5B.0.4.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.5B.4.4	Test description

7.5B.4.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.5.1, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-2 [9] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each inter-band EN-DC including FR2 configuration specified in clause 5.5B.5.1, and the configuration for NR carrier are shown in TS 38.521-2 [9] Table 7.5.4.1-1.

For initial conditions as in clause 7.5.4.1 in TS 38.521-2 [9], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

Step 6 of initial conditions as in clause 7.5.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.5B.4.4.2	Test Procedure

Same test procedure as specified in clause 7.5.4.2 in TS 38.521-2 [9] with the following exceptions for E-UTRA anchor

On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.5B.4.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1.

7.5B.4.5	Test requirement

Same test requirement as specified in clause 7.5.5 of TS 38.521-2 [9] for the NR carrier(s).

### 7.5B.4_1	Adjacent Channel Selectivity for inter-band EN-DC including FR2 (>1 NR CC)

#### 7.5B.4_1.1	Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	This test case is not complete. Following aspects are either missing or not yet determined:

-	Working assumption: to avoid LTE CA testing in inter-band EN-DC including FR2 and only PCC band is configured.

-	MU and TT are FFS.

-	[Table 7.5A.1.4.1-1] in TS 38.521-2 [9] where the configuration for NR CA carriers are shown is FFS.

-	[clause 7.5A.1.4.1] in TS 38.521-2 [9] where the initial conditions for NR CA is FFS.

-	[clause 7.5A.1.4.2] in TS 38.521-2 [9] where the test procedure for NR CA is FFS.

-	[Clause 7.5A.1.5] in TS 38.521-2 [9] where the test requirements for NR CA is FFS.

-	[Clause 7.5A.1.4] in TS 38.521-2 [9] where the test description for NR CA is FFS.

-	How to choose the LTE anchor when LTE CA is implemented is FFS.

7.5B.4_1.1.1	Test purpose

Same test purpose as in clause 7.5B.1.1.

7.5B.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR DL CCs.

7.5B.4_1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.5B.0.4.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.5B.4_1.1.4	Test description

For inter-band EN-DC including FR2 UE configured as "2 NR DL CCs and 1 LTE DL CC", the test description of 2DL FR2 CA for adjacent channel selectivity is the same as in corresponding clause 7.5A.1.4 in TS 38.521-2 [9] for FR2 with the exceptions described below.

7.5B.4_1.1.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.5.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-2 [9] clause 5.3A and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each inter-band EN-DC including FR2 configuration specified in clause 5.5B.5.2, and the configuration for NR carriers are shown in TS 38.521-2 [9] [Table 7.5A.1.4.1-1].

For initial conditions as in [clause 7.5A.1.4.1] in TS 38.521-2 [9], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

Step 6 of initial conditions as in [clause 7.5A.1.4.1] in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.5B.4_1.1.4.2	Test Procedure

Same test procedure as specified in [clause 7.5A.1.4.2] in TS 38.521-2 [9] with the following exceptions for E-UTRA anchor

On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.5B.4_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1.

7.5B.4_1.1.5	Test requirement

Same test requirement as specified in TS 38.521-2 [9] [Clause 7.5A.1.5].

#### 7.5B.4_1.2	Void

#### 7.5B.4_1.3	Void

#### 7.5B.4_1.4	Void

### 7.5B.4D	Adjacent Channel Selectivity for inter-band EN-DC including FR2 for UL MIMO

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach can be applied and only NR carriers need to be tested.

No test case details are specified. Given UE’s Rx performance would not be impacted by the Tx configuration in NR FR2 TDD bands, the requirements in this test case can be well covered in 7.5B.4 and 7.5B.4_1 and don’t need to be tested again.

### 7.5B.5	Adjacent Channel Selectivity for inter-band EN-DC including both FR1 and FR2

7.5B.5.1	Test purpose

Same test purpose as in 7.5.1 in TS 38.521-1 [8] for NR FR1 carrier and 7.5.1 in TS 38.521-2 [9] for NR FR2 carrier.

7.5B.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for adjacent channel selectivity apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 7.5B.

### 7.5B.5D	Adjacent Channel Selectivity for inter-band EN-DC including FR1 and FR2 for UL MIMO

7.5B.5D.1	Test purpose

Same test purpose as in 7.5D in TS 38.521-1 [8] for NR FR1 carrier and 7.5D in TS 38.521-2 [9] for NR FR2 carrier.

7.5B.5D.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for adjacent channel selectivity apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 7.5B.

## 7.6	Void

## 7.6A	Blocking characteristics for CA

7.6A.1	Test purpose

Same test purpose as in clause 7.6 in TS 38.521-1 [8] for NR FR1 carrier(s) and clause 7.6 in TS 38.521-2 [9] for NR FR2 carrier(s).

7.6A.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The NR/5GC requirements for blocking characteristics apply and are tested in TS 38.521-1 [8] clause 7.6 and 7.6A and TS 38.521-2 [9] clauses 7.6 and 7.6A.

## 7.6B	Blocking characteristics for DC

### 7.6B.1	General

The blocking characteristic for EN-DC in FR1 is a measure of the receiver's ability of an UE that support EN-DC in FR1 to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

### 7.6B.2	Inband blocking for DC

#### 7.6B.2.0	Minimum Conformance Requirements

##### 7.6B.2.0.1	Intra-band contiguous EN-DC

Intra-band contiguous EN-DC in-band blocking requirement and parameters are defined in Table 7.6B.2.0.1-1.

Table 7.6B.2.0.1-1: In-band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, per CC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | PW 1 | 16.8 | 17.5 | 18 |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.6.1.1A-1 in TS 36.101 [5]. NOTE 2: Interferer values are specified from Table 7.6.1.1A-2 in TS 36.101 [5]. NOTE 3: Jammer BW and offset is from Table 7.6.1.1A-1 in TS 36.101 [5] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier. NOTE 4: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 5: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.2.1.

##### 7.6B.2.0.2	Intra-band non-contiguous EN-DC

For the E-TRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.6.1.1 for single carrier operation and in clause 7.6.1.1A for CA in TS 36.101 [5].

For the NR sub-block, the requirement is defined in clause 7.6.2 in TS 38.101-1 [2].

The blocker configuration is defined in the general clause 7.1.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.2.2.

##### 7.6B.2.0.3	Inter-band EN-DC within FR1

Inband blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.2 and 7.6A.2 of TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.2.3.

##### 7.6B.2.0.3a	Inter-band NE-DC within FR1

Inband blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.2 and 7.6A.2 of TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.2.3a.

##### 7.6B.2.0.4	Inter-band EN-DC including FR2

Inband blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.2 and 7.6A.2 of TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.2.4.

##### 7.6B.2.0.5	Inter-band EN-DC including both FR1 and FR2

Inband blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.2 and 7.6A.2 of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.2.5.

#### 7.6B.2.1	Inband blocking for intra-band contiguous EN-DC (2 CCs)

7.6B.2.1.1	Test Purpose

In-band blocking is defined for an unwanted interfering signal falling into the range from 15MHz below to 15MHz above the UE receive band, at which the relative throughput shall meet or exceed the requirement for the specified measurement channels. The lack of in-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.2.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.6B.2.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6 B.2.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.6B.2.1.4	Test Description

7.6 B.2.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.2.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.2.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  | Lowest |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS | QPSK | REFSENS |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2.3-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.1 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2 , A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.2.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.6B.2.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.2.1.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.2.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.2.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.2.1.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Set the parameters of the signal generator for an interfering signal below the aggregated component carriers in Case 1 according to Table 7.6B.2.1.5-1.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers in Case 1 at step 4.

7.	Repeat steps from 4 to 6, using interfering signals in Case 2 at steps 4 and 6.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.6B.2.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.6B.2.1.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.6B.2.1.5-1: In-band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | PW 1 | 16.8 | 17.5 | 18 |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.6.1.1A-1 in TS 36.101 [5]. NOTE 2: Interferer values are specified from Table 7.6.1.1A-2 in TS 36.101 [5]. NOTE 3: Jammer BW and offset is from Table 7.6.1.1A-1 in TS 36.101 [5] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier. NOTE 4: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in TS 38.101-1 [2], Table 7.3.2-3 with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 5: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in TS 36.101 [5], Table 7.3.1-2 with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

Table 7.6B.2.1.5-2: Void

Table 7.6B.2.1.5-3: Void

#### 7.6B.2.2	Inband blocking for intra-band non-contiguous EN-DC (2 CCs)

7.6B.2.2.1	Test Purpose

Same test purpose as in clause 7.6.2.1 in TS 38.521-1 [8] for the NR carrier.

7.6B.2.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC in FR1 with 2 DL CCs.

7.6B.2.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.2.0.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.2.2.4	Test Description

Same test description as in clause 7.6.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 7.6B.2.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | High range with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1 except for the parameters specified in Table 7.6B.2.2.4-1.

For Initial conditions as in clause 7.6.2.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add steps 7 and 8 to Initial conditions in clause 7.6.2.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Step 3 of Test procedure as in clause 7.6.2.4.2 in TS 38.521-1 [8] shall treat the in-gap tests as below: For intra-band non-contiguous EN-DC of two sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing requirements apply for in-gap tests only if the corresponding interferer frequency offsets satisfy the following condition in relation to the sub-block gap size Wgap, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset)| – BWChannel

Step 4 of Test procedure as in clause 7.6.2.4.2 in TS 38.521-1 [8] is replaced by:

4.	Set the downlink signal level for NR CC according to the Table 7.6.2.5-1 or 7.6.2.5-3 in TS 38.521-1 [8] as appropriate. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of (PCMAX_L,c - 29dB) for E-UTRA CC, and of 4dB below PCMAX_L,f,c for NR CC for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

7.6B.2.2.5	Test Requirement

Same test requirement as in clause 7.6.2.5 in TS 38.521-1 [8].

#### 7.6B.2.3	Inband blocking for inter-band EN-DC within FR1 (1 NR CC)

7.6B.2.3.1	Test Purpose

Same test purpose as in clause 7.6.2.1 in TS 38.521-1 [8] for the NR carrier.

7.6B.2.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR DL CC.

7.6B.2.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.2.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.2.3.4	Test Description

Same test description as in clause 7.6.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.6.2.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6.2.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.6B.2.3.5	Test Requirement

Same test requirement as in clause 7.6.2.5 in TS 38.521-1 [8].

#### 7.6B.2.3_1	Inband blocking for EN-DC within FR1 (>2 CCs)

##### 7.6B.2.3_1.1	Inband blocking for EN-DC within FR1 (3 CCs)

7.6B.2.3_1.1.1	Test Purpose

For intra-band contiguous EN-DC within FR1 with 3 DL CCs: In-band blocking is defined for an unwanted interfering signal falling into the range from 15MHz below to 15MHz above the UE receive band, at which the relative throughput shall meet or exceed the requirement for the specified measurement channels. The lack of in-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

For inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs): Same test purpose as in clause 7.6A.2.1.1 in TS 38.521-1 [8].

7.6B.2.3_1.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC within FR1 with 3 DL CCs or inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs).

7.6B.2.3_1.1.3	Minimum Conformance Requirements

For intra-band contiguous EN-DC within FR1 with 3 DL CCs: The minimum conformance requirements are defined in clause 7.6 B.2.0.1. Exception requirements for both NR and E-UTRA are defined and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

For inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs): The minimum conformance requirements are defined in clause 7.6B.2.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.2.3_1.1.4	Test Description

7.6B.2.3_1.1.4.1 Intra-band contiguous EN-DC within FR1 with 3 DL CCs

7.6B.2.3_1.1.4.1.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.2.3_1.1.4.1.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.2.3_1.1.4.1.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  | Mid range |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Lowest |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| PCC – E-UTRA |  | SCC – EUTRA |  | NR |  |
| UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation |
| QPSK/QPSK | REFSENS/Full RB | NA/QPSK | NA/Full RB | DFT-s-OFDM QPSK/CP-OFDM QPSK | REFSENS/Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2.3-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.1 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2, A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.2.3_1.1.4.1.3.

7.6B.2.3_1.1.4.1.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.2.3_1.1.4.1.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.2.3_1.1.4.1.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.2.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.2.1.5-1 +( [10log(S_LCRB/NRB_alloc)] for NR CC, [10log(P_LCRB/NRB_alloc)] for E-UTRA CC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Set the parameters of the signal generator for an interfering signal below the aggregated component carriers in Case 1 according to Table 7.6B.2.1.5-1.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers in Case 1 at step 4.

7.	Repeat steps from 4 to 6, using interfering signals in Case 2 at steps 4 and 6.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.2.3_1.1.4.1.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.6B.2.3_1.1.4.2	Inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs)

Same test description as in clause 7.6A.2.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.6A.2.1.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6A.2.1.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6A.2.1.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.6B.2.3_1.1.5	Test Requirement

For intra-band contiguous EN-DC within FR1 with 3 DL CCs: Same test requirement as in clause 7.6B.2.1.5.

For inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs): Same test requirement as in clause 7.6A.2.1.5 in TS 38.521-1 [8].

##### 7.6B.2.3_1.2	Inband blocking for EN-DC within FR1 (4 CCs)

7.6B.2.3_1.2.1	Test Purpose

For intra-band contiguous EN-DC within FR1 with 4 DL CCs: In-band blocking is defined for an unwanted interfering signal falling into the range from 15MHz below to 15MHz above the UE receive band, at which the relative throughput shall meet or exceed the requirement for the specified measurement channels. The lack of in-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

For inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs): Same test purpose as in clause 7.6A.2.2.1 in TS 38.521-1 [8].

7.6B.2.3_1.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC within FR1 with 4 DL CCs or inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs).

7.6B.2.3_1.2.3	Minimum Conformance Requirements

For intra-band contiguous EN-DC within FR1 with 4 DL CCs: The minimum conformance requirements are defined in clause 7.6 B.2.0.1. Exception requirements for both NR and E-UTRA are defined and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

For inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs): The minimum conformance requirements are defined in clause 7.6B.2.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.2.3_1.2.4	Test Description

7.6B.2.3_1.2.4.1	Intra-band contiguous EN-DC within FR1 with 4 DL CCs

7.6B.2.3_1.2.4.1.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.2.3_1.2.4.1.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.2.3_1.2.4.1.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  | Mid range |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Lowest |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| PCC – E-UTRA |  | SCC1 – EUTRA and SCC2 – EUTRA |  | NR |  |
| UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation |
| QPSK/QPSK | REFSENS/Full RB | NA/QPSK | NA/Full RB | DFT-s-OFDM QPSK/CP-OFDM QPSK | REFSENS/Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2.3-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.1 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2, A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.2.3_1.2.4.1.3.

7.6B.2.3_1.2.4.1.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.2.3_1.2.4.1.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.2.3_1.2.4.1.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.2.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.2.1.5-1 +( [10log(S_LCRB/NRB_alloc)] for NR CC, [10log(P_LCRB/NRB_alloc)] for E-UTRA CC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Set the parameters of the signal generator for an interfering signal below the aggregated component carriers in Case 1 according to Table 7.6B.2.1.5-1.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers in Case 1 at step 4.

7.	Repeat steps from 4 to 6, using interfering signals in Case 2 at steps 4 and 6.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.2.3_1.2.4.1.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.6B.2.3_1.2.4.2	Iinter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs)

Same test description as in clause 7.6A.2.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.6A.2.2.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6A.2.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6A.2.2.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.6B.2.3_1.2.5	Test Requirement

For intra-band contiguous EN-DC within FR1 with 4 DL CCs: Same test requirement as in clause 7.6B.2.1.5.

For inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs): Same test requirement as in clause 7.6A.2.2.5 in TS 38.521-1 [8].

##### 7.6B.2.3_1.3	Inband blocking for EN-DC within FR1 (5 CCs)

7.6B.2.3_1.3.1	Test Purpose

Same test purpose as in clause 7.6B.2.3.1.

7.6B.2.3_1.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 16 and forward, supporting inter-band EN-DC within FR1 with 5 DL CCs (4 NR DL CCs).

7.6B.2.3_1.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.2.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.2.3_1.3.4	Test Description

Same test description as in clause 7.6A.2.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.6A.2.3.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6A.2.3.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6A.2.3.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.6B.2.3_1.3.5	Test Requirement

Same test requirement as in clause 7.6A.2.3.5 in TS 38.521-1 [8].

##### 7.6B.2.3_1.4	Void

#### 7.6B.2.3a	In-band blocking for inter-band NE-DC within FR1 (2 CCs)

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 7.6.2 and 7.6A.2 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 7.6.1 and 7.6.1A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

#### 7.6B.2.4	Inband blocking for inter-band EN-DC including FR2 (1 NR CC)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement uncertainty is FFS for power classes other than 1, 3 and 5.

7.6B.2.4.1	Test Purpose

Same test purpose as in clause 7.6.2.1 in TS 38.521-2 [9] for the NR carrier.

7.6B.2.4.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR DL CC.

7.6B.2.4.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.2.0.4.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.2.4.4	Test Description

Same test description as in clause 7.6.2.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 7.6.2.4.1 in TS 38.521-2 [9], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6.2.4.1 in TS 38.521-2 [9] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.7.

7.6B.2.4.5	Test Requirement

Same test requirement as in clause 7.6.2.5 in TS 38.521-2 [9].

#### 7.6B.2.4_1	Inband blocking for inter-band EN-DC including FR2 (>1 NR CC)

##### 7.6B.2.4_1.1	Inband blocking for inter-band EN-DC including FR2 (2 NR CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 7.6A.2.1 in TS 38.521-2 is incomplete.

7.6B.2.4_1.1.1	Test Purpose

Same test purpose as in clause 7.6B.2.4.1.

7.6B.2.4_1.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 and FR2 inter-band 2DL CA.

7.6B.2.4_1.1.3	Minimum Conformance Requirements

Same minimum conformance requirements as in clause 7.4B.2.4.3

7.6B.2.4_1.1.4	Test Description

Same test description as in clause 7.6A.2.1.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.7-1.

For Initial conditions as in clause 7.6A.2.1.4.1 in TS 38.521-2 [9], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6A.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6A.2.1.4.1 in TS 38.521-2 [9] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.7.

7.6B.2.4_1.1.5	Test Requirement

Same test requirement as in clause 7.6A.2.1.5 in TS 38.521-2 [9].

##### 7.6B.2.4_1.2	Void

##### 7.6B.2.4_1.3	Void

##### 7.6B.2.4_1.4	Void

#### 7.6B.2.4D	Inband blocking for inter-band EN-DC including FR2 for UL MIMO

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach can be applied and only NR carriers need to be tested.

No test case details are specified. Given UE’s Rx performance would not be impacted by the Tx configuration in NR FR2 TDD bands, the requirements in this test case can be well covered in clauses 7.6B.2.4 and 7.6B.2.4_1, and don’t need to be tested again.

#### 7.6B.2.5	Inband blocking for inter-band EN-DC including both FR1 and FR2

7.6B.2.5.1	Test purpose

Same test purpose as in clause 7.6.2.1 in TS 38.521-1 [8] for NR FR1 carrier and clause 7.6.2.1 in TS 38.521-2 [9] for NR FR2 carrier.

7.6B.2.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for Inband blocking apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 7.6B.2.

#### 7.6B.2.5D	Inband blocking for inter-band EN-DC including FR1 and FR2 for UL MIMO

7.6B.2.5D.1	Test purpose

Same test purpose as in clause 7.6D.2 in TS 38.521-1 [8] for NR FR1 carrier and clause 7.6D in TS 38.521-2 [9] for NR FR2 carrier.

7.6B.2.5D.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for Inband blocking apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 7.6B.2.

### 7.6B.3	Out-of-band blocking for DC

#### 7.6B.3.0	Minimum Conformance Requirements

##### 7.6B.3.0.1	Intra-band contiguous EN-DC

Intra-band contiguous EN-DC out-of-band requirement and parameters are defined in Table 7.6B.3.0.1-1.

Table 7.6B.3.0.1-1: Out-of-band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [5]. For inter-band combinations where the intra-band requirements are applicable, in which the E-UTRA band is a subset of an NR-only band, the NR band interferer values and offsets specified from Table 7.6A.3-2 in TS 38.101-1 [2] apply to both E-UTRA and NR carriers. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

For Table 7.6.2.1A-2 from TS 36.101 [4] in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7B.1 Spurious response are applicable.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.3.1.

##### 7.6B.3.0.2	Intra-band non-contiguous EN-DC

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.6.2.1 for single carrier operation and in clause 7.6.2.1A for CA in TS 36.101 [5].

For the NR sub-block, the requirement is defined in clause 7.6.3 in TS 38.101-1 [2].

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.3.2.

##### 7.6B.3.0.3	Inter-band EN-DC within FR1

Out-of-band blocking requirements for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5B.4.1 with following conditions:

-	one E-UTRA uplink carrier with the output power set to 4dB below PCMAX_L,c and the NR band whose downlink is being tested has its uplink carrier output power set to 29 dB below PCMAX_L,f,c.

-	one NR uplink carrier with the output power set to 4dB below PCMAX_L,f,c on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to 29 dB below PCMAX_L,c.

If CW interferer falls in a gap between FDL_high of the E-UTRA or NR band and FDL_low of the NR or EUTRA band, where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

If FDL_high of the lower E-UTRA or NR band is greater than or equal to the FDL_low of the upper NR or E-UTRA band as in overlapping RX frequency ranges, then the OOB range shall start from the FDL_low of the lower E-UTRA or NR band, and from the FDL_high of the upper NR or E-UTRA band.

For EN-DC combination listed in Table 7.6B.3.0.3-1 under the first test condition above, exceptions to the requirement specified in Table 7.6B.3.0.3-2 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 7.6B.3.0.3-1: EN-DC combination with exceptions allowed

EN-DC combination

DC_5_n78

DC_8_n77

DC_8_n78

DC_11_n77

DC_18_n77

DC_18_n78

DC_18_n79

DC_19_n77

DC_19_n78

DC_19_n79

DC_20_n78

DC_21_n77

DC_26_n77

DC_26_n78

DC_26_n79

DC_28_n77

DC_28_n78

DC_28_n79

Table 7.6B.3.0.3-2: Exceptions allowed

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when fInterferer± fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

For each of the two test cases in clauses 7.6.2.1 and 7.6.2.1A of [5] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] for all interferer frequency ranges a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.3.3.

##### 7.6B.3.0.3a	Inter-band NE-DC within FR1

Out-of-band blocking requirements for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level NE-DC fallbacks (two bands) in clause 5.5B.4a.1 with following conditions:

-	one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L,c and the NR band whose downlink is being tested has its uplink carrier output power set to 29 dB below PCMAX_L,f,c.

-	one NR uplink carrier with the output power set to 4 dB below PCMAX_L,f,c on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to 29 dB below PCMAX_L,c.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.3.3a.

##### 7.6B.3.0.4	Inter-band EN-DC including FR2

Out-of-band blocking requirements specified for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [5] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5B.5.1 with only E-UTRA UL with output power as in TS 36.101 [5] (4dB below PCMAX_L).

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.3.4.

##### 7.6B.3.0.5	Inter-band EN-DC including both FR1 and FR2

Out-of-band blocking requirements specified for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (three bands) in clause 5.5B.6.2 with only E-UTRA UL with output power as in TS 36.101 [5] (4dB below PCMAX_L).

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.3.5.

#### 7.6B.3.1	Out-of-band blocking for intra-band contiguous EN-DC (2 CCs)

7.6B.3.1.1	Test Purpose

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in clause 7.6B.2.1 and clause 7.5B.1 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.3.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.6B.3.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.3.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.6B.3.1.4	Test Description

7.6B.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.3.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.3.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  | Highest NRB_agg (NOTE 3) |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  | Lowest |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_NR | QPSK | REFSEN_LTE S |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS _NR and REFSENS_LTE refers to Uplink configuration in Table 7.3.2.4.1-3 in 38.521-1 [8] and Table 7.3.4.1-1 in 36.521 [10] for NR and E-UTRA CC respectively. NOTE 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.2 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2, A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.3.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.6B.3.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.3.1.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.3.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.3.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.3.1.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6B.3.1.5-1. The frequency step size is 1MHz.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2 in TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

6.	Record the frequencies for which the throughput doesn't meet the requirements.

7.	Repeat steps from 4 to 6, using an interfering signal above the aggregated component carriers at step 4.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.6B.3.1.5	Test Requirement

Except for the spurious response frequencies recorded in step 6 of test procedure, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.6B.3.1.5-1 for the specified wanted signal mean power in the presence of interfering signals.

The number of spurious response frequencies recorded in step 6 of test procedure shall not exceed exceptions in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of clause 7.7B Spurious Response are applicable.

Table 7.6B.3.1.5-1: Out-of-band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [5]. For inter-band combinations where the intra-band requirements are applicable, in which the E-UTRA band is a subset of an NR-only band, the NR band interferer values and offsets specified from Table 7.6A.3-2 in TS 38.101-1 [2] apply to both E-UTRA and NR carriers. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 8.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

Table 7.6B.3.1.5-2: Void

Table 7.6B.3.1.5-3: Void

#### 7.6B.3.2	Out-of-band blocking for intra-band non-contiguous EN-DC (2 CCs)

7.6B.3.2.1	Test Purpose

Same test purpose as in clause 7.6.3.1 in TS 38.521-1 [8] for the NR carrier.

7.6B.3.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC in FR1 with 2 DL CCs.

7.6B.3.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.3.0.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.3.2.4	Test Description

Same test description as in clause 7.6.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 7.6B.3.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | High range with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1 except for the parameters specified in Table 7.6B.3.2.4-1.

For Initial conditions as in clause 7.6.3.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6.3.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add steps 7 and 8 to Initial conditions in clause 7.6.3.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Step 4 of Test procedure as in clause 7.6.3.4.2 in TS 38.521-1 [8] is replaced by:

4.	Set the downlink signal level for NR CC according to the Table 7.6.3.5-1 or 7.6.3.5-3 in TS 38.521-1 [8] as appropriate. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of (PCMAX_L,c - 29dB) for E-UTRA CC, and of 4dB below PCMAX_L,f,c for NR CC for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

7.6B.3.2.5	Test Requirement

Same test requirement as in clause 7.6.3.5 in TS 38.521-1 [8].

#### 7.6B.3.3	Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs)

7.6B.3.3.1	Test Purpose

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz or 3*CBW below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz or 3*CBW below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in clause 7.6B.2.3 and clause 7.5B.3 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.3.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.6B.3.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.3.0.3.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.6B.3.3.4	Test Description

7.6B.3.3.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, DC configuration specified in clause 5.5B.4 and test channel bandwidths specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1, and sub-carrier spacing based on NR operating bands specified in TS 38.521-1 [8] clause 5.3. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.6B.3.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) and OCNG patterns are specified in TS 36.521-1 [10] Annexe A for E-UTRA, and TS 38.521-1 [8] Annex A for NR. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C and in TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively.

Table 7.6B.3.3.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause4.3.1 |  |  |  | Mid range for E-UTRA and Mid range for NR (NOTE 3) |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Highest for E-UTRA and Highest for NR |  |  |  |
| NR Test SCS as specified in TS 38.508-1 [6] Table 5.3.5-1 |  |  |  | Lowest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| E-UTRA Cell |  | NR Cell |  | E-UTRA Cell |  | NR Cell |  |
| Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation |
| QPSK | NOTE 1 | CP-OFDM QPSK | NOTE 1 | QPSK | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3B.2.3.4.2.1-1. NOTE 2: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequency. NOTE 4: For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.2 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2 , A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.3.3.4.3.

7.6B.3.3.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.3.3.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.3.3.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.3.3.5-1, Table 7.6B.3.3.5-3, or Table 7.6B.3.3.5-5 for E-UTRA CC and NR CC respectively. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of (PCMAX_L,c - 4dB) for E-UTRA CC, and of 29 dB below PCMAX_L,f,c for NR CC for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6B.3.3.5-4 or Table 7.6B.3.3.5-6. The frequency step size is  MHz.

If CW interferer falls in a gap between FDL_high of the E-UTRA or NR band and FDL_low of the NR or EUTRA band, where the corresponding OOB ranges 1 and 2 in Table 7.6B.3.3.5-2 and Table 7.6B.3.3.5-4 or Table 7.6B.3.3.5-6 overlap, then the lower level interferer limit of the overlapping OOB ranges applies. CW interferer is eliminated from FDL_low -15MHz to FDL_high + 15MHz of E-UTRA carrier.

If FDL_high of the lower E-UTRA or NR band is greater than or equal to the FDL_low of the upper NR or E-UTRA band as in overlapping RX frequency ranges, then the OOB range shall start from the FDL_low of the lower E-UTRA or NR band, and from the FDL_high of the upper NR or E-UTRA band.

For EN-DC combination listed in Table 7.6B.3.3.5-7, exceptions to the requirement specified in Table 7.6B.3.3.5-8 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

5.	Measure the average throughput of NR CC for a duration sufficient to achieve statistical significance according to Annex H.2. Record the frequencies for which the throughput doesn't meet the requirements.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers at step 4.

7.	Set the Downlink signal level to the value as defined in Table 7.6B.3.3.5-1, Table 7.6B.3.3.5-3, or Table 7.6B.3.3.5-5 for E-UTRA CC and NR CC respectively. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of (PCMAX_L,f,c - 4dB) for NR CC, and of 29 dB below PCMAX_L,c for E-UTRA CC for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

8.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6B.3.3.5-2, Table 7.6B.3.3.5-4 or Table 7.6B.3.3.5-6 for E-UTRA CC and NR CC testing respectively. The frequency step size is  MHz.

If CW interferer falls in a gap between FDL_high of the E-UTRA or NR band and FDL_low of the NR or EUTRA band, where the corresponding OOB ranges 1 and 2 in Table 7.6B.3.3.5-2 and Table 7.6B.3.3.5-4 or Table 7.6B.3.3.5-6 overlap, then the lower level interferer limit of the overlapping OOB ranges applies. CW interferer is eliminated from FDL_low -15MHz to FDL_high + 15MHz of E-UTRA and NR carriers.

If FDL_high of the lower E-UTRA or NR band is greater than or equal to the FDL_low of the upper NR or E-UTRA band as in overlapping RX frequency ranges, then the OOB range shall start from the FDL_low of the lower E-UTRA or NR band, and from the FDL_high of the upper NR or E-UTRA band.

9.	Measure the average throughput of E-UTRA CC and NR CC respectively for a duration sufficient to achieve statistical significance according to Annex H.2. Record the frequencies for which the throughput doesn't meet the requirements.

10.	Repeat steps from 8 to 9, using an interfering signal above the aggregated component carriers at step 8.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 with the following exceptions.

Table 7.6B.3.3.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.6B.3.3.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.6B.3.3.5	Test Requirement

For E-UTRA bands, except for the spurious response frequencies recorded in test procedure, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 of TS 36.521-1 [10] with parameters specified in Tables 7.6B.3.3.5-1 and 7.6B.3.3.5-2.

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, except for the spurious response frequencies recorded in test procedure, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 of TS 38.521-1 [8] with parameters specified in Tables 7.6B.3.3.5-3 and 7.6B.3.3.5-4.

For NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, except for the spurious response frequencies recorded in test procedure, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 of TS 38.521-1 [8] with parameters specified in Tables 7.6B.3.3.5-5 and 7.6B.3.3.5-6.

If CW interferer falls in a gap between FDL_high of the E-UTRA or NR band and FDL_low of the NR or EUTRA band, where the corresponding OOB ranges 1 and 2 in Table 7.6B.3.3.5-2 and Table 7.6B.3.3.5-4 or Table 7.6B.3.3.5-6 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

If FDL_high of the lower E-UTRA or NR band is greater than or equal to the FDL_low of the upper NR or E-UTRA band as in overlapping RX frequency ranges, then the OOB range shall start from the FDL_low of the lower E-UTRA or NR band, and from the FDL_high of the upper NR or E-UTRA band.

For EN-DC combination listed in Table 7.6B.3.3.5-7 under the first test condition above, exceptions to the requirement specified in Table 7.6B.3.3.5-8 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

For all interferer frequency ranges a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7B.3 apply.

Table 7.6B.3.3.5-1: Out-of-band blocking parameters for E-UTRA bands

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| Note 1: The reference measurement channel is specified in Annex A.3.2 of TS 36.521-1 [10] with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 of TS 36.521-1 [10]. Note 2: The REFSENS power level is specified in Table 7.3.3-1 of TS 36.521-1 [10] for two and four antenna ports, respectively. |  |  |  |  |  |  |  |

Table 7.6B.3.3.5-2: Out of band blocking for E-UTRA bands

| E-UTRA band | Parameter | Units | Frequency |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | range 1 | range 2 | range 3 | range 4 |
|  | PInterferer | dBm | -44 | -30 | -15 | -15 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 30, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42 (Note 3), 43 (Note 3), 44, 45, 53, 65, 66, 68, 70, 71, 72, 73, 74 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz | - |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz | - |
| 2, 5, 12, 17 | FInterferer | MHz | - | - | - | FUL_low - FUL_high |
| Note 1: Range 3 shall be tested only with the highest channel bandwidth. Note 2: For the UE which supports both Band 11 and Band 21 the out of blocking is FFS. Note 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. Note 4: For the UE that supports both Band 4 and Band 66, the out-of-blocking frequency range for Band 4 is defined relative to FDL_low and FDL_high of Band 66. |  |  |  |  |  |  |

Table 7.6B.3.3.5-3: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| NOTE: The transmitter shall be at the minimum UL configuration specified in Table 7.3.2.3-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |

Table 7.6B.3.3.5-4: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, n5, n7, n8, n12, n20, n25, n28, n34, n38, n39, n40, n41, n50, n51, n65, n66, n70, n71, n74, n75, n76 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. |  |  |  |  |  |

Table 7.6B.3.3.5-5: Out-of-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 |  |
| NOTE: The transmitter shall be at the minimum UL configuration specified in Table 7.3.2.3-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |

Table 7.6B.3.3.5-6: Out of-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low ≤ -3CBW or 3CBW ≤ f – FDL_high < 60 | -200 < f – FDL_low ≤ -MAX(60,3CBW) or MAX(60,3CBW) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3CBW) or FDL_high + MAX(200,3CBW) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3CBW) or MAX(60,3CBW) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3CBW) or FDL_high + MAX(150,3CBW) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

Table 7.6B.3.3.5-7: EN-DC combination with exceptions allowed

EN-DC combination

DC_5_n78

DC_8_n77

DC_8_n78

DC_11_n77

DC_18_n77

DC_18_n78

DC_18_n79

DC_19_n77

DC_19_n78

DC_19_n79

DC_20_n78

DC_21_n77

DC_26_n77

DC_26_n78

DC_26_n79

DC_28_n77

DC_28_n78

DC_28_n79

Table 7.6B.3.3.5-8: Exceptions allowed

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when , where and are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. and are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

#### 7.6B.3.3_1	Out-of-band blocking for EN-DC within FR1 (>2 CCs)

##### 7.6B.3.3_1.1	Out-of-band blocking for EN-DC within FR1 (3 CCs)

7.6B.3.3_1.1.1	Test Purpose

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in clause 7.6B.2.1 and clause 7.5B.1 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.3.3_1.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC within FR1 with 3 DL CCs.

7.6B.3.3_1.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.3.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.6B.3.3_1.1.4	Test Description

7.6B.3.3_1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.3.3_1.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.3.3_1.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  | Mid range |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  | Highest NRB_agg (NOTE 3) |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Lowest |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| PCC – E-UTRA |  | SCC – EUTRA |  | NR |  |
| UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation |
| QPSK/QPSK | REFSENS/Full RB | NA/QPSK | NA/Full RB | DFT-s-OFDM QPSK/CP-OFDM QPSK | REFSENS/Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2.3-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.2 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2 , A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.3.3_1.1.4.3.

7.6B.3.3_1.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.3.3_1.1.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.3.3_1.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.3.3_1.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.3.3_1.1.5-1 +( [10log(S_LCRB/NRB_alloc)] for NR CC, [10log(P_LCRB/NRB_alloc)] for E-UTRA CC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6B.3.3_1.1.5-1. The frequency step size is 1MHz.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers at step 4.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.3.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.6B.3.3_1.1.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.6B.3.3_1.1.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.6B.3.3_1.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.6B.3.3_1.1.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.6B.3.3_1.1.5-1: Out-of-band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [5]. For inter-band combinations where the intra-band requirements are applicable, in which the E-UTRA band is a subset of an NR-only band, the NR band interferer values and offsets specified from Table 7.6A.3-2 in TS 38.101-1 [2] apply to both E-UTRA and NR carriers. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 8.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

##### 7.6B.3.3_1.2	Out-of-band blocking for EN-DC within FR1 (4 CCs)

7.6B.3.3_1.2.1	Test Purpose

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in clause 7.6B.2.1 and clause 7.5B.1 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.3.3_1.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC within FR1 with 4 DL CCs.

7.6B.3.3_1.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.3.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.6B.3.3_1.2.4	Test Description

7.6B.3.3_1.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.3.3_1.2.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.3.3_1.2.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  | Mid range |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  | Highest NRB_agg (NOTE 3) |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Lowest |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| PCC – E-UTRA |  | SCC1 – EUTRA and SCC2 – EUTRA |  | NR |  |
| UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation |
| QPSK/QPSK | REFSENS/Full RB | NA/QPSK | NA/Full RB | DFT-s-OFDM QPSK/CP-OFDM QPSK | REFSENS/Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2.3-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.2 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2 , A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.3.3_1.2.4.3.

7.6B.3.3_1.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.3.3_1.2.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.3.3_1.2.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.3.3_1.2.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.3.3_1.2.5-1 +( [10log(S_LCRB/NRB_alloc)] for NR CC, [10log(P_LCRB/NRB_alloc)] for E-UTRA CC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6B.3.3_1.2.5-1. The frequency step size is 1MHz.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers at step 4.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.3.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.6B.3.3_1.2.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.6B.3.3_1.2.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.6B.3.3_1.2.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.6B.3.3_1.2.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.6B.3.3_1.2.5-1: Out-of-band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [5]. For inter-band combinations where the intra-band requirements are applicable, in which the E-UTRA band is a subset of an NR-only band, the NR band interferer values and offsets specified from Table 7.6A.3-2 in TS 38.101-1 [2] apply to both E-UTRA and NR carriers. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 8.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

### 7.6B.4	Narrow band blocking for DC

#### 7.6B.4.0	Minimum Conformance Requirements

##### 7.6B.4.0.1	Intra-band contiguous EN-DC

Intra-band contiguous EN-DC narrow band blocking requirement and parameters are defined in Table 7.6B.4.0.1-1.

Table 7.6B.4.0.1-1: Narrow band blocking parameters for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 16 |  |  |  |
| PUW, dBm (CW) | -55 |  |  |  |
| NOTE 1: Jammer offset is from Table 7.6.3.1A-1 in TS 36.101 [5] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2]with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 4: If NR carrier BW > 40 MHz, no narrow band blocking requirements apply when blocker is applied at the edge of the NR carrier. |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.4.1.

##### 7.6B.4.0.2	Intra-band non-contiguous EN-DC

For the E-TRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.6.3.1 for single carrier operation and in clause 7.6.3.1A for CA in TS 36.101 [5].

For the NR sub-block, the requirement is defined in clause 7.6.4 in TS 38.101-1 [2].

The blocker configuration is defined in the general clause 7.1.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.4.2.

##### 7.6B.4.0.3	Inter-band EN-DC within FR1

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.4 and 7.6A.4 of TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.4.3.

##### 7.6B.4.0.3a	Inter-band NE-DC within FR1

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.4 and 7.6A.4 of TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.4.3a.

##### 7.6B.4.0.4	Inter-band EN-DC including FR2

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [5] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.4.4.

##### 7.6B.4.0.5	Inter-band EN-DC including both FR1 and FR2

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.6.4 and 7.6A.4 of TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6B.4.5.

#### 7.6B.4.1	Narrow band blocking for intra-band contiguous EN-DC (2 CCs)

7.6B.4.1.1	Test Purpose

Verifies a receiver's ability to receive EN-DC signals at its assigned channel frequencies in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The lack of narrow-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.4.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.6B.4.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.4.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.6B.4.1.4	Test Description

7.6B.4.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.4.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.4.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  | Lowest |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS | QPSK | REFSENS |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2.3-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.2 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2 , A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.4.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.6B.4.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.4.1.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.4.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.4.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.4.1.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) = 1.7dB, where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) = 2.0dB, where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size.

4.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6B.4.1.5-1.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers at step 4.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.6B.4.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.6B.4.1.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.6B.4.1.5-1: Narrow band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 16 |  |  |  |
| PUW, dBm (CW) | -55 |  |  |  |
| NOTE 1: Jammer offset is from Table 7.6.3.1A-1 in TS 36.101 [5] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 4: If NR carrier BW > 40 MHz, no narrow band blocking requirements apply when blocker is applied at the edge of the NR carrier. |  |  |  |  |

Table 7.6B.4.1.5-2: Void

Table 7.6B.4.1.5-3: Void

#### 7.6B.4.2	Narrow band blocking for intra-band non-contiguous EN-DC (2 CCs)

7.6B.4.2.1	Test Purpose

Same test purpose as in clause 7.6.4.1 in TS 38.521-1 [8] for the NR carrier.

7.6B.4.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC in FR1 with 2 DL CCs.

7.6B.4.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.4.0.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.4.2.4	Test Description

Same test description as in clause 7.6.4.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 7.6B.4.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | High range with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1 except for the parameters specified in Table 7.6B.4.2.4-1.

For Initial conditions as in clause 7.6.4.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6.4.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add steps 7 and 8 to Initial conditions in clause 7.6.4.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

8.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Step 3 of Test procedure as in clause 7.6.4.4.2 in TS 38.521-1 [8] shall treat the in-gap tests as below: For intra-band non-contiguous EN-DC of two sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing requirements apply for in-gap tests only if the corresponding interferer frequency offsets satisfy the following condition in relation to the sub-block gap size Wgap, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset)| – BWChannel

Step 4 of Test procedure as in clause 7.6.4.4.2 in TS 38.521-1 [8] is replaced by:

4.	Set the downlink signal level for NR CC according to the Table 7.6.4.5-1 in TS 38.521-1 [8] as appropriate. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of (PCMAX_L,c - 29dB) for E-UTRA CC, and of 4dB below PCMAX_L,f,c for NR CC for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

7.6B.4.2.5	Test Requirement

Same test requirement as in clause 7.6.4.5 in TS 38.521-1 [8].

#### 7.6B.4.3	Narrow band blocking for inter-band EN-DC within FR1 (1 NR CC)

7.6B.4.3.1	Test Purpose

Same test purpose as in clause 7.6.4.1 in TS 38.521-1 [8] for the NR carrier.

7.6B.4.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR DL CC.

7.6B.4.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.4.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.4.3.4	Test Description

Same test description as in clause 7.6.4.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.6.4.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6.4.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6.4.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.6B.4.3.5	Test Requirement

Same test requirement as in clause 7.6.4.5 in TS 38.521-1 [8].

#### 7.6B.4.3_1	Narrow band blocking for EN-DC within FR1 (>2 CCs)

##### 7.6B.4.3_1.1	Narrow band blocking for EN-DC within FR1 (3 CCs)

7.6B.4.3_1.1.1	Test Purpose

Verifies a receiver's ability to receive EN-DC signals at its assigned channel frequencies in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The lack of narrow-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.4.3_1.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC within FR1 with 3 DL CCs or inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs).

7.6B.4.3_1.1.3	Minimum Conformance Requirements

For intra-band contiguous EN-DC within FR1 with 3 DL CCs: The minimum conformance requirements are defined in clause 7.6B.4.0.1. Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

For inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs): The minimum conformance requirements are defined in clause 7.6B.4.0.3. No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.4.3_1.1.4	Test Description

7.6B.4.3_1.1.4.1 Intra-band contiguous EN-DC within FR1 with 3 DL CCs

7.6B.4.3_1.1.4.1.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.4.3_1.1.4.1.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.4.3_1.1.4.1.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  | Mid range |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Lowest |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| PCC – E-UTRA |  | SCC – EUTRA |  | NR |  |
| UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation |
| QPSK/QPSK | REFSENS/Full RB | NA/QPSK | NA/Full RB | DFT-s-OFDM QPSK/CP-OFDM QPSK | REFSENS/Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2.3-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.2 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2 , A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.4.3_1.1.4.1.3.

7.6B.4.3_1.1.4.1.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.4.3_1.1.4.1.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.4.3_1.1.4.1.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.4.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.4.1.5-1 +( [10log(S_LCRB/NRB_alloc)] for NR CC, [10log(P_LCRB/NRB_alloc)] for E-UTRA CC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6B.4.1.5-1.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers at step 4.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.4.3_1.1.4.1.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.6B.4.3_1.1.4.2 Inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs)

Same test description as in clause 7.6.4.4 or 7.6A.4.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.6A.4.1.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6A.4.1.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6A.4.1.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.6B.4.3_1.1.5	Test Requirement

For intra-band contiguous EN-DC within FR1 with 3 DL CCs: Same test requirement as in clause 7.6B.4.1.5.

For inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs): Same test requirement as in clause 7.6A.4.1.5 in TS 38.521-1 [8].

##### 7.6B.4.3_1.2	Narrow band blocking for EN-DC within FR1 (4 CCs)

7.6B.4.3_1.2.1	Test Purpose

Verifies a receiver's ability to receive EN-DC signals at its assigned channel frequencies in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The lack of narrow-band blocking ability will decrease the coverage area when other NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.4.3_1.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC within FR1 with 4 DL CCs or inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs).

7.6B.4.3_1.2.3	Minimum Conformance Requirements

For intra-band contiguous EN-DC within FR1 with 4 DL CCs: The minimum conformance requirements are defined in clause 7.6B.4.0.1. Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

For inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs): The minimum conformance requirements are defined in clause 7.6B.4.0.3. No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.4.3_1.2.4	Test Description

7.6B.4.3_1.2.4.1 Intra-band contiguous EN-DC within FR1 with 4 DL CCs

7.6B.4.3_1.2.4.1.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in Table 7.6B.4.3_1.2.4.1.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.6B.4.3_1.2.4.1.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  | Mid range |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Lowest |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |
| PCC – E-UTRA |  | SCC1 – EUTRA and SCC2 – EUTRA |  | NR |  |
| UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation | UL/DL Modulation | UL/DL allocation |
| QPSK/QPSK | REFSENS/Full RB | NA/QPSK | NA/Full RB | DFT-s-OFDM QPSK/CP-OFDM QPSK | REFSENS/Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each EN-DC band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2.3-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 Annex A, in Figure A.3.1.4.2 for SS diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C and TS 38.521-1 [8] Annex C for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL and DL Reference Measurement channels are TS 36.521-1 [10] Annex A.2, A.3 and TS 38.521-1 [8] Annex A.2 , A.3 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.6B.4.3_1.2.4.1.3.

7.6B.4.3_1.2.4.1.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.4.3_1.2.4.1.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.4.3_1.2.4.1.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.6B.4.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6B.4.1.5-1 +( [10log(S_LCRB/NRB_alloc)] for NR CC, [10log(P_LCRB/NRB_alloc)] for E-UTRA CC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6B.4.1.5-1.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 4 to 5, using an interfering signal above the aggregated component carriers at step 4.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.6B.4.3_1.2.4.1.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.6B.4.3_1.2.4.2 Inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs)

Same test description as in clause 7.6A.4.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.6A.4.2.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6A.4.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6A.4.2.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.6B.4.3_1.2.5	Test Requirement

For intra-band contiguous EN-DC within FR1 with 4 DL CCs: Same test requirement as in clause 7.6B.4.1.5.

For inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs): Same test requirement as in clause 7.6A.4.2.5 in TS 38.521-1 [8].

##### 7.6B.4.3_1.3	Narrow band blocking for EN-DC within FR1 (5 CCs)

7.6B.4.3_1.3.1	Test Purpose

Same test purpose as in clause 7.6B.4.3.1.

7.6B.4.3_1.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 16 and forward, supporting inter-band EN-DC within FR1 with 5 DL CCs (4 NR DL CCs).

7.6B.4.3_1.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6B.4.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.6B.4.3_1.3.4	Test Description

Same test description as in clause 7.6A.4.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.6A.4.3.4.1 in TS 38.521-1 [8], add step 2.1 and step 3.1 as follows:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.6A.4.3.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

Add step 7 to Initial conditions in clause 7.6A.4.3.4.1 in TS 38.521-1 [8] as follows:

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.6B.4.3_1.3.5	Test Requirement

Same test requirement as in clause 7.6A.4.3.5 in TS 38.521-1 [8].

##### 7.6B.4.3_1.4	Void

#### 7.6B.4.3a	Narrow band blocking for inter-band NE-DC within FR1 (2 CCs)

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 7.6.4 and 7.6A.4 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 7.6.3 and 7.6.3A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

## 7.6E	Blocking characteristics for V2X in FR1

### 7.6E.0	Minimum conformance requirements

For intra-band V2X operation, the blocking characteristics specified in clause 7.6.1.1G in TS 36.101 [5] and specified in clause 7.6E in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For inter-band con-current NR V2X operation, the blocking characteristics requirements shall be applied per each component carrier. The in-band blocking and out of band blocking requirement specified in clause 7.6E in TS 38.101-1 [2] shall apply on NR V2X carrier and the requirement specified in clause 7.6 in TS 36.101 [5] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active. The requirements specified in subclause 7.6.1.1G and 7.6.2.1G of TS 36.1 01 [5] shall apply for the E-UTRA sidelink reception and the requirements specified in subclause 7.6 of TS 38.101-1 [2] shall apply for the NR downlink reception while all downlink carriers are active. PInterferer power is increased by ΔRIB,c in the requirement.

No narrow band blocking requirement applied for NR V2X carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.6E.

### 7.6E.1	In-band blocking for V2X operation

7.6E.1.1	Test purpose

In-band blocking is defined for an unwanted interfering signal falling into the frequency range (see clause 7.6.1 and 7.6.1G of TS 36.521-1 [10] and clause 7.6.2 and 7.6E.2 of TS 38.521-1 [8]), at which the relative throughput shall meet or exceed the requirement for the specified measurement channels.

The lack of in-band blocking ability will decrease the coverage area when other transmitters exist (except in the adjacent channels and spurious response).

7.6E.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 7.6.1 and 7.6.1G of TS 36.521-1 [10] and clause 7.6.2 and 7.6E.2 of TS 38.521-1 [8] and don’t need to be tested again.

### 7.6E.2	Out-of-band blocking for V2X operation

7.6E.2.1	Test purpose

Out-of-band blocking for V2X is defined for an unwanted CW interfering signal falling into the frequency range ( see clause 7.6.2 and 7.6.2G of TS 36.521-1 [10] and clause 7.6.3 and 7.6E.3 of TS 38.521-1 [8] ), at which a given average throughput shall meet or exceed the requirement for the specified measurement channels in aggregated signals.

The lack of out-of-band blocking ability will decrease the coverage area when other transmitters exist (except in the adjacent channels and spurious response).

7.6E.2.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 7.6.2 and 7.6.2G of TS 36.521-1 [10] and clause 7.6.3 and 7.6E.3 of TS 38.521-1 [8] and don’t need to be tested again.

## 7.7	Void

## 7.7A	Spurious response for CA

7.7A.1	Test purpose

Same test purpose as in clause 7.7 in TS 38.521-1 [8] for NR FR1 carrier(s).

7.7A.2	Test applicability

The requirements are only for FR1. Therefore, only the conducted requirements are tested.

No test case details are specified. The NR/5GC requirements for Spurious response apply and are tested in TS 38.521-1 [8] clause 7.7 and 7.7A.

## 7.7B	Spurious response for DC

### 7.7B.0	Minimum Conformance Requirements

#### 7.7B.0.1	Intra-band contiguous EN-DC

Intra-band contiguous EN-DC spurious response requirement and parameters are defined in Table 7.7B.0.1-1.

Table 7.7B.0.1-1: Spurious Response Parameters for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 7.7B.1.

#### 7.7B.0.2	Intra-band non-contiguous EN-DC

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.7.1 for single carrier operation and in clause 7.7.1A for CA in TS 36.101 [5].

For the NR sub-block, the requirement is defined in clause 7.7 in TS 38.101-1 [2].

The normative reference for this requirement is TS 38.101-3 [4] clause 7.7B.2.

#### 7.7B.0.3	Inter-band EN-DC within FR1

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5B.4.1 with following conditions:

-	one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L,c and the NR band whose downlink is being tested has its uplink carrier output power set to 29 dB below PCMAX_L,f,c.

-	one NR uplink carrier with the output power set to 4 dB below PCMAX_L,f,c on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to 29 dB below PCMAX_L,c.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.7B.3.

#### 7.7B.0.3a	Inter-band NE-DC within FR1

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level NE-DC fallbacks (two bands) in clause 5.5B.4a.1 with following conditions:

-	one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L,c and the NR band whose downlink is being tested has its uplink carrier output power set to 29 dB below PCMAX_L,f,c.

-	one NR uplink carrier with the output power set to 4 dB below PCMAX_L,f,c on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to 29 dB below PCMAX_L,c.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.7B.3a.

#### 7.7B.0.4	Inter-band EN-DC including FR2

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A  of TS 36.101 [5] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5B.5.1 with only E-UTRA UL with output power as in TS 36.101 [5] (4 dB below PCMAX_L).

The normative reference for this requirement is TS 38.101-3 [4] clause 7.7B.4.

#### 7.7B.0.5	Inter-band EN-DC including both FR1 and FR2

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (three bands) in clause 5.5B.6.2 with only E-UTRA UL with output power as in TS 36.101 [5] (4 dB below PCMAX_L).

The normative reference for this requirement is TS 38.101-3 [4] clause 7.7B.5.

### 7.7B.1	Spurious Response for intra-band contiguous EN-DC (2 CCs)

7.7B.1.1	Test Purpose

Spurious response for EN-DC verifies the receiver's ability to receive a wanted aggregated signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in clause 7.6B.3.1 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7B.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.7B.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7B.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.7B.1.4	Test Description

7.7B.1.4.1	Initial condition

The initial conditions shall be the same as in clause 7.6B.3.1.4.1 in order to test spurious responses obtained in clause 7.6B.3.1 under the same conditions.

7.7B.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.3.1.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.3.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.7B.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.7B.1.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7B.1.5-1. The spurious frequencies are taken from records in the final step of test procedures in clause 7.6B.3.1.4.2.

5.	For each spurious frequency, Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.7B.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.7B.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.7B.1.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.7B.1.5-1: Spurious Response for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

Table 7.7B.1.5-2: Void

Table 7.7B.1.5-3: Void

### 7.7B.2	Spurious Response for intra-band non-contiguous EN-DC (2 CCs)

7.7B.2.1	Test Purpose

Same test purpose as in clause 7.7.1 in TS 38.521-1 [8] for the NR carrier.

7.7B.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC in FR1 with 2 DL CCs.

7.7B.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7B.0.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.7B.2.4	Test Description

Same test description as in clause 7.7.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial conditions shall be the same as the initial conditions in clause 7.6B.3.2.4 in order to test spurious responses obtained in clause 7.6B.3.2 under the same conditions.

Step 4 of Test procedure as in clause 7.7.4.2 in TS 38.521-1 [8] is replaced by:

4.	Set the downlink signal level for NR CC according to the Table 7.7.5-1 or 7.7.5-1a in TS 38.521-1 [8] as appropriate. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of (PCMAX_L,c - 29dB) for E-UTRA CC, and of 4dB below PCMAX_L,f,c for NR CC for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

7.7B.2.5	Test Requirement

Same test requirement as in clause 7.7.5 in TS 38.521-1 [8].

### 7.7B.3	Spurious Response for inter-band EN-DC within FR1 (2 CCs)

7.7B.3.1	Test Purpose

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in subclause 7.6B.3.3 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7B.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.7B.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7B.0.3.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.7B.3.4	Test Description

7.7B.3.4.1	Initial Conditions

The initial conditions shall be the same as in clause 7.6B.3.3.4.1 in order to test spurious responses obtained in clause 7.6B.3.3 under the same conditions.

7.7B.3.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.3.3.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.3.3.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.7B.3.5-1, Table 7.7B.3.5-3, or Table 7.7B.3.5-4 for E-UTRA CC and NR CC respectively. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of (PCMAX_L,c - 4dB) for E-UTRA CC, and of 29 dB below PCMAX_L,f,c for NR CC for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7B.3.5-2. The spurious frequencies are taken from records in test procedures in clause 7.6B.3.3.4.2.

5.	For the spurious frequency, measure the average throughput of NR CC for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Set the Downlink signal level to the value as defined in Table 7.7B.3.5-1, Table 7.7B.3.5-3, or Table 7.7B.3.5-4 for E-UTRA CC and NR CC respectively. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of (PCMAX_L,f,c - 4dB) for NR CC, and of 29 dB below PCMAX_L,c for E-UTRA CC for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

7.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7B.3.5-2. The spurious frequencies are taken from records in test procedures in clause 7.6B.3.3.4.2 for E-UTRA CC and NR CC testing respectively..

8.	For the spurious frequency, measure the average throughput of E-UTRA CC and NR CC respectively for a duration sufficient to achieve statistical significance according to Annex H.2.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.7B.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 with the following exceptions.

Table 7.7B.3.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.7B.3.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.7B.3.5	Test Requirement

For E-UTRA bands, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 of TS 36.521-1 [10] with parameters specified in Tables 7.7B.3.5-1 and 7.7B.3.5-2.

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 of TS 38.521-1 [8] with parameters specified in Tables 7.7B.3.5-3 and 7.7B.3.5-2.

For NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 of TS 38.521-1 [8] with parameters specified in Tables 7.7B.3.5-4 and 7.7B.3.5-2.

Table 7.7B.3.5-1: Spurious response parameters for E-UTRA bands

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| Note 1: The reference measurement channel is specified in Annex A.3.2 of TS 36.521-1 [10] with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 of TS 36.521-1 [10]. Note 2: The REFSENS power level is specified in Table 7.3.3-1 of TS 36.521-1 [10] for two and four antenna ports, respectively. |  |  |  |  |  |  |  |

Table 7.7B.3.5-2: Spurious Response for E-UTRA bands and NR bands

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

Table 7.7B.3.5-3: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| NOTE: The transmitter shall be at the minimum UL configuration specified in Table 7.3.2.3-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |

Table 7.7B.3.5-4: Spurious response parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 |  |
| NOTE: The transmitter shall be at the minimum UL configuration specified in Table 7.3.2.3-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |

### 7.7B.3_1	Spurious Response for EN-DC within FR1 (>2 CCs)

#### 7.7B.3_1.1	Spurious Response for EN-DC within FR1 (3 CCs)

7.7B.3_1.1.1	Test Purpose

Spurious response for EN-DC verifies the receiver's ability to receive a wanted aggregated signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in clause 7.6B.3.3_1.1 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7B.3_1.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 3 DL CCs.

7.7B.3_1.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7B.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.7B.3_1.1.4	Test Description

7.7B.3_1.1.4.1	Initial condition

The initial conditions shall be the same as in clause 7.6B.3.3_1.1.4.1 in order to test spurious responses obtained in clause 7.6B.3.3_1.1 under the same conditions.

7.7B.3_1.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.3.3_1.1.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.3.3_1.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.7B.3_1.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.7B.3_1.1.5-1 +( [10log(S_LCRB/NRB_alloc)] for NR CC, [10log(P_LCRB/NRB_alloc)] for E-UTRA CC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7B.3_1.1.5-1. The spurious frequencies are taken from records in the final step of test procedures in clause 7.6B.3.3_1.1.4.2.

5.	For each spurious frequency, Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.7B.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.7B.3_1.1.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.7B.3_1.1.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.7B.3_1.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.7B.3_1.1.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.7B.3_1.1.5-1: Spurious Response for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

#### 7.7B.3_1.2	Spurious Response for EN-DC within FR1 (4 CCs)

7.7B.3_1.2.1	Test Purpose

Spurious response for EN-DC verifies the receiver's ability to receive a wanted aggregated signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in clause 7.6B.3.3_1.2 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7B.3_1.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 4 DL CCs.

7.7B.3_1.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7B.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.7B.3_1.2.4	Test Description

7.7B.3_1.2.4.1	Initial condition

The initial conditions shall be the same as in clause 7.6B.3.3_1.2.4.1 in order to test spurious responses obtained in clause 7.6B.3.3_1.2 under the same conditions.

7.7B.3_1.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6B.3.3_1.2.4.1-1 on E-UTRA CC and NR CC respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6B.3.3_1.2.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.7B.3_1.2.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.7B.3_1.2.5-1 +( [10log(S_LCRB/NRB_alloc)] for NR CC, [10log(P_LCRB/NRB_alloc)] for E-UTRA CC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

4.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7B.3_1.2.5-1. The spurious frequencies are taken from records in the final step of test procedures in clause 7.6B.3.3_1.2.4.2.

5.	For each spurious frequency, Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.7B.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.7B.3_1.2.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.7B.3_1.2.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.7B.3_1.2.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.7B.3_1.2.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.7B.3_1.2.5-1: Spurious Response for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

## 7.7E	Spurious response for V2X in FR1

### 7.7E.0	Minimum conformance requirements

For intra-band V2X operation, the spurious response specified in clause 7.7.1G in TS 36.101 [5] and specified in clause 7.7E in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For the inter-band con-current NR V2X operation, the spurious response requirements shall be applied per each component carrier. The requirements specified in subclause 7.7E of TS 38.101-1 [2] shall apply for the NR sidelink reception in Band n47 and the requirements specified in subclause 7.7.1 of TS 36.101 [5] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active. The requirements specified in subclause 7.7.1G of TS 36.101 [5] shall apply for the E-UTRA sidelink reception and the requirements specified in subclause 7.7 of TS 38.101-1 [2] shall apply for the NR downlink reception while all downlink carriers are active.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.7E.

### 7.7E.1	Spurious response for V2X operation

7.7E.1.1	Test purpose

Spurious response verifies the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in sub-clause 7.6E.2 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7E.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 7.7 and 7.7G of TS 36.521-1 [10] and clause 7.7 and 7.7E of TS 38.521-1 [8] and don’t need to be tested again.

## 7.8	Void

## 7.8B	Intermodulation characteristics for DC

### 7.8B.1	General

### 7.8B.2	Wide band Intermodulation

#### 7.8B.2.0	Minimum Conformance Requirements

##### 7.8B.2.0.1	Intra-band contiguous EN-DC

Intra-band contiguous EN-DC wide band intermodulation requirement and parameters are defined in Table 7.8B.2.0.1-1.

Table 7.8B.2.0.1-1: Wide band intermodulation

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | 16.8 | 17.5 | 18.0 |
| Pinterferer 1, dBm (CW)2 | -46 |  |  |  |
| Pinterferer 2, dBm (Modulated)2 | -46 |  |  |  |
| NOTE 1: PW is wanted signal power level from Table 7.8.1A-1 in TS 36.101 [5] NOTE 2: Jammer BW and offsets is from Table 7.8.1A-1 [5] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 3: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3-3 with PCMAX_L,f,c as defined in clause 6.2B.4. NOTE 4: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L,c at the minimum uplink configuration specified in Table 7.3-1-2 with PCMAX_L,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 7.8B.2.1.

##### 7.8B.2.0.2	Intra-band non-contiguous EN-DC

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.8.1 for single carrier operation and in clause 7.8.1A for CA in TS 36.101 [5].

For the NR sub-block, the requirement is defined in clause 7.8.2 in TS 38.101-1 [2].

The blocker configuration is defined in the general clause 7.1 and the requirement only apply for out of gap interferers.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.8B.2.2.

##### 7.8B.2.0.3	Inter-band EN-DC within FR1

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.8.2 and 7.8A.2 of TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.8B.2.3.

##### 7.8B.2.0.4	Inter-band EN-DC including FR2

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [5] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.8B.2.4.

##### 7.8B.2.0.5	Inter-band EN-DC including both FR1 and FR2

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.8.2 and 7.8A.2 of TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.8B.2.5.

#### 7.8B.2.1	Wideband Intermodulation for intra-band contiguous EN-DC (2 CCs)

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	UL Power configuration is TBD

7.8B.2.1.1	Test Purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8B.2.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.8B.2.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.8B.2.0.1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.8B.2.1.4	Test Description

7.8B.2.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 7.8B.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.8B.2.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  | Highest |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS | QPSK | REFSENS |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. NOTE 4: REFSENS refers to Uplink configuration in Table 7.3.2-3 in [8] and Table 7.3.3-2 in [10] for NR and E-UTRA CC respectively. NOTE 5: In an E-UTRA band or NR FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.8B.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 7.8B.2.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the Downlink signal level to the value as defined in Table 7.8B.2.1.5-1. For NR CC and E-UTRA CC, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.8B.2.1.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size. and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of  TS 36.521-1 [10].

3.	Set the Interfering signal levels to the values as defined in Table 7.8B.2.1.5-1 and frequency below the wanted signal

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

5.	Repeat steps from 2 to 4, using an interfering signal above the wanted signal at step 3.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

7.8B.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 with DFT-s-OFDM condition in Table 4.6.3-118 PUSCH-Config.

7.8B.2.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Table 7.8B.2.1.5-1 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8B.2.1.5-1: Wide band intermodulation

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | 16.8 | 17.5 | 18.0 |
| Pinterferer 1, dBm (CW)2 | -46 |  |  |  |
| Pinterferer 2, dBm (Modulated)2 | -46 |  |  |  |
| NOTE 1: PW is wanted signal power level from Table 7.8.1A-1 in TS 36.101 [5] NOTE 2: Jammer BW and offsets is from Table 7.8.1A-1 in TS 36.101 [5] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 3: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3-3 with PCMAX_L,f,c as defined in clause 6.2.4 from [2]. NOTE 4: For E-UTRA carrier, the transmitter shall be set to 29dB below PCMAX_L,c at the minimum uplink configuration specified in Table 7.3-1-2 with PCMAX_L,c as defined in clause 6.2.5 for single carrier and in Table 7.3-1A-1 with PCMAX_L as defined in clause 6.2.5A for LTE-CA from TS 36.101 [5]. |  |  |  |  |

Table 7.8B.2.1.5-2: Void

#### 7.8B.2.2	Wideband Intermodulation for intra-band non-contiguous EN-DC (2 CCs)

7.8B.2.2.1	Test Purpose

Same test purpose as in clause 7.8.2.1 in TS 38.521-1 [8] for the NR carrier.

7.8B.2.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC in FR1 with 2 DL CCs.

7.8B.2.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.8B.2.0.2.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.8B.2.2.4	Test Description

Same test description as in clause 7.8.2.4 in TS 38.521-1 [8] with the following exceptions:

Table 7.8B.2.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | High range with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg, only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 7.9B.2.4-1.

For Initial conditions as in clause 7.8.2.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Step 6 of Initial conditions as in clause 7.8.2.4.2 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

Step 4 of Test procedure is replaced by:

4.	Set the Interfering signal levels to the values as defined in Table 7.8.2.5-1 and frequency at the out-of-gap of the sub-blocks.

Step 6 of Test procedure is removed.

7.8B.2.2.5	Test Requirement

Same test requirement as in clause 7.8.2.5 in TS 38.521-1 [8].

#### 7.8B.2.3	Wideband Intermodulation for inter-band EN-DC within FR1 (2 CCs)

7.8B.2.3.1	Test Purpose

Same test purpose as in clause 7.8.2.1 in TS 38.521-1 [8] for the NR carrier.

7.8B.2.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.8B.2.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.8B.2.0.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

7.8B.2.3.4	Test Description

Same test description as in clause 7.8.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

For band n28 / 28 used in EN-DC configurations with NOTE 8 of Table 5.5B.4.1-1, test channel bandwidth of 20MHz or more is tested with Low range test frequency.

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.8.2.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Step 6 of Initial conditions as in clause 7.8.2.4.2 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.8B.2.3.5	Test Requirement

Same test requirement as in clause 7.8.2.5 in TS 38.521-1 [8].

#### 7.8B.2.3_1	Wideband Intermodulation for EN-DC within FR1 (>2 CCs)

##### 7.8B.2.3_1.1	Wideband Intermodulation for EN-DC within FR1 (3 CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	Test configuration for DC_(n)XAA-nYA, DC_XA-(n)YAA, DC_XA-nXA-nYA, DC_XA-YA_nYA is missing in Table 7.8B.2.3_1.1.4.1-1.

7.8B.2.3_1.1.1	Test Purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8B.2.3_1.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC within FR1 (3 CCs).

7.8B.2.3_1.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.8B.2.0.

7.8B.2.3_1.1.4	Test Description

7.8B.2.3_1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.2, 5.5B.3, and 5.5B.4, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.5B.2, 5.5B.3, and 5.5B.4 and are shown in table 7.8B.2.3_1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.8B.2.3_1.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  | Highest NRB_agg |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Highest |
| Network signalling value |  |  |  |  | NS_01 by default |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |
| EN-DC Configurations | Environment | Frequency | Bandwidth Combination | SCS | Other Parameter Settings |
| DC_(n)XCA | Default | Default | Default | Default | As per DC_(n)XCA in Table 7.3B.2.3_1.1.4.1-0 |
| DC_XC-nXA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.2) |
| DC_XA_XA-nXA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.2) |
| DC_XA_nYC | Default | Default | Default | Default | As per DC_XA-nYC in Table 7.3B.2.3_1.14.1-0 |
| DC_XA_nY(2A) | Default | Default | Default | Default | As per DC_XA_nY(2A) in Table 7.3B.2.3_1.14.1-0 |
| DC_XA_nYA-nZA | Default | Default | Default | Default | As per DC_XA_nYA-nZA in Table 7.3B.2.3_1.14.1-0 |
| DC_XC_nYA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3) |
| DC_XA-XA_nYA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3) |
| DC_XA-YA_nZA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3) |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: X, Y and Z in this table correspond to different bands i.e. X != Y != Z NOTE 5: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA NOTE 6: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

7.	For the E-UTRA CC(s) where anchor agnostic apply according to NOTE 1 in Table 7.3B.2.3_1.1.4.1-0, downlink signal level and uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10]. Disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.8B.2.3_1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 7.8B.2.3_1.1.4.2-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the Downlink signal level to the value as defined in Table 7.8B.2.3_1.1.4.2-1. For E-UTRA CC and NR CC where uplink is allocated according to Table 7.8B.2.3_1.1.4.1-1, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to - (MU + Uplink power control window size) dB of the 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2B.4 for at least the duration of the Throughput measurement.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than, or no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

3.	Set the Interfering signal levels to the values as defined in Table 7.8B.2.3_1.1.4.2-1 and frequency below the wanted signal.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G, clause G.2.

5.	Repeat steps from 2 to 4, using an interfering signal above the wanted signal at step 3.

6.	Repeat steps 2 to 5 for component carriers listed in Table 7.8B.2.3_1.1.4.2-1.

Table 7.8B.2.3_1.1.4.2-1: Test repetition and measurement configuration

| DC configuration | Test parameters to select (NOTE 2) |
| --- | --- |
| DC_(n)XCA | 7.8B.2.1.5 |
| DC_XA_nYC | 7.8B.2.3.5 |
| DC_XA_nY(2A) |  |
| DC_XA_nYA-nZA |  |
| NOTE 1: Void NOTE 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. NOTE 3: Measure throughput on DL allocated CC. Where there are multiple rows for a single Test point ID, the test is repeated for each row. |  |

7.8B.2.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 with DFT-s-OFDM condition in Table 4.6.3-118 PUSCH-Config.

7.8B.2.3_1.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A, clause A.3.2 with parameters specified in Table 7.8B.2.3_1.1.4.2-1 for the specified wanted signal mean power in the presence of two interfering signals.

##### 7.8B.2.3_1.2	Wideband Intermodulation for EN-DC within FR1 (4 CCs)

7.8B.2.3_1.2.1	Test Purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8B.2.3_1.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC within FR1 (4 CCs).

7.8B.2.3_1.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.8B.2.0.

7.8B.2.3_1.2.4	Test Description

7.8B.2.3_1.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.2, 5.5B.3, and 5.5B.4, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 7.8B.2.3_1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.8B.2.3_1.2.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  | Highest NRB_agg |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Highest |
| Network signalling value |  |  |  |  | NS_01 by default |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |
| EN-DC Configurations | Environment | Frequency | Bandwidth Combination | SCS | Other Parameter Settings |
| DC_(n)XDA | Default | Default | Default | Default | As per DC_(n)XDA in Table 7.3B.2.3_1.2.4.1-1 |
| DC_XD_nXA | - | - | - | - | No test required (LTE 2CC fallback is tested in 7.8B.2.2) |
| DC_XA-XC_nXA | - | - | - | - | No test required (LTE 2CC fallback is tested in 7.8B.2.2) |
| DC_XA_nY(2A)-nZA | Default | Default | Default | Default | As per DC_XA_nY(2A)-nZA in Table 7.3B.2.3_1.2.4.1-1 |
| DC_XA_nYA-nZC | Default | Default | Default | Default | As per DC_XA_nYA-nZC in Table 7.3B.2.3_1.2.4.1-1 |
| DC_XD_nYA DC_XA-YC_nZA DC_XA-XA-YA_nZA DC_XA-YA-ZA_nRA | - | - | - | - | No test required (LTE 2CC fallback is tested in 7.8B.2.2) |
| DC_XC_nYC DC_XA-XA_nYC DC_XA-YA_nYC | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3_1.1) |
| DC_XC_nY(2A) DC_XA-XA_nY(2A) DC_XA-YA_nY(2A) | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3_1.1) |
| DC_XC_nYA-nZA DC_XA-XA_nYA-nZA DC_XA-YA_nZA-nRA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3_1.1) |
| DC_XA-YA_(n)ZAA DC_XC_(n)YAA | - | - | - | - | No test required (LTE 2CC fallback is tested in 7.8B.2.2) |
| DC_(n)XCA-nYA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3_1.1) |
| DC_XA-YA-ZA_nZA | - | - | - | - | No test required (LTE 2CC fallback is tested in 7.8B.2.2) |
| DC_XA-YA_nYA-nZA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3_1.1) |
| NOTE 1: X, Y, Z and R in this table correspond to different bands i.e. X != Y != Z != R NOTE 2: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

7.	For the E-UTRA CC(s) where anchor agnostic apply according to NOTE 4 in Table 7.3B.2.3_1.2.4.1-1, downlink signal level and uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10]. Disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.8B.2.3_1.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 7.8B.2.3_1.2.4.2-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the Downlink signal level to the value as defined in Table 7.8B.2.3_1.2.4.2-1. For E-UTRA CC and NR CC where uplink is allocated according to Table 7.8B.2.3_1.2.4.1-1, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to - (MU + Uplink power control window size) dB of the 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2B.4 for at least the duration of the Throughput measurement.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than, or no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

3.	Set the Interfering signal levels to the values as defined in Table 7.8B.2.3_1.2.4.2-1 and frequency below the wanted signal.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G, clause G.2.

5.	Repeat steps from 2 to 4, using an interfering signal above the wanted signal at step 3.

6.	Repeat steps 2 to 5 for component carriers listed in Table 7.8B.2.3_1.2.4.2-1.

Table 7.8B.2.3_1.2.4.2-1: Test repetition and measurement configuration

| DC configuration | Test parameters to select (NOTE 1) |
| --- | --- |
| DC_(n)XDA | 7.8B.2.1.5 |
| DC_XA_nY(2A)-nZA | 7.8B.2.3.5 |
| DC_XA_nYA-nZC |  |
| NOTE 1: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. NOTE 2: Measure throughput on DL allocated CC. Where there are multiple rows for a single Test point ID, the test is repeated for each row. |  |

7.8B.2.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 with DFT-s-OFDM condition in Table 4.6.3-118 PUSCH-Config.

7.8B.2.3_1.2.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A, clause A.3.2 with parameters specified in Table 7.8B.2.3_1.2.4.2-1 for the specified wanted signal mean power in the presence of two interfering signals.

##### 7.8B.2.3_1.3	Wideband Intermodulation for EN-DC within FR1 (5 CCs)

Editor's note:	The Table 7.8B.2.3_1.3.4.2-1 are either missing or not yet determined.

7.8B.2.3_1.3.1	Test Purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8B.2.3_1.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC within FR1 (5 CCs).

7.8B.2.3_1.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.8B.2.0.

7.8B.2.3_1.3.4	Test Description

7.8B.2.3_1.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.2, 5.5B.3, and 5.5B.4, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 7.8B.2.3_1.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.8B.2.3_1.3.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  | Highest NRB_agg |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Highest |
| Network signalling value |  |  |  |  | NS_01 by default |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |
| EN-DC Configurations | Environment | Frequency | Bandwidth Combination | SCS | Other Parameter Settings |
| DC_XA-XA-YC_nZA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3_1.2) |
| DC_XA-YC_nZ(2A) | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3_1.2) |
| NOTE 1: X, Y, Z and R in this table correspond to different bands i.e. X != Y != Z != R NOTE 2: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

7.	For the E-UTRA CC(s) where anchor agnostic apply according to NOTE 4 in Table 7.3B.2.3_1.2.4.1-1, downlink signal level and uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10]. Disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.8B.2.3_1.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 7.8B.2.3_1.3.4.2-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the Downlink signal level to the value as defined in Table 7.8B.2.3_1.3.4.2-1. For E-UTRA CC and NR CC where uplink is allocated according to Table 7.8B.2.3_1.3.4.1-1, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to - (MU + Uplink power control window size) dB of the 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2B.4 for at least the duration of the Throughput measurement.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than, or no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

3.	Set the Interfering signal levels to the values as defined in Table 7.8B.2.3_1.3.4.2-1 and frequency below the wanted signal.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G, clause G.2.

5.	Repeat steps from 2 to 4, using an interfering signal above the wanted signal at step 3.

6.	Repeat steps 2 to 5 for component carriers listed in Table 7.8B.2.3_1.3.4.2-1.

Table 7.8B.2.3_1.3.4.2-1: Test repetition and measurement configuration

FFS

7.8B.2.3_1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 with DFT-s-OFDM condition in Table 4.6.3-118 PUSCH-Config.

7.8B.2.3_1.3.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A, clause A.3.2 with parameters specified in Table 7.8B.2.3_1.3.4.2-1 for the specified wanted signal mean power in the presence of two interfering signals.

##### 7.8B.2.3_1.4	Wideband Intermodulation for EN-DC within FR1 (6 CCs)

TBD

#### 7.8B.2.3a	Wideband Intermodulation for inter-band NE-DC within FR1 (2 CCs)

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 7.8.2 and 7.8A.2 of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 7.8.1 and 7.8.1A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

#### 7.8B.2.4 to 7.8B.2.9	Void

## 7.8E	Intermodulation characteristics for V2X operation in FR1

### 7.8E.1	Wide band Intermodulation for V2X

#### 7.8E.1.0	Minimum conformance requirements

For intra-band V2X operation, the intermodulation characteristics specified in clause 7.8.1G in TS 36.101 [5] and specified in clause 7.8E in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For inter-band NR V2X con-current operation, the intermodulation characteristics requirements shall be applied per each component carrier. The wideband inter-modulation requirement specified in clause 7.8E in TS 38.101-1 [2] shall apply on NR V2X carrier and the requirement specified in clause 7.8.1 in TS 36.101 [5] shall apply on E-UTRA downlink reception in licensed band while all downlink carriers are active. The requirements specified in subclause 7.8.1G of TS 36.101 [5] shall apply for the E-UTRA sidelink reception and the requirements specified in subclause 7.8 of TS 38.101-1 [2] shall apply for the NR downlink reception while all downlink carriers are active. PInterferer power is increased by ΔRIB,c in the requirement.

#### 7.8E.1.1	Wide band Intermodulation for V2X operation

7.8E.1.1.1	Test purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8E.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 7.8.1 and 7.8.1G of TS 36.521-1 [10] and clause 7.8.2 and 7.8E.2 of TS 38.521-1 [8] and don’t need to be tested again.

## 7.9	Void

## 7.9B	Spurious emissions for DC

### 7.9B.0	Minimum Conformance Requirements

#### 7.9B.0.1	Intra-band contiguous EN-DC

The requirement is defined in clause 7.9A.1 in TS 38.101-1 [2].

The normative reference for this requirement is TS 38.101-3 [4] clause 7.9B.1.

#### 7.9B.0.2	Intra-band non-contiguous EN-DC

Spurious emissions requirement for E-UTRA single carrier and CA operation specified in clauses 7.9.1 and 7.9.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.9 and 7.9A of TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.9B.2.

#### 7.9B.0.3	Inter-band EN-DC within FR1

E-UTRA requirements from TS 36.101 [5] and NR requirements from TS 38.101-1 [2] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.9B.3.

#### 7.9B.0.4	Inter-band EN-DC including FR2

Spurious emissions requirement for E-UTRA single carrier and CA operation specified in clauses 7.9.1 and 7.9.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clause 7.9 of TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.9B.4.

#### 7.9B.0.5	Inter-band EN-DC including both FR1 and FR2

Spurious emissions requirement for E-UTRA single carrier and CA operation specified in clauses 7.9.1 and 7.9.1A of TS 36.101 [5] and for NR single carrier and CA operation specified in clauses 7.9 and 7.9A of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.9B.5.

### 7.9B.1	Spurious Emissions for intra-band contiguous EN-DC (2 CCs)

7.9B.1.1	Test purpose

Same test purpose as in clause 7.9.1 in TS 38.521-1 [8] for the NR carrier.

7.9B.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.9B.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.9B.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.9B.1.4	Test description

Same test description as in clause 7.9.4 in TS 38.521-1 [8] with the following exceptions:

Table 7.9B.1.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different DC bandwidth classes. | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 7.9B.1.4-1.

For Initial conditions as in clause 7.9.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Step 6 of Initial conditions as in clause 7.9.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.9B.1.5	Test requirement

Same test requirement as in clause 7.9.5 in TS 38.521-1 [8].

### 7.9B.2	Spurious Emissions for intra-band non-contiguous EN-DC (2 CCs)

7.9B.2.1	Test purpose

Same test purpose as in clause 7.9.1 in TS 38.521-1 [8] for the NR carrier.

7.9B.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous in FR1 with 2 DL CCs.

7.9B.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.9B.0.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

7.9B.2.4	Test description

Same test description as in clause 7.9.4 in TS 38.521-1 [8] with the following exceptions:

Table 7.9B.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different DC bandwidth classes | MaxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 7.9B.2.4-1.

For Initial conditions as in clause 7.9.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Step 6 of Initial conditions as in clause 7.9.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

7.9B.2.5	Test requirement

Same test requirement as in clause 7.9.5 in TS 38.521-1 [8].

### 7.9B.3	Spurious Emissions for inter-band EN-DC within FR1 (1 NR CC)

7.9B.3.1	Test purpose

Same test purpose as in clause 7.9.1 in TS 38.521-1 [8] for the NR carrier.

7.9B.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR DL CC.

7.9B.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.9B.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.9B.3.4	Test description

Same test description as in clause 7.9.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.9.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Step 6 of Initial conditions as in clause 7.9.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.9B.3.5	Test requirement

Same test requirement as in clause 7.9.5 in TS 38.521-1 [8].

### 7.9B.3_1	Spurious Emissions for EN-DC within FR1 (>2 CCs)

#### 7.9B.3_1.1	Spurious Emissions for EN-DC within FR1 (3 CCs)

7.9B.3_1.1.1	Test purpose

Same test purpose as in clause 7.9B.1.

7.9B.3_1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC within FR1 with 3CCs(1 LTE CC + 2 Inter-band NR CCs with a DL-only NR band).

7.9B.3_1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.9B.0.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.9B.3_1.1.4	Test description

Same test description as in clause 7.9.4 or 7.9.A.1.4 in TS 38.521-1 [8] with the following exceptions:

Table 7.9B.1.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different DC bandwidth classes. | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

The initial test configurations for E-UTRA as specified in Table 4.6-1 except for the parameters specified in Table 7.9B.1.4-1.

For Initial conditions as in clause 7.9.4.1 or 7.9A.1.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Step 6 of Initial conditions as in clause 7.9.4.1 or 7.9A.1.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.9B.3_1.1.5	Test requirement

Same test requirement as in clause 7.9.5 in TS 38.521-1 [8].

### 7.9B.3a	Spurious Emissions for inter-band NE-DC within FR1 (2 CCs)

No exception requirements applicable to NR or E-UTRA.

No test case details are specified. The requirements for NR carrier(s) in this test case are tested in 7.9 and 7.9A of TS 38.521-1 [8], and the requirements for LTE carrier(s) in this test case are tested in 7.9 and 7.9A of TS 36.521-1 [10]. Neither NR carrier(s) nor LTE carrier(s) needs to be tested again.

### 7.9B.4	Spurious Emissions for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	For n259, measurement uncertainty is TBD in the range 80 GHz < f <= 87 GHz. Up to 80GHz, for n259 and PC3 this clause is complete.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 2, and 4.

7.9B.4.1	Test purpose

Same test purpose as in clause 7.9.1 in TS 38.521-2 [9] for the NR carrier.

7.9B.4.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR DL CC.

7.9B.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.9.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.9B.4.

7.9B.4.4	Test description

Same test description as in clause 7.9.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For initial conditions as in clause 7.9.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.9.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 7.9.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.9B.4.5	Test requirements

Same test requirement as in clause 7.9.5 in TS 38.521-2 [9] for the NR carrier.

### 7.9B.5	Spurious Emissions for inter-band EN-DC including both FR1 and FR2 (3 CCs)

TBD

Annex A (normative):
Measurement Channels

Please refer to Annex A in TS 38.521-1 [8] and 38.521-2 [9] for appropriate details as needed for test cases in this test specification. EN-DC exceptions will be added.

# A.1	General

The throughput values defined in the measurement channels specified in Annex A, are calculated and are valid per datastream (codeword). For multi-stream (more than one codeword) transmissions, the throughput referenced in the minimum requirements is the sum of throughputs of all datastreams (codewords).

The UE category entry in the definition of the reference measurement channel in Annex A is only informative and reveals the UE categories, which can support the corresponding measurement channel. Whether the measurement channel is used for testing a certain UE category or not is specified in the individual minimum requirements.

# A.2	UL reference measurement channels for E-UTRA TDD Config 2

## A.2.1	General

The measurement channels in the following clauses are defined to derive the requirements in clause 6 (Transmitter Characteristics) and clause 7 (Receiver Characteristics). The measurement channels represent example configurations of physical channels for different data rates.

## A.2.2	Reference measurement channels for E-UTRA

### A.2.2.1	Full RB allocation

#### A.2.2.1.1	QPSK

Table A.2.2.1.1-1: Reference Channels for QPSK with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (Note 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (Note 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/5 | 1/6 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 600 | 1544 | 2216 | 5160 | 4392 | 4584 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 1728 | 4320 | 7200 | 14400 | 21600 | 28800 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [13] Note 3: As per Table 4.2-1 in TS 36.211 [13] |  |  |  |  |  |  |  |

#### A.2.2.1.2	16-QAM

Table A.2.2.1.2-1: Reference Channels for 16-QAM with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (Note 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (Note 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding rate |  | 3/4 | 1/2 | 1/3 | 3/4 | 1/2 | 1/3 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 2600 | 4264 | 4968 | 21384 | 21384 | 19848 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 1 | 1 | 4 | 4 | 4 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 3456 | 8640 | 14400 | 28800 | 43200 | 57600 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [13] Note 3: As per Table 4.2-1 in TS 36.211 [13] |  |  |  |  |  |  |  |

#### A.2.2.1.3	64-QAM

Table A.2.2.1.3-1: Reference Channels for 64-QAM with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (Note 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (Note 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 3752 | 9528 | 15840 | 31704 | 46888 | 63776 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 2 | 3 | 6 | 8 | 11 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 5184 | 12960 | 21600 | 43200 | 64800 | 86400 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category (Note 4) |  | 5, 8 | 5, 8 | 5, 8 | 5, 8 | 5, 8 | 5, 8 |
| UE UL Category (Note 4) |  | 5, 8, 13, 14 | 5, 8, 13, 14 | 5, 8, 13, 14 | 5, 8, 13, 14 | 5, 8, 13, 14 | 5, 8, 13, 14 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [13] Note 3: As per Table 4.2-1 in TS 36.211 [13] Note 4: If UE does not report UE UL category, then the applicability of reference channel is determined by UE category. If UE reports UE UL category, then the applicability of reference channel is determined by UE UL category. |  |  |  |  |  |  |  |

#### A.2.2.1.4	256 QAM

Table A.2.2.1.4-1: Reference Channels for 256 QAM with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (Note 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (Note 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Target Coding rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 5160 | 12960 | 21384 | 42368 | 63776 | 84760 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 3 | 4 | 8 | 11 | 15 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 6912 | 17280 | 28800 | 57600 | 86400 | 115200 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE UL Category |  | ≥ 15 | ≥ 15 | ≥ 15 | ≥ 15 | ≥ 15 | ≥ 15 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [13] Note 3: As per Table 4.2-1 in TS 36.211 [13] |  |  |  |  |  |  |  |

### A.2.2.2	Partial RB allocation

#### A.2.2.2.1	QPSK

Table A.2.2.2.1-1: Reference Channels for QPSK with partial RB allocation

| Parameter | Ch BW | Allocated RBs | UL-DL Configuration (Note 2) | Special subframe configuration (Note 3) | DFT-OFDM Symbols per Sub-Frame | Mod'n | Target Coding rate | Payload size for Sub-Frame 2, 7 | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 7 | Total symbols per Sub-Frame for Sub-Frame 2, 7 | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | ≥ 1 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | ≥ 1 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | ≥ 1 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | ≥ 1 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | ≥ 1 |
|  | 3-20 | 6 | 2 | 7 | 12 | QPSK | 1/3 | 600 | 24 | 1 | 1728 | 864 | ≥ 1 |
|  | 3-20 | 8 | 2 | 7 | 12 | QPSK | 1/3 | 808 | 24 | 1 | 2304 | 1152 | ≥ 1 |
|  | 3-20 | 9 | 2 | 7 | 12 | QPSK | 1/3 | 776 | 24 | 1 | 2592 | 1296 | ≥ 1 |
|  | 3-20 | 10 | 2 | 7 | 12 | QPSK | 1/3 | 872 | 24 | 1 | 2880 | 1440 | ≥ 1 |
|  | 3-20 | 12 | 2 | 7 | 12 | QPSK | 1/3 | 1224 | 24 | 1 | 3456 | 1728 | ≥ 1 |
|  | 5-20 | 15 | 2 | 7 | 12 | QPSK | 1/3 | 1320 | 24 | 1 | 4320 | 2160 | ≥ 1 |
|  | 5-20 | 16 | 2 | 7 | 12 | QPSK | 1/3 | 1384 | 24 | 1 | 4608 | 2304 | ≥ 1 |
|  | 5-20 | 18 | 2 | 7 | 12 | QPSK | 1/3 | 1864 | 24 | 1 | 5184 | 2592 | ≥ 1 |
|  | 5-20 | 20 | 2 | 7 | 12 | QPSK | 1/3 | 1736 | 24 | 1 | 5760 | 2880 | ≥ 1 |
|  | 5-20 | 24 | 2 | 7 | 12 | QPSK | 1/3 | 2472 | 24 | 1 | 6912 | 3456 | ≥ 1 |
|  | 10-20 | 25 | 2 | 7 | 12 | QPSK | 1/3 | 2216 | 24 | 1 | 7200 | 3600 | ≥ 1 |
|  | 10-20 | 27 | 2 | 7 | 12 | QPSK | 1/3 | 2792 | 24 | 1 | 7776 | 3888 | ≥ 1 |
|  | 10-20 | 30 | 2 | 7 | 12 | QPSK | 1/3 | 2664 | 24 | 1 | 8640 | 4320 | ≥ 1 |
|  | 10-20 | 32 | 2 | 7 | 12 | QPSK | 1/3 | 2792 | 24 | 1 | 9216 | 4608 | ≥ 1 |
|  | 10-20 | 36 | 2 | 7 | 12 | QPSK | 1/3 | 3752 | 24 | 1 | 10368 | 5184 | ≥ 1 |
|  | 10-20 | 40 | 2 | 7 | 12 | QPSK | 1/3 | 4136 | 24 | 1 | 11520 | 5760 | ≥ 1 |
|  | 10-20 | 45 | 2 | 7 | 12 | QPSK | 1/3 | 4008 | 24 | 1 | 12960 | 6480 | ≥ 1 |
|  | 10-20 | 48 | 2 | 7 | 12 | QPSK | 1/3 | 4264 | 24 | 1 | 13824 | 6912 | ≥ 1 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | QPSK | 1/3 | 5160 | 24 | 1 | 14400 | 7200 | ≥ 1 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | QPSK | 1/3 | 4776 | 24 | 1 | 15552 | 7776 | ≥ 1 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | QPSK | 1/4 | 4264 | 24 | 1 | 17280 | 8640 | ≥ 1 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | QPSK | 1/4 | 4584 | 24 | 1 | 18432 | 9216 | ≥ 1 |
|  | 15 - 20 | 72 | 2 | 7 | 12 | QPSK | 1/4 | 5160 | 24 | 1 | 20736 | 10368 | ≥ 1 |
|  | 20 | 75 | 2 | 7 | 12 | QPSK | 1/5 | 4392 | 24 | 1 | 21600 | 10800 | ≥ 1 |
|  | 20 | 80 | 2 | 7 | 12 | QPSK | 1/5 | 4776 | 24 | 1 | 23040 | 11520 | ≥ 1 |
|  | 20 | 81 | 2 | 7 | 12 | QPSK | 1/5 | 4776 | 24 | 1 | 23328 | 11664 | ≥ 1 |
|  | 20 | 90 | 2 | 7 | 12 | QPSK | 1/6 | 4008 | 24 | 1 | 25920 | 12960 | ≥ 1 |
|  | 20 | 96 | 2 | 7 | 12 | QPSK | 1/6 | 4264 | 24 | 1 | 27648 | 13824 | ≥ 1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [13] Note 3: As per Table 4.2-1 in TS 36.211 [13] |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.2.2.2.2	16-QAM

Table A.2.2.2.2-1: Reference Channels for 16QAM with partial RB allocation

| Parameter | Ch BW | Allocated RBs | UL-DL Configuration (Note 2) | Special subframe configuration (Note 3) | DFT-OFDM Symbols per Sub-Frame | Mod'n | Target Coding rate | Payload size for Sub-Frame 2, 7 | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 7 | Total symbols per Sub-Frame for Sub-Frame 2, 7 | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | 16QAM | 3/4 | 408 | 24 | 1 | 576 | 144 | ≥ 1 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | 16QAM | 3/4 | 840 | 24 | 1 | 1152 | 288 | ≥ 1 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | 16QAM | 3/4 | 1288 | 24 | 1 | 1728 | 432 | ≥ 1 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | 16QAM | 3/4 | 1736 | 24 | 1 | 2304 | 576 | ≥ 1 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | 16QAM | 3/4 | 2152 | 24 | 1 | 2880 | 720 | ≥ 1 |
|  | 3-20 | 6 | 2 | 7 | 12 | 16QAM | 3/4 | 2600 | 24 | 1 | 3456 | 864 | ≥ 1 |
|  | 3-20 | 8 | 2 | 7 | 12 | 16QAM | 3/4 | 3496 | 24 | 1 | 4608 | 1152 | ≥ 1 |
|  | 3-20 | 9 | 2 | 7 | 12 | 16QAM | 3/4 | 3880 | 24 | 1 | 5184 | 1296 | ≥ 1 |
|  | 3-20 | 10 | 2 | 7 | 12 | 16QAM | 3/4 | 4264 | 24 | 1 | 5760 | 1440 | ≥ 1 |
|  | 3-20 | 12 | 2 | 7 | 12 | 16QAM | 3/4 | 5160 | 24 | 1 | 6912 | 1728 | ≥ 1 |
|  | 5-20 | 15 | 2 | 7 | 12 | 16QAM | 1/2 | 4264 | 24 | 1 | 8640 | 2160 | ≥ 1 |
|  | 5-20 | 16 | 2 | 7 | 12 | 16QAM | 1/2 | 4584 | 24 | 1 | 9216 | 2304 | ≥ 1 |
|  | 5-20 | 18 | 2 | 7 | 12 | 16QAM | 1/2 | 5160 | 24 | 1 | 10368 | 2592 | ≥ 1 |
|  | 5-20 | 20 | 2 | 7 | 12 | 16QAM | 1/3 | 4008 | 24 | 1 | 11520 | 2880 | ≥ 1 |
|  | 5-20 | 24 | 2 | 7 | 12 | 16QAM | 1/3 | 4776 | 24 | 1 | 13824 | 3456 | ≥ 1 |
|  | 10-20 | 25 | 2 | 7 | 12 | 16QAM | 1/3 | 4968 | 24 | 1 | 14400 | 3600 | ≥ 1 |
|  | 10-20 | 27 | 2 | 7 | 12 | 16QAM | 1/3 | 4776 | 24 | 1 | 15552 | 3888 | ≥ 1 |
|  | 10-20 | 30 | 2 | 7 | 12 | 16QAM | 3/4 | 12960 | 24 | 3 | 17280 | 4320 | ≥ 2 |
|  | 10-20 | 32 | 2 | 7 | 12 | 16QAM | 3/4 | 13536 | 24 | 3 | 18432 | 4608 | ≥ 2 |
|  | 10-20 | 36 | 2 | 7 | 12 | 16QAM | 3/4 | 15264 | 24 | 3 | 20736 | 5184 | ≥ 2 |
|  | 10-20 | 40 | 2 | 7 | 12 | 16QAM | 3/4 | 16992 | 24 | 3 | 23040 | 5760 | ≥ 2 |
|  | 10-20 | 45 | 2 | 7 | 12 | 16QAM | 3/4 | 19080 | 24 | 4 | 25920 | 6480 | ≥ 2 |
|  | 10-20 | 48 | 2 | 7 | 12 | 16QAM | 3/4 | 20616 | 24 | 4 | 27648 | 6912 | ≥ 2 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | 16QAM | 3/4 | 21384 | 24 | 4 | 28800 | 7200 | ≥ 2 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | 16QAM | 3/4 | 22920 | 24 | 4 | 31104 | 7776 | ≥ 2 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | 16QAM | 2/3 | 23688 | 24 | 4 | 34560 | 8640 | ≥ 2 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | 16QAM | 2/3 | 25456 | 24 | 4 | 36864 | 9216 | ≥ 2 |
|  | 15 - 20 | 72 | 2 | 7 | 12 | 16QAM | 1/2 | 20616 | 24 | 4 | 41472 | 10368 | ≥ 2 |
|  | 20 | 75 | 2 | 7 | 12 | 16QAM | 1/2 | 21384 | 24 | 4 | 43200 | 10800 | ≥ 2 |
|  | 20 | 80 | 2 | 7 | 12 | 16QAM | 1/2 | 22920 | 24 | 4 | 46080 | 11520 | ≥ 2 |
|  | 20 | 81 | 2 | 7 | 12 | 16QAM | 1/2 | 22920 | 24 | 4 | 46656 | 11664 | ≥ 2 |
|  | 20 | 90 | 2 | 7 | 12 | 16QAM | 2/5 | 20616 | 24 | 4 | 51840 | 12960 | ≥ 2 |
|  | 20 | 96 | 2 | 7 | 12 | 16QAM | 2/5 | 22152 | 24 | 4 | 55296 | 13824 | ≥ 2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [13] Note 3: As per Table 4.2-1 in TS 36.211 [13] |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.2.2.2.3	64-QAM

Table A.2.2.2.3-1: Reference Channels for 64-QAM with partial RB allocation

| Parameter | Ch BW | Allocated RBs | UL-DL Configuration (Note 2) | Special subframe configuration (Note 3) | DFT-OFDM Symbols per Sub-Frame | Mod'n | Target Coding rate | Payload size for Sub-Frame 2, 7 | Trans-port block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 7 | Total symbols per Sub-Frame for Sub-Frame 2, 7 | UE Category (Note 4) | UE UL Category (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  |  | Bits | Bits |  | Bits |  |  |  |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | 64QAM | 3/4 | 616 | 24 | 1 | 864 | 144 | 5,8 | 5, 8, 13, 14 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | 64QAM | 3/4 | 1256 | 24 | 1 | 1728 | 288 | 5,8 | 5, 8, 13, 14 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | 64QAM | 3/4 | 1864 | 24 | 1 | 2592 | 432 | 5,8 | 5, 8, 13, 14 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | 64QAM | 3/4 | 2536 | 24 | 1 | 3456 | 576 | 5,8 | 5, 8, 13, 14 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | 64QAM | 3/4 | 3112 | 24 | 1 | 4320 | 720 | 5,8 | 5, 8, 13, 14 |
|  | 3-20 | 6 | 2 | 7 | 12 | 64QAM | 3/4 | 3752 | 24 | 1 | 5184 | 864 | 5,8 | 5, 8, 13, 14 |
|  | 3-20 | 8 | 2 | 7 | 12 | 64QAM | 3/4 | 5160 | 24 | 1 | 6912 | 1152 | 5,8 | 5, 8, 13, 14 |
|  | 3-20 | 9 | 2 | 7 | 12 | 64QAM | 3/4 | 5736 | 24 | 1 | 7776 | 1296 | 5,8 | 5, 8, 13, 14 |
|  | 3-20 | 10 | 2 | 7 | 12 | 64QAM | 3/4 | 6200 | 24 | 2 | 8640 | 1440 | 5,8 | 5, 8, 13, 14 |
|  | 3-20 | 12 | 2 | 7 | 12 | 64QAM | 3/4 | 7480 | 24 | 2 | 10368 | 1728 | 5,8 | 5, 8, 13, 14 |
|  | 5-20 | 15 | 2 | 7 | 12 | 64QAM | 3/4 | 9528 | 24 | 2 | 12960 | 2160 | 5,8 | 5, 8, 13, 14 |
|  | 5-20 | 16 | 2 | 7 | 12 | 64QAM | 3/4 | 10296 | 24 | 2 | 13824 | 2304 | 5,8 | 5, 8, 13, 14 |
|  | 5-20 | 18 | 2 | 7 | 12 | 64QAM | 3/4 | 11448 | 24 | 2 | 15552 | 2592 | 5,8 | 5, 8, 13, 14 |
|  | 5-20 | 20 | 2 | 7 | 12 | 64QAM | 3/4 | 12576 | 24 | 3 | 17280 | 2880 | 5,8 | 5, 8, 13, 14 |
|  | 5-20 | 24 | 2 | 7 | 12 | 64QAM | 3/4 | 15264 | 24 | 3 | 20736 | 3456 | 5,8 | 5, 8, 13, 14 |
|  | 10-20 | 25 | 2 | 7 | 12 | 64QAM | 3/4 | 15840 | 24 | 3 | 21600 | 3600 | 5,8 | 5, 8, 13, 14 |
|  | 10-20 | 27 | 2 | 7 | 12 | 64QAM | 3/4 | 16992 | 24 | 3 | 23328 | 3888 | 5,8 | 5, 8, 13, 14 |
|  | 10-20 | 30 | 2 | 7 | 12 | 64QAM | 3/4 | 19080 | 24 | 4 | 25920 | 4320 | 5,8 | 5, 8, 13, 14 |
|  | 10-20 | 32 | 2 | 7 | 12 | 64QAM | 3/4 | 20616 | 24 | 4 | 27648 | 4608 | 5,8 | 5, 8, 13, 14 |
|  | 10-20 | 36 | 2 | 7 | 12 | 64QAM | 3/4 | 22920 | 24 | 4 | 31104 | 5184 | 5,8 | 5, 8, 13, 14 |
|  | 10-20 | 40 | 2 | 7 | 12 | 64QAM | 3/4 | 25456 | 24 | 5 | 34560 | 5760 | 5,8 | 5, 8, 13, 14 |
|  | 10-20 | 45 | 2 | 7 | 12 | 64QAM | 3/4 | 28336 | 24 | 5 | 38880 | 6480 | 5,8 | 5, 8, 13, 14 |
|  | 10-20 | 48 | 2 | 7 | 12 | 64QAM | 3/4 | 30576 | 24 | 5 | 41472 | 6912 | 5,8 | 5, 8, 13, 14 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | 64QAM | 3/4 | 31704 | 24 | 6 | 43200 | 7200 | 5,8 | 5, 8, 13, 14 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | 64QAM | 3/4 | 34008 | 24 | 6 | 46656 | 7776 | 5,8 | 5, 8, 13, 14 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | 64QAM | 3/4 | 37888 | 24 | 7 | 51840 | 8640 | 5,8 | 5, 8, 13, 14 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | 64QAM | 3/4 | 40576 | 24 | 7 | 55296 | 9216 | 5,8 | 5, 8, 13, 14 |
|  | 15 - 20 | 72 | 2 | 7 | 12 | 64QAM | 3/4 | 45352 | 24 | 8 | 62208 | 10368 | 5,8 | 5, 8, 13, 14 |
|  | 20 | 75 | 2 | 7 | 12 | 64QAM | 3/4 | 46888 | 24 | 8 | 64800 | 10800 | 5,8 | 5, 8, 13, 14 |
|  | 20 | 80 | 2 | 7 | 12 | 64QAM | 3/4 | 51024 | 24 | 9 | 69120 | 11520 | 5,8 | 5, 8, 13, 14 |
|  | 20 | 81 | 2 | 7 | 12 | 64QAM | 3/4 | 51024 | 24 | 9 | 69984 | 11664 | 5,8 | 5, 8, 13, 14 |
|  | 20 | 90 | 2 | 7 | 12 | 64QAM | 3/4 | 51024 | 24 | 9 | 77760 | 12960 | 5,8 | 5, 8, 13, 14 |
|  | 20 | 96 | 2 | 7 | 12 | 64QAM | 3/4 | 61664 | 24 | 11 | 82944 | 13824 | 5,8 | 5, 8, 13, 14 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [13] Note 3: As per Table 4.2-1 in TS 36.211 [13] Note 4: If UE does not report UE UL category, then the applicability of reference channel is determined by UE category. If UE reports UE UL category, then the applicability of reference channel is determined by UE UL category |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.2.2.2.4	256 QAM

Table A.2.2.2.4-1: Reference Channels for 256 QAM with partial RB allocation

| Parameter | Ch BW | Allocated RBs | UL-DL Configuration (Note 2) | Special Slot Configuration (Note 3) | DFT-OFDM Symbols per Sub-Frame | Mod'n | Target Coding rate | Payload size for Sub-Frame 2, 7 | Trans-port block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 7 | Total symbols per Sub-Frame for Sub-Frame 2, 7 | UE UL Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | 256QAM | 3/4 | 840 | 24 | 1 | 1152 | 144 | ≥ 15 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | 256QAM | 3/4 | 1672 | 24 | 1 | 2304 | 288 | ≥ 15 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | 256QAM | 3/4 | 2536 | 24 | 1 | 3456 | 432 | ≥ 15 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | 256QAM | 3/4 | 3368 | 24 | 1 | 4608 | 576 | ≥ 15 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | 256QAM | 3/4 | 4264 | 24 | 1 | 5760 | 720 | ≥ 15 |
|  | 3-20 | 6 | 2 | 7 | 12 | 256QAM | 3/4 | 5160 | 24 | 1 | 6912 | 864 | ≥ 15 |
|  | 3-20 | 8 | 2 | 7 | 12 | 256QAM | 3/4 | 6712 | 24 | 2 | 9216 | 1152 | ≥ 15 |
|  | 3-20 | 9 | 2 | 7 | 12 | 256QAM | 3/4 | 7736 | 24 | 2 | 10368 | 1296 | ≥ 15 |
|  | 3-20 | 10 | 2 | 7 | 12 | 256QAM | 3/4 | 8504 | 24 | 2 | 11520 | 1440 | ≥ 15 |
|  | 3-20 | 12 | 2 | 7 | 12 | 256QAM | 3/4 | 10296 | 24 | 2 | 13824 | 1728 | ≥ 15 |
|  | 5-20 | 15 | 2 | 7 | 12 | 256QAM | 3/4 | 12960 | 24 | 3 | 17280 | 2160 | ≥ 15 |
|  | 5-20 | 16 | 2 | 7 | 12 | 256QAM | 3/4 | 13536 | 24 | 3 | 18432 | 2304 | ≥ 15 |
|  | 5-20 | 18 | 2 | 7 | 12 | 256QAM | 3/4 | 15264 | 24 | 3 | 20736 | 2592 | ≥ 15 |
|  | 5-20 | 20 | 2 | 7 | 12 | 256QAM | 3/4 | 16992 | 24 | 3 | 23040 | 2880 | ≥ 15 |
|  | 5-20 | 24 | 2 | 7 | 12 | 256QAM | 3/4 | 20616 | 24 | 4 | 27648 | 3456 | ≥ 15 |
|  | 10-20 | 25 | 2 | 7 | 12 | 256QAM | 3/4 | 21384 | 24 | 4 | 28800 | 3600 | ≥ 15 |
|  | 10-20 | 27 | 2 | 7 | 12 | 256QAM | 3/4 | 22920 | 24 | 4 | 31104 | 3888 | ≥ 15 |
|  | 10-20 | 30 | 2 | 7 | 12 | 256QAM | 3/4 | 25456 | 24 | 5 | 34560 | 4320 | ≥ 15 |
|  | 10-20 | 32 | 2 | 7 | 12 | 256QAM | 3/4 | 27376 | 24 | 5 | 36864 | 4608 | ≥ 15 |
|  | 10-20 | 36 | 2 | 7 | 12 | 256QAM | 3/4 | 30576 | 24 | 6 | 41472 | 5184 | ≥ 15 |
|  | 10-20 | 40 | 2 | 7 | 12 | 256QAM | 3/4 | 34008 | 24 | 6 | 46080 | 5760 | ≥ 15 |
|  | 10-20 | 45 | 2 | 7 | 12 | 256QAM | 3/4 | 37888 | 24 | 7 | 51840 | 6480 | ≥ 15 |
|  | 10-20 | 48 | 2 | 7 | 12 | 256QAM | 3/4 | 40576 | 24 | 8 | 55296 | 6912 | ≥ 15 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | 256QAM | 3/4 | 42368 | 24 | 8 | 57600 | 7200 | ≥ 15 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | 256QAM | 3/4 | 46888 | 24 | 8 | 62208 | 7776 | ≥ 15 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | 256QAM | 3/4 | 51024 | 24 | 9 | 69120 | 8640 | ≥ 15 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | 256QAM | 3/4 | 55056 | 24 | 9 | 73728 | 9216 | ≥ 15 |
|  | 15 - 20 | 72 | 2 | 7 | 12 | 256QAM | 3/4 | 61664 | 24 | 11 | 82944 | 10368 | ≥ 15 |
|  | 20 | 75 | 2 | 7 | 12 | 256QAM | 3/4 | 63776 | 24 | 11 | 86400 | 10800 | ≥ 15 |
|  | 20 | 80 | 2 | 7 | 12 | 256QAM | 3/4 | 68808 | 24 | 12 | 92160 | 11520 | ≥ 15 |
|  | 20 | 81 | 2 | 7 | 12 | 256QAM | 3/4 | 68808 | 24 | 12 | 93312 | 11664 | ≥ 15 |
|  | 20 | 90 | 2 | 7 | 12 | 256QAM | 3/4 | 76208 | 24 | 13 | 103680 | 12960 | ≥ 15 |
|  | 20 | 96 | 2 | 7 | 12 | 256QAM | 3/4 | 81176 | 24 | 14 | 110592 | 13824 | ≥ 15 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [13] Note 3: As per Table 4.2-1 in TS 36.211 [13] |  |  |  |  |  |  |  |  |  |  |  |  |  |

# A.3	DL reference measurement channels for E-UTRA

## A.3.1	General

The number of available channel bits varies across the sub-frames due to PBCH and PSS/SSS overhead. The payload size per sub-frame is varied in order to keep the code rate constant throughout a frame.

Unless otherwise stated, no user data is scheduled on subframes #5 in order to facilitate the transmission of system information blocks (SIB).

The algorithm for determining the payload size A is as follows; given a desired coding rate R and radio block allocation NRB

1.	Calculate the number of channel bits Nch that can be transmitted during the first transmission of a given sub-frame.

2.	Find A such that the resulting coding rate is as close to R as possible, that is,

,

subject to

a)	A is a valid TB size according to clause 7.1.7 of TS 36.213 [6] assuming an allocation of NRB resource blocks.

b)	C is the number of Code Blocks calculated according to clause 5.1.2 of TS 36.212 [5].

3.	If there is more than one A that minimizes the equation above, then the larger value is chosen per default and the chosen code rate should not exceed 0.93.

4.	For TDD, the measurement channel is based on DL/UL configuration ratio of 3DL+DwPTS (10 OFDM symbol SSF7): 1UL.

Table A.3.1-1: PUCCH-ConfigDedicated-v1020-DEFAULT: Additional Configuration for LTE TDD 1CC

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.6.3-9A: PUCCH-ConfigDedicated-v1020-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigDedicated-v1020 ::= SEQUENCE { |  |  |  |
| pucch-Format-r10 CHOICE { |  |  | UL_RB_allocation=0 |
| channelSelection-r10 SEQUENCE { |  |  |  |
| n1PUCCH-AN-CS-r10 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| n1PUCCH-AN-CS-List-r10 SEQUENCE{ | 2 |  |  |
| N1PUCCH-AN-CS-r10 | 4 |  |  |
|  | 101 |  |  |
|  | 102 |  |  |
|  | 103 |  |  |
|  | 104 |  |  |
| N1PUCCH-AN-CS-r10 | 4 |  |  |
|  | 105 |  |  |
|  | 106 |  |  |
|  | 107 |  |  |
|  | 108 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL_RB_allocation=0 | TC is tested with no or N/A UL RB allocation (no PUSCH) |

Table A.3.1-2: RadioResourceConfigCommonSIB-DEFAULT: Additional Configuration for LTE TDD 1CC

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.6.3-14: RadioResourceConfigCommonSIB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-DEFAULT ::= SEQUENCE { |  |  |  |
| uplinkPowerControlCommon-v1020 | UplinkPowerControlCommon-v1020-DEFAULT: Additional Configuration for LTE TDD 1CC | Table A.3.1-3 | UL_RB_allocation=0 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL_RB_allocation=0 | TC is tested with no or N/A UL RB allocation (no PUSCH) |

Table A.3.1-3: UplinkPowerControlCommon-v1020-DEFAULT: Additional Configuration for LTE TDD 1CC

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.6.3-25AA: UplinkPowerControlCommon-v1020-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-v1020-DEFAULT ::= SEQUENCE { |  |  | UL_RB_allocation=0 |
| deltaF-PUCCH-Format1bCS-r10 | deltaF1 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL_RB_allocation=0 | TC is tested with no or N/A UL RB allocation (no PUSCH) |

### A.3.1.1	QPSK

Table A.3.1.1-1: Fixed Reference Channel for Receiver Requirements (TDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel Bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (NOTE 5) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 6) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| Allocated subframes per Radio Frame (D+S) |  | 3 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 |
| Number of HARQ Processes | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Maximum number of HARQ transmission |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload per Sub-Frame | Bits |  |  |  |  |  |  |
| For Sub-Frame 3, 4, 8, 9 |  | 408 | 1320 | 2216 | 4392 | 6712 | 8760 |
| For Sub-Frame 1, 6 |  | N/A | 776 | 1288 | 2664 | 4008 | 5352 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 208 | 1064 | 1800 | 4392 | 6712 | 8760 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame (NOTE 4) |  |  |  |  |  |  |  |
| For Sub-Frame 3, 4, 8, 9 |  | 1 | 1 | 1 | 1 | 2 | 2 |
| For Sub-Frame 1, 6 |  | N/A | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 1 | 1 | 1 | 1 | 2 | 2 |
| Binary Channel Bits Per Sub-Frame | Bits |  |  |  |  |  |  |
| For Sub-Frame 3, 4, 8, 9 |  | 1368 | 3780 | 6300 | 13800 | 20700 | 27600 |
| For Sub-Frame 1, 6 |  | N/A | 2616 | 4456 | 9056 | 13656 | 18256 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 672 | 3084 | 5604 | 13104 | 20004 | 26904 |
| Max. Throughput averaged over 1 frame | kbps | 102.4 | 564 | 932 | 1965.6 | 3007.2 | 3970.4 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 |
| NOTE 1: For normal subframes(0,3,4,5,8,9), 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For special subframe (1&6), only 2 OFDM symbols are allocated to PDCCH for all BWs. NOTE 2: For 1.4MHz, no data shall be scheduled on special subframes(1&6) to avoid problems with insufficient PDCCH performance NOTE 3: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [7] NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: As per Table 4.2-2 in TS 36.211 [7] NOTE 6: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

### A.3.1.2	64-QAM

Table A.3.1.2-1: Fixed Reference Channel for Maximum input level for UE Categories ≥ 3 (TDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Uplink-Downlink Configuration (NOTE 5) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 6) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| Allocated subframes per Radio Frame |  | 2 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | ¾ | ¾ | ¾ | ¾ | ¾ | ¾ |
| Number of HARQ Processes | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3, 4, 8, 9 | Bits | 2984 | 8504 | 14112 | 30576 | 46888 | 61664 |
| For Sub-Frames 1,6 | Bits | N/A | 5544 | 9528 | 19848 | 30576 | 40576 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 6968 | 12576 | 30576 | 45352 | 61664 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame (NOTE 4) |  |  |  |  |  |  |  |
| For Sub-Frames 3, 4, 8, 9 |  | 1 | 2 | 3 | 5 | 8 | 11 |
| For Sub-Frames 1,6 |  | N/A | 2 | 2 | 4 | 6 | 8 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | N/A | 2 | 3 | 5 | 8 | 11 |
| Binary Channel Bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3, 4, 8, 9 | Bits | 4104 | 11340 | 18900 | 41400 | 62100 | 82800 |
| For Sub-Frames 1,6 |  | N/A | 7848 | 13368 | 27168 | 40968 | 54768 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 9252 | 16812 | 39312 | 60012 | 80712 |
| Max. Throughput averaged over 1 frame | kbps | 596.8 | 3791.2 | 6369.6 | 13910 | 20945 | 27877 |
| NOTE 1: For normal subframes(0,3,4,5,8,9), 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For special subframe (1&6), only 2 OFDM symbols are allocated to PDCCH for all BWs. NOTE 2: For 1.4MHz, no data shall be scheduled on special subframes(1&6) to avoid problems with insufficient PDCCH performance. NOTE 3: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [7]. NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: As per Table 4.2-2 in TS 36.211 [7]. NOTE 6: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

### A.3.1.3	256-QAM

Table A.3.1.3-1: Fixed Reference Channel for Maximum input level for UE Categories 11/12 and UE DL categories ≥ 11 (TDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Uplink-Downlink Configuration (NOTE 5) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 6) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| Allocated subframes per Radio Frame |  | 2 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Number of HARQ Processes | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 4392 | 12216 | 19848 | 42368 | 63776 | 84760 |
| For Sub-Frames 1,6 | Bits | N/A | 10464 | 17824 | 36224 | 54624 | 73024 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 9912 | 17568 | 42368 | 63776 | 84760 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame (NOTE 4) |  |  |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 |  | 1 | 2 | 4 | 7 | 11 | 14 |
| For Sub-Frames 1,6 |  | N/A | 2 | 3 | 6 | 9 | 13 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | N/A | 2 | 3 | 7 | 11 | 14 |
| Binary Channel Bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 5472 | 15120 | 25200 | 55200 | 82800 | 110400 |
| For Sub-Frames 1,6 |  | N/A | 8248 | 13536 | 27376 | 40576 | 55056 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 12336 | 22416 | 52416 | 80016 | 107616 |
| Max. Throughput averaged over 1 frame | kbps | 878.4 | 5570.4 | 9240 | 20049.6 | 30144 | 40503.2 |
| NOTE 1: For normal subframes(0,3,4,5,8,9), 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For special subframe (1&6), only 2 OFDM symbols are allocated to PDCCH for all BWs. NOTE 2: For 1.4MHz, no data shall be scheduled on special subframes(1&6) to avoid problems with insufficient PDCCH performance. NOTE 3: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [7]. NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: As per Table 4.2-2 in TS 36.211 [7]. NOTE 6: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

Annex B (normative): Propagation Conditions

Please refer to Annex B in TS 38.521-1 [8] and 38.521-2 [9] for appropriate details as needed for test cases in this test specification. EN-DC exceptions will be added,

Annex C (normative): Downlink Physical Channels

Please refer to Annex C in TS 38.521-1 [8] and 38.521-2 [9] for appropriate details as needed for test cases in this test specification. EN-DC exceptions will be added,

Annex D (normative): Characteristics of the Interfering Signal

Please refer to Annex D in TS 38.521-1 [8] and 38.521-2 [9] for appropriate details as needed for test cases in this test specification. EN-DC exceptions will be added,

Annex E (normative): Global In-Channel Tx Test

Please refer to Annex E in TS 38.521-1 [8] and 38.521-2 [9] for appropriate details as needed for test cases in this test specification. EN-DC exceptions will be added,

Annex F (informative): Measurement uncertainties and Test Tolerances

# F.1	Acceptable uncertainty of Test System (normative)

## F.1.0	General

### F.1.0.1	Conducted Measurements

Same as F.1.0 in TS 38.521-1 [8].

### F.1.0.2	Radiated Measurements

Same as F.1.0 in TS 38.521-2 [9].

## F.1.1	Measurement of test environments

### F.1.1.1	Conducted Measurements

Same as F.1.1 in TS 38.521-1 [8].

### F.1.1.2	Radiated Measurements

Same as F.1.1 in TS 38.521-2 [9].

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Clause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC |  | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |  |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC |  | MAX (MULTE, MUSA) MULTE ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz MUSA f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  | MULTE is MU of LTE specified in clause 6.2.2 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.1 in TS 38.521-1 [8]. |  |
| 6.2B.1.3 UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | MAX (MULTE, MUSA) MULTE ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz MUSA f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  | MULTE is MU of LTE specified in clause 6.2.2 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.1 in TS 38.521-1 [8]. |  |
| 6.2B.1.3_1 UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TRP |  | Same as clause 6.2.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage |  | Same as clause 6.2.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.1.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.1.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - Spherical Coverage |  | Same as clause 6.2A.1.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.2.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.2.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) -Spherical Coverage |  | Same as clause 6.2A.1.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.3.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.3.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) –Spherical Coverage |  | Same as clause 6.2A.1.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.4 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) –Spherical Coverage |  | Same as clause 6.2A.1.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.5.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.5 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.5.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) –Spherical Coverage |  | Same as clause 6.2A.1.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.6.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.6 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.6.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) –Spherical Coverage |  | Same as clause 6.2A.1.2.6 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4D.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO |  | Same as clause 6.2D.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4D.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO- Spherical coverage |  | Same as clause 6.2D.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.1 UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC |  | Same as clause 6.2B.1.1 |  |  |  |
| 6.2B.2.2 UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  | Same as clause 6.2B.1.2 |  |  |  |
| 6.2B.2.3 UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 |  | Same as clause 6.2B.1.3 |  |  |  |
| 6.2B.2.4 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 |  | Same as clause 6.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4a UE Maximum Output Power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.2_1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4_1.1 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.2A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4_1.2 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.2A.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4_1.3 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.2A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.3.1 UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC |  | Same as clause 6.2B.1.1 |  |  |  |
| 6.2B.3.2 UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  | Same as clause 6.2B.1.2 |  |  |  |
| 6.2B.3.3 UE Additional Maximum Output Power reduction for Inter-Band EN-DC within FR1 |  | Same as clause 6.2B.1.3 |  |  |  |
| 6.2B.3.4 UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.4.1.1 Configured Output Power Level for Intra-Band Contiguous EN-DC |  | Same as clause 6.2B.1.1 |  |  |  |
| 6.2B.4.1.2 Configured Output Power for Intra-Band Non-Contiguous EN-DC |  | Same as clause 6.2B.1.2 |  |  |  |
| 6.2B.4.1.3 Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | Same as clause 6.2B.1.3 |  |  |  |
| 6.2B.4.1.3_1 Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.4.1.4_1 Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.4_1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.1 Minimum Output Power for intra-band contiguous EN-DC |  | Same as clause 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.2 Minimum output power for intra-band non-contiguous EN-DC |  | Same as clause 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.3 Minimum output power for inter-band EN-DC within FR1 |  | Same as clause 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.4 Minimum Output Power for EN-DC Interband including FR2 |  | Same as clause 6.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.1 Minimum output power for inter-band EN-DC including FR2 (3 CCs) |  | Same as 6.3A.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.2 Minimum output power for inter-band EN-DC including FR2 (4 CCs) |  | Same as 6.3A.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.3 Minimum output power for inter-band EN-DC including FR2 (5 CCs) |  | Same as 6.3A.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4D Minimum output power for inter-band EN-DC including FR2 for UL MIMO |  | Same as 6.3D.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.1 Transmit OFF Power for intra-band contiguous EN-DC |  | Same as clause 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.2 Transmit OFF Power for intra-band non-contiguous EN-DC |  | Same as clause 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.3 Transmit OFF Power for inter-band EN-DC within FR1 |  | Same as clause 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.4 Transmit OFF Power for inter-band EN-DC including FR2 |  | Same as clause 6.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4D Transmit OFF Power for inter-band EN-DC including FR2 for UL-MIMO |  | Same as 6.3D.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.1 Tx ON/OFF time mask for intra-band contiguous EN-DC |  | Same as clause 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3_1.1 E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC |  | MAX (MULTE, MUNR) MULTE is MU of LTE specified in clause 6.2.3 in TS 36.521-1 [10]. MUNR is MU of FR1 NR specified in clause 6.2D.2 in TS 38.521-1 [8]. |  |  |  |
| 6.3B.2.4_1.1 Transmit OFF Power for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 6.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4_1.2 Transmit OFF Power for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 6.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4_1.3 Transmit OFF Power for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 6.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.2 Tx ON/OFF time mask for intra-band non-contiguous EN-DC |  | Same as clause 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3.3 Tx ON/OFF time mask for inter-band EN-DC within FR1 |  | Same as clause 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3.4 Transmit ON/OFF time mask for inter-band EN-DC including FR2 |  | Same as clause 6.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.4_1.1 Transmit ON/OFF time mask for inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.3A.3.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.4.3 PRACH Time Mask for inter-band EN-DC within FR1 |  | Same as clause 6.3.3.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.1 Absolute power tolerance for intra-band contiguous EN-DC |  | Same as clause 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.2 Absolute power tolerance for intra-band non-contiguous EN-DC |  | Same as clause 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.3 Absolute power tolerance for inter-band EN-DC within FR1 |  | Same as clause 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.4 Absolute power tolerance for inter-band EN-DC including FR2 |  | Same as clause 6.3.4.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.8.2.1 Relative power tolerance for intra-band contiguous EN-DC |  | Same as clause 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.2 Relative power tolerance for intra-band non-contiguous EN-DC |  | Same as clause 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.3 Relative power tolerance for inter-band EN-DC within FR1 |  | Same as clause 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.4 Relative power tolerance for inter-band EN-DC including FR2 |  | Same as clause 6.3.4.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.8.3.1 Aggregate power tolerance for intra-band contiguous EN-DC |  | Same as clause 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.2 Aggregate power tolerance for intra-band non-contiguous EN-DC |  | Same as clause 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.3 Aggregate power tolerance for inter-band EN-DC within FR1 |  | Same as clause 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.4 Aggregate power tolerance for inter-band EN-DC including FR2 |  | Same as clause 6.3.4.4 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.1 Frequency Error for intra-band contiguous EN-DC |  | TBD |  |  |  |
| 6.4B.1.2 Frequency Error for intra-band non-contiguous EN-DC |  | TBD |  |  |  |
| 6.4B.1.3 Frequency Error for inter-band EN-DC within FR1 |  | Same as clause 6.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.1.4 Frequency Error for inter-band EN-DC including FR2 |  | Same as clause 6.4.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.1 Frequency Error for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 6.4A.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.2 Frequency Error for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 6.4A.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.3 Frequency Error for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 6.4A.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.1.1 Error Vector Magnitude for intra-band contiguous EN-DC |  | Same as clause 6.4.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.3B.1.1. |  |  |  |
| 6.4B.2.1.2 Carrier Leakage for intra-band contiguous EN-DC |  | Same as clause 6.4.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.1.3 In-band Emissions for intra-band contiguous EN-DC |  | Same as clause 6.4.2.3 in TS 38.521-1 [8] for ENBW ≤100MHz. Uplink power measurement for steps 2 and 8 same as 6.2B.1.1. Uplink power measurement for steps 4, 6, 10, and 12 same as 6.3B.1.1. |  |  |  |
| 6.4B.2.1.4 EVM Equalizer Flatness for intra-band contiguous EN-DC |  | Same as clause 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.1 Error Vector Magnitude for intra-band non-contiguous EN-DC |  | Same as clause 6.4.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.3B.1.2. |  |  |  |
| 6.4B.2.2.2 Carrier Leakage for intra-band non-contiguous EN-DC |  | Same as clause 6.4.2.2 in TS 38.521-1 [8] Uplink power measurement for step 2 and step 4 same as 6.2B.1.2. Uplink power measurement for step 6 and step 8 same as 6.3B.1.2. |  |  |  |
| 6.4B.2.2.3 In-band Emissions for intra-band non-contiguous EN-DC |  | Same as clause 6.4.2.3 in TS 38.521-1 [8] Uplink power measurement for steps 2 and 8 same as 6.2B.1.2. Uplink power measurement for steps 4, 6, 10, and 12 same as 6.3B.1.2. |  |  |  |
| 6.4B.2.2.4 EVM Equalizer Flatness for intra-band non-contiguous EN-DC |  | Same as clause 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.1 Error Vector Magnitude for inter-band EN-DC within FR1 |  | Same as clause 6.4.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.3B.1.3. |  |  |  |
| 6.4B.2.3.2 Carrier Leakage for inter-band EN-DC within FR1 |  | Same as clause 6.4.2.2 in TS 38.521-1 [8] Uplink power measurement for step 2 and step 4 same as 6.2B.1.3. Uplink power measurement for step 6 and step 8 same as 6.3B.1.3. |  |  |  |
| 6.4B.2.3.3 In-band Emissions for inter-band EN-DC within FR1 |  | Same as clause 6.4.2.3 in TS 38.521-1 [8] Uplink power measurement for steps 1.2, 1.4, 2.2, and 2.4 same as 6.2B.1.3. Uplink power measurement for steps 1.6, 1.8, 2.6, and 2.8 same as 6.3B.1.3. |  |  |  |
| 6.4B.2.3.4 EVM Equalizer Flatness for inter-band EN-DC within FR1 |  | Same as clause 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.4.1 Error Vector Magnitude for inter-band EN-DC including FR2 |  | Same as clause 6.4.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.1a Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.4.2.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.2 Carrier Leakage for inter-band EN-DC including FR2 |  | Same as clause 6.4.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.3 In-band Emissions for inter-band EN-DC including FR2 |  | Same as clause 6.4.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.4 EVM Equalizer Flatness for inter-band EN-DC including FR2 |  | Same as clause 6.4.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.5 EVM spectral flatness for pi/2 BPSK modulation for inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.4.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC |  | 1.5% of aggregated channel bandwidth |  |  |  |
| 6.5B.1.2 Occupied bandwidth for Intra-Band Non-Contiguous EN-DC |  | Same as clause 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.3 Occupied bandwidth for Inter-Band EN-DC within FR1 |  | Same as clause 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.4 Occupied bandwidth for Inter-Band EN-DC including FR2 |  | Same as clause 6.5.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.1.4_1.1 Occupied bandwidth for Inter-band EN-DC including FR2 (3 CCs) |  | TBD |  |  |  |
| 6.5B.1.4_1.2 Occupied bandwidth for Inter-band EN-DC including FR2 (4 CCs) |  | TBD |  |  |  |
| 6.5B.1.4_1.3 Occupied bandwidth for Inter-band EN-DC including FR2 (5 CCs) |  | TBD |  |  |  |
| 6.5B.1.4D Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO |  | Same as clause 6.5D.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.1.1 Spectrum emissions mask for intra-band contiguous EN-DC |  | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.1.2 Additional spectrum emissions mask for intra-band contiguous EN-DC |  | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.1.3 Adjacent channel leakage ratio for intra-band contiguous EN-DC |  | Same as clause 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.1 Spectrum emissions mask for intra-band non-contiguous EN-DC |  | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.2 Additional Spectrum emissions mask for intra-band non-contiguous EN-DC |  | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.3 Adjacent channel leakage ratio for intra-band non-contiguous EN-DC |  | TBD |  |  |  |
| 6.5B.2.3.1 Spectrum emissions mask for Inter-band EN-DC within FR1 |  | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.2 Additional Spectrum emissions mask for Inter-band EN-DC within FR1 |  | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.3.1NR - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) |  | Same as clause 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.3.2 UTRA - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) |  | Same as clause 6.5.2.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.4.1 Spectrum emissions mask for Inter-band EN-DC including FR2 |  | Same as clause 6.5.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1a Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.5.2.1.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.1 Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.2.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.2 Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.2.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.3 Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 |  | Same as clause 6.5.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.1 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.2.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.2 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.2.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.2.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.4 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 NR CCs) |  | Same as clause 6.5A.2.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.5 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (6 NR CCs) |  | Same as clause 6.5A.2.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.6 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (7 NR CCs) |  | Same as clause 6.5A.2.2.6 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.7 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (8 NR CCs) |  | Same as clause 6.5A.2.2.7 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.4.1 General Spurious Emissions for Inter-band including FR2 (2 CCs) |  | Same as clause 6.5.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.1 General Spurious Emissions for Inter-band including FR2 (2 NR CCs) |  | Same as clause 6.5A.3.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.2 General Spurious Emissions for Inter-band including FR2 (3 NR CCs) |  | Same as clause 6.5A.3.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.3 General Spurious Emissions for Inter-band including FR2 (4 NR CCs) |  | Same as clause 6.5A.3.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.4 General Spurious Emissions for Inter-band including FR2 (5 NR CCs) |  | Same as clause 6.5A.3.1.4 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.5 General Spurious Emissions for Inter-band including FR2 (6 NR CCs) |  | Same as clause 6.5A.3.1.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.6 General Spurious Emissions for Inter-band including FR2 (7 NR CCs) |  | Same as clause 6.5A.3.1.6 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.7 General Spurious Emissions for Inter-band including FR2 (8 NR CCs) |  | Same as clause 6.5A.3.1.7 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2 Spurious emission band UE co-existence for Inter-band including FR2 |  | Same as clause 6.5.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.2_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.1 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.3.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.2 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.3.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.3 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.3.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.4 Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.3_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.1 Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) |  | Same as clause 6.5A.3.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) |  | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) |  | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 |  | Same as clause 6.5.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP |  | Same as clause 6.6.1 in TS 38.521-2 [9] |  |  |  |
| 6.6B.5 Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP |  | Same as clause 6.6.2 in TS 38.521-2 [9] |  |  |  |

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Clause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3_1.1 Reference sensitivity for Inter-band EN-DC within FR1 (3 CCs) |  | Same as clause 7.3A.1 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.4 Reference sensitivity for Inter-band EN-DC including FR2 |  | Same as clause 7.3.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.1 Reference sensitivity for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 7.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.2 Reference sensitivity for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 7.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.3 Reference sensitivity for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 7.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.4 Reference sensitivity for Inter-band EN-DC including FR2 (6 CCs) |  | Same as clause 7.3A.2.4 in TS 38.521-2 [9] |  |  |  |
| 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 |  | Same as clause 7.3.4 in TS 38.521-2 [9] |  |  |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) |  | MU for NR CC downlink power same as clause 7.4 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.4B.2 Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) |  | Same as clause 7.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.4B.3 Maximum Input Level for Inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.4B.3_1.1 Maximum Input Level for Inter-band EN-DC within FR1 (3 CCs) |  | Same as clause 7.4A.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.1 Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] for NR CC Same as clause 7.5 in TS 36.521 [10] Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.5B.2 Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.5B.3 Adjacent Channel Selectivity for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.1 Adjacent Channel Selectivity for EN-DC within FR1 (3 CCs) |  | Same as clause 7.5A in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.2 Adjacent Channel Selectivity for EN-DC within FR1 (4 CCs) |  | Same as clause 7.5A in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.3 Adjacent Channel Selectivity for EN-DC within FR1 (5 CCs) |  | Same as clause 7.5A in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.4 Adjacent Channel Selectivity for EN-DC within FR1 (6 CCs) |  | Same as clause 7.5A in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2CCs) |  | Same as clause 7.5 in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.1 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.2 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.3 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (6 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.1 Inband blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.2 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.2.2 Inband blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.2.3 Inband blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.1 Inband blocking for EN-DC within FR1 (3 CCs) |  | Same as clause 7.6A.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.2 Inband blocking for EN-DC within FR1 (4 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.3 Inband blocking for EN-DC within FR1 (5 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.4 Inband blocking for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.6.2 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.4_1.1 Inband blocking for inter-band EN-DC including FR2 (3 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.2 Inband blocking for inter-band EN-DC including FR2 (4 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.3 Inband blocking for inter-band EN-DC including FR2 (5 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.4 Inband blocking for inter-band EN-DC including FR2 (6 CCs) |  | TBD |  |  |  |
| 7.6B.3.1 Out-of-band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.3 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.3.2 Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.3 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.3.3 Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.3 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 (3 CCs) |  | Same as clause 7.6A.3.1 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.1 Narrow band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.4 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.4.2 Narrow band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.4.3 Narrow band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.1 Narrow band blocking for EN-DC within FR1 (3 CCs) |  | Same as clause 7.6A.4.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.2 Narrow band blocking for EN-DC within FR1 (4 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.3 Narrow band blocking for EN-DC within FR1 (5 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.7B.1 Spurious Response for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.7 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.7B.2 Spurious Response for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.7B.3 Spurious Response for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.7B.3_1.1 Spurious Response for EN-DC within FR1 (3 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.7B.3_1.2 Spurious Response for EN-DC within FR1 (4 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.8B.2.1 Wideband Intermodulation for intra-band contiguous EN-DC in FR1 |  | MU for NR CC downlink power same as clause 7.8.2 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.8B.2.2 Wideband Intermodulation for intra-band non-contiguous EN-DC in FR1 |  | Same as clause 7.8.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.2. |  |  |  |
| 7.8B.2.3 Wideband Intermodulation for inter-band EN-DC in FR1 |  | Same as clause 7.8.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.3_1.1 Wideband Intermodulation for EN-DC within FR1 (3 CCs) |  | Same as clause 7.8A.2.1 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.3_1.2 Wideband Intermodulation for EN-DC within FR1 (4 CCs) |  | Same as clause 7.8A.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.3_1.3 Wideband Intermodulation for EN-DC within FR1 (5 CCs) |  | Same as clause 7.8A.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.3_1.4 Wideband Intermodulation for EN-DC within FR1 (6 CCs) |  | Same as clause 7.8A.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.9B.1 Spurious Emissions for intra-band contiguous EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.2 Spurious Emissions for intra-band non-contiguous EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3 Spurious Emissions for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3_1.1 Spurious Emissions for EN-DC within FR1 (3 CCs) |  | Same as clause 7.9A.1 in TS 38.521-1 [8] |  |  |  |
| 7.9B.4 Spurious Emissions for inter-band EN-DC including FR2 (2 CCs) |  | Same as clause 7.9 in TS 38.521-2 [9] |  |  |  |

# F.2	Interpretation of measurement results (normative)

TBD

# F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

## F.3.1	Measurement of test environments

TBD

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC |  | Same as 6.2.1 in TS 38.521-1 [8] |  |  |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC |  | MAX (TTLTE, TTSA) TTLTE 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 4.2GHz TTSA f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz |  | TTLTE is TT of LTE specified in 6.2.2 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.1 in TS 38.521-1 [8]. |  |
| 6.2B.1.3 UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | MAX (TTLTE, TTSA) TTLTE 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 4.2GHz TTSA f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz |  | TTLTE is TT of LTE specified in 6.2.3 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.2 in TS 38.521-1 [8]. |  |
| 6.2B.1.3_1 UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.1.4 UE Maximum Output Power for Inter-Band EN-DC including FR2 |  | Same as 6.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TR |  | Same as 6.2.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage |  | Same as 6.2.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.1.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - EIRP and TRP |  | Same as 6.2A.1.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.1.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - Spherical Coverage |  | Same as 6.2A.1.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.2.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.2.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - Spherical Coverage |  | Same as clause 6.2A.1.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.3.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.3.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) – Spherical Coverage |  | Same as clause 6.2A.1.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.4 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) – Spherical Coverage |  | Same as clause 6.2A.1.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.5.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.5 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.5.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) – Spherical Coverage |  | Same as clause 6.2A.1.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.6.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.6 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4_1.6.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) – Spherical Coverage |  | Same as clause 6.2A.1.2.6 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4D.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO |  | Same as clause 6.2D.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4D.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO- Spherical coverage |  | Same as clause 6.2D.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.1 UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC |  | Same as 6.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.2B.2.2 UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  | Same as 6.2B.1.2 |  |  |  |
| 6.2B.2.3 UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.2.4 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 |  | Same as clause 6.2.2 and 6.2A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4a UE Maximum Output Power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.2_1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4_1.1 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.2A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4_1.2 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.2A.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4_1.3 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.2A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.3.1 UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC |  | Same as 6.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.2B.3.2 UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  | Same as 6.2B.1.2 |  |  |  |
| 6.2B.3.3 UE Additional Maximum Output Power reduction for Inter-Band EN-DC within FR1 |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.3.4 UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.4.1.1 Configured Output Power Level for Intra-Band Contiguous EN-DC |  | Same as 6.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.2B.4.1.2 Configured Output Power for Intra-Band Non-Contiguous EN-DC |  | Same as 6.2B.1.2 |  |  |  |
| 6.2B.4.1.3 Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.4.1.3_1 Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.4.1.4_1 Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.4_1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.1 Minimum Output Power for intra-band contiguous EN-DC |  | Same as 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.2 Minimum output power for intra-band non-contiguous EN-DC |  | Same as 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.3 Minimum output power for inter-band EN-DC within FR1 |  | Same as 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.4 Minimum Output Power for EN-DC Interband including FR2 |  | Same as 6.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.1 Minimum output power for inter-band EN-DC including FR2 (3 CCs) |  | Same as 6.3A.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.2 Minimum output power for inter-band EN-DC including FR2 (4 CCs) |  | Same as 6.3A.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.3 Minimum output power for inter-band EN-DC including FR2 (5 CCs) |  | Same as 6.3A.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4D Minimum output power for inter-band EN-DC including FR2 for UL MIMO |  | Same as 6.3D.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.1 Transmit OFF Power for intra-band contiguous EN-DC |  | Same as 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.2 Transmit OFF Power for intra-band non-contiguous EN-DC |  | Same as 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.3 Transmit OFF Power for inter-band EN-DC within FR1 |  | Same as 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.4 Transmit OFF Power for inter-band EN-DC including FR2 |  | Same as 6.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4D Transmit OFF Power for inter-band EN-DC including FR2 for UL-MIMO |  | Same as 6.3D.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.1 Tx ON/OFF time mask for intra-band contiguous EN-DC |  | Same as 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3_1.1 E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC |  | MAX (TTLTE, TTNR) TTLTE is TT of LTE specified in clause 6.2.3 in TS 36.521-1 [10]. TTNR is TT of FR1 NR specified in clause 6.2D.2 in TS 38.521-1 [8]. |  |  |  |
| 6.3B.2.4_1.1 Transmit OFF Power for Inter-band EN-DC including FR2 (3 CCs) |  | Same as 6.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4_1.2 Transmit OFF Power for Inter-band EN-DC including FR2 (4 CCs) |  | Same as 6.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4_1.3 Transmit OFF Power for Inter-band EN-DC including FR2 (5 CCs) |  | Same as 6.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.2 Tx ON/OFF time mask for intra-band non-contiguous EN-DC |  | Same as 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3.3 Tx ON/OFF time mask for inter-band EN-DC within FR1 |  | Same as 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3.4 Transmit ON/OFF time mask for inter-band EN-DC including FR2 |  | Same as 6.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.4_1.1 Transmit ON/OFF time mask for inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.3A.3.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.4.3 PRACH Time Mask for inter-band EN-DC within FR1 |  | Same as 6.3.3.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.1 Absolute power tolerance for intra-band contiguous EN-DC |  | Same as 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.2 Absolute power tolerance for intra-band non-contiguous EN-DC |  | Same as 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.3 Absolute power tolerance for inter-band EN-DC within FR1 |  | Same as 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.4 Absolute power tolerance for inter-band EN-DC including FR2 |  | Same as 6.3.4.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.8.2.1 Relative power tolerance for intra-band contiguous EN-DC |  | Same as 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.2 Relative power tolerance for intra-band non-contiguous EN-DC |  | Same as 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.3 Relative power tolerance for inter-band EN-DC within FR1 |  | Same as 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.4 Relative power tolerance for inter-band EN-DC including FR2 |  | Same as 6.3.4.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.8.3.1 Aggregate power tolerance for intra-band contiguous EN-DC |  | Same as 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.2 Aggregate power tolerance for intra-band non-contiguous EN-DC |  | Same as 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.3 Aggregate power tolerance for inter-band EN-DC within FR1 |  | Same as 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.4 Aggregate power tolerance for inter-band EN-DC including FR2 |  | Same as 6.3.4.4 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.1 Frequency Error for intra-band contiguous EN-DC |  | Same as 6.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.1.2 Frequency Error for intra-band non-contiguous EN-DC |  | Same as 6.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.1.3 Frequency Error for inter-band EN-DC within FR1 |  | Same as 6.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.1.4 Frequency Error for inter-band EN-DC including FR2 |  | Same as 6.4.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.1 Frequency Error for Inter-band EN-DC including FR2 (3 CCs) |  | Same as 6.4A.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.2 Frequency Error for Inter-band EN-DC including FR2 (4 CCs) |  | Same as 6.4A.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.3 Frequency Error for Inter-band EN-DC including FR2 (5 CCs) |  | Same as 6.4A.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.1.1 Error Vector Magnitude for intra-band contiguous EN-DC |  | Same as 6.4.2.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.1.2 Carrier Leakage for intra-band contiguous EN-DC |  | Same as 6.4.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.1.3 In-band Emissions for intra-band contiguous EN-DC |  | Same as 6.4.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.1.4 EVM Equalizer Flatness for intra-band contiguous EN-DC |  | Same as 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.1 Error Vector Magnitude for intra-band non-contiguous EN-DC |  | Same as 6.4.2.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.2 Carrier Leakage for intra-band non-contiguous EN-DC |  | Same as 6.4.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.3 In-band Emissions for intra-band non-contiguous EN-DC |  | Same as 6.4.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.4 EVM Equalizer Flatness for intra-band non-contiguous EN-DC |  | Same as 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.1 Error Vector Magnitude for inter-band EN-DC within FR1 |  | Same as 6.4.2.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.2 Carrier Leakage for inter-band EN-DC within FR1 |  | Same as 6.4.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.3 In-band Emissions for inter-band EN-DC within FR1 |  | Same as 6.4.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.4 EVM Equalizer Flatness for inter-band EN-DC within FR1 |  | Same as 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.4.1 Error Vector Magnitude for inter-band EN-DC including FR2 |  | Same as 6.4.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.1a Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.4.2.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.2 Carrier Leakage for inter-band EN-DC including FR2 |  | Same as 6.4.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.3 In-band Emissions for inter-band EN-DC including FR2 |  | Same as 6.4.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.4 EVM Equalizer Flatness for inter-band EN-DC including FR2 |  | Same as 6.4.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.5 EVM spectral flatness for pi/2 BPSK modulation for inter-band EN-DC including FR2 (1 NR CC) |  | Same as 6.4.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC |  | Same as 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.2 Occupied bandwidth for Intra-Band Non-Contiguous EN-DC |  | Same as 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.3 Occupied bandwidth for Inter-Band EN-DC within FR1 |  | Same as 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.4 Occupied bandwidth for Inter-Band EN-DC including FR2 |  | Same as 6.5.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.1.4_1.1 Occupied bandwidth for Inter-band EN-DC including FR2 (3 CCs) |  | TBD |  |  |  |
| 6.5B.1.4_1.2 Occupied bandwidth for Inter-band EN-DC including FR2 (4 CCs) |  | TBD |  |  |  |
| 6.5B.1.4_1.3 Occupied bandwidth for Inter-band EN-DC including FR2 (5 CCs) |  | TBD |  |  |  |
| 6.5B.1.4D Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO |  | Same as 6.5D.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.1.1 Spectrum emissions mask for intra-band contiguous EN-DC |  | Same as 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.1.2 Additional spectrum emissions mask for intra-band contiguous EN-DC |  | Same as 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.1.3 Adjacent channel leakage ratio for intra-band contiguous EN-DC |  | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.1 Spectrum emissions mask for intra-band non-contiguous EN-DC |  | Same as 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.2 Additional Spectrum emissions mask for intra-band non-contiguous EN-DC |  | Same as 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.3 Adjacent channel leakage ratio for intra-band non-contiguous EN-DC |  | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.1 Spectrum emissions mask for Inter-band EN-DC within FR1 |  | Same as 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.2 Additional Spectrum emissions mask for Inter-band EN-DC within FR1 |  | Same as 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.3.1 NR - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) |  | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.3.2 UTRA - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) |  | Same as 6.5.2.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.4.1 Spectrum emissions mask for Inter-band EN-DC including FR2 |  | Same as 6.5.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1a Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.5.2.1.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.1 Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.2.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.2 Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.2.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.3 Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 |  | Same as 6.5.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.1 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 6.5A.2.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.2 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 6.5A.2.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 6.5A.2.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.4 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 NR CCs) |  | Same as clause 6.5A.2.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.5 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (6 NR CCs) |  | Same as clause 6.5A.2.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.6 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (7 NR CCs) |  | Same as clause 6.5A.2.2.6 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.7 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (8 NR CCs) |  | Same as clause 6.5A.2.2.7 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC |  | Same as 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC |  | Same as 6.5.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC |  | Same as 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC |  | Same as 6.5.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 |  | Same as 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 |  | Same as 6.5.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.4.1 General Spurious Emissions for Inter-band including FR2 (1 NR CC) |  | Same as 6.5.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.1 General Spurious Emissions for Inter-band including FR2 (2 NR CCs) |  | Same as clause 6.5A.3.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.2 General Spurious Emissions for Inter-band including FR2 (3 NR CCs) |  | Same as clause 6.5A.3.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.3 General Spurious Emissions for Inter-band including FR2 (4 NR CCs) |  | Same as clause 6.5A.3.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.4 General Spurious Emissions for Inter-band including FR2 (5 NR CCs) |  | Same as clause 6.5A.3.1.4 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.5 General Spurious Emissions for Inter-band including FR2 (6 NR CCs) |  | Same as clause 6.5A.3.1.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.6 General Spurious Emissions for Inter-band including FR2 (7 NR CCs) |  | Same as clause 6.5A.3.1.6 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.7 General Spurious Emissions for Inter-band including FR2 (8 NR CCs) |  | Same as clause 6.5A.3.1.7 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2 Spurious emission band UE co-existence for Inter-band including FR2 |  | Same as 6.5.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.2_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.1 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.3.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.2 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.3.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.3 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.3.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.4 Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.3_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.1 Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) |  | Same as clause 6.5A.3.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) |  | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) |  | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 |  | Same as 6.5.4 in TS 38.521-1 [8] |  |  |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP |  | Same as clause 6.6.1 in TS 38.521-2 [9] |  |  |  |
| 6.6B.5 Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP |  | Same as clause 6.6.2 in TS 38.521-2 [9] |  |  |  |

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3_1.1 Reference sensitivity for Inter-band EN-DC within FR1 (3 CCs) |  | Same as 7.3A.1 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.4 Reference sensitivity for Inter-band EN-DC including FR2 |  | Same as 7.3.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.1 Reference sensitivity for Inter-band EN-DC including FR2 (3 CCs) |  | Same as 7.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.2 Reference sensitivity for Inter-band EN-DC including FR2 (4 CCs) |  | Same as 7.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.3 Reference sensitivity for Inter-band EN-DC including FR2 (5 CCs) |  | Same as 7.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.4 Reference sensitivity for Inter-band EN-DC including FR2 (6 CCs) |  | Same as 7.3A.2.4 in TS 38.521-2 [9] |  |  |  |
| 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 |  | Same as 7.3.4 in TS 38.521-2 [9] |  |  |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.2 Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.3 Maximum Input Level for Inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.3_1.1 Maximum Input Level for Inter-band EN-DC within FR1 (3 CCs) |  | Same as 7.4A.1 in TS 38.521-1 [8] |  |  |  |
| 7.5B.1 Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.2 Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3 Adjacent Channel Selectivity for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3_1.1 Adjacent Channel Selectivity for EN-DC within FR1 (3 CCs) |  | Same as 7.5A.1 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3_1.2 Adjacent Channel Selectivity for EN-DC within FR1 (4 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2CCs) |  | Same as clause 7.5 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.1 Inband blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.2 Inband blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.3 Inband blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.3_1.1 Inband blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.3_1.2 Inband blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.3_1.3 Inband blocking for EN-DC within FR1 (5 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.4 Inband blocking for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.6.2 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.4_1.1 Inband blocking for inter-band EN-DC including FR2 (3 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.2 Inband blocking for inter-band EN-DC including FR2 (4 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.3 Inband blocking for inter-band EN-DC including FR2 (5 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.4 Inband blocking for inter-band EN-DC including FR2 (6 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.3.1 Out-of-band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.2 Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3 Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.3.3_1.2 Out-of-band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.1 Narrow band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.2 Narrow band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.3 Narrow band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.3_1.1 Narrow band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.3_1.2 Narrow band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.3_1.3 Narrow band blocking for EN-DC within FR1 (5 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.7B.1 Spurious Response for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.7B.2 Spurious Response for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.7B.3 Spurious Response for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.3.3_1.2 Out-of-band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.8B.2.1 Wideband Intermodulation for intra-band contiguous EN-DC in FR1 |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.2 Wideband Intermodulation for intra-band non-contiguous EN-DC in FR1 |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3 Wideband Intermodulation for inter-band EN-DC in FR1 |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3_1.1 Wideband Intermodulation for EN-DC within FR1 (3 CCs) |  | Same as 7.8A.2.1 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3_1.2 Wideband Intermodulation for EN-DC within FR1 (4 CCs) |  | Same as 7.8A.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3_1.3 Wideband Intermodulation for EN-DC within FR1 (5 CCs) |  | Same as 7.8A.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3_1.4 Wideband Intermodulation for EN-DC within FR1 (6 CCs) |  | Same as 7.8A.2 in TS 38.521-1 [8] |  |  |  |
| 7.9B.1 Spurious Emissions for intra-band contiguous EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.2 Spurious Emissions for intra-band non-contiguous EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3 Spurious Emissions for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3_1.1 Spurious Emissions for EN-DC within FR1 (3 CCs) |  | Same as 7.9A.1 in TS 38.521-1 [8] |  |  |  |
| 7.9B.4 Spurious Emissions for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.9 in TS 38.521-2 [9] |  |  |  |

# F.4	Uplink power window

## F.4.1	Introduction

A number of Tx and Rx Test cases set the UE uplink power to be within a defined window to ensure the test is carried out in the intended conditions. This clause gives the method for calculating the uplink power window used in Tx test cases and Rx Test cases.

## F.4.2	Setting the power window above a requirement

### F.4.2.1	NR FR1

Information from the core requirements in TS 38.101-1 [2], TS 38.213 [19] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window to ensure UE is not tested outside Core requirements.

This process is shown in the diagram below, using values for f ≤ 3GHz and BW ≤ 40MHz and taking an example where the target value is 0dBm (lower end of a UE Core requirement side condition range of 0 dBm ≤ Output power ≤ 10 dBm):

Figure F.4.2.1-1: Example NR FR1 uplink power setting to be above a requirement

The smallest UE Power step size is defined in TS 38.213 [9] Table 7.1.1-1, for absolute .

The UE Power step size tolerance is defined in TS 38.101-1 [2] Table 6.3.4.3-1, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods, and for a power step ΔP ≤ 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is within the Uplink power window, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.2.1-1.

### F.4.2.2	NR FR2

Information from the core requirements in TS 38.101-2 [3], TS 38.213 [19] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window to ensure UE is not tested outside Core requirements.

This process is shown in the diagram below, using values for FR2a and PUMAX ≥ P > Pint and taking an example where the target value is Pint:

Figure F.4.2.2-1: Example NR FR2 uplink power setting to be above a requirement

The smallest UE Power step size is defined in TS 38.213 [9] Table 7.1.1-1, for absolute .

The UE Power step size tolerance is defined in TS 38.101-2 [3] Table 6.3.4.3-1 and Table 6.3.4.3-2, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, Guard Periods, and for a power step ΔP = 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is within the Uplink power window, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.2.2-1.

### F.4.2.3	E-UTRA

Information from the core requirements in TS 36.101 [5], TS 36.213 [20] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window to ensure UE is not tested outside Core requirements.

This process is shown in the diagram below, using values for f ≤ 3GHz and BW ≤ 40MHz and taking an example where the target value is 0dBm (lower end of a UE Core requirement side condition range of 0 dBm ≤ Output power ≤ 10 dBm):

Figure F.4.2.3-1: Example E-UTRA uplink power setting to be above a requirement

The smallest UE Power step size is defined in TS 36.213 [20] Table 5.1.1.1-2, for absolute .

The UE Power step size tolerance is defined in TS 36.101 [5] Table 6.3.5.2.1-1, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods, and for a power step ΔP ≤ 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is within the Uplink power window, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.2.3-1.

## F.4.3	Setting the power window below a requirement

### F.4.3.1	NR FR1

Information from the core requirements in TS 38.101-1 [2], TS 38.213 [19] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window to ensure UE is not tested outside Core requirements.

This process is shown in the diagram below, using values for f ≤ 3GHz and BW ≤ 40MHz and taking an example where the target value is 4dB below PCMAX_L (UE Core requirement side condition):

Figure F.4.3.1-1: Example NR FR1 uplink power setting to be below a requirement

The smallest UE Power step size is defined in TS 38.213 [19] Table 7.1.1-1, for absolute .

The UE Power step size tolerance is defined in TS 38.101-1 [5] Table 6.3.4.3-1, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods, and for a power step ΔP ≤ 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is within the Uplink power window, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.3.1-1.

### F.4.3.2	NR FR2

Information from the core requirements in TS 38.101-2 [3], TS 38.213 [19] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window to ensure UE is not tested outside Core requirements.

This process is shown in the diagram below, using values for FR2a and PUMAX ≥ P > Pint and taking an example where the target value is PUMAX:

Figure F.4.3.2-1: Example NR FR2 uplink power setting to be below a requirement

The smallest UE Power step size is defined in TS 38.213 [19] Table 7.1.1-1, for absolute .

The UE Power step size tolerance is defined in TS 38.101-1 [3] Table 6.3.4.3-1 and Table 6.3.4.3-2, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, Guard Periods, and for a power step ΔP = 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is within the Uplink power window, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.3.2-1.

### F.4.3.3	E-UTRA

Information from the core requirements in TS 36.101 [5], TS 36.213 [20] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window to ensure UE is not tested outside Core requirements.

This process is shown in the diagram below, using values for f ≤ 3GHz and BW ≤ 40MHz and taking an example where the target value is 4dB below PCMAX_L (UE Core requirement side condition):

Figure F.4.3.3-1: Example E-UTRA uplink power setting to be below a requirement

The smallest UE Power step  size is defined in TS 36.213 [20] Table 5.1.1.1-2, for absolute .

The UE Power step size tolerance is defined in TS 36.101 [5] Table 6.3.5.2.1-1, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods, and for a power step ΔP ≤ 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is within the Uplink power window, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.3.3-1.

## F.4.4	Setting the power window centred on a target value

### F.4.4.1	NR FR1

Information from the core requirements in TS 38.101-1 [2], TS 38.213 [19] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window centred on the target value.

This process is shown in the diagram below, using values for f ≤ 3GHz and BW ≤ 40MHz and taking an example where the target value is +10dBm:

Figure F.4.4.1-1: Example NR FR1 uplink power setting centred on a target value

The smallest UE Power step size is defined in TS 38.213 [9] Table 7.1.1-1, for absolute .

The UE Power step size tolerance is defined in TS 38.101-1 [2] Table 6.3.4.3-1, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods, and for a power step ΔP ≤ 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is centred on the target value, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.4.1-1.

### F.4.4.2	NR FR2

Information from the core requirements in TS 38.101-2 [3], TS 38.213 [19] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window centred on the target value.

This process is shown in the diagram below, using values for FR2a and PUMAX ≥ P > Pint and taking an example where the target value is +15dBm:

Figure F.4.4.2-1: Example NR FR2 uplink power setting centred on a target value

The smallest UE Power step size is defined in TS 38.213 [9] Table 7.1.1-1, for absolute .

The UE Power step size tolerance is defined in TS 38.101-2 [3] Table 6.3.4.3-1 and Table 6.3.4.3-2, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, Guard Periods, and for a power step ΔP = 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is centred on the target value, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.4.2-1.

### F.4.4.3	E-UTRA

Information from the core requirements in TS 36.101 [5], TS 36.213 [20] and the uncertainties in Annex F applicable to the Test case are used to derive the uplink power window. There are 4 stages:

-	Find the uplink power target value.

-	Determine how closely the uplink power can be set to the target value.

-	Include the effect of test system uncertainty.

-	Position the Uplink power window centred on the target value.

This process is shown in the diagram below, using values for f ≤ 3GHz and BW ≤ 40MHz and taking an example where the target value is -10dBm:

Figure F.4.4.3-1: Example E-UTRA uplink power setting centred on a target value

The smallest UE Power step size is defined in TS 36.213 [20] Table 5.1.1.1-2, for absolute .

The UE Power step size tolerance is defined in TS 36.101 [5] Table 6.3.5.2.1-1, for PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods, and for a power step ΔP ≤ 1 dB.

The Test system uncertainties are defined in Annex F of the present document.

To ensure that the actual UE uplink power is centred on the target value, UE uplink power measured by the test system should remain within the smaller Uplink power control window shown in Figure F.4.4.3-1.

Annex G (normative):
Uplink Physical Channels

Please refer to Annex G in TS 38.521-1 [8] and TS 38.521-2 [9] for appropriate details as needed for test cases in this test specification. EN-DC exceptions will be added,

Annex H (normative):
Statistical Testing

Editor's Note:

-	Further investigate the technical details behind this statistical method to ensure that this is applicable for FR2 radiated test cases.

## H.1	General

This annex specifies mapping throughput to error ratio, pass fail limits and pass fail decision rules that are needed for measuring average throughput for a duration sufficient to achieve statistical significance for testing receiver characteristics.

## H.2	Statistical testing of receiver characteristics

### H.2.1	General

The test of receiver characteristics is twofold.

1.	A signal or a combination of signals is offered to the RX port(s) of the receiver.

2.	The ability of the receiver to demodulate /decode this signal is verified by measuring the throughput.

In (2) is the statistical aspect of the test and is treated here.

The minimum requirement for all receiver tests is >95% of the maximum throughput.

All receiver tests are performed in static propagation conditions. No fading conditions are applied.

### H.2.2	Mapping throughput to error ratio

a)	The measured information bit throughput R is defined as the sum (in kilobits) of the information bit payloads successfully received during the test interval, divided by the duration of the test interval (in seconds).

b)	In measurement practice the UE indicates successfully received information bit payload by signalling an ACK to the SS.
If payload is received, but damaged and cannot be decoded, the UE signals a NACK.

c)	Only the ACK and NACK signals, not the data bits received, are accessible to the SS.
The number of bits is known in the SS from knowledge of what payload was sent.

d)	For the reference measurement channel, applied for testing, the number of bits is different in different slots, however in a radio frame it is fixed during one test.

e)	The time in the measurement interval is composed of successfully received slots (ACK), unsuccessfully received slots (NACK) and no reception at all (DTX-slots).

f)	DTX-slots may occur regularly according the applicable reference measurement channel (regDTX).
In real live networks this is the time when other UEs are served. In TDD these are the UL and special slots.
regDTX vary from test to test but are fixed within the test.

g)	Additional DTX-slots occur statistically when the UE is not responding ACK or NACK where it should. (statDTX)
This may happen when the UE was not expecting data or decided that the data were not intended for it.

The pass / fail decision is done by observing the:

-	number of NACKs

-	number of ACKs and

-	number of statDTXs (regDTX is implicitly known to the SS)

The ratio (NACK + statDTX)/(NACK+ statDTX + ACK)is the Error Ratio (ER). Taking into account the time consumed by the ACK, NACK, and DTX-TTIs (regular and statistical), ER can be mapped unambiguously to throughput for any single reference measurement channel test.

### H.2.3	Design of the test

The test is defined by the following design principles (see clause H.x, Theory….):

1.	The early decision concept is applied.

2.	A second limit is introduced: Bad DUT factor M>1

3.	To decide the test pass:

-	Supplier risk is applied based on the Bad DUT quality

-	To decide the test fail

-	Customer Risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1.	Limit ER = 0.05 (Throughput limit = 95%)

2.	Bad DUT factor M=1.5 (selectivity)

3.	Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

### H.2.4	Numerical definition of the pass fail limits

Table H.2.4-1: pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 67 | NA | 39 | 763 | 500 | 78 | 1366 | 1148 | 117 | 1951 | 1828 |
| 1 | 95 | NA | 40 | 778 | 516 | 79 | 1381 | 1166 | 118 | 1965 | 1845 |
| 2 | 119 | NA | 41 | 794 | 532 | 80 | 1396 | 1183 | 119 | 1980 | 1863 |
| 3 | 141 | NA | 42 | 810 | 548 | 81 | 1412 | 1200 | 120 | 1995 | 1881 |
| 4 | 162 | NA | 43 | 826 | 564 | 82 | 1427 | 1217 | 121 | 2010 | 1899 |
| 5 | 183 | NA | 44 | 842 | 580 | 83 | 1442 | 1234 | 122 | 2025 | 1916 |
| 6 | 202 | NA | 45 | 858 | 596 | 84 | 1457 | 1252 | 123 | 2039 | 1934 |
| 7 | 222 | NA | 46 | 873 | 612 | 85 | 1472 | 1269 | 124 | 2054 | 1952 |
| 8 | 241 | NA | 47 | 889 | 629 | 86 | 1487 | 1286 | 125 | 2069 | 1969 |
| 9 | 259 | NA | 48 | 905 | 645 | 87 | 1502 | 1303 | 126 | 2084 | 1987 |
| 10 | 278 | 76 | 49 | 920 | 661 | 88 | 1517 | 1321 | 127 | 2099 | 2005 |
| 11 | 296 | 88 | 50 | 936 | 678 | 89 | 1532 | 1338 | 128 | 2113 | 2023 |
| 12 | 314 | 100 | 51 | 952 | 694 | 90 | 1547 | 1355 | 129 | 2128 | 2040 |
| 13 | 332 | 113 | 52 | 967 | 711 | 91 | 1562 | 1373 | 130 | 2143 | 2058 |
| 14 | 349 | 126 | 53 | 983 | 727 | 92 | 1577 | 1390 | 131 | 2158 | 2076 |
| 15 | 367 | 140 | 54 | 998 | 744 | 93 | 1592 | 1407 | 132 | 2172 | 2094 |
| 16 | 384 | 153 | 55 | 1014 | 760 | 94 | 1607 | 1425 | 133 | 2187 | 2111 |
| 17 | 401 | 167 | 56 | 1029 | 777 | 95 | 1623 | 1442 | 134 | 2202 | 2129 |
| 18 | 418 | 181 | 57 | 1045 | 793 | 96 | 1637 | 1459 | 135 | 2217 | 2147 |
| 19 | 435 | 195 | 58 | 1060 | 810 | 97 | 1652 | 1477 | 136 | 2231 | 2165 |
| 20 | 452 | 209 | 59 | 1076 | 827 | 98 | 1667 | 1494 | 137 | 2246 | 2183 |
| 21 | 469 | 224 | 60 | 1091 | 844 | 99 | 1682 | 1512 | 138 | 2261 | 2201 |
| 22 | 486 | 238 | 61 | 1106 | 860 | 100 | 1697 | 1529 | 139 | 2275 | 2218 |
| 23 | 503 | 253 | 62 | 1122 | 877 | 101 | 1712 | 1547 | 140 | 2290 | 2236 |
| 24 | 519 | 268 | 63 | 1137 | 894 | 102 | 1727 | 1564 | 141 | 2305 | 2254 |
| 25 | 536 | 283 | 64 | 1153 | 911 | 103 | 1742 | 1582 | 142 | 2320 | 2272 |
| 26 | 552 | 298 | 65 | 1168 | 928 | 104 | 1757 | 1599 | 143 | 2334 | 2290 |
| 27 | 569 | 313 | 66 | 1183 | 944 | 105 | 1772 | 1617 | 144 | 2349 | 2308 |
| 28 | 585 | 328 | 67 | 1199 | 961 | 106 | 1787 | 1634 | 145 | 2364 | 2326 |
| 29 | 602 | 343 | 68 | 1214 | 978 | 107 | 1802 | 1652 | 146 | 2378 | 2344 |
| 30 | 618 | 359 | 69 | 1229 | 995 | 108 | 1817 | 1669 | 147 | 2393 | 2361 |
| 31 | 634 | 374 | 70 | 1244 | 1012 | 109 | 1832 | 1687 | 148 | 2408 | 2379 |
| 32 | 650 | 389 | 71 | 1260 | 1029 | 110 | 1847 | 1704 | 149 | 2422 | 2397 |
| 33 | 667 | 405 | 72 | 1275 | 1046 | 111 | 1861 | 1722 | 150 | 2437 | 2415 |
| 34 | 683 | 421 | 73 | 1290 | 1063 | 112 | 1876 | 1740 | 151 | 2452 | 2433 |
| 35 | 699 | 436 | 74 | 1305 | 1080 | 113 | 1891 | 1757 | 152 | 2466 | 2451 |
| 36 | 715 | 452 | 75 | 1321 | 1097 | 114 | 1906 | 1775 | 153*) | NA | 2469 |
| 37 | 731 | 468 | 76 | 1336 | 1114 | 115 | 1921 | 1793 |  |  |  |
| 38 | 747 | 484 | 77 | 1351 | 1131 | 116 | 1936 | 1810 | *) note 2 in H.2.5 |  |  |

NOTE 1:	The first column is the number of errors (ne = number of NACK + statDTX)

NOTE 2:	The second column is the number of samples for the pass limit (nsp, ns=Number of Samples= number of NACK + statDTX + ACK)

NOTE 3:	The third column is the number of samples for the fail limit (nsf)

### H.2.5	Pass fail decision rules

The pass fail decision rules apply for a single test, comprising one component in the test vector. The over all Pass /Fail conditions are defined in clause H.2.6and H.2A.6

Having observed   0  errors, pass the test at      67+    samples,                                                 otherwise continue

Having observed   1  error,   pass the test at     95+  otherwise continue

Having observed   2  errors,  pass the test at     119+  samples, fail the test at       2- samples,  otherwise continue

Etc. etc.

Having observed 151 errors,  pass the test at  2452+  samples, fail the test at 2433- samples,  otherwise continue

Having observed 152 errors,  pass the test at  2466+  samples, fail the test at 2451- samples.

Where x+ means: x or more,        x- means x or less

NOTE 1:	an ideal DUT passes after 67 samples. The maximum test time is 2466 samples.

NOTE 2:	It is allowed to deviate from the early decision concept  by  postponing  the decision (pass/fail or continue). Postponing the decision to or beyond the end of Table H.2.4-1 requires a pass fail decision against the test limit: pass the DUT for ER<0.0618, otherwise fail.

Annex I (normative):
Void

Annex J (normative):
Test applicability per permitted test method

Please refer to Annex J in TS 38.521-1 [8] and 38.521-2 [9] for appropriate details as needed for test cases in this test specification. Exceptions for EN-DC may be added as needed.

Annex K (normative):
EIRP, TRP, and EIS measurement procedures

Please refer to Annex K in 38.521-2 [9] for appropriate details as needed for test cases in this test specification. Exceptions for EN-DC may be added as needed.

Annex L (normative):
Void

Annex M (normative):
Dual uplink interferer

UE is mandated to support operation in dual and triple uplink mode for EN-DC configuration in NR FR1 listed in Table 5.5B.2-1, Table 5.5B.3-1, and Table 5.5B.4.1-1 and indicated by column single uplink allowed, Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.2-0, Table 7.3B.2.0.3.5.2-1 or NE-DC configuration in NR FR1 listed in Table 5.5B.4a.1-1 and indicated by column single uplink allowed if the intermodulation products caused by the dual uplink operation do not interfere with its own primary downlink transmission channel bandwidth. For intermodulation products falling into LTE secondary downlink channel bandwidth, UE single UL capability is not considered.

Formula for determining if the EN-DC in NR FR1 configuration with dual uplink operation interferes with its own downlink reception.

Interference bandwidth: IBW = |a| * CBW1 + |b| * CBW2

-	|a| + |b| = 2 (or 3)

-	CBW1 and CBW2 are the transmission bandwidth configurations of the UL channels

Center frequency of IBW:  fIBW = |a * f1 + b * f2|

-	f1 and f2 are center frequency of the transmission bandwidth configurations of each UL channel

The range of IMD 2 (or 3): [fIBW – IBW/2, fIBW + IBW/2]

NOTE 1:	UE shall be able to apply operations which are configured by RRC reconfiguration and corresponding HARQ timing on the transmission bandwidth.

NOTE 2:	For identified difficult band combination, during two adjacent RRC reconfiguration, the changing of transmission bandwidth should not introduce IM2 and IM3, which will result in UE changing from 2Tx to 1Tx. Otherwise, UE behavior is not specified.

For DC_3A_n3A intra-band non-contiguous EN-DC combination, only single switched UL is supported in Rel-15.

For DC_2A_n2A, DC_66A_n66A intra-band non-contiguous EN-DC combination, only single switched UL is supported.

Annex N (normative):
Modified MPR behaviour

Editor’s note: The modifiedMPRbehavior is given in the Annex H.1 of TS 38.101-3.

# N.1	Indication of modified MPR behaviour

This annex contains the definitions of the bits in the field modifiedMPRbehavior indicated in the IE RF-Parameters [13] by a UE supporting an MPR or A-MPR modified in a later release of this specification. modifiedMPRbehavior is indicated in TS 36.211 [13] by an 8-bit bitmap per NR band.

Table N.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n41 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | - EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | - EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
| n71 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |

Annex O (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN5#76 | R5-174710 | - | - | - | Draft skeleton | 0.0.1 |
| 2018-01 | RAN5#1-5G-NR Adhoc | R5-180086 | - | - | - | TP to add clause 6.2B.3.3 UE A-MPR intra-band EN-DC to 38.521-3 | 0.1.0 |
| 2018-01 | RAN5#1-5G-NR Adhoc | R5-180087 | - | - | - | TP to add clause 6.5B.2.1.2 Additional Spectrum emissions mask (contiguous sub-blocks) for intra-band EN-DC to 38.521-3 | 0.1.0 |
| 2018-02 | RAN5#78 | R5-181509 | - | - | - | Updated 38.521-3 for new Annex A Dual uplink interferer information | 0.2.0 |
| 2018-02 | RAN5#78 | R5-181690 | - | - | - | Updated 38.521-3 for channel bandwidth information | 0.2.0 |
| 2018-03 | RAN5#2-5G-NR Adhoc | R5-181760 | - | - | - | Draft TS 38.521-3 0.3.0 | 0.3.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-182035 | - | - | - | 5G-NR Text Proposal to add spurious emissions test case to 38.521-3 | 0.4.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-182016 | - | - | - | TP for new test case: 6.5B.2.1.3, Adjacent channel leakage ratio for intra-band contiguous EN-DC | 0.4.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-182017 | - | - | - | TP to update clause 6.2B.3.1 UE A-MPR intra-band EN-DC to 38.521-3 | 0.4.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-182018 | - | - | - | TP to update clause 6.5B.2.1.2 Additional spectrum emission mask to 38.521-3 | 0.4.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-181807 | - | - | - | Update to Operating bands of 38.521-3 | 0.4.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-181808 | - | - | - | Update to clause 3 and clause 4 of 38.521-3 | 0.4.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-181828 | - | - | - | Updated 38.521-3 for channel bandwidth information with new structure | 0.4.0 |
| 2018-07 | RAN5#79 | R5-183961 | - | - | - | 5G_FR1_EN_DC_RF_sensitivity_for_DC | 0.5.0 |
| 2018-07 | RAN5#79 | R5-183962 | - | - | - | Introduction of TC 6.2B.1.3 for EN-DC | 0.5.0 |
| 2018-07 | RAN5#79 | R5-183949 | - | - | - | Statistical Testing Annex for 38.521-3 | 0.5.0 |
| 2018-07 | RAN5#79 | R5-182995 | - | - | - | Corrections annex for EIRP and TRP metric definition in TS 38.521-3 | 0.5.0 |
| 2018-07 | RAN5#79 | R5-183707 | - | - | - | TP for updating test case 6.2B.2.1, UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | 0.5.0 |
| 2018-07 | RAN5#79 | R5-183708 | - | - | - | Updated clause 5.5B Configuration for DC to 38.521-3 | 0.5.0 |
| 2018-07 | RAN5#79 | R5-183709 | - | - | - | TP to add Occupied BW EN-DC test case | 0.5.0 |
| 2018-07 | RAN5#79 | R5-183710 | - | - | - | TP to add SEM EN-DC test case | 0.5.0 |
| 2018-07 | RAN5#79 | R5-183711 | - | - | - | TP to add ACLR EN-DC test case | 0.5.0 |
| 2018-09 | RAN5#80 | R5-185563 | - | - | - | FR2_StoreTxRxBeamPeakCoordinates_38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185520 | - | - | - | Addition of TRx MU and TT in TS 38.521-3 Annex | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185503 | - | - | - | Add Clause 7.5B.1 into TS 38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185504 | - | - | - | Add Clause 7.5B.2 into TS 38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185505 | - | - | - | Add Clause 7.5B.3 into TS 38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-184579 | - | - | - | Updated EN-DC configuration information in clause 5 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-184580 | - | - | - | TIB value add for EN-DC band in 38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-184671 | - | - | - | Update of References in clause 2 of 38.521-3 spec | 1.0.0 |
| 2018-09 | RAN5#80 | R5-184672 | - | - | - | Updates to Operating Bands in clause 5.2 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-184737 | - | - | - | Dual uplink interferer updated to 38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-184737 | - | - | - | Dual uplink interferer updated to 38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185332 | - | - | - | Addition of 6.2B.4.1.1 Configured OP for Intra-Band Contiguous EN-DC | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185333 | - | - | - | Addition of 6.2B.4.1.2 Configured OP for Intra-Band Non-Contiguous EN-DC | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185507 | - | - | - | Addition of 6.2B.4.1.3 Configured OP for Inter-Band within FR1 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185198 | - | - | - | Addition of 6.2B.4.1.4 Configured OP for Inter-Band EN-DC including FR2 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185199 | - | - | - | Addition of 6.2B.4.1.5 Configured OP for Inter-Band EN-DC including both FR1 and FR2 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185469 | - | - | - | TP for updating test case 6.2B.3.1 UE AMPR for Intra-band contiguous EN-DC | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185470 | - | - | - | TP for updating test case 6.2B.3.2 UE AMPR for Intra-band non-contiguous EN-DC | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185200 | - | - | - | TP for updating test case 6.5B.2.1.2 Additional spectrum emission mask for intra-band contiguous EN-DC | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185556 | - | - | - | FR2_UE_BeamlockInvoke_38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185472 | - | - | - | Update of TC 6.2B.1.1 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185473 | - | - | - | Introduction of TC 6.2B.1.2 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185474 | - | - | - | Update of 6.2B.1.3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185201 | - | - | - | Introduction of TC 7.4B.1 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185202 | - | - | - | Introduction of 7.4B.2 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185203 | - | - | - | Introduction of 7.4B.3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185479 | - | - | - | Update Occupied Bandwidth for interband EN-DC within FR1 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185480 | - | - | - | Update SEM interband EN-DC within FR1 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185481 | - | - | - | Update ACLR for interband EN-DC within FR1 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185204 | - | - | - | 5G NR_EN_DC with FR1_Text update for RX sensitivity | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185205 | - | - | - | 5G NR_EN_DC with FR1_Text_proposal for_TX_Spurious_emission | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185422 | - | - | - | Alignment of Annex numbering with core spec | 1.0.0 |
| 2018-09 | RAN5#80 | R5-184897 | - | - | - | Updates to Channel Arrangement clause in 38.521-3 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185206 | - | - | - | Addition of TC6.3B.1.1 Minimum Output power for intra-band contiguous EN-DC | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185207 | - | - | - | Addition of TC6.3B.1.2 Minimum output power for intra-band non-contiguous EN-DC | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185208 | - | - | - | Addition of TC6.3B.1.3 Minimum output power for inter-band EN-DC within FR1 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185351 | - | - | - | Update across EN-DC RF test cases in TS 38.521-3 | 1.0.0 |
| 2018-09 | RAN#81 | - | - | - | - | raised to v15.0.0 with editorial changes only | 15.0.0 |
| 2018-12 | RAN#82 | R5-186503 | 0033 | - | F | FR2 Spurious Emission test case updates | 15.1.0 |
| 2018-12 | RAN#82 | R5-186506 | 0034 | - | F | Update Text on Store Beam Peak Coordinate | 15.1.0 |
| 2018-12 | RAN#82 | R5-186507 | 0035 | - | F | 38.521-3 Applicability Rules | 15.1.0 |
| 2018-12 | RAN#82 | R5-186601 | 0039 | - | F | 5G NR_EN_DC with FR1_Text update for Intra-Band Contiguous RX sensitivity | 15.1.0 |
| 2018-12 | RAN#82 | R5-186602 | 0040 | - | F | 5G NR_Text update for TX spurious emission intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-186608 | 0042 | - | F | Spurious emission band UE co-existence for Inter-band EN-DC within FR1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-186672 | 0044 | - | F | Updating test case 6.2B.3.1 Additional Maximum Output Power reduction for Intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-186673 | 0045 | - | F | Updating test case 6.5B.2.1.2 Additional spectrum emissions mask for intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-186681 | 0046 | - | F | Updates to EN-DC test case 6.2B.2.1, UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-186684 | 0047 | - | F | Updates to test case 6.2B.2.3, UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-186788 | 0049 | - | F | Minor update OBW, SEM and ACLR inter-band FR1 test cases | 15.1.0 |
| 2018-12 | RAN#82 | R5-187153 | 0061 | - | F | Updated EN-DC configuration information in clause 5 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187371 | 0076 | - | F | Addition of TC6.3B.2.1 Transmit OFF Power for intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187372 | 0077 | - | F | Addition of TC6.3B.2.3 Transmit OFF Power for inter-band EN-DC within FR1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187373 | 0078 | - | F | Addition of TC6.3B.2.2 Transmit OFF Power for intra-band non-contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187552 | 0083 | - | F | Updates to TS 38.521-3 common sections 1-4 to align with core spec | 15.1.0 |
| 2018-12 | RAN#82 | R5-187559 | 0084 | - | F | Updates to TS 38.521-3 clause 5 to align with core spec | 15.1.0 |
| 2018-12 | RAN#82 | R5-187562 | 0085 | - | F | Update to TC6.5B.3.2.1 - General Spurious Emissions for intra-band non-contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187563 | 0086 | - | F | Update to 7.3B.2.2 - REFSENS for Intra-band Non-Contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187565 | 0087 | - | F | Updates to TS 38.521-3 clause 4 with LTE anchor details | 15.1.0 |
| 2018-12 | RAN#82 | R5-187614 | 0094 | - | F | Updates to EN-DC test case 6.2B.2.2, UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187816 | 0048 | 1 | F | Adding test case 6.2B.2.4, UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187819 | 0053 | 1 | F | Update general parameter Connection without release in initial conditions in TS 38.521-3 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187820 | 0043 | 1 | F | Updates to test case 6.5B.2.1.3, Adjacent channel leakage ratio for intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187821 | 0052 | 1 | F | Addition OBW intraband non contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187822 | 0055 | 1 | F | Introduction of New test case 6.4B.2.2.1 Error Vector Magnitude for intra-band non-contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187823 | 0056 | 1 | F | Introduction of New test case 6.4B.2.2.2 Carrier Leakage for intra-band non-contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187825 | 0058 | 1 | F | Introduction of New test case 6.4B.2.3.1 Error Vector Magnitude for inter-band EN-DC within FR1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187826 | 0059 | 1 | F | Introduction of New test case 6.4B.2.3.2 Carrier Leakage for inter-band EN-DC within FR1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187827 | 0060 | 1 | F | Introduction of New test case 6.4B.2.3.3 In-band Emissions for inter-band EN-DC within FR1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187828 | 0070 | 1 | F | Introduction of Error Vector Magnitude for intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187829 | 0071 | 1 | F | Introduction of Carrier Leakage for intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-187831 | 0088 | 1 | F | FR2 General Spurious Emission test case update | 15.1.0 |
| 2018-12 | RAN#82 | R5-187832 | 0089 | 1 | F | FR2 Reference Sensitivity test case update | 15.1.0 |
| 2018-12 | RAN#82 | R5-187833 | 0092 | 1 | F | Updates to clause 7.3B.3.4 in TS 38.521-3 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187834 | 0090 | 1 | F | Updates to sections 1-4 in TS 38.521-3 to align with core spec | 15.1.0 |
| 2018-12 | RAN#82 | R5-187835 | 0091 | 1 | F | Updates to Clause 5 in TS 38.521-3 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187913 | 0067 | 1 | F | Addition of notes to clarify test point selection into general clause of TS 38.521-3 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188012 | 0057 | 1 | F | Introduction of New test case 6.4B.2.2.3 In-band Emissions for intra-band non-contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188013 | 0050 | 1 | F | Addition OBW intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188014 | 0051 | 1 | F | Addition SEM intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188015 | 0064 | 1 | F | Additional Spurious Emissions for Intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188016 | 0065 | 1 | F | Additional Spurious Emissions for Intra-band non-contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188017 | 0066 | 1 | F | Additional Spurious emission for inter-band EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188018 | 0068 | 1 | F | Spurious emission band UE co-existence for intra-band non-contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188019 | 0072 | 1 | F | Introduction of In-band Emissions for intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188020 | 0073 | 1 | F | Addition of TC6.3B.3.1 Tx ON/OFF time mask for intra-band contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188021 | 0074 | 1 | F | Addition of TC6.3B.3.2 Tx ON/OFF time mask for intra-band non-contiguous EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188022 | 0075 | 1 | F | Addition of TC6.3B.3.3 Tx ON/OFF time mask for inter-band EN-DC within FR1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188023 | 0080 | 1 | F | Update of test case 6.5B.2.1.2 Additional spectrum emission mask for intra-band contiguous EN-DC for NS_04 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188024 | 0081 | 1 | F | Update of test case 6.2B.3.1 UE A-MPR for Intra-band contiguous EN-DC for NS_04 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188025 | 0038 | 1 | F | Update Clause 7.5B.3 in TS 38.521-3 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188026 | 0041 | 1 | F | 5G NR_EN_DC with FR1_Text update for Inter-Band RX sensitivity | 15.1.0 |
| 2018-12 | RAN#82 | R5-188027 | 0082 | 1 | F | Update TC 7.4B.3 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188028 | 0036 | 1 | F | Updates of MU in TS 38.521-3 Annex F during RAN5#81 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188029 | 0037 | 1 | F | Updates of TT in TS 38.521-3 Annex F during RAN5#81 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188039 | 0093 | 1 | F | LTE Anchor Link configuration for FR2 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188219 | 0062 | 1 | F | Introduction of receiver spurious emission tests for FR1 inter-band EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188220 | 0063 | 1 | F | Introduction of wideband intermodulation tests for FR1 inter-band EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188221 | 0054 | 1 | F | LTE TDD configuration for UE Tx test in EN-DC | 15.1.0 |
| 2018-12 | RAN#82 | R5-188222 | 0069 | 1 | F | Core alignment CR to capture TS 38.101-3 updates during RAN4#89 | 15.1.0 |
| 2019-03 | RAN#83 | R5-191057 | 0165 | - | F | Introduction of TC 7.5B.0 | 15.2.0 |
| 2019-03 | RAN#83 | R5-191157 | 0174 | - | F | Updated EN-DC configuration information in clause 5 | 15.2.0 |
| 2019-03 | RAN#83 | R5-191231 | 0175 | - | F | Adding missing reference to 38.521-3 | 15.2.0 |
| 2019-03 | RAN#83 | R5-191336 | 0176 | - | F | Updates to EN-DC test case 6.2B.2.1, UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-191339 | 0177 | - | F | Updates to EN-DC test case 6.2B.2.2, UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-191340 | 0178 | - | F | Adding test case 6.5A.2.3, Adjacent channel leakage ratio for CA without EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-191510 | 0183 | - | F | Shared Risk clarification in TS 38.521-3 | 15.2.0 |
| 2019-03 | RAN#83 | R5-191637 | 0193 | - | F | Updates of TT in TS 38.521-3 Annex F during RAN5#NR4 | 15.2.0 |
| 2019-03 | RAN#83 | R5-191845 | 0200 | - | F | Text update for 7.3B.3 deltaRIB,c deltaRIBNC for EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-191867 | 0205 | - | F | Addition of 7.3B.2.0 Reference sensitivity Minimum Conformance Requirements for EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192004 | 0215 | - | F | Update of TC 6.2B.1.1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192005 | 0216 | - | F | Update of TC 6.2B.1.2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192006 | 0217 | - | F | Update of TC 6.2B.1.3 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192176 | 0224 | - | F | TIB,c updated for CA and EN-DC cases | 15.2.0 |
| 2019-03 | RAN#83 | R5-192177 | 0225 | - | F | Updated to Annex M Dual uplink interferer | 15.2.0 |
| 2019-03 | RAN#83 | R5-192206 | 0228 | - | F | 38.521-3 Common clause updates to clarify leverage across architecture options | 15.2.0 |
| 2019-03 | RAN#83 | R5-192207 | 0229 | - | F | Formatting updates and index correction in TS 38.521-3 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192208 | 0230 | - | F | 38.521-1 Common clause updates to clarify leverage across architecture options | 15.2.0 |
| 2019-03 | RAN#83 | R5-192209 | 0231 | - | F | 38.521-2 Common clause updates to clarify leverage across architecture options | 15.2.0 |
| 2019-03 | RAN#83 | R5-192242 | 0237 | - | F | Update of test case 6.2B.3.1 UE A-MPR for Intra-band contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192243 | 0238 | - | F | Update of test case 6.2B.3.4 UE A-MPR for Inter-Band EN-DC including FR2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192244 | 0239 | - | F | Update of test case 6.5B.2.1.2 UE Additional spectrum emissions mask for intra-band contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192423 | 0157 | 1 | F | Update of TC 7.5B.1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192424 | 0159 | 1 | F | Introduction of TC 7.5B.4 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192425 | 0166 | 1 | F | Update of TC 7.5B.3 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192427 | 0180 | 1 | F | Introduction of NSA FR1 7.6B.2.1 Inband blocking for intra-band contiguous EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192428 | 0182 | 1 | F | Introduction of NSA FR1 7.6B.2.3 Inband blocking for inter-band EN-DC within FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192429 | 0184 | 1 | F | Introduction of NSA FR1 7.6B.3.1 Out-of-band blocking for intra-band contiguous EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192430 | 0185 | 1 | F | Introduction of NSA FR1 7.6B.3.2 Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192431 | 0186 | 1 | F | Introduction of NSA FR1 7.6B.3.3 Out-of-band blocking for inter-band EN-DC within FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192432 | 0187 | 1 | F | Introduction of NSA FR1 7.6B.4.1 Narrow band blocking for intra-band contiguous EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192433 | 0188 | 1 | F | Introduction of NSA FR1 7.6B.4.2 Narrow band blocking for intra-band non-contiguous EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192434 | 0189 | 1 | F | Introduction of NSA FR1 7.6B.4.3 Narrow band blocking for inter-band EN-DC within FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192435 | 0190 | 1 | F | Introduction of NSA FR1 7.7B.1 Spurious Response for intra-band contiguous EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192436 | 0191 | 1 | F | Introduction of NSA FR1 7.7B.2 Spurious Response for intra-band non-contiguous EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192437 | 0192 | 1 | F | Introduction of NSA FR1 7.7B.3 Spurious Response for inter-band EN-DC within FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192438 | 0207 | 1 | F | Text Update for 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192439 | 0234 | 1 | F | TS 38.521-3 corrections and clean-up to TC categories within Clause 7 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192440 | 0179 | 1 | F | Editorial: Band combinations for Inter-band CA between FR1 and FR2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192441 | 0232 | 1 | F | TS 38.521-3 clause 5 updates to align with core specification | 15.2.0 |
| 2019-03 | RAN#83 | R5-192442 | 0195 | 1 | F | Addition of TC6.3B.4.3 PRACH Time Mask for inter-band EN-DC within FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192443 | 0220 | 1 | F | Update of EN-DC 6.2B.4.1.3 Configured transmitted power inter-band within FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192445 | 0233 | 1 | F | TS 38.521-3 corrections and clean-up to TC categories within Clause 6 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192453 | 0198 | 1 | F | FR2 NSA Spurious Emission Coexistence test case | 15.2.0 |
| 2019-03 | RAN#83 | R5-192454 | 0199 | 1 | F | FR2 NSA Frequency Error test case | 15.2.0 |
| 2019-03 | RAN#83 | R5-192455 | 0221 | 1 | F | Addition of transmit modulation quality test cases for inter-band EN-DC including FR2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192456 | 0222 | 1 | F | Introduction 6.5B.1.4 OBW interband EN-DC including FR2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192457 | 0223 | 1 | F | Introduction 6.5B.2.4.1 SEM interband EN-DC including FR2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192458 | 0226 | 1 | F | Introduction 6.5B.2.4.3 ACLR interband EN-DC including FR2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192459 | 0236 | 1 | F | Addition of TC6.3B.1.4 - Minimum Output Power for EN-DC Interband including FR2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192535 | 0194 | 1 | F | Clean up of occupied bandwidth for EN-DC within FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192600 | 0196 | 1 | F | Addition of TC6.3B.4.1 PRACH Time Mask for intra-band contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192601 | 0197 | 1 | F | Addition of TC6.3B.4.2 PRACH Time Mask for intra-band non-contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192603 | 0201 | 1 | F | Text Update for 6.5B.3.2 Spurious Emissions for intra-band non-contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192604 | 0202 | 1 | F | Text Update for 6.5B.3.1 Spurious Emissions for intra-band contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192605 | 0203 | 1 | F | Text Update for 6.5B.3.3 Spurious Emissions for Inter-band EN-DC within FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192606 | 0204 | 1 | F | Text Update for 6.5B.4 Additional Spurious Emissions for EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192607 | 0209 | 1 | F | Update to Carrier Leakage for intra-band contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192608 | 0210 | 1 | F | Introduction of Error Vector Magnitude for intra-band contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192609 | 0211 | 1 | F | Update to In-band Emissions for intra-band contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192610 | 0227 | 1 | F | Clarification on UL slots in OBW, SEM and ACLR in TS 38.521-3 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192611 | 0158 | 1 | F | Update of TC 7.5B.2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192617 | 0167 | 1 | F | Introducing Wideband Intermodulation for intra-band EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192618 | 0168 | 1 | F | Introducing receiver spurious emission for intra-band EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192619 | 0181 | 1 | F | Introduction of NSA FR1 7.6B.2.2 Inband blocking for intra-band non-contiguous EN-DC in FR1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192620 | 0206 | 1 | F | Text Update for 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192622 | 0170 | 1 | F | Updates of MU in TS 38.521-3 Annex F during RAN5#82 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192623 | 0171 | 1 | F | Updates of TT in TS 38.521-3 Annex F during RAN5#82 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192626 | 0214 | 1 | F | Update of TC 7.4B.2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192681 | 0235 | 1 | F | Addition of TC6.2B.1.4 - Max Output Power for EN-DC Interband including FR2 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192689 | 0218 | 1 | F | Update of 6.2B.4.1.1 Configured output power Intra-band contiguous EN-DC | 15.2.0 |
| 2019-03 | RAN#83 | R5-192690 | 0219 | 1 | F | Update of EN-DC 6.2B.4.1.2 Configured transmitted power Intra-band non-contiguous | 15.2.0 |
| 2019-03 | RAN#83 | R5-192844 | 0213 | 2 | F | Update of TC 7.4B.1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192847 | 0212 | 1 | F | Addition of 7.4B.0 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192863 | 0172 | 1 | F | Introduction of TxIM (inter-band EN-DC within FR1) | 15.2.0 |
| 2019-03 | RAN#83 | - | - | - | - | Editorial correction of references to TS 38.508-1 clause 4.6 tables | 15.2.0 |
| 2019-06 | RAN#84 | R5-193539 | 0294 | - | F | Adding missing reference in 38.521-3 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193546 | 0295 | - | F | Updates to 6.2B.2.3, UE Maximum Output power reduction for inter-band EN-DC within FR1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193547 | 0296 | - | F | Adding test case 6.2B.2.5, UE Maximum Output power reduction for inter-band EN-DC including both FR1 and FR2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193548 | 0297 | - | F | Update of test case 6.2B.2.1, UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193714 | 0309 | - | F | Update of Clause 5.5B Configuration for DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193719 | 0311 | - | F | Update of TC 7.5B.2 ACS for intra-band non-contiguous 2CC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193720 | 0312 | - | F | Update of TC 7.5B.3 ACS for inter-band FR1 2CC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193722 | 0314 | - | F | Update of TC 7.5B.4.1 ACS for inter-band FR2 2CC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193724 | 0316 | - | F | Introduction of TC 7.5B.4.3 ACS for inter-band FR2 4CC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193725 | 0317 | - | F | Introduction of TC 7.5B.4.4 ACS for inter-band FR2 5CC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193726 | 0318 | - | F | Introduction of TC 7.5B.4.5 ACS for inter-band FR2 6CC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193727 | 0319 | - | F | Introduction of TC 7.5B.5 inter-band FR1 FR2 3CC | 15.3.0 |
| 2019-06 | RAN#84 | R5-193886 | 0320 | - | F | Introduction of NSA FR2 7.6B.2.4.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193888 | 0322 | - | F | Update of NSA FR1 7.6B.2.0 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193896 | 0324 | - | F | Update of NSA FR1 7.6B.2.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193899 | 0326 | - | F | Update of NSA FR1 7.6B.3.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193900 | 0327 | - | F | Update of NSA FR1 7.6B.3.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193901 | 0328 | - | F | Update of NSA FR1 7.6B.4.0 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193902 | 0329 | - | F | Update of NSA FR1 7.6B.4.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193903 | 0330 | - | F | Update of NSA FR1 7.6B.4.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193904 | 0331 | - | F | Update of NSA FR1 7.7B.0 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193905 | 0332 | - | F | Update of NSA FR1 7.7B.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193906 | 0333 | - | F | Update of NSA FR1 7.7B.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193947 | 0343 | - | F | Removing invalid test IDs from test case 6.5B.2.1.3 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194016 | 0344 | - | F | 38.521-3 Annex re-alignment | 15.3.0 |
| 2019-06 | RAN#84 | R5-194128 | 0347 | - | F | Addition SEM intraband non-contiguous EN-DC in TS 38.521-3 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194129 | 0348 | - | F | Addition ACLR intraband non-contiguous EN-DC in TS 38.521-3 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194164 | 0351 | - | F | Update of test case 6.2B.3.1 UE A-MPR for Intra-band contiguous EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194166 | 0353 | - | F | Update of test case 6.2B.3.3 UE A-MPR for Inter-Band EN-DC within FR1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194317 | 0359 | - | F | Text Update for 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194318 | 0360 | - | F | Text Update for 7.3B.2.1 Ref sensitivity for Intra-band Contiguous EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194319 | 0361 | - | F | Text Update for 6.5B.4 Additional Spurious Emissions for EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194320 | 0362 | - | F | Text Update for 7.3B.3 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194374 | 0366 | - | F | Text Update for 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band EN-DC within FR1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194375 | 0367 | - | F | Text Update for 7.3B.2.2 Ref sensitivity for Intra-band Non-Contiguous EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194376 | 0368 | - | F | Text Update for 7.3B.2.0 Min Requirements of Ref sensitivity for EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194400 | 0372 | - | F | Update to Carrier Leakage and In-band Emissions for intra-band contiguous EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194463 | 0378 | - | F | Update to 6.2.3 A-MPR FR2 NSA | 15.3.0 |
| 2019-06 | RAN#84 | R5-194484 | 0380 | - | F | Update EN-DC Transmit modulation quality test cases | 15.3.0 |
| 2019-06 | RAN#84 | R5-194621 | 0381 | - | F | Update of transmit modulation quality test cases for inter-band EN-DC including FR2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194706 | 0382 | - | F | TS 38.521-3 clause 5 updates to align with core specification | 15.3.0 |
| 2019-06 | RAN#84 | R5-194707 | 0383 | - | F | TS 38.521-3 updates across clause 6 test cases | 15.3.0 |
| 2019-06 | RAN#84 | R5-194708 | 0384 | - | F | TS 38.521-3 updates across clause 7 test cases | 15.3.0 |
| 2019-06 | RAN#84 | R5-194725 | 0388 | - | F | Update to clarify number of LTE CCs config for anchor agnostic EN-DC RF tests | 15.3.0 |
| 2019-06 | RAN#84 | R5-194934 | 0334 | 1 | F | Update of NSA FR1 RF 6.2B.1.1 MOP | 15.3.0 |
| 2019-06 | RAN#84 | R5-194935 | 0335 | 1 | F | Update of NSA FR1 RF 6.2B.1.2 MOP | 15.3.0 |
| 2019-06 | RAN#84 | R5-194936 | 0337 | 1 | F | Update of NSA FR1 RF 7.4B.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194937 | 0338 | 1 | F | Update of NSA FR1 RF 7.4B.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194938 | 0298 | 1 | F | Adding test case 6.5A.2.2, Additional Spectrum emissions mask for CA without EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194941 | 0350 | 1 | F | Introduction of New test case 6.4B.2.3.4 EVM Equalizer Flatness for inter-band EN-DC within FR1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194942 | 0373 | 1 | F | Update of test case 6.2A.2.1, UE maximum output power reduction for inter-band NR CA between FR1 and FR2 without EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194943 | 0374 | 1 | F | Update of 6.2B.3.1 A-MPR Intra-band contiguous for NS_35 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194944 | 0375 | 1 | F | Update of 6.2B.3.2 A-MPR for Intra-band non-contiguous with additional test frequencies | 15.3.0 |
| 2019-06 | RAN#84 | R5-194945 | 0376 | 1 | F | Correction of editorial note in 6.5B.2.1.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194946 | 0377 | 1 | F | Update to 6.2.3 A-MPR FR1 and FR2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194947 | 0389 | 1 | F | Updates to E-UTRA, FR1,FR2 IW tests in line with agreed way forward | 15.3.0 |
| 2019-06 | RAN#84 | R5-194948 | 0369 | 1 | F | Addition of msg content in TC 6.3B.4.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194949 | 0370 | 1 | F | Addition of msg content in TC 6.3B.4.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194950 | 0371 | 1 | F | Addition of msg content in TC 6.3B.4.3 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194951 | 0310 | 1 | F | Update of TC 7.5B.0 ACS for EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-194952 | 0321 | 1 | F | Introduction of NSA FR2 7.6B.2.5 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194953 | 0323 | 1 | F | Update of NSA FR1 7.6B.2.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194954 | 0363 | 1 | F | Updated to EN-DC band and TIB information | 15.3.0 |
| 2019-06 | RAN#84 | R5-194955 | 0385 | 1 | F | TS 38.521-3 clause 5 updates to align with core specification | 15.3.0 |
| 2019-06 | RAN#84 | R5-194975 | 0315 | 1 | F | Introduction of TC 7.5B.4.2 ACS for inter-band FR2 3CC | 15.3.0 |
| 2019-06 | RAN#84 | R5-195046 | 0356 | 1 | F | Update to Wideband Intermodulation for EN-DC in FR1- 2CCs | 15.3.0 |
| 2019-06 | RAN#84 | R5-195047 | 0379 | 1 | F | Update to 6.2.3 A-MPR FR1 and FR2 NSA | 15.3.0 |
| 2019-06 | RAN#84 | R5-195049 | 0357 | 1 | F | Correction to 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-195050 | 0358 | 1 | F | Introducing Wideband Intermodulation for EN-DC including FR1 - 3 CCs | 15.3.0 |
| 2019-06 | RAN#84 | R5-195051 | 0391 | - | F | Update of 6.5B.3.3.2 spurious co-existence inter-band EN-DC FR1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-195091 | 0392 | 1 | F | Update of EN-DC ON_ON time mask test cases | 15.3.0 |
| 2019-06 | RAN#84 | R5-195162 | 0349 | 1 | F | Introduction of New test case 6.4B.2.2.4 EVM Equalizer Flatness for intra-band non-contiguous EN-DC | 15.3.0 |
| 2019-06 | RAN#84 | R5-195163 | 0364 | 1 | F | Clean up FR2 Inter-band EN-DC test cases | 15.3.0 |
| 2019-06 | RAN#84 | R5-195164 | 0390 | 1 | F | Update to UE Maximum Output Power for Inter-Band EN-DC within FR1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-195165 | 0325 | 1 | F | Update of NSA FR1 7.6B.3.0 | 15.3.0 |
| 2019-06 | RAN#84 | R5-195411 | 0386 | 1 | F | TS 38.521-3 updates across clause 6 test cases | 15.3.0 |
| 2019-06 | RAN#84 | R5-195412 | 0387 | 1 | F | TS 38.521-3 updates across clause 7 test cases | 15.3.0 |
| 2019-06 | RAN#84 | R5-195419 | 0393 | 1 | F | EN-DC implementation of FR2 UL demod OTA tests using single pol Rx TE | 15.3.0 |
| 2019-06 | RAN#84 | R5-195436 | 0336 | 1 | F | Update of NSA FR1 RF 6.2B.1.3 MOP | 15.3.0 |
| 2019-06 | RAN#84 | R5-195437 | 0299 | 1 | F | Updates of MU and TT in TS 38.521-3 | 15.3.0 |
| 2019-06 | RAN#84 | R5-195446 | 0394 | 1 | F | Introduction of Tx test cases for FR2 NSA UL CA | 15.3.0 |
| 2019-06 | RAN#84 | R5-193922 | 0340 | - | F | Update of 6.2B.1.3 MOP inter-band adding Rel-16 EN-DC configurations | 16.0.0 |
| 2019-06 | RAN#84 | R5-193923 | 0341 | - | F | Update of 7.3B.2.3 REFSENS inter-band adding Rel-16 EN-DC configurations | 16.0.0 |
| 2019-06 | RAN#84 | R5-194059 | 0345 | - | F | Update of general sections adding Rel-16 EN-DC configurations, Editorial | 16.0.0 |
| 2019-06 | RAN#84 | R5-195054 | 0342 | 1 | F | Update of 6.5B.3.3.2 spurious co-existence inter-band adding Rel-16 EN-DC configurations | 16.0.0 |
| 2019-09 | RAN#85 | R5-197441 | 0397 | 1 | F | Correction of test case numbering for UL CA | 16.1.0 |
| 2019-09 | RAN#85 | R5-197346 | 0398 | 1 | F | Update of UE A_MPR intra band contiguous EN DC test case in 6.2B.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197347 | 0399 | 1 | F | Update of UE A_MPR intra band non contiguous EN DC test case in 6.2B.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197348 | 0401 | 1 | F | Addition of test case 6.5B.2.1.2 Additional Spectrum emissions mask for intra band contiguous EN DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197349 | 0402 | 1 | F | Addition of test case 6.5B.2.2.2 Additional Spectrum emissions mask for intra band non contiguous EN DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197350 | 0403 | 1 | F | Addition of test case 6.5B.2.3.2 Additional Spectrum emissions mask for Inter band EN DC within FR1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197561 | 0404 | 1 | F | Refsens test case updates | 16.1.0 |
| 2019-09 | RAN#85 | R5-197351 | 0405 | 1 | F | Spurious test case updates | 16.1.0 |
| 2019-09 | RAN#85 | R5-197331 | 0406 | 1 | F | Correction to ACLR inter-band EN-DC FR1 test case | 16.1.0 |
| 2019-09 | RAN#85 | R5-196160 | 0407 | - | F | Clean up test cases 6.4B.2.2 Transmit Modulation Quality | 16.1.0 |
| 2019-09 | RAN#85 | R5-196161 | 0408 | - | F | Update to Annex F for Tx modulation quality test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196200 | 0409 | - | F | Correction of uplink power setting for NSA transmitter test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197516 | 0410 | 1 | F | Correction of uplink power setting for NSA receiver test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196241 | 0411 | - | F | Correction to FR1 Transmit OFF Power | 16.1.0 |
| 2019-09 | RAN#85 | R5-197646 | 0412 | 1 | F | Correction to NR power control in FR1 Out-of-band blocking | 16.1.0 |
| 2019-09 | RAN#85 | R5-197352 | 0414 | 1 | F | Correction to FR1 PRACH time mask for EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197353 | 0415 | 1 | F | Correction of NR uplink RB allocation for FR1 Inter-Band EN-DC MOP | 16.1.0 |
| 2019-09 | RAN#85 | R5-196290 | 0416 | - | F | Add Annex F.4 Uplink Power window explanation for interworking test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196296 | 0417 | - | F | Addition of Clause 7.5A in TS 38.521-3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197549 | 0418 | 1 | F | Update of 6.2B.1.1 MOP for Intra-band contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197550 | 0419 | 1 | F | Update of 6.2B.1.2 MOP for Intra-band non-contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-196446 | 0421 | - | F | Editorial update of general sections adding Rel-16 EN-DC configurations | 16.1.0 |
| 2019-09 | RAN#85 | R5-196449 | 0422 | - | F | Correction to description of Table 4.5.1-2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197354 | 0423 | 1 | F | Update for 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197553 | 0424 | 1 | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197333 | 0425 | 1 | F | Update for 7.3B.2.0 Minimum Conformance Requirements of Reference sensitivity for EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197636 | 0426 | 2 | F | Update for 7.3B.2.3 Ref sensitivity for Inter-band EN-DC within FR1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197359 | 0427 | 1 | F | Update for 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197334 | 0428 | 1 | F | Update for 7.3B.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197335 | 0429 | 1 | F | Update for 7.3B.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197337 | 0430 | 1 | F | Update for 7.3B.3.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197338 | 0431 | 1 | F | Updated to EN-DC band information | 16.1.0 |
| 2019-09 | RAN#85 | R5-197336 | 0433 | 1 | F | Update of TCs in 7.6B and 7.7B | 16.1.0 |
| 2019-09 | RAN#85 | R5-197554 | 0434 | 1 | F | Updates to 6.2B.2.1, UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197555 | 0435 | 1 | F | Update of TC 6.3B.1.1 Minimum Output Power for intra-band contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197556 | 0436 | 1 | F | Update of TC 6.3B.1.2 Minimum output power for intra-band non-contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197340 | 0437 | 1 | F | Update of TC 6.3B.2.1 Transmit OFF Power for intra-band contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197341 | 0438 | 1 | F | Update of TC 6.3B.2.2 Transmit OFF Power for intra-band non-contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197342 | 0440 | 1 | F | Update of TC 6.4B.1.1 Frequency error for Intra-band contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197557 | 0441 | 1 | F | Update of TC 6.4B.1.2 Frequency error for Intra-band non-contiguous EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-197343 | 0442 | 1 | F | Update of TC 6.4B.1.3 Frequency error for Inter-band EN-DC within FR1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197647 | 0443 | 1 | F | Correction to 7.8B.2.6 Wideband Intermodulation for EN-DC including FR1 - 3 CCs | 16.1.0 |
| 2019-09 | RAN#85 | R5-197562 | 0448 | 1 | F | Corrections to Reference sensitivity for EN-DC | 16.1.0 |
| 2019-09 | RAN#85 | R5-196842 | 0449 | - | F | Editorial corrections to Additional Spurious Emission test case | 16.1.0 |
| 2019-09 | RAN#85 | R5-197360 | 0450 | 1 | F | Correction to test case 7.4B.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197558 | 0452 | 1 | F | Correction to EN-DC Spurious Emissions | 16.1.0 |
| 2019-09 | RAN#85 | R5-197345 | 0456 | 1 | F | Update OBW EN-DC FR2 test case | 16.1.0 |
| 2019-09 | RAN#85 | R5-197339 | 0457 | 1 | F | TS 38.521-3 Section 5 updates to align with core specification (Covered by CR0431R1) | 16.1.0 |
| 2019-09 | RAN#85 | R5-197542 | 0458 | 1 | F | TS 38.521-3 updates across section 6 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197563 | 0459 | 1 | F | TS 38.521-3 updates across section 7 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197559 | 0460 | 1 | F | Updates to 6.2B.2.4, UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196946 | 0461 | - | F | Corrections on UE maximum output power for DC in 38.521-3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197332 | 0462 | 1 | F | Corrections on Minimum conformance requirements of A-MPR in 38.521-3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196948 | 0463 | - | F | Corrections on clause 5 in 38.521-3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196949 | 0464 | - | F | Corrections on clause 2-4 in 38.521-3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197633 | 0465 | - | F | Removing test points for CP-OFDM PI/2 BPSK in test case 6.5B.2.1.1 | 16.1.0 |
| 2019-12 | RAN#86 | R5-197940 | 0472 | - | F | Addition of 6.2B.1.5 MOP for Inter-Band EN-DC including FR1 and FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197944 | 0475 | - | F | Update of 6.2B.4.1.4 configured transmitted power for Inter-Band EN-DC including FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197945 | 0476 | - | F | Update of 6.2B.4.1.5 configured transmitted power for Inter-Band EN-DC including FR1 and FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197946 | 0477 | - | F | Addition of 7.4A Maximum Input Level for FR1-FR2 CA | 16.2.0 |
| 2019-12 | RAN#86 | R5-197949 | 0480 | - | F | Addition of 7.4B.5 Maximum Input Level for inter-band EN-DC including FR1 and FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198045 | 0485 | - | F | Update of TC 7.5B.0 ACS for EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-198046 | 0486 | - | F | Update of TC 7.5B.1 ACS for intra-band contiguous EN-DC 2CCs | 16.2.0 |
| 2019-12 | RAN#86 | R5-198172 | 0488 | - | F | Updating incorrect note in test procedure | 16.2.0 |
| 2019-12 | RAN#86 | R5-198280 | 0490 | - | F | Introduction of New TC 6.4B.2.1.4 EVM Equalizer Flatness for intra-band contiguous EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-198403 | 0501 | - | F | Correction to TC 6.4B.1.3 test description | 16.2.0 |
| 2019-12 | RAN#86 | R5-198404 | 0502 | - | F | Correction to Additional Spurious Emissions for Inter-band EN-DC within FR1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198406 | 0504 | - | F | Correction to Spurious emission band UE co-existence for Inter-band within FR1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198537 | 0509 | - | F | Updates to 6.2B.3.1, UE A-MPR for Intra-band Contig EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-198538 | 0510 | - | F | Updates to 6.2B.3.2, UE A-MPR for Intra-band Non-Contig EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-198539 | 0511 | - | F | Updates to 6.5B.2.1.2, Additional spectrum emission mask for intra-band Contig EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-198559 | 0512 | - | F | Updated to EN-DC band information Rel-16 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198637 | 0513 | - | F | Updated to EN-DC General clause and band information Rel-15 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198685 | 0515 | - | F | Update single allowed info and MPR test description to EN-DC configuration in 38.521-3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198686 | 0516 | - | F | Corrections on delta TIB for EN-DC configurations in 38.521-3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198722 | 0519 | - | F | Update for 7.3B.1 General | 16.2.0 |
| 2019-12 | RAN#86 | R5-198734 | 0523 | - | F | Update for 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198735 | 0524 | - | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199088 | 0466 | 1 | F | Updates of MU and TT in TS 38.521-3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199342 | 0483 | 1 | F | Updates to test case 6.2B.2.1, UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-199343 | 0487 | 1 | F | Editorial correction of test description in TC 6.4B.2.3.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199344 | 0478 | 1 | F | Addition of 7.4B.3 Maximum Input Level for inter-band EN-DC within FR1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199345 | 0491 | 1 | F | Update of minimum conformance requirements of 7.6B.3 and editorial correction in 7.6B and 7.7B | 16.2.0 |
| 2019-12 | RAN#86 | R5-199346 | 0492 | 1 | F | Addition of 7.6A Blocking Characteristics for CA | 16.2.0 |
| 2019-12 | RAN#86 | R5-199347 | 0493 | 1 | F | Addition of 7.6B.2.3_1 Inband blocking for EN-DC within FR1 (>2 CCs) | 16.2.0 |
| 2019-12 | RAN#86 | R5-199348 | 0495 | 1 | F | Addition of 7.6B.4.3_1 Narrow band blocking for EN-DC within FR1 (>2 CCs) | 16.2.0 |
| 2019-12 | RAN#86 | R5-199349 | 0468 | 1 | F | Add Uplink Power window explanation when centred on a target | 16.2.0 |
| 2019-12 | RAN#86 | R5-199350 | 0489 | 1 | F | Correction and addition of uplink power measurement MUs for NSA FR1 TCs | 16.2.0 |
| 2019-12 | RAN#86 | R5-199351 | 0508 | 1 | F | Addition of MU and TT for NSA 7.6B.2.3_1, 7.6B.2.4, 7.6B.2.4_1 and 7.6B.4.3_1 TCs in F.1.3 and F.3.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199352 | 0529 | 1 | F | TS 38.521-3 Section 1-5 and Annex updates to align with core specification | 16.2.0 |
| 2019-12 | RAN#86 | R5-199377 | 0471 | 1 | F | Update of 6.2B.1.4 MOP for Inter-Band EN-DC including FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199378 | 0479 | 1 | F | Addition of 7.4B.4 Maximum Input Level for inter-band EN-DC including FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199413 | 0482 | 1 | F | Addition of 2A-7A-7A-66A_n66A | 16.2.0 |
| 2019-12 | RAN#86 | R5-199498 | 0500 | 1 | F | Correction to minimum output power for intra-band EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-199506 | 0496 | 1 | F | Removal of 7.7A Spurious Response for CA | 16.2.0 |
| 2019-12 | RAN#86 | R5-199508 | 0484 | 1 | F | Updates to test case 6.2B.2.2, UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-199514 | 0469 | 1 | F | Update of 6.2B.1.1 MOP for Intra-Band contiguous EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-199518 | 0467 | 1 | F | Updates of test procedure for MOP and co-existence tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-199519 | 0517 | 1 | F | Update for 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-199520 | 0518 | 1 | F | Update for 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-199521 | 0527 | 1 | F | Update for Additional Spurious Emissions for Intra-band contiguous EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-199522 | 0530 | 1 | F | TS 38.521-3 updates across section 6 test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-199523 | 0526 | 1 | F | Update for 7.3B.2.0 Minimum Conformance Requirements of Reference sensitivity for EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | R5-199524 | 0531 | 1 | F | TS 38.521-3 updates across section 7 test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-199543 | 0520 | 1 | F | Correction of E-UTRA Mid channel bandwidth | 16.2.0 |
| 2019-12 | RAN#86 | R5-199546 | 0498 | 1 | F | Corrections to DC Config and dual UL interferer | 16.2.0 |
| 2019-12 | RAN#86 | R5-199547 | 0499 | 1 | F | Corrections to EN-DC and NE-DC Configurations | 16.2.0 |
| 2019-12 | RAN#86 | R5-199566 | 0494 | 1 | F | Addition of 7.6B.2.4_1 Inband blocking for inter-band EN-DC including FR2 (>2 CCs) | 16.2.0 |
| 2019-12 | RAN#86 | R5-199567 | 0522 | 1 | F | Update to test case 7.3B.2.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199568 | 0525 | 1 | F | Update for 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199569 | 0528 | 1 | F | Update 4.5 Applicability and test coverage rules | 16.2.0 |
| 2020-03 | RAN#87 | R5-200351 | 0538 | - | F | Updated to EN-DC band information Rel-16 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200446 | 0549 | - | F | Correction to FR1 EN-DC Spurious Test Case | 16.3.0 |
| 2020-03 | RAN#87 | R5-200448 | 0551 | - | F | Correction to LTE specialSubframePatterns in Maximum Output Power Test Case | 16.3.0 |
| 2020-03 | RAN#87 | R5-200449 | 0552 | - | F | Correction to tdm-PatternConfig in FR1 EN-DC Maximum Output Power Test Case | 16.3.0 |
| 2020-03 | RAN#87 | R5-200599 | 0553 | - | F | Update of 6.2B.1.3 on inter-band EN-DC MOP in 38.521-3 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200651 | 0558 | - | F | Updates to 6.2B.2.1, UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200653 | 0559 | - | F | Updates to 6.2B.2.2, UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200657 | 0560 | - | F | Updates to 6.2B.2.3 and 6.2B.3.3 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200668 | 0561 | - | F | Correction of reference clause for 7.3B.2.3 Reference Sensitivity Test Procedure | 16.3.0 |
| 2020-03 | RAN#87 | R5-200670 | 0563 | - | F | Update reference in Test Requirement section of 6.5B.4.3.5 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200728 | 0568 | - | F | Correction of test cases in 7.6B.2 Inband blocking for DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200730 | 0569 | - | F | Correction to reference table number for TC 6.2B.2.3 and TC 6.2B.2.4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200731 | 0570 | - | F | Correction of test cases in 7.6B.3 Out-of-band blocking for DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200732 | 0571 | - | F | Correction of test cases in 7.6B.4 Narrow band blocking for DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200733 | 0572 | - | F | Correction of test cases in 7.7B Spurious response for DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200776 | 0576 | - | F | TS 38.521-3 Section 1-5 and Annex updates to align with core specification | 16.3.0 |
| 2020-03 | RAN#87 | R5-200801 | 0577 | - | F | Update of Non-Standalone FR2 A-MPR test case | 16.3.0 |
| 2020-03 | RAN#87 | R5-200814 | 0579 | - | F | Correction to Test Case 6.5B.3.3.2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200913 | 0534 | 1 | F | Update of Clause 4 in TS 38.521-3 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200925 | 0541 | 1 | F | Adding common uplink configuration of E-UTRA carrier for EN-DC testing | 16.3.0 |
| 2020-03 | RAN#87 | R5-200926 | 0543 | 1 | F | Updating configured output power for inter-band EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200927 | 0545 | 1 | F | Updating configured output power for intra-band contiguous EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200928 | 0547 | 1 | F | Updating configured output power for intra-band non-contiguous EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200929 | 0567 | 1 | F | Core spec alignment of EN-DC MOP and configured output power | 16.3.0 |
| 2020-03 | RAN#87 | R5-200963 | 0536 | 1 | F | Update of TC 7.5B.2 ACS for intra-band non-contiguous EN-DC 2CCs | 16.3.0 |
| 2020-03 | RAN#87 | R5-200964 | 0535 | 1 | F | Update of TC 7.5B.1 ACS for intra-band contiguous EN-DC 2CCs | 16.3.0 |
| 2020-03 | RAN#87 | R5-200982 | 0557 | 1 | F | Updates to 6.5B.2.1.3, Adjacent channel leakage ratio for intra-band contiguous EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200983 | 0575 | 1 | F | Corrections to 6.2B.3.1, UE A_MPR for Intra-band Contig EN_DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200984 | 0533 | 1 | F | Addition of Clause 7.5B.0.3a | 16.3.0 |
| 2020-03 | RAN#87 | R5-200995 | 0578 | 1 | F | Correction of Reference Sensitivity Test Case 7.3B.2.3 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201019 | 0555 | 1 | F | Update of general sections on EN-DC configurations in 38.521-3 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201055 | 0542 | 1 | F | Updating MOP for inter-band EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-201066 | 0564 | 1 | F | Add TX test for Rel16_DC_2_n41 and 66_n41 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201067 | 0565 | 1 | F | Add RX test for Rel16_DC_2_n41 and 66_n41 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201165 | 0574 | 1 | F | Correction of REFSENS for inter-band EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-201187 | 0548 | 1 | F | Addition of a few R16 EN-DC configurations | 16.3.0 |
| 2020-03 | RAN#87 | R5-201193 | 0532 | 1 | F | Updates of MU and TT in TS 38.521-3 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201198 | 0556 | 1 | F | Correcting usage of modifiedMPR-Behaviour | 16.3.0 |
| 2020-06 | RAN#88 | R5-201721 | 0587 | - | F | Update Annex F.4 Uplink Power window explanation | 16.4.0 |
| 2020-06 | RAN#88 | R5-201855 | 0594 | - | F | Update of 6.2B.1.3 MOP for inter-band EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-201859 | 0596 | - | F | Updating A-SE to align test points with A-MPR | 16.4.0 |
| 2020-06 | RAN#88 | R5-201870 | 0598 | - | F | Update of test configuration in 6.5B.3.3.2 for UE co-existence | 16.4.0 |
| 2020-06 | RAN#88 | R5-201942 | 0605 | - | F | Updated to EN-DC Rel-16 band information in clause 5 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202032 | 0611 | - | F | Correction of Initial condition for OOBB intra-band contiguous EN-DC 2 CCs in FR1 in TC 7.6B.3.1 R15 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202293 | 0626 | - | F | Correction to 7.3B.2.3 test configuration for EN-DC 26_n79 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202294 | 0627 | - | F | Correction to Frequency error for Intra-band contiguous EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202295 | 0628 | - | F | Correction to PRACH configuration for intra-band EN-DC tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202393 | 0630 | - | F | Addition of Intra-band non-contiguous EN-DC combination | 16.4.0 |
| 2020-06 | RAN#88 | R5-202421 | 0633 | - | F | Update Uplink power control window size for NSA TX TCs | 16.4.0 |
| 2020-06 | RAN#88 | R5-202429 | 0637 | - | F | Update for 7.3A.3 for CA without EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202475 | 0638 | - | F | TS 38.521-3 Section 1-5 and Annex updates to align with core specification | 16.4.0 |
| 2020-06 | RAN#88 | R5-202725 | 0585 | 1 | F | Correction on the channel bandwidth to use in section 6.5B.3.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202726 | 0586 | 1 | F | Setting p-MaxEUTRA to avoid limiting NR power in section 6.5B | 16.4.0 |
| 2020-06 | RAN#88 | R5-202727 | 0590 | 1 | F | Correction to TC 6.2B.4.1.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202728 | 0595 | 1 | F | Updating 6.2B.4.x configured output power test cases for FR1 EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202729 | 0614 | 1 | F | correction 6.3B.1.3 to include anchor agnostic approach applies | 16.4.0 |
| 2020-06 | RAN#88 | R5-202730 | 0616 | 1 | F | Clarification of disabling Tx diversity for FR2 UE for NSA FR2 testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202731 | 0618 | 1 | F | Updation of 6.5B.3.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202732 | 0639 | 1 | F | Addition of UL-MIMO EN-DC tests in Clause 6 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202733 | 0641 | 1 | F | Common updates across tests in Clause 6 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202734 | 0640 | 1 | F | Addition of UL-MIMO EN-DC tests in Clause 7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202735 | 0642 | 1 | F | Common updates across tests in Clause 7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202782 | 0635 | 1 | F | Update Rx TC for 5 Rel_16_DC_combos | 16.4.0 |
| 2020-06 | RAN#88 | R5-202809 | 0632 | 1 | F | Receiver characteristics testing update to 38.521-3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202829 | 0601 | 1 | F | Addition of 6.2B.1.5D UE Maximum Output Power for Inter-Band EN-DC including both FR1 and FR2 for UL-MIMO | 16.4.0 |
| 2020-06 | RAN#88 | R5-202830 | 0602 | 1 | F | Addition of 6.2B.5.1.1 Configured Output Power Level for Inter-band NR-DC between FR1 and FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202901 | 0631 | 1 | F | Power control for EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202902 | 0588 | 1 | F | Updated structure for RefSens for EN-DC within FR1 more than 2CCs | 16.4.0 |
| 2020-06 | RAN#88 | R5-202903 | 0597 | 1 | F | Updating REFSENS for 1A_n78A and 3A_n78A | 16.4.0 |
| 2020-06 | RAN#88 | R5-202904 | 0613 | 1 | F | Introduction of Rx test cases for EN-DC within FR1 3CCs | 16.4.0 |
| 2020-06 | RAN#88 | R5-202905 | 0629 | 1 | F | Update to NSA FR2 Receiver Spurious Emission Test Case | 16.4.0 |
| 2020-06 | RAN#88 | R5-202906 | 0636 | 1 | F | Add 7.3B.2.3_1.2 Reference sensitivity for EN-DC within FR1v - 4 CCs | 16.4.0 |
| 2020-06 | RAN#88 | R5-202907 | 0624 | 1 | F | Updates of FR2 MU and TT in TS 38.521-3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202925 | 0634 | 1 | F | Update Tx TC for 5 Rel_16_DC_combos | 16.4.0 |
| 2020-06 | RAN#88 | R5-202934 | 0604 | 1 | F | Addition of TDD-TDD PC2 inter-band EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202945 | 0592 | 1 | F | Updating 6.2B.1.1 MOP for intra-band contiguous EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202946 | 0593 | 1 | F | Updating 6.2B.1.2 MOP for intra-band non-contiguous EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202947 | 0606 | 1 | F | Updates to 6.2B.2.1, UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202948 | 0607 | 1 | F | Updates to 6.5B.2.1.1, Spectrum emissions mask for intra-band contiguous EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202949 | 0608 | 1 | F | Updates to 6.5B.2.1.3, Adjacent channel leakage ratio for intra-band contiguous EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202959 | 0620 | 1 | F | Updates of Tx test cases for EN-DC including FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202964 | 0622 | 1 | F | Update to configured output power relaxation due to inter-band EN-DC in 38.521-3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202965 | 0623 | 1 | F | Update to REFSENS relaxation due to inter-band EN-DC in 38.521-3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202991 | 0609 | 1 | F | Completion of TC 7.6B.3.3 OOBB Inter-band EN-DC 2CCs within FR1 R15 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202992 | 0610 | 1 | F | Completion of TC 7.7B.3 Spurious Response Inter-band EN-DC 2CCs within FR1 R15 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202993 | 0612 | 1 | F | Alignment of requirements in 7.6B and 7.7B with core spec for Intra-band contiguous EN-DC in FR1 R15 | 16.4.0 |
| 2020-09 | RAN#89 | R5-203227 | 0644 | - | F | Adding receiver requirements for Rel-16 inter-band EN-DC FR1 band combinations | 16.5.0 |
| 2020-09 | RAN#89 | R5-203294 | 0645 | - | F | Correction of MU definition for test case 6.5B.2.1.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203295 | 0646 | - | F | definition of MU and TT for test case 6.3B.4.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203296 | 0647 | - | F | definition of MU and TT for test case 6.5B.4.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203297 | 0648 | - | F | Definition of MU and TT for test case 7.3B.2.4 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203492 | 0650 | - | F | In-band Blocking FR2 MU updates in 38.521-3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203515 | 0651 | - | F | Editorial correction of test case 6.5B.3.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203637 | 0655 | - | F | Introduction of additional Rel-16 EN-DC inter-band configurations to EN-DC MOP test case 6.2B.1.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203753 | 0660 | - | F | Update test procedure to 6.5B.3.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203763 | 0661 | - | F | Editorial correction of referenced clause (6.2.3) in test case 6.2B.2.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203872 | 0668 | - | F | Adding RRCConnectionReconfiguration Table for E-UTRA on FDD band and UE does not support dynamic power sharing in 6.5B.3.3.1 and 6.5B.3.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203907 | 0669 | - | F | Corrections on delta RIB due to Rel-16 EN-DC configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-203941 | 0674 | - | F | Addition of the test description to apply LTE agnostic approach in 7.4B.2 MIL | 16.5.0 |
| 2020-09 | RAN#89 | R5-203960 | 0680 | - | F | Correction to additional test points for spurious emission UE co-existence for a few Rel-15 inter-band EN-DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203973 | 0684 | - | F | Cleaning up of delta_TIB | 16.5.0 |
| 2020-09 | RAN#89 | R5-204154 | 0693 | - | F | Updated to Annex M for EN-DC Rel-16 configuration requirement | 16.5.0 |
| 2020-09 | RAN#89 | R5-204170 | 0697 | - | F | Update for TC 7.5B.0 Minimum Conformance Requirements of ACS for DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204176 | 0703 | - | F | Update for TC 7.5B.4 ACS for inter-band EN-DC including FR2 2CCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204177 | 0704 | - | F | Update for TC 7.5B.4_1.1 ACS for inter-band EN-DC including FR2 3CCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204178 | 0705 | - | F | Update for TC 7.5B.4_1.2 ACS for inter-band EN-DC including FR2 4CCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204179 | 0706 | - | F | Update for TC 7.5B.4_1.3 ACS for inter-band EN-DC including FR2 5CCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204180 | 0707 | - | F | Update for TC 7.5B.4_1.4 ACS for inter-band EN-DC including FR2 6CCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204198 | 0713 | - | F | Update for 7.3B.2.0 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204199 | 0714 | - | F | Update for Spurious Emissions for intra-band EN-DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204209 | 0717 | - | F | Update Uplink power control window size for NSA RX TCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204210 | 0718 | - | F | Update Tx TC for Rel-16 DC combos | 16.5.0 |
| 2020-09 | RAN#89 | R5-204250 | 0721 | - | F | FR2 Minimum output power measurement period definition | 16.5.0 |
| 2020-09 | RAN#89 | R5-204296 | 0726 | - | F | Addition of test case 6.4B.2.4.1D | 16.5.0 |
| 2020-09 | RAN#89 | R5-204297 | 0727 | - | F | Addition of test case 6.4B.2.4.2D | 16.5.0 |
| 2020-09 | RAN#89 | R5-204298 | 0728 | - | F | Addition of test case 6.4B.2.4.3D | 16.5.0 |
| 2020-09 | RAN#89 | R5-204299 | 0729 | - | F | Addition of test case 6.4B.2.4.4D | 16.5.0 |
| 2020-09 | RAN#89 | R5-204310 | 0730 | - | F | Update of test case 6.2B.3.1 UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204313 | 0733 | - | F | TS 38.521-3 Section 1-5 and Annex updates to align with core specification | 16.5.0 |
| 2020-09 | RAN#89 | R5-204322 | 0736 | - | F | Update for TC 7.5B.5 ACS for inter-band EN-DC including both FR1 and FR2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204349 | 0738 | - | F | Editorial correction of Rx spurious emissions for FR1 EN-DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204705 | 0739 | - | F | Addition of test configuration for 71AA in 6.2B.1.1 MOP | 16.5.0 |
| 2020-09 | RAN#89 | R5-204707 | 0740 | - | F | Addition of MOP testing for DC_28A_n3A in 6.2B.1.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204719 | 0715 | 1 | F | Update for 7.3B.2.3_1.3 Reference sensitivity for EN-DC within FR1 5CCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204724 | 0662 | 1 | F | Updated to clause 6 for EN-DC Rel-16 band requirement | 16.5.0 |
| 2020-09 | RAN#89 | R5-204766 | 0665 | 1 | F | Spurious inter-band EN-DC FR2 UL MIMO test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204767 | 0673 | 1 | F | Addition of test requirement of dropped NR carrier for non-DPS UE in 6.2B.4.x | 16.5.0 |
| 2020-09 | RAN#89 | R5-204768 | 0678 | 1 | F | Addition of additional test points for spurious emission UE co-existence for a few Rel-15 inter-band EN-DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204769 | 0723 | 1 | F | Editorial correction to EN-DC co-existence requirements | 16.5.0 |
| 2020-09 | RAN#89 | R5-204770 | 0664 | 1 | F | FR2 RefSens inter-band EN-DC UL MIMO test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204771 | 0670 | 1 | F | Update of 7.3B.3.3 for REFSENS relaxation due to inter-band EN-DC within FR1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204772 | 0676 | 1 | F | Addition of new test case 7.4B.5D MIL for inter-band EN-DC including FR1 and FR2 for UL-MIMO | 16.5.0 |
| 2020-09 | RAN#89 | R5-204773 | 0709 | 1 | F | Addition of TC 7.5B.5D ACS for inter-band EN-DC including FR1 and FR2 for UL-MIMO | 16.5.0 |
| 2020-09 | RAN#89 | R5-204809 | 0643 | 1 | F | Adding transmitter requirements for Rel-16 inter-band EN-DC FR1 band combinations | 16.5.0 |
| 2020-09 | RAN#89 | R5-204810 | 0654 | 1 | F | Editorial correction to spurious emission test case 6.5B.3.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204811 | 0656 | 1 | F | Introduction of dTIBc for additional Rel-16 EN-DC inter-band configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-204812 | 0657 | 1 | F | Introduction of Rel-16 EN-DC configuration DC_40A_n1A to spurious emission test case 6.5B.3.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204813 | 0658 | 1 | F | Introduction of Rel-16 EN-DC configuration DC_40A_n78A to spurious emission test case 6.5B.3.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204814 | 0679 | 1 | F | Addition of additional test points for spurious emission UE co-existence for a few Rel-16 inter-band EN-DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204815 | 0694 | 1 | F | Updated to clause 6 for EN-DC Rel-16 configuration test requirement | 16.5.0 |
| 2020-09 | RAN#89 | R5-204816 | 0692 | 1 | F | Updated to EN-DC Rel-16 Configuration information in clause 5 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204840 | 0696 | 1 | F | Updated to EN-DC general clauses for NRSL eV2X | 16.5.0 |
| 2020-09 | RAN#89 | R5-204866 | 0689 | 1 | F | CR to update MU and TT in 38.521-3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204867 | 0716 | 1 | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence | 16.5.0 |
| 2020-09 | RAN#89 | R5-204869 | 0649 | 1 | F | Adjacent Channel Selectivity FR2 MU and TT updates in 38.521-3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204924 | 0666 | 1 | F | Update of Test applicability for some Inter-Band EN-DC including FR2 TCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204925 | 0687 | 1 | F | Clean up complete status for EN-DC test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204926 | 0731 | 1 | F | Addition of pending UL-MIMO tests (FR2) in Clause 6 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204928 | 0681 | 1 | F | Update of NSA Rx test cases for 4Rx UEs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204929 | 0700 | 1 | F | Addition of TC 7.5B.3_1.2 ACS for EN-DC within FR1 4CC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204930 | 0711 | 1 | F | Addition of TC 7.6B.2.5D Inband blocking for inter-band EN-DC including FR1 and FR2 for UL-MIMO | 16.5.0 |
| 2020-09 | RAN#89 | R5-204931 | 0712 | 1 | F | Update for 7.3B.2.3 Ref sense for Inter-band EN-DC within FR1-2 CCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204932 | 0719 | 1 | F | Re-organization of EN-DC refsens test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204957 | 0695 | 1 | F | Updated to clause 7 for EN-DC Rel-16 configuration test requirement | 16.5.0 |
| 2020-09 | RAN#89 | R5-204958 | 0737 | 1 | F | Add Rx TC for 2Rel 16 DC combos | 16.5.0 |
| 2020-09 | RAN#89 | R5-205005 | 0741 | - | F | Update of OOBB and Spurious Response of Inter-band EN-DC within FR1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-205006 | 0690 | 1 | F | Update Refsense exceptions for EN-DC including n78 | 16.5.0 |
| 2020-12 | RAN#90 | R5-205489 | 0749 | - | F | Correction to Reference Sensitivity for Inter-band EN-DC | 16.6.0 |
| 2020-12 | RAN#90 | R5-205500 | 0750 | - | F | Minor correction of section 6.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205501 | 0751 | - | F | Correction of spec style in section 7.6B and 7.7B and correction of table number in 7.6B.3.3.4.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205502 | 0752 | - | F | Addition of intra-band contiguous EN-DC testing and update of inter-band EN-DC testing for 7.6B.2.3_1.1 Inband blocking for EN-DC within FR1 3 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205503 | 0753 | - | F | Addition of intra-band contiguous EN-DC testing and update of inter-band EN-DC testing for 7.6B.2.3_1.2 Inband blocking for EN-DC within FR1 4 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205505 | 0755 | - | F | Addition of new test case 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 3 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205506 | 0756 | - | F | Addition of new test case 7.6B.3.3_1.2 Out-of-band blocking for EN-DC within FR1 4 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205507 | 0757 | - | F | Addition of intra-band contiguous EN-DC testing and update of inter-band EN-DC testing for 7.6B.4.3_1.1 Narrow band blocking for EN-DC within FR1 3 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205508 | 0758 | - | F | Addition of intra-band contiguous EN-DC testing and update of inter-band EN-DC testing for 7.6B.4.3_1.2 Narrow band blocking for EN-DC within FR1 4 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205510 | 0760 | - | F | Addition of new test case 7.7B.3_1.1 Spurious Response for EN-DC within FR1 3 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205511 | 0761 | - | F | Addition of new test case 7.7B.3_1.2 Spurious Response for EN-DC within FR1 4 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205515 | 0763 | - | F | Correction of section 7.1 and update of 7.6B.3.0.3A and 7.7B.0.3A as per RAN4 spec | 16.6.0 |
| 2020-12 | RAN#90 | R5-205516 | 0764 | - | F | Correction of uplink power and in gap test requirement for 7.6B.2.2, 7.6B.3.2, 7.6B.4.2 and 7.7B.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205534 | 0768 | - | F | Editorial, removing duplication of text in test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-205568 | 0776 | - | F | Introduction of DC_3A-40A_n1A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-205678 | 0778 | - | F | Addition of Clause 7.5B.4D ACS for inter-band EN-DC including FR2 for UL-MIMO | 16.6.0 |
| 2020-12 | RAN#90 | R5-205679 | 0779 | - | F | Addition of Clause 7.6B.2.4D Inband blocking for inter-band EN-DC including FR2 for UL-MIMO | 16.6.0 |
| 2020-12 | RAN#90 | R5-205696 | 0782 | - | F | Editorial correction to EN-DC test cases 6.5B.2.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205775 | 0785 | - | F | Adding delta TIB for a few Rel-16 inter-band EN-DC configurations within FR1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205776 | 0786 | - | F | Adding refsens exceptions for DC_3_n78 due to receiver harmonic mixing | 16.6.0 |
| 2020-12 | RAN#90 | R5-205778 | 0788 | - | F | Adding delta RIB for DC_2-7-66_n78 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205784 | 0790 | - | F | Update of UE co-existence spurious emissions for Rel-16 inter-band EN-DC DC_13_n66 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205788 | 0792 | - | F | Correction to UE co-existence spurious emissions for inter-band EN-DC within FR1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205803 | 0793 | - | F | Introduction of New TC 6.4B.2.4.2_1.1 Carrier Leakage for inter-band EN-DC including FR2 with 3 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205804 | 0794 | - | F | Introduction of New TC 6.4B.2.4.2_1.2 Carrier Leakage for inter-band EN-DC including FR2 with 4 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205806 | 0795 | - | F | Introduction of New TC 6.4B.2.4.2_1.3 Carrier Leakage for inter-band EN-DC including FR2 with 5 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-205855 | 0798 | - | F | Correction of channel bandwidth for EN-DC MOP TC 6.2B.1.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205911 | 0803 | - | F | Introduction of DC_3A-20A_n78A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-205934 | 0805 | - | F | Update of DC_1A-7A_n78A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-205935 | 0806 | - | F | Update DC_7A-20A_n78A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-205994 | 0808 | - | F | Updates to Reference sensitivity for EN-DC within FR1 3CC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206018 | 0811 | - | F | Correction of 7.3B.3.3 for delta RIB with inter-band EN-DC configurations | 16.6.0 |
| 2020-12 | RAN#90 | R5-206029 | 0813 | - | F | Update spurious emission band UE co-existence for DC_2_n41 and DC_66_n41 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206236 | 0832 | - | F | Correction to Rel-16 EN-DC configuration DC_40A_n1A and DC_40A_n78 in spurious emission test case 6.5B.3.3.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206237 | 0833 | - | F | Introduction of Rel-16 EN-DC configuration DC_1A_n3A to spurious emission test case 6.5B.3.3.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206238 | 0834 | - | F | Introduction of Rel-16 EN-DC configuration DC_20A_n3A to spurious emission test case 6.5B.3.3.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206255 | 0835 | - | F | Update for 6.5B.3.2.2 Spurious emission band UE co-existence for intra-band non-contiguous EN-DC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206648 | 0766 | 1 | F | Addition of TC6.3B.1.4D Minimum output power for inter-band EN-DC including FR2 for UL-MIMO | 16.6.0 |
| 2020-12 | RAN#90 | R5-206649 | 0767 | 1 | F | Addition of TC6.3B.3.4 Transmit ON/OFF time mask for inter-band EN-DC including FR2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206650 | 0777 | 1 | F | Update on Transmit OFF power for EN-DC including FR2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206651 | 0789 | 1 | F | Update of UE co-existence spurious emissions for a few Rel-15 inter-band EN-DC configurations | 16.6.0 |
| 2020-12 | RAN#90 | R5-206652 | 0796 | 1 | F | Addition of 6.5B.2.4D.3 Adjacent channel leakage ratio for inter-band EN-DC including FR2 for UL-MIMO | 16.6.0 |
| 2020-12 | RAN#90 | R5-206653 | 0799 | 1 | F | Update of 6.2B.1.4_1 MOP for Inter-Band EN-DC including FR2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206654 | 0821 | 1 | F | Update for 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206655 | 0822 | 1 | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206656 | 0780 | 1 | F | Update for Clause F.1.3 Measurement of receiver | 16.6.0 |
| 2020-12 | RAN#90 | R5-206657 | 0810 | 1 | F | Correction of 7.3B.3.3 for allowed reference sensitivity relaxation | 16.6.0 |
| 2020-12 | RAN#90 | R5-206658 | 0819 | 1 | F | Update the interferer range for OOBB and Spurious Response of Inter-band EN-DC within FR1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206659 | 0823 | 1 | F | Update for 7.3B.2.0 Minimum Conformance Requirements of Reference sensitivity for EN-DC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206660 | 0826 | 1 | F | Editor’s notes updates for FR1 inter-band 3CC DL CA | 16.6.0 |
| 2020-12 | RAN#90 | R5-206661 | 0762 | 1 | F | Addition of measurement uncertainties and test tolerance for blocking test cases for EN-DC within FR1 with 3CCs, 4CCs and 5CCs and correction of some spec styles | 16.6.0 |
| 2020-12 | RAN#90 | R5-206662 | 0817 | 1 | F | 6.4B.2.1.1 Error Vector Magnitude for intra-band contiguous EN DC Measurement Uncertainty update | 16.6.0 |
| 2020-12 | RAN#90 | R5-206663 | 0818 | 1 | F | 6.4B.2.1.2 Carrier Leakage for intra-band contiguous EN-DC Measurement Uncertainty update | 16.6.0 |
| 2020-12 | RAN#90 | R5-206664 | 0820 | 1 | F | FR1 Intra-band CA General Tx ON OFF time mask measurement uncertainties and test tolerances corrections | 16.6.0 |
| 2020-12 | RAN#90 | R5-206665 | 0836 | 1 | F | TS 38.521-3 Section 1-5 and Annex updates to align with core specification | 16.6.0 |
| 2020-12 | RAN#90 | R5-206720 | 0742 | 1 | F | Introduction of New test case 6.3B.1.4_1.1 Minimum output power for inter-band EN-DC including FR2 - 3 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206721 | 0743 | 1 | F | Introduction of New test case 6.3B.1.4_1.2 Minimum output power for inter-band EN-DC including FR2 - 4 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206722 | 0744 | 1 | F | Introduction of New test case 6.3B.1.4_1.3 Minimum output power for inter-band EN-DC including FR2 - 5 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206723 | 0745 | 1 | F | Introduction of New test case 6.4B.2.4.1_1.1 Error Vector Magnitude for inter-band EN-DC including FR2 - 3 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206724 | 0746 | 1 | F | Introduction of New test case 6.4B.2.4.1_1.2 Error Vector Magnitude for inter-band EN-DC including FR2 - 4 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206725 | 0747 | 1 | F | Introduction of New test case 6.4B.2.4.1_1.3 Error Vector Magnitude for inter-band EN-DC including FR2 - 5 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206727 | 0787 | 1 | F | Adding refsens exceptions for DC_3_n1 due to cross band isolation | 16.6.0 |
| 2020-12 | RAN#90 | R5-206728 | 0783 | 1 | F | Update to EN-DC R16 Configuration information in clause 5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206772 | 0781 | 1 | F | Addition of PC2 Tx requirements for DC_3A_n41A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206773 | 0801 | 1 | F | Addition of PC2 Tx requirements for DC_3A_n78A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206784 | 0802 | 1 | F | Updating Rel-16 EN-DC PC2 MOP to add UE power class capability for NR part | 16.6.0 |
| 2020-12 | RAN#90 | R5-206827 | 0829 | 1 | F | Minimum output power editor's notes | 16.6.0 |
| 2020-12 | RAN#90 | R5-206828 | 0828 | 1 | F | Update FR2 TRx MU and TT in 38.521-3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206869 | 0809 | 1 | F | Updates to Reference sensitivity for EN-DC within FR1 4CC re-structuring test configuration tables | 16.6.0 |
| 2020-12 | RAN#90 | R5-206870 | 0825 | 1 | F | Measurement uncertainties and test tolerances updates for FR1 inter-band 3CC DL CA | 16.6.0 |
| 2020-12 | RAN#90 | R5-206883 | 0772 | 1 | F | Introduction of DC_3A_n1A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206884 | 0773 | 1 | F | Introduction of DC_1A_n3A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206885 | 0774 | 1 | F | Introduction of DC_20A_n3A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206886 | 0775 | 1 | F | Introduction of DC_40A_n78A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206902 | 0769 | 1 | F | Moving test configurations table for EN-DC 20_n78 to a general test configuration table for 2CC reference sensitivity exceptions | 16.6.0 |
| 2020-12 | RAN#90 | R5-206903 | 0770 | 1 | F | Introduction of DC_1A_n78A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206904 | 0771 | 1 | F | Introduction of DC_3A_n78A to reference sensitivity test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206913 | 0754 | 1 | F | Update of 7.6B.2.3_1.3 Inband blocking for EN-DC within FR1 5 CCs and removal of 7.6B.2.3_1.4 Inband blocking for EN-DC within FR1 6 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206914 | 0759 | 1 | F | Update of 7.6B.4.3_1.3 Narrow band blocking for EN-DC within FR1 5 CCs and removal of 7.6B.4.3_1.4 Narrow band blocking for EN-DC within FR1 6 CCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206915 | 0807 | 1 | F | Updates to Reference sensitivity for EN-DC within FR1 2CC for UE supporting single UL | 16.6.0 |
| 2020-12 | RAN#90 | R5-206916 | 0827 | 1 | F | Update for 7.3B.3.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206918 | 0804 | 1 | F | Addition of PC2 Rx requirements for DC_3A_n41A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206919 | 0814 | 1 | F | Addition of PC2 ENDC DC_3A-n78A into TC7.3B.2.3 | 16.6.0 |
| 2021-03 | RAN#91 | R5-210093 | 0842 | - | F | Introduction of DC_7A-20A_n3A to reference sensitivity test | 16.7.0 |
| 2021-03 | RAN#91 | R5-210301 | 0847 | - | F | Completion of OBW intra-band non-contiguous test 6.5B.1.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210302 | 0848 | - | B | Addition of new test case 6.5B.1.4D OBW for inter-band EN-DC FR2 UL MIMO | 16.7.0 |
| 2021-03 | RAN#91 | R5-210387 | 0857 | - | F | Correction to EN-DC OoB emissions | 16.7.0 |
| 2021-03 | RAN#91 | R5-210415 | 0858 | - | F | Addition of new test case 6.4B.2.4.3_1.1 In-band Emissions for inter-band EN-DC including FR2 with 3 CCs | 16.7.0 |
| 2021-03 | RAN#91 | R5-210416 | 0859 | - | F | Addition of new test case 6.4B.2.4.3_1.2 In-band Emissions for inter-band EN-DC including FR2 with 4 CCs | 16.7.0 |
| 2021-03 | RAN#91 | R5-210417 | 0860 | - | F | Addition of new test case 6.4B.2.4.3_1.3 In-band Emissions for inter-band EN-DC including FR2 with 5 CCs | 16.7.0 |
| 2021-03 | RAN#91 | R5-210497 | 0861 | - | F | Correction of test frequencies for NR band n28 30MHz test channel bandwidth of 6.2B.1.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210498 | 0862 | - | F | Addition of editor note to the incomplete test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210499 | 0863 | - | F | Correction of test applicability of 6.5B.5.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210500 | 0864 | - | F | Correction of test configuration tables in section 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210501 | 0865 | - | F | Completion of 7.6B.2.3_1.3 Inband blocking for EN-DC within FR1 5 CCs | 16.7.0 |
| 2021-03 | RAN#91 | R5-210502 | 0866 | - | F | Correction of test frequencies for NR band n28 30MHz test channel bandwidth of 7.6B.3.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210548 | 0872 | - | F | Default message exceptions for LTE carriers in EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-210725 | 0874 | - | F | Omitting of NSA Rx cases with UL-MIMO on TDD bands | 16.7.0 |
| 2021-03 | RAN#91 | R5-210909 | 0882 | - | F | Editorial correction to test case 6.2B.4.1.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210943 | 0883 | - | F | Adding delta TIB and delta RIB for DC_2-7-7-66_n78 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210989 | 0889 | - | F | Update for 7.3B.2.0 Minimum Conformance Requirements of Reference sensitivity for EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211005 | 0893 | - | F | Update to EN-DC R16 Configuration information in clause 5 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211010 | 0895 | - | F | Update of reference sensitivity for intra-band contiguous EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211014 | 0899 | - | F | Adding in-gap tests to ACS for intra-band non-contiguous EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211017 | 0901 | - | F | Update of test coverage for reference sensitivity for 3CC EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211020 | 0902 | - | F | Adding EN-DC configurations DC_1A-28A_n3A and DC_7A-28A_n3A to clause 5.5B.4.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211099 | 0917 | - | F | Correction to editors note about number of E-UTRA carriers | 16.7.0 |
| 2021-03 | RAN#91 | R5-211111 | 0921 | - | F | Corrections to subclauses in 38.521-3 with appropriate subclause level and heading styles | 16.7.0 |
| 2021-03 | RAN#91 | R5-211125 | 0922 | - | F | Update of 5.3B for UE channel bandwidth for EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211240 | 0934 | - | F | Introduction of DC_7A-20A_n1A to reference sensitivity test | 16.7.0 |
| 2021-03 | RAN#91 | R5-211241 | 0935 | - | F | Introduction of DC_7A- 28A_n3A to referce sensitivity test | 16.7.0 |
| 2021-03 | RAN#91 | R5-211694 | 0844 | 1 | F | Spectrum emissions mask for intra-band non-contiguous EN-DC Test Definition | 16.7.0 |
| 2021-03 | RAN#91 | R5-211695 | 0849 | 1 | F | ACLR for intra-band non-contiguous EN-DC Test Definition | 16.7.0 |
| 2021-03 | RAN#91 | R5-211696 | 0856 | 1 | F | Update Test description of 6.5B.1.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211697 | 0871 | 1 | F | EN-DC FR2 UL CA Frequency error test cases update | 16.7.0 |
| 2021-03 | RAN#91 | R5-211698 | 0876 | 1 | F | Correction of test requirements for EN-DC configured output power | 16.7.0 |
| 2021-03 | RAN#91 | R5-211699 | 0906 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_8A_n77A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211700 | 0907 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_11A_n77A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211701 | 0908 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_11A_n78A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211702 | 0909 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_11A_n79A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211703 | 0910 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_25A_n41A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211704 | 0911 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_26A_n41A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211705 | 0912 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_26A_n77A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211706 | 0913 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_26A_n78A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211707 | 0914 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_26A_n79A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211708 | 0915 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_41A_n77A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211709 | 0916 | 1 | F | Introduction of Rel-15 EN-DC configuration DC_41A_n78A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211710 | 0853 | 1 | F | Correction of MSD test point on Table 7.3B.2.0.3.5.2-1 DC_1A-8A_n78A | 16.7.0 |
| 2021-03 | RAN#91 | R5-211711 | 0854 | 1 | F | Update of 7.3B.2.3_1.1 RefSens DC_3A-8A_n78A | 16.7.0 |
| 2021-03 | RAN#91 | R5-211712 | 0855 | 1 | F | Addition of new test case 7.3B.4 for EIS Spherical Coverage | 16.7.0 |
| 2021-03 | RAN#91 | R5-211713 | 0867 | 1 | F | Editorial correction for errors in 7.6B.4.3_1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211714 | 0868 | 1 | F | Correction of test configuration tables in section 7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211715 | 0877 | 1 | F | Clarification of tested Rx antenna numbers on E-UTRA band | 16.7.0 |
| 2021-03 | RAN#91 | R5-211766 | 0837 | 1 | F | Introduction of Rel-16 EN-DC configuration DC_7A_n3A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211767 | 0838 | 1 | F | Introduction of Rel-16 EN-DC configuration DC_8A_n3A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211768 | 0839 | 1 | F | Introduction of Rel-16 EN-DC configuration DC_20A_n1A to spurious emission test case 6.5B.3.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211769 | 0887 | 1 | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence_Rel16 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211770 | 0903 | 1 | F | Adding Delta TIB,c for DC_1A-28A_n3A, DC_7A-20A_n1A and DC_7A-28A_n3A to clause 6.2B.4.2.3.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211771 | 0840 | 1 | F | Introduction of DC_7A_n3A to reference sensitivity test | 16.7.0 |
| 2021-03 | RAN#91 | R5-211772 | 0841 | 1 | F | Introduction of DC_8A_n1A and DC_8A_n3A to reference sensitivity test | 16.7.0 |
| 2021-03 | RAN#91 | R5-211773 | 0869 | 1 | F | Adding Inter-band EN-DC combination within FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211840 | 0878 | 1 | F | Updating Rel-16 EN-DC PC2 MOP to include powerClassNRPart-r16 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211841 | 0879 | 1 | F | Updating Rel-16 EN-DC PC2 MPR to include powerClassNRPart-r16 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211842 | 0880 | 1 | F | Updating Rel-16 EN-DC PC2 A-MPR to include powerClassNRPart-r16 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211843 | 0881 | 1 | F | Updating Rel-16 EN-DC PC2 configured output power to include powerClassNRPart-r16 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211852 | 0888 | 1 | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence | 16.7.0 |
| 2021-03 | RAN#91 | R5-211854 | 0890 | 1 | F | Correction to EN-DC Wideband Intermodulation tests | 16.7.0 |
| 2021-03 | RAN#91 | R5-211870 | 0843 | 1 | F | Editorial addition of editors notes in 6.3B.8.1.4, 6.3B.8.2.4 and 6.3B.8.3.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211871 | 0875 | 1 | F | Correcting EN-DC A-MPR test requirements for non-overlapping test points | 16.7.0 |
| 2021-03 | RAN#91 | R5-211872 | 0886 | 1 | F | Correction to the TDM pattern configuration for EN-DC Tx test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211873 | 0918 | 1 | F | Correction to MOP and MPR test procedures for PC2 in TC 6.2B.1.3 and 6.2B.2.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211874 | 0845 | 1 | F | Correction of LTE frequency for 19-n79 combo in 7.3B.2.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211875 | 0870 | 1 | F | Update to EN-DC Reference Sensitivity | 16.7.0 |
| 2021-03 | RAN#91 | R5-211876 | 0896 | 1 | F | Update of reference sensitivity for intra-band non-contiguous EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211877 | 0897 | 1 | F | Update of reference sensitivity for inter-band 2CC EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211878 | 0898 | 1 | F | Correction to refsens test requirements for DC_1A-7A_n78A | 16.7.0 |
| 2021-03 | RAN#91 | R5-211879 | 0900 | 1 | F | Update of test configuration for inter-band 2CC EN-DC configurations affected by reference sensitivity exceptions | 16.7.0 |
| 2021-03 | RAN#91 | R5-211880 | 0924 | 1 | F | Update of 2CC refsens test case 7.3B.2.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211881 | 0925 | 1 | F | Update of 3CC refsens test case 7.3B.2.3_1.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211882 | 0926 | 1 | F | Correction of configurations not to be tested in 4CC refsens test case 7.3B.2.3_1.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211883 | 0927 | 1 | F | Addition of DC_8A_n77A in test case 7.3B.2.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211884 | 0928 | 1 | F | Addition of DC_11A_n79A in test case 7.3B.2.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211885 | 0929 | 1 | F | Addition of DC_26A_n41A in test case 7.3B.2.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211886 | 0930 | 1 | F | Addition of DC_26A_n77A and DC_26A_n78A in test case 7.3B.2.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211887 | 0931 | 1 | F | Addition of DC_26A_n79A in test case 7.3B.2.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211888 | 0932 | 1 | F | Addition of DC_41A_n77A and DC_41A_n78A in test case 7.3B.2.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211905 | 0933 | 1 | F | Introduction of DC_1A-28A_n3A to reference sensitivity test | 16.7.0 |
| 2021-03 | RAN#91 | R5-211926 | 0851 | 1 | F | MU definition for UE MOP for Inter-Band EN-DC including FR2 (3CCs) | 16.7.0 |
| 2021-03 | RAN#91 | R5-211927 | 0852 | 1 | F | MU and TT definition for REFSENS EN-DC including FR2 up to 5CCs | 16.7.0 |
| 2021-03 | RAN#91 | R5-211928 | 0920 | 1 | F | Update FR2 MU and TT in 38.521-3 | 16.7.0 |
| 2021-03 | RAN#91 | - | - | - | - | Administrative release upgrade to match the release of TS 38.508-1, TS 38.508-2 and TS 38.521-1 which were upgraded at RAN#91 to Rel-17 due to Rel-17 relevant CRs | 17.0.0 |
| 2021-06 | RAN#92 | R5-212345 | 0944 | - | F | Define MU and TT for test case 7.9B.3_1.1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-212349 | 0948 | - | F | Update MU and TT in 38.521-3 for Transmit OFF Power FR2 CA tests | 17.1.0 |
| 2021-06 | RAN#92 | R5-212350 | 0949 | - | F | ACS and IBB - FR2 MU definition in 38.521-3 | 17.1.0 |
| 2021-06 | RAN#92 | R5-212531 | 0954 | - | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence for Rel 16 | 17.1.0 |
| 2021-06 | RAN#92 | R5-212646 | 0976 | - | F | Correcting test case title in 6.5B.2.2.2 | 17.1.0 |
| 2021-06 | RAN#92 | R5-212734 | 0977 | - | F | Update UL-MIMO to UL MIMO in clause 6 to align with other specs | 17.1.0 |
| 2021-06 | RAN#92 | R5-212735 | 0978 | - | F | Update UL-MIMO to UL MIMO in clause 7 to align with other specs | 17.1.0 |
| 2021-06 | RAN#92 | R5-212736 | 0979 | - | F | Update UL-MIMO to UL MIMO in Common and Annexes to align with other specs | 17.1.0 |
| 2021-06 | RAN#92 | R5-212749 | 0983 | - | F | Update to EN-DC R15 common section | 17.1.0 |
| 2021-06 | RAN#92 | R5-212824 | 0986 | - | F | Correction of 6.2B.1.2 for test of UE maximum output power for intra-band non-contiguous EN-DC | 17.1.0 |
| 2021-06 | RAN#92 | R5-212825 | 0987 | - | F | Correction of 6.2B.1.3 for test of UE maximum output power for EN-DC within FR1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-212867 | 0992 | - | F | Correction of the section order in 7.9B | 17.1.0 |
| 2021-06 | RAN#92 | R5-212869 | 0994 | - | F | Unify the SCS definitions in the test configuration tables | 17.1.0 |
| 2021-06 | RAN#92 | R5-212976 | 0996 | - | F | Updating H.2.2 for NR NSA testing | 17.1.0 |
| 2021-06 | RAN#92 | R5-213018 | 0998 | - | F | Update of E-UTRA TDD configuration for overlapping UL transmission | 17.1.0 |
| 2021-06 | RAN#92 | R5-213019 | 0999 | - | F | Update of TDM pattern configuration in EN-DC MOP and A-MPR cases | 17.1.0 |
| 2021-06 | RAN#92 | R5-213020 | 1000 | - | F | Update of EN-DC Tx cases to enable DFT-s-OFDM modulation for NR uplink carrier | 17.1.0 |
| 2021-06 | RAN#92 | R5-213045 | 1006 | - | F | Correction to reference sensitivity for DC_41A_n77A and DC_41A_n78A | 17.1.0 |
| 2021-06 | RAN#92 | R5-213046 | 1007 | - | F | Correction to reference sensitivity for a few Rel-16 EN-DC combinations | 17.1.0 |
| 2021-06 | RAN#92 | R5-213047 | 1008 | - | F | Correction to test description in 7.3B.2 refsens for EN-DC within FR1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213048 | 1009 | - | F | Update of test requirements for exception avoiding test points in 7.3B.2.3 refsens for inter-band EN-DC | 17.1.0 |
| 2021-06 | RAN#92 | R5-213059 | 1011 | - | F | Update of 6.4B.2.2.3 In-band Emissions for intra-band non-contiguous EN-DC | 17.1.0 |
| 2021-06 | RAN#92 | R5-213060 | 1012 | - | F | Correction of 7.4B.3_1 Maximum Input Level for EN-DC within FR1 with more than 2 CCs | 17.1.0 |
| 2021-06 | RAN#92 | R5-213200 | 1015 | - | F | Correction to EN-DC FR1 TC6.3B.1.1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213377 | 1028 | - | F | Correcting test procedure and test requirement for MPR Intra-Band Contiguous EN-DC | 17.1.0 |
| 2021-06 | RAN#92 | R5-213843 | 0971 | 1 | F | Correction of power control in 38.521-3 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213844 | 1020 | 1 | F | FR2 Inter-band Carrier Aggregation Minimum Output power updates | 17.1.0 |
| 2021-06 | RAN#92 | R5-213845 | 1010 | 1 | F | Adding refsens testing per band for all inter-band 2CC EN-DC FR1 non-exception requirements | 17.1.0 |
| 2021-06 | RAN#92 | R5-213846 | 1019 | 1 | F | Measurement uncertainties and test tolerances for FR2 Inter-band Carrier Aggregation Minimum Output power | 17.1.0 |
| 2021-06 | RAN#92 | R5-213905 | 0947 | 1 | F | Update 6.2B.1.1 according to core requirements | 17.1.0 |
| 2021-06 | RAN#92 | R5-213906 | 0952 | 1 | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence for Rel 15 DC_2A_n71A | 17.1.0 |
| 2021-06 | RAN#92 | R5-213907 | 0955 | 1 | F | Update for 6.5B.3.3.2 for Rel 15 requirement | 17.1.0 |
| 2021-06 | RAN#92 | R5-213908 | 0975 | 1 | F | References to voided clause 5.2B.2 corrected | 17.1.0 |
| 2021-06 | RAN#92 | R5-213909 | 0990 | 1 | F | Removal of test cases in 6.3B.2.4_1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213910 | 0997 | 1 | F | Correction of test frequencies for NR band n28 30MHz test channel bandwidth in 6.2B.4.1.3 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213911 | 1001 | 1 | F | Update of EN-DC Tx test cases with LTE anchor agnostic approach applied | 17.1.0 |
| 2021-06 | RAN#92 | R5-213912 | 1023 | 1 | F | Updates to Frequency Error for Inter-band EN-DC including FR2 (>2 CCs) | 17.1.0 |
| 2021-06 | RAN#92 | R5-213913 | 1025 | 1 | F | Addition of missing EN-DC Beam Correspondence requirements | 17.1.0 |
| 2021-06 | RAN#92 | R5-213914 | 0963 | 1 | F | Addition of 7.5B.3_1.3 ACS for EN-DC within FR1 5 CCs | 17.1.0 |
| 2021-06 | RAN#92 | R5-213915 | 0964 | 1 | F | Addition of 7.5B.3_1.4 ACS for EN-DC within FR1 6 CCs | 17.1.0 |
| 2021-06 | RAN#92 | R5-213916 | 0973 | 1 | F | Update of FR1 EN-DC intermodulation with 4CC in TC 7.8B.2.3_1.2 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213917 | 1029 | 1 | F | Correction to EN-DC Rx test case and format | 17.1.0 |
| 2021-06 | RAN#92 | R5-213918 | 0967 | 1 | F | Update of Annex F.1.3 for ACS for EN-DC within FR1 5CCs and 6CCs | 17.1.0 |
| 2021-06 | RAN#92 | R5-213973 | 0942 | 1 | F | Correction to EN-DC configuration DC_7A_n3A spurious emission test requirement | 17.1.0 |
| 2021-06 | RAN#92 | R5-213974 | 0953 | 1 | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence for Rel 16 DC_66A_n2A | 17.1.0 |
| 2021-06 | RAN#92 | R5-213975 | 0988 | 1 | F | Update of 6.5B.3.3 for spurious emission for DC_2_n41 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213976 | 0956 | 1 | F | Correction for DC_7A-20A_n3A reference sensitivity test | 17.1.0 |
| 2021-06 | RAN#92 | R5-214008 | 1018 | 1 | F | Update of PC2 ENDC DC_3A-n78A into 38.521-3 TC7.3B.2.3 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214030 | 1014 | 1 | F | Addition of PC2 ENDC DC_1A-n78A into 38.521-3 TC7.3B.2.3 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214041 | 0972 | 1 | F | Correction of ON OFF time mask in 38.521-3 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214052 | 0993 | 1 | F | Correction of for further study notes about FR2 ETC testing | 17.1.0 |
| 2021-06 | RAN#92 | R5-214053 | 1002 | 1 | F | Correction to UE co-existence spurious emissions for intra-band contiguous EN-DC | 17.1.0 |
| 2021-06 | RAN#92 | R5-214054 | 1003 | 1 | F | Correction to UE co-existence spurious emissions for intra-band non-contiguous EN-DC | 17.1.0 |
| 2021-06 | RAN#92 | R5-214055 | 1004 | 1 | F | Correction to test configuration for general spurious emissions for inter-band EN-DC | 17.1.0 |
| 2021-06 | RAN#92 | R5-214056 | 1005 | 1 | F | Updating test requirements for general spurious emissions for inter-band EN-DC | 17.1.0 |
| 2021-06 | RAN#92 | R5-214057 | 1024 | 1 | F | Update of Spurious emission band UE co-existence test case | 17.1.0 |
| 2021-06 | RAN#92 | R5-214072 | 0950 | 1 | F | Update of 6.5B.3.3.2 for Rel-16 combo DC_14A_n2A and DC_14A_n66A | 17.1.0 |
| 2021-06 | RAN#92 | R5-214079 | 0958 | 1 | F | Update of Applicability for Tx inter-band EN-DC including FR2 under R15 WI | 17.1.0 |
| 2021-06 | RAN#92 | R5-214080 | 0974 | 1 | F | Editor's note for Wgap can be removed in 6.5B.4.2 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214081 | 0980 | 1 | F | Update of applicability for EN-DC Tx tests within FR1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214082 | 0959 | 1 | F | Update of Applicability for Rx inter-band EN-DC including FR2 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214083 | 0961 | 1 | F | Correction of 7.5B.3_1.1 ACS for EN-DC within FR1 3 CCs | 17.1.0 |
| 2021-06 | RAN#92 | R5-214084 | 0962 | 1 | F | Correction of 7.5B.3_1.2 ACS for EN-DC within FR1 4 CCs | 17.1.0 |
| 2021-06 | RAN#92 | R5-214085 | 0966 | 1 | F | Update of Applicability for 7.5B.3 ACS for inter-band EN-DC within FR1 2 CCs | 17.1.0 |
| 2021-06 | RAN#92 | R5-214086 | 0981 | 1 | F | Update of applicability for EN-DC Rx tests within FR1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214087 | 0982 | 1 | F | Correction to EN-DC inter-band FR2 test cases 7.3B.2.4_1.x | 17.1.0 |
| 2021-06 | RAN#92 | R5-214094 | 0957 | 1 | F | Update of Applicability for Tx inter-band EN-DC including FR2 under R16 WI | 17.1.0 |
| 2021-06 | RAN#92 | R5-214095 | 0984 | 1 | F | Update to EN-DC R16 common section | 17.1.0 |
| 2021-09 | RAN#93 | R5-214188 | 1030 | - | F | Update of R17 CADC configurations into TS38.521-3 clause 5 | 17.2.0 |
| 2021-09 | RAN#93 | R5-214280 | 1032 | - | F | Introduction of Rel-15 EN-DC DC_1A_n77A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214281 | 1033 | - | F | Introduction of Rel-15 EN-DC DC_1A_n79A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214282 | 1034 | - | F | Introduction of Rel-15 EN-DC DC_3A_n28A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214283 | 1035 | - | F | Introduction of Rel-15 EN-DC DC_3A_n77A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214290 | 1042 | - | F | Introduction of Rel-15 EN-DC DC_21A_n77A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214291 | 1043 | - | F | Introduction of Rel-15 EN-DC DC_21A_n78A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214292 | 1044 | - | F | Introduction of Rel-15 EN-DC DC_21A_n79A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214293 | 1045 | - | F | Introduction of Rel-15 EN-DC DC_28A_n77A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214296 | 1048 | - | F | Introduction of Rel-15 EN-DC DC_42A_n77A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214298 | 1050 | - | F | Update of Rel-15 EN-DC DC_3A_n79A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214299 | 1051 | - | F | Update of Rel-15 EN-DC DC_5A_n66A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214301 | 1053 | - | F | Update of Rel-15 EN-DC DC_7A_n78A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214302 | 1054 | - | F | Update of Rel-15 EN-DC DC_11A_n77A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214303 | 1055 | - | F | Update of Rel-15 EN-DC DC_11A_n78A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214304 | 1056 | - | F | Update of Rel-15 EN-DC DC_11A_n79A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214307 | 1059 | - | F | Update of Rel-15 EN-DC DC_26A_n78A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214308 | 1060 | - | F | Update of Rel-15 EN-DC DC_39A_n79A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214309 | 1061 | - | F | Update of Rel-15 EN-DC DC_41A_n77A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214310 | 1062 | - | F | Update of Rel-15 EN-DC DC_41A_n78A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214311 | 1063 | - | F | Update of Rel-15 EN-DC DC_41A_n79A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214312 | 1064 | - | F | Update of Rel-15 EN-DC DC_66A_n5A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214313 | 1065 | - | F | Update of Rel-15 EN-DC DC_66A_n78A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214318 | 1067 | - | F | Update of Rel-16 EN-DC DC_40A_n78A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214384 | 1070 | - | F | Update clause 7 for R17 DC RX characteristics in TS 38.521-3 | 17.2.0 |
| 2021-09 | RAN#93 | R5-214481 | 1071 | - | F | Update of Applicability and Titles for ACS for EN-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-214483 | 1072 | - | F | Clarification on NSA Option 3 Tx test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214484 | 1073 | - | F | Clarification on NSA Option 3 Rx test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214846 | 1094 | - | F | Correction to EN-DC receiver spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215047 | 1096 | - | F | Correcting references in EN-DC TX test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215061 | 1098 | - | F | Update of REFSENS for inter-band EN-DC 2CC adding DC_28A_n78A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215062 | 1099 | - | F | Update of REFSENS for inter-band EN-DC 3CC adding DC_1A-28A_n78A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215063 | 1100 | - | F | Update of REFSENS for inter-band EN-DC 3CC adding DC_3A-28A_n78A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215194 | 1108 | - | F | Correction to 6.2B.2.1 MPR for intra-band contiguous EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215195 | 1109 | - | F | Correction to 6.2B.2.2 MPR for intra-band non-contiguous EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215209 | 1113 | - | F | Correction to 6.5B.2.1.1 SEM for intra-band contiguous EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215210 | 1114 | - | F | Correction to 6.5B.2.1.3 ACLR for intra-band contiguous EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215223 | 1118 | - | F | Correction to 6.5B.3.1.2 and 6.5B.3.2.2 UE co-existence spurious emissions | 17.2.0 |
| 2021-09 | RAN#93 | R5-215228 | 1119 | - | F | Correction to 6.4B.2 in-band emission for intra-band contiguous EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215230 | 1120 | - | F | Addition of test case body to 6.5B.5 transmit intermodulation | 17.2.0 |
| 2021-09 | RAN#93 | R5-215232 | 1122 | - | F | Addition of reference sensitivity testing for DC_1A_n28A-n78A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215233 | 1123 | - | F | Addition of reference sensitivity testing for DC_1A-3A_n28A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215234 | 1124 | - | F | Addition of reference sensitivity testing for DC_1A-7A_n28A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215235 | 1125 | - | F | Addition of reference sensitivity testing for DC_3A-7A_n28A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215254 | 1126 | - | F | Update of reference sensitivity test requirements for DC_41A_n77A and DC_41A_n78A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215255 | 1127 | - | F | Correction to reference sensitivity test configuration for 3CC EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215257 | 1129 | - | F | Update of reference sensitivity test coverage for 4CC EN-DC configurations | 17.2.0 |
| 2021-09 | RAN#93 | R5-215266 | 1130 | - | F | Added refsens deltaRIB test case for EN-DC including FR1 and FR2 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215292 | 1135 | - | F | Updating 7.3B.2.3 REFSENS testing for DC_3A-20A_n28A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215294 | 1136 | - | F | Updating 7.3B.2.3 REFSENS testing for DC_7A-20A_n28A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215296 | 1137 | - | F | Editorial correction to clause 7.3B.2.0.3 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215323 | 1138 | - | F | Correction of power control in 38.521-3 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215337 | 1140 | - | F | Correction of test CBW for n28 in 6.2B.1.3 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215376 | 1142 | - | F | Update to EN-DC R15 common section | 17.2.0 |
| 2021-09 | RAN#93 | R5-215378 | 1144 | - | F | Update to EN-DC R17 common section | 17.2.0 |
| 2021-09 | RAN#93 | R5-215379 | 1145 | - | F | Update of R15 EN-DC Tx tests | 17.2.0 |
| 2021-09 | RAN#93 | R5-215380 | 1146 | - | F | Update of applicability and title for R16 EN-DC Tx tests | 17.2.0 |
| 2021-09 | RAN#93 | R5-215381 | 1147 | - | F | Updated to title of clause 6.5B.5.x | 17.2.0 |
| 2021-09 | RAN#93 | R5-215460 | 1155 | - | F | Correction of 5.4B.1 for channel spacing for intra-band EN-DC carriers | 17.2.0 |
| 2021-09 | RAN#93 | R5-215521 | 1160 | - | F | Updates to Editors note for spurious emission CA test case | 17.2.0 |
| 2021-09 | RAN#93 | R5-215668 | 1179 | - | F | Correcting references in EN-DC RX test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215673 | 1180 | - | F | EN-DC including FR2 DL CA up to 8 NR CCs REFSENS measurement uncertainties | 17.2.0 |
| 2021-09 | RAN#93 | R5-215804 | 1181 | - | F | Update of 6.5B.2.3 out of band emissions for inter-band EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215832 | 1162 | 1 | F | EN-DC including FR2 DL CA up to 8 NR CCs REFSENS test cases addition | 17.2.0 |
| 2021-09 | RAN#93 | R5-215833 | 1169 | 1 | F | Measurement Uncertainties and test tolerances for NSA FR2 CA Maximum Output Power and Spectrum Emission Mask | 17.2.0 |
| 2021-09 | RAN#93 | R5-215864 | 1038 | 1 | F | Introduction of Rel-15 EN-DC DC_19A_n78A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215865 | 1039 | 1 | F | Introduction of Rel-15 EN-DC DC_19A_n79A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215866 | 1041 | 1 | F | Introduction of Rel-15 EN-DC DC_20A_n78A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215867 | 1047 | 1 | F | Introduction of Rel-15 EN-DC DC_28A_n79A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215868 | 1052 | 1 | F | Update of Rel-15 EN-DC DC_5A_n78A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215869 | 1057 | 1 | F | Update of Rel-15 EN-DC DC_25A_n41A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215870 | 1069 | 1 | F | Clarification of SA and NSA support in the UE | 17.2.0 |
| 2021-09 | RAN#93 | R5-215871 | 1074 | 1 | F | Addition of 6.4B.1.3A Frequency Error for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215872 | 1075 | 1 | F | Addition of 6.4B.2.3A.1 Error Vector Magnitude for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215873 | 1076 | 1 | F | Addition of 6.4B.2.3A.2 Carrier Leakage for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215874 | 1077 | 1 | F | Addition of 6.4B.2.3A.3 In-band Emissions for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215875 | 1079 | 1 | F | Addition of 6.5B.2.3A.1 Spectrum emissions mask for Inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215876 | 1080 | 1 | F | Addition of 6.5B.2.3A.2 Additional Spectrum emissions mask for Inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215877 | 1081 | 1 | F | Addition of 6.5B.2.3A.3 Adjacent channel leakage ratio for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215878 | 1082 | 1 | F | Addition of 6.5B.3.3A.1 General Spurious Emissions for Inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215879 | 1084 | 1 | F | Addition of 6.5B.5.3A Transmit Intermodulation for Inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215880 | 1095 | 1 | F | Editorial correction to test applicability in 6.2B.1.4_1.1.1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215881 | 1097 | 1 | F | Addition of spurious emission for DC 1A_n78A and 20A_n78A and 28A_n78A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215882 | 1104 | 1 | F | Addition of TC6.3B.2.4D Transmit OFF Power for inter-band EN-DC including FR2 for UL-MIMO | 17.2.0 |
| 2021-09 | RAN#93 | R5-215883 | 1106 | 1 | F | Addition of TC6.3B.3.4D Transmit ON/OFF time mask for inter-band EN-DC including FR2 for UL-MIMO | 17.2.0 |
| 2021-09 | RAN#93 | R5-215884 | 1107 | 1 | F | Addition of TC6.3B.4.4 PRACH Time Mask for inter-band EN-DC including FR2 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215885 | 1110 | 1 | F | Update of test applicability for 6.2B.2 MPR and 6.2B.3 A-MPR for inter-band EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215886 | 1121 | 1 | F | Update of MOP requirements for DC_3A_n3A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215887 | 1131 | 1 | F | Updated EN-DC spur emissions including FR2 editor notes | 17.2.0 |
| 2021-09 | RAN#93 | R5-215888 | 1156 | 1 | F | Addition of test case 6.4B.2.4.5 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215889 | 1167 | 1 | F | Message content updates for intra-band contiguous EN-DC additional spectrum emission mask test | 17.2.0 |
| 2021-09 | RAN#93 | R5-215890 | 1168 | 1 | F | Message contents addition for intra-band non-contiguous EN-DC SEM, A-SEM and ACLR test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215891 | 1086 | 1 | F | Addition of 7.4B.3A Maximum Input Level for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215892 | 1087 | 1 | F | Addition of 7.5B.3A ACS for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215893 | 1088 | 1 | F | Addition of 7.6B.2.3A In-band blocking for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215894 | 1089 | 1 | F | Addition of 7.6B.4.3A Narrow band blocking for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215895 | 1090 | 1 | F | Addition of 7.8B.2.3A Wide band Intermodulation for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215896 | 1091 | 1 | F | Addition of 7.9B.3A Spurious Emissions for inter-band NE-DC within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215897 | 1093 | 1 | F | Addition of 7.5B.0.4a Inter-band NE-DC including FR2 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215898 | 1133 | 1 | F | Updating 7.3B.2.3 REFSENS testing for DC_3A_n28A-n78A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215899 | 1134 | 1 | F | Updating 7.3B.2.3 REFSENS testing for DC_7A_n28A-n78A | 17.2.0 |
| 2021-09 | RAN#93 | R5-215900 | 1141 | 1 | F | Editors note correction to reference sensitivity for inter-band EN-DC including FR2 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215926 | 1161 | 1 | F | Spurious co-existence corrections for band combo DC_8_n41 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215927 | 1173 | 1 | F | Update for 6.5B.3.3.1 for Rel 16 combos | 17.2.0 |
| 2021-09 | RAN#93 | R5-215928 | 1143 | 1 | F | Update to EN-DC R16 common section | 17.2.0 |
| 2021-09 | RAN#93 | R5-216007 | 1031 | 1 | F | Introduction of Rel-15 EN-DC DC_1A_n28A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-216008 | 1036 | 1 | F | Introduction of Rel-15 EN-DC DC_7A_n28A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-216009 | 1040 | 1 | F | Introduction of Rel-15 EN-DC DC_20A_n28A to spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-216010 | 1058 | 1 | F | Update of Rel-15 EN-DC DC_26A_n77A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-216011 | 1111 | 1 | F | Cleanup for TS 38.521-3 spurious emission for UE co-existence table for Rel-15 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216012 | 1112 | 1 | F | Cleanup for TS 38.521-3 spurious emission for UE co-existence table Rel-16 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216013 | 1170 | 1 | F | Update for 6.5B.3.3.1 for Rel 15 combos | 17.2.0 |
| 2021-09 | RAN#93 | R5-216015 | 1117 | 1 | F | Update of general spurious emissions test requirements for Rel-16 inter-band EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-216016 | 1154 | 1 | F | Update of 6.5B.3.3.2.3 for the requirements of spurious emission band UE co-existence limits for Rel-16 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216038 | 1049 | 1 | F | Update of Rel-15 EN-DC DC_1A_n78A in spurious emission test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-216039 | 1182 | 1 | F | Addition of cl 6.2B.1.3A for RF | 17.2.0 |
| 2021-09 | RAN#93 | R5-216093 | 1132 | 1 | F | Updated editors note to indicate missing LO retrieval RRC framework | 17.2.0 |
| 2021-09 | RAN#93 | R5-216094 | 1128 | 1 | F | Update of reference sensitivity test coverage for 3CC EN-DC configurations | 17.2.0 |
| 2021-09 | RAN#93 | R5-216095 | 1178 | 1 | F | Update to Rel.15 EN-DC FR2 Band Combination Tables | 17.2.0 |
| 2021-09 | RAN#93 | R5-216108 | 1171 | 1 | F | Update for reference sensitivity for DC_48A_n66A | 17.2.0 |
| 2021-09 | RAN#93 | R5-216119 | 1115 | 1 | F | Correction to 6.5B.2.2.1 SEM for intra-band non-contiguous EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-216120 | 1116 | 1 | F | Correction to 6.5B.2.2.3 ACLR for intra-band non-contiguous EN-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-216131 | 1159 | 1 | F | Spurious co-existence core requirement updates for Dual connectivity including band n28 and other core requirement alignments | 17.2.0 |
| 2021-12 | RAN#94 | R5-216518 | 1184 | - | F | 6.5B.2.2.1 SEM IBNC update as per TP analysis update | 17.3.0 |
| 2021-12 | RAN#94 | R5-216519 | 1185 | - | F | 6.5B.2.2.3 ACLR IBNC update as per TP analysis update | 17.3.0 |
| 2021-12 | RAN#94 | R5-216919 | 1189 | - | F | Addition of TC6.4B.1.4D Frequency error for inter-band EN-DC including FR2 for UL-MIMO | 17.3.0 |
| 2021-12 | RAN#94 | R5-217094 | 1191 | - | F | Update of Reference Sensitivity Test Cases for EN-DC with FR2 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217116 | 1192 | - | F | FR2 EN-DC Refsens 6 to 8 NR CCs - Editorial correction | 17.3.0 |
| 2021-12 | RAN#94 | R5-217117 | 1193 | - | F | UL modulation correction in 7.3B.2.3_1.1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217176 | 1195 | - | F | Adding Power Class 1.5 for LTE Band 41and NR Band n41 to Annex N.1 Indication of modified MPR behaviour | 17.3.0 |
| 2021-12 | RAN#94 | R5-217232 | 1199 | - | F | Tx Spurious emission-Editorial corrections | 17.3.0 |
| 2021-12 | RAN#94 | R5-217295 | 1202 | - | F | Update of 6.5B.2.1.1 SEM test configuration for intra-band contiguous EN-DC | 17.3.0 |
| 2021-12 | RAN#94 | R5-217296 | 1203 | - | F | Update of 6.2B.2.1 MPR for intra-band contiguous EN-DC | 17.3.0 |
| 2021-12 | RAN#94 | R5-217300 | 1204 | - | F | Addition of reference sensitivity testing for DC_20A_n28A-n78A | 17.3.0 |
| 2021-12 | RAN#94 | R5-217305 | 1206 | - | F | Addition of 4Rx reference sensitivity test requirements for DC_3A_n41A | 17.3.0 |
| 2021-12 | RAN#94 | R5-217382 | 1211 | - | F | Addition of 6.2E.1.1 MOP for intra-band contiguous V2X operation | 17.3.0 |
| 2021-12 | RAN#94 | R5-217383 | 1212 | - | F | Addition of 6.2E.1.2 MOP for intra-band non-contiguous V2X operation | 17.3.0 |
| 2021-12 | RAN#94 | R5-217384 | 1213 | - | F | Addition of 6.2E.1.3.1 MOP for inter-band E-UTRA Uu and NR Sidelink operation | 17.3.0 |
| 2021-12 | RAN#94 | R5-217385 | 1214 | - | F | Addition of 6.2E.1.3.2 MOP for inter-band NR Uu and E-UTRA V2X Sidelink operation | 17.3.0 |
| 2021-12 | RAN#94 | R5-217386 | 1215 | - | F | Addition of 6.2E.2.1 MPR for intra-band V2X operation | 17.3.0 |
| 2021-12 | RAN#94 | R5-217387 | 1216 | - | F | Addition of 6.2E.2.2.1 MPR for inter-band E-UTRA Uu and NR Sidelink operation | 17.3.0 |
| 2021-12 | RAN#94 | R5-217388 | 1217 | - | F | Addition of 6.2E.2.2.2 MPR for inter-band NR Uu and E-UTRA V2X Sidelink operation | 17.3.0 |
| 2021-12 | RAN#94 | R5-217389 | 1218 | - | F | Addition of 6.3E Output power dynamics for V2X | 17.3.0 |
| 2021-12 | RAN#94 | R5-217428 | 1220 | - | F | Correction to note of DC_1_n3 and DC_3_n1 in 6.5B.3.3.2 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217554 | 1228 | - | F | Correction of Reference Sensitivity for DC_28A_n41A | 17.3.0 |
| 2021-12 | RAN#94 | R5-217562 | 1229 | - | F | Update of modulation quality for inter-band EN-DC with FR2 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217610 | 1230 | - | F | Correction to 5G V2X common sections | 17.3.0 |
| 2021-12 | RAN#94 | R5-217615 | 1231 | - | F | Addition of R15 EN-DC configurations in clause 5 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217616 | 1232 | - | F | Correction to EN-DC Tx test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217617 | 1233 | - | F | Correction to title and test applicability to EN-DC Rx test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217705 | 1237 | - | F | 38.521-3 Beam correspondence Measurement Uncertainties | 17.3.0 |
| 2021-12 | RAN#94 | R5-217720 | 1238 | - | F | Introduction of EN-DC FR2 Beam Correspondence Test Case | 17.3.0 |
| 2021-12 | RAN#94 | R5-217724 | 1239 | - | F | Update to Rel.17 EN-DC FR2 Band Combination Tables | 17.3.0 |
| 2021-12 | RAN#94 | R5-217726 | 1241 | - | F | Update to Rel.15 EN-DC FR2 Band Combination Tables | 17.3.0 |
| 2021-12 | RAN#94 | R5-217740 | 1242 | - | F | Correction to DC_1A_n79A spurious emission test case 6.5B.3.3.2 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217741 | 1243 | - | F | Correction of spurious emission test case 6.5B.3.3.2 for DC_5A-n66A, DC_25A-n41A and DC_19A_n77A | 17.3.0 |
| 2021-12 | RAN#94 | R5-218211 | 1250 | - | F | Adding new test case spurious UE coex for inter-band EN-DC including FR2 and editors note updates | 17.3.0 |
| 2021-12 | RAN#94 | R5-218243 | 1207 | 1 | F | Update of 6.5B.3.3.1 general spurious emissions test requirements for inter-band EN-DC | 17.3.0 |
| 2021-12 | RAN#94 | R5-218244 | 1205 | 1 | F | Addition of 4Rx reference sensitivity test requirements for DC_3A_n78A | 17.3.0 |
| 2021-12 | RAN#94 | R5-218245 | 1221 | 1 | F | Correction to test configuration of DC_41A_n78A in 7.3B.2.3 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218246 | 1188 | 1 | F | Handling of fallbacks for FR2 CA for EN-DC | 17.3.0 |
| 2021-12 | RAN#94 | R5-218247 | 1210 | 1 | F | Correction of clause 4 for minimum requirements and test applicability rules | 17.3.0 |
| 2021-12 | RAN#94 | R5-218271 | 1197 | 1 | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence | 17.3.0 |
| 2021-12 | RAN#94 | R5-218273 | 1249 | 1 | F | Adding RX requirements for Rel-16 FR1 EN-DC band combinations | 17.3.0 |
| 2021-12 | RAN#94 | R5-218274 | 1234 | 1 | F | Addition of R16 EN-DC configurations in clause 5 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218275 | 1240 | 1 | F | Update to Rel.16 EN-DC FR2 Band Combination Tables | 17.3.0 |
| 2021-12 | RAN#94 | R5-218293 | 1208 | 1 | F | Adding UL switching time mask test for inter-band EN-DC | 17.3.0 |
| 2021-12 | RAN#94 | R5-218433 | 1183 | 1 | F | 6.2B.2.2 MPR IBNC update as per TP analysis update | 17.3.0 |
| 2021-12 | RAN#94 | R5-218434 | 1236 | 1 | F | Update of test case 6.2B.3.4 EN-DC A-MPR for FR2 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218455 | 1200 | 1 | F | Update for EN_DC reference sensitivity_r16 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218472 | 1227 | 1 | F | Updating 6.2B.1.3 UE Maximum Output Power for Rel-17 NR inter-band EN-DC configurations | 17.3.0 |
| 2021-12 | RAN#94 | R5-218473 | 1226 | 1 | F | Updating Rel-17 NR inter-band EN-DC configurations | 17.3.0 |
| 2021-12 | RAN#94 | R5-218485 | 1225 | 1 | F | Clarification on cl 4.5.1 test coverage across 5G NR architecture options for RF | 17.3.0 |
| 2022-03 | RAN#95 | R5-220063 | 1251 | - | F | Correction of Test applicability of 6.2B.2.3 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220068 | 1254 | - | F | Correction of test config table for 6.3B.3_1.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220069 | 1255 | - | F | Correction of reference section numbers in 6.4B.2.4.5.4.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220070 | 1256 | - | F | Correction of Editor Note and reference section numbers in 6.5B.3.4.2_1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220087 | 1257 | - | F | Introduction of new V2X test cases in 7.6E | 17.4.0 |
| 2022-03 | RAN#95 | R5-220088 | 1258 | - | F | Introduction of new V2X test cases in 7.7E | 17.4.0 |
| 2022-03 | RAN#95 | R5-220089 | 1259 | - | F | Introduction of new V2X test cases in 7.8E | 17.4.0 |
| 2022-03 | RAN#95 | R5-220260 | 1260 | - | F | FR1 NSA IBC - ACLR clean up to leverage MPR test definition | 17.4.0 |
| 2022-03 | RAN#95 | R5-220275 | 1261 | - | F | Clarifications on 5G NR connectivity options for RF FR1 and FR2 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220314 | 1264 | - | F | Introduction of Output power requirements for Rel-16 inter-band EN-DC FR1 two band combinations | 17.4.0 |
| 2022-03 | RAN#95 | R5-220315 | 1265 | - | F | Introduction of General Spurious emissions requirements for Rel-16 inter-band EN-DC FR1 two band combinations | 17.4.0 |
| 2022-03 | RAN#95 | R5-220322 | 1267 | - | F | Adding Reference sensitivity exceptions and MSD test points for Rel-16 inter-band EN-DC FR1 two band combinations | 17.4.0 |
| 2022-03 | RAN#95 | R5-220323 | 1268 | - | F | Adding reference sensitivity requirements for Rel-16 inter-band EN-DC FR1 two band combinations | 17.4.0 |
| 2022-03 | RAN#95 | R5-220369 | 1270 | - | F | Update general spurious emissions 6.5B.3.3.1 for 4 Rel-17 ENDC combos | 17.4.0 |
| 2022-03 | RAN#95 | R5-220371 | 1271 | - | F | Update for 6.5B.3.3.2 Spurious emission band UE co-existence for 4 Rel-17 combos | 17.4.0 |
| 2022-03 | RAN#95 | R5-220380 | 1273 | - | F | Introduction of DC_1A-n5A reference sensitivity test requirements | 17.4.0 |
| 2022-03 | RAN#95 | R5-220382 | 1274 | - | F | Introduction of DC_3A-n5A reference sensitivity test requirements | 17.4.0 |
| 2022-03 | RAN#95 | R5-220384 | 1275 | - | F | Introduction of DC_7A-n5A reference sensitivity test requirements | 17.4.0 |
| 2022-03 | RAN#95 | R5-220387 | 1276 | - | F | Introduction of DC_7A-n78A reference sensitivity test requirements | 17.4.0 |
| 2022-03 | RAN#95 | R5-220390 | 1277 | - | F | Introduction of DC_28A_n7A-n78A reference sensitivity test requirements | 17.4.0 |
| 2022-03 | RAN#95 | R5-220431 | 1280 | - | F | Update to R16 Configuration for DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220432 | 1281 | - | F | Update to R17 Configuration for DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220538 | 1282 | - | F | Correction on test requirements for TC 6.5B.3.3.2 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220539 | 1283 | - | F | Addition of 6.4E.1 Frequency error for V2X | 17.4.0 |
| 2022-03 | RAN#95 | R5-220540 | 1284 | - | F | Addition of 6.4E.2.1 Error Vector Magnitude for V2X | 17.4.0 |
| 2022-03 | RAN#95 | R5-220655 | 1286 | - | F | Adding RIB,c for Inter-band NE-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-221698 | 1289 | 1 | F | Updating on 6.5B.3.3.2 Spurious emission for UE co-existence for inter-band within FR1 including n1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220759 | 1290 | - | F | Introduction of maximum output power test requirements for DC_1A_n5A, DC_1A_n7A, DC_3A_n5A, DC_7A_n5A and DC_28A_n7A | 17.4.0 |
| 2022-03 | RAN#95 | R5-220766 | 1291 | - | F | Update Tx test cases for DC_n28A_3A, DC_n28A_3C, DC_n28A_39A, DC_n28A_39C | 17.4.0 |
| 2022-03 | RAN#95 | R5-220896 | 1296 | - | F | Definition of MTSU for 7.6B.3.3_1.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220902 | 1298 | - | F | Correction to measurement timing for EN-DC combination with FDD and TDD | 17.4.0 |
| 2022-03 | RAN#95 | R5-220904 | 1299 | - | F | Correction to test frequency of EN-DC 28_n51 in 7.3B.2.3 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220961 | 1301 | - | F | Addition of common uplink configuration for E-UTRA intra-band contiguous CA | 17.4.0 |
| 2022-03 | RAN#95 | R5-220967 | 1305 | - | F | Update of 6.2B.1.3 Maximum Output Power for Inter-Band EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220968 | 1306 | - | F | Update of 6.2B.4.1.3 Configured Output Power for Inter-Band EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220976 | 1308 | - | F | Correction to reference sensitivity for intra-band contiguous EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220977 | 1309 | - | F | Correction to Maximum Input Level for intra-band contiguous EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220978 | 1310 | - | F | Correction to Adjacent Channel Selectivity for intra-band contiguous EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220979 | 1311 | - | F | Update of Adjacent Channel Selectivity for intra-band non-contiguous EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220980 | 1312 | - | F | Correction to out-of-band blocking for intra-band contiguous EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-220981 | 1313 | - | F | Update of Annex F for Adjacent Channel Selectivity for intra-band EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-221053 | 1314 | - | F | Correction of 6.2B.1.1 for intra-band contiguous EN-DC maximum output power | 17.4.0 |
| 2022-03 | RAN#95 | R5-221170 | 1316 | - | F | Addition of 6.2B.2.3a MPR for inter-band NE-DC within FR1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221171 | 1317 | - | F | Addition of 6.4B.2.3a.4 EVM Equalizer Flatness for inter-band NE-DC within FR1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221172 | 1318 | - | F | Addition of 6.2B.4.2.3a TIB,c for Inter-band NE-DC within FR1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221268 | 1323 | - | F | Update of E-UTRA configuration | 17.4.0 |
| 2022-03 | RAN#95 | R5-221317 | 1326 | - | F | Update for 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-221321 | 1327 | - | F | Update for reference sensitivity for EN_DC_r15 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221325 | 1328 | - | F | Editorial Update for 6.2B.4.2.3.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221329 | 1330 | - | F | Update Ref sense for r16 DC combos | 17.4.0 |
| 2022-03 | RAN#95 | R5-221337 | 1332 | - | F | Addition of PC2 ENDC 4 combos into 38.521-3 TC7.3B.2 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221693 | 1252 | 1 | F | Correction of Test applicability of 6.2B.3.3 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221694 | 1253 | 1 | F | Correction of 6.5B.2.3.3 to include 6.5.2.4.2 of 38.521-1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221695 | 1262 | 1 | F | Update MOP for inter-band NE-DC within FR1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221696 | 1269 | 1 | F | FR2 NSA EVM test case editor notes update | 17.4.0 |
| 2022-03 | RAN#95 | R5-221697 | 1287 | 1 | F | Clarification on clause number of NE-DC for Tx test cases | 17.4.0 |
| 2022-03 | RAN#95 | R5-221699 | 1293 | 1 | F | Correction of ON OFF time mask for inter-band EN-DC including FR2 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221700 | 1294 | 1 | F | Definition of MTSU and TT for Intra-band EN-DC additional spurious emissions test cases | 17.4.0 |
| 2022-03 | RAN#95 | R5-221701 | 1295 | 1 | F | Correction to test procedure of FR1 EN-DC Spurious test for EN-DC only capable UE | 17.4.0 |
| 2022-03 | RAN#95 | R5-221702 | 1324 | 1 | F | Editorial correction for 6.5B.3.3 Spurious emission | 17.4.0 |
| 2022-03 | RAN#95 | R5-221703 | 1288 | 1 | F | Clarification on clause number of NE-DC for Rx test cases | 17.4.0 |
| 2022-03 | RAN#95 | R5-221704 | 1331 | 1 | F | Update for 7.3B.2 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221705 | 1278 | 1 | F | Regrouping DC Configuration in clause 5 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221706 | 1315 | 1 | F | Update of 3.2 and 3.3 on symbols and abbreviations | 17.4.0 |
| 2022-03 | RAN#95 | R5-221767 | 1266 | 1 | F | Introduction of Spurious emissions band UE co-existence requirements for Rel-16 inter-band EN-DC FR1 two band combinations | 17.4.0 |
| 2022-03 | RAN#95 | R5-221768 | 1297 | 1 | F | Correction to test requirement of DC_xxA_n41A in 6.5B.3.3.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221769 | 1300 | 1 | F | Correction of General Spurious emissions requirements for Rel-16 inter-band EN-DC FR1 two band combinations | 17.4.0 |
| 2022-03 | RAN#95 | R5-221770 | 1320 | 1 | F | Introduction of DC_1A_n5A, DC_1A_n7A, DC_3A_n5A, DC_7A_n5A, DC_28A_n7A to general spurious emission test case | 17.4.0 |
| 2022-03 | RAN#95 | R5-221771 | 1321 | 1 | F | Introduction of DC_1A_n5A, DC_1A_n7A, DC_3A_n5A, DC_7A_n5A, DC_28A_n7A to UE co-existence spurious emission test case | 17.4.0 |
| 2022-03 | RAN#95 | R5-221772 | 1322 | 1 | F | Addition of new CADC MPR TC 6.2B.2.4_1.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221773 | 1329 | 1 | F | Update for 7.3B.2.0 Min Requirements of Ref sensitivity for EN-DC | 17.4.0 |
| 2022-03 | RAN#95 | R5-221883 | 1272 | 1 | F | Update Tx TC for 4 Rel-17 combos | 17.4.0 |
| 2022-03 | RAN#95 | R5-221884 | 1325 | 1 | F | Update Rx Requirements for 4 Rel-17 ENDC combos | 17.4.0 |
| 2022-03 | RAN#95 | R5-221885 | 1333 | 1 | F | MSD test configurations modification for US inter-band EN-DC combinations with n77 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221886 | 1285 | 1 | F | Update NE-DC configurations for DC_n28A_3A, DC_n28A_3C, DC_n28A_39A, DC_n28A_39C | 17.4.0 |
| 2022-03 | RAN#95 | R5-221892 | 1292 | 1 | F | Update of MOP TC for PC2 ENDC configurations | 17.4.0 |
| 2022-03 | RAN#95 | R5-221908 | 1302 | 1 | F | Addition of new test case 6.2B.1.3_1 for Maximum Output Power for inter-band EN-DC with 3 uplink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221909 | 1303 | 1 | F | Addition of new test case 6.2B.4.1.3_1 for Configured Output Power for inter-band EN-DC with 3 uplink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221910 | 1304 | 1 | F | Addition of annex F for test cases for EN-DC configurations with 3 uplink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221931 | 1307 | 1 | F | Addition of transmit power configuration for EN-DC reference sensitivity | 17.4.0 |
| 2022-06 | RAN#96 | R5-222192 | 1335 | - | F | Correction of minimum requirement and test requirement of 6.2B.1.3 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222194 | 1337 | - | F | Correction of reference section numbers in 6.4E and title of 6.4E.2.1.2 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222195 | 1338 | - | F | Correction of test requirement of 6.2B.2.1 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222196 | 1339 | - | F | Separation of 6.2B.1.4D into two test cases | 17.5.0 |
| 2022-06 | RAN#96 | R5-222197 | 1340 | - | F | Correction of clause numbers in 6.2B.1.3a | 17.5.0 |
| 2022-06 | RAN#96 | R5-222344 | 1353 | - | F | 6.6B.4 Beam Correspondence test case editor note update | 17.5.0 |
| 2022-06 | RAN#96 | R5-222345 | 1354 | - | F | MU and TT definition and clean up in 38.521-3 annex F | 17.5.0 |
| 2022-06 | RAN#96 | R5-222421 | 1355 | - | F | Addition of 6.5E.1 Occupied bandwidth for V2X | 17.5.0 |
| 2022-06 | RAN#96 | R5-222422 | 1356 | - | F | Addition of 6.5E.2 Out of band emission for V2X | 17.5.0 |
| 2022-06 | RAN#96 | R5-222473 | 1359 | - | F | Addition Delta TIB,c for FR1 EN-DC | 17.5.0 |
| 2022-06 | RAN#96 | R5-222484 | 1362 | - | F | Editorial correction in 6.2B.4.1.3 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222486 | 1363 | - | F | Correction about test configuration in 6.5B.3.3.2 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222487 | 1364 | - | F | Correction to title of 7.6B.2.4 and editorial correction for Rx test cases | 17.5.0 |
| 2022-06 | RAN#96 | R5-222543 | 1365 | - | F | Correction of Refsens CA test case | 17.5.0 |
| 2022-06 | RAN#96 | R5-222700 | 1367 | - | F | Editorial correction to EN-DC test cases | 17.5.0 |
| 2022-06 | RAN#96 | R5-222702 | 1368 | - | F | Update to R15 common part and DC configurations in clause 5 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222703 | 1369 | - | F | Update to R16 Configuration for DC | 17.5.0 |
| 2022-06 | RAN#96 | R5-222743 | 1379 | - | F | Update for 7.3B.2.0 Min Requirements of Ref sensitivity for EN-DC | 17.5.0 |
| 2022-06 | RAN#96 | R5-222748 | 1381 | - | F | Update 6.5B.3.3.2 for R17 DC_14_n2 and DC_14_n66 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222829 | 1382 | - | F | Update of 6.2B.1.3_1 UE Maximum Output Power for inter-Band EN-DC with 2 E-UTRA CCs and 1 NR CC | 17.5.0 |
| 2022-06 | RAN#96 | R5-222830 | 1383 | - | F | Update of 6.2B.4.1.3_1 Configured Output Power for inter-Band EN-DC with 2 E-UTRA CCs and 1 NR CC | 17.5.0 |
| 2022-06 | RAN#96 | R5-222884 | 1386 | - | F | Addition of 6.4E.2.3 In-band emissions for V2X | 17.5.0 |
| 2022-06 | RAN#96 | R5-223061 | 1387 | - | F | Addition of 6.5B.3.3.1 requirements for NR inter-band EN-DC configurations including n1 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223231 | 1389 | - | F | Correction to 6.2B.1.3 for UE capability IE for inter-band EN-DC UE maximum output power | 17.5.0 |
| 2022-06 | RAN#96 | R5-223245 | 1391 | - | F | Add delta RIBc for inter-band DC_28A_n7A-n78A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223658 | 1341 | 1 | F | Introduction of Output power requirements for DC_1A_n8A, DC_7A_n8A and DC_8A_n28A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223659 | 1342 | 1 | F | Introduction of Allowed maximum configured output power relaxation for DC_1_n5, DC_1_n8, DC_3_n5, DC_7_n5, DC_7_n8 and DC_8_n28 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223660 | 1343 | 1 | F | Introduction of General Spurious emissions requirements for DC_1A_n8A, DC_7A_n8A, DC_8A_n28A and DC_20A_n8A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223661 | 1344 | 1 | F | Introduction of Spurious emissions band UE co-existence limits Rel-16 for DC_1A_n8A, DC_7A_n8A, DC_8A_n28A and DC_20A_n8A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223662 | 1345 | 1 | F | Introduction of Spurious emissions band UE co-existence Test description for DC_1A_n8A, DC_7A_n5A, DC_7A_n8A, DC_8A_n28A and DC_20A_n8A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223663 | 1346 | 1 | F | Introduction of Spurious emissions band UE co-existence Rel-16 Test requirements for DC_1A_n8A, DC_7A_n8A, DC_8A_n28A and DC_20A_n8A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223664 | 1366 | 1 | F | Addition of new CADC MOP TC | 17.5.0 |
| 2022-06 | RAN#96 | R5-223665 | 1371 | 1 | F | Addition of ACLR Test Case for Inter-band EN-DC including FR2 5 NR CCs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223666 | 1372 | 1 | F | Addition of ACLR Test Case for Inter-band EN-DC including FR2 6 NR CCs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223667 | 1373 | 1 | F | Addition of ACLR Test Case for Inter-band EN-DC including FR2 7 NR CCs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223668 | 1374 | 1 | F | Addition of ACLR Test Case for Inter-band EN-DC including FR2 8 NR CCs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223669 | 1375 | 1 | F | Addition of General Spurious Emissions Test Case for Inter-band EN-DC including FR2 5 NR CCs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223670 | 1376 | 1 | F | Addition of General Spurious Emissions Test Case for Inter-band EN-DC including FR2 6 NR CCs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223671 | 1377 | 1 | F | Addition of General Spurious Emissions Test Case for Inter-band EN-DC including FR2 7 NR CCs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223672 | 1378 | 1 | F | Addition of General Spurious Emissions Test Case for Inter-band EN-DC including FR2 8 NR CCs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223673 | 1380 | 1 | F | Update 6.5B.3.3.2 for R16 DC_14_n2 and DC_14_n66 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223674 | 1390 | 1 | F | Add delta TIBc for inter-band DC_28A_n7A-n78A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223675 | 1393 | 1 | F | Introduction of Allowed reference sensitivity relaxation for DC_3A-8A_n28A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223676 | 1347 | 1 | F | Introduction of DC_1A-20A_n8A reference sensitivity test requirements | 17.5.0 |
| 2022-06 | RAN#96 | R5-223677 | 1348 | 1 | F | Introduction of DC_3A-7A_n5A reference sensitivity test requirements | 17.5.0 |
| 2022-06 | RAN#96 | R5-223678 | 1349 | 1 | F | Introduction of DC_7A-8A_n3A reference sensitivity test requirements | 17.5.0 |
| 2022-06 | RAN#96 | R5-223679 | 1350 | 1 | F | Introduction of DC_7A-20A_n8A reference sensitivity test requirements | 17.5.0 |
| 2022-06 | RAN#96 | R5-223680 | 1351 | 1 | F | Introduction of DC_7A-28A_n5A reference sensitivity test requirements | 17.5.0 |
| 2022-06 | RAN#96 | R5-223681 | 1360 | 1 | F | Addition Minimum Conformance Requests of REFSENS for FR1 EN-DC | 17.5.0 |
| 2022-06 | RAN#96 | R5-223703 | 1385 | 1 | F | Addition of 6.4E.2.2 Carrier leakage for V2X | 17.5.0 |
| 2022-06 | RAN#96 | R5-223744 | 1370 | 1 | F | Update to R17 Configuration for DC | 17.5.0 |
| 2022-06 | RAN#96 | R5-223833 | 1336 | 1 | F | Correction of Transmitter power test requirements for EN-DC within FR1 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223834 | 1357 | 1 | F | Editorial correction for 6.3B.8 Power control for EN-DC | 17.5.0 |
| 2022-06 | RAN#96 | R5-223835 | 1358 | 1 | F | Clarifications on Common Uplink Configuration updates | 17.5.0 |
| 2022-06 | RAN#96 | R5-223836 | 1384 | 1 | F | Adding missing configurations in SE co-ex Rel-17 table | 17.5.0 |
| 2022-09 | RAN#97 | R5-224152 | 1398 | - | F | Introduction of Allowed maximum configured output power relaxation for DC_3_n8 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224153 | 1399 | - | F | Introduction of General Spurious emissions requirements for DC_3A_n8A | 17.6.0 |
| 2022-09 | RAN#97 | R5-224154 | 1400 | - | F | Introduction of Spurious emissions band UE co-existence limits Rel-16 for DC_3A_n8A | 17.6.0 |
| 2022-09 | RAN#97 | R5-224156 | 1402 | - | F | Introduction of Spurious emissions band UE co-existence Rel-16 Test requirements for DC_3A_n8A | 17.6.0 |
| 2022-09 | RAN#97 | R5-224159 | 1403 | - | F | Introduction of DC_3A-7A-20A_n8A reference sensitivity 2-band fallback test requirements | 17.6.0 |
| 2022-09 | RAN#97 | R5-224160 | 1404 | - | F | Introduction of DC_3A-7A-20A_n8A reference sensitivity 3-band fallback test requirements | 17.6.0 |
| 2022-09 | RAN#97 | R5-224161 | 1405 | - | F | Introduction of Allowed reference sensitivity relaxation for DC_3A-7A-20A_n8A | 17.6.0 |
| 2022-09 | RAN#97 | R5-224308 | 1408 | - | F | FR2 NSA EVM test case editor notes update | 17.6.0 |
| 2022-09 | RAN#97 | R5-224437 | 1412 | - | F | Addition of new CADC MOP TC | 17.6.0 |
| 2022-09 | RAN#97 | R5-224622 | 1413 | - | F | Addition of test requirement for E-UTRA cell of PC2 UE in 6.2B.4.1 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224626 | 1414 | - | F | Correction to test frequency definition for DC combination including n28 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224629 | 1415 | - | F | Correction to E-UTRA output power in intra-band contiguous EN-DC Rx test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-224653 | 1417 | - | F | Correction of reference to test configuration table for intra band contiguous EN-DC test case | 17.6.0 |
| 2022-09 | RAN#97 | R5-224809 | 1418 | - | F | Editorial correction for 6.2B.1.3 Maximum output power for Inter-band EN-DC within FR1 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224936 | 1423 | - | F | Corrections on test configuration table in spurious emission band UE co-existence for Rel-15 inter-band EN-DC configuration | 17.6.0 |
| 2022-09 | RAN#97 | R5-224939 | 1425 | - | F | Introduction of DC_2A-66A_n41A reference sensitivity test requirements | 17.6.0 |
| 2022-09 | RAN#97 | R5-224942 | 1426 | - | F | Corrections on general spurious emission test requirements for DC_2A_n5A | 17.6.0 |
| 2022-09 | RAN#97 | R5-224943 | 1427 | - | F | Update of Rel-16 EN-DC configuration DC_2A_n41A in spurious emission test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-224946 | 1428 | - | F | Update of DC_8A_n41A, DC_3A_n41A, DC_25A_n41A, DC_26A_n41A, DC_39A_n41A and DC_40A_n41A in spurious emission test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-224950 | 1429 | - | F | Update of Rel-16 EN-DC configuration DC_48A_n5A in spurious emission test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-224952 | 1430 | - | F | Update of Rel-16 EN-DC configuration DC_48A_n66A in spurious emission test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-224954 | 1431 | - | F | Update of Rel-16 EN-DC configuration DC_66A_n41A in spurious emission test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-225000 | 1434 | - | F | Update to R16 Configuration for DC | 17.6.0 |
| 2022-09 | RAN#97 | R5-225001 | 1435 | - | F | Update to R17 Configuration for DC | 17.6.0 |
| 2022-09 | RAN#97 | R5-225005 | 1436 | - | F | Editorial correction to reference table ID to TC6.2B.1.3 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225006 | 1437 | - | F | Update to test case 6.5B.3.3.2 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225031 | 1438 | - | F | Add EN-DC Enhanced Beam Correspondence Test Case | 17.6.0 |
| 2022-09 | RAN#97 | R5-225661 | 1394 | 1 | F | New test case addition: 6.2B.4.1.4_1 Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC). | 17.6.0 |
| 2022-09 | RAN#97 | R5-225662 | 1395 | 1 | F | Measurement uncertainties and test tolerances for test case 6.2B.4.1.4_1 Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC) | 17.6.0 |
| 2022-09 | RAN#97 | R5-225668 | 1409 | 1 | F | PC1 MU - General Editor notes update in 38.521-3 FR2 Rx tests | 17.6.0 |
| 2022-09 | RAN#97 | R5-225669 | 1396 | 1 | F | Rel-15 Beam correspondence test tolerance definition | 17.6.0 |
| 2022-09 | RAN#97 | R5-225678 | 1410 | 1 | F | Update of FR2 5CC to 8CCs Transmission Test Cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-225705 | 1397 | 1 | F | Introduction of Output power requirements for DC_3A_n8A | 17.6.0 |
| 2022-09 | RAN#97 | R5-225706 | 1401 | 1 | F | Introduction of Spurious emissions band UE co-existence Test description for DC_3A_n8A | 17.6.0 |
| 2022-09 | RAN#97 | R5-225707 | 1419 | 1 | F | Update 6.5B.3.3.2 for DC_5A_n2A | 17.6.0 |
| 2022-09 | RAN#97 | R5-225708 | 1424 | 1 | F | Corrections on test configuration table in spurious emission band UE co-existence for Rel-16 inter-band EN-DC configuration | 17.6.0 |
| 2022-09 | RAN#97 | R5-225709 | 1421 | 1 | F | Update for reference sensitivity for DC_48A_n66A | 17.6.0 |
| 2022-09 | RAN#97 | R5-225736 | 1422 | 1 | F | Update for reference sensitivity for DC_5A_n77A | 17.6.0 |
| 2022-09 | RAN#97 | R5-225799 | 1406 | 1 | F | Correction of Power Class 2 test requirements in 6.2B.1.3.5 for Interband EN-DC FDD and TDD Duplex-mode | 17.6.0 |
| 2022-09 | RAN#97 | R5-225800 | 1411 | 1 | F | Replacing the word LTE by E-UTRA in description of exception requirement | 17.6.0 |
| 2022-09 | RAN#97 | R5-225801 | 1439 | 1 | F | Removal of brackets for DCI format for Tx test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-225802 | 1445 | 1 | F | Introduction of test section for UL MIMO EN-DC MPR with FR1 and FR2 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225803 | 1446 | 1 | F | Introduction of test section for UL MIMO EN-DC A-MPR with FR1 and FR2 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225804 | 1447 | 1 | F | Introduction of test tase for UL MIMO EN-DC MPR in FR2 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225805 | 1448 | 1 | F | Introduction of test tase for UL MIMO EN-DC A-MPR in FR2 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225806 | 1449 | 1 | F | P-MaxUE-FR1-r15 correction for PC2 ENDC test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-225807 | 1420 | 1 | F | Update for ref sensitivity for R15_combos | 17.6.0 |
| 2022-09 | RAN#97 | R5-225808 | 1441 | 1 | F | Correction of reference sensitivity test case | 17.6.0 |
| 2022-09 | RAN#97 | R5-225881 | 1440 | 1 | F | Addition of new test case additional spurious for FR2 | 17.6.0 |
| 2022-12 | RAN#98 | R5-226296 | 1451 |  | F | Move 6.5B.4.4 which is in front of 6.5B.3.5 to be after 6.5B.4.3 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226297 | 1452 |  | F | Clause styles and clause number correction in 6.6B.5 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226480 | 1454 |  | F | Introduction of reference sensitivity for three bands within FR1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226744 | 1462 |  | F | Clean-up pending R15 configurations in clause 5.3 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226746 | 1464 |  | F | Clean-up pending R16 configurations in clause 5 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226747 | 1465 |  | F | Clean-up pending R17 configurations in clause 5 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226749 | 1467 |  | F | Clean-up pending R16 configurations in clause 6 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226750 | 1468 |  | F | Clean-up pending R17 configurations in clause 6 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226751 | 1469 |  | F | Clean-up pending R15 configurations in clause 7 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226752 | 1470 |  | F | Clean-up pending R16 configurations in clause 7 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226754 | 1471 |  | F | Editorial correction to the wording used for NR FRx | 17.7.0 |
| 2022-12 | RAN#98 | R5-226761 | 1472 |  | F | Editorial correction to editor note of completed TC6.3B.1.4_1.x minimum output power for EN-DC FR2 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226815 | 1473 |  | F | Update of FR2 Test Case 6.4B.2.4.4 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226886 | 1474 |  | F | New test case addition: 6.4B.2.4.1a Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC) | 17.7.0 |
| 2022-12 | RAN#98 | R5-226887 | 1475 |  | F | New test case addition: 6.5B.2.4.1a Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC) | 17.7.0 |
| 2022-12 | RAN#98 | R5-226888 | 1476 |  | F | New test case addition: 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | 17.7.0 |
| 2022-12 | RAN#98 | R5-226889 | 1477 |  | F | New test case addition: 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC) | 17.7.0 |
| 2022-12 | RAN#98 | R5-226890 | 1478 |  | F | New test case addition: 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | 17.7.0 |
| 2022-12 | RAN#98 | R5-226939 | 1479 |  | F | New test case addition: 6.2B.2.4a UE maximum output power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC) | 17.7.0 |
| 2022-12 | RAN#98 | R5-226941 | 1480 |  | F | Measurement uncertainties and test tolerances for mpr-PowerBoost tests 6.4B.2.4.1a, 6.5B.2.4.1a, 6.5B.3.4.1a, 6.5B.3.4.2a and 6.5B.4.4a | 17.7.0 |
| 2022-12 | RAN#98 | R5-226994 | 1481 |  | F | Measurement uncertainties and test tolerances for test 6.2B.2.4a UE maximum output power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC) | 17.7.0 |
| 2022-12 | RAN#98 | R5-227085 | 1495 |  | F | Correction to test description of 6.2B.1.4D.1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227284 | 1498 |  | F | Corrections on reference sensitivity test requirements for EN-DC configurations | 17.7.0 |
| 2022-12 | RAN#98 | R5-227285 | 1499 |  | F | Corrections on reference sensitivity test requirements on TT for DC_28A_n7A-n78A | 17.7.0 |
| 2022-12 | RAN#98 | R5-227286 | 1500 |  | F | Corrections on reference sensitivity test requirements on TT for R15 EN-DC configurations | 17.7.0 |
| 2022-12 | RAN#98 | R5-227287 | 1501 |  | F | Corrections on REFSEN for inter-band CA configurations | 17.7.0 |
| 2022-12 | RAN#98 | R5-227289 | 1503 |  | F | Corrections on test configuration table in spurious emission band UE co-existence for Rel-17 inter-band EN-DC configuration | 17.7.0 |
| 2022-12 | RAN#98 | R5-227290 | 1504 |  | F | Corrections on general description for MSD test frequencies | 17.7.0 |
| 2022-12 | RAN#98 | R5-227786 | 1456 | 1 | F | 7.3B.2 cleaning up_R16 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227787 | 1466 | 1 | F | Clean-up pending R15 configurations in clause 6 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227788 | 1458 | 1 | F | Update Ref sense for R15 combos | 17.7.0 |
| 2022-12 | RAN#98 | R5-227789 | 1507 | 1 | F | Updated to clause 5 for simultaneous RxTx clarification | 17.7.0 |
| 2022-12 | RAN#98 | R5-227790 | 1453 | 1 | F | Update some frequency selections in test configuration table | 17.7.0 |
| 2022-12 | RAN#98 | R5-227791 | 1482 | 1 | F | Addition of new test case 6.3B.1.4_1.4 Minimum output power for inter-band EN-DC including FR2 - 5 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227792 | 1483 | 1 | F | Addition of new test case 6.3B.1.4_1.5 Minimum output power for inter-band EN-DC including FR2 - 6 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227793 | 1484 | 1 | F | Addition of new test case 6.3B.1.4_1.6 Minimum output power for inter-band EN-DC including FR2 - 7 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227794 | 1485 | 1 | F | Addition of new test case 6.3B.1.4_1.7 Minimum output power for inter-band EN-DC including FR2 - 8 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227795 | 1486 | 1 | F | Addition of new test case 6.4B.2.4.1_1.4 Error Vector Magnitude for inter-band EN-DC including FR2 - 5 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227796 | 1487 | 1 | F | Addition of new test case 6.4B.2.4.1_1.5 Error Vector Magnitude for inter-band EN-DC including FR2 - 6 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227797 | 1488 | 1 | F | Addition of new test case 6.4B.2.4.1_1.6 Error Vector Magnitude for inter-band EN-DC including FR2 - 7 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227798 | 1489 | 1 | F | Addition of new test case 6.4B.2.4.1_1.7 Error Vector Magnitude for inter-band EN-DC including FR2 - 8 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227799 | 1490 | 1 | F | Addition of new test case 6.4B.2.4.2_1.4 Carrier Leakage for inter-band EN-DC including FR2 - 5 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227800 | 1491 | 1 | F | Addition of new test case 6.4B.2.4.2_1.5 Carrier Leakage for inter-band EN-DC including FR2 - 6 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227801 | 1492 | 1 | F | Addition of new test case 6.4B.2.4.2_1.6 Carrier Leakage for inter-band EN-DC including FR2 - 7 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227802 | 1493 | 1 | F | Addition of new test case 6.4B.2.4.2_1.7 Carrier Leakage for inter-band EN-DC including FR2 - 8 NR CCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227803 | 1455 | 1 | F | Introduction of spurious emissions test cases for 18A_n77A and 18A_n78A | 17.7.0 |
| 2022-12 | RAN#98 | R5-227804 | 1497 | 1 | F | Corrections on MOP band edge relaxation for intra-band contiguous and non-contiguous EN-DC band combinations | 17.7.0 |
| 2022-12 | RAN#98 | R5-227952 | 1508 | 1 | F | Updates to EN-DC Enhanced Beam Corr test 6.6B.5 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227966 | 1450 | 1 | F | PC1 FR2 - Editor notes updates in 38.521-3 | 17.7.0 |
| 2022-12 | RAN#98 | R5-228032 | 1496 | 1 | F | Additions to the clarification on intra-band contiguous and non-contiguous EN-DC configurations | 17.7.0 |
| 2022-12 | RAN#98 | R5-228046 | 1461 | 1 | F | Clean-up pending R15 configurations in clause 5.2 | 17.7.0 |
| 2022-12 | RAN#98 | R5-228047 | 1463 | 1 | F | Clean-up pending R15 configurations in clause 5.5 | 17.7.0 |
| 2023-03 | RAN#99 | R5-230181 | 1511 | - | F | Missing MU and TT in annex F for Spurious co-existence EN-DC FR2 CA tests | 17.8.0 |
| 2023-03 | RAN#99 | R5-230246 | 1527 | - | F | Addition of reference sensitivity for new EN-DC comb within FR1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230313 | 1532 | - | F | Editorial - missing reference to 38.101 in section 7.3B | 17.8.0 |
| 2023-03 | RAN#99 | R5-230568 | 1534 | - | F | Correction of test tolerance for Tx power test cases | 17.8.0 |
| 2023-03 | RAN#99 | R5-230569 | 1535 | - | F | Move 6.4B.2.4.4D to be after 6.4B.2.4.4 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230570 | 1536 | - | F | Move 6.5B.4.4a to be after 6.5B.4.4 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230571 | 1537 | - | F | Editorial correction for content style in 6.6B.5.5 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230573 | 1539 | - | F | Addition of F.1.0 and F.1.1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230574 | 1540 | - | F | Addition of 6.2B.2.1 in F.3.2 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230659 | 1541 | - | F | Addition of 6.5E.3.1 General Spurious emissions for V2X | 17.8.0 |
| 2023-03 | RAN#99 | R5-230883 | 1544 | - | F | Update Configured Output Power Level for inter-band EN-DC | 17.8.0 |
| 2023-03 | RAN#99 | R5-230892 | 1545 | - | F | Update 6.2B.4.2.3.1 for DC_71A_n66A and DC_12A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230893 | 1546 | - | F | Update 6.2B.4.2.3.1 for DC_71A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230895 | 1547 | - | F | Update 6.2B.1.3 for R17 combo DC_71A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230899 | 1548 | - | F | Update for reference sensitivity for DC_48A_n66A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230901 | 1550 | - | F | Remove pending combo from 7.2B.2.3 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230906 | 1554 | - | F | Update ref sense min requirement for DC_71A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230907 | 1555 | - | F | Update ref sense min requirement for DC_71A_n66A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230940 | 1560 | - | F | Correction to reference sensitivity test configuration for DC_1A_n28A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230941 | 1561 | - | F | Correction to reference sensitivity test configuration for DC_8A_n41A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230943 | 1563 | - | F | Addition of reference sensitivity for DC_2A-66A_n5A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230948 | 1564 | - | F | Correction to reference sensitivity requirements for EN-DC with 4 Rx support | 17.8.0 |
| 2023-03 | RAN#99 | R5-230949 | 1565 | - | F | Correction to NR test SCS for DC_(n)71AA across clause 6 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230959 | 1567 | - | F | Update 6.2B.1.3 for R16 combos DC_71A_n66A and DC_12A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230978 | 1569 | - | F | Correction to the MOP measurement for simultaneous transmission | 17.8.0 |
| 2023-03 | RAN#99 | R5-231058 | 1573 | - | F | Introduction of reference sensitivity for 21A_n28A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231180 | 1576 | - | F | Add editors note to TC6.2B.3.4D with incomplete state | 17.8.0 |
| 2023-03 | RAN#99 | R5-231182 | 1578 | - | F | Update to R16 Configuration for DC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231294 | 1583 | - | F | Corrections on test requirements for reference sensitivity exceptions for DC_7A-20A_n1A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231301 | 1584 | - | F | Correction of maximum output power test case | 17.8.0 |
| 2023-03 | RAN#99 | R5-231312 | 1586 | - | F | Update of 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) for DC_25A_n41A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231670 | 1585 | 1 | F | Editorial correction of E-UTRA reference for FR2 test cases | 17.8.0 |
| 2023-03 | RAN#99 | R5-231671 | 1526 | 1 | F | Addition of delta TIBc for new EN-DC comb within FR1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231672 | 1523 | 1 | F | Addition of delta TIBc for new 3CC EN-DC comb within FR1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231673 | 1524 | 1 | F | Addition of reference sensitivity for new 3CC EN-DC comb within FR1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231674 | 1542 | 1 | F | Addition of MOP and spurious emissions for new 2CC EN-DC comb within FR1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231675 | 1543 | 1 | F | Updating spurious coex for inter-band EN-DC CA to add 5 to 8 CCs | 17.8.0 |
| 2023-03 | RAN#99 | R5-231676 | 1558 | 1 | F | Update Tx spurious co-exist for DC_71A_n66A and DC_12A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231677 | 1559 | 1 | F | General SE for DC_71A_n66A and DC_12A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231678 | 1549 | 1 | F | Remove pending combo from 7.2B.2.3 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231679 | 1552 | 1 | F | Update 7.3B.2.3 for DC_71A_n66A and DC_12A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231680 | 1556 | 1 | F | Update Tx spurious co-exist for DC_71A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231681 | 1557 | 1 | F | General SE for DC_71A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231682 | 1553 | 1 | F | Update 7.3B.2.3 for DC_71_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231684 | 1562 | 1 | F | Correction to reference sensitivity test configuration for DC_12A_n78A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231685 | 1579 | 1 | F | Update to R17 Configuration for DC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231686 | 1577 | 1 | F | Update to R15 Configuration for DC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231687 | 1572 | 1 | F | Introduction of spurious emissions test cases for 21A_n28A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231688 | 1574 | 1 | F | Introduction of DC_28A_n78A PC2 MOP test requirements | 17.8.0 |
| 2023-03 | RAN#99 | R5-231689 | 1529 | 1 | F | Updates for Table 7.3B.2.3.4.2.1-6 due to frequency selections in test configuration table partly non-implementable | 17.8.0 |
| 2023-03 | RAN#99 | R5-231690 | 1520 | 1 | F | Update of MOP TC for PC2 ENDC configurations | 17.8.0 |
| 2023-03 | RAN#99 | R5-231691 | 1521 | 1 | F | Addition of PC2 ENDC combo into 38.521-3 TC 7.3B.2 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231692 | 1587 | 1 | F | Introduction of EIRP with UL-Gaps test for EN-DC with FR2 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231693 | 1568 | 1 | F | Correction to time offset for TDD intra-band EN-DC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231694 | 1582 | 1 | F | Corrections on reference sensitivity for configuration DC_66A_n41A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231695 | 1580 | 1 | F | Corrections on applicability of minimum requirements for intra-band EN-DC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231783 | 1519 | 1 | F | Update of editors note for PC1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231840 | 1509 | 1 | F | Update switching time mask for UL tx switching for EN-DC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231847 | 1510 | 1 | F | PC1 FR2 - Editor notes updates in 38.521-3 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231860 | 1551 | 1 | F | Ref sensitivity correction for DC_1A_n77A and DC_21_n79A | 17.8.0 |
| 2023-03 | RAN#99 | R5-231864 | 1566 | 1 | F | Update of EN-DC reference sensitivity to handle simultaneous Rx/Tx capability | 17.8.0 |
| 2023-03 | RAN#99 | R5-231876 | 1570 | 1 | F | Clarification on power class of LTE band in 6.2B.4.1.3 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231887 | 1538 | 1 | F | Correction of referenced clause numbers in 7.5B.4_1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231892 | 1528 | 1 | F | Corrections of test requirement tables for spurious emission for UE co-existence for EN-DC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231965 | 1581 | 1 | F | Corrections on intra-band EN-DC configuration for DC_n41 | 17.8.0 |
| 2023-06 | RAN#100 | R5-232349 | 1594 | - | F | Editorial update Tx spurious co-existence for DC_71A_n2A | 17.9.0 |
| 2023-06 | RAN#100 | R5-232365 | 1596 | - | F | Correction to spurious emissions test cases for 21A_n28A | 17.9.0 |
| 2023-06 | RAN#100 | R5-232383 | 1598 | - | F | Correction of the DC_28A_n78A PC2 MOP test requirements | 17.9.0 |
| 2023-06 | RAN#100 | R5-232514 | 1601 | - | F | Update the Initial Conditions of four 6.2B.x TCs | 17.9.0 |
| 2023-06 | RAN#100 | R5-232609 | 1603 | - | F | Update Ref sense for DC_7A_n66A DC_7A_n71A DC_7A_n77A and DC_66A_n25A | 17.9.0 |
| 2023-06 | RAN#100 | R5-232623 | 1604 | - | F | Adding SE Coex Inter band ENDC FR2 UL-MIMO test case | 17.9.0 |
| 2023-06 | RAN#100 | R5-232700 | 1605 | - | F | Addition of 6.5E.4 Transmit intermodulation for V2X | 17.9.0 |
| 2023-06 | RAN#100 | R5-232754 | 1607 | - | F | Correction to test ID for PC2 fallback PC3 testing | 17.9.0 |
| 2023-06 | RAN#100 | R5-232772 | 1608 | - | F | Addition of reference sensitivity for 21A_n28A | 17.9.0 |
| 2023-06 | RAN#100 | R5-232840 | 1610 | - | F | Update of PC2 UE configured output power for inter-band EN-DC configurations | 17.9.0 |
| 2023-06 | RAN#100 | R5-232841 | 1611 | - | F | Update of PC2 UE maximum output power for inter-band EN-DC configurations | 17.9.0 |
| 2023-06 | RAN#100 | R5-233001 | 1612 | - | F | Correction to reference of RMC for E-UTRA TDD in FR1 EN-DC test cases | 17.9.0 |
| 2023-06 | RAN#100 | R5-233002 | 1613 | - | F | Correction to 6.2B.4.1.3 and editorial correction to Tx test cases | 17.9.0 |
| 2023-06 | RAN#100 | R5-233025 | 1615 | - | F | Adding time delay to intra-band EN-DC test cases | 17.9.0 |
| 2023-06 | RAN#100 | R5-233157 | 1617 | - | F | Update 7.3B.3.3.1 for R16 DC combos | 17.9.0 |
| 2023-06 | RAN#100 | R5-233168 | 1618 | - | F | Addition of Additional Spurious Emissions FR2 CA test cases | 17.9.0 |
| 2023-06 | RAN#100 | R5-233175 | 1619 | - | F | Update of in-band blocking for CA test cases | 17.9.0 |
| 2023-06 | RAN#100 | R5-233182 | 1620 | - | F | Update to R15 Configuration for DC | 17.9.0 |
| 2023-06 | RAN#100 | R5-233183 | 1621 | - | F | Update to R16 Configuration for DC | 17.9.0 |
| 2023-06 | RAN#100 | R5-233190 | 1623 | - | F | Additional editors note to Power Boost relevant test cases | 17.9.0 |
| 2023-06 | RAN#100 | R5-233520 | 1602 | 1 | F | Update Ref sense for DC_38A_n78A, DC_18A_n77A and DC_19A_n77A | 17.9.0 |
| 2023-06 | RAN#100 | R5-233545 | 1616 | 1 | F | Clarification of spurious emission testing configuration - Part 3 | 17.9.0 |
| 2023-06 | RAN#100 | R5-233563 | 1599 | 1 | F | Addition of new CADC MOP TC | 17.9.0 |
| 2023-06 | RAN#100 | R5-233564 | 1600 | 1 | F | Addition of new CADC TC 6.3B.3.4_1.1 | 17.9.0 |
| 2023-06 | RAN#100 | R5-233576 | 1625 | 1 | F | Corrections on test requirements for MSD due to dual uplink for EN-DC | 17.9.0 |
| 2023-06 | RAN#100 | R5-233577 | 1624 | 1 | F | Corrections on higher power class indication for EN-DC configuration | 17.9.0 |
| 2023-06 | RAN#100 | R5-233638 | 1589 | 1 | F | FR2 MUs - Editor notes updates in 38.521-3 | 17.9.0 |
| 2023-06 | RAN#100 | R5-233703 | 1590 | 1 | F | Update PC2 MSD minimum requirements and test requirements for DC_2A_n77A, DC_13A_n77A, and DC_66A_n77A | 17.9.0 |
| 2023-06 | RAN#100 | R5-233724 | 1595 | 1 | F | NSA beam correspondence test applicability inconsistent with SA test | 17.9.0 |
| 2023-06 | RAN#100 | R5-233725 | 1606 | 1 | F | Correction to 6.2B.4.1.3 configured output power for EN-DC | 17.9.0 |
| 2023-09 | RAN#101 | R5-234040 | 1629 | - | F | Correction to ref sense configuration for DC_71A_n2A | 17.10.0 |
| 2023-09 | RAN#101 | R5-234054 | 1632 | - | F | Update 6.2B.4.2.3.1 for DC_48A_n46 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234055 | 1633 | - | F | Update 6.2B.1.3 for DC_48A_n46A | 17.10.0 |
| 2023-09 | RAN#101 | R5-234057 | 1634 | - | F | Update Ch5 for DC_48A_n46A | 17.10.0 |
| 2023-09 | RAN#101 | R5-234117 | 1637 | - | F | Correction of reference sensitivity test point for DC_48A_n66A | 17.10.0 |
| 2023-09 | RAN#101 | R5-234182 | 1642 | - | F | Correction for NSA band combo DC_7A_n71A in 7.3B.2.3 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234185 | 1645 | - | F | Correction for NSA band combos DC_19A_n77A and DC_1A_n77A in 7.3B.2.3 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234227 | 1646 | - | F | Update the Initial Conditions of 6.3B.x and 6.4B.x TCs | 17.10.0 |
| 2023-09 | RAN#101 | R5-234564 | 1647 | - | F | Update to R15 Configuration for DC | 17.10.0 |
| 2023-09 | RAN#101 | R5-234777 | 1651 | - | F | Addition of new test case 6.4B.2.4.3_1.4 In-band Emissions for inter-band EN-DC including FR2 (5 NR CCs) | 17.10.0 |
| 2023-09 | RAN#101 | R5-234778 | 1652 | - | F | Addition of new test case 6.4B.2.4.3_1.5 In-band Emissions for inter-band EN-DC including FR2 (6 NR CCs) | 17.10.0 |
| 2023-09 | RAN#101 | R5-234790 | 1653 | - | F | Update of higher power limit requirements for inter-band EN-DC | 17.10.0 |
| 2023-09 | RAN#101 | R5-234912 | 1655 | - | F | Correction to message exception in EVM test case for intra-band EN-DC | 17.10.0 |
| 2023-09 | RAN#101 | R5-234913 | 1656 | - | F | Editorial correction to Reference sensitivity for EN-DC | 17.10.0 |
| 2023-09 | RAN#101 | R5-235069 | 1658 | - | F | Correction to referred clause number of CADC MOP TCs | 17.10.0 |
| 2023-09 | RAN#101 | R5-235141 | 1661 | - | F | Correction of Refsens FR1 EN-DC | 17.10.0 |
| 2023-09 | RAN#101 | R5-235148 | 1663 | - | F | Editorial correction of Refsens for Rel-16 configuration | 17.10.0 |
| 2023-09 | RAN#101 | R5-235202 | 1664 | - | F | Correction to title of 6.2B.1.4D.1 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235203 | 1665 | - | F | Updated to editors note for EN-DC FR2 TC completion status | 17.10.0 |
| 2023-09 | RAN#101 | R5-235623 | 1662 | 1 | F | Editorial correction of UE co-existence Inter-band FR1 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235672 | 1643 | 1 | F | Correction for NSA band combo DC_14A_n77A in 7.3B.2.3 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235673 | 1659 | 1 | F | Corrections on cross-band isolation MSD for the more than 2 bands configuration | 17.10.0 |
| 2023-09 | RAN#101 | R5-235674 | 1660 | 1 | F | Corrections on test requirements for MSD due to dual uplink for R15 EN-DC configurations | 17.10.0 |
| 2023-09 | RAN#101 | R5-235675 | 1648 | 1 | F | Update to R16 Configuration for DC | 17.10.0 |
| 2023-09 | RAN#101 | R5-235676 | 1630 | 1 | F | Update 7.3B.2.0 for DC_48A_n46A | 17.10.0 |
| 2023-09 | RAN#101 | R5-235677 | 1666 | 1 | F | Update of EIRP with UL-Gaps test for EN-DC with FR2 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235750 | 1628 | 1 | F | FR2 MUs - Editor notes updates in 38.521-3 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235820 | 1657 | 1 | F | Addition of new CADC MOP TC for 8 NR CCs | 17.10.0 |
| 2023-09 | RAN#101 | R5-235870 | 1654 | 1 | F | Clarification of unwanted emission testing configuration - Part 3 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235958 | 1636 | 1 | F | Correction of 7.3B.2.3 for exception and no exception requirements | 17.10.0 |
| 2023-09 | RAN#101 | R5-235959 | 1638 | 1 | F | Correction of 6.5B.3.3.1 for exception and non-exception requirements | 17.10.0 |
| 2023-09 | RAN#101 | R5-235960 | 1639 | 1 | F | Correction of requirements of Spurious emissions for UE co-existence of intra band ENDC | 17.10.0 |
| 2023-09 | RAN#101 | R5-235961 | 1640 | 1 | F | Correction of 6.5B.3.3.2 for exception and non-exception requirements | 17.10.0 |
| 2023-09 | RAN#101 | R5-234097 | 1635 | - | F | Introduction of DC for n78 and n79 PC2 MOP test requirements | 18.0.0 |
| 2023-09 | RAN#101 | R5-235618 | 1650 | 1 | F | Updates to spurious emissions UE coexistence test cases as part of introduction of LTE Band 54 | 18.0.0 |
| 2023-12 | RAN#102 | R5-236067 | 1667 |  | F | FR2 MUs - Editor notes updates for Tx test cases in 38.521-3 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236328 | 1672 |  | F | Missing notes for DC_1A_n41A test IDs in 7.3B.2.3 test configuration table | 18.1.0 |
| 2023-12 | RAN#102 | R5-236339 | 1676 |  | F | Defined MU and TT for EUTRA and NR UL Switching test case 6.3B.3_1.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236340 | 1677 |  | F | Editorial correction in test procedure of NSA ACLR test 6.5B.2.1.3 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236351 | 1679 |  | F | Addition of max power requirements for several EN-DC combos | 18.1.0 |
| 2023-12 | RAN#102 | R5-236352 | 1680 |  | F | Addition of spurious emissions test for several EN-DC combos | 18.1.0 |
| 2023-12 | RAN#102 | R5-236369 | 1681 |  | F | Update minimum requirements for additional ENDC configurations | 18.1.0 |
| 2023-12 | RAN#102 | R5-236370 | 1682 |  | F | Update test configurations and test requirements for additional three band ENDC configurations | 18.1.0 |
| 2023-12 | RAN#102 | R5-236461 | 1685 |  | F | New addition of RF Rel-16 Test Case 6.2B.2.4_1.2 UE Maximum Output power reduction for inter-band EN-DC including FR2 (3 NR CCs) | 18.1.0 |
| 2023-12 | RAN#102 | R5-236467 | 1687 |  | F | Addition of new test case 6.4B.2.4.3_1.6 In-band Emissions for inter-band EN-DC including FR2 (7 NR CCs) | 18.1.0 |
| 2023-12 | RAN#102 | R5-236468 | 1688 |  | F | Addition of new test case 6.4B.2.4.3_1.7 In-band Emissions for inter-band EN-DC including FR2 (8 NR CCs) | 18.1.0 |
| 2023-12 | RAN#102 | R5-236502 | 1691 |  | F | Addition of REFSENS for 2CC EN-DC for n79 in PC2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236545 | 1693 |  | F | Update to R15 common section | 18.1.0 |
| 2023-12 | RAN#102 | R5-236661 | 1697 |  | F | Updating FR2 MOP for 5UL-8UL CA test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-236816 | 1702 |  | F | Correction to DL RMC for TDD E-UTRA | 18.1.0 |
| 2023-12 | RAN#102 | R5-236846 | 1704 |  | F | New addition of RF Rel-16 Test Case 6.2B.2.4_1.3 UE Maximum Output power reduction for inter-band EN-DC including FR2 (4 NR CCs) | 18.1.0 |
| 2023-12 | RAN#102 | R5-236946 | 1705 |  | F | Corrections to 7.3A.3 and 7.3B.3 on delta RIB value for CA and DC in REFSENS requirement | 18.1.0 |
| 2023-12 | RAN#102 | R5-237261 | 1709 |  | F | Editorial correction of FR2 spurious emissions test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237636 | 1690 | 1 | F | Update of REFSENS for 3CC EN-DC for n78 and n79 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237663 | 1694 | 1 | F | Update to R16 Configuration for DC | 18.1.0 |
| 2023-12 | RAN#102 | R5-237664 | 1678 | 1 | F | Addition of reference sensitivity test for several EN-DC combos | 18.1.0 |
| 2023-12 | RAN#102 | R5-237665 | 1695 | 1 | F | Update to R17 Configuration for DC | 18.1.0 |
| 2023-12 | RAN#102 | R5-237666 | 1700 | 1 | F | Correction to test condition for test frequency restriction of DC_xA_n28A | 18.1.0 |
| 2023-12 | RAN#102 | R5-237667 | 1711 |  | F | Correction to message contents in Rx test cases for inter-band EN-DC with FR1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237668 | 1708 | 1 | F | Correction of Refsens EN-DC within FR1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237669 | 1710 | 1 | F | Update to Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations | 18.1.0 |
| 2023-12 | RAN#102 | R5-237732 | 1707 | 1 | F | Editors notes update in FR2 UL MIMO test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237733 | 1668 | 1 | F | FR2 MUs - Editor notes updates for Rx test cases in 38.521-3 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237784 | 1689 | 1 | F | Adding DeltaTIB for DC_1-3-7-28_n78 and DC_1-7_28-n78 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237862 | 1706 | 1 | F | Corrections to spurious emissions co-existence for 2 UL Inter-band CA (38.521-3) | 18.1.0 |
| 2023-12 | RAN#102 | R5-237955 | 1692 | 1 | F | Correction to 6.5B.3.3.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237963 | 1703 | 1 | F | Clarifications for FR2 testing with NR-DC and NR-CA | 18.1.0 |