### 5.5A.3	Configurations for inter-band CA

Table 5.5A.3-1: Void

Table 5.5A.3-2: Void

Table 5.5A.3-3: Void

#### 5.5A.3.0	General

For the NR inter-band CA configurations in sub-clause 5.5A.3, when the capability [BandCombination-UplinkTxSwitch-r18] is present, three or four bands can be configured in the uplink with simultaneous uplink transmission on up to two bands, and the corresponding inter-band CA requirements with uplink assigned to one or two bands shall apply. For each uplink band pair in the NR inter-band CA configurations, according to the capability [uplinkTxSwitchingOptionForBandPair],

–	if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with uplink assigned to one band on band X or band Y apply;

–	if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with uplink CA between the two uplink bands apply. For a UE supporting  [BandOrdering1T1Tto1T1T] for parallel uplink transmission switching for a band combination consisting with four different bands, the UE is allowed to report capability [preferredBandPairs] via band-ordering approach to indicate the UE’s preferred switching band pairs for which it supports dualUL and perform the switching case configured by network.

#### 5.5A.3.1	Configurations for inter-band CA (two bands)

##### Table 5.5A.3.1-1a ~ Table 5.5A.3.1-1e

Table 5.5A.3.1-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
| CA_n1B-n3A | CA_n1A-n3A | n1 | CA_n1B_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | CA_n1B_BCS0 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3(2A) | CA_n1A-n3A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
| CA_n1(2A)-n3A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3(2A) | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
| CA_n1(2A)-n3B | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
| CA_n1A-n5A | CA_n1A-n5A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1(2A)-n5A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1A-n7A | CA_n1A-n7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7B | CA_n1A-n7A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1(2A)-n7A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n8A | CA_n1A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1(2A)-n8A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n18A | CA_n1A-n18A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
| CA_n1A-n20A | CA_n1A-n20A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n26A | CA_n1A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n26(2A) | CA_n1A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n28A | CA_n1A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n1(2A)-n28A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n38A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n40A | CA_n1A-n40A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n40B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
| CA_n1A-n41A | n418 CA_n1A-n41A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n46A | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
| CA_n1A-n46C | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n1A-n46D | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n1A-n46(2A) | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
| CA_n1A-n67A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n74A | CA_n1A-n74A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n1A-n75A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n77A | n778,9 CA_n1A-n77A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n77(2A) | n778,9 CA_n1A-n77A8 CA_n77(2A)8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  | n778 CA_n1A-n77A8 | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n1A-n77(3A) | CA_n1A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n1A-n78A | n18 n788,9 CA_n1A-n78A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 3 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  | CA_n78(2A) CA_n1A-n78A | n1 | 5, 10, 15, 20 | 2 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n1A-n78C | CA_n78C CA_n1A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  | CA_n78C CA_n1A-n78A CA_n1A-n78C | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n1(2A)-n78A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n79A | n798,9 CA_n1A-n79A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n79C | CA_n1A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n1(2A)-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n79 | 40, 60, 80, 100 |  |
|  |  | n1 | CA_n1(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1(2A)-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n1 | CA_n1(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n1A-n102A | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n102(2A) | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n102B | CA_n1A-n102A CA_n1A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n102C | CA_n1A-n102A CA_n1A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n102D | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n102E | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n1A-n105A | CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

Table 5.5A.3.1-1b: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n5A | CA_n2A-n5A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n2A-n5B | CA_n2A-n5A CA_n5B | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | CA_n5B_BCS0 |  |
| CA_n2(2A)-n5A | CA_n2A-n5A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n2A-n7A | CA_n2A-n7A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n2A-n7(2A) | CA_n2A-n7A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
| CA_n2(2A)-n7A | CA_n2A-n7A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n2A-n12A | CA_n2A-n12A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n2(2A)-n12A | CA_n2A-n12A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n2A-n14A | CA_n2A-n14A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n2(2A)-n14A | CA_n2A-n14A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n2A-n29A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n2(2A)-n29A | - | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n2A-n30A | CA_n2A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n30A | CA_n2A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n38A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n38 | 5, 10, 15, 20, 40 |  |
| CA_n2A-n41A | CA_n2A-n41A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n2(2A)-n41A | CA_n2A-n41A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
| CA_n2A-n48B | CA_n48B CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n48C | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n2A-n48(2A) | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n48(A-B) | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n2A-n48(A-C) | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(A-C)_BCS0 |  |
| CA_n2A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n66A | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n66(2A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n66(2A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n66(3A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n66(3A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n66B | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n2A-n71A | CA_n2A-n71A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | - | n2 | See n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n71A | CA_n2A-n71A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | - | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n77A13,14 | n778,9 CA_n2A-n77A8, 13,14 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n77(2A) | n778,9 CA_n2A-n77A8 CA_n77(2A)7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n77B | - | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n2A-n77C | n778, 9 CA_n77C CA_n2A-n77A8 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2(2A)-n77A | n778, 9 CA_n2A-n77A8 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n77B | - | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n2(2A)-n77(2A) | n778,9 CA_n2A-n77A8 CA_n77(2A)7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n77(3A) | CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n2(2A)-n77C | n778, 9 CA_n77C CA_n2A-n77A8 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n78A | CA_n2A-n78A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | See n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n78(2A) | CA_n2A-n78A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n2 | See n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

Table 5.5A.3.1-1c: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n5A | CA_n3A-n5A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n5A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n3A-n7A | CA_n3A-n7A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7B | CA_n3A-n7A CA_n7B | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n3(2A)-n7A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | CA_n3(2A)_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3B-n7A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3B-n7B | CA_n3A-n7A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n3 | CA_n3B _BCS 4 and 5 | 4 and 5 |
|  |  | n7 | CA_n7B_BCS 4 and 5 |  |
| CA_n3A-n8A | CA_n3A-n8A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3(2A)-n8A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n3A-n18A | CA_n3A-n18A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
| CA_n3A-n20A | CA_n3A-n20A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n26A | CA_n3A-n26A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n3A-n26(2A) | CA_n3A-n26A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3B-n26A | CA_n3A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n3B-n26(2A) | CA_n3A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3A-n28A | CA_n3A-n28A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 | 3 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3B-n28A | CA_n3A-n28A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n28A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n34A | CA_n3A-n34A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n34 | 5, 10, 15 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n38A | CA_n3A-n38A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n38 | 5, 10, 15, 20, 40 |  |
| CA_n3B-n38A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3(2A)-n38A | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A-n40A | CA_n3A-n40A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 2 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41A | n418,9 CA_n3A-n41A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41B | CA_n3A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
| CA_n3A-n41C | CA_n41C CA_n3A-n41A CA_n3A-n41C | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n3A-n41(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41(2A)_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 |  |
| CA_n3A-n67A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n74A | CA_n3A-n74A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n3A-n75A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n77A | n778,9 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n77(2A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n3A-n77(3A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n3A-n78A | n38 n788,9 CA_n3A-n78A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n78C | CA_n78C CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  | CA_n78C CA_n3A-n78A CA_n3A-n78C | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n3A-n78(2A) | CA_n3A-n78A CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n3(2A)-n78A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | CA_n3(2A)_BCS1 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n78A | CA_n3A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3B-n78(2A) | CA_n3A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n3A-n79A | n38 n798,9 CA_n3A-n79A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3(2A)-n79A | CA_n3A-n79A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | CA_n3(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n79C | n38 n798,9 CA_n79C8 CA_n3A-n79A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3(2A)-n79C | CA_n3A-n79A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n3 | CA_n3(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3B-n79A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3B-n79C | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3A-n102A | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n3A-n102(2A) | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n3A-n102B | CA_n3A-n102A CA_n3A-n102B | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n3A-n102C | CA_n3A-n102A CA_n3A-n102C | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n3A-n102D | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n3A-n102E | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n3A-n105A | CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

Table 5.5A.3.1-1d: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n7A | CA_n5A-n7A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n7B | CA_n5A-n7A CA_n7B | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n5A-n8A15 | - | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |
| CA_n5A-n12A | CA_n5A-n12A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n5B-n12A | CA_n5A-n12A CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n5A-n14A | CA_n5A-n14A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n5B-n14A | CA_n5A-n14A CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n5A-n25A | CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25(2A) | CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
| CA_n5A-n28A | CA_n5A-n28A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n5A-n29A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n5B-n29A | CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n5A-n30A | CA_n5A-n30A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n5A-n40A | CA_n5A-n40A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n40 | 55, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80,90,100 |  |
| CA_n5A-n41A | CA_n5A-n41A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
| CA_n5A-n48(2A) | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n5A-n48B | CA_n48B CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n5A-n48C | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n5A-n48(A-B) | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n5A-n66A | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5B-n66A | CA_n5A-n66A CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n66(2A) | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n66(3A) | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n5B-n66(2A) | CA_n5A-n66A CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n71A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n77A | n778,9 CA_n5A-n77A8,13,14 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n77B | CA_n5A-n77A n778,9 | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n5A-n77(2A) | n778,9 CA_n5A-n77A8 CA_n77(2A)8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n5A-n77(3A) | n778,9 CA_n77(2A)8 CA_n5A-n77A8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n5(2A)-n77A | n778,9 CA_n5A-n77A8 | n5 | CA_n5(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n77C | 778,9 CA_n5A-n77A8 CA_n77C | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5(2A)-n77C | n778,9 CA_n77C CA_n5A-n77A8 | n5 | CA_n5(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | CA_n5(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5B-n77A | n778,9 CA_n5A-n77A8 CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5B-n77C | n778,9 CA_n5A-n77A8 CA_n5B CA_n77C | n5 | CA_n5B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | CA_n5B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n78A | n788,9 CA_n5A-n78A8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n78(2A) | n788,9 CA_n5A-n78A8 CA_n78(2A)8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n5A-n78C | CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n5A-n79A | CA_n5A-n79A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n79C | CA_n5A-n79A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

Table 5.5A.3.1-1e: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n8A | CA_n7A-n8A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n7A-n12A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n7A-n20A | CA_n7A-n20A | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n25A | CA_n7A-n25A | n7 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A) | CA_n7A-n25A | n7 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
| CA_n7(2A)-n25A | CA_n7A-n25A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n25(2A) | CA_n7A-n25A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
| CA_n7A-n26A | CA_n7A-n26A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n7A-n26(2A) | CA_n7A-n26A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n7B-n26A | CA_n7A-n26A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n7B-n26(2A) | CA_n7A-n26A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n7A-n28A | CA_n7A-n28A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n7B-n28A | CA_n7A-n28A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n7A-n40A | CA_n7A-n40A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46A | CA_n7A-n46A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
| CA_n7A-n46C | CA_n7A-n46A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n7A-n46D | CA_n7A-n46A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n7A-n46(2A) | CA_n7A-n46A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
| CA_n7A-n66A | CA_n7A-n66A | n7 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 10, 15, 20, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n66(2A) | CA_n7A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7(2A)-n66A | CA_n7A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n66(2A) | CA_n7A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7A-n67A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n75A | - | n7 | 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n77A | n778,9 CA_n7A-n77A8,13,14 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n77A | n778,9 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n77(2A) | n778,9 CA_n77(2A)8 CA_n7A-n77A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n7(2A)-n77(2A) | n778,9 CA_n77(2A)8 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n77(3A) | n778,9 CA_n77(2A)8 CA_n7A-n77A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7(2A)-n77(3A) | n778,9 CA_n77(2A)8 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n78A | n788,9 CA_n7A-n78A8 | n7 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n78C | n788,9 CA_n7A-n78A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n7B-n78A | n788 CA_n7A-n78A8 CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n7 | CA_n7B_BCS4 and 5 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7B-n78(2A) | CA_n7A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | CA_n7B_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n7A-n78(2A) | n788,9 CA_n7A-n78A8 CA_n78(2A)8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n7(2A)-n78A | n788,9 CA_n7A-n78A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | CA_n7(2A)_BCS0 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n7 | CA_n7(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7(2A)-n78(2A) | n788,9 CA_n7A-n78A8 CA_n78(2A) | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | CA_n7(2A)_BCS0 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n7 | CA_n7(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n7A-n79A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n79C | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n7A-n102A | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n7A-n102(2A) | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n7A-n102B | CA_n7A-n102A CA_n7A-n102B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n7A-n102C | CA_n7A-n102A CA_n7A-n102C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n7A-n102D | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n7A-n102E | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n7A-n105A | CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n8A-n20A | CA_n8A-n20A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
| CA_n8A-n28A | CA_n8A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n8A-n34A | n88 n348,9 CA_n8A-n34A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n34 | 5, 10, 15 |  |
| CA_n8A-n38A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n8A-n39A | n88 n398,9 CA_n8A-n39A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n8A-n40A | n88 n408,9 CA_n8A-n40A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n41A | CA_n8A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n41C | CA_n41C CA_n8A-n41A CA_n8A-n41C | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n8A-n75A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n8 | 5, 10,15, 20 | 1 |
|  |  | n75 | 5, 10,15, 20, 25, 30, 40, 50 |  |
| CA_n8A-n77A | n778,9 CA_n8A-n77A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n77(2A) | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n8A-n78A | n788,9 CA_n8A-n78A8,13 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n78C | CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n78C CA_n8A-n78A CA_n8A-n78C | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n8A-n79A | n798,9 CA_n8A-n79A8 CA_n79C8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 10, 20, 40, 50, 60, 80, 100 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n79C | CA_n79C CA_n8A-n79A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  | CA_n79C CA_n8A-n79A CA_n8A-n79C | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS0 |  |

##### Table 5.5A.3.1-1f ~ Table 5.5A.3.1-1j

Table 5.5A.3.1-1f: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n12A-n25A | CA_n12A-n25A | n12 | 5, 10, 15 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n12A-n30A | CA_n12A-n30A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n12A-n41A | - | n12 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n48A | - | n12 | 5, 10, 15 | 0 |
|  |  | n48 | 10, 15, 20, 30, 40 |  |
| CA_n12A-n66A | CA_n12A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n12A-n66(2A) | CA_n12A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n12A-n66(3A) | CA_n12A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n12A-n71A | - | n12 | 5, 10, 15 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n12A-n77A | n778, 9 CA_n12A-n77A8 | n12 | 5, 10, 15 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n12 | n12 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n12A-n77B | - | n12 | n12 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n12A-n77C | - | n12 | n12 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n12A-n77(2A) | n778, 9 CA_n12A-n77A8 | n12 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n78A | CA_n12A-n78A | n12 | 5, 10, 15 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n78(2A) | CA_n12A-n78A | n12 | 5, 10, 15 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n13A-n25A | CA_n13A-n25A | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n13A-n66A | CA_n13A-n66A | n13 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n13 | 5, 10, | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n13A-n77A | n778, 9 CA_n13A-n77A8 | n13 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n77(2A) | n778,9 CA_n77(2A)8 CA_n13A-n77A8 | n13 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n13A-n77C | n778,9 CA_n77C CA_n13A-n77A8 | n13 | 5, 10 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n14A-n30A | CA_n14A-n30A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n14A-n66A | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n66(2A) | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n14A-n66(3A) | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n14A-n77A | n778, 9 CA_n14A-n77A8 | n14 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n77(2A) | n778, 9 CA_n14A-n77A8 | n14 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n28A | CA_n18A-n28A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
| CA_n18A-n40A | CA_n18A-n40A | n18 | 5, 10, 15 | 0 |
|  |  | n40 | 10, 15, 20, 30, 40 |  |
| CA_n18A-n41A | n418 CA_n18A-n41A8 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n74A | CA_n18A-n74A | n18 | 5, 10, 15 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n18A-n77A | n778 CA_n18A-n77A8 | n18 | 5, 10, 15 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n18A-n77(2A) | n778 CA_n18A-n77A | n18 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n18A-n77(3A) | CA_n18A-n77A | n18 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n18A-n78A | CA_n18A-n78A | n18 | 5, 10, 15 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n18A-n78(2A) | CA_n18A-n78A | n18 | 5, 10, 15 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

Table 5.5A.3.1-1g: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n20A-n28A | CA_n20A-n28A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n20 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n20 | 5, 10, 15, 20 | 2 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n20A-n40A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n20A-n67A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n20A-n75A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n20 | 5, 10,15, 20 | 1 |
|  |  | n75 | 5, 10,15, 20, 25, 30, 40, 50 |  |
| CA_n20A-n78A | CA_n20A-n78A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n20A-n78C | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n20A-n78(2A) | CA_n20A-n78A CA_n78(2A) | n20 | See n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n24A-n41A | CA_n24A-n41A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 |  |
| CA_n24A-n48A | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n24A-n48B | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n24A-n48(3A) | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n24A-n77A | CA_n24A-n77A | n24 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n24A-n77C | CA_n24A-n77A | n24 | 5, 10 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n24A-n77(2A) | CA_n24A-n77A | n24 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n25A-n29A | - | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n25A-n38A | CA_n25A-n38A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A | CA_n25A-n38A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n41A13,14 | n418,9 CA_n25A-n41A8, 13,14 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A | n418,9 CA_n25A-n41A8 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C | n418,9 CA_n25A-n41A8 CA_n25A-n41C CA_n41C8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | CA_n41C_BCS2 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A) | n418,9 CA_n25A-n41A 8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | CA_n41(2A)_BCS3 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
| CA_n25A-n41C | n418,9 CA_n25A-n41A8 CA_n41C8 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  | n418,9 CA_n25A-n41A8 CA_n41C8 CA_n25A-n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
| CA_n25A-n41(2A) | n418,9 CA_n25A-n41A8 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS3 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
| CA_n25A-n41(3A) | n418,9 CA_n25A-n41A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(3A)_BCS0 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 |  |
| CA_n25A-n41(A-C) | n418,9 CA_n25A-n41A8 CA_n41C8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(A-C)_BCS0 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 |  |
| CA_n25(2A)-n41(3A) | n418,9 CA_n25A-n41A8 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 |  |
| CA_n25(2A)-n41(A-C) | n418,9 CA_n41C8 CA_n25A-n41A8 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 |  |
| CA_n25A-n46A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
| CA_n25A-n48A | CA_n25A-n48A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n25A-n48(2A) | CA_n25A-n48A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n25A-n48C | CA_n25A-n48A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n25A-n66A | n25A8 n66A8 CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A) | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n66A | CA_n25A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 10, 15, 20, 30, 40 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66(2A) | CA_n25A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 2 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n25A-n71A | CA_n25A-n71A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71B | CA_n25A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n71(2A) | CA_n25A-n71A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n71A | CA_n25A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71(2A) | CA_n25A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n71B | CA_n25A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n77A | n778,9 CA_n25A-n77A8,13,14 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77(2A) | n778,9 CA_n77(2A)8 CA_n25A-n77A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n77(3A) | n778,9 CA_n77(2A)8 CA_n25A-n77A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n25(2A)-n77A | n778,9 CA_n25A-n77A8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n77(2A) | n778,9 CA_n77(2A)8 CA_n25A-n77A8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n77(3A) | n778,9 CA_n25(2A) CA_n77(2A)8 CA_n25A-n77A8 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n25A-n78A | n788,9 CA_n25A-n78A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n78(2A) | n788,9 CA_n25A-n78A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A)8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n25(2A)-n78A | n788,9 CA_n25A-n78A8 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n78(2A) | n788,9 CA_n25A-n78A8 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n25 | CA_n25(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n25A-n85A | n258 CA_n25A-n85A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n85A | CA_n25A-n85A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

Table 5.5A.3.1-1h: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n28A | CA_n26A-n28A16 | n26 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n26A-n29A | - | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n26A-n48A | CA_n26A-n48A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
| CA_n26A-n48(2A) | CA_n26A-n48A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n26A-n66A | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n66(2A) | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n26A-n66(3A) | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n26A-n70A | CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n71A | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n77A | CA_n26A-n77A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n26A-n78A | CA_n26A-n78A13 | n26 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n26(2A)-n78A | CA_n26A-n78A | n26 | CA_n26(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n26A-n78(2A) | CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n26(2A)-n78(2A) | CA_n26A-n78A | n26 | CA_n26(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n28A-n34A | n348,9 CA_n28A-n34A8 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n34 | 5, 10, 15 |  |
| CA_n28A-n38A | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n28A-n39A | n398 CA_n28A-n39A8 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n40A | CA_n28A-n40A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n40B | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
| CA_n28A-n41A | n418,9 CA_n28A-n41A8,13 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 30 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n41B | CA_n28A-n41A | n28 | 5, 10 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
| CA_n28A-n41C | n418,9 CA_n41C8 CA_n28A-n41A8 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  | CA_n41C CA_n28A-n41A CA_n28A-n41C | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n28A-n46A | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
| CA_n28A-n46C | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n28A-n46D | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n28A-n46(2A) | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
| CA_n28A-n50A | CA_n28A-n50A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n50 | 5, 10, 15, 20, 40, 50, 60, 801 |  |
| CA_n28A-n71A | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n28A-n74A | CA_n28A-n74A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n28A-n75A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20 | 1 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 2 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n28A-n77A | n778,9 CA_n28A-n77A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n77C | CA_n28A-n77A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n28A-n77(2A) | n778,9 CA_n77(2A)8 CA_n28A-n77A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n28A-n77(3A) | n778,9 CA_n77(2A) CA_n28A-n77A8 | n28 | 5, 10 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |
| CA_n28A-n78A | n788,9 CA_n28A-n78A8,13 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 30 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n78C | CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n28A-n78(2A) | CA_n78(2A) CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n28A-n79A | n798,9 CA_n28A-n79A8 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n79C | n798,9 CA_n79C8 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n28A-n94A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n94 | 5, 10, 15, 20 |  |
| CA_n28A-n102A | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n28A-n102(2A) | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n28A-n102B | CA_n28A-n102A CA_n28A-n102B | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n28A-n102C | CA_n28A-n102A CA_n28A-n102C | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n28A-n102D | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n28A-n102E | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n28A-n105A | - | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n29A-n30A | - | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n29A-n48A | - | n29 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
| CA_n29A-n66A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n29 | 5, 10 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n29A-n66B | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n29A-n66(2A) | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n29 | 5, 10 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n29A-n66(3A) | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n29A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n70 | 5, 10, 15, 201,, 251 |  |
| CA_n29A-n71A | - | n29 | 5, 10 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n77A | n778, 9 | n29 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n77(2A) | n778, 9 | n29 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |

Table 5.5A.3.1-1i: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n30A-n66A | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n30A-n66(2A) | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n30A-n66(3A) | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n30A-n77A | n778, 9 CA_n30A-n77A8 | n30 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n77(2A) | n778, 9 CA_n77(2A) CA_n30A-n77A8 | n30 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n34A-n39A | n348,9 n398 CA_n34A-n39A8 | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n40A | n348,9 n408,9 CA_n34A-n40A8 | n34 | 5, 10, 15 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n41A | n348,9 n418,9 CA_n34A-n41A8 | n34 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30 ,40 ,50, 60, 70, 80, 90, 100 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n41C | CA_n41C CA_n34A-n41A CA_n34A-n41C | n34 | 5, 10, 15 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n34A-n79A | n348,9 n798,9 CA_n34A-n79A8 | n34 | 5, 10, 15 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n79C | CA_n34A-n79A | n34 | 5, 10, 15 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n38A-n40A | - | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66A | CA_n38A-n66A | n38 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n38A-n66(2A) | CA_n38A-n66A | n38 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n38A-n71A | - | n38 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n38A-n78A | CA_n38A-n78A | n38 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n38 | See n38 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n38A-n78(2A) | CA_n38A-n78A | n38 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n38 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n38 | See n38 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n38A-n79A | - | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n38 | See n38 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n38A-n79C | - | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n38 | See n38 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n39A-n40A | n398 n408,9 CA_n39A-n40A8 | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n41A | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n41C | CA_n41C CA_n39A-n41A CA_n39A-n41C | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n39A-n41(2A) | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS0 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 |  |
| CA_n39A-n79A | n398 n798,9 CA_n39A-n79A8 | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n79C | CA_n79C CA_n39A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n40A-n41A | n418 CA_n40A-n41A8 | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n41C | CA_n41C12 CA_n40A-n41A CA_n40A-n41C12 | n40 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n40A-n77A | CA_n40A-n77A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n40A-n77(2A) | CA_n40A-n77A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n40A-n77C | - | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n40B-n77A | n778 CA_n40A-n77A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n40B-n77(2A) | CA_n40A-n77A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n40B-n77C | CA_n40A-n77A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n40A-n78A | CA_n40A-n78A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n78(2A) | CA_n40A-n78A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n40A-n78C | CA_n40A-n78A | n40 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n40B-n78A | - | n40 | CA_n40B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n40A-n78A | n40 | CA_n40B_BCS1 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n40B-n78(2A) | CA_n40A-n78A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n40B-n78C | CA_n40A-n78A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n40A-n79A | n408,9 n798,9 CA_n40A-n79A8 | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n79C | n408,9 n798,9 CA_n79C8 CA_n40A-n79A8 | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  | CA_n79C CA_n40A-n79A CA_n40A-n79C | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n40A-n105A | CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

Table 5.5A.3.1-1j: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n48A | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n48B | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41A-n48C | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41A-n48(2A) | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
| CA_n41C-n48A | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n41C-n48B | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41C-n48C | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41(2A)-n48A | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n48B | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41(2A)-n48C | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41(2A)-n48(2A) | CA_n41A-n48A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
| CA_n41A-n50A | CA_n41A-n50A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n50 | 5, 10, 15, 20, 40, 50, 60, 801 |  |
| CA_n41A-n66A13,14 | n418,9 CA_n41A-n66A8, 13,14 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A | n418, 9 CA_n41A-n66A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A) | n418, 9 CA_n41A-n66A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41C-n66A | n418, 9 CA_n41A-n66A8 CA_n41C | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | n418, 9 CA_n41A-n66A8 CA_n41C CA_n41C-n66A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A) | n418, 9 CA_n41A-n66A8 CA_n41C-n66A CA_n41C | n41 | CA_n41C_BCS2 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A) | n418, 9 CA_n41A-n66A8 | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(3A)-n66A | n418, 9 CA_n41A-n66A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40, |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66(2A) | n418,9 CA_n41A-n66A8 | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(A-C)-n66A | n418, 9 CA_n41C8 CA_n41A-n66A8 | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n66(2A) | n418,9 CA_n41C8 CA_n41A-n66A8 | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41A-n70A | CA_n41A-n70A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n71A | n418,9 CA_n41A-n71A8, 13, 14 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B | n418,9 CA_n41A-n71A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41A-n71(2A) | n418,9 CA_n41A-n71A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71A | n418, 9 CA_n41A-n71A8 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | n418, 9 CA_n41A-n71A8 CA_n41C8 CA_n41C-n71A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71(2A) | n418,9 CA_n41A-n71A8 CA_n41C-n71A CA_n41C8 | n41 | CA_n41C_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n71A | n418, 9 CA_n41A-n71A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A)_BCS3 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71(2A) | n418,9 CA_n41A-n71A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n71B | n418,9 CA_n41A-n71A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41(3A)-n71A | n418,9 CA_n41A-n71A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n71B | n418,9 CA_n41A-n71A8 | n41 | CA_n41(3A)_BCS4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS4 and 5 |  |
| CA_n41(3A)-n71(2A) | n418,9 CA_n41A-n71A8 | n41 | CA_n41(3A)_BCS4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS4 and 5 |  |
| CA_n41(A-C)-n71A | n418,9 CA_n41A-n71A8 CA_n41C8 | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n71B | n418,9 CA_n41A-n71A8 CA_n41C8 | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41(A-C)-n71(2A) | n418,9 CA_n41A-n71A8 CA_n41C8 | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71B | n418,9 CA_n41A-n71A8 CA_n41C-n71A CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41A-n74A | CA_n41A-n74A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n41A-n77A | n418,9 n778,9 CA_n41A-n77A8, 13 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41B-n77A | CA_n41A-n77A | n41 | CA_n41B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41(2A)-n77A | n418,9 n778,9 CA_n41A-n77A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n77A | n418,9 n778,9 CA_n41A-n77A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n77A | n418,9 n778,9 CA_n41C8 CA_n41A-n77A8 | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n77A | n418,9 n778,9 CA_n41A-n77A8 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n418,9 n778,9 CA_n41A-n77A8 CA_n41C8 CA_n41C-n77A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n77(2A) | n418,9 n778,9 CA_n77(2A) CA_n41A-n77A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  | n418,9 n778,9 CA_n41A-n77A8 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41C-n77(2A) | n418,9 n778,9 CA_n41A-n77A8 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41A-n77(3A) | n418,9 n778,9 CA_n41A-n77A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(3A)_BCS 4 and 5 |  |
| CA_n41(2A)-n77(2A) | - | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  | n418,9 n778,9 CA_n41A-n77A8 | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41A-n77C | CA_n41A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n41A-n78A | CA_n41A-n78A | n41 | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n78(2A) | CA_n41A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n41A-n78C | CA_n41A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n41A-n79A | n418,9 n798,9 CA_n41A-n79A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n79C | n418,9 n798,9 CA_n41A-n79A8 CA_n79C8 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n41C-n79A | n418,9 n798,9 CA_n41A-n79A8 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n79C | CA_n41C CA_n79C CA_n41A-n79A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n41A-n85A | CA_n41A-n85A | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n85A | CA_n41A-n85A CA_n41C | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n85A | CA_n41A-n85A | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n85A | CA_n41A-n85A | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n85A | CA_n41A-n85A CA_n41C | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

##### Table 5.5A.3.1-1k ~ Table 5.5A.3.1-1n

Table 5.5A.3.1-1k: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n46A-n48A | CA_n46A-n48A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | 20 |  |
|  |  | n46 | 20, 40, 60, 80 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n46A-n48(2A) | CA_n46A-n48A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46A-n48(3A) | CA_n46A-n48A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46A-n48(4A) | CA_n46A-n48A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46A-n48B | CA_n46A-n48A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46A-n48C | CA_n46A-n48A CA_n46A-n48B | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46B-n48A | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 20 |  |
|  |  | n46 | CA_n46B_BCS0 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n46B-n48(2A) | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46B-n48(3A) | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46B-n48(4A) | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46B-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46B-n48C | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46C-n48A | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 20 |  |
|  |  | n46 | CA_n46C_BCS0 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n46C-n48(2A) | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46C-n48(3A) | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46C-n48(4A) | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46C-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46C-n48C | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46D-n48A | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 20 |  |
|  |  | n46 | CA_n46D_BCS0 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n46D-n48(2A) | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46D-n48(3A) | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46D-n48(4A) | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46D-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46D-n48C | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46M-n48A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | 20 |  |
| CA_n46M-n48(2A) | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46M-n48(3A) | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46M-n48(4A) | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46M-n48B | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46M-n48C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46N-n48A | CA_n46A-n48A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 |  |
| CA_n46N-n48(2A) | CA_n46A-n48A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46N-n48(3A) | CA_n46A-n48A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46N-n48(4A) | CA_n46A-n48A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46N-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46N-n48C | CA_n46A-n48A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46A-n66A | - | n46 | 20, 40, 60, 80 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n46A-n77A | CA_n46A-n77A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46C-n77A | CA_n46A-n77A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46D-n77A | CA_n46A-n77A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46A-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46C-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46D-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46(2A)-n77A | CA_n46A-n77A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46(2A)-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46A-n78A | CA_n46A-n78A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46C-n78A | CA_n46A-n78A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46D-n78A | CA_n46A-n78A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46A-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46C-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46D-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46(2A)-n78A | CA_n46A-n78A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46(2A)-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46A-n96A | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n96A | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n96A | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n96A | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46M-n96A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n96A | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n96B | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n96B | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n96B | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n96B | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46M-n96B | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n96B | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n96C | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n96C | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n96C | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n96C | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46M-n96C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n96C | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n96D | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n96D | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n96D | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n96D | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46M-n96D | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n96D | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n96E | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n96E | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n96E | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n96E | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46M-n96E | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n96E | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n102A | - | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46A-n102(2A) | - | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46A-n102B | CA_n102B | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46A-n102C | CA_n102C | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46A-n102D | - | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46A-n102E | - | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46(2A)-n102A | - | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46(2A)-n102(2A) | - | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46(2A)-n102B | CA_n102B | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46(2A)-n102C | CA_n102C | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46(2A)-n102D | - | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46(2A)-n102E | - | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46C-n102A | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46C-n102(2A) | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46C-n102B | CA_n102B | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46C-n102C | CA_n102C | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46C-n102D | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46C-n102E | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46D-n102A | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46D-n102(2A) | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46D-n102B | CA_n102B | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46D-n102C | CA_n102C | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46D-n102D | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46D-n102E | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

Table 5.5A.3.1-1l: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n53A | - | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n53 | 5, 10 |  |
| CA_n48(2A)-n53A | - | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n53 | 5, 10 |  |
| CA_n48A-n66A | CA_n48A-n66A | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 806, 906, 1006 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40, |  |
| CA_n48A-n66B | CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n48A-n66(2A) | CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n48B-n66A | CA_n48B CA_n48A-n66A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48B-n66(2A) | CA_n48B CA_n48A-n66A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n48 | CA_n48B_BCS2 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | CA_n66(2A)_BCS2 |  |
| CA_n48C-n66A | CA_n48B CA_n48A-n66A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48C_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(2A)-n66A | CA_n48A-n66A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(2A)-n66B | CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n48(2A)-n66(2A) | CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n48(A-B)-n66A | CA_n48A-n66A | n48 | CA_n48(A-B)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(A-C)-n66A | CA_n48A-n66A | n48 | CA_n48(A-C)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48(A-C)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48A-n70A | CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n70A | CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| CA_n48B-n70A | CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n71A | CA_n48A-n71A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n71(2A) | CA_n48A-n71A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48(2A)-n71A | CA_n48A-n71A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n71(2A) | CA_n48A-n71A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48(3A)-n71A | CA_n48A-n71A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(4A)-n71A | CA_n48A-n71A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n71A | CA_n48A-n71A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n71(2A) | CA_n48A-n71A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48C-n71A | CA_n48A-n71A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n77A | n778,9 | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48A-n77C | n778,9 CA_n77C | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48A-n77(2A) | - | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n48(2A)-n77A | n778,9 | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48(2A)-n77C | n778,9 CA_n77C | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48(2A)-n77(2A) | - | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n48(3A)-n77A | - | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48B-n77A | CA_n48B n778,9 | n48 | CA_n48B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48B-n77C | CA_n48B n778,9 CA_n77C | n48 | CA_n48B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48B_BCS2 | 3 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48(A-B)-n77A | n778,9 | n48 | CA_n48(A-B)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(A-B)-n77C | n778,9 CA_n48B CA_n77C | n48 | CA_n48(A-B)_BCS1 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48A-n96A | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48(2A)-n96A | CA_n48A-n96A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48(2A)-n96B | CA_n48A-n96B | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48(2A)-n96C | CA_n48A-n96A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48(2A)-n96D | CA_n48A-n96A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48(2A)-n96E | CA_n48A-n96A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48(3A)-n96A | CA_n48A-n96A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48(3A)-n96B | CA_n48A-n96B | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48(3A)-n96C | CA_n48A-n96A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48(3A)-n96D | CA_n48A-n96A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48(3A)-n96E | CA_n48A-n96A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48(4A)-n96A | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48(4A)-n96B | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48(4A)-n96C | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48(4A)-n96D | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48(4A)-n96E | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48A-n96B | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48A-n96C | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48A-n96D | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48A-n96E | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48B-n96A | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48B-n96B | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48B-n96C | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48B-n96D | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48B-n96E | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48C-n96A | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48C-n96B | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48C-n96C | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48C-n96D | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48C-n96E | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n50A-n78A | CA_n50A-n78A | n50 | 5, 10, 15, 20, 30, 40, 50, 60, 801 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

Table 5.5A.3.1-1m: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n70A | - | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66B-n70A | - | n66 | CA_n66B_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66(2A)-n70A | - | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66(3A)-n70A | - | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66A-n71A | n668 n718 CA_n66A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71B | CA_n66A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n66A-n71(2A) | CA_n66A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n66(2A)-n71A | CA_n66A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n71B | CA_n66A-n71A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n66(2A)-n71(2A) | CA_n66A-n71A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n66(3A)-n71A | CA_n66A-n71A | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66B-n71A | CA_n66A-n71A | n66 | CA_n66B_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n77A | n778,9 CA_n66A-n77A8,13,14 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n77B | CA_n66A-n77A | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n66(2A)-n77A | n778,9 CA_n66A-n77A8 | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n77B | CA_n66A-n77A | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n66A-n77(2A) | n778,9 CA_n66A-n77A8 CA_n77(2A)8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n66(3A)-n77A | n778,9 CA_n66A-n77A8 | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n77(2A) | n778,9 CA_n66A-n77A8 CA_n77(2A) | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n66(3A)-n77(2A) | n778,9 CA_n66A-n77A8 | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n66A-n77C | n778,9 CA_n77C CA_n66A-n77A8 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n66A-n77(3A) | n778,9 CA_n77(2A)8 CA_n66A-n77A8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n66(2A)-n77(3A) | CA_n77(2A) CA_n66A-n77A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n66(2A)-n77C | n778,9 CA_n77C CA_n66A-n77A8 | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n66(3A)-n77C | n778,9 CA_n77C CA_n66A-n77A8 |  | CA_n66(3A)_BCS0 | 0 |
|  |  |  | CA_n77C_BCS1 |  |
| CA_n66B-n77A | n778,9 CA_n66A-n77A8 | n66 | CA_n66B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66B-n77C | n778,9 CA_n77C CA_n66A-n77A8 | n66 | CA_n66B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n66 | CA_n66B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n66A-n78A | n788,9 CA_n66A-n78A8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n78(2A) | n788,9 CA_n66A-n78A8 | n66 | 5, 10, 15, 20, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  | CA_n78(2A)8 | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n66(2A)-n78A | n788,9 CA_n66A-n78A8 | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n78(2A) | n788,9 CA_n66A-n78A8 | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n66 | CA_n66(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n66A-n85A | n668 CA_n66A-n85A | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n85A | CA_n66A-n85A | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n67A-n78A | - | n67 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n67A-n78(2A) | CA_n78(2A) | n67 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |

Table 5.5A.3.1-1n: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n70A-n71A | CA_n70A-n71A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n70A-n71(2A) | CA_n70A-n71A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n70A-n77A | CA_n70A-n77A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n70A-n78A | CA_n70A-n78A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n71A-n77A | n778, 9 CA_n71A-n77A8,13 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71A-n77(2A) | n778, 9 CA_n77(2A)8 CA_n71A-n77A8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71A-n77(3A) | 778, 9 CA_n77(2A)8 CA_n71A-n77A8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n71A-n77B | - | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n71A-n77C | - | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n71B-n77A | n778, 9 CA_n71A-n77A8 | n71 | CA_n71B_BCS2 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71B-n77(2A) | n778, 9 CA_n71A-n77A8 | n71 | CA_n71B_BCS2 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71(2A)-n77A | n778, 9 CA_n71A-n77A8 | n71 | CA_n71(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71(2A)-n77(2A) | n778, 9 CA_n71A-n77A8 | n71 | CA_n71(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71(2A)-n77B | - | n71 | CA_n71(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n71(2A)-n77C | - | n71 | CA_n71(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n71A-n78A | n788,9 CA_n71A-n78A8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71A-n78(2A) | n788,9 CA_n71A-n78A8 | n71 | 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n71A-n85A | - | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71(2A)-n85A | - | n71 | CA_n71(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71B-n85A | - | n71 | CA_n71B BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n74A-n77A | CA_n74A-n77A | n74 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n74A-n78A | CA_n74A-n78A | n74 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n75A-n78A | - | n75 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n75A-n78(2A) | - | n75 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n76A-n78A | - | n76 | 5 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n77A-n78A2 | - | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n78C2 | - | n77 | 10,15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n77A-n78(2A)2 | - | n77 | 10,15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n77A-n79A | n778,9 n798,9 CA_n77A-n79A8 | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n77(2A)-n79A | n778,9 n798,9 CA_n77(2A)12 CA_n77A-n79A8 | n77 | CA_n77(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n77(3A)-n79A | n778,9 n798,9 CA_n77(2A)12 CA_n77A-n79A8 | n77 | CA_n77(3A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n77A-n85A | n778,9 CA_n77A-n85A8 | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77(2A)-n85A | CA_n77A-n85A | n77 | CA_n77(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n102A | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n77A-n102(2A) | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n77A-n102B | CA_n77A-n102A CA_n77A-n102B | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n77A-n102C | CA_n77A-n102A CA_n77A-n102C | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n77A-n102D | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n77A-n102E | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n77(2A)-n102A | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n77(2A)-n102(2A) | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n77(2A)-n102B | CA_n77(2A) CA_n77A-n102A CA_n77A-n102B | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n77(2A)-n102C | CA_n77(2A) CA_n77A-n102A CA_n77A-n102C | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n77(2A)-n102D | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n77(2A)-n102E | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78A-n79A | n78A8,9 n79A8,9 CA_n78A-n79A8 | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78A-n79C | - | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n78(2A)-n79A | CA_n78A-n79A | n78 | CA_n78(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78A-n92A | CA_n78A-n92A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n92 | 5, 10, 15, 20 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n92 | See n92 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78(2A)-n92A | CA_n78A-n92A | n78 | CA_n78(2A)_BCS0 | 0 |
|  |  | n92 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n92 | See n92 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78A-n94A | - | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n94 | 5, 10, 15, 20 |  |
| CA_n78A-n102A | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78A-n102(2A) | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n78A-n102B | CA_n78A-n102A CA_n78A-n102B | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n78A-n102C | CA_n78A-n102A CA_n78A-n102C | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n78A-n102D | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78A-n102E | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102A | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78(2A)-n102B | CA_n78A-n102A CA_n78(2A) CA_n78A-n102B | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n78(2A)-n102C | CA_n78A-n102A CA_n78(2A) CA_n78A-n102C | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n78(2A)-n102D | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78(2A)-n102E | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102(2A) | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n78A-n105A | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

The following notes are applied to the above tables:

NOTE 1:	This UE channel bandwidth is applicable only to downlink.

NOTE 2:	The minimum requirements for intra-band contiguous or non-contiguous CA apply.

NOTE 3:	The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1.

NOTE 4:	This UE channel bandwidth is optional in this release of the specification.

NOTE 5:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration.

NOTE 6:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration

NOTE 7:	Limited to operation at 3450-3550 MHz and 3700–3980 MHz.

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

NOTE 11: The CA configurations are given in Table 5.5A.1-1 or Table 5.5A.2-1 in this specification

NOTE 12: UL configurations are for non simultaneous Rx/Tx operation.

NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

NOTE 15: Uplink is only in n5 for CA_n5-n8.

NOTE 16: For UEs only supporting DL CA_n26-n28, uplink support in band n26 is optional, if the UE supports CA_n26-n28 UL configuration, it should also support UL in band n26 and n28.

#### 5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: Void

##### Table 5.5A.3.2-1a

Table 5.5A.3.2-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A | CA_n1A-n3A CA_n1A-n5A CA_n3A-n5A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3(2A)-n7A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1(2A)-n3A-n7A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3B-n7A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1(2A)-n3B-n7A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1(2A)-n3(2A)-n7A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3B-n7B | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3A-n8A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n18A | CA_n1A-n3A CA_n1A-n18A CA_n3A-n18A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n18 | 5, 10, 15 |  |
| CA_n1A-n3A-n20A | CA_n1A-n3A CA_n1A-n20A CA_n3A-n20A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n26A | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n26(2A) | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3B-n26A | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n1A-n3B-n26(2A) | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 201, 301 |  |
| CA_n1A-n3B-n28A | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3B-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3A-n38A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3B-n38A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3(2A)-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3(2A)-n38A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n40A | CA_n1A-n3A CA_n1A-n40A CA_n3A-n40A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 30, 35, 40, 45, 50 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n41A | n417 CA_n1A-n3A CA_n1A-n41A7 CA_n3A-n41A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n67A | CA_n1A-n3A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n75A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n77A | n777,9 CA_n1A-n3A CA_n1A-n77A7 CA_n3A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n77(2A) | n777,9 CA_n1A-n3A CA_n1A-n77A7 CA_n3A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n77(3A) | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A7 CA_n3A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n78C | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n3B-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n78(2A) | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n79A | CA_n1A-n3A CA_n1A-n79A CA_n3A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3B-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3B-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3B-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3B-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3(2A)-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3(2A)-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3(2A)-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3(2A)-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3A-n105A | CA_n1A-n3A CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n5A-n7A | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n5A-n7B | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n5A-n28A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  | CA_n1A-n5A CA_n1A-n28A CA_n5A-n28A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n40A | CA_n1A-n5A CA_n1A-n40A CA_n5A-n40A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n78A | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n78C | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n1A-n5A-n79A | CA_n1A-n5A CA_n1A-n79A CA_n5A-n79A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n8A | CA_n1A-n7A CA_n1A-n8A CA_n7A-n8A | n1 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n26A | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n26(2A) | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n7B-n26A | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n7B-n26(2A) | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n7A-n28A | CA_n1A-n7A CA_n1A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n7B-n28A | CA_n1A-n28A CA_n1A-n7A CA_n7A-n28A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n38A10 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n7A-n38A10 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n7A-n40A | CA_n1A-n7A CA_n1A-n40A CA_n7A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n7A-n67A | CA_n1A-n7A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n75A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 901, 100 |  |
|  | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7B-n78A | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n1A-n7B-n78(2A) | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7A-n78C | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n7A-n105A | CA_n1A-n7A CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n8A-n28A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n28 | 10, 15, 20 |  |
| CA_n1A-n8A-n40A | CA_n1A-n8A CA_n1A-n40A CA_n8A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n8A-n77A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n8A-n77(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n8A-n78A | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | - | n1 | 5, 10, 15, 20 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n8A-n78C | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n1A-n8A-n78(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
| CA_n1A-n8A-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n18A-n28A | CA_n1A-n18A CA_n1A-n28A CA_n18A-n28A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n1A-n18A-n41A | CA_n1A-n18A CA_n1A-n41A CA_n18A-n41A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n77A | n777 CA_n1A-n18A CA_n1A-n77A CA_n18A-n77A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n77(2A) | CA_n1A-n18A CA_n1A-n77A CA_n18A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n20A-n67A | CA_n1A-n20A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n20A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26A-n78A | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n28A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n28A-n40A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n28A-n40B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | CA_n40B_BCS0 |  |
| CA_n1A-n28A-n41A | n417 CA_n1A-n28A CA_n1A-n41A7 CA_n28A-n41A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n28A-n46A | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
| CA_n1A-n28A-n46C | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n1A-n28A-n46D | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n1A-n28A-n46(2A) | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
| CA_n1A-n28A-n75A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n28A-n77A | n777,9 CA_n1A-n28A CA_n1A-n77A7 CA_n28A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n77(2A) | n777,9 CA_n1A-n28A CA_n1A-n77A7 CA_n28A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n28A-n77(3A) | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n28A-n78C | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n28A-n79A | CA_n1A-n28A CA_n1A-n79A CA_n28A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n28A-n102A | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n28A-n102B | CA_n1A-n28A CA_n1A-n102A CA_n1A-n102B CA_n28A-n102A CA_n28A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n28A-n102C | CA_n1A-n28A CA_n1A-n102A CA_n1A-n102C CA_n28A-n102A CA_n28A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n28A-n102D | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n28A-n102E | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n1A-n28A-n102(2A) | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n40A-n77A | CA_n1A-n40A CA_n1A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n40A-n77(2A) | CA_n1A-n40A CA_n1A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n40A-n78A | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n40B-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n40A-n105A | CA_n1A-n40A CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n41A-n77A | n417 n777,9 CA_n1A-n41A7 CA_n1A-n77A7 CA_n41A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n41A-n77(2A) | n417 n777,9 CA_n1A-n41A7 CA_n1A-n77A7 CA_n41A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n41A-n77(3A) | CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n1A-n41A-n79A | CA_n1A-n41A CA_n1A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n46A-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n46C-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n46D-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n46(2A)-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n46A-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78(2A) | n46 | 10, 20, 40, 60, 80 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n46C-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78(2A) | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n46D-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78(2A) | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n46(2A)-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78(2A) | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n67A-n78A | CA_n1A-n78A | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n67A-n78(2A) | CA_n1A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n75A-n78A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n77A-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n77(2A)-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n77(3A)-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n78A-n79A5 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n78(2A)-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n78A-n102A | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n78A-n102B | CA_n1A-n78A CA_n1A-n102A CA_n1A-n102B CA_n78A-n102A CA_n78A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n78A-n102C | CA_n1A-n78A CA_n1A-n102A CA_n1A-n102C CA_n78A-n102A CA_n78A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n78A-n102D | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n78A-n102E | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n1A-n78A-n102(2A) | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n78(2A)-n102A | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n78(2A)-n102B | CA_n1A-n78A CA_n1A-n102A CA_n1A-n102B CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n78(2A)-n102C | CA_n1A-n78A CA_n1A-n102A CA_n1A-n102C CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n78(2A)-n102D | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n78(2A)-n102E | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n1A-n78(2A)-n102(2A) | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n78A-n105A | CA_n1A-n78A CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n2A-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n5A-n41A | CA_n2A_n5A CA_n2A_n41A CA_n5A_n41A | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n2A-n5A-n48A | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
| CA_n2A-n5A-n48B | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A CA_n48B | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n5A-n48(2A) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n5A-n48(A-B) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n2(2A)-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n5A-n66(3A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n5A-n77A | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n77C | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n5A-n77(2A) | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n5A-n77A | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n77(2A) | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n7A-n12A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n12 | 5, 10, 15 |  |
| CA_n2A-n7A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2A-n7A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n7A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n12A-n41A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n2A-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n12A-n66(3A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n12A-n71A | CA_n2A-n12A CA_n2A-n71A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n12A-n77A | n777,9 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n77A | n777,9 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n77(2A) | n777,9 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n77(2A) | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n66(3A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n14A-n77A | n777,9 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n77(2A) | n777,9 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n77A | n777,9 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n14A-n77(2A) | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n30A | CA_n2A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n29A-n30A | CA_n2A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n29A-n66A | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n29A-n66A | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n29A-n66(2A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n66(2A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n29A-n77A | n777,9 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n77A | n777,9 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n77(2A) | 777,9 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n77(2A) | n777 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2(2A)-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2(2A)-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n30A-n66(3A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n30A-n77A | n777,9 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n77(2A) | n777,9 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n30A-n77A | n777,9 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n30A-n77(2A) | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n41A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2A-n41A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n48A-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(A-B)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48B-n66A | CA_n48B CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(2A)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48A-n77A | n777,9 CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n77C | n777,9 CA_n2A-n48A CA_n2A-n77A7 CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48(2A)-n77C | n777,9 CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48B-n77C | n777,9 CA_n48B CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48B-n77A | n777,9 CA_n48B CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48(2A)-n77A | n777,9 CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n66A-n77A | n777, 9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n66A-n77A | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66(2A)-n77A | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77C | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n66A-n77(2A) | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n66(2A)-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n66(2A)-n77(2A) | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n66A-n77(2A) | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66(2A)-n77(2A) | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66(3A)-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66(3A)-n77(2A) | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66A-n78A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n78(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n2A-n71A-n77A | CA_n2A-n71A CA_n2A-n77A CA_n71A-n77A | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n71A-n77(2A) | CA_n2A-n71A CA_n2A-n77A CA_n71A-n77A | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n71A-n78A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n71A-n78(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n5A-n7A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3A-n5A-n7B | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n3A-n5A-n28A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  | CA_n3A-n5A CA_n3A-n28A CA_n5A-n28A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n5A-n78A | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A-n79A | CA_n3A-n5A CA_n3A-n79A CA_n5A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n8A | CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
| CA_n3A-n7A-n20A | CA_n3A-n7A CA_n3A-n20A CA_n7A-n20A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n26A | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n3A-n7A-n26(2A) | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3A-n7B-n26A | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n3A-n7B-n26(2A) | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3B-n7A-n26A | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n3B-n7A-n26(2A) | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3B-n7B-n26A | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n3B-n7B-n26(2A) | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3A-n7A-n28A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n7B-n28A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3B-n7A-n28A | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3B-n7B-n28A | CA_n7B CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n7A-n38A10 | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3B-n7A-n38A10 | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3(2A)-n7A-n38A10 | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A-n7A-n67A | CA_n3A-n7A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n75A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n78C | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3A-n7B-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n3A-n7B-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7A-n78(2A) | CA_n78(2A) CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7B-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7A-n79A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n79C | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3B-n7A-n79A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3(2A)-n7A-n79A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n79C | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3(2A)-n7A-n79C | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3A-n7A-n105A | CA_n3A-n7A CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n8A-n28A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n3A-n8A-n41A | CA_n3A-n8A CA_n3A-n41A CA_n8A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n77A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n77(2A) | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n8A-n78A | CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n79A | CA_n3A-n8A CA_n3A-n79A CA_n8A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n18A-n28A | CA_n3A-n18A CA_n3A-n28A CA_n18A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n3A-n18A-n41A | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n77A | n777 CA_n3A-n18A CA_n3A-n77A7 CA_n18A-n77A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n77(2A) | n777 CA_n3A-n18A CA_n3A-n77A7 CA_n18A-n77A7 | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n20A-n67A | CA_n3A-n20A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n20A-n28A | CA_n3A-n20A CA_n3A-n28A CA_n20A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n3A-n20A-n78A | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n20A-n78(2A) | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A CA_n78(2A) | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n3A-n26A-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n26A-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n28A-n38A | - | n3 | 5, 10, 15, 20, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n38 | 5, 10, 15, 20, 30, 40 |  |
| CA_n3A-n28A-n40A | CA_n3A-n28A CA_n3A-n40A CA_n28A-n40A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n40 | 20, 40 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A | n417,9 CA_n3A-n28A CA_n3A-n41A7 CA_n28A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n41B | CA_n3A-n28A CA_n3A-n41A CA_n28A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | CA_n41B_BCS0 |  |
| CA_n3A-n28A-n77A | n777,9 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 2 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n77(2A) | n777,9 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  | CA_n77(2A)7 | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n28A-n77(3A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n3A-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n78C | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3A-n28A-n78(2A) | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n78(2A) CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n28A-n78(2A) | CA_n78(2A) CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n28A-n79A | n797,9 CA_n3A-n28A CA_n3A-n79A7 CA_n28A-n79A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n38A-n40A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n38A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n40A-n78A | CA_n3A-n40A CA_n3A-n78A CA_n40A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n40A-n105A | CA_n3A-n40A CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n77A-n79A4 | n777,9 n797,9 CA_n3A-n77A7 CA_n3A-n79A7 CA_n77A-n79A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n77(2A)-n79A4 | n777,9 n797,9 CA_n3A-n77A7 CA_n3A-n79A7 CA_n77A-n79A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n77(3A)-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n40A-n41A | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n40A-n41C | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n3A-n40A-n77A | CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 30, 35, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n40A-n77(2A) | CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 30, 35, 40, 45, 50 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n41A-n77A | n417 n777 CA_n3A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A7 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41B-n77A | CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41A-n77(2A) | n417 n777 CA_n3A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n41A-n77(3A) | CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n3A-n41A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  | CA_n3A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n78A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n41A-n78(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n78A | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n41A-n79A | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41A-n79C | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3A-n41C-n79A | CA_n41C CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41C-n79C | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3A-n67A-n78A | CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n67A-n78(2A) | CA_n78(2A) CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n75A-n78A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n78A-n79A | n787,9 | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n78A-n79C | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3B-n78A-n79A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3B-n78A-n79C | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3(2A)-n78A-n79A | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3(2A)-n78A-n79C | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n3A-n78A-n105A | CA_n3A-n78A CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n5A-n7A-n25A | CA_n5A-n7A CA_n5A-n25A CA_n7A-n25A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
| CA_n5A-n7A-n25(2A) | CA_n5A-n7A CA_n5A-n25A CA_n7A-n25A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n25 | CA_n25(2A) |  |
| CA_n5A-n7A-n28A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n5A-n7A-n66A | CA_n5A-n7A CA_n5A-n66A CA_n7A-n66A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
| CA_n5A-n7A-n77A | n777,9 CA_n5A-n7A CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n7A-n77(2A) | n777,9 CA_n77(2A)7 CA_n5A-n7A CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n5A-n7A-n77(3A) | n777,9 CA_n77(2A)7 CA_n5A-n7A CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n5A-n7A-n78A | CA_n5A-n78A7 CA_n7A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n5 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n5A-n7B-n78A | CA_n5A-n78A7 CA_n7A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n5 | 5, 10, 15, 20 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n5A-n12A-n77A | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n12A-n77(2A) | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n14A-n77A | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n14A-n77(2A) | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n29A | CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n29 | 5, 10 |  |
| CA_n5A-n25A-n41A | CA_n5A-n25A CA_n5A-n41A CA_n25A-n41A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
| CA_n5A-n25(2A)-n41A | CA_n5A-n25A CA_n5A-n41A CA_n25A-n41A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n25 | CA_n25(2A) |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
| CA_n5A-n25A-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25(2A)-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25A-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n25(2A)-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n25A-n77A | n777,9 CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  | CA_n5A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n25A-n77A7 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n77A | n777,9 CA_n5A-n25A CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n77(2A) | n777,9 CA_n5A-n25A CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n77(3A) | n777,9 CA_n77(2A) CA_n5A-n25A CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n5A-n25(2A)-n77(2A) | n777,9 CA_n5A-n25A CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n78A | n787,9 CA_n5A-n25A CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n78A | n787,9 CA_n5A-n25A CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n78(2A) | n787,9 CA_n5A-n25A CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25(2A)-n78(2A) | n787,9 CA_n5A-n25A CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n28A-n78A | CA_n5A-n28A CA_n5A-n78A CA_n28A-n78A | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n28A-n79A | CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n28A-n105A | CA_n5A-n28A CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n5A-n29A-n66A | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n29A-n77A | n777 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n29A-n77(2A) | n777 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n30A-n66A | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n5A-n30A-n66(2A) | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n5A-n30A-n66(3A) | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n5A-n30A-n77A | n777,9 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n30A-n77(2A) | n777,9 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n40A-n78A | CA_n5A-n40A CA_n5A-n78A CA_n40A-n78A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n40 | 58, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 |  |
| CA_n5A-n41A-n66A | CA_n5A-n41A CA_n5A-n66A CA_n41A-n66A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
| CA_n5A-n41A-n77A | CA_n5A-n41A CA_n5A-n77A CA_n41A-n77A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n41A-n77(2A) | CA_n5A-n41A CA_n5A-n77A CA_n41A-n77A | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n77 | CA_n77(2A) |  |
| CA_n5A-n48A-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(A-B)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48B-n66A | CA_n48B CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(2A)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48A-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48B-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48B-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | CA_n77C BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | CA_n77C BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | CA_n77C BCS1 |  |
| CA_n5A-n48(2A)-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48(2A)-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66A-n77A | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n77A | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n77(2A) | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n66(3A)-n77A | n777,9 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(3A)-n77(2A) | n777 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n66A-n77C | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66A-n77(2A) | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 CA_n77(2A)7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n5A-n66A-n77(3A) | CA_n77(2A) CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n5A-n66A-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n66(2A)-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n78A-n79A | CA_n5A-n78A CA_n5A-n79A CA_n78A-n79A | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n8A-n28A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n7A-n8A-n40A | CA_n7A-n8A CA_n7A-n40A CA_n8A-n40A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n7A-n8A-n78A | CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n12A-n25A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n12A-n66A | - | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
| CA_n7A-n12A-n71A | CA_n7A-n12A CA_n7A-n71A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n12A-n77A | - | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n20A-n67A | CA_n7A-n20A | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | See n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n20A-n78A | CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n20A-n78(2A) | CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A CA_n78(2A) | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n7A-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A)-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A)-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7A-n25A-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n25(2A)-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n25A-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7A-n25A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n25A-n77A | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n77A | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n77(2A) | n777,9 CA_n77(2A)7 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n77(3A) | n777,9 CA_n77(2A)7 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n25(2A)-n77(2A) | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n77A | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25(2A)-n77A | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n77(2A) | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n77(2A) | n777,9 CA_n7A-n25A CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n78A | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7(2A)-n25A-n78A | - | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7A-n25(2A)-n78A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7(2A)-n25(2A)-n78A | - | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7A-n25A-n78(2A) | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n25A-n78(2A) | - | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n25(2A)-n78(2A) | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7(2A)-n25(2A)-n78(2A) | - | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n26A-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n26A-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n26(2A)-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n26(2A)-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7B-n26A-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7B-n26A-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n7B CA_n26A-n78A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7B-n26(2A)-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7B-n26(2A)-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n28A-n38A11 | - | n7 | 5, 10, 15, 20, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n38 | 5, 10, 15, 20, 30, 40 |  |
| CA_n7A-n28A-n78(2A) | CA_n78(2A) CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n28A-n78A | CA_n7A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n28A-n78A7 | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7A-n28A-n78C | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n7B-n28A-n78A | CA_n7A-n78A7 CA_n28A-n78A7 | n7 | CA_n7B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n38A-n78A10 | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n40A-n78A | CA_n7A-n40A CA_n7A-n78A CA_n40A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5, 10, 15, 20, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n40A-n105A | CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5,10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n7A-n46A-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46C-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46D-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46(2A)-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46(2A)-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n46A-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n46C-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n46D-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n66A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n66A-n77A | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n77A | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n77(2A) | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66A-n77(3A) | n777,9 CA_n77(2A)7 CA_n77(2A) 7 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n66(2A)-n77(2A) | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66A-n77A | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n77A | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66A-n77(2A) | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66(2A)-n77(2A) | n777,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66A-n78A | n787,9 CA_n7A-n66A CA_n7A-n78A7 CA_n66A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n78(2A) | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66(2A)-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n67A-n78A | CA_n7A-n78A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n67A-n78(2A) | CA_n7A-n78A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n71A-n77A | n777,9 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n71A-n77(2A) | n777,9 CA_n77(2A)7 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n7A-n71A-n77(3A) | n777,9 CA_n77(2A)7 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n7A-n75A-n78A | - | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n78A-n102A | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n7A-n78A-n102B | CA_n7A-n78A CA_n7A-n102A CA_n7A-n102B CA_n78A-n102A CA_n78A-n102B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n7A-n78A-n102C | CA_n7A-n78A CA_n7A-n102A CA_n7A-n102C CA_n78A-n102A CA_n78A-n102C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n7A-n78A-n102D | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n7A-n78A-n102E | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n7A-n78A-n102(2A) | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n7A-n78(2A)-n102A | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n7A-n78(2A)-n102B | CA_n7A-n78A CA_n7A-n102A CA_n7A-n102B CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n7A-n78(2A)-n102C | CA_n7A-n78A CA_n7A-n102A CA_n7A-n102C CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n7A-n78(2A)-n102D | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n7A-n78(2A)-n102E | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n7A-n78(2A)-n102(2A) | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n7A-n78A-n105A | CA_n7A-n78A CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

##### Table 5.5A.3.2-1b

Table 5.5A.3.2-1b: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n20A-n28A | CA_n8A-n20A | n8 | 5, 10, 15, 20 | 0 |
|  | CA_n8A-n28A | n20 | 5, 10, 15, 20 |  |
|  | CA_n20A-n28A | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n8A-n20A-n75A | - | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n28A-n75A | - | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n28A-n78A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n38A-n40A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n39A-n41A | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  | - | n8 | 5, 10, 15, 20 | 1 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n39A-n41C | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n8A-n39A-n79A | CA_n8A-n39A CA_n8A-n79A CA_n39A-n79A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n40A-n41C | CA_n41C CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n8A-n40A-n78A | CA_n8A-n40A CA_n8A-n78A CA_n40A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n41A-n79A | CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n41C-n79A | CA_n41C CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n78A-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n8A-n78(2A)-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n12A-n25A-n41A | - | n12 | 5, 10, 15 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n12A-n25A-n66A | - | n12 | 5, 10, 15 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n12A-n30A-n66A | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n12A-n30A-n66(2A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n12A-n30A-n66(3A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n12A-n30A-n77A | n777,9 CA_n12A-n30A, CA_n12A-n77A7 CA_n30A-n77A7 | n12 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n30A-n77(2A) | n777,9 CA_n12A-n30A CA_n12A-n77A7 CA_n30A-n77A7 | n12 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n41A-n66A | - | n12 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n12A-n41A-n77A | - | n12 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66A-n77A | n777,9 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66(2A)-n77A | n777,9 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66A-n77(2A) | n777,9 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n66(2A)-n77(2A) | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n66(3A)-n77A | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66(3A)-n77(2A) | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n71A-n77A | CA_n12A-n77A CA_n71A-n77A | n12 | 5, 10, 15 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n25A-n66A | CA_n13A-n25A CA_n13A-n66A CA_n25A-n66A | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n13A-n25A-n77A | n777,9 CA_n13A-n25A CA_n13A-n77A7 CA_n25A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n25A-n77(2A) | n777,9 CA_n77(2A)7 CA_n13A-n25A CA_n13A-n77A7 CA_n25A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n13A-n66A-n77A | n777, 9 CA_n13A-n66A CA_n13A-n77A7 CA_n66A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n66A-n77(2A) | n777,9 CA_n77(2A)7 CA_n13A-n66A CA_n13A-n77A7 CA_n66A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n30A-n66A | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n30A-n66(2A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n14A-n30A-n66(3A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n14A-n30A-n77A | n777,9 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n30A-n77(2A) | n777,9 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n66A-n77A | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66(2A)-n77A | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66A-n77(2A) | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n66(2A)-n77(2A) | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n66(3A)-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66(3A)-n77(2A) | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n28A-n41A | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n28A-n77A | n777 CA_n18A-n28A CA_n18A-n77A7 CA_n28A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n28A-n77(2A) | n777 CA_n18A-n28A CA_n18A-n77A CA_n28A-n77A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n41A-n77A | n417 n777 CA_n18A-n41A7 CA_n18A-n77A7 CA_n41A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n41A-n77(2A) | n777 CA_n18A-n41A7 CA_n18A-n77A7 CA_n41A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n20A-n28A-n75A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n75 | 5, 10, 15, 20 |  |
| CA_n20A-n28A-n78A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n20A-n28A-n78C | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n20A-n67A-n78A | CA_n20A-n78A | n20 | See n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n67 | See n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n20A-n67A-n78(2A) | CA_n20A-n78A CA_n78(2A) | n20 | See n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n67 | See n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n24A-n41A-n48A | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
| CA_n24A-n41(2A)-n48A | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
| CA_n24A-n41A-n48(2A) | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A) BCS0 |  |
| CA_n24A-n41(2A)-n48(2A) | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n48 | CA_n48(2A) BCS0 |  |
| CA_n24A-n41A-n77A | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41(2A)-n77A | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41A-n77(2A) | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n41(2A)-n77(2A) | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n48A-n77A | - | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n48(2A)-n77A | - | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A) BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n48A-n77(2A) | - | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n48(2A)-n77(2A) | - | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A) BCS0 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n25A-n29A-n66A | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n25A-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n25A-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n41A-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25A-n41C-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 CA_n25A-n41C CA_n41C-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25A-n41(A-C)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(A-C)-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(3A)-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41(A-C)-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A CA_n41C | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n25 | 5, 10, 15, 20, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41A-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(3A)-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n41C-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 CA_n25A-n41C CA_n41C-n71A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41C-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n41(A-C)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(A-C)-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41(A-C)-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A)-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25(2A)-n41(3A)-n71A | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25(2A)-n41(A-C)-n71A | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(3A)-n77A | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n77(2A) | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n77(2A) | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n77A | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n77(2A) | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(A-C)-n77A | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n77(2A) | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n77(2A) | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A)-n77(2A) | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41(3A)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(A-C)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n78A | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41A-n78(2A) | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n41A-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n48A-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n48(2A)-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n48C-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n66A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n66A-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n66(2A)-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A)-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n66(2A)-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n66A-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66(2A)-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66A-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25(2A)-n66A-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A)-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n77(2A) | n777,9 CA_n77(2A)7 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n66A-n77(3A) | n777,9 CA_n77(2A)7 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n25A-n66(2A)-n77(2A) | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n66A-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66(2A)-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66A-n77(2A) | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n66(2A)-n77(2A) | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n78A | n787,9 CA_n25A-n66A CA_n25A-n78A7 CA_n66A-n78A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n78A | n787,9 CA_n25A-n66A CA_n25A-n78A7 CA_n66A-n78A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78(2A) | n787 CA_n78(2A)7 CA_n25A-n66A CA_n25A-n78A7 CA_n66A-n78A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n66A-n85A | CA_n25A-n66A CA_n25A-n85A CA_n66A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71A-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71A-n77(2A) | n777,9 CA_n77(2A)7 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n71A-n77(3A) | n777,9 CA_n77(2A)7 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A) BCS1 |  |
| CA_n25A-n71B-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71B-n77(2A) | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n71(2A)-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71(2A)-n77(2A) | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n71A-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71A-n77(2A) | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n71B-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71B-n77(2A) | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n71(2A)-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71(2A)-n77(2A) | n77 CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n71A-n78A | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71A-n78(2A) | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n71A-n85A | CA_n25A-n71A CA_n25A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77A-n85A | CA_n25A-n77A CA_n25A-n85A CA_n77A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77(2A)-n85A | CA_n25A-n77A CA_n25A-n85A CA_n77A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n77A-n85A | CA_n25A-n77A CA_n25A-n85A CA_n77A-n85A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n26A-n29A-n66A | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n29A-n66(2A) | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n26A-n29A-n66(3A) | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n26A-n29A-n70A | CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| CA_n26A-n48A-n66A | CA_n26A-n48A CA_n26A-n66A CA_n48A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n48(2A)-n66A | CA_n26A-n48A CA_n26A-n66A CA_n48A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n48A-n66(2A) | CA_n26A-n48A CA_n26A-n66A CA_n48A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n26A-n48(2A)-n66(2A) | CA_n26A-n48A CA_n26A-n66A CA_n48A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n26A-n48A-n70A | CA_n26A-n48A CA_n26A-n70A CA_n48A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(3A)-n70A | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66A-n71A | CA_n26A-n66A CA_n66A-n71A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n66(2A)-n71A | CA_n26A-n66A CA_n66A-n71A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n66(3A)-n71A | CA_n66A-n71A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n66A-n77A | CA_n26A-n66A CA_n26A-n77A CA_n66A-n77A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n70A-n71A | CA_n26A-n70A CA_n70A-n71A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n70A-n77A | CA_n26A-n70A CA_n26A-n77A CA_n70A-n77A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40 |  |
| CA_n28A-n38A-n78A | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n39A-n40A | CA_n28A-n39A CA_n28A-n40A CA_n39A-n40A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
| CA_n28A-n39A-n41A | CA_n28A-n39A CA_n28A-n41A CA_n39A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n39A-n41C | CA_n28A-n39A CA_n28A-n41A CA_n39A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n28A-n39A-n79A | CA_n28A-n39A CA_n28A-n79A CA_n39A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n40A-n41A | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n40A-n41C | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | CA_n41C_BCS0 |  |
| CA_n28A-n40A-n78A | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n40A-n77A | CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n40A-n77(2A) | CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n28A-n40B-n78A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n40A-n79A | CA_n28A-n40A CA_n28A-n79A CA_n40A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n41A-n77A | n417,9 n777,9 CA_n28A-n41A7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  | CA_n28A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41B-n77A | CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n28 | 5, 10 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41A-n77(2A) | n417,9 n777,9 CA_n28A-n41A7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  | CA_n28A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n41A-n77(3A) | CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n28 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n41A-n78A CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n41A-n78(2A) | CA_n78(2A) | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n41A-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n41A-n79C | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n28A-n41C-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n41C-n79C | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n28A-n46A-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46C-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46D-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46(2A)-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46(2A)-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n46A-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n46C-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n46D-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n75A-n78A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n77A-n79A4 | n777,9 n797,9 CA_n28A-n77A7 CA_n28A-n79A7 CA_n77A-n79A7 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n77(2A)-n79A4 | n777,9 n797,9 CA_n28A-n77A7 CA_n28A-n79A7 CA_n77A-n79A7 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n77(3A)-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n78A-n79A | n787,9 n797,9 CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n78A-n102A | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n28A-n78A-n102B | CA_n28A-n78A CA_n28A-n102A CA_n28A-n102B CA_n78A-n102A CA_n78A-n102B | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n28A-n78A-n102C | CA_n28A-n78A CA_n28A-n102A CA_n28A-n102C CA_n78A-n102A CA_n78A-n102C | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n28A-n78A-n102D | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n28A-n78A-n102E | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n28A-n78A-n102(2A) | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n28A-n78(2A)-n102A | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n28A-n78(2A)-n102B | CA_n28A-n78A CA_n28A-n102A CA_n28A-n102B CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n28A-n78(2A)-n102C | CA_n28A-n78A CA_n28A-n102A CA_n28A-n102C CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n28A-n78(2A)-n102D | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n28A-n78(2A)-n102E | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n28A-n78(2A)-n102(2A) | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n29A-n30A-n66A | CA_n30A-n66A | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n29A-n30A-n66(2A) | CA_n30A-n66A | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n29A-n30A-n77A | n777,9 CA_n30A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n30A-n77(2A) | n777,9 CA_n30A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66B-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(3A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66A-n71A | CA_n66A-n71A | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n66(2A)-n71A | CA_n66A-n71A | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n66(3A)-n71A | CA_n66A-n71A | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n66A-n77A | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66(2A)-n77A | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66A-n77(2A) | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66(3A)-n77A | n777 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66(2A)-n77(2A) | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66(3A)-n77(2A) | n777 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n70A-n71A | CA_n70A-n71A | n29 | 5, 10 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n30A-n66A-n77A | n777,9 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66(2A)-n77A | n777,9 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66A-n77(2A) | n777,9 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n30A-n66(2A)-n77(2A) | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n30A-n66(3A)-n77A | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66(3A)-n77(2A) | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n34A-n39A-n40A | CA_n34A-n39A CA_n34A-n40A CA_n39A-n40A | n34 | 5, 10, 15 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n39A-n41A | CA_n34A-n39A CA_n34A-n41A CA_n39A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n39A-n41C | CA_n34A-n39A CA_n34A-n41A CA_n39A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n34A-n40A-n41A | CA_n34A-n40A CA_n34A-n41A CA_n40A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n40A-n41C | CA_n34A-n40A CA_n34A-n41A CA_n40A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n34A-n41A-n79A | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n41A-n79C | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n34A-n41C-n79A | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n41C-n79C | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n38A-n66A-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66A-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n38A-n66(2A)-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66(2A)-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n40A-n41C | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n40A-n79A CA_n39A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n41A-n79C | CA_n39A-n41A CA_n39A-n79A CA_n41A-n79A | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n39A-n41A-n79A | CA_n39A-n41A CA_n39A-n79A CA_n41A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n40A-n41A-n79A | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | , 40, 50, 60, 80, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | , 40, 50, 60, 80, 100 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n41C-n79A | CA_n41C CA_n41A-n79A CA_n40A-n41A CA_n40A-n79A | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n78A-n105A | CA_n40A-n78A CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n41A-n66A-n70A | CA_n41A-n66A CA_n41A-n70A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n71B | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41A-n66A-n71(2A) | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41A-n66(2A)-n71A | n417,9 CA_n41A-n66A7 CA_n66A-n71A CA_n41A-n71A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n71B | n417,9 CA_n41A-n66A7 CA_n66A-n71A CA_n41A-n71A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41A-n66(2A)-n71(2A) | n417,9 CA_n41A-n66A7 CA_n66A-n71A CA_n41A-n71A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71B | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(2A)-n66A-n71(2A) | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A)-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66(2A)-n71(2A) | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A)-n71B | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(3A)-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66(2A)-n71A | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66A-n71B | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(3A)-n66A-n71(2A) | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41C-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71B | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41C-n66A-n71(2A) | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41C-n66(2A)-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A)-n71(2A) | CA_n41A-n71A CA_n41A-n66A CA_n41C CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41C-n66(2A)-n71B | CA_n41A-n71A CA_n41A-n66A CA_n41C CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(A-C)-n66A-n71A | n417,9 CA_n41C7 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n66A-n71B | CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(A-C)-n66A-n71(2A) | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A CA_n41C | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41(A-C)-n66(2A)-n71A | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A CA_n41C | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n77(2A) | n417,9 n777,9 CA_n41A-n77A7 CA_n41A-n66A7 CA_n66A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41A-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A CA_n66A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n77(2A) | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n77(2A) | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66(2A)-n77(2A) | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(3A)-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n77(2A) | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41C-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A)-n77(2A) | CA_n41A-n66A CA_n41A-n77A CA_n41C CA_n66A-n77A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(A-C)-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A CA_n41C CA_n66A-n77A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n66(2A)-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66(2A)-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n66A-n85A | CA_n41A-n66A CA_n41A-n85A CA_n66A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n85A | CA_n41A-n66A CA_n41A-n85A CA_n66A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n85A | CA_n41A-n66A CA_n41A-n85A CA_n66A-n85A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n85A | CA_n41A-n66A CA_n41A-n85A CA_n41C CA_n66A-n85A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n70A-n78A | CA_n41A-n70A CA_n41A-n78A CA_n70A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41A-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71A-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71A-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(3A)-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n71B-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n71(2A)-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(3A)-n71B-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71A-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41C-n71B-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41C-n71(2A)-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(A-C)-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n71B-n77A | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71A-n78A | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n78(2A) | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n71A-n85A | CA_n41A-n71A CA_n41A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n85A | CA_n41A-n71A CA_n41A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n85A | CA_n41A-n71A CA_n41A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n77A-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n77(2A)-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n77(3A)-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n77A-n85A | CA_n41A-n77A CA_n41A-n85A CA_n77A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n77A-n85A | CA_n41A-n77A CA_n41A-n85A CA_n77A-n85A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n77(2A)-n85A | CA_n41A-n77A CA_n41A-n85A CA_n77A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n77A-n85A | CA_n41A-n77A CA_n41A-n85A CA_n41C CA_n77A-n85A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |

##### Table 5.5A.3.2-1c

Table 5.5A.3.2-1c: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n46A-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46M-n48A-n96A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48A-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46M-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46M-n48C-n96A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 1001 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46M-n48A-n96B | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48A-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48A-n96C | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48A-n96C | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48A-n96C | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48A-n96C | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46M-n48A-n96C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48A-n96C | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46M-n48B-n96C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48C-n96C | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48C-n96C | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48C-n96C | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48C-n96C | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46M-n48C-n96C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48C-n96C | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48A-n96D | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48A-n96D | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48A-n96D | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48A-n96D | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46M-n48A-n96D | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48A-n96D | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46M-n48C-n96D | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48A-n96E | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48A-n96E | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48A-n96E | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48A-n96E | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46M-n48A-n96E | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48A-n96E | - | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46M-n48C-n96E | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46M-n48(2A)-n96A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46M-n48(2A)-n96B | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46M-n48(2A)-n96C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46M-n48(2A)-n96D | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46M-n48(2A)-n96E | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46M-n48(3A)-n96A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46M-n48(3A)-n96B | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46M-n48(3A)-n96C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46M-n48(3A)-n96D | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46M-n48(3A)-n96E | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46M-n48(4A)-n96A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46M-n48(4A)-n96B | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46M-n48(4A)-n96C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46M-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46M-n48(4A)-n96E | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS1 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n78A-n102A | CA_n46A-n78A CA_n78A-n102A | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46A-n78A-n102B | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102B | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46A-n78A-n102C | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102C | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46A-n78A-n102D | CA_n46A-n78A CA_n78A-n102A | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46A-n78A-n102E | CA_n46A-n78A CA_n78A-n102A | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46A-n78A-n102(2A) | CA_n46A-n78A CA_n78A-n102A | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46(2A)-n78A-n102A | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46(2A)-n78A-n102B | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102B | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46(2A)-n78A-n102C | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102C | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46(2A)-n78A-n102D | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46(2A)-n78A-n102E | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46(2A)-n78A-n102(2A) | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46C-n78A-n102A | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46C-n78A-n102B | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102B | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46C-n78A-n102C | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102C | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46C-n78A-n102D | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46C-n78A-n102E | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46C-n78A-n102(2A) | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46D-n78A-n102A | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46D-n78A-n102B | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102B | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46D-n78A-n102C | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102C | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46D-n78A-n102D | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46D-n78A-n102E | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46D-n78A-n102(2A) | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46A-n78(2A)-n102A | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46A-n78(2A)-n102B | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46A-n78(2A)-n102C | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46A-n78(2A)-n102D | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46A-n78(2A)-n102E | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46A-n78(2A)-n102(2A) | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | 10,20, 40, 60, 80, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46(2A)-n78(2A)-n102A | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46(2A)-n78(2A)-n102B | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46(2A)-n78(2A)-n102C | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46(2A)-n78(2A)-n102D | CA_n46A-n78A CA_n78A-n102A | n46 | CA_n46(2A)_BCS0 | 0 |
|  | CA_n78(2A) | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46(2A)-n78(2A)-n102E | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46(2A)-n78(2A)-n102(2A) | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46C-n78(2A)-n102A | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46C-n78(2A)-n102B | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46C-n78(2A)-n102C | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46C-n78(2A)-n102D | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46C-n78(2A)-n102E | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46C-n78(2A)-n102(2A) | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46D-n78(2A)-n102A | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46D-n78(2A)-n102B | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46D-n78(2A)-n102C | CA_n46A-n78A CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46D-n78(2A)-n102D | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46D-n78(2A)-n102E | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46D-n78(2A)-n102(2A) | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66A-n71A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66(2A)-n71A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n66A-n71A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n66A-n71A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n71(2A) | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48A-n66A-n77A | n777,9 CA_n48A-n66A CA_n66A-n77A7 | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66(2A)-n77A | CA_n48A-n66A CA_n66A-n77A7 | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66A-n77C | n777,9 CA_n48A-n66A CA_n66A-n77A7 CA_n77C | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48B-n66A-n77C | n777,9 CA_n48A-n66A CA_n66A-n77A7 CA_n77C | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48B-n66A-n77A | n777,9 CA_n48A-n66A CA_n66A-n77A7 | n48 | CA_n48B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77A | n777,9 CA_n48A-n66A CA_n66A-n77A7 | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77C | n777,9 CA_n77C CA_n48A-n66A CA_n66A-n77A7 | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48A-n70A-n71A | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n70A-n71A | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n70A-n71A | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n70A-n71(2A) | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48A-n70A-n77A | CA_n48A-n70A CA_n70A-n77A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n70A-n77A | CA_n48A-n70A CA_n70A-n77A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n71A-n77A | CA_n48A-n71A CA_n71A-n77A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n71A-n77A | CA_n48A-n71A CA_n71A-n77A | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n70A-n78A | CA_n66A-n78A CA_n70A-n78A | n66 | 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66B_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(3A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n70A-n77A | CA_n66A-n77A CA_n70A-n77A | n66 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n70A-n77A | CA_n66A-n77A CA_n70A-n77A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71A-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71B-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71B-n77(2A) | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n66A-n71(2A)-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71(2A)-n77(2A) | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n66(2A)-n71A-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | CA_n66(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71A-n77(2A) | n777,9 CA_n77(2A)7 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n66A-n71A-n77(3A) | n777,9 CA_n77(2A)7 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n66(2A)-n71B-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n71B-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n66(2A)-n71(2A)-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n71A-n77(2A) | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n66(2A)-n71(2A)-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | CA_n66(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n66A-n71A-n78A | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71A-n78(2A) | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n66(2A)-n71A-n78A | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n71A-n78(2A) | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n66A-n71A-n85A | CA_n66A-n71A CA_n66A-n85A | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n77A-n85A | CA_n66A-n77A CA_n66A-n85A CA_n77A-n85A | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n77(2A)-n85A | CA_n66A-n77A CA_n66A-n85A CA_n77A-n85A | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n77A-n85A | CA_n66A-n77A CA_n66A-n85A CA_n77A-n85A | n66 | CA_n66(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n70A-n71A-n77A | CA_n70A-n71A CA_n70A-n77A CA_n71A-n77A | n70 | 5, 10, 15, 20, 25 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

The following notes are applied to the above tables.

NOTE 1:	This UE channel bandwidth is applicable only to downlink

NOTE 2:	For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz.

NOTE 3: 	The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1.

NOTE 4:	The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations.

NOTE 5:	Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation.

NOTE 6:	Only single uplink carriers with power class other than PC3 are listed.

NOTE 7:	Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination

NOTE 8:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration.

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination

NOTE 10:	For a band combination which include band n7 and n38 simultaneously, carriers in band n7 and n38 can only be configured as downlink carriers. Power imbalance between downlink carriers on Band n7 and Band n38 is assumed to be within 6dB.

NOTE 11: UL carrier shall be supported in Band n28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB.

NOTE 12:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration

#### 5.5A.3.3	Configurations for inter-band CA (four bands)

Table 5.5A.3.3-1: Void

##### Table 5.5A.3.3-1a

Table 5.5A.3.3-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier 4 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A-n7A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n7A CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3A-n5A-n7B | CA_n1A-n3A CA_n1A-n5A CA_n1A-n7A CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3A-n5A-n28A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n28A CA_n3A-n5A CA_n3A-n28A CA_n5A-n28A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n5A-n78A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n78A CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n8A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n26A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7A-n26A | CA_n3B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7B-n26A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7B-n26A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n26(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3B-n7A-n26(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3A-n7B-n26(2A) | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3B-n7B-n26(2A) | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3A-n7A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
| CA_n1A-n3A-n7B-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  | CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7A-n28A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7B-n28A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3A-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3B-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3B-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3(2A)-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3(2A)-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n7A-n67A | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n75A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3B-n7A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7B-n78A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n7B-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7A-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n7B-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3B-n7B-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3A-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3A-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3B-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3B-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3B-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3B-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3(2A)-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3(2A)-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3(2A)-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3(2A)-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3A-n7A-n105A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n105A CA_n3A-n7A CA_n3A-n105A CA_n7A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10,15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n8A-n77A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n8A-n77(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n8A-n78A | CA_n1A-n3A CA_n1A-n8A CA_n1A-n78A CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
| CA_n1A-n3A-n18A-n28A | CA_n1A-n3A CA_n1A-n18A CA_n1A-n28A CA_n3A-n18A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n1A-n3A-n18A-n41A | CA_n1A-n3A CA_n1A-n18A CA_n1A-n41A CA_n3A-n18A CA_n3A-n41A CA_n18A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n18A-n77A | CA_n1A-n3A CA_n1A-n18A CA_n1A-n77A CA_n3A-n18A CA_n3A-n77A CA_n18A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n20A-n67A | CA_n1A-n3A CA_n1A-n20A CA_n3A-n20A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A) BCS0 |  |
| CA_n1A-n3A-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3B-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3B-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n28A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n28A-n41A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n3A-n28A CA_n3A-n41A CA_n28A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n28A-n77A | n775,6 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | n775 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n28A-n77(2A) | n775 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n3A-n28A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202,302 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202, 302 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3B-n28A-n78A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n28A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n28A-n79A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n3A-n28A CA_n3A-n79A CA_n28A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n40A-n77A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n77A CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n40A-n105A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n105A CA_n3A-n40A CA_n3A-n105A CA_n40A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n41A-n77A | n415 n775 CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n41A-n77(2A) | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n3A-n41A-n79A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n67A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n67A-n78(2A) | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n75A-n78A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n77A-n79A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n77(2A)-n79A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n78A-n105A | CA_n1A-n3A CA_n1A-n78A CA_n1A-n105A CA_n3A-n78A CA_n3A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25,30, 35 |  |
| CA_n1A-n5A-n7A-n78A | CA_n1A-n5A CA_n1A-n7A CA_n1A-n78A CA_n5A-n7A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n7B-n78A | CA_n1A-n5A CA_n1A-n7A CA_n1A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n28A-n78A | CA_n1A-n5A CA_n1A-n28A CA_n1A-n78A CA_n5A-n28A CA_n5A-n78A CA_n28A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n28A-n79A | CA_n1A-n5A CA_n1A-n28A CA_n1A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n40A-n78A | CA_n1A-n5A CA_n1A-n40A CA_n1A-n78A CA_n5A-n40A CA_n5A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n78A-n79A | CA_n1A-n5A CA_n1A-n78A CA_n1A-n79A CA_n5A-n78A CA_n5A-n79A CA_n78A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n8A-n40A | CA_n1A-n7A CA_n1A-n8A CA_n1A-n40A CA_n7A-n8A CA_n7A-n40A CA_n8A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n7A-n8A-n78A | CA_n1A-n7A CA_n1A-n8A CA_n1A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n26A-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n26A-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A) BCS0 |  |
| CA_n1A-n7A-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7B-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7B-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7A-n28A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n7A-n28A-n78A | CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n28A-n78A | CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n7B CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7A-n38A-n78A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n40A-n78A | CA_n1A-n7A CA_n1A-n40A CA_n1A-n78A CA_n7A-n40A CA_n7A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n40A-n105A | CA_n1A-n7A CA_n1A-n40A CA_n1A-n105A CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n7A-n67A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n67A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7A-n75A-n78A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n78A-n105A | CA_n1A-n7A CA_n1A-n78A CA_n1A-n105A CA_n7A-n78A CA_n7A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n8A-n40A-n78A | CA_n1A-n8A CA_n1A-n40A CA_n1A-n78A CA_n8A-n40A CA_n8A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n8A-n78A-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n8A-n78(2A)-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n18A-n28A-n41A | CA_n1A-n18A CA_n1A-n28A CA_n1A-n41A CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n28A-n77A | CA_n1A-n18A CA_n1A-n28A CA_n1A-n77A CA_n18A-n28A CA_n18A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n18A-n41A-n77A | CA_n1A-n18A CA_n1A-n41A CA_n1A-n77A CA_n18A-n41A CA_n18A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40A-n77A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40A-n78A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n78A CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40B-n78A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n78A CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n41A-n77A | n415 n775,6 CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n41A-n77(2A) | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n28A-n41A-n79A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n28A-n75A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n75 | 5, 10, 15, 20, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n77A-n79A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n28A-n78A-n79A | CA_n1A-n28A CA_n1A-n78A CA_n1A-n79A CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n28A-n77(2A)-n79A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n40A-n78A-n105A | CA_n1A-n40A CA_n1A-n78A CA_n1A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n41A-n77A-n79A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n41A-n77(2A)-n79A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A-n30A-n66A | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n30A-n66A | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n30A-n66(2A) | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n5A-n30A-n77A | n775,6 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n30A-n77A | n775 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n30A-n77(2A) | n775 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n30A-n77(2A) | n775 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n48A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48B-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48(2A)-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48(A-B)-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48A-n77C | n775,6 CA_n77C CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | n775,6 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n5A-n48B-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48(2A)-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66A-n77A | n775,6 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n66A-n77A | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30,40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66(2A)-n77A | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66A-n77(2A) | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n66(2A)-n77(2A) | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n5A-n66A-n77(2A) | n775 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n66A-n77C | n775,6 CA_n77C CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n77A5 CA_n5A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n12A-n30A-n66A | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n12A-n30A-n66A | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n12A-n30A-n66(2A) | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n12A-n30A-n77A | n775,6 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n30A-n77A | n775 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n30A-n77(2A) | n775 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n30A-n77(2A) | n775 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n12A-n66A-n77A | n775,6 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n66A-n77A | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n66(2A)-n77A | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n66A-n77(2A) | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n12A-n66(2A)-n77(2A) | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n66A-n77(2A) | n775 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A-n66A | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n30A-n66A | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n14A-n30A-n66(2A) | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n30A-n77A | n775,6 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n14A-n30A-n77A | n775 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n30A-n77(2A) | n775 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n30A-n77(2A) | n775 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n66A-n77A | n775,6 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n14A-n66A-n77A | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n66(2A)-n77A | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n66A-n77(2A) | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n66(2A)-n77(2A) | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n66A-n77(2A) | n775 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n29A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n29A-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n29A-n30A-n77A | n775,6 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n30A-n77A | n775 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n30A-n77(2A) | n775 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n30A-n77(2A) | n775 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n66A-n77A | n775,6 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n66A-n77A | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n66(2A)-n77A | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n66A-n77(2A) | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n66A-n77(2A) | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n66(2A)-n77(2A) | n775 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66A-n77A | n775,6 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n30A-n66A-n77A | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n66(2A)-n77A | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n66A-n77(2A) | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66(2A)-n77(2A) | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n30A-n66A-n77(2A) | n775 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n41A-n66A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n48A-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48B-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48(2A)-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A-n77C | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | n775,6 CA_n77C CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n66A-n71A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n71A-n77(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |
| CA_n2A-n66A-n71A-n78A | - | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n71A-n78(2A) | - | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n5A-n7A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n3A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A-n7B-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n3A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A-n28A-n78A | CA_n3A-n5A CA_n3A-n28A CA_n3A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n5A-n28A-n79A | CA_n3A-n5A CA_n3A-n28A CA_n3A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n8A-n78A | CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n7A-n20A-n67A | CA_n3A-n7A CA_n3A-n20A CA_n7A-n20A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n20A-n78A | CA_n3A-n7A CA_n3A-n20A CA_n3A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n20A-n78(2A) | CA_n3A-n7A CA_n3A-n20A CA_n3A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n7A-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A) BCS0 |  |
| CA_n3A-n7A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7A-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7B-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n7B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7A-n28A-n38A7 | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A-n7A-n28A-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n28A-n78(2A) | CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n7B-n28A-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n7B CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7A-n28A-n78A | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n28A-n78(2A) | CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7B-n28A-n78A | CA_n7B CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n7A-n38A-n78A7 | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n40A-n105A | CA_n3A-n7A CA_n3A-n40A CA_n3A-n105A CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n7A-n67A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n67A-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n7A-n75A-n78A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n78A-n105A | CA_n3A-n7A CA_n3A-n78A CA_n3A-n105A CA_n7A-n78A CA_n7A-n105A CA_n78A-n105A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n18A-n28A-n41A | CA_n3A-n18A CA_n3A-n28A CA_n3A-n41A CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n28A-n77A | CA_n3A-n18A CA_n3A-n28A CA_n3A-n77A CA_n18A-n28A CA_n18A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n18A-n41A-n77A | CA_n3A-n18A CA_n3A-n41A CA_n3A-n77A CA_n18A-n41A CA_n18A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n20A-n67A-n78A | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n20A-n67A-n78(2A) | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n28A-n38A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n40A-n77A | CA_n3A-n28A CA_n3A-n40A CA_n3A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n40 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n77A | CA_n3A-n28A CA_n3A-n41A5 CA_n3A-n77A5 CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A5 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n77(2A) | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n28A-n41A-n78A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n78A CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n78(2A) | CA_n3A-n28A CA_n3A-n41A CA_n3A-n78A CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n28A-n41A-n79A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n28A-n77A-n79A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n28A-n77(2A)-n79A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n40A-n78A-n105A | CA_n3A-n40A CA_n3A-n78A CA_n3A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25,30, 35 |  |
| CA_n3A-n41A-n77A-n79A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n41A-n77(2A)-n79A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

##### Table 5.5A.3.3-1b

Table 5.5A.3.3-1b: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier 4 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n25A-n29A-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
| CA_n5A-n25A-n66A-n77A | n775,6 CA_n5A-n25A CA_n5A-n66A CA_n5A-n77A5 CA_n25A-n66A CA_n25A-n77A5 CA_n66A-n77A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n66A-n77A | n775,6 CA_n5A-n25A CA_n5A-n66A CA_n5A-n77A5 CA_n25A-n66A CA_n25A-n77A5 CA_n66A-n77A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n66(2A)-n77A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n66A-n77(2A) | n775,6 CA_n5A-n25A CA_n5A-n66A CA_n5A-n77A5 CA_n25A-n66A CA_n25A-n77A5 CA_n66A-n77A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25(2A)-n66(2A)-n77A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n66A-n77(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n66(2A)-n77(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25(2A)-n66(2A)-n77(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n66A-n78A | n785 CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A5 CA_n25A-n66A CA_n25A-n78A5 CA_n66A-n78A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n66A-n78A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n66(2A)-n78A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n66A-n78(2A) | n785 CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A5 CA_n25A-n66A CA_n25A-n78A5 CA_n66A-n78A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25(2A)-n66(2A)-n78A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n66A-n78(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25A-n66(2A)-n78(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25(2A)-n66(2A)-n78(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n28A-n78A-n79A | CA_n5A-n28A CA_n5A-n78A CA_n5A-n79A CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n30A-n66A-n77A | n775,6 CA_n5A-n30A CA_n5A-n66A CA_n5A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n30A-n66(2A)-n77A | n775 CA_n5A-n30A CA_n5A-n66A CA_n5A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n30A-n66(2A)-n77(2A) | n775 CA_n5A-n30A CA_n5A-n66A CA_n5A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |
| CA_n5A-n30A-n66A-n77(2A) | n775 CA_n5A-n30A CA_n5A-n66A CA_n5A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n48A-n66A-n77A | n775,6 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n5 | 5, 10, 15, 20, 25 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n66A-n77C | n775,6 CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48B-n66A-n77A | n775,6 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n5 | 5, 10, 15, 20, 25 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20, 25 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20, 25 | 3 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48(2A)-n66A-n77A | n775,6 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n5 | 5, 10, 15, 20, 25 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20, 25 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n8A-n40A-n78A | CA_n7A-n8A CA_n7A-n40A CA_n7A-n78A CA_n8A-n40A CA_n8A-n78A CA_n40A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n12A-n25A-n66A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n20A-n67A-n78A | CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n20A-n67A-n78(2A) | CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A CA_n78(2A) | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n7A-n25A-n66A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n25A-n66A-n77A | n775,6 CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A5 CA_n25A-n66A CA_n25A-n77A5 CA_n66A-n77A5 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n66A-n77A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n66A-n77A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n66(2A)-n77A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n66A-n77(2A) | n775,6 CA_n77(2A)5 CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A5 CA_n25A-n66A CA_n25A-n77A5 CA_n66A-n77A5 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n66A-n77A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n66(2A)-n77A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n66A-n77(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25(2A)-n66(2A)-n77A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n66A-n77(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n66(2A)-n77(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n66(2A)-n77A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n66(2A)-n77(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n66A-n77(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25(2A)-n66(2A)-n77(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n66(2A)-n77(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n66A-n78A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n66A-n78A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n66(2A)-n78A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n66A-n78(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n25A-n66A-n78A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n66A-n78(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n25(2A)-n66(2A)-n78A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n66(2A)-n78(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n25(2A)-n66A-n78A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n66(2A)-n78A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n66A-n78(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n25(2A)-n66(2A)-n78(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n25(2A)-n66A-n78(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n25(2A)-n66(2A)-n78A | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n66(2A)-n78(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n25(2A)-n66(2A)-n78(2A) | CA_n7A-n25A CA_n7A-n66A CA_n7A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n28A-n38A-n78A7 | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n40A-n78A-n105A | CA_n7A-n40A CA_n7A-n78A CA_n7A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n8A-n20A-n28A-n75A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n12A-n30A-n66A-n77A | n775,6 CA_n12A-n30A CA_n12A-n66A CA_n12A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n12 | 5, 10,15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n30A-n66(2A)-n77A | n775 CA_n12A-n30A CA_n12A-n66A CA_n12A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n12 | 5, 10,15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n30A-n66A-n77(2A) | n775 CA_n12A-n30A CA_n12A-n66A CA_n12A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n12 | 5, 10,15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n30A-n66(2A)-n77(2A) | n775 CA_n12A-n30A CA_n12A-n66A CA_n12A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n12 | 5, 10,15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n13A-n25A-n66A-n77A | CA_n13A-n25A CA_n13A-n66A CA_n13A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n25A-n66A-n77(2A) | CA_n77(2A) CA_n13A-n25A CA_n13A-n66A CA_n13A-n77A CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n30A-n66A-n77A | n775,6 CA_n14A-n30A CA_n14A-n66A CA_n14A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n30A-n66(2A)-n77A | n775 CA_n14A-n30A CA_n14A-n66A CA_n14A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n30A-n66A-n77(2A) | n775 CA_n14A-n30A CA_n14A-n66A CA_n14A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n30A-n66(2A)-n77(2A) | n775 CA_n14A-n30A CA_n14A-n66A CA_n14A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n28A-n41A-n77A | CA_n18A-n28A CA_n18A-n41A CA_n18A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n38A-n66A-n78A | CA_n25A-n38A CA_n25A-n66A CA_n25A-n78A CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n38A-n66A-n78A | CA_n25A-n38A CA_n25A-n66A CA_n25A-n78A CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n38A-n66(2A)-n78A | CA_n25A-n38A CA_n25A-n66A CA_n25A-n78A CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n38A-n66A-n78(2A) | CA_n25A-n38A CA_n25A-n66A CA_n25A-n78A CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n38A-n66(2A)-n78A | CA_n25A-n38A CA_n25A-n66A CA_n25A-n78A CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n38A-n66A-n78(2A) | CA_n25A-n38A CA_n25A-n66A CA_n25A-n78A CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n38A-n66(2A)-n78(2A) | CA_n25A-n38A CA_n25A-n66A CA_n25A-n78A CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n38A-n66(2A)-n78(2A) | CA_n25A-n38A CA_n25A-n66A CA_n25A-n78A CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n41A-n66A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | n415,6 CA_n25A-n41A CA_n25A-n66A5 CA_n25A-n71A CA_n41A-n66A5 CA_n41A-n71A5 CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(A-C)-n66A-n71A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66(2A)-n71A | n415,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A5 CA_n41A-n71A5 CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66A-n71(2A) | n415,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A5 CA_n41A-n71A5 CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25A-n41A-n66A-n71B | n415,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A5 CA_n41A-n71A5 CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n66A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | n415,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A5 CA_n41A-n71A5 CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A-n71(2A) | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n66A-n71B | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n66(2A)-n71A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | n415,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A5 CA_n41A-n71A5 CA_n41C5 CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A-n71(2A) | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25A-n41C-n66A-n71B | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n41C-n66(2A)-n71A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n66A-n71A | n415,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A5 CA_n41A-n71A5 CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n66A-n71A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C-n66A-n71A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(3A)-n66A-n71A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66A-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n77A5 CA_n41A-n66A5 CA_n41A-n77A5 CA_n66A-n77A5 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(A-C)-n66A-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41C CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n77A5 CA_n41A-n66A5 CA_n41A-n77A5 CA_n41C5 CA_n66A-n77A5 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A-n77(2A) | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41C CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41C-n66(2A)-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n41C CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | CA_66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n77A5 CA_n41A-n66A5 CA_n41A-n77A5 CA_n66A-n77A5 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A-n77(2A) | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n66A-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66(2A)-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n77A5 CA_n41A-n66A5 CA_n41A-n77A5 CA_n66A-n77A5 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66A-n77(2A) | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n77A5 CA_n41A-n66A5 CA_n41A-n77A5 CA_n66A-n77A5 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41A-n66(2A)-n77(2A) | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n66(2A)-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n66A-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n66A CA_n25A-n77A5 CA_n41A-n66A5 CA_n41A-n77A5 CA_n66A-n77A5 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n66A-n77(2A) | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n66(2A)-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C-n66A-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n66A-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n77A CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66A-n78A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n78A CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41A-n66A-n78(2A) | CA_n25A-n41A CA_n25A-n66A CA_n25A-n78A CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n41A-n66A-n85A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n85A CA_n41A-n66A CA_n41A-n85A CA_n66A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71A-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n71A CA_n25A-n77A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n71A-n77A5 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71A-n77(2A) | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n71A CA_n25A-n77A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n71A-n77A5 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41A-n71B-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n71A CA_n25A-n77A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n71A-n77A5 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71B-n77(2A) | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41A-n71(2A)-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n71A CA_n25A-n77A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n71A-n77A5 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71(2A)-n77(2A) | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41(A-C)-n71A-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41C CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71A-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n71A CA_n25A-n77A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n41C5 CA_n71A-n77A5 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71A-n77(2A) | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41C CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41C-n71B-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71(2A)-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71A-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n71A CA_n25A-n77A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n71A-n77A5 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71A-n77(2A) | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n71A-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71B-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71(2A)-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n71A-n77A | n415,6 n775,6 CA_n25A-n41A5 CA_n25A-n71A CA_n25A-n77A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n71A-n77A5 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n71(2A)-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n71B-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n71A-n77(2A) | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n71A-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n71A-n77A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71A-n78A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n78A CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41A-n71A-n85A | CA_n25A-n41A CA_n25A-n71A CA_n25A-n85A CA_n41A-n71A CA_n41A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n77A-n85A | CA_n25A-n41A CA_n25A-n77A CA_n25A-n85A CA_n41A-n77A CA_n41A-n85A CA_n77A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n71A-n77A | n775,6 CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A)-n71A-n77A | n775,6 CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A)-n71A-n77(2A) | CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n71B-n77A | n775,6 CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n71(2A)-n77A | n775,6 CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n71A-n77(2A) | n775,6 CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n71(2A)-n77(2A) | CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n71B-n77(2A) | CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n66A-n71A-n77A | n775,6 CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66A-n71A-n77(2A) | CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n71A-n78A | CA_n25A-n66A CA_n25A-n71A CA_n25A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66(2A)-n71A-n78A | CA_n25A-n66A CA_n25A-n71A CA_n25A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n71A-n78(2A) | CA_n25A-n66A CA_n25A-n71A CA_n25A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n66(2A)-n71A-n78(2A) | CA_n25A-n66A CA_n25A-n71A CA_n25A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n66A-n77A-n85A | CA_n25A-n66A CA_n25A-n77A CA_n25A-n85A CA_n66A-n77A CA_n66A-n85A CA_n77A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n41A-n77A-n79A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n41A-n77(2A)-n79A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n29A-n30A-n66A-n77A | n775,6 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n30A-n66(2A)-n77A | n775 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n30A-n66A-n77(2A) | n775 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n30A-n66(2A)-n77(2A) | n775 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41A-n66A-n70A-n78A | CA_n41A-n66A CA_n41A-n70A CA_n41A-n78A CA_n66A-n78A CA_n70A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n71A-n77A | n415,6 n775,6 CA_n41A-n66A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n71B-n77A | n415,6 n775,6 CA_n41A-n66A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n71B-n77(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41A-n66A-n71(2A)-n77A | n415,6 n775,6 CA_n41A-n66A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n71(2A)-n77(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41A-n66(2A)-n71A-n77(2A) | n415,6 n775,6 CA_n41A-n66A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41(A-C)-n66A-n71A-n77A | CA_n41C CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71A-n77A | n415,6 n775,6 CA_n41A-n66A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n41C5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n41 | CA_n41C_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71A-n77(2A) | CA_n41C CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41C-n66A-n71B-n77A | CA_n41C CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71(2A)-n77A | CA_n41C CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A)-n71A-n77A | CA_n41C CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71A-n77(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_77(2A)_BCS 4 and 5 |  |
| CA_n41(3A)-n66A-n71A-n77A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71A-n77A | n415,6 n775,6 CA_n41A-n66A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71B-n77A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71(2A)-n77A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66(2A)-n71A-n77A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n71A-n77A | n415,6 n775,6 CA_n41A-n66A5 CA_n41A-n71A5 CA_n41A-n77A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n71(2A)-n77A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n71B-n77A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n71A-n77(2A) | n415,6 n775,6 CA_n41A-n66A5 CA_n41A-n77A5 CA_n41A-n71A5 CA_n66A-n71A CA_n66A-n77A5 CA_n71A-n77A5 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41A-n66A-n71A-n78A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66(2A)-n71A-n78A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n71A-n78(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n66(2A)-n71A-n78(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n66A-n71A-n85A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n85A CA_n66A-n71A CA_n66A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n77A-n85A | CA_n41A-n66A CA_n41A-n77A CA_n41A-n85A CA_n66A-n77A CA_n66A-n85A CA_n77A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48A-n66A-n70A-n71A | CA_n48A-n66A CA_n48A-n70A CA_n48A-n71A CA_n66A-n71A CA_n70A-n71A | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n70A-n77A | CA_n48A-n66A CA_n48A-n70A CA_n66A-n77A CA_n70A-n77A | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66(2A)-n70A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66A-n71A-n77A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66(2A)-n71A-n77A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n70A-n71A-n77A | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A CA_n70A-n77A CA_n71A-n77A | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n70A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n70A-n71A CA_n70A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

The following notes are applied to the above tables.

NOTE 1:	This UE channel bandwidth is optional in this release of the specification.

NOTE 2:	For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz.

NOTE 3:	The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1.

NOTE 4: 	Only single uplink carriers with power class other than PC3 are listed.

NOTE 5:	Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination.

NOTE 6:	Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination.

NOTE 7:	For a band combination which includes band n7 and n38 simultaneously, carriers in band n7 and n38 can only be configured as downlink carriers. Power imbalance between downlink carriers on Band n7 and Band n38 is assumed to be within 6dB.

NOTE 8:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration

#### 5.5A.3.4	Configurations for inter-band CA (five bands)

Table 5.5A.3.4-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (five bands)

| NR CA configuration | Uplink configuration or single uplink carrier 2 | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A-n7A-n78A | CA_n1A-n3A CA_n1A-n5A | n1 | 5, 10, 15, 20 | 0 |
|  | CA_n1A-n7A CA_n1A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n3A-n5A CA_n3A-n7A | n5 | 5, 10, 15, 20 |  |
|  | CA_n3A-n78A CA_n5A-n7A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | CA_n5A-n78A CA_n7A-n78A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n5A-n7B-n78A | CA_n1A-n3A CA_n1A-n5A | n1 | 5, 10, 15, 20 | 0 |
|  | CA_n1A-n7A CA_n1A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n3A-n5A CA_n3A-n7A | n5 | 5, 10, 15, 20 |  |
|  | CA_n3A-n78A CA_n5A-n7A | n7 | CA_n7B_BCS0 |  |
|  | CA_n5A-n78A CA_n7A-n78A CA_n7B | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n5A-n28A-n78A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n28A CA_n1A-n79A CA_n3A-n5A CA_n3A-n28A CA_n3A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7A-n8A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n1A-n78A CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n7A-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3B-n7A-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7A-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7A-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3B-n7A-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3B-n7B-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7B-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7B-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A) BCS0 |  |
| CA_n1A-n3B-n7B-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n7B-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7B-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7B-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n7B-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n7A-n28A-n38A4 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n7A-n28A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A | n1 | 5, 10, 15, 20 | 1 |
|  | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n7A-n28A CA_n7A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | CA_n28A-n78A | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7B-n28A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n7B CA_n28A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n1A-n28A CA_n1A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | CA_n3A-n7A CA_n3A-n28A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | CA_n3A-n78A CA_n7A-n28A | n28 | 5, 10, 15, 20, 30 |  |
|  | CA_n7A-n78A CA_n28A-n78A | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3B-n7A-n28A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3B-n7B-n28A-n78A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n7A-n38A-n78A4 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n40A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n40A CA_n1A-n78A CA_n3A-n7A CA_n3A-n40A CA_n3A-n78A CA_n7A-n40A CA_n7A-n78A CA_n40A-n78A | n1 | 5, 10,15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10,15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n40A-n105A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n40A CA_n1A-n105A CA_n3A-n7A CA_n3A-n40A CA_n3A-n105A CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n1 | 5, 10,15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10,15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10,15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n7A-n67A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n67A-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n7A-n75A-n78A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7A-n78A-n105A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n1A-n105A CA_n3A-n7A CA_n3A-n78A CA_n3A-n105A CA_n7A-n78A CA_n7A-n105A CA_n78A-n105A | n1 | 5, 10,15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10,15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10,15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n28A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n28A-n41A-n77A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n28A-n41A-n79A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n28A-n77A-n79A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n40A-n78A-n105A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n78A CA_n1A-n105A CA_n3A-n40A CA_n3A-n78A CA_n3A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n40 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n41A-n77A-n79A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n5A-n28A-n78A-n79A | CA_n1A-n5A CA_n1A-n28A CA_n1A-n78A CA_n1A-n79A CA_n5A-n28A CA_n5A-n78A CA_n5A-n79A CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n28A-n38A-n78A4 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n40A-n78A-n105A | CA_n1A-n7A CA_n1A-n40A CA_n1A-n78A CA_n1A-n105A CA_n7A-n40A CA_n7A-n78A CA_n7A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n28A-n41A-n77A-n79A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A-n30A-n66A-n77A | n773 CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A3 CA_n5A-n30A CA_n5A-n66A CA_n5A-n77A3 CA_n30A-n66A CA_n30A-n77A3 CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n30A-n66A-n77(2A) | n773 CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A3 CA_n5A-n30A CA_n5A-n66A CA_n5A-n77A3 CA_n30A-n66A CA_n30A-n77A3 CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n48A-n66A-n77A | n773,5 CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A3 CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A3 CA_n48A-n66A CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48B-n66A-n77A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A3 CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A3 CA_n48A-n66A CA_n48B CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48A-n66A-n77C | n773,5 CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A3 CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A3 CA_n48A-n66A CA_n66A-n77A3 CA_n77C | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n12A-n30A-n66A-n77A | n773 CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A3 CA_n12A-n30A CA_n12A-n66A CA_n12A-n77A3 CA_n30A-n66A CA_n30A-n77A3 CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n30A-n66A-n77(2A) | n773 CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A3 CA_n12A-n30A CA_n12A-n66A CA_n12A-n77A3 CA_n30A-n66A CA_n30A-n77A3 CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A-n66A-n77A | n773 CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A3 CA_n14A-n30A CA_n14A-n66A CA_n14A-n77A3 CA_n30A-n66A CA_n30A-n77A3 CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n30A-n66A-n77(2A) | n773 CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A3 CA_n14A-n30A CA_n14A-n66A CA_n14A-n77A3 CA_n30A-n66A CA_n30A-n77A3 CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n30A-n66A-n77A | n773 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A3 CA_n30A-n66A CA_n30A-n77A3 CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n30A-n66A-n77(2A) | n773 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A3 CA_n30A-n66A CA_n30A-n77A3 CA_n66A-n77A3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n7A-n20A-n67A-n78A | CA_n3A-n7A CA_n3A-n20A CA_n3A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n20A-n67A-n78(2A) | CA_n3A-n7A CA_n3A-n20A CA_n3A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n7A-n28A-n38A-n78A4 | - | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n40A-n78A-n105A | CA_n3A-n7A CA_n3A-n40A CA_n3A-n78A CA_n3A-n105A CA_n7A-n40A CA_n7A-n78A CA_n7A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n28A-n41A-n77A-n79A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n25A-n41A-n66A-n71A-n77A | n413,4 n773,4 CA_n25A-n41A3 CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A3 CA_n41A-n66A3 CA_n41A-n71A3 CA_n41A-n77A3 CA_n66A-n71A CA_n66A-n77A3 CA_n71A-n77A3 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66A-n71A-n77(2A) | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n41A-n66(2A)-n71A-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66A-n71(2A)-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66A-n71B-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A-n71A-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A-n71A-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n66A-n71A-n77A | CA_n25A-n41A CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n41A-n66A CA_n41A-n71A CA_n41A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 1: The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively. NOTE 2: Only single uplink carriers with power class other than PC3 are listed. NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 4: For a band combination which includes band n7 and n38 simultaneously, carriers in band n7 and n38 can only be configured as downlink carriers. Power imbalance between downlink carriers on Band n7 and Band n38 is assumed to be within 6dB. NOTE 5: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration |  |  |  |  |

#### 5.5A.3.5	Configurations for inter-band CA (six bands)

Table 5.5A.3.5-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (six bands)

| NR CA configuration | Uplink configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n7A-n28A-n38A-n78A2 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n40A-n78A-n105A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n40A CA_n1A-n78A CA_n1A-n105A CA_n3A-n7A CA_n3A-n40A CA_n3A-n78A CA_n3A-n105A CA_n7A-n40A CA_n7A-n78A CA_n7A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n1 | 5, 10,15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10,15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10,15, 20, 25, 30, 35 |  |
| NOTE 1: The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively. NOTE 2: For a band combination which includes band n7 and n38 simultaneously, carriers in band n7 and n38 can only be configured as downlink carriers. Power imbalance between downlink carriers on Band n7 and Band n38 is assumed to be within 6dB. |  |  |  |  |