3GPP TR 37.863-01-01 V15.4.0 (2020-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of LTE 1 Down Link (DL) / 1 Up Link (UL) and 1 NR band

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

1	Scope	16

2	References	18

3	Definitions,	symbols and abbreviations	19

3.1	Definitions	19

3.2	Symbols	19

3.3	Abbreviations	19

4	Background	20

4.1	TR Maintenance	20

5	DC band combination of LTE 1DL/1UL + one NR band: General Part	20

5.1	UE RF General	20

5.1.1	Basic DC UE RF architectures at range 1	20

5.1.2	Basic DC UE RF architectures at range 2	22

5.2	Single Tx Switched UL	23

5.3	Harmonics and IMD study	23

5.3	Minimum requirements	24

6	DC band combination of LTE 1DL/1UL + one NR band: Specific Band Combination Part	25

6.1	DC_1A-n77	25

6.1.1	Operating bands for DC	25

6.1.2	Channel bandwidths per operating band for DC	26

6.1.5	MSD	29

6.1.6	Self-interference analysis	30

6.2	DC_1A-n78	31

6.2.1	Operating bands for DC	31

6.2.2	Channel bandwidths per operating band for DC	31

6.2.3	Co-existence studies	31

6.2.4	∆TIB and ∆RIB values	33

6.2.5	MSD	34

6.2.6	Self-interference analysis	34

6.3	DC_1A-n79	35

6.3.1	Operating bands for DC	35

6.3.2	Channel bandwidths per operating band for DC	35

6.3.3	Co-existence studies	35

6.3.4	∆TIB and ∆RIB values	37

6.3.5	MSD	37

6.4	DC_3A-n77	38

6.4.1	Operating bands for DC	38

6.4.2	Channel bandwidths per operating band for DC	38

6.4.3	Co-existence studies	38

6.4.5	MSD	41

6.4.6	Self-interference analysis	42

6.5	DC_3A-n78	42

6.5.1	Operating bands for DC	42

6.5.2	Channel bandwidths per operating band for DC	43

6.5.3	Co-existence studies	43

6.5.4	∆TIB and ∆RIB values	45

6.5.5	MSD	46

6.5.6	Self-interference analysis	47

6.6	DC_3A-n79	47

6.6.2	Channel bandwidths per operating band for DC	48

6.6.3	Co-existence studies	48

6.6.4	∆TIB and ∆RIB values	50

6.6.5	MSD	50

6.7	DC_11A-n77A	51

6.7.1	Operating bands for DC	51

6.7.2	Channel bandwidths per operating band for DC	51

6.7.3	Co-existence studies	51

6.7.4	∆TIB and ∆RIB values	53

6.7.5	MSD	53

6.8	DC_11A-n78A	54

6.8.1	Operating bands for DC	54

6.8.2	Channel bandwidths per operating band for DC	54

6.8.3	Co-existence studies	54

6.8.4	∆TIB and ∆RIB values	56

6.8.5	MSD	57

6.9	DC_11A-n79A	57

6.9.1	Operating bands for DC	57

6.9.2	Channel bandwidths per operating band for DC	57

6.9.3	Co-existence studies	57

6.9.4	∆TIB and ∆RIB values	59

6.9.5	MSD	60

6.9.6	Self-interference analysis	60

6.10	DC_11A-n257A	61

6.10.1	Operating bands for DC	61

6.10.2	Channel bandwidths per operating band for DC	61

6.10.3	Co-existence studies	61

6.10.4	∆TIB and ∆RIB values	63

6.10.5	MSD	63

6.11	DC_18A-n77A	64

6.11.1	Operating bands for DC	64

6.11.2	Channel bandwidths per operating band for DC	64

6.11.3	Co-existence studies	64

6.11.4	∆TIB and ∆RIB values	66

6.11.5	MSD	67

6.11.6	Self-interference analysis	67

6.12	DC_18A-n78A	68

6.12.1	Operating bands for DC	68

6.12.2	Channel bandwidths per operating band for DC	69

6.12.3	Co-existence studies	69

6.12.4	∆TIB and ∆RIB values	71

6.12.5	MSD	72

6.12.6	Self-interference analysis	72

6.13	DC_18A-n79A	73

6.13.1	Operating bands for DC	73

6.13.2	Channel bandwidths per operating band for DC	73

6.13.3	Co-existence studies	73

6.13.4	∆TIB and ∆RIB values	75

6.13.5	MSD	76

6.13.6	Self-interference analysis	76

6.14	DC_18A-n257A	76

6.14.1	Operating bands for DC	76

6.14.2	Channel bandwidths per operating band for DC	77

6.14.3	Co-existence studies	77

6.14.4	∆TIB and ∆RIB values	79

6.14.5	MSD	80

6.15	DC_19A-n77	80

6.15.1	Operating bands for DC	80

6.15.2	Channel bandwidths per operating band for DC	80

6.15.3	Co-existence studies	80

6.15.4	∆TIB and ∆RIB values	82

6.15.5	MSD	82

6.15.6	Self-interference analysis	83

6.16	DC_19A-n78	84

6.16.1	Operating bands for DC	84

6.16.2	Channel bandwidths per operating band for DC	84

6.16.3	Co-existence studies	84

6.16.4	∆TIB and ∆RIB values	86

6.16.5	MSD	86

6.16.6	Self-interference analysis	87

6.17	DC_19A-n79	87

6.17.1	Operating bands for DC	87

6.17.2	Channel bandwidths per operating band for DC	88

6.17.3	Co-existence studies	88

6.17.4	∆TIB and ∆RIB values	90

6.17.5	MSD	91

6.17.6	Self-interference analysis	91

6.18	DC_21A-n77	92

6.18.1	Operating bands for DC	92

6.18.2	Channel bandwidths per operating band for DC	92

6.18.3	Co-existence studies	92

6.18.4	∆TIB and ∆RIB values	94

6.18.5	MSD	94

6.19	DC_21A-n78	95

6.19.1	Operating bands for DC	95

6.19.2	Channel bandwidths per operating band for DC	95

6.19.3	Co-existence studies	95

6.19.4	∆TIB and ∆RIB values	97

6.19.5	MSD	97

6.20	DC_21A-n79	98

6.20.1	Operating bands for DC	98

6.20.2	Channel bandwidths per operating band for DC	98

6.20.3	Co-existence studies	98

6.20.4	∆TIB and ∆RIB values	100

6.20.5	MSD	100

6.20.6	Self-interference analysis	101

6.21	DC_26A-n77A	102

6.21.1	Operating bands for DC	102

6.21.2	Channel bandwidths per operating band for DC	102

6.21.3	Co-existence studies	102

6.21.4	∆TIB and ∆RIB values	104

6.21.5	MSD	105

6.21.6	Self-interference analysis	106

6.22	DC_26A-n78A	107

6.22.1	Operating bands for DC	107

6.22.2	Channel bandwidths per operating band for DC	107

6.22.3	Co-existence studies	107

6.22.4	∆TIB and ∆RIB values	109

6.22.5	MSD	110

6.22.6	Self-interference analysis	110

6.23	DC_26A-n79A	111

6.23.1	Operating bands for DC	111

6.23.2	Channel bandwidths per operating band for DC	111

6.23.3	Co-existence studies	112

6.23.4	∆TIB and ∆RIB values	114

6.23.5	MSD	115

6.23.6	Self-interference analysis	115

6.24	DC_26A-n257A	116

6.24.1	Operating bands for DC	116

6.24.2	Channel bandwidths per operating band for DC	116

6.24.3	Co-existence studies	116

6.24.4	∆TIB and ∆RIB values	118

6.24.5	MSD	118

6.25	DC_28A-n77	119

6.25.1	Operating bands for DC	119

6.25.2	Channel bandwidths per operating band for DC	119

6.25.3	Co-existence studies	119

6.25.4	∆TIB and ∆RIB values	121

6.25.5	MSD	122

6.25.6	Self-interference analysis	123

6.26	DC_28A-n78	123

6.26.1	Operating bands for DC	123

6.26.2	Channel bandwidths per operating band for DC	124

6.26.3	Co-existence studies	124

6.26.4	∆TIB and ∆RIB values	126

6.26.6	Self-interference analysis	127

6.27	DC_28A-n79	127

6.27.1	Operating bands for DC	127

6.27.2	Channel bandwidths per operating band for DC	128

6.27.3	Co-existence studies	128

6.27.4	∆TIB and ∆RIB values	130

6.27.5	MSD	131

6.27.6	Self-interference analysis	131

6.28	DC_41A-n257A	132

6.28.1	Operating bands for DC	132

6.28.2	Channel bandwidths per operating band for DC	132

6.28.3	Co-existence studies	132

6.28.4	∆TIB and ∆RIB values	134

6.28.5	MSD	134

6.29	DC_42A-n77	134

6.29.1	Operating bands for DC	134

6.29.2	Channel bandwidths per operating band for DC	135

6.29.3	Co-existence studies	135

6.29.4	∆TIB and ∆RIB values	135

6.29.5	MSD	135

6.30	DC_42A-n78	136

6.30.1	Operating bands for DC	136

6.30.2	Channel bandwidths per operating band for DC	136

6.30.3	Co-existence studies	136

6.30.4	∆TIB and ∆RIB values	136

6.30.5	MSD	136

6.31	DC_42A-n79	137

6.31.1	Operating bands for DC	137

6.31.2	Channel bandwidths per operating band for DC	137

6.31.3	Co-existence studies	137

6.31.4	∆TIB and ∆RIB values	139

6.31.5	MSD	139

6.32	DC_20A-n28A	140

6.32.1	Operating bands for DC	140

6.32.2	Channel bandwidths per operating band for DC	140

6.32.3	Co-existence studies	140

6.32.4	∆TIB and ∆RIB values	142

6.32.5	MSD	142

6.33	DC_39A-n78A	143

6.33.1	Operating bands for DC	143

6.33.2	Channel bandwidths per operating band for DC	143

6.33.3	Co-existence studies	143

6.33.4	∆TIB and ∆RIB values	145

6.33.5	MSD	146

6.33.6	Self-interference analysis	146

6.34 DC_39A-n258A	147

6.34.1	Operating bands for DC	147

6.34.2	Channel bandwidths per operating band for DC	147

6.34.3	Co-existence studies	147

6.34.4	∆TIB and ∆RIB values	149

6.34.5	MSD	150

6.35	DC_1A-n257	150

6.35.1	Operating bands for DC	150

6.35.2	Channel bandwidths per operating band for DC	150

6.35.3	Co-existence studies	150

6.35.4	∆TIB and ∆RIB values	152

6.35.5	MSD	152

6.36	DC_3A-n257	153

6.36.1	Operating bands for DC	153

6.36.2	Channel bandwidths per operating band for DC	153

6.36.3	Co-existence studies	153

6.36.5	MSD	155

6.37	DC_3A_n258A	156

6.37.1	Operating bands for DC	156

6.37.2	Channel bandwidths per operating band for DC	156

6.37.3	Co-existence studies	156

6.37.4	∆TIB and ∆RIB values	158

6.37.5	MSD	158

6.38	DC_5A_n257A	159

6.38.1	Operating bands for DC	159

6.38.2	Channel bandwidths per operating band for DC	159

6.38.3	Co-existence studies	159

6.38.5	MSD	162

6.39	DC_7A-n78	162

6.39.1	Operating bands for DC	162

6.39.2	Channel bandwidths per operating band for DC	162

6.39.3	Co-existence studies	162

6.39.4	∆TIB and ∆RIB values	165

6.39.5	MSD	165

6.40	DC_7A_n258A	166

6.40.1	Operating bands for DC	166

6.40.2	Channel bandwidths per operating band for DC	166

6.40.3	Co-existence studies	166

6.40.5	MSD	168

6.41	DC_8A-n78A	169

6.41.1	Operating bands for DC	169

6.41.2	Channel bandwidths per operating band for DC	169

6.41.3	Co-existence studies	169

6.41.4	∆TIB and ∆RIB values	171

6.41.5	MSD	172

6.41.6	Self-interference analysis	172

6.42	DC_8A-n79A	173

6.42.1	Operating bands for DC	173

6.42.2	Channel bandwidths per operating band for DC	174

6.42.3	Co-existence studies	174

6.42.4	∆TIB and ∆RIB values	176

6.42.5	MSD	176

6.42.6	Self-interference analysis	177

6.43	DC_8A-n258A	178

6.43.1	Operating bands for DC	178

6.43.2	Channel bandwidths per operating band for DC	178

6.43.3	Co-existence studies	178

6.43.4	∆TIB and ∆RIB values	180

6.44	DC_19A-n257	181

6.44.1	Operating bands for DC	181

6.44.2	Channel bandwidths per operating band for DC	181

6.44.3	Co-existence studies	181

6.44.4	∆TIB and ∆RIB values	183

6.44.5	MSD	183

6.45	DC_20A-n78A	184

6.45.1	Operating bands for DC	184

6.45.2	Channel bandwidths per operating band for DC	184

6.45.3	Co-existence studies	184

6.45.4	∆TIB and ∆RIB values	186

6.45.5	MSD	187

6.45.6	Self-interference analysis	187

6.46	DC_21A-n257	188

6.46.1	Operating bands for DC	188

6.46.2	Channel bandwidths per operating band for DC	189

6.46.3	Co-existence studies	189

6.46.4	∆TIB and ∆RIB values	191

6.46.5	MSD	191

6.47	DC_28A-n257	192

6.47.1	Operating bands for DC	192

6.47.2	Channel bandwidths per operating band for DC	192

6.47.3	Co-existence studies	192

6.47.5	MSD	195

6.48	DC_28A_n258A	195

6.48.1	Operating bands for DC	195

6.48.2	Channel bandwidths per operating band for DC	195

6.48.3	Co-existence studies	195

6.48.4	∆TIB and ∆RIB values	197

6.48.5	MSD	198

6.49	DC_38A-n78A	198

6.49.1	Operating bands for DC	198

6.49.2	Channel bandwidths per operating band for DC	198

6.49.3	Co-existence studies	198

6.49.4	∆TIB and ∆RIB values	198

6.49.5	MSD	199

6.50	DC_39A-n79A	199

6.50.1	Operating bands for DC	199

6.50.2	Channel bandwidths per operating band for DC	199

6.50.3	Co-existence studies	199

6.50.4	∆TIB and ∆RIB values	201

6.50.5	MSD	202

6.51	DC_41A-n77A	202

6.51.1	Operating bands for DC	202

6.51.2	Channel bandwidths per operating band for DC	202

6.51.3	Co-existence studies	202

6.51.4	∆TIB and ∆RIB values	204

6.51.5	MSD	205

6.51.6	Self-interference analysis	206

6.52	DC_41A-n78A	207

6.52.1	Operating bands for DC	207

6.52.2	Channel bandwidths per operating band for DC	207

6.52.3	Co-existence studies	207

6.52.4	∆TIB and ∆RIB values	209

6.52.5	MSD	210

6.52.6	Self-interference analysis	211

6.53	DC_41A-n79A	212

6.53.1	Operating bands for DC	212

6.53.2	Channel bandwidths per operating band for DC	212

6.53.3	Co-existence studies	212

6.53.4	∆TIB and ∆RIB values	214

6.53.5	MSD	215

6.53.6	Self-interference analysis	215

6.54	DC_41A-n258A	216

6.54.1	Operating bands for DC	216

6.54.2	Channel bandwidths per operating band for DC	216

6.54.3	Co-existence studies	216

6.54.4	∆TIB and ∆RIB values	218

6.54.5	MSD	218

6.55	DC_42A_n257	219

6.55.1	Operating bands for DC	219

6.55.2	Channel bandwidths per operating band for DC	219

6.55.3	Co-existence studies	219

6.55.5	MSD	221

6.56	DC_1A_n28	222

6.56.1	Operating bands for DC	222

6.56.2	Channel bandwidths per operating band for DC	222

6.56.3	Co-existence studies	222

6.56.4	∆TIB and ∆RIB values	225

6.56.5	MSD	226

6.56.6	Self-interference analysis	226

6.57	DC_3A_n7A	227

6.57.1	Operating bands for DC	227

6.57.2	Channel bandwidths per operating band for DC	227

6.57.3	Co-existence studies	227

6.57.4	∆TIB and ∆RIB values	230

6.58	DC_3A_n28	231

6.58.1	Operating bands for DC	231

6.58.2	Channel bandwidths per operating band for DC	231

6.58.3	Co-existence studies	231

6.58.4	∆TIB and ∆RIB values	234

6.58.5	MSD	234

6.59	DC_5A-n78A	235

6.59.1	Operating bands for DC	235

6.59.2	Channel bandwidths per operating band for DC	235

6.59.3	Co-existence studies	235

6.59.4	∆TIB and ∆RIB values	238

6.59.5	MSD	238

6.60	DC_7A_n28	239

6.60.1	Operating bands for DC	239

6.60.2	Channel bandwidths per operating band for DC	240

6.60.3	Co-existence studies	240

6.60.4	∆TIB and ∆RIB values	242

6.60.5	MSD	242

6.61	DC_7A-n257A	243

6.61.1	Operating bands for DC	243

6.61.2	Channel bandwidths per operating band for DC	243

6.61.3	Co-existence studies	243

6.61.4	∆TIB and ∆RIB values	245

6.61.5	MSD	245

6.62	DC_66_n257	246

6.62.1	Operating bands for DC	246

6.62.2	Channel bandwidths per operating band for DC	246

6.62.3	Co-existence studies	247

6.62.4	∆TIB and ∆RIB values	248

6.62.5	MSD	249

6.63	DC_8A-n77A	249

6.63.6	Self-interference analysis	253

6.64	DC_8A-n257A	254

6.64.1	Operating bands for DC	254

6.64.2	Channel bandwidths per operating band for DC	254

6.64.3	Co-existence studies	254

6.64.4	∆TIB and ∆RIB values	257

6.64.5	MSD	258

6.65	DC_25A_n41A	258

6.65.1	Operating bands for DC	258

6.65.5	MSD	261

6.66	DC_26A_n41A	261

6.66.1	Operating bands for DC	261

6.66.2	Channel bandwidths per operating band for DC	262

6.66.3	Co-existence studies	262

6.66.4	∆TIB and ∆RIB values	264

6.66.5	MSD	265

6.66.6	Self-interference analysis	266

6.67	DC_41A_n41A, DC_(n)41AA	267

6.67.6	SEM for DC_41A_n41A	270

6.67.6a	SEM for DC_(n)41AA	270

6.67.8	Bandwidth Combination Sets	270

6.68	DC_66A_n71A	271

6.68.6	Self-interference analysis	274

6.69	DC_(n)71B	275

6.69.1	Operating bands for DC	275

6.69.2	Channel bandwidths per operating band for DC	275

6.69.3	Co-existence studies	275

6.69.4	∆TIB and ∆RIB values	275

6.69.5	MSD	276

6.69.6	SEM for DC_(n)71B	276

6.69.7	A-MPR	276

6.70	DC_20A-n8A	278

6.70.1	Operating bands for DC	278

6.70.2	Channel bandwidths per operating band for DC	278

6.70.3	Co-existence studies	278

6.70.4	∆TIB and ∆RIB values	280

6.70.5	MSD	281

6.70.6	Self-interference analysis	281

6.71	DC_46A_n78A	282

6.71.1	Operating bands for DC	282

6.71.2	Channel bandwidths per operating band for DC	282

6.71.3	Co-existence studies	282

6.71.4	∆TIB and ∆RIB values	282

6.71.5	MSD	282

6.72	DC_1A-n50A	283

6.72.3	Co-existence studies	283

6.72.4	∆TIB and ∆RIB values	286

6.72.5	MSD	286

6.73	DC_1A-n51A	287

6.73.2	Channel bandwidths per operating band for DC	287

6.73.3	Co-existence studies	287

6.73.4	∆TIB and ∆RIB values	290

6.73.5	MSD	290

6.74	DC_2A-n71A	291

6.74.2	Channel bandwidths per operating band for DC	291

6.74.3	Co-existence studies	291

6.74.4	∆TIB and ∆RIB values	293

6.74.5	MSD	294

6.74.5.1	Band 2 MSD	294

6.74.5.2	Band n71 MSD	295

6.74.6	Self-interference analysis	296

6.75	DC_3A-n50A	296

6.75.2	Void	296

6.75.3	Void	297

6.75.4	Void	297

6.75.5	Void	297

6.76	DC_3A-n51A	297

6.76.1	Operating bands for DC	297

6.76.2	Channel bandwidths per operating band for DC	297

6.76.3	Co-existence studies	297

6.76.5	MSD	300

6.77	DC_7A-n50A	301

6.77.1	Operating bands for DC	301

6.77.2	Channel bandwidths per operating band for DC	301

6.77.3	Co-existence studies	301

6.77.4	∆TIB and ∆RIB values	304

6.77.5	MSD	304

6.78	DC_7A-n51A	305

6.78.1	Operating bands for DC	305

6.78.2	Channel bandwidths per operating band for DC	305

6.78.3	Co-existence studies	305

6.78.4	∆TIB and ∆RIB values	307

6.78.5	MSD	308

6.79	DC_20A-n50A	308

6.79.1	Operating bands for DC	308

6.79.2	Channel bandwidths per operating band for DC	308

6.79.3	Co-existence studies	308

6.79.4	∆TIB and ∆RIB values	311

6.79.5	MSD	311

6.80	DC_20A-n51A	311

6.80.1	Operating bands for DC	311

6.80.2	Channel bandwidths per operating band for DC	312

6.80.3	Co-existence studies	312

6.80.4	∆TIB and ∆RIB values	314

6.80.5	MSD	315

6.81	DC_28A-n50A	315

6.81.1	Operating bands for DC	315

6.81.2	Channel bandwidths per operating band for DC	315

6.81.3	Co-existence studies	315

6.81.4	∆TIB and ∆RIB values	318

6.81.5	MSD	318

6.81.6	Self-interference analysis	319

6.82	DC_28A-n51A	320

6.82.2	Channel bandwidths per operating band for DC	320

6.82.3	Co-existence studies	320

6.82.4	∆TIB and ∆RIB values	323

6.82.5	MSD	323

6.82.6	Self-interference analysis	324

6.83	DC_42A-n50A	325

6.83.1	Operating bands for DC	325

6.83.2	Channel bandwidths per operating band for DC	325

6.83.3	Co-existence studies	325

6.83.4	∆TIB and ∆RIB values	328

6.83.5	MSD	328

6.84	DC_42A-n51A	328

6.84.1	Operating bands for DC	328

6.84.2	Channel bandwidths per operating band for DC	328

6.84.3	Co-existence studies	328

6.84.4	∆TIB and ∆RIB values	330

6.84.5	MSD	331

6.85	DC_66A_n260	331

6.85.1	Operating bands for DC	331

6.85.2	Channel bandwidths per operating band for DC	331

6.85.3	Co-existence studies	334

6.85.4	∆TIB and ∆RIB values	336

6.85.5	REFSENs requirements	336

6.86	DC_2_n257	336

6.86.1	Operating bands for DC	336

6.86.2	Channel bandwidths per operating band for DC	337

6.86.3	Co-existence studies	337

6.86.4	∆TIB and ∆RIB values	340

6.86.5	MSD	340

6.87	DC_13A-n257A	340

6.87.1	Operating bands for DC	340

6.87.2	Channel bandwidths per operating band for DC	340

6.87.3	Co-existence studies	340

6.87.4	∆TIB and ∆RIB values	343

6.87.5	MSD	343

6.88	DC_48A-n257A	343

6.88.1	Operating bands for DC	343

6.88.2	Channel bandwidths per operating band for DC	343

6.88.3	Co-existence studies	343

6.88.4	∆TIB and ∆RIB values	345

6.88.5	MSD	345

6.89	DC_5A-n260	346

6.89.1	Operating bands for DC	346

6.89.2	Channel bandwidths per operating band for DC	346

6.89.3	Co-existence studies	348

6.89.4	∆TIB and ∆RIB values	349

6.89.5	MSD	350

6.90	DC_13A-n260A	350

6.90.1	Operating bands for DC	350

6.90.2	Channel bandwidths per operating band for DC	350

6.90.3	Co-existence studies	350

6.90.4	∆TIB and ∆RIB values	352

6.90.5	MSD	353

6.91	DC_48A-n260A	353

6.91.1	Operating bands for DC	353

6.91.2	Channel bandwidths per operating band for DC	353

6.91.3	Co-existence studies	353

6.91.4	∆TIB and ∆RIB values	355

6.91.5	MSD	355

6.92	DC_5A-n261	356

6.92.1	Operating bands for DC	356

6.92.2	Channel bandwidths per operating band for DC	356

6.92.3	Co-existence studies	358

6.92.4	∆TIB and ∆RIB values	360

6.92.5	MSD	360

6.93	DC_66A-n261	360

6.94.1	Operating bands for DC	364

6.94.2	Channel bandwidths per operating band for DC	365

6.94.3	Co-existence studies	365

6.94.4	∆TIB and ∆RIB values	368

6.94.5	REFSENS requirements	368

6.95	DC_2A_n5A	369

6.95.1	Operating bands for DC	369

6.95.2	Channel bandwidths per operating band for DC	369

6.95.3	Co-existence studies	369

6.95.4	∆TIB and ∆RIB values	372

6.95.5	REFSENS requirements	372

6.96	DC_2A_n66A	372

6.96.1	Operating bands for DC	372

6.96.2	Channel bandwidths per operating band for DC	372

6.96.4	∆TIB and ∆RIB values	375

6.96.5	REFSENS requirements	375

6.96.6	Self-interference analysis	376

6.97	DC_2A_n78A	377

6.97.1	Operating bands for DC	377

6.97.2	Channel bandwidths per operating band for DC	377

6.97.3	Co-existence studies	377

6.97.4	∆TIB and ∆RIB values	378

6.97.5	MSD	378

6.97.6	Self-interference analysis	379

6.98	DC_2A_n260A and DC_2A_n260(2A)	380

6.98.1	Operating bands for DC	380

6.98.2	Channel bandwidths per operating band for DC	381

6.98.3	Co-existence studies	381

6.98.4	∆TIB and ∆RIB values	384

6.98.5	REFSENS requirements	384

6.99	DC_3A_n3A	384

6.99.1	Operating bands for DC	384

6.99.2	Channel bandwidths per operating band for DC	384

6.99.3	UE co-existence studies	384

6.99.4	ΔTIB and ΔRIB values	385

6.99.5	MSD, A-MPR	385

6.99.6	Bandwidth Combination Sets	385

6.99.7	delta R INBC values	385

6.100	DC_3A_n40A	387

6.100.1	Operating bands for DC	387

6.100.2	Channel bandwidths per operating band for DC	387

6.100.3	Co-existence studies	387

6.100.4	∆TIB and ∆RIB values	390

6.100.5	REFSENS requirements	390

6.101	DC_5A_n40A	391

6.101.1	Operating bands for DC	391

6.101.2	Channel bandwidths per operating band for DC	391

6.101.3	Co-existence studies	391

6.101.4	∆TIB and ∆RIB values	394

6.102	DC_5A_n66A	394

6.102.1	Operating bands for DC	394

6.102.2	Channel bandwidths per operating band for DC	394

6.102.3	Co-existence studies	395

6.102.4	∆TIB and ∆RIB values	397

6.102.5	REFSENS requirements	397

6.102.6	Self-interference analysis	398

6.103	DC_8A_n40A	399

6.103.1	Operating bands for DC	399

6.103.2	Channel bandwidths per operating band for DC	399

6.103.3	Co-existence studies	399

6.103.4	∆TIB and ∆RIB values	402

6.103.5	REFSENS requirements	402

6.104	DC_12A_n5A	403

6.104.1	Operating bands for DC	403

6.104.2	Channel bandwidths per operating band for DC	403

6.104.3	Co-existence studies	403

6.104.4	∆TIB and ∆RIB values	406

6.104.5	REFSENS requirements	406

6.105	DC_12A_n66A	407

6.105.1	Operating bands for DC	407

6.105.2	Channel bandwidths per operating band for DC	407

6.105.3	Co-existence studies	407

6.105.4	∆TIB and ∆RIB values	410

6.105.5	REFSENS requirements	410

6.105.6	Self-interference analysis	411

6.106	DC_12A_n260A	412

6.106.1	Operating bands for DC	412

6.106.2	Channel bandwidths per operating band for DC	412

6.106.3	Co-existence studies	412

6.106.4	∆TIB and ∆RIB values	414

6.106.5	REFSENS requirements	415

6.107	DC_20A-n77A	415

6.107.1	Operating bands for DC	415

6.107.2	Channel bandwidths per operating band for DC	415

6.107.3	Co-existence studies	415

6.107.4	∆TIB and ∆RIB values	417

6.107.5	MSD	418

6.107.6	Self-interference analysis	419

6.108	DC_20A_n258A	419

6.108.1	Operating bands for DC	419

6.108.2	Channel bandwidths per operating band for DC	420

6.108.3	Co-existence studies	420

6.108.4	∆TIB and ∆RIB values	421

6.108.5	MSD	421

6.109	DC_30A_n5A	421

6.109.1	Operating bands for DC	421

6.109.2	Channel bandwidths per operating band for DC	422

6.109.3	Co-existence studies	422

6.109.4	∆TIB and ∆RIB values	425

6.109.5	REFSENS requirements	425

6.110	DC_30A_n66A	426

6.110.1	Operating bands for DC	426

6.110.2	Channel bandwidths per operating band for DC	426

6.110.3	Co-existence studies	426

6.110.4	∆TIB and ∆RIB values	429

6.110.5	REFSENS requirements	429

6.111	DC_30A_n260A	429

6.111.1	Operating bands for DC	429

6.111.2	Channel bandwidths per operating band for DC	430

6.111.3	Co-existence studies	430

6.111.4	∆TIB and ∆RIB values	432

6.111.5	REFSENS requirements	433

6.112	DC_40A-n77A	433

6.112.1	Operating bands for DC	433

6.112.2	Channel bandwidths per operating band for DC	433

6.112.3	Co-existence studies	433

6.112.4	∆TIB and ∆RIB values	433

6.112.5	MSD	434

6.113	DC_66A_n5A	434

6.113.1	Operating bands for DC	434

6.113.2	Channel bandwidths per operating band for DC	434

6.113.3	Co-existence studies	434

6.113.4	∆TIB and ∆RIB values	437

6.113.6	Self-interference analysis	438

6.114	DC_66A_n78A	439

6.114.1	Operating bands for DC	439

6.114.2	Channel bandwidths per operating band for DC	439

6.114.3	Co-existence studies	439

6.114.4	∆TIB and ∆RIB values	439

6.114.5	MSD	440

6.114.6	Self-interference analysis	441

Annex A: Change history	442

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band under Rel-15 time frame. The purpose is to gather the relevant background information and studies in order to address Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band for the Rel-15 band combinations in Table 1-1.

Table 1-1: Release 15 DC band combination of LTE 1DL/1UL + one NR band

| DC combination | REL independent from | Single UL allowed (NOTE 1) |
| --- | --- | --- |
| DC_1A-n77A | Rel-15 | Yes |
| DC_1A-n78A | Rel-15 | No |
| DC_1A-n79A | Rel-15 | No |
| DC_3A-n77A | Rel-15 | Yes |
| DC_3A-n78A | Rel-15 | Yes |
| DC_3A-n79A | Rel-15 | No |
| DC_11A-n77A | Rel-15 | No |
| DC_11A-n78A | Rel-15 | No |
| DC_11A-n79A | Rel-15 | No |
| DC_11A-n257A | Rel-15 | No |
| DC_18A-n77A | Rel-15 | No |
| DC_18A-n78A | Rel-15 | No |
| DC_18A-n79A | Rel-15 | No |
| DC_18A-n257A | Rel-15 | No |
| DC_19A-n77A | Rel-15 | No |
| DC_19A-n78A | Rel-15 | No |
| DC_19A-n79A | Rel-15 | No |
| DC_21A-n77A | Rel-15 | No |
| DC_21A-n78A | Rel-15 | No |
| DC_21A-n79A | Rel-15 | No |
| DC_26A-n77A | Rel-15 | No |
| DC_26A-n78A | Rel-15 | No |
| DC_26A-n79A | Rel-15 | No |
| DC_26A-n257A | Rel-15 | No |
| DC_28A-n77A | Rel-15 | No |
| DC_28A-n78A | Rel-15 | No |
| DC_28A-n79A | Rel-15 | No |
| DC_41A-n257A | Rel-15 | No |
| DC_42A-n77A | Rel-15 | No |
| DC_42A-n78A | Rel-15 | No |
| DC_42A-n79A | Rel-15 | No |
| DC_20A-n28A | Rel-15 | No |
| DC_39A-n78A | Rel-15 | No |
| DC_39A-n258A | Rel-15 | No |
| DC_1A_n257 | Rel-15 | No |
| DC_3A_n257 | Rel-15 | No |
| DC_3A-n258A | Rel-15 | No |
| DC_5A-n257A | Rel-15 | No |
| DC_7A-n78 | Rel-15 | No |
| DC_7A-n258A | Rel-15 | No |
| DC_8A-n78A | Rel-15 | No |
| DC_8A-n79A | Rel-15 | No |
| DC_8A-n258A | Rel-15 | No |
| DC_19A-n257 | Rel-15 | No |
| DC_20A-n78A | Rel-15 | No |
| DC_21A-n257 | Rel-15 | No |
| DC_28A-n257 | Rel-15 | No |
| DC_28A-n258A | Rel-15 | No |
| DC_38A-n78A | Rel-15 | No |
| DC_39A-n79A | Rel-15 | No |
| DC_41A-n77A | Rel-15 | No |
| DC_41A-n78A | Rel-15 | No |
| DC_41A-n79A | Rel-15 | No |
| DC_41A-n258A | Rel-15 | No |
| DC_42A-n257 | Rel-15 | No |
| DC_1A-n28 | Rel-15 | No |
| DC_3A-n7 | Rel-15 | No |
| DC_3A-n28 | Rel-15 | No |
| DC_5A-n78A | Rel-15 | No |
| DC_7A_n28 | Rel-15 | No |
| DC_7A-n257A | Rel-15 | No |
| DC_66A-n257A | Rel-15 | No |
| DC_8A_n77A | Rel-15 | No |
| DC_8A_n257A | Rel-15 | No |
| DC_25A_n41A | Rel-15 | No |
| DC_26A_n41A | Rel-15 | No |
| DC_41A_n41A | Rel-15 | No |
| DC_66A_n71A | Rel-15 | No |
| DC_(n)71B | Rel-15 | No |
| DC_20A_n8A | Rel-15 | No |
| DC_46A_n78A | Rel-15 | No |
| DC_1A_n50A | Rel-15 | No |
| DC_1A_n51A | Rel-15 | No |
| DC_2A_n71A | Rel-15 | No |
| DC_3A_n50A | Rel-15 | No |
| DC_3A_n51A | Rel-15 | No |
| DC_7A_n50A | Rel-15 | No |
| DC_7A_n51A | Rel-15 | No |
| DC_20A_n50A | Rel-15 | No |
| DC_20A_n51A | Rel-15 | No |
| DC_28A_n50A | Rel-15 | Yes |
| DC_28A_n51A | Rel-15 | No |
| DC_42A_n50A | Rel-15 | No |
| DC_42A_n51A | Rel-15 | No |
| DC_66A_n260 | Rel-15 | No |
| DC_2A_n257A | Rel-15 | No |
| DC_13A_n257A | Rel-15 | No |
| DC_48A_n257A | Rel-15 | No |
| DC_5A_n260 | Rel-15 | No |
| DC_13A_n260A | Rel-15 | No |
| DC_48A_n260A | Rel-15 | No |
| DC_(n)41AA | Rel-15 | No |
| DC_5A_n261 | Rel-15 | No |
| DC_66A_n261 | Rel-15 | No |
| DC_1A_n40A | Rel-15 | No |
| DC_2A_n66A | Rel-15 | No |
| DC_2A_n78A | Rel-15 | No |
| DC_2A_n260A | Rel-15 | No |
| DC_3A_n3A(NOTE 2) | Rel-15 | Yes |
| DC_3A_n40A | Rel-15 | No |
| DC_5A_n40A | Rel-15 | No |
| DC_5A_n66A | Rel-15 | No |
| DC_8A_n40A | Rel-15 | No |
| DC_12A_n260A | Rel-15 | No |
| DC_12A_n5A | Rel-15 | No |
| DC_20A_n77A | Rel-15 | No |
| DC_20A_n258A | Rel-15 | No |
| DC_30A_n66A | Rel-15 | No |
| DC_30A_n260A | Rel-15 | No |
| DC_40A_n77A | Rel-15 | No |
| DC_66A_n5A | Rel-15 | No |
| DC_66A_n78A | Rel-15 | No |

NOTE 1:	Details on how to support it can be found in Section 5.2.

NOTE 2:	only single tx switched UL mode is supported in Rel.15.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radiose".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable NR operating band

CC	Component Carriers

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

NR	New RAT

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

# 4	Background

The present document is a technical report for Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band under Rel-15 time frame. It covers both the UE and BS side. The document is divided in two different parts:

-	General part: this part covers BS and UE specific which is band combination independent.

-	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	DC band combination of LTE 1DL/1UL + one NR band: General Part

## 5.1	UE RF General

## 5.1.1	Basic DC UE RF architectures at range 1

In a few previous RAN4 meeting, some key features in NR UE 3.5GHz were agreed to support at 3.5GHz NR new bands as below.

-	Specify both PC2 and PC3 UE for 3.5GHz NR frequency range in Rel-15

-	Shall support 4Rx requirements

-	UL 256QAM supporting (w/ QPSK,16QAM, 64QAM)

-	Specify 2Tx (for diversity and UL-MIMO) requirements

Also, current 5G NR UE should be support some general features as below.

-	Wideband operation in NR band

-	NSA UE support LTE (xDL/1UL) + NR (1CC)

From the above observations, we designed a example RF architecture for NSA DC UE.

Figure 5.1.1-1. NR UE RF architecture w/ sharing antennas at Range 1

Figure 5.1.1-1 shows the common antenna sharing NR UE RF architectures for LTE and 5G NR system to support UL-MIMO and 4Rx diversity. In this figure, the main Tx/Rx antenna can use for both LTE and NR system.

The Figure 5.1.1-1 is used for NSA UE to support LTE and NR new bands except some overlapped frequency range at 3.3~4.2 GHz.

The following Figure 5.1.1-2 described the NR UE RF architecture considering separate antenna between LTE system and NR bands at sub-6GHz.

Figure 5.1.1-2. NR UE RF architecture w/ separate antennas at Range 1

The separate antenna RF architecture in figure 5.1.1-2 can be used for NR NSA UE to support LTE and 3.5GHz NR new bands since the LTE band 22, band 42 and 43 is overlapped with 3.5GHz NR new bands. In case, it is quite difficult to divide NR signal and LTE signal in UE side when we consider antenna sharing RF architecture.

Hence this architecture in Figure 5.1.1-2 also used some specific band combinations for NR DC UE.

### 5.1.2	Basic DC UE RF architectures at range 2

Based on the current assumptions at mmWave UE, we designed the RF architectures

-	At least 4Tx antenna to guarantee spherical coverage

-	Use multiple ant. elements with beamforming to get more directive gain

-	Consider both single polarization and dual polarization gain

-	Consider 2Rx or 4Rx diversity at mmWave

Also, current 5G NR UE should be support some general features as below.

-	Wideband operation in NR band

-	NSA UE support LTE (xDL/1UL) + NR (1CC)

From the above observations, we designed a example DC UE RF architecture between LTE band and NR new band at mmWave.

Figure 5.1.2-1. NR UE RF architecture w/ separate antennas at Range 2

Figure 5.1.2-1 shows the NR UE RF architectures with separate antenna structure for NSA DC UE between LTE bands and NR new bands at mmWave.

When NR UE support dual polarization antennas, we need twice antennas and RFIC at mmWave in a example RF architecture.

## 5.2	Single Tx Switched UL

In some LTE-NR inter-band combinations, as indicated in the “Difficult for simultaneous UL” column in Table 1-1, it is difficult for UE to perform simultaneous LTE+NR UL transmission. The band combinations in which the 2nd order IMD of the simultaneous UL bands fall into its own Rx band are categorized as difficult. And the band combinations for L-L (<1GHz) and H-H (1.695GHz-2.695GHz) in which the 3rd order IMD of the simultaneous UL bands fall into its own Rx band are categorized as difficult. For non-difficult band combinations, simultaneous UL transmission is mandatory.

The following formulas define which channel combinations within difficult band combinations are not affected by IM2 and IM3. Simultaneous UL transmission is mandatory for the unaffected channel combinations in difficult band combinations.

Interference bandwidth: IBW = |a| x CBW1 + |b| x CBW2

|a| + |b| = 2 (or 3)

CBW1 and CBW2 are UL channels

Center frequency of IBW:  fIBW = a x f1 + b x f2

f1 and f2 are center frequency of each UL channel

The range of IMD 2 (or 3):  [fIBW – IBW/2, fIBW + IBW/2]

Note1: The DL channel and UL channels are the transmission bandwidth, on which UE shall be able to apply operations which are configured by RRC reconfiguration and corresponding HARQ timing.

Note 2: For identified difficult band combination, during two adjacent RRC reconfiguration, the changing of transmission bandwidth should not introduce IM2 and IM3, which will result in UE changing from 2Tx to 1Tx. Otherwise, UE behavior is not specified.

For DC_3A_n3A intra-band non-contiguous EN-DC combination, only single tx switched UL mode is supported in this release.

## 5.3	Harmonics and IMD study

For each inter-band DC band combination, the harmonic and IMD products can be calculated by the formula shown in Table 5.3-1. For 2UL inter-band DC, up to 3rd harmonics and up to 5th IMD shall be considered in the analysis.

Table 5.3-1: Harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |

Instead, a concise table in Table 5.3-2 shall be provided for each DC band combination.

Table 5.3-2: Harmonics and IMD products

| NR DC Configuration | E-UTRA/NR band | DL frequency range (MHz) | Rx Impact | Harmonic/IMD orders | Notes |
| --- | --- | --- | --- | --- | --- |
| DC_xA-nyA | x | xxxx-xxxx | Yes/No | 2nd Harmonic, IMD2 |  |
|  | ny | xxxx-xxxx | Yes/No | 4th Harmonic, IMD4 |  |

When 2UL inter-band EN-DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems.

Table 5.3-3: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Harmonic/IMD |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 |  | US |  |
|  | 5150 | - | 5350 |  | Europe |  |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 |  | Asia |  |

Table 5.3-4 specifies the requirements for the specified DC configurations for coexistence with protected bands.

Table 5.3-4: Spurious emissions for uplink inter-band EN-DC (two bands)

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_xA-yA | E-UTRA Band xx, yy, zz NR band aa, bb, cc | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE1: The protected E-UTRA/NR bands are the common set of the protected bands for the constituted bands. |  |  |  |  |  |  |  |

## 5.3	Minimum requirements

The following text is taken from 38.101-1 with table numbers modified. The text also has been modified by adding the a line to table 5.3-2 for -25 dBm for 1 GHz to 12.75 GHz when NS_04 is signalled for EN-DC combinaitons with n41 signalled. NS_04 signalling will only be used in the US, and in the US the spurious emission limit is -25 dBm/MHz, not -30 dBm/MHz. When NS_04 is not signalled, EN-DC combinaitons with n41will be required to meet the -30 dBm/MHz spurious emission requirement. Also, Table 5.3-1 is modified as Table 5.3-1 to be able to handle additional NR BWs.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 5.3-1 from the edge of the channel bandwidth. The spurious emission limits in Table 5.3-2 apply for all transmitter band configurations (NRB) and channel bandwidths.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 5.3-1: Boundary between E-UTRA out of band and spurious emission domain

| Channel bandwidth | 1.4 MHz | 3.0 MHz | X MHz |
| --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 2.8 | 6 | X+5 |

Table 5.3-2: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 1 GHz f < 12.75 GHz | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26GHz | -30dBm | 1MHz | 2 |
| NOTE 1: Applies for Band 22, Band 42, Band 43 and Band 48. NOTE 2: Applies for Band 46 and Band 47. NOTE 3: Applies for EN-DC combinations that include n41 only when NS_04 is signalled. . |  |  |  |

# 6	DC band combination of LTE 1DL/1UL + one NR band: Specific Band Combination Part

## 6.1	DC_1A-n77

### 6.1.1	Operating bands for DC

Table 6.1.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.1.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-n77C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

# 6.1.3	Co-existence studies

Table 6.1.3-1 lists Band 1 +Band n77 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.3-1: Band 1 and Band n77 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 |  | 3960 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 |  | 5940 | 9900 | 12600 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 | 2280 |  | 5220 | 6180 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 |  | 660 | 4620 | 6480 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 |  | 8160 | 8520 | 10380 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1820 |  | 2080 | 3280 | 4220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 | 2640 |  | 7920 | 10680 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2640 |  | 4560 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 |  | 10140 | 11820 | 14580 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 |  | 12360 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220 |  | 14880 | 3480 | 4620 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940 |  | 8760 | 660 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 |  | 18780 | 10980 | 12120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 |  | 16560 | 12360 | 14340 |

Based on Table 6.1.3-1;

-	2nd order harmonic may fall into own Rx of band n77

-	3rd order harmonic may fall into Rx frequencies of bands 46 and 47

-	2nd order IMD may also fall into own Rx of band 1and  Rx frequencies of bands 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 45, 46, 47, 65, 66 and 70

-	3rd order IMD may also fall into Rx frequencies of bands 2, 3, 9, 22, 25, 31, 33, 34, 35, 36, 37, 39, 42, 43, 46, 47, 48, 70, n78 and n79

-	4th order IMD may also fall into own Rx of band 1 and Rx frequencies of bands 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 48, 65, 66, 69, 70, n78 and n79

-	5th order IMD may also fall into own Rx of band 1and Rx frequencies of bands 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 48, 65, 66, 67, 68, 69, 70, n78 and n79

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.3-2: 2UL Band 1 +Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 4, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 4, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4, 5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4, 5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD2, 3 |

Table 6.1.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.1.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-n77A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | RB restriction |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | RB restriction |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | RB restriction |

### 6.1.4	∆TIB and ∆RIB values

Table 6.1.4-1: Simulation results provided in R4-1707507 for Band n77 and n78

| Parameter | Frequency range | Vendor 1 |  | Vendor 2 |  | Vendor 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band n78 BPF | Band n77 BPF | Band n78 BPF | Band n77 BPF | Band n78 BPF | Band n77 BPF |
| Insertion loss (ETC) | 3300-3400 MHz | 1.1 dB | 1.1 dB | 2.0 dB | 2.0 dB | 1.9 dB | 1.9 dB |
|  | 3400-3800 MHz | 0.95 dB | 0.95 dB | 1.5 dB | 1.5 dB |  |  |
|  | 3800-4200 MHz | - | 1.2 dB | - |  | - |  |
| Attenuation (Typ) | 698-2690 MHz | 35 dB | 39.8 dB | 45 dB | 45 dB | 51.8 dB | 50.5 dB |
|  | 5150-5925 MHz | 27.8 dB | 31.1 dB | 35 dB | 30 dB | 25 dB | 25.3 dB |

For DC_1A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.1.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-n77 | 1 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: HTF is assumed for Band 1. |  |  |

Table 6.1.4-3: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: HTF is assumed for Band 1. |  |  |

### 6.1.5	MSD

Tables below list the MSD required for the dual connectivity configuration due to IMD and 2nd harmonic.

Table 6.1.5-1: MSD for the DC configuration for IMD2 and 4

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-n77A | 1 | 1950 | 5 | 25 | 2140 | [29.8] | FDD | IMD24 |
|  |  |  |  |  |  | [32.55] |  |  |
|  | n77 | 4090 | 10 | 25 | 4090 | N/A | TDD | N/A |
| DC_1A-n77A | 1 | 1950 | 5 | 25 | 2140 | [8.0] | FDD | IMD44 |
|  |  |  |  |  |  | [10.75] |  |  |
|  | n77 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE : This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

MSD values for Band n77 due to 2nd harmonic of Band 1 in LTE-NR DC_1-n77 are captured in Table 6.1.5-2.

Table 6.1.5-2: MSD due to harmonic issue for DC_1-n77

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 1 | n771,2 | 27.1 | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n773 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |

Regarding the uplink configuration, 40MHz for n77 should be considered additionally. Therefore, 20MHz with 100RB allocation for Band 1 should be added.

Table 6.5.5-3 Uplink configuration for the low band (exceptions) for E-UTRA

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | Duplex mode |
| DC_1-n77 | 1 |  |  | 12 | 26 | 39 | 531 | 1002 | FDD |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |

### 6.1.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.6-1.

Table 6.1.6-1: Band 1 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n77A | 1 | a | -1 | b | -1 | 2 | 1 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_1A_n77A | 1 | a | 3 | b | -1 | 4 | 1 | IMD |
|  | n77 | c | -1 | d | 0 |  |  |  |
| DC_1A_n77A | 1 | a | -3 | b | -1 | 5 | 1 | IMD |
|  | n77 | c | 2 | d | 0 |  |  |  |
| DC_1A_n77A | 1 | a | 2 | b | 0 | 2 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |

Equations (6.1.6.1) and (6.1.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.1.6-1 and CBW stands for channel bandwidth.

(6.1.6.1)

(6.1.6.2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.1.6.3-1)

(6.1.6.3-2)

## 6.2	DC_1A-n78

### 6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-n78C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.2.3	Co-existence studies

Table 6.2.3-1 lists Band 1 +Band n78 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.2.3-1: Band 1 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 |  | 3960 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 |  | 5940 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 | 1880 |  | 5220 | 5780 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 |  | 660 | 4620 | 5680 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 |  | 7760 | 8520 | 9580 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1820 |  | 2080 | 3280 | 3820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 | 2640 |  | 7920 | 9480 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2640 |  | 3760 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 |  | 9740 | 11820 | 13380 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 |  | 11560 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220 |  | 13280 | 3880 | 4620 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940 |  | 7560 | 660 | 1840 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 |  | 17180 | 10980 | 11720 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 |  | 15360 | 12360 | 13540 |

Based on Table 6.2.3-1;

-	2nd order harmonic may fall into Rx of band n77

-	3rd order harmonic may fall into Rx frequencies of bands 46 and 47

-	2nd order IMD may fall into Rx frequencies of bands 3, 9, 11, 21, 24, 32, 35, 45, 46

-	3rd order IMD may fall into Rx frequencies of bands 2, 3, 9, 22, 25, 31, 33, 34, 35, 36, 37, 39, 42, 43, 46, 48, 70, n77, n78 and n79.

-	4th order IMD may fall into Rx frequencies of bands 1, 2, 4, 7, 10, 22, 23, 25, 30, 33, 34, 36, 38, 40, 41, 42, 43, 48, 65, 66, 69, 70, n77 and n78

-	5th order IMD may also fall into own Rx of band 1and Rx frequencies of bands 3, 5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 67, 68, n77 and n79

It should be noted that IMD will not be an issue for band n78 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band. When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.2.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.2.3-2: 2UL Band 1 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD2, 3 |

Table 6.2.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.2.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-n78A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | RB restriction |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | RB restriction |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | RB restriction |

### 6.2.4	∆TIB and ∆RIB values

For DC_1A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-n78 | 1 | 0.3 |
|  | n78 | 0.8 |

Table 6.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-n78 | 1 | 0 |
|  | n78 | 0.5 |

### 6.2.5	MSD

Table 6.2.5-1 lists the MSD required for the dual connectivity configuration.

| Table 6.2.5-1: MSD for the DC configurationE-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-n78A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.7 5 |  |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

### 6.2.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.2.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.2.6-1.

Table 6.2.6-1: Band 1 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78A | 1 | a | 3 | b | -1 | 4 | 1 | IMD |
|  | n78 | c | -1 | d | 0 |  |  |  |

Equations (6.2.6.1) and (6.2.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.2.6-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (6.2.6.3-1) and (6.2.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

## 6.3	DC_1A-n79

### 6.3.1	Operating bands for DC

Table 6.3.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.3.2	Channel bandwidths per operating band for DC

Table 6.3.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-n79C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.3.3	Co-existence studies

Table 6.3.3-1 lists Band 1 +Band n79 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.3.3-1: Band 1 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 |  | 3960 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 |  | 5940 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2420 | 3080 |  | 6320 | 6980 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 440 |  | 1160 | 6820 | 8080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8240 |  | 8960 | 10720 | 11980 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1820 |  | 2080 | 4380 | 5020 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 760 | 1540 |  | 11220 | 13080 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 4840 |  | 6160 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10160 |  | 10940 | 15120 | 16980 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 12640 |  | 13960 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15620 |  | 18080 | 2680 | 3520 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9240 |  | 11160 | 2860 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19520 |  | 21980 | 12080 | 12920 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17040 |  | 18960 | 14560 | 15940 |

Based on Table 6.3.3-1;

-	2nd order harmonic may fall into Rx frequency of band n77 (3.3 – 4.2 GHz)

-	3rd order harmonic may fall into Rx frequencies of bands 46 and 47

-	2nd order IMD may also fall into Rx frequencies of bands 7, 38, 41 and 69

-	3rd order IMD may also fall into Rx frequencies of bands2, 3, 5, 6, 8, 9, 12, 13, 14, 17, 18, 19, 20, 25, 26, 27, 28, 29, 31, 33, 34, 35, 36, 37, 39, 44, 67, 68, 70

-	4th order IMD may also fall into Rx frequencies of bands 5, 6, 8, 11, 14, 18, 19, 20, 21, 24, 26, 27, 28, 44, 45, 46, 47, 68

-	5th order IMD may also fall into Rx frequencies of bands 7, 22, 41, 42, 43, 48, n77 (3.3 – 4.2 GHz) and n78 (3.3 –3.8 GHz)

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.3.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.3.3-2: 2UL Band 1 +Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD4 |

Table 6.3.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.3.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-n79A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | RB restriction |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | RB restriction |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | RB restriction |

### 6.3.4	∆TIB and ∆RIB values

For DC_1A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-n79 | 1 | 0 |
|  | n79 | 0 |

Table 6.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-n79 | 1 | 0 |
|  | n79 | 0 |

### 6.3.5	MSD

There is no MSD issue for this DC configuration.

## 6.4	DC_3A-n77

### 6.4.1	Operating bands for DC

Table 6.4.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.4.2	Channel bandwidths per operating band for DC

Table 6.4.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Config. | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-n77C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.4.3	Co-existence studies

Table 6.4.3-1 lists Band 3 +Band n77 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.4.3-1: Band 3 and Band n77 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3570 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5355 | 9900 | 12600 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2490 |  | 5010 | 5985 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 270 |  | 780 | 4815 | 6690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 |  | 7770 | 8310 | 10185 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 |  | 1885 | 3280 | 4220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 | 2055 |  | 8115 | 10890 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3030 |  | 4980 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 |  | 9555 | 11610 | 14385 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 |  | 11970 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415 |  | 15090 | 2640 | 3840 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 |  | 9180 | 1245 | 3270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 |  | 18585 | 10140 | 11340 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 |  | 16170 | 11730 | 13755 |

Based on Table 6.4.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, n77 and n78

-	3rd order harmonic may fall into Rx frequencies of band 46

2nd order IMD may also fall into own Rx of band 3 Rx frequencies of bands 1, 2, 4, 9, 10, 23, 24, 25, 30, 33, 34, 35, 36, 37, 39, 40, 46, 47, 65, 66 and 70

3rd order IMD may also fall into own Rx of band 3 and Rx frequencies of bands 9, 12, 13, 14, 17, 22, 28, 29, 31, 35, 39, 42, 43, 44, 46, 47, 48, 67, 68, n78 and n79

4th order IMD may also fall into own Rx of band 3 and Rx frequencies of bands 2, 8, 9, 11, 10, 21, 22, 24, 25, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 70, n78 and n79

5th order IMD may also fall into own Rx of band 3 and Rx frequencies of bands 1, 2, 4, 7, 9, 10, 11, 21, 22, 23, 24, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45, 48, 65, 66, 69, 70 and n78

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.4.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.4.3-2: 2UL Band 3 +Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 4, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 4, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2, 5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2, 5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonic3, IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD2, 3 |

Table 6.4.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.4.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-n77A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.4.4	∆TIB and ∆RIB values

For DC_3A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-n77 | 3 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: HTF is assumed for Band 3. |  |  |

Table 6.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: HTF is assumed for Band 3. |  |  |

### 6.4.5	MSD

Tables  below list the MSD required for the dual connectivity configuration due to IMD and 2nd harmonic.

Table 6.4.5-1: MSD for the DC configuration for IMD2 and 4

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-n77A | 3 | 1740 | 5 | 25 | 1835 | [26] | FDD | IMD24 |
|  |  |  |  |  |  | [28.7 5] |  |  |
|  | n77 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
| DC_3A-n77A | 3 | 1765 | 5 | 25 | 1860 | [8.0] | FDD | IMD44 |
|  |  |  |  |  |  | [10.75] |  |  |
|  | n77 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

MSD values for Band n77 due to 2nd harmonic of Band 3 in LTE-NR DC_3-n77 are captured in Table 6.4.5-2.

Table 6.4.5-2: MSD due to harmonic issue for DC_3-n77

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 3 | n771,2 | 27.1 | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n773 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |

Regarding the uplink configuration, 40MHz for n77 should be considered additionally. Therefore, 20MHz with 100RB allocation for Band 3 should be added.

Table 6.5.5-3 Uplink configuration for the low band (exceptions) for E-UTRA

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | Duplex mode |
| DC_3-n77 | 3 |  |  | 12 | 26 | 39 | 531 | 1002 | FDD |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |

### 6.4.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.4.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.4.6-1.

Table 6.4.6-1: Band 3 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n77A | 3 | a | -1 | b | -1 | 2 | 3 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_3A_n77A | 3 | a | 3 | b | -1 | 4 | 3 | IMD |
|  | n77 | c | -1 | d | 0 |  |  |  |
| DC_3A_n77A | 3 | a | -3 | b | -1 | 5 | 3 | IMD |
|  | n77 | c | 2 | d | 0 |  |  |  |
| DC_3A_n77A | 3 | a | 2 | b | 0 | 2 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |

Equations (6.4.6.1) and (6.4.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.4.6-1 and CBW stands for channel bandwidth.

(6.4.6.1)

(6.4.6.2)

Formula (6.4.6.3-1) and (6.4.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.4.6.3-1)

(6.4.6.3-2)

## 6.5	DC_3A-n78

### 6.5.1	Operating bands for DC

Table 6.5.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.5.2	Channel bandwidths per operating band for DC

Table 6.5.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Config. | E-UTRA and NR Band | Subcarrier spacing [kHz] | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-n78A | 3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |  |
|  | n78 | 15 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| DC_3A-n78C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.5.3	Co-existence studies

Table 6.5.3-1 lists Band 3 +Band n78 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.5.3-1: Band 3 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2090 | 5010 | 5585 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 270 | 380 | 4815 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7370 | 8310 | 9385 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 | 1885 | 3280 | 3820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2055 | 8115 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3030 | 4180 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9155 | 11610 | 13185 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 | 11170 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415 | 13490 | 3040 | 3840 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 | 7980 | 1245 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16985 | 10140 | 10940 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14970 | 11730 | 12955 |

Based on Table 6.5.3-1;

2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, n77 and n78

3rd order harmonic may fall into Rx frequencies of band 46

2nd order IMD may fall into Rx frequencies of bands 2, 3, 9, 24, 25, 33, 34, 35, 36, 37, 39, 46, and 70

3rd order IMD may fall into Rx frequencies of bands 3, 9, 22, 35, 39, 42, 43, 48, n77, n78 and n79

4th order IMD may fall into Rx frequencies of bands 2, 3, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 70, n77 and n78

5th order IMD may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 22, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 39, 40, 42, 43, 45, 48, 65, 66, 70, n77 and n78.

It should be noted that IMD will not be an issue for band n78 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.5.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.5.3-2: 2UL Band 3 +Band 77n harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 4, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 4, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonic3, IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD2, 3 |

Table 6.5.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.5.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-n78A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.5.4	∆TIB and ∆RIB values

For DC_3A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 3. |  |  |

Table 6.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 3. |  |  |

### 6.5.5	MSD

Tables below lists the MSD required for the dual connectivity configuration due to IMD and 2nd harmonic.

Table 6.5.5-2: MSD for the DC configuration for IMD2 and 4

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |
| DC_3A-n78A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

MSD values for Band n78 due to 2nd harmonic of Band 3 in LTE-NR DC_3-n78 are captured in Table 6.5.5-5.

Table 6.5.5-2: MSD due to harmonic issue for DC_3-n78

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 3 | n781,2 | 27.1 | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n783 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |

Regarding the uplink configuration, 40MHz for n78 should be considered additionally. Therefore, 20MHz with 100RB allocation for Band 3 should be added.

Table 6.5.5-3 Uplink configuration for the low band (exceptions) for E-UTRA

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | Duplex mode |
| DC_3-n78 | 3 |  |  | 12 | 26 | 39 | 531 | 1002 | FDD |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |

### 6.5.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.5.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.5.6-1.

Table 6.5.6-1: Band 3 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n78A | 3 | a | -1 | b | -1 | 2 | 3 | IMD |
|  | n78 | c | 1 | d | 0 |  |  |  |
| DC_3A_n78A | 3 | a | 3 | b | -1 | 4 | 3 | IMD |
|  | n78 | c | -1 | d | 0 |  |  |  |
| DC_3A_n78A | 3 | a | -3 | b | -1 | 5 | 3 | IMD |
|  | n78 | c | 2 | d | 0 |  |  |  |
| DC_3A_n78A | 3 | a | 2 | b | 0 | 2 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (6.5.6.1) and (6.5.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.5.6-1 and CBW stands for channel bandwidth.

(6.5.6.1)

(6.5.6.2)

Formula (6.5.6.3-1) and (6.5.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.5.6.3-1)

(6.5.6.3-2)

## 6.6	DC_3A-n79

# 6.6.1	Operating bands for DC

Table 6.6.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.6.2	Channel bandwidths per operating band for DC

Table 6.6.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-n79C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.6.3	Co-existence studies

Table 6.6.3-1 lists Band 3 +Band n79 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.6.3-1: Band 3 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3570 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5355 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2615 | 3290 |  | 6110 | 6785 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 830 |  | 1580 | 7015 | 8290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7820 |  | 8570 | 10510 | 11785 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 |  | 1885 | 4380 | 5020 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 130 | 955 |  | 11415 | 13290 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 5230 |  | 6580 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9530 |  | 10355 | 14910 | 16785 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 12220 |  | 13570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15815 |  | 18290 | 1840 | 2740 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9630 |  | 11580 | 3445 | 4870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19310 |  | 21785 | 11240 | 12140 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 16620 |  | 18570 | 13930 | 15355 |

Based on Table 6.6.3-1;

2nd order harmonic may fall into Rx frequency of bands 22, 42, 48, n77 (3.3 – 4.2 GHz) and n78 (3.3 – 3.8 GHz)

3rd order harmonic may fall into Rx frequencies of bands 46

2nd order IMD may also fall into Rx frequencies of bands 7, 38, 41 and 69

3rd order IMD may also fall into own Rx of band 3 and Rx frequencies of bands 5, 6, 8, 9, 11, 18, 19, 21, 24, 26, 27, 35, 39, 45

4th order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 47, 67, 68

5th order IMD may also fall into own Rx of band 3 and Rx frequencies of bands 1, 2, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 48, 65, 66, 69, 70 and n77 (3.3 – 4.2 GHz)

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.6.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.6.3-2: 2UL Band 3 +Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonic3, IMD4 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD4 |

Table 6.6.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.6.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-n79A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.6.4	∆TIB and ∆RIB values

For DC_3A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-n79 | 3 | 0 |
|  | n79 | 0 |

Table 6.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-n79 | 3 | 0 |
|  | n79 | 0 |

### 6.6.5	MSD

There is no MSD issue for this DC configuration.

## 6.7	DC_11A-n77A

### 6.7.1	Operating bands for DC

Table 6.7.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11A-n77A | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.7.2	Channel bandwidths per operating band for DC

Table 6.7.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_11A-n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  | 110 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.7.3	Co-existence studies

Table 6.7.3-1 lists Band11+3.3-4.2GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.7.3-1: Band 11 and Band n77 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 9900 | 12600 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1852.1 | 2772.1 | 4727.9 | 5647.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1344.2 | 404.2 | 5152.1 | 6972.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6155.8 | 7095.8 | 8027.9 | 9847.9 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| IMD frequency limits (MHz) | 1327.9 | 1547.9 | 3280 | 4220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 83.7 | 1043.7 | 8452.1 | 11172.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3704.2 | 5544.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7583.7 | 8543.7 | 11327.9 | 14047.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 9455.8 | 11295.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15372.1 | 11752.1 | 2491.6 | 1511.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9744.2 | 7004.2 | 2256.3 | 4116.3 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14627.9 | 18247.9 | 9011.6 | 9991.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12755.8 | 15495.8 | 10883.7 | 12743.7 |

Based on the table, 3rd order IMD may fall into own Rx of band 11 and  3rd, 4th and 5th order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.7.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.7.3-2: Band 11 and Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD5 |
| Galileo | 1559 | - | 1591 | YES |  | IMD5 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD5 |
| GPS | 1563 | - | 1587 | YES |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | IMD5 |
|  | 2400 | - | 2494 | YES | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | IMD2, IMD3, IMD4 |
|  | 5150 | - | 5350 | YES | Europe | IMD2, IMD3, IMD4 |
|  | 5470 | - | 5725 | YES |  | IMD2, IMD3, IMD4 |
|  | 5150 | - | 5825 | YES | Asia | IMD2, IMD3, IMD4 |

Table 6.7.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.7.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11A-n77A | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.7.4	∆TIB and ∆RIB values

For DC_11A-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11A-n77A | 11 | 0.4 |
|  | n77 | 0.8 |

Table 6.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11A-n77A | 11 | 0 |
|  | n77 | 0.5 |

### 6.7.5	MSD

As approved in R4-1710977, no MSD due to IMD and harmonic should be specified in this combination and MSD requirement is shown in table 6.7.5-1.

Table 6.7.5-1: MSD requirement for dual uplink DC_11A-n77A

| E-UTRA+NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA+NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| DC_11A-n77A | 11 | N/A | N/A | N/A | N/A | N/A | FDD |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD |

## 6.8	DC_11A-n78A

### 6.8.1	Operating bands for DC

Table 6.8.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11A-n78A | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.8.2	Channel bandwidths per operating band for DC

Table 6.8.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11A-n78A | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.8.3	Co-existence studies

Table 6.8.3-1 lists Band11+3.3-3.8GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.8.3-1: Band 11 and Band n78 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1852.1 | 2372.1 | 4727.9 | 5247.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 944.2 | 404.2 | 5152.1 | 6172.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6155.8 | 6695.8 | 8027.9 | 9047.9 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| IMD frequency limits (MHz) | 1327.9 | 1547.9 | 3280 | 3820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 483.7 | 1043.7 | 8452.1 | 9972.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3704.2 | 4744.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7583.7 | 8143.7 | 11327.9 | 12847.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 9455.8 | 10495.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13772.1 | 11752.1 | 2491.6 | 1911.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8544.2 | 7004.2 | 2256.3 | 3316.3 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14627.9 | 16647.9 | 9011.6 | 9591.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12755.8 | 14295.8 | 10883.7 | 11943.7 |

Based on the table, 3rd order IMD may fall into own Rx of band 11, and  3rd, 4th and 5th order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.8.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.8.3-2: Band 11 and Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | IMD2, IMD3 |
|  | 5150 | - | 5350 | YES | Europe | IMD2, IMD3 |
|  | 5470 | - | 5725 | YES |  | IMD3 |
|  | 5150 | - | 5825 | YES | Asia | IMD2, IMD3 |

Table 6.8.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.8.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11A-n78A | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.8.4	∆TIB and ∆RIB values

For DC_11A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11A-n78A | 11 | 0.4 |
|  | n78 | 0.8 |

Table 6.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11A-n78A | 11 | 0 |
|  | n78 | 0.5 |

### 6.8.5	MSD

As approved in R4-1710977, no MSD due to IMD and harmonic should be specified in this combination and MSD requirement is shown in table 6.8.5-1.

Table 6.8.5-1: MSD requirement for dual uplink DC_11A-n78A

| E-UTRA+NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA+NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| DC_11A-n78A | 11 | N/A | N/A | N/A | N/A | N/A | FDD |
|  | n78 | N/A | N/A | N/A | N/A | N/A | TDD |

## 6.9	DC_11A-n79A

### 6.9.1	Operating bands for DC

Table 6.9.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11A-n79A | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.9.2	Channel bandwidths per operating band for DC

Table 6.9.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_11A-n79A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  | 110 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.9.3	Co-existence studies

Table 6.9.3-1 lists Band11+4.4-5GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.9.3-1: Band 11 and Band n79 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2952.1 | 3572.1 | 5827.9 | 6447.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2144.2 | 1504.2 | 7352.1 | 8572.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7255.8 | 7895.8 | 10227.9 | 11447.9 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦f1_low –f2_BWmax¦ | ¦f1_high + f2_BW max¦ | ¦f2_low – f1_BW max¦ | ¦f2_high + f1_BW max¦ |
| IMD frequency limits (MHz) | 1327.9 | 1547.9 | 4380 | 5020 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 716.3 | 56.3 | 11752.1 | 13572.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5904.2 | 7144.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8683.7 | 9343.7 | 14627.9 | 16447.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11655.8 | 12895.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 18572.1 | 16152.1 | 1391.6 | 711.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 12144.2 | 10304.2 | 4456.3 | 5716.3 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19027.9 | 21447.9 | 10111.6 | 10791.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 16055.8 | 17895.8 | 13083.7 | 14343.7 |

Based on the table, 3rd order IMD may fall into own Rx of band 11, and  3rd and 5th order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.9.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.9.3-2: Band 11 and Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | IMD4, IMD5 |
|  | 5150 | - | 5350 | YES | Europe | IMD5 |
|  | 5470 | - | 5725 | YES |  | IMD5 |
|  | 5150 | - | 5825 | YES | Asia | IMD5 |

Table 6.9.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.9.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11A-n79A | E-UTRA Band 1, 3, 18, 19, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.9.4	∆TIB and ∆RIB values

For DC_11A-n79A, the TIB,c and RIB values are given in the tables below.

Table 6.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11A-n79A | 11 | 0 |
|  | n79 | 0 |

Table 6.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11A-n79A | 11 | 0 |
|  | n79 | 0 |

### 6.9.5	MSD

As approved in R4-1710977, no MSD due to IMD and harmonic should be specified in this combination and MSD requirement is shown in table 6.9.5-1.

Table 6.9.5-1: MSD requirement for dual uplink DC_11A-n79A

| E-UTRA+NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA+NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| DC_11A-n79A | 11 | N/A | N/A | N/A | N/A | N/A | FDD |
|  | n79 | N/A | N/A | N/A | N/A | N/A | TDD |

### 6.9.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.9.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.9.6-1.

Table 6.9.6-1: Band 11 and Band n79 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_11A_n79A | 11 | a | 0 | b | 3 | 3 | 11 | Harmonic mixing |
|  | n79 | c | -1 | d | 0 |  |  |  |

Equations (6.9.6.1) and (6.9.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.9.6-1 and CBW stands for channel bandwidth.

(6.9.6.1)

(6.9.6.2)

Formula (6.9.6.3-1) and (6.9.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.9.6.3-1)

(6.9.6.3-2)

## 6.10	DC_11A-n257A

### 6.10.1	Operating bands for DC

Table 6.10.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11A_n257 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.10.2	Channel bandwidths per operating band for DC

Table 6.10.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_11A_n257 | 11 | 15 | Yes | Yes |  |  |  |  |  |  | 410 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.10.3	Co-existence studies

Table 6.10.3-1 lists Band11+26.5-29.5GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.10.3-1: Band 11 and Band n257 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25052.1 | 28072.1 | 27927.9 | 30947.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26644.2 | 23604.2 | 51552.1 | 57572.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29355.8 | 32395.8 | 54427.9 | 60447.9 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| IMD frequency limits (MHz) | 1027.9 | 1847.9 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 25216.3 | 22156.3 | 78052.1 | 87072.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 50104.2 | 56144.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 30783.7 | 33843.7 | 80927.9 | 89947.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 55855.8 | 61895.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116572.1 | 104552.1 | 20708.4 | 23788.4 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 85644.2 | 76604.2 | 48656.3 | 54716.3 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107427.9 | 119447.9 | 32211.6 | 35291.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82355.8 | 91395.8 | 57283.7 | 63343.7 |

Based on the table, 3rd order IMD may fall into own Rx of band 11, and  2nd and 3rd order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.10.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.10.3-2: Band 11 and Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 |  | US |  |
|  | 5150 | - | 5350 |  | Europe |  |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 |  | Asia |  |

Table 6.10.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.10.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11A-n257A | E-UTRA Band 1, 3, 18, 19, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

### 6.10.4	∆TIB and ∆RIB values

For DC_11A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11A_n257 | 11 | 0 |
|  | n257 | 0 |

Table 6.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11A_n257 | 11 | 0 |
|  | n257 | 0 |

### 6.10.5	MSD

As approved in R4-1710977, no MSD due to IMD and harmonic should be specified in this combination and MSD requirement is shown in table 6.10.5-1.

Table 6.10.5-1: MSD requirement for dual uplink DC_11A-n257A

| E-UTRA+NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA+NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| DC_11A-n257A | 11 | N/A | N/A | N/A | N/A | N/A | FDD |
|  | N257 | N/A | N/A | N/A | N/A | N/A | TDD |

## 6.11	DC_18A-n77A

### 6.11.1	Operating bands for DC

Table 6.11.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-n77A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.11.2	Channel bandwidths per operating band for DC

Table 6.11.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_18A-n77A | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.11.3	Co-existence studies

Table 6.11.3-1 lists Band18+3.3-4.2GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.11.3-1: Band 18 and Band n77 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3260 | 3320 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4075 | 4150 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2470 | 3385 | 4115 | 5030 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2570 | 1640 | 5770 | 7585 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4930 | 5860 | 7415 | 9230 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| IMD frequency limits (MHz) | 715 | 930 | 3280 | 4220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1755 | 810 | 9070 | 11785 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4940 | 6770 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5745 | 6690 | 10715 | 13430 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8230 | 10060 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15985 | 12370 | 20 | 940 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10970 | 8240 | 4110 | 5955 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14015 | 17630 | 6560 | 7520 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11530 | 14260 | 9045 | 10890 |

Based on the table, 4th, 5th order harmonic and 3rd, 4th, 5th order IMD may fall into own Rx of band 18, and  2nd, 3rd and 5th order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.11.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.11.3-2: Band 18 and Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD4 |
| Galileo | 1559 | - | 1591 | YES |  | IMD4 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD4 |
| GPS | 1563 | - | 1587 | YES |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd harmonic, IMD2, IMD3 |
|  | 2400 | - | 2494 | YES | Asia | 3rd harmonic, IMD2, IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | YES | Europe | IMD3, IMD4, IMD5 |
|  | 5470 | - | 5725 | YES |  | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5825 | YES | Asia | IMD3, IMD4, IMD5 |

Table 6.11.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.11.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_18A-n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.11.4	∆TIB and ∆RIB values

For DC_18A-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18A-n77A | 18 | 0.3 |
|  | n77 | 0.8 |

Table 6.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-n77A | 18 | 0 |
|  | n77 | 0.5 |

### 6.11.5	MSD

Although table 6.11.3-1 shows 3rd, 4th and 5th order IMD may fall into own Rx of band 18, only impacts from 5th IMD should be studied as approved in R4-1709965. In R4-1709964, it is approved that 5th IMD of DC_19A-n77A need not to be specified. Since band18 has similar frequency range as band 19 that results in the same IMD level, no IMD issue should be studied for own Rx range in this combination and MSD requirement due to IMD is shown in table 6.11.5-1.

Table 6.11.5-1: MSD due to IMD requirement for dual uplink DC_18A-n77A

| E-UTRA+NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA+NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| DC_18A-n77A1 | 18 | N/A | N/A | N/A | N/A | N/A | FDD |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD |
| NOTE1: The reference sensitivity should only be verified when all configured RE’s of n77 uplink are confined within frequencies 3600 – 4200 MHz. Requirement of other uplink frequency ranges is FFS. |  |  |  |  |  |  |  |

Table 6.11.5-2 lists the MSD due to 4th harmonic required for the dual connectivity configuration.

Table 6.11.5-2: MSD due to 5th harmonic for DC_18A-n77A

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | n774,5 |  | [10.4] | [8.9] | [7.8] |  | [4.7] | [3.7] | [3] | [1.7] | [0.7] |

Table 6.11.5-3 lists the UL configuration for the EN-DC configuration DC_18A-n77A.

Table 6.11.5-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | n77 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |  |  |

### 6.11.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD) , or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.11.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.11.6-1.

Table 6.11.6-1: Band 18 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n77A | 18 | a | -3 | b | -1 | 4 | 18 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_18A_n77A | 18 | a | -4 | b | -1 | 5 | 18 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_18A_n77A | 18 | a | 4 | b | 0 | 4 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |
| DC_18A_n77A | 18 | a | 5 | b | 0 | 5 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |

Equations (6.11.6.1) and (6.11.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.11.6-1 and CBW stands for channel bandwidth.

(6.11.6.1)

(6.11.6.2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.11.6.3-1)

(6.11.6.3-2)

## 6.12	DC_18A-n78A

### 6.12.1	Operating bands for DC

Table 6.12.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-n78A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.12.2	Channel bandwidths per operating band for DC

Table 6.12.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_18A-n78A | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.12.3	Co-existence studies

Table 6.12.3-1 lists Band18+3.3-3.8GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.12.3-1: Band 18 and Band n78 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3260 | 3320 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4075 | 4150 | 16500 | 19000 |
| IMD frequency limits (MHz) | 2470 | 2985 | 4115 | 4630 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2170 | 1640 | 5770 | 6785 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4930 | 5460 | 7415 | 8430 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| IMD frequency limits (MHz) | 715 | 930 | 3280 | 3820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1355 | 810 | 9070 | 10585 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4940 | 5970 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5745 | 6290 | 10715 | 12230 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8230 | 9260 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14385 | 12370 | 20 | 540 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9770 | 8240 | 4110 | 5155 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14015 | 16030 | 6560 | 7120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11530 | 13060 | 9045 | 10090 |

Based on the table, 4th harmonic and 3rd, 4th order IMD may fall into own Rx of band 18 and  3rd order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.12.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.12.3-2: Band 18 and Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd harmonic, IMD2 |
|  | 2400 | - | 2494 | YES | Asia | 3rd harmonic, IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | YES | Europe | IMD3, IMD4, IMD5 |
|  | 5470 | - | 5725 | YES |  | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5825 | YES | Asia | IMD3, IMD4, IMD5 |

Table 6.12.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.12.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_18A-n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.12.4	∆TIB and ∆RIB values

For DC_18A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18A-n78A | 18 | 0.3 |
|  | n78 | 0.8 |

Table 6.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18A-n78A | 18 | 0 |
|  | n78 | 0.5 |

### 6.12.5	MSD

Although table 6.12.3-1 shows  3rd and 4th order IMD may fall into own Rx of band 18, there is no IMD issue for own Rx range in this combination taking actual spectrum holding into account as approved in R4-1709965 and the MSD requirement due to IMD is shown in table 6.12.5-1.

Table 6.12.5-1: MSD due to IMD requirement for dual uplink DC_18A-n78A

| E-UTRA+NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA+NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| DC_18A-n78A1 | 18 | N/A | N/A | N/A | N/A | N/A | FDD |
|  | n78 | N/A | N/A | N/A | N/A | N/A | TDD |
| NOTE1: The reference sensitivity should only be verified when all configured RE’s of n78 uplink are confined within frequencies 3600 – 3800 MHz. Requirement of other uplink frequency ranges is FFS. |  |  |  |  |  |  |  |

For 4th order harmonic, no MSD should be specified considering spectrum allocation.

### 6.12.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.12.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.12.6-1.

Table 6.12.6-1: Band 18 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n78A | 18 | a | -3 | b | -1 | 4 | 18 | IMD |
|  | n78 | c | 1 | d | 0 |  |  |  |
| DC_18A_n78A | 18 | a | 4 | b | 0 | 4 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (6.12.6.1) and (6.12.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.12.6-1 and CBW stands for channel bandwidth.

(6.12.6.1)

(6.12.6.2)

Formula (6.12.6.3-1) and (6.12.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.12.6.3-1)

(6.12.6.3-2)

## 6.13	DC_18A-n79A

### 6.13.1	Operating bands for DC

Table 6.13.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-n79A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.13.2	Channel bandwidths per operating band for DC

Table 6.13.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_18A-n79A | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.13.3	Co-existence studies

Table 6.13.3-1 lists Band18+4.4-5GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.13.3-1: Band 18 and Band n79 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3570 | 4185 | 5215 | 5830 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3370 | 2740 | 7970 | 9185 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6030 | 6660 | 9615 | 10830 |
| Two-tone 3rd order IMD products (withmaximum channel bandwidth) | ¦f1_low –f2_BWmax¦ | ¦f1_high + f2_BW max¦ | ¦f2_low – f1_BW max¦ | ¦f2_high + f1_BW max¦ |
| IMD frequency limits (MHz) | 715 | 930 | 4380 | 5020 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2555 | 1910 | 12370 | 14185 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7140 | 8370 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6845 | 7490 | 14015 | 15830 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10430 | 11660 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 19185 | 16770 | 1080 | 1740 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 13370 | 11540 | 6310 | 7555 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18415 | 20830 | 7660 | 8320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14830 | 16660 | 11245 | 12490 |

Based on the table, 3rd order IMD may fall into own Rx of band 18, and  3rd order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.13.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.13.3-2: Band 18 and Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD5 |
| Galileo | 1559 | - | 1591 | YES |  | IMD5 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD5 |
| GPS | 1563 | - | 1587 | YES |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | IMD2 |
|  | 5150 | - | 5350 | YES | Europe | IMD2 |
|  | 5470 | - | 5725 | YES |  | IMD2 |
|  | 5150 | - | 5825 | YES | Asia | IMD2 |

Table 6.13.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.13.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_18A-n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.13.4	∆TIB and ∆RIB values

For DC_18A-n79A, the TIB,c and RIB values are given in the tables below.

Table 6.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18A-n79A | 18 | 0 |
|  | n79 | 0 |

Table 6.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18A-n79A | 18 | 0 |
|  | n79 | 0 |

### 6.13.5	MSD

There is no MSD issue for this DC configuration

### 6.13.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.13.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.13.6-1.

Table 6.13.6-1: Band 18 and Band n79 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n79A | 18 | a | 6 | b | 0 | 6 | n79 | Harmonic |
|  | n79 | c | 0 | d | -1 |  |  |  |

Equations (6.13.6.1) and (6.13.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.13.6-1 and CBW stands for channel bandwidth.

(6.13.6.1)

(6.13.6.2)

Formula (6.13.6.3-1) and (6.13.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.13.6.3-1)

(6.13.6.3-2)

## 6.14	DC_18A-n257A

### 6.14.1	Operating bands for DC

Table 6.14.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n257 | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.14.2	Channel bandwidths per operating band for DC

Table 6.14.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n257 | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  | 415 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.14.3	Co-existence studies

Table 6.14.3-1 lists Band18+26.5-29.5GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.14.3-1: Band 18 and Band n257 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25670 | 28685 | 27315 | 30330 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27870 | 24840 | 52170 | 58185 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28130 | 31160 | 53815 | 59830 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| IMD frequency limits (MHz) | 415 | 1230 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27055 | 24010 | 78670 | 87685 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 51340 | 57370 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28945 | 31990 | 80315 | 89330 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54630 | 60660 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117185 | 105170 | 23180 | 26240 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 86870 | 77840 | 50510 | 56555 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106815 | 118830 | 29760 | 32820 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81130 | 90160 | 55445 | 61490 |

Based on the table, 3rd order IMD may fall into own Rx of band 18, and  2nd, 3rd and 4th order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.14.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.14.3-2: Band 18 and Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 |  | US |  |
|  | 5150 | - | 5350 |  | Europe |  |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 |  | Asia |  |

Table 6.14.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.14.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_18A-n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

### 6.14.4	∆TIB and ∆RIB values

For DC_18A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18A_n257 | 18 | 0 |
|  | n257 | 0 |

Table 6.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18A_n257 | 18 | 0 |
|  | n257 | 0 |

### 6.14.5	MSD

There is no MSD issue for this DC configuration.

## 6.15	DC_19A-n77

### 6.15.1	Operating bands for DC

Table 6.15.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.15.2	Channel bandwidths per operating band for DC

Table 6.15.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-n77A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-n77C | 3 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 215 |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.15.3	Co-existence studies

Table 6.15.3-1 lists Band 19 +Band n77 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.15.3-1: Band 19 and Band n77 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 |  | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 |  | 1690 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 |  | 2535 | 9900 | 12600 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2455 | 3370 |  | 4130 | 5045 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1610 |  | 2540 | 5755 | 7570 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4960 |  | 5890 | 7430 | 9245 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 730 |  | 945 | 3280 | 4220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 765 | 1710 |  | 9055 | 11770 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 4910 |  | 6740 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5790 |  | 6735 | 10730 | 13445 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8260 |  | 10090 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12355 |  | 15970 | 80 | 880 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8210 |  | 10940 | 4065 | 5910 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14030 |  | 17645 | 6620 | 7580 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11560 |  | 14290 | 9090 | 10935 |

Based on Table 6.15.3-1;

3rd order harmonic may fall into Rx frequencies of bands 41

2nd order IMD may also fall into Rx frequencies of band 7, 38, 41, 69 and n78

3rd order IMD may also fall into own Rx of band 19 and Rx frequencies of bands 1, 2, 3, 4, 5, 6, 8, 9, 10, 12, 13, 14, 17, 18, 20, 22, 23, 25, 26, 27, 28, 30, 33, 34, 35, 36, 37, 39, 40, 41, 42, 43, 44, 46, 47, 48, 65, 66, 67, 68, 70, n78 and n79

4th order IMD may also fall into own Rx of band 19 and Rx frequencies of bands 5, 6, 8, 11, 14, 18, 20, 21, 24, 26, 27, 28, 44, 45, 46, 47, 68 and n79

5th order IMD may also fall into own Rx of band 19 and Rx frequencies of bands 5, 6, 12, 13, 14, 17, 18, 20, 26, 27, 28, 29, 31, 44, 46, 47, 67, 68 and n79

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.15.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.15.3-2: 2UL Band 19 +Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2, 3 |
|  | 2400 | - | 2494 | Yes | Asia | Harmonic3, IMD2, 3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4, 5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4, 5 |

Table 6.15.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.15.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_19A-n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.15.4	∆TIB and ∆RIB values

For DC_19A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-n77 | 19 | 0.3 |
|  | n77 | 0.8 |

Table 6.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-n77 | 19 | 0 |
|  | n77 | 0.5 |

### 6.15.5	MSD

Table 6.15.5-1 lists the MSD due to 5th harmonic required for the dual connectivity configuration.

Table 6.15.5-1: Reference sensitivity for the DC configuration for 5th harmonic

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Duplex mode |
| CA_19A-n77A | 19 | -99.8 | -96.8 | -95 | -93.8 |  |  |  |  |  | FDD |
|  | n77 |  | [-85.4] | [-85.1] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | TDD |

For IMD5, the worst case is that there is 17.5 MHz frequency offset between the IM center of 865 MHz and the victim center frequency of 877.5 MHz. Considering the correction factor, no MSD should be specified for this case.

### 6.15.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics or inter-modulation products (IMD) based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.15.3 co-existence studies which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.15.6-1.

Table 6.15.6-1: Band 19 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A_n77 | 19 | a | -3 | b | -1 | 4 | 19 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_19A_n77 | 19 | a | -4 | b | -1 | 5 | 19 | IMD |
|  | n77 | c | -1 | d | 0 |  |  |  |
| DC_19A_n77 | 19 | a | 4 | b | 0 | 4 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |
| DC_19A_n77 | 19 | a | 5 | b | 0 | 5 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |

Equations (6.15.6.1) and (6.15.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.15.6-1 and CBW stands for channel bandwidth.

(6.15.6.1)

(6.15.6.2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.15.6.3-1)

(6.15.6.3-2)

## 6.16	DC_19A-n78

### 6.16.1	Operating bands for DC

Table 6.16.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.16.2	Channel bandwidths per operating band for DC

Table 6.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-n78A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-n78C | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 215 |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.16.3	Co-existence studies

Table 6.16.3-1 lists Band 19 +Band n78 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.16.3-1: Band 19 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 |  | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 |  | 1690 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 |  | 2535 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2455 | 2970 |  | 4130 | 4645 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1610 |  | 2140 | 5755 | 6770 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4960 |  | 5490 | 7430 | 8445 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 730 |  | 945 | 3280 | 3820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 765 | 1310 |  | 9055 | 10570 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 4910 |  | 5940 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5790 |  | 6335 | 10730 | 12245 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8260 |  | 9290 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12355 |  | 14370 | 80 | 480 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8210 |  | 9740 | 4065 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14030 |  | 16045 | 6620 | 7180 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11560 |  | 13090 | 9090 | 10135 |

Based on Table 6.16.3-1;

3rd order harmonic may fall into Rx frequencies of band 41

2nd order IMD may also fall into Rx frequencies of band 7, 38, 41, 69, n77, n79

3rd order IMD may also fall into own Rx of band 19 and Rx frequencies of bands 1, 2, 3, 4, 5, 6, 8, 9, 10, 12, 13, 14, 17, 18, 20, 22, 25, 26, 27, 28, 33, 34, 35, 36, 37, 39, 42, 43, 44, 46, 47, 48, 65, 66, 67, 68, 70, n77 and n79

4th order IMD may also fall into own Rx of band 19 and Rx frequencies of bands 5, 6, 8, 14, 18, 20, 26, 27, 28, 44, 46, 47, 68 and n79

5th order IMD may also fall into Rx frequencies of bands 31, n77 and n79

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.16.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.16.3-2: 2UL Band 19 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | Harmonic3, IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4 |

Table 6.16.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.16.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_19A-n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.16.4	∆TIB and ∆RIB values

For DC_19A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-n78 | 19 | 0.3 |
|  | n78 | 0.8 |

Table 6.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-n78 | 19 | 0 |
|  | n78 | 0.5 |

### 6.16.5	MSD

There is no MSD issue for this DC configuration.

### 6.16.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.16.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.16.6-1.

Table 6.16.6-1: Band 19 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A_n78A | 19 | a | -3 | b | -1 | 4 | 19 | IMD |
|  | n78 | c | 1 | d | 0 |  |  |  |
| DC_19A_n78A | 19 | a | 4 | b | 0 | 4 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (6.16.6.1) and (6.16.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.16.6-1 and CBW stands for channel bandwidth.

(6.16.6.1)

(6.16.6.2)

Formula (6.16.6.3-1) and (6.16.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.16.6.3-1)

(6.16.6.3-2)

## 6.17	DC_19A-n79

### 6.17.1	Operating bands for DC

Table 6.17.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.17.2	Channel bandwidths per operating band for DC

Table 6.17.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-n79A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-n79C | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 215 |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.17.3	Co-existence studies

Table 6.17.3-1 lists Band 19 +Band n79 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.17.3-1: Band 19 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 |  | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 |  | 1690 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 |  | 2535 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3555 | 4170 |  | 5230 | 5845 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2710 |  | 3340 | 7955 | 9170 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6060 |  | 6690 | 9630 | 10845 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 730 |  | 945 | 4385 | 5015 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1865 | 2510 |  | 12355 | 14170 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 7110 |  | 8340 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6890 |  | 7535 | 14030 | 15845 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10460 |  | 11690 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16755 |  | 19170 | 1020 | 1680 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11510 |  | 11340 | 6265 | 7510 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18430 |  | 20845 | 7720 | 8380 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14860 |  | 16690 | 11290 | 12535 |

Based on Table 6.17.3-1;

3rd order harmonic may fall into Rx frequencies of bands 41

2nd order IMD may also fall into Rx frequencies of bands 22, 42, 43, 46, 48, n77 (3.3 – 4.2 GHz) and n78 (3.3 – 3.8 GHz)

3rd order IMD may also fall into own Rx of band 19 and Rx frequencies of bands5, 6, 8, 12, 13, 14, 17, 18, 20, 26, 27, 28, 44, 67, 68, n77 (3.3 – 4.2 GHz) and n78 (3.3 – 3.8 GHz)

4th order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, 41, 65, 66, 70

5th order IMD may also fall into Rx frequencies of bands 11, 21, 24, 45

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.17.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.17.3-2: 2UL Band 19 +Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | Harmonic3, IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2 |
|  | 5470 | - | 5725 | Yes |  | IMD2 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2 |

Table 6.17.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.17.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_19A-n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.17.4	∆TIB and ∆RIB values

For DC_19A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-n79 | 19 | 0 |
|  | n79 | 0 |

Table 6.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-n79 | 19 | 0 |
|  | n79 | 0 |

### 6.17.5	MSD

For 6th harmonic, Even if HTF is implemented, little IL impact is expected considering that reported MSD in LTE Band 46 discussion is up to 8.8 dB. Therefore it is agreed to specify no MSD requirement for the 6th harmonic cases and not to assume additional IL i.e. Tib and Rib due to HTF. For 5th harmonic mixing, the MSD values are given in Table 6.17.5-1.

Table 6.17.5-1 MSD due to harmonic mixing for DC_19-n79

| MSD due to harmonic mixing exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n79 | 19 | 29.5 | 26.5 | 24.7 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |

Uplink configuration in Table shall be considered for the harmonic mixing test for DC_19-n79. The purpose is to verify the REFSENS degradation of the low band, therefore, there is no need to configure all the CBWs for the NR band. For larger SCS for the NR band, the RB allocation should be as close as the bandwidth for15kHz SCS.

Table 6.17.5-2 Uplink configuration for the high band (exceptions) for NR

| NR Band / Channel bandwidth of the high band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | SCS | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| DC_19-n79 | n79 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |
| NOTE 1: RB allocations in this table are corresponding to channel bandwidth of Band 19. |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.17.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.17.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.17.6-1.

Table 6.17.6-1: Band 19 and Band n79 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A_n79A | 19 | a | 0 | b | -5 | 5 | 19 | Harmonic mixing |
|  | n79 | c | 1 | d | 0 |  |  |  |
| DC_19A_n79A | 19 | a | 6 | b | 0 | 6 | n79 | Harmonic |
|  | n79 | c | 0 | d | -1 |  |  |  |

Equations (6.17.6.1) and (6.17.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.17.6-1 and CBW stands for channel bandwidth.

(6.17.6.1)

(6.17.6.2)

Formula (6.17.6.3-1) and (6.17.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.17.6.3.1)

(6.17.6.3.2)

## 6.18	DC_21A-n77

### 6.18.1	Operating bands for DC

Table 6.18.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.18.2	Channel bandwidths per operating band for DC

Table 6.18.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-n77A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-n77C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 215 |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.18.3	Co-existence studies

Table 6.18.3-1 lists Band 21 +Band n77 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.18.3-1: Band 21 and Band n77 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 |  | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 |  | 2925.8 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 |  | 4388.7 | 9900 | 12600 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1837.1 | 2752.1 |  | 4747.9 | 5662.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 374.2 |  | 1304.2 | 5137.1 | 6952.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6195.8 |  | 7125.8 | 8047.9 | 9862.9 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1347.9 |  | 1562.9 | 3285 | 4215 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 143.7 | 1088.7 |  | 8437.1 | 11152.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3674.2 |  | 5504.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7643.7 |  | 8588.7 | 11347.9 | 14062.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 9495.8 |  | 11325.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11737.1 |  | 15352.1 | 1591.6 | 2551.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6974.2 |  | 9704.2 | 2211.3 | 4056.3 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14647.9 |  | 18262.9 | 9091.6 | 10051.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12795.8 |  | 15525.8 | 10943.7 | 12788.7 |

Based on Table 6.18.3-1;

2nd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 46, 65, 66, 69, 70 and n79

3rd order IMD may also fall into own Rx of band 21 and Rx frequencies of bands 5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 22, 24, 26, 27, 28, 29, 31, 42, 43, 44, 45, 46, 47, 48, 67, 68, 70 and n78

4th order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 43, 44, 46, 48, 67, 68, n78 and n79

5th order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 48, 65, 66, 69, 70 and n78

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.18.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.18.3-2: 2UL Band 21 +Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2, 5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2, 5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, 3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2, 3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, 3, 4 |

Table 6.18.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.18.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_21A-n77A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.18.4	∆TIB and ∆RIB values

For DC_21A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-n77 | 21 | 0.4 |
|  | n77 | 0.8 |

Table 6.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A-n77 | 21 | 0 |
|  | n77 | 0.5 |

### 6.18.5	MSD

There is no MSD issue for this DC configuration.

## 6.19	DC_21A-n78

### 6.19.1	Operating bands for DC

Table 6.19.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.19.2	Channel bandwidths per operating band for DC

Table 6.19.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-n78A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-n78C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 215 |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.19.3	Co-existence studies

Table 6.19.3-1 lists Band 21 +Band n78 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.19.3-1: Band 21 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 |  | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 |  | 2925.8 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 |  | 4388.7 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1837.1 | 2352.1 |  | 4747.9 | 5262.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 374.2 |  | 904.2 | 5137.1 | 6152.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6195.8 |  | 6725.8 | 8047.9 | 9062.9 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1347.9 |  | 1562.9 | 3285 | 3815 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 543.7 | 1088.7 |  | 8437.1 | 9952.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3674.2 |  | 4704.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7643.7 |  | 8188.7 | 11347.9 | 12862.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 9495.8 |  | 10525.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11737.1 |  | 13752.1 | 1991.6 | 2551.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6974.2 |  | 8504.2 | 2211.3 | 3256.3 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14647.9 |  | 16662.9 | 9091.6 | 9651.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12795.8 |  | 14325.8 | 10943.7 | 11988.7 |

Based on Table 6.19.3-1;

2nd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, 46, 65, 66, 70 and n79

3rd order IMD may also fall into own Rx of band 21 and Rx frequencies of bands 5, 6, 11, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 43, 44, 48, 67, 68 and n77

4th order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 43, 44, 48, 67, 68 n77 and n79

5th order IMD may also fall into Rx frequencies of bands 1, 4, 7, 10, 23, 25, 30, 34, 38, 40, 41, 65, 66, 69 and 70

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.19.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.19.3-2: 2UL Band 21 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD3 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, 3 |

Table 6.19.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.19.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_21A-n78A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.19.4	∆TIB and ∆RIB values

For DC_21A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-n78 | 21 | 0.4 |
|  | n78 | 0.8 |

Table 6.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A-n78 | 21 | 0 |
|  | n78 | 0.5 |

### 6.19.5	MSD

There is no MSD issue for this DC configuration.

## 6.20	DC_21A-n79

### 6.20.1	Operating bands for DC

Table 6.20.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.20.2	Channel bandwidths per operating band for DC

Table 6.20.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-n79A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-n79C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 215 |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.20.3	Co-existence studies

Table 6.20.3-1 lists Band 21 +Band n79 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.20.3-1: Band 21 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 |  | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 |  | 2925.8 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 |  | 4388.7 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2937.1 | 3552.1 |  | 5847.9 | 6462.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1474.2 |  | 2104.2 | 7337.1 | 8552.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7295.8 |  | 7925.8 | 10247.9 | 14462.9 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1347.9 |  | 1562.9 | 4385 | 5015 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 11.3 | 656.3 |  | 11737.1 | 13552.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 5874.2 |  | 7104.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8743.7 |  | 9388.7 | 14647.9 | 16462.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11695.8 |  | 12925.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137.1 |  | 18552.1 | 791.6 | 1451.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10274.2 |  | 12104.2 | 4411.3 | 5656.3 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19047.9 |  | 21462.9 | 10191.6 | 10851.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 16095.8 |  | 17925.8 | 13143.7 | 14388.7 |

Based on Table 6.20.3-1;

2nd order IMD may also fall into Rx frequencies of bands 22, 42, 46, 47, 48, n77 (3.3 – 4.2 GHz) and n78 (3.3 – 3.8 GHz)

3rd order IMD may also fall into own Rx of band 21 and Rx frequencies of bands2, 3, ,9, 11, 24, 25, 33, 34, 35, 36, 37, 39, 45, 70

4th order IMD may also fall into Rx frequencies of bands 31, 46, 47

5th order IMD may also fall into Rx frequencies of bands 5, 6, 8, 18, 19, 20, 26, 27, 28, 44, 45, 46

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.20.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.20.3-2: 2UL Band 21 +Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD5 |

Table 6.20.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.20.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_21A-n79A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.20.4	∆TIB and ∆RIB values

For DC_21A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-n79 | 21 | 0 |
|  | n79 | 0 |

Table 6.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A-n79 | 21 | 0 |
|  | n79 | 0 |

### 6.20.5	MSD

Table 6.20.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.20.5-1: MSD for the DC configuration for IMD3

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_21A-n79A | 21 | IMD3 | ¦fB79 -2*fB21¦ | 1457.5 | 5 | 25 | 1505.5 | 5 | 1.8 | [18.4] |
|  | n79 |  |  | 4420.5 | 40 | 216 | 4420.5 | 40 |  | N/A |

Table 6.20.5-2: Reference sensitivity for the DC configuration for 3rd harmonic mixing

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Duplex mode |
| CA_21A-n79A | 21 | -60.7 | -60.7 | -60.7 |  |  |  |  |  |  | FDD |
|  | n79 |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TDD |

### 6.20.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.20.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.20.6-1.

Table 6.20.6-1: Band 21 and Band n79 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_21A_n79A | 21 | a | -2 | b | -1 | 3 | 21 | IMD |
|  | n79 | c | 1 | d | 0 |  |  |  |
| DC_21A_n79A | 21 | a | 0 | b | 3 | 3 | 21 | Harmonic mixing |
|  | n79 | c | -1 | d | 0 |  |  |  |

Equations (6.20.6.1) and (6.20.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.20.6-1 and CBW stands for channel bandwidth.

(6.20.6.1)

(6.20.6.2)

Formula (6.20.6.3-1) and (6.20.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.20.6.3-1)

(6.20.6.3-2)

## 6.21	DC_26A-n77A

### 6.21.1	Operating bands for DC

Table 6.21.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_26A-n77A | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.21.2	Channel bandwidths per operating band for DC

Table 6.21.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_26A-n77A | 26 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.21.3	Co-existence studies

Table 6.21.3-1 lists Band26+3.3-4.2GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.21.3-1: Band 26 and Band n77 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 | 3386 | 4114 | 5049 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2572 | 1602 | 5751 | 7586 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4928 | 5898 | 7414 | 9249 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| IMD frequency limits (MHz) | 714 | 949 | 3280 | 4220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1758 | 753 | 9051 | 11786 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4902 | 6772 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5742 | 6747 | 10714 | 13449 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8228 | 10098 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15986 | 12351 | 96 | 944 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10972 | 8202 | 4053 | 5958 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14014 | 17649 | 6556 | 7596 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11528 | 14298 | 9042 | 10947 |

Based on the table, 3rd, 4th, 5th order IMD may fall into own Rx of band 26, 4th , 5th order harmonic and  2nd, 3rd, 5th order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.21.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.21.3-2: Band 26 and Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD4 |
| Galileo | 1559 | - | 1591 | YES |  | IMD4 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD4 |
| GPS | 1563 | - | 1587 | YES |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd harmonic, IMD2, IMD3 |
|  | 2400 | - | 2494 | YES | Asia | 3rd harmonic, IMD2, IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | YES | Europe | IMD3, IMD4, IMD5 |
|  | 5470 | - | 5725 | YES |  | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5825 | YES | Asia | IMD3, IMD4, IMD5 |

Table 6.21.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.21.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_26A-n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.21.4	∆TIB and ∆RIB values

For DC_26A-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_26A-n77A | 26 | 0.3 |
|  | n77 | 0.8 |

Table 6.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_26A-n77A | 26 | 0 |
|  | n77 | 0.5 |

### 6.21.5	MSD

Table 6.21.5-1 lists the MSD due to 4th harmonic required for EN-DC configuration DC_26A-n77A.

Table 6.21.5-1: MSD due to 4th harmonic for DC_26A-n77A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 26 | n771,2 |  | 10.8 | 9.1 | 8 |  | 6 |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.21.5-2 lists the UL configuration for the EN-DC configuration DC_26A-n77A.

Table 6.21.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | n77 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.21.5-3 lists the MSD due to 5th harmonic required for EN-DC configuration DC_26A-n77A.

Table 6.21.5-3: MSD due to 5th harmonic for DC_26A-n77A

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | n774,5 |  | [10.4] | [8.9] | [7.8] |  | [4.7] | [3.7] | [3] | [1.7] | [0.7] |

Table 6.21.5-4 lists the UL configuration for the EN-DC configuration DC_26A-n77A.

Table 6.21.5-4: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | n77 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of equal of larger than 40MHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.21.5-5 lists the MSD required for the dual connectivity configuration.

Table 6.21.5-5: MSD due to IMD issue for DC_26A-n77A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_26A-n77A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD41 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.21.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics or inter-modulation products (IMD) based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.21.3 co-existence studies which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.21.6-1.

Table 6.21.6-1: Band 26 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_26A_n77 | 26 | a | -3 | b | -1 | 4 | 26 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_26A_n77 | 26 | a | -4 | b | -1 | 5 | 26 | IMD |
|  | n77 | c | -1 | d | 0 |  |  |  |
| DC_26A_n77 | 26 | a | 4 | b | 0 | 4 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |
| DC_26A_n77 | 26 | a | 5 | b | 0 | 5 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |

Equations (6.21.6.1) and (6.21.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.21.6-1 and CBW stands for channel bandwidth.

(6.21.6.1)

(6.21.6.2)

Formula (6.21.6.3-1) and (6.21.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.21.6.3-1)

(6.21.6.3-2)

## 6.22	DC_26A-n78A

### 6.22.1	Operating bands for DC

Table 6.22.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_26A-n78A | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.22.2	Channel bandwidths per operating band for DC

Table 6.22.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_26A-n78A | 26 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.22.3	Co-existence studies

Table 6.22.3-1 lists Band26+3.3-3.8GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.22.3-1: Band 26 and Band n78 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 | 2986 | 4114 | 4649 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2172 | 1602 | 5751 | 6786 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4928 | 5498 | 7414 | 8449 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fx_low – fy_BW max¦ | ¦fx_high + fy_BW max¦ |
| IMD frequency limits (MHz) | 714 | 949 | 3280 | 3820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1358 | 753 | 9051 | 10586 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4902 | 5972 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5742 | 6347 | 10714 | 12249 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8228 | 9298 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14386 | 12351 | 96 | 544 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9772 | 8202 | 4053 | 5158 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14014 | 16049 | 6556 | 7196 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11528 | 13098 | 9042 | 10147 |

Based on the table, 4th order harmonic and 3rd, 4th order IMD may fall into own Rx of band 26, and  3rd order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.22.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.22.3-2: Band 26 and Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  | IMD3 |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe | 3rd harmonic, IMD2 |
|  | 2400 | - | 2494 |  | Asia | 3rd harmonic, IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 |  | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 |  | Europe | IMD3, IMD4, IMD5 |
|  | 5470 | - | 5725 |  |  | IMD3, IMD4 |
|  | 5150 | - | 5825 |  | Asia | IMD3, IMD4, IMD5 |

Table 6.22.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.22.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_26A-n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.22.4	∆TIB and ∆RIB values

For DC_26A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_26A-n78A | 26 | 0.3 |
|  | n78 | 0.8 |

Table 6.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_26A-n78A | 26 | 0 |
|  | n78 | 0.5 |

### 6.22.5	MSD

Table 6.22.5-1 lists the MSD due to 4th harmonic required for the dual connectivity configuration.

Table 6.22.5-1: MSD due to 4th harmonic for DC_26A-n78A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 26 | n781,2 |  | 10.8 | 9.1 | 8 |  | 6 |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.22.5-2 lists the UL configuration for the EN-DC configuration DC_26A-n78A.

Table 6.22.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | n78 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.22.5-3 lists the MSD required for the dual connectivity configuration.

Table 6.22.5-3: MSD due to IMD issue for DC_26A-n78A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_26A-n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |

### 6.22.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.22.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.22.6-1.

Table 6.22.6-1: Band 26 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_26A_n78A | 26 | a | -3 | b | -1 | 4 | 26 | IMD |
|  | n78 | c | 1 | d | 0 |  |  |  |
| DC_26A_n78A | 26 | a | 4 | b | 0 | 4 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (1) and (2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.22.6-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

## 6.23	DC_26A-n79A

### 6.23.1	Operating bands for DC

Table 6.23.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_26A-n79A | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.23.2	Channel bandwidths per operating band for DC

Table 6.23.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_26A-n79A | 26 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 115 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.23.3	Co-existence studies

Table 6.23.3-1 lists Band26+4.4-5GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.23.3-1: Band 26 and Band n79 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3551 | 4186 | 5214 | 5849 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3372 | 2702 | 7951 | 9186 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6028 | 6698 | 9614 | 10849 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦f1_low –f2_BWmax¦ | ¦f1_high + f2_BW max¦ | ¦f2_low – f1_BW max¦ | ¦f2_high + f1_BW max¦ |
| IMD frequency limits (MHz) | 714 | 949 | 4380 | 5020 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2558 | 1853 | 12351 | 14186 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7102 | 8372 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6842 | 7547 | 14014 | 15849 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10428 | 11698 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 19186 | 16751 | 1004 | 1744 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 13372 | 11502 | 6253 | 7558 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18414 | 20849 | 7656 | 8396 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14828 | 16698 | 11242 | 12547 |

Based on the table, 3rd order IMD may fall into own Rx of band 26, and 3rd order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.23.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.23.3-2: Band 26 and Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  | IMD5 |
| Galileo | 1559 | - | 1591 |  |  | IMD5 |
| GLONASS | 1591 | - | 1610 |  |  | IMD5 |
| GPS | 1563 | - | 1587 |  |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe | 3rd harmonic, IMD4 |
|  | 2400 | - | 2494 |  | Asia | 3rd harmonic, IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 |  | US | IMD2 |
|  | 5150 | - | 5350 |  | Europe | IMD2 |
|  | 5470 | - | 5725 |  |  | IMD2 |
|  | 5150 | - | 5825 |  | Asia | IMD2 |

Table 6.23.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.23.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_26A-n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.23.4	∆TIB and ∆RIB values

For DC_26A-n79A, the TIB,c and RIB values are given in the tables below.

Table 6.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_26A-n79A | 26 | 0 |
|  | n79 | 0 |

Table 6.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_26A-n79A | 26 | 0 |
|  | n79 | 0 |

### 6.23.5	MSD

Table 6.23.5-1 gives the MSD values for DC_26A-n79A due to harmonic mixing.

Table 6.23.5-1 MSD due to harmonic mixing for DC_26A-n79A

| MSD due to harmonic mixing exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n79 | 26 | 27 | 24 | 22.2 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |

Uplink configuration in Table shall be considered for the harmonic mixing test for DC_26A-n79A. The purpose is to verify the REFSENS degradation of the low band, therefore, there is no need to configure all the CBWs for the NR band. For larger SCS for the NR band, the RB allocation should be as close as the bandwidth for15kHz SCS.

Table 6.23.5-2 Uplink configuration for the high band (exceptions) for NR

| NR Band / Channel bandwidth of the high band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | SCS | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| DC_26-n79 | n79 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |
| NOTE 1: RB allocations in this table are corresponding to channel bandwidth of Band 26. |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.23.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.23.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.23.6-1.

Table 6.23.6-1: Band 26 and Band n79 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_26A_n79A | 26 | a | 0 | b | -5 | 5 | 26 | Harmonic mixing |
|  | n79 | c | 1 | d | 0 |  |  |  |
| DC_26A_n79A | 26 | a | 6 | b | 0 | 6 | n79 | Harmonic |
|  | n79 | c | 0 | d | -1 |  |  |  |

Equations (6.23.6.1) and (6.23.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.23.6-1 and CBW stands for channel bandwidth.

(6.23.6.1)

(6.23.6.2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.23.6.3-1)

(6.23.6.3-2)

## 6.24	DC_26A-n257A

### 6.24.1	Operating bands for DC

Table 6.24.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_26A_n257 | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.24.2	Channel bandwidths per operating band for DC

Table 6.24.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_26A_n257 | 26 | 15 | Yes | Yes | Yes |  |  |  |  |  | 415 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.24.3	Co-existence studies

Table 6.24.3-1 lists Band26+26.5-29.5GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.24.3-1: Band 26 and Band n257 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25651 | 28686 | 27314 | 30349 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27872 | 24802 | 52151 | 58186 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28128 | 31198 | 53814 | 59849 |
| Two-tone 3rd order IMD products (with | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| maximum channel bandwidth) |  |  |  |  |
| IMD frequency limits (MHz) | 414 | 1249 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27058 | 23953 | 78651 | 87686 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 51302 | 57372 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28942 | 32047 | 80314 | 89349 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54628 | 60698 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117186 | 105151 | 23104 | 26244 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 86872 | 77802 | 50453 | 56558 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106814 | 118849 | 29756 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81128 | 90198 | 55442 | 61547 |

Based on the table, 3rd order IMD may fall into own Rx of band 26, and 2nd, 3rd and 4th order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.24.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.24.3-2: Band 26 and Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 |  | US |  |
|  | 5150 | - | 5350 |  | Europe |  |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 |  | Asia |  |

Table 6.24.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.24.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_26A-n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

### 6.24.4	∆TIB and ∆RIB values

For DC_26A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_26A_n257 | 26 | 0 |
|  | n257 | 0 |

Table 6.24.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_26A_n257 | 26 | 0 |
|  | n257 | 0 |

### 6.24.5	MSD

There is no MSD issue for this DC configuration.

## 6.25	DC_28A-n77

### 6.25.1	Operating bands for DC

Table 6.25.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-n77 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.25.2	Channel bandwidths per operating band for DC

Table 6.25.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_28A-n77A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| DC_28A-n77C | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |  |

### 6.25.3	Co-existence studies

For 2UL/2DL UE coexistence study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.25.3-1

Table 6.25.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3497 | 4003 | 4948 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2794 | 1804 | 5852 | 7697 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5696 | 7303 | 9148 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2091 | 1056 | 9152 | 11897 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6444 | 10603 | 13348 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 6994 | 5104 | 8006 | 9896 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16097 | 12452 | 308 | 1388 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 17548 | 6112 | 7192 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11194 | 8404 | 4356 | 6291 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 14096 | 8709 | 10644 |

Based on the table 6.25.3-1, the 5th order IMD products may fall in the own Rx of Band 28. The 2nd order IMD products may fall in the own Rx of NR Band (3300-4200MHz).In addition, 2nd and 3rd order harmonic and 2nd, 3rd, 4th and 5th IMD products may also fall into Rx frequencies of bands 1-14, 17-21, 23-42, and 44-70.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.25.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.25.3-2: 2UL Band 28 +Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4, 5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4, 5 |

Table 6.25.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.25.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28A-n77A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -36 dBm/MHz for 718-748 MHz |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -38 dBm/MHz for 718-748 MHz |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.25.4	∆TIB and ∆RIB values

Table 6.25.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-n77 | 28 | 0.5 |
|  | n77 | 0.8 |
| NOTE 1: HTF is assumed for Band 28. |  |  |

Table 6.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A-n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: HTF is assumed for Band 28. |  |  |

### 6.25.5	MSD

Table 6.25.5-1: Reference sensitivity for the dual connectivity configuration for 5th harmonic

| EUTRA/NR DC Configuration | EUTRA/NR band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_28A-n77A | 28 | -98.3 | -95.3 | -93.5 | -90.8 |  |  |  |  |  | FDD |
|  | n77 |  | [-85.4] | [-85.1] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | TDD |

Table 6.25.5-2: MSD for the dual connectivity configuration for IMD5

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_28A-n77A | 28 | 705.5 | 5 | 25 | 760.5 | [5.5] | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 25 | 3582.5 | N/A | TDD | N/A |

Table 6.25.5-3: MSD for the DC configuration for 5th harmonic mixing

| MSD due to harmonic mixing exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n77 | 28 | 28 | 25 | 23.2 | 22 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |

Uplink configuration in Table shall be considered for the harmonic mixing test for DC_28-n77. The purpose is to verify the REFSENS degradation of the low band, therefore, there is no need to configure all the CBWs for the NR band. For larger SCS for the NR band, the RB allocation should be as close as the bandwidth for15kHz SCS.

Table 6.20.5-4 Uplink configuration for the high band (exceptions) for NR

| NR Band / Channel bandwidth of the high band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | SCS | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| DC_28-n77 | n77 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |
| NOTE 1: RB allocations in this table are corresponding to channel bandwidth of Band 28. |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.25.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.25.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.25.6-1.

Table 6.25.6-1: Band 28 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n77A | 28 | a | -4 | b | -1 | 5 | 28 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_28A_n77A | 28 | a | 5 | b | 0 | 5 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |
| DC_28A_n77A | 28 | a | 0 | b | -5 | 5 | 28 | Harmonic mixing |
|  | n77 | c | 1 | d | 0 |  |  |  |

Equations (6.25.6.1) and (6.25.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.25.6-1 and CBW stands for channel bandwidth.

(6.25.6.1)

(6.25.6.2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.25.6.3-1)

(6.25.6.3-2)

## 6.26	DC_28A-n78

### 6.26.1	Operating bands for DC

Table 6.26.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.26.2	Channel bandwidths per operating band for DC

Table 6.26.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A-n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_28A-n78C | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.26.3	Co-existence studies

For 2UL/2DL UE coexistence study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.26.3-1

Table 6.26.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3097 | 4003 | 4548 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 | 8348 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 | 10697 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 | 12148 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 | 9096 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 | 988 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 | 5491 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 | 9844 |

Based on the table 6.26.3-1, the 5 order IMD products may fall in the own Rx of Band 28. In addition, 2nd and 3rd order harmonic and 2nd, 3rd, 4th and 5th IMD products may also fall into Rx frequencies of bands 1-14, 17-21, 23-41 and 44-70.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.26.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.26.3-2: 2UL Band 28 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD4, 5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4, 5 |

Table 6.26.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.26.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28A-n78A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -36 dBm/MHz for 718-748 MHz |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -38 dBm/MHz for 718-748 MHz |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.26.4	∆TIB and ∆RIB values

Table 6.26.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-n78 | 28 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 28. |  |  |

Table 6.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A-n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 28. |  |  |

### 6.26.5	MSD

Table 6.26.5-1: Reference sensitivity for the dual connectivity configuration for 5th harmonic

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Duplex mode |
| CA_28A-n78A | 28 | -98.3 | -95.3 | -93.5 | -90.8 |  |  |  |  |  | FDD |
|  | n78 |  | [-85.4] | [-85.1] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | [-84.9] | TDD |

Table 6.25.5-2: MSD for the dual connectivity configuration for IMD5

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_28A-n78A | 28 | 705.5 | 5 | 25 | 760.5 | [5.5] | FDD | IMD5 |
|  | n78 | 3582.5 | 10 | 25 | 3582.5 | N/A | TDD | N/A |

### 6.26.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.26.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.26.6-1.

Table 6.26.6-1: Band 28 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n78A | 28 | a | -4 | b | -1 | 5 | 28 | IMD |
|  | n78 | c | 1 | d | 0 |  |  |  |
| DC_28A_n78A | 28 | a | 5 | b | 0 | 5 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |
| DC_28A_n78A | 28 | a | 0 | b | -5 | 5 | 28 | Harmonic mixing |
|  | n78 | c | 1 | d | 0 |  |  |  |

Equations (6.26.6.1) and (6.26.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.26.6-1 and CBW stands for channel bandwidth.

(6.26.6.1)

(6.26.6.2)

Formula (6.26.6.3-1) and (6.26.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.26.6.3-1)

(6.26.6.3-2)

## 6.27	DC_28A-n79

### 6.27.1	Operating bands for DC

Table 6.27.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-n79 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.27.2	Channel bandwidths per operating band for DC

Table 6.27.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A-n79A | 28 | 15 | Yes | Yes |  |  |  |  |  |  |  | 110 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_28A-n79C | 28 | 15 | Yes | Yes |  |  |  |  |  |  |  | 210 |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.27.3	Co-existence studies

Table 6.27.3-1 lists Band 28 +Band n79 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.27.3-1: Band 28 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 |  | 1496 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 |  | 2244 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3652 | 4297 |  | 5103 | 5748 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2904 |  | 3594 | 8052 | 9297 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5806 |  | 6496 | 9503 | 10748 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 603 |  | 848 | 4390 | 5010 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2156 | 2891 |  | 12452 | 14297 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 7304 |  | 8594 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6509 |  | 7244 | 13903 | 15748 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10206 |  | 11496 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16852 |  | 19297 | 1408 | 2188 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11704 |  | 13594 | 6556 | 7891 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18303 |  | 20748 | 7212 | 7992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14606 |  | 16496 | 10909 | 12244 |

Based on Table 6.27.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 11, 21, 45

-	3rd order harmonic may fall into Rx frequencies of bands 1, 4, 10, 23, 65, 66

-	2nd order IMD may also fall into Rx frequencies of bands 43, 46, 48, n77 (3.3 – 4.2 GHz) and n78 (3.3 – 3.8 GHz)

-	3rd order IMD may also fall into own Rx of band 28 and Rx frequencies of bands12, 13, 14, 17, 20, 22, 29, 42, 44, 46, 47, 48, 67, 68, n77 (3.3 – 4.2 GHz) and n78 (3.3 – 3.8 GHz)

-	4th order IMD may also fall into Rx frequencies of bands 1, 7, 10, 23, 30, 38, 40, 41, 65, 66, 69

-	5th order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 45, 65, 66, 70

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.27.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.27.3-2: 2UL Band 28 +Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, IMD3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2 |
|  | 5470 | - | 5725 | Yes |  | IMD2 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, IMD3 |

Table 6.27.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.27.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28A-n79A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -36 dBm/MHz for 718-748 MHz |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -38 dBm/MHz for 718-748 MHz |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

### 6.27.4	∆TIB and ∆RIB values

For DC_28A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-n79 | 28 | 0 |
|  | n79 | 0 |

Table 6.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A-n79 | 28 | 0 |
|  | n79 | 0 |

### 6.27.5	MSD

Even if HTF is implemented, little IL impact is expected considering that reported MSD in LTE Band 46 discussion is up to 8.8 dB. Therefore it is agreed to specify no MSD requirement for the 6th and 7th harmonics cases and not to assume additional IL i.e. Tib and Rib due to HTF.

### 6.27.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.27.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.27.6-1.

Table 6.27.6-1: Band 28 and Band n79 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n79A | 28 | a | 6 | b | 0 | 6 | n79 | Harmonic |
|  | n79 | c | 0 | d | -1 |  |  |  |
| DC_28A_n79A | 28 | a | 7 | b | 0 | 7 | n79 | Harmonic |
|  | n79 | c | 0 | d | -1 |  |  |  |

Equations (6.27.6.1) and (6.27.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.27.6-1 and CBW stands for channel bandwidth.

(6.27.6.1)

(6.27.6.2)

Formula (6.27.6.3-1) and (6.27.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.27.63-1)

(6.27.63-2)

## 6.28	DC_41A-n257A

### 6.28.1	Operating bands for DC

Table 6.28.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n257 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.28.2	Channel bandwidths per operating band for DC

Table 6.28.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n257 | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.28.3	Co-existence studies

Table 6.28.3-1 lists Band41+26.5-29.5GHz 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.28.3-1: Band 41 and Band n257 UL harmonics and IMD products

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 23810 | 27004 | 28996 | 32190 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 24508 | 21120 | 50310 | 56504 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 31492 | 34880 | 55496 | 61690 |
| Two-tone 3rd order IMD products (with maximum channel bandwidth) | ¦fx_low –fy_BWmax¦ | ¦fx_high + fy_BW max¦ | ¦fy_low – fx_BW max¦ | ¦fy_high + fx_BW max¦ |
| IMD frequency limits (MHz) | 2096 | 3090 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22012 | 18430 | 76810 | 86004 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 47620 | 54008 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 33988 | 37570 | 81996 | 91190 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 57992 | 64380 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 115504 | 103310 | 15740 | 19516 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 83508 | 74120 | 44930 | 51512 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 108496 | 120690 | 36484 | 40260 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 84492 | 93880 | 60488 | 67070 |

Based on the table, 3rd order IMD may fall into own Rx of band 41, and 2nd and 3rd order IMD may fall into own Rx of NR band.

It should be noted that IMD will not be an issue for NR band (no self-interference) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.28.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.28.3-2: Band 41 and Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | IMD3 |
|  | 2400 | - | 2494 | YES | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 2nd harmonic |
|  | 5150 | - | 5350 | YES | Europe | 2nd harmonic |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 | YES | Asia | 2nd harmonic |

Table 6.28.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.28.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41A-n257A | E-UTRA Band 1, 3, 5, 8, 9, 11, 18, 19, 21, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | PHS |

### 6.28.4	∆TIB and ∆RIB values

For DC_41A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A_n257 | 41 | 0 |
|  | n257 | 0 |

Table 6.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A_n257 | 41 | 0 |
|  | n257 | 0 |

### 6.28.5	MSD

There is no MSD issue for this DC configuration.

## 6.29	DC_42A-n77

### 6.29.1	Operating bands for DC

Table 6.29.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42A-n77 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.29.2	Channel bandwidths per operating band for DC

Table 6.29.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42A-n77A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_42A-n77C | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.29.3	Co-existence studies

Since simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe, no co-existence studies are needed at this moment.

### 6.29.4	∆TIB and ∆RIB values

For DC_42A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42A-n77 | 42 | 0 |
|  | n77 | 0 |

Table 6.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42A-n77 | 42 | 0 |
|  | n77 | 0 |

### 6.29.5	MSD

There is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n77 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe

## 6.30	DC_42A-n78

### 6.30.1	Operating bands for DC

Table 6.30.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42A-n78 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.30.2	Channel bandwidths per operating band for DC

Table 6.30.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42A-n78A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_42A-n78C | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.30.3	Co-existence studies

Since simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe, no co-existence studies are needed at this moment.

### 6.30.4	∆TIB and ∆RIB values

For DC_42A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42A-n78 | 42 | 0 |
|  | n78 | 0 |

Table 6.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42A-n78 | 42 | 0 |
|  | n78 | 0 |

### 6.30.5	MSD

There is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n78 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe

## 6.31	DC_42A-n79

### 6.31.1	Operating bands for DC

Table 6.31.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42A-n79 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.31.2	Channel bandwidths per operating band for DC

Table 6.31.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42A-n79A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_42A-n79C | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 220 |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.31.3	Co-existence studies

Table 6.31.3-1 lists Band 42 +Band n79 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.31.3-1: Band 42 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |  | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 |  | 7200 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 |  | 10800 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 800 | 1600 |  | 7800 | 8600 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1800 |  | 2800 | 5200 | 6600 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11200 |  | 12200 | 12200 | 13600 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 3300 |  | 3700 | 4380 | 5020 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5200 | 6400 |  | 9600 | 11600 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1600 |  | 3200 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14600 |  | 15800 | 16600 | 18600 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 15600 |  | 17200 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14000 |  | 16600 | 8600 | 10000 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6000 |  | 8200 | 200 | 2000 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 21000 |  | 23600 | 18000 | 19400 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 20000 |  | 22200 | 19000 | 20800 |

Based on Table 6.31.3-1;

-	2nd order IMD may also fall into Rx frequencies of bands 5, 6, 8, 11, 18, 19, 20, 21, 24, 26, 27, 28, 44, 45

-	3rd order IMD may also fall into Rx frequencies of bands1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 46, 47, 48, 65, 66, 69, 70, n77 (3.3 – 4.2 GHz) and n78 (3.3 – 3.8 GHz)

-	4th order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 46, 47, 65, 66, 69, 70

-	5th order IMD may also fall into Rx frequencies of bands 2, 3, 5, 6, 8, 9, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 31, 33, 35, 36, 37, 39, 44, 45, 67, 68, 70

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.31.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.31.3-2: 2UL Band 42 +Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, IMD4, IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, IMD4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD4 |

Table 6.31.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.31.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_42A-n79A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-25] | 1 |  |

### 6.31.4	∆TIB and ∆RIB values

For DC_42A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42A-n79 | 42 | 0 |
|  | n79 | 0 |

Table 6.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42A-n79 | 42 | 0 |
|  | n79 | 0 |

### 6.31.5	MSD

There is no MSD issue for this DC configuration based on the followings.

-	IMD doesn’t matter since both bands are TDD

-	Frequency proximity doesn’t matter since there is a large frequency gap of 800 MHz between two bands

## 6.32	DC_20A-n28A

### 6.32.1	Operating bands for DC

Table 6.32.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A-n28A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 28 | 703 MHz | – | 733 MHz | 758 MHz | – | 788 MHz |  |

NOTE:	Band 28 is defined from UL 703 to 748 MHz and DL 758 to 803 MHz

### 6.32.2	Channel bandwidths per operating band for DC

Table 6.32.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A-n28A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 40 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.32.3	Co-existence studies

Tables 6.32.3-1, Band 20A+Band n28A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.32.3-1: Band 20 and Band n28 (lower duplex) UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 703 | 733 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 1406 – 1466 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 2109 – 2199 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 99 – 159 |  |  | 1535 – 1595 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 931 – 1021 |  |  | 544 – 634 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2367 – 2472 |  |  | 2238 – 2358 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 812 – 882 |  |  | 683 – 753 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1763 – 1883 |  |  | 1247 – 1367 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 198 – 318 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199 – 3319 |  |  | 2941 – 3061 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 3070 – 3190 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 – 2100 |  |  | 2595– 2745 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 385 – 535 |  |  | 1030 – 1180 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644 – 3794 |  |  | 4031 – 4181 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3373 – 3923 |  |  | 3902 – 4052 |  |  |

Based on Table 6.32.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 11, 21, 32, 45

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 38, 41, 65, 66, and 69

-	2nd order IMD may fall into Rx frequencies of Band 24

-	3rd order IMD may fall into Rx frequencies of Band 5, 8, 12, 13, 17, 18, 20, 26, 27, 29, 30, 40, 44, 67 and 68

-	4th order IMD may fall into Rx frequencies of Band 3, 9, 35, 39, n77, and n78

-	5th order IMD may fall into Rx frequencies of Band 1, 2, 4, 7, 10, 25, 31, 34, 36, 38, 41, 43, 48, 65, 66, 69, 70, n77, and n78

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.X.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.32.3-2: 2UL Band 20 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US | - |
|  | 5150 | - | 5350 | No | Europe | - |
|  | 5470 | - | 5725 | No |  | - |
|  | 5150 | - | 5825 | No | Asia | - |

Table 6.32.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.32.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_20A-n28A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.32.4	∆TIB and ∆RIB values

For DC_20A-n28A, the TIB,c and RIB values are given in the tables below.

Table 6.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-n28A | 20 | 0.5 |
|  | n28 | 0.5 |

Table 6.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-n28A | 20 | 0 |
|  | n28 | 0 |

### 6.32.5	MSD

No MSD is required.

## 6.33	DC_39A-n78A

### 6.33.1	Operating bands for DC

Table 6.33.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39A-n78A | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.33.2	Channel bandwidths per operating band for DC

Table 6.33.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_39A-n78A | 39 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.33.3	Co-existence studies

Table 6.33.3-1 lists Band 39+Band n78 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.33.3-1: Band 39 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1880 | 1920 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3760–3840 |  |  | 6600–7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5640–5760 |  |  | 9900–11400 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1380–1920 |  |  | 5180–5720 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 40–540 |  |  | 4680–5720 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7060–7640 |  |  | 8480–9520 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1780–2020 |  |  | 3285–3815 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1840–2460 |  |  | 7980–9520 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2760–3840 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8940–9560 |  |  | 11780–13320 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10360–11440 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11280–13320 |  |  | 3720–4380 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6060–7640 |  |  | 840–1960 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15080–17120 |  |  | 10820–11480 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13660–15240 |  |  | 12240–13360 |  |  |

Based on Table 6.33.3-1,

-	The 2th order harmonic of Band 39 may fall into Band n78. However, this band combination will only be used in China and the impacted frequency range 3760-3840MHz will not be allocated to IMT in China. So the 2nd harmonic issue will not be a problem for this combination.

-	The 3rd 4th and 5th order IMD product may also fall into Band n78. However IMD will not be an issue for Band n78 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.33.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.33.3-2: Band 39 and Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic IMD3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3 |
|  | 5470 | - | 5725 | Yes |  | 3rd harmonic IMD3 |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic IMD3 |

Table 6.33.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.33.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_39A-n78A | E-UTRA Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | N |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | N |
|  | NR Band n258 | FDL_low | - | FDL_high | [-5] | 100 |  |
| NOTE N: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

### 6.33.4	∆TIB and ∆RIB values

For DC_39A-n78A, the TIB,c and RIB values are given in Table 6.33.4-1 Table 6.33.4-2.

Table 6.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39A-n78A | 39 | 0.3 |
|  | n78 | 0.8 |

Table 6.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_39A-n78A | 39 | 0 |
|  | n78 | 0.5 |

### 6.33.5	MSD

According to the interference analysis, only harmonic issue in asynchronous operation may be a problem for this band combination. However, the harmonic issue only impacts the frequency range 3760-3800MHz in Band n78. According to the spectrum situation in China where this combination is applicable, only the frequency below 3600Mz is allocated to IMT and it is far from the impacted frequency range. So there is no harmonic problem for this combination. No MSD needs to be specified for this combination. While reduced frequency range need to be reflected for the combination in TS 38.101-3 [5] Table 5.2B.3.1-1 by a note “The frequency range above 3600MHz in Band n78 is not used in this combination.”

### 6.33.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.33.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.33.6-1.

Table 6.33.6-1: Band 39 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n78A | 39 | a | 2 | b | 0 | 2 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (6.33.6.1) and (6.33.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.33.6-1 and CBW stands for channel bandwidth.

(6.33.6.1)

(6.33.6.2)

Formula (6.33.6.3-1) and (6.33.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.33.6.3-1)

(6.33.6.3-2)

## 6.34 DC_39A-n258A

### 6.34.1	Operating bands for DC

Table 6.34.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39A-n258A | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.34.2	Channel bandwidths per operating band for DC

Table 6.34.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n257 | 39 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n258 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.34.3	Co-existence studies

Table 6.34.3-1 lists Band 39+Band n258 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.34.3-1: Band 39 and Band n258 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1880 | 1920 |  | 24250 | 27500 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3760–3840 |  |  | 48500–55000 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5640–5760 |  |  | 72750–82500 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 22330–25620 |  |  | 26130–29420 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 20410–23740 |  |  | 46580–53120 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 28010–31340 |  |  | 50380–56920 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1680–2120 |  |  | 24235–27515 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 21860–18490 |  |  | 70830–80620 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 51240–44660 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 29890–33260 |  |  | 74630–84420 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 52260–58840 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 108120–95080 |  |  | 16570–19980 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 78740–68910 |  |  | 42740–49360 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 98880–111920 |  |  | 31770–35180 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 76510–86340 |  |  | 54140–60760 |  |  |

Based on Table 6.34.3-1, 3rd IMD product may fall in to Band 39. 2nd 3rd order IMD product may also fall into Band n258. It should be noted that IMD will not be an issue (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.34.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.34.3-2: Band 39 and Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.34.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.34.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_39A-n258A | Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | N |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | N |
| NOTE N: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

### 6.34.4	∆TIB and ∆RIB values

For DC_39A_n258A, the TIB,c and RIB values are given in the tables below.

Table 6.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39A_n258 | 39 | 0 |
|  | n258 | 0 |

Table 6.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_39A_n258 | 39 | 0 |
|  | N258 | 0 |

### 6.34.5	MSD

No IMD issue is identified.

## 6.35	DC_1A-n257

### 6.35.1	Operating bands for DC

Table 6.35.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.35.2	Channel bandwidths per operating band for DC

Table 6.35.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_1A_n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A_n257E | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 620 |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A_n257F | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 820 |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.35.3	Co-existence studies

Table 6.35.3-1 lists Band 1 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.35.3-1: Band 1 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 |  | 3960 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 |  | 5940 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24520 | 27580 |  | 28420 | 31480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22540 |  | 25660 | 51020 | 57080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30340 |  | 33460 | 54920 | 60980 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1520 |  | 2380 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 20560 | 23740 |  | 77520 | 86580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 49040 |  | 55160 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32260 |  | 35440 | 81420 | 90480 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 56840 |  | 62960 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 104020 |  | 116080 | 18580 | 21820 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 75540 |  | 84660 | 47060 | 53240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107920 |  | 119980 | 34180 | 37420 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83340 |  | 92460 | 58760 | 64940 |

Based on Table 6.35.3-1;

-	2nd order harmonic may fall into Rx frequency of band n77

-	3rd order harmonic may fall into Rx frequencies of bands 46 and 47

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 24, 25, 30, 33, 34, 35, 36, 37, 39, 40, 65, 66, 70, n257 and n258

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.35.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.35.3-2: 2UL Band 1 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3 |

Table 6.35.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.35.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | RB restriction |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | RB restriction |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | RB restriction |

### 6.35.4	∆TIB and ∆RIB values

For DC_1A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n257 | 1 | 0 |
|  | n257 | 0 |

Table 6.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n257 | 1 | 0 |
|  | n257 | 0 |

### 6.35.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.36	DC_3A-n257

### 6.36.1	Operating bands for DC

Table 6.36.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n257 | 3 | 1710 MHz | – | 1785MHz | 1805MHz | – | 1880MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.36.2	Channel bandwidths per operating band for DC

Table 6.36.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_3A_n257D | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A_n257E | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 620 |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A_n257F | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 820 |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.36.3	Co-existence studies

Table 6.36.3-1 lists Band 3 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.36.3-1: Band 3 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3570 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5355 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24715 | 27790 |  | 28210 | 31285 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22930 |  | 26080 | 51215 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 |  | 33070 | 54710 | 60785 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1310 |  | 2185 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 21145 | 24370 |  | 77715 | 86790 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 49430 |  | 55580 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 |  | 34855 | 81210 | 90285 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 56420 |  | 62570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 104215 |  | 116290 | 19360 | 22660 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 75930 |  | 85080 | 47645 | 53870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 |  | 119785 | 33340 | 36640 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 |  | 92070 | 58130 | 64355 |

Based on Table 6.36.3-1;

-	2nd order harmonic may fall into Rx frequency of band 22, 42, 48, n77 and n78

-	3rd order harmonic may fall into Rx frequencies of bands 46

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 45, 65, 66, 70, n257 and n258

-	4th order IMD may also fall into Rx frequencies of bands n258

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.36.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.36.3-2: 2UL Band 3 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonic3 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3 |

Table 6.36.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.36.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, n77, n78 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |

### 6.36.4	∆TIB and ∆RIB values

For DC_3A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n257 | 3 | 0 |
|  | n257 | 0 |

Table 6.36.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n257 | 3 | 0 |
|  | n257 | 0 |

### 6.36.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.37	DC_3A_n258A

### 6.37.1	Operating bands for DC

Table 6.37.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n258A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n258 | 24.25 GHz | – | 27.5 GHz | 24.25 GHz | – | 27.5 GHz | TDD |

### 6.37.2	Channel bandwidths per operating band for DC

Table 6.37.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n258A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n258 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | [Yes] |  |  | Yes | Yes | Yes |  |

### 6.37.3	Co-existence studies

Table 6.37.3-1 lists Band 3 +Band n258 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.37.3-1: Band 3 and Band n258 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 24250 | 27500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3570 | 48500 | 55000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5355 | 72750 | 82500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 22465 | 25790 |  | 25960 | 29285 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 24080 |  | 20680 | 46715 | 53290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27670 |  | 31070 | 50210 | 56785 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1310 |  | 2185 | 24230 | 27520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22370 | 18895 |  | 70965 | 80790 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 51580 |  | 44930 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 29380 |  | 32855 | 74460 | 84285 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 51920 |  | 58570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 108290 |  | 95215 | 17110 | 20660 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 79080 |  | 69180 | 43145 | 49870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 98710 |  | 111785 | 31090 | 34640 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 76170 |  | 86070 | 53630 | 60355 |

Based on Table 6.37.3-1;

-	2nd order IMD may fall into own Rx of Band n258 and Rx frequencies of band n257.

-	3rd order IMD may fall into own Rx of Band n258 and Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 37, 39, 45, 65, 66, 70 and n257.

-	5th order IMD may fall into Rx frequencies of band n259.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.37.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.37.3-2: 2UL Band 3 +Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.37.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.37.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A_n258A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, n77, n78 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |

### 6.37.4	∆TIB and ∆RIB values

For DC_3A_n258A, the TIB,c and RIB values are given in the tables below.

Table 6.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n258A | 3 | 0 |
|  | n258 | 0 |

Table 6.37.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n258A | 3 | TBD0 |
|  | n258 | 0 |

### 6.37.5	MSD

There is no MSD issue for this DC configuration.

## 6.38	DC_5A_n257A

### 6.38.1	Operating bands for DC

Table 6.38.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-n257A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.38.2	Channel bandwidths per operating band for DC

Table 6.38.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  | 410 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.38.3	Co-existence studies

Table 6.38.3-1 lists Band 5 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.38.3-1: Band 5 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25651 | 28676 |  | 27324 | 30349 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27852 |  | 24802 | 52151 | 58176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28148 |  | 31198 | 53824 | 59849 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 424 |  | 1249 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27028 | 23953 |  | 78651 | 87676 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 57352 |  | 51302 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28972 |  | 32047 | 80324 | 89349 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54648 |  | 60698 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117176 |  | 105151 | 23104 | 26204 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 86852 |  | 77802 | 50453 | 56528 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106824 |  | 118849 | 29796 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81148 |  | 90198 | 55472 | 61547 |

Based on Table 6.38.3-1;

-	3rd order harmonic may fall into Rx frequencies of band 41

-	2nd order IMD may also fall into own Rx of band n257 and Rx frequencies of band n258

-	3rd order IMD may also fall into own Rx of band n257 and Rx frequencies of band n258

-	4th order IMD may also fall into own Rx of band n257 and Rx frequencies of bands n258 and n259

-	5th order IMD may also fall into Rx frequencies of bands n258 and n259

The 2nd, 3rd and 4th IMD will be impact to the own received band n257. However, the NR TDD band has no impact from dual uplink transmission since there was no simultaneous Tx/Rx transmission in the band n257.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.38.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.38.3-2: 2UL Band 5 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.38.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.38.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A_n257A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 1, 2 |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 2: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

### 6.38.4	∆TIB and ∆RIB values

For DC_5A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.38.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A_n257A | 5 | 0 |
|  | n257 | 0 |

Table 6.38.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A_n257A | 5 | 0 |
|  | n257 | 0 |

### 6.38.5	MSD

From the coexistence analysis Table 6.38.1-1, there was no harmonic/IMD problem into own Rx of bands for DC_5A_n257A UE.

## 6.39	DC_7A-n78

### 6.39.1	Operating bands for DC

Table 6.39.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.39.2	Channel bandwidths per operating band for DC

Table 6.39.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-n78A | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.39.3	Co-existence studies

Table 6.39.3-1 lists Band 7 +Band n78 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.39.3-1: Band X and Band nY UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

Based on Table 6.39.3-1;

-	There was no harmonic issues at legacy E-UTRA bands and NR new band 3.3~3.8 GHz

-	2nd order IMD may also fall into B5, B8, B12, B13, B14, B17, B18, B19, B20, B26, B27, B28, B67, B68 and TDD B44, B46 and B47.

-	3rd order IMD may also fall into B3, B11, B21, B24, B32 and TDD B45, n77

-	4th order IMD may also fall into B1, B2, B3, B4, B9, B10, B11, B21, B23, B24, B25, B30, B32, B65, B66, TDD B33, B34, B35, B36, B37, B38, B39, B40, B41, B43, B45, n77, n78, n79

-	5th order IMD may also fall into B5, B8, B12, B13, B14, B17, B18, B19, B20, B26, B27, B28, B29, B31, B67, B68, TDD B44, B46, B47 and n79.

The 4th IMD will be impact to the own received band n78. However, the NR TDD band is not any impacts from dual uplink transmission since there was no simultaneous Tx/Rx transmission in the n78 NR band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.39.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.39.3-2: 2UL Band X +Band nY harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd & 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd & 5th IMD |

Table 6.39.3-3 lists the protected bands required for the dual connectivity configuration.

| Table 6.39.3-3: Protected bands for the dual connectivity configurationE-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A-n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1,2,3 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1,2,3 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 1,2 |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 1,3,4 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 1,3,4 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 1,4 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 4: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD - 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. |  |  |  |  |  |  |  |

### 6.39.4	∆TIB and ∆RIB values

For DC_7A-n78A, the TIB,c and RIB values are defined to reuse them of the legacy LTE CA_7A-42A UE as given in the tables below.

Table 6.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-n78A | 7 | 0.5 |
|  | n78 | 0.8 |

Table 6.39.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-n78A | 7 | 0 |
|  | n78 | 0.5 |

### 6.39.5	MSD

From the coexistence analysis Table 6.39.3-1, there was no harmonic/IMD problems into the own RX frequency bands for DC_7A-n78A UE.

## 6.40	DC_7A_n258A

### 6.40.1	Operating bands for DC

Table 6.40.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A_n258A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n258 | 24.25 GHz | – | 27.5 GHz | 24.25 GHz | – | 27.5 GHz | TDD |

### 6.40.2	Channel bandwidths per operating band for DC

Table 6.40.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n258A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n258 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | [Yes] |  |  | Yes | Yes | Yes |  |

### 6.40.3	Co-existence studies

Table 6.40.3-1 lists Band 7 +Band n258 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.40.3-1: Band 7 and Band n258 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 24250 | 27500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 |  | 5140 | 48500 | 55000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 |  | 7710 | 72750 | 82500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 21680 | 25000 |  | 26750 | 30070 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22500 |  | 19110 | 45930 | 52500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29250 |  | 32640 | 51000 | 57570 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2100 |  | 2970 | 24230 | 27520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 20000 | 16540 |  | 70180 | 80000 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 50000 |  | 43360 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31750 |  | 35210 | 75250 | 85070 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 53500 |  | 60140 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 107500 |  | 94430 | 13970 | 17500 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 77500 |  | 67610 | 40790 | 47500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 99500 |  | 112570 | 34250 | 37780 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 77750 |  | 87640 | 56000 | 62710 |

Based on Table 6.40.3-1;

-	3rd order IMD may fall into Rx frequencies of bands 1, 4, 7, 10, 23, 30, 38, 40, 41, 65, 66, 69 and n257.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.40.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.40.3-2: 2UL Band 7 +Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.40.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.40.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A-n258A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 |  |

### 6.40.4	∆TIB and ∆RIB values

For DC_7A_n258A, the TIB,c and RIB values are given in the tables below.

Table 6.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A_n258A | 7 | 0 |
|  | n258 | 0 |

Table 6.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A_n258A | 7 | 0 |
|  | n258 | 0 |

### 6.40.5	MSD

There is no MSD issue for this DC configuration

## 6.41	DC_8A-n78A

### 6.41.1	Operating bands for DC

Table 6.41.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A-n78A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.41.2	Channel bandwidths per operating band for DC

Table 6.41.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A-n78A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  | 110 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.41.3	Co-existence studies

Table 6.41.3-1 lists Band 8 +Band n78 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.41.3-1: Band 8 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 |  | 1830 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 |  | 2745 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 2920 |  | 4180 | 4715 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 |  | 1470 | 5685 | 6720 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 |  | 5630 | 7480 | 8515 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 780 |  | 1015 | 3290 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 555 |  | 8985 | 10520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 5840 |  | 4770 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 |  | 6545 | 10780 | 12315 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8360 |  | 9430 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 |  | 12285 | 360 | 280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 |  | 8070 | 3855 | 4960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 |  | 16115 | 6820 | 7460 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 |  | 13230 | 9240 | 10345 |

Based on Table 6.41.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 3

-	3rd order harmonic may fall into Rx frequencies of bands 7, 41

-	2nd order IMD may also fall into Rx frequencies of band 7, 40, 41, 69, n79

-	3rd order IMD may also fall into own Rx of band 8 and Rx frequencies of bands 2, 3, 5, 6, 9, 11, 18, 19, 20, 21, 22, 24, 25, 26, 27, 28, 32, 33, 34, 35, 36, 37, 39, 42, 43, 44, 46, 47, 48, 68, 70,and  n77

-	4th order IMD may also fall into own Rx of band 8 and Rx frequencies of bands 5, 6, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 44, 46, 67, 68 and n79

-	5th order IMD may also fall into Rx frequencies of bands  n77 and n79

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.41.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.41.3-2: 2UL Band 8 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4 |

Table 6.41.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.41.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8A-n78A | E-UTRA Band 1,8, 20, 28, 34, 39, 40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | RB restriction and centre frequency range restriction |
|  | Frequency range | 860 | - | 890 | -40 | 1 | RB restriction and centre frequency range restriction |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
|  | NR Band n258 | 24250 | - | 27500 | [-5] | 100 |  |

### 6.41.4	∆TIB and ∆RIB values

For DC_8A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-n78A | 8 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 8. |  |  |

Table 6.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-n78A | 8 | 0.2 |
|  | n78 | 0.2 |
| NOTE 1: HTF is assumed for Band 8. |  |  |

### 6.41.5	MSD

Table 6.41.5-1 lists the MSD required for the 4th harmonic issue for this DC combination.

Table 6.41.5-1:MSD due to 4th harmonic issue

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHZ | 80 MHZ | 100 MHZ |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 8 | n78 | NA | 10.8 | 9.1 | 8.0 | 5.1 | 4.2 | 3.5 | 2.3 | 1.4 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |

Table 6.41.5-2 lists the UL configuration for the EN-DC configuration DC_8A-n78A.

Table 6.41.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for low band

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 |

Although Table 6.41.3-1 shows that 3rd and 4th order IMD may fall into own Rx of Band 8,only impact from 4th order IMD shall be considered. because according to R4-1711846, MSD caused by IM3 of “FLTE UL - max NR BW” should not be specified due to lower PSD of NR transmission.

Table 6.41.5-3 lists the MSD required for the dual connectivity configuration.

Table 6.41.5-3: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_8A-n78A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 52 | 3635 | N/A | TDD | N/A |

### 6.41.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.41.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.41.6-1.

Table 6.41.6-1: Band 8 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n78A | 8 | a | -3 | b | -1 | 4 | 8 | IMD |
|  | n78 | c | 1 | d | 0 |  |  |  |
| DC_8A_n78A | 8 | a | 4 | b | 0 | 4 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (6.41.6.1) and (6.41.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.41.6-1 and CBW stands for channel bandwidth.

(6.41.6.1)

(6.41.6.2)

Formula (6.41.6.3-1) and (6.41.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.41.6.3-1)

(6.41.6.3-2)

## 6.42	DC_8A-n79A

### 6.42.1	Operating bands for DC

Table 6.42.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A-n79A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000MHz | TDD |

### 6.42.2	Channel bandwidths per operating band for DC

Table 6.42.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A-n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  | 110 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.42.3	Co-existence studies

Table 6.42.3-1 lists Band 8 +Band n79 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.42.3-1: Band 8 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 880 | 915 | 4400 | 5000 |
| 2nd order harmonics frequency range (MHz) | 1760 | 1830 | 8800 | 10000 |
| 3rd order harmonics frequency range (MHz) | 2640 | 2745 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 3485 | 4120 | 5280 | 5915 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3240 | 2570 | 7885 | 9120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6160 | 6830 | 9680 | 10915 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 780 | 1015 | 4390 | 5010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 8240 | 6970 | 10560 | 11830 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2360 | 1655 | 12285 | 14120 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7040 | 7745 | 14080 | 15880 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 19120 | 16685 | 740 | 1480 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18480 | 20915 | 7920 | 8660 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 13240 | 11370 | 6055 | 7360 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14960 | 16830 | 11440 | 12745 |

Based on Table 6.42.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 3

-	3rd order harmonic may fall into Rx frequencies of bands 7 and 41

-	2nd order IMD may also fall into Rx frequencies of band 22, 42, 43, 46, 47, n77 and  n78

-	3rd order IMD may also fall into Rx frequencies of bands 5~8, 18~20, 26~28, 38, 41, 68, 69 and n79.  However, TDD band does not transmit and receive simultaneously in a single band, thus the own band of n79 would not be interfered

-	4th order IMD may also fall into Rx frequencies of bands 1~4, 9, 10, 23, 25, 30, 33~37, 39, 40, 65, 66 and 70

-	5th order IMD may also fall into Rx frequencies of bands 5, 6, 8, 11~14, 17~20, 26~28, 32, 44, 45, 67 and 68

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.42.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.42.3-2: 2UL Band 8 +Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2 |
|  | 5470 | - | 5725 | Yes |  | IMD2 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2 |

Table 6.42.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.42.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8A-n79A | E-UTRA Band 1,8,28,34,39,40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3,41,42 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | RB restriction and centre frequency range restriction |
|  | Frequency range | 860 | - | 890 | -40 | 1 | RB restriction and centre frequency range restriction |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
| NR | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
|  | NR Band n258 | 24250 | - | 27500 | [-5] | 100 |  |

### 6.42.4	∆TIB and ∆RIB values

For DC_8A-n79A, the TIB,c and RIB values are given in the tables below.

Table 6.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-n79A | 8 | 0 |
|  | n79 | 0 |

Table 6.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-n79A | 8 | 0 |
|  | n79 | 0 |

### 6.42.5	MSD

Table 6.42.5-1 lists the MSD required for the 5th harmonic issue for this DC combination.

Table 6.42.5-1:MSD due to 5th harmonic issue

| UL band | DL band | [40] MHz | 50 MHz | [60] MHZ | 80 MHZ | 100 MHZ |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | dB | dB | dB | dB | dB |
| 8 | n79 | [6.8] | 6.2 | [5.6] | 4.9 | 4.4 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |

Table 6.42.5-2 lists the UL configuration for the EN-DC configuration DC_8A-n79A.

Table 6.42.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | n79 |  |  |  |  |  |  | 25 | 25 | 25 | 25 | 25 |

Although Table 6.42.3-1 shows that 3rd and 5th order IMD may fall into own Rx of Band 8, only impact from 5th order IMD shall be considered and the MSD caused by IM3 of “FLTE UL - max NR BW” should not be specified due to lower PSD of NR transmission.

Table 6.42.5-3 lists the MSD required for the dual connectivity configuration.

Table 6.42.5-3: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_8A-n79A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |

### 6.42.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.42.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.42.6-1.

Table 6.42.6-1: Band 8 and Band n79 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n79A | 8 | a | -4 | b | -1 | 5 | 8 | IMD |
|  | n79 | c | 1 | d | 0 |  |  |  |
| DC_8A_n79A | 8 | a | 5 | b | 0 | 5 | n79 | Harmonic |
|  | n79 | c | 0 | d | -1 |  |  |  |
| DC_8A_n79A | 8 | a | 0 | b | -5 | 5 | 8 | Harmonic mixing |
|  | n79 | c | 1 | d | 0 |  |  |  |

Equations (6.42.6.1) and (6.42.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.42.6-1 and CBW stands for channel bandwidth.

(6.42.6.1)

(6.42.6.2)

Formula (6.42.6.3-1) and (6.42.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.42.6.3-1)

(6.42.6.3-2)

## 6.43	DC_8A-n258A

### 6.43.1	Operating bands for DC

Table 6.43.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A-n258A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.43.2	Channel bandwidths per operating band for DC

Table 6.43.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A-n258A | 8 | 15 | Yes | Yes |  |  |  |  |  |  | 410 |
|  | n258 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.43.3	Co-existence studies

Table 6.43.3-1 lists Band 8 +Band n258 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.43.3-1: Band 8 and Band n258 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 880 | 915 | 24250 | 27500 |
| 2nd order harmonics frequency range (MHz) | 1760 | 1830 | 48500 | 55000 |
| 3rd order harmonics frequency range (MHz) | 2640 | 2745 | 72750 | 82500 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 23335 | 26620 | 25130 | 28415 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25740 | 22420 | 47585 | 54120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 26010 | 29330 | 49380 | 55915 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 480 | 1315 | 24240 | 27510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 53240 | 46670 | 50260 | 56830 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 24860 | 21505 | 71835 | 81620 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 26890 | 30245 | 73630 | 83380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 109120 | 96085 | 20590 | 23980 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 97880 | 110915 | 27770 | 31160 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 80740 | 70920 | 45755 | 52360 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 74510 | 84330 | 51140 | 57745 |

Based on Table 6.43.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 3

-	3rd order harmonic may fall into Rx frequencies of bands 7 and 41.

-	2nd order and 4th IMD may also fall into own Rx of band n258 and Rx frequencies of band n257. However, TDD band does not transmit and receive simultaneously in a single band, thus the own band of n258 would not be interfered.

-	3rd order IMD may also fall into own Rx of band 8 and n258 and Rx frequencies of bands 5, 6, 12~14, 17~20, 26~29, 44, 67, 68 and n257.

-	5th order IMD may also fall into Rx frequencies of bands n257.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.43.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.43.3-2: 2UL Band 8 +Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.43.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.43.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8A-n258A | E-UTRA Band 1,8, 20, 28, 34, 39, 40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41,42,n78,n79 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | RB restriction and centre frequency range restriction |
|  | Frequency range | 860 | - | 890 | -40 | 1 | RB restriction and centre frequency range restriction |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |

### 6.43.4	∆TIB and ∆RIB values

For DC_8A-n258A, the TIB,c and RIB values are given in the tables below.

Table 6.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-n258A | 8 | 0 |
|  | n258 | 0 |

Table 6.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-n258A | 8 | 0 |
|  | n258 | 0 |

### 6.43.5	MSD

There is no IMD issue for own Rx range in this combination taking actual spectrum holding into account and the MSD requirement is shown in table 6.43.5-1.

Table 6.43.5-1: MSD for DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| CA_8A-n258 | 8 | N/A | N/A | N/A | N/A | N/A | FDD |
|  | n258 | N/A | N/A | N/A | N/A | N/A | TDD |

## 6.44	DC_19A-n257

### 6.44.1	Operating bands for DC

Table 6.44.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A_n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.44.2	Channel bandwidths per operating band for DC

Table 6.44.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 415 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_19A_n257D | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 415 |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_19A_n257E | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 615 |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_19A_n257F | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 815 |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.44.3	Co-existence studies

Table 6.44.3-1 lists Band 19 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.44.3-1: Band 19 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 |  | 1690 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 |  | 2535 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25655 | 28670 |  | 27330 | 30345 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 24810 |  | 27840 | 52155 | 58170 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28160 |  | 31190 | 53830 | 59845 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 430 |  | 1245 | 26485 | 29515 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23965 | 27010 |  | 78655 | 87670 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 51310 |  | 57340 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28990 |  | 32035 | 80330 | 89345 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54660 |  | 60690 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 105155 |  | 117170 | 23120 | 26180 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 77810 |  | 86840 | 50465 | 56510 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106830 |  | 118845 | 29820 | 32880 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81160 |  | 90190 | 55490 | 61535 |

Based on Table 6.44.3-1;

-	3rd order harmonic may fall into Rx frequencies of bands 41

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, n257 and n258

-	4th order IMD may also fall into Rx frequencies of bands n257 and n258

-	5th order IMD may also fall into Rx frequencies of bands n258

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.44.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.44.3-2: 2UL Band 19 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | Yes | Asia | Harmonic3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | Yes | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.44.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.44.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_19A-n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

### 6.44.4	∆TIB and ∆RIB values

For DC_19A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A_n257 | 19 | 0 |
|  | n257 | 0 |

Table 6.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A_n257 | 19 | 0 |
|  | n257 | 0 |

### 6.44.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.45	DC_20A-n78A

### 6.45.1	Operating bands for DC

Table 6.45.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A-n78A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.45.2	Channel bandwidths per operating band for DC

Table 6.45.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A-n78A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.45.3	Co-existence studies

For 2UL/2DL UE coexistence study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.45.3-1

Table 6.45.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 | 2968 | 4132 | 4662 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 | 1576 | 5738 | 6768 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 | 5524 | 7432 | 8462 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1304 | 714 | 9038 | 10568 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5796 | 6386 | 10732 | 12262 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5936 | 4876 | 8264 | 9324 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 | 12338 | 148 | 472 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 | 16062 | 6628 | 7248 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 | 8176 | 4014 | 5104 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 | 13124 | 9096 | 10186 |

Based on the table 6.45.3-1, the 4 order IMD products may fall in the own Rx of Band 20. In addition, 3rd order harmonic and 2nd, 3rd, 4th and 5th IMD products may also fall into Rx frequencies of bands 1-10, 12-14, 17-20, 25-29, 31, 33-39, 41, 44, 46-70 and n77.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.45.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.45.3-2: 2UL Band x +Band y harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD 3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD 3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD 3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD 3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD 2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD 2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4 |

Table 6.45.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.45.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_20A-n78A | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  |  |  |  |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.45.4	∆TIB and ∆RIB values

For DC_20A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-n78A | 20 | 0.6 |
|  | n78 | 0.8 |

Table 6.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-n78A | 20 | 0 |
|  | n78 | 0.5 |

### 6.45.5	MSD

Table 6.45.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.45.5-1: MSD due to IMD issue for DC_20-n78A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_20A-n78A | 20 | 850 | 5 | 25 | 810 | 21.7 | FDD | IMD44 |
|  | n78 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |

The MSD level for Band n78 by 4th harmonic from Band 20 can reuse the value from LTE CA_20A-42A. Table 6.45.5-2 shows the MSD level for DC_20A-n78A. The MSD for 40MHz was calculated based on 20MHz, since the interference are same.

Table 6.45.5-2: MSD due to harmonic issue for DC_20-n78A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 20 | n781,2 |  | 10.8 | 9.1 | 8 |  | 6 |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |

Regarding the uplink configuration, 40MHz for n78 should be considered additionally. Therefore, 20MHz with 100RB allocation for Band 20 should be added.

Table 6.45.5-3: Uplink configuration for the low band (exceptions) for E-UTRA

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | Duplex mode |
| DC_20A-n78A | 20 |  |  | 8 | 16 | 25 | 251 | 252 | FDD |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |

### 6.45.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.45.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.45.6-1.

Table 6.45.6-1: Band 20 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n78A | 20 | a | -3 | b | -1 | 4 | 20 | IMD |
|  | n78 | c | 1 | d | 0 |  |  |  |
| DC_20A_n78A | 20 | a | 4 | b | 0 | 4 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (6.45.6.1) and (6.45.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.45.6-1 and CBW stands for channel bandwidth.

(6.45.6.1)

(6.45.6.2)

Formula (6.45.6.3-1) and (6.45.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.45.6.3-1)

(6.45.6.3-2)

## 6.46	DC_21A-n257

### 6.46.1	Operating bands for DC

Table 6.46.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A_n257 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.46.2	Channel bandwidths per operating band for DC

Table 6.46.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_21A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 415 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_21A_n257D | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 415 |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_21A_n257E | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 615 |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_21A_n257F | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 815 |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.46.3	Co-existence studies

Table 6.46.3-1 lists Band 21 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.46.3-1: Band 21 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 |  | 2925.8 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 |  | 4388.7 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25037.1 | 28052.1 |  | 27947.9 | 30962.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 23574.2 |  | 26604.2 | 51537.1 | 57552.1 |
| Two-tone 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) |  | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 29395.8 |  | 32425.8 | 54447.9 | 60462.9 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1047.9 |  | 1862.9 | 26485 | 29515 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22111.3 | 25156.3 |  | 78037.1 | 87052.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 50074.2 |  | 56104.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 30843.7 |  | 33888.7 | 80947.9 | 89962.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 55895.8 |  | 61925.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 104537.1 |  | 116552.1 | 20648.4 | 23708.4 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 76574.2 |  | 85604.2 | 48611.3 | 54656.3 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107447.9 |  | 119462.9 | 32291.6 | 35351.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82395.8 |  | 91425.8 | 57343.7 | 63388.7 |

Based on Table 6.46.3-1;

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 3, 9, 11, 21, 24, 35, 45, n257 and n258

-	4th order IMD may also fall into Rx frequencies of bands n258

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.46.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.46.3-2: 2UL Band 21 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.46.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.46.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_21A-n257A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

### 6.46.4	∆TIB and ∆RIB values

For DC_21A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A_n257 | 21 | 0 |
|  | n257 | 0 |

Table 6.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A_n257 | 21 | 0 |
|  | n257 | 0 |

### 6.46.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.47	DC_28A-n257

### 6.47.1	Operating bands for DC

Table 6.47.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n257 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.47.2	Channel bandwidths per operating band for DC

Table 6.47.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n257A | 28 | 15 | Yes | Yes |  |  |  |  |  |  | 410 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_28A_n257D | 28 | 15 | Yes | Yes |  |  |  |  |  |  | 410 |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_28A_n257E | 28 | 15 | Yes | Yes |  |  |  |  |  |  | 610 |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_28A_n257F | 28 | 15 | Yes | Yes |  |  |  |  |  |  | 810 |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.47.3	Co-existence studies

Table 6.47.3-1 lists Band 28 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.47.3-1: Band 28 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 |  | 1496 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 |  | 2244 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25752 | 28797 |  | 27203 | 30248 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25004 |  | 28094 | 52252 | 58297 |
| Two-tone 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) |  | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 27906 |  | 30996 | 53703 | 59748 |
| Two-tone 3rd order IMD products | (f1_low – max BW f2) |  | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency limits (MHz) | 303 |  | 1148 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 24256 | 27391 |  | 78752 | 87797 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 51504 |  | 57594 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28609 |  | 31744 | 80203 | 89248 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54406 |  | 60496 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 105252 |  | 117297 | 23508 | 26688 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 78004 |  | 87094 | 50756 | 56891 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106703 |  | 118748 | 29312 | 32492 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 80906 |  | 89996 | 55109 | 61244 |

Based on Table 6.47.3-1;

-	2nd order harmonic may fall into Rx frequency of band 11, 21 and 45

-	3rd order harmonic may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, n257 and n258

-	4th order IMD may also fall into Rx frequencies of bands n257 and n258

-	5th order IMD may also fall into Rx frequencies of bands n257 and n258

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.47.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.47.3-2: 2UL Band 28 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.47.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.47.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28A-n257A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -36 dBm/MHz for 718-748 MHz |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -38 dBm/MHz for 718-748 MHz |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |

### 6.47.4	∆TIB and ∆RIB values

For DC_28A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A_n257 | 28 | 0 |
|  | n257 | 0 |

Table 6.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A_n257 | 28 | 0 |
|  | n257 | 0 |

### 6.47.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.48	DC_28A_n258A

### 6.48.1	Operating bands for DC

Table 6.48.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n258A | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n258 | 24.25 GHz | – | 27.5 GHz | 24.25 GHz | – | 27.5 GHz | TDD |

### 6.48.2	Channel bandwidths per operating band for DC

Table 6.48.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n258A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n258 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | [Yes] |  |  | Yes | Yes | Yes |  |

### 6.48.3	Co-existence studies

Table 6.48.3-1 lists Band 28 +Band n258 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.48.3-1: Band 28 and Band n258 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 24250 | 27500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 |  | 1496 | 48500 | 55000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 |  | 2244 | 72750 | 82500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 23502 | 26797 |  | 24953 | 28248 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26094 |  | 22754 | 47752 | 54297 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25656 |  | 28996 | 49203 | 55748 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 303 |  | 1148 | 24230 | 27520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 25391 | 22006 |  | 72002 | 81797 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 53594 |  | 47004 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 26359 |  | 29744 | 73453 | 83248 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 49906 |  | 56496 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 109297 |  | 96252 | 21258 | 24688 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 81094 |  | 71254 | 46256 | 52891 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 97703 |  | 110748 | 27062 | 30492 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 74156 |  | 83996 | 50609 | 57244 |

Based on Table 6.48.3-1;

-	3rd order IMD may fall into own Rx of Band n258 and Rx frequencies of bands 5, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68 and n257.

-	4th order IMD may fall into own Rx of Band n258 and Rx frequencies of band n257.

-	5th order IMD may fall into own Rx of Band n258 and Rx frequencies of band n257.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.48.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.48.3-2: 2UL Band 28 +Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.48.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.48.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28A_n258A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -36 dBm/MHz for 718-748 MHz |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -38 dBm/MHz for 718-748 MHz |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |

### 6.48.4	∆TIB and ∆RIB values

For DC_28A_n258A, the TIB,c and RIB values are given in the tables below.

Table 6.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A_n258A | 28 | 0 |
|  | n258 | 0 |

Table 6.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A_n258A | 28 | 0 |
|  | n258 | 0 |

### 6.48.5	MSD

There is no MSD issue for this DC configuration

## 6.49	DC_38A-n78A

### 6.49.1	Operating bands for DC

Table 6.49.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_38A-n78A | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.49.2	Channel bandwidths per operating band for DC

Table 6.49.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-38A-n78 |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.49.3	Co-existence studies

Since simultaneous transmission of LTE Band 38 and NR Band n78 are not supported, at least in Rel-15, co-existence studies are not needed.

### 6.49.4	∆TIB and ∆RIB values

For DC_38A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_38A-n78A | 38 | 0 |
|  | n78 | 0.5 |

Table 6.49.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_38A-n78A | 38 | 0.4 |
|  | n78 | 0.5 |

### 6.49.5	MSD

There is no MSD issue for this DC configuration based on the following conditions:

-	LTE Band 38 and NR Band n78 are synchronised

-	Simultaneous transmission of LTE Band 38 and NR Band n78 are not supported at least in Rel-15 timeframe

## 6.50	DC_39A-n79A

### 6.50.1	Operating bands for DC

Table 6.50.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39A-n79A | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.50.2	Channel bandwidths per operating band for DC

Table 6.50.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_39A-n79A | 39 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.50.3	Co-existence studies

Table 6.50.3-1 lists up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 39+Band n79 2UL DC.

Table 6.50.3-1: Band 39 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1880 | 1920 |  | 4400 | 5000 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3760-3840 |  |  | 8800-10000 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5640-5760 |  |  | 13200-15000 |  |  |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low |  | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7520-7680 |  |  | 17600-20000 |  |  |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5*fy_low |  | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 9400-9600 |  |  | 22000-18000 |  |  |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6*fy_low |  | 6*fy_high |
| 6th harmonics frequency limits (MHz) | 11280-11520 |  |  | 26400-3000 |  |  |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7*fy_low |  | 7*fy_high |
| 7th harmonics frequency limits (MHz) | 13160-13440 |  |  | 30800-35000 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2480-3120 |  |  | 6280-6920 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1240-560 |  |  | 6880-8120 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8160-8840 |  |  | 10680-11920 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1780-2020 |  |  | 4380-5020 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 640-1360 |  |  | 11280-13120 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 6240-4960 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10040-10760 |  |  | 15080-16920 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 12560-13840 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 18120-15680 |  |  | 3280-2520 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11240-9360 |  |  | 3040-4360 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19480-21920 |  |  | 11920-12680 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 16960-18840 |  |  | 14440-15760 |  |  |

Based on Table 6.50.3-1, the 3rd 4th order IMD product may also fall into Band 39. However it should be noted that IMD will not be an issue for TDD bands (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.50.3-2 lists if up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.50.3-2: Band 39 and Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | NO |  |  |
| Galileo | 1559 | - | 1591 | NO |  |  |
| GLONASS | 1591 | - | 1610 | NO |  |  |
| GPS | 1563 | - | 1587 | NO |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 2nd IMD |
|  | 2400 | - | 2494 | YES | Asia | 2nd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic |
|  | 5150 | - | 5350 | NO | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd Harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonic |

Table 6.50.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.50.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_39A-n79A | E-UTRA Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | N |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | N |
|  | NR Band n258 | FDL_low | - | FDL_high | [-5] | 100 |  |
| NOTE N: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

### 6.50.4	∆TIB and ∆RIB values

For DC_39A-n79A, the TIB,c and RIB values are given in Table 6.50.4-1 Table 6.50.4-2 for UEs not supporting simultaneous Tx/Rx.

Table 6.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39A-n79A | 39 | 0.3 |
|  | n79 | 0.8 |

Table 6.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_39A-n79A | 39 | 0 |
|  | n79 | 0.5 |

### 6.50.5	MSD

No IMD issue is identified for this combination.

## 6.51	DC_41A-n77A

### 6.51.1	Operating bands for DC

Table 6.51.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A-n77A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.51.2	Channel bandwidths per operating band for DC

Table 6.51.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A-n77A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.51.3	Co-existence studies

Table 6.51.3-1 lists up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 41+Band n77 2UL DC.

Table 6.51.3-1: Band 41 and Band n77 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 |  | 3300 | 4200 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 4992-5380 |  |  | 6600-8400 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488-8070 |  |  | 9900-12600 |  |  |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low |  | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9984-10760 |  |  | 13200-16800 |  |  |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5*fy_low |  | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 12480-13450 |  |  | 16500-21000 |  |  |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6*fy_low |  | 6*fy_high |
|  | 14976-16140 |  |  | 19800-25200 |  |  |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7*fy_low |  | 7*fy_high |
| 7th harmonics frequency limits (MHz) | 17472-18830 |  |  | 23100-29400 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 610-1704 |  |  | 5796-6890 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 792-2080 |  |  | 3910-5904 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8292-9580 |  |  | 9096-11090 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2396-2790 |  |  | 3280-4220 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3288-4770 |  |  | 7210-10104 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1220-3408 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788-12270 |  |  | 12396-15290 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11592-13780 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10510-14304 |  |  | 5784-7460 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4520-7608 |  |  | 912-1470 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696-19490 |  |  | 13284-14960 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892-17980 |  |  | 14088-16470 |  |  |

Based on Table 6.51.3-1, the 3rd and 4th order IMD may fall into Band 41 and Band n77. However, it should be noted that IMD will not be an issue for TDD bands (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.51.3-2 lists if up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.51.3-2: Band 41 and Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 2nd, 3rd, 4th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 2nd, 3rd, 4th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 2nd, 3rd, 4th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 2nd, 3rd, 4th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd, 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 3rd, 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, 2nd, 3rd IMD |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, , 3rd IMD |
|  | 5470 | - | 5725 | Yes |  | 3rd IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, 2nd, 3rd IMD |

Table 6.51.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.51.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41A-n77A | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |

### 6.51.4	∆TIB and ∆RIB values

Table 6.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-n77A | 41 | 0.3 |
|  | n77 | 0.8 |

Table 6.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-n77A | 41 | 0 |
|  | n77 | 0.5 |

### 6.51.5	MSD

B41 and n77 are both TDD and close in frequency and lack of isolation in the band filters will result in Rx bands noise from the Tx path of one band de-sensing the Rx of the other band. Three scenarios of desensitization are identified for this combination.

Firstly, B41 Rx is also affected due to n77 Tx leakage (4.5dB). This MSD is capatured in Table 6.51.5-1 with uplink configuration shown in Table 6.51.5-2.

Secondly, n77 Rx is affected by B41 Tx wideband noise leakage that causes MSD =8.3 dB when 2x n77 Tx = 3x B41 Rx is satisfied.

Thirdly, 3rd order harmonic mixing at B41 Rx would be caused by the leakage of 2nd harmonics of n77 Tx, which happens when 2x n77 Tx = 3x B41 Rx is satisfied..

Two MSDs for harmonic mixing are captured in Table 6.51.5-3 with uplink configuration in Table 6.51.5-4.

Note that these MSDs only happen when two bands are operated in asynchronous manner.

Table 6.51.5-1: MSD for reference sensitivity exceptions due to cross band isolation for EN-DC.

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Duplex mode |
| CA_41A-n77A | 411 | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  | TDD |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.51.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC

| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 41 | 30 | 273 | 273 | 273 | 273 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 6.51.5-3: MSD for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | n771 | N/A | 8.3 | 8.0 | 6.9 | N/A | 3.9 | 3 | 2.3 | 1.2 | 0.4 |
| n77 | 412 | 10.4 | 10.4 | 10.4 | 10.4 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.51.5-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | n77 | 15 | 25 | 25 | 25 | 25 | N/A | N/A | N/A | N/A | N/A | N/A |
| n77 | 41 | 30 | N/A | 50 | 50 | 50 | N/A | 50 | 50 | 50 | 50 | 50 |

### 6.51.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.51.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.51.6-1.

Table 6.51.6-1: Band 41 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n77A | 41 | a | 3 | b | 0 | 2 | n77 | Harmonic mixing |
|  | n77 | c | 0 | d | -1 |  |  |  |
|  | 41 | a | 0 | b | -2 | 3 | 41 | Harmonic mixing |
|  | n77 | c | 2 | d | 0 |  |  |  |

Equations (6.51.6.1) and (6.51.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.51.6-1 and CBW stands for channel bandwidth.

(6.51.6.1)

(6.51.6.2)

Formula (6.51.6.3-1) and (6.51.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.51.6.3-1)

(6.51.6.3-2)

## 6.52	DC_41A-n78A

### 6.52.1	Operating bands for DC

Table 6.52.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A-n78A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.52.2	Channel bandwidths per operating band for DC

Table 6.52.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A-n78A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.52.3	Co-existence studies

Table 6.52.3-1 lists up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 41+Band n78 2UL DC.

Table 6.52.3-1: Band 41 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 4992-5380 |  |  | 6600-7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488-8070 |  |  | 9900-11400 |  |  |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low |  | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9984-10760 |  |  | 13200-15200 |  |  |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5*fy_low |  | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 12480-13450 |  |  | 16500-19000 |  |  |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6*fy_low |  | 6*fy_high |
|  | 14976-16140 |  |  | 19800-22800 |  |  |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7*fy_low |  | 7*fy_high |
| 7th harmonics frequency limits (MHz) | 17472-18830 |  |  | 23100-26600 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 610-1304 |  |  | 5796-6490 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1192-2080 |  |  | 3910-5104 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8292-9180 |  |  | 9096-10290 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2396-2790 |  |  | 3280-3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688-4770 |  |  | 7210-8904 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1220-2680 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788-11870 |  |  | 12396-14090 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11592-12980 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10510-12704 |  |  | 6184-7460 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4520-6408 |  |  | 112-1470 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696-17890 |  |  | 13248-14560 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892-16780 |  |  | 14088-15670 |  |  |

Based on Table 6.52.3-1, the 3rd and 4th order IMD may fall into Band 41 and Band n78. However, it should be noted that IMD will not be an issue for TDD bands (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.52.3-2 lists if up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.52.3-2: Band 41 and Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd, 4th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd, 4th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd, 4th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd, 4th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd, 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 3rd, 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, 2nd, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, 2nd, 5th IMD |

Table 6.52.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.52.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41A-n78A | E-UTRA Band 1, 3, 8, 34, 39, 40, 44, 45 or NR Band n1, n8, n34, n40, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | FDL_low | - | FDL_high | [-5] | 100 |  |

### 6.52.4	∆TIB and ∆RIB values

For DC_41A-n78A, the TIB,c and RIB values are given in Table 6.52.4-1 Table 6.52.4-2.

Table 6.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-n78A | 41 | 0.3 |
|  | n78 | 0.8 |

Table 6.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-n78A | 41 | 0 |
|  | n78 | 0.5 |

### 6.52.5	MSD

Band 41 and n78 are both TDD bands and are close in frequency. For unsynchronized operation, Rx de-sensing in one band will be caused by another band due to lack of isolation in the band filters. Three scenarios of desensitization are identified for this combination as below.

-	Firstly, B41 Rx is also affected due to n78 Tx leakage (4.5dB). This MSD is capatured in Table 6.52.5-1 with uplink configuration shown in Table 6.52.5-2..

-	Secondly, n78 Rx is affected by B41 Tx wideband noise leakage that causes MSD =8.3 dB when 2x n78 Tx = 3x B41 Rx is satisfied..

-	Thirdly, 3rd order harmonic mixing at B41 Rx would be caused by the leakage of 2nd harmonics of n77 Tx, which happens when 2x n77 Tx = 3x B41 Rx is satisfied..

Two MSDs for harmonic mixing are captured in Table 6.52.5-3 with uplink configuration in Table 6.52.5-4.

Note that these MSDs only happen when two bands are operated in asynchronous manner.

Table 6.52.5-1: MSD for reference sensitivity exceptions due to cross band isolation for EN-DC

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Duplex mode |
| CA_41A-n78A | 411 | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  | TDD |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.52.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC

| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n78 | 41 | 30 | 273 | 273 | 273 | 273 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 6.52.5-3: MSD for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | n781 | N/A | 8.3 | 8.0 | 6.9 | N/A | 3.9 | 3 | 2.3 | 1.2 | 0.4 |
| n78 | 412 | 10.4 | 10.4 | 10.4 | 10.4 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.52.5-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | n78 | 15 | 25 | 25 | 25 | 25 | N/A | N/A | N/A | N/A | N/A | N/A |
| n78 | 41 | 30 | N/A | 50 | 50 | 50 | N/A | 50 | 50 | 50 | 50 | 50 |

### 6.52.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.52.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.52.6-1.

Table 6.52.6-1: Band 41 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n78A | 41 | a | 3 | b | 0 | 2 | n78 | Harmonic mixing |
|  | n78 | c | 0 | d | -1 |  |  |  |
|  | 41 | a | 0 | b | -2 | 3 | 41 | Harmonic mixing |
|  | n78 | c | 2 | d | 0 |  |  |  |

Equations (6.52.6.1) and (6.52.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.52.6-1 and CBW stands for channel bandwidth.

(6.52.6.1)

(6.52.6.2)

Formula (6.52.6.3-1) and (6.52.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.52.6.3-1)

(6.52.6.3-2)

## 6.53	DC_41A-n79A

### 6.53.1	Operating bands for DC

Table 6.53.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A-n79A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.53.2	Channel bandwidths per operating band for DC

Table 6.53.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A-n79A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.53.3	Co-existence studies

Table 6.53.3-1 lists up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 41+Band n79 2UL DC.

Table 6.53.3-1: Band 41 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 |  | 4400 | 5000 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 4992-5380 |  |  | 8800-10000 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488-8070 |  |  | 13200-15000 |  |  |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low |  | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9984-10760 |  |  | 17600-20000 |  |  |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5*fy_low |  | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 12480-13450 |  |  | 22000-25000 |  |  |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6*fy_low |  | 6*fy_high |
|  | 14976-16140 |  |  | 26400-30000 |  |  |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7*fy_low |  | 7*fy_high |
| 7th harmonics frequency limits (MHz) | 17472-18830 |  |  | 30800-35000 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1710-2504 |  |  | 6896-7690 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 8-980 |  |  | 6110-7504 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 9392-10380 |  |  | 11296-12690 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2396-2790 |  |  | 4380-5020 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2488-3670 |  |  | 10510-12504 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 5008-3420 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11888-13070 |  |  | 15696-17690 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 13792-15380 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17504-14910 |  |  | 6360-4984 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10008-7820 |  |  | 730-2512 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20096-22690 |  |  | 14384-15760 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18192-20380 |  |  | 16288-18070 |  |  |

Based on Table 6.53.3-1,

-	The 2nd harmonic of Band 41 may fall into Band n79 due to UE transmitting in the lower part of the Band 41. However, the lower part of Band 41 is only used in US where there is no plan on using n79. So the harmonic issue will not be a problem for this combination.

-	The 2nd, 3rd, 4th and 5th order IMD may fall into Band 41 and the 3rd, 4th, 5th order IMD product may fall into Band n79. However it should be noted that IMD will not be an issue for TDD bands (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.53.3-2 lists if up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.53.3-2: Band 41 and Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 2nd, 3rd, 4th, 5th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 2nd, 3rd, 4th, 5th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, 5th IMD |

Table 6.53.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.53.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41A-n79A | E-UTRA Band 1, 3, 5, 8, 9, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 or NR Band n1, n3, n8, n28, n34, n40, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | Note 1 |
|  | NR Band n257, n258 | FDL_low | - | FDL_high | [-5] | 100 |  |
| NOTE 1: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

### 6.53.4	∆TIB and ∆RIB values

For DC_41A-n79A, the TIB,c and RIB values are given in Table 6.53.4-1 Table 6.53.4-2 for UEs not supporting simultaneous Tx/Rx.

Table 6.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-n79A | 41 | 0.3 |
|  | n79 | 0.8 |

Table 6.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-n79A | 41 | 0 |
|  | n79 | 0.5 |

### 6.53.5	MSD

For DC 41A-n79, there are two operation modes, synchronous operation and asynchronous operation.

-	In synchronous operation, there is no MSD issue as TX and RX slots between B41 and n79 are aligned.

-	In asynchronous operation, there are small frequency ranges where large MSD occurs as shown in Table 6.53.5-1.

Table 6.53.5-1: MSD frequency range

| MSD Issue | Band | Frequency Low MHz | Frequency high Mhz | comments |
| --- | --- | --- | --- | --- |
| 2nd harmonic B41 TX on n79 RX | B41 | 2496 | 2506 | Small range of problematic frequencies but very large MSD (35.4 dB) |
|  | n79 | 4992 | 5000 |  |
| n79 TX on 2nd harmonic mixing B41 RX | B41 | 2496 | 2506 | Small range of problematic frequencies and significant MSD (13 dB) |
|  | n79 | 4992 | 5000 |  |

However, considering the current spectrum situation for n79 (China, Japan), the lowest edges of Band 41 actually used are far away from the range creating MSD issue.

-	The lowest usable frequency is 2545MHz in Japan,

-	The lowest usable frequency may go down to 2515 MHz in China (Note that 2500 - 2515 MHz may be considered as guard band for protection of services below 2500MHz.)

So the harmonic issue is not a problem for this combination considering the spectrum situation clarified above. No MSD needs to be specified for this combination. A note is proposed for TS38.101 Table 5.2B.3.1-1 “The frequency range below 2506 MHz for Band 41 is not used in this combination.”

### 6.53.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.53.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.53.6-1.

Table 6.53.6-1: Band 41 and Band n79 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n79A | 41 | a | 2 | b | 0 | 2 | n79 | Harmonic |
|  | n79 | c | 0 | d | -1 |  |  |  |

Equations (6.53.6.1) and (6.53.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.53.6-1 and CBW stands for channel bandwidth.

(6.53.6.1)

(6.53.6.2)

Formula (6.53.6.3-1) and (6.53.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.53.6.3-1)

(6.53.6.3-2)

## 6.54	DC_41A-n258A

### 6.54.1	Operating bands for DC

Table 6.34.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A-n258A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.54.2	Channel bandwidths per operating band for DC

Table 6.34.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n257 | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n258 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.54.3	Co-existence studies

Table 6.54.3-1 lists up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 41+Band n258 2UL DC.

Table 6.54.3-1: Band 41 and Band n258 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 |  | 24250 | 27500 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 4992-5380 |  |  | 48500-55000 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488-8070 |  |  | 72750-82500 |  |  |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low |  | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9984-10760 |  |  | 97000-110000 |  |  |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5*fy_low |  | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 12480-13450 |  |  | 121250-137500 |  |  |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6*fy_low |  | 6*fy_high |
| 6th harmonics frequency limits (MHz) | 14976-16140 |  |  | 145500-165000 |  |  |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7*fy_low |  | 7*fy_high |
| 7th harmonics frequency limits (MHz) | 17472-18830 |  |  | 169750-192500 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 21560-25004 |  |  | 26746-30190 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 18870-22508 |  |  | 45810-52504 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 29242-32880 |  |  | 50996-57690 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2296-2890 |  |  | 24230-27520 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 16180-20012 |  |  | 70060-80004 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 43120-50008 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31738-35570 |  |  | 75246-85190 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 53492-60380 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 94310-107504 |  |  | 13490-17516 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 67370-77508 |  |  | 40430-47512 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 99496-112690 |  |  | 34234-38260 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 77742-87880 |  |  | 55988-63070 |  |  |

Based on Table 6.54.3-1, the 2nd and 3rd order IMD may fall into Band n258 and 3rd order IMD product may fall into Band 41. However it should be noted that IMD will not be an issue for TDD bands (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.54.3-2 lists if up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.54.3-2: Band 41 and Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd IMD |
|  | 2400 | - | 2494 | Yes | Asia | 3rd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic |

Table 6.54.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41A-n258A | E-UTRA Band 1, 3, 8, 34, 39, 40, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.54.4	∆TIB and ∆RIB values

For DC_41A_n258A, the TIB,c and RIB values are given in the tables below.

Table 6.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A_n258 | 41 | 0 |
|  | n258 | 0 |

Table 6.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A_n258 | 41 | 0 |
|  | N258 | 0 |

### 6.54.5	MSD

No IMD issue is identified for this combination.

## 6.55	DC_42A_n257

### 6.55.1	Operating bands for DC

Table 6.55.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42A_n257 | 42 | 3400MHz | – | 3600 MHz | 3400MHz | – | 3600 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.55.2	Channel bandwidths per operating band for DC

Table 6.55.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_42A_n257A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_42A_n257D | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_42A_n257E | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 620 |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_42A_n257F | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 820 |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.55.3	Co-existence studies

Table 6.55.3-1 lists Band 42 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.55.3-1: Band 42 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 |  | 7200 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 |  | 10800 | 79500 | 88500 |
| 2nd order IMD products | (f2_low – f1_high) | (f2_high – f1_low) |  | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 22900 | 26100 |  | 29900 | 33100 |
| Two-tone 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ |  | (2*f2_low – f1_high) | (2*f2_high – f1_low) |
| IMD frequency limits (MHz) | 19300 |  | 22700 | 49400 | 55600 |
| Two-tone 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) |  | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 33300 |  | 36700 | 56400 | 62600 |
| Two-tone 3rd order IMD products | (f1_low – max BW f2) |  | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency limits (MHz) | 3000 |  | 4000 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 15700 | 19300 |  | 75900 | 85100 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 45800 |  | 52200 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 36700 |  | 40300 | 82900 | 92100 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 59800 |  | 66200 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 102400 |  | 114600 | 12100 | 15900 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 72300 |  | 81700 | 42200 | 48800 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 109400 |  | 121600 | 40100 | 43900 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 86300 |  | 95700 | 63200 | 69800 |

Based on Table 6.55.3-1;

-	2nd order IMD may also fall into Rx frequencies of bands n258

-	3rd order IMD may also fall into Rx frequencies of bands 22, 42, 43, 48, n77, n78, n257 and n258

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.55.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.55.3-2: 2UL Band 42 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.55.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.55.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_42A-n257A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.55.4	∆TIB and ∆RIB values

For DC_42A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42A_n257 | 42 | 0 |
|  | n257 | 0 |

Table 6.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42A_n257 | 42 | 0 |
|  | n257 | 0 |

### 6.55.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.56	DC_1A_n28

### 6.56.1	Operating bands for DC

Table 6.56.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n281 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.56.2	Channel bandwidths per operating band for DC

Table 6.56.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n28 |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 40 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.56.3	Co-existence studies

Table 6.56.3-1 lists 1A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.56.3-1: Band 1 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1187 – 1277 |  |  | 2623 – 2713 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 454 – 574 |  |  | 3107 – 3257 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3326 – 3446 |  |  | 4543 – 4693 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 129 – 279 |  |  | 5027 – 5237 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2374 – 2554 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4029 – 4179 |  |  | 6463 – 6673 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 5246 – 5426 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6947 – 7217 |  |  | 832 – 1012 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4294 – 4534 |  |  | 1641 – 1851 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 – 8653 |  |  | 4732 – 4912 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 – 7406 |  |  | 5949 – 6159 |  |  |

Based on Table 6.56.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 32, 45, 50, 51, 75, 76, and n77

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 46, 47, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 7 and 41

-	3rd order IMD may fall into Rx frequencies of Band 2, 12, 13,  17, 25, 29, 31, 33, 35, 36, 37, 39, 42, 44, 67, 68,  n77, and n78

-	4th order IMD may fall into Rx frequencies of Band 30, 40, 41, 46, and n77

-	5th order IMD may fall into Rx frequencies Band 3, 5, 6, 8, 9, 18, 19, 26, 27, 35, and n79

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Tables 6.56.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.56.3-2: 2UL Band 1 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD4 |

Table 6.X.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.X.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_1A-n28A | E-UTRA Band 18, 19, 27, 31, 32, 72 NR band n5, n7,n8, n20, n26, n38, n40, n41, n50, n51, n74 | FDL_low |  | - | FDL_high | -50 |  | 1 |  |
|  | E-UTRA Band42, 43 NR band n78, n75, n76 | FDL_low |  | - | FDL_high | -50 |  | 1 | 2 |
|  | NR band n3, n34 | FDL_low |  | - | FDL_high | -50 |  | 1 | 3 |
|  | E-UTRA Band 11, 21 | FDL_low |  | - | FDL_high | -50 |  | 1 | 5, 21 |
|  | E-UTRA Band 65 NR band n1 | FDL_low |  | - | FDL_high | -50 |  | 1 | 5, 6 |
|  | Frequency range | 470 |  | - | 694 | -42 |  | 8 | 3, 22 |
|  | Frequency range | 470 |  | - | 710 | -26.2 |  | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 |  | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 |  | 1 |  |
|  | Frequency range | 662 |  | - | 694 | -26.2 |  | 6 | 3 |
|  | Frequency range | 1880 |  |  | 1895 | -40 |  | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 |  | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 |  | 5 | 3, 12, 13 |
|  | Frequency range | 1839.9 |  | - | 1879.9 | -50 |  | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 |  | 0.3 | 5, 7 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in TS 36.101 [2] Table 6.6.3.1-1 and TS 36.101 [2] Table 6.6.3.1A-1 in from the edge of the aggregated channel bandwidth. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 7: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 12: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE13: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 21: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 22: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. |  |  |  |  |  |  |  |  |  |  |

### 6.56.4	∆TIB and ∆RIB values

For DC_1A-n28A, the TIB,c and RIB values are given in the tables below.

Table 6.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-n28A | 1 | 0.3 |
|  | n28 | 0.6 |

Table 6.56.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-n28A | 1 | 0 |
|  | n28 | 0.2 |

### 6.56.5	MSD

Table 6.56.5-1 lists the MSD required for the dual connectivity configuration due to 3rd harmonic of Band n28, reusing values specified in LTE for CA_1A-28A..

Table 6.56.5-1: MSD due to 3rd harmonic issue for DC_1A-n28A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
|  |  | dB | dB | dB | dB |
| n28 | 11,2,3 | [10.2] | [7.6] | [6.2] | [5.3] |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 3: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |

Table 6.56.5-2 Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| DC_1A-n28A | n28 | 8 | 16 | 25 | 25 | FDD |

### 6.56.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.56.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.56.6-1.

Table 6.56.6-1: Band 1 and Band n28 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n28A | 1 | a | 0 | b | -1 | 3 | 1 | Harmonic |
|  | n28 | c | 3 | d | 0 |  |  |  |

Equations (6.56.6.1) and (6.56.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.56.6-1 and CBW stands for channel bandwidth.

(6.56.6.1)

(6.56.6.2)

Formula (6.56.6.3-1) and (6.56.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.56.6.3-1)

(6.56.6.3-2)

## 6.57	DC_3A_n7A

### 6.57.1	Operating bands for DC

Table 6.57.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n7 | 3 | 1710 MHz | – | 1785MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

### 6.57.2	Channel bandwidths per operating band for DC

Table 6.57.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 70 |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |

### 6.57.3	Co-existence studies

For Band 3 +Band n7 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis were calculated and listed in Table 6.57.3-1.

Table 6.57.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

Based on Table 6.57.3-1;

-	There is no harmonic issue at legacy E-UTRA bands and NR band n7

-	2nd order IMD may also fall into B12, B13, B14, B17, B20, B26, B27 B28, B29, B67, B68 and TDD B44, n79.

-	3rd order IMD may also fall into B5, B6, B8, B18, B19, B26, B27 and TDD B42, B46, B47, n77, n78.

-	4th order IMD may also fall into B7, B11, B21, B32, B69, B74, B75, B76 and TDD B38, B41, B45, B46, B47, B50, B51.

-	5th order IMD may also fall into TDD n77, n79.

The 4th IMD will be impact to the own received band n7.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.57.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.57.3-2: 2UL Band 3 +Band n7 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia | - |
| ISM band (5GHz) | 5150 | - | 5925 | - | US | - |
|  | 5150 | - | 5350 | Yes | Europe | 3rd order harmonic B3 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 3rd order harmonic B3, IMD4 |

Table 6.57.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.57.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A_n7A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 NR Band n1, n5, n7, n8, n20, n28, n50, n51, n74, n75, n76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 3, 14 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in TS 36.101 [2] Table 6.6.3.1-1 and TS 36.101 [2] Table 6.6.3.1A-1 in from the edge of the aggregated channel bandwidth. NOTE13: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 14: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

### 6.57.4	∆TIB and ∆RIB values

For DC_3A-n7A, the TIB,c and RIB values are given in the tables below.

Table 6.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-n7A | 3 | 0.5 |
|  | n7 | 0.5 |

Table 6.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-n7A | 3 | 0 |
|  | n7 | 0 |

### 6.57.5	MSD

Table 6.57.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.57.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | 4th IMD |
|  | n7 | 2535 | 10 | 52 | 2655 | [10.25] | FDD |  |

6.57.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.57.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.57.6-1.

Table 6.57.6-1: Band 3 and Band n7 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n7A | 3 | a | 3 | b | 0 | 4 | n7 | IMD |
|  | n7 | c | -1 | d | -1 |  |  |  |

Equations (6.57.6.1) and (6.57.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.57.6-1 and CBW stands for channel bandwidth.

(6.57.6.1)

(6.57.6.2)

Formula (6.57.6.3-1) and (6.57.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.57.6.3-1)

(6.57.6.3-2)

## 6.58	DC_3A_n28

### 6.58.1	Operating bands for DC

Table 6.58.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n281 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.58.2	Channel bandwidths per operating band for DC

Table 6.58.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n28 |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 30 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.58.3	Co-existence studies

Table 6.58.3-1 lists 3A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.58.3-1: Band 3 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 977 – 1082 |  |  | 2413 – 2518 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 244 – 379 |  |  | 2687 – 2867 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3116 – 3251 |  |  | 4123 – 4303 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 1690 – 1805 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 324 – 489 |  |  | 4397 – 4652 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1954 – 2164 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 – 3984 |  |  | 5833 – 6088 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 4826 – 5036 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6107 – 6437 |  |  | 1027 – 1222 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3664 – 3949 |  |  | 1221 – 1461 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 – 7873 |  |  | 4522 – 4717 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 – 6821 |  |  | 5529 – 5769 |  |  |

Based on Table 6.58.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band  22, 32, 42, 45, 48, 50, 51, 75, 76, n77, and n78

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 46, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 41

-	3rd order IMD may fall into Rx frequencies of Band 7, 12, 13, 17,  29, 41, 44, 67, 68, and n77

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 4, 10, 25, 31, 36,  65, 66, 70, n77, and n79

-	and 5th order IMD may fall into Rx frequencies of Band 32, 43, 45, 46, 50, 51, 75, 76, n77, n78, and n79

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.58.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.58.3-2: 2UL Band 3 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | H3 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD5 |

Table 6.58.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.58.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_3A-n28A | E-UTRA Band 42, 43, 65 NR band n1, n50, n51, n74, n75, n76, n78 | FDL_low |  | - | FDL_high | -50 |  | 1 | 2 |
|  | NR band n1 | FDL_low |  | - | FDL_high | -50 |  | 1 | 5, 6 |
|  | NR band n3 | FDL_low |  | - | FDL_high | -50 |  | 1 | 3 |
|  | E-UTRA Band 27, 31, 72 NR band n5, n7, n8, n20, n26, n34, n38, n40, n41 | FDL_low |  | - | FDL_high | -50 |  | 1 |  |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 |  | 1 | 10 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 |  | 0.3 | 10 |
|  | Frequency range | 470 |  | - | 710 | -26.2 |  | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 |  | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 |  | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 |  | 0.3 | 4, 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in TS 36.101 [2] Table 6.6.3.1-1 and TS 36.101 [2] Table 6.6.3.1A-1 in from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE10: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. |  |  |  |  |  |  |  |  |  |  |

### 6.58.4	∆TIB and ∆RIB values

For DC_3A-n28A, the TIB,c and RIB values are given in the tables below.

Table 6.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-n28A | 3 | 0.3 |
|  | n28 | 0.3 |

Table 6.58.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-n28A | 3 | 0 |
|  | n28 | 0 |

### 6.58.5	MSD

Based on co-existence studies of band 3 + band n28, there is no harmonic and IMD falling into own receive bands. Thus, no MSD is required for this combination.

## 6.59	DC_5A-n78A

### 6.59.1	Operating bands for DC

Table 6.59.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-n78A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.59.2	Channel bandwidths per operating band for DC

Table 6.59.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A-n78A |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  | 110 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.59.3	Co-existence studies

Tables 6.59.3-1 lists Band 5A + Band n78A 2UL DC 2nd, 3rd, 4th and 5th  order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.59.3-1: Band 5 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 |  | 3396 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 | 2976 |  | 4124 | 4649 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2152 |  | 1602 | 5751 | 6776 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4948 |  | 5498 | 7424 | 8449 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 724 |  | 949 | 3290 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1328 | 753 |  | 9051 | 10576 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 5952 |  | 4902 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5772 |  | 6347 | 10724 | 12249 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8248 |  | 9298 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14376 |  | 12351 | 96 | 504 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9752 |  | 8202 | 4053 | 5128 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 |  | 16049 | 6596 | 7196 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 |  | 13098 | 9072 | 10147 |

Based on Table 6.59.3-1,

-	3rd order harmonic may fall into Rx frequencies of band 41

-	4th order harmonic may fall into Rx frequencies of bands n77 and n78.

-	5th order harmonic may fall into Rx frequencies of band n77.

-	2nd order IMD may also fall into own Rx of band n78 and Rx frequencies of bands 7, 38, 41, 69 and n79

-	3rd order IMD may also fall into own Rx of bands 5 and n78 and Rx frequencies of bands 1, 2, 3, 4, 6, 8, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 29, 33, 34, 35, 36, 37, 39, 42, 43, 44, 46, 47, 48, 65, 66, 67, 68, 70, n77 and n79

-	4th order IMD may also fall into own Rx of band 5 and Rx frequencies of bands 6, 8, 13, 14, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, 68 and n79

-	5th order IMD may also fall into Rx frequencies of bands 31, n77 and n79

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.59.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.59.3-2: 2UL Band 5 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3 |

Table 6.59.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.59.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 1, 2, 3 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 1, 2, 3 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 1, 3 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 3: The requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD – 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 5: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 6: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

### 6.59.4	∆TIB and ∆RIB values

For DC_5A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.59.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-n78A | 5 | 0.6 |
|  | n78 | 0.8 |

Table 6.59.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-n78A | 5 | 0.2 |
|  | n78 | 0.5 |

### 6.59.5	MSD

Table 6.59.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.59.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 ¦fB78-3*fB5¦ |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |

Table 6.59.5-2: MSD due to 4th harmonic for DC_5A-n78A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 5 | n781,2 |  | 10.5 | 8.9 | 7.8 |  | 5.4 |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.59.5-3 lists the UL configuration for the EN-DC configuration DC_5A-n78A.

Table 6.59.5-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | n78 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |  |  |

## 6.60	DC_7A_n28

### 6.60.1	Operating bands for DC

Table 6.60.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A_n281 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.60.2	Channel bandwidths per operating band for DC

Table 6.60.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n28 |  | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  | 40 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.60.3	Co-existence studies

Table 6.60.3-1 lists 7A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.60.3-1: Band 7 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1767 – 1867 |  |  | 3203 – 3303 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1034 – 1164 |  |  | 4267 – 4437 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3906 – 4036 |  |  | 5703 – 5873 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 301 – 461 |  |  | 6767 – 7007 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3534 – 3734 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4609 – 4769 |  |  | 8203 – 8443 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 6406 – 6606 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9267 – 9577 |  |  | 242 – 432 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6034 – 6304 |  |  | 2801 – 3031 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10703 – 11013 |  |  | 5312 – 5502 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8906 – 9176 |  |  | 7109 – 7339 |  |  |

Based on Table 6.60.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band  32, 45, 50, 51, 75, and 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 3, 35, n77, and n78

-	3rd order IMD may fall into Rx frequencies of Band 12, 13, 17, 29, 38, 41, 44, 46, 47, 67, 68, 69, n77, and n79

-	4th order IMD may fall into Rx frequencies of Band 22, 42, 43, 48, n77, n78, and n79

-	5th order IMD may fall into Rx frequencies of Band  46

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.60.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.60.3-2: 2UL Band 7 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD5 |

Table 6.60.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.60.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_7A-n28A | E-UTRA Band 27, 31, 72 NR band n2, n3, n5, n7, n8, n20, n26, n34, n40 | FDL_low |  | - | FDL_high | -50 |  | 1 |  |
|  | E-UTRA Band 4, 10, 42, 43, 65 NR band n1, n50, n51, n66, n74, n75, n76, n78 | FDL_low |  | - | FDL_high | -50 |  | 1 | 2 |
|  | NR band n1 | FDL_low |  | - | FDL_high | -50 |  | 1 | 5, 6 |
|  | Frequency range | 758 |  | - | 773 | -32 |  | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 |  | 1 |  |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 |  | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 |  | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 |  | 1 | 3, 14 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in TS 36.101 [2] Table 6.6.3.1-1 and TS 36.101 [2] Table 6.6.3.1A-1 in from the edge of the aggregated channel bandwidth. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE13: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 14: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |  |  |

### 6.60.4	∆TIB and ∆RIB values

For DC_7A-n28A, the TIB,c and RIB values are given in the tables below.

Table 6.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-n28A | 7 | 0.3 |
|  | n28 | 0.3 |

Table 6.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-n28A | 7 | 0 |
|  | n28 | 0 |

### 6.60.5	MSD

Based on co-existence studies of band 7 + band n28, there is no harmonic and IMD falling into own receive bands. Thus, no MSD is required for this combination.

## 6.61	DC_7A-n257A

### 6.61.1	Operating bands for DC

Table 6.61.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-n257A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.61.2	Channel bandwidths per operating band for DC

Table 6.61.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.61.3	Co-existence studies

Table 6.61.3-1 lists Band 7 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.61.3-1: Band 7 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 |  | 5140 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 |  | 7710 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 23930 | 27000 |  | 29000 | 32070 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 21360 |  | 24500 | 50430 | 56500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 31500 |  | 34640 | 55500 | 61570 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2100 |  | 2970 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22000 | 18790 |  | 76930 | 86000 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 54000 |  | 47860 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 34000 |  | 37210 | 82000 | 91070 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 58000 |  | 64160 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 115500 |  | 103430 | 16220 | 19500 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 83500 |  | 74360 | 45290 | 51500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 108500 |  | 120570 | 36500 | 39780 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 84500 |  | 93640 | 60500 | 66710 |

Based on Table 6.61.3-1;

-	3rd order IMD may fall into Rx frequencies of bands 1, 4, 7, 10, 23, 30, 34, 40, 41, 65, 66, 69, n257, n258 and n259.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.61.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.61.3-2: 2UL Band 7 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.61.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.61.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 |  |

### 6.61.4	∆TIB and ∆RIB values

For DC_7A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.61.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-n257A | 7 | 0 |
|  | n257 | 0 |

Table 6.61.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-n257A | 7 | 0 |
|  | n257 | 0 |

### 6.61.5	MSD

No IMD problems are expected at both LTE band and NR band at FR2. Hence, not necessary for study on IMD problems for LTE-NR DC UE between sub6GHz and FR2. And up to 7th order harmonics are not impact to Rx frequency of both LTE and NR bands.

## 6.62	DC_66_n257

### 6.62.1	Operating bands for DC

Table 6.62.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n257 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2220 MHz | FDD |
|  | n257 | 26.5 GHz | – | 29.5 GHz | 26.5 GHz | – | 29.5 GHz | TDD |

### 6.62.2	Channel bandwidths per operating band for DC

Table 6.62.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n257A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_66A_n257G | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |
|  | n257 | 60 | See CA_n257G CA bandwidth combination set 0 for Fallback group 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n257H | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  | n257 | 60 | See CA_n257H CA bandwidth combination set 0 for Fallback group 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n257I | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n257 | 60 | See CA_n257I CA bandwidth combination set 0 for Fallback group 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n257J | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  | n257 | 60 | See CA_n257J CA bandwidth combination set 0 for Fallback group 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n257K | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |
|  | n257 | 60 | See CA_n257K CA bandwidth combination set 0 for Fallback group 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n257L | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |
|  | n257 | 60 | See CA_n257L CA bandwidth combination set 0 for Fallback group 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n257M | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |
|  | n257 | 60 | See CA_n257M CA bandwidth combination set 0 for Fallback group 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.62.2-2: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band (non-contiguous)

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_66A_n257(2A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 | See CA_n257(2A) CA bandwidth combinations in TS 38.101-2 [4]. |  |  |  |  |  |  |  |  |  |  | 420 |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |

### 6.62.3	Co-existence studies

Table 6.62.3-1 lists Band 66 +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.62.3-1: Band 66 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 27790 | 24720 |  | 28210 | 31280 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 26080 |  | 22940 | 51220 | 57290 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 29920 |  | 33060 | 54710 | 60780 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
|  | -55580 |  | -49440 | 56420 | 62560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
|  | -24370 | -21160 |  | 77720 | 86790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
|  | 31630 |  | 34840 | 81210 | 90280 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | -116290 |  | -104220 | 19380 | 22660 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 107710 |  | 119780 | 33340 | 36620 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | -85080 |  | -75940 | 47660 | 53870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 82920 |  | 92060 | 58130 | 64340 |

Based on Table 6.62.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77 and 78.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD may fall into own Rx frequencies of band n257  (which can be disregarded, since no self-interference in TDD) as well as Rx frequencies of band n258.

-	3rd order IMD may fall into Rx frequencies of band n258.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.62.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.62.3-2: 2UL Band 66 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.62.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.62.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66_n257 | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.62.4	∆TIB and ∆RIB values

For DC_66_n257, the TIB,c and RIB values are given in the tables below.

Table 6.62.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n257 | 66 | 0 |
|  | n257 | 0 |

Table 6.62.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n257 | 66 | 0 |
|  | n257 | 0 |

### 6.62.5	MSD

There is no MSD issue for this DC configuration.

## 6.63	DC_8A-n77A

### 6.63.1	Operating bands for DC

Table 6.63.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A-n77A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.63.2	Channel bandwidths per operating band for DC

Table 6.63.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A-n77A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.63.3	Co-existence studies

Table 6.63.3-1 lists Band 8 +Band n77 2UL DC 2nd to 6th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.63.3-1: Band 8 and Band n77 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 |  | 1830 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 |  | 2745 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 |  | 3660 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 |  | 4575 | 16500 | 21000 |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 |  | 5490 | 19800 | 25200 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 3320 |  | 4180 | 5115 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1470 |  | 2440 | 5685 | 7520 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 |  | 6030 | 7480 | 9315 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 555 | 1560 |  | 8985 | 11720 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 6640 |  | 4770 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 |  | 6945 | 10780 | 13515 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8360 |  | 10230 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15920 |  | 12285 | 680 | 360 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10840 |  | 8070 | 5760 | 3855 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 |  | 17715 | 6820 | 7860 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 |  | 14430 | 9240 | 11145 |

Based on Table 6.63.3-1;

-	22nd order harmonic may fall into Rx frequencies of bands 3,

-	3rd order harmonic may fall into Rx frequencies of bands 7 and 41,

-	4th order harmonic may fall into Rx frequencies of bands 22, 42, 43, 48, 49, n77 and n78,

-	5th order harmonic may fall into Rx frequencies of bands n79,

-	6th order harmonic may fall into Rx frequencies of bands 46 and n258,

-	2nd order IMD may also fall into Rx frequencies of band 7, 38, 40, 41, 69, n77, n78 and n79,

-	3rd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 22, 24, 25, 30, 32, 33, 34, 35, 36, 37, 39, 40, 46, 47, 50, 51, 65, 66, 70, 74 and 75,

-	4th order IMD may also fall into Rx frequencies of bands 5, 6, 8, 11, 12, 13, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 46, 47, 50, 51, 67, 68, 71, 74, 75, 76 and n79,

-	5th order IMD may also fall into Rx frequencies of bands 31, 46, 71, 72, n77 and n79.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.63.3-2 lists if up to 6th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.63.3-2: 2UL Band 8 +Band n77 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3,4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3,4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2,3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2,3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H6, IMD3,4,5 |
|  | 5150 | - | 5350 | Yes | Europe | H6, IMD3,4,5 |
|  | 5470 | - | 5725 | Yes |  | H6, IMD3,4,5 |
|  | 5150 | - | 5825 | Yes | Asia | H6, IMD3,4,5 |

Table 6.63.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.63.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8A-n77A | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. |  |  |  |  |  |  |  |

### 6.63.4	∆TIB and ∆RIB values

For DC_8A-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-n77A | 8 | 0.6 |
|  | n77 | 0.8 |

Table 6.63.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-n77A | 8 | 0.2 |
|  | n77 | 0.5 |

### 6.63.5	MSD

Table 6.63.5-1 lists the MSD required for the dual connectivity configuration. As mentioned above, between Band 8 and n77, the 4th order harmonic of Band 8 Tx falls down to n77 Rx and IMD4 falls down to Band 8. (Note that IMDs falling down to n77 do not affect n77 Rx since n77 is TDD.)  The results are as follows:

Table 6.63.5-1:MSD due to harmonic issue

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHZ | 80 MHZ | 100 MHZ |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 8 | n77 | NA | 10.8 | 9.1 | 8.0 | 5.1 | 4.2 | 3.5 | 2.3 | 1.4 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |

Table 6.63.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | n77 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 |

Table 6.63.5-3: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_8A-n77A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n77 | 3635 | 10 | 52 | 3635 | N/A | TDD | H4 |

### 6.63.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.63.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.63.6-1.

Table 6.63.6-1: Band 8 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n77A | 8 | a | -3 | b | -1 | 4 | 8 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_8A_n77A | 8 | a | 4 | b | 0 | 4 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |

Equations (6.63.6.1) and (6.63.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.63.6-1 and CBW stands for channel bandwidth.

(6.63.6.1)

(6.63.6.2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.63.6.3-1)

(6.63.6.3-2)

## 6.64	DC_8A-n257A

### 6.64.1	Operating bands for DC

Table 6.64.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A-n257A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.64.2	Channel bandwidths per operating band for DC

Table 6.64.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.64.3	Co-existence studies

Table 6.64.3-1 lists Band 8 +Band n257 2UL DC 2nd to 6th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.64.3-1: Band 8 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 |  | 1830 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 |  | 2745 | 79500 | 88500 |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 |  | 3660 | 106000 | 118000 |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 |  | 4575 | 132500 | 147500 |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 |  | 5490 | 159000 | 177000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25585 | 28620 |  | 27380 | 30415 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 24670 |  | 27740 | 52085 | 58120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28260 |  | 31330 | 53880 | 59915 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23775 | 26860 |  | 78585 | 87620 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 51170 |  | 57240 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 29140 |  | 32245 | 80380 | 89415 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54760 |  | 60830 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 105085 |  | 117120 | 22840 | 25980 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 77670 |  | 86740 | 50255 | 56360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106880 |  | 118915 | 30020 | 33160 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81260 |  | 90330 | 55640 | 61745 |

Based on Table 6.64.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 3,

-	3rd order harmonic may fall into Rx frequencies of bands 7 and 41,

-	4th order harmonic may fall into Rx frequencies of bands 22, 42, 43, 48, 49, n77 and n78,

-	5th order harmonic may fall into Rx frequencies of band n79,

-	6th order harmonic may fall into Rx frequencies of band 46,

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258,

-	3rd order IMD may also fall into Rx frequencies of bands n257 and n258,

-	4th order IMD may also fall into Rx frequencies of bands n257 and n258

-	5th order IMD may also fall into Rx frequencies of band n258.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.64.3-2 lists if up to 6th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.64.3-2: 2UL Band 8 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H6 |
|  | 5150 | - | 5350 | Yes | Europe | H6 |
|  | 5470 | - | 5725 | Yes |  | H6 |
|  | 5150 | - | 5825 | Yes | Asia | H6 |

Table 6.64.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.64.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8A-n257A | E-UTRA Band 1, 28, 34, 38, 69, 74, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. |  |  |  |  |  |  |  |

### 6.64.4	∆TIB and ∆RIB values

For DC_8A-n257A, the TIB,c and RIB are proposed as follows, assuming separate RF chains are employed for these bands.

Table 6.64.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-n257A | 8 | 0 |
|  | n257 | 0 |

Table 6.64.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-n257A | 8 | 0 |
|  | n257 | 0 |

### 6.64.5	MSD

As mentioned above, between Band 8 and n257, there is no harmonic relation and IMD2-4 fall down to Band n257. But these IMDs do not affect n257 Rx since n257 is TDD. Thus, there is no need to address MSD due to IMDs.

## 6.65	DC_25A_n41A

### 6.65.1	Operating bands for DC

Table 6.65.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC25A_n41A | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 GHz | – | 2690 MHz | 2496 GHz | – | 2690 MHz | TDD |

### 6.65.2	Channel bandwidths per operating band for DC

Table 6.65.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |  | Maximum aggregated bandwidth [MHz] |
| DC_25A_n41A | 25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 120 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | [Yes |  |  |

### 6.65.3	Co-existence studies

Table 6.65.3-1 lists Band 25 + Band n41 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.65.3-1: Band 25 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1915 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3830 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5745 | 7488 | 8070 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 840 | 581 | 4346 | 4605 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1010 | 1334 | 3077 | 3530 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6196 | 6520 | 6842 | 7295 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1680 | 1162 | 8692 | 9210 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2860 | 3249 | 5573 | 6220 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8046 | 8435 | 9338 | 9985 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8910 | 8069 | 5164 | 4710 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11834 | 12675 | 9896 | 10350 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4370 | 3658 | 753 | 170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11188 | 11900 | 10542 | 11125 |

Based on Table 6.65.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 43, 46, n77, n78 and n79

-	3rd order harmonic may fall into Rx frequencies of band 46

-	2nd order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 28, 29, 44, 67, 68, 71 and n79.

-	3rd order IMD may fall into Rx frequencies of bands 22, 42, n77 and n78.

-	4th order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 47, 74, 75, 76,

-	5th order IMD may fall into Rx frequencies of bands 12, 13, 17, 29, 31, 43, 44, 46, 48, 67, 68, 71, 72, n77, n78, n79

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.65.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.65.3-2: 2UL Band 25 +Band n41 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  | IMD4 |
| Galileo | 1559 | - | 1591 | No |  | IMD4 |
| GLONASS | 1591 | - | 1610 | No |  | IMD4 |
| GPS | 1563 | - | 1587 | No |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe | IMD3 |
|  | 2400 | - | 2494 | No | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, 3rd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, IMD5 |
|  | 5470 | - | 5725 | Yes |  | 3rd harmonic, IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, 3rd harmonic, IMD4, IMD5 |

Table 6.65.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.65.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_25A_n41A | NR band n5, n28, n66, n71 E-UTRA/NR Band 4, 10, 12, 13 , 14, 17, 24, 26, 27, 29, 30, 42, 45, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n2 E-UTRA/NR Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | EUTRA/NR Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

### 6.65.4	∆TIB and ∆RIB values

For DC_25A_n41A, the TIB,c and RIB values are given in the tables below.

Table 6.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_25A_n41A | 25 | 0.5 |
|  | n41 | 0.5 |

Table 6.65.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_25A_n41A | 25 | 0 |
|  | n41 | 0 |

### 6.65.5	MSD

The MSD due to cross band isolation is analysed based on assumptions in R4-1902111 and adopt shared antenna FE architecture (R4-1902457). MSD numbers are proposed in table 6.65.5-1.

Table 6.65.5-1: Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

|  |  | E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n41 | 25 |  | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |

Table 6.65.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

|  |  | E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n41 | 25 |  | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |

## 6.66	DC_26A_n41A

### 6.66.1	Operating bands for DC

Table 6.66.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_26A_n41A | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n41 | 2496 GHz | – | 2690 MHz | 2496 GHz | – | 2690 MHz | TDD |

### 6.66.2	Channel bandwidths per operating band for DC

Table 6.66.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |  | Maximum aggregated bandwidth [MHz] |
| DC_26A_n41A | 26 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 120 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.66.3	Co-existence studies

Table 6.66.3-1 lists Band 26 + Band n41 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.66.3-1: Band 26 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7488 | 8070 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1876 | 1647 | 3310 | 3539 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1062 | 798 | 4143 | 4566 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4124 | 4388 | 5806 | 6229 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3752 | 3294 | 6620 | 7078 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 248 | 51 | 6639 | 7256 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4938 | 5237 | 8302 | 8919 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9946 | 9135 | 900 | 566 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10798 | 11609 | 5752 | 6086 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6442 | 5790 | 2445 | 2938 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9116 | 9768 | 7434 | 7927 |

Based on Table 6.66.3-1;

-	2nd order harmonics may fall into Rx frequencies of bands  46,  n79

-	3rd order harmonics may fall into Rx frequencies of bands 41

-	2nd order IMD may fall into Rx frequencies of bands 3,9, 22, 35, 42, n77 and n78

-	3rd order IMD may fall into Rx frequencies of band 5, 6, 8, 18, 19, 20, 26, 27, 28, 44, 46, 47, n77 and n79

-	4th order IMD may fall into Rx frequencies of band 22, 42, 43, 46, n77, n78, n79

-	5th order IMD may fall into Rx frequencies of band 5, 6, 7, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 38, 41, 44, 46, 48, 67. 68, 69, 71

There is a third order harmonic of the Band 26 uplink in n41. However, if the allocation for Band 26 is limited to the lowest 10 MHz of Band 26 (814-824/859-869 MHz) the harmonic no longer falls in n41. There is a third order IMD in the downlink frequencies of Band 26. However, if the Band 26 allocation is limited to the low 10 MHz of Band 26 (814-824/859-869 MHz) and the allocation for n41 is limited to the high 170 MHz of the band (2520 – 2690 MHz) the IM3 does not fall into the 10 MHz allocation for Band 26. There is also a 5th order IMD into n41, but since n41 is a TDD band, the UE wont’t be transmitting and receiving on n41 simultaneously.

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.66.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.66.3-3: Band 26 and Band n41 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd harmonic, IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | 3rd harmonic, IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD3, IMD4, IMD5 |

Table 6.66.3-4 lists the protected bands required for the dual connectivity configuration.

Table 6.66.3-4: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_26A_n41A | E-UTRA/NR Band 1, 2, 3, 4, 5, 10, 12, 13 , 14, 17, 24, 25, 26, 28, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 1,2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| NOTE 1: This requirement applies when the NR carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 2: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.66.4	∆TIB and ∆RIB values

For DC_26A_n41A, the TIB,c and RIB values are given in the tables below.

Table 6.66.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_26A_n41A | 26 | TBD |
|  | n41 | TBD |

Table 6.66.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_26A_n41A | 26 | TBD |
|  | n41 | 0 |

### 6.66.5	MSD

Table 6.66.5-1 lists the MSD required for the dual connectivity configuration for the cases of IMD interference. Table 6.66.5-2 lists the harmonic interference which falls into the n41 RX frequency. Table 6.66.5-3 lists the case that n41 TX falls into the B26 3rd RX frequency.

Table 6.66.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD3 |
|  | n41 | 2562 | 10 | 52 | 2562 | N/A | TDD | N/A |

Table 6.66.5-2 lists the MSD due to 3rd harmonic required for EN-DC configuration DC_26A_n41A.

Table 6.66.5-2: MSD due to 3rd harmonic for DC_26A_n41A

| MSD due to harmonic for the DL band |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 26 | n41 | NA | 10.3 | 8.4 | 7.4 | 5 | 4.3 | 3.9 | 3.1 | 2.7 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |

Table 6.66.5-3 lists the B26 UL configuration for n41 MSD test due to 3rd order harmonic interference.

Table 6.66.5-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | n41 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |

Table 6.66.5-4 lists the MSD values for DC_26A_n41A due to 3rd order harmonic mixing.

Table 6.66.5-4: MSD due to 3rd order harmonic mixing for DC_26A_n41A

| UL band | DL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dB | dB | dB | dB | dB | dB |
| n41 | 26 | N/A | N/A | 24.3 | 24.3 | 22.5 | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |

Table 6.66.5-5 summarizes the n41 UL configuration for B26 3rd order harmonic mixing MSD test.

Table 6.66.5-5 n41 uplink configuration for B26 3rd order harmonic mixing MSD test

| NR Band / Channel bandwidth of the high band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | SCS | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| DC_26_n41 | n41 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |
| NOTE 1: RB allocations in this table are corresponding to channel bandwidth of Band 26. |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.66.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.66.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.66.6-1.

Table 6.66.6-1: Band 26 and Band n41 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_26A_n41A | 26 | a | -2 | b | -1 | 3 | 26 | IMD |
|  | n41 | c | 1 | d | 0 |  |  |  |
| DC_26A_n41A | 26 | a | 4 | b | -1 | 5 | 26 | IMD |
|  | n41 | c | -1 | d | 0 |  |  |  |
| DC_26A_n41A | 26 | a | 3 | b | 0 | 3 | n41 | Harmonic |
|  | n41 | c | 0 | d | -1 |  |  |  |
| DC_26A_n41A | 26 | a | 0 | b | 3 | 3 | 26 | Harmonic mixing |
|  | n41 | c | -1 | d | 0 |  |  |  |

Equations (6.66.6.1) and (6.66.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.66.6-1 and CBW stands for channel bandwidth.

(6.66.6.1)

(6.66.6.2)

Formula (6.66.6.3-1) and (6.66.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.66.6.3-1)

(6.66.6.3-2)

## 6.67	DC_41A_n41A, DC_(n)41AA

### 6.67.1	Operating bands for DC

Table 6.67.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n41A, DC_(n)41AA | 41 | 2496 GHz | – | 2690 MHz | 2496 GHz | – | 2690 MHz | TDD |
|  | n41 | 2496 GHz | – | 2690 MHz | 2496 GHz | – | 2690 MHz | TDD |

### 6.67.2	Channel bandwidths per operating band for DC

Table 6.67.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |  | Maximum aggregated bandwidth [MHz] |
| DC_41A_n41A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 120 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.67.3	Co-existence studies

Table 6.67.3-1 lists Band 41 + Band n41 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.67.3-1: Band 41 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 7488 | 8070 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 194 | 194 | 4992 | 5380 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2302 | 2884 | 2302 | 2884 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7488 | 8070 | 7488 | 8070 |
| 3rd order IMD products (max BW) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
| IMD frequency limits (MHz) | 2396 | 2790 | 2476 | 2710 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 388 | 388 | 9984 | 10760 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4798 | 5574 | 4798 | 5574 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9984 | 10760 | 9984 | 10760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8264 | 7294 | 8264 | 7294 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 12480 | 13450 | 12480 | 13450 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3078 | 2108 | 3078 | 2108 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12480 | 13450 | 12480 | 13450 |

Based on Table 6.67.3-1;

-	2nd order harmonics may fall into Rx frequencies of bands 46 and n79

-	2nd order IMD may fall into Rx frequencies of bands 46 and n79

-	3rd order IMD may fall into Rx frequencies of band 7, 30, 38, 40. 41 and 69

-	4th order IMD may fall into Rx frequencies of band 46 and 79

-	5th order IMD may fall into Rx frequencies of band 1, 4, 7, 10, 23, 30, 38, 40, 41, 65. 66 and 69

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.67.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.67.3-2: 2UL Band 41 +Band n41 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YesNOTE | US/Europe | IMD3 |
|  | 2400 | - | 2494 | YesNOTE | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD2, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd Harmonic, IMD2, IMD4 |
|  | 5470 | - | 5725 | No |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD2, IMD4 |
| NOTE: depends on the B41 and n41 allocation |  |  |  |  |  |  |

Table 6.67.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.67.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41A_n41A, DC_(n)41AA | E-UTRA/NR Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 1,2 |
| NOTE 1: This requirement applies when the E-UTRA carrier is confined within 2545-2575 MHz or 2595-2645 MHz and the channel bandwidth is 10 or 20 MHz NOTE 2: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

### 6.67.4	∆TIB and ∆RIB values

For DC_41A_n41A, the TIB,c and RIB values are given in the tables below.

Table 6.67.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A_n41A | 41 | 0 |
|  | n41 | 0 |

Table 6.67.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A_n41A | 41 | 0 |
|  | n41 | 0 |

### 6.67.5	MSD

Since Band 41 and n41 are both TDD bands, no MSD is required for the dual connectivity configuration.

### 6.67.6	SEM for DC_41A_n41A

The spectral emission mask for DC_41A_n41 is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used. When NS_04 is signalled the emissions must also meet the additional requirements. Also, when NS_04 is signalled, the general spurious emission mask is -25 dBm/MHz, not -30 dBm/MHz..

### 6.67.6a	SEM for DC_(n)41AA

The SEM for DC_(n)41AA will be based on the aggregated emisission bandwidth. The aggregated emission bandwidth determines the frequency where the emission mask drops from -13 dBm/MHz to -25 dBm/MHz. When NS_04 is signalled the emissions must also meet the additional requirements. Also, when NS_04 is signalled, the general spurious emission mask is -25 dBm/MHz, not -30 dBm/MHz.

Table 6.67.6a-1: DC_(n)41AA spectrum emission mask

| ΔfOOB (MHz) | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 0 MHz  f < 1 MHz | See Table 6.67.6a-2 | 30kHz |
| 1 MHz  f < 5 MHz | -10 | 1 MHz |
| 5 MHz  f < ENBW | -13 | 1 MHz |
| ENBW  f < ENBW + 5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |

Table 6.67.6a-2: DC_(n)41AA Requirement for 1st MHz

| ENBW | 10MHz | 15MHz | 20MHz | 40MHz | 50MHz | >50MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1st 1MHz Requirement | -18 | -20 | -21 | -24 | -25 | -25 |

### 6.67.7	A-MPR

Band 41 and n41 will need to meet the emission mask for ns_04. The requirements are TBD.

### 6.67.8	Bandwidth Combination Sets

At RAN4 #86 bis it was agreed to allow Bandwidth Combination Sets for intra-band EN-DC in R4-1804220.

Table 6.67.8-1: Bandwidth Combination Sets for DC_(n)41AA and DC_41A_n41A

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for LTE carrier [MHz] | Channel bandwidths NR for carrier [MHz] | Channel bandwidths for LTE carrier [MHz] |  |  |
| DC_(n)41AA | DC_(n)41AA | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
| DC_41A_n41A | DC_41A_n41A | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |

## 6.68	DC_66A_n71A

### 6.68.1	Operating bands for DC

Table 6.68.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n71A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.68.2	Channel bandwidths per operating band for DC

Table 6.68.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 50 MHz | 80 MHz | 100 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 40 |
|  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 6.68.3	Co-existence studies

Table 6.68.3-1 lists Band 66 +Band n71 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.68.3-1: Band 66 and Band n71 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 |  | 1396 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 |  | 2094 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 1117 | 1012 |  | 2373 | 2478 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 454 |  | 314 | 2722 | 2897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 3036 |  | 3176 | 4083 | 4258 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
|  | 2234 |  | 2024 | 4746 | 4956 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
|  | 209 | 384 |  | 4432 | 4677 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
|  | 3699 |  | 3874 | 5793 | 6038 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 6457 |  | 6142 | 1082 | 872 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 7503 |  | 7818 | 4362 | 4572 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 4014 |  | 3734 | 1326 | 1571 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 6456 |  | 6736 | 5409 | 5654 |

Based on Table 6.68.3-1;

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 77 and 78.

-	3rd order harmonic may fall into Rx frequencies of bands 2, 25, 34, 36, 46 and 70.

-	2nd order IMD may fall into Rx frequencies of band 30 and 40.

-	3rd order IMD may fall into Rx frequencies of band 77.

-	4th order IMD may fall into own Rx frequencies of band 66 as well as Rx frequencies of band 1, 4, 10, 23, 34, 43, 47, 48, 65, 77, 79 and 79.

-	5th order IMD may fall into Rx frequencies of band 5, 6, 8, 11, 18, 19, 21, 24, 26, 32, 43, 45, 46, 50, 74, 75, 76, 77, 78 and 79.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.68.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.68.3-2: 2UL Band 66 +Band n71 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD4, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

Table 6.68.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.68.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n71A | E-UTRA Band 4, 5, 7,10, 13, 14, 17, 22, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.68.4	∆TIB and ∆RIB values

For DC_66A_n71A, the TIB,c and RIB values are given in the tables below.

Table 6.68.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A_n71A | 66 | 0.3 |
|  | n71 | 0.3 |

Table 6.68.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66A_n71A | 66 | 0 |
|  | n71 | 0 |

### 6.68.5	MSD

A combination of Band 66 and band 71 have similarities to CA_1A-8A and CA_3A-5A. Both these are a low band plus a high band, and both have IMD4 affecting the high band. CA_3A-5A has 4 dB and CA_1A-8A has 6 dB as MSD, and for a combination of band 66 and band 71 the MSD is set to an average value of 5 dB.

Table 6.68.5-1 lists the MSD required for the dual connectivity configuration DC_66A_n71A.

Table 6.68.5-1: MSD for the DC_66A_n71A configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A |  | N/A |

### 6.68.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.68.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.68.6-1.

Table 6.68.6-1: Band 66 and Band n71 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n71A | 66 | a | 2 | b | -1 | 4 | 66 | IMD |
|  | n71 | c | -2 | d | 0 |  |  |  |

Equations (6.68.6.1) and (6.68.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.68.6-1 and CBW stands for channel bandwidth.

(6.68.6.1)

(6.68.6.2)

Formula (6.68.6.3-1) and (6.68.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.68.6.3-1)

(6.68.6.3-2)

## 6.69	DC_(n)71B

### 6.69.1	Operating bands for DC

Table 6.69.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| DC_(n)71B | 71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
|  | n71 |  |  |  |

### 6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| EN-DC configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for LTE carrier [MHz] | Channel bandwidths for NR carrier [MHz] |  |  |
| DC_(n)71B | DC_(n)71B | 15 | 5 | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |
|  |  | 5 | 5, 10, 15 |  |  |

### 6.69.3	Co-existence studies

DC_(n)71B is a contiguous allocation thus only uplink harmonic analysis is needed.

Table 6.69.3-1: DC_(n)71B UL harmonics

| Uplink band |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 |

From Table 6.69.3-1 it can be seen that uplink harmonics 3rd harmonic overlaps with bands 3, 25, 34 and 70.

### 6.69.4	∆TIB and ∆RIB values

No dTib or dRib is required for DC_(n)71B as it is intraband DC.

### 6.69.5	MSD

Table 6.69.5-1: MSD for the DC_(n)71B configuration

| MSD / DC bandwidth class B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation [LCRB] | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71B | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
|  | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
|  | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
|  | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | [24.3] |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | [35.2] |  |

### 6.69.6	SEM for DC_(n)71B

FCC has defined spectrum emission maks that is relevant for operation on band 71 frequency range. SEM is presented in Table 6.69.6-1.

6.69.6-1: SEM for band 71 frequency range

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

### 6.69.7	A-MPR

Figure 6.69.7-1 shows required back-off as function of allocation ratio. Figure contains data for all included bandwidth combinations, SCSs in the worst-case position (upper band edge). From this data we have derived a unified A-MPR definition for the whole band n71.

Figure 6.69.7-1: Back-off vs. allocation ratio for entire band n71 (all bandwidth combinations and SCSs)

When LTE and NR know each other’s allocation, the A-MPR(1) is defined in terms of the total transmission powers as

where

The allocation ratio is

When LTE and NR do not know each other’s allocation, The A-MPR(2) is defined carrier wise as

where

Above, the worst-case allocation ratios for each carrier are as follows where LTE modems assumes that NR allocation is 1 as it will lead to maximum needed A-MPR and vice versa.

and the worst-case division of power between carriers is represented by

and

Table 6.69.7-1. Symbols for DC_(n)71B A-MPR

|  | Total transmission power (LTE+NR) |
| --- | --- |
|  | Transmission power of the LTE channel |
|  | Transmission power of the NR channel |
|  | Allocation size (in RB) in LTE channel |
|  | Allocation size (in RB) in NR channel |
|  | Transmission bandwidth configuration in LTE channel |
|  | Transmission bandwidth configuration in NR channel for its configured SCS |
|  | Transmission bandwidth configuration in NR channel for the SCS assumed by the LTE branch (minimum SCS valid for the NR channel bandwidth) |

## 6.70	DC_20A-n8A

### 6.70.1	Operating bands for DC

Table 6.70.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| DC_20A-n8A | 20 | 832 MHz | – | 862 MHz |  | 791 MHz | – | 821 MHz |  | FDD |
|  | n8 | 880 MHz | – | 915 MHz |  | 925 MHz | – | 960 MHz |  |  |

### 6.70.2	Channel bandwidths per operating band for DC

Table 6.70.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_20A-n8A |  | 20 | 15 | Yes | Yes |  |  |  |  |  |  |  | 30 | 0 |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |

### 6.70.3	Co-existence studies

Tables 6.70.3-1, Band 20A+Band n8A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.70.3-1: Band 20 and Band n8 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 880 | 915 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 1760 – 1830 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 2640 – 2745 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 18 – 83 |  |  | 1712 – 1777 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 749 – 844 |  |  | 898 – 998 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2544 – 2639 |  |  | 2592 – 2692 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 812 – 882 |  |  | 870 – 925 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1581 – 1706 |  |  | 1778 – 1913 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 36 – 166 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 – 3501 |  |  | 3472 – 3607 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 3424 – 3554 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2658 –2828 |  |  | 2413 – 2568 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 916 – 1081 |  |  | 666 – 826 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4352 – 4522 |  |  | 4208 – 4363 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4304 – 4469 |  |  | 4256 – 4416 |  |  |

Based on Table 6.70.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 3

-	3rd order harmonics may fall into Rx frequencies of Band 7, 38, 41, 69

-	3rd order IMD may fall into Rx frequencies of Band 5, 6, 7, 8, 13, 14, 18, 19, 20, 26, 27, 28, 38, 41, 44, 67, 68, and 69

-	4th order IMD may fall into Rx frequencies of Band 3, 9, 22, 33, 35, 37, 39, 42, 43, 48, n77, and n78

-	5th order IMD may fall into Rx frequencies of Band 7, 8, 12, 13, 14, 17, 20, 28, 29, 41, 44, 67, and 68

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.70.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.70.3-2: 2UL Band 20 + Band n8 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US | - |
|  | 5150 | - | 5350 | No | Europe | - |
|  | 5470 | - | 5725 | No |  | - |
|  | 5150 | - | 5825 | No | Asia | - |

Table 6.70.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.70.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_20A-n8A | E-UTRA Band 1, 3, 7, 22, 28, 31, 32, 34, 38, 42, 43, 65, 75, 76, n78 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.70.4	∆TIB and ∆RIB values

For DC_20A-n8A, the TIB,c and RIB values are given in the tables below.

Table 6.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-n28A | 20 | 0.4 |
|  | n8 | 0.4 |

Table 6.70.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-n28A | 20 | 0 |
|  | n8 | 0 |

### 6.70.5	MSD

Table 6.70.5-1: MSD for the DC configuration

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A-n8A | 20 | 849.5 | 5 | 25 | 808.5 | 21 | FDD | IMD3 | Yes |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | FDD | IMD3 |  |

### 6.70.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.70.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.70.6-1.

Table 6.70.6-1: Band 20 and Band n8 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n8A | 20 | a | 2 | b | -1 | 3 | 20 | IMD |
|  | n8 | c | -1 | d | 0 |  |  |  |
| DC_20A_n8A | 20 | a | 3 | b | -1 | 5 | 20 | IMD |
|  | n8 | c | -2 | d | 0 |  |  |  |
| DC_20A_n8A | 20 | a | -2 | b | 0 | 5 | n8 | IMD |
|  | n8 | c | 3 | d | -1 |  |  |  |

Equations (6.70.6.1) and (6.70.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.70.6-1 and CBW stands for channel bandwidth.

(6.70.6.1)

(6.70.6.2)

Formula (6.70.6.3-1) and (6.70.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.70.6.3-1)

(6.70.6.3-2)

## 6.71	DC_46A_n78A

### 6.71.1	Operating bands for DC

Table 6.71.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_46A_n78A | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.71.2	Channel bandwidths per operating band for DC

Table 6.71.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_46A_n78A1 | 46 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes |  | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 1: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.71.3	Co-existence studies

N/A

### 6.71.4	∆TIB and ∆RIB values

For DC_46A-n78A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46A-n78A | n78 | 0.8 |

Table 6.71.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46A-n78A | n78 | 0.5 |

### 6.71.5	MSD

DC_46A_n78A is subjected to MSD due to cross band isolation referring same mechanism of LTE CA_46A-48A result. The MSD due to cross band isolation is calculated as below tables:

Table 6.71.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 46 |  |  |  | 7 |  |  |  |  |  |  |  |  |

Table 6.71.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

|  | E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n78 | 46 | 30 |  |  |  | 270 |  |  |  |  |  |  |  |  |

## 6.72	DC_1A-n50A

### 6.72.1	Operating bands for DC

Table 6.72.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-n50A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |

### 6.72.2	Channel bandwidths per operating band for DC

Table 6.72.2-1

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-n50A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 100 |
|  | n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.72.3	Co-existence studies

Table 6.72.3-1 lists Band 1 + Band n50 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.72.3-1: Band 1 and Band n50 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1432 | 1517 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2864 | 3034 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4296 | 4551 | 5760 | 5940 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 403 | 548 | 3352 | 3497 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 884 | 1114 | 2323 | 2528 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 4784 | 5014 | 5272 | 5477 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1412 | 1537 | 1860 | 2040 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 2316 | 2631 | 4243 | 4508 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 806 | 1096 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 6216 | 6531 | 7192 | 7457 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 6704 | 6994 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 6163 | 6488 | 3748 | 4148 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2726 | 3076 | 711 | 336 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 9112 | 9437 | 7648 | 8048 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 8624 | 8974 | 8136 | 8511 |

Based on Table 6.72.3-1

-	2rd order harmonic may fall into Rx frequencies of band n77.

-	3rd order harmonic may fall into Rx frequencies of band 46, n79.

-	2nd order IMD may fall into Rx frequencies of bands 31, 42, 72, n77 and n78.

-	3rd order IMD may fall into Rx frequencies of bands 3, 5, 6, 8, 9, 19, 24, 26, 30, 33, 34, 35, 39, 40, 41, 46, 51, 70 and n79

-	4th order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 18, 19, 20, 26, 27, 30, 38, 40, 41, 69, and n79

-	5th order IMD may fall into Rx frequencies of bands 43, 44, n77 and n78

It should be noted that IMD will not be an issue for band n50 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.72.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.72.3-2: 2UL Band 1 +Band n50 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, 4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, 4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3 |
|  | 5470 | - | 5725 | Yes |  | IMD3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD3 |

Table 6.72.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.72.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-n50A | E-UTRA Band 22, 28, 43, 67, 68, 69, 73 | FDL_low FDL_low | --- | FDL_high FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15, 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
|  | E-UTRA Band 5, 6, 7, 8, 20, 26, 27, 30, 31, 38, 40, 41, 42, 44, 46, 72 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

### 6.72.4	∆TIB and ∆RIB values

For DC_1A-n50A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-n50A | 1 | 0.6 |
|  | n50 | 0.6 |

Table 6.72.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-n50A | n50 | 0.2 |

### 6.72.5	MSD

There is no specific MSD’s requirement.

## 6.73	DC_1A-n51A

### 6.73.1	Operating bands for DC

Table 6.73.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-n51A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |

### 6.73.2	Channel bandwidths per operating band for DC

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-n51A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 25 |
|  | n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.73.3	Co-existence studies

Table 6.73.3-1 lists Band 1 + Band n51 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.73.3-1: Band 1 and Band n51 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427 | 1432 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2854 | 2864 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4281 | 4296 | 5760 | 5940 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 488 | 553 | 3347 | 3412 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 874 | 944 | 2408 | 2533 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 4774 | 4844 | 5267 | 5392 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1407 | 1452 | 1915 | 1985 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 2301 | 2376 | 4328 | 4513 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 976 | 1106 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 6201 | 6276 | 7187 | 7372 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 6694 | 6824 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 6248 | 6493 | 3728 | 3808 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2896 | 3086 | 456 | 321 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 9107 | 9352 | 7628 | 7708 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 8614 | 8804 | 8121 | 8256 |

Based on Table 6.73.3-1

-	2rd order harmonic may fall into Rx frequencies of band n77.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD may fall into Rx frequencies of bands 42, n77 and n78.

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 8, 18, 19, 26, 32, 33, 39, 41, 45, 46, 50, 75 and n79

-	4th order IMD may fall into Rx frequencies of bands 30, 40 and n79

-	5th order IMD may fall into Rx frequencies of bands 43, n77 and n78.

It should be noted that IMD will not be an issue for band n51 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.73.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.73.3-2: 2UL Band 1 +Band n51 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD3 |

Table 6.73.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.73.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-n51A | E-UTRA Band 7, 12, 13, 17, 20, 22, 27, 28, 29, 31, 38, 44, 48, 67, 68, 69, 72, 73 | FDL_low FDL_low | --- | FDL_high FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
|  | E-UTRA Band 5, 6, 8, 26, 30, 40, 41, 42, 43, 46 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

### 6.73.4	∆TIB and ∆RIB values

For DC_1A-n51A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.73.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-n51A | 1 | 0.6 |
|  | n51 | 0.6 |

Table 6.73.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-n51A | n51 | 0.1 |

### 6.73.5	MSD

There is no specific MSD’s requirement.

## 6.74	DC_2A-n71A

### 6.74.1	Operating bands for DC

Table 6.74.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-n71A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | TDD |

### 6.74.2	Channel bandwidths per operating band for DC

Table 6.74.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 40 |
|  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.74.3	Co-existence studies

Table 6.1.3-1 lists Band 2 +Band n71 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.74.3-1: Band 2 and Band n71 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 |  | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 |  | 3820 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 |  | 5730 | 1989 | 2094 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1247 | 1152 |  | 2513 | 2608 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3002 |  | 3157 | 584 | 454 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4363 |  | 3122 | 3176 | 3306 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1830 |  | 1930 | 643 | 718 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4852 | 5067 |  | 79 | 244 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2304 |  | 2494 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6213 |  | 6428 | 3839 | 4004 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5026 |  | 5216 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 942 |  | 742 | 6977 | 6702 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1606 |  | 1831 | 4404 | 4154 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4502 |  | 4702 | 8063 | 8338 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5689 |  | 5914 | 6876 | 7126 |

The uplink of Band n71 is 663 - 698 MHz for which third harmonic products fall within 1989 – 2094 MHz, while the downlink of Band 2 is 1930 - 1990 MHz.  Thus, it can be seen that there is potential for third harmonic interference from uplink in Band 71 to downlink in Band 2.  However, the potential is limited to edge channels since the overlap is only 1 MHz from 1989 – 1990 MHz.  Only an uplink allocation in the lowest edge of Band 71 paired with simultaneous downlink in the uppermost edge of Band 2 could be impacted.  Moreover, due to the guard bands inherent in the definition of the transmission bandwidth configuration for LTE and NR, there may not be any direct overlap at all.  However, there may still be some effect due to spectral spreading of the third harmonic even if direct overlap is averted.

Another potential mechanism for harmonic interference is mixing of the uplink in Band 2 with the third harmonic of the Rx LO.  In this case, the frequency range impacted is much larger.  The third harmonic of Rx extends from 1851 to 1956 MHz while the uplink in Band 2 ranges from 1850 to 1910 MHz.  Therefore, there is nearly complete overlap between these two ranges.  Depending upon the particular frequency assignments and allocation of PRB’s for uplink within Band 2 and downlink within Band n71, there may be interference from this mechanism.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.74.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.74.3-2: 2UL Band 2 +Band n71 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | Yes |  | H3, IMD4, 5 |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD4, 5 |

Table 6.74.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.74.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-n71A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| Note 1 : As exceptions, measurements with a level up to the applicable requirements defined in Table 6.74.3-3 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.74.3-3 and [Table x.x.x.x-x] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

Note:	To simplify Table 6.74.3-3, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation. NR band numbers are listed for bands which are specified only for NR operation or both E-UTRA and NR operation

### 6.74.4	∆TIB and ∆RIB values

For DC_2A-n71A, the TIB,c and RIB values are reused from LTE CA_2A-71A and given in the tables below.

Table 6.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-n71A | 2 | 0.3 |
|  | n71 | 0.3 |

Table 6.74.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-n71A | 2 | 0 |
|  | n71 | 0 |

### 6.74.5	MSD

MSD due to harmonics affects both Band 2 and Band n71.  When there is uplink on low frequency channels of Band n71, its third harmonic and regrowth reach the uppermost downlink channel in Band 2.  When there is uplink on Band 2, the third harmonic of Rx LO in Band n71 mixes with the uplink and generates harmonic interference in Band n71 Rx.

#### 6.74.5.1	Band 2 MSD

The MSD from LTE CA_2A-71A is reused for this DC combination.  Although the NR spectrum utilization in Band n71 is larger than the LTE spectrum utilization in Band 71, the worst case interference mechanism is expected to be single RB transmissions.  The harmonic noise from single RB transmissions in Band n71 is not expected to increase due to the higher spectrum utilization.

Table 6.74.5.1-1.  Summary of MSD for all aggressor and victim bandwidths

| Band n71 CBW | Band 2 CBW | MSD |
| --- | --- | --- |
| 5 | 5 | 4.6 |
| 5 | 10 | 1.0 |
| 5 | 15 | 0.6 |
| 5 | 20 | 0.5 |
| 10 | 5 | 1.2 |
| 10 | 10 | 1.0 |
| 10 | 15 | 0.7 |
| 10 | 20 | 0.6 |
| 15 | 5 | 1.1 |
| 15 | 10 | 0.6 |
| 15 | 15 | 0.7 |
| 15 | 20 | 0.6 |
| 20 | 5 | 1.2 |
| 20 | 10 | 0.6 |
| 20 | 15 | 0.4 |
| 20 | 20 | 0.6 |

The reference sensitivity can be defined as follows where the maximum MSD for a given Band 2 channel bandwidth is applied.  The 5 MHz channel bandwidth requires an amendment when the Band n71 Tx bandwidth is also 5 MHz since the MSD is significantly larger for this case.  Also, it is noted that the MSD only applies when the channels are located in proximity of  the lower edge of Band n71 and the upper edge of Band 2.

Table 6.74.5.1-2: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | 2NOTE 1 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |
|  | 2NOTE 2 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 2: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.74.5.1-3: Uplink configuration for the low band (exceptions due to harmonic issue)

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | 2 | 25X | 25X | 20X | 20X |  |  |  |  |  |  |  |
| n71 | 2 | 8Y | 8Y | 8Y | 8Y |  |  |  |  |  |  |  |

#### 6.74.5.2	Band n71 MSD

The reference sensitivity of Band n71 is reused from Band 71 in LTE CA_2A-71A.  Although the NR spectrum utilization in Band n71 is larger than the LTE spectrum utilization in Band 71, the reference sensitivity of Band n71 is dominated by the harmonic mixing noise greater than .  Therefore, the slight increase in thermal noise and Tx noise due to higher spectrum utilization in Band n71 is insignificant in comparison.

Table 6.74.5.2-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA and NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| DC_2A-n71A15,16 | 2 | 15 | -98 | -95 | -93.2 | -92 | FDD |
|  | n71 | 15 | -70.4 | -70.4 | -70.4 | -70.4 |  |
| NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |

Table 6.74.5.2-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | UL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| DC_2A-n71A | 2 | 15 | 25 | 50 | 50 | 50 | FDD |

### 6.74.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.74.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.74.6-1.

Table 6.74.6-1: Band 2 and Band n71 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n71A | 2 | a | 0 | b | -1 | 3 | 2 | Harmonic |
|  | n71 | c | 3 | d | 0 |  |  |  |
| DC_2A_n71A | 2 | a | -1 | b | 0 | 3 | n71 | Harmonic mixing |
|  | n71 | c | 0 | d | 3 |  |  |  |

Equations (6.74.6.1) and (6.74.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.74.6-1 and CBW stands for channel bandwidth.

(6.74.6.1)

(6.74.6.2)

Formula (6.74.6.3-1) and (6.74.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.74.6.3-1)

(6.74.6.3-2)

## 6.75	DC_3A-n50A

The imcompleted combination is removed in Rel-15

### 6.75.1	Void

### 6.75.2	Void

### 6.75.3	Void

### 6.75.4	Void

### 6.75.5	Void

## 6.76	DC_3A-n51A

### 6.76.1	Operating bands for DC

Table 6.76.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-n51A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |

### 6.76.2	Channel bandwidths per operating band for DC

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-n51A |  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 25 |
|  |  |  | n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.76.3	Co-existence studies

Table 6.76.3-1 lists Band 3 + Band n51 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.76.3-1: Band 3 and Band n51 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427 | 1432 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2854 | 2864 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4281 | 4296 | 5130 | 5355 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 278 | 358 | 3137 | 3217 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 1069 | 1154 | 1988 | 2143 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 4564 | 4649 | 4847 | 5002 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1407 | 1452 | 1705 | 1790 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 2496 | 2586 | 3698 | 3928 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 556 | 716 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 5991 | 6081 | 6557 | 6787 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 6274 | 6434 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 5408 | 5713 | 3923 | 4018 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2266 | 2501 | 876 | 711 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 8267 | 8572 | 7418 | 7513 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 7984 | 8219 | 7701 | 7866 |

Based on Table 6.76.3-1

-	2rd order harmonic may fall into Rx frequencies of bands 22, 42, n77 and n78.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	3rd order IMD may fall into Rx frequencies of bands 1, 21, 32, 34, 36, 45, 50, 65, 66, 70, 75, and n79.

-	4th order IMD may fall into Rx frequencies of bands 38, 41, 43, 44, 69, 71, n77 and n78

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 19, 26, 30, 40, 41, 46 and n77.

It should be noted that IMD will not be an issue for band n51 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.76.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.76.3-2: 2UL Band 3 +Band n51 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonic3 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, IMD5 |

Table 6.76.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.76.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-n51A | E-UTRA Band 7, 8, 12, 13, 17, 20, 27, 28, 31, 33, 38, 48, 67, 68, 69, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 1, 5, 6, 22, 26, 30, 34, 36, 40, 41, 42, 43, 44, 46, 65, 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.76.4	∆TIB and ∆RIB values

For DC_3A-n51A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-n51A | 3 | 0.3 |
|  | n51 | 0.3 |

Table 6.76.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-n51A | 3 | 0.2 |
|  | n51 | 0.2 |

### 6.76.5	MSD

DC_3A_n51A is subjected to MSD due to cross band isolation referring same mechanism of DC_11A_n3A (R4-2002558) result with further calculations to all channel bandwidths. The MSD due to cross band isolation is calculated as below tables:

Table 6.75.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 3 | n51 | 6.4 |  |  |  |  |  |  |  |  |  |  |  |

Table 6.75.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

|  | E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 3 | n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |

## 6.77	DC_7A-n50A

### 6.77.1	Operating bands for DC

Table 6.77.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-n50A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |

### 6.77.2	Channel bandwidths per operating band for DC

Table 6.77.2-1

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-n50A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 100 |
|  | n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.77.3	Co-existence studies

Table 6.77.3-1 lists Band 7 + Band n50 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.77.3-1: Band 1 and Band n50 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1432 | 1517 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2864 | 3034 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4296 | 4551 | 7500 | 7710 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 983 | 1138 | 3932 | 4087 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 294 | 534 | 3483 | 3708 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 5364 | 5604 | 6432 | 6657 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1412 | 1537 | 2440 | 2630 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 1726 | 2051 | 5983 | 6278 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 1966 | 2276 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 6796 | 7121 | 8932 | 9227 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 7864 | 8174 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 8483 | 8848 | 3158 | 3568 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4466 | 4846 | 449 | 844 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 11432 | 11797 | 8228 | 8638 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 10364 | 10744 | 9296 | 9691 |

Based on Table 6.77.3-1

3rd order harmonic may fall into Rx frequencies of band n79.

2nd order IMD may fall into Rx frequencies of band n77.

3rd order IMD may fall into Rx frequencies of bands 22, 24, 31, 38, 42, 43, 46, 69, 72, n77 and n78,

4th order IMD may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 25, 33, 34, 35, 36, 37, 39, 65, 66 and 70.

5th order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 22, 28, 29, 31, 42, 44, 67, 68, 71, 72, n77, n78 and n79.

It should be noted that IMD will not be an issue for band n50 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.77.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.77.3-2: 2UL Band 7 +Band n50 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD3 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3 |

Table 6.77.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.77.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A-n50A | E-UTRA Band 2, 3, 5, 8, 26, 30, 31, 33, 34, 39, 40, 48, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
|  | E-UTRA Band 1, 4, 10, 12, 13, 14, 17, 20, 22, 23, 27, 28, 29, 42, 43, 44, 46, 65, 66, 67, 68 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

### 6.77.4	∆TIB and ∆RIB values

For DC_7A-n50A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-n50A | 7 | 0.3 |
|  | n50 | 0.3 |

Table 6.77.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-n50A | n50 | 0.2 |

### 6.77.5	MSD

There is no specific MSD’s requirement.

## 6.78	DC_7A-n51A

### 6.78.1	Operating bands for DC

Table 6.78.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-n51A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |

### 6.78.2	Channel bandwidths per operating band for DC

Table 6.78.2-1

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-n51A |  |  | 7 | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 25 |
|  |  |  | n51 | 15 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.78.3	Co-existence studies

Table 6.78.3-1 lists Band 7 + Band n51 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.78.3-1: Band 7 and Band n51 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427 | 1432 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2854 | 2864 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4281 | 4296 | 7500 | 7710 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 1068 | 1143 | 3927 | 4002 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 284 | 364 | 3568 | 3713 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 5354 | 5434 | 6427 | 6572 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1407 | 1452 | 2495 | 2575 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 1711 | 1796 | 6068 | 6283 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 2136 | 2286 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 6781 | 6866 | 8927 | 9142 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 7854 | 8004 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 8568 | 8853 | 3138 | 3228 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4636 | 4856 | 704 | 859 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 11427 | 11712 | 8208 | 8298 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 10354 | 10574 | 9281 | 9436 |

Based on Table 6.78.3-1

2nd order IMD may fall into Rx frequencies of band n77.

3rd order IMD may fall into Rx frequencies of bands 21, 22, 32, 38, 42, 43, 45, 46, 50, 69, 75, n77 and n78.

4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66

5th order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 27, 28, 29, 44, 67, 68 and n79.

It should be noted that IMD will not be an issue for band n51 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.78.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.78.3-2: 2UL Band 7 +Band n51 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD3 |

Table 6.78.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.78.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A-n51A | E-UTRA Band 2, 3, 5, 8, 26, 30, 31, 32, 33, 34, 40, 48, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
|  | E-UTRA Band 1, 4, 10, 12, 13, 14, 17, 20, 22, 23, 27, 28, 29, 42, 43, 44, 46, 65, 66, 67, 68 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

### 6.78.4	∆TIB and ∆RIB values

For DC_7A-n51A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.78.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-n51A | 7 | 0.3 |
|  | n51 | 0.3 |

Table 6.78.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-n51A | n51 | 0.2 |

### 6.78.5	MSD

There is no specific MSD’s requirement.

## 6.79	DC_20A-n50A

### 6.79.1	Operating bands for DC

Table 6.79.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A-n50A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |

### 6.79.2	Channel bandwidths per operating band for DC

Table 6.79.2-1

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A-n50A |  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 100 |
|  |  |  | n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.79.3	Co-existence studies

Table 6.79.3-1 lists Band 20 + Band n50 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.79.3-1: Band 20 and Band n50 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 1432 | 1517 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 2864 | 3034 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 4296 | 4551 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 570 | 685 | 2264 | 2379 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 147 | 292 | 2002 | 2202 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 3096 | 3241 | 3696 | 3896 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 772 | 922 | 1412 | 1537 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 979 | 1154 | 3434 | 3719 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 1140 | 1370 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 3928 | 4103 | 5128 | 5413 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 4528 | 4758 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 4866 | 5236 | 1811 | 2016 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2572 | 2887 | 278 | 538 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 6560 | 6930 | 4760 | 4965 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 5960 | 6275 | 5360 | 5620 |

Based on Table 6.79.3-1

3rd order harmonic may fall into Rx frequencies of bands 38, 41, 69 and n79.

2nd order IMD may fall into Rx frequencies of bands 30, 40 and 71.

3rd order IMD may fall into Rx frequencies of bands 1, 4, 6, 8, 10, 13, 18, 23, 24, 26, 28, 34, 43, 65, 66, 67, 68, 70, n77 and n78.

4th order IMD may fall into Rx frequencies of bands 22, 42, 43, 46, n77, n78 and n79.

5th order IMD may fall into Rx frequencies of bands 2, 3, 7, 9, 25, 31, 33, 34, 35, 36, 37, 38, 39, 41, 46, 69, 70, 72, and n79

It should be noted that IMD will not be an issue for band n50 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.79.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.79.3-2: 2UL Band 20 +Band n50 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, 5 |

Table 6.79.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.79.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_20A-n50A | E-UTRA Band 48 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | E-UTRA Band 1, 2, 3, 4, 7, 8, 9, 10, 13, 22, 23, 25, 30, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 46, 65, 66, 67, 69, 70, 72 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.79.4	∆TIB and ∆RIB values

For DC_20A-n50A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.79.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-n50A | 20 | 0.5 |
|  | n50 | 0.5 |

Table 6.79.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-n50A | n50 | 0.2 |

### 6.79.5	MSD

There is no specific MSD’s requirement.

## 6.80	DC_20A-n51A

### 6.80.1	Operating bands for DC

Table 6.80.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A-n51A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |

### 6.80.2	Channel bandwidths per operating band for DC

Table 6.80.2-1

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A-n51A |  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 25 |
|  |  |  | n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.80.3	Co-existence studies

Table 6.80.3-1 lists Band 20 + Band n51 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.80.3-1: Band 20 and Band n51 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 1427 | 1432 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 2854 | 2864 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 4281 | 4296 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 565 | 600 | 2259 | 2294 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 232 | 297 | 1992 | 2032 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 3091 | 3156 | 3686 | 3726 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 827 | 867 | 1407 | 1452 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 1064 | 1159 | 3419 | 3464 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 1130 | 1200 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 3923 | 4018 | 5113 | 5158 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 4518 | 4588 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 4846 | 4896 | 1896 | 2021 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2557 | 2632 | 268 | 368 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 6540 | 6590 | 4755 | 4880 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 5945 | 6020 | 5350 | 5450 |

Based on Table 6.80.3-1

3rd order harmonic may fall into Rx frequencies of bands 38, 41 and 69.

3rd order IMD may fall into Rx frequencies of bands 25, 32, 34, 43, 45, 50, 70, 75, n77 and n78

4th order IMD may fall into Rx frequencies of bands 42, 46, n77, n78 and n79.

5th order IMD may fall into Rx frequencies of bands 2, 7, 25, 33, 34, 35, 36, 37, 38, 39, 41, 46, 69, 70, and n79.

It should be noted that IMD will not be an issue for band n51 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.80.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.80.3-2: 2UL Band 20 +Band n51 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, 5 |

Table 6.80.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.80.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_20A-n51A | E-UTRA Band 1, 3, 4, 8, 17, 22, 28, 29, 31, 40, 43, 48, 65, 66, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 32, 33, 34, 35, 36, 37, 38, 39, 41, 42, 46, 69, 70 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.80.4	∆TIB and ∆RIB values

For DC_20A-n51A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.80.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-n51A | 20 | 0.5 |
|  | n51 | 0.5 |

Table 6.80.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-n51A | n51 | 0.2 |

### 6.80.5	MSD

There is no specific MSD’s requirement.

## 6.81	DC_28A-n50A

### 6.81.1	Operating bands for DC

Table 6.81.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-n50A | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |

### 6.81.2	Channel bandwidths per operating band for DC

Table 6.81.2-1

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A-n50A |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 100 |
|  |  |  | n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.81.3	Co-existence studies

Table 6.81.3-1 lists Band 28 + Band n50 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.81.3-1: Band 28 and Band n50 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1432 | 1517 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 2864 | 3034 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 4296 | 4551 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 684 | 814 | 2135 | 2265 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 64 | 111 | 2116 | 2331 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 2838 | 3013 | 3567 | 3782 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 643 | 808 | 1412 | 1537 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 592 | 812 | 3548 | 3848 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 1368 | 1628 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 3541 | 3761 | 4999 | 5299 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 4270 | 4530 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 4980 | 5365 | 1295 | 1560 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2800 | 3145 | 620 | 925 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 6431 | 6816 | 4244 | 4509 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 5702 | 6047 | 4973 | 5278 |

Based on Table 6.81.3-1

3rd order harmonic may fall into Rx frequencies of band 1, 4, 10, 23, 65, 66 and n79.

2nd order IMD may fall into Rx frequencies of bands 1, 4, 10, 20, 23, 28, 65, 66 and 67.

3rd order IMD may fall into Rx frequencies of bands 1, 4, 10, 20, 22, 23, 24, 28, 40, 42, 43, 65, 66, 67, 71, n77, and n78.

4th order IMD may fall into Rx frequencies of bands 20, 22, 24, 28, 42, 43, 46, 67, 71, n77, n78, and n79

5th order IMD may fall into Rx frequencies of bands 5, 6,, 18, 19, 20, 24, 26, 27, 28, 46, 67, 71, and n79

It should be noted that IMD will not be an issue for band n50 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.81.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.81.3-2: 2UL Band 28 +Band n50 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, 5 |

Table 6.81.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.81.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28A-n50A | E-UTRA Band 2, 3, 7, 8, 25, 29, 31, 34, 38, 39, 41, 48, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 6, 10, 22, 23, 24, 20, 40, 42, 43, 46, 65, 66, 73 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 19, 25 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

### 6.81.4	∆TIB and ∆RIB values

For DC_28A-n50A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.81.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-n50A | 28 | 0.5 |
|  | n50 | 0.5 |

Table 6.81.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A-n50A | n50 | 0.2 |

### 6.81.5	MSD

It may have 2nd, 3rd, 4th, 5th order IMDs when the UP of the band 28 and the UP of the band n50 is used in the same time without precaution. Thus, if the UL of the both bands are not used in the same time, all IMD issues are canceled, and there is no MSD issue for this DC configuration

Table 6.81.5-1 lists the MSD required for the dual connectivity configuration DC_28A_n50A.

Table 6.81.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode / MSD / Single UL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_28A-n50A | 28 | 725.5 | 20 | 25 | 765.5 | 5 | FDD | IMD 4, 5 | Yes |
|  | n50 | 1474.5 | 60 | 50 | 1474.5 | 5 | TDD | IMD 4, 5 |  |

### 6.81.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.81.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.81.6-1.

Table 6.81.6-1: Band 28 and Band n50 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n50A | 28 | a | -1 | b | -1 | 2 | 28 | IMD |
|  | n50 | c | 1 | d | 0 |  |  |  |
| DC_28A_n50A | 28 | a | 3 | b | -1 | 4 | 28 | IMD |
|  | n50 | c | -1 | d | 0 |  |  |  |
| DC_28A_n50A | 28 | a | -3 | b | -1 | 5 | 28 | IMD |
|  | n50 | c | 2 | d | 0 |  |  |  |
| DC_28A_n50A | 28 | a | 2 | b | 0 | 2 | n50 | Harmonic |
|  | n50 | c | 0 | d | -1 |  |  |  |

Equations (6.81.6.1) and (6.81.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.81.6-1 and CBW stands for channel bandwidth.

(6.81.6.1)

(6.81.6.2)

Formula (6.81.6.3-1) and (6.81.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.81.6.3-1)

(6.81.6.3-2)

## 6.82	DC_28A-n51A

### 6.82.1	Operating bands for DC

Table 6.82.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-n51A | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |

### 6.82.2	Channel bandwidths per operating band for DC

Table 6.82.2-1

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A-n51A |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 25 |
|  |  |  | n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.82.3	Co-existence studies

Table 6.82.3-1 lists Band 28 + Band n51 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.82.3-1: Band 28 and Band n51 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1427 | 1432 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 2854 | 2864 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 4281 | 4296 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 679 | 729 | 2130 | 2180 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 26 | 69 | 2106 | 2161 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 2833 | 2928 | 3557 | 3612 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 698 | 753 | 1407 | 1452 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 677 | 817 | 3533 | 3593 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 1358 | 1458 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 3536 | 3676 | 4984 | 5044 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 4260 | 4360 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 4960 | 5025 | 1380 | 1565 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2785 | 2890 | 610 | 755 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 6411 | 6476 | 4239 | 4424 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 5687 | 5792 | 4963 | 5108 |

Based on Table 6.82.3-1

2nd order harmonic may fall into Rx frequencies of band 21, 32, 45, 50, n51 own RX range, 75 and 76.

3rd order harmonic may fall into Rx frequencies of band 1, 4, 10, 23, 65 and 66.

2nd order IMD may fall into Rx frequencies of bands 1, 4, 10, 65 and 66.

3rd order IMD may fall into Rx frequencies of bands 1, 4, 10, 22, 32, 42, 43, 45, 50, 65, 66, 75, n77 and n78.

4th order IMD may fall into Rx frequencies of bands 20, 22, 28, 32, 42, 43, 45, 50, 67, 75, n77, n78 and n79.

5th order IMD may fall into Rx frequencies of bands 21, 24, 32, 45, 46, 50, 67, 71, 75, and n79.

It should be noted that IMD will not be an issue for band n51 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.82.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.82.3-2: 2UL Band 28 +Band n51 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD5 |

Table 6.82.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.82.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28A-n51A | E-UTRA Band 2, 3, 5, 7, 8, 25, 26, 31, 34, 38, 40, 41, 66, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 20, 22, 24, 32, 42, 43, 45, 46, 65, 66, 71, 73 NR band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 19, 25 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

### 6.82.4	∆TIB and ∆RIB values

For DC_28A-n51A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.82.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-n51A | 28 | 0.5 |
|  | n51 | 0.5 |

Table 6.82.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A-n51A | n51 | 0.2 |

### 6.82.5	MSD

It may have 2nd, 3rd, 4th, 5th order IMDs when the UP of the band 28 and the UP of the band n51 is used in the same time without precaution. Thus, if the UL of the both bands are not used in the same time, all IMD issues are canceled, and there is no MSD issue for this DC configuration

Table 6.81.5-1 lists the MSD required for the dual connectivity configuration DC_28A_n51A.

Table 6.81.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode / MSD / Single UL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_28A-n51A | 28 | 725.5 | 20 | 25 | 765.5 | 5 | FDD | IMD 4, 5 | Yes |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | 5 | TDD | IMD 4, 5 |  |

MSD due to B28 uplink harmonic is derived from LTE CA_28A-32A.

Table 6.81.5-2: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 28 | n512 | 27.8 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.81.5-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 28 | n51 | 12 |  |  |  |  |  |  |  |  |  |  |  |

### 6.82.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.82.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.82.6-1.

Table 6.82.6-1: Band 28 and Band n51 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n51A | 28 | a | 3 | b | -1 | 4 | 28 | IMD |
|  | n51 | c | -1 | d | 0 |  |  |  |
| DC_28A_n51A | 28 | a | 2 | b | 0 | 2 | n51 | Harmonic |
|  | n51 | c | 0 | d | -1 |  |  |  |

Equations (6.82.6.1) and (6.82.6.2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.82.6-1 and CBW stands for channel bandwidth.

(6.82.6.1)

(6.82.6.2)

Formula (6.82.6.3-1) and (6.82.6.3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.82.6.3-1)

(6.82.6.3-2)

## 6.83	DC_42A-n50A

### 6.83.1	Operating bands for DC

Table 6.83.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42A-n50A | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |

### 6.83.2	Channel bandwidths per operating band for DC

Table 6.83.2-1

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42A-n50A |  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 100 |
|  |  |  | n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.83.3	Co-existence studies

Table 6.83.3-1 lists Band 42 + Band n50 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.83.3-1: Band 42 and Band n50 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1432 | 1517 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2864 | 3034 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4296 | 4551 | 10200 | 10800 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 1883 | 2168 | 4832 | 5117 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 366 | 736 | 5283 | 5768 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 6264 | 6634 | 8232 | 8717 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1412 | 1537 | 3340 | 3660 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 696 | 1151 | 8683 | 9368 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 3766 | 4336 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 7696 | 8151 | 11632 | 12317 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 9664 | 10234 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 12083 | 12968 | 2128 | 2668 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7166 | 7936 | 2249 | 2904 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 15032 | 15917 | 9128 | 9668 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 13064 | 13834 | 11096 | 11751 |

Based on Table 6.83.3-1

3rd order harmonic may fall into Rx frequencies of band n79.

2nd order IMD may fall into Rx frequencies of bands 1, 2, 4, 10, 25, 33, 34, 36, 37, 39, 65, 67, 70 and n79.

3rd order IMD may fall into Rx frequencies of bands 12, 17, 24, 29, 31, 44, 46, 71 and 72.

4th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 43, 44, 67 and 68.

5th order IMD may fall into Rx frequencies of bands 1, 4, 7, 10, 23, 30, 38, 40, 41, 65, 66 and 69.

It should be noted that IMD will not be an issue for band n50 or band 42 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.83.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.83.3-2: 2UL Band 42 +Band n50 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3 |
|  | 5470 | - | 5725 | Yes |  | IMD3 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3 |

Table 6.83.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.83.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_42A-n50A | E-UTRA Band 3, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 2, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 20, 23, 25, 26, 27, 28, 29, 30, 31, 33, 34, 36, 37, 38, 39, 40, 41, 44, 46, 65, 66, 67, 68, 69 NR Band 79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

### 6.83.4	∆TIB and ∆RIB values

For DC_42A-n50A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.83.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42A-n50A | 42 | 0.6 |
|  | n50 | 0.8 |

Table 6.83.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42A-n50A | n50 | 0.2 |

### 6.83.5	MSD

There is no specific MSD’s requirement.

## 6.84	DC_42A-n51A

### 6.84.1	Operating bands for DC

Table 6.84.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42A-n51A | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |

### 6.84.2	Channel bandwidths per operating band for DC

Table 6.82.2-1

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42A-n51A |  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 25 |
|  |  |  | n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.84.3	Co-existence studies

Table 6.84.3-1 lists Band 42 + Band n51 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.84.3-1: Band 42 and Band n51 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427 | 1432 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2854 | 2864 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4281 | 4296 | 10200 | 10800 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 1968 | 2173 | 4827 | 5032 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 536 | 746 | 5368 | 5773 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 6254 | 6464 | 8227 | 8632 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1407 | 1452 | 3395 | 3605 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 681 | 896 | 8768 | 9373 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) |  |  |
| IMD frequency limits (MHz) | 3936 | 4346 |  |  |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 7681 | 7896 | 11627 | 12232 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) |  |  |
| IMD frequency limits (MHz) | 9654 | 10064 |  |  |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 12168 | 12973 | 2108 | 2328 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7336 | 7946 | 2504 | 2919 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 15027 | 15832 | 9108 | 9328 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 13054 | 13664 | 11081 | 11496 |

Based on Table 6.84.3-1

2nd order IMD may fall into Rx frequencies of bands 1, 2, 4, 10, 25, 34, 36, 65, 66 and 70.

3rd order IMD may fall into Rx frequencies of bands 12, 17, 21, 22, 24, 29, 32, 43, 44, 45, 46, 50, 67, 71, 75, n77 and n78

4th order IMD may fall into Rx frequencies of bands 5, 6, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 44, 67, 68 and n77.

5th order IMD may fall into Rx frequencies of bands 1, 4, 7, 10, 23, 38, 40, 41, 65, 66 and 69.

It should be noted that IMD will not be an issue for band n51 or band 42 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.84.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.84.3-2: 2UL Band 42 +Band n51 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD3 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3 |

Table 6.84.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.84.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_42A-n51A | E-UTRA Band 3, 8, 20, 25, 30, 31, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 2, 4, 5, 6, 7, 10, 12, 13, 14, 17, 23, 24, 26, 27, 28, 29, 32, 38, 40, 44, 46, 65, 66, 67, 68, 70, 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

### 6.84.4	∆TIB and ∆RIB values

For DC_42A-n51A, the ΔTIB,c and ΔRIB values are given in the tables below.

Table 6.84.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42A-n51A | 42 | 0.6 |
|  | n51 | 0.8 |

Table 6.84.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42A-n51A | n51 | 0.2 |

### 6.84.5	MSD

There is no specific MSD’s requirement.

## 6.85	DC_66A_n260

### 6.85.1	Operating bands for DC

Table 6.85.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n260 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.85.2	Channel bandwidths per operating band for DC

Table 6.85.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | [40 MHz] |  | 50 MHz | [60 MHz] |  | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  | Fallback Group |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 420 (220 for Band n260 with 60kHz SCS) |  |  |  |
|  | n260 | 60 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |
| DC_66A_n260(2A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 820 (420 for Band n260 with 60 kHz SCS) |  |  |  |
|  | n260 | 60 | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 [4] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n260A(3A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 1220 (620 for Band n260 with 60 kHz SCS) |  |  |  |
|  | n260 | 60 | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 [4] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n260A(4A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 1620 (820 for Band n260 with 60 kHz SCS) |  |  |  |
|  | n260 | 60 | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 [4] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-n260B | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  | n260 | 60 | See CA_n260B Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |  |
| DC_66A-n260C | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260C Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1220 |  |  |  |
| DC_66A-n260D | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 420 |  | 2 |  |
|  | n260 | 60 | See CA_n260D Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 420 |  |  |  |
| DC_66A-n260E | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 620 |  |  |  |
|  | n260 | 60 | See CA_n260E Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |  |
| DC_66A-n260F | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 820 |  |  |  |
|  | n260 | 60 | See CA_n260F Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |  |
| DC_66A-n260G | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |  | 3 |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 220 |  |  |  |
| DC_66A-n260H | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 320 |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 320 |  |  |  |
| DC_66A-n260I | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 420 |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 420 |  |  |  |
| DC_66A-n260J | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 520 |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  |  |  |
| DC_66A-n260K | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 620 |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |  |
| DC_66A-n260L | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 720 |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 720 |  |  |  |
| DC_66A-n260M | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 820 |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |  |
| DC_66A-n260O | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |  | 4 |  |
|  | n260 | 60 | See CA_n260O Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 220 |  |  |  |
| DC_66A-n260P | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 320 |  |  |  |
|  | n260 | 60 | See CA_n260P Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 320 |  |  |  |
| DC_66A-n260Q | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 420 |  |  |  |
|  | n260 | 60 | See CA_n260Q Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 420 |  |  |  |

Table 6.85.2-2: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] |  | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A-n260(2A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 420 |  |  |
|  | n260 | 60 | See CA_n260(2A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(3A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260(3A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 1220 |  |  |
| DC_66A-n260(4A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(4A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 1620 |  |  |
| DC_66A-n260(D-G) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260(D-G) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |
| DC_66A-n260(D-H) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 720 |  |  |
|  | n260 | 60 | See CA_n260(D-H) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 720 |  |  |
| DC_66A-n260(D-I) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(D-I) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(D-O) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260(D-O) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |
| DC_66A-n260(D-P) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 720 |  |  |
|  | n260 | 60 | See CA_n260(D-P) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 720 |  |  |
| DC_66A-n260(D-Q) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(D-Q) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(E-O) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(E-O) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(E-P) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 920 |  |  |
|  | n260 | 60 | See CA_n260(E-P) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 920 |  |  |
| DC_66A-n260(E-Q) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1020 |  |  |
|  | n260 | 60 | See CA_n260(E-Q) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 1020 |  |  |
| DC_66A-n260(A-I) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 820 |  |  |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n260(G-I) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 620 |  |  |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |

### 6.85.3	Co-existence studies

Table 6.85.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 66+Band n260 2UL DC.

Table 6.85.3-1: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 111000 | 120000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 148000 | 160000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 185000 | 200000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 222000 | 240000 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 259000 | 280000 |
| 2nd order IMD products | ¦f2_low–f1_high¦ |  | ¦f2_high–f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 35220 |  | 38290 | 38710 | 41780 |
| 3rd order IMD products | ¦f2_low–2*f1_high¦ |  | ¦f2_high–2*f1_low¦ | ¦2*f2_low–f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 33440 |  | 36580 | 72220 | 78290 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ |  | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 40420 |  | 43560 | 75710 | 81780 |
| 4rd order IMD products | ¦3*f1_low – f2_high¦ |  | ¦3*f1_high – f2_low¦ | ¦3*f2_low–f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limits (MHz) | 31660 |  | 34870 | 109220 | 118290 |
| 4rd order IMD products | ¦3*f1_low +f2_low¦ |  | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limits (MHz) | 42130 |  | 45340 | 112710 | 121780 |
| 4rd order IMD products | ¦2*f1_low-2*f2_high ¦ |  | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low+2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limits (MHz) | 70440 |  | 76580 | 77420 | 83560 |
| 5rd order IMD products | ¦f1_low –4*f2_high ¦ |  | ¦f1_high –4*f2_low¦ | ¦f2_low–4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limits (MHz) | 146220 |  | 158290 | 29880 | 33160 |
| 5rd order IMD products | ¦f1_low +4*f2_low¦ |  | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limits (MHz) | 149710 |  | 161780 | 43840 | 47120 |
| 5rd order IMD products | ¦2*f1_low-3*f2_high¦ |  | ¦2*f1_high–3*f2_low¦ | ¦2*f2_low-3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limits (MHz) | 107440 |  | 116580 | 68660 | 74870 |
| 5rd order IMD products | ¦2*f1_low +3*f2_low¦ |  | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limits (MHz) | 114420 |  | 123560 | 79130 | 85340 |

Based on Table 6.85.3-1, it can be seen that

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48, n77, n78

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, n1, 2, n2, 3, n3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, n50, 51, n51, 65, 66, n66, 70, n70, 74, n74, 75, n75, 76, n76, n80, n84

Therefore it can be concluded that there is no harmonic interference and IMD product falling into their own Rx bands.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.85.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.85.3-2: Band 66 and Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 3rd harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | Yes | China | 3rd, 4th, 5th IMD |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.85.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.85.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n260A | Bands 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.85.4	∆TIB and ∆RIB values

For DC_66A_n260, the TIB,c and RIB,c values are given in the tables below.

Table 6.85.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A_n260 | 66 | 0 |
|  | n260 | 0 |

Table 6.85.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66A_n260 | 66 | 0 |
|  | n260 | 0 |

### 6.85.5	REFSENs requirements

There is no specific REFSENs requirement.

## 6.86	DC_2_n257

### 6.86.1	Operating bands for DC

Table 6.86.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n257 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.86.2	Channel bandwidths per operating band for DC

Table 6.86.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2A_n257A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 220 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 420 |  |

Table 6.86.2-2: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band (non-contiguous)

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2A_n257(2A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 | See CA_n257(2A) CA bandwidth combinations in TS 38.101-2 [4]. |  |  |  |  |  |  |  |  |  |  | 420 |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |

### 6.86.3	Co-existence studies

Table 6.86.3-1 lists Band 2A +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.86.3-1: Band 2 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27650 | 24590 | 28350 | 31410 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25800 | 22680 | 51090 | 57150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30200 | 33320 | 54850 | 60910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55300 | 49180 | 56700 | 62820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23950 | 20770 | 77590 | 86650 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32050 | 35230 | 81350 | 90410 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116150 | 104090 | 18860 | 22100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107850 | 119910 | 33900 | 37140 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 84800 | 75680 | 47270 | 53450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83200 | 92320 | 58550 | 64730 |

Based on Table 6.86.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74,75 and band n257, n258

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n257

-	5th order IMD products may fall into Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.X.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.86.3-2: 2UL Band 2 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.86.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.86.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A_-n257 | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1; Table 6.5.3.2-1 of TS38.101 NOTE 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.86.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.86.4	∆TIB and ∆RIB values

For DC_2-n257, the TIB,c and RIB values are given in the tables below.

Table 6.86.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n257 | 2 | 0 |
|  | n257 | 0 |

Table 6.86.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n257 | 2 | 0 |
|  | n257 | 0 |

### 6.86.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.87	DC_13A-n257A

### 6.87.1	Operating bands for DC

Table 6.87.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13A-n257A | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.87.2	Channel bandwidths per operating band for DC

Table 6.87.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_13A-n257A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 210 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 410 |  |

### 6.87.3	Co-existence studies

Table 6.87.3-1 lists Band 13A +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.87.3-1: Band 13 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28723 | 25713 | 27277 | 30287 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27946 | 24926 | 52213 | 58223 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28054 | 31074 | 53777 | 59787 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 377 | 1187 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | -57446 | -51426 | 54554 | 60574 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -27169 | -24139 | 78713 | 87723 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28831 | 31861 | 80277 | 89287 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -117223 | -105213 | 23352 | 26392 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106777 | 118787 | 29608 | 32648 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -86946 | -77926 | 50639 | 56669 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81054 | 90074 | 55331 | 61361 |

Based on Table 6.87.3-1

-	2nd order harmonics may fall into frequencies of band 24

-	3rd order harmonic may fall into frequencies of band 30 and 40

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, 72, 73 and band n258

-	3rd order IMD products may fall into own Rx frequencies of Band 13 and band n257

-	4th order IMD products may fall into Rx frequencies of band n257 and n258

-	5th order IMD products may fall into Rx frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.87.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.87.3-2: 2UL Band 13 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | 2nd harmonics |
| Galileo | 1559 | - | 1591 | YES |  | 2nd harmonics |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | YES |  | 2nd harmonics |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.87.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.87.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13A-n257A | Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 14 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: Table 6.5.3.2-1 of TS38.101 NOTE 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.87.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.87.4	∆TIB and ∆RIB values

For DC_13A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.87.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13A-n257A | 13 | 0 |
|  | n257 | 0 |

Table 6.87.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13A-n257A | 13 | 0 |
|  | n257 | 0 |

### 6.87.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.88	DC_48A-n257A

### 6.88.1	Operating bands for DC

Table 6.88.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48A-n257A | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.88.2	Channel bandwidths per operating band for DC

Table 6.88.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_48A-n257A | 48 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 220 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 420 |  |

### 6.88.3	Co-existence studies

Table 6.88.3-1 lists Band 48A +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.88.3-1: Band 48 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25950 | 22800 | 30050 | 33200 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22400 | 19100 | 49300 | 55450 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 33600 | 36900 | 56550 | 62700 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 3150 | 4100 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 51900 | 45600 | 60100 | 66400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 18850 | 15400 | 75800 | 84950 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 37150 | 40600 | 83050 | 92200 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 114450 | 102300 | 11700 | 15300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 109550 | 121700 | 40700 | 44300 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 81400 | 72100 | 41900 | 48350 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 86600 | 95900 | 63650 | 70100 |

Based on Table 6.88.3-1

-	2nd order IMD products may fall into frequencies of band n258

-	3rd order IMD products may fall into frequencies of band 22, 42, 43 and 48

-	3rd order IMD products may fall into own Rx frequencies of band n257 and n258

-	4th order IMD products may fall into frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.88.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.88.3-2: 2UL Band 48 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.88.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.88.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48A-n257A | Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.88.4	∆TIB and ∆RIB values

For DC_48A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.88.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48A-n257A | 48 | 0 |
|  | n257 | 0 |

Table 6.88.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48A-n257A | 48 | 0 |
|  | n257 | 0 |

### 6.88.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.89	DC_5A-n260

### 6.89.1	Operating bands for DC

Table 6.89.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-n260 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.89.2	Channel bandwidths per operating band for DC

Table 6.89.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  | Fallback Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-n260A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 210 |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 410 |  |  |
| DC_5A-n260B | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | n260 | 60 | See NR CA_n260B Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260C | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See NR CA_n260C Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 1210 |  |  |
| DC_5A-n260D | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  | 2 |
|  | n260 | 60 | See NR CA_n260D Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |
| DC_5A-n260E | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n260 | 60 | See NR CA_n260E Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
| DC_5A-n260F | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n260 | 60 | See NR CA_n260F Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260G | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |  | 3 |
|  | n260 | 60 | See NR CA_n260G Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 210 |  |  |
| DC_5A-n260H | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |  |  |
|  | n260 | 60 | See NR CA_n260H Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 310 |  |  |
| DC_5A-n260I | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  |  |
|  | n260 | 60 | See NR CA_n260I Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |
| DC_5A-n260J | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |  |  |
|  | n260 | 60 | See NR CA_n260J Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 510 |  |  |
| DC_5A-n260K | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n260 | 60 | See NR CA_n260K Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
| DC_5A-n260L | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |  |  |
|  | n260 | 60 | See NR CA_n260L Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 710 |  |  |
| DC_5A-n260M | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n260 | 60 | See NR CA_n260M Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260O | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |  | 4 |
|  | n260 | 60 | See NR CA_n260O Bandwidth Combination Fall-back Group 4 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 210 |  |  |
| DC_5A-n260P | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |  |  |
|  | n260 | 60 | See NR CA_n260P Bandwidth Combination Fall-back Group 4 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 310 |  |  |
| DC_5A-n260Q | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  |  |
|  | n260 | 60 | See NR CA_n260Q Bandwidth Combination Fall-back Group 4 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |

Table 6.89.2-2: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz |  | [40 MHz] | 50 MHz |  | [60 MHz] |  |  | 80 MHz | 100 MHz |  | 200 MHz | 400 MHz |  | Maximum aggregated bandwidth [MHz] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-n260(2A) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |
|  | n260 | 60 | See NR CA_n260 (2A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260(3A) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n260 | 60 | See NR CA_n260(3A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1210 |  |  |
| DC_5A-n260(4A) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n260 | 60 | See NR CA_n260(4A) Bandwidth Combination in Table 8.9.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1610 |  |  |
| DC_5A-n260(D-G) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n260 | 60 | See CA_n260(D-G) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
| DC_5A-n260(D-H) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |  |
|  | n260 | 60 | See CA_n260(D-H) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |  |
| DC_5A-n260(D-I) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n260 | 60 | See CA_n260(D-I) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260(D-O) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n260 | 60 | See NR CA_n260(D-O) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
| DC_5A-n260(D-P) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |  |
|  | n260 | 60 | See NR CA_ n260(D-P) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |  |
| DC_5A-n260(D-Q) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n260 | 60 | See NR CA_ n260(D-Q) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260(E-O) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See NR CA_n260(E-O) Bandwidth Combination in Table 8.9.2-22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260(E-P) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See NR CA_n260(E-P) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 910 |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 910 |  |  |
| DC_5A-n260(E-Q) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See NR CA_n260(E-Q) Bandwidth Combination in Table 8.9.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1010 |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1010 |  |  |
| DC_5A-n260(A-I) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n260(G-I) | 5 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |

### 6.89.3	Co-existence studies

Table 6.89.3-1 lists Band 5A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.89.3-1: Band 5 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39176 | 36151 | 37824 | 40849 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38352 | 35302 | 73151 | 79176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38648 | 41698 | 74824 | 80849 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 36990 | 40010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78352 | 72302 | 75648 | 81698 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37528 | 34453 | 110151 | 119176 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39472 | 42547 | 111824 | 120849 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159176 | 147151 | 33604 | 36704 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148824 | 160849 | 40296 | 43396 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118352 | 109302 | 71453 | 77528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112648 | 121698 | 76472 | 82547 |

Based on Table 6.89.3-1

-	3rd order harmonic may fall into frequencies of band 41, n41

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into frequencies of band 5, n5,6, 8, n8, 12, n12, 13, n13, 14, 17, 18, 19, 20, n20, 26, 27, 28, n28, 29, 31, 44, 67, 68, 71, n71, 72, 73, n81, n82 and n83

-	4th order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.89.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.89.3-2: 2UL Band 5 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.89.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.89.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.89.4	∆TIB and ∆RIB values

For DC_5A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-n260A | 5 | 0 |
|  | n260 | 0 |

Table 6.89.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-n260A | 5 | 0 |
|  | n260 | 0 |

### 6.89.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.90	DC_13A-n260A

### 6.90.1	Operating bands for DC

Table 6.90.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13A-n260A | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.90.2	Channel bandwidths per operating band for DC

Table 6.90.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_13A-n260A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 210 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 410 |  |

### 6.90.3	Co-existence studies

Table 6.90.3-1 lists Band 13A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.90.3-1: Band 13 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39223 | 36213 | 37777 | 40787 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38446 | 35426 | 73213 | 79223 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38554 | 41574 | 74777 | 80787 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 377 | 1187 | 36990 | 40010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78446 | 72426 | 75554 | 81574 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37669 | 34639 | 110213 | 119223 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39331 | 42361 | 111777 | 120787 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159223 | 147213 | 33852 | 36892 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148777 | 160787 | 40108 | 43148 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118446 | 109426 | 71639 | 77669 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112554 | 121574 | 76331 | 82361 |

Based on Table 6.90.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 24

-	3rd order harmonic may fall into LTE frequencies of Band 30 and 40

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band  5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71 and 72

-	4th order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.90.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.90.3-2: 2UL Band 13 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | 2nd harmonics |
| Galileo | 1559 | - | 1591 | YES |  | 2nd harmonics |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | YES |  | 2nd harmonics |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 4th, 5th IMD |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.90.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.90.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13A-n260A | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 14 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: Table 6.5.3.2-1 of TS38.101 NOTE 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.90.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.90.4	∆TIB and ∆RIB values

For DC_13A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.90.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13A-n260A | 13 | 0 |
|  | n260 | 0 |

Table 6.90.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13A-n260A | 13 | 0 |
|  | n260 | 0 |

### 6.90.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.91	DC_48A-n260A

### 6.91.1	Operating bands for DC

Table 6.91.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48A-n260A | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.91.2	Channel bandwidths per operating band for DC

Table 6.91.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_48A-n260A | 48 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 220 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 420 |  |

### 6.91.3	Co-existence studies

Table 6.91.3-1 lists Band 48A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.91.3-1: Band 48 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 36450 | 33300 | 40550 | 43700 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 32900 | 29600 | 70300 | 76450 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 44100 | 47400 | 77550 | 83700 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 3150 | 4100 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | -72900 | -66600 | 81100 | 87400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -29350 | -25900 | 107300 | 116450 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 47650 | 51100 | 114550 | 123700 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -156450 | -144300 | 22200 | 25800 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 151550 | 163700 | 51200 | 54800 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -112900 | -103600 | 62900 | 69350 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 118100 | 127400 | 84650 | 91100 |

Based on Table 6.91.3-1

-	3rd order IMD products may fall into frequencies of Band 22, 42, 43, 76 and n260

-	4th order IMD products may fall into frequencies of band n257 and n258

-	5th order IMD products may fall into frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.91.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.91.3-2: 2UL Band 48 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.91.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.91.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48A-n260A | Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.91.4	∆TIB and ∆RIB values

For DC_48A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.91.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48A-n260A | 48 | 0 |
|  | n260 | 0 |

Table 6.91.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48A-n260A | 48 | 0 |
|  | n260 | 0 |

### 6.91.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.92	DC_5A-n261

### 6.92.1	Operating bands for DC

Table 6.92.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-n261 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

### 6.92.2	Channel bandwidths per operating band for DC

Table 6.92.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  | Fallback Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-n261B | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | n261 | 60 | See NR CA_n261B Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n261C | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See NR CA_n261C Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810* |  |  |
| DC_5A-n261D | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  | 2 |
|  | n261 | 60 | See NR CA_n261D Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |
| DC_5A-n261E | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n261 | 60 | See NR CA_n261E Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
| DC_5A-n261F | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n261 | 60 | See NR CA_n261F Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n261G | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |  | 3 |
|  | n261 | 60 | See NR CA_n261G Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 210 |  |  |
| DC_5A-n261H | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |  |  |
|  | n261 | 60 | See NR CA_n261H Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 310 |  |  |
| DC_5A-n261I | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  |  |
|  | n261 | 60 | See NR CA_n261I Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 410 |  |  |
| DC_5A-n261J | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |  |  |
|  | n261 | 60 | See NR CA_n261J Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 510 |  |  |
| DC_5A-n261K | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |  |  |
|  | n261 | 60 | See NR CA_n261K Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 610 |  |  |
| DC_5A-n261L | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |  |  |
|  | n261 | 60 | See NR CA_n261L Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 710 |  |  |
| DC_5A-n261M | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |  |  |
|  | n261 | 60 | See NR CA_n261M Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 810 |  |  |
| DC_5A-n261O | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |  | 4 |
|  | n261 | 60 | See NR CA_n261O Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 210 |  |  |
| DC_5A-n261P | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |  |  |
|  | n261 | 60 | See NR CA_n261P Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 310 |  |  |
| DC_5A-n261Q | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |  |  |
|  | n261 | 60 | See NR CA_n261Q Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 210 |  |  |

Note*:	The maximum bandwidth of band n261 is 850MHz.

Table 6.92.2-2: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | [40 MHz] | 50 MHz |  |  | 60 MHz] |  | 80 MHz | 100 MHz |  |  |  | 200 MHz | 400 MHz |  | Maximum aggregated bandwidth [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-n261(2A) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_ n261(2A) Bandwidth Combination in Table 8.10.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810* |  |
| DC_5A-n261(3A) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See NR CA_ n261(3A) Bandwidth Combination in Table 8.10.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810* |  |
| DC_5A-n261(4A) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See NR CA_ n261(4A) Bandwidth Combination in Table 8.10.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810* |  |
| DC_5A-n261(D-G) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
|  | n261 | 60 | See NR CA_ n261(D-G) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
| DC_5A-n261(D-H) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |
|  | n261 | 60 | See NR CA_ n261(D-H) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |
| DC_5A-n261(D-I) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  | n261 | 60 | See NR CA_ n261(D-I) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
| DC_5A-n261(D-O) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
|  | n261 | 60 | See NR CA_ n261(D-O) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 610 |  |
| DC_5A-n261(D-P) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |
|  | n261 | 60 | See NR CA_ n261(D-P) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 710 |  |
| DC_5A-n261(D-Q) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  | n261 | 60 | See NR CA_ n261(D-Q) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
| DC_5A-n261(E-O) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
|  | n261 | 60 | See NR CA_n261(E-O) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810 |  |
| DC_5A-n261(E-P) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810* |  |
|  | n261 | 60 | See NR CA_n261(E-P) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810* |  |
| DC_5A-n261(E-Q) | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810* |  |
|  | n261 | 60 | See NR CA_n261(E-Q) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 810* |  |

### 6.92.3	Co-existence studies

Table 6.92.3-1 lists Band 66A+Band n261A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.92.3-1: Band 5 and Band n261 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 27500 | 28350 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 55000 | 56700 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 82500 | 85050 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27526 | 26651 | 28324 | 29199 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26702 | 25802 | 54151 | 55876 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29148 | 30048 | 55824 | 57549 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 27490 | 28360 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55052 | 53302 | 56648 | 58398 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 25878 | 24953 | 81651 | 84226 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 29972 | 30897 | 83324 | 85899 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 112576 | 109151 | 24104 | 25054 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 110824 | 114249 | 30796 | 31746 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 83402 | 80802 | 52453 | 54228 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 84148 | 86748 | 57472 | 59247 |

Based on Table 6.92.3-1

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n257, n258 and n261

-	3rd order IMD products may fall into Rx frequencies of Band 5, n5, 6, 8, 12, n12, 13, n13, 14, 17, 18, 19, 20, n20, 26, 27 , 28, n28, 29, 31, 44, 67, 68, 71, n71, 72, 73, n81, n82 and n83

-	3rd order IMD products may fall into Rx frequencies of band n257 and n258

-	4th order IMD products may fall into Rx frequencies of band n258

-	5th order IMD products may fall into Rx frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.92.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.92.3-2: 2UL Band 5 +Band n261 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 5th IMD |
|  | 59000 | - | 64000 | YES | China | 5th IMD |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.92.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.92.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-n261A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | 41 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.92.4	∆TIB and ∆RIB values

For DC_5A-n261A, the TIB,c and RIB values are given in the tables below.

Table 6.92.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-n261A | 5 | 0 |
|  | n261 | 0 |

Table 6.92.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-n261A | 5 | 0 |
|  | n261 | 0 |

### 6.92.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.93	DC_66A-n261

### 6.93.1	Operating bands for DC

Table 6.93.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band |  | E-UTRA and NR Band |  | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  |  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_66A-n261 |  | 66 |  | 1710 MHz |  | – | 1780 MHz | 2110 MHz |  | – | 2220 MHz | FDD |  |
|  |  | n261 |  | 27500 MHz |  | – | 28350 MHz | 27500 MHz |  | – | 28350 MHz | TDD |  |

### 6.93.2	Channel bandwidths per operating band for DC

Table 6.93.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  | Fallback Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A-n261B | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 1 |
|  | n261 | 60 | See CA_n261B Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n261C | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261C Bandwidth Combination Fall-back Group 1 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 1220 |  |  |
| DC_66A-n261D | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  | 2 |
|  | n261 | 60 | See CA_n261D Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 420 |  |  |
| DC_66A-n261E | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |  |
|  | n261 | 60 | See CA_n261E Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |
| DC_66A-n261F | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |  |
|  | n261 | 60 | See CA_n261F Bandwidth Combination Fall-back Group 2 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n261G | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |  | 3 |
|  | n261 | 60 | See CA_n261G Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 220 |  |  |
| DC_66A-n261H | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |  |
|  | n261 | 60 | See CA_n261H Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 320 |  |  |
| DC_66A-n261I | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |  |
|  | n261 | 60 | See CA_n261I Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 420 |  |  |
| DC_66A-n261J | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |  |
|  | n261 | 60 | See CA_n261J Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 520 |  |  |
| DC_66A-n261K | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |  |
|  | n261 | 60 | See CA_n261K Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 620 |  |  |
| DC_66A-n261L | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |  |
|  | n261 | 60 | See CA_n261L Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 720 |  |  |
| DC_66A-n261M | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |  |
|  | n261 | 60 | See CA_n261M Bandwidth Combination Fall-back Group 3 in Table 6.X.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |
| DC_66A-n261O | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |  | 4 |
|  | n261 | 60 | See CA_n261B Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 220 |  |  |
| DC_66A-n261P | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |  |
|  | n261 | 60 | See CA_n261P Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 320 |  |  |
| DC_66A-n261Q | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |  |
|  | n261 | 60 | See CA_n261Q Bandwidth Combination Fall-back Group 4 in Table 6.X.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 420 |  |  |

Table 6.93.2-2: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A-n261(2A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  | n261 | 60 | See CA_ n261(2A) Bandwidth Combination in Table 8.10.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |
| DC_66A-n261(3A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  | n261 | 60 | See CA_ n261(3A) Bandwidth Combination in Table 8.10.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820* |  |
| DC_66A-n261(4A) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  | n261 | 60 | See CA_ n261(4A) Bandwidth Combination in Table 8.10.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820* |  |
| DC_66A-n261(D-G) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  | n261 | 60 | See CA_n261(D-G) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 620 |  |
| DC_66A-n261(D-H) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  | n261 | 60 | See CA_n261(D-H) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 720 |  |
| DC_66A-n261(D-I) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  | n261 | 60 | See CA_n261(D-I) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |
| DC_66A-n261(D-O) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  | n261 | 60 | See CA_ n261(D-O) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 620 |  |
| DC_66A-n261(D-P) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  | n261 | 60 | See CA_ n261(D-P) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 720 |  |
| DC_66A-n261(D-Q) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  | n261 | 60 | See CA_ n261(D-Q) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |
| DC_66A-n261(E-O) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  | n261 | 60 | See CA_n261(E-O) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |
| DC_66A-n261(E-P) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820* |  |
|  | n261 | 60 | See CA_n261(E-P) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820* |  |
| DC_66A-n261(E-Q) | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820* |  |
|  | n261 | 60 | See CA_n261(E-Q) Bandwidth Combination in Table 8.10.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820* |  |

### 6.93.3	Co-existence studies

Table 6.93.3-1 lists Band 66A+Band n261 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.93.3-1: Band 66 and Band n261 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 27500 | 28350 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 55000 | 56700 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 82500 | 85050 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 26640 | 25720 | 29210 | 30130 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 24930 | 23940 | 53220 | 54990 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30920 | 31910 | 56710 | 58480 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 27480 | 28370 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 53280 | 51440 | 58420 | 60260 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23220 | 22160 | 80720 | 83340 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32630 | 33690 | 84210 | 86830 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 111690 | 108220 | 20380 | 21510 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 111710 | 115180 | 34340 | 35470 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 81630 | 78940 | 49660 | 51570 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 85920 | 88610 | 60130 | 62040 |

Based on Table 6.93.3-1;

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48, n77, and n78

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, n1, 2, n2, 3, n3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, n50, 51, n51, 65, 66, n66, 70, n70, 74, n74, 75, n75, 76, n76, n80, and n84

-	3rd order IMD products may fall into own Rx frequencies of  band n257, n258 and n261

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.93.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.93.3-2: 2UL Band 66 +Band n261 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | YES | Europe | 3rd order harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |

Table 6.93.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.93.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A-n261 | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.93.4	∆TIB and ∆RIB values

For DC_66A-n261A, the TIB,c and RIB values are given in the tables below.

Table 6.93.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A-n261A | 66 | 0 |
|  | n261 | 0 |

Table 6.93.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66A-n261A | 66 | 0 |
|  | n261 | 0 |

### 6.93.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

# 6.94	DC_1A_n40A

### 6.94.1	Operating bands for DC

Table 6.94.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n40A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.94.2	Channel bandwidths per operating band for DC

Table 6.94.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n40A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 70 |
|  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |

### 6.94.3	Co-existence studies

Table 6.94.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 1+Band n40 2UL DC.

Table 6.94.3-1: Band 1 and Band n40 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 |  | 3960 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 |  | 5940 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 |  | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 |  | 9900 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 11520 |  | 11880 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 13440 |  | 13860 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 480 |  | 320 | 4220 | 4380 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 |  | 1660 | 2620 | 2880 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦e |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 |  | 6360 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 960 |  | 640 | 8440 | 8760 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 |  | 3640 | 4920 | 5280 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 |  | 8340 | 8820 | 9180 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 |  | 7220 | 5620 | 5280 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 |  | 11580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 |  | 2940 | 1340 | 960 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 |  | 11160 | 10360 | 10740 |

Based on Table 6.94.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands n77 and n79.

-	3rd order harmonic may fall into Rx frequencies of band 46 or 47.

-	2nd order IMD may fall into Rx frequencies of bands 31 or 72.

-	3rd order IMD may fall into Rx frequencies of bands 21, 24, 32, 38, 41, 45, 50, 69, 74 or 75.

-	4th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 17, 18, 19, 20, 22, 26, 27, 28, 29, 42, 43, 44, 46, 48, 49, 67, 68, 71, 77, 78 or 79.

-	5th order IMD may fall into Rx frequencies of bands 8, 46, 77 or 78.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.94.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.94.3-2: Band 1 and Band n40 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD4, IMD5 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | IMD4, IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.94.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.94.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_1A_n40A | Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |  |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |  |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |  |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27 |  |
| NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

### 6.94.4	∆TIB and ∆RIB values

For DC_1A_n40A, the TIB,c and RIB,c values are given in the tables below, and they are reused from LTE combination CA_1A-40A.

Table 6.94.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n40A | 1 | 0.5 |
|  | n40 | 0.5 |

Table 6.94.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1A_n40A | 1 | 0 |
|  | n40 | 0 |

### 6.94.5	REFSENS requirements

No IMD issues are expected for this DC configuration and furthermore, up to 7th order harmonics have no impact to Rx frequency in either the LTE band or the NR band. But according to TS 36.101 in table 7.3.1A-0bE, there is a cross band isolation issue for DC_1A_n40A. The MSD for DC_40_n1 is shown as below.

Table 6.94.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |

Table 6.94.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |
| n40 | 1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |

## 6.95	DC_2A_n5A

### 6.95.1	Operating bands for DC

Table 6.95.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A_n5A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

### 6.95.2	Channel bandwidths per operating band for DC

Table 6.95.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A_n5A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 40 |
|  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.95.3	Co-existence studies

Table 6.95.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 2+Band n5 2UL DC.

Table 6.95.3-1: Band 2 and Band n5 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 1850 | 1910 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 5550 | 5730 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 |  | 5550 | 5730 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 9250 | 9550 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4944 |  | 5094 | 11100 | 11460 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5768 |  | 5943 | 12950 | 13370 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 |  | 1001 | 2674 | 2759 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 262 |  | 152 | 2851 | 2996 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3498 |  | 3608 | 4524 | 4669 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2172 |  | 2002 | 5348 | 5518 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 562 |  | 697 | 4701 | 4906 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4322 |  | 4457 | 6374 | 6579 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6816 |  | 6551 | 1546 | 1386 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8224 |  | 8489 | 5146 | 5306 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4082 |  | 3852 | 1153 | 1348 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7198 |  | 7428 | 6172 | 6367 |

Based on Table 6.95.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 43, 48, 49, 77 and 78.

-	33d order harmonic may fall into Rx frequencies of bands 41 and 46.

-	2nd order IMD products may fall into Rx frequencies of bands 7 and 41.

-	33d order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77, 78 and 79.

-	4th order IMD products may fall into Rx frequencies of bands 1, 4, 10, 34, 46, 65, 66, 70, 71 and 79.

-	5th order IMD products may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 50, 51, 74, 75, 76 and 77.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.95.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.95.3-2: Band 2 and Band n5 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | Yes | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th, 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 4th IMD 3rd harmonic |
|  | 5150 | - | 5825 | Yes | Asia | 4th, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.95.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.95.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_2A_n5A | Bands 4, 5, 10, 12, 13, 14, 17, 24, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, n71, 74, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Bands 2, 25, 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |  |
|  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.95.4	∆TIB and ∆RIB values

For DC_2A_n5A, the TIB,c and RIB,c values are derived from LTE combination CA_2A-5A and are given in the tables below.

Table 6.95.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A_n5A | 2 | 0.3 |
|  | n5 | 0.3 |

Table 6.95.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A_n5A | 2 | 0 |
|  | n5 | 0 |

### 6.95.5	REFSENS requirements

Based on the co-existence studies in 6.95.3, MSD it not needed to be defined for DC_2A_n5A.

## 6.96	DC_2A_n66A

### 6.96.1	Operating bands for DC

Table 6.96.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A_n66A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.96.2	Channel bandwidths per operating band for DC

Table 6.96.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 60 |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.96.3	Co-existence studies

Table 6.96.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 2+Band n66 2UL DC.

Table 6.96.3-1: Band 2 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 1850 | 1910 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 5550 | 5730 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 7400 | 7640 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 9250 | 9550 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 11100 | 11460 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 12950 | 13370 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 |  | 70 | 3560 | 3690 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 |  | 1710 | 1920 | 2110 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 |  | 5470 | 5410 | 5600 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 400 |  | 140 | 7120 | 7380 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3220 |  | 3490 | 3770 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6980 |  | 7250 | 7260 | 7510 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 |  | 5620 | 5270 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 |  | 9420 | 8690 | 9030 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 |  | 1990 | 1640 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 |  | 9290 | 8830 | 9160 |

Based on Table 6.96.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77 and 78.

-	33d order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77 and 78.

-	33d order IMD products may fall into Rx frequencies of bands 1, 4, 10, 21, 24, 25, 33, 34, 36, 37, 39, 46, 50, 65, 70, 74, 75, or its own Rx frequencies of band 2 and 66.

-	4th order IMD products may fall into Rx frequencies of bands 42, 43, 77 and 78.

-	5th order IMD products may fall into Rx frequencies of bands 1, 4, 10, 11, 21, 23, 24, 25, 32, 34, 36, 40, 45, 46, 47, 50, 51, 65, 70, 74, 75, 76, 79, or its own Rx frequencies of band 66.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.96.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.96.3-2: Band 2 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd, 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd, 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 3rd, 5th IMD 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 3rd, 5th IMD 3rd harmonic |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.96.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.96.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_2A_n66A | Bands 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, n71, 74, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |  |  |
|  | Bands 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |  |

### 6.96.4	∆TIB and ∆RIB values

For DC_2A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_2A-66A and are given in the tables below.

Table 6.96.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A_n66A | 2 | 0.5 |
|  | n66 | 0.5 |

Table 6.96.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A_n66A | 2 | 0.3 |
|  | n66 | 0.3 |

### 6.96.5	REFSENS requirements

Due to 3rd and 5th order IMD MSD are needed and are defined in table 6.96.5-1. Values are reused from existing LTE combination CA_2A-66A.

Table 6.96.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |  |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A |  | N/A |  |
| DC_2A_n66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |  |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 |  | IMD5 |  |

### 6.96.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.96.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.96.6-1.

Table 6.96.6-1: Band 2 and Band n66 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n66A | 2 | a | 2 | b | -1 | 3 | 2 | IMD |
|  | n66 | c | -1 | d | 0 |  |  |  |
| DC_2A_n66A | 2 | a | 3 | b | 0 | 5 | n66 | IMD |
|  | n66 | c | -2 | d | -1 |  |  |  |

Equations (6.96.6-1) and (6.96.6-2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.96.6-1 and CBW stands for channel bandwidth.

(6.96.6-1)

(6.96.6-2)

Formula (6.96.6-3-1) and (6.96.6-3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.96.6-3-1)

(6.96.6-3-2)

## 6.97	DC_2A_n78A

### 6.97.1	Operating bands for DC

Table 6.97.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-n78A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.97.2	Channel bandwidths per operating band for DC

Table 6.97.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.97.3	Co-existence studies

Table 6.x.3-1 provides the self-interference study for DC_2A-n78A.

Table 6.x.3-1 Harmonics and IMD products

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | DL frequency range (MHz) | Rx Impact | Harmonic/IMD orders | Notes |
| --- | --- | --- | --- | --- | --- |
| DC_2A-n78A | 2 | 1930 MHz -1990 MHz | Yes | IMD2, IMD4, IMD5 |  |
|  | n78 | 3300 MHz -3800 MHz | Yes | 2th Harmonic |  |

Table 6.97.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.97.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A_n78A | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | NR Band 78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.97.3-3 and [Table x.x.x.x-x] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.97.4	∆TIB and ∆RIB values

For DC_2A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.97.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-n78A | 2 | 0.6 |
|  | n78 | 0.8 |

Table 6.97.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-n78A | 2 | 0.2 |
|  | n78 | 0.5 |

### 6.97.5	MSD

Tables below lists the MSD required for the dual connectivity configuration due to IMD and 2nd harmonic.

Table 6.5.5-1: MSD for the DC configuration for IMD2 and 4

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-n78A | 2 | 1855 | 5 | 25 | 1940 | 26 | FDD | IMD24 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n78 | 3795 | 10 | 25 | 3795 | N/A | TDD | N/A |
| DC_2A-n78A | 2 | 1885 | 5 | 25 | 1955 | [8.0] | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3700 | 10 | 25 | 3700 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

MSD values for Band n78 due to 2nd harmonic of Band 3 in LTE-NR DC_2-n78 are captured in Table 6.5.5-5.

Table 6.5.5-2: MSD due to harmonic issue for DC_2-n78

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 2 | n781,2 | 27.1 | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n783 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |

Regarding the uplink configuration, 40MHz for n78 should be considered additionally. Therefore, 20MHz with 100RB allocation for Band 2 should be added.

Table 6.5.5-3 Uplink configuration for the low band (exceptions) for E-UTRA

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | Duplex mode |
| DC_2-n78 | 2 |  |  | 12 | 26 | 39 | 531 | 1002 | FDD |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |

### 6.97.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.97.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.97.6-1.

Table 6.97.6-1: Band 2 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n78A | 2 | a | -1 | b | -1 | 2 | 2 | IMD |
|  | n78 | c | 1 | d | 0 |  |  |  |
| DC_2A_n78A | 2 | a | 3 | b | -1 | 4 | 2 | IMD |
|  | n78 | c | -1 | d | 0 |  |  |  |
| DC_2A_n78A | 2 | a | -3 | b | -1 | 5 | 2 | IMD |
|  | n78 | c | 2 | d | 0 |  |  |  |
| DC_2A_n78A | 2 | a | 2 | b | 0 | 2 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (6.97.6-1) and (6.97.6-2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.97.6-1 and CBW stands for channel bandwidth.

(6.97.6-1)

(6.97.6-2)

Formula (6.97.6-3-1) and (6.97.6-3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.97.6-3-1)

(6.97.6-3-2)

## 6.98	DC_2A_n260A and DC_2A_n260(2A)

### 6.98.1	Operating bands for DC

Table 6.98.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A_n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.98.2	Channel bandwidths per operating band for DC

Table 6.98.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A_n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 (220 for Band n260 with 60kHz SCS) |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_2A_n260(2A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 (420 for Band n260 with 60 kHz SCS) |
|  | n260 | 60 | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 [4] |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260G | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260H | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260I | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260J | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260K | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260L | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260M | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260(A-I) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n260(G-I) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.98.3	Co-existence studies

Table 6.98.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 2+Band n260 2UL DC.

Table 6.98.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 |  | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 |  | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 |  | 5730 | 111000 | 120000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 |  | 148000 | 160000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 |  | 9550 | 185000 | 200000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 11100 |  | 11460 | 222000 | 240000 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 12950 |  | 13370 | 259000 | 280000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 |  | 35090 | 38850 | 41910 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 |  | 33180 | 72090 | 78150 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 |  | 43820 | 75850 | 81910 |
| 4rd order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 |  | 70180 | 77700 | 83820 |
| 4rd order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 |  | 31270 | 109090 | 118150 |
| 4rd order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 |  | 45730 | 112850 | 121910 |
| 5rd order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 |  | 146090 | 29360 | 32600 |
| 5rd order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 |  | 161910 | 44400 | 47640 |
| 5rd order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 |  | 107180 | 68270 | 74450 |
| 5rd order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 |  | 123820 | 79550 | 85730 |

Based on Table 6.98.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 43, 48, 49, n77 and n78.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD may fall into own Rx frequencies of band n260 (which can be disregarded, since no self-interference in TDD).

-	4th order IMD may fall inteo RX frequency of band n259.

-	5th order IMD may fall inteo RX frequency of bands n257 or n259.

Therefore, it can be concluded that there is no harmonic interference and IMD product falling into their own Rx bands.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.98.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.98.3-2: Band 2 and Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | 3rd harmonic |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | Yes | China | 3rd, 4th, 5th IMD |
|  | 47200 | - | 48400 | Yes | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.98.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.98.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_2A_n260A | Bands 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 42, 48, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.98.4	∆TIB and ∆RIB values

For DC_2A_n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.98.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A_n260A | 2 | 0 |
|  | n260 | 0 |

Table 6.98.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A_n260A | 2 | 0 |
|  | n260 | 0 |

### 6.98.5	REFSENS requirements

There is no specific REFSENS requirement.

## 6.99	DC_3A_n3A

### 6.99.1	Operating bands for DC

Table 6.99.1-1: DC band combination of LTE 1DL/1UL + one NR band

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n3 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 6.99.2	Channel bandwidths per operating band for DC

Table 6.99.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| CA operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n3A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 50 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 6.99.3	UE co-existence studies

Table 6.99.3-1 gives the UL 2nd, 3rd harmonics for DC_3A_n3A.

Table 6.99.3-1: Band 3 and Band n3 UL harmonics products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3570 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5355 | 5130 | 5355 |

Based on Table 6.99.3-1;

2nd order harmonic may fall into Rx frequency of bands 22, 42, 48, n77 and n78

3rd order harmonic may fall into Rx frequencies of bands 46

In Rel.15, Only Single Tx Switched UL mode is supported for this combination, hence, no IMD analysis is need.

### 6.99.4	ΔTIB and ΔRIB values

No dTib or dRib is required for DC_3A_n3A as it is intra-band DC.

### 6.99.5	MSD, A-MPR

Note that only Single Tx Switched UL mode is supported for this combination in Rel.15, neither MSD nor A-MPR requirement is needed.

### 6.99.6	Bandwidth Combination Sets

For intra-band EN-DC combination, it was agreed to allow introduction of bandwidth combination sets. The bandwidth combination set 0 of DC_3A_n3A in Rel.15 is shown in Table 6.99.6-1.

Table 6.99.6-1: Bandwidth Combination Sets for DC_3A_n3A

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for LTE carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |
| DC_3A_n3A | DC_3A_n3A(1) |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 0 |

### 6.99.7	delta R INBC values

For the UE which supports intra-band EN-DC DC_3A_n3A configuration, the minimum requirement for reference sensitivity shall be increased by the amount given in ΔRIBNC in Table 6.99.7-1. According to the bandwidth combination set in Table 6.99.6-1, the LTE UL will always be closer to the DL band, so that the UL allocation is for the LTE part, and the ΔRIBNC is applied to the NR part.

Table 6.99.7-1: Delta R IBNC for DC_3A_n3A

| DC configuration | Aggregated channel bandwidth (LTE+NR) | Wgap / (MHz) | UL LTE allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| DC_3A_n3A | 5MHz+5MHz | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 | FDD |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 5MHz+10MHz | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 5MHz+15MHz | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 5MHz+20MHz | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 5MHz+25MHz | 25.0 < Wgap ≤ 45.0 | 121 | 3.2 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 5MHz+30MHz | 20.0 < Wgap ≤ 40.0 | 121 | 3.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 10MHz+5MHz | 30.0 < Wgap ≤ 60.0 | 125 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 10MHz+10MHz | 25.0 < Wgap ≤ 55.0 | 125 | 4.3 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 10MHz+15MHz | 20.0 < Wgap ≤ 50.0 | 125 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 10MHz+20MHz | 15.0 < Wgap ≤ 45.0 | 125 | 3.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 10MHz+25MHz | 10.0 < Wgap ≤ 40.0 | 125 | 3.2 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 10MHz+30MHz | 5.0 < Wgap ≤ 35.0 | 125 | 2.8 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15MHz+5MHz | 25.0 < Wgap ≤ 55.0 | 126 | 6.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 15MHz+10MHz | 20.0 < Wgap ≤ 50.0 | 126 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 15MHz+15MHz | 15.0 < Wgap ≤ 45.0 | 126 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15MHz+20MHz | 10.0 < Wgap ≤ 40.0 | 126 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 15MHz+25MHz | 5.0 < Wgap ≤ 35.0 | 126 | 3.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15MHz+30MHz | 0.0 < Wgap ≤ 30.0 | 126 | 3.3 |  |
|  | 20MHz+5MHz | 15.0 < Wgap ≤ 50.0 | 167 | 6.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 20MHz+10MHz | 10.0 < Wgap ≤ 45.0 | 167 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20MHz+15MHz | 5.0 < Wgap ≤ 40.0 | 167 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 20MHz+20MHz | 0.0 < Wgap ≤ 35.0 | 167 | 4.1 |  |
|  | 20MHz+25MHz | 0.0 < Wgap ≤ 30.0 | 167 | 3.8 |  |
|  | 20MHz+30MHz | 0.0 < Wgap ≤ 25.0 | 167 | 3.6 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=25. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=35. NOTE 7: 7 refers to the UL resource blocks shall be located at RBstart=50. |  |  |  |  |  |

## 6.100	DC_3A_n40A

### 6.100.1	Operating bands for DC

Table 6.100.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n40A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.100.2	Channel bandwidths per operating band for DC

Table 6.100.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n40A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 70 |
|  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |

### 6.100.3	Co-existence studies

Table 6.100.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 3+Band n40 2UL DC.

Table 6.100.3-1: Band 3 and Band n40 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3570 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5355 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 |  | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8925 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10710 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12495 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 690 |  | 515 | 4010 | 4185 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1020 |  | 1270 | 2815 | 3090 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5720 |  | 5970 | 6310 | 6585 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1380 |  | 1030 | 8020 | 8370 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2730 |  | 3055 | 5115 | 5490 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7430 |  | 7755 | 8610 | 8985 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7890 |  | 7415 | 4840 | 4440 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10910 |  | 11385 | 9140 | 9540 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3780 |  | 3330 | 755 | 330 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10320 |  | 10770 | 9730 | 10155 |

Based on Table 6.100.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77, 78 and 79.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD may fall into Rx frequencies of bands 71 or 77.

-	3rd order IMD may fall into Rx frequencies of bands 46 or 47.

-	4th order IMD may fall into Rx frequencies of band 46.

-	5th order IMD may fall into Rx frequencies of bands 42, 43, 44, 48, 49, 67, 68, 71, 77, 78, 79.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.100.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.100.3-2: Band 3 and Band n40 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD4 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, IMD4 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | IMD3, IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD4 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.100.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.100.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_3A_n40A | Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |  |
|  | Band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |  |

### 6.100.4	∆TIB and ∆RIB values

For DC_3A_n40A, the TIB,c and RIB,c values are given in the tables below, and they are reused from LTE combination CA_3A-40A.

Table 6.100.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n40A | 3 | 0.5 |
|  | n40 | 0.5 |

Table 6.100.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3A_n40A | 3 | 0 |
|  | n40 | 0 |

### 6.100.5	REFSENS requirements

No IMD issues are expected for this DC configuration and furthermore, up to 7th order harmonics have no impact to Rx frequency in either the LTE band or the NR band.

## 6.101	DC_5A_n40A

### 6.101.1	Operating bands for DC

Table 6.101.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A_n40A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.101.2	Channel bandwidths per operating band for DC

Table 6.101.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n40A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |

### 6.101.3	Co-existence studies

Table 6.101.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 5+Band n40 2UL DC.

Table 6.101.3-1: Band 5 and Band n40 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 |  | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 |  | 5094 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 |  | 5943 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1576 |  | 1451 | 3124 | 3249 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 752 |  | 602 | 3751 | 3976 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3948 |  | 4098 | 5424 | 5649 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3152 |  | 2902 | 6248 | 6498 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 72 |  | 247 | 6051 | 6376 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4772 |  | 4947 | 7724 | 8049 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8776 |  | 8351 | 1096 | 896 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10024 |  | 10449 | 5596 | 5796 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5552 |  | 5202 | 2053 | 2328 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8548 |  | 8898 | 7072 | 7347 |

Based on Table 6.101.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band 79.

-	3rd order harmonic may fall into Rx frequencies of band 41.

-	2nd order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 50, 74 and 75.

-	3rd order IMD may fall into Rx frequencies of bands 12, 13, 17, 29, 43, 44, 46, 67, 71, 77, 78.

-	4th order IMD may fall into Rx frequencies of band 79.

-	5th order IMD may fall into Rx frequencies of bands 1, 4, 8 10, 23, 46, 65 and 66, and also the own RX frequency of band 40..

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.101.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.101.3-2: Band 5 and Band n40 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | Yes | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3. IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD3, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD5 |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.101.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.101.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_5A_n40A | Band 1, 3, 5, 7, 8, 28, 31, 34, 38, 42, 43, 45, 65, 73 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |  |
|  | Band 41, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |  |

### 6.101.4	∆TIB and ∆RIB values

For DC_5A_n40A, the TIB,c and RIB,c values are given in the tables below, and they are reused from LTE combination CA_5A-40A.

Table 6.101.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A_n40A | 5 | 0.3 |
|  | n40 | 0.3 |

Table 6.101.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5A_n40A | 5 | 0 |
|  | n40 | 0 |

### 6.101.5	REFSENS requirements

No issues up to 4th order IMD are expected for this DC configuration and furthermore, up to 7th order harmonics have no impact to Rx frequency in either the LTE band or the NR band.

## 6.102	DC_5A_n66A

### 6.102.1	Operating bands for DC

Table 6.102.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A_n66A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.102.2	Channel bandwidths per operating band for DC

Table 6.102.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n66A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 50 |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.102.3	Co-existence studies

Table 6.102.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 5+Band n66 2UL DC.

Table 6.102.3-1: Band 5 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 |  | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 8550 | 8900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 |  | 5094 | 10260 | 10680 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 |  | 5943 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 956 |  | 861 | 2534 | 2629 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 132 |  | 12 | 2571 | 2736 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 |  | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1912 |  | 1722 | 5068 | 5258 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 692 |  | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4182 |  | 4327 | 5954 | 6189 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6296 |  | 5991 | 1686 | 1516 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 |  | 7969 | 5006 | 5176 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3692 |  | 3432 | 873 | 1088 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 |  | 7038 | 5892 | 6107 |

Based on Table 6.102.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77 and 78.

-	33d order harmonic may fall into Rx frequencies of bands 41 and 46.

-	2nd order IMD products may fall into Rx frequencies of bands 6, 7, 8, 18, 19, 26, 27, 38, 41 and 69, or its own RX frequency in band 5.

-	33d order IMD products may fall into Rx frequencies of bands 7, 38, 41, 42, 69, 77, 78 and 79.

-	4th order IMD products may fall into Rx frequencies of bands 3, 9, 12, 13, 17, 20, 28, 29, 33, 35, 37, 39, 44, 46, 67, 68, 77 and 79.

-	5th order IMD products may fall into Rx frequencies of bands 6, 8, 18, 19, 22, 24, 26, 42, 43, 46, 47, 48, 49, 50, 74, 75, 77 and 78, or its own RX frequency in band 5.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.102.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.102.3-2: Band 5 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | Yes | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th, 5th IMD 3rd harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 4th, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.102.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.102.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_5A_n66A | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |  |
|  | Bands 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 5 |  |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2 |  |  |  |  |  |  |  |  |

### 6.102.4	∆TIB and ∆RIB values

For DC_5A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_5A-66A and are given in the tables below.

Table 6.102.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A_n66A | 5 | 0.3 |
|  | n66 | 0.3 |

Table 6.102.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5A_n66A | 5 | 0 |
|  | n66 | 0 |

### 6.102.5	REFSENS requirements

Due to 2nd order IMD MSD are needed and are defined in table 6.102.5-1. Values are reused from existing LTE combination CA_5A-66A.

Table 6.102.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD21 |  |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |  |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

### 6.102.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.102.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.102.6-1.

Table 6.102.6-1: Band 5 and Band n66 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n66A | 5 | a | -1 | b | -1 | 2 | 5 | IMD |
|  | n66 | c | 1 | d | 0 |  |  |  |
| DC_5A_n66A | 5 | a | -3 | b | -1 | 5 | 5 | IMD |
|  | n66 | c | 2 | d | 0 |  |  |  |

Equations (6.102.6-1) and (6.102.6-2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.102.6-1 and CBW stands for channel bandwidth.

(6.102.6-1)

(6.102.6-2)

Formula (6.102.6-3-1) and (6.102.6-3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.102.6-3-1)

(6.102.6-3-2)

## 6.103	DC_8A_n40A

### 6.103.1	Operating bands for DC

Table 6.103.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n40A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.103.2	Channel bandwidths per operating band for DC

Table 6.103.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n40A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |

### 6.103.3	Co-existence studies

Table 6.103.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 8+Band n40 2UL DC.

Table 6.103.3-1: Band 8 and Band n40 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 |  | 1830 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 |  | 2745 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 |  | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 |  | 4575 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 |  | 5490 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 6160 |  | 6405 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 |  | 1385 | 3180 | 3315 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 640 |  | 470 | 3685 | 3920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4060 |  | 4230 | 5480 | 5715 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3040 |  | 2770 | 6360 | 6630 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 240 |  | 445 | 5985 | 6320 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4940 |  | 5145 | 7780 | 8115 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8720 |  | 8285 | 1360 | 1120 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10080 |  | 10515 | 5820 | 6060 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5440 |  | 5070 | 1855 | 2160 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8660 |  | 9030 | 7240 | 7545 |

Based on Table 6.103.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 3 and 79.

-	3rd order harmonic may fall into Rx frequencies of bands 7 and 41.

-	2nd order IMD may fall into Rx frequencies of bands 11, 21, 32, 45, 50, 51, 74, 75, 76, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 43, 46, 48, 49, 71, 77 and 78.

-	4th order IMD may fall into Rx frequencies of band 79.

-	5th order IMD may fall into Rx frequencies of bands 1,2, 3, 4, 9, 10, 25, 33, 34, 35, 36, 37, 39, 46, 47, 65, 66 and 70.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.103.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.103.3-2: Band 8 and Band n40 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD3, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD5 |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.103.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.103.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_8A_n40A | Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Band 3, 7, 22, 41, 42, 43, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |  |
|  | Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. |  |  |  |  |  |  |  |  |

### 6.103.4	∆TIB and ∆RIB values

For DC_8A_n40A, the TIB,c and RIB,c values are given in the tables below, and they are reused from LTE combination CA_8A-40A.

Table 6.103.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A_n40A | 8 | 0.3 |
|  | n40 | 0.3 |

Table 6.103.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_8A_n40A | 8 | 0 |
|  | n40 | 0 |

### 6.103.5	REFSENS requirements

No IMD issues are expected for this DC configuration and furthermore, up to 7th order harmonics have no impact to Rx frequency in either the LTE band or the NR band.

## 6.104	DC_12A_n5A

### 6.104.1	Operating bands for DC

Table 6.104.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12A_n5A | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

### 6.104.2	Channel bandwidths per operating band for DC

Table 6.104.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n5A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 30 |
|  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.104.3	Co-existence studies

Table 6.104.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 12+Band n5 2UL DC.

Table 6.104.3-1: Band 12 and Band n5 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 |  | 824 | 849 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1398 |  | 1432 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2097 |  | 2148 | 2472 | 2547 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 |  | 3296 | 3396 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3495 |  | 3580 | 4120 | 4245 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4194 |  | 4296 | 4944 | 5094 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 4893 |  | 5012 | 5768 | 5943 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 150 |  | 108 | 1523 | 1565 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 549 |  | 608 | 932 | 999 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2222 |  | 2281 | 2347 | 2414 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 300 |  | 216 | 3046 | 3130 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1248 |  | 1324 | 1756 | 1848 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2921 |  | 2997 | 3171 | 3263 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2697 |  | 2580 | 2040 | 1947 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3995 |  | 4112 | 3620 | 3713 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1149 |  | 1040 | 500 | 399 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3870 |  | 3979 | 3745 | 3846 |

Based on Table 6.104.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 50, 51, 75 and 76.

-	33d order harmonic may fall into Rx frequencies of bands 1, 4, 10, 41, 65 and 66.

-	2nd order IMD products may fall into Rx frequencies of band 24.

-	33d order IMD products may fall into Rx frequencies of bands 8, 30 and 40.

-	4th order IMD products may fall into Rx frequencies of bands 3 and 9.

-	5th order IMD products may fall into Rx frequencies of bands 2, 7 25, 31, 34, 36, 38, 41, 43, 48, 49, 69, 70, 72, 77 and 78.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.104.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.104.3-2: Band 12 and Band n5 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 2nd IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 2nd IMD |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | 2nd IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd IMD 3rd harmonic |
|  | 2400 | - | 2494 | Yes | Asia | 3rd IMD 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.104.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.104.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_12A_n5A | Bands 2, 5, 12, 13, 14, 17, 24, 25, 30, 42, 43 50, 51, 71, n71, 74, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Bands 4, 10, 41, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |  |  |
|  | Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.104.4	∆TIB and ∆RIB values

For DC_12A_n5A, the TIB,c and RIB,c values are derived from LTE combination CA_5A-12A and are given in the tables below.

Table 6.104.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12A_n5A | 12 | 0.4 |
|  | n5 | 0.8 |

Table 6.104.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12A_n5A | 12 | 0.3 |
|  | n5 | 0.5 |

### 6.104.5	REFSENS requirements

Based on the co-existence studies in 6.104.3, MSD it not needed to be defined for DC_12A_n5A.

## 6.105	DC_12A_n66A

### 6.105.1	Operating bands for DC

Table 6.105.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12A_n66A | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.105.2	Channel bandwidths per operating band for DC

Table 6.105.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n66A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 50 |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.105.3	Co-existence studies

Table 6.105.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 12+Band n66 2UL DC.

Table 6.105.3-1: Band 12 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 |  | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 |  | 2148 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 |  | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 |  | 3580 | 8550 | 8900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4194 |  | 4296 | 10260 | 10680 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4893 |  | 5012 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1081 |  | 994 | 2409 | 2496 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 |  | 278 | 2704 | 2861 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 |  | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2162 |  | 1988 | 4818 | 4992 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 317 |  | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3807 |  | 3928 | 5829 | 6056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 |  | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 |  | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 |  | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 |  | 6772 | 5517 | 5708 |

Based on Table 6.105.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 50, 51, 75, 76, 77, 78 and 79.

-	33d order harmonic may fall into Rx frequencies of bands 1, 4, 10, 46 and 65 or its own RX frequency in band 66.

-	2nd order IMD products may fall into Rx frequencies of band 41.

-	33d order IMD products may fall into Rx frequencies of band 77.

-	4th order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 25, 34, 36, 46, 47, 65, 70, 77 and 79 or its own RX frequency in band 66.

-	5th order IMD products may fall into Rx frequencies of bands 32, 43, 45, 46, 48, 49, 50, 51, 75, 76, 77, 78 and 79.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.105.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.105.3-2: Band 12 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 2nd IMD |
|  | 2400 | - | 2494 | Yes | Asia | 2nd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 5th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.105.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.105.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_12A_n66A | Bands 2, 4, 5, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 70, 71, n71, 74, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Bands 4, 10, 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
|  | Bands 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth |  |  |  |  |  |  |  |  |

### 6.105.4	∆TIB and ∆RIB values

For DC_12A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_12A-66A and are given in the tables below.

Table 6.105.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12A_n66A | 12 | 0.8 |
|  | n66 | 0.3 |

Table 6.105.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12A_n66A | 12 | 0.5 |
|  | n66 | 0 |

### 6.105.5	REFSENS requirements

Table 6.105.5-1 lists the MSD required for the dual connectivity configuration due to 3rd harmonic of Band 12, reusing values specified in LTE for CA_12A-66A.

Table 6.105.5-1: MSD due to 3rd harmonic issue for DC_12A-n66A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |  |
|  |  | dB | dB | dB | dB | dB |  |
| 12 | n661,2,3 | 10 | 7.5 | 6.2 | 5.5 | 2.4 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 3: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |

Table 6.105.5-2 Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | Duplex mode |
| DC_12A_n66A | 12 | 8 | 16 | 20 | 20 | 20 | FDD |

### 6.105.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.105.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.105.6-1.

Table 6.105.6-1: Band 12 and Band n66 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n66A | 12 | a | 3 | b | 0 | 3 | n66 | Harmonic |
|  | n66 | c | 0 | d | -1 |  |  |  |

Equations (6.105.6-1) and (6.105.6-2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.105.6-1 and CBW stands for channel bandwidth.

(6.105.6-1)

(6.105.6-2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.105.6-3-1)

(6.105.6-3-2)

## 6.106	DC_12A_n260A

### 6.106.1	Operating bands for DC

Table 6.106.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12A_n260A | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.106.2	Channel bandwidths per operating band for DC

Table 6.106.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n260A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 (210 for Band n260 with 60 kHz SCS) |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_12A_n260G | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260H | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260I | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260J | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260K | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260L | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260M | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260(A-I) | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n260(G-I) | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.106.3	Co-existence studies

Table 6.106.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 12+Band n260 2UL DC.

Table 6.106.3-1: Band 12 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 619 | 716 |  | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 |  | 1432 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 |  | 2148 | 111000 | 120000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 |  | 148000 | 160000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3095 |  | 3580 | 185000 | 200000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 3714 |  | 4296 | 222000 | 240000 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4333 |  | 5012 | 259000 | 280000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39301 |  | 36284 | 37699 | 40716 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38602 |  | 35568 | 73284 | 79301 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38398 |  | 41432 | 74699 | 80716 |
| 4rd order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78602 |  | 72568 | 75398 | 81432 |
| 4rd order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37903 |  | 34852 | 110284 | 119301 |
| 4rd order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39097 |  | 42148 | 111699 | 120716 |
| 5rd order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159301 |  | 147284 | 34136 | 37204 |
| 5rd order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148699 |  | 160716 | 39796 | 42864 |
| 5rd order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118602 |  | 109568 | 71852 | 77903 |
| 5rd order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112398 |  | 121432 | 76097 | 82148 |

Based on Table 6.106.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 50, 51, n75 and n76.

-	3rd order harmonic may fall into Rx frequencies of band 1, 4, 10, 65 or 66.

-	2nd 3rd 4th and 5th order IMD may fall into own Rx frequencies of band n260 (which can be disregarded, since no self-interference in TDD).

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.106.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.106.3-2: Band 12 and Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.106.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.106.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_12A_n260A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 41, 48, 71 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | E-UTRA Band 4, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.106.4	∆TIB and ∆RIB values

For DC_12A_n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.106.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12A_n260A | 12 | 0 |
|  | n260 | 0 |

Table 6.106.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12A_n260A | 12 | 0 |
|  | n260 | 0 |

### 6.106.5	REFSENS requirements

No IMD issues are expected for DC configuration between the LTE bands below 6 GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency in either the LTE band or the NR band.

## 6.107	DC_20A-n77A

### 6.107.1	Operating bands for DC

Table 6.107.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A-n77A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.107.2	Channel bandwidths per operating band for DC

Table 6.107.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A-n77A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.107.3	Co-existence studies

For 2UL/2DL UE coexistence study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.107.3-1

Table 6.45.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 | 3368 | 4132 | 5062 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 | 1576 | 5738 | 7568 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 | 5924 | 7432 | 9262 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1704 | 714 | 9038 | 11768 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5796 | 6786 | 10732 | 13462 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 6736 | 4876 | 8264 | 10124 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 | 12338 | 148 | 872 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 | 17662 | 6628 | 7648 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 | 8176 | 4014 | 5904 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 | 14324 | 9096 | 10986 |

Based on the table 6.107.3-1, the 4th and 5th order IMD products may fall into the own Rx of Band 20, and 2nd and 5th order IMD may fall into the own RX of band n77. It should be noted that IMD will not be an issue for n77 (no self-interference) even through the IMD products may fall into the concerning band.

In addition, 3rd order harmonic and 2nd, 3rd, 4th and 5th IMD products may also fall into Rx frequencies of bands 1-14, 17-21, 23-31, 33-41, 44-70 and n77.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.107.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.45.3-2: 2UL Band x +Band y harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD 2, 3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD 2, 3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4, 5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4, 5 |

Table 6.107.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.107.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_20A-n77A | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  |  |  |  |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.107.4	∆TIB and ∆RIB values

For DC_20A-n77A, the TIB,c and RIB values for DC_20A-n78A are reused.

Table 6.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-n77A | 20 | 0.6 |
|  | n77 | 0.8 |

Table 6.107.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-n77A | 20 | 0 |
|  | n77 | 0.5 |

### 6.107.5	MSD

Based on the results in table 6.107.3-1, the 4th and 5th order IMD products may fall into the own Rx of Band 20.

Table 6.107.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.107.5-1: MSD due to IMD issue for DC_20-n77A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_20A-n77A | 20 | 850 | 5 | 25 | 810 | 11 | FDD | IMD44 |
|  | n77 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |
| DC_20A-n78A | 20 | 840 | 5 | 25 | 800 | 6.5 | FDD | IMD5 |
|  | n77 | 4160 | 10 | 50 | 4150 | N/A | TDD | N/A |

The MSD level for Band n77 by 4th harmonic from Band 20 can reuse the value from LTE CA_20A-42A. Table 6.45.5-2 shows the MSD level for DC_20A-n77A. The MSD for 40MHz was calculated based on 20MHz, since the interference are same.

Table 6.107.5-2: MSD due to harmonic issue for DC_20-n77A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 20 | n771,2 |  | 10.8 | 9.1 | 8 |  | 6 |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |

Regarding the uplink configuration, 40MHz for n77 should be considered additionally. Therefore, 20MHz with 100RB allocation for Band 20 should be added.

Table 6.107.5-3: Uplink configuration for the low band (exceptions) for E-UTRA

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | Duplex mode |
| DC_20A-n77A | 20 |  |  | 8 | 16 | 25 | 251 | 252 | FDD |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |

### 6.107.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.107.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.107.6-1.

Table 6.107.6-1: Band 20 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n77A | 20 | a | -3 | b | -1 | 4 | 20 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_20A_n77A | 20 | a | -4 | b | -1 | 5 | 20 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_20A_n77A | 20 | a | 4 | b | 0 | 4 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |
| DC_20A_n77A | 20 | a | 5 | b | 0 | 5 | n77 | Harmonic |
|  | n77 | c | 0 | d | -1 |  |  |  |

Equations (6.107.6-1) and (6.107.6-2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.107.6-1 and CBW stands for channel bandwidth.

(6.107.6-1)

(6.107.6-2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.107.6-3-1)

(6.107.6-3-2)

## 6.108	DC_20A_n258A

### 6.108.1	Operating bands for DC

Table 6.108.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A-n258A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.108.2	Channel bandwidths per operating band for DC

Table 6.108.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A-n258A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 420 |
|  | n258 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.108.3	Co-existence studies

The 2nd, 3rd and 4th IMD will be impact to the own received band n258. However, the NR TDD band has no impact from dual uplink transmission since there was no simultaneous Tx/Rx transmission in the band n258.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.108.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.108.3-2: 2UL Band 20 +Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.108.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.108.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_20A_n258A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 38, 42, 52, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 of TS 36.101 [2] from the edge of the channel bandwidth. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

### 6.108.4	∆TIB and ∆RIB values

For DC_20A-n258A, the TIB,c and RIB values are given in the tables below.

Table 6.108.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-n258A | 20 | 0 |
|  | n258 | 0 |

Table 6.108.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-n258A | 20 | 0 |
|  | n258 | 0 |

### 6.108.5	MSD

From the coexistence analysis Table 6.108.1-1, there was no harmonic/IMD problem into own Rx of bands for DC_20A-n258A UE.

## 6.109	DC_30A_n5A

### 6.109.1	Operating bands for DC

Table 6.109.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_30A_n5A | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

### 6.109.2	Channel bandwidths per operating band for DC

Table 6.109.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_30A_n5A | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 30 |
|  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.109.3	Co-existence studies

Table 6.109.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 30+Band n5 2UL DC.

Table 6.109.3-1: Band 30 and Band n5 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 2305 | 2315 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 6915 | 6945 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 |  | 9220 | 9260 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 11525 | 11575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4944 |  | 5094 | 13830 | 13890 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5768 |  | 5943 | 16135 | 16205 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1491 |  | 1456 | 3129 | 3164 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 667 |  | 607 | 3761 | 3806 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3953 |  | 4013 | 5434 | 5479 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2982 |  | 2912 | 6258 | 6328 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 157 |  | 242 | 6066 | 6121 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4777 |  | 4862 | 7739 | 7794 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8436 |  | 8371 | 1091 | 981 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10044 |  | 10109 | 5601 | 5711 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5297 |  | 5217 | 2063 | 2158 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8563 |  | 8643 | 7082 | 7177 |

Based on Table 6.109.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band 79.

-	33d order harmonic may fall into Rx frequencies of band 41.

-	2nd order IMD products may fall into Rx frequencies of band 11, 32, 45, 50, 74 and 75.

-	33d order IMD products may fall into Rx frequencies of bands 43, 46, 71, 77 and 78.

-	4th order IMD products may fall into Rx frequencies of band 79.

-	5th order IMD products may fall into Rx frequencies of bands 1, 4, 10, 46, 65 and 66.

It is concluded that there is no harmonic interference and IMD product falling into their own Rx bands.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.109.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.109.3-2: Band 30 and Band n5 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | Yes | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 3rd, 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 5th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.109.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.109.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_30A_n5A | Bands 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |  |  |
|  | Bands 41, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 5 |  |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2 |  |  |  |  |  |  |  |  |

### 6.109.4	∆TIB and ∆RIB values

For DC_30A_n5A, the TIB,c and RIB,c values are derived from LTE combination CA_30A-5A and are given in the tables below.

Table 6.109.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30A_n5A | 30 | 0.3 |
|  | n5 | 0.3 |

Table 6.109.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_30A_n5A | 30 | 0 |
|  | n5 | 0 |

### 6.109.5	REFSENS requirements

Based on the findings in the co-existence studies in clause 6.109.3, no IMD issues are expected for this DC configuration.

## 6.110	DC_30A_n66A

### 6.110.1	Operating bands for DC

Table 6.110.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_30A_n66A | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.110.2	Channel bandwidths per operating band for DC

Table 6.110.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_30A_n66A | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 50 |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.110.3	Co-existence studies

Table 6.110.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 30+Band n66 2UL DC.

Table 6.110.3-1: Band 30 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 2305 | 2315 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 6915 | 6945 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 9220 | 9260 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 11525 | 11575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 13830 | 13890 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 16135 | 16205 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 605 |  | 525 | 4015 | 4095 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1105 |  | 1255 | 2830 | 2920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5725 |  | 5875 | 6320 | 6410 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1210 |  | 1050 | 8030 | 8190 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2815 |  | 3035 | 5135 | 5235 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7435 |  | 7655 | 8625 | 8725 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7550 |  | 7440 | 4815 | 4525 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10930 |  | 11040 | 9145 | 9435 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3525 |  | 3355 | 730 | 500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10335 |  | 10505 | 9740 | 9970 |

Based on Table 6.110.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77, 78 or 79.

-	33d order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of band 77

-	33d order IMD products may fall into Rx frequencies of bands 46 or 47

-	4th order IMD products may fall into Rx frequencies of band 46.

-	5th order IMD products may fall into Rx frequencies of bands 12, 22, 29, 42, 44, 71, 77, 78 or 79.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.110.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.110.3-2: Band 30 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 4th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th IMD 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 3rd IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 4th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.110.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.110.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_30A_n66A | Bands 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.110.4	∆TIB and ∆RIB values

For DC_30A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_30A-66A and are given in the tables below.

Table 6.110.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30A_n66A | 30 | 0.5 |
|  | n66 | 0.8 |

Table 6.110.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_30A_n66A | 30 | 0.5 |
|  | n66 | 0.4 |

### 6.110.5	REFSENS requirements

Referring to DC_1A_n40A, according to TS 36.101 in table 7.3.1A-0bE, there is a cross band isolation issue for DC_30A_n66A. The MSD for DC_30A_n66A is shown as below.

Table 6.110.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 |  |  | 8.3 |  |  |  |  |  |

Table 6.110.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 30 | n66 | 15 | 25 | 25 | 25 | 25 |  |  | 25 |  |  |  |  |  |

## 6.111	DC_30A_n260A

### 6.111.1	Operating bands for DC

Table 6.111.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_30A_n260A | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.111.2	Channel bandwidths per operating band for DC

Table 6.111.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_30A_n260A | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 (210 for Band n260 with 60 kHz SCS) |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_30A_n260G | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260H | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260I | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260J | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260K | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260L | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260M | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260(A-I) | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |
|  | n260 | 60 | See CA_n260(A-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A_n260(G-I) | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |
|  | n260 | 60 | See CA_n260(G-I) Bandwidth Combination in Table 5.5A.2-2 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.111.3	Co-existence studies

Table 6.111.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 30+Band n260 2UL DC.

Table 6.111.3-1: Band 30 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 |  | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4610 |  | 4630 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6915 |  | 6945 | 111000 | 120000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9220 | 9260 |  | 148000 | 160000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11525 |  | 11575 | 185000 | 200000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 13830 |  | 13890 | 222000 | 240000 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 16135 |  | 16205 | 259000 | 280000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 37695 |  | 34685 | 39305 | 42315 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 35390 |  | 32370 | 71685 | 77695 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 41610 |  | 44630 | 76305 | 82315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 75390 |  | 69370 | 78610 | 84630 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 33085 |  | 30055 | 108685 | 117695 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 43915 |  | 46945 | 113305 | 122315 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 157695 |  | 145685 | 27740 | 30780 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 150305 |  | 162315 | 46220 | 49260 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 115390 |  | 106370 | 67055 | 73085 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 115610 |  | 124630 | 80915 | 86945 |

Based on Table 6.111.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band n79.

-	2nd order IMD may fall into own Rx frequencies of band n260 (which can be disregarded, since no self-interference in TDD).

-	4th order IMD may fall into band n259.

-	5th order IMD may fall into band n257.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.111.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.111.3-2: Band 30 and Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | Yes | China | 3rd IMD |
|  | 47200 | - | 48400 | Yes | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.111.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.111.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_30A_n260A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |  |

### 6.111.4	∆TIB and ∆RIB values

For DC_30A_n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.111.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30A_n260A | 30 | 0 |
|  | n260 | 0 |

Table 6.111.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_30A_n260A | 30 | 0 |
|  | n260 | 0 |

### 6.111.5	REFSENS requirements

No IMD issues are expected for DC configuration between the LTE bands below 6 GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency in either the LTE band or the NR band.

## 6.112	DC_40A-n77A

### 6.112.1	Operating bands for DC

Table 6.112.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40A-n77A | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.112.2	Channel bandwidths per operating band for DC

Table 6.112.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_40A-n77A | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.112.3	Co-existence studies

Since simultaneous Rx/Tx transmission are not supported, at least in Rel-15, co-existence studies are not needed.

### 6.112.4	∆TIB and ∆RIB values

For DC_40A-n77A, the TIB,c and RIB values for LTE CA_40A-42A are reused.

Table 6.112.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40A-n77A | 40 | 0 |
|  | n77 | 0.5 |

Table 6.112.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_40A-n77A | 40 | 0.4 |
|  | n77 | 0.5 |

### 6.112.5	MSD

There is no MSD issue for this DC configuration based on that simultaneous Rx/Tx transmission are not supported at least in Rel-15.

## 6.113	DC_66A_n5A

### 6.113.1	Operating bands for DC

Table 6.113.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n5A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

### 6.113.2	Channel bandwidths per operating band for DC

Table 6.113.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n5A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 40 |
|  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.113.3	Co-existence studies

Table 6.113.3-1 lists up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 66+Band n5 2UL DC.

Table 6.113.3-1: Band 66 and Band n5 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 5130 | 5340 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 |  | 6840 | 7120 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 8550 | 8900 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4944 |  | 5094 | 10260 | 10680 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5768 |  | 5943 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1491 |  | 1456 | 3129 | 3164 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 667 |  | 607 | 3761 | 3806 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3953 |  | 4013 | 5434 | 5479 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2982 |  | 2912 | 6258 | 6328 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 157 |  | 242 | 6066 | 6121 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4777 |  | 4862 | 7739 | 7794 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8436 |  | 8371 | 1091 | 981 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10044 |  | 10109 | 5601 | 5711 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5297 |  | 5217 | 2063 | 2158 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8563 |  | 8643 | 7082 | 7177 |

Based on Table 6.113.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77 and 78.

-	33d order harmonic may fall into Rx frequencies of bands 41 and 46.

-	2nd order IMD products may fall into Rx frequencies of bands 6, 7, 8, 18, 19, 26, 27, 38, 41 and 69, or its own RX frequency in band 5.

-	33d order IMD products may fall into Rx frequencies of bands 7, 38, 41, 42, 69, 77, 78 and 79.

-	4th order IMD products may fall into Rx frequencies of bands 3, 9, 12, 13, 17, 20, 28, 29, 33, 35, 37, 39, 44, 46, 67, 68, 77 and 79.

-	5th order IMD products may fall into Rx frequencies of bands 6, 8, 18, 19, 22, 24, 26, 42, 43, 46, 47, 48, 49, 50, 74, 75, 77 and 78, or its own RX frequency in band 5.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.113.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.113.3-2: Band 66 and Band n5 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | Yes | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th, 5th IMD 3rd harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 4th, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.113.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.113.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_66A_n5A | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |  |
|  | Bands 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 5 |  |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1of TS 36.101 [2]. NOTE 2: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 of TS 38.101-1 [3] and 38.101-2 [4]. NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2 |  |  |  |  |  |  |  |  |

### 6.113.4	∆TIB and ∆RIB values

For DC_66A_n5A, the TIB,c and RIB,c values are derived from LTE combination CA_66A-5A and are given in the tables below.

Table 6.113.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A_n5A | 66 | 0.3 |
|  | n5 | 0.3 |

Table 6.113.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66A_n5A | 66 | 0 |
|  | n5 | 0 |

### 6.113.5	REFSENS requirements

Due to 2nd order IMD MSD are needed and are defined in table 6.113.5-1 Values are reused from existing LTE combination CA_5A-66A.

Table 6.113.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD21 |  |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |  |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

### 6.113.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.113.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.113.6-1.

Table 6.113.6-1: Band 66 and Band n5 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n5A | n5 | a | -1 | b | -1 | 2 | n5 | IMD |
|  | 66 | c | 1 | d | 0 |  |  |  |
| DC_66A_n5A | n5 | a | -3 | b | -1 | 5 | n5 | IMD |
|  | 66 | c | 2 | d | 0 |  |  |  |

Equations (6.113.6-1) and (6.113.6-2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.113.6-1 and CBW stands for channel bandwidth.

(6.113.6-1)

(6.113.6-2)

Formula (6.113.6-3-1) and (6.113.6-3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.113.6-3-1)

(6.113.6-3-2)

## 6.114	DC_66A_n78A

### 6.114.1	Operating bands for DC

Table 6.114.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2220 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.114.2	Channel bandwidths per operating band for DC

Table 6.114.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A-n78A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.114.3	Co-existence studies

Table 6.114.3-1 provides the self-interference study for DC_66A_n78A.

Table 6.114.3-1: Harmonics and IMD products

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | DL frequency range (MHz) | Rx Impact | Harmonic/IMD orders | Notes |
| --- | --- | --- | --- | --- | --- |
| DC_66A-n78A | 66 | 2110 MHz -2220 MHz | Yes | IMD5 |  |
|  | n78 | 3300 MHz -3800 MHz | Yes | 2th Harmonic |  |

Table 6.114.3-2 lists the protected bands required for the dual connectivity configuration.

Table 6.114.3-2: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.114.4	∆TIB and ∆RIB values

For DC_66A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.114.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A-n78A | 66 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 66. |  |  |

Table 6.114.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66A-n78A | 66 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 66. |  |  |

### 6.114.5	MSD

MSD values for Band n78 due to 2nd harmonic of Band 66 for DC_66A-n78A are captured in Table 6.114.5-1 with the uplink configuration in Table 6.114.5-2.

Table 6.114.5-1: MSD due to harmonic issue for DC_66A-n78A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 66 | n781,2 |  | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.114.5-2 Uplink configuration for the low band (exceptions)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | Duplex mode |
| DC_66-n78 | 66 |  |  |  | 26 | 39 | 53 | 100 | FDD |

MSD for IMD5 is less than 2dB based on the analysis, so the MSD requirement is not needed to be defined for these band combination.

Table 6.114.5-3: Void

### 6.114.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.114.3 co-existence studies except for harmonic mixing which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.114.6-1.

Table 6.114.6-1: Band 66 and Band n78 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n78A | 66 | a | -3 | b | -1 | 5 | 66 | IMD |
|  | n78 | c | 2 | d | 0 |  |  |  |
| DC_66A_n78A | 66 | a | 2 | b | 0 | 2 | n78 | Harmonic |
|  | n78 | c | 0 | d | -1 |  |  |  |

Equations (6.114.6-1) and (6.114.6-2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.114.6-1 and CBW stands for channel bandwidth.

(6.114.6-1)

(6.114.6-2)

Formula (6.114.6-3-1) and (6.114.6-3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(6.114.6-3-1)

(6.114.6-3-2)

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN4#84 | R4-1707513 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2017-09 | RAN4 NR#3 | R4-1709388 |  |  |  | TR 37.863-01-01_V0.1.0_Rel15_DC band combinations of LTE 1DL1UL + one NR band | 0.1.0 |
|  |  | R4-1707189 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707190 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707209 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707210 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707211 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707213 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707234 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707252 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707278 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1707279 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1708066 |  |  |  | TP for TR 37.863-01-01 operating bands, channel bandwidths for 2DL2UL DC_28A-xA_BCS0 |  |
|  |  | R4-1708068 |  |  |  | TP for TR 37.863-01-01 operating bands, channel bandwidths for 2DL2UL DC_28A-yA_BCS0 |  |
|  |  | R4-1708416 |  |  |  | TP for TR 37.863-01-01: operating bands and channel bandwidths for DC_1A-n78A_BCS0 |  |
|  |  | R4-1708418 |  |  |  | TP for TR 37.863-01-01: operating bands and channel bandwidths for DC_3A-n78A_BCS0 |  |
|  |  | R4-1708840 |  |  |  | TP for TR 37.863-01-01: DC band combination of LTE Band 20 + NR Band 28 |  |
|  |  | R4-1708894 |  |  |  | TP for TR 37.863-01-01: 2DL/2UL DC_1A-n79A_BCS0 |  |
|  |  | R4-1708895 |  |  |  | TP for TR 37.863-01-01: 2DL/2UL DC_3A-n79A_BCS0 |  |
|  |  | R4-1708896 |  |  |  | TP for TR 37.863-01-01: 2DL/2UL DC_19A-n79A_BCS0 |  |
|  |  | R4-1708897 |  |  |  | TP for TR 37.863-01-01: 2DL/2UL DC_21A-n79A_BCS0 |  |
|  |  | R4-1708898 |  |  |  | TP for TR 37.863-01-01: 2DL/2UL DC_28A-n79A_BCS0 |  |
|  |  | R4-1708899 |  |  |  | TP for TR 37.863-01-01: 2DL/2UL DC_42A-n79A_BCS0 |  |
|  |  | R4-1708967 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1708968 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1708969 |  |  |  | TP for TR 37.863-01-01: Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band |  |
|  |  | R4-1708974 |  |  |  | TP for TR 37.863-01-01 DC_1A-n77A |  |
|  |  | R4-1708975 |  |  |  | TP for TR 37.863-01-01 DC_3A-n77A |  |
|  |  | R4-1708976 |  |  |  | TP for TR 37.863-01-01 DC_19A-n77A |  |
|  |  | R4-1708977 |  |  |  | TP for TR 37.863-01-01 DC_19A-n78A |  |
|  |  | R4-1708978 |  |  |  | TP for TR 37.863-01-01: co-existence studies, UE requirements relaxation for 2DL/2UL DC_28A-xA_BCS0 |  |
|  |  | R4-1708979 |  |  |  | TP for TR 37.863-01-01: co-existence studies, UE requirements relaxation for 2DL/2UL DC_28A-yA_BCS0 |  |
|  |  | R4-1708980 |  |  |  | TP for TR 37.863-01-01: co-existence studies and MDS for DC_3A-n78A_BCS0 |  |
|  |  | R4-1708981 |  |  |  | TP for TR 37.863-01-01 co-existence study for DC_39A-n78A_BCS0 |  |
|  |  | R4-1708982 |  |  |  | TP for TR 37.863-01-01 co-existence study for DC_39A-n258A_BCS0 |  |
|  |  | R4-1708983 |  |  |  | TP for TR 37.863-01-01: co-existence studies and MSD for DC_1A-n78A_BCS0 |  |
|  |  | R4-1708987 |  |  |  | TP for TR 37.863-01-01 DC_21A-n77A |  |
|  |  | R4-1708988 |  |  |  | TP for TR 37.863-01-01 DC_21A-n78A |  |
|  |  | R4-1708989 |  |  |  | TP for TR 37.863-01-01 DC_42A-n77A |  |
|  |  | R4-1708990 |  |  |  | TP for TR 37.863-01-01 DC_42A-n78A |  |
| 2017-10 | 3GPP RAN4#84bis | R4-1711073 |  |  |  | TR 37.863-01-01_V0.2.0_Rel15_DC band combinations of LTE 1DL1UL + one NR band | 0.2.0 |
|  |  | R4-1709390 |  |  |  | Feasibility of DC_42A-n77A and 42A-n78A |  |
|  |  | R4-1709434 |  |  |  | TP for TR 37.863-01-01 co-existence study for DC_39A-n79A_BCS0 |  |
|  |  | R4-1709435 |  |  |  | TP for TR 37.863-01-01 co-existence study for DC_41A-n78A_BCS0 |  |
|  |  | R4-1709436 |  |  |  | TP for TR 37.863-01-01 co-existence study for DC_41A-n79A_BCS0 |  |
|  |  | R4-1709437 |  |  |  | TP for TR 37.863-01-01 co-existence study for DC_41A-n258A_BCS0 |  |
|  |  | R4-1709438 |  |  |  | TP for TR 37.863-01-01 co-existence study for DC_41A-n77A_BCS0 |  |
|  |  | R4-1709490 |  |  |  | TP for TR 37.863-01-01 for DC_3A-n258A |  |
|  |  | R4-1709491 |  |  |  | TP for TR 37.863-01-01 for DC_7A-n258A |  |
|  |  | R4-1709492 |  |  |  | TP for TR 37.863-01-01 for DC_28A-n258A |  |
|  |  | R4-1709507 |  |  |  | TP for TR 37.863-01-01: Operating bands, Channel bandwidths for DC_8A-n79A |  |
|  |  | R4-1709510 |  |  |  | TP for TR 37.863-01-01: Co-existence studies and MSD for DC_8A-n79A |  |
|  |  | R4-1709511 |  |  |  | TP for TR 37.863-01-01: Operating bands,Channel bandwidths for DC_8A-n78A |  |
|  |  | R4-1709512 |  |  |  | TP for TR 37.863-01-01: Operating bands, Channel bandwidths for DC_8A-n258A |  |
|  |  | R4-1709523 |  |  |  | TP for TR 37.863-01-01:Co-existence studies and MSD for DC_8A-n78A |  |
|  |  | R4-1709524 |  |  |  | TP for TR 37.863-01-01: Co-existence studies and MSD for DC_8A-n258A |  |
|  |  | R4-1709577 |  |  |  | TP on TR37.863-01-01 for DC_7A-n78A |  |
|  |  | R4-1709593 |  |  |  | TP for TR 37.863-01-01: DC band combination of LTE Band 38 and NR Band n78 |  |
|  |  | R4-1709660 |  |  |  | TP for TR 37.863-01-01 operating bands, channel bandwidths for 2DL2UL DC_20A-n78A_BCS0 |  |
|  |  | R4-1709661 |  |  |  | TP for TR 37.863-01-01 co-existence studies, UE requirements relaxation for 2DL2UL DC_20A-n78A_BCS0 |  |
|  |  | R4-1709680 |  |  |  | TP for TR 37.863-01-01 DC_5A-n257A |  |
|  |  | R4-1709759 |  |  |  | TP for TR 37.863-01-01 DC_1A_n257 |  |
|  |  | R4-1709760 |  |  |  | TP for TR 37.863-01-01 DC_3A_n257 |  |
|  |  | R4-1709761 |  |  |  | TP for TR 37.863-01-01 DC_19A_n257 |  |
|  |  | R4-1709762 |  |  |  | TP for TR 37.863-01-01 DC_21A_n257 |  |
|  |  | R4-1709763 |  |  |  | TP for TR 37.863-01-01 DC_28A_n257 |  |
|  |  | R4-1709764 |  |  |  | TP for TR 37.863-01-01 DC_42A_n257 |  |
|  |  | R4-1710032 |  |  |  | TP for TR 37.863-01-01: DC band combination of LTE Band 20 and NR Band n28 |  |
| 2017-10 | RAN4#85 | R4-1712817 |  |  |  | TR 37.863-01-01_V0.3.0_Rel15_DC band combinations of LTE 1DL1UL + one NR band | 0.3.0 |
|  |  | R4-1710574 |  |  |  | TP for TR 37.863-01-01 for DC_5A-n78A |  |
|  |  | R4-1710694 |  |  |  | TP for TR 37.863-01-01 DC_5A-n78A |  |
|  |  | R4-1710695 |  |  |  | TP for TR 37.863-01-01 DC_7A-n257A |  |
|  |  | R4-1710710 |  |  |  | TP on UE RF architectures for NSA DC UE |  |
|  |  | R4-1710773 |  |  |  | TP for TR 37.863-01-01 DC_3A_n7 |  |
|  |  | R4-1710850 |  |  |  | TP for TR 37.863-01-01 DC_66A-n257A |  |
|  |  | R4-1710893 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_18A_n77 |  |
|  |  | R4-1710894 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_19A_n77 |  |
|  |  | R4-1710895 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_26A_n77 |  |
|  |  | R4-1710948 |  |  |  | TP for TR 37.863-01-01: MSD for DC_18A-n77A |  |
|  |  | R4-1710949 |  |  |  | TP for TR 37.863-01-01: MSD for DC_18A-n78A |  |
|  |  | R4-1711072 |  |  |  | TP for TR 37.863-01-01 Delta values for DC combinations including Band n77, n78, n79 and n257 |  |
|  |  | R4-1711091 |  |  |  | TP for TR 37.863-01-01: DC band combination of LTE Band 20 and NR Band n28 |  |
|  |  | R4-1711361 |  |  |  | TP for TR 37.863-01-01: DC_1A_n28 |  |
|  |  | R4-1711362 |  |  |  | TP for TR 37.863-01-01: DC_3A_n28 |  |
|  |  | R4-1711364 |  |  |  | TP for TR 37.863-01-01: DC_7A_n28 |  |
|  |  | R4-1711837 |  |  |  | TP for TR 37.863-01-01 MSD for 2DL/2UL DC combinations including Band n77, n78 and n79 |  |
| 2018-01 | RAN4AH-1801 | R4-1801110 |  |  |  | TR 37.863-01-01_V0.4.0_Rel15_DC band combo of LTE 1DL1UL + one NR band | 0.4.0 |
|  |  | R4-1712160 |  |  |  | TP on TR 37.863-01-01 for DC_8A-n77A |  |
|  |  | R4-1712162 |  |  |  | TP on TR 37.863-01-01 for DC_8A-n257A |  |
|  |  | R4-1712214 |  |  |  | TP for TR 37.863-01-01 DC_7A-n257A |  |
|  |  | R4-1712285 |  |  |  | TP for TR 37.863-01-01 DC_41A-n41A |  |
|  |  | R4-1712286 |  |  |  | TP for TR 37.863-01-01 DC_25A-n41A |  |
|  |  | R4-1712287 |  |  |  | TP for TR 37.863-01-01 DC_26A-n41A |  |
|  |  | R4-1712422 |  |  |  | DC_66A_n257A |  |
|  |  | R4-1712461 |  |  |  | TP to TR 37.863-01-01:DC_71b_n71b |  |
|  |  | R4-1712512 |  |  |  | TP for TR 37.863-01-01 UE requirement for DC_3A_n7A |  |
|  |  | R4-1712606 |  |  |  | TP on TR 37.863-01-01 for DC_39A-n78A_BCS0 |  |
|  |  | R4-1712607 |  |  |  | TP on TR 37.863-01-01 for DC_41A-n78A_BCS0 |  |
|  |  | R4-1712608 |  |  |  | TP on TR 37.863-01-01 for DC_39A-n79A_BCS0 |  |
|  |  | R4-1712609 |  |  |  | TP on TR 37.863-01-01 for DC_41A-n79A_BCS0 |  |
|  |  | R4-1712610 |  |  |  | TP on TR 37.863-01-01 for DC_39A-n258A_BCS0 |  |
|  |  | R4-1712611 |  |  |  | TP on TR 37.863-01-01 for DC_41A-n258A_BCS0 |  |
|  |  | R4-1712741 |  |  |  | DC_3A_n258A |  |
|  |  | R4-1712742 |  |  |  | DC_7A_n258A |  |
|  |  | R4-1712743 |  |  |  | DC_28A_n258A |  |
|  |  | R4-1712761 |  |  |  | TP for TR 37.863-01-01: delt Tib and Rib, MSD for DC_8A-n79A |  |
|  |  | R4-1712764 |  |  |  | TP for TR 37.863-01-01: ?TIB and ?RIB, MSD for DC_8A-n258A |  |
|  |  | R4-1712821 |  |  |  | TP for TR 37.863-01-01 No MSD for DC including Band n257 |  |
|  |  | R4-1712894 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_1A_n77A |  |
|  |  | R4-1712895 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_1A_n78A |  |
|  |  | R4-1712896 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_3A_n77A |  |
|  |  | R4-1712897 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_3A_n78A |  |
|  |  | R4-1712898 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_11A_n79A |  |
|  |  | R4-1712899 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_18A_n78A |  |
|  |  | R4-1712900 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_18A_n79A |  |
|  |  | R4-1712901 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_19A_n78A |  |
|  |  | R4-1712902 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_19A_n79A |  |
|  |  | R4-1712903 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_21A_n79A |  |
|  |  | R4-1712904 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_26A_n78A |  |
|  |  | R4-1712905 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_26A_n79A |  |
|  |  | R4-1712906 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_28A_n77A |  |
|  |  | R4-1712907 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_28A_n78A |  |
|  |  | R4-1712908 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_28A_n79A |  |
|  |  | R4-1712909 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_39A_n78A |  |
|  |  | R4-1712910 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_8A_n78A |  |
|  |  | R4-1712918 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_8A_n79A |  |
|  |  | R4-1712920 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_20A_n78A |  |
|  |  | R4-1712921 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_41A_n79A |  |
|  |  | R4-1712926 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_1A_n28A |  |
|  |  | R4-1712927 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_3A_n7A |  |
|  |  | R4-1712928 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_5A_n78A |  |
|  |  | R4-1712929 |  |  |  | TP for TR 37.863-01-01 corrections on interference mixing coefficients |  |
|  |  | R4-1713259 |  |  |  | TP on protection bands, MSD level and additional ILs for DC_7A-n78A |  |
|  |  | R4-1713546 |  |  |  | TP for TR 37.863-01-01: DC_1A_n28 |  |
|  |  | R4-1713548 |  |  |  | TP for TR 37.863-01-01: DC_3A_n28 |  |
|  |  | R4-1713550 |  |  |  | TP for TR 37.863-01-01: DC_7A_n28 |  |
|  |  | R4-1713972 |  |  |  | TP for TR 37.863-01-01 for DC combinations of LTE bands 11, 18, 26, 41 and NR bands n77, n78, n79, n257 |  |
|  |  | R4-1714006 |  |  |  | TP for TR 37.863-01-01 DC including LTE Band 42 |  |
|  |  | R4-1714007 |  |  |  | TP for TR 37.863-01-01 MSD for DC including Band n77, n78 and n79 |  |
|  |  | R4-1714008 |  |  |  | TP for TR 37.863-01-01 UE-to-UE co-existence for DC including Band n77, n78, n79 and n257 |  |
|  |  | R4-1714025 |  |  |  | TP for TR37.863-01-01:MSD requirements for DC_8A-n78A |  |
|  |  | R4-1714045 |  |  |  | TP on protected bands, additional ILs and MSD level for TR 37.863-01-01 DC_5A_n78A |  |
|  |  | R4-1714046 |  |  |  | TP for TR 37863-01-01 UE requirements study for DC_20A-n78A |  |
|  |  | R4-1714086 |  |  |  | TP on protected bands, additional ILs and MSD level for TR 37.863-01-01 DC_5A_n257A |  |
|  |  | R4-1714164 |  |  |  | TP for TR 37.863-01-01: DC band combination of LTE Band 20 and NR Band n28 |  |
|  |  | R4-1714173 |  |  |  | TP for TR 37.863-01-01 Harmonic mixing MSD for DC_28-n77 and DC_19-n79 |  |
|  |  | R4-1714339 |  |  |  | TP for TR 37.863-01-01: Requirements on B3_n78, B3_n77 and B1_n77 |  |
|  |  | R4-1714349 |  |  |  | DC_66A_n71A |  |
|  |  | R4-1714333 |  |  |  | TP on TR 37.863-01-01 for Single Tx Switched UL |  |
| 2018-02 | RAN4#86 | R4-1803462 |  |  |  | Updated TR 37.863-01-01 V0.5.0 Rel-15 DC band combination of LTE 1DL/1UL + one NR band | 0.5.0 |
|  |  | R4-1800061 |  |  |  | TP for TR 37.863-01-01: DC_(n)71B |  |
|  |  | R4-1800294 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_26A_n41A |  |
|  |  | R4-1800295 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_66A_n71A |  |
|  |  | R4-1800408 |  |  |  | TP for TR 37.863-01-01: DC_20A-n8A |  |
|  |  | R4-1800428 |  |  |  | TP on TR 37.863-01-01 for DC_8A-n77A: MSD and missing parts |  |
|  |  | R4-1800430 |  |  |  | TP on TR 37.863-01-01 for DC_8A-n257A: addition of band protection requirements |  |
|  |  | R4-1800455 |  |  |  | TP for TR 37.863-01-01: Harmonic mixing MSD for DC_26A-n79A |  |
|  |  | R4-1800456 |  |  |  | TP for 37.863-01-01: Harmonic MSD for DC_18A-n77A, DC_26A-n77A and DC_26A-n78A |  |
|  |  | R4-1800457 |  |  |  | TP for 37.863-01-01: IMD MSD for DC_26A-n77A and DC_26A-n78A |  |
|  |  | R4-1800501 |  |  |  | TP for TR 37.863-01-01 DC_46A_n78A |  |
|  |  | R4-1800517 |  |  |  | TP for TR 37.863-01-01 for DC combinations of LTE bands 18, 26, 41 and NR bands n77, n78, n79, n257 |  |
|  |  | R4-1800818 |  |  |  | TP for TR 37.863-01-01 for DC_1A-n50A |  |
|  |  | R4-1800819 |  |  |  | TP for TR 37.863-01-01 for DC_1A-n51A |  |
|  |  | R4-1800820 |  |  |  | TP for TR 37.863-01-01 for DC_3A-n50A |  |
|  |  | R4-1800821 |  |  |  | TP for TR 37.863-01-01 for DC_3A-n51A |  |
|  |  | R4-1800822 |  |  |  | TP for TR 37.863-01-01 for DC_7A-n50A |  |
|  |  | R4-1800823 |  |  |  | TP for TR 37.863-01-01 for DC_7A-n51A |  |
|  |  | R4-1800824 |  |  |  | TP for TR 37.863-01-01 for DC_20A-n50A |  |
|  |  | R4-1800825 |  |  |  | TP for TR 37.863-01-01 for DC_20A-n51A |  |
|  |  | R4-1800828 |  |  |  | TP for TR 37.863-01-01 for DC_42A-n50A |  |
|  |  | R4-1800829 |  |  |  | TP for TR 37.863-01-01 for DC_42A-n51A |  |
|  |  | R4-1800925 |  |  |  | TP for TR 37.863-01-01 DC_25A_n41A |  |
|  |  | R4-1800926 |  |  |  | TP for TR 37.863-01-01 DC_26A_n41A |  |
|  |  | R4-1800927 |  |  |  | TP for TR 37.863-01-01 DC_41A_n41A |  |
|  |  | R4-1800972 |  |  |  | TP for TR 37.863-01-01 DC_25A_n41A delta Rib, Tib |  |
|  |  | R4-1800973 |  |  |  | TP for TR 37.863-01-01 DC_26A_n41A delta Rib, Tib |  |
|  |  | R4-1801093 |  |  |  | TP for TR37.863-01-01:UE to UE coexistence for the combinations of band 8 and n78,n79 |  |
|  |  | R4-1801130 |  |  |  | TP for TR 37.863-01-01: DC_2A-n71A |  |
|  |  | R4-1801185 |  |  |  | TP for TR 37.863-01-01 for DC_28A-n50A |  |
|  |  | R4-1801186 |  |  |  | TP for TR 37.863-01-01 for DC_28A-n51A |  |
|  |  | R4-1801188 |  |  |  | TP on TR 37.863-01-01 for Single Tx Switched UL |  |
|  |  | R4-1801127 |  |  |  | DC_66A_n257A |  |
|  |  | R4-1801128 |  |  |  | DC_66A_n71A |  |
|  |  | R4-1800722 |  |  |  | DC_3A_n258A |  |
|  |  | R4-1800723 |  |  |  | DC_7A_n258A |  |
|  |  | R4-1800724 |  |  |  | DC_28A_n258A |  |
|  |  | R4-1801126 |  |  |  | DC_66A-n260A TP for TR 37 863-01-01 |  |
|  |  | R4-1801151 |  |  |  | TP for TR 37.863 DC_2A-n257A |  |
|  |  | R4-1801155 |  |  |  | TP for TR 37.863 DC_13A-n257A |  |
|  |  | R4-1801157 |  |  |  | TP for TR 37.863 DC_48A-n257A |  |
|  |  | R4-1801170 |  |  |  | TP for TR 37.863 DC_5A-n260A |  |
|  |  | R4-1801171 |  |  |  | TP for TR 37.863 DC_13A-n260A |  |
|  |  | R4-1801174 |  |  |  | TP for TR 37.863 DC_48A-n260A |  |
|  |  | R4-1801481 |  |  |  | TP for TR37.863-01-01:Protected bands for the combinations of band 8 and n258 |  |
|  |  | R4-1801876 |  |  |  | TP to 37.863-01-01 to correct channel BW for DC_66A_n257A |  |
|  |  | R4-1801993 |  |  |  | TP on TR 37.863-01-01 for DC_41A-n77A: MSD and protection requirement |  |
|  |  | R4-1802028 |  |  |  | TP for TR 37.863-01-01 for DC combinations of LTE bands 11, 18, 26, 41 and NR bands n77, n78, n79, n257 |  |
|  |  | R4-1802170 |  |  |  | TP for TR 37.863-01-01: DC_3A_n28 |  |
|  |  | R4-1802171 |  |  |  | TP for TR 37.863-01-01: DC_7A_n28 |  |
|  |  | R4-1802199 |  |  |  | TP for TR 37.863-01-01: DC_25A_n41 MSD |  |
|  |  | R4-1801644 |  |  |  | TP for 37.863-01-01: general co-existence study |  |
|  |  | R4-1802227 |  |  |  | TP to TR 37.836-01-01 MSD DC Combination B41+n79 |  |
|  |  | R4-1802264 |  |  |  | TP for TR 37.863-01-01: Introduction of DC_(n)41C |  |
|  |  | R4-1803185 |  |  |  | TP to TR 37.863-01-01: MSD for DC_66A_n71A |  |
|  |  | R4-1803268 |  |  |  | TP for TR 37.863-01-01: DC_1A_n28 |  |
|  |  | R4-1803269 |  |  |  | TP for TR 37.863-01-01: DC_41A_n41 SEM |  |
| 2018-03 | RAN#79 | RP-180150 |  |  |  | Presented to RAN plenary for information | 1.0.0 |
| 2018-04 | RAN4#86bis | R4-1804371 |  |  |  | Updated TR 37.863-01-01 V1.1.0 Rel-15 DC band combination of LTE 1DL/1UL + one NR band | 1.1.0 |
|  |  | R4-1803696 |  |  |  | TP for 37.863-01-01: DC_39A-n79A |  |
|  |  | R4-1803697 |  |  |  | TP for 37.863-01-01: DC_39A-n258A |  |
|  |  | R4-1803700 |  |  |  | TP for 37.863-01-01: DC_41A-n258A |  |
|  |  | R4-1803745 |  |  |  | TP on TR 37.863-01-01 for DC_8A-n77A: UL configuration for MSD for H4 |  |
|  |  | R4-1803755 |  |  |  | TP for TR37.863-01-01 UL configuration for DC combinations Band 8 and n78,n79 |  |
|  |  | R4-1804324 |  |  |  | TP for TR 37.863-01-01 on MSD for DC_26A_n41A |  |
|  |  | R4-1804940 |  |  |  | TP to TR 37.863-01-01: DC_66_n257 (CA_n257 Fallback group 3 BCS0) |  |
|  |  | R4-1805127 |  |  |  | TP for TR 37.863-01-01: DC_38A_n78A |  |
|  |  | R4-1805222 |  |  |  | TP for TR 37.863-01-01 for DC_7A-n50A |  |
|  |  | R4-1805223 |  |  |  | TP for TR 37.863-01-01 for DC_7A-n51A |  |
|  |  | R4-1805224 |  |  |  | TP for TR 37.863-01-01 for DC_20A-n50A |  |
|  |  | R4-1805225 |  |  |  | TP for TR 37.863-01-01 for DC_20A-n51A |  |
|  |  | R4-1805450 |  |  |  | TP for TR 37.863-01-01 correction on DC_66A_n71A |  |
|  |  | R4-1805451 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_20A_n8A |  |
|  |  | R4-1805453 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_2A_n71A |  |
|  |  | R4-1805454 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_28A_n50A |  |
|  |  | R4-1805456 |  |  |  | TP for TR 37.863-01-01 self-interference analysis for DC_28A_n51A |  |
|  |  | R4-1805630 |  |  |  | TP for 37.863-01-01: DC_39A-n78A |  |
|  |  | R4-1805631 |  |  |  | TP for 37.863-01-01: DC_41A-n78A |  |
|  |  | R4-1805632 |  |  |  | TP for 37.863-01-01: DC_41A-n79A |  |
|  |  | R4-1805633 |  |  |  | TP for TR37.863-01-01 for EN-DC CA_2A-n71A MSD and uplink configuration |  |
|  |  | R4-1805687 |  |  |  | TP for TR 37.863-01-01 for DC_1A-n50A |  |
|  |  | R4-1805688 |  |  |  | TP for TR 37.863-01-01 for DC_1A-n51A |  |
|  |  | R4-1805689 |  |  |  | TP for TR 37.863-01-01 for DC_3A-n50A |  |
|  |  | R4-1805690 |  |  |  | TP for TR 37.863-01-01 for DC_3A-n51A |  |
|  |  | R4-1805691 |  |  |  | TP for TR 37.863-01-01 for DC_28A-n50A |  |
|  |  | R4-1805692 |  |  |  | TP for TR 37.863-01-01 for DC_28A-n51A |  |
|  |  | R4-1805693 |  |  |  | TP for TR 37.863-01-01 for DC_42A-n50A |  |
|  |  | R4-1805694 |  |  |  | TP for TR 37.863-01-01 for DC_42A-n51A |  |
|  |  | R4-1805732 |  |  |  | TP for TR 37.863-01-01 DC_(n)41C DC_41A_n41A SEM |  |
|  |  | R4-1805762 |  |  |  | TP for TR 37.863-01-01 DC_5A-n260B, DC_5A-n260C, DC_5A-n260D, DC_5A-n260E, DC_5A-n260F, DC_5A-n260G, DC_5A-n260H, DC_5A-n260I, DC_5A-n260J, DC_5A-n260K, DC_5A-n260L, DC_5A-n260M, DC_5A-n260O, DC_5A-n260P DC_5A-n260Q |  |
|  |  | R4-1805763 |  |  |  | TP for TR 37.863-02-01 DC_5A-n261B, DC_5A-n261C, DC_5A-n261D, DC_5A-n261E, DC_5A-n261F, DC_5A-n261G, DC_5A-n261H, DC_5A-n261I, DC_5A-n261J, DC_5A-n261K, DC_5A-n261L, DC_5A-n261M, DC_5A-n261O, DC_5A-n261P, DC_5A-n261Q |  |
|  |  | R4-1805764 |  |  |  | TP for TR 37.863-01-01 DC_66A-n260B, DC_66A-n260C, DC_66A-n260D, DC_66A-n260E, DC_66A-n260F, DC_66A-n260G, DC_66A-n260H, DC_66A-n260I, DC_66A-n260J, DC_66A-n260K, DC_66A-n260L, DC_66A-n260M, DC_66A-n260O, DC_66A-n260P DC_66A-n260Q |  |
|  |  | R4-1805765 |  |  |  | TP for TR 37.863-02-01 DC_66A-n261B, DC_66A-n261C, DC_66A-n261D, DC_66A-n261E, DC_66A-n261F, DC_66A-n261G, DC_66A-n261H, DC_66A-n261I, DC_66A-n261J, DC_66A-n261K, DC_66A-n261L, DC_66A-n261M, DC_66A-n261O, DC_66A-n261P, DC_66A-n261Q |  |
| 2018-05 | RAN4#87 | R4-1806517 |  |  |  | Updated TR 37.863-01-01 V1.2.0 Rel-15 DC band combination of LTE 1DL/1UL + one NR band | 1.2.0 |
|  |  | R4-1806214 |  |  |  | TP for TR 37.863-01-01 Introduction of DC_2A_n257(2A) |  |
|  |  | R4-1806215 |  |  |  | TP for TR 37.863-01-01 Introduction of DC_66A_n257(2A) |  |
|  |  | R4-1806253 |  |  |  | TP to TR 37.863-01-01: Removal of square brackets around the 40 MHz channel bandwidth option, for the following EN-DC configurations: DC_1A_n78A, DC_3A_n78A, DC_7A_n78A and DC_20A_n78A |  |
|  |  | R4-1806512 |  |  |  | TP for TR 37.863-01-01 to add n77C, n78C, n79C and n257DEF for EN-DC including B1, 3, 19, 21, 28 and 42 |  |
|  |  | R4-1806576 |  |  |  | TP to TR 37.863-01-01: Introduction of new NR band n7 channel bandwidth options, for DC_3A_n7A |  |
|  |  | R4-1806619 |  |  |  | TP for TR 37.863-01-01 Inter-band DC_5A-n260 Carrier Aggregation |  |
|  |  | R4-1806622 |  |  |  | TP for TR 37.863-01-01 Inter-band DC_66A-n260 Carrier Aggregation |  |
|  |  | R4-1806623 |  |  |  | TP for TR 37.863-01-01 Inter-band DC 5A-n261 Carrier Aggregation |  |
|  |  | R4-1806629 |  |  |  | TP for TR 37.863-01-01 Inter-band DC_66A-n261 CA |  |
|  |  | R4-1806769 |  |  |  | TP on TR 37.863-01-01 for DC_41A-n77A and 41A-n78A: Corrections |  |
|  |  | R4-1806824 |  |  |  | TP for TR 37.863-01-01 MSD for DC_1A_n78A |  |
|  |  | R4-1806825 |  |  |  | TP for TR 37.863-01-01 MSD for DC_3A_n78A |  |
|  |  | R4-1806826 |  |  |  | TP for TR 37.863-01-01 Remove the duplicate sections for DC_5A_n78A in the TR |  |
|  |  | R4-1806832 |  |  |  | TP for TR 37.863-01-01: LTE band 3 plus NR band n3 EN-DC non-contiguous combination |  |
|  |  | R4-1806981 |  |  |  | TP for 37.863-01-01:DC_(n)71B |  |
|  |  | R4-1807041 |  |  |  | TP to TR 37.863-01-01 MSD for DC_26A_n41A due to IMD3 |  |
|  |  | R4-1807109 |  |  |  | TP 37.863-01-01 DC_2A_n260A, DC_2A_n260(2A) |  |
|  |  | R4-1807110 |  |  |  | TP 37.863-01-01 DC_66A_n260(2A), DC_66A_n260(3A), DC_66A_n260(4A) |  |
|  |  | R4-1807111 |  |  |  | TP 37.863-01-01 DC_12A_n260A |  |
|  |  | R4-1807112 |  |  |  | TP 37.863-01-01 DC_30A_n260A |  |
|  |  | R4-1807113 |  |  |  | TP 37.863-01-01 DC_2A_n66A |  |
|  |  | R4-1807114 |  |  |  | TP 37.863-01-01 DC_5A_n66A |  |
|  |  | R4-1807115 |  |  |  | TP 37.863-01-01 DC_12A_n66A |  |
|  |  | R4-1807116 |  |  |  | TP 37.863-01-01 DC_30A_n66A |  |
|  |  | R4-1807117 |  |  |  | TP 37.863-01-01 DC_2A_n5A |  |
|  |  | R4-1807118 |  |  |  | TP 37.863-01-01 DC_12A_n5A |  |
|  |  | R4-1807119 |  |  |  | TP 37.863-01-01 DC_30A_n5A |  |
|  |  | R4-1807120 |  |  |  | TP 37.863-01-01 DC_66A_n5A |  |
|  |  | R4-1807121 |  |  |  | TP 37.863-01-01 DC_1A_n40A |  |
|  |  | R4-1807122 |  |  |  | TP 37.863-01-01 DC_3A_n40A |  |
|  |  | R4-1807123 |  |  |  | TP 37.863-01-01 DC_5A_n40A |  |
|  |  | R4-1807124 |  |  |  | TP 37.863-01-01 DC_8A_n40A |  |
|  |  | R4-1807242 |  |  |  | TP for TR 37.863-01-01: DC_66A_n78A |  |
|  |  | R4-1807243 |  |  |  | TP for TR 37.863-01-01: DC_2A_n78A |  |
|  |  | R4-1807249 |  |  |  | TP for TR 37.863-01-01: DC_20A_n258A |  |
|  |  | R4-1807533 |  |  |  | TP for TR 37.863-01-01: DC_40A_n77A |  |
|  |  | R4-1807534 |  |  |  | TP for TR 37.863-01-01: DC_20A_n77A |  |
|  |  | R4-1807690 |  |  |  | TP for TR 37.863-01-01 for DC_1A-n50A |  |
|  |  | R4-1807691 |  |  |  | TP for TR 37.863-01-01 for DC_1A-n51A |  |
|  |  | R4-1807692 |  |  |  | TP for TR 37.863-01-01 for DC_3A-n50A |  |
|  |  | R4-1807693 |  |  |  | TP for TR 37.863-01-01 for DC_3A-n51A |  |
|  |  | R4-1807694 |  |  |  | TP for TR 37.863-01-01 for DC_7A-n50A |  |
|  |  | R4-1807695 |  |  |  | TP for TR 37.863-01-01 for DC_7A-n51A |  |
|  |  | R4-1807696 |  |  |  | TP for TR 37.863-01-01 for DC_20A-n50A |  |
|  |  | R4-1807697 |  |  |  | TP for TR 37.863-01-01 for DC_20A-n51A |  |
|  |  | R4-1807698 |  |  |  | :TP for TR 37.863-01-01 for DC_28A-n50A |  |
|  |  | R4-1807699 |  |  |  | TP for TR 37.863-01-01 for DC_28A-n51A |  |
|  |  | R4-1807700 |  |  |  | TP for TR 37.863-01-01 for DC_42A-n50A |  |
|  |  | R4-1807701 |  |  |  | TP for TR 37.863-01-01 for DC_42A-n51A |  |
|  |  | R4-1807819 |  |  |  | TP for TR 37.863-01-01 add BCS for DC_(n)41AA and DC_41A_n41A |  |
|  |  | R4-1808056 |  |  |  | TP for TR 37.863-01-01: MSD for DC_20A-n8A |  |
|  |  | R4-1806698 |  |  |  | UE-UE co-existence for DC_41A-n258A |  |
|  |  | R4-1806843 |  |  |  | UL configuration for EN-DC CA_2A-n71A with B71 Rx 3rd order harmonic mixing |  |
| 2018-06 | RAN#80 | RP-180691 |  |  |  | V2.0.0 submitted for plenary approval | 2.0.0 |
| 2018-06 | RAN#80 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2018-09 | RAN#81 | RP-181896 | 0001 | 1 | F | Extendng loewer frequecy range in Band 41 for EN DC_41_79 | 15.1.0 |
| 2018-09 | RAN#81 | RP-181896 | 0002 |  | F | Correction for DC_3_n3 to Rel.15 TR 37.863-01-01 | 15.1.0 |
| 2018-12 | RAN#82 | RP-182359 | 0003 | 1 | F | CR to 37.863-01-01:Corrections on TR for IMD calculations and add TP for MSD for DC_5A_n78A | 15.2.0 |
| 2018-12 | RAN#82 | RP-182359 | 0004 |  | F | CR on MSD for EN-DC including Band 66 and n78 to TR 37.863-01-01 | 15.2.0 |
| 2018-12 | RAN#82 | RP-182359 | 0006 |  | F | CR towards TR 37.863-01-01 to correct n260 BW classes | 15.2.0 |
| 2019-06 | RAN#84 | RP-191237 | 0007 |  | F | CR to reflect agreed MSD analysis of DC_25A-n41A for TR37.863-01-01 | 15.3.0 |
| 2020-06 | RAN#88 | RP-200984 | 0008 | 1 | F | CR for TR37.863-01-01: TP for missing MSD due to UL harmonic and cross band isolation for band combinations | 15.4.0 |