| 3GPP TS 29.509 V18.5.0 (2024-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Authentication Server Services; Stage 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - France Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	8

2	References	8

3	Definitions and abbreviations	9

3.1	Definitions	9

3.2	Abbreviations	9

4	Overview	10

4.1	Introduction	10

5	Services offered by the AUSF	11

5.1	Introduction	11

5.2	Nausf_UEAuthentication Service	11

5.2.1	Service Description	11

5.2.2	Service Operations	11

5.2.2.1	Introduction	11

5.2.2.2	Authenticate	12

5.2.2.2.1	General	12

5.2.2.2.2	5G AKA	12

5.2.2.2.3	EAP-based authentication method	14

5.2.2.2.4	Authentication for FN-RG	15

5.2.2.2.5	Authentication Result Removal with 5G AKA method	16

5.2.2.2.6	Authentication Result Removal with EAP-AKA' method	17

5.2.2.3	Deregister	17

5.2.2.3.1	General	17

5.2.2.4	ProseAuthenticate	18

5.2.2.4.1	General	18

5.2.2.4.2	ProSe Authentication Result Removal with EAP-AKA' method	20

5.3	Nausf_SoRProtection Service	20

5.3.1	Service Description	20

5.3.2	Service Operations	21

5.3.2.1	Introduction	21

5.3.2.2	Protect	21

5.3.2.2.1	General	21

5.4	Nausf_UPUProtection Service	22

5.4.1	Service Description	22

5.4.2	Service Operations	22

5.4.2.1	Introduction	22

5.4.2.2	Protect	22

5.4.2.2.1	General	22

6	API Definitions	23

6.1	Nausf_UEAuthentication Service API	23

6.1.1	API URI	23

6.1.2	Usage of HTTP	23

6.1.2.1	General	23

6.1.2.2	HTTP standard headers	23

6.1.2.2.1	General	23

6.1.2.2.2	Content type	23

6.1.2.3	HTTP custom headers	23

6.1.2.3.1	General	23

6.1.3	Resources	24

6.1.3.1	Overview	24

6.1.3.2	Resource: ue-authentications (Collection)	25

6.1.3.2.1	Description	25

6.1.3.2.2	Resource Definition	25

6.1.3.2.3	Resource Standard Methods	26

6.1.3.2.4	Resource Custom Operations	28

6.1.3.3	Resource: 5g-aka-confirmation (Document)	29

6.1.3.3.1	Description	29

6.1.3.3.2	Resource Definition	29

6.1.3.3.3	Resource Standard Methods	29

6.1.3.4	Resource: eap-session (Document)	31

6.1.3.4.1	Description	31

6.1.3.4.2	Resource Definition	31

6.1.3.4.3	Resource Standard Methods	32

6.1.3.5	Resource: rg-authentications (Collection)	34

6.1.3.5.1	Description	34

6.1.3.5.2	Resource Definition	34

6.1.3.5.3	Resource Standard Methods	34

6.1.3.6	Resource: prose-authentications (Collection)	35

6.1.3.6.1	Description	35

6.1.3.6.2	Resource Definition	36

6.1.3.6.3	Resource Standard Methods	36

6.1.3.7	Resource: prose-auth (Document)	38

6.1.3.7.1	Description	38

6.1.3.7.2	Resource Definition	38

6.1.3.7.3	Resource Standard Methods	38

6.1.4	Custom Operations without associated resources	40

6.1.4.1	Overview	40

6.1.5	Notifications	40

6.1.5.1	General	40

6.1.6	Data Model	41

6.1.6.1	General	41

6.1.6.2	Structured data types	42

6.1.6.2.1	Introduction	42

6.1.6.2.2	Type: AuthenticationInfo	43

6.1.6.2.3	Type: UEAuthenticationCtx	44

6.1.6.2.4	Type: 5gAuthData	44

6.1.6.2.5	Type: Av5gAka	44

6.1.6.2.6	Type: ConfirmationData	44

6.1.6.2.7	Type: EapSession	45

6.1.6.2.8	Type: ConfirmationDataResponse	45

6.1.6.2.9	Type: RgAuthenticationInfo	45

6.1.6.2.10	Type: RgAuthCtx	46

6.1.6.2.11	Type: DeregistrationInfo	46

6.1.6.2.12	Type: ProSeAuthenticationInfo	46

6.1.6.2.13	Type: ProSeAuthenticationCtx	46

6.1.6.2.14	Type: ProSeEapSession	47

6.1.6.2.15	Type: ProSeAuthData	47

6.1.6.2.16	Type: ProSeAuthenticationResult	47

6.1.6.3	Simple data types and enumerations	48

6.1.6.3.1	Introduction	48

6.1.6.3.2	Simple data types	48

6.1.6.3.3	Enumeration: AuthType	48

6.1.6.3.4	Enumeration: AuthResult	48

6.1.6.3.5	Relation Types	48

6.1.6.4	Binary data	49

6.1.6.4.1	Introduction	49

6.1.7	Error Handling	49

6.1.7.1	General	49

6.1.7.2	Protocol Errors	49

6.1.7.3	Application Errors	49

6.1.8	Security	50

6.1.9	Feature Negotiation	50

6.1.10	HTTP redirection	50

6.2	Nausf_SoRProtection Service API	51

6.2.1	API URI	51

6.2.2	Usage of HTTP	51

6.2.2.1	General	51

6.2.2.2	HTTP standard headers	51

6.2.2.2.1	General	51

6.2.2.2.2	Content type	51

6.2.2.3	HTTP custom headers	51

6.2.2.3.1	General	51

6.2.3	Resources	51

6.2.3.1	Overview	51

6.2.3.2	Resource: supi (Custom operation)	52

6.2.3.2.1	Description	52

6.2.3.2.2	Resource Definition	52

6.2.3.2.3	Resource Standard Methods	52

6.2.3.2.4	Resource Custom Operations	53

6.2.4	Custom Operations without associated resources	54

6.2.4.1	Overview	54

6.2.5	Notifications	54

6.2.5.1	General	54

6.2.6	Data Model	54

6.2.6.1	General	54

6.2.6.2	Structured data types	55

6.2.6.2.1	Introduction	55

6.2.6.2.2	Type: SorInfo	55

6.2.6.2.3	Type: SorSecurityInfo	55

6.2.6.2.4	Type: SteeringInfo	56

6.2.6.2.5	Type: SteeringContainer	56

6.2.6.3	Simple data types and enumerations	56

6.2.6.3.1	Introduction	56

6.2.6.3.2	Simple data types	56

6.2.6.3.3	Enumeration: AccessTech	57

6.2.7	Error Handling	57

6.2.7.1	General	57

6.2.7.2	Protocol Errors	57

6.2.7.3	Application Errors	57

6.2.8	Security	57

6.2.9	Feature Negotiation	58

6.2.10	HTTP redirection	58

6.3	Nausf_UPUProtection Service API	58

6.3.1	API URI	58

6.3.2	Usage of HTTP	59

6.3.2.1	General	59

6.3.2.2	HTTP standard headers	59

6.3.2.2.1	General	59

6.3.2.2.2	Content type	59

6.3.2.3	HTTP custom headers	59

6.3.2.3.1	General	59

6.3.3	Resources	59

6.3.3.1	Overview	59

6.3.3.2	Resource: supi (Custom operation)	60

6.3.3.2.1	Description	60

6.3.3.2.2	Resource Definition	60

6.3.3.2.3	Resource Standard Methods	60

6.3.3.2.4	Resource Custom Operations	60

6.3.4	Custom Operations without associated resources	61

6.3.4.1	Overview	61

6.3.5	Notifications	61

6.3.5.1	General	61

6.3.6	Data Model	62

6.3.6.1	General	62

6.3.6.2	Structured data types	62

6.3.6.2.1	Introduction	62

6.3.6.2.2	Type: UpuInfo	63

6.3.6.2.3	Type: UpuSecurityInfo	63

6.3.6.2.4	Type: UpuData	64

6.3.6.3	Simple data types and enumerations	64

6.3.6.3.1	Introduction	64

6.3.6.3.2	Simple data types	64

6.3.6.3.3	Void	64

6.3.7	Error Handling	65

6.3.7.1	General	65

6.3.7.2	Protocol Errors	65

6.3.7.3	Application Errors	65

6.3.8	Security	65

6.3.9	Feature Negotiation	65

6.3.10	HTTP redirection	66

Annex A (normative):	OpenAPI specification	67

A.1	General	67

A.2	Nausf_UEAuthentication API	67

A.3	Nausf_SoRProtection API	80

A.4	Nausf_UPUProtection API	82

Annex B (Informative):	Use of EAP-TLS	85

B.1	General	85

B.2	EAP method: EAP-TLS	85

Annex C (informative):	Withdrawn API versions	87

C.1	General	87

C.2	Nausf_SoRProtection API	87

Annex D (informative):	Change history	88

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nausf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the AUSF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 33.501 [8].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 9113: "HTTP/2".

[7]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[8]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[9]	Void

[10]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[11]	IETF RFC 9457: "Problem Details for HTTP APIs".

[12]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[15]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[16]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[17]	IETF RFC 9048: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[18]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[19]	IETF RFC 4648: "The Base16, Base32 and Base64 Data Encodings".

[20]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[21]	3GPP TR 21.900: "Technical Specification Group working methods".

[22]	3GPP TS 29.544: "5G System; SP-AF Services; Stage 3".

[23]	3GPP TS 23.316: " Wireless and wireline convergence access support for the 5G System (5GS); Release 16".

[24]	3GPP TS 29.524: "5G System; Cause codes mapping between 5GC interfaces; Stage 3".

[25]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0

[26]	3GPP TS 33.503: " Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G Prose	5G Proximity-based Services

AMF	Access and Mobility Management Function

API	Application Programming Interface

AUSF	Authentication Server Function

CH	Credentials Holder

DCS	Default Credentials Server

FN-RG	Fixed Network RG

MAC	Message Authentication Code

N5GC	Non-5G-Capable

NF	Network Function

RG	Residential Gateway

SEAF	SEcurity Anchor Function

SNPN	Stand-alone Non-Public Network

SoR	Steering of Roaming

URI	Uniform Resource Identifier

UPU	UE Parameters Update

W-AGF	Wireline Access Gateway Function

# 4	Overview

## 4.1	Introduction

The Network Function (NF) Authentication Server Function (AUSF) is the network entity in the 5G Core Network (5GC) supporting the following functionalities:

-	Authenticate the UE for the requester NF,

-	Provide keying material to the requester NF,

-	Protect the Steering Information List for the requester NF.

-	Protect the UE Parameter Update Data for the requester NF.

Figure 4-1 shows the reference architecture for the AUSF:

Figure 4-1: AUSF in 5G System architecture

This figure represents the AUSF architecture in the Service-based Architecture model. In the reference point model, the interface between the AMF and the AUSF is named N12, the interface between the NSWOF and the AUSF is named N60. In this release, the SEAF function is collocated with the AMF. The AUSF may provide the service to the UDM.

Figure 4-1 illustates PLMN level scenarios, but this architecture is also applicable to the SNPN scenarios, as explained below.

In the case of SNPN, the AUSF provides services e.g. in the following scenarios:

-	For an SNPN for which roaming is not supported (see 3GPP TS 23.501 [2], clause 5.30.2.0)

-	For the case of UE access to SNPN using credentials from Credentials Holder (see 3GPP TS 23.501 [2], clause 5.30.2.9)

-	For the case of Onboarding of UEs for SNPNs (see 3GPP TS 23.501 [2], clause 5.30.2.10)

-	For the case of supports the authentication of a 5G ProSe Remote UE or a 5G ProSe End UE  (see 3GPP TS 33.503 [26], clause 7.3)

# 5	Services offered by the AUSF

## 5.1	Introduction

The AUSF offers to NF Service Consumers (e.g. AMF) the following services:

-	Nausf_UEAuthentication

-	Nausf_SoRProtection

-	Nausf_UPUProtection

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nausf_UEAuthentication | 6.1 | AUSF UE Authentication Service | TS29509_Nausf_UEAuthentication.yaml | nausf-auth | A.2 |
| Nausf_SoRProtection | 6.2 | AUSF SoR Protection Service | TS29509_Nausf_SoRProtection.yaml | nausf-sorprotection | A.3 |
| Nausf_UPUProtection | 6.3 | AUSF UPU Protection Service | TS29509_Nausf_UPUProtection.yaml | nausf-upuprotection | A.4 |

AUSF provides services to the following SNPN scenarios (see clauses 5.30.2 in 3GPP TS 23.501 [2]):

-	In a SNPN where roaming is not supported;

-	In the case of UE access to SNPN using credentials from Credentials Holder with AAA-S;

-	In the case of UE access to SNPN using credentials from Credentials Holder with AUSF and UDM;

-	In the case of Onboarding of UEs for SNPNs without using Default Credentials Server;

-	In the case of Onboarding of UEs for SNPNs using Default Credentials Server with AAA-S;

-	In the case of Onboarding of UEs for SNPNs using Default Credentials Server with AUSF and UDM.

## 5.2	Nausf_UEAuthentication Service

### 5.2.1	Service Description

The AUSF is acting as NF Service Producer. It provides UE authentication service to the requester NF. The NF Service Consumer is the AMF.

For this service, the following service operations are defined:

-	Authenticate

This service permits to authenticate the UE and to provide one or more master keys which are used by the AMF to derived subsequent keys.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operation defined for the Nausf_UEAuthentication is as follows:

-	Authenticate: It allows the AMF to authenticate the UE and allows the AMF to inform AUSF to remove the UE authentication result in the UDM.

-	Deregister: It allows UDM to request the AUSF to clear the Security Context.

-	ProseAuthenticate: It allows the AUSF of the 5G ProSe Remote UE to support the authentication of a 5G ProSe Remote UE via the AMF of the 5G ProSe UE-to-Network Relay or allows the AUSF of the 5G ProSe End UE to support the authentication of a 5G ProSe End UE via the AMF of the 5G ProSe UE-to-UE Relay.

#### 5.2.2.2	Authenticate

##### 5.2.2.2.1	General

The service operation "Authenticate" permits the requester NF to initiate the Authentication of the UE by providing the following information to the AUSF:

-	UE id (i.e. SUPI or SUCI)

-	Serving Network Name

The AUSF retrieves the UE's subscribed authentication method from the UDM and depending on the information provided by the UDM, the AUSF enters in one of the following procedures:

-	5G-AKA

-	EAP-based authentication'

For those two different procedures a new resource is generated by the AUSF. The content of the resource will depend on the procedure and will be returned to the AMF.

This service operation "Authenticate" also permits the requester NF to initiate the Authentication of the FN-RG registration via W-AGF by providing the following information to the AUSF:

-	UE id (i.e. SUCI)

-	Indication that the W-AGF has authenticated the FN-RG

The AUSF retrieves the UE's SUPI, indication that authentication is not required for the FN-RG from the UDM, and AUSF shall not perform the authentication.

The service operation "Authenticate" also permits the requester NF to inform the AUSF to remove the UE authentication result in the UDM.

##### 5.2.2.2.2	5G AKA

In this procedure, the NF Service Consumer (AMF) requests the authentication of the UE by providing UE related information and the Serving Network Name to the NF Service Producer (AUSF), which retrieves UE related data and authentication method from the UDM. In this case the retrieved authentication method is 5G AKA. The NF Service Consumer (AMF) shall then return to the AUSF the result received from the UE:

Figure 5.2.2.2.2-1: 5G AKA

1.	The NF Service Consumer (AMF) shall send a POST request to the AUSF. The content shall contain at least the UE Id and the Serving Network Name.

2a.	On success, "201 Created" shall be returned. The content shall contain the representation of the resource created and the "Location" header shall contain the URI of the created resource (e.g. .../ue_authentications/{authCtxId}). The AUSF generates a sub-resource "5g-aka-confirmation". There shall be only one sub-resource "5g-aka-confirmation" per UE per Serving Network identified by the supiOrSuci and servingNetworkName in AuthenticationInfo. The AUSF shall provide an hypermedia link towards this sub-resource in the content to indicate to the AMF where it shall send a PUT for the confirmation.

2b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3. If the serving network is not authorized, the AUSF shall use the SERVING_NETWORK_NOT_AUTHORIZED "cause".

3.	Based on the relation type, the NF Service Consumer (AMF) deduces that it shall send a PUT containing the "RES*" provided by the UE to the URI provided by the AUSF or derived by itself. The NF Service Consumer (AMF) shall also send a PUT containing null value in the RES* to indicate the failure to the AUSF for the following cases:

-	if the UE is not reached, and the RES* is never received by the NF Service Consumer (AMF);

-	the comparation of the HRES* and HXRES* is unsuccessful in the NF Service Consumer (AMF);

-	the authentication failure is received from the UE, e.g. synchronization failure or MAC failure;

4a.	On success, "200 OK" shall be returned. If the UE is not authenticated, e.g. the verification of the RES* was not successful in the AUSF, the AUSF shall set the value of AuthResult to AUTHENTICATION_FAILURE.

In SNPN onboarding scenarios, if the UE is authenticated successfully, the AUSF may include in the response the FQDN(s) and/or IP address(es) of an onboarding Provisioning Server (PVS) to the NF Service Consumer (AMF); see 3GPP TS 23.501 [2], clause 5.30.2.10.

4b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.3.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.3.1-3.

##### 5.2.2.2.3	EAP-based authentication method

5.2.2.2.3.1	General

In this procedure, the NF Service Consumer requests the authentication of the UE by providing UE related information and the serving network and the EAP-based authentication is selected (see IETF RFC 3748 [18]). EAP messages are exchanged between a UE acting as EAP peer, an NF Service Consumer (AMF/SEAF, NSWOF) acting as a pass-through authenticator and the AUSF acting as the EAP server.

5.2.2.2.3.2	EAP method: EAP-AKA'

EAP-AKA' is the EAP method used in this procedure

Figure 5.2.2.2.3-1: EAP-based authentication with EAP-AKA' method

1.	The NF Service Consumer (AMF, NSWOF) shall send a POST request to the AUSF. The content shall contain at least the UE Id, Serving Network Name. If the consumer is an NSWOF the NSWO Indicator shall be present in the content.

2a.	On success, "201 Created" shall be returned. The content shall contain the representation of the resource generated and the "Location" header shall contain the URI of the generated resource (e.g. .../ue_authentications/{authCtxId}). The AUSF generates a sub-resource "eap-session". There shall be only one sub-resource "eap-session" per UE per Serving Network identified by the supiOrSuci and servingNetworkName in AuthenticationInfo. The AUSF shall provide a hypermedia link towards this sub-resource in the payload to indicate to the AMF or NSWOF where it shall send a POST containing the EAP packet response. The content shall also contain the EAP packet EAP-Request/AKA'-Challenge.

2b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3. In particular, if the serving network is not authorized, the AUSF shall use the "Cause" SERVING_NETWORK_NOT_AUTHORIZED.

3.	Based on the relation type, the NF Service Consumer (AMF, NSOWF) shall send a POST request including the EAP-Response/AKA' Challenge received from the UE. The POST request is sent to the URI provided by the AUSF or derived by the NF Service Consumer (AMF, NSWOF).

4a.	On success, and if the AUSF and the UE have indicated the use of protected successful result indications as in IETF RFC 9048 [17], the AUSF shall reply with a "200 OK" HTTP message containing the EAP Request/AKA' Notification and an hypermedia link towards the sub-resource "eap-session".

4b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.4.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.4.3.1-3.

NOTE: Steps 4 to 5 are optional.

5.	The NF Service Consumer (AMF, NSWOF) shall send a POST request including the EAP Response/AKA' Notification received from the UE. The POST request is sent to the URI provided by the AUSF or derived by the NF Service Consumer (AMF, NSWOF).

6a.	If the EAP authentication exchange is successfully completed (with or without the optional Notification Request/Response messages exchange), "200 OK" shall be returned to the NF Service Consumer (AMF, NSWOF). The content shall contain the result of the authentication, an EAP success/failure and if the authentication is successful the Kseaf if the consumer is an AMF or the MSK if the consumer is a NSWOF (as indicated by the NSWO indicator received in step 1). If the UE is not authenticated, the AUSF shall set the authResult to AUTHENTICATION_FAILURE.

In SNPN onboarding scenarios, if the UE is authenticated successfully, the AUSF may include in the response the FQDN(s) and/or IP address(es) of an onboarding Provisioning Server (PVS) to the NF Service Consumer (AMF) ; see 3GPP TS 23.501 [2], clause 5.30.2.10.

6b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.4.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.4.3.1-3.

5.2.2.2.3.3	EAP method: EAP-TLS

The EAP-TLS method can be used in private networks as an EAP method (see 3GPP TS 33.501 [8] Annex B.1). The corresponding stage 3 implementation is described in Annex B.

The EAP-TLS method is applicable for N5GC devices behind Cable RGs in private networks or in deployment scenarios with wireline access; see 3GPP TS 33.501 [8] Annex O.

5.2.2.2.3.4	EAP method: EAP-TTLS

The EAP-TTLS method can be used as an EAP method (see 3GPP TS 33.501 [8] Annex U) in case of UE access to SNPN using credentials from Credential Holder with AAA Server.

##### 5.2.2.2.4	Authentication for FN-RG

In this procedure, the NF Service Consumer (AMF) requests the authentication of the FN-RG registration via W-AGF by providing the SUCI of the FN-RG and the authenticated indication.

Figure 5.2.2.2.4-1: Authentication for FN-RG

1.	The NF Service Consumer (AMF) shall send a POST request to the AUSF. The content shall contain at least the UE Id and the authenticated indication.

2a.	On success, "201 Created" shall be returned. The content shall contain the representation of the resource created and the "Location" header shall contain the URI of the created resource (e.g. .../rg-authentications/{authCtxId}).

2b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.4.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.4.3.1-3.

##### 5.2.2.2.5	Authentication Result Removal with 5G AKA method

In the case that the Purge of subscriber data in AMF after the UE deregisters from the network or the NAS SMC fails following the successful authentication in the registration procedure, the NF Service Consumer (AMF) requests the AUSF to inform the UDM to remove the authentication result:

Figure 5.2.2.2.5-1: Authentication Result Removal with 5G AKA method

1.	The NF Service Consumer (AMF) shall send a DELETE request to the resource URI representing the sub-resource "5g-aka-confirmation". The request body shall be empty.

2a.	On success, "204 No Content" shall be returned. The AUSF shall send a DELETE request to the UDM for removing the authentication result of the UE after receiving the above DELETE request message.

2b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.3.3.2-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.3.2-3..

##### 5.2.2.2.6	Authentication Result Removal with EAP-AKA' method

In the case that the Purge of subscriber data in AMF after the UE deregisters from the network or the NAS SMC fails following the successful authentication the registration procedure, the NF Service Consumer (AMF) requests the AUSF to inform the UDM to remove the authentication result:

Figure 5.2.2.2.6-1: Authentication Result Removal with EAP-AKA' method

1.	The NF Service Consumer (AMF) shall send a DELETE request to the resource URI representing the sub-resource "eap-session". The request body shall be empty.

2a.	On success, "204 No Content" shall be returned. The AUSF shall send a DELETE request to the UDM for removing the authentication result of the UE after receiving the above DELETE request message.

2b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.4.3.2-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.4.3.2-3.

#### 5.2.2.3	Deregister

##### 5.2.2.3.1	General

The Deregister service operation is used in the following scenario:

-	Deletion of security context in AUSF

The NF Service Consumer (e.g. UDM) uses this service operation to request the AUSF to clear the stale security context, after the UE has been successfully (re)authenticated in same or different Serving Network via another AUSF Instance, e.g. due to registration via another access-type; so as to ensure only latest Kausf is maintained in the network. The service may also be used by UDM when the UE is no longer registered via any access-type or serving-network. It is responsibility of NF Service Consumers to ensure that security context being deleted does not hold the latest Kausf if UE is also connected via another Serving-Network.

Figure 5.2.2.3.1-1: UE Context Clean-up in AUSF

1.	The NF Service Consumer (e.g. UDM) shall send a POST request to the AUSF that was used to authenticate the UE. The content shall contain the UE id (e.g. SUPI).

2a.	On success, "204 No Content" shall be returned.

2b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.2.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.4.2.2-2.

#### 5.2.2.4	ProseAuthenticate

##### 5.2.2.4.1	General

The ProseAuthenticate service operation is used in the following scenario:

-	Authenticate the 5G ProSe Remote UE in AUSF or 5G ProSe End UE

The NF Service Consumer (AMF) requests the authentication of the 5G ProSe Remote UE or the 5G ProSe End UE  by providing 5G ProSe Remote UE or the 5G ProSe End UE  related information (SUCI or CP-PRUK ID), the Relay Service Code and Nonce_1 to the NF Service Producer (AUSF) in the initial authentication request. When CP-PRUK ID is provided, the AUSF shall retrieve the CP-PRUK from PAnF; when SUCI is provided, the AUSF retrieves 5G ProSe Remote UE or the 5G ProSe End UE  related data and authentication method from the UDM. In this case the retrieved authentication method is EAP-AKA. The NF Service Consumer (AMF) shall then return to the AUSF the result received from the 5G ProSe Remote UE or the 5G ProSe End UE  to continue the authentication:

Figure 5.2.2.4.1-1: ProSe Authentication

1.	The NF Service Consumer (AMF) shall send a POST request to the AUSF. The content shall contain the UE Id (SUCI) or CP-PRUK ID, Relay Service Code and Nonce_1.

2a.	On success, "201 Created" shall be returned if UE Id (SUCI) is received. The content shall contain the representation of the resource generated and the "Location" header shall contain the URI of the generated resource (e.g. .../prose_authentications/{authCtxId}). The AUSF generates a sub-resource "prose-auth". There shall be only one sub-resource "prose-auth" per UE identified by the supiOrSuci in ProSeAuthenticationInfo. The AUSF shall provide a hypermedia link towards this sub-resource in the content to indicate to the AMF where it shall send a POST containing the EAP packet response. The content shall also contain the EAP packet EAP-Request/AKA'-Challenge.

2b. On success, "200 OK" shall be returned if CP-PRUK ID is received. The payload body shall contain the KNR_ProSe and Nonce_2. Step 3 to 6 are skipped.

2c.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3.

3.	Based on the relation type, the NF Service Consumer (AMF) shall send a POST request including the EAP-Response/AKA' Challenge received from the 5G ProSe Remote UE or the 5G ProSe End UE. The POST request is sent to the URI provided by the AUSF or derived by the NF Service Consumer (AMF).

4a.	On success, and if the AUSF and the UE have indicated the use of protected successful result indications as in IETF RFC 5448 [9] (to be superseded by draft-ietf-emu-rfc5448bis [17]), the AUSF shall reply with a "200 OK" HTTP message containing the EAP Request/AKA' Notification and an hypermedia link towards the sub-resource "prose-auth".

4b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.4.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.4.3.1-3.

NOTE: Steps 4 to 5 are optional.

5.	The NF Service Consumer (AMF) shall send a POST request including the EAP Response/AKA' Notification received from the UE. The POST request is sent to the URI provided by the AUSF or derived by the NF Service Consumer (AMF).

6a.	If the ProSe authentication exchange is successfully completed (with or without the optional Notification Request/Response messages exchange), "200 OK" shall be returned to the NF Service Consumer (AMF). The content shall contain the result of the authentication, an EAP success/failure. The content shall also contain the KNR_ProSe, Nonce_2 and CP-PRUK ID if the authentication is successful. If the 5G ProSe Remote UE or the 5G ProSe End UE  is not authenticated, the AUSF shall set the authResult to AUTHENTICATION_FAILURE.

6b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.4.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.4.3.1-3.

##### 5.2.2.4.2	ProSe Authentication Result Removal with EAP-AKA' method

In the case that the authentication of a 5G ProSe Remote UE or the 5G ProSe End UEfails, the NF Service Consumer (AMF) requests the AUSF to inform the UDM to remove the authentication result:

Figure 5.2.2.4.2-1: ProSe Authentication Result Removal with EAP-AKA' method

1.	The NF Service Consumer (AMF) shall send a DELETE request to the resource URI representing the sub-resource "prose-auth". The request body shall be empty.

2a.	On success, "204 No Content" shall be returned. The AUSF shall send a DELETE request to the UDM for removing the authentication result of the UE after receiving the above DELETE request message.

2b.	On failure or redirection, one of the HTTP status code listed in table 6.1.3.4.3.2-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.4.3.2-3.

## 5.3	Nausf_SoRProtection Service

### 5.3.1	Service Description

The AUSF is acting as NF Service Producer. It provides SoRProtection service to the NF Service Consumer.

This service permits to provide the NF Service Consumer (e.g. UDM) with the SoR-MAC-IAUSF and CounterSoR to protect the Steering Information from being tampered with or removed by the VPLMN.

NOTE:	If the Steering Information is not available or HPLMN determines that no steering of the UE is required, a SOR transparent container information element with an HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE protected by SoR-MAC-IAUSF and CounterSoR is still sent to the UE during registration. The Steering Information in such a case, the NF Service Consumer shall send an empty list to the AUSF when consuming the Nausf_SoRProtection Service.

In option this service also allows to provide the NF Service Consumer (e.g. UDM) with the SoR-XMAC-IUE that allows the NF Service Consumer (e.g. UDM) to verify that the UE received the Steering Information List.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operation defined for the Nausf_SoRProtection is as follows:

-	Protect

#### 5.3.2.2	Protect

##### 5.3.2.2.1	General

The Protect service operation is used in the following procedures:

-	Procedure for steering of UE in VPLMN during registration (see clause 6.14.2.1 of 3GPP TS 33.501 [8]);

-	Procedure for steering of UE in VPLMN after registration (see clause 6.14.2.2 of 3GPP TS 33.501 [8]).

The NF Service Consumer (e.g. UDM) uses this service operation to request the AUSF to compute the SoR-MAC-IAUSF and the CounterSoR by providing Steering Information. The NF Service Consumer (e.g. UDM) may also request the AUSF to compute the SoR-XMAC-IUE by providing the indication that an acknowledgement is requested from the UE.

Figure 5.3.2.2.1-1: Steering of UE in VPLMN

1.	The NF Service Consumer (e.g. UDM) shall send a POST request to the AUSF that was used to authenticate the UE. The content shall contain the Steering Information and the acknowledge indication.

2a.	On success, "200 OK" shall be returned. The content shall contain the requested security material (e.g. SoR-MAC-IAUSF, CounterSoR, SoR-XMAC-IUE) necessary to protect the Steering of Roaming procedure.

SoR Header shall be used to form the input as one of multiple parameters to calculate the SoR-MAC-IAUSF. If SoRHeader attribute is not provided by NF Service Consumer (e.g. UDM) as part of SorInfo, SoR Header shall be constructed by AUSF based on the information received in the request and encoded as specified in clause 9.11.3.51 of 3GPP TS 24.501[20].

2b.	On failure or redirection, one of the HTTP status code listed in table 6.2.3.2.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.2.3.2.4.2.2-21. If the CounterSoR associated with the KAUSF of the UE, is about to wrap around, the AUSF shall use the "COUNTER-WRAP" cause.

## 5.4	Nausf_UPUProtection Service

### 5.4.1	Service Description

The AUSF is acting as NF Service Producer. It provides UPUProtection service to the NF Service Consumer.

This service permits to provide the NF Service Consumer (e.g. UDM) with the UPU-MAC-IAUSF and CounterUPU to protect the UE Parameters Update Data from being tampered with or removed.

In option this service also allows to provide the NF Service Consumer (e.g. UDM) with the UPU-XMAC-IUE that allows the NF Service Consumer (e.g. UDM) to verify that the UE received UE Parameters Update Data correctly.

### 5.4.2	Service Operations

#### 5.4.2.1	Introduction

The service operation defined for the Nausf_UPUProtection is as follows:

-	Protect

#### 5.4.2.2	Protect

##### 5.4.2.2.1	General

The Protect service operation is used in the following procedures:

-	Procedure for UE Parameters Update (see clause 6.15.2.1 of 3GPP TS 33.501 [8]).

The NF Service Consumer (e.g. UDM) uses this service operation to request the AUSF to compute the UPU-MAC-IAUSF and CounterUPU by providing the UE Parameters Update Data (UPU Data). The NF Service Consumer (e.g. UDM) may also request the AUSF to compute the UPU-XMAC-IUE by providing the indication that an acknowledgement is requested from the UE.

Figure 5.4.2.2-1: UE Parameters Update in VPLMN

1.	The NF Service Consumer (e.g. UDM) shall send a POST request to the AUSF that was used to authenticate the UE and stores the latest KAUSF for the UE. The content shall contain the UE Parameters Update Data (UPU Data), the UPU Header and the acknowledge indication.

2a.	On success, "200 OK" shall be returned. The content shall contain the requested security material necessary to protect the UE Parameters Update procedure.

2b.	On failure or redirection, one of the HTTP status code listed in table 6.3.3.2.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.3.3.2.4.2.2-2. If the CounterUPU associated with the KAUSF of the UE, is about to wrap around, the AUSF shall use the "COUNTER-WRAP" cause.

# 6	API Definitions

## 6.1	Nausf_UEAuthentication Service API

### 6.1.1	API URI

URIs of this API shall have the following root:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "nausf-auth".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, as defined in IETF RFC 9113 [6], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

The usage of HTTP standard headers is specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.1.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [7], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 9457 [11]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json"

-	The 3GPP hypermedia format as defined in 3GPP TS 29.501 [5]. The use of the 3GPP hypermedia format in a HTTP response body shall be signalled by the content type "application/3gppHal+json"

#### 6.1.2.3	HTTP custom headers

##### 6.1.2.3.1	General

The usage of HTTP custom headers shall be supported as specified in clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.1.3	Resources

#### 6.1.3.1	Overview

The structure of the Resource URIs of the Nausf_UEAuthentication service is shown in Figure 6.1.3.1-1

Figure 6.1.3.1-1: Resource URI structure of the AUSF API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ue-authentications (Collection) | /ue-authentications | POST | Initiate the authentication process by providing inputs related to the UE |
|  | /ue-authentications/deregister | deregister (POST) | Clear the security context of the UE |
| Individual UE authentication (Document) | /ue-authentications/{authCtxId} |  | See NOTE 1 |
| 5g-aka-confirmation (Document) | /ue-authentications/{authCtxId}/5g-aka-confirmation | PUT | Put the UE response from the 5G-AKA process. |
|  |  | DELETE | DELETE the authentication result. |
| eap-session (Document) | /ue-authentications/{authCtxId}/eap-session | POST | Post the EAP response from the UE. See NOTE. |
|  |  | DELETE | DELETE the authentication result. |
| rg-authentications (Collection) | /rg-authentications | POST | Initiate the authentication process by providing inputs related to the FN-RG |
| Individual RG authentication (Document) | /rg-authentications/{authCtxId} |  | See NOTE 3 |
| prose-authentications (Collection) | /prose-authentications | POST | Initiate the authentication process by providing inputs related to the 5G ProSe Remote UE or the 5G ProSe End UE. |
| prose-auth (Document) | /prose-authentications/{authCtxId}/prose-auth | POST | Post the EAP response from the 5G ProSe Remote UE or the 5G ProSe End UE. See NOTE. |
|  |  | DELETE | DELETE the authentication result. |
| NOTE 1: This resource represents the created individual UE authentication, the URI of the created resource is contained in the "Location" header of the "201 Created" response (See step 2a of Figure 5.2.2.2.2-1 and Figure 5.2.2.2.3.2-1). There are no service operations defined on this resource. NOTE 2: This POST is used to provide EAP response to the AUSF in a sub-resource (Document) generated by the first POST operation. As this operation is not idempotent (it triggers subsequent EAP operations), a PUT was not adequate. NOTE 3: This resource represents the created individual RG authentication, the URI of the created resource is contained in the "Location" header of the "201 Created" response (See step 2a of Figure 5.2.2.2.4-1). There are no service operations defined on this resource. |  |  |  |

#### 6.1.3.2	Resource: ue-authentications (Collection)

##### 6.1.3.2.1	Description

This resource represents a collection of the ue-authentication resources generated by the AUSF.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nausf-auth/<apiVersion>/ue-authentications

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AuthenticationInfo | M | 1 | Contains the UE id (i.e. SUCI or SUPI as specified in 3GPP TS 33.501 [8]) and the serving network name. It may also contain Trace Data as specified in 3GPP TS 23.501 [2]. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| UEAuthenticationCtx | M | 1 | 201 Created | Upon success, if 5G AKA is selected, the response body will contain one AV and "link" for the AMF to PUT the confirmation. If an EAP-based method is selected, the response body will contain the EAP method selected, the corresponding EAP packet request and a "link" for the AMF to POST the EAP response. The HTTP response shall include a "Location" header that contains the resource URI of the created resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents the failure to start authentication service because of input parameter error. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This case represents when the UE is not allowed to be authenticated. The "cause" attribute may be used to indicate one of the following application errors: - AUTHENTICATION_REJECTED - SERVING_NETWORK_NOT_AUTHORIZED - INVALID_HN_PUBLIC_KEY_IDENTIFIER - INVALID_SCHEME_OUTPUT |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This case represents the failure in starting the authentication service because of a server internal error. If the error is due to a problem with UDM not able to generate the requested AV, the AUSF shall indicate the following application error: "AV_GENERATION_PROBLEM" |
| ProblemDetails | O | 0..1 | 501 Not Implemented | The "cause" attribute may be used to indicate one of the following application errors: - UNSUPPORTED_PROTECTION_SCHEME This response shall not be cached. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource according to the structure: {apiRoot}/nausf-auth/<apiVersion>/ue-authentications/{authCtxId} |

Table 6.1.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

##### 6.1.3.2.4	Resource Custom Operations

6.1.3.2.4.1	Overview

Table 6.1.3.2.4.1-1: Custom operations

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/nausf-auth/<apiVersion>/ue-authentications/deregister | POST | Clear the Security Context of the UE |

6.1.3.2.4.2	Operation: deregister (POST)

6.1.3.2.4.2.1	Description

This custom operation is used by the NF service consumer (e.g. UDM) to request the AUSF to clear the Security Context, after the UE has been successfully re-authenticated in same Serving Network, or has been successfully authenticated in another Serving Network, e.g. due to registration via another access-type.

6.1.3.2.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 6.1.3.2.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.2.4.2.2-2.

Table 6.1.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeregistrationInfo | M | 1 | See 6.1.6.2.11. |

Table 6.1.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents the handover is cancelled successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of this error. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.2.4.4.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.2.4.4.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

#### 6.1.3.3	Resource: 5g-aka-confirmation (Document)

##### 6.1.3.3.1	Description

The subresource "5g-aka-confirmation" is generated by the AUSF. This subresource should not persist after the AUSF has read its content.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nausf-auth/<apiVersion>/ue-authentications/{authCtxId}/5g-aka-confirmation

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| authCtxId | string | Represents a specific ue-authentication per UE per serving network |

##### 6.1.3.3.3	Resource Standard Methods

6.1.3.3.3.1	PUT

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConfirmationData | M | 1 | Contains the "RES*" generated by the UE and provided to the AMF. |

Table 6.1.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| ConfirmationDataResponse | M | 1 | 200 OK | This case indicates that the AUSF has performed the verification of the 5G AKA confirmation. The response body shall contain the result of the authentication and the Kseaf if the authentication is successful. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents a 5G AKA confirmation failure because of input parameter error. This indicates that the AUSF was not able to confirm the authentication. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This case represents a 5G AKA confirmation failure because of a server internal error. |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

6.1.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

#### 6.1.3.4	Resource: eap-session (Document)

##### 6.1.3.4.1	Description

The "eap-session" is generated by the AUSF if an EAP-based authentication method is selected. This resource is used to handle the EAP session. This subresource should not persist after the EAP exchanges.

##### 6.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/nausf-auth/<apiVersion>/ue-authentications/{authCtxId}/eap-session

This resource shall support the resource URI variables defined in table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| authCtxId | string | Represents a specifc ue-authentication per UE per serving network |

##### 6.1.3.4.3	Resource Standard Methods

6.1.3.4.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.4.3.1-1.

Table 6.1.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.1-2 and the response data structures and response codes specified in table 6.1.3.4.3.1-3.

Table 6.1.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EapSession | M | 1 | Contains the EAP packet response (see IETF RFC 3748 [18]) from the UE and transferred by the AMF |

Table 6.1.3.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| EapSession | M | 1 | 200 OK | During an EAP session, the body response shall contain the EAP packet Response and a hypermedia link. At the end of the EAP session, the body response shall contain the EAP packet Success or Failure (see IETF RFC 3748 [18]) and the Kseaf if the authentication is successful |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents an EAP session failure because of input parameter error. This indicates that the AUSF was not able to continue the EAP session. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This case represents an EAP session failure because of a server internal error. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4] |  |  |  |  |

Table 6.1.3.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

6.1.3.4.3.2	DELETE

This method shall support the URI query parameters specified in table 6.1.3.4.3.2-1.

Table 6.1.3.4.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.2-2 and the response data structures and response codes specified in table 6.1.3.4.3.2-3.

Table 6.1.3.4.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.4.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.4.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.4.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

#### 6.1.3.5	Resource: rg-authentications (Collection)

##### 6.1.3.5.1	Description

This resource represents a collection of the rg-authentication resources generated by the AUSF.

##### 6.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/nausf-auth/<apiVersion>/rg-authentications

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.5.3	Resource Standard Methods

6.1.3.5.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.5.3.1-1.

Table 6.1.3.5.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.5.3.1-2 and the response data structures and response codes specified in table 6.1.3.5.3.1-3.

Table 6.1.3.5.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RgAuthenticationInfo | M | 1 | Contains the UE id (i.e. SUCI as specified in 3GPP TS 23.316 [23] or 3GPP TS 33.501 [8]) and the authenticated indication. |

Table 6.1.3.5.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RgAuthCtx | M | 1 | 201 Created | Upon success, the response body will contain the SUPI of the UE and the authentication indication. The HTTP response shall include a "Location" header that contains the resource URI of the created resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents the failure to start authentication service because of input parameter error. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This case represents when the UE is not allowed to be authenticated. The "cause" attribute may be used to indicate one of the following application errors: - AUTHENTICATION_REJECTED - INVALID_SCHEME_OUTPUT |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

#### 6.1.3.6	Resource: prose-authentications (Collection)

##### 6.1.3.6.1	Description

This resource represents a collection of the prose-authentication resources generated by the AUSF.

##### 6.1.3.6.2	Resource Definition

Resource URI: {apiRoot}/nausf-auth/<apiVersion>/prose-authentications

This resource shall support the resource URI variables defined in table 6.1.3.6x.2-1.

Table 6.1.3.6x.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.6.3	Resource Standard Methods

6.1.3.6.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.6.3.1-1.

Table 6.1.3.6.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.6.3.1-2 and the response data structures and response codes specified in table 6.1.3.6x.3.1-3.

Table 6.1.3.6.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ProSeAuthenticationInfo | M | 1 | Contains the UE Id (i.e. SUCI as specified in 3GPP TS 33.503 [26]) or the CP-PRUK ID, the Relay Service Code and Nonce_1. |

Table 6.1.3.6.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| ProSeAuthenticationCtx | M | 1 | 201 Created | Upon success, when UE Id (i.e. SUCI) was received in the request,the response body will contain the EAP method selected, the corresponding EAP packet request and a "link" for the AMF to POST the EAP response. The HTTP response shall include a "Location" header that contains the resource URI of the created resource. |
| ProSeAuthenticationResult | M | 1 | 200 OK | Upon success, when CP-PRUK ID was received in the request, the response body will contain the KNR_ProSe and Nonce_2. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents the failure to start authentication service because of input parameter error. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This case represents when the 5G ProSe Remote UE or the 5G ProSe End UE is not allowed to be authenticated. The "cause" attribute may be used to indicate one of the following application errors: - AUTHENTICATION_REJECTED - INVALID_HN_PUBLIC_KEY_IDENTIFIER - INVALID_SCHEME_OUTPUT |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This case represents the failure in starting the authentication service because of a server internal error. If the error is due to a problem with UDM not able to generate the requested AV, the AUSF shall indicate the following application error: "AV_GENERATION_PROBLEM" |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.6.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource according to the structure: {apiRoot}/nausf-auth/<apiVersion>/prose-authentications/{authCtxId} |

Table 6.1.3.6.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.6.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

#### 6.1.3.7	Resource: prose-auth (Document)

##### 6.1.3.7.1	Description

The "prose-auth" is generated by the AUSF if an EAP-based authentication method is selected. This resource is used to handle the EAP session. This subresource should not persist after the EAP exchanges.

##### 6.1.3.7.2	Resource Definition

Resource URI: {apiRoot}/nausf-auth/<apiVersion>/prose-authentications/{authCtxId}/prose-auth

This resource shall support the resource URI variables defined in table 6.1.3.7.2-1.

Table 6.1.3.7.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| authCtxId | string | Represents a specific prose-authentication per UE |

##### 6.1.3.7.3	Resource Standard Methods

6.1.3.7.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.7.3.1-1.

Table 6.1.3.7.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.7.3.1-2 and the response data structures and response codes specified in table 6.1.3.7.3.1-3.

Table 6.1.3.7.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ProSeEapSession | M | 1 | Contains the EAP packet response (see IETF RFC 3748 [18]) from the 5G ProSe Remote UE or the 5G ProSe End UE and transferred by the AMF |

Table 6.1.3.7.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| ProSeEapSession | M | 1 | 200 OK | During an EAP session, the body response shall contain the EAP packet Response and an hypermedia link. At the end of the EAP session, the body response shall contain the EAP packet Success or Failure (see IETF RFC 3748 [18]) and the KNR_ProSe if the authentication is successful |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents an EAP session failure because of input parameter error. This indicates that the AUSF was not able to continue the EAP session. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | This case represents an EAP session failure because of a server internal error. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4] |  |  |  |  |

Table 6.1.3.7.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | String | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.7.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

6.1.3.7.3.2	DELETE

This method shall support the URI query parameters specified in table 6.1.3.7y.3.2-1.

Table 6.1.3.7.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.7.3.2-2 and the response data structures and response codes specified in table 6.1.3.7.3.2-3.

Table 6.1.3.7.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.7.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.7.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.7.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

### 6.1.4	Custom Operations without associated resources

#### 6.1.4.1	Overview

There is no Custom Operation in the current version of this API.

### 6.1.5	Notifications

#### 6.1.5.1	General

There is no use of notification in the current version of this API.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nausf_UEAuthentication service based interface protocol.

Table 6.1.6.1-1: Nausf_UEAuthentication specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationInfo | 6.1.6.2.2 | Contains the UE id (i.e. SUCI or SUPI) and the Serving Network Name. |
| UEAuthenticationCtx | 6.1.6.2.3 | Contains the information related to the resource generated to handle the UE authentication. It contains at least the UE id, Serving Network, the Authentication Method and related EAP information or related 5G-AKA information. |
| 5gAuthData | 6.1.6.2.4 | Contains 5G authentication related information. |
| Av5gAka | 6.1.6.2.5 | Contains Authentication Vector for method 5G AKA. |
| ConfirmationData | 6.1.6.2.6 | Contains the "RES*" generated by the UE. |
| EapSession | 6.1.6.2.7 | Contains information related to the EAP session. |
| ConfirmationDataResponse | 6.1.6.2.8 | Contains the result of the authentication. |
| RgAuthenticationInfo | 6.1.6.2.9 | Contains the UE id (i.e. SUCI) and the authenticated indication. |
| RgAuthCtx | 6.1.6.2.10 | Contains the UE id (i.e. SUPI) and the authentication indication. |
| DeregistrationInfo | 6.1.6.2.11 | Contains the UE id (i.e. SUPI). |
| ProSeAuthenticationInfo | 6.1.6.2.12 | Contains the information related to the Prose authentication received from the 5G ProSe Remote UE or the 5G ProSe End UE. |
| ProSeAuthenticationCtx | 6.1.6.2.13 | Contains the information related to the resource generated to handle the ProSe authentication. |
| ProSeEapSession | 6.1.6.2.14 | Contains information related to the EAP session for the 5G ProSe Remote UE or the 5G ProSe End UE. |
| ProSeAuthData | 6.1.6.2.15 | Contains ProSe authentication related information. |
| ProSeAuthenticationResult | 6.1.6.2.16 | ProSe Authentication Result |
| EapPayload | 6.1.6.3.2 | Contains the EAP packets. |
| ResStar | 6.1.6.3.2 | Contains the RES*. |
| Kseaf | 6.1.6.3.2 | Contains the Kseaf. |
| HxresStar | 6.1.6.3.2 | Contains the HXRES*. |
| Suci | 6.1.6.3.2 | Contains the SUCI. |
| KnrProSe | 6.1.6.3.2 | Contains the KNR_ProSe. |
| Nonce1 | 6.1.6.3.2 | Contains the Nonce1. |
| Nonce2 | 6.1.6.3.2 | Contains the Nonce2. |
| Msk | 6.1.6.3.2 | Contains the Master Session Key. |
| AuthType | 6.1.6.3.3 | Indicates the authentication method used. |
| AuthResult | 6.1.6.3.4 | Indicates the result of the authentication. |

Table 6.1.6.1-2 specifies data types re-used by the Nausf_UEAuthentication service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nausf_UEAuthentication service based interface.

Table 6.1.6.1-2: Nausf_UEAuthentication re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ResynchronizationInfo | 3GPP TS 29.503 [12] |  |
| ServingNetworkName | 3GPP TS 29.503 [12] |  |
| Autn | 3GPP TS 29.503 [12] |  |
| Rand | 3GPP TS 29.503 [12] |  |
| LinksValueSchema | 3GPP TS 29.571 [10] | 3GPP Hypermedia link |
| ProblemDetails | 3GPP TS 29.571 [10] | Common Data Type used in response bodies |
| Supi | 3GPP TS 29.571 [10] |  |
| Uri | 3GPP TS 29.571 [10] |  |
| SupiOrSuci | 3GPP TS 29.571 [10] |  |
| Pei | 3GPP TS 29.571 [10] |  |
| TraceData | 3GPP TS 29.571 [10] |  |
| NfGroupId | 3GPP TS 29.571 [10] |  |
| CagId | 3GPP TS 29.571 [10] |  |
| SupportedFeatures | 3GPP TS 29.571 [10] | Supported Features |
| ServerAddressingInfo | 3GPP TS 29.571 [10] |  |
| RelayServiceCode | 3GPP TS 29.571 [10] |  |
| 5GPrukId | 3GPP TS 29.571 [10] |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

The following clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: AuthenticationInfo

Table 6.1.6.2.2-1: Definition of type AuthenticationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supiOrSuci | SupiOrSuci | M | 1 | Contains the SUPI or SUCI of the UE. |
| servingNetworkName | ServingNetworkName | M | 1 | Contains the Serving Network Name. |
| resynchronizationInfo | ResynchronizationInfo | O | 0..1 | Contains RAND and AUTS; see 3GPP 33.501 [8] clause 9.4. |
| pei | Pei | O | 0..1 | Permanent Equipment Identifier |
| traceData | TraceData | O | 0..1 | Contains TraceData provided by the UDM to the AMF |
| udmGroupId | NfGroupId | O | 0..1 | Identity of the UDM group serving the SUPI |
| routingIndicator | String | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. Pattern: '^[0-9]{1,4}$' |
| cellCagInfo | array(CagId) | O | 1..N | CAGList of the CAG cell. |
| n5gcInd | boolean | O | 0..1 | N5GC device indicator (see 3GPP TS 33.501 [8]) When present, this IE shall be set as follows: - true: authentication is for a N5GC device; - false (default): authentication is not for a N5GC device. See NOTE |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |
| pvsInfo | array(ServerAddressingInfo) | O | 1..N | FQDN(s) and/or IP address(es) of the SNPN UE onboarding Provisioning Servers (PVS). |
| nswoInd | boolean | O | 0..1 | NSWO Indicator (see 3GPP TS 33.501 [8]) When present, this IE shall be set as follows: - true: Non-Seamless WLAN Offload is applied; - false (default): Non-Seamless WLAN Offload is not applied. |
| disasterRoamingInd | boolean | O | 0..1 | Disaster Roaming Indicator (see 3GPP TS 23.502 [3]). When present, this IE shall be set as follows: - true: Disaster Roaming service is applied; - false (default): Disaster Roaming service is not applied. |
| onboardingInd | boolean | O | 0..1 | UE Onboarding Indicator for the authentication When present, this IE shall be set as follows: - true: authentication is for UE onboarding; - false (default): authentication is not for UE onboarding. |
| aun3Ind | boolean | O | 0..1 | AUN3 Indicator (see 3GPP TS 33.501 [8]) When present, this IE shall be set as follows: - true: authentication is for an AUN3 device behind RG; - false (default): authentication is not for an AUN3 device behind RG. |
| NOTE: The attribute n5gcInd is used for EAP-TLS, which is described in the informative annex O of 3GPP TS 33.501 [8] and is not mandatory to support. |  |  |  |  |

##### 6.1.6.2.3	Type: UEAuthenticationCtx

Table 6.1.6.2.3-1: Definition of type UEAuthenticationCtx

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authType | AuthType | M | 1 | Indicates the authentication method used for this UE i.e. "5G-AKA-Confirmation", "EAP-AKA'"; "EAP-TLS" or "EAP-TTLS". See clause 6.1.6.3.3 |
| _links | map(LinksValueSchema) | M | 1..N | If 5G-AKA has been selected, this IE shall contain a member whose name is set to "5g-aka" and the URI to perform the confirmation. If an EAP-based method has been selected, this IE shall contain a member whose name is set to "eap-session" and the URI to perform the EAP session. See NOTE |
| 5gAuthData | 5gAuthData | M | 1 | Contains either 5G-AKA or EAP related information. |
| servingNetworkName | ServingNetworkName | O | 0..1 | Contains the Serving Network Name. |
| NOTE: In the current version of this API, only one hypermedia link is provided |  |  |  |  |

##### 6.1.6.2.4	Type: 5gAuthData

Table 6.1.6.2.4-1: Definition of type 5gAuthData as a list of mutually exclusive alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| Av5gAka | 1 | Contains the 5G AV if 5G-AKA has been selected. |
| EapPayload | 1 | Contains the EAP packet request. |

##### 6.1.6.2.5	Type: Av5gAka

Table 6.1.6.2.5-1: Definition of type Av5gAka

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rand | Rand | M | 1 |  |
| autn | Autn | M | 1 |  |
| hxresStar | HxresStar | M | 1 |  |

##### 6.1.6.2.6	Type: ConfirmationData

Table 6.1.6.2.6-1: Definition of type ConfirmationData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| resStar | ResStar | M | 1 | Contains the "RES*" provided by the UE to the AMF. If no RES* has been provided by the UE the null value is conveyed to the AUSF. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |

##### 6.1.6.2.7	Type: EapSession

Table 6.1.6.2.7-1: Definition of type EapSession

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eapPayload | EapPayload | M | 1 | Contains the EAP packet (see IETF RFC 3748 [18]). If no EAP packet has been provided by the UE the null value is conveyed to the AUSF. |
| kSeaf | Kseaf | C | 0..1 | Shall be absent for N5GC device authentication; otherwise: If the authentication is successful and the consumer is an AMF, the Kseaf shall be included |
| _links | map(LinksValueSchema) | C | 1..N | If the EAP session requires another exchange e.g. for EAP-AKA' notification, this IE shall contain a member whose name is "eap-session" and the URI to continue the EAP session. See NOTE. |
| authResult | AuthResult | C | 0..1 | Indicates the result of the authentication. |
| supi | Supi | C | 0..1 | If the authentication is successful and if the AMF had provided a SUCI, this IE shall contain the SUPI of the UE. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |
| pvsInfo | array(ServerAddressingInfo) | O | 1..N | FQDN(s) and/or IP address(es) of the SNPN UE onboarding Provisioning Servers (PVS). |
| msk | Msk | C | 0..1 | If the authentication is successful and the consumer is an NSWOF as indicated by the NSWO indicator received within the AuthenticationInfo, the MSK shall be included (see 3GPP TS 33.501 [8] annex S) If the authentication is successful and the MSK indicator is received from UDM, the MSK shall be included. (see clause 7B.7 of 3GPP TS 33.501 [8]) |
| NOTE: In the current version of this API, only 0 or 1 hypermedia link is provided. |  |  |  |  |

##### 6.1.6.2.8	Type: ConfirmationDataResponse

Table 6.1.6.2.8-1: Definition of type ConfirmationDataResponse

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authResult | AuthResult | M | 1 | Indicates the result of the authentication |
| supi | Supi | C | 0..1 | If the authentication is successful and if the AMF had provided a SUCI, this IE shall contain the SUPI of the UE |
| kseaf | Kseaf | C | 0..1 | Contains the Kseaf if authentication is successful. |
| pvsInfo | array(ServerAddressingInfo) | O | 1..N | FQDN(s) and/or IP address(es) of the SNPN UE onboarding Provisioning Servers (PVS). |

##### 6.1.6.2.9	Type: RgAuthenticationInfo

Table 6.1.6.2.9-1: Definition of type RgAuthenticationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| suci | Suci | M | 1 | Contains the SUCI of the FN-RG. |
| authenticatedInd | boolean | M | 1 | This IE shall be set as follows: - true: authenticated by the W-AGF; - false (default): unauthenticated by the W-AGF. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |

##### 6.1.6.2.10	Type: RgAuthCtx

Table 6.1.6.2.10-1: Definition of type RgAuthCtx

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authResult | AuthResult | M | 1 | Indicates the result of the authentication |
| supi | Supi | C | 0..1 | If the authentication is successful and if the AMF had provided a SUCI, this IE shall contain the SUPI of the UE. |
| authInd | boolean | C | 0..1 | When present, this IE shall be set as follows: - true: authentication is not required; - false (default): authentication is required. |

##### 6.1.6.2.11	Type: DeregistrationInfo

Table 6.1.6.2.11-1: Definition of type DeregistrationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | Contains the SUPI of the UE. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |

##### 6.1.6.2.12	Type: ProSeAuthenticationInfo

Table 6.1.6.2.12-1: Definition of type ProSeAuthenticationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supiOrSuci | SupiOrSuci | C | 0..1 | This IE shall be present if received from 5G ProSe Remote UE or the 5G ProSe End UE. When received, this IE shall contain the SUCI of the 5G ProSe Remote UE or the 5G ProSe End UE. |
| 5gPrukId | 5GPrukId | C | 0..1 | This IE shall be present if the CP-PRUK is received from 5G ProSe Remote UE or the 5G ProSe End UE. When present, this IE shall Indicate the CP-PRUK ID received from the 5G ProSe Remote UE or the 5G ProSe End UE. |
| relayServiceCode | RelayServiceCode | M | 1 | Indicates Relay Service Code. See 3GPP TS 29.571 [7] clause 5.4.2 |
| nonce1 | Nonce1 | M | 1 | Indicates Nonce_1. |
| servingNetworkName | ServingNetworkName | M | 1 | Serving network name of the UE-to-Network relay or the UE-to-UE relay |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |

##### 6.1.6.2.13	Type: ProSeAuthenticationCtx

Table 6.1.6.2.13-1: Definition of type ProSeAuthenticationCtx

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authType | AuthType | M | 1 | Indicates the authentication method used for this UE i.e. "EAP-AKA". |
| _links | map(LinksValueSchema) | M | 1..N | This IE shall contain a member whose name is set to "prose-auth" and the URI to perform the EAP session. See NOTE |
| proSeAuthData | ProSeAuthData | M | 1 | Contains ProSe Authentication related information |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |
| NOTE: In the current version of this API, only one hypermedia link is provided |  |  |  |  |

##### 6.1.6.2.14	Type: ProSeEapSession

Table 6.1.6.2.14-1: Definition of type ProSeEapSession

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eapPayload | EapPayload | M | 1 | Contains the EAP packet (see IETF RFC 3748 [18]). If no EAP packet has been provided by the 5G ProSe Remote UE or the 5G ProSe End UE the null value is conveyed to the AUSF. |
| knrProSe | KnrProSe | C | 0..1 | If the authentication is successful and no EAP packet has been provided by the 5G ProSe Remote UE or the 5G ProSe End UE, the KNR_ProSe shall be included |
| _links | map(LinksValueSchema) | C | 1..N | If the EAP session requires another exchange e.g. for EAP-AKA' notification, this IE shall contain a member whose name is "prose-auth" and the URI to continue the EAP session. See NOTE. |
| authResult | AuthResult | C | 0..1 | This IE shall be present if no EAP packet has been provided by the 5G ProSe Remote UE or the 5G ProSe End UE. When present, this IE shall indicate the result of the authentication. |
| nonce2 | Nonce2 | C | 0..1 | If the authentication is successful and no EAP packet has been provided by the 5G ProSe Remote UE or the 5G ProSe End UE, the Nonce_2 shall be included. |
| 5gPrukId | 5GPrukId | C | 0..1 | This IE shall be present if authentication is successful and no EAP packet has been provided by the 5G ProSe Remote UE or the 5G ProSe End UE. When present, this IE shall contain the CP-PRUK ID to the 5G ProSe Remote UE or the 5G ProSe End UE. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |
| NOTE: In the current version of this API, only 0 or 1 hypermedia link is provided. |  |  |  |  |

##### 6.1.6.2.15	Type: ProSeAuthData

Table 6.1.6.2.15-1: Definition of type ProSeAuthData

| Data type | Cardinality | Description |
| --- | --- | --- |
| EapPayload | 1 | Contains the EAP packet request. |

##### 6.1.6.2.16	Type: ProSeAuthenticationResult

Table 6.1.6.2.16-1: Definition of type ProSeAuthenticationResult

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| knrProSe | KnrProSe | C | 0..1 | If the authentication is successful, the KNR_ProSe shall be included |
| nonce2 | Nonce2 | C | 0..1 | If the authentication is successful, the Nonce_2 shall be included. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| EapPayload | string | The EAP packet is encoded using base64 (see IETF RFC 4648 [19]) and represented as a String. Format: byte |
| ResStar | string | pattern: "^[A-Fa-f0-9]{32}$"; nullable |
| Kseaf | string | pattern: "^[A-Fa-f0-9]{64}$" |
| HxresStar | string | pattern: "^[A-Fa-f0-9]{32}$" |
| Suci | string | String containing a SUCI. Pattern: "^(suci-(0-[0-9]{3}-[0-9]{2,3}¦[1-7]-.+)-[0-9]{1,4}-(0-0-.+¦[a-fA-F1-9]-([1-9]¦[1-9][0-9]¦1[0-9]{2}¦2[0-4][0-9]¦25[0-5])-[a-fA-F0-9]+)¦.+)$" |
| KnrProSe | string | String contain the KNR_ProSe pattern: "^[A-Fa-f0-9]{64}$" |
| Nonce1 | string | The Nonce1 is encoded using base64 (see IETF RFC 4648 [19]) and represented as a String. Format: byte |
| Nonce2 | string | The Nonce2 is encoded using base64 (see IETF RFC 4648 [19]) and represented as a String. Format: byte |
| Msk | string | pattern: "^[A-Fa-f0-9]{128}$" |

##### 6.1.6.3.3	Enumeration: AuthType

Table 6.1.6.3.3-1: Enumeration AuthType

| Enumeration value | Description |
| --- | --- |
| 5G_AKA | 5G AKA |
| EAP_AKA_PRIME | EAP-AKA' |
| EAP_TLS | EAP-TLS is only used in the case where the Annex B is supported. |
| EAP_TTLS | EAP-TTLS is used in the case where the Annex U of 3GPP TS 33.501 [8] is supported. |

##### 6.1.6.3.4	Enumeration:	AuthResult

Table 6.1.6.3.4-1: Enumeration AuthResult

| Enumeration value | Description |
| --- | --- |
| AUTHENTICATION_SUCCESS | This value is used to indicate that the AUSF successfully authenticate the UE |
| AUTHENTICATION_FAILURE | This value is used to indicate that the AUSF fails to authenticate the UE. |
| AUTHENTICATION_ONGOING | This value is used during an EAP Session to indicate that the EAP session is still ongoing. |

##### 6.1.6.3.5	Relation Types

6.1.6.3.5.1	General

This clause describes the possible relation types defined within AUSF API.

Table 6.1.6.3.5-1: supported registered relation types

Relation Name

5g-aka

eap-session

6.1.6.3.5.2	The "5g-aka" Link relation

The value "5g-aka" specifies that the value of the href attribute is the URI where NF Service Consumer shall send a PUT containing the result "RES*" received from the UE.

6.1.6.3.5.3	The "eap-session" Link relation

The value "eap-session" specifies that the value of the href attribute is the URI that will be used  by the NF  Service Consumer to provide EAP packet response during an EAP exchange. The NF Service Consumer shall use a POST to provide the EAP Packet Response to the AUSF to the corresponding URI.

#### 6.1.6.4	Binary data

##### 6.1.6.4.1	Introduction

There is no binary data in the current version of this API.

### 6.1.7	Error Handling

#### 6.1.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

The Cause codes mapping performed by AMF between the following HTTP responses returned by the AUSF services to the AMF and the 5GMM related values is specified in clause 4.2.2 of 3GPP TS 29.524 [24].

#### 6.1.7.2	Protocol Errors

Protocol errors shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.1.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nausf_UEAuthentication service. The following application errors listed in Table 6.1.7.3-1 are specific for the Nausf_UEAuthentication service.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| SERVING_NETWORK_NOT_AUTHORIZED | 403 Forbidden | The serving network is not authorized, e.g. serving PLMN. |
| AUTHENTICATION_REJECTED | 403 Forbidden | The user cannot be authenticated with this authentication method e.g. only SIM data available |
| INVALID_HN_PUBLIC_KEY_IDENTIFIER | 403 Forbidden | Invalid HN public key identifier received |
| INVALID_SCHEME_OUTPUT | 403 Forbidden | SUCI cannot be decrypted with received data |
| CONTEXT_NOT_FOUND | 404 Not Found | The AUSF cannot found the resource corresponding to the URI provided by the NF Service Consumer. |
| USER_NOT_FOUND | 404 Not Found | The user does not exist in the HPLMN |
| UPSTREAM_SERVER_ERROR | 504 Gateway Timeout | No response is received from a remote peer, e.g. from the UDM |
| NETWORK_FAILURE | 504 Gateway Timeout | The request is rejected due to a network problem. |
| AV_GENERATION_PROBLEM | 500 Internal Server Error | The UDM has indicated that it was not able to generate AV. |
| UNSUPPORTED_PROTECTION_SCHEME | 501 Not implemented | The received protection scheme is not supported by HPLMN |

### 6.1.8	Security

As indicated in 3GPP TS 33.501 [8], the access to the Nausf_UEAuthentication Service API may be authorized by means of the Oauth2 protocol (see IETF RFC 6749 [13]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [14]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming service offered by the Nausf_UEAuthentication Service API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [14], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nausf_UEAuthentication service.

The Nausf_UEAuthentication Service API defines the following scopes.

Table 6.1.8-1: OAuth2 scopes defined in Nausf_UEAuthentication API

| Scope | Description |
| --- | --- |
| "nausf-auth" | Access to the Nausf_UEAuthentication API. |
| "nausf-auth:ue-authentications" | Access to service operations applying to the collection of the ue-authentication resources and the subresources, i.e. 5g-aka-confirmation and eap-session. |
| "nausf-auth:rg-authentications" | Access to service operations applying to the collection of the rg-authentication resources. |
| "nausf-auth:prose-authentications" | Access to service operations applying to the collection of the prose-authentication resources and the subresources, i.e. prose-auth. |

### 6.1.9	Feature Negotiation

The features in table 6.1.9-1 are defined for the Nausf_UEAuthentication API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.9-1: Supported Features

| Feature number | Feature Name | M/O | Description |
| --- | --- | --- | --- |
| 1 | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. AMF) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the UEAuthentication service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |

### 6.1.10	HTTP redirection

An HTTP request may be redirected to a different AUSF service instance, within the same AUSF or a different AUSF of an AUSF set, e.g. when an AUSF service instance is part of an AUSF (service) set or when using indirect communications (see 3GPP TS 29.500 [4]). See also the ES3XX feature in clause 6.1.10.

An SCP that reselects a different AUSF producer instance will return the NF Instance ID of the new AUSF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an AUSF within an AUSF set redirects a service request to a different AUSF of the set using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new AUSF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

## 6.2	Nausf_SoRProtection Service API

### 6.2.1	API URI

URIs of this API shall have the following root:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "nausf-sorprotection".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.2.3.

### 6.2.2	Usage of HTTP

#### 6.2.2.1	General

HTTP/2, as defined in IETF RFC 9113 [6], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

#### 6.2.2.2	HTTP standard headers

##### 6.2.2.2.1	General

The usage of HTTP standard headers is specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.2.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [7], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 9475 [11]. The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json"

#### 6.2.2.3	HTTP custom headers

##### 6.2.2.3.1	General

In this version of the API, no specific custom headers are defined for the "Nausf_SoRProtection" service.

For 3GPP specific HTTP custom headers used across all service based interfaces, see clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.2.3	Resources

#### 6.2.3.1	Overview

The structure of the Resource URIs of the Nausf_SoRProtection service is shown in Figure 6.2.3.1-1

Figure 6.2.3.1-1: Resource URI structure of the SoRProtection API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| supi (Custom operation) | /{supi}/ue-sor/ | ue-sor(POST) | Resource for SoR security material computation |

#### 6.2.3.2	Resource: supi (Custom operation)

##### 6.2.3.2.1	Description

It is the resource to which the custom operation used to generate the SoR security material is associated with.

##### 6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/nausf-sorprotection/<apiVersion>/{supi}/

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| supi | Supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [10] |

##### 6.2.3.2.3	Resource Standard Methods

No Standard Methods are supported for this resource.

##### 6.2.3.2.4	Resource Custom Operations

6.2.3.2.4.1	Overview

Table 6.2.3.2.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| ue-sor | /ue-sor | POST | The AUSF calculates the SoR-MAC-IAUSF and the CounterSoR to protect the Steering Information List provided. It may also calculate the SoR-XMAC-IUE to verify that the UE received the Steering Information List if the indication that an acknowledgement is requested from the UE. |

6.2.3.2.4.2	Operation: ue-sor

6.2.3.2.4.2.1	Description

This custom operation is used by the NF service consumer (e.g. UDM) to request the AUSF to compute the security material (SoR-MAC-IAUSF, CounterSoR and SoR-XMAC-IUE) needed to ensure the protection of the SoR procedure (see 3GPP TS 33.501 [8]).

6.2.3.2.4.2.2	Operation Definition

This method shall support the request data structures specified in table 6.2.3.2.4.2.2-1 and the response data structures and response codes specified in table 6.2.3.2.4.2.2-2.

Table 6.2.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SorInfo | M | 1 | Contains the Steering Information List and shall contain the indication of whether an acknowledgement is requested from the UE or not (as specified in 3GPP TS 33.501 [8]). |

Table 6.2.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SorSecurityInfo | M | 1 | 200 OK | Upon success, the response body will contain SoR-MAC-IAUSF and CounterSoR and may contain the SoR-XMAC-IUE. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 503 Service Unavailable | The "cause" attribute may be used to indicate one of the following application errors: - COUNTER_WRAP See table 6.2.7.3-1 for the description of these errors. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.2.3.2.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.2.3.2.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

### 6.2.4	Custom Operations without associated resources

#### 6.2.4.1	Overview

There is no Custom Operation in the current version of this API.

### 6.2.5	Notifications

#### 6.2.5.1	General

There is no use of notification in the current version of this API.

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nausf_SORProtection service based interface protocol.

Table 6.2.6.1-1: Nausf_SORProtection specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SorInfo | 6.2.6.2.2 | Contains the Steering Information |
| SorSecurityInfo | 6.2.6.2.3 | Contains the material generated for securing of SoR. It contains at least the SoR-MAC-IAUSF and CounterSoR. |
| SteeringInfo | 6.2.6.2.4 | Contains a combination of one PLMN identity and zero or more access technologies. |
| SteeringContainer | 6.2.6.2.5 | Contains the information sent to UE. |
| SorMac | 6.2.6.3.2 | MAC value for protecting SOR procedure (SoR-MAC-IAUSF and SoR-XMAC-IUE) |
| CounterSor | 6.2.6.3.2 | CounterSoR |
| AckInd | 6.2.6.3.2 | Contains indication whether the acknowledgement from UE is needed |
| SecuredPacket | 6.2.6.3.2 | Contains a secure packet. |
| AccessTech | 6.2.6.3.3 | Access Technology |
| SorHeader | 6.2.6.3.2 | Contains the SoR Header. |
| SorTransparentInfo | 6.2.6.3.2 | Contains steering information encoded as transparent containers. |

Table 6.2.6.1-2 specifies data types re-used by the Nausf_SORProtection service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nausf_SORProtection service based interface.

Table 6.2.6.1-2: Nausf_SORProtection re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| PlmnId | 3GPP TS 29.571 [10] | PLMN ID |
| SupportedFeatures | 3GPP TS 29.571 [10] | Supported Features |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

The following clauses define the structures to be used in resource representations.

##### 6.2.6.2.2	Type: SorInfo

Table 6.2.6.2.2-1: Definition of type SorInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ackInd | AckInd | M | 1 | Contains the indication whether the acknowledgement from UE is needed. |
| steeringContainer | SteeringContainer | C | 0..1 | When present, this information contains the information needed to update the "Operator Controlled PLMN Selector with Access Technology" list stored in the USIM. It may contain an array of preferred PLMN/AccessTechnologies combinations in priority order. The first entry in the array indicates the highest priority and the last entry indicates the lowest. Or it may contain a secured packet. If no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the USIM is needed then this attribute shall be absent. |
| sorHeader | SorHeader | O | 0..1 | This attribute contains SoR Header encoded as defined in clause 6.2.6.3.2 and shall be present if AUSF supports receiving SoR Information encoded as transparent containers. |
| sorTransparentInfo | SorTransparentInfo | O | 0..1 | This attribute contains steering information encoded as defined in clause 6.2.6.3.2, and may be present if AUSF supports receiving SoR Information encoded as transparent containers. It may be absent if no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the USIM is needed. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.2.9 is supported. |

##### 6.2.6.2.3	Type: SorSecurityInfo

Table 6.2.6.2.3-1: Definition of type SorSecurityInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sorMacIausf | SorMac | M | 1 | Contains the SoR-MAC-IAUSF. |
| counterSor | CounterSor | M | 1 | Contains the CounterSoR. |
| sorXmacIue | SorMac | O | 0..1 | When present, contains the SoR-XMAC-IUE. It shall be included, if the UDM requests the acknowledgement from the UE. |

##### 6.2.6.2.4	Type: SteeringInfo

Table 6.2.6.2.4-1: Definition of type SteeringInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | Contains a preferred PLMN identity. |
| accessTechList | array(AccessTech) | C | 1..N | When present it contains the referred access technologies as listed in clause 4.2.5 of 3GPP TS 31.102 [15]. If absent it means that all access technologies are equivalently preferred in this PLMN. |

##### 6.2.6.2.5	Type: SteeringContainer

Table 6.2.6.2.5.-1: Definition of type SteeringContainer as a list of mutually exclusive alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| array(SteeringInfo) | 1..N | List of PLMN/AccessTechnologies combinations. |
| SecuredPacket | 1 | A secured packet containing one or more APDUs commands dedicated to Remote File Management. |

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Simple data types

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| SorMac | string | pattern: "^[A-Fa-f0-9]{32}$" |
| CounterSor | string | pattern: "^[A-Fa-f0-9]{4}$" |
| AckInd | boolean | true indicates that the SoR-XMAC-IUE shall be computed and returned in the response |
| SecuredPacket | string | Contains a secure packet as specified in 3GPP TS 24.501 [20]. It is encoded using base64 and represented as a String. Format: byte |
| SorHeader | Bytes | String with format "byte" as defined in OpenAPI Specification [25], i.e. base64-encoded characters, encoding the "SOR Header" IE as specified in clause 9.11.3.51 of 3GPP TS 24.501 [20] (octet 4). |
| SorTransparentInfo | Bytes | String with format "byte" as defined in OpenAPI Specification [25], i.e. base64-encoded characters, encoding the "SOR transparent container" IE as specified in clause 9.11.3.51 of 3GPP TS 24.501 [20] (starting from octet 23). |

##### 6.2.6.3.3	Enumeration: AccessTech

Table 6.2.6.3.3-1: Enumeration AccessTech

| Enumeration value | Description |
| --- | --- |
| "NR" |  |
| "EUTRAN_IN_WBS1_MODE_AND_NBS1_MODE" |  |
| "EUTRAN_IN_NBS1_MODE_ONLY" |  |
| "EUTRAN_IN_WBS1_MODE_ONLY" |  |
| "UTRAN" |  |
| "GSM_AND_ECGSM_IoT" |  |
| "GSM_WITHOUT_ECGSM_IoT" |  |
| "ECGSM_IoT_ONLY" |  |
| "CDMA_1xRTT" |  |
| "CDMA_HRPD" |  |
| "GSM_COMPACT" |  |

NOTE:	Some of the values in the enumeration deviate from the naming conventions indicated in 3GPP TS 29.501 [5] Clause 5.1.4 (i.e., to use UPPER_WITH_UNDERSCORE); 
however, it is kept as currently defined in this specification to maintain backwards compatibility.

### 6.2.7	Error Handling

#### 6.2.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.2.7.2	Protocol Errors

Protocol Error Handling shall be supported as specified in clause 5.2.7.2 of 3GPP TS 29.500 [4].

#### 6.2.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nausf_SoRProtection service. The following application errors listed in Table 6.2.7.3-1 are specific for the Nausf_SoRProtection service.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| COUNTER_WRAP | 503 Service Unavailable | The CounterSoR associated with the KAUSF of the UE is about to wrap around. The AUSF suspends the SoR protection service for the UE until a new KAUSF is generated. |

### 6.2.8	Security

As indicated in 3GPP TS 33.501 [8], the access to the Nausf_SoRProtection API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [13]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [14]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nausf_SoRProtection API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [14], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nausf_SoRProtection service.

The Nausf_SoRProtection Service API defines a single scope nausf-sorprotection (as specified in 3GPP TS 33.501 [8]), and it does not define any additional scopes at resource or operation level.

### 6.2.9	Feature Negotiation

The features in table 6.2.9-1 are defined for the Nausf_SoRProtection API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.9-1: Supported Features

| Feature number | Feature Name | M/O | Description |
| --- | --- | --- | --- |
| 1 | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. UDM) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the SoRProtection service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| 2 | sorTransparentSupport | O | This flag is used by AUSF to register (in NRF) its support of receiving SoR Transparent Information instead of individual IEs from UDM. |

### 6.2.10	HTTP redirection

An HTTP request may be redirected to a different AUSF service instance, within the same AUSF or a different AUSF of an AUSF set, e.g. when an AUSF service instance is part of an AUSF (service) set or when using indirect communications (see 3GPP TS 29.500 [4]). See also the ES3XX feature in clause 6.2.9.

An SCP that reselects a different AUSF producer instance will return the NF Instance ID of the new AUSF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an AUSF within an AUSF set redirects a service request to a different AUSF of the set using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new AUSF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

## 6.3	Nausf_UPUProtection Service API

### 6.3.1	API URI

URIs of this API shall have the following root:

{apiRoot}/{apiName}/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "nausf-upuprotection".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.3.3.

### 6.3.2	Usage of HTTP

#### 6.3.2.1	General

HTTP/2, as defined in IETF RFC 7540 [6], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

#### 6.3.2.2	HTTP standard headers

##### 6.3.2.2.1	General

The usage of HTTP standard headers is specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.3.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [7], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 9457 [11]. The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json"

#### 6.3.2.3	HTTP custom headers

##### 6.3.2.3.1	General

In this version of the API, no specific custom headers are defined for the "Nausf_UPUProtection" service.

For 3GPP specific HTTP custom headers used across all service based interfaces, see clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.3.3	Resources

#### 6.3.3.1	Overview

The structure of the Resource URIs of the Nausf_UPUProtection service is shown in Figure 6.3.3.1-1

Figure 6.3.3.1-1: Resource URI structure of the UPUProtection API

Table 6.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.3.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| supi (Custom operation) | /{supi}/ue-upu/ | ue-upu (POST) | Resource for UPU security material computation |

#### 6.3.3.2	Resource: supi (Custom operation)

##### 6.3.3.2.1	Description

It is the resource to which the custom operation used to generate the UPU security material is associated with.

##### 6.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/nausf-upuprotection/<apiVersion>/{supi}/

This resource shall support the resource URI variables defined in table 6.3.3.2.2-1.

Table 6.3.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.3.1 |
| supi | Supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [10] |

##### 6.3.3.2.3	Resource Standard Methods

No Standard Methods are supported for this resource.

##### 6.3.3.2.4	Resource Custom Operations

6.3.3.2.4.1	Overview

Table 6.3.3.2.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| ue-upu | /ue-upu | POST | The AUSF calculates the UPU-MAC-IAUSF and the CounterUPU to protect the UE Parameters Update Data provided. It may also calculate the UPU-XMAC-IUE to verify that the UE received the UE Parameters Update Data if the indication that an acknowledgement is requested from the UE is provided. |

6.3.3.2.4.2	Operation: ue-upu

6.3.3.2.4.2.1	Description

This custom operation is used by the NF service consumer (e.g. UDM) to request the AUSF to compute the security material (UPU-MAC-IAUSF, CounterUPU and UPU-XMAC-IUE) needed to ensure the protection of the UPU procedure (see 3GPP TS 33.501 [8]).

6.3.3.2.4.2.2	Operation Definition

This method shall support the request data structures specified in table 6.3.3.2.4.2.2-1 and the response data structures and response codes specified in table 6.3.3.2.4.2.2-2.

Table 6.3.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UpuInfo | M | 1 | Contains the UE Parameters Update Data and shall contain the indication of whether an acknowledgement is requested from the UE or not (as specified in 3GPP TS 33.501 [8]). |

Table 6.3.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UpuSecurityInfo | M | 1 | 200 OK | Upon success, the response body will contain UPU-MAC-IAUSF and CounterUPU and may contain the UPU-XMAC-IUE. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 503 Service Unavailable | The "cause" attribute may be used to indicate one of the following application errors: - COUNTER_WRAP See table 6.3.7.3-1 for the description of these errors. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.3.3.2.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.3.3.2.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same AUSF or AUSF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

### 6.3.4	Custom Operations without associated resources

#### 6.3.4.1	Overview

There is no Custom Operation in the current version of this API.

### 6.3.5	Notifications

#### 6.3.5.1	General

There is no use of notification in the current version of this API.

### 6.3.6	Data Model

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the Nausf_UPUProtection service based interface protocol.

Table 6.3.6.1-1: Nausf_UPUProtection specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| UpuInfo | 6.3.6.2.2 | Contains the UE parameters update Information |
| UpuSecurityInfo | 6.3.6.2.3 | Contains the material generated for securing of UPU. It contains at least the UPU-MAC-IAUSF and CounterUPU. |
| UpuData | 6.3.6.2.4 | Contains UE parameters update data set (e.g., the updated Routing ID Data or the Default configured NSSAI). |
| UpuMac | 6.3.6.3.2 | MAC value for protecting UPU procedure (UPU-MAC-IAUSF and UPU-MAC-IUE) |
| CounterUpu | 6.3.6.3.2 | CounterUPU |
| UpuAckInd | 6.3.6.3.2 | Contains the indication of whether the acknowledgement from UE is needed |
| UpuHeader | 6.3.6.3.2 | Contains the "UPU Header" IE as specified in clause 9.11.3.53A of 3GPP TS 24.501 [20] (octet 4), |
| UpuTransparentInfo | 6.3.6.3.2 | Contains UPU information encoded as transparent container. |

Table 6.3.6.1-2 specifies data types re-used by the Nausf_UPUProtection service based interface protocol from other specifications and from other service APIs in current specification, including a reference to the respective specifications and when needed, a short description of their use within the Nausf_UPUProtection service based interface.

Table 6.3.6.1-2: Nausf_UPUProtection re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Snssai | 3GPP TS 29.571 [10] | Default configured NSSAI |
| SecuredPacket | 6.2.6.3.2 | Defined in Nausf_SORProtection API. Secured Packet |
| RoutingId | 3GPP TS 29.544 [22] | Routing ID |
| SupportedFeatures | 3GPP TS 29.571 [10] | Supported Features |

#### 6.3.6.2	Structured data types

##### 6.3.6.2.1	Introduction

The following clauses define the structures to be used in resource representations.

##### 6.3.6.2.2	Type: UpuInfo

Table 6.3.6.2.2-1: Definition of type UpuInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upuDataList | array(UpuData) | M | 1..N | This information defines the UE Parameters Update (UPU). A secured packed with the Routing indicator update data and/or the Default configured NSSAI update data are included. See clause 6.3.6.2.4. This attribute shall be ignored by AUSF when upuTransparentInfo is present. |  |
| upuHeader | UpuHeader | O | 0..1 | This attribute contains UPU Header encoded as defined in clause 6.3.6.3.2. |  |
| upuAckInd | UpuAckInd | M | 1 | Contains the indication of whether the acknowledgement from UE is needed. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.3.9 is supported. |  |
| upuTransparentInfo | UpuTransparentInfo | O | 0..1 | This attribute contains UPU information encoded as defined in clause 6.3.6.3.2, and may be present if AUSF supports receiving UPU Information encoded as transparent container. | upuTransparentSupport |

##### 6.3.6.2.3	Type: UpuSecurityInfo

Table 6.3.6.2.3-1: Definition of type UpuSecurityInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upuMacIausf | UpuMac | M | 1 | Contains the UPU-MAC-IAUSF. |
| counterUpu | CounterUpu | M | 1 | Contains the CounterUPU. |
| upuXmacIue | UpuMac | O | 0..1 | When present, contains the UPU-XMAC-IUE. It shall be included, if the UDM requests the acknowledgement from the UE. |

##### 6.3.6.2.4	Type: UpuData

Table 6.3.6.2.4-1: Definition of type UpuData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| secPacket | SecuredPacket | C | 0..1 | Presents if the Routing indicator update data is required to be updated, and contains a secured packet with the Routing indicator to be updated. |
| defaultConfNssai | array(Snssai) | C | 1..N | Presents if the Default configured NSSAI is required to be updated, and contains the Default configured NSSAI to be updated. |
| RoutingId | RoutingId | C | 0..1 | May be present when sent from UDR to UDM. The UDM shall make use of Nspaf services (see 3GPP TS 29.544 [22] to encapsulate the routing id in a secured packet which is then conveyed to the AUSF and AMF. |
| drei | boolean | C | 0..1 | Disaster Roaming Enabled Indication (DREI). May be present when sent from UDR to UDM. - false: Disaster roaming is disabled in the UE - true: Disaster roaming is enabled in the UE |
| aol | boolean | C | 0..1 | Indication of 'applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN'. May be present when sent from UDR to UDM. - false: the list of PLMNs provided by the VPLMN is not applicable - true: the list of PLMNs provided by the VPLMN is applicable |
| disasterCondPlmnIds | array(PlmnId) | C | 1..N | List of PLMN Ids identifying PLMNs allowed to be used in Disaster Conditions. May be present when sent from UDR to UDM. |

#### 6.3.6.3	Simple data types and enumerations

##### 6.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.3.6.3.2	Simple data types

Table 6.3.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| UpuMac | string | pattern: "^[A-Fa-f0-9]{32}$" |
| CounterUpu | string | pattern: "^[A-Fa-f0-9]{4}$" |
| UpuAckInd | boolean | true indicates that the UPU-XMAC-IUE shall be computed and returned in the response |
| UpuHeader | string | It contains the "UPU Header" IE as specified in clause 9.11.3.53A of 3GPP TS 24.501 [20] (octet 4), encoded as 2 hexadecimal characters. Pattern: "^[A-Fa-f0-9]{2}$" |
| UpuTransparentInfo | Bytes | String with format "byte" as defined in OpenAPI Specification [25], i.e. base64-encoded characters, encoding the "UPU transparent container" IE as specified in clause 9.11.3.53A of 3GPP TS 24.501 [20] (starting from octet 23). |

##### 6.3.6.3.3	Void

### 6.3.7	Error Handling

#### 6.3.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.3.7.2	Protocol Errors

Protocol Error Handling shall be supported as specified in clause 5.2.7.2 of 3GPP TS 29.500 [4].

#### 6.3.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nausf_UPUProtection service. The following application errors listed in Table 6.3.7.3-1 are specific for the Nausf_UPUProtection service.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| COUNTER_WRAP | 503 Service Unavailable | The CounterUPU associated with the KAUSF of the UE is about to wrap around. The AUSF suspends the UPU protection service for the UE until a new KAUSF is generated. |

### 6.3.8	Security

As indicated in 3GPP TS 33.501 [8], the access to the Nausf_UPUProtection API may be authorized by means of the Oauth2 protocol (see IETF RFC 6749 [13]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [14]) plays the role of the authorization server.

If Oauth2 is used, an NF Service Consumer, prior to consuming services offered by the Nausf_UPUProtection API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [14], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nausf_UPUProtection service.

The Nausf_UPUProtection Service API does not define any scopes for Oauth2 authorization as specified in 3GPP TS 33.501 [8]; it defines a single scope consisting on the name of the service (i.e., "nausf-upuprotection"), and it does not define any additional scopes at resource or operation level.

### 6.3.9	Feature Negotiation

The features in table 6.3.9-1 are defined for the Nausf_UPUProtection API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.3.9-1: Supported Features

| Feature number | Feature Name | M/O | Description |
| --- | --- | --- | --- |
| 1 | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. UDM) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the UPUProtection service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| 2 | upuTransparentSupport | O | This flag is used by AUSF to register (in NRF) its support of receiving UPU Transparent Information instead of individual Ies from UDM |

### 6.3.10	HTTP redirection

An HTTP request may be redirected to a different AUSF service instance, within the same AUSF or a different AUSF of an AUSF set, e.g. when an AUSF service instance is part of an AUSF (service) set or when using indirect communications (see 3GPP TS 29.500 [4]). See also the ES3XX feature in clause 6.3.9.

An SCP that reselects a different AUSF producer instance will return the NF Instance ID of the new AUSF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an AUSF within an AUSF set redirects a service request to a different AUSF of the set using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new AUSF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the Nausf Service API(s). It consists of OpenAPI 3.0.0 specifications in YAML format.

NOTE 1:	OpenAPI 3.0 does not support description of API using HATEOAS. Indeed, only relative paths can be used and as a consequence the URI provided in the "href" cannot be reused as it is.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [21] clause 5B).

# A.2	Nausf_UEAuthentication API

openapi: 3.0.0

info:

version: 1.3.0

title: AUSF API

description: |

AUSF UE Authentication Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.509 V18.5.0; 5G System; 3GPP TS Authentication Server services.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.509'

servers:

- url: '{apiRoot}/nausf-auth/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

paths:

/ue-authentications:

post:

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:ue-authentications

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AuthenticationInfo'

required: true

responses:

'201':

description: UEAuthenticationCtx

content:

application/3gppHal+json:

schema:

$ref: '#/components/schemas/UEAuthenticationCtx'

headers:

Location:

description: >

'Contains the URI of the newly created resource according to the structure:

{apiRoot}/nausf-auth/<apiVersion>/ue-authentications/{authCtxId}'

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

description: Bad Request from the AMF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: Forbidden due to serving network not authorized

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: User does not exist in the HPLMN

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'501':

description: Received protection scheme is not supported by HPLMN

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/ue-authentications/deregister:

post:

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:ue-authentications

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/DeregistrationInfo'

required: true

responses:

'204':

description: Expected response to a successful removal of security context

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/ue-authentications/{authCtxId}/5g-aka-confirmation:

put:

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:ue-authentications

parameters:

- name: authCtxId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ConfirmationData'

responses:

'200':

description: Request processed (EAP success or Failure)

content:

application/json:

schema:

$ref: '#/components/schemas/ConfirmationDataResponse'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

description: Bad Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

delete:

summary: Deletes the authentication result in the UDM

operationId: Delete5gAkaAuthenticationResult

tags:

- Authentication Result Deletion

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:ue-authentications

parameters:

- name: authCtxId

in: path

required: true

schema:

type: string

responses:

'204':

description: Expected response to a successful authentication result removal

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/ue-authentications/{authCtxId}/eap-session:

post:

operationId: EapAuthMethod

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:ue-authentications

parameters:

- name: authCtxId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/EapSession'

responses:

'200':

description: Use to handle or close the EAP session

content:

application/json:

schema:

$ref: '#/components/schemas/EapSession'

application/3gppHal+json:

schema:

type: object

properties:

eapPayload:

$ref: '#/components/schemas/EapPayload'

_links:

type: object

description: >

A map (list of key-value pairs) where the key is set to "eap-session"

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

minProperties: 1

required:

- eapPayload

- _links

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

description: Bad Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

delete:

summary: Deletes the authentication result in the UDM

operationId: DeleteEapAuthenticationResult

tags:

- Authentication Result Deletion

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:ue-authentications

parameters:

- name: authCtxId

in: path

required: true

schema:

type: string

responses:

'204':

description: Expected response to a successful authentication result removal

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/rg-authentications:

post:

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:rg-authentications

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/RgAuthenticationInfo'

required: true

responses:

'201':

description: RgAuthCtx

content:

application/json:

schema:

$ref: '#/components/schemas/RgAuthCtx'

headers:

Location:

description: >

'Contains the URI of the newly created resource according to the structure:

{apiRoot}/nausf-auth/<apiVersion>/rg-authentications/{authCtxId}'

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

description: Bad Request from the AMF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: The UE is not allowed to be authenticated

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: User does not exist in the HPLMN

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/prose-authentications:

post:

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:prose-authentications

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ProSeAuthenticationInfo'

required: true

responses:

'200':

description: Successful authentication with CP-PRUK ID

content:

application/json:

schema:

$ref: '#/components/schemas/ProSeAuthenticationResult'

'201':

description: ProSeAuthenticationCtx

content:

application/3gppHal+json:

schema:

$ref: '#/components/schemas/ProSeAuthenticationCtx'

headers:

Location:

description: >

'Contains the URI of the newly created resource according to the structure:

{apiRoot}/nausf-auth/<apiVersion>/prose-authentications/{authCtxId}'

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/prose-authentications/{authCtxId}/prose-auth:

post:

operationId: proseAuth

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:prose-authentications

parameters:

- name: authCtxId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ProSeEapSession'

responses:

'200':

description: >

Use to handle or close the EAP session for 5G ProSe Remote UE or a 5G ProSe End UE

content:

application/json:

schema:

$ref: '#/components/schemas/ProSeEapSession'

application/3gppHal+json:

schema:

type: object

properties:

eapPayload:

$ref: '#/components/schemas/EapPayload'

_links:

type: object

description: >

A map (list of key-value pairs) where the key is set to

"prose-auth"

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

minProperties: 1

required:

- eapPayload

- _links

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

delete:

summary: Deletes the authentication result in the UDM

operationId: DeleteProSeAuthenticationResult

tags:

- Authentication Result Deletion

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

- oAuth2ClientCredentials:

- nausf-auth

- nausf-auth:prose-authentications

parameters:

- name: authCtxId

in: path

required: true

schema:

type: string

responses:

'204':

description: Expected response to a successful authentication result removal

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nausf-auth: Access to Nausf_UEAuthentication API

nausf-auth:ue-authentications: >

Access to service operations applying to the collection of the ue-authentication

resources and the subresources, i.e. 5g-aka-confirmation and eap-session

nausf-auth:rg-authentications: >

Access to service operations applying to the collection of the rg-authentication

resources

nausf-auth:prose-authentications: >

Access to service operations applying to the collection of the prose-authentication

resources and the subresources, i.e. prose-auth.

schemas:

AuthenticationInfo:

description: Contains the UE id (i.e. SUCI or SUPI) and the Serving Network Name.

type: object

properties:

supiOrSuci:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupiOrSuci'

servingNetworkName:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

resynchronizationInfo:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ResynchronizationInfo'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

pattern: '^[0-9]{1,4}$'

cellCagInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CagId'

minItems: 1

n5gcInd:

type: boolean

default: false

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

nswoInd:

type: boolean

default: false

disasterRoamingInd:

type: boolean

default: false

onboardingInd:

type: boolean

default: false

aun3Ind:

type: boolean

default: false

required:

- supiOrSuci

- servingNetworkName

UEAuthenticationCtx:

description: >

Contains the information related to the resource generated to handle

the UE authentication. It contains at least the UE id, Serving Network,

the Authentication Method and related EAP information or related 5G-AKA information.

type: object

properties:

authType:

$ref: '#/components/schemas/AuthType'

5gAuthData:

oneOf:

- $ref: '#/components/schemas/Av5gAka'

- $ref: '#/components/schemas/EapPayload'

_links:

type: object

description: >

A map(list of key-value pairs) where the key is set to "eap-session" or "5g-aka"

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

servingNetworkName:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

required:

- authType

- 5gAuthData

- _links

Av5gAka:

description: Contains Authentication Vector for method 5G AKA.

type: object

required:

- rand

- hxresStar

- autn

properties:

rand:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

hxresStar:

$ref: '#/components/schemas/HxresStar'

autn:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Autn'

ConfirmationData:

description: Contains the result of the authentication.

type: object

required:

- resStar

properties:

resStar:

$ref: '#/components/schemas/ResStar'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ConfirmationDataResponse:

description: Contains the result of the authentication

type: object

properties:

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

kseaf:

$ref: '#/components/schemas/Kseaf'

pvsInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServerAddressingInfo'

minItems: 1

required:

- authResult

EapSession:

description: Contains information related to the EAP session.

type: object

properties:

eapPayload:

$ref: '#/components/schemas/EapPayload'

kSeaf:

$ref: '#/components/schemas/Kseaf'

_links:

type: object

description: A map(list of key-value pairs) where the key is set to "eap-session"

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pvsInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServerAddressingInfo'

minItems: 1

msk:

$ref: '#/components/schemas/Msk'

required:

- eapPayload

DeregistrationInfo:

description: Contains the UE id (i.e. SUPI).

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supi

RgAuthenticationInfo:

description: Contains the UE id (i.e. SUCI) and the authenticated indication.

type: object

properties:

suci:

$ref: '#/components/schemas/Suci'

authenticatedInd:

type: boolean

default: false

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- suci

- authenticatedInd

RgAuthCtx:

description: Contains the UE id (i.e. SUPI) and the authentication indication.

type: object

properties:

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

authInd:

type: boolean

default: false

required:

- authResult

AuthResult:

description: Indicates the result of the authentication.

type: string

enum:

- AUTHENTICATION_SUCCESS

- AUTHENTICATION_FAILURE

- AUTHENTICATION_ONGOING

EapPayload:

type: string

format: byte

description: contains an EAP packet

nullable: true

Kseaf:

description: Contains the Kseaf.

type: string

pattern: '[A-Fa-f0-9]{64}'

Msk:

description: Contains the Master Session Key.

type: string

pattern: '[A-Fa-f0-9]{128}'

ResStar:

description: Contains the RES*.

type: string

pattern: '[A-Fa-f0-9]{32}'

nullable: true

HxresStar:

description: Contains the HXRES*.

type: string

pattern: "[A-Fa-f0-9]{32}"

Suci:

description: Contains the SUCI.

type: string

pattern: '^(suci-(0-[0-9]{3}-[0-9]{2,3}|[1-7]-.+)-[0-9]{1,4}-(0-0-.+|[a-fA-F1-9]-([1-9]|[1-9][0-9]|1[0-9]{2}|2[0-4][0-9]|25[0-5])-[a-fA-F0-9]+)|.+)$'

AuthType:

description: Indicates the authentication method used.

anyOf:

- type: string

enum:

- 5G_AKA

- EAP_AKA_PRIME

- EAP_TLS

- EAP_TTLS

- type: string

ProSeAuthenticationInfo:

description: >

Contains the UE id (i.e. SUCI) or CP-PRUK ID, Relay Service Code and Nonce_1.

type: object

properties:

supiOrSuci:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupiOrSuci'

5gPrukId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GPrukId'

relayServiceCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RelayServiceCode'

nonce1:

$ref: '#/components/schemas/Nonce1'

servingNetworkName:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- relayServiceCode

- nonce1

- servingNetworkName

ProSeAuthenticationCtx:

description: >

Contains the information related to the resource generated to handle

the ProSe authentication.

type: object

properties:

authType:

$ref: '#/components/schemas/AuthType'

_links:

type: object

description: A map(list of key-value pairs) where the key is set to "prose-auth"

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

proSeAuthData:

$ref: '#/components/schemas/ProSeAuthData'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- authType

- _links

- proSeAuthData

ProSeEapSession:

description: >

Contains information related to the EAP session. If present the 5gPrukId IE shall

carry the CP-PRUK ID.

type: object

properties:

eapPayload:

$ref: '#/components/schemas/EapPayload'

knrProSe:

$ref: '#/components/schemas/KnrProSe'

_links:

type: object

description: A map(list of key-value pairs) where the key is set to "prose-auth"

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

authResult:

$ref: '#/components/schemas/AuthResult'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

nonce2:

$ref: '#/components/schemas/Nonce2'

5gPrukId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GPrukId'

required:

- eapPayload

ProSeAuthData:

$ref: '#/components/schemas/EapPayload'

ProSeAuthenticationResult:

description: Successful authentication for CP-PRUK ID.

type: object

properties:

knrProSe:

$ref: '#/components/schemas/KnrProSe'

nonce2:

$ref: '#/components/schemas/Nonce2'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

KnrProSe:

description: Contains the KNR_ProSe.

type: string

pattern: '[A-Fa-f0-9]{64}'

Nonce1:

type: string

format: byte

description: contains an Nonce1

nullable: true

Nonce2:

type: string

format: byte

description: contains an Nonce2

nullable: true

# A.3	Nausf_SoRProtection API

openapi: 3.0.0

info:

version: 1.3.0

title: Nausf_SoRProtection Service

description: |

AUSF SoR Protection Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.509 V18.5.0; 5G System; Authentication Server Services

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.509'

servers:

- url: '{apiRoot}/nausf-sorprotection/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nausf-sorprotection

paths:

/{supi}/ue-sor:

post:

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SorInfo'

required: true

responses:

'200':

description: SorSecurityInfo

content:

application/json:

schema:

$ref: '#/components/schemas/SorSecurityInfo'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nausf-sorprotection: Access to the Nausf_SoRProtection API

schemas:

#

# COMPLEX TYPES:

#

SorInfo:

description: Contains the Steering Information.

type: object

properties:

steeringContainer:

$ref: '#/components/schemas/SteeringContainer'

ackInd:

$ref: '#/components/schemas/AckInd'

sorHeader:

$ref: '#/components/schemas/SorHeader'

sorTransparentInfo:

$ref: '#/components/schemas/SorTransparentInfo'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ackInd

SorSecurityInfo:

description: >

Contains the material generated for securing of SoR. It contains at least the

SoR-MAC-IAUSF and CounterSoR.

type: object

properties:

sorMacIausf:

$ref: '#/components/schemas/SorMac'

counterSor:

$ref: '#/components/schemas/CounterSor'

sorXmacIue:

$ref: '#/components/schemas/SorMac'

required:

- sorMacIausf

- counterSor

SteeringContainer:

description: Contains the information sent to UE.

oneOf:

- type: array

items:

$ref: '#/components/schemas/SteeringInfo'

minItems: 1

- $ref: '#/components/schemas/SecuredPacket'

SteeringInfo:

description: Contains a combination of one PLMN identity and zero or more access technologies.

type: object

properties:

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

accessTechList:

type: array

items:

$ref: '#/components/schemas/AccessTech'

minItems: 1

required:

- plmnId

#

# SIMPLE TYPES:

#

SorMac:

description: MAC value for protecting SOR procedure (SoR-MAC-IAUSF and SoR-XMAC-IUE).

type: string

pattern: '^[A-Fa-f0-9]{32}$'

CounterSor:

description: CounterSoR.

type: string

pattern: '^[A-Fa-f0-9]{4}$'

AckInd:

description: Contains indication whether the acknowledgement from UE is needed.

type: boolean

SecuredPacket:

description: Contains a secure packet.

type: string

format: byte

SorHeader:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

SorTransparentInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

#

# ENUMS:

AccessTech:

description: Represents the access technology

anyOf:

- type: string

enum:

- NR

- EUTRAN_IN_WBS1_MODE_AND_NBS1_MODE

- EUTRAN_IN_NBS1_MODE_ONLY

- EUTRAN_IN_WBS1_MODE_ONLY

- UTRAN

- GSM_AND_ECGSM_IoT

- GSM_WITHOUT_ECGSM_IoT

- ECGSM_IoT_ONLY

- CDMA_1xRTT

- CDMA_HRPD

- GSM_COMPACT

- type: string

# A.4	Nausf_UPUProtection API

openapi: 3.0.0

info:

version: 1.3.0

title: Nausf_UPUProtection Service

description: |

AUSF UPU Protection Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.509 V18.5.0; 5G System; Authentication Server Services

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.509'

servers:

- url: '{apiRoot}/nausf-upuprotection/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nausf-upuprotection

paths:

/{supi}/ue-upu:

post:

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UpuInfo'

required: true

responses:

'200':

description: UpuSecurityInfo

content:

application/json:

schema:

$ref: '#/components/schemas/UpuSecurityInfo'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nausf-upuprotection: Access to the Nausf_UPUProtection API

schemas:

#

# COMPLEX TYPES:

#

UpuInfo:

description: Contains the UE parameters update Information.

type: object

properties:

upuDataList:

type: array

items:

$ref: '#/components/schemas/UpuData'

minItems: 1

upuHeader:

$ref: '#/components/schemas/UpuHeader'

upuAckInd:

$ref: '#/components/schemas/UpuAckInd'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

upuTransparentInfo:

$ref: '#/components/schemas/UpuTransparentInfo'

required:

- upuDataList

- upuAckInd

UpuSecurityInfo:

description: >

Contains the material generated for securing of UPU. It contains at least

the UPU-MAC-IAUSF and CounterUPU.

type: object

properties:

upuMacIausf:

$ref: '#/components/schemas/UpuMac'

counterUpu:

$ref: '#/components/schemas/CounterUpu'

upuXmacIue:

$ref: '#/components/schemas/UpuMac'

required:

- upuMacIausf

- counterUpu

UpuData:

description: >

Contains UE parameters update data set (e.g., the updated Routing ID Data

or the Default configured NSSAI or the disaster roaming parameters).

type: object

properties:

secPacket:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SecuredPacket'

defaultConfNssai:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

routingId:

$ref: 'TS29544_Nspaf_SecuredPacket.yaml#/components/schemas/RoutingId'

drei:

type: boolean

aol:

type: boolean

disasterCondPlmnIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

#

# SIMPLE TYPES:

#

UpuMac:

description: MAC value for protecting UPU procedure (UPU-MAC-IAUSF and UPU-MAC-IUE).

type: string

pattern: '^[A-Fa-f0-9]{32}$'

CounterUpu:

description: CounterUPU.

type: string

pattern: '^[A-Fa-f0-9]{4}$'

UpuAckInd:

description: Contains the indication of whether the acknowledgement from UE is needed.

type: boolean

UpuHeader:

description: >

Contains the "UPU Header" IE as specified in clause 9.11.3.53A of

3GPP TS 24.501 (octet 4), encoded as 2 hexadecimal characters.

type: string

pattern: '^[A-Fa-f0-9]{2}$'

UpuTransparentInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

Annex B (Informative):
Use of EAP-TLS

# B.1	General

The Annex B of 3GPP TS 33.501 [8] describes the use of EAP-TLS as an alternative authentication method in the case of private network. This annex describes corresponding stage 3.

# B.2	EAP method: EAP-TLS

EAP-TLS as defined in IETF RFC 5216 [16] is the EAP method used in this procedure. This procedure is described in Annex B.2.1 of 3GPP TS 33.501 [8].

Figure B.2-1: EAP method

1.	The NF Service Consumer (AMF) shall send a POST request to the AUSF. The content shall contain at least the UE Id and Serving Network Name.

2a.	On success, "201 Created" shall be returned. The content shall contain the representation of the resource generated and the "Location" header shall contain the URI of the generated resource (e.g. …/ue_authentications/{authCtxId}/eap-session). The AUSF generates a sub-resource "eap-session". The AUSF shall provide a hypermedia link towards this sub-resource in the content to indicate to the AMF where it shall send a POST containing the EAP packet response. The content shall also contain the EAP packet EAP-Request/EAP-Type=EAP-TLS (TLS Start)

2b.	On failure or redirection, one of the HTTP status code listed in table 6.1.7.3-1 shall be returned with the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1. In particular, if the serving network is not authorized, the AUSF shall use the "Cause" SERVING_NETWORK_NOT_AUTHORIZED.

3.	Based on the relation type, the NF Service Consumer (AMF) shall send a POST request including the EAP-Response/EAP-Type=EAP-TLS (TLS client_hello) received from the UE. The POST request is sent to the URI provided by the AUSF or derived by the NF Service Consumer (AMF).

4a.	On success, the AUSF shall reply with a "200 OK" HTTP message containing the EAP Request as described in Annex B.2.1 of 3GPP TS 33.501 [8] and a hypermedia link towards the sub-resource "eap-session".

4b.	On failure or redirection, one of the HTTP status code listed in table 6.1.7.3-1 shall be returned with the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

5.	The NF Service Consumer (AMF) shall send a POST request including the EAP Response received from the UE. The POST request is sent to the URI provided by the AUSF or derived by the NF Service Consumer (AMF).

6a.	On success, the AUSF shall reply with a "200 OK" HTTP message containing the EAP Request as described in Annex B.2.1 of 3GPP TS 33.501 [8] and a hypermedia link towards the sub-resource "eap-session".

6b. On failure or redirection, one of the HTTP status code listed in table 6.1.7.3-1 shall be returned with the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

7.	The NF Service Consumer (AMF) shall send a POST request including the EAP Response received from the UE. The POST request is sent to the URI provided by the AUSF or derived by the NF Service Consumer (AMF).

8a.	If the EAP authentication exchange is successfully completed (with or without the optional Notification Request/Response messages exchange), "200 OK" shall be returned to the NF Service Consumer (AMF). The content shall contain the result of the authentication, an EAP success/failure and the Kseaf if the authentication is successful.

8b.	On failure or redirection, one of the HTTP status code listed in table 6.1.7.3-1 shall be returned with the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

Annex C (informative):
Withdrawn API versions

# C.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. 3GPP TS 29.501 [5] clause 4.3.1.6 describes the withdrawal of API versions.

# C.2	Nausf_SoRProtection API

The API versions listed in table C.2-1 are withdrawn for the Nausf_SoRProtection API.

Table C.2-1: Withdrawn API versions of the Nausf_SoRProtection service

| API version number | Reason for withdrawal |
| --- | --- |
| 1.0.0 | The version 1.0.0 indicates that a 201 "Created" must be sent in response to the POST Custom Operations while it should be a "200 OK" as indicated in clause 6.2.3.2.4.2.2. The version 1.0.1 corrects this mistake. As a consequence, the version 1.0.0 is withdrawn in order to avoid interoperability problems with further version of the Nausf_SoRProtection service. |

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-10 | CT4#80 | C4-175268 |  |  |  | Initial Draft.(Agreed Skeleton) | 0.1.0 |
| 2017-10 | CT4#80 | C4-175394 |  |  |  | Inclusion of pCR agreeds during CT4#80: C4-175269 and C4-175270 | 0.2.0 |
| 2017-12 | CT4#81 | C4-176437 |  |  |  | Inclusion of pCR agreeds during CT4#81: C4-176267, C4-176269, C4-176426, C4-17427 | 0.3.0 |
| 2018-01 | CT4#82 | C4-181391 |  |  |  | Inclusion of pCR agreeds during CT4#82: C4-181341, C4-181342, C4-181343, C4-181344, C4-181345,C4-181346, C4-181347,C4-181155 | 0.4.0 |
| 2018-03 | CT4#83 | C4-182434 |  |  |  | Inclusion of pCRs agreeds during CT4#83: C4-182283 and C4-182279 | 0.5.0 |
| 2018-03 | CT#79 | CP-180031 |  |  |  | Presented for information | 1.0.0 |
| 2018-04 | CT4#84 | C4-183516 |  |  |  | Inclusion of pCRs agreed during CT4#84: C4-183309, C4-183313, C4-183346, C4-183347 and C4-183448 | 1.1.0 |
| 2018-05 | CT4#85 | C4-184623 |  |  |  | Inclusion of PCRs agreeds during CT4#83: C4-184219, C4-184220, C4-184224, C4-184227, C4-184227, C4-184362, C4-184363, C4-184367, C4-184368, C4-184370, C4-184376, C4-184380, C4-184584, C4-184624 | 1.2.0 |
| 2018-06 | CT#80 | CP-181104 |  |  |  | Presented for approval | 2.0.0 |
| 2018-06 | CT#80 |  |  |  |  | Approved in CT#80. | 15.0.0 |
| 2018-09 | CT#81 | CP-182059 | 0002 | 2 | F | Requester ID in Authentication Info | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0003 | 1 | F | HTTP method in figure 5.2.2.2.2-1 (Note: clause 6.1.3.1 is not included, already covered) | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0004 | 4 | F | SoRProtection service operation | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0010 | 1 | F | Adding TS 33.501 reference | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0011 | - | F | HTTP Custom Header | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0013 | 1 | F | SUPI sends to AMF | 15.1.0 |
| 2018-09 | CT#81 | CP-182068 | 0014 | 2 | B | 5G Trace for AUSF | 15.1.0 |
| 2018-09 | CT#81 | CP-182013 | 0015 | 2 | F | Making Oauth 2.0 optional in OAS description | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0016 | 1 | F | Editorial Corrections | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0017 | 1 | F | Error code correction | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0018 | 1 | F | Add support to EAP-TLS (Optional) | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0019 | - | F | Correcting Presentation of resources for AUSF API | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0020 | 1 | F | Correcting confirmation message | 15.1.0 |
| 2018-09 | CT#81 | CP-182059 | 0021 | - | F | API version number update | 15.1.0 |
| 2018-12 | CT#82 | CP-183017 | 0026 | - | F | Remove the "supiOrSuci" in Confirmation Data | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0027 | - | F | Correcting Resource URI structure of the SoRProtection Service | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0030 | - | F | Cardinality | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0031 | - | F | Add supi and authResult to EapSession in OpenAPI definitions | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0022 | - | F | Requester ID not needed in initial request from AMF | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0023 | - | F | Delaying transmission of Kseaf | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0024 | 1 | F | Correcting the reference to EAP-AKA' | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0025 | 1 | F | Adding a reference to the Annex in the Specification | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0028 | 1 | F | Error handling in AUSF | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0029 | 1 | F | Add a reference to the IETF RFC 3748 on EAP Framework | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0032 | - | F | Base64 reference | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0033 | - | F | APIRoot Clarification | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0034 | - | F | Reference correction | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0036 | - | F | OpenAPI version number for Nausf_UEAuthentication service | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0037 | 1 | F | OpenAPI version number for Nausf_SoRProtection | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0038 | 1 | F | Correct "externalDocs" for Nausf_UEAuthentication OAS | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0039 | 1 | F | Clarification on the 200 OK returned by AUSF in case of authentication failure | 15.2.0 |
| 2018-12 | CT#82 | CP-183017 | 0040 | - | F | Secured packet in SorInfo | 15.2.0 |
| 2018-12 | CT#82 | CP-183170 | 0035 | 2 |  | Location Header in OpenAPI | 15.2.0 |
| 2018-12 | CT#82 | CP-183172 | 0041 | - | F | Alignement for Oauth scopes - Nausf_UEAuthentication | 15.2.0 |
| 2018-12 | CT#82 | CP-183173 | 0042 | - | F | Alignement for Oauth scopes - Nausf_SoRProtection | 15.2.0 |
| 2018-12 | CT#82 | CP-183203 | 0043 | - | F | externalDocs for Nausf_SoRProtection OpenAPI Annex | 15.2.0 |
| 2019-03 | CT#83 | CP-190022 | 0047 | 1 | F | Mandatory HTTP status codes | 15.3.0 |
| 2019-03 | CT#83 | CP-190022 | 0049 | 1 | F | SoR Protection response code alignment | 15.3.0 |
| 2019-03 | CT#83 | CP-190022 | 0044 | 3 | F | Authentication Failure scenarios | 15.3.0 |
| 2019-03 | CT#83 | CP-190153 | 0046 | 7 | F | UE parameters update support (indicated as C4-190618 + C4-190618_rev7I) | 15.3.0 |
| 2019-03 | CT#83 | CP-19205 | 0050 | 1 | F | 3GPP TS 29.509 API version update | 15.3.0 |
| 2019-06 | CT#84 | CP-191033 | 0051 | - | F | Correct the expression of URI variables in 5g-aka-confirmation resource | 15.4.0 |
| 2019-06 | CT#84 | CP-191033 | 0053 | 2 | F | Storage of OpenAPI specification files | 15.4.0 |
| 2019-06 | CT#84 | CP-191033 | 0056 | - | F | AUSF Tracing Targeting a PEI | 15.4.0 |
| 2019-06 | CT#84 | CP-191033 | 0055 | 1 | F | Determination of the Authentication Method | 15.4.0 |
| 2019-06 | CT#84 | CP-191218 | 0059 | 4 | F | Add withdrawn API version Annex | 15.4.0 |
| 2019-06 | CT#84 | CP-191033 | 0058 | 1 | F | Essential correction to add Copyright on OpenAPI specifications | 15.4.0 |
| 2019-06 | CT#84 | CP-191057 | 0054 | 1 | F | Non cacheable 501 response | 16.0.0 |
| 2019-06 | CT#84 | CP-191057 | 0057 | 1 | B | UDM service discovery based on GroupID and/or RoutingID | 16.0.0 |
| 2019-06 | CT#84 | CP-191223 | 0060 | 2 | F | 3GPP TS 29.509 API version update | 16.0.0 |
| 2019-09 | CT#85 | CP-192106 | 0063 | 1 | F | Missing status codes | 16.1.0 |
| 2019-09 | CT#85 | CP-192120 | 0066 | - | F | API version and ExternalDocs update | 16.1.0 |
| 2019-10 |  |  |  |  |  | Corrupted references fixed | 16.1.1 |
| 2019-12 | CT#86 | CP-193063 | 0072 | - | F | Add UPU protection in AUSF functionality | 16.2.0 |
| 2019-12 | CT#86 | CP-193058 | 0067 | 1 | B | RoutingId | 16.2.0 |
| 2019-12 | CT#86 | CP-193134 | 0069 | 2 | B | Authentication Indication from W-AGF (tmp) | 16.2.0 |
| 2019-12 | CT#86 | CP-193241 | 0071 | 2 | F | Move ExternalDocs in OpenAPI specifications | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0068 | 1 | F | EAP Payload | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0070 | 1 | F | Add reference to TS 29.524 | 16.2.0 |
| 2019-12 | CT#86 | CP-193044 | 0074 | - | F | 3GPP TS 29.509 API version update | 16.2.0 |
| 2020-03 | CT#87-e | CP-200020 | 0076 | 1 | F | Reference to Data Type SteeringInfo | 16.3.0 |
| 2020-03 | CT#87-e | CP-200242 | 0082 | - | F | Initial Registration procedure on a CAG Cell | 16.3.0 |
| 2020-03 | CT#87-e | CP-200039 | 0075 | 1 | F | Add Corresponding API descriptions in clause 5.1 | 16.3.0 |
| 2020-03 | CT#87-e | CP-200039 | 0077 | 2 | F | Correction - formatting consistency | 16.3.0 |
| 2020-03 | CT#87-e | CP-200035 | 0078 | 2 | F | Editorial corrections | 16.3.0 |
| 2020-03 | CT#87-e | CP-200035 | 0081 | 1 | F | SUPI pattern | 16.3.0 |
| 2020-03 | CT#87-e | CP-200242 | 0083 | 1 | F | AUSF service update for the authentication result removal | 16.3.0 |
| 2020-03 | CT#87-e | CP-200020 | 079 | 2 | F | Optionality of ProblemDetails | 16.3.0 |
| 2020-03 | CT#87-e | CP-200052 | 0084 | - | F | 3GPP TS 29.509 Rel16 API version and External doc update | 16.3.0 |
| 2020-03 | CT#87-e | CP-200252 | 0085 | - | F | OTAF NF name change to SP-AF | 16.3.0 |
| 2020-06 | CT#88-e | CP-201034 | 0093 | - | D | Editorial Clarifications | 16.4.0 |
| 2020-06 | CT#88-e | CP-201034 | 0095 | - | B | Maintain only latest Kausf in network | 16.4.0 |
| 2020-06 | CT#88-e | CP-201034 | 0096 | - | F | AUSF service update for the authentication result removal | 16.4.0 |
| 2020-06 | CT#88-e | CP-201034 | 0097 | - | F | Miscellaneous Corrections | 16.4.0 |
| 2020-06 | CT#88-e | CP-201048 | 0094 | 1 | B | N5GC device Authentication | 16.4.0 |
| 2020-06 | CT#88-e | CP-201063 | 0086 | 1 | F | Supported Headers Tables for Response code 201 | 16.4.0 |
| 2020-06 | CT#88-e | CP-201063 | 0091 | 1 | F | Datatype column in Resource URI variables Table | 16.4.0 |
| 2020-06 | CT#88-e | CP-201063 | 0092 | 1 | F | Add custom operation Name | 16.4.0 |
| 2020-06 | CT#88-e | CP-201063 | 0088 | 2 | F | Editorial Error Corrections | 16.4.0 |
| 2020-06 | CT#88-e | CP-201073 | 0098 | - | F | 29.509 Rel16 API version and External doc update | 16.4.0 |
| 2020-09 | CT#89-e | CP-202088 | 0100 | - | F | Custom Operation Correction | 16.5.0 |
| 2020-09 | CT#89-e | CP-202043 | 0103 | - | F | SoR Header | 16.5.0 |
| 2020-09 | CT#89-e | CP-202115 | 0104 | - | F | Corrections on SoRProtection service | 16.5.0 |
| 2020-09 | CT#89-e | CP-202110 | 0105 | - | F | Corrections on UPUProtection service | 16.5.0 |
| 2020-09 | CT#89-e | CP-202089 | 0101 | 1 | F | Storage of YAML files in 3GPP Forge | 16.5.0 |
| 2020-12 | CT#90-e | CP-203042 | 0110 | 2 | F | Initial Registration procedure on a CAG Cell | 16.6.0 |
| 2020-12 | CT#90-e | CP-203036 | 0111 | - | F | API Version and External Doc Update (R16) | 16.6.0 |
| 2020-12 | CT#90-e | CP-203063 | 0108 | 1 | C | Evolution of SoR delivery mechanism – AUSF API Changes | 17.0.0 |
| 2020-12 | CT#90-e | CP-203055 | 0112 | - | F | API Version and External Doc Update (R17) | 17.0.0 |
| 2021-02 |  |  |  |  |  | Clauses 5.2.2.X/5.2.2.X.1 numbered correctly | 17.0.1 |
| 2021-03 | CT#91-e | CP-210021 | 0114 | - | D | Miscellaneous Corrections | 17.1.0 |
| 2021-03 | CT#91-e | CP-210029 | 0121 | - | F | 29.509 Rel-17 API version and External Doc update | 17.1.0 |
| 2021-03 | CT#91-e | CP-210034 | 0120 | 1 | F | OpenAPI Reference and description field for map data types | 17.1.0 |
| 2021-03 | CT#91-e | CP210037 | 0119 | 1 | F | HTTP 3xx redirection | 17.1.0 |
| 2021-06 | CT#92-e | CP-211028 | 0123 | - | F | Data Types Descriptions | 17.2.0 |
| 2021-06 | CT#92-e | CP-211028 | 0125 | 1 | F | Resource for Individual Authentication | 17.2.0 |
| 2021-06 | CT#92-e | CP-211046 | 0124 | - | F | Data Type Correction | 17.2.0 |
| 2021-06 | CT#92-e | CP-211050 | 0126 | - | F | 29.509 Rel-17 API version and External doc update | 17.2.0 |
| 2021-09 | CT#93-e | CP-212082 | 0130 | 1 | A | UPU Header within UPU Data Protection | 17.3.0 |
| 2021-09 | CT#93-e | CP-212026 | 0131 | - | F | Base64 Encoding of binary attributes in JSON objects | 17.3.0 |
| 2021-09 | CT#93-e | CP-212059 | 0132 | - | F | 29.509 Rel-17 API version and External doc update | 17.3.0 |
| 2021-12 | CT#94-e | CP-213085 | 0135 | - | F | Feature numbering | 17.4.0 |
| 2021-12 | CT#94-e | CP-213085 | 0136 | - | F | Corrections to the API URI | 17.4.0 |
| 2021-12 | CT#94-e | CP-213087 | 0138 | - | F | Corrections related to the description fields in the OpenAPI descriptions | 17.4.0 |
| 2022-03 | CT#95-e | CP-220026 | 0149 | - | F | Routing Indicator | 17.5.0 |
| 2022-03 | CT#95-e | CP-220047 | 0141 | - | F | SNPN onboarding impacts on AUSF services – R17 | 17.5.0 |
| 2022-03 | CT#95-e | CP-220047 | 0152 | - | B | EAP-TTLS support in SNPN (TS 29.509) | 17.5.0 |
| 2022-03 | CT#95-e | CP-220047 | 0144 | 3 | B | PVS Info | 17.5.0 |
| 2022-03 | CT#95-e | CP-220053 | 0143 | 1 | B | NSWO authentication | 17.5.0 |
| 2022-03 | CT#95-e | CP-220069 | 0151 | - | A | 307/308 redirection | 17.5.0 |
| 2022-03 | CT#95-e | CP-220066 | 0155 | - | F | 29.509 Rel-17 API version and External doc update | 17.5.0 |
| 2022-06 | CT#96 | CP-221039 | 0162 | 1 | B | UPU Transparent Container | 17.6.0 |
| 2022-06 | CT#96 | CP-221027 | 0157 | - | F | Correction of typos in description fields | 17.6.0 |
| 2022-06 | CT#96 | CP-221039 | 0158 | - | B | Disaster Roaming Indicator | 17.6.0 |
| 2022-06 | CT#96 | CP-221043 | 0161 | - | B | Overview of New ProseAuthenticate service operation | 17.6.0 |
| 2022-06 | CT#96 | CP-221043 | 0160 | 1 | B | New ProseAuthenticate service operation | 17.6.0 |
| 2022-06 | CT#96 | CP-221053 | 0159 | - | F | Reused data types | 17.6.0 |
| 2022-06 | CT#96 | CP-221053 | 0164 | - | F | Editorial corrections | 17.6.0 |
| 2022-06 | CT#96 | CP-221056 | 0163 | 1 | A | IETF has published RFC 9048 | 17.6.0 |
| 2022-06 | CT#96 | CP-221338 | 0168 | 1 | F | 29.509 Rel-17 API version and External doc update | 17.6.0 |
| 2022-09 | CT#97 | CP-222026 | 0170 | - | F | Description alignment with yaml | 17.7.0 |
| 2022-09 | CT#97 | CP-222034 | 0171 | 1 | F | MSK in EapSession | 17.7.0 |
| 2022-09 | CT#97 | CP-222035 | 0172 | - | F | 5G ProSe Authentication Service Description Alignment | 17.7.0 |
| 2022-09 | CT#97 | CP-222035 | 0173 | 1 | F | 5G ProSe Authentication API Update | 17.7.0 |
| 2022-09 | CT#97 | CP-222035 | 0174 | 1 | F | Resolve EN For 5G_AKA support | 17.7.0 |
| 2022-09 | CT#97 | CP-222037 | 0169 | - | F | PVS related alignement | 17.7.0 |
| 2022-09 | CT#97 | CP-222058 | 0176 | - | F | 29.509 Rel-17 API version and External doc update | 17.7.0 |
| 2022-12 | CT#98 | CP-223049 | 0180 | - | F | UE onboarding in authentication request | 17.8.0 |
| 2022-12 | CT#98 | CP-223054 | 0185 | 1 | F | Update on Nausf_UEAuthentication_Authenticate service operation | 17.8.0 |
| 2022-12 | CT#98 | CP-223054 | 0179 | 1 | F | 5GPRUK Name Alignment | 17.8.0 |
| 2022-12 | CT#98 | CP-223027 | 0178 | 1 | F | Missing Mandatory Status Codes in OpenAPI | 18.0.0 |
| 2022-12 | CT#98 | CP-223033 | 0186 | - | F | 29.509 Rel-18 API version and External doc update | 18.0.0 |
| 2022-12 | CT#98 | CP-223063 | 0181 | 1 | F | boolean type IEs definition correction | 18.0.0 |
| 2022-12 | CT#98 | CP-223063 | 0182 | 1 | D | Editorial corrections | 18.0.0 |
| 2023-06 | CT#100 | CP-231346 | 0191 | 1 | B | OAuth2 scopes in the Nausf_UEAuthentication API | 18.1.0 |
| 2023-06 | CT#100 | CP-231026 | 0189 | 3 | F | Location header description | 18.1.0 |
| 2023-06 | CT#100 | CP-231041 | 0195 | 1 | B | Update the reference model | 18.1.0 |
| 2023-06 | CT#100 | CP-231078 | 0194 | - | A | Add serving network name in ProSeAuthenticationInfo | 18.1.0 |
| 2023-06 | CT#100 | CP-231345 | 0198 | - | F | 29.509 Rel-18 API version and External doc update | 18.1.0 |
| 2023-09 | CT#101 | CP-232033 | 0202 | - | F | Adding the missing description fields | 18.2.0 |
| 2023-09 | CT#101 | CP-232056 | 0199 | - | B | Authentication of AUN3 devices behind RG | 18.2.0 |
| 2023-09 | CT#101 | CP-232056 | 0200 | 1 | B | Update the description of MSK to support authentication for AUN3 devices behind 5G-RG | 18.2.0 |
| 2023-09 | CT#101 | CP-232060 | 0203 | - | F | 29.509 Rel-18 API version and External doc update | 18.2.0 |
| 2023-12 | CT#102 | CP-233027 | 0204 |  | F | HTTP RFC obsoleted by IETF RFC 9113 | 18.3.0 |
| 2023-12 | CT#102 | CP-233030 | 0207 |  | F | ProblemDetails RFC 7807 obsoleted by 9457 | 18.3.0 |
| 2023-12 | CT#102 | CP-233055 | 0206 | 1 | B | Enhancement to support UE-to-UE relay | 18.3.0 |
| 2023-12 | CT#102 | CP-233060 | 0208 |  | F | 29.509 Rel-18 API version and External doc update | 18.3.0 |
| 2024-03 | CT#103 | CP-240067 | 0210 |  | F | Disaster Roaming Indication | 18.4.0 |
| 2024-03 | CT#103 | CP-240056 | 0211 | - | F | 29.509 Rel-18 API version and External doc update | 18.4.0 |
| 2024-06 | CT#104 | CP-241413 | 0217 | 1 | F | Clarification on deviation of naming-convention for enum values in Nausf specific Data Type | 18.5.0 |
| 2024-06 | CT#104 | CP-241455 | 0216 | 1 | F | Definition of ProseAuthData | 18.5.0 |
| 2024-06 | CT#104 | CP-241475 | 0214 | 2 | A | Disaster Condition PLMN List | 18.5.0 |
| 2024-06 | CT#104 | CP-241544 | 0215 | 3 | F | Style Corrections of Nausf APIs | 18.5.0 |
| 2024-06 | CT#104 | CP-242158 | 0220 | - | F | Feature negotiation correction | 18.5.0 |
| 2024-06 | CT#104 | CP-242214 | 0221 | - | F | Update on incorrect attribute name | 18.5.0 |
| 2024-06 | CT#104 | CP-242355 | 0219 | 1 | F | Clarifying the self-references and editorial corrections | 18.5.0 |
| 2024-06 | CT#104 | CP-242505 | 0222 | - | F | 29.509 Rel-18 API version and External doc update | 18.5.0 |