3GPP TS 29.522 V19.2.0 (2025-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Network Exposure Function Northbound APIs;

Stage 3

(Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	39

1	Scope	40

2	References	40

3	Definitions and abbreviations	43

3.1	Definitions	43

3.2	Abbreviations	43

4	NEF Northbound Interface	44

4.1	Overview	44

4.2	Reference model	47

4.3	Functional elements	48

4.3.1	NEF	48

4.3.2	AF	48

4.4	Procedures over NEF Northbound Interface	48

4.4.1	Introduction	48

4.4.2	Procedures for Monitoring	48

4.4.3	Procedures for Device Triggering	58

4.4.4	Procedures for resource management of Background Data Transfer	58

4.4.5	Procedures for CP Parameters Provisioning	59

4.4.6	Procedures for PFD Management	60

4.4.7	Procedures for Traffic Influence	61

4.4.7.1	General	61

4.4.7.2	AF request identified by UE address	62

4.4.7.3	AF request not identified by UE address	62

4.4.7.4	Handling of Event Notifications	63

4.4.7.4.1	Handling of UP path management event notification	63

4.4.7.4.2	Handling of Traffic routing requirements enforcement outcome event notification	64

4.4.7.5	Processing AF requests to influence traffic routing for HR-SBO session	64

4.4.7.6	Handling of payload headers event notification	65

4.4.8	Procedures for changing the chargeable party at session set up or during the session	65

4.4.9	Procedures for AF required QoS	66

4.4.9.1	General	66

4.4.9.2	Procedures for AF setting up an AF session with required QoS for target UE identified by UE address or for target list of UEs identified by list of UE addresses	66

4.4.9.2.0	General	66

4.4.9.2.1	Procedure for QoS handling and QoS monitoring for list of UEs	67

4.4.9.2.2	Subscriptions to QoS handling and QoS Monitoring	68

4.4.9.2.3	Provisioning of TSCAI input Information and QoS related data	72

4.4.9.2.4	Subscription to the access type change and plmn change	73

4.4.9.2.5	Provisioning of ToS traffic class	74

4.4.9.2.6	Provisioning of multi-modal services	74

4.4.9.2.7	Provisioning of Round-Trip latency requirements	75

4.4.9.2.8	Subscription to the report of L4S support information	76

4.4.9.2.9	Provisioning of PDU Set handling	76

4.4.9.2.10	Provisioning of traffic information for UE power saving management	76

4.4.9.2.11	Provisioning of the QoS timing information	76

4.4.9.2.12	Provisioning of Dynamic ChangingTraffic Characteristics	77

4.4.9.2.13	Provisioning of the Multiplexed Media Information	77

4.4.9.2.14 Provisioning of Media Related Information deliver	77

4.4.9.2.15	Subscription to the Rate Limitation information reporting	77

4.4.9.3	Procedures for AF requested QoS for a target UE or group of UE(s) not identified by UE address(es)	77

4.4.10	Procedures for MSISDN-less Mobile Originated SMS	78

4.4.11	Procedures for Network Configuration Parameters Provisioning	78

4.4.12	Procedures for Non-IP data delivery	79

4.4.12.1	General	79

4.4.12.2	NIDD configuration Triggered by the NEF	79

4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery	80

4.4.13	Procedures for RACS Parameter Provisioning	80

4.4.14	Procedures for analytics information exposure	80

4.4.14.1	Subscription/unsubscription to notification of analytics information	80

4.4.14.2	Fetch analytics information	82

4.4.15	Procedures for 5G LAN Parameter Provisioning	83

4.4.15.1	General	83

4.4.15.2	Creation of a new subscription for 5G LAN parameter provisioning	83

4.4.15.3	Modification of an existing subscription for 5G LAN parameter provisioning	83

4.4.15.4	Deletion of an existing subscription for 5G LAN parameter provisioning	84

4.4.15.5	5G LAN parameter provisioning event notification	84

4.4.16	Procedures for applying BDT policy	84

4.4.17	Procedures for Enhanced Coverage Restriction Control	85

4.4.18	Procedures for IPTV Configuration	85

4.4.20	Procedures for service specific parameter provisioning	87

4.4.21	Procedures for ACS configuration parameter provisioning	90

4.4.22	Procedures for Mobile Originated Location Request	91

4.4.22.1	General	91

4.4.22.2	Location Update Notification triggered by UE	91

4.4.23	Procedures for AKMA	91

4.4.23.1	General	91

4.4.23.2	AKMA Application Key Request	91

4.4.23.3	AKMA Service Disablement Notification	92

4.4.24	Procedures for Time Synchronization Exposure	92

4.4.24.0	General	92

4.4.24.1	Subscription and unsubscription to notification of Time Synchronization Capabilites	92

4.4.24.2	Time Synchronization Exposure Configuration	94

4.4.24.3	Management of 5G access stratum time distribution	95

4.4.25	Procedures for ECS address Provisioning	96

4.4.26	Procedures for AM Policy Authorization	97

4.4.26.1	General	97

4.4.26.2	Creation of a new Individual Application AM Context	97

4.4.26.3	Modification of an existing individual Application AM Context	98

4.4.26.4	Deletion of an existing individual Application AM Context	98

4.4.26.5	Create or modify subscription to notification of AM policy event	98

4.4.26.6	Unsubscription to notification of AM policy event	99

4.4.26.7	Notification of AM policy event	99

4.4.27	Procedures for AF triggered Access and Mobility Influence	99

4.4.27.1	General	99

4.4.27.2	Create the AM Influence Subscription	99

4.4.27.3	Modifiy the AM Influence Subscription	100

4.4.27.4	Delete the AM Influence Subscription	100

4.4.27.5	Notification of AM influence events	100

4.4.28	Procedures for Northbound EAS Deployment Information management	101

4.4.28.1	General	101

4.4.28.2	Creation of a new Individual EAS Deployment information resource	101

4.4.28.3	Modification of an existing individual EAS Deployment Information resource	102

4.4.28.4	Deletion of an existing individual EAS Deployment Information resource	102

4.4.28.5	Deletion of EAS Deplyoment Information based on given criteria	102

4.4.29	Procedures for MBS Management	103

4.4.29.1	General	103

4.4.29.2	Procedures for MBS TMGI management	103

4.4.29.2.1	General	103

4.4.29.2.2	Procedure for MBS TMGI(s) allocation or MBS TMGI(s) expiry time refresh	103

4.4.29.2.3	Procedure for MBS TMGI(s) deallocation	104

4.4.29.2.4	Procedure for MBS TMGI(s) timer expiry notification	104

4.4.29.3	Procedures for MBS session management	105

4.4.29.3.1	General	105

4.4.29.3.2	Procedure for MBS session creation	105

4.4.29.3.3	Procedure for MBS session update	107

4.4.29.3.4	Procedure for MBS session deletion	108

4.4.29.3.5	Procedure for MBS session status subscription	108

4.4.29.3.6	Procedure for MBS session status unsubscription	109

4.4.29.3.7	Procedure for MBS session status notification	109

4.4.29.4	Procedures for MBS Parameters Provisioning	110

4.4.29.4.1	General	110

4.4.29.4.2	Procedure for multicast MBS Session Authorization information provisioning	110

4.4.29.4.3	Procedure for multicast MBS Session Assistance information provisioning	111

4.4.29.5	Procedures for MBS User Service management	112

4.4.29.5.1	General	112

4.4.29.5.2	Procedure for MBS User Service creation	112

4.4.29.5.3	Procedure for MBS User Service retrieval	112

4.4.29.5.4	Procedure for MBS User Service update/modification	112

4.4.29.5.5	Procedure for MBS User Service deletion	113

4.4.29.6	Procedures for MBS User Data Ingest Session management	113

4.4.29.6.1	General	113

4.4.29.6.2	Procedure for MBS User Data Ingest Session creation	113

4.4.29.6.3	Procedure for MBS User Data Ingest Session retrieval	114

4.4.29.6.4	Procedure for MBS User Data Ingest Session update/modification	114

4.4.29.6.5	Procedure for MBS User Data Ingest Session deletion	115

4.4.29.6.6	Procedure for MBS User Data Ingest Session Status Subscription	115

4.4.29.6.7	Procedure for MBS User Data Ingest Session Status update/modification	115

4.4.29.6.8	Procedure for MBS User Data Ingest Session Status Unsubscription	116

4.4.29.6.9	Procedure for MBS User Data Ingest Session Status Notification	116

4.4.29.7	Procedures for MBS Group Message Delivery Management	117

4.4.29.7.1	General	117

4.4.29.7.2	Procedure for MBS Group Message Delivery Creation	117

4.4.29.7.3	Procedure for MBS Group Message Delivery Update	117

4.4.29.7.4	Procedure for MBS Group Message Delivery Deletion	118

4.4.29.7.5	Procedure for MBS Group Message Delivery Status Notification	118

4.4.30	Procedures for Data Reporting	118

4.4.30.1	General	118

4.4.30.2	Procedure for Data Reporting Session Management	119

4.4.30.3	Procedure for Data Report	119

4.4.31	Procedures for Data Reporting Provisioning	119

4.4.31.1	General	119

4.4.31.2	Procedure for Data Reporting Provisioning Session Management	119

4.4.31.3	Procedure for Data Reporting Configuration management	120

4.4.32	Procedures for UE ID retrieval	121

4.4.32.1	General	121

4.4.32.2	Retrieve AF specific UE ID service operation	121

4.4.32.3	Retrieve UE ID in the form of MSISDN	122

4.4.32.4	UE ID Mapping Information Provisioning	123

4.4.32.5	Verify UE ID in the form of MSISDN	124

4.4.33	Procedures for Media Streaming Event Exposure	125

4.4.33.1	General	125

4.4.33.2	Procedure for Media Streaming Event Exposure Subscription Creation	125

4.4.33.3	Procedure for Media Streaming Event Exposure Subscription Update	125

4.4.33.4	Procedure for Media Streaming Event Exposure Unsubscription	126

4.4.33.5	Procedure for Media Streaming Event Exposure Notification	126

4.4.34	Procedures for DNAI Mapping	126

4.4.34.1	General	126

4.4.34.2	Creation of a new subscription for DNAI Mapping	126

4.4.34.3	Deletion of an existing individual DNAI Mapping subscription	127

4.4.34.4	Notification for updated DNAI Mapping information	127

4.4.35	Procedures for negotiation of planned data transfer with QoS requirements	127

4.4.36	Procedures for Member UE Selection Assistance	128

4.4.36.1	General	128

4.4.36.2	Procedure for Member UE Selection Assistance Subscription Creation	128

4.4.36.3	Procedure for Member UE Selection Assistance Subscription Update	129

4.4.36.4	Procedure for Member UE Selection Assistance Subscription Unsubscription	129

4.4.36.5	Procedure for Member UE Selection Assistance Notification	129

4.4.37	Procedures for Group Parameters Provisioning	130

4.4.37.1	General	130

4.4.37.2	Procedures for DNN and S-NSSAI specific Group Parameters Provisioning	130

4.4.38	Procedures for Network Slice Parameters Provisioning	131

4.4.38.1	General	131

4.4.38.2	Procedures for Network Slice Usage Control Parameters Provisioning	132

4.4.39	Procedures for UE Address Information Management	132

4.4.39.1	General	132

4.4.39.2	Procedures for UE Address(es) Information Retrieval	132

4.4.39.3	UE Address Subscription Creation	133

4.4.39.4	UE Address Subscription Deletion	133

4.4.39.5	Notification for UE Address information	134

4.4.40	Procedures for ECS Address Configuration Information provisioning in roaming	134

4.4.40.1	General	134

4.4.40.2	Creation of new ECS Address Configuration Information Set	134

4.4.40.3	Modification of existing ECS Address Configuration Information Set	135

4.4.40.4	Deletion of existing Individual ECS Address Configuration Information Set	135

4.4.40.5	Deletion of ECS Address Configuration Information based on given criteria	135

4.4.41.1	General	136

4.4.41.2	Procedures for RSLPPI Parameters Provisioning	136

4.4.42.1	General	137

4.4.42.2	Procedures for Static IP Addressing Parameters Provisioning	137

4.4.43.1	General	138

4.4.43.2	Procedure for UAV Flight Assistance Configuration Creation	138

4.4.43.3	Procedure for UAV Flight Assistance Configuration Update	139

4.4.43.4	Procedure for UAV Flight Assistance Configuration Deletion	139

4.4.43.5	Procedure for UAV Flight Assistance Notification	139

4.4.44	Procedures for CAG Information Parameters Provisioning	140

4.4.44.1	General	140

4.4.45	Procedures for UAV Flight information Retrieval	141

4.4.45.1	General	141

4.4.45.2	Procedures for UAV Flight information Retrieval	141

4.4.46	Procedures for IMS Session Management	141

4.4.46.1	General	141

4.4.46.2	Creation a new IMS session	141

4.4.46.3	Modification of an existing IMS session	142

4.4.46.4	Release an existing IMS session	142

4.4.46.5	Procedure for IMS EE Notification	142

4.4.47	Procedures for IMS Event Exposure Services	143

4.4.47.1	General	143

4.4.47.2	Creation of IMS Event Exposure Service Subscription	143

4.4.47.3	Procedure for IMS Event Exposure Service Subscription Update	143

4.4.47.4	Unsubscription of IMS Event Exposure Service	143

4.4.47.5	Notification of IMS Event(s)	144

4.4.48	Procedures for IMS Parameters Provision Service	144

4.4.48.1	General	144

4.4.48.2	Procedures for IMS Parameters Provisioning	144

5	NEF Northbound APIs	145

5.1	Introduction	145

5.2	Information applicable to several APIs	148

5.3	Reused APIs	148

5.4	TrafficInfluence API	149

5.4.0	Introduction	149

5.4.1	Resources	150

5.4.1.1	Overview	150

5.4.1.2	Resource: Traffic Influence Subscription	150

5.4.1.2.1	Introduction	150

5.4.1.2.2	Resource Definition	150

5.4.1.2.3	Resource Methods	151

5.4.1.2.3.1	General	151

5.4.1.2.3.2	GET	151

5.4.1.2.3.3	POST	152

5.4.1.3	Resource: Individual Traffic Influence Subscription	152

5.4.1.3.1	Introduction	152

5.4.1.3.2	Resource Definition	152

5.4.1.3.3	Resource Methods	153

5.4.1.3.3.1	General	153

5.4.1.3.3.2	GET	153

5.4.1.3.3.3	PUT	154

5.4.1.3.3.4	PATCH	154

5.4.1.3.3.5	DELETE	155

5.4.1A	Custom Operations without associated resources	156

5.4.2	Notifications	156

5.4.2.1	Introduction	156

5.4.2.2	Event Notification	157

5.4.2.2.1	Description	157

5.4.2.2.2	Target URI	157

5.4.2.2.3	Operation Definition	157

5.4.2.3	Acknowledgement of event notification	158

5.4.2.3.1	Description	158

5.4.2.3.2	Target URI	158

5.4.2.3.3	Operation Definition	159

5.4.2.3.3.1	Notification via HTTP POST	159

5.4.2.4	Event Notification of Payload Headers	159

5.4.2.4.1	Description	159

5.4.2.4.2	Target URI	159

5.4.2.4.3	Operation Definition	160

5.4.3	Data Model	161

5.4.3.1	General	161

5.4.3.2	Reused data types	161

5.4.3.3	Structured data types	162

5.4.3.3.1	Introduction	162

5.4.3.3.2	Type: TrafficInfluSub	162

5.4.3.3.3	Type: TrafficInfluSubPatch	169

5.4.3.3.4	Type: EventNotification	174

5.4.3.3.5	Type: AfResultInfo	177

5.4.3.3.6	Type AfAckInfo	177

5.4.3.3.7	Type TrafficDataSet	177

5.4.3.3.8	Type TrafficDataSetRm	178

5.4.3.3.9	Type: HeaderHandlingReport	178

5.4.3.4	Simple data types and enumerations	178

5.4.3.4.1	Introduction	178

5.4.3.4.2	Simple data types	178

5.4.3.4.3	Enumeration: SubscribedEvent	178

5.4.3.4.4	Enumeration: AfResultStatus	179

5.4.4	Used Features	179

5.4.5	Error handling	180

5.4.5.1	General	180

5.4.5.2	Protocol Errors	181

5.4.5.3	Application Errors	181

5.5	NiddConfigurationTrigger API	181

5.5.0	Introduction	181

5.5.1	Resources	181

5.5.1A	Custom Operations without associated resources	181

5.5.2	Notifications	181

5.5.2.1	Introduction	181

5.5.2.2	Event Notification	182

5.5.2.3	Operation Definition	182

5.5.2.3.1	Notification via HTTP POST	182

5.5.2.3.2	Notification via Websocket	183

5.5.3	Data Model	183

5.5.3.1	General	183

5.5.3.2	Reused data types	183

5.5.3.3	Structured data types	183

5.5.3.3.1	Introduction	183

5.5.3.3.2	Type: NiddConfigurationTrigger	183

5.5.3.3.3	Type: NiddConfigurationTriggerReply	183

5.5.3.4	Simple data types and enumerations	184

5.5.3.4.1	Introduction	184

5.5.3.4.2	Simple data types	184

5.5.4	Used Features	184

5.5.5	Error handling	184

5.5.5.1	General	184

5.5.5.2	Protocol Errors	184

5.5.5.3	Application Errors	184

5.6	AnalyticsExposure API	185

5.6.0	Introduction	185

5.6.1	Resources	185

5.6.1.1	Overview	185

5.6.1.2	Resource: Analytics Exposure Subscriptions	186

5.6.1.2.1	Introduction	186

5.6.1.2.2	Resource Definition	186

5.6.1.2.3	Resource Methods	186

5.6.1.2.3.1	General	186

5.6.1.2.3.2	GET	186

5.6.1.2.3.3	POST	187

5.6.1.3	Resource: Individual Analytics Exposure Subscription	188

5.6.1.3.1	Introduction	188

5.6.1.3.2	Resource Definition	188

5.6.1.3.3	Resource Methods	188

5.6.1.3.3.1	General	188

5.6.1.3.3.2	GET	188

5.6.1.3.3.3	PUT	189

5.6.1.3.3.4	DELETE	190

5.6.1A	Custom Operations without associated resources	191

5.6.1A.1	Overview	191

5.6.1A.2	Operation: fetch	192

5.6.1A.2.1	Description	192

5.6.1A.2.2	Operation Definition	192

5.6.2	Notifications	193

5.6.2.1	Introduction	193

5.6.2.2	Event Notification	193

5.6.2.3	Operation Definition	193

5.6.2.3.1	Notification via HTTP POST	193

5.6.2.3.2	Notification via Websocket	194

5.6.3	Data Model	194

5.6.3.1	General	194

5.6.3.2	Reused data types	195

5.6.3.3	Structured data types	199

5.6.3.3.1	Introduction	199

5.6.3.3.2	Type: AnalyticsExposureSubsc	200

5.6.3.3.3	Type: AnalyticsEventNotification	201

5.6.3.3.4	Type: AnalyticsEventNotif	202

5.6.3.3.5	Type: AnalyticsEventSubsc	206

5.6.3.3.6	Type: AnalyticsEventFilterSubsc	207

5.6.3.3.7	Type TargetUeId	213

5.6.3.3.8	Void	214

5.6.3.3.9	Type UeMobilityExposure	214

5.6.3.3.10	Type UeLocationInfo	215

5.6.3.3.11	Void	215

5.6.3.3.12	Type: AnalyticsRequest	215

5.6.3.3.13	Type AnalyticsEventFilter	216

5.6.3.3.14	Type AnalyticsData	221

5.6.3.3.15	Type AbnormalExposure	224

5.6.3.3.16	Type CongestInfo	225

5.6.3.3.17	Type CongestionAnalytics	225

5.6.3.3.18	Type QosSustainabilityExposure	226

5.6.3.3.19	Type NetworkPerfExposure	227

5.6.3.3.20	Type AnalyticsFailureEventInfo	227

5.6.3.3.21	Type WlanPerformInfo	228

5.6.3.4	Simple data types and enumerations	228

5.6.3.4.1	Introduction	228

5.6.3.4.2	Simple data types	228

5.6.3.4.3	Enumeration: AnalyticsEvent	228

5.6.3.4.4	Enumeration: AnalyticsFailureCode	229

5.6.4	Used Features	230

5.6.5	Error handling	234

5.6.5.1	General	234

5.6.5.2	Protocol Errors	234

5.6.5.3	Application Errors	234

5.7	5GLANParameterProvision API	235

5.7.0	Introduction	235

5.7.1	Resources	236

5.7.1.1	Overview	236

5.7.1.2	Resource: 5GLAN Parameters Provision Subscriptions	236

5.7.1.2.1	Introduction	236

5.7.1.2.2	Resource Definition	236

5.7.1.2.3	Resource Methods	237

5.7.1.2.3.1	General	237

5.7.1.2.3.2	GET	237

5.7.1.2.3.3	POST	238

5.7.1.3	Resource: Individual 5GLAN Parameters Provision Subscription	238

5.7.1.3.1	Introduction	238

5.7.1.3.2	Resource Definition	238

5.7.1.3.3	Resource Methods	239

5.7.1.3.3.1	General	239

5.7.1.3.3.2	GET	239

5.7.1.3.3.3	PUT	240

5.7.1.3.3.4	DELETE	240

5.7.1.3.3.5	PATCH	241

5.7.1A	Custom Operations without associated resources	242

5.7.1B	Notifications	242

5.7.1B.1	General	242

5.7.1B.2	5G LAN Parameter Provisioning Event Notification	243

5.7.1B.2.1	Description	243

5.7.1B.2.2	Target URI	243

5.7.1B.2.3	Operation Definition	243

5.7.1B.2.3.1	Notification via HTTP POST	243

5.7.1B.2.3.2	Void	244

5.7.2	Data Model	244

5.7.2.1	General	244

5.7.2.2	Reused data types	244

5.7.2.3	Structured data types	245

5.7.2.3.1	Introduction	245

5.7.2.3.2	Type: 5GLanParametersProvision	246

5.7.2.3.3	Type: 5GLanParameters	246

5.7.2.3.4	Type: AppDescriptor	249

5.7.2.3.5	Type: 5GLanParametersProvisionPatch	250

5.7.2.3.6	Type: 5GLanParametersPatch	250

5.7.2.3.7	Type: AppDescriptorRm	251

5.7.2.3.8	Void	251

5.7.2.3.9	Type: CpParams	251

5.7.2.3.10	Type: NpConfigParams	252

5.7.2.3.11	Type: LpiParams	252

5.7.2.3.12	Type: AcsParams	252

5.7.2.3.13	Type: ECSAddrParams	252

5.7.2.3.14	Type: DnnSnssaiParams	253

5.7.2.3.15	Type: 5GLanParamProvNotif	253

5.7.2.3.16	Type: NpConfigNotif	253

5.7.2.3.17	Type: MaxGrpDataRateInfo	253

5.7.2.4	Simple data types and enumerations	254

5.7.2.4.1	Introduction	254

5.7.2.4.2	Simple data types	254

5.7.2.4.3	Enumeration: AaaUsage	254

5.7.3	Used Features	254

5.7.4	Error handling	255

5.7.4.1	General	255

5.7.4.2	Protocol Errors	255

5.7.4.3	Application Errors	255

5.8	ApplyingBdtPolicy API	255

5.8.0	Introduction	255

5.8.1	Resources	255

5.8.1.1	Overview	255

5.8.1.2	Resource: Applied BDT Policy Subscriptions	256

5.8.1.2.1	Introduction	256

5.8.1.2.2	Resource Definition	256

5.8.1.2.3	Resource Methods	257

5.8.1.2.3.1	General	257

5.8.1.2.3.2	GET	257

5.8.1.2.3.3	POST	258

5.8.1.3	Resource: Individual Applied BDT Policy Subscription	258

5.8.1.3.1	Introduction	258

5.8.1.3.2	Resource Definition	258

5.8.1.3.3	Resource Methods	258

5.8.1.3.3.1	General	258

5.8.1.3.3.2	GET	259

5.8.1.3.3.3	PATCH	259

5.8.1.3.3.4	DELETE	260

5.8.1A	Custom Operations without associated resources	261

5.8.2	Notifications	261

5.8.3	Data Model	261

5.8.3.1	General	261

5.8.3.2	Reused data types	262

5.8.3.3	Structured data types	262

5.8.3.3.1	Introduction	262

5.8.3.3.2	Type: AppliedBdtPolicy	262

5.8.3.3.3	Type: AppliedBdtPolicyPatch	262

5.8.3.4	Simple data types and enumerations	263

5.8.3.4.1	Introduction	263

5.8.3.4.2	Simple data types	263

5.8.4	Used Features	263

5.8.5	Error handling	263

5.8.5.1	General	263

5.8.5.2	Protocol Errors	263

5.8.5.3	Application Errors	263

5.9	IPTVConfiguration API	264

5.9.0	Introduction	264

5.9.1	Resources	264

5.9.1.1	Overview	264

5.9.1.2	Resource: IPTV Configurations	265

5.9.1.2.1	Introduction	265

5.9.1.2.2	Resource Definition	265

5.9.1.2.3	Resource Methods	265

5.9.1.2.3.1	General	265

5.9.1.2.3.2	GET	265

5.9.1.2.3.3	POST	266

5.9.1.3	Resource: Individual IPTV Configuration	267

5.9.1.3.1	Introduction	267

5.9.1.3.2	Resource Definition	267

5.9.1.3.3	Resource Methods	267

5.9.1.3.3.1	General	267

5.9.1.3.3.2	GET	267

5.9.1.3.3.3	PUT	268

5.9.1.3.3.4	DELETE	269

5.9.1.3.3.5	PATCH	270

5.9.1A	Custom Operations without associated resources	271

5.9.1B	Notifications	271

5.9.2	Data Model	271

5.9.2.1	General	271

5.9.2.2	Reused data types	271

5.9.2.3	Structured data types	271

5.9.2.3.1	Introduction	271

5.9.2.3.2	Type: IptvConfigData	272

5.9.2.3.3	Type: MulticastAccessControl	272

5.9.2.3.4	Type: IptvConfigDataPatch	273

5.9.2.4	Simple data types and enumerations	273

5.9.2.4.1	Introduction	273

5.9.2.4.2	Simple data types	273

5.9.2.4.3	Enumeration: AccessRightStatus	273

5.9.3	Used Features	273

5.9.4	Error handling	273

5.9.4.1	General	273

5.9.4.2	Protocol Errors	274

5.9.4.3	Application Errors	274

5.10	LpiParameterProvision API	274

5.10.0	Introduction	274

5.10.1	Resources	274

5.10.1.1	Overview	274

5.10.1.2	Resource: LPI Parameters Provisionings	275

5.10.1.2.1	Introduction	275

5.10.1.2.2	Resource Definition	275

5.10.1.2.3	Resource Methods	275

5.10.1.2.3.1	General	275

5.10.1.2.3.2	GET	275

5.10.1.2.3.3	POST	276

5.10.1.3	Resource: Individual LPI Parameters Provisioning	277

5.10.1.3.1	Introduction	277

5.10.1.3.2	Resource Definition	277

5.10.1.3.3	Resource Methods	277

5.10.1.3.3.1	General	277

5.10.1.3.3.2	GET	277

5.10.1.3.3.3	PUT	278

5.10.1.3.3.3A	PATCH	279

5.10.1.3.3.4	DELETE	280

5.10.1A	Custom Operations without associated resources	281

5.10.1B	Notifications	281

5.10.2	Data Model	281

5.10.2.1	General	281

5.10.2.2	Reused data types	281

5.10.2.3	Structured data types	282

5.10.2.3.1	Introduction	282

5.10.2.3.2	Type: LpiParametersProvision	282

5.10.2.3.3	Type: LpiParametersProvisionPatch	283

5.10.2.4	Simple data types and enumerations	283

5.10.2.4.1	Introduction	283

5.10.2.4.2	Simple data types	283

5.10.3	Used Features	283

5.10.4	Error handling	283

5.10.4.1	General	283

5.10.4.2	Protocol Errors	283

5.10.4.3	Application Errors	284

5.11	ServiceParameter API	284

5.11.0	Introduction	284

5.11.1	Resources	284

5.11.1.1	Overview	284

5.11.1.2	Resource: Service Parameter Subscriptions	285

5.11.1.2.1	Introduction	285

5.11.1.2.2	Resource Definition	285

5.11.1.2.3	Resource Methods	285

5.11.1.2.3.1	General	285

5.11.1.2.3.2	GET	285

5.11.1.2.3.3	POST	287

5.11.1.3	Resource: Individual Service Parameter Subscription	287

5.11.1.3.1	Introduction	287

5.11.1.3.2	Resource Definition	287

5.11.1.3.3	Resource Methods	287

5.11.1.3.3.1	General	287

5.11.1.3.3.2	GET	288

5.11.1.3.3.3	PUT	288

5.11.1.3.3.4	DELETE	289

5.11.1.3.3.5	PATCH	290

5.11.1A	Notifications	291

5.11.1A.1	Introduction	291

5.11.1A.2	AF Notifications	291

5.11.1A.2.1	Description	291

5.11.1A.2.2	Target URI	291

5.11.1A.3	Operation Definition	292

5.11.1A.3.1	Notification via HTTP POST	292

5.11.1A.3.2	Notification via Websocket	293

5.11.1B	Custom Operations without associated resources	293

5.11.2	Data Model	293

5.11.2.1	General	293

5.11.2.2	Reused data types	297

5.11.2.3	Structured data types	297

5.11.2.3.1	Introduction	297

5.11.2.3.2	Type: ServiceParameterData	298

5.11.2.3.3	Type: ServiceParameterDataPatch	302

5.11.2.3.4	Type: UrspRuleRequest	304

5.11.2.3.5	Type: RouteSelectionParameterSet	305

5.11.2.3.6	Type: AfNotification	305

5.11.2.3.7	Type: EventInfo	306

5.11.2.3.8	Type: TrafficDescriptorComponents	307

5.11.2.3.9	Type: NetworkDescription	309

5.11.2.3.10	Void	309

5.11.2.3.11	Void	309

5.11.2.3.12	Type: Non3gppDeviceInformation	309

5.11.2.4	Simple data types and enumerations	309

5.11.2.4.1	Introduction	309

5.11.2.4.2	Simple data types	310

5.11.2.4.3	Enumeration: Event	313

5.11.2.4.4	Enumeration: AuthorizationResult	314

5.11.2.4.5	Enumeration: Failure	314

5.11.2.4.6	Enumeration: ConnectionCapabilities	314

5.11.3	Used Features	315

5.11.4	Error handling	318

5.11.4.1	General	318

5.11.4.2	Protocol Errors	318

5.11.4.3	Application Errors	318

5.12	ACSParameterProvision API	318

5.12.0	Introduction	318

5.12.1	Resources	318

5.12.1.1	Overview	318

5.12.1.2	Resource: ACS Configuration Subscriptions	319

5.12.1.2.1	Introduction	319

5.12.1.2.2	Resource Definition	319

5.12.1.2.3	Resource Methods	320

5.12.1.2.3.1	General	320

5.12.1.2.3.2	GET	320

5.12.1.2.3.3	POST	321

5.12.1.3	Resource: Individual ACS Configuration Subscription	321

5.12.1.3.1	Introduction	321

5.12.1.3.2	Resource Definition	321

5.12.1.3.3	Resource Methods	321

5.12.1.3.3.1	General	321

5.12.1.3.3.2	GET	322

5.12.1.3.3.3	PUT	322

5.12.1.3.3.3A	PATCH	323

5.12.1.3.3.4	DELETE	324

5.12.1A	Custom Operations without associated resources	325

5.12.1B	Notifications	325

5.12.2	Data Model	325

5.12.2.1	General	325

5.12.2.2	Reused data types	326

5.12.2.3	Structured data types	326

5.12.2.3.1	Introduction	326

5.12.2.3.2	Type: AcsConfigurationData	326

5.12.2.3.3	Type: AcsConfigurationDataPatch	327

5.12.2.4	Simple data types and enumerations	327

5.12.2.4.1	Introduction	327

5.12.2.4.2	Simple data types	327

5.12.3	Used Features	327

5.12.4	Error handling	327

5.12.4.1	General	327

5.12.4.2	Protocol Errors	327

5.12.4.3	Application Errors	327

5.13	MoLcsNotify API	328

5.13.0	Introduction	328

5.13.1	Resources	328

5.13.1A	Custom Operations without associated resources	328

5.13.2	Notifications	328

5.13.2.1	Introduction	328

5.13.2.2	Event Notification	328

5.13.2.3	Operation Definition	329

5.13.2.3.1	Notification via HTTP POST	329

5.13.3	Data Model	329

5.13.3.1	General	329

5.13.3.2	Reused data types	329

5.13.3.3	Structured data types	330

5.13.3.3.1	Introduction	330

5.13.3.3.2	Type: LocUpdateData	330

5.13.3.3.3	Type: LocUpdateDataReply	330

5.13.3.4	Simple data types and enumerations	330

5.13.3.4.1	Introduction	330

5.13.3.4.2	Simple data types	331

5.13.4	Used Features	331

5.13.5	Error handling	331

5.13.5.1	General	331

5.13.5.2	Protocol Errors	331

5.13.5.3	Application Errors	331

5.14	AKMA API	331

5.14.1	Introduction	331

5.14.2	Resources	332

5.14.3	Custom Operations without associated resources	332

5.14.3.1	Overview	332

5.14.3.2	Operation: Retrieve	332

5.14.3.2.1	Description	332

5.14.3.2.2	Operation Definition	332

5.14.4	Notifications	333

5.14.4.1	General	333

5.14.4.2	AKMA Service Disablement Notification	333

5.14.4.2.1	Description	333

5.14.4.2.2	Target URI	334

5.14.4.2.3	Operation Definition	334

5.14.5	Data Model	335

5.14.5.1	General	335

5.14.5.2	Reused data types	335

5.14.5.3	Structured data types	335

5.14.5.3.1	Introduction	335

5.14.5.3.2	Type: AkmaAfKeyRequest	336

5.14.5.3.3	Type: AkmaAfKeyData	336

5.14.5.3.4	Type: ServiceDisableNotif	336

5.14.5.4	Simple data types and enumerations	337

5.14.5.4.1	Introduction	337

5.14.5.4.2	Simple data types	337

5.14.6	Used Features	337

5.14.7	Error handling	337

5.14.7.1	General	337

5.14.7.2	Protocol Errors	338

5.14.7.3	Application Errors	338

5.15	TimeSyncExposure API	338

5.15.0	Introduction	338

5.15.1	Resources	338

5.15.1.1	Overview	338

5.15.1.2	Resource: Time Synchronization Exposure Subscriptions	339

5.15.1.2.1	Introduction	339

5.15.1.2.2	Resource Definition	340

5.15.1.2.3	Resource Methods	340

5.15.1.2.3.1	General	340

5.15.1.2.3.2	GET	340

5.15.1.2.3.3	POST	341

5.15.1.3	Resource: Individual Time Synchronization Exposure Subscription	342

5.15.1.3.1	Introduction	342

5.15.1.3.2	Resource Definition	342

5.15.1.3.3	Resource Methods	342

5.15.1.3.3.1	General	342

5.15.1.3.3.2	GET	342

5.15.1.3.3.3	PUT	343

5.15.1.3.3.4	DELETE	344

5.15.1.4	Resource: Time Synchronization Exposure Configurations	345

5.15.1.4.1	Introduction	345

5.15.1.4.2	Resource Definition	345

5.15.1.4.3	Resource Methods	346

5.15.1.4.3.1	General	346

5.15.1.4.3.2	GET	346

5.15.1.4.3.3	POST	347

5.15.1.5	Resource: Individual Time Synchronization Exposure Configuration	348

5.15.1.5.1	Introduction	348

5.15.1.5.2	Resource Definition	348

5.15.1.5.3	Resource Methods	348

5.15.1.5.3.1	General	348

5.15.1.5.3.2	GET	348

5.15.1.5.3.3	PUT	349

5.15.1.5.3.4	DELETE	350

5.15.2	Custom Operations without associated resources	351

5.15.3	Notifications	351

5.15.3.1	Introduction	351

5.15.3.2	Time Synchronization Capability Notification	352

5.15.3.2.1	Description	352

5.15.3.2.2	Callback URI	352

5.15.3.2.3	Operation Definition	352

5.15.3.2.3.1	Notification via HTTP POST	352

5.15.3.2.3.2	Notification via Websocket	353

5.15.3.3	Time Synchronization Configuration Notification	353

5.15.3.3.1	Description	353

5.15.3.3.2	Callback URI	353

5.15.3.3.3	Operation Definition	353

5.15.3.3.3.1	Notification via HTTP POST	353

5.15.3.3.3.2	Void	354

5.15.4	Data Model	354

5.15.4.1	General	354

5.15.4.2	Reused data types	355

5.15.4.3	Structured data types	356

5.15.4.3.1	Introduction	356

5.15.4.3.2	Type: TimeSyncExposureSubsc	357

5.15.4.3.3	Type: TimeSyncCapability	359

5.15.4.3.4	Void	360

5.15.4.3.5	Void	360

5.15.4.3.6	Type: TimeSyncExposureConfig	360

5.15.4.3.7	Type: TimeSyncExposureSubsNotif	362

5.15.4.3.8	Type SubsEventNotification	362

5.15.4.3.9	Type: TimeSyncExposureConfigNotif	362

5.15.4.3.10	Type: EventFilter	362

5.15.4.3.11	Type: PtpCapabilitiesPerUe	363

5.15.4.3.12	Type: PtpInstance	363

5.15.4.3.13	Void	364

5.15.4.3.14	Void	364

5.15.4.3.15	Void	364

5.15.4.3.16	Void	364

5.15.4.3.17	Type: StateOfConfiguration	364

5.15.4.3.18	Type: ConfigForPort	365

5.15.4.3.19	Type: StateOfDstt	367

5.15.4.3.20	Void	368

5.15.4.3.21	Void	368

5.15.4.3.22	Type: AvailReport	368

5.15.4.4	Simple data types and enumerations	368

5.15.4.4.1	Introduction	368

5.15.4.4.2	Simple data types	368

5.15.4.4.3	Void	368

5.15.4.4.4	Enumeration: Protocol	368

5.15.4.4.5	Enumeration: GmCapable	368

5.15.4.4.6	Enumeration: SubscribedEvent	369

5.15.4.4.7	Enumeration: InstanceType	369

5.15.4.4.8	Enumeration: AsTimeResource	369

5.15.4.4.9	Enumeration: AcceptanceCriteriaResultIndication	369

5.15.4.4.10	Enumeration: AvailStatus	370

5.15.4.5	Data types describing alternative data types or combinations of data types	370

5.15.4.5.1	Type: ReportedCabability	370

5.15.5	Used Features	370

5.15.6	Error handling	370

5.15.6.1	General	370

5.15.6.2	Protocol Errors	370

5.15.6.3	Application Errors	371

5.16	EcsAddressProvision API	371

5.16.0	Introduction	371

5.16.1	Resources	371

5.16.1.1	Overview	371

5.16.1.2	Resource: ECS Address Provision Configurations	372

5.16.1.2.1	Introduction	372

5.16.1.2.2	Resource Definition	372

5.16.1.2.3	Resource Methods	372

5.16.1.2.3.1	General	372

5.16.1.2.3.2	GET	372

5.16.1.2.3.3	POST	373

5.16.1.3	Resource: Individual ECS Address Provision Configuration	374

5.16.1.3.1	Introduction	374

5.16.1.3.2	Resource Definition	374

5.16.1.3.3	Resource Methods	374

5.16.1.3.3.1	General	374

5.16.1.3.3.2	GET	374

5.16.1.3.3.3	PUT	375

5.16.1.3.3.4	DELETE	376

5.16.1A	Custom Operations without associated resources	377

5.16.1B	Notifications	377

5.16.2	Data Model	377

5.16.2.1	General	377

5.16.2.2	Reused data types	377

5.16.2.3	Structured data types	378

5.16.2.3.1	Introduction	378

5.16.2.3.2	Type: EcsAddressProvision	378

5.16.2.4	Simple data types and enumerations	378

5.16.2.4.1	Introduction	378

5.16.2.4.2	Simple data types	379

5.16.3	Used Features	379

5.16.4	Error handling	379

5.16.4.1	General	379

5.16.4.2	Protocol Errors	379

5.16.4.3	Application Errors	379

5.17	AMPolicyAuthorization API	379

5.17.0	Introduction	379

5.17.1	Resources	380

5.17.1.1	Overview	380

5.17.1.2	Resource: Application AM Contexts	381

5.17.1.2.1	Introduction	381

5.17.1.2.2	Resource Definition	381

5.17.1.2.3	Resource Methods	381

5.17.1.2.3.1	General	381

5.17.1.2.3.2	POST	381

5.17.1.3	Resource: Individual Application AM Context	382

5.17.1.3.1	Introduction	382

5.17.1.3.2	Resource Definition	382

5.17.1.3.3	Resource Methods	382

5.17.1.3.3.1	General	382

5.17.1.3.3.2	GET	382

5.17.1.3.3.3	PATCH	383

5.17.1.3.3.4	DELETE	385

5.17.1.4	Resource: AM Policy Events Subscription	386

5.17.1.4.1	Introduction	386

5.17.1.4.2	Resource Definition	386

5.17.1.4.3	Resource Methods	386

5.17.1.4.3.1	General	386

5.17.1.4.3.2	PUT	386

5.17.1.4.3.3	DELETE	388

5.17.1A	Custom Operations without associated resources	388

5.17.2	Notifications	389

5.17.2.1	Introduction	389

5.17.2.2	AM Event Notification	389

5.17.2.2.1	Description	389

5.17.2.2.2	Callback URI	389

5.17.2.2.3	Operation Definition	389

5.17.2.2.3.1	Notification via HTTP POST	389

5.17.2.2.3.2	Notification via Websocket	390

5.17.3	Data Model	390

5.17.3.1	General	390

5.17.3.2	Reused data types	391

5.17.3.3	Structured data types	392

5.17.3.3.1	Introduction	392

5.17.3.3.2	Type: AppAmContextExpData	392

5.17.3.3.3	Type: AppAmContextExpUpdateData	393

5.17.3.3.4	Type: GeographicalArea	393

5.17.3.3.5	Type: SliceReplReq	393

5.17.3.4	Simple data types and enumerations	394

5.17.3.4.1	Introduction	394

5.17.3.4.2	Simple data types	394

5.17.3.5	Data types describing alternative data types or combinations of data types	394

5.17.3.5.1	Type: AppAmContextExpRespData	394

5.17.4	Used Features	394

5.17.5	Error handling	394

5.17.5.1	General	394

5.17.5.2	Protocol Errors	395

5.17.5.3	Application Errors	395

5.18	AMInfluence API	395

5.18.0	Introduction	395

5.18.1	Resources	395

5.18.1.1	Overview	395

5.18.1.2	Resource: AM Influence Subscription	396

5.18.1.2.1	Introduction	396

5.18.1.2.2	Resource Definition	396

5.18.1.2.3	Resource Methods	396

5.18.1.2.3.1	General	396

5.18.1.2.3.2	GET	397

5.18.1.2.3.3	POST	397

5.18.1.3	Resource: Individual AM Influence Subscription	398

5.18.1.3.1	Introduction	398

5.18.1.3.2	Resource Definition	398

5.18.1.3.3	Resource Methods	398

5.18.1.3.3.1	General	398

5.18.1.3.3.2	GET	398

5.18.1.3.3.3	PUT	399

5.18.1.3.3.4	PATCH	400

5.18.1.3.3.5	DELETE	401

5.18.1A	Custom Operations without associated resources	402

5.18.2	Notifications	402

5.18.2.1	Introduction	402

5.18.2.2	Event Notification	402

5.18.2.2.1	Description	402

5.18.2.2.2	Target URI	402

5.18.2.2.3	Operation Definition	403

5.18.3	Data Model	404

5.18.3.1	General	404

5.18.3.2	Reused data types	404

5.18.3.3	Structured data types	404

5.18.3.3.1	Introduction	404

5.18.3.3.2	Type: AmInfluSub	405

5.18.3.3.3	Type: AmInfluSubPatch	408

5.18.3.3.4	Type: AmInfluEventNotif	410

5.18.3.3.5	Type: DnnSnssaiInformation	410

5.18.3.4	Simple data types and enumerations	410

5.18.3.4.1	Introduction	410

5.18.3.4.2	Simple data types	410

5.18.3.4.3	Enumeration: AmInfluEvent	410

5.18.4	Used Features	411

5.18.5	Error handling	411

5.18.5.1	General	411

5.18.5.2	Protocol Errors	411

5.18.5.3	Application Errors	411

5.19	MBSTMGI API	411

5.19.1	Introduction	411

5.19.2	Resources	412

5.19.3	Custom Operations without associated resources	412

5.19.3.1	Overview	412

5.19.3.2	Operation: Allocate	412

5.19.3.2.1	Description	412

5.19.3.2.2	Operation Definition	413

5.19.3.3	Operation: Deallocate	413

5.19.3.3.1	Description	413

5.19.3.3.2	Operation Definition	413

5.19.4	Notifications	414

5.19.4.1	General	414

5.19.4.2	Notification of Allocated MBS TMGI(s) Timer Expiry	415

5.19.4.2.1	Description	415

5.19.4.2.2	Target URI	415

5.19.4.2.3	Operation Definition	415

5.19.4.2.3.1	Notification via HTTP POST	415

5.19.4.2.3.2	Notification via Websocket	416

5.19.5	Data Model	416

5.19.5.1	General	416

5.19.5.2	Structured data types	417

5.19.5.2.1	Introduction	417

5.19.5.2.2	Type: TmgiAllocRequest	418

5.19.5.2.3	Type: TmgiAllocResponse	419

5.19.5.2.4	Type: TmgiDeallocRequest	419

5.19.5.2.5	Type: ExpiryNotif	419

5.19.5.2.6	Type: ReducedMbsServArea	420

5.19.5.3	Simple data types and enumerations	420

5.19.5.3.1	Introduction	420

5.19.5.3.2	Simple data types	420

5.19.5.4	Data types describing alternative data types or combinations of data types	421

5.19.5.4.1	Type: ProblemDetailsTmgiAlloc	421

5.19.6	Used Features	421

5.19.7	Error handling	421

5.19.7.1	General	421

5.19.7.2	Protocol Errors	421

5.19.7.3	Application Errors	421

5.20	MBSSession API	422

5.20.1	Introduction	422

5.20.2	Resources	422

5.20.2.1	Overview	422

5.20.2.2	Resource: MBS sessions	424

5.20.2.2.1	Introduction	424

5.20.2.2.2	Resource Definition	424

5.20.2.2.3	Resource Methods	425

5.20.2.2.3.1	POST	425

5.20.2.2.4	Resource Custom Operations	425

5.20.2.3	Resource: Individual MBS Session	425

5.20.2.3.1	Introduction	425

5.20.2.3.2	Resource Definition	425

5.20.2.3.3	Resource Standard Methods	426

5.20.2.3.3.1	PATCH	426

5.20.2.3.3.3	DELETE	427

5.20.2.3.4	Resource Custom Operations	428

5.20.2.4	Resource: MBS Session Subscriptions	428

5.20.2.4.1	Introduction	428

5.20.2.4.2	Resource Definition	428

5.20.2.4.3	Resource Methods	428

5.20.2.4.3.1	GET	428

5.20.2.4.3.2	POST	429

5.20.2.4.4	Resource Custom Operations	430

5.20.2.5	Resource: Individual MBS Session Subscription	430

5.20.2.5.1	Introduction	430

5.20.2.5.2	Resource Definition	430

5.20.2.5.3	Resource Methods	430

5.20.2.5.3.1	GET	430

5.20.2.5.3.2	DELETE	431

5.20.2.5.4	Resource Custom Operations	432

5.20.2.6	Resource: MBS Parameters Provisionings	432

5.20.2.6.1	Introduction	432

5.20.2.6.2	Resource Definition	432

5.20.2.6.3	Resource Methods	433

5.20.2.6.3.1	GET	433

5.20.2.6.3.2	POST	434

5.20.2.6.4	Resource Custom Operations	434

5.20.2.7	Resource: Individual MBS Parameters Provisioning	434

5.20.2.7.1	Introduction	434

5.20.2.7.2	Resource Definition	435

5.20.2.7.3	Resource Methods	435

5.20.2.7.3.1	GET	435

5.20.2.7.3.2	PUT	436

5.20.2.7.3.3	PATCH	437

5.20.2.7.3.4	DELETE	438

5.20.2.7.4	Resource Custom Operations	439

5.20.3	Custom Operations without associated resources	439

5.20.4	Notifications	439

5.20.4.1	General	439

5.20.4.2	MBS Session Status Notification	439

5.20.4.2.1	Description	439

5.20.4.2.2	Target URI	439

5.20.4.2.3	Operation Definition	440

5.20.4.2.3.1	Notification via HTTP POST	440

5.20.5	Data Model	440

5.20.5.1	General	440

5.20.5.2	Structured data types	441

5.20.5.2.1	Introduction	441

5.20.5.2.2	Type: MbsSessionCreateReq	442

5.20.5.2.3	Type: MbsSessionCreateRsp	442

5.20.5.2.4	Type: MbsSessionSubsc	442

5.20.5.2.5	Type: MbsSessionStatusNotif	442

5.20.5.2.6	Type: MbsPpData	443

5.20.5.2.7	Type: MbsSessAuthData	443

5.20.5.2.8	Type: MbsPpDataPatch	444

5.20.5.2.9	Type: MbsSessAssistInfo	444

5.20.5.2.10	Type: MbsSessionUpdateResp	444

5.20.5.3	Simple data types and enumerations	444

5.20.5.3.1	Introduction	444

5.20.5.3.2	Simple data types	445

5.20.6	Used Features	445

5.20.7	Error handling	445

5.20.7.1	General	445

5.20.7.2	Protocol Errors	445

5.20.7.3	Application Errors	445

5.21	EASDeployment API	446

5.21.0	Introduction	446

5.21.1	Resources	446

5.21.1.1	Overview	446

5.21.1.2	Resource: EAS Deployment Information	447

5.21.1.2.1	Introduction	447

5.21.1.2.2	Resource Definition	447

5.21.1.2.3	Resource Methods	447

5.21.1.2.3.1	General	447

5.21.1.2.3.2	GET	448

5.21.1.2.3.3	POST	448

5.21.1.3	Resource: Individual EAS Deployment Information	449

5.21.1.3.1	Introduction	449

5.21.1.3.2	Resource Definition	449

5.21.1.3.3	Resource Methods	449

5.21.1.3.3.1	General	449

5.21.1.3.3.2	GET	449

5.21.1.3.3.3	PUT	450

5.21.1.3.3.4	DELETE	451

5.21.2	Custom Operations without associated resources	452

5.21.2.1	Overview	452

5.21.2.2	Operation: remove-edis	453

5.21.2.2.1	Description	453

5.21.2.2.2	Operation Definition	453

5.21.3	Notifications	453

5.21.4	Data Model	453

5.21.4.1	General	453

5.21.4.2	Reused data types	454

5.21.4.3	Structured data types	454

5.21.4.3.1	Introduction	454

5.21.4.3.2	Type: EasDeployInfo	455

5.21.4.3.3	Type: DnaiInformation	455

5.21.4.3.4	Type: DnsServerIdentifier	456

5.21.4.3.5	Type: EdiDeleteCriteria	456

5.21.4.4	Simple data types and enumerations	457

5.21.4.4.1	Introduction	457

5.21.4.4.2	Simple data types	457

5.21.5	Used Features	457

5.21.6	Error handling	457

5.21.6.1	General	457

5.21.6.2	Protocol Errors	457

5.21.6.3	Application Errors	457

5.22	ASTI API	458

5.22.0	Introduction	458

5.22.1	Resources	458

5.22.1.1	Overview	458

5.22.1.2	Resource: ASTI Configurations	459

5.22.1.2.1	Introduction	459

5.22.1.2.2	Resource Definition	459

5.22.1.2.3	Resource Methods	459

5.22.1.2.3.1	General	459

5.22.1.2.3.2	GET	459

5.22.1.2.3.3	POST	460

5.22.1.2.4	Resource Custom Operations	461

5.22.1.2.4.1	Overview	461

5.22.1.2.4.2	Operation: retrieve	461

5.22.1.2.4.2.1	Description	461

5.22.1.2.4.2.2	Operation Definition	461

5.22.1.3	Resource: Individual ASTI Configuration	462

5.22.1.3.1	Introduction	462

5.22.1.3.2	Resource Definition	462

5.22.1.3.3	Resource Methods	462

5.22.1.3.3.1	General	462

5.22.1.3.3.2	GET	462

5.22.1.3.3.3	PUT	463

5.22.1.3.3.4	DELETE	464

5.22.2	Custom Operations without associated resources	465

5.22.3	Notifications	465

5.22.3.1	General	465

5.22.3.2	ASTI Notification	465

5.22.3.2.1	Description	465

5.22.3.2.2	Target URI	465

5.22.3.2.3	Operation Definition	465

5.22.3.2.3.1	Notification via HTTP POST	465

5.22.3.2.3.2	Void	466

5.22.4	Data Model	466

5.22.4.1	General	466

5.22.4.2	Reused data types	467

5.22.4.3	Structured data types	467

5.22.4.3.1	Introduction	467

5.22.4.3.2	Type: AccessTimeDistributionData	468

5.22.4.3.3	Type: StatusRequestData	468

5.22.4.3.4	Type: StatusResponseData	469

5.22.4.3.5	Type: ActiveUe	469

5.22.4.3.6	Type AstiConfigNotification	469

5.22.4.3.7	Type AstiConfigStateNotification	469

5.22.4.4	Simple data types and enumerations	469

5.22.4.4.1	Introduction	469

5.22.4.4.2	Simple data types	470

5.22.5	Used Features	470

5.22.6	Error handling	470

5.22.6.1	General	470

5.22.6.2	Protocol Errors	470

5.22.6.3	Application Errors	470

5.23	DataReporting API	471

5.23.1	Introduction	471

5.23.2	Resources	471

5.23.2.1	Overview	471

5.23.2.2	Resource: Data Reporting Sessions	472

5.23.2.2.1	Introduction	472

5.23.2.2.2	Resource definition	472

5.23.2.2.3	Resource Methods	472

5.23.2.2.3.1	POST	472

5.23.2.3	Resource: Individual Data Reporting Session	473

5.23.2.3.1	Introduction	473

5.23.2.3.2	Resource Definition	473

5.23.2.3.3	Resource standard methods	473

5.23.2.3.3.1	GET	473

5.23.2.3.3.2	PUT	474

5.23.2.3.3.3	DELETE	476

5.23.2.3.4	Resource custom operations	477

5.23.3	Custom Operations without associated resources	478

5.23.4	Notifications	478

5.23.5	Data Model	478

5.23.3.1	General	478

5.23.6	Used Features	478

5.23.7	Error handling	478

5.23.7.1	General	478

5.23.7.2	Protocol Errors	478

5.23.7.3	Application Errors	478

5.24	DataReportingProvisioning API	479

5.24.1	Introduction	479

5.24.2	Resources	479

5.24.2.1	Overview	479

5.24.2.2	Resource: Data Reporting Provisioning Sessions	481

5.24.2.2.1	Introduction	481

5.24.2.2.2	Resource definition	482

5.24.2.2.3	Resource Methods	482

5.24.2.2.3.1	POST	482

5.24.2.3	Resource: Individual Data Reporting Provisioning Session	483

5.24.2.3.1	Introduction	483

5.24.2.3.2	Resource Definition	483

5.24.2.3.3	Resource standard methods	483

5.24.2.3.3.1	GET	483

5.24.2.3.3.2	Void	484

5.24.2.3.3.3	DELETE	484

5.24.2.4	Resource: Data Reporting Configurations	485

5.24.2.4.1	Introduction	485

5.24.2.4.2	Resource definition	486

5.24.2.4.3	Resource Methods	486

5.24.2.4.3.1	POST	486

5.24.2.5	Resource: Individual Data Reporting Configuration	488

5.24.2.5.1	Introduction	488

5.24.2.5.2	Resource Definition	488

5.24.2.5.3	Resource standard methods	488

5.24.2.5.3.2	GET	488

5.24.2.5.3.3	PUT	489

5.24.2.5.3.3A	PATCH	491

5.24.2.5.3.4	DELETE	492

5.24.3	Custom Operations without associated resources	492

5.24.4	Notifications	493

5.24.5	Data Model	493

5.24.5.1	General	493

5.24.6	Used Features	493

5.24.7	Error handling	493

5.24.7.1	General	493

5.24.7.2	Protocol Errors	493

5.24.7.3	Application Errors	494

5.25	UEId API	494

5.25.1	Introduction	494

5.25.2	Resources	494

5.25.2.1	Overview	494

5.25.2.2	Resource: UE ID Mapping Information Provisionings	495

5.25.2.2.1	Introduction	495

5.25.2.2.2	Resource Definition	495

5.25.2.2.3	Resource Methods	495

5.25.2.2.3.1	GET	495

5.25.2.2.3.2	POST	496

5.25.2.2.4	Resource Custom Operations	497

5.25.2.3	Resource: Individual UE ID Mapping Provisioning	497

5.25.2.3.1	Introduction	497

5.25.2.3.2	Resource Definition	497

5.25.2.3.3	Resource Methods	498

5.25.2.3.3.1	GET	498

5.25.2.3.3.2	PUT	499

5.25.2.3.3.3	PATCH	500

5.25.2.3.3.4	DELETE	501

5.25.3	Custom Operations without associated resources	502

5.25.3.1	Overview	502

5.25.3.2	Operation: Retrieve	503

5.25.3.2.1	Description	503

5.25.3.2.2	Operation Definition	503

5.25.3.2.3	Void	504

5.25.3.3	Operation: GetMsisdn	504

5.25.3.3.1	Description	504

5.25.3.3.2	Operation Definition	504

5.25.3.4	Operation: VerifyMsisdn	505

5.25.3.4.1	Description	505

5.25.3.4.2	Operation Definition	505

5.25.4	Notifications	506

5.25.5	Data Model	506

5.25.5.1	General	506

5.25.5.2	Structured data types	507

5.25.5.2.1	Introduction	507

5.25.5.2.2	Type: UeIdReq	508

5.25.5.2.3	Type: UeIdInfo	508

5.25.5.2.4	Type: MsisdnReq	509

5.25.5.2.5	Type: MsisdnInfo	509

5.25.5.2.6	Type: UeIdMappingInfo	510

5.25.5.2.7	Type: RangSlUeIdMappInfo	510

5.25.5.2.8	Type: UeIdMappingInfoPatch	510

5.25.5.2.9	Type: MsisdnVerifReq	511

5.25.5.2.10	Type: MsisdnVerifResp	511

5.25.5.3	Simple data types and enumerations	512

5.25.5.3.1	Introduction	512

5.25.5.3.2	Simple data types	512

5.25.6	Used Features	512

5.25.7	Error handling	512

5.25.7.1	General	512

5.25.7.2	Protocol Errors	512

5.25.7.3	Application Errors	512

5.25.8	Security	513

5.26	MBSUserService API	513

5.26.1	Introduction	513

5.26.2	Resources	513

5.26.2.1	Overview	513

5.26.2.2	Resource: MBS User Services	514

5.26.2.2.1	Introduction	514

5.26.2.2.2	Resource Definition	514

5.26.2.2.3	Resource Standard Methods	514

5.26.2.2.3.1	GET	514

5.26.2.2.3.2	POST	515

5.26.2.2.4	Resource Custom Operations	516

5.26.2.3	Resource: Individual MBS User Service	516

5.26.2.3.1	Introduction	516

5.26.2.3.2	Resource Definition	516

5.26.2.3.3	Resource Standard Methods	516

5.26.2.3.3.1	GET	516

5.26.2.3.3.2	PUT	517

5.26.2.3.3.3	PATCH	518

5.26.2.3.3.4	DELETE	519

5.26.2.3.4	Resource Custom Operations	520

5.26.3	Custom Operations without associated resources	520

5.26.4	Notifications	520

5.26.5	Data Model	520

5.26.5.1	General	520

5.26.5.2	Structured data types	521

5.26.5.2.1	Introduction	521

5.26.5.3	Simple data types and enumerations	521

5.26.5.3.1	Introduction	521

5.26.5.3.2	Simple data types	521

5.26.6	Used Features	521

5.26.7	Error handling	522

5.26.7.1	General	522

5.26.7.2	Protocol Errors	522

5.26.7.3	Application Errors	522

5.27	MBSUserDataIngestSession API	522

5.27.1	Introduction	522

5.27.2	Resources	522

5.27.2.1	Overview	522

5.27.2.2	Resource: MBS User Data Ingest Sessions	524

5.27.2.2.1	Introduction	524

5.27.2.2.2	Resource Definition	524

5.27.2.2.3	Resource Standard Methods	524

5.27.2.2.3.1	GET	524

5.27.2.2.3.2	POST	525

5.27.2.2.4	Resource Custom Operations	526

5.27.2.3	Resource: Individual MBS User Data Ingest Session	526

5.27.2.3.1	Introduction	526

5.27.2.3.2	Resource Definition	526

5.27.2.3.3	Resource Standard Methods	526

5.27.2.3.3.1	GET	526

5.27.2.3.3.2	PUT	527

5.27.2.3.3.3	PATCH	528

5.27.2.3.3.4	DELETE	530

5.27.2.3.4	Resource Custom Operations	530

5.27.2.4	Resource: MBS User Data Ingest Session Status Subscriptions	531

5.27.2.4.1	Introduction	531

5.27.2.4.2	Resource Definition	531

5.27.2.4.3	Resource Standard Methods	531

5.27.2.4.3.1	GET	531

5.27.2.4.3.2	POST	532

5.27.2.4.4	Resource Custom Operations	533

5.27.2.5	Resource: Individual MBS User Data Ingest Session Status Subscription	533

5.27.2.5.1	Introduction	533

5.27.2.5.2	Resource Definition	533

5.27.2.5.3	Resource Standard Methods	533

5.27.2.5.3.1	GET	533

5.27.2.5.3.2	PUT	534

5.27.2.5.3.3	PATCH	535

5.27.2.5.3.4	DELETE	536

5.27.2.5.4	Resource Custom Operations	537

5.27.3	Custom Operations without associated resources	537

5.27.4	Notifications	537

5.27.4.1	General	537

5.27.4.2	MBS User Data Ingest Session Status Change Notification	538

5.27.4.2.1	Description	538

5.27.4.2.2	Target URI	538

5.27.4.2.3	Operation Definition	538

5.27.4.2.3.1	Notification via HTTP POST	538

5.27.4.2.3.2	Void	539

5.27.5	Data Model	539

5.27.5.1	General	539

5.27.5.2	Structured data types	540

5.27.5.2.1	Introduction	540

5.27.5.3	Simple data types and enumerations	540

5.27.5.3.1	Introduction	540

5.27.5.3.2	Simple data types	540

5.27.6	Used Features	540

5.27.7	Error handling	541

5.27.7.1	General	541

5.27.7.2	Protocol Errors	541

5.27.7.3	Application Errors	541

5.28	MSEventExposure API	542

5.28.1	Introduction	542

5.28.2	Resources	543

5.28.2.1	Overview	543

5.28.2.2	Resource: Media Streaming Event Exposure Subscriptions	543

5.28.2.2.1	Introduction	543

5.28.2.2.2	Resource Definition	544

5.28.2.2.3	Resource Standard Methods	544

5.28.2.2.3.1	GET	544

5.28.2.2.3.2	POST	545

5.28.2.2.4	Resource Custom Operations	545

5.28.2.3	Resource: Individual Media Streaming Event Exposure Subscription	545

5.28.2.3.1	Introduction	545

5.28.2.3.2	Resource Definition	546

5.28.2.3.3	Resource Standard Methods	546

5.28.2.3.3.1	GET	546

5.28.2.3.3.2	PUT	547

5.28.2.3.3.3	DELETE	548

5.28.2.3.4	Resource Custom Operations	549

5.28.3	Custom Operations without associated resources	549

5.28.4	Notifications	549

5.28.4.1	General	549

5.28.4.2	Media Streaming Event Exposure Notification	549

5.28.4.2.1	Description	549

5.28.4.2.2	Target URI	549

5.28.4.2.3	Operation Definition	549

5.28.4.2.3.1	Notification via HTTP POST	549

5.28.4.2.3.2	Void	550

5.28.5	Data Model	550

5.28.5.1	General	550

5.28.5.2	Structured data types	551

5.28.5.2.1	Introduction	551

5.28.5.3	Simple data types and enumerations	551

5.28.5.3.1	Introduction	551

5.28.5.3.2	Simple data types	551

5.28.6	Used Features	551

5.28.7	Error handling	551

5.28.7.1	General	551

5.28.7.2	Protocol Errors	552

5.28.7.3	Application Errors	552

5.29	MBSGroupMsgDelivery API	552

5.29.1	Introduction	552

5.29.2	Resources	552

5.29.2.1	Overview	552

5.29.2.2	Resource: MBS Group Message Deliveries	553

5.29.2.2.1	Introduction	553

5.29.2.2.2	Resource Definition	553

5.29.2.2.3	Resource Standard Methods	554

5.29.2.2.3.1	GET	554

5.29.2.2.3.2	POST	555

5.29.2.2.4	Resource Custom Operations	555

5.29.2.3	Resource: Individual MBS Group Message Delivery	555

5.29.2.3.1	Introduction	555

5.29.2.3.2	Resource Definition	555

5.29.2.3.3	Resource Standard Methods	556

5.29.2.3.3.1	GET	556

5.29.2.3.3.2	PATCH	557

5.29.2.3.3.3	DELETE	558

5.29.2.3.4	Resource Custom Operations	559

5.29.3	Custom Operations without associated resources	559

5.29.4	Notifications	559

5.29.4.1	General	559

5.29.4.2	MBS Group Message Delivery Notification	559

5.29.4.2.1	Description	559

5.29.4.2.2	Target URI	559

5.29.4.2.3	Operation Definition	560

5.29.5	Data Model	560

5.29.5.1	General	560

5.29.5.2	Structured data types	561

5.29.5.2.1	Introduction	561

5.29.5.2.2	Void	562

5.29.5.2.3	Type: MbsGroupMsgDel	562

5.29.5.2.4	Void	563

5.29.5.2.5	Type: MbsGroupMsgDelPatch	563

5.29.5.2.6	Type: MbsGroupMsgDelStatusNotif	563

5.29.5.3	Simple data types and enumerations	563

5.29.5.3.1	Introduction	563

5.29.5.3.2	Simple data types	563

5.29.5.4	Data types describing alternative data types or combinations of data types	564

5.29.5.4.1	Type: MbsServArea	564

5.29.6	Used Features	564

5.29.7	Error handling	564

5.29.7.1	General	564

5.29.7.2	Protocol Errors	564

5.29.7.3	Application Errors	564

5.30	DNAIMapping API	564

5.30.2	Resources	565

5.30.2.1	Overview	565

5.30.2.2	Resource: DNAI Mapping Subscriptions	566

5.30.2.2.1	Introduction	566

5.30.2.2.2	Resource Definition	566

5.30.2.2.3	Resource Standard Methods	566

5.30.2.2.3.1	Void	566

5.30.2.2.3.2	GET	566

5.30.2.2.3.3	POST	567

5.30.2.2.4	Resource Custom Operations	568

5.30.2.3	Resource: Individual DNAI Mapping Subscription	568

5.30.2.3.1	Introduction	568

5.30.2.3.2	Resource Definition	568

5.30.2.3.3	Resource Standard Methods	568

5.30.2.3.3.1	Void	568

5.30.2.3.3.2	GET	568

5.30.2.3.3.3	DELETE	569

5.30.2.3.4	Resource Custom Operations	570

5.30.4	Notifications	571

5.30.4.1	General	571

5.30.4.2	DNAI Mapping Information Update Notification	571

5.30.4.2.1	Description	571

5.30.4.2.2	Target URI	571

5.30.4.2.3	Operation Definition	571

5.30.4.2.3.1	Notification via HTTP POST	571

5.30.4.2.3.2	Void	572

5.30.5	Data Model	572

5.30.5.1	General	572

5.30.5.2	Structured data types	573

5.30.5.2.1	Introduction	573

5.30.5.2.2	Type: DnaiMapSub	574

5.30.5.2.3	Type: DnaiMapUpdateNotif	575

5.30.5.2.4	Void	575

5.30.5.3	Simple data types and enumerations	575

5.30.5.3.1	Introduction	575

5.30.5.3.2	Simple data types	575

5.30.5.4	Data types describing alternative data types or combinations of data types	575

5.30.6	Used Features	575

5.30.7	Error handling	576

5.30.7.1	General	576

5.30.7.2	Protocol Errors	576

5.30.7.3	Application Errors	576

5.31	PdtqPolicyNegotiation API	576

5.31.0	Introduction	576

5.31.1	Resources	576

5.31.1.1	Overview	576

5.31.1.2	Resource: PDTQ Policy Subscriptions	577

5.31.1.2.1	Introduction	577

5.31.1.2.2	Resource Definition	577

5.31.1.2.3	Resource Standard Methods	578

5.31.1.2.3.1	Void	578

5.31.1.2.3.2	GET	578

5.31.1.2.3.3	POST	579

5.31.1.2.4	Resource Custom Operations	579

5.31.1.3	Resource: Individual PDTQ Policy Subscription	579

5.31.1.3.1	Introduction	579

5.31.1.3.2	Resource Definition	580

5.31.1.3.3	Resource Standard Methods	580

5.31.1.3.3.1	Void	580

5.31.1.3.3.2	GET	580

5.31.1.3.3.3	PATCH	581

5.31.1.3.3.4	DELETE	582

5.31.1.3.4	Resource Custom Operations	583

5.31.1A	Custom Operations without associated resources	583

5.31.2	Notifications	583

5.31.2.1	General	583

5.31.2.2	PDTQ Warning Notification	584

5.31.2.2.1	Description	584

5.31.2.2.2	Target URI	584

5.31.2.2.3	Operation Definition	584

5.31.2.2.3.1	Notification via HTTP POST	584

5.31.3	Data Model	585

5.31.3.1	General	585

5.31.3.2	Void	585

5.31.3.3	Structured data types	585

5.31.3.3.1	Introduction	585

5.31.3.3.2	Type: Pdtq	586

5.31.3.3.3	Type: PdtqPatch	588

5.31.3.3.4	Type: PdtqNotification	588

5.31.3.4	Simple data types and enumerations	588

5.31.3.4.1	Introduction	588

5.31.3.4.2	Simple data types	588

5.31.3.4	Data types describing alternative data types or combinations of data types	589

5.31.4	Used Features	589

5.31.5	Error handling	589

5.31.5.1	General	589

5.31.5.2	Protocol Errors	589

5.31.5.3	Application Errors	589

5.32	MemberUESelectionAssistance API	589

5.32.1	Introduction	589

5.32.2	Resources	590

5.32.2.1	Overview	590

5.32.2.2	Resource: Member UE Selection Assistance Subscriptions	591

5.32.2.2.1	Introduction	591

5.32.2.2.2	Resource definition	591

5.32.2.2.3	Resource Standard Methods	592

5.32.2.2.3.1	POST	592

5.32.2.2.3.2	GET	592

5.32.2.2.4	Resource Custom Operations	593

5.32.2.3	Resource: Individual Member UE Selection Assistance Subscription	593

5.32.2.3.1	Introduction	593

5.32.2.3.2	Resource Definition	594

5.32.2.3.3	Resource Standard Methods	594

5.32.2.3.3.1	GET	594

5.32.2.3.3.2	PUT	595

5.32.2.3.3.3	PATCH	596

5.32.2.3.3.4	DELETE	597

5.32.2.3.4	Resource Custom Operations	598

5.32.3	Custom Operations without associated resources	598

5.32.4	Notifications	598

5.32.5	Data Model	600

5.32.5.1	General	600

5.32.5.2	Structured data types	602

5.32.5.2.1	Introduction	602

5.32.5.2.2	Type: MemUeSelectAssistSubsc	603

5.32.5.2.3	Type: MemUeSeletAssistNotif	605

5.32.5.2.4	Type: QoSFilterCriteria	606

5.32.5.2.5	Type: AccessRatTypeFilterCriteria	606

5.32.5.2.6	Type: E2ETransTimeFilterCriteria	607

5.32.5.2.7	Type: UeLocFilterCriteria	607

5.32.5.2.8	Type: UeHisLocFilterCriteria	608

5.32.5.2.9	Type: UeDirectionFilterCriteria	608

5.32.5.2.10	Type: UeDistanceFilterCriteria	608

5.32.5.2.11	Type: ServiceExpFilterCriteria	609

5.32.5.2.12	Type: DnnFilterCriteria	609

5.32.5.2.13	Type: CandiUeListInfo	610

5.32.5.2.14	Type: MemUeSeletReport	610

5.32.5.2.15	Type: MemUeSelectAssistSubscPatch	611

5.32.5.2.16	Type: CandUeInfo	612

5.32.5.3	Simple data types and enumerations	612

5.32.5.3.1	Introduction	612

5.32.5.3.2	Simple data types	612

5.32.5.3.3	Enumeration: FilterCriterionType	613

5.32.5.4	Data types describing alternative data types or combinations of data types	613

5.32.6	Used Features	613

5.32.7	Error handling	613

5.32.7.1	General	613

5.32.7.2	Protocol Errors	613

5.32.7.3	Application Errors	613

5.33	GroupParametersProvisioning API	614

5.33.1	Introduction	614

5.33.2	Resources	614

5.33.2.1	Overview	614

5.33.2.2	Resource: Group Parameters Provisionings	615

5.33.2.2.1	Introduction	615

5.33.2.2.2	Resource Definition	615

5.33.2.2.3	Resource Methods	615

5.33.2.2.3.1	GET	615

5.33.2.2.3.2	POST	616

5.33.2.2.4	Resource Custom Operations	617

5.33.2.3	Resource: Individual Group Parameters Provisioning	617

5.33.2.3.1	Introduction	617

5.33.2.3.2	Resource Definition	617

5.33.2.3.3	Resource Methods	618

5.33.2.3.3.1	GET	618

5.33.2.3.3.2	PUT	619

5.33.2.3.3.3	PATCH	620

5.33.2.3.3.4	DELETE	621

5.33.2.3.4	Resource Custom Operations	622

5.33.3	Custom Operations without associated resources	622

5.33.4	Notifications	622

5.33.5	Data Model	622

5.33.5.1	General	622

5.33.5.2	Structured data types	623

5.33.5.2.1	Introduction	623

5.33.5.2.2	Type: GrpPpData	623

5.33.5.2.3	Type: GrpPpDataPatch	623

5.33.5.2.4	Type: DnnSnssaiGrpData	624

5.33.5.2.5	Type: AfReqDefaultQoS	624

5.33.5.2.6	Type: LadnServArea	624

5.33.5.3	Simple data types and enumerations	624

5.33.5.3.1	Introduction	624

5.33.5.3.2	Simple data types	625

5.33.5.4	Data types describing alternative data types or combinations of data types	625

5.33.6	Used Features	625

5.33.7	Error handling	625

5.33.7.1	General	625

5.33.7.2	Protocol Errors	625

5.33.7.3	Application Errors	625

5.34	SliceParamProvision API	625

5.34.1	Introduction	625

5.34.2	Resources	626

5.34.2.1	Overview	626

5.34.2.2	Resource: Slice Parameters Provisionings	627

5.34.2.2.1	Introduction	627

5.34.2.2.2	Resource Definition	627

5.34.2.2.3	Resource Methods	627

5.34.2.2.3.1	GET	627

5.34.2.2.3.2	POST	628

5.34.2.2.4	Resource Custom Operations	629

5.34.2.3	Resource: Individual Slice Parameters Provisioning	629

5.34.2.3.1	Introduction	629

5.34.2.3.2	Resource Definition	629

5.34.2.3.3	Resource Methods	630

5.34.2.3.3.1	GET	630

5.34.2.3.3.2	PUT	631

5.34.2.3.3.3	PATCH	632

5.34.2.3.3.4	DELETE	633

5.34.2.3.4	Resource Custom Operations	634

5.34.3	Custom Operations without associated resources	634

5.34.4	Notifications	634

5.34.5	Data Model	634

5.34.5.1	General	634

5.34.5.2	Structured data types	635

5.34.5.2.1	Introduction	635

5.34.5.2.2	Type: SlicePpData	635

5.34.5.2.3	Type: SlicePpDataPatch	635

5.34.5.3	Simple data types and enumerations	635

5.34.5.3.1	Introduction	635

5.34.5.3.2	Simple data types	635

5.34.6	Used Features	636

5.34.7	Error handling	636

5.34.7.1	General	636

5.34.7.2	Protocol Errors	636

5.34.7.3	Application Errors	636

5.35	UeAddress API	636

5.35.1	Introduction	636

5.35.2	Resources	637

5.35.2.1	Overview	637

5.35.2.2	Resource: UE Address Subscriptions	637

5.35.2.2.1	Introduction	637

5.35.2.2.2	Resource Definition	637

5.35.2.2.3	Resource Standard Methods	638

5.35.2.2.3.1	Void	638

5.35.2.2.3.2	GET	638

5.35.2.2.3.3	POST	639

5.35.2.2.4	Resource Custom Operations	640

5.35.2.3	Resource: Individual UE Address Subscription	640

5.35.2.3.1	Introduction	640

5.35.2.3.2	Resource Definition	640

5.35.2.3.3	Resource Standard Methods	640

5.35.2.3.3.1	Void	640

5.35.2.3.3.2	GET	640

5.35.2.3.3.3	DELETE	641

5.35.2.3.4	Resource Custom Operations	642

5.35.3	Custom Operations without associated resources	643

5.35.3.1	Overview	643

5.35.3.2	Operation: Retrieve	643

5.35.3.2.1	Description	643

5.35.3.2.2	Operation Definition	643

5.35.4	Notifications	644

5.35.4.1	General	644

5.35.4.2	UE Address Notification	645

5.35.4.2.1	Description	645

5.35.4.2.2	Target URI	645

5.35.4.2.3	Standard Methods	645

5.35.4.2.3.1	POST	645

5.35.5	Data Model	646

5.35.5.1	General	646

5.35.5.2	Structured data types	646

5.35.5.2.1	Introduction	646

5.35.5.2.2	Type: UeAddressReq	647

5.35.5.2.3	Type: UeAddressInfo	647

5.35.5.2.4	Type: UeAddrSubsc	648

5.35.5.2.5	Type: UeAddrInfo	649

5.35.5.2.6	Type: UeAddrNotif	649

5.35.5.2.7	Void	649

5.35.5.3	Simple data types and enumerations	649

5.35.5.3.1	Introduction	649

5.35.5.3.2	Simple data types	649

5.35.5.4	Data types describing alternative data types or combinations of data types	650

5.35.6	Used Features	650

5.35.7	Error handling	650

5.35.7.1	General	650

5.35.7.2	Protocol Errors	650

5.35.7.3	Application Errors	650

5.36	ECSAddress API	650

5.36.1	Introduction	650

5.36.2	Resources	651

5.36.2.1	Overview	651

5.36.2.2	Resource: ECS Address Configuration Information Sets	652

5.36.2.2.1	Introduction	652

5.36.2.2.2	Resource Definition	652

5.36.2.2.3	Resource Standard Methods	652

5.36.2.2.3.1	Void	652

5.36.2.2.3.2	GET	652

5.36.2.2.3.3	POST	653

5.36.2.2.4	Resource Custom Operations	654

5.36.2.3	Resource: Individual ECS Address Configuration Information Set	654

5.36.2.3.1	Introduction	654

5.36.2.3.2	Resource Definition	654

5.36.2.3.3	Resource Standard Methods	655

5.36.2.3.3.1	Void	655

5.36.2.3.3.2	GET	655

5.36.2.3.3.3	PUT	656

5.36.2.3.3.3A	PATCH	657

5.36.2.3.3.4	DELETE	658

5.36.2.3.4	Resource Custom Operations	659

5.36.3	Custom Operations without associated resources	659

5.36.3.1	Overview	659

5.36.3.2	Operation: RemoveEcsAddr	660

5.36.3.2.1	Description	660

5.36.3.2.2	Operation Definition	660

5.36.4	Notifications	661

5.36.5	Data Model	661

5.36.5.1	General	661

5.36.5.2	Void	662

5.36.5.3	Structured data types	662

5.36.5.3.1	Introduction	662

5.36.5.3.2	Type: EcsAddrInfo	662

5.36.5.3.3	Type: EcsAddrDeleteCriteria	663

5.36.5.3.4	Type: EcsAddrInfoPatch	663

5.36.5.4	Simple data types and enumerations	663

5.36.5.4.1	Introduction	663

5.36.5.4.2	Simple data types	664

5.36.5.4	Data types describing alternative data types or combinations of data types	664

5.36.6	Used Features	664

5.36.7	Error handling	664

5.36.7.1	General	664

5.36.7.2	Protocol Errors	664

5.36.7.3	Application Errors	664

5.37	RSLPPIParametersProvisioning API	664

5.37.1	Introduction	664

5.37.2	Resources	665

5.37.2.1	Overview	665

5.37.2.2	Resource: RSLPPI Parameters Provisionings	666

5.37.2.2.1	Introduction	666

5.37.2.2.2	Resource Definition	666

5.37.2.2.3	Resource Methods	666

5.37.2.2.3.1	GET	666

5.37.2.2.3.2	POST	667

5.37.2.2.4	Resource Custom Operations	668

5.37.2.3	Resource: Individual RSLPPI Parameters Provisioning	668

5.37.2.3.1	Introduction	668

5.37.2.3.2	Resource Definition	668

5.37.2.3.3	Resource Methods	669

5.37.2.3.3.1	GET	669

5.37.2.3.3.2	PUT	670

5.37.2.3.3.3	PATCH	671

5.37.2.3.3.4	DELETE	672

5.37.2.3.4	Resource Custom Operations	673

5.37.3	Custom Operations without associated resources	673

5.37.4	Notifications	673

5.37.5	Data Model	673

5.37.5.1	General	673

5.37.5.2	Structured data types	674

5.37.5.2.1	Introduction	674

5.37.5.2.2	Type: RslppiData	674

5.37.5.2.3	Type: RslppiPpData	674

5.37.5.2.4	Type: RslppiPpDataPatch	674

5.37.5.3	Simple data types and enumerations	674

5.37.5.3.1	Introduction	674

5.37.5.3.2	Simple data types	675

5.37.5.4	Data types describing alternative data types or combinations of data types	675

5.37.6	Used Features	675

5.37.7	Error handling	675

5.37.7.1	General	675

5.37.7.2	Protocol Errors	675

5.37.7.3	Application Errors	675

5.38	AddressingParamProvision API	675

5.38.1	Introduction	675

5.38.2	Resources	676

5.38.2.1	Overview	676

5.38.2.2	Resource: Addressing Parameters Provisionings	677

5.38.2.2.1	Introduction	677

5.38.2.2.2	Resource Definition	677

5.38.2.2.3	Resource Standard Methods	677

5.38.2.2.3.1	GET	677

5.38.2.2.3.2	POST	678

5.38.2.2.4	Resource Custom Operations	679

5.38.2.3	Resource: Individual Addressing Parameters Provisioning	679

5.38.2.3.1	Introduction	679

5.38.2.3.2	Resource Definition	679

5.38.2.3.3	Resource Standard Methods	680

5.38.2.3.3.1	GET	680

5.38.2.3.3.2	PUT	681

5.38.2.3.3.3	PATCH	682

5.38.2.3.3.4	DELETE	683

5.38.2.3.4	Resource Custom Operations	684

5.38.3	Custom Operations without associated resources	684

5.38.4	Notifications	684

5.38.5	Data Model	684

5.38.5.1	General	684

5.38.5.2	Structured data types	685

5.38.5.2.1	Introduction	685

5.38.5.2.2	Type: AddrPpData	685

5.38.5.2.3	Type: StaticIpAddrParams	685

5.38.5.2.4	Type: AddrPpDataPatch	685

5.38.5.3	Simple data types and enumerations	686

5.38.5.3.1	Introduction	686

5.38.5.3.2	Simple data types	686

5.38.5.4	Data types describing alternative data types or combinations of data types	686

5.38.5.5	Binary data	686

5.38.5.5.1	Binary Data Types	686

5.38.6	Used Features	686

5.38.7	Error handling	686

5.38.7.1	General	686

5.38.7.2	Protocol Errors	686

5.38.7.3	Application Errors	687

5.39	UAVFlightAssistance API	687

5.39.1	Introduction	687

5.39.2	Resources	687

5.39.2.1	Overview	687

5.39.2.2	Resource: UAV Flight Assistance Configurations	688

5.39.2.2.1	Introduction	688

5.39.2.2.2	Resource definition	689

5.39.2.2.3	Resource Standard Methods	689

5.39.2.2.3.0	GET	689

5.39.2.2.3.1	POST	690

5.39.2.2.4	Resource Custom Operations	691

5.39.2.3	Resource: Individual UAV Flight Assistance Configuration	691

5.39.2.3.1	Introduction	691

5.39.2.3.2	Resource Definition	691

5.39.2.3.3	Resource Standard Methods	691

5.39.2.3.3.0	GET	691

5.39.2.3.3.1	PUT	692

5.39.2.3.3.1A	PATCH	694

5.39.2.3.3.2	DELETE	695

5.39.2.3.4	Resource Custom Operations	696

5.39.3	Custom Operations without associated resources	696

5.39.4	Notifications	696

5.39.4.1	General	696

5.39.5	Data Model	697

5.39.5.1	General	697

5.39.5.2	Structured data types	698

5.39.5.2.1	Introduction	698

5.39.5.2.2	Type: UavFlightAssistance	699

5.39.5.2.3	Type: FlightPath	699

5.39.5.2.4	Type: FlightPathSegInfo	699

5.39.5.2.5	Type: UAVFlightAssistNotif	700

5.39.5.2.6	Type: UAVFlightAssistNotifResp	700

5.39.5.2.7	Type: UavFlightAssistancePatch	700

5.39.5.2.8	Type: AltReportReqs	701

5.39.5.2.9	Type: AltitudeReport	701

5.39.5.2.10	Type: AltRange	701

5.39.5.3	Simple data types and enumerations	701

5.39.5.3.1	Introduction	701

5.39.5.3.2	Simple data types	701

5.39.5.3.3	Enumeration: UavStatus	701

5.39.5.3.4	Enumeration: RequestPurpose	702

5.39.5.3.5	Enumeration: AltReportEvent	702

5.39.6	Used Features	702

5.39.7	Error handling	702

5.39.7.1	General	702

5.39.7.2	Protocol Errors	702

5.39.7.3	Application Errors	703

5.40	CagInfoParamProvision API	703

5.40.1	Introduction	703

5.40.2	Resources	703

5.40.2.1	Overview	703

5.40.2.2	Resource: CAG Information Parameters Provisionings	704

5.40.2.2.1	Introduction	704

5.40.2.2.2	Resource Definition	704

5.40.2.2.3	Resource Standard Methods	704

5.40.2.2.3.1	GET	704

5.40.2.2.3.2	POST	705

5.40.2.2.4	Resource Custom Operations	706

5.40.2.3	Resource: Individual CAG Information Parameters Provisioning	706

5.40.2.3.1	Introduction	706

5.40.2.3.2	Resource Definition	706

5.40.2.3.3	Resource Standard Methods	707

5.40.2.3.3.1	GET	707

5.40.2.3.3.2	PUT	708

5.40.2.3.3.3	PATCH	709

5.40.2.3.3.4	DELETE	710

5.40.2.3.4	Resource Custom Operations	711

5.40.3	Custom Operations without associated resources	711

5.40.4	Notifications	711

5.40.5	Data Model	712

5.40.5.1	General	712

5.40.5.2	Structured data types	712

5.40.5.2.1	Introduction	712

5.40.5.2.2	Type: CagInfoPpData	712

5.40.5.2.3	Type: CagInfoPpDataPatch	713

5.40.5.2.4	Type: CagInformation	713

5.40.5.3	Simple data types and enumerations	713

5.40.5.3.1	Introduction	713

5.40.5.3.2	Simple data types	713

5.40.5.5	Binary data	713

5.40.5.5.1	Binary Data Types	713

5.40.6	Used Features	713

5.40.7	Error handling	714

5.40.7.1	General	714

5.40.7.2	Protocol Errors	714

5.40.7.3	Application Errors	714

5.41	RetrieveInfoUAVFlight API	714

5.41.1	Introduction	714

5.41.2	Resources	714

5.41.3	Custom Operations without associated resources	714

5.41.3.1	Overview	714

5.41.3.2	Operation: Retrieve	715

5.41.3.2.1	Description	715

5.41.3.2.2	Operation Definition	715

5.41.4	Notifications	716

5.41.5	Data Model	716

5.41.5.1	General	716

5.41.5.2	Structured data types	717

5.41.5.2.1	Introduction	717

5.41.5.2.2	Type: RetrieveUAVFlightReq	717

5.41.5.2.3	CandidateFlightPathInfo	717

5.41.5.2.4	Type: RetrievedUAVFlightInfo	718

5.41.5.3	Simple data types and enumerations	718

5.41.5.3.1	Introduction	718

5.41.5.3.2	Simple data types	718

5.41.5.3.3	Enumeration: RetrievePurpose	718

5.41.5.4	Data types describing alternative data types or combinations of data types	718

5.41.6	Used Features	718

5.41.7	Error handling	719

5.41.7.1	General	719

5.41.7.2	Protocol Errors	719

5.41.7.3	Application Errors	719

5.42	ImsSessionManagement API	719

5.42.1	Introduction	719

5.42.2	Resources	720

5.42.2.1	Overview	720

5.42.2.2	Resource: IMS Sessions	720

5.42.2.2.1	Description	720

5.42.2.2.2	Resource Definition	720

5.42.2.2.3	Resource Standard Methods	721

5.42.2.2.3.1	POST	721

5.42.2.2.3.2	GET	721

5.42.2.2.4	Resource Custom Operations	722

5.42.2.3	Resource: Individual IMS Session	722

5.42.2.3.1	Description	722

5.42.2.3.2	Resource Definition	723

5.42.2.3.3	Resource Standard Methods	723

5.42.2.3.3.0A	GET	723

5.42.2.3.3.0B	PUT	724

5.42.2.3.3.1	PATCH	725

5.42.2.3.3.2	DELETE	726

5.42.2.3.4	Resource Custom Operations	727

5.42.3	Custom Operations without associated resources	727

5.42.4	Notifications	727

5.42.5	Data Model	729

5.42.5.1	General	729

5.42.6	Used Features	730

5.42.7	Error Handling	730

5.42.7.1	General	730

5.42.7.2	Protocol Errors	730

5.42.7.3	Application Errors	730

5.43	ImsEventExposure API	730

5.43.1	Introduction	730

5.43.2	Resources	731

5.43.2.1	Overview	731

5.43.2.2	Resource: IMS EE Subscriptions	732

5.43.2.2.1	Introduction	732

5.43.2.2.2	Resource definition	732

5.43.2.2.3	Resource Standard Methods	732

5.43.2.2.3.1	POST	732

5.43.2.2.3.2	GET	733

5.43.2.2.4	Resource Custom Operations	734

5.43.2.3	Resource: Individual IMS EE Subscription	734

5.43.2.3.1	Introduction	734

5.43.2.3.2	Resource Definition	734

5.43.2.3.3	Resource Standard Methods	734

5.43.2.3.3.1	GET	734

5.43.2.3.3.2	PUT	735

5.43.2.3.3.3	PATCH	736

5.43.2.3.3.4	DELETE	737

5.43.2.3.4	Resource Custom Operations	738

5.43.3	Custom Operations without associated resources	738

5.43.4	Notifications	738

5.43.5	Data Model	740

5.43.5.1	General	740

5.43.5.2	Structured data types	740

5.43.5.2.1	Introduction	740

5.43.5.2.2	Type: ImsEESubsc	741

5.43.5.2.3	Type: ImsEENotif	741

5.43.5.2.5	Type: ImsEventRequirement	742

5.43.5.2.6	Type: ImsEventReport	742

5.43.5.3	Simple data types and enumerations	742

5.43.5.3.1	Introduction	742

5.43.5.3.2	Simple data types	742

5.43.5.3.3	Enumeration: ImsEventType	742

5.43.5.4	Data types describing alternative data types or combinations of data types	743

5.43.6	Used Features	743

5.43.7	Error handling	743

5.43.7.1	General	743

5.43.7.2	Protocol Errors	743

5.43.7.3	Application Errors	743

5.44	ImsParamProvision API	743

5.44.1	Introduction	743

5.44.2	Resources	744

5.44.2.1	Overview	744

5.44.2.2	Resource: IMS Parameters Provisionings	745

5.44.2.2.1	Introduction	745

5.44.2.2.2	Resource Definition	745

5.44.2.2.3	Resource Methods	745

5.44.2.2.3.1	GET	745

5.44.2.2.3.2	POST	746

5.44.2.2.4	Resource Custom Operations	747

5.44.2.3	Resource: Individual IMS Parameters Provisioning	747

5.44.2.3.1	Introduction	747

5.44.2.3.2	Resource Definition	747

5.44.2.3.3	Resource Methods	748

5.44.2.3.3.1	GET	748

5.44.2.3.3.2	PUT	749

5.44.2.3.3.3	PATCH	750

5.44.2.3.3.4	DELETE	751

5.44.2.3.4	Resource Custom Operations	752

5.44.3	Custom Operations without associated resources	752

5.44.4	Notifications	752

5.44.5	Data Model	752

5.44.5.1	General	752

5.44.5.2	Structured data types	753

5.44.5.2.1	Introduction	753

5.44.5.2.2	Type: ImsData	753

5.44.5.2.3	Type: ImsPpData	753

5.44.5.2.4	Type: ImsPpDataPatch	753

5.44.5.3	Simple data types and enumerations	754

5.44.5.3.1	Introduction	754

5.44.5.3.2	Simple data types	754

5.44.5.4	Data types describing alternative data types or combinations of data types	754

5.44.6	Used Features	754

5.44.7	Error handling	754

5.44.7.1	General	754

5.44.7.2	Protocol Errors	754

5.44.7.3	Application Errors	754

6	Security	755

7	Using Common API Framework	755

7.1	General	755

7.2	Security	755

Annex A (normative):	OpenAPI representation for NEF Northbound APIs	757

A.1	General	757

A.2	TrafficInfluence API	757

A.3	NiddConfigurationTrigger API	769

A.4	AnalyticsExposure API	771

A.5	5GLANParameterProvision API	788

A.6	ApplyingBdtPolicy API	798

A.7	IPTVConfiguration API	801

A.8	LpiParameterProvision API	807

A.9	ServiceParameter API	812

A.10	ACSParameterProvision API	826

A.11	MoLcsNotify API	831

A.12	AKMA API	833

A.13	TimeSyncExposure API	835

A.14	EcsAddressProvision API	850

A.15	AMPolicyAuthorization API	854

A.16	AMInfluence API	861

A.17	MBSTMGI API	868

A.18	MBSSession API	871

A.19	EASDeployment API	882

A.20	ASTI API	888

A.21	DataReporting API	894

A.22	DataReportingProvisioning API	898

A.23	UEId API	903

A.24	MBSUserService API	911

A.25	MBSUserDataIngestSession API	916

A.26	MSEventExposure API	925

A.27	MBSGroupMsgDelivery API	929

A.28	DNAIMapping API	934

A.29	PDTQPolicyNegotiation API	939

A.30	MemberUESelectionAssistance API	944

A.31	GroupParametersProvisioning API	955

A.32	SliceParamProvision API	960

A.33	UEAddress API	965

A.34	ECSAddress API	970

A.35	RSLPPIParametersProvisioning API	976

A.36	AddressingParamProvision API	981

A.37	UAVFlightAssistance API	986

A.38	CagInfoParamProvision API	993

A.39	RetrieveInfoUAVFlight API	998

A.40	ImsSessionManagement API	1000

A.41	ImsEventExposure API	1000

A.42	ImsParamProvision API	1006

Annex B (informative):	Change history	1012