ETSI/STC/SMG1	Tdoc SMG1 _366___/95

Bergen, Norway

21 - 25 August 1995

Title:		Support of Optimal Routing

Source:	SMG1 Rapporteur

Support of Optimal Routing (SOR)


Service description, Stage 1

Proposed Version 0.65.0

21 August, 1995

INTRODUCTION

Editor's note on the remaining work:

This version reflects the decision taken at the SMG1 WPA meeting 8th/9th June 1995, in Sophia Antipolis 22nd/23rd August in Bergen.

The stage 1 text has been reduced to the two most important functionalities

§	tromboning back to the Home CountryPLMN of called parties (mostly due to conditional call forwarding back to subscribers voice mail);

§	mobile to mobile calls in the same countrynetworks.

All further call scenarios, including multiple call forwarding, are left to subsequent phases of SOR. Subsequent phases of SOR shall be backwards compatible with this first phase.

Table of contents

1	Scope	3

2	References	3

3	Definitions and abbreviations	3

4	Description of Optimal Routing	4

5	Functional requirements	4

5.1	General	4

5.2	Normal procedures	4

5.2.1	Normal operation with successful outcome	4

5.2.2	Normal operation with successful outcome	5

5.3	Exceptional procedures or unsuccessful outcome	6

5.3.1	Roaming into networks not supporting SOR	6

5.3.2	Non-support of SOR in the HPLMN	6

6	Adaptations for Support of Optimal Routing	6

6.1	Operator Determined Barring (GSM 02.41)	6

6.2	Supplementary Services	6

6.2.1	Call Barring (GSM 02.88)	6

6.2.2	Call Transfer (GSM 02.91)	6

6.3	CAMEL (Customised Application for Mobile Network Enhanced Logic, 02.78, ...)	6

§

# 1	Scope

This Stage 1 description of the first phase of SOR

§	compiles the basic service requirements for SOR, based on the principles elaborated by GSM MoU;

§	describes the interactions for Supplementary Services (SS) in order to cater for SOR;

§	refers to modifications to network features required by SOR.

This ETS does not address the following:

§	There is no need for optimisation of the routing of calls originally directed to a fixed network subscriber, because the physical address of a fixed network terminating line cannot differ from its logical address.

§	SOR in non-GSM mobile networks is not a subject of this ETS, but might be possible by bilateral arrangement between GSM network operators and those non-GSM mobile network operators.

The first phase of SOR caters for

§	tromboning back to the Home CountryPLMN of called parties (mostly due to conditional call forwarding back to subscribers voice mail);

§	mobile to mobile calls in the same countrynetworks.

All further call scenarios, including multiple call forwarding, are left to subsequent phases of SOR. Subsequent phases of SOR shall be backwards compatible with this first phase.

# 2	References

<to be inserted>

# 3	Definitions and abbreviations

§	Support of Optimal Routing (SOR):

It is a network feature which allows the routing of a call according to the rules specified in this standard. All PLMNs involved must support this feature otherwise reduced functionality, as described, will be experienced.

§	Direct route:

A call uses the direct route if it is routed from the serving network of the original calling party to the serving network of the ultimate called party without any intermediate exchange being involved in the computation of the charge for the call.

§	HPLMN route:

A call uses the HPLMN route if the destination is defined by the MSISDN of the called party; this forces the call to be routed via the HPLMN of the called party. If the called party is registered in the HPLMN, the direct route and the HPLMN route are the same.

§	HPLMN leg:

The HPLMN leg is that part of the HPLMN route from the originating switch to the HPLMN of the called party.

§	IPLMN:

The IPLMN is the PLMN which interrogates the HPLMN of a called or forwarded-to party to determine the whereabouts of that party.

# 4	Description of Optimal Routing

Support of Optimal Routing (SOR) is a network feature, by which calls directed to a mobile subscriber, who has roamed away from his HPLMN, are routed directly (instead of via the HPLMN) to the mobile subscriber's actual location, or the forwarded-to destination.

NOTE:	The principles for SOR may also apply within the HPLMN, but this is outside the scope of this ETS.

This ETS is elaborated upon the following charging principles:

SOR shall  be used only if financial benefits are obtained and provided acceptable quality of service can be maintained. The application of SOR shall not lead to any one user being charged more than would have been the case without SOR. The financial benefits of SOR shall be applied in the following order of priority:

1.	forwarding party .

2.	ultimate destination party (i.e. neither the calling party nor a forwarding party).

3.	calling party.

4.	network operator or operators.

NOTE:	Charging principles are not subject of this ETS, but they are a underlying precondition for the elaboration of this ETS. The charging principles agreed by GSM MoU have been applied and reprinted as appropriate.

NOTE:	Reduced costs due to the optimised routing of a given call may reveal that the subscriber is not at the location expected.

NOTE:	Due to the limited scope of this first phase of SOR, there will be no charges raised neither for international legs.the forwarding nor the called party.

# 5	Functional requirements

## 5.1	General

The serving network shall provide means for optimisation of the route of a given call from the calling party to the ultimate destination, taking account of call forwarding, before the call is actually completed, independent of the location of the ultimate destination or any forwarding party.

If the serving network supports SOR, it shall  also support the consequential modifications to existing SS as described below.

If the serving network of the ultimate destination supports SOR, it shall be able to generate a call record for the ultimate destination indicating that the call has been optimally routed.

If any party involved in a call is a fixed network party, the equivalent of the VPLMN and the HPLMN is the fixed network serving that party.

SOR shall be provided for all circuit switched Telecommunication Services, except emergency calls, Dedicated PAD and Dedicated Packet Access (GPRS is for further study). The network feature is offered to all home and visiting subscribers served by the network. It shall be possible as a network operator option to deny optimal routing of calls directed to a specified subscriber.

SOR shall be invoked automatically by the network at every call set-up. The route to the ultimate destination shall be  optimised.

## 5.2	Normal procedures

### 5.2.1	Normal operation with successful outcome

SOR shall be provided for all circuit switched Telecommunication Services, except emergency calls, Dedicated PAD and Dedicated Packet Access (GPRS is for further study). The network feature is offered to all home and visiting subscribers served by the network. It shall be possible as a network operator option to deny optimal routing of calls directed to a specified subscriber.

SOR shall be invoked automatically by the network at every call set-up. The route to the ultimate destination shall be  optimised. This means that if a call is forwarded the route is the optimal one from the original calling party to the forwarded-to party.

### 5.2.12	Normal operation with successful outcome

In this first phase of SOR the routes of calls will be optimised in case the following two scenarios occur. Under all other circumstances, the GSM basic call handling is applied.

Fig.1:	B is roamed mobile subscriber:

A call is to be set up from a calling mobile subscriber A to a mobile subscriber B. The originating network interrogates the HPLMN of subscriber B and gets back the MSRN and the status of the subscriber (IMSI attach/detach). The call shall be routed directly from A to subscriber B if B is attached and if the VPLMN country of subscriber B and the VPLMN of subscriber A are located in the same countryis identical with the country of subscriber A. Subscriber A will be charged with equivalent costs of a national call within that VPLMN, no charge will be levied to subscriber B.

Fig.2:	B is roamed mobile subscriber, with CF active to a subscriber in his home country (fixed or mobile):

A call is to be set up from a calling subscriber A (mobile or fixed) to a mobile subscriber B.

In case A is a mobile subscriber, the originating network interrogates the HPLMN of subscriber B and gets back the MSRN and the status of the subscriber (IMSI attach/detach). If subscriber B is detached and conditional call forwarding is active and the forwarding to number belongs to the home country of subscriber B the call shall be directly routed to the forwarded-to number.

In case A is a fixed subscriber the call will be delivered to the GMSC of the HPLMN of subscriber B. The GMSC interrogates the HPLMN of subscriber B and gets back the MSRN and the status of the subscriber (IMSI attach/detach). If subscriber B is detached and conditional call forwarding is active and the forwarding to number belongs to the home country of subscriber B the call shall be directly routed to the forwarded-to number.

Subscriber A will be charged with costs according to numbers he has dialled, the subscriber may be charged  for a national forwarding legno charge will be levied to subscriber B.

Where SOR is invoked and the called party has subscribed to Advice of Charge (AoC), AoC parameters for the called party shall be set to zero.

## 5.3	Exceptional procedures or unsuccessful outcome

### 5.3.1	Roaming into networks not supporting SOR

If a subscriber whose HPLMN supports SOR registers in a VPLMN which does not support SOR, the VPLMN shall inform the HPLMN that the VPLMN does not support SOR.

### 5.3.2	Non-support of SOR in the HPLMN

If the HPLMN of the called party does not support SOR, SOR cannot be invoked and the route of the call as far as the called party’s VPLMN cannot be optimised (the call would normally be routed via the HPLMN of the called party).

# 6	Adaptations for Support of Optimal Routing

## 6.1	Operator Determined Barring (GSM 02.41)

The principles for the interaction between operator determined barring and SOR are the same as those for the interaction between Supplementary Service Call Barring and SOR.

## 6.2	Supplementary Services

Supplementary services shall be invoked based on the outcome of an optimal routing attempt unless it is stated differently below.

[Editor´s note: This general rule does not apply to Call Forwarding as there is a close interaction between CF and SOR during the call set-up phase, described under 5.2.2.]

### 6.2.1	Call Barring (GSM 02.88)

The basis of the interactions between SOR and Supplementary Service Call Barring is that barring of outgoing calls and barring of incoming calls when roaming outside the HPLMN country (BIC-Roam) are used for cost control.

If Call Barring does not prevent a call which is not optimally routed than it shall not prevent the use of optimal routing for that call.

If a subscriber has subscribed to Barring of Outgoing International Calls (BOIC) or Barring of Outgoing International Calls except to the Home PLMN Country (BOIC-exHC), calls which would be prevented without the invocation of SOR might be permitted.

It is in general not possible to predict at the time when registration or activation of call forwarding or activation of call barring is requested whether a call forwarding would be prevented by a call barring programme. The check for the interactions between Call Forwarding and Call Barring therefore has to be carried out when the call forwarding is to be invoked, rather than when registration or activation of call forwarding or activation of call barring is requested. The only exception to this principle is the interaction between BIC-Roam and Call Forwarding on No Reply can be checked at registration/activation.

### 6.2.2	Call Transfer (GSM 02.91)

The two transferred calls shall follow the general rules for the optimal routing as defined for the basic call set-up.

Note:	Optimal routing can apply after the connection of the remote parties. The location of the connection can be re-routed to another MSC. But this needs further investigation.

## 6.3	CAMEL (Customised Application for Mobile Network Enhanced Logic, 02.78, ...)

If CAMEL support is invoked at call set-up, SOR shall be applied to the outcome of the invocation of CAMEL.