| 3GPP TS 23.435 V0.1.0 (2022-07) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Procedures for Network Slice Capability Exposure for Application Layer Enablement Service (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Overview	8

4.x	<Functionality>	8

5	Business models and relationships for NSCALE	8

6	Architectural requirements	8

6.1	General Description	8

6.2	General requirements	8

6.3	Lifecycle management requirements	8

6.x	<Architectural requirements>	9

7	Application architecture for NSCALE	9

8	Identities and commonly used values	9

8.1	General	9

8.2	NSCE server ID	9

8.3	NSCE client ID	9

8.4	Slice coverage area	9

8.5	NSCE service area	9

9	Procedures and information flows	10

9.1	General	10

9.2	VAL server registration	10

9.2.1	General	10

9.2.2	Procedure	10

9.2.2.1	VAL server registration	10

9.2.2.2	VAL server de-registration	10

9.2.3	Information flows	10

9.2.3.1	General	10

9.2.3.2	VAL server registration request	10

9.2.3.3	VAL server registration response	11

9.2.3.4	Network slice capability de-registration request	11

9.2.3.5	Network slice capability de-registration response	11

9.2.4	APIs	12

9.2.4.1	General	12

9.2.4.2	Ensce_VALRegistration_Request operation	12

9.2.4.3	Ensce_VALRegistration_Deregister operation	12

9.x	<procedure name>	12

9.x.1	General	12

9.x.2	Procedure	12

9.x.3	Information flows	12

9.x.4	APIs (if applicable)	12

Annex A (informative): Deployment models	13

Annex B (informative): Change history	14

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Network slice capability enablement(NSCE) is a service that enabling the network slice related capabilities towards 3rd party on the basis of R17 SEAL. The NSCE service providing additional functionality and expose slice capabilities based on SA5 MnS services and SA2 service (e.g. NEF APIs, NWDAF APIs, NSACF APIs). This technical specification provides procedures for enabling NSCE service over 3GPP networks while the architecture is defined in TS 23.434[x].

# 1	Scope

The present document specifies the procedures and information flows necessary for Network Slice Capability Exposure for Application Layer Enablement on the basis of R17 SEAL.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

…

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Overview

Clauses under this clause shall document the functionalities of the Network Slice Capability Exposure for Application Layer Enablement.

## 4.x	<Functionality>

Add a copy of this clause for each  functionality, adding a title and description.

# 5	Business models and relationships for NSCALE

Clauses under this clause shall document the business relationships for NSCALE.

# 6	Architectural requirements

Clauses under this clause shall document the architectural requirements for the architecture.

## 6.1	General Description

The following clauses specify the requirements for network slice capability enablement services for application layers.

## 6.2	General requirements

[AR-6.2-1] The NSCE architecture shall support the NSCE server to communicate with the VAL server with one or more applications.

[AR-6.2-2] The NSCE architecture shall support the NSCE server to communicate with one or more VAL servers.

[AR-6.2-3] The application enablement layer shall support the interactions with 3GPP network management system to consume network slice management service provided by MNO.

## 6.3	Lifecycle management requirements

[AR-6.3-1] The NSCE architecture shall enable lifecycle management of network slice by authorized VAL server.

[AR-6.3-2] The NSCE architecture shall enable lifecycle management of network slice communication service by authorized VAL server.

## 6.x	<Architectural requirements>

Add a copy of this clause for each architectural requirements, adding a title and description.

# 7	Application architecture for NSCALE

Add a reference to TS 23.434.

# 8	Identities and commonly used values

## 8.1	General

The common identities for SEAL refer to TS 23.434[2]. The following clauses list the additional identities and commonly used values for Network Slice Capability Enablement.

## 8.2	NSCE server ID

The NSCE server ID uniquely identifies the Network Slice Capability enablement server.

## 8.3	NSCE client ID

The Network Slice Capability enablement client ID identifies a particular NSCE client.

## 8.4	Slice coverage area

The slice coverage area is the area where the network slice is available in the whole PLMN or in one or more Tracking Areas of the PLMN. The slice coverage area can be expressed as a Topological Service Area (e.g. a list of TA), a Geographical Service Area (e.g. geographical coordinates) or both.

## 8.5	NSCE service area

The NSCE service area is the area where the Network Slice Capability Enablement server owner provides its services. It is equal to the collection of coverage area of slices it can enable.

The NSCE service area can be expressed as a Topological Service Area (e.g. a list of TA), a Geographical Service Area (e.g. geographical coordinates) or both.

NOTE:	The NSCE server service area shall not smaller than the collection of slice(s) coverage area(s) the NSCE server can enable.

# 9	Procedures and information flows

Subclause of this clause shall describe the procedures for the application architecture for enabling Edge Application.

## 9.1	General

## 9.2	VAL server registration

### 9.2.1	General

The NSCE layer supports the exposure of network slice related capabilities to the VAL. To access network slice related enablement capabilities, e.g. monitoring of network slice performance, the VAL server should register on the NSCE layer.

### 9.2.2	Procedure

#### 9.2.2.1	VAL server registration

For registration of the VAL server to be a recognized user of the NSCE server, the VAL server triggers registration procedure which is defined as onboarding the API invoker in TS 23.222 clause 8.1.3. In such procedure, the NSCE server could play the role of CAPIF core function.

#### 9.2.2.2	VAL server de-registration

For de-registration of the VAL server, the VAL server triggers de-registration procedure which is defined as offboarding the API invoker in TS 23.222 clause 8.2.3.

### 9.2.3	Information flows

#### 9.2.3.1	General

The following information flows are specified for service provisioning:

-	VAL server registration request and response; and

-	VAL server de-registration request and response.

#### 9.2.3.2	VAL server registration request

Table 9.2.3.2-1 describes information elements for the VAL server registration request from the VAL server to the NSCE server.

Table 9.2.3.2-1: VAL server registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration information | M | The information of the VAL server |
| >VAL server ID | M | The identifier of the VAL server |
| >VAL Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the VAL. |
| >Security credentials | M | Security credentials resulting from a successful authorization for the NSCE service. |
| >Proposed expiration time | O | Proposed expiration time for the registration |
| APIs for enrollment | O | Indicates interested NSCE services. |

#### 9.2.3.3	VAL server registration response

The information elements specified in the Table 9.2.3.3-1-1 is used for the VAL server registration response sent from the NSCE server to the VAL server.

Table 9.2.3.3-1: VAL server registration response

| Information element | Status | Description |
| --- | --- | --- |
| Registration status | M | The result of Registration request |
| >Successful response (NOTE1) | O | Indicates that the VAL server registration request was successful. |
| > >Registration ID | M | Identifier of the registration. |
| > >Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| >Failure response (NOTE1) | O | Indicates that the VAL server registration request failed. |
| Registration information | O (NOTE 2) | Information allow the VAL server to be authenticated and to obtain authorization for service APIs |
| Service API information | O (NOTE 3) | NSCE service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| Reason | O (NOTE 4) | Indicates the cause of VAL server registration request failure |
| NOTE 1: One of these IEs shall be present in the message. NOTE 2: Information element shall be present when registration status is successful. NOTE 3: Information element may be present when registration status is successful. NOTE 4: Information element shall be present when registration status is failure. |  |  |

#### 9.2.3.4	Network slice capability de-registration request

Table 9.2.3.4-1 describes information elements for the network slice capability de-registration request from the VAL server to the NSCE server.

Table 9.2.3.4-1: Network slice capability de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the registration. |
| Security credentials | M | Security credentials of the EAS |
| Reason | O | Indicate the reason of de-registration |

#### 9.2.3.5	Network slice capability de-registration response

The information elements specified in the Table 9.2.3.5-1-1 is used for the Network slice capability de-registration response sent from the NSCE server to the VAL server.

Table 9.2.3.5-1: Network slice capability de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the Network slice capability de-registration request was successful. |
| Failure response (NOTE) | O | Indicates that the Network slice capability de-registration request failed. |
| > Cause | O | Indicates the cause of Network slice capability de-registration request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

### 9.2.4	APIs

##### 9.2.4.1	General

Table 9.2.4.1-1 illustrates the API for VAL server registration.

Table 9.2.4.1-1: Ensce_VALRegistration API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_VALRegistration | Request | Request/Response | VAL |
|  | Deregister |  |  |

##### 9.2.4.2	Ensce_VALRegistration_Request operation

API operation name: Ensce_VALRegistration_Request

Description: The consumer requests to register the VAL server on the NSCE server.

Inputs: See clause 9.2.3.2.

Outputs: See clause 9.2.3.3.

See clause 9.2.2.1 for details of usage of this operation.

##### 9.2.4.3	Ensce_VALRegistration_Deregister operation

API operation name: Ensce_VALRegistration_Deregister

Description: The consumer requests to deregister the VAL server from the NSCE server.

Inputs: See clause 9.2.3.4.

Outputs: See clause 9.2.3.5.

See clause 9.2.2.2 for details of usage of this operation.

## 9.x	<procedure name>

Add a copy of this clause for each procedure, adding a title, a general description, information flows and the procedure. Include an APIs clause if APIs are needed for the procedure.

### 9.x.1	General

### 9.x.2	Procedure

### 9.x.3	Information flows

### 9.x.4	APIs (if applicable)

This clause may be skipped if the procedure under 10.x does not require specification of APIs.

Annex A (informative): Deployment models

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

Annex B (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-06 | SA6#49-bis-e |  |  |  |  | TS skeleton | 0.0.0 |
| 2022-07 | SA6#49-bis-e |  |  |  |  | TS Skeleton agreed in SA6#44: S6-221640, Implemented pCRs approved in SA6#49-bis-e: S6-221641, S6-221642, S6-221643, S6-221822, S6-221818. | 0.1.0 |