| 3GPP TS 23.436 V0.3.0 (2023-01) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Procedures for Application Data Analytics Enablement Service; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Architectural requirements	8

4.1	General Description	8

4.2	General Requirements	8

4.x	<Architectural requirements>	8

5	Application architecture for ADAES	8

5.1 	General	8

5.2	Functional architecture	9

5.2.1	General	9

5.2.2	On-network Functional Architecture	9

5.2.3	Off-network Functional Architecture	10

5.3 	ADAE internal architecture	11

6	ADAE layer Functional Description	12

6.1	Support for application performance analytics	12

6.2	Support for slice-specific application performance analytics	12

6.3	Support for UE-to-UE application performance analytics	12

6.4	Support for location accuracy analytics	12

6.5	Support for service API analytics	12

6.x	<Functionality>	13

7	Identities and commonly used values	13

7.1	General	13

7.2	ADAE Server ID	13

7.3	ADAE client ID	13

7.4	A-ADRF ID	13

7.5	A-DCCF ID	13

7.6	Data Producer ID	13

7.7	ADAE service area	13

7.8	Analytics ID	13

8	Procedures and information flows	14

8.1	General	14

8.2	Procedure on support for application performance analytics	14

8.2.1	General	14

8.2.2	Procedure on VAL server performance analytics	14

8.2.3	Procedure on VAL session performance analytics	16

8.2.4	Information flows	18

8.2.4.1	General	18

8.2.4.2	VAL performance analytics subscription request	18

8.2.4.3	VAL performance analytics subscription response	19

8.2.4.4	Data collection subscription request	19

8.2.4.5	Data collection subscription response	20

8.2.4.6	Data Notification	20

8.2.4.7	Analytics Notification	20

8.3	Procedure on support for slice-specific application performance analytics	21

8.3.1	General	21

8.3.2	Procedure	21

8.3.3	Information flows	23

8.3.3.1	General	23

8.3.3.2	Slice-specific performance analytics subscription request	23

8.3.3.3	Slice-specific performance analytics subscription response	23

8.3.3.4	Slice-specific performance analytics notification	24

8.4	Procedure on support for UE-to-UE application performance analytics	24

8.4.1	General	24

8.4.2	Procedure	24

8.4.3	Information flows	26

8.4.3.1	General	26

8.4.3.2	UE-to-UE session performance analytics subscription request	26

8.4.3.3	UE-to-UE session performance analytics subscription response	26

8.4.3.4	UE-to-UE analytics request	27

8.4.3.5	UE-to-UE analytics response	27

8.4.3.6	ADAE Analytics Notification	27

8.5	Procedure on support for location accuracy analytics	28

8.5.1	General	28

8.5.2	Procedure	28

8.5.3	Information flows	30

8.5.3.1	General	30

8.5.3.2	Location accuracy analytics subscription request	30

8.5.3.3	Location accuracy analytics subscription response	30

8.5.3.4	Location accuracy data request	30

8.5.3.5	Location accuracy data response	31

8.5.3.6	Location accuracy analytics notification	31

8.6	Procedure for supporting service API analytics	32

8.6.1	General	32

8.6.2	Procedure	32

8.6.3	Information flows	33

8.6.3.1	General	33

8.6.3.2	Service API event subscription request	33

8.6.3.3	Service API event subscription response	33

8.6.3.4	Historical service API logs request	33

8.6.3.5	Historical service API logs response	34

8.6.3.6	Service API analytics notification	34

8.x	<procedure name>	35

8.x.1	General	35

8.x.2	Procedure	35

8.x.3	Information flows	35

9	ADAE layer APIs	35

Annex A (informative): Deployment scenarios	35

A.1	General	35

A.2	Deployment model #1: Cloud-deployed ADAES	36

A.3	Deployment model #2 Edge-deployed ADAES	36

A.4	Deployment model #3: Coordinated ADAES deployment	37

Annex B (informative): Change history	39

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Considering vertical-specific applications and edge applications as the major consumers of 3GPP-provided data analytics services, the application enablement layer can play role on the exposure of data analytics services from different 3GPP domains to the vertical/ASP in a unified manner; and on defining, at an overarching layer, value-add application data analytics services which cover stats/predictions for the end-to-end application service.

This technical specification provides procedures for enabling ADAE service over 3GPP networks, while the architecture is defined in TS 23.434 [2].

# 1	Scope

The present document specifies the procedures, information flows and APIs necessary for Application Data Analytics Enablement SEAL Service.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture"

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[6]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architectural requirements

Clauses under this clause shall document the architectural requirements for the architecture.

## 4.1	General Description

The following clauses specify the requirements for application data analytics enablement service.

## 4.2	General Requirements

[AR-4.2-a] 	The ADAE client and the ADAE server shall support one or more VAL applications.

[AR-4.2-b] 	Supported ADAE capabilities shall be offered as APIs to the VAL applications.

[AR-4.2-c]	The ADAE shall support interaction with 3GPP network system to consume network and management data analytics services.

[AR-4.2-d] 	The ADAE client shall be capable to communicate with one or more ADAE servers of the same ADAE service provider.

## 4.x	<Architectural requirements>

Add a copy of this clause for each architectural requirements, adding a title and description.

# 5	Application architecture for ADAES

## 5.1 	General

This clause provides the functional architecture for ADAE. This includes the on-network and off-network functional models which are provided in detail in clause 5.2.

In addition, the ADAE internal architecture is described in 5.3, which aligns with the 3GPP data analytics framework (specified in TS 23.288 [4]) and introduces new logical entities within ADAE framework, such as the A-DCCF and A-ADRF.

## 5.2	Functional architecture

### 5.2.1	General

The functional architecture for the application data analytics enablement is based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [2]. It is organized into functional entities to describe a functional architecture which addresses the support for application data analytics enablement aspects for vertical applications.

### 5.2.2	On-network Functional Architecture

For the on-network functional architecture, both service-based representation and reference point representation are provided.

Figure 5.2.2-1 depicts the application data analytics enablement architecture in the non-roaming case, using the reference point representation showing how various entities interact with each other.

Figure 5.2.2-1: Architecture for application data analytics enablement – reference points representation

The application data analytics enablement client communicates with the application data analytics enablement server over the ADAE-UU reference point. The application data analytics enablement client provides the support for application data analytics enablement functions to the VAL client(s) over ADAE-C reference point. The VAL server(s) communicates with the application data analytics enablement server over the ADAE-S reference point. The application data analytics enablement server, acting as AF, may communicate with the 5G Core Network functions (over N33 reference point to NEF and N6 reference point to UPF) and OAM (over ADAE-OAM interface).

Figure 5.2.2-2 exhibits the service-based interfaces for providing and consuming application data analytics enablement services. The application data analytics enablement server could provide service to VAL server and ADAE client through interface SAdae.

Figure 5.2.2-2: Architecture for application data analytics enablement – Service based representation

Figure 5.2.2-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [a].

Figure 5.2.2-3: Architecture for application data analytics enablement utilizing the 5GS network services based on the 5GS SBA – Service based representation

### 5.2.3	Off-network Functional Architecture

Figure 5.2.3-1 illustrates the generic off-network functional model for ADAE.

Figure 5.2.3-1: Generic off-network functional model

In the vertical application layer, the VAL client of UE1 communicates with VAL client of UE2 over VAL-PC5 reference point. An application data analytics enablement client of UE1 interacts with the corresponding application data analytics enablement client of UE2 over ADAE-PC5 reference points. The UE1, if connected to the network via Uu reference point, can also act as a UE-to-network relay, to enable UE2 to access the VAL server(s) over the VAL-UU reference point.

The service-based interface representation is specified in clause 15 of 3GPP TS 23.434 [2].

## 5.3 	ADAE internal architecture

In ADAE framework, A-DCCF and A-ADRF can be defined as functionalities within the internal ADAE architecture and can offer the following functionalities:

-	Application layer - Data Collection and Coordination Function (A-DCCF) coordinates the collection and distribution of data requested by the consumer (ADAE server). Data Collection Coordination is supported by a A-DCCF. ADAE server can send requests for data to the A-DCCF rather than directly to the Data Sources. A-DCCF may also perform data processing/abstraction and data preparation based on the VAL server requirements.

-	Application layer – Analytics and Data Repository Function (A-ADRF) stores historical data and/or analytics, i.e., data and/or analytics related to past time period that has been obtained by the consumer (e.g. ADAE server). After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an A-ADRF. Whether the consumer directly contacts the A-ADRF or goes via the A-DCCF is based on configuration.

Figure 5.3-1 illustrates the generic functional model for ADAE when re-using the 3GPP network data analytics model.

Figure 5.3-1: ADAE internal functional architecture

In this model, an A-DCCF is used to fetch data or put data into an application-level entity (e.g. A-ADRF, Data Source). Such A-DCCF coordinates the collection and distribution of data requested by ADAE server (over ADCCF-1, ADAE-X). ADAE server can also directly interact with the Data Sources via ADAE-Y.

Also, Application layer – Analytics and Data Repository Function (A-ADRF) can be used to store historical data and/or analytics, i.e., data and/or analytics related to past time period that has been obtained by the ADAE server (via AADRF-1) or other NFs/NWDAF. ADAE server can also fetch historical data from ADRF. Whether the ADAE server directly contacts the ADRF or goes via the A-DCCF is based on configuration.

Data Sources can be 5GS data sources (5GC, OAM) or enablement layer data sources (SEAL, EEL) or external data sources at the DN side (VAL server/ EAS) and VAL UEs.  A-DCCF and A-ADRF can be used only for interacting with certain data sources (e.g., 5GC, OAM) based on configuration, and can be hidden from the VAL layer.

NOTE:	If the Data Source is the VAL UE, then the data collection mechanism shall reuse the SA4 mechanism based on EVEX study (TS 26.531 [3]).

# 6	ADAE layer Functional Description

Clauses under this clause shall document the functionalities of the Application Data Analytics Enablement layer.

## 6.1	Support for application performance analytics

This feature supports the derivation and exposure of application layer analytics to provide insight on the operation and performance of an application (VAL server or EAS, application session), and in particular statistics or prediction on parameters related to e.g. VAL server number of connections for a given time and area, VAL server rate of connection requests, connection probability failure rates, RTT and deviations for a VAL server or VAL UE session, packet loss rates etc.

## 6.2	Support for slice-specific application performance analytics

This feature introduces application layer analytics to provide insight on the performance of the VAL applications when using a given network slice (from a list of subscribed slices for the VAL customer). Such capability provides an analytics service to a consumer who can be either the VAL server (for helping to identify what slice it will use for its applications) or for other consumers such as SEAL NSCE to support on providing analytics (since NSCE doesn't contain an analytics engine for providing analytics on top of NWDAF [4] /MDAS [5]).

## 6.3	Support for UE-to-UE application performance analytics

This feature supports the derivation and exposure of application layer analytics to predict the performance of an application session among two or more VAL UEs within a service or group. Such prediction relates to application QoS attributes prediction for a given time horizon and area. This can be requested by the VAL server during the session, or the VAL server can subscribe to receive predicted application QoS downgrade indication for an ongoing session. Such analytics will help improving the application service experience and allow the VAL layer to pro-actively adapt to predicted application QoS changes.

## 6.4	Support for location accuracy analytics

This feature supports application layer analytics enablement to allow a VAL server to be notified based on analytics whether the accuracy of a location can be met for a given application and optionally for a given UE/group route. For example, a VAL server may request the ADAE server to provide analytics whether the accuracy of a location for the UEs within a VAL application is predicted to be sustainable or is expected to downgrade in a specific area or for an expected route from location A to location B.

## 6.5	Support for service API analytics

This feature introduces service API analytics to allow a VAL server or any other consumer (e.g. API provider) to be notified on the predicted /statistic availability and service level for the requested service API analytics. Such analytics may be utilized by the API provider to perform actions to avoid service API invocation failures or other actions like throttling/rate limitations. Also, such analytics will support the VAL server to identify if/when to perform an API invocation request based on the API expected status at the given area and time horizon.

## 6.x	<Functionality>

Add a copy of this clause for each functionality, adding a title and description.

# 7	Identities and commonly used values

## 7.1	General

The common identities for SEAL refer to TS 23.434[2]. The following clauses list the additional identities and commonly used values for Application Data Analytics Enablement Service.

## 7.2	ADAE Server ID

The ADAE server ID uniquely identifies the application data analytics enablement server, and each ADAE server ID is unique within PLMN domain.

## 7.3	ADAE client ID

The ADAE client ID uniquely identifies the application data analytics enablement client.

## 7.4	A-ADRF ID

The A-ADRF ID uniquely identifies the application data analytics repository function.

## 7.5	A-DCCF ID

The A-DCCF ID uniquely identifies the application data collection and coordination function.

## 7.6	Data Producer ID

The Data Producer ID uniquely identifies the data producer / source which is used as input for application data analytics enablement services. Data Producer based on the analytics event, can be either a network function or a management domain function/service or an application server or client or an edge / cloud service.

## 7.7	ADAE service area

The ADAE service area is the area where the Application Data Analytics Enablement server owner provides its analytics services. It is equal to the coverage area for which analytics apply.

The ADAE service area can be expressed as a Topological Service Area (e.g. a list of TA), a Geographical Service Area (e.g. geographical coordinates) or both.

## 7.8	Analytics ID

The analytics ID (or analytics event ID) identifies the application layer analytics event which corresponds to the specified ADAE analytics services.

# 8	Procedures and information flows

Subclause of this clause shall describe the procedures for the application architecture for enabling application data analytics.

## 8.1	General

This clause describes the procedures and the information flows related to the ADAE capabilities, as introduced in clause 6.

## 8.2	Procedure on support for application performance analytics

### 8.2.1	General

In this functionality, two procedures are described in more detail in clause 8.2.2 and 8.2.3 accordingly:

-	one procedure for VAL server related analytics where an example in provided for VAL server performance,

-	one procedure for VAL session/UE related analytics.

### 8.2.2	Procedure on VAL server performance analytics

Figure 8.2.2-1 illustrates the procedure where the VAL server performance analytics are performed based on data collected from the ongoing VAL sessions as well as data from the DN (VAL server, DN database or networking stack at the DN).

Pre-conditions:

1.	ADAE Client (ADAEC) is connected to ADAES.

Figure 8.2.2-1: ADAES support for VAL server performance analytics

1.	The consumer of the ADAES analytics service sends a VAL performance analytics subscription request to ADAES.

2.	The ADAES sends a subscription response as a positive or negative acknowledgement to the consumer of the analytics service.

3.	The ADAES maps the analytics event ID to a list of data collection event identifiers, and optionally a list of data producer IDs. Such mapping may be preconfigured by OAM or may be configured at ADAES based on the analytics event type / vertical type.

4.	The ADAES sends a data collection subscription request to the Data Producers (at the DN side or UE side) with the respective Data Collection Event ID and the requirement for data collection. Such data producers include the A-ADRF, the VAL server, SEALDD server, or the VAL UEs.

5.	The Data Producer(s) sends a subscription response as a positive or negative acknowledgement to the ADAES.

NOTE:	The ADAES acting as AF may also subscribe to NEF/SMF/PCF/NWDAF to monitor network/UE situation or network data analytics required for the application data analytics event.

6.	The ADAES based on subscription, may receive offline stats/data from A-ADRF on the VAL server performance based on the analytics/data collection event ID. Such offline data can be average/peak throughput, average/maximum e2e delay, jitter, av. PER, availability, VAL server load, number of failed transactions, and can be for a given area and time of the day (based on the time/area of the request).

A session starts between the VAL server #1 and a UE (this could happen for more than one UEs)

7.	The Data Producer at DN side, starts collecting/listening to real-time networking or application data (from networking start at DN or VAL server itself). Such data can be the RTT, the PER, throughput etc.

8a.	The Data Producer sends the real-time data to the ADAES, where the data correspond to the data collection ID or the analytics event ID for which the ADAES subscribed.

8b.	The ADAES may receive also data (periodically or if a threshold is reached based on configuration) from the application of the UE within the ongoing session (via ADAEC). Such data can be about the RTT, average/peak throughput, jitter, QoE measurements (MOS, stalling events, stalling ratios, etc), QoS profile load, VAL server load, etc.

9.	When the VAL UE session with VAL server finishes, the ADAEC notifies the ADAEC of the completion of the reporting.

10.	The ADAES abstracts or correlates the data based on the analytics event and the data collection configuration. Such correlation can be filtering of data for the same metrics but with different granularities or be combining/aggregating the data of segments of the end-to-end path (end to end is between VAL client and server). The outcome is an abstracted/correlated/filtered set of data.

11.	The ADAES derives application layer analytics on VAL server #1 performance, based on the analytics ID and type of request. Such analytics can be stats or prediction for a given area/time and based on the event type for a given network configuration.

12.	The ADAES sends the analytics to the consumer, where these analytics include the VAL server 1 predicted or statistic performance for a given area and time horizon, including also the confidence level, whether offline/online analytics were used.

### 8.2.3	Procedure on VAL session performance analytics

Figure 8.2.3-1 illustrates the procedure where the VAL session performance analytics are performed based on data collected from the ongoing VAL sessions.

Pre-conditions:

1.	ADAEC is connected to ADAES.

Figure 8.2.3-1: ADAES support for VAL session performance analytics

1.	The consumer of the ADAES analytics service sends a VAL performance analytics subscription request to ADAES and provides the analytics event ID e.g. "VAL UE perf prediction", the target VAL UE ID, VAL server ID/VAL application ID, the time validity and area of the request, the required confidence level, exposure level for providing UE analytics. If the consumer is the VAL server, the VAL server can provide to ADAEC application data related to the UE expected route/trajectory and VAL application traffic schedule / expected session time.

2.	The ADAES sends a subscription response as an ACK to the consumer.

3.	The ADAES selects the corresponding ADAEC of the VAL UE for which the local analytics need to be performed.

4a.	The ADAES sends a subscription request to the ADAEC with the analytics event ID and the configuration of the reporting required (e.g., periodic, based on threshold or event)

4b.	The ADAEC sends a subscription response to ADAES

5.	The ADAEC maps the analytics event ID to a list of data collection event identifiers or data collected IDs at the VAL UE or other UEs within the service and in proximity (in group-based communications)

6.	The ADAEC subscribes to the VAL clients and/or requests UE local data based on the respective Data Collection Event ID (or the analytics event ID if they already know the mapping). This data may come from the PDU layer of the UE (via listening the traffic), or via VAL client of one or more UEs (if an application consists of a group of UEs).

A session starts between the VAL UE #1 and a VAL server.

7.	The ADAEC (after being aware from the VAL client that the session started) sends a notification to ADAES that a session started, and it could be possible to provide real-time data analytics for VAL UE performance in the target area.

8.	The ADAEC starts collecting data from the corresponding VAL UE(s) based on subscription. Such data can be about the RTT, throughput, jitter, QoE measurements, QoS profile load, etc.  It can be also possible that VAL client provides to ADAEC application data related to the UE expected route/trajectory and VAL application traffic schedule / expected session time.

9.	The ADAEC filters or correlates the data based on the analytics event and the data collection configuration.

10.	When the VAL UE session finishes, the ADAEC (optionally) derives VAL session analytics to ADAES on VAL UE #1 performance, based on the analytics ID and type of request. Such analytics (if performed at the ADAEC can be stats or predictions on the RTT or RTT deviation, average/peak throughput, av. jitter, QoE measurements (MOS, stalling events, buffer related events), QoS profile load, VAL application traffic load etc. In case of prediction, a confidence level shall be also present and a time horizon for the predicted parameters.

11.	The ADAEC sends the data of step 8 or the analytics of step 9 (if ADAEC performs analytics) to the ADAES.

12.	The ADAES derives application layer analytics on VAL session performance (based on the data or analytics received by the ADAEC), based on the analytics ID and type of request. Such analytics can be stats or prediction for a given area/time and based on the event type for a given network configuration. Such analytics (if no analytics is performed at ADAEC) at ADAES can be stats or predictions on the RTT or RTT deviation, average/peak throughput, av. jitter, QoE measurements, QoS profile load, VAL application traffic load etc. In case of prediction, a confidence level shall be also present and a time horizon for the predicted parameters.

13.	The ADAES sends the analytics to the consumer, where these analytics include the VAL UE #1 session predicted performance for a given area and time horizon, including also the confidence level, whether offline/online analytics were used.

### 8.2.4	Information flows

#### 8.2.4.1	General

The following information flows are specified for VAL performance analytics based on 8.2.2 and 8.2.3.

#### 8.2.4.2	VAL performance analytics subscription request

Table 8.2.4.2-1 describes information elements for the VAL performance analytics subscription request from the VAL server / Consumer to the ADAE server.

Table 8.2.4.2-1: VAL performance analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the analytics consumer |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “VAL server performance analytics” for procedure in 8.2.2, or “VAL session performance analytics” for procedure in 8.2.3. |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics |
| VAL service ID | M | The identifier of the VAL service for which analytics subscription apply |
| Target VAL UE ID(s) | O | The VAL UE(s) for which the analytics subscription applies |
| Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies (for procedure in 8.2.2) |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

#### 8.2.4.3	VAL performance analytics subscription response

Table 8.2.4.3-1 describes information elements for the VAL performance analytics subscription response from the ADAE server to the consumer.

Table 8.2.4.3-1: VAL performance analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement) |

#### 8.2.4.4	Data collection subscription request

Table 8.2.4.4-1 describes information elements for the VAL performance analytics subscription request from the ADAE server to the Data Producer or the A-DCCF.

Table 8.2.4.4-1: Data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Data Collection Event ID | M | The identifier of the data collection event |
| Data Collection requirements | M | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed |
| Target VAL UE ID(s) and address | O | The VAL UE(s) identifiers and IP address(es) for which the data collection subscription apply |
| Target VAL server ID | O | This identifier shows the target VAL server for which the data collection subscription applies (for procedure in 8.2.2) |
| Area of Interest | O | The geographical or service area for which the requirement request applies |
| Time validity | O | The time validity of the request |

#### 8.2.4.5	Data collection subscription response

Table 8.2.4.5-1 describes information elements for the Data collection subscription response from the Data Producer (or the A-DCCF) to the ADAE server.

Table 8.2.4.5-1: Data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the data collection subscription request (positive or negative acknowledgement) |

#### 8.2.4.6	Data Notification

Table 8.2.4.6-1 describes information elements for the Data Notification from the Data Producer to the ADAE server.

Table 8.2.4.6-1: Data notification

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The result of the data collection subscription request (positive or negative acknowledgement) |
| Target VAL UE ID and address | M (NOTE) | The VAL UE(s) identifiers and IP address(es) for which the data apply |
| Target VAL server ID | M (NOTE) | This identifier of the target VAL server for which the data applies (for procedure in 8.2.2) |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example “VAL server performance analytics” for procedure in 8.2.2, or “VAL session performance analytics” for procedure in 8.2.3. |
| Data Type | O | The type of reported data samples which can be UE data, network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non real time). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter (e.g. RTT deviation) based on subscription |
| NOTE: One of these shall be present based on the data collection event |  |  |

#### 8.2.4.7	Analytics Notification

Table 8.2.4.7-1 describes information elements for the Analytics Notification from the ADAE server to the VAL server / Consumer.

Table 8.2.4.7-1: Analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “VAL server performance analytics” for procedure in 8.2.2, or “VAL session performance analytics” for procedure in 8.2.3. |
| Analytics Type | O | The type of analytics based on the event, which can be offline or online analytics, ML-enabled analytics, statistics, or predictive analytics. |
| Analytics Output | M | The predictive or statistical parameter, which can be: - A VAL server predicted or expected performance change or sustainability - A VAL session predicted or expected performance change of sustainability |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

## 8.3	Procedure on support for slice-specific application performance analytics

### 8.3.1	General

This clause describes the procedure for supporting slice-tailored application performance analytics.

### 8.3.2	Procedure

Figure 8.3.2-1 illustrates the procedure where the VAL server performance analytics are performed based on data collected from the ongoing VAL sessions as well as data from the DN (VAL server, DN database or networking stack at DN) for a specific slice.

Pre-conditions:

1.	ADAEC is connected to ADAES.

Figure 8.3.2-1: ADAES support for slice-related performance analytics

1.	The consumer of the ADAES analytics service sends a subscription request to ADAES and provides the analytics event ID e.g. "slice perf prediction", the target S-NSSAI, DNN, NSI ID, the time validity and area of the request, the required confidence level, whether offline and/or online analytics are needed etc.

2.	The ADAES sends a subscription response as an ACK to the consumer.

3.	The ADAES subscribes to the Data Sources with the respective Data Collection Event ID and the requirement for data collection related to the request slice(s). Such requests can be towards:

-	OAM for providing PM data related to the requested slice / NSI. Alternatively, if the interaction to OAM happens via NSCE layer (see TS 23.435 [6]), such subscription can be performed to NSCE (where ADAES is acting as VAL server).

-	NWDAF for providing slice related analytics for the given area and time horizon (indicated in step 1). Such analytics can be the slice load level related network data analytics, or the service experience related network data analytics for a given slice

4.	The ADAES based on subscription, receives PM data notification from OAM or from NSCE (via OAM APIs or NSCE-S APIs)

5.	The ADAES based on subscription, receives the requested NWDAF analytics outputs. Such analytics can be:

-	network slice or NSI statistics or predictions (clause 6.3.3 of TS 23.288 [4])

-	per slice instance service experience stats or predictions (clause 6.4.3 of TS 23.288 [4])

6.	The ADAES can also provide analytics on the VAL session performance (based on the procedure of clause 8.2.2 step 11 or clause 8.2.3 step 12) and filters the analytics only for the sessions which are connected to that requested slice for the area of interest.

7.	The ADAES abstracts or correlates the data/analytics from steps 4-6 and provides analytics on the slice or NSI performance for the the target VAL application/server. For example, such analytics can be about the min/average/max predicted RTT / end to end latency for the VAL application/server if this server uses a given slice/NSI (or for a list of given slices) within an area of interest.

8.	The ADAES sends the analytics to the consumer, as a slice-specific performance analytics notification message.

### 8.3.3	Information flows

#### 8.3.3.1	General

The following information flows are specified for slice-specific application performance analytics based on 8.3.2.

#### 8.3.3.2	Slice-specific performance analytics subscription request

Table 8.3.3.2-1 describes information elements for the slice-specific performance analytics subscription request from the consumer (VAL server / NSCE server) to the ADAE server.

Table 8.3.3.2-1: Slice-specific performance analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the analytics consumer |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “slice-specific application performance analytics” |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics |
| Slice identifier | M | The identifier of the target slice or slice instance, i.e. S-NSSAI, NSI ID or ENSI. |
| DNN | O | The target DNN for which the request applies |
| Target VAL UE ID(s) | O | The VAL UE(s) for which the analytics subscription applies |
| Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies (for procedure in 8.2.2) |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

#### 8.3.3.3	Slice-specific performance analytics subscription response

Table 8.3.3.3-1 describes information elements for the slice-specific performance analytics subscription response from the ADAE server to the consumer (VAL/NSCE server).

Table 8.3.3.3-1: Slice-specific performance analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement) |

#### 8.3.3.4	Slice-specific performance analytics notification

Table 8.3.3.4-1 describes information elements for the slice-specific performance analytics notification from the ADAE server to the Consumer.

Table 8.3.3.4-1: Slice-specific performance analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “slice-specific application performance analytics” |
| Analytics Type | O | The type of analytics based on the event, which can be offline or online analytics, ML-enabled analytics, statistics, or predictive analytics. |
| Analytics Output | M | The predictive or statistical parameter, which can be for a target slice or slice instance the: - A VAL server predicted or expected performance change or sustainability - A VAL session predicted or expected performance change of sustainability |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

## 8.4	Procedure on support for UE-to-UE application performance analytics

### 8.4.1	General

This clause describes the procedure for supporting UE-to-UE application performance analytics.

### 8.4.2	Procedure

Figure 8.4.2-1 illustrates the procedure where the VAL session performance analytics are performed based on data collected from the ongoing VAL UE-to-UE sessions.

Pre-conditions:

1.	ADAECs are connected to ADAES.

Figure 8.4.2-1: ADAES support for VAL session performance analytics

1. The consumer of the ADAES analytics service sends a subscription request to ADAES and provides the analytics event ID e.g. "VAL UE to UE session prediction", the target VAL UE ID or group of UE IDs, the VAL session / service ID, the time validity and area of the request, the required confidence level, exposure level for providing UE to UE analytics. Such request can also include whether the analytics notification shall be periodic or based on an expected application QoS change (in that case also the thresholds can be provided at the request)

2.	The ADAES sends a subscription response as an ACK to the consumer.

3.	The ADAES selects the corresponding ADAEC #1 of the VAL UE 1 where the session performance analytics need to be performed. Such UE can be for example a capable and authorized UE from the involved VAL UEs within the service or group, e.g. a group lead.

4.	The ADAES sends a UE to UE analytics request to the ADAEC #1 with the analytics event ID and the configuration of the reporting required (e.g., periodic, based on threshold or event). Such request also includes the application QoS attributes to be analyzed (latency, jitter, PER,..)A session starts between the VAL UE #1 and a VAL UE #2 (or more VAL UEs).

5.	The ADAEC #1 starts collecting data from the corresponding VAL UE(s) based on the request. Such data can be about the latency, throughput, jitter, QoE measurements, PQI load, etc.  The data can be collected by ADAEC #1 from other ADAECs via ADAE-C interface, or from the VAL clients (VAL client to VAL client interaction is out of scope).

6.	The ADAEC either detects or predicts an application QoS change (depending on the authorization of ADAEC to perform analytics). Such change can be for example an application QoS downgrade related to the UE-to-UE session latency, or the PER/channel losses higher than a predefined threshold, for a given time horizon with a certain confidence level.

7.	The ADAEC sends the analytics to the ADAES in a UE-to-UE analytics response message.

8.	The ADAES based on the received response, confirms/verifies the analytics received or provides analytics (in case that data were reported) for the UE-to-UE session. Such analytics can be about predicting the application QoS change for the UE-to-UE session.

9.	The ADAES sends the derived analytics notification to the consumer.

NOTE:	The mechanism for analytics collection from the UE side (steps 4, 7) shall align with the SA4 mechanism for generic data collection from the UE (TS 26.531 [3]).

### 8.4.3	Information flows

#### 8.4.3.1	General

The following information flows are specified for UE-to-UE session performance analytics based on 8.4.2

#### 8.4.3.2	UE-to-UE session performance analytics subscription request

Table 8.4.3.2-1 describes information elements for the UE-to-UE session performance analytics subscription request from the consumer (VAL server) to the ADAE server.

Table 8.4.3.2-1: UE-to-UE session performance analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the analytics consumer (VAL server) |
| Analytics ID | M | The identifier of the analytics event. This ID can be equivalent to “UE-to-UE session performance analytics” |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics |
| List of VAL UE IDs and addresses | M | The VAL UEs for which the analytics subscription applies |
| VAL service ID | O | The VAL service for which the subscription applies |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

#### 8.4.3.3	UE-to-UE session performance analytics subscription response

Table 8.4.3.3-1 describes information elements for the UE-to-UE session performance analytics subscription response from the ADAE server to the VAL server.

Table 8.4.3.3-1: UE-to-UE session performance analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement) |

#### 8.4.3.4	UE-to-UE analytics request

Table 8.4.3.4-1 describes information elements for the UE-to-UE Analytics request from the ADAE server to the ADAE client.

Table 8.4.3.4-1: UE-to-UE analytics request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Analytics ID | M | The identifier of the analytics event (Analytics ID=’UE to UE session analytics’). |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the data/analytics apply |
| Application QoS attributes | M | The QoS attributes (latency, rate, PER,..) to be analyzed at the ADAE client. |
| Reporting configuration | O | The configuration of analytics reporting including the format, frequency of analytics report, data collection requirements, abstraction needed, etc |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

#### 8.4.3.5	UE-to-UE analytics response

Table 8.4.3.5-1 describes information elements for the UE-to-UE analytics response from the ADAE client to the ADAE server.

Table 8.4.3.5-1: UE-to-UE analytics response

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the analytics apply |
| Analytics Output | M | The reported analytics for the UE to UE sessions, which can be in form of offline stats/historical data or predictions on the requested QoS parameter based on the analytics event. |

#### 8.4.3.6	ADAE Analytics Notification

Table 8.4.3.6-1 describes information elements for the ADAE Analytics Notification from the ADAE server to the VAL server / Consumer.

Table 8.4.3.6-1: ADAE Analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be “UE to UE session performance analytics” |
| Analytics Type | O | The type of analytics based on the event, which can be offline or online analytics, ML-enabled analytics, statistics, or predictive analytics. |
| Analytics Output | M | The predictive or statistical parameter, which can be: - A VAL UE to UE session predicted or expected performance change - A VAL UE to UE session performance sustainability over a given time horizon/area |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

## 8.5	Procedure on support for location accuracy analytics

### 8.5.1	General

This clause describes the procedure for supporting location accuracy analytics.

### 8.5.2	Procedure

Figure 8.5.2-1 illustrates the procedure for location accuracy analytics enablement solution.

Pre-conditions:

1.	ADAES is connected to A-ADRF.

2.	ADAES has discovered SEAL LMS or FLS.

Figure 8.5.2-1: Location accuracy analytics procedure

1.	The VAL server makes a subscription request to ADAE server for location accuracy prediction/stats, including an analytics event ID (e.g. "location accuracy prediction" or "location accuracy sustainability"), an analytics request type (if not identified specifically at the event ID) which can be the location accuracy prediction for a given location X and/or for a given UE/app. The request may include also the target area, a target VAL service, or a VAL UE, group of UEs or the VAL service, time of day, accuracy threshold and requirements. If the VAL UEs are provided by the VAL server, this request may also include the expected route or a set of waypoints for the UEs of the VAL application.

2.	The ADAE server sends a location accuracy analytics subscription response as an ACK to the VAL server.

3.	The ADAE server discovers and maps the Data Sources with the respective analytics event ID for collecting location data for the corresponding VAL UEs or VAL service or area.

4.	The ADAE server subscribes for NWDAF UE mobility analytics per VAL UE (for all the VAL UEs) and gets notification on the per UE location/mobility analytics based on TS 23.288 clause 6.7.2. Such analytics may be requested for a list of waypoints per UE route (if indicated at step 1). The ADAE server subscribes also for SEAL LMS location reports for the respective VAL UEs or location reports from all VAL UEs within the requested area.

5.	The ADAE server optionally requests and receive location accuracy historical analytics /data from A-ADRF for the corresponding VAL UEs or VAL service area.

6.	Based on the request, the ADAE server receives location accuracy historical analytics /data from A-ADRF for the corresponding VAL UEs or VAL service area.

7.	The ADAE server abstracts or correlates the data/analytics from steps 4-6 and provides analytics on the location accuracy for the target VAL application. Depending on the event ID in step 1, the ADAE server can indicate whether the location accuracy is sustainable or is predicted to be downgraded or can be upgraded and become more granular (e.g. from meter to decimetre).

8.	The ADAE server sends the location accuracy analytics notification to the consumer.

### 8.5.3	Information flows

#### 8.5.3.1	General

The following information flows are specified for location accuracy analytics based on 8.5.2

#### 8.5.3.2	Location accuracy analytics subscription request

Table 8.5.3.2-1 describes information elements for the location accuracy analytics subscription request from the VAL server to the ADAE server.

Table 8.5.3.2-1: Location accuracy analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Analytics ID | M | The identifier of the location accuracy analytics event. This ID can be for example "location accuracy prediction" or "location accuracy sustainability" depending on the expected outcome |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics, and whether analytics are offline / online. |
| VAL UE ID(s) or Group ID | M | The identity of the VAL UE(s) or group of UEs for which the analytics subscription applies |
| VAL service ID | O | The identifier of the V2X service for which location accuracy analytics is requested |
| Location accuracy requirements | M | The accuracy threshold and VAL requirements |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |
| UE mobility / route information | O | Information on the target UE or group UE mobility including the expected route/set of waypoints |

#### 8.5.3.3	Location accuracy analytics subscription response

Table 8.5.3.3-1 describes information elements for the location accuracy analytics subscription response from the ADAE server to the VAL server.

Table 8.5.3.3-1: Location accuracy analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement) |

#### 8.5.3.4	Location accuracy data request

Table 8.5.3.4-1 describes information elements for the location accuracy data request from the ADAE server to the A-ADRF.

Table 8.5.3.4-1: Location accuracy data request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Analytics ID | M | The identifier of the analytics event |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the data/analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Reporting configuration | O | The configuration of data reporting including the format, data collection requirements, abstraction needed, etc |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

#### 8.5.3.5	Location accuracy data response

Table 8.5.3.5-1 describes information elements for the location accuracy data response from the A-ADRF to the ADAE server.

Table 8.5.3.5-1: Location accuracy data response

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Analytics Output | M | The reported analytics for the location accuracy, which can be in form of offline stats/historical data for a specific VAL service or for particular UE(s) or group of UEs |

#### 8.5.3.6	Location accuracy analytics notification

Table 8.5.3.6-1 describes information elements for the location accuracy analytics notification from the ADAE server to the VAL server.

Table 8.5.3.6-1: Location accuracy analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| Analytics Type | O | The type of analytics based on the event, which can be offline or online analytics, ML-enabled analytics, statistics, or predictive analytics. |
| Analytics Output | M | The predictive or statistical parameter, which can be: - A predicted or expected location accuracy change (downgrade or upgrade) for a particular VAL service or UEs - the location accuracy sustainability for a VAL service or UE/group of UEs over a given time horizon/area |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

## 8.6	Procedure for supporting service API analytics

### 8.6.1	General

This clause describes the procedure for supporting service API analytics. Such analytics can be for one or more service APIs for a service produced by one or more service producers within the 5GS or enablement layer or the DN side (e.g., application server).

### 8.6.2	Procedure

Figure 8.6.2-1 illustrates the procedure for service API analytics enablement solution.

Pre-conditions:

1.	ADAES is registered to CCF

Figure 8.6.2 -1: Service API analytics procedure

1.	The subscribing entity (VAL server / API invoker, API provider) sends a service API event subscription request to the ADAE server to receive analytics for one or more service APIs.

2.	Upon receiving the event subscription request from the subscribing entity, the ADAE server checks for the relevant authorization for the event subscription. If the authorization is successful, the ADAE server stores the subscription information.

3.	The ADAE server sends a service API event subscription response indicating successful subscription

4.	Upon sending the subscription response, the ADAE server requests to collect API logs to be used to derive analytics and triggers API invocation log pull request towards the CAPIF core function. The API invocation log fetch request indicates the API (or list of APIs) for which logs are required. Based on the ADAE server deployment, this can be a Query service API log request which is performed via CAPIF_Auditing API as specified in 3GPP TS 23.222 [x].

5.	The CCF authorizes the request and fetches the API logs from the storage unit. CCF then sends the requested information to the ADAE server via a query service API log response

6.	The ADAES may also request service API historical analytics /data from A-ADRF for the corresponding service APIs.

7.   Based on the request, the ADAES receives historical analytics/data for the service APIs.

8.	The ADAE server authorizes and anonymizes the API logs (if not performed by CCF) and abstracts based on exposure level. The exposure level can be known based on pre-configuration by the OAM or based on the subscription and type of invoker. The ADAE server then derives analytics on the target service API(s) based on the logs received from the CCF. Such analytics are predictions/stats for the API status based on the analytics event.

9.	The ADAE server sends the analytics as event notifications to all the subscribing entities that have subscribed for the event matching the criteria. If a notification reception information is available as part of the subscribing entity event subscription, then the notification reception information is used by the ADAE server to send event notifications to the subscribing entity.

### 8.6.3	Information flows

#### 8.6.3.1	General

The following information flows are specified for service API analytics based on 8.6.2.

#### 8.6.3.2	Service API event subscription request

Table 8.6.3.2-1 describes information elements for the service API event subscription request from the consumer (VAL server, API provider) to the ADAE server.

Table 8.6.3.2-1: Service API event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The information to determine the identity of the subscribing entity (consumer) |
| Service API information | M | The service API name or type |
| Analytics ID and criteria | M | The event criteria include event type information relevant to the prediction or stats on the number of failure API invocations, API availability, frequency and occurrence of API version changes, API location changes for the target API, etc |
| Time Validity and horizon | O | Time validity of the request and time horizon for the predictions |
| Area of interest | O | Geographical or topological area for which the subscription applies |
| Notification reception information | O | The information of the subscribing entity for receiving the notifications for the event. |

#### 8.6.3.3	Service API event subscription response

Table 8.6.3.3-1 describes information elements for the service API event subscription response to the consumer (VAL server, API provider) from the ADAE server.

Table 8.6.3.3-1: Service API event subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement) |

#### 8.6.3.4	Historical service API logs request

Table 8.6.3.4-1 describes information elements for the historical service API logs request from the ADAE server to the A-ADRF.

Table 8.6.3.4-1: Historical service API logs request

| Information element | Status | Description |
| --- | --- | --- |
| Service API log requestor information | M | Identity information of the originated application querying service API log request |
| ADAES ID | M | Identity information of the ADAES |
| Service ID or UE ID | M | Identity of the application service or UE for which the historical API invocations apply |
| Target API(s) information | M | Information on target API or list of target APIs (name or type) |
| >Query information | O | List of query filters such as invoker's ID and IP address, service API name and version, input parameters, and invocation result |
| > API aggregation abstraction flag | O | What type of aggregation or abstraction/filtering needs to be applied |
| Reporting format configuration | O | The logs reporting configuration (frequency, periodicity etc) |
| Area of validity | O | The geographical area for which the request applies |
| Time validity | O | The time validity for the request |
| Exposure level requirement | O | The level of exposure requirement (e.g. permissions on the logs like read/write/delete..) for the logs to be exposed |

#### 8.6.3.5	Historical service API logs response

Table 8.6.3.5-1 describes information elements for the historical service API logs response to the ADAE server from the A-ADRF.

Table 8.6.3.5-1: Historical service API logs response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Identity information of the originated application querying service API log request |
| Service ID or UE ID | M | Identity of the application service or UE for which the API invocations apply |
| Target API (s) information | M | The target service API name or type |
| >Target API(s) logs | M | The API logs based on the subscription event. This may include the number of failure API invocations, API availability, frequency and occurrence of API version changes, API location changes for the target API, API throttling events, number of API invocations for a given area and time etc |
| >Reporting info | O | The time and area for which the reporting applies |

#### 8.6.3.6	Service API analytics notification

Table 8.6.3.5-1 describes information elements for the service API analytics notification to the subscriber/consumer from the ADAE server.

Table 8.6.3.6-1: Service API analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The information to determine the identity of the subscribing entity (consumer) |
| Service API information | M | The service API name or type for which analytics apply |
| Analytics ID | M | The analytics ID or event ID for which service API analytics is performed |
| Analytics Output | M | Stats or predictions based on abstracted or anonymized API logs (for example number of failure API invocations, API availability, frequency and occurrence of API version changes, API location changes for the target API, API throttling events, number of API invocations for a given area and time, API load statistics for a given edge network, etc) |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| Area of validity | O | Geographical or topological area for which the analytics apply. |

## 8.x	<procedure name>

Add a copy of this clause for each procedure, adding a title, a general description, information flows and the procedure. Include an APIs clause if APIs are needed for the procedure.

### 8.x.1	General

### 8.x.2	Procedure

### 8.x.3	Information flows

# 9	ADAE layer APIs

This clause describes the ADAE layer APIs which are needed for the specified ADAE capabilities.

Annex A (informative):
Deployment scenarios

# A.1	General

This clause provides the different deployment models for ADAE services. There could be three deployment options:

-	ADAES can be deployed at a centralized cloud platform, and collects data from multiple EDNs

-	ADAES can be deployed at the edge platform

-	Coordinated ADAES deployment, where multiple ADAE services are deployed in edge or central clouds. Such deployment allows for local-global analytics for system wide optimization

# A.2	Deployment model #1: Cloud-deployed ADAES

In this deployment, as shown in Figure A.2-1, the ADAES is centrally located and can provide analytics services to different consumers including, edge servers, VAL servers, as well as to other SEAL servers (e.g. NSCE).

The statistics/predictions that the ADAES provides are applicable to the ADAES service area, which can be provided for the entire PLMN.

Figure A.2-1: Cloud deployed ADAES

# A.3	Deployment model #2 Edge-deployed ADAES

In this deployment, as shown in Figure A.3-1, the ADAES is located at the EDN and provides analytics services to the EAS and EES at the edge platform. ADAES can be deployed by the ECSP or the MNO to provide analytics for the application or edge parameters.

The statistics/predictions that the edge deployed ADAES are applicable to the ADAES service areas (as shown in the example in Fig A.2-2), which are equivalent to the EES/EAS service areas. Such analytics can be about the edge load or the EAS performance and can be provided to consumers within EDN.

In this deployment the interaction between edge deployed ADAES is possible for exchanging edge/application analytics for application mobility scenarios or for cases when ADAES #1 and #2 service areas have overlapping coverage.

Figure A.3-1: Edge deployed ADAES

# A.4	Deployment model #3: Coordinated ADAES deployment

In this deployment, multiple ADAESs can be located at different EDNs/DNs and can be deployed by the same ADAE provider.  Such coordinated deployments allow the local – global analytics derivation (which may be needed for improving the analytics confidence level). The centrally deployed ADAES can also act as ADAE analytics aggregator entity and configures the edge deployed ADAES to derive analytics on different sub-areas.

One example is the use of analytics for the EDN#1 or EDN#2 load which will help predicting the VAL server performance at a centrally located ADAES. Such deployment is also applicable for ML-based analytics methods, like supervised learning, where the centrally located ADAES acts as ML model training entity, and the edge located ADAESs can act as ML model inference entities (using edge data to improve the prediction accuracy).

The statistics/predictions that the edge deployed ADAES correspond to the ADAES service areas (as shown in the example in Fig A.4-1), which is equivalent to the EES/EAS service areas. The central ADAE server covers all PLMN area and is used to coordinate or jointly perform analytics with the distributed ADAES. Such analytics services can be provided to consumers at the central DN, like the VAL servers or SEAL services or even at the PLMN side (e.g. NWDAF consuming service experience analytics).

Figure A.4-1: Coordinated deployment of ADAES

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-10 | SA6#51-e |  |  |  |  | TS skeleton | 0.0.0 |
| 2022-10 | SA6#51-e |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-222940, S6-222941, S6-222942, S6-222943 | 0.1.0 |
| 2022-11 | SA6#52 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-223235, S6-223237, S6-223455, S6-223456, S6-223494 | 0.2.0 |
| 2023-01 | SA6#52-bis-e |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-230262, S6-230363, S6-230247 | 0.3.0 |