| 3GPP TR 23.700-12 V0.4.0 (2021-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Study on enhanced IP Multimedia Subsystem (IMS) to 5GC integration Phase 2; CT WG4 aspects (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Architecture Assumptions	7

5	Key Issues	7

5.1	Key issue #1: Routing of IMS traffic via a localized UPF	7

5.1.1	Description	7

5.1.2	Requirements	7

5.2	Key issue #2: How can IMS utilize services provided by 5GC NFs other than PCF	7

5.2.1	Description	7

5.2.2	Requirements	7

5.x	Key issue #X: <Key Issue Title>	8

5.x.1	Description	8

5.x.2	Requirements	8

6	Solutions	8

6.1	Solution #1: Conveying UPF FQDN to IMS nodes	8

6.1.1	Description	8

6.1.2	Impacts on existing nodes and functions	8

6.1.2.1	SMF	8

6.1.2.2	UDM	8

6.1.2.3	UDR	8

6.1.2.4	HSS	8

6.1.2.5	S-CSCF	9

6.1.2.6	BGCF	9

6.1.2.7	IMS-AS	9

6.2	Solution #2: IMS utilize services provided by AMF	9

6.2.1	Description	9

6.2.2	Impacts on existing nodes and functions	11

6.2.3	Solution Evaluation	12

6.3	Solution #3: P-CSCF directly communicates UDM	12

6.3.1	Description	12

6.3.2	Impacts on existing nodes and functions	12

6.3.2.1	P-CSCF	12

6.3.3	Solution Evaluation	12

6.4	Solution #4: Conveying UPF service area to IMS nodes via PCF N5	13

6.4.1	Description	13

6.4.2	Impacts on existing nodes and functions	14

6.4.2.1	SMF	14

6.4.2.2	PCF	14

6.4.2.3	P-CSCF	14

6.4.2.4	IMS-AS	14

6.4.2.5	MGCF	14

6.4.2.6	BGCF	14

6.5	Solution #5: Discovery of NFs from IMS nodes	14

6.5.1	Description	14

6.5.2	Impacts on existing nodes and functions	15

6.5.2.1	IMS-AS	15

6.5.2.2	MGCF	15

6.5.2.3	NRF	15

6.x	Solution #X: <Solution Title>	15

6.x.1	Description	15

6.x.2	Impacts on existing nodes and functions	15

6.x.3	Solution Evaluation	15

7	Overall Evaluation	15

7.1	Evaluation of solutions addressing key issue #1	15

8	Conclusions	16

Annex <A> (informative): Change history	17

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The aim of this Technical Report is to study and specify potential enhancements to the IMS architecture to enable IMS FE to integrate with the 5GC network functions to enable IMS applications to directly leverage the features and capabilities of 5GC.

The specific capabilities of 5GC included in this study:

-	How can IMS utilize services provided by 5GC NFs other than PCF, such as HSS/UDM and AMF.

-	How can localized routing of IMS media and signalling traffic be leveraged.

Editor's note:	Additional scope needs to be studied.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[3]	3GPP TS 29.562: "Home Subscriber Server (HSS) Services for interworking with the IP Multimedia Subsystem (IMS); Stage 3".

[4]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

# 4	Architecture Assumptions

# 5	Key Issues

## 5.1	Key issue #1: Routing of IMS traffic via a localized UPF

### 5.1.1	Description

The 5GC enables traffic to route via localized UPF close to the edge of the network (in some cases adjacent to the RAN nodes). Some IMS services may be able to benefit from the lower latency and/or lower backhaul requirements that such deployments can enable, however current IMS applications and services are not able to leverage these capabilities. This key issue investigates the interactions required to leverage localized routing of IMS media and signalling traffic, the changes to enable effective control and management of such routing, and impacts of mobility and roaming.

### 5.1.2	Requirements

The existing 5GC and IMS architecture does not specifically support having user plane and IMS media plane entities in the same location and hence user plane path handling is not optimized. To optimise the user plane path, a location of the MGCF and MRF close to the actual UPF (that the user plane needs to traverse) is desirable. For instance, UPF and MGCF could be located in the same data centre.

When selecting an MGCF or MRF, IMS nodes shall be enabled to base this selection on the actually used UPF.

## 5.2	Key issue #2: How can IMS utilize services provided by 5GC NFs other than PCF

### 5.2.1	Description

All of the 5GC control plane NFs are specified as service based and any NF can be consumer of the services provided by other NFs, if supported by the procedures.

### 5.2.2	Requirements

This key issue addresses IMS and 5GC service interfaces aspects on:

-	How IMS can utilize services provided by 5GC NFs other than PCF.

-	How to determine the UE location in IMS entities with service operations (e.g. determination of the visited PLMN ID in case of Home-Routed VoLTE roaming).

## 5.x	Key issue #X: <Key Issue Title>

### 5.x.1	Description

### 5.x.2	Requirements

# 6	Solutions

## 6.1	Solution #1: Conveying UPF FQDN to IMS nodes

### 6.1.1	Description

This is a solution to key issue #1.

This solution proposes to convey the UPF FQDN or a more generic information like "user-plane-locality" or "voice-media-locality" for the IMS PDU session from SMF to UDM during SMF registration. The UDM stores the IMS PDU session's UPF FQDN (or "user-plane-locality" or voice-media-locality") in the UDR as part of the SMF registration. The HSS retrieves the IMS PDU session's UPF FQDN (or "user-plane-locality" or voice-media-locality") from the UDR and forwards it to the S-CSCF via Cx when an S-CSCF registers. The UPF FQDN (or "user-plane-locality" or voice-media-locality") is then conveyed from S-CSCF via SIP to the BGCF.

When the BGCF determines that selection of an MGCF close to the actual UPF is required, it makes use of the conveyed UPF FQDN (or "user-plane-locality" or voice-media-locality") and maps it based on a locally configured mapping table to the optimal MGCF.

When the IMS-AS determines that selection of an MGCF or MRF close to the actual UPF is required, it retrieves the UPF FQDN (or "user-plane-locality" or voice-media-locality") via HSS from the UDR and selects (based on a locally configured mapping table) the optimal MRF for the actual UPF.

### 6.1.2	Impacts on existing nodes and functions

#### 6.1.2.1	SMF

When the SMF makes use of the Nudm_UECM_registration service operation to register for the IMS PDU session at the UDM, it needs to convey the actual UPF FQDN (or "user-plane-locality" or voice-media-locality") to the UDM.

#### 6.1.2.2	UDM

When the UDM receives the actual UPF FQDN (or "user-plane-locality" or voice-media-locality") within the Nudm_UECM_registration service operation, it needs to store the UPF FQDN (or "user-plane-locality" or voice-media-locality") as part of the SmfRegistration in the UDR.

#### 6.1.2.3	UDR

When the UDM makes use of the Nudr_DR_Create service operation to store the SMF registration for the IMS PDU session, it needs to store the received UPF FQDN (or "user-plane-locality" or voice-media-locality").

When the HSS makes use of the Nudr_DR_Query service operation to retrieve SMF registration for the IMS PDU session, the UDR needs to respond with the SmfRegistration including the stored UPF FQDN (or "user-plane-locality" or voice-media-locality").

#### 6.1.2.4	HSS

When the HSS receives a registration request from the S-CSCF or a request from the IMS-AS to provide the UPF FQDN (or "user-plane-locality" or voice-media-locality"), in needs to make use of the Nudr_DR_Query service operation to retrieve the UPF FQDN (or "user-plane-locality" or voice-media-locality") from the UDR as part of the SmfRegistration for the IMS PDU session, and return the retrieved UPF FQDN (or "user-plane-locality" or voice-media-locality") to the IMS-AS or S-CSCF.

#### 6.1.2.5	S-CSCF

The S-CSCF forwards the received UPF FQDN (or "user-plane-locality" or voice-media-locality") to the BGCF.

#### 6.1.2.6	BGCF

When the BGCF determines that an MGCF close to the actual UPF need to be selected, it makes use of the UPF FQDN (or "user-plane-locality" or voice-media-locality") received from the S-CSCF and maps it according to a locally configured mapping table to an optimal MGCF.

#### 6.1.2.7	IMS-AS

When the IMS-AS determine that an MRF close to the actual UPF need to be selected, it needs to retrieve the actual UPF FQDN (or "user-plane-locality" or voice-media-locality") for the IMS PDU session from the HSS. The IMS-AS then makes use of the UPF FQDN (or "user-plane-locality" or voice-media-locality") and maps it according to a locally configured mapping table to an optimal MRF.

## 6.2	Solution #2: IMS utilize services provided by AMF

### 6.2.1	Description

This solution addresses Key Issue #2: How can IMS utilize services provided by 5GC NFs other than PCF.

In IMS network, the P-CSCF needs to be notified of some events related to access network, e.g. UE location information and other access network related events. This information will be used in IMS network for billing, LI, service trigger and so on. In 4G, the P-CSCF subscribes to notification of these events over Rx interface. Since 5GC supports SBI, IMS network is able to use the SBI provided by AMF to subscribe to notification of these events.

Figure 6.2.1-1 illustrates how IMS obtains the AMF address serving the UE during IMS registration procedure.

Figure 6.2.1-1: IMS obtains the AMF instance ID during IMS registration procedure

1-12.	Normal IMS registration procedure.

13.	The S-CSCF requests the AMF instance ID in SAR from the HSS/UDM.

14.	The HSS/UDM sends AMF instance ID in the SAA to the S-CSCF.

15.	The S-CSCF sends the AMF instance ID to the P-CSCF in the SIP 200 OK.

16.	The P-CSCF sends SIP 200 OK to the UE as in normal IMS registration procedure.

17.	(Option A) The P-CSCF subscribes to AMF change event from S-CSCF.

18.	(Option A) The S-CSCF responds to P-CSCF.

19.	The S-CSCF subscribes to notification of AMF change to the HSS/UDM.

20.	When the AMF changes, the HSS/UDM notifies the S-CSCF.

21a.	(Option A) The S-CSCF sends SIP NOTIFY with the new AMF instance ID to the P-CSCF.

21b.	(Option B) The S-CSCF sends SIP MESSAGE with the new AMF instance ID to the P-CSCF.

Editor's note:	If the solution is agreed, Nhss services specified in 3GPP TS 29.562 [3] needs to be enhanced in the normative stage.

Once the IMS (P-CSCF) obtains the instance ID of the AMF serving the UE, the IMS (P-CSCF) can subscribe to notification of the events supported by AMF e.g. events related to UE location information and access network information. Before the IMS (P-CSCF) invoking services provided by AMF, it needs to query NRF with the received AMF instance ID to get the AMF endpoints to which it can invoke service request.

Figure 6.2.1-2 illustrates the P-CSCF utilizes services provides by AMF to subscribe to notification of the UE location information.

Figure 6.2.1-2: P-CSCF utilizes Services provided by AMF to subscribe to notification of the UE location information

1.	The P-CSCF receives the SIP INVITE message.

2.	The P-CSCF queries NRF with previously received AMF instance ID, to get the AMF profile.

3.	The NRF responds to P-CSCF with the AMF profile.

4.	The P-CSCF invokes Namf_EventExposure_Subscribe service operation, based on the AMF profile received in step 3, for event subscription to UE location changes.

NOTE:	The Namf_EventExposure service allows one time reporting, i.e. the subscription to notification is only for one time reporting.

5.	When the UE location changes, the AMF invokes Namf_EventExposure_Notify to notify the P-CSCF of the UE location.

When the UE moves from the old AMF to the new AMF, the S-CSCF receives notification from the UDM/HSS, and the S-CSCF notifies the P-CSCF of the new AMF address. The P-CSCF invokes Namf_EventExposure_UnSubscribe service operation on the old AMF to unsubscribe the event notification and invokes Namf_EventExposure_Subscribe service operation on the new AMF service as defined in clause 5.2.2 of the TS 23.502 [2].

Editor's note:	It is FFS how this solution works in the case there are two AMFs serving a single UE for 3GPP access and non 3GPP access respectively.

### 6.2.2	Impacts on existing nodes and functions

Editor's note:	This clause describes impacts to existing entities and interfaces.

### 6.2.3	Solution Evaluation

Editor's note:	This clause provides an evaluation of the solution.

## 6.3	Solution #3: P-CSCF directly communicates UDM

### 6.3.1	Description

This solution addresses Key Issue #2: How can IMS utilize services provided by 5GC NFs other than PCF. This solution differs from solution #2 mainly on how the P-CSCF gets the serving AMF of the UE.

This solution proposes P-CSCF directly communicates with UDM to get the necessary information, the procedure is shown in Figure 6.3.1-1.

Figure 6.3.1-1: P-CSCF directly communicates with UDM

1.	The P-CSCF invoke Nnrf_NFDiscovery service towards a preconfigured NRF to get the UDM which can serve the UE, in the service request the P-CSCF include the SUPI or GPSI of the UE as query. The P-CSCF derives the SUPI or GPSI from the IMPI or IMPU of the UE.

2.	The NRF responds to P-CSCF with the profile of the UDM which can serve the UE.

3.	The P-CSCF may invoke Nudm_UECM service towards the UDM to get the context information of the UE.

4.	The P-CSCF may invoke Nudm_SDM service towards the UDM to get the subscription information of the UE.

5.	The P-CSCF may invoke Nudm_EventExposure service towards the UDM to subscribe events of interest.

### 6.3.2	Impacts on existing nodes and functions

#### 6.3.2.1	P-CSCF

P-CSCF shall be able to invoke services provided by NRF and UDM.

### 6.3.3	Solution Evaluation

Editor's note:	This clause provides an evaluation of the solution.

## 6.4	Solution #4: Conveying UPF service area to IMS nodes via PCF N5

### 6.4.1	Description

This is a solution to key issue #1.

This solution proposes to convey the UPF service area for the IMS PDU session from SMF to PCF and on to P-CSCF during multimedia session establishment. The PCF either stores the UPF service area as part of the SM policy or fetches the UPF service area from the SMF when requested by P-CSCF. PCF may also subscribe to a new event "UPF service area change" upon establishment of the SM policy association.

When the P-CSCF receives a multimedia session request, it fetches the UPF service area (and possibly network provided location information, see Annex B.8 in 3GPP TS 29.514 [4]) and selects (based on a locally configured mapping from UPF service area) the optimal MGW (e.g. ATGW) for the actual UPF service area.

The P-CSCF forwards the UPF service area retrieved from PCF in the SIP request (e.g. as part of the PANI header). Other IMS functions (e.g. MGCF, IMS-AS) may use the information to select, e.g. optimal IM-MGW, MRF.

Figure 6.4.1-1 illustrates how IMS obtains the UPF service area at multimedia session establishment.

Figure 6.4.1-1: IMS obtains the UPF service area

1.	P-CSCF receives an initial session request.

2.	Based on local policy, P-CSCF requests the current UPF service area (possibly with NPLI) via Npcf_PolicyAuthorization_Create.

3.	Either UPF service area is available at PCF (e.g. SMF included the information based on local policy at Npcf_SMPolicyControl_Create) or PCF requests the information from SMF (Npcf_SMPolicyControl_UpdateNotify).

4.	PCF returns the UPF service area (possibly with NPLI, if requested previously).

5.	Based on current UPF service area, P-CSCF selects an optimal MGW/ATGW, using local configuration.

6.	Before forwarding the SIP request, P-CSCF includes the UPF service area (e.g. in SIP PANI header) so that IMS nodes (e.g. IMS-AS) may use it to assist a media plane function (e.g. MRF).

### 6.4.2	Impacts on existing nodes and functions

#### 6.4.2.1	SMF

When the SMF makes use of the Npcf_SMPolicyControl service, it may send (based on local policy) the UPF service area (serving scope) as received from NRF during UPF NF discovery.

#### 6.4.2.2	PCF

When the PCF receives the Npcf_SMPolicyAuthorization service operation, it may request SMF (based on local policy) to be notified about UPF service area change.

#### 6.4.2.3	P-CSCF

When the P-CSCF receives the initial SIP request for a multimedia session, it makes use of the Npcf_PolicyAuthorization_Create (based on local policy) to retrieve the UPF service area. The P-CSCF uses the information to select the optimal MGW and include the information in the SIP request before forwarding the request to S-CSCF.

#### 6.4.2.4	IMS-AS

When the IMS-AS selects an MRF, it may make use of the UPF service area received in the SIP request.

#### 6.4.2.5	MGCF

When the MGCF selects an IM-MGW, it may make use of the UPF service area received in the SIP request.

#### 6.4.2.6	BGCF

When the BGCF selects an MGCF, it may make use of the UPF service area received in the SIP request.

## 6.5	Solution #5: Discovery of NFs from IMS nodes

### 6.5.1	Description

This is a solution to key issue #2.

The NRF can be provisioned (e.g. via O&M) with new types of NF profiles (e.g. NF types ATGW, IM-MGW, MRF) so that SBI capable P-CSCFs, IMS-ASs, etc. can discover MGWs, MRFs, etc. Such NF profile(s) may contain information about codecs supported, transcoding support, service area (serving scope), etc. without the need of configuring, e.g. ATGWs per UPF service area and the related information on each and every P-CSCF, IMS-AS, MGCF, etc. instance in the network.

Figure 6.5.1-1 illustrates how IMS nodes discover ATGW, IM-MGW, MRF.

Figure 6.5.1-1: IMS nodes discover ATGW, IM-MGW, MRF

1.	IMS node receiving the UPF information (e.g. P-CSCF, IMS-AS) discovers the media plane IMS NF (e.g. ATGW, MRF) with the required parameters (e.g. UPF service area, codecs required to be supported)

2.	NRF returns the matching NF profiles.

### 6.5.2	Impacts on existing nodes and functions

#### 6.5.2.1	IMS-AS

The IMS-AS discovers and selects an MRF, making use of the UPF service area received in the SIP request.

#### 6.5.2.2	MGCF

The MGCF discovers and selects an IM-MGW, making use of the UPF service area received in the SIP request.

#### 6.5.2.3	NRF

New type(s) of NF(s) are defined (e.g. ATGW), with the associated information (e.g. codecs supported, UPF service area) so that they can be discovered by IMS nodes.

## 6.x	Solution #X: <Solution Title>

### 6.x.1	Description

### 6.x.2	Impacts on existing nodes and functions

### 6.x.3	Solution Evaluation

# 7	Overall Evaluation

## 7.1	Evaluation of solutions addressing key issue #1

Solution #4: Conveying UPF service area to IMS nodes via PCF N5

The solution has the following advantages:

-	it does not impact multimedia control plane IMS functions (e.g. HSS, S-CSCF), but only IMS nodes managing media plane

-	it does not impact legacy diameter interfaces

-	it is independent of how many UPF instances are added in the network since it uses the UPF service area (serving scope), regardless of how many UPF instances serve the same area. This result in a much simpler configuration and maintenance in IMS nodes.

-	the media plane selection is based on the current UPF when the multimedia session is established, not in the UPF selected at PDU session establishment.

-	it reuses existing SBI service for IMS AF/P-CSCF produced by PCF in a natural way to retrieve, in addition to Network Provided Location information, the UPF service area.

Editor's Note:	Evaluation of Solution #1 and comparison is FFS.

# 8	Conclusions

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-09 | CT4#99e | C4-204476 | - | - | - | Draft Skeleton | 0.0.0 |
| 2020-11 | CT4#101e | C4-205688 C4-205502 C4-205503 |  |  |  | Agreed pCRs in CT WG4#101e | 0.1.0 |
| 2021-02 | CT4#101bis-e | C4-210125 |  |  |  | Agreed pCR in CT WG4#101bis-e | 0.2.0 |
| 2021-03 | CT4#102e | C4-211171 C4-211339 C4-211340 C4-211405 |  |  |  | Agreed pCRs in CT WG4#102e | 0.3.0 |
| 2021-04 | CT4#103e | C4-212552 C4-212553 |  |  |  | Agreed pCRs in CT WG4#103e | 0.4.0 |