| 3GPP TR 23.700-22 V1.0.0 (2024-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on CAPIF Phase 3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

Introduction	10

1	Scope	11

2	References	11

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	12

3.3	Abbreviations	12

4	 Gap analysis and requirements	12

4.1	CAPIF impacts due to charging considerations	12

4.2	Requirements for RNAA	12

4.3	Requirements for CAPIF interconnection	12

5	Key issues	13

5.1	Key issue #1: Managing resource owner consent	13

5.1.1	Description	13

5.1.2	Open issues	13

5.2	Key issue #2: Supporting Single Sign-On	13

5.2.1	Description	13

5.2.2	Open issues	14

5.3	Key issue #3: RNAA architecture enhancements	14

5.3.1	Description	14

5.3.2	Open issues	14

5.4	Key issue #4: CAPIF interconnection	14

5.4.1	Description	14

5.4.2	Open issues	15

5.5	Key issue #5: Enhancing support to API Invoker on-boarding	15

5.5.1	Description	15

5.5.2	Open issues	16

5.6	Key issue #6: UE-deployed API invoker accessing resources not owned by that UE	16

5.6.1	Description	16

5.6.2	Open issues	16

5.7	Key issue #7: CAPIF enhancement for AEF status and service API status	17

5.7.1	Description	17

5.7.2	Open issues	17

6	Solutions	17

6.1	Mapping of solutions to key issues	17

6.2	Solution #1: Backend For Frontend	18

6.2.1	Solution description	18

6.2.2	Architecture Impacts	18

6.2.3	Corresponding APIs	21

6.2.4	Solution evaluation	21

6.3	Solution #2: User consent for nested API invocation	22

6.3.1	Solution description	22

6.3.1.1	General	22

6.3.1.2	Enhancement to clause 8.32.3 of 3GPP TS 23.222	22

6.3.2	Architecture Impacts	23

6.3.3	Corresponding APIs	23

6.3.4	Solution evaluation	23

6.4	Solution #3: Finer granularity of access control for service API	24

6.4.1	Solution description	24

6.4.1.0	General	24

6.4.1.1	Impact to existing CAPIF procedures	24

8.3.2.1	Service API publish request	25

8.7.2.1	Service API discover request	25

8.11.3	Procedure	26

6.4.2	Architecture Impacts	27

6.4.3	Corresponding APIs	27

6.4.4	Solution evaluation	27

6.5	Solution #4: CAPIF interconnection	27

6.5.1	Solution description	27

6.5.1.1	General	27

6.5.1.3	Procedures to support CAPIF interconnection	27

6.5.1.3.1	API invoker obtaining authorization for service API access in CAPIF interconnection	27

6.5.1.3.2	Procedure for CAPIF revoking API invoker authorization in CAPIF interconnection	28

6.5.1.3.3	Procedure for obtaining access control policy in CAPIF interconnection	29

6.5.1.3.4	Procedure for obtaining security information in CAPIF interconnection	29

6.5.2	Architecture Impacts	30

6.5.3	Corresponding APIs	30

6.5.4	Solution evaluation	30

6.6	Solution #5: Enhancing API Invoker onboarding	30

6.6.1	Solution description	30

6.6.2	Architecture Impacts	32

6.6.3	Corresponding APIs	32

6.6.4	Solution evaluation	32

6.7	Solution #6: API instantiation based on service discovery	32

6.7.1	Solution description	32

6.7.2	Architecture Impacts	33

6.7.3	Corresponding APIs	33

6.7.4	Solution evaluation	33

6.8	Solution #7: API based activation for service API discover	33

6.8.1	Solution description	33

6.8.1.1	General	33

6.8.1.2	Procedure	34

8.3.2.1	Service API publish request	34

8.7.3	Procedure	34

6.8.2	Architecture Impacts	36

6.8.3	Corresponding APIs	36

6.8.4	Solution evaluation	36

6.9	Solution #8: AEF based instantiation for service API discover	36

6.9.1	Solution description	36

6.9.1.1	General	36

6.9.1.2	Procedure	36

8.7.3	Procedure	36

6.9.2	Architecture Impacts	37

6.9.3	Corresponding APIs	37

6.9.4	Solution evaluation	38

6.10	Solution #9: Support more API invoker info in RNAA	38

6.10.1	Solution description	38

6.10.1.1	General	38

6.10.1.3	Procedures	38

6.10.1.3.1	Authorization	38

6.10.2	Architecture Impacts	38

6.10.3	Corresponding APIs	39

6.10.4	Solution evaluation	39

6.11	Solution #10: Access Control Management of Service API	39

6.11.1	Solution description	39

6.11.1.1	General	39

6.11.2	Architecture Impacts	39

6.11.3	Corresponding APIs	39

6.11.4	Solution evaluation	39

6.12	Solution #11: Resource Owner Consent Revocation	39

6.12.1	Solution description	39

6.12.1.1	General	39

6.12.1.2	 Procedure for resource owner consent revocation	40

6.12.2	Architecture Impacts	40

6.12.3	Corresponding APIs	41

6.12.4	Solution evaluation	41

6.13	Solution #12: Capturing resource owner consent	41

6.13.1	Solution description	41

6.13.1.1	General	41

6.13.1.2	Procedure for capturing resource owner consent	41

6.13.1.3	Enhancement to clauses in 3GPP TS 23.222	42

6.13.2	Architecture Impacts	42

6.13.3	Corresponding APIs	42

6.13.4	Solution evaluation	42

6.15	Solution #14: Enhancement to API invoker authorization with the Purpose of Data Processing	44

6.15.1	Solution description	44

6.15.1.1	General	44

6.15.1.2	Impact to existing CAPIF procedures	45

8.31.3	Procedure	45

6.15.2	Architecture Impacts	46

6.15.3	Corresponding APIs	46

6.15.4	Solution evaluation	46

6.16	Solution #15: Enhancing the description of authorization function capabilities	46

6.16.1	Solution description	46

6.3.7	Authorization function	46

6.16.2	Architecture Impacts	46

6.16.3	Corresponding APIs	46

6.16.4	Solution evaluation	46

6.17	Solution #16: Obtaining Resource Owner Consent	47

6.17.1	Solution description	47

6.17.1.1	General	47

6.17.1.2	 Procedure for obtaining resource owner consent	47

6.17.2	Architecture Impacts	48

6.17.3	Corresponding APIs	48

6.18	Solution #17: Enhanced CAPIF events for onboarded API invoker	48

6.18.1	Solution description	48

6.18.1.1	General	48

6.18.1.2	CAPIF events to support onboarded API invoker	48

8.8.6	List of CAPIF events	48

6.18.2	Architecture Impacts	48

6.18.3	Corresponding APIs	49

6.18.4	Solution evaluation	49

7	Deployment scenarios	49

7.1	General	49

7.2	Deployment model #1: Updates to RNAA deployments	49

7.5	RNAA deployments	49

8	Business Relationships	50

5.2	CAPIF business relationships for RNAA	50

9	Overall evaluation	51

9.1	Evaluation of key issue#1	51

9.2	Evaluation of key issue#2	51

9.3	Evaluation of key issue#3	52

9.4	Evaluation of key issue#4	52

9.5	Evaluation of key issue#5	52

9.6	Evaluation of key issue#7	52

10	Conclusions	53

10.1	General conclusions	53

10.2	Conclusions of key issue #1	53

10.3	Conclusions of key issue #2	54

10.4	Conclusions of key issue #3	54

10.5	Conclusions of key issue #4	54

10.6	Conclusions of key issue #5	54

10.7	Conclusions of key issue #7	54

Annex A: Change history	56

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

CAPIF as specified in 3GPP TS 23.222 [2] is a common API framework for controlled exposure of service APIs. CAPIF is now considered by different 3GPP groups (e.g. SA2, SA5) and external bodies like GSMA, ETSI, ORAN Alliance to enable exposure of APIs. In Rel.18, CAPIF was enhanced considering some RNAA requirements. Further CAPIF enhancements are envisioned to enable use of CAPIF as the API framework for APIs exposure considered by different groups and external bodies.

This TR provides a thorough study of potential enhancements to CAPIF under the scope provided in clause 1.

# 1	Scope

The present document studies the potential enhancements to CAPIF (as specified in 3GPP TS 23.222[2]) to support: a) authentication and authorization interactions between Resource Owner and Authorization Functionality, b) UE-deployed API invoker accessing resources not owned by that UE, c) more granular access control (e.g., at the level of service operations or resources), d) CAPIF with further enhancements to service API status and AEF status, e) CAPIF enhancements identified or originated from other WGs or SDOs/industry forums and f) more CAPIF interconnection services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	IETF draft-ietf-oauth-browser-based-apps-17: "OAuth 2.0 for Browser-Based Apps".

NOTE:	The IETF work in "draft-ietf-oauth-browser-based-apps-17 is in draft stage.

[6]	IETF 6749: "The OAuth 2.0 Authorization Framework".

[7]	3GPP TR 28.849: "Study on charging aspects of Common API Framework for Northbound APIs (CAPIF) phase 2".

[8]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

API invoker frontend: An API invoker incapable of maintaining the confidentiality of their credentials (e.g., clients executing on the device used by the resource owner, such as an installed native application or a web browser-based application), and incapable of secure invoker authentication via any other means.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BFF	Backend For Frontend

# 4		Gap analysis and requirements

## 4.1	CAPIF impacts due to charging considerations

The gaps and impacts on CAPIF architecture due to charging considerations are studied by SA5 in 3GPP TR 28.849 [7].

NOTE:	Any impact on CAPIF architecture due to the study conclusions in SA5 can be handled during normative work.

## 4.2	Requirements for RNAA

[AR-4.2-a] The CAPIF shall provide mechanisms to manage (provide, obtain, revoke) user consents for API invoker(s) to access the resources owned by resource owners.

[AR-4.2-b] The CAPIF shall provide mechanisms to enable API invoker on a UE to access resources owned by the resource owner(s) of another UE or group of UE(s).

NOTE:	Additional RNAA requirements will be coordinated with SA3.

## 4.3	Requirements for CAPIF interconnection

[AR-4.3-a] The CAPIF shall provide mechanisms for authentication and authorization for access to the service API(s) exposed by a 3rd party CAPIF provider.

NOTE:	Additional CAPIF interconnection security requirements will be coordinated with SA3.

# 5	Key issues

## 5.1	Key issue #1: Managing resource owner consent

### 5.1.1	Description

With the introduction of support for Subscriber-aware Northbound API access in 3GPP Rel-18 a number of requirements were added at stage 1, one of which was motivated by the desire for a UE to be able to control whether or not to provide information considered private to a 3rd party entity. Specifically, the requirement is that the 5G system shall be able to allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party. At stage 2 the CAPIF-8 reference point was introduced to CAPIF where the aspect of consent was highlighted through the statement that the resource owner communicates with the authorization function in the CAPIF core function to manage resource owner consent, with such communication being expected to be performed over CAPIF-8. However, the mechanism for managing such consent was not specified with the functionalities over CAPIF-8 being is FFS and out of scope of the Rel-18 of the specification. Authentication of resource owner is required over CAPIF-8.

Further, user consent may also be required during group communication to access the information related to the members of the group. For example, the VAL server hosting dynamic platooning group requires access to the location of the members e.g. for performing operations of a group, joining the group.

### 5.1.2	Open issues

The CAPIF does not address management of resource owner consent in the context of supporting RNAA. The open issues are:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	Whether (and how) “purpose of data processing” will be captured and where will it be stored.

3.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

4.	How to enable CAPIF resource owner’s consent in the context of group.

5.	How to authenticate the resource owner via CAPIF-8

NOTE:	Aspects pertaining to the definition of resource owner consent/authorization over CAPIF-8 are in the scope of SA3, noting that the R18 security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [3].

## 5.2	Key issue #2: Supporting Single Sign-On

### 5.2.1	Description

For the Authorization code flow in RNAA, CAPIF-8 reference point (between User Agent/ROF and Authorization Server/CCF) is used to support end-user interactions with the resource owner e.g., obtaining permission from resource owner (also known as consent) and user authentication. According to 3GPP TS 23.222 clause 6.4.16, the CAPIF-8 reference point between CCF and ROF is kept out of Release 18.

The RO is authenticating (via CAPIF-8) to CCF as part of the interaction between API invoker and authorization function (which part of CCF).

Single sign-on (SSO) is an authentication method that enables users to securely authenticate with multiple applications by using just one set of credentials. In CAPIF RNAA context, Single Sign-On (SSO) can enable an Resource Owner to use one authentication procedure to authenticate multiple API invokers to use one or more AEFs exposing resources related to the resource owner. Specifically, to implement Single Sign-On (SSO) in RNAA, a Resource Owner needs to determine in advance which API invokers can be authorized with the authentication information for Single Sign-On (SSO), and then they provide to API invokers this Single Sign-On (SSO) information. Therefore, how to manage and provide information on API invokers subject to Single sign-on (SSO) authentication for the Resource Owner should be considered.

### 5.2.2	Open issues

1.	Whether and how to enhance CAPIF architecture and procedures considering Single sign-on (SSO)?

NOTE:	The detailed security aspects related to address Single sign-on (SSO)should be provided by SA3.

## 5.3	Key issue #3: RNAA architecture enhancements

### 5.3.1	Description

Clause 6.2.3 of 3GPP TS 23.222 [2] specifies the high-level functional architecture for CAPIF supporting Resource owner-aware northbound API access (RNAA). The security procedures for CAPIF supporting RNAA are specified in 3GPP TS 33.122 [3]. As per the architecture and security procedures the role and responsibilities of Resource Owner Function to support RNAA are not fully specified in Release 18.

It is required to study the role and responsibilities of Resource Owner Function and its interactions with CAPIF entities considering the requirements specified in 3GPP TS 22.261 [4] as shown below:

The 5G system shall be able to: - provide a third-party with secure access to APIs (e.g. triggered by an application that is visible to the 5G system), by authenticating and authorizing both the third-party and the UE using the third-party’s service. - provide a UE with secure access to APIs (e.g. triggered by an application that is not visible to the 5G system), by authenticating and authorizing the UE. - allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party. - preserve the confidentiality of the UE’s external identity (e.g. MSISDN) against the third-party. - provide a third-party with information to identify networks and APIs on those networks.

### 5.3.2	Open issues

Open issues to study:

1.	Further enhancements to CAPIF architecture considering Resource Owner Function functionalities and its interactions with CAPIF entities (e.g., CAPIF-8 related interactions).

NOTE:	The security aspects for the architecture enhancements need to be coordinated with SA3.

## 5.4	Key issue #4: CAPIF interconnection

### 5.4.1	Description

Service federation between different service providers is important in application enabler to support service sharing. Two organizations with a business relationship that have each deployed CAPIF may need to interoperate to allow API invokers in each trust domain to utilize service APIs from both CAPIFs as illustrated in Figure 4.12.1-1 of 3GPP TS 23.222 [2].

Figure 5.4.1-1 is used as an example to show the architectural model for the CAPIF interconnection which allows API invokers of a CAPIF provider to utilize the service APIs from the 3rd party CAPIF provider.

Figure 5.4.1-1: High level functional architecture for CAPIF interconnection with multiple CAPIF provider domains (as described in clause 6.2.1 of 3GPP TS 23.222 [2])

The existing CAPIF procedures supporting interconnection are described in 3GPP TS 23.222 [2] clause 8.25 which includes service API publish/retrieval/update/unpublish and discovery over CAPIF-6/6e reference point.

However, in above architecture example, the API provider domain functions are registered in a CCF within the same trusted domain (domain-A). The CCFs are connected via CAPIF-6e, in order to share service APIs. The API provider domain functions (e.g. AEF) don’t see the interconnected CCF in another domain, but is able to provide AEF service APIs to API invoker onboarded in a CCF in another domain (domain-B) via CAPIF-2e.

In above figure, the AEF of domain-B authenticates and authorizes the API access (together with CCF in domain-B). The API invoker obtains authorization from CCF in domain-A. It is under-specified how authentication and authorization are done in CAPIF interconnection.

Also, it is worth intestigating what existing CAPIF event exposure services in clause 10.4.1 of 3GPP TS 23.222 [2] are applicable for CAPIF-6/6e and if any new event exposure service is needed to support CAPIF interconnection.

### 5.4.2	Open issues

Solutions to this key issue will address the following aspects in CAPIF interconnection:

-	How to authenticate and authorize service API access for the AEF service API(s) exposed via CAPIF-6/6e;

NOTE:	Coordination with SA3 is needed for security details.

-	Investigate applicable events and new events (if any) for CAPIF-6/6e.

## 5.5	Key issue #5: Enhancing support to API Invoker on-boarding

### 5.5.1	Description

Currently in 3GPP TS 23.222 clause 8.1 On-boarding of API Invoker to the CAPIF procedure, there is an assumption that API Invoker has sufficient API information to make decision of making an on-boarding request to CCF. However, in reality the API Provider exposes limited or only information that can be shared commonly to the API Invoker. The reason for this is beacause the trust relationship is yet to be established between the two parties (i.e. API Invoker and the API Provider).

In another scenario, after the API Invoker successfully on boards to the CAPIF, the API Invoker realizes that certain features or services that the API Invoker wishes to consume with assistance of CAPIF, are not supported. Then the API Invoker may off board from the CAPIF or take certain other actions like not consuming northbound APIs via the registered CCF. In such a scenario, it is waste of resources for the API invoker and the CCF to perform the on boarding and maintain the API Invoker profile information. The features that API Invoker may be looking to consume from CCF and not supported by CCF, may include, the AEFs serving certain set of service API(s), availability of set of service APIs, support for certain security methods, support for certain security methods for certain AEFs / service APIs, interconnection with a given set of CCFs etc.

To address these cases, it is essential to study enhancing support to API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources.

### 5.5.2	Open issues

This key issue will study the following aspects:

1.	How to enhance the support of API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources?

2.	Any enhancements required to other CAPIF procedures e.g. Registering the API provider domain functions, Publish Service APIs to CCF?

## 5.6	Key issue #6: UE-deployed API invoker accessing resources not owned by that UE

### 5.6.1	Description

An API invoker may be either an application on a server or an application on a UE. According to clause 7.5 in 3GPP TS 23.222 [2], the API invoker may be deployed in any of the following ways:

a.	API invoker may be deployed as AF on the UE (i.e. 3rd party application).

b.	API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE.

c.	API invoker may be deployed on the network as AF.

As from clause 4.17.1 in 3GPP TS 23.222 [2], (for Release 18) the scope of an API invoker on a UE (i.e., options a and b) in Resource owner-aware northbound API access (RNAA) is limited to accessing its own resources only, i.e., resource owner is a user of the UE hosting the API invoker that can authorize the API access. This is also acknowledged by SA3 in clause 6.5.3 in 3GPP TS 33.222 [3], in which “only a UE accessing its own resources is considered if the API invoker is on a UE.”

However, there are cases in which it is needed to support for API invoker(s) which are deployed on the UE accessing resources of other resource owners (users), e.g., consider the clause 6.2.2 in 5GAA - C-V2X Use Cases and Service Level Requirements Volume I, dealing with Vehicle Health Monitoring in fleet management, in which an Application Client on UE 1 could request access to fetch location and/or vehicle health issues to another user (UE 2).

### 5.6.2	Open issues

This key issue will study:

1.	Whether (and how) RNAA can support the scenario where API invoker(s) which are deployed on the UE can access resources (hosted in the network) of other resource owners (users) (e.g., application client on UE is fetching location of another UE or setting QoS for PDU sessions of another UE)

NOTE 1:	The security aspects need to be coordinated with SA3.

NOTE 2:	The unwanted interactions with the RO user in this case need to be minimized.

## 5.7	Key issue #7: CAPIF enhancement for AEF status and service API status

### 5.7.1	Description

For the previous CAPIF procedure specified in 3GPP TS 23.222 [2], the status of AEF instance is assumed to be instantiated. The service API can be published through service API publish procedure by APF, and dicovered by API invoker by using the service API discovery procedure. However, the AEF instance may have multiple possible status, in addition to the instatiated status, which may impact the current CAPIF procedures. Futhermore, there are possible richer service API status. Therefore, further study the CAPIF enhancement for AEF status and service API status is required.

### 5.7.2	Open issues

This key issue includes the following aspects:

-	Whether and what kinds of AEF availability status and service API status should be considered for service discovery.

-	Whether and how to enhance the current CAPIF mechnisms considering AFF status and service API status.

NOTE: Coordination and alignment with SA5 are required for AEF status and service API status related to instantiation.

# 6	Solutions

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |
| Sol #15 | X |  |  |  |  |  |  |
| Sol #16 | X |  |  |  |  |  |  |
| Sol #17 |  |  |  |  | X |  |  |

## 6.2	Solution #1: Backend For Frontend

### 6.2.1	Solution description

This solution relates to KI#3 on RNAA architecture enhancements. Specifically, it proposes the Backend For Frontend (BFF) pattern (as described, for example, in clause 6.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5]) to be included as an architectural option.

In 3GPP TS 23.222 [2] clause 7.5, the RNAA deployment option in which the API invoker is deployed on a UE is presented. A UE can be considered as hosting public clients according to the OAuth 2.0 Authorization Framework [6]. Such public clients lack the ability to maintain the confidentiality of secrets such as their client credentials or even access tokens provided to them. Regarding this vulnerability, as an example, clause 5.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5] highlights the danger of malicious JavaScript in relation to browser-based applications (highlighted as an example of a user-agent-based application public client in the OAuth 2.0 Authorization Framework [6]). This IETF draft presents the BFF application architecture pattern as an approach in which applications are built to rely on a backend component for handling OAuth responsibilities and then all requests are proxied through this backend component. Such an architectural approach is applicable to CAPIF as described in the next sub-clause.

NOTE:	The security aspects of the CAPIF architectural variant described in this solution are out of scope of the present document.

### 6.2.2	Architecture Impacts

In relation to the OAuth 2.0 Authorization Framework [6] the CAPIF resource owner function is responsible for interactions with the resource owner, which is anticipated to be in a similar manner to the way in which interactions between the resource owner’s user agent are described by the OAuth framework.

With OAuth there is also a client (or client application) that interacts with the user-agent and authorization server in order to gain access to information exposed by a resource server.

The OAuth resource server is considered as the CAPIF API exposing function, aligned with clause 6.5.3.1 of 3GPP TS 33.122 [3].

The OAuth client is considered as the CAPIF API invoker (aligned with clause 6.5.3.1 of 3GPP TS 33.122 [3]), where that API invoker can be UE hosted as previously highlighted.

Then in this solution (as presented in Figure 6.2.2-1) it is proposed to support an option in which the API invoker hosted by a UE (API invoker frontend) interacts via a new reference point (CAPIF-X) with a “helper” API invoker (API invoker backend) that is placed server-side (network-side) and is considered alongside the Backend For Frontend component in the Application Architecture presented in clause 6.1.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5].

According to that IETF draft, the BFF:

1.	Interacts with the authorization server as a confidential client

2.	Manages access and refresh tokens and does not share those with the client, i.e., the tokens are inaccessible to the client

3.	Proxies all requests from the client to the resource server, augmenting them with the correct access token before forwarding them to the resource server

In the IETF draft, an OAuth user-agent-based application is considered in which the client application runs within the user-agent. On the other hand, for an OAuth native application, the user-agent and client application are presented as separate entities, which is also the case the architectural solution proposal in Figure 6.2.2-1.

In the proposed architecture for CAPIF supporting BFF, the API invoker is split between a frontend and backend component. The backend acts as a proxy on behalf of the frontend. The backend is a server-side component. Through the API invoker backend, the API invoker frontend can leverage the capabilities of CAPIF including service discovery and authorization prior to accessing the service API over CAPIF-1(e) and service API invocation over CAPIF-2(e). However, it is the API invoker backend that onboards to the CAPIF core function (CCF) and mutually authenticates with it.

Figure 6.2.2-1: High level functional architecture for CAPIF supporting BFF

NOTE:	Impacts to the reference points with their assocaited procedures between the API provider domain functions and the CAPIF core function are not anticipated (and hence have not been depicted in Figure 6.2.2-1).

Figure 6.2.2-2: BFF based procedure for service API access

Figure 6.2.2-2 illustrates the BFF based procedure.

Pre-condition:

The API invoker frontend and API invoker backend have an established secure session.

Firstly, in step 1, the UE hosted API invoker (API invoker frontend) initiates the authorization flow, where the API invoker’s objective is to gain access to particular resources offered through the service APIs exposed by the API provider domain.

In step 2, the API invoker backend, initiates the authorization flow towards the CCF authorization function (CCF(AuthF)). The request is redirected via the UE hosted User agent,.

In step 3, mutual authentication is performed between the CCF(AuthF) and the Resource owner via the Resource owner function and any required user consent obtained.

Once mutual authentication has been successfully perform, in step 4 the CCF(AuthF) provides indication of that granted authorization to the API invoker backend using redirection via the User agent.

Next in step 5, the API invoker backend requests access to the API exposing function’s service APIs from the CCF(AuthF) using the authorization obtained in step 4.

The API invoker backend maintains an association between its session with API invoker frontend and access information it obtains in step 6 from the CCF(AuthF).

The API invoker backend then provides authorization granted notification to the API invoker frontend that service API access is permitted via the User agent in step 7. The access security token itself is not passed to the API invoker frontend, but is stored by the User agent.

When the API invoker frontend subsequently initiates a service API request (to the AEF via API invoker backend), it does so via the User agent. The User agent then adds the access security token (that it obtained from the CCF(AuthF) in step 8) to the request.

The API invoker backend then uses its granted service API access that it received from the authorization function (that is associated with the API invoker frontend’s resource owner function to API invoker backend session) to make the request towards the service API (step 9).

The response from the service API (step 10) is then forwarded towards the API invoker frontend by the API invoker backend (step 11).

### 6.2.3	Corresponding APIs

The following API calls are described in the context of those described in the BFF IETF RFC draft, draft-ietf-oauth-browser-based-apps-17 [5].

CAPIF-X (API invoker frontend – API invoker backend)

-	Initiate authorization request, from API invoker frontend to API invoker backend (step 1, clause 6.2.2)

NOTE: API invoker backend issues the “authorization request” towards the CCF(AuthF) triggered by the “initiate authorization request” where the mapping between these two messages and their content is out of scope of the present document.

-	Access granted indication, from API invoker backend to API invoker frontend via User agent (step 7, clause 6.2.2)

The two steps above relate to the RNAA related authorization procedures, i.e., step 1 in both the API invoker obtaining authorization from resource owner (clause 8.31 of 3GPP TS 23.222 [2]) and reducing authorization information inquiry in a nested API invocation (clause 8.32 of 3GPP TS 23.222 [2]) procedures, where such security aspects are out of scope of the present document.

-	API service request, from API invoker frontend to API invoker backend (step 8, clause 6.2.2)

-	API service response, from API invoker backend to API invoker frontend (step 11, clause 6.2.2)

The two steps above relate to service API invocation with AEF authorization as described in clause 8.16 of 3GPP TS 23.222 [2]).

-	In addition to the steps highlighted above that are required in support of the BBF pattern, it is anticipated that CAPIF-X would be utilised for API service discovery (clause 8.7 & 8.25.3.2 of 3GPP TS 23.222 [2]), subscription related procedures (clause 8.8 & 8.9 of 3GPP TS 23.222 [2]), revocation of API authorization (clause 8.23 of 3GPP TS 23.222 [2]), update of API invoker’s API list (clause 8.26 of 3GPP TS 23.222 [2]). In each instance, the API invoker frontend would initiate the procedure towards the API invoker backend and then the API invoker backend would utilise either the CCF (CAPIF-1) or AEF (CAPIF-2) procedures as appropriate. Alternatively, the API invoker backend would initiate the procedure towards the API invoker frontend when triggered by the CCF (e.g., the backend forwarding the revoke API invoker authorization notify to the frontend in the case of CAPIF revoking API invoker authorization (clause 8.23 of 3GPP TS 23.222 [2]).

CAPIF-8 (ROF – CCF(AuthF))

-	Granting authorization and user consent, from CCF(AuthF) to ROF (step 3, clause 6.2.2)

The specification of CAPIF-8 is in scope of this specification, however it is not the focus of this solution and is therefore not further detailed here.

CAPIF-1 (API invoker backend – CCF)

-	Authorization request, from API invoker backend to CCF(AuthF) redirected via User agent (step 2, clause 6.2.2)

-	Authorization response from CCF(AuthF) to API invoker backend, redirected via User agent (step 4, clause 6.2.2)

-	API invoker backend makes authorization access request to CCF(AuthF) (step 5, clause 6.2.2)

-	CCF(AuthF) provides authorization access grant to API invoker backend (step 6, clause 6.2.2)

### 6.2.4	Solution evaluation

This solution addresses KI#3 on RNAA architecture enhancements, specifically proposing the BFF patten as decribed for instance in IETF RFC draft, draft-ietf-oauth-browser-based-apps-17 [5]. That IEFT RFC draft focuses on browser-based applications, i.e., application clients run from within the browser, typically JavaScript based. According to the OAuth 2.0 framework [6], such clients are considered public and are incapable of maintaining the confidentiality of their credentials where is recognized that such clients are susceptible to various attack vectors, such as cross-site scripting (XSS).

With the BFF pattern, a server-side backend component is introduced to handle OAuth responsibilities and act as a proxy for all (application) client requests. The backend component is expected to behave as a confidential client able to conceal sensitive information from the frontend and therefore potential attackers.

Solution #1 presents the BFF pattern in the context of the CAPIF, whereby the API invoker is split between a backend and frontend component. The API invoker backend component utilizes the existing capabilities offered through CAPIF-1 (e.g., authentication, authorization and service API discovery) and CAPIF-2 (e.g., authorization verification and service API invocation), where the difference is that the associated procedures are initiated by the API invoker frontend through the proposed CAPIF-X reference point (e.g., authorization, service API discovery and service API invocation) and API invoker backend relays those.

NOTE 1: The specific relaying functions provided by the API invoker backend will be further specified in the normative phase, for example mapping of identity information and hiding of confidential information from API invoker frontend.

CAPIF-8 also has a role in the overall solution (i.e., the granting of authorization and user consent), but it is highlighted that those roles are not specific to this solution and are therefore the specification of that reference point is left to other solutions.

The impact on API invoker backend onboarding on CAPIF in this scenario is to be further discussed during normative work.

NOTE 2: As noted in clause 6.2.1, security aspects of this solution are not in scope of this specification. However, prior to inclusion of this solution in the normative phase, coordination with SA3 is required on the security considerations and associated procedures for supporting the BFF pattern.

## 6.3	Solution #2: User consent for nested API invocation

### 6.3.1	Solution description

#### 6.3.1.1	General

This solution maps to KI#1 on managing resource owner consent, specifically on how consent of the resource owner can be managed through communication between the resource owner (through the resource owner function) and authorization function in the CAPIF core function. This solution proposes to extend the existing text in clause 8.32 of 3GPP TS 23.222 [2] on reducing authorization information inquiry in a nested API invocation, specifically the procedure described in clause 8.32.3.

The existing procedure focuses on ensuring the (primary) API invoker and secondary API invoker (API exposure function 1 (AEF-1)) have the necessary authorization to invoke the service APIs of their respective API exposure functions, AEF-1 & AEF-2 respectively. Such authorization may include a specific scope to provide course-grained access to resources exposed by the service API, for example limiting access to only certain attributes.

In this solution it is proposed that the secondary API invoker (AEF-1) performs an additional check to ensure suitable user consent has been provided before making service API invocation requests towards a secondary API exposing function (AEF-2). This enables the AEF-1 to check before inadvertently exposing user sensitive information provided in the request from the (primary) API invoker for which appropriate user consent has not been obtained.

The solution is described in relation to nested API invocation, but it is considered to have applicability for the general case of a single API invoker performing service invocation towards an API exposure function.

#### 6.3.1.2	Enhancement to clause 8.32.3 of 3GPP TS 23.222

With this solution the procedure to reducing authorization information inquiry in a nested API invocation in clause 8.32.3 is enhanced with the following:

New pre-condition added:

Optionally, AEF-1 is configured with policy for user consent checking before making nested API service invocation requests.

New descriptive text is added after the existing text of Step 3 (which is “Based on the service API invocation request, API exposing function 1 decides to invoke another service API exposed by API exposing function 2”):

Before making the request, if the AEF is configured with policy regarding user consent checking, the AEF will check that the service API invocation request contents is not in breach of those requirements.

An example for the user consent check could involve checking associated attributes in the request itself (e.g., a signed token relating to user consent). Another example may require the AEF to further interact with the resource owner function to gain the necessary permissions to expose the attributes provided in the request from the API invoker (possibly via the CCF / authorization function), which could require enhancement to the procedures related to step 4 (which is “API exposing function 1, acting as an API invoker, obtains from the CCF the authorization information to access the service API exposed by API exposing function 2”).

If the AEF doesn’t obtain the necessary user consent permissions, the (primary) API invoker’s service invocation request will be rejected with an appropriate failure result.

The additions are highlighted (in bold) in Figure 6.3.1.2-1.

Figure 6.3.1.2-1: Enhancements to Figure 8.32.3-1 of 3GPP TS 23.222.

NOTE:	The mechanism to ascertain appropriate user consent at the AEF is in the scope of SA3.

### 6.3.2	Architecture Impacts

None.

### 6.3.3	Corresponding APIs

NOTE:	Whether new or enhanced APIs are required in support of this solution is in scope of SA3.

### 6.3.4	Solution evaluation

This solution addresses KI#1 on managing resource owner consent by proposing that the AEF supports the ability use a configured policy to enforce user consent checking during service API invocation. Then when such policy is applied to the AEF, it will check that received service API invocation requests are not in breach of the requirements described by the policy. API invoker requests will be rejected if the necessary user consent (according to the user consent checking policy) cannot be ascertained. The need for ROF to be involved for checking user consent during service API invocation requires further investigation.

NOTE:	The feasibility of providing a security mechanism to enforce user consent checking at an AEF, which is also aligned with CAPIF RNAA, needs coordination with SA3 prior to normative work.

## 6.4	Solution #3: Finer granularity of access control for service API

### 6.4.1	Solution description

#### 6.4.1.0	General

This solution addresses the KI#1 and enables the finer granularity of the access control for service API, i.e.:

1.	Service operation level access, the API invoker may request and the CAPIF may grant the access for the given service operation(s) of the service API.

2.	Resource level access, the API invoker may request and the CAPIF may grant the access for the given resource(s) (i.e., the object or component of the API on which the operations are acted upon as defined in clause 3.1 of 3GPP TS 23.222 [2]) of the service API.

NOTE:	The feature level access can be discussed during the normative phase.

#### 6.4.1.1	Impact to existing CAPIF procedures

The CAPIF procedure in 3GPP TS 23.222 [2] can be enhanced (highlighted in bold italics) as follows.

#### 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M; The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), Service API context (optional), and Network Slice Info (optional).. Shareable information; O (see NOTE); Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained.. NOTE: If the shareable information is not present, the service API is not allowed to be shared.. The Service KPIs is defined as below: Table 8.3.2.1-2: Service KPIs Information element; Status; Description. Maximum Request rate; O; Maximum request rate from the API Invoker supported by the server.. Maximum Response time; O; The maximum response time advertised for the API Invoker's service requests.. Availability; O; Advertised percentage of time the server is available for the API Invoker's use.. Available Compute; O; The maximum compute resource available for the API Invoker.. Available Graphical Compute; O; The maximum graphical compute resource available for the API Invoker.. Available Memory; O; The maximum memory resource available for the API Invoker.. Available Storage; O; The maximum storage resource available for the API Invoker.. Connection Bandwidth; O; The connection bandwidth in Kbit/s advertised for the API Invoker's use.. The Service API context is defined as below: Table 8.3.2.1-3: Service API context Information element; Status; Description. Service operations; O; The list of the supported service operations.. Resources; O; The list of supported resources, i.e, the object or component of the API on which the operations are acted upon.. #### 8.7.2.1 Service API discover request Table 8.7.2.1-1 describes the information flow service API discover request from the API invoker to the CAPIF core function. Table 8.7.2.1-1: Service API discover request Information element; Status; Description. API invoker identity information; M; Identity information of the API invoker discovering service APIs. Query information; M; Criteria for discovering matching service APIs (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), required API provider name (optional), UE IP address (optional), interfaces, protocols, Service KPIs (optional), Service API context (optional), and Network Slice Info (optional)). (see NOTE). NOTE: It should be possible to discover all the service APIs.. ### 8.11.3 Procedure Figure 8.11.3-1 illustrates the procedure for obtaining authorization to access the service API. Pre-condition: 1. The API invoker is onboarded and has received an API invoker identity. Figure 8.11.3-1: Procedure for the API invoker obtaining authorization for service API access 1. The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API with optional Service API context by including the API invoker identity information and any information required for authentication of the API invoker. 2. The CAPIF core function validates the authentication of the API invoker (using authentication information) and checks whether the API invoker is permitted to access the requested service API with optional Service API context. NOTE 1: The authentication process is specified in subclause 6.5.2.3 of 3GPP TS 33.122 [12]. 3. Based on the API invoker's subscription information the authorization information to access the service APIs with optional Service API context is sent to the API invoker in the obtain service API authorization response. NOTE 2: The mechanism for distribution of the authorization information for the API invoker to the API exposing function is specified in subclause 6.5.2.3 of 3GPP TS 33.122 [12].

### 6.4.2	Architecture Impacts

This solution is based on architecture of CAPIF as described in 3GPP TS 23.222 [2].

### 6.4.3	Corresponding APIs

This solution impacts the CAPIF_Security API defined in clause 10.6 of 3GPP TS 23.222 [2].

### 6.4.4	Solution evaluation

The coordination with SA3 is needed to align the information flows and procedures in 3GPP TS 33.122 [3].

This solution addresses KI#1 and enables the finer granularity of the access control for service API.

## 6.5	Solution #4: CAPIF interconnection

### 6.5.1	Solution description

#### 6.5.1.1	General

This solution enables authorization for service API access in CAPIF interconnection for CCFs in different CAPIF provider domains. The proposed change is also applicable for CAPIF interconnection within a CAPIF provider domain where different CCFs of the same CAPIF provider are involved.

In addition, since the API invoker is onboarded in a different CCF when the AEF is trying to obtain information from its registered CCF, the inter-CCF interaction is needed to support authentication/authorization/policy information transfer.

#### 6.5.1.3	Procedures to support CAPIF interconnection

##### 6.5.1.3.1	API invoker obtaining authorization for service API access in CAPIF interconnection

Pre-condition:

1.	The API invoker has discovered service APIs provided by an AEF via procedure defined in step 1 and 2 of clause 8.25.3.2 of 3GPP TS 23.222 [2].

2.	The API invoker and the CCF-B are in the same trusted domain.

3.	The AEF and the CCF-A are in the same trusted domain.

4.	The CCF-A and the CCF-B are connected to each other, and they have business agreement for service API authorization.

5.	The CCF-A is the authorization function for service API access on the AEF.

Figure 6.5.1.3.1-1: Procedure for the API invoker obtaining authorization for service API access

1.	The API invoker sends an obtain service API authorization request to the CCF-B for obtaining permission to access the service API by including the API invoker identity information and any information required for authentication of the API invoker.

2.	After successful authentication validation of the API invoker, if the CCF-B determines that the service API access authorization cannot be done by itself alone, then the CCF-B sends an obtain API authorization from the CCF-A. In the request, the CCF-B can also send information about the API invoker so that the CCF-A can execute the authorization.

3.	Based on the API invoker's subscription information, the authorization information to access the service APIs is sent to the API invoker in the obtain service API authorization response.

##### 6.5.1.3.2	Procedure for CAPIF revoking API invoker authorization in CAPIF interconnection

Pre-conditions:

1.	The CCF-A is triggered to revoke API invoker authorization for service API access.

Figure 6.5.1.3.2-1: Procedure for revoking API invoker authorization in CAPIF interconnection

1.	If the CCF-B was delegated with service API authorization, the CCF-A sends revoke API invoker authorization request to the CCF-B with the details of the API invoker, the AEF and the service API.

2.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CCF-B invalidates the API invoker authorization corresponding to the service API.

3.	The CCF-B sends a revoke API invoker authorization response to the CCF-A.

4.	Instead of step 1 to 3, if the CCF-A performed service API authorization, the CCF-A sends a revoke API invoker authorization notify to the CCF-B.

5.	The CCF-A invalidates the API invoker authorization corresponding to the service API.

6.	The CCF-B sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

##### 6.5.1.3.3	Procedure for obtaining access control policy in CAPIF interconnection

Pre-condition:

1.	The AEF is hosting the service API but the policy to perform access control is not available with AEF.

2.	The CCF-B has available access control policies corresponding to one or more service APIs.

3.	The AEF and the CCF-A are in the same trusted domain.

Figure 6.5.1.3.3-1: Procedure for obtaining access control policy in CAPIF interconnection

1.	The AEF sends an obtain access control policy request to the CCF-A for obtaining the policy to perform the access control on service API invocations by including the details of the hosted service API.

2.	After successful authentication validation of the AEF, if the CCF-A determines that the service API access control policy authorization cannot be done by itself alone, then the CCF-A sends an obtain access control policy request to the CCF-B.

NOTE 1:	If the CCF-A has enough access control policy available, it executes the procedure defined in clause 8.12.3 of 3GPP TS 23.222 [2] without interaction with the CCF-B.

3-4.	The CCF-B determines appropriate access control policy for service APIs upon the request sent by the CCF-A. The access control policy information is sent to the AEF (via the CCF-A) in the obtain access control policy response.

NOTE 2:	To maintain synchronization between the AEF and the CCF for the policy cached at AEF, the AEF can subscribe to the policy update event at the CCF-A according to the procedure in clause 8.8.3 of 3GPP TS 23.222 [2] and receive notifications about any updated policy at the CCF-A according to the procedure in clause 8.8.4. Correspondingly, the CCF-A can subscribe to policy update event at the CCF-B if service API authorization is performed by the CCF-B.

##### 6.5.1.3.4	Procedure for obtaining security information in CAPIF interconnection

Pre-condition:

1.	The AEF has no security information available for authentication and/or authorization

2.	The CCF-B has security information available for authentication and/or authorization corresponding to one or more service APIs.

3.	The AEF and the CCF-A are in the same trusted domain.

Figure 6.5.1.3.4-1: Procedure for obtaining security information in CAPIF interconnection

1.	The AEF obtains the API invoker information required for authentication and/or authorization by the AEF from the CCF-A by including the details of the API invoker and hosted service API. After successful authentication validation of AEF, if the CCF-A determines that it has no security information, then the CCF-A obtains security information request from the CCF-B. Then the security information required for service API invocations is sent to the AEF.

NOTE 1:	If the CCF-A has security information available, it executes the procedure defined in clause 8.14.3, clause 8.15.3 or clause 8.16.3 of 3GPP TS 23.222 [2] without interaction with the CCF-B.

NOTE 2:	The CCF-A can subscribe to API invoker onboarding event at the CCF-B according to the procedure in clause 8.8.3 of 3GPP TS 23.222 [2] and receive notifications about any onboarded API invoker and its security related onboarding information at the CCF-B according to the procedure in clause 8.8.4 of 3GPP TS 23.222 [2].

### 6.5.2	Architecture Impacts

Existing CAPIF architecture in 3GPP TS 23.222 [2] is used to accommodate new interactions between CCFs in this solution.

### 6.5.3	Corresponding APIs

This solution impacts the CAPIF-6/6e interface with new or updated APIs. Security related API(s) will be addressed by SA3.

### 6.5.4	Solution evaluation

This solution addresses KI#4 regarding CAPIF interconnection including authorization, authentication and access control aspects.

NOTE:	Security details will be addressed by SA3.

## 6.6	Solution #5: Enhancing API Invoker onboarding

### 6.6.1	Solution description

This solution is related to Key issue #5: Enhancing support to API Invoker onboarding, proposing to enhance the API invoker onboarding procedure specified in 3GPP TS 23.222. The enhanced procedure enables the CCF to onboard an API Invoker to CAPIF, based on meeting the criteria information from the API invoker. The API invoker may include the criteria information in the API Invoker onboarding request message, to inform that it wishes to onboard to CCF only if the criteria information in the request is met by CCF.

Figure 6.6.1-1 illustrates the procedure for enhanced onboarding of the API Invoker to the CAPIF.

Figure 6.6.1-1: Enhanced API Invoker onboarding procedure

1.	The API Invoker sends the onboard API invoker request message to the CCF, to onboard itself to the CAPIF, as specified in 3GPP TS 23.222, along with additional onboarding criteria information. The onboarding criteria information may include the information about AEFs serving certain set of service API(s), availability of AEFs serving a set of service APIs, support of security methods for certain AEFs / AEF types/ service APIs/ Service API categories, interconnection with a given set of CCFs etc.

Table 6.6.3-1: On-board API Invoker request

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding information | M | The information of the API invoker including enrolment details, required for on boarding |
| APIs for enrolment | O | List of APIs being enrolled for. |
| Proposed expiration time | O | Proposed expiration time for the onboarding. |
| Onboarding criteria | O | API Invoker indicates to the CCF that the API Invoker wishes to on board itself to the CCF only if the CCF supports the features in Criteria information (For example, AEFs serving certain set of service API(s), availability of AEFs serving a set of service APIs, support of certain security methods, support of certain security methods for certain AEFs / AEF types/ service APIs/ Service API categories, interconnection with a given set of CCFs and like so). |

2.	On receipt of onboard API invoker request message, the CCF begins the onboarding process by verifying whether all the necessary information has been provided to onboard the API invoker. If the onboard API invoker request message includes criteria information, then CCF verifies whether the CCF can support the criteria information. The CCF further proceeds with API invoker onboarding procedure, only if the CCF supports the criteria information from the API Invoker.

3.	If the API invoker is granted permission to be onboarded, the CCF responds with the onboard API invoker with success message, including information about the provisioned API invoker profile/API Invoker enrolment details, which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs.

Figure 6.6.1-2 illustrates the new procedure for notifying the API invoker from CCF, once the onboarding criteria information is no longer supported by the CCF.

Pre-condition:  CCF decides to notify the API invoker due to certain conditions like, onboarding criteria information not met.

Figure 6.6.1-2: notification procedure

1.	 CCF sends onboarding notification request to API Invoker, including the notification reason (onboarding condition not met)..

2.	The API invoker acknowledges the onboarding notification request message.

3.	The API invoker, based on the notification reason from CCF in step 1, may renew the API invoker’s enrolment to CCF using the procedures in clause 8.1 of 3GPP TS 23.222 with new criteria information or cancel the API invoker’s enrolment to CCF using the procedures in clause 8.2 of 3GPP TS 23.222.

### 6.6.2	Architecture Impacts

The existing CAPIF architecture as specified in 3GPP TS 23.222 [2], is used for the interactions between API Invoker and CCF defined in this solution.

### 6.6.3	Corresponding APIs

This solution impacts the CAPIF_API_Invoker_Management API as specified in clause 10.5 of 3GPP TS 23.222 [2]

### 6.6.4	Solution evaluation

This solution addresses the key issue #5 by enabling the CCF to onboard an API Invoker to CAPIF based on meeting the criteria information from the API Invoker. Initial on-boarding of API Invoker is avoided when the CCF does not meet the criteria information, if provided by API invoker. Potentially, this results in API invoker actively consuming the CAPIF services / northbound APIs. Also, the solution proposes CCF sending notification to the API invoker when CCF cannot meet the onboarding criteria request from the API invoker. Thus, the enhanced onboarding and notification procedures result in, avoid any unnecessary on-boarding, managing dormant API Invoker profile information at CCF and to enhance the experience of onboarding management.

## 6.7	Solution #6: API instantiation based on service discovery

### 6.7.1	Solution description

Figure 6.7.1-1 illustrates the procedure for Service API discovery procedure.

Figure 6.7.1-1: Procedure to trigger service API instantiation

1-2.	Same as step 1 and 2 in clause 8.7.3 of 3GPP TS 23.222 [2].

3.	If the CCF determines that the service API being discovered is not instantiated, then the CCF invokes AMF to perform service/AEF instantiation. The AMF is responsible for triggering service/AEF instantiation.

Editor's Note:		If service API level instantiation is not available by SA5 mechanisms, AMF would ask for the instantiation of the AEF providing such a service API. it is FFS whether SA5 can support service API level instantiation.

4.	Same as clause 8.3.3 of 3GPP TS 23.222 [2].

5.	Same as step 3 in clause 8.7.3 of 3GPP TS 23.222 [2].

### 6.7.2	Architecture Impacts

Existing CAPIF architecture in 3GPP TS 23.222 [2] is used to accommodate new interactions between CCF and AMF in this solution.

### 6.7.3	Corresponding APIs

This solution impacts the CAPIF-5/5e interface with a new AMF API for triggering service instantiation.

### 6.7.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 6.8	Solution #7: API based activation for service API discover

### 6.8.1	Solution description

#### 6.8.1.1	General

This paper proposes a solution for key issue #7 on CAPIF enhancement for service API status, to introduce the API based activation in service API discover procedure.

The API can be activated during the service API discover, if the required servcie API is inactive without API interface details, and the related AEF is already instantiated.

#### 6.8.1.2	Procedure

The following text captures the solution by describing the neccesary changes in bold font compared with 3GPP TS 23.222 v18.3.0 as shown below:

* * * Enhancement based on TS 23.222 v18.3.0 * * * * #### 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M (see NOTE 1); The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, Service KPIs (optional).. Shareable information; O (see NOTE 2); Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained.. NOTE 1: The interface details shall be not present, if the API status is inactive. NOTE 2: If the shareable information is not present, the service API is not allowed to be shared.. * * * Enhancement based on TS 23.222 v18.3.0 * * * * ### 8.7.3 Procedure Figure 8.7.3-1 illustrates the procedure for discover service APIs. The service API discover mechanism is supported by the CAPIF core function. Pre-conditions: 1. The API invoker is onboarded and has received an API invoker identity. 2. The CAPIF core function is configured with a discover policy information (e.g. to restrict discover to category of APIs) for API invoker(s). 3. The CAPIF core function obtains the service API information with the corresponding API status through service API publish procedure. Figure 8.7.3-1: Discover service APIs 1. The API invoker sends a service API discover request to the CAPIF core function. It includes the API invoker identity, and may include query information. 2. Upon receiving the service API discover request, the CAPIF core function verifies the identity of the API invoker (via authentication). The CAPIF core function retrieves the stored service API(s) information from the CAPIF core function (API registry) as per the query information in the service API discover request. Further, the CAPIF core function applies the discover policy and performs filtering of service APIs information retrieved from the CAPIF core function. 3-4. If the required service API is inactive, the CAPIF core function may request the API management function to influence the require service API to be active, the CAPIF core function can obtain the required service API information including the API interface details after the API is activated by API management function. NOTE 1: This solution assumes that the AEF instance is already instantiated. NOTE 2: the detailed API of service API activation between CAPIF core function and API management function is to be specified in normative work. 5. The API publish update procedure is performed, with the updated API status (i.e. active). 6. The CAPIF core function sends a service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

### 6.8.2	Architecture Impacts

This solution is presented based on the existing CAPIF architecture in 3GPP TS 23.222.

### 6.8.3	Corresponding APIs

This solution is based on service API publish and service API discover, no new APIs are introduced.

### 6.8.4	Solution evaluation

This solution proposes the API-based service API activation as part of the service API discover procedure. For situations where the AEF instance is already instantiated but the required service API is not active, the CAPIF core function can request the API management function to trigger the activation of the required service API. This solution solves the two open issues listed in key issue#7.

## 6.9	Solution #8: AEF based instantiation for service API discover

### 6.9.1	Solution description

#### 6.9.1.1	General

This paper proposes a solution for key issue #7 on CAPIF enhancement for AEF status, to introduce the API based instantiation in service API discover procedure.

For the current CAPIF specificiation, the status of AEF instance is assumed to be instantiated. However, the AEF instance may be instantiable but not yet instantiated, and its service API information will be available after the AEF instance is instantiated by management system.

Therefore, this solution proposes that the AEF status including instantiated, and instantiable but not yet instantiated should be added, and the related service API discover should be enhanced to trigger the AEF instantiation if the required API is not available.

#### 6.9.1.2	Procedure

The following text captures the solution by describing the neccesary changes in bold font compared with 3GPP TS 23.222 v18.3.0 as shown below:

* * * Enhancement based on 3GPP TS 23.222 v18.3.0 * * * * ### 8.7.3 Procedure Figure 8.7.3-1 illustrates the procedure for discover service APIs. The service API discover mechanism is supported by the CAPIF core function. Pre-conditions: 1. The API invoker is onboarded and has received an API invoker identity. 2. The CAPIF core function is configured with a discover policy information (e.g. to restrict discover to category of APIs) for API invoker(s). 3. The CAPIF core function has obtained the AEF status from the management function (e.g. API management function), the AEF status may include instantiated, and instantiable but not yet instantiated. NOTE: The interface details (e.g. IP address, port number, URI) and service API status shall not be present, if the AEF status is instantiable but not yet instantiated. Figure 8.7.3-1: Discover service APIs 1. The API invoker sends a service API discover request to the CAPIF core function. It includes the API invoker identity, and may include query information. 2. Upon receiving the service API discover request, the CAPIF core function verifies the identity of the API invoker (via authentication). The CAPIF core function retrieves the stored service API(s) information from the CAPIF core function (API registry) as per the query information in the service API discover request. Further, the CAPIF core function applies the discover policy and performs filtering of service APIs information retrieved from the CAPIF core function. 3. If the required service API is not available due to AEF status is instantiable but not be instantiated, the CAPIF core function may request the management function (e.g. OAM system) to instantiate the related AEF instance. After the AEF is instantiated, the CAPIF core function may obtain the required service API information including the API interface details (e.g. IP address, port number, URI). NOTE: This solution is assumed that CCF will directly interact with OAM system for AEF instantiation. 4. The API publish procedure is performed with the service API information. 5. The CAPIF core function sends a service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

### 6.9.2	Architecture Impacts

This solution is presented based on the existing CAPIF architecture in 3GPP TS 23.222.

### 6.9.3	Corresponding APIs

This solution is based on service API publish and service API discover, no new APIs are introduced.

### 6.9.4	Solution evaluation

This solution proposes the AEF based instantiation in service API discover procedure. For the AEF instance is instantiable but not be instantiated, the CAPIF core function can request the management function to make service API available. This solution solves the two open issues in key issue#7. The feasibility of this solution is to be evaluated on whether CCF can directly interact with OAM system for AEF instantiation considering the CAPIF architecture. Further for the AEF status, coordination is required with SA5.

## 6.10	Solution #9: Support more API invoker info in RNAA

### 6.10.1	Solution description

#### 6.10.1.1	General

This solution provides more API invoker info related to RNAA so that the resource owner can know better about which user and/or which service (and its provider) is asking for sensitive information (e.g. location) of the resource owner.

#### 6.10.1.3	Procedures

##### 6.10.1.3.1	Authorization

Figure 6.10.1.3.1-1: Procedure for the API invoker obtaining authorization for service API access

The procedure is similar to clause 8.11.3 of 3GPP TS 23.222 [2] with the following difference:

-	In step 1, more information about API invoker (application service info like ASP id, application id) and purpose of data processing (e.g. location resource to be used for tracking on the map) can be sent from API invoker to CCF during service API authorization procedure.

NOTE 1:	Application service info are applicable for both AF side API invoker and UE side API invoker.

-	In step 2, if received, the CCF additionally verifies application service info.

NOTE 2:	To support RNAA, the CCF checks whether the API invoker is permitted to access the requested service API based on the API invoker's subscription information and resource owner consent.

### 6.10.2	Architecture Impacts

Existing CAPIF architecture in 3GPP TS 23.222 [2] is used to accommodate new impact to CAPIF-1/1e reference point.

The CAPIF architecture shall support CAPIF-8 reference point between CCF and resource owner.

### 6.10.3	Corresponding APIs

Existing CAPIF service API authorization API is enhanced.

### 6.10.4	Solution evaluation

This solution addresses KI#1 with more granular information about API invoker in the context of RNAA. It also addresses the purpose of data processing in KI#1. The impacted CAPIF procedure includes service API authorization.

## 6.11	Solution #10: Access Control Management of Service API

### 6.11.1	Solution description

#### 6.11.1.1	General

This solution is for Key issue #1 to give a description of the service API invocation procedure based on authorization for granular service access.

The API invoker performs the authorization based on granular service access (e.g., per service API or per service operation or per service resource). The subsequent service API invocation over CAPIF-2/2e by API invoker on the AEF should comply with the received authorization.

NOTE:	The details of authorization based on granular service access is the responsibility of SA3

### 6.11.2	Architecture Impacts

This solution is based on the existing CAPIF architecture in 3GPP TS 23.222 [2].

### 6.11.3	Corresponding APIs

The security APIs specified in clause 10.6 of 3GPP TS 23.222 [2] is to be used.

### 6.11.4	Solution evaluation

This solution addresses the Key issue#1 by emphasizing the need for authorization based on granular service access. The authorization solution itself is in the scope of SA3.

## 6.12	Solution #11: Resource Owner Consent Revocation

### 6.12.1	Solution description

#### 6.12.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

In current specification, the consent of resource access will only be requested by API invoker before service API invocation procedure. However, the scenario where the resource owner's consent is revoked after the API invoker obtains it has not been considered (e.g., for a certain API invoker, the location permission may change from allowed to not allowed). To support the resource owner consent revocation a procedure of resource owner consent revocation is needed.

#### 6.12.1.2		Procedure for resource owner consent revocation

Pre-condition:

1.	The resource owner can communicate with the API invoker.

2.	The service API access requires obtaining consent from resource owner.

3.	The resource owner function has provided the user consent for the API invoker.

Figure 6.12.1.2-1:	Procedure for resource owner consent revocation

1.	The resource owner function sends a resource owner consent revocation request to CAPIF core function/Authorization function via CAPIF-8 reference point in order to provide resource owner revocation information.

2.	The CAPIF core function/Authorization function processes the request and sends a revocation response to the resource owner function. Then triggers the procedure for revoking API invoker authorization as described in subclause 8.23.4 of 3GPP TS 23.222 [2] via CAPIF-8 reference point.

3.	The CAPIF core function/Authorization function triggers the procedure for revoking API invoker authorization as described in subclause 8.23.4 of 3GPP TS 23.222 [2]. The CAPIF core function/Authorization function sends revoke API invoker authorization request to the AEF with the details of the API invoker and the service API providing resource accessing.

### 6.12.2	Architecture Impacts

This solution is based on architecture of CAPIF as described in 3GPP TS 23.222 [2].

The CAPIF architecture shall support resource owner consent revocation procedure over CAPIF-8 reference point.

### 6.12.3	Corresponding APIs

NOTE:	APIs corresponding to this solution will be specified during normative phase based on SA3 outcome.

### 6.12.4	Solution evaluation

This technical solution addresses KI#1 and is a technically feasible solution for resource owner consent revocation to be considered for consent management.

## 6.13	Solution #12: Capturing resource owner consent

### 6.13.1	Solution description

#### 6.13.1.1	General

Even though consent can be captured through a variety of methods and techniques (e.g., ticking a box on a website or writing/accepting a letter confirming the grant for processing personal data), having the consent captured during runtime is a well-established approach for some scenarios in which the resource owner grants or denies the API invoker’s access request.

While capturing the consent during runtime, the RO is presented the purpose (of data processing), indicating what the API invoker intends to do with the personal information resources of the RO (e.g., read location for spatial analytics or read network activity for fraud detection). In turn, the RO may grant access to personal information resources based on the purpose, for instance: grant access to personal information resources for fraud detection but deny access for other purposes (e.g., advertising).

#### 6.13.1.2	Procedure for capturing resource owner consent

Figure 6.13.1.2-1 presents the procedure for capturing and storing the consent using RNAA.

Pre-condition:

1.	The API invoker is onboarded as in clause 8.1 in 3GPP TS 23.222 [2] and has received an API invoker identity.

2.	The notification criteria (e.g. allowed timeslot for notification) towards ROF is available at CCF.

Figure 6.13.1.2-1: Procedure for capturing resource owner consent

1.	The API invoker(s) send Obtain service API authorization request(s) to the CAPIF core function for obtaining permission(s) to access the service API(s) by including the API invoker(s) identity information and any information required for authentication of the API invoker(s). The API invoker(s) include in the request information about the purpose and the targeted resources and operations to be performed on those resources.

NOTE 2:		Authorization details will be specified by SA3.

2.	The CCF validates the authentication of the API invoker(s) (using authentication information) and checks whether the API invoker is permitted to access the requested resources, i.e., checks if a consent record for the signalled purpose is already in place. If the consent for accessing the requested resources for the signalled purpose is already granted, the information flow continues in step 4 of Figure 6.13.1.2-1.

NOTE 3:	The authentication process is specified in clause 6.5.2 of 3GPP TS 33.122 [3].

NOTE 4:	Whether the CCF is allowed to hold/cache consent records depends on local regulations.

- If the consent needs to be captured from the RO, the bulk processing of requests takes place as soon as the notification criteria are met. The bulk processing may include the CCF sending one event notification to the ROF (aggregating the multiple requests from the API invoker(s)) and consent capture from RO.

NOTE 5:	The details of the interaction between RO and ROF for capturing the consent are considered out-of-scope of 3GPP.

4.	If the RO grants the consent for accessing personal information resources, the authorization information to access the service APIs is sent to the API invoker(s) in the obtain service API authorization response(s).

5.	The service API invocation(s) take place considering the authorization information received in the previous step.

#### 6.13.1.3	Enhancement to clauses in 3GPP TS 23.222

Potential changes to clauses 8.8.1, 8.8.6, 10.4.1 (among others) in 3GPP TS 23.222 will be based on SA3 solution for procedures on CAPIF-8 ad further elaborated during the normative phase.

### 6.13.2	Architecture Impacts

None.

### 6.13.3	Corresponding APIs

NOTE:	Whether new or enhanced APIs are required in support of this solution is in scope of SA3.

### 6.13.4	Solution evaluation

This solution addresses the following open issue in KI#1:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

This solution provides a feasible solution for managing user consent where the CCF can combine one or more user consent events and notify the ROF. This approach saves multiple communications between ROF and CCF (AuthF) in scenarios where too many user consent interactions are triggered to the end user’s application interactions.

## 6.14	Solution #13: Requesting user consent

### 6.14.1	Solution description

#### 6.14.1.1	General

This solution is for Key issue #1 on the issue of how the consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function (CCF).

3GPP TS 23.222 [2] provides a procedure in clause 8.31 for an API invoker to obtain authorization from resource owner. In the procedure it is the CCF authorization function that obtains the authorization by interacting with the resource owner via the resource owner function (ROF). The trigger for the interaction between the CCF and ROF to obtain such authorization is the API invoker request to obtain resource owner authorization information to invoke the service API exposed by the API exposing function.

In order to reduce the required number of interactions between the CCF and ROF, this solution enables the CCF authorization function to use such an API invoker request as a trigger to pre-emptively request for resource owner authorization information for additional API exposing functions and / or API invokers. The decision to make a request with a wider authorization scope is based on CCF configuration.

It is critical to highlight that any authorization response sent to an API invoker by the CCF will only include authorization information specific to that API invoker and for the specific service requested by that API invoker.

#### 6.14.1.2		Procedure for requesting user consent

Pre-condition:

1.	The API invoker is onboarded and has received an API invoker identity.

2.	The ROF can communicate with the CCF (Authorization function).

3.	The service API access requires obtaining consent from the resource owner.

Figure 6.14.1.2-1: Procedure for resource owner consent revocation

1.	The CCF is configured for requesting bulk resource owner consent.

The bulk resource owner consent configuration includes lists of API invokers and associated AEF service APIs, where the assumption is that there is an association between the API exposing functions and API invokers listed in the CCF configuration. The configuration indicates that should a request be received for one of the identified API invokers, resource owner consent should be sought for each of the associated AEF service APIs for all the API invokers in the list. For example, if the list includes API invoker 1 & 2 each with associated AEF service API service-A, resource owner consent should be requested for service-A for both API invoker 1 & 2 if either API invoker 1 or 2 makes a to service API authorization request to the CCF for service-A.

2. 	The API invoker sends an obtain service API authorization request to the CCF for obtaining permission to access the service API by including the API invoker identity information and any information required for authentication of the API invoker (reference step 1 of clause 8.11.3 of 3GPP TS 23.222 [2]).

3. 	The CCF validates the authentication of the API invoker (using authentication information) and checks whether the API invoker is permitted to access the requested service API (reference step 2 of clause 8.11.3 of 3GPP TS 23.222 [2]), which entails checking whether a consent record is already in place for the API invoker.

NOTE 1:	Whether the CCF is allowed to retain consent records depends on local regulations.

4.	If authorization information is not available regarding whether the API invoker is permitted to access the requested service API (i.e., there is no applicable consent record available at the CCF), the authorization function determines the authorization by interacting with the resource owner via the ROF. Based on the configured bulk resource owner consent information at the CCF, the scope of the request towards the ROF can be expanded to include additional API invokers and or AEFs based on the assumption that there is an association between the requesting API invoker and the API exposing functions and API invokers listed in the CCF configuration, The expansion of the scope in the request is an enhancement to step 1 of clause 8.31.3 of 3GPP TS 23.222 [2]).

If authorization to access the service API for the API invoker the is already available at the CCF, i.e., through a previous interaction with the ROF, step 4 is skipped.

NOTE 2:	The detailed procedure to obtain the resource owner's authorization information is in scope of SA3. However, it is recommended that the resource owner is provided with at least the option to only grant authorization for only the API invoker initiating the authorization request and then only for the specific service being requested.

5.	Based on the RO provided consent, the authorization information (with scope that is specific to only that API invoker) to access the specifically requested service API is sent to the API invoker in the obtain service API authorization response (reference step 3 of clause 8.11.3 of 3GPP TS 23.222 [2]).

6.	The API invoker sends service API invocation request to the AEF with the resource owner authorization information received in step 5 (reference step 2 of clause 8.31.3 of 3GPP TS 23.222 [2]).

7.	The API invoker receives the service API invocation response resulting from the service API invocation once the AEF has checked whether the API invoker is authorized to invoke that service API based on the authorization information (reference step 3 of clause 8.31.3 of 3GPP TS 23.222 [2]).

### 6.14.2	Architecture Impacts

This solution is based on the existing architecture of CAPIF as described in 3GPP TS 23.222 [2] and relates to the CAPIF-8 reference point.

### 6.14.3	Corresponding APIs

There are no impacts anticipated for the existing obtain service API authorization request (or response) sent over CAPIF 1/1e.

This solution impacts the CAPIF-8 interface with new APIs.

NOTE: Security related API(s) will be addressed by SA3.

Editor’s note: Potential impacts to the reference points between the CCF and API provider domain functions, and their associated APIs, are FFS with regards to providing the bulk resource owner consent information configuration to the CCF.

### 6.14.4	Solution evaluation

This solution addresses open issue 1 of KI#1 and provides a mechanism for obtaining resource owner consent as part of the overall resource owner consent management solution that can reduce the required number of interactions between the CCF (Authorization Function) and ROF.

## 6.15	Solution #14: Enhancement to API invoker authorization with the Purpose of Data Processing

### 6.15.1	Solution description

#### 6.15.1.1	General

This paper is for KI #1 to manage the purpose of data processing.

According to clause Annex V.2 and V.3 in 3GPP TS 33.501, the user consent parameters shall be bound to the purpose of data processing. The AEF shall determine the purpose of data processing prior to the data processing. If the purpose of data processing is not obtained from the service request, the AEF shall request it or otherwise deny the service. Thus, some enhancement is necessary to support obtain the purpose of data processing from API invoker.

#### 6.15.1.2	Impact to existing CAPIF procedures

The CAPIF procedure in 3GPP TS 23.222 can be enhanced (highlighted in bold italics) as follows.

### 8.31.3 Procedure Figure 8.31.3-1 illustrates the procedure for API invoker obtaining authorization from resource owner. Pre-conditions: 1. The resource owner function can communicate with the API invoker. 2. The service API access requires obtaining authorization from resource owner. Figure 8.31.3-1: Procedure for API invoker obtaining authorization from resource owner - The API invoker requests to obtain resource owner authorization information to invoke the service API exposed by the API exposing function. The authorization function provides the authorization by interacting with the resource owner via the resource owner function. More information about API invoker (application service info like ASP id, application id) and the purpose of data processing can be sent from API invoker to CCF. And CCF will include this purpose of data processing to obtain user consent. NOTE1: The detailed procedure to obtain the resource owner's authorization information is specified in TS 33.122 [12]. NOTE2: Coordination with SA3 is required on the step 1. 2. The API invoker sends service API invocation request to the API exposing function with the resource owner authorization information received in step 1. 3. The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the authorization information.

### 6.15.2	Architecture Impacts

This solution is based on architecture of CAPIF as described in 3GPP TS 23.222 [2].

### 6.15.3	Corresponding APIs

No new APIs are introduced.

### 6.15.4	Solution evaluation

This solution addresses the open issue of purpose of data processing in KI #1, the impacted CAPIF procedure is the procedure of API invoker obtaining authorization from resource owner.

## 6.16	Solution #15: Enhancing the description of authorization function capabilities

### 6.16.1	Solution description

This solution adds the relevant capabilities for storing the resource owner consent information to authorization function’s functionality description in 3GPP TS 23.222 (highlighted in bold italics) as follows.

### 6.3.7 Authorization function The authorization function consists of the following capabilities: - Receiving and storing authorization from the resource owner; and - Providing the API invoker with the authorization information which is needed to access the resource owner’s resources. NOTE1: In the current release, the authorization function is an internal entity of the CAPIF core function. NOTE2: Mechanisms for storing authorization information is in the scope of SA3.

### 6.16.2	Architecture Impacts

This solution is based on architecture of CAPIF as described in 3GPP TS 23.222.

### 6.16.3	Corresponding APIs

No new APIs are introduced.

### 6.16.4	Solution evaluation

This solution addresses KI #1, by adding relevant capabilities to the authorization function’s functionality description for resource owner consent management.

## 6.17	Solution #16: Obtaining Resource Owner Consent

### 6.17.1	Solution description

#### 6.17.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

From the description of authorization mechanism in 3GPP TS 33.122, CCF will check and store the resource owner consent information. However, there is no relevant statements about how to get the authorization from resource owner. This solution enables the CCF to obtain resource owner consents which are used to authorize subsequent service API invocation.

#### 6.17.1.2		Procedure for obtaining resource owner consent

Pre-condition:

- The service API access requires obtaining consent from resource owner.

- The CCF has received service API authorization request from API invoker.

Figure 6.17.1.2-1: Procedure for obtaining resource owner consent

1. The CAPIF core function sends a resource owner consent request to the resource owner client with its CAPIF core function ID when it determines to obtain the resource owner consent. For example, in order to reduce real-time interruptions to the resource owner and reduce waiting time for API invokers to obtain authorization, the CCF is configured to obtain consents in advance.

2. The resource owner client responds to the CAPIF core function with its resource owner ID and consents.

3. The CAPIF core function stores the resource owner consents with the resource owner ID.

### 6.17.2	Architecture Impacts

This solution is based on the existing architecture of CAPIF as described in 3GPP TS 23.222 [2] and relates to the CAPIF-8 reference point.

### 6.17.3	Corresponding APIs

This solution impacts the CAPIF-8 interface with new APIs. The APIs will be specified in normative work.

### 6.17.4	Solution evaluation

This solution addresses open issue 1 of KI#1 and provides a mechanism for the CCF to obtain resource owner consents which are used to authorize subsequent service API invocation. This solution can reduce waiting time for API invokers to obtain authorization. Besides, this solution support obtaining bulk resource owner consent information.

## 6.18	Solution #17: Enhanced CAPIF events for onboarded API invoker

### 6.18.1	Solution description

#### 6.18.1.1	General

This solution is related to key issue#5. Due to various considerations at CCF (e.g. expiry of credentials) more events can be notified to the API invoker. New CAPIF events for onboarded API invoker are introduced based on the procedures in clause 8.8 of 3GPP TS 23.222 [2].

NOTE:	The considerations related to security for event notifications from CCF to API invoker require coordination with SA3.

#### 6.18.1.2	CAPIF events to support onboarded API invoker

The following updates (in bold) to clause 8.8.6 of 3GPP TS 23.222 [2] are proposed for supporting new events for onboarded API invoker.

### 8.8.6 List of CAPIF events Table 8.8.6-1 provides a non-exhaustive list of CAPIF events. Table 8.8.6-1: List of CAPIF events Events; Events Description. Availability of service APIs; Availability events of service APIs (e.g. active, inactive). Service API updated; Events related to change in service API information. Monitoring service API invocations; Events corresponding to service API invocations. API invoker status; Events related to API invoker status in CAPIF (onboarded, offboarded, credentials due to be expired). API topology hiding status; Events related to API topology hiding status in CAPIF (created, revoked). System related events; Alarm events providing fault information. Performance related events; Events related to system load conditions.

### 6.18.2	Architecture Impacts

It is based on existing CAPIF architecture specified in 3GPP TS 23.222 [2].

### 6.18.3	Corresponding APIs

The CAPIF_Events API specified in clause 10.4 of 3GPP TS 23.222 [2] will be enhanced to accommodate the new events related to onboarded API invoker.

### 6.18.4	Solution evaluation

This solution addresses the key issue#5 and proposes a viable technical solution for addressing new events for onboarded API invoker.

# 7	Deployment scenarios

## 7.1	General

This clause provides the additional or enhanced deployment scenarios for CAPIF as specified in clause 7 of 3GPP TS 23.222 [2].

## 7.2	Deployment model #1: Updates to RNAA deployments

The following updates (in bold) are proposed to the RNAA deployments specified in clause 7.5 of 3GPP TS 23.222 [2].

## 7.5 RNAA deployments CAPIF supports RNAA and has enabled API invoker(s) to have authorized access to resources of a resource owner provided by service APIs offered by the AEF. The CCF acts as the Authorization Function and supports the authentication and authorization of the resource owner. Based on resource owner's authorization, the CCF provides the access token for a service API access to the API invoker. The API invoker performs service API invocations on the AEF by utilizing the access token. The API invoker may be deployed in the following ways: a. API invoker may be deployed as AF on the UE (i.e. 3rd party application). b. API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE. c. API invoker may be deployed on the network as AF. The resource owner is connected via a UE and can use a Resource Owner Function deployed on the UE to interact with the CCF acting as the Authorization Function for authentication and authorization i.e., granting permission to the API invoker to access resource(s) of the resource owner provided by the service API. NOTE: The details of the protocol for CAPIF supporting RNAA is specified in 3GPP TS 33.122 [12]. When API invoker is deployed on a UE (cases a and b), with suitable authorization, the API invoker is allowed to access the resources of the resource owner corresponding to the UE. When API invoker is deployed on a UE (cases a and b), with suitable authorization, the API invoker is allowed to access the resources of other UEs.

# 8	Business Relationships

The following updates (in bold) are proposed to the RNAA business relationships specified in clause 5.2 of 3GPP TS 23.222 [2].

## 5.2 CAPIF business relationships for RNAA Figure 5.2-1 shows the CAPIF business relationships for the resource owner-aware northbound API access (RNAA). Figure 5.2-1: CAPIF business relationships for RNAA The business relationships the API invoker, the CAPIF provider, and the API provider follow the description in the clause 5.1. In addition to them, the resource owner is an entity capable of granting access to a protected resource related to the resource exposed by the API provider. The API invoker and the resource owner can be the same entity or separate entities. In the current release, the resource owner is a user of a UE and can provide authorization information using the UE. The CAPIF provider and the API provider can belong to the same organization (e.g. PLMN operator), in which case the service API arrangement is not required explicitly. The CAPIF provider and the API provider (e.g. 3rd party CAPIF provider or 3rd party API provider) can belong to different organizations, in which case the service API arrangement is required. The specification of service API arrangement is out of scope of 3GPP. NOTE: In the current release, both the CAPIF provider and the API provider should belong to the same organization (e.g., PLMN operator) and the service API arrangement is not required explicitly.

# 9	Overall evaluation

## 9.1	Evaluation of key issue#1

Clause 5.1 captures the key issue on managing resource owner consent. The following open issues are listed:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	Whether (and how) “purpose of data processing” will be captured and where will it be stored?

3.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

Solution#2 specified in clause 6.3 proposes a solution to address open issue#1 in nested API scenario. This solution requires coordination and feedback on the security aspects for the appropriate user consent at AEF.

Solution#3 specified in clause 6.4 proposes a solution to address open issue#3 on finer granular access control for service API by enhancing the existing access control by introducing the resource level access.

Solution#9 specified in clause 6.10 proposes a solution to address open issue#1 and open issue#2 by enabling more granular information about API invoker being used in the context of RNAA.

Solution#10 specified in clause 6.11 proposes a solution to address open issue#3 by emphasing the need for authorization based on granular service access (e.g., per service API or per service operation or per service resource). The impact on stage 2 aspects of this procedure are covered by Solution#3 and the specific impact for authorization requires further study.

Solution#11 specified in clause 6.12 proposes a solution to address open issue#1 on communication between resource owner and Authorization function (CCF) for user consent revocation.

Solution#16 specified in clause 6.17 proposes a solution to address open issue#1 on communication between resource owner function and Authorization function (CCF) for obtaining user consent.

Solution#12 specified in clause 6.13 addresses open issue#1 to manage consent of resource owner through communication between ROF and CCF (Authorization Function) for processing user consent requests related to one or more API invokers.

Solution #13 specified in clause 6.14 addresses open issue#1 on obtaining resource owner consent via the CAPIF-8 reference point, whereby a request for such consent can be on behalf of one or more API invokers and relate to one or more API exposing functions to reduce the required number of interactions between the CCF (Authorization Function) and resource owner via the ROF.

Solution#13 specified in clause 6.14 proposes a solution to address open issue#1 on communication between resource owner function and Authorization function (CCF) for bulk user consent requests processing for pre-configured API invokers.

Solution#14 specified in clause 6.15 proposes a solution to address open issue#2 on inclusion of purpose for data processing in the mechanim for API invoker requesting/obtaining authorization information.

Solution#15 specified in clause 6.16 proposes a solution to address open issue#1 by describing the role of Authorization function in storage of authorization related information from the resource owner.

Editor's note:	Further evaluation of KI#1 considering the related solutions is FFS.

## 9.2	Evaluation of key issue#2

Clause 5.2 captures the key issue on supporting single sign-on. The following open issues are listed:

1.	Whether and how to enhance CAPIF architecture and procedures considering Single sign-on (SSO)?

SSO is a technical mechanism to be applied for Authenticating multiple API invokers of the same Resource Owner. This key issue should be considered for developing security solution. Any stage 2 impact due to the security solution(s) developed for this key issue will be considered during normative work.

## 9.3	Evaluation of key issue#3

Clause 5.3 captures the key issue on RNAA architecture enhancements. The following open issues are listed:

1.	Further enhancements to CAPIF architecture considering Resource Owner Function functionalities and its interactions with CAPIF entities (e.g., CAPIF-8 related interactions).

Solution#1 specified in clause 6.1 addresses open issue#1 by enhancing CAPIF architecture based on Backend for Frontend mechanism specified by IETF draft-ietf-oauth-browser-based-apps-17 [5]. The security aspects related to the proposed enhanced architecture are to be further studied.

## 9.4	Evaluation of key issue#4

Clause 5.4 captures the key issue on CAPIF interconnection. The following open issues are listed:

1.	How to authenticate and authorize service API access for the AEF service API(s) exposed via CAPIF-6/6e.

2.	Investigate applicable events and new events (if any) for CAPIF-6/6e.

Solution#4 specified in clause 6.5 provides a proposal to address open issue#1 and open issue#2.for obtaining authorization for an API invoker accessing service APIs of 3rd party CAPIF provider as specified in clause 6.5.1.3.1, for revoking authorization involving a 3rd party CAPIF provider as specified in clause 6.5.1.3.2, and for access control aspects over CAPIF-6 as specified in clause 6.5.1.3.3. The security aspects related to this solution are to be further addressed.

## 9.5	Evaluation of key issue#5

Clause 5.5 captures the key issue on Enhancing support to API Invoker on-boarding. The following open issues are listed:

1.	How to enhance the support of API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources?

2.	Any enhancements required to other CAPIF procedures e.g. Registering the API provider domain functions, Publish Service APIs to CCF?

Solution#5 specified in clause 6.6 provides a proposal to address open issue#1 and open issue#2 by enhancing the API invoker onboarding with onboarding criteria information from API invoker and onboarding is completed based on meeting the onboarding criteria information. Further, the solution proposes CCF sending notification to the API invoker when CCF cannot meet the onboarding criteria information at later point of time after the API invoker is onboarded. This enables API Invoker with information to either renew enrolment or offboard from CCF.

Solution#17 specified in clause 6.18 provides a proposal to address open issue#1 and open issue#2 by re-using existing CAPIF event notification procedures and introducing new events to support the onboarded API invokers. The events related to security are to be coordinated with SA3.

## 9.6	Evaluation of key issue#7

Clause 5.7 captures the key issue on CAPIF enhancement for AEF status and service API status. The following open issues are listed:

1.	Whether and what kinds of AEF availability status and service API status should be considered for service discovery?

2.	Whether and how to enhance the current CAPIF mechnisms considering AFF status and service API status?

Solution#6 specified in clause 6.7 provides a proposal to address open issue#1 and open issue#2 by enhancing the service API discover mechanism to enable service API level instantation where CAPIF's AMF will interact with the OAM system specified by SA5 for such service API level or AEF level instantiation.

Solution#7 specified in clause 6.8 provides a proposal to address open issue#1 and open issue#2 by enhancing the service API publish and service API discover mechanisms to enable service API activation. The service API activation triggers the CAPIF's AMF to interact with the OAM system specified by SA5 for AEF instantiation related to the service API.

Solution#8 specified in clause 6.9 provides a proposal to address open issue#1 and open issue#2 by enhancing the service API discover mechanism to enable AEF instantiation where CAPIF's CCF will interact with OAM system specified by SA5 for such AEF level instantiation.Coordination with SA5 is required in terms of support for AEF level or Service API level instantation by OAM system.

CAPIF's AMF is responsible for service API lifecycle management and is responsible for the service API status determination. Solution#6 and Solution#7 are technically viable based on CAPIF architecture which involve CAPIF's AMF to support the service API lifecycle aspects. Whereas, Solution#8 proposes CAPIF's CCF direct interaction with OAM system for service API lifecycle aspects.

All the solutions propose AEF instantiation inline with service API discovery, which can significantly slow down the service API discovery response time. Hence, it is to be further ascertained during normative work if service API discovery is only used as trigger or should be enhanced to support AEF/Service API level instantiations.

# 10	Conclusions

## 10.1	General conclusions

This technical report fulfills the objectives of the CAPIF Phase 3 study based on the CAPIF specified in 3GPP TS 23.222 [2], including the following:

1)	Identification of key issues (clause 5) and corresponding architecture requirements (clause 4) for enhancements to common API framework towards northbound APIs.

2)	Clause 4 identifies the requirements which may originate from other WGs like SA5 and such requirements will be addressed in the normative work.

3)	Individual solutions (clause 6) addressing the key issues including architectural impacts.

4)	Impact to deployment scenarios (clause 7) and business relationships (clause 8).

5)	Overall evaluation (clause 9) of all the key issues and their corresponding solutions

6)	Clause 10.2 to 10.z provides the conclusion for all the key issues and solutions. The candidate solutions for normative work are identified. During normative work, the candidate solutions can be subjected to enhancements or can be merged to a single solution.

NOTE:	Solutions having dependency on other working groups are identified, in particular SA3 and SA5 WGs.

## 10.2	Conclusions of key issue #1

Solution#3 and Solution#9 can be considered as candidate for normative work for addressing the aspects of granular access and granular API invoker information.

Solution#11, Solution#16, Solution#12 and Solution#13 can be considered as candidate for normative work for addressing the communication between resource owner and Authorization function (CCF) over CAPIF-8. The authentication and authorization procedures for CAPIF-8 are required.

Solution#14 can be considered as candidate for normative work for addressing the inclusion of purpose for data processing in the mechanism for API invoker requesting/obtaining authorization information.

Solution#15 can be considered as candidate for normative work for enhancing the description of the Authorization function role.

Solution#2 requires coordination and feedback on security aspects for the appropriate user consent at AEF to be considered for stage 2 impacts in normative work.

NOTE 1:	Security aspects are in scope of SA3 and further coordination with SA3 is required.

NOTE 2:	Solution#10 is related to security aspect and is in SA3 scope.

NOTE 3:	A consistent terminology for the terms "user consent", "resource owner consent", "resource owner authorization" is to be defined during normative work.

## 10.3	Conclusions of key issue #2

SSO is a technical mechanism to be applied for Authenticating multiple API invokers of the same Resource Owner. This key issue should be considered for developing security solution. Any stage 2 impact due to the security solution(s) developed for this key issue will be considered during normative work.

NOTE:	The security solutions for this key issue is in the scope of SA3.

## 10.4	Conclusions of key issue #3

The stage 2 aspects related to solution#1 are to be further evaluated for security aspects considering the overall evaluation specified in clause 9.3. Based on feedback received related to security aspects, any stage 2 impact can be considered during the normative stage.

NOTE:	The evaluation of security aspects is in the scope of SA3.

## 10.5	Conclusions of key issue #4

Solution#4 can be considered as candidate for normative work for addressing the CAPIF interconnection aspects for obtaining authorization for an API invoker accessing service APIs of 3rd party CAPIF provider as specified in clause 6.5.1.3.1, for revoking authorization involving a 3rd party CAPIF provider as specified in clause 6.5.1.3.2, and for access control aspects over CAPIF-6 as specified in clause 6.5.1.3.3. The security aspects related to this solution are to be further addressed.

Editor's note:	Overall evaluation and conclusion of Key issue#4 considering all solutions and procedures are FFS.

## 10.6	Conclusions of key issue #5

Solution#5 and Solution#17 can be considered as candidate for normative work for addressing the CAPIF event notifications for API invoker onboarding. The events related to security are to be coordinated with SA3.

Solution#5 can be considered as candidate for normative work for enhancements related to API invoker onboarding.

## 10.7	Conclusions of key issue #7

Solution#6 and Solution#7 can be considered as candidates for normative work for the aspect of CAPIF's AMF being utilized by CCF to enable AEF level or service API level instantiation.

It is to be further ascertained during normative work if service API discovery is only used as trigger or should be enhanced to support AEF/Service API level instantiations.

Coordination with SA5 is required in terms of support for AEF level or Service API level instantation by OAM system.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-02 | SA6#59 |  |  |  |  | TR Skeleton as approved by 3GPP SA6 in S6-240385 | 0.0.0 |
| 2024-03 | SA6#59 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-240491, S6-240783, S6-240492, S6-240493, S6-240530, S6-240751, S6-240577 | 0.1.0 |
| 2024-04 | SA6#60 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-241341, S6-241597, S6-241343, S6-241647, S6-241348, S6-241349 | 0.2.0 |
| 2024-05 | SA6#61 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-242325, S6-242326, S6-242311, S6-242327, S6-242757, S6-242356, S6-242358, S6-242359, S6-242331, S6-242360, S6-242361, S6-242362, S6-242758 | 0.3.0 |
| 2024-07 | SA6#62-ad hoc-e |  |  |  |  | Implementation of the following pCRs approved by SA6: S6a240366, S6a240367, S6a240190, S6a240368, S6a240249, S6a240231, S6a240369, S6a240331, S6a240296, S6a240297, S6a240298, S6a240248 | 0.4.0 |
| 2024-08 | SA6#62 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-243358, S6-243511, S6-243713, S6-243664, S6-243512, S6-243663, S6-243513, S6-243468, S6-243325, S6-243514, S6-243332, S6-243326, S6-243515, S6-243471, S6-243472, S6-243473, S6-243720, S6-243516, S6-243476, S6-243760, S6-243585, S6-243586, S6-243587, S6-243517, S6-243341, S6-243342, S6-243331 | 0.5.0 |
| 2024-09 | SA#105 | SP-241201 |  |  |  | Submitted to SA#105 for information | 1.0.0 |