| 3GPP TR 23.946 V0.6.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Guidelines for CAPIF Usage; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Overview of CAPIF	6

4.1	Introduction	6

4.2	Functional Architecture	6

4.3	Functional Entities	8

4.4	RNAA and CAPIF	8

4.5	Relationship between RNAA and OAuth 2.0	9

4.6	Overview CAPIF operations	10

4.6.1	General	10

4.6.2	Usage of CAPIF by API invoker	10

4.6.3	Usage of CAPIF by API provider	11

5	Role of Stakeholders for API Exposure	14

5.1	Stakeholders in CAPIF	14

5.2	Basic roles	14

5.2.1	CAPIF provider	14

5.2.2	API provider	14

5.2.3	API invoker	15

5.2.4	Resource owner	15

5.3	Mapping of stakeholders to CAPIF roles	15

6	Exemplary Use Cases and Adoption	15

6.1	ETSI MEC deployment based on CAPIF	15

6.2	GSMA Operator Platform deployment via CAPIF	15

6.3	Open-source implementation of CAPIF	16

6.4	NEF Publishes an API	17

6.5	Usage of CAPIF in different phases of an Application	29

6.6	The deployment of the API Invoker as AF on the UE for RNAA	30

6.7	The deployment of the API Invoker as AF on the network for RNAA	32

6.8	API consumption from an API Invoker	34

7	Summary	45

Annex A: Examples from OpenCAPIF	46

Annex B: Change history	46

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

This TR provides guidelines for CAPIF usage for the benefit of the Application developer and API provider communities.

1	Scope

This TR provides guidelines for CAPIF usage for the benefit of the Application developer and API provider communities. This document also describes the usage and deployment options in CAPIF.

The work takes into consideration the work done for CAPIF in 3GPP TS 23.222 [2] and 3GPP TS 29.222 [3] and 3GPP TS 33.122 [4], and other related work outside 3GPP.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[4]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[5]		ETSI GS MEC 011 V3.1.1: "Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[6]	3GPP TR 23.958: "Edge Application Standards in 3GPP and Alignment with External Organizations ".

[7]	EVOLVED-5G ICT-41 project, https://evolved-5g.eu

[8]	NEF emulator version 2.2.3, February 2024. https://github.com/EVOLVED-5G/NEF_emulator

[9]	ETSI Software Development Group: "OpenCAPIF", https://ocf.etsi.org

[10]	ETSI Software Development Group GitLab repository: "OpenCAPIF", https://labs.etsi.org/rep/ocf/capif.

[11]		GSMA PRD OPG.02 - "Operator Platform Telco Edge Requirements Version 6.0", https://www.gsma.com/futurenetworks/wp-content/uploads/2024/02/OPG.02-v6.0-Operator-Platform-Requirements-and-Architecture.pdf.

[12]		OCF Project Postman Collection. https://ocf.etsi.org/documentation/develop/testing/postman/.

[13]		OCF Project Common Operations, https://ocf.etsi.org/documentation/develop/testing/testplan/common_operations/.

[14]		OCF Project Postman Collection. https://ocf.etsi.org/documentation/develop/testing/postman/.

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the terms given in clause 3 of 3GPP TS 23.222 [2] and clause 3 of 3GPP TS 29.222 [3] shall also apply.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the abbreviations given in clause 3 of 3GPP TS 23.222 [2] and clause 3 of 3GPP TS 29.222 [3] shall also apply.

4	Overview of CAPIF

4.1	Introduction

3GPP specifies multiple northbound API-related specifications. To avoid duplication and inconsistency of approaches between different API specifications and to specify common services (e.g. authorization), 3GPP developed a common API framework (CAPIF) that includes aspects applicable to any northbound service APIs. The key CAPIF specifications are:

-	3GPP TS 23.222 [2] defines the CAPIF architecture and procedures and was developed by the 3GPP SA6 3GPP working group;

-	3GPP TS 29.222 [3] defines the API messages and protocol for CAPIF APIs and was developed by the 3GPP CT3 working group; and

-	3GPP TS 33.122 [4] defines CAPIF security procedures and was developed by the 3GPP SA3 working group.

4.2	Functional Architecture

The CAPIF functional architecture is specified in 3GPP TS 23.222 [2] and consists of CAPIF core function, API Provider Domain Functions and API invoker. CAPIF architecture is based on the well-known Service Oriented Architecture (SOA) design paradigm, where a Service producer (i.e. API provider) is able to publish (1) the offered service APIs which can be discovered (2) by the Service consumers (e.g. API invokers) and further can invoke (3) the discovered service APIs as per the authorization.

Figure 4.2-1: CAPIF based on SOA

Figure 4.2-1 provides an illustration of CAPIF based on SOA with the following relationship:

1.	The functionalities related to Service Access Controller are supported by CAPIF core function.

2.	The Service producer is the API Provider Domain Functions.

3.	The Service consumer is the API Invoker.

Most CAPIF APIs are provided by a CAPIF core function entity and can be consumed by applications (API invokers) and application providers (CAPIF defines an API provider as three functions: API exposure, API publishing, and API management). CAPIF core function APIs enable onboarding, publishing, discovery, authentication, registration, authorization, logging, charging, monitoring, configuration, topology hiding, and other procedures. One CAPIF API (AEF security) is provided by the API exposure function.

Figure 4.2-2: Functional model for the CAPIF to support 3rd party API providers

(adapted from 3GPP TS 23.222 [2] Figure 6.2.1-1: Functional model for the CAPIF to support 3rd party API providers).

The CAPIF architecture includes multiple deployment models e.g. centralized vs. distributed, single vs. multiple API providers (see 3GPP TS23.222 [2] clause 7).

4.3	Functional Entities

API invoker: Typically provided by a 3rd party application provider who has service agreement with PLMN operator

The API invoker supports several CAPIF capabilities such as supporting authentication, obtaining authorization, and discovering service APIs and invoking service APIs.

CAPIF core function (CCF): Supports capabilities used by other functional entities, for example the following:

-	For the API invoker: authenticating the API invoker, providing authorization information, and service API discovery.

-	For the API exposing function: (AEF) providing the service API access policy, providing API routing information, and charging of service API invocations.

-	For the API publishing function: publishing and storing the service APIs information.

-	For the API management function: providing the service API invocation log for auditing, storing configurations of the API provider policies, and updating registration information of API provider domain functions.

A CAPIF core function can also interact with another CAPIF core function for API publishing and discovery.

API exposing function (AEF): The API exposing function (AEF) is the provider of the Service APIs and is also the service communication entry point of the Service API to API invokers.

API publishing function (APF): The API publishing function (APF) enables the API provider to publish the Service APIs information to enable the discovery of Service APIs by the API invoker.

API management function (AMF): The API management function (AMF) enables the API provider to manage service APIs such as querying the Service API invocation log for auditing, monitoring the events, and configuring the API provider policies.

Authorization function: The authorization function enables receiving authorization from the resourse owner function and providing the API invoker with the authorization information which is needed to access the resource owner’s resources. This function is used in RNAA scenarios.

Resource owner function (ROF): The resource owner function enables the authorization for resource access and managing and revoking authorization for resource access. This function is used in RNAA scenarios.

4.4	RNAA and CAPIF

RNAA (Resource owner-aware Northbound API Access) is an OAuth 2.0-based authorization mechanism for CAPIF and enables API invokers to have authorized access to resources of a resource owner provided by service APIs offered by the API exposing function.

The relationship between the RNAA and CAPIF is described in this subclause. The CAPIF architecture given in subclause 6.2.0 of 3GPP TS 23.222 [2] can use OAuth 2.0 token's mechanism to authorize API invokers. As per OAuth 2.0, API invoker performs the function of the client, the CAPIF core function performs the function of the authorization server, and the API exposing function performs the exposure of the protected resources. The API invoker is authorized with an authorization grant type of client credentials described in subclause 6.5 of 3GPP TS 33.122 [4].

The RNAA architecture given in subclause 6.2.3 of 3GPP TS 23.222 [2] supports an authorization grant type of authorization code grant. The resource owner function in the RNAA architecture has the role of the resource owner, authorizing the API invoker to invoke the API exposing function.

Figure 4.4-1 shows the architectural model for the RNAA which allows the resource owner to provide authorization to the API invocation. The authorization function and the resource owner function are used in RNAA scenarios.

Figure 4.4-1: High level functional architecture for CAPIF supporting RNAA

4.5	Relationship between RNAA and OAuth 2.0

This subclause shows the relationship between RNAA and OAuth 2.0. The table 4.5-1 shows the mapping of OAuth 2.0 and RNAA. The details of OAuth 2.0 are specified in IETF RFC 6749 [15] and the details of RNAA are in 3GPP TS 23.222 [2].

Table 4.5-1: RNAA relationship with OAuth 2.0

| Role | OAuth 2.0 | RNAA |
| --- | --- | --- |
| The entity capable of granting access to a protected resource. | Resource Owner | Resource owner via Resource owner function |
| The server hosting the protected resources, capable of accepting and responding to protected resource requests using access tokens. | Resource Server | API exposing function |
| The application making protected resource requests on behalf of the resource owner and with its authorization. | Client | API invoker |
| The server issuing access tokens to the client after successfully authenticating the resource owner and obtaining authorization. | Authorization Server | Authorization function |
| The software agent responsible for retrieving and facilitating end-user interaction for authorization. | User Agent | Resource owner function hosting environment |

4.6	Overview CAPIF operations

4.6.1	General

This subclause provides the overview of CAPIF operations from the perspective of an Application (API invoker) and API provider. The detail of the CAPIF overall operations is shown in Annex A of 3GPP TS 23.222 [2].

4.6.2	Usage of CAPIF by API invoker

Figure 4.6.2-1 describes the usage of CAPIF by API invoker for APIs which does not involve resource owner authorization.

Figure 4.6.2-1: Usage of CAPIF by API invoker for APIs which does not involve resource owner

1.	API invoker onboards to the CAPIF core function, as specified in subclause 8.1 of 3GPP TS 23.222 [2].

2.	The API invoker authenticates with the CAPIF core function, as specified in subclause 8.10 of 3GPP TS 23.222 [2].

3.	The API invoker discovers the service API, as specified in subclause 8.7 of 3GPP TS 23.222 [2].

4.	To access the service API, the API invoker obtains authorization with the CAPIF core function, as specified in subclause 8.11 of 3GPP TS 23.222 [2].

5.	The API invoker performs service API invocation, as specified in subclause 8.16 of 3GPP TS 23.222 [2].

Figure 4.6.2-2 describes the usage of CAPIF by API invoker for APIs which involves resource owner authorization.

Figure 4.6.2-2: Usage of CAPIF by API invoker for APIs which involve resource owner

1.	API invoker onboards to the CAPIF core function, as specified in subclause 8.1 of 3GPP TS 23.222 [2].

2.	The API invoker authenticates with the CAPIF core function, as specified in subclause 8.10 of 3GPP TS 23.222 [2].

3.	The API invoker discovers the service API, as specified in subclause 8.7 of 3GPP TS 23.222 [2].

4.	To access the service API, the API invoker obtains authorization with the CAPIF core function, and then requests service API invocation as specified in subclause 8.31 of 3GPP TS 23.222 [2].

5.	The API invoker performs service API invocation, as specified in subclause 8.16 of 3GPP TS 23.222 [2].

4.6.3	Usage of CAPIF by API provider

Figure 4.6.3-1 describes the usage of CAPIF by API provider for APIs which does not involve resource owner authorization.

Figure 4.6.3-1: Usage of CAPIF by API provider for APIs which does not involve resource owner

- The API publishing function sends service API publish request to the CAPIF core function, as specified in subclause 8.3 of 3GPP TS 23.222 [2].

2.	The API provider authenticates the API invoker prior to the service API invocation, utilizes the API exposing function, as specified in subclause 8.14 of 3GPP TS 23.222 [2].

3.	The API provider authenticates the API invoker upon invocation of the service APIs, utilizes the API exposing function, as specified in subclause 8.15 of 3GPP TS 23.222 [2].

4.	The API provider authorizes the API invoker to access, as specified in subclause 8.16 of 3GPP TS 23.222 [2].

5.	The API provider controls the access of the service API by the API invoker based on policy or usage limits, as specified in subclause 8.16 and 8.17 of 3GPP TS 23.222 [2].

6.	The API provider maintains the log of the API invocations at the CAPIF core function for services such as charging, invocation history, utilizes the API exposing function, as specified in subclause 8.19 of 3GPP TS 23.222 [2].

7.	The API provider facilitates charging of the API invocations, utilizes the API exposing function, as specified in subclause 8.20 of 3GPP TS 23.222 [2].

8.	The API provider facilitates monitoring such as API invoker's ID and IP address, utilizes the API management function, as specified in subclause 8.21 of 3GPP TS 23.222 [2].

9.	The API provider utilizes the API management function for auditing, as specified in subclause 8.22 of 3GPP TS 23.222 [2].

Figure 4.6.3-2 describes the usage of CAPIF by API provider for APIs which involves resource owner authorization.

Figure 4.6.3-2: Usage of CAPIF by API provider for APIs which involve resource owner

- The API publishing function sends service API publish request to the CAPIF core function, as specified in subclause 8.3 of 3GPP TS 23.222 [2].

2.	The API provider authenticates the API invoker prior to the service API invocation, utilizes the API exposing function, as specified in subclause 8.14 of 3GPP TS 23.222 [2].

3.	The API provider authenticates the API invoker upon invocation of the service APIs, utilizes the API exposing function, as specified in subclause 8.15 of 3GPP TS 23.222 [2].

4.	API authorization is based on the authorization information obtained from the resource owner function, as specified in subclause 8.31 of 3GPP TS 23.222 [2].

5.	The API provider authorizes the API invoker to access, as specified in subclause 8.16 of 3GPP TS 23.222 [2].

6.	The API provider controls the access of the service API by the API invoker based on policy or usage limits, as specified in subclause 8.16 and 8.17 of 3GPP TS 23.222 [2].

7.	The API provider maintains the log of the API invocations at the CAPIF core function for services such as charging, invocation history, utilizes the API exposing function, as specified in subclause 8.19 of 3GPP TS 23.222 [2].

8.	The API provider facilitates charging of the API invocations, utilizes the API exposing function, as specified in subclause 8.20 of 3GPP TS 23.222 [2].

9.	The API provider facilitates monitoring such as API invoker's ID and IP address, utilizes the API management function, as specified in subclause 8.21 of 3GPP TS 23.222 [2].

10.	The API provider utilizes the API management function for auditing, as specified in subclause 8.22 of 3GPP TS 23.222 [2].

5	Role of Stakeholders for API Exposure

5.1	Stakeholders in CAPIF

Several stakeholders are considered to interact within the API exposure ecosystem, for instance: MNO, NPN owners, aggregators, Application Service Developers (ASP), authorization service providers, identity service providers, resource owners, API invokers.

The functional model description for CAPIF is given in subclause 6.2.0 of 3GPP TS 23.222 [2]. Different business relationships are considered for CAPIF as follows:

-	Basic CAPIF business relationships are given in subclause 5.1 of 3GPP TS 23.222 [2].

-	CAPIF business relationships for RNAA are provided in subclause 5.2 of 3GPP TS 23.222 [2].

-	In case of interconnection among CAPIF providers, CAPIF business relationships are given in subclause 4.12 of 3GPP TS 23.222 [2]

Some exemplary business relationships are reproduced in Figure 5.1-1.

Figure 5.1-1: Basic business relationships in CAPIF

CAPIF has three main stakeholder roles (CAPIF provider, API provider, and API invoker) and RNAA has resource owner role.

5.2	Basic roles

5.2.1	CAPIF provider

The CAPIF provider contains an instance of CAPIF core function, and API provider, and API invokers and it could be a PLMN, SNPN or 3rd party. The CAPIF provider and the API provider can be part of the same organization (e.g. PLMN operator), in which case the business relationship between the two is internal to a single organization. The CAPIF provider and the API provider can be part of different organizations, in which case the business relationship between the two must exist in subclause 5.1 of 3GPP TS 23.222 [2].

5.2.2	API provider

The API provider hosts one or more service APIs and has a service API arrangement with CAPIF provider to offer the service APIs to the API invoker.

Requirements for enabling API invoker is given in subclause 4.1.2 of 3GPP TS 23.222 [2].

5.2.3	API invoker

The API invoker is typically provided by a 3rd party application provider who has service agreement with PLMN operator. There are some models that the API invoker resides within the same trust domain as the PLMN operator network andthe API invoker is either an application on a server or an application on a UE in subclause 6.3.2 of 3GPP TS 23.222 [2].

Requirements for supporting 3rd party API providers is given in subclause 4.1.3 of 3GPP TS 23.222 [2].

5.2.4	Resource owner

The resource owner is a UE user or an MNO subscriber capable of granting access to a protected resource related to the resource exposed by the API provider. In the current release, it is a user of the UE hosting the API invoker that can authorize the API access in subclause 6.3.8 of 3GPP TS 23.222 [2].

Requirements for supporting RNAA is given in subclause 4.17.2 of 3GPP TS 23.222 [2].

5.3	Mapping of stakeholders to CAPIF roles

NOTE: The mapping of the stakeholders to the CAPIF roles is FFS.

6	Exemplary Use Cases and Adoption

6.1	ETSI MEC deployment based on CAPIF

The ETSI MEC platform is the collection of essential functionalities required to run MEC applications on a particular Virtualisation infrastructure and enable them to provide and consume MEC services. In clause B.2.2 of ETSI GS MEC 011 V3.1.1 [5] it is shown the mapping service management APIs and CAPIF APIs. Additionally, in subclause 7.2 of 3GPP TR 23.958 [6] it is possible to find a deployment option for ETSI MEC based on CAPIF.

6.2	GSMA Operator Platform deployment via CAPIF

The GSMA Operator Platform defines a common platform exposing operator services/capabilities to customers/application providers in the 5G-era. It is defined the requirement of an Operator Platform’s SBI-NR with CAPIF in clause 5.1.4.2.2 of GSMA PRD OPG.02 [11] and it is described the application session continuity support for handovers between 4G and 5G by based on combined SCEF+NEF via CAPIF in clause 5.2.2.6.7 of GSMA PRD OPG.02 [11].

Figure 6.2-1 illustrates integrated fundamental GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF. In this deployment model, the GSMA Operator Platform functions as the API invoker.

NOTE:	An Operator Platform's SBI-NR can work without CAPIF. If CAPIF is not supported, the SBI-NR API will provide an alternate means of providing these functions.

Figure 6.2-1: Integrated GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF

6.3	Open-source implementation of CAPIF

As stated in Annex  B.2.2.3 of 3GPP TS 23.222 [2], the NEF can implement the functionalities of the API provider domain functions, as reproduced in Figure 6.3-1.

Figure 6.3-1: NEF implements the service specific aspect compliant with the CAPIF architecture

Evolved-5G [7] (ICT-41-2020 project from call H2020-ICT-2018-2020) developed an open-source project of the CAPIF Release 17 under the Apache-2.0 license. EVOLVED-5G implementation of CAPIF works with an open-source implementation of a NEF emulator available in [8]. The NEF emulator used for Evolved-5G implementation supports two services, namely Nnef_EventExposure and Nnef_AFsessionWithQoS [8].

Evolved-5G implementation has been used as the seed code for OpenCAPIF SDG [9] created by ETSI in January 2024.

OpenCAPIF has adopted the following APIs from CAPIF:

-	CAPIF API Invoker Management API

-	CAPIF Publish Service API

-	CAPIF Discover Service API

-	CAPIF Security API, including obtaining the OAuth 2.0 authorization information with JWT access tokens

-	CAPIF Events API

-	CAPIF API Provider Management API

-	CAPIF Access Control Policy API

-	CAPIF Logging API Invocation API

-	CAPIF Auditing API

The implementation also includes a Testing Suite of the above services.

Every CAPIF API is implemented in a separate container for future scalability adaptation. All APIs are exposed and support mutual TLS authentication as specified in 3GPP TS 33.122 [4].

The Source Code of OpenCAPIF and the instructions to deploy it are included in the ETSI GitLab repository [10].

6.4	NEF Publishes an API

The 5GS network exposure function (NEF) can use CAPIF capabilities to publish and manage its APIs, including managing charging. One of several possible deployment arrangements is shown in Figure 6.4-1.

Figure 6.4-1: NEF implements the CAPIF API provider domain functions

Signalling for the NEF to publish its APIs to the CAPIF core function is shown in Figure 6.4-2, followed by tables detailing the mandatory attributes of the associated request and response schema and example content for those requests and responses, which is based on publicly available examples provided by the OpenCAPIF [9].

A pre-condition is that the NEF (API management function) has been made aware of the appropriate CAPIF core function endpoint (apiRoot) with which to initiate the registration, as well as having been provided with a token to access the CAPIF core function.

Figure 6.4-2: Publish a service API

Details of the signalling flow in Figure 6.4-2 are as follows:

NOTE 1:	The JSON schema and example snippets provided in the following clauses are aligned with version 18.5.0 of 3GPP TS 29.222 [3], in which apiVersion is specified as "v1". The example snippets themselves can be tested using the OpenCAPIF Postman collection presented in Annex A and also in the OpenCAPIF Postman documentation [14].

1. CAPIF_API_Provider_Managment_API

The API provider includes:

-	Authorization HTTP header (access token)

-	Security information enabling the CCF to validate the registration (regSec)

-	An array (apiProvFuncs) of at least one API provider domain function profiles-	Optionally API provider domain information (apiProvDomInfo) and the API provider name (apiProvName)

Table 6.4-1: Register API_provider request (schema)

HTTP POST {apiRoot}/api-provider-management/v1/registrations Request headers: Authorization: "string" Request body: { "regSec": "string", "apiProvFuncs": [ { "regInfo": { "apiProvPubKey": "string", }, "apiProvFuncRole": "string", "apiProvFuncInfo": "string" } ], "apiProvDomInfo": "string", "suppFeat": "string", "apiProvName": "string" }

Given the schema provided in Table 6.4-1, an example of the API Provider registration process using OpenCAPIF open-source implementation is now described and then provided in Table 6.4-1-OCF. The presented traces contain actual values to illustrate the usage of the information elements in the JSON schema.

NOTE 2:	The security related information provided in the examples are truncated for readability, e.g., the value associated with regSec and apiProvPubKey.

Before carrying out onboarding, the provider requires the:

-	OpenCAPIF access token, that will allow the creation of the provider.

-	CA_ROOT certificate, necessary to validate the Certificates in all requests.

-	URL to which to make the request, i.e., <{apiRoot}/api-provider-management/v1/registrations>.

NOTE 3:	In OpenCAPIF, all this information is obtained in a dedicated user registration process [13].

In the presented example, the OpenCAPIF instance is deployed at capif.mobilesandbox.cloud:37211. This instance URL is assumed throughout the presented example traces.

To carry out the onboarding of a provider, the API Provider is required to provide the following fields:

-	regSec: Contains the OpenCAPIF access token obtained during registration process for the creation of the provider. The Token is also provided in the Authorization HTTP Header.

-	apiProvFuncRole: Type of provider function to be created. It can be AMF, APF or AEF.

-	apiProvPubKey: It is the certificate signing request (CSR) of the Provider’s function that OpenCAPIF will use to issue the provider’s valid certificates. Each function needs to include its own CSR to request a valid Certificate.

-	apiProvFuncInfo: Information about the provider function that is being created.

-	apiProvDomInfo: Information about the provider domain that is being created.

The API Provider onboarding request example is shown in Table 6.4-1-OCF. The example request is to onboards an API Provider with three functions: AEF, APF and AMF. Therefore, 3 CSRs are provided to obtain 3 Certificates.

Table 6.4-1-OCF: Register API provider request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations Request headers: Authorization: Bearer eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9 Content-Type: application/json Request body: { "regSec": "eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9", "apiProvFuncs": [ { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "AEF", "apiProvFuncInfo": "AEF for NEF example" }, { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "APF", "apiProvFuncInfo": "APF for NEF example" }, { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "AMF", "apiProvFuncInfo": "AMF for NEF example" } ], "apiProvDomInfo": "NEF example", "apiProvName": "OCF_NEF" }

Upon getting this Request, Open CAPIF core core function assigns identity IDs for the API provider (apiProvDomId) and each of its functions (apiProvFuncId) and returns them in the response (see next section). Furthermore, it provides certificates (apiProvCert) for each of the API provider functions based on each function’s CSR  (apiProvPubKey).

2. 201 Created

The API provider domain has been registered successfully. The URI of the created resource is returned in the "Location" HTTP header, including the assigned registration identifier (registrationId).

Table 6.4-2: Register API provider response (schema)

201 Response headers: Location: {apiRoot}/api-provider-management/v1/registrations/{registrationId} Response body: { "apiProvDomId": "string", "regSec": "string", "apiProvFuncs": [ { "apiProvFuncId": "string", "regInfo": { "apiProvPubKey": "string", "apiProvCert": "string" }, "apiProvFuncRole": "string", "apiProvFuncInfo": "string" } ], "apiProvDomInfo": "string", "suppFeat": "string", "apiProvName": "string" }

The response received from the successful onboarding request using OpenCAPIF project for the creation of a new API Provider includes the following fields:

-	apiProvDomId: Identifier of the created provider, which is also the ID returned as part of the ‘Location’ header.

-	apiProvCert: Certificate of the provider function created from the CSR sent in the onboarding request, which will be used for subsequent requests from the provider towards OpenCAPIF. Each function is provided with its own Certificate (i.e., one each for the AEF, APF and AMF functions).

-	apiProvFuncId: Identifier of the provider function created.

The example Register API provider response is shown in Table 6.4-2-OCF.

Table 6.4-2-OCF: Register API provider response (OCF example)

Response headers: Location: https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations/3a4df5f6e30a5ee34de346004ee6bb Response body: { "apiProvDomId": "3a4df5f6e30a5ee34de346004ee6bb", "regSec": "eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9", "apiProvFuncs": [ { "apiProvFuncId": "AEFb1a522967511fd6d6def0cbb3b5f05", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "AEF", "apiProvFuncInfo": "AEF for NEF example" }, { "apiProvFuncId": "APF54f49ee44f67c81fc7d7f321195531", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "APF", "apiProvFuncInfo": "APF for NEF example" }, { "apiProvFuncId": "AMFOdbe115f6759f0fec943321dc2184d", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "AMF", "apiProvFuncInfo": "AMF for NEF example" } ], "apiProvDomInfo": "NEF example", "apiProvName": "OCF_NEF" }

3. CAPIF_Publish_Service_API

The API publishing function sends information on the offered APIs by providing the API exposing function profiles array (aefProfiles). The published APIs resource created in response to the request represents all published service APIs of the NEF.

NOTE 4:	The example content in Table 6.4.3 is based on the assumption that additional supportedFeatures (and is hence set to "fffff") and apiSuppFeats (and hence is omitted) are not supported by the Service API. In addition, the attribute apiStatus is omitted, implying the Service API is active at all AEF(s) present in the aefProfiles attribute. Furthermore, it is assumed the exchange is over CAPIF-4, rather than CAPIF-6/6e. As a consequence, the pubApiPath and ccfId attributes are omitted.

Table 6.4-3: Publish API request (schema)

HTTP POST {apiRoot}/published-apis/v1/{apfId}/service-apis Request body: { "apiName": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "string", "expiry": "string", "resources": [ { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" }, { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" } ], "custOperations": [ { "commType": "string", "custOpName": "string", "operations": [ "string" ], "description": "string" } ] } ], "protocol": "string", "dataFormat": "string", "securityMethods": [ "string", ], "interfaceDescriptions": [ { "ipv4Addr": "string", "port": "int", "securityMethods": [ "string" ] } ] } ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string", "apiProvName": "string" }

To publish a service using OpenCAPIF, it is necessary to specify the following fields:

-	apiName: Name of the API to be published, which must be unique in CAPIF.

-	apiStatus: Represents the API status. In addition, if this attribute is omitted, it is understood that the API is available in all AEFs of aefProfiles. It contains the aefIds field which is a list of the AEFs that expose the API. If this list is empty or the attribute is omitted, it is understood that the API is inactive in all aefProfiles.

-	aefProfiles: List of Information about AEFs that expose the API. This information contains the following fields:

  - aefId: AEF identifier of the provider that exposes the service.

  - versions: List of information about API versions. Each version contains the resources supported by the API. Each version contains:

    - apiVersion: usable version of the API.

    - expiry: date on which the API version expires and will no longer be available.

    - resources: A list of resources that contain the API version. Each resource contains the following information:

      - resourceName: Name of the resource for which its information is specified.

      - commType: Type of use of the resource, which can be receiving a response when making a request (REQUEST_RESPONSE) or a subscription to receive notifications (SUBSCRIBE_NOTIFY)

      - uri: URI to make the request to use the described API resource.

      - custOpName: Part of the URI structure used for a custom operation associated with the resource. The attributes custOpName and custOperations are mutually exclusive.

      - custOperations: List of the resource's custom operations associated to this resource. The attributes custOpName and custOperations are mutually exclusive.

      - operations: List of HTTP methods supported by the API resource.

      - description: Descriptive text of the API resource.

    - custOperations: List of custom operations without resource association.

  - protocol: Indicates a protocol and protocol version used by the API.

  - dataFormat: Indicates the data sending format which can be JSON, XML or PROTOBUF3.

  - securityMethods: Indicates the possible security methods of the API.

  - domainName: Domain name to which the API belongs.

  - interfaceDescriptions: List of interfaces to which API requests can be made.

Only domainName or interfaceDescription can be specified.

-	shareableinfo: Indicates whether the service API and/or the service API category can be shared to the list of CAPIF provider domains.

-	serviceAPICategory: The service API category to which the service API belongs to.

-	apiProvName: API provider Name that publishes the API.

The example API Publish request to publish “3gpp-as-session-with-qos” API from NEF is shown in Table 6.4-3-OCF.

Table 6.4-3-OCF: Publish API request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/published-apis/v1/APF54f49ee44f67c81fc7d7f321195531/service-apis Request body: { "apiName": "3gpp-as-session-with-qos/", "apiStatus": { "aefIds": ["AEFb1a522967511fd6d6def0cbb3b5f05" ] }, "aefProfiles": [ { "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "versions": [ { "apiVersion": "v1", "expiry": "2100-11-30T10:32:02.004Z", "resources": [ { "resourceName": "QOS_SUBSCRIPTIONS", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions", "custOpName": "http_post", "operations": [ "GET", "POST" ], "description": "Endpoint to manage monitoring subscriptions" }, { "resourceName": "QOS_SUBSCRIPTION_SINGLE", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions/{subscriptionId}", "custOpName": "http_get", "operations": [ "GET", "PUT", "DELETE" ], "description": "Endpoint to manage single subscription" } ], "custOperations": [ { "commType": "REQUEST_RESPONSE", "custOpName": "string", "operations": [ "POST" ], "description": "string" } ] } ], "protocol": "HTTP_1_1", "dataFormat": "JSON", "securityMethods": [ "OAUTH", "PSK" ], "interfaceDescriptions": [ { "ipv4Addr": "127.0.0.1", "port": 4443, "securityMethods": [ "OAUTH" ] } ] } ], "description": "3gpp-as-session-with-qos NEF API", "supportedFeatures": "fffff", "shareableInfo": { "isShareable": true, "capifProvDoms": [ "mobilesandbox.cloud" ] }, "serviceAPICategory": "NEF", "apiProvName": "OCF_NEF" }

4. 201 Created

The service API has been published successfully The URI of the created resource is returned in the "Location" HTTP header, including the assigned service API identifier (serviceApiId) that is also equal to the API identifier (apiId) in the response body.

Table 6.4-4: Publish API response (schema)

{ 201 Response headers: Location: {apiRoot}/published-apis/v1/{apfId}/service-apis/{serviceApiId} Response body: { "apiName": "string", "apiId": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ "object" ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string" "apiProvName": "string" }

The response to a successful API Publish request using OpenCAPIF contains the following information:

-	Location: URL with the resource created for the API Published.

-	apiId: The assigned Identifier to the Published API. It is required for the UPDATE or DELETE API procedures once the API has been published.

The information from the API that was sent in the request will be obtained as a response with a new field, the "apiId", which is the identifier of the API that has been created and will enable subsequent modify or delete procedures.

Table 6.4-4-OCF: Publish API response (OCF example)

Response headers: Location: https://capif.mobilesandbox.cloud:37211/published-apis/v1/APF54f49ee44f67c81fc7d7f321195531/service-apis/f55731ec5f8ce703ac1f69605ad095 Response body: { "apiName": "3gpp-as-session-with-qos", "apiId": "f55731ec5f8ce703ac1f69605ad095", "aefProfiles": [ { "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "versions": [ { "apiVersion": "v1", "expiry": "2100-11-30T10:32:02.004Z", "resources": [ { "resourceName": "QOS_SUBSCRIPTIONS", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions", "custOpName": "http_post", "operations": [ "GET", "POST" ], "description": "Endpoint to manage monitoring subscriptions" }, { "resourceName": "QOS_SUBSCRIPTION_SINGLE", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions/{subscriptionId}", "custOpName": "http_get", "operations": [ "GET", "PUT", "DELETE" ], "description": "Endpoint to manage single subscription" } ], "custOperations": [ { "commType": "REQUEST_RESPONSE", "custOpName": "string", "operations": [ "POST" ], "description": "string" } ] } ], "protocol": "HTTP_1_1", "dataFormat": "JSON", "securityMethods": [ "OAUTH", "PSK" ], "interfaceDescriptions": [ { "ipv4Addr": "127.0.0.1", "port": 4443, "securityMethods": [ "OAUTH" ] } ] } ], "description": "3gpp-as-session-with-qos NEF API", "supportedFeatures": "fffff", "shareableInfo": { "isShareable": true, "capifProvDoms": [ "mobilesandbox.cloud" ] }, "serviceAPICategory": "NEF", "apiProvName": "OCF_NEF" }

5. CAPIF administrator creates policy

The procedures for creation of the access control policy at the CAPIF core function by the CAPIF administrator is outside the scope of 3GPP TS 23.222 [2] and can be triggered because of the API management function issuing the "Register API provider" request or prior to that. Either way, sufficient information must be provided to enable the AEF specific access control policy list resource for the service API to be created. Specifically, each policy must include the CCF assigned API invoker identifier (apiInvokerId) and can include allowed total invocations (allowedTotalInvocations), invocations per second (allowedInvocationsPerSecond), invocation time range list (allowedInvocationTimeRangeList). Since an API invoker identifier is mandatory, it is a prerequisite that API invoker has been onboarded prior to invoking the highlighted access control policy procedures.

6. CAPIF_Access_Control_Policy_API

The Access Control List (ACL) service allows the provider to use AEF to obtain the access control list from invokers for an exposing API. The provider is in charge of making the request to the CAPIF Core Function and using the list received to control an invoker's access to one of its APIs.

Table 6.4-6: GET access control policy request (schema, then example content).

Table 6.4-6: GET access control policy request (schema)

HTTP GET {apiRoot}/access-control-policy/v1/accessControlPolicyList/{serviceApiId}?aef-id="string"

The AEF may make a request to OpenCAPIF to obtain all the ACLs of a service that it is exposing, for this it must specify in the URL of the request the identifier of the service (apiId received in the API Publish response) for which it wants the list of ACLs, in addition to the AEF’s own identifier to check that it has access to that list.

Table 6.4-6-OCF: ACL Request (OCF example)

HTTP GET https://capif.mobilesandbox.cloud:37211/access-control-policy/v1/accessControlPolicyList/f55731ec5f8ce703ac1f69605ad095?aef-id=AEFb1a522967511fd6d6def0cbb3b5f05

7. 200 OK

The request has been successfully received and the CAPIF Core Function returns the invoker access control information for the requested API. The AEF will use this information to allow or not the use of the API by the invoker.

Table 6.4-7: GET access control policy response (schema)

200 Response body: { "apiInvokerPolicies": [ { "apiInvokerId": "string", "allowedTotalInvocations": "integer", "allowedInvocationsPerSecond": "integer", "allowedInvocationTimeRangeList": [ { "startTime": "string", "stopTime": "string" } ] } ] }

OpenCAPIF provides the access control policy information for the NEF API Published "3gpp-as-session-with-qos" containing the following information:

-	apiInvokerId: identifier of the invoker to which the access policy belongs.

-	allowedTotalInvocations: Total number of invocations allowed on the service API by the API invoker.

-	allowedInvocationsPerSecond: invocations per second allowed on the service API by the API invoker.

-	allowedInvocationTimeRangeList: the time ranges during which the invocations are allowed on the service API by the API invoker.

Table 6.4-7-OCF: GET access control policy response (OCF example)

200 Response body: { "apiInvokerPolicies": [ { "apiInvokerId”: "INVd4d70e16c52bace2b8f40ed6f6badS", "allowedTotalInvocations": 1200, "allowedInvocationsPerSecond": 2, "allowedInvocationTimeRangeList": [ { "startTime": "2024-08-19T09:00:00.000000+02:00", "stopTime": "2024-08-23T16:00:00.000000+02:00" } ] } ] }

8. Offboarding API Provider

The provider can request its deletion from CAPIF using the offboarding service. By using this service, all information stored in the CAPIF Core Function related to the provider will be deleted, including APIs published by the provider, security contexts of those APIs, ACLs, etc.

Table 6.4-8: DELETE API provider request (schema)

HTTP DELETE {apiRoot}/api-provider-management/v1/registrations/{registrationId}

For the process of deleting a Provider in OpenCAPIF, you will need the AMF certificate and the identifier of the provider that you want to delete. This data is obtained in the provider creation process and by simply making the corresponding request we will eliminate the provider.

Table 6.4-8-OCF: DELETE API provider request (OCF example)

HTTP DELETE https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations/3a4df5f6e30a5ee34de346004ee6bb

9. 204 No Content

The response will be a 204 No Content, which indicates that the provider has been successfully deleted along with everything related to it, APIs Published, security contexts associated with its services, ACLs of its services, etc.

6.5	Usage of CAPIF in different phases of an Application

This clause shows the practical example of CAPIF usage on real business system. This example assumes three roles as shown below:

Application developer: A developer who develops application software using CAPIF API, such as videogame, map, and so on.

Application software: A software developed by the application developer. This software plays the role of API invoker described in clause 5.2.3.

API provider: Described in clause 5.2.2.

Figure 6.5-1 shows the example of CAPIF usage in different phases of the Application. The overall procedure consists of three parts.

1.	Application software development preparation phase: Before the application developer develops the application software, the developer needs to prepare for using APIs. First, the developer searches the API(s) information, e.g. available API, API name, and API gateway URL via an API portal of a API provider. Then, the developer creates an account and gets authorization to access CAPIF core function. This procedure follows onboarding of API invoker as specified in subclause 8.1 of 3GPP TS 23.222 [2].

2.	Application software development and testing phase: The application developer performs API discovery using CAPIF and obtains the detailed service API information which enables the application developer to develop the software to use the API. Further, the application developer can test their application usage of the API (e.g. in a sandbox mode).

Editor's Note: The API discovery can occur before onboarding, further elaboration of the procedure is FFS.

3.	Application deployment and execution phase: After the development, the application software is deployed. The deployed application software (acting as API invoker) requires to access the API. The API invoker obtains the authorization from CAPIF core function to invoker the API as specified in clause 8.11 of 3GPP TS 23.222 [2]. Upon receiving the access token from CAPIF core function, the API invoker requests service API invocation, as specified in subclause 8.16 of 3GPP TS 23.222 [2]. In RNAA scenario, the API invoker obtains authorization with the CAPIF core function as specified in clause 8.31 of 3GPP TS 23.222 [2], and then requests service API invocation.

Figure 6.5-1: Usage of CAPIF in different phases of an Application

6.6	The deployment of the API Invoker as AF on the UE for RNAA

Several RNAA deployments are described in subclause 7.5 of 3GPP TS 23.222 [2], and the following is an example of a RNAA deployment in which API invoker is deployed as AF on the UE.

In this deployment scenario, UE, which functions as both the API Invoker (AF) and the resource owner function, requests resource owner consent from the resource owner function through CAPIF core function to obtain the resource (e.g., QoS).

Figure 6.6-1 illustrates integrated fundamental deployment of the API Invoker as AF on the UE for RNAA.

Figure 6.6-1: The deployment of the API Invoker as AF on the UE for RNAA

Figure 6.6-2 illustrates the procedure for the deployment of the API Invoker as AF on the UE for RNAA.

Figure 6.6-2: The deployment of the API Invoker as AF on the UE for RNAA

## Editor's Note: The API discovery can occur before onboarding, further elaboration of the procedure is FFS.

## Editor's Note: Whether the procedure should be moved to another proposal clause is FFS.

The following procedure is for the deployment of the API Invoker as AF on the UE for RNAA:

1.	The API invoker triggers onboard API invoker request towards the CAPIF core function. The CAPIF core function then verifies the request and responds to the API invoker with the result.

2.	The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API. The CAPIF core function validates the authentication of the API invoker and checks whether the API invoker is permitted to access the requested service API. Then, the CAPIF core function responds to the API invoker with the authorization information required to access the service APIs.

3.	 The API invoker sends a service API discover request to the CAPIF core function and the CAPIF core function verifies the identity of the API invoker. Then the CAPIF core function sends a service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

4.	The AEF sends an obtain access control policy request to the CAPIF core function for obtaining the policy to perform the access control on service API invocations. The CAPIF core function checks whether the AEF is authorized to receive the access control policy corresponding to the service APIs requested. The AEF is provided the access control policy for the service API via an obtain access control policy response.

5.	The API invoker requests the resource owner's authorization information to invoke the service API, exposed by the API exposing function. The authorization function facilitates this request by involving the resource owner. Then, the API invoker sends a service API invocation request to the API exposing function, which in turn sends a response back to the API invoker.

6.	The API invoker (UE) sends service API invocation request to the and the AEF checks whether the API invoker is authorized to invoke that service API, based on the authorization information. The API invoker receives the service API invocation response as a result of the service API invocation.

6.7	The deployment of the API Invoker as AF on the network for RNAA

In this deployment scenario, the UE, which functions as both the application and the resource owner function, initiates the application's requests. The API Invoker then requests resource owner consent from the resource owner function through CAPIF core function to obtain the resource (e.g., QoS).

Figure 6.7-1 illustrates integrated fundamental deployment of the API Invoker as AF on the network for RNAA.

Figure 6.7-1: The deployment of the API Invoker as AF on the network for RNAA

Figure 6.7-2 illustrates the procedure for the deployment of the API Invoker as AF on the network for RNAA.

Figure 6.7-2: The procedure for the deployment of the API Invoker as AF on the network for RNAA

Editor's Note: The API discovery can occur before onboarding, further elaboration of the procedure is FFS.

Editor's Note: Whether the procedure should be moved to another proposal clause is FFS.

The following procedure is for API Invoker deployment on the network as AF for RNAA:

1.	The API invoker triggers onboard API invoker request towards the CAPIF core function. The CAPIF core function then verifies the request and responds to the API invoker with the result.

2.	The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API. The CAPIF core function validates the authentication of the API invoker and checks whether the API invoker is permitted to access the requested service API. Then, the CAPIF core function responds to the API invoker with the authorization information required to access the service APIs.

3.	The API invoker sends a service API discover request to the CAPIF core function and the CAPIF core function verifies the identity of the API invoker. Then the CAPIF core function sends a service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

4.	The AEF sends an obtain access control policy request to the CAPIF core function for obtaining the policy to perform the access control on service API invocations. The CAPIF core function checks whether the AEF is authorized to receive the access control policy corresponding to the service APIs requested. The AEF is provided the access control policy for the service API via an obtain access control policy response.

5.	The API exposing function obtains the resource owner consent from the resource owner. The API invoker requests the resource owner's authorization information to invoke the service API, exposed by the API exposing function. The authorization function facilitates this request by involving the resource owner. Then, the API invoker sends a service API invocation request to the API exposing function, which in turn sends a response back to the API invoker.

6.	From the application’s request on UE, the API invoker sends service API invocation request to the AEF and the AEF checks whether the API invoker is authorized to invoke that service API, based on the authorization information. The API invoker receives the service API invocation response as a result of the service API invocation.

6.8	API consumption from an API Invoker

This section explains how an API Invoker can use OpenCAPIF [4] to discover and consume APIs, based on 3GPP TS 23.222 [2] and 3GPP TS 29.222 [3] for the schema. The API Invoker application needs to obtain the following information from OpenCAPIF during user registration, similarly to the API Provider onboarding:

-	CA_ROOT: Root certificate from the OpenCAPIF Certificate Authority to validate Certificates.

-	Token: Token for the API Invoker onboarding request.

-	Onboarding URL: URL for requesting the API Invoker onboarding.

This section explains:

-	how to onboard an API Invoker in OpenCAPIF, to be able to discover all the APIs published in the CAPIF Core Function.

-	request the creation of a security context, which is necessary to consume an API.

-	request the required OAuth token to consume the API; and finally, how to make the request directly to the published API.

All the requests shown in this document can be tested using the OpenCAPIF Postman collection found in Annex A and in the OpenCAPIF Postman documentation [14].

- CAPIF_API_Invoker_Management (Registration)

Before carrying out onboarding, the invoker will need the CAPIF access token that will allow the creation of the invoker, the CA_ROOT certificate necessary to validate all the requests, and the URL to which to make the request <{apiRoot}/api-invoker-management/v1/onboardedInvokers>.

In OpenCAPIF, all this information is obtained in the user registration process [13].

To onboard an API Invoker in CAPIF, it is necessary to utilize the token received in the Authentication HTTP header. In addition, a Certificate Signing Request (CSR) will have to be created with two keys so that the CAPIF Core Function can issue a valid certificate that will be used by the invoker to be able to make the subsequent requests. In the following example, the {apiList} field has been omitted, as it will discover the APIs that can be used with the discover service.

Table 6.8-1: Register API Invoker request (schema)

HTTP POST {apiRoot}/api-invoker-management/v1/onboardedInvokers Request headers: Authorization: "string" Request body: { "onboardingInformation": { "apiInvokerPublicKey": "string" }, "notificationDestination": "string", "apiInvokerInformation": "string", "websockNotifConfig": { "websocketUri": "string", "requestWebsocketURI": "boolean" } }

To perform the onboarding of an API Invoker, the following data will have to be specified:

-	apiInvokerPublicKey: It is the certificate signing request of the invoker that CAPIF will use to issue the invoker’s valid certificate.

-	apiInvokerInformation: Generic information related to the API invoker such as details of the device or the application

-	notificationDestination: URL where the API invoker can receive CAPIF notifications.

Table 6.8-1-OCF: Register API Invoker request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/api-invoker-management/v1/onboardedInvokers/ Request headers: Authorization: Bearer eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9 Content-Type: application/json Request body: { "onboardingInformation": { "apiInvokerPublicKey": "-----BEGIN CERTIFICATE REQUEST----- \nMIICeTCCAWECAQAHISNoGA1ERGAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST----\n" }, "notificationDestination": " http://nef_notification_destination:8080/", "apiInvokerInformation": "NEF Invoker", "websockNotifConfig": { "websocketUri": "websocketUri", "requestWebsocketURI": true } }

The CAPIF Core Function assigns an identity for the API invoker (apiInvokerId) and a certificate (apiInvokerCertificate) based on provided public key(apiInvokerPublicKey).

2. 201 Created

The API Invoker has been registered successfully. The URI of the created resource is returned in the “Location” HTTP header including the assigned registration identifier (apiInvokerId).

Table 6.8-2: Register API-Invoker response (schema, then example content)

201 Response headers: Location: {apiRoot}/api-invoker-management/v1/onboardedInvokers/{apiInvokerId} Response body: { "apiInvokerId": "string", "onboardingInformation": { "apiInvokerPublicKey": "string", "apiInvokerCertificate": "string", "onboardingSecret": "string" }, "notificationDestination": "string", "apiInvokerInformation": "string", "websockNotifConfig": { "websocketUri": "string", "requestWebsocketURI": "boolean" } }

The fields that are returned when onboarding an invoker, that are populated by the CCF, are:

-	apiInvokerId: Identifier of the invoker created.

-	apiInvokerCertificate: Certificate of the Invoker created from the CSR sent in the onboarding request, which will be used for the following requests from the Invoker to CAPIF Core Function.

Table 6.8-2-OCF: Register API-Invoker response (OCF example)

Response headers: Location: https://capif.mobilesandbox.cloud:37211/api-invoker-management/v1/onboardedInvokers/INVaa03f9911db37be051b243ce3caad9 Response body: { "apiInvokerId": "INVaa03f9911db37be051b243ce3caad9", "onboardingInformation": { "apiInvokerPublicKey": "-----BEGIN CERTIFICATE REQUEST----- \nMIICeTCCAWECAQAHISNoGA1ERGAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST----\n", "apiInvokerCertificate": "-----BEGIN CERTIFICATE-----\nMIIDgjCCAmqgAwIBAgIUS18nvjZYU5B+ptuAw/ x3gQWYEJVxDha9112mP7lclNg5yd5o1Xwj1Imbht2z33ZJ4jxA=\n-----END CERTIFICATE-----\n" }, "notificationDestination": "http://nef_notification_destination:8080/", "apiInvokerInformation": "NEF Invoker", "websockNotifConfig": { "websocketUri": "websocketUri", "requestWebsocketURI": true } }

3. CAPIF_Discovery_Service_API

After onboarding the API Invoker, the API Discovery function is used by invokers to obtain all the APIs published in CAPIF, providing all the necessary information that an API Invoker needs to be able to make a request to each such API.

Table 6.8-3: Discover APIs request (schema, then example content)

HTTP GET {apiRoot}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId}

The API Invoker must make the request to CAPIF using its identifier {apiInvokerId} in order to verify that the invoker is registered in CAPIF.

Table 6.8-3-OCF: Discover APIs request (OCF example)

HTTP GET https://capif.mobilesandbox.cloud:37211/service-apis/v1/allServiceAPIs?api-invoker-id=INVaa03f9911db37be051b243ce3caad9

4. OK 200

Upon successful request, the API Invoker will receive the 200 OK code and all the information about the APIs published in the CAPIF Core Function containing the AEF profiles exposing the APIs. Using this information, the API Invoker can decide which APIs it wants to use.

Table 6.8-4: Discover APIs response (schema)

200 Response body: { "serviceAPIDescription":[ { "apiName": "string", "apiId": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "string", "expiry": "string", "resources": [ { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" }, { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" } ], "custOperations": [ { "commType": "string", "custOpName": "string", "operations": [ "string" ], "description": "string" } ] } ], "protocol": "string", "dataFormat": "string", "securityMethods": [ "string", ], "interfaceDescriptions": [ { "ipv4Addr": "string", "port": "int", "securityMethods": [ "string" ] } ] } ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string" "apiProvName": "string" } ] }

The API Invoker will receive a list with the APIs published in CAPIF, including the following information:

-	apiName: Name of the API to be published, which must be unique in CAPIF.

-	apiStatus: Represents the API status, in addition, if this attribute is omitted, it is understood that the API is available in all AEFs of aefProfiles. It contains the aefIds field which is a list of the AEFs that expose the API. If this list is empty or the attribute is omitted, it is understood that the API is inactive in all aefProfiles.

-	aefProfiles: List of Information about AEFs that expose the API. This information contains the following fields:

  - aefId: AEF identifier of the provider that exposes the service.

  - versions: list of information about API versions. Each version contains the resources supported by the API. Each version contains:

    - apiVersion: usable version of the API.

    - expiry: date on which the API version expires and will no longer be available.

    - resources: A list of resources that contain the API version. Each resource contains the following information:

      - resourceName: Name of the resource for which its information is specified.

      - commType: Type of use of the resource, which can be receiving a response when making a request (REQUEST_RESPONSE) or a subscription to receive notifications (SUBSCRIBE_NOTIFY)

      - uri: URI to make the request to use the described API resource.

      - custOpName: Part of the URI structure used for a custom operation associated with the resource. The attributes custOpName and custOperations are mutually exclusive.

      - custOperations: List of the resource's custom operations associated to this resource. The attributes custOpName and custOperations are mutually exclusive.

      - operations: list of HTTP methods supported by the API resource.

      - description: descriptive text of the API resource.

    - custOperations: List of custom operations without resource association.

  - protocol: indicates a protocol and protocol version used by the API.

  - dataFormat: Indicates the data sending format which can be JSON, XML or PROTOBUF3.

  - securityMethods: Indicates the possible security methods of the API.

  - domainName: Domain name to which the API belongs.

  - interfaceDescriptions: List of interfaces to which API requests can be made.

Only domainName or interfaceDescription can be specified.

-	shareableinfo: Indicates whether the service API and/or the service API category can be shared to the list of CAPIF provider domains.

-	serviceAPICategory: The service API category to which the service API belongs to.

-	apiProvName: API provider Name that publishes the API.

Table 6.8-4: Discover APIs response (OCF example)

200 Response body: { "serviceAPIDescription":[ { "apiName": "3gpp-as-session-with-qos", "apiId": "f55731ec5f8ce703ac1f69605ad095", "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "v1", "expiry": "2100-11-30T10:32:02.004Z", "resources": [ { "resourceName": "QOS_SUBSCRIPTIONS", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions", "custOpName": "http_post", "operations": [ "GET", "POST" ], "description": "Endpoint to manage monitoring subscriptions" }, { "resourceName": "QOS_SUBSCRIPTION_SINGLE", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions/{subscriptionId}", "custOpName": "http_get", "operations": [ "GET", "PUT", "DELETE" ], "description": "Endpoint to manage single subscription" } ], "custOperations": [ { "commType": "REQUEST_RESPONSE", "custOpName": "string", "operations": [ "POST" ], "description": "string" } ] } ], "protocol": "HTTP_1_1", "dataFormat": "JSON", "securityMethods": [ "OAUTH", "PSK" ], "interfaceDescriptions": [ { "ipv4Addr": "127.0.0.1", "port": 4443, "securityMethods": [ "OAUTH" ] } ] } ], "description": "3gpp-as-session-with-qos NEF API", "supportedFeatures": "fffff", "shareableInfo": { "isShareable": true, "capifProvDoms": [ "mobilesandbox.cloud" ] }, "serviceAPICategory": "NEF", "apiProvName": "OCF_NEF" } ] }

5. CAPIF_Security_API

This request is responsible for requesting the creation of a security context to be able to use an API. If this step were not carried out, even if the API invoker had the information on an API it wished to use, the token giving access to the API would be missing, which can only be obtained once the security context has been created.

Table 6.8-5: Create Security Context request (schema)

HTTP PUT {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId} Request body: { "securityInfo": [ { "prefSecurityMethods": [ "string" ], "aefId": "string", "apiId": "string", "authenticationInfo": "string", "authorizationInfo": "string" } ], "notificationDestination": "string", "supportedFeatures": "string" }

In this request we find the following fields:

-	securityInfo: List of information that represents the details of the interfaces and security methods of the APIs the API invoker wishes to use.

-	prefSecurityMethods: List of preferred security methods for using an API. In the examples, OAuth tokens are used.

-	aefId: Identifier of the API exposing function that expose the API.

-	apiId: Identifier of the API that that the API invoker wishes to use.

-	notificationDestination: Address where the notifications should be delivered to.

The following example asks for OAUTH as the preferred security method. When this method is selected by the CAPIF Core Function, the API invoker must make another request to obtain the necessary security access token to consume the API.

Table 6.8-5-OCF: Create Security Context request (OCF example)

HTTP PUT https://capif.mobilesandbox.cloud:37211/capif-security/v1/trustedInvokers/INVaa03f9911db37be051b243ce3caad9 Request body: { "securityInfo": [ { "prefSecurityMethods": ["OAUTH"], "selSecurityMethod": "OAUTH", "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "apiId": "f55731ec5f8ce703ac1f69605ad095", "authenticationInfo": "authenticationinfo", "authorizationInfo": "authorizationInfo" } ], "notificationDestination": "http://nef_notification_destination:8080/", "supportedFeatures": "fffff" }

OpenCAPIF selects one of the preferred methods to make requests to the API based in AEF profile. In this example, only one Security Method is requested by the API Invoker, so OpenCAPIF checks that this Security Method is supported by the API Exposer.

6. 201 Created

The security context has been created successfully and the security context created with the selected security method is returned in the response body.

Table 6.8-6: Create Security Context response (schema)

201 Response headers: Location: {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId} Response body: { "securityInfo": [ { "prefSecurityMethods": [ "string" ], "selSecurityMethod": "string", "aefId": "string", "apiId": "string", "authenticationInfo": "string", "authorizationInfo": "string" } ], "notificationDestination": "string", "supportedFeatures": "string" }

The {selSecurityMethod} is returned, which specifies the security method that will be used so that the API Invoker can consume the API and the URI of the created resource in the “Location” HTTP header that includes the {apiInvokerId}.

Table 6.8-6-OCF: Create Security Context response (OCF example)

201 Response headers: Location: https://capif.mobilesandbox.cloud:3721/capif-security/v1/trustedInvokers/INVaa03f9911db37be051b243ce3caad9 Response body: { "securityInfo": [ { "prefSecurityMethods": ["OAUTH"], "selSecurityMethod": "OAUTH", "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "apiId": "f55731ec5f8ce703ac1f69605ad095", "authenticationInfo": "authenticationinfo", "authorizationInfo": "authorizationInfo" } ], "notificationDestination": "http://nef_notification_destination:8080/", "supportedFeatures": "fffff" }

7. CAPIF_Security_API (Obtain Token)

When the security method selected is OAUTH in the security context created by the API Invoker, the API Invoker must make a request to the CAPIF Core Function to obtain the access token to consume the API.

Table 6.8-7: Obtain Access Token request (schema, then example content)

HTTP POST {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId}/token Request body: { "grantType": "string", "clientId": "string", "scope": "string" }

The fields necessary to request the access token are:

-	grantType: It is used to specify whether the token request is made by the API Invoker themselves (client_credentials) or for the use of RNAA (authorization_code)

-	clientId: Identifier of the invoker that is requesting the token.

-	scope: a String that contains a list of AEF identifiers and its associated API names for which the access token is authorized for use. It takes the format of: 3gpp#aefId1:apiName1,apiName2;aefId2:apiName1,apiName2

The request to obtain the OAuth token in OpenCAPIF is displayed in the following table:

Table 6.8-7-OCF: Obtain Access Token request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/capif-security/v1/trustedInvokers/INVaa03f9911db37be051b243ce3caad9/token Request body: { "grantType": "client_credentials", "clientId": "INVaa03f9911db37be051b243ce3caad9", "scope": "3gpp#AEFb1a522967511fd6d6def0cbb3b5f05:/nef/api/v1/3gpp-as-session-with-qos/" }

8. 200 OK

In response, the API Invoker will obtain the token that is needed to consume the API that was specified in the request.

Table 6.8-8: Obtain Access Token response (schema)

200 Response body: { "access_token": "string", "token_type": "string", "expires_in": "integer", "scope": "string" }

The fields returned in the response are:

-	access_token: OAuth access token necessary to make the request to the selected API.

-	token_type: Contains the type of token that is returned (i.e. “Bearer”).

-	expires_in: Contains the number of seconds after which the access_token expires.

-	scope: It is the scope requested in the request and that uses the token to enable the use of an API. It takes the following format: 3gpp#aefId1:apiName1,apiName2;aefId2:apiName1,apiName2

Table 6.8-8-OCF: Obtain Access Token response (OCF example)

200 Response body: { "access_token": "eyJhbGciOiJSUzI1NiIsInR5cCI6IkpXVCJ9", "token_type": "Bearer", "expires_in": 600, "scope": "3gpp#AEFb1a522967511fd6d6def0cbb3b5f05:/nef/api/3gpp-as-session-with-qos/v1" }

9. CAPIF_API_Invocation

The API invoker’s request to the consuming API does not go through CAPIF, rather it is made directly to the API Exposer. For this request, the data obtained from AEF Profile in the Discover request is used (IP/hostname, port, endpoint, method, etc...), it is also necessary to use the access token obtained from CAPIF that will be in the header of the request.

As an example, the request is to the API that has been published as a NEF provider and the OAuth token that has been obtained as an API Invoker.

Table 6.8-9: Invoke API request (schema)

HTTP {operation} {ipv4Addr}:{port}{uri} Request headers: Authorization: "string" Content-Type: "string" Request body: { API BODY }

To make the request to the API, the access token obtained from the CAPIF Core Function must be entered in the Authorization header, from there, everything depends on the description of the published API.

Each description contains either the ipv4Addr or the domainName, both along with the port to which the request can be made. Also, depending on the resource to be called, the appropriate operation and uri specific to that resource. This is in addition to having to pass data and use the format according to the dataFormat field and also specifying this format in the header.

The following example shows how an invoker would use the NEF API that has been published:

Table 6.8-9-OCF: Invoke API request (OCF example)

HTTP POST 127.0.0.1:4443/nef/api/3gpp-as-session-with-qos/v1/{scsAsId}/subscriptions Request headers: Authorization: Bearer eyJhbGciOiJSUzI1NiIsInR5cCI6IkpXVCJ9 Content-Type: "application/json" Request body: { "ipv4Addr": "10.0.0.3", "notificationDestination": "http://localhost:80/api/v1/utils/session-with-qos/callback", "snssai": { "sst": 1, "sd": "000001" }, "dnn": "province1.mnc01.mcc202.gprs", "qosReference": 9, "altQoSReferences": [0], "usageThreshold": { "duration": 0, "totalVolume": 0, "downlinkVolume": 0, "uplinkVolume": 0 }, "qosMonInfo": { "reqQosMonParams": ["DOWNLINK"], "repFreqs": ["EVENT_TRIGGERED"], "latThreshDl": 0, "latThreshUl": 0, "latThreshRp": 0, "waitTime": 0, "repPeriod": 0 } }

This would make a request to the NEF API that was used as an example. In this case, the API Invoker would receive the code 201 Created as it is used to create a subscription to the AsSessionWithQoS.

10. CAPIF_API_Invoker_Management (Deregistration)

The invoker can request its deletion from CAPIF using the offboarding service. By using this service, all information stored in the CAPIF Core Function related to the invoker will be deleted, including all the security contexts created for the APIs, ACLs, events subscriptions, etc.

Table 6.8-10: DELETE API Invoker request (schema, then example content)

HTTP DELETE {apiRoot}/api-invoker-management/v1/onboardedInvokers/{apiInvokerId}

For the process of deleting an invoker in CAPIF, the certificate and identifier of the invoker to be deleted is required. That information is obtained through the invoker creation process. Then by making the following request, the invoker is eliminated.

Table 6.8-10-OCF: DELETE API Invoker request (OCF example)

HTTP DELETE https://capif.mobilesandbox.cloud:37211/api-invoker-management/v1/onboardedInvokers/INVaa03f9911db37be051b243ce3caad9

11. 204 No Content

The successful response will be a 204 No Content, which indicates that the invoker has been successfully deleted along with everything related to it: security context of the APIs; ACLs of the services; events subscriptions etc.

7	Summary

This guide presents a number of use cases and adoption strategies for CAPIF. Several stakeholders (e.g., developers, aggregators, CSPs, etc.) can compare them with their own use cases to understand how to apply/map CAPIF capabilities to their own needs.

Annex A:
Examples from OpenCAPIF

All the requests shown in this document for OpenCAPIF can be tested using the OpenCAPIF Postman collection [14]. The collection includes a guide on its use and how to correctly test each of the requests. The following table shows the mapping between each of the requests presented in this document to the corresponding request in the OpenCAPIF Postman collection.

| Table A-1: The OpenCAPIF Postman collectionOpenCAPIF Postman collection |  |  |
| --- | --- | --- |
| Request | Section | OCF Postman Example |
| CAPIF_API_Provider_Management_API | 6.4.1 | https://ocf.etsi.org/documentation/develop/testing/postman/#04-onboard_provider |
| CAPIF_Publish_Service_API | 6.4.3 | https://ocf.etsi.org/documentation/develop/testing/postman/#05-publish_api |
| CAPIF_API_Invoker_Management_API | 6.8.1 | https://ocf.etsi.org/documentation/develop/testing/postman/#07-onboard_invoker |
| CAPIF_Discover_Service_API | 6.8.3 | https://ocf.etsi.org/documentation/develop/testing/postman/#08-discover |
| CAPIF_Security_API create context | 6.8.5 | https://ocf.etsi.org/documentation/develop/testing/postman/#09-security_context |
| CAPIF_Security_API authorization info | 6.8.7 | https://ocf.etsi.org/documentation/develop/testing/postman/#10-get_token |
| API Invocation | 6.8.9 | https://ocf.etsi.org/documentation/develop/testing/postman/#11-call_service |

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-10 | SA6#57 |  |  |  |  | TR Initial Version as per S6-233407 | 0.0.0 |
| 2023-10 | SA6#57 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-233386 | 0.1.0 |
| 2023-11 | SA6#58 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-233904, S6-233908, S6-233909, S6-233910. | 0.2.0 |
| 2024-03 | SA6#59 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-240528, S6-240529, S6-240531, S6-240533, S6-240582. | 0.3.0 |
| 2024-04 | SA6#60 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-241358, S6-241351. | 0.4.0 |
| 2024-05 | SA6#61 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-242323, S6-242703, S6-242704, S6-242705. | 0.5.0 |
| 2024-08 | SA6#62 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-243646, S6-243061, S6-243062, S6-243724, S6-243518, S6-243628, S6-243629, S6-243067, S6-243519, S6-243069, S6-243631, S6-243265, S6-243510, S6-243633. | 0.6.0 |