| 3GPP TR 23.958 V0.4.0 (2023-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Edge Application Standards in 3GPP and Alignment with External Organizations; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Related work in other SDOs	7

4.1	General	7

4.2	GSMA OPG	7

4.3	ETSI ISG MEC	7

5	Alignment of EDGEAPP with ETSI MEC	8

5.1	General	8

5.2 	EDGE-3 and Mp1 reference points	8

5.2.1 	EASProfile and AppInfo	8

5.2.2 	Procedures for Application Registration	8

5.3 	EDGE-9 and Mp3 reference points	8

6	Alignment of EDGEAPP with GSMA OP	9

7	Deployment Considerations	9

7.1	Deployment Architecture-1	9

7.2	Deployment Architecture-2	9

8	Recommendations	9

Annex A (informative): Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This TR provides recommendations on alignment and deployment aspects of EDGEAPP with ETSI MEC and GSMA Operator Platform (OP).

# 1	Scope

This TR provides the recommendations for the alignment of EDGEAPP architecture with ETSI MEC reference architecture and GSMA Operator Platform (OP) for the industry stakeholders.

This document provides recommendations of using specifications from 3GPP, ETSI ISG MEC and GSMA OPG to deploy an aligned edge system.

The present document is based on 3GPP TS 23.558 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[3]		GSMA PRD OPG.02 - “Operator Platform Telco Edge Requirements Version 3.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2022/10/Operator-Platform-Telco-Edge-Requirements.-v.3-October22.pdf.

[4]	ETSI GS MEC 003 (V3.1.1), "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[5]	ETSI GS MEC 011 (V3.1.1), “Multi-access Edge Computing (MEC); Edge Platform Application Enablement”.

[6]	ETSI GS MEC 040 (V3.1.1), “Multi-access Edge Computing (MEC); Federation enablement APIs”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Related work in other SDOs

This section includes the existing/on-going work in GSMA/ETSI MEC which is relevant to the alignment work with EDGEAPP e.g., about OP roles, existing mapping/alignment aspects captured by GSMA OP/ETSI MEC,

## 4.1	General

## 4.2	GSMA OPG

The Operator Platform (OP) as defined by GSMA OPG in [3], provides technical requirements, functional blocks and interfaces characteristics of a generic platform to facilitate access to the Edge Cloud capability of an Operator or federation of operators and their partners. It also provides mapping of these requirements and architectures to the specifications from certain SDOs.

In Figure 2 in [3], GSMA OPG presents different OP roles and interfaces reference architecture for an operator platform.

Editor's Note: Figure 2 in [3] can be inserted here once the copyright is obtained from GSMA.

## 4.3	ETSI ISG MEC

Figure 4.3-1 shows the MEC reference architecture as specified by ETSI ISG MEC. This reference architecture describes the functional elements that comprise the multi-access edge system and the reference points between them. It consists of MEC host and MEC management system necessary to run MEC Applications within an operator network or a subset of an operator network.

The MEC platform is the collection of essential functionalities required to run MEC applications on a particular
Virtualisation infrastructure and enable them to provide and consume MEC services.

MEC applications are instantiated on the Virtualisation infrastructure of the MEC host based on configuration or
requests validated by the MEC management. An already instantiated MEC application can optionally register with MEC platform.  The application registration procedure allows an authorized MEC application instance to provide its information to the MEC platform [5].

The Mp1 reference point between the MEC platform and the MEC applications provides service registration, service discovery, and communication support for services. It also provides other functionality such as application availability, session state relocation support procedures, traffic rules and DNS rules activation, access to persistent storage and time of day information, etc.

The Mp3 reference point between MEC platforms is used for control communication between MEC platforms.

ETSI ISG MEC also provides Multi-access system reference architecture variants for the deployment in an NFV environment and for MEC federation [4].  In [6], ETSI ISG MEC specifies “Federation enablement APIs” that enable the shared usage of MEC services and applications across different systems (e.g., MEC system, Cloud system).

Figure 4.3-1: Multi-access Edge System reference architecture [4]

# 5	Alignment of EDGEAPP with ETSI MEC

This section catptures the alignment of EDGEAPP with ETSI MEC as per TS 23.558 which focuss on requirements and mapping of EDGEAPP information elements, APIs, functional blocks etc. with ETSI MEC which are required to support deployment of aligned architectures in an operator network.

## 5.1	General

TBD

## 5.2 	EDGE-3 and Mp1 reference points

### 5.2.1 	EASProfile and AppInfo

TBD

### 5.2.2 	Procedures for Application Registration

TBD

## 5.3 	EDGE-9 and Mp3 reference points

TBD

# 6	Alignment of EDGEAPP with GSMA OP

This section captures the alignment of EDGEAPP with GSMA OP as per TS 23.558 which mainly provides any mapping guidance and interpretations of different EDGEAPP schemas, IEs, APIs etc. with GSMA requirements e.g. mapping of data models, application profiles etc.

# 7	Deployment Considerations

This section captures different deployment possibilities of aligned EDGEAPP and ETSI MEC in an operator network and any other deployment considerations w.r.t EDGEAPP, ETSI MEC and GSMA OP as per TS 23.558.

## 7.1	Deployment Architecture-1

These sections capture deployment of EDGEAPP and ETSI MEC architecture as an aligned edge systemin an operator network, their architectural requirements etc. as per TS 23.558.

## 7.2	Deployment Architecture-2

# 8	Recommendations

This section may provide recommendations or best practices for usage of the aligned EDGEAPP and ETSI MEC architectures as specified in TS 23.558 by SA6 to be followed w.r.t deployment, usage of IEs, and APIs etc.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | SA6#50-e |  |  |  |  | TR Initial Version | 0.0.0 |
| 2022-09 | SA6#50-e |  |  |  |  | TR Skeleton (as per TR template) | 0.0.1 |
| 2022-09 | SA6#50-e |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-222591, S6-222592 | 0.1.0 |
| 2022-10 | SA6#51-e |  |  |  |  | Added Introduction through pCR: S6-222662 | 0.2.0 |
| 2023-03 | SA6#53 |  |  |  |  | Added background and updated index through pCR: S6-231043 and S6-231078 | 0.3.0 |
| 2023-03 | SA6#53 |  |  |  |  | Corrected the mistake in implementing S6-231043 | 0.3.1 |
| 2023-04 | SA6#54-e |  |  |  |  | Implemented pCR: S6-231508 | 0.4.0 |