| 3GPP TS 24.546 V17.1.0 (2021-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Configuration management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPPOrganizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPPonly. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms and abbreviations	9

3.1	Terms	9

3.2	Abbreviations	9

4	General description	9

5	Functional entities	10

5.1	SEAL configuration management client (SCM-C)	10

5.2	SEAL configuration management server (SCM-S)	10

6	Configuration management procedures	11

6.1	General	11

6.2	On-network procedures	11

6.2.1	General	11

6.2.1.1	Authenticated identity in HTTP request	11

6.2.1.2	Authenticated identity in CoAP request	11

6.2.2	Common procedures	11

6.2.2.1	Management of configuration update event subscription	11

6.2.2.1.1	SIP based procedures	11

6.2.2.1.2	HTTP based procedures	13

6.2.2.1.3	CoAP based procedures	14

6.2.2.1.3.1	General	14

6.2.2.1.3.2	Create a subscription	15

6.2.2.1.3.3	Delete a subscription	15

6.2.2.2	Notifications	15

6.2.2.2.1	SIP based procedures	15

6.2.2.2.2	HTTP based procedures	15

6.2.2.2.3	CoAP based procedures	16

6.2.2.2.3.1	Client procedure	16

6.2.2.2.3.2	Server procedure	16

6.2.3	VAL UE configuration data	16

6.2.3.1	Client procedure	16

6.2.3.2	Server procedure	17

6.2.4	VAL user profile data	17

6.2.4.1	SCM client HTTP procedure	17

6.2.4.2	SCM server HTTP procedure	17

6.2.4.3	SCM client CoAP procedure	17

6.2.4.4	SCM server CoAP procedure	18

6.2.5	Update VAL user profile data	18

6.2.5.1	SCM client HHTP procedure	18

6.2.5.2	SCM server HTTP procedure	19

6.2.5.3	SCM client CoAP procedure	19

6.2.5.4	SCM server CoAP procedure	20

c)	shall replace the user profile documents pointed at by the CoAP URI with the "ProfileDoc" received in the request.6.3 Off-network procedures	20

7	Coding	20

7.1	VAL user profile document	20

7.1.1	General	20

7.1.2	Application unique ID	20

7.1.3	Data structure	20

7.1.4	XML Schema	20

7.1.5	Semantics	21

7.1.6	MIME type	22

7.1.7	IANA registration template	22

7.2	VAL UE configuration document	23

7.2.1	General	23

7.2.2	Application unique ID	23

7.2.3	Data structure	23

7.2.4	XML schema	24

7.2.5	Semantics	26

7.2.6	MIME type	26

7.2.7	IANA registration template	26

Annex A (normative): Parameters for different operations	29

A.1	Creating configuration update event subscription	29

A.1.1	General	29

A.1.2	Client side parameters	29

A.1.3	Server side parameters	29

A.2	Retrieve VAL UE configuration data	30

A.2.1	Client side parameters	30

Annex B (normative): Parameters for notifications	30

B.1	General	30

B.2	Configuration update notification	30

Annex C (normative): CoAP resource representation and encoding	30

C.1	General	30

C.1.1	Resource URI structure	30

C.1.2	Use of cache	31

C.1.3	Error handling	31

C.2	Resource representation and APIs for VAL user profile	33

C.2.1	SU_UserProfile API	33

C.2.1.1	API URI	33

C.2.1.2	Resources	33

C.2.1.2.1	Overview	33

C.2.1.2.2	Resource: User Profiles	34

C.2.1.2.2.1	Description	34

C.2.1.2.2.2	Resource Definition	34

C.2.1.2.2.3	Resource Standard Methods	34

C.2.1.2.2.3.1	GET	34

C.2.1.2.2.3.2	POST	35

C.2.1.2.3	Resource: Individual User Profile	35

C.2.1.2.3.1	Description	35

C.2.1.2.3.2	Resource Definition	35

C.2.1.2.3.3	Resource Standard Methods	36

C.2.1.2.3.3.1	GET	36

C.2.1.2.3.3.2	PUT	36

C.2.1.2.3.3.3	DELETE	37

C.2.1.4	Data Model	37

C.2.1.4.1	General	37

C.2.1.4.2	Structured data types	38

C.2.1.4.2.1	Type: ProfileDoc	38

C.2.1.4.2.2	Type: ProfileInfo	38

C.2.1.4.2.3	Type: ProfileConfig	38

C.2.1.4.2.4	Type: ValTargetUe	38

C.2.1.4.3	Simple data types and enumerations	39

C.2.1.4.3.1	Enumeration: ProfileConfigType	39

C.2.1.5	Error Handling	39

C.2.1.6	CDDL Specification	39

C.2.1.6.1	Introduction	39

C.2.1.6.2	CDDL document	39

C.2.1.7	Media Type	40

Annex C (informative): Change history	41

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol aspects for the configuration management capability of SEAL to support vertical applications (e.g. V2X) over the 3GPP system.

The present document is applicable to the User Equipment (UE) supporting the configuration management client functionality as described in 3GPP TS 23.434 [2], to the application server supporting the configuration management server functionality as described in 3GPP TS 23.434 [2] and to the application server supporting the vertical application server (VAL server) functionality as defined in specific vertical application service (VAL service) specification.

NOTE:	The specification of the VAL server for a specific VAL service is out of scope for present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows;".

[3]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[4]	OMA OMA-TS-XDM_Core-V2_1-20120403-A: "XML Document Management (XDM) Specification".

[5]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification;".

[6]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[7]	IETF RFC 7159: "The JavaScript Object Notation (JSON) Data Interchange Format".

[8]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[9]	IETF RFC 5875: "An Extensible Markup Language (XML) Configuration Access Protocol (XCAP) Diff Event Package".

[10]	IETF RFC 6050 (November 2010): "A Session Initiation Protocol (SIP) Extension for the Identification of Services".

[11]	IETF RFC 6665 (July 2012): "SIP-Specific Event Notification".

[12]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[13]	IETF RFC 7959: "Block-Wise Transfers in the Constrained Application Protocol (CoAP) ".

[14]	IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

[15]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[16]	IETF RFC 8516: ""Too Many Requests" Response Code for the Constrained Application Protocol".

[17]	IETF RFC 8949: “Concise Binary Object Representation (CBOR)”.

[18]	IETF RFC 8610: "Concise Data Definition Language (CDDL): A Notational Convention to Express Concise Binary Object Representation (CBOR) and JSON Data Structures".

[19]	Constrained RESTful Environments (CoRE) Parameters at IANA, https://www.iana.org/assignments/core-parameters/core-parameters.xhtml

[20]	Internet draft draft-ietf-core-problem-details-01: "Problem Details For CoAP APIs".

[21]	Internet draft draft-ietf-core-new-block-14: "Constrained Application Protocol (CoAP) Block-

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

SEAL configuration management client: An entity that provides the client side functionalities corresponding to the SEAL configuration management service.

SEAL configuration management server: An entity that provides the server side functionalities corresponding to the SEAL configuration management service.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.434 [2] apply:

SEAL client

SEAL server

SEAL service

VAL server

VAL service

VAL user

Vertical

Vertical application

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MIME	Multipurpose Internet Mail Extensions

SCM-C	SEAL Configuration Management Client

SCM-S	SEAL Configuration Management Server

SEAL	Service Enabler Architecture Layer for verticals

# 4	General description

Configuration management is a SEAL service that provides the configuration management related capabilities to one or more vertical applications. The present document enables a SEAL configuration management client (SCM-C) and a VAL server to manage configuration data in a SEAL configuration management server (SCM-S).

# 5	Functional entities

## 5.1	SEAL configuration management client (SCM-C)

The SCM-C functional entity acts as the application client for configuration related transactions.

To be compliant with the HTTP procedures in the present document the SCM-C:

-	shall support the role of XCAP client as specified in IETF RFC 4825 [3];

-	shall support the role of XDMC as specified in OMA OMA-TS-XDM_Core-V2_1 [4];

-	shall support the procedures in clause 6.2.2;

-	shall support the procedures in clause 6.2.3; and

-	shall support the procedures in clause 6.2.4.

To be compliant with the CoAP procedures in the present document the SCM-C:

-	shall support the role of CoAP client as specified in IETF RFC 7252 [12];

-	shall support the capability to observe resources as specified in IETF RFC 7641 [14];

-	shall support the block-wise transfer as specified in IETF RFC 7959 [13];

-	may support the robust block transfer as specified in IETF draft draft-ietf-core-new-block-14 [21];

-	should support CoAP over TCP and Websocket as specified in IETF RFC 8323 [15];

-	shall support CBOR encoding as specified in IETF RFC 8949 [17];

-	shall support the procedures in clause 6.2.2;

-	shall support the procedures in clause 6.2.3; and

-	shall support the procedures in clause 6.2.4.

NOTE 1:	The security mechanism to be supported for the CoAP procedures is described in 3GPP TS 24.547 [5].

NOTE 2:	Support for TCP for the CoAP procedures is required if the client connects over the network which blocks or impedes the use of UDP, e.g. when NATs are present in the communication path.

NOTE 3:	The CoAP protocol supports mechanism for reliable message exchange over UDP. Use of TCP can also be beneficial if reliable transport is required for other reasons, e.g. better observability of resources. Usage of CoAP over TCP is an implementation choice.

NOTE 4:	Support for the robust block transfer mechanism for the CoAP procedures is beneficial in environments where packet loss is highly asymmetrical and where performance optimization of block transfers is required.

## 5.2	SEAL configuration management server (SCM-S)

The SCM-S is a functional entity used to configure one or more vertical applications with 3GPP system related vertical applications provisioning information and configure data on the SEAL configuration management client.

To be compliant with the HTTP procedures in the present document the SCM-S:

-	shall support the role of XCAP server as specified in IETF RFC 4825 [3];

-	shall support the role of XDMS as specified in OMA OMA-TS-XDM_Core-V2_1 [4];

-	shall support the procedures in clause 6.2.2;

-	shall support the procedures in clause 6.2.3; and

-	shall support the procedures in clause 6.2.4.

To be compliant with the CoAP procedures in the present document the SCM-C:

-	shall support the role of CoAP server as specified in IETF RFC 7252 [12];

-	shall support the capability to observer resources as specified in IETF RFC 7641 [14];

-	shall support the block-wise transfer as specified in IETF RFC 7959 [13];

-	shall support the robust block transfer as specified in IETF draft draft-ietf-core-new-block-14 [21];

-	shall support CoAP over TCP and Websocket as specified in IETF RFC 8323 [15];

-	shall support CBOR encoding as specified in IETF RFC 8949 [17];

-	shall support the procedures in clause 6.2.2;

-	shall support the procedures in clause 6.2.3; and

-	shall support the procedures in clause 6.2.4.

NOTE:	The security mechanism to be supported for the CoAP procedures is described in 3GPP TS 24.547 [5]

# 6	Configuration management procedures

## 6.1	General

## 6.2	On-network procedures

### 6.2.1	General

#### 6.2.1.1	Authenticated identity in HTTP request

Upon receiving an HTTP request, the SCM-S shall authenticate the identity of the sender of the HTTP request as specified in 3GPP TS 24.547 [5], and if authentication is successful, the SCM-S shall use the identity of the sender of the HTTP request as an authenticated identity.

#### 6.2.1.2	Authenticated identity in CoAP request

Upon receiving an CoAP request, the SCM-S shall authenticate the identity of the sender of the CoAP request as specified in 3GPP TS 24.547 [5], and if authentication is successful, the SCM-S shall use the identity of the sender of the CoAP request as an authenticated identity.

### 6.2.2	Common procedures

#### 6.2.2.1	Management of configuration update event subscription

##### 6.2.2.1.1	SIP based procedures

6.2.2.1.1.1	General

The VAL service will use the same identity which has been authenticated by VAL service with SIP core using SIP based REGISTER message. If VAL service do not support SIP protocol, then HTTP based method needs to be used.

The SCM-C shall use mechanism provided by VAL service to add access-token in SIP messages. The SCM-S shall identify the originating VAL user ID from the access-token received from SCM-C using the mechanism defined in VAL service specification.

6.2.2.1.1.2	Create subscription

In order to subscribe to notification of changes of one or more group documents of VAL groups identified by VAL group IDs, a SCM-C shall send an initial SIP SUBSCRIBE request to the network according to the UE originating procedures specified in 3GPP TS 24.229 [8] and IETF RFC 5875 [9]. In the initial SIP SUBSCRIBE request, the SCM-C:

a)	shall set the Request-URI to the configured public service identity for performing subscription proxy function of the SCM-S;

b)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.seal" (coded as specified in 3GPP TS 24.229 [8]), in a P-Preferred-Service header field according to IETF RFC 6050 [10];

c)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.seal" in the Contact header field;

d)	shall include an application/resource-lists+xml MIME body. In the application/resource-lists+xml MIME body, the SCM-C shall include one <entry> element for each configuration document to be subscribed to, such that the "uri" attribute of the <entry> element contains a relative path reference to XCAP URI identifying an XML document to be subscribed to;

e)	if the VAL server wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [11], to zero. Otherwise, shall set the Expires header field to the duration for which VAL user has requested for subscription;

Upon reception of an initial SIP SUBSCRIBE request:

a)	with the Event header field set to xcap-diff;

b)	with the Request-URI set to own public service identity for performing subscription proxy function of the SCM-S;

c)	with an application/resource-lists+xml MIME body; and

d)	with the ICSI value "urn:urn-7:3gpp-service.ims.icsi.seal" (coded as specified in 3GPP TS 24 229 [8]), in a P-Asserted-Service header field according to IETF RFC 6050 [10];

the SCM-S:

d)	shall identify the originating VAL user ID and shall use the originating VAL user ID as an authenticated identity when performing the authorization;

b)	if the authenticated identity is not authorized to subscribe to notification of changes of any resource in the application/resource-lists+xml MIME body, shall reject the request with a SIP 403 (Forbidden) response and shall not continue with rest of the steps;

e)	act as a notifier according to IETF RFC 5875 [9].

6.2.2.1.1.3	Modify subscription

In order to modify or refresh subscription, the SCM-C shall send SIP re-SUBSCRIBE request on the same dialog as the existing subscription, and with the same "Event" header. The SCM-C shall follow the steps specified in clause 6.2.2.1.1.2.1 to create SIP SUBSCRIBE request.

Upon reception of a SIP re-SUBSCRIBE request:

a)	with the Event header field set to xcap-diff; and

b)	with an application/resource-lists+xml MIME body;

the SCM-S:

a)	act as a notifier according to IETF RFC 5875 [9].

6.2.2.1.1.4	Delete subscription

In order to delete the subscription, the SCM-C shall send SIP re-SUBSCRIBE request on the same dialog as the existing subscription, and with the same "Event" header. The SCM-C shall follow the steps specified in clause 6.2.2.1.1.2.1 to create SIP SUBSCRIBE request with following clarification:

a)	shall set the Expires header field to zero.

Upon reception of a SIP re-SUBSCRIBE request:

a)	with the Event header field set to xcap-diff; and

b)	with Expires header field set to zero;

the SCM-S:

a)	act as a notifier according to IETF RFC 5875 [9].

##### 6.2.2.1.2	HTTP based procedures

6.2.2.1.2.1	Creating subscription

Upon successful service authorization of the VAL service, the SCM-C shall create a subscription for configuration events by sending an HTTP POST request to the SCM-S. In the HTTP POST request, the SCM-C:

a)	shall set the Request URI to the URI of the SCM-S appended with VAL service identity and the value "/configurationEventsSubscription";

b)	shall include the Host header with public user identity of SCM-S;

c)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [6]; and

c)	include the parameters specified in clause A.1.2 serialized into a JavaScript Object Notation (JSON) structure as specified in IETF RFC 7159 [7].

Upon reception of an HTTP POST request from SCM-C where the Request-URI of the HTTP POST request contains "/configurationEventsSubscription", the SCM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 6.2.1.1, and:

1)	if the identity of the sender of the received HTTP POST request is not authorized user, shall respond with an HTTP 403 (Forbidden) response to the HTTP POST request and skip rest of the steps;

b)	shall generate unique subscription identity and store the subscription details for the authorized user; and

c)	shall send an HTTP 200 (OK) response including parameters specified in clause A.1.3.

6.2.2.1.2.2	Modify a subscription

Upon receiving a request from VAL user to modify existing subscription identified with unique subscription identity, the SCM-C:

a)	shall generate an HTTP PUT request. In the HTTP PUT request:

1)	shall set the Request URI to the same Request URI used while creating subscription in clause 6.2.2.1.2.1.1 appended with subscription identity;

2)	shall include the Host header with public user identity of SCM-S;

3)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [6]; and

4)	include the parameters specified in clause A.1.2 serialized into a JavaScript Object Notation (JSON) structure as specified in IETF RFC 7159 [7].

b)	shall send the HTTP PUT request to the SCM-S.

Upon reception of an HTTP PUT request from SCM-C where the Request-URI of the HTTP PUT request contains "/configurationEventsSubscription" appended with subscription identity, the SCM-S:

a)	shall determine the identity of the sender of the received HTTP PUT request as specified in clause 6.2.1.1, and:

1)	if the identity of the sender of the received HTTP PUT request is not authorized user, shall respond with an HTTP 403 (Forbidden) response to the HTTP PUT request and skip rest of the steps;

b)	shall determine whether subscription for configuration events exists or not based on received subscription identity in request URI; and

1)	if subscription does not exist, shall respond with an HTTP 406 (Not Acceptable) response to the HTTP PUT request and skip rest of the steps;

c)	shall update the subscription details based on received parameters from the HTTP PUT request; and

d)	shall send an HTTP 200 (OK) response including parameters specified in clause A.1.3.

6.2.2.1.2.3	Delete a subscription

Upon receiving a request from VAL user to delete existing subscription identified with unique subscription identity, the SCM-C:

a)	shall generate an HTTP DELETE request. In the HTTP DELETE request:

1)	shall set the Request URI to the same Request URI used while creating subscription in clause 6.2.2.1.2.1.1 appended with subscription identity;

2)	shall include the Host header with public user identity of SCM-S; and

3)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [6]; and

b)	shall send the HTTP DELETE request to the SCM-S.

Upon reception of an HTTP DELETE request from SCM-C where the Request-URI of the HTTP DELETE request contains "/configurationEventsSubscription" appended with subscription identity, the SCM-S:

a)	shall determine the identity of the sender of the received HTTP DELETE request as specified in clause 6.2.1.1, and:

1)	if the identity of the sender of the received HTTP DELETE request is not authorized user, shall respond with an HTTP 403 (Forbidden) response to the HTTP DELETE request and skip rest of the steps;

b)	shall determine whether subscription for configuration events exists or not based on received subscription identity in request URI; and

1)	if subscription does not exist, shall respond with an HTTP 406 (Not Acceptable) response to the HTTP DELETE request and skip rest of the steps;

c)	shall delete the subscription details based on received parameters from the HTTP DELETE request; and

d)	shall send an HTTP 200 (OK) response to the SCM-C.

##### 6.2.2.1.3	CoAP based procedures

###### 6.2.2.1.3.1	General

CoAP based procedures shall use the mechanisms to observe a resource as specified in IETF RFC 7641 [14].

NOTE:	CoAP "observe" mechanism uses the principle of eventual consistency where an intermediate state change can be lost when UDP is used. If it is critical for the client to receive every change in the resource state (and not just the latest state), TCP can be used to avoid missing notifications.

###### 6.2.2.1.3.2	Create a subscription

In order to subscribe to changes of a configuration document the SCM-C shall send an extended CoAP GET request with the CoAP URI set to the URI of an observable configuration document and with the Observe option set to 0 (Register) as specified in IETF RFC 7641 [14].

Upon reception of such an extended CoAP request from SCM-C where the CoAP URI of the request points at an observable configuration document and with the Observe option set to 0 (Register), the SCM-S:

a)	shall perform the steps as for a normal CoAP GET request for a configuration document as defined in clause 6.2.4.4;

b)	shall register the SCM-C as an observer as per IETF RFC 7641 [14]; and

c)	shall send a CoAP 2.05 (Content) response including the current content of the resource and the Observer option with the initial sequence number of the notifications.

###### 6.2.2.1.3.3	Delete a subscription

In order to unsubscribe from changes of a configuration document the SCM-C shall send a CoAP GET request matching the CoAP GET request used to create the subscription but with the Observe option set to 1 (Deregister) as specified in IETF RFC 7641 [14].

Upon reception of a CoAP GET that matches an active subscription but with the Observe option set to 1 (Deregister), the SCM-S:

a)	shall perform the steps as for a normal CoAP GET request for a configuration document as defined in clause 6.2.4.4;

b)	shall deregister the SCM-C as an observer as per IETF RFC 7641 [14]; and

c)	shall send a CoAP 2.05 (Content) response including the current content of the resource and shall not include the Observer option.

#### 6.2.2.2	Notifications

##### 6.2.2.2.1	SIP based procedures

6.2.2.2.1.1	Client procedure

Upon receiving a SIP NOTIFY request associated with a subscription created as result of the sent initial SIP SUBSCRIBE request, the SCM-S:

a)	shall handle the SIP NOTIFY request according to IETF RFC 5875 [9].

6.2.2.2.1.2	Server procedure

In order to send notification of group document update event, the SCM-S shall send SIP NOTIFY to SCM-C according to IETF RFC 5875 [9].

##### 6.2.2.2.2	HTTP based procedures

6.2.2.2.2.1	Receiving configuration update notification

Upon receiving an HTTP POST request over a call back URI which was given to SCM-S at time of the configuration update event subscription message, the SCM-C:

a)	shall validate the subscription identity received in the "Identity" parameter of the HTTP POST request. If the subscription identity is not valid, the SCM-C:

1) shall send an HTTP 406 (Not Acceptable) response and skip rest of the steps;

b)	shall send an HTTP 200 (OK) message; and

c)	shall notify the VAL user about the modification of configuration document based on the "Event" parameter.

Based on VAL user’s request, the SCM-C may also retrieve the configuration document as specified in clause 6.2.3 or in clause 6.2.4.

6.2.2.2.2.2	Sending group modify notification

Upon successful modification of VAL user profile document or VAL UE configuration document, the SCM-S sends a notification to SCM-C. The SCM-S:

a)	shall check whether valid configuration update event subscription exists for event SUBSCRIBE_USER_PROFILE_MODIFICATION (0x01) OR SUBSCRIBE_UE_CONFIG_MODIFICATION (0x02) as defined in clause A.1.2 or not;

1)	if valid subscription does not exist, shall skip rest of the steps;

b)	shall generate an HTTP POST message to notify configuration update notification. In HTTP POST message:

1)	shall set the request URI to call back URI received in the creating subscription procedure;

2)	shall set the Content-Type header to "application/json"; and

3)	shall include an HTTP request entity-body with the parameters specified in clause B.2 serialized into a JavaScript Object Notation (JSON) structure; and

c)	shall sent an HTTP POST request towards SCM-C.

##### 6.2.2.2.3	CoAP based procedures

###### 6.2.2.2.3.1	Client procedure

Upon receiving a CoAP 2.05 (Content) response that matches the extended CoAP GET request which initiated the subscription and which contains the Observe option, the SCM-C:

a)	shall handle the response according to IETF RFC 7641 [14]; and

b)	shall notify the VAL user about the modification of the configuration document.

###### 6.2.2.2.3.2	Server procedure

In order to send a notification when the configuration document is modified, the SCM-S shall send a CoAP 2.05 (Content) response to SCM-C containing the modified document and the Observe option according to IETF RFC 7641 [14]. The Content-Format specified in a 2.xx notification shall be the same as the one used in the initial response to the GET request received for the subscription.

### 6.2.3	VAL UE configuration data

#### 6.2.3.1	Client procedure

Upon receiving a request from the VAL user to retrieve a VAL UE configuration data, the SCM-C shall send an HTTP GET request to the SCM-S according to procedures specified in IETF RFC 4825 [3] "Fetch a Document". In HTTP GET request, the SCM-C:

a)	shall set the Request-URI to a XCAP URI identifying the XML document to be retrieved. In the Request-URI:

1)	the "XCAP Root" is set to the URI of the SCM-S;

2)	the "auid" is set to specific VAL service identity; and

3)	the document selector is set to a document URI pointing to the VAL UE configuration document;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [6]; and

c)	may include the parameters specified in clause A.2.1 serialized into a JavaScript Object Notation (JSON) structure as specified in IETF RFC 7159 [7]

#### 6.2.3.2	Server procedure

Upon reception of an HTTP GET request where the Request-URI of the HTTP GET request identifies a UE configuration document as specified in the specific vertical application, the SCM-S:

a)	shall determine the identity of the sender of the received HTTP GET request as specified in clause 6.2.1.1, and:

1)	if the identity of the sender of the received HTTP GET request is not authorized to fetch requested configuration document, shall respond with a HTTP 403 (Forbidden) response to the HTTP GET request and skip rest of the steps; and

b)	shall support handling an HTTP GET request from a SCM-C according to procedures specified in IETF RFC 4825 [3] "GET Handling".

### 6.2.4	VAL user profile data

#### 6.2.4.1	SCM client HTTP procedure

Upon receiving a request from the VAL user to retrieve a VAL user profile data, the SCM-C shall send an HTTP GET request to the SCM-S according to procedures specified in IETF RFC 4825 [3] "Fetch a Document". In HTTP GET request, the SCM-C:

a)	shall set the Request-URI to a XCAP URI identifying the XML document to be retrieved. In the Request-URI:

1)	the "XCAP Root" is set to the URI of the SCM-S;

21)	the "auid" is set to specific VAL service identity; and

3)	the document selector is set to a document URI pointing to the VAL user profile document; and

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [6].

#### 6.2.4.2	SCM server HTTP procedure

Upon reception of an HTTP GET request where the Request-URI of the HTTP GET request identifies a user profile document as specified in the specific vertical application, the SCM-S follow the procedure as described in clause 6.2.3.2.

#### 6.2.4.3	SCM client CoAP procedure

Upon receiving a request from the VAL user to retrieve a VAL user profile data, the SCM-C shall send a CoAP GET request to the SCM-S. In the CoAP GET request, the SCM-C:

a)	shall set the CoAP URI identifying the user profile document to be retrieved according to the resource API definition in Annex C.2.1:

1)	the "apiRoot" is set to the SCM-S URI;

2)	the "valServiceId" is set to specific VAL service; and

3)	if the SCM-C does not know the "profileDocId" of the user profile document at the SGM-S, the SCM-C:

i)	shall use the User Profiles resource GET, as described in Annex C.2.1.2.2.3.1, and shall set val-tgt-ue to either the VAL user identity or VAL UE identity; or

ii)	shall use the Individual User Profile resource GET, as described in Annex C.2.1.2.3.3.1, and shall set "profileDocId" to point to the VAL user profile document; and

b)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [5].

Editor's note:	The method to protect the request will be decided by SA3, and necessary alignment with SA3 is FFS.

#### 6.2.4.4	SCM server CoAP procedure

Upon reception of an CoAP GET request where the CoAP URI of the CoAP GET request identifies User Profiles resource as described in Annex C.2.1.2.2.3.1, the SCM-S:

a)	shall determine the identity of the sender of the received CoAP GET request as specified in clause 6.2.1.2, and:

1)	if the identity of the sender of the received CoAP GET request is not authorized to fetch requested user profile document(s), shall respond with a CoAP 4.03 (Forbidden) response to the CoAP GET request and skip rest of the steps;

b)	shall support handling a CoAP GET request from a SCM-C according to procedures specified in IETF RFC 7252 [12]; and

c)	shall search for the user profile documents for the given VAL service and VAL user or VAL UE and:

1)	shall return all the found documents in the 2.05 (Content) response; or

2)	shall return a 4.04 (Not found) response if no document matching the given criteria was found.

Upon reception of an CoAP GET request where the CoAP URI of the CoAP GET request identifies Individual User Profile resource as described in Annex C.2.1.2.3.3.1, the SCM-S:

a)	shall determine the identity of the sender of the received CoAP GET request as specified in clause 6.2.1.2, and:

1)	if the identity of the sender of the received CoAP GET request is not authorized to fetch requested user profile document(s), shall respond with a CoAP 4.03 (Forbidden) response to the CoAP GET request and skip rest of the steps;

b)	shall support handling an CoAP GET request from a SCM-C according to procedures specified in IETF RFC 7252 [12]; and

c)	shall search for the user profile documents pointed at by the CoAP URI and:

1)	shall return the document in the 2.05 (Content) response if the document is found; or

2)	shall return a 4.04 (Not found) response if the document is not found.

### 6.2.5	Update VAL user profile data

#### 6.2.5.1	SCM client HHTP procedure

Upon receiving a request from the VAL user to update the VAL user profile configuration document, the SCM-C shall create an XML document as specified in coding of the specific vertical application and shall send the XML document to the SCM-S according to procedures specified in IETF RFC 4825 [3] "Create or Replace a Document". In the HTTP POST request, the SCM-C:

a)	shall set the Request URI to a XCAP URI identifying an XML document to be updated. In the Request-URI:

1)	the "XCAP Root" is set to the URI of the SCM-S;

2)	the "auid" is set to specific VAL service identity; and

3)	the document selector is set to the VAL user profile;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [6];

c)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-user-profile-info+xml"; and

d)	shall include an application/vnd.3gpp.seal-user-profile-info+xml MIME body and in the <seal-user-profile> root element:

1)	may include <ProfileName> element indicating name of the profile;

2)	may include <Status> element indicating status of the profile;

3)	may include <isDefault> element indicating that the current profile is the selected profile for the requesting user;

4)	shall include <user-profile-index> element indicating the unique profile number; and

5)	shall include <profile-configuration> element as specified in clause 7.

#### 6.2.5.2	SCM server HTTP procedure

Upon reception of an HTTP PUT request where the Request-URI of the HTTP PUT request identifies an XML document as specified in the specific vertical application, the SCM-S:

a)	shall determine the identity of the sender of the received HTTP PUT request as specified in clause 6.2.1.1, and:

1)	if the identity of the sender of the received HTTP PUT request is not authorized to update the configuration document, shall respond with a HTTP 403 (Forbidden) response to the HTTP PUT request and skip rest of the steps; and

b)	shall support receiving an XML document as specified in application usage of the specific vertical application according to procedures specified in IETF RFC 4825 [3] "PUT Handling".

#### 6.2.5.3	SCM client CoAP procedure

Upon receiving a request from the VAL user to update the VAL user profile configuration document, the SCM-C shall send a CoAP PUT request to the SCM-S. In the CoAP PUT request, the SCM-C:

a)	shall set the CoAP URI identifying the user profile document to be retrieved according to the resource definition in Annex C.2.1.2.3.3.2:

1)	the "apiRoot" is set to the SCM-S URI;

2)	the "valServiceId" is set to specific VAL service; and

3)	the "profileDocId" to point to the VAL user profile document;

b)	shall include Content-Format option set to "application/cbor";

Editor's Note:	consider defining a media type for the user profile, such as "vnd.3gpp.seal-user-profile-info+cbor".

c)	shall include "ProfileDoc" object with "profileInformation" which:

1)	may contain "profileName" element indicating name of the profile;

2)	may contain "status" element indicating status of the profile;

3)	may contain "isDefault" element indicating that the current profile is the selected profile for the requesting user; and

4)	shall contain "profileConfig" elements; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [5].

#### 6.2.5.4	SCM server CoAP procedure

Upon reception of an CoAP PUT request where the CoAP URI of the request identifies Individual User Profile resource as described in Annex C.2.1.2.3.3.2, the SCM-S:

a)	shall determine the identity of the sender of the received CoAP GET request as specified in clause 6.2.1.2, and:

1)	if the identity of the sender of the received CoAP PUT request is not authorized to update requested user profile document(s), shall respond with a CoAP 4.03 (Forbidden) response to the CoAP GET request and skip rest of the steps;

b)	shall support handling an CoAP PUT request from a SCM-C according to procedures specified in IETF RFC 7252  [12]; and

## c)	shall replace the user profile documents pointed at by the CoAP URI with the "ProfileDoc" received in the request.6.3	Off-network procedures

The off-network procedures are out of scope of the present document in this release of the specification.

# 7	Coding

## 7.1	VAL user profile document

### 7.1.1	General

### 7.1.2	Application unique ID

The AUID shall be set to the VAL service ID as specified in specific VAL service specification.

### 7.1.3	Data structure

The <seal-user-profile> element shall be the root element of the VAL user-profile configuration document.

The <seal-user-profile> element:

a)	may include a <ProfileName> element;

b)	shall include a <Status> element;

c)	may include a <Pre-selected-indication> element;

d)	shall include a <user-profile-index> element;

e)	shall include a <profile-configuration> element;

1)	may include a <Common> element;

2)	may include a <OnNetwork> element; and

3)	may include a <OffNetwork> element; and

f)	may include any other attribute for the purposes of extensibility.

### 7.1.4	XML Schema

The seal user profile configuration document shall be composed according to the following XML schema:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns="urn:3gpp:ns:seal:SealUserProfile:1.0"

targetNamespace="urn:3gpp:ns:seal:SealUserProfile:1.0"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:sealup="urn:3gpp:ns:seal:SealUserProfile:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified">

<xs:element name="seal-user-profile">

<xs:complexType>

<xs:choice minOccurs="1" maxOccurs="unbounded">

<xs:element name="ProfileName" type="sealup:NameType"/>

<xs:element name="Status" type="xs:boolean"/>

<xs:element name="isDefault" type="xs:boolean"/>

<xs:element name="profile-configuration" type="sealup:ProfileConfigurationType"/>

<xs:element name="anyExt" type="sealuec:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

<xs:attribute name="user-profile-index" type="xs:unsignedByte" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:element>

<xs:complexType name="NameType">

<xs:simpleContent>

<xs:extension base="xs:token">

<xs:attribute ref="xml:lang"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="ProfileConfigurationType">

<xs:choice minOccurs="1" maxOccurs="unbounded">

<xs:element name="Common" type="sealup:CommonType"/>

<xs:element name="OnNetwork" type="sealup:OnNetworkType"/>

<xs:element name="OffNetwork" type="sealup:OffNetworkType"/>

<xs:element name="anyExt" type="sealuec:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="CommonType" />

<xs:complexType name="OnNetworkType" />

<xs:complexType name="OffNetworkType" />

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

### 7.1.5	Semantics

The <seal-user-profile> element is the root element of the XML document.

The <ProfileName> element of <seal-user-profile> element specifies the name of the SEAL user profile configuration document.

The <Status> element of <seal-user-profile> element is of type "Boolean" and indicates whether this particular SEAL user profile is enabled or disabled.

The <isDefault> element of <seal-user-profile> element is of type "Boolean" and indicates whether this particular SEAL user profile is default profile for VAL user or not.

The <user-profile-index> element of <seal-user-profile> element contains a positive number which provides profile id. This element is used only when multiple user-profile for a VAL user is supported.

The <profile-configuration> element of <seal-user-profile> element contains actual profile configuration. The VAL application which uses SEAL user-profile may provide its own profile configuration specific to VAL application.

The VAL service may further extend the <Common> element of the <profile-configuration> element of the <seal-user-profile> element to include VAL service specific common user profile configuration.

The VAL service may further extend the <OnNetwork> element of the <profile-configuration> element of the <seal-user-profile> element to include VAL service specific user profile configuration for on-network features.

The VAL service may further extend the <OffNetwork> element of the <profile-configuration> element of the <seal-user-profile> element to include VAL service specific user profile configuration for off-network features.

### 7.1.6	MIME type

The MIME type for VAL user profile configuration shall be set to "vnd.3gpp.seal-user-profile-info+xml".

### 7.1.7	IANA registration template

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.seal-user-profile-info+xml

Required parameters:

None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP or in HTTP. So the security considerations from IETF RFC 3261 apply while exchanging information in SIP and the security considerations from IETF RFC 2616 apply while exchanging information in HTTP.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.546 "Configuration management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification", available via http://www.3gpp.org/specs/numbering.htm.

Applications Usage:

Applications supporting the SEAL configuration management procedures as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

## 7.2	VAL UE configuration document

### 7.2.1	General

### 7.2.2	Application unique ID

The AUID shall be set to the VAL service ID as specified in specific VAL service specification.

### 7.2.3	Data structure

The SEAL UE configuration document structure is specified in this clause.

The <seal-UE-configuration> document:

1)	shall include a "domain" attribute;

2)	may include a <VAL-UE-id> element;

3)	may include a <VAL-service-id> element;

4)	may include a <name> element;

5)	may include a <common> element;

6)	may include an <on-network> element; and

7)	may include any other attribute for the purposes of extensibility.

The <VAL-UE-id> element:

1)	may contain a list of <Instance-ID-URN> elements; and

2)	may contain a list of <IMEI-range> elements.

The <IMEI-range> element:

1)	shall contain a <TAC> element;

2)	may contain a list of <SNR> elements; and

3)	may contain <SNR-range> element.

The <SNR-range> element:

1)	shall contain a <Low-SNR> element; and

2)	shall contain a <High-SNR> element.

### 7.2.4	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns="urn:3gpp:ns:seal:sealUEConfig:1.0"

targetNamespace="urn:3gpp:ns:seal:sealUEConfig:1.0"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:sealuec="urn:3gpp:ns:seal:sealUEConfig:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified">

<xs:element name="seal-UE-configuration">

<xs:complexType>

<xs:sequence>

<xs:choice minOccurs="0" maxOccurs="unbounded">

<xs:element name="VAL-UE-id" type="sealuec:VALUEIDType"/>

<xs:element name="VAL-service-id" type="xs:string"/>

<xs:element name="name" type="sealuec:NameType"/>

</xs:choice>

<xs:element name="common" type="sealuec:CommonType"/>

<xs:element name="on-network" type="sealuec:On-networkType"/>

<xs:element name="anyExt" type="sealuec:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="domain" type="xs:anyURI" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:element>

<xs:complexType name="NameType">

<xs:simpleContent>

<xs:extension base="xs:token">

<xs:attribute ref="xml:lang"/>

<xs:attributeGroup ref="sealuec:IndexType"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="VALUEIDType">

<xs:choice minOccurs="0" maxOccurs="unbounded">

<xs:element name="Instance-ID-URN" type="xs:anyURI"/>

<xs:element name="IMEI-range" type="sealuec:IMEI-rangeType"/>

<xs:element name="anyExt" type="sealuec:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:attributeGroup ref="sealuec:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="IMEI-rangeType">

<xs:sequence>

<xs:element name="TAC" type="sealuec:tacType"/>

<xs:choice minOccurs="0" maxOccurs="unbounded">

<xs:element name="SNR" type="sealuec:snrType"/>

<xs:element name="SNR-range" type="sealuec:SNR-rangeType"/>

</xs:choice>

<xs:element name="anyExt" type="sealuec:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attributeGroup ref="sealuec:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="SNR-rangeType">

<xs:sequence>

<xs:element name="Low-SNR" type="sealuec:snrType"/>

<xs:element name="High-SNR" type="sealuec:snrType"/>

<xs:element name="anyExt" type="sealuec:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attributeGroup ref="sealuec:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tac-baseType">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="8"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tacType">

<xs:simpleContent>

<xs:extension base="sealuec:tac-baseType">

<xs:attributeGroup ref="sealuec:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:simpleType name="snr-baseType">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="6"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="snrType">

<xs:simpleContent>

<xs:extension base="sealuec:snr-baseType">

<xs:attributeGroup ref="sealuec:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="CommonType" />

<xs:complexType name="On-networkType" />

<xs:attributeGroup name="IndexType">

<xs:attribute name="index" type="xs:token"/>

</xs:attributeGroup>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

### 7.2.5	Semantics

The "domain" attribute of the <seal-UE-configuration> element contains the domain name of the VAL service.

The <name> element of the <seal-UE- configuration> element contains the user displayable name of the SEAL UE configuration document.

The creator of the SEAL UE configuration document may include an <VAL-UE-id> element in the version of the SEAL UE configuration document that is uploaded to the SCM-S. If an <VAL-UE-id> element is included then the SEAL UE configuration document applies only to the VAL UE(s) identified by the <VAL-UE-id> element. If no <VAL-UE-id> element is included then the SEAL UE configuration document applies to all the VAL UEs of the domain.

The <VAL-Service-id> element contains identify of the VAL service for which the configuration document is applicable.

If one or more optional <Instance-ID-URN> elements is included in the <VAL-UE-id> element then the SEAL UE configuration document applies to the VAL UE with an instance ID equal to the instance ID contained in the <Instance-ID-URN> element.

The <TAC> element of the <IMEI-range> element contains the Type Allocation Code of the VAL UE.

The optional <SNR> element of the <IMEI-range> element contains the individual serial number uniquely identifying VAL UE within the Type Allocation Code contained in the <TAC> element that the SEAL UE configuration document applies to.

If an optional <SNR-range> element is included within the <IMEI-range> element then the SEAL UE configuration document applies to all VAL UEs within the Type Allocation Code contained in the <TAC> element with the serial number equal or greater than the serial number contained in the <Low-SNR> element and less than or equal to the serial number contained in the <High-SNR> element.

If no <SNR> element nor <SNR-range> element is included within the <IMEI-range> element then the SEAL UE configuration document applies to all the VAL UE(s) with the Type Allocation Code contained within the <TAC> element  of the <IMEI-range> element.

If no <VAL-UE-id> element is included then the SEAL UE configuration document applies to all VAL UEs of the VAL service identified in the "domain" attribute.

The VAL service may further extend the <Common> element of the <seal-UE-configuration> to include VAL service specific common UE configuration.

The VAL service may further extend the <on-network> element of the <seal-UE-configuration> to include VAL service specific UE configuration for on-network features.

### 7.2.6	MIME type

The MIME type for VAL user profile configuration shall be set to "vnd.3gpp.seal-ue-config-info+xml".

### 7.2.7	IANA registration template

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.seal-ue-config-info+xml

Required parameters:

None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP or in HTTP. So the security considerations from IETF RFC 3261 apply while exchanging information in SIP and the security considerations from IETF RFC 2616 apply while exchanging information in HTTP.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.546 "Configuration management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification", available via http://www.3gpp.org/specs/numbering.htm.

Applications Usage:

Applications supporting the SEAL configuration management procedures as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

Annex A (normative):
Parameters for different operations

# A.1	Creating configuration update event subscription

## A.1.1	General

The information in this annex provides a normative description of the parameters which will be sent by SCM-C while creating configuration update event subscription and the parameters which will be sent by SCM-S as a response to request for creating subscription.

## A.1.2	Client side parameters

The SCM-C shall convey the following parameters while sending request for creating configuration update event subscription.

Table A.1.2-1: Client side parameters for creating configuration update event subscription

| Parameter | Description |
| --- | --- |
| Callback-URI | REQUIRED. Represents where to send HTTP notifications |
| Subscription Info | REQUIRED. Represents a space-separated list of the subscription type information as specified in table A.1.2-2. |

Table A.1.2-2: Subscription information

| Parameter | Description |
| --- | --- |
| Event | REQUIRED. Represents the type of notification which client requires. This specification defines following type of notifications: - 0x01: SUBSCRIBE_USER_PROFILE_MODIFICATION - 0x02: SUBSCRIBE_UE_CONFIG_MODIFICATION |
| expiry time | REQUIRED. Represents the time in seconds up to which the subscription is desired to be kept active and the time after which the subscribed event shall stop generating notifications. |

## A.1.3	Server side parameters

The SCM-S shall convey the following parameters while sending response to the creating configuration update event subscription request.

Table A.1.3-1: Server side parameters for response to creating configuration update event subscription

| Parameter | Description |
| --- | --- |
| Identity | REQUIRED. A unique string representing subscription identity. |

# A.2	Retrieve VAL UE configuration data

## A.2.1	Client side parameters

The SGM-C shall convey the following parameters, if available, while sending request to retrieve a VAL UE configuration data.

Table A.1.2-1: Client side parameters to retrieve VAL UE configuration data

| Parameter | Description |
| --- | --- |
| VAL UE Information | OPTIONAL. Represents additional UE related information required to identify the configuration data (e.g. device type, device vendor, etc). |

Annex B (normative):
Parameters for notifications

# B.1	General

The information in this annex provides a normative description of the parameters which will be sent by SCM-S while sending different types of notification

# B.2	Configuration update notification

The SCM-S shall convey the following parameters while sending configuration notification to SCM-C.

Table B.2-1: Parameters for configuration update notification

| Parameter | Description |
| --- | --- |
| Identity | REQUIRED. A unique string representing notification channel identity. |
| Event | REQUIRED. Shall be set to one of the event as specified in table A.1.2-2 based on which configuration document is updated. |

Annex C (normative):
CoAP resource representation and encoding

# C.1	General

The information in this annex provides a normative description of CoAP resource representation and encoding.

## C.1.1	Resource URI structure

All API URIs of SEAL-UU APIs shall be specified as follows:

{apiRoot}/<apiName>/<apiVersion>

"apiRoot" is configured by means outside the scope of the present document. It includes one of the schemes ("coaps", "coaps+tcp", "coaps+ws"), host and optional port, and an optional prefix string. "apiName" and "apiVersion" shall be set dependent on the API, as defined in the corresponding clauses below.

All resource URIs specified for SEAL-UU APIs shall be defined relative to the above root API URI.

URIs which differ only in the scheme shall point to the same resource.

NOTE:	The "apiVersion" will only be increased if the new API version contains backward incompatible changes.

The root structure may be followed by "apiSpecificSuffixes" that are dependent on the API and are defined separately for each API as resource URI where they apply:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

## C.1.2	Use of cache

Editor's Note:	The use of cache and the related CoAP options is FFS.

## C.1.3	Error handling

Table C.1.3-1 lists response payload types that are applicable to all APIs and as responses for all requests in the present specification unless otherwise specified. The CoAP client shall mandatorily support the processing of the status code for all the applicable methods, when received in a CoAP response message.

Table C.1.3-1: Response payloads supported for responses to all requests.

| Response body | Data type | Cardinality | Response Codes (NOTE) | Remarks | Applied Methods |
| --- | --- | --- | --- | --- | --- |
|  | ProblemDetails | 1 | 4.00 Bad Request | Incorrect parameters were passed in the request. | GET, POST PUT, PATCH, DELETE |
|  | ProblemDetails | 1 | 4.01 Unauthorized | The client is not authorized. | GET, POST, PUT, PATCH, DELETE |
|  | ProblemDetails | 1 | 4.02 Bad Option | The request could not be understood by the server due to one or more unrecognized or malformed options. | GET, POST, PUT, PATCH, DELETE |
|  | ProblemDetails | 1 | 4.03 Forbidden | This represents the case when the server is able to understand the request but unable to fulfil the request due to errors (e.g. the requested parameters are out of range). More information may be provided in the "invalidParams" attribute of the "ProblemDetails" structure. | GET, POST, PUT, PATCH, DELETE |
|  | ProblemDetails | 1 | 4.04 Not Found | The resource URI was incorrect. | GET, POST, PUT, PATCH, DELETE |
|  | ProblemDetails | 1 | 4.06 Not Acceptable | The content format provided in the "Accept" option is not acceptable by the server. | GET |
|  | ProblemDetails | 1 | 4.13 Request Entity Too Large | If the received CoAP request contains entity larger than the server is able to process, the server shall reject the CoAP request with this status code. The server should include Size1 option in the response with the maximum size of the request entity it can handle. | POST, PUT, PATCH |
|  | ProblemDetails | 1 | 4.15 Unsupported Content-Format | The code indicates that the resource is in a format which is not supported by the server for the method. | POST, PUT, PATCH |
|  | ProblemDetails | 1 | 4.29 Too Many Requests | The code indicates that due to excessive traffic which, if continued over time, may lead to (or may increase) an overload situation. The CoAP option "Max-Age" may be added in the response to indicate how long the client has to wait before making a new request. | GET, POST, PUT, PATCH, DELETE |
|  | ProblemDetails | 1 | 5.00 Internal Server Error | The server encountered an unexpected condition that prevented it from fulfilling the request. | GET, POST, PUT, PATCH, DELETE |
|  | ProblemDetails | 1 | 5.03 Service Unavailable | The server is unable to handle the request. | GET, POST, PUT, PATCH, DELETE |
| NOTE: In addition to the above response codes, the CoAP server may also send other valid CoAP response codes, if applicable. The list of all valid CoAP response codes can be found in CoAP Response Code Registry at IANA [19]. |  |  |  |  |

Editor's Note:	Handling of "ProblemDetails" indicated in Table C.1.3-1 based on the IETF draft [20] is FFS.

Editor's Note:	Handling of the PATCH method indicated in Table C.1.3-1 is FFS.

Specific errors are contained in the related API definition for each API.

# C.2	Resource representation and APIs for VAL user profile

## C.2.1	SU_UserProfile API

### C.2.1.1	API URI

The CoAP URIs used in CoAP requests from SCM-C towards the SCM-S shall have the Resource URI structure as defined in clause C.1.1 with the following clarifications:

-	the <apiName> shall be "su-up";

-	the <apiVersion> shall be "v1"; and

-	the <apiSpecificSuffixes> shall be set as described in clause C.2.1.2.

### C.2.1.2	Resources

#### C.2.1.2.1	Overview

Figure C.2.1.2.1-1: Resource URI structure of the SU_UserProfile API

Table C.2.1.2.1-1 provides an overview of the resources and applicable CoAP methods.

Table C.2.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| User Profiles | /val-services/{valServiceId}/user-profiles | GET | Retrieve VAL user or VAL UE's user profiles for a given VAL service, according to query criteria. |
|  |  | POST | Create user profile. |
| Individual User Profile | /val-services/{valServiceId}/user-profiles/{profileDocId} | GET | Retrieve an individual user profile. |
|  |  | PUT | Update an individual user profile. |
|  |  | DELETE | Delete an individual user profile. |

Editor’s note: Whether any changes required in the API along with its data model based on limitations of constrained devices is FFS.

#### C.2.1.2.2	Resource: User Profiles

##### C.2.1.2.2.1	Description

The User Profiles resource allows a SCM-C to retrieve all the user profiles of a VAL user or a VAL UE for a specific VAL service that are available at a given SCM-S, or allows to create a new user profile.

##### C.2.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/su-up/<apiVersion>/val-services/{valServiceId}/user-profiles

This resource shall support the resource URI variables defined in the table C.2.1.2.2.2-1.

Table C.2.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 |
| apiVersion | string | See clause C.2.1.1 |
| valServiceId | string | Identifier of a VAL service. |

##### C.2.1.2.2.3	Resource Standard Methods

###### C.2.1.2.2.3.1	GET

This operation retrieves VAL user or VAL UE profile information satisfying the filter criteria.

This method shall support the URI query parameters specified in table C.2.1.2.2.3.1-1.

Table C.2.1.2.2.3.1-1: URI query parameters supported by the GET Request on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-tgt-ue | ValTargetUe | M | 1 | Identifying a VAL target UE. |

This method shall support the response data structures and response codes specified in table C.2.1.2.2.3.1-2.

Table X.2.1.2.2.3.1-2: Data structures supported by the GET Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(ProfileDoc) | M | 1..N | 2.05 Content | List of VAL user / VAL UE profile documents. This response shall include user profile information matching the query parameters provided in the request. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 shall also apply. |  |  |  |

###### C.2.1.2.2.3.2	POST

This operation creates a VAL user or VAL UE profile information at the SCM-S for a given VAL service.

This method shall support the request data structures specified in table C.2.1.2.2.3.2-1, the response data structures and response codes specified in table C.2.1.2.2.3.2-2, and the response options specified in table C.2.1.2.2.3.2-3.

Table C.2.1.2.2.3.2-1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table C.2.1.2.2.3.2-2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ProfileDoc | O | 1 | 2.01 Created | The user profile was created successfully. The "profileDocId" of the created resource shall be returned in the "Location-Path" option. |
| NOTE: The mandatory CoAP error status codes for the POST method listed in table C.1.3-1 shall also apply. |  |  |  |  |

Table C.2.1.2.2.3.2-3: Options supported by the 2.01 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location-Path | string | M | 1 | Contains the location path of the newly created resource relative to the request URI. It contains the profileDocId segment of the complete resource URI according to the structure: {apiRoot}/su-up/<apiVersion>/val-services/{valServiceId}/user-profiles/{profileDocId} |

#### C.2.1.2.3	Resource: Individual User Profile

##### C.2.1.2.3.1	Description

The Individual User Profile resource represents an individual user profile that is created at the SCM-S for a given VAL service. This resource is observable.

##### C.2.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/su-up/<apiVersion>/val-services/{valServiceId}/user-profiles/{profileDocId}

This resource shall support the resource URI variables defined in the table C.2.1.2.3.2-1.

Table C.2.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 |
| apiVersion | string | See clause C.2.1.1 |
| valServiceId | string | Identifier of a VAL service. |
| profileDocId | string | Represents an individual user profile resource. |

##### C.2.1.2.3.3	Resource Standard Methods

###### C.2.1.2.3.3.1	GET

This operation retrieves the user profile document.

This method shall support the request options specified in table C.2.1.2.3.3.1-1, the response data structures and response codes specified in table C.2.1.2.3.3.1-2, and the response options specified in table C.2.1.2.3.3.1-3.

Table C.2.1.2.3.3.1-1: Options supported by the GET Request on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Observe | uint | O | 1 | When set to 0 (Register) it extends the GET request to subscribe to the changes of this resource. When set to 1 (Deregister) it cancels the subscription. |
| NOTE: Other request options also apply in accordance with normal CoAP procedures. |  |  |  |  |

Table C.2.1.2.3.3.1-2: Data structures supported by the GET Response payload on this resource

| Data type |  | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- | --- |
| ProfileDoc |  | M | 1 | 2.05 Content | The User profile information based on the request from the SCM-C. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 shall also apply. |  |  |  |  |

Table C.2.1.2.3.3.1-3: Options supported by the 2.05 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Observe | uint | O | 1 | Sequence number of the notification. |
| NOTE: Other response options also apply in accordance with normal CoAP procedures. |  |  |  |  |

###### C.2.1.2.3.3.2	PUT

This operation updates the user profile document.

This method shall support the request data structures specified in table C.2.1.2.3.3.2-1 and the response data structures and response codes specified in table C.2.1.2.3.3.2-2.

Table C.2.1.2.3.3.2-1: Data structures supported by the PUT Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ProfileDoc | M | 1 | Updated details of the user profile document. |

Table C.2.1.2.3.3.2-2: Data structures supported by the PUT Response payload on this resource

| Data type |  | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- | --- |
| ProfileDoc |  | O | 1 | 2.04 Changed | The user profile document updated successfully and the updated user profile document may be returned in the response. |
| NOTE: The mandatory CoAP error status codes for the PUT method listed in table C.1.3-1 shall also apply. |  |  |  |  |

###### C.2.1.2.3.3.3	DELETE

This operation deletes the user profile document.

This method shall support the response data structures and response codes specified in table C.2.1.2.3.3.3-1.

Table C.2.1.2.3.3.3-1: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | The individual User profile document matching the profileDocId is deleted. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 shall also apply. |  |  |  |  |

### C.2.1.4	Data Model

#### C.2.1.4.1	General

Table C.2.1.4.1-1 specifies the data types defined specifically for the SU_UserProfile API service.

Table C.2.1.4.1-1: SU_UserProfile API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ProfileDoc | C.2.1.4.2.1 | Profile information associated with VAL user ID or VAL UE ID. |  |
| ProfileInfo | C.2.1.4.2.2 | Profile information including profile configurations. |  |
| ProfileConfig | C.2.1.4.2.3 | Profile configuration including configuration data. |  |
| ProfileConfigType | C.2.1.4.3.1 | Specifies type of features for which the configuration data is applicable. |  |
| ValTargetUe | C.2.1.4.2.4 | Information identifying a VAL user ID or VAL UE ID. |  |

#### C.2.1.4.2	Structured data types

##### C.2.1.4.2.1	Type: ProfileDoc

Table C.2.1.4.2.1-1: Definition of type ProfileDoc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| profileDocId | string | O | 1 | Contains the profileDocId of the complete resource URI of this user profile document according to the structure: {apiRoot}/su-up/<apiVersion>/val-services/{valServiceId}/user-profiles/{profileDocId} This attribute shall be provided by the SCM-S in CoAP responses. |  |
| profileInformation | ProfileInfo | M | 1 | Profile information associated with a VAL user or a VAL UE as specified in valTgtUe. |  |
| valTgtUe | ValTargetUe | M | 1 | Unique identifier of a VAL user or a VAL UE. |  |

##### C.2.1.4.2.2	Type: ProfileInfo

Table C2.1.4.2.2-1: Definition of type ProfileInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| profileName | string | O | 1 | Name of the user profile. |  |
| status | boolean | M | 1 | Indicates whether the user profile is enabled or disabled. |  |
| array(ProfileConfig) | ProfileConfig | O | 1..N | List of profile configurations. |  |
| isDefault | boolean | O | 1 | Indicates whether the user profile is the default profile for VAL user or not. |  |

##### C.2.1.4.2.3	Type: ProfileConfig

Table C.2.1.4.2.3-1: Definition of type ProfileConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| configType | ProfileConfigType | M | 1 | Indicates the type of the profile configuration. |  |
| configData | string | M | 1 | Actual user profile configuration data. |  |

##### C.2.1.4.2.4	Type: ValTargetUe

Table C.2.1.4.2.4-1: Definition of type ValTargetUe

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUserId | string | O | 0..1 | Unique identifier of a VAL user. |  |
| valUeId | string | O | 0..1 | Unique identifier of a VAL UE. |  |
| NOTE: Either "valUserId" or "valUeId" shall be present. |  |  |  |  |  |

#### C.2.1.4.3	Simple data types and enumerations

##### C.2.1.4.3.1	Enumeration: ProfileConfigType

Table C.2.1.4.3.1-1: Enumeration ProfileConfigType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| COMMON | Indicates VAL service specific common user profile configuration. |  |
| ON_NETWORK | Indicates VAL service specific user profile configuration for on-network features. |  |
| OFF_NETWORK | Indicates VAL service specific user profile configuration for off-network features. |  |

### C.2.1.5	Error Handling

General error responses are defined in clause C.1.3.

### C.2.1.6	CDDL Specification

#### C.2.1.6.1	Introduction

The data model described in clause C.2.1.4 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [17].

Clause C.2.1.6.2 uses the Concise Data Definition Language described in IETF RFC 8610 [18] and provides corresponding representation of the SU_UserProfile API data model.

#### C.2.1.6.2	CDDL document

;;; ProfileDoc

;;+ Represents user profile information associated with a VAL user ID or a VAL UE ID.

ProfileDoc = {

? profileDocId: text

profileInformation: ProfileInfo

valTgtUe: ValTargetUe

}

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

ValTargetUe = {

(

valUserId: text                 ; Unique identifier of a VAL user.

//

valUeId: text                   ; Unique identifier of a VAL UE.

)

}

;;; ProfileInfo

;;+ User profile information.

ProfileInfo = {

? profileName: text             ; Name of the profile

status: bool                    ; Indicates whether the user profile is enabled or disabled.

? profileConfigs: [+ ProfileConfig]

? isDefault: bool               ; Indicates whether the user profile is the default profile for VAL user or not.

}

;;; ProfileConfig

;;+ Profile configuration.

ProfileConfig = {

configType: ProfileConfigType

configData: text                ; Actual user profile configuration data.

}

;;; ProfileConfigType

;;+ Indicates the type of the profile configuration.

ProfileConfigType = "COMMON" / "ON_NETWORK" / "OFF_NETWORK" / text

### C.2.1.7	Media Type

The media type for a user profile document shall be "application/cbor".

Editor's Note:	Consider defining a media type for the user profile, such as "vnd.3gpp.seal-user-profile-info+cbor".

Editor's Note:	It is possible to specify other payload format for CoAP than CBOR, and the details about other payload format is FFS.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | CT1#120 | C1-196120 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2019-10 | CT1#120 |  |  |  |  | Implementing the following p-CR agreed by CT1: C1-196607, C1-196609, C1-196853, C1-196854 | 0.1.0 |
| 2019-11 | CT1#121 |  |  |  |  | Implementing the following p-CR agreed by CT1: C1-198620, C1-198815, C1-198816 | 0.2.0 |
| 2019-12 | CT-86 | CP-193153 |  |  |  | Presentation for information at TSG CT | 1.0.0 |
| 2020-02 | CT1#122-e |  |  |  |  | Implementing the following p-CR agreed by CT1: C1-200645, C1-200646, C1-200873, C1-200872, C1-200649, C1-201005, C1-200823 | 1.1.0 |
| 2020-03 | CT-87e | CP-200170 |  |  |  | Presentation for approval at TSG CT | 2.0.0 |
| 2020-03 | CT-87e |  |  |  |  | Version 16.0.0 created after approval | 16.0.0 |
| 2020-06 | CT-88e | CP-201129 | 0001 |  | B | SIP based subscribe/notify procedures for configuration management | 16.1.0 |
| 2020-06 | CT-88e | CP-201129 | 0002 | 1 | F | Removal of Editor’s notes. | 16.1.0 |
| 2020-06 | CT-88e | CP-201129 | 0003 |  | F | Corrections in HTTP request-uri value | 16.1.0 |
| 2020-06 | CT-88e | CP-201129 | 0004 |  | B | Adding IANA registration template for VAL user profile and UE configuration document | 16.1.0 |
| 2020-06 | CT-88e | CP-201129 | 0005 |  | F | Using proper element names in VAL UE Configuration | 16.1.0 |
| 2020-09 | CT-89e | CP-202163 | 0006 |  | D | Removing Heading level-7 as per drafting rules | 16.2.0 |
| 2021-09 | CT-93e | CP-212138 | 0007 | 1 | C | add VAL UE Information to configuration management procedure | 17.0.0 |
| 2021-12 | CT-94e | CP-213052 | 0008 | 1 | B | Addition of functional entity requirements for CoAP support | 17.1.0 |
| 2021-12 | CT-94e | CP-213052 | 0009 | - | B | Authenticated identity in CoAP request | 17.1.0 |
| 2021-12 | CT-94e | CP-213052 | 0010 | 1 | B | Addition of CoAP event subscription procedures | 17.1.0 |
| 2021-12 | CT-94e | CP-213052 | 0011 | 1 | B | Addition of CoAP notifications procedure | 17.1.0 |
| 2021-12 | CT-94e | CP-213052 | 0012 | 1 | B | Addition of CoAP VAL user profile data procedures | 17.1.0 |
| 2021-12 | CT-94e | CP-213052 | 0013 | 2 | B | Addition of CoAP Update VAL user profile data procedures | 17.1.0 |
| 2021-12 | CT-94e | CP-213052 | 0014 | 1 | B | Addition of CoAP resource representation and encoding | 17.1.0 |