| 3GPP TR 26.854V0.1.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects Study on Haptics in 5G Media Services (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	9

5	Use cases and requirements	9

5.1 	Introduction	10

5.2 	Haptic-enhanced Communication	10

5.3 	Immersive Entertainment	11

5.4 	Immersive multi-modal XR and metaverse 	13

5.X Use Case Template	14

6	Haptic input formats and device types	15

6.1	Introduction	15

6.2	Haptics input formats	15

6.2.1	Introduction	15

6.3	Haptics device types	15

6.3.1	Introduction	15

6.3.2	Haptics device type 1: Basic sensory feedback	15

6.3.3	Haptics device type 2: Sensorial texture feedback or Spatial sensory feedback	15

6.3.4	Haptics device type 3: Full-body and complex motion feedback	16

6.3.5	Haptics device type 4: Interactive and spatialised feedback	16

7 	Candidate technologies	16

7.1	Codecs	16

7.2	Storage format	16

7.3	Transport protocols	16

8	3GPP services	16

8.1 	5GMS	16

8.2	RTC	16

8.3	QoS	16

9	Conclusions and proposed next steps	16

Annex <A> (normative): <Normative annex for a Technical Specification>	17

Annex <B> (informative): <Informative annex for a Technical Specification>	18

B.1	Heading levels in an annex	18

Annex <B>: <Informative annex title for a Technical Report>	19

Annex <C> (informative): Bibliography	20

Annex <D> (informative): Index	21

Annex <X> (informative): Change history	22

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This clause is mandatory; do not alter the text in any way other than to choose between "Specification" and "Report".

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

This clause shall start on a new page.

The present document …

Editor’s note: SID objectives

- Identify relevant use cases and requirements already defined in TR 22.847, TR 22.856 related to the integration of haptics into media streaming and communication services, and refine them as necessary.

- -	Collect and document market-relevant device types and technologies (for example existing APIs)  providing support for haptic playback and/or capture.

- -	Collect and describe the candidate input and representation formats for haptic experience, relevant to the above use cases and device types.

- -	Define relevant QoE metrics for suitable haptics experiences

- -	Identify 3GPP streaming and communication services that may benefit from the haptics media type and identify the gaps and requirements need to integrate haptics experiences.

- -	Identify candidate technologies that address the gaps including digital representation and compression formats, transport formats and potential QoS requirements in context with the services.

- -	Provide potential needs and benefits for normative work

- .

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR22.847: “Study on supporting tactile and multi-modality communication services”

[3]	3GPP TR26.813: “Study of Avatars in Real-Time Communication Services”

[4]	3GPP TR22.856: “Study on Localized Mobile Metaverse Services”

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

AI	Artificial Intelligence

HMD	Head Mounted Display

MECS	Mobile Edge Computing Server

XR	Extended Reality

# 4	Background

Editor’s note: general intro to haptics media, characteristics, work done in SA1…

# 5	Use cases and requirements

The main text of the document should start here, after the above clauses have been added.

The following styles and editing techniques are aimed to help in the formatting of the document using the 3GPP Template: 3GPP_70.dot, available from the 3GPP FTP site (ftp://ftp.3gpp.org/Information).

## 5.1 	Introduction

The following use cases have been extracted from 3GPP TR22.847 [2], TR26.813 [3] and TR22.856 [4]. Only the use cases and requirements where Haptics can be used are considered and detailed. Use cases with similar haptics considerations have been grouped into one.

NOTE: The representative use-cases described in this section aimed at identifying the use, requirements and gaps when considering the support of haptics media signals in the 5GMS streaming and Communication architecture. Some of the functions of the described use-cases may not be fully studied.

Ed Notes: to be reviewed when all use cases are finalised.

Editor’s note 2: We may keep a summary of the scenarios in this section and move the tables with the details of all scenarios in an Annex.

## 5.2 	Haptic-enhanced Communication

Use Case Name:

Haptic-enhanced Communication From: TR 22.856 5.9 “Synchronized predictive avatars” TR 22.856 5.11 “IMS-based 3D Avatar Communication” TR 26.813 UC1 “Avatar Communication”

Description:

In this use case, three users are using the 5GS to join an immersive mobile metaverse activity (which may be an IMS multimedia telephony call using AR/MR/VR). The users Bob, Lukas, and Yong are located in the USA, Germany and China, respectively. Each of the users can be served by a local mobile metaverse service edge computing server (MECS) hosted in the 5GS, each of the mobile metaverse servers is located close to the user it is serving. In case of IMS such MECS could be an AR Media Function that provides network assisted AR media processing. When a user joins a mobile metaverse activity, such as a joint game or teleconference, the avatar of the user is loaded in the MECS of the other users. For instance, the MECS close to Bob hosts the avatars of Yong and Lukas. This use case identifies two fundamental scenarios and one sub-scenario for 3D Avatar Communication by means of the IMS. The intention of the proposal is to fully specify this system in 3GPP, to provide a standard for a new form of media to be used in telecommunication by mobile users. In the terminology of this use case, the avatar is a digital representation of a user, and this digital representation is exchanged (with other media, notably audio), with one or more users as mobile metaverse media. This use case is about one-to-one and multi-party communication with spatial audio rendering, where avatars and audio of each participant in avatar call are transmitted and spatially rendered in the direction of their geolocation. An avatar call is similar to a video call in that both are visual, interactive, provide live feedback to participants regarding their emotions, attentiveness and other social information. Each participant is equipped with display devices (phones, AR glasses, etc.) with external or built-in headphones. 3D audio can be captured and transmitted instead of mono, which leads to enhancements when sharing the audio experience. Editor’s note: the above is copy pasted from the references. Note 1: this can include help to people with disabilities. Note 2: Emotional haptic is relevant in this use case. Haptic modalities included here: mostly vibrotactile. Haptic characteristics: Includes 2-way communication (talks, social media, phone calls…) and 1-way (alerts messages, information…). Includes haptic emoji.

Categorization

Type: 2D/3D audio-video sessions Delivery: Conversational, Split Device: headphones, smartphones, smartwatches, simple wearables

Preconditions

- Terminal equipment capable of initiate a session. - Phone participant: for video and audio communication with associated haptic actuators for physical feedback. - Accessories to capture information and generate haptic effects.

Requirements

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering - Integration into Media applications - Integration into hardware devices - Availability of sensors to capture live information (accelerometer, pressure, temperature…) Connected devices in various form factors exist that are capable of decoding and rendering haptics (smartwatches, mobile phones, rings…).

Interoperability considerations

Haptic data should be distributed and rendered with various applications, environments and devices. Adaptation to the rendering capabilities should be supported. User adaptation could be provided.

Potential Standardization Status and Needs

- representation and coding (section 6 and 7) - Haptics data carriage, transport and streaming (section 7) - Integration in 5G architecture and services. (Section 8)

Table 5.2: use case Haptics-enhanced Communication

## 5.3 	Immersive Entertainment

Use Case Name:

Immersive entertainment (live events, sport, gaming, movies, music) From: TR 22.847 5.6 “Live Event Selective Immersion” TR 22.856 5.6 “Mobile Metaverse for Immersive Gaming and Live Shows” TR 22.856 5.22 “Mobile Metaverse Live Concert”

Description:

Media including here movies, TV series, social networks, gaming... The later could be multi-party or single user. In this scenario users are playing content (local or streamed) with video (2D or 3D immersive), audio (potentially spatialized) and haptics. The haptic signal is synchronized with the content to add physical feedback in the form of some haptic effects. The user is using at least a smartphone with vibrotactile actuators, or more advanced devices such as XR glasses and potentially multi wearable devices to spatialized haptic effects. Mobile metaverse services allow people to enjoy an online digital concert with their avatars beyond the limitation of time and space. In order to provide immersive interactive location agnostic service experience to mobile metaverse service customers, large amount of computing resources is needed to perform real-time processing for audio, video, and interactive data, etc. The thing to realize here is that different customers will use terminals e.g. XR glasses with different brands and different processing capabilities, some of the glasses will not have enough computing resources to perform the real-time rendering. Through split rendering, most of the computing work task can offload to the network, the high speed and low latency transmission provided by 5G system can cooperate with the edge cloud side for real-time rendering, and combine with the local optimized rendering of the XR terminal side to provide the immersive and unbounded XR experience. In addition to this, similar to the real world, people are more likely to watch a concert together with their friends, further, the mobile metaverse live concert service is also provides private boxes for group of avatars to enjoy the concert privately, and different types of social authority can be provided in the private box on demand. Ed Notes: the above text is copy pasted from the references. Haptic modalities included here: mostly thermal, kinesthetic and vibrotactile, but acceleration can be used for some events Haptic characteristics: Mono-directional, low data-rates.

Categorization

Type: VR, AR, mobile, TV Delivery: Device: HMD, headphone, cushion, chair, VR controller, mobile phone

Preconditions

- Terminal equipment capable of initiating and playing an event session. - Capture sensors for feedback from the live events - Participant: Haptic actuators, displays and headphones. - Phone participant: Phone with haptic actuators (generally vibrotactile).

Requirements

Audio-Tactile delay = 50 ms Tactile-Audio delay = 25 ms Visual-Tactile delay = 15 ms Tactile-Visual delay = 50 ms Maximal end-to-end latency = 5ms (esp. for gaming) Reliability = 99.9% Bit-rates = In the range of 8kbps-16kbps for reasonable quality (32-64 kbps for high quality) per channel. Number of Channels = between 1 and 20+ with non-continuous packets. Live applications could be continuous if direct sensors information is recorded and distributed. Format: both signal (time samples) and parametric.

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering - Editing tools to create Haptic effects - Integration into Media applications - Integration into hardware devices - Availability of sensors to capture live information (accelerometer, pressure, temperature…) Connected devices in various form factors exist that are capable of decoding and rendering haptics (cushion, controllers, suits…). On site production units can ingest sensors signals and integrate synthesizers to generate haptic effects directly in the production vans for live haptics.

Interoperability considerations

Haptic data should be delivered and rendered with various applications, environments and devices. Adaptation to the rendering capabilities should be supported. User adaptation could be provided.

Potential Standardization Status and Needs

- Haptics data representation and coding: (Section 6 and 7) - Haptics data carriage, transport and streaming (Section 7) - Integration in 5G architecture and services. (Section 8)

Table 5.3: Use-case - Immersive Entertainment

## 5.4 	Immersive multi-modal XR and metaverse

Use Case Name:

Immersive multi-modal XR and metaverse (VR, AR, multi-users gaming) From: TR 22.847 5.1 “Immersive multi-modal Virtual Reality (VR) application” TR 26.813 UC3 “Multi-user Gaming” TR 22.856 5.7 “AR Enabled Immersive Experience” TR 22.856 5.12 “Virtual humans in metaverse”

Description:

Immersive multi-modal VR application describes the case of a human interacting with virtual entities in a remote environment such that the perception of interaction with a real physical world is achieved. Users are supposed to perceive multiple senses (vision, sound, touch) for full immersion in the virtual environment. Virtual humans (or digital representations of humans, also referred to as 'avatars' in this use case) are simulations of human beings on computers. There is a wide range of applications using avatars, such as games, film and TV productions, financial industry (smart adviser), telecommunications (avatars), etc. In the coming era, the technology of virtual humans is one of foundations of mobile metaverse service. A virtual human can be a digital representation of a natural person in a mobile metaverse service, which is driven by the natural person, or a virtual human also can be a digital representation of a digital assistant driven by AI model. Mobile metaverse services offer an important opportunity for socialization and entertainment, where user experience of the virtual world and the real world combine. This use case focuses on the scenario of a natural person's digital embodiment in a metaverse as a location agnostic service experience. A virtual human is customized according to a user's personal characteristics and shape preferences. Users wear motion capture devices, vibrating backpacks, haptic gloves, VR glasses to drive the virtual human in a meta-universe space for semi-open exploration. The devices mentioned above are 5G UEs, which need to collaborate with each other to complete the actions of user and get real-time feedback. Ed Notes: the above is copy-pasted from the references. Haptic modalities included here: force, motion, thermal and vibrotactile. Haptic characteristics: Bi-directional, interactive, spatialized, multi-users, low latency, high data-rates.

Categorization

Type: VR /AR Delivery: Device: HMD, glove, suit, VR controller, motion platform

Preconditions

- devices that support haptic modalities - VR/AR/metaverse platform to manage sessions

Requirements

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering - Editing tools to create Haptic effects - Integration into Media applications - Integration into hardware devices Connected devices in various form factors exist that are capable of decoding and rendering haptics (HMD, gloves, controllers, suits…).

Interoperability considerations

Haptic data should be able to be transmitted and rendered with various applications, environments and devices. Adaptation to the rendering capabilities should be supported. User adaptation could be provided.

Potential Standardization Status and Needs

- Haptics data representation and coding (section 6 and 7) - Haptics data carriage, transport and streaming (section 7) - Integration in 5G architecture and services. (Section 8)

Table 5.3: use case Immersive multi-modal XR and metaverse

## 5.X Use Case Template

Use Case Name:

xxx

Description:

xxx

Categorization

Type: Delivery: Device: Other:

Preconditions

xxx

Requirements

xxx

Feasibility

xxx

Interoperability considerations

xxx

Potential Standardization Status and Needs

xxx

# 6	Haptic input formats and device types

## 6.1	Introduction

Haptics refers to the sense of touch, and encompasses the generation, manipulation, and perception of tactile sensations, forces, and motions. In the media domain it relates to the capture and rendering of physical information about objects and environments due to interactions with the user. The capture is generally performed with sensors and the rendering with hardware actuators allowing to render different modalities.

In general, three types of modalities are considered, tactile (vibration, temperature, pression), kinaesthetic (force) and proprioception (motion, acceleration). Those modalities are rendered in the human body through different mechanoreceptors. The density and properties of those receptors are different depending on the modality and the body parts. In addition, each individual has different sensibility, which can be managed through users’ profiles and adaptation.

Finally, the rendering is performed with specific hardware actuators embedded into user devices. The different Haptic modalities are rendered with different actuators. Typically, tactile sensations can be rendered with vibrotactile (e.g. linear resonant actuators (LRA), eccentric rotating mass (ERM), temperature (piezzo), wind (fan), force) … devices.

## 6.2	Haptics input formats

### 6.2.1	Introduction

Haptic input formats are essential in capturing and transmitting tactile sensations in a variety of applications, ranging from simple vibrations in mobile devices to complex force feedback in virtual reality systems. Two types of input formats exist, time-sampled signals (typically PCM encoded) captured by sensors, and parametric description of haptic effects.

## 6.3	Haptics device types

### 6.3.1	Introduction

Haptics devices may be categorized into types defining their primary capability. Each category reflects the device's intended application and the type of haptic experience it provides.

### 6.3.2	Haptics device type 1: Basic sensory feedback

Devices in this category are designed to provide passive, non-spatialised sensory feedback, in applications such as simple mobile game and entertainment, alerting, or communication. They include smartphone, smartwatch, wearables, finger UI, headphone, HMD. They usually have a low number of actuators

### 6.3.3	Haptics device type 2: Sensorial texture feedback or Spatial sensory feedback

Devices in this category are designed to deliver highly detailed and precise haptics (e.g., tactile, kinaesthetic, proprioception) sensations. These devices simulate the feel of textures, pressure, and other subtle interactions, enabling users to experience intricate touch-based feedback. They are commonly used in applications where detailed touch interaction is crucial, such as virtual reality gloves or touch-sensitive surfaces. Fine-tuned sensory feedback devices include Haptic gloves and Haptic touchpads and screens.

### 6.3.4	Haptics device type 3: Full-body and complex motion feedback

Devices in this category provide immersive, whole-body sensations. These devices use a network of actuators to simulate a wide range of physical experiences, such as vibrations, impacts, and movements across the body. They are ideal for enhancing immersion in virtual environments or simulation scenarios by delivering comprehensive, multi-sensory feedback. Haptics suits and furniture such as racing game seats, motion platforms and simulators are example of Full-Body and complex feedback devices.

### 6.3.5	Haptics device type 4: Interactive and spatialised feedback

Devices in this category are focused on providing targeted, spatialised haptics (e.g., tactile, kinaesthetic, proprioception) feedback. These devices simulate specific actions or interactions, such as button presses or in-game effects, through vibrations or adaptive triggers. This category includes haptic controllers such as game console controllers offering localized feedback through vibration motors and adaptive triggers to simulate various in-game sensations and interactions.

# 7 	Candidate technologies

Editor’s note:  existing standards/work done in other SDOs.

## 7.1	Codecs

## 7.2	Storage format

## 7.3	Transport protocols

# 8	3GPP services

## 8.1 	5GMS

## 8.2	RTC

## 8.3	QoS

# 9	Conclusions and proposed next steps

xxxx

Annex <A> (normative):
<Normative annex for a Technical Specification>

Start each annex on a new page.

Annexes are labelled A, B, C, etc. and designated either "normative" or "informative" depending on their content.

Normative annexes only to appear in Technical Specifications. Use style "Heading 8".

Annex <B> (informative):
<Informative annex for a Technical Specification>

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

# B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Annex <B>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <C> (informative):
Bibliography

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Bibliography is optional. If it exists, it shall follow the last technical annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

<Publication>: "<Title>".

Annex <D> (informative):
Index

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Index is optional. If it exists, it shall immediately precede the Changes history annex.

Generate the index using MS Word's index field feature.

Annex <X> (informative):
Change history

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | S4-129e |  |  |  |  | Proposed skeleton TR | 0.0.1 |
| 2024-08 | S4-129e |  |  |  |  | Includes: Use Cases for Immersive entertainment (S4-241506), Haptics-enhanced Communication (S4-241771) Immersive multi-modal XR and metaverse (S4-241770); Device types (S4-241761); input formats (S4-241769) Editor’s clean-up | 0.1.0 |