| 3GPP TS 28.572 V0.2.0 (2024-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management of planned configurations (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concept and overview	6

5	Requirements	6

6	Solution description	8

6.1	Planned configurations	8

6.1.1	Definition	8

6.1.1	Format of planned configurations	9

6.1.2	Meta data for planned configurations	9

6.1.3	Planned configuration descriptor	9

6.2	Planned configuration groups	9

6.2.1	Definition	9

6.2.2	Planned configuration group members	10

6.2.2	Conflicts between planned configuration group members	10

6.2.3	Meta data for planned configuration groups	10

6.2.4	Planned configuration group descriptor	10

6.2a	Conditions	10

6.3	Validation of planned configurations	11

6.3.1	Definition	11

6.3.2	Potential problems of planned configurations	11

6.3.3	Validation process	12

6.3.4	Validation of multiple planned configurations	12

6.4	Activation of planned configurations	12

6.4.1	Definition	12

6.4.2	Potential problems of planned configurations	12

6.4.4	Activation of multiple planned configurations	12

6.4.5	Activation control parameters	12

6.5	Conditional activation of planned configurations	13

6.6	Asynchronous updates	13

6.7	Fallback	13

6.8	Planned configuration lifecycle	13

6.9	Managing planned configurations	13

6.10	Managing planned configuration groups	13

6.10a	Managing conditions	14

6.11	Managing validations	15

6.12	Managing activations	16

6.13	Notifications	17

7	Model	17

Annex A (normative):  Solution sets	17

A.1	RESTful HTTP-based solution set	17

Annex B (informative): Change history	21

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[3]	IETF RFC 8072: " YANG Patch Media Type".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Concept and overview

# 5	Requirements

Requirements related to planned configurations

Req-PCG-1: The 3GPP management system shall support creating, reading and deleting planned configurations for managed systems.

Req-PCG-2: The 3GPP management system should support updating planned configurations for managed systems.

Req-PCG-3: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configurations.

Requirements related to control parameters for validation and activation of planned configurations

Req-PCC-1: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer if validation and activation shall be processed in best effort or atomic mode.

Requirements related to planned configuration groups

Req-PGG-1: The 3GPP management system should allow MnS consumers to group planned configurations into planned configuration groups.

Req-PGG-2: The 3GPP management system should allow MnS consumers to group planned configuration groups into planned configuration groups.

Req-PGG-3: The 3GPP management system shall support creating, reading and deleting planned configuration groups.

Req-PGG-4: The 3GPP management system should support updating planned configuration groups.

Req-PGG-5: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configuration groups.

Requirements related to control parameters for validation and activation of planned configuration groups

Req-PGC-1: The 3GPP management system shall support creating, reading, updating, and deleting control parameters for planned configuration groups.

General plan validation requirements

Req-PVG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to validate a planned configuration.

Req-PVG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan validation.

Req-PVG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan validation.

Req-PVG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan validation.

Req-PVG-5: The 3GPP management system should allow for multiple sequential validation of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously validated.

Requirements related to the validation of multiple plans

Req-PVM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to validate multiple planned configurations with a single request.

Req-PVM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be validated with a single request.

Req-PVM-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the behavior if plan conflicts are detected, for the case where multiple plans are requested to be validated with a single request.

General plan activation requirements

Req-PAG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to activate a planned configuration.

Req-PAG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan activation.

Req-PAG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan activation.

Req-PAG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan activation.

Req-PAG-5: The 3GPP management system should allow for multiple sequential activations of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously activated.

Req-PAG-7: The 3GPP management system shall validate a planned configuration automatically when activation of the planned configuration is requested without prior validation.

Requirements related to control parameters for activation

Req-PAC-1: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if activation should be done with the least possible service impact or within the shortest possible time.

Req-PAC-2: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if fallback shall be enabled.

Requirements related to the activation of multiple plans

Req-PAM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to activate multiple planned configurations with a single request.

Req-PAM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be activated with a single request.

Req-PAM-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the behavior if plan conflicts are detected, for the case where multiple plans are requested to be activated with a single request.

Requirements related to conditional activation of plans

Req-CAP-1: The 3GPP management system should allow MnS consumers to create conditions on MnS producers that specify when planned configurations are activated.

# 6	Solution description

## 6.1	Planned configurations

### 6.1.1	Definition

The current configuration is the configuration currently used by the managed system. It is represented by (a tree of) configuration data nodes. State data nodes are not included. The format of the configuration data node tree is defined by (stage 2) NRM definitions and (stage 3) schema definitions.

Editor's note: The name of the current configuration is tbc. May also be actual configuration or operational configuration.

Note that the data node tree representing a manged system typically comprises besides configuration data nodes also (read-only) state data nodes, that represent the current state of the managed system. Configuration data nodes and state data nodes may be separated in dedicated subtrees, but they may be also contained in one tree.

A planned configuration is a configuration that can be manipulated without impacting the current configuration of the system. It relates to configuration data nodes and not to state data nodes. It is maintained side-by-side with the current configuration.

### 6.1.1	Format of planned configurations

A planned configuration is represented by a set of operations to be applied to the current configuration. The operations are described by the input parameters of the "changeMOIs" operation specified in TS 28.532, with the addition that each operation in the operation set has a key for identifying the operation. The operations in the operation set are applied to the current configuration in the order as they appear in the operation set.

Editor's note: It is ffs if this strict requirement on the order should be relaxed, for example with "Operations affecting the same data nodes SHALL be applied to the current configuration in the order as they appear in the planned configuration. Operations affecting different data nodes MAY be executed in any order. The producer SHOULD ensure that dependencies like containment relationships (parent-child) are handled in an appropriate order.".

The following guidelines may ease the processing of operation sets during validation and activation:

- Parent objects should be created before child objects.

- Objects that are referenced should be created before the reference.

### 6.1.2	Meta data for planned configurations

Each planned configuration is described by meta data. This includes the name, the version, a user label, and a human readable textual description. It is also possible to specify additional MnS consumer defined properties (key value pairs) to further describe and qualify the planned configuration, for example to specify who created the planned configuration.

Furthermore, it includes the content type of the planned configuration and a pointer to the schema describing the data node tree of the current configuration.

Editor's note: The details of the pointer to the schema is ffs.

Furthermore, it is specified if the operations of the operations set are an atomic set, where all operations need to be validated and activated successfully, or if some operations are allowed to fail (non-atomic or best effort set). In other words, for atomic sets, all operations of the set must be processed successfully or no operation at all. For best effort sets, the operations that can be processed successfully are processed, and those that for whatever reason cannot be processed successfully are not processed.

Editor's note: "stop-on-error" shall be added.

For example, subscriptions to alarm notifications on multiple Network Functions may have value also in case the subscription on some Network Functions fails. When analytics are computed based on measurements collected at some well selected Network Functions, then all measurement collection jobs must be created. If one job cannot be created it does not make sense to collect the remaining measurements.

The date and time at which information in the planned configuration was modified the last time is provided as well.

### 6.1.3	Planned configuration descriptor

The operation set and related meta data are specified in a planned configuration descriptor. Each descriptor has a unique identifier.

## 6.2	Planned configuration groups

### 6.2.1	Definition

A planned configuration group is a set of planned configurations that shall be processed (validated and activated) together.

For example, one plan may create a new cell, and another plan may start the collection of measurements for the new cell. Or one plan that configures one base station and another plan configures another base station, when both base stations need to be consistently configured to enable handovers between them.

### 6.2.2	Planned configuration group members

A planned configuration belonging to a planned configuration group is also called a member of that planned configuration group. A planned configuration can be a member of more than one planned configuration group.

The members of a planned configuration group shall be considered as atomic or non-atomic (best effort), in the same way as the operations of an operation set may form an atomic or a non-atomic (best effort) set.

Furthermore, the members of a planned configuration group can be ordered or not ordered. If ordered, the planned configuration group members are processed in the order as specified in the planned configuration group. If not ordered, the members can be processed in any order or in parallel.

For example, when the configuration for a new Network Function is in one plan and the creation of a performance metric collection job in another plan, then the plans must be ordered (assuming the job object is contained under a configuration object).

Not ordered are for example plans adding performance metric collection jobs to already configured Network Functions.

Editor's note: A planned configuration group should be allowed to be a member of a planned configuration group. This is ffs.

### 6.2.2	Conflicts between planned configuration group members

Conflicting operations may be contained in planned configuration group members. For example, a planned configuration sets an attribute to some value and another planned configuration sets the same attribute to some other value.

If the configuration group members are ordered, the result is deterministic. The conflicting operations are processed in the order as they appear in the ordered plans. The last processed operation from the ordered set of operations manipulating the same node determines the result. If the configuration group members are not ordered, the result is not deterministic. Depending on which operation from the unordered set of operations manipulating the same node is processed last, the result may be different.

It is configurable if a detected conflict should invalidate the planned configuration group.

### 6.2.3	Meta data for planned configuration groups

Each planned configuration group is described by meta data. This includes the name, the version, and a human readable textual description.

Furthermore, it includes the information if the group members are atomic or non-atomic, and if they are ordered or unordered.

The date and time at which information in the planned configuration group was modified the last time is provided as well.

### 6.2.4	Planned configuration group descriptor

The planned configuration group members and its meta data are specified in a planned configuration group descriptor. Each descriptor has a unique identifier.

## 6.2a	Conditions

Conditions are provided for conditional activation of planned configurations. The planned configuration is activated automatically when certain conditions are met.

For example, additional performance metric collection jobs or assurance functions shall be created when metrics that are always collected cross certain thresholds.

The trigger for activation is specified as follows:

- a condition expression evaluating to true or false

- the information if the trigger is the transition from false to true or true to false

- the information, if the transition from false to true or true to false triggers activation only once or every time the transition occurs

The condition expression works on the data node tree of the current configuration.

Editor's note: Scope and content of the condition expression is ffs.

Editor's note: The situation where the evaluation result of the condition expression toggles between "True" and "False" needs furher investigation to avoid repititive activations of the same planned configuration.

## 6.3	Validation of planned configurations

### 6.3.1	Definition

Planned configurations may have problems and it may be impossible to activate them. Validation is the process to unveil potential problems with planned configurations before activation of the planned configuration.

The validation function has two input arguments: the operation set representing the planned configuration and a copy of the current configuration. The planned configuration is applied to the copy of the current configuration. The resulting configuration data node tree must comply to the (stage 2) NRM definitions and (stage 3) NRM schema definitions.

### 6.3.2	Potential problems of planned configurations

Validation discovers problems of planned configuration. These problems may be classified as follows:

- Information model problems

  - Problems of the planned configuration itself

  - Problems of the planned configuration in relationship to the current configuration

- Application layer problems

Examples for problems of the planned configuration itself:

- Update of an attribute with a value does not match the data type defined for the value of the attribute

- Creation of an attribute name value pair that is not defined

- Update of an existing attribute that is defined as read-only

- Creation of an object with an object class that is not defined

- Creation of an object with a representation does not match the object class defined for the object

Examples for problems of the planned configuration in relationship to the current configuration:

- Creation of objects under parents that do not exist

- Deletion of leaf objects that do not exist

- Deletion of objects that are no leaves

- Violation of object cardinality constraints

- Violation of the multiplicity property of an attribute

Note that some operations may result inevitably in intermediate states that do not comply to the constraints defined by the NRM schema. For example, an object of class A shall contain always exactly five objects of class B. To replace an object of class B, the object to be replaced is deleted in the first step. In the second step the new object is added. The other possibility is to add the new object first and delete the object to be removed afterwards. In both cases the cardinality has a temporary invalid state. The validation process needs to take this into account. In general, only after applying the complete planned configuration to the current configuration, the resulting data node tree must comply to all constraints defined in the information model.

Examples for application layer problems

- An attribute value in the planned configuration is in the value range allowed by the attribute properties but the value is not allowed in the current state of the application.

### 6.3.3	Validation process

The purpose of validation is to detect problems of the planned configuration. As described in the previous clause, problems can be categorized into three groups.

Problems of the operations itself are detected by analyzing the operations together with the NRM schema. The current configuration is not involved in this process. Problematic operations are always invalid. If an atomic operation set contains at least one invalid operation, the complete planned configuration is invalid. If a non-atomic operation set contains a bad operation, only the operation is invalid. The operation set with the other operations may still be valid, depending on the validation result for the other operations.

Problems of the planned configuration in relationship to the current configuration are detected by applying the operations of the operation set one by one to a copy of the current configuration. This may result in intermediate states that do not comply to the constraints defined by the NRM schema.

For example, an object of class A shall contain always exactly five objects of class B. To replace an object of class B, the object to be replaced is deleted in the first step. In the second step the new object is added. The other possibility is to add the new object first and delete the object to be removed afterwards. In both cases the cardinality has a temporary invalid state.

Only after processing all operations, all constraints defined in the NRM schema must be respected. All constraint violations shall be reported. In atomic mode the complete validation shall succeed or if any constraints are violated the complete validation fails. In best effort mode the complete validation may fail, may partially succeed or succeed depending on the presence of constraint violations (or their absence).

### 6.3.4	Validation of multiple planned configurations

## 6.4	Activation of planned configurations

### 6.4.1	Definition

Activation is the process of applying a planned configuration to the current configuration (instead of to a copy of the current configuration as for validation). This process can take a bit of time.

### 6.4.2	Potential problems of planned configurations

For activation the validation considerations apply.

### 6.4.4	Activation of multiple planned configurations

For activation the validation considerations apply.

### 6.4.5	Activation control parameters

Editor's note: Explain "enable-fallback", "service-impact and "immediate-activation". Also "cancel-request".

## 6.5	Conditional activation of planned configurations

## 6.6	Asynchronous updates

## 6.7	Fallback

## 6.8	Planned configuration lifecycle

## 6.9	Managing planned configurations

Planned configuration descriptors are created, updated and deleted by MnS consumers on MnS producers.

Multiple planned configuration descriptors can be maintained on a MnS producer. Each planned configuration is identified by an identifier. The identifier is assigned by the MnS producer.

The following table specifies the information elements of a planned configuration descriptor.

| Information element name | Description |
| --- | --- |
| pcd-id | The identifier of the planned configuration. |
| name | The name of the planned configuration. |
| version | The version of the planned configuration. |
| user-label | The user label of the planned configuration. |
| description | The textual human-readable description of the planned configuration. |
| additional-properties | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. |
| configuration-content-type | The format of the planned configuration. |
| current-conf-schema | The pointer to the schema of the current configuration data node tree. Editor's note: Details are ffs. |
| base-object-address | The address of the base object instance that together with the path offsets specified in the operations identifies the data nodes to be modified. |
| operation-set-is-atomic | The indication if the operations in the operation set are atomic. |
| last-modified | The date and time at which the planned configuration descriptor was modified the last time. |
| operation-set | The operation set. |

## 6.10	Managing planned configuration groups

Planned configuration group descriptors are created, updated and deleted by MnS consumers on MnS producers. Planned configurations referred to in a planned configuration group must exist.

Multiple planned configuration group descriptors can be maintained on a MnS producer. Each planned configuration group is identified by an identifier. The identifier is assigned by the MnS producer.

The following table specifies the information elements of a planned configuration group descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| pcgd-id | M | The identifier of the planned configuration group descriptor | Type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| name | M | The name of the planned configuration group. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: None |
| version | M | The version of the planned configuration group. Format is implementation specific. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| description | M | The textual human-readable description of the planned configuration group. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: None |
| additional-properties | O | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | Type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None |
| pcd-ids | M | The identifiers of the planned configurations that are members of the planned configuration group. | Type: String multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None |
| pcg-is-atomic | M | Specifies if the members of the planned configuration group are atomic. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: True |
| pcg-is-ordered | M | Specifies if the members of the planned configuration group are ordered. When ordered, the planned configuration group members shall be validated/activated in the order specified. When not ordered the planned configuration group members can be validated/activated in any order. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: True |
| fail-on-conflict | M | Specifies if the activation shall fail on detection of conflicts, or if the operations shall be processed as if there were no conflicts. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: True |
| last-modified | M | The date and time at which the planned configuration group descriptor was modified the last time. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isWritable: False |

## 6.10a	Managing conditions

Conditions are created, updated and deleted by MnS consumers on MnS producers.

The following table specifies the information elements of a condition

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| condition-expression | M | The condition expression. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| condition-trigger | M | The condition trigger. allowedValues: - FALSE_TO_TRUE - TRUE_TO_FALSE | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| trigger-only-once | M | The boolean indication, if the trigger is disarmed after the first firing. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| activation-jobs | M | The address of one or more activation jobs that shall be triggered by the condition. | Type: String multiplicity: 1...* isOrdered: False isUnique: True defaultValue: None |

## 6.11	Managing validations

Validation of a planned configuration is requested by MnS consumers and performed by MnS producers. To request validation the MnS consumer needs to create a validation job on the MnS producer. Each job is identified by an identifier. The identifier is assigned by the MnS producer.

The planned configuration to be validated is specified by including the identifier of the corresponding planned configuration descriptor in the job creation request. Alternatively, for validating a planned configuration group, a planned configuration group descriptor can be specified.

In case no planned configuration descriptor or no planned configuration group descriptor has been created for the planned configurations to be validated, a descriptor may be specified in the validation job creation request.

When only one planned configuration is requested to be validated no control parameter needs to be specified in the request. For validating planned configuration groups the "fail-on-conflict" parameter needs to be specified.

The MnS producer adds the following information elements to the job: "state", "start-time", "stop-time", "result", and "error-details".

The information element "error-details" is structured with the following properties:

-	The mandatory "type" property that provides high level error information.

-	The optional "title" that provides a short, human-readable summary of the problem type. It shall not change from occurrence to occurrence of the problem.

-	The optional "reason" property" that provides more details on the error conditions than the "type" property.

-	The mandatory "badOp" property that specifies the operation, that cannot be satisfied.

Further details can be found in TS 32.158.

The following table specifies the information elements of a validation job.

| Information element name | Description |
| --- | --- |
| job-id | The identifier of the validation job. |
| name | The name of the validation job. |
| description | The textual human-readable description of the validation job. |
| pcd-id | The identifier of the planned configuration descriptor, whose operation set is requested to be validated. |
| pcgd-id | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be validated. |
| pcd | or, alternatively, the planned configuration descriptor to be activated, if no planned configuration descriptor was created earlier. |
| pcgd | or, alternatively, the planned configuration group descriptor to be validated, if no planned configuration group descriptor was created earlier. |
| job-state | The validation job state, either "NOT_STARTED", "RUNNING", "CANCELLED" or "COMPLETED". Editor's note: It is ffs if more detailed state and progress info needs to be added |
| start-time | The date and time at which the validation process started. |
| stop-time | The date and time at which the validation process stopped. |
| result | The validation result, either "VALIDATED" or "VALIDATION_FAILED". When the activated operation set is non-atomic or the activated planned configuration group is non-atomic, also "VALIDATION_PARTIALLY_FAILED". |
| error-details >> type >> reason >> title >> bad-operation | In case validation failed or partially failed, error reasons are specified here. |

## 6.12	Managing activations

Activation of a planned configuration is requested by MnS consumers and performed by MnS producers. To request activation the MnS consumer needs to create an activation job on the MnS producer. Each job is identified by an identifier. The identifier is assigned by the MnS producer.

The planned configuration to be activated is specified by including the identifier of the corresponding planned configuration descriptor in the job creation request. Alternatively, for validating a planned configuration group, a planned configuration group descriptor can be specified.

In case no planned configuration descriptor or no planned configuration group descriptor has been created for the planned configurations to be activated, a descriptor may be specified also directly in the activation request.

Furthermore, the following control parameters need to be specified in the job creation request: "enable-fallback", and "service-impact".

The MnS producer adds the following information elements to the job: "state", "start-time", "stop-time", "result", and "error-details".

The following table specifies the information elements of an activation job.

| Information element name | Description |
| --- | --- |
| job-id | The identifier of the activation job. |
| name | The name of the activation job. |
| description. | The textual human-readable description of the activation job. |
| pcd-id | The identifier of the planned configuration descriptor, whose operation set is requested to be activated. |
| pcgd-id | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be activated. |
| pcd | or, alternatively, the planned configuration descriptor to be activated, if no planned configuration descriptor was created earlier. |
| Pcgd | or, alternatively, the planned configuration group descriptor to be activated, if no planned configuration group descriptor was created earlier. |
| enable-fallback | This boolean attribute allows to specify if the MnS producer shall create a plan with the current configuration to allow for a later fallback. Default value is "False", e.g. no fallback is enabled. The plan-id of the fallback plan shall be returned to the MnS consumer. |
| service-impact | The service impact mode, either "LEAST_SERVICE_IMPACT" or "SHORTEST_TIME". |
| immediate-activation | This Boolean attribute specifies if the activation job shall start immediately or, alternatively, by conditional activation. |
| cancel-request | This boolean attribute allows to request to cancel the activation process. |
| job-state | The activation job state, either "NOT_STARTED", "RUNNING", "CANCELLED" or "COMPLETED". Editor's note: It is ffs if more detailed state and progress info needs to be added |
| start-time | The date and time at which the activation process started. |
| stop-time | The date and time at which the activation process stopped. |
| result | The activation result, either "ACTIVATED" or "ACTIVATION_FAILED". When the activated operation set is non-atomic or the activated planned configuration group is non-atomic, also "ACTIVATION_PARTIALLY_FAILED". |
| error-details >> type >> reason >> title >> bad-operation | In case validation failed or partially failed, error reasons are specified here. |

Editor's note: Default values for attributes should be specified whenever possible.

## 6.13	Notifications

# 7	Model

Annex A (normative): 
Solution sets

## A.1	RESTful HTTP-based solution set

Format of planned configurations

A planned configuration might target MnS producers whose data node tree is described using YANG or OpenAPI.

When described in OpenAPI, the value of the "configuration-content-type" is either "application/vnd.3gpp.json-patch+json" defined in TS 32.158 [2] or "application/json-patch+json".

When described in YANG, the value of the "configuration-content-type" is "application/yang-patch+json", defined in RFC 8072 [3].

A single planned configuration shall only include one configuration-content-type. If some items are modelled in YANG while others in OpenAPI these should be placed in separate planned configurations and made members of a plan configuration group.

Editor's note: Can the content types above be used given that a key is added for identifying an operation in the operation set?

Note that JSON Patch is not used together with HTTP Patch. Therefore, the atomicity and synchronicity properties of HTTP Patch do not apply here.

CRUD operations

Resource structure

The resource structure on the MnS producer is as follows:

…/{MnSName}/{/MnSVersion}/mopc/plan-descriptors/{id}

…/{MnSName}/{/MnSVersion}/mopc/plan-group-descriptors/{id}

…/{MnSName}/{/MnSVersion}/mopc/plan-validation-jobs/{id}

…/{MnSName}/{/MnSVersion}/mopc/plan-activation-jobs/{id}

Editor's note: When aligned with the guidelines in TS 32.158 then the resource structure looks like, which is ffs

…/{MnSName}/{/MnSVersion}/mopc={id}/plan-descriptor={id}

…/{MnSName}/{/MnSVersion}/mopc={id}/plan-group-descriptor={id}

…/{MnSName}/{/MnSVersion}/mopc={id}/plan-validation-job={id}

…/{MnSName}/{/MnSVersion}/mopc={id}/plan-activation-job={id}

Editor's note: The exact name for "mopc" is ffs, might be also "plan-management".

Creating, updating and deleting planned configurations

The data node tree on the MnS producer is as follows upon system start up.

{ "mopc": { "plan-descriptors": {}, "plan-group-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} } }

A new item of the collection resource "mopc/plan-descriptors" is created by MnS consumers using HTTP POST.

POST 3gpp/ProvMnS/1900/mopc/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json { "meta-data": { "name": "NewBts10Plan", "version": "2.0", "user-label": "Plan-created-by-Fred-for-rollout-in-area-xy.", "description": "This is the plan for the new BTS 10.", "operation-set-is-atomic": true, "configuration-content-type": "application/vnd.3gpp.json-patch+json", "current-conf-schema": "http://example.org/3gpp/ProvMnS/1900/schemas", "current-conf-address": "http://example.org/3gpp/ProvMnS/1900", }, "operation-set": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The MnS producer allocates the identifier "p1" for the new resource and returns the response. The location header contains the URI of the new resource. The response body contains the representation of the new resource which is equal to the representation received in the request with the "last-modified" property added.

HTTP/1.1 201 Created Date: Wed, 21 Aug 2024 15:39:57 GMT Location: http://example.org/3gpp/ProvMnS/1900/mopc/plan-descriptors/p1 Content-Type: application/json { "meta-data": {...}, "operation-set": [...] }

The resource structure on the MnS producer contains the new resource.

{ "mopc": { "plan-descriptors": { "p1": { "meta-data": { "name": "NewBts10Plan", "version": "2.0", "user-label": "Plan-created-by-Fred-for-rollout-in-area-xy.", "description": "This is the plan for the new BTS 10.", "operation-set-is-atomic": true, "configuration-content-type": "application/vnd.3gpp.json-patch+json", "current-conf-schema": "http://example.org/3gpp/ProvMnS/1900/schemas", "current-conf-address": "http://example.org/3gpp/ProvMnS/1900", "last-modified": "2024-08-21T16:39:57-01:00" }, "operation-set": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] } }, "plan-group-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} } }

The next example shows how the value of the "location" attribute in the operation can be changed from "Castle Charlottenburg" to "Summer palace Charlottenburg".

PATCH 3gpp/ProvMnS/1900/plans/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/operation-set/0/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

To delete the planned configuration "p1" a MnS consumer might send.

DELETE 3gpp/ProvMnS/1900/mopc/plan-descriptors/p1 HTTP/1.1 Host: example.org

In case of success, the MnS producer returns the following message.

HTTP/1.1 204 No Content Date: Wed, 21 Aug 2024 16:12:45 GMT

The next example shows how a planned configuration can be created for the case where the current configuration data node tree is specified with YANG.

POST 3gpp/ProvMnS/1900/mopc/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "planxyz", "activation-mode" : "BEST_EFFORT", # Execute in a best effort manner trying all operations "configuration-content-type" : "application/yang-patch+json" "configuration": { "ietf-yang-patch:yang-patch" : { "patch-id" : "NRCellDU", "edit" : [ { "operation": "create", "edit-id" : "opId-001", "target": "/_3gpp-common-subnetwork:SubNetwork=Irl/3gpp-common-mecontext:MeContext=Dublin-1/3gpp-common-managed-element:ManagedElement=Dublin-1/3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1", "value": { "NRCellDU": [ { "id": "1", "attributes": { "typeISinglePanelRiRestriction": "15", "ailgDlPrbLoadLevel": "0", "drxOnDurationTimer": "ONDURATIONTIMER_8MS", "trsResourceShifting": "DEACTIVATED", "ulStartCrb": "0", ... } } ] } } } }

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | SA5#157 | S5-245426 |  |  |  | Input Draft TS 28.572 Management of planned configurations | 0.0.0 |
| 2024-10 | SA5#157 | S5-246080 |  |  |  | pCR 28.572 Add skeleton | 0.1.0 |
| 2024-10 | SA5#157 | S5-246081 |  |  |  | pCR 28.572 Add requirements for plans | 0.1.0 |
| 2024-10 | SA5#157 | S5-246082 |  |  |  | pCR 28.572 Add requirements for transaction | 0.1.0 |
| 2024-10 | SA5#157 | S5-246083 |  |  |  | pCR 28.572 Add requirements for validation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246084 |  |  |  | pCR 28.572 Add requirements for activation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246085 |  |  |  | pCR 28.572 Add content for plans | 0.1.0 |
| 2024-10 | SA5#157 | S5-246086 |  |  |  | pCR 28.572 Add content for transactions | 0.1.0 |
| 2024-10 | SA5#157 | S5-246087 |  |  |  | pCR 28.572 Add content for validation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246088 |  |  |  | pCR 28.572 Add content for activation | 0.1.0 |
| 2024-11 | SA5#155 | S5-247183 |  |  |  | pCR 28.572 Add conditional activation of plans | 0.2.0 |
| 2024-11 | SA5#155 | S5-247184 |  |  |  | pCR 28.572 Improve description of clause Planned configurations | 0.2.0 |
| 2024-11 | SA5#155 | S5-247185 |  |  |  | pCR 28.572 Improve description of clause Planned configuration groups | 0.2.0 |
| 2024-11 | SA5#155 | S5-247186 |  |  |  | pCR 28.572 Improve description of clause Validation of planned configurations | 0.2.0 |
| 2024-11 | SA5#155 | S5-247187 |  |  |  | pCR 28.572 Improve description of clause Activation of planned configurations | 0.2.0 |
| 2024-11 | SA5#155 | S5-247188 |  |  |  | pCR 28.572 Improve description of clauses about managing planned configurations | 0.2.0 |
| 2024-11 | SA5#155 | S5-247189 |  |  |  | pCR 28.572 Add examples for REST | 0.2.0 |