| 3GPP TS 29.579 V0.3.0 (2022-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Interworking MSC For Short Message Services Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	Overview	8

5	Services offered by the SMS-IWMSC	8

5.1	Introduction	8

5.2	Niwmsc_SMService Service	9

5.2.1	Service Description	9

5.2.2	Service Operations	9

5.2.2.1	Introduction	9

5.2.2.2	MoForwardSm	9

5.2.2.2.1	General	9

5.2.2.2.2	SBI-based MO SM transfer	9

6	API Definitions	10

6.1	Niwmsc_SMService Service API	10

6.1.1	Introduction	10

6.1.2	Usage of HTTP	10

6.1.2.1	General	10

6.1.2.2	HTTP standard headers	10

6.1.2.2.1	General	10

6.1.2.2.2	Content type	10

6.1.2.3	HTTP custom headers	11

6.1.2.4	HTTP multipart messages	11

6.1.3	Resources	11

6.1.3.1	Overview	11

6.1.3.2	Resource: MoSmInfo	12

6.1.3.2.1	Description	12

6.1.3.2.2	Resource Definition	12

6.1.3.2.3	Resource Standard Methods	12

6.1.3.2.4	Resource Custom Operations	13

6.1.4	Custom Operations without associated resources	14

6.1.5	Notifications	14

6.1.6	Data Model	14

6.1.6.1	General	14

6.1.6.2	Structured data types	15

6.1.6.3	Simple data types and enumerations	15

6.1.6.3.1	Introduction	15

6.1.6.3.2	Simple data types	15

6.1.6.4	Data types describing alternative data types or combinations of data types	15

6.1.6.5	Binary data	15

6.1.6.5.1	Binary Data Types	15

6.1.6.5.2	SMS Payload Information	15

6.1.7	Error Handling	16

6.1.7.1	General	16

6.1.7.2	Protocol Errors	16

6.1.7.3	Application Errors	16

6.1.8	Feature negotiation	16

6.1.9	Security	16

Annex A (normative): OpenAPI specification	17

A.1	General	17

A.2	Niwmsc_SMService API	17

Annex B (informative): Withdrawn API versions	19

B.1	General	19

B.2	Niwmsc_SMService API	19

Annex C (informative): Change history	20

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Niwmsc Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the SMS-IWMSC.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

Stage 2 requirements for the Niwmsc services are specified in 3GPP TS 23.540 [14].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.540: "Technical realization of Service Based Short Message Service; Stage 2".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[16]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[17]	3GPP TS 29.577: "5G System; IP Short Message Gateway and SMS Router For Short Message Service; Stage 3".

[18]	3GPP TS 24.011: " Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Niwmsc:	Service-based interface exhibited by the SMS-IWMSC

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SM MO	Short Message Mobile Originated

# 4	Overview

The SMS-IWMSC offers services to the SMSF via the Niwmsc service based interface (see 3GPP TS 23.501 [2], 3GPP TS 23.502 [3], and 3GPP TS 23.540 [14]).

Figure 4.1-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the SMS-IWMSC.

Figure 4.1-1: Reference model – SMS-IWMSC

The functionalities supported by the SMS-IWMSC are listed in clause 6.3 of 3GPP TS 23.540 [14].

# 5	Services offered by the SMS-IWMSC

## 5.1	Introduction

The SMS-IWMSC offers the following services via the Niwmsc interface:

-	Niwmsc_SMService Service

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Niwmsc_SMService | 6.1 | SMS-IWMSC short message service | TS29579_Niwmsc_ SMService.yaml | niwmsc-smservice | A.2 |

## 5.2	Niwmsc_SMService Service

### 5.2.1	Service Description

See 3GPP TS 23.540 [14] clause 6.3.1

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

For the Niwmsc_SMService service the following service operations are defined:

-	MoForwardSm

The Niwmsc_SMService Service is used by Consumer NFs (SMSF) to transfer MO short message by means of the MoForwardSm service operation.

#### 5.2.2.2	MoForwardSm

##### 5.2.2.2.1	General

This clause provides a general description of the MoForwardSm service operation.

##### 5.2.2.2.2	SBI-based MO SM transfer

The MoForwardSm service operation shall be used to transmit uplink SMS message via SMS-IWMSC.

It is used in the following procedures:

-	Successful Mobile Originated short message transfer via SMS-IWMSC (see clause 5.2.2 of 3GPP TS 23.540 [14]).

-	Unsuccessful Mobile Originated short message transfer via SMS-IWMSC (see clause 5.2.3 of 3GPP TS 23.540 [14]).

The NF Service Consumer (e.g. SMSF) shall transmit uplink SMS message to the SMS-IWMSC by using the HTTP POST method as shown in Figure 5.2.2.2.1-1.

Figure 5.2.2.2.2-1: SBI-based MO SM transfer

1.	The NF Service Consumer shall send a POST request to the resource representing the UE's Mobile Originated Short Message Information resource (i.e. …/mo-sm-info/{supi}/sendsms) of the SMS-IWMSC. The payload body of the POST request shall contain the SMS message to be sent.

2a.	On success, "200 OK" shall be returned with "SmsDeliveryData" object contains the MO SMS Delivery Report in the response body.

2b.	On failure, or redirection, one of the HTTP status code listed in Table 6.1.3.2.4.2.2-2 shall be returned.

# 6	API Definitions

## 6.1	Niwmsc_SMService Service API

### 6.1.1	Introduction

The Niwmsc_SMService shall use the Niwmsc_SMService API.

The API URI of the Niwmsc_SMService API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "niwmsc-smservice".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Niwmsc_SMService API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

Multipart messages shall also be supported (see clause 6.1.2.4) using the content type "multipart/related", comprising:

-	one JSON body part with the "application/json" content type; and

-	one binary body part with 3gpp vendor specific content subtypes.

The 3gpp vendor specific content subtypes defined in Table 6.1.2.2.2-1 shall be supported.

Table 6.1.2.2.2-1: 3GPP vendor specific content subtypes

| content subtype | Description |
| --- | --- |
| vnd.3gpp.sms | Binary encoded payload, encoding SMS payload, as specified in 3GPP TS 23.040 [16] and 3GPP TS 24.011 [17]. |
| NOTE: Using 3GPP vendor content subtypes allows to describe the nature of the opaque payload (e.g. SMS payload) without having to rely on metadata in the JSON payload. |  |

See clause 6.1.2.4 for the binary payloads supported in the binary body part of multipart messages.

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

#### 6.1.2.4	HTTP multipart messages

HTTP multipart messages shall be supported, to transfer opaque SMS payload (e.g. SMS message, CP Ack, etc.), in the following service operations (and HTTP messages):

-	MoForwardSm service operation;

HTTP multipart messages shall include one JSON body part and one binary body part comprising content of SMS payload content (see clause 6.1.6.5).

The JSON body part shall be the "root" body part of the multipart message. It shall be encoded as the first body part of the multipart message. The "Start" parameter does not need to be included.

The multipart message shall include a "type" parameter (see IETF RFC 2387 [18]) specifying the media type of the root body part, i.e. "application/json".

NOTE:	The "root" body part (or "root" object) is the first body part the application processes when receiving a multipart/related message, see IETF RFC 2387 [18]. The default root is the first body within the multipart/related message. The "Start" parameter indicates the root body part, e.g. when this is not the first body part in the message.

A binary body part shall include a Content-ID header (see IETF RFC 2045 [19]), and the JSON body part shall make a reference to the binary body part using the Content-ID header field.

Examples of multipart/related messages can be found in Annex B.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Niwmsc_SMService API.

Figure 6.1.3.1-1: Resource URI structure of the Niwmsc_SMService API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| SMService | /mo-sm-info/{supi}/sendsms | sendsms (POST) | MO short message transfer |

#### 6.1.3.2	Resource: MoSmInfo

##### 6.1.3.2.1	Description

This resource represents the collection of Mobile Originated Short Message Information in SMS-IWMSC.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/<apiName>/<apiVersion>/mo-sm-info{supi}

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| supi | Supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [15] |

##### 6.1.3.2.3	Resource Standard Methods

No HTTP method has been defined for the Mobile Originated Short Message Information collection resource.

##### 6.1.3.2.4	Resource Custom Operations

6.1.3.2.4.1	Overview

Table 6.1.3.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| sendsms | /mo-sm-infos/{supi}/sendsms | POST | Send MO SMS message or the related Delivery Report. |

6.1.3.2.4.2	Operation: sendsms

6.1.3.2.4.2.1	Description

This custom operation is used for NF Service Consumers to send SMS message in uplink direction.

6.1.3.2.4.2.2	Operation Definition

This custom operation is used to send a SMS payload to an UE's Mobile Originated Short Message Information resource in the SMS-IWMSC.

This operation shall support the request data structures specified in table 6.1.3.2.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.2.4.2.2-2.

Table 6.1.3.2.4.2.2-1: Data structures supported by the sendsms Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmsData | M | 1 | Representation of the MO SMS message to be sent. |

Table 6.1.3.2.4.2.2-2: Data structures supported by the sendsms Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmsDeliveryData | M | 1 | 200 OK | This case represents the successful of sending SMS message in uplink direction, with necessary response data on the received delivery report. |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents an unsuccessful delivery of SMS message. The "cause" attribute may be used to indicate one of the following application errors: - SMS_PAYLOAD_MISSING, if the expected SMS payload content is missing; - SMS_PAYLOAD_ERROR, if error exists in the SMS payload content. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This case represents an unsuccessful delivery of SMS message. The "cause" attribute may be used to indicate one of the following application errors: - UNKNOWN_SERVICE_CENTRE_ADDRESS, if the SMS-SC was unknown; - SERVICE_CENTRE_CONGESTION, if the SMS-SC was in congestion; - USER_NOT_SERVICE_CENTER, if the user didn't belongs to the SMS-SC; - FACILITY_NOT_SUPPORTED, if the facility not supported; - INVALID_SME_ADDRESS, if the SME address is invalid.. |
| ProblemDetails | O | 0..1 | 504 Gateway Timeout | This case represents an unsuccessful delivery of SMS message. The "cause" attribute may be used to indicate one of the following application errors: - UNREACHABLE_SMS_SC, if the response is timeout. |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.1.4	Custom Operations without associated resources

In this release of this specification, no custom operations without associated resources are defined.

### 6.1.5	Notifications

In this release of this specification, no notification procedures are defined.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Niwmsc service based interface protocol.

Table 6.1.6.1-1: Niwmsc specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 6.1.6.1-2 specifies data types re-used by the Niwmsc service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Niwmsc service based interface.

Table 6.1.6.1-2: Niwmsc re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [15] | Common Data Type used in response bodies |  |
| RedirectResponse | 3GPP TS 29.571 [15] | Redirect Response |  |
| Supi | 3GPP TS 29.571 [15] | Subscription Permanent Identifier |  |
| RefToBinaryData | 3GPP TS 29.571 [15] | Information for indicating the binary content of SMS payload. |  |
| Ipv4Addr | 3GPP TS 29.571 [15] | IPv4 address |  |
| Ipv6Addr | 3GPP TS 29.571 [15] | IPv6 address |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Supported Features |  |
| SmsData | 3GPP TS 29.577 [17] | Information within request message invoking MoForwardSm service operation, for delivering MO SMS. |  |
| SmsDeliveryData | 3GPP TS 29.571 [17] | Information within response message invoking MoForwardSm service operation, for delivering MO SMS Delivery Report. |  |

#### 6.1.6.2	Structured data types

In this release of this specification, no structure to be used in resource representations is defined.

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| N/A |  |  |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

None.

#### 6.1.6.5	Binary data

##### 6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| SMS Payload Information | 6.1.6.4.2 | vnd.3gpp.sms |

##### 6.1.6.5.2	SMS Payload Information

SMS Payload Information shall encode a SMS payload as specified in 3GPP TS 23.040 [16] and 3GPP TS 24.011 [18], using the vnd.3gpp.sms content-type.

SMS Payload Information may encode e.g. the following content:

-	CP-DATA, CP-ACK, CP-ERROR as specified in 3GPP TS 23.040 [16] and 3GPP TS 24.011 [18].

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Niwmsc_SMService API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Niwmsc_SMService API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Niwmsc_SMService service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Niwmsc_SMService service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| SMS_PAYLOAD_MISSING | 400 Bad Request | The expected SMS payload content is missing. |
| SMS_PAYLOAD_ERROR | 400 Bad Request | Errors exist in the format of SMS payload. |
| SERVICE_CENTRE_CONGESTION | 403 Forbidden | The delivery of the MO short message failed because SMS-SC was in congestion. |
| USER_NOT_SERVICE_CENTER | 403 Forbidden | The delivery of the short message failed because the user didn't belongs to the SMS-SC. |
| FACILITY_NOT_SUPPORTED | 403 Forbidden | The delivery of the MO short message failed because of facility not supported. |
| INVALID_SME_ADDRESS | 403 Forbidden | The delivery of the MO short message failed because the SME address is invalid. |
| UNREACHABLE_SMS_SC | 504 Gateway Timeout | The delivery of the MO short message failed because the response is timeout. |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Niwmsc_SMService API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| N/A |  |  |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Niwmsc_SMService API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Niwmsc_SMService API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Niwmsc_SMService service.

The Niwmsc_SMService API defines a single scope "niwmsc-smservice" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

# A.2	Niwmsc_SMService API

openapi: 3.0.0

info:

title: 'Niwmsc_SMService'

version: '1.0.0-alpha.1'

description: |

SMS-IWMSC Short Message Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.579 V0.1.0; 5G System; SMS Services; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.579/

servers:

- url: '{apiRoot}/niwmsc-smservice/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- oAuth2ClientCredentials:

- niwmsc-smservice

- {}

paths:

/mo-sm-infos/{supi}/sendsms:

post:

summary: Send SMS payload for a given UE

operationId: SendSMS

tags:

- Send MO SMS message and the delivery report

parameters:

- name: supi

in: path

required: true

description: Subscription Permanent Identifier (SUPI)

schema:

type: string

requestBody:

content:

multipart/related:

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/SmsData'

binaryPayload:

type: string

format: binary

encoding:

jsonData:

contentType: application/json

binaryPayload:

contentType: application/vnd.3gpp.sms

headers:

Content-Id:

schema:

type: string

required: true

responses:

'200':

description: sending delivery report

content:

multipart/related:

schema:

$ref: '#/components/schemas/SmsDeliveryData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

niwmsc-smservice: Access to the niwmsc-smservice API

schemas:

SmsData:

description: >

Information within resquest message invoking MoForwardSm service operation,

for delivering MO SMS.

type: object

required:

- smsPayload

properties:

smsPayload:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

SmsDeliveryData:

description: >

Information within response message invoking MoForwardSm service operation,

for delivering MO SMS Delivery Report.

type: object

required:

- smsPayload

properties:

smsPayload:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

# COMPLEX TYPES:

# SIMPLE TYPES:

# ENUMS:

Annex B (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. 3GPP TS 29.501 [5] clause 4.3.1.6 describes the withdrawal of API versions.

# B.2	Niwmsc_SMService API

The API versions listed in table B.2-1 are withdrawn for the Niwmsc_SMService API.

Table B.2-1: Withdrawn API versions of the Niwmsc_SMService service

| API version number | Remarks |
| --- | --- |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | CT4#109e | C4-222331 |  |  |  | C4-222331 as basis | 0.1.0 |
| 2022-04 | CT4#109e | C4-222343 |  |  |  | Implementation of C4-222343 in CT4#109e | 0.2.0 |
| 2022-05 | CT4#110e | C4-223452 |  |  |  | Implementation of C4-223219 and C4-223224 in CT4#110e | 0.3.0 |