| 3GPP TS 29.580 V0.3.0 (2022-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Multicast/Broadcast Service Function Services Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Overview	10

5	Services offered by the MBSF	11

5.1	Introduction	11

5.2	Nmbsf_MBSUserService Service	11

5.2.1	Service Description	11

5.2.2	Service Operations	11

5.2.2.1	Introduction	11

5.2.2.2	Nmbsf_MBSUserService_Create service operation	12

5.2.2.2.1	General	12

5.2.2.2.2	Provision a new MBS User Service	12

5.2.2.3	Nmbsf_MBSUserService_Retrieve service operation	13

5.2.2.3.1	General	13

5.2.2.3.2	Retrieve the properties of an existing MBS User Service	13

5.2.2.4	Nmbsf_MBSUserService_Update service operation	14

5.2.2.4.1	General	14

5.2.2.4.2	Update the properties of an existing MBS User Service	14

5.2.2.5	Nmbsf_MBSUserService_Delete service operation	14

5.2.2.5.1	General	14

5.2.2.5.2	Release the resources of the associated MBS User Service	15

5.3	Nmbsf_MBSUserDataIngestSession Service	15

5.3.1	Service Description	15

5.3.2	Service Operations	16

5.3.2.1	Introduction	16

5.3.2.2	Nmbsf_MBSUserDataIngestSession_Create service operation	16

5.3.2.2.1	General	16

5.3.2.2.2	Create an MBS User Data Ingest Session including a set of subordinate MBS Distribution Session(s)	16

5.3.2.3	Nmbsf_MBSUserDataIngestSession_Retrieve service operation	17

5.3.2.3.1	General	17

5.3.2.3.2	Retrieve the properties of an existing MBS User Data Ingest Session	17

5.3.2.4	Nmbsf_MBSUserDataIngestSession_Update service operation	18

5.3.2.4.1	General	18

5.3.2.4.2	Update the properties of an existing MBS User Data Ingest Session	18

5.3.2.5	Nmbsf_MBSUserDataIngestSession_Delete service operation	19

5.3.2.5.1	General	19

5.3.2.5.2	Delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s)	19

5.3.2.6	Nmbsf_MBSUserDataIngestSession_StatusSubscribe service operation	20

5.3.2.6.1	General	20

5.3.2.6.2	Subscribe to monitor event(s) relevant to the MBS User Dta Ingest Session	20

5.3.2.7	Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe service operation	21

5.3.2.7.1	General	21

5.3.2.7.2	Remove an existing status subscription to the MBS User Data Ingest Session	21

5.3.2.8	Nmbsf_MBSUserDataIngestSession_StatusNotify service operation	22

5.3.2.8.1	General	22

5.3.2.8.2	Notify status change of the MBS User Data Ingest Session or the status of a file	22

6	API Definitions	23

6.1	Nmbsf_MBSUserService Service API	23

6.1.1	Introduction	23

6.1.2	Usage of HTTP	24

6.1.2.1	General	24

6.1.2.2	HTTP standard headers	24

6.1.2.2.1	General	24

6.1.2.2.2	Content type	24

6.1.2.3	HTTP custom headers	24

6.1.3	Resources	24

6.1.3.1	Overview	24

6.1.3.2	Resource: MBS User Services	25

6.1.3.2.1	Description	25

6.1.3.2.2	Resource Definition	25

6.1.3.2.3	Resource Standard Methods	25

6.1.3.2.3.1	GET	25

6.1.3.2.3.2	POST	26

6.1.3.2.4	Resource Custom Operations	27

6.1.3.3	Resource: Individual MBS User Service	27

6.1.3.3.1	Description	27

6.1.3.3.2	Resource Definition	27

6.1.3.3.3	Resource Standard Methods	27

6.1.3.3.3.1	GET	27

6.1.3.3.3.2	PUT	28

6.1.3.3.3.3	PATCH	29

6.1.3.3.3.4	DELETE	30

6.1.4	Custom Operations without associated resources	31

6.1.5	Notifications	31

6.1.6	Data Model	31

6.1.6.1	General	31

6.1.6.2	Structured data types	32

6.1.6.2.1	Introduction	32

6.1.6.2.2	Type: MBSUserService	33

6.1.6.2.3	Type: ServiceNameDescription	33

6.1.6.2.4	Type: MBSUserServicePatch	34

6.1.6.3	Simple data types and enumerations	34

6.1.6.3.1	Introduction	34

6.1.6.3.2	Simple data types	34

6.1.6.3.3	Enumeration: ServiceAnnouncementMode	34

6.1.6.3.4	Enumeration: ServiceType	34

6.1.6.4	Data types describing alternative data types or combinations of data types	35

6.1.6.4.1	Type: <TypeName 1>	35

6.1.6.4.2	Type: <TypeName 2>	36

6.1.6.5	Binary data	36

6.1.6.5.1	Binary Data Types	36

6.1.6.5.2	< Binary Data 1 >	36

6.1.7	Error Handling	36

6.1.7.1	General	36

6.1.7.2	Protocol Errors	36

6.1.7.3	Application Errors	36

6.1.8	Feature negotiation	36

6.1.9	Security	37

6.2	Nmbsf_MBSUserDataIngestSession Service API	37

6.2.1	Introduction	37

6.2.2	Usage of HTTP	37

6.2.2.1	General	37

6.2.2.2	HTTP standard headers	37

6.2.2.2.1	General	37

6.2.2.2.2	Content type	38

6.2.2.3	HTTP custom headers	38

6.2.3	Resources	38

6.2.3.1	Overview	38

6.2.3.2	Resource: MBS User Data Ingest Sessions	39

6.2.3.2.1	Description	39

6.2.3.2.2	Resource Definition	39

6.2.3.2.3	Resource Standard Methods	40

6.2.3.2.3.1	GET	40

6.2.3.2.3.2	POST	40

6.2.3.2.4	Resource Custom Operations	41

6.2.3.3	Resource: Individual MBS User Data Ingest Session	41

6.2.3.3.1	Description	41

6.2.3.3.2	Resource Definition	41

6.2.3.3.3	Resource Standard Methods	42

6.2.3.3.3.1	GET	42

6.2.3.3.3.2	PATCH	43

6.2.3.3.3.3	DELETE	44

6.2.3.3.4	Resource Custom Operations	44

6.2.3.4	Resource: MBS User Data Ingest Session Status Subscriptions	45

6.2.3.4.1	Description	45

6.2.3.4.2	Resource Definition	45

6.2.3.4.3	Resource Standard Methods	45

6.2.3.4.3.1	GET	45

6.2.3.4.3.2	POST	46

6.2.3.4.4	Resource Custom Operations	47

6.2.3.5	Resource: Individual MBS User Data Ingest Session Status Subscription	47

6.2.3.5.1	Description	47

6.2.3.5.2	Resource Definition	47

6.2.3.5.3	Resource Standard Methods	47

6.2.3.5.3.1	GET	47

6.2.3.5.3.2	DELETE	48

6.2.3.5.4	Resource Custom Operations	49

6.2.4	Custom Operations without associated resources	49

6.2.5	Notifications	49

6.2.5.1	General	49

6.2.5.2	MBS User Data Ingest Session Status Notification	50

6.2.5.2.1	Description	50

6.2.5.2.2	Target URI	50

6.2.5.2.3	Standard Methods	50

6.2.5.2.3.1	POST	50

6.2.6	Data Model	51

6.2.6.1	General	51

6.2.6.2	Structured data types	52

6.2.6.2.1	Introduction	52

6.2.6.2.2	Type: MBSUserDataIngSession	52

6.2.6.2.3	Type: MBSDistributionSessionInfo	53

6.2.6.2.4	Type: MBSUserDataIngSessionPatch	54

6.2.6.2.5	Type: ObjectDistrMethInfo	54

6.2.6.2.6	Type: PacketDistrMethInfo	54

6.2.6.2.7	Type MBSUserDataIngStatSubsc	55

6.2.6.2.8	Type MBSUserDataIngStatNotif	55

6.2.6.2.9	Type EventNotification	55

6.2.6.3	Simple data types and enumerations	55

6.2.6.3.1	Introduction	55

6.2.6.3.2	Simple data types	55

6.2.6.3.3	Enumeration: DistributionMethod	56

6.2.6.3.4	Enumeration: DistributionOperatingMode	56

6.2.6.3.5	Enumeration: State	56

6.2.6.3.6	Enumeration: Event	57

6.2.6.3.7	Enumeration: ObjectAcqMethod	57

6.2.6.3.8	Enumeration: PacketIngestMethod	57

6.2.6.4	Data types describing alternative data types or combinations of data types	58

6.2.6.5	Binary data	58

6.2.6.5.1	Binary Data Types	58

6.2.7	Error Handling	58

6.2.7.1	General	58

6.2.7.2	Protocol Errors	58

6.2.7.3	Application Errors	58

6.2.8	Feature negotiation	58

6.2.9	Security	58

Annex A (normative): OpenAPI specification	60

A.1	General	60

A.2	Nmbsf_MBSUserService API	60

A.3	Nmbsf_MBSUserDataIngestSession API	63

Annex B (informative): Withdrawn API versions	63

B.1	General	63

B.2	Nmbsf_MBSUserService API	63

B.3	Nmbsf_MBSUserDataIngestSession API	64

Annex C (informative): Change history	65

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nmbsf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the MBSF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]. The stage 2 architecture and procedures for 5G Multicast/Broadcast Services are specified in 3GPP TS 23.247 [14] and 3GPP TS 26.502 [15].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[15]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[16]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purpose of the present document, the terms and definitions given in clause 3 of 3GPP TS 23.247 [14] and clause 3 of 3GPP TS 26.502 [15] also apply, including the ones referencing other specifications.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

FEC	Forward Erasure Correction

MBSF	Multicast/Broadcast Service Function

MBSTF	Multicast/Broadcast Service Transport Function

QoS	Quality of Service

TMGI	Temporary Mobile Group Identity

URI	Uniform Resource Identifier

# 4	Overview

In the frame of Multicast/Broadcast Services (MBS), the Multicast/Broadcast Service Function (MBSF) provides services to NF service consumers (e.g. AF, NEF) via the Nmbsf service based interface. The MBSF supports for this purpose the functionalities defined in 3GPP TS 26.502 [15] and 3GPP TS 23.247 [14], i.e. service level functionalities to support MBS and control of the MBSTF, when used.

Figures°4-1 and 4.2 depict the Multicast/Broadcast related reference architecture of the MBSF respectively in SBI representation and reference point representation.

Figure 4-1: Reference model for the MBSF Services – SBI representation

Figure 4-2: Reference Model for the MBSF Services – Reference point representation

Editor's Note:	There may be updates depending on the progress of the related stage 2 work.

# 5	Services offered by the MBSF

## 5.1	Introduction

The MBSF provides the following services:

-	Nmbsf_MBSUserService

-	Nmbsf_MBSUserDataIngestSession

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nmbsf_MBSUserService | 5.2 | MBS User Management Service | TS29580_ Nmbsf_MBSUserService.yaml | nmbsf-mbsuserserv | A.2 |
| Nmbsf_MBSUserDataIngestSession | 5.3 | MBS User Data Ingest Session Management Service | TS29580_ Nmbsf_MBSUserDataIngestSession.yaml | nmbsf-mbsuserdataing | A.3 |

## 5.2	Nmbsf_MBSUserService Service

### 5.2.1	Service Description

The Nmbsf_MBSUserService service exposed by the MBSF Server enables an NF service consumer to:

-	provision a new MBS User Service;

-	retrieve the properties of an existing MBS User Service;

-	update the properties of an existing MBS User Service; and

-	release the resources of an existing MBS User Service.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the Nmbsf_MBSUserService service are shown in table 5.2.2.1-1.

Table 5.2.2.1-1: Nmbsf_MBSUserService Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Nmbsf_MBSUserService_Create | This service operation enables the NF service consumer to provision a new MBS User Service. | AF, NEF |
| Nmbsf_MBSUserService_Retrieve | This service operation enables the NF service consumer to retrieve the properties of an existing MBS User Service. | AF, NEF |
| Nmbsf_MBSUserService_Update | This service operation enables the NF service consumer to update the properties of an existing MBS User Service. | AF, NEF |
| Nmbsf_MBSUserService_Delete (NOTE) | This service operation enables the NF service consumer to release the resources of an existing MBS User Service. | AF, NEF |
| NOTE: This service operation corresponds to the Nmbsf_MBSUserService_Destroy service operation defined in 3GPP TS 26.502 [15]. |  |  |

#### 5.2.2.2	Nmbsf_MBSUserService_Create service operation

##### 5.2.2.2.1	General

This service operation is used by an NF service consumer to provision the parameters of a new MBS User Service to the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserServie_Create" service operation:

-	provision a new MBS User Service by the NF service consumer to the MBSF as described in 3GPP TS 26.502 [15].

##### 5.2.2.2.2	Provision a new MBS User Service

Figure 5.2.2.2.2-1 depicts a scenario where an NF service consumer provision the parameters of a new MBS User Service to the MBSF.

Figure 5.2.2.2.2-1: NF service consumer provision a new MBS User Service

In order to provision a new MBS User Service, the NF service consumer shall send an HTTP POST request message to the resource URI "{apiRoot}/nmbsf-mbsuserserv/<apiVersion>/mbs-user-services" as shown in step 1 of figure 5.2.2.2.2-1, the request body shall include the MBSUserService data structure which shall include:

-	an external service identifier as the "extServiceId" attribute;

-	an MBS User Service distribution type as the the "servType" attribute;

-	a service class as the "servClass" attribute;

-	service announcement mode(s) as the "servAnnModes" attribute; and

-	service name(s) and service description(s) as the "servNameDescs" attribute.

and may include:

-	the main language of this MBS User Service in the "mainServLang" attribute.

Editor's Note:	It's FFS on the contents of MBSUserService to be aligned with TS 26.502.

Upon successful reception of an HTTP POST request, the MBSF shall create an " Individual MBS User Service" resource, the MBSF shall respond with "201 Created" status code with the message body containing a representation of the created subscription, as shown in figure 5.2.2.2.2-1, step 2, including the MBSUserService data structure in the response body. The MBSF shall include a Location HTTP header field. The Location header field shall contain the URI of the created subscription i.e. "{apiRoot}/nmbsf-mbsuserserv/<apiVersion>/mbs-user-services/{mbsUserServId}".

If the MBSF cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the MBSF shall send an HTTP error response as specified in clause 6.1.7.

#### 5.2.2.3	Nmbsf_MBSUserService_Retrieve service operation

##### 5.2.2.3.1	General

This service operation is used by an NF service consumer to retrieve the properties of an existing MBS User Service.

The following procedures are supported by the "Nmbsf_MBSUserServie_Retrieve" service operation:

-	retrieve the properties of an existing MBS User Service by the NF service consumer to the MBSF as described in 3GPP TS 26.502 [15].

##### 5.2.2.3.2	Retrieve the properties of an existing MBS User Service

Figure 5.2.2.3.2-1 depicts a scenario where an NF service consumer retrieve the properties of an existing MBS User Service from the MBSF.

Figure 5.2.2.3.2-1: NF service consumer retrieve the properties of an existing MBS User Service

In order to retrieve the properties of an existing MBS User Service, the NF service consumer shall send an HTTP GET request message to the resource URI "{apiRoot}/nmbsf-mbsuserserv/<apiVersion>/mbs-user-services/{mbsUserServId}" as shown in step 1 of figure 5.2.2.3.2-1.

If the MBSF determines the received HTTP GET request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

Upon the reception of the HTTP GET request message, the MBSF shall respond with an HTTP "200 OK" status code as shown in figure 5.2.2.3.2-1, step 2, the response body shall include the retrieved MBSUserService data structure which shall include:

-	the properties of an existing MBS User Service with the corresponding information as described in clause 5.2.2.2.2.

If the MBSF cannot successfully fulfil the received HTTP GET request due to an internal error or an error in the HTTP GET request message, the MBSF shall send an HTTP error response as specified in clause 6.1.7.

#### 5.2.2.4	Nmbsf_MBSUserService_Update service operation

##### 5.2.2.4.1	General

This service operation is used by an NF service consumer to update the properties of an existing MBS User Service.

The following procedures are supported by the "Nmbsf_MBSUserServie_Update" service operation:

-	update the properties of an existing MBS User Service by the NF service consumer to the MBSF as described in 3GPP TS 26.502 [15].

##### 5.2.2.4.2	Update the properties of an existing MBS User Service

Figure 5.2.2.4.2-1 depicts a scenario where an NF service consumer update the properties of an existing MBS User Service to the MBSF.

Figure 5.2.2.4.2-1: NF service consumer update the properties of an existing MBS User Service

In order to update the properties of an existing MBS User Service, the NF service consumer shall send an HTTP PUT or PATCH request message to the resource URI "{apiRoot}/nmbsf-mbsuserserv/<apiVersion>/mbs-user-services/{mbsUserServId}" as shown in step 1 of figure 5.2.2.4.2-1, with the MBSUserService data structure as the request body in an HTTP PUT request message or with the MBSUserServicePatch data structure as the request body in an HTTP PATCH request message.

If the MBSF determines the received HTTP PUT or PATCH request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

Upon the reception of the HTTP PUT or PATCH request message, the MBSF shall update an "Individual MBS User Service" resource, and shall respond with:

a)	HTTP "200 OK" status code with the message body containing the updated MBSUserService data structure, as shown in figure 5.2.2.4.2-1, step 2a; or

b)	HTTP "204 No Content" status code, as shown in figure 5.2.2.4.2-1, step 2b.

If the MBSF cannot successfully fulfil the received HTTP PUT or PATCH request due to an internal error or an error in the HTTP GET request message, the MBSF shall send an HTTP error response as specified in clause 6.1.7.

#### 5.2.2.5	Nmbsf_MBSUserService_Delete service operation

##### 5.2.2.5.1	General

This service operation is used by the NF service consumer to release the resources of the associated MBS User Service in MBSF.

The following procedures are supported by the "Nmbsf_MBSUserServie_Delete" service operation:

-	release the resources of the associated MBS User Service to the MBSF as described in 3GPP TS 26.502 [15].

##### 5.2.2.5.2	Release the resources of the associated MBS User Service

Figure 5.2.2.5.2-1 depicts a scenario where an NF service consumer release the resources of the associated MBS User Service to the MBSF.

Figure 5.2.2.5.2-1: NF service consumer release the resources of the associated MBS User Service

In order to release the resources of the associated MBS User Service, the NF service consumer shall send an HTTP DELETE request message to the resource URI "{apiRoot}/nmbsf-mbsuserserv/<apiVersion>/mbs-user-services/{mbsUserServId}" as shown in step 1 of figure 5.2.2.5.2-1.

If the MBSF determines the received HTTP DELETE request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

Upon the reception of the HTTP DELETE request message, the MBSF shall:

-	remove the corresponding Individual MBS User Service resource; and

-	respond with HTTP "204 No Content" status code, as shown in figure 5.2.2.5.2-1, step 2.

If the MBSF cannot successfully fulfil the received DELETE request due to an internal error or an error in the HTTP DELETE request message, the MBSF shall send an HTTP error response as specified in clause 6.1.7.

## 5.3	Nmbsf_MBSUserDataIngestSession Service

### 5.3.1	Service Description

The Nmbsf_MBSUserDataIngestSession service exposed by the MBSF Server enables an NF service consumer to:

-	create an MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s);

-	retrieve the properties of an existing MBS User Data Ingest Session;

-	update the properties of an existing MBS User Data Ingest Session and its set of subordinate MBS Distribtion Session(s);

-	delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s);

-	create a subscription to monitor event(s) related to the MBS User Data Ingest Session;

-	remove an existing subscription; and

-	receive notification(s) about the event(s) related to the MBS User Data Ingest Session or the status of a file.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operations defined for the Nmbsf_MBSUserDataIngestSession service are shown in table 5.3.2.1-1.

Table 5.3.2.1-1: Nmbsf_MBSUserDataIngestSession Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Nmbsf_MBSUserDataIngestSession_Create | This service operation enables the NF service consumer to create an MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s). | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_Retrieve | This service operation enables the NF service consumer to retrieve the properties of an existing MBS User Data Ingest Session. | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_Update | This service operation enables the NF service consumer to update the properties of an existing MBS User Data Ingest Session and its set of subordinate MBS Distribution Session(s). | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_Delete (NOTE) | This service operation enables the NF service consumer to delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s). | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_StatusSubscribe | This service operation enables the NF service consumer to create a subscription to monitor event(s) related to an MBS User Data Ingest Session. | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe | This service operation enables the NF service consumer to request the deletion of an existing subscription to monitoring event(s) related to an MBS User Data Ingest Session. | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_StatusNotify | This service operation enables the NF service consumer to receive notification(s) from the MBSF about the event(s) related to an MBS User Data Ingest Session or the status of a file. | MBSF |
| NOTE: This service operation corresponds to the Nmbsf_MBSUserDataIngestSession_Destroy service operation defined in 3GPP TS 26.502 [15]. |  |  |

#### 5.3.2.2	Nmbsf_MBSUserDataIngestSession_Create service operation

##### 5.3.2.2.1	General

This service operation is used by the NF service consumer to create an MBS User Data Ingest Session including a set of subordinate MBS Distribution Session(s).

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_Create" service operation:

-	create an MBS User Data Ingest Session including a set of subordinate MBS Distribution Session(s) to the MBSF as described in 3GPP TS 26.502 [15].

##### 5.3.2.2.2	Create an MBS User Data Ingest Session including a set of subordinate MBS Distribution Session(s)

Figure 5.3.2.2.2-1 depicts a scenario where an NF service consumer create an MBS User Data Ingest Session including a set of subordinate MBS Distribution Session(s) to the MBSF.

Figure 5.3.2.2.2-1: NF service consumer create an MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s)

In order to create an MBS User Data Ingest Session including a set of subordinate MBS Distribution Session(s), the NF service consumer shall send an HTTP POST request message to the resource URI "{apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/userdataing-sessions" as shown in step 1 of figure 5.3.2.2.2-1, the request body shall include the MBSUserDataIngSession data structure which shall include:

-	one or more MBS Distribution Sessions composed in the MBS User Data Ingest Session as the "mbsDisSessInfos" attribute.

and may include:

-	period(s) of time during which the MBS User Data Ingest Session is active in the MBS System.in the "actPeriods" attribute.

Editor's Note:	It's FFS on the contents of MBSUserDataIngSession to be aligned with TS 26.502.

Upon successful reception of an HTTP POST request, the MBSF shall create an " Individual MBS User Data Ingest Session" resource, the MBSF shall respond with "201 Created" status code with the message body containing a representation of the created resource, as shown in figure 5.3.2.2.2-1, step 2, including the MBSUserDataIngSession data structure in the response body. The MBSF shall include a Location HTTP header field. The Location header field shall contain the URI of the created subscription i.e. "{apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/userdataing-sessions/{sessionId}".

If the MBSF cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the MBSF shall send an HTTP error response as specified in clause 6.2.7.

#### 5.3.2.3	Nmbsf_MBSUserDataIngestSession_Retrieve service operation

##### 5.3.2.3.1	General

This service operation is used by the NF service consumer to retrieve the properties of an existing MBS User Data Ingest Session.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_Retrieve" service operation:

-	retrieve the properties of an existing MBS User Data Ingest Session by the NF service consumer to the MBSF as described in 3GPP TS 26.502 [15].

##### 5.3.2.3.2	Retrieve the properties of an existing MBS User Data Ingest Session

Figure 5.3.2.3.2-1 depicts a scenario where an NF service consumer retrieve the properties of an existing MBS User Data Ingest Session from the MBSF.

Figure 5.3.2.3.2-1: NF service consumer retrieve the properties of an existing MBS User Data Ingest Session

In order to retrieve the properties of an existing MBS User Data Ingest Session, the NF service consumer shall send an HTTP GET request message to the resource URI "{apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/userdataing-sessions/{sessionId}" as shown in step 1 of figure 5.3.2.3.2-1.

If the MBSF determines the received HTTP GET request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

Upon the reception of the HTTP GET request message, the MBSF shall respond with an HTTP "200 OK" status code as shown in figure 5.3.2.3.2-1, step 2, the response body shall include the MBSUserDataIngSession data structure which shall include:

-	the properties of an existing MBS User Data Ingest Session with the corresponding information as described in clause 5.3.2.2.2.

If the MBSF cannot successfully fulfil the received HTTP GET request due to an internal error or an error in the HTTP GET request message, the MBSF shall send an HTTP error response as specified in clause 6.2.7.

#### 5.3.2.4	Nmbsf_MBSUserDataIngestSession_Update service operation

##### 5.3.2.4.1	General

This service operation is used by the NF service consumer to update the properties of an existing MBS User Data Ingest Session and its set of subordinate MBS Distribtion Session(s).

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_Update" service operation:

-	update the properties of an existing MBS User Data Ingest Session and its set of subordinate MBS Distribtion Session(s) by the NF service consumer to the MBSF as described in 3GPP TS 26.502 [15].

##### 5.3.2.4.2	Update the properties of an existing MBS User Data Ingest Session

Figure 5.3.2.4.2-1 depicts a scenario where an NF service consumer update the properties of an existing MBS User Data Ingest Session and its set of subordinate MBS Distribtion Session(s) to the MBSF.

Figure 5.3.2.4.2-1: NF service consumer update the properties of an existing MBS User Data Ingest Session and and its set of subordinate MBS Distribtion Session(s)

In order to update the properties of an existing MBS User Data Ingest Session and its set of subordinate MBS Distribtion Session(s), the NF service consumer shall send an HTTP PUT or PATCH request message to the resource URI "{apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/userdataing-sessions/{sessionId}" as shown in step 1 of figure 5.3.2.4.2-1, with the MBSUserDataIngSession data structure as the request body in HTTP PUT request message or with the MBSUserDataIngSessionPatch data structure as the request body in HTTP PATCH request message.

If the MBSF determines the received HTTP PUT or PATCH request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

Upon the reception of the HTTP PUT or PATCH request message, the MBSF shall respond with:

a)	HTTP "200 OK" status code with the message body containing the updated MBSUserDataIngSession data structure, as shown in figure 5.3.2.4.2-1, step 2a; or

b)	HTTP "204 No Content" status code, as shown in figure 5.3.2.4.2-1, step 2b.

If the MBSF cannot successfully fulfil the received HTTP PUT or PATCH request due to an internal error or an error in the HTTP GET request message, the MBSF shall send an HTTP error response as specified in clause 6.1.7.

#### 5.3.2.5	Nmbsf_MBSUserDataIngestSession_Delete service operation

##### 5.3.2.5.1	General

This service operation is used by the NF service consumer to delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s) in the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_Delete" service operation:

-	delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s) as described in 3GPP TS 26.502 [15].

##### 5.3.2.5.2	Delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s)

Figure 5.3.2.5.2-1 depicts a scenario where an NF service consumer release the resources of the associated MBS User Data Ingest Session to the MBSF.

Figure 5.3.2.5.2-1: NF service consumer delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s).

In order to delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s), the NF service consumer shall send an HTTP DELETE request message to the resource URI "{apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/userdataing-sessions/{sessionId}" as shown in step 1 of figure 5.3.2.5.2-1.

If the MBSF determines the received HTTP DELETE request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

Upon the reception of the HTTP DELETE request message, the MBSF shall:

-	remove the corresponding Individual MBS User Data Ingest Session resource; and

-	respond with HTTP "204 No Content" status code, as shown in figure 5.3.2.5.2-1, step 2.

If the MBSF cannot successfully fulfil the received DELETE request due to an internal error or an error in the HTTP DELETE request message, the MBSF shall send an HTTP error response as specified in clause 6.2.7.

#### 5.3.2.6	Nmbsf_MBSUserDataIngestSession_StatusSubscribe service operation

##### 5.3.2.6.1	General

This service operation is invoked by the NF service consumer to subscribe to the MBSF to monitor at least one event relevant to the status of the MBS User Data Ingest Session. The AF may subscribe to multiple events in a subscription.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_StatusSubscribe" service operation:

-	subscribe to monitor at least one event relevant to the status of the MBS User Data Ingest Session. The AF may subscribe to multiple events in a subscription. as described in 3GPP TS 26.502 [15].

##### 5.3.2.6.2	Subscribe to monitor event(s) relevant to the MBS User Dta Ingest Session

Figure 5.3.2.6.2-1 depicts a scenario where an NF service consumer subscribe to monitor at least one event relevant to the status of the MBS User Data Ingest Session to the MBSF.

Figure 5.3.2.6.2-1: NF service consumer subscribe to monitor at least one event relevant to the status of the MBS User Data Ingest Session

In order to subscribe to monitor at least one event relevant to the status of the MBS User Data Ingest Session, the NF service consumer shall send an HTTP POST request message to the resource URI "{apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/status-subscriptions" as shown in step 1 of figure 5.3.2.6.2-1, the request body shall include the MBSUserDataIngStatSubsc data structure which shall include:

-	an MBS Distribution Session Identifier as the "mbsIngSessionId" attribute;

-	an URI where to receive the requested notifications as the "notifUri" attribute; and

-	event Id(s) as the "eventsubscs" attribute.

Upon successful reception of an HTTP POST request, the MBSF shall create an "Individual MBS User Data Ingest Session Status Subscription" resource, the MBSF shall respond with "201 Created" status code with the message body containing a representation of the created subscription, as shown in figure 5.3.2.6.2-1, step 2, including the MBSUserDataIngStatSubsc data structure in the response body. The MBSF shall include a Location HTTP header field. The Location header field shall contain the URI of the created subscription i.e. "{apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/status-subscriptions/{subscriptionId}".

If the MBSF cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the MBSF shall send an HTTP error response as specified in clause 6.2.7.

#### 5.3.2.7	Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe service operation

##### 5.3.2.7.1	General

This service operation is used by the NF service consumer to remove an existing status subscription to the MBS User Data Ingest Session in the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe" service operation:

-	remove an existing status subscription to the MBS User Data Ingest Session as described in 3GPP TS 26.502 [15].

##### 5.3.2.7.2	Remove an existing status subscription to the MBS User Data Ingest Session

Figure 5.3.2.7.2-1 depicts a scenario where an NF service consumer remove an existing status subscription to the MBS User Data Ingest Session.

Figure 5.3.2.7.2-1: NF service consumer remove an existing status subscription to the MBS User Data Ingest Session

In order to remove an existing status subscription to the MBS User Data Ingest Session, the NF service consumer shall send an HTTP DELETE request message to the resource URI "{apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/status-subscriptions/{subscriptionId}" as shown in step 1 of figure 5.3.2.7.2-1.

If the MBSF determines the received HTTP DELETE request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

Upon the reception of the HTTP DELETE request message, the MBSF shall:

-	remove the corresponding Individual MBS User Data Ingest Session Status Subscription resource; and

-	respond with HTTP "204 No Content" status code, as shown in figure 5.3.2.7.2-1, step 2.

If the MBSF cannot successfully fulfil the received DELETE request due to an internal error or an error in the HTTP DELETE request message, the MBSF shall send an HTTP error response as specified in clause 6.2.7.

#### 5.3.2.8	Nmbsf_MBSUserDataIngestSession_StatusNotify service operation

##### 5.3.2.8.1	General

This service operation is used by the MBSF to notify the subscribed NF service consumer about the status change of the MBS User Data Ingest Session or the status of a file.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_StatusNotify" service operation:

-	MBSF notify the NF service consumer about the status change of the MBS User Data Ingest Session or the status of a file.as described in 3GPP TS 26.502 [15].

##### 5.3.2.8.2	Notify status change of the MBS User Data Ingest Session or the status of a file

Figure 5.3.2.8.2-1 depicts a scenario where the MBSF notify the subscribed NF service consumer about the status change of the MBS User Data Ingest Session or the status of a file.

Figure 5.3.2.8.2-1: Notification about subscribed MBS User Data Ingest Session status change(s)

Upon the occurrence of a previously subscribed event, the MBSF shall invoke the Nmbsf_MBSUserData_Notify service operation to notify the subscribed status change event(s) to the NF service consumer. The MBSF shall send an HTTP POST request with "{notifiUri}" received in the Nmbsf_MBSUserDataIngestSession_Subscribe service operation as the Resource URI and MBSUserDataIngStatNotif data structure as request body that shall include:

-	an MBS Distribution Session Identifier as "mbsIngSessionId" attribute; and

-	description of the notified event as "eventNotifs" attribute.

Upon the reception of an HTTP POST request, if the NF service consumer successfully processed and accepted the received HTTP POST request, the NF Service Consumer shall store the notification and respond with HTTP "204 No Content" status code.

If the NF service consumer determines the received HTTP POST request message needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

If the NF service consumer cannot successfully fulfil the received POST request due to an internal error or an error in the HTTP POST request message, the NF service consumer shall send an HTTP error response as specified in clause 6.2.7.

# 6	API Definitions

## 6.1	Nmbsf_MBSUserService Service API

### 6.1.1	Introduction

The Nmbsf_MBSUserService service shall use the Nmbsf_MBSUserService  API.

The API URI of the Nmbsf_MBSUserService API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nmbsf-mbsuserserv".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmbsf_MBSUserService API is contained in Annex A.2.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

Add specific information for the API if applicable.

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the Nmbsf_MBSUserService API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Services | /mbs-user-services | GET | Retrieve all the active MBS User Sessions managed by the MBSF. |
|  |  | POST | Request the creation of a new MBS User Session. |
| Individual MBS User Service | /mbs-user-services/{mbsUserServId} | GET | Retrieve an existng MBS User Session managed by the MBSF. |
|  |  | PUT | Update an existng MBS User Session managed by the MBSF. |
|  |  | PATCH | Modify an existng MBS User Session managed by the MBSF. |
|  |  | DELETE | Delete an existng MBS User Session managed by the MBSF. |

Editor's Note:	Whether the PUT method on the Individual MBS User Service resource should also be supported is FFS.

#### 6.1.3.2	Resource: MBS User Services

##### 6.1.3.2.1	Description

This resource represents the collection of MBS User Services managed by the MBSF.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbsuserserv/<apiVersion>/mbs-user-services

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve all the active MBS User Services managed by the MBSF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserService) | M | 1..N | 200 OK | Successful case. All the active MBS User Services managed by the MBSF are returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 6.1.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

###### 6.1.3.2.3.2	POST

The POST method allows an NF service consumer (e.g. AF, NEF) to request the creation of a new MBS User Service.

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserService | M | 1 | Contains the parameters to request the creation of a new MBS User Service. |

Table 6.1.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 201 Created | Successful case. The MBS User Service is successfully created and a representation of the created Individual MBS User Service resource is returned. An HTTP "Location" header that contains the resource URI of the created Individual MBS User Service resource shall also be included. |
| NOTE: The mandatory HTTP error status code for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.1.3.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmbsf-mbsuserserv/<apiVersion>/mbs-user-services/{mbsUserServId} |

##### 6.1.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.3.3	Resource: Individual MBS User Service

##### 6.1.3.3.1	Description

This resource represents an Individual MBS User Service resource managed by the MBSF.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbsuserserv/<apiVersion>/mbs-user-services/{mbsUserSevId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| mbsUserServId | string | Represents the unique identifier of the individual MBS User Service resource assigned by the MBSF. |

##### 6.1.3.3.3	Resource Standard Methods

###### 6.1.3.3.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve an existing Individual MBS User Service resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The requested Individual MBS User Service resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

###### 6.1.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserService | M | 1 | Contains the updated representation of the existing Individual MBS User Service resource that is to be updated. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Service resource is successfully updated and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Service resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.1.3.3.3.3	PATCH

This method shall support the URI query parameters specified in table 6.1.3.3.3.3-1.

Table 6.1.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3-3.

Table 6.1.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserServicePatch | M | 1 | Contains the parameters to request the modification of an existing Individual MBS User Service resource. |

Table 6.1.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Service resource is successfully modified and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Service resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.1.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.1.3.3.3.4	DELETE

This method shall support the URI query parameters specified in table 6.1.3.3.3.4-1.

Table 6.1.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.3.3.3.4-3.

Table 6.1.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Service resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.1.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

### 6.1.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.1.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the Nmbsf_MBSUserService API.

Table 6.1.6.1-1 specifies the data types defined for the Nmbsf_MBSUserService service based interface protocol.

Table 6.1.6.1-1: Nmbsf_MBSUserService specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MBSUserService | 6.1.6.2.2 | Represents MBS User Service parameters. |  |
| MBSUserServicePatch | 6.1.6.2.4 | Represents MBS User Service parameters may be modified. |  |
| ServiceAnnouncementMode | 6.1.6.3.3 | Represents Service Announcement Mode. |  |
| ServiceNameDescription | 6.1.6.2.3 | Represents Service Name and Service Description per language. |  |
| ServiceType | 6.1.6.3.4 | Represents MBS User Service distribution type. |  |

Table 6.1.6.1-2 specifies data types re-used by the Nmbsf_MBSUserService service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nmbsf_MBSUserService service based interface.

Table 6.1.6.1-2: Nmbsf_MBSUserService re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| NetworkAreaInfo | 3GPP TS 29.554 [16] | Contains the service areas in which this MBS User Service is to be made available. |  |
| SupportedFeatures | 3GPP TS 29.571 [17] | Used to negotiate the applicability of optional features. |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: MBSUserService

Table 6.1.6.2.2-1: Definition of type MBSUserService

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extServiceIds | array(string) | M | 1..N | Represents External Service Identifier(s) as the unique identifier(s) for this MBS User Service. |  |
| servType | ServiceType | M | 1 | Indicates this MBS User Service distribution type. |  |
| servClass | string | M | 1 | Represents the class of this MBS User Service, expressed as a term identifier from a controlled vocabulary. |  |
| servAnnModes | array(ServiceAnnouncementMode) | M | 1..N | Represents the MBS User Service Announcement Mode(s) compiled by the MBSF advertised to the MBSF Client. |  |
| servNameDescs | array(ServiceNameDescription) | M | 1..N | Contains a set of distinguishing names and descriptions for this MBS User Service, one per language. |  |
| mainServLang | string | O | 0..1 | Represents the main language of this MBS User Service. |  |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 6.1.8. This attribute may be provided in the HTTP POST request and response. |  |

Editor's Note:	It's FFS on the contents of MBSUserService to be aligned with TS 26.502.

##### 6.1.6.2.3	Type: ServiceNameDescription

Table 6.1.6.2.3-1: Definition of type ServiceNameDescription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servName | string | M | 1 | Represents a distinguishing name for this MBS User Service per language. |  |
| servDescrip | string | M | 1 | Contains description of this MBS User Service per language. |  |
| language | string | M | 1 | Represents the language of the distinguishing name and description for this MBS User Service. |  |

Editor's Note:	It's FFS on the contents of ServiceNameDescription to be aligned with TS 26.502.

##### 6.1.6.2.4	Type: MBSUserServicePatch

Table 6.1.6.2.4-1: Definition of type MBSUserServicePatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servAnnModes | array(ServiceAnnouncementMode) | O | 1..N | Represents the MBS User Service Announcement Mode(s) compiled by the MBSF advertised to the MBSF Client. |  |
| servNameDescs | array(ServiceNameDescription) | O | 1..N | Contains a set of distinguishing names and descriptions for this MBS User Service, one per language. |  |
| mainServLang | string | O | 0..1 | Represnets the main language of this MBS User Service. |  |

Editor's Note:	It's FFS on the contents of MBSUserServicePatch to be aligned with TS 26.502.

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

##### 6.1.6.3.3	Enumeration: ServiceAnnouncementMode

The enumeration ServiceAnnouncementMode represents Service Announce Mode. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration ServiceAnnouncementMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| VIA_MBS-5 | Indicates the MBS User Service Announcement compiled by the MBSF is advertised to the MBSF Client at reference point MBS-5. |  |
| VIA_MBS_DISTRIBUTION_SESSION | Indicates the MBS User Service Announcement compiled by the MBSF is advertised to the MBSF Client via the MBS Distribution Session. |  |
| PASSED_BACK | MBS User Service Announcement compiled by the MBSF is passed back to the MBS Application Provider. |  |

##### 6.1.6.3.4	Enumeration: ServiceType

The enumeration ServiceType represents MBS User Service distribution type. It shall comply with the provisions defined in table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: Enumeration ServiceType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| VIA_MULTICAST_MBS_SESSIONS | Indicates this MBS User Service is distributed via Multicast MBS Session(s). |  |
| VIA_BROADCAST_MBS_SESSIONS | Indicates this MBS User Service is distributed via Broadcast MBS Session(s). |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

##### 6.1.6.4.1	Type: <TypeName 1>

The data types describing alternative data types or combinations of data types shall represent an OpenAPI schema object using the "oneOf", "anyOf" or "allOf"  keyword to list alternative or to be combined data types (see the OpenAPI specification [4] and https://swagger.io/docs/specification/data-models/oneof-anyof-allof-not/).

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches , i.e. a list of mutually exclusive alternative data types., as described using the OpenAPI "oneOf" keyword, if the instance matches against one and only one of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of non-exclusive alternative data types, as described using the OpenAPI "anyOf" keyword, if the instance matches against one or more of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of to be combined data types, as described using the OpenAPI "allOf" keyword, if the instance matches against all of the to be combined data types."Attribute name": Name of attributes that belong to the specified data type. The attribute names within a structured data type shall be unique, and their relative order inside the structured data type shall not imply any specific ordering of the corresponding JSON elements in a JSON object.

"Data type": Data type of the alternative or to be combined data type. If the data type is indicated as "<type>", the alternative or to be combined data type shall be of data type <type>. If the data type is indicated as "array(<type>)", the alternative or to be combined data type shall be an array (see IETF RFC 8259 [3]) that contains elements of data type <type>. If the data type is indicated as "map(<type>)", the alternative or to be combined data type shall be an object (see IETF RFC 8259 [3]) encoded in the corresponding OpenAPI specification as a map which values are of data type <type>. <type> can either be "integer", "number", "string" or "boolean" (as defined in the OpenAPI specification [4]), or a data type defined in a 3GPP specification.

"Cardinality": Defines the allowed number of occurrence of data type <type>. A cardinality of "M..N", is only allowed for data types "array(<type>)" and "map(<type>)" and indicates the number of elements within the array or map; the values M and N can either be the characters "M" and "N", respectively, or integer numbers; with M being greater than or equal 0, and N being greater than 0. For data type "<type>", the cardinality shall be set to "1".

"Description": Describes the meaning and use of the attribute and may contain normative statements.Applicability: If the attribute is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [4], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the attribute can be used with any feature.

Applicability: If the type is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [2], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the type can be used with any feature. If no optional features are defined for an API, the applicability column can be omitted for that API.

Table 6.1.6.4.1-1: Definition of type <Type name 1> as a list of <"mutually exclusive alternatives" / "non-exclusive alternatives" / "to be combined data types">

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "1" or "M..N" | <only if applicable> |  |

##### 6.1.6.4.2	Type: <TypeName 2>

And so on if there are more types to specify.

#### 6.1.6.5	Binary data

This clause will specify what is encoded in binary part, if multipart media type is agreed to be supported by CT4 and is supported by the API. It shall be omitted if not applicable.

##### 6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| < Binary Data 1 > e.g. N1 SM Message | < clause > e.g. 6.1.6.5.2 | <content type> e.g. vnd.3gpp.5gnas |

##### 6.1.6.5.2	< Binary Data 1 >

And so on if there are more binary data to specify

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Nmbsf_MBSUserService API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmbsf_MBSUserService API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Nmbsf_MBSUserService service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Nmbsf_MBSUserService service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nmbsf_MBSUserService API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmbsf_MBSUserService API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmbsf_MBSUserService API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmbsf_MBSUserService service.

The Nmbsf_MBSUserService API defines a single scope "nmbsf-mbsuserserv" for the entire service, and it does not define any additional scopes at resource or operation level.

## 6.2	Nmbsf_MBSUserDataIngestSession Service API

### 6.2.1	Introduction

The Nmbsf_MBSUserDataIngestSession service shall use the Nmbsf_MBSUserDataIngestSession API.

The API URI of the Nmbsf_MBSUserDataIngestSession API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nmbsf-mbsuserdataing".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.2.3.

### 6.2.2	Usage of HTTP

#### 6.2.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmbsf_MBSUserDataIngestSession API is contained in Annex A.3.

#### 6.2.2.2	HTTP standard headers

##### 6.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.2.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 6.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.2.3	Resources

#### 6.2.3.1	Overview

Figure 6.2.3.1-1: Resource URI structure of the Nmbsf_MBSUserDataIngestSession API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Data Ingest Sessions | /userdataing-sessions | GET | Retrieve all the active MBS User Data Ingest Sessions managed by the MBSF. |
|  |  | POST | Request the creation of a new MBS User Data Ingest Session. |
| Individual MBS User Data Ingest Session | /userdataing-sessions/{sessionId} | GET | Retrieve an existng MBS User Data Ingest Session managed by the MBSF. |
|  |  | PATCH | Modify an existng MBS User Data Ingest Session managed by the MBSF. |
|  |  | DELETE | Delete an existng MBS User Data Ingest Session managed by the MBSF. |
| MBS User Data Ingest Session Status Subscriptions | /status-subscriptions | GET | Retrieve all the active MBS User Data Ingest Sessions status subscriptions managed by the MBSF. |
|  |  | POST | Request the creation of a new MBS User Data Ingest Session status subscription. |
| Individual MBS User Data Ingest Session Status Subscription | /status-subscriptions/{subscriptionId} | GET | Retrieve an existng MBS User Data Ingest Session status subscription managed by the MBSF. |
|  |  | DELETE | Delete an existng MBS User Data Ingest Session status subscription managed by the MBSF. |

Editor's Note:	Whether the PUT method on the Individual MBS User Data Ingest Session resource should also be supported is FFS.

Editor's Note:	Whether the PUT/PATCH methods on the Individual MBS User Data Ingest Session Status Subscription resource should also be supported is FFS.

#### 6.2.3.2	Resource: MBS User Data Ingest Sessions

##### 6.2.3.2.1	Description

This resource represents the collection of MBS User Data Ingest Sessions managed by the MBSF.

##### 6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/userdataing-sessions

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |

##### 6.2.3.2.3	Resource Standard Methods

###### 6.2.3.2.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve all the active MBS User Data Ingest Sessions managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserDataIngSession) | M | 1..N | 200 OK | Successful case. All the active MBS User Data Ingest Sessions managed by the MBSF are returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 6.2.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

###### 6.2.3.2.3.2	POST

The POST method allows an NF service consumer (e.g. AF, NEF) to request the creation of a new MBS User Data Ingest Session including one or several corresponding subordinate MBS Distribution Session(s).

This method shall support the URI query parameters specified in table 6.2.3.2.3.2-1.

Table 6.2.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.2-2 and the response data structures and response codes specified in table 6.2.3.2.3.2-3.

Table 6.2.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | Contains the parameters to request the creation of a new MBS User Data Ingest Session. |

Table 6.2.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 201 Created | Successful case. The MBS User Data Ingest Session is successfully created and a representation of the created Individual MBS User Data Ingest Session resource is returned. An HTTP "Location" header that contains the resource URI of the created Individual MBS User Data Ingest Session resource shall also be included. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.2.3.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/userdataing-sessions/{sessionId} |

##### 6.2.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.3	Resource: Individual MBS User Data Ingest Session

##### 6.2.3.3.1	Description

This resource represents an Individual MBS User Data Ingest Session resource managed by the MBSF.

##### 6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/userdataing-sessions/{sessionId}

This resource shall support the resource URI variables defined in table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |
| sessionId | string | Represents the unique identifier of the individual MBS User Data Ingest Session resource assigned by the MBSF. |

##### 6.2.3.3.3	Resource Standard Methods

###### 6.2.3.3.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve an existing Individual MBS User Data Ingest Session resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 200 OK | Successful case. The requested Individual MBS User Data Ingest Session resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

###### 6.2.3.3.3.2	PATCH

The PATCH method allows an NF service consumer (e.g. AF, NEF) to modify an existing Individual MBS User Data Ingest Session resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngSessionPatch | M | 1 | Contains the parameters to request the modification of an existing Individual MBS User Data Ingest Session resource. |

Table 6.2.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Data Ingest Session resource is successfully modified and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Data Ingest Session resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.3.3.3	DELETE

The DELETE method allows an NF service consumer (e.g. AF, NEF) to delete an existing Individual MBS User Data Ingest Session resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3-3.

Table 6.2.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Data Ingest Session resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

##### 6.2.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.4	Resource: MBS User Data Ingest Session Status Subscriptions

##### 6.2.3.4.1	Description

This resource represents the collection of MBS User Data Ingest Session Status Subscriptions managed by the MBSF.

##### 6.2.3.4.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/status-subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.4.2-1.

Table 6.2.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |

##### 6.2.3.4.3	Resource Standard Methods

###### 6.2.3.4.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve all the active MBS User Data Ingest Session Status Subscriptions managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.4.3.1-1.

Table 6.2.3.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.4.3.1-2 and the response data structures and response codes specified in table 6.2.3.4.3.1-3.

Table 6.2.3.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserDataIngStatSubsc) | M | 1..N | 200 OK | Successful case. All the active MBS User Data Ingest Session Status Subscriptions managed by the MBSF are returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 6.2.3.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

###### 6.2.3.4.3.2	POST

The POST method allows an NF service consumer (e.g. AF, NEF) to request the creation of a new MBS User Data Ingest Sesstion Status Subscription.

This method shall support the URI query parameters specified in table 6.2.3.4.3.2-1.

Table 6.2.3.4.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.4.3.2-2 and the response data structures and response codes specified in table 6.2.3.4.3.2-3.

Table 6.2.3.4.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | Contains the parameters to request the creation of a new MBS User Data Ingest Session Status Subscription. |

Table 6.2.3.4.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 201 Created | Successful case. The MBS User Data Ingest Session Status Subscription is successfully created and a representation of the created Individual MBS User Data Ingest Subscription resource is returned. An HTTP "Location" header that contains the resource URI of the created Individual MBS User Data Ingest Subscription resource shall also be included. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.2.3.4.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/status-subscriptions/{subscriptionId} |

##### 6.2.3.4.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.5	Resource: Individual MBS User Data Ingest Session Status Subscription

##### 6.2.3.5.1	Description

This resource represents an Individual MBS User Data Ingest Session Status Subscription resource managed by the MBSF.

##### 6.2.3.5.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbsuserdataing/<apiVersion>/status-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.3.5.2-1.

Table 6.2.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |
| subscriptionId | string | Represents the unique identifier of the individual MBS User Data Ingest Session Status Subscription resource assigned by the MBSF. |

##### 6.2.3.5.3	Resource Standard Methods

###### 6.2.3.5.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve an existing Individual MBS User Data Ingest Session Status Subscription resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.5.3.1-1.

Table 6.2.3.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.5.3.1-2 and the response data structures and response codes specified in table 6.2.3.5.3.1-3.

Table 6.2.3.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 200 OK | Successful case. The requested Individual MBS User Data Ingest Session Status Subscription resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 6.2.3.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

###### 6.2.3.5.3.2	DELETE

The DELETE method allows an NF service consumer (e.g. AF, NEF) to delete an existing Individual MBS User Data Ingest Session Status Subscription resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.5.3.2-1.

Table 6.2.3.5.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.5.3.2-2 and the response data structures and response codes specified in table 6.2.3.5.3.2-3.

Table 6.2.3.5.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.5.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Data Ingest Session Status Subscription resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error cases and the related status codes are FFS.

Table 6.2.3.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

##### 6.2.3.5.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 6.2.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.2.5	Notifications

#### 6.2.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Data Ingest Session Status Notification | {notifUri} | POST | This operation enables the MBSF to notify the NF service consumer (e.g. AF, NEF) about status change of an MBS User Data Ingest Session or the status of a file. |

#### 6.2.5.2	MBS User Data Ingest Session Status Notification

##### 6.2.5.2.1	Description

The MBS User Data Ingest Session Status Notification is used by the MBSF to notify the NF service consumer (e.g. AF, NEF) about status change of an MBS User Data Ingest Session or the status of a file.

##### 6.2.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.2.5.2.2-1.

Table 6.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | String formatted as URI with the Callback Uri. |

##### 6.2.5.2.3	Standard Methods

###### 6.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.2.5.2.3.1-1 and the response data structures and response codes specified in table 6.2.5.2.3.1-2.

Table 6.2.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatNotif | M | 1 | Represents an MBS User Data Ingest Session Status Notification. |

Table 6.2.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MBS User Data Ingest Session Status Notification is successfully received. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

Table 6.2.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the Nmbsf_MBSUserDataIngestSession API.

Table 6.2.6.1-1 specifies the data types defined for the Nmbsf_MBSUserDataIngestSession service based interface protocol.

Table 6.2.6.1-1: Nmbsf_MBSUserDataIngestSession specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| DistributionMethod | 6.2.6.3.3 | Represents MBS Distribution method. |  |
| DistributionOperatingMode | 6.2.6.3.4 | Represents MBS Distribution operating mode. |  |
| Event | 6.2.6.3.6 | Represents status change event. |  |
| EventNotification | 6.2.6.2.9 | Represents Event Notification. |  |
| State | 6.2.6.3.5 | Represents the current state of the MBS Distribution Session. |  |
| MBSDistributionSessionInfo | 6.2.6.2.3 | Represents MBS Distribution Session information. |  |
| MBSUserDataIngSession | 6.2.6.2.2 | Represents MBS User Data Ingest Session. |  |
| MBSUserDataIngSessionPatch | 6.2.6.2.4 | Represents MBS User Data Ingest Session parameters may be modified. |  |
| MBSUserDataIngStatNotif | 6.2.6.2.8 | Represents MBS User Data Ingest Session Status Notification. |  |
| MBSUserDataIngStatSubsc | 6.2.6.2.7 | Represents MBS User Data Ingest Session Status Subscription. |  |
| ObjectAcqMethod | 6.2.6.3.7 | Represents the Object Acquisition Method. |  |
| ObjectDistrMethInfo | 6.2.6.2.5 | Represents additional MBS Distribution Session parameters for Object Distribution Method. |  |
| PacketDistrMethInfo | 6.2.6.2.6 | Represents additional MBS Distribution Session parameters for Packet Distribution Method. |  |
| PacketIngestMethod | 6.2.6.3.8 | Represents packets ingest method. |  |

Table 6.2.6.1-2 specifies data types re-used by the Nmbsf_MBSUserDataIngestSession service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nmbsf_MBSUserDataIngestSession service based interface.

Table 6.2.6.1-2: Nmbsf_MBSUserDataIngestSession re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [n] | Represents Bit Rate. |  |
| 5Qi | 3GPP TS 29.571 [n] | Represents Unsigned integer representing a 5G QoS Identifier. |  |
| MbsSessionId | 3GPP TS 29.571 [n] | Represents an MBS Session Identifier. |  |
| PacketDelBudget | 3GPP TS 29.571 [n] | Represents Unsigned integer indicating Packet Delay Budget expressed in milliseconds. |  |
| TimeWindow | 3GPP TS 29.122 [o] | Represents a Time Window. |  |
| Tmgi | 3GPP TS 29.571 [n] | Represents Temporary Mobile Group Identity. |  |
| Uri | 3GPP TS 29.571 [n] | Represents Uniform Resource Identifier. |  |
| SupportedFeatures | 3GPP TS 29.571 [n] | Used to negotiate the applicability of optional features. |  |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.2.6.2.2	Type: MBSUserDataIngSession

Table 6.2.6.2.2-1: Definition of type MBSUserDataIngSession

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| actPeriods | array(TimeWindow) | O | 1..N | Represents periods of time during which the MBS User Data Ingest Session is active in the MBS System. If omitted, the session is active until further notice. |  |
| mbsDistSessInfos | array(MBSDistributionSessionInfo) | M | 1..N | Represents one or more MBS Distribution Sessions composed in the MBS User Data Ingest Session and these shall be provisioned in the same operation as the enclosing MBS User Data Ingest Session. |  |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 6.2.8. |  |

Editor's Note:	It's FFS on the contents of MBSUserDataIngSession to be aligned with TS 26.502.

##### 6.2.6.2.3	Type: MBSDistributionSessionInfo

Table 6.2.6.2.3-1: Definition of type MBSDistributionSessionInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | O | 0..1 | Represents the MBS Distribution Session Identifier with the Temporary Mobile Group Identity (TMGI) or Source-Specific Multicast (SSM) IP address of the MBS Session supporting this MBS Distribution Session. |  |
| qosInfo | 5Qi | M | 1 | Represents a 5G QoS Identifier (5QI) to be applied to the traffic flow for this MBS Distribution Session. |  |
| maxBitRate | BitRate | M | 1 | Represents the maximum bit rate for this MBS Distribution Session. |  |
| maxDelay | PacketDelBudget | O | 0..1 | Represents the maximum end-to-end distribution delay that is tolerated for this MBS Distribution Session by the MBS Application Provider. |  |
| distrMethod | DistributionMethod | M | 1 | Represents the distribution method for this MBS Distribution Session. |  |
| distrOpMode | DistributionOperatingMode | O | 0..1 | Represents the operating mode in the case where multiple modes are defined for the indicated distribution method. |  |
| fecConfig | string | O | 0..1 | Represents configuration for FEC information added by the MBSTF to protect this MBS Distribution Session. |  |
| objDistrInfo | ObjectDistrMethInfo | O | 0..1 | Represents MBS Distribution Session parameters for Object Distribution Method. May only be present when the "distrMethod" attribute value is set as "OBJECT". |  |
| pckDistrInfo | PacketDistrMethInfo | O | 0..1 | Represents MBS Distribution Session parameters for Packet Distribution Method. May only be additionally present when the "distrMethod" attribute value is set as "PACKET". |  |

Editor's Note:	It's FFS on the contents of MBSDistributionSessionInfo to be aligned with TS 26.502.

##### 6.2.6.2.4	Type: MBSUserDataIngSessionPatch

Table 6.2.6.2.4-1: Definition of type MBSUserDataIngSessionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| actPeriods | array(TimeWindow) | O | 1..N | Represents periods of time during which the MBS User Data Ingest Session is active in the MBS System. |  |
| mbsDistSessInfos | array(MBSDistributionSessionInfo) | O | 1..N | Contains one or more MBS Distribution Sessions composed in the MBS User Data Ingest Session may be modified. |  |

Editor's Note:	It's FFS on the contents of MBSUserDataIngSessionPatch to be aligned with TS 26.502.

##### 6.2.6.2.5	Type: ObjectDistrMethInfo

Table 6.2.6.2.5-1: Definition of type ObjectDistrMethInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| objAcqMethod | ObjectAcqMethod | M | 1 | Represents the object(s) acquisition method. |  |
| objAcqIds | array(Uri) | M | 1..N | Represents the object(s) to be ingested and distributed by the MBSTF during this MBS Distribution Session. |  |
| objIngUri | Uri | O | 0..1 | Represents a prefix substituted by the MBSTF with the Object distribution base URL prior to distribution of ingested objects. |  |
| objDistrUri | Uri | O | 0..1 | Represents a prefix substituted by the MBSTF in place of the Object ingest base URL prior to distribution of ingested objects. |  |

Editor's Note:	It's FFS on the contents of AddObjectDistrMethInfo to be aligned with TS 26.502.

##### 6.2.6.2.6	Type: PacketDistrMethInfo

Table 6.2.6.2.6-1: Definition of type PacketDistrMethInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pckIngMethod | PacketIngestMethod | M | 1 | Represents packets ingest method. |  |

Editor's Note:	It's FFS on the contents of AddPacketDistrMethInfo to be aligned with TS 26.502.

##### 6.2.6.2.7	Type MBSUserDataIngStatSubsc

Table 6.2.6.2.7-1: Definition of type MBSUserDataIngStatSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventsubscs | array(Event) | M | 1..N | Represents the required event(s) to be notified. |  |
| mbsIngSessionId | string | M | 1 | Represents an identifier for this MBS Distribution Session that is unique within the scope of the parent MBS User Service. |  |
| notifUri | Uri | M | 1 | Notification URI for event reporting. |  |

Editor's Note:	It's FFS on the contents of MBSUserDataIngStatSubsc to be aligned with TS 26.502.

##### 6.2.6.2.8	Type MBSUserDataIngStatNotif

Table 6.2.6.2.8-1: Definition of type MBSUserDataIngStatNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventNotifs | array(EventNotification) | M | 1..N | Represents Notifications about Individual Events. |  |
| mbsIngSessionId | string | M | 1 | Represents the identifier for the MBS user data ingestSession that is unique within the scope of the parent MBS User Service. |  |

Editor's Note:	It's FFS on the contents of MBSUserDataIngStatNotif to be aligned with TS 26.502.

##### 6.2.6.2.9	Type EventNotification

Table 6.2.6.2.9-1: Definition of type EventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsIngSessionId | string | M | 1 | Represents the identifier for this MBS Distribution Session that is unique within the scope of the parent MBS User Service. |  |
| statusEvent | Event | M | 1..N | Represents status change event(s). |  |
| timeStamp | DateTime | M | 1 | Represents the time at which the event is observed. |  |

Editor's Note:	It's FFS on the contents of EventNotification to be aligned with TS 26.502 and whether the timeStamp attribute is needed or not.

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

##### 6.2.6.3.3	Enumeration: DistributionMethod

The enumeration DistributionMethod represents MBS Distribution Method. It shall comply with the provisions defined in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration DistributionMehod

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| OBJECT | The Object Distribution Method is used to deliver binary objects to the MBS Client over an MBS Session that have been received from the MBS Application Provider over reference point Nmb8. |  |
| PACKET | The Packet Distribution Method is used to deliver packet streams to the MBS Client over an MBS Session that have been received from the MBS Application Provider over reference point Nmb8. |  |

##### 6.2.6.3.4	Enumeration: DistributionOperatingMode

The enumeration DistributionOperatingMode represents MBS distribution operating mode. It shall comply with the provisions defined in table 6.2.6.3.4-1.

Table 6.2.6.3.4-1: Enumeration DistributionOperatingMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| OBJECT_SINGLE | A single object is ingested by the MBSTF and distributed once. |  |
| OBJECT_COLLECTION | A set of objects described by a manifest is ingested by the MBSTF and distributed once. |  |
| OBJECT_CAROUSEL | A set of objects described by a manifest is ingested by the MBSTF and distributed multiple times. |  |
| OBJECT_STREAMING | A sequence of objects is ingested by the MBSTF and streamed in real time according to a schedule described in a presentation manifest (e.g. DASH MPD). |  |
| PACKET_FORWARD_ONLY | The payloads of IP packets ingested by the MBSTF are forwarded to the MB-UPF in new IP packets. (Layer 3 proxying). |  |
| PACKET_PROXY | The payloads of UDP packets ingested by the MBSTF are forwarded to the MB-UPF in new UDP packets (Layer 4 proxying). |  |
| PACKET_STREAMING | RTP packet payloads ingested by the MBSTF are forwarded to the MB-UPF in new RTP packets in real time according to the RTP packet timestamp. |  |

##### 6.2.6.3.5	Enumeration: State

The enumeration State represents the current state of the MBS Distribution Session. It shall comply with the provisions defined in table 6.2.6.3.5-1.

Table 6.2.6.3.5-1: Enumeration State

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACTIVE | The current state of the MBS Distribution Session is active. |  |
| DEACTIVATING | The current state of the MBS Distribution Session is deactivating. |  |
| ESTABLISHED | The current state of the MBS Distribution Session is established. |  |
| INACTIVE | The current state of the MBS Distribution Session is inactive. |  |

##### 6.2.6.3.6	Enumeration: Event

The enumeration Event represents MBS User Data Ingest Session status change event. It shall comply with the provisions defined in table 6.2.6.3.6-1.

Table 6.2.6.3.6-1: Enumeration Event

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DATA_INGEST_FAILURE | The MBS User Data Ingest is failed. |  |
| DELIVERY_STARTED | The MBS User Data delivery is started. |  |
| SESSION_TERMINATED | The MBS User Data Ingest Session is terminated. |  |

Editor's Note:	It's FFS on the contents of Event to be aligned with TS 26.502.

##### 6.2.6.3.7	Enumeration: ObjectAcqMethod

The enumeration ObjectAcqMethod represents the object(s) acquisition method. It shall comply with the provisions defined in table 6.2.6.3.7-1.

Table 6.2.6.3.7-1: Enumeration ObjectAcqMethod

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PULL | Represents the objects(s) are to be pulled by the MBSTF from the MBS Application Provider. |  |
| PUSH | Represents the objects(s) are to be pushed to the MBSTF by the MBS Application Provider. |  |

Editor's Note:	It's FFS on the contents of Event to be aligned with TS 26.502.

##### 6.2.6.3.8	Enumeration: PacketIngestMethod

The enumeration PacketIngestMethod represents packets ingest method. It shall comply with the provisions defined in table 6.2.6.3.8-1.

Table 6.2.6.3.8-1: Enumeration PacketIngestMethod

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| MULTICAST_INGEST | Indicates multicast ingest. |  |
| UNICAST_INGEST | Indicates unicast ingest. |  |

#### 6.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.2.6.5	Binary data

##### 6.2.6.5.1	Binary Data Types

Table 6.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.2.7	Error Handling

#### 6.2.7.1	General

For the Nmbsf_MBSUserDataIngestSession API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmbsf_MBSUserDataIngestSession API.

#### 6.2.7.2	Protocol Errors

No specific procedures for the Nmbsf_MBSUserDataIngestSession service are specified.

#### 6.2.7.3	Application Errors

The application errors defined for the Nmbsf_MBSUserDataIngestSession service are listed in Table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Nmbsf_MBSUserDataIngestSession API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmbsf_MBSUserDataIngestSession API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmbsf_MBSUserDataIngestSession API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmbsf_MBSUserDataIngestSession service.

The Nmbsf_MBSUserDataIngestSession API defines a single scope "nmbsf-mbsuserdataing" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI  specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

# A.2	Nmbsf_MBSUserService API

openapi: 3.0.0

info:

title: <API Name>

version: 1.0.0-alpha.1

description: |

<API Name> Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.<xxx> V<x.y.z>; <TS Name>.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.def/

servers:

- url: '{apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- <API name in lower letters with undesrscores>

paths:

# API specific definitions , below is an example

/subscriptions:

post:

summary: subscribe to notifications

operationId: CreateIndividualSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/<version>/subscriptions/{subId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<yyy>'

responses:

'204':

description: No Content, Notification was succesfull

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

get:

summary: retrieve subscription

operationId: GetIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: update subscription

operationId: ReplaceIndividualSubcription

tags:

- IndividualSubscription (Document)

tags:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was succesfully modified and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'204':

description: No Content. Resource was succesfully modified

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: unsubscribe from notifications

operationId: DeleteIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

<API name in lower letters . Composed names are separated with a hyphen>: Access to the <API Name> API

# API specific definitions

# A.3	Nmbsf_MBSUserDataIngestSession API

And so on if there are more than two services supported by the NF.

Annex B (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. Clause 4.3.1.6 of 3GPP TS 29.501 [5] describes the withdrawal of API versions.

# B.2	Nmbsf_MBSUserService API

The API versions listed in table B.2-1 are withdrawn for the Nmbsf_MBSUserService API.

Table B.2-1: Withdrawn API versions of the Nmbsf_MBSUserService service

| API version number | Remarks |
| --- | --- |

# B.3	Nmbsf_MBSUserDataIngestSession API

The API versions listed in table B.3-1 are withdrawn for the Nmbsf_MBSUserDataIngestSession API.

Table B.3-1: Withdrawn API versions of the Nmbsf_MBSUserDataIngestSession service

| API version number | Remarks |
| --- | --- |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | CT3#120-e |  | - | - | - | Skeleton for the new MBSF Services TS | 0.0.0 |
| 2022-02 | CT3#120-e | C3-221312 | - | - | - | Inclusion of C3-221312, C3-221313. | 0.1.0 |
| 2022-04 | CT3#121-e | C3-222484 | - | - | - | Inclusion of C3-222373, C3-222374, C3-222408, C3-222475, C3-222476. | 0.2.0 |
| 2022-05 | CT3#122-e | C3-223507 | - | - | - | Inclusion of C3-223604, C3-223310, C3-223311, C3-223605, C3-223313, C3-223314, C3-223315, C3-223316, C3-223317, C3-223410, C3-223543, C3-223412, C3-223544, C3-223545, C3-223546, C3-223547, C3-223417, C3-223418, C3-223419, C3-223745, C3-223421, C3-223422, C3-223423, C3-223747, C3-223748, C3-223750. | 0.3.0 |