| 3GPP TS 36.101 V17.2.0 (2021-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	32

1	Scope	34

2	References	34

3	Definitions, symbols and abbreviations	34

3.1	Definitions	34

3.2	Symbols	36

3.3	Abbreviations	39

4	General	41

4.1	Relationship between minimum requirements and test requirements	41

4.2	Applicability of minimum requirements	41

4.3	Void	41

4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE)	41

4.4	RF requirements in later releases	43

5	Operating bands and channel arrangement	43

5.1	General	43

5.2	Void	43

5.3	Void	43

5.4	Void	43

5.5	Operating bands	43

5.5A	Operating bands for CA	46

5.5B	Operating bands for UL-MIMO	66

5.5C	Operating bands for Dual Connectivity	66

5.5D	Operating bands for ProSe	69

5.5E	Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis	70

5.5F	Operating bands for category NB1 and NB2	70

5.5G	Operating bands for V2X Communication	71

5.6	Channel bandwidth	72

5.6.1	Channel bandwidths per operating band	73

5.6A	Channel bandwidth for CA	76

5.6A.1	Channel bandwidths per operating band for CA	78

5.6B	Channel bandwidth for UL-MIMO	186

5.6B.1	Void	186

5.6C	Channel bandwidth for Dual Connectivity	186

5.6C.1	Void	186

5.6D	Channel bandwidth for ProSe	186

5.6D.1	Channel bandwidths per operating band for ProSe	186

5.6F	Channel bandwidth for category NB1 and NB2	187

5.6G	Channel bandwidth for V2X Communication	188

5.6G.1	Channel bandwidths per operating band for V2X Communication	188

5.7	Channel arrangement	190

5.7.1	Channel spacing	190

5.7.1A	Channel spacing for CA	190

5.7.1F	Channel spacing for category NB1 and NB2	191

5.7.2	Channel raster	191

5.7.2A	Channel raster for CA	191

5.7.2F	Channel raster for category NB1 and NB2	191

5.7.3	Carrier frequency and EARFCN	191

5.7.3F	Carrier frequency and EARFCN for category NB1 and NB2	194

5.7.4	TX–RX frequency separation	194

5.7.4A	TX–RX frequency separation for CA	195

5.7.4E	TX–RX frequency separation for category M1 and M2	195

5.7.4F	TX–RX frequency separation for category NB1 and NB2	196

6	Transmitter characteristics	196

6.1	General	196

6.2	Transmit power	196

6.2.1	Void	196

6.2.2	UE maximum output power	196

6.2.2A	UE maximum output power for CA	199

6.2.2B	UE maximum output power for UL-MIMO	204

6.2.2C	 Void	207

6.2.2D	UE maximum output power for ProSe	207

6.2.2E	UE maximum output power for Category M1 and M2 UE	207

6.2.2F	UE maximum output power for category NB1 and NB2	208

6.2.2G	UE maximum output power for V2X Communication	209

6.2.3	UE maximum output power for modulation / channel bandwidth	211

6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA	212

6.2.3B	UE maximum output power for modulation / channel bandwidth for UL-MIMO	217

6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe	217

6.2.3E	UE maximum output power for modulation / channel bandwidth for category M1 and M2	218

6.2.3F	UE maximum output power for modulation / channel bandwidth for category NB1 and NB2	220

6.2.3G	UE maximum output power for modulation / channel bandwidth for V2X Communication	220

6.2.3G.1	MPR for Power class 3 V2X UE	220

6.2.3G.2	MPR for Power class 2 V2X UE	222

6.2.4	UE maximum output power with additional requirements	222

6.2.4A	UE maximum output power with additional requirements for CA	241

6.2.4A.1	A-MPR for CA_NS_01 for CA_1C	243

6.2.4A.2	A-MPR for CA_NS_02 for CA_1C	244

6.2.4A.3	A-MPR for CA_NS_03 for CA_1C	244

6.2.4A.4	A-MPR for CA_NS_04	245

6.2.4A.5	A-MPR for CA_NS_05 for CA_38C	248

6.2.4A.6	A-MPR for CA_NS_06	249

6.2.4A.7	A-MPR for CA_NS_07	250

6.2.4A.8	A-MPR for CA_NS_08	251

6.2.4A.9	Void	252

6.2.4A.10	A-MPR for CA_NS_10	252

6.2.4B	UE maximum output power with additional requirements for UL-MIMO	257

6.2.4D	UE maximum output power with additional requirements for ProSe	257

6.2.4E	UE maximum output power with additional requirements for category M1 and M2 UE	257

6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE	266

6.2.4G	UE maximum output power with additional requirements for V2X Communication	267

6.2.5	Configured transmitted power	269

6.2.5A	Configured transmitted power for CA	304

6.2.5B	Configured transmitted power for UL-MIMO	307

6.2.5C	Configured transmitted power for Dual Connectivity	308

6.2.5D	Configured transmitted power for ProSe	309

6.2.5F	Configured transmitted Power for category NB1 and NB2	311

6.2.5G	Configured transmitted power for V2X Communication	312

6.3	Output power dynamics	316

6.3.1	(Void)	316

6.3.2	Minimum output power	316

6.3.2.1	Minimum requirement	316

6.3.2A	 UE Minimum output power for CA	316

6.3.2A.1	Minimum requirement for CA	317

6.3.2B	 UE Minimum output power for UL-MIMO	317

6.3.2B.1	Minimum requirement	317

6.3.2C	 Void	317

6.3.2D	UE Minimum output power for ProSe	317

6.3.2F	UE Minimum output power for category NB1 and NB2	318

6.3.2G	UE Minimum output power for V2X Communication	318

6.3.3	Transmit OFF power	318

6.3.3.1.	Minimum requirement	318

6.3.3A	 UE Transmit OFF power for CA	319

6.3.3A.1	Minimum requirement for CA	319

6.3.3B	 UE Transmit OFF power for UL-MIMO	319

6.3.3B.1	Minimum requirement	319

6.3.3D	Transmit OFF power for ProSe	320

6.3.3F	Transmit OFF power for category NB1 and NB2	320

6.3.3G	Transmit OFF power for V2X Communication	320

6.3.4	ON/OFF time mask	321

6.3.4.1	General ON/OFF time mask	321

6.3.4.2	PRACH and SRS time mask	322

6.3.4.2.1	PRACH time mask	322

6.3.4.2.2	SRS time mask	323

6.3.4.3	Slot / Sub frame boundary time mask for subframe TTI	325

6.3.4.4	PUCCH / PUSCH / SRS time mask for subframe TTI	327

6.3.4.5	Symbol / Subslot boundary time mask for subslot TTI	329

6.3.4.6	Subslot PUCCH / subslot PUSCH / SRS time mask for subslot TTI	330

6.3.4.7	Symbol / Slot boundary time mask for slot TTI	333

6.3.4.8	Slot PUCCH / slot PUSCH / SRS time mask for slot TTI	334

6.3.4.9	Consecutive subslot and slot TTI or consecutive subslot and subframe TTI  time mask	334

6.3.4.10	Consecutive subframe and subslot TTI or consecutive slot and subslot TTI time mask	334

6.3.4.11	Consecutive TTI and slot TTI or consecutive slot TTI and TTI time mask	335

6.3.4A	ON/OFF time mask for CA	336

6.3.4B	ON/OFF time mask for UL-MIMO	336

6.3.4D	ON/OFF time mask for ProSe	336

6.3.4D.1	 General time mask for ProSe	336

6.3.4D.2	 PSSS/SSSS time mask	337

6.3.4D.3	 PSSS / SSSS / PSBCH time mask	338

6.3.4D.4	 PSSCH / SRS time mask	338

6.3.4F	ON/OFF time mask for category NB1 and NB2	338

6.3.4F.1	General ON/OFF time mask	338

6.3.4F.2	NPRACH time mask	339

6.3.4G	ON/OFF time mask for V2X Communication	339

6.3.4G.1	PSSS / SSSS / PSBCH time mask	340

6.3.5	Power Control	340

6.3.5.1	Absolute power tolerance	340

6.3.5.1.1	 Minimum requirements	340

6.3.5.2	Relative Power tolerance	341

6.3.5.2.1	Minimum requirements	341

6.3.5.3	Aggregate power control tolerance	342

6.3.5.3.1	Minimum requirement	342

6.3.5A	Power control for CA	342

6.3.5A.1	Absolute power tolerance	342

6.3.5A.1.1	 Minimum requirements	342

6.3.5A.2	Relative power tolerance	343

6.3.5A.2.1	Minimum requirements	343

6.3.5A.3	Aggregate power control tolerance	343

6.3.5A.3.1	 Minimum requirements	343

6.3.5B	Power control for UL-MIMO	344

6.3.5D	Power Control for ProSe	344

6.3.5D.1	Absolute power tolerance	344

6.3.5E	Power control for category M1 and M2	344

6.3.5E.1	Absolute power tolerance	344

6.3.5E.2	Relative Power tolerance	344

6.3.5E.3	Aggregate power control tolerance	345

6.3.5E.3.1	Minimum requirement	345

6.3.5F	Power Control for category NB1 and NB2	345

6.3.5F.1	Absolute power tolerance	346

6.3.5F.2	Relative power tolerance	346

6.3.5F.3	Aggregate power control tolerance for category NB1 and NB2	347

6.3.5F.3.1	Minimum requirement	347

6.3.5G	Power Control for V2X Communication	347

6.3.5G.1	Absolute power tolerance	347

6.4	Void	348

6.5	Transmit signal quality	348

6.5.1	Frequency error	348

6.5.1A 	Frequency error for CA	348

6.5.1B	Frequency error for UL-MIMO	348

6.5.1D	Frequency error for ProSe	348

6.5.1E	Frequency error for UE category M1 and M2	348

6.5.1F	Frequency error for UE category NB1 and NB2	349

6.5.1G	Frequency error for V2X Communication	349

6.5.2	Transmit modulation quality	349

6.5.2.1	 Error Vector Magnitude	350

6.5.2.1.1	Minimum requirement	350

6.5.2.2	 Carrier leakage	350

6.5.2.2.1	 Minimum requirements	351

6.5.2.3	In-band emissions	351

6.5.2.3.1	 Minimum requirements	351

6.5.2.4	 EVM equalizer spectrum flatness	352

6.5.2.4.1	Minimum requirements	352

6.5.2A	Transmit modulation quality for CA	353

6.5.2A.1	Error Vector Magnitude	354

6.5.2A.2	Carrier leakage for CA	354

6.5.2A.2.1	Minimum requirements	354

6.5.2A.3	In-band emissions	354

6.5.2A.3.1	Minimum requirement for CA	354

6.5.2B	Transmit modulation quality for UL-MIMO	357

6.5.2B.1	Error Vector Magnitude	357

6.5.2B.2	Carrier leakage	357

6.5.2B.3	In-band emissions	357

6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO	357

6.5.2D	Transmit modulation quality for ProSe	357

6.5.2D.1	Error Vector Magnitude	357

6.5.2D.2	Carrier leakage	358

6.5.2D.3	In-band emissions	358

6.5.2D.4	EVM equalizer spectrum flatness for ProSe	358

6.5.2E	Transmit modulation quality for category M1 and M2	358

6.5.2E.1	Error Vector Magnitude	358

The Error Vector Magnitude is defined in section 6.5.2.1.	358

6.5.2E.2	Carrier leakage	358

6.5.2E.2.1	Minimum requirements	358

6.5.2E.3	In-band emissions	358

6.5.2E.3.1	Minimum requirements	358

6.5.2F	Transmit modulation quality for Category NB1 and NB2	360

6.5.2F.1	Error Vector Magnitude	360

6.5.2F.2	Carrier leakage	360

6.5.2F.3	In-band emissions	360

6.5.2G	Transmit modulation quality for V2X Communication	361

6.5.2G.1	Error Vector Magnitude	361

6.5.2G.2	Carrier leakage	362

6.5.2G.3	In-band emissions	362

6.5.2G.4	EVM equalizer spectrum flatness	362

6.6	Output RF spectrum emissions	362

6.6.1	Occupied bandwidth	362

6.6.1.1	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	363

6.6.1A	Occupied bandwidth for CA	363

6.6.1B	 Occupied bandwidth for UL-MIMO	363

6.6.1F	Occupied bandwidth for category NB1 and NB2	364

6.6.1G	Occupied bandwidth for V2X Communication	364

6.6.2	Out of band emission	364

6.6.2.1	Spectrum emission mask	364

6.6.2.1.1	Minimum requirement	364

6.6.2.1A	Spectrum emission mask for CA	365

6.6.2.2	Additional spectrum emission mask	366

6.6.2.2.1	Minimum requirement (network signalled value "NS_03", “NS_11”, "NS_20", and “NS_21”)	366

6.6.2.2.2	Minimum requirement (network signalled value "NS_04")	367

6.6.2.2.3	Minimum requirement (network signalled value "NS_06" or “NS_07”)	367

6.6.2.2.4	Minimum requirement (network signalled value "NS_33" or “NS_34”)	368

6.6.2.2.5	Minimum requirement (network signalled value “NS_27” and “NS_43”)	368

6.6.2.2.6	Minimum requirement (network signalled value "NS_28”)	369

6.6.2.2.7	Minimum requirement (network signalled value "NS_35")	369

6.6.2.2A	Additional Spectrum Emission Mask for CA	370

6.6.2.2A.1	Minimum requirement (network signalled value "CA_NS_04")	370

6.6.2.2A.2	Minimum requirement CA_66B (network signalled value "CA_NS_09")	370

6.6.2.2A.3	Minimum requirement CA_66C (network signalled value "CA_NS_09")	371

6.6.2.2A.4	Minimum requirement CA_48B and CA_48C (network signalled value "CA_NS_10")	371

6.6.2.3	Adjacent Channel Leakage Ratio	372

6.6.2.3.1	Minimum requirement E-UTRA	372

6.6.2.3.1a	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	373

6.6.2.3.1A	Void	374

6.6.2.3.1Aa	Void	374

6.6.2.3.2	Minimum requirements UTRA	374

6.6.2.3.2A	Minimum requirement UTRA for CA	375

6.6.2.3.3A	Minimum requirements for CA E-UTRA	377

6.6.2.4	Void	378

6.6.2.4.1	Void	378

6.6.2A	Void	378

6.6.2B	 Out of band emission for UL-MIMO	378

6.6.2C	 Void	378

6.6.2D	 Out of band emission for ProSe	378

6.6.2F	Out of band emission for category NB1 and NB2	379

6.6.2F.1	Spectrum emission mask	379

6.6.2F.2	Additional Spectrum Emission Mask for Category NB1 and NB2	379

6.6.2F.2.1	Minimum requirement (network signalled value "NS_02")	379

6.6.2F.2.2	Minimum requirement (network signalled value "NS_03")	379

6.6.2F.3	Adjacent Channel Leakage Ratio for category NB1 and NB2	380

6.6.2G	 Out of band emission for V2X Communication	380

6.6.3	Spurious emissions	381

6.6.3.1	Minimum requirements	381

6.6.3.1A	Minimum requirements for CA	382

6.6.3.2	Spurious emission band UE co-existence	383

6.6.3.2A	Spurious emission band UE co-existence for CA	392

6.6.3.3	Additional spurious emissions	408

6.6.3.3.1 	Minimum requirement (network signalled value "NS_05")	409

6.6.3.3.2 	Minimum requirement (network signalled value “NS_07”)	409

6.6.3.3.3 	Minimum requirement (network signalled value “NS_08”)	409

6.6.3.3.4 	Minimum requirement (network signalled value “NS_09”)	409

6.6.3.3.5 	Minimum requirement (network signalled value "NS_12")	410

6.6.3.3.6 	Minimum requirement (network signalled value “NS_13”)	410

6.6.3.3.7 	Minimum requirement (network signalled value “NS_14”)	410

6.6.3.3.8 	Minimum requirement (network signalled value “NS_15”)	411

6.6.3.3.9 	Minimum requirement (network signalled value “NS_16”)	411

6.6.3.3.10 	Minimum requirement (network signalled value “NS_17”)	411

6.6.3.3.11 	Minimum requirement (network signalled value “NS_18”)	411

6.6.3.3.12 	Minimum requirement (network signalled value “NS_19”)	412

6.6.3.3.13 	Minimum requirement (network signalled value “NS_11”)	412

6.6.3.3.14	Minimum requirement (network signalled value “NS_20”)	412

6.6.3.3.15	Minimum requirement (network signalled value “NS_21”)	412

6.6.3.3.16	Minimum requirement (network signalled value "NS_22")	413

6.6.3.3.17	Minimum requirement (network signalled value “NS_23”)	413

6.6.3.3.18	Void	414

6.6.3.3.19	Minimum requirement (network signalled value "NS_04")	414

6.6.3.3.20	Minimum requirement (network signalled value “NS_24”)	414

6.6.3.3.21	Minimum requirement (network signalled value “NS_25”)	415

6.6.3.3.22	Minimum requirement (network signalled value “NS_26”)	415

6.6.3.3.23	Minimum requirement (network signalled value “NS_27” and “NS_43”)	415

6.6.3.3.24	Minimum requirement (network signalled value “NS_28”)	415

6.6.3.3.25	Minimum requirement (network signalled value “NS_29”)	416

6.6.3.3.26	Minimum requirement (network signalled value “NS_30”)	416

6.6.3.3.27	Minimum requirement (network signalled value “NS_31”)	417

6.6.3.3.28	Minimum requirement (network signalled value “NS_36”)	418

6.6.3.3.29	Minimum requirement (network signalled value “NS_38”)	418

6.6.3.3.30	Minimum requirement (network signalled value “NS_39”)	418

6.6.3.3.31	Minimum requirement (network signalled value “NS_40” and “NS_41”)	419

6.6.3.3.32	Minimum requirement (network signalled value “NS_42”)	419

6.6.3.3.33	Minimum requirement (network signalled value “NS_44”)	419

6.6.3.3.34	Minimum requirement (network signalled value “NS_45”)	419

6.6.3.3.35	Minimum requirement (network signalled value “NS_56”)	420

6.6.3.3A	Additional spurious emissions for CA	420

6.6.3.3A.1	Minimum requirement for CA_1C (network signalled value "CA_NS_01")	421

6.6.3.3A.2 	Minimum requirement for CA_1C (network signalled value "CA_NS_02")	421

6.6.3.3A.3	Minimum requirement for CA_1C (network signalled value "CA_NS_03")	421

6.6.3.3A.4	Minimum requirement for CA_38C (network signalled value "CA_NS_05")	422

6.6.3.3A.5	Minimum requirement for CA_7C (network signalled value "CA_NS_06")	422

6.6.3.3A.6	Minimum requirement for CA_39C and CA_39C-41A (network signalled value "CA_NS_07")	422

6.6.3.3A.7	Minimum requirement for CA_42C (network signalled value "CA_NS_08")	422

6.6.3.3A.8	Minimum requirement for CA_41C and CA_41D (network signalled value "CA_NS_04")	423

6.6.3.3A.9	Void	423

6.6.3.3A.10	Minimum requirement for CA_48B and CA_48C (network signalled value "CA_NS_10")	423

6.6.3A	Void	423

6.6.3B	 Spurious emission for UL-MIMO	424

6.6.3C	 Void	424

6.6.3D	 Spurious emission for ProSe	424

6.6.3F	Spurious emission for category NB1 and NB2	424

6.6.3G	Spurious emission for V2X Communication	424

6.6A	Void	428

6.6B	Void	428

6.7	Transmit intermodulation	428

6.7.1	Minimum requirement	428

6.7.1A	Minimum requirement for CA	429

6.7.1B	Minimum requirement for UL-MIMO	429

6.7.1F	Minimum requirement for category NB1 and NB2	429

6.7.1G	Minimum requirement for V2X Communication	430

6.8	Void	430

6.8A	Void	430

6.8B	 Time alignment error for UL-MIMO	430

6.8B.1	 Minimum Requirements	430

6.8C	Void	431

6.8D	Void	431

6.8E	Void	431

6.8F	Void	431

6.8G	Time alignment error	431

7	Receiver characteristics	431

7.1	General	431

7.2	Diversity characteristics	432

7.3	Reference sensitivity power level	432

7.3.1	Minimum requirements (QPSK)	432

7.3.1A	Minimum requirements (QPSK) for CA	471

7.3.1B	Minimum requirements (QPSK) for UL-MIMO	573

7.3.1D	Minimum requirements (QPSK) for ProSe	573

7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis	575

7.3.1F	Minimum requirements for UE category NB1 and NB2	584

7.3.1F.1	Reference sensitivity for UE category NB1 and NB2	584

7.3.1F.2	Void	585

7.3.1G	Minimum requirements (QPSK) for V2X	585

7.3.2	Void	587

7.4	Maximum input level	587

7.4.1	Minimum requirements	588

7.4.1A	Minimum requirements for CA	588

7.4.1B	Minimum requirements for UL-MIMO	589

7.4.1D	Minimum requirements for ProSe	589

7.4.1F	Minimum requirements for category NB1 and NB2	590

7.4.1G	Minimum requirements for V2X	590

7.4A	Void	591

7.4A.1	Void	591

7.5	Adjacent Channel Selectivity (ACS)	591

7.5.1	Minimum requirements	591

7.5.1A	Minimum requirements for CA	592

7.5.1B	Minimum requirements for UL-MIMO	596

7.5.1D	Minimum requirements for ProSe	596

7.5.1F	Minimum requirements for category NB1 and NB2	597

7.5.1G	Minimum requirements for V2X	598

7.6	Blocking characteristics	600

7.6.1	In-band blocking	600

7.6.1.1	Minimum requirements	600

7.6.1.1A	Minimum requirements for CA	602

7.6.1.1D	Minimum requirements for ProSe	605

7.6.1.1F	Minimum requirements for category NB1 and NB2	606

7.6.1.1G	Minimum requirements for V2X	606

7.6.2	Out-of-band blocking	608

7.6.2.1	Minimum requirements	608

7.6.2.1A	Minimum requirements for CA	610

7.6.2.1D	Minimum requirements for ProSe	613

7.6.2.1F	Minimum requirements for category NB1 and NB2	613

7.6.2.1G	Minimum requirements for V2X	614

7.6.3	Narrow band blocking	615

7.6.3.1	Minimum requirements	615

7.6.3.1A	Minimum requirements for CA	616

7.6.3.1D	Minimum requirements for ProSe	617

7.6A	Void	618

7.6B	Blocking characteristics for UL-MIMO	618

7.7	Spurious response	618

7.7.1	Minimum requirements	618

7.7.1A	Minimum requirements for CA	619

7.7.1B	Minimum requirements for UL-MIMO	620

7.7.1D	Minimum requirements for ProSe	620

7.7.1F	Minimum requirements for UE category NB1 and NB2	621

7.7.1G	Minimum requirements for V2X	621

7.8	Intermodulation characteristics	622

7.8.1	Wide band intermodulation	622

7.8.1.1	Minimum requirements	622

7.8.1A	Minimum requirements for CA	623

7.8.1B	Minimum requirements for UL-MIMO	626

7.8.1D	Minimum requirements for ProSe	626

7.8.1F	Minimum requirements for category NB1 and NB2	627

7.8.1G	Minimum requirements	627

7.8.2	Void	628

7.9	Spurious emissions	628

7.9.1	Minimum requirements	629

7.9.1A	Minimum requirements	629

7.10	Receiver image	630

7.10.1	Void	630

7.10.1A	Minimum requirements for CA	630

7.10.1G	Minimum requirements for V2X Communication	630

8	Performance requirement	617

8.1	General	617

8.1.1	Receiver antenna capability	617

8.1.1.1	Simultaneous unicast and MBMS operations	618

8.1.1.2	Dual-antenna receiver capability in idle mode	618

8.1.2	Applicability of requirements	618

8.1.2.1	Applicability of requirements for different channel bandwidths	618

8.1.2.2	Definition of CA capability	618

8.1.2.2A	Definition of dual connectivity capability	623

8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets	624

8.1.2.3A	Applicability and test rules for different dual connectivity configuration and bandwidth combination set	627

8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	628

8.1.2.3C	Applicability and test rules for SDR tests for 4Rx capable UEs	630

8.1.2.3D	Applicability and test rules for different CA with LAA SCell(s) configurations and bandwidth combination sets	630

8.1.2.3E	Applicability and test rules for SDR tests for 8Rx capable UEs	631

8.1.2.4	Test coverage for different number of component carriers	632

8.1.2.5	Applicability of performance requirements for Type B receiver	633

8.1.2.6	Applicability of performance requirements for 4Rx capable UEs	633

8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx	633

8.1.2.6.2	Applicability rule and antenna connection for CA and DC tests with 2Rx	635

8.1.2.6.3	Applicability rule and antenna connection for single carrier tests with 4Rx	635

8.1.2.6.4	Applicability rule for 256QAM tests	636

8.1.2.6.5	Applicability rule and antenna connection for CA and DC tests with 4Rx	636

8.1.2.6.6	Applicability rule for Type C with 4Rx	640

8.1.2.6.7	Applicability rule for 1024QAM tests	640

8.1.2.7	Applicability of Enhanced Downlink Control Channel Performance Requirements	640

8.1.2.8	Applicability of performance requirements for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations	642

8.1.2.8A	Applicability of performance requirements for UE supporting coverage enhancement	642

8.1.2.9	Applicability of SDR requirements for CA and LAA	643

8.1.2.10	Applicability of performance requirements for Multi-user Superposed Transmission	644

8.1.2.11	Applicability CRS interference mitigation receivers performance requirements	644

8.1.2.12	Applicability of performance requirements for 8Rx capable UEs	645

8.1.2.12.1	Applicability rule and antenna connection for single carrier PDSCH tests	645

8.1.2.12.2	Applicability rule and antenna connection for control channel tests	651

8.1.2.12.3	Applicability rule and antenna connection for CA and DC tests	651

8.1.3	UE category and UE DL category	652

8.2	Demodulation of PDSCH (Cell-Specific Reference Symbols)	652

8.2.1	FDD (Fixed Reference Channel)	652

8.2.1.1	Single-antenna port performance	653

8.2.1.1.1	Minimum Requirement	653

8.2.1.1.2	Void	659

8.2.1.1.3	Void	659

8.2.1.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	659

8.2.1.1.4A	Minimum Requirement 1 PRB allocation in presence of FeMBMS Unicast-mixed Cell under CA	659

8.2.1.2	Transmit diversity performance	660

8.2.1.2.1	Minimum Requirement 2 Tx Antenna Port	660

8.2.1.2.2	Minimum Requirement 4 Tx Antenna Port	661

8.2.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	661

8.2.1.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	663

8.2.1.2.4	Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	665

8.2.1.2.5	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	667

8.2.1.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	668

8.2.1.2.7	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	669

8.2.1.3	Open-loop spatial multiplexing performance	669

8.2.1.3.1	Minimum Requirement 2 Tx Antenna Port	669

8.2.1.3.1A	Soft buffer management test	673

8.2.1.3.1B	Enhanced Performance Requirement Type C –2Tx Antenna Ports	674

8.2.1.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	675

8.2.1.3.2	Minimum Requirement 4 Tx Antenna Port	676

8.2.1.3.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	676

8.2.1.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	680

8.2.1.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	682

8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	683

8.2.1.4	Closed-loop spatial multiplexing performance	684

8.2.1.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	684

8.2.1.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	685

8.2.1.4.1B	Enhanced Performance Requirement Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	685

8.2.1.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	687

8.2.1.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	690

8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	692

8.2.1.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	693

8.2.1.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	694

8.2.1.4.2A	Enhanced Performance Requirement Type C – Multi-layer Spatial Multiplexing 2Tx Antenna Ports	695

8.2.1.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	695

8.2.1.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	699

8.2.1.4.4	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	701

8.2.1.5	MU-MIMO	702

8.2.1.6	[Control channel performance: D-BCH and PCH]	702

8.2.1.7	Carrier aggregation with power imbalance	702

8.2.1.7.1	Minimum Requirement	702

8.2.1.8	Intra-band non-contiguous carrier aggregation with timing offset	703

8.2.1.8.1	Minimum Requirement	703

8.2.1.9	HST-SFN performance	704

8.2.1.9.1	Minimum Requirement	704

8.2.1.9.2	Minimum Requirement for Rel-16 further enhanced HST	707

8.2.1.10	Intra-band contiguous carrier aggregation with minimum channel spacing	707

8.2.1.10.1	Minimum Requirement	708

8.2.2	TDD (Fixed Reference Channel)	708

8.2.2.1	Single-antenna port performance	709

8.2.2.1.1	Minimum Requirement	709

8.2.2.1.2	Void	714

8.2.2.1.3	Void	714

8.2.2.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	714

8.2.2.2	Transmit diversity performance	714

8.2.2.2.1	Minimum Requirement 2 Tx Antenna Port	714

8.2.2.2.2	Minimum Requirement 4 Tx Antenna Port	715

8.2.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	716

8.2.2.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	717

8.2.2.2.4	Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	719

8.2.2.2.5	Minimum Requirement 2 Tx Antenna Port (when EIMTA-MainConfigServCell-r12 is configured)	721

8.2.2.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	721

8.2.2.2.7	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	723

8.2.2.2.8	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	724

8.2.2.3	Open-loop spatial multiplexing performance	725

8.2.2.3.1	Minimum Requirement 2 Tx Antenna Port	725

8.2.2.3.1A	Soft buffer management test	728

8.2.2.3.1B	Enhanced Performance Requirement Type C - 2Tx Antenna Ports	728

8.2.2.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	729

8.2.2.3.2	Minimum Requirement 4 Tx Antenna Port	730

8.2.2.3.3	Minimum Requirement 2Tx antenna port (demodulation subframe overlaps with aggressor cell ABS)	731

8.2.2.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	735

8.2.2.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	737

8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	738

8.2.2.4	Closed-loop spatial multiplexing performance	739

8.2.2.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	739

8.2.2.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	740

8.2.2.4.1B	Enhanced Performance Requirement Type A – Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	740

8.2.2.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	742

8.2.2.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	744

8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	746

8.2.2.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	747

8.2.2.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	749

8.2.2.4.2A	Enhanced Performance Requirement Type C Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	749

8.2.2.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	750

8.2.2.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	754

8.2.2.4.4	Void	755

8.2.2.4.5	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	755

8.2.2.5	MU-MIMO	756

8.2.2.6	[Control channel performance: D-BCH and PCH]	756

8.2.2.7	Carrier aggregation with power imbalance	756

8.2.2.7.1	Minimum Requirement	756

8.2.2.8	Intra-band contiguous carrier aggregation with minimum channel spacing	757

8.2.2.8.1	Minimum Requirement	757

8.2.2.9	HST-SFN performance	758

8.2.2.9.1	Minimum Requirement	758

8.2.2.9.2	Minimum Requirement for Rel-16 further enhanced HST	760

8.2.3	TDD FDD CA (Fixed Reference Channel)	761

8.2.3.1	Single-antenna port performance	762

8.2.3.1.1	Minimum Requirement for FDD PCell	762

8.2.3.1.2	Minimum Requirement for TDD PCell	766

8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port	770

8.2.3.2.1	Minimum Requirement for FDD PCell	770

8.2.3.2.1A	Soft buffer management test for FDD PCell	774

8.2.3.2.2	Minimum Requirement for TDD PCell	775

8.2.3.2.2A	Soft buffer management test for TDD PCell	779

8.2.3.3	Closed-loop spatial multiplexing performance 4Tx Antenna Port	780

8.2.3.3.1	Minimum Requirement for FDD PCell	780

8.2.3.3.2	Minimum Requirement for TDD PCell	784

8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity	788

8.2.3.5	HST-SFN performance	790

8.2.3.5.0	General	790

8.2.3.5.1	Minimum Requirement for FDD PCell	790

8.2.3.5.2	Minimum Requirement for TDD PCell	793

8.2.4	LAA	796

8.2.4.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port	796

8.2.4.1.1	FDD PCell (FDD single carrier)	796

8.2.4.1.2	TDD PCell (TDD single carrier)	800

8.3	Demodulation of PDSCH (User-Specific Reference Symbols)	803

8.3.1	FDD	803

8.3.1.1	Single-layer Spatial Multiplexing	804

8.3.1.1A	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	806

8.3.1.1B	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	809

8.3.1.1C	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference model	812

8.3.1.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	814

8.3.1.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference model	815

8.3.1.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	816

8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)	818

8.3.1.1H	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	820

8.3.1.1I	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	821

8.3.1.2	Dual-Layer Spatial Multiplexing	823

8.3.1.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	824

8.3.1.3	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	825

8.3.1.3.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	825

8.3.1.3.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	827

8.3.1.3.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	829

8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	831

8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	833

8.3.1.3.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	836

8.3.1.4	Performance Requirements for semiOpenLoop transmission	838

8.3.2	TDD	840

8.3.2.1	Single-layer Spatial Multiplexing	840

8.3.2.1A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	842

8.3.2.1B	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	844

8.3.2.1C	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	847

8.3.2.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference	850

8.3.2.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	852

8.3.2.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference	854

8.3.2.1G	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	855

8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)	857

8.3.2.1I	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	859

8.3.2.1J	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	860

8.3.2.2	Dual-Layer Spatial Multiplexing	862

8.3.2.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	862

8.3.2.3	Dual-Layer Spatial Multiplexing (with multiple CSI-RS configurations)	863

8.3.2.4	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	864

8.3.2.4.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	864

8.3.2.4.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	866

8.3.2.4.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	868

8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	870

8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	872

8.3.2.4.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	875

8.3.2.5	Performance Requirements for semiOpenLoop transmission	877

8.3.3	LAA	879

8.3.3.1	Dual-Layer Spatial Multiplexing with DM-RS	879

8.3.3.1.1	FDD PCell (FDD single carrier)	879

8.3.3.1.2	TDD Pcell (TDD single carrier)	884

8.4	Demodulation of PDCCH/PCFICH	888

8.4.1	FDD	888

8.4.1.1	Single-antenna port performance	889

8.4.1.2	Transmit diversity performance	889

8.4.1.2.1	Minimum Requirement 2 Tx Antenna Port	889

8.4.1.2.2	Minimum Requirement 4 Tx Antenna Port	889

8.4.1.2.3 Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	890

8.4.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	895

8.4.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port under Asynchronous Network	901

8.4.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	902

8.4.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	903

8.4.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	904

8.4.1.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	905

8.4.2	TDD	906

8.4.2.1	Single-antenna port performance	907

8.4.2.2	Transmit diversity performance	907

8.4.2.2.1	Minimum Requirement 2 Tx Antenna Port	907

8.4.2.2.2	Minimum Requirement 4 Tx Antenna Port	908

8.4.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	908

8.4.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	912

8.4.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	916

8.4.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	917

8.4.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	918

8.4.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	919

8.4.2.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	920

8.4.3	LAA	921

8.4.3.1	Transmit diversity performance	921

8.4.3.1.1	FDD Pcell (FDD single carrier)	921

8.4.3.1.2	TDD Pcell (TDD single carrier)	922

8.5	Demodulation of PHICH	923

8.5.1	FDD	924

8.5.1.1	Single-antenna port performance	924

8.5.1.2	Transmit diversity performance	924

8.5.1.2.1	Minimum Requirement 2 Tx Antenna Port	924

8.5.1.2.2	Minimum Requirement 4 Tx Antenna Port	925

8.5.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	925

8.5.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	927

8.5.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports under Asynchronous Network	930

8.5.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	931

8.5.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	932

8.5.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	933

8.5.2	TDD	934

8.5.2.1	Single-antenna port performance	935

8.5.2.2	Transmit diversity performance	935

8.5.2.2.1	Minimum Requirement 2 Tx Antenna Port	935

8.5.2.2.2	Minimum Requirement 4 Tx Antenna Port	936

8.5.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	936

8.5.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	938

8.5.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	940

8.5.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	941

8.5.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	942

8.5.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	943

8.6	Demodulation of PBCH	944

8.6.1	FDD	944

8.6.1.1	Single-antenna port performance	944

8.6.1.2	Transmit diversity performance	945

8.6.1.2.1	Minimum Requirement 2 Tx Antenna Port	945

8.6.1.2.2	Minimum Requirement 4 Tx Antenna Port	945

8.6.1.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	945

8.6.2	TDD	947

8.6.2.1	Single-antenna port performance	947

8.6.2.2	Transmit diversity performance	947

8.6.2.2.1	Minimum Requirement 2 Tx Antenna Port	947

8.6.2.2.2	Minimum Requirement 4 Tx Antenna Port	947

8.6.2.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	948

8.7	Sustained downlink data rate provided by lower layers	949

8.7.1	FDD (single carrier and CA)	949

8.7.2	TDD (single carrier and CA)	966

8.7.3	FDD (EPDCCH scheduling)	970

8.7.4	TDD (EPDCCH scheduling)	972

8.7.5	TDD FDD CA	974

8.7.5.1	Minimum Requirement FDD PCell	975

8.7.5.2	Minimum Requirement TDD PCell	983

8.7.6	FDD (DC)	992

8.7.7	TDD (DC)	999

8.7.8	TDD FDD (DC)	1002

8.7.9	FDD (4 Rx)	1005

8.7.10	TDD (4 Rx)	1007

8.7.11	TDD FDD CA (4 Rx)	1009

8.7.11.1	Void	1011

8.7.12	LAA	1011

8.7.12.1	FDD CA in licensed bands	1011

8.7.12.2	TDD CA in licensed bands	1013

8.7.12.3	TDD-FDD CA in licensed bands	1015

8.7.13	FDD DC (4 Rx)	1018

8.7.14	TDD DC (4 Rx)	1019

8.7.15	TDD FDD DC (4 Rx)	1021

8.7.16	FDD (1024QAM and up to 4Rx supported)	1023

8.7.17	TDD (1024QAM and up to 4 Rx supported)	1026

8.7.18	TDD FDD CA (1024QAM and up to 4 Rx supported)	1028

8.7.19	TDD (8 Rx)	1030

8.8	Demodulation of EPDCCH	1032

8.8.1	Distributed Transmission	1032

8.8.1.1	FDD	1032

8.8.1.1.1	Void	1033

8.8.1.2	TDD	1033

8.8.1.2.1	Void	1034

8.8.2	Localized Transmission with TM9	1034

8.8.2.1	FDD	1034

8.8.2.1.1	Void	1035

8.8.2.1.2	Void	1036

8.8.2.2	TDD	1036

8.8.2.2.1	Void	1037

8.8.2.2.2	Void	1037

8.8.3	Localized transmission with TM10 Type B quasi co-location type	1037

8.8.3.1	FDD	1037

8.8.3.2	TDD	1040

8.8.4	Enhanced Downlink Control Channel Performance Requirements Type A - Localized Transmission with CRS Interference Model	1043

8.8.4.1	FDD	1043

8.8.4.2	TDD	1044

8.8.5	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM9 Interference Model	1046

8.8.5.1	TDD	1046

8.8.6	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM3 Interference Model	1047

8.8.6.1	FDD	1047

8.9	Demodulation (single receiver antenna)	1048

8.9.1	PDSCH	1048

8.9.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1048

8.9.1.1.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1048

8.9.1.1.2	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1049

8.9.1.1.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1051

8.9.1.2	TDD (Fixed Reference Channel)	1054

8.9.1.2.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1054

8.9.1.2.2	 Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1055

8.9.1.2.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1058

8.9.2	PHICH	1060

8.9.2.1	FDD and half-duplex FDD	1060

8.9.2.1.1	Transmit diversity performance	1060

8.9.2.2	TDD	1060

8.9.2.2.1	Transmit diversity performance	1060

8.9.3	PBCH	1061

8.9.3.1	FDD and half-duplex FDD	1061

8.9.3.1.1	Transmit diversity performance	1061

8.9.3.2	TDD	1061

8.9.3.2.1	Transmit diversity performance	1061

8.9.4	PDCCH/PCFICH	1061

8.9.4.1	FDD and half-duplex FDD	1061

8.9.4.1.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1061

8.9.4.1.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1062

8.9.4.2	TDD	1063

8.9.4.2.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1063

8.9.4.2.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1064

8.10	Demodulation (4 receiver antenna ports)	1065

8.10.1	PDSCH	1065

8.10.1.1	FDD (Fixed Reference Channel)	1065

8.10.1.1.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1066

8.10.1.1.1A	Transmit diversity performance wit Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	1067

8.10.1.1.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1068

8.10.1.1.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1068

8.10.1.1.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (Cell-Specific Reference Symbols)	1069

8.10.1.1.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1070

8.10.1.1.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1071

8.10.1.1.5A	Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1074

8.10.1.1.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1075

8.10.1.1.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1076

8.10.1.1.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1078

8.10.1.1.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1079

8.10.1.1.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1080

8.10.1.1.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1080

8.10.1.1.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas (Cell-Specific Reference Symbols)	1081

8.10.1.1.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1081

8.10.1.1.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1083

8.10.1.1.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1084

8.10.1.1.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1085

8.10.1.1.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1086

8.10.1.1.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1087

8.10.1.1.14	HST-SFN performance	1091

8.10.1.2	TDD (Fixed Reference Channel)	1091

8.10.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1092

8.10.1.2.1A	Transmit diversity performance with Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	1093

8.10.1.2.2	Open-loop spatial multiplexing performance  with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1094

8.10.1.2.3	 Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1094

8.10.1.2.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1095

8.10.1.2.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1096

8.10.1.2.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1097

8.10.1.2.5A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	1100

8.10.1.2.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1101

8.10.1.2.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1102

8.10.1.2.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1104

8.10.1.2.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1105

8.10.1.2.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1106

8.10.1.2.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1106

8.10.1.2.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas	1107

8.10.1.2.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1108

8.10.1.2.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1109

8.10.1.2.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1110

8.10.1.2.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1112

8.10.1.2.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1113

8.10.1.2.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1115

8.10.1.2.14	HST-SFN performance	1119

8.10.2	PDCCH/PCFICH	1120

8.10.2.1	FDD	1120

8.10.2.1.1	Single-antenna port performance	1120

8.10.2.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1120

8.10.2.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1121

8.10.2.1.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1121

8.10.2.2	TDD	1123

8.10.2.2.1	Single-antenna port performance	1123

8.10.2.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1123

8.10.2.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1124

8.10.2.2.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1124

8.10.3	PHICH	1125

8.10.3.1	 FDD	1125

8.10.3.1.1	Single Tx Antenna Port performance	1125

8.10.3.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1126

8.10.3.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1126

8.10.3.2	TDD	1126

8.10.3.2.1	Single Tx Antenna Port performance	1127

8.10.3.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1127

8.10.3.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1128

8.10.4	ePDCCH	1128

8.10.4.1	Distributed Transmission with 4Rx	1128

8.10.4.1.1	FDD	1128

8.10.4.1.2	TDD	1129

8.10.4.2	Localized Transmission with TM9 and 4Rx	1130

8.10.4.2.1	FDD	1130

8.10.4.2.2	TDD	1131

8.11	Demodulation (UE supporting coverage enhancement)	1132

8.11.1	PDSCH	1133

8.11.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1133

8.11.1.1.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1133

8.11.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1137

8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1139

8.11.1.2	TDD (Fixed Reference Channel)	1144

8.11.1.2.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1144

8.11.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1149

8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1151

8.11.2	MPDCCH	1155

8.11.2.1	FDD and half-duplex FDD	1156

8.11.2.1.1	CE Mode A	1157

8.11.2.1.2	CE Mode B	1158

8.11.2.1.3	CE Mode A with TM9 interference model	1158

8.11.2.1.4	CE Mode A with CRS interference model	1160

8.11.2.1.5	CE Mode A and CE Mode B when CRS-ChEstMPDCCH-Config is configured	1161

8.11.2.2.5	CE Mode A and CE Mode B when CRS-ChEstMPDCCH-Config is configured	1162

8.11.2.2	TDD	1164

8.11.2.2.1	CE Mode A	1165

8.11.2.2.2	CE Mode B	1166

8.11.2.2.3	CE Mode A with TM9 interference model	1166

8.11.2.2.4	CE Mode A with CRS interference model	1167

8.11.3	PBCH	1168

8.11.3.1	FDD and half-duplex FDD	1169

8.11.3.1.1	Transmit diversity performance	1169

8.11.3.2	TDD	1170

8.11.3.2.1	Transmit diversity performance	1170

8.12	Demodulation of Narrowband IoT	1170

8.12.1	NPDSCH	1170

8.12.1.1.1	Minimum Requirements for In-band	1171

8.12.1.1.2	Minimum Requirements for Standalone/Guard-band	1172

8.12.1.1.3	Minimum Requirements for Standalone for UE Category NB2	1173

8.12.1.1.4	Minimum Requirements for Standalone for UE with multiple TBs interleaved transmission	1174

8.12.1.2	TDD	1175

8.12.1.2.1	Minimum Requirements for In-band	1175

8.12.1.2.2	Minimum Requirements for Standalone/Guard-band	1176

8.12.1.2.3	Minimum Requirements for Standalone for UE Category NB2	1177

8.12.2	NPDCCH	1177

8.12.2.1	Half-duplex FDD	1177

8.12.2.1.1	Single-antenna performance	1178

8.12.2.1.2	Transmit diversity performance	1179

8.12.2.2	TDD	1179

8.12.2.2.1	Single-antenna performance	1181

8.12.2.2.2	Transmit diversity performance	1181

8.12.3	Demodulation of NPBCH	1181

8.12.3.1	HD-FDD	1182

8.12.3.1.1	Single-antenna port performance with single NPBCH TTI	1182

8.12.3.1.2	Transmit diversity performance	1182

8.12.3.2	TDD	1182

8.12.3.2.1	Single-antenna port performance with single NPBCH TTI	1183

8.12.3.2.2	Transmit diversity performance	1183

8.13	Demodulation of PDSCH CA and DC(4 receiver antenna ports)	1183

8.13.1	FDD (CA and DC)	1183

8.13.1.1	Closed-loop spatial multiplexing performance	1183

8.13.1.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1183

8.13.1.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1187

8.13.1.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1189

8.13.1.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1191

8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1192

8.13.1.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1192

8.13.1.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1195

8.13.1.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1195

8.13.1.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1197

8.13.1.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1197

8.13.2	TDD (CA and DC)	1200

8.13.2.1	Closed-loop spatial multiplexing performance	1201

8.13.2.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1201

8.13.2.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1203

8.13.2.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1204

8.13.2.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1205

8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1207

8.13.2.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1207

8.13.2.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1211

8.13.2.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1211

8.13.3	TDD-FDD (CA and DC)	1213

8.13.3.1	 Closed-loop spatial multiplexing performance 4Tx Antenna Port	1214

8.13.3.1.1	Minimum Requirement for FDD PCell	1214

8.13.3.1.2	Minimum Requirement for TDD PCell	1217

8.13.3.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1221

8.13.3.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for FDD PCell	1221

8.13.3.2.2	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for TDD PCell	1224

8.13.3.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1227

8.13.3.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for FDD PCell	1227

8.13.3.3.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for TDD PCell	1229

8.13.3.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1231

8.13.3.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for FDD PCell	1231

8.13.3.4.2	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for TDD PCell	1234

8.13.3.5	 Closed-loop spatial multiplexing performance 4Tx Antenna Port for DC	1237

8.13.3.5.1	Minimum Requirement for FDD PCell	1237

8.13.3.5.2	Minimum Requirement for TDD PCell	1239

8.13.3.6	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with 256QAM	1241

8.13.3.6.1	Minimum Requirement for FDD PCell	1241

8.13.3.6.2	Minimum Requirement for TDD PCell	1243

8.13.3.7	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with Four layers	1244

8.13.3.7.1	Minimum Requirement for FDD PCell	1244

8.13.3.7.2	Minimum Requirement for TDD PCell	1246

8.14	Demodulation (UE supporting Short TTI)	1248

8.14.1	Slot-PDSCH and Subslot-PDSCH	1248

8.14.1.1	FDD (Fixed Reference Channel)	1248

8.14.1.1.1	Open-loop spatial multiplexing performance	1248

8.14.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1249

8.14.1.2	TDD (Fixed Reference Channel)	1251

8.14.1.2.1	Open-loop spatial multiplexing performance	1251

8.14.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1252

8.14.2	SPDCCH	1253

8.14.2.1	FDD	1253

8.14.2.1.1	Mimimum requirement	1254

8.14.2.2	TDD	1254

8.14.2.2.1	Mimimum requirement	1255

8.15	Demodulation (8 receiver antenna ports)	1256

8.15.1	PDSCH	1256

8.15.1.1	Void	1256

8.15.1.2	TDD (Fixed Reference Channel)	1256

8.15.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1256

8.15.1.2.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1257

8.15.1.2.3	8 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1257

8.15.2	CA	1259

8.15.2.1	Void	1259

8.15.2.2	TDD	1259

8.15.2.2.1	Eight Layer Spatial Multiplexing (User-Specific Reference Symbols)	1259

9	Reporting of Channel State Information	1261

9.1	General	1261

9.1.1	Applicability of requirements	1261

9.1.1.1	Applicability of requirements for different channel bandwidths	1261

9.1.1.2	Applicability and test rules for different CA configurations and bandwidth combination sets	1261

9.1.1.2A	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	1262

9.1.1.3	Test coverage for different number of componenet carriers	1263

9.1.1.4	Applicability of performance requirements for 4Rx capable UEs	1263

9.1.1.4.1	Applicability rule and antenna connection for single carrier tests with 2Rx	1263

9.1.1.4.2	Applicability rule and antenna connection for CA tests with 2Rx	1265

9.1.1.4.3	Applicability rule and antenna connection for single carrier tests with 4Rx	1266

9.1.1.5	Applicability of requirements for UEs supporting coverage enhancement	1266

9.2	 CQI reporting definition under AWGN conditions	1266

9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols)	1266

9.2.1.1	FDD	1266

9.2.1.2	TDD	1267

9.2.1.3	FDD (CSI measurements in case two CSI subframe sets are configured)	1268

9.2.1.4	TDD (CSI measurements in case two CSI subframe sets are configured)	1271

9.2.1.5	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1274

9.2.1.6	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1277

9.2.1.7	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1280

9.2.1.8	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1281

9.2.1.9	FDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1281

9.2.1.10	TDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1282

9.2.2	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1283

9.2.2.1	FDD	1283

9.2.2.2	TDD	1284

9.2.3	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1285

9.2.3.1	FDD	1285

9.2.3.1A	FDD (With channelMeasRestriction configured)	1286

9.2.3.2	TDD	1287

9.2.3.2A	TDD (With channelMeasRestriction configured)	1288

9.2.4	Minimum requirement PUCCH 1-1 (With Single CSI Process)	1289

9.2.4.1	FDD	1290

9.2.4.1A	FDD (With interferenceMeasRestriction configured)	1292

9.2.4.2	TDD	1295

9.2.4.2A	TDD (With interferenceMeasRestriction configured)	1298

9.2.5	Minimum requirement PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured)	1301

9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1304

9.2.6.1	Frame structure type 3 with FDD Pcell	1304

9.2.6.2	Frame structure type 3 with TDD Pcell	1306

9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1308

9.2.7.1	Frame structure type 3 wth FDD Pcell	1308

9.2.7.2	Frame structure type 3 wth TDD Pcell	1310

9.3	CQI reporting under fading conditions	1314

9.3.1	Frequency-selective scheduling mode	1314

9.3.1.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1314

9.3.1.1.1	FDD	1314

9.3.1.1.2	TDD	1315

9.3.1.1.3	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1316

9.3.1.1.4	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1320

9.3.1.1.5	TDD (when csi-SubframeSet –r12 is configured)	1323

9.3.1.2	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1325

9.3.1.2.1	FDD	1325

9.3.1.2.2	TDD	1326

9.3.1.2.3	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1328

9.3.1.2.4	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1329

9.3.1.2.5	Void	1330

9.3.1.2.6	TDD (when csi-SubframeSet –r12 is configured with one CSI process)	1330

9.3.2	Frequency non-selective scheduling mode	1334

9.3.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1334

9.3.2.1.1	FDD	1334

9.3.2.1.2	TDD	1336

9.3.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1338

9.3.2.2.1	FDD	1338

9.3.2.2.2	TDD	1339

9.3.3	Frequency-selective interference	1341

9.3.3.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbol)	1341

9.3.3.1.1	FDD	1341

9.3.3.1.2	TDD	1342

9.3.3.2	Void	1343

9.3.3.2.1	Void	1343

9.3.3.2.2	Void	1343

9.3.4	UE-selected subband CQI	1343

9.3.4.1	Minimum requirement PUSCH 2-0 (Cell-Specific Reference Symbols)	1343

9.3.4.1.1	FDD	1343

9.3.4.1.2	TDD	1344

9.3.4.2	Minimum requirement PUCCH 2-0 (Cell-Specific Reference Symbols)	1345

9.3.4.2.1	FDD	1345

9.3.4.2.2	TDD	1347

9.3.5	Additional requirements for enhanced receiver Type A	1349

9.3.5.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1349

9.3.5.1.1	FDD	1349

9.3.5.1.2	TDD	1350

9.3.5.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1352

9.3.5.2.1	FDD	1352

9.3.5.2.2	TDD	1355

9.3.6	Minimum requirement (With multiple CSI processes)	1358

9.3.6.1	FDD	1358

9.3.6.2	TDD	1363

9.3.7	Minimum requirement PUSCH 3-2	1367

9.3.7.1	FDD	1367

9.3.7.2	TDD	1368

9.3.8	Additional requirements for enhanced receiver Type B	1370

9.3.8.1	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1370

9.3.8.1.1	FDD	1370

9.3.8.1.2	TDD	1371

9.3.8.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1373

9.3.8.2.1	FDD	1373

9.3.8.2.2	TDD	1375

9.3.8.3	Minimum requirement with CSI process	1378

9.3.8.3.1	FDD	1378

9.3.8.3.2	TDD	1381

9.4	Reporting of Precoding Matrix Indicator (PMI)	1384

9.4.1	Single PMI	1385

9.4.1.1	Minimum requirement PUSCH 3-1 (Cell-Specific Reference Symbols)	1385

9.4.1.1.1	FDD	1385

9.4.1.1.2	TDD	1386

9.4.1.2	 Minimum requirement PUCCH 2-1 (Cell-Specific Reference Symbols)	1387

9.4.1.2.1	FDD	1387

9.4.1.2.2	TDD	1389

9.4.1.3	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1390

9.4.1.3.1	FDD	1390

9.4.1.3.2	TDD	1393

9.4.1.3.3	FDD (with Class A 12Tx codebook)	1396

9.4.1.3.4	TDD (with Class A 12Tx codebook)	1398

9.4.1.3.5	FDD (with Class A 24Tx codebook)	1401

9.4.1.3.6	TDD (with Class A 24Tx codebook)	1403

9.4.1.4	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1406

9.4.1.4.1	FDD (with 4Tx enhanced codebook)	1406

9.4.1.4.2	TDD (with 4Tx enhanced codebook)	1408

9.4.1.4.3	FDD (with Class B alternative codebook for one CSI-RS resource configured)	1411

9.4.1.4.4	TDD (with Class B alternative codebook for one CSI-RS resource configured)	1413

9.4.1a	Void	1416

9.4.1a.1	Void	1416

9.4.1a.1.1	Void	1416

9.4.1a.1.2	Void	1416

9.4.2	Multiple PMI	1416

9.4.2.1	Minimum requirement PUSCH 1-2 (Cell-Specific Reference Symbols)	1416

9.4.2.1.1	FDD	1416

9.4.2.1.2	TDD	1417

9.4.2.2	 Minimum requirement PUSCH 2-2 (Cell-Specific Reference Symbols)	1418

9.4.2.2.1	FDD	1418

9.4.2.2.2	TDD	1419

9.4.2.3	Minimum requirement PUSCH 1-2 (CSI Reference Symbol)	1420

9.4.2.3.1	FDD	1420

9.4.2.3.2	TDD	1422

9.4.2.3.3	FDD (with 4Tx enhanced codebook)	1425

9.4.2.3.4	TDD (with 4Tx enhanced codebook)	1427

9.4.2.3.5	FDD (with Class A 16Tx codebook)	1429

9.4.2.3.6	TDD (with Class A 16Tx codebook)	1432

9.4.2.3.7	FDD (with Class A 32Tx codebook)	1435

9.4.2.3.8	TDD (with Class A 32Tx codebook)	1438

9.4.2.3.9	FDD (with Class A 16Tx advanced codebook)	1441

9.4.2.3.10	TDD (with Class A 16Tx advanced codebook)	1444

9.4.3	Void	1447

9.5	Reporting of Rank Indicator (RI)	1447

9.5.1	Minimum requirement (Cell-Specific Reference Symbols)	1447

9.5.1.1	FDD	1447

9.5.1.2	TDD	1448

9.5.2	Minimum requirement (CSI Reference Symbols)	1449

9.5.2.1	FDD	1449

9.5.2.2	TDD	1451

9.5.3	Minimum requirement (CSI measurements in case two CSI subframe sets are configured)	1453

9.5.3.1	FDD	1453

9.5.3.2	TDD	1456

9.5.4	Minimum requirement (CSI measurements in case two CSI subframe sets are configured and CRS assistance information are configured)	1459

9.5.4.1	FDD	1459

9.5.4.2	TDD	1462

9.5.5	Minimum requirement (with CSI process)	1465

9.5.5.1	FDD	1466

9.5.5.2	TDD	1469

9.6	 Additional requirements for carrier aggregation	1472

9.6.1	 Periodic reporting on multiple cells (Cell-Specific Reference Symbols)	1472

9.6.1.1	FDD	1472

9.6.1.2	TDD	1478

9.6.1.3	TDD-FDD CA with FDD PCell	1484

9.6.1.4	TDD-FDD CA with TDD PCell	1489

9.7	CSI reporting (Single receiver antenna)	1495

9.7.1	CQI reporting definition under AWGN conditions	1496

9.7.1.1	FDD and half-duplex FDD	1496

9.7.1.2	TDD	1496

9.7.1.3 FDD (Category 1bis UE)	1497

9.7.1.4 TDD (Category 1bis UE)	1498

9.7.2	CQI reporting under fading conditions	1499

9.7.2.1	FDD and half-duplex FDD	1499

9.7.2.2	TDD	1500

9.7.2.3 FDD (Category 1bis UE)	1501

9.7.2.4 TDD (Category 1bis UE)	1502

9.8	CSI reporting (UE supporting coverage enhancement)	1503

9.8.1	CQI reporting definition under AWGN conditions	1503

9.8.1.1	FDD and half-duplex FDD	1503

9.8.1.2	TDD	1504

9.8.2	UE-selected subband CQI	1505

9.8.2.1	FDD and half-duplex FDD	1505

9.8.2.2	TDD	1509

9.8.3	CQI reporting definition for UE supporting 64QAM under AWGN	1512

9.8.3.1	FDD and half-duplex FDD	1512

9.8.3.2	TDD	1513

9.8.4	CQI reporting definition for UE supporting alternative table under AWGN	1514

9.8.4.1	FDD and half-duplex FDD	1514

9.8.4.2	TDD	1515

9.8.5	PMI reporting with PUCCH 1-1 (CSI Reference Symbol)	1516

9.8.5.1	FDD	1517

9.8.5.2	TDD	1518

9.9	CSI reporting for 4Rx UE	1519

9.9.1	CQI reporting definition under AWGN conditions	1519

9.9.1.1	Minimum requirement PUCCH 1-0 with Rank 1 (Cell-Specific Reference Symbols)	1520

9.9.1.1.1	FDD	1520

9.9.1.1.2	TDD	1520

9.9.1.2	Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)	1521

9.9.1.2.1	FDD	1521

9.9.1.2.2	TDD	1522

9.9.1.3	 Minimum requirement PUCCH 1-1 with Rank 4 (Cell-Specific Reference Symbols)	1523

9.9.1.3.1	FDD	1524

9.9.1.3.2	TDD	1524

9.9.1.4	Minimum requirement PUCCH 1-1 with Rank 3 (CSI Reference Symbols)	1525

9.9.1.4.1	FDD	1525

9.9.1.4.2	TDD	1526

9.9.2	CQI reporting definition under fading conditions	1527

9.9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol) for enhanced receiver Type A	1527

9.9.2.1.1	FDD	1528

9.9.2.1.2	TDD	1529

9.9.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol) for enhanced receiver Type A	1531

9.9.2.2.1	FDD	1531

9.9.2.2.2	TDD	1534

9.9.3	Reporting of Precoding Matrix Indicator (PMI) for 4Rx UE	1537

9.9.3.1	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1538

9.9.3.1.1	TDD	1538

9.9.4	 Reporting of Rank Indicator (RI)	1540

9.9.4.1	Minimum requirement (Cell-Specific Reference Symbols)	1540

9.9.4.1.1	FDD	1540

9.9.4.1.2	TDD	1542

9.9.4.2	Minimum requirement (CSI Reference Symbols)	1543

9.9.4.2.1	FDD	1543

9.9.4.2.2	TDD	1545

9.10	Reporting of CSI-RS Resource Indicator (CRI)	1547

9.10.1	Minimum requirement (PUSCH 3-1)	1548

9.10.1.1	FDD	1548

9.10.1.2	TDD	1550

9.10.2	Minimum requirement (PUSCH 3-1, QCL Type C)	1552

9.10.2.1	FDD	1552

9.10.2.2	TDD	1555

9.11	Reporting of Hybrid Channel state information	1558

9.11.1	Minimum requirement (with eMIMO-Type configured as Class B with more than one CSI-RS resource configured and eMIMO-Type2 as Class B with one CSI-RS resource configured)	1558

9.11.1.1	FDD	1559

9.11.1.2	TDD	1561

9.12	CSI reporting (UE supporting Short TTI)	1563

9.12.1	CQI reporting under fading conditions (Cell-Specific Reference Symbol)	1563

9.12.1.1	FDD	1563

9.12.1.2	TDD	1566

9.12.2	CQI reporting under fading conditions (CSI Reference Symbol)	1567

9.12.2.1	FDD	1567

9.12.2.2	TDD	1570

9.13	CSI reporting for 8Rx UE	1572

9.13.1	CQI reporting definition under AWGN conditions	1572

9.13.1.1	Minimum requirement PUCCH 1-1 with Rank 4 (CSI Reference Symbols)	1572

9.13.1.2.1	Void	1572

9.13.1.2.2	TDD	1572

10	Performance requirement (MBMS)	1573

10.1	FDD (Fixed Reference Channel)	1573

10.1.1	Minimum requirement	1574

10.2	TDD (Fixed Reference Channel)	1575

10.2.1	Minimum requirement	1575

10.3	FDD (Fixed Reference Channel) with FeMBMS	1576

10.3.1	Minimum requirement for FeMBMS Unicast-mixed Cell under CA	1576

10.3.1.1	Minimum requirement with 1.25kHz subcarrier spacing	1576

10.3.1.2	Minimum requirement with 7.5kHz subcarrier spacing	1577

10.3.2	Minimum requirement for FeMBMS Unicast-mixed Cell as Non-Serving Cell	1578

10.3.2.1	Minimum requirement with 1.25kHz subcarrier spacing	1578

10.3.2.2	Minimum requirement with 7.5kHz subcarrier spacing	1579

10.3.3	Minimum requirement for MBMS Dedicated cell	1580

10.3.3.1	Minimum requirement with 1.25kHz subcarrier spacing	1580

10.3.3.2	Minimum requirement with 7.5kHz subcarrier spacing	1581

10.3.3.3	Minimum requirement with 15kHz subcarrier spacing	1582

10.4 FDD with 5G terrestrial broadcast	1583

10.4.1	Minimum requirement for PMCH decoding	1583

10.4.1.1	Minimum requirement with 0.37kHz subcarrier spacing	1583

10.4.1.2	Minimum requirement with 2.5kHz subcarrier spacing	1584

10.4.2	Minimum requirement for CAS detection	1585

10.4.2.1	Minimum requirement for PBCH detection	1585

11	Performance requirement (ProSe Direct Discovery)	1585

11.1	General	1586

11.1.1	Applicability of requirements	1586

11.1.2	Reference DRX configuration	1586

11.2	Demodulation of PSDCH (single link performance)	1586

11.2.1	FDD (in-coverage)	1586

11.2.2	TDD (in-coverage)	1587

11.2.3	FDD (out-of-coverage)	1588

11.3	Power imbalance performance with two links	1589

11.3.1	FDD	1589

11.3.2	TDD	1590

11.4	Multiple timing reference test	1591

11.4.1	FDD	1592

11.5	Maximum Sidelink processes test	1593

11.5.1	FDD	1593

11.5.2	TDD	1594

12	Performance requirement (ProSe Direct Communication)	1596

12.1	General	1596

12.1.1	Applicability of requirements	1596

12.1.1.1	Applicability of requirements for different channel bandwidths	1596

12.1.1.2	Test coverage for different number of component carriers	1596

12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets	1596

12.1.2	Reference DRX configuration	1597

12.2	Demodulation of PSSCH	1597

12.2.1	FDD	1597

12.3	Demodulation of PSCCH	1598

12.3.1	FDD	1599

12.4	Demodulation of PSBCH	1600

12.4.1	FDD	1600

12.5	Power imbalance performance with two links	1600

12.5.1	 FDD	1600

12.6	Multiple timing reference test	1602

12.6.1	FDD	1602

12.7	Maximum Sidelink processes test	1605

12.7.1	FDD	1605

12.8	Sustained downlink data rate with active Sidelink	1606

13	Void	1608

14	Performance requirement (V2X Sidelink Communication)	1608

14.1	General	1608

14.1.1	Applicability of requirements	1608

14.2	Demodulation of PSSCH	1609

14.3	Demodulation of PSCCH	1610

14.4	Power imbalance performance with two links	1610

14.5	Demodulation of PSBCH	1611

14.6	Demodulation of PSSCH with eNB based synchronization	1612

14.7	Soft buffer test	1612

14.8	PSCCH/PSSCH decoding capability test	1613

14.9	Sustained downlink data rate with active sidelink	1614

14.10	Soft buffer test (CA)	1615

14.11	PSCCH/PSSCH decoding capability test (CA)	1616

Annex A (normative):  Measurement channels	1618

A.1	General	1618

A.2	UL reference measurement channels	1618

A.2.1	General	1618

A.2.1.1	Applicability and common parameters	1618

A.2.1.2	Determination of payload size	1618

A.2.1.3	Overview of UL reference measurement channels	1619

A.2.2	Reference measurement channels for FDD	1642

A.2.2.1	Full RB allocation	1642

A.2.2.1.1	QPSK	1642

A.2.2.1.2	16-QAM	1644

A.2.2.1.3	64-QAM	1646

A.2.2.1.4	256 QAM	1647

A.2.2.2	Partial RB allocation	1647

A.2.2.2.1	QPSK	1648

A.2.2.2.2	16-QAM	1651

A.2.2.2.3	64-QAM	1653

A.2.2.2.4	256 QAM	1655

A.2.2.3	Void	1655

A.2.2.4	subPRB allocation	1656

A.2.3	Reference measurement channels for TDD	1656

A.2.3.1	Full RB allocation	1656

A.2.3.1.1	QPSK	1656

A.2.3.1.2	16-QAM	1659

A.2.3.1.3	64-QAM	1661

A.2.3.1.4	256 QAM	1662

A.2.3.2	Partial RB allocation	1662

A.2.3.2.1	QPSK	1663

A.2.3.2.2	16-QAM	1667

A.2.3.2.3	64-QAM	1672

A.2.3.2.4	256 QAM	1674

A.2.3.3	Void	1674

A.2.3.4	subPRB allocation	1675

A.2.4	Reference measurement channels for UE category NB1	1675

A.2.5	Reference measurement channels for LAA	1676

A.2.5.1	Full RB allocation	1676

A.2.5.1.1	QPSK	1676

A.2.5.1.2	16QAM	1677

A.2.5.1.3	64QAM	1677

A.2.5.2	Partial RB allocation	1677

A.2.5.2.1	QPSK	1678

A.2.5.2.2	16QAM	1678

A.2.5.2.3	64QAM	1679

A.3	DL reference measurement channels	1679

A.3.1	General	1679

A.3.1.1	Overview of DL reference measurement channels	1680

A.3.2	Reference measurement channel for receiver characteristics	1705

A.3.3	Reference measurement channels for PDSCH performance requirements (FDD)	1735

A.3.3.1	Single-antenna transmission (Common Reference Symbols)	1735

A.3.3.2	Multi-antenna transmission (Common Reference Symbols)	1740

A.3.3.2.1	Two antenna ports	1740

A.3.3.2.2	Four antenna ports	1748

A.3.3.3	Reference Measurement Channel for UE-Specific Reference Symbols	1753

A.3.3.3.0	Two antenna ports (no CSI-RS)	1753

A.3.3.3.1	Two antenna port (CSI-RS)	1754

A.3.3.3.2	Four antenna ports (CSI-RS)	1757

A.3.3.3.2A	Eight antenna ports (CSI-RS)	1764

A.3.3.3.3	Twelve antenna port (CSI-RS)	1766

A.3.3.3.4	Sixteen antenna port (CSI-RS)	1767

A.3.3.3.5	Twenty-four antenna port (CSI-RS)	1768

A.3.3.3.6	Thirty-two antenna port (CSI-RS)	1769

A.3.4	Reference measurement channels for PDSCH performance requirements (TDD)	1771

A.3.4.1	Single-antenna transmission (Common Reference Symbols)	1771

A.3.4.2	Multi-antenna transmission (Common Reference Signals)	1779

A.3.4.2.1	Two antenna ports	1779

A.3.4.2.2	Four antenna ports	1792

A.3.4.3	Reference Measurement Channels for UE-Specific Reference Symbols	1799

A.3.4.3.1	Single antenna port (Cell Specific)	1799

A.3.4.3.2	Two antenna ports (Cell Specific)	1800

A.3.4.3.3	Two antenna ports (CSI-RS)	1802

A.3.4.3.4	Four antenna ports (CSI-RS)	1810

A.3.4.3.5	Eight antenna ports (CSI-RS)	1817

A.3.4.3.6	Twelve antenna ports (CSI-RS)	1823

A.3.4.3.7	Sixteen antenna ports (CSI-RS)	1824

A.3.4.3.8	Twenty-four antenna ports (CSI-RS)	1825

A.3.4.3.9	Thirty-two antenna ports (CSI-RS)	1826

A.3.5	Reference measurement channels for PDCCH/PCFICH performance requirements	1828

A.3.5.1	FDD	1828

A.3.5.2	TDD	1828

A.3.5.3	LAA	1829

A.3.6	Reference measurement channels for PHICH performance requirements	1829

A.3.7	Reference measurement channels for PBCH performance requirements	1829

A.3.8	Reference measurement channels for MBMS performance requirements	1830

A.3.8.1	FDD	1830

A.3.8.2	TDD	1835

A.3.9	Reference measurement channels for sustained downlink data rate provided by lower layers	1837

A.3.9.1	FDD	1837

A.3.9.2	TDD	1841

A.3.9.3	FDD (EPDCCH scheduling)	1855

A.3.9.4	TDD (EPDCCH scheduling)	1856

A.3.9.5	LAA	1858

A.3.10	Reference Measurement Channels for EPDCCH performance requirements	1860

A.3.10.1	FDD	1860

A.3.10.2	TDD	1860

A.3.11	Reference Measurement Channels for MPDCCH performance requirements	1860

A.3.11.1	FDD and half-duplex FDD	1860

A.3.11.2	TDD	1861

A.3.12	Reference measurement channels for NPDSCH performance requirements	1862

A.3.12.1	In-band	1862

A.3.12.1.2	Two-antenna transmission	1862

A.3.12.2	Standalone/Guard-band	1864

A.3.12.2.1 Single-antenna transmission	1864

A.3.13	Reference measurement channels for NPDCCH performance requirements	1867

A.3.13.1	Half-duplex FDD	1867

A.3.13.2	TDD	1868

A.3.14	Reference measurement channels for NPBCH performance requirements for Cat NB1 UEs	1868

A.3.15	Reference Measurement Channels for LAA SCell with frame structure Type-3	1869

A.3.15.1	Multi-antenna transmission (Common Reference Symbols)	1869

A.3.15.1.1	Four antenna ports	1869

A.3.15.2	Reference Measurement Channel for UE-Specific Reference Symbols	1869

A.3.15.2.1	Two antenna ports (CSI-RS)	1869

A.3.16	Reference measurement channels for Slot-PDSCH and Subslot-PDSCH performance requirements	1871

A.3.16.1	FDD	1871

A.3.16.2	TDD	1875

A.3.17	Reference measurement channels for SPDCCH performance requirements	1876

A.3.17.1	FDD	1876

A.3.17.2	TDD	1877

A.4	CSI reference measurement channels	1877

A.5	OFDMA Channel Noise Generator (OCNG)	1892

A.5.1	OCNG Patterns for FDD	1892

A.5.1.1	OCNG FDD pattern 1: One sided dynamic OCNG FDD pattern	1892

A.5.1.2	OCNG FDD pattern 2: Two sided dynamic OCNG FDD pattern	1893

A.5.1.3	OCNG FDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1893

A.5.1.3A	OCNG FDD pattern 3A: 49 RB OCNG allocation with MBSFN enhancement in 10 MHz	1894

A.5.1.4	OCNG FDD pattern 4: One sided dynamic OCNG FDD pattern for MBMS transmission	1894

A.5.1.4A	OCNG FDD pattern 4A: One sided dynamic OCNG FDD pattern for enhanced MBMS transmission	1895

A.5.1.5	OCNG FDD pattern 5: One sided dynamic 16QAM modulated OCNG FDD pattern	1896

A.5.1.6	OCNG FDD pattern 6: dynamic OCNG FDD pattern when user data is in 2 non-contiguous blocks	1897

A.5.1.8	OCNG FDD pattern 8: Dynamic OCNG FDD pattern for TM10 transmission	1898

A.5.2	OCNG Patterns for TDD	1899

A.5.2.1	OCNG TDD pattern 1: One sided dynamic OCNG TDD pattern	1899

A.5.2.2	OCNG TDD pattern 2: Two sided dynamic OCNG TDD pattern	1900

A.5.2.3	OCNG TDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1901

A.5.2.4	OCNG TDD pattern 4: One sided dynamic OCNG TDD pattern for MBMS transmission	1901

A.5.2.5	OCNG TDD pattern 5: One sided dynamic 16QAM modulated OCNG TDD pattern	1902

A.5.2.6	OCNG TDD pattern 6: dynamic OCNG TDD pattern when user data is in 2 non-contiguous blocks	1903

A.5.2.8	OCNG TDD pattern 8: Dynamic OCNG TDD pattern for TM10 transmission	1904

A.5.3	OCNG Patterns for Narrowband IoT	1905

A.5.3.1	Narrowband IoT OCNG pattern 1	1905

A.5.4	OCNG Patterns for frame structure type 3	1906

A.5.4.1	OCNG FS3 pattern 1: One sided dynamic OCNG frame structure type 3 pattern	1906

A.5.4.2	OCNG FS3 pattern 2: Two sided dynamic OCNG frame structure 3 pattern	1907

A.6	Sidelink reference measurement channels	1908

A.6.1	General	1908

A.6.1.1	Overview of ProSe reference measurement channels	1908

A.6.2	Reference measurement channel for receiver characteristics	1909

A.6.3	Reference measurement channels for PSDCH performance requirements	1911

A.6.4	Reference measurement channels for PSCCH performance requirements	1912

A.6.5	Reference measurement channels for PSSCH performance requirements	1912

A.6.6	Reference measurement channels for PSBCH performance requirements	1913

A.7	Sidelink reference resource pool configurations	1914

A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests	1914

A.7.1.1	FDD	1914

A.7.1.2	TDD	1917

A.7.2	Reference resource pool configurations for ProSe Direct Communication demodulation tests	1919

A.7.2.1	FDD	1919

A.8	V2X reference measurement channels	1924

A.8.1	General	1924

A.8.1.1	Overview of V2X reference measurement channels	1925

A.8.2	Reference measurement channel for receiver characteristics	1925

A.8.3	Reference measurement channel for transmitter characteristics	1927

A.8.4	Reference measurement for PSCCH performance requirements	1930

A.8.5	Reference measurement for PSSCH performance requirements	1931

A.8.6	Reference measurement for PSBCH performance requirements	1931

A.9	V2X reference resource pool configurations	1932

Annex B (normative):  Propagation conditions	1935

B.1	Static propagation condition	1935

B.1.1	UE Receiver with 2Rx	1935

B.1.2	UE Receiver with 4Rx	1935

B.1.3	UE Receiver with 8Rx	1936

B.2	Multi-path fading propagation conditions	1937

B.2.1	Delay profiles	1937

B.2.2	Combinations of channel model parameters	1938

B.2.3	MIMO Channel Correlation Matrices	1939

B.2.3.1	Definition of MIMO Correlation Matrices	1939

B.2.3.2	MIMO Correlation Matrices at High, Medium and Low Level	1945

B.2.3A	MIMO Channel Correlation Matrices using cross polarized antennas	1948

B.2.3A.1	Definition of MIMO Correlation Matrices using cross polarized antennas	1949

B.2.3A.2	Spatial Correlation Matrices using cross polarized antennas at eNB and UE sides	1949

B.2.3A.2.1	Spatial Correlation Matrices at eNB side	1949

B.2.3A.2.2	Spatial Correlation Matrices at UE side	1950

B.2.3A.4	Beam steering approach	1953

B.2.3B	MIMO Channel Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1953

B.2.3B.1	Definition of MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1954

B.2.3B.2	Spatial Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1955

B.2.3B.2.1	Spatial Correlation Matrices at eNB side	1955

B.2.3B.2.2	Spatial Correlation Matrices at UE side	1955

B.2.3B.3	MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1955

B.2.3B.4	Beam steering approach	1958

B.2.3B.4A	Beam steering approach with dual cluster beams	1959

B.2.4	Propagation conditions for CQI tests	1960

B.2.4.1	Propagation conditions for CQI tests with multiple CSI processes	1960

B.2.5	Void	1960

B.2.6	MBSFN Propagation Channel Profile	1960

B.2.6.1	Subcarrier spacing 15kHz or 7.5kHz	1960

B.2.6.2	Subcarrier spacing 1.25kHz	1961

B.2.6.3	Subcarrier spacing 0.37kHz	1962

B.2.6.4	Subcarrier spacing 2.5kHz	1962

B.3	High speed train scenario	1963

B.3A	HST-SFN scenario	1964

B.3B	HST-SFN scenario for 500 km/h speed	1968

B.3C	HST scenario for 500 km/h speed	1968

B.4	Beamforming Model	1968

B.4.1	Single-layer random beamforming (Antenna port 5, 7, or 8)	1968

B.4.1A	Single-layer random beamforming (Antenna port 7, 8, 11 or 13 with enhanced DMRS table configured)	1969

B.4.2	Dual-layer random beamforming (antenna ports 7 and 8)	1969

B.4.3	Generic beamforming model (antenna ports 7-14)	1970

B.4.4	Random beamforming for EPDCCH distributed transmission (Antenna port 107 and 109)	1970

B.4.5	Random beamforming for EPDCCH localized transmission (Antenna port 107, 108, 109 or 110)	1971

B.4.6	Beamforming model for CRI test	1971

B.5	Interference models for enhanced performance requirements Type-A	1972

B.5.1	Dominant interferer proportion	1973

B.5.2	Transmission mode 3 interference model	1973

B.5.3	Transmission mode 4 interference model	1973

B.5.4	Transmission mode 9 interference model	1974

B.6	Interference models for enhanced performance requirements Type-B	1974

B.6.1	Transmission mode 2 interference model	1974

B.6.2	Transmission mode 3 interference model	1974

B.6.3	Transmission mode 4 interference model	1975

B.6.4	Transmission mode 9 interference model	1975

B.6.5	CRS interference model	1976

B.6.6	Random interference model	1976

B.7	Interference models for enhanced downlink control channel performance requirements Type A and B	1977

B.7.1	PDCCH, PCFICH and PHICH interference model	1977

B.8	Burst transmission models for Frame structure type 3	1978

B.8.1	Burst transmission model for one LAA SCell	1978

B.8.2	Burst transmission model for multiple LAA SCell(s)	1978

Annex C (normative):  Downlink Physical Channels	1980

C.1	General	1980

C.2	Set-up	1980

C.3	Connection	1980

C.3.1	Measurement of Receiver Characteristics	1980

C.3.2	Measurement of Performance requirements	1981

C.3.3	Aggressor cell power allocation for Measurement of Performance Requirements when ABS is Configured	1982

C.3.4	Power Allocation for Measurement of Performance Requirements when Quasi Co-location Type B: same Cell ID	1983

C.3.5	Simplified CA testing method	1983

C.3.6	Measurement of Receiver Characteristics for Narrowband IoT	1984

Annex D (normative):  Characteristics of the interfering signal	1985

D.1	General	1985

D.2	Interference signals	1985

Annex E (normative):  Environmental conditions	1986

E.1	 General	1986

E.2	 Environmental	1986

E.2.1	Temperature	1986

E.2.2	Voltage	1986

E.2.3	Vibration	1987

Annex F (normative):  Transmit modulation	1988

F.1	Measurement Point	1988

F.2	Basic Error Vector Magnitude measurement	1988

F.3	Basic in-band emissions measurement	1989

F.4	Modified signal under test	1989

F.5	Window length	1991

F.5.1	Timing offset	1991

F.5.2	Window length	1991

F.5.3	Window length for normal CP	1991

F.5.4	Window length for Extended CP	1992

F.5.5	Window length for PRACH	1992

F.5.F	Window length for category NB1	1993

F.6	Averaged EVM	1993

F.6.F	Averaged EVM for category NB1	1994

F.7	Spectrum Flatness	1994

Annex G (informative):  Reference sensitivity level in lower SNR	1995

G.1	General	1995

G.2	Typical receiver sensitivity performance (QPSK)	1995

G.3	Reference measurement channel for REFSENSE in lower SNR	2001

Annex H (normative):  Modified MPR behavior	2004

H.1	Indication of modified MPR behavior	2004

Annex I (normative):  Supported Post Antenna Gain	2005

I.1	Declared Supported Post Antenna Gain for UE	2005

Annex J (informative):  Change history	2006