3GPP TS 36.508 V18.7.0 (2024-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC);

Common test environments for User Equipment (UE) conformance testing

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

mobile, UE, terminal, testing, LTE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	29

Introduction	29

1	Scope	30

2	References	30

3	Definitions, symbols and abbreviations	33

3.1	Definitions	33

3.2	Symbols	33

3.3	Abbreviations	34

4	Common test environment	34

4.1	Environmental conditions	34

4.1.1	Temperature	34

4.1.2	Voltage	35

4.2	Common requirements of test equipment	35

4.2.1	General functional requirements	35

4.2.2	Minimum functional requirements	36

4.2.2.1	Supported Cell Configuration	36

4.2.2.1.1	Supported Channels	36

4.2.2.2	Support of Tcell timing offset	37

4.2.2.3	Supported Sidelink Configuration	37

4.2.2.3.1	Supported Sidelink Channels	38

4.3	Reference test conditions	38

4.3.1	Test frequencies	38

4.3.1.1	FDD Mode Test frequencies	43

4.3.1.1.1	FDD reference test frequencies for operating band 1	43

4.3.1.1.1A	FDD reference test frequencies for CA in operating band 1	44

4.3.1.1.2	FDD reference test frequencies for operating band 2	44

4.3.1.1.2A	FDD reference test frequencies for CA in operating band 2	45

4.3.1.1.3	FDD reference test frequencies for operating band 3	47

4.3.1.1.3A	FDD reference test frequencies for CA in operating band 3	47

4.3.1.1.4	FDD reference test frequencies for operating band 4	48

4.3.1.1.4A	FDD reference test frequencies for CA in operating band 4	49

4.3.1.1.5	FDD reference test frequencies for operating band 5	50

4.3.1.1.5A	FDD reference test frequencies for CA in operating band 5	50

4.3.1.1.6	FDD reference test frequencies for operating band 6	51

4.3.1.1.7	FDD reference test frequencies for operating band 7	51

4.3.1.1.7A	FDD reference test frequencies for CA in operating band 7	52

4.3.1.1.8	FDD reference test frequencies for operating band 8	53

4.3.1.1.8A	FDD reference test frequencies for CA in operating band 8	54

4.3.1.1.9	FDD reference test frequencies for operating band 9	54

4.3.1.1.10	FDD reference test frequencies for operating band 10	54

4.3.1.1.11	FDD reference test frequencies for operating band 11	55

4.3.1.1.12	FDD reference test frequencies for operating band 12	55

4.3.1.1.12A	FDD reference test frequencies for CA in operating band 12	55

4.3.1.1.13	FDD reference test frequencies for operating band 13	56

4.3.1.1.14	FDD reference test frequencies for operating band 14	56

4.3.1.1.15	FDD reference test frequencies for operating band 15	56

4.3.1.1.16	FDD reference test frequencies for operating band 16	56

4.3.1.1.17	FDD reference test frequencies for operating band 17	56

4.3.1.1.18	FDD reference test frequencies for operating band 18	57

4.3.1.1.19	FDD reference test frequencies for operating band 19	57

4.3.1.1.20	FDD reference test frequencies for operating band 20	57

4.3.1.1.21	FDD reference test frequencies for operating band 21	58

4.3.1.1.22	FDD reference test frequencies for operating band 22	58

4.3.1.1.23	FDD reference test frequencies for operating band 23	58

4.3.1.1.23A	FDD reference test frequencies for CA in operating band 23	59

4.3.1.1.24	FDD reference test frequencies for operating band 24	59

4.3.1.1.25	FDD reference test frequencies for operating band 25	59

4.3.1.1.25A	FDD reference test frequencies for CA in operating band 25	60

4.3.1.1.26	FDD reference test frequencies for operating band 26	60

4.3.1.1.27	FDD reference test frequencies for operating band 27	61

4.3.1.1.27A	FDD reference test frequencies for CA in operating band 27	61

4.3.1.1.28	FDD reference test frequencies for operating band 28	62

4.3.1.1.29	FDD reference test frequencies for CA in operating band 29	63

4.3.1.1.31	FDD reference test frequencies for operating band 31	63

4.3.1.1.32	FDD reference test frequencies for CA in operating band 32	63

4.3.1.1.33 to 4.3.1.1.64	Void	64

4.3.1.1.65	FDD reference test frequencies for operating band 65	64

4.3.1.1.66	FDD reference test frequencies for operating band 66	65

4.3.1.1.66A	FDD reference test frequencies for CA in operating band 66	66

4.3.1.1.67	FDD reference test frequencies for CA in operating band 67	86

4.3.1.1.68	FDD reference test frequencies for operating band 68	87

4.3.1.1.69	FDD reference test frequencies for operating band 69	87

4.3.1.1.70	FDD reference test frequencies for operating band 70	87

4.3.1.1.70A	FDD reference test frequencies for CA in operating band 70	88

4.3.1.1.71	FDD reference test frequencies for operating band 71	88

4.3.1.1.72	FDD reference test frequencies for operating band 72	88

4.3.1.1.73	FDD reference test frequencies for operating band 73	89

4.3.1.1.74	FDD reference test frequencies for operating band 74	89

4.3.1.1.75 to 4.3.1.1.84	FFS	89

4.3.1.1.85	FDD reference test frequencies for operating band 85	89

4.3.1.1.86	FFS	90

4.3.1.1.87	FDD reference test frequencies for operating band 87	90

4.3.1.1.88	FDD reference test frequencies for operating band 88	90

4.3.1.1.89 to 4.3.1.1.105	FFS	90

4.3.1.1.106	FDD reference test frequencies for operating band 106	90

4.3.1.1.107 to 4.3.1.1.252	FFS	90

4.3.1.1.253	FDD reference test frequencies for operating band 253	91

4.3.1.1.254	FDD reference test frequencies for operating band 254	91

4.3.1.1.255	FDD reference test frequencies for operating band 255	91

4.3.1.1.256	FDD reference test frequencies for operating band 256	91

4.3.1.2	TDD Mode Test frequencies	92

4.3.1.2.1	TDD reference test frequencies for Operating Band 33	92

4.3.1.2.2	TDD reference test frequencies for Operating Band 34	92

4.3.1.2.3	TDD reference test frequencies for Operating Band 35	92

4.3.1.2.4	TDD reference test frequencies for Operating Band 36	93

4.3.1.2.5	TDD reference test frequencies for Operating Band 37	93

4.3.1.2.6	TDD reference test frequencies for Operating Band 38	93

4.3.1.2.6A	TDD reference test frequencies for CA in operating band 38	94

4.3.1.2.7	TDD reference test frequencies for Operating Band 39	94

4.3.1.2.7A	TDD reference test frequencies for CA in Operating Band 39	95

4.3.1.2.8	TDD reference test frequencies for Operating Band 40	95

4.3.1.2.8A	TDD reference test frequencies for CA in operating band 40	96

4.3.1.2.9	TDD reference test frequencies for Operating Band 41	102

4.3.1.2.9A	TDD reference test frequencies for CA in operating band 41	103

4.3.1.2.10	TDD reference test frequencies for Operating Band 42	118

4.3.1.2.10A	TDD reference test frequencies for CA in operating band 42	118

4.3.1.2.11	TDD reference test frequencies for Operating Band 43	128

4.3.1.2.12	TDD reference test frequencies for Operating Band 44	128

4.3.1.2.13	TDD reference test frequencies for Operating Band 45	128

4.3.1.2.14	TDD reference test frequencies for Operating Band 46	129

4.3.1.2.14A	TDD reference test frequencies for CA in operating Band 46	129

4.3.1.2.15	TDD reference test frequencies for Operating Band 47	131

4.3.1.2.16	TDD reference test frequencies for Operating Band 48	132

4.3.1.2.16A	TDD reference test frequencies for CA in operating band 48	133

4.3.1.2.17	TDD reference test frequencies for Operating Band 53	134

4.3.1.2.18	TDD reference test frequencies for Operating Band 54	134

4.3.1.3	HRPD Test frequencies	135

4.3.1.3.1	HRPD test frequencies for Band Class 0	135

4.3.1.3.2	HRPD test frequencies for Band Class 1	135

4.3.1.3.3	HRPD test frequencies for Band Class 3	135

4.3.1.3.4	HRPD test frequencies for Band Class 4	135

4.3.1.3.5	HRPD test frequencies for Band Class 6	135

4.3.1.3.6	HRPD test frequencies for Band Class 10	136

4.3.1.3.7	HRPD test frequencies for Band Class 15	136

4.3.1.4	1xRTT Test frequencies	136

4.3.1.4.1	1xRTT test frequencies for Band Class 0	136

4.3.1.4.2	1xRTT test frequencies for Band Class 1	136

4.3.1.4.3	1xRTT test frequencies for Band Class 3	136

4.3.1.4.4	1xRTT test frequencies for Band Class 4	137

4.3.1.4.5	1xRTT test frequencies for Band Class 6	137

4.3.1.4.6	1xRTT test frequencies for Band Class 10	137

4.3.1.4.7	1xRTT test frequencies for Band Class 15	137

4.3.1.5	MFBI Test frequencies	137

4.3.1.5.1	MFBI Test frequencies for operation band 2 overlapping with band 25	137

4.3.1.5.2	MFBI Test frequencies for operation band 3 overlapping with band 9	138

4.3.1.5.3	MFBI Test frequencies for operation band 4 overlapping with band 10	138

4.3.1.5.4	MFBI Test frequencies for operation band 5 overlapping with band 18	138

4.3.1.5.5	MFBI Test frequencies for operation band 5 overlapping with band 19	139

4.3.1.5.6	MFBI Test frequencies for operation band 5 overlapping with band 26	139

4.3.1.5.7	MFBI Test frequencies for operation band 9 overlapping with band 3	139

4.3.1.5.8	MFBI Test frequencies for operation band 10 overlapping with band 4	139

4.3.1.5.9	MFBI Test frequencies for operation band 12 overlapping with band 17	139

4.3.1.5.10	MFBI Test frequencies for operation band 17 overlapping with band 12	140

4.3.1.5.11	MFBI Test frequencies for operation band 18 overlapping with band 5	140

4.3.1.5.12	MFBI Test frequencies for operation band 18 overlapping with band 26	140

4.3.1.5.13	MFBI Test frequencies for operation band 18 overlapping with band 27	140

4.3.1.5.14	MFBI Test frequencies for operation band 19 overlapping with band 5	140

4.3.1.5.15	MFBI Test frequencies for operation band 19 overlapping with band 26	140

4.3.1.5.16	MFBI Test frequencies for operation band 25 overlapping with band 2	141

4.3.1.5.17	MFBI Test frequencies for operation band 26 overlapping with band 5	141

4.3.1.5.18	MFBI Test frequencies for operation band 26 overlapping with band 18	141

4.3.1.5.19	MFBI Test frequencies for operation band 26 overlapping with band 19	142

4.3.1.5.20	MFBI Test frequencies for operation band 26 overlapping with band 27	142

4.3.1.5.21	MFBI Test frequencies for operation band 27 overlapping with band 18	143

4.3.1.5.22	MFBI Test frequencies for operation band 27 overlapping with band 26	143

4.3.1.5.23	MFBI Test frequencies for operation band 33 overlapping with band 39	143

4.3.1.5.24	MFBI Test frequencies for operation band 38 overlapping with band 41	143

4.3.1.5.25	MFBI Test frequencies for operation band 39 overlapping with band 33	143

4.3.1.5.26	MFBI Test frequencies for operation band 41 overlapping with band 38	144

4.3.1.5.27	MFBI Test frequencies for operation band 66 overlapping with band 4	144

4.3.1.5.28	MFBI Test frequencies for operation band 66 overlapping with band 10	144

4.3.1.6	WLAN Test frequencies	145

4.3.1.6.1	WLAN Test frequencies for 2.4 GHz ISM Band	145

4.3.1.6.2	WLAN Test frequencies for 5 GHz ISM Band	145

4.3.1.7	Bluetooth Test frequencies	145

4.3.1.7.1	Bluetooth Test frequencies for 2.4 GHz ISM Band	145

4.3.2	Radio conditions	145

4.3.2.1	Normal propagation condition	145

4.3.3	Physical channel allocations	145

4.3.3.1	Antennas	145

4.3.3.2	Downlink physical channels and physical signals	146

4.3.3.3	Mapping of downlink physical channels and signals to physical resources	146

4.3.3.4	Uplink physical channels and physical signals	153

4.3.3.5	Mapping of uplink physical channels and signals to physical resources	153

4.3.4	Signal levels	153

4.3.4.1	Downlink signal levels	153

4.3.4.2	Uplink signal levels	154

4.3.5	Standard test signals	154

4.3.5.1	Downlink test signals	154

4.3.5.2	Uplink test signals	154

4.3.6	Physical layer parameters	154

4.3.6.1	Downlink physical layer parameters	154

4.3.6.1.1	Physical layer parameters for DCI format 0	154

4.3.6.1.1A	Physical layer parameters for DCI format 0C	155

4.3.6.1.1B	Physical layer parameters for DCI format 0A	156

4.3.6.1.1C	Physical layer parameters for DCI format 0B	156

4.3.6.1.2	Physical layer parameters for DCI format 1	157

4.3.6.1.3	Physical layer parameters for DCI format 1A	157

4.3.6.1.3A	Physical layer parameters for DCI format 1B	158

4.3.6.1.4	Physical layer parameters for DCI format 1C	158

4.3.6.1.5	Physical layer parameters for DCI format 2	159

4.3.6.1.6	Physical layer parameters for DCI format 2A	160

4.3.6.1.6A	Physical layer parameters for DCI format 3B	160

4.3.6.1.6B	Physical layer parameters for DCI format 4A	161

4.3.6.1.6C	Physical layer parameters for DCI format 4B	161

4.3.6.1.7	Physical layer parameters for DCI format 5	162

4.3.6.1.7A	Physical layer parameters for DCI format 5A	162

4.3.6.1.8	Physical layer parameters for DCI format 6-0A	162

4.3.6.1.9	Physical layer parameters for DCI format 6-0B	164

4.3.6.1.10	Physical layer parameters for DCI format 6-1A	165

4.3.6.1.11	Physical layer parameters for DCI format 6-1B	167

4.3.6.1.12	Physical layer parameters for DCI format 6-2	168

4.3.6.1.13	Physical layer parameters for DCI format 7-0A	169

4.3.6.1.14	Physical layer parameters for DCI format 7-0B	170

4.3.6.1.15	Physical layer parameters for DCI format 7-1A	171

4.3.6.1.16	Physical layer parameters for DCI format 7-1B	172

4.3.6.1.17	Physical layer parameters for DCI format 7-1C	173

4.3.6.1.18	Physical layer parameters for DCI format 7-1D	174

4.3.6.1.19	Physical layer parameters for DCI format 7-1E	175

4.3.6.1.20	Physical layer parameters for DCI format 7-1F	176

4.3.6.1.21	Physical layer parameters for DCI format 7-1G	177

4.4	Reference system configurations	177

4.4.1	Simulated network scenarios	177

4.4.1.1	Single cell network scenarios	177

4.4.1.2	E-UTRA single mode multi cell network scenarios	178

4.4.1.3	E-UTRA dual mode multi cell network scenarios	178

4.4.1.4	3GPP Inter-RAT network scenarios	178

4.4.1.5	3GPP2 Inter-RAT network scenarios	178

4.4.1.6	WLAN Inter-RAT network scenarios	178

4.4.1.7	Bluetooth Inter-RAT network scenarios	178

4.4.2	Simulated cells	178

4.4.3	Common parameters for simulated E-UTRA cells	186

4.4.3.1	Common configurations of system information blocks	186

4.4.3.1.1	Combinations of system information blocks	186

4.4.3.1.2	Scheduling of system information blocks	191

4.4.3.2	Common contents of system information messages	196

-	MasterInformationBlock	196

-	SystemInformation	197

-	SystemInformation-BR-r13	198

-	SystemInformationBlockType1	199

-	SystemInformationBlockType1-BR-r13	203

4.4.3.3	Common contents of system information blocks	208

-	SystemInformationBlockType2	208

-	SystemInformationBlockType3	211

-	SystemInformationBlockType4	212

-	SystemInformationBlockType5	212

-	SystemInformationBlockType6	216

-	SystemInformationBlockType7	218

-	SystemInformationBlockType8	219

-	SystemInformationBlockType9	222

-	SystemInformationBlockType10	222

-	SystemInformationBlockType11	224

-	SystemInformationBlockType12	226

-	SystemInformationBlockType13	227

-	SystemInformationBlockType14	227

-	SystemInformationBlockType15	228

-	SystemInformationBlockType17	229

-	SystemInformationBlockType18	229

-	SystemInformationBlockType19	236

-	SystemInformationBlockType20	241

-	SystemInformationBlockType21	241

-	SystemInformationBlockType24	242

-	SystemInformationBlockType31	245

-	SystemInformationBlockType32	245

4.4.3.4	Channel-bandwidth-dependent parameters in system information blocks	246

4.4.4	Common parameters for simulated UTRA cells	246

4.4.4.1	Common contents of system information blocks for UTRA cells	247

-	System Information Block type 19	247

4.4.4.2	UTRA SIB scheduling for inter EUTRA - UTRA test	251

4.4.4.3	UTRA SIB scheduling for inter EUTRA – UTRA - GERAN test	251

4.4.5	Common parameters for simulated GERAN cells	252

4.4.6	Common parameters for simulated CDMA2000 cells	255

4.4.7	Default parameters specific for simulated cells	256

4.4.7.1	Common contents of HRPD Overhead messages	256

4.4.7.2	Common contents of 1XRTT Overhead messages	261

4.4.7.2.1	Configuration sequence number	261

4.4.7.2.2	Over Head messages	262

4.4.8	Common parameters for simulated WLAN AP’s	271

4.4.9	Common parameters for simulated Bluetooth beacon’s	272

4.5	Generic procedures	273

4.5.1	UE test states	273

4.5.2	UE Registration (State 2)	278

4.5.2A	UE Registration, UE Test Mode Activated (State 2A)	287

4.5.2A.1	Initial conditions	287

4.5.2AA	UE Registration in cell supporting BL/CE UE (State 2-CE)	291

4.5.2AA.1	Initial conditions	291

4.5.2AA.2	Definition of system information messages	291

4.5.2AA.3	Procedure	292

4.5.2AA.4	Specific message contents	292

4.5.2AB	UE Registration, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-CE)	292

4.5.2AB.1	Initial conditions	292

4.5.2AB.2	Definition of system information messages	292

4.5.2AB.3	Procedure	293

4.5.2AB.4	Specific message contents	293

4.5.2B	UE Registration, pre-registration on HRPD (State 2B)	293

4.5.2B.1	Initial conditions	293

4.5.2B.2	Definition of system information messages	293

4.5.2B.3	Procedure	294

4.5.2B.4	Specific message contents	299

4.5.2C	UE Registration, pre-registration on 1xRTT (State 2C)	300

4.5.2C.1	Initial conditions	300

4.5.2C.2	Definition of system information messages	300

4.5.2C.3	Procedure	302

4.5.2C.4	Specific message contents	302

4.5.2D	UE Registration, 2 PDN for RAN Assisted WLAN Interworking (State 2)	308

4.5.2D.1	Initial conditions	308

4.5.2D.2	Definition of system information messages	308

4.5.2D.3	Procedure	309

4.5.2D.4	Specific message contents	309

4.5.2E	MUSIM UE Registration	311

4.5.2E.1	Initial conditions	311

4.5.2E.2	Definition of system information messages	312

4.5.2E.3	Procedure	312

4.5.2E.4	Specific message contents	312

4.5.3	Generic Radio Bearer Establishment (State 3)	313

4.5.3.1	Initial conditions	313

4.5.3.2	Definition of system information messages	313

4.5.3.3	Procedure	314

4.5.3.4	Specific message contents	316

4.5.3A	Generic Radio Bearer Establishment, UE Test Mode Activated (State 3A)	316

4.5.3A.1	Initial conditions	316

4.5.3A.2	Definition of system information messages	316

4.5.3A.3	Procedure	317

4.5.3A.4	Specific message contents	317

4.5.3AA	Generic Radio Bearer Establishment (State 3-CE)	317

4.5.3AA.1	Initial conditions	317

4.5.3AA.2	Definition of system information messages	317

4.5.3AA.3	Procedure	317

4.5.3AA.4	Specific message contents	317

4.5.3AB	Generic Radio Bearer Establishment, UE Test Mode Activated (State 3A-CE)	317

4.5.3AB.1	Initial conditions	317

4.5.3AB.2	Definition of system information messages	318

4.5.3AB.3	Procedure	318

4.5.3AB.4	Specific message contents	318

4.5.3B	Generic Radio Bearer Establishment, pre-registered on HRPD (State 3B)	318

4.5.3B.1	Initial conditions	318

4.5.3B.2	Definition of system information messages	318

4.5.3B.3	Procedure	318

4.5.3B.4	Specific message contents	318

4.5.3C	Generic Radio Bearer Establishment, pre-registered on 1xRTT (State 3C)	318

4.5.3C.1	Initial conditions	318

4.5.3C.2	Definition of system information messages	319

4.5.3C.3	Procedure	319

4.5.3C.4	Specific message contents	319

4.5.3D	Generic Radio Bearer Establishment for RAN Assisted WLAN Interworking (State 3)	319

4.5.3D.1	Initial conditions	319

4.5.3D.2	Definition of system information messages	319

4.5.3D.3	Procedure	320

4.5.3D.4	Specific message contents	320

4.5.3E	Control plane CIoT connection request (State 3-CP)	320

4.5.3E.1	Initial conditions	320

4.5.3E.2	Definition of system information messages	320

4.5.3E.3	Procedure	321

4.5.3E.4	Specific message contents	321

4.5.3EA	Control plane CIoT connection request, UE Test Mode Activated (State 3A-CP)	321

4.5.3EA.1	Initial conditions	321

4.5.3EA.2	Definition of system information messages	321

4.5.3EA.3	Procedure	321

4.5.3EA.4	Specific message contents	321

4.5.3F	User plane CIoT connection request (State 3-UP)	322

4.5.3F.1	Initial conditions	322

4.5.3F.2	Definition of system information messages	322

4.5.3F.3	Procedure	322

4.5.3F.4	Specific message contents	322

4.5.3FA	User plane CIoT connection request, UE Test Mode Activated (State 3A-UP)	322

4.5.3FA.1	Initial conditions	322

4.5.3FA.2	Definition of system information messages	323

4.5.3FA.3	Procedure	323

4.5.3FA.4	Specific message contents	323

4.5.4	Loopback Activation (State 4)	323

4.5.4.1	Initial conditions	323

4.5.4.2	Definition of system information messages	323

4.5.4.3	Procedure	323

4.5.4.4	Specific message contents	323

4.5.4A	Loopback Activation in cell supporting BL/CE UE (State 4-CE)	323

4.5.4A.1	Initial conditions	323

4.5.4A.2	Definition of system information messages	324

4.5.4A.3	Procedure	324

4.5.4A.4	Specific message contents	324

4.5.4B	Loopback Activation user plane (State 4A-UP)	324

4.5.4B.1	Initial conditions	324

4.5.4B.2	Definition of system information messages	324

4.5.4B.3	Procedure	324

4.5.4B.4	Specific message contents	324

4.5.5	HRPD registration (State H2)	325

4.5.5.1	Initial conditions	325

4.5.5.2	Definition of system information messages	325

4.5.5.3	Procedure	325

4.5.5.4	Specific message contents	325

4.5.5A	HRPD registration, pre-registration on E-UTRAN (State H2A)	325

4.5.5A.1	Initial conditions	325

4.5.5A.2	Definition of system information messages	325

4.5.5A.3	Procedure	325

4.5.5A.4	Specific message contents	325

4.5.6	HRPD session establishment (State H3)	326

4.5.6.1	Initial conditions	326

4.5.6.2	Definition of system information messages	326

4.5.6.3	Procedure	326

4.5.6.4	Specific message contents	326

4.5.6A	HRPD session establishment, pre-registered on E-UTRAN (State H3A)	326

4.5.6A.1	Initial conditions	326

4.5.6A.2	Definition of system information messages	326

4.5.6A.3	Procedure	326

4.5.6A.4	Specific message contents	326

4.5.7	Out of Coverage (State 5)	327

4.5.7.1	Initial conditions	327

4.5.7.2	Definition of system information messages	327

4.5.7.3	Procedure	327

4.5.8	Out of Coverage, V2X setup (State 5-V2X)	327

4.5.8.1	Initial conditions	327

4.5.8.2	Definition of system information messages	327

4.5.8.3	Procedure	327

4.5.9	Out of Coverage, Test Loopback Activation, V2X setup (State 5A-V2X)	327

4.5.9.1	Initial conditions	327

4.5.9.2	Definition of system information messages	328

4.5.9.3	Procedure	328

4.5.9.4	Specific message contents	328

4.5A	Other generic procedures	329

4.5A.1	Procedure for IP address allocation in the U-plane	329

4.5A.2	Tracking area updating procedure	329

4.5A.3	Procedure for IMS signalling	330

4.5A.3.1	Specific message contents	331

4.5A.3A	Procedure for IMS Signalling over UTRA	331

4.5A.3A.1	Initial conditions	331

4.5A.3A.2	Procedure	332

4.5A.3A.3	Specific message contents	333

4.5A.3B	Procedure for preventing IMS Signalling over GERAN	335

4.5A.3B.1	Initial conditions	335

4.5A.3B.2	Procedure	336

4.5A.3B.3	Specific message contents	336

4.5A.4	Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service	337

4.5A.4.1	Initial conditions	337

4.5A.4.2	Definition of system information messages	337

4.5A.4.3	Procedure	337

4.5A.4.4	Specific message contents	340

4.5A.5	Generic Test Procedure for IMS Emergency call establishment in EUTRA: Limited Service	341

4.5A.5.1	Initial conditions	341

4.5A.5.2	Definition of system information messages	341

4.5A.5.3	Procedure	342

4.5A.5.4	Specific message contents	345

4.5A.6	Generic Test Procedure for IMS MO speech call establishment in E-UTRA	346

4.5A.6.1	Initial conditions	346

4.5A.6.2	Definition of system information messages	346

4.5A.6.3	Procedure	347

4.5A.6.4	Specific message contents	348

4.5A.7	Generic Test Procedure for IMS MT Speech call establishment in E-UTRA	348

4.5A.7.1	Initial conditions	348

4.5A.7.2	Definition of system information messages	348

4.5A.7.3	Procedure	349

4.5A.7.4	Specific message contents	351

4.5A.8	Generic Test Procedure for IMS MO video call establishment in E-UTRA	351

4.5A.8.1	Initial conditions	351

4.5A.8.2	Definition of system information messages	351

4.5A.8.3	Procedure	352

4.5A.8.4	Specific message contents	353

4.5A.9	Generic Test Procedure for IMS MT video call establishment in E-UTRA	353

4.5A.9.1	Initial conditions	353

4.5A.9.2	Definition of system information messages	353

4.5A.9.3	Procedure	354

4.5A.9.4	Specific message contents	354

4.5A.10	Generic Test Procedure for IMS MO speech and aSRVCC in E-UTRA	355

4.5A.10.1	Initial conditions	355

4.5A.10.2	Definition of system information messages	355

4.5A.10.3	Procedure	356

4.5A.10.4	Specific message contents	357

4.5A.11	Generic Test Procedure for IMS MO add video establishment in E-UTRA	357

4.5A.11.1	Initial conditions	357

4.5A.11.2	Definition of system information messages	357

4.5A.11.3	Procedure	357

4.5A.11.4	Specific message contents	358

4.5A.12	Generic Test Procedure for IMS MT add video establishment in E-UTRA	358

4.5A.12.1	Initial conditions	358

4.5A.12.2	Definition of system information messages	358

4.5A.12.3	Procedure	358

4.5A.12.4	Specific message contents	359

4.5A.14	Generic Test Procedure for IMS XCAP establishment in EUTRA	359

4.5A.14.1	Initial conditions	359

4.5A.14.2	Definition of system information messages	359

4.5A.14.3	Procedure	360

4.5A.14.4	Specific message contents	360

4.5A.15	Generic Test Procedure for EPS Bearer Deactivation	360

4.5A.15.1	Initial conditions	360

4.5A.15.2	Definition of system information messages	360

4.5A.15.3	Procedure	361

4.5A.15.4	Specific message contents	361

4.5A.15A	Generic Test Procedure for User or Network Initiated EPS Bearer Deactivation	362

4.5A.15A.1	Initial conditions	362

4.5A.15A.2	Definition of system information messages	362

4.5A.15A.3	Procedure	362

4.5A.15A.4	Specific message contents	363

4.5A.16	Generic Test Procedure to establish additional PDN connectivity	363

4.5A.16.1	Initial conditions	363

4.5A.16.2	Definition of system information messages	363

4.5A.16.3	Procedure	364

4.5A.16.4	Specific message contents	364

4.5A.17	Generic Test Procedure for user initiated release of additional PDN connectivity	365

4.5A.17.1	Initial conditions	365

4.5A.17.2	Definition of system information messages	365

4.5A.17.3	Procedure	366

4.5A.17.4	Specific message contents	366

4.5A.18	Generic Test Procedure for network initiated release of additional PDN connectivity	367

4.5A.18.1	Initial conditions	367

4.5A.18.2	Definition of system information messages	368

4.5A.18.3	Procedure	368

4.5A.18.4	Specific message contents	368

4.5A.19	Generic Test Procedure for IMS MO speech call establishment in E-UTRA / EVS	369

4.5A.19.1	Initial conditions	369

4.5A.19.2	Definition of system information messages	369

4.5A.19.3	Procedure	369

4.5A.19.4	Specific message contents	370

4.5A.20	Generic Test Procedure for IMS MT speech call establishment in E-UTRA / EVS	370

4.5A.20.1	Initial conditions	370

4.5A.20.2	Definition of system information messages	370

4.5A.20.3	Procedure	370

4.5A.20.4	Specific message contents	370

4.5A.21	Generic Test Procedure for IMS MO Customized Alerting Tones and speech establishment in E-UTRA	371

4.5A.21.1	Initial conditions	371

4.5A.21.2	Definition of system information messages	371

4.5A.21.3	Procedure	371

4.5A.21.4	Specific message contents	371

4.5A.22	Communication with the ProSe Function: Initial Access	371

4.5A.22.1	Initial conditions	371

4.5A.22.2	Definition of system information messages	371

4.5A.22.3	Procedure	372

4.5A.22.4	Specific message contents	374

4.5A.22A	Communication with the ProSe Function: Subsequent Access	374

4.5A.22A.1	Initial conditions	374

4.5A.22A.2	Definition of system information messages	374

4.5A.22A.3	Procedure	374

4.5A.22A.4	Specific message contents	375

4.5A.23	Generic Test Procedure for IMS registration in EPC / WLAN	375

4.5A.23.1	Initial conditions	375

4.5A.23.2	Definition of system information messages	375

4.5A.23.3	Procedure	376

4.5A.23.4	Specific message contents	376

4.5A.23A	Generic Test Procedure for IPsec Tunnel Disconnection in EPC / WLAN	376

4.5A.23A.1	Initial conditions	376

4.5A.23A.2	Definition of system information messages	376

4.5A.23A.3	Procedure	376

4.5A.23A.4	Specific message contents	376

4.5A.24	Generic Test Procedure for IMS emergency call establishment in EPC / WLAN	377

4.5A.24.1	Initial conditions	377

4.5A.24.2	Definition of system information messages	377

4.5A.24.3	Procedure	377

4.5A.24.4	Specific message contents	377

4.5A.25	Generic Test Procedure for XCAP establishment in EPC / WLAN	377

4.5A.25.1	Initial conditions	377

4.5A.25.2	Definition of system information messages	377

4.5A.25.3	Procedure	378

4.5A.25.4	Specific message contents	378

4.5A.26	Generic Test Procedure for eCall over IMS establishment in EUTRA: Normal Service	378

4.5A.26.1	Initial conditions	378

4.5A.26.2	Definition of system information messages	378

4.5A.26.3	Procedure	378

4.5A.26.4	Specific message contents	380

4.5A.27	Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support	381

4.5A.27.1	Initial conditions	381

4.5A.27.2	Definition of system information messages	381

4.5A.27.3	Procedure	381

4.5A.27.4	Specific message contents	384

4.5A.28	Generic Test Procedure for Converged IP Communications establishment in EUTRA	385

4.5A.28.1	Initial conditions	385

4.5A.28.2	Definition of system information messages	385

4.5A.28.3	Procedure	385

4.5A.28.4	Specific message contents	385

4.5A.29	Generic Test Procedure for Converged IP Communications establishment in EPC / WLAN	386

4.5A.29.1	Initial conditions	386

4.5A.29.2	Definition of system information messages	387

4.5A.29.3	Procedure	387

4.5A.29.4	Specific message contents	387

4.5A.30	Generic Test Procedure for IMS Re-registration in E-UTRAN	387

4.5A.30.1	Initial conditions	387

4.5A.30.2	Definition of system information messages	387

4.5A.30.3	Procedure	387

4.5A.30.4	Specific message contents	388

4.5A.31	Generic Test Procedure for UUAA-SM in EPS	388

4.5A.31.1	Initial conditions	388

4.5A.31.2	Definition of system information messages	388

4.5A.31.3	Procedure	389

4.5A.31.4	Specific message contents	389

4.6	Default RRC message and information elements contents	392

4.6.1	Contents of RRC messages	392

–	CounterCheck	392

–	CounterCheckResponse	393

–	CSFBParametersRequestCDMA2000	393

–	CSFBParametersResponseCDMA2000	393

–	DLInformationTransfer	394

–	HandoverFromEUTRAPreparationRequest	394

-	LoggedMeasurementConfiguration	395

-	MasterInformationBlock-SL	396

-	MasterInformationBlock-SL-V2X	397

–	MBMSCountingRequest	397

–	MBMSCountingResponse	398

–	MBMSInterestIndication	398

–	MBSFNAreaConfiguration	399

–	MeasurementReport	399

–	MobilityFromEUTRACommand	400

–	Paging	400

–	RRCConnectionReconfiguration	401

–	RRCConnectionReconfiguration (SideLink)	410

–	RRCConnectionReconfiguration (V2X)	417

–	RRCConnectionReconfiguration-CPA	418

–	RRCConnectionReconfigurationComplete	419

–	RRCConnectionReestablishment	420

–	RRCConnectionReestablishmentComplete	420

–	RRCConnectionReestablishmentReject	420

–	RRCConnectionReestablishmentRequest	421

–	RRCConnectionReject	421

–	RRCConnectionRelease	422

–	RRCConnectionRequest	422

–	RRCConnectionResume	423

–	RRCConnectionResumeComplete	423

–	RRCConnectionResumeRequest	424

–	RRCConnectionSetup	424

–	RRCConnectionSetupComplete	425

–	SCGFailureInformationNR	426

–	SCPTMConfiguration	428

–	SCPTMConfiguration-BR	429

–	SecurityModeCommand	430

–	SecurityModeComplete	430

–	SecurityModeFailure	430

–	SidelinkUEInformation	431

–	SidelinkUEInformation (V2X)	432

–	UECapabilityEnquiry	433

–	ULDedicatedMessageSegment	434

–	UECapabilityInformation	435

–	UEInformationRequest	442

–	UEInformationResponse	443

-	ULHandoverPreparationTransfer	443

–	ULInformationTransfer	444

–	UEAssistanceInformation	444

–	ULInformationTransferMRDC	445

4.6.2	System information blocks	445

4.6.3	Radio resource control information elements	445

–	BCCH-Config-DEFAULT	445

–	CellSelectionInfoCE-r13-DEFAULT	446

–	CQI-ReportAperiodic-r10-DEFAULT	446

–	CQI-ReportConfig-DEFAULT	447

–	CQI-ReportConfig-r10-DEFAULT	447

–	CQI-ReportConfig-v1130-eIMTA	452

–	CQI-ReportConfig-v1250-DEFAULT	454

–	CQI-ReportConfigSCell-r10-DEFAULT	455

–	CQI-ReportPeriodic-r10-DEFAULT	455

–	CSI-RS-ConfigNZP-r11-DEFAULT	456

–	CSI-RS-ConfigZP-r11-DEFAULT	456

–	DMRS-Config-r11-DEFAULT	457

–	DRB-ToAddModList-RECONFIG	457

–	EPDCCH-Config-r11-DEFAULT	457

–	EPDCCH-Config-r11-eIMTA	460

–	FreqHoppingParameters-r13-DEFAULT	462

–	PCCH-Config-DEFAULT	463

–	PCCH-Config-v1310-DEFAULT	463

–	PHICH-Config-DEFAULT	463

–	PDSCH-ConfigCommon-DEFAULT	464

–	PDSCH-ConfigCommon-v1310-DEFAULT	464

–	PDSCH-ConfigDedicated-DEFAULT	464

–	PDSCH-ConfigDedicated-v1130-DEFAULT	465

–	PhysicalConfigDedicatedSCell-r10-DEFAULT	466

–	PhysicalConfigDedicatedSCell-r10-eIMTA	469

–	PRACH-Config-DEFAULT	470

-	PRACH-Config-v1310-DEFAULT	471

-	PRACH-ConfigSIB-DEFAULT	473

-	PRACH-ConfigSIB-v1310-DEFAULT	474

-	PRACH-ConfigSIB-v1530-DEFAULT	479

–	PUCCH-ConfigCommon-DEFAULT	482

-	PUCCH-ConfigCommon-v1310-DEFAULT	482

–	PUCCH-ConfigDedicated-DEFAULT	483

–	PUCCH-ConfigDedicated-v1020-DEFAULT	484

–	PUCCH-ConfigDedicated-v1130-DEFAULT	485

–	PUCCH-ConfigDedicated-v1250-DEFAULT	485

–	PUCCH-ConfigDedicated-r13-DEFAULT	486

–	PUSCH-ConfigCommon-DEFAULT	487

–	PUSCH-ConfigCommon-v1310DEFAULT	487

–	PUSCH-ConfigDedicated-r13DEFAULT	488

–	PUSCH-ConfigDedicated-v1130-DEFAULT	488

–	PUSCH-ConfigDedicated-v1250-DEFAULT	488

–	PUSCH-ConfigDedicated--DEFAULT	489

–	PUSCH-EnhancementsConfig-r14-DEFAULT	489

–	RACH-ConfigCommon-DEFAULT	490

–	Rach-ConfigDedicated-DEFAULT	493

–	RadioResourceConfigCommon-DEFAULT	494

–	RadioResourceConfigCommonSCell-r10-DEFAULT	497

–	RadioResourceConfigCommonSIB-DEFAULT	500

–	RadioResourceConfigDedicated-SRB1	502

–	RadioResourceConfigDedicated-SRB2-DRB(n,m)	503

–	RadioResourceConfigDedicated-DRB(n,m)	504

–	RadioResourceConfigDedicated-SRB4	504

–	RadioResourceConfigDedicated-HO-TO-EUTRA(n,m)	505

-	RadioResourceConfigDedicated-AM-DRB-ADD(bid)	506

-	RadioResourceConfigDedicated-UM-DRB-ADD(bid)	506

-	RadioResourceConfigDedicated- DRB-REL(bid)	507

–	RadioResourceConfigDedicated-HO	507

–	RadioResourceConfigDedicatedSCell-r10-DEFAULT	507

–	RadioResourceConfigDedicated-SCell_AddMod	508

–	RadioResourceConfigDedicated-V2X	508

-	RadioResourceConfigDedicated-EN-DC-Split	509

-	RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid)	510

–	RadioResourceConfigDedicated-EN-DC-SRB2-DRB	510

–	RLC-Config-DRB-AM-RECONFIG	511

–	RLC-Config-DRB-UM-RECONFIG	511

–	RLC-Config-SRB-AM-RECONFIG	511

–	SCellToAddMod-r10-DEFAULT	512

–	SCellToRelease-r10-DEFAULT	512

–	SCG-Configuration-r12-DEFAULT	513

–	SCG-Configuration-r12-NE-DC	517

–	SchedulingRequestConfig-DEFAULT	520

–	SchedulingRequestConfigSCell-r13-DEFAULT	520

–	SL-CommResourcePoolV2X-r14-DEFAULT	521

–	SL-CommTxPoolSensingConfig-r14-DEFAULT	522

–	SL-InterFreqInfoV2X-r14-DEFAULT	523

–	SlotOrSubslotPDSCH-Config-r15-DEFAULT	523

–	SlotOrSubslotPUSCH-Config-r15-DEFAULT	524

–	SPDCCH-Config-r15-DEFAULT	525

–	SPUCCH-Config-r15-DEFAULT	526

–	SL-PSSCH-TxConfig-r14-DEFAULT	527

–	SL-TxPoolToAddMod-r14-DEFAULT	527

–	SL-TxPoolToReleaseListV2X-r14-DEFAULT	528

–	SL-V2X-ConfigDedicated-r14-DEFAULT	529

–	SL-V2X-InterFreqUE-Config-r14-DEFAULT	531

–	SL-V2X-PreconfigCommPool-r14-DEFAULT	532

–	SL-V2X-PreconfigFreqInfo-r14-DEFAULT	533

–	SoundingRS-UL-ConfigCommon-DEFAULT	534

–	SoundingRS-UL-ConfigDedicated-DEFAULT	534

–	SoundingRS-UL-ConfigDedicatedAperiodic-r10-DEFAULT	535

–	SRB-ToAddModList-RECONFIG	535

–	SRS-TPC-PDCCH-Config-r14-DEFAULT	536

–	TDD-Config-DEFAULT	536

–	TPC-PDCCH-Config-DEFAULT	536

–	TPC-PDCCH-ConfigSCell-r13-DEFAULT	537

–	UplinkPowerControlCommon-DEFAULT	537

–	UplinkPowerControlCommonSCell-r10-DEFAULT	538

–	UplinkPowerControlCommon-v1020-DEFAULT	538

–	UplinkPowerControlCommon-v1530-DEFAULT	538

–	UplinkPowerControlCommonSCell-v1310-DEFAULT	539

–	UplinkPowerControlDedicated-DEFAULT	539

–	UplinkPowerControlDedicated-v1020-DEFAULT	540

–	UplinkPowerControlDedicated-v1130-DEFAULT	540

–	UplinkPowerControlDedicated-v1250-DEFAULT	540

–	UplinkPowerControlDedicated-v1530-DEFAULT	541

–	UplinkPowerControlDedicatedSCell-r10-DEFAULT	541

–	UplinkPowerControlDedicatedSCell-v1310-DEFAULT	541

-	RadioResourceConfigDedicated-DRB-Mod	541

-	RadioResourceConfigDedicated-PCell-PATTERN	542

-	OtherConfig-r9	542

-	WLAN-OffloadConfig-r12	544

–	EIMTA-MainConfig-r12-DEFAULT	545

–	EIMTA-MainConfigServCell-r12-DEFAULT	545

–	LWIP-Configuration-GENERIC	546

–	LWA-Configuration-GENERIC	547

–	LWIP-Configuration-W2-GENERIC	548

–	LWA-Configuration-W2-GENERIC	549

4.6.4	Security control information elements	549

–	SecurityConfigHO-DEFAULT	549

–	SecurityConfigSMC-DEFAULT	550

4.6.5	Mobility control information elements	551

–	MobilityControlInfo-HO	551

–	ConditionalReconfiguration-r16-HO	553

–	ConditionalReconfiguration-r16-CPA	554

–	ConditionalReconfiguration-r16-InterSN_CPC	554

–	CondReconfigurationToAddModList-r16	554

–	ConditionalReconfigurationId	554

4.6.6	Measurement information elements	555

-	MeasConfig-DEFAULT	556

-	MeasGapConfig-GP1	557

-	MeasGapConfig-CE	557

-	MeasGapConfig-GP2	557

-	MeasDS-Config-DEFAULT	558

-	MeasCSI-RS-Config-DEFAULT	558

–	MeasObjectCDMA2000-GENERIC	559

–	ReportConfigToAddModList_DEFAULT	559

–	MeasIdToAddModList_DEFAULT	559

-	MeasObjectEUTRA-GENERIC	560

-	MeasObjectGERAN-GENERIC	561

-	MeasObjectNR-GENERIC	561

-	MeasObjectUTRA-GENERIC	562

–	MeasObjectWLAN-W2-GENERIC	562

-	QuantityConfig-DEFAULT	563

-	ReportConfigEUTRA-A1	565

-	ReportConfigEUTRA-A2	566

-	ReportConfigEUTRA-A3	567

-	ReportConfigEUTRA-A4	568

-	ReportConfigEUTRA-A5	569

-	ReportConfigEUTRA-A6	571

-	ReportConfigEUTRA-PERIODICAL	571

-	ReportConfigInterRAT-B1-GERAN	572

-	ReportConfig-B1-NR-r15	572

-	ReportConfigInterRAT-B1-UTRA	573

-	ReportConfigInterRAT-B1-NR	574

-	ReportConfigInterRAT-B2-CDMA2000	575

-	ReportConfigInterRAT-B2-GERAN	576

-	ReportConfigInterRAT-B2-UTRA	577

-	ReportConfigInterRAT-B2-NR	578

-	ReportConfigInterRAT-SFTD	579

-	ReportConfigInterRAT-PERIODICAL	579

–	ReportConfigInterRAT-W2-WLAN	580

–	ReportConfigInterRAT-W3-WLAN	580

-	ReportConfigEUTRA-C1	581

-	ReportConfigEUTRA-C2	582

-	ReportConfigEUTRA-PERIODICAL-CSI-RS	583

-	ReportConfigEUTRA-V1	583

-	ReportConfigEUTRA-V2	584

-	ReportConfigEUTRA-H1	584

-	ReportConfigEUTRA-H2	585

–	ReportConfigEUTRA-A3-CHO	585

–	ReportConfigEUTRA-A5-CHO	586

4.6.7	Other information elements	587

–	FlightPathInfoReportConfig	587

–	RRC-TransactionIdentifier-DL	587

–	RRC-TransactionIdentifier-UL	588

4.6.8	Channel-bandwidth-dependent parameters	588

4.7	Default NAS message and information element contents	588

4.7.1	Security protected NAS messages	589

4.7.2	Contents of EMM messages	590

-	ATTACH ACCEPT	590

-	ATTACH COMPLETE	594

-	ATTACH REJECT	594

-	ATTACH REQUEST	595

-	AUTHENTICATION FAILURE	597

-	AUTHENTICATION REJECT	598

-	AUTHENTICATION REQUEST	598

-	AUTHENTICATION RESPONSE	599

-	CS SERVICE NOTIFICATION	599

-	CONTROL PLANE SERVICE REQUEST	600

-	DETACH ACCEPT (UE originating detach)	600

-	DETACH ACCEPT (UE terminated detach)	601

-	DETACH REQUEST (UE originating detach)	601

-	DETACH REQUEST (UE terminated detach)	602

-	DOWNLINK NAS TRANSPORT	602

-	EMM INFORMATION	603

-	EMM STATUS	603

-	EXTENDED SERVICE REQUEST	604

-	GUTI REALLOCATION COMMAND	604

-	GUTI REALLOCATION COMPLETE	605

-	IDENTITY REQUEST	605

-	IDENTITY RESPONSE	605

-	SECURITY MODE COMMAND	606

-	SECURITY MODE COMPLETE	607

-	SECURITY MODE REJECT	607

-	SERVICE ACCEPT	608

-	SERVICE REJECT	608

-	SERVICE REQUEST	609

-	TRACKING AREA UPDATE ACCEPT	610

-	TRACKING AREA UPDATE COMPLETE	613

-	TRACKING AREA UPDATE REJECT	614

-	TRACKING AREA UPDATE REQUEST	615

-	UPLINK NAS TRANSPORT	618

4.7.3	Contents of ESM messages	618

-	ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT	618

-	ACTIVATE DEDICATED EPS BEARER CONTEXT REJECT	619

-	ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST	620

-	ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT	622

-	ACTIVATE DEFAULT EPS BEARER CONTEXT REJECT	623

-	ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST	624

-	BEARER RESOURCE ALLOCATION REJECT	630

-	BEARER RESOURCE ALLOCATION REQUEST	630

-	BEARER RESOURCE MODIFICATION REJECT	631

-	BEARER RESOURCE MODIFICATION REQUEST	632

-	DEACTIVATE EPS BEARER CONTEXT ACCEPT	633

-	DEACTIVATE EPS BEARER CONTEXT REQUEST	633

-	ESM DATA TRANSPORT	634

-	ESM DUMMY MESSAGE	634

-	ESM INFORMATION REQUEST	635

-	ESM INFORMATION RESPONSE	636

-	ESM STATUS	637

-	MODIFY EPS BEARER CONTEXT ACCEPT	638

-	MODIFY EPS BEARER CONTEXT REJECT	638

-	MODIFY EPS BEARER CONTEXT REQUEST	639

-	NOTIFICATION	640

-	PDN CONNECTIVITY REJECT	640

-	PDN CONNECTIVITY REQUEST	641

-	PDN DISCONNECT REJECT	643

-	PDN DISCONNECT REQUEST	644

4.7A	Default TC message and information element contents	644

-	ACTIVATE TEST MODE	645

-	ACTIVATE TEST MODE COMPLETE	645

-	CLOSE UE TEST LOOP	646

-	CLOSE UE TEST LOOP COMPLETE	650

-	DEACTIVATE TEST MODE	650

-	DEACTIVATE TEST MODE COMPLETE	650

-	OPEN UE TEST LOOP	651

-	OPEN UE TEST LOOP COMPLETE	651

-	UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST	651

-	UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE	651

-	UE TEST LOOP PROSE PACKET COUNTER REQUEST	652

-	UE TEST LOOP PROSE PACKET COUNTER RESPONSE	652

-	UE TEST LOOP MODE F SCPTM PACKET COUNTER REQUEST	652

-	UE TEST LOOP MODE F SCPTM PACKET COUNTER RESPONSE	653

4.7B	Default UTRA message and information element contents	654

4.7B.1	UTRA RRC messages	654

–	HANDOVER TO UTRAN COMMAND	654

–	HANDOVER FROM UTRAN COMMAND	666

–	MEASUREMENT CONTROL	666

–	MEASUREMENT REPORT	668

–	PHYSICAL CHANNEL RECONFIGURATION	669

–	PHYSICAL CHANNEL RECONFIGURATION COMPLETE	671

–	RRC CONNECTION REQUEST	671

–	SECURITY MODE COMMAND	672

–	SECURITY MODE COMPLETE	672

–	UTRAN MOBILITY INFORMATION	672

–	UTRAN MOBILITY INFORMATION CONFIRM	672

4.7B.2	UTRA NAS messages	672

4.7C	Default DS-MIPv6 message and information element contents	682

4.7C.1	IKEv2 messages	682

-	IKEv2 IKE_SA_INIT Request	682

-	IKE_SA_INIT Response	685

-	IKE_AUTH_Request	686

-	IKE_AUTH Response	689

4.7C.2	Messages used to perform DS-MIPv6 registration and deregistration	693

-	Router Advertisement	693

-	Binding Update	694

-	Binding Acknowledgement	695

-	Binding Revocation Indication	696

-	Binding Revocation Acknowledgement	697

4.7D	Default GERAN message and information element contents	698

4.7D.1	GPRS message	698

–	PS HANDOVER COMMAND	698

4.7E	Default HTTP messages for communication with the ProSe Function	699

-	HTTP Request	699

-	HTTP Response	699

4.7F	Default ProSe messages	700

4.7F.1	ProSe discovery messages	700

-	DISCOVERY_REQUEST	700

-	DISCOVERY_RESPONSE	701

-	MATCH_REPORT	705

-	MATCH_REPORT_ACK	706

-	PC5_DISCOVERY	707

4.7F.2	Messages transmitted over the PC3ch interface	711

-	USAGE_INFORMATION_REPORT_LIST	711

-	USAGE_INFORMATION_REPORT_LIST_RESPONSE	713

4.7F.3	ProSe Direct Communication Messages	714

-	DIRECT_COMMUNICATION_ACCEPT	714

-	DIRECT_COMMUNICATION_KEEPALIVE	714

-	DIRECT_COMMUNICATION_KEEPALIVE_ACK	715

-	DIRECT_COMMUNICATION_RELEASE	715

-	DIRECT_COMMUNICATION_RELEASE_ACCEPT	715

-	DIRECT_COMMUNICATION_REQUEST	716

-	DIRECT_SECURITY_MODE_COMMAND	717

-	DIRECT_SECURITY_MODE_COMPLETE	718

-	KEY_REQUEST	718

-	KEY_RESPONSE	719

-	MIKEY Key Delivery Message	720

-	MIKEY Verification Message	723

4.7G	Default IKEv2 message and information element contents	724

-	IKE_SA_INIT request	724

-	IKE_SA_INIT response	726

-	IKE_AUTH_request	726

-	IKE_AUTH response	727

4.7H	Default TLS message and information element contents	727

-	ClientHello	727

-	ServerHello	727

-	ServerKeyExchange	728

-	ServerHelloDone	728

-	ClientKeyExchange	728

-	ChangeCipherSpec	728

-	Finished	728

4.7I	Default AT Command message and information element	729

-	AT Command +CATM	729

-	AT Command +CCUTLE	729

-	AT Command +CUTCR	729

-	AT Command +CUSPCREQ	730

4.8	Reference radio bearer configurations	730

4.8.1	General	730

4.8.2	SRB and DRB parameters and combinations	730

4.8.2.1	SRB and DRB parameters	730

4.8.2.1.1	SRB configurations	730

4.8.2.1.2	DRB PDCP configurations	731

4.8.2.1.3	DRB RLC configurations	733

4.8.2.1.4	DRB Logical Channel configurations	734

4.8.2.1.5	MAC configurations	735

4.8.2.1.6	Physical Layer configurations	739

4.8.2.1.7	DRB configurations	749

4.8.2.2	SRB and DRB combinations	749

4.8.2.2.1	Combinations on DL-SCH and UL-SCH	749

4.8.3	UTRA reference radio parameters and combinations	749

4.8.4	GERAN reference PDP context parameters	750

4.9	Common test USIM, CSIM and ISIM parameters	750

4.9.1	General	750

4.9.1.1	Definitions	750

4.9.1.2	Definition of the test algorithm for authentication	750

4.9.1.2.1	Authentication and key derivation in the test USIM, CSIM and ISIM and SS	750

4.9.1.2.2	Generation of re-synchronization parameters in the USIM, CSIM and ISIM	751

4.9.1.2.3	Using the authentication test algorithm for UE conformance testing	751

4.9.2	Default parameters for the test USIM, CSIM and ISIM	751

4.9.3	Default settings for the Elementary Files (EFs)	751

4.9.3.1	Modified contents of the USIM Elementary Files and additional USIM Elements files at the DF ProSe level	751

4.9.3.2	Modified contents of the CSIM Elementary Files	758

4.9.3.3	Modified contents of the USIM Elementary Files and additional USIM Elements files at the MCPTT level	769

4.9.3.4	Modified contents of the USIM Elementary Files and additional USIM Elements files at the DFV2X level	770

4.9.3.5	Modified contents of the Elementary Files for eCall over IMS	771

4.10	V2X reference configuration	771

4.10.1	Reference configuration for V2X Sidelink Communication	771

4.10.1.1	V2X Sidelink Communication Preconfiguration for out-of-network coverage operation	771

4.10.2	V2X management data object configuration	772

4.11	GNSS Requirements for V2X and MCS testing	775

4.11.1	General	775

4.11.2	GNSS Scenarios	775

4.12	GNSS Requirements for aerial testing	780

4.12.1	General	780

4.12.2	GNSS Scenarios	780

4.13	UE position requirements for NTN testing	782

4.14	Common test environment for Vertical UEs	782

4.14.1	eMTC NTN	782

5	Test environment for RF test	783

5.1	Requirements of test equipment	783

5.2	RF Reference system configurations	783

5.2.1	Common parameters for simulated E-UTRA cells	783

5.2.1.1	Combinations of system information blocks	783

5.2.1.2	Scheduling of system information blocks	784

5.2.1.3	Common contents of system information messages	784

5.2A	Generic RF procedures	786

5.2A.1	UE RF test states	787

5.2A.1A	Registered, Idle Mode, UE Test Mode Activated (State 2A-RF)	788

5.2A.1A.1	Initial conditions	788

5.2A.1A.2	Definition of system information messages	788

5.2A.1A.3	Procedure	789

5.2A.1A.4	Specific message contents	790

5.2A.1AA	Registered, Idle Mode, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-RF-CE)	792

5.2A.1AA.1	Initial conditions	792

5.2A.1AA.2	Definition of system information messages	792

5.2A.1AA.3	Procedure	792

5.2A.1AA.4	Specific message contents	793

5.2A.2	Generic Default Radio Bearer Establishment, UE Test Mode Activated (State 3A-RF)	793

5.2A.2.1	Initial conditions	793

5.2A.2.2	Definition of system information messages	793

5.2A.2.3	Procedure	793

5.2A.2.4	Specific message contents	794

5.2A.2A	DC MCG/SCG Dedicated RB established, UE Test Mode Activate (State 3A-RF-DC1)	794

5.2A.2A.1	Initial conditions	794

5.2A.2A.2	Definition of system information messages	794

5.2A.2A.3	Procedure	794

5.2A.2A.4	Specific message contents	794

5.2A.2AA	Generic Default Radio Bearer Establishment, UE Test Mode Activated in cell supporting BL/CE UE (State 3A-RF-CE)	795

5.2A.2AA.1	Initial conditions	795

5.2A.2AA.2	Definition of system information messages	795

5.2A.2AA.3	Procedure	796

5.2A.2AA.4	Specific message contents	796

5.2A.2B	DC Split Default RB established, UE Test Mode Activate (State 3A-RF-DC2)	797

5.2A.2B.1	Initial conditions	797

5.2A.2B.2	Definition of system information messages	797

5.2A.2B.3	Procedure	797

5.2A.2B.4	Specific message contents	798

5.2A.2C	Generic Default Radio Bearer Establishment, UE Test Mode Activated, V2X Setup (State 3A-RF-V2X)	798

5.2A.2C.1	Initial conditions	798

5.2A.2C.2	Definition of system information messages	798

5.2A.2C.3	Procedure	798

5.2A.2C.4	Specific message contents	798

5.2A.3	Loopback Activation without looped data (State 4A-RF)	798

5.2A.3.1	Initial conditions	798

5.2A.3.2	Definition of system information messages	799

5.2A.3.3	Procedure	799

5.2A.3.4	Specific message contents	799

5.2A.3A	DC MCG/SCG DRBs Loopback Activation without looped data (State 4A-RF-DC1)	799

5.2A.3A.1	Initial conditions	799

5.2A.3A.2	Definition of system information messages	800

5.2A.3A.3	Procedure	800

5.2A.3A.4	Specific message contents	800

5.2A.3AA	Loopback Activation without looped data in cell supporting BL/CE UE (State 4A-RF-CE)	800

5.2A.3AA.1	Initial conditions	800

5.2A.3AA.2	Definition of system information messages	800

5.2A.3AA.3	Procedure	801

5.2A.3AA.4	Specific message contents	801

5.2A.3B	DC Split DRB Loopback Activation without looped data (State 4A-RF-DC2)	801

5.2A.3B.1	Initial conditions	801

5.2A.3B.2	Definition of system information messages	801

5.2A.3B.3	Procedure	801

5.2A.3B.4	Specific message contents	801

5.2A.3C	Loopback Activation without looped data, V2X Setup (State 4A-RF-V2X)	801

5.2A.3C.1	Initial conditions	801

5.2A.3C.2	Definition of system information messages	802

5.2A.3C.3	Procedure	802

5.2A.3C.4	Specific message contents	802

5.2A.4	Procedure to configure SCC	802

5.2 A.41.	Specific message contents	802

5.2A.4.1.1	Exceptions for all CA tests	802

5.2A.4.1.2	Exceptions for UL CA tests	803

5.2A.5	Exceptions for feICIC tests	804

5.2A.5.1	Specific message contents	804

5.2A.5.1.1	Neighbour cell info for all feICIC test cases	804

5.2A.6	Exceptions for NAICS tests	805

5.2A.6.1	NAICS specific RRC Connection reconfiguration procedure	805

5.2A.6.1.1	Procedure	805

5.2A.6.1.1	Specific message contents	805

5.2A.6.2	Specific message contents	805

5.2A.6.2.1	RRCConnectionReconfiguration for setting up and releasing NAICS configuration in NAICS test cases	806

5.2A.7	Procedure to retrieve additional UE Capabilities for Rel-11 and higher UEs that support frequencyBandRetrieval_r11	807

5.2A.7.1	Initial conditions	807

5.2A.7.2	Definition of system information messages	808

5.2A.7.3	Procedure	808

5.2A.7.4	Specific message contents	808

5.2A.8	Exceptions for DL Control channel interference mitigation (Type A and Type B) tests	809

5.2A.8.1	Specific message contents	809

5.2A.8.1.1	Neighbour cell info for all CCIM test cases	809

5.3	Default RRC message and information elements contents	809

5.3.1	Radio resource control information elements	809

5.3.2	Measurement information elements	811

5.4	Default NAS message and information elements contents	811

5.5	Reference radio bearer configurations	811

5.5.1	SRB and DRB parameters	811

5.5.1.1	MAC configurations	811

5.5.1.2	Physical Layer configurations	813

5.5.1.3	SRB and DRB combinations	814

5.5.1.3.1	Combinations on DL-SCH and UL-SCH	814

5.6	Test environment for NTN testing	814

5.6.1	UE location	814

5.6.2	Ephemeris Information	814

5.6.2.0	Assumptions for Ephemeris generation	814

5.6.2.1	Ephemeris for zero Doppler conditions	814

5.6.3	eMTC NTN message contents for RF Tx/Rx, Demodulation tests	816

5.6.3.1	ServingSatelliteInfo-r17 values in SIB31	816

6	Test environment for Signalling test	817

6.1	Requirements of test equipment	817

6.2	Reference test conditions	817

6.2.1	Physical channel allocations	817

6.2.1.1	Antennas	817

6.2.1.2	Downlink physical channels and physical signals	817

6.2.1.3	Mapping of downlink physical channels and signals to physical resources	818

6.2.1.4	Uplink physical channels and physical signals	818

6.2.1.5	Mapping of uplink physical channels and signals to physical resources	818

6.2.2	Signal levels	818

6.2.2.1	Downlink signal levels	818

6.2.2.2	Measurement accuracy and side conditions	819

6.2.2.3	Uplink signal levels	820

6.2.3	Default test frequencies	821

6.2.3.1	Test frequencies for signalling test	821

6.2.3.2	Test frequencies for CA signalling test	824

6.2.3.3	Test frequencies for ProSe signalling test	836

6.2.3.4	Test frequencies for MFBI frequency band priority adjustment signalling test	837

6.2.3.5	Test frequencies for V2X Communication	837

6.3	Reference system configurations	838

6.3.1	Default parameter specific for simulated cells	838

6.3.1.1	Intra-frequency neighbouring cell list in SIB4 for E-UTRA cells	838

6.3.1.2	Inter-frequency carrier frequency list in SIB5 for E-UTRA cells	838

6.3.1.3	UTRA carrier frequency list in SIB6 for E-UTRA cells	839

6.3.1.4	GERAN carrier frequency group list in SIB7 for E-UTRA cells	840

6.3.1.5	CDMA2000 HRPD carrier frequency list in SIB8 for E-UTRA cells	840

6.3.1.6	CDMA2000 1xRTT carrier frequency list in SIB8 for E-UTRA cells	841

6.3.1.7	E-UTRA carrier frequency list in SIB19 for UTRA cells	841

6.3.1.8	NR carrier frequency list in SIB24 for E-UTRA cells	841

6.3.2	Default configurations for NAS test cases	841

6.3.2.1	Simulated network scenarios for NAS test cases	841

6.3.2.2	Simulated NAS cells	842

6.3.2.3	Broadcast system information	843

6.3.2.3.1	Intra-frequency neighbouring cell list in SIB4 for E-UTRA NAS cells	843

6.3.2.3.2	Inter-frequency carrier frequency list in SIB5 for E-UTRA NAS cells	844

6.3.3	Cell configurations	844

6.3.3.1	Full cell configuration	845

6.3.3.2	Minimum uplink cell configuration	845

6.3.3.3	Broadcast only cell configuration	845

6.3.3.3A	Virtual cell configuration	845

6.3.3.4	Application of different cell configurations	845

6.3.4	SCell configurations	846

6.3.5	Satellite ephemeris information for NTN	846

6.3.5.1	GSO scenario	846

6.3.5.2	NGSO scenario	846

6.3.6	Signalling test environment for Vertical UEs	847

6.3.6.1	eMTC NTN	847

6.4	Generic procedures	848

6.4.1	Initial UE states and setup procedures	848

6.4.1.1	Initial UE states and setup procedures	848

6.4.1.2	Dedicated Bearer Establishment (to state 5)	849

6.4.1.2.1	Initial conditions	849

6.4.1.2.2	Definition of system information messages	850

6.4.1.2.3	Procedure	850

6.4.1.2.4	Specific message contents	850

6.4.1.2A	DC MCG/SCG Dedicated Bearer Establishment (to state 5A)	850

6.4.1.2A.1	Initial conditions	850

6.4.1.2A.2	Definition of system information messages	850

6.4.1.2A.3	Procedure	850

6.4.1.2A.4	Specific message contents	851

6.4.1.2B	DC Split Dedicated Bearer Establishment (to state 5B)	851

6.4.1.2B.1	Initial conditions	851

6.4.1.2B.2	Definition of system information messages	851

6.4.1.2B.3	Procedure	851

6.4.1.2B.4	Specific message contents	852

6.4.1.3	Loopback Activation (to state 6)	852

6.4.1.3.1	Initial conditions	852

6.4.1.3.2	Definition of system information messages	852

6.4.1.3.3	Procedure	853

6.4.1.3.4	Specific message contents	853

6.4.1.3A	DC MCG/SCG DRB Loopback Activation (to state 6A)	853

6.4.1.3A.1	Initial conditions	853

6.4.1.3A.2	Definition of system information messages	853

6.4.1.3A.3	Procedure	853

6.4.1.3A.4	Specific message contents	853

6.4.1.3B	DC Split DRB Loopback Activation (to state 6B)	853

6.4.1.3B.1	Initial conditions	853

6.4.1.3B.2	Definition of system information messages	854

6.4.1.3B.3	Procedure	854

6.4.1.3B.4	Specific message contents	854

6.4.2	Test procedures	854

6.4.2.1	Introduction	854

6.4.2.2	Test procedure to check RRC_IDLE state	854

6.4.2.3	Test procedure to check RRC_CONNECTED state	855

6.4.2.4	Test procedure Paging (for NAS testing)	855

6.4.2.5	Test procedure for no response to paging (for NAS testing)	855

6.4.2.6	Test procedure to check that a dedicated EPS bearer context is active (for NAS testing)	856

6.4.2.7	Test procedure to check that UE is camped on a new E-UTRAN cell	856

6.4.2.7A	Test procedure to check that UE is camped on E-UTRAN cell upon mobility from another RAT	857

6.4.2.7B	Test procedure to check that UE is camped on a new E-UTRAN cell / UP CIoT	861

6.4.2.8	Test procedure to check that UE is camped on a new UTRAN cell	861

6.4.2.9	Test procedure to check that UE is camped on a new GERAN cell	862

6.4.2.10	Test procedure to check that UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA	863

6.4.3	Reference test procedures for TTCN development	865

6.4.3.1	UE triggered establishment of a dedicated EPS bearer context	866

6.4.3.2	UE triggered establishment of a default EPS bearer context associated with an additional PDN	867

6.4.3.3	UE triggered modification of an EPS bearer context	869

6.4.3.4	UE triggered deletion of an EPS bearer context	870

6.4.3.5	UE triggered CS call	871

6.4.3.6	UE triggered MO SMS over SGs	872

6.4.3.7	CS fallback to UTRAN procedures (LAI of UTRAN cell same as the LAI received in combined Attach procedure in EUTRA cell)	872

6.4.3.7.1	CS fallback to UTRAN with redirection / MT call (PS bearers not established)	873

6.4.3.7.2	CS fallback to UTRAN with redirection / MO call (PS bearers not established)	874

6.4.3.7.3	CS fallback to UTRAN with redirection / MT call (PS bearer established)	875

6.4.3.7.4	CS fallback to UTRAN with redirection / MO call (PS bearer established)	875

6.4.3.7.5	CS fallback to UTRAN with Handover / MT call	876

6.4.3.7.5.1	Specific message contents	877

6.4.3.7.6	CS fallback to UTRAN with Handover / MO call	878

6.4.3.7.6.1	Specific message contents	879

6.4.3.7.7	CS fallback to UTRAN with Handover / emergency call	880

6.4.3.7.7.1	Specific message contents	881

6.4.3.8	CS fallback to GERAN procedures (LAI of GERAN cell same as the LAI received in combined Attach procedure in EUTRA cell)	881

6.4.3.8.1	CS fallback to GERAN with redirection or CCO / MT call (DTM not supported)	882

6.4.3.8.2	CS fallback to GERAN with redirection or CCO / MO call (DTM not supported)	883

6.4.3.8.3	CS fallback to GERAN with PS Handover / MT call (EDTM not supported)	883

6.4.3.8.4	CS fallback to GERAN with PS Handover / MO call (EDTM not supported)	883

6.4.3.8.5	CS fallback to GERAN with PS Handover / MT call (EDTM supported)	883

6.4.3.9	SRVCC Handover to UTRA	883

6.4.3.9.1	Specific message contents	884

6.4.3.10	Offload to WLAN	884

6.4.3.10.1	Specific message contents	885

6.4.3.11	Offload from WLAN	885

6.4.3.11.1	Specific message contents	886

6.4.3.12	Check UE does not offload to WLAN	886

6.4.3.12.1	Specific message contents	886

6.4.3.13	Check UE does not Offload to E-UTRAN	886

6.4.3.14	Procedure for UE initiated detach at non-switch-off	887

6.4.3.13.1	Specific message contents	887

6.4.3A	Test case postambles	887

6.4.3A.1	Introduction	887

6.4.3A.2	Reference end states	887

6.5	Default RRC message and information element contents	890

6.5.1	Measurement information elements	891

-	MeasConfig-DEFAULT	891

-	MeasGapConfig-GP1	891

-	MeasGapConfig-GP2	892

-	MeasGapConfig-CE	892

6.6	Default NAS message and information element contents	892

6.6.1	Reference default EPS bearer contexts	892

6.6.2	Reference dedicated EPS bearer contexts	893

6.6A	Default SMS over SGs message and information element contents	903

6.6A.1	CM-sublayer messages	903

-	CP-ACK	903

-	CP-DATA	904

6.6A.2	Short Message Relay Layer (SM-RL) messages	904

-	RP-ACK RPDU	904

-	RP-DATA RPDU	905

6.6A.3	Short Message Transfer Layer (SM-TL) messages	905

-	SMS-DELIVER	905

-	SMS-SUBMIT	906

6.6B	Reference radio bearer configurations	906

6.6B.1	SRB and DRB parameters and combinations	906

6.6B.1.1	SRB and DRB parameters	906

6.6B.1.1.1	Physical Layer configurations	906

6.7	Timer Tolerances	907

6.8	SideLink reference configuration	907

6.8.1	Reference configuration for Direct Communication	907

6.8.1.1	ProSe Direct Communication Preconfiguration for out-of-network coverage operation	907

6.8.2	Reference configuration for V2X Sidelink Communication	910

6.8.2.1	V2X Sidelink Communication Preconfiguration for out-of-network coverage operation	910

7	Test environment for RRM tests	910

7.1	Requirements of test equipment	910

7.2	RRM Reference system configurations	910

7.2.1	Common parameters for simulated E-UTRA cells	910

7.2.1.1	Combinations of system information blocks	910

7.2.1.2	Scheduling of system information blocks	910

7.2.1.3	Common contents of system information messages	910

7.2.2	Common parameters for simulated GERAN cells	912

7.2.2.1	Mapping of GERAN cells	912

7.2A	Generic RRM procedures	913

7.2A.1	UE RRM test states	913

7.2A.2	UE Registration, UE Test Mode Activated (State 2A-RF)	913

7.2A.2A	UE Registration, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-RF-CE)	913

7.2A.3	Generic Default Radio Bearer Establishment, UE Test Mode Activated (State 3A-RF)	914

7.2A.3A	DC MCG/SCG Dedicated RB established, UE Test Mode Activated (State 3A-RF-DC1)	914

7.2A.3AA	Generic Default Radio Bearer Establishment, UE Test Mode Activated in cell supporting BL/CE UE (State 3A-RF-CE)	914

7.2A.3B	DC Split Default RB established, UE Test Mode Activated (State 3A-RF-DC2)	914

7.2A.3C	Generic Default Radio Bearer Establishment, UE Test Mode Activated, V2X Setup (State 3A-RF-V2X)	914

7.2A.4	Generic Default Radio Bearer Establishment, UE Test Mode Activated, pre-registration on HRPD (State 3B-RF)	914

7.2A.4.1	Initial conditions	914

7.2A.4.2	Definition of system information messages	914

7.2A.4.3	Procedure	915

7.2A.4.4	Specific message contents	915

7.2A.5	Procedure to configure SCC	915

7.2A.6	Exceptions for feICIC tests	915

7.2B	Other generic RRM procedures	915

7.2B.1	Tracking area updating procedure	915

7.3	Default RRC message and information elements contents	917

7.3.1	Contents of RRC messages	917

7.3.2	Radio resource control information elements	917

7.3.3	Measurement information elements	918

7.3A	Default UTRA message and information element contents	919

7.3A.1	UTRA RRC messages	919

7.4	Default NAS message and information elements contents	919

7.5	Reference radio bearer configurations	919

7.5.1	SRB and DRB parameters	919

7.5.1.1	MAC configurations	919

7.6	Test environment for NTN testing	920

7.6.1	UE location	920

7.6.2	Ephemeris Information	920

7.6.2.0	Assumptions for Ephemeris generation	920

7.6.2.1	Ephemeris for elevation angle 30º	920

7.6.2.2	Ephemeris for eMTC NTN RRM UL timing accuracy test cases	925

7.6.3	eMTC NTN message contents for RRM tests	927

7.6.3.1	ServingSatelliteInfo-r17 values in SIB31	927

7.6.3.2	SystemInformationBlockType33	929

8	NB-IoT test environment	930

8.1	NB-IoT Common test environment	930

8.1.1	NB-IoT Environmental conditions	930

8.1.2	NB-IoT Common requirements of test equipment	930

8.1.3	NB-IoT Reference test conditions	930

8.1.3.1	NB-IoT Test frequencies	930

8.1.3.1.1	NB-IoT FDD Mode Test frequencies	931

8.1.3.1.2	NB-IoT TDD Mode Test frequencies	962

8.1.3.2	NB-IoT Radio conditions	964

8.1.3.2.1	NB-IoT Normal propagation condition	964

8.1.3.3	NB-IoT Physical channel allocations	964

8.1.3.3.1	NB-IoT Antennas	964

8.1.3.3.2	NB-IoT Downlink physical channels and physical signals	964

8.1.3.3.3	NB-IoT Mapping of downlink physical channels and signals to physical resources	965

8.1.3.3.4	NB-IoT Uplink physical channels and physical signals	967

8.1.3.3.5	NB-IoT Mapping of uplink physical channels and signals to physical resources	968

8.1.3.4	NB-IoT Signal levels	968

8.1.3.4.1	NB-IoT Downlink signal levels	968

8.1.3.4.2	NB-IoT Uplink signal levels	968

8.1.3.5	NB-IoT Standard test signals	968

8.1.3.5.1	NB-IoT Downlink test signals	968

8.1.3.5.2	NB-IoT Uplink test signals	968

8.1.3.6	NB-IoT Physical layer parameters	969

8.1.3.6.1	NB-IoT Downlink physical layer parameters	969

8.1.4	NB-IoT Reference system configurations	971

8.1.4.1	NB-IoT Simulated network scenarios	972

8.1.4.1.1	NB-IoT Single cell network scenarios	972

8.1.4.1.2	NB-IoT single mode multi cell network scenarios	972

8.1.4.2	NB-IoT Simulated cells	972

8.1.4.3	NB-IoT Common parameters for simulated cells	976

8.1.4.3.1	NB-IoT Common configurations of system information blocks	976

8.1.4.3.1.1	NB-IoT Combinations of system information blocks	976

8.1.4.3.1.2	NB-IoT Scheduling of system information blocks	977

8.1.4.3.2	NB-IoT Common contents of system information messages	979

-	MasterInformationBlock-NB	979

-	MasterInformationBlock-TDD-NB	981

-	SystemInformation-NB	983

-	SystemInformationBlockType1-NB	984

8.1.4.3.3	NB-IoT Common contents of system information blocks	986

-	SystemInformationBlockType2-NB	986

-	SystemInformationBlockType3-NB	987

-	SystemInformationBlockType4-NB	987

-	SystemInformationBlockType5-NB	988

-	SystemInformationBlockType14-NB	989

-	SystemInformationBlockType15-NB	990

-	SystemInformationBlockType16-NB	990

-	SystemInformationBlockType20-NB	990

-	SystemInformationBlockType22-NB	991

-	SystemInformationBlockType23-NB	991

-	SystemInformationBlockType31-NB	992

-	SystemInformationBlockType32-NB	993

-	SystemInformationBlockType33-NB	993

8.1.5	NB-IoT Generic procedures	994

8.1.5.0	General	994

8.1.5.1	NB-IoT UE test states	994

8.1.5.2	NB-IoT UE Attach, Connected mode (State 2-NB)	996

8.1.5.2.0	General	996

8.1.5.2.1	Initial conditions	997

8.1.5.2.2	Definition of system information messages	997

8.1.5.2.3	Procedure	998

8.1.5.2.4	Specific message contents	1000

8.1.5.2A	NB-IoT UE Attach, Connected mode, UE Test Mode Activated (State 2A-NB)	1000

8.1.5.2A.0	General	1000

8.1.5.2A.1	Initial conditions	1000

8.1.5.2A.2	Definition of system information messages	1001

8.1.5.2A.3	Procedure	1001

8.1.5.2A.4	Specific message contents	1001

8.1.5.2B	NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB)	1002

8.1.5.2B.1	Initial conditions	1002

8.1.5.2B.2	Definition of system information messages	1002

8.1.5.2B.3	Procedure	1002

8.1.5.2B.4	Specific message contents	1002

8.1.5.3	NB-IoT UE Registered, Idle Mode (State 3-NB)	1003

8.1.5.3.1	Initial conditions	1003

8.1.5.3.2	Definition of system information messages	1004

8.1.5.3.3	Procedure	1004

8.1.5.3.4	Specific message contents	1004

8.1.5.3A	NB-IoT UE Registered, Idle Mode, UE Test Mode Activated (State 3A-NB)	1004

8.1.5.3A.1	Initial conditions	1004

8.1.5.3A.2	Definition of system information messages	1004

8.1.5.3A.3	Procedure	1004

8.1.5.3A.4	Specific message contents	1004

8.1.5.4	Void	1005

8.1.5A	Other generic procedures	1005

8.1.5A.1	Procedure for IP address allocation in the CP CIoT	1005

8.1.5A.2	Test procedure to check UE response to Paging for Control Plane CIoT MT access	1005

8.1.5A.2.1	Initial conditions	1005

8.1.5A.2.2	Definition of system information messages	1005

8.1.5A.2.3	Procedure	1006

8.1.5A.2.4	Specific message contents	1007

8.1.5A.3	Test procedure to check UE initiation of Control Plane CIoT MO user data transfer non-SMS transport	1010

8.1.5A.3.1	Initial conditions	1010

8.1.5A.3.2	Definition of system information messages	1010

8.1.5A.3.3	Procedure	1011

8.1.5A.3.4	Specific message contents	1011

8.1.5A.3A	Test procedure to check UE initiation of Control Plane CIoT MO user data transfer SMS transport	1015

8.1.5A.3A.1	Initial conditions	1015

8.1.5A.3A.2	Definition of system information messages	1015

8.1.5A.3A.3	Procedure	1016

8.1.5A.3A.4	Specific message contents	1017

8.1.5A.4	Test procedure to check release of PDN connectivity before leaving RRC-CONNECTED for attach without PDN	1020

8.1.5A.4.1	Initial conditions	1020

8.1.5A.4.2	Definition of system information messages	1020

8.1.5A.4.3	Procedure	1021

8.1.5A.4.4	Specific message contents	1021

8.1.5A.5	Test procedure to check that NB-IoT UE is camped on a new NB-IOT cell	1021

8.1.5A.6	Test procedure to check that NB-IoT UE resume RRC connection on a new NB-IOT cell	1022

8.1.5A.7		1023

8.1.5A.8	Test procedure to check RRC_CONNECTED state for NB-IoT	1023

8.1.5A.9	Test Procedure to establish radio bearers in User Plane	1023

8.1.5A.9.1	Initial conditions	1023

8.1.5A.9.2	Definition of system information messages	1023

8.1.5A.9.3	Procedure	1024

8.1.5A.9.4	Specific message contents	1024

8.1.5B	Common test environment for Vertical UEs	1024

8.1.5B.1	NB-IoT NTN	1024

8.1.6	NB-IoT Default RRC message and information elements contents	1024

8.1.6.1	NB-IoT Contents of RRC messages	1025

–	DLInformationTransfer-NB	1025

–	Paging-NB	1025

–	RRCConnectionReconfiguration-NB	1026

–	RRCConnectionReconfigurationComplete-NB	1026

–	RRCConnectionReestablishment-NB	1027

–	RRCConnectionReestablishmentComplete-NB	1027

–	RRCConnectionReestablishmentRequest-NB	1028

–	RRCConnectionReject-NB	1028

–	RRCConnectionRelease-NB	1029

–	RRCConnectionRequest-NB	1029

–	RRCConnectionResume-NB	1030

–	RRCConnectionResumeComplete-NB	1030

–	RRCConnectionResumeRequest-NB	1031

–	RRCConnectionSetup-NB	1031

–	RRCConnectionSetupComplete-NB	1032

–	UECapabilityEnquiry-NB	1033

–	UECapabilityInformation-NB	1033

–	ULInformationTransfer-NB	1034

8.1.6.2	NB-IoT System information blocks	1034

8.1.6.3	NB-IoT Radio resource control information elements	1034

–	BCCH-Config-NB-DEFAULT	1034

–	DL-CarrierConfigCommon-NB-DEFAULT	1034

–	PCCH-Config-NB-DEFAULT	1035

–	NPDCCH-ConfigDedicated-NB-DEFAULT	1035

–	NPDSCH-ConfigCommon-NB-DEFAULT	1035

–	NPRACH-ConfigSIB-NB-DEFAULT	1036

–	NPUSCH-ConfigCommon-NB-DEFAULT	1036

–	NPUSCH-ConfigDedicated-NB-DEFAULT	1036

–	RACH-ConfigCommon-NB-DEFAULT	1037

–	RadioResourceConfigCommonSIB-NB-DEFAULT	1038

–	RadioResourceConfigDedicated-NB-SRB	1039

–	RadioResourceConfigDedicated-NB-DRB(n)	1039

-	RadioResourceConfigDedicated-NB-DRB-ADD(bid)	1040

-	RadioResourceConfigDedicated-NB-DRB-REL(bid)	1040

–	RLC-Config-NB-SRB-RECONFIG	1040

–	SRB-ToAddModList-NB-RECONFIG	1041

–	UplinkPowerControlCommon-NB-DEFAULT	1041

–	UplinkPowerControlDedicated-NB-DEFAULT	1041

-	RadioResourceConfigDedicated-NB-DRB-Mod	1041

-	NPRACH-ConfigSIB-NB-v1530-DEFAULT	1042

-	NPRACH-ConfigSIB-NB-v1550-DEFAULT	1042

8.1.6.4	NB-IoT Security control information elements	1042

8.1.6.5	NB-IoT Other information elements	1043

–	RRC-TransactionIdentifier-DL	1043

–	RRC-TransactionIdentifier-UL	1043

8.1.7	NB-IoT Default NAS message and information element contents	1043

8.1.7A	NB-IoT Default TC message and information element contents	1043

8.1.8	NB-IoT Reference radio bearer configurations	1044

8.1.8.1	General	1044

8.1.8.2	NB-IoT SRB and DRB parameters and combinations	1044

8.1.8.2.1	NB-IoT SRB and DRB parameters	1044

8.1.9	NB-IoT Common test USIM parameters	1046

8.1.9.1	General	1046

8.2	NB-IoT Test environment for RF test	1047

8.2.1	NB-IoT Requirements of test equipment	1047

8.2.2	NB-IoT RF Reference system configurations	1047

8.2.2.1	NB-IoT Common parameters for simulated E-UTRA cells	1047

8.2.2.1.1	 NB-IoT Combinations of system information blocks	1047

8.2.2.1.2	NB-IoT Scheduling of system information blocks	1047

8.2.2.1.3	NB-IoT Common contents of system information messages	1047

8.2.2A	NB-IoT Generic RF procedures	1048

8.2.3	NB-IoT Default RRC message and information elements contents	1048

8.2.3.1	NB-IoT Radio resource control information elements	1048

8.2.4	NB-IoT Default NAS message and information elements contents	1049

8.2.5	NB-IoT Reference radio bearer configurations	1049

8.2.5.1	NB-IoT SRB and DRB parameters	1049

8.2.6	Test environment for NB-IoT NTN RF/Demod testing	1049

8.2.6.1	UE location	1049

8.2.6.2	Ephemeris Information	1049

8.2.6.2.0	Assumptions for Ephemeris generation	1049

8.2.6.2.1	Ephemeris for zero Doppler conditions	1049

8.2.6.3	NB-IoT NTN message contents for RF Tx/Rx, Demodulation tests	1051

8.2.6.3.1	ServingSatelliteInfo-r17 values in SIB31-NB	1051

8.3	NB-IoT Test environment for Signalling test	1051

8.3.1	NB-IoT Requirements of test equipment	1051

8.3.2	NB-IoT Reference test conditions	1051

8.3.2.1	NB-IoT Physical channel allocations	1051

8.3.2.2	NB-IoT Signal levels	1051

8.3.2.2.1	NB-IoT Downlink signal levels	1051

8.3.2.3	NB-IoT Default test frequencies	1052

8.3.2.3.1	NB-IoT FDD Mode Test frequencies for signalling test	1052

8.3.2.3.2	NB-IoT TDD Mode Test frequencies for signalling test	1054

8.3.3	NB-IoT Reference system configurations	1054

8.3.3.1	NB-IoT Default parameters specific for simulated cells	1055

8.3.3.1.1	Intra-frequency neighbouring cell list in SIB4-NB for NB-IoT cells	1055

8.3.3.1.2	Inter-frequency carrier frequency list in SIB5-NB for NB-IoT cells	1055

8.3.3.2	NB-IoT Default configurations for NAS test cases	1056

8.3.3.3	NB-IoT Cell configurations	1057

8.3.4	NB-IoT Generic signalling procedures	1057

8.3.4.1	NB-IoT Initial UE states and setup procedures	1057

8.3.4.2	NB-IoT Reference procedures and test procedures for TTCN development	1057

8.3.4.3	NB-IoT Test case postambles for TTCN development	1057

8.3.5	NB-IoT Default RRC message and information element contents	1057

8.3.6	NB-IoT Default NAS message and information element contents	1057

8.3.7	NB-IoT Timer tolerances	1057

8.3.8	Signalling test environment for Vertical UEs	1057

8.3.8.1	NB-IoT NTN	1057

8.4	NB-IOT Test environment for RRM tests	1058

8.4.1	NB-IoT Requirements of test equipment	1058

8.4.2	NB-IoT RRM Reference system configurations	1058

8.4.2.1	NB-IoT Common parameters for simulated NB-IoT cells	1058

8.4.2.1.1	NB-IoT Combinations of system information blocks	1058

8.4.2.1.2	NB-IoT Scheduling of system information blocks	1058

8.4.2.1.3	NB-IoT Common contents of system information messages	1058

8.4.2A	NB-IoT Generic RRM procedures	1060

8.4.3	NB-IoT Default RRC message and information elements contents	1060

8.4.3.1	NB-IoT Radio resource control information elements	1060

8.4.4	NB-IoT Default NAS message and information elements contents	1060

8.4.5	NB-IoT Reference radio bearer configurations	1060

8.4.5.1	NB-IoT SRB and DRB parameters	1060

8.4.6	Test environment for NB-IoT NTN RRM testing	1060

8.4.6.1	UE location	1060

8.4.6.2	Ephemeris Information	1060

8.4.6.2.0	Assumptions for Ephemeris generation	1060

8.4.6.2.1	Ephemeris for elevation angle 30º	1060

8.4.6.2.2	Ephemeris for NB-IoT NTN RRM UL timing accuracy test cases	1065

8.4.6.3	NB-IoT NTN message contents for RRM tests	1067

8.4.6.3.1	ServingSatelliteInfo-r17 values in SIB31-NB	1067

8.4.6.3.2	SystemInformationBlockType33-NB	1068

Annex A (informative): Connection Diagrams	1069

Annex B (normative): Void	1209

Annex C (informative): Change history	1210