Annex A (informative):
Connection Diagrams

Definition of Terms

GNSS: In this clause the term GNSS also includes the case where the only satellite system used is GPS.

System Simulator or SS: A device or system, that is capable of generating simulated Node B and/or eNode B signalling and analysing UE signalling responses on one RF channel, in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Control of the UE Tx output power through TPC commands.

2.	Measurement of signalling timing and delays.

3.	Ability to simulate UTRAN and/or E-UTRAN and/or NR signalling.

GNSS System Simulator or GSS: A device or system, that is capable of generating simulated GNSS satellite transmissions in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Control of the output power of individual satellites and the simulation of atmospheric delays and multi-path.

2.	Generation of appropriate assistance data to be transmitted to the UE via the SS.

3.	Ability to synchronize with UTRAN and/or E-UTRAN and/or NR timing in the SS.

MBS System Simulator or MSS: A device or system, that is capable of generating simulated MBS transmissions in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Control of the output power of individual beacons and the simulation of delays and multi-path.

2.	Generation of appropriate messaging to be transmitted to the UE via the SS.

WLAN System Simulator or WSS: A device or system, that is capable of generating simulated WLAN beacons in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Control of the output power of individual beacons and the simulation of delays and AWGN.

BLE System Simulator or BSS: A device or system, that is capable of generating simulated BLE advertising signals in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Control of the output power of individual BLE signals and the simulation of delays and AWGN.

Test System: A combination of devices brought together into a system for the purpose of making one or more measurements on a UE in accordance with the test case requirements. The following diagrams are all examples of Test Systems.

NOTE:	The above terms are logical definitions to be used to describe the test methods used in the present document, in practice, real devices called "System Simulators" may also include additional measurement capabilities or may only support those features required for the test cases they are designed to perform.

Figure A.1: Connection for A-GNSS Minimum Performance requirements tests
for UE with combined UTRAN and/or E-UTRAN and/or NR and GNSS antenna

Figure A.2: Connection for A-GNSS Minimum Performance requirements tests
for UE with separate UTRAN and/or E-UTRAN and/or NR and GNSS antennas

Figure A.3: Connection for 2 cells OTDOA tests with static propagation

Figure A.3a: Connection for 2 cells OTDOA tests with static propagation for 4Rx capable UE

Figure A.4: Connection for 3 cells OTDOA tests with multipath fading propagation conditions

Figure A.5: Connection for 1 cell ECID tests with static propagation conditions

Figure A.5a: Connection 1 cell ECID tests with static propagation for 4Rx capable UE

Figure A.6: Connection for MBS Minimum Performance requirements tests
for UE with combined UTRAN and/or E-UTRAN and/or NR and MBS antenna

Figure A.7: Connection for MBS Minimum Performance requirements tests
for UE with separate UTRAN and/or E-UTRAN and/or NR and MBS antennas

Figure A.8: Connection for WLAN tests
for UE with separate UTRAN and/or E-UTRAN and/or NR and WLAN antennas

Figure A.9: Connection for BLE tests
for UE with separate UTRAN and/or E-UTRAN and/or NR and BLE antennas

Figure A.10: Connection for A-GNSS Minimum Performance requirements tests
for UE with combined and/or E-UTRAN and/or NR FR1 and GNSS antenna and separate NR FR2 OTA connection

Figure A.11: Connection for A-GNSS Minimum Performance requirements tests
for UE with separate and/or E-UTRAN and/or NR FR1 and GNSS antennas and separate NR FR2 OTA connection

Figure A.12: Connection for 3 cells DL-TDOA tests with static propagation conditions in NR FR1

Figure A.13: Connection for 2 cells DL-TDOA and/or Multi-RTT and/or DL-AoD tests with static propagation conditions in NR FR1

Figure A.14: Connection for DL-TDOA and/or Multi-RTT and/or DL-AoD tests with static propagation conditions in NR FR2

Annex B (normative):
Converting A-GNSS UE-assisted measurement reports into position estimates

## B.1	Introduction

In this clause the terms GNSS and A-GNSS also include the cases where the only satellite system used is GPS unless otherwise stated.

To convert the A-GNSS UE measurement reports in case of UE-assisted mode of A-GNSS into position errors, a transformation between the "measurement domain" (code-phases, etc.) into the "state" domain (position estimate) is necessary. Such a transformation procedure is outlined in the following clauses. The details can be found in [8-10] and [12-17].

## B.2	UTRAN UE measurement reports for A-GPS L1 C/A only

In case of UTRAN UE-assisted A-GPS L1 C/A only, the measurement parameters are contained in the RRC UE POSITIONING GPS MEASURED RESULTS IE (clause 10.3.7.93 in 3GPP TS 25.331 [30]). The measurement parameters required for calculating the UE position are:

1)	Reference Time: The UE has two choices for the Reference Time:

a)	"UE GPS timing of cell frames";

b)	"GPS TOW msec".

2)	Measurement Parameters: 1 to <maxSat>:

a)	"Satellite ID (SV PRN)";

b)	"Whole GPS chips";

c)	"Fractional GPS Chips";

d)	"Pseudorange RMS Error".

Additional information required at the system simulator:

1)	"UE positioning GPS reference UE position" (clause 10.3.8.4c in 3GPP TS 25.331 [30]): 
Used for initial approximate receiver coordinates.

2)	"UE positioning GPS navigation model" (clause 10.3.7.94 in 3GPP TS 25.331 [30]): 
Contains the GPS ephemeris and clock correction parameters as specified in [8]; used for calculating the satellite positions and clock corrections.

3)	"UE positioning GPS ionospheric model" (clause 10.3.7.92 in 3GPP TS 25.331 [30]): 
Contains the ionospheric parameters which allow the single frequency user to utilize the ionospheric model as specified in [8] for computation of the ionospheric delay.

## B.3	UTRAN UE measurement reports for A-GNSS

In case of UTRAN UE-assisted A-GNSS, the measurement parameters are contained in the RRC UE POSITIONING GANSS MEASURED RESULTS IE (clause 10.3.7.93a in 3GPP TS 25.331 [30]). In case the UE provides also measurements on the GPS L1 C/A signal, the measurement parameters are contained in the RRC UE POSITIONING GPS MEASURED RESULTS IE (clause 10.3.7.93 in 3GPP TS 25.331 [30]). The measurement parameters required for calculating the UE position are:

1)	Reference Time: The UE has two choices for the Reference Time:

a)	"UE GANSS Timing of Cell Frames" and/or "UE GPS Timing of Cell Frames";

b)	"GANSS TOD msec" and/or "GPS TOW msec" if GPS L1 C/A signal measurements are also provided.

NOTE:	It is not expected that an UE will ever report both a GANSS TOD and a GPS TOW. However if two time stamps are provided and they derive from different user times, be aware that no compensation is made for this difference and this could affect the location accuracy.

2)	Measurement Parameters for each GANSS and GANSS Signal: 1 to <maxGANSSSat>:

a)	"Satellite ID"; mapping according to table 10.3.7.88b in 3GPP TS 25.331 [30];

b)	"GANSS Code Phase";

c)	"GANSS Integer Code Phase";

d)	"GANSS Integer Code Phase Extension";

e)	"Code Phase RMS Error";

3)	Additional Measurement Parameters in case of GPS L1 C/A signal measurements are also provided: 1 to <maxSat>:

a)	"Satellite ID (SV PRN)";

b)	"Whole GPS chips";

c)	"Fractional GPS Chips";

d)	"Pseudorange RMS Error".

Additional information required at the system simulator:

1)	"UE Positioning GANSS Reference UE Position" or "UE Positioning GPS Reference UE Position" (clause 10.3.8.4c in 3GPP TS 25.331 [30]):
Used for initial approximate receiver coordinates.

2)	"UE Positioning GANSS Navigation Model" and "UE Positioning GANSS Additional Navigation Models" (clauses 10.3.7.94a and 10.3.7.94b in 3GPP TS 25.331 [30]):
Contains the ephemeris and clock correction parameters as specified in the relevant ICD of each supported GANSS; used for calculating the satellite positions and clock corrections.

3)	"UE Positioning GANSS Ionospheric Model" (clause 10.3.7.92a in 3GPP TS 25.331 [30]):
Contains the ionospheric parameters which allow the single frequency user to utilize the ionospheric model as specified in [15] for computation of the ionospheric delay.

4)	"UE Positioning GANSS Additional Ionospheric Model" (clause 10.3.7.92b in 3GPP TS 25.331 [30]):
Contains the ionospheric parameters which allow the single frequency user to utilize the ionospheric model as specified in the relevant ICD of each supported GANSS [14], [37]  for computation of the ionospheric delay.

5)	"UE Positioning GANSS Time Model" (clause 10.3.7.97a in 3GPP TS 25.331 [30]):
Contains the GNSS-GNSS Time Offset for each supported GANSS. Note, that "UE Positioning GANSS Time Model" IE contains only the sub-ms part of the offset. Any potential integer seconds offset may be obtained from "UE Positioning GPS UTC Model" (clause 10.3.7.97 in 3GPP TS 25.331 [30]), "UE Positioning GANSS UTC Model" (clause 10.3.7.97c in 3GPP TS 25.331 [30]), or "UE Positioning GANSS Additional UTC Models" (clause 10.3.7.97d in 3GPP TS 25.331 [30]).

6)	"UE Positioning GPS Navigation Model" (clause 10.3.7.94 in 3GPP TS 25.331 [30]):
Contains the GPS ephemeris and clock correction parameters as specified in [8]; used for calculating the GPS satellite positions and clock corrections in case of GPS L1 C/A signal measurements are the only GPS measurements provided in addition to GANSS measurements.

7)	"UE Positioning GPS Ionospheric Model" (clause 10.3.7.92 in 3GPP TS 25.331 [30]):
Contains the ionospheric parameters which allow the single frequency user to utilize the ionospheric model as specified in [8] for computation of the ionospheric delay.

## B.4	E-UTRAN and NR UE measurement reports

In case of E-UTRAN and NR UE-assisted A-GNSS, the measurement parameters are contained in the LPP GNSS- SignalMeasurementInformation IE (clause 6.5.2.6 in 3GPP TS 36.355 [4] and 3GPP TS 37.355 [49]). The measurement parameters required for calculating the UE position are:

1)	Reference Time: The UE has two choices for the Reference Time:

a)	"networkTime";

b)	"gnss-TOD-msec".

2)	Measurement Parameters for each GNSS and GNSS signal: 1 to 64:

a)	" svID";

b)	"codePhase";

c)	"integerCodePhase";

d)	"codePhaseRMSError".

Additional information required at the system simulator:

1)	"GNSS-ReferenceLocation" (clause 6.5.2.2 in 3GPP TS 36.355 [4] and 3GPP TS 37.355 [49]):
Used for initial approximate receiver coordinates.

2)	"GNSS-NavigationModel" (clause 6.5.2.2 in 3GPP TS 36.355 [4] and 3GPP TS 37.355 [49]):
Contains the GNSS ephemeris and clock correction parameters as specified in the relevant ICD of each supported GNSS; used for calculating the satellite positions and clock corrections.

3)	"GNSS-IonosphericModel" (clause 6.5.2.2 in 3GPP TS 36.355 [4] and 3GPP TS 37.355 [49]):
Contains the ionospheric parameters which allow the single frequency user to utilize the ionospheric model as specified in the relevant ICD of each supported GNSS [8], [14], [15] and [37] for computation of the ionospheric delay.

## B.5	WLS position solution

The WLS position solution problem is concerned with the task of solving for four unknowns; xu, yu, zu the receiver coordinates in a suitable frame of reference (usually ECEF) and bu the receiver clock bias. It typically requires the following steps:

Step 1: Formation of pseudo-ranges

The observation of code phase reported by the UE for each satellite SVi is related to the pseudo-range/c modulo the "GNSS Code Phase Ambiguity" (UTRAN), or "gnss-CodePhaseAmbiguity" (E-UTRAN and NR), or modulo 1 ms (the length of the C/A code period) in case of GPS L1 C/A signal measurements. For the formation of pseudo-ranges, the integer number of milliseconds to be added to each code-phase measurement has to be determined first. Since 1 ms corresponds to a travelled distance of 300 km, the number of integer ms can be found with the help of reference location and satellite ephemeris. The distance between the reference location and each satellite SVi is calculated and the integer number of milliseconds to be added to the UE code phase measurements is obtained.

Step 2: Correction of pseudo-ranges for the GNSS-GNSS time offsets

In the case that the UE reports measurements for more than a single GNSS, the pseudo-ranges are corrected for the time offsets between the GNSSs relative to the selected reference time using the GNSS-GNSS time offsets available at the system simulator:

,

where is the measured pseudo-range of satellite i of GNSSm. The system time of GNSSk is the reference time frame, andis the available GNSS-GNSS time offset, and c is the speed of light.

Step 3: Formation of weighting matrix

The UE reported "codePhaseRMSError" (E-UTRAN and NR) or "Code Phase RMS Error" and/or "Pseudorange RMS Error" (UTRAN) values are used to calculate the weighting matrix for the WLS algorithm [9]. According to 3GPP TS 25.331 [30] and 3GPP TS 36.355 [4] and 3GPP TS 37.355 [49], the encoding for this field is a 6 bit value that consists of a 3 bit mantissa, Xi and a 3 bit exponent, Yi for each SVi:

The weighting Matrix W is defined as a diagonal matrix containing the estimated variances calculated from the "codePhaseRMSError" (E-UTRAN and NR) or "Code Phase RMS Error" and/or "Pseudorange RMS Error" (UTRAN) values:

Step 4: WLS position solution

The WLS position solution is described in reference [9] and usually requires the following steps:

1)	Computation of satellite locations at time of transmission using the ephemeris parameters and user algorithms defined in the relevant ICD of the particular GNSS. The satellite locations are transformed into WGS-84 reference frame, if needed.

2)	Computation of clock correction parameters using the parameters and algorithms as defined in the relevant ICD of the particular GNSS.

3)	Computation of atmospheric delay corrections using the parameters and algorithms defined in the relevant ICD of the particular GNSS for the ionospheric delay, and using the Gupta model in reference [10] p.121 equation (2) for the tropospheric delay. For GNSSs which do not natively provide ionospheric correction models (e.g., GLONASS), the ionospheric delay is determined using the available ionospheric model adapted to the particular GNSS frequency.

4)	The WLS position solution starts with an initial estimate of the user state (position and clock offset). The Reference Location is used as initial position estimate. The following steps are required:

a)	Calculate geometric range (corrected for Earth rotation) between initial location estimate and each satellite included in the UE measurement report.

b)	Predict pseudo-ranges for each measurement including clock and atmospheric biases as calculated in 1) to 3) above and defined in the relevant ICD of the particular GNSS and [9].

c)	Calculate difference between predicted and measured pseudo-ranges

d)	Calculate the "Geometry Matrix" G as defined in [9]:

with  where rsGNSSm,i is the satellite position vector for SVi of GNSSm (calculated in 1) above), and is the estimate of the user location.

e)	Calculate the WLS solution according to [9]:

f)	Adding the to the initial state estimate gives an improved estimate of the state vector:

.

5)	This new state vector can be used as new initial estimate and the procedure is repeated until the change in is sufficiently small.

Step 5: Transformation from Cartesian coordinate system to Geodetic coordinate system

The state vector  calculated in Step 4 contains the UE position in ECEF Cartesian coordinates together with the UE receiver clock bias relative to the selected GNSS system time. Only the user position is of further interest. It is usually desirable to convert from ECEF coordinates xu, yu, zu to geodetic latitude , longitude  and altitude h on the WGS84 reference ellipsoid.

Step 6: Calculation of "2-D Position Errors"

The latitude  / longitude obtained after Step 5 is used to calculate the 2-D position error.

Annex C (normative):
General test conditions and declarations

The requirements of this clause apply to all applicable tests in the present document.

In all the relevant clauses in this clause all 2D position error measurements shall be carried out according to the general rules for statistical testing in Annex D.

In this clause, the terms GNSS and A-GNSS also include the cases where the only satellite system used is GPS unless otherwise stated.

The test tolerances may not be valid for operating bands above 4200 MHz since some test system uncertainties are changed for frequencies above 4200 MHz. The test tolerances for bands above 4200 MHz are For Further Study [FFS].

## C.1	Acceptable uncertainty of Test System

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

It should be noted that the uncertainties in clause C.1 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the DUT and the Test System.

### C.1.1	Measurement of test environments

The measurement accuracy of the UE environmental test conditions, defined in Annex G or TS 36.508 [18] clause 4.1, shall be:

| Pressure | 5 kPa |
| --- | --- |
| Temperature | 2 degrees |
| Relative Humidity | 5 % |
| DC Voltage | 1.0 % |
| AC Voltage | 1.5 % |
| Vibration | 10 % |
| Vibration frequency | 0.1 Hz |

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

### C.1.2	A-GNSS Minimum Performance requirements

Table C.1.1: Maximum Test System Uncertainty for A-GNSS Minimum Performance tests

| Clause | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |
| --- | --- | --- | --- |
| 5.2.1, 6.2.1, 7.1.1, 13.2.1 Sensitivity Coarse Time Assistance | Coarse Time Assistance | 200 ms |  |
|  | Absolute GNSS signal level | 1 dB |  |
|  | Position error | 0.05 m | Position error consists of 0.05 m system uncertainty. The effect of position reporting resolution of approximately 1.2 m (see note) is not included in the allowable test system uncertainty but is included in the Test Parameter Relaxations since this resolution limitation limits the reporting capability of the UE. For simplicity the combined Test Parameter Relaxation is given as 1.3 m |
|  | Response time |  300 ms |  |
| 5.2.2, 6.2.2, 7.1.2, 13.2.2 Sensitivity Fine Time Assistance | Coarse Time Assistance | 200 ms |  |
|  | Fine Time Assistance | 1 us |  |
|  | Absolute GNSS signal level | 1 dB |  |
|  | Position error | 0.05 m | Position error as above |
|  | Response time |  300 ms |  |
| 5.3, 6.3, 7.2, 13.3 Nominal Accuracy | Coarse Time Assistance | 200 ms |  |
|  | Absolute GNSS signal level | 1 dB |  |
|  | Position error | 0.05 m | Position error as above |
|  | Response time |  300 ms |  |
| 5.4, 6.4, 7.3, 13.4 Dynamic Range | Coarse Time Assistance | 200 ms |  |
|  | Absolute GNSS signal level | 1 dB |  |
|  | Relative GNSS signal level | 0.2 dB |  |
|  | Position error | 0.05 m | Position error as above |
|  | Response time |  300 ms |  |
| 5.5, 6.5, 7.4, 13.5 Multi-path scenario | Coarse Time Assistance | 200 ms |  |
|  | Absolute GNSS signal level | 1 dB |  |
|  | Relative GNSS signal level | 0.2 dB |  |
|  | Position error | 0.05 m | Position error as above |
|  | Response time |  300 ms |  |
| 5.6, 6.6, 7.5, 7.5A, 13.6, 13.7 Moving scenario and periodic update | Absolute GNSS signal level | 1 dB |  |
|  | Position error | 0.05 m | Position error as above |
|  | Differential response time  100 ms |  100 ms |  |

NOTE:	For UE based mode the effect of position reporting resolution is given by:

meters, where R is the radius of the earth and φ is the latitude of the location. For the GNSS scenarios defined in TS 37.571-5 [20] this equates to approximately.

Editor’s note: this needs checking once the GNSS scenarios are agreed [TBD] m. For simplicity this is given as 1.2 m.

For UE assisted mode it is assumed that the output from the WLS position solution calculation in Annex B is coded using the same position coding method as for UE based mode before being used to calculate position error. Therefore the effect of reporting resolution will be the same as for UE based mode.

### C.1.3	ECID and OTDOA Measurement requirements

Table C.1.3-1: Maximum Test System Uncertainty for ECID and OTDOA Measurement Requirements

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.1.1 E-UTRAN FDD UE Rx – Tx time difference case (Rel-9 to Rel-11) | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | Note: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [26] |
| 8.1.1A E-UTRAN FDD UE Rx – Tx time difference case (Rel-12 onwards) | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.1B E-UTRAN FDD UE Rx – Tx time difference case for UE Category 1bis | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.2 E-UTRAN TDD UE Rx – Tx time difference case (Rel-9 to Rel-11) | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.2A E-UTRAN TDD UE Rx – Tx time difference case (Rel-12 onwards) | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.2B E-UTRAN TDD UE Rx – Tx time difference case for UE Category 1bis | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.3 E-UTRAN FDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB dB averaged over BWConfig ±3TS Uplink signal transmit timing relative to downlink | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [26] |
| 8.1.4 E-UTRAN TDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Same as 8.1.3 | Same as 8.1.3 |
| 8.1.5 E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB dB averaged over BWConfig Ês3 / Noc ±0.3 dB dB averaged over BWConfig ±3Ts Uplink signal transmit timing relative to downlink | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Ês3 / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [26] |
| 8.1.6 E-UTRAN TDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 8.1.5 | Same as 8.1.5 |
| 8.1.7 E-UTRAN FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.8 E-UTRAN HD-FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.9 E-UTRAN TDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 9.1.1 FDD RSTD Measurement Reporting Delay | Noc ±1.0 dB averaged over BWConfig PRS Ês1 / Noc ±0.6 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig PRS Ês2 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig PRS Ês3 / Noc ±0.6 dB averaged over BWConfig Ês3 / Noc ±0.6 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN PRS Ês / Noc and Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: PRS Ês / Noc and Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 9.1.1A FDD RSTD Measurement Reporting Delay for UE Category 1bis | Same as 9.1.1 |  |
| 9.1.2 TDD RSTD Measurement Reporting Delay | Same as 9.1.1 |  |
| 9.1.2A TDD RSTD Measurement Reporting Delay for UE Category 1bis | Same as 9.1.1 |  |
| 9.1.3 FDD RSTD Measurement Accuracy | Noc ±1.0 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 1 Ts | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 9.1.3A FDD RSTD Measurement Accuracy for UE Category 1bis | Same as 9.1.3 |  |
| 9.1.4 TDD RSTD Measurement Accuracy | Same as 9.1.3 |  |
| 9.1.4A TDD RSTD Measurement Accuracy for UE Category 1bis | Same as 9.1.3 |  |
| 9.2.1 FDD-FDD inter-frequency RSTD measurement reporting delay | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.6 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig PRS Ês3 / Noc2 ±0.6 dB averaged over BWConfig Ês3 / Noc2 ±0.6 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 PRS Ês3 / Noc2 and Ês3 / Noc2 are the ratios of cell 3 signal / AWGN for frequency 2 PRS Ês / Noc and Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: PRS Ês / Noc and Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 9.2.1A FDD-FDD inter-frequency RSTD measurement reporting delay for UE Category 1bis | Same as 9.2.1 |  |
| 9.2.2 TDD-TDD inter-frequency RSTD measurement reporting delay | Same as 9.2.1 |  |
| 9.2.2A TDD-TDD inter-frequency RSTD measurement reporting delay for UE Category 1bis | Same as 9.2.1 |  |
| 9.2.4 FDD-FDD inter frequency RSTD Accuracy | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 2 Ts | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 |
| 9.2.4A FDD-FDD inter frequency RSTD Accuracy for UE Category 1bis | Same as 9.2.4 |  |
| 9.2.5 TDD-TDD inter frequency RSTD Accuracy | Same as 9.2.4 |  |
| 9.2.5A TDD-TDD inter frequency RSTD Accuracy for UE Category 1bis | Same as 9.2.4 |  |
| 9.3.1.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.1.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.2.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.2.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.3.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.3.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.4.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.4.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.5.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.5.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.6.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.6.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.7.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.7.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.8.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.8.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.9.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.9.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.10.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.10.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.11.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.11.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.12.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.12.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.13 E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.14 E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.15 E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.16 E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.17 E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.18 E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions | Same as 9.1.3 | Same as 9.1.3 |
| 9.4.1.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.1.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.2.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.2.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.3.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.3.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.4.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.4.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.5.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.5.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.6.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.6.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.7.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.7.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.8.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.8.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.9.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.9.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.10.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.10.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.11.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.11.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.12.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.12.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.13 E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.14 E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.15 E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.16 E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.17 E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.18 E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions | Same as 9.2.4 | Same as 9.2.4 |
| 9.5.1 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | Noc ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 1 Ts | Note: NPRS Ês1 / Noc and Ês1 / Noc are the ratios of nCell 1 signal / AWGN NPRS Ês2 / Noc and Ês2 / Noc are the ratios of nCell 2 signal / AWGN |
| 9.5.2 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.5.1 | Same as 9.5.1 |
| 9.5.3 HD-FDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Noc ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig NPRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | Note: NPRS Ês1 / Noc and Ês1 / Noc are the ratios of nCell 1 signal / AWGN NPRS Ês2 / Noc and Ês2 / Noc are the ratios of nCell 2 signal / AWGN |
| 9.6.1 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 2 Ts | Note: NPRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of nCell 1 signal / AWGN for frequency 1 NPRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of nCell 2 signal / AWGN for frequency 2 |
| 9.6.2 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.6.1 | Same as 9.6.1 |
| 9.6.3 HD-FDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Same as 9.5.3 | Same as 9.5.3 |
| 9.7.1 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | Noc ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 1 Ts | Note: NPRS Ês1 / Noc and Ês1 / Noc are the ratios of nCell 1 signal / AWGN NPRS Ês2 / Noc and Ês2 / Noc are the ratios of nCell 2 signal / AWGN |
| 9.7.2 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.7.1 | Same as 9.7.1 |
| 9.7.3 TDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Noc ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig NPRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | Note: NPRS Ês1 / Noc and Ês1 / Noc are the ratios of nCell 1 signal / AWGN NPRS Ês2 / Noc and Ês2 / Noc are the ratios of nCell 2 signal / AWGN |
| 9.8.1 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 2 Ts | Note: NPRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of nCell 1 signal / AWGN for frequency 1 NPRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of nCell 2 signal / AWGN for frequency 2 |
| 9.8.2 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.8.1 | Same as 9.8.1 |
| 9.8.3 TDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Same as 9.7.3 | Same as 9.7.3 |
| 10.1 FDD RSTD Measurement Reporting Delay for Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.6 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig PRS Ês3 / Noc2 ±0.6 dB averaged over BWConfig Ês3 / Noc2 ±0.6 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 PRS Ês3 / Noc2 and Ês3 / Noc2 are the ratios of cell 3 signal / AWGN for frequency 2 PRS Ês / Noc and Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: PRS Ês / Noc and Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 10.1A FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz | Same as 10.1 | Same as 10.1 |
| 10.1B FDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth | Same as 10.1 | Same as 10.1 |
| 10.1C FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.1 | Same as 10.1 |
| 10.2 TDD RSTD Measurement Reporting Delay for Carrier Aggregation | Same as 10.1 | Same as 10.1 |
| 10.2A TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz | Same as 10.1 | Same as 10.1 |
| 10.2B TDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth | Same as 10.1 | Same as 10.1 |
| 10.2C TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.1 | Same as 10.1 |
| 10.2D TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz +10 MHz Bandwidth | Same as 10.1 | Same as 10.1 |
| 10.3 FDD RSTD Measurement Accuracy for Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig PRS Ês3 / Noc2 ±0.3 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 1 Ts | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 PRS Ês3 / Noc2 and Ês3 / Noc2 are the ratios of cell 3 signal / AWGN for frequency 2 |
| 10.3A FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-10 and Rel-11) | Same as 10.3 |  |
| 10.3A_1 FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-12 onwards) | Same as 10.3 | Same as 10.3 |
| 10.3B FDD RSTD Measurement Accuracy for Carrier Aggregation for 5 MHz+5 MHz Bandwidth | Same as 10.3 | Same as 10.3 |
| 10.3C FDD RSTD Measurement Accuracy for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.3 | Same as 10.3 |
| 10.4 TDD RSTD Measurement Accuracy for Carrier Aggregation | Same as 10.3 |  |
| 10.4A TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-10 and Rel-11) | Same as 10.3 |  |
| 10.4A_1 TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-12 onwards) | Same as 10.3 | Same as 10.3 |
| 10.4B TDD RSTD Measurement Accuracy for Carrier Aggregation for 5 MHz+5 MHz Bandwidth | Same as 10.3 | Same as 10.3 |
| 10.4C TDD RSTD Measurement Accuracy for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.3 | Same as 10.3 |
| 10.4D TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz+10 MHz Bandwidth | Same as 10.3 | Same as 10.3 |
| 10.5 FDD 3 DL CA RSTD Measurement Reporting Delay | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.6 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig PRS Ês3 / Noc3 ±0.6 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig PRS Ês4 / Noc3 ±0.6 dB averaged over BWConfig Ês4 / Noc3 ±0.6 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 PRS Ês3 / Noc3 and Ês3 / Noc3 are the ratios of cell 3 signal / AWGN for frequency 3 PRS Ês4 / Noc3 and Ês4 / Noc3 are the ratios of cell 4 signal / AWGN for frequency 3 PRS Ês / Noc and Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: PRS Ês / Noc and Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 10.6 TDD 3 DL CA RSTD Measurement Reporting Delay | Same as 10.5 | Same as 10.5 |
| 10.7 FDD RSTD Measurement Accuracy for 3DL Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig PRS Ês3 / Noc3 ±0.3 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig PRS Ês4 / Noc3 ±0.3 dB averaged over BWConfig Ês4 / Noc3 ±0.3 dB averaged over BWConfig Cell Timing Difference (Intra-freq) = ± 1 Ts Cell Timing Difference (Inter-freq) = ± 2 Ts | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 PRS Ês3 / Noc3 and Ês3 / Noc3 are the ratios of cell 3 signal / AWGN for frequency 3 PRS Ês4 / Noc3 and Ês4 / Noc3 are the ratios of cell 4 signal / AWGN for frequency 3 |
| 10.8 TDD RSTD Measurement Accuracy for 3DL Carrier Aggregation | Same as 10.7 | Same as 10.7 |
| In addition, the following Test System uncertainties and related constraints apply. Any additional constraints are defined in the specific tests. |  |  |
| AWGN Bandwidth |  | ≥ 1.08 MHz, 2.7 MHz, 4.5 MHz, 9 MHz, 13.5 MHz, 18 MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty |  | Test-specific |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  | ±2 dB |
| AWGN peak to average ratio |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty |  | Test-specific |
| Fading profile power uncertainty |  | ±0.5 dB |
| Fading profile delay uncertainty, relative to frame timing |  | ±5 ns (excludes absolute errors related to baseband timing) |

### C.1.4	MBS Minimum Performance requirements

Table C.1.4-1: Maximum Test System Uncertainty for MBS Minimum Performance tests

| Clause | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |
| --- | --- | --- | --- |
| 11.1, 11.1A MBS Measurement Reporting Delay | Beacon power level | ±2 dB |  |
|  | Response time | ±300 ms |  |
| 11.2, 11.2A MBS Sensitivity Measurement Accuracy | Beacon power level | ±2 dB |  |
|  | Code phase delay difference | ±5 ns | Code phase delay difference error value of +/- 5ns, being derived from 10% of the most stringent code phase delay measurement accuracy requirement |
| 11.3, 11.3A MBS Nominal Measurement Accuracy | Beacon power level | ±2 dB |  |
|  | Code phase delay | ±5 ns | Code phase delay error as above |
| 11.4, 11.4A MBS Dynamic Range Measurement Accuracy | Beacon power level | ±2 dB |  |
|  | Code phase delay | ±5 ns | Code phase delay error as above |
| 11.5, 11.5A MBS Measurement Accuracy in Multipath | Beacon power level | ±2 dB |  |
|  | Code phase delay | ±5 ns | Code phase delay error as above |
| Note: Code phase delay is equal to the propagation delay from the (simulated) beacon transmitter to the UE receive antenna based on the propagation distance in the test case. |  |  |  |

### C.1.5	WLAN and BLE measurement requirements

Table C.1.5-1: Maximum Test System Uncertainty for WLAN and BLE measurement tests

| Clause | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |
| --- | --- | --- | --- |
| 12.1.1 WLAN AP Identification and reporting delay under nominal conditions | Response time | ±300 ms |  |
| 12.1.2 WLAN AP Identification and reporting delay under dynamic range conditions | Response time | ±300 ms |  |
|  | AP Power Level Difference | ±1 dB |  |
| 12.2.1 Bluetooth identification | Response time | ±300 ms |  |

Note: AP Power Level Difference is the difference between the WLAN Received Power Level from the high power WLAN AP with respect to the low power WLAN APs.

### C.1.6	NR PRS-based measurement requirements

Table C.1.6-1: Maximum Test System Uncertainty for NR PRS-based Measurement Requirements

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig PRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = FFS | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Same as 14.2.1 |  |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig PRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = FFS | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Same as 14.2.3 |  |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = FFS | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = FFS | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 14.3.1 |  |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Same as 14.3.2 |  |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR1 SA | Same as 15.2.1 | Same as 15.2.1 |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA | Same as 15.2.3 | Same as 15.2.3 |

## C.2	Test Parameter Relaxations (This clause is informative)

The Test Parameter Relaxations defined in this clause have been used to relax the Conformance requirement to derive the Test Requirements.

The Test Parameter Relaxations are derived from Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the Test Parameter Relaxations may sometimes be set to zero.

The Test Parameter Relaxations should not be modified for any reason e.g. to take account of commonly known test system errors (such as mismatch, cable loss, etc.).

### C.2.1	A-GNSS Minimum Performance requirements

Table C.2.1: Test Parameter Relaxations for A-GNSS Minimum Performance tests

| Clause | Test Parameter Relaxation |  |
| --- | --- | --- |
| 5.2.1, 6.2.1, 7.1.1, 13.2.1 Sensitivity Coarse Time Assistance | Coarse Time Assistance | 200 ms |
|  | Absolute GNSS signal level | 1 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.2.2, 6.2.2, 7.1.2, 13.2.2 Sensitivity Fine Time Assistance | Coarse Time Assistance | 200 ms |
|  | Fine Time Assistance | 1 us |
|  | Absolute GNSS signal level | 1 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.3, 6.3, 7.2, 13.3 Nominal Accuracy | Coarse Time Assistance | 200 ms |
|  | Absolute GNSS signal level | 0 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.4, 6.4, 7.3, 13.4 Dynamic Range | Coarse Time Assistance | 200 ms |
|  | Absolute GNSS signal level | 1 dB |
|  | Relative GNSS signal level | 0.2 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.5, 6.5, 7.4, 13.5 Multi-path scenario | Coarse Time Assistance | 200 ms |
|  | Absolute GNSS signal level | 0 dB |
|  | Relative GNSS signal level | 0.2 dB |
|  | Position error | 1.3 m |
|  | Response time | 300 ms |
| 5.6, 6.6, 7.5, 7.5A, 13.6, 13.7 Moving scenario and periodic update | Absolute GNSS signal level | 0 dB |
|  | Position error | 1.3 m |
|  | Differential Response Time | 100 ms |

### C.2.2	ECID and OTDOA Measurement requirements

Table C.2.2: Test Parameter Relaxations for ECID and OTDOA Measurement requirements

| Clause |  | Test Parameter Relaxation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 8.1.1 E-UTRAN FDD UE Rx – Tx time difference case (Rel-9 to Rel-11) |  | Parameters Noc: -98dBm/15kHz Ês / Noc: -3.00dB RxTx time difference value: |  | Test Tolerance 0dB +0.3dB ±3Ts |  |
| 8.1.1A E-UTRAN FDD UE Rx – Tx time difference case (Rel-12 onwards) |  | Same as 8.1.1 |  | Same as 8.1.1 |  |
| 8.1.1B E-UTRAN FDD UE Rx – Tx time difference case for UE Category 1bis |  | Same as 8.1.1 |  | Same as 8.1.1 |  |
| 8.1.2 E-UTRAN TDD UE Rx – Tx time difference case (Rel-9 to Rel-11) |  | Same as 8.1.1 |  | Same as 8.1.1 |  |
| 8.1.2A E-UTRAN TDD UE Rx – Tx time difference case (Rel-12 onwards) |  | Same as 8.1.1 |  | Same as 8.1.1 |  |
| 8.1.2B E-UTRAN TDD UE Rx – Tx time difference case for UE Category 1bis |  | Same as 8.1.1 |  | Same as 8.1.1 |  |
| 8.1.3 E-UTRAN FDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) |  | Parameters Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +1.00dB |  | Test Tolerance 0dB +0.3dB 0dB |  |
| 8.1.4 E-UTRAN TDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) |  | Same as 8.1.3 |  | Same as 8.1.3 |  |
| 8.1.5 E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  | Parameters Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB |  | Test Tolerance 0dB +0.4dB 0dB 0dB |  |
| 8.1.6 E-UTRAN TDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  | Same as 8.1.5 |  | Same as 8.1.5 |  |
| 8.1.7 E-UTRAN FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA |  | Same as 8.1.1 |  | Same as 8.1.1 |  |
| 8.1.8 E-UTRAN HD-FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA |  | Same as 8.1.1 |  | Same as 8.1.1 |  |
| 8.1.9 E-UTRAN TDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA |  | Same as 8.1.1 |  | Same as 8.1.1 |  |
| 9.1.1 FDD RSTD Measurement Reporting Delay |  | Response time |  | 300 ms |  |
| 9.1.1A FDD RSTD Measurement Reporting Delay for UE Category 1bis |  | Response time |  | 300 ms |  |
| 9.1.2 TDD RSTD Measurement Reporting Delay |  | Response time |  | 300 ms |  |
| 9.1.2A TDD RSTD Measurement Reporting Delay for UE Category 1bis |  | Response time |  | 300 ms |  |
| 9.1.3 FDD RSTD Measurement Accuracy |  | For Test 2 and Test 4: PRS Ês1 / Noc averaged over BWConfig PRS Ês2 / Noc averaged over BWConfig For all tests: Cell Timing Difference |  | +0.3 dB +0.3 dB ± 1 Ts |  |
| 9.1.3A FDD RSTD Measurement Accuracy for UE Category 1bis |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.1.4 TDD RSTD Measurement Accuracy |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.1.4A TDD RSTD Measurement Accuracy for UE Category 1bis |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.2.1 FDD-FDD inter-frequency RSTD measurement reporting delay |  | Response time |  | 300 ms |  |
| 9.2.1A FDD-FDD inter-frequency RSTD measurement reporting delay for UE Category 1bis |  | Response time |  | 300 ms |  |
| 9.2.2 TDD-TDD inter-frequency RSTD measurement reporting delay |  | Response time |  | 300 ms |  |
| 9.2.2A TDD-TDD inter-frequency RSTD measurement reporting delay for UE Category 1bis |  | Response time |  | 300 ms |  |
| 9.2.4 FDD-FDD inter frequency RSTD Accuracy |  | PRS Ês1 / Noc1 averaged over BWConfig PRS Ês2 / Noc2 averaged over BWConfig Cell Timing Difference |  | +0.3 dB +0.3 dB ± 2 Ts |  |
| 9.2.4A FDD-FDD inter frequency RSTD Accuracy for UE Category 1bis |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.2.5 TDD-TDD inter frequency RSTD Accuracy |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.2.5A TDD-TDD inter frequency RSTD Accuracy for UE Category 1bis |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.3.1.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.1.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.2.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.2.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.3.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.3.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.4.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.4.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.5.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.5.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.6.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.6.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Same as 9.1.1 |  | Same as 9.1.1 |  |
| 9.3.7.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.7.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.8.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.8.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.9.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.9.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.10.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.10.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.11.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.11.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.12.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.12.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.13 E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.14 E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.15 E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.16 E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.17 E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.3.18 E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.4.1.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.1.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.2.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.2.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.3.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.3.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.4.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.4.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.5.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.5.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.6.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.6.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Same as 9.2.1 |  | Same as 9.2.1 |  |
| 9.4.7.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.7.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.8.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.8.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.9.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.9.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.10.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.10.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.11.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.11.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.12.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.12.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.13 E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.14 E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.15 E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.16 E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.17 E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.4.18 E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.5.1 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage |  | PRS Ês1 / Noc averaged over BWConfig PRS Ês2 / Noc averaged over BWConfig Cell Timing Difference |  | +0.3 dB +0.3 dB ± 1 Ts |  |
| 9.5.2 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage |  | Same as 9.5.1 |  | Same as 9.5.1 |  |
| 9.5.3 HD-FDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage |  | Response time |  | 300 ms |  |
| 9.6.1 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage |  | PRS Ês1 / Noc1 averaged over BWConfig PRS Ês2 / Noc2 averaged over BWConfig Cell Timing Difference |  | +0.3 dB +0.3 dB ± 2 Ts |  |
| 9.6.2 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage |  | Same as 9.6.1 |  | Same as 9.6.1 |  |
| 9.6.3 HD-FDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage |  | Response time |  | 300 ms |  |
| 9.7.1 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage |  | PRS Ês1 / Noc averaged over BWConfig PRS Ês2 / Noc averaged over BWConfig Cell Timing Difference |  | +0.3 dB +0.3 dB ± 1 Ts |  |
| 9.7.2 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage |  | Same as 9.7.1 |  | Same as 9.7.1 |  |
| 9.7.3 TDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage |  | Response time |  | 300 ms |  |
| 9.8.1 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage |  | PRS Ês1 / Noc1 averaged over BWConfig PRS Ês2 / Noc2 averaged over BWConfig Cell Timing Difference |  | +0.3 dB +0.3 dB ± 2 Ts |  |
| 9.8.2 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage |  | Same as 9.8.1 |  | Same as 9.8.1 |  |
| 9.8.3 TDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage |  | Response time |  | 300 ms |  |
| 10.1 FDD RSTD Measurement Reporting Delay for Carrier Aggregation |  | Response time |  | 300 ms |  |
| 10.1A FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz |  | Response time |  | 300 ms |  |
| 10.1B FDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth |  | Response time |  | 300 ms |  |
| 10.1C FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth |  | Response time |  | 300 ms |  |
| 10.2 TDD RSTD Measurement Reporting Delay for Carrier Aggregation |  | Response time |  | 300 ms |  |
| 10.2A TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz |  | Response time |  | 300 ms |  |
| 10.2B TDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth |  | Response time |  | 300 ms |  |
| 10.2C TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth |  | Response time |  | 300 ms |  |
| 10.2D TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz +10 MHz Bandwidth |  | Response time |  | 300 ms |  |
| 10.3 FDD RSTD Measurement Accuracy for Carrier Aggregation |  | PRS Ês2 / Noc2 averaged over BWConfig PRS Ês3 / Noc2 averaged over BWConfig Cell Timing Difference |  | +0.3 dB +0.3 dB ± 1 Ts |  |
| 10.3A FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-10 and Rel-11) |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.3A_1 FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-12 onwards) |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.3B FDD RSTD Measurement Accuracy for Carrier Aggregation for 5MHz+5MHz Bandwidth |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.3C FDD RSTD Measurement Accuracy for Carrier Aggregation for 10MHz+5MHz Bandwidth |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.4 TDD RSTD Measurement Accuracy for Carrier Aggregation |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.4A TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-10 and Rel-11) |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.4A_1 TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-12 onwards) |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.4B TDD RSTD Measurement Accuracy for Carrier Aggregation for 5MHz+5MHz Bandwidth |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.4C TDD RSTD Measurement Accuracy for Carrier Aggregation for 10MHz+5MHz Bandwidth |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.4D TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz+10 MHz Bandwidth |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.5 FDD 3 DL CA RSTD Measurement Reporting Delay |  | Response time |  | 300 ms |  |
| 10.6 TDD 3 DL CA RSTD Measurement Reporting Delay |  | Response time |  | 300 ms |  |
| 10.7 FDD RSTD Measurement Accuracy for 3DL Carrier Aggregation |  | PRS Ês3 / Noc3 averaged over BWConfig PRS Ês4 / Noc3 averaged over BWConfig Cell Timing Difference (Intra-band) Cell Timing Difference (Inter-band) |  | +0.3 dB +0.3 dB ± 1 Ts ± 2 Ts |  |
| 10.8 TDD RSTD Measurement Accuracy for 3DL Carrier Aggregation |  | Same as 10.7 |  | Same as 10.7 |  |

### C.2.3	MBS Minimum Performance requirements

Table C.2.3-1: Test Parameter Relaxations for MBS Minimum Performance tests

| Clause | Test Parameter Relaxation |  |
| --- | --- | --- |
| 11.1,11.1A MBS Measurement Reporting Delay | Beacon power level | 0 dB (no relaxation) |
|  | Response time | 300 ms |
| 11.2,11.2A MBS Sensitivity Measurement Accuracy | Beacon power level | 2 dB |
|  | Code phase difference | 5 ns |
| 11.3,11.3A MBS Nominal Measurement Accuracy | Beacon power level | 0 dB (no relaxation) |
|  | Code phase difference | 5 ns |
| 11.4,11.4A MBS Dynamic Range Measurement Accuracy | Beacon power level | 2 dB |
|  | Code phase difference | 5 ns |
| 11.5,11.5A MBS Measurement Accuracy in Multipath | Beacon power level | 0 dB (no relaxation) |
|  | Code phase difference | 5 ns |

### C.2.4	WLAN and BLE measurement requirements

Table C.2.4-1: Test Parameter Relaxations for WLAN and BLE measurement tests

| Clause | Test Parameter Relaxation |  |
| --- | --- | --- |
| 12.1.1 WLAN AP Identification and reporting delay under nominal conditions | Response time | 300 ms |
| 12.1.2 WLAN AP Identification and reporting delay under dynamic range conditions | Response time | 300 ms |
|  | Low Power WLAN APs Received Power Level | 1 dB |
| 12.2.1 Bluetooth identification | Response time | 300 ms |

### C.2.5	NR PRS-based Measurement requirements

Table C.2.5-1: Test Parameter Relaxations for NR PRS-based Measurement requirements

| Clause | Test Parameter Relaxation |  |
| --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Response time | FFS |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Response time | FFS |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Response time | FFS |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Response time | FFS |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | FFS | FFS |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | FFS | FFS |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 14.3.1 | Same as 14.3.1 |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Same as 14.3.2 | Same as 14.3.2 |

Table C.4.5: Derivation of Test Requirements for UE Rx–Tx time difference Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Response Time = 11s | 300ms | Time + TPR: 11.3s |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR1 SA | Response Time = 20s | 300ms | Time + TPR: 20.3s |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Response Time = 42s | 300ms | Time + TPR: 42.3s |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA | Response Time = 82s | 300ms | Time + TPR: 62.3s |

## C.3	Interpretation of measurement results

The measurement results returned by the Test System are compared - without any modification - against the Test Requirements as defined by the shared risk principle.

The Shared Risk principle is defined in TR 102 273-1-2 [7], clause 6.5.

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause C.1.

If the Test System for a test is known to have a measurement uncertainty greater than that specified in clause C.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows.

Any additional uncertainty in the Test System over and above that specified in clause C.1 shall be used to tighten the Test Requirement - making the test harder to pass. (This may require modification of stimulus signals). This procedure will ensure that a Test System not compliant with clause C.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause C.1 had been used.

## C.4	Derivation of Test Requirements (This clause is informative)

The Test Requirements have been calculated by relaxing the Conformance requirement of the core specification using the Test Parameter Relaxations defined in clause C.2. When the Test Parameter Relaxation is zero, the Test Requirement will be the same as the Conformance requirement. When the Test Parameter Relaxation is non-zero, the Test Requirements will differ from the Conformance requirement, and the formula used for this relaxation is given in table C.4.1, C.4.2, C.4.3 and C.4.4.

Table C.4.1: Derivation of Test Requirements for A-GNSS Minimum Performance tests

| Test | Conformance requirement in TS 25.171 [31] or TS 25.172 [19] or TS 36.171 [3] or 38.171 [43] |  | Test Parameter Relaxation (TPR) | Test Requirement |
| --- | --- | --- | --- | --- |
| 5.2.1, 6.2.1, 7.1.1, 13.2.1 Sensitivity Coarse Time Assistance | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Absolute GPS L1 C/A signal level (test 5.2.1, 7.1.1, 13.2.1, sub-test 1) | -142, -147 dBm | 1 dB | Level + TPR: -141, -146 dBm |
|  | Absolute GNSS signal level (Galileo) | -142, -147 dBm | 1 dB | Level + TPR: -141, -146 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.2.1, 7.1.1, 13.2.1, sub-tests 4, 5, 8, and 10 to 13) | -142, -147 dBm | 1 dB | Level + TPR: -141, -146 dBm |
|  | Absolute GNSS signal level (GLONASS) | -142, -147 dBm | 1 dB | Level + TPR: -141, -146 dBm |
|  | Absolute GNSS signal level (BDS) | -136, -145 dBm | 1 dB | Level + TPR: -135, -144 dBm |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.2.2, 6.2.2, 7.1.2, 13.2.2 Sensitivity Fine Time Assistance | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Fine Time Assistance | 10 us | 1 us | UL-TPR, LL+TPR: 9 us |
|  | Absolute GPS L1 C/A signal level (test 5.2.2, 7.1.2, 13.2.2, sub-test 1) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Absolute GNSS signal level (Galileo) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.2.2, 7.1.2, 13.2.2, sub-tests 4, 5, 8 and 10 to 13) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Absolute GNSS signal level (GLONASS) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Absolute GNSS signal level (BDS) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.3, 6.3, 7.2, 13.3 Nominal Accuracy | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Absolute GPS L1 C/A signal level (test 5.3, 7.2, 13.3, sub-test 1) | -130 dBm | 0 dB | Formulas: Level + TPR: -130 dBm |
|  | Absolute GNSS signal level (Galileo) | -127 dBm | 0 dB | Level + TPR: -127 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.3, 7.2, 13.3, sub-tests 4, 5, 8 and 10 to 13) | -128.5 dBm | 0 dB | Level + TPR: -128.5 dBm |
|  | Absolute GNSS signal level (GLONASS) | -131 dBm | 0 dB | Level + TPR: -131 dBm |
|  | Absolute GNSS signal level (QZSS) | -128.5 dBm | 0 dB | Level + TPR: -128.5 dBm |
|  | Absolute GNSS signal level (SBAS) | -131 dBm | 0 dB | Level + TPR: -131 dBm |
|  | Absolute GNSS signal level (BDS) | -133 dBm | 0 dB | Level + TPR: -133 dBm |
|  | Position error | 30 m | 1.3 m | Error +TPR: 31.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.4, 6.4, 7.3, 13.4 Dynamic Range | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Absolute GPS L1 C/A signal level (test 5.4, 7.3, 13.4, sub-test 1) | -129 to -147 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Absolute GNSS signal level (Galileo) | -127.5 to -147 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.4, 7.3, 13.4, sub-tests 4, 5, 8 and 10 to 13) | -129 to -147 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Absolute GNSS signal level (GLONASS) | -131.5 to -147 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Absolute GNSS signal level (BDS) | -133.5 to -145 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Relative GPS L1 C/A signal level (test 5.4, 7.3, 13.4, sub-test 1) | 18 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -128.2 dBm |
|  | Relative GNSS signal level (Galileo) | 19.5 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -126.7 dBm |
|  | Relative GNSS signal level (GPS) (test 6.4, 7.3, 13.4, sub-tests 4, 5, 8 and 10 to 13) | 18 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -128.2 dBm |
|  | Relative GNSS signal level (GLONASS) | 15.5 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -130.7 dBm |
|  | Relative GNSS signal level (BDS) | 11.5 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -132.7 dBm |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.5, 6.5, 7.4, 13.5 Multi-path scenario | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Absolute GPS L1 C/A signal level (test 5.5, 7.4, 13.5, sub-test 1) | -130 dBm | 0 dB | Formulas: Level + TPR: -130 dBm |
|  | Absolute GNSS signal level (Galileo) | -127 dBm | 0 dB | Level + TPR: -127 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.5, 7.4, 13.5, sub-tests 4, 5, 8 and 10 to 13) | -128.5 dBm | 0 dB | Level + TPR: -128.5 dBm |
|  | Absolute GNSS signal level (GLONASS) | -131 dBm | 0 dB | Level + TPR: -131 dBm |
|  | Absolute GNSS signal level (BDS) | -133 dBm | 0 dB | Level + TPR: -133 dBm |
|  | Relative GPS L1 C/A signal level (test 5.5, 7.4, 13.5, sub-test 1) -142, -147 dBm | 6 dB | 0.2 dB | Relative level + TPR: relative level + 0.2dB: 6.2 dB |
|  | Relative GNSS signal level (all GNSSs) (test 6.5, 7.4, 13.5, sub-tests 4, 5, 8 and 10 to 13) | Y dB where “Y” is given in Table 4.2.2 | 0.2 dB | Relative level + TPR: relative level + 0.2dB: Y + 0.2 dB |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.6, 6.6, 7.5, 7.5A., 13.6, 13.7 Moving scenario and periodic update | Absolute GPS L1 C/A Signal level (test 5.6, 7.5, 7.5A., 13.6, 13.7, sub-test 1) | -130 dBm | 0 dB | Formulas: Level + TPR: -130 dBm |
|  | Absolute GNSS signal level (Galileo) | -127 dBm | 0 dB | Level + TPR: -127 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.6, 7.5, 7.5A., 13.6, 13.7, sub-tests 4, 5, 8 and 10 to 13) | -128.5 dBm | 0 dB | Level + TPR: -128.5 dBm |
|  | Absolute GNSS signal level (GLONASS) | -131 dBm | 0 dB | Level + TPR: -131 dBm |
|  | Absolute GNSS signal level (BDS) | -133 dBm | 0 dB | Level + TPR: -133 dBm |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Differential response time | 2s +/- 20 % | 100 ms | Time +TPR: 1.5 s and 2.5 s |

Table C.4.2: Derivation of Test Requirements for ECID and OTDOA Measurement tests

| Test |  | Minimum Requirement in TS 36.133 |  | Test Parameter Relaxation (TPR) |  | Test Requirement in TS 36.571-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.1.1 E-UTRAN FDD UE Rx - Tx time difference case (Rel-9 to Rel-11) |  | Test 1: Noc: -98dBm/15kHz Ês / Noc: -3.0dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês / Noc: -3.0dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  | Test 1: 0dB 0.3dB Via mapping Test 2: 0dB 0.3dB Via mapping |  | Test 2: Noc: -98dBm/15kHz Ês / Noc: -2.7.0dB (Measured value from step 7 - 23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  |
| 8.1.1A E-UTRAN FDD UE Rx – Tx time difference case (Rel-12 onwards) |  | Same as 8.1.1 |  | Same as 8.1.1 |  | Test 1: Noc: -98dBm/15kHz Ês / Noc: -2.7.0dB (Measured value from step 7 - 23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB Measured value from step 7 -10) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +10) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  |
| 8.1.1B E-UTRAN FDD UE Rx – Tx time difference case for UE Category 1bis |  | Same as 8.1.1 |  | Same as 8.1.1 |  | Same as 8.1.1A |  |
| 8.1.2 E-UTRAN TDD UE Rx - Tx time difference case (Rel-9 to Rel-11) |  | Same as 8.1.1 except use Table 4.6.3-2 |  | Same as 8.1.1 |  | Same as 8.1.1 except use Table 4.6.3-2 |  |
| 8.1.2A E-UTRAN TDD UE Rx – Tx time difference case (Rel-12 onwards) |  | Same as 8.1.1 except use Table 4.6.3-2 |  | Same as 8.1.1 |  | Same as 8.1.1A except use Table 4.6.3-2 |  |
| 8.1.2B E-UTRAN TDD UE Rx – Tx time difference case for UE Category 1bis |  | Same as 8.1.1 except use Table 4.6.3-2 |  | Same as 8.1.1 |  | Same as 8.1.1A except use Table 4.6.3-2 |  |
| 8.1.3 E-UTRAN FDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) |  | Test 1: Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +1.00dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +1.00dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  | Test 1: 0dB 0.3dB 0dB Via mapping Test 2: 0dB 0.3dB 0dB Via mapping |  | Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -2.70dB Ês2 / Noc: +1.00dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -2.70dB Ês2 / Noc: +1.00dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  |
| 8.1.4 E-UTRAN TDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) |  | Same as 8.1.3 except use Table 4.6.3-2 |  | Same as 8.1.3 |  | Same as 8.1.3 except use Table 4.6.3-2 |  |
| 8.1.5 E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  | Test 1: Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  | Test 1: 0dB 0.4dB 0dB 0dB Via mapping Test 2: 0dB 0.4dB 0dB 0dB Via mapping |  | Test 1: Noc: -98dBm/15kHz Ês1 / Noc: -2.60dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -2.60dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  |
| 8.1.6 E-UTRAN TDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  | Same as 8.1.5 except use Table 4.6.3-2 |  | Same as 8.1.5 |  | Same as 8.1.5 except use Table 4.6.3-2 |  |
| 8.1.7 E-UTRAN FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA |  | Noc: -98dBm/15kHz Ês / Noc: -3.0dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  | 0dB 0.3dB Via mapping |  | Noc: -98dBm/15kHz Ês / Noc: -2.7.0dB Category M1: (Measured value from step 7 - 23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Category M2: (Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |  |
| 8.1.8 E-UTRAN HD-FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA |  | Same as 8.1.7 |  | Same as 8.1.7 |  | Same as 8.1.7 |  |
| 8.1.9 E-UTRAN TDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA |  | Same as 8.1.7 except use Table 4.6.3-2 |  | Same as 8.1.7 |  | Same as 8.1.7 except use Table 4.6.3-2 |  |
| 9.1.1 FDD RSTD Measurement Reporting Delay |  | Response Time = 3 s |  | 300 ms |  | Time + TPR: 3.3 s |  |
| 9.1.1A FDD RSTD Measurement Reporting Delay for UE Category 1bis |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.1.2 TDD RSTD Measurement Reporting Delay |  | Response Time = 3 s |  | 300 ms |  | Time + TPR: 3.3 s |  |
| 9.1.2A TDD RSTD Measurement Reporting Delay for UE Category 1bis |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.1.3 FDD RSTD Measurement Accuracy |  | For Test 2 and Test 4: PRS Ês1 / Noc = -6dB PRS Ês2 / Noc = -13dB Cell timing difference: Test 1 and 2 = ±15 Ts Test 3 and 4 = ±5 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±16 Ts ±6 Ts |  |
| 9.1.3A FDD RSTD Measurement Accuracy for UE Category 1bis |  | Same as 9.1.3 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.1.4 TDD RSTD Measurement Accuracy |  | Same as 9.1.3 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.1.4A TDD RSTD Measurement Accuracy for UE Category 1bis |  | Same as 9.1.3 |  | Same as 9.1.3 |  | Same as 9.1.3 |  |
| 9.2.1 FDD-FDD inter-frequency RSTD measurement reporting delay |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.2.1A FDD-FDD inter-frequency RSTD measurement reporting delay for UE Category 1bis |  | Response Time = 11 s |  | 300 ms |  | Time + TPR: 11.3 s |  |
| 9.2.2 TDD-TDD inter-frequency RSTD measurement reporting delay |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.2.2A TDD-TDD inter-frequency RSTD measurement reporting delay for UE Category 1bis |  | Response Time = 11 s |  | 300 ms |  | Time + TPR: 11.3 s |  |
| 9.2.4 FDD-FDD inter frequency RSTD Accuracy |  | PRS Ês1 / Noc1 = -6dB PRS Ês2 / Noc2 = -13dB Cell timing difference: Test 1 = ±21 Ts Test 2 = ±9 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±23 Ts ±11 Ts |  |
| 9.2.4A FDD-FDD inter frequency RSTD Accuracy for UE Category 1bis |  | Same as 9.2.4 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.2.5 TDD-TDD inter frequency RSTD Accuracy |  | Same as 9.2.4 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.2.5A TDD-TDD inter frequency RSTD Accuracy for UE Category 1bis |  | Same as 9.2.4 |  | Same as 9.2.4 |  | Same as 9.2.4 |  |
| 9.3.1.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.3.1.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.3.2.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.3.2.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.3.3.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.3.3.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.3.4.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Response Time = 13 s |  | 300 ms |  | Time + TPR: 13.3 s |  |
| 9.3.4.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Test 1: Response Time 13 s Test 2: Response Time 6 s |  | 300 ms |  | Test 1: Time + TPR: 13.3 s Test 2: Time + TPR: 6.3 s |  |
| 9.3.5.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Response Time = 13 s |  | 300 ms |  | Time + TPR: 13.3 s |  |
| 9.3.5.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Test 1: Response Time 13 s Test 2: Response Time 6 s |  | 300 ms |  | Test 1: Time + TPR: 13.3 s Test 2: Time + TPR: 6.3 s |  |
| 9.3.6.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Response Time = 13 s |  | 300 ms |  | Time + TPR: 13.3 s |  |
| 9.3.6.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Test 1: Response Time 13 s Test 2: Response Time 6 s |  | 300 ms |  | Test 1: Time + TPR: 13.3 s Test 2: Time + TPR: 6.3 s |  |
| 9.3.7.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | PRS Ês1 / Noc1 = -6 dB PRS Ês2 / Noc2 = -13 dB Cell timing difference = ±15 Ts |  | +0.3 dB +0.3 dB ±1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±16 Ts |  |
| 9.3.7.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | PRS Ês1 / Noc1 = -6 dB PRS Ês2 / Noc2 = -13 dB Cell timing difference= ±6 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±7 Ts |  |
| 9.3.8.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.3.7.1 |  | Same as 9.3.7.1 |  | Same as 9.3.7.1 |  |
| 9.3.8.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.3.7.2 |  | Same as 9.3.7.2 |  | Same as 9.3.7.2 |  |
| 9.3.9.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.3.7.1 |  | Same as 9.3.7.1 |  | Same as 9.3.7.1 |  |
| 9.3.9.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.3.7.2 |  | Same as 9.3.7.2 |  | Same as 9.3.7.2 |  |
| 9.3.10.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | PRS Ês1 / Noc1 = -15dB PRS Ês2 / Noc2 = -15dB Cell timing difference= ±15 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -14.7 dB Level + TPR, -14.7 dB Timing difference ± TPR, ±16 Ts |  |
| 9.3.10.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | PRS Ês1 / Noc1 = -15 dB PRS Ês2 / Noc2 = -15 dB Cell timing difference= ±6 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -14.7 dB Level + TPR, -14.7 dB Timing difference ± TPR, ±7 Ts |  |
| 9.3.11.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.3.10.1 |  | Same as 9.3.10.1 |  | Same as 9.3.10.1 |  |
| 9.3.11.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.3.10.2 |  | Same as 9.3.10.2 |  | Same as 9.3.10.2 |  |
| 9.3.12.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.3.10.1 |  | Same as 9.3.10.1 |  | Same as 9.3.10.1 |  |
| 9.3.12.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.3.10.2 |  | Same as 9.3.10.2 |  | Same as 9.3.10.2 |  |
| 9.3.13 E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Response Time = 3 s |  | 300 ms |  | Time + TPR: 3.3 s |  |
| 9.3.14 E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Response Time = 3 s |  | 300 ms |  | Time + TPR: 3.3 s |  |
| 9.3.15 E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Response Time = 3 s |  | 300 ms |  | Time + TPR: 3.3 s |  |
| 9.3.16 E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Response Time = 3 s |  | 300 ms |  | Time + TPR: 3.3 s |  |
| 9.3.17 E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Response Time = 3 s |  | 300 ms |  | Time + TPR: 3.3 s |  |
| 9.3.18 E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.4.1.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Response Time = 16 s |  | 300 ms |  | Time + TPR: 16.3 s |  |
| 9.4.1.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Test 1: Response Time 16 s Test 2: Response Time 11 s |  | 300 ms |  | Test 1: Time + TPR: 16.3 s Test 2: Time + TPR: 11.3 s |  |
| 9.4.2.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Response Time = 16 s |  | 300 ms |  | Time + TPR: 16.3 s |  |
| 9.4.2.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Test 1: Response Time 16 s Test 2: Response Time 11 s |  | 300 ms |  | Test 1: Time + TPR: 16.3 s Test 2: Time + TPR: 11.3 s |  |
| 9.4.3.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 |  | Response Time = 16 s |  | 300 ms |  | Time + TPR: 16.3 s |  |
| 9.4.3.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 |  | Test 1: Response Time 16 s Test 2: Response Time 11 s |  | 300 ms |  | Test 1: Time + TPR: 16.3 s Test 2: Time + TPR: 11.3 s |  |
| 9.4.4.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Response Time = 42 s |  | 300 ms |  | Time + TPR: 42.3 s |  |
| 9.4.4.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Test 1: Response Time 42 s Test 2: Response Time 11 s |  | 300 ms |  | Test 1: Time + TPR: 42.3 s Test 2: Time + TPR: 11.3 s |  |
| 9.4.5.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Response Time = 42 s |  | 300 ms |  | Time + TPR: 42.3 s |  |
| 9.4.5.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Test 1: Response Time 42 s Test 2: Response Time 11 s |  | 300 ms |  | Test 1: Time + TPR: 42.3 s Test 2: Time + TPR: 11.3 s |  |
| 9.4.6.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 |  | Response Time = 42 s |  | 300 ms |  | Time + TPR: 42.3 s |  |
| 9.4.6.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 |  | Test 1: Response Time 42 s Test 2: Response Time 11 s |  | 300 ms |  | Test 1: Time + TPR: 42.3 s Test 2: Time + TPR: 11.3 s |  |
| 9.4.7.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | PRS Ês1 / Noc1 = -6 dB PRS Ês2 / Noc2 = -13 dB Cell timing difference= ±21 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±23 Ts |  |
| 9.4.7.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | PRS Ês1 / Noc1 = -6 dB PRS Ês2 / Noc2 = -13 dB Cell timing difference= ±10 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±12 Ts |  |
| 9.4.8.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.4.7.1 |  | Same as 9.4.7.1 |  | Same as 9.4.7.1 |  |
| 9.4.8.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.4.7.2 |  | Same as 9.4.7.2 |  | Same as 9.4.7.2 |  |
| 9.4.9.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 |  | Same as 9.4.7.1 |  | Same as 9.4.7.1 |  | Same as 9.4.7.1 |  |
| 9.4.9.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 |  | Same as 9.4.7.2 |  | Same as 9.4.7.2 |  | Same as 9.4.7.2 |  |
| 9.4.10.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | PRS Ês1 / Noc1 = -15 dB PRS Ês2 / Noc2 = -15 dB Cell timing difference= ±21 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -14.7 dB Level + TPR, -14.7 dB Timing difference ± TPR, ±23 Ts |  |
| 9.4.10.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | PRS Ês1 / Noc1 = -15 dB PRS Ês2 / Noc2 = -[13] dB Cell timing difference= ±10 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -14.7 dB Level + TPR, -[12.7] dB Timing difference ± TPR, ±12 Ts |  |
| 9.4.11.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.4.10.1 |  | Same as 9.4.10.1 |  | Same as 9.4.10.1 |  |
| 9.4.11.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.4.10.2 |  | Same as 9.4.10.2 |  | Same as 9.4.10.2 |  |
| 9.4.12.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 |  | Same as 9.4.10.1 |  | Same as 9.4.10.1 |  | Same as 9.4.10.1 |  |
| 9.4.12.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 |  | Same as 9.4.10.2 |  | Same as 9.4.10.2 |  | Same as 9.4.10.2 |  |
| 9.4.13 E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.4.14 E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.4.15 E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.4.16 E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.4.17 E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Response Time = 6 s |  | 300 ms |  | Time + TPR: 6.3 s |  |
| 9.4.18 E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions |  | Response Time = 11 s |  | 300 ms |  | Time + TPR: 11.3 s |  |
| 9.5.1 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage |  | NPRS Ês1 / Noc = -6dB NPRS Ês2 / Noc = -13dB Cell timing difference= ±20 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±21 Ts |  |
| 9.5.2 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage |  | NPRS Ês1 / Noc = -15dB NPRS Ês2 / Noc = -15dB Cell timing difference= ±32 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -14.7 dB Level + TPR, -14.7 dB Timing difference ± TPR, ±33 Ts |  |
| 9.5.3 HD-FDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage |  | Response Time = 78 s |  | 300 ms |  | Time + TPR: 78.3 s |  |
| 9.6.1 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage |  | NPRS Ês1 / Noc1 = -6dB NPRS Ês2 / Noc2 = -13dB Cell timing difference= ±28 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±30 Ts |  |
| 9.6.2 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage |  | NPRS Ês1 / Noc1 = -15dB NPRS Ês2 / Noc2 = -15dB Cell timing difference= ±40 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -14.7 dB Level + TPR, -14.7 dB Timing difference ± TPR, ±42 Ts |  |
| 9.6.3 HD-FDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage |  | Same as 9.5.3 |  | Same as 9.5.3 |  | Same as 9.5.3 |  |
| 9.7.1 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage |  | NPRS Ês1 / Noc = -6dB NPRS Ês2 / Noc = -13dB Cell timing difference= ±20 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±21 Ts |  |
| 9.7.2 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage |  | NPRS Ês1 / Noc = -15dB NPRS Ês2 / Noc = -15dB Cell timing difference= ±32 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -14.7 dB Level + TPR, -14.7 dB Timing difference ± TPR, ±33 Ts |  |
| 9.7.3 TDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage |  | Response Time = 79 s |  | 300 ms |  | Time + TPR: 79.3 s |  |
| 9.8.1 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage |  | NPRS Ês1 / Noc1 = -6dB NPRS Ês2 / Noc2 = -13dB Cell timing difference= ±28 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±30 Ts |  |
| 9.8.2 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage |  | NPRS Ês1 / Noc1 = -15dB NPRS Ês2 / Noc2 = -15dB Cell timing difference= ±40 Ts |  | +0.3 dB +0.3 dB ± 2 Ts |  | Level + TPR, -14.7 dB Level + TPR, -14.7 dB Timing difference ± TPR, ±42 Ts |  |
| 9.8.3 TDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage |  | Response Time = 90 s |  | 300 ms |  | Time + TPR: 90.3 s |  |
| 10.1 FDD RSTD Measurement Reporting Delay for Carrier Aggregation |  | Test 1: Response Time = 3 s Test 2: Response Time = 6 s |  | 300 ms 300 ms |  | Time + TPR: 3.3 s Time + TPR: 6.3 s |  |
| 10.1A FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.1B FDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.1C FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.2 TDD RSTD Measurement Reporting Delay for Carrier Aggregation |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.2A TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.2B TDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.2C TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.2D TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz +10 MHz Bandwidth |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.3 FDD RSTD Measurement Accuracy for Carrier Aggregation |  | PRS Ês2 / Noc2 = -6dB PRS Ês3 / Noc2 = -13dB Cell timing difference= ±5 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±6 Ts |  |
| 10.3A FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-10 and Rel-11) |  | Same as 10.3 |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.3A_1 FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-12 onwards) |  | PRS Ês2 / Noc2 = -6dB PRS Ês3 / Noc2 = -13dB Cell timing difference= ±4 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±5 Ts |  |
| 10.3B FDD RSTD Measurement Accuracy for Carrier Aggregation for 5 MHz+5 MHz Bandwidth |  | PRS Ês2 / Noc2 = -6dB PRS Ês3 / Noc2 = -13dB Cell timing difference= ±6 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±7 Ts |  |
| 10.3C FDD RSTD Measurement Accuracy for Carrier Aggregation for 10 MHz+5 MHz Bandwidth |  | Same as 10.3B |  | Same as 10.3B |  | Same as 10.3B |  |
| 10.4 TDD RSTD Measurement Accuracy for Carrier Aggregation |  | Same as 10.3 |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.4A TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-10 and Rel-11) |  | Same as 10.3 |  | Same as 10.3 |  | Same as 10.4 |  |
| 10.4A_1 TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-12 onwards) |  | PRS Ês2 / Noc2 = -6dB PRS Ês3 / Noc2 = -13dB Cell timing difference= ±4 Ts |  | +0.3 dB +0.3 dB ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, ±5 Ts |  |
| 10.4B TDD RSTD Measurement Accuracy for Carrier Aggregation for 5 MHz+5 MHz Bandwidth |  | Same as 10.3B |  | Same as 10.3B |  | Same as 10.3B |  |
| 10.4C TDD RSTD Measurement Accuracy for Carrier Aggregation for 10 MHz+5 MHz Bandwidth |  | Same as 10.3B |  | Same as 10.3B |  | Same as 10.3B |  |
| 10.4D TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz+10 MHz Bandwidth |  | Same as 10.3 |  | Same as 10.3 |  | Same as 10.3 |  |
| 10.5 FDD 3 DL CA RSTD Measurement Reporting Delay |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.6 TDD 3 DL CA RSTD Measurement Reporting Delay |  | Same as 10.1 |  | Same as 10.1 |  | Same as 10.1 |  |
| 10.7 FDD RSTD Measurement Accuracy for 3DL Carrier Aggregation |  | PRS Ês3 / Noc3 = -6dB PRS Ês4 / Noc3 = -13dB Cell timing difference= Cell 1: ±10 Ts, ±9 Ts, ±8 Ts Cell 2: ±10 Ts, ±9 Ts, ±8 Ts Cell 4: ±6 Ts, ±5 Ts, ±4 Ts |  | +0.3 dB +0.3 dB ± 2 Ts ± 2 Ts ± 1 Ts |  | Level + TPR, -5.7 dB Level + TPR, -12.7 dB Timing difference ± TPR, Cell 1: ±12 Ts, ±11 Ts, ±10 Ts Cell 2: ±12 Ts, ±11 Ts, ±10 Ts Cell 4: ±7 Ts, ±6 Ts, ±5 Ts |  |
| 10.8 TDD RSTD Measurement Accuracy for 3DL Carrier Aggregation |  | Same as 10.7 |  | Same as 10.7 |  | Same as 10.7 |  |

Table C.4.3: Derivation of Test Requirements for MBS Minimum Performance tests

| Test | Conformance requirements in 11.1.3, 11.2.3, 11.3.3, 11.4.3 and 11.5.3 |  | Test Parameter Relaxation (TPR) | Test Requirement |
| --- | --- | --- | --- | --- |
| 11.1, 11.1A MBS Measurement Reporting Delay | Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | Response time | 12 seconds | 300 ms | Time+TPR: 12.3 s |
| 11.2 MBS Sensitivity Measurement Accuracy | Beacon power level | -130 dBm | 2 dB | Level+TPR: -128 dBm |
|  | Code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |
| 11.2A MBS Sensitivity Measurement Accuracy | Beacon power level | -130 dBm | 2 dB | Level+TPR: -128 dBm |
|  | Code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |
|  | Code phase difference | 9.3 10-5 ms | 5 ns | Error+TPR: 9.8 10-5 ms |
| 11.3 MBS Nominal Measurement Accuracy | Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | Code phase difference | 7.1 10-5 ms | 5 ns | Error+TPR: 7.6 10-5 ms |
| 11.3A MBS Nominal Measurement Accuracy | Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | TB1 Code phase difference | 7.1 10-5 ms | 5 ns | Error+TPR: 7.6 10-5 ms |
|  | TB2 Code phase difference | 2.8 10-5 ms | 5 ns | Error+TPR: 3.3 10-5 ms |
| 11.4 MBS Dynamic Range Measurement Accuracy | High Power Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | Code phase difference | 7.1 10-5 ms | 5 ns | Error+TPR: 7.6 10-5 ms |
|  | Low Power Beacon power level | -130 dBm | 2 dB | Level+TPR: -128 dBm |
|  | Code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |
| 11.4A MBS Dynamic Range Measurement Accuracy | High Power Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | TB1 High Power code phase difference | 7.1 10-5 ms | 5 ns | Error+TPR: 7.6 10-5 ms |
|  | TB2 High Power code phase difference | 2.8 10-5 ms | 5 ns | Error+TPR: 3.3 10-5 ms |
|  | Low Power Beacon power level | -130 dBm | 2 dB | Level+TPR: -128 dBm |
|  | TB1 Low Power code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |
|  | TB2 Low Power code phase difference | 9.3 10-5 ms | 5 ns | Error+TPR: 9.8 10-5 ms |
| 11.5, 11.5A MBS Measurement Accuracy in Multipath | Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | Code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |

Table C.4.4: Derivation of Test Requirements for WLAN and BLE measurement tests

| Test | Conformance requirements in 12.1.1, 12.1.2 and 12.2.1 |  | Test Parameter Relaxation (TPR) | Test Requirement |
| --- | --- | --- | --- | --- |
| 12.1.1 WLAN AP Identification and reporting delay under nominal conditions | Response time | 20.85 seconds | 300 ms | Time+TPR: 21.15 s |
| 12.1.2 WLAN AP Identification and reporting delay under dynamic range conditions | Response time | 20.85 seconds | 300 ms | Time+TPR: 21.15 s |
|  | Low Power WLAN APs Received Power Level | WLAN 2.4 GHz band: -74 dBm WLAN 5 GHz band: -79 dBm | 1 dB | Power+TPR: WLAN 2.4 GHz band: -73 dBm WLAN 5 GHz band: -78 dBm |
| 12.2.1 Bluetooth identification | Response time | 10.85 seconds | 300 ms | Time+TPR: 11.15 s |

Table C.4.5: Derivation of Test Requirements for NR RSTD Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Response Time = FFS | FFS | Time + TPR: FFS |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Response Time = FFS | FFS | Time + TPR: FFS |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Response Time = FFS | FFS | Time + TPR: FFS |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Response Time = FFS | FFS | Time + TPR: FFS |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Response Time = FFS | FFS | Time + TPR: FFS |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Response Time = FFS | FFS | Time + TPR: FFS |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Response Time = FFS | FFS | Time + TPR: FFS |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Response Time = FFS | FFS | Time + TPR: FFS |

Annex D (normative):
Rules for statistical testing

## D.1	Test Method

In this clause the terms GNSS and A-GNSS also include the cases where the only satellite system used is GPS unless otherwise stated.

Each test is performed in the following manner:

a)	Setup the required test conditions.

b)	Start each repetition after having applied the message ‘RESET UE POSITIONING STORED INFORMATION’. This ensures that each result is independent from the previous one.

c)	Make the required measurement a repeated number of times. The results, measured, are simplified to:

good result, if the measured result is ≤ limit.

bad result, if the measured result is > limit

For the relevant A-GNSS test cases measure the 2D position and Time to First Fix (TTFF) a repeated number of times. Measure the 2D position and Time to First Fix (if applicable) repeated times. Start each repetition after having applied the message ‘RESET UE POSITIONING STORED INFORMATION’. This ensures that each result is independent from the previous one. The results, measured, are simplified to:

good result, if the 2D position and TTFF are ≤ limit.

bad result, if the 2D position or TTFF or both are > limit

d)	Record the number of results (ns) and the number of bad results (ne)

e)	Stop the test at a pass or a fail event.

f)	Once the test is stopped, decide according to the pass fail decision rules (D.4.2)

## D.2	Error Ratio (ER)

The Error Ratio (ER) is defined as the ratio of bad results (ne) to all results (ns).

(1-ER is the success ratio)

## D.3	Test Design

A statistical test is characterised by:

Test-time, Selectivity and Confidence level

### D.3.1	Confidence level

The outcome of a statistical test is a decision. This decision may be correct or in-correct. The Confidence Level CL describes the probability that the decision is a correct one. The complement is the wrong decision probability (risk) D = 1-CL

### D.3.2	Introduction: Supplier Risk versus Customer Risk

There are two targets of decision:

a)	A measurement on the pass-limit shows, that the DUT has the specified quality or is better with probability CL (CL e.g.95%) This shall lead to a “pass decision”

The pass-limit is on the good side of the specified DUT-quality. A more stringent CL (CL e.g.99%) shifts the pass-limit further into the good direction. Given that the quality of the DUTs is distributed, a greater CL passes less and better DUTs.

A measurement on the bad side of the pass-limit is simply “not pass” (undecided)

aa)	Complementary:

A measurement on the fail-limit shows, that the DUT is worse than the specified quality with probability CL.

The fail-limit is on the bad side of the specified DUT-quality. A more stringent CL shifts the fail-limit further into the bad direction. Given that the quality of the DUTs is distributed, a greater CL fails less and worse DUTs.

A measurement on the good side of the fail-limit is simply “not fail”.

b)	A DUT, known to have the specified quality, shall be measured and decided pass with probability CL. This leads to the pass limit.

For CL e.g. 95%, the pass limit is on the bad side of the specified DUT-quality. CL e.g.99% shifts the pass-limit further into the bad direction. Given that the DUT-quality is distributed, a greater CL passes more and worse DUTs.

bb)	A DUT, known to be an (ε0) beyond the specified quality, shall be measured and decided fail with probability CL.

For CL e.g.95%, the fail limit is on the good side of the specified DUT-quality.

Note the different sense for CL in (a), (aa) versus (b), (bb).

NOTE: For constant CL in all 4 bullets, (a) is equivalent to (bb) and (aa) is equivalent to (b).

### D.3.3	Supplier Risk versus Customer Risk

The table below summarizes the different targets of decision.

Table D.3.3: Equivalent statements

|  | Equivalent statements, using different cause-to-effect-directions, and assuming CL = constant >0.5 |  |
| --- | --- | --- |
| cause-to-effect-directions | Known measurement result estimation of the DUT’s quality | Known DUT’s quality estimation of the measurement’s outcome |
| Supplier Risk | A measurement on the pass-limit shows, that the DUT has the specified quality or is better (a) | A DUT, known to have an (ε0) beyond the specified DUT-quality, shall be measured and decided fail (bb) |
| Customer Risk | A measurement on the fail-limit shall shows, that the DUT is worse than the specified quality (aa) | A DUT, known to have the specified quality, shall be measured and decided pass (b) |

NOTE:	The bold text shows the obvious interpretation of Supplier Risk and Customer Risk.
The same statements can be based on other DUT-quality-definitions.

### D.3.4	Introduction: Standard test versus early decision concept

In standard statistical tests, a certain number of results (ns) is predefined in advance of the test. After ns results the number of bad results (ne) is counted and the error ratio (ER) is calculated as ne/ns.

Applying statistical theory, a decision limit can be designed, against which the calculated ER is compared to derive the decision. Such a limit is one decision point and is characterised by:

-	D: the wrong decision probability (a predefined parameter)

-	ns: the number of results (a fixed predefined parameter)

-	ne: the number of bad results (the limit based on just ns)

In the formula for the limit, D and ns are parameters and ne is the variable. In the standard test ns and D are constant. The property of such a test is: It discriminates between two states only, depending on the test design:

-	pass (with CL) / undecided (undecided in the sense: finally undecided)

-	fail (with CL) / undecided (undecided in the sense: finally undecided)

-	pass (with CL) / fail (with CL) (however against two limits).

In contrast to the standard statistical tests, the early decision concept predefines a set of (ne, ns) co-ordinates, representing the limit-curve for decision. After each result a preliminary ER is calculated and compared against the limit-curve. After each result one may make the decision or not (undecided for later decision). The parameters and variables in the limit-curve for the early decision concept have a similar but not equal meaning:

-	D: the wrong decision probability (a predefined parameter)

-	ns: the number of results (a variable parameter)

-	ne: the number of bad results (the limit. It varies together with ns)

To avoid a “final undecided” in the standard test, a second limit must be introduced and the single decision co-ordinate (ne, ns) needs a high ne, leading to a fixed (high) test time. In the early decision concept, having the same selectivity and the same confidence level an “undecided” does not need to be avoided, as it can be decided later. A perfect DUT will hit the decision coordinate (ne, ns) with ne=0. This test time is short.

### D.3.5	Standard test versus early decision concept

For Supplier Risk:
The wrong decision probability D in the standard test is the probability, to decide a DUT in-correctly in the single decision point. In the early decision concept there is a probability of in-correct decisions d at each point of the limit-curve. The sum of all those wrong decision probabilities accumulate to D. Hence d<D

For Customer Risk:
The correct decision probability CL in the standard test is the probability, to decide a DUT correctly in the single decision point. In the early decision concept there is a probability of correct decisions cl at each point of the limit-curve. The sum of all those correct decision probabilities accumulate to CL. Hence cl<CL or d>D

### D.3.6	Selectivity

There is no statistical test which can discriminate between a limit-DUT-quality and a DUT-quality which is an (ε0) apart from the limit in finite time and confidence level CL>1/2. Either the test discriminates against one limit with the results pass (with CL)/undecided or fail (with CL)/undecided, or the test ends in a result pass (with CL)/fail (with CL) but this requires a second limit.

For CL>0.5, a (measurement-result = specified-DUT-quality), generates undecided in test “supplier risk against pass limit” (a in clause D.3.2) and also in the equivalent test against the fail limit (aa in clause D.3.2)

For CL>0.5, a DUT, known to be on the limit, will be decided pass for the test “customer risk against pass limit” (b in clause D.3.2) and also in the equivalent test against fail limit (bb in clause D.3.2).

This overlap or undecided area is not a fault or a contradiction, however it can be avoided by introducing a Bad or a Good DUT quality according to:

-	Bad DUT quality: specified DUT-quality * M (M>1)

-	Good DUT quality: specified DUT-quality * m (m<1)

Using e.g. M>1 and CL=95% the test for different DUT qualities yield different pass probabilities:

Figure D.3.6: Pass probability versus DUT quality

### D.3.7	Design of the test

The test is defined according to the following design principles:

1.	The early decision concept is applied.

2.	A second limit is introduced: Bad DUT factor M>1

3.	To decide the test pass:

Supplier risk is applied based on the Bad DUT quality

To decide the test fail

Customer Risk is applied based on the specified DUT quality

The A-GNSS test cases are defined using the following parameters:

1.	Specified DUT quality: ER = 0.05

2.	Bad DUT quality: M=1.5 (selectivity)

3.	Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

The ECID and OTDOA test cases are defined using the following parameters:

1.	Specified DUT quality: ER = 0.1

2.	Bad DUT quality: M=1.5 (selectivity)

3.	Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

This has the following consequences:

a)	A measurement on the fail limit is connected with 2 equivalent statements:

| A measurement on the fail-limit shows, that the DUT is worse than the specified DUT-quality | A DUT, known to have the specified quality, shall be measured and decided pass |
| --- | --- |

A measurement on the pass limit is connected with the complementary statements:

| A measurement on the pass limit shows, that the DUT is better than the Bad DUT-quality. | A DUT, known to have the Bad DUT quality, shall be measured and decided fail |
| --- | --- |

The left column is used to decide the measurement.

The right column is used to verify the design of the test by simulation.

The simulation is based on the two fulcrums A and B only in Figure D.3.6. There is freedom to shape the remainder of the function.

b)	Test time

1.	The minimum and maximum test time is fixed.

2.	The average test time is a function of the DUT’s quality.

3.	The individual test time is not predictable (except ideal DUT).

c)	The number of decision co-ordinates (ne, ns) in the early decision concept is responsible for the selectivity of the test and the maximum test time. Having fixed the number of decision co-ordinates there is still freedom to select the individual decision co-ordinates in many combinations, all leading to the same confidence level.

## D.4	Pass fail decisions

### D.4.1	Numerical definition of the pass fail limits for A-GNSS test cases

Table D.4.1: Pass/fail limits for A-GNSS test cases

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 77 | NA | 43 | 855 | 576 | 86 | 1525 | 1297 | 129 | 2173 | 2050 |
| 1 | 106 | NA | 44 | 871 | 592 | 87 | 1540 | 1314 | 130 | 2188 | 2067 |
| 2 | 131 | NA | 45 | 887 | 608 | 88 | 1556 | 1331 | 131 | 2203 | 2085 |
| 3 | 154 | NA | 46 | 903 | 625 | 89 | 1571 | 1349 | 132 | 2218 | 2103 |
| 4 | 176 | NA | 47 | 919 | 641 | 90 | 1586 | 1366 | 133 | 2233 | 2121 |
| 5 | 197 | NA | 48 | 935 | 657 | 91 | 1601 | 1383 | 134 | 2248 | 2139 |
| 6 | 218 | 42 | 49 | 951 | 674 | 92 | 1617 | 1401 | 135 | 2263 | 2156 |
| 7 | 238 | 52 | 50 | 967 | 690 | 93 | 1632 | 1418 | 136 | 2277 | 2174 |
| 8 | 257 | 64 | 51 | 982 | 706 | 94 | 1647 | 1435 | 137 | 2292 | 2192 |
| 9 | 277 | 75 | 52 | 998 | 723 | 95 | 1662 | 1453 | 138 | 2307 | 2210 |
| 10 | 295 | 87 | 53 | 1014 | 739 | 96 | 1677 | 1470 | 139 | 2322 | 2227 |
| 11 | 314 | 100 | 54 | 1030 | 756 | 97 | 1692 | 1487 | 140 | 2337 | 2245 |
| 12 | 333 | 112 | 55 | 1046 | 772 | 98 | 1708 | 1505 | 141 | 2352 | 2263 |
| 13 | 351 | 125 | 56 | 1061 | 789 | 99 | 1723 | 1522 | 142 | 2367 | 2281 |
| 14 | 369 | 139 | 57 | 1077 | 805 | 100 | 1738 | 1540 | 143 | 2381 | 2299 |
| 15 | 387 | 152 | 58 | 1093 | 822 | 101 | 1753 | 1557 | 144 | 2396 | 2317 |
| 16 | 405 | 166 | 59 | 1108 | 839 | 102 | 1768 | 1574 | 145 | 2411 | 2335 |
| 17 | 422 | 180 | 60 | 1124 | 855 | 103 | 1783 | 1592 | 146 | 2426 | 2352 |
| 18 | 440 | 194 | 61 | 1140 | 872 | 104 | 1798 | 1609 | 147 | 2441 | 2370 |
| 19 | 457 | 208 | 62 | 1155 | 889 | 105 | 1813 | 1627 | 148 | 2456 | 2388 |
| 20 | 474 | 222 | 63 | 1171 | 906 | 106 | 1828 | 1644 | 149 | 2470 | 2406 |
| 21 | 492 | 237 | 64 | 1186 | 922 | 107 | 1844 | 1662 | 150 | 2485 | 2424 |
| 22 | 509 | 251 | 65 | 1202 | 939 | 108 | 1859 | 1679 | 151 | 2500 | 2442 |
| 23 | 526 | 266 | 66 | 1217 | 956 | 109 | 1874 | 1697 | 152 | 2515 | 2460 |
| 24 | 543 | 281 | 67 | 1233 | 973 | 110 | 1889 | 1714 | 153 | 2530 | 2478 |
| 25 | 560 | 295 | 68 | 1248 | 990 | 111 | 1904 | 1732 | 154 | 2544 | 2496 |
| 26 | 577 | 310 | 69 | 1264 | 1007 | 112 | 1919 | 1750 | 155 | 2559 | 2513 |
| 27 | 593 | 325 | 70 | 1279 | 1024 | 113 | 1934 | 1767 | 156 | 2574 | 2531 |
| 28 | 610 | 341 | 71 | 1295 | 1040 | 114 | 1949 | 1785 | 157 | 2589 | 2549 |
| 29 | 627 | 356 | 72 | 1310 | 1057 | 115 | 1964 | 1802 | 158 | 2603 | 2567 |
| 30 | 643 | 371 | 73 | 1326 | 1074 | 116 | 1979 | 1820 | 159 | 2618 | 2585 |
| 31 | 660 | 387 | 74 | 1341 | 1091 | 117 | 1994 | 1838 | 160 | 2633 | 2603 |
| 32 | 676 | 402 | 75 | 1357 | 1108 | 118 | 2009 | 1855 | 161 | 2648 | 2621 |
| 33 | 693 | 418 | 76 | 1372 | 1126 | 119 | 2024 | 1873 | 162 | 2662 | 2639 |
| 34 | 709 | 433 | 77 | 1387 | 1143 | 120 | 2039 | 1890 | 163 | 2677 | 2657 |
| 35 | 725 | 449 | 78 | 1403 | 1160 | 121 | 2054 | 1908 | 164 | 2692 | 2675 |
| 36 | 742 | 465 | 79 | 1418 | 1177 | 122 | 2069 | 1926 | 165 | 2707 | 2693 |
| 37 | 758 | 480 | 80 | 1433 | 1194 | 123 | 2084 | 1943 | 166 | 2721 | 2711 |
| 38 | 774 | 496 | 81 | 1449 | 1211 | 124 | 2099 | 1961 | 167 | 2736 | 2729 |
| 39 | 790 | 512 | 82 | 1464 | 1228 | 125 | 2114 | 1979 | 168 | 2751 | 2747 |
| 40 | 807 | 528 | 83 | 1479 | 1245 | 126 | 2128 | 1997 | 169 | 2765 | NA |
| 41 | 823 | 544 | 84 | 1495 | 1263 | 127 | 2143 | 2014 |  |  |  |
| 42 | 839 | 560 | 85 | 1510 | 1280 | 128 | 2158 | 2032 |  |  |  |

NOTE:	The first column is the number of bad results (ne)
The second column is the number of results for the pass limit (nsp)
The third column is the number of results for the fail limit (nsf)

### D.4.2	Pass fail decision rules for A-GNSS test cases

Having observed 0 bad results, pass the test at ≥77 results, otherwise continue

Having observed 1 bad result, pass the test at ≥106 results, otherwise continue

Having observed 2 bad results, pass the test at ≥131 results, otherwise continue

etc. until

Having observed 6 bad results, pass the test at ≥218 results, fail the test at ≤ 42 results, otherwise continue

Having observed 7 bad results, pass the test at ≥238 results, fail the test at ≤ 52 results, otherwise continue

etc. until

Having observed 168 bad results, pass the test at ≥2751 results, fail the test at ≤2747 results, otherwise continue

Having observed 169 bad results, pass the test at ≥2765 results, otherwise fail

NOTE:	an ideal DUT passes after 77 results. The maximum test time is 2765 results.

### D.4.3	Numerical definition of the pass fail limits for ECID, OTDOA, MBS, WLAN and BLE test cases

Table D.4.3: Pass/fail limits for ECID, OTDOA, MBS, WLAN and BLE test cases

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 33 | NA | 43 | 408 | 283 | 86 | 737 | 644 | 129 | 1056 | 1021 |
| 1 | 46 | NA | 44 | 416 | 291 | 87 | 745 | 653 | 130 | 1064 | 1030 |
| 2 | 58 | 2 | 45 | 424 | 299 | 88 | 752 | 661 | 131 | 1071 | 1039 |
| 3 | 69 | 5 | 46 | 432 | 307 | 89 | 760 | 670 | 132 | 1078 | 1048 |
| 4 | 79 | 8 | 47 | 440 | 315 | 90 | 767 | 679 | 133 | 1086 | 1057 |
| 5 | 89 | 12 | 48 | 447 | 324 | 91 | 775 | 687 | 134 | 1093 | 1066 |
| 6 | 99 | 17 | 49 | 455 | 332 | 92 | 782 | 696 | 135 | 1100 | 1074 |
| 7 | 109 | 22 | 50 | 463 | 340 | 93 | 790 | 705 | 136 | 1108 | 1083 |
| 8 | 118 | 27 | 51 | 471 | 348 | 94 | 797 | 713 | 137 | 1115 | 1092 |
| 9 | 127 | 33 | 52 | 478 | 356 | 95 | 804 | 722 | 138 | 1122 | 1101 |
| 10 | 136 | 39 | 53 | 486 | 365 | 96 | 812 | 731 | 139 | 1130 | 1110 |
| 11 | 145 | 45 | 54 | 494 | 373 | 97 | 819 | 739 | 140 | 1137 | 1119 |
| 12 | 154 | 51 | 55 | 502 | 381 | 98 | 827 | 748 | 141 | 1144 | 1128 |
| 13 | 163 | 58 | 56 | 509 | 389 | 99 | 834 | 757 | 142 | 1152 | 1137 |
| 14 | 172 | 64 | 57 | 517 | 398 | 100 | 842 | 766 | 143 | 1159 | 1147 |
| 15 | 180 | 71 | 58 | 525 | 406 | 101 | 849 | 774 | 144 | 1166 | 1155 |
| 16 | 189 | 78 | 59 | 532 | 414 | 102 | 857 | 783 | 145 | 1174 | 1164 |
| 17 | 197 | 85 | 60 | 540 | 423 | 103 | 864 | 792 | 146 | 1181 | 1173 |
| 18 | 206 | 92 | 61 | 548 | 431 | 104 | 871 | 801 | 147 | NA | 1182 |
| 19 | 214 | 99 | 62 | 555 | 440 | 105 | 879 | 809 | 148 |  |  |
| 20 | 223 | 106 | 63 | 563 | 448 | 106 | 886 | 818 | 149 |  |  |
| 21 | 231 | 113 | 64 | 571 | 456 | 107 | 894 | 827 | 150 |  |  |
| 22 | 239 | 120 | 65 | 578 | 465 | 108 | 901 | 836 | 151 |  |  |
| 23 | 248 | 128 | 66 | 586 | 473 | 109 | 909 | 844 | 152 |  |  |
| 24 | 256 | 135 | 67 | 594 | 482 | 110 | 916 | 853 | 153 |  |  |
| 25 | 264 | 142 | 68 | 601 | 490 | 111 | 923 | 862 | 154 |  |  |
| 26 | 272 | 150 | 69 | 609 | 499 | 112 | 931 | 871 | 155 |  |  |
| 27 | 281 | 157 | 70 | 616 | 507 | 113 | 938 | 880 | 156 |  |  |
| 28 | 289 | 165 | 71 | 624 | 516 | 114 | 946 | 888 | 157 |  |  |
| 29 | 297 | 173 | 72 | 632 | 524 | 115 | 953 | 897 | 158 |  |  |
| 30 | 305 | 180 | 73 | 639 | 533 | 116 | 960 | 906 | 159 |  |  |
| 31 | 313 | 188 | 74 | 647 | 541 | 117 | 968 | 915 | 160 |  |  |
| 32 | 321 | 196 | 75 | 654 | 550 | 118 | 975 | 924 | 161 |  |  |
| 33 | 329 | 204 | 76 | 662 | 558 | 119 | 983 | 933 | 162 |  |  |
| 34 | 337 | 211 | 77 | 669 | 567 | 120 | 990 | 941 | 163 |  |  |
| 35 | 345 | 219 | 78 | 677 | 575 | 121 | 997 | 950 | 164 |  |  |
| 36 | 353 | 227 | 79 | 684 | 584 | 122 | 1005 | 959 | 165 |  |  |
| 37 | 361 | 235 | 80 | 692 | 592 | 123 | 1012 | 968 | 166 |  |  |
| 38 | 369 | 243 | 81 | 700 | 601 | 124 | 1019 | 977 | 167 |  |  |
| 39 | 377 | 251 | 82 | 707 | 610 | 125 | 1027 | 986 | 168 |  |  |
| 40 | 385 | 259 | 83 | 715 | 618 | 126 | 1034 | 994 | 169 |  |  |
| 41 | 393 | 267 | 84 | 722 | 627 | 127 | 1042 | 1003 |  |  |  |
| 42 | 400 | 275 | 85 | 730 | 635 | 128 | 1049 | 1012 |  |  |  |

The first column is the number of errors (ne = number of exceeded delays or number of wrong reports)

The second column is the number of samples for the pass limit (nsp , ns=Number of samples= number of successes + number of exceedings or number of reports)

The third column is the number of samples for the fail limit (nsf)

### D.4.4	Pass fail decision rules for ECID, OTDOA, MBS, WLAN and BLE test cases

Having observed 0 errors, pass the test at 33+ samples, otherwise continue

Having observed 1 error, pass the test at 46+ samples, otherwise continue

Having observed 2 errors, pass the test at 58+ samples, fail the test at 2 samples, otherwise continue

Having observed 146 errors, pass the test at 1181+ samples, fail the test at 1173- samples, otherwise continue

Having observed 147 errors, fail the test at 1182- samples,

Where x+ means: x or more, x- means x or less

NOTE: an ideal DUT passes after 33 samples. The maximum test time is 1181 samples.

### D.4.5	Background information to the pass fail limits

There is freedom to design the decision co-ordinates (ne, ns).

The binomial distribution and its inverse is used to design the pass and fail limits. Note that this method is not unique and that other methods exist.

Where

fail(..) is the error ratio for the fail limit

pass(..) is the error ratio for the pass limit

ER is the specified error ratio e.g. 0.05

ne is the number of bad results. This is the variable in both equations

M is the Bad DUT factor M=1.5

df is the wrong decision probability of a single (ne, ns) co-ordinate for the fail limit.
It is found by simulation to be df = 0.004

clp is the confidence level of a single (ne, ns) co-ordinate for the pass limit.
It is found by simulation to be clp = 0.9975

qnbinom(..): The inverse cumulative function of the negative binomial distribution

The simulation works as follows:

A large population of limit DUTs with true ER = 0.05 is decided against the pass and fail limits.

clp    and df   are tuned such that CL (95%) of the population passes and D (5%) of the population fails.

A population of Bad DUTs with true ER = M*0.05 is decided against the same pass and fail limits.

clp    and df   are tuned such that CL (95%) of the population fails and D (5%) of the population passes.

This procedure and the relationship to the measurement is justified in clause D.3.7. The number of DUTs decreases during the simulation, as the decided DUTs leave the population. That number decreases with an approximately exponential characteristics. After 169 bad results all DUTs of the population are decided.

NOTE:	The exponential decrease of the population is an optimal design goal for the decision co-ordinates (ne, ns), which can be achieved with other formulas or methods as well.

Annex E (normative):
Conditions for ECID and OTDOA requirements

## E.1	Conditions for E-CID UE Rx – Tx time difference Measurements

This clause defines the E-UTRAN RSRP applicable for ECID UE Rx-Tx time difference Measurements for a corresponding operating band

The conditions for E-UTRAN ECID UE Rx-Tx time difference measurements are as defined in Table E.1-1.

Table E.1-1: Conditions for ECID UE Rx-Tx time difference measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in clause 4.4.2. |  |  |

### E.1.1	Conditions for E-CID UE Rx – Tx time difference by UE Category M1/M2

This clause defines the E-UTRAN RSRP applicable for ECID UE Rx-Tx time difference Measurements for a corresponding operating band for UE Category M1 and M2.

The conditions for CE mode A intra-frequency E-UTRAN FDD, HD-FDD and TDD measurements are defined in Table E.1.1-1.

Table B.2.14-1: E-UTRAN ECID UE Rx-Tx time difference measurements for FDD, HD-FDD and TDD for CE mode A

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD-M1_A, TDD-M1_A | -127 |
|  | FDD-M1_D | -125.5 |
|  | FDD-M1_E, TDD-M1_E | -125 |
|  | FDD-M1_F | -124.5 Note 2 |
|  | FDD-M1_G | -124 |
|  | FDD-M1_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in clause 4.4.2. |  |  |

## E.2	Conditions for OTDOA intra-frequency RSTD Measurements

This clause defines the E-UTRAN intra-frequency PRP_1,2 applicable for OTDOA intra-frequency RSTD measurements for a corresponding operating band.

The conditions for E-UTRAN OTDOA intra-frequency RSTD measurements are as defined in Table E.2-1

Table E.2-1: Conditions for OTDOA intra-frequency RSTD measurements

| Parameter | E-UTRA operating band group s Note 3 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133[23]. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in clause 4.4.2. |  |  |

### E.2.1	Conditions for OTDOA intra-frequency RSTD Measurements by UE Category M1 and M2

This clause defines the E-UTRAN intra-frequency PRP1,2 applicable for a corresponding operating band.

The conditions for CE mode A intra-frequency E-UTRAN HD-FDD, FDD and TDD measurements are defined in Table E.2.1-1.

The conditions for CE mode B for intra-frequency E-UTRAN HD-FDD, FDD and TDD measurements are defined in Table E.2.1-2.

Table E.2.1-1: E-UTRAN intra-frequency measurements for HD-FDD, FDD and TDD for CE mode A

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 1 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 4.4.2. |  |  |

Table E.2.1-2: E-UTRAN intra-frequency measurements for HD-FDD, FDD and TDD for CE mode B

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -136 |
|  | FDD_D | -134.5 |
|  | FDD_E, TDD_E | -134 |
|  | FDD_F | -133.5 Note 1 |
|  | FDD_G | -133 |
|  | FDD_N | -129.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 4.4.2. |  |  |

## E.3	Conditions for OTDOA inter-frequency RSTD Measurements

This clause defines the E-UTRAN inter-frequency PRP_1,2 applicable for OTDOA Inter-frequency RSTD measurements for a corresponding operating band.

The conditions for E-UTRAN OTDOA inter-frequency RSTD measurements are as defined in Table E.2-1.

### E.3.1	Conditions for OTDOA inter-frequency RSTD Measurements by UE Category M1 and M2

This clause defines the E-UTRAN inter-frequency PRP1,2 applicable for a corresponding operating band.

The conditions for CE mode A inter-frequency E-UTRAN HD-FDD, FDD and TDD measurements are defined in Table E.3.1-1.

The conditions for CE mode B for inter-frequency E-UTRAN HD-FDD, FDD and TDD measurements are defined in Table E.3.1-2.

Table E.3.1-1: E-UTRAN inter-frequency measurements for HD-FDD, FDD and TDD for CE mode A

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 1 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 4.4.2. |  |  |

Table E.3.1-2: E-UTRAN inter-frequency measurements for HD-FDD, FDD and TDD for CE mode B

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -136 |
|  | FDD_D | -134.5 |
|  | FDD_E, TDD_E | -134 |
|  | FDD_F | -133.5 Note 1 |
|  | FDD_G | -133 |
|  | FDD_N | -129.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 4.4.2. |  |  |

## E.4	Conditions for UE Rx-Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information

This clause defines the E-UTRAN RSRP applicable for UE Rx-Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information for a corresponding operating band.

The conditions for UE Rx-Tx time difference measurements, when time domain measurement resource restriction pattern and CRS assistance information are provided, are as defined in Table  E.1-1.

## E.5	Conditions for NB-IOT OTDOA intra-frequency RSTD Measurements

This clause defines the NB-IoT intra-frequency PRP1,2 applicable for a corresponding operating band.

The conditions for intra-frequency RSTD measurements in normal coverage are defined in Table E.5-1.

The conditions for intra-frequency RSTD measurements in enhanced coverage are defined in Table E.5-2.

Table E.5-1: NB-IoT intra-frequency RSTD measurements for HD-FDD in normal coverage

| Parameter | E-UTRA operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -129 |
| NOTE 1: E-UTRA operating band groups for NB-IoT are as defined in Section 4.11.1 |  |  |

Table E.5-2: NB-IoT intra-frequency RSTD measurements for HD-FDD in enhanced coverage

| Parameter | E-UTRA operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -135 |
| NOTE 1: E-UTRA operating band groups for NB-IoT are as defined in Section 4.11.1 |  |  |

## E.6	Conditions for NB-IOT OTDOA inter-frequency RSTD Measurements

This clause defines the NB-IoT intra-frequency PRP1,2 applicable for a corresponding operating band.

The conditions for inter-frequency RSTD measurements in normal coverage are defined in Table E.5-1.

The conditions for inter-frequency RSTD measurements in enhanced coverage are defined in Table E.5-2.

Annex F (normative):
UTRAN Generic procedures

## F.1	General

This normative annex specifies the set up and release procedure that shall be used for each UTRAN test case.

In this clause the terms GNSS and A-GNSS also include the cases where the only satellite system used is GPS unless otherwise stated.

## F.2	UTRAN connection set up

### F.2.1	Initial conditions

System Simulator:

-	1 cell, default parameters. The default system information, as specified in clause 6.1 of TS 34.108 [28], is broadcast with the exceptions of SIB15, SIB15.1, SIB15.2 and SIB15.3 which are not broadcast.

User Equipment:

-	The UE shall be operated in Normal Propagation Conditions as specified in clause 5.2.1 of TS 34.108 [28].

-	The UE is in state "MM idle" state with valid TMSI and CKSN.

-	The UE is in state "PMM idle" with valid P-TMSI.

### F.2.2	Procedures

CS Domain

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | SYSTEM INFORMATION (BCCH) | Broadcast |
| 2 | <-- |  | PAGING TYPE1 (PCCH) | Paging (CS domain, TMSI) |
| 3 | --> |  | RRC CONNECTION REQUEST (CCCH) | RRC |
| 4 | <-- |  | RRC CONNECTION SETUP (CCCH) | RRC |
| 5 | --> |  | RRC CONNECTION SETUP COMPLETE (DCCH) | RRC |
| 6 | --> |  | PAGING RESPONSE | RR |
| 7 | <-- |  | AUTHENTICATION REQUEST | MM |
| 8 | --> |  | AUTHENTICATION RESPONSE | MM |
| 9 | <-- |  | SECURITY MODE COMMAND | RRC |
| 10 | --> |  | SECURITY MODE COMPLETE | RRC |

PS Domain

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | PAGING TYPE1 (PCCH) | Paging (PS domain, PMSI or IMSI) |
| 2 | --> |  | RRC CONNECTION REQUEST (CCCH) | RRC |
| 3 | <-- |  | RRC CONNECTION SETUP (CCCH) | RRC |
| 4 | --> |  | RRC CONNECTION SETUP COMPLETE (DCCH) | RRC (Transport Channel: DCH or FACH) |
| 5 | --> |  | SERVICE REQUEST | GMM |
| 6 | <-- |  | AUTHENTICATION REQUEST | GMM |
| 7 | --> |  | AUTHENTICATION RESPONSE | GMM |
| 8 | <-- |  | SECURITY MODE COMMAND | RRC |
| 9 | --> |  | SECURITY MODE COMPLETE | RRC |

### F.2.3	Specific message contents

The default message contents specified in clause 9.1 of TS 34.108 [28] will be used for the Moving Scenario and Periodic Update test. For all Minimum Performance TTFF Tests the default message contents specified in clause 9.1 of TS 34.108 [28] will be used with the following exception.

Contents of PAGING TYPE1:

| Information Element | Value/remark |
| --- | --- |
| Paging Cause | Terminating High Priority Signalling |

Contents of RRC CONNECTION SETUP:

For A-GNSS performance testing in CELL_DCH state: The RRC Connection Setup is defined in clause 9.1.1 of TS 34.108 [28] "Contents of RRC CONNECTION SETUP message: UM (Transition to CELL_DCH)".

For A-GNSS performance testing in CELL_FACH state: The RRC Connection Setup is defined in clause 9.1.1 of TS 34.108 [28] "Contents of RRC CONNECTION SETUP message: UM (Transition to CELL_FACH)".

Contents of RRC CONNECTION SETUP COMPLETE:

| Information Element | Value/remark |
| --- | --- |
| UE radio access capability |  |
| - UE positioning capability | Defines the A-GNSS mode the UE supports (UE-based, UE-assisted, or both). UE shall be tested for all modes it supports. |

## F.3	UTRAN connection release

### F.3.1	Procedure

Table F.3.1: UTRAN connection release procedure

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | RRC CONNECTION RELEASE | RRC |
| 2 | --> |  | RRC CONNECTION RELEASE COMPLETE | RRC |

### F.3.2	Specific message contents

The default message contents specified in clause 9.1 of TS 34.108 [28] are used.

Annex G (normative):
Environmental conditions

## G.1	General

This normative annex specifies the environmental requirements of the UE. Within these limits the requirements of the present documents shall be fulfilled.

## G.2	Environmental requirements

The requirements in this clause apply to all types of UE(s).

### G.2.1	Temperature

The UE shall fulfil all the requirements in the full temperature range of:

Table G.2.1.1

| +15°C to +35°C | for normal conditions (with relative humidity up to 75 %) |
| --- | --- |

### G.2.2	Voltage

The UE shall fulfil all the requirements in the full voltage range, i.e. the voltage range between the extreme voltages.

The manufacturer shall declare the lower and higher extreme voltages and the approximate shutdown voltage. For the equipment that can be operated from one or more of the power sources listed below, the lower extreme voltage shall not be higher, and the higher extreme voltage shall not be lower than that specified below.

Table G.2.2.1

| Power source | Normal conditions voltage |
| --- | --- |
| AC mains | nominal |
| Regulated lead acid battery | 1.1 nominal |
| Non regulated batteries: - Leclanché / lithium - Mercury/nickel and cadmium | Nominal Nominal |

Annex H (informative):
MBS Beacon parameters

## H.1	General

This informative annex consolidates a description of the MBS beacon parameters that are specified in the MBS ICD [38]. They are provided here for reference.

## H.2	Beacon parameters

This is a summary of the MBS beacon parameters to be used for MBS testing:

Beacon PN Code: 1023 chip length for TB1. Actual PN codes are listed it the MBS ICD [38]

MBS Beacon Configuration: TB1 (2.046 MHz, contains data) [38]

MBS Packet Type: Type 2 [38]

MBS Transmitter ID (TxID): Field used to signal a unique ID that identifies each transmitter within one major deployment area, such as within North America. Range: [0, 215-1] [38]

Slot Index: This is the physical time slot within a MBS transmission period, in which a transmitter is transmitting. Each slot is 100 ms in duration and a MBS transmission period is 1 sec long. [38]

All other fields: Set to the min value (bit value equal to 0) for testing [38]

Annex I (normative):
Conditions for NR PRS-based measurements

## I.1	General

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

## I.2	General

The conditions are defined in Table I.2-1 for FR1 NR cells.

The conditions are defined in Table I.2-2 for FR2 NR cells.

Editor’s notes for Table I.2-1and Table I.2-2:

- The Table is not complete, FFS for PRS-RSRP and UE Rx-Tx.

Table I.2-1: Conditions for NR PRS-based measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  |  | PRS Ês/Iot for RSTD measurement reference cell PRS resource |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  |  | dB |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz | SCSPRS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -6 -13 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G | -124 | -121 | -118 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in TS 38.101-1 [54] clause 3.5.2. |  |  |  |  |  |

Table I.2-2: Conditions for NR PRS-based measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum PRP1,2 Note 2, Note 3 |  |  |  |  | PRS Ês/Iot for RSTD measurement reference cell PRS resource |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSPRS |  |  |  |  | dB |
|  |  |  | SCSPRS = 120 kHz |  |  |  | SCSPRS = 60 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSPRS = 120 kHz) - 3dB | -6 -13 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSPRS = 120 kHz) - 3dB | -6 -13 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [55]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum PRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [55]. |  |  |  |  |  |  |  |  |

Annex J (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment TS 36.571-1 | Old | New |
| 2010-08 | RAN5#48 | R5-104316 |  |  | Initial draft TS 36.571-1 created |  | 0.0.0 |
| 2010-11 | RAN5#49 | R5-106613 |  |  | V1.0.0 created for presentation to RAN Plenary | 0.0.0 | 1.0.0 |
| 2011-02 | RAN5#50 | R5-110124 |  |  | Various values and corrections added | 1.0.0 | 1.1.0 |
| 2011-08 | RAN5#52 | R5-113133 |  |  | Text changes from R5-112139, R5-112386, R5-112837, R5-112838, R5-112839 added | 1.1.0 | 1.2.0 |
| 2011-08 | RAN5#53 |  |  |  | Text changes from R5-113135, R5-113150, R5-114066, R5-113587 added | 1.2.0 | - |
| 2011-11 | RAN5#53 | R5-115206 |  |  | Initial draft TS 37.571-1 created from TS 36.571-1, TS 34.171 and TS 34.172 | - | 1.0.0 |
| 2011-11 | RAN5#53 | R5-115207 |  |  | V2.0.0 created for presentation to RAN Plenary with additions from R5-115246, R5-115247, R5-115248, R5-115809 | 1.0.0 | 2.0.0 |
| 2011-12 | RAN#54 | - | - | - | Moved to Rel-9 with editorial changes only | 2.0.0 | 9.0.0 |
| 2012-03 | RAN#55 | R5-120087 | 0001 | - | Modify OTDOA connection diagrams | 9.0.0 | 9.1.0 |
| 2012-03 | RAN#55 | R5-120089 | 0002 | - | OTDOA parameter corrections | 9.0.0 | 9.1.0 |
| 2012-03 | RAN#55 | R5-120414 | 0003 | - | Adding ECID test cases to Annexes in TS 37.571-1 | 9.0.0 | 9.1.0 |
| 2012-03 | RAN#55 | R5-120822 | 0004 | - | Correct A-GNSS signalling | 9.0.0 | 9.1.0 |
| 2012-03 | RAN#55 | R5-120823 | 0005 | - | ECID procedure modifications | 9.0.0 | 9.1.0 |
| 2012-03 | RAN#55 | R5-120893 | 0006 | - | OTDOA procedure updates | 9.0.0 | 9.1.0 |
| 2012-06 | RAN#56 | R5-121126 | 0007 | - | Update to Figure 9.1.1.3-1 | 9.1.0 | 9.2.0 |
| 2012-06 | RAN#56 | R5-121127 | 0008 | - | Clarification to notes in tests 9.1.3 & 9.1.4 | 9.1.0 | 9.2.0 |
| 2012-06 | RAN#56 | R5-121128 | 0009 | - | Clarifications to frequencies and bandwidths to be used | 9.1.0 | 9.2.0 |
| 2012-06 | RAN#56 | R5-121129 | 0010 | - | Setting responseTime in ECID test cases | 9.1.0 | 9.2.0 |
| 2012-06 | RAN#56 | R5-121130 | 0011 | - | Modifications to signalling used in OTDOA test cases | 9.1.0 | 9.2.0 |
| 2012-06 | RAN#56 | R5-121908 | 0012 | - | Adding operating band 26 to TS 37.571-1 | 9.1.0 | 9.2.0 |
| 2012-06 | RAN#56 | - | - | - | Added missing contents from R5-121126, R5-121127, R5-121128 | 9.2.0 | 9.2.1 |
| 2012-06 | RAN#56 | - | - | - | Upgraded to v10.0.0 with no change. | 9.2.1 | 10.0.0 |
| 2012-09 | RAN#57 | R5-123066 | 0013 | - | Correction to RSTD Measurement Accuracy Tests 9.1.3 and 9.1.4 | 10.0.0 | 10.1.0 |
| 2012-09 | RAN#57 | R5-123913 | 0014 | - | Addition of RRM Test Case 9.8.4 TDD inter-frequency RSTD Accuracy | 10.0.0 | 10.1.0 |
| 2012-12 | RAN#58 | R5-125136 | 0015 | - | Corrections to references | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125188 | 0016 | - | Correction to LPP Message Content for GNSS Moving Scenario Test | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125806 | 0018 | - | New test case 10.1 FDD RSTD Measurement Reporting Delay for Carrier Aggregation | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125807 | 0019 | - | New test case 10.2 TDD RSTD Measurement Reporting Delay for Carrier Aggregation | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125808 | 0020 | - | New test case 10.3 FDD RSTD Measurement Accuracy for Carrier Aggregation | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125809 | 0021 | - | New test case 10.4.TDD RSTD Measurement Accuracy for Carrier Aggregation | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125831 | 0022 | - | Adding bands 28 and 44 to TS 37.571-1 | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125847 | 0023 | - | Corrections to procedures for RSTD tests | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125848 | 0024 | - | Correction of OCNG Patterns for UE Rx - Tx Time Difference Test Cases | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-125916 | 0025 | - | Add editor's note for value of Iprs for test case 9.1.4 | 10.1.0 | 10.2.0 |
| 2012-12 | RAN#58 | R5-124120 | 0026 | - | New common text for test cases 10.1 - 10.4 for RSTD for Carrier Aggregation | 10.1.0 | 10.2.0 |
| 2013-03 | RAN#59 | R5-130959 | 0027 |  | LBS Perf: Corrections to TCs 8.1.1 and 8.1.2 | 10.2.0 | 10.3.0 |
| 2013-06 | RAN#60 | R5-131097 | 0028 | - | Removal of Note 1 from OTDOA parameter tables | 10.3.0 | 10.4.0 |
| 2013-06 | RAN#60 | R5-131176 | 0029 | - | Clarification to RSTD Delay Test procedures | 10.3.0 | 10.4.0 |
| 2013-06 | RAN#60 | R5-131943 | 0030 | - | New Test Case for FDD-FDD inter-frequency RSTD Accuracy | 10.3.0 | 10.4.0 |
| 2013-06 | RAN#60 | R5-131944 | 0031 | - | New Test Case for FDD-FDD inter-frequency RSTD measurement reporting delay | 10.3.0 | 10.4.0 |
| 2013-06 | RAN#60 | R5-131945 | 0032 | - | OTDOA test case alignment with RAN 4 | 10.3.0 | 10.4.0 |
| 2013-06 | RAN#60 | R5-131946 | 0033 | - | Corrections to ECID and OTDOA tests Note: same contents as R5-131945 was submitted by accident. | 10.3.0 | 10.4.0 |
| 2013-06 | RAN#60 | R5-131947 | 0034 | - | RSTD test parameter updates | 10.3.0 | 10.4.0 |
| 2013-06 | RAN#60 | R5-131993 | 0035 | - | Test Description for TDD inter-frequency accuracy test case | 10.3.0 | 10.4.0 |
| 2013-06 | RAN#60 | R5-131994 | 0036 | - | New test case for TDD inter-frequency RSTD reporting delay 9.2.2 | 10.3.0 | 10.4.0 |
| 2013-09 | RAN#61 | R5-133173 | 0037 | - | Tidy up of Table 9.2.1.4.1-1 | 10.4.0 | 10.5.0 |
| 2013-09 | RAN#61 | R5-133174 | 0038 | - | Corrections to ECID and OTDOA tests | 10.4.0 | 10.5.0 |
| 2013-09 | RAN#61 | R5-133375 | 0039 | - | Uncertainties and Test Tolerances for RSTD test cases 9.1.1 and 9.1.2 | 10.4.0 | 10.5.0 |
| 2013-09 | RAN#61 | R5-133378 | 0040 | - | Uncertainties and Test Tolerances for RSTD test cases 9.1.3 and 9.1.4 | 10.4.0 | 10.5.0 |
| 2013-09 | RAN#61 | R5-133848 | 0041 | - | LBS Perf: Uncertainties and test tolerances for TCs 8.1.1 and 8.1.2 | 10.4.0 | 10.5.0 |
| 2013-09 | RAN#61 | R5-133885 | 0042 | - | LBS Perf: Revision of test procedure for TC-s 8.1.1-2 | 10.4.0 | 10.5.0 |
| 2013-12 | RAN#62 | R5-134200 | 0043 | - | Updates to ECID and RSTD tests following RAN 4 updates | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-134202 | 0044 | - | Addition of Capability exchange in ECID and RSTD tests | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-134205 | 0045 | - | Addition of Applicabilities for 9.2.1 - 9.2.5 | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-134849 | 0046 | - | Addition of missing acknowledgements in ECID tests | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-134850 | 0047 | - | Corrections to references for OCNG and RMC | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-134899 | 0048 | - | Introduction 8.1.3 E-UTRAN FDD UE Rx-Tx time difference (feICIC) | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-134970 | 0049 | - | Introduction 8.1.4 E-UTRAN TDD UE Rx-Tx time difference (feICIC) | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-134979 | 0050 | - | Addition of new tests 10.1a, 10.2a, 10.3a and 10.4a for 20MHz CA | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-134980 | 0051 | - | LBS Perf: Corrections to RSTD reporting tests | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-135016 | 0052 | - | Uncertainties and Test Tolerances for RSTD test cases 9.2.1 and 9.2.2 | 10.5.0 | 10.6.0 |
| 2013-12 | RAN#62 | R5-135018 | 0053 | - | Uncertainties and Test Tolerances for RSTD test cases 9.2.4 and 9.2.5 | 10.5.0 | 10.6.0 |
| 2014-03 | RAN#63 | R5-140107 | 0054 | - | Corrections to PRS_RA in RSTD tests | 10.6.0 | 10.7.0 |
| 2014-03 | RAN#63 | R5-140278 | 0055 | - | Addition of E-UTRA band groups | 10.6.0 | 10.7.0 |
| 2014-03 | RAN#63 | R5-140308 | 0056 | - | LBS RF: Aperiodic CQI configuration for 1.4 MHz bandwidth subtests | 10.6.0 | 10.7.0 |
| 2014-03 | RAN#63 | R5-141033 | 0057 | - | RSTD test case updates | 10.6.0 | 10.7.0 |
| 2014-03 | RAN#63 | R5-140875 | 0058 | - | Additions to TC 8.1.6 E-UTRAN TDD UE Rx-Tx time difference (feICIC) | 10.7.0 | 11.0.0 |
| 2014-03 | RAN#63 | R5-141010 | 0059 | - | Additions to TC 8.1.5 E-UTRAN FDD UE Rx-Tx time difference (feICIC) | 10.7.0 | 11.0.0 |
| 2014-06 | RAN#64 | R5-142098 | 0060 | - | Corrections for OCNG patterns defined in RSTD Tables | 11.0.0 | 11.1.0 |
| 2014-06 | RAN#64 | R5-142211 | 0061 | - | Additions to feICIC UE Rx-Tx test cases in Annex C | 11.0.0 | 11.1.0 |
| 2014-06 | RAN#64 | R5-142302 | 0062 | - | LBS RF: Aperiodic CQI configuration for 1.4 MHz bandwidth tests | 11.0.0 | 11.1.0 |
| 2014-06 | RAN#64 | R5-143109 | 0063 | - | Additions to TC 8.1.6 E-UTRAN TDD UE Rx-Tx time difference (feICIC) | 11.0.0 | 11.1.0 |
| 2014-06 | RAN#64 | R5-143111 | 0064 | - | Additions to FDD interruption requirements for SCell | 11.0.0 | 11.1.0 |
| 2014-06 | RAN#64 | R5-143180 | 0065 | - | Additions to TC 8.1.5 E-UTRAN FDD UE Rx-Tx time difference (feICIC) | 11.0.0 | 11.1.0 |
| 2014-06 | RAN#64 | R5-143211 | 0066 | - | LBS RF: Update of RSTD tests | 11.0.0 | 11.1.0 |
| 2014-09 | RAN#65 | R5-144080 | 0068 | - | Corrections to RSTD Measurement Reporting Delay for Carrier Aggregation tests | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144125 | 0069 | - | Corrections to Physical Cell Id (PCI) Configuration Conditions in UE Rx-Tx time difference feICIC | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144193 | 0072 | - | Cell-specific test parameters for E-UTRAN | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144215 | 0081 | - | Clarification to RSTD Reporting Delay tests | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144409 | 0083 | - | Update to initial conditions and measurement procedure in for UTRA A-GPS and A-GNSS tests | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144553 | 0084 | - | Removal of editors note in TC 9.1.4 | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144620 | 0085 | - | Updates OTDOA Neighbour Cell Info List | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144763 | 0086 | - | Correction for RSTD Measurement Accuracy in CA requirements in RRM | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144789 | 0070 | - | Uncertainties and Test Tolerances for RSTD test cases 10.1, 10.1A, 10.2 and 10.2A | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144790 | 0071 | - | Uncertainties and Test Tolerances for RSTD test cases 10.3, 10.3A, 10.4 and 10.4A | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144791 | 0082 | - | Updates to Annex E of TS 37.571-1 | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144792 | 0087 | - | RSTD tests RAN 4 alignment | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144865 | 0088 | - | Editor's note to PRS levels with fading | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144870 | 0089 | - | Corrections to Note 3 for RSTD CA tests | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144914 | 0090 | - | Corrections to TDD RSTD Measurement Reporting Delay for Carrier Aggregation | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144915 | 0091 | - | RSTD CA Measurement Accuracy connection diagrams | 11.1.0 | 11.2.0 |
| 2014-09 | RAN#65 | R5-144203 | 0073 | - | Addition of new TC 10.1B FDD RSTD Measurement Reporting Delay CA for 5+5MHz | 11.2.0 | 12.0.0 |
| 2014-09 | RAN#65 | R5-144204 | 0074 | - | Addition of new TC 10.1C FDD RSTD Measurement Reporting Delay CA for 10+5MHz | 11.2.0 | 12.0.0 |
| 2014-09 | RAN#65 | R5-144205 | 0075 | - | Addition of new TC 10.2B TDD RSTD Measurement Reporting Delay CA for 5+5MHz | 11.2.0 | 12.0.0 |
| 2014-09 | RAN#65 | R5-144206 | 0076 | - | Addition of new TC 10.2C TDD RSTD Measurement Reporting Delay CA for 10+5MHz | 11.2.0 | 12.0.0 |
| 2014-09 | RAN#65 | R5-144207 | 0077 | - | Addition of new TC 10.3B FDD RSTD Measurement Accuracy CA for 5+5MHz | 11.2.0 | 12.0.0 |
| 2014-09 | RAN#65 | R5-144208 | 0078 | - | Addition of new TC 10.3C FDD RSTD Measurement Accuracy CA for 10+5MHz | 11.2.0 | 12.0.0 |
| 2014-09 | RAN#65 | R5-144209 | 0079 | - | Addition of new TC 10.4B TDD RSTD Measurement Accuracy CA for 5+5MHz | 11.2.0 | 12.0.0 |
| 2014-09 | RAN#65 | R5-144210 | 0080 | - | Addition of new TC 10.4C TDD RSTD Measurement Accuracy CA for 10+5MHz | 11.2.0 | 12.0.0 |
| 2014-12 | RAN#66 | R5-145133 | 0092 | - | Update Galileo ICD reference | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145211 | 0093 | - | Io value incorrectly calculated | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145212 | 0094 | - | Editorial Note clarification | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145213 | 0095 | - | Alignment of Es/Nos value | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145214 | 0096 | - | Duplicated Io values listed in RSTD tables | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145215 | 0097 | - | Correction to References in Specification | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145254 | 0098 | - | LBS Perf: Corrections to measurement gap configuration | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145490 | 0099 | - | Test Tolerances for TC 8.1.5 E-UTRAN FDD UE Rx-Tx time difference (feICIC) | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145491 | 0100 | - | Test Tolerances for TC 8.1.6 E-UTRAN TDD UE Rx-Tx time difference (feICIC) | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145492 | 0101 | - | Uncertainties and Test Tolerances to Annex C for feICIC UE Rx-Tx test cases | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145502 | 0102 | - | Correction to periodicity of ABS pattern in UE RX-TX time difference for feICIC | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145503 | 0103 | - | Introduction of BDS testing in Annex C of 37.571 | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145510 | 0104 | - | Correction to Annex E notes and tables | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145836 | 0105 | - | Changes to RSTD tests to align with RAN 4 | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145843 | 0106 | - | Introduction of feICIC applicability statement for UE Rx-TX Time Difference test cases | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145864 | 0107 | - | Introduction of content for BDS and UTRA TDD UE in section 1-3 of TS 37.571-1 | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145865 | 0108 | - | Introduction of content for BDS in section 4 in TS 37.571-1 | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145870 | 0109 | - | Corrections to measurement procedures for UTRA A-GPS and A-GNSS tests | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145871 | 0110 | - | Correction to UE Rx-Tx Time difference tests | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145920 | 0111 | - | Introduction of test cases for BDS and UTRA TDD UE in section 6 of TS 37.571-1 | 12.0.0 | 12.1.0 |
| 2014-12 | RAN#66 | R5-145930 | 0112 | - | Introduction of test cases for BDS in section 7 of TS 37.571-1 | 12.0.0 | 12.1.0 |
| 2015-03 | RAN#67 | R5-150051 | 0113 | - | Updates to RSTD values and terminology following changes in RAN 4 | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150074 | 0114 | - | Remove incorrect note from CA RSTD accuracy tests | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150082 | 0115 | - | Uncertainties and Test Tolerances for RSTD test cases 10.1, 10.1A, 10.1B, 10.1C, 10.2, 10.2A, 10.2B, and 10.2C | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150085 | 0116 | - | Uncertainties and Test Tolerances for RSTD test cases 10.3B, 10.3C, 10.4B, and 10.4C | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150107 | 0117 | - | Corrections to table headings in CA RSTD tests | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150378 | 0118 | - | Very minor corrections to references for feICIC test cases, 8.1.5 and 8.1.6 | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150609 | 0119 | - | Abbreviation Corrections for BDS in 37.571-1 | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150833 | 0120 | - | Addition of BDS ICD reference | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150834 | 0121 | - | Corrections to BDS Test Requirements for Minimum Performance tests | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150835 | 0122 | - | The lo Unit Parameter is Incorrect | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150836 | 0123 | - | Inconsistent Text Referenced | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150837 | 0124 | - | Missing Abbreviations in Specification | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150888 | 0125 | - | New TC: TDD RSTD Measurement Accuracy for Carrier Aggregation for 20MHz+10MHz bandwidth | 12.1.0 | 12.2.0 |
| 2015-03 | RAN#67 | R5-150911 | 0126 | - | New TC: TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20MHz +10MHz Bandwidth | 12.1.0 | 12.2.0 |
| 2015-06 | RAN#68 | R5-151070 | 0128 | - | Delete "FFS" from ECID test conditions | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151083 | 0129 | - | Uncertainties and Test Tolerances for RSTD test case 10.2D | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151085 | 0130 | - | Uncertainties and Test Tolerances for RSTD test case 10.4D | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151086 | 0131 | - | RSTD accuracy changes for Rel-12 | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151088 | 0132 | - | Formatting error in Parameter Sensitivity Coarse Tables | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151089 | 0133 | - | Incorrect Expected RSTD value in Table 9.2.5.4.1-1 | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151162 | 0135 | - | Correction of the TPR of Absolute GNSS signal level for Dynamic Range | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151331 | 0136 | - | Addition of band 32 to 37.571-1 | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151335 | 0137 | - | Corrections to message contents for feICIC TCs in 37.571-1 | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151912 | 0134 | 1 | LPP responseTime update and correction | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151913 | 0138 | 1 | Introduction of new test case 8.1.3 to 37.571-1 | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-151914 | 0139 | 1 | Introduction of new test case 8.1.4 to 37.571-1 | 12.2.0 | 12.3.0 |
| 2015-06 | RAN#68 | R5-152014 | 0127 | 1 | Add TDD to A-GNSS testing | 12.2.0 | 12.3.0 |
| 2015-09 | RAN#69 | R5-153253 | 0140 | - | CA LBS: Clarification of PHICH configuration | 12.3.0 | 12.4.0 |
| 2015-09 | RAN#69 | R5-153661 | 0144 | - | Update of Galileo OS SIS ICD reference | 12.3.0 | 12.4.0 |
| 2015-09 | RAN#69 | R5-153863 | 0143 | 1 | Update of feICIC Test cases 8.1.5 and 8.1.6 | 12.3.0 | 12.4.0 |
| 2015-09 | RAN#69 | R5-153864 | 0141 | 1 | Update of eICIC Test case 8.1.3 | 12.3.0 | 12.4.0 |
| 2015-09 | RAN#69 | R5-153865 | 0142 | 1 | Update of eICIC Test case 8.1.4 | 12.3.0 | 12.4.0 |
| 2015-09 | RAN#69 | - | - | - | update of the "non-specific references" in section 2 according to the approved R5-153582 and an action point on ETSI MCC | 12.3.0 | 12.4.0 |
| 2015-12 | RAN#70 | R5-155018 | 0145 | - | Uncertainties and Test Tolerances for RSTD Test Cases 10.3A_1 and 10.4A_1 | 12.4.0 | 12.5.0 |
| 2015-12 | RAN#70 | R5-155035 | 0146 | - | Incorrect Table Note referenced in LPP Request Table | 12.4.0 | 12.5.0 |
| 2015-12 | RAN#70 | R5-155063 | 0149 | - | Incorrect references in TDD test cases | 12.4.0 | 12.5.0 |
| 2015-12 | RAN#70 | R5-155064 | 0150 | - | Reference [2] has no explanation | 12.4.0 | 12.5.0 |
| 2015-12 | RAN#70 | R5-155065 | 0151 | - | Remove square brackets from RSTD tests | 12.4.0 | 12.5.0 |
| 2015-12 | RAN#70 | R5-155066 | 0152 | - | Incorrect Section number referenced | 12.4.0 | 12.5.0 |
| 2015-12 | RAN#70 | R5-155081 | 0153 | - | Editorial changes to correct Section and Table references | 12.4.0 | 12.5.0 |
| 2015-12 | RAN#70 | R5-155875 | 0154 | 1 | Two new 3 DL CA RSTD Measurement Reporting Delay test cases | 12.4.0 | 12.5.0 |
| 2015-12 | RAN#70 | R5-156111 | 0155 | 1 | Two new 3 DL CA RSTD Measurement Accuracy test cases | 12.4.0 | 12.5.0 |
| 2016-03 | RAN#71 | R5-160041 | 0156 | - | Correction to Cells in OTDOA 3DL RSTD Measurement | 12.5.0 | 12.6.0 |
| 2016-03 | RAN#71 | R5-160900 | 0164 | - | Add Cell values in RSTD Table for 3DL RSTD | 12.5.0 | 12.6.0 |
| 2016-03 | RAN#71 | R5-160909 | 0157 | 1 | Correction of Cell Time offset in RSTD CA | 12.5.0 | 12.6.0 |
| 2016-03 | RAN#71 | R5-161016 | 0161 | 1 | Add Cell values in OTDOA table for 3DL RSTD Measurement Reporting Delay | 12.5.0 | 12.6.0 |
| 2016-03 | RAN#71 | R5-161053 | 0158 | 1 | Correction to Trstd values in 3DL RSTD Measurement Accuracy test cases | 12.5.0 | 12.6.0 |
| 2016-03 | RAN#71 | R5-161054 | 0160 | 1 | Addition of antenna diagram Figure for 3DL CA test cases | 12.5.0 | 12.6.0 |
| 2016-06 | RAN#72 | R5-162114 | 0165 | - | Uncertainties and Test Tolerances for RSTD Test Cases 10.5 and 10.6 | 12.6.0 | 12.7.0 |
| 2016-06 | RAN#72 | R5-162116 | 0166 | - | Uncertainties and Test Tolerances for RSTD Test Cases 10.7 and 10.8 | 12.6.0 | 12.7.0 |
| 2016-06 | RAN#72 | R5-163116 | 0167 | 1 | Uncertainties and Test tolerances for TS 37.571-1 Test cases 8.1.3 and 8.1.4 | 12.6.0 | 12.7.0 |
| 2016-06 | RAN#72 | R5-162970 | 0168 | 1 | Add missing LTE FDD TDD bands to E-UTRA Band Groups | 12.7.0 | 13.0.0 |
| 2016-06 | RAN#72 | R5-162971 | 0169 | 1 | Add missing LTE FDD band to Annex E | 12.7.0 | 13.0.0 |
| 2016-09 | RAN#73 | R5-165350 | 0179 | - | Incorrect FDD Band reference noted for Band 32 | 13.0.0 | 13.1.0 |
| 2016-09 | RAN#73 | R5-165360 | 0181 | - | Correct editorial changes in Annex C of 37.571-1 | 13.0.0 | 13.1.0 |
| 2016-09 | RAN#73 | R5-166125 | 0173 | 1 | Updates to the UE Rx – Tx Time Difference tests for Rel-12 onwards | 13.0.0 | 13.1.0 |
| 2016-09 | RAN#73 | R5-166126 | 0178 | 1 | Add missing references to GPS and Galileo and A-GPS and A-Galileo | 13.0.0 | 13.1.0 |
| 2016-09 | RAN#73 | R5-166127 | 0180 | 1 | Add Derivation of Test Requirements for test cases 8.1.5 and 8.1.6 | 13.0.0 | 13.1.0 |
| 2016-09 | RAN#73 | R5-166168 | 0182 | 1 | Addition of performance test specification for Indoor Positioning Enhancements (MBS) | 13.0.0 | 13.1.0 |
| 2016-09 | RAN#73 | R5-166179 | 0171 | 1 | Addition of editor’s notes for TDD UE Rx-TX tests | 13.0.0 | 13.1.0 |
| 2016-09 | RAN#73 | R5-166181 | 0174 | 1 | Unification of Channel BW testing requirements for OTDOA 3 DL CA test cases | 13.0.0 | 13.1.0 |
| 2016-09 | RAN#73 | R5-166283 | 0183 | - | Change of Reference Channel for 1.4 MHz RSTD tests | 13.0.0 | 13.1.0 |
| 2016-12 | RAN#74 | R5-168060 | 0185 | - | Change references to Reference Channel for RSTD tests | 13.1.0 | 13.2.0 |
| 2016-12 | RAN#74 | R5-168063 | 0186 | - | Change of applicability of UE Rx-Tx tests for TDD | 13.1.0 | 13.2.0 |
| 2016-12 | RAN#74 | R5-168147 | 0187 | - | Corrections for errors in 37.571-1 | 13.1.0 | 13.2.0 |
| 2016-12 | RAN#74 | R5-169602 | 0184 | 1 | Addition of test tolerances to the performance test specification for Indoor Positioning Enhancements (MBS) | 13.1.0 | 13.2.0 |
| 2016-12 | RAN#74 | R5-169611 | 0190 | 1 | Clarification of MBS beacon code phase delay | 13.1.0 | 13.2.0 |
| 2016-12 | RAN#74 | R5-169663 | 0188 | 1 | Editorial correction on OTDOA TC10.6 | 13.1.0 | 13.2.0 |
| 2016-12 | RAN#74 | R5-168396 | 0189 | - | Band 70 addition to band groups in 37.571-1 | 13.2.0 | 14.0.0 |
| 2017-01 | RAN#74 | - | - | - | correction of floating point of R5-169602 in Table 11.4.5-3 | 14.0.0 | 14.0.1 |
| 2017-03 | RAN#75 | R5-171304 | 0195 | - | Removal of square brackets in the performance test specification for Indoor Positioning Enhancements (MBS) | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171902 | 0191 | 1 | Update TS 37.571-1 with Addition of LTE Band 48 | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171904 | 0192 | 1 | Clarification on DRX for Single Mode OTDOA Measurement Reporting Delay Test Cases | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171905 | 0193 | 1 | Clarification on DRX for 2CC OTDOA Measurement Reporting Delay Test Cases | 14.0.1 | 14.1.0 |
| 2017-03 | RAN#75 | R5-171906 | 0194 | 1 | Clarification on DRX for 3CC OTDOA Measurement Reporting Delay Test Cases | 14.0.1 | 14.1.0 |
| 2017-06 | RAN#76 | R5-172179 | 0197 | - | Add Minimum Performance Sub-tests for 3 GNSS | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-172623 | 0198 | - | Introduction of periodical reporting capability for GNSS | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173364 | 0200 | 1 | Introduction of MBS Assistance Data Measurement Test Cases | 14.1.0 | 14.2.0 |
| 2017-06 | RAN#76 | R5-173414 | 0201 | 1 | Addition of ACKs in step 5 of test procedures | 14.1.0 | 14.2.0 |
| 2017-09 | RAN#77 | R5-173569 | 0202 | - | Correction of PRS Subframe Offset for TC 10.5 and 10.6 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173570 | 0203 | - | Correction of SRS-Bandwidth for ECID TC 8.1.3 and 8.1.4 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173571 | 0204 | - | Correction of SRS-Bandwidth for ECID TC 8.1.5 and 8.1.6 | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173572 | 0205 | - | Correction of message contents for ECID (Editorial Change) | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173576 | 0209 | - | WLAN and BLE Annex D updates (Editorial Change) | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-173863 | 0214 | - | Editorial change to clarify the MBS test cases applicability | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-174053 | 0215 | - | Update Statement Concerning Test System Uncertainties for Operating Bands Above 3 GHz | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175116 | 0206 | 1 | New Abbreviations and References for WLAN and BLE (Editorial Change) | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175117 | 0207 | 1 | WLAN test conditions | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175118 | 0208 | 1 | WLAN and BLE Connection Diagrams | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175119 | 0212 | 1 | BLE test conditions | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175186 | 0210 | 1 | New WLAN AP Identification in Nominal Accuracy Test | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175187 | 0211 | 1 | New WLAN AP Identification in Dynamic Range Test | 14.2.0 | 14.3.0 |
| 2017-09 | RAN#77 | R5-175188 | 0213 | 1 | New BLE Reporting Test | 14.2.0 | 14.3.0 |
| 2017-12 | RAN#78 | R5-176110 | 0219 | - | Add 4Rx support for OTDOA/ECID tests – Tests | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176111 | 0220 | - | Add 4Rx support for OTDOA/ECID tests – Connection Diagrams | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177118 | 0222 | 1 | Adapt LTE A-GNSS test cases for BL/CE devices | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177411 | 0216 | 1 | Complete WLAN and BLE test cases | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177413 | 0217 | 1 | Editorial - Alignment of 2CC 5MHz OTDOA Nprs with core spec | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177414 | 0218 | 1 | Add 4Rx support for OTDOA/ECID tests – Common Sections | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-177415 | 0221 | 1 | Add release information for sub-tests of test case 7.5. | 14.3.0 | 14.4.0 |
| 2017-12 | RAN#78 | R5-176791 | 0223 | - | Band 72 addition to band groups in 37.571-1 | 14.4.0 | 15.0.0 |
| 2017-12 | RAN#78 | R5-176816 | 0224 | - | Band 71 addition to band groups in 37.571-1 | 14.4.0 | 15.0.0 |
| 2018-03 | RAN#79 | R5-180295 | 0228 | - | New OTDOA Cat1bis TC 9.1.3A and 9.1.4A | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180296 | 0229 | - | New OTDOA Cat1bis TC 9.2.4A and 9.2.5A | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180297 | 0230 | - | Annex C OTDOA Cat1bis tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180298 | 0231 | - | UE Category M1/M2 General Sections for OTDOA | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180299 | 0232 | - | New OTDOA Cat M1/M2 reporting delay normal mode tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180300 | 0233 | - | New OTDOA Cat M1/M2 reporting delay enhanced mode tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180301 | 0234 | - | New OTDOA Cat M1/M2 reporting accuracy normal mode tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180303 | 0236 | - | New OTDOA Cat M1/M2 inter-freq reporting delay normal mode tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180304 | 0237 | - | New OTDOA Cat M1/M2 inter-freq reporting delay enhanced mode tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180307 | 0240 | - | New ECID Cat M1/M2 tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180308 | 0241 | - | Annex C OTDOA and ECID Cat M1/M2 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180311 | 0244 | - | NB-IOT Annex E | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180325 | 0245 | - | Band 68 addition to band groups in 37.571-1 | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180352 | 0246 | - | [Editorial] Correct normative reference for minimum conformance requirements | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180583 | 0247 | - | feMTC Annex E | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180584 | 0248 | - | 4Rx support for OTDOA 2CC | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-180585 | 0249 | - | 4Rx support for OTDOA 3CC | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181360 | 0250 | 2 | WLAN core specification updates | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181584 | 0235 | 1 | New OTDOA Cat M1/M2 reporting accuracy enhanced mode tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181585 | 0238 | 1 | New OTDOA Cat M1/M2 inter-freq reporting accuracy normal mode tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181586 | 0239 | 1 | New OTDOA Cat M1/M2 inter-freq reporting accuracy enhanced mode tests | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181599 | 0226 | 1 | New OTDOA Cat1bis TC 9.1.1A and 9.1.2A | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181600 | 0227 | 1 | New OTDOA Cat1bis TC 9.2.1A and 9.2.2A | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181605 | 0242 | 1 | NB-IOT General Sections | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181606 | 0243 | 1 | NB-IOT OTDOA Test Cases | 15.0.0 | 15.1.0 |
| 2018-03 | RAN#79 | R5-181613 | 0251 | 1 | Addition of the Band 74 information into TS 37.571-1 | 15.0.0 | 15.1.0 |
| 2018-06 | RAN#80 | R5-182218 | 0256 | - | Common clauses updates for new NB-IOT OTDOA tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182219 | 0257 | - | Annex C updates for NB-IOT OTDOA | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182279 | 0258 | - | New ECID Cat1bis tests | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182280 | 0259 | - | New ECID Cat1bis tests - Annexes | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182382 | 0261 | - | Corrections to WLAN dynamic range test case | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-182384 | 0263 | - | Corrections to maximum response time for WLAN test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183252 | 0252 | 1 | Completion of OTDOA NB-IOT TC 9.5.1 and 9.5.2 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183253 | 0253 | 1 | Completion of OTDOA NB-IOT TC 9.6.1 and 9.6.2 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183261 | 0264 | 1 | Clarifications for RSSI reporting in WLAN and BLE test cases | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183844 | 0254 | 1 | New OTDOA NB-IOT TC 9.5.3 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183845 | 0255 | 1 | New OTDOA NB-IOT TC 9.6.3 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183847 | 0260 | 1 | Removing editor note from A-GNSS min perf test cases for Cat M1 | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183848 | 0262 | 1 | Clarifications and additions to EUTRAN, WLAN and BLE test frequencies and bandwidths | 15.1.0 | 15.2.0 |
| 2018-06 | RAN#80 | R5-183849 | 0265 | 1 | Corrections to WLAN and BLE applicabilities | 15.1.0 | 15.2.0 |
| 2018-09 | RAN#81 | R5-184041 | 0266 | - | Clarifications and corrections to Bluetooth identification test | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-184189 | 0271 | - | Band groups added to specification | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185416 | 0268 | 1 | Correction to nrs-CRS-PowerOffset-r13 for NB-IOT OTDOA tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185417 | 0269 | 1 | NB-IOT OTDOA reporting delay test cases not testable | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185419 | 0270 | 1 | Changes to eMTC OTDOA tests | 15.2.0 | 15.3.0 |
| 2018-09 | RAN#81 | R5-185552 | 0267 | 1 | Applicability of tests for types and Categories of UE | 15.2.0 | 15.3.0 |
| 2018-12 | RAN#82 | R5-186489 | 0272 | - | Resubmission of CR 0269 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186615 | 0274 | - | Clarification of the meaning of A-GPS | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186616 | 0275 | - | Addition of two missing triple-GNSS test cases | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-186617 | 0276 | - | Updates to Table 4B.2-1 | 15.3.0 | 15.4.0 |
| 2018-12 | RAN#82 | R5-187983 | 0277 | 1 | Editorial Changes for TS 37.571-1 | 15.3.0 | 15.4.0 |
| 2019-03 | RAN#83 | R5-191607 | 0281 | - | Editorial Changes for TS 37.571-1 | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192506 | 0278 | 1 | Addition of general NR information | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192646 | 0282 | 1 | Editorial updates for RSTD NB-IOT tests | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192851 | 0279 | 1 | Corrections to RSTD reporting accuracy NB-IOT tests | 15.4.0 | 15.5.0 |
| 2019-03 | RAN#83 | R5-192642 | 0280 | 1 | Band 53 introduction in TS 37-571-1 | 15.5.0 | 16.0.0 |
| 2019-06 | RAN#84 | R5-194099 | 0283 | - | Addition of A-GNSS tests for NR | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194100 | 0284 | - | Updates to Table 4B.2-1 for WLAN and BLE for NR | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-194260 | 0285 | - | 38.509 reference update | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195009 | 0286 | 1 | MBS Tests for NR | 16.0.0 | 16.1.0 |
| 2019-06 | RAN#84 | R5-195450 | 0287 | 2 | Alignment 9.5.3 and 9.6.3 with core spec | 16.0.0 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196604 | 0292 | - | Alignment of 9.5.3 and 9.6.3 to core spec | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197356 | 0294 | - | Missing Bands in Tables | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197357 | 0295 | - | Corrections to GNSS Multi-path Tests | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197358 | 0296 | - | Correct RSTD Test Parameter Values | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197381 | 0288 | 1 | Corrections to RSTD tests for NB-IoT | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197471 | 0289 | 1 | Alignment of NR terminology | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197473 | 0290 | 2 | Add default conditions for FR2 positioning tests for performance | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197477 | 0291 | 1 | Clarify LPP transport for min perf positioning | 16.1.0 | 16.2.0 |
| 2019-09 | RAN#85 | R5-197478 | 0293 | 1 | Correct MBS Applicability Table and Editorial Updates | 16.1.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198063 | 0297 | - | Addition of User Plane testing of A-GNSS to clause 7 for LTE | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198817 | 0302 | - | Editorial Correction to Multi-Path Scenario Test Parameter Table | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198821 | 0303 | - | Add Reference to RRM Specifications for Frequency Band Groups | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-198822 | 0304 | - | Editorial Corrections for RSTD Test Cases | 16.2.0 | 16.3.0 |
| 2019-12 | RAN#86 | R5-199090 | 0300 | 1 | Update to min perf positioning tests | 16.2.0 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201013 | 0305 | 1 | Editorial changes to TS 37.571-X titles to remove references to individual RATs | 16.3.0 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202803 | 0308 | 1 | Update of the Note 1 in the Power level and satellite allocation tables for the Sensitivity Coarse time assistance requirements and tests for E-UTRA and NR. | 16.4.0 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204979 | 0309 | 1 | Clarification on frequency band test conditions for A-GNSS | 16.5.0 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206774 | 0311 | 1 | Addition of BDS B1C Signal test contents in TS 37.571-1 | 16.6.0 | 16.7.0 |
| 2020-12 | RAN#90 | R5-206783 | 0310 | 1 | Definition of A-GNSS Sensitivity testing conditions for EN-DC | 16.6.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210757 | 0318 | - | Update to Annex C for TDD NB-IOT RSTD measurement test cases | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211796 | 0312 | 1 | Addition of TDD NB-IOT RSTD measurement test case 9.7.1 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211797 | 0313 | 1 | Addition of TDD NB-IOT RSTD measurement test case 9.7.2 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211798 | 0314 | 1 | Addition of TDD NB-IOT RSTD measurement test case 9.7.3 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211799 | 0315 | 1 | Addition of TDD NB-IOT RSTD measurement test case 9.8.1 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211800 | 0316 | 1 | Addition of TDD NB-IOT RSTD measurement test case 9.8.2 | 16.7.0 | 16.8.0 |
| 2021-03 | RAN#91 | R5-211801 | 0317 | 1 | Addition of TDD NB-IOT RSTD measurement test case 9.8.3 | 16.7.0 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212200 | 0320 | - | Addition of Derivation of Test Requirements for NB-IOT and CA RSTD tests | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212201 | 0321 | - | Corrections to RSTD accuracy requirements values for UE Category M1 and M2 test cases | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212202 | 0322 | - | Corrections to RSTD accuracy requirements values for NB-IOT test cases | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212234 | 0323 | - | Addition of Derivation of Test Requirements for RSTD and Category M1 and M2 RSTD tests | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212235 | 0324 | - | Correction to Derivation of Test Requirements for UE Rx-Tx tests for Category M1 and M2 UEs | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212237 | 0325 | - | Updates to Correction to Derivation of Test Requirements for UE Rx-Tx tests for Category M1 and M2 UEs | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212238 | 0326 | - | Addition of Test Parameter Relaxations for UE Rx - Tx time difference cases | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212242 | 0327 | - | Clarifications to FDD NB-IoT RSTD test case applicabilities and other corrections | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212244 | 0328 | - | Corrections to RSTD test cases for Category M1 and M2 UEs | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212247 | 0329 | - | Updates to requirements for RSTD test cases for Category M1 and M2 UEs | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-212397 | 0330 | - | Update Release 15 and onwards references for TS 36.355 to TS 37.355 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213134 | 0332 | - | Add OTDOA feMTC test case 9.3.14 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213135 | 0333 | - | Add OTDOA feMTC test case 9.3.15 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213138 | 0336 | - | Add OTDOA feMTC test case 9.3.18 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213139 | 0337 | - | OTDOA feMTC test cases - Annexes | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213992 | 0331 | 1 | Add OTDOA feMTC test case 9.3.13 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213993 | 0334 | 1 | Add OTDOA feMTC test case 9.3.16 | 16.8.0 | 16.9.0 |
| 2021-06 | RAN#92 | R5-213994 | 0335 | 1 | Add OTDOA feMTC test case 9.3.17 | 16.8.0 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215973 | 0338 | 1 | Addition of Multi-RTT, Dl-AoD and DL-TDOA positioning method test conditions | 16.9.0 | 16.10.0 |
| 2021-09 | RAN#93 | R5-215974 | 0340 | 1 | Addition of conditions for NR PRS-based measurements and connection diagrams | 16.9.0 | 16.10.0 |
| 2021-12 | RAN#94 | R5-217133 | 0346 | - | Update number of satellites for multi-GNSS for LTE tests | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-217134 | 0347 | - | Update number of satellites for multi-GNSS for NR tests | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-217183 | 0355 | - | EN-DC frequency bands for testing of A-GNSS sensitivity requirements | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-218283 | 0348 | 1 | Add OTDOA feMTC test case 9.4.13 | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-218284 | 0349 | 1 | Add OTDOA feMTC test case 9.4.14 | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-218285 | 0350 | 1 | Add OTDOA feMTC test case 9.4.15 | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-218286 | 0351 | 1 | Add OTDOA feMTC test case 9.4.16 | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-218287 | 0352 | 1 | Add OTDOA feMTC test case 9.4.17 | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-218288 | 0353 | 1 | Add OTDOA feMTC test case 9.4.18 | 16.10.0 | 16.11.0 |
| 2021-12 | RAN#94 | R5-218289 | 0354 | 1 | OTDOA feMTC test cases - Annexes | 16.10.0 | 16.11.0 |
| 2022-03 | RAN#95 | R5-220518 | 0356 | - | Addition of test cases for UE Rx-Tx time difference measurement period | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-220534 | 0358 | - | Update test applicability to allow for support of limited GNSS combinations for 5G tests | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-220535 | 0359 | - | Update test applicability to allow for support of limited GNSS combinations | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221201 | 0360 | - | Update for the signal conditions for FR2 test cases | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221202 | 0361 | - | New NR RSTD test case 14.2.1 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221203 | 0362 | - | New NR RSTD test case 14.2.2 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221204 | 0363 | - | New NR RSTD test case 14.3.1 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221205 | 0364 | - | New NR RSTD test case 14.3.2 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221206 | 0365 | - | New NR PRS-RSRP test case 16.2.1 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221207 | 0366 | - | New NR PRS-RSRP test case 16.2.2 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221208 | 0367 | - | New NR PRS-RSRP test case 16.3.1 | 16.11.0 | 16.12.0 |
| 2022-03 | RAN#95 | R5-221887 | 0357 | 1 | Addition of UE Rx-Tx time difference measurement test uncertainties and test parameter relaxations | 16.11.0 | 16.12.0 |
| 2022-06 | RAN#96 | R5-222596 | 0368 | - | Addition of measurement period requirements in Multi-RTT test conditions, DL-TDOA test conditions and DL-AoD test conditions | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222597 | 0369 | - | Correction of NR RSTD test cases 14.2.1, 14.2.2, 14.3.1 and 14.3.2 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222598 | 0370 | - | Addition of SRS configuration in UE Rx-Tx time difference measurement period test cases | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222599 | 0371 | - | Addition of new RSTD accuracy test case 14.2.3 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222600 | 0372 | - | Addition of new RSTD accuracy test case 14.2.4 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222601 | 0373 | - | Addition of new RSTD accuracy test case 14.3.3 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222603 | 0375 | - | Addition of new UE Rx-TX time difference accuracy test case 15.3.1 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-222604 | 0376 | - | Addition of new UE Rx-TX time difference accuracy test case 15.3.2 | 16.12.0 | 16.13.0 |
| 2022-06 | RAN#96 | R5-223748 | 0374 | 1 | Addition of new RSTD accuracy test case 14.3.4 | 16.12.0 | 16.13.0 |