| 3GPP TS 38.533 V18.1.0 (2023-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	48

1	Scope	49

2	References	49

3	Definitions, symbols and abbreviations	50

3.1	Definitions	50

3.2	Symbols	51

3.3	Abbreviations	52

3A	Requirements for the support of RRM	54

3A.1	General	54

3A.1.0	Overview of RRM requirements	54

3A.1.1	Test coverage across 5G NR connectivity options	55

3A.2	Requirements Classification for Statistical Testing	55

3A.3	Antenna Configuration	56

3A.4	NR band groups	56

3A.4.0	General	56

3A.4.1	NR operating bands in FR1	56

3A.4.1A	NR operating bands for satellite access in FR1	58

3A.4.2	NR operating bands in FR2	58

3A.5	NR operating band configuration	59

3A.6	UE with Multiband Capability	60

4	EN-DC with all NR cells in FR1	60

4.0	General	60

4.1	Void	61

4.2	Void	61

4.3	RRC_CONNECTED state mobility	61

4.3.1	Void	61

4.3.2	RRC connection mobility control	61

4.3.2.1	Void	61

4.3.2.2	Random access	61

4.3.2.2.1	EN-DC FR1 contention based random access	61

4.3.2.2.2	EN-DC FR1 non-contention based random access	67

4.3.2.2.3	EN-DC FR1 2-step contention based random access	74

4.3.2.2.4	EN-DC FR1 2-step non-contention based random access	79

4.3.2.3	Void	85

4.4	Timing	85

4.4.1	UE transmit timing	85

4.4.1.0	Minimum conformance requirements	85

4.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	85

4.4.1.1	EN-DC FR1 UE transmit timing accuracy	86

4.4.2	UE timer accuracy	92

4.4.3	Timing advance	92

4.4.3.0	Minimum conformance requirements	92

4.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	92

4.4.3.0.2	Minimum conformance requirements for timing advance adjustment delay	93

4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	93

4.5	Signaling characteristics	98

4.5.1	Radio link monitoring	98

4.5.1.00	General	98

4.5.1.0	Minimum conformance requirements	98

4.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	98

4.5.1.0.2	Void	99

4.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	99

4.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	100

4.5.1.0.5	Requirements for UE configured with Relaxed Measurement Criteria	102

4.5.1.1	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	103

4.5.1.2	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	108

4.5.1.3	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	113

4.5.1.4	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	118

4.5.1.5	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	123

4.5.1.6	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	128

4.5.1.7	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	133

4.5.1.9	EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	144

4.5.1.10	149

4.5.1.11	149

4.5.11.1	EN-DC FR1 Conditional PSCell Addition Delay	149

4.5.2	Interruption	154

4.5.2.0	Minimum conformance requirements	154

4.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX.	154

4.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	155

4.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	156

4.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	156

4.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	158

4.5.2.0.6	Minimum conformance requirements for interruptions due to RRM and RLM/BFD measurements on deactivated NR SCG	159

4.5.2.1	EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	159

4.5.2.2	EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	163

4.5.2.3	EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	167

4.5.2.4	EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	173

4.5.2.5	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	179

4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	185

4.5.2.7	189

4.5.2.8	EN-DC FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	190

4.5.2.9	EN-DC FR1 interruptions at E-UTRA SRS carrier based switching	195

4.5.2.10	EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	201

4.5.3	SCell activation and deactivation delay	205

4.5.3.0	Minimum conformance requirements	205

4.5.3.0.2	Fast SCell Activation Delay Requirement for Deactivated SCell	209

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	211

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	222

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX	224

4.5.3.5	EN-DC FR1 direct SCell activation at SCell addition of known SCell	226

4.5.3.6	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	235

4.5.3.7	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	244

4.5.4	UE UL carrier RRC reconfiguration delay	247

4.5.4.1	EN-DC FR1 UE UL carrier RRC reconfiguration delay	247

4.5.5	Link recovery procedures	255

4.5.5.0	Minimum conformance requirements	255

4.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	257

4.5.5.0.4	Requirements for Beam Failure Recovery in SCell	258

4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX	258

4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX	264

4.5.5.3	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	271

4.5.5.4	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX	277

4.5.5.5	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	284

4.5.5.6	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	290

4.5.5.7	EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX	297

4.5.5.8	EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	305

4.5.6	Active BWP switch delay	313

4.5.6.1	DCI-based and time-based active BWP switch	313

4.5.6.1.0	Minimum conformance requirements	313

4.5.6.1.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	316

4.5.6.1.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	323

4.5.6.2	RRC-based active BWP switch	333

4.5.6.2.0	Minimum conformance requirements	333

4.5.6.2.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	333

4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	339

4.5.6.3.0	Minimum conformance requirements	339

4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	341

4.5.6.4	350

4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	350

4.5.6.5.0	Minimum conformance requirements	350

4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	351

4.5.7	PSCell addition and release delay	359

4.5.7.0	Minimum conformance requirements	359

4.5.7.0.1	NR PSCell Addition Delay Requirement	359

4.5.7.0.2	NR PSCell Release Delay Requirement	360

4.5.7.1	EN-DC FR1 addition and release delay of known PSCell	360

4.5.8	UL switching	365

4.5.8.0	Minimum conformance requirements	365

4.5.8.1	EN-DC FR1 interruptions at switching between two uplink carriers	366

4.5.9	UE specific CBW change	375

4.5.9.0	Minimum conformance requirements	375

4.5.9.0.1	Minimum conformance requirements for UE specific CBW change delay	375

4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN-DC	376

4.5.10	PSCell activation and deactivation delay	380

4.5.10.0	Minimum conformance requirements	380

4.5.10.1	EN-DC FR1 PSCell activation and deactivation delay	381

4.5.11	PSCell addition and release delay	388

4.5.11.0	Minimum conformance requirements	388

4.5.11.0.1	Measurement time	389

4.5.11.1	EN-DC FR1 Conditional PSCell Addition Delay	389

4.6	Measurement procedures	394

4.6.1	Intra-frequency measurements	394

4.6.1.0	Minimum conformance requirements	394

4.6.1.0.1	Minimum conformance requirements for event-triggered reporting without gap	394

4.6.1.0.2	Minimum conformance requirements for event-triggered measurements with gap	399

4.6.1.1	EN-DC FR1 event-triggered reporting without gap in non-DRX	401

4.6.1.2	EN-DC FR1 event-triggered reporting without gap in DRX	406

4.6.1.3	EN-DC FR1 event-triggered reporting with gap in non-DRX	409

4.6.1.4	EN-DC FR1 event-triggered reporting with gap in DRX	415

4.6.1.5	EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection	420

4.6.1.6	EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection	423

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	428

4.6.1.8	EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17	432

4.6.2	Inter-frequency measurements	438

4.6.2.0	Minimum conformance requirements for Inter-frequency measurements	438

4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX	441

4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX	446

4.6.2.3	Void	451

4.6.2.4	Void	451

4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	451

4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection	456

4.6.2.7	Void	461

4.6.2.8	Void	461

4.6.2.9	EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	461

4.6.3	Void	466

4.6.4	L1-RSRP measurement for beam reporting	466

4.6.4.0	Minimum conformance requirements	466

4.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	466

4.6.4.1	EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX	470

4.6.4.1.5	Test requirement	473

4.6.4.2	EN-DC FR1 SSB-based L1-RSRP measurement in DRX	474

4.6.4.2.3	Minimum conformance requirements	474

4.6.4.3	EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX	477

4.6.4.4	EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX	481

4.6.4.5	EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	485

4.6.5	CLI measurements	489

4.6.5.0	Minimum conformance requirements	489

4.6.5.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	490

4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX	491

4.6.6	Measurements with autonomous gaps	499

4.6.6.0	Minimum conformance requirements	499

4.6.6.0.1	Minimum conformance requirements for measurements with autonomous gaps	499

4.6.6.0.1.1	Minimum conformance requirements for E-UTRA FDD - NR measurements with autonomous gaps	499

4.6.6.0.1.2	Minimum conformance requirements for E-UTRA TDD - NR measurements with autonomous gaps	500

4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	500

4.6.7	L1-SINR measurement for beam reporting	505

4.6.7.0	Minimum conformance requirements	505

4.6.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	510

4.6.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	516

4.6.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	521

4.7	Measurement performance requirements	525

4.7.1	SS-RSRP	525

4.7.1.0	Minimum conformance requirements	525

4.7.1.1	Intra-frequency measurements	530

4.7.1.2	Inter-frequency measurements	538

4.7.2	SS-RSRQ	545

4.7.2.0	Minimum conformance requirements	545

4.7.2.1	EN-DC FR1 SS-RSRQ measurement accuracy	547

4.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	552

4.7.2.2.1	EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy	552

4.7.2.2.2	EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy	556

4.7.3	SS-SINR	558

4.7.3.0	Minimum conformance requirements	558

4.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	558

4.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	559

4.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	560

4.7.3.1	EN-DC FR1 SS-SINR measurement accuracy	560

4.7.3.2	Inter-Frequency SS-SINR measurement accuracy	564

4.7.3.2.1	EN-DC FR1-FR1 SS-SINR absolute measurement accuracy	564

4.7.3.2.2	EN-DC FR1-FR1 SS-SINR relative measurement accuracy	568

4.7.4	L1-RSRP	570

4.7.4.0	Minimum conformance requirements	570

4.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	570

4.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	573

4.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	574

4.7.4.1	SSB based L1-RSRP measurements	575

4.7.4.1.1	EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy	575

4.7.4.1.2	EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy	580

4.7.4.2	CSI-RS based L1-RSRP measurements	582

4.7.4.2.1	EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy	582

4.7.4.2.2	EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy	586

4.7.5	SFTD	588

4.7.5.0	Minimum conformance requirements	588

4.7.5.0.1	SFTD Accuracy Requirement	588

4.7.5.1	EN-DC FR1 SFTD measurement accuracy	589

4.7.6	CLI measurements	593

4.7.6.0	Minimum conformance requirements	593

4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy	593

4.7.6.0.1.1	SRS-RSRP report mapping	594

4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	595

4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	595

4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	604

4.7.7	L1-SINR measurement for beam reporting	607

4.7.7.0	Minimum conformance requirements	607

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	607

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	610

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	612

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	616

4.7.7.1.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	616

4.7.7.1.2	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	621

4.7.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	623

4.7.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement	628

4.7.7.3.1	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	628

4.7.7.3.2	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	634

4A	NE-DC with all NR cells in FR1	636

4A.0	General	636

4A.1	Signalling characteristics	637

4A.1.1	E-UTRAN PSCell addition	637

4A.1.1.0	Minimum conformance requirements	637

4A.1.1.0.1	E-UTRA PSCell Addition Delay Requirement	637

4A.1.1.0.2	E-UTRA PSCell Release Delay Requirement	637

4A.1.1.1	NE-DC FR1 addition and release delay of known PSCell	637

4A.1.2	Active BWP switch delay	643

4A.1.2.0	Minimum conformance requirements	643

4A.1.2.1	NE-DC FR1 DCI-based and timer-based DL active BWP switch in non-DRX in synchronous NE-DC	643

4A.2	Measurement performance requirements	647

4A.2.1	SFTD accuracy	647

4A.2.1.0	Minimum conformance requirements	647

4A.1.1.0.1	NE-DC SFTD accuracy Requirement	647

4A.2.1.1	NE-DC FR1 SFTD accuracy	649

5	EN-DC with at least one NR cell in FR2	655

5.0	General	655

5.1	Void	655

5.2	Void	655

5.3	RRC_CONNECTED state mobility	655

5.3.1	Void	655

5.3.2	RRC connection mobility control	655

5.3.2.1	Void	655

5.3.2.2	Random access	655

5.3.2.2.1	EN-DC FR2 contention based random access	655

5.3.2.2.2	EN-DC FR2 non-contention based random access	663

5.3.2.2.3	EN-DC FR2 2-step contention based random access	671

5.3.2.2.4	EN-DC FR2 2-step non-contention based random access	675

5.3.2.3	Void	679

5.4	Timing	679

5.4.1	UE transmit timing	679

5.4.1.0	Minimum Conformance Requirements	679

5.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	679

5.4.1.1	EN-DC FR2 UE transmit timing accuracy	681

5.4.2	UE timer accuracy	687

5.4.3	Timing advance	687

5.4.3.0	Minimum conformance requirements	687

5.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	687

5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	687

5.5	Signaling characteristics	692

5.5.1	Radio link monitoring	692

5.5.1.0	Minimum conformance requirements	692

5.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	692

5.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	693

5.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	694

5.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	697

5.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	700

5.5.1.0.6	Requirements for UE configured with Relaxed Measurement Criteria	701

5.5.1.1	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	701

5.5.1.2	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	708

5.5.1.3	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	713

5.5.1.4	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	719

5.5.1.5	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	724

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	729

5.5.1.7	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	734

5.5.1.9	EN-DC FR2 radio link monitoring UE scheduling restrictions	747

5.5.1.10	EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	752

5.5.2	Interruption	757

5.5.2.0	Minimum conformance requirements	757

5.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX	757

5.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	758

5.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	759

5.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	759

5.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	761

5.5.2.1	EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	762

5.5.2.2	EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	766

5.5.2.3	EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	770

5.5.2.4	EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	776

5.5.2.5	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	781

5.5.2.6	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	785

5.5.2.7	EN-DC FR2 interruptions at E-UTRA SRS carrier based switching	789

5.5.2.8	EN-DC FR2 interruptions at NR SRS carrier based switching	795

5.5.3	SCell activation and deactivation delay	801

5.5.3.1	EN-DC FR2 SCell activation and deactivation intra-band in non-DRX	801

5.5.3.2 to 5.5.3.6	809

5.5.3.7	EN-DC FR2 direct SCell activation at SCell addition of known SCell	809

5.5.3.8	EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band	813

5.5.4	UE UL carrier RRC reconfiguration delay	822

5.5.5	Link recovery procedures	822

5.5.5.0	Minimum conformance requirements	822

5.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	822

5.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	826

5.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	831

5.5.5.0.4	Requirements for Beam Failure Recovery in SCell	832

5.5.5.0.5	Requirements for SSB based beam failure detection for UE fulfilling relaxed measurement criteria	832

5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX	833

5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX	840

5.5.5.3	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	847

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX	853

5.5.5.5	EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX	860

5.5.5.6	EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX	865

5.5.5.7	EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX	872

5.5.5.8	EN-DC FR2 CSI-RS-based PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode	878

5.5.5.9	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion	885

5.5.6	Active BWP switch delay	892

5.5.6.1	DCI-based and time-based active BWP switch	892

5.5.6.1.0	Minimum conformance requirements	892

5.5.6.1.1	EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	892

5.5.6.1.2	EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	893

5.5.6.2	RRC-based active BWP switch	893

5.5.6.2.0	Minimum conformance requirements	893

5.5.6.2.1	EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	894

5.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	901

5.5.6.3.0	Minimum conformance requirements	901

5.5.6.3.0.1	Minimum conformance requirements for simultaneous DCI-based and timer-based active BWP switch on multiple CCs	901

5.5.6.3.0.1.1	Minimum conformance requirements for simultaneous DCI-based active BWP switch on multiple CCs	901

5.5.6.3.0.1.2	Minimum conformance requirements for simultaneous timer-based active BWP switch on multiple CCs	903

5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	903

5.5.6.4	SCell dormancy switch	907

5.5.6.4.0	Minimum conformance requirements	907

5.5.6.4.0.1	Minimum conformance requirements for interruptions due to SCell dormancy switch	907

5.5.6.4.0.2	Minimum conformance requirements for interruptions due to CQI measurements during SCell dormancy	908

5.5.6.4.0.3	Minimum conformance requirements for interruptions due to RRM measurements during SCell dormancy	908

5.5.6.4.0.4	Minimum conformance requirements for DCI and timer-based BWP switch delay on a single CC	908

5.5.6.4.0.5	Minimum conformance requirements for simultaneous DCI based BWP switch delay on multiple CCs	909

5.5.6.4.0.6	Minimum conformance requirements for interruptions due to NR SCell dormancy switch	911

5.5.6.4.0.7	Minimum conformance requirements for interruptions due to CSI and RRM measurements during SCell dormancy	911

5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 Scell inside active time	911

5.5.7	Void	915

5.5.8	Active TCI state switch delay	915

5.5.8.0	Minimum conformance requirements	915

5.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	915

5.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	916

5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch	917

5.5.8.2	EN-DC FR2 RRC based active TCI state switch	921

5.5.9	Uplink spatial relation switch delay	925

5.5.9.0	Minimum conformance requirements	925

5.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	925

5.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	925

5.5.9.1.1	EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation	926

5.5.9.2	RRC based uplink spatial relation switch	930

5.5.9.2.1	EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS	930

5.5.10	UE specific CBW change	934

5.5.10.0	Minimum conformance requirements	934

5.5.10.0.1	Minimum conformance requirements for UE specific CBW change	934

5.5.11	Unified TCI state switch delay	939

5.5.11.0	Minimum conformance requirements	939

5.5.11.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	939

5.5.11.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	941

5.5.11.1	EN-DC FR2 MAC-CE based active joint TCI state switch	942

5.5.11.2	EN-DC FR2 MAC-CE based active uplink TCI state switch	947

5.5.11.3	EN-DC FR2 MAC-CE based active downlink TCI state switch	951

5.5.12	PSCell activation and deactivation delay	955

5.5.12.0	Minimum conformance requirements	955

5.5.12.1	EN-DC FR2 PSCell activation and deactivation delay	956

5.5.13	Conditional PSCell addition and release delay	960

5.5.13.1	EN-DC FR2 Addition and Release Delay of NR PSCell	960

5.6	Measurement procedures	965

5.6.1	Intra-frequency measurements	965

5.6.1.0	Minimum conformance requirements	965

5.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	965

5.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	968

5.6.1.1	EN-DC FR2 event-triggered reporting without gap in non-DRX	969

5.6.1.2	EN-DC FR2 event-triggered reporting without gap in DRX	975

5.6.1.3	EN-DC FR2 event-triggered reporting with gap in non-DRX	979

5.6.1.4	EN-DC FR2 event-triggered reporting with gap in DRX	985

5.6.2	Inter-frequency measurements	990

5.6.2.0	Minimum conformance requirements for Inter-frequency measurements	990

5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX	992

5.6.2.2	EN-DC FR2-FR2 event-triggered reporting in DRX	996

5.6.2.3	EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	1001

5.6.2.4	EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection	1005

5.6.2.5	EN-DC FR1-FR2 event-triggered reporting in non-DRX	1010

5.6.2.6	EN-DC FR1-FR2 event-triggered reporting in DRX	1015

5.6.2.7	EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	1021

5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection	1026

5.6.3	L1-RSRP measurement for beam reporting	1031

5.6.3.0	Minimum conformance requirements	1031

5.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1031

5.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1033

5.6.3.1	EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX	1036

5.6.3.2	EN-DC FR2 SSB-based L1-RSRP measurement in DRX	1040

5.6.3.3	EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX	1044

5.6.3.4	EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX	1048

5.6.4	CLI measurements	1051

5.6.4.0	Minimum conformance requirements	1051

5.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	1052

5.6.4.1	EN-DC FR2 SRS-RSRP measurement in non-DRX	1053

5.6.4.2	EN-DC FR2 CLI-RSSI measurement in non-DRX	1056

5.6.5	Measurements with autonomous gaps	1060

5.6.5.0	Minimum conformance requirements	1060

5.6.5.0.1	Minimum conformance requirements for EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1060

5.6.5.0.1.1	Introduction	1060

5.6.5.0.1.2	CGI identification of an NR cell with autonomous gaps	1060

5.6.5.0.1.3	CGI reporting delay	1061

5.6.5.1	EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1061

5.6.6	L1-SINR measurement for beam reporting	1066

5.6.6.0	Minimum conformance requirements	1066

5.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1066

5.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1069

5.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1071

5.6.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1074

5.6.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1078

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1084

5.7	Measurement performance requirements	1089

5.7.1	SS-RSRP	1089

5.7.1.0	Minimum conformance requirements	1089

5.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	1089

5.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	1090

5.7.1.1	EN-DC FR2 SS-RSRP measurement accuracy	1092

5.7.1.2	EN-DC FR2-FR2 SS-RSRP measurement accuracy	1099

5.7.1.3	EN-DC FR1-FR2 SS-RSRP measurement accuracy	1106

5.7.2	SS-RSRQ	1112

5.7.2.0	Minimum conformance requirements	1112

5.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1112

5.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	1112

5.7.2.1	EN-DC FR2 SS-RSRQ measurement accuracy	1114

5.7.2.2	EN-DC FR2-FR2 SS-RSRQ measurement accuracy	1118

5.7.3	SS-SINR	1123

5.7.3.0	Minimum conformance requirements	1123

5.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1123

5.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	1123

5.7.3.1	EN-DC FR2 SS-SINR measurement accuracy	1125

5.7.3.2	EN-DC FR2-FR2 SS-SINR measurement accuracy	1129

5.7.4	L1-RSRP	1134

5.7.4.0	Minimum conformance requirements	1134

5.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	1135

5.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	1135

5.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	1136

5.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	1137

5.7.4.1	EN-DC FR2 SSB based L1-RSRP measurement accuracy	1138

5.7.4.2	EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy	1144

5.7.5	SRS-RSRP	1150

5.7.5.0	Minimum conformance requirements	1150

5.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	1150

5.7.5.1	EN-DC FR2 SRS-RSRP measurement accuracy	1152

5.7.5.2	EN-DC FR2 CLI-RSSI measurement accuracy	1157

5.7.5.2.1	Test purpose	1158

5.7.5.2.2	Test applicability	1158

5.7.5.2.3	Minimum conformance requirements	1158

5.7.5.2.4	Test description	1158

5.7.5.2.5	Test requirement	1159

5.7.6	L1-SINR measurement for beam reporting	1162

5.7.6.0	Minimum conformance requirements	1162

5.7.6.0.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1162

5.7.6.0.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1163

5.7.6.0.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1166

5.7.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	1168

5.7.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1173

5.7.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1178

6	NR standalone in FR1	1184

6.0	General	1184

6.1	RRC_IDLE state mobility	1184

6.1.1	NR cell re-selection	1184

6.1.1.0	Minimum conformance requirements	1184

6.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	1184

6.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	1184

6.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with highSpeedMeasFlag-r16	1187

6.1.1.0.4	Minimum conformance requirements for intra-frequency cell re-selection when UE configured with relaxed measurement criterion	1187

6.1.1.0.5	Minimum conformance requirements for inter-frequency cell re-selection when UE configured with relaxed measurement criterion	1189

6.1.1.1	NR SA FR1 cell re-selection	1191

6.1.1.1.5	Test requirement	1195

6.1.1.2	NR SA FR1-FR1 cell re-selection	1197

6.1.1.3	NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1201

6.1.1.4	NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1208

6.1.1.5	NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1213

6.1.1.6	NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1220

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16	1226

6.1.1.8	NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17	1231

6.1.2	NR – E-UTRA cell re-selection	1237

6.1.2.0	Minimum conformance requirements	1237

6.1.2.0.1	Minimum conformance requirements for NR – E-UTRA cell re-selection	1237

6.1.2.0.2	Minimum conformance requirement for inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	1239

6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA	1240

6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA	1246

6.1.2.3	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1252

6.1.2.4	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1258

6.1.2.5	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16	1263

6.2	RRC_INACTIVE state mobility	1269

6.2.0	General	1269

6.2.0.1	Minimum conformance requirements for TA validation using CG-SDT	1269

6.2.1	NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)	1270

6.2.1.1	Test Purpose	1270

6.2.1.2	Test Applicability	1270

6.2.1.3	Minimum Conformance Requirement	1270

6.2.1.4	Test Description	1270

6.2.1.4.1	Initial conditions	1271

6.2.1.4.2	Test Procedure	1273

6.2.1.5	Test Requirement	1275

6.3	RRC_CONNECTED state mobility	1276

6.3.1	Handover	1276

6.3.1.0	Minimum conformance requirements	1276

6.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	1276

6.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	1276

6.3.1.0.3	Minimum conformance requirements for NR – UTRAN handover	1277

6.3.1.0.4	Minimum conformance requirements for NR FR1- NR FR1 DAPS handover	1278

6.3.1.1	NR SA FR1 handover with known target cell	1280

6.3.1.2	NR SA FR1 handover with unknown target cell	1284

6.3.1.3	NR SA FR1-FR1 handover with unknown target cell	1287

6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell	1290

6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell	1298

6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell	1303

6.3.1.7	NR SA FR1 synchronous DAPS handover	1309

6.3.1.8	NR SA FR1 asynchronous DAPS handover	1317

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover	1322

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover	1331

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover	1337

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover	1346

6.3.2	RRC connection mobility control	1354

6.3.2.1	RRC re-establishment	1354

6.3.2.1.0	Minimum conformance requirements	1354

6.3.2.1.1	NR SA FR1 RRC re-establishment	1356

6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment	1360

6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing	1364

6.3.2.2	Random access	1368

6.3.2.2.0	Minimum conformance requirements	1368

6.3.2.2.1	NR SA FR1 contention based random access	1370

6.3.2.2.2	NR SA FR1 non-contention based random access	1376

6.3.2.2.3	NR SA FR1 2-step contention based random access	1383

6.3.2.2.4	NR SA FR1 2-step non-contention based random access	1387

6.3.2.3	RRC connection release with redirection	1391

6.3.2.3.0	Minimum conformance requirements	1391

6.3.2.3.1	NR SA FR1 RRC connection release with redirection	1393

6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection	1396

6.3.3	Conditional handover	1401

6.3.3.0	Minimum conformance requirements	1401

6.3.3.0.1	Minimum conformance requirements for NR FR1 intra-frequency conditional handover	1401

6.3.3.0.2	Minimum conformance requirements for NR FR1 inter-frequency conditional handover	1404

6.3.3.1	NR SA FR1 conditional handover	1407

6.3.3.2	NR SA FR1-FR1 conditional handover	1413

6.4	Timing	1422

6.4.1	UE transmit timing	1422

6.4.1.0	Minimum conformance requirements	1422

6.4.1.1	NR SA FR1 UE transmit timing accuracy	1423

6.4.2	UE timer accuracy	1429

6.4.3	Timing advance	1429

6.4.3.0	Minimum conformance requirement	1429

6.4.3.0.1	Minimum conformance requirement for timing advance adjustment	1429

6.4.3.1	NR SA FR1 timing advance adjustment accuracy	1429

6.5	Signaling characteristics	1434

6.5.1	Radio link monitoring	1434

6.5.1.00	General	1434

6.5.1.0	Minimum conformance requirements	1436

6.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	1436

6.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	1437

6.5.1.0.3	Minimum conformance requirements for out-of-sync and in-sync CSI-RS based RLM	1439

6.5.1.0.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	1440

6.5.1.0.5	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	1440

6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1441

6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1447

6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	1453

6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	1458

6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1465

6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1472

6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1478

6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1485

6.5.1.9	SA FR1 radio link monitoring out-of-sync Test for PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	1491

6.5.1.9.1	Test purpose	1491

6.5.1.9.2	Test applicability	1492

6.5.1.9.3	Minimum conformance requirements	1492

6.5.1.9.4	Test description	1492

6.5.1.9.5	Test requirement	1497

6.5.2	Interruption	1499

6.5.2.0	Minimum conformance requirements	1499

6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	1499

6.5.2.0.2	 Interruptions at NR SRS carrier based switching	1500

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC	1502

6.5.2.2	SA FR1 interruptions at NR SRS carrier based switching	1507

6.5.3	SCell activation and deactivation delay	1512

6.5.3.0	Minimum conformance requirements	1512

6.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	1512

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	1513

6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	1526

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX	1527

6.5.3.4	NR SA FR1 direct SCell activation at SCell addition of known SCell	1529

6.5.3.5	NR SA FR1 direct SCell activation at handover with known SCell	1538

6.5.3.6 to 6.5.3.9	1547

6.5.3.10	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	1547

6.5.3.11	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	1555

6.5.4	UE UL carrier RRC reconfiguration delay	1558

6.5.4.0	Minimum conformance requirements	1558

6.5.4.0.1	Minimum conformance requirements for UL carrier RRC reconfiguration delay	1558

6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay	1558

6.5.5	Link recovery procedures	1566

6.5.5.0	Minimum conformance requirements	1566

6.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	1566

6.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	1566

6.5.5.0.4	Requirements for Beam Failure Recovery in SCell	1566

6.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	1566

6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX	1568

6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX	1576

6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	1584

6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX	1591

6.5.5.5	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	1598

6.5.5.6	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	1606

6.5.5.7	NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	1613

6.5.6	Active BWP switch delay	1621

6.5.6.1	DCI-based and time-based active BWP switch	1621

6.5.6.1.0	Minimum conformance requirements	1621

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX	1622

6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX	1633

6.5.6.2	RRC-based active BWP switch	1642

6.5.6.2.0	Minimum conformance requirements	1642

6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX	1642

6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1650

6.5.6.3.0	Minimum conformance requirements	1650

6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1650

6.5.6.4	1660

6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1660

6.5.6.5.0	Minimum conformance requirements	1660

6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch in non-DRX on multiple CCs	1661

6.5.7	DL interruptions at switching between two uplink carriers	1668

6.5.7.0	Minimum conformance requirements	1668

6.5.7.1	NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA	1668

6.5.7.2	NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA	1680

6.5.7A	NR SA FR1 DL interruptions at switching between two uplink carriers with two transmit antenna connectors	1688

6.5.7A.0	Minimum conformance requirements	1688

6.5.7A.1	Void	1689

6.5.7A.2	NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA	1689

6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	1697

6.5.7B.0	Minimum conformance requirements	1697

6.5.7B.1	Void	1698

6.5.7B.2	NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA	1698

6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	1707

6.5.7C.0	Minimum conformance requirements	1707

6.5.7C.1	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	1708

6.5.7C.1.1	Test purpose	1708

6.5.7C.1.2	Test applicability	1708

6.5.7C.1.3	Minimum conformance requirements	1708

6.5.7C.1.4	Test description	1708

6.5.7C.1.4.1	Initial conditions	1708

6.5.7C.2	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	1719

6.5.7C.2.1	Test purpose	1719

6.5.7C.2.2	Test applicability	1719

6.5.7C.2.3	Minimum conformance requirements	1719

6.5.7C.2.4	Test description	1720

6.5.7C.2.4.1	Initial conditions	1720

6.5.8	UE specific CBW change	1731

6.5.8.0	Minimum conformance requirements	1731

6.5.8.0.1	Minimum conformance requirements for UE specific CBW change	1731

6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1731

6.6	Measurement procedures	1738

6.6.1	Intra-frequency measurements	1738

6.6.1.0	Minimum conformance requirements	1738

6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1738

6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1738

6.6.1.0.3	Void	1739

6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading	1739

6.6.1.0.5	 Void	1741

6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX	1741

6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX	1745

6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX	1750

6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX	1756

6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading	1761

6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading	1765

6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	1769

6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17	1774

6.6.2	Inter-frequency measurements	1779

6.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1779

6.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps	1779

6.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps	1779

6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX	1782

6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX	1788

6.6.2.3	Void	1794

6.6.2.4	Void	1794

6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	1794

6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection	1800

6.6.2.7	Void	1806

6.6.2.8	Void	1806

6.6.2.9	NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern	1806

6.6.2.10	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used	1811

6.6.2.11	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used	1817

6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	1823

6.6.3	Inter-RAT Measurements	1829

6.6.3.0	Minimum conformance requirements	1829

6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	1829

6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	1831

6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX	1834

6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX	1841

6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16	1847

6.6.4	L1-RSRP measurement for beam reporting	1855

6.6.4.0	Minimum conformance requirements	1855

6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1855

6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1855

6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX	1855

6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX	1859

6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX	1863

6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX	1867

6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	1870

6.6.5	UTRAN inter-RAT measurement	1876

6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX	1876

6.6.6	CLI Measurements	1883

6.6.6.0	Minimum conformance requirements	1883

6.6.6.0.1	Minimum conformance requirements for SRS-RSRP measurement	1883

6.6.6.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	1883

6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX	1883

6.6.7	NR measurements with autonomous gaps	1890

6.6.7.0	Minimum conformance requirements	1890

6.6.7.0.1	Minimum conformance requirements for SA intra-frequency CGI identification of NR neighbour cell in FR1	1890

6.6.7.0.1.1	Introduction	1890

6.6.7.0.1.2	CGI identification of an NR cell with autonomous gaps	1890

6.6.7.0.1.3	CGI reporting delay	1891

6.6.7.0.2	Minimum conformance requirements for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1891

6.6.7.0.2.1	CGI identification of an E-UTRA cell with autonomous gaps	1891

6.6.7.0.2.2	CGI reporting delay	1892

6.6.7.1	SA intra-frequency CGI identification of NR neighbour cell in FR1	1892

6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1896

6.6.8	L1-SINR measurement for beam reporting	1900

6.6.8.0	Minimum conformance requirements	1900

6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1900

6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1900

6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1900

6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1900

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1906

6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1912

6.6.9	Idle Mode CA/DC Measurements	1918

6.6.9.0	Minimum conformance requirements	1918

6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1	1920

6.6.10 to 6.6.14	1930

6.6.15	Idle Mode inter-RAT CA/DC Measurements	1930

6.6.15.0	Minimum conformance requirements	1930

6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1931

6.6.16	1944

6.6.17	NR SA FR1 event triggered reporting tests with Pre-MG	1944

6.6.17.1	NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG	1946

6.6.17.2	NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	1952

6.6.18	SA event triggered reporting tests with concurrent gaps	1960

6.6.18.0	Minimum conformance requirements	1960

6.6.18.0.1	Minimum conformance requirements for Intra-frequency measurement	1960

6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement	1961

6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement	1962

6.6.18.0.4	Minimum conformance requirements for PRS measurement	1963

6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers	1963

6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers	1971

6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	1978

6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	1990

6.6.19	SA event triggered reporting tests with NCSG	1999

6.6.19.0	Minimum conformance requirements	1999

6.6.19.0.1	Minimum conformance requirements for intra-frequency measurement	1999

6.6.19.0.2	Minimum conformance requirements for inter-frequency measurement	2001

6.6.19.0.3	Minimum conformance requirements for inter-RAT measurement	2002

6.6.19.0.4	Minimum conformance requirements for interruptions	2003

6.6.19.1	NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1	2004

6.6.19.2	NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2010

6.6.19.3	NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2016

6.6.19.4	NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2024

6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2030

6.6.20.0	Minimum conformance requirements	2030

6.6.20.1	NR SA FR1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC	2033

6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS	2037

6.6.21.0	Minimum conformance requirements	2037

6.6.21.1	NR SA FR1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC	2040

6.7	Measurement performance requirements	2044

6.7.1	SS-RSRP	2044

6.7.1.0	Minimum conformance requirements	2044

6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	2044

6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	2044

6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	2044

6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	2045

6.7.1.1	Intra-frequency measurements	2045

6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy	2045

6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy	2052

6.7.1.2	Inter-frequency measurements	2054

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy	2054

6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy	2061

6.7.2	SS-RSRQ	2063

6.7.2.0	Minimum conformance requirements	2063

6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2063

6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	2063

6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	2063

6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy	2063

6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	2067

6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy	2067

6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy	2072

6.7.3	SS-SINR	2074

6.7.3.0	Minimum conformance requirements	2074

6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2074

6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	2074

6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	2074

6.7.3.1	NR SA FR1 SS-SINR measurement accuracy	2074

6.7.3.2	Inter-Frequency SS-SINR measurement accuracy	2078

6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy	2078

6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy	2082

6.7.4	L1-RSRP	2084

6.7.4.0	Minimum conformance requirements	2084

6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	2084

6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	2084

6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	2084

6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	2084

6.7.4.1	SSB based L1-RSRP measurements	2084

6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy	2084

6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy	2089

6.7.4.2	CSI-RS based L1-RSRP measurements	2091

6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy	2091

6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy	2095

6.7.5	E-UTRAN RSRP	2097

6.7.5.0	Minimum conformance requirements	2097

6.7.5.0.1	E-UTRAN RSRP absolute accuracy	2097

6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy	2098

6.7.6	E-UTRAN RSRQ	2105

6.7.6.0	Minimum conformance requirements	2105

6.7.6.0.1	E-UTRAN RSRQ absolute accuracy	2105

6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy	2106

6.7.7	E-UTRAN RS-SINR	2113

6.7.7.0	Minimum conformance requirements	2113

6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy	2113

6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy	2114

6.7.8	CLI Measurements	2121

6.7.8.0	Minimum conformance requirements	2121

6.7.8.0.1	Minimum conformance requirements for SRS-RSRP accuracy	2121

6.7.8.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	2121

6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy	2121

6.7.8.1.1	Test purpose	2121

6.7.8.1.2	Test applicability	2121

6.7.8.1.3	Minimum conformance requirements	2122

6.7.8.1.4	Test description	2122

6.7.8.1.5	Test requirement	2124

6.7.9	L1-SINR measurement for beam reporting	2131

6.7.9.0	Minimum conformance requirements	2131

6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2131

6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	2134

6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	2136

6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	2140

6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	2140

6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	2143

6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2145

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2148

6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2149

6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	2152

7	NR standalone with at least one NR cell in FR2	2155

7.0	General	2155

7.1	RRC_IDLE state mobility	2155

7.1.1	NR cell re-selection	2155

7.1.1.0	Minimum conformance requirements	2155

7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	2155

7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	2158

7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion	2159

7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion	2160

7.1.1.1	NR SA FR2 cell re-selection	2162

7.1.1.2	NR SA FR2-FR2 cell re-selection	2167

7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2174

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2179

7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2184

7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2191

7.2	RRC_INACTIVE state mobility	2203

7.2.1	Small Data Transmission	2203

7.2.1.0	Minimum conformance requirements	2203

7.2.1.0.1	Minimum conformance requirements for CG-SDT TA validation	2203

7.2.1.1	TA Validation for CG-SDT in FR2	2204

7.3	RRC_CONNECTED state mobility	2213

7.3.1	Handover	2213

7.3.1.0	Minimum conformance requirements	2213

7.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	2213

7.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR2 handover	2214

7.3.1.0.3	Minimum conformance requirements for NR FR1 – NR FR2 DAPS handover	2215

7.3.1.1	NR SA FR1-FR2 Inter-frequency handover; unknown target cell	2216

7.3.1.2	NR SA FR2 Intra-frequency handover; unknown target cell	2221

7.3.1.3	NR SA FR2-FR2 Inter-frequency handover; unknown target cell	2225

7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover	2229

7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover	2239

7.3.2	RRC connection mobility control	2246

7.3.2.1	RRC re-establishment	2246

7.3.2.1.0	Minimum conformance requirements	2246

7.3.2.1.1	NR SA FR2 RRC re-establishment	2247

7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment	2251

7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	2254

7.3.2.2	Random access	2259

7.3.2.2.0	Minimum conformance requirements	2259

7.3.2.2.1	NR SA FR2 contention based random access	2260

7.3.2.2.2	NR SA FR2 non-contention based random access	2268

7.3.2.2.3	NR SA FR2 2-step contention based random access	2275

7.3.2.2.4	NR SA FR2 2-step non-contention based random access	2282

7.3.2.3	RRC connection release with redirection	2289

7.3.2.3.0	Minimum conformance requirements	2289

7.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection	2290

7.3.3	Conditional handover	2294

7.3.3.0	Minimum conformance requirements	2294

7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover	2294

7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover	2298

7.3.3.1	NR SA FR2 conditional handover	2302

7.3.3.2	NR SA FR2-FR2 conditional handover	2309

7.4	Timing	2316

7.4.1	UE transmit timing	2316

7.4.1.0	Minimum Conformance Requirements	2316

7.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	2316

7.4.1.1	NR SA FR2 UE transmit timing accuracy	2317

7.4.2	UE timer accuracy	2323

7.4.3	Timing advance	2323

7.4.3.0	Minimum conformance requirements	2323

7.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	2323

7.4.3.1	NR SA FR2 timing advance adjustment accuracy	2324

7.5	Signalling characteristics	2330

7.5.1	Radio link monitoring	2330

7.5.1.0	Minimum conformance requirements	2330

7.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	2330

7.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	2330

7.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	2330

7.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	2330

7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	2330

7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2330

7.5.1.1.1	Test purpose	2331

7.5.1.1.2	Test applicability	2331

7.5.1.1.3	Minimum conformance requirement	2331

7.5.1.1.4	Test description	2331

7.5.1.1.5	Test Requirement	2335

7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2336

7.5.1.2.1	Test purpose	2336

7.5.1.2.2	Test applicability	2336

7.5.1.2.3	Minimum conformance requirement	2336

7.5.1.2.4	Test description	2336

7.5.1.2.5	Test Requirement	2342

7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2343

7.5.1.3.1	Test purpose	2343

7.5.1.3.2	Test applicability	2343

7.5.1.3.3	Minimum conformance requirement	2343

7.5.1.3.4	Test description	2343

7.5.1.3.5	Test Requirement	2347

7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2348

7.5.1.4.1	Test purpose	2348

7.5.1.4.2	Test applicability	2348

7.5.1.4.3	Minimum conformance requirement	2348

7.5.1.4.4	Test description	2348

7.5.1.4.5	Test Requirement	2354

7.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	2355

7.5.1.5.1	Test purpose	2355

7.5.1.5.2	Test applicability	2355

7.5.1.5.3	Minimum conformance requirement	2355

7.5.1.5.4	Test description	2355

7.5.1.5.5	Test Requirement	2359

7.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2360

7.5.1.6.1	Test purpose	2361

7.5.1.6.2	Test applicability	2361

7.5.1.6.3	Minimum conformance requirement	2361

7.5.1.6.4	Test description	2361

7.5.1.6.5	Test Requirement	2366

7.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM RS in DRX mode	2367

7.5.1.7.1	Test purpose	2367

7.5.1.7.2	Test applicability	2367

7.5.1.7.3	Minimum conformance requirement	2367

7.5.1.7.4	Test description	2367

7.5.1.7.5	Test Requirement	2371

7.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2372

7.5.1.8.1	Test purpose	2372

7.5.1.8.2	Test applicability	2372

7.5.1.8.3	Minimum conformance requirement	2372

7.5.1.8.4	Test description	2372

7.5.1.8.5	Test Requirement	2377

7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	2378

7.5.1.9.1	Test purpose	2379

7.5.1.9.2	Test applicability	2379

7.5.1.9.3	Minimum conformance requirement	2379

7.5.1.9.4	Test description	2379

7.5.1.9.5	Test Requirement	2381

7.5.2	Interruption	2383

7.5.2.0	Minimum conformance requirements	2383

7.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	2383

7.5.2.0.2	Minimum conformance requirements for SA interruptions at NR SRS carrier-based switching	2384

7.5.2.1	NR SA FR2 interruptions during measurements on deactivated NR SCC	2386

7.5.2.1.1	Test purpose	2386

7.5.2.1.2	Test applicability	2386

7.5.2.1.3	Minimum conformance requirements	2386

7.5.2.1.4	Test description	2386

7.5.2.1.5	Test requirement	2388

7.5.2.2	SA interruptions at NR SRS carrier-based switching	2391

7.5.2.2.1	Test purpose	2391

7.5.2.2.2	Test applicability	2391

7.5.2.2.3	Minimum conformance requirements	2391

7.5.2.2.4	Test description	2391

7.5.2.2.5	Test requirement	2392

7.5.3	SCell activation and deactivation delay	2394

7.5.3.0	Minimum conformance requirements	2394

7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell	2394

7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell	2397

7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay	2397

7.5.3.1.1	Test purpose	2398

7.5.3.1.2	Test applicability	2398

7.5.3.1.3	Minimum conformance requirements	2398

7.5.3.1.4	Test description	2398

7.5.3.1.5	Test requirement	2399

7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay	2399

7.5.3.2.1	Test purpose	2400

7.5.3.2.2	Test applicability	2400

7.5.3.2.3	Minimum conformance requirements	2400

7.5.3.2.4	Test description	2400

7.5.3.2.5	Test requirement	2401

7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	2402

7.5.3.3.1	Test purpose	2402

7.5.3.3.2	Test applicability	2402

7.5.3.3.3	Minimum conformance requirements	2402

7.5.3.3.4	Test description	2402

7.5.3.3.5	Test requirement	2407

7.5.3.4	NR SA FR2 direct SCell activation at SCell addition of known SCell	2410

7.5.3.4.1	Test Purpose	2410

7.5.3.4.2	Test applicability	2410

7.5.3.4.3	Minimum conformance requirements	2410

7.5.3.4.4	Test description	2410

7.5.3.4.5	Test requirement	2415

7.5.3.5	NR SA FR2 direct SCell activation at handover with known SCell	2417

7.5.3.5.1	Test Purpose	2417

7.5.3.5.2	Test applicability	2417

7.5.3.5.3	Minimum conformance requirements	2417

7.5.3.5.4	Test description	2417

7.5.3.5.5	Test requirement	2422

7.5.3.6 to 7.5.3.12	2425

7.5.3.13	NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX	2425

7.5.3.13.1	Test Purpose	2425

7.5.3.13.2	Test applicability	2425

7.5.3.13.3	Minimum conformance requirements	2425

7.5.3.13.4	Test description	2425

7.5.3.13.5	Test requirement	2430

7.5.3.14	NR SA FR2 SCell Activation for known SCell in FR2 inter-band	2432

-	This test case is incomplete for UE power class other than PC3.7.5.3.14.1 Test purpose	2433

7.5.3.14.2	Test applicability	2433

7.5.3.14.3	Minimum conformance requirements	2433

7.5.3.14.4	Test description	2433

7.5.3.14.5	Test requirement	2438

7.5.4	Void	2439

7.5.5	Link recovery procedures	2439

7.5.5.0	Minimum conformance requirements	2439

7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	2439

7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	2439

7.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	2440

7.5.5.0.4	Requirements for Beam Failure Recovery in SCell	2440

7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX	2440

7.5.5.1.1	Test purpose	2440

7.5.5.1.2	Test applicability	2440

7.5.5.1.3	Minimum conformance requirements	2440

7.5.5.1.4	Test description	2440

7.5.5.1.5	Test requirement	2444

7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX	2445

7.5.5.2.1	Test purpose	2446

7.5.5.2.2	Test applicability	2446

7.5.5.2.3	Minimum conformance requirements	2446

7.5.5.2.4	Test description	2446

7.5.5.2.5	Test requirement	2450

7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	2452

7.5.5.3.1	Test purpose	2452

7.5.5.3.2	Test applicability	2452

7.5.5.3.3	Minimum conformance requirements	2452

7.5.5.3.4	Test description	2452

7.5.5.3.5	Test requirement	2456

7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX	2457

7.5.5.4.1	Test purpose	2458

7.5.5.4.2	Test applicability	2458

7.5.5.4.3	Minimum conformance requirements	2458

7.5.5.4.4	Test description	2458

7.5.5.4.5	Test requirement	2462

7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX	2464

7.5.5.5.1	Test purpose	2464

7.5.5.5.2	Test applicability	2464

7.5.5.5.3	Minimum conformance requirements	2464

7.5.5.5.4	Test description	2464

7.5.5.5.5	Test requirement	2468

7.5.5.6	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX	2469

7.5.5.6.1	Test purpose	2469

7.5.5.6.2	Test applicability	2470

7.5.5.6.3	Minimum conformance requirements	2470

7.5.5.6.4	Test description	2470

7.5.5.6.5	Test requirement	2474

7.5.5.7	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX	2476

7.5.5.7.1	Test purpose	2476

7.5.5.7.2	Test applicability	2476

7.5.5.7.3	Minimum conformance requirements	2476

7.5.5.7.4	Test description	2476

7.5.5.7.5	Test requirement	2480

7.5.5.8	FFS	2482

7.5.5.9	NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	2482

7.5.5.9.1	Test purpose	2482

7.5.5.9.2	Test applicability	2482

7.5.5.9.3	Minimum conformance requirements	2482

7.5.5.9.4	Test description	2482

7.5.5.9.5	Test requirements	2487

7.5.5.10	NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX	2488

7.5.5.10.1	Test purpose	2488

7.5.5.10.2	Test applicability	2488

7.5.5.10.3	Minimum conformance requirements	2488

7.5.5.10.4	Test description	2488

7.5.5.10.5	Test requirement	2493

7.5.6	Active BWP switch delay	2495

7.5.6.1	DCI-based and time-based active BWP switch	2495

7.5.6.1.0	Minimum conformance requirements	2495

7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2495

7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX	2496

7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2497

7.5.6.2	RRC-based active BWP switch	2498

7.5.6.2.0	Minimum conformance requirements	2498

7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX	2499

7.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	2501

7.5.6.3.0	Minimum conformance requirements	2501

7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	2504

7.5.7	PSCell addition and release delay	2508

7.5.7.0	Minimum conformance requirements	2508

7.5.7.0.1	Minimum conformance requirements for PSCell addition delay	2508

7.5.7.0.2	Minimum conformance requirements for PSCell release delay	2509

7.5.7.1	NR SA FR2 addition and release delay of known PSCell	2509

7.5.7.1.1	Test purpose	2510

7.5.7.1.2	Test applicability	2510

7.5.7.1.3	Minimum conformance requirements	2510

7.5.7.1.4	Test description	2510

7.5.7.1.5	Test requirement	2511

7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell	2511

7.5.7.2.1	Test purpose	2512

7.5.7.2.2	Test applicability	2512

7.5.7.2.3	Minimum conformance requirements	2512

7.5.7.2.4	Test description	2512

7.5.7.2.5	Test requirement	2513

7.5.8	Active TCI state switch delay	2513

7.5.8.0	Minimum conformance requirements	2513

7.5.8.0.0	Known conditions for TCI state	2513

7.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	2514

7.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	2514

7.5.8.0.3 	Minimum conformance requirements for MAC-CE based active TCI state switch in HST FR2	2515

7.5.8.1	NR SA FR2 MAC-CE based active TCI state switch	2516

7.5.8.1.1	NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state	2516

7.5.8.2	NR SA FR2 RRC based active TCI state switch	2522

7.5.8.2.1	NR SA Pcell FR2 RRC based active TCI state switch for a known TCI state	2522

7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	2529

7.5.8.3.1	NR SA FR2 HST active TCI state switch for a known TCI state	2529

7.5.9	Uplink spatial relation switch delay	2535

7.5.9.0	Minimum conformance requirements	2535

7.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	2535

7.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	2535

7.5.9.1	MAC-CE based uplink spatial relation switch	2536

7.5.9.1.1	NR PCell FR2 uplink spatial relation switch associated with known DL-RS	2536

The rate of correct events observed during repeated tests shall be at least [90]%.7.5.9.2	RRC based uplink spatial relation switch	2539

7.5.9.2.1	NR PCell FR2 uplink spatial relation switch associated with known DL-RS	2539

7.5.10	UE specific CBW change	2543

7.5.10.0	Minimum conformance requirements	2543

7.5.10.0.1	Minimum conformance requirements for NR FR2 UE specific CBW change of PCell with non-DRX in SA	2543

7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	2544

7.5.10.1.1	Test purpose	2544

7.5.10.1.2	Test applicability	2544

7.5.10.1.3	Minimum conformance requirements	2544

7.5.10.1.4	Test description	2544

7.5.10.1.5	Test requirement	2545

7.5.11	UE UL carrier RRC reconfiguration delay	2547

7.5.11.1	UE UL carrier RRC reconfiguration delay	2547

7.5.11.1.1	Test purpose	2547

7.5.11.1.2	Test applicability	2548

7.5.11.1.3	Minimum conformance requirements	2548

7.5.11.1.4	Test description	2548

7.5.11.1.5	Test requirement	2550

7.5.12	Conditional PSCell addition and release delay (FR2 SA)	2553

7.5.12.1	NR SA FR2 Addition and Release Delay of PSCell	2553

7.5.12.1.1	Test purpose	2553

7.5.12.1.2	Test applicability	2553

7.5.12.1.3	Minimum conformance requirements	2553

7.5.12.1.4	Test description	2553

7.5.12.1.5	Test requirement	2555

7.5.13	Unified TCI state switch delay	2557

7.5.13.0	Minimum conformance requirements	2557

7.5.13.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	2557

7.5.13.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	2558

7.5.13.1	NR SA FR2 MAC-CE based active joint TCI state switch	2558

7.5.13.1.1	Test purpose	2558

7.5.13.1.2	Test applicability	2558

7.5.13.1.3	Minimum conformance requirements	2558

7.5.13.1.4	Test description	2558

7.5.13.1.5	Test requirement	2560

7.5.13.2	NR SA FR2 MAC-CE based active uplink TCI state switch	2561

7.5.13.2.1	Test purpose	2561

7.5.13.2.2	Test applicability	2561

7.5.13.2.3	Minimum conformance requirements	2561

7.5.13.2.4	Test description	2562

7.5.13.2.5	Test requirement	2563

7.5.13.3	NR SA FR2 MAC-CE based active downlink TCI state switch	2565

7.5.13.3.1	Test purpose	2565

7.5.13.3.2	Test applicability	2565

7.5.13.3.3	Minimum conformance requirements	2565

7.5.13.3.4	Test description	2565

7.5.13.3.5	Test requirement	2567

7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	2569

7.5.14.1	Test purpose	2569

7.5.14.2	Test applicability	2569

7.5.14.3	Minimum conformance requirements	2569

7.5.14.3.1	SCG Activation Delay Requirement	2569

7.5.14.3.2	SCG Deactivation Delay Requirement	2571

7.5.14.4	Test description	2571

7.5.14.4.1	Initial conditions	2571

7.5.14.4.2	Test procedure	2572

7.5.14.4.3	Message contents	2573

7.5.14.5	Test requirement	2573

7.6	Measurement procedures	2576

7.6.1	Intra-frequency measurements	2576

7.6.1.0	Minimum conformance requirements	2576

7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	2576

7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	2577

7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX	2579

7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX	2584

7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX	2589

7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	2595

7.6.1.5	NR SA FR2 event triggered reporting test without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17	2601

7.6.2	Inter-frequency measurements	2605

7.6.2.0	Minimum conformance requirements for Inter-frequency measurements	2605

7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX	2605

7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX	2611

7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	2617

7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection	2622

7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX	2628

7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX	2634

7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	2640

7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection	2646

7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	2652

7.6.3	L1-RSRP measurement for beam reporting	2657

7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	2657

7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	2657

7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	2657

7.6.3.0.3	Minimum conformance requirements for Inter-cell SSB based L1-RSRP Reporting	2657

7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX	2659

7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX	2664

7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX	2668

7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX	2672

7.6.3.5	NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	2675

7.6.3.6	NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	2681

7.6.4	CLI measurements	2688

7.6.4.0	Minimum conformance requirements	2688

7.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	2688

7.6.4.1	NR SA FR2 SRS-RSRP measurement in non-DRX	2689

7.6.4.2	NR SA FR2 CLI-RSSI measurement in non-DRX	2693

7.6.4.2.1	Test purpose	2693

7.6.4.2.2	Test applicability	2693

7.6.4.2.3	Minimum conformance requirements	2693

7.6.4.2.4	Test description	2693

7.6.4.2.5	Test requirement	2695

7.6.5	NR measurements with autonomous gaps	2696

7.6.5.0	Minimum conformance requirements	2696

7.6.5.0.1	Minimum conformance requirements for SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	2696

7.6.5.0.1.1	Minimum conformance requirements for interruptions when identifying CGI of an NR cell with autonomous gaps	2696

7.6.5.0.1.2	Minimum conformance requirements for NR measurements with autonomous gaps	2697

7.6.5.0.1.2.1	Introduction	2697

7.6.5.0.1.2.2	CGI identification of an NR cell with autonomous gaps	2697

7.6.5.0.1.2.3	CGI reporting delay	2698

7.6.5.1	SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	2698

7.6.6	L1-SINR measurement for beam reporting	2703

7.6.6.0	Minimum conformance requirements	2703

7.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	2703

7.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	2703

7.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	2703

7.6.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	2703

7.6.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	2707

7.6.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	2712

7.6.7 to 7.6.12	2718

7.6.13	UE Rx-Tx time difference measurements for PDC	2718

7.6.13.0	Minimum conformance requirements	2718

7.6.13.0.1	PRS based UE Rx-Tx time difference for propagation delay compensation	2718

7.6.13.0.2	TRS based UE Rx-Tx time difference for propagation delay compensation	2720

7.6.13.1	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS	2722

7.6.13.2	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS	2726

7.6.14	SA event triggered reporting tests with Pre-MG	2730

7.6.14.0	Minimum conformance requirements	2730

7.6.14.1	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2733

7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	2739

7.6.15	SA event triggered reporting tests with concurrent gaps	2744

7.6.15.0	Minimum conformance requirements	2744

7.6.15.0.1	Minimum conformance requirements for Intra-frequency measurement	2744

7.6.15.0.2	Minimum conformance requirements for Inter-frequency measurement	2745

7.6.15.0.3	Minimum conformance requirements for PRS measurement	2745

7.6.15.1	NR FR2 SA event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	2746

7.6.15.2	NR SA FR2 event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	2751

7.6.15.3	NR SA FR2 event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	2760

7.6.16	SA event triggered reporting tests with NCSG	2767

7.6.16.0	Minimum conformance requirements	2767

7.6.16.0.1	Minimum conformance requirements for intra-frequency measurement	2767

7.6.16.0.2	Minimum conformance requirements for inter-frequency measurement	2769

7.6.16.0.3	Minimum conformance requirements for interruptions	2770

7.6.16.1	NR FR2 SA event triggered reporting test with per-UE NCSG under non-DRX	2771

7.6.16.2	NR SA FR2 event-triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used(PCell in FR2)	2777

7.6.16.3	NR SA FR2 Event triggered reporting test on deactivated SCell measurement via NCSG in FR2 in non-DRX	2783

7.7	Measurement performance requirements	2788

7.7.1	SS-RSRP	2788

7.7.1.0	Minimum conformance requirements	2788

7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	2788

7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	2788

7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy	2788

7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy	2796

7.7.1.3	Inter-frequency measurements between FR1 and FR2	2804

7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy	2804

7.7.1.3.2	Void	2811

7.7.2	SS-RSRQ	2811

7.7.2.0	Minimum conformance requirements	2811

7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2811

7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	2811

7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy	2811

7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy	2815

7.7.3	SS-SINR	2821

7.7.3.0	Minimum conformance requirements	2821

7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2821

7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	2821

7.7.3.1	NR SA FR2 SS-SINR measurement accuracy	2821

7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy	2826

7.7.4	L1-RSRP	2833

7.7.4.0	Minimum conformance requirements	2833

7.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	2833

7.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	2833

7.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	2833

7.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	2833

7.7.4.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	2833

7.7.4.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy	2838

7.7.5	SRS-RSRP	2844

7.7.5.0	Minimum conformance requirements	2844

7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	2844

7.7.5.1	NR SA FR2 SRS-RSRP measurement accuracy	2846

7.7.5.2	NR SA FR2 CLI-RSSI measurement accuracy	2851

7.7.5.2.1	Test purpose	2852

7.7.5.2.2	Test applicability	2852

7.7.5.2.3	Minimum conformance requirements	2852

7.7.5.2.4	Test description	2852

7.7.5.2.4	Test description	2852

7.7.5.2.5	Test requirement	2853

7.7.6	L1-SINR	2856

7.7.6.0	Minimum conformance requirements	2856

7.7.6.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2856

7.7.6.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR L1-SINR measurement accuracy	2858

7.7.6.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2860

7.7.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	2863

7.7.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy	2868

7.7.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2873

8.0	General	2879

8.1	Void	2879

8.2.1.0	Minimum conformance requirements	2879

8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection	2879

8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell	2881

8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2888

8.2.2.0	Minimum conformance requirements	2895

8.2.2.1	E-UTRA – NR FR1 Early Measurement Reporting	2897

8.2.2.2	E-UTRA – NR FR2 Early Measurement Reporting	2905

8.3.1.0	Minimum conformance requirements	2910

8.3.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 handover	2910

8.3.1.1	E-UTRA - NR FR1 handover with known target cell	2911

8.3.1.1.3	Minimum conformance requirements	2911

8.4.1.0	Minimum conformance requirements	2916

8.4.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 SFTD measurement delay	2916

8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX	2917

8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX	2923

8.4.2.0	Minimum conformance requirements	2928

8.4.2.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	2928

8.4.2.0.2	Void	2932

8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX	2932

8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX	2938

8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX	2944

8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX	2950

8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX	2956

8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX	2961

8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX	2966

8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX	2971

8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2976

8.5.1	SFTD measurement accuracy	2982

8.5.1.0	Minimum conformance requirements	2982

8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	2982

8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy	2983

8.5.2	Inter-RAT measurement accuracy	2989

8.5.2.1	SS-RSRP	2989

8.5.2.1.0	Minimum conformance requirements	2989

8.5.2.1.1	SS-RSRP with NR FR1 target cell	2990

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell	2995

8.5.2.2	SS-RSRQ	3000

8.5.2.2.0	Minimum conformance requirements	3000

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell	3000

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell	3003

8.5.2.3	SS-SINR	3008

8.5.2.3.0	Minimum conformance requirements	3008

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell	3008

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell	3011

9	NR sidelink	3016

9.1	NR sidelink in FR1	3016

9.1.1	UE transmit timing	3016

9.1.1.0	Minimum conformance requirements	3016

9.1.1.0.1	Minimum conformance requirements for GNSS as synchronization reference source	3016

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	3016

9.1.1.0.3	Minimum conformance requirements for FR1 NR Cell as synchronization reference source	3016

9.1.1.1	NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source	3017

9.1.1.2	NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source	3019

9.1.1.3	NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source	3022

9.1.2	Initiation/Cease of S-SSB transmission	3027

9.1.2.0	Minimum conformance requirements	3027

9.1.2.0.1	Minimum conformance requirements for FR1 NR cell as synchronization reference source	3027

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	3028

9.1.2.1	NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source	3029

9.1.2.2	NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source	3034

9.1.3	Synchronization reference selection/reselection	3038

9.1.3.0	Minimum conformance requirements	3038

9.1.3.0.1	Minimum conformance requirements for GNSS configured as the highest priority synchronization reference source	3038

9.1.3.0.2	Minimum conformance requirements for eNB/gNB configured as the highest priority synchronization reference source	3039

9.1.3.1	NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source	3041

9.1.3.2	NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source	3047

9.1.4	L1 SL-RSRP measurements	3052

9.1.4.0	Minimum conformance requirements	3052

9.1.4.0.1	Minimum conformance requirements for resource selection/reselection, re-evaluation and pre-emption	3052

9.1.4.1	NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection	3053

9.1.4.2	NR SA FR1 L1 SL-RSRP measurement for resource pre-emption	3059

9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation	3065

9.1.5	Congestion control measurement	3072

9.1.5.0	Minimum conformance requirements	3072

9.1.5.0.1	Minimum conformance requirements for congestion control measurements	3072

9.1.5.1	NR SA FR1 congestion control measurement for concurrent operation	3073

9.1.5.2	NR SA FR1 congestion control measurement for PC5-only operation	3081

9.1.6	Congestion control measurement	3087

9.1.6.0	Minimum conformance requirements	3087

9.1.6.0.1	Minimum conformance requirements for interruption to WAN due to NR sidelink communication	3087

9.1.6.1	NR SA FR1 interruption to WAN due to NR sidelink communication	3087

10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	3091

10.1	RRC_CONNECTED state mobility	3091

10.2	Timing	3091

10.3	Signalling characteristics	3092

10.3.1	Radio link monitoring	3092

10.3.1.0	General	3092

10.3.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring under CCA	3093

10.3.1.1	Requirements for determining UE in-sync or out-of-sync status	3095

10.3.1.2	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	3096

10.3.1.3	EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	3104

10.3.1.4	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell under CCA configured with SSB-based RLM RS in DRX mode	3110

10.3.1.5	EN-DC FR1 radio link monitoring in-sync test for PSCell under CCA configured with SSB-based RLM RS in DRX mode	3110

10.3.2	Interruption	3111

10.3.2.0	Minimum Conformance Requirements	3111

10.3.2.1	EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA	3111

10.3.3	SCell activation and deactivation delay	3117

10.3.3.0	Minimum Conformance Requirements	3117

10.3.3.0.1	SCell Activation Delay Requirement for Deactivated SCell under CCA	3117

10.3.3.0.2	SCell Deactivation Delay Requirement for Activated SCell under CCA	3122

10.3.3.1	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	3122

10.3.3.2	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	3129

10.3.3.3	EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	3133

10.3.4	Beam failure detection and link recovery procedures	3137

10.3.4.0	General	3137

10.3.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	3138

10.3.4.0.3	Scheduling availability of UE during beam failure detection under CCA	3140

10.3.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	3140

10.3.4.1	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode	3140

10.3.4.2	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode	3148

10.3.5	Active BWP switching	3156

10.3.5.1	VOID	3156

10.3.5.2	DCI-based and Timer-based Active BWP Switch	3156

10.3.5.2.0	Minimum conformance requirements	3156

10.3.5.2.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	3159

10.3.5.2.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA	3167

10.3.5.3	RRC-based Active BWP Switch	3175

10.3.5.3.0	Minimum conformance requirements	3175

10.3.5.3.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	3176

10.4	Measurement procedures	3182

10.4.1	Intra-frequency measurements	3182

10.4.1.0	Minimum conformance requirements	3182

10.4.1.1	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA	3196

10.4.1.2	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA	3198

10.4.1.3	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA	3201

10.4.1.4	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA	3204

10.4.2	Inter-frequency measurements	3207

10.4.2.0	Minimum conformance requirements	3207

10.4.2.1	EN-DC FR1-FR1 RSSI measurement reporting under CCA	3213

10.4.2.2	EN-DC FR1-FR1 Channel occupancy measurement reporting under CCA	3214

10.4.2.3	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used	3216

10.4.2.4	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used	3222

10.4.2.5	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used	3229

10.4.2.6	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used	3235

10.4.2.7	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used	3242

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.10.4.2.8 EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used	3249

10.4.2.9	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used	3257

10.4.2.10	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used	3265

10.5	Measurement Performance	3273

10.5.1	SS-RSRP	3273

10.5.1.0	Minimum Conformance Requirements	3273

10.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3273

10.5.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	3274

10.5.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3275

10.5.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	3275

10.5.1.1	EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell	3276

10.5.1.2	EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell	3281

10.5.2	SS-RSRQ	3287

10.5.2.0	Minimum Conformance Requirements	3287

10.5.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3287

10.5.2.0.2	Void	3288

10.5.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3288

10.5.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3289

10.5.2.1	EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	3289

10.5.2.2	EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	3295

10.5.3	SS-SINR	3302

10.5.3.0	Minimum Conformance Requirements	3302

10.5.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	3302

10.5.3.0.2	Void	3302

10.5.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	3302

10.5.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	3303

10.5.3.1	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA	3304

10.5.3.2	EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA	3309

10.5.3.3	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA	3315

11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	3321

11.1	RRC_IDLE state mobility	3322

11.2	RRC_CONNECTED state mobility	3322

11.2.1	3322

11.2.2	RRC connection mobility control	3322

11.2.2.1	RRC re-establishment	3322

11.2.2.2	Random Access	3322

11.2.2.3	RRC connection release with redirection	3322

11.2.2.3.0	Minimum conformance requirements	3322

11.2.2.3.1	NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	3323

11.2.2.3.2	NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA	3328

11.3	Timing	3333

11.4	Signalling characteristics	3334

11.4.1	Radio link monitoring	3334

11.4.1.0	General	3334

11.4.1.0.1	Minimum conformance requirements for SSB based radio link monitoring under CCA	3334

11.4.1.1	Requirements for determining UE in-sync or out-of-sync status	3334

11.4.1.2	NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	3334

11.4.1.3	NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	3342

11.4.2	Interruptions	3348

11.4.3	SCell activation and deactivation delay	3348

11.4.4	Beam failure detection and link recovery procedures	3349

11.4.4.0	General	3349

11.4.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	3349

11.4.4.0.2	Void	3349

11.4.4.0.3	Scheduling availability of UE during beam failure detection under CCA	3349

11.4.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	3349

11.4.4.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA	3349

11.4.4.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA	3358

11.5	3366

11.6	Measurement Performance	3366

11.6.1	SS-RSRP	3366

11.6.1.0	Minimum Conformance Requirements	3366

11.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3366

11.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	3366

11.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3366

11.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	3366

11.6.1.1	NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA	3366

11.6.1.2	NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	3372

11.6.2	SS-RSRQ	3377

11.6.2.0	Minimum Conformance Requirements	3377

11.6.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3377

11.6.2.0.2	Intra-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3378

11.6.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3378

11.6.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3378

11.6.2.1	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	3378

11.6.2.2	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	3384

11.6.2.3	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA	3389

11.6.2.4	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA	3396

12 to 13	FFS	3404

14	NR standalone tests for Satellite access	3404

14.0	General	3404

14.0.1	Principle of testing GSO and NGSO scenarios	3404

14.0.2	Principle of testing different RRM requirements	3404

14.0.3	Principle of testing different ephemeris formats	3405

14.0.4	General setup for SIB19	3407

14.1	RRC_IDLE state mobility	3407

14.2	RRC_CONNECTED state mobility	3408

14.3	Timing for Satellite Access	3408

14.4	Signalling characteristics	3408

14.5	Measurement procedure	3408

14.5.1	Intra-frequency Measurements for SAN	3408

14.5.1.0	Minimum conformance requirements	3408

14.5.1.0.1	Minimum conformance requitements for intra-frequency measurements without measurement gaps	3408

14.5.1.0.2	Minimum conformance requitements for intra-frequency measurements with measurement gaps	3411

14.5.1.1	NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access	3413

14.5.1.2	NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access	3417

14.5.1.3	NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access	3420

14.5.1.4	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	3424

14.5.1.5	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access	3428

14.5.1.6	NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	3433

14.5.2	Inter-frequency Measurements for SAN	3438

14.5.2.0	Minimum conformance requirements	3438

14.5.2.1	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	3441

14.5.2.2	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access	3447

14.5.2.3	NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access	3453

14.5.2.4	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access	3459

14.5.2.5	Void	3465

14.5.2.6	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	3465

15	FFS	3471

16	NR standalone tests with all NR cells in FR1 for RedCap	3471

16.0	General	3471

16.1	RRC_IDLE state mobility	3471

16.1.1	NR cell re-selection	3471

16.1.1.0	Minimum conformance requirements	3471

16.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	3471

16.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	3473

16.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3477

16.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3480

16.1.1.1	NR SA FR1 Cell reselection for 1 Rx UE	3483

16.1.1.2	NR SA FR1 Cell reselection for 2 Rx UE	3491

16.1.1.3	NR SA FR1-FR1 Cell reselection for 1 Rx UE	3495

16.1.1.4	NR SA FR1-FR1 Cell reselection for 2 Rx UE	3500

16.1.1.5	NR SA FR1  Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3504

16.1.1.6	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3512

16.1.1.7	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3517

16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3525

16.1.2	NR – E-UTRA cell re-selection	3533

16.1.2.0	Minimum conformance requirements	3533

16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx	3533

16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx	3535

16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection	3535

16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX	3537

16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX	3543

16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX	3547

16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX	3553

16.1.2.5	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3557

16.1.2.6	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3563

16.2	RRC_INACTIVE state mobility for RedCap	3567

16.2.1	Configured Grant based Small Data Transmissions for RedCap	3567

16.2.1.0	Minimum conformance requirements	3567

16.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	3567

16.2.1.1	NR SA FR1 CG-SDT for 1Rx UE	3569

16.2.1.2	NR SA FR1 CG-SDT for 2Rx UE	3572

16.3	RRC_CONNECTED state mobility for RedCap	3576

16.3.1	Handover	3576

16.3.1.0	Minimum conformance requirements	3576

16.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	3576

16.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	3576

16.3.1.1	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE	3577

16.3.1.2	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE	3580

16.3.1.3	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	3584

16.3.1.4	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	3590

16.3.1.5	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	3595

16.3.1.6	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	3601

16.3.1.7	NR SA FR1 - E-UTRA handover for 1Rx UE	3608

16.3.1.8	NR SA FR1 - E-UTRA handover for 2Rx UE	3613

16.3.1.9	NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE	3618

16.3.1.10	NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE	3622

16.3.2	RRC connection mobility control for RedCap	3626

16.3.2.1	RRC re-establishment for RedCap	3626

16.3.2.1.0	Minimum conformance requirements	3626

16.3.2.1.1	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	3626

16.3.2.1.2	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	3629

16.3.2.1.3	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	3632

16.3.2.1.4	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	3635

16.3.2.1.5	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	3638

16.3.2.1.6	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	3641

16.3.2.2	Random access for RedCap	3644

16.3.2.2.0	Minimum conformance requirements	3644

16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	3645

16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	3648

16.3.2.2.3	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	3651

16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	3654

16.3.2.2.5	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	3657

16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	3660

16.3.2.2.7	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	3663

16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	3666

16.3.2.3	RRC connection release with redirection for RedCap	3669

16.3.2.3.0	Minimum conformance requirements	3669

16.3.2.3.1	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	3669

16.3.2.3.2	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	3673

16.3.2.3.3	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	3676

16.3.2.3.4	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	3681

16.4	Timing for RedCap	3686

16.4.1	UE transmit timing for RedCap	3686

16.4.1.0	Minimum conformance requirements	3686

16.4.1.1	NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	3687

16.4.1.2	NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	3691

16.4.2	3695

16.4.3	Timing advance for RedCap	3695

16.4.3.0	Minimum conformance requirement	3695

16.4.3.1	NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE	3695

16.4.3.2	NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE	3698

16.5	Signalling characteristics for RedCap	3701

16.5.1	Radio link monitoring for RedCap	3701

16.5.1.0	Minimum requirements	3701

16.5.1.0.1	General	3701

16.5.1.0.2	Minimum conformance requirements for SSB-based RLM	3705

16.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	3706

16.5.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	3707

16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	3711

16.5.1.3	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	3715

16.5.1.4	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	3720

16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	3724

16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	3729

16.5.1.7	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	3734

16.5.1.8	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	3739

16.5.1.9	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3743

16.5.1.10	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	3746

16.5.1.11	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3749

16.5.1.12	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	3752

16.5.1.13	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3756

16.5.1.14	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	3759

16.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	3762

16.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	3765

16.5.2	Beam Failure Detection and Link recovery procedures for RedCap	3768

16.5.2.0	Minimum requirements	3768

16.5.2.0.1	General	3768

16.5.2.0.2	Minimum conformance requirements for SSB-based BFD and CBD	3770

16.5.2.0.3	Minimum conformance requirements for CSI-RS based RLM	3772

16.5.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	3774

16.5.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	3779

16.5.2.3	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE	3784

16.5.2.4	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE	3790

16.5.2.5	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	3795

16.5.2.6	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	3800

16.5.2.7	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	3803

16.5.2.8	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	3806

16.5.3	Active BWP switch delay for RedCap	3811

16.5.3.1	DCI-based and time-based active BWP switch for RedCap	3811

16.5.3.1.0	Minimum conformance requirements	3811

16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE	3812

16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE	3816

16.5.3.2	RRC-based active BWP switch for RedCap	3821

16.5.3.2.0	Minimum conformance requirements	3821

16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE	3822

16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE	3826

16.5.4	UE specific CBW change for RedCap	3831

16.5.4.0	Minimum conformance requirements	3831

16.5.4.0.1	Minimum conformance requirements for UE specific CBW change	3831

16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE	3832

16.5.4.2	NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE	3834

16.6	Measurement procedures for RedCap	3836

16.6.1	Intra-frequency measurements for RedCap	3837

16.6.1.0	Minimum conformance requirements	3837

16.6.1.0.1	General	3837

16.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	3838

16.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	3840

16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE	3841

16.6.1.2	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE	3847

16.6.1.3	NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE	3853

16.6.1.4	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE	3855

16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	3858

16.6.1.6	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	3861

16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	3864

16.6.1.8	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	3867

16.6.1.9	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	3870

16.6.1.10	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	3876

16.6.1.11	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	3881

16.6.1.12	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2Rx UE	3884

16.6.2	Inter-frequency measurements for RedCap	3887

16.6.2.0	Minimum conformance requirements	3887

16.6.2.0.1	Minimum conformance requirements for inter-frequency event-triggered measurement with measurement gaps	3887

16.6.2.0.2	Minimum conformance requirements for inter-frequency event-triggered measurements without measurement gaps	3889

16.6.2.1	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	3892

16.6.2.2	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	3897

16.6.2.3	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	3903

16.6.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	3908

16.6.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	3912

16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	3916

16.6.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	3920

16.6.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	3924

16.6.2.9	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	3929

16.6.2.10	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	3934

16.6.2.11	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	3939

16.6.2.12	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	3944

16.6.3	Inter-RAT measurements for RedCap	3949

16.6.3.0	Minimum conformance requirements	3949

16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE	3949

16.6.3.2	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE	3949

16.6.3.3	NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE	3949

16.6.3.4	NR - E-UTRA event-triggered reporting in DRX in FR1 for 2 Rx UE	3949

16.6.4	L1-RSRP measurement for beam reporting for RedCap	3949

16.6.4.0	Minimum conformance requirements	3949

16.6.4.0.1	General	3949

16.6.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	3954

16.6.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	3958

16.6.4.3	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	3961

16.6.4.4	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	3965

16.6.4.5	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	3968

16.6.4.6	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	3971

16.6.4.7	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	3973

16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	3976

16.6.5	NR measurements with autonomous gaps	3978

16.6.5.0	Minimum conformance requirements	3978

16.6.5.0.1	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	3978

16.6.5.0.2	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	3980

16.6.5.0.3	Minimum conformance requirements for CGI identification of an E-UTRA cell with autonomous gaps	3980

16.6.5.1	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	3981

16.6.5.2	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	3984

16.6.5.3	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	3987

16.6.5.4	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	3995

16.7	Measurement performance requirements for RedCap	4000

16.7.1	SS-RSRP for RedCap	4000

16.7.1.0	Minimum conformance requirements	4000

16.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	4000

16.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	4001

16.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	4002

16.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	4003

16.7.1.1	Intra-frequency measurements for RedCap	4004

16.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	4004

16.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	4010

16.7.1.2.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	4012

16.7.1.2.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	4018

16.7.2	SS-RSRQ	4020

16.7.2.0	Minimum conformance requirements	4020

16.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	4020

16.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	4021

16.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	4022

16.7.2.1	4023

16.7.2.2	4023

16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4023

16.7.2.3.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4023

16.7.2.3.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4027

16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4029

16.7.2.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4029

16.7.2.4.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4033

16.7.3	SS-SINR	4035

16.7.3.0	Minimum conformance requirements	4035

16.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	4035

16.7.3.0.2	Inter-frequency SS-SINR absolute measurement accuracy requirements	4036

16.7.3.0.3	Inter-frequency SS-SINR relative measurement accuracy requirements	4036

16.7.3.1	NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4037

16.7.3.2	NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4041

16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4045

16.7.3.3.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4045

16.7.3.3.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4049

16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4051

16.7.3.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4051

16.7.3.4.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4055

16.7.4	L1-RSRP measurement for beam reporting	4057

16.7.4.0	Minimum conformance requirements	4057

16.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	4057

16.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	4058

16.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	4059

16.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	4059

16.7.4.1	SSB based L1-RSRP measurements for 1 Rx UE	4059

16.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 1 Rx UE	4059

16.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement for 1 Rx UE	4063

16.7.4.2	SSB based L1-RSRP measurements for 2 Rx UE	4065

16.7.4.2.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 2 Rx UE	4065

16.7.4.2.2	NR SA FR1 SSB based L1-RSRP relative measurement for 2 Rx UE	4069

16.7.4.3	CSI-RS based L1-RSRP measurements for 1 Rx UE	4071

16.7.4.3.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 1 Rx UE	4071

16.7.4.3.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 1 Rx UE	4075

16.7.4.4	CSI-RS based L1-RSRP measurements for 2 Rx UE	4077

16.7.4.4.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 2 Rx UE	4077

16.7.4.4.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE	4081

16.7.5	E-UTRAN RSRP	4083

16.7.5.0	Minimum conformance requirements	4083

16.7.5.0.1	E-UTRAN RSRP absolute accuracy	4083

16.7.5.1	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE	4084

16.7.5.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE	4088

16.7.6	E-UTRAN RSRQ	4092

16.7.6.0	Minimum conformance requirements	4092

16.7.6.0.1	E-UTRAN RSRQ absolute accuracy	4092

16.7.6.1	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE	4093

16.7.6.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE	4098

17	NR standalone tests with at least one NR cell in FR2 for RedCap	4103

17.1	RRC_IDLE state mobility	4103

17.1.1	NR cell re-selection	4103

17.1.1.0	Minimum conformance requirements	4103

17.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	4103

17.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	4105

17.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	4107

17.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	4109

17.1.1.1	NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx	4111

17.1.1.2	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx	4117

17.1.1.3	NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4123

17.1.1.4	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	4128

17.2	RRC_INACTIVE state mobility for RedCap	4135

17.2.1	Configured Grant based Small Data Transmissions for RedCap	4135

17.2.1.0	Minimum conformance requirements	4135

17.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	4135

17.2.1.1	NR SA FR2 TA validation for CG-SDT for 2 Rx	4136

17.3	RRC_CONNECTED state mobility for RedCap	4139

17.3.1	Handover for RedCap	4139

17.3.1.0	Minimum conformance requirements	4139

17.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	4139

17.3.1.1	NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	4141

17.3.1.2	NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	4143

17.3.2	RRC connection mobility control for RedCap	4150

17.3.2.1	RRC re-establishment for RedCap	4150

17.3.2.1.0	Minimum conformance requirements	4150

17.3.2.1.1	NR SA FR2 Intra-frequency RRC Re-establishment in FR2	4150

17.3.2.1.2	NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2	4152

17.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	4154

17.3.2.2	Random access for RedCap	4157

17.3.2.2.0	Minimum conformance requirements	4157

17.3.2.2.0.1	Minimum conformance requirements for Contention based random access	4157

17.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access	4157

17.3.2.2.1	NR SA FR2 4-step RA type contention based random access test in FR2 for NR Standalone	4158

17.3.2.2.2	NR SA FR2 4-step RA type non-contention based random access test in FR2 for NR Standalone	4161

17.3.2.2.3	NR SA FR2 2-step RA type contention based random access test in FR2 for NR Standalone	4164

17.3.2.2.4	NR SA FR2 2-step RA type non-contention based random access test in FR2 for NR Standalone	4167

17.3.2.3	RRC Connection Release with Redirection for RedCap	4170

17.3.2.3.0	Minimum conformance requirements	4170

17.3.2.3.0.1	Minimum conformance requirements for FR2-FR2 RRC connection release with redirection	4170

17.3.2.3.1	NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2	4170

17.4	Timing for RedCap	4172

17.4.1	UE transmit timing for RedCap	4172

17.4.1.0	Minimum Conformance Requirements	4173

17.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	4173

17.4.1.1	NR SA FR2 NR UE Transmit Timing Test for FR2	4174

17.4.2	UE timer accuracy for RedCap	4177

17.4.3	Timing advance for RedCap	4177

17.4.3.0	Minimum conformance requirements	4177

17.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	4177

17.5	Signalling characteristics for RedCap	4181

17.5.1	Radio link monitoring for RedCap	4181

17.5.1.0	Minimum requirements	4181

17.5.1.0.1	General	4181

17.5.1.0.2	Minimum conformance requirements for SSB-based RLM	4184

17.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	4186

17.5.1.1	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	4188

17.5.1.2	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	4191

17.5.1.4	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	4196

17.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	4199

17.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	4202

17.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	4210

17.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	4214

17.5.2	Beam Failure Detection and Link recovery procedures for RedCap	4217

17.5.2.0	Minimum conformance requirements	4217

17.5.2.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	4217

17.5.2.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	4217

17.5.2.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	4217

17.5.2.1 – 17.5.2.2	4217

17.5.2.3	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	4217

17.5.2.4	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	4221

17.5.2.5	NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	4225

17.5.3	4229

17.5.4	Active TCI state switch delay for RedCap	4229

17.5.4.0	Minimum conformance requirements	4229

17.5.4.0.1	Known conditions for TCI state	4229

17.5.4.0.2	Minimum conformance requirements for MAC-CE based active TCI state switch	4229

17.5.4.0.3	Minimum conformance requirements for RRC based active TCI state switch	4229

17.5.4.1	MAC-CE based active TCI state switch for RedCap	4229

17.5.4.2	RRC based active TCI state switch for RedCap	4232

17.5.4.2.1	NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state	4232

17.6	Measurement procedures for RedCap	4235

17.6.1	Intra-frequency measurements for RedCap	4235

17.6.1.0	Minimum conformance requirements	4235

17.6.1.0.1	General	4235

17.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	4236

17.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	4239

17.6.1.1	NR SA FR2 Event triggered reporting test without gap under non-DRX	4240

17.6.1.2	NR SA FR2 Event triggered reporting test without gap under DRX	4242

17.6.1.3	NR SA FR2 Event triggered reporting test with per-UE gaps under non-DRX	4244

17.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	4251

17.6.2	4254

17.6.3	L1-RSRP measurement for beam reporting for RedCap	4254

17.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	4254

17.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	4254

17.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	4254

17.6.3.1	NR SA FR2 SSB based L1-RSRP measurement when DRX is not used	4254

17.6.3.2	NR SA FR2 SSB based L1-RSRP measurement when DRX is used	4256

17.6.3.3	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is not used	4258

17.6.3.4	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used	4260

17.6.4	NR Measurements with autonomous gaps for RedCap	4263

17.6.4.0	Minimum conformance requirements	4263

17.6.4.0.1	Minimum conformance requirements for CGI identification of an NR cell with autonomous gaps	4263

17.6.4.1	NR SA FR2 Interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	4264

18	E-UTRA - NR inter-RAT with E-UTRA serving cell	4270

18.1	RRC_IDLE state mobility	4270

18.1.1	Inter-rat NR cell re-selection	4270

18.1.1.0	Minimum conformance requirements	4270

18.1.1.1	E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1	4270

18.2	RRC_CONNECTED state mobility for RedCap	4274

18.2.1	Handover for RedCap	4274

18.2.1.0	Minimum conformance requirements	4274

18.2.1.1	E-UTRA - NR FR1 handover  for 2Rx UE	4275

18.2.2.0	Minimum conformance requirements	4278

18.2.2.0.1	Redirection from E-UTRA to FR1 RedCap UE	4278

18.2.2.1	Redirection from E-UTRA to NR SA FR1 for redcap UE	4279

18.3	Measurement procedure for RedCap	4286

18.3.1	E-UTRA - NR Measurements for RedCap	4286

18.3.1.0	Minimum conformance requirements	4286

18.3.1.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	4286

18.3.1.1	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4289

18.3.1.2	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4293

18.3.1.3	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4297

18.3.1.4	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4301

18.3.1.5	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4305

18.3.1.6	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4307

18.3.1.7	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4310

18.3.1.8	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4312

Annex A (normative): RRM test configurations	4315

A.1	Reference measurement channels	4315

A.1.0	General	4315

A.1.1	PDSCH	4315

A.1.1.1	FDD	4315

A.1.1.2	TDD	4316

A.1.2	CORESET for RMSI scheduling	4319

A.1.2.1	FDD	4319

A.1.2.2	TDD	4320

A.1.3	CORESET for RMC scheduling	4323

A.1.3.1	FDD	4323

A.1.3.2	TDD	4324

A.1.4	CSI-RS	4326

A.1.4.1	FDD	4326

A.1.4.2	TDD	4329

A.1.4A	CSI-RS for tracking	4334

A.1.4A.1	FR1	4334

A.1.4A.1.1	FDD	4334

A.1.4A.1.2	TDD	4335

A.1.4A.2	FR2	4336

A.1.4A.2.1	TDD	4336

A.1.4B	CSI-IM configurations	4337

A.1.4B.1	FDD	4337

A.1.4B.2	TDD	4337

A.1.5	TDD UL/DL configuration	4339

A.1.6	PUSCH	4340

A.1A	Reference measurement channels under CCA	4341

A.1A.0	General	4341

A.1A.1	PDSCH	4341

A.1A.1.1	FDD	4341

A.1A.2	CORESET for RMSI scheduling	4342

A.1A.2.1	TDD	4342

A.1A.3	CORESET for RMC scheduling	4343

A.1A.3.1	TDD	4343

A.1A.4	TDD UL/DL configuration	4344

A.1A.4.1	TDD	4344

A.1A.5	RMC burst transmission model	4344

A.2	Reference OCGN configuration	4344

A.2.1 Generic OFDMA channel noise generator (OCNG)	4344

A.3	Reference SSB configuration	4347

A.3.1	SSB configuration for FR1	4347

A.3.2	SSB configuration for FR2	4347

A.3A	Reference SSB configuration under CCA	4348

A.3A.1	SSB configuration under CCA for FR1	4348

A.3A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	4348

A.3A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	4348

A.3A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	4349

A.3A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	4349

A.3B	Reference SSB configuration for RedCap	4350

A.3B.1	SSB configuration for FR1	4350

A.3B.2	SSB configuration for FR2	4351

A.4	Reference SMTC configuration	4351

A.4A	Reference SMTC configuration for RedCap	4351

A.5	Reference DRX configurations	4351

A.6	EN-DC test setup	4352

A.6.1	E-UTRA serving cell parameters	4352

A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1	4352

A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2	4354

A.6A	NR FR1-FR2 test setup	4356

A.6B	EN-DC test setup with unlicensed bands	4356

A.6B.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	4356

A.7	Reference PRACH configurations	4357

A.7.1	PRACH configurations for FR1	4357

A.7.1A	PRACH configurations for FR1 under CCA	4358

A.7.1A.1	FR1 PRACH configuration 1 under CCA	4358

A.7.1A.2	FR1 PRACH configuration 2 under CCA	4359

A.7.2	PRACH configurations for FR2	4360

A.7A	Reference MsgA configurations	4361

A.7A.1	MsgA configurations for FR1	4361

A.7A.1A	MsgA configurations for FR1 under CCA	4363

A.7A.1A.1	FR1 MsgA configuration 1 under CCA	4363

A.7A.1A.2	FR1 MsgA configuration 2 under CCA	4364

A.7A.2	MsgA configurations for FR2	4366

A.8	Reference BWP configurations	4368

A.8.1	Downlink BWP configurations	4368

A.8.2	Uplink BWP configurations	4368

A.8A	Reference BWP configurations for RedCap	4369

A.8A.1	Downlink BWP configurations	4369

A.8A.2	Uplink BWP configurations	4369

A.9	Angle of Arrival (AoA) for FR2 RRM test cases	4369

A.9.1	Setup 1: Single AoA in Rx beam peak direction	4370

A.9.2	Setup 2: Single AoA in non Rx beam peak direction	4370

A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	4370

A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	4370

A.9.3	Setup 3: 2 AoAs	4370

A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	4371

A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	4371

A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	4371

A.10	TCI State Configuration	4371

A.10.1	Introduction	4371

A.10.2	TCI states	4371

A.10A	Unified TCI State Configuration	4372

A.10A.1	Introduction	4372

A.10A.2	DLorJoint TCI states	4372

A.10A.3	UL TCI states	4372

A.11	NR sidelink communication	4373

A.11.1	Introduction	4373

A.11.2	Reference resource pool configurations for NR Sidelink Communication	4373

A.11.3	Reference measurement channels for NR Sidelink Communication	4376

A.12	Discovery Burst Transmission Window configuration under CCA	4377

A.12.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	4377

A.13	Channel bandwidth (CBW) configurations	4377

A.13.1	DL UE specific CBW	4377

A.13.2	UL UE specific CBW	4377

A.14	Satellite access configuration	4378

A.14.2	Satellite specific configuration for neighbour cell	4378

A.15	SRS Configurations	4379

A.15.1	SRS Configuration for SCS=15kHz	4379

A.15.2	SRS Configuration for SCS=30kHz	4380

A.15.3	SRS Configuration for SCS=30kHz	4381

Annex B (normative): Conditions for RRM requirements applicability for operating bands	4381

B.1	Conditions for NR RRC_IDLE state mobility	4381

B.1.1	Introduction	4381

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	4382

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	4383

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	4385

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	4386

B.2	Conditions for NR RRC_CONNECTED state	4386

B.2.1	Introduction	4386

B.2.2	Conditions for NR intra-frequency measurements	4386

B.2.3	Conditions for NR inter-frequency measurements	4387

B.2.4	Conditions for NR L1-RSRP reporting	4388

B.2.4.1	Conditions for SSB based L1-RSRP reporting	4388

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	4389

B.2.5	Conditions for RRC connection release with redirection to NR	4390

B.2.6	Conditions for UE transmit timing	4391

B.2.6.1	Conditions for SSB based UE transmit timing	4391

B.2.7	Conditions for NR intra-frequency measurements for RedCap	4392

B.2.8	Conditions for NR inter-frequency measurements for RedCap	4394

B.2.9	Conditions for NR intra-frequency measurements under CCA	4396

B.2.10	Conditions for NR inter-frequency measurements under CCA	4397

B.2.11	Conditions for NR L1-RSRP reporting under CCA	4397

B.2.11.1	Conditions for SSB based L1-RSRP reporting	4397

B.2.12 to B.2.16	4397

B.2.17	Conditions for NR intra-frequency measurements for satellite access	4397

B.2.18	Conditions for NR inter-frequency measurements for satellite access	4398

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	4398

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	4398

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	4398

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	4398

B.3	RRM requirement exceptions	4399

B.3.1	Introduction	4399

B.3.2	Receiver sensitivity relaxation for CA	4399

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	4399

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	4399

B.3.2.2.1	Inter-band carrier aggregation	4399

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	4399

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	4399

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	4400

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	4400

B.3.2.4.1	Intra-band contiguous carrier aggregation	4400

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	4400

B.3.3	Receiver sensitivity relaxation for DC	4400

B.3.4	Receiver sensitivity relaxation for SUL	4400

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	4400

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	4400

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	4400

B.4	Conditions for NR sidelink	4401

B.4.1	Test parameters for GNSS signals	4401

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	4401

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4401

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	4402

B.5	High level test procedure for SAN RRM tests	4402

Annex C (normative): Downlink physical channels and propagation conditions	4403

C.1	Downlink physical channels	4403

C.1.1	General	4403

C.1.2	Default downlink signal levels	4403

C.1.3	Default connection setup	4404

C.2	Propagation conditions	4404

C.2.0	General	4404

C.2.1	No interference	4405

C.2.2	Static propagation conditions	4405

C.2.2.0	General	4405

C.2.2.1	UE receiver with 2Rx antenna connectors	4405

C.2.2.2	UE receiver with 4Rx antenna connectors	4405

C.2.3	Multi-path fading propagation conditions	4406

Annex D (normative): Deviations from standard test configuration	4407

D.1	Test cases with different numerologies	4407

D.2	EN-DC test cases with different EN-DC configurations	4407

D.2.0	General	4407

D.2.1	Principle of testing	4407

D.3	Carrier aggregation test cases with different CA configurations	4407

D.3.0	General	4407

D.3.1	Principle of testing	4407

D.4	Antenna connection for 4Rx capable UEs	4407

D.4.0	General	4407

D.4.1	Principle of testing	4408

D.4.1.1	Single carrier tests	4408

D.4.1.2	Carrier aggregation tests	4409

D.4.1.3	EN-DC tests	4409

D.4.2	Antenna connection	4409

D.4.2.1	Antenna connection for NR bands where 2Rx is supported	4409

D.4.2.2	Antenna connection for NR bands where only 4Rx is supported	4409

D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported	4409

D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported	4410

D.4A	Antenna configurations with unlicensed bands for 4Rx capable UEs	4410

D.4A.1	Introduction	4410

D.4A.2	Principle of Testing	4410

D.4A.2.1	Single Carrier Tests	4410

D.4A.2.2	Carrier Aggregation Tests	4411

D.4A.2.3	EN-DC Tests	4411

D.4A.2.4	Antenna connection for bands where 2RX is supported	4411

D.4A.2.5	Antenna connection for bands where 4RX is supported	4411

D.4A.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported	4411

D.4A.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported	4411

D.5	Test Cases with Different Channel Bandwidths	4412

D.5.1	Test Cases with Different E-UTRA Channel Bandwidths	4412

D.5.1.1	Introduction	4412

D.5.1.2	Principle of testing	4412

D.6	Test Cases for Synchronous and Asynchronous DC Operations	4412

D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	4412

D.6.1.1	Introduction	4412

D.6.1.2	Principle of Testing	4412

D.7	CCA model	4412

D.7.1	Introduction	4412

D.7.2	CCA model for operation on a carrier frequency with CCA in FR1	4412

D.7.2.1	DL CCA model	4412

D.7.2.2	UL CCA model	4414

D.7.3	Test Cases with at Least One Cell on a Carrier Frequency with CCA	4414

D.8	Test Cases in SA and EN-DC Operations	4414

D.8.0	General	4414

D.8.1	Principle of testing	4415

Annex E (normative): Cell configuration mapping	4416

E.0	General	4416

E.1	Test frequency selection	4416

E.1.0	General	4416

E.1.1	E-UTRA PCell for EN-DC test cases	4416

E.1.2	Test cases with one NR cell	4416

E.1.3	Test cases with more than one NR cell	4416

E.1.3.1	Intra-frequency test cases	4416

E.1.3.2	Inter-frequency test cases	4416

E.1.4	Carrier aggregation test cases	4417

E.1.4.1	Inter-band carrier aggregation	4417

E.1.4.2	Intra-band contiguous carrier aggregation	4417

E.1.4.3	Intra-band non-contiguous carrier aggregation	4417

E.1.5	E-UTRA – NR inter RAT test cases	4417

E.1.6	Intra-band EN-DC test cases	4417

E.1.6.1	Intra-band non-contiguous EN-DC	4417

E.1.6.1.1	Inter frequency neighbour cell	4417

E.1.6.2	Intra-band contiguous EN-DC	4417

E.1.6.1.1	E-UTRA PCell	4417

E.1.7	NR sidelink test cases	4418

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4	4418

E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5	4421

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6	4424

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7	4430

E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8	4434

E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9	4435

E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10	4435

E.9 to E.13	4437

E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16	4437

E.15	Cell configuration mapping for SA FR2 test cases for RedCap in Chapter 17	4443

E.16	Cell configuration mapping for E-UTRAN – SA test cases for RedCap in Chapter 18	4445

Annex F (normative): Measurement uncertainties and test tolerances	4446

F.1	Measurement uncertainties and test tolerances for FR1 and FR2	4446

F.1.1	Acceptable uncertainty of test system (normative)	4446

F.1.1.1	Measurement of test environments	4446

F.1.1.2	Measurement of RRM requirements	4446

F.1.2	Interpretation of measurement results (normative)	4502

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)	4502

F.1.3.1	Measurement of test environments	4502

F.1.3.2	Measurement of RRM requirements	4502

Annex G (normative): Statistical testing	4621

G.1	General	4621

G.2	Statistical testing of delay and UE measurement performance in RRM tests	4621

G.2.1	General	4621

G.2.2	Design of the test	4621

G.2.3	Numerical definition of the pass fail limits	4621

G.2.4	Pass fail decision rules	4622

G.2.5	Void	4623

G.2.6	Test conditions for delay tests and UE measurement performance	4623

G.3	Statistical testing of NR sidelink CBR measurement tests	4623

G.3.1	General	4623

G.3.2	Design of the test	4623

G.3.3	Numerical definition of the pass fail limits	4624

G.3.4	Pass fail decision rules	4625

G.4	Theory to derive the numbers in Table G.2.3-1 (informative)	4625

G.5	Theory to derive the numbers in Table G.3.3-1 (informative)	4625

G.5.1	Algorithm to calculate pass and fail probabilities	4625

G.5.2	Method of designing pass/fail limit to approach given risk probability	4629

Annex H (normative): Default message contents for RRM	4633

H.1	Void	4633

H.2	System information blocks message content exceptions	4633

H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection	4633

H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection	4635

H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection	4636

H.3	RRC message content exceptions	4637

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration	4637

H.3.2	RRC messages and information elements contents exceptions for handover	4659

H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover	4660

H.3.4	E-UTRA RRC messages and information elements contents exceptions	4661

H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting	4672

H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting	4675

H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used	4681

H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay	4684

H.3.9	RRC messages and information elements contents exceptions for UL timing	4685

H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access	4685

Annex I (normative): RRM OTA procedures	4687

I.0	Test applicability per permitted test method	4687

I.1	Direct far field (DFF)	4687

I.1.1	RX beam peak direction search	4687

I.1.2	Search for directions corresponding to the EIS spherical coverage percentile	4687

I.2	Direct far field (DFF) simplification	4687

I.2.1	RX beam peak direction search	4687

I.2.2	Search for directions corresponding to the EIS spherical coverage percentile	4688

I.3	Indirect far field (IFF)	4688

I.3.1	RX beam peak direction search	4688

I.3.2	Search for directions corresponding to the EIS spherical coverage percentile	4688

I.3A	Enhanced indirect far field (Enhanced IFF)	4688

I.3A.1	RX beam peak direction search	4688

I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile	4688

I.4	Procedures to search test directions for RRM FR2	4688

I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search	4688

Annex J (informative): Change history	4690