# 8	E-UTRA - NR inter-RAT with E-UTRA serving cell

## 8.0	General

This clause contains test scenarios for E-UTRA - NR inter-RAT with the serving cell in E-UTRA. The NR cells can be in FR1, FR2 or both.

For conformance testing involving FR2 test cases in this specification, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

## 8.1	Void

## 8.2	RRC_IDLE state mobility

### 8.2.1	Inter-RAT cell re-selection

#### 8.2.1.0	Minimum conformance requirements

##### 8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection

[TS 36.133, clause 4.2.1]

When the UE is in either Camped Normally state or Camped on Any Cell state on a cell, the UE shall attempt to detect, synchronise, and monitor intra-frequency, inter-frequency and inter-RAT cells indicated by the serving cell. UE measurement activity is also controlled by measurement rules defined in TS36.304, allowing the UE to limit its measurement activity.

[TS 36.133, clause 4.2.2]

The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds when the UE is not configured with eDRX_IDLE cycle, and at least every Thigher_priority_search = MAX(60 * Nlayers, one eDRX_IDLE cycle) when UE is configured with eDRX_IDLE cycle, where Nlayers is the total number of configured higher priority E-UTRA, UTRA FDD, UTRA TDD, CDMA2000 1x, HRPD and NR carrier frequencies and is additionally increased by one if one or more groups of GSM frequencies is configured as a higher priority.

[TS 36.133, clause 4.2.2.5.6]

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT NR layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in TS 36.133 clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT NR layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT NR layers shall be the same as that defined below for lower priority RATs.

The requirements in this clause apply for inter-RAT NR measurements. When the measurement rules indicate that inter-RAT NR cells are to be measured, the UE shall measure SS-RSRP and SS-RSRQ of detected NR cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, a carrier is indicated to meet high speed requirement if highSpeedInterRAT-NR-r16 is configured and the carrier to be measured is configured with highSpeedCarrierNR-r16. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE is required to meet non high speed requirements no matter whether highSpeedInterRAT-NR-r16 or highSpeedCarrierNR-r16 is configured or not. The parameter NNR_carrier is the number of configured NR carriers indicated to meet non high speed requirement in the neighbour frequency list. NNR_carrier_HST is the number of configured carriers for reselection indicated to meet high speed requirements. The parameter NNR_carrier for a UE configured with idle mode DC measurements (while T331 is running) is the combined number of configured NR carriers indicated to meet non high speed requirement in the neighbour frequency list, and NR carriers configured for idle mode DC measurements.

NOTE:	Combined total number means that if a carrier is indicated in the neighbour frequency list and additionally a carrier configured for idle mode DC measurements, it only counts as one carrier.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured NR cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

The UE shall be able to evaluate whether a newly detectable inter-RAT NR cell meets the reselection criteria defined in TS 36.304 [1] within NNR_carrier_HST * Tdetect, NR_HST + NNR_carrier  * Tdetect, NR

when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or 6 dB in FR1 or 7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or 4 dB in FR1 and 4 dB in FR2 for SS-RSRQ reselections based on absolute priorities.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT NR cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

Cells which have been detected shall be measured at least every NNR_carrier_HST * Tmeasure, NR_HST + NNR_carrier  * Tmeasure, NR  when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT NR cell has met reselection criterion defined in TS 36.304 [1] within NNR_carrier_HST * Tevaluate, NR_HST + NNR_carrier  * Tevaluate, NR  when Treselection = 0 as specified in Table 8.2.1.0.1-1 and  table 8.2.1.0.1-2 provided that the reselection criteria is met by a margin of at least 5dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or 6 dB in FR1 or 7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or 4 dB in FR1 and 4 dB in FR2 for SS-RSRQ reselections based on absolute priorities.

If Treselection timer has a non-zero value and the inter-RAT NR cell is satisfied with the reselection criteria which are defined in TS 36.304 [1], the UE shall evaluate this NR cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 8.2.1.0.1-1: Tdetect,NR, TmeasureNR, and Tevaluate,NR

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x 1.5 x N1 (36 x 1.5 x N1) | 1.28 x 1.5 x N1 (4 x 1.5 x N1) | 5.12 x 1.5 x N1 (16 x 1.5 x N1) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. |  |  |  |  |  |

Table 8.2.1.0.1-2: Tdetect,NR_HST, TmeasureNR_HST, and Tevaluate,NR_HST for UE configured with highSpeedInterRAT-NR-r16

| DRX cycle length [s] | Tdetect,NR_HST [s] (number of DRX cycles) | Tmeasure,NR_HST [s] (number of DRX cycles) | Tevaluate,NR_HST [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 2 | 0.64 x M3 (2 x M3)Note 2 | 0.96 x M4 (3 x M4) Note 2 |
| 0.64 | 7.68 (12)) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| NOTE 1: FR2 high speed requirements are not specified. NOTE 2: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=1. |  |  |  |

The normative reference for this requirement is TS 36.133 clause 4.2.1, 4.2.2, 4.2.2.5.6.

#### 8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell

8.2.1.1.1	Test purpose

The purpose of this test is to verify the requirement for the E-UTRAN to NR inter-RAT cell reselection requirements specified in TS 36.133 [23] clause 4.2.2.5.6.

8.2.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

8.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.2.1.1.

8.2.1.1.4	Test description

There are two cells configured in this test, the E-UTRA Cell 1 and NR Cell 2. NR Cell 2 is the PCell and E-UTRA Cell 1 is the neighbour cell. This test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. E-UTRA Cell 1 is already identified by the UE prior to the start of the test. Cell 2 is of higher priority than cell 1.

8.2.1.1.4.1	Initial conditions

This test shall be tested under any of the test configuration in Table 8.2.1.1.4.1-1.

Table 8.2.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.2.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.2.1.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.2.1.1.4.1-2.

Table 8.2.1.1.4.1-2: Initial conditions for E-UTRA - NR FR1 cell re-selection to
higher priority NR target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.2.1.1.4.1-3.

2.	Message contents are defined in clause 8.2.1.1.4.3.

3.	The test scenario comprises of one NR cell and one E-UTRAN cell. NR Cell 2 is the PCell and E-UTRA Cell 1 is the neighbour cell. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clauses C.1.0 and C.1.1 and NR Cell 2 is configured according to clauses C.1.1 and C.1.2.

Table 8.2.1.1.4.1-3: General test parameters for E-UTRA -
NR FR1 cell re-selection to higher priority NR target cell

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE camps on cell 2 in the initial phase |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T1 end condition | Active cell |  |  | Cell1 | During T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  |  | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2, 5 | 3 s | Synchronous cells |
|  |  |  | 3, 6 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

8.2.1.1.4.2	Test procedure

Two cells are deployed in the test, which are one NR PCell (NR Cell 2) and a E-UTRA neighbour cell (E-UTRA Cell 1). The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. E-UTRA Cell 1 is already identified by the UE prior to the start of the test. NR Cell 2 is of higher priority than E-UTRA Cell 1.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure.

1.	The SS shall send a RRCRelease message on NR Cell 2 to ensure the UE is in State RRC_IDLE with generic procedure parameters connectivity = NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

2.	The SS shall set the parameters according to T1 in Table 8.2.1.1.5-1 and 8.2.1.1.5-2 at the same time instance RRCRelease message is sent on NR Cell 2, T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, E-UTRA Cell 1.

4.	If the UE responds on lower priority cell, E-UTRA Cell 1 during time duration T1 within 8 seconds from the beginning of time period T1, then count a success for the event "Re-select lower priority Cell 1". Otherwise count a fail for the event "Re-select lower priority Cell 1".

5.	If the UE has re-selected E-UTRA Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5a. Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 11.

5a.	The SS shall send an RRCConnectionRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 8.2.1.1.5-1. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE, T2 starts.

7.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 8.2.1.1.5-1, T3 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, NR Cell 2.

9.	If the UE responds on higher priority cell, NR Cell 2 during time duration T3 within 68 seconds from the beginning of time period T3, then count a success for the event "Re-select higher priority Cell 2". Otherwise count a fail for the event "Re-select higher priority Cell 2".

10.	If the UE has re-selected Cell 2 within T3, after the re-selection or when T3 expires, continues with step 10a.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and continue with step 11.

10a.The SS shall send an RRCRelease message on NR Cell 2 to ensure that the UE is in state RRC_IDLE with generic procedure parameters connectivity = NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

11.	The SS shall set the parameters according to T1 in Table 8.2.1.1.5-1 and 8.2.1.1.5-2 at the same time instance RRCRelease message is sent on NR Cell 2.

12.	Repeat step 2-11 until a test verdict has been achieved.
Each of the events "Re-select lower priority Cell 1" and "Re-select higher priority Cell 2" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

8.2.1.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.2.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with condition lower priority Table H.2.3-3 with condition lower priority |
| Default RRC messages and information elements contents exceptions |  |

Table 8.2.1.1.4.3-2: SystemInformationBlockType3

| Derivation Path: TS 36.508 Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 25 | Actual value = 50 dB |  |
| threshServingLow | 22 | Actual value = 44 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.4.3-3: SystemInformationBlockType24

| Derivation Path: TS 36.508, Table 4.4.3.3-20 with Condition FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| measTimingConfig-r15[1] SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20-r15 | 0 |  |  |
| } |  |  |  |
| ssb-Duration-r15 | sf5 | Asynchronous cells | Configuration 1,4 |
|  | sf2 | Synchronous cells | Configuration 2,3,5,6 |
| } |  |  |  |
| subcarrierSpacingSSB-r15[1] | kHz15 |  | Configuration 1,2,4,5 |
|  | kHz30 |  | Configuration 3,6 |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| q-RxLevMin-r15[1] | -70 | Actual value = -140 dBm | Configuration 1,2,4,5 |
|  | -69 | Actual value = -137 dBm, Round down | Configuration 3,6 |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.4.3-4: Void

Table 8.2.1.1.4.3-5: Void

8.2.1.1.5	Test requirement

Tables 8.2.1.1.4.1-3, 8.2.1.1.5-1 and 8.2.1.1.5-2 define the primary level settings including test tolerances for higher priority NR cell re-selection test case.

Table 8.2.1.1.5-1: NR Cell specific test parameters for E-UTRA -
NR FR1 cell re-selection to higher priority NR target cell

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.2 FR1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6 | SS-RSRP |  |  |
|  | dB | 1, 4 | -3.65 | -infinity | 13.55 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -99.90 | -98 | -98 |
|  |  | 2, 5 | -99.90 | -98 | -98 |
|  |  | 3, 6 | -96.90 | -95 | -95 |
| Note2 | dBm/15 kHz | 1, 4 | -99.90 | -98 | -98 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4 | -3.65 | -infinity | 13.55 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -103.55 | -infinity | -84.45 |
|  |  | 2, 5 | -103.55 | -infinity | -84.45 |
|  |  | 3, 6 | -100.54 | -infinity | -81.44 |
| Io | dBm/9.36 MHz | 1, 4 | -70.39 | -70.05 | -56.31 |
|  | dBm/9.36 MHz | 2, 5 | -70.39 | -70.05 | -56.31 |
|  | dBm/38.16 MHz | 3, 6 | -64.29 | -63.94 | -50.21 |
| Treselection | s | 1, 2, 3, 4, 5, 6 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 8.2.1.1.5-2: E-UTRA Cell specific test parameters for for E-UTRA - NR FR1 cell re-selection to higher priority NR target cell

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -82.45 |
|  | dB | 14 | 14 | 15.55 |
|  | dB | 14 | 14 | 15.55 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | 50 |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

The cell reselection delay to a higher priority NR cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Thigher_priority_search is the higher priority search period, Thigher_priority_search = 60 s according to TS 36.133 [23], clause 4.2.2;

Tevaluate, NR is the evaluation time for NR Cell, Tevaluate, NR = 6400 ms according TS 36.133 [23], Table 4.2.2.5.6-1;

TSI-NR is the maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; TSI-NR = 1280 ms is assumed in this test case.

Tevaluate, E-UTRAN is the evaluation time for E-UTRAN Cell, Tevaluate, E-UTRAN = 6400 ms according TS 38.133 [6], Table 4.2.2.5-1;

TSI-E-UTRA is the maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; TSI-E-UTRA = 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority NR cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

#### 8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

8.2.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the E-UTRAN to NR inter-RAT cell reselection requirements specified in TS 36.133 [23] clause 4.2.2.5.6.

8.2.1.2.2	Test applicability

This test applies to all types of NR UE configured with highSpeedInterRAT-NR-r16 from Release 15 onwards.

8.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.2.1.2.

8.2.1.2.4	Test description

8.2.1.2.4.1	Initial conditions

This test shall be tested under any of the test configuration in Table 8.2.1.2.4.1-1.

Table 8.2.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.2.1.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.2.1.2.4.1-2.

Table 8.2.1.2.4.1-2: Initial conditions for E-UTRA - NR FR1 cell re-selection to lower priority NR target cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.2.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.2.1.2.4.1-3.

2.	Message contents are defined in clause 8.2.1.2.4.3.

3.	The test scenario comprises of one NR cell and one E-UTRAN cell. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1 and NR Cell 2 is configured according to clauses C.1.1 and C.1.2.

4.	In SIB of the E-UTRA cell, highSpeedInterRAT-NR-r16 is configured and the carrier of NR cell is configured with highSpeedCarrierNR-r16.

Table 8.2.1.2.4.1-3: General test parameters for E-UTRA - NR FR1 cell re-selection
to lower priority NR target cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2, 5 | 3 s | Synchronous cells |
|  |  |  | 3, 6 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.32 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

8.2.1.2.4.2	Test procedure

Two cells are deployed in the test, which are one E-UTRA Cell 1 and NR Cell 2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cells are already identified by the UE prior to the start of the test. NR Cell 2 is of lower priority than E-UTRA Cell 1. UE is camped on E-UTRA Cell 1 during the initial phase of the test.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure.

1.	Ensure the UE is in State RRC_IDLE with generic procedure parameters Connectivity EUTRA/EPC with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T2 in Table 8.2.1.2.5-1 and 8.2.1.2.5-2. Then wait 5s to ensure that Cell 2 has been detected by the UE.

3.	Set the parameters according to T1 in Table 8.2.1.2.5-1 and 8.2.1.2.5-2, T1 starts.

4.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority NR Cell 2.

5.	If the UE responds on lower priority cell, NR Cell 2 during time duration T1 within 3 seconds from the beginning of time period T1, then count a success for the event "Re-select lowe priority Cell 2".
Otherwise count a fail for the event "Re-select lower priority Cell 2".

6.	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, continue with step 7. 
Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, skip to step 10.

7. The SS shall switch the power setting from T1 to T2 as specified in Table Table 8.2.1.2.5-1 and 8.2.1.2.5-2. T2 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority E-UTRA Cell 1.

9.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, skip to step 11.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and continue with step 10.

10.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity EUTRA/EPC with Test Mode On according to TS 38.508-1 [14] clause 4.5 in Cell 1.

11.	Repeat step 3-9 the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.2.1.2.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.2.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-3 with Condition HighSpeedMeas Table H.2.3-1 Table H.2.3-2 with condition higher priority Table H.2.3-3 with condition higher priority |
| Default RRC messages and information elements contents exceptions |  |

Table 8.2.1.2.4.3-2: SystemInformationBlockType3

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 25 | Actual value = 50 dB |  |
| threshServingLow | 22 | Actual value = 44 dB |  |
| cellReselectionPriority | 5 |  | higher priority E-UTRA |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.2.4.3-3: highSpeedCarrierNR-r16 in SystemInformationBlockType24

| Derivation Path: TS 36.508, Table 4.4.3.3-20 with Condition FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| measTimingConfig-r15[n] SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20 | 0 |  |  |
| } |  |  |  |
| ssb-Duration-r15 | sf5 | Asynchronous cells | Configuration 1,4 |
|  | sf2 | Synchronous cells | Configuration 2,3,5,6 |
| } |  |  |  |
| subcarrierSpacingSSB-r15[1] | kHz15 |  | Configuration 1,2,4,5 |
|  | kHz30 |  | Configuration 3,6 |
| cellReselectionPriority-r15[n] | 4 |  | higher priority E-UTRA |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| q-RxLevMin-r15[1] | -70 | Actual value = -140 dBm | Configuration 1,2,4,5 |
|  | -69 | Actual value = -137 dBm, Round down | Configuration 3,6 |
| } |  |  |  |
| carrierFreqListNR-v1610 ::= SEQUENCE {(SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| highSpeedCarrierNR-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.2.1.2.4.3-4: highSpeedInterRAT-NR-r16 configuration

| Derivation Path: TS 36.508 Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB::= SEQUENCE { |  |  |  |
| highSpeedInterRAT-NR-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.2.1.2.4.3-5: SIB2

| Derivation Path: TS 38.508-1, Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.2.4.3-6: SIB5

| Derivation Path: Table H.2.3-3 with Condition higher priority |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA-v1610 SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA-v1610 { | 1 entry | carrierFreqListEUTRA-v1610 and carrierFreqListEUTRA contain same number of entries. And the entries are listed in the same order |  |
| CarrierFreqEUTRA-v1610[1] SEQUENCE { |  | entry 1 |  |
| highSpeedEUTRACarrier-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.2.1.2.5	Test requirement

Tables 8.2.1.2.4.1-3, 8.2.1.2.5-1 and 8.2.1.2.5-2 define the primary level settings including test tolerances for higher priority E-UTRA cell re-selection test case.

Table 8.2.1.2.5-1: NR Cell specific test parameters for E-UTRA - NR FR1 cell re-selection to
lower priority NR target cell for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |
|  |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6 | SS-RSRP |  |
|  | dB | 1, 4 | 17.45 | 14 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Note2 | dBm/SCS | 1, 4 | -99.9 |  |
|  |  | 2, 5 | -99.9 |  |
|  |  | 3, 6 | -96.9 |  |
| Note2 | dBm/15 kHz | 1, 4 | -99.9 |  |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | 17.45 | 14 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -82.45 | -85.90 |
|  |  | 2, 5 | -82.45 | -85.90 |
|  |  | 3, 6 | -79.45 | -82.90 |
| Io | dBm/9.36 MHz | 1, 4 | -54.42 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -54.42 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -48.32 | -51.68 |
| Treselection | s | 1, 2, 3, 4, 5, 6 | 0 |  |
| Snonintrasearch | dB | 1, 2, 3, 4, 5, 6 | Not sent |  |
| Threshx, high | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, low | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, low | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 4, 56 | AWGN 1944 Note 4 |  |
|  |  | 3, 6 | AWGN 3334 Note 5 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. NOTE 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |

Table 8.2.1.2.5-2: E-UTRA Cell specific test parameters for for E-UTRA - NR FR1 cell re-selection to
lower priority NR target cell for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [23] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
| Note 2 | dBm/15 kHz | -99.9 |  |
| RSRP Note 3 | dBm/15 KHz | -103.55 | -84.45 |
|  | dB | -3.65 | 15.45 |
|  | dB | -3.65 | 15.45 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | 50 |  |
| Threshx, high | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void |  |  |  |

The cell reselection delay to a lower priority NR cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR_HST + TSI-NR,

Where:

Tevaluate, NR_HST	See Table 4.2.2.5.6-2 in clause 4.2.2.5.6 in TS 36.133 [23]

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 2.24 s, allow 3 s for the cell re-selection delay to a lower priority NR cell.

### 8.2.2	E-UTRA - NR Inter-RAT Early Measurement Reporting

#### 8.2.2.0	Minimum conformance requirements

A UE supporting ca-IdleModeMeasurements shall perform the idle mode measurement on the inter-frequency CA candidate frequencies/cells indicated by higher layers and meet the requirement specified in this clause. A UE supporting nr-IdleInactiveMeasFR1-r16 and/or nr-IdleInactiveMeasFR2-r16 shall perform the idle mode measurement on the NR inter-RAT EN-DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this section.

Additionally, a UE supporting ca-IdleModeMeasurements or nr-IdleInactiveMeasFR1-r16/nr-IdleInactiveMeasFR2-r16 shall perform the idle mode measurement on serving cell and meet the requirement specified in this section.

A UE only supporting ca-IdleModeMeasurements shall fulfil the requirements defined for a UE supporting ca-IdleModeMeasurements as defined in this section.

A UE only supporting nr-IdleInactiveMeasFR1-r16/nr-IdleInactiveMeasFR2-r16 shall fulfil the requirements defined for a UE supporting nr-IdleInactiveMeasFR1-r16/nr-IdleInactiveMeasFR2-r16 as defined in this section.

A UE supporting both ca-IdleModeMeasurements and nr-IdleInactiveMeasFR1-r16/nr-IdleInactiveMeasFR2-r16 shall fulfil the requirements for a UE supporting ca-IdleModeMeasurements and nr-IdleInactiveMeasFR1-r16/nr-IdleInactiveMeasFR2-r16 as defined in this section.

The requirements in clause 4.9 apply provided that the UE is provided with a valid timer T331 by dedicated RRC signalling and T331 is running.

For a UE which supports ca-IdleModeMeasurements the UE shall support the idle mode CA measurements on the serving cell, overlapping and non-overlapping carriers.

For inter-frequency idle mode measurements, an overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements. A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements.

This clause defines the requirements for the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an E-UTRAN carrier aggregation or EN-DC capable UE which has been configured with one or more downlink SCells or NR PSCell and one or more downlink NR SCells during the Connected mode and which supports ca-IdleModeMeasurements or nr-IdleInactiveMeasFR1-r16/nr-IdleInactiveMeasFR2-r16. The requirements are applicable for E-UTRAN FDD and TDD SCell(s), and NR PSCell and SCells.

Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signalling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signalling, and

-	For inter-frequency carrier, the detected cell remains detectable until UE reconnects to the network and transmits the early measurement report, or for NR inter-RAT carrier, the detected cell and SSBs of the detected cell remains detectable until UE reconnects to the network and transmits the early measurement report.

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-frequency cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.1 and Annex B.1.2 for a corresponding Band. An inter-RAT NR cell is considered detectable according to the conditions in B.1.3 of [50] for a corresponding band. An SSB of an inter-RAT NR cell is considered detectable according to SSB_RP and SSB Ês/Iot defined in Annex B.1.3 of [50] for a corresponding Band.

In the absence or expiration of T331, it is up to UE implementation to apply the requirements on the detected cell status in this subclause.

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in section 4.2.2.1 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in clauses 9.1.2B.2 and 9.1.5B.2, respectively.

While T331 is running, the UE shall perform measurement on the configured NR inter-RAT carriers for idle mode measurement reporting.

In addition to the requirements defined in section 4.2.2.9 and 4.2.2.9a, a UE which supports nr-IdleInactiveMeasFR1-r16 or nr-IdleInactiveMeasFR2-r16 shall be able to support idle mode DC measurements of.

-	at least 8 inter-RAT NR carriers which are also configured for inter-RAT mobility measurements, and

-	at least 2 inter-RAT NR carrier which are not configured for inter-RAT mobility measurements.

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least 8 inter-RAT NR carriers for idle mode DC measurements comprising of carriers configured for inter-RAT mobility measurements and carriers not configured for inter-RAT mobility measurements.

For idle mode DC measurements on NR inter-RAT carriers, if Srxlev ≤ SnonIntraSearchP and Squal ≤ SnonIntraSearchQ, the NR inter-RAT measurement requirements defined in clause 4.2.2.5.6 shall apply, where UE shall search for and measure inter-RAT layers configured for idle mode DC measurements in preparation for possible reporting. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for NR inter-RAT layers configured for idle mode DC measurements at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2, where UE shall search for and measure NR inter-RAT layers configured for idle mode DC measurements in preparation for possible reporting.

For UE supporting nr-IdleInactiveBeamMeasFR1-r16 and/or nr-IdleInactiveBeamMeasFR2-r16, if the UE is configured with beamMeasConfigIdle-r16 on one or more carriers for idle mode DC measurement, the UE, on each carrier, shall be able to

-	detect a newly detectable inter-RAT NR cell and perform RSRP/RSRQ measurement in preparation for reporting, and

-	detect and acquire the SSB index for a newly detectable inter-RAT NR cell if beamMeasConfigIdle-r16 is configured on this carrier and perform RSRP/RSRQ measurement in preparation for reporting,

within the requirements defined in clause 4.2.2.5.6 plus k*TSSB_index,NR, where k is the number of carriers configured for idle mode DC measurement with beamMeasConfigIdle-r16,  and TSSB_index,NR is the additional time period used to acquire the index of the SSB being measured as defined in Table 4.9.2.4-1.

Table 4.9.2.4-1: TSSB_index,NR

| DRX cycle length [s] | Scaling Factor (N1) |  | TSSB_index,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |
| 0.32 | 1 | 8 | N2 x 1.28 x 1.5 x N1 (N2 x 4 x 1.5 x N1) |
| 0.64 |  | 5 | N2 x 1.28 x N1 (N2 x 2 x N1) |
| 1.28 |  | 4 | N2 x 1.28 x N1 (N2 x 1 x N1) |
| 2.56 |  | 3 | N2 x 2.56 x N1 (N2 x 1 x N1) |
| NOTE 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. NOTE 2: N2 = 3 if the NR inter-RAT carrier for idle mode DC measurement reporting is in FR1, and N2= 5 if the NR inter-RAT carrier for idle mode DC measurement reporting is in FR2. |  |  |  |

For UE supporting nr-IdleInactiveBeamMeasFR1-r16 and/or nr-IdleInactiveBeamMeasFR2-r16, if the UE is configured with beamMeasConfigIdle-r16 for idle mode DC measurement, the UE shall be capable of performing SS-RSRP, SS-RSRQ for at least

-	7 SSBs with different SSB index and/or PCI on an NR inter- RAT layer in FR1,

-	10 SSBs with different SSB index and/or PCI on an NR inter- RAT layer in FR2.

In the absence or expiration of T331, it is up to UE implementation to perform the idle mode DC measurement.

The UE shall be capable of performing SS-RSRP and SS-RSRQ measurements of the carriers for idle mode DC measurements, and the UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the carriers for idle mode DC measurements to higher layers, with measurement accuracy as specified in sub-clauses 9.11.1A and 9.11.2A, respectively. The UE shall be able to report idle mode DC measurements when idle mode DC measurement reporting is requested by the network.

#### 8.2.2.1	E-UTRA – NR FR1 Early Measurement Reporting

8.2.2.1.1	Test purpose

The purpose of this test is to verify the requirement for the E-UTRAN to NR inter-RAT Idle mode DC measurement requirements specified in TS 36.133 [23] clause 4.9.2.4 and verify to the accuracy requirement for the E-UTRAN to NR inter-RAT Idle mode DC measurement requirements specified in TS 36.133 [23] clause 9.11.1A and 9.11.2A.

8.2.2.1.2	Test applicability

This test applies to all types of E-UTRA UE from Release 16 onwards, capable of NR measurements and supporting eNB-configured SSB-based RRM measurements of NR FR1 carrier(s) in RRC_IDLE and in RRC_INACTIVE.

8.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.2.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.2.2.1.

8.2.2.1.4	Test description

8.2.2.1.4.1	Initial conditions

This test shall be tested under any of the test configuration in Table 8.2.2.1.4.1-1.

Table 8.2.2.1.4.1-1: Supported test configurations for E-UTRA - NR FR1 Inter-RAT Early Measurement Reporting

| Test Case ID | Description |
| --- | --- |
| 8.2.2.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.2.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.2.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.2.2.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.2.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.2.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test requirement and the DUT according to the parameters in Table 8.2.2.1.4.1-2.

Table 8.2.2.1.4.1-2: Initial conditions for E-UTRA – NR FR1 Inter-RAT Early Measurement Reporting

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1. 6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.2.2.1.4.1-3.

2.	Message contents are defined in clause 8.2.2.1.4.3.

3.	The test scenario comprises of one NR cell and one E-UTRAN cell. E-UTRA Cell 1 is the PCell and Cell 2 is the neighbour cell. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clauses C.1.0 and C.1.1 and Cell 2 is configured according to clauses C.1.1 and C.1.2.

Table 8.2.1.1.4.1-3: General test parameters for E-UTRA – NR FR1 Inter-RAT Early Measurement Reporting

| Parameter | Unit | Test | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  | configuration |  |  |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRAN Cell 1 |  |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR Cell 2 | Cell 2 is of higher priority. |
| RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1: Cell 1 2: Cell 2 |  |
| DRX cycle length | s | 1, 2, 3, 4, 5, 6 | 1.28 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2, 3, 4, 5, 6 | 3 s |  |
| T1 | s | 1, 2, 3, 4, 5, 6 | 0.5 |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 71 |  |
| T3 | s | 1, 2, 3, 4, 5, 6 | 2 |  |
| T331 | s | 1, 2, 3, 4, 5, 6 | 300 |  |

8.2.2.1.4.2	Test procedure

In this test there are two cells, E-UTRA PCell (Cell 1) and neighbour NR Cell (Cell 2). The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Prior to the start of the test the UE shall be connected to Cell 1. During T1, NR Cell 2 shall be powered off.

At the end of T1, the RRC connection to Cell 1 is released and UE is configured Idle mode DC measurement on the carrier frequency of Cell 2.

1.	Ensure the UE is in State RRC_IDLE with generic procedure parameters Connectivity EUTRA/EPC with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall set the parameters for T1 according to Table 8.8.2.2.1.5-1 and Table 8.8.2.2.1.5-2 as appropriate. T1 starts.

3.	After T1 expires, the SS shall send the RRCConnectionRelease with configuration of Idle mode DC measurement on the carrier frequency of Cell 2.

4.	The SS shall set the parameters for T2 according to Table 8.8.2.2.1.5-1 and Table 8.8.2.2.1.5-2 as appropriate. T2 starts.

5.	During the time period T2 the UE is in Idle mode configured to perform inter-RAT idle mode CA/DC measurements on Cell 2. The UE shall not perform reselection.

6.	After T2 expires, the SS shall set the parameters for T3 according to Table 8.8.2.2.1.5-1 and Table 8.8.2.2.1.5-2 as appropriate. T3 starts.

7.	The SS transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity). During the connection setup the UE is reports idleMeasAvailable-r15 in the RRCConnectionSetupComplete message.

8.	The SS shall send the UEInformationRequest message, requesting UE to report idle mode CA/DC measurements.

9.	If UE responds with UEInformationResponse message containing the idle mode measurement results with SS-RSRP values within the limits in Table 8.2.2.1.5-3 for configuration 1, 2, 4 and 5 and Table 8.2.2.1.5-4 for configuration 3 and 6 and SS-RSRQ values within the limits in Table 8.2.2.1.5-5 for configuration 1, 2, 3, 4, 5 and 6 the number of passed iterations is increased by one. Otherwise, the number of failed iterations is increased by one.

10.	If paging in step 7 succeeds, go to step 12, otherwise switch off the UE.

11.	Switch on the UE and ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR with Connected without release On according to TS 38.508-1 [14] clause 4.5.

12.	Repeat step 2-12 until a test verdict has been achieved.

8.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.2.2.1.4.3-2: SystemInformationBlockType2

| Derivation Path: 36.508 Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| idleModeMeasurementsNR-r16 | true |  |  |
| } |  |  |  |

Table 8.2.2.1.4.3-2: RRCConnectionRelease

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| measIdleConfig-r15 SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 ::= SEQUENCE { |  |  |  |
| carrierFreqNR-r16 | ARFCN of Cell2 |  |  |
| subcarrierSpacingSSB-r16 | kHz15 | 8.2.2.1-1 8.2.2.1-2 8.2.2.1-4 8.2.2.1-5 |  |
| subcarrierSpacingSSB-r16 | kHz30 | 8.2.2.1-3 8.2.2.1-6 |  |
| reportQuantitiesNR-r16 | both |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.1.4.3-2: RRCConnectionSetupComplete

| Derivation Path: 36.508 Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleMeasAvailable-r15 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.1.4.3-3: UEInformationRequest

| Derivation Path: 36.508 Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r15 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.6.1-23B: UEInformationResponse

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| measResultListIdleNR-r16 SEQUENCE { |  |  |  |
| carrierFreqNR-r16 | ARFCN of Cell2 |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE { |  |  |  |
| physCellIdNR-r16 | Cell ID of Cell2 |  |  |
| rsrpResultNR-r16 | INTEGER (0..127) |  |  |
| rsrqResultNR-r16 | INTEGER (0..127) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.2.2.1.5	Test requirement

Tables 8.2.2.1.5-1 and 8.2.2.1.5-2 define the primary level settings including test tolerances for and tables 8.2.2.1.5-3, 8.2.2.1.5-4, 8.2.2.1.5-5 and 8.2.2.1.5-6 define accuracy requirements  E-UTRA - NR FR1 Early Measurement Reporting test case.

Table 8.2.2.1.5-1: Cell specific test parameters for NR cell 2 for E-UTRA – NR FR1 Inter-RAT Early Measurement Reporting

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.2 FR1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6 | SS-RSRP |  |  |
|  | dB | 1, 4 | -infinity | -4 | 4 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4 | -infinity | -3.7 | -3.7 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -infinity | -101.7 | -101.7 |
|  |  | 2, 5 | -infinity | -101.7 | -101.7 |
|  |  | 3, 6 | -infinity | -98.7 | -98.7 |
| SS-RSRQ Note3 | dB | 1, 4 | -infinity | -16.03 | -16.03 |
|  |  | 2, 5 | -infinity | -16.03 | -16.03 |
|  |  | 3, 6 | -infinity | -16.03 | -16.03 |
| Io | dBm/9.36 MHz | 1, 4 | -70.05 | -68. 51 | -68. 51 |
|  | dBm/9.36 MHz | 2, 5 | -70.05 | -68. 51 | -68. 51 |
|  | dBm/38.16 MHz | 3, 6 | -63.94 | -62.40 | -62.40 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 8.2.2.1.5-2: Cell specific test parameters for E-UTRA cell 1 for E-UTRA – NR FR1 Inter-RAT Early Measurement Reporting

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -84 |
| RSRQ Note 3 | dB | -10.96 | -10.96 | -10.96 |
|  | dB | 14 | 14 | 14 |
|  | dB | 14 | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | 50 |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| beamMeasConfigIdle |  | True |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 8.2.2.1.5-3: Absolute accuracy requirements for the Cell 2 SS-RSRP reported values for test configurations 1 and 2

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 2) | 44 |
| Highest reported value (Cell 2) | 66 |

Table 8.2.2.1.5-4: Absolute accuracy requirements for the Cell 2 SS-RSRP reported values for test configurations 3

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 2) | 47 |
| Highest reported value (Cell 2) | 69 |

Table 8.2.2.1.5-5: Absolute accuracy requirements for the Cell 2 SS-RSRQ reported values for test configurations 1, 2 and 3

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 2) | 44 |
| Highest reported value (Cell 2) | 65 |

At the beginning of the time-period T2 the connection is released, and UE enters idle mode. During the time period T2 the UE is in Idle mode and Cell 2 is active. The UE shall not perform reselection. The UE shall perform Idle Mode DC measurement according to clause 4.9.2.4 in TS 36.133 [23]. UE shall be able to detect, acquire the SSB index and measure the SS-RSRP and SS-RSRQ from Cell 2 for Idle mode DC measurement during T2.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Section 9.1.2B in TS 36.133 [23] and Section 9.1.3B, respectively and test ends.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

#### 8.2.2.2	E-UTRA – NR FR2 Early Measurement Reporting

Editor's Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

- E-UTRA - FR2 OTA testability is still FFS.

8.2.2.2.1	Test purpose

The purpose of this test is to verify the requirement for the E-UTRAN to NR inter-RAT Idle mode DC measurement requirements specified in TS 36.133 [23] clause 4.9.2.4 and verify to the accuracy requirement for the E-UTRAN to NR inter-RAT Idle mode DC measurement requirements specified in TS 36.133 [23] clause 9.11.1A and 9.11.2A.

8.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards, capable of NR measurements and supporting eNB-configured SSB-based RRM measurements of NR FR2 carrier(s) in RRC_IDLE and in RRC_INACTIVE.

8.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.2.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.2.2.2.

8.2.2.2.4	Test description

8.2.2.2.4.1	Initial conditions

This test shall be tested under any of the test configuration in Table 8.2.2.2.4.1-1.

Table 8.2.2.2.4.1-1: Supported test configurations for E-UTRA - NR FR2 Inter-RAT Early Measurement Reporting

| Test ID | Description |
| --- | --- |
| 8.2.2.2-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.2.2.2-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test requirement and the DUT according to the parameters in Table 8.2.2.2.4.1-2.

Table 8.2.2.2.4.1-2: Initial conditions for E-UTRA - NR FR2 Inter-RAT Early Measurement Reporting

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.2.2.2.4.1-3.

2.	Message contents are defined in clause 8.2.2.2.4.3.

3.	The test scenario comprises of one NR cell and one E-UTRAN cell. E-UTRA Cell 1 is the PCell and Cell 2 is the neighbour cell. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clauses C.1.0 and C.1.1 and Cell 2 is configured according to clauses C.1.1 and C.1.2.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I. Ensure that the UE is receiving NR Cell 2 from the Rx Beam Peak direction.

Table 8.2.2.2.4.1-3: General test parameters for E-UTRA – NR FR2 Inter-RAT Early Measurement Reporting

| Parameter | Unit | Test | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  | configuration |  |  |
| Active cell |  | 1, 2 | E-UTRAN Cell 1 | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.3.7.2.2. |
| Neighbour cell |  | 1, 2 | NR Cell 2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 | One FR2 NR carrier frequency is used. |
| DRX cycle length | s | 1, 2 | 1.28 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  |
| T1 | s | 1, 2 | 0.5 |  |
| T2 | s | 1, 2 | 128 |  |
| T3 | s | 1, 2 | 2 |  |
| T331 | s | 1, 2 | 300 |  |

8.2.2.2.4.2	Test procedure

Same test procedure as in 8.2.2.1.4.2.

8.2.2.2.4.3	Message contents

Same message content as in 8.2.2.1.4.3.

8.2.2.2.5	Test requirement

Table8.2.2.2.5-1 define the primary level settings including test tolerances and tables 8.2.2.2.5-2, 8.2.2.2.5-3 define accuracy requirements for E-UTRA - NR FR2 Early Measurement Reporting test case.

Table 8.2.2.2.5-1: Cell specific test parameters for NR cell 2 for E-UTRA – NR FR2 Inter-RAT Early Measurement Reporting

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  |
| PDSCH Reference measurement channel |  | 1, 2 | SR.3.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1, 2 | CR.3.1 TDD |  |  |
| RMC CORESET Reference Channel |  | 1, 2 | CCR.3.1 TDD |  |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |  |
| BWchannel | MHz | 1, 2 | 100 |  |  |
| Data RBs allocated |  | 1, 2 | 66 |  |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| SSB configuration |  | 1, 2 | SSB.1 FR2 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  |
| RLM-RS |  |  | SSB |  |  |
| Qrxlevmin |  |  | -140 |  |  |
| Pcompensation |  |  | 0 |  |  |
| Qhysts |  |  | 0 |  |  |
| Qoffsets, n |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  |  | SS-RSRP |  |  |
| AoA setup |  |  | Setup 1 defined in A.9.1 |  |  |
| Beam assumptionNote 4 |  |  | Rough |  |  |
| UE baseband | dB | 1, 4 | -infinity | -3.66 | -3.66 |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |
| Note2 | dBm/15kHz | 1, 2 | -107 |  |  |
|  | dB | 1, 4 | -infinity | -2.4 | -2.4 |
| SSbRP Note3 | dBm/SCS | 1, 4 | -infinity | -100.4 | -100.4 |
| SS-RSRQ Note3 | dB | 1, 4 | -infinity | -16.01 | -16.01 |
| Io | dBm/95.04MHz | 1, 4 | -68.98 | -67.01 | -67.01 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP, SS-RSRQ levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |

Table 8.2.2.2.5-2: Cell specific test parameters for E-UTRA cell 1 for E-UTRA – NR FR2 Inter-RAT Early Measurement Reporting

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -84 |
| RSRQ Note 3 | dB | -10.96 | -10.96 | -10.96 |
|  | dB | 14 | 14 | 14 |
|  | dB | 14 | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | N/A |  |  |
| Threshx, highP | dB | 51 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| beamMeasConfigIdle |  | False |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 8.2.2.2.5-3: Absolute accuracy requirements for the Cell 2 SS-RSRP reported values

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 2) | 28 |
| Highest reported value (Cell 2) | 86 |

Table 8.2.2.2.5-4: Absolute accuracy requirements for the Cell 2 SS-RSRQ reported values

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 2) | 44 |
| Highest reported value (Cell 2) | 65 |

At the beginning of the time-period T2 the connection is released, and UE enters idle mode. During the time period T2 the UE is in Idle mode and Cell 2 is active. The UE shall not perform reselection. The UE shall perform Idle Mode DC measurement according to clause 4.9.2.4 in TS 36.133 [23]. UE shall be able to detect, acqure the SSB index and measure the SS-RSRP and SS-RSRQ from Cell 2 for Idle mode DC measurement during T2.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Section 9.1.2B in TS 36.133 [23] and Section 9.1.3B, respectively and test ends.

The rate of correct events observed during repeated tests shall be at least 90% with confidence level of 95%.

## 8.3	RRC_CONNECTED state mobility

### 8.3.1	Inter-RAT cell handover

#### 8.3.1.0	Minimum conformance requirements

##### 8.3.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 handover

[TS 36.133, clause 5.3.4.2]

When the UE receives a RRC message implying inter-RAT handover to the UE shall be ready to start the transmission of the uplink PRACH channel in NR within Dhandover seconds from the end of the last TTI containing the RRC command. Dhandover is defined as:

Dhandover = TRRC_procedure_delay + Tinterruption

Where:

TRRC_procedure_delay: it is the RRC procedure delay which is [50] ms.

Tinterruption: it is the time between end of the last TTI containing the RRC command on the PDSCH in E-UTRAN and the time the UE starts transmission of the PRACH in NR, excluding TRRC_procedure_delay. Tinterruption is defined in TS 36.133 clause 5.3.4.3.

[TS 36.133, clause 5.3.4.3]

When inter-RAT handover to NR is commanded, the interruption time shall be less than Tinterruption

Tinterruption = Tsearch + TIU + Trs+20 ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknowncell and target cell Es/Iot ≥ [-2] dB, then Tsearch = 3▪Trs + 2 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

Trs is the SMTC period of the target NR cell if the UE has been provided with an SMTC configuration for the target cell prior to, or in the handover command, otherwise Trs is the target cell SSB transmission period, if such is provided. If the UE is not provided with an SMTC configuration or SSB transmission period, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If UE is provided with both SMTC configuration and SSB transmission period the requirement shall be based on SMTC periodicity.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in clauses 8.1.2.4.21 and 8.1.2.4.22.

The normative reference for this requirement is TS 38.133 [6] clause 5.3.4.2 and 5.3.4.3.

#### 8.3.1.1	E-UTRA - NR FR1 handover with known target cell

8.3.1.1.1	Test purpose

The purpose of this test is to verify the E-UTRAN to NR FR1 handover requirements as specified in TS 36.133 [23] clause 5.3.4.

8.3.1.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements.

##### 8.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.3.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.3.1.1.

8.3.1.1.4	Test description

The test comprises of one E-UTRA carrier and one NR carrier. There are two cells and one cell on each carrier. Cell 1 is the E-UTRAN PCell and Cell 2 is an inter-RAT NR neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 of TS 36.133 [23] is configured before T2 begins to enable inter-RAT frequency monitoring.

8.3.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.3.1.1.4.1-1.

Table 8.3.1.1.4.1-1: Supported test configurations for
E-UTRA - NR FR1 handover with known target cell

| Configuration | Description |
| --- | --- |
| 8.3.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.3.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.3.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.3.1.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.3.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.3.1.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.3.1.1.4.1-2.

Table 8.3.1.1.4.1-2: Initial conditions for E-UTRA - NR FR1 handover with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.3.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.3.1.1.4.1-3.

2.	Message contents are defined in clause 8.3.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1.

Table 8.3.1.1.4.1-3: General test parameters for E-UTRA - NR FR1 handover with known target cell

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | E-UTRAN cell |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | -83 | Absolute E-UTRAN RSRP threshold for event B2 |
| b2-Threshold2NR |  | dBm | As specified in Table 8.3.1.1.5-2 | Absolute NR SS-RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts [23] |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

8.3.1.1.4.2	Test procedure

The test comprises of one E-UTRA carrier and one NR carrier. There are two cells and one cell on each carrier. Cell 1 is the E-UTRAN and Cell 2 is an inter-RAT NR neighbour cell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.3.1.1.4.1-3, 8.3.1.1.5-1 and 8.3.1.1.5-2, respectively.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 of TS 36.133 [23] is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.3.1.1.5-1 and Table 8.3.1.1.5-2 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.3.1.1.5-1 and Table 8.3.1.1.5-2 respectively. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B2.

7.	SS shall transmit a MobilityFromEUTRACommand message implying handover to Cell 2.

8.	The start of T3 is the instant when the last TTI containing the MobilityFromEUTRACommand message implying handover is sent to the UE, at that instant the SS shall switch the power setting from T2 to T3 as specified in Table 8.3.1.1.5-1 and Table 8.3.1.1.5-2.

9.	The UE shall transmit an RRCReconfigurationComplete message on Cell 2.

10.	If the UE transmits the uplink PRACH channel to Cell 2 less than 112 ms from the beginning of time period T3 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

11.	After T3 expires,

-	The SS could switch the parameters setting from T3 to T1 as specified in Table 8.3.1.1.5-1 and Table 8.3.1.1.5-2 respectively.

-	If the UE handover back to E-UTRA Cell 1, then T1 starts, skip to step 3. Otherwise, switch off  and on the UE. Then ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3. E-UTRA Cell 1 is the active cell.

-	Or, switch off and on the UE Then ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3. E-UTRA Cell 1 is the active cell.

12.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved

8.3.1.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.3.1.1.4.3-1: Common Exception messages for E-UTRA -
NR FR1 handover with known target cell

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.3-2; Table H.3.4-1a Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapUE Table H.3.4-5 with Condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 8.3.1.1-1, 8.3.1.1-2, 8.3.1.1-4 and 8.3.1.1-5 | Table H.3.4-6 with Condition SSB.1 FR1, SMTC.1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.3.1.1-3 and 8.3.1.1-6 | Table H.3.4-6 with Condition SSB.2 FR1, SMTC.1 and Asynchronous cells |

8.3.1.1.5	Test requirement

Table 8.3.1.1.4.1-3, 8.3.1.1.5-1 and 8.3.1.1.5-2 define the primary level settings including test tolerances for E-UTRA - NR FR1 handover with known target cell.

Table 8.3.1.1.5-1: E-UTRAN PCell specific test parameters for E-UTRA -
NR FR1 handover with known target cell

| Parameter | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |  |
|  |  | 4, 5, 6 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 7 | 5.45 | 5.45 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | 7 | 5.45 | 5.45 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -91 | -92.55 | -92.55 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -91 | -92.55 | -92.55 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -62.43 | -63.68 | -63.68 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. NOTE 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |  |

Table 8.3.1.1.5-2: NR neighbour cell specific test parameters for E-UTRA -
NR FR1 handover with known target cell

| Parameter |  | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |
| Duplex mode |  |  | 1, 4 | FDD |  |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |  |
| b2-Threshold2NR |  | dBm | 1, 2, 4, 5 | -106 |  |  |
|  |  |  | 3, 6 | -103 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -98 |  |  |
|  |  |  | 3, 6 | -95 |  |  |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | -infinity | 1.55 | 1.55 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | -infinity | 1.55 | 1.55 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -infinity | -96.45 | -96.45 |
|  |  |  | 3, 6 | -infinity | -93.44 | -93.44 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -70.05 | -66.19 | -66.19 |
|  |  | dBm/38.16 MHz | 3, 6 | -63.94 | -60.09 | -60.09 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 112 ms from the beginning of time period T3.

The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

-	RRC procedure delay = 50 ms and is specified in TS 36.133 [23].

-	Tinterrupt = 62 ms in the test; Tinterrupt is defined in TS 36.133 [23] clause 5.3.4.3.

This gives a total of 112 ms.

The rate of correct handovers observed during repeated tests shall be at least 90%.

## 8.4	Measurement procedures

### 8.4.1	SFTD measurement delay

#### 8.4.1.0	Minimum conformance requirements

##### 8.4.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 SFTD measurement delay

[TS 36.133, clause 8.1.2.4.25.1]

The UE shall perform inter-RAT SFTD measurement and report SFTD result with/without SS-RSRP after the network requests with reportSFTD-Meas set to neighborCells. The overall delay includes RRC procedure delay to be defined in clause 11.2 in TS 36.331 and SFTD measurement reporting delay in TS 36.133 clause 8.1.2.4.25.3.

[TS 36.133, clause 8.1.2.4.25.2]

The requirements on SFTD measurement delay defined in this clause are applicable under the side condition SCH Ês/Iot ≥ -3 dB for the NR cell. Depending on configuration, the SFTD measurement may be carried out with or without the support of configured measurement gaps. In the current release, indication on whether to carry out the SFTD measurement with or without measurement gaps is implicit and depending on whether measurement gaps are configured.

The UE shall be able to detect, identify and measure SFTD of up to 3 of the strongest NR cells on the carrier frequency provided in the SFTD measurement configuration. Further depending on the SFTD measurement configuration, the UE shall additionally report SS-RSRP for the one or more NR cells. The UE may or may not be configured with cellsForWhichToReportSFTD. The UE does not expect cellsForWhichToReportSFTD to change during an ongoing SFTD measurement.

When no measurement gaps are provided, the UE shall be capable of finding the NR cell regardless of its SSB position in the SMTC period. The SFTD measurement shall be conducted with sustained connection to the E-UTRA PCell and activated SCell(s), however, the UE may be allowed to cause a certain amount of interruptions for reconfiguration of the radio receiver, as specified in TS 36.133 clause 7.35.

When measurement gaps are provided, the UE shall be capable of finding the NR cell under the additional condition that the SSB at least occasionally falls within the measurement gap.

When no MCG DRX is used, the UE shall be capable of determining SFTD within a physical layer measurement period of Tmeasure_SFTD1 as follows:

-	For SFTD measurements without measurement gaps, and without additional SS-RSRP reporting:

-	For NR carrier in FR1: Tmeasure_SFTD1 = [14] SMTC periods

-	For SFTD measurements in measurement gaps, and without additional SS-RSRP reporting:

-	For NR carrier in FR1: Tmeasure_SFTD1 = [Nfreq × 8 × max(MGRP, SMTC period)]

-	For SFTD measurements without measurement gaps, and with additional SS-RSRP reporting:

-	For NR carrier in FR1: Tmeasure_SFTD1 = [19] SMTC periods

-	For SFTD measurements in measurement gaps, and with additional SS-RSRP reporting:

-	For NR carrier in FR1: Tmeasure_SFTD1 = [Nfreq × 13 × max(MGRP, SMTC period)]

Where Nfreq is the number of carriers monitored in measurement gaps.

When MCG DRX is used, the same Tmeasure_SFTD1 as for non-DRX applies, but the reporting delay depends on the DRX cycle length in use.

In case an NR PSCell is added, the UE shall terminate the inter-RAT SFTD measurement.

In case PCell is changed due to handover, the UE shall terminate the inter-RAT SFTD measurement.

[TS 36.133, clause 8.1.2.4.25.3]

The SFTD measurement reporting delay is defined as the time between a command that will trigger an SFTD measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When the UE is configured to perform SRS carrier-based switching, an additional delay can be expected.

The SFTD measurement reporting delay shall be less than Tmeasure_SFTD1 defined in TS 36.133, clause 8.1.2.4.25.2.

#### 8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX

8.4.1.1.1	Test purpose

The purpose of this test is to partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell in FR1 is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [23] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and no DRX is configured.

8.4.1.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of inter-RAT SFTD measurements.

8.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.1.1.

8.4.1.1.4	Test description

Two carriers are used in this test: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier with the NR neighbour cell (Cell 2).

This test consists of a single time period of duration T1. Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 1 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report. No measurement gaps are provided and no DRX is configured in this test.

8.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.1.1.4.1-1.

Table 8.4.1.1.4.1-1: Supported test configurations for E-UTRA -
NR FR1 SFTD measurement delay in non-DRX

| Config | Description |
| --- | --- |
| 8.4.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.1.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.1.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.1.1.4.1-2.

Table 8.4.1.1.4.1-2: Initial conditions for E-UTRA - NR FR1 SFTD measurement delay in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.1.1.4.1-3.

2.	Message contents are defined in clause 8.4.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup for this test. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clauses C.1.0 and C.1.1.

Table 8.4.1.1.4.1-3: General test parameters for E-UTRA -
NR FR1 SFTD measurement delay in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One NR FR1 carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| SSB configuration |  | Config 1,4 | SSB.1 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Frame time offset between serving and neighbour cells | ms | Config 1,2,4,5 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | s | Config 3,6 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| T1 | s | Config 1,2,3,4,5,6 | 1 |  |  |

8.4.1.1.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general test parameters are given in Table 8.4.1.1.4.1-3. The test has two subtests with different frame time offset and SFN offset. Cell specific test parameters for the E-UTRA PCell and neighbour cell are given in Table 8.4.1.1.5-1 and 8.4.1.1.5-2, respectively.

In the measurement control information, it is indicated to the UE that inter-RAT SFTD measurement on Cell 2 is used. This test consists of single time period with time duration of T1. Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only.

Configure the frame time offset and SFN offset according to Test 1 in Table 8.4.1.1.4.1-3.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 2 according to T1 in Table 8.4.1.1.5-2, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	UE shall transmit a MeasurementReport message triggered by SFTD measurement reporting. If the overall delays measured from the beginning of time period T1 is less than 297 ms then the number of successful tests is increased by one. If the UE fails to report within the overall delays measured requirement then the number of failure tests is increased by one.

6.	After the SS receive the MeasurementReport message in step 6) or when T1 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

8.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

10	Set the frame time offset and SFN offset for Test 2 according to Table 8.4.1.1.4.1-3 and repeat steps 1 to 9.

8.4.1.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.1.1.4.3-1: Common Exception messages for E-UTRA -
NR FR1 SFTD measurement delay in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with Condition Inter-RAT and SFTD; Table H.3.4-4 with Condition INTER-RAT NR, SFTD and GAPLESS |
| Specific message contents exceptions for Test Configuration 8.4.1.1-1 8.4.1.1-2, 8.4.1.1-4 and 8.4.1.1-5 | Table H.3.4-6 with Condition SSB.1 FR1 andSMTC.1 |
| Specific message contents exceptions for Test Configuration 8.4.1.1-3 and 8.4.1.1-6 | Table H.3.4-6 with Condition SSB.2 FR1 andSMTC.1 |

8.4.1.1.5	Test requirement

Table 8.4.1.1.4.1-3, 8.4.1.1.5-1 and 8.4.1.1.5-2 define the primary level settings including test tolerances for E-UTRA - NR FR1 SFTD measurement delay in non-DRX.

Table 8.4.1.1.5-1: E-UTRA Cell specific test parameters for E-UTRA -
NR FR1 SFTD measurement delay in non-DRX

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note5 | dBm/15 kHz | -87 |
| SCH_RP Note5 | dBm/15 kHz | -87 |
| Io Note5 | dBm/Ch BW | -59.13+10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table 8.4.1.1.5-2: NR Cell specific test parameters for E-UTRA -
NR FR1 SFTD measurement delay in non-DRX

| Parameter | Unit | Test configuration | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |
| Duplex mode |  | Config 1,4 | FDD |
|  |  | Config 2,3,5,6 | TDD |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |
|  |  | Config 2,5 | 10: NRB,c = 52 |
|  |  | Config 3,6 | 40: NRB,c = 106 |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |
|  |  | Config 3,6 | TDDConf.2.1 |
| OCNG Pattern defined in A.3.2.1.1 |  | Config 1,2,3,4,5,6 | OP.1 |
| SMTC configuration |  | Config 1,2,3,4,5,6 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |
|  |  | Config 3,6 | 30 |
| EPRE ratio of PSS to SSS | dB | Config 1,2,3,4,5,6 | 0 |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 | dB |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 | dB |  |  |
| Noc Note2 | dBm/15kHz |  | -98 |
| Noc Note2 | dBm/SCS | Config 1,2,4,5 | -98 |
|  |  | Config 3,6 | -95 |
| SS-RSRP Note 3, 4 | dBm/SCS | Config 1,2,4,5 | -94 |
|  |  | Config 3,6 | -91 |
| Ês/Iot | dB | Config 1,2,3,4,5,6 | 4 |
| Ês/Noc | dB | Config 1,2,3,4,5,6 | 4 |
| Io Note 3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 |
|  | dBm/38.16MHz | Config 3,6 | -58.49 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T1, to the moment the UE send one SFTD measurement triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

-	Overall delays measured = measurement reporting delay + TTI insertion uncertainty

-	Measurement reporting delay = TRRC_procedure_delay + Tmeasure_SFTD1

TRRC_procedure_delay = 15 ms, is the RRC procedure delay for RRC reconfiguration, which is defined in TS 36.331 [2] Clause 11.2

Tmeasure_SFTD1 = 280 ms, is the SFTD measurement report delay defined in TS 36.133 [23] clause 8.1.2.4.25 and 8.1.2.4.26 for FDD and TDD E-UTRA Cell 1, respectively.

-	TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 297 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX

8.4.1.2.1	Test purpose

The purpose of this test is:

-	To partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell in FR1 is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [23] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and DRX is configured.

8.4.1.2.2	Test applicability

This test applies to all types of E-UTRA UE Release 15 and forward supporting inter-RAT SFTD measurements  and long DRX cycle.

8.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.1.2.

8.4.1.2.4	Test description

Two carriers are used in this test: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier with the NR neighbour cell (Cell 2).

This test consists of a single time period of duration T1. Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 1 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report. No measurement gaps are provided and DRX is configured in this test.

8.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.1.2.4.1-1.

Table 8.4.1.2.4.1-1: Supported test configurations for E-UTRA -
NR FR1 SFTD measurement delay in DRX

| Config | Description |
| --- | --- |
| 8.4.1.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.1.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.1.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.1.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.1.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.1.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.1.2.4.1-2.

Table 8.4.1.2.4.1-2: Initial conditions for E-UTRA - NR FR1 SFTD measurement delay in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.1.2.4.1-3.

2.	Message contents are defined in clause 8.4.1.2.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup for this test. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clauses C.1.0 and C.1.1.

Table 8.4.1.2.4.1-3: General test parameters for E-UTRA - NR FR1 SFTD measurement delay in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One NR FR1 carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| SSB Configuration |  | Config 1,4 | SSB.1 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2,3,4,5,6 | DRX.4 |  | DRX configuration as specified in TS 38.133 [6] clause A.3.3.4 |
| Frame time offset between serving and neighbour cells | ms | Config 1,2,4,5 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | s | Config 3,6 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| T1 | s | Config 1,2,3,4,5,6 | 1 |  |  |

8.4.1.2.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general test parameters are given in Table 8.4.1.2.4.1-3. The test has two subtests with different frame time offset and SFN offset. Cell specific test parameters for the E-UTRA PCell and neighbour cell are given in Table 8.4.1.2.5-1 and 8.4.1.2.5-2, respectively.

In the measurement control information, it is indicated to the UE that inter-RAT SFTD measurement on Cell 2 is used. This test consists of single time period with time duration of T1. Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only.

Configure the frame time offset and SFN offset according to Test 1 in Table 8.4.1.2.4.1-3.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 2 according to T1 in Table 8.4.1.2.5-2, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	UE shall transmit a MeasurementReport message triggered by SFTD measurement reporting. . If the overall delays measured from the beginning of time period T1 is less than 455 ms then the number of successful tests is increased by one, otherwise the number of failure tests is increased by one.

6.	After the SS receive the MeasurementReport message in step 6) or when T1 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod1008) for next iteration of the test procedure loop.

8.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

10	Set the frame time offset and SFN offset for Test 2 according to Table 8.4.1.2.4.1-3 and repeat steps 1 to 9.

8.4.1.2.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions.

Table 8.4.1.2.4.3-1: Common Exception messages for E-UTRA -
NR FR1 SFTD measurement delay in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with Condition Inter-RAT and SFTD; Table H.3.4-4 with Condition INTER-RAT NR, SFTD and GAPLESS Table H.3.7-2 with Condition DRX.4 |
| Specific message contents exceptions for Test Configuration 8.4.1.1-1, 8.4.1.1-2, 8.4.1.1-4 and 8.4.1.1-5 | Table H.3.4-6 with Condition SSB.1 FR1 and SMTC.1 and |
| Specific message contents exceptions for Test Configuration 8.4.1.1-3 and 8.4.1.1-6 | Table H.3.4-6 with Condition SSB.2 FR1 and SMTC.1 |

8.4.1.2.5	Test requirement

Table 8.4.1.2.4.1-3, 8.4.1.2.5-1 and 8.4.1.2.5-2 define the primary level settings including test tolerances for E-UTRA - NR FR1 SFTD measurement delay in DRX.

Table 8.4.1.2.5-1: E-UTRA Cell specific test parameters for E-UTRA -
NR FR1 SFTD measurement delay in DRX

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note5 | dBm/15 kHz | -87 |
| SCH_RP Note5 | dBm/15 kHz | -87 |
| Io Note5 | dBm/Ch BW | -59.13+10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table 8.4.1.2.5-2: NR Cell specific test parameters for E-UTRA -
NR FR1 SFTD measurement delay in DRX

| Parameter | Unit | Test configuration | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |
| Duplex mode |  | Config 1,4 | FDD |
|  |  | Config 2,3,5,6 | TDD |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |
|  |  | Config 2,5 | 10: NRB,c = 52 |
|  |  | Config 3,6 | 40: NRB,c = 106 |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |
|  |  | Config 3,6 | TDDConf.2.1 |
| OCNG Pattern defined in A.3.2.1.1 |  | Config 1,2,3,4,5,6 | OP.1 |
| SMTC configuration |  | Config 1,2,3,4,5,6 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |
|  |  | Config 3,6 | 30 |
| EPRE ratio of PSS to SSS | dB | Config 1,2,3,4,5,6 | 0 |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 | dB |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 | dB |  |  |
| Noc Note2 | dBm/15kHz |  | -98 |
| Noc Note2 | dBm/SCS | Config 1,2,4,5 | -98 |
|  |  | Config 3,6 | -95 |
| SS-RSRP Note 3, 4 | dBm/SCS | Config 1,2,4,5 | -94 |
|  |  | Config 3,6 | -91 |
| Ês/Iot | dB | Config 1,2,3,4,5,6 | 4 |
| Ês/Noc | dB | Config 1,2,3,4,5,6 | 4 |
| Io Note 3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 |
|  | dBm/38.16MHz | Config 3,6 | -58.49 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T1, to the moment the UE send one SFTD measurement triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

-	Overall delays measured = measurement reporting delay + TTI insertion uncertainty

-	Measurement reporting delay = TRRC_procedure_delay + Tmeasure_SFTD1 + TDRX_transmission

TRRC_procedure_delay = 15 ms, is the RRC procedure delay for RRC reconfiguration, which is defined in TS 36.331 [2] Clause 11.2

Tmeasure_SFTD1 = 280 ms, is the SFTD measurement report delay defined in TS 36.133 [23] clause 8.1.2.4.25 and 8.1.2.4.26 for FDD and TDD E-UTRA Cell 1, respectively. When DRX is used, the same Tmeasure_SFTD1 as for non-DRX applies.

TDRX_transmission = 158 ms, it is the time until the next transmission in the worst case that the last PDDCH is missed at 280ms from the beginning of T1.

-	TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 455 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 8.4.2	Inter-RAT measurements

#### 8.4.2.0	Minimum conformance requirements

##### 8.4.2.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement

The UE shall be able to identify new inter-RAT E-UTRAN  NR cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-RAT cells if carrier frequency information is provided by the PCell, even if no explicit neighbour list with physical layer cell identities is provided.

When measurement gaps are scheduled, the UE shall be able to identify a new detectable cell within Tidentify_irat_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise, UE shall be able to identify a new detectable inter-RAT frequency cell within Tidentify_irat_with_index. The UE shall be able to identify a new detectable inter-RAT SS block of an already detected cell within Tidentify_irat_without_index.

Tidentify_irat_without_index = (TPSS/SSS_sync_irat + T SSB_measurement_period_irat) ms

Tidentify_irat_with_index = (TPSS/SSS_sync_irat + T SSB_measurement_period_irat + TSSB_time_index_irat) ms

Where:

TPSS/SSS_sync_irat: it is the time period used in PSS/SSS detection given in table 8.4.2.0.1-1 and table 8.4.2.0.1-2.

TSSB_time_index_irat: it is the time period used to acquire the index of the SSB being measured given in table 8.4.2.0.1-3 and table 8.4.2.0.1-4.

T SSB_measurement_period_irat: equal to a measurement period of SSB based measurement given in table 8.4.2.0.1-5 and table 8.4.2.0.1-6.

Mpss/sss_sync_irat: For a UE supporting FR2 power class 1, Mpss/sss_sync_ira=64 samples. For a UE supporting FR2 power class 2 (vehicle mounted), Mpss/sss_sync_irat=40 samples. For a UE supporting FR2 power class 3 (handheld), Mpss/sss_sync_irat=40 samples. For a UE supporting FR2 power class 4, Mpss/sss_syn_iratc=40 samples.

MSSB_index_irat: For a UE supporting power class 1, MSSB_index_irat =40 samples. For a vehicle mounted UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_irat=24 samples.For a UE supporting power class 3 (handheld), MSSB_index_irat =24 samples. For a UE supporting power class 4, Mmeas_period_irat =24 samples.

Mmeas_period_irat: For a UE supporting FR2 power class 1, Mmeas_period_irat =64 samples. For a vehicle mounted UE supporting FR2 power class 2 (vehicle mounted), Mpss/sss_sync_irat=40 samples.For a UE supporting FR2 power class 3 (handheld), Mmeas_period_irat =40 samples. For a UE supporting FR2 power class 4, Mmeas_period_irat =40 samples.

Nfreq is defined in TS 36.133 [23] clause 8.1.2.1.1.

For per-FR measurement gap capable UE, when serving cells are in E-UTRA and measurement objects are only in FR2:

-	UE can perform such measurements without gap; and

-	UE fulfils the requirements for FR2 measurement objects based on effective MGRP = 20 ms.

Table 8.4.2.0.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | max[600ms, 8 x max(MGRP, SMTC period)] x Nfreq |
| DRX cycle ≤ 320ms | max[600ms, ceil(8x1.5) x max(MGRP, SMTC period, DRX cycle)] x Nfreq |
| DRX cycle > 320ms | 8 x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 8.4.2.0.1-1A: Time period for PSS/SSS detection for UE configured
with highSpeedInterRAT-r16 (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle < 320ms | Max(600ms, ceil( 8 × M) × max(MGRP, SMTC period, DRX cycle)) ×Nfreq |
| DRX cycle ≥ 320ms | 8× DRX cycle ×Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 of TS 38.133 [3]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 of TS 38.133 [3] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms. |  |

Table 8.4.2.0.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, Mpss/sss_sync_irat x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(600ms, (1.5 x Mpss/sss_sync_irat) x max(MGRP, SMTC period, DRX cycle)) x Nfreq |
| DRX cycle > 320ms | Mpss/sss_sync_irat x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 8.4.2.0.1-3: Time period for time index detection (Frequency range FR1)

| Condition NOTE1,2 | TSSB_time_index_irat |
| --- | --- |
| No DRX | Max(120ms, 3 x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(120ms, ceil(3 x 1.5) x max(MGRP, SMTC period,DRX cycle)) x Nfreq |
| DRX cycle > 320ms | 3 x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 8.4.2.0.1-3A: Time period for time index detection for UE configured
with highSpeedInterRAT-r16 (Frequency range FR1)

| Condition NOTE1,2 | TSSB_time_index_irat |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle < 320ms | Max(120ms, Ceil(3 M) Max(MGRP, SMTC period, DRX cycle)) Nfreq |
| DRX cycle ≥ 320ms | 3 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 of TS 38.133 [3]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 of TS 38.133 [3] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms. |  |

Table 8.4.2.0.1-4: Time period for time index detection (Frequency range FR2)

| Condition NOTE1,2 | TSSB_time_index_irat |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_irat x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 x MSSB_index_irat) x max(MGRP, SMTC period, DRX cycle)) x Nfreq |
| DRX cycle > 320ms | MSSB_index_irat x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

In the requirements, an NR cell is considered to be detectable when:

-	NR SS-RSRP related conditions in the accuracy requirements in clause 9.11.1 are fulfilled for a corresponding Band, together with the corresponding side conditions in clause B.2.3 of TS 38.133 [6],

-	NR SS-RSRQ related conditions in the accuracy requirements in clause 9.11.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in clause B.2.3 of TS 38.133 [6],

-	NR SS-SINR related conditions in the accuracy requirements in clause 9.11.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in clause B.2.3 of TS 38.133 [6].

When measurement gaps are scheduled for NR measurements the UE physical layer shall be capable of reporting NR SS-RSRP, SS-RSRQ, and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 36.133 [23] clause 9.11, with measurement period as shown in table 8.4.2.0.1-5 and table 8.4.2.0.1-6:

Table 8.4.2.0.1-5: Measurement period for inter-RAT frequency measurements (Frequency FR1)

| Condition NOTE1,2 | T SSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, ceil(8 x 1.5) x max(MGRP, SMTC period, DRX cycle)) x Nfreq |
| DRX cycle > 320ms | 8 x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 8.4.2.0.1-5A: Measurement period for inter-RAT measurements for UE configured
with highSpeedInterRAT-r16 (Frequency range FR1)

| Condition NOTE1,2 | TSSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle < 320ms | Max(200ms, ceil(8 × M) x max(MGRP, SMTC period, DRX cycle))×Nfreq |
| DRX cycle ≥ 320ms | 4× M DRX cycle ×Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 of TS 38.133 [50]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 of TS 38.133 [50] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms. |  |

Table 8.4.2.0.1-6: Measurement period for inter-RAT  frequency measurements (Frequency FR2)

| Condition NOTE1,2 | T SSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(400ms, Mmeas_period_irat x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(400ms, (1.5 x Mmeas_period_irat) x max(MGRP, SMTC period, DRX cycle)) x Nfreq |
| DRX cycle > 320ms | Mmeas_period_irat x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] clause 3.6.1. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

The UE shall be capable of performing NR SS-RSRP, SS-RSRQ, and SS-SINR for up to 7 NR carrier frequencies.

For each RAT E-UTRAN - NR layer on FR1 or FR2, the UE shall be capable of monitoring at least 4 cells.

For each RAT E-UTRAN - NR layer on FR1, during each layer 1 measurement period, the UE shall be capable of monitoring at least 7 SSBs with different SSB index and/or PCI on the RAT E-UTRA -NR.

For each RAT E-UTRA - NR layer on FR2, during each layer 1 measurement period, the UE shall be capable of monitoring at least 10 SSBs with different SSB index and/or PCI on the RAT E-UTRAN - NR layer. The UE shall be capable of monitoring at least one SSB per cell.

The NR SS-RSRP measurement accuracy for all measured cells shall be as specified in TS 36.133 [23] clause 9.11.1. The NR SS-RSRQ measurement accuracy for all measured cells shall be as specified in TS 36.133 [23] clause 9.11.2. The NR SS-SINR measurement accuracy for all measured cells shall be as specified in TS 36.133 [23] clause 9.11.3.

Reported measurements in event triggered measurement reports shall meet the requirements in TS 36.133 [23] clause 9.

The UE shall not send any event triggered measurement reports, as long as the reporting criteria is not fulfilled.

The measurement reporting delay is defined as the time between any events that will trigger a measurement report until the UE starts to transmit the measurement report over the Uu interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is twice the TTI of the uplink DCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_irat_without_index or Tidentify_irat_with_index for the minimum requirements. When L3 filtering is used or IDC autonomous denial or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_irat_without_index or Tidentify_irat_with_index for the minimum requirements and then triggers the measurement report as per TS 36.331 [2], the event triggered measurement reporting delay shall be less than Tmeasurement_NR_FDD provided the timing to that cell has not changed more than 3200 Tc while measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

The normative reference for this requirement is TS 36.133 [23] clause 8.1.2.4.21 and 8.1.2.4.22.

##### 8.4.2.0.2	Void

#### 8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX

8.4.2.1.1	Test purpose

The purpose of this test isto verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements for E-UTRA FDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.21 and for E-UTRAN TDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.22.

8.4.2.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

8.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.1.

8.4.2.1.4	Test description

The test consists of two sub-tests with two cells configured, the E-UTRA PCell and NR neighbour cell; the difference between the two sub-tests is whether per-FR measurement gap is configured or not. Each sub-test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR neighbour cell. In all sub-tests the UE is not required to report SSB time index.

8.4.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.1.4.1-1.

Table 8.4.2.1.4.1-1: Supported test configurations for E-UTRA - NR FR1 event-triggered
reporting without SSB time index detection in non-DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.1.4.1-2.

Table 8.4.2.1.4.1-2: Initial conditions for E-UTRA - NR FR1 event-triggered reporting
without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.1.4.1-3.

2.	Message contents are defined in clause 8.4.2.1.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 8.4.2.1.5-1 for this test. Cell 1 is configured according to clauses C.1.1 and C.1.2.

Table 8.4.2.1.4.1-3: General test parameters for E-UTRA - NR FR1 event-triggered reporting
without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  | 1, 2, 3, 4, 5, 6 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | 19 | As specified in TS 36.331 [29]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [29] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [29] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 1 | 1 |  |
| NOTE 1: The value of b2-Threshold1 is defined in Table 8.4.2.1.5-1. NOTE 2: The value of b2-Threshold2NR is defined in Table 8.4.2.1.5-2. |  |  |  |  |  |

8.4.2.1.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.1.4.1-3, 8.4.2.1.5-1 and 8.4.2.1.5-2, respectively.

In sub-test 1 measurement gap pattern configuration #0 as defined in Table 8.4.2.1.4.1-3 is provided for UE that does not support per-FR gap and in sub-test 2 measurement gap pattern configuration #4 as defined in Table 8.4.2.1.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.1.5-1 and Table 8.4.2.1.5-2 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.1.5-1 and Table 8.4.2.1.5-2 respectively. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

-	922 for sub-test 1,

-	802 for sub-test 2.

7.	After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch off the UE.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	The SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions.

Table 8.4.2.1.4.3-1: Common Exception messages for E-UTRA - NR FR1 event-triggered
reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with Condition Inter-RAT and EVENT B2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 for Test 1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 for Test 2; Table H.3.4-5 with Condition Pattern #0 for Test 1; Table H.3.4-5 with Condition Pattern #4 for Test 2; |
| Specific message contents exceptions for Test Configuration 8.4.2.1-1 and 8.4.2.1-4 | Table H.3.4-6 with Conditions SMTC.2, SSB.1 FR1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.1-2 and 8.4.2.1-5 | Table H.3.4-6 with Conditions SMTC.1, SSB.1 FR1 and Synchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.1-3 and 8.4.2.1-6 | Table H.3.4-6 with Conditions SMTC.1, SSB.2 FR1 and Synchronous cells |

Table 8.4.2.1.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: TS 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.1-1/2/3 |
|  | 22 |  | Config 8.4.2.1-4/5/6 |
| } |  |  |  |
| } |  |  |  |

8.4.2.1.5	Test requirement

Table 8.4.2.1.4.1-3, 8.4.2.1.5-1 and 8.4.2.1.5-2 define the primary level settings including test tolerances for E-UTRA - NR FR1 event-triggered reporting without SSB time index detection in non-DRX.

Table 8.4.2.1.5-1: E-UTRAN PCell specific test parameters for E-UTRA - NR FR1
event-triggered reporting without SSB time index detection in non-DRX

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -56.12 | -56.12 |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 8.4.2.1.5-2: NR neighbour cell specific test parameters for E-UTRA - NR FR1
event-triggered reporting without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.94 | -56.15 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 922 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 802 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is not required to report SSB time index.

NOTE:	The overall delay requirement is defined as the delay requirement in TS 38.133 plus 2 ms corresponding to the TTI insertion uncertainty of the measurement report in DCCH.

#### 8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX

8.4.2.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements given in clause 8.1.2.4.21 of TS 36.133 for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 for E-UTRAN TDD-NR measurements.

8.4.2.2.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards supporting long DRX cycle and capable of NR measurements. Test 1 and Test 2 are applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

8.4.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.2.

8.4.2.2.4	Test description

The test consists of four sub-tests with two cells configured, the E-UTRA PCell and NR neighbour cell; the difference between the four sub-tests is whether per-FR measurement gap is configured or not and the DRX configuration parameters. Each sub-test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR neighbour cell. In all sub-tests the UE is not required to report SSB time index.

8.4.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.2.4.1-1.

Table 8.4.2.2.4.1-1: Supported test configurations for E-UTRA -
NR FR1 event-triggered reporting without SSB time index detection in DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.2.4.1-2.

Table 8.4.2.2.4.1-2: Initial conditions for E-UTRA - NR FR1 event-triggered reporting
without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.2.4.1-3.

2.	Message contents are defined in clause 8.4.2.2.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 8.4.2.2.5-1 for this test. Cell 1 is configured according to clauses C.1.1 and C.1.2.

Table 8.4.2.2.4.1-3: General test parameters for E-UTRA - NR FR1 event-triggered reporting
without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  | 19 |  | As specified in TS 36.331 [29]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  |  |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [29] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [29] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 | DRX.9 | DRX.12 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 | 11 | 2 | 11 |  |
| NOTE 1: The value of b2-Threshold1 is defined in Table 8.4.2.2.5-1. NOTE 2: The value of b2-Threshold2NR is defined in Table 8.4.2.2.5-2. |  |  |  |  |  |  |  |

8.4.2.2.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.2.4.1-3, 8.4.2.2.5-1 and 8.4.2.2.5-2, respectively.

In sub-test 1 and sub-test 2 measurement gap pattern configuration #0 as defined in Table 8.4.2.2.4.1-3 is provided for UE that does not support per-FR gap and in sub-test 3 and sub-test 4 measurement gap pattern configuration #4 as defined in Table 8.4.2.2.4.1-3 is provided for UE that supports per-FR gap.

DRX cycle = 40 ms is used in sub-test 1 and sub-test 3, DRX cycle = 640 ms is used in sub-test 2 and sub-test 4. The UE is allocated with PUSCH resource at every DRX cycle.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.2.5-1 and Table 8.4.2.2.5-2 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.2.5-1 and Table 8.4.2.2.5-2 respectively. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

-	1082 for sub-test 1 and sub-test 3,

-	10242 for sub-test 2 and sub-test 4.

7.	.After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

- switch off the UE.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	The SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.2.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.2.4.3-1: Common Exception messages for E-UTRA - NR FR1 event-triggered
reporting without SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with Condition Inter-RAT and EVENT B2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 for Test 1 and Test 2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 for Test 3 and Test 4; Table H.3.4-5 with Condition Pattern #0 for Test 1 and Test 2; Table H.3.4-5 with Condition Pattern #4 for Test 3 and Test 4; Table H.3.7-2 with Condition DRX.9 and Gap for Test 1 and Test 3 Table H.3.7-2 with Condition DRX.12 and Gap for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 8.4.2.2-1 and 8.4.2.2-4 | Table H.3.4-6 with Conditions SMTC.2, SSB.1 FR1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.2-2 and 8.4.2.2-5 | Table H.3.4-6 with Conditions SMTC.1, SSB.1 FR1 and Synchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.2-3 and 8.4.2.2-6 | Table H.3.4-6 with Conditions SMTC.1, SSB.2 FR1 and Synchronous cells |

Table 8.4.2.2.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: TS 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.2-1/2/3 |
|  | 22 |  | Config 8.4.2.2-4/5/6 |
| } |  |  |  |
| } |  |  |  |

8.4.2.2.5	Test requirement

Table 8.4.2.2.4.1-3, 8.4.2.2.5-1 and 8.4.2.2.5-2 define the primary level settings including test tolerances for E-UTRA - NR FR1 event-triggered reporting without SSB time index detection in DRX.

Table 8.4.2.2.5-1: E-UTRAN PCell specific test parameters for E-UTRA - NR FR1 event-triggered
reporting without SSB time index detection in DRX

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -56.12 | -56.12 |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 8.4.2.2.5-2: NR neighbour cell specific test parameters for E-UTRA - NR FR1 event-triggered reporting without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.15 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1082 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 10242 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1082 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 10242 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is not required to report SSB time index.

NOTE:	The overall delay requirement is defined as the delay requirement in TS 38.133 plus 2 ms corresponding to the TTI insertion uncertainty of the measurement report in DCCH.

#### 8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX

8.4.2.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements for E-UTRA FDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.21 and for E-UTRAN TDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.22.

8.4.2.3.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

8.4.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.3.

8.4.2.3.4	Test description

The test consists of two sub-tests with two cells configured, the E-UTRA PCell and NR neighbour cell; the difference between the two sub-tests is whether per-FR measurement gap is configured or not. Each sub-test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR neighbour cell. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell.

8.4.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.3.4.1-1.

Table 8.4.2.3.4.1-1: Supported test configurations for E-UTRA - NR FR1 event-triggered
reporting with SSB time index detection in non-DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.3-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.3-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.3-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.3-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.3-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.3-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.3.4.1-2.

Table 8.4.2.3.4.1-2: Initial conditions for E-UTRA - NR FR1 event-triggered reporting
with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.3.4.1-3.

2.	Message contents are defined in clause 8.4.2.3.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 8.4.2.3.5-1 for this test. Cell 1 is configured according to clauses C.1.1 and C.1.2.

Table 8.4.2.3.4.1-3: General test parameters for E-UTRA - NR FR1 event-triggered reporting
with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  | 1, 2, 3, 4, 5, 6 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | 19 | As specified in TS 36.331 [29]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [29] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [29] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 | 1 |  |
| NOTE 1: The value of b2-Threshold1 is defined in Table 8.4.2.3.5-1. NOTE 2: The value of b2-Threshold2NR is defined in Table 8.4.2.3.5-2. |  |  |  |  |  |

8.4.2.3.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.3.4.1-3, 8.4.2.3.5-1 and 8.4.2.3.5-2, respectively.

In sub-test 1 measurement gap pattern configuration #0 as defined in Table 8.4.2.3.4.1-3 is provided for UE that does not support per-FR gap and in sub-test 2 measurement gap pattern configuration #4 as defined in Table 8.4.2.3.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.3.5-1 and Table 8.4.2.3.5-2 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.3.5-1 and Table 8.4.2.3.5-2 respectively. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

-	1042 for sub-test 1,

-	922 for sub-test 2.

7.	.	After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

- switch off the UE.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	The SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.3.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions.

Table 8.4.2.3.4.3-1: Common Exception messages for E-UTRA - NR FR1 event-triggered
reporting with SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with Condition Inter-RAT and EVENT B2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 for Test 1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 for Test 2; Table H.3.4-5 with Condition Pattern #0 for Test 1; Table H.3.4-5 with Condition Pattern #4 for Test 2; Table H.3.4-8 with Condition SSB-Index |
| Specific message contents exceptions for Test Configuration 8.4.2.3-1 and 8.4.2.3-4 | Table H.3.4-6 with Conditions SMTC.2, SSB.1 FR1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.3-2 and 8.4.2.3-5 | Table H.3.4-6 with Conditions SMTC.1, SSB.1 FR1 and Synchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.3-3 and 8.4.2.3-6 | Table H.3.4-6 with Conditions SMTC.1, SSB.2 FR1 and Synchronous cells |

Table 8.4.2.3.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.3-1/2/3 |
|  | 22 |  | Config 8.4.2.3-4/5/6 |
| } |  |  |  |
| } |  |  |  |

8.4.2.3.5	Test requirement

Table 8.4.2.3.4.1-3, 8.4.2.3.5-1 and 8.4.2.3.5-2 define the primary level settings including test tolerances for E-UTRA - NR FR1 event-triggered reporting with SSB time index detection in non-DRX.

Table 8.4.2.3.5-1: E-UTRAN PCell specific test parameters for E-UTRA - NR FR1 event-triggered reporting with SSB time index detection in non-DRX

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -56.12 | -56.12 |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 8.4.2.3.5-2: NR neighbour cell specific test parameters for E-UTRA - NR FR1 event-triggered reporting with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.15 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1042 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 922 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is required to report SSB time index.

NOTE:	The overall delay requirement is defined as the delay requirement in TS 38.133 plus 2 ms corresponding to the TTI insertion uncertainty of the measurement report in DCCH.

#### 8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX

8.4.2.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements for E-UTRA FDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.21 and for E-UTRAN TDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.22.

8.4.2.4.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards supporting long DRX cycle and capable of NR measurements. Test 1 and Test 2 are applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

8.4.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.4.

8.4.2.4.4	Test description

The test consists of four sub-tests with two cells configured, the E-UTRA PCell and NR neighbour cell; the difference between the four sub-tests is whether per-FR measurement gap is configured or not and the DRX configuration. Each subtest consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR neighbour cell. In measurements configuration UE shall be indicated to report the SSB index of the identified NR cell.

8.4.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.4.4.1-1.

Table 8.4.2.4.4.1-1: Supported test configurations for E-UTRA -
NR FR1 event-triggered reporting with SSB time index detection in DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.4-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.4-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.4-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.4-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.4-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.4-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.4.4.1-2.

Table 8.4.2.4.4.1-2: Initial conditions for E-UTRA - NR FR1 event-triggered reporting
with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.4.4.1-3.

2.	Message contents are defined in clause 8.4.2.4.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 8.4.2.4.5-1 for this test. Cell 1 is configured according to clauses C.1.1 and C.1.2.

Table 8.4.2.4.4.1-3: General test parameters for E-UTRA -
NR FR1 event-triggered reporting with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  | 19 |  | As specified in TS 36.331 [29]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  |  |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [29] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [29] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 | DRX.9 | DRX.12 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 | 13 | 2 | 13 |  |
| NOTE 1: The value of b2-Threshold1 is defined in Table 8.4.2.4.5-1. NOTE 2: The value of b2-Threshold2NR is defined in Table 8.4.2.4.5-2. |  |  |  |  |  |  |  |

8.4.2.4.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.4.4.1-3, 8.4.2.4.5-1 and 8.4.2.4.5-2, respectively.

In sub-test 1 and sub-test 2 measurement gap pattern configuration #0 as defined in Table 8.4.2.4.4.1-3 is provided for UE that does not support per-FR gap and in sub-test 3 and sub-test 4 measurement gap pattern configuration #4 as defined in Table 8.4.2.4.4.1-3 is provided for UE that supports per-FR gap.

DRX cycle = 40 ms is used in sub-test 1 and sub-test 3, DRX cycle = 640 ms is used in sub-test 2 and sub-test 4. The UE is allocated with PUSCH resource at every DRX cycle.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.4.5-1 and Table 8.4.2.4.5-2 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.4.5-1 and Table 8.4.2.4.5-2 respectively. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

-	1282 for sub-test 1 and sub-test 3,

-	12162 for sub-test 2 and sub-test 4.

7.	.	After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

- switch off the UE.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	The SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.4.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.4.4.3-1: Common Exception messages for E-UTRA -
NR FR1 event-triggered reporting with SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with Condition Inter-RAT and EVENT B2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 for Test 1 and Test 2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 for Test 3 and Test 4; Table H.3.4-5 with Condition Pattern #0 for Test 1 and Test 2; Table H.3.4-5 with Condition Pattern #4 for Test 3 and Test 4; Table H.3.4-8 with Condition SSB-Index Table H.3.7-2 with Condition DRX.9 and Gap for Test 1 and Test 3 Table H.3.7-2 with Condition DRX.12 and Gap for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 8.4.2.4-1 and 8.4.2.4-4 | Table H.3.4-6 with Conditions SMTC.2, SSB.1 FR1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.4-2 and 8.4.2.4-5 | Table H.3.4-6 with Conditions SMTC.1, SSB.1 FR1 and Synchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.4-3 and 8.4.2.4-6 | Table H.3.4-6 with Conditions SMTC.1, SSB.2 FR1 and Synchronous cells |

Table 8.4.2.4.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.4-1/2/3 |
|  | 22 |  | Config 8.4.2.4-4/5/6 |
| } |  |  |  |
| } |  |  |  |

8.4.2.4.5	Test requirement

Table 8.4.2.4.4.1-3, 8.4.2.4.5-1 and 8.4.2.4.5-2 define the primary level settings including test tolerances for E-UTRA - NR FR1 event-triggered reporting with SSB time index detection in DRX.

Table 8.4.2.4.5-1: E-UTRAN PCell specific test parameters for E-UTRA -
NR FR1 event-triggered reporting with SSB time index detection in DRX

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -56.12 | -56.12 |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 8.4.2.4.5-2: NR neighbour cell specific test parameters for E-UTRA -
NR FR1 event-triggered reporting with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.15 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1282 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 12162 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1282 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 12162 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is required to report SSB time index.

NOTE:	The overall delay requirement is defined as the delay requirement in TS 38.133 plus 2 ms corresponding to the TTI insertion uncertainty of the measurement report in DCCH.

#### 8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX

Editor's Note: This test case is incomplete:

-	EUTRA – NR FR2 testability issue is not cleared

-	The TT analysis is complete for test frequency f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	The test is incomplete for UE power classes other than PC3

-	The test is incomplete for test frequencies > 40.8 GHz

8.4.2.5.1	Test purpose

To verify the UE's ability to make a correct reporting of an event without SSB time index detection in non-DRX within E-UTRA - NR FR2 cell search requirements.

8.4.2.5.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting Gap Pattern Id 4.

8.4.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.5.

8.4.2.5.4	Test description

8.4.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.5.4.1-1.

Table 8.4.2.5.4.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in non-DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.5-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.4.2.5-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.5.4.1-2.

Table 8.4.2.5.4.1-2: Initial conditions for E-UTRA - NR FR2 event-triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.5.4.1-3.

2.	Message contents are defined in clause 8.4.2.5.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR FR2 neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to clause A.6.1.2 for this test.

Table 8.4.2.5.4.1-3: General test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.6.1.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [29]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [29]. |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used. |
| DRX |  | 1, 2 | OFF |  | DRX is not used. |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 10 |  |  |
| T2 | s | 1, 2 | 6 | 3 |  |
| NOTE 1: The value of b1-ThresholdNR is defined in Table 8.4.2.5.5-1. |  |  |  |  |  |

8.4.2.5.4.2	Test procedure

In test 1 measurement gap pattern configuration # 0 as defined in Table 8.4.2.5.4.1-3 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 8.4.2.5.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [29] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have timing information of NR cell 2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to Table A.6.1.2-1 and T1 in Table 8.4.2.5.5-1 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.5.5-1. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B1. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

-	3202 for sub-test 1,

-	1602 for sub-test 2.

7.	.	After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

- switch off the UE.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.5.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.5.4.3-1: Common Exception messages for E-UTRA - NR FR2 event-triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with condition INTER-RAT and EVENT B1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 for Test 1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and GAPLESS for Test 2; Table H.3.4-5 with Condition Pattern #0 for Test 1; Table H.3.4-5 with Condition Pattern #4 for Test 2 |
| Specific message contents exceptions for Test Configuration 8.4.2.5-1 | Table H.3.4-6 with Conditions SMTC.2, SSB.3 FR2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.5-2 | Table H.3.4-6 with Conditions SMTC.1, SSB.3 FR2 and Synchronous cells |

Table 8.4.2.5.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.5-1/2/3 |
|  | 22 |  | Config 8.4.2.5-4/5/6 |
| } |  |  |  |
| } |  |  |  |

8.4.2.5.5	Test requirement

Tables 8.4.2.5.4.1-3 and 8.4.2.5.5-1 define the primary level settings including test tolerances for E-UTRA - NR FR2 event-triggered reporting without SSB time index detection in non-DRX.

Table 8.4.2.5.5-1: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| AoA setup defined in A.9.2.1 |  |  | 1, 2 | Setup 2a |  |
| Assumption for UE beamsNote 5 |  |  | 1, 2 | Rough |  |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 24 |  |
| OCNG patterns defined in A.3.2.1.1 |  |  | 1, 2 | OP. 3 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -118 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Ês |  | dBm/SCS | 1, 2 | - Infinity | -80.6 |
| SS B_RP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -80.6 |
| BB Note 6 |  | dB | 1, 2 | -Infinity | 8.3 |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -Infinity | -56.0 |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |

Table 8.4.2.5.5-2: Test requirements for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Test case | Measurement reporting delay (ms) |  |
| --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms |
| UE power class 3 | 3202 | 1602 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays specified in 36.133 [23] because of TTI insertion uncertainty of the measurement report in DCCH.

In test 1 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is not required to report SSB time index.

#### 8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX

Editor's Note: This test case is incomplete:

-	EUTRA – NR FR2 testability issue is not cleared

-	The TT analysis is complete for test frequency f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	The test is incomplete for UE power classes other than PC3

-	The test is incomplete for test frequencies > 40.8 GHz

8.4.2.6.1	Test purpose

To verify the UE's ability to make a correct reporting of an event without SSB time index detection in DRX within E-UTRA - NR FR2 cell search requirements.

8.4.2.6.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards supporting long DRX cycle and capable of NR FR2 measurements. Test 1 and Test 2 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting Gap Pattern Id 4.

8.4.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.6.

8.4.2.6.4	Test description

8.4.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.6.4.1-1.

Table 8.4.2.6.4.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.6-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.4.2.6-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.6.4.1-2.

Table 8.4.2.6.4.1-2: Initial conditions for E-UTRA - NR FR2 event-triggered reporting without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.6.4.1-3.

2.	Message contents are defined in clause 8.4.2.6.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR FR2 neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to clause A.6.1.2 for this test.

Table 8.4.2.6.4.1-3: General test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  | Test 3 | Test 4 |  |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  |  |  |  |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.6.1.2. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  |  | 4 |  |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  |  | 19 |  |  | As specified in TS 36.331 [29]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  |  |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [29]. |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  | L3 filtering is not used. |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 |  | DRX.9 | DRX.12 |  | As specified in clause A.3.3. |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  |  |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  |  |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 6 |  | 83 | 6 |  | 83 |  |
| NOTE: The value of b1-ThresholdNR is defined in Table 8.4.2.6.5-1. |  |  |  |  |  |  |  |  |  |

8.4.2.6.4.2	Test procedure

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table 8.4.2.6.4.1-3 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table 8.4.2.6.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [29] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have timing information of NR cell 2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to Table A.6.1.2-1 and T1 in Table 8.4.2.6.5-1 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.6.5-1. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B1. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

-	4802 for sub-test 1 and sub-test 3,

-	51202 for sub-test 2 and sub-test 4.

7.	After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

- switch off the UE

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.6.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.6.4.3-1: Common Exception messages for E-UTRA - NR FR2 event-triggered reporting without SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with condition INTER-RAT and EVENT B1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 for Test 1 and Test 2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and GAPLESS for Test 3 and Test 4; Table H.3.4-5 with Condition Pattern #0 for Test 1 and Test 2; Table H.3.4-5 with Condition Pattern #4 for Test 3 and Test 4; Table H.3.7-2 with Condition DRX.9 for Test 1 and Test 3 Table H.3.7-2 with Condition DRX.12 for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 8.4.2.6-1 | Table H.3.4-6 with Conditions SMTC.2, SSB.3 FR2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.6-2 | Table H.3.4-6 with Conditions SMTC.1, SSB.3 FR2 and Synchronous cells |

Table 8.4.2.6.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.6-1/2/3 |
|  | 22 |  | Config 8.4.2.6-4/5/6 |
| } |  |  |  |
| } |  |  |  |

8.4.2.6.5	Test requirement

Tables 8.4.2.6.4.1-3 and 8.4.2.6.5-1 define the primary level settings including test tolerances for E-UTRA - NR FR2 event-triggered reporting without SSB time index detection in DRX.

Table 8.4.2.6.5-1: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| AoA setup defined in A.9.1 |  |  | 1, 2 | Setup 1 |  |
| Assumption for UE beamsNote 5 |  |  | 1, 2 | Rough |  |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.3.2.1.1 (OP.1) |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -106 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/15kHz | 1, 2 | -104.7 |  |
| Note2 |  | dBm/SCS | 1, 2 | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -87.7 |
|  |  | dB | 1, 2 | -Infinity | 8 |
|  |  | dB | 1, 2 | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -66.7 | -58.0 |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Table 8.4.2.6.5-2: Test requirements for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Test case | Measurement reporting delay (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms | Test 3: D3 ms | Test 4: D4 ms |
| UE power class 3 | 4802 | 51202 | 4802 | 51202 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays specified in 36.133 [23] because of TTI insertion uncertainty of the measurement report in DCCH.

In test 1 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D3 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D4 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is not required to report SSB time index.

#### 8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX

Editor's Note: This test case is incomplete:

-	EUTRA – NR FR2 testability issue is not cleared

-	The TT analysis is complete for test frequency f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	The test is incomplete for UE power classes other than PC3

-	The test is incomplete for test frequencies > 40.8 GHz

8.4.2.7.1	Test purpose

To verify the UE's ability to make a correct reporting of an event with SSB time index detection in non-DRX within E-UTRA - NR FR2 cell search requirements.

8.4.2.7.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting Gap Pattern Id 4.

8.4.2.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.7.

8.4.2.7.4	Test description

8.4.2.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.7.4.1-1.

Table 8.4.2.7.4.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR2 in non-DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.7-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.4.2.7-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.7.4.1-2.

Table 8.4.2.7.4.1-2: Initial conditions for E-UTRA - NR FR2 event-triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.7.4.1-3.

2.	Message contents are defined in clause 8.4.2.7.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR FR2 neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to clause A.6.1.2 for this test.

Table 8.4.2.7.4.1-3: General test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  | 1, 2 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Numbers |  | 1, 2 | 1 |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.6.1.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [29]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [29] |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used. |
| DRX |  | 1, 2 | OFF |  | DRX is not used. |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 5 | 3 |  |
| NOTE: The value of b1-ThresholdNR is defined in Table A.8.4.2.7.5-1. |  |  |  |  |  |

8.4.2.7.4.2	Test procedure

In test 1 measurement gap pattern configuration # 0 as defined in Table 8.4.2.7.4.1-3 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 8.4.2.7.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [29] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to Table A.6.1.2-1 and T1 in Table 8.4.2.7.5-1 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.7.5-1. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B1. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

-	4162 for sub-test 1,

-	2082 for sub-test 2.

7.	After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

-	transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch off the UE.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.7.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.7.4.3-1: Common Exception messages for E-UTRA - NR FR2 event-triggered reporting with SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with condition INTER-RAT and EVENT B1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 for Test 1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and GAPLESS for Test 2; Table H.3.4-5 with Condition Pattern #0 for Test 1; Table H.3.4-5 with Condition Pattern #4 for Test 2 |
| Specific message contents exceptions for Test Configuration 8.4.2.7-1 | Table H.3.4-6 with Conditions SMTC.2, SSB.3 FR2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.7-2 | Table H.3.4-6 with Conditions SMTC.1, SSB.3 FR2 and Synchronous cells |

Table 8.4.2.7.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.7-1/2/3 |
|  | 22 |  | Config 8.4.2.7-4/5/6 |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.7.4.3-3: ReportConfigInterRAT

| Derivation Path: 36.508 [25] Table 4.6.6-7BA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1-NR-r15 SEQUENCE{ |  |  |  |
| b1-Threshold2NR-r15 CHOICE{ |  |  |  |
| nr-RSRP-r15 | 50 | Actual value = 50 -156 = -106dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportRS-Index-r15 | 2 |  |  |
| reportQuantityRS-IndexNR-r15 SEQUENCE { |  |  |  |
| ss-rsrp | TRUE |  |  |
| ss-rsrq | FALSE |  |  |
| ss-sinr | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |

8.4.2.7.5	Test requirement

Tables 8.4.2.7.4.1-3 and 8.4.2.7.5-1 define the primary level settings including test tolerances for E-UTRA - NR FR2 event-triggered reporting with SSB time index detection in non-DRX.

Table 8.4.2.7.5-1: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| AoA setup defined in A.9.1 |  |  | 1, 2 | Setup 1 |  |
| Assumption for UE beamsNote 5 |  |  | 1, 2 | Rough |  |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.3.2.1.1 |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -106 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/15kHz | 1, 2 | -104.7 |  |
| Note2 |  | dBm/SCS | 1, 2 | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -87.7 |
|  |  | dB | 1, 2 | -Infinity | 8 |
|  |  | dB | 1, 2 | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -66.7 | -58.0 |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Table 8.4.2.7.5-2: Test requirements for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Test case | Measurement reporting delay (ms) |  |
| --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms |
| UE power class 3 | 4162 | 2082 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays specified in 36.133 [23] because of TTI insertion uncertainty of the measurement report in DCCH.

In test 1 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is required to report SSB time index.

#### 8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX

Editor's Note: This test case is incomplete:

-	EUTRA – NR FR2 testability issue is not cleared

-	The TT analysis is complete for test frequency f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	The test is incomplete for UE power classes other than PC3

-	The test is incomplete for test frequencies > 40.8 GHz

8.4.2.8.1	Test purpose

To verify the UE's ability to make a correct reporting of an event with SSB time index detection in DRX within E-UTRA - NR FR2 cell search requirements.

8.4.2.8.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards supporting long DRX cycle and capable of NR FR2 measurements. Test 1 and Test 2 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting Gap Pattern Id 4.

8.4.2.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.8.

8.4.2.8.4	Test description

8.4.2.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.8.4.1-1.

Table 8.4.2.8.4.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.8-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.4.2.8-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.8.4.1-2.

Table 8.4.2.8.4.1-2: Initial conditions for E-UTRA - NR FR2 event-triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.8.4.1-3.

2.	Message contents are defined in clause 8.4.2.8.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR FR2 neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to clause A.6.1.2 for this test.

Table 8.4.2.8.4.1-3: General test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Numbers |  | 1, 2 | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Numbers |  | 1, 2 | 1 |  |  |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.6.1.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2 | 39 |  | 19 |  | As specified in TS 36.331 [29]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [29]. |
| Hysteresis | dB | 1, 2 | 0 |  |  |  |  |
| CP length |  | 1, 2 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2 | 0 |  |  |  | L3 filtering is not used. |
| DRX |  |  | DRX.9 | DRX.12 | DRX.9 | DRX.12 | As specified in clause A.3.3. |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |  |  |
| T2 | s | 1, 2 | 7 | 70 | 7 | 70 |  |
| NOTE: The value of b1-ThresholdNR is defined in Table 8.4.2.8.5-1 |  |  |  |  |  |  |  |

8.4.2.8.4.2	Test procedure

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table 8.4.2.8.4.1-3 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table 8.4.2.8.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [29] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to Table A.6.1.2-1 and T1 in Table 8.4.2.8.5-1 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.8.5-1. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B1. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

-	4802 for sub-test 1 and sub-test 3,

-	51202 for sub-test 2 and sub-test 4.

7.	.	After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

- switch off the UE.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.8.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.8.4.3-1: Common Exception messages for E-UTRA - NR FR2 event-triggered reporting with SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with condition INTER-RAT and EVENT B1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 for Test 1 and Test 2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and GAPLESS for Test 3 and Test 4; Table H.3.4-5 with Condition Pattern #0 for Test 1 and Test 2; Table H.3.4-5 with Condition Pattern #4 for Test 3 and Test 4; Table H.3.7-2 with Condition DRX.9 for Test 1 and Test 3; Table H.3.7-2 with Condition DRX.12 for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 8.4.2.8-1 | Table H.3.4-6 with Conditions SMTC.2, SSB.3 FR2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.8-2 | Table H.3.4-6 with Conditions SMTC.1, SSB.3 FR2 and Synchronous cells |

Table 8.4.2.8.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.8-1/2/3 |
|  | 22 |  | Config 8.4.2.8-4/5/6 |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.8.4.3-3: ReportConfigInterRAT

| Derivation Path: 36.508 [25] Table 4.6.6-7BA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1-NR-r15 SEQUENCE{ |  |  |  |
| b1-Threshold2NR-r15 CHOICE{ |  |  |  |
| nr-RSRP-r15 | 50 | Actual value = 50 -156 = -106dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportRS-Index-r15 | 2 |  |  |
| reportQuantityRS-IndexNR-r15 SEQUENCE { |  |  |  |
| ss-rsrp | TRUE |  |  |
| ss-rsrq | FALSE |  |  |
| ss-sinr | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |

8.4.2.8.5	Test requirement

Tables 8.4.2.8.4.1-3 and 8.4.2.8.5-1 define the primary level settings including test tolerances for E-UTRA - NR FR2 event-triggered reporting with SSB time index detection in DRX.

Table 8.4.2.8.5-1: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| AoA setup defined in A.9.1 |  |  | 1, 2 | Setup 1 |  |
| Assumption for UE beamsNote 5 |  |  | 1, 2 | Rough |  |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.3.2.1.1 |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -106 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/15kHz | 1, 2 | -104.7 |  |
| Note2 |  | dBm/SCS | 1, 2 | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -87.7 |
|  |  | dB | 1, 2 | -Infinity | 8 |
|  |  | dB | 1, 2 | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -66.7 | -58.0 |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Table 8.4.2.8.5-2: Test requirements for NR inter-RAT event triggered reporting
for FR2 with SSB time index detection in DRX

| Test case | Measurement reporting delay (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms | Test 3: D3 ms | Test 4: D4 ms |
| UE power class 3 | 6242 | 66562 | 6242 | 66562 |

In test 1 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D3 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D4 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays specified in 36.133 [23] because of TTI insertion uncertainty of the measurement report in DCCH.

#### 8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16

8.4.2.9.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements for E-UTRA FDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.21 and for E-UTRAN TDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.22 when UE is configured with highSpeedInterRAT-NR-r16.

8.4.2.9.2	Test applicability

This test applies to all E-UTRA UEs capable of NR measurements and configured with highSpeedInterRAT-NR-r16 from release 15 onwards.

8.4.2.9.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.9.

8.4.2.9.4	Test description

8.4.2.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.9.4.1-1.

Table 8.4.2.9.4.1-1: Supported test configurations for NR inter-RAT event triggered reporting
for FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Configuration | Description |
| --- | --- |
| 8.4.2.9-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.9-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.9-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.9-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.9-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.9-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.9.4.1-2.

Table 8.4.2.9.4.1-2: Initial conditions for NR inter-RAT event triggered reporting
for FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 8.4.2.9.4.1-3: General test parameters for NR inter-RAT event triggered reporting
for FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.5 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 5 |  |
| NOTE 1: The value of b2-Threshold1 is defined in Table A.8.4.2.9.1-3. NOTE 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.9.1-4. |  |  |  |  |

8.4.2.9.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.9.4.1-3, 8.4.2.9.5-1 and 8.4.2.9.5-2, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2. UE is configured with highSpeedInterRAT-NR-r16.

1. Ensure the UE is in State 3A-RF according to TS 36.508 clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.9.5-1 and Table 8.4.2.9.5-2 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.9.5-1 and Table 8.4.2.9.5-2 respectively. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 4.8s ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.9.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.9.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks on condition of HighSpeedMeas with exceptions | Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with Condition Inter-RAT and EVENT B2; Table H.3.4-4 with Condition INTER-RAT NR and EVENT B2 Table H.3.4-5 with Condition Pattern #0 Table H.3.4-8 with Condition SSB-Index Table H.3.7-2 with Condition DRX.5 and Gap |

Table 8.4.2.9.4.3-2: highSpeedCarrierNR-r16 in SystemInformationBlockType24

| Derivation Path: TS 36.508, Table 4.4.3.3-20 with Condition FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| CarrierFreqNR-v1610 ::= SEQUENCE {(SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| highSpeedCarrierNR-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.4.2.9.4.3-3: highSpeedInterRAT-NR-r16 configuration

| Derivation Path: 36.508 Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB::= SEQUENCE { |  |  |  |
| highSpeedInterRAT-NR-r16 | TRUE |  |  |
| } |  |  |  |

8.4.2.9.5	Test requirement

Tables 8.4.2.9.4.1-3, 8.4.2.9.5-1 and 8.4.2.9.5-2 define the primary level settings including test tolerances for E-UTRA - NR FR2 event-triggered reporting with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16.

Table 8.4.2.9.5-1: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered
reporting with NR neighbour cell in FR1 with SSB time index detection for UE configured
with highSpeedInterRAT-NR-r16

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table 8.4.2.9.5-2: NR neighbour cell specific test parameters for NR inter-RAT event triggered
reporting for FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2, 4, 5 | AWGN1944 |  |
|  |  | 3,6 | AWGN3334 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 4.8s from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## 8.5	Measurement performance requirements

### 8.5.1	SFTD measurement accuracy

#### 8.5.1.0	Minimum conformance requirements

##### 8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements

The SFN and frame timing difference (SFTD) is measured between PCell and NR cell for inter-RAT SFTD. The inter-RAT SFTD measurement can only be configured for E-UTRA - NR band combinations that are supported by the UE.

The accuracy requirements in Table 8.5.1.0.1-3 are applicable under the following conditions:

For PCell SFN and frame timing measurement:

-	Cell specific reference signals are transmitted either from one, two or four antenna ports.

-	Conditions defined in TS 36.101 clause 7.3 for reference sensitivity are fulfilled.

-	No changes to the uplink transmission timing are applied during the measurement period.

-	RSRP|dBm according to TS 36.133 Annex B.3.5 for a corresponding Band.

-	Io range defined in Table 8.5.1.0.1-1.

Table 8.5.1.0.1-1: PCell Io range conditions for SFTD measurement accuracy

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | E-UTRA operating band groups Note 4, 5 | Minimum Io | Maximum Io |
|  |  | dBm/15kHz Note 2, 3 | dBm/BWChannel |
| Conditions | FDD_A, TDD_A | -121 | -50 |
|  | FDD_C, TDD_C | -120 | -50 |
|  | FDD_D | -119.5 | -50 |
|  | FDD_E, TDD_E | -119 | -50 |
|  | FDD_F | -118.5 | -50 |
|  | FDD_G | -118 | -50 |
|  | FDD_H | -117.5 | -50 |
|  | FDD_N | -114.5 | -50 |
| NOTE 1: When in dBm/15kHz, the minimum Io condition is expressed as the average Io per RE over all REs in that symbol. Io may be different in different symbols within a subframe. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in clause B.4.2 and B.4.3. NOTE 3: The condition level is increased by MSD as defined in clause 7.3B in TS 38.101-3 [54], if applicable depending on E-UTRA - NR band combination. NOTE 4: E-UTRA operating band groups are as defined in clause 3.5. NOTE 5: Only E-UTRA bands within EN-DC band combinations as specified in clause 5.5B in TS 38.101-3 [54] are applicable. |  |  |  |

For NR PSCell, or NR cell SFN and frame timing measurement in FR1:

-	Conditions defined in TS 38.101-1 clause 7.3 for reference sensitivity are fulfilled.

-	Io range defined in Table 8.5.1.0.1-2.

Table 8.5.1.0.1-2: NR PSCell, or NR cell Io range conditions for SFTD measurement accuracy in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 4, 5 | Minimum Io Note 2, 3 |  | Maximum Io |
|  |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in clause 7.3B in TS 38.101-3 [54], depending on E-UTRA - NR band combination. NOTE 3: The condition level is increased by MSD as defined in clause 7.3B in TS 38.101-3 [54], if applicable depending on E-UTRA - NR band combination. NOTE 4: NR operating band groups are as defined in clause 3.5. NOTE 5: Only NR bands within EN-DC band combinations as specified in clause 5.5B in TS 38.101-3 [54] are applicable. |  |  |  |  |

Table 8.5.1.0.1-3: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot | Frequency range |
| Ts Note 1 | dB |  |
| 40 | -3 dB | FR1 |
| NOTE 1: Ts is the basic timing unit defined in TS 36.211 [16]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

#### 8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy

8.5.1.1.1	Test purpose

The purpose of this test is to verify that the SFTD measurement accuracy is within the specified limits and to verify the requirements as specified in clause 9.1.27 in TS 36.133 [23] for inter-RAT FR1 SFTD measurements.

8.5.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards and support inter-RAT NR SFTD measurements.

8.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.1.1.

8.5.1.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.1.1.4.1-1.

Table 8.5.1.1.4.1-1: Supported test configurations for E-UTRA - NR FR1 SFTD measurement accuracy

| Configuration | Description |
| --- | --- |
| 8.5.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.1.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.1.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.1.1.4.1-2.

Table 8.5.1.1.4.1-2: Initial conditions for E-UTRA - NR FR1 SFTD measurement accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.5.1.1.4.1-3.

2.	Message contents are defined in clause 8.5.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1.

4.  The SFTD between PCell and NR neighbour cell shall be set to one of the conditions in Table 8.5.1.1.5-3. For negative Frame boundary offset, it can be achieved delaying NR neighbour cell in regards to PCell OR advancing PCell in regards to NR neighbour cell. For positive Frame boundary offset, it can be achieved delaying PCell in regards to NR neighbour cell OR advancing NR neighbour cell in regards to PCell.

8.5.1.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.1.1.5-1 and Table 8.5.1.1.5-3 as appropriate. (Condition = 3)

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit a MeasurementReport message triggered by SFTD measurement.

6.	The SS shall check the reported values of SFN offset and frame boundary offset in the MeasurementReport. The SFN offset value and frame boundary offset value between E-UTRA Cell 1 and Cell 2 reported by the UE is compared to the expected SFN offset value and frame boundary offset, respectively. The number of failed iterations is increased by one, if

-	The value of sfn-OffsetResult is different with the expected SFN offset value, or

-	The value of frameBoundaryOffsetResultis outside the range given in Table 8.5.1.1.5-5, or

-	The UE fails to report the measurement value for Cell 2.

Otherwise the number of successful iterations is increased by one.

7.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3); or

-	switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

8.	Repeat steps 1-7 until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.5.1.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.5.1.1.4.3-1: Common Exception messages for E-UTRA -
NR FR1 SFTD measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 with Condition Inter-RAT and SFTDTable H.3.4-4 with Condition INTER-RAT NR, SFTD and GAPLESS |
| Specific message contents exceptions for Test Configuration 8.5.1.1-1 and 8.5.1.1-4 | Table H.3.4-6 with Condition SSB.1 FR1 and SMTC.1 |
| Specific message contents exceptions for Test Configuration 8.5.1.1-2 and 8.5.1.1-5 | Table H.3.4-6 with Condition SSB.1 FR1 and SMTC.1 |
| Specific message contents exceptions for Test Configuration 8.5.1.1-3 and 8.5.1.1-6 | Table H.3.4-6 with Condition SSB.2 FR1 and SMTC.1 |

8.5.1.1.5	Test requirement

Table 8.5.1.1.5-1 and 8.5.1.1.5-2 defines the primary level settings including test tolerances for E-UTRA - NR FR1 SFTD measurement accuracy.

The SFN offset and frame boundary offset between E-UTRA PCell and the inter-RAT neighbour cell shall be set to one of the configurations in Table 8.5.1.1.5-3. The reported SFTD accuracy shall fulfil the accuracy requirements in Table 8.5.1.1.5-4. The SFN offset in reported SFTD shall match the values in Table 8.5.1.1.5-3 and the frame boundary offset in reported SFTD shall be within the range given in Table 8.5.1.1.5-5.

Table 8.5.1.1.5-1: E-UTRA cell specific test parameters for E-UTRA -
NR FR1 SFTD measurement accuracy

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | -2.2 |
| Ês/Iot | dB | -2.2 |
| RSRP Note5 | dBm/15 kHz | -106.2 |
| SCH_RP Note5 | dBm/15 kHz | -106.2 |
| Io Note5 | dBm/Ch BW | -74.17 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table 8.5.1.1.5-2: NR cell specific test parameters for E-UTRA - NR FR1 SFTD measurement accuracy

| Parameter |  | Config | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~6 |  | freq1 |
| Duplex mode |  | 1,4 |  | FDD |
|  |  | 2,5 |  | TDD |
|  |  | 3,6 |  | TDD |
| TDD Configuration |  | 1,4 |  | N/A |
|  |  | 2,5 |  | TDDConf.1.1 |
|  |  | 3,6 |  | TDDConf.2.1 |
| BWchannel |  | 1,4 | MHz | 10: NRB,c = 52 |
|  |  | 2,5 |  | 10: NRB,c = 52 |
|  |  | 3,6 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1,4 |  | SR.1.1 FDD |
|  |  | 2,5 |  | SR.1.1 TDD |
|  |  | 3,6 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1,4 |  | CR.1.1 FDD |
|  |  | 2,5 |  | CR.1.1 TDD |
|  |  | 3,6 |  | CR.2.1 TDD |
| RMC CORESET Reference Channel |  | 1,4 |  | CCR.1.1 FDD |
|  |  | 2,5 |  | CCR.1.1 TDD |
|  |  | 3,6 |  | CCR.2.1 TDD |
| SSB configuration |  | 1,4 |  | SSB.1 FR1 |
|  |  | 2,5 |  | SSB.1 FR1 |
|  |  | 3,6 |  | SSB.2 FR1 |
| SMTC configuration |  | 1~6 |  | SMTC.1 |
| DL BWP configuration |  | 1~6 |  | DLBWP.1.1 |
| UL BWP configuration |  | 1~6 |  | ULBWP.1.1 |
| OCNG Patterns |  | 1~6 |  | OP.1 |
| EPRE ratio of PSS to SSS |  | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1~6 | dBm/15kHz | -104 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/SSB SCS | -104 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 |  | -101 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  |  | 1~6 | dB | -2.7 |
|  |  | 1~6 | dB | -2.7 |
| SS-RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/SCS | -106.7 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 |  | -103.7 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/9.36 MHz | -74.18 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 | dBm/38.16 MHz | -68.08 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Propagation condition |  | 1~6 |  | AWGN |
| Antenna configuration |  | 1~6 |  | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table 8.5.1.1.5-3: Timing offsets for E-UTRA - NR FR1 SFTD measurement accuracy

| Condition | SFN offset between PCell and PSCell | Frame boundary offset between PCell and PSCell (Ts) |
| --- | --- | --- |
| 1 | 100 | -122000 |
| 2 | 300 | -60540 |
| 3 | 500 | 1000 |
| 4 | 700 | 62540 |
| 5 | 900 | 124000 |

Table 8.5.1.1.5-4: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot | Frequency range |
| Ts Note 1 | dB |  |
| 40 | -3 dB | FR1 |
| NOTE 1: Ts is the basic timing unit defined in TS 36.211 [24]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

Table 8.5.1.1.5-5: E-UTRA - NR FR1 SFTD measurement accuracy requirements for the value of frameBoundaryOffsetResult in reported SFTD

| Normal and Extreme Conditions | frameBoundaryOffsetResult |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Condition 1 | Condition 2 | Condition 3 | Condition 4 | Condition 5 |
| Lowest reported value | -24408 | -12116 | 192 | 12500 | 24792 |
| Highest reported value | -24392 | -12100 | 208 | 12516 | 24808 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 8.5.2	Inter-RAT measurement accuracy

#### 8.5.2.1	SS-RSRP

##### 8.5.2.1.0	Minimum conformance requirements

The measurement period of NR SS-RSRP measurements in RRC_CONNECTED state is specified in TS 36.133 [23] Clause 8.1.2.4.21 for UE with FDD PCell not configured with E-UTRA-NR Dual Connectivity operation. The measurement period of NR SS-RSRP measurements in RRC_CONNECTED state is specified in TS 36.133 [23] Clause 8.1.2.4.22 for UE with TDD PCell not configured with E-UTRA-NR Dual Connectivity operation.

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 36.133 [23] clause 9.11.1.

8.5.2.1.0.1	Inter-RAT E-UTRA - NR FR1 SS-RSRP measurement accuracy requirements

The accuracy requirements of NR SS-RSRP measurements in FR1 and the corresponding side conditions shall be the same as the inter-frequency SS-RSRP absolute accuracy requirements in clause 4.7.1.0.3.

8.5.2.1.0.2	Inter-RAT E-UTRA - NR FR2 SS-RSRP minimum conformance requirements

The accuracy requirements of NR SS-RSRP measurements in FR2 and the corresponding side conditions shall be the same as the inter-frequency SS-RSRP absolute accuracy requirements in clause 5.7.1.0.2.

##### 8.5.2.1.1	SS-RSRP with NR FR1 target cell

8.5.2.1.1.1	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell

8.5.2.1.1.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRP absolute measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.1.1.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements.

8.5.2.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.1.1.1.

8.5.2.1.1.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.1.1.1.4.1-1.

Table 8.5.2.1.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.1.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.1.1.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.1.1.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.1.1.1.4.1-2.

Table 8.5.2.1.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | Without faders |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.1.1.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.1.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1.

8.5.2.1.1.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.1.1.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in clause 8.5.2.1.1.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.1.1.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.1.1.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.5.2.1.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.1.1.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-rsrp | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.1.1.1.5	Test requirement

Table 8.5.2.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 8.5.2.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.1.1.1.5-2 for test configurations 1, 2, 4 and 5, and the corresponding absolute accuracy requirements in Table 8.5.2.1.1.1.5-3 for test configurations 3 and 6.

Table 8.5.2.1.1.1.5-1: SS-RSRP inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -94.65 |  | -117 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/15kHz | -96.00 |  | -117 + ΔBG_offset |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -94.65 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -93.00 |  | -114 + ΔBG_offset |  |
|  |  |  | dB | 10 |  | -3.2 |  |
|  |  |  | dB | 10 |  | -3.2 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -84.65 |  | -120.2 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -83.00 |  | -117.2 + ΔBG_offset |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -56.28 |  | -87.35 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -51.53 |  | -81.25 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

Table 8.5.2.1.1.1.5-2: SS-RSRP Inter RAT absolute accuracy requirements
for the reported values for test configurations 1, 2, 4 and 5

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 30 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 31 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 32 |
|  |  | Bands NR_FDD_FR1_F | 33 |
|  |  | Bands NR_FDD_FR1_G | 33 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 43 |
|  |  | Bands NR_FDD_FR1_B | 43 |
|  |  | Bands NR_TDD_FR1_C | 44 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 44 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 45 |
|  |  | Bands NR_FDD_FR1_F | 45 |
|  |  | Bands NR_FDD_FR1_G | 46 |
|  |  | Bands NR_FDD_FR1_H | 46 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 26 |
|  |  | Bands NR_FDD_FR1_B | 26 |
|  |  | Bands NR_TDD_FR1_C | 27 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 27 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 28 |
|  |  | Bands NR_FDD_FR1_F | 28 |
|  |  | Bands NR_FDD_FR1_G | 29 |
|  |  | Bands NR_FDD_FR1_H | 29 |
| Highest reported value (Cell 2) | 85 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 47 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 48 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 49 |
|  |  | Bands NR_FDD_FR1_F | 50 |
|  |  | Bands NR_FDD_FR1_G | 50 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| NOTE: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 8.5.2.1.1.1.5-3: SS-RSRP Inter RAT absolute accuracy requirements
for the reported values for test configurations 3 and 6

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 64 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  | Bands NR_FDD_FR1_B | 34 |
|  |  | Bands NR_TDD_FR1_C | 34 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
|  |  | Bands NR_FDD_FR1_F | 36 |
|  |  | Bands NR_FDD_FR1_G | 36 |
|  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 46 |
|  |  | Bands NR_FDD_FR1_B | 46 |
|  |  | Bands NR_TDD_FR1_C | 47 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 47 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 48 |
|  |  | Bands NR_FDD_FR1_F | 48 |
|  |  | Bands NR_FDD_FR1_G | 49 |
|  |  | Bands NR_FDD_FR1_H | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 29 |
|  |  | Bands NR_FDD_FR1_B | 29 |
|  |  | Bands NR_TDD_FR1_C | 30 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 30 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 31 |
|  |  | Bands NR_FDD_FR1_F | 31 |
|  |  | Bands NR_FDD_FR1_G | 32 |
|  |  | Bands NR_FDD_FR1_H | 32 |
| Highest reported value (Cell 2) | 86 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  | Bands NR_FDD_FR1_B | 51 |
|  |  | Bands NR_TDD_FR1_C | 51 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 52 |
|  |  | Bands NR_FDD_FR1_F | 53 |
|  |  | Bands NR_FDD_FR1_G | 53 |
|  |  | Bands NR_FDD_FR1_H | 54 |
| NOTE: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

8.5.2.1.1.2	Void

##### 8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell

Editor's Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

8.5.2.1.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRP measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.1.2.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements.

8.5.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.1.2.

8.5.2.1.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.1.2.4.1-1.

Table 8.5.2.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.1.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.1.2.4.1-2.

Table 8.5.2.1.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in clause C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.1.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.1.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.1.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.1.2.5-1 and Table 8.5.2.1.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in clause 8.5.2.1.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.1.2.5-1 and 8.5.2.1.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.1.2.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.5.2.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.1.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| rsrp | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.1.2.5	Test requirement

Table 8.5.2.1.2.5-1 defines the general test parameters and Table 8.5.2.1.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 8.5.2.1.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.1.2.5-3

Table 8.5.2.1.2.5-1: SS-RSRP Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void. NOTE 3: Void. NOTE 4: Void. |  |  |  |

Table 8.5.2.1.2.5-2: SS-RSRP Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to A.9.1 | Setup 1 according to A.9.1 |
| Assumption for UE beamsNote 9 |  | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105.1 | N/A |
| Note1 | dBm/SCS Note4 | -96.07 | N/A |
|  | dB | 11 | N/A |
| Es | dBm/SCS Note 4 |  | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +6.65dB) |
| SSB_RPNote2 | dBm/SCS Note4 | -85.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +6.65dB) |
| BB Note 7 | dB | 9.95 | 1.84 |
| IoNote2 | dBm/95.04 MHz Note4 | -55.75 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +35.65dB) |
| NOTE 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Void NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: Void NOTE 6: Void NOTE 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [3] Table 6.2.1.3-4. NOTE 8: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table 8.5.2.1.2.5-3: evaluation limits for the reported values

| UE power class 3 |  |  |  |
| --- | --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 48 | n257, n258, n261 | 29 |
|  |  | n260 | 32 |
|  |  | n259 | 33 |
| Highest reported value (Cell 2) | 105 | n257, n258, n261 | 87 |
|  |  | n260 | 89 |
|  |  | n259 | 90 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 45 | n257, n258, n261 | 26 |
|  |  | n260 | 29 |
|  |  | n259 | 30 |
| Highest reported value (Cell 2) | 108 | n257, n258, n261 | 90 |
|  |  | n260 | 92 |
|  |  | n259 | 93 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 8.5.2.2	SS-RSRQ

##### 8.5.2.2.0	Minimum conformance requirements

The measurement period of NR SS-RSRQ measurements is the same as the measurement period of NR SS-RSRP measurements defined in clause 8.5.2.1.0.

The reporting range of SS-RSRQ is defined from -43 dB to 20 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 4.7.2.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

The normative reference for this requirement is TS 36.133 [23] clause 9.11.2.

8.5.2.2.0.1	Inter-RAT E-UTRA - NR FR1 SS-RSRQ minimum conformance requirements

The accuracy requirements of NR SS-RSRQ measurements in FR1 and the corresponding side conditions shall be the same as the inter-frequency SS-RSRQ absolute accuracy requirements in clause 4.7.2.0.2.

8.5.2.2.0.2	Inter-RAT E-UTRA - NR FR2 SS-RSRQ minimum conformance requirements

The accuracy requirements of NR SS-RSRP measurements in FR2 and the corresponding side conditions shall be the same as the inter-frequency SS-RSRP absolute accuracy requirements in clause 5.7.2.0.2.

##### 8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell

8.5.2.2.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRQ measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.2.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements.

8.5.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.2.1.

8.5.2.2.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.2.1.4.1-1.

Table 8.5.2.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.2.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.2.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.2.1.4.1-2.

Table 8.5.2.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.2.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.2.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1.

8.5.2.2.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.2.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in clause 8.5.2.2.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.2.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.2.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.5.2.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.2.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-rsrq | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.2.1.5	Test requirement

Table 8.5.2.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.2.1.5-2.

Table 8.5.2.2.1.5-1: SS-RSRQ inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -81.68 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3,6 |  |  | -87.80 |  | -113 |  | Same as Noc for 15kHz for Config 1,2,4,5 |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -81.68 |  | -106 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -84.80 |  | -110 |  | -113 + ΔBG_offset |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -83.43 |  | -107.75 |  | -117.75 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -86.55 |  | -111.75 |  | -114.75 + ΔBG_offset |  |
| SS-RSRQ Note3 |  |  | dB | -14.77 |  | -14.76 |  | -14.76 |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -50 |  | -75.83 |  | -85.83 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -50 |  | -76.73 |  | -79.73 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

Table 8.5.2.2.1.5-2: SS-RSRQ accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_51 | SS-RSRQ_51 | SS-RSRQ_51 |
| Highest reported value (Cell 2) | SS-RSRQ_63 | SS-RSRQ_63 | SS-RSRQ_63 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_48 | SS-RSRQ_48 | SS-RSRQ_48 |
| Highest reported value (Cell 2) | SS-RSRQ_66 | SS-RSRQ_66 | SS-RSRQ_66 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell

Editor's Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

8.5.2.2.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRQ measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.2.2.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements.

8.5.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.2.2.

8.5.2.2.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.2.2.4.1-1.

Table 8.5.2.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.2.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.2.2.4.1-2.

Table 8.5.2.2.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in clause C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.2.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.2.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.2.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.2.2.5-1 and Table 8.5.2.2.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in clause 8.5.2.2.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.2.2.5-1 and 8.5.2.2.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.2.2.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.5.2.2.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.2.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| rsrq | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.2.2.5	Test requirement

Table 8.5.2.2.2.5-1 defines the general test parameters and Table 8.5.2.2.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.2.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.2.2.5-3

Table 8.5.2.2.2.5-1: SS-RSRQ Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

Table 8.5.2.2.2.5-2: SS-RSRQ Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 | Setup 1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak -5dB) (Note 7) |
| Note1 | dBm/SCS Note4 | -95.97 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +4dB) (Note 7) |
|  | dB | -0.5 | 3.55 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.47 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +7.55dB) (Note 8) |
| SS-RSRQNote2 | dB | -14.06 | -12.38 |
| Note2 | dB | -0.5 | 3.55 |
| IoNote2 | dBm/95.04 MHz Note4 | -64.21 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +43.16dB) (Note 9) |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SSB_RP, SS-RSRQ, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Void NOTE 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. NOTE 5: Void NOTE 6: Void NOTE 7: Noc for SCS 15kHz is applied at -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Noc for SCS 120kHz is applied at 4dB above the minimum level specified in Table B.2.3-2 for beam peak. NOTE 8: SSB_RP is applied at 2.25dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. NOTE 9: Io is applied at 10log10(792)+6.22dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. NOTE 10: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation. |  |  |  |

Table 8.5.2.2.2.5-3: evaluation limits for the reported values

| Normal Conditions | Test 1 | Test 2 |
| --- | --- | --- |
| Lowest reported value (Cell 2) | 23 | 26 |
| Highest reported value (Cell 2) | 95 | 98 |
| Extreme Conditions | Test 1 | Test 2 |
| Lowest reported value (Cell 2) | 20 | 23 |
| Highest reported value (Cell 2) | 98 | 101 |

#### 8.5.2.3	SS-SINR

##### 8.5.2.3.0	Minimum conformance requirements

The measurement period of NR SS-SINR measurements is the same as the measurement period of NR SS-RSRP measurements defined in clause 8.5.2.1.0.

The reporting range of SS-SINR and CSI-SINR is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 4.7.3.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

The normative reference for this requirement is TS 36.133 [23] clause 9.11.3.

8.5.2.3.0.1	Inter-RAT E-UTRA - NR FR1 SS-SINR minimum conformance requirements

The accuracy requirements of NR SS-SINR measurements in FR1 and the corresponding side conditions shall be the same as the inter-frequency SS-SINR absolute accuracy requirements in clause 4.7.3.0.2.

8.5.2.3.0.2	Inter-RAT E-UTRA - NR FR2 SS-SINR minimum conformance requirements

The accuracy requirements of NR SS-SINR measurements in FR2 and the corresponding side conditions shall be the same as the inter-frequency SS-SINR absolute accuracy requirements in clause 5.7.3.0.2.

##### 8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell

8.5.2.3.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-SINR measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.3.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards supporting ss-SINR-Meas and capable of NR measurements.

8.5.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.3.1.

8.5.2.3.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.3.1.4.1-1.

Table 8.5.2.3.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.3.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.3.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.3.1.4.1-2.

Table 8.5.2.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.3.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.3.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1.

8.5.2.3.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.3.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the expected SS- SINR. If the value is outside the limits in clause 8.5.2.3.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.3.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.3.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.5.2.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.3.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-sinr | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.3.1.5	Test requirement

Table 8.5.2.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.3.1.5-2.

Table 8.5.2.3.1.5-1: SS-SINR inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -88 |  | -108.5 |  | -119.5 + ΔBG_offset |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -88 |  | -108.5 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -85 |  | -105.5 |  | -116.5 + ΔBG_offset |  |
|  |  |  | dB | -1.75 |  | 20 |  | -3.2 |  |
|  |  |  | dB | -1.75 |  | 20 |  | -3.2 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -89.75 |  | -88.5 |  | -122.7 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -86.75 |  | -85.5 |  | -119.7 + ΔBG_offset |  |
| SS-SINR Note3 |  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -57.83 |  | -60.5 |  | -89.85 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -51.73 |  | -54.41 |  | -83.75 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

Table 8.5.2.3.1.5-2: SS-SINR accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_35 | SS-SINR_79 | SS-SINR_32 |
| Highest reported value (Cell 2) | SS-SINR_51 | SS-SINR_94 | SS-SINR_49 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_33 | SS-SINR_77 | SS-SINR_31 |
| Highest reported value (Cell 2) | SS-SINR_53 | SS-SINR_96 | SS-SINR_50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell

Editor's Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

8.5.2.3.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-SINR measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.3.2.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards supporting ss-SINR-Meas and capable of NR FR2 measurements.

8.5.2.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.3.2.

8.5.2.3.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.3.2.4.1-1.

Table 8.5.2.3.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.3.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.3.2.4.1-2.

Table 8.5.2.3.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in clause C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.3.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.3.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] clause C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.3.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.3.2.5-1 and Table 8.5.2.3.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the expected SS- SINR. If the value is outside the limits in clause 8.5.2.3.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.3.2.5-1 and 8.5.2.3.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.3.2.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.5.2.3.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.3.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| sinr | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.3.2.5	Test requirement

Table 8.5.2.3.2.5-1 defines the general test parameters and Table 8.5.2.3.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.3.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.3.2.5-3.

Table 8.5.2.3.2.5-1: SS-SINR Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 | freq1 |
| Duplex mode |  | TDD | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |
| PDSCH Reference measurement channel |  | - | - | - |
| RMSI CORESET Reference Channel |  | - | - | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table 8.5.2.3.2.5-2: SS-SINR Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 | Setup 1 | Setup 1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105 | -105.1 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak -5dB) (Note 7) |
| Note1 | dBm/SCS Note4 | -95.97 | -96.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +4dB) (Note 7) |
|  | dB | -0.5 | 11 | 3.7 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.47 | -85.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +7.6dB) (Note 8) |
| SS-SINRNote2 | dB | -0.5 | 11 | 3.6 |
| Note2 | dB | -0.5 | 11 | 3.6 |
| IoNote2 | dBm/95.04 MHz Note4 | -64.21 | -55.75 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +43.19dB) (Note 9) |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SSB_RP, SS-SINR, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Void NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. NOTE 5: Void NOTE 6: Void NOTE 7: Noc for SCS 15kHz is applied at -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Noc for SCS 120kHz is applied at 4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 8: SSB_RP is applied at 3dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 9: Io is applied at level 10log10(792)+6.54dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 10: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation. |  |  |  |  |

Table 8.5.2.3.2.5-3: evaluation limits for the reported values

| Normal Conditions | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 9 | 32 | 17 |
| Highest reported value (Cell 2) | 82 | 105 | 90 |
| Extreme Conditions | Test 1 | Test 2 | Test 3 |
| Lowest reported value (Cell 2) | 7 | 30 | 15 |
| Highest reported value (Cell 2) | 84 | 107 | 92 |

# 9	NR sidelink

## 9.1	NR sidelink in FR1

### 9.1.1	UE transmit timing

#### 9.1.1.0	Minimum conformance requirements

##### 9.1.1.0.1	Minimum conformance requirements for GNSS as synchronization reference source

The requirements in this subclause are applicable when the reference timing used by the UE for NR sidelink communication is derived from GNSS.

The sidelink transmissions takes place NTA,SL+NTA offset×Tc before the subframe starting boundary as defined in TS 38.331 [13], where NTA offset=0 and NTA,SL=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 9.1.1.0.1-1.

Table 9.1.1.0.1-1: Te Timing Error Limit

| Frequency Range of sidelink | Te_ |
| --- | --- |
| FR1 | 12*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7]. |  |

The normative reference for this requirement is TS 38.133 [6] clause 12.2.2.

##### 9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for deriving sidelink transmission is from SyncRef UE transmitting sidelink synchronization signals.

The sidelink transmissions takes place NTA,SL+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding timing reference frame from the SyncRef UE, where NTA offset=0 and NTA,SL=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 9.1.1.0.2-1.

Table 9.1.1.0.2-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of sidelink signals (kHz) | Te |
| --- | --- | --- |
| FR1 | 15 | 12*64*Tc |
|  | 30 | 8*64*Tc |
|  | 60 | 5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7]. |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 12.2.5.

##### 9.1.1.0.3	Minimum conformance requirements for FR1 NR Cell as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is a NR serving cell on a non-NR sidelink carrier.

The sidelink transmissions takes place NTA,SL+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where NTA offset=0 and NTA,SL=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 9.1.1.0.3-1.

Table 9.1.1.0.3-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of SSB signals ( kHz) | SCS of sidelink signals (kHz) | Te |
| --- | --- | --- | --- |
| FR1 | 15 | 15 | 14*64*Tc |
|  |  | 30 | 12*64*Tc |
|  |  | 60 | 12*64*Tc |
|  | 30 | 15 | 10*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 9*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7]. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 12.2.3.

#### 9.1.1.1	NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source

9.1.1.1.1	Test purpose

The purpose of this test is to verify the UE timing requirements as specified in TS 38.133 [6] clause 12.2.2, when the GNSS is used as timing reference.

9.1.1.1.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting NR sidelink communication.

9.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.1.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.1.1.

9.1.1.1.4	Test description

9.1.1.1.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.1.1.4.1-1.

Table 9.1.1.1.4.1-1: Initial conditions for NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. 1 |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.1.1.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.1.1.4.3

3.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [14] Table 4.11.2-2.

Table 9.1.1.1.4.1-2: NR sidelink test parameters for NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| Active cell |  | None |  |
| Active SyncRef UE |  | None |  |
| NR sidelink communication pre-configuration |  | As specified in section A.11.2 | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  | CC.1A HD | As specified in Table A.11.3-1 |
| PSSCH Reference Measurement Channel |  | CD.1A HD | As specified in Table A.11.3-2 |
| Propagation condition |  | AWGN |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

9.1.1.1.4.2	Test procedure

For this test, the UE is triggered by the test loop function to transmit for NR sidelink communication. There is one GNSS based synchronization source during the test. The test system can emulate and send the GNSS signal to the test UE.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On and GNSS Sync = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1.

3.	Wait at least 20 seconds for the UE to acquire the GNSS signal.

4.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit.

5.	The UE starts to transmit NR sidelink communication data over the PC5 interface in accordance of pre-configuration and to schedule PSSCH.

6.	After the UE is synchronized to the GNSS synchronization source, the SS shall monitor all PSSCH transmissions and verify that, for each received PSSCH, the timing is within NTA,SL+NTA offset×Tc±Te before the subframe starting boundary determined by the GNSS reference time in the GNSS simulator, where NTA offset=0 and NTA,SL=0, and Te is given in Table 9.1.1.1.5-1.

7.	The SS sends AT command +CCUTLE to open test loop function.

9.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.1.1.4.3-1: +CCUTLE (Test procedure, step 4)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.1.1.4.3-2: +CCUTLE (Test procedure, step 7)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.1.1.5	Test requirement

The sidelink transmissions takes place NTA,SL+NTA offset×Tc before the subframe starting boundary as defined in TS 38.331 [13] clause 5.8.12, where NTA offset=0 and NTA,SL=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 9.1.1.1.5-1.

Table 9.1.1.1.5-1: Te Timing Error Limit

| Frequency Range of sidelink | Te Note 2 |
| --- | --- |
| FR1 | 15*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7]. Note 2: Including the test tolerance given in annex F. |  |

#### 9.1.1.2	NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source

9.1.1.2.1	Test purpose

The purpose of this test is to verify the UE timing requirements as specified in TS 38.133 [6] clause 12.2.5, when SyncRef UE is used as timing reference.

9.1.1.2.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting NR sidelink communication.

9.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.1.0.2

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.1.2.

9.1.1.2.4	Test description

9.1.1.2.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.1.2.4.1-1.

Table 9.1.1.2.4.1-1: Initial conditions for NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. 1 |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.1.2.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1 |  |
| Exceptions to connection diagram | GNSS System Simulator is not used. |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.1.2.4.3

3.	There is one NR-SS-UE (SyncRef UE 1) specified in this test. SyncRef UE 1 is configured according to the parameters in Table 9.1.1.2.4.1-1.

Table 9.1.1.2.4.1-2: NR sidelink test parameters for NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 3 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Active cell |  |  | None |  |
| Active SyncRef UE |  |  | SyncRef UE 1 | Transmitting S-SSB on RF channel number 1 |
| NR sidelink communication preconfiguration |  |  | As specified in section A.11.2 | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  |  | CC.1A HD | As specified in Table A.11.3-1 |
| PSSCH Reference Measurement Channel |  |  | CD.1A HD | As specified in Table A.11.3-2 |
| Noc Note1,2 |  | dBm/30kHz | -95 |  |
| SyncRef UE 1 | sl-SSB-TimeAllocation |  | sl-SSB-TimeAllocation1 |  |
|  | slssid |  | 30 |  |
|  | inCoverage |  | TRUE | In MIB-SL |
|  | networkControlledSyncTx |  | ON |  |
|  | NR sidelink communication configuration |  | As specified in section A.11.2 | IE values unless specified otherwise in this test. |
|  | Ês/Noc |  | 3 |  |
|  | PSBCH-RSRP Note1, Note 2 | dBm/30kHz | -92 |  |
| Propagation condition |  |  | AWGN |  |
| Note 1: PSBCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: S-PSS Ês/Noc and S-PSS Ês/Noc are set the same as PSBCH Ês/Noc. Note 3: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

9.1.1.2.4.2	Test procedure

In this test there is one active NR-SS-UE (SyncRef UE 1) and no serving cell nor GNSS signals. The UE is triggered by the test loop function to transmit for NR sidelink communication.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit.

3.	The UE starts to transmit NR sidelink communication data over the PC5 interface in accordance of pre-configuration and to schedule PSSCH.

4.	Power on SyncRef UE 1 and wait at least 8 seconds for the UE to be synchronized to SyncRef UE 1.

5.	After the UE is synchronized to SyncRef UE 1, the SS shall monitor all PSSCH transmissions and verify that, for each received PSSCH, the timing is within NTA,SL+NTA offset×Tc±Te before the subframe starting boundary determined by the reception of the first detected path (in time) of corresponding timing reference frame from SyncRef UE 1, where NTA offset=0 and NTA,SL=0, and Te is given in Table 9.1.1.2.5-1.

6.	The SS sends AT command +CCUTLE to open test loop function.

9.1.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.1.2.4.3-1: +CCUTLE (Test procedure, step 2)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.1.2.4.3-2: MasterInformationBlockSidelink (SyncRef UE 1)

Derivation Path: TS 38.508-1 [14], Table 4.6.1A-1 with condition NB_SYNC and RX

Table 9.1.1.2.4.3-3: SL-ConfigDedicatedNR (SyncRef UE 1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-7 with condition SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { | 1 entry |  |  |
| SL-FreqConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] | SL-SyncConfig | entry 1 Table 9.1.1.2.4.3-4 |  |
| } |  |  |  |
| sl-SyncPriority-r16 | gnbEnb |  |  |
| } |  |  |  |
| } |  |  |  |
| networkControlledSyncTx-r16 | on |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.1.2.4.3-4: SL-SyncConfig (Table 9.1.1.2.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-SyncConfig-r16 ::= SEQUENCE { |  |  |  |
| sl-SSID-r16 | 30 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.1.2.4.3-5: +CCUTLE (Test procedure, step 6)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.1.2.5	Test requirement

The sidelink transmissions takes place NTA,SL+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding timing reference frame from SyncRef UE 1, where NTA offset=0 and NTA,SL=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 9.1.1.2.5-1.

Table 9.1.1.2.5-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of sidelink signals (kHz) | Te Note 2 |
| --- | --- | --- |
| FR1 | 30 | 10*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7]. Note 2: Including the test tolerance given in annex F. |  |  |

#### 9.1.1.3	NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source

9.1.1.3.1	Test purpose

The purpose of this test is to verify the UE timing requirements as specified in TS 38.133 [6] clause 12.2.5, when the downlink timing of the serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) on a non-sidelink NR carrier is used as timing reference.

9.1.1.3.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting NR Uu and sidelink communication.

9.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.1.0.3

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.1.3.

9.1.1.3.4	Test description

9.1.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 9.1.1.3.4.1-1.

Table 9.1.1.3.4.1-1: Supported test configurations for FR1 NR Cell

| Configuration | Description |
| --- | --- |
| 9.1.1.3-1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 9.1.1.3-2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 9.1.1.3-3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 9.1.1.3.4.1-2.

Table 9.1.1.3.4.1-2: Initial conditions for NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | Uu & PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8.2.1 |  |  |
| Channel bandwidth | Uu: As specified by the test configuration selected from Table 9.1.1.3.4.1-1 PC5: As specified by the test configuration selected from Table 9.1.1.3.4.1-3. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.3 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are defined in clause 9.1.1.3.4.3

3.	There is one NR Uu carrier and one NR Cell (Cell 1) specified in this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

4.	Test parameters for Cell 1 are given in Table 9.1.1.3.4.1-3.

Table 9.1.1.3.4.1-3: NR sidelink test parameters for NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| Active cell |  | Cell 1 |  |
| Active SyncRef UE |  | None |  |
| V2X sidelink communication configuration |  | As specified in section A.11.2 | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  | CC.1A HD | As specified in Table A.11.3-1 |
| PSSCH Reference Measurement Channel |  | CD.1A HD | As specified in Table A.11.3-2 |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

Table 9.1.1.3.4.1-3: Cell Test Parameters for NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| RF Channel Number |  |  | 2 |
| Duplex Mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 40: NRB,c = 106 |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| DRX Cycle |  |  | N/A |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.2 |
| OCNG Patterns |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Noc Note2 | Config 1,2,3 | dBm/15 kHz | -98 |
| Noc Note2 | Config 1,2 | dBm/SCS | -98 |
|  | Config 3 |  | -95 |
| Ês/Noc |  | dB | 3 |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -95 |
|  | Config 3 |  | -92 |
| Io Note 3 | Config 1,2 | dBm/9.36 MHz | -65.2 |
|  | Config 3 | dBm/38.1 MHz | -59.2 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

9.1.1.3.4.2	Test procedure

In this test there is one active cell (Cell 1). For this test, the UE is triggered by the test loop function or the upper layers to transmit for NR sidelink communication.

1.	Ensure the UE is in state 1N-B with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit.

3.	The UE starts to transmit NR sidelink communication data over the PC5 interface in according to Pre-configuration and to schedule PSSCH.

4.	The SS shall monitor all PSSCH transmissions and verify that, for each received PSSCH, the timing is within NTA,SL+NTA offset×Tc±Te before the subframe starting boundary determined by the reception of the first detected path (in time) of the corresponding downlink frame from the Cell 1, where NTA offset=0 and NTA,SL=0, and Te is given in Table 9.1.1.3.5-1.

5.	The SS sends AT command +CCUTLE to open test loop function.

6.	Ensure the UE is in state 3N-B with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

7.	The SS sends an RRCReconfiguration message with sl-ConfigDedicatedNR to provide sidelink configuration.

8.	The UE sends an RRCReconfigurationComplete message.

9.	The SS sends a CLOSE UE TEST LOOP message to close test loop function and trigger UE to transmit.

10.	The UE sends a CLOSE UE TEST LOOP COMPLETE message.

11.	The SS shall monitor all PSSCH transmissions and verify that, for each received PSSCH, the timing is within NTA,SL+NTA offset×Tc±Te before the subframe starting boundary determined by the reception of the first detected path (in time) of the corresponding downlink frame from the Cell 1, where NTA offset=0 and NTA,SL=0, and Te is given in Table 9.1.1.3.5-1.

12.	The SS sends an OPEN UE TEST LOOP message to open test loop function.

13.	The UE sends an OPEN UE TEST LOOP COMPLETE message.

9.1.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions: Table 9.1.1.3.4.3-1: Common Exception messages for NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO and Synchronous cells, where smtc1 field is set to SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3: with condition SMTC.2. |

Table 9.1.1.3.4.3-2: SIB12-IEs-r16 (Preamble and test procedure, Cell 1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncPriority-r16 | gnbEnb |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.1.3.4.3-3: +CCUTLE (Test procedure, step 2)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.1.3.4.3-4: +CCUTLE (Test procedure, step 5)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

Table 9.1.1.3.4.3-5: RRCReconfiguration (Test procedure, step 7)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and SIDELINK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  |
| setup | SL-ConfigDedicatedNR | Table 9.1.1.3.4.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.1.3.4.3-6: SL-ConfigDedicatedNR (Table 9.1.1.3.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-7 with condition SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { | 1 entry |  |  |
| SL-FreqConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncPriority-r16 | gnbEnb |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.1.3.4.3-7: CLOSE UE TEST LOOP (Test procedure, step 9)

| Derivation Path: 36.508 [25] Table 4.7A-3 with condition UE TEST LOOP MODE E(V2X Transmission) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode E LB setup |  |  |  |
| Communication Transmit or Receive | 0 0 0 0 0 0 0 1 | '01' indicates V2X UE triggered to transmit NR sidelink communication with single spatial layer. |  |

9.1.1.3.5	Test requirement

The sidelink transmissions takes place NTA,SL+NTA offset×Tc before the subframe starting boundary determined by the reception of the first detected path (in time) of the corresponding downlink frame from the Cell 1, where NTA offset=0 and NTA,SL=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 9.1.1.3.5-1.

Table 9.1.1.3.5-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of SSB signals ( kHz) | SCS of sidelink signals (kHz) | TeNote2 |
| --- | --- | --- | --- |
| FR1 | 15 | 30 | 15*64*Tc |
|  | 30 | 30 | 13*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7]. Note 2: Including the test tolerance given in Annex F. |  |  |  |

### 9.1.2	Initiation/Cease of S-SSB transmission

#### 9.1.2.0	Minimum conformance requirements

##### 9.1.2.0.1	Minimum conformance requirements for FR1 NR cell as synchronization reference source

The requirements apply when the NR Cell is used as synchronization reference source and when the UE is

-	out of coverage on the NR sidelink carrier and in-coverage with a serving cell on a NR non-sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331[13] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SIB12. The UE shall be capable of measuring the RSRP of the cell used as synchronization reference source to evaluate to initiate/cease SLSS transmissions within Tevaluate,SLSS

where,

-	Tevaluate,SLSS is as specified in Table 9.1.2.0.1-1 when UE performs SSB based measurements without measurement gaps.

-	Tevaluate,SLSS is as specified in Table 9.1.2.0.1-2 when UE performs SSB based measurements with measurement gaps.

Table 9.1.2.0.1-1: Tevaluate,SLSS for measurements without measurement gaps when NR cell is used as synchronization reference source (FR1)

| DRX cycle in NR cell | Tevaluate,SLSS |
| --- | --- |
| No DRX | max(400ms, ceil( 2 x 5 x Kp) x SMTC period)Note 1 |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x 2 x 5 x Kp) x max(SMTC period, DRX cycle)) |
| DRX cycle>320ms | ceil( 7 x Kp ) x DRX cycle |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.1.2.0.1-2: Tevaluate,SLSS for measurements with measurement gaps when NR cell is used as synchronization reference source (FR1)

| DRX cycle in NR cell | Tevaluate,SLSS |
| --- | --- |
| No DRX | max(400ms, 2 x 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(2 x 1.5x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra |

If higher layer filtering is configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the NR cell as synchronization reference source:

-	SS-RSRP related side conditions given in 38.133[6] clauses 10.1.2 for FR1, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in 38.133[6] clauses 10.1.7 for FR1, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in 38.133[6] clauses 10.1.12 for FR1, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

The normative reference for this requirement is TS 38.133 [6] clause 12.3.1.1.

##### 9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source

The requirements apply when SyncRef UE is used as synchronization reference source and when the UE is

-	in any cell selection state, or

-	out of coverage on the NR sidelink carrier and is associated with a serving cell on a non-sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331[13] are met and when SyncRef UE is used as synchronization reference source and if syncTxThreshOoC is included in the pre-configuration parameters.

The UE shall be capable of measuring the PSBCH-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease SLSS transmissions within Tevaluate,SLSS = 4 S-SSB periods.

If higher layer filtering for PSBCH-RSRP measurements is pre-configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the selected SyncRef UE as defined in TS 38.331 [2] used to derive transmission timing for NR sidelink communication:

-	PSBCH-RSRP related side conditions given in 38.133[6] clause 10.4 for a corresponding Band are fulfilled,

-	NR S-SSB_RP and S-SSB Ês/Iot according to Annex B.4 for a corresponding Band are fulfilled.

The requirements for absolute accuracy of PSBCH-RSRP apply to a NR sidelink synchronization source on the same frequency as that of the own NR sidelink UE performing the measurement in FR1.

The accuracy requirements in Table 9.1.1.0.2-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 9.1.1.0.2-1: Intra-frequency PSBCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR sidelink operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 4.5 | 9 | -6 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | -6 | NR_TDD_FR1_B, NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |  |

The relative accuracy of PSBCH-RSRP is defined as the PSBCH-RSRP measured from one NR sidelink synchronization source compared to the PSBCH-RSRP measured from another NR sidelink synchronization source on the same frequency in FR1.

The accuracy requirements in Table 9.1.1.0.2-2 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP accuracy measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 9.1.1.0.2-2: Intra-frequency PSBCH-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR sidelink operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 2 | 3 | -3 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -50 |
| 3 | 3 | -6 | Note 4 | Note 4 | Note 4 | Note 4 | N/A | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3A.4.1. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |  |

The normative references for this requirement are TS 38.133 [6] clause 10.4.2 and 12.3.1.4.

#### 9.1.2.1	NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source

9.1.2.1.1	Test purpose

The purpose of this test is to verify that the NR sidelink UE meets the requirements related to the maximum evaluation time allowed to initiate and cease S-SSB transmissions defined in TS 38.133 [6] clause 12.3.1.1, when the reference timing used for sidelink transmissions is a NR serving cell in FR1 on a non-sidelink carrier.

9.1.2.1.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting NR Uu and sidelink communication.

9.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.2.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.2.1.

9.1.2.1.4	Test description

9.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 9.1.2.1.4.1-1.

Table 9.1.2.1.4.1-1: Supported test configurations for FR1 NR Cell

| Configuration | Description |
| --- | --- |
| 9.1.2.1-1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 9.1.2.1-2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 9.1.2.1-3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 9.1.2.1.4.1-2.

Table 9.1.2.1.4.1-2: Initial conditions for Initiation/Cease of S-SSB Transmission Test for FR1 NR cell as synchronization reference source

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | Uu & PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8.2.1 |  |  |
| Channel bandwidth | Uu: As specified by the test configuration selected from Table 9.1.1.3.4.1-1 PC5: As specified by the test configuration selected from Table 9.1.1.3.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.3 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are configured according to Table 9.1.2.1.4.1-3.

2.	Message content exceptions are given in clause 9.1.2.1.4.3

3.	There is one NR Uu carrier and one NR Cell (Cell 1) specified in this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

4.	General test parameters for Cell 1 are given in Table 9.1.2.1.4.1-3.

Table 9.1.2.1.4.1-3: NR sidelink test parameters and NR Cell general test parameters for Initiation/Cease of S-SSB Transmission Test for FR1 NR cell as synchronization reference source

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCS | kHz | 30 |  |
| Active cell |  | Cell 1 | Serving cell on RF channel number 1 |
| Active SyncRef UE |  | None |  |
| Active NR sidelink UE |  | NR sidelink UE | Transmitting S-SSB on RF channel number 2(HD carrier in Band n47 or n38) |
| NR sidelink communication configuration |  | As specified in A.11.2 | IE values unless specified otherwise in this test |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshIC | dBm/SCS | -110 | In SIB12 |
| DRX |  | OFF |  |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

9.1.2.1.4.2	Test procedure

In this test there is one active cell (Cell 1). The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

1.	Ensure the UE is in state 1N-B with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The SS shall set parameters according to T1 in Table 9.1.2.1.5-1. T1 starts.

3.	If the SS observes that no SL SSB is transmitted by the UE until T1 expires then count a success for the event "No S-SSB transmission", otherwise count a fail for the event "No S-SSB transmission".

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.2.1.5-1. T2 starts.

5.	If the UE starts to send S-SSB within 560 ms from the start of T2 then count a success for the event "S-SSB transmission initiation", otherwise count a fail for the event "S-SSB transmission initiation".

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 in Table 9.1.2.1.5-1. T3 starts.

7.	If the UE stops to send S-SSB within 560 ms from the start of T3 then count a success for the event "S-SSB transmission cease", otherwise count a fail for the event "S-SSB transmission cease".

8.	Repeat step 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

Each of the events "No S-SSB transmission", "S-SSB transmission initiation" and "S-SSB transmission cease" is evaluated independently for the statistic, resulting in an event verdict pass or fail. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

9.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.2.1.4.3-1: Common Exception messages for NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-1 with condition SMTC.2 and Asynchronous cells for test configuration 9.1.2.1-1 Table H.2.1-1 with condition SMTC.1 and Synchronous cells for test configuration 9.1.2.1-2 and 9.1.2.1-3 |

Table 9.1.2.1.4.3-2: SIB12-IEs-r16 (Preamble and test procedure, Cell 1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncPriority-r16 | gnbEnb |  |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] | SL-SyncConfig | entry 1 Table 9.1.2.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.2.1.4.3-3: SL-SyncConfig (Table 9.1.2.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-SyncConfig-r16 ::= SEQUENCE { |  |  |  |
| sl-filterCoefficient-r16 | fc0 | L3 filtering is not used to avoid introducing additional delay |  |
| txParameters-r16 SEQUENCE { |  |  |  |
| syncTxThreshIC-r16 | 2 | actual threshold is -120+2*5 = -110 dBm |  |
| syncTxThreshOoC-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

9.1.2.1.5	Test requirement

Cell specific test parameters for NR Cell 1 is given in Table 9.1.2.1.5-1.

Table 9.1.2.1.5-1: FR1 NR Cell-specific test parameters for Initiation/Cease of S-SSB Transmission Test for FR1 NR cell as synchronization reference source

| Parameter |  | Unit | Cell1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| NR RF Channel Number |  |  | 1 |  |  |
| Duplex Mode | Config 1 |  | FDD |  |  |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10:NRB,c = 52 |  |  |
|  | Config 3 |  | 40:NRB,c = 106 |  |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| DRX Cycle |  | ms | N/A |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| SMTC Configuration | Config 1 |  | SMTC.2 |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote2 |  | Config 1,2 | dBm/15 kHz |  |  |
|  |  | Config 3 |  |  |  |
|  |  | Config 1,2,3 | dBm/SCS |  |  |
| Ês/Noc |  | dB | 7.6 Note 5 | -4.5 | 7.6 Note 5 |
| Ês/Iot |  | dB | 7.6 | -4.5 | 7.6 |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -103.95 | 116.05 | -103.95 |
|  | Config 3 |  | -103.95 | -116.05 | -103.95 |
| IoNote3 | Config 1,2 | dBm/9.36MHz | -75.30 | -82.28 | -75.30 |
|  | Config 3 | dBm/38.16MHz | -72.20 | -79.18 | -72.20 |
| Propagation condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Including the test tolerance given in Annex F. |  |  |  |  |  |

During T1, the SS-RSRP of the PCell is above syncTxThreshIC and the UE is not expected to be transmitting S-SSB.

During T2, the SS-RSRP of the PCell is lowered below syncTxThreshIC and the UE is expected to initiate S-SSB transmissions.

During T3, the SS-RSRP of the PCell is increased back to be above syncTxThreshIC and the UE is expected to cease S-SSB transmissions.

The S-SSB transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the S-SSB transmission.

The S-SSB transmission initiation delay shall be less than Tevaluate,SLSS + S-SSB period = 560ms, where,

Tevaluate,SLSS = max(400 ms, ceil (10 ∙ Kp) ∙ SMTC period) = 400 ms, is the evaluation time specified in 38.133 [6] Table 12.3.1.1-1, where

SMTC period = 20ms according to Table 9.1.2.1.5-1.

Kp = 1 according to Table 9.1.2.1.5-1.

S-SSB period = 160 ms.

The S-SSB transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the S-SSB transmission.

The S-SSB transmission cease delay shall be less than Tevaluate,SLSS + S-SSB period = 560ms.

The rate of correct initiation/cease delay of S-SSB transmissions observed during repeated tests shall be at least 90%.

#### 9.1.2.2	NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source

9.1.2.2.1	Test purpose

The purpose of this test is to verify that the NR sidelink UE meets the requirements related to the maximum evaluation time allowed to initiate and cease S-SSB transmissions defined in TS 38.133 [6] clause 12.3.1.4, when the reference timing used for sidelink transmissions is a SyncRef UE.

9.1.2.2.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting NR sidelink communication.

9.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.2.0.2

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.2.2.

9.1.2.2.4	Test description

9.1.2.2.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.2.2.4.1-1.

Table 9.1.2.2.4.1-1: Initial conditions for NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.1.3.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1 |  |
| Exceptions to connection diagram | GNSS System Simulator is not used. |  |  |

1.	The parameter settings for the NR sidelink communication are configured according to Table 9.1.2.2.4.1-2.

2.	Message content exceptions are given in clause 9.1.2.2.4.3

3.	There is one NR-SS-UE (SyncRef UE 1) specified in this test. SyncRef UE 1 is configured according to the parameters in Table 9.1.1.2.4.1-2.

Table 9.1.2.2.4.1-2: NR sidelink test parameters for NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCS | kHz | 30 |  |
| Active cell |  | None |  |
| Active SyncRef UE |  | SyncRef UE 1 | Transmitting S-SSB on RF channel number 1(HD carrier in Band n47 or n38) |
| Active NR sidelink UE |  | NR sidelink UE | Transmitting S-SSB on RF channel number 1(HD carrier in Band n47 or n38) |
| NR sidelink communication preconfiguration |  | As specified in section A.11.2 | IE values unless specified otherwise in this test |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshOoC | dBm/30kHz | -100 |  |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

9.1.2.2.4.2	Test procedure

In this test there is one active NR-SS-UE (SyncRef UE 1) and no serving cell nor GNSS signals. The test system shall emulate SyncRef UE 1 to transmit S-SSB every synchronization period with the SLSSID and inCoverage indicator as specified in Table 9.1.2.2.5-1.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The SS powers on SyncRef UE 1, sets parameters according to T3 in Table 9.1.2.2.5-1. Then the SS waits at least 8 seconds for the UE to be synchronized to SyncRef UE 1.

3.	The SS sets parameters according to T1 in Table 9.1.2.2.5-1, T1 starts.

4.	During T1, if the SS observes that no SL SSB is transmitted by the UE until T1 expires, then count a success for the event "No S-SSB transmission", otherwise count a fail for the event "No S-SSB transmission".

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.2.2.5-1. T2 starts.

6.	If the UE

a)	starts to send S-SSB within 800 ms from the start of T2, and

b)	uses SLSSID = 30, and

c)	inCoverage = FALSE in MasterInformationBlockSidelink

then count a success for the event "S-SSB transmission initiation", otherwise count a fail for the event "S-SSB transmission initiation".

7.	When T2 expires, the SS shall switch the power setting from T2 to T3 in Table 9.1.2.2.5-1. T3 starts.

8.	If the UE transmits no SL SSB from 800 ms after the start of T3 then count a success for the event "S-SSB transmission cease", otherwise count a fail for the event "S-SSB transmission cease".

9.	Repeat step 3-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

Each of the events "No S-SSB transmission", "S-SSB transmission initiation" and "S-SSB transmission cease" is evaluated independently for the statistic, resulting in an event verdict pass or fail. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

9.1.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.2.2.4.3-1: MasterInformationBlockSidelink (SyncRef UE 1)

Derivation Path: TS 38.508-1 [14], Table 4.6.1A-1 with condition NB_SYNC and RX

Table 9.1.2.2.4.3-2: SL-ConfigDedicatedNR (SyncRef UE 1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-7 with condition SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { | 1 entry |  |  |
| SL-FreqConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] | SL-SyncConfig | entry 1 Table 9.1.2.2.4.3-3 |  |
| } |  |  |  |
| sl-SyncPriority-r16 | gnbEnb |  |  |
| } |  |  |  |
| } |  |  |  |
| networkControlledSyncTx-r16 | on |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.2.2.4.3-3: SL-SyncConfig (Table 9.1.2.2.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-SyncConfig-r16 ::= SEQUENCE { |  |  |  |
| sl-SSID-r16 | 30 |  |  |
| } |  |  |  |
| } |  |  |  |

9.1.2.2.5	Test requirement

SyncRef UE Specific test parameters for SyncRef UE 1 is given in Table 9.1.2.2.5-1.

Table 9.1.2.2.5-1: SyncRef UE Specific test parameters for NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source

| Parameter | Unit | SyncRef UE 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| NR RF Channel Number |  | 1 |  |  |
| V2X SL communication resource pool configuration |  | As specified in section A.11.2 |  |  |
| Channel Bandwidth (BWchannel) Note3 | MHz | 20(NRB,c = 50) or 40(NRB,c = 100) |  |  |
| SLSSID |  | 30 |  |  |
| inCoverage |  | TRUE |  |  |
| networkControlledSyncTx |  | ON |  |  |
| NocNote1 | dBm/30 kHz | -99.35Note 5 | -102.55 Note 5 | -99.35 Note 5 |
| Ês/Noc | dB | 5.5 | -3.5 | 5.5 |
| PSBCH Ês/Noc | dB | 5.5 | -3.5 | 5.5 |
| PSBCH-RSRPNote2 | dBm/30 kHz | -93.85 | -106.05 | -93.85 |
| IoNote2 | dBm /18MHz | -71.57 | -79.74 | -71.57 |
|  | dBm /36MHz | -71.57 | -79.74 | -71.57 |
| Propagation condition |  | AWGN |  |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: PSBCH-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io level is based on the allocated RBs for S-PSS/S-SSS/PSBCH symbols. Note 3: The UE is only required to be tested in one of the supported test configurations. Note 4: S-PSS Es/Noc and S-SSS Es/Noc are set the same as PSBCH Es/Noc. Note 5: Including the test tolerance given in Annex F. |  |  |  |  |

During T1, the PSBCH-RSRP of SyncRef UE 1 is above syncTxThreshOOC and the UE is not expected to be transmitting S-SSB.

During T2, the PSBCH-RSRP of SyncRef UE 1 is lowered below syncTxThreshOOC and the UE is expected to initiate S-SSB transmissions.

During T3, the PSBCH-RSRP of SyncRef UE 1 is increased back to be above syncTxThreshOOC and the UE is expected to cease S-SSB transmissions.

The S-SSB transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the S-SSB transmission.

The S-SSB transmission initiation delay shall be less than Tevaluate,SLSS + S-SSB period = 800 ms, where,

Tevaluate,SLSS = 4 ∙ S-SSB period, is the evaluation time specified in 38.133 [6] clause 12.3.1.4,

S-SSB period = 160 ms.

The S-SSB transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the S-SSB transmission.

The S-SSB transmission cease delay shall be less than Tevaluate,SLSS + S-SSB period = 800 ms.

The rate of correct initiation/cease delay of S-SSB transmissions observed during repeated tests shall be at least 90%.

### 9.1.3	Synchronization reference selection/reselection

#### 9.1.3.0	Minimum conformance requirements

##### 9.1.3.0.1	Minimum conformance requirements for GNSS configured as the highest priority synchronization reference source

The requirements defined in this clause do not apply to the UEs that do not support transmission and reception of SLSS.

A SyncRef UE is considered to be detectable when

-	PSBCH-RSRP related side conditions given in 38.133 [6] clause 10.4 are fulfilled for a corresponding Band,

-	S-SSB_RP and S-SSBÊs/Iot according to Annex B.4.3 for a corresponding Band are fulfilled.

When GNSS synchronization reference source is configured as the highest priority and

-	UE is synchronized to GNSS directly,

-	UE shall not drop any NR SLSS and data transmission for the purpose of selection/reselection to the SyncRef UE.

-	UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly,

-	UE shall not drop any NR sidelink data transmission for the purpose of selection/reselection to the SyncRef UE. The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[13]. Tdetect,SyncRef UE_V2X is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of 30% of its SLSS transmissions during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	in other case

-	The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[13]. Tdetect,SyncRef UE_V2X is defined as 8 seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of 6 % of its NR sidelink data and SLSS transmissions during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its NR sidelink data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its NR sidelink data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

UE shall be capable of performing PSBCH-RSRP measurements for 3 identified intra-frequency SyncRef UEs with the measurement period of 320 ms. It is assumed that the SyncRef UE do not drop or delay any SLSS transmission within the measurement period. Otherwise, the measurement period may be extended.

When UE is synchronized to GNSS directly, before selection / reselection of the new synchronization reference source UE shall evaluate the GNSS synchronization source reliability for at least 20 seconds before changing the synchronization reference from GNSS to another synchronization reference source. UE shall be always synchronized to GNSS directly during the evaluation of GNSS synchronization source reliability.

The requirements for absolute accuracy of PSBCH-RSRP apply to a NR sidelink synchronization source on the same frequency as that of the own NR sidelink UE performing the measurement in FR1.

The accuracy requirements in Table 9.1.3.0.1-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 9.1.3.0.1-1: Intra-frequency PSBCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR Sidelink operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 4.5 | 9 | -6 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | -6 | NR_TDD_FR1_B, NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |  |

The relative accuracy of PSBCH-RSRP is defined as the PSBCH-RSRP measured from one NR sidelink synchronization source compared to the PSBCH-RSRP measured from another NR sidelink synchronization source on the same frequency in FR1.

The accuracy requirements in Table 9.1.3.0.1-2 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP accuracy measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 9.1.3.0.1-2: Intra-frequency PSBCH-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR sidelink operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 2 | 3 | -3 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -50 |
| 3 | 3 | -6 | Note 4 | Note 4 | Note 4 | Note 4 | N/A | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3A.4.1. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |  |

The normative references for this requirement are TS 38.133 [6] clause 10.4.2 and 12.4.

##### 9.1.3.0.2	Minimum conformance requirements for eNB/gNB configured as the highest priority synchronization reference source

The requirements defined in this clause do not apply to the UEs that do not support transmission and reception of SLSS.

A SyncRef UE is considered to be detectable when

-	PSBCH-RSRP related side conditions given in 38.133 [6] clause 10.4 are fulfilled for a corresponding Band,

-	S-SSB_RP and S-SSB Ês/Iot according to Annex B.4.3 for a corresponding Band are fulfilled.

When serving cell/PCell synchronization reference source is configured as the highest priority,

-	UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331 [13]. Tdetect,SyncRef UE_V2X is defined as 8  seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the NR sidelink UE is allowed to drop a maximum of 6 % of its NR sidelink data and SLSS transmissions for the purpose of selection / reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its NR sidelink data reception per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its NR sidelink data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

UE shall be capable of performing PSBCH-RSRP measurements for 3 identified intra-frequency SyncRef UEs with the measurement period of 320 ms. It is assumed that the SyncRef UE do not drop or delay any SLSS transmission within the measurement period. Otherwise, the measurement period may be extended.

The requirements for absolute accuracy of PSBCH-RSRP apply to a NR sidelink synchronization source on the same frequency as that of the own NR sidelink UE performing the measurement in FR1.

The accuracy requirements in Table 9.1.3.0.2-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 9.1.3.0.2-1: Intra-frequency PSBCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR Sidelink operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 4.5 | 9 | -6 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | -6 | NR_TDD_FR1_B, NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |  |

The relative accuracy of PSBCH-RSRP is defined as the PSBCH-RSRP measured from one NR sidelink synchronization source compared to the PSBCH-RSRP measured from another NR sidelink synchronization source on the same frequency in FR1.

The accuracy requirements in Table 9.1.3.0.2-2 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP accuracy measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 9.1.3.0.2-2: Intra-frequency PSBCH-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR sidelink operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 2 | 3 | -3 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -50 |
| 3 | 3 | -6 | Note 4 | Note 4 | Note 4 | Note 4 | N/A | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3A.4.1. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |  |

#### 9.1.3.1	NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source

9.1.3.1.1	Test purpose

The purpose of this test is to verify the requirements related to SyncRef UE (re)selection as specified in TS 38.133 [6] clause 12.4, when GNSS is configured as the highest priority.

9.1.3.1.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting NR sidelink communication.

9.1.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.3.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.3.1.

9.1.3.1.4	Test description

9.1.3.1.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.3.1.4.1-1.

Table 9.1.3.1.4.1-1: Initial conditions for NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.3.1.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1 |  |
| Exceptions to connection diagram | GNSS System Simulator is not used. |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.3.1.4.3

3.	There are three NR-SS-UEs (SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3) specified in this test. SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3 are configured according to the generic parameters in Table 9.1.3.1.4.1-2.

Table 9.1.3.1.4.1-2: NR sidelink test parameters for NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active synchronization source |  | Sync Ref UE 1 | DUT transmits for NR Sidelink Communication and S-SSB with SLSS ID = 30 and in-coverage set as FALSE in MIB-SL. |
| T2 end condition | Active synchronization source |  | Sync Ref UE 2 | DUT transmits for NR Sidelink Communication and S-SSB with SLSS ID = 336 and in-coverage set as FALSE in MIB-SL. |
| Final condition | Active synchronization source |  | Sync Ref UE 3 | UE transmits for NR Sidelink Communication and S-SSB with SLSS ID = 0 and in-coverage set as FALSE in MIB-SL. |
| Active SyncRef UEs |  |  | SyncRef UE 1 SyncRef UE 2 SyncRef UE 3 | Transmitting S-SSB on RF channel number 1 (HD carrier in Band n47 or n38) |
| Timing offset among SyncRef UEs |  | s | CP/2 | Synchronous |
| Frequency offset of SyncRef UE 1,2 and 3 |  | ppm | 0 |  |
| NR sidelink Communication configuration |  |  | As specified in clause A.11.2 | IE values unless specified otherwise in this test. |
| sl-SyncPriority |  |  | gnss |  |
| syncTxThreshOoC |  |  | +infinity |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 16 |  |
| T3 |  | s | 3.2 |  |

9.1.3.1.4.2	Test procedure

In this test there are three active SyncRef UEs (SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3) and no GNSS signals nor Cells. The test system shall emulate SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3 to transmit S-SSB every S-SSB period with the SLSSIDs and inCoverage indicators specified in Table 9.1.3.1.5-1.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3 are all switched off before starting the test. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The UE transmits S-SSB with SLSSID belonging to id_oon set and inCoverage = false in SL MIB.

3.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit.

4.	The SS sets parameters according to T1 in Table 9.1.3.1.5-1. T1 starts.

5.	If the UE

a)	starts to transmit S-SSB within 8800 ms from the start of T1 with SLSSID = 30 and inCoverage = false in MasterInformationBlockSidelink, and

b)	drops no more than 6 % of its PSSCH and S-SSB transmissions within 8000 ms from the start of T1.

then count a success for the event "select SyncRef UE 1", otherwise count a fail for the event "select SyncRef UE 1".

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.3.1.5-1. T2 starts.

7.	If the UE

a)	starts to transmit S-SSB within 8800 ms from the start of T2 with SLSSID = 336 and inCoverage = false in MasterInformationBlockSidelink, and

b)	drops no more than 6 % of its PSSCH and S-SSB transmissions within 8000 ms from the start of T2.

then count a success for the event "reselect SyncRef UE 2", otherwise count a fail for the event "reselect SyncRef UE 2".

8.	When T2 expires, the SS shall switch the power setting from T2 to T3 in Table 9.1.3.1.5-1. T3 starts.

9.	If the UE

a)	starts to transmit S-SSB within 2400 ms from the start of T3 with SLSSID = 0 and inCoverage = false in MasterInformationBlockSidelink, and

b)	drops no more than 30 % of its S-SSB transmissions within 1600 ms from the start of T3.

then count a success for the event "reselect SyncRef UE 3", otherwise count a fail for the event" reselect SyncRef UE 3".

10.	When T3 expires, the SS shall switch off all three SyncRef UEs and wait at least 8 seconds.

11.	The SS sends AT command +CCUTLE to open test loop function and trigger UE to stop transmitting.

12.	Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

Each of the events " select SyncRef UE 1", " reselect SyncRef UE 2" and " reselect SyncRef UE 3" is evaluated independently for the statistic, resulting in an event verdict pass or fail. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

9.1.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.3.1.4.3-1: SL-ConfigDedicatedNR (SyncRef UE 1 and SyncRef UE 3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-7 with condition SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { | 1 entry |  |  |
| SL-FreqConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] | SL-SyncConfig | entry 1 Table 9.1.3.1.4.3-3 |  |
| } |  |  |  |
| sl-SyncPriority-r16 | gnbEnb |  | SyncRef UE 1 |
|  | gnss |  | SyncRef UE 3 |
| } |  |  |  |
| } |  |  |  |
| networkControlledSyncTx-r16 | on |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.3.1.4.3-2: SL-PreconfigurationNR (SyncRef UE 2)

| Derivation Path: TS 38.508-1 [14], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-PreconfigFreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] | SL-SyncConfig | entry 1 Table 9.1.3.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.3.1.4.3-3: SL-SyncConfig (Table 9.1.3.1.4.3-1 and Table 9.1.3.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-SyncConfig-r16 ::= SEQUENCE { |  |  |  |
| sl-SSID-r16 | 30 |  | SyncRef UE 1 |
| txParameters-r16 SEQUENCE { |  |  | SyncRef UE 2 |
| syncTxThreshOoC-r16 | 13 | the special value corresponding to +infinity |  |
| } |  |  |  |
| gnss-Sync-r16 | true |  | SyncRef UE 2 SyncRef UE 3 |
| } |  |  |  |

Table 9.1.3.1.4.3-4: SL-PreconfigurationNR (The UE under test)

| Derivation Path: TS 38.508-1 [14], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-PreconfigFreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncRefMinHyst-r16 | dB0 |  |  |
| sl-filterCoefficient-r16 | fc0 |  |  |
| txParameters-r16 SEQUENCE { |  |  |  |
| syncTxThreshOoC-r16 | 13 | the special value corresponding to +infinity |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.3.1.4.3-5: MasterInformationBlockSidelink (SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1A-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlockSidelink ::= SEQUENCE { |  |  |  |
| inCoverage-r16 | true |  | SyncRef UE 1, SyncRef UE 3 |
|  | false |  | SyncRef UE 2 |
| } |  |  |  |

Table 9.1.3.1.4.3-6: MasterInformationBlockSidelink (Test procedure, step 2,5 and 7, the UE under test)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1A-1 with condition TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlockSidelink ::= SEQUENCE { |  |  |  |
| inCoverage-r16 | false |  |  |
| } |  |  |  |

Table 9.1.3.1.4.3-7: +CCUTLE (Test procedure, step 2)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.3.1.4.3-8: +CCUTLE (Test procedure, step 11)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.3.1.5	Test requirement

SyncRef UE Specific test parameters for SyncRef UEs are given in Table 9.1.3.1.5-1.

Table 9.1.3.1.5-1: SyncRef UE Specific Test Parameters for NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source

| Parameter | Unit | SyncRef UE 1 |  |  | SyncRef UE 2 |  |  | SyncRef UE 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1(HD carrier in Band n47 or n38) |  |  |  |  |  |  |  |  |
| Channel Bandwidth (BWchannel) Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |  |  |  |  |  |  |
| V2X Sidelink Communication resource pool configuration |  | As specified in Table A.11.2-2 |  |  |  |  |  |  |  |  |
| networkControlledSyncTx |  | ON |  |  | N/A |  |  | ON |  |  |
| syncTxThreshOoC | dBm/SCS | N/A |  |  | +∞ |  |  | N/A |  |  |
| SLSSID |  | 30 |  |  | 0 |  |  | 0 |  |  |
| inCoverage (in MIB-SL) |  | TRUE |  |  | FALSE |  |  | TRUE |  |  |
| Noc Note1 | dBm/SCS | -95 |  |  |  |  |  |  |  |  |
| Ês/Noc Note2 | dB | 0.35 Note 5 | 0 | 0 | -∞ | 0.35 Note 5 | 0 | -∞ | -∞ | 3.35 Note 5 |
| Ês/Iot | dB | 0.35 | 0 | -5.00 | -∞ | 0.35 | 0 | -∞ | -∞ | 0.34 |
| Io Note2 | dBm/18 MHz | -70.61 | -70.78 | -67.60 | -73.79 | -70.61 | -70.78 | -70.61 | -70.78 | -67.60 |
|  | dBm/36 MHz | -70.61 | -70.78 | -67.60 | -73.79 | -70.61 | -70.78 | -70.61 | -70.78 | -67.60 |
| PSBCH-RSRPNote2, Note 3 | dBm/SCS | -94.65 | -95 | -95 | -∞ | -94.65 | -95 | -∞ | -∞ | -91.65 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: Ês/Iot, Io, PSBCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: S-PSS/S-SSS Es/Iot is set the same as PSBCH Es/Iot. Note 4: The UE is only required to be tested in one of the supported test configurations. Note 5: Including test tolerance given in Annex F. |  |  |  |  |  |  |  |  |  |  |

During T1, the UE shall detect, evaluate and select SyncRef UE 1 as synchronization source. Then it shall change its S-SSB transmission timing with SLSSID = 30 and inCoverage = FALSE in MasterInformationBlockSidelink after SyncRef UE selection delay from start of T1.

During T2, the UE shall detect, evaluate and reselect SyncRef UE 2 as synchronization source. Then it shall change its S-SSB transmission timing with SLSSID = 336 and inCoverage = FALSE in MasterInformationBlockSidelink after SyncRef UE reselection delay from start of T2.

During T3, the UE will detect, evaluate and reselect SyncRef UE 3 as synchronization source. Then it shall change its S-SSB transmission timing with SLSSID = 0 and inCoverage = FALSE in MasterInformationBlockSidelink after SyncRef UE reselection delay from start of T2.

SyncRef UE (re)selection delay is defined as the time from the beginning of corresponding time instant to the time UE is synchronized to SyncRef UE and changes its S-SSB transmissions timing and SLSSID to follow SyncRef UE as the synchronization source. The SyncRef UE (re)selection delay can be expressed as:

SyncRef UE selection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + S-SSB period

-	Tdetect,SyncRef UE = 1.6 seconds, is the time to identify newly detectable intra-frequency SyncRef UE as specified in 38.133 [6] clause 12.4 when UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly, otherwise Tdetect,SyncRef UE = 8 seconds if UE is not directly synchronized to GNSS.

-	Tevaluate,SLSS = 4 S-SSB periods, is the time to measure the PSBCH-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease S-SSB transmissions.

-	S-SSB period = 160 ms.

Which gives SyncRef UE (re)selection delay = 8800 ms for T1 and T2, and 2400 ms for T3.

The test system will also verify that the UE does not drop or delay more than 6% of its sidelink data and S-SSB transmissions during the duration of T1 and T2, and does not drop or delay more than 30% of its S-SSB transmissions during the duration of T3.

The rate of correct SyncRef UE (re)selection observed during repeated tests shall be at least 90%.

#### 9.1.3.2	NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source

9.1.3.2.1	Test purpose

The purpose of this test is to verify the requirements related to SyncRef UE (re)selection as specified in TS 38.133 [6] clause 12.4, when gNB is configured as the highest priority.

9.1.3.2.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting gNB as synchronization source and NR sidelink communication.

9.1.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.3.0.2

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.3.2.

9.1.3.2.4	Test description

9.1.3.2.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.3.2.4.1-1.

Table 9.1.3.2.4.1-1: Initial conditions for NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.3.2.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.3.2.4.3

3.	There are two NR-SS-UEs (SyncRef UE 1 and SyncRef UE 2) specified in this test. SyncRef UE 1 and SyncRef UE 2 are configured according to the generic parameters in Table 9.1.3.2.4.1-2.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [14] Table 4.11.2-2.

Table 9.1.3.2.4.1-2: NR sidelink test parameters for NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| SCS |  | kHz | 30 |  |
| Initial condition | Active synchronization source |  | GNSS | DUT transmits for NR Sidelink Communication and S-SSB with SLSSID = 0 and inCoverage set as TRUE in MIB-SL. |
| T2 end condition | Active synchronization source |  | Sync Ref UE 1 | DUT transmits for NR Sidelink Communication and S-SSB with SLSSID = 395 and inCoverage set as FALSE in MIB-SL. |
| Final condition | Active synchronization source |  | Sync Ref UE 2 | DUT transmits for V2X Sidelink Communication and S-SSB with SLSSID = 30 and inCoverage set as FALSE in MIB-SL. |
| Active cell |  |  | None |  |
| Active SyncRef UEs |  |  | SyncRef UE 1 SyncRef UE 2 | Transmitting S-SSB on RF channel number 1 |
| Timing offset between SyncRef UE 1 and SyncRef UE 2 |  | ms | 3 | Asynchronous |
| Frequency offset of SyncRef UE 1 and 2 |  | ppm | 0 |  |
| NR sidelink Communication preconfiguration |  |  | As specified in Clause A.11.2 | IE values unless specified otherwise in this test. |
| syncPriority |  |  | gnb |  |
| syncTxThreshOoC |  |  | 13 (+infinity) |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 16 |  |
| T3 |  | s | 16 |  |

9.1.3.2.4.2	Test procedure

In this test there are two active SyncRef UEs (SyncRef UE 1 and SyncRef UE 2). There are no active cells and GNSS is reliable during the whole test. The test system can emulate and send the GNSS signal to the test UE. The test system shall emulate SyncRef UE 1 and SyncRef UE 2 to transmit S-SSB every S-SSB period with the SLSSIDs and inCoverage indicators specified in Table 9.1.3.2.5-1.

The test consists of three successive time periods with time duration of T1, T2 and T3 respectively. SyncRef UE 1 and SyncRef UE 2 are all switched off before starting the test. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1.

3.	Wait at least 20 seconds for the UE to acquire the GNSS signal.

4.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit.

5.	The SS sets parameters according to T1 in Table 9.1.3.2.5-1. T1 starts.

6.	The UE transmits S-SSB with SLSSID = 0 and inCoverage = true in MasterInformationBlockSidelink.

7.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.3.2.5-1. T2 starts.

8.	If the UE

a)	starts to transmit S-SSB within 8800 ms from the start of T2 with SLSSID = 395 and inCoverage = false in MasterInformationBlockSidelink, and

b)	drops no more than 6 % of its PSSCH and S-SSB transmissions within 8000 ms from the start of T2

then count a success for the event "reselect SyncRef UE 1", otherwise count a fail for the event "reselect SyncRef UE 1".

9	When T2 expires, the SS shall switch the power setting from T2 to T3 in Table 9.1.3.2.5-1. T3 starts.

10.	If the UE

a)	starts to transmit S-SSB within 8800 ms from the start of T3 with SLSSID = 30 and inCoverage = false in MasterInformationBlockSidelink, and

b)	drops no more than 6 % of its PSSCH and S-SSB transmissions within 8000 ms from the start of T2

then count a success for the event "reselect SyncRef UE 2", otherwise count a fail for the event" reselect SyncRef UE 2".

11.	When T3 expires, the SS shall switch off all SyncRef UEs.

12.	The SS sends AT command +CCUTLE to open test loop function and trigger UE to stop transmitting.

13.	Repeat step 3-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

Each of the events " reselect SyncRef UE 1" and " reselect SyncRef UE 2" is evaluated independently for the statistic, resulting in an event verdict pass or fail. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

9.1.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.3.2.4.3-1: SL-PreconfigurationNR (SyncRef UE 1)

| Derivation Path: TS 38.508-1 [14], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-PreconfigFreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncPriority-r16 | gnbEnb |  |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] | SL-SyncConfig | entry 1 Table 9.1.3.2.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.3.2.4.3-2: SL-ConfigDedicatedNR (SyncRef UE 2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-7 with condition SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { | 1 entry |  |  |
| SL-FreqConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] | SL-SyncConfig | entry 1 Table 9.1.3.2.4.3-3 |  |
| } |  |  |  |
| sl-SyncPriority-r16 | gnbEnb |  |  |
| } |  |  |  |
| } |  |  |  |
| networkControlledSyncTx-r16 | on |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.3.2.4.3-3: SL-SyncConfig (Table 9.1.3.2.4.3-1 and Table 9.1.3.2.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-SyncConfig-r16 ::= SEQUENCE { |  |  |  |
| sl-SSID-r16 | 30 |  | SyncRef UE 2 |
| txParameters-r16 SEQUENCE { |  |  | SyncRef UE 1 |
| syncTxThreshOoC-r16 | 13 | the special value corresponding to +infinity |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.3.2.4.3-4: SL-PreconfigurationNR (The UE under test)

| Derivation Path: TS 38.508-1 [14], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-PreconfigFreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncPriority-r16 | gnbEnb |  |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { | 1 entry |  |  |
| SL-SyncConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-SyncRefMinHyst-r16 | dB0 |  |  |
| sl-filterCoefficient-r16 | fc0 |  |  |
| txParameters-r16 SEQUENCE { |  |  |  |
| syncTxThreshOoC-r16 | 13 | the special value corresponding to +infinity |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.3.2.4.3-5: MasterInformationBlockSidelink (SyncRef UE 1 and SyncRef UE 2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1A-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlockSidelink ::= SEQUENCE { |  |  |  |
| inCoverage-r16 | true |  | SyncRef UE 2, |
|  | false |  | SyncRef UE 1 |
| } |  |  |  |

Table 9.1.3.2.4.3-6: MasterInformationBlockSidelink (Test procedure, step 6,8 and 10, the UE under test)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1A-1 with condition TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlockSidelink ::= SEQUENCE { |  |  |  |
| inCoverage-r16 | true |  | Step 6 |
|  | false |  | Step 8, Step 10 |
| } |  |  |  |

Table 9.1.3.2.4.3-7: +CCUTLE (Test procedure, step 4)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.3.2.4.3-8: +CCUTLE (Test procedure, step 12)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.3.2.5	Test requirement

SyncRef UE Specific test parameters for SyncRef UEs are given in Table 9.1.3.2.5-1.

Table 9.1.3.2.5-1: SyncRef UE Specific Test Parameters for NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source

| Parameter | Unit | SyncRef UE 1 |  |  | SyncRef UE 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1(HD carrier in Band n47 or n38) |  |  |  |  |  |
| Channel Bandwidth (BWchannel) Note 4 | MHz | 20(NRB,c = 50) or 40(NRB,c = 100) |  |  |  |  |  |
| NR Sidelink Communication resource pool configuration |  | As specified in Table A.11.2-2 |  |  | As specified in Table A.11.2-2 |  |  |
| networkControlledSyncTx |  | N/A |  |  | ON |  |  |
| syncTxThreshOoC | dBm/15 kHz | +infinity |  |  | N/A |  |  |
| SLSSID |  | 59 |  |  | 30 |  |  |
| inCoverage (in MIB-SL) |  | FALSE |  |  | TRUE |  |  |
| Noc Note1 | dBm/30 kHz | -95 |  |  |  |  |  |
| Ês/Noc | dB | -∞ | 0 | 0 | -∞ | -∞ | 0 |
|  | dB | -∞ | 0 | 0 | -∞ | -∞ | 0 |
| PSBCH-RSRP Note2, Note 3 | dBm/30 kHz | -∞ | -95 | -95 | -∞ | -∞ | -95 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: PSBCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: S-PSS Es/Iot and S-SSS Es/Iot are set the same as PSBCH Es/Iot. Note 4: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |  |  |  |

During T1, both SyncRef UE 1 and SyncRef UE 2 are powered off. The UE will select GNSS as synchronization source.

During T2, the UE shall detect, evaluate and reselect SyncRef UE 1 as synchronization source. Then it shall change its S-SSB transmission timing with SLSSID = 395 and inCoverage = false in MasterInformationBlockSidelink after SyncRef UE reselection delay from start of T2.

During T3, the UE will detect, evaluate and reselect SyncRef UE 2 as synchronization source. Then it shall change its S-SSB transmission timing with SLSSID = 30 and inCoverage = false in MasterInformationBlockSidelink after SyncRef UE reselection delay from start of T2.

SyncRef UE reselection delay is defined as the time from the beginning of corresponding time instant to the time UE is synchronized to SyncRef UE and changes its S-SSB transmissions timing and SLSSID to follow SyncRef UE as the synchronization source. The SyncRef UE (re)selection delay can be expressed as:

SyncRef UE reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + S-SSB period

-	Tdetect,SyncRef UE = 8 seconds, is the time to identify newly detectable intra-frequency SyncRef UE as specified in 38.133 [6] clause 12.4 when UE is not directly synchronized to GNSS.

-	Tevaluate,SLSS = 4 S-SSB periods, is the time to measure the PSBCH-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease S-SSB transmissions.

-	S-SSB period = 160 ms.

Which gives SyncRef UE reselection delay = 8800 ms.

The test system will also verify that the UE does not drop or delay more than 6% of its sidelink data and S-SSB transmissions during the duration of T1 and T2.

The rate of correct SyncRef UE reselection observed during repeated tests shall be at least 90%.

### 9.1.4	L1 SL-RSRP measurements

#### 9.1.4.0	Minimum conformance requirements

##### 9.1.4.0.1	Minimum conformance requirements for resource selection/reselection, re-evaluation and pre-emption

The UE physical layer shall be capable of performing the L1 SL-RSRP measurements on the carrier operating NR sidelink communication for determining the subset of resources to be excluded in PSSCH resource selection in sidelink transmission mode 2. The L1 SL-RSRP measurement period corresponds to one slot and the measurement shall meet the L1 SL-RSRP measurement accuracy requirement in 38.133 [6] clause 10.4.4. After resource (re-)selection procedure, re-evaluation is performed on the reserved resources by L1 SL-RSRP measurements before transmission of SCI with reservation when the conditions specified in TS 38.214 [9] are satisfied.

When the pre-emption mechanism is enabled for the resource pool that UE is monitoring and selecting resource from, after UE selects from the resource not excluded based on L1 SL-RSRP measurement procedure, the UE shall be capable of triggering reselection of already signalled resource(s) as a resource reservation when the conditions specified in TS 38.214 [9] are satisfied.

The requirements for absolute accuracy of L1 SL-RSRP in this clause apply to a UE performing PSCCH-RSRP and/or PSSCH-RSRP measurements on the same frequency as used by operating NR sidelink communication.

The accuracy requirements in Table 9.1.4.0.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH and/or PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm and/or PSSCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 9.1.4.0.1-1: Intra-frequency L1 SL-RSRP absolute accuracy for UE capable of NR sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR Sidelink operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |  |  |
| 4.5 | 9 | 0 dB | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | 0 dB | NR_TDD_FR1_B NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |  |

The normative references for this requirement are TS 38.133 [6] clause 10.4.4 and 12.5.

#### 9.1.4.1	NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection

9.1.4.1.1	Test purpose

The purpose of this test is to verify the requirements related to autonomous resource selection / reselection for NR sidelink UE in mode 2 defined in TS 38.133 [6] clause 12.5.

9.1.4.1.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting and NR sidelink communication.

9.1.4.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.4.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.4.1.

9.1.4.1.4	Test description

9.1.4.1.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.4.1.4.1-1.

Table 9.1.4.1.4.1-1: Initial conditions for NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.4.1.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.4.1.4.3

3.	There are 50 NR-SS-UEs (active sidelink UE i, i=0,1,...,49) specified in this test. active sidelink UE i, i=0,1,...,49, is configured according to the generic parameters in Table 9.1.4.1.4.1-2.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [14] Table 4.11.2-2.

Table 9.1.4.1.4.1-2: NR sidelink test parameters for NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 2 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| NR sidelink communication pre-configuration |  |  | As specified in clause A.11.2 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 1111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [8]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 5 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize-r16 included in SL-ResourcePool |  |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| Number of Active Sidelink UEs |  |  | 50 | Active Sidelink UE i = 0, .., 49 |
| SL-Thres-RSRP |  |  | 15Note 3 | Corresponding -100 dBm as defined in Section 6.3.5 in TS 38.331[13] Same for all priority level pairs. |
| Active Sidelink UEs (UE i = 0, .., 49) | NR sidelink Communication preconfiguration |  | As specified in clause A.11.2 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i} Note1 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [8]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool |  | floor(i/10)x10 | Indicates the lowest RB index of the subchannel with the lowest index. UE 0~9 start RB=0; UE 10~19 start RB=10; UE 20~29 start RB=20; UE 30~39 start RB=30; UE 40~49 start RB=40; |
|  | sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| Timing offset among Active Sidelink UEs |  | s | CP/2 | Synchronous |
| Note 1: {1i} is a sequence of nine 0’s with one 1 in (mod(i,10 )+1’th position. Note 2: The UE is only required to be tested in one of the supported test configurations. Note 3: Including test tolerance given in Annex F. |  |  |  |  |

9.1.4.1.4.2	Test procedure

In this test there are 50 active sidelink UEs (UE i, i=0,1,...,49). There are no active cells and GNSS is reliable during the whole test. The test system can emulate and send the GNSS signal to the test UE. DUT and all active sidelink UEs select GNSS as synchronization reference source. The test system shall emulate the active sidelink UEs to transmit PSCCH/PSSCH every 5ms as specified in Table 9.1.4.1.4.1-2.

The test consists of two successive time periods with time duration of T1, T2 respectively. T1 and T2 should be long enough. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1.

3.	Wait at least 20 seconds for the UE to acquire the GNSS signal.

4.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit.

5.	The SS switches on all sidelink UEs and sets parameters according to T1 in Table 9.1.4.1.5-1. T1 starts.

6.	During T1, the SS records:

-	Nresource, total: the number of resources occupied by the PSSCH transmissions of the DUT on all subchannels.

-	Nresource, subch 2 the number of resources occupied by the PSSCH transmissions of the DUT on subchannel #2.

If Nresource, subch 2 (corresponds to the "ne" in Annex G.2.3) and Nresource, total (corresponds to the "ns" in Annex G.2.3) achieve the confidence level according to Tables G.2.3-1, T1 expires.

7.	If the ratio Nresource, subch 2/ Nresource, total is less than 10%, count a success for the event "subchannel #2 resource not available", otherwise count a fail for the event "subchannel #2 resource not available ".

8.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.4.1.5-1. T2 starts.

9.	During T2, the SS records:

-	Nresource, total: the number of resources occupied by PSSCH transmission of the DUT on all subchannels.

-	Nresource, non-subch 2 the number of resources occupied by PSSCH transmission of the DUT on subchannels other than subchannel #2.

If Nresource, non-subch 2 (corresponds to the "ne" in Annex G.2.3) and Nresource, total (corresponds to the "ns" in Annex G.2.3) achieve the confidence level according to Tables G.2.3-1, T2 expires.

10.	If the ratio Nresource, non-subch 2/ Nresource, total is less than 10%, count a success for the event "subchannel #2 resource available", otherwise count a fail for the event " subchannel #2 resource available ".

12.	When T2 expires, the SS sends AT command +CCUTLE to open test loop function and trigger UE to stop transmitting.

Each of the events " subchannel #2 resource not available" and " subchannel #2 resource available" is evaluated independently, resulting in an event verdict pass or fail. If all events pass, the test passes. If one event fails, the test fails.

9.1.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.4.1.4.3-1: SL-ResourcePool (UE under test)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-NumSubchannel-r16 | 5 |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-Thres-RSRP-List-r16 SEQUENCE (SIZE (64)) OF SL-Thres-RSRP-r16 { | 64 entries |  |  |
| SL-Thres-RSRP-r16[k, k=1,..64] | 15 | entry k actual value is -130 +2*15 = -100 dBm |  |
| } |  |  |  |
| } |  |  |  |
| sl-PreemptionEnable-r16 | Not present |  |  |
| sl-TimeResource-r16 | 1111111111 |  |  |
| } |  |  |  |

Table 9.1.4.1.4.3-2: SL-ResourcePool (active sidelink UE 0 ~ 49)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-StartRB-Subchannel-r16 | floor(i/10)x10 | i is the index of active sidelink UE, i = 0, 1,..., 49 | active sidelink UE i |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-Thres-RSRP-List-r16 SEQUENCE (SIZE (64)) OF SL-Thres-RSRP-r16 { | 64 entries |  |  |
| SL-Thres-RSRP-r16[k, k=1,..64] | 0 | entry k special value corresponds to -infinity |  |
| } |  |  |  |
| } |  |  |  |
| sl-PreemptionEnable-r16 | Not present |  |  |
| sl-TimeResource-r16 | {1i} | {1i} is a sequence of nine 0’s with one 1 in (mod(i,10 )+1’th position. | active sidelink UE i |
| } |  |  |  |

Table 9.1.4.1.4.3-3: +CCUTLE (Test procedure, step 4)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.4.1.4.3-4: +CCUTLE (Test procedure, step 12)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.4.1.5	Test requirement

Active sidelink UE specific test parameters are given in Table 9.1.4.1.5-1.

Table 9.1.4.1.5-1: SyncRef UE Specific Test Parameters for NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection

| Parameter | Unit | Active Sidelink UE i (i = 0, .., 49) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel)Note 5 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| PSCCH RMC | - | CC.1A HD |  |
| PSSCH RMC | - | CD.1A HD |  |
| Noc Note1 | dBm/30 kHz | -111 | -121 |
| Ês/Noc Note3 | dB | 10 |  |
| Ês/Iot Note2,3 | dB | 10 |  |
| Ês/Noc Note3 | dB | 20.55 Note 6 | 10 |
| Ês/Iot Note2,3 | dB | 20.55 | 10.00 |
| Ês/Noc Note4 | dB | 0.35 Note 6 | 30.55 Note 6 |
| Ês/Iot Note2,4 | dB | 0 | 30.55 |
| PSSCH-RSRP1 Note 2,3 | dBm/SCS | -90.45 | -111 |
| PSSCH-RSRP2 Note 2,4 | dBm/SCS | -110.65 | -90.45 |
| Io Note 2 | dBm/18 MHz | -69.31 | -63.62 |
|  | dBm/36 MHz | -69.31 | -63.62 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: Es/Iot, PSSCH-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: For UE 20 to 29, occupying subchannel #2 Note 4: For UE 0 to 19 and 30 to 49, occupying subchannel #0/1/3/4 Note 5: The UE is only required to be tested in one of the supported test configurations. Note 6 Including test tolerance given in Annex F. |  |  |  |

During T1, the signal from test equipment are configured such that

-	The measured PSSCH-RSRP for 10 active NR sidelink UEs(UE 20~UE 29) is above SL-Thres-RSRP. Then the resources occupied by these 10 active NR sidelink UEs (resources on subchannel #2) are expected to be excluded in the resource selection procedure and,

-	The measured PSSCH-RSRP for other 40 active NR sidelink UEs(UE 0~UE 19, UE 30~UE 49) is below the SL-Thres-RSRP, and the resources occupied by these 40 active NR sidelink UEs (resources on subchannels other than subchannel #2) are expected to be included in the resource selection procedure.

The rate of PSSCH transmissions on the resources on subchannel #2 shall be less than 10% during T1.

During T2, the signal from test equipment are configured such that

-	the measured PSSCH-RSRP for the 10 active NR sidelink UEs (UE 20~UE 29) is below SL-Thres-RSRP. Then the resources occupied by these 10 active NR sidelink UEs (resources on subchannel #2) are expected to be included in the resource selection procedure and,

-	the measured PSSCH-RSRP for other 40 active NR sidelink UEs (UE 0~UE 19, UE 30~UE 49) is above SL-Thres-RSRP, and the resources occupied by these 40 active NR sidelink UEs (subchannels other than subchannel #2) are expected to be excluded in the resource selection procedure.

The rate of PSSCH transmissions on the resources on subchannel #2 shall be larger than 90% during T2.

Figure 9.1.4.1.5-2 shows the two different time durations and the corresponding variation of the power level in the active sidelink UEs to emulate change of candidate resource set.

Figure 9.1.4.1.5-2: Power level variation of active sidelink UEs for NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection

#### 9.1.4.2	NR SA FR1 L1 SL-RSRP measurement for resource pre-emption

9.1.4.2.1	Test purpose

The purpose of this test is to verify the requirements related to autonomous resource pre-emption for NR sidelink UE in mode 2 defined in TS 38.133 [6] clause 12.5.

9.1.4.2.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting and NR sidelink communication.

9.1.4.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.4.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.4.2.

9.1.4.2.4	Test description

9.1.4.2.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.4.2.4.1-1.

Table 9.1.4.2.4.1-1: Initial conditions for NR SA FR1 L1 SL-RSRP measurement for resource pre-emption

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.4.2.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.4.2.4.3

3.	There are 1 NR-SS-UEs (active NR sidelink UE) specified in this test. active NR sidelink UE is configured according to the generic parameters in Table 9.1.4.2.4.1-2.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [14] Table 4.11.2-2.

Table 9.1.4.2.4.1-2: NR sidelink test parameters for NR SA FR1 L1 SL-RSRP measurement for resource pre-emption

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | HD carrier in Band n47 and n38 |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool in SL-ResourcePool in sl-TxPoolSelectedNormal-r16 |  |  | 10000000000000000000 | Indicates the time resource of resource pool within sl-Period. (see TS 38.213 [8]) Note that this is for Tx pool. |
| sl-TimeResource-r16 included in SL-ResourcePool in sl-RxPool-r16 |  |  | 11111111111111111111 | Indicates the time resource of resource pool within sl-Period. (see TS 38.213 [8]) Note that this is for Rx pool. |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | Indicates the number of subchannels in the corresponding resource pool, which consists of contiguous PRBs only |
| sl-SubchannelSize-r16 included in SL-ResourcePool |  |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| sl-StartRB-Subchannel-r16 included in SL-ResourcePool |  |  | 10 | Indicates the lowest RB index of the subchannel with the lowest index. |
| Number of Active Sidelink UEs |  |  | 1 |  |
| SL-Thres-RSRP |  |  | 12 | Corresponding -106 dBm as defined in Section 6.3.8 in TS38.331[13] |
| Active Sidelink UEs | V2X sidelink Communication preconfiguration |  | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[8]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool |  | 0 | Indicates the lowest RB index of the subchannel with the lowest index. |
|  | Sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| Timing offset among Active Sidelink Ues |  | s | CP/2 | Synchronous |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

9.1.4.2.4.2	Test procedure

In this test there is one active sidelink UE. There are no active cells and GNSS is reliable during the whole test. The test system can emulate and send the GNSS signal to the test UE. Both DUT and active sidelink UE select GNSS as synchronization reference source.

The test consists of two successive time periods with time duration of T1, T2 respectively. T1 and T2 should be long enough. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1.

3.	Wait at least 20 seconds for the UE to acquire the GNSS signal.

4.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit.

5.	The SS switches on active sidelink UE and sets parameters according to T1 in Table 9.1.4.2.5-1. T1 starts.

6.	The SS keeps monitoring PSCCH sent by the UE. If the SS detecting a PSCCH sent by the UE which reserves multiple PSSCH resources, T1 expires.

7.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.4.2.5-1. T2 starts.

8.	Denoting the slot occupied by the second PSCCH scheduled by the PSSCH in step 6 as "slot n". The active sidelink UE shall send PSCCH no later than slot n-5 which:

-	indicates higher priority than the PSCCH in step 6, and

-	schedules the same resource in slot n as the PSCCH in step 6.

9.	If the UE sends no PSSCH on the resource in slot n, count a success for the test, otherwise count a fail for the test. T2 expires.

10.	When T2 expires, the SS sends AT command +CCUTLE to open test loop function and trigger UE to stop transmitting

11.	Repeat step 4-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved

9.1.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.4.2.4.3-1: SL-BWP-PoolConfigCommon (UE under test)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-BWP-PoolConfigCommon-r16 ::= SEQUENCE { |  |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { | 1 entry |  |  |
| SL-ResourcePool-r16[1] | SL-ResourcePool-TX | entry 1 Table 9.1.4.2.4.3-2 |  |
| } |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-ResourcePool-r16 | SL-ResourcePool-RX | Table 9.1.4.2.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.4.2.4.3-2: SL-ResourcePool-TX (Table 9.1.4.2.4.3-1)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-Thres-RSRP-List-r16 SEQUENCE (SIZE (64)) OF SL-Thres-RSRP-r16 { | 64 entries |  |  |
| SL-Thres-RSRP-r16[k, k=1,..64] | 12 | entry k actual value is -130 +2*12 = -106 dBm |  |
| } |  |  |  |
| sl-MultiReserveResource-r16 | enabled |  |  |
| } |  |  |  |
| sl-TimeResource-r16 | 10000000000000000000 |  |  |
| } |  |  |  |

Table 9.1.4.2.4.3-3: SL-ResourcePool-RX (Table 9.1.4.2.4.3-1)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-Thres-RSRP-List-r16 SEQUENCE (SIZE (64)) OF SL-Thres-RSRP-r16 { | 64 entries |  |  |
| SL-Thres-RSRP-r16[k, k=1,..64] | 12 | entry k actual value is -130 +2*12 = -106 dBm |  |
| } |  |  |  |
| sl-MultiReserveResource-r16 | enabled |  |  |
| } |  |  |  |
| sl-TimeResource-r16 | 11111111111111111111 |  |  |
| } |  |  |  |

Table 9.1.4.2.4.3-4: SL-LogicalChannelConfig (UE under test)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-12 with condition LO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-LogicalChannelConfig-r16 ::= SEQUENCE { |  |  |  |
| sl-Priority-r16 | 8 | PSSCH sents from DUT has the lowest priority |  |
| } |  |  |  |

Table 9.1.4.2.4.3-5: SL-ResourcePool (active sidelink UE)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-TimeResource-r16 | 11111111111111111111 |  |  |
| } |  |  |  |

Table 9.1.4.2.4.3-6: +CCUTLE (Test procedure, step 4)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.4.2.4.3-7: +CCUTLE (Test procedure, step 10)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.4.2.5	Test requirement

Active sidelink UE specific test parameters are given in Table 9.1.4.2.5-1.

Table 9.1.4.2.5-1: SyncRef UE Specific Test Parameters for NR SA FR1 L1 SL-RSRP measurement for resource pre-emption

| Parameter | Unit | Active Sidelink UE |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel) Note 3 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| PSCCH RMC | - | CC.1A HD |  |
| PSSCH RMC | - | CD.1A HD |  |
| Noc Note1 | dBm/30 kHz | N/A | -100 |
| PSCCH Ês/Noc | dB |  | 5 |
| PSSCH Ês/Noc | dB |  | 5 |
| PSCCH Ês/Iot Note2 | dB |  | 5 |
| PSSCH Ês/Iot Note2 | dB |  | 5 |
| PSSCH-RSRP Note 2 | dBm/30kHz |  | -95 |
| Io Note 2 | dBm/18MHz |  | -73.01 |
|  | dBm/36MHz |  | -73.01 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other Ues and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: Es/Iot, PSSCH-RSRP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

During T1, The UE under test keep transmitting PSCCH/PSSCH and reserve resources in slot n for future transmission.

During T2, The UE under test is required to trigger resource reselection and not to transmit on the reserved resource at slot n when the high priority reservation is transmitted by the active sidelink UE before n-Tpre-empt, where,

Tpre-empt = T3+Tproc,0

-	T3 = 5 slots, is given in 38.214 Table 8.1.4-2.

-	Tproc,0 = 1 slot, is given in 38.214 Table 8.1.4-1.

Which gives a total of Tpre-empt = 6 slots.

The rate of PSSCH transmissions on the resources at slot n shall be less than 10% during repeated tests.

Figure 9.1.4.2.5-2 shows the two different time durations and the corresponding variation of the power level in the active sidelink UE to emulate resource pre-emption.

Figure 9.1.4.2.5-2: Power level variation of active sidelink UE for NR SA FR1 L1 SL-RSRP measurement for resource pre-emption

#### 9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation

9.1.4.3.1	Test purpose

The purpose of this test is to verify the requirements related to autonomous resource re-evaluation for NR sidelink UE in mode 2 defined in TS 38.133 [6] clause 12.5.

9.1.4.3.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting and NR sidelink communication.

9.1.4.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.4.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.4.3.

9.1.4.3.4	Test description

9.1.4.3.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.4.3.4.1-1.

Table 9.1.4.3.4.1-1: Initial conditions for NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.4.3.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.4.3.4.3

3.	There are 130 NR-SS-UEs (active NR sidelink UE i, i = 0, 1, ..., 129) specified in this test. Active NR sidelink UEs are configured according to the generic parameters in Table 9.1.4.3.4.1-2.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [14] Table 4.11.2-2.

Table 9.1.4.3.4.1-2: NR sidelink test parameters for NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | HD carrier in Band n47 and n38 |
| Channel Bandwidth (BWchannel) Note 2 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize-r16 included in SL-ResourcePool |  |  | 10 |  |
| sl-StartRB-Subchannel-r16 |  |  | 0 |  |
| Number of Active Sidelink UEs |  |  | 130 | Active Sidelink UE i = 0, .., 129 |
| SL-Thres-RSRP-r16 |  |  | 14Note 3 | Corresponding -102 dBm as defined in Section 6.3.5 in TS38.331[2] |
| Active Sidelink UEs(UE i=0-99) | V2X sidelink Communication preconfiguration |  | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the size of sub-channels for TX resource pool |
|  | sl-ResourceReservePeriod2-r16 | ms | 50 |  |
| Active Sidelink UEs(UE i= 100-129) | V2X sidelink Communication preconfiguration |  | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize included in SL-ResourcePool |  | 10 | Indicates the size of sub-channels for TX resource pool |
|  | sl-MultiReserveResource-r16 |  | enabled |  |
|  | sl-MaxNumPerReserve-r16 |  | n2 |  |
|  | sl-ResourceReservePeriod2-r16 |  | 0 | Unit:ms |
| Timing offset among Active Sidelink UEs |  | s | CP/2 | Synchronous |
| T0 |  | s | 1 |  |
| T1 |  | ms | 50 |  |
| T2 |  | ms | 50 |  |
| Note 1: {1i} is a sequence of ninety-nine 0’s with one 1 in (mod(i,100)+1)’th position. Note 2: The UE is only required to be tested in one of the supported test configurations. Note 3: Including the test tolerance given in Annex F. |  |  |  |  |

9.1.4.3.4.2	Test procedure

In this test there is 130 active sidelink UEs (denoted as UE i, i = 0,1,...,129). There are no active cells and GNSS is reliable during the whole test. The test system can emulate and send the GNSS signal to the test UE. DUT and all active sidelink UEs select GNSS as synchronization reference source. The first 100 active sidelink UEs are scheduled to send with 50ms periodicity. The last 30 active sidelink UEs are aperiodic service UE with reservation period of 15ms.

The test consists of three successive time periods with time duration of T0, T1 and T2 respectively. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1.

3.	Wait at least 20 seconds for the UE to acquire the GNSS signal.

4.	The SS switches on UE 0~UE 99, triggers UE 0 ~UE 99 to transmit PSCCH/PSSCH with 50ms periodicity, and set power level of UE 0~UE 99 to ensure their measured PSSCH-RSRP is higher than SL-Thres-RSRP. T0 starts.

5.	When T0 expires, the SS sets parameters according to T1 in Table 9.1.4.3.5-1. T1 starts.

6.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit at the end of T1.

7.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.4.3.5-1. T2 starts.

8.	The SS switches off UE 0~UE 29, switches on UE 101~UE 129, and triggers UE 101~UE 129 to transmit PSCCH/PSSCH aperiodically with a time resource assignment interval of 15ms.

9.	During T2, the SS accumulates:

-	Nresource, total: the number of resources occupied by the PSSCH transmissions of the DUT during T2 since the beginning of the test.

-	Nresource, 65-84 the number of resources in slot (i mod 100 = 65,66...,84) occupied by the PSSCH transmissions of the DUT during T2 since the beginning of the test.

10.	When T2 expires, the SS sends AT command +CCUTLE to open test loop function and trigger UE to stop transmitting

11.	The SS switches off all active sidelink UEs.

12.	Repeat step 4-11 until Nresource, total - Nresource, 65-84 (corresponds to the "ne" in Annex G.2.3) and Nresource, total (corresponds to the "ns" in Annex G.2.3) achieve the confidence level according to Tables G.2.3-1.

If the ratio Nresource, 65-84/ Nresource, total is larger than 90%, count a success for the test, otherwise count a fail for the test.

9.1.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.4.3.4.3-1: SL-ResourcePool (UE under test)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-Thres-RSRP-List-r16 SEQUENCE (SIZE (64)) OF SL-Thres-RSRP-r16 { | 64 entries |  |  |
| SL-Thres-RSRP-r16[k, k=1,..64] | 14 | entry k actual value is -130 +2*14 = -102 dBm |  |
| } |  |  |  |
| sl-ResourceReservePeriodList-r16 SEQUENCE (SIZE (1..16)) OF SL-ResourceReservePeriod-r16 { | 2 entries |  |  |
| SL-ResourceReservePeriod-r16[1] CHOICE { |  | entry 1 |  |
| sl-ResourceReservePeriod1-r16 | ms0 |  |  |
| } |  |  |  |
| SL-ResourceReservePeriod-r16[2] CHOICE { |  | entry 2 |  |
| sl-ResourceReservePeriod2-r16 | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-PreemptionEnable-r16 | Not present |  |  |
| sl-TimeResource-r16 | ones(1,100) | ones(1,100) is a sequence of 100 "1"s |  |
| } |  |  |  |

Table 9.1.4.3.4.3-2: SL-ResourcePool (active sidelink UE 0~129)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-Thres-RSRP-List-r16 SEQUENCE (SIZE (64)) OF SL-Thres-RSRP-r16 { | 64 entries |  |  |
| SL-Thres-RSRP-r16[k, k=1,..64] | 14 | entry k actual value is -130 +2*14 = -102 dBm |  |
| } |  |  |  |
| sl-MultiReserveResource-r16 | Not present |  | sidelink UE 0~99 |
|  | enabled |  | sidelink UE 100~129 |
| sl-ResourceReservePeriodList-r16 SEQUENCE (SIZE (1..16)) OF SL-ResourceReservePeriod-r16 { | 2 entries |  |  |
| SL-ResourceReservePeriod-r16[1] CHOICE { |  | entry 1 |  |
| sl-ResourceReservePeriod1-r16 | ms0 |  |  |
| } |  |  |  |
| SL-ResourceReservePeriod-r16[2] CHOICE { |  | entry 2 |  |
| sl-ResourceReservePeriod2-r16 | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-PreemptionEnable-r16 | Not present |  |  |
| sl-TimeResource-r16 | {1i} | {1i} is a sequence of ninety-nine 0’s with one 1 in (mod(i,100)+1)’th position i = 0,1,...,129 | sidelink UE i |
| } |  |  |  |

Table 9.1.4.3.4.3-3: Physical layer parameters for SCI format 1-A (Sidelink UE 0~129)

| Derivation Path: TS 38.508-1 [14] Table 4.3.6.2.1.1-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency resource assignment | Not present | - |  |
| Time domain resource assignment | Indicates only one resource is scheduled | "00000"B | Sidelink UE 0~99 |
|  | Indicate 2 resources are scheduled and the time interval between the 1st resource and 2nd resource is 30 slot (i.e. 15ms) | "11110"B | Sidelink UE 10~129 |
| Resource reservation period | Indicates the 2nd entry in sl-ResourceReservePeriodList, i.e. 50 ms | "1"B | Sidelink UE 0~99 |
|  | Indicates the 1st entry in sl-ResourceReservePeriodList, i.e. 0 ms | "0"B | Sidelink UE 10~129 |

Table 9.1.4.3.4.3-4: +CCUTLE (Test procedure, step 6)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.4.3.4.3-5: +CCUTLE (Test procedure, step 10)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.4.3.5	Test requirement

Active sidelink UE specific test parameters for UE 0~UE 99 and UE 100~UE 129 are given in Table 9.1.4.3.5-1 and Table 9.1.4.3.5-2 respectively.

Table 9.1.4.3.5-1: SyncRef UE Specific Test Parameters for NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation (UE #0...99)

| Parameter | Unit | Active Sidelink UE i (i = 0, .., 99) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel) Note 7 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| PSCCH RMC | - | CC.1A HD |  |
| PSSCH RMC | - | CD.1A HD |  |
| Noc Note1 | dBm/SCS | -110.05Note 13 |  |
| PSSCH1 Ês/Noc Note 3 | dB | 32.60 Note 13 | 32.60 Note 13 |
| PSSCH2 Ês/Noc Note 4 | dB | 2 | 2 |
| PSSCH3 Ês/Noc Note 5 | dB | 14.10 Note 13 | -infinity |
| PSSCH4 Ês/Noc Note 6 | dB | 14.1 Note 13 | 14.10 Note 13 |
| PSSCH1 Ês/Iot Note2,3,10 | dB | 32.60 | -0.45 |
| PSSCH1 Ês/Iot Note2,3,11 |  | 32.60 | 32.60 |
| PSSCH2 Ês/Iot Note2,4 | dB | 2 | -31.05 |
| PSSCH3 Ês/Iot Note2,5 | dB | 14.10 | -infinity |
| PSSCH4 Ês/Iot Note2,6 | dB | 14.10 | 14.10 |
| PSSCH -RSRP1 Note 2, 3 | dBm/SCS | -77.45 | -77.45 |
| PSSCH -RSRP2 Note 2, 4 | dBm/SCS | -108.05 | -108.05 |
| PSSCH -RSRP3 Note 2, 5 | dBm/SCS | -95.95 | -infinity |
| PSSCH -RSRP4 Note 2, 6 | dBm/SCS | -95.95 | -95.95 |
| Io Note 2, 8 | dBm/18 MHz | -74.99 | -56.21 |
|  | dBm/36 MHz | -74.99 | -56.21 |
| Io Note 2, 9 | dBm/18 MHz | -85.13 | -56.20 |
|  | dBm/36 MHz | -85.13 | -56.20 |
| Io Note 2, 10 | dBm/18 MHz | -56.66 | -53.42 |
|  | dBm/36 MHz | -56.66 | -53.42 |
| Io Note 2, 11 | dBm/18 MHz | -56.66 | -56.66 |
|  | dBm/36 MHz | -56.66 | -56.66 |
| Io Note 2, 12 | dBm/18 MHz | -74.99 | -74.99 |
|  | dBm/36 MHz | -74.99 | -74.99 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: Ês/Iot, PSSCH-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: UE #50~64 and UE #85~99 will periodically occupy the subchannels on the slot with "#slot index mod 100" = #50-64 and #85-99. Note 4: UE #30~49 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #30-49. Note 5: UE #0~29 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #0-29. Note 6: UE #65~84 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #65-84. Note 7: The UE is only required to be tested in one of the supported configurations. Note 8: Calculated on slot with “#slot index mod 100” = #0~29. Note 9: Calculated on slot with “#slot index mod 100” = #30~49. Note 10: Calculated on slot with “#slot index mod 100” = #50~59. Note 11: Calculated on slot with “#slot index mod 100” = #60~64 and slot #85~99. Note 12: Calculated on slot with “#slot index mod 100” = #65~84. Note 13 Including test tolerance given in Annex F. |  |  |  |

Table 9.1.4.3.5-2: SyncRef UE Specific Test Parameters for NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation (UE #100...129)

| Parameter | Unit | Active Sidelink UE i (i = 100, ..., 129) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel) Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| PSCCH RMC | - | CC.1A HD |  |
| PSSCH RMC | - | CD.1 A HD |  |
| Noc Note1 | dBm/SCS | -110.05Note 8 |  |
| PSSCH Ês/Noc | dB | -infinity | 33.05 Note 8 |
| PSSCH Ês/Iot Note2,5 | dB | -infinity | 33.05 |
| PSSCH Ês/Iot Note2,6 |  | -infinity | 28.93 |
| PSSCH Ês/Iot Note2,7 |  | -infinity | 0.45 |
| PSSCH-RSRP Note 2, Note 3 | dBm/SCS | -infinity | -77.0 |
| Io Note 2, 5 | dBm/18 MHz | -74.99 | -56.21 |
|  | dBm/36 MHz | -74.99 | -56.21 |
| Io Note 2, 6 | dBm/18 MHz | -85.13 | -56.20 |
|  | dBm/36 MHz | -85.13 | -56.20 |
| Io Note 2, 7 | dBm/18 MHz | -56.66 | -53.42 |
|  | dBm/36 MHz | -56.66 | -53.42 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: Es/Iot, PSSCH-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: UE #100~129 will occupy the subchannels on the slots with “#slot index mod 100”= #0-29 during T2. Note 4: The UE is only required to be tested in one of the supported configurations. Note 5: Calculated on slot with “#slot index mod 100” = #0~29. Note 6: Calculated on slot with “#slot index mod 100” = #30~49. Note 7: Calculated on slot with “#slot index mod 100” = #50~59. Note 8: Including the test tolerance given in Annex F. |  |  |  |

During T0, the SS shall configure UE 0~UE 99 to transmit PSCCH/PSSCH every 50ms. The measured PSSCH-RSRP of UE 0 ~UE 99 is above SL-Thres-RSRP. Hence the resources occupied by UE 0 ~UE 99 are expected to be excluded in the resource selection procedure.

During T1, the SS shall configure UE 0~UE 99 to transmit PSCCH/PSSCH every 50ms. The measured PSSCH-RSRP of UE 0~UE 29, UE 50 ~ UE 99 is above SL-Thres-RSRP, and the measured PSSCH-RSRP of UE 30~UE 49 is below SL-Thres-RSRP. Hence the resource occupied by UE 30 ~UE 49 are expected to be included in the resource selection procedure and other resources are expected to be excluded in the resource selection procedure.

At the end of T1, the UE under test shall be triggered to periodically transmit PSCCH/PSSCH.

During T2, the SS switches off UE 0~UE 29. and switches on UE 100~UE 129. UE 100~UE 129 transmit PSCCH/PSSCH with the maximum number of reserved PSCCH/PSSCH resources equalling n2 and time resource assignment interval as 15ms in the beginning 30 slots. The measured PSSCH-RSRP of UE 100~UE 129 is above SL-Thres-RSRP. Hence the resource occupied by UE 100 ~UE 129 are expected to be excluded in the resource re-evaluation procedure. The UE under test is expected to reselect the resources and transmit the PSCCH/PSSCH in the newly re-evaluated resources.

The rate of PSSCH transmissions on the resources which are occupied by UE #65-84 shall be more than 90% during T2.

Figure 9.1.4.3.5-3 shows the two different time durations and the corresponding variation of the power level in the active sidelink UEs to emulate change of candidate resource set.

Figure 9.1.4.3.5-3: Power level variation of active sidelink UEs for NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation

### 9.1.5	Congestion control measurement

#### 9.1.5.0	Minimum conformance requirements

##### 9.1.5.0.1	Minimum conformance requirements for congestion control measurements

The UE shall be capable of estimating the channel busy ratio for one or more transmission pools indicated by higher layers in TS 38.331[13], based on SL-RSSI measurements provided by the physical layer.

When no sidelink transmissions occur, the UE physical layer shall perform a single-shot SL-RSSI measurement for each sub-channel included in all the slots configured as transmission pools.

The SL-RSSI measurement performed according to this clause shall meet the SL-RSSI measurement accuracy requirements defined in 38.133 [6] clause 10.4.

The UE shall perform channel busy ratio (CBR) measurement based on SL-RSSI measurements as described in TS 38.215 [10].

The intra-frequency SL-RSSI requirements are specified in Table 9.1.5.0.1-1. The requirements apply for measurement period of 1 slot and for any configured measurement bandwidth larger than 10 RBs, provided that:

-	All symbols during each RSSI measurement duration are available for RSSI sampling within the same measurement interval.

Table 9.1.5.0.1-1: Intra-frequency SL-RSSI absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR sidelink operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm/SCSSL |  |  | dBm/BWChannel |
|  |  |  | SCSSL = 15kHz | SCSSL = 30kHz | SCSSL = 60kHz |  |
| 2.5 | 5.5 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | -50 |
|  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | -50 |
| 4.5 | 7.5 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups are as defined in Section 3A.4 for the corresponding NR operating bands. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

The normative references for this requirement are TS 38.133 [6] clause 10.4.3 and 12.6.

#### 9.1.5.1	NR SA FR1 congestion control measurement for concurrent operation

9.1.5.1.1	Test purpose

The purpose of this test is to verify the congestion control measurement requirements in TS 38.133 [6] clause 12.6. This test will also verify that UE makes correct reporting of an event for CBR measurement.

9.1.5.1.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting both Uu and sidelink communication.

9.1.5.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.5.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.5.

9.1.5.1.4	Test description

9.1.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 9.1.5.1.4.1-1.

Table 9.1.5.1.4.1-1: Supported test configurations for FR1 NR Cell

| Configuration | Description |
| --- | --- |
| 9.1.5.1-1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 9.1.5.1-2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 9.1.5.1-3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 9.1.5.1.4.1-2.

Table 9.1.5.1.4.1-2: Initial conditions for NR SA FR1 congestion control measurement for concurrent operation

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | Uu&PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8.2.1. |  |  |
| Channel bandwidth | Uu: As specified by the test configuration selected from Table 9.1.5.1.4.1-1 PC5: As specified by the test configuration selected from Table 9.1.5.1.4.1-3. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.3 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.5.1.4.3

3.	There is one NR Uu carrier and one NR Cell (Cell 1) specified in this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

4.	Test parameters for Cell 1 are given in Table 9.1.5.1.5-2.

5.	There are 4 NR-SS-UEs (active NR sidelink UE i, i = 0,1,2,3) specified in this test. Active NR sidelink UEs are configured according to the generic parameters in Table 9.1.5.1.4.1-3.

Table 9.1.5.1.4.1-3: NR sidelink test parameters for NR SA FR1 congestion control measurement for concurrent operation

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel)Note 2 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication configuration |  |  | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[8]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | ENUMERATED {n1} |
| sl-SubchannelSize included in SL-ResourcePool |  |  | 10 | ENUMERATED {n10} |
| sl-StartRB-Subchannel-r16 |  |  | 0 |  |
| threshS-RSSI-CBR |  |  | 19 | Corresponding -74dBm as defined in Section 6.3.8 in TS38.331[13] |
| Number of Active Sidelink UEs every 50ms |  |  | 4 | Active Sidelink UE i, where i = 0, 1, 2, 3 |
| Active Sidelink UEs (i = 0,1,2,3) | V2X sidelink Communication configuration |  | As specified in Table A.3.21.2-1 and A.3.21.2-3 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[8]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 |  |
|  | sl-SubchannelSize included in SL-ResourcePool |  | 10 |  |
| Timing offset between V2X UE and Active Sidelink UEs |  | s | CP/2 | Synchronous |
| c1-Threshold-r16 |  |  | 2 | Corresponding 0.02 as defined in Section 6.3.2 in TS38.331[13] |
| Hysteresis |  |  | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: {1i} is a sequence of ninety nine 0’s with one 1 in i+1’th position. Note 2: The UE is only required to be tested in one of the channel bandwidths. |  |  |  |  |

9.1.5.1.4.2	Test procedure

In this test there is 4 active sidelink UEs (denoted as UE i, i = 0,1,2,3). There are 1 active cells (Cell 1) and no GNSS during the whole test. DUT and all active sidelink UEs select Cell 1 as synchronization reference source. All active sidelink UEs are scheduled to transmit PSCCH/PSSCH with 50ms periodicity.

The test consists of two successive time periods with time duration of T1 and T2 respectively.

1.	Ensure the UE is in state 3N-B with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The SS switches on all active sidelink UEs, triggers all active sidelink UEs to transmit PSCCH/PSSCH with 50ms periodicity.

3.	The SS sets parameters according to T1 in Table 9.1.5.1.5-1. T1 starts.

4.	The SS sends a RRCReconfiguration message with sl-ConfigDedicatedNR and measConfig to configure event C1 triggered CBR measurement and reporting.

5.	The UE sends a RRCReconfigurationComplete message.

6.	If the UE send no measurmentReport message before T1 expires, then count a success for the event "entry condition is not satisfied", otherwise count a fail for the event "entry condition is not satisfied".

7.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.5.1.5-1. T2 starts.

8.	If the UE sends a measurmentReport message before T2 expires, then count a success for the event "entry condition is satisfied", otherwise count a fail for the event "entry condition is satisfied".

9.	The SS sends a RRCReconfiguration message with measConfig to release configure event C1 triggered CBR measurement and reporting.

10.	The UE sends a RRCReconfigurationComplete message.

11.	Repeat step 3-10 until the confidence level according to Table G.3.3-1 is achieved.

9.1.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.5.1.4.3-1: RRCReconfiguration (Test procedure, step 4 and step 9)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and SIDELINK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 9.1.5.1.4.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-ConfigDedicatedNR-r16 | SL-ConfigDedicatedNR specified in 38.508-1 [14] Table 4.6.6-7 with condition SELECTED |  | Step 4 |
|  | Not present |  | Step 9 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.5.1.4.3-2: MeasConfig (Table 9.1.5.1.4.3-1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList | Not present |  | step 9 |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  | step 4 |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO | MO for PCell |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR-SL-r16 | MeasObjectNR-SL specified in 38.508-1 [14] Table 4.6.3-76A | MO for CBR |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList | Not present |  | step 9 |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  | step 4 |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR-SL-r16 | ReportConfigNR-SL | Table 9.1.5.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToRemoveList | Not present |  | step 4 |
| measIdToRemoveList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId { | 1 entry |  | step 9 |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| measIdToAddModList | Not present |  | step 9 |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  | step 4 |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.5.1.4.3-3: ReportConfigNR-SL (Table 9.1.5.1.4.3-2)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-142A with condition EVENT_C1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR-SL-r16 ::= SEQUENCE { |  |  |  |
| reportType-r16 CHOICE { |  |  |  |
| eventTriggered-r16 SEQUENCE { |  |  |  |
| eventId-r16 CHOICE { |  |  |  |
| eventC1 SEQUENCE { |  |  |  |
| c1-Threshold-r16 | 2 | actual CBR threshold value is 2*0.01 = 2% |  |
| hysteresis-r16 | 0 |  |  |
| timeToTrigger-r16 | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.5.1.4.3-4: SL-ResourcePool (UE under test)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-ThreshS-RSSI-CBR-r16 | 19 | actual value is -112 + 2*19 = -74 dBm |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-ResourceReservePeriodList-r16 SEQUENCE (SIZE (1..16)) OF SL-ResourceReservePeriod-r16 { | 2 entries |  |  |
| SL-ResourceReservePeriod-r16[1] CHOICE { |  | entry 1 |  |
| sl-ResourceReservePeriod1-r16 | ms0 |  |  |
| } |  |  |  |
| SL-ResourceReservePeriod-r16[2] CHOICE { |  | entry 2 |  |
| sl-ResourceReservePeriod2-r16 | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-FilterCoefficient-r16 | fc0 | L3 filtering is disabled |  |
| sl-TimeResource-r16 | ones(1,100) | ones(1,100) is a sequence of 100 "1"s |  |
| } |  |  |  |

Table 9.1.5.1.4.3-5: SL-ResourcePool (active sidelink UE 0~3)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-ResourceReservePeriodList-r16 SEQUENCE (SIZE (1..16)) OF SL-ResourceReservePeriod-r16 { | 2 entries |  |  |
| SL-ResourceReservePeriod-r16[1] CHOICE { |  | entry 1 |  |
| sl-ResourceReservePeriod1-r16 | ms0 |  |  |
| } |  |  |  |
| SL-ResourceReservePeriod-r16[2] CHOICE { |  | entry 2 |  |
| sl-ResourceReservePeriod2-r16 | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-TimeResource-r16 | {1i} | {1i} is a sequence of ninety-nine 0’s with one 1 in (mod(i,100)+1)’th position i = 0,1,2,3 | sidelink UE i |
| } |  |  |  |

Table 9.1.5.1.4.3-6: Physical layer parameters for SCI format 1-A (Sidelink UE 0~3)

| Derivation Path: TS 38.508-1 [14] Table 4.3.6.2.1.1-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency resource assignment | Not present | - |  |
| Time domain resource assignment | Indicates only one resource is scheduled | "00000"B |  |
| Resource reservation period | Indicates the 2nd entry in sl-ResourceReservePeriodList, i.e. 50 ms | "1"B |  |

Table 9.1.5.1.4.3-7: MeasurementReport (Test procedure, step 8)

| Derivation Path: TS 38.38.508-1[14] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { | MeasResults |  |  |
| measId | 1 |  |  |
| measResultServingMOList | Not checked |  |  |
| measResultsSL-r16 SEQUENCE { |  |  |  |
| measResultsListSL-r16 CHOICE { |  |  |  |
| measResultNR-SL-r16 SEQUENCE { |  |  |  |
| measResultListCBR-NR-r16 SEQUENCE (SIZE (1.. maxNrofSL-PoolToMeasureNR-r16)) OF MeasResultCBR-NR-r16 { | 1 entry |  |  |
| MeasResultCBR-NR-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-poolReportIdentity-r16 | 1 |  |  |
| sl-CBR-ResultsNR-r16 | (3..4) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

9.1.5.1.5	Test requirement

Active sidelink UE specific test parameters for UE 0~UE 4 are given in Table 9.1.5.1.5-1. Cell specific test parameters for Cell 1 are given in Table 9.1.5.1.5-2.

Table 9.1.5.1.5-1: SyncRef UE Specific Test Parameters for NR SA FR1 congestion control measurement for concurrent operation

| Parameter | Unit | Active Sidelink UE i (i = 0, 1, 2, 3) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |  |
| NR RF Channel Number |  | 1 |  |  |  |
| Channel Bandwidth (BWchannel) Note 7 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |  |
| PSCCH RMC (defined in A.3.21.3) |  | CC.1A HD |  |  |  |
| PSSCH RMC (defined in A.3.21.3) |  | CD.1A HD |  |  |  |
| Noc Note1 | dBm/30 kHz | -104.55 Note 8 |  |  |  |
| Ês/Noc | dB | 4.35 |  | 13.62 Note 8 |  |
| SL-RSSI1 Note 2,3 | dBm/3.6 MHz | -78.05 |  | -69.95 |  |
| SL-RSSI2 Note 2,4 | dBm/3.6 MHz | -83.76 |  | -83.76 |  |
| Io1 Note 2,5 | dBm/18 MHz | -78.05 |  | -69.95 |  |
|  | dBm/36 MHz | -78.05 |  | -69.95 |  |
| Io2 Note 2,6 | dBm/18 MHz | -83.76 |  | -83.76 |  |
|  | dBm/36 MHz | -83.76 |  | -83.76 |
| Propagation Condition | - | AWGN |  |  |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SL-RSSI1, SL-RSSI2, Io1 and Io2 levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SL-RSSI1 is the SL-RSSI level measured on the slot# 0 - 3 with “SFN mod 5 = 0”. Note 4: SL-RSSI2 is the SL-RSSI level measured on the slot# 4-9 with “SFN mod 5 = 0” and the slot# 0-9 with “SFN mod 5 = 1,…, 4”. Note 5: Io1 is the Io level measured on the slot# 0 - 3 with “SFN mod 5 = 0”. Note 6: Io2 is the Io level measured on the slot# 4-9 with “SFN mod 5 = 0” and the slot# 0-9 with “SFN mod 5 = 1,…, 4”. Note 7: The UE is only required to be tested in one of the supported test configurations. Note 8: Including test tolerance given in Annex F. |  |  |  |  |  |

Table 9.1.5.1.5-2: Cell specific Test Parameters for NR SA FR1 congestion control measurement for concurrent operation

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| RF Channel Number |  |  | 2 |
| Duplex Mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 40: NRB,c = 106 |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| DRX Cycle |  |  | N/A |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.2 |
| OCNG Patterns |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Noc Note2 | Config 1,2,3 | dBm/15 kHz | -98 |
| Noc Note2 | Config 1,2 | dBm/SCS | -98 |
|  | Config 3 |  | -95 |
| Ês/Noc |  | dB | 3 |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -95 |
|  | Config 3 |  | -92 |
| Io Note 3 | Config 1,2 | dBm/9.36 MHz | -65.2 |
|  | Config 3 | dBm/38.1 MHz | -59.2 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

During T1, all of active sidelink UEs are configured to transmit PSCCH/PSSCH every 50ms such that the SL-RSSI on the resources occupied by active sidelink UEs is lower than threshS-RSSI-CBR. Then the measured CBR is lower than c1-Threshold. No CBR measurement reporting shall be triggered.

During T2, all of active sidelink UEs are configured to transmit PSCCH/PSSCH every 50ms such that the SL-RSSI on the resources occupied by active sidelink UEs is higher than threshS-RSSI-CBR. Then the measured CBR is higher than c1-Threshold. CBR measurement reporting shall be triggered.

The rate of correct events observed during repeated tests shall be at least 98%.

#### 9.1.5.2	NR SA FR1 congestion control measurement for PC5-only operation

9.1.5.2.1	Test purpose

The purpose of this test is to verify the congestion control measurement requirements in TS 38.133 [6] clause 12.6.

9.1.5.2.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting only sidelink communication.

9.1.5.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.5.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.5.

9.1.5.2.4	Test description

9.1.5.2.4.1	Initial conditions

Configure the test equipment and the DUT according to the parameters in Table 9.1.5.2.4.1-1.

Table 9.1.5.2.4.1-1: Initial conditions for NR SA FR1 congestion control measurement for PC5-only operation

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8. |  |  |
| Channel bandwidth | PC5: As specified by the test configuration selected from Table 9.1.5.2.4.1-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.1FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.1FFS |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are given in clause 9.1.5.2.4.3

3.	There are 4 NR-SS-UEs (active NR sidelink UE i, i = 0,1,2,3) specified in this test. Active NR sidelink UEs are configured according to the generic parameters in Table 9.1.5.2.4.1-2.

Table 9.1.5.2.4.1-2: NR sidelink test parameters for NR SA FR1 congestion control measurement for PC5-only operation

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel)Note 2 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication configuration |  |  | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[8]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | ENUMERATED {n1} |
| sl-SubchannelSize included in SL-ResourcePool |  |  | 10 | ENUMERATED {n10} |
| sl-StartRB-Subchannel-r16 |  |  | 0 |  |
| threshS-RSSI-CBR |  |  | 19 | Corresponding -74dBm as defined in Section 6.3.8 in TS38.331[13] |
| Number of Active Sidelink UEs every 50ms |  |  | 4 | Active Sidelink UE i, where i = 0, 1, 2, 3 |
| Active Sidelink UEs (i = 0,1,2,3) | V2X sidelink Communication configuration |  | As specified in Table A.3.21.2-1 and A.3.21.2-3 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[8]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 |  |
|  | sl-SubchannelSize included in SL-ResourcePool |  | 10 |  |
| Timing offset between V2X UE and Active Sidelink UEs |  | s | CP/2 | Synchronous |
| sl-CBR-RangeConfigList-r16 |  |  | [2 100] | Two ranges are defined by this list: 0 to 0.02 and 0.02 to 1 |
| sl-CR-Limit-r16 |  |  | 10000 and 10 | Corresponding to the two CBR ranges: if CBR > 0.02, CR ≤ 0.001, otherwise CR > 0.001 |
| sl-Thres-RSRP-r16 |  |  | 12 | Configure threshold <-98.64dBm/30kHz to ensure not blocking transmission |
| Hysteresis |  |  | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: {1i} is a sequence of ninety nine 0’s with one 1 in i+1’th position. Note 2: The UE is only required to be tested in one of the channel bandwidths. |  |  |  |  |

9.1.5.2.4.2	Test procedure

In this test there is 4 active sidelink UEs (denoted as UE i, i = 0,1,2,3). There are 1 active cells (Cell 1) and no GNSS during the whole test. DUT and all active sidelink UEs select Cell 1 as synchronization reference source. All active sidelink UEs are scheduled to transmit PSCCH/PSSCH with 50ms periodicity.

The test consists of two successive time periods with time duration of T1 and T2 respectively.

1.	Ensure the UE is in state 4-A with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The SS switches on all active sidelink UEs, triggers all active sidelink UEs to transmit PSCCH/PSSCH with 50ms periodicity.

3.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to transmit.

4.	The SS sets parameters according to T1 in Table 9.1.5.2.5-1. T1 starts.

5.	During T1, the SS recodes the number of resources occupied by the PSSCH transmission of the UE under test (denoted as Nresource, low CBR:) until T1 expires. If Nresource, low CBR > 10, then count as success for the event "high CR limit", otherwise count a fail for the event "high CR limit".

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.5.2.5-1. T2 starts.

7.	During T2, the SS recodes the number of resources occupied by the PSSCH transmission of the UE under test (denoted as Nresource, high CBR:) until T2 expires. If Nresource, low CBR ≤ 10, then count as success for the event "low CR limit", otherwise count a fail for the event "low CR limit".

8.	When T2 expires, the SS sends AT command +CCUTLE to open test loop function and trigger UE to stop transmitting.

9.	Repeat step 3-7 until the confidence level according to Table G.3.3-1 is achieved.

9.1.5.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.5.2.4.3-1: SL-ResourcePool (UE under test)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-ThreshS-RSSI-CBR-r16 | 19 | actual value is -112 + 2*19 = -74 dBm |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-CBR-PriorityTxConfigList-r16 | SL-CBR-PriorityTxConfigList | Table 9.1.5.2.4.3-2 |  |
| sl-ResourceReservePeriodList-r16 SEQUENCE (SIZE (1..16)) OF SL-ResourceReservePeriod-r16 { | 2 entries |  |  |
| SL-ResourceReservePeriod-r16[1] CHOICE { |  | entry 1 |  |
| sl-ResourceReservePeriod1-r16 | ms0 |  |  |
| } |  |  |  |
| SL-ResourceReservePeriod-r16[2] CHOICE { |  | entry 2 |  |
| sl-ResourceReservePeriod2-r16 | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-FilterCoefficient-r16 | fc0 | L3 filtering is disabled |  |
| sl-TimeResource-r16 | ones(1,100) | ones(1,100) is a sequence of 100 "1"s |  |
| } |  |  |  |

Table 9.1.5.2.4.3-2: SL-CBR-PriorityTxConfigList (Table 9.1.5.2.4.3-1)

| Derivation Path: TS 38.508-1 [14] Table 4.6.6-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CBR-PriorityTxConfigList -r16 ::= SEQUENCE (SIZE (1..8)) OF SL-PriorityTxConfigIndex-r16 { | 8 entries |  |  |
| SL-PriorityTxConfigIndex-r16[k, k=1..8] SEQUENCE { |  | entry k |  |
| sl-DefaultTxConfigIndex-r16 | Not present |  |  |
| sl-Tx-ConfigIndexList-r16 SEQUENCE (SIZE (1.. maxCBR-Level-r16)) OF SL-TxConfigIndex-r16 { | 2 entries |  |  |
| SL-TxConfigIndex-r16[1] | 0 | entry 1 First entry in sl-CBR-PSSCH-TxConfigList is used for the first CBR range in sl-CBR-RangeConfigList, i.e. 0<=CBR<0.02 |  |
| SL-TxConfigIndex-r16[2] | 1 | entry 2 Second entry in sl-CBR-PSSCH-TxConfigList is used for the second CBR range in sl-CBR-RangeConfigList, i.e. 0.02<=CBR<1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.5.2.4.3-3: SL-CBR-CommonTxConfigList (UE under test)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CBR-CommonTxConfigList-r16 ::= SEQUENCE { |  |  |  |
| sl-CBR-RangeConfigList-r16 SEQUENCE (SIZE (1..maxCBR-Config-r16)) OF SL-CBR-LevelsConfig-r16 { | 1 entry |  |  |
| SL-CBR-LevelsConfig-r16[1] SEQUENCE (SIZE (1..maxCBR-Level-r16)) OF SL-CBR-r16 { | 2 entries | entry 1 |  |
| SL-CBR-r16[1] | 2 | entry 1 0<=CBR<0.02 |  |
| SL-CBR-r16[2] | 100 | entry 2 0.02<=CBR<1 |  |
| } |  |  |  |
| } |  |  |  |
| sl-CBR-PSSCH-TxConfigList-r16 SEQUENCE (SIZE (1.. maxTxConfig-r16)) OF SL-CBR-PSSCH-TxConfig-r16 { | 2 entries |  |  |
| SL-CBR-PSSCH-TxConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-CR-Limit-r16 | 10000 | 0.001 <CR <=1 |  |
| sl-TxParameters-r16 | Not present |  |  |
| } |  |  |  |
| SL-CBR-PSSCH-TxConfig-r16[2] SEQUENCE { |  | entry 2 |  |
| sl-CR-Limit-r16 | 10 | CR <=0.001 |  |
| sl-TxParameters-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.1.5.2.4.3-4: SL-ResourcePool (active sidelink UE 0~3)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-ResourceReservePeriodList-r16 SEQUENCE (SIZE (1..16)) OF SL-ResourceReservePeriod-r16 { | 2 entries |  |  |
| SL-ResourceReservePeriod-r16[1] CHOICE { |  | entry 1 |  |
| sl-ResourceReservePeriod1-r16 | ms0 |  |  |
| } |  |  |  |
| SL-ResourceReservePeriod-r16[2] CHOICE { |  | entry 2 |  |
| sl-ResourceReservePeriod2-r16 | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-TimeResource-r16 | {1i} | {1i} is a sequence of ninety-nine 0’s with one 1 in (mod(i,100)+1)’th position i = 0,1,2,3 | sidelink UE i |
| } |  |  |  |

Table 9.1.5.2.4.3-5: Physical layer parameters for SCI format 1-A (Sidelink UE 0~3)

| Derivation Path: TS 38.508-1 [14] Table 4.3.6.2.1.1-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency resource assignment | Not present | - |  |
| Time domain resource assignment | Indicates only one resource is scheduled | "00000"B |  |
| Resource reservation period | Indicates the 2nd entry in sl-ResourceReservePeriodList, i.e. 50 ms | "1"B |  |

Table 9.1.5.2.4.3-6: +CCUTLE (Test procedure, step 3)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Transmit

Table 9.1.5.2.4.3-7: +CCUTLE (Test procedure, step 8)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.5.2.5	Test requirement

Active sidelink UE specific test parameters for UE 0~UE 4 are given in Table 9.1.5.2.5-1.

Table 9.1.5.2.5-1: SyncRef UE Specific Test Parameters for NR SA FR1 congestion control measurement for PC5-only operation

| Parameter | Unit | Active Sidelink UE i (i = 0, 1, 2, 3) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |
| NR RF Channel Number |  | 1 |  |  |
| Channel Bandwidth (BWchannel) Note 7 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |
| PSCCH RMC (defined in A.3.21.3) |  | CC.1A HD |  |  |
| PSSCH RMC (defined in A.3.21.3) |  | CD.1A HD |  |  |
| Noc Note1 | dBm/30 kHz | -104.55 Note 8 |  |  |
| Ês/Noc | dB | 4.35 |  | 13.62 Note 8 |
| SL-RSSI1 Note 2,3 | dBm/3.6 MHz | --78.05 |  | -69.95 |
| SL-RSSI2 Note 2,4 | dBm/3.6 MHz | -83.76 |  | -83.76 |
| Io1 Note 2,5 | dBm/18 MHz | -78.05 |  | -69.95 |
|  | dBm/36 MHz | -78.05 |  | -69.95 |
| Io2 Note 2,6 | dBm/18 MHz | -83.76 |  | -83.76 |
|  | dBm/36 MHz | -83.76 |  | -83.76 |
| Propagation Condition | - | AWGN |  |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SL-RSSI1, SL-RSSI2, Io1 and Io2 levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SL-RSSI1 is the SL-RSSI level measured on the slot# 0 - 3 with “SFN mod 5 = 0”. Note 4: SL-RSSI2 is the SL-RSSI level measured on the slot# 4-9 with “SFN mod 5 = 0” and the slot# 0-9 with “SFN mod 5 = 1,…, 4”. Note 5: Io1 is the Io level measured on the slot# 0 - 3 with “SFN mod 5 = 0”. Note 6: Io2 is the Io level measured on the slot# 4-9 with “SFN mod 5 = 0” and the slot# 0-9 with “SFN mod 5 = 1,…, 4”. Note 7: The UE is only required to be tested in one of the supported test configurations. Note 8: Including test tolerance given in Annex F. |  |  |  |  |

During T1, all of active sidelink UEs are configured to transmit PSCCH/PSSCH every 50ms such that the SL-RSSI on the resources occupied by active sidelink UEs is lower than threshS-RSSI-CBR. Then the measured CBR is lower than 2%. The UE needs to ensure its channel occupancy ratio higher than 1% in T1.

During T2, all of active sidelink UEs are configured to transmit PSCCH/PSSCH every 50ms such that the SL-RSSI on the resources occupied by active sidelink UEs is higher than threshS-RSSI-CBR. Then the measured CBR is higher than 2%. The UE needs to ensure its channel occupancy ratio lower than 1% in T2.

The rate of correct events observed during repeated tests shall be at least 98%.

### 9.1.6	Congestion control measurement

#### 9.1.6.0	Minimum conformance requirements

##### 9.1.6.0.1	Minimum conformance requirements for interruption to WAN due to NR sidelink communication

This clause contains the requirements related to the interruptions on the PCell/serving cell due to NR sidelink communication.

A UE capable of NR sidelink communication may indicate its interest (initiation or termination) in NR sidelink communication to the connected gNB using IE SidelinkUEInformationNR in TS 38.331 [13].

The UE is allowed an interruption of up to the duration shown in Table 9.1.6.0.1-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the V2X sidelink communication configuration message SL-ConfigDedicatedNR in TS 38.331 [13] (setup and release). This interruption is for both uplink and downlink of the PCell/serving cell.

Table 9.1.6.0.1-1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

The normative references for this requirement are TS 38.133 [6] clause 12.7.

#### 9.1.6.1	NR SA FR1 interruption to WAN due to NR sidelink communication

9.1.6.1.1	Test purpose

The purpose of this test is to verify the requirements related to interruptions due to NR sidelink communication as specified in TS 38.133 [6] clause 12.7.1, when UE is out of coverage on the NR sidelink carrier and is associated with a serving cell on a non-sidelink carrier.

9.1.6.1.2	Test applicability

This test applies to all types of NR UEs from release 16 onwards and supporting NR Uu and sidelink communication.

9.1.6.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.6.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.6.1.

9.1.6.1.4	Test description

9.1.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 9.1.6.1.4.1-1.

Table 9.1.6.1.4.1-1: Supported test configurations for FR1 NR Cell

| Configuration | Description |
| --- | --- |
| 9.1.6.1-1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 9.1.6.1-2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 9.1.6.1-3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 9.1.6.1.4.1-2.

Table 9.1.6.1.4.1-2: Initial conditions for NR SA FR1 interruption to WAN due to NR sidelink communication

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | Uu&PC5: As specified in TS 38.508-1 [14] clause 4.3.1.8.2.1. |  |  |
| Channel bandwidth | Uu: As specified by the test configuration selected from Table 9.1.6.1.4.1-1 PC5: As specified by the test configuration selected from Table 9.1.6.1.4.1-4 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.9.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.7.3 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are defined in clause 9.1.6.1.4.3

3.	There is one NR Uu carrier and one NR Cell (Cell 1) specified in this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

4.	Test parameters for Cell 1 are given in Table 9.1.6.1.4.1-3.

5.	There are 8 active sidelink UEs (UE i, i=0,1,...,7) in this test. Test parameters for active sidelink UEs are given in Table 9.1.6.1.4.1-3 and Table 9.1.6.1.4.1-4.

Table 9.1.6.1.4.1-3: General test parameters for NR SA FR1 interruption to WAN due to NR sidelink communication

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number | - | 1, 2 | RF channel 1 is non-sidelink carrier RF channel 2 is sidelink carrier |
| SCS | kHz | 30 |  |
| Active cell | - | Cell 1 | PCell on RF channel number 1 |
| CP length of Cell 1 | - | Normal |  |
| T1 | s | 5.12 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to 2 second if UE does not transmit SidelinkUEInformationNR during this period. |  |
| T3 | s | 10 |  |

Table 9.1.6.1.4.1-4: Sidelink test parameters for NR SA FR1 interruption to WAN due to NR sidelink communication

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | - | 2 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Sidelink Communication configuration |  | - | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
| Number of Active Sidelink UEs |  | - | 8 | Active Sidelink UE i = 0, .., 7 |
| Active Sidelink UEs (UE i = 0, .., 7) | Sidelink Communication configuration | - | As specified in Annex A.11.2 | IE values unless specified otherwise in this test. |
|  | PSCCH Reference Measurement Channel | - | CC.1A HD | As specified in Table A.11.2.2-1 |
|  | PSSCH Reference Measurement Channel | - | CD.1A HD | As specified in Table A.11.2.2-2 |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool | - | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool | - | i | Indicates the lowest RB index of the subchannel with the lowest index for active Sidelink UE i = 0, .., 7. |
|  | PSBCH-RSRP | dBm/30kHz | -95 |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

9.1.6.1.4.2	Test procedure

In this test there is one active cell (Cell 1) and 8 active sidelink UEs transmitting sidelink communication. The UE under test and all active sidelink UEs select the active cell as synchronization source.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

For this test, the UE is triggered by the test loop function or the upper layers to monitor NR sidelink communication.

1.	Ensure the UE is in state 1N-B with generic procedure parameters Test Mode = On according to TS 38.508-1 [14] clause 4.4A.2.

2.	The SS switches on all active sidelink UEs. Active sidelink UEs are configured to transmit PSCCH/PSSCH according to the parameters specified in Table 9.1.6.1.4.1-4.

3.	The SS sends AT command +CCUTLE to close test loop function and trigger UE to receive.

4.	The SS sets parameters according to T1 in Table 9.1.6.1.5-1. T1 starts.

5,	When T1 expires, the SS sends a paging message including matched ng-5G-S-TMSI.

6.	The UE sends an RRCSetupRequest message.

7.	The SS sends an RRCSetup message.

8.	The UE sends an RRCSetupComplete message.

9.	The SS shall switch the power setting from T1 to T2 in Table 9.1.6.1.5-1 upon receiving the RRCSetupComplete message sent by the UE, T2 starts.

10.	If the UE sends a SidelinkUEInformationNR message within 2 seconds from the beginning of T2. continues with step 11, otherwise skips to step 13.

11.	The SS sends an RRCReconfiguration message with SL-ConfigDedicatedNR to provide sidelink communication configuration to the UE.

12.	The UE sends a RRCReconfigurationComplete message.

13.	T2 expires. The SS shall switch the power setting from T2 to T3 in Table 9.1.6.1.5-1. T3 starts.

14. During T3, The SS continuously schedules the UE to perform DL reception in every DL slots and monitoring corresponding ACK/NACK feedback in UL slots.

15.	If the SS observes on missing ACK/NACKs until T3 expiring, then count a success for the test, otherwise count a fail for the test.

16.	When T3 expires, the SS sends AT command +CCUTLE to open test loop function.

9.1.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.6.1.4.3-1: +CCUTLE (Test procedure, step 3)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Close and Receive

Table 9.1.6.1.4.3-2: +CCUTLE (Test procedure, step 16)

Derivation Path: 38.508-1[14] Table 4.7B-1 with condition Open

9.1.6.1.5	Test requirement

Cell-specific test parameters for Cell 1 are given in Table 9.1.6.1.5-1.

Table 9.1.6.1.5-1: Cell specific test parameters for NR SA FR1 interruption to WAN due to NR sidelink communication

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF Channel Number |  |  | 1 |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |
| Duplex Mode | Config 1 |  | FDD |  |  |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| DRX Cycle |  |  | N/A |  |  |
| PDSCH Reference measurement channel | Config 1 |  | N/A | None | SR.1.1 FDD |
|  | Config 2 |  | N/A | None | SR.1.1 TDD |
|  | Config 3 |  | N/A | None | SR.2.1 TDD |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| SMTC Configuration |  |  | SMTC.2 |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Noc Note2 | Config 1,2,3 | dBm/15 kHz | -98 |  |  |
| Noc Note2 | Config 1,2 | dBm/SCS | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Ês/Noc |  | dB | 3 |  |  |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -95 |  |  |
|  | Config 3 |  | -92 |  |  |
| Io Note 3 | Config 1,2 | dBm/9.36 MHz | -65.3 |  |  |
|  | Config 3 | dBm/38.1 MHz | -59.2 |  |  |
| Antenna Configuration |  |  | 1x2 |  |  |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

During T1, the UE is in RRC_IDLE and monitoring the sidelink communication transmission from other active Sidelink UEs on the sidelink communication resources.

During T2, the test system establishes a RRC connection with the UE. No PDSCH is scheduled for UE, and the UE is expected to transmit SidelinkUEInformationNR indicating sl-RxInterestedFreqList. On reception of SidelinkUEInformationNR, the test system shall send RRCReconfiguration message to the UE and wait for the UE to respond with RRCReconfigurationComplete message before transitioning to T3. If the UE does not transmit SidelinkUEInformationNR for up to 2 second, the test system shall transition to T3.

During T3, the UE is continuously scheduled with PDSCH on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during NR sidelink communication. 100% of all expected ACK/NACKs shall be transmitted by the UE.

# 10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1

Editor’s note: Test cases for EN-DC with NR PSCell under CCA and SCell under CCA are also included here.

## 10.1	RRC_CONNECTED state mobility

FFS

## 10.2	Timing

FFS

## 10.3	Signalling characteristics

### 10.3.1	Radio link monitoring

#### 10.3.1.0	General

The requirements in this section apply for radio link monitoring on a carrier frequency with CCA for the following cells:

-	PCell in SA NR operation mode,

-	PSCell in EN-DC operation mode.

-	PSCell in NR-DC operation mode.

The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell and PSCell as specified in TS 38.213 [8]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP.

On each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout,CCA and Qin,CCA for the purpose of monitoring downlink radio link quality of the cell.

The threshold Qout,CCA is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate (BLERout,CCA) as defined in Table 10.3.1.0-1. For SSB based radio link monitoring, Qout_SSB,CCA is derived based on the hypothetical PDCCH transmission parameters listed in Table 10.3.1.0.1-1.

The threshold Qin,CCA is defined as the level at which the downlink radio link quality can be received with significantly higher reliability than at Qout,CCA and shall correspond to the in-sync block error rate (BLERin) as defined in Table 10.3.1.0-1. For SSB based radio link monitoring, Qin_SSB,CCA is derived based on the hypothetical PDCCH transmission parameters listed in Table 10.3.1.0.1-2.

The out-of-sync block error rate (BLERout,CCA) and in-sync block error rate (BLERin,CCA) are determined from the network configuration via parameter rlmInSyncOutOfSyncThreshold signalled by higher layers. When UE is not configured with rlmInSyncOutOfSyncThreshold from the network, UE determines out-of-sync and in-sync block error rates from Configuration #0 in Table 10.3.1.0-1 as default. All requirements in clause 10.3.1.0 are applicable for BLER Configuration #0 in Table 10.3.1.0-1.

Table 10.3.1.0-1: Out-of-sync and in-sync block error rates

| Configuration | BLERout,CCA | BLERin,CCA |
| --- | --- | --- |
| 0 | 10% | 2% |

UE shall be able to monitor up to NRLM RLM-RS resources of the same or different types in each corresponding carrier frequency range, depending on a maximum number Lmax of SSBs per half frame according to TS 38.213 [8], where NRLM is specified in Table 10.3.1.0-2, and meet the requirements as specified in clause 10.3.1.0. UE is not required to meet the requirements in clause 10.3.1.0 if RLM-RS is not configured and no TCI state for PDCCH is activated.

Table 10.3.1.0-2: Maximum number of RLM-RS resources NRLM

| Carrier frequency range of PCell/PSCell | Lmax | Maximum number of RLM-RS resources, NRLM |
| --- | --- | --- |
| FR1 | 8 | 4 |

In the requirements of clause 10.3.1.0, the term RLM-RS SSB occasion not available at the UE refers to when the RLM-RS SSB is configured by gNB in a cell on a carrier frequency subject to CCA, but the first two successive candidate SSB positions for the same SSB index within the set of configured RLM-RS resources are not available at the UE due to DL CCA failures at gNB during the corresponding evaluation period; otherwise the RLM-RS SSB is considered as available at the UE.

The requirements in clause 10.3.1.0 apply for any channelAccessMode configuration [TS 38.331, 13].

The normative reference for this requirement is TS 38.133 [6] clause 8.1A.1.

##### 10.3.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring under CCA

The requirements in this clause apply for each SSB based RLM-RS resource configured for PCell or PSCell, provided that the SSB configured for RLM are actually configured to be transmitted within UE active DL BWP during the entire evaluation period specified in clause 10.3.1.0.1.1 but occasionally may not be transmitted due to CCA operation.

Table 10.3.1.0.1-1: PDCCH transmission parameters for out-of-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4 dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 10.3.1.0.1-2: PDCCH transmission parameters for in-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

The normative reference for this requirement is TS 38.133 [6] clause 8.1A.2.1.

10.3.1.0.1.1	Minimum requirement for SSB based Radio Link Monitoring under CCA

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB,CCA [ms] period becomes worse than the threshold Qout_SSB,CCA within TEvaluate_out_SSB,CCA [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,CCA [ms] period becomes better than the threshold Qin_SSB,CCA within TEvaluate_in_SSB,CCA [ms] evaluation period. During the in-sync evaluation procedure, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers when Lin exceeds Lin,max, where Lin and Lin,max are defined in Table 10.3.1.0.1.1-1.

TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA are defined in Table 10.3.1.0.1.1-1 for FR1.

When concurrent gaps are configured,

-	P value for an RLM-RS resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

Otherwise, for a UE not supporting concurrentMeasGap-r17 or when concurrent gaps are not configured,

For FR1,

-	P=11-TSSBxGP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, and these GAPs are overlapping with some but not all occasions of the SSB RLM-RS resources; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB RLM-RS resources.

When a measurement gap is configured and the measurement gap is not NCSG,

-	an RLM-RS resource is considered to be overlapped with the GAP f it overlaps a measurement gap occasion, and

-	xRP = MGRP

Otherwise, when NCSG measurement gap is configured,

-	an RLM-RS resource is considered to be overlapped with the GAP if it overlaps the VIL1 or VIL2 of NCSG, and

-	xRP = VIRP

If the UE is configured with Pre-MG, an RLM-RS resource is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

When concurrent gaps are configured, an RLM-RS is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to 38.133 [6] section 9.1.8.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

Table 10.3.1.0.1.1-1: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA for FR1

| Configuration | TEvaluate_out_SSB,CCA (ms) |  | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- | --- |
|  | RLM-RS SSB Es/IotNote4 ≥-7 dB | RLM-RS SSB Es/Iot Note4 <-7 dB |  |
| no DRX | Max(200, Ceil(17*P)*TSSB) | Max(200, Ceil(24*P)*TSSB) | Max(100, Ceil((5+Lin)*P)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)) | Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil(13*P)*TDRX | Ceil(16*P)*TDRX | Ceil((5+Lin)*P)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS SSB occasion is not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(10ms, P * TSSB) if no DRX is used, Once per Max(10ms, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320ms, Once per P * TDRX if DRX cycle > 320ms. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.1A.2.2.

10.3.1.0.1.2	Measurement restrictions for SSB based RLM under CCA

The UE is required to be capable of measuring SSB for RLM without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following clauses.

For FR1, when the SSB for RLM is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for RLM without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for RLM without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure SSB for RLM.

The normative reference for this requirement is TS 38.133 [6] clause 8.1A.2.3.

#### 10.3.1.1	Requirements for determining UE in-sync or out-of-sync status

In the test cases specified in clause 10.3.1, any uplink signal transmitted by the UE is used for detecting the in-/out-of-sync state of the UE. In terms of measurement, the uplink signal is verified based on the UE output power:

-	UE output power higher than Transmit OFF power -50 dBm (as defined in TS 38.101-3 [4]) means uplink signal

-	UE output power equal to or less than Transmit OFF power -50 dBm (as defined in TS 38.101-3 [4]) means no uplink signal.

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.1.1.

#### 10.3.1.2	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.3.1.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out-of-sync and in-sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR1 PSCell radio link monitoring requirements in clause 10.3.1.0

10.3.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier, and supporting UL and SSB-based RLM on shared channel access.

10.3.1.2.3	Minimum conformance requirement

The minimum requirements are specified in clause 10.3.1.0.1. The measurement restrictions for SSB based RLM under CCA are specified in clause 10.3.1.0.2. DRX configuration is not used for this test.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.1.2.

10.3.1.2.4	Test description

In the test, UE is configured to perform RLM based on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in Table 10.3.1.2.4.1-1. The test parameters are given in Tables 10.3.1.2.4.1-3, 10.3.1.2.5-1, and 10.3.1.2.5-2 below.

The test consists of three successive time periods, with time duration of T1, T2 and T3, respectively. Figure 10.3.1.2.4-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE transmits according to UL CCA model. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40 ms) in the test.

Figure 10.3.1.2.4-1: SNR variation for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

10.3.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.3.1.2.4.1-1.

Table 10.3.1.2.4.1-1: Supported test configurations for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Configuration | Description |
| --- | --- |
| 10.3.1.2-1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.3.1.2-2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.1.2.4.1-2.

Table 10.3.1.2.4.1-2: Initial conditions for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 10.3.1.2.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is NR FR1 PSCell. The connection setup is done according to the settings in clause C.1.3, and the downlink signal levels as per clause C.1.2.

3.	The test parameters are given in Table 10.3.1.2.4.1-3 below.

4.	Downlink signals for NR cell are initially set up according to clauses C.1.2 and C.1.3.

Table 10.3.1.2.4.1-3: General test parameters for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 1 |
| Active PSCell |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 2 |
| DL CCA model |  |  |  | As specified in clause D.7.2.1 |
| UL CCA model |  |  |  | As specified in clause D.7.2.2 |
| Duplex mode |  | Config 1,2 |  | TDD |
| BWchannel |  | Config 1,2 | MHz | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1,2 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2 |  | ULBWP.1.1 |
| TDD configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |
| CORESET Reference Channel |  | Config 1,2 |  | CR.1.1 CCA |
| SSB configuration for semi-static channel accessNote 4, 6 |  | Config 1,2 |  | SSB.1 CCA |
| SSB configuration for dynamic channel accessNote 5, 6 |  | Config 1,2 |  | SSB.2 CCA |
| DBT window configuration |  | Config 1,2 |  | DBT.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2 |  | FR1 PRACH configuration 1 under CCA |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1,2 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1,2 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 1.04 |
| T3 |  |  | s | 1.04 |
| D1 |  |  | s | 1 |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. NOTE 3: E-UTRAN is in non-DRX mode under test. NOTE 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

10.3.1.2.4.2	Test Procedure

There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell which operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH format 2 with a reporting periodicity as mentioned in the above table 10.3.1.2.4.1-4. The UE transmits the reporting according to UL CCA model. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40 ms) in test 1. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

2.	The SS shall transmit an RRCConnectionReconfiguration message (embedded in RRCConnectionReconfiguration message) configuring the UE for inter-frequency measurements with the corresponding gap pattern and periodic CSI reporting.

3.	The UE shall transmit RRCReconfigurationComplete message (embedded in RRCConnectionReconfigurationComplete message).

4.	Set the parameters according to T1 in Table 10.3.1.2.5-1. Propagation conditions are set according to clause C.2.3. The SS shall enable DL and UL CCA model according to Table 10.3.1.2.5-1. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 10.3.1.2.5-1

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 10.3.1.2.5-1

7.	If the SS:

a)	detects uplink power equal to or higher than minimum output power, according to section 10.3.1.1, in each uplink slot configured for CSI transmission (according to configured CSI periodicity on PUCCH format 2), provided that such slot is not subject to CCA failure (as determined by PCCA_UL in Table 10.3.1.2.5-1), during the period from time point A to time point B; and

b)	does not detect any uplink power higher than OFF power, according to section 10.3.1.1, from time point C (D1 after the start of T3) until T3 expires, the number of successful tests is increased by one.

Otherwise, the number of failed tests is increased by one and proceed to step 10.

8.	When T3 expires the SS shall change the SNR value to T1 as specified in Table 10.3.1.2.5-1.

9.	If the UE has re-established the connection within 2 seconds, proceed with step 11. Otherwise, proceed with step 10.

10.	The SS shall switch off and then on the UE and then proceed with step 1.

11.	Repeat steps 4-9 until the confidence level according to [Tables G TBD in Annex G clause G. TBD] is achieved.

10.3.1.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 10.3.1.2.4.3-1: Common Exception messages for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier centre of [High] range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-6 with conditions gapUE and RLM Table H.3.1-8 with Condition SSB RLM Table H.3.4-4 with Condition gapUE Table H.3.5-4 Table H.3.5-9 Table H.3.10-1 with Condition SSB.1 CCA and SemiStatic, for semi-static channel access; or Condition SSB.2 CCA and Dynamic, for dynamic channel access |

Table 10.3.1.2.4.3-2: RLF-TimersAndConstants for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

10.3.1.2.5	Test Requirement

Table 10.3.1.2.5-1 and Table 10.3.1.2.5-2 define the cell specific primary level settings including test tolerances and the measurement gap configuration for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA.

Table 10.3.1.2.5-1: Cell-specific test parameters for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| DL CCA probability PCCA_DL | Note 6,8 |  |  | PCCA_DL=0.9375 |  |  |
|  | Note 7,8 |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| UL CCA probability PCCA_UL |  |  |  | 1 |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |
| SNRNote 3,4 on RLM-RS |  | Config 1,2 | dB | 1+TT | -7]+TT | -15+TT |
| SNR on other channels and signals |  | Config 1,2 | dB | 1+TT |  |  |
|  |  | Config 1,2 | dBm/SCS | -95+TT |  |  |
| Propagation condition |  |  |  | TDL-C 300 ns 100 Hz |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT windows. NOTE 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3, respectively, in Figure 10.3.1.2.4-1. NOTE 5: The SNR values are specified for testing a UE which supports 2 RX on at least one band. For testing of a UE which supports 4 RX on all bands, the SNR during T3 is defined in section D.4A. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |

Table 10.3.1.2.5-2: Measurement gap configuration for EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| NOTE 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. NOTE 2: Ensure that RLM RS is partially overlapped with measurement gap. |  |

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows.

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting, provided that such slots are not subject to CCA failure.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90 %.

#### 10.3.1.3	EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.3.1.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR1 PSCell radio link monitoring requirements in clause 10.3.1.0.

10.3.1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting UL and SSB-based RLM on shared channel access.

10.3.1.3.3	Minimum conformance requirements

The minimum requirements are specified in clause 10.3.1.0.1. The measurement restrictions for SSB based RLM under CCA are specified in clause 10.3.1.0.2. DRX configuration is not used for this test.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.1.3.

10.3.1.3.4	Test description

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table10.3.1.3.4.1-1. The test parameters are given in Tables10.3.1.3.4.1-3, and 10.3.1.3.5-1 below.

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 10.3.1.3.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE transmits according to UL CCA model.

Figure 10.3.1.3.4-1: SNR variation for EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

10.3.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.3.1.3.4.1-1.

Table 10.3.1.3.4.1-1: Supported test configurations for EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Configuration | Description |
| --- | --- |
| 10.3.1.3-1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.3.1.3-2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.1.3.4.1-2.

Table 10.3.1.3.4.1-2: Initial conditions for EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 10.3.1.3.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is NR FR1 PSCell. The connection setup is done according to the settings in clause C.1.3, and the downlink signal levels as per clause C.1.2

3.	The general test parameters are given in Table 10.3.1.3.4.1-4.

4.	Downlink signals for NR cell are initially set up according to clauses C.1.2 and C.1.3.

Table 10.3.1.3.4.1-4: General test parameters for EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 1 |
| Active PSCell |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 2 |
| DL CCA model |  |  |  | As specified in clause D.7.2.1 |
| UL CCA model |  |  |  | As specified in clause D.7.2.2 |
| Duplex mode |  | Config 1,2 |  | TDD |
| BWchannel |  | Config 1,2 | MHz | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1,2 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |
| CORESET Reference Channel |  | Config 1,2 |  | CR.1.1 CCA |
| SSB configuration for semi-static channel accessNote 3, 5 |  | Config 1,2 |  | SSB.1 CCA |
| SSB configuration for dynamic channel accessNote 4,5 |  | Config 1,2 |  | SSB.2 CCA |
| DBT window configuration |  | Config 1,2 |  | DBT.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2 |  | FR1 PRACH configuration 1 under CCA |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N/A |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 2000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1,2 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking | Config 1,2 |  |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.52 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 2.04 |
| D1 |  |  | s | 2 |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. NOTE 3: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 4: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 5: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

10.3.1.3.4.2	Test procedure

There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell which operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH format 2 with a reporting periodicity as mentioned in the above table 10.3.1.3.4.1-4. The UE transmits the reporting according to UL CCA model. In the test, DRX configuration is not enabled. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

2.	The SS shall transmit an RRCConnectionReconfiguration message (embedded in RRCConnectionReconfiguration message) to configure periodic CSI reporting.

3.	The UE shall transmit RRCReconfigurationComplete message (embedded in RRCConnectionReconfigurationComplete message).

4.	Set the parameters of NR Cell according to T1 in Table 10.3.1.3.5-1. Propagation conditions are set according to clause C.2.3. The SS shall enable DL and UL CCA model according to Table 10.3.1.3.5-1. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 10.3.1.2.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 10.3.1.2.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 10.3.1.3.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 10.3.1.3.5-1. T5 starts.

9.	If the SS detects uplink power equal to or higher than minimum output power, according to section 10.3.1.1, in each uplink slot configured for CSI transmission (according to configured CSI periodicity on PUCCH format 2), provided that such slot is not subject to CCA failure (as determined by PCCA_UL in Table 10.3.1.3.5-1), during the period from time point A to time point F (D1 seconds after the start of time duration T5), the number of successful tests is increased by one. After the expiration of T5, the SS shall proceed with step 11 for the next iteration, or with step 10.

Otherwise, the number of failed tests is increased by one and proceed with step 10.

10.	The SS shall switch off and then on the UE and then proceed with step 11.

11.	Repeat steps 1-9 until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

10.3.1.3.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 10.3.1.3.4.3-1: Common Exception messages for EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier centre of [High] range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-8 with Condition SSB RLM Table H.3.5-4 Table H.3.5-9 Table H.3.10-1 with Condition SSB.1 CCA and SemiStatic, for semi-static channel access; or Condition SSB.2 CCA and Dynamic, for dynamic channel access |

Table 10.3.1.3.4.3-2: RLF-TimersAndConstants for EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms2000 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

10.3.1.3.5	Test Requirement

Table 10.3.1.3.5-1 defines the cell specific primary level settings including test tolerances for EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA.

Table 10.3.1.3.5-1: Cell-specific test parameters for EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA_DL |  | Note 6,8 |  | PCCA_DL=0.9375 |  |  |  |  |
|  |  | Note 7,8 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| UL CCA probability PCCA_UL |  |  |  | 1 |  |  |  |  |
| LCCA_DL |  |  |  | 7 |  |  |  |  |
| WCCA_DL |  |  | ms | TEvaluate_in_SSB,CCANOTE 9 |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1,2 |  | dB | 1+TT | -7 +TT | -15 +TT | -4.5 +TT | 1+TT |
| SNR on other channels and signals | Config 1,2 |  | dB | 1+TT |  |  |  |  |
|  | Config 1,2 |  | dBm/SCS | -95+TT |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT windows. NOTE 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure 10.3.1.3.4-1. NOTE 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4 RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4A. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. NOTE 9: As defined in Table 10.3.1.0.1.1-1. |  |  |  |  |  |  |  |  |

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting, provided that such slots are not subject to CCA failure.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence interval of 95 %.

#### 10.3.1.4	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell under CCA configured with SSB-based RLM RS in DRX mode

FFS

#### 10.3.1.5	EN-DC FR1 radio link monitoring in-sync test for PSCell under CCA configured with SSB-based RLM RS in DRX mode

FFS

### 10.3.2	Interruption

#### 10.3.2.0	Minimum Conformance Requirements

Same minimum conformance as described in section 4.5.2.0 and 10.3.3.0

The normative reference for this requirement is TS 38.133 [6] clauses 8.2.1 and 8.3A.

#### 10.3.2.1	EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents is FFS

- Call setup and test procedure need to be updated

- Applicability may need to be updated

- Statistical analysis to determine test case verdict is FFS

10.3.2.1.1	Test purpose

The purpose of this test is to verify that the E-UTRAN PCell and NR PSCell interruptions during Scell operations on an NR SCC under CCA, are within the requirements.

10.3.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting 2DL CA in NR.

10.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.3.2.0 for E-UTRAN PCell and NR PSCell interruptions during Scell operations on an NR SCC under CCA.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.2.1.

10.3.2.1.4	Test description

10.3.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in this clause. The supported test configurations for LTE PCell and NR PSCell and SCell are shown in Table 10.3.2.1.4.1-1.

This test shall be tested using any of the test configurations in Table 10.3.2.1.4.1-1.

Table 10.3.2.1.4.1-1: Supported test configurations for EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA

| Config | Description |
| --- | --- |
| 10.3.2.1-1 | LTE FDD NR without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.3.2.1-2 | LTE TDD NR without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.2.1.4.1-2.

Table 10.3.2.1.4.1-2: Initial conditions for EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.2.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 10.3.2.1.4.1-3: General test parameters for EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to Cell1, Cell2 and Cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| T1 | s | <10 |  |
| T2 | s | <10 |  |
| T3 | s | <10 |  |
| T4 | s | <10 |  |
| T5 | s | <10 |  |

1.	Message contents are defined in clause 10.3.2.1.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR FR1 cells. Cell 2 is the PSCell and Cell 3 is the deactivated SCell.

3.   There are one E-UTRAN carrier and two NR carriers and three cells specified in the test. Cell 1 is the PCell on E-UTRAN carrier, Cell 2 is the PSCell on one NR carrier and Cell 3 is the NR SCell on the other NR carrier. Both of cell 2 and cell 3 are subject to CCA. Cell 1 is the cell used for connection setup with the power level set according to clause A.6B. Cell 2 and Cell 3 shall be configured according to clauses C.1.1 and C.1.2.

10.3.2.1.4.2	Test procedure

The test consists of five time periods, with duration of T1, T2, T3, T4 and T5. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. Throughout the test, the LTE PCell and NR PSCell are continuously scheduled in DL. The power of signals on cell 1,2 and 3 is not modified during the test.

Prior to T1, a connection is started with cell 2 as the PSCell, and measurements of cell 3 are configured with gap pattern 0, such that cell 3 is reported. This ensures that cell 3 is known at the start of time period T1 and is not itself part of the tested requirement.

The point in time at which the RRC message implying SCell addition is received at the UE antenna connector, defines the start of time period T1. Measurement gap pattern 0 shall be stopped when the SCell is configured.

The point in time at which the RRC message implying SCell addition is received at the UE antenna connector, defines the start of time period T1.

The point in time at which the MAC-CE message implying SCell activation is received at the UE antenna connector, defines the start of time period T2.

The point in time at which the MAC-CE message implying SCell deactivation is received at the UE antenna connector, defines the start of time period T3.

The point in time at which deactivation delay requirement in TS 38.133 [6] section 8.3A are satisfied defines the start of time period T4

The point in time at which the RRC message implying SCell release is received at the UE antenna connector, defines the start of time period T5.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG_and_SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Configure MCG according to TS 36.521-3 [26] clause C.0, C.1 and SCG according to clause C.1.1 and C.1.2 for all downlink physical channels.

3.	The SS shall configure SCell (Cell 3) on the SCC as per TS 38.508-1 [14] clause 7.5.2, with the message content exceptions defined in clause 10.3.2.1.4.3. NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete.

4.	Set the parameters according to T1 in Table 10.3.2.1.5-1. Propagation conditions are set according to clause C.2.1. T1 starts.

5.	SS schedules on PCell and PSCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor DTX on PCell and ACK/NACK/DTX on PSCell.

6.	If more than 99.5% of uplink transmissions on PSCell are received by SS then count a success for the event "ACK/NACK". Otherwise count a fail for the event "ACK/NACK".

6a.	If no longer than X consecutive DTX on PCell is observed by the SS, then count a success for the event “PCell DTX”. Otherwise count a fail for the event “PCell DTX”. Where,

-	X = 1 for inter-band EN-DC, and X = 3 for intra-band EN-DC.7.	If no longer than X consecutive DTX on PSCell is observed by the SS, then count a success for the event "PSCell DTX". Otherwise count a fail for the event "PSCell DTX". Where,

7.	The SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

8.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If any the reconfiguration fails, switch off and on the UE and ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5

9.	Repeat step 2-8 until a test verdict has been achieved.

Each of the events "ACK/NACK", "PCell DTX" and "PSCell DTX" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.TBD in Annex G.TBD is achieved. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

10.3.2.1.4.3	Message contents

FFS

10.3.2.1.5	Test requirements

Table 10.3.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 10.3.2.1.5-1: Cell specific test parameters for known SCell activation case with NR PSCell and SCell under CCA, 160 ms SCell measurement cycle

| Parameter |  | Unit | Cell2 |  |  |  |  | Cell3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| TDD configuration | Config 1,2 |  | TDDConf.1.1 CCA |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 40: NRB,c = 106 |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |
| DL CCA model | Config 1,2 |  | As specified in clause D.7.2.1 |  |  |  |  | As specified in clause D.7.2.1 |  |  |  |  |
| DL CCA probability for semi-static channel accessNote6,8 | PCCA_DL |  | 0.9375 |  |  |  |  | 0.9375 |  |  |  |  |
| DL CCA probability for dynamic channel accessNote7,8 | PCCA_DL_1 |  | 0.75 |  |  |  |  | 0.75 |  |  |  |  |
|  | PCCA_DL_2 |  | 0.75 |  |  |  |  | 0.75 |  |  |  |  |
| UL CCA model | Config 1,2 |  | As specified in clause D.7.2.2 |  |  |  |  | --- |  |  |  |  |
| UL CCA probability for semi-static channel access | PCCA_UL |  | 0.87 |  |  |  |  | --- |  |  |  |  |
| UL CCA probability for dynamic channel access | PCCA_UL |  | 0.75 |  |  |  |  | --- |  |  |  |  |
| Initial BWP Configuration | Config 1,2 |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Dedicated DL BWP Configuration | Config 1,2 |  | DLBWP.1.1 |  |  |  |  | DLBWP.1.1 |  |  |  |  |
| Initial UL BWP Configuration | Config 1,2 |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Dedicated UL BWP Configuration | Config 1,2 |  | ULBWP.1.1 |  |  |  |  | ULBWP.1.1 |  |  |  |  |
| PDSCH reference measurement channel | Config 1,2 |  | SR.1.1 CCA |  |  |  |  | - |  |  |  |  |
| RMSI CORESET Parameters | Config 1,2 |  | CR.1.1 CCA |  |  |  |  | CR.1.1 CCA |  |  |  |  |
| PDCCH CORESET Parameters | Config 1,2 |  | CCR.1.1 CCA |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
| TRS configuration | Config 1,2 |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  | OP.1 |  |  |  |  |
| SSB configuration for semi-static channel accessNote6,8 | Config 1,2 |  | SSB.1 CCA |  |  |  |  | SSB.1 CCA |  |  |  |  |
| SSB configuration for dynamic channel accessNote7,8 | Config 1,2 |  | SSB.2 CCA |  |  |  |  | SSB.2 CCA |  |  |  |  |
| SMTC Configuration | Config 1,2 |  | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |
| DBT window configuration | Config 1,2 |  | DBT.1 |  |  |  |  | DBT.1 |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  | TCI.State.0 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  | 1x2 Low |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104+TT |  |  |  |  | -104+TT |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87+TT |  |  |  |  | -87+TT |  |  |  |  |
| Ês/Iot |  | dB | 17+TT |  |  |  |  | 17+TT |  |  |  |  |
| Ês/Noc |  | dB | 17+TT |  |  |  |  | 17+TT |  |  |  |  |
| IoNote3 | Config 1,2 | dBm/38.16MHz | -52.86+TT |  |  |  |  | -52.86+TT |  |  |  |  |
| Time offset to Cell1 Note 4 |  | ms | 3 |  |  |  |  | 3 |  |  |  |  |
| Time offset to Cell2 Note 5 |  | s | - |  |  |  |  | 3 |  |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. OCNG is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference of signals received between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell at the UE antenna connector including time alignment error between the two cells Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 8: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |  |  |

The UE shall meet the interruption requirements for SCell addition on both the victim PSCC in TS 38.133 [6] clause 8.2.1 and the vicitim LTE PCell in TS 36.133 [23] clause 7.32 of [15] during time T1.

The UE shall meet the interruption requirements for SCell activation on both the victim PSCell in TS 38.133 [6] clause 8.2.1 and the vicitim LTE PCell in TS 36.133 [23] clause 7.32 of [15] during time T2. There shall be a single interruption with time window as specified in TS 38.133 [6] clause 8.3A.2.

The UE shall meet the interruption requirements for SCell deactivation on both the victim PSCell in TS 38.133 [6] clause 8.2.1 and the vicitim LTE PCell in TS 36.133 [23] clause 7.32 of [15] during time T3. There shall be a single interruption with time window as specified in TS 38.133 [6] clause 8.3A.3.

The UE shall meet the interruption requirements for deactivated SCell measurements on both the victim PSCell in TS 38.133 [6] clause 8.2.1 and the vicitim LTE PCell in TS 36.133 [23] clause 7.32 of [15] during time T4. The interruptions shall be within the time window as specified in TS 38.133 [6] clause 8.3A.3.

The UE shall meet the interruption requirements for SCell release on both the victim PSCell in TS 38.133 [6] clause 8.2.1 and the vicitim LTE PCell in TS 36.133 [23] clause 7.32 of [15] during time T5.

The rate of correct events observed during repeated tests shall be at least 90%.

### 10.3.3	SCell activation and deactivation delay

#### 10.3.3.0	Minimum Conformance Requirements

##### 10.3.3.0.1	SCell Activation Delay Requirement for Deactivated SCell under CCA

The requirements in this clause shall apply for the UE configured with one downlink SCell operating with CCA in EN-DC or in standalone NR carrier aggregation and when one SCell operating with CCA is being activated but none of the RRC parameters CO-DurationPerCell-r16, SlotFormatIndicator, and CSI-RS-ValidationWith-DCI-r16 is configured and all of the CSI reporting resources for being-activated SCell are available.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot  n + (THARQ + Tactivation_time_withCCA + TCSI_reporting_withCCA)/NR_slot_length, where:

-	THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. In the event of UE not being able to transmit the acknowledgment due to UL CCA failures: THARQ is extended to also include the time to all next HARQ feedback transmission and retransmission opportunities, until the time of its successful transmission, as specified in TS 38.213 [3]; no extension of THARQ due to UL CCA failures is allowed for Type 2C UL channel access procedure as defined in TS 37.213 [57].

-	Tactivation_time_withCCA is the SCell activation delay in millisecond.

-	If the SCell is known and belongs to FR1, Tactivation_time_withCCA is:

-	TFirstSSB + L1*Trs + 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms.

-	TFirstSSB_MAX + L2,1*TSMTC_MAX + (1 +L2,2)*Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

-	If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled and the SCell can be successfully detected in one attempt, Tactivation_time_withCCA is:

-	TFirstSSB_MAX + (1 + L3,1)*TSMTC_MAX + (2 + L3,2)*Trs + 5ms.-	If the SCell being activated belongs to FR2-2 and if there is at least one active serving cell on that FR2-2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time_withCCA is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2-2 band.

-	If the SCell being activated belongs to FR2-2 and if there is no active serving cell on that FR2-2 band provided that PCell or PSCell is in FR1 or in FR2-2:

-	If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time_withCCA is:

-	3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

-	If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

-	If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_withCCA is:

-	6ms + TFirstSSB_MAX + (23+[N*L4,1])*TSMTC_MAX + (12+[ N *L4,2])*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

-	If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_withCCA is:

-	3ms + TFirstSSB_MAX + (23+[ N*L5,1])*TSMTC_MAX + (12+[ N *L5,2])*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Where,

TSMTC_MAX:

-	In case of intra-band SCell activation, TSMTC_MAX is the longest SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot;

-	In case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated;

-	TSMTC_MAX is bounded to a minimum value of 10ms.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms

TFirstSSB: is the time to the end of the first complete configured SSB burst indicated by the SMTC after slot n + (THARQ+3ms)/NR_slot_length

TFirstSSB_MAX: is the time to the end of first complete configured SSB burst indicated by the SMTC after slot n + (THARQ+3ms)/NR_slot_length when all active serving cells and SCells being activated or released have configured SSB bursts in the same slot for intra-band scenario. In case of inter-band SCell activation, TFirstSSB_MAX is the time to the end of the first complete configured SSB burst of the SCell being activated. In FR2-2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

L1 (L1 ≤ L1,max) is the number of configured SMTC occasions not available at the UE. L1,max = 2 if Trs ≤ 40 ms; otherwise L1,max = 1.

L2,1 (L2,1 ≤ L2,1,max) and L3,1 (L3,1 ≤ L3,1,max) are the numbers of configured SMTC occasions not available at the UE, for a known and unknown SCell activation respectively,

in the SCell being activated, for inter-band scenario, or

in any of the SCells already activated or being activated provided their cell specific reference signals are configured in the same slot, for intra-band scenario

and L2,1,max = 2 if TSMTC_MAX ≤ 40 ms; otherwise L2,1,max = 1. L3,1,max = 2 if TSMTC_MAX ≤ 40 ms; otherwise L3,1,max = 1.

L2,2 (L2,2 ≤ L2,2,max) and L3,2 (L3,2 ≤ L3,2,max)  are the number of configured SMTC occasions not available at the UE in the SCell being activated. L2,2,max = 2 if Trs ≤ 40 ms; otherwise L2,2,max = 1. L3,2,max = 2 if Trs ≤ 40 ms; otherwise L3,2,max = 1.

N = TBD for an FR2-2 unknown SCell activation.

L4,1 (L4,1 ≤ L4,1,max) and L5,1 (L5,1 ≤ L5,1,max) are the numbers of SMTC occasions groups not available at the UE, for an FR2-2 unknown SCell activation,

in the SCell being activated, for inter-band scenario, or

in any of the SCells already activated or being activated provided their cell specific reference signals are configured in the same slot, for intra-band scenario

and L4,1,max = 2 if TSMTC_MAX ≤ 40 ms; otherwise L4,1,max = 1. L5,1,max = 2 if TSMTC_MAX ≤ 40 ms; otherwise L5,1,max = 1.

L4,2 (L4,2 ≤ L4,2,max) and L5,2 (L5,2 ≤ L5,2,max)  are the number of SMTC occasions groups not available at the UE in the FR2-2 unknown SCell being activated. L4,2,max = 2 if Trs ≤ 40 ms; otherwise L4,2,max = 1. L5,2,max = 2 if Trs ≤ 40 ms; otherwise L5,2,max = 1.

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB_CCA ms as defined in clause 9.5A.4.1 with the assumption of M=1.

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS38.331 [2].

Longer delays for RRM measurement requirements, and in case of FR2-2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

When absoluteFrequencySSB is not configured in DownlinkConfigCommon for target SCell but SMTC for target SCell is configured, no requirement would be applied.

TCSI_reporting_withCCA = TCSI_reporting + TCSI_ReportingDelay , where

TCSI_reporting	is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

TCSI_ReportingDelay is the additional delay in transmission of CSI reporting due to UL CCA failures at the UE. If there are no uplink resources for reporting the valid CSI, then the UE shall use the next available opportunities for reporting the corresponding valid CSI as specified in TS 38.213 [3].

Upon exceeding any of the maximum numbers L1,max, L2,1,max, L2,2,max, L3,1,max, and L3,2,max of SMTC occasions or CSI-RS occasions, respectively, not available at the UE, the UE shall abandon the SCell activation procedure.

SCell operating with CCA in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5 measCycleSCell,  5 DRX cycles) before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable in the SMTC occasions available at the UE, according to the cell identification conditions specified in clause 9.2A and 9.3A.

-	the SSB measured during the period equal to max(5 measCycleSCell, 5 DRX cycles) also remains detectable -	the SSB measured during the period equal to max(5 measCycleSCell, 5 DRX cycles) also remains detectable in the SMTC occasions available at the UE during the SCell activation delay according to the cell identification conditions specified in clause 9.2A and 9.3A.

Otherwise SCell operating with CCA in FR1 is unknown.

For the first SCell activation with CCA in FR2-2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell with CCA in FR2-2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

For intra-band CA, the starting point of an interruption window on SpCell or any activated SCell as specified in clause 8.2, shall not occur before slot n+1+ THARQNR slot length  and not occur after slot n+1+THARQ+3+TXNR slot length , where TX is:

-	TFirstSSB, for known SCell activation when SCell measurement cycle is equal to or smaller than 160ms;

-	TFirstSSB_MAX + L2,1* TSMTC_MAX for known SCell activation when SCell measurement cycle is greater than 160ms;

-	TFirstSSB_MAX + L3,1* TSMTC_MAX for unknown SCell activation

For inter-band CA, the starting point of an interruption window on SpCell or any activated SCell as specified in clause 8.2, shall not occur before slot n+1+ THARQNR slot length  and not occur after slot n+1+THARQ+3+TXNR slot length , where TX is:

-	TFirstSSB, for known SCell activation when SCell measurement cycle is equal to, or smaller than, 160ms.

For intra-band CA,

-	While the SCell being activated is known with measurement cycle equal to or smaller than 160ms, no more than one interruption is allowed during SCell activation.

-	While the SCell being activated is known with measurement cycle greater than 160ms, up to 1+L2,1 interruptions are allowed during SCell activation,

-	While the SCell being activated is unknown, up to 1+L3,1 interruptions are allowed during SCell activation. When L3,1>0, performance degradation may be expected on any activated intra-band victim cells during the SCell activation

-	For a single interruption (L=0), interruption window length at SCell activation does not depend on DL CCA failures.

For inter-band CA,

-	For any active cell in the same band with the SCell being activated, the interruption requirements (i.e. number of interruptions and starting point of an interruption) for intra-band CA apply.

-	For any active cell outside the band with the SCell being activated, a single interruption applies

The number of interruptions and length of each interruption window may be different for different victim cells and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell. For a single interruption (L=0), the interruption window length at SCell activation does not depend on DL CCA failures.

Starting from slot n + THARQ + 3 ms where slot n is the slot where SCell activation command is received (as specified in clause 4.3 of TS 38.213 [3]) and until the SCell activation completion at UE, after at least one CSI-RS transmission occasion for the channel measurement and reporting (specified in clause 5.2.2.5 of TS 38.214 [26]), the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

The requirements in this section do not apply when sCellDeactivationTimer [2] is not configured and when Tactivation_time_withCCA exceeds 1280 ms.

The normative reference for this requirement is TS 38.133 [6] clause 8.3A.

##### 10.3.3.0.2	SCell Deactivation Delay Requirement for Activated SCell under CCA

The requirements in this clause shall apply for the UE configured with one downlink SCell operating with CCA in EN-DC or in standalone NR carrier aggregation.

Upon receiving SCell deactivation command in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n+(THARQ +3ms)/NR_slot_length. The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length and not occur after slot n+1+THARQ+3msNR slot length, where NR slot length is with respect to the numerology used in the SCell being deactivated.

Upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n + 3msNR slot length. The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1 and not occur after slot n+1+ 3msNR slot length, where NR slot length is with respect to the numerology used in the SCell being deactivated.The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this section do not apply when sCellDeactivationTimer [2] is not configured and when SCell deactivation delay exceeds 1280 ms.

The normative reference for this requirement is TS 38.133 [6] clause 8.3A.

#### 10.3.3.1	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle

Editor's Note:

- MU and TT analysis is incomplete

- Message contents is FFS

- Call setup and test procedure may need to be updated

- Applicability may need to be updated in 38.522

- Statistical analysis to determine test case verdict is FFS

10.3.3.1.1	Test purpose

The purpose of this test is to verify that the SCell activation and deactivation delays for NR SCell, with NR PSCell and NR SCell both under CCA, are within the requirements, when the SCell is known by the UE at the time of activation and the configured SCell measurement cycle is 160 ms.

10.3.3.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting 2DL CA in NR.

10.3.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.3.3.0.1 and 10.3.3.0.2 for SCell Activation Delay Requirement for Deactivated SCell under CCA and SCell Deactivation Delay Requirement for Activated SCell under CCA correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.3.1.

10.3.3.1.4	Test description

10.3.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in this clause. The supported test configurations for LTE PCell and NR PSCell and SCell are shown in Table 10.3.3.1.4.1-1.

This test shall be tested using any of the test configurations in Table 10.3.3.1.4.1-1.

Table 10.3.3.1.4.1-1: Supported test configurations for EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle

| Configuration | Description |
| --- | --- |
| 10.3.3.1-1 | PCC: LTE FDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.3.3.1-2 | PCC: LTE TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.3.1.4.1-2.

Table 10.3.3.1.4.1-2: Initial conditions for EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 10.3.3.1.4.1-3: General test parameters for EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | Three radio channels (1, 2, 3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 3 timing offset to Cell 2 | s | 0 |  |
| Time alignment error between Cell 3 and Cell 2 | s | TAE as specified in TS 38.104 [28] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time PCell and PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] depends on UE’s capability |
| TCSI_Reporting | ms | 10+5⋅2µDL | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214 [9]) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13] µDL is the subcarrier spacing configuration for DL |

1.	Message contents are defined in clause 10.3.3.1.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR FR1 cells. Cell 2 is the PSCell and Cell 3 is the deactivated SCell.

10.3.3.1.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell: Cell 1 (PCell) on radio channel 1 (PCC) in E-UTRA, Cell 2 (PSCell) on radio channel 2 (PSCC) in NR, and Cell3 (SCell) on radio channel 3 (SCC) in NR. Before the test starts the UE is connected to Cell 1 and Cell 2, but is not aware of Cell 3, as the UE is only monitoring PCC and PSCC. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

The point in time at which UE receives an RRC message by which the SCell (Cell 3) becomes configured on radio channel 2 defines the start of time period T1. During T1, UE starts monitoring the SCC. At the end of T1, the test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report a valid CSI in PSCell for the activated SCell at latest in slot m +  (THARQ+ Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, as defined in 38.133 [6] clause 8.3A.2. The UE shall start reporting CSI in PSCell in first available uplink resource for CSI reporting after at least one CSI-RS transmission occasion for channel measurement and reporting following slot m+ THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption shall fall within the time window specified in TS 38.133 [6] clause 8.3A.2.

Time period T3 starts when the MAC message is received by at the UE antenna connector, in a slot # denoted n, The UE shall complete the activation at latest in slot n+THARQ+3msNR slot length. Any PSCell interruption shall fall within the time window specified in TS 38.133 [6] clause 8.3A.3.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Tables 10.3.3.1.5-1 and A.6B.1.1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	T1 starts. Immediately after, the SS shall configure SCell (Cell 3) on the SCC as per TS 38.508-1 [14] clause 7.5.2, with the message content exceptions defined in clause 10.3.3.1.4.3. NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete.

3a.	The UE send a MeasurementReport message embedded in ULInformationTransferMRDC.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted m and T2 starts in slot m. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 6, otherwise go to step 9.

After at least one CSI-RS transmission occasion for channel measurement, the UE shall start sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PSCell according to the following criteria:

-	If the first CSI report for SCell is received by the SS no later than slot sfirst_CSI=m+1+THARQ+3ms+TCSI_ReportingNR slot length,

-	or slot sfirst_CSIint=m+1+THARQ+3ms+TCSI_Reporting+TXNR slot length+Ninterruption+1 if the slot sfirst_CSI was subject to interruption,

-	or the next CSI report occasion if there is no CSI report occasion in slots sfirst_CSI or sfirst_CSIint,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to slot snonzero_CQI=m+THARQ+Tactivation_time+TCSI_ReportingNR slot length,

-	or slot snonzero_CQIint=maxsfirst_CSIint,snonzero_CQI if slot snonzero_CQI was subject to interruption,

-	or the next available CSI report occasion if there is no CSI report occasion for reporting the valid CSI in slot snonzero_CQI, or slot snonzero_CQIint if snonzero_CQI was subject to interruption,

-	and DTX is not observed by the SS outside the slot m+1+THARQNR slot length to m+1+THARQ+3ms+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T2

-	Then the number of successes for the event "Activation" is increased by one. Otherwise, count a fail for the event "Activation" and go to step 9.

7.	When T2 expires, the SS deactivate SCC by sending the deactivation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted n and T3 starts in slot n. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 8, otherwise go to step 9.

8.	The UE shall stop sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PSCell during SCell deactivation.

-	If the last CSI report is received by the SS earlier than or equal to slot n+THARQ+3msNR slot length

-	and DTX is not observed by the SS outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length+Ninterruption+THARQNR slot length up to the end of T3,

-	Then the number of successes for the event "Deactivation" is increased by one. Otherwise, count a fail for the event "Deactivation".

9.	When T3 expires, or Activation in step 5 was not acknowledged, or a fail was counted for the event "Activation" in step 6, or Deactivation in step 7 was not acknowledged, the SS shall transmit an RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508[25] Table 4.6.1-8 to release NR PSCell. The UE shall transmit an RRCConnectionReconfigurationComplete message.

10.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

11.	The SS shall transmit an RRCConnectionReconfiguration message with condition MCG and SCG according to TS 36.508 [25] Table 4.6.1-8 to ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. The UE shall transmit an RRCConnectionReconfigurationComplete message.

12.	Repeat steps 2-11 until a test verdict has been achieved.

Each of the events "Activation" and "Deactivation" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.TBD in Annex G.TBD is achieved. Different events may require different times for a verdict. If all events pass, the test passes. If one event fails, the test fails.

10.3.3.1.4.3	Message contents

FFS

10.3.3.1.5	Test requirements

Table 10.3.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 10.3.3.1.5-1: Cell specific test parameters for EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle

| Parameter |  | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1,2 |  | TDD |  |  | TDD |  |  |
| TDD configuration | Config 1,2 |  | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |  |
| BWchannel | Config 1,2 | MHz | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DL CCA model |  |  | As specified in clause D.7.2.1 |  |  | As specified in clause D.7.2.1 |  |  |
| UL CCA model |  |  | As specified in clause D.7.2.2 |  |  | As specified in clause D.7.2.2 |  |  |
| DL CCA probability for semi-static channel accessNote5,7 | PCCA_DL |  | 0.9375 |  |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel accessNote6,7 | PCCA_DL_1 |  | 0.75 |  |  | 0.75 |  |  |
|  | PCCA_DL_2 |  | 0.75 |  |  | 0.75 |  |  |
| UL CCA probability for semi-static channel access | PCCA_UL |  | 0.87 |  |  | 0.87 |  |  |
| UL CCA probability for dynamic channel access | PCCA_UL |  | 0.75 |  |  | 0.75 |  |  |
| LCCA_DL Note 8 |  |  | 2 |  |  | 2 |  |  |
| WCCA_DL Note 8 |  |  | Tactivation_time_withCCA |  |  | Tactivation_time_withCCA |  |  |
| Initial downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Initial uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Dedicated downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Dedicated uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1,2 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1,2 |  | SR.1.1 CCA |  |  | SR.1.1 CCA |  |  |
| Dedicated CORESET parameters | Config 1,2 |  | CCR.1.3 CCA |  |  | CCR.1.3 CCA |  |  |
| RMSI CORESET parameters | Config 1,2 |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |  |
| OCNG Patterns Note1 |  |  | OP.1 |  |  | OP.1 |  |  |
| SSB Configuration for semi-static channel accessNote5,7 | Config 1,2 |  | SSB.1 CCA |  |  | SSB.1 CCA |  |  |
| SSB Configuration for dynamic channel accessNote6,7 | Config 1,2 |  | SSB.2 CCA |  |  | SSB.2 CCA |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  | SMTC.1 |  |  |
| DBT window configuration |  |  | DBT.1 |  |  | DBT.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | dBm/15kHz | -104+TT |  |  | -104+TT |  |  |
| Noc Note2 | Config 1,2 | dBm/SCS | -101+TT |  |  | -101+TT |  |  |
| Ês/Iot |  | dB | 17+TT |  |  | 17+TT |  |  |
| Ês/Noc |  | dB | 17+TT |  |  | 17+TT |  |  |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -84+TT |  |  | -84+TT |  |  |
| IoNote3 | Config 1,2 | dBm/ 38.16MHz | -52.87+TT |  |  | -52.87+TT |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that resources in the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. OCNG is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 8: As specified in 38.133 [6] clause 8.3A for L1,max, L2,1,max, L2,2,max, L3,1,max, and L3,2,max |  |  |  |  |  |  |  |  |

During T2, starting after at least one CSI-RS transmission occasion for channel measurement and reporting from the slot specified in clause 4.3 of TS 38.213 [8] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting no later than slot m + (THARQ+ Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB + L1*Trs + 5ms, as specified in TS 38.133 [6] clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3A.3.

During T2, interruption on PSCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB.

During T3, interruption on PSCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PSCell shall not be more than specified for EN-DC in TS 38.133 [6] clause 8.2.1.2.4.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

#### 10.3.3.2	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle

Editor's Note:

- MU and TT analysis is incomplete

- Message contents is FFS

- Call setup and test procedure may need to be updated

- Applicability may need to be updated

- Statistical analysis to determine test case verdict is FFS

10.3.3.2.1	Test purpose

The purpose of this test is to verify that the SCell activation and deactivation delays for NR SCell, with NR PSCell and NR SCell both under CCA, are within the requirements, when the SCell is known by the UE at the time of activation and the configured SCell measurement cycle is 640 ms.

10.3.3.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting 2DL CA in NR.

10.3.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.3.3.0.1 and 10.3.3.0.2 for SCell Activation Delay Requirement for Deactivated SCell under CCA and SCell Deactivation Delay Requirement for Activated SCell under CCA correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.3.2.

10.3.3.2.4	Test description

10.3.3.2.4.1	Initial conditions

Same initial conditions as described in section 10.3.3.1.4.1 with following exception:

-	The listed parameter values in Tables 10.3.3.2.4.1-3 will replace the values of corresponding parameters in Tables 10.3.3.1.4.1-3.

Table 10.3.3.2.4.1-3: General test parameters for EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |

10.3.3.2.4.2	Test procedure

Same test procedure as described in section 10.3.3.1.4.2.

10.3.3.2.4.3	Message contents

FFS

10.3.3.2.5	Test requirements

Table 10.3.3.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 10.3.3.2.5-1: Cell specific test parameters for EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle

| Parameter |  | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1,2 |  | TDD |  |  | TDD |  |  |
| TDD configuration | Config 1,2 |  | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |  |
| BWchannel | Config 1,2 | MHz | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DL CCA model |  |  | As specified in clause D.7.2.1 |  |  | As specified in clause D.7.2.1 |  |  |
| UL CCA model |  |  | As specified in clause D.7.2.2 |  |  | As specified in clause D.7.2.2 |  |  |
| DL CCA probability for semi-static channel accessNote5,7 | PCCA_DL |  | 0.9375 |  |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel accessNote6,7 | PCCA_DL_1 |  | 0.75 |  |  | 0.75 |  |  |
|  | PCCA_DL_2 |  | 0.75 |  |  | 0.75 |  |  |
| UL CCA probability for semi-static channel access | PCCA_UL |  | 0.87 |  |  | 0.87 |  |  |
| UL CCA probability for dynamic channel access | PCCA_UL |  | 0.75 |  |  | 0.75 |  |  |
| LCCA_DL Note 8 |  |  | 2 |  |  | 2 |  |  |
| WCCA_DL Note 8 |  |  | Tactivation_time_withCCA |  |  | Tactivation_time_withCCA |  |  |
| Initial downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Initial uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Dedicated downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Dedicated uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1,2 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1,2 |  | SR.1.1 CCA |  |  | SR.1.1 CCA |  |  |
| Dedicated CORESET parameters | Config 1,2 |  | CCR.1.3 CCA |  |  | CCR.1.3 CCA |  |  |
| RMSI CORESET parameters | Config 1,2 |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |  |
| OCNG Patterns Note1 |  |  | OP.1 |  |  | OP.1 |  |  |
| SSB Configuration for semi-static channel accessNote5,7 | Config 1,2 |  | SSB.1 CCA |  |  | SSB.1 CCA |  |  |
| SSB Configuration for dynamic channel accessNote6,7 | Config 1,2 |  | SSB.2 CCA |  |  | SSB.2 CCA |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  | SMTC.1 |  |  |
| DBT window configuration |  |  | DBT.1 |  |  | DBT.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | dBm/15kHz | -104+TT |  |  | -104+TT |  |  |
| Noc Note2 | Config 1,2 | dBm/SCS | -101+TT |  |  | -101+TT |  |  |
| Ês/Iot |  | dB | 17+TT |  |  | 17+TT |  |  |
| Ês/Noc |  | dB | 17+TT |  |  | 17+TT |  |  |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -84+TT |  |  | -84+TT |  |  |
| IoNote3 | Config 1,2 | dBm/ 38.16MHz | -52.87+TT |  |  | -52.87+TT |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that resources in the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. OCNG is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 8: As specified in 38.133 [6] clause 8.3A for L1,max, L2,1,max, L2,2,max, L3,1,max, and L3,2,max |  |  |  |  |  |  |  |  |

During T2, starting after at least one CSI-RS transmission occasion for channel measurement and reporting from the slot specified in clause 4.3 of TS 38.213 [8] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting no later than slot m + (THARQ+ Tactivation_time_withCCA + TCSI_reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB_MAX + L2,1*TSMTC_MAX + (1 +L2,2)*Trs + 5ms, as specified in TS 38.133 [6] clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3A.3.

During T2, interruption on PSCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB_MAX + L2,1* TSMTC_MAX.

During T3, interruption on PSCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PSCell shall not be more than specified for EN-DC in TS 38.133 [6] clause 8.2.1.2.4.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

#### 10.3.3.3	EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents is FFS

- Call setup and test procedure may need to be updated

- Applicability may need to be updated

- Statistical analysis to determine test case verdict is FFS

10.3.3.3.1	Test purpose

The purpose of this test is to verify that the SCell activation and deactivation delays for NR SCell, with NR PSCell and NR SCell both under CCA, are within the requirements, when the when the SCell is unknown to the UE at the time of activation.

10.3.3.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting 2DL CA in NR.

10.3.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.3.3.3.1 and 10.3.3.3.2 for SCell Activation Delay Requirement for Deactivated SCell under CCA and SCell Deactivation Delay Requirement for Activated SCell under CCA correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.3.3.

10.3.3.3.4	Test description

10.3.3.3.4.1	Initial conditions

Same initial conditions as described in section 10.3.3.1.4.1 with following exception:

-	The listed parameter values in Tables 10.3.3.3.4.1-3 will replace the values of corresponding parameters in Tables 10.3.3.1.4.1-3.

Table 10.3.3.3.4.1-3: General test parameters for EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |

10.3.3.3.4.2	Test procedure

Same test procedure as described in section 10.3.3.3.4.2, except step 3 and step 5 are replaced by following steps:

3.	T1 starts. Immediately after, the SS shall configure SCell (Cell 3) on the SCC as per TS 38.508-1 [14] clause 7.5.2, with the message content exceptions defined in clause 10.3.3.3.4.3. NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. The SCell (Cell 3) shall be powered OFF till T2 starts.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted m, power ON the SCell (Cell3), T2 starts in slot m. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 6, otherwise go to step 9.

and:

- Step 3a is removed.

10.3.3.3.4.3	Message contents

FFS

10.3.3.3.5	Test requirements

Table 10.3.3.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 10.3.3.3.5-1: Cell specific test parameters for EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA

| Parameter |  | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1,2 |  | TDD |  |  | TDD |  |  |
| TDD configuration | Config 1,2 |  | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |  |
| BWchannel | Config 1,2 | MHz | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DL CCA model |  |  | As specified in clause D.7.2.1 |  |  | As specified in clause D.7.2.1 |  |  |
| UL CCA model |  |  | As specified in clause D.7.2.2 |  |  | As specified in clause D.7.2.2 |  |  |
| DL CCA probability for semi-static channel accessNote5,7 | PCCA_DL |  | 0.9375 |  |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel accessNote6,7 | PCCA_DL_1 |  | 0.75 |  |  | 0.75 |  |  |
|  | PCCA_DL_2 |  | 0.75 |  |  | 0.75 |  |  |
| UL CCA probability for semi-static channel access | PCCA_UL |  | 0.87 |  |  | 0.87 |  |  |
| UL CCA probability for dynamic channel access | PCCA_UL |  | 0.75 |  |  | 0.75 |  |  |
| LCCA_DL Note 8 |  |  | 2 |  |  | 2 |  |  |
| WCCA_DL Note 8 |  |  | Tactivation_time_withCCA |  |  | Tactivation_time_withCCA |  |  |
| Initial downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Initial uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Dedicated downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Dedicated uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1,2 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1,2 |  | SR.1.1 CCA |  |  | SR.1.1 CCA |  |  |
| Dedicated CORESET parameters | Config 1,2 |  | CCR.1.3 CCA |  |  | CCR.1.3 CCA |  |  |
| RMSI CORESET parameters | Config 1,2 |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |  |
| OCNG Patterns Note1 |  |  | OP.1 |  |  | OP.1 |  |  |
| SSB Configuration for semi-static channel accessNote5,7 | Config 1,2 |  | SSB.1 CCA |  |  | SSB.1 CCA |  |  |
| SSB Configuration for dynamic channel accessNote6,7 | Config 1,2 |  | SSB.2 CCA |  |  | SSB.2 CCA |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  | SMTC.1 |  |  |
| DBT window configuration |  |  | DBT.1 |  |  | DBT.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | dBm/15kHz | -104+TT |  |  | -104+TT |  |  |
| Noc Note2 | Config 1,2 | dBm/SCS | -101+TT |  |  | -101+TT |  |  |
| Ês/Iot |  | dB | 17+TT |  |  | 17+TT |  |  |
| Ês/Noc |  | dB | 17+TT |  |  | 17+TT |  |  |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -84+TT |  |  | -84+TT |  |  |
| IoNote3 | Config 1,2 | dBm/ 38.16MHz | -52.87+TT |  |  | -52.87+TT |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that resources in the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. OCNG is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 8: As specified in 38.133 [6] clause 8.3A for L1,max, L2,1,max, L2,2,max, L3,1,max, and L3,2,max |  |  |  |  |  |  |  |  |

During T2, starting after at least one CSI-RS transmission occasion for channel measurement and reporting from the slot specified in clause 4.3 of TS 38.213 [8] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting no later than slot m + (THARQ+ Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB + L1*Trs + 5ms, as specified in TS 38.133 [6] clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3A.3.

During T2, interruption on PSCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB.

During T3, interruption on PSCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PSCell shall not be more than specified for EN-DC in TS 38.133 [6] clause 8.2.1.2.4.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

### 10.3.4	Beam failure detection and link recovery procedures

#### 10.3.4.0	General

The requirements for link recovery procedure in the clause apply when CCA is used on a serving frequency on the downlink.

The UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set q0 as specified in TS 38.213 [8] in order to detect beam failure on:

-	PCell in SA operation mode,

-	PSCell in EN-DC operation mode.

-	PSCell in NR-DC operation mode.

The RS resource configurations in the set q0 can be periodic SSBs. UE is not required to perform beam failure detection outside the active DL BWP. UE is not required to meet the requirements in clause 10.3.4.0.1 and 10.3.4.0.2 if UE does not have set q0.

On each RS resource configuration in the set q0, the UE shall estimate the radio link quality and compare it to the threshold Qout_LR,CCA for the purpose of accessing downlink radio link quality of the serving cell beams.

The threshold Qout_LR,CCA is defined as the level at which the downlink radio level link of a given resource configuration on set q0 cannot be reliably received and shall correspond to the BLERout,CCA = 10% block error rate of a hypothetical PDCCH transmission. For SSB based beam failure detection, Qout_LR_SSB,CCA is derived based on the hypothetical PDCCH transmission parameters listed in Table 10.3.4.0.1-1.

Upon request the UE shall deliver configuration indexes from the set q1 as specified in TS 38.213 [8] , to higher layers,  and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR,CCA, which is indicated by higher layer parameter rsrp-ThresholdSSB. The UE applies the Qin_LR,CCA threshold to the L1-RSRP measurement obtained from an SSB. The RS resource configurations in the set  q1 can be periodic SSBs. UE is not required to perform candidate beam detection outside the active DL BWP.

In the requirements of this clause, the term CBD-RS SSB occasions not available at the UE refers to when the CBD-RS SSB is configured by gNB in a cell on a carrier frequency subject to CCA, but the first two successive candidate SSB positions for the same SSB index within the set of configured CBD-RS resources are not available at the UE due to DL CCA failures at gNB during the corresponding evaluation period; otherwise the CBD-RS SSB is considered as available at the UE.

The requirements in this clause apply for any channelAccessMode configuration [TS 38.331, 13].

The normative reference for this requirement is TS 38.133 [6] clause 8.5A.1.

##### 10.3.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA

The requirements in this clause apply for each SSB resource in the set q0 configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 10.3.4.0.1, but occasionally may not be transmitted due to CCA operation.

Table 10.3.4.0.1-1: PDCCH transmission parameters for beam failure instance under CCA

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

The normative reference for this requirement is TS 38.133 [6] clause 8.5A.2.

10.3.4.0.1.1	Minimum requirement for SSB based Beam Failure Detection under CCA

UE shall be able to evaluate whether the downlink radio link quality on the configured BFD-RS SSB resource in set q0 estimated over the last TEvaluate_BFD_SSB_CCA ms period becomes worse than the threshold Qout_LR_SSB,CCA within TEvaluate_BFD_SSB_CCA ms period.

The value of TEvaluate_BFD_SSB_CCA is defined in Table 10.3.4.0.1.1-1 for FR1.

When concurrent gaps are configured,

-	P value for a BFD-RS resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Otherwise, for a UE not supporting concurrentMeasGap-r17 or when concurrent gaps are not configured,

-	P=11-TSSBxGP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the BFD-RS SSB.

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the BFD-RS SSB.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a BFD-RS resource is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a BFD-RS resource is considered to be overlapped with the GAP if it overlaps the VIL1 or VIL2 of NCSG, and

-	xRP = VIRP

-	When concurrent gaps are configured, a BFD-RS is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to 38.133 [6] section 9.1.8.

Longer evaluation period would be expected if the combination of BFD-RS SSB resource, SMTC occasion and  GAP configurations does not meet pervious conditions.

Table 10.3.4.0.1-1: Evaluation period TEvaluate_BFD_SSB_CCA for FR1

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |  |
| --- | --- | --- |
|  | BFD-RS SSB Es/Iot Note2 ≥ -7 dB | BFD-RS SSB Es/Iot Note2 < -7 dB |
| no DRX | Max(50, Ceil((10 P) TSSB)) | Max(50, Ceil((12 P) TSSB)) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 8 P) Max(TDRX,TSSB)) | Max(50, Ceil(1.5 10 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(7 P) TDRX | Ceil(8 P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. Note 2: BFD-RS SSB Es/Iot is the averaged BFD-RS SSB Es/Iot over the most recent previous evaluation period. |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.5A.2.2.

10.3.4.0.1.2	Measurement restriction for SSB based Beam Failure Detection under CCA

The UE is required to be capable of measuring SSB for BFD without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following clauses.

For FR1, when the SSB for BFD measurement is in the same OFDM symbol as CSI-RS for BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure SSB for BFD measurement.

The normative reference for this requirement is TS 38.133 [6] clause 8.5A.2.3.

10.3.4.0.2	Void

##### 10.3.4.0.3	Scheduling availability of UE during beam failure detection under CCA

10.3.4.0.3.1	Scheduling availability of UE performing beam failure detection under CCA with a same subcarrier spacing as PDSCH/PDCCH

In this clause, the same requirements apply as in Clause 4.5.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause 8.5A.7.1.

##### 10.3.4.0.4	Scheduling availability of UE during candidate beam detection under CCA

10.3.4.0.4.1	Scheduling availability of UE performing L1-RSRP measurement under CCA with a same subcarrier spacing as PDSCH/PDCCH on FR1

In this clause, the same requirements apply as in Clause 4.5.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause 8.5A.8.1.

#### 10.3.4.1	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.3.4.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 10.3.4.0.1.

10.3.4.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting UL and SSB-based RLM and link recovery on shared channel access.

10.3.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.3.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.4.1.

10.3.4.1.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 10.3.4.1.4-1 shows the variation of the downlink SNR of the PSCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure 10.3.4.1.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 10.3.4.1.4-1: SNR and L1-RSRP variation for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

10.3.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.3.4.1.4.1-1.

Table 10.3.4.1.4.1-1: Supported test configurations for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Configuration | Description |
| --- | --- |
| 10.3.4.1-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.3.4.1-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.4.1.4.1-2.

Table 10.3.4.1.4.1-2: Initial conditions for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.3.4.1.4.1-3.

2.	Message contents are defined in clause 10.3.4.1.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 10.3.4.1.4.1-3: General test parameters for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active E-UTRA PCell |  |  |  |  | Cell 1 |  |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  |
| Active PSCell |  |  |  |  | Cell 2 |  |
| RF Channel Number |  |  |  |  | 2 |  |
| DL CCA model |  |  |  |  | As specified in D.7.2.1 |  |
| UL CCA model |  |  |  |  | As specified in D.7.2.2 |  |
| Duplex mode |  |  | Config 1, 2 |  | TDD |  |
| BWchannel |  |  | Config 1, 2 | MHz | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  | Config 1, 2 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2 |  | ULBWP.1.1 |  |
| TDD configuration |  |  | Config 1, 2 |  | TDDConf.1.1 CCA |  |
| CORESET Reference Channel |  |  | Config 1, 2 |  | CR.1.1 CCA |  |
| SSB Configuration |  |  | Config 1, 2 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |  |
| DBT Window Configuration |  |  | Config 1, 2 |  | DBT.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 30 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | PRACH.2 FR1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure |  | DCI format |  |  | 1-0 |  |
| detection transmission parameters |  | Number of Control OFDM symbols |  |  | 2 |  |
|  |  | Aggregation level |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  | gp0 |  |
| gapOffset |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (38.133 [6] Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1, 2 |  |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting |  | Config 1, 2 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  | Config 1, 2 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  | 0,1 |  |
| T310 timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 0.93 |  |
| T3 |  |  |  | s | 0.52 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 0.45 |  |
| D1 |  |  |  | s | 0.41 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

10.3.4.1.4.2	Test procedure

There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell which operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. The UE transmits the reporting according to UL CCA model. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40 ms) in test 1. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS sends an RRCReconfiguration (embedded in RRCConnectionReconfiguration message) message to the UE to configure inter-frequency measurement with the corresponding gap pattern.

3.	The UE sends an RRCReconfigurationComplete (embedded in RRCConnectionReconfigurationComplete message) message.

4.	Set the parameters of NR Cell according to T1 in Table 10.3.4.1.5-1. Propagation conditions are set according to clause C.2.3. The SS shall enable DL and UL CCA model according to Table 10.3.4.1.5-1. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 10.3.4.1.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 10.3.4.1.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 10.3.4.1.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 10.3.4.1.5-1. T5 starts.

9.	If the SS:

a)	detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according to CSI reporting on PUCCH), provided that such slot is not subject to CCA failure (as determined by PCCA_UL in Table 10.3.4.1.5-1), during the period from time point A to time point B; and

b)	does not detect preamble on a beam associated with the candidate beam set q1 before time point B; and

c)	detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5), the number of successful tests is increased by one.

Otherwise, the number of failed tests is increased by one.

10.	If the iteration or random access procedure for BFD fails, the SS shall first attempt to release and add the PSCell, by disabling the CCA model and then ensuring the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5, then proceed with step 2. Otherwise, the UE is switched OFF/ON, then proceed with step 1.

11.	Repeat steps 2-10 for all subtests until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

10.3.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.3.4.1.4.3-1: Common Exception messages for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier centre of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.4-4 with Condition gapUE Table H.3.4-5 with Condition BFD Table H.3.5-4 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

Table 10.3.4.1.4.3-2: PDCCH Search Space for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.4.1.4.3-3: RLF-TimersAndConstants for EN-D FR1 C Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 10.3.4.1.4.3-4: PDCCH-Config for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 10.3.4.1.4.3-5: ControlResourceSet for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| } |  |  |  |

10.3.4.1.5	Test requirements

Tables 10.3.4.1.4.1-3 and 10.3.4.1.5-1 define the primary level settings including test tolerances for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

Table 10.3.4.1.5-1: Cell specific test parameters for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA,DL | Note 10, 12 |  |  | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  | Note 11, 12 |  |  | 1.0/1.0 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA,UL |  |  |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| LCCA_DL |  |  |  | N/A | 7 |  |  |  |
| WCCA_DL |  |  | ms | N/A | TEvaluate_CBD_SSB_CCA Note 13 |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 |  | Config 1, 2 | dB | 5+TT | -3+TT | -12+TT | -12+TT | -12+TT |
| SNR_SSB of set q1 |  | Config 1, 2 | dB | -10+TT | -10+TT | 10+TT | 10+TT | 10+TT |
| SSB_RP of set q1 |  | Config 1, 2 | dBm/SCS kHz | -105+TT | -105+TT | -85+TT | -85+TT | -85+TT |
|  |  | Config 1, 2 | dBm/15 KHz | -98+TT |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 10.3.4.1.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4A. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 13: As defined in 38.133 [6] Table 8.5A.5.2-1. |  |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1, provided that such slots are not subject to CCA failure.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1, provided that such slots are not subject to CCA failure.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 410 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 10.3.4.2	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.3.4.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 10.3.4.0.1.

10.3.4.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier, and supporting UL and SSB-based RLM and link recovery on shared channel access.

10.3.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.3.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.4.2.

10.3.4.2.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 10.3.4.2.4-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure 10.3.4.2.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 10.3.4.2.4-1: SNR and L1-RSRP variation for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

10.3.4.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.3.4.2.4.1-1.

Table 10.3.4.2.4.1-1: Supported test configurations for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Configuration | Description |
| --- | --- |
| 10.3.4.2-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.3.4.2-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.4.2.4.1-2.

Table 10.3.4.2.4.1-2: Initial conditions for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.4.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.3.4.2.4.1-3.

2.	Message contents are defined in clause 10.3.4.2.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 10.3.4.2.4.1-3: General test parameters for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active E-UTRA PCell |  |  |  |  | Cell 1 |  |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  |
| Active PSCell |  |  |  |  | Cell 2 |  |
| RF Channel Number |  |  |  |  | 2 |  |
| DL CCA model |  |  |  |  | As specified in D.7.2.1 |  |
| UL CCA model |  |  |  |  | As specified in D.7.2.2 |  |
| Duplex mode |  |  | Config 1, 2 |  | TDD |  |
| BWchannel |  |  | Config 1, 2 | MHz | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  | Config 1, 2 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2 |  | ULBWP.1.1 |  |
| TDD configuration |  |  | Config 1, 2 |  | TDDConf.1.1 CCA |  |
| CORESET Reference Channel |  |  | Config 1, 2 |  | CR.1.1 CCA |  |
| SSB Configuration |  |  | Config 1, 2 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |  |
| DBT Window Configuration |  |  | Config 1, 2 |  | DBT.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 30 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | PRACH.2 FR1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure |  | DCI format |  |  | 1-0 |  |
| detection transmission parameters |  | Number of Control OFDM symbols |  |  | 2 |  |
|  |  | Aggregation level |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 | Table A.5-1 |
| Gap pattern ID |  |  |  |  | N.A. |  |
| gapOffset |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (38.133 [6] Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1, 2 |  |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting |  | Config 1, 2 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  | Config 1, 2 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  | 0,1 |  |
| T310 timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 9.01 |  |
| T3 |  |  |  | s | 5.16 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 3.89 |  |
| D1 |  |  |  | s | 3.85 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

10.3.4.2.4.2	Test procedure

There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell which operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. The UE transmits the reporting according to UL CCA model. In the test, DRX configuration is enabled in PSCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. No inter-frequency gap is configured on the UE. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell according to T1 in Table 10.3.4.2.5-1. Propagation conditions are set according to clause C.2.3. The SS shall enable DL and UL CCA model according to Table 10.3.4.2.5-1. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 10.3.4.2.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 10.3.4.2.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 10.3.4.2.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 10.3.4.2.5-1. T5 starts.

7.	If the SS:

a)	detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according to CSI reporting on PUCCH), provided that such slot is not subject to CCA failure (as determined by PCCA_UL in Table 10.3.4.2.5-1), during the period from time point A to time point B; and

b)	does not detect preamble on a beam associated with the candidate beam set q1 before time point B; and

c)	detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5), the number of successful tests is increased by one.

Otherwise, the number of failed tests is increased by one.

8.	If the iteration or random access procedure for BFD fails, the SS shall first attempt to release and add the PSCell, by disabling the CCA model and then ensuring the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5, then proceed with step 2. Otherwise, the UE is switched OFF/ON, then proceed with step 1.

9.	Repeat steps 2-8 for all subtests until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

10.3.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.3.4.2.4.3-1: Common Exception messages for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.4-5 with Condition BFD Table H.3.5-4 Table H.3.7-1 with Condition DRX.7 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

Table 10.3.4.2.4.3-2: PDCCH Search Space for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.4.2.4.3-3: RLF-TimersAndConstants for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 10.3.4.2.4.3-4: PDCCH-Config for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 10.3.4.2.4.3-5: ControlResourceSet for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| } |  |  |  |

10.3.4.2.5	Test requirements

Tables 10.3.4.2.4.1-3 and 10.3.4.2.5-1 define the primary level settings including test tolerances for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode .

Table 10.3.4.2.5-1: Cell specific test parameters for EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA,DL | Note 10, 12 |  |  | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  | Note 11, 12 |  |  | 1.0/1.0 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA,UL |  |  |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 |  | Config 1, 2 | dB | 5+TT | -3+TT | -12+TT | -12+TT | -12+TT |
| SNR_SSB of set q1 |  | Config 1, 2 | dB | -10+TT | -10+TT | 10+TT | 10+TT | 10+TT |
| SSB_RP of set q1 |  | Config 1, 2 | dBm/SCS kHz | -105+TT | -105+TT | -85+TT | -85+TT | -85+TT |
|  |  | Config 1, 2 | dBm/15 KHz | -98+TT |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 10.3.4.2.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4A. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1, provided that such slots are not subject to CCA failure.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1, provided that such slots are not subject to CCA failure.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 3850 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

### 10.3.5	Active BWP switching

#### 10.3.5.1	VOID

#### 10.3.5.2	DCI-based and Timer-based Active BWP Switch

##### 10.3.5.2.0	Minimum conformance requirements

10.3.5.2.0.1	Minimum conformance requirements for DCI-based and timer-based active BWP switch

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n. Where,

-	Y=0, if the serving cell where UE receives DCI for BWP switch request is same as the serving cell on which BWP switch occurs.

-	Y equals to the length of 1 slot, if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. In this scenario, TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change. If both scheduling cell and scheduled cell are in FR2-2, Y shall follow the SCS of 120 KHz.

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n except DCI triggering BWP switch on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this clause when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where slot n is the first slot of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [13] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay after bwp-InactivityTimer [13] expires on the cell where timer-based BWP switch occurs.

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 10.3.5.2.0.1-1.

Table 10.3.5.2.0.1-1: BWP switch delay

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| 5 | 0.03125 | 20 | 65 |
| 6 | 0.015625 | 39 | 129 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Provided the UE does not have the required TCI-state information to receive PDCCH and PDSCH in the new BWP, the UE shall use old TCI-states before the BWP switch until a new MAC CE updating the required TCI-state information for PDCCH and PDSCH is received after the BWP switch.

If UE has the information on the required TCI-state information to receive PDCCH and PDSCH in the new BWP,

-	UE shall be able to receive PDCCH and PDSCH with old TCI-states before the delay as specified in 38.133 [6] Clause 8.10 in the new BWP.

-	UE shall be able to receive PDCCH and PDSCH with new TCI-states after the delay as specified in 38.133 [6] Clause 8.10 in the new BWP.

Provided the UE does not have the required activated TCI-state(s) information to receive PDCCH/ PDSCH and to transmit PUSCH/PUCCH/SRS in the new BWP, the UE shall use old TCI-state(s) before the BWP switch until a new MAC CE updating the required activated TCI-state(s) information is received after the BWP switch.

If the BWP switch is triggered within or outside DRX active time, and one of the two BWPs in a BWP switching is a dormant BWP [TS 38.321, 12], UE shall be able to complete active BWP switching within the time duration of

-	TdormantBWPswitchDelay =TBWPswitchDelay+ X, provided that the dormancy indication is received in any of the first 3 OFDM symbols of a slot in the serving cell where DCI for dormancy indication is receiveds, or

-	TdormantBWPswitchDelay =TBWPswitchDelay + X + Z, provided that the dormancy indication is received after the first 3 OFDM symbols of a slot in the serving cell where DCI for dormancy indication is received, where

-	TBWPswitchDelay is defined in Table 10.3.5.2.0.1-1 corresponding to the smallest value among the SCS of the serving cell where UE receives dormancy indication and the SCSs of the dormant BWP and the active BWP immediately before or after switching the BWP of the serving cell where BWP switching occurs;

-	X equals to the length of 1 slot corresponding to the smallest value among the SCS of the serving cell where UE receives dormancy indication and the SCSs of the dormant BWP and the active BWP immediately before or after switching the BWP of the serving cell where BWP switching occurs. If both scheduling cell and scheduled cell are in FR2-2, X shall follow the SCS of 120 KHz.

-	Z equals to the length of 1  slot corresponding to the SCS of the serving cell where UE receives dormancy indication.

When either of the DCI-based, timer-based or RRC-based downlink BWP switch and/or uplink BWP switch occur on multiple CCs simultaneously or over partially overlapping period, the interruption requirements described in this clause apply for each BWP switch.

When UE receives a DCI indicating UE to switch its active BWP involving changes in any of the parameters listed in Table 10.3.5.2.0.1-3, the UE is allowed to cause interruption of up to X slot to other active serving cells if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 10.3.5.2.0.1-3 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 10.3.5.2.0.1-2. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in 38.133 [6] clause 8.6.2 when BWP switch occurs on a single CC. The starting time of interruption caused by each BWP switch is only allowed within the BWP switch delay TMultipleBWPswitchDelay +Y as defined in 38.133 [6] clause 8.6.2A.1 when BWP switch occurs on multiple CCs. Interruptions are not allowed during BWP switch involving any other parameter change.

When a BWP timer bwp-InactivityTimer defined in TS 38.331 [13] expires, UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 10.3.5.2.0.1-3 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 10.3.5.2.0.1-3 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 10.3.5.2.0.1-2. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in 38.133 [6] clause 8.6.2 when BWP switch occurs on a single CC. The starting time of interruption caused by each BWP switch is only allowed within the BWP switch delay TMultipleBWPswitchDelay as defined in 38.133 [6] clause 8.6.2B.1 when BWP switch occurs on multiple CCs simultaneously or TMultipleBWPswitchDelayTotal as defined in 38.133 [6] clause 8.6.2B.2 when BWP switch occurs on multiple CCs over partially overlapping time period. Interruptions are not allowed during BWP switch involving any other parameter change.

When UE receives an RRC reconfiguration that only requests UE to switch its active BWP on one single CC, the UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 10.3.5.2.0.1-3 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 10.3.5.2.0.1-3 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 10.3.5.2.0.1-2. The interruption is only allowed within the delay TRRCprocessingDelay + TBWPswitchDelayRRC defined in 38.133 [6] clause 8.6.3 when BWP switch occurs on a single CC. The interruption is only allowed within the delay TRRCprocessingDelay + TBWPswitchDelayRRC + DRRC*(N-1) as defined in 38.133 [6] clause 8.6.3A when BWP switch occurs on multiple CCs.

When UL BWP switch is triggered by consistent uplink CCA failures [12], the UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active UL BWP involving changes in any of the parameters listed in Table 10.3.5.2.0.1-3 if the UE is not capable of per-FR gap, or if the UL BWP switching involves SCS changing. When the UL BWP switch imposes changes in any of the parameters listed in Table 10.3.5.2.0.1-3 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing UL BWP switching. X is defined in Table 10.3.5.2.0.1-2. The starting time of interruption is only allowed within the UL BWP switching delay TBWPswitchDelay as defined in 38.133 [6] clause 8.6.2. Interruptions are not allowed during UL BWP switch involving other parameter change.

Table 10.3.5.2.0.1-2: interruption length X

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: void |  |  |

Table 10.3.5.2.0.1-3: Parameters which cause interruption other than SCS

| Parameters | Comment |
| --- | --- |
| locationAndBandwidth | From TS 38.331 [13] |
| nrofSRS-Ports |  |
| maxMIMO-Layers-r16 |  |

DCI-based or timer-based downlink BWP and/or uplink BWP switching due to change in any of the parameters listed in Table 8.2.1.2.7-2 of TS 38.133 [6] or SCS in NR PSCell or in any NR SCell may cause an interruption on PCell or on activated SCell(s) in the MCG. Interruptions are not allowed during BWP switch involving other parameter change.

Uplink BWP switching on a NR PSCell triggered by consistent uplink LBT failures on the NR PSCell may cause an interruption on PCell or on activated SCell(s) in the MCG.

The starting time of interruption due to DCI-based or timer-based downlink BWP and/or uplink BWP switching or due to uplink BWP switching on a NR PSCell triggered by consistent uplink LBT failures on the NR PSCell is only allowed within the BWP switching delay TBWPswitchDelay as defined in clause 8.6.2 of TS 38.133 [6].

The normative reference for this requirement is TS 38.133 [6] clauses 8.6.2, 8.2.1.2.7 and TS 36.133 [23] clause 7.32.2.7.

##### 10.3.5.2.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Test procedure may need to be updated

- Statistical analysis to determine test case verdict is FFS

10.3.5.2.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6 with NR PSCell under CCA, and interruption requirement for E-UTRA victim cell defined in TS 36.133 [23] clause 7.32.2.7.

10.3.5.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 onwards, supporting EN-DC, RRM measurements and UL with a shared spectrum NR carrier, BWP adaptation of at least 2BWPs, DCI and timer-based active BWP switching delay Type1 or Type2 on shared channel access.

10.3.5.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.3.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.5.2.1.

10.3.5.2.1.4	Test description

Two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and PSCell (Cell 2) with CCA transmitting SSBs in DBT windows according to DL CCA model: PSCell (Cell 2). The test parameters for the two cells are given in Table 10.3.5.2.1.4.1-1 and 10.3.5.2.1.4.1-3 below.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

10.3.5.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.3.5.2.1.4.1-1.

Table 10.3.5.2.1.4.1-1: Supported test configurations for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Config | Description |
| --- | --- |
| 10.3.5.2.1-1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.3.5.2.1-2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: A UE which fulfils the requirements in test case 10.3.5.2.2 can skip the test cases in 10.3.5.2.1. Note 3: The UE supporting EN-DC with only NR band(s) with shared spectrum access is required to be tested. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.5.2.1.4.1-2.

Table 10.3.5.2.1.4.1-2: Initial conditions for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.5.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 10.3.5.2.1.4.1-3.

2.	Message contents are defined in clause 10.3.5.2.1.4.3.

3.	The test scenario comprises of one E-UTRA PCell (Cell 1), and one NR PSCell (Cell 2) with CCA transmitting SSBs in DBT windows according to DL CCA model. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is configured according to clause C.1.2 and C.1.3.

4. By step 4 of the test procedure:

-	UE is connected to Cell 1 (E-UTRA PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PSCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PSCell.

-	UE is configured with a bwp-InactivityTimer timer value for PSCell.

Table 10.3.5.2.1.4.1-3: General test parameters for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| DL CCA model |  | As specified in clause D.7.2.1 |  |
| UL CCA model |  | As specified in clause D.7.2.2 |  |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

10.3.5.2.1.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

PDCCHs indicating new transmissions shall be sent continuously on E-UTRA PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

PDCCHs indicating new transmissions shall be sent continuously on PSCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

All cells have constant signal levels throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG_and_SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 10.3.5.2.1.4.1-3 and 10.3.5.2.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	The SS shall transmit an RRCConnectionReconfiguration message releasing the dedicated configuration of the initialDownlinkBWP and the initialUplinkBWP. This message also configures 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, which always include the bandwidth of the initial DL BWP and SSB. The SS indicates BWP-1 as the active DL BWP using firstActiveDownlinkBWP-Id, according to Table 10.3.5.2.1.4.3-2. UE is configured with a bwp-InactivityTimer timer value for PSCell.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.3.5.2.1.5-1.

5.	The SS shall send a DCI format 1_1 command for PSCell DL BWP switch.

6.	The UE shall receive the DCI format 1_1 command in PSCell's slot # denoted i, then T1 starts and the UE switch its bandwidth part from BWP-1 to BWP-2:

a)	If the UE starts to report valid ACK/NACK for PSCell from the first UL slot that occurs after the beginning of the DL slot (i+TBWPswitchDelay+k1); and

b)	If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of the DL slot (i+TBWPswitchDelay+ 1 subframe + k); and

c)	If the number of consecutive missing ACK/NACK for PCell is no more than 1.

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1.

7.	If the UE sends valid ACK/NACK for the PSCell on BWP-2, T2 starts. During T2, the SS shall not transmit DCI format for PDSCH reception on PSCell.

8.	T3 starts from the first slot #j of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires and the SS restarts to send DCI format for PDSCH reception on PSCell. Then, the UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part - BWP-1 on PSCell:

a)	If the UE starts to report valid ACK/NACK for PSCell from the first UL slot that occurs after the beginning of the DL slot (j+TBWPswitchDelay+k1); and

b)	If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of the DL slot (j+TBWPswitchDelay+ 1 subframe + k); and

c)	If the number of consecutive missing ACK/NACK for PCell is no more than 1

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

9.	Repeat steps 5-8 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

The SS verifies the DL BWP switch time in PSCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The SS verifies that potential interruption to E-UTRA PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of PSCell, respectively.

"k" is the length (slot) between E-UTRA PCell PDSCH and its corresponding ACK/NACK as specified in TS 36.213 [33].

If all subtests pass, the test passes. If one subtest fails, the test fails.

10.3.5.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.3.5.2.1.4.3-1: Common Exception messages for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

Table 10.3.5.2.1.4.3-1A: RRCReconfiguration (Step3) for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition EN-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | Table 10.3.5.2.1.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.1.4.3-1B: CellGroupConfig (Table 10.3.5.2.1.4.3-1A) for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | 1 |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex of NR PSCell |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 10.3.5.2.1.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.1.4.3-2: ServingCellConfig (Table 10.3.5.2.1.4.3-1B) for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP1 | entry 1 Table 10.3.5.2.1.4.3-3 |  |
| BWP-Downlink[2] | BWP-Downlink with condition BWP2 | entry 2 Table 10.3.5.2.1.4.3-3 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | According to BWP-1 |  |
| bwp-InactivityTimer | ms200 |  |  |
| defaultDownlinkBWP-Id | 1 | According to BWP-1 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| srs-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP1 | entry 1 Table 10.3.5.2.1.4.3-4 |  |
| BWP-Uplink[2] | BWP-Uplink with condition BWP2 | entry 2 Table 10.3.5.2.1.4.3-4 |  |
| firstActiveUplinkBWP-Id | 1 | According to BWP-1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.1.4.3-3: BWP-Downlink (Table 10.3.5.2.1.4.3-2) for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.3 |  | BWP2 |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon | Table 10.3.5.2.1.4.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.1.4.3-4: BWP-Uplink (Table 10.3.5.2.1.4.3-2) for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.1 | BWP-1 | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.3 | BWP-2 | BWP2 |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.1.4.3-5: PDSCH-TimeDomainResourceAllocationList (Table 10.3.5.2.1.4.3-6) for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 4 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 |  |
| k0 | TBWPswitchDelay | Defined in Table 10.3.5.2.0.1-1 | The DCI indicating BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[4] SEQUENCE { |  | entry 4 |  |
| k0 | 1 |  | First DCI right after DCI-based BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.1.4.3-6: PDSCH-ConfigCommon (Table 10.3.5.2.1.4.3-3) for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-101 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList | Table 10.3.5.2.1.4.3-5 |  |
| } |  |  |  |

10.3.5.2.1.5	Test requirements

Tables 10.3.5.2.1.4.1-3 and 10.3.5.2.1.5-1 define the primary level settings including test tolerances for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA.

Table 10.3.5.2.1.5-1: NR Cell specific test parameters for EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Parameter |  |  | Unit | Cell 2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1,2 |  | TDD |
| TDD configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |
| BWchannel |  | Config 1,2 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  | Config 1,2 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  | Config 1,2 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  | Config 1,2 |  | DLBWP.1.3 Note 4 |
| Initial UL BWP Configuration |  | Config 1,2 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  | Config 1,2 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  | Config 1,2 |  | ULBWP.1.3 Note 4 |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 CCA |
| RMSI CORESET parameters |  | Config 1,2 |  | CR.1.1 CCA |
| Dedicated CORESET parameters |  | Config 1,2 |  | CCR.1.1 CCA |
| OCNG Patterns |  | Config 1,2 |  | OP.1 |
| SSB Configuration | Semi- static channel acces | Config 1,2 |  | SSB.1 CCA |
|  | Dymamic channel acces | Config 1,2 |  | SSB.2 CCA |
| SMTC Configuration |  | Config 1,2 |  | SMTC.1 |
| TRS Configuration |  | Config 1,2 |  | TRS.1.2 TDD |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1,2 |  | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  | Config 1,2 |  | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  | Config 1,2 |  | 1 |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1,2 |  | 1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1,2 | dBm/SCS | -101+TT |
| SS-RSRP Note 3 |  | Config 1,2 | dBm/SCS | -84+TT |
| Ês/Iot |  | Config 1,2 | dB | 17+TT |
| Ês/Noc |  | Config 1,2 | dB | 17+TT |
| IoNote3 |  | Config 1,2 | dBm/38.16MHz | -59+TT |
| Propagation Condition |  |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause D.7.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |

During T1, the UE shall start to send the ACK for PSCell in the DL slot right after DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK for PSCell in the DL slot right after DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in 38.214 [9].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in TS 38.133 [6] Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1, the start time of PCell interruption during PSCell active BWP switch shall not happen outside the BWP switch delay.

During T3, the start time of PCell interruption of during PSCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in TS 36.133 [23] Clause 7.32.2.7.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK in the DL slot right after DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK.

##### 10.3.5.2.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Test procedure may need to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements is still FFS in 38.133 [6]

10.3.5.2.2.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6 with NR PSCell and SCell under CCA, and interruption requirements for NR victim cell defined in TS 38.133 [6] clause 8.2.1.2.7 and interruption requirement for E-UTRA victim cell defined in clause 7.32.2.7 of TS 36.133 [23].

10.3.5.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 onwards, supporting EN-DC, RRM measurements and UL with a shared spectrum NR carrier, BWP adaptation of at least 2BWPs, DCI and timer-based active BWP switching delay Type1 or Type2 and 2DL CA on shared channel access.

10.3.5.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.3.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.5.2.2.

10.3.5.2.2.4	Test description

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and PSCell(Cell 2) on one NR carrier and NR SCell (Cell 3) on the other NR carrier with CCA transmitting SSBs in DBT windows according to DL CCA model.The test parameters for the three cells are given in Table 10.3.5.2.2.4.1-1 and 10.3.5.2.2.4.1-3 below.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

10.3.5.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in this clause. Supported test configurations for LTE PCell and NR PSCell are shown in Table 10.3.5.2.2.4.1-1. Test configuration for LTE PCell and NR PSCell and test configuration for NR SCell are chosen independently.

Table 10.3.5.2.2.4.1-1: Supported test configurations for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Config | Description |
| --- | --- |
| 10.3.5.2.2-1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.3.5.2.2-2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: A UE which fulfils the requirements in test case 10.3.5.2.2 can skip the test cases in 10.3.5.2.1. Note 3: NR configuration is the same for PSCell and SCells. Note 4: The UE supporting EN-DC with only NR band(s) with shared spectrum access is required to be tested. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.5.2.2.4.1-2.

Table 10.3.5.2.2.4.1-2: Initial conditions for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.5.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 10.3.5.2.2.4.1-3.

2.	Message contents are defined in clause 10.3.5.2.2.4.3.

3.	There are one E-UTRAN carrier and two NR carriers and three cells specified in the test. Cell 1 is the PCell on E-UTRAN carrier, Cell 2 is the PSCell on one NR carrier and Cell 3 is the NR SCell on the other NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Annex A.6B. Cell 2 and Cell 3 are configured according to clauses C.1.2 and C.1.3.

4. By step 4 of the test procedure:

-	UE is connected to Cell 1 (E-UTRA PCell) on radio channel 1 (PCC), Cell 2 (PSCell) on radio channel 2 (PSCC) and Cell 3 (SCell) on radio channel 3 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for SCell, BWP-1 and BWP-2, in Cell 3 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for PSCell, BWP-0 in Cell 2 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PSCell.

-	UE is configured with a bwp-InactivityTimer timer value for SCell.

Table 10.3.5.2.2.4.1-3: General test parameters for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2, 3 | Two NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| Active SCell |  | Cell 3 | SCell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DL CCA model |  | As specified in clause D.7.2.1 |  |
| UL CCA model |  | As specified in clause D.7.2.2 |  |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| Cell3 timing offset to cell2 | s | 3 | Synchronous cells |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

10.3.5.2.2.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

PDCCHs indicating new transmissions shall be sent continuously on E-UTRA PCell (Cell 1) and PSCell (Cell 2) to ensure that the UE will have ACK/NACK sending.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 3) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 3 and the time duration of T2.

All cells have constant signal levels throughout the test:

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG_and_SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall configure SCell (Cell 3) on the SCC as per TS 38.508-1 [14] clause 7.5.1. Set the parameters according to Tables 10.3.5.2.2.4.1-3 and 10.3.5.2.2.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	The SS shall transmit an RRCConnectionReconfiguration message releasing the dedicated configuration of the initialDownlinkBWP and the initialUplinkBWP of Cell 3. This message also configures 2 different UE-specific bandwidth parts for Cell 3, BWP-1 and BWP-2,which always include the bandwidth of the initial DL BWP and SSB. The SS indicates BWP-1 as the active DL BWP using firstActiveDownlinkBWP-Id in SCell, and BWP-0 as the active DL BWP using firstActiveDownlinkBWP-Id in PSCell, according to Table 10.3.5.2.2.4.3-2. UE is configured with a bwp-InactivityTimer timer value for PSCell.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.3.5.2.2.5-1.

5.	The SS shall send a DCI format 1_1 command for SCell DL BWP switch.

6.	The UE shall receive the DCI format 1_1 command in SCell's slot # denoted i, then T1 starts and the UE switch its bandwidth part from BWP-1 to BWP-2:

a)	If the UE starts to report valid ACK/NACK for SCell on PSCell from the first UL slot that occurs after the beginning of the DL slot (i+TBWPswitchDelay+k1); and

b)	If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of the DL slot (i+TBWPswitchDelay+ 1 subframe + k); and

c)	If the UE starts to report valid ACK/NACK for PSCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+ Interruption length + k1); and

d)	If the number of consecutive missing ACK/NACK for PCell is no more than 1; and

e)	If the number of consecutive missing ACK/NACK for PSCell is no more than 1.

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1.

7.	If the UE sends valid ACK/NACK for the SCell on PSCell on BWP-2, T2 starts. During T2, the SS shall not transmit DCI format for PDSCH reception on SCell.

8.	T3 starts from the first slot #j of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires and the SS restarts to send DCI format for PDSCH reception on SCell. Then, the UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part, BWP-1, on SCell:

a)	If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of the DL slot (j+TBWPswitchDelay+ 1 subframe + k); and

b)	If the UE starts to report valid ACK/NACK for SCell on PSCell from the first UL slot that occurs after the beginning of the DL slot (j+TBWPswitchDelay+k1); and

c)	If the UE starts to report valid ACK/NACK for PSCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+ Interruption length + k1); and

d)	If the number of consecutive missing ACK/NACK for PCell is no more than 1; and

e)	If the number of consecutive missing ACK/NACK for PSCell is no more than 1.Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

9.	Repeat steps 5-8 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

The SS verifies the DL BWP switch time in SCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The SS verifies that potential interruption to E-UTRA PCell and NR PSCell is carried out in the correct time span by monitoring ACK/NACK sent in E-UTRA PCell and PSCell during BWP switch of SCell, respectively.

Interruption length is defined in TS 38.133 [6] Table 8.2.1.2.7-1.

k is the length (slot) between E-UTRA PCell PDSCH and its corresponding ACK/NACK as specified in TS 36.213 [33].

k1 is the timing between NR DL data receiving and acknowledgement as specified in TS 38.214 [9].

If all subtests pass, the test passes. If one subtest fails, the test fails.

10.3.5.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.3.5.2.2.4.3-1: Common Exception messages for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access. |

Table 10.3.5.2.2.4.3-1A: RRCReconfiguration (Step 3) for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition EN-DC_SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | Table 10.3.5.2.2.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.2.4.3-1B: CellGroupConfig (Table 10.3.5.2.2.4.3-1A) for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | 1 |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig | Table 10.3.5.2.2.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.2.4.3-2: ServingCellConfig (Table 10.3.5.2.2.4.3-1B) for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP1 | entry 1 Table 10.3.5.2.2.4.3-3 |  |
| BWP-Downlink[2] | BWP-Downlink with condition BWP2 | entry 2 Table 10.3.5.2.2.4.3-3 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | According to BWP-1 |  |
| bwp-InactivityTimer | ms200 |  |  |
| defaultDownlinkBWP-Id | 1 | According to BWP-1 |  |
| } |  |  |  |

Table 10.3.5.2.2.4.3-3: BWP-Downlink (Table 10.3.5.2.2.4.3-2) for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.3 |  | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | BWP2 |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon | Table 10.3.5.2.2.4.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.2.4.3-4: PDSCH-TimeDomainResourceAllocationList (Table 10.3.5.2.2.4.3-5) for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 4 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 |  |
| k0 | TBWPswitchDelay | Defined in Table 10.3.5.2.0.1-1 | The DCI indicating BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[4] SEQUENCE { |  | entry 4 |  |
| k0 | 1 |  | First DCI right after DCI-based BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.2.2.4.3-5: PDSCH-ConfigCommon (Table 10.3.5.2.2.4.3-3) for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-101 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList | Table 10.3.5.2.2.4.3-4 |  |
| } |  |  |  |

10.3.5.2.2.5	Test requirements

Tables 10.3.5.2.2.4.1-3 and 10.3.5.2.2.5-1 define the primary level settings including test tolerances for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA.

Table 10.3.5.2.2.5-1: NR Cell specific test parameters for EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA

| Parameter |  |  | Unit | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |  |
| Duplex mode |  | Config 1,2 |  | TDD |  |
| TDD configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| BWchannel |  | Config 1,2 |  | 40 MHz: NRB,c = 106 |  |
| Active BWP ID |  |  |  | 1, 2 | 0 |
| Initial BWP Configuration |  | Config 1,2 |  | DLBWP.0.2 | DLBWP.0.2 |
| Active BWP-0 Configuration |  | Config 1,2 |  | NA | DLBWP.0.2 |
| Active BWP-1 Configuration |  | Config 1,2 |  | DLBWP.1.3 | NA |
| Active BWP-2 Configuration |  | Config 1,2 |  | DLBWP.1.1 | NA |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 CCA |  |
| RMSI CORESET parameters |  | Config 1,2 |  | CR.1.1 CCA |  |
| Dedicated CORESET parameters |  | Config 1,2 |  | CCR.1.1 CCA |  |
| OCNG Patterns |  | Config 1,2 |  | OP.1 |  |
| SSB Configuration | Semi- static channel acces | Config 1,2 |  | SSB.1 CCA |  |
|  | Dymamic channel acces | Config 1,2 |  | SSB.2 CCA |  |
| SMTC Configuration |  | Config 1,2 |  | SMTC.1 |  |
| TRS Configuration |  | Config 1,2 |  | TRS.1.2 TDD |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1,2 |  | 1 | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  | Config 1,2 |  | 1 | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  | Config 1,2 |  | 1 | 1 |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1,2 |  | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 |  |
| Propagation Condition |  |  |  | AWGN |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 | dBm/SCS kHz | -101+TT | -101+TT |
| SS-RSRP Note 3 |  | Config 1,2 | dBm/SCS kHz | -84+TT | -84+TT |
| Ês/Iot |  | Config 1,2 | dB | 17+TT | 17+TT |
| Ês/Noc |  | Config 1,2 | dB | 17+TT | 17+TT |
| IoNote3 |  | Config 1,2 | dBm/38.16MHz | -59+TT | -59+TT |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause D.7.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

During T1, the UE shall start to send the ACK for PSCell in the DL slot right after slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK for PSCell in the DL slot right after slot (j+TBWPswitchDelay+k11).

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1, the start of the interruption of PCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

During T3, the start of the interruption of PCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in clause 7.32.2.7 of TS 36.133 [15].

During T1, the start of the interruption of SCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

During T3, the start of the interruption of SCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in TS 38.133 [6] clause 8.6.2.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK in the DL slot right after slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS value of k1 for type 1 and type 2 UE

#### 10.3.5.3	RRC-based Active BWP Switch

##### 10.3.5.3.0	Minimum conformance requirements

10.3.5.3.0.1	Minimum conformance requirements for RRC-based active BWP switch

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where:

DL slot n is the last slot overlapping with the PDSCH containing the RRC command, and

NR Slot length is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch if the BWP switch involves changing of SCS.

TRRCprocessingDelay is the length of the RRC procedure delay in ms as defined in clause 11.2 in TS 36.331 [29] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the length of the RRC procedure delay in ms as defined in clause 12 in TS 38.331 [13], and

TBWPswitchDelayRRC=6ms is the time used by the UE to perform BWP switch.

The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC on the cell where RRC-based BWP switch occurs. When  THARQ> TRRCprocessingDelay a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [8].

The normative reference for this requirement is TS 38.133 [6] clauses 8.6.3.

##### 10.3.5.3.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Test procedure may need to be updated

- Statistical analysis to determine test case verdict is FFS

10.3.5.3.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch under CCA defined in TS 38.133 [6] clause 8.6.3.

10.3.5.3.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 onwards supporting EN-DC, RRM measurements and UL with a shared spectrum NR carrier, BWP adaptation of at least 2BWPs on shared channel access.

10.3.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.3.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.10.3.5.3.1.

10.3.5.3.1.4	Test description

Two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and PSCell (Cell 2) with CCA transmitting SSBs in DBT windows according to DL CCA model: PSCell (Cell 2). The test parameters for the two cells are given in Table 10.3.5.3.1.4.1-1 and 10.3.5.3.1.4.1-3 below.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

10.3.5.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.3.5.3.1.4.1-1.

Table 10.3.5.3.1.4.1-1: Supported test configurations for EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Config | Description |
| --- | --- |
| 10.3.5.3.1-1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.3.5.3.1-2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE supporting EN-DC with only NR band(s) with shared spectrum access is required to be tested. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.3.5.3.1.4.1-2.

Table 10.3.5.3.1.4.1-2: Initial conditions for EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.3.5.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 10.3.5.3.1.4.1-3.

2.	Message contents are defined in clause 10.3.5.3.1.4.3.

3.	The test scenario comprises of one E-UTRA PCell (Cell 1) and one NR PSCell (Cell 2). The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 are configured according to clause C.1.2 and C.1.3.

4. By step 4 of the test procedure:

-	UE is connected to Cell 1 (E-UTRA PCell) on radio channel 1 (PCC) and to Cell 2 (PSCell) on radio channel 2 (PSCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 2 (PSCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PSCell

Table 10.3.5.3.1.4.1-3: General test parameters for EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DL CCA model |  | As specified in clause D.7.2.1 |  |
| UL CCA model |  | As specified in clause D.7.2.2 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | 0.2 |  |

10.3.5.3.1.4.2	Test procedure

The test consists of 1 time period, with duration of T1. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

PDCCHs indicating new transmissions shall be sent continuously on E-UTRA PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

PDCCHs indicating new transmissions shall be sent continuously on PSCell (Cell 2) to ensure that the UE will have ACK/NACK sending.

All cells have constant signal levels throughout the test:

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG_and_SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 10.3.5.3.1.4.1-3 and 10.3.5.3.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	The SS shall send an RRCConnectionReconfiguration message releasing the dedicated configuration of the initialDownlinkBWP and the initialUplinkBWP. This message also configures another UE-specific bandwidth part, BWP-1, and indicates BWP-1 as the active DL BWP using firstActiveDownlinkBWP-Id, according to the initial condition of Active BWP-1 in Table 10.3.5.3.1.5-1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.3.5.3.1.5-1.

5.	The SS shall send an RRCConnectionReconfiguration message with updated bandwidth part configuration for PSCell DL BWP switch, changing the BWP according to the final condition of Active BWP-1 in Table 10.3.5.3.1.5-1. T1 starts.

6.	The UE shall receive the RRCConnectionReconfiguration in PSCell's slot # denoted i and reconfigure its bandwidth part with the updated bandwidth part configuration:

7	If the UE starts to report valid ACK/NACK for PSCell from the first UL slot that occurs after the beginning of DL slot i+X+k1 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one. Where,

-	X = 52 for test configuration 10.3.5.3.1-1 and 10.3.5.3.1-2;

8.	After the SS receives the ACK/NACK in step 5) or when T1 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1). If either of the reconfiguration in step 6 or step 7 fails, switch off and on the UE and go to step 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

The SS verifies the DL BWP switch time in PSCell by counting the slots from the time when the RRCConnectionReconfiguration message including updated BWP configuration is sent till the time when a valid ACK/NACK is received.

10.3.5.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.3.5.3.1.4.3-1: Common Exception messages for EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

Table 10.3.5.3.1.4.3-1A: RRCReconfiguration (Step 3, Step 5) EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition EN-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | Table 10.3.5.3.1.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.3.1.4.3-1B: CellGroupConfig (Table 10.3.5.3.1.4.3-1A) EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | 1 |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex of NR PSCell |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 10.3.5.3.1.4.3-1C |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.3.1.4.3-1C: ServingCellConfig (Table 10.3.5.3.1.4.3-1B) EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] SEQUENCE { | BWP-Downlink | entry 1 Table 10.3.5.3.1.4.3-1D |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWP-1 |  |
| defaultDownlinkBWP-Id | 1 | BWP-1 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| srs-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 10.3.5.3.1.4.3-1E |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 |  |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.3.1.4.3-1D: BWP-Downlink (Table 10.3.5.3.1.4.3-1C) EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.3 |  | Step 3 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | Step 5 |
| } |  |  |  |
| } |  |  |  |

Table 10.3.5.3.1.4.3-1E: BWP-Uplink (Table 10.3.5.3.1.4.3-1C) EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.3 |  | Step 3 |
|  | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.1 |  | Step 5 |
| } |  |  |  |
| } |  |  |  |

10.3.5.3.1.5	Test requirements

Tables 10.3.5.3.1.4.1-3 and 10.3.5.3.1.5-1 define the primary level settings including test tolerances for EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA.

Table 10.3.5.3.1.5-1: NR Cell specific test parameters for EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA

| Parameter |  |  |  | Unit | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1,2 |  | TDD |
| TDD configuration |  |  | Config 1,2 |  | TDDConf.1.1 CCA |
| BWchannel |  |  | Config 1,2 |  | 40 MHz: NRB,c = 106 |
| Active DL BWP ID |  |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | Config 1,2 |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  |  | Config 1,2 |  | ULBWP.0.2 |
| Initial Condition |  | Active DL BWP-1 Configuration | Config 1,2 |  | DLBWP.1.3 |
| Final Condition |  | Active DL BWP-1 Configuration | Config 1,2 |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  |  | Config 1,2 |  | ULBWP.0.2 |
| Active UL BWP-1 Configuration |  |  | Config 1,2 |  | ULBWP.1.3 |
| Active UL BWP-2 Configuration |  |  | Config 1,2 |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  |  | Config 1,2 |  | SR.1.1 CCA |
| RMSI CORESET parameters |  |  | Config 1,2 |  | CR.1.1 CCA |
| Dedicated CORESET parameters |  |  | Config 1,2 |  | CCR.1.1 CCA |
| OCNG Patterns |  |  | Config 1,2 |  | OP.1 |
| SSB Configuration | Semi- static channel acces |  | Config 1,2 |  | SSB.1 CCA |
|  | Dymamic channel acces |  | Config 1,2 |  | SSB.2 CCA |
| SMTC Configuration |  |  | Config 1,2 |  | SMTC.1 |
| TRS Configuration |  |  | Config 1,2 |  | TRS.1.2 TDD |
| DL CCA probability for semi-static channel access (PCCA_DL) |  |  | Config 1,2 |  | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  |  | Config 1,2 |  | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  |  | Config 1,2 |  | 1 |
| DL CCA probability for semi-static channel access (PCCA_DL) |  |  | Config 1,2 |  | 1 |
| Antenna Configuration |  |  |  |  | 1x2 |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  |  | Config 1,2 | dBm/SCS kHz | -101+TT |
| SS-RSRP Note 3 |  |  | Config 1,2 | dBm/SCS kHz | -84+TT |
| Ês/Iot |  |  | Config 1,2 | dB | 17+TT |
| Ês/Noc |  |  | Config 1,2 | dB | 17+TT |
| IoNote3 |  |  | Config 1,2 | dBm/38.16MHz | -59+TT |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause D.7.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

During T1, the UE shall be ready for the reception of uplink grant on PSCell from the first DL slot occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length, and starts to report valid ACK/NACK for the PSCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1

Where,

TRRCprocessingDelay = 20 ms, is the RRC procedure delay in ms as defined in clause 11.2 in TS 36.331 [29];

TBWPSwitchDelayRRC = 6 ms, is the time used by the UE to perform BWP switch;

NR slot length = 1ms for SCS = 15kHz, and NR slot length = 0.5 ms for SCS = 30kHz.

k1 is the timing between DL data receiving and acknowledgement as specified in [12].

Which gives TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length=

26 slots, for SCS = 15kHz,

52 slots, for SCS = 30kHz.

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

## 10.4	Measurement procedures

### 10.4.1	Intra-frequency measurements

#### 10.4.1.0	Minimum conformance requirements

10.4.1.0.1	Minimum conformance requitements for intra-frequency measurements without measurement gaps

10.4.1.0.1.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index_CCA if UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_CCA. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_CCA.

Tidentify_intra_without_index_CCA = (TPSS/SSS_sync_intra_CCA + T SSB_measurement_period_intra_CCA) ms

Tidentify_intra_with_index CCA = (TPSS/SSS_sync_intra_CCA + T SSB_measurement_period_intra_CCA + TSSB_time_index_intra_CCA) ms

Where:

TPSS/SSS_sync_intra_CCA: it is the time period used in PSS/SSS detection given in table 10.4.1.0.1.1-1, 10.4.1.0.1.1-3 (deactivated Scell) .

TSSB_time_index_intra_CCA: it is the time period used to acquire the index of the SSB being measured given in table 10.4.1.0.1.1-2 or  10.4.1.0.1.1-4 (deactivated SCell).

T SSB_measurement_period_intra_CCA: equal to a measurement period of SSB based measurement given in table 10.4.1.0.1.2-1, 10.4.1.0.1.2-2 (deactivated Scell).	CSSFintra: it is a carrier specific scaling factor and is determined

-	according to CSSFoutside_gap,i in 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps,  or according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

-	Mpss/sss_sync_w/o_gaps_CCA: For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps CCA = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps CCA = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps CCA = 36.

-	Mmeas_period_w/o_gaps_CCA: For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps CCA = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps CCA = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps CCA = 36.

-	MSSB_index_intra_CCA: For a UE supporting FR2-2 power class 1, MSSB_index_intra CCA = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra CCA = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra CCA = 48 samples.

N: is the UE Rx beam sweeping scaling factor. N = 12.

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp=1

When intra-frequency SMTC is partially overlapping with measurent gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP.

For FR2-2,

Klayer1_measurement=TBD

If MCG DRX is in use, intra-frequency cell identification requirements specified in Table 10.4.1.0.1.1-1, Table 10.4.1.0.1.1-2, Table 10.4.1.0.1.1-3, Table 10.4.1.0.1.1-4, Table 10.4.1.0.1.1-5 and Table 10.4.1.0.1.1-6 shall depend on the MCG DRX cycle. If SCG DRX is in use, intra-frequency cell identification requirements specified in Table 10.4.1.0.1.1-1, Table 10.4.1.0.1.1-2, Table 10.4.1.0.1.1-3, Table 10.4.1.0.1.1-4, Table 10.4.1.0.1.1-5 and Table 10.4.1.0.1.1-6 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

The requirements apply provided any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known defined in 38.133 [6] clause 9.2A.4.3.

Table 10.4.1.0.1.1-1: Time period for PSS/SSS detection (FR1)

| Condition | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | max( 600ms, ceil((5+LPSS/SSS) x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x (5+LPSS/SSS) x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil((5+LPSS/SSS) x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, LPSS/SSS is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. When DRX is configured, LPSS/SSS is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 3: LPSS/SSS,max =7 for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS,max =5 for 40ms<Max(DRX cycle,SMTC period)≤320ms, LPSS/SSS,max = 3 for DRX cycle>320ms. NOTE 4: Upon exceeding LPSS/SSS,max, the UE is not required to meet the requirements for PSS/SSS detection. |  |

Table 10.4.1.0.1.1-2: Time period for time index detection (FR1)

| Condition | TSSB_time_index_intra_CCA |
| --- | --- |
| No DRX | max(120ms, ceil((3+Lind) x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x (3+Lind) x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil((3+Lind) x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, Lind is the number of SMTC occasions not available at the UE during TSSB_time_index_intra_CCA for index detection, where Lind ≤ Lind,max. When DRX is configured, Lind is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for index detection, where Lind ≤ Lind,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 3: Lind,max = 5 for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, Lind,max = 3 for 40ms<Max(DRX cycle,SMTC period)≤320ms, Lind,max =2 for DRX cycle>320ms. NOTE 4: Upon exceeding Lind,max over the period of time TSSB_time_index_intra_CCA, the UE has to restart the time index detection procedure. |  |

Table 10.4.1.0.1.1-3: Time period for PSS/SSS detection, deactivated SCell (FR1)

| Condition | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | (5 + LPSS/SSS,deact) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | (5 + LPSS/SSS, deact) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle>320ms | (5 + LPSS/SSS, deact) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, LPSS/SSS, deact is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS, deact< LPSS/SSS, deact,max. When DRX is configured, LPSS/SSS, deact is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS, deact< LPSS/SSS, deact,max.When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement cycles, the UE is not required to determine the availability of SMTC occasions more frequent than once per measurement cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: LPSS/SSS, deact,max, = 7 for Max(DRX cycle, measCycleSCell)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS, deact,max = 5 for 40ms<Max(DRX cycle, measCycleSCell)≤320ms, LPSS/SSS, deact,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding LPSS/SSS, deact,max,, the UE is not required to meet the requirements for PSS/SSS detection. |  |

Table 10.4.1.0.1.1-4: Time period for time index detection, deactivated SCell (FR1)

| Condition | TSSB_time_index_intra_CCA |
| --- | --- |
| No DRX | (3+Lind,deact) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | (3+Lind,deact) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle>320ms | (3+Lind,deact) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lind,deact is the number of SMTC occasions not available at the UE during TSSB_time_index_intra_CCA for index detection, where Lind,deact < Lind,deact,max. When DRX is configured, Lind,deact is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for index detection, where Lind,deact < Lind,deact,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement cycles, the UE is not required to determine the availability of SMTC occasions more frequent than once per measurement cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: Lind,deact,max, = 5 for Max(DRX cycle, measCycleSCell)≤40ms where DRX cycle is 0 for non-DRX, Lind,deact,max = 3 for 40ms<Max(DRX cycle, measCycleSCell)≤320ms, Lind,deact,max = 2 for DRX cycle>320ms. NOTE 3: Upon exceeding Lind,deact,max over the period of time TSSB_time_index_intra_CCA, the UE has to restart the time index detection procedure. |  |

Table 10.4.1.0.1.1-5: Time period for PSS/SSS detection, (Frequency range FR2-2)

| Condition | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | max(600ms, ceil((Mpss/sss_sync_w/o_gaps_CCA x KFR + N x LPSS/SSS) x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x (Mpss/sss_sync_w/o_gaps_CCA x KFR + N x LPSS/SSS) x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil((Mpss/sss_sync_w/o_gaps_CCA x KFR + N x LPSS/SSS) x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, LPSS/SSS is the number of SMTC occasion groups not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. A SMTC occasion group consists of N consecutive SMTC occasions. An SMTC occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. When DRX is configured, LPSS/SSS is the number of [DRX cycle groups] in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. [A [DRX occasion group consists of N consecutive DRX cycles. A DRX occasion group occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB.] When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. NOTE 3: LPSS/SSS,max =[7] for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS,max =[5] for 40ms<Max(DRX cycle,SMTC period)≤320ms, LPSS/SSS,max = [3] for DRX cycle>320ms. NOTE 4: Upon exceeding LPSS/SSS,max, the UE is not required to meet the requirements for PSS/SSS detection. NOTE 5: KFR is a scaling factor depending on the frequency range and SSB SCS. KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR= 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 10.4.1.0.1.1-6: Time period for PSS/SSS detection, deactivated SCell (FR2-2)

| DRX cycle | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | Ceil((Mpss/sss_sync_w/o_gaps x KFR + N x LPSS/SSS,deact) x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil((Mpss/sss_sync_w/o_gaps x KFR + N x LPSS/SSS,deact) x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil((Mpss/sss_sync_w/o_gaps x KFR + N x LPSS/SSS,deact) x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, LPSS/SSS, deact is the number of SMTC occasions groups not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS, deact< LPSS/SSS, deact,max. A SMTC occasion group consists of N consecutive SMTC occasions. An SMTC occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. When DRX is configured, LPSS/SSS, deact is the number of [DRX cycle groups] in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS, deact< LPSS/SSS, deact,max.When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. [A DRX occasion group consists of N consecutive DRX cycles. A DRX occasion group occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB.] When configured with measurement cycles, the UE is not required to determine the availability of SMTC occasions more frequent than once per measurement cycle. NOTE 2: LPSS/SSS, deact,max, = [7] for Max(DRX cycle, measCycleSCell)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS, deact,max = [5] for 40ms<Max(DRX cycle, measCycleSCell)≤320ms, LPSS/SSS, deact,max = [3] for DRX cycle>320ms. NOTE 3: Upon exceeding LPSS/SSS, deact,max,, the UE is not required to meet the requirements for PSS/SSS detection. NOTE 4: Void NOTE 5: KFR is a scaling factor depending on the frequency range and SSB SCS. KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR= 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 10.4.1.0.1.1-7: Time period for time index detection (FR2-2)

| DRX cycle | TSSB_time_index_intra_CCA |
| --- | --- |
| No DRX | Max(200ms, ceil((MSSB_index_intra_CCA + N ´ Lind) ´ Kp) ´ SMTC period) ´ CSSFintra |
| DRX cycle≤ 320ms | Max(200ms, ceil(1.5 ´ ((MSSB_index_intra + N ´ Lind) ´ Kp) ´ Max(SMTC period, DRX cycle)) ´ CSSFintra |
| DRX cycle> 320ms | ceil((MSSB_index_intra + N ´ Lind) ´ Kp) ´ DRX cycle ´ CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, Lind is the number of SMTC occasion groups not available at the UE during TSSB_time_index_intra_CCA for index detection, where Lind ≤ Lind,max. When DRX is configured, Lind is the number of DRX cycles groups in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for index detection, where Lind ≤ Lind,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. NOTE 3: Lind,max = 5 for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, Lind,max = 3 for 40ms<Max(DRX cycle,SMTC period)≤320ms, Lind,max =2 for DRX cycle>320ms. NOTE 4: Upon exceeding Lind,max over the period of time TSSB_time_index_intra_CCA, the UE has to restart the time index detection procedure. |  |

Table 10.4.1.0.1.1-8: Time period for time index detection, deactivated SCell (FR2-2)

| DRX cycle | TSSB_time_index_intra_CCA |
| --- | --- |
| No DRX | Max(200ms, ceil((MSSB_index_intra_CCA + N ´ Lind,deact) ´ Kp) ´ SMTC period) ´ CSSFintra |
| DRX cycle≤ 320ms | Max(200ms, ceil(1.5 ´ ((MSSB_index_intra + N ´ Lind,deact) ´ Kp) ´ Max(SMTC period, DRX cycle)) ´ CSSFintra |
| DRX cycle> 320ms | ceil((MSSB_index_intra + N ´ Lind,deact) ´ Kp) ´ DRX cycle ´ CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, Lind,deact is the number of SMTC occasion groups not available at the UE during TSSB_time_index_intra_CCA for index detection, where Lind,deact ≤ Lind,deact,max. When DRX is configured, Lind,deact is the number of DRX cycles groups in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for index detection, where Lind,deact ≤ Lind,deact,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. NOTE 3: Lind,deact,max = 5 for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, Lind,deact,max = 3 for 40ms<Max(DRX cycle,SMTC period)≤320ms, Lind,deact,max =2 for DRX cycle>320ms. NOTE 4: Upon exceeding Lind,deact,max over the period of time TSSB_time_index_intra_CCA, the UE has to restart the time index detection procedure. |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.2A.5.1.

10.4.1.0.1.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 10.4.1.0.1.2-1, 10.4.1.0.1.2-2 (deactivated SCell).

If SCG DRX is in use, intra-frequency measurement period requirements specified in Table 10.4.1.0.1.2-1, Table 10.4.1.0.1.2-2 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

The requirements apply provided any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known defined in 38.133 [6] clause 9.2A.4.3.

When the time period of unsuccessful measurement attempts due to exceeding the maximum number of unavailable at the UE SMTC occasions of an already identified cell exceeds the maximum time requirement for the cell to remain known defined in 38.133 [6] clause 9.2A.4.3, UE shall stop the measurement attempts on this SSB and perform the detection procedure again like for any other SSB.

Table 10.4.1.0.1.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| Condition | T SSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | max(200ms, ceil((5+Lmeas) x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x (5+Lmeas) x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil((5+Lmeas) x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, Lmeas is the number of SMTC occasions not available at the UE during T SSB_measurement_period_intra_CCA for measurement, where Lmeas <Lmeas,max. When DRX is configured, Lmeas is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during T SSB_measurement_period_intra_CCA for measurement, where Lmeas <Lmeas,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 3: Lmeas,max = 7 for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, Lmeas,max = 5 for 40ms<Max(DRX cycle,SMTC period)≤320ms, Lmeas,max = 3 for DRX cycle>320ms. NOTE 4: Upon exceeding Lmeas,max over the period of time T SSB_measurement_period_intra_CCA, the UE has to restart the measurement procedure. |  |

Table 10.4.1.0.1.2-2: Measurement period for intra-frequency measurements without gaps, deactivated SCell (FR1)

| Condition | T SSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | (5+Lmeas,deact) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | (5+Lmeas, deact) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle>320ms | (5+Lmeas, deact) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lmeas,deact is the number of SMTC occasions not available at the UE during T SSB_measurement_period_intra_CCA for measurement, where Lmeas,deact <Lmeas, ,deact ,max. When DRX is configured, Lmeas,deact is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during T SSB_measurement_period_intra_CCA for measurement, where Lmeas,deact <Lmeas, ,deact ,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement cycles, the UE is not required to determine the availability of SMTC occasions more frequent than once per measurement cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: Lmeas, ,deact ,max, = 7 for Max(DRX cycle, measCycleSCell)≤40ms where DRX cycle is 0 for non-DRX, Lmeas, ,deact ,max = 5 for 40ms<Max(DRX cycle, measCycleSCell)≤320ms, Lmeas, ,deact ,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding Lmeas,deact,max over the period of time T SSB_measurement_period_intra_CCA, the UE has to restart the measurement procedure. |  |

Table 10.4.1.0.1.2-3: Measurement period for intra-frequency measurements without gaps (FR2-2)

| DRX cycle | T SSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | max(400ms, ceil((Mmeas_period_w/o_gaps_CCA + N x Lmeas) x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x (Mmeas_period_w/o_gaps_CCA + N x Lmeas) x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil((Mmeas_period_w/o_gaps_CCA + N x Lmeas) x Kp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, Lmeas is the number of SMTC occasion groups not available at the UE during T SSB_measurement_period_intra_CCA for measurement, where Lmeas <Lmeas,max. A SMTC occasion group consists of N consecutive SMTC occasions. An SMTC occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. When DRX is configured, Lmeas is the number of [DRX cycle groups] in which at least one SMTC occasion is not available at the UE during T SSB_measurement_period_intra_CCA for measurement, where Lmeas <Lmeas,max. . [A DRX occasion group consists of N consecutive DRX cycles. A DRX occasion group occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB.] When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. NOTE 3: Lmeas,max = [7] for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, Lmeas,max = [5] for 40ms<Max(DRX cycle,SMTC period)≤320ms, Lmeas,max = [3] for DRX cycle>320ms. NOTE 4: Upon exceeding Lmeas,max over the period of time T SSB_measurement_period_intra_CCA, the UE has to restart the measurement procedure. |  |

Table 10.4.1.0.1.2-4: Measurement period for intra-frequency measurements without gaps, deactivated Scell (FR2-2)

| DRX cycle | T SSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | Ceil((Mmeas_period_w/o_gaps_CCA + N x Lmeas,deact) x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil((Mmeas_period_w/o_gaps_CCA + N x Lmeas,deact) x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil((Mmeas_period_w/o_gaps_CCA + N x Lmeas,deact) x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lmeas,deact is the number of SMTC occasion groups not available at the UE during T SSB_measurement_period_intra_CCA for measurement, where Lmeas,deact <Lmeas, ,deact ,max. A SMTC occasion group consists of N consecutive SMTC occasions. An SMTC occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. When DRX is configured, Lmeas,deact is the number of [DRX cycle groups] in which at least one SMTC occasion is not available at the UE during T SSB_measurement_period_intra_CCA for measurement, where Lmeas,deact <Lmeas, ,deact ,max. [A DRX occasion group consists of N consecutive DRX cycles. A DRX occasion group occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB.] When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement cycles, the UE is not required to determine the availability of SMTC occasions more frequent than once per measurement cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: Lmeas, ,deact ,max, = [7] for Max(DRX cycle, measCycleSCell)≤40ms where DRX cycle is 0 for non-DRX, Lmeas, ,deact ,max = [5] for 40ms<Max(DRX cycle, measCycleSCell)≤320ms, Lmeas, ,deact ,max = [3] for DRX cycle>320ms. NOTE 3: Upon exceeding Lmeas,deact,max over the period of time T SSB_measurement_period_intra_CCA, the UE has to restart the measurement procedure. |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.2A.5.2.

10.4.1.0.2	Minimum conformance requitements for intra-frequency measurements with measurement gaps

10.4.1.0.2.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index_CCA if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_CCA. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index CCA.

Tidentify_intra_without_index CCA = TPSS/SSS_sync_intra_CCA + T SSB_measurement_period_intra_CCA  ms

Tidentify_intra_with_index_CCA = TPSS/SSS_sync_intra_CCA + T SSB_measurement_period_intra_CCA + TSSB_time_index_intra_CCA

Where:

TPSS/SSS_sync_intra_CCA: it is the time period used in PSS/SSS detection given in table 10.4.1.0.2.1-1.

TSSB_time_index_intra_CCA: it is the time period used to acquire the index of the SSB being measured given in table 10.4.1.0.2.1-2.

T SSB_measurement_period_intra_CCA: equal to a measurement period of SSB based measurement given in table 10.4.1.0.2.2-1 or 10.4.1.0.2.1-3.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps.

Mpss/sss_sync_w/o_gaps_CCA: For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps CCA = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps CCA = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps CCA = 36.

Mmeas_period_w/o_gaps_CCA: For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps CCA = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps CCA = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps CCA = 36.

-	MSSB_index_intra_CCA: For a UE supporting FR2-2 power class 1, MSSB_index_intra CCA = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra CCA = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra CCA = 48 samples.

N: is the UE Rx beam sweeping scaling factor. N = 12.

If MCG DRX is in use, intra-frequency cell identification requirements specified in Table 10.4.1.0.2.1-1, Table 10.4.1.0.2.1-2 and Table 10.4.1.0.2.1-3 shall depend on the MCG DRX cycle. If SCG DRX is in use, intra-frequency cell identification requirements specified in Table 10.4.1.0.2.1-1, Table 10.4.1.0.2.1-2 and Table 10.4.1.0.2.1-3 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

The requirements apply provided any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known defined in 38.133 [6] clause 9.2A.4.3.

Table 10.4.1.0.2.1-1: Time period for PSS/SSS detection (FR1)

| Condition | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | max(600ms, (5+LPSS/SSS,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5x (5+LPSS/SSS,gaps)) x max(DRX cycle, MGRP, SMTC period)) x CSSFintra |
| DRX cycle>320ms | (5+LPSS/SSS,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, LPSS/SSS,gaps is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS,gaps <LPSS/SSS,gaps,max. When DRX is configured, LPSS/SSS,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS,gaps <LPSS/SSS,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: LPSS/SSS,gaps,max, =7 for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS,gaps,max =5 for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, LPSS/SSS,gaps,max =3 for DRX cycle>320ms. NOTE 3: Upon exceeding LPSS/SSS,gaps,max, the UE is not required to meet the requirements for PSS/SSS detection. |  |

Table 10.4.1.0.2.1-2: Time period for time index detection (FR1)

| Condition | TSSB_time_index_intra_CCA |
| --- | --- |
| No DRX | max(120ms, (3+Lind,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5x (3+Lind,gaps)) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | (3+Lind,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lind,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_intra_CCA for index detection where Lind,gaps < Lind,gaps,max. When DRX is configured, Lind,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for index detection where Lind,gaps < Lind,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: Lind,gaps,max, = 5 for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lind,gaps,max = 3 for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lind,gaps,max = 2 for DRX cycle>320ms. NOTE 3: Upon exceeding Lind,gaps,max over the TSSB_time_index_intra_CCA period of time, the UE has to restart the time index detection procedure. |  |

Table 10.4.1.0.2.1-3: Time period for PSS/SSS detection (FR2-2)

| DRX cycle | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | max(600ms, (Mpss/sss_sync_with_gaps_CCA x KFR + N x Lind,gaps ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5x (Mpss/sss_sync_with_gaps_CCA x KFR + N x Lind,gaps )) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | (Mpss/sss_sync_with_gaps_CCA x KFR + N x Lind,gaps ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lind,gaps is the number of SMTC occasion groups not available at the UE during TSSB_time_index_intra_CCA for index detection where Lind,gaps < Lind,gaps,max. A SMTC occasion group consists of N consecutive SMTC occasions. An SMTC occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. When DRX is configured, Lind,gaps is the number of [DRX cycle groups] in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for index detection where Lind,gaps < Lind,gaps,max.[A DRX occasion group consists of N consecutive DRX cycles. A DRX occasion group occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB.] When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. NOTE 2: Lind,gaps,max, = [5] for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lind,gaps,max = [3] for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lind,gaps,max = [2] for DRX cycle>320ms. NOTE 3: Upon exceeding Lind,gaps,max over the TSSB_time_index_intra_CCA period of time, the UE has to restart the time index detection procedure. NOTE 4: KFR is a scaling factor depending on the frequency range and SSB SCS. KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.2A.6.1.

10.4.1.0.2.2	Intra-frequency measurement period

The measurement period for intra-frequency measurements with gaps is as shown in table 10.4.1.0.2.2-1.

If MCG DRX is in use, intra-frequency cell identification requirements specified in Table 10.4.1.0.2.2-1 and 10.4.1.0.2.2-2 shall depend on the MCG DRX cycle. If SCG DRX is in use, intra-frequency measurement period requirements specified in Table 10.4.1.0.2.2-1 and 10.4.1.0.2.2-2 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

The requirements apply provided any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known defined in 38.133 [6] clause 9.2A.4.3.

When the time period of unsuccessful measurement attempts due to exceeding the maximum number of unavailable at the UE SMTC occasions of an already identified cell exceeds the maximum time requirement for the cell to remain known defined in 38.133 [6] clause 9.2A.4.3, UE shall stop the measurement attempts on this SSB and perform the detection procedure again like for any other SSB.

Table 10.4.1.0.2.2-1: Measurement period for intra-frequency measurements with gaps (FR1)

| Condition | T SSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | max(200ms, (5+Lmeas,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x (5+Lmeas,gaps)) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | (5+Lmeas,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lmeas,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. When DRX is configured, Lmeas,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: Lmeas,gaps,max = 7 for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lmeas,gaps,max = 5 for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lmeas,gaps,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding Lmeas,gaps,max over the T SSB_measurement_period_intra_CCA period of time, the UE has to restart the measurement procedure. |  |

Table 10.4.1.0.2.2-2: Measurement period for intra-frequency measurements with gaps (FR2-2)

| DRX cycle | T SSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | max(400ms, (Mmeas_period with_gaps_CCA + N x Lmeas,gaps ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x (Mmeas_period with_gaps_CCA + N x Lmeas,gaps )) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| DRX cycle>320ms | (Mmeas_period with_gaps_CCA + N x Lmeas,gaps ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lmeas,gaps is the number of SMTC occasion groups not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. A SMTC occasion group consists of N consecutive SMTC occasions. An SMTC occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. When DRX is configured, Lmeas,gaps is the number of DRX cycle groups in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. A DRX occasion group consists of N consecutive DRX cycles. A DRX occasion group occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. NOTE 2: Lmeas,gaps,max = 7 for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lmeas,gaps,max = 5 for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lmeas,gaps,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding Lmeas,gaps,max over the T SSB_measurement_period_intra_CCA period of time, the UE has to restart the measurement procedure. |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.2A.6.2.

10.4.1.0.3	Minimum conformance requitements for intra-frequency RSSI and channel occupancy measurements

10.4.1.0.3.1	Intra-frequency RSSI measurements

An RSSI measurement is defined as an intra-frequency measurement provided that the RSSI measurement bandwidth is fully contained within the current carrier bandwidth of the UE.

The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [10] on one or more serving carriers operating with CCA, TS 37.213 [37], if the carrier(s) are indicated by higher layers [2], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [2] occurring with a configured RSSI measurement timing configuration periodicity [2], rmtc-Periodicity.

The UE can perform RSSI measurements without measurement gaps if RSSI measurement bandwidth is fully within the active DL BWP of the UE.

For performing intra-frequency RSSI measurement in FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier in FR2-2.

For RSSI measurement in FR2-2 with SCS of 480 kHz or 960 kHz, the RSSI measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC, and there is no requirement when measDurationSymbols is configured as 1.

The measurement period for intra-frequency RSSI measurements without measurement gaps is as shown in Table 10.4.1.0.3.1-1 and Table 10.4.1.0.3.1-2, for FR1, and in 38.133 [6] Table 9.2A.7.4-1 and 38.133 [6] Table 9.2A.7.5-1 for FR2-2. The measurement period for intra-frequency RSSI measurements with measurement gaps is as shown in Table 10.4.1.0.3.1-3, for FR1 and in Table 10.4.1.0.3.1-6 for FR2-2.

Table 10.4.1.0.3.1-1: Measurement period for intra-frequency RSSI measurements without measurement gaps when SMTC and RMTC are overlapping (FR1)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, rmtc-Periodicity*CSSFoutside_gap,i) |
| DRX | max(reportInterval, max(rmtc-Periodicity, DRX cycle) *CSSFoutside_gap,i) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFoutside_gap, i is a carrier specific scaling factor and is determined according to CSSF outside_gap,i in 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gap. |  |

Table 10.4.1.0.3.1-2: Measurement period for intra-frequency RSSI measurements without measurement gaps when SMTC and RMTC are not overlapping (FR1)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, Nintra-MO*rmtc-Periodicity) |
| DRX | max(reportInterval, Nintra-MO*max(rmtc-Periodicity, DRXcycle length)) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: Nintra-MO is defined as the number of measurement objects that can be measured without gaps |  |

Table 10.4.1.0.3.1-3: Measurement period for intra-frequency RSSI measurements with measurement gaps (FR1)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, max(rmtc-Periodicity, MGRP) x CSSFintra) |
| DRX | max(reportInterval, max(rmtc-Periodicity, MGRP,DRX cycle length) x CSSFintra) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFintra is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps. |  |

Table 10.4.1.0.3.1-4: Measurement period for intra-frequency RSSI measurements without measurement gaps when SMTC and RMTC are overlapping (FR2-2)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, rmtc-Periodicity*CSSFoutside_gap,i) |
| DRX | max(reportInterval, max(rmtc-Periodicity, DRX cycle) *CSSFoutside_gap,i) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFoutside_gap, i is a carrier specific scaling factor and is determined according to CSSF outside_gap,i in 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gap. |  |

Table 10.4.1.0.3.1-5: Measurement period for intra-frequency RSSI measurements without measurement gaps when SMTC and RMTC are not overlapping (FR2-2)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, Nintra-MO*rmtc-Periodicity) |
| DRX | max(reportInterval, Nintra-MO*max(rmtc-Periodicity, DRX cycle)) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: Nintra-MO is defined as the number of measurement objects that can be measured without gaps |  |

Table 10.4.1.0.3.1-6: Measurement period for intra-frequency RSSI measurements with measurement gaps (FR2-2)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, max(rmtc-Periodicity, MGRP) x CSSFintra) |
| DRX | max(reportInterval, max(rmtc-Periodicity, MGRP,DRX cycle) x CSSFintra) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFintra is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps. |  |

If the UE requires measurement gaps to perform intra-frequency measurements, a single measurement gap pattern is used for all concurrent intra-frequency measurements, including intra-frequency RSSI measurements. The RSSI measurement duration and the measurement gap should be aligned, and the following additional condition should be fulfilled:

-	Entire RSSI measurement duration should be contained in the measurement gap.

The RSSI measurement performed and reported according to this clause shall meet the RSSI measurement accuracy requirement in 38.133 [6] clause 10.1.34.1. The reported RSSI measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in 38.133 [6] clause 10.1.34.3.

The normative reference for this requirement is TS 38.133 [6] clause 9.2A.7.1.

10.4.1.0.3.2	Intra-frequency channel occupancy measurements

The UE shall be capable of estimating the channel occupancy on one or more serving carrier frequencies indicated by higher layers [2], based on RSSI samples provided by the physical layer.

The UE can perform channel occupancy measurements without measurement gaps if RSSI measurement bandwidth is fully within the active DL BWP of the UE.

The measurement period for intra-frequency channel occupancy measurements without measurement gap is as shown in Table 10.4.1.0.3.2-1 and Table 10.4.1.0.3.1-2 for FR1, and in Table 10.4.1.0.3.2-4 and Table 10.4.1.0.3.1-5 for FR2-2. The measurement period for intra-frequency RSSI measurements with measurement gaps is as shown in Table 10.4.1.0.3.2-3 for FR1, and in Table 10.4.1.0.3.2-6 for FR2-2 .

Table 10.4.1.0.3.2-1: Measurement period for intra-frequency Channel Occupancy measurements without measurement gaps when SMTC and RMTC are overlapping (FR1)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, rmtc-Periodicity*CSSFoutside_gap,i) |
| DRX | max(reportInterval, max(rmtc-Periodicity, DRX cycle) *CSSFoutside_gap,i) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFoutside_gap, iis a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gap. |  |

Table 10.4.1.0.3.2-2: Measurement period for intra-frequency Channel Occupancy measurements without measurement gaps when SMTC and RMTC are not overlapping (FR1)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, Nintra-MO*rmtc-Periodicity) |
| DRX | max(reportInterval, Nintra-MO*max(rmtc-Periodicity, DRXcycle length)) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: Nintra-MO is defined as the number of measurement objects that can be measured without gaps |  |

Table 10.4.1.0.3.2-3: Measurement period for intra-frequency Channel Occupancy measurements with measurement gaps (FR1)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, max(rmtc-Periodicity, MGRP) x CSSFintra) |
| DRX | max(reportInterval, max(rmtc-Periodicity, MGRP,DRX cycle length) x CSSFintra) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFintra is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps. |  |

Table 10.4.1.0.3.2-4: Measurement period for intra-frequency Channel Occupancy measurements without measurement gaps when SMTC and RMTC are overlapping (FR2-2)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, rmtc-Periodicity*CSSFoutside_gap,i) |
| DRX | max(reportInterval, max(rmtc-Periodicity, DRX cycle) *CSSFoutside_gap,i) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFoutside_gap, i is a carrier specific scaling factor and is determined according to CSSF outside_gap,i in 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gap. |  |

Table 10.4.1.0.3.2-5: Measurement period for intra-frequency Channel Occupancy measurements without measurement gaps when SMTC and RMTC are not overlapping (FR2-2)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, Nintra-MO*rmtc-Periodicity) |
| DRX | max(reportInterval, Nintra-MO*max(rmtc-Periodicity, DRX cycle)) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: Nintra-MO is defined as the number of measurement objects that can be measured without gaps |  |

Table 10.4.1.0.3.2-6: Measurement period for intra-frequency Channel Occupancy measurements with measurement gaps (FR2-2)

| Condition NOTE1,2 | T RSSI_measurement_period_intra_cca |
| --- | --- |
| No DRX | max(reportInterval, max(rmtc-Periodicity, MGRP) x CSSFintra) |
| DRX | max(reportInterval, max(rmtc-Periodicity, MGRP,DRX cycle) x CSSFintra) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: CSSFintra is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps. |  |

If the UE requires measurement gaps to perform intra-frequency measurements, a single measurement gap pattern is used for all concurrent intra-frequency measurements, including intra-frequency RSSI measurements. The RSSI measurement duration and the measurement gap should be aligned, and the following additional condition should be fulfilled:

-	Entire RSSI measurement duration should be contained in the measurement gap.

The channel occupancy measurement performed and reported according to this clause shall meet the channel occupancy measurement accuracy requirements in Clause 10.1.35.1. The reported channel occupancy measurement values contained in measurement reports shall be based on the measurement reporting range specified in TS 38.331 [13].

The normative reference for this requirement is TS 38.133 [6] clause 9.2A.7.2.

#### 10.4.1.1	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Test description, requirements, general test parameters and cell-specific test parameters are still FFS or TBD in 38.133 [6]

10.4.1.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 10.4.1.0.1.1 and 10.4.1.0.1.2.

10.4.1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting intra-freq measurements on shared channel access.

10.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.1.1.

10.4.1.1.4	Test description

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and two cells on the same carrier frequency with CCA transmitting SSBs in DBT windows according to DL CCA model: PSCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table 10.4.1.1.4.1-1 and 10.4.1.1.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

-	In the first test (Test 1), the UE is configured with SS-RSRP as Event A3 measurement quantity.

-	In the second test (Test 2), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

-	In the third test (Test 3), the UE is configured with SS-SINR as Event A3 measurement quantity.

10.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.1.1.4.1-1.

Table 10.4.1.1.4.1-1: Supported test configurations for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA

| Configuration | Description |
| --- | --- |
| 10.4.1.1-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.1.1-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.1.1.4.1-2.

Table 10.4.1.1.4.1-2: Initial conditions for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.1.1.4.1-3.

2.	Message contents are defined in clause 10.4.1.1.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 10.4.1.1.4.1-3: General test parameters for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA

TBD

10.4.1.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.1.1.5-1.

3.	SS shall transmit an RRCConnectionReconfiguration message with event A3 configured with MeasTriggerQuantityOffset set to rsrp for Test 1, rsrq for Test 2 and sinr for Test 3, with the corresponding offset according to Table 10.4.1.1.4.1-3.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.1.1.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.1.1.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [FFS1 ms for Test 1, FFS2 ms for Test 2, or FFS3 ms for Test 3], then the number of successful tests is increased by one for the corresponding Test. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one for the corresponding Test.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC PSCell release according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR Cell 2 (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If any the reconfiguration fails, switch off and on the UE and proceed with step 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

11.	Repeat steps 1-10 for each Test in Table 10.4.1.1.4.1-3 as appropriate.

10.4.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.1.1.4.3-1: Common Exception messages for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-4 with A3-offset set as per Table 10.4.1.1.4.1-3, and Condition RSRQ for Test 2 or Condition SINR for Test 3 Table H.3.1-5 Table H.3.1-7 with Condition RSRQ for Test 2 or Condition SINR for Test 3 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

10.4.1.1.5	Test requirements

Tables 10.4.1.1.4.1-3 and 10.4.1.1.5-1 define the primary level settings including test tolerances for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA.

Table 10.4.1.1.5-1: Cell specific test parameters for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA

TBD

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

D1=TBD

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.1.2	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Test description, requirements, general test parameters and cell-specific test parameters are still FFS or TBD in 38.133 [6]

10.4.1.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 10.4.1.0.1.1 and 10.4.1.0.1.2.

10.4.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier, supporting intra-freq measurements on shared channel access and long DRX.

10.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.1.2.

10.4.1.2.4	Test description

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and two cells on the same carrier frequency with CCA transmitting SSBs in DBT windows according to DL CCA model: PSCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table 10.4.1.2.4.1-1 and 10.4.1.2.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. DRX is configured on the UE.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

-	In the first set of tests (Tests 1 and 2), the UE is configured with SS-RSRP as Event A3 measurement quantity.

-	In the second set of tests (Test 3 and 4), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

-	In the third set of tests (Tests 5 and 6), the UE is configured with SS-SINR as Event A3 measurement quantity.

10.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.1.2.4.1-1.

Table 10.4.1.2.4.1-1: Supported test configurations for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA

| Configuration | Description |
| --- | --- |
| 10.4.1.2-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.1.2-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.1.2.4.1-2.

Table 10.4.1.2.4.1-2: Initial conditions for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.1.2.4.1-3.

2.	Message contents are defined in clause 10.4.1.2.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 10.4.1.2.4.1-3: General test parameters for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA

TBD

10.4.1.2.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.1.2.5-1.

3.	SS shall transmit an RRCConnectionReconfiguration message with event A3 configured with MeasTriggerQuantityOffset set to rsrp for Test 1, rsrq for Test 2 and sinr for Test 3, with the corresponding offset according to Table 10.4.1.2.4.1-3.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.1.2.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.1.2.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [FFS1 ms for Test 1, FFS2 ms for Test 2, FFS3 ms for Test 3, FFS4 ms for Test 4, FFS5 ms for Test 5 or FFS6 ms for Test 6], then the number of successful tests is increased by one for the corresponding Test. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one for the corresponding Test.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC PSCell release according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR Cell 2 (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If any the reconfiguration fails, switch off and on the UE and proceed with step 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

11.	Repeat steps 1-10 for each Test in Table 10.4.1.2.4.1-3 as appropriate.

10.4.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.1.2.4.3-1: Common Exception messages for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-4 with A3-offset set as per Table 10.4.1.2.4.1-3, and Condition RSRQ for Tests 3,4 or Condition SINR for Tests 5, 6 Table H.3.1-5 Table H.3.1-7 with Condition RSRQ for Tests 3,4 or Condition SINR for Tests 5,6 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 Table H.3.7-1 with condition DRX.TBD and Offset for test TBD Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

10.4.1.2.5	Test requirements

Tables 10.4.1.2.4.1-3 and 10.4.1.2.5-1 define the primary level settings including test tolerances for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA.

Table 10.4.1.2.5-1: Cell specific test parameters for EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA

TBD

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

D1=TBD (Editor's note: D1 is different for different DRX configurations)

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.1.3	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Test description, requirements, general test parameters and cell-specific test parameters are still FFS or TBD in 38.133 [6]

10.4.1.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 10.4.1.0.2.1 and 10.4.1.0.2.2.

10.4.1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier, supporting inter-freq measurements on shared channel access and [gap pattern TBD, in case of non-mandatory].

10.4.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.1.3.

10.4.1.3.4	Test description

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and two cells on the same carrier frequency with CCA transmitting SSBs in DBT windows according to DL CCA model: PSCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table 10.4.1.3.4.1-1 and 10.4.1.3.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Gap pattern TBD is configured on the UE.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

-	In the first test (Test 1), the UE is configured with SS-RSRP as Event A3 measurement quantity.

-	In the second test (Test 2), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

-	In the third test (Test 3), the UE is configured with SS-SINR as Event A3 measurement quantity.

10.4.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.1.3.4.1-1.

Table 10.4.1.3.4.1-1: Supported test configurations for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA

| Configuration | Description |
| --- | --- |
| 10.4.1.3-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.1.3-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.1.3.4.1-2.

Table 10.4.1.3.4.1-2: Initial conditions for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.1.3.4.1-3.

2.	Message contents are defined in clause 10.4.1.3.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 10.4.1.3.4.1-3: General test parameters for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA

TBD

10.4.1.3.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.1.3.5-1.

3.	SS shall transmit an RRCConnectionReconfiguration message with gap pattern TBD and event A3 configured with MeasTriggerQuantityOffset set to rsrp for Test 1, rsrq for Test 2 and sinr for Test 3, with the corresponding offset according to Table 10.4.1.3.4.1-3.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.1.3.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.1.3.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [FFS1 ms for Test 1, FFS2 ms for Test 2, or FFS3 ms for Test 3], then the number of successful tests is increased by one for the corresponding Test. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one for the corresponding Test.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC PSCell release according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR Cell 2 (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If any the reconfiguration fails, switch off and on the UE and proceed with step 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

11.	Repeat steps 1-10 for each Test in Table 10.4.1.3.4.1-3 as appropriate.

10.4.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.1.3.4.3-1: Common Exception messages for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condtion GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-4 with A3-offset set as per Table 10.4.1.3.4.1-3, and Condition RSRQ for Test 2 or Condition SINR for Test 3 Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern TBD and gap offset TBD [Table H.3.1-6 may need to be updated, depending on the gap config] Table H.3.1-7 with Condition RSRQ for Test 2 or Condition SINR for Test 3 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

10.4.1.3.5	Test requirements

Tables 10.4.1.3.4.1-3 and 10.4.1.3.5-1 define the primary level settings including test tolerances for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA.

Table 10.4.1.3.5-1: Cell specific test parameters for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA

TBD

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

D1=TBD

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.1.4	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Test description, requirements, general test parameters and cell-specific test parameters are still FFS or TBD in 38.133 [6]

10.4.1.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 10.4.1.0.2.1 and 10.4.1.0.2.2.

10.4.1.4.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier, supporting intra-freq measurements on shared channel access, [gap pattern TBD, in case of non-mandatory] and long DRX.

10.4.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.1.4.

10.4.1.4.4	Test description

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and two cells on the same carrier frequency with CCA transmitting SSBs in DBT windows according to DL CCA model: PSCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table 10.4.1.4.4.1-1 and 10.4.1.4.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Gap pattern TBD and DRX are configured on the UE.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

-	In the first set of tests (Tests 1 and 2), the UE is configured with SS-RSRP as Event A3 measurement quantity.

-	In the second set of tests (Test 3 and 4), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

-	In the third set of tests (Tests 5 and 6), the UE is configured with SS-SINR as Event A3 measurement quantity.

10.4.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.1.4.4.1-1.

Table 10.4.1.4.4.1-1: Supported test configurations for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA

| Configuration | Description |
| --- | --- |
| 10.4.1.4-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.1.4-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.1.4.4.1-2.

Table 10.4.1.4.4.1-2: Initial conditions for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.1.4.4.1-3.

2.	Message contents are defined in clause 10.4.1.4.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 10.4.1.4.4.1-3: General test parameters for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA

TBD

10.4.1.4.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.1.4.5-1.

3.	SS shall transmit an RRCConnectionReconfiguration message with gap pattern TBD and event A3 configured with MeasTriggerQuantityOffset set to rsrp for Test 1, rsrq for Test 2 and sinr for Test 3, with the corresponding offset according to Table 10.4.1.4.4.1-3.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.1.1.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.1.4.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [FFS1 ms for Test 1, FFS2 ms for Test 2, FFS3 ms for Test 3, FFS4 ms for Test 4, FFS5 ms for Test 5 or FFS6 ms for Test 6], then the number of successful tests is increased by one for the corresponding Test. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one for the corresponding Test.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC PSCell release according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR Cell 2 (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If any the reconfiguration fails, switch off and on the UE and proceed with step 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

11.	Repeat steps 1-10 for each Test in Table 10.4.1.4.4.1-3 as appropriate.

10.4.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.1.4.4.3-1: Common Exception messages for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-4 with A3-offset set as per Table 10.4.1.4.4.1-3, and Condition RSRQ for Tests 3,4 or Condition SINR for Tests 5,6] Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern TBD and gap offset TBD [Table H.3.1-6 may needed to be updated, depending on the gap config] Table H.3.1-7 with Condition RSRQ for Tests 3,4 or Condition SINR for Tests 5,6 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.7-1 with condition DRX.TBD and Offset for test TBD Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

10.4.1.4.5	Test requirements

Tables 10.4.1.4.4.1-3 and 10.4.1.4.5-1 define the primary level settings including test tolerances for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA.

Table 10.4.1.4.5-1: Cell specific test parameters for EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA

TBD

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

D1=TBD (Editor's note: D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 10.4.2	Inter-frequency measurements

#### 10.4.2.0	Minimum conformance requirements

10.4.2.0.1	Minimum conformance requirements for inter-frequency measurements

10.4.2.0.1.1	Inter-frequency cell identification

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_cca_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter-frequency cell, in carrier frequencies with CCA, within Tidentify_inter_cca_with_index. The UE shall be able to identify a new detectable inter-frequency SS block, in carrier frequencies with CCA, of an already detected cell within Tidentify_inter_cca_without_index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms

Where:

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in Table 10.4.2.0.1.1-1 and Table 10.4.2.0.1.1-3.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in Table 10.4.2.0.1.1-2 and Table 10.4.2.0.1.1-4.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in Table 10.4.2.0.1.2-1 and 10.4.2.0.1.2-2.

CSSFinter : it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps.

Mpss/sss_sync_inter_CCA: For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter CCA = 96. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter CCA = 60. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter CCA = 60.

MSSB_index_inter_CCA: For a UE supporting FR2-2 power class 2 or 3, MSSB_index_inter CCA = 48 samples. For a UE supporting FR2 power class 1, MSSB_index_inter CCA = 72 samples

Mmeas_period_inter_CCA: For a UE supporting FR2-2 power class 1, Mmeas_period_inter CCA = 96. For a UE supporting FR2-2 power class 2, Mmeas_period_inter  CCA = 60. For a UE supporting FR2-2 power class 3, Mmeas_period_inter CCA = 60

Table 10.4.2.0.1.1-1: Time period for PSS/SSS detection (FR1)

| Condition NOTE1,2,3,4 | TPSS/SSS_sync_inter_cca |
| --- | --- |
| No DRX | max(600ms, (8+LPSS/SSS,gaps) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(600ms, ceil((8+LPSS/SSS,gaps)x1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+LPSS/SSS,gaps) x DRX cycle x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 38.133 [6] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 38.133 [6] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When DRX is not configured, LPSS/SSS,gaps is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_inter_cca, for PSS/SSS detection, where LPSS/SSS,gaps ≤ LPSS/SSS,gaps,max. When DRX is configured, LPSS/SSS,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_inter_cca, for PSS/SSS detection, where LPSS/SSS,gaps ≤ LPSS/SSS,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFinter. NOTE 4: LPSS/SSS,gaps,max = 12 for max(DRX cycle, SMTC period, MGRP) ≤ 40 ms LPSS/SSS,gaps,max = 8 for 40 ms < max(DRX cycle, SMTC period, MGRP) ≤ 320 ms, and LPSS/SSS,gaps,max = 5 for DRX cycle > 320 ms. |  |

Upon exceeding LPSS/SSS,gaps,max, the UE is not required to meet the corresponding PSS/SSS detection requirement. The requirements apply provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

Table 10.4.2.0.1.1-2: Time period for time index detection (FR1)

| Condition NOTE1,2,3,4 | TSSB_time_index_inter_cca |
| --- | --- |
| No DRX | max(120ms, (3+ Lind,gaps) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(120ms, ceil((3+ Lind,gaps) x 1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (3 + Lind,gaps) x DRX cycle x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 38.133 [6] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 38.133 [6] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When DRX is not configured, Lind,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_inter_cca for time index identification, where Lind,gaps ≤ Lind,gaps,max. When DRX is configured, Lind,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_inter_cca for time index identification, where Lind,gaps ≤ Lind,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFinter. NOTE 4: Lind,gaps,max = 5 for max(DRX cycle, SMTC period, MGRP) ≤ 40 ms, Lind,gaps,max = 3 for 40 ms < max(DRX cycle, SMTC period, MGRP) ≤ 320 ms, and Lind,gaps,max = 2 for DRX cycle > 320 ms. |  |

The UE shall restart the time index detection upon exceeding Lind,gaps,max. The requirements apply provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

Table 10.4.2.0.1.1-3: Time period for PSS/SSS detection, (Frequency range FR2-2)

| Condition NOTE | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, ceil(Mpss/sss_sync_inter_CCA+ [NRxBeam] x LPSS/SSS,gaps)) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, ceil(1.5 Mpss/sss_sync_inter_CCA+ [NRxBeam] x LPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(Mpss/sss_sync_inter_CCA+ [NRxBeam] x LPSS/SSS,gaps) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 38.133 [6] clause 3.6.1 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 3: When DRX is not configured, LPSS/SSS,gaps is the number of SMTC occasion groups not available at the UE during TPSS/SSS_sync_inter_CCA for PSS/SSS detection, where LPSS/SSS,gaps < LPSS/SSS,gaps,max. A SMTC occasion group consists of NRxBeam consecutive SMTC occasions. An SMTC occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. When DRX is configured, LPSS/SSS is the number of [DRX cycle groups] in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_inter_CCA for PSS/SSS detection, where LPSS/SSS,gaps < LPSS/SSS,gaps,max . [A DRX occasion group consists of NRxBeam consecutive DRX cycles. A DRX occasion group occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB.] When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. NOTE 4: LPSS/SSS,gaps = 12 for max(DRX cycle, SMTC period, MGRP) ≤ 40 ms LPSS/SSS,gaps = 8 for 40 ms < max(DRX cycle, SMTC period, MGRP) ≤ 320 ms, and LPSS/SSS,gaps = 5 for DRX cycle > 320 ms |  |

Upon exceeding LPSS/SSS,gaps,max, the UE is not required to meet the corresponding PSS/SSS detection requirement. The requirements apply provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

Table 10.4.2.0.1.1-4: Time period for time index detection (FR2-2)

| Condition NOTE1,2,3,4 | TSSB_time_index_inter_cca |
| --- | --- |
| No DRX | max(120ms, ceil(MSSB_index_inter_CCA + [NRxBeam] x Lind,gaps) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(120ms, ceil(MSSB_index_inter_CCA + [NRxBeam] x Lind,gaps) x 1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | ceil(MSSB_index_inter_CCA + [NRxBeam] x Lind,gaps) x DRX cycle x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 38.133 [6] clause 3.6.1 NOTE 2: When DRX is not configured, Lind,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_inter_cca, for for time index identification, where Lind,gaps ≤ Lind,gaps,max. When DRX is configured, Lind,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_inter_cca, for for time index identification, where Lind,gaps ≤ Lind,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFinter. NOTE 3: Lind,gaps,max = [5] for max(DRX cycle, SMTC period, MGRP) ≤ 40 ms, Lind,gaps,max = [3] for 40 ms < max(DRX cycle, SMTC period, MGRP) ≤ 320 ms, and Lind,gaps,max = [2] for DRX cycle > 320 ms. |  |

The UE shall restart the time index detection upon exceeding Lind,gaps,max. The requirements apply provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

The normative reference for this requirement is TS 38.133 [6] clause 9.3A.4.

10.4.2.0.1.2	Inter-frequency measurements

When measurement gaps are provided for inter-frequency measurements in carrier frequencies with CCA, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in 38.133 [6] clauses 10.1.28, 10.1.30 and 10.1.32, respectively, as shown in Table 10.4.2.0.1.2-1:

Table 10.4.2.0.1.2-1: Measurement period for inter-frequency measurements with gaps

| Condition NOTE1,2,3,4 | T SSB_measurement_period_inter_cca |
| --- | --- |
| No DRX | max(200ms, (8+ Lmeas) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(200ms, ceil((8+ Lmeas) x 1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+ Lmeas) x DRX cycle x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 38.133 [6] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 38.133 [6] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When DRX is not configured, Lmeas is the number of SMTC occasions not available at the UE during T SSB_measurement_period_NR_cca, for inter-frequency measurements with gaps, where Lmeas ≤ Lmeas,max. When DRX is configured, Lmeas is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during T SSB_measurement_period_NR_cca, for inter-frequency measurements with gaps, where Lmeas ≤ Lmeas,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFinter. NOTE 4: Lmeas,max = 12 for max(DRX cycle, SMTC period, MGRP) ≤ 40 ms, Lmeas,max = 8 for 40 ms < max(DRX cycle, SMTC period, MGRP) ≤ 320 ms, and Lmeas,max = 5 for DRX cycle > 320 ms. |  |

The UE shall restart the measurement upon exceeding Lmeas,max. The requirements apply provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

The UE shall stop the measurement attempts on the SSB of a cell and perform the detection procedure again, like for any other SSB, when the following conditions are met:

-	Lmeas > Lmeas,max, and

-	The time period of unsuccessful measurement attempts exceeds the maximum time required for the cell to remain known as defined in TS 38.133 [6] clause 9.3A.6.3.

Table 10.4.2.0.1.2-2: Measurement period for inter-frequency measurements with gaps (FR2-2)

| Condition NOTE1,2,3,4 | T SSB_measurement_period_inter_cca |
| --- | --- |
| No DRX | max(200ms, ceil(Mmeas_period_inter_CCA + [NRxBeam] x Lmeas,gaps) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(200ms, ceil(Mmeas_period_inter_CCA + [NRxBeam] x Lmeas,gaps) x 1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | ceil(Mmeas_period_inter_CCA + [NRxBeam] x Lmeas,gaps) x DRX cycle x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 38.133 [6] clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 38.133 [6] clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When DRX is not configured, Lmeas,gaps is the number of SMTC occasions not available at the UE during T SSB_measurement_period_inter_cca, for inter-frequency measurements with gaps, where Lmeas,gaps ≤ Lmeas,gaps,max. When DRX is configured, Lmeas,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during T SSB_measurement_period_NR_cca, for inter-frequency measurements with gaps, where Lmeas,gaps ≤ Lmeas,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFinter. NOTE 4: Lmeas,gaps,max = [12] for max(DRX cycle, SMTC period, MGRP) ≤ 40 ms, Lmeas,gaps,max = [8] for 40 ms < max(DRX cycle, SMTC period, MGRP) ≤ 320 ms, and Lmeas,gaps,max = [5] for DRX cycle > 320 ms. |  |

The UE shall restart the measurement upon exceeding Lmeas,gap,max. The requirements apply provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

The normative reference for this requirement is TS 38.133 [6] clause 9.3A.5.

10.4.2.0.1.3	Inter-frequency RSSI measurements

An RSSI measurement is defined as an inter-frequency measurement provided that the RSSI measurement bandwidth is not contained within the current carrier bandwidth of the UE.

The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [10] on one or more inter-frequency carriers operating with CCA, TS 37.213 [37], if the carrier(s) are indicated by higher layers [13], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [13] occurring with a configured RSSI measurement timing configuration periodicity [13], rmtc-Periodicity. The requirements apply if rmtc-SubframeOffset [13] is configured.

For performing inter-frequency RSSI measurement in FR2-2,

-	If a UE has serving cell in FR2-2,

-	If the TCI state is provided in RMTC configuration of FR2-2, the UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. The UE does not expect to be configured with an explicit TCI-state in RMTC-Config with a reference serving cell in FR1 or FR2-1. If no TCI state is provided in the RMTC configuration, the UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a serving carrier in FR2-2.

-	If a UE has no serving cell in FR2-2,

-	The UE does not expect that a TCI state is provided in RMTC configuration. It is up to UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2, and no requirements are defined.

For RSSI measurement in FR2-2 with SCS of 480 kHz or 960 kHz, the RSSI measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC, and there is no requirements when measDurationSymbols is configured as 1.

Table 10.4.2.0.1.3-1: Measurement period for inter-frequency RSSI measurements with gaps

| Condition NOTE1,2,3,4 | T RSSI_measurement_period_inter_cca |
| --- | --- |
| No DRX | max(reportInterval, max(rmtc-Periodicity, MGRP) x CSSFinter) |
| DRX | max(reportInterval, max(rmtc-Periodicity, MGRP,DRX cycle) x CSSFinter) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFinter is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps. |  |

If the UE requires measurement gaps to perform inter-frequency measurements, a single measurement gap pattern is used for all concurrent inter-frequency measurements, including inter-frequency RSSI measurements. The RSSI measurement duration and the measurement gap shall be aligned, and the following additional condition shall be fulfilled:

-	Entire RSSI measurement duration is contained in the measurement gap.

The RSSI measurement performed and reported according to this clause shall meet the RSSI measurement accuracy requirement in 38.133 [6] Clause 10.1.34.2. The reported RSSI measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in 38.133 [6] Clause 10.1.34.3.

The normative reference for this requirement is TS 38.133 [6] clause 9.3A.8.

10.4.2.0.2	Minimum conformance requirements for channel occupancy measurements

10.4.2.0.2.1	Channel occupancy measurement reporting

The UE shall be capable of estimating the channel occupancy on one or more carrier frequencies indicated by higher layers [2], based on RSSI samples provided by the physical layer. The requirements apply if rmtc-SubframeOffset [2] is configured.

Table 10.4.2.0.2.1-1: Measurement period for inter-frequency Channel Occupancy measurements with gaps

| Condition NOTE1,2,3,4 | T CO_measurement_period_inter_cca |
| --- | --- |
| No DRX | max(reportInterval, max(rmtc-Periodicity, MGRP) x CSSFinter) |
| DRX | max(reportInterval, max(rmtc-Periodicity, MGRP,DRX cycle) x CSSFinter) |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: CSSFinter is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps. |  |

If the UE requires measurement gaps to perform inter-frequency measurements, a single measurement gap pattern is used for all concurrent inter-frequency measurements, including inter-frequency channel occupancy measurements. The RSSI measurement duration used for channel occupancy measurement and the measurement gap shall be aligned, and the following additional condition shall be fulfilled:

-	Entire RSSI measurement duration is contained in the measurement gap.

The channel occupancy measurement performed and reported according to this clause shall meet the channel occupancy measurement accuracy requirements in 38.133 [6] Clause 10.1.35.2. The reported channel occupancy measurement values contained in measurement reports shall be based on the measurement reporting range specified in TS 38.331 [13].

The normative reference for this requirement is TS 38.133 [6] clause 9.3A.9.

#### 10.4.2.1	EN-DC FR1-FR1 RSSI measurement reporting under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure need to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements, general test parameters and cell-specific test parameters are still FFS or TBD in 38.133 [6]

10.4.2.1.1	Test purpose

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the inter-frequency RSSI measurement reporting requirements in Section 10.4.2.0.1.1.

10.4.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting RSSI measurements and channel occupancy on shared channel access.

10.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.1.

10.4.2.1.4	Test description

Two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) on a licensed band and PSCell (Cell 2) with CCA transmitting SSBs in DBT windows according to DL CCA model. The test parameters for the three cells are given in Table 10.4.2.1.4.1-1 and 10.4.2.1.4.1-3 below. The RSSI measurement is performed on an inter-frequency under CCA.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

10.4.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.1.4.1-1.

Table 10.4.2.1.4.1-1: Supported test configurations for Inter-frequency RSSI measurement reporting under CCA

| Configuration | Description |
| --- | --- |
| 10.4.2.1-1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.4.2.1-2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.1.4.1-2.

Table 10.4.2.1.4.1-2: Initial conditions for Inter-frequency RSSI measurement reporting under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.1.4.1-3.

2.	Message contents are defined in clause 10.4.2.1.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 10.4.2.1.4.1-3: General test parameters for Inter-frequency RSSI measurement reporting under CCA

TBD

10.4.2.1.4.2	Test procedure

TBD

10.4.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

10.4.2.1.5	Test requirements

Tables 10.4.2.1.4.1-3 and 10.4.2.1.5-1 define the primary level settings including test tolerances for Inter-frequency RSSI measurement reporting under CCA.

Table 10.4.2.1.5-1: Cell specific test parameters for Inter-frequency RSSI measurement reporting under CCA

TBD

#### 10.4.2.2	EN-DC FR1-FR1 Channel occupancy measurement reporting under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure need to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements, general test parameters and cell-specific test parameters are still FFS or TBD in 38.133 [6]

10.4.2.2.1	Test purpose

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the inter-frequency RSSI measurement reporting requirements in Section 10.4.2.0.2.1.

10.4.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting RSSI measurements and channel occupancy on shared channel access.

10.4.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.2.

10.4.2.2.4	Test description

Two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) on a licensed band and PSCell (Cell 2) with CCA transmitting SSBs in DBT windows according to DL CCA model. The test parameters for the three cells are given in Table 10.4.2.2.4.1-1 and 10.4.2.2.4.1-3 below. The channel occupancy measurement is performed on an inter-frequency under CCA.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

10.4.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.2.4.1-1.

Table 10.4.2.2.4.1-1: Supported test configurations for Inter-frequency Channel occupancy measurement reporting under CCA

| Configuration | Description |
| --- | --- |
| 10.4.2.2-1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.4.2.2-2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.2.4.1-2.

Table 10.4.2.2.4.1-2: Initial conditions for Inter-frequency Channel occupancy measurement reporting under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.2.4.1-3.

2.	Message contents are defined in clause 10.4.2.2.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 10.4.2.2.4.1-3: General test parameters for Inter-frequency RSSI measurement reporting under CCA

TBD

10.4.2.2.4.2	Test procedure

TBD

10.4.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

10.4.2.2.5	Test requirements

Tables 10.4.2.2.4.1-3 and 10.4.2.2.5-1 define the primary level settings including test tolerances for Inter-frequency RSSI measurement reporting under CCA.

Table 10.4.2.2.5-1: Cell specific test parameters for Inter-frequency RSSI measurement reporting under CCA

TBD

#### 10.4.2.3	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.4.2.3.1	Test purpose

The purpose of the test is to verify that the UE makes correct reporting of an event in non-DRX without SSB time index detection and with CCA. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clauses 10.4.2.0.1.1 and 10.4.2.0.1.2.

10.4.2.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and RRM measurements with a shared spectrum NR carrier. Test 2 is applicable only to UEs supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Gap Pattern Id 4, otherwise Test 1 is applicable.

10.4.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.3.

10.4.2.3.4	Test description

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test configurations and parameters are given in Tables 10.4.2.3.4.1-1, 10.4.2.3.4.1-2, and 10.4.2.3.4.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table 10.4.2.3.4.1-3 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 10.4.2.3.4.1-3 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise, it is only required to pass test 1.

10.4.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.3.4.1-1.

Table 10.4.2.3.4.1-1: Supported test configurations for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used

| Config | Description |
| --- | --- |
| 10.4.2.3-1 | E-UTRAN cell: LTE FDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.3-2 | E-UTRAN cell: LTE TDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.3.4.1-2.

Table 10.4.2.3.4.1-2: Initial conditions for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.3.4.1-3.

2.	Message contents are defined in clause 10.4.2.3.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) in different frequencies with the power levels set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is switched off during the initial connection setup.

Table 10.4.2.3.4.1-3: General test parameters for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used

| Parameter | Unit | Test | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 with CCA (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 with CCA is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1,2 | As specified in clause D.7.2.1 |  |  |
| UL CCA model |  | Config 1,2 | As specified in clause D.7.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2 | 0 | 4 | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |
| CP length |  | Config 1,2 | Normal |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |
| T2 | s | Config 1,2 | 1.7 | 1.7 |  |

10.4.2.3.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test. The SS shall provide sufficient UL grants to the UE such that no SR is required for measurement reporting.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.2.3.5-1.

3.	The SS shall transmit an RRCConnectionReconfiguration message with event A3 configured on Cell 1.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.2.3.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.2.3.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [1602] ms for Test1 or [1002] ms for Test2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

10.4.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.2.3.4.3-1: Common Exception messages for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE (Test 1), or gapFR1 (Test 2) Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 (Test 1), or Pattern #4 and gap offset = 9 (Test 2) Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |
| Specific message contents exceptions for Test Configuration 10.4.2.3-1 and 10.4.2.3-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

10.4.2.3.5	Test requirements

Table 10.4.2.3.5-1 defines the primary level settings including test tolerances for all tests for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used.

Table 10.4.2.3.5-1: Cell specific test parameters for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used

| Parameter |  | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,2 | TDD |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  |  | Config 1,2 | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| Initial DL BWP |  |  | Config 1,2 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  |  | Config 1,2 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  |  | Config 1,2 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  |  | Config 1,2 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 CCA |  | - |  |
| CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 CCA |  | - |  |
| SSB parameters | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  | SSB.1 CCA |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | SSB.2 CCA |  | SSB.2 CCA |  |
| DBT window configuration |  |  | Config 1,2 | As defined in A.12.1-1 |  | As defined in A.12.1-1 |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH |  | kHz | Config 1,2 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_UL=1 |  | PCCA_UL=1 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  |  | Config 1,2 | 12 |  | 12 |  |
| WCCA_DL |  | ms | Config 1,2 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -104+TT |  | -104+TT |  |
| Note2 |  | dBm/SCS | Config 1,2 | -101+TT |  | -101+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91+TT | -91+TT | -Infinity | -88+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/38.16MHz | Config 1,2 | -58.49+TT | -58.49+TT | -63.94+TT | -56.15+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table Table 10.4.2.0.1.1-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table Table 10.4.2.0.1.2-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.2.4	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.4.2.4.1	Test purpose

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX without SSB time index detection and with CCA. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clauses 10.4.2.0.1.1 and 10.4.2.0.1.2.

10.4.2.4.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and RRM measurements with a shared spectrum NR carrier, and long DRX cycle. Tests 3 and 4 are applicable only to UEs supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Gap Pattern Id 4, otherwise Tests 1 and 2 are applicable.

10.4.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.4.

10.4.2.4.4	Test description

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2. The test parameters and configurations are given in Tables 10.4.2.4.4.1-1, 10.4.2.4.4.1-2, 10.4.2.4.4.1-3, 10.4.2.4.5-2 and 10.4.2.4.5-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 10.4.2.4.4.1-3 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 10.4.2.4.4.1-3 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise, it is only required to pass test 1&2.

10.4.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.4.4.1-1.

Table 10.4.2.4.4.1-1: Supported test configurations for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used

| Config | Description |
| --- | --- |
| 10.4.2.4-1 | E-UTRAN cell: LTE FDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.4-2 | E-UTRAN cell: LTE TDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.4.4.1-2.

Table 10.4.2.4.4.1-2: Initial conditions for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.4.4.1-3.

2.	Message contents are defined in clause 10.4.2.4.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) in different frequencies with the power levels set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is switched off during the initial connection setup.

Table 10.4.2.4.4.1-3: General test parameters for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  |  |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 with CCA (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 with CCA is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1,2 | As specified in clause D.7.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2 | As specified in clause D.7.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2 | 0 |  | 4 |  | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |  |  |
| CP length |  | Config 1,2 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2 | DRX.1 | DRX.2 | DRX.1 | DRX.2 |  |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |  |  |
| T2 | s | Config 1,2 | 2.5 | 17 | 2.5 | 17 |  |

10.4.2.4.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.2.4.5-1.

3.	The SS shall transmit an RRCConnectionReconfiguration message with event A3 configured on Cell 1.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.2.4.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.2.4.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [2402] ms for Test 1 and Test3 or [16642] ms for Test2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

11.	Repeat steps 1-10 for each Test in Tables 10.4.2.4.4.1-3 and 10.4.2.4.5-1 as appropriate.

10.4.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.2.4.4.3-1: Common Exception messages for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE (Test 1, Test 2), or gapFR1 (Test 3, Test 4) Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 (Test 1, Test 2), or Pattern #4 and gap offset = 9 (Test 3, Test 4) Table H.3.7-1 with Condition Gap, INTER-FREQ, DRX.1 (Test 1, Test 3) and DRX.2 (Test 2, Test 4) Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |
| Specific message contents exceptions for Test Configuration 10.4.2.4-1 and 10.4.2.4-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

10.4.2.4.5	Test requirements

Table 10.4.2.4.5-1 defines the primary level settings including test tolerances for all tests for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used.

Table 10.4.2.4.5-1: Cell specific test parameters for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used

| Parameter |  | Unit | Test | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,2 | TDD |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1,2 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 CCA |  |  |  |
| Initial DL BWP |  |  | Config 1,2 | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2 | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2 | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2 | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference |  |  | Config 1,2 | SR.1.1 CCA |  |  |  | - |  |  |  |
| CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 CCA |  |  |  | - |  |  |  |
| SSB parameters | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  |  |  | SSB.1 CCA |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | SSB.2 CCA |  |  |  | SSB.2 CCA |  |  |  |
| DBT window configuration |  |  | Config 1,2 | As defined in A.12.1-1 |  |  |  | As defined in A.12.1-1 |  |  |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH |  | kHz | Config 1,2 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_UL=1 |  |  |  | PCCA_UL=1 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_UL=1 |  |  |  | PCCA_UL=1 |  |  |  |
| LCCA_DL |  |  | Config 1,2 | 5 |  |  |  | 5 |  |  |  |
| WCCA_DL |  | ms | Config 1,2 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -104+TT |  |  |  | -104+TT |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -101+TT |  |  |  | -101+TT |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91+TT |  | -91+TT |  | -Infinity |  | -88+TT |  |
|  |  | dB | Config 1,2 | 4+TT |  | 4+TT |  | -Infinity |  | 7+TT |  |
|  |  | dB | Config 1,2 | 4+TT |  | 4+TT |  | -Infinity |  | 7+TT |  |
| IoNote3 |  | dBm/38.16MHz | Config 1,2 | -58.49+TT |  | -58.49+TT |  | -63.94+TT |  | -56.15+TT |  |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table 10.4.2.4.5-2: DRX-Configuration for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [13] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | ms640 |  |
| shortDRX | disable | disable |  |

Table 10.4.2.4.5-3: TimeAlignmentTimer Configuration for EN-DC FR1 vent triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [13] |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 10.4.2.0.1.1-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 10.4.2.0.1.2-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.2.5	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.4.2.5.1	Test purpose

The purpose of the test is to verify that the UE makes correct reporting of an event in non-DRX with SSB time index detection and with CCA. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clauses 10.4.2.0.1.1 and 10.4.2.0.1.2.

10.4.2.5.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and RRM measurements with a shared spectrum NR carrier. Test 2 is applicable only to UEs supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Gap Pattern Id 4, otherwise Tests 1 is applicable.

10.4.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.5.

10.4.2.5.4	Test description

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2. The test parameters and configurations are given in Tables 10.4.2.5.4.1-1, 10.4.2.5.4.1-2, and 10.4.2.5.4.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table 10.4.2.5.4.1-3 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 10.4.2.5.4.1-3 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

10.4.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.5.4.1-1.

Table 10.4.2.5.5.1-1: Supported test configurations for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used

| Config | Description |
| --- | --- |
| 10.4.2.5-1 | E-UTRAN cell: LTE FDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.5-2 | E-UTRAN cell: LTE TDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.5.4.1-2.

Table 10.4.2.5.4.1-2: Initial conditions for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.5.4.1-3.

2.	Message contents are defined in clause 10.4.2.5.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) in different frequencies with the power levels set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is switched off during the initial connection setup.

Table 10.4.2.5.4.1-3: General test parameters for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used

| Parameter | Unit | Test | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 with CCA (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 with CCA is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1,2 | As specified in clause D.7.2.1 |  |  |
| UL CCA model |  | Config 1,2 | As specified in clause D.7.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2 | 0 | 4 | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |
| CP length |  | Config 1,2 | Normal |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |
| T2 | s | Config 1,2 | 2 | 2 |  |

10.4.2.5.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test. The SS shall provide sufficient UL grants to the UE such that no SR is required for measurement reporting.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.2.5.5-1.

3.	The SS shall transmit an RRCConnectionReconfiguration message with event A3 configured on Cell 1.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.2.5.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.2.5.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [1922] ms for Test1 or [1162] ms for Test2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

10.4.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.2.5.4.3-1: Common Exception messages for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and with Condition SSB Index Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ and SSB Index Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE (Test 1), or gapFR1 (Test 2) Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 (Test 1), or Pattern #4 and gap offset = 9 (Test 2) Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |
| Specific message contents exceptions for Test Configuration 10.4.2.5-1 and 10.4.2.5-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

10.4.2.5.5	Test requirements

Table 10.4.2.5.5-1 defines the primary level settings including test tolerances for all tests for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used.

Table 10.4.2.5.5-1: Cell specific test parameters for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used

| Parameter |  | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,2 | TDD |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  |  | Config 1,2 | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| Initial DL BWP |  |  | Config 1,2 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  |  | Config 1,2 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  |  | Config 1,2 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  |  | Config 1,2 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1,2 | SR.1.1 CCA |  | - |  |
| CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 CCA |  | - |  |
| SSB parameters | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  | SSB.1 CCA |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | SSB.2 CCA |  | SSB.2 CCA |  |
| DBT window configuration |  |  | Config 1,2 | As defined in A.12.1-1 |  | As defined in A.12.1-1 |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH |  | kHz | Config 1,2 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_UL=1 |  | PCCA_UL=1 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  |  | Config 1,2 | 5 |  | 5 |  |
| WCCA_DL |  | ms | Config 1,2 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -104+TT |  | -104+TT |  |
| Note2 |  | dBm/SCS | Config 1,2 | -101+TT |  | -101+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91+TT | -91+TT | -Infinity | -88+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/38.16MHz | Config 1,2 | -58.49+TT | -58.49+TT | -63.94+TT | -56.15+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 10.4.2.0.1.1-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 10.4.2.0.1.1-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 10.4.2.0.1.2-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.2.6	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.4.2.6.1	Test purpose

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX without SSB time index detection and with CCA. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clauses 10.4.2.0.1.1 and 10.4.2.0.1.2.

10.4.2.6.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and RRM measurements with a shared spectrum NR carrier, and long DRX cycle. Tests 3 and 4 are applicable only to UEs supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Gap Pattern Id 4, otherwise Tests 1 and 2 are applicable.

10.4.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.6.

10.4.2.6.4	Test description

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2. The test parameters and configurations are given in Tables 10.4.2.6.4.1-1, 10.4.2.6.4.1-2, 10.4.2.6.4.1-3, 10.4.2.6.5-2 and 10.4.2.6.5-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 10.4.2.6.4.1-3 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 10.4.2.6.4.1-3 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise, it is only required to pass test 1&2.

10.4.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.6.4.1-1.

Table 10.4.2.6.4.1-1: Supported test configurations for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used

| Config | Description |
| --- | --- |
| 10.4.2.6-1 | E-UTRAN cell: LTE FDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.6-2 | E-UTRAN cell: LTE TDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.6.4.1-2.

Table 10.4.2.6.4.1-2: Initial conditions for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.6.4.1-3.

2.	Message contents are defined in clause 10.4.2.6.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) in different frequencies with the power levels set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is switched off during the initial connection setup.

Table 10.4.2.6.4.1-3: General test parameters for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  |  |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 with CCA (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 with CCA is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1,2 | As specified in clause D.7.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2 | As specified in clause D.7.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2 | 0 |  | 4 |  | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |  |  |
| CP length |  | Config 1,2 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2 | DRX.1 | DRX.2 | DRX.1 | DRX.2 |  |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |  |  |
| T2 | s | Config 1,2 | 3 | 20 | 3 | 20 |  |

10.4.2.6.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.2.6.5-1.

3.	The SS shall transmit an RRCConnectionReconfiguration message with event A3 configured on Cell 1.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.2.6.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.2.6.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [2882] ms for Test 1 and Test3 or [19842] ms for Test2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

11.	Repeat steps 1-10 for each Test in Tables 10.4.2.6.4.1-3 and 10.4.2.6.5-1 as appropriate.

10.4.2.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.2.6.4.3-1: Common Exception messages for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and with Condition SSB Index Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ and SSB Index Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE (Test 1, Test 2), or gapFR1 (Test 3, Test 4) Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 (Test 1, Test 2), or Pattern #4 and gap offset = 9 (Test 3, Test 4) Table H.3.7-1 with Condition Gap, INTER-FREQ, DRX.1 (Test 1, Test 3) and DRX.2 (Test 2, Test 4) Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |
| Specific message contents exceptions for Test Configuration 10.4.2.6-1 and 10.4.2.6-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

10.4.2.6.5	Test requirements

Table 10.4.2.6.5-1 defines the primary level settings including test tolerances for all tests for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used.

Table 10.4.2.6.5-1: Cell specific test parameters for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used

| Parameter |  | Unit | Test | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,2 | TDD |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1,2 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 CCA |  |  |  |
| Initial DL BWP |  |  | Config 1,2 | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2 | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2 | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2 | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference |  |  | Config 1,2 | SR.1.1 CCA |  |  |  | - |  |  |  |
| CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 CCA |  |  |  | - |  |  |  |
| SSB parameters | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  |  |  | SSB.1 CCA |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | SSB.2 CCA |  |  |  | SSB.2 CCA |  |  |  |
| DBT window configuration |  |  | Config 1,2 | As defined in A.12.1-1 |  |  |  | As defined in A.12.1-1 |  |  |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH |  | kHz | Config 1,2 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_UL=1 |  |  |  | PCCA_UL=1 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_UL=1 |  |  |  | PCCA_UL=1 |  |  |  |
| LCCA_DL |  |  | Config 1,2 | 2 |  |  |  | 2 |  |  |  |
| WCCA_DL |  | ms | Config 1,2 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -104+TT |  |  |  | -104+TT |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -101+TT |  |  |  | -101+TT |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91+TT |  | -91+TT |  | -Infinity |  | -88+TT |  |
|  |  | dB | Config 1,2 | 4+TT |  | 4+TT |  | -Infinity |  | 7+TT |  |
|  |  | dB | Config 1,2 | 4+TT |  | 4+TT |  | -Infinity |  | 7+TT |  |
| IoNote3 |  | dBm/38.16MHz | Config 1,2 | -58.49+TT |  | -58.49+TT |  | -63.94+TT |  | -56.15+TT |  |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table 10.4.2.6.5-2: DRX-Configuration for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [13] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | ms640 |  |
| shortDRX | disable | disable |  |

Table 10.4.2.6.5-3: TimeAlignmentTimer Configuration for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [13] |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 10.4.2.0.1.1-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 10.4.2.0.1.1-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 10.4.2.0.1.2-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.2.7	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.4.2.7.1	Test purpose

The purpose of the test is to verify that the UE makes correct reporting of an event in non-DRX without SSB time index detection. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clauses 10.4.2.0.1.1 and 10.4.2.0.1.2.

10.4.2.7.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and RRM measurements with a shared spectrum NR carrier. Test 2 is applicable only to UEs supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Gap Pattern Id 4, otherwise Tests 1 is applicable.

10.4.2.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.7.

10.4.2.7.4	Test description

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.  The test parameters and configurations are given in Tables 10.4.2.7.4.1-1, 10.4.2.7.4.1-2, and 10.4.2.7.4.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table 10.4.2.7.4.1-3 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 10.4.2.7.4.1-3 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

10.4.2.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.7.4.1-1.

Table 10.4.2.73.4.1-1: Supported test configurations for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used

| Config | Description |
| --- | --- |
| 10.4.2.7-1 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.7-2 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.7-3 | E-UTRAN cell: LTE FDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.7-4 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.7-5 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.7-6 | E-UTRAN cell: LTE TDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.7.4.1-2.

Table 10.4.2.7.4.1-2: Initial conditions for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.7.4.1-3.

2.	Message contents are defined in clause 10.4.2.7.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) in different frequencies with the power levels set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is switched off during the initial connection setup.

Table 10.4.2.7.4.1-3: General test parameters for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used

| Parameter | Unit | Test | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  | Two FR1 NR carrier frequencies are used. NR RF channel 1 is with CCA. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause D.7.2.1 |  |  |
| UL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause D.7.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 | 4 | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 3 ms |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 1.7 | 1.7 |  |

10.4.2.7.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test. The SS shall provide sufficient UL grants to the UE such that no SR is required for measurement reporting.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.2.7.5-1.

3.	The SS shall transmit an RRCConnectionReconfiguration message with event A3 configured on Cell 1.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.2.7.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.2.7.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [1602] ms for Test1 or [1002] ms for Test2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

10.4.2.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.2.7.4.3-1: Common Exception messages for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE (Test 1), or gapFR1 (Test 2) Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 (Test 1), or Pattern #4 and gap offset = 9 (Test 2) Table H.3.10-1 with Condition SSB.1 CCA and SemiStatic, for semi-static channel access; or Condition SSB.2 CCA and Dynamic, for dynamic channel access |
| Test Configuration 10.4.2.7-1, 10.4.2.7-2, 10.4.2.7-4 and 10.4.2.7-5 | Table H.3.4-6 with Condition SSB.1 FR1 |
| Test Configuration 10.4.2.7-3, 10.4.2.7-6, | Table H.3.4-6 with Condition SSB.2 FR1 |
| Specific message contents exceptions for Test Configuration 10.4.2.7-2, 10.4.2.7-3, 10.4.2.7-5 and 10.4.2.7-6 Test Configuration 10.4.2.7-1 and 10.4.2.7-4 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |

10.4.2.7.5	Test requirements

Table 10.4.2.7.5-1 defines the primary level settings including test tolerances for all tests for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used

Table 10.4.2.7.5-1: Cell specific test parameters for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used

| Parameter |  | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,4 | TDD |  | FDD |  |
|  |  |  | Config 2,3,5,6 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| TDD configuration |  |  | Config 1,4 | TDDConf.1.1 CCA |  | NA |  |
|  |  |  | Config 2,5 | TDDConf.1.1 CCA |  | TDDConf.1.1 |  |
|  |  |  | Config 3,6 | TDDConf.1.1 CCA |  | TDDConf.2.1 |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2,3,4,5,6 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2,3,4,5,6 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 CCA |  | SR.1.1 FDD |  |
|  |  |  | Config 2,5 | SR.1.1 CCA |  | SR.1.1 TDD |  |
|  |  |  | Config 3,6 | SR.1.1 CCA |  | SR.2.1 TDD |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 CCA |  | CR.1.1 FDD |  |
|  |  |  | Config 2,5 | CR.1.1 CCA |  | CR.1.1 TDD |  |
|  |  |  | Config 3,6 | CR.1.1 CCA |  | CR.2.1 TDD |  |
| SSB | Semi-static channel access Note 5,7 |  | Config 1,4 | SSB.1 CCA |  | SSB.1 FR1 |  |
| parameters |  |  | Config 2,5 | SSB.1 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 3,6 | SSB.1 CCA |  | SSB.2 FR1 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,4 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 2,5 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 3,6 | SSB.2 CCA |  | SSB.2 FR1 |  |
| DBT window configuration |  |  | Config 1,2,3,4,5,6 | As defined in A.12.1-1 |  | Not applicable |  |
| SMTC configuration |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2,3,5,6 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH |  | kHz | Config 1,2,4,5 | 30 |  | 15 |  |
| subcarrier spacing |  |  | Config 3,6 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_DL=0.9375 |  | Not applicable |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | Not applicable |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  | Not applicable |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  | Not applicable |  |
| LCCA_DL |  |  | Config 1,2,3,4,5,6 | 12 |  | 12 |  |
| WCCA_DL |  | ms | Config 1,2,3,4,5,6 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3,4,5,6 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4,5,6 | -104+TT |  | -98+TT |  |
| Note2 |  | dBm/SCS | Config 1,2,4,5 | -101+TT |  | -98+TT |  |
|  |  |  | Config 3,6 | -101+TT |  | -95+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4,5 | -94+TT | -94+TT | -Infinity | -91+TT |
|  |  |  | Config 3,6 | -91+TT | -91+TT | -Infinity | -88+TT |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT | 4+TT | -Infinity | 7+TT |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT | 4+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/9.36MHz | NR Config 1,2,4,5 | -58.49+TT | -58.49+TT | -70.05+TT | -62.26+TT |
|  |  | dBm/38.16MHz | NR Config 3,6 | -58.49+TT | -58.49+TT | -63.94+TT | -56.15+TT |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 10.4.2.0.1.1-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 10.4.2.0.1.2-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

#### NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.10.4.2.8	EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.4.2.8.1	Test purpose

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX without SSB time index detection and with CCA. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clauses 10.4.2.0.1.1 and 10.4.2.0.1.2.

10.4.2.8.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and RRM measurements with a shared spectrum NR carrier, and long DRX cycle. Tests 3 and 4 are applicable only to UEs supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Gap Pattern Id 4, otherwise Tests 1 and 2 are applicable.

10.4.2.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.8.

10.4.2.8.4	Test description

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2. The test parameters and configurations are given in Tables 10.4.2.8.4.1-1, 10.4.2.8.4.1-2, 10.4.2.8.4.1-3, 10.4.2.8.5-2 and 10.4.2.8.5-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 10.4.2.8.4.1-3 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 10.4.2.8.4.1-3 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

10.4.2.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.8.4.1-1.

Table 10.4.2.8.4.1-1: Supported test configurations for EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used

| Config | Description |
| --- | --- |
| 10.4.2.8-1 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.8-2 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.8-3 | E-UTRAN cell: LTE FDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.8-4 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.8-5 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.8-6 | E-UTRAN cell: LTE TDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.8.4.1-2.

Table 10.4.2.8.4.1-2: Initial conditions for EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.8.4.1-3.

2.	Message contents are defined in clause 10.4.2.8.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) in different frequencies with the power levels set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is switched off during the initial connection setup.

Table 10.4.2.8.4.1-3: General test parameters for EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. NR RF channel 1 is with CCA. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause D.7.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause D.7.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 4 |  | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | DRX.1 | DRX.2 | DRX.1 | DRX.2 |  |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 3 ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 2.5 | 17 | 2.5 | 17 |  |

10.4.2.8.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.2.8.5-1.

3.	The SS shall transmit an RRCConnectionReconfiguration message with event A3 configured on Cell 1.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.2.8.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.2.8.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [2402] ms for Test 1 and Test3 or [16642] ms for Test2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

11.	Repeat steps 1-10 for each Test in Tables 10.4.2.8.4.1-3 and 10.4.2.8.5-1 as appropriate.

10.4.2.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.2.8.4.3-1: Common Exception messages for EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE (Test 1, Test 2), or gapFR1 (Test 3, Test 4) Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 (Test 1, Test 2), or Pattern #4 and gap offset = 9 (Test 3, Test 4) Table H.3.7-1 with Condition Gap, INTER-FREQ, DRX.1 (Test 1, Test 3) and DRX.2 (Test 2, Test 4) Table H.3.10-1 with Condition SSB.1 CCA and SemiStatic, for semi-static channel access; or Condition SSB.2 CCA and Dynamic, for dynamic channel access |
| Test Configuration 10.4.2.8-1, 10.4.2.8-2, 10.4.2.8-4 and 10.4.2.8-5 | Table H.3.4-6 with Condition SSB.1 FR1 |
| Test Configuration 10.4.2.8-3, 10.4.2.8-6, | Table H.3.4-6 with Condition SSB.2 FR1 |
| Specific message contents exceptions for Test Configuration 10.4.2.8-2, 10.4.2.8-3, 10.4.2.8-5 and 10.4.2.8-6 Test Configuration 10.4.2.8-1 and 10.4.2.8-4 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |

10.4.2.8.5	Test requirements

Table 10.4.2.8.5-1 defines the primary level settings including test tolerances for all tests for EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used.

Table 10.4.2.8.5-1: Cell specific test parameters for EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used

| Parameter |  | Unit | Test | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,4 | TDD |  |  |  | FDD |  |  |  |
|  |  |  | Config 2,3,5,6 | TDD |  |  |  | TDD |  |  |  |
| BWchannel |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  |  | Config 1,4 | TDDConf.1.1 CCA |  |  |  | NA |  |  |  |
|  |  |  | Config 2,5 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3,6 | TDDConf.1.1 CCA |  |  |  | TDDConf.2.1 |  |  |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1,2,3,4,5,6 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns |  |  | Config 1,2,3,4,5,6 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 CCA |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2,5 | SR.1.1 CCA |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3,6 | SR.1.1 CCA |  |  |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 CCA |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2,5 | CR.1.1 CCA |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3,6 | CR.1.1 CCA |  |  |  | CR.2.1 TDD |  |  |  |
| SSB | Semi-static channel access Note 5,7 |  | Config 1,4 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
| parameters |  |  | Config 2,5 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 | SSB.1 CCA |  |  |  | SSB.2 FR1 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,4 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 2,5 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 | SSB.2 CCA |  |  |  | SSB.2 FR1 |  |  |  |
| DBT window configuration |  |  | Config 1,2,3,4,5,6 | As defined in A.12.1-1 |  |  |  | Not applicable |  |  |  |
| SMTC configuration |  |  | Config 1,4 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
|  |  |  | Config 2,3,5,6 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH |  | kHz | Config 1,2,4,5 | 30 |  |  |  | 15 |  |  |  |
| subcarrier spacing |  |  | Config 3,6 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_DL=0.9375 |  |  |  | Not applicable |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | Not applicable |  |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  |  |  | Not applicable |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  |  |  | Not applicable |  |  |  |
| LCCA_DL |  |  | Config 1,2,3,4,5,6 | 5 |  |  |  | 5 |  |  |  |
| WCCA_DL |  | ms | Config 1,2,3,4,5,6 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3,4,5,6 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4,5,6 | -104+TT |  |  |  | -98+TT |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2,4,5 | -101+TT |  |  |  | -98+TT |  |  |  |
|  |  |  | Config 3,6 | -101+TT |  |  |  | -95+TT |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4,5 | -94+TT |  | -94+TT |  | -Infinity |  | -91+TT |  |
|  |  |  | Config 3,6 | -91+TT |  | -91+TT |  | -Infinity |  | -88+TT |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT |  | 4+TT |  | -Infinity |  | 7+TT |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT |  | 4+TT |  | -Infinity |  | 7+TT |  |
| IoNote3 |  | dBm/9.36MHz | NR Config 1,2,4,5 | -58.49+TT |  | -58.49+TT |  | -70.05+TT |  | -62.26+TT |  |
|  |  | dBm/38.16MHz | NR Config 3,6 | -58.49+TT |  | -58.49+TT |  | -63.94+TT |  | -56.15+TT |  |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table 10.4.2.8.5-2: DRX-Configuration for EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [13] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | ms640 |  |
| shortDRX | disable | disable |  |

Table 10.4.2.8.5-3: TimeAlignmentTimer Configuration for EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [13] |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 10.4.2.0.1.1-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 10.4.2.0.1.2-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.2.9	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.4.2.9.1	Test purpose

The purpose of the test is to verify that the UE makes correct reporting of an event in non-DRX with SSB time index detection. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clauses 10.4.2.0.1.1 and 10.4.2.0.1.2.

10.4.2.9.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and RRM measurements with a shared spectrum NR carrier. Test 2 is applicable only to UEs supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Gap Pattern Id 4, otherwise Tests 1 is applicable.

10.4.2.9.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.9.

10.4.2.9.4	Test description

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2. The test parameters and configurations are given in Tables 10.4.2.9.4.1-1, 10.4.2.9.4.1-2, and 10.4.2.9.4.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table 10.4.2.9.4.1-3 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 10.4.2.9.4.1-3 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

10.4.2.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.9.4.1-1.

Table 10.4.2.9.4.1-1: Supported test configurations for EN-DC FR1-FR1 Event-triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used

| Config | Description |
| --- | --- |
| 10.4.2.9-1 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.9-2 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.9-3 | E-UTRAN cell: LTE FDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.9-4 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.9-5 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mod |
| 10.4.2.9-6 | E-UTRAN cell: LTE TDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.9.4.1-2.

Table 10.4.2.9.4.1-2: Initial conditions for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.9.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.9.4.1-3.

2.	Message contents are defined in clause 10.4.2.9.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) in different frequencies with the power levels set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is switched off during the initial connection setup.

Table 10.4.2.9.4.1-3: General test parameters for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used

| Parameter | Unit | Test | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  | Two FR1 NR carrier frequencies are used. NR RF channel 1 is with CCA. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause D.7.2.1 |  |  |
| UL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause D.7.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 | 4 | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 3 ms |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 2 | 2 |  |

10.4.2.9.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test. The SS shall provide sufficient UL grants to the UE such that no SR is required for measurement reporting.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.2.9.5-1.

3.	The SS shall transmit an RRCConnectionReconfiguration message with event A3 configured on Cell 1.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.2.9.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.2.9.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [1922] ms for Test1 or [1162] ms for Test2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

10.4.2.9.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.2.9.4.3-1: Common Exception messages for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and with Condition SSB Index Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ and SSB Index Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE (Test 1), or gapFR1 (Test 2) Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 (Test 1), or Pattern #4 and gap offset = 9 (Test 2) Table H.3.10-1 with Condition SSB.1 CCA and SemiStatic, for semi-static channel access; or Condition SSB.2 CCA and Dynamic, for dynamic channel access |
| Test Configuration 10.4.2.9-1, 10.4.2.9-2, 10.4.2.9-4 and 10.4.2.9-5 | Table H.3.4-6 with Condition SSB.1 FR1 |
| Test Configuration 10.4.2.9-3, 10.4.2.9-6, | Table H.3.4-6 with Condition SSB.2 FR1 |
| Specific message contents exceptions for Test Configuration 10.4.2.9-2, 10.4.2.9-3, 10.4.2.9-5 and 10.4.2.9-6 Test Configuration 10.4.2.9-1 and 10.4.2.9-4 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |

10.4.2.9.5	Test requirements

Table 10.4.2.9.5-1 defines the primary level settings including test tolerances for all tests for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used.

Table 10.4.2.9.5-1: Cell specific test parameters for EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used

| Parameter |  | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,4 | TDD |  | FDD |  |
|  |  |  | Config 2,3,5,6 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| TDD configuration |  |  | Config 1,4 | TDDConf.1.1 CCA |  | NA |  |
|  |  |  | Config 2,5 | TDDConf.1.1 CCA |  | TDDConf.1.1 |  |
|  |  |  | Config 3,6 | TDDConf.1.1 CCA |  | TDDConf.2.1 |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2,3,4,5,6 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2,3,4,5,6 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 CCA |  | SR.1.1 FDD |  |
|  |  |  | Config 2,5 | SR.1.1 CCA |  | SR.1.1 TDD |  |
|  |  |  | Config 3,6 | SR.1.1 CCA |  | SR.2.1 TDD |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 CCA |  | CR.1.1 FDD |  |
|  |  |  | Config 2,5 | CR.1.1 CCA |  | CR.1.1 TDD |  |
|  |  |  | Config 3,6 | CR.1.1 CCA |  | CR.2.1 TDD |  |
| SSB | Semi-static channel access Note 5,7 |  | Config 1,4 | SSB.1 CCA |  | SSB.1 FR1 |  |
| parameters |  |  | Config 2,5 | SSB.1 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 3,6 | SSB.1 CCA |  | SSB.2 FR1 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,4 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 2,5 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 3,6 | SSB.2 CCA |  | SSB.2 FR1 |  |
| DBT window configuration |  |  | Config 1,2,3,4,5,6 | As defined in A.12.1-1 |  | Not applicable |  |
| SMTC configuration |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2,3,5,6 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH |  | kHz | Config 1,2,4,5 | 30 |  | 15 |  |
| subcarrier spacing |  |  | Config 3,6 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_DL=0.9375 |  | Not applicable |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | Not applicable |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  | Not applicable |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  | Not applicable |  |
| LCCA_DL |  |  | Config 1,2,3,4,5,6 | 5 |  | 5 |  |
| WCCA_DL |  | ms | Config 1,2,3,4,5,6 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3,4,5,6 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4,5,6 | -104+TT |  | -98+TT |  |
| Note2 |  | dBm/SCS | Config 1,2,4,5 | -101+TT |  | -98+TT |  |
|  |  |  | Config 3,6 | -101+TT |  | -95+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4,5 | -94+TT | -94+TT | -Infinity | -91+TT |
|  |  |  | Config 3,6 | -91+TT | -91+TT | -Infinity | -88+TT |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT | 4+TT | -Infinity | 7+TT |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT | 4+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/9.36MHz | NR Config 1,2,4,5 | -58.49+TT | -58.49+TT | -70.05+TT | -62.26+TT |
|  |  | dBm/38.16MHz | NR Config 3,6 | -58.49+TT | -58.49+TT | -63.94+TT | -56.15+TT |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 10.4.2.0.1.1-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 10.4.2.0.1.1-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 10.4.2.0.1.2-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 10.4.2.10	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

10.4.2.10.1	Test purpose

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX without SSB time index detection and with CCA. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clauses 10.4.2.0.1.1 and 10.4.2.0.1.2.

10.4.2.10.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and RRM measurements with a shared spectrum NR carrier, and long DRX cycle. Tests 3 and 4 are applicable only to UEs supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Gap Pattern Id 4, otherwise Tests 1 and 2 are applicable.

10.4.2.10.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.4.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.4.2.10.

10.4.2.10.4	Test description

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2. The test parameters and configurations are given in Tables 10.4.2.10.4.1-1, 10.4.2.10.4.1-2, 10.4.2.10.4.1-3, 10.4.2.10.5-2 and 10.4.2.10.5-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 10.4.2.10.4.1-3 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 10.4.2.10.4.1-3 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

10.4.2.10.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.4.2.10.4.1-1.

Table 10.4.2.10.4.1-1: Supported test configurations for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used

| Config | Description |
| --- | --- |
| 10.4.2.10-1 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.10-2 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.10-3 | E-UTRAN cell: LTE FDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.10-4 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.4.2.10-5 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mod |
| 10.4.2.10-6 | E-UTRAN cell: LTE TDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 10.4.2.10.4.1-2.

Table 10.4.2.10.4.1-2: Initial conditions for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.2.10.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 10.4.2.10.4.1-3.

2.	Message contents are defined in clause 10.4.2.10.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are NR cells (PSCell and neighbour cell, respectively) in different frequencies with the power levels set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is switched off during the initial connection setup.

Table 10.4.2.10.4.1-3: General test parameters for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. NR RF channel 1 is with CCA. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause D.7.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause D.7.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 4 |  | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.5 |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 3 ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 3 | 20 | 3 | 20 |  |

10.4.2.10.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 10.4.2.10.5-1.

3.	The SS shall transmit an RRCConnectionReconfiguration message with event A3 configured on Cell 1.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall enable DL and UL CCA model according to Table 10.4.2.10.5-1. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 10.4.2.10.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than [2882] ms for Test 1 and Test3 or [19842] ms for Test2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. The SS shall disable the CCA model (both DL and UL, i.e. PCCA_DL_i= PCCA_UL=1).

8.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.TBD in Annex G clause G.TBD is achieved.

11.	Repeat steps 1-10 for each Test in Tables 10.4.2.10.4.1-3 and 10.4.2.10.5-1 as appropriate.

10.4.2.10.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.4.2.10.4.3-1: Common Exception messages for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and with Condition SSB Index Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ and SSB Index Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE (Test 1, Test 2), or gapFR1 (Test 3, Test 4) Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 (Test 1, Test 2), or Pattern #4 and gap offset = 9 (Test 3, Test 4) Table H.3.7-1 with Condition Gap, INTER-FREQ, DRX.1 (Test 1, Test 3) and DRX.2 (Test 2, Test 4) Table H.3.10-1 with Condition SSB.1 CCA and SemiStatic, for semi-static channel access; or Condition SSB.2 CCA and Dynamic, for dynamic channel access |
| Test Configuration 10.4.2.10-1, 10.4.2.10-2, 10.4.2.10-4 and 10.4.2.10-5 | Table H.3.4-6 with Condition SSB.1 FR1 |
| Test Configuration 10.4.2.10-3, 10.4.2.10-6, | Table H.3.4-6 with Condition SSB.2 FR1 |
| Specific message contents exceptions for Test Configuration 10.4.2.10-2, 10.4.2.10-3, 10.4.2.10-5 and 10.4.2.10-6 Test Configuration 10.4.2.10-1 and 10.4.2.10-4 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |

10.4.2.10.5	Test requirements

Table 10.4.2.10.5-1 defines the primary level settings including test tolerances for all tests for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used.

Table 10.4.2.10.5-1: Cell specific test parameters for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used

| Parameter |  | Unit | Test | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,4 | TDD |  |  |  | FDD |  |  |  |
|  |  |  | Config 2,3,5,6 | TDD |  |  |  | TDD |  |  |  |
| BWchannel |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  |  | Config 1,4 | TDDConf.1.1 CCA |  |  |  | NA |  |  |  |
|  |  |  | Config 2,5 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3,6 | TDDConf.1.1 CCA |  |  |  | TDDConf.2.1 |  |  |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1,2,3,4,5,6 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns |  |  | Config 1,2,3,4,5,6 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 CCA |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2,5 | SR.1.1 CCA |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3,6 | SR.1.1 CCA |  |  |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 CCA |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2,5 | CR.1.1 CCA |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3,6 | CR.1.1 CCA |  |  |  | CR.2.1 TDD |  |  |  |
| SSB | Semi-static channel access Note 5,7 |  | Config 1,4 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
| parameters |  |  | Config 2,5 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 | SSB.1 CCA |  |  |  | SSB.2 FR1 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,4 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 2,5 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 | SSB.2 CCA |  |  |  | SSB.2 FR1 |  |  |  |
| DBT window configuration |  |  | Config 1,2,3,4,5,6 | As defined in A.12.1-1 |  |  |  | Not applicable |  |  |  |
| SMTC configuration |  |  | Config 1,4 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
|  |  |  | Config 2,3,5,6 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH |  | kHz | Config 1,2,4,5 | 30 |  |  |  | 15 |  |  |  |
| subcarrier spacing |  |  | Config 3,6 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_DL=0.9375 |  |  |  | Not applicable |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | Not applicable |  |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  |  |  | Not applicable |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  |  |  | Not applicable |  |  |  |
| LCCA_DL |  |  | Config 1,2,3,4,5,6 | 2 |  |  |  | 2 |  |  |  |
| WCCA_DL |  | ms | Config 1,2,3,4,5,6 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3,4,5,6 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4,5,6 | -104+TT |  |  |  | -98+TT |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2,4,5 | -101+TT |  |  |  | -98+TT |  |  |  |
|  |  |  | Config 3,6 | -101+TT |  |  |  | -95+TT |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4,5 | -94+TT |  | -94+TT |  | -Infinity |  | -91+TT |  |
|  |  |  | Config 3,6 | -91+TT |  | -91+TT |  | -Infinity |  | -88+TT |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT |  | 4+TT |  | -Infinity |  | 7+TT |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT |  | 4+TT |  | -Infinity |  | 7+TT |  |
| IoNote3 |  | dBm/9.36MHz | NR Config 1,2,4,5 | -58.49+TT |  | -58.49+TT |  | -70.05+TT |  | -62.26+TT |  |
|  |  | dBm/38.16MHz | NR Config 3,6 | -58.49+TT |  | -58.49+TT |  | -63.94+TT |  | -56.15+TT |  |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table 10.4.2.10.5-2: DRX-Configuration for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [13] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | ms640 |  |
| shortDRX | disable | disable |  |

Table 10.4.2.10.5-3: TimeAlignmentTimer Configuration for EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [13] |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 10.4.2.0.1.1-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 10.4.2.0.1.1-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 10.4.2.0.1.2-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## 10.5	Measurement Performance

### 10.5.1	SS-RSRP

#### 10.5.1.0	Minimum Conformance Requirements

##### 10.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell under CCA.

The accuracy requirements in Table 10.5.1.0.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.8 for a corresponding Band for each relevant SSB.

Table 10.5.1.0.1-1: SS-RSRP intra-frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 8 | 11 | -6 | NR_CCA_FR1_I | N/A | N/A | -70 | -50 |
|  |  |  | NR_CCA_FR1_J |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.36.1.1.

##### 10.5.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell under CCA.

The accuracy requirements in Table 10.5.1.0.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.8 for a corresponding Band for each relevant SSB.

Table 10.5.1.0.2-1: SS-RSRP intra-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2 | 3 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.36.1.2.

##### 10.5.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA

The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency under CCA that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.5.1.0.3-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.9 for a corresponding Band for each relevant SSB.

Table 10.5.1.0.3-1: SS-RSRP inter-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 8 | 11 | -6 | NR_CCA_FR1_I | N/A | N/A | -70 | -50 |
|  |  |  | NR_CCA_FR1_J |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.37.1.1.

##### 10.5.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA

The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency compared to the RSRP measured from another cell on a different frequency, with at least one of the two frequencies being under CCA.

The accuracy requirements in Table 10.5.1.0.4-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.9 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.5.1.0.4-1: SS-RSRP inter-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 6 | -6 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.37.1.2.

#### 10.5.1.1	EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

10.5.1.1.1	Test purpose

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.5.1.0.1 and 10.5.1.0.2 when the serving cell is subject to CCA.

10.5.1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier andsupporting intra-frequency measurements on shared channel access.

10.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.5.1.0.1 and 10.5.1.0.2 for absolute and relative accuracy, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.1.1.

10.5.1.1.4	Test description

Supported test configurations are shown in Table 10.5.1.1.4.1-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in Table 10.5.1.1.5-1. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.6B. In all test cases, Cell 2 is the PSCell, and Cell 3 is the target cell.

10.5.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.1.1.4.1-1.

Table 10.5.1.1.4.1-1: Supported test configurations for EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell

| Configuration | Description |
| --- | --- |
| 10.5.1.1-1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.5.1.1-2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.1.1.4.1-2.

Table 10.5.1.1.4.1-2: Initial conditions for EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.1.1.5-1.

2.	Message contents are defined in clause 10.5.1.1.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.1.1.4.2	Test procedure

In this set of test cases there are three cells, E-UTRAN PCell (Cell 1), FR1 PSCell under CCA (Cell 2), and target neighbour FR1 Cell 3. Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 10.5.1.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform intra-frequency measurements in DBT windows as specified in section 10.5.1.1.4.3.

4.	The UE shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 10.5.1.1.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 3 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 10.5.1.1.5-2 or the UE fails to report the measurement value for Cell 3 [for FFS consecutive transmissions], the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 2-7 for each subtest defined in Table 10.5.1.1.5-1.

10.5.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.1.1.4.3-1: Common Exception messages for EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

Table 10.5.1.1.4.3-2: ReportConfigNR for EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

10.5.1.1.5	Test requirements

Table 10.5.1.1.5-1 defines the primary level settings including test tolerances for EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell. Table 10.5.1.1.5-2 and Table 10.5.1.1.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 10.5.1.1.5-1: Cell specific test parameters for EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Physical cell ID |  |  |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  |  |  |
| Duplex mode |  | Config 1, 2 |  | TDD |  |  |  |
| TDD configuration |  | Config 1, 2 |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  | Config 1, 2 | MHz | 40: NRB,c = 106 |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  | Config 1, 2 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1, 2 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| DL CCA model |  |  |  | As specified in clause D.7.2.1 |  |  |  |
| UL CCA model |  |  |  | As specified in clause D.7.2.2 |  |  |  |
| PCCA_DL for dynamic channel access Note 7,8 |  | Config 1, 2 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |
| PCCA_DL for semi-static channel access Note 6.8 |  | Config 1, 2 |  | PCCA_DL=0.9375 | PCCA_DL=0.9375 | PCCA_DL=0.9375 | PCCA_DL=0.9375 |
| PCCA_UL |  | Config 1, 2 |  | 1 | 1 | 1 | 1 |
| SSB configuration | Semi-static channel access | Config 1, 2 |  | SSB.1 CCA (As defined in A.3A.1) | SSB.1 CCA (As defined in A.3A.1) | SSB.1 CCA (As defined in A.3A.1) | SSB.1 CCA (As defined in A.3A.1) |
|  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3A.1) | SSB.2 CCA (As defined in A.3A.1) | SSB.2 CCA (As defined in A.3A.1) | SSB.2 CCA (As defined in A.3A.1) |
| Time offset with Cell 2 |  | Config 1, 2 | s | - | 3 | - | 3 |
| SMTC configuration |  | Config 1, 2 |  | SMTC.1 |  |  |  |
| DBT Window Configuration |  | Config 1, 2 |  | DBT.1 (As defined in A.12.1) |  |  |  |
| DL CCA model |  | Config 1, 2 |  | As specified in clause D.7.2.1 |  |  |  |
| UL CCA model |  | Config 1, 2 |  | As specified in clause D.7.2.2 |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 30kHz |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config 1, 2 | NR_CCA_FR1_I | dBm/15Khz | -94+TT |  | -110+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  | -109.5+TT |  |
| Note2 | Config 1, 2 | NR_CCA_FR1_I | dBm/SCS | -91+TT |  | -107.0+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  | -106.5+TT |  |
|  |  |  | dB | 2.46+TT | -5.97+TT | -2.01+TT | -3.54+TT |
|  |  |  | dB | 6+TT | 1+TT | 1+TT | 0+TT |
| SS-RSRPNote3 | Config 1, 2 | NR_CCA_FR1_I | dBm/SCS | -85+TT | -90+TT | -106.00+TT | -107.00+TT |
|  |  | NR_CCA_FR1_J |  |  |  | -105.50+TT | -106.50+TT |
| IoNote3 | Config 1, 2 | NR_CCA_FR1_I | dBm/ 38.16MHz | -51.99+TT |  | -70.82+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  | -70.32+TT |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 8: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |

Table 10.5.1.1.5-2: Absolute accuracy requirements for the reported values for EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest SS-RSRP reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Highest SS-RSRP reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1 |  |  |  |

Table 10.5.1.1.5-3: Relative accuracy requirements for the reported values for EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 3) | RSRP_x – FFS | RSRP_x – FFS |
| Highest reported value (Cell 3) | RSRP_x – FFS | RSRP_x + FFS |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 3) | RSRP_x – FFS | RSRP_x – FFS |
| Highest reported value (Cell 3) | RSRP_x - FFS | RSRP_x + FFS |
| RSRP_x is the reported value of Cell 2 |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 10.5.1.2	EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- SMTC config is still TBD

10.5.1.2.1	Test purpose

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.5.1.0.3 and 10.5.1.0.4 when both the serving cell and the target cell are subject to CCA.

10.5.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting inter-frequency measurements on shared channel access.

10.5.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.5.1.0.3 and 10.5.1.0.4 for absolute and relative accuracy, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.1.2.

10.5.1.2.4	Test description

Supported test configurations are shown in Table 10.5.1.2.4.1-1. Both absolute and relative accuracy of SS-RSRP inter-frequency measurements are tested by using the parameters in Table 10.5.1.2.5-1. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.6B. In all test cases, Cell 2 is the PSCell, and Cell 3 is the target cell. The inter-frequency measurements are supported by a measurement gap.

10.5.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.1.2.4.1-1.

Table 10.5.1.2.4.1-1: Supported test configurations for EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell

| Configuration | Description |
| --- | --- |
| 10.5.1.2-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.5.1.2-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.1.2.4.1-2.

Table 10.5.1.2.4.1-2: Initial conditions for EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.1.2.5-1.

2.	Message contents are defined in clause 10.5.1.2.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.1.2.4.2	Test procedure

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on a different frequency than the PSCell. Both Cell 2 and Cell 3 are subject to CCA and transmit SSBs in DBT windows according to DL CCA model. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 10.5.1.2.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform measurements in DBT windows in the target neighbouring frequency, as specified in section 10.5.1.2.4.3.

4.	The UE shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 10.5.1.2.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 3 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 10.5.1.2.5-2 or the UE fails to report the measurement value for Cell 3 [for FFS consecutive transmissions], the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 2-7 for each subtest defined in Table 10.5.1.2.5-1.

10.5.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.1.2.4.3-1: Common Exception messages for EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTER-FREQ MO Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.4-4 with Condition gapUE Table H.3.4-5 with Condition Pattern#0 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access |

Table 10.5.1.2.4.3-2: ReportConfigNR for EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

10.5.1.2.5	Test requirements

Table 10.5.1.2.5-1 defines the primary level settings including test tolerances for EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell. Table 10.5.1.2.5-2 and Table 10.5.1.2.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 10.5.1.2.5-1: Cell specific test parameters for EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell

| Parameter |  | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  | 1, 2 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel |  | 1, 2 | MHz | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| Gap pattern ID |  |  |  | 0 |  | 0 |  |
| Duplex mode |  | 1, 2 |  | TDD |  | TDD |  |
| TDD configuration |  | 1, 2 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| PDSCH Reference measurement channel |  | 1, 2 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | 1, 2 |  | CR.1.1 CCA | - | CR.1.1 CCA | - |
| Dedicated CORESET Reference Channel |  | 1, 2 |  | CCR.1.1 CCA | - | CCR.1.1 CCA | - |
| SSB configuration | Semi-static channel access | 1, 2 |  | SSB.1 CCA (As defined in A.3A.1) |  | SSB.1 CCA (As defined in A.3A.1) |  |
|  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3A.1) |  | SSB.2 CCA (As defined in A.3A.1) |  |
| OCNG Patterns |  | 1, 2 |  | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration |  | 1, 2 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1, 2 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 2 |  | 1, 2 | s | - | 3 | - | 3 |
| SMTC configuration |  | 1, 2 |  | TBD |  | TBD |  |
| DBT Window Configuration |  | 1, 2 |  | As defined in A.12.1 |  | As defined in A.12.1 |  |
| DL CCA model |  |  |  | As specified in clause D.7.2.1 |  |  |  |
| UL CCA model |  |  |  | As specified in clause D.7.2.2 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | 1, 2 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | NR_CCA_FR1_I | 1, 2 | dBm/15kHz | -94.65+TT |  | ( for Cell 3 +8dB)+TT | -111+TT |
|  | NR_CCA_FR1_J |  |  |  |  |  | -110.5+TT |
| Note2 | NR_CCA_FR1_I | 1, 2 | dBm/SSB SCS | -91.65+TT |  | ( for Cell 3 +8dB)+TT | -109.00+TT |
|  | NR_CCA_FR1_J |  |  |  |  |  | -108.50+TT |
|  |  | 1, 2 | dB | 10+TT | 10+TT | 13+TT | -3+TT |
| SS-RSRPNote3 | NR_CCA_FR1_I | 1, 2 | dBm/SCS | -81.65+TT |  | (RSRP for Cell 3 +25dB)+TT | -111.00+TT |
|  | NR_CCA_FR1_J |  |  |  |  |  | -110.50+TT |
| IoNote3 | R_CCA_FR1_I | 1, 2 | dBm/ 38.16MHz | -50.19+TT |  | (Io for Channel 3 +19.75dB)+TT | -75.19+TT |
|  | NR_CCA_FR1_J |  |  |  |  |  | -74.69+TT |
|  |  | 1, 2 | dB | 10+TT | 10+TT | 13+TT | -3+TT |
| Propagation condition |  | 1, 2 | - | AWGN |  | AWGN |  |
| Antenna configuration |  |  |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

Table 10.5.1.2.5-2: Absolute accuracy requirements for the reported values for EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest SS-RSRP reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Highest SS-RSRP reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1 |  |  |  |

Table 10.5.1.2.5-3: Relative accuracy requirements for the reported values for EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 3) | RSRP_x - FFS | RSRP_x - FFS |
| Highest reported value (Cell 3) | RSRP_x - FFS | RSRP_x + FFS |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 3) | RSRP_x - FFS | RSRP_x - FFS |
| Highest reported value (Cell 3) | RSRP_x - FFS | RSRP_x + FFS |
| RSRP_x is the reported value of Cell 2 |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 10.5.2	SS-RSRQ

#### 10.5.2.0	Minimum Conformance Requirements

##### 10.5.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell under CCA.

The accuracy requirements in Table 10.5.2.0.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.9 for a corresponding Band for each relevant SSB.

Table 10.5.2.0.1-1: SS-RSRQ intra-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2.5 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.29.1.1.

##### 10.5.2.0.2	Void

##### 10.5.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency under CCA that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.5.2.0.3-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.10 for a corresponding Band for each relevant SSB.

Table 10.5.2.0.3-1: SS-RSRQ inter-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2.5 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.30.1.1.

##### 10.5.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA

The relative accuracy of SS-RSRQ in inter-frequency case is defined as the RSRQ measured from one cell on a frequency compared to the RSRQ measured from another cell on a different frequency, with at least one of the two frequencies being under CCA.

The accuracy requirements in Table 10.5.2.0.4-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.9 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.5.2.0.4-1: SS-RSRQ inter-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.30.1.2.

#### 10.5.2.1	EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

10.5.2.1.1	Test purpose

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.5.2.0.1 when the serving and target cell are subject to CCA.

10.5.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC, supporting NR-U and [intra-frequency measurements on shared channel access].

10.5.2.1.3	Minimum conformance requirements

The minimum conformance requirement is specified in clause 10.5.2.0.1 for absolute RSRQ accuracy.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.2.1.

10.5.2.1.4	Test description

Supported test configurations are shown in Table 10.5.2.1.4.1-1. Absolute accuracy of SS-RSRQ intra-frequency measurements is tested by using the parameters in Table 10.5.2.1.5-1. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.6B. In all test cases, Cell 2 is the PSCell, and Cell 3 is the target cell.

10.5.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.2.1.4.1-1.

Table 10.5.2.1.4.1-1: Supported test configurations for EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell

| Configuration | Description |
| --- | --- |
| 10.5.2.1-1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.5.2.1-2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.2.1.4.1-2.

Table 10.5.2.1.4.1-2: Initial conditions for EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.2.1.5-1.

2.	Message contents are defined in clause 10.5.2.1.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.2.1.4.2	Test procedure

In this test case all cells are on the same carrier frequency. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in Table 10.5.2.1.5-1. In all test cases, Cell 2 is the PSCell and Cell 3 is the target cell.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 10.5.2.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform [intra-frequency measurements in DBT windows] as specified in section 10.5.2.1.4.3.

4.	The UE shall transmit an RRCReconfigurationComplete message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 3 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 10.5.2.1.5-2 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 10.5.2.1.5-1.

10.5.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.2.1.4.3-1: Common Exception messages for EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy on a CCA serving cell

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 10.5.2.1.4.3-2: ReportConfigNR for EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

10.5.2.1.5	Test requirements

Table 10.5.2.1.5-1 defines the primary level settings including test tolerances for EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy on a CCA serving cell. Table 10.5.2.1.5-2 defines the absolute accuracy requirements.

Table 10.5.2.1.5-1: Cell specific test parameters for EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell

| Parameter |  |  |  | Unit | Test 1 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  |  | freq1 |  |  |  |
| Duplex mode |  | Config 1, 2 |  |  | TDD |  |  |  |
| TDD configuration |  | Config 1, 2 |  |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  | Config 1, 2 |  | MHz | 40: NRB,c = 106 |  |  |  |
| BWP configuration |  | Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1, 2 |  |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS configuration |  | Config 1, 2 |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  |  | OP. 1 |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 2 |  | Config 1, 2 |  | s | - | 3 | - | 3 |
| SMTC configuration |  | Config 1, 2 |  |  | TBD |  |  |  |
| SSB configuration |  | Semi-static channel access | Config 1, 2 |  | SSB.1 CCA (As defined in A.3A.1.1) |  |  |  |
|  |  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3A.1.2) |  |  |  |
| PDSCH/PDCCH |  | Config 1, 2 |  | kHz | 30kHz |  |  |  |
| subcarrier spacing |  |  |  |  |  |  |  |  |
| DBT Window Configuration |  | Config 1, 2 |  |  | DBT.1 (As defined in A.12.1) |  |  |  |
| DL CCA model |  | Config 1, 2 |  |  | As specified in clause D.7.2.1 |  |  |  |
| UL CCA model |  | Config 1, 2 |  |  | As specified in clause D.7.2.2 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config1, 2 | NR_CCA_FR1_I |  | Bm/15kHz | -91+TT |  | -110+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  | -109.5+TT |  |
| Note2 | Config 1, 2 | NR_CCA_FR1_I |  | dBm/SC S | -88+TT |  | -107+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  | -106.5+TT |  |
|  |  |  |  | dB | -1.76+TT |  | -5.46+TT | -5.46+TT |
|  |  |  |  | dB | 3+TT | 3+TT | -4+TT | -4+TT |
| SS-RSRPNote3 | Config 1, 2 | NR_CCA_FR1_I |  | dBm/SCS | -85+TT | -85+TT | -111+TT | -111+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  | -110.5+TT | -110.5+TT |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I |  | dB | -14.77+TT | -14.77+TT | -17.34+TT | -17.34+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |
| IoNote3 | Config 1, 2 | NR_CCA_FR1_I |  | dBm/ 38.16MHz | -50+TT |  |  | -73.4+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -72.9+TT |
| Propagation condition |  |  |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3A.4.1. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |

Table 10.5.2.1.5-2: Absolute accuracy requirements for the reported values for EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell

| Normal Conditions | Test 1 All bands | Test 3 |  |
| --- | --- | --- | --- |
| Lowest SS-RSRQ reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Highest SS-RSRQ reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in clause 3A.4.1, Table 3A.4.1-2A |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 10.5.2.2	EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

- SMTC config is still TBD

10.5.2.2.1	Test purpose

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.5.2.0.3 and 10.5.2.0.4 when both the serving cell and the target cell are subject to CCA.

10.5.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC, supporting NR-U and [inter-freq measurements on shared channel access].

10.5.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.5.2.0.3 and 10.5.2.0.4 for absolute and relative accuracy, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.2.2.

10.5.2.2.4	Test description

Supported test configurations are shown in Table 10.5.2.2.4.1-1. Both absolute and relative accuracy of SS-RSRQ inter-frequency measurements are tested by using the parameters in Table 10.5.2.2.5-1. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.6B. In all test cases, Cell 2 is the PSCell, and Cell 3 is the target cell. The inter-frequency measurements are supported by a measurement gap.

10.5.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.2.2.4.1-1.

Table 10.5.2.2.4.1-1: Supported test configurations for EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with CCA serving cell and CCA target cell

| Configuration | Description |
| --- | --- |
| 10.5.2.2-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.5.2.2-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.2.2.4.1-2.

Table 10.5.2.2.4.1-2: Initial conditions for EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with CCA serving cell and CCA target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.2.2.5-1.

2.	Message contents are defined in clause 10.5.2.2.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.2.2.4.2	Test procedure

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on a different frequency than the PSCell. Both Cell 2 and Cell 3 are subject to CCA and transmit SSBs in DBT windows according to DL CCA model.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 10.5.2.2.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform [measurements in DBT windows] in the target neighbouring frequency, as specified in section 10.5.2.2.4.3.

4.	The UE shall transmit an RRCReconfigurationComplete message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 3 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 10.5.2.2.5-2 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 10.5.2.2.5-1.

10.5.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.2.2.4.3-1: Common Exception messages for EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with CCA serving cell and CCA target cell

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTER-FREQ MO Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.4-4 with Condition gapUE Table H.3.4-5 with Condition Pattern#0 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 10.5.2.2.4.3-2: ReportConfigNR for EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with CCA serving cell and CCA target cell

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

10.5.2.2.5	Test requirements

Table 10.5.2.2.5-1 defines the primary level settings including test tolerances for EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with CCA serving cell and CCA target cell. Table 10.5.2.2.5-2 and Table 10.5.2.2.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 10.5.2.2.5-1: Cell specific test parameters for EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with CCA serving cell and CCA target cell

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | Config 1, 2 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1, 2 |  | TDDConf.1.1 CCA |  |  |  |  |  |
| BWchannel |  | Config 1, 2 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1, 2 | MHz | 40: NRB,c = 106 |  |  |  |  |  |
| Gap pattern ID |  |  |  | 0 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  | SR.1.1 CCA |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 CCA |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  | Config 1, 2 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS configuration |  | Config 1, 2 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| Time offset with Cell 2 |  | Config 1, 2 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1, 2 |  | TBD |  |  |  |  |  |
| SSB configuration | Semi-static channel access | Config 1, 2 |  | SSB.1 CCA |  |  |  |  |  |
|  | Dynamic channel access |  |  | SSB.2 CCA |  |  |  |  |  |
| DBT Window Configuration |  | Config 1, 2 |  | DBT.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1, 2 | NR_CCA_FR1_I | dBm/15kHz | -86.27+TT | -86.27+TT | -113+TT | -113+TT | -112+TT | -112+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -111.5+TT | -111.5+TT |
| Note2 | Config 1, 2 | NR_CCA_FR1_I | dBm/SCS | -83.27+TT | -83.27+TT | -110+TT | -110+TT | -109+TT | -109+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -108.5+TT | -108.5+TT |
|  |  |  | dB | -1.75+TT | -1.75+TT | -1.75+TT | -1.75+TT | 3+TT | -1.75+TT |
|  |  |  | dB | -1.75+TT | -1.75+TT | -1.75+TT | -1.75+TT | 3+TT | -1.75+TT |
| SS-RSRPNote3 | Config 1, 2 | NR_CCA_FR1_I | dBm/SCS | -85.02+TT | -85.02+TT | -111.75+TT | -111.75+TT | -106+TT | -110.75+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -105.5+TT | -110.25+TT |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I | dB | -14.77+TT | -14.77+TT | -40.59+TT | -40.59+TT | -12.56+TT | -14.76+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  |
| IoNote3 | Config 1, 2 | NR_CCA_FR1_I | dBm/ 38.16MHz | -50+TT | -50+TT | -76.73+TT | -76.73+TT | -73.19+TT | -75.23+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -72.69+TT | -74.73+TT |
| Propagation condition |  |  |  | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3A.4.1. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |

Table 10.5.2.2.5-2: Absolute accuracy requirements for the reported values for EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with CCA serving cell and CCA target cell

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest SS-RSRQ reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Highest SS-RSRQ reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in clause 3A.4.1, Table 3A.4.1-2A |  |  |  |

Table 10.5.2.2.5-3: Relative accuracy requirements for the reported values for EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with CCA serving cell and CCA target cell

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 3) | RSRQ_x - FFS | RSRQ_x - FFS |
| Highest reported value (Cell 3) | RSRQ_x + FFS | RSRQ_x + FFS |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 3) | RSRQ_x - FFS | RSRQ_x - FFS |
| Highest reported value (Cell 3) | RSRQ_x + FFS | RSRQ_x + FFS |
| RSRQ_x is the reported value of Cell 2 |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 10.5.3	SS-SINR

#### 10.5.3.0	Minimum Conformance Requirements

##### 10.5.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell under CCA.

The accuracy requirements in Table 10.5.3.0.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.9 for a corresponding Band for each relevant SSB.

Table 10.5.3.0.1-1: SS-SINR intra-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3.0 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.31.1.1.

##### 10.5.3.0.2	Void

##### 10.5.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency under CCA that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.5.3.0.3-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.10 for a corresponding Band for each relevant SSB.

Table 10.5.3.0.3-1: SS-SINR inter-frequency absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3.0 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | 116.5 | -113.5 |  |  |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.32.1.1.

##### 10.5.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency compared to the SS-SINR measured from another cell on a different frequency, with at least one of the two frequencies being under CCA.

The accuracy requirements in Table 10.5.3.0.4-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.10 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.5.3.0.4-1: SS-SINR inter-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| 3.5 | 4 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 5: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.32.1.2.

#### 10.5.3.1	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

10.5.3.1.1	Test purpose

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.5.3.0.1 when the serving and target cell are subject to CCA.

10.5.3.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC, supporting NR-U and [intra-frequency measurements on shared channel access].

10.5.3.1.3	Minimum conformance requirements

The minimum conformance requirement is specified in clause 10.5.3.0.1 for absolute SS-SINR accuracy.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.3.1.

10.5.3.1.4	Test description

Supported test configurations are shown in Table 10.5.3.1.4.1-1. Absolute accuracy of SS-SINR intra-frequency measurements is tested by using the parameters in Table 10.5.3.1.5-1. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.6B. In all test cases, Cell 2 is the PSCell under CCA, and Cell 3 is the target cell under CCA.

10.5.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.3.1.4.1-1.

Table 10.5.3.1.4.1-1: Supported test configurations for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA

| Configuration | Description |
| --- | --- |
| 10.5.3.1-1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.5.3.1-2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.3.1.4.1-2.

Table 10.5.3.1.4.1-2: Initial conditions for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.3.1.5-1.

2.	Message contents are defined in clause 10.5.3.1.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.3.1.4.2	Test procedure

In this test case all cells are on the same carrier frequency. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in Table 10.5.3.1.5-1. In all test cases, Cell 2 is the PSCell with CCA and Cell 3 is the target cell with CCA. Two sub-tests (Test 1 and Test 2) are provided different Noc on Cells 2 and 3.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 10.5.3.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform [intra-frequency measurements in DBT windows] as specified in section 10.5.3.1.4.3.

4.	The UE shall transmit an RRCReconfigurationComplete message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 3 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 10.5.3.1.5-2 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 10.5.3.1.5-1.

10.5.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.3.1.4.3-1: Common Exception messages for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 10.5.3.1.4.3-2: ReportConfigNR for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

10.5.3.1.5	Test requirements

Table 10.5.3.1.5-1 defines the primary level settings including test tolerances for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on a CCA serving cell. Table 10.5.3.1.5-2 defines the absolute accuracy requirements.

Table 10.5.3.1.5-1: Cell specific test parameters for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| DL CCA model |  | Config 1,2 |  | As specified in clause D.7.2.1 |  |  |  |
| UL CCA model |  | Config 1,2 |  | As specified in clause D.7.2.2 |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - |
| Duplex mode |  | Config 1,2 |  | TDD |  |  |  |
| TDD configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| TRS Configuration |  | Config 1,2 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 CCA |  | SR1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1,2 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  | Config 1,2 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 2 |  | Config 1,2 | s | - | 3 | - | 3 |
| DBT Window Configuration |  | Config 1,2 |  | DBT.1 |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15kHz | -93+TT |  | -112+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  | -111.5+TT |  |
| Note2 | Config 1,2 | NR_CCA_FR1_I | dBm/SCS | -90+TT |  | -109+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  | -108.5+TT |  |
|  |  |  | dB | 0+TT | -3.19+TT | -5.46+TT | -5.46+TT |
|  |  |  | dB | 4.54+TT | 2.66+TT | -4+TT | -4+TT |
| SS-RSRPNote3 | Config 1,2 | NR_CCA_FR1_I | dBm/SCS | -85.46+TT | -87.34+TT | -113+TT | -113+TT |
|  |  | NR_CCA_FR1_J |  |  |  | -112.5+TT | -112.5+TT |
| SS-SINR Note3 |  | NR_CCA_FR1_I | dB | 0+TT | -3.19+TT | -5.46+TT | -5.46+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_CCA_FR1_I | dBm/ 38.16MHz | -51.41+TT |  | -75.41+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  | -74.91+TT |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3A.4.1. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

Table 10.5.3.1.5-2: Absolute accuracy requirements for the reported values for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest SS-SINR reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Highest SS-SINR reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in clause 3A.4.1, Table 3A.4.1-2A |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 10.5.3.2	EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

- SMTC config is still TBD

10.5.3.2.1	Test purpose

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.5.3.0.3 and 10.5.3.0.4 when both the serving cell and the target cell are subject to CCA.

10.5.3.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC, supporting NR-U and [inter-freq measurements on shared channel access].

10.5.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.5.3.0.3 and 10.5.3.0.4 for absolute and relative accuracy, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.3.2.

10.5.3.2.4	Test description

In this test case the two NR cells (i.e., Cell 2 and Cell 3) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table 10.5.3.2.4.1-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table 10.5.3.2.5-1. In all test cases, Cell 2 is the PSCell with CCA and Cell 3 is target cell with CCA. Cell 1 is the E-UTRA cell of which specific test parameters for this test case are specified in Table A.6B.

10.5.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.3.2.4.1-1.

Table 10.5.3.2.4.1-1: Supported test configurations for EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA

| Configuration | Description |
| --- | --- |
| 10.5.3.2-1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.5.3.2-2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.3.2.4.1-2.

Table 10.5.3.2.4.1-2: Initial conditions for EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.3.2.5-1.

2.	Message contents are defined in clause 10.5.3.2.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.3.2.4.2	Test procedure

Three sub-tests (Test 1, Test 2 and Test 3) are provided different Noc on Cells 2 and 3. In all test cases, Cell 2 is the PSCell under CCA, and Cell 3 is the target cell under CCA. The inter-frequency measurements are supported by a measurement gap.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 10.5.3.2.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform [measurements in DBT windows] in the target neighbouring frequency, as specified in section 10.5.3.2.4.3.

4.	The UE shall transmit an RRCReconfigurationComplete message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 3 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 10.5.3.2.5-2 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 10.5.3.2.5-1.

10.5.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.3.2.4.3-1: Common Exception messages for EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTER-FREQ MO Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.4-4 with Condition gapUE Table H.3.4-5 with Condition Pattern#0 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 10.5.3.2.4.3-2: ReportConfigNR for EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

10.5.3.2.5	Test requirements

Table 10.5.3.2.5-1 defines the primary level settings including test tolerances for EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy with CCA serving cell and CCA target cell. Table 10.5.3.2.5-2 and Table 10.5.3.2.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 10.5.3.2.5-1: Cell specific test parameters for EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| DL CCA model |  | Config 1,2 |  | As specified in clause D.7.2.1 |  |  |  |  |  |
| UL CCA model |  | Config 1,2 |  | As specified in clause D.7.2.2 |  |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - | 1.0 | - |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - | 0.75 | - |
| Duplex mode |  | Config 1,2 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| Gap pattern ID |  |  |  | 0 | - | 0 | - | 0 | - |
| TRS configuration |  | Config 1,2 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 CCA |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1,2 |  | CR.1.1 CCA |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  | Config 1,2 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| Time offset with Cell 2 |  | Config 1,2 | s | - | 3 | - | 3 | - | 3 |
| DBT Window configuration |  | Config 1,2 |  | DBT.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15kHz | -88+TT |  | -108.5+TT |  | -115.5+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -116+TT |  |
| Note2 | Config 1,2 | NR_CCA_FR1_I | dBm/SCS | -85+TT |  | -105.5+TT |  | -112.5+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -113+TT |  |
|  |  |  | dB | -1.75+TT |  | 20+TT |  | -4.0+TT |  |
|  |  |  | dB | -1.75+TT |  | 20+TT |  | -4.0+TT |  |
| SS-RSRPNote3 | Config 1,2 | NR_CCA_FR1_I |  | -86.75+TT |  | -85.5+TT |  | -116.5+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -116+TT |  |
| SS-SINR Note3 |  | NR_CCA_FR1_I | dB | -1.75+TT |  | 20+TT |  | -4.0+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_CCA_FR1_I | dBm/ 38.16MHz | -51.73+TT |  | -54.41+TT |  | -80+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -79.5+TT |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3A.4.1. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |

Table 10.5.3.2.5-2: Absolute accuracy requirements for the reported values for EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SS-SINR reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Highest SS-SINR reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in clause 3A.4.1, Table 3A.4.1-2A |  |  |  |  |

Table 10.5.3.2.5-3: Relative accuracy requirements for the reported values for EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 3) | SINR_x - FFS | SINR_x - FFS | SINR_x - FFS |
| Highest reported value (Cell 3) | SINR_x + FFS | SINR_x + FFS | SINR_x + FFS |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 3) | SINR_x - FFS | SINR_x - FFS | SINR_x - FFS |
| Highest reported value (Cell 3) | SINR_x + FFS | SINR_x + FFS | SINR_x + FFS |
| SINR_x is the reported value of Cell 2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 10.5.3.3	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

10.5.3.3.1	Test purpose

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.5.3.0.1 when the serving and target cell are subject to CCA.

10.5.3.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC, supporting NR-U and [intra-frequency measurements on shared channel access].

10.5.3.3.3	Minimum conformance requirements

The minimum conformance requirement is specified in clause 10.5.3.0.1 for absolute SS-SINR accuracy.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.3.1.

10.5.3.3.4	Test description

Supported test configurations are shown in Table 10.5.3.3.4.1-1. Absolute accuracy of SS-SINR intra-frequency measurements is tested by using the parameters in Table 10.5.3.3.5-1. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.6B. In all test cases, Cell 2 is the PSCell with CCA, Cell 3 is the SCell with CCA, and Cell 4 is the target cell with CCA.

10.5.3.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.3.3.4.1-1.

Table 10.5.3.3.4.1-1: Supported test configurations for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA

| Configuration | Description |
| --- | --- |
| 10.5.3.3-1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10.5.3.3-2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.3.3.4.1-2.

Table 10.5.3.3.4.1-2: Initial conditions for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.3.3.5-1.

2.	Message contents are defined in clause 10.5.3.3.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 and Cell 3 are the NR cell (PSCell) and NR SCell respectively, with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 4 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.3.3.4.2	Test procedure

In this test case Cell 2 (PSCell) in on frequency 1 while Cell 3 (SCell) and Cell 4 (target cell) which are intra-frequency neighbors, are on frequency 2. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in Table 10.5.3.3.5-1. Cell 2 is the PSCell with CCA, Cell 3 is the SCell with CCA, and Cell 4 is the target cell with CCA. Two sub-tests (Test 1 and Test 2) are provided different Noc on Cells 2, 3 and 4.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall configure SCell (Cell 3) on the NR-U SCC as per TS 38.508-1 [14] [clause 7.5.2], with the message content exceptions defined in clause 10.5.3.3.4.3. NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Cell 3 is an activated SCell and is scheduled throughout the test as specified in Table 10.5.3.3.5-1.

3.	Set the parameters according to Table 10.5.3.3.5-1 as appropriate.4.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform [SCC intra-frequency measurements in DBT windows] as specified in section 10.5.3.3.4.3.

5.	The UE shall transmit an RRCReconfigurationComplete message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration.

6.	The UE shall transmit periodical MeasurementReport messages.

7.	After 10s wait from Step 6, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 4 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 10.5.3.3.5-2 or the UE fails to report the measurement value for Cell 4, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

8.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

9.	Repeat steps 1-8 for each subtest defined in Table 10.5.3.3.5-1.

10.5.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.3.3.4.3-1: Common Exception messages for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 10.5.3.3.4.3-2: RRCReconfiguration (step 2, SCell addition) for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA

| Derivation Path: TS 38.508-1 [14], [Table 4.6.1-13] with condition [NR_MEAS and SCell_add] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT | Measurements configuration | NR_MEAS |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-SCell(n) | n is number of SCC to be added | SCell_add |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.5.3.3.4.3-3: ReportConfigNR for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

10.5.3.3.5	Test requirements

Table 10.5.3.3.5-1 defines the primary level settings including test tolerances for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on a CCA serving SCell. Table 10.5.3.3.5-2 defines the absolute accuracy requirements.

Table 10.5.3.3.5-1: Cell specific test parameters for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 / Cell 3 | Cell 4 | Cell 2 / Cell 3 |  | Cell 4 |
| SSB ARFCN |  |  |  | freq1 for Cell 2 freq2 for Cell 3 | freq2 | freq1 for Cell 2 freq2 for Cell 3 |  | freq2 |
| DL CCA model |  | Config 1,2 |  | As specified in clause D.7.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2 |  | As specified in clause D.7.2.2 |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - |  |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - |  |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - |  |
| Duplex mode |  | Config 1,2 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |
| TRS Configuration |  | Config 1,2 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 CCA |  | SR1.1 CCA |  |  |
| RMSI CORESET Reference Channel |  | Config 1,2 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,2 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |
| Time offset with Cell 2 |  | Config 1,2 | s | 3 (for Cell 3) | 3 | 3 (for Cell 3) |  | 3 |
| DBT Window Configuration |  | Config 1,2 |  | DBT.1 |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15kHz | -93+TT |  | -112+TT |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -111.5+TT |  |  |
| Note2 | Config 1,2 | NR_CCA_FR1_I | dBm/SCS | -90+TT |  | -109+TT |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -108.5+TT |  |  |
|  |  |  | dB | 0+TT | -3.19+TT | -5.46+TT |  | -5.46+TT |
|  |  |  | dB | 4.54+TT | 2.66+TT | -4+TT |  | -4+TT |
| SS-RSRPNote3 | Config 1,2 | NR_CCA_FR1_I | dBm/SCS | -85.46+TT | -87.34+TT | -113+TT |  | -113+TT |
|  |  | NR_CCA_FR1_J |  |  |  | -112.5+TT |  | -112.5+TT |
| SS-SINR Note3 |  | NR_CCA_FR1_I | dB | 0+TT | -3.19+TT | -5.46+TT |  | -5.46+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_CCA_FR1_I | dBm/ 38.16MHz | -51.41+TT |  | -75.41+TT |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -74.91+TT |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3A.4.1. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |

Table 10.5.3.3.5-2: Absolute accuracy requirements for the reported values for EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest SS-SINR reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Highest SS-SINR reported value | FFS | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
|  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in clause 3A.4.1, Table 3A.4.1-2A |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

# 11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1

Editor’s note: Test cases for NR SA with NR PCell under CCA and SCell under CCA are also included here.

## 11.1	RRC_IDLE state mobility

FFS

## 11.2	RRC_CONNECTED state mobility

### 11.2.1

### 11.2.2	RRC connection mobility control

#### 11.2.2.1	RRC re-establishment

#### 11.2.2.2	Random Access

#### 11.2.2.3	RRC connection release with redirection

##### 11.2.2.3.0	Minimum conformance requirements

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell subject to CCA within Tconnection_release_redirect_NR_CCA.

The time delay (Tconnection_release_redirect_NR_CCA) is the time between the end of the last slot containing the RRC command, “RRCRelease” (TS 38.331 [2]) on the NR PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR_CCA) shall be less than:

Tconnection_release_redirect_NR_CCA = TRRC_procedure_delay + Tidentify-NR_CCA + TSI-NR_CCA + TRACH_CCA

The target NR cell shall be considered detectable when for each relevant SSB, the side conditions should be met that,

-	the conditions of SSB_RP and SSB Ês/Iot as defined in Annex B.2.5 of TS 38.133 for a corresponding NR Band are fulfilled.

-	TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCRelease” as defined in clause 6.2.2 of TS 38.331 [2].

-	Tidentify-NR_CCA: It is the time to identify the target NR cell and is defined as:

-	Tidentify-NR_CCA = TPSS/SSS-sync + Tmeas; TPSS/SSS-sync is the cell search time and Tmeas is the measurement time due to cell selection criteria evaluation.

-	For FR1 target NR cell: Tidentify-NR_CCA = MAX (680 ms, (L1+11)  Trs);

-	where L1 is the number of SMTC occasions not available at the UE due to DL CCA failures  and L1´ is the number of SMTC occasion groups not available at the UE due to DL CCA failures. An SMTC occasion group consists of N consecutive SMTC occasions. An SMTC occasion group is not available, when at least one SMTC occasion in the group is not transmitted by the gNB. N is equal to 12. If L1 > L1,max or L1´ > L1,max then the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1]; where L1,max is defined in Table 11.2.2.3.0-1.

-	TSI-NR_CCA: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released.

-	TRACH_CCA: It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell:

-	TRACH_CCA = (1+L2)TSSB,RO + 10 ms TPRACH; where:

-	L2 is the consecutive number of SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failures. L2 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. L3 = 0 if ra-ChannelAccess-r17 is not configured in FR2-2.

-	TSSB,RO is the SSB to PRACH occasion association period as defined in the table 8.1-1 of TS 38.213 [3].

-	The value of L2 is limited by PREAMBLE_TRANSMISSION_COUNTER, which is increased when PRACH occasion is unavailable for PRACH transmission due to UL CCA failure as specified in TS 38.321 [7]. The UE behaviour when PREAMBLE_TRANSMISSION_COUNTER reaches the preambleTransMax is specified in TS 38.321 [7].

-	Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

-	the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms;

-	otherwise, there is no requirement if the SSB transmission periodicity is larger than 20ms.

Table 11.2.2.3.0-1: Maximum allowed number of missed SMTC occasions during cell identification

| SMTC periodicity (Trs) [ms] | Maximum allowed number of missed SMTC occasions (L1,max) |
| --- | --- |
| Trs ≤ 40 | 8 |
| Trs > 40 | 4 |

The normative reference for this requirement is TS 38.133 [6] clause 6.2.3.2.3.

##### 11.2.2.3.1	NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA

Editor's Note:

-	MU and TT analysis is incomplete

-	Test procedure and message contents are incomplete

-	Test applicability in TS 38.522 is missing

11.2.2.3.1.1	Test purpose

The purpose of this test is to verify that UE shall perform RRC connection release with redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA within limits.

11.2.2.3.1.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR-U.

11.2.2.3.1.3	Minimum conformance requirement

The minimum requirements are specified in clause 11.2.2.3.0

The normative reference for this requirement is TS 38.133 [6] clause A.11.2.2.3.1.

11.2.2.3.1.4	Test description

11.2.2.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.2.2.3.1.4.1-1.

Table 11.2.2.3.1.4.1-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 11.2.2.3.1.4.1-2.

Table 11.2.2.3.1.4.1-2: NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.2.2.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 11.2.2.3.1.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 11.2.2.3.1.4.1-3.

Table 11.2.2.3.1.4.1-3: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≥ Tconnection_release_redirect_NR_CCA | Tconnection_release_redirect_NR_CCA is defined in clause 6.2.3.2.3 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

11.2.2.3.1.4.2	Test Procedure

TBD

11.2.2.3.1.4.3	Message Contents

TBD

11.2.2.3.1.5	Test Requirement

Table 11.2.2.3.1.5-1 defines the cell-specific test parameters for redirection from NR to NR test case.

Table 11.2.2.3.1.5-1: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 1 |  | 1 |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 1 |  | 1 |  |
| LCCA_DL Note 7 |  |  |  | N/A |  | 8 |  |
| WCCA_DL Note 7 |  |  | ms | N/A |  | Tidentify-NR_CCA |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 CCA |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 CCA |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| DBT configuration |  |  |  | DBT.1 |  |  |  |
| SSB configuration for semi-static channel access Note 4, 6 |  | Config 1 |  | SSB.1 CCA |  |  |  |
| SSB configuration for dynamic channel access Note 5, 6 |  | Config 1 |  | SSB.2 CCA |  |  |  |
| ssb-PositionQCL |  | Config 1 |  | [1] |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98+TT |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -95+TT |  |  |  |
|  |  |  | dB | 4+TT | 4+TT | -infinity | 4+TT |
|  |  |  | dB | 4+TT | 4+TT | -infinity | 4+TT |
| IoNote3 | Config 1 |  | dBm/ 38.16MHz | -58.49+TT | -58.49+TT | -63.94 | -58.49+TT |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 7: As defined in clause 6.2.3.2.3 for Trs ≤ 40 ms. |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than Tconnection_release_redirect_NR_CCA ms from the beginning of time period T2, where Tconnection_release_redirect_NR_CCA is defined in clause 11.2.2.3.0.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR_CCA = TRRC_procedure_delay + Tidentify-NR_CCA + TSI-NR_CCA + TRACH_CCA,

where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR_CCA = MAX (680 ms, (L1+11)  20 ms) in the test.

TSI-NR_CCA = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH_CCA is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell.

L1 is the number of SMTC occasions not available at the UE due to DL CCA failures. The test equipment ensures that number of L1 in target cell does not exceed L1,max using the configured LCCA_DL as in clause A.3.26.2.1 of TS 38.133[6].

##### 11.2.2.3.2	NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA

Editor's Note:

-	MU and TT analysis is incomplete

-	Test procedure and message contents are incomplete

-	Test applicability in TS 38.522 is missing

11.2.2.3.2.1	Test purpose

The purpose of this test is to verify that UE shall perform RRC connection release with redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA within limits.

11.2.2.3.2.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR-U.

11.2.2.3.2.3	Minimum conformance requirement

The minimum requirements are specified in clause 11.2.2.3.0

The normative reference for this requirement is TS 38.133 [6] clause A.11.2.2.3.2.

11.2.2.3.2.4	Test description

11.2.2.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.2.2.3.2.4.1-1.

Table 11.2.2.3.2.4.1-1: Redirection from NR to NR test configurations

| Configuration | Source cell without CCA | Target cell with CCA |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Configure the test equipment and the DUT according to the parameters in Table 11.2.2.3.2.4.1-2.

Table 11.2.2.3.2.4.1-2: NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.2.2.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 11.2.2.3.2.4.3

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 11.2.2.3.2.4.1-3.

Table 11.2.2.3.2.4.1-3: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier without CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≥ Tconnection_release_redirect_NR_CCA | Tconnection_release_redirect_NR_CCA is defined in clause 6.2.3.2.3 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

11.2.2.3.2.4.2	Test Procedure

TBD

11.2.2.3.2.4.3	Message Contents

TBD

11.2.2.3.2.5	Test Requirement

Table 11.2.2.3.2.5-1 defines the cell-specific test parameters for redirection from NR to NR test case.

Table 11.2.2.3.2.5-1: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  |  | N/A |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  |  | N/A |  | PCCA_DL=0.9375 | PCCA_DL=0.9375 |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  |  | N/A |  | 1 | 1 |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  |  | N/A |  | 1 | 1 |  |
| LCCA_DL Note 7 |  |  |  | N/A |  | 8 |  |  |
| WCCA_DL Note 7 |  |  | ms | N/A |  | Tidentify-NR_CCA |  |  |
| Duplex mode |  | Config 1 |  | FDD |  | TDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  | TDDConf.1.1 CCA |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 CCA |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  | TDDConf.1.1 CCA |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | SR.1.1 CCA |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 CCA |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR.1.1 CCA |  |  |
| RMSI CORESET RMC configuration |  | Config 1 |  | CR.1.1 FDD |  | CR.1.1 CCA |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 CCA |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.1.1 CCA |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 FDD |  | CCR.1.1 CCA |  |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 CCA |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.1.1 CCA |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration | Semi-static channel acces | Config 1,2 |  | SSB.1 FR1 |  | SSB.1 CCA |  |  |
|  | Dymamic channel acces | Config 3 |  | SSB.2 FR1 |  | SSB.2 CCA |  |  |
|  | Semi-static channel acces | Config 1,2 |  | SSB.1 FR1 |  | SSB.1 CCA |  |  |
|  | Dymamic channel acces | Config 3 |  | SSB.2 FR1 |  | SSB.2 CCA |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  | SMTC.2 FR1 |  |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  | 30 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  | 30 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA in Table A.3.8A.2.1-1 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98+TT |  |  |  |  |
| Note2 |  | Config 1,2 | dBm/SCS | -98+TT |  | -95+TT |  |  |
|  |  | Config 3 |  | -95+TT |  |  |  |  |
|  |  |  | dB | 4+TT | 4+TT | -infinity |  | 4+TT |
|  |  |  | dB | 4+TT | 4+TT | -infinity |  | 4+TT |
| IoNote3 |  | Config 1,2 | dBm/9.36MHz | -64.59+TT | -64.59+TT | N/A |  | N/A |
|  |  | Config 3 | dBm/38.16MHz | -58.49+TT | -58.49+TT | -63.94+TT |  | -58.49+TT |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 7: As defined in clause 6.2.3.2.3 for Trs ≤ 40 ms. |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than Tconnection_release_redirect_NR_CCA ms from the beginning of time period T2, where Tconnection_release_redirect_NR_CCA is defined in clause 11.2.2.3.0.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR_CCA = TRRC_procedure_delay + Tidentify-NR_CCA + TSI-NR_CCA + TRACH_CCA,

where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR_CCA = MAX (680 ms, (L1+11)  20 ms) in the test.

TSI-NR_CCA = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH_CCA is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TRACH_CCA = (1+L2)´TSSB,RO + 10 ms; where TSSB,RO = 10 ms for FR1 PRACH configuration 1.

L1 is the number of SMTC occasions not available at the UE due to DL CCA failures. The test equipment ensures that number of L1 in target cell does not exceed L1,max using the configured LCCA_DL as in clause A.3.26.2.1 of TS 38.133[6];

L2 is the consecutive number of SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failures. L2 = 0 in the test.

The total delay, Tconnection_release_redirect_NR_CCA, shall be less than 1410 + MAX (680, (L1+11)´20) ms.

## 11.3	Timing

FFS

## 11.4	Signalling characteristics

### 11.4.1	Radio link monitoring

#### 11.4.1.0	General

Same as in clause 10.3.1.0.

##### 11.4.1.0.1	Minimum conformance requirements for SSB based radio link monitoring under CCA

Same as in clause 10.3.1.0.1.

11.4.1.0.1.1	Minimum requirement for SSB based Radio Link Monitoring under CCA

Same as in clause 10.3.1.0.1.1.

11.4.1.0.1.2	Measurement restrictions for SSB based RLM under CCA

Same as in clause 10.3.1.0.1.2.

#### 11.4.1.1	Requirements for determining UE in-sync or out-of-sync status

In the test cases specified in clause 11.4.1, any uplink signal transmitted by the UE is used for detecting the in-/out-of-sync state of the UE. In terms of measurement, the uplink signal is verified based on the UE output power:

-	UE output power higher than Transmit OFF power -50 dBm (as defined in TS 38.101-1 [2]) means uplink signal.

-	UE output power equal to or less than Transmit OFF power -50 dBm (as defined in TS 38.101-1 [2]) means no uplink signal.

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

The normative reference for this requirement is TS 38.133 [6] clause A.11.4.1.1.

#### 11.4.1.2	NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

Editor's Note:

-	MU and TT analysis is incomplete

-	Message contents may need to be updated

-	Call setup and test procedure may need to be updated

-	Applicability may need to be updated

-	Statistical analysis to determine test case verdict is FFS

-	Some test parameters are still in brackets

11.4.1.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out-of-sync and in-sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 PCell radio link monitoring requirements in clause 11.4.1.0.

11.4.1.2.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR-U and supporting UL and SSB-based RLM on shared channel access.

11.4.1.2.3	Minimum conformance requirement

The minimum requirements are specified in clause 11.4.1.0.1. The measurement restrictions for SSB based RLM under CCA are specified in clause 11.4.1.0.2. DRX configuration is not used for this test.

The normative reference for this requirement is TS 38.133 [6] clause A.11.4.1.2.

11.4.1.2.4	Test description

In the test, UE is configured to perform RLM based on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in Table 11.4.1.2.4.1-1. The test parameters are given in Tables 11.4.1.2.4.1-3, 11.4.1.2.5-1, and 11.4.1.2.5-2 below. There is one cell (Cell 1), which is the active NR cell in FR1, in the test. Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

The test consists of three successive time periods, with time duration of T1, T2 and T3, respectively. Figure 11.4.1.2.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE transmits according to UL CCA model. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40 ms) in the test.

Figure 11.4.1.2.4-1: SNR variation for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

11.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.4.1.2.4.1-1.

Table 11.4.1.2.4.1-1: Supported test configurations for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Configuration | Description |
| --- | --- |
| 11.4.1.2-1 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Configure the test equipment and the DUT according to the parameters in Table 11.4.1.2.4.1-2.

Table 11.4.1.2.4.1-2: Initial conditions for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 11.4.1.2.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 11.4.1.2.4.1-3.

Table 11.4.1.2.4.1-3: General test parameters for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| DL CCA model |  |  |  | As specified in clause D.7.2.1 |
| UL CCA model |  |  |  | As specified in clause D.7.2.2 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1 |  | [DLBWP.0.1] |
| DL dedicated BWP configuration |  | Config 1 |  | [DLBWP.1.1] |
| UL initial BWP configuration |  | Config 1 |  | [ULBWP.0.1] |
| UL dedicated BWP configuration |  | Config 1 |  | [ULBWP.1.1] |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |
| SSB configuration for semi-static channel accessNote 3, 5 |  | Config 1 |  | SSB.1 CCA |
| SSB configuration for dynamic channel accessNote 4, 5 |  | Config 1 |  | SSB.2 CCA |
| DBT window configuration |  | Config 1 |  | DBT.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 30 kHz |
| PRACH Configuration |  | Config 1 |  | FR1 PRACH configuration 1 under CCA |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | [OP.1] |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | [1-0] |
|  | Number of Control OFDM symbols |  |  | [2] |
|  | Aggregation level |  | CCE | [8] |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | [4] |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | [4] |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | [6] |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | [CSI-RS.2.1 TDD] |
| CSI-RS for tracking |  | Config 1 |  | [TRS.1.2 TDD] |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 1.04 |
| T3 |  |  | s | 1.04 |
| D1 |  |  | s | 1 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

11.4.1.2.4.2	Test Procedure

The test consists of a single NR cell (PCell). Prior to the start of the time duration T1, the UE shall be fully synchronized to the PCell. The UE shall be configured for periodic CSI reporting in PUCCH format 2 with a reporting periodicity as mentioned in the above table 11.4.1.2.4.1-3.

1.	Ensure the UE is in [State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On] according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	The SS sends an RRCReconfiguration message to the UE to configure inter-frequency measurement with the corresponding gap pattern and periodic CSI reporting.

3.	The UE sends an RRCReconfigurationComplete message.

4.	Set the parameters of NR Cell according to T1 in Table 11.4.1.2.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 11.4.1.2.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 11.4.1.2.5-1. T3 starts.

7.	If the SS:

a)	detects uplink power equal to or higher than minimum output power, according to section 11.4.1.1, in each subframe configured for CSI transmission (according to configured CSI periodicity on [PUCCH format 2]) during the period from time point A to time point B; and

b)	does not detect any uplink power higher than OFF power, according to section 11.4.1.1, from time point C (D1 after the start of T3) until T3 expires,	the number of successful tests is increased by one.

Otherwise, the number of failed tests is increased by one and proceed with step 10.

8.	When T3 expires the SS shall change the SNR value to T1 as specified in Table 11.4.1.2.5-1.

9.	If the UE has re-established the connection within [2] seconds, proceed with step 11. Otherwise, proceed with step 10.

10.	The SS shall switch off and then on the UE and then proceed with step 1.

11.	Repeat steps 1-9 until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

11.4.1.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 11.4.1.2.4.3-1: Common Exception messages for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier centre of [High] range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-6 with conditions gapUE and RLM Table H.3.1-8 with Condition SSB RLM Table H.3.5-4 Table H.3.5-9 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.4.1.2.4.3-2: RLF-TimersAndConstants for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

11.4.1.2.5	Test Requirement

Table 11.4.1.2.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA.

Table 11.4.1.2.5-1: Cell-specific test parameters for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| DL CCA probability PCCA_DL |  | Note 6,8 |  | PCCA_DL=0.9375 |  |  |
|  |  | Note 7,8 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| UL CCA probability PCCA_UL |  |  |  | 1 |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |
| SNRNote 3,4 on RLM-RS | Config 1 |  | dB | 1+TT | [-7]+TT | -15+TT |
| SNR on other channels and signals | Config 1 |  | dB | 1+TT |  |  |
|  | Config 1 |  | dBm/SCS | -95+TT |  |  |
| Propagation condition |  |  |  | TDL-C 300 ns 100 Hz |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT windows. NOTE 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3, respectively, in Figure 11.4.1.2.4-1. NOTE 5: The SNR values are specified for testing a UE which supports 2 RX on at least one band. For testing of a UE which supports 4 RX on all bands, the SNR during T3 is D.4A. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |

Table 11.4.1.2.5-2: Measurement gap configuration for NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Field | Test 1 |  |
| --- | --- | --- |
|  | Value |  |
| gapOffset | 0 |  |
| NOTE: Ensure that RLM RS is partially overlapped with measurement gap0 |  |  |

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

-	During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

-	The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

#### 11.4.1.3	NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

Editor's Note:

-	MU and TT analysis is incomplete

-	Message contents may need to be updated

-	Call setup and test procedure may need to be updated

-	Applicability may need to be updated

-	Statistical analysis to determine test case verdict is FFS

-	Some test parameters are still in brackets

11.4.1.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out-of-sync and in-sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 PCell radio link monitoring requirements in clause 11.4.1.0.

11.4.1.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR-U and supporting UL and SSB-based RLM on shared channel access.

11.4.1.3.3	Minimum conformance requirement

The minimum requirements are specified in clause 11.4.1.0.1. The measurement restrictions for SSB based RLM under CCA are specified in clause 11.4.1.0.2. DRX configuration is not used for this test.

The normative reference for this requirement is TS 38.133 [6] clause A.11.4.1.3.

11.4.1.3.4	Test description

In the test, UE is configured to perform RLM based on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table 11.4.1.3.4.1-1. The test parameters are given in Tables 11.4.1.3.4.1-3 and 11.4.1.3.5-1. There is one cell (Cell 1), which is the active NR cell in FR1, in the test. Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5, respectively. Figure 11.4.1.3.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE transmits according to UL CCA model.

Figure 11.4.1.3.4-1: SNR variation for NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

11.4.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.4.1.3.4.1-1.

Table 11.4.1.3.4.1-1: Supported test configurations for NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Configuration | Description |
| --- | --- |
| 11.4.1.3-1 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Configure the test equipment and the DUT according to the parameters in Table 11.4.1.3.4.1-2.

Table 11.4.1.3.4.1-2: Initial conditions for NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.4.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 11.4.1.3.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 11.4.1.3.4.1-3.

Table 11.4.1.3.4.1-3: General test parameters for NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| DL CCA model |  |  |  | As specified in clause D.7.2.1 |
| UL CCA model |  |  |  | As specified in clause D.7.2.2 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1 |  | [DLBWP.0.1] |
| DL dedicated BWP configuration |  | Config 1 |  | [DLBWP.1.1] |
| UL initial BWP configuration |  | Config 1 |  | [ULBWP.0.1] |
| UL dedicated BWP configuration |  | Config 1 |  | [ULBWP.1.1] |
| TDD Configuration |  | Config 1 |  | TDDConf.1.1 CCA |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |
| SSB configuration for semi-static channel accessNote 3, 5 |  | Config 1 |  | SSB.1 CCA |
| SSB configuration for dynamic channel accessNote 4,5 |  | Config 1 |  | SSB.2 CCA |
| DBT window configuration |  | Config 1 |  | DBT.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 30 kHz |
| PRACH Configuration |  | Config 1 |  | FR1 PRACH configuration 1 under CCA |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | [OP.1] |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  | [1-0] |
|  | Number of Control OFDM symbols |  |  | [2] |
|  | Aggregation level |  | CCE | [4] |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | [0] |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | [0] |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | [6] |
| Out of sync transmission parameters | DCI format |  |  | [1-0] |
|  | Number of Control OFDM symbols |  |  | [2] |
|  | Aggregation level |  | CCE | [8] |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | [4] |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | [4] |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | [6] |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N/A |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 2000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking | Config 1 |  |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.52 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 2.04 |
| D1 |  |  | s | 2 |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. NOTE 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

11.4.1.3.4.2	Test Procedure

The test consists of a single NR cell (PCell). Prior to the start of the time duration T1, the UE shall be fully synchronized to the PCell. The UE shall be configured for periodic CSI reporting in PUCCH format 2 with a reporting periodicity as mentioned in the above table 11.4.1.3.4.1-3.

1.	Ensure the UE is in [State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On] according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	The SS sends an RRCReconfiguration message to the UE to configure periodic CSI reporting.

3.	The UE sends an RRCReconfigurationComplete message.

4.	Set the parameters of NR Cell according to T1 in Table 11.4.1.3.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 11.4.1.3.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 11.4.1.3.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 11.4.1.3.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 11.4.1.3.5-1. T5 starts.

9.	If the SS detects uplink power equal to or higher than minimum output power, according to section 11.4.1.1, in each subframe configured for CSI transmission (according to configured CSI periodicity on [PUCCH format 2]) during the period from time point A to time point F (D1 seconds after the start of time duration T5), the number of successful tests is increased by one. After the expiration of T5, the SS shall proceed with step 11 for the next iteration, or with step 10.

Otherwise, the number of failed tests is increased by one and proceed with step 10.

10.	The SS shall switch off and then on the UE and then proceed with step 11.

11.	Repeat steps 1-9 until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

11.4.1.3.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 11.4.1.3.4.3-1: Common Exception messages for NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition L3 FILTERING NEEDED, Table H.3.1-3 with Condition RLM Table H.3.1-4 with A3-offset = 0 Table H.3.1-8 with Condition SSB RLM Table H.3.5-4 Table H.3.5-9 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.4.1.3.4.3-2: RLF-TimersAndConstants for NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms2000 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

11.4.1.3.5	Test Requirement

Table 11.4.1.3.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA.

Table 11.4.1.3.5-1: Cell-specific test parameters for NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA_DL |  | Note 6,8 |  | PCCA_DL=0.9375 |  |  |  |  |
|  |  | Note 7,8 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| UL CCA probability PCCA_UL |  |  |  | 1 |  |  |  |  |
| LCCA_DL |  |  |  | 7 |  |  |  |  |
| WCCA_DL |  |  | ms | TEvaluate_in_SSB,CCANOTE 9 |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 |  | dB | 1+TT | [-7]+TT | [-15] +TT | [-4.5] +TT | 1+TT |
| SNR on other channels and signals | Config 1 |  | dB | 1+TT |  |  |  |  |
|  | Config 1 |  | dBm/SCS | -95+TT |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT windows. NOTE 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure 11.4.1.3.4-1. NOTE 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4 RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4A. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. NOTE 9: As defined in Table 11.4.1.0.1-1. |  |  |  |  |  |  |  |  |

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

-	During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

### 11.4.2	Interruptions

FFS

### 11.4.3	SCell activation and deactivation delay

FFS

### 11.4.4	Beam failure detection and link recovery procedures

#### 11.4.4.0	General

Same as in clause 10.3.4.0.

##### 11.4.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA

Same as in clause 10.3.4.0.1.

11.4.4.0.1.1	Minimum requirement for SSB based Beam Failure Detection under CCA

Same as in clause 10.3.4.0.1.1.

11.4.4.0.1.2	Measurement restriction for SSB based Beam Failure Detection under CCA

Same as in clause 10.3.4.0.1.2.

##### 11.4.4.0.2	Void

Same as in clause 10.3.4.0.2.

##### 11.4.4.0.3	Scheduling availability of UE during beam failure detection under CCA

Same as in clause 10.3.4.0.3.

11.4.4.0.3.1	Scheduling availability of UE performing beam failure detection under CCA with a same subcarrier spacing as PDSCH/PDCCH

Same as in clause 10.3.4.0.3.1.

##### 11.4.4.0.4	Scheduling availability of UE during candidate beam detection under CCA

Same as in clause 10.3.4.0.4.

11.4.4.0.4.1	Scheduling availability of UE performing L1-RSRP measurement under CCA with a same subcarrier spacing as PDSCH/PDCCH on FR1

Same as in clause 10.3.4.0.4.1.

#### 11.4.4.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

Editor's Note:

-	MU and TT analysis is incomplete

-	Message contents may need to be updated

-	Call setup and test procedure may need to be updated

-	Applicability may need to be updated

-	Statistical analysis to determine test case verdict is FFS

-	Some test parameters are still in brackets

11.4.4.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 11.4.4.0.1.

11.4.4.1.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR-U and supporting UL and SSB-based RLM and link recovery on shared channel access.

11.4.4.1.3	Minimum conformance requirement

The minimum requirements are specified in clause 11.4.4.0.1. DRX configuration is not used for this test.

The normative reference for this requirement is TS 38.133 [6] clause A.11.4.4.1.

11.4.4.1.4	Test description

The test parameters are given in Tables 11.4.4.1.4.1-3 and 11.4.4.1.5-1 below. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 11.4.4.1.4-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure 11.4.4.1.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 11.4.4.1.4-1: SNR variation for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

11.4.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.4.4.1.4.1-1.

Table 11.4.4.1.4.1-1: Supported test configurations for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Configuration | Description |
| --- | --- |
| 11.4.4.1-1 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Configure the test equipment and the DUT according to the parameters in Table 11.4.4.1.4.1-2.

Table 11.4.4.1.4.1-2: Initial conditions for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.4.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 11.4.4.1.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 11.4.4.1.4.1-3.

Table 11.4.4.1.4.1-3: General test parameters for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| DL CCA model |  |  |  | As specified in D.7.2.1 |  |
| UL CCA model |  |  |  | As specified in D.7.2.2 |  |
| Duplex mode |  | Config 1 |  | TDD |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |
| SSB Configuration |  | Config 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |  |
| DBT Window Configuration |  | Config 1 |  | DBT.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 30 KHz |  |
| PRACH Configuration |  | Config 1 |  | [FR1 PRACH configuration 2] |  |
| SSB Index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  | 1 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | OFF |  |
| Gap pattern ID |  |  |  | gp0 |  |
| gapOffset |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (TS 38.133 [6] Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1 |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking | Config 1 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0,1 |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.93 |  |
| T3 |  |  | s | 0.52 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.45 |  |
| D1 |  |  | s | 0.41 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

11.4.4.1.4.2	Test Procedure

There is one cell, cell 1 which is the active cell, in the test. Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. The UE transmits the reporting according to UL CCA mode. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40 ms) in test 1.

1.	Ensure the UE is in [State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On] according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	The SS sends an RRCReconfiguration message to the UE to configure inter-frequency measurement with the corresponding gap pattern and periodic CSI reporting.

3.	The UE sends an RRCReconfigurationComplete message.

4.	Set the parameters of NR Cell according to T1 in Table 11.4.4.1.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 11.4.4.1.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 11.4.4.1.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 11.4.4.1.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 11.4.4.1.5-1. T5 starts.

9.	If the SS:

a)	detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according to CSI reporting on PUCCH) during the period from time point A to time point B; and

b)	does not detect preamble on a beam associated with the candidate beam set q1 before time point B; and

c)	detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5), the number of successful tests is increased by one. Proceed with step 10.

Otherwise, the number of failed tests is increased by one. Switch off and then on the UE and proceed with step 1.

10.	Repeat steps 2-9 for all subtests until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

11.4.4.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 11.4.4.1.4.3-1: Common Exception messages for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier centre of [High] range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-6 with conditions gapUE and BFD Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.5-4 Table H.3.5-9 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.4.4.1.4.3-2: PDCCH Search Space for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.4.4.1.4.3-3: RLF-TimersAndConstants for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms1000 |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 11.4.4.1.4.3-4: PDCCH-Config for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 11.4.4.1.4.3-5: ControlResourceSet for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| } |  |  |  |

11.4.4.1.5	Test Requirement

Table 11.4.4.1.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA.

Table 11.4.4.1.5-1: Cell-specific test parameters for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA,DL |  | Note 10, 12 |  | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  |  | Note 11, 12 |  | 1.0/1.0 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA,UL |  |  |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| LCCA_DL |  |  |  | N/A | 7 |  |  |  |
| WCCA_DL |  |  | ms | N/A | TEvaluate_CBD_SSB_CCA Note 13 |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 |  | dB | 5+TT | -3+TT | -12+TT | -12+TT | -12+TT |
| SNR_SSB of set q1 | Config 1 |  | dB | -10+TT | -10+TT | 10+TT | 10+TT | 10+TT |
| SSB_RP of set q1 | Config 1 |  | dBm/SCS kHz | -105+TT | -105+TT | -85+TT | -85+TT | -85+TT |
|  | Config 1 |  | dBm/15 KHz | -98+TT |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 11.4.4.1.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4A. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. Note 13: As defined in TS 38.133 [6] Table 8.5A.5.2-1. |  |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 410 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 11.4.4.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

Editor's Note:

-	MU and TT analysis is incomplete

-	Message contents may need to be updated

-	Call setup and test procedure may need to be updated

-	Applicability may need to be updated

-	Statistical analysis to determine test case verdict is FFS

-	Some test parameters are still in brackets

11.4.4.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 11.4.4.0.

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 11.4.4.0.

11.4.4.2.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR-U and supporting UL and SSB-based RLM and link recovery on shared channel access.

11.4.4.2.3	Minimum conformance requirement

The minimum requirements are specified in clause 11.4.4.0. DRX configuration is not used for this test.

The normative reference for this requirement is TS 38.133 [6] clause A.11.4.4.2.

11.4.4.2.4	Test description

The test parameters are given in Tables 11.4.4.2.4.1-3 and 11.4.4.2.5-1 below. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 11.4.4.2.4-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure 11.4.4.2.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 11.4.4.2.4-1: SNR variation for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

11.4.4.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.4.4.2.4.1-1.

Table 11.4.4.2.4.1-1: Supported test configurations for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Configuration | Description |
| --- | --- |
| 11.4.4.2-1 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Configure the test equipment and the DUT according to the parameters in Table 11.4.4.2.4.1-2.

Table 11.4.4.2.4.1-2: Initial conditions for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.4.4.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 11.4.4.2.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 11.4.4.2.4.1-3.

Table 11.4.4.2.4.1-3: General test parameters for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| DL CCA model |  |  |  | As specified in D.7.2.1 |  |
| UL CCA model |  |  |  | As specified in D.7.2.2 |  |
| Duplex mode |  | Config 1 |  | TDD |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |
| SSB Configuration |  | Config 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |  |
| DBT Window Configuration |  | Config 1 |  | DBT.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 30 KHz |  |
| PRACH Configuration |  | Config 1 |  | [FR1 PRACH configuration 2] |  |
| SSB Index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  | 1 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 |  |
| Gap pattern ID |  |  |  | N.A. |  |
| gapOffset |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (38.133 [6] Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1 |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking | Config 1 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0,1 |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 9.01 |  |
| T3 |  |  | s | 5.16 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 3.89 |  |
| D1 |  |  | s | 3.85 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

11.4.4.2.4.2	Test Procedure

There is one cell, cell 1 which is the active cell, in the test. Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. The UE transmits the reporting according to UL CCA mode. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

1.	Ensure the UE is in [State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On] according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	The SS sends an RRCReconfiguration message to the UE to configure DRX and periodic CSI reporting.

3.	The UE sends an RRCReconfigurationComplete message.

4.	Set the parameters of NR Cell according to T1 in Table 11.4.4.2.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 11.4.4.2.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 11.4.4.2.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 11.4.4.2.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 11.4.4.2.5-1. T5 starts.

9.	If the SS:

a)	detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according to CSI reporting on PUCCH) during the period from time point A to time point B; and

b)	does not detect preamble on a beam associated with the candidate beam set q1 before time point B; and

c)	detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5), the number of successful tests is increased by one. Proceed with step 10.

Otherwise, the number of failed tests is increased by one. Switch off and then on the UE and proceed with step 1.

10.	Repeat steps 2-9 for all subtests until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

11.4.4.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 11.4.4.2.4.3-1: Common Exception messages for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.5-4 Table H.3.5-9 Table H.3.7-1 with Condition DRX.7 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.4.4.2.4.3-2: PDCCH Search Space for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.4.4.2.4.3-3: RLF-TimersAndConstants for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms1000 |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 11.4.4.2.4.3-4: PDCCH-Config for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 11.4.4.2.4.3-5: ControlResourceSet for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| } |  |  |  |

11.4.4.2.5	Test Requirement

Table 11.4.4.2.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA.

Table 11.4.4.2.5-1: Cell-specific test parameters for NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA,DL |  | Note 10, 12 |  | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  |  | Note 11, 12 |  | 1.0/1.0 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA,UL |  |  |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| LCCA_DL |  |  |  | N/A | 7 |  |  |  |
| WCCA_DL |  |  | ms | N/A | TEvaluate_CBD_SSB_CCA Note 13 |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 |  | dB | 5+TT | -3+TT | -12+TT | -12+TT | -12+TT |
| SNR_SSB of set q1 | Config 1 |  | dB | -10+TT | -10+TT | 10+TT | 10+TT | 10+TT |
| SSB_RP of set q1 | Config 1 |  | dBm/SCS kHz | -105+TT | -105+TT | -85+TT | -85+TT | -85+TT |
|  | Config 1 |  | dBm/15 KHz | -98+TT |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 11.4.4.2.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4A. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. Note 13: As defined in TS 38.133 [6] Table 8.5A.5.2-1. |  |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 410 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

## 11.5

## 11.6	Measurement Performance

### 11.6.1	SS-RSRP

#### 11.6.1.0	Minimum Conformance Requirements

##### 11.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA

Same as in clause 10.5.1.0.1.

##### 11.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA

Same as in clause 10.5.1.0.2.

##### 11.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA

Same as in clause 10.5.1.0.3.

##### 11.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA

Same as in clause 10.5.1.0.4.

#### 11.6.1.1	NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

11.6.1.1.1	Test purpose

The purpose of this test is to verify that the SS-RSRP measurement accuracy on the carrier frequency with CCA is within the specified limits. This test will verify the requirements in clauses 11.6.1.0.1 and 11.6.1.0.2 for intra-frequency measurements under CCA.

11.6.1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting standalone NR-U and [supporting intra-frequency measurements on shared channel access].

11.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 11.6.1.0.1 and 11.6.1.0.2 for absolute and relative accuracy, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.11.6.1.1.

11.6.1.1.4	Test description

Supported test configurations are shown in Table 11.6.1.1.4.1-1. Both absolute and relative accuracies of SS-RSRP intra-frequency measurements are tested by using the parameters in Table 11.6.1.1.5-1. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

11.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.6.1.1.4.1-1.

Table 11.6.1.1.4.1-1: Supported test configurations for NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA

| Configuration | Description |
| --- | --- |
| 11.6.1.1-1 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 11.6.1.1.4.1-2.

Table 11.6.1.1.4.1-2: Initial conditions for NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.9-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | LTE link is not used in this test |  |  |

1.	The general test parameter settings are set up according to Table 11.6.1.1.5-1.

2.	Message contents are defined in clause 11.6.1.1.4.3.

3.	Cell 1 is the NR PCell with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 2 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

11.6.1.1.4.2	Test procedure

In this set of test cases all cells are on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 11.6.1.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1 configuring the UE to perform [intra-frequency measurements in DBT windows] as specified in section 11.6.1.1.4.3.

4.	The UE shall transmit an RRCReconfiguration message to acknowledge the configuration.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 11.6.1.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 11.6.1.1.5-1.

11.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 11.6.1.1.4.3-1: Common Exception messages for NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.6.1.1.4.3-2: ReportConfigNR for NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

11.6.1.1.5	Test requirements

Table 11.6.1.1.5-1 defines the primary level settings including test tolerances for NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA. Table 11.6.1.1.5-2 and Table 11.6.1.1.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 11.6.1.1.5-1: Cell specific test parameters for NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DL CCA model |  |  |  | As specified in clause D.7.2.1 |  |  |  |  |  |
| UL CCA model |  |  |  | As specified in clause D.7.2.2 |  |  |  |  |  |
| PCCA_DL for dynamic channel access Note 7,9 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |  |
| PCCA_DL for semi-static channel access Note 8,9 |  |  |  | PCCA_DL=0.9375 |  |  |  |  |  |
| PCCA_UL |  |  |  | 1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA | - | SR.1.1 CCA | - | SR.1.1 CCA | - |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA | - | CR.1.1 CCA | - | CR.1.1 CCA | - |
| Control channel RMC |  | Config 1 |  | CR.1.1 CCA | - | CR.1.1 CCA | - | CR.1.1 CCA | - |
| SSB configuration for semi-static channel access |  | Config 1 |  | SSB.1 CCA | SSB.1 CCA | SSB.1 CCA | SSB.1 CCA | SSB.1 CCA | SSB.1 CCA |
| SSB configuration for dynamic channel access |  | Config 1 |  | SSB.2 CCA | SSB.2 CCA | SSB.2 CCA | SSB.2 CCA | SSB.2 CCA | SSB.2 CCA |
| DBT window configuration |  | Config 1,2,3 |  | DBT.1 | DBT.1 | DBT.1 | DBT.1 | DBT.1 | DBT.1 |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1 | NR_CCA_FR1_I |  | Not applicableNote 5 |  | -94+TT |  | -110+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -109.5+TT |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | Not applicableNote 5 |  | -91+TT |  | -107.0+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -106.5+TT |  |
| Note6 |  |  | dB | 2.46+TT | -5.97+TT | 2.46+TT | -5.97+TT | -2.01+TT | -3.54+TT |
| Note6 |  |  | dB | 6+TT | 1+TT | 6+TT | 1+TT | 1+TT | 0+TT |
| SS-RSRPNote3,6 | Config 1 | NR_CCA_FR1_I | dBm/SCS | Not applicableNote 5 | Not applicableNote 5 | -85+TT | -90+TT | -106.00+TT | -107.00+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -105.50+TT | -106.50+TT |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16MHz | Not applicableNote 5- |  | -51.99+TT |  | -70.82+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -70.32+TT |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Subtest 1 is not used when testing with 30kHz SSB SCS. NOTE 6: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |

Table 11.6.1.1.5-2: Absolute accuracy requirements for the reported values for NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SS-RSRP reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Highest SS-RSRP reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in [clause 3A.4, Table 3A.4.1-2] |  |  |  |  |

Table 11.6.1.1.5-3: Relative accuracy requirements for the reported values for NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRP_x – FFS | RSRP_x – FFS | RSRP_x – FFS |
| Highest reported value (Cell 3) | RSRP_x – FFS | RSRP_x + FFS | RSRP_x + FFS |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRP_x – FFS | RSRP_x – FFS | RSRP_x – FFS |
| Highest reported value (Cell 3) | RSRP_x – FFS | RSRP_x + FFS | RSRP_x + FFS |
| RSRP_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 11.6.1.2	NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

11.6.1.2.1	Test purpose

The purpose of this test is to verify that the SS-RSRP measurement accuracy on the carrier frequency with CCA is within the specified limits. This test will verify the requirements in clauses 11.6.1.0.1 and 11.6.1.0.2 for intra-frequency measurements under CCA.

11.6.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting standalone 2DL CA NR-U and [supporting intra-frequency measurements on shared channel access].

11.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 11.6.1.0.1 and 11.6.1.0.2 for absolute and relative accuracy, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.11.6.1.2.

11.6.1.2.4	Test description

Supported test configurations are shown in Table 11.6.1.2.4.1-1. Both absolute and relative accuracies of SS-RSRP intra-frequency measurements are tested by using the parameters in Table 11.6.1.2.5-1. In all test cases, Cell 1 is the PCell, Cell 2 is the SCell, and Cell 3 is the target cell.

11.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.6.1.2.4.1-1.

Table 11.6.1.2.4.1-1: Supported test configurations for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

| Configuration | Description |
| --- | --- |
| 11.6.1.2-1 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 11.6.1.2.4.1-2.

Table 11.6.1.2.4.1-2: Initial conditions for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.9-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | LTE link is not used in this test |  |  |

1.	The general test parameter settings are set up according to Table 11.6.1.2.5-1.

2.	Message contents are defined in clause 11.6.1.2.4.3.

3.	Cell 1 is the NR PCell, Cell 2 is the NR SCell, Cell 3 is the NR target cell, all of which have power levels set according to clauses C.1.2 and C.1.3 for this test.

11.6.1.2.4.2	Test procedure

Three cells are deployed in the test, which are FR1 PCell (Cell 1) on the carrier frequency with CCA, and two cells on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model: SCell (Cell 2) and a neighbour cell (Cell 3).

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5. Both Cell 1 (PCell) and Cell 2 (SCell) are configured on the UE.

2.	Set the parameters according to Table 11.6.1.2.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1 configuring the UE to perform [intra-frequency measurements in SCC DBT windows] as specified in section 11.6.1.2.4.3.

4.	The UE shall transmit an RRCReconfiguration message to acknowledge the configuration.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 3 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 11.6.1.2.5-2 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 11.6.1.2.5-1.

11.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 11.6.1.2.4.3-1: Common Exception messages for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.6.1.2.4.3-2: ReportConfigNR for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

11.6.1.2.5	Test requirements

Table 11.6.1.2.5-1 defines the primary level settings including test tolerances for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA. Table 11.6.1.2.5-2 and Table 11.6.1.2.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 11.6.1.2.5-1: Cell specific test parameters for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DL CCA model |  |  |  | As specified in clause D.7.2.1 |  |  |  |  |  |
| UL CCA model |  |  |  | As specified in clause D.7.2.2 |  |  |  |  |  |
| PCCA_DL for dynamic channel access Note 7,9 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |  |
| PCCA_DL for semi-static channel access Note 8,9 |  |  |  | PCCA_DL=0.9375 |  |  |  |  |  |
| PCCA_UL |  |  |  | 1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA | - | SR.1.1 CCA | - | SR.1.1 CCA | - |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA | - | CR.1.1 CCA | - | CR.1.1 CCA | - |
| Control channel RMC |  | Config 1 |  | CR.1.1 CCA | - | CR.1.1 CCA | - | CR.1.1 CCA | - |
| SSB configuration for semi-static channel access |  | Config 1 |  | SSB.1 CCA | SSB.1 CCA | SSB.1 CCA | SSB.1 CCA | SSB.1 CCA | SSB.1 CCA |
| SSB configuration for dynamic channel access |  | Config 1 |  | SSB.2 CCA | SSB.2 CCA | SSB.2 CCA | SSB.2 CCA | SSB.2 CCA | SSB.2 CCA |
| DBT window configuration |  | Config 1,2,3 |  | DBT.1 | DBT.1 | DBT.1 | DBT.1 | DBT.1 | DBT.1 |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1 | NR_CCA_FR1_I |  | Not applicableNote 5 |  | -94+TT |  | -110+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -109.5+TT |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | Not applicableNote 5 |  | -91+TT |  | -107.0+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -106.5+TT |  |
| Note6 |  |  | dB | 2.46+TT | -5.97+TT | 2.46+TT | -5.97+TT | -2.01+TT | -3.54+TT |
| Note6 |  |  | dB | 6+TT | 1+TT | 6+TT | 1+TT | 1+TT | 0+TT |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I | dBm/SCS | Not applicableNote 5 | Not applicableNote 5 | -85+TT | -90+TT | -106.00+TT | -107.00+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -105.50+TT | -106.50+TT |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16MHz | Not applicableNote 5- |  | -51.99+TT |  | -70.82+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -70.32+TT |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Subtest 1 is not used when testing with 30kHz SSB SCS. NOTE 6: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |

Table 11.6.1.2.5-2: Absolute accuracy requirements for the reported values for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SS-RSRP reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Highest SS-RSRP reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in [clause 3A.4, Table 3A.4.1-2] |  |  |  |  |

Table 11.6.1.2.5-3: Relative accuracy requirements for the reported values for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRP_x – FFS | RSRP_x – FFS | RSRP_x – FFS |
| Highest reported value (Cell 3) | RSRP_x – FFS | RSRP_x + FFS | RSRP_x + FFS |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRP_x – FFS | RSRP_x – FFS | RSRP_x – FFS |
| Highest reported value (Cell 3) | RSRP_x – FFS | RSRP_x + FFS | RSRP_x + FFS |
| RSRP_x is the reported value of Cell 2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 11.6.2	SS-RSRQ

#### 11.6.2.0	Minimum Conformance Requirements

##### 11.6.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA

Same as in clause 10.5.2.0.1.

##### 11.6.2.0.2	Intra-frequency relative SS-RSRQ measurement accuracy requirements under CCA

Same as in clause 10.5.2.0.2.

##### 11.6.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA

Same as in clause 10.5.2.0.3.

##### 11.6.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA

Same as in clause 10.5.2.0.4.

#### 11.6.2.1	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

Editor’s note: This test case is incomplete in following aspects:

- MU and TT analysis is incomplete.

- Message contents may need to be updated.

- Call setup and test procedure may need to be updated.

- Applicability needs to be updated.

- Statistical analysis to determine test case verdict is FFS.

- Test requirements are FFS.

- Some parameters and/or references are still in brackets.

11.6.2.1.1	Test purpose

The purpose of this test is to verify that the SS-RSRQ measurement accuracy on the carrier frequency with CCA is within the specified limits.

11.6.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting standalone NR-U and [supporting intra-frequency measurements on shared channel access].

11.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 11.6.2.0.1 and 11.6.2.0.2 for absolute and relative accuracy under CCA, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.11.6.2.1.

11.6.2.1.4	Test description

Supported test configurations are shown in Table 11.6.2.1.4.1-1. Both absolute and relative accuracies of SS-RSRQ intra-frequency measurements are tested by using the parameters in Table 11.6.2.1.5-1. In all test cases, Cell 1 is the PCell with CCA and Cell 2 is the target cell with CCA.

11.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.6.2.1.4.1-1.

Table 11.6.2.1.4.1-1: Supported test configurations for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Configuration | Description |
| --- | --- |
| 11.6.2.1-1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 11.6.2.1.4.1-2.

Table 11.6.2.1.4.1-2: Initial conditions for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.9-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | LTE link is not used in this test |  |  |

1.	The general test parameter settings are set up according to Table 11.6.2.1.5-1.

2.	Message contents are defined in clause 11.6.2.1.4.3.

3.	Cell 1 is the NR PCell with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 2 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test. All cells are on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model.

11.6.2.1.4.2	Test procedure

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1).

2.	Set the parameters according to Table 11.6.2.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1 configuring the UE to perform [intra-frequency measurements in DBT windows] as specified in section 11.6.2.1.4.3.

4.	The UE shall transmit an RRCReconfiguration message to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 11.6.2.1.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 11.6.2.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 11.6.2.1.5-1.

11.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 11.6.2.1.4.3-1: Common Exception messages for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.6.2.1.4.3-2: ReportConfigNR for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

11.6.2.1.5	Test requirements

Table 11.6.2.1.5-1 defines the primary level settings including test tolerances for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA. Table 11.6.2.1.5-2 and Table 11.6.2.1.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 11.6.2.1.5-1: Cell specific test parameters for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| DL CCA model |  | Config 1 |  | As specified in clause D.7.2.1 |  |  |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause D.7.2.2 |  |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - | 1.0 | - |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - | 0.75 | - |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |  |  |  |  |
| Gap Pattern ID |  |  |  | 0 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR1.1 CCA |  | SR1.1 CCA |  | SR1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS Configuration |  | Config 1 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 | - | 3 |
| DBT Window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15kHz | -91+TT |  | - |  | -110+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -109.5+TT |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -88+TT |  | - |  | -107+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -106.5+TT |  |
|  |  |  | dB | -1.76+TT |  | -4.7+TT |  | -5.46+TT | -5.46+TT |
|  |  |  | dB | 3+TT | 3+TT | -2.9+TT | -2.9+TT | -4+TT | -4+TT |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I |  | -85+TT | -85+TT | - | - | -111+TT | -111+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -110.5+TT | -110.5+TT |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I | dB | -14.77+TT | -14.77+TT | -16.76+TT | -16.76+TT | -17.34+TT | -17.34+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16MHz | -50 |  | - |  | -73.4 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -72.9 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |

Table 11.6.2.1.5-2: Absolute accuracy requirements for the reported values for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SS-RSRQ reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Highest SS-RSRQ reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in [clause 3A.4, Table 3A.4.1-2] |  |  |  |  |

Table 11.6.2.1.5-3: Relative accuracy requirements for the reported values for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x – FFS | RSRQ_x – FFS |
| Highest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x + FFS | RSRQ_x + FFS |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x – FFS | RSRQ_x – FFS |
| Highest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x + FFS | RSRQ_x + FFS |
| RSRQ_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 11.6.2.2	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

Editor’s note: This test case is incomplete in following aspects:

- MU and TT analysis is incomplete.

- Message contents may need to be updated.

- Call setup and test procedure may need to be updated.

- Applicability needs to be updated.

- Statistical analysis to determine test case verdict is FFS.

- Test requirements are FFS.

- Some parameters and/or references are still in brackets.

11.6.2.2.1	Test purpose

The purpose of this test is to verify that the SS-RSRQ measurement accuracy on the carrier frequency with CCA is within the specified limits.

11.6.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting standalone NR-U and [supporting inter-frequency measurements on shared channel access].

11.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 11.6.2.0.3 and 11.6.2.0.4 for absolute and relative accuracy under CCA, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.11.6.2.2.

11.6.2.2.4	Test description

Supported test configurations are shown in Table 11.6.2.2.4.1-1. Both absolute and relative accuracies of SS-RSRQ inter-frequency measurements are tested by using the parameters in Table 11.6.2.2.5-1. In all test cases, Cell 1 is the PCell with CCA and Cell 2 is target cell with CCA. The inter-frequency measurements are supported by a measurement gap.

11.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.6.2.2.4.1-1.

Table 11.6.2.2.4.1-1: Supported test configurations for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Configuration | Description |
| --- | --- |
| 11.6.2.2-1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 11.6.2.2.4.1-2.

Table 11.6.2.2.4.1-2: Initial conditions for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.9-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | LTE link is not used in this test |  |  |

1.	The general test parameter settings are set up according to Table 11.6.2.2.5-1.

2.	Message contents are defined in clause 11.6.2.2.4.3.

3.	Cell 1 is the NR PCell with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 2 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test. The two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies, with CCA and transmit SSBs in DBT windows according to DL CCA model.

11.6.2.2.4.2	Test procedure

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1.

2.	Set the parameters according to Table 11.6.2.2.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1 configuring the UE to perform [inter-frequency measurements in DBT windows] as specified in section 11.6.2.2.4.3.

4.	The UE shall transmit an RRCReconfiguration message to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 11.6.2.2.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 11.6.2.2.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 11.6.2.2.5-1.

11.6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 11.6.2.2.4.3-1: Common Exception messages for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.6.2.2.4.3-2: ReportConfigNR for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

11.6.2.2.5	Test requirements

Table 11.6.2.2.5-1 defines the primary level settings including test tolerances for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA. Table 11.6.2.2.5-2 and Table 11.6.2.2.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 11.6.2.2.5-1: Cell specific test parameters for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| DL CCA model |  | Config 1 |  | As specified in clause D.7.2.1 |  |  |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause D.7.2.2 |  |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - | 1.0 | - |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - | 0.75 | - |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |  |  |  |  |
| Gap pattern ID |  | Config 1 |  | 0 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS Configuration |  | Config 1 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 | - | 3 |
| DBT Window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15kHz | -86.27+TT |  | -113+TT |  | -112+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -111.5+TT |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -83.27+TT |  | -110+TT |  | -109+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -108.5+TT |  |
|  |  |  | dB | -1.75+TT |  | -1.75+TT |  | 3+TT | -1.75+TT |
|  |  |  | dB | -1.75+TT |  | -1.75+TT |  | 3+TT | -1.75+TT |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I |  | -85.02+TT | -85.02+TT | -111.75+TT | -111.75+TT | -106+TT | -110.75+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -105.5+TT | -110-.25+TT |
| SS-RSRQNote3 |  | NR_CCA_FR1_I | dB | -14.77+TT | -14.77+TT | -40.59+TT | -40.59+TT | 12.56+TT | 14.76+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -50 |  | -76.73 |  | -73.19 | -75.73 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -72.69 | -75.23 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |

Table 11.6.2.2.5-2: Absolute accuracy requirements for the reported values for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SS-RSRQ reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Highest SS-RSRQ reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in [clause 3A.4, Table 3A.4.1-2] |  |  |  |  |

Table 11.6.2.2.5-3: Relative accuracy requirements for the reported values for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x – FFS | RSRQ_x – FFS |
| Highest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x + FFS | RSRQ_x + FFS |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x – FFS | RSRQ_x – FFS |
| Highest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x + FFS | RSRQ_x + FFS |
| RSRQ_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 11.6.2.3	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

11.6.2.3.1	Test purpose

The purpose of this test is to verify that the SS-RSRQ measurement accuracy on the carrier frequency with CCA is within the specified limits.

11.6.2.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting standalone 2DL CA NR-U and [supporting intra-frequency measurements on shared channel access].

11.6.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 11.6.2.0.1 and 11.6.2.0.2 for absolute and relative accuracy under CCA, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.11.6.2.3.

11.6.2.3.4	Test description

Supported test configurations are shown in Table 11.6.2.3.4.1-1. Both absolute and relative accuracies of SS-RSRQ intra-frequency measurements are tested by using the parameters in Table 11.6.2.3.5-1. In all test cases, Cell 1 is the PCell with CCA, Cell 2 is the SCell with CCA, and Cell 3 is the target cell with CCA.

11.6.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.6.2.3.4.1-1.

Table 11.6.2.3.4.1-1: Supported test configurations for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA

| Configuration | Description |
| --- | --- |
| 11.6.2.3-1 | SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 11.6.2.3.4.1-2.

Table 11.6.2.3.4.1-2: Initial conditions for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.9-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.6.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | LTE link is not used in this test |  |  |

1.	The general test parameter settings are set up according to Table 11.6.2.3.5-1.

2.	Message contents are defined in clause 11.6.2.3.4.3.

3.	Cell 1 is the PCell with CCA, Cell 2 is the SCell with CCA, and Cell 3 is the target cell with CCA, all of which have power levels set according to clauses C.1.2 and C.1.3 for this test. PCell (Cell 1) is on one carrier frequency with CCA, Cell 2 and Cell 3 are both on a different carrier frequency than Cell 1.

11.6.2.3.4.2	Test procedure

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5. Both Cell 1 (PCell) and Cell 2 (SCell) are configured on the UE. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1).

2.	Set the parameters according to Table 11.6.2.3.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1 configuring the UE to perform [intra-frequency measurements in SCC DBT windows] as specified in section 11.6.2.3.4.3.

4.	The UE shall transmit an RRCReconfiguration message to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 11.6.2.3.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 3 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 11.6.2.3.5-2 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 11.6.2.3.5-1.

11.6.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 11.6.2.3.4.3-1: Common Exception messages for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.6.2.3.4.3-2: ReportConfigNR for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

11.6.2.3.5	Test requirements

Table 11.6.2.3.5-1 defines the primary level settings including test tolerances for NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA. Table 11.6.2.3.5-2 and Table 11.6.2.3.5-3 define the absolute and relative accuracy requirements, correspondingly.

Table 11.6.2.3.5-1: Cell specific test parameters for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 / Cell 2 |  | Cell 3 | Cell 1 / Cell 2 |  | Cell 3 | Cell 1 / Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  | freq1 for Cell 1 freq2 for Cell 2 |  | freq2 | freq1 for Cell 1 freq2 for Cell 2 |  | freq2 | freq1 for Cell 1 freq2 for Cell 2 | freq2 |
| DL CCA model |  | Config 1 |  | As specified in clause D.7.2.1 |  |  |  |  |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause D.7.2.2 |  |  |  |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - |  | 1.0 | - |  | 1.0 | - |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - |  | 0.9375 | - |  | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - |  | 0.75 | - |  | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - |  | 0.75 | - |  | 0.75 | - |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| Gap Pattern ID |  |  |  | 0 |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR1.1 CCA |  |  | SR1.1 CCA |  |  | SR1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 CCA |  |  | CCR.1.1 CCA |  |  | CCR.1.1 CCA |  |
| TRS Configuration |  | Config 1 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | 3 (for Cell 2) | 3 |  | 3 (for Cell 2) | 3 |  | 3 (for Cell 2) | 3 |
| DBT Window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 | 0 |  | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15kHz | -91+TT |  |  | - |  |  | -110+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  | -109.5+TT |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -88+TT |  |  | - |  |  | -107+TT |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  | -106.5+TT |  |
|  |  |  | dB | -1.76+TT |  |  | -4.7+TT |  |  | -5.46+TT | -5.46+TT |
|  |  |  | dB | 3+TT |  | 3+TT | -2.9+TT |  | -2.9+TT | -4+TT | -4+TT |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I |  | -85+TT |  | -85+TT | - |  | - | -111+TT | -111+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  | -110.5 | -110.5+TT |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I | dB | -14.77+TT |  | -14.77+TT | -16.76+TT |  | -16.76+TT | -17.34+TT+TT | -17.34+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16MHz | -50 |  |  | - |  |  | -73.4 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  | -72.9 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |  |

Table 11.6.2.3.5-2: Absolute accuracy requirements for the reported values for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SS-RSRQ reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Highest SS-RSRQ reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in [clause 3A.4, Table 3A.4.1-2] |  |  |  |  |

Table 11.6.2.3.5-3: Relative accuracy requirements for the reported values for NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x – FFS | RSRQ_x – FFS |
| Highest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x + FFS | RSRQ_x + FFS |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x – FFS | RSRQ_x – FFS |
| Highest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x + FFS | RSRQ_x + FFS |
| RSRQ_x is the reported value of Cell 2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 11.6.2.4	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure may need to be updated

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements are FFS

- Some parameters and/or references are still in brackets

11.6.2.4.1	Test purpose

The purpose of this test is to verify that the SS-RSRQ measurement accuracy on the carrier frequency with CCA is within the specified limits.

11.6.2.4.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting standalone NR-U and [supporting inter-frequency measurements on shared channel access].

11.6.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 11.6.2.0.3 and 11.6.2.0.4 for absolute and relative accuracy under CCA, correspondingly.

The normative reference for this requirement is TS 38.133 [6] clause A.11.6.2.4.

11.6.2.4.4	Test description

Supported test configurations are shown in Table 11.6.2.4.4.1-1. Both absolute and relative accuracies of SS-RSRQ inter-frequency measurements are tested by using the parameters in Table 11.6.2.4.5-1. In all test cases, Cell 1 is the PCell and Cell 2 is target cell with CCA.

11.6.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.6.2.4.4.1-1.

Table 11.6.2.4.4.1-1: Supported test configurations for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA

| Configuration | Description |
| --- | --- |
| 11.6.2.4-1 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 11.6.2.4-2 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 11.6.2.4-3 | Without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 11.6.2.4.4.1-2.

Table 11.6.2.4.4.1-2: Initial conditions for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.9-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 11.6.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | LTE link is not used in this test |  |  |

1.	The general test parameter settings are set up according to Table 11.6.2.4.5-1.

2.	Message contents are defined in clause 11.6.2.4.4.3.

3.	Cell 1 is the PCell and Cell 2 is target cell with CCA, all of which have power levels set according to clauses C.1.2 and C.1.3 for this test. The two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies, Cell 2 is with CCA and transmit SSBs in DBT windows according to DL CCA model.

11.6.2.4.4.2	Test procedure

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On], according to TS 38.508-1 [14] clause 4.5. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1).

2.	Set the parameters according to Table 11.6.2.4.5-1 and Table 11.6.2.4.5-2 appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1 configuring the UE to perform [inter-frequency measurements in SCC DBT windows] as specified in section 11.6.2.4.4.3.

4.	The UE shall transmit an RRCReconfiguration message to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 11.6.2.4.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 11.6.2.4.5-3 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to [Tables G.TBD in Annex G clause G.TBD] is achieved.

8.	Repeat steps 1-7 for each subtest defined in Table 11.6.2.4.5-1.

11.6.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 11.6.2.4.4.3-1: Common Exception messages for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 [Table 7.3.1-3 in TS 38.508-1 [14] with condition DBT.1] |

Table 11.6.2.4.4.3-2: ReportConfigNR for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

11.6.2.4.5	Test requirements

Table 11.6.2.4.5-1 defines the primary level settings including test tolerances for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA. Table 11.6.2.4.5-3 and Table 11.6.2.4.5-4 define the absolute and relative accuracy requirements, correspondingly.

Table 11.6.2.4.5-1: Cell specific test parameters for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA for target cell

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  |  |  | freq2 | freq2 | freq2 |
| DL CCA model |  | Config 1, 2, 3 |  | As specified in clause D.7.2.1 |  |  |
| UL CCA model |  | Config 1, 2, 3 |  | As specified in clause D.7.2.2 |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 |  |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 |  |  |
|  |  | PCCA_DL_2 |  | 0.75 |  |  |
| Duplex mode |  | Config 1, 2, 3 |  | TDD |  |  |
| TDD configuration |  | Config 1, 2, 3 |  | TDDConf.1.1 CCA |  |  |
| BWchannel |  | Config 1, 2, 3 | MHz | 40: NRB,c = 106 |  |  |
| Gap pattern ID |  | Config 1, 2, 3 |  | 0 |  |  |
| BWP BW |  | Config 1, 2, 3 |  | 40: NRB,c = 106 |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| Time offset with Cell 1 |  | Config 1, 2, 3 | s | 3 |  |  |
| DBT Window configuration |  | Config 1, 2, 3 |  | DBT.1 |  |  |
| SSB configuration |  | Config 1, 2, 3 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |
| SMTC configuration |  | Config 1, 2, 3 |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 3 | kHz | 30 kHz |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15kHz | -86.27+TT | -113+TT | -112 |
|  |  | NR_CCA_FR1_J |  |  |  | -111.5 |
| Note2 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/SCS | -83.27+TT | -110+TT | -109 |
|  |  | NR_CCA_FR1_J |  |  |  | -108.5 |
|  |  |  | dB | -1.75+TT | -1.75+TT | -1.75+TT |
|  |  |  | dB | -1.75+TT |  | -1.75+TT |
| SS-RSRPNote3 | Config 1, 2, 3 | NR_CCA_FR1_I |  | -85.02+TT | -111.75+TT | -110.75+TT |
|  |  | NR_CCA_FR1_J |  |  |  | -110-.25+TT |
| SS-RSRQNote3 |  | NR_CCA_FR1_I | dB | -14.77+TT | -40.59+TT | 14.76+TT |
|  |  | NR_CCA_FR1_J |  |  |  |  |
| IoNote3 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/SCS | -50 | -76.73 | -75.73 |
|  |  | NR_CCA_FR1_J |  |  |  | -75.23 |
| Propagation condition |  |  | - | AWGN |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |

Table 11.6.2.4.5-2: Cell specific test parameters for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA for NR PCell

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 | Cell 1 |
| SSB ARFCN |  |  |  |  | freq1 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |
|  |  | Config 2,3 |  |  | TDD |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz |  | 10: NRB,c = 52 |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |
| Gap Pattern ID |  |  |  |  | 0 |  |
| BWP configuration |  | Initial DL BWP |  |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| DRX Cycle |  |  | ms |  | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |  |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  |  | CR.2.1 TDD |  |
| Control Channel RMC |  | Config 1 |  |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  |  | OP. 1 |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |
| SMTC configuration |  | Config 1 |  |  | SMTC.2 |  |
|  |  | Config 2,3 |  |  | SMTC.1 |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  |  | SSB.2 FR1 |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz |  | 15 kHz |  |
|  |  | Config 3 |  |  | 30 kHz |  |
| EPRE ratio of PSS to SSS |  |  | dB |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | Depending on band group NOTE 6 | dBm/15kHz | -85+TT | -101 +TT | -114 + ΔBG_offset +TT |
|  | Config 3 | Depending on band group NOTE 6 |  | -91 +TT | - | -114 + ΔBG_offset +TT |
| Noc Note2 | Config 1,2 | Depending on band group NOTE 6 | dBm/SCS | -85 +TT | -101 +TT | -114+ ΔBG_offset +TT |
|  | Config 3 | Depending on band group NOTE 6 |  | -88 +TT | - | -111+ ΔBG_offset +TT |
|  |  |  | dB | -1.76 +TT | -4.7 +TT | -5.46 |
|  |  |  | dB | 3 +TT | -2.9 +TT | -4 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group NOTE 6 | dBm/SCS | -82 +TT | -103.9 +TT | -118 + ΔBG_offset +TT |
|  | Config 3 | Depending on band group NOTE 6 |  | -85 +TT | - | -115 + ΔBG_offset +TT |
| SS-RSRQ Note3 |  | Depending on band group NOTE 6 | dB | -14.77 +TT | -16.76 +TT | -17.34 + ΔBG_offset +TT |
| IoNote3 | Config 1,2 | Depending on band group NOTE 6 | dBm/ 9.36MHz | -50 | -70 | -83.5 + ΔBG_offset +TT |
|  | Config 3 | Depending on band group NOTE 6 | dBm/ 38.16MHz | -50 | - | -77.4 + ΔBG_offset +TT |
| Propagation condition |  |  | - | AWGN |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. Note 7: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |  |  |  |

Table 11.6.2.4.5-3: Absolute accuracy requirements for the reported values for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SS-RSRQ reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Highest SS-RSRQ reported value | FFS | FFS | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
|  |  |  | Bands FFS | FFS |
| Note 1: NR operating band groups are defined in [clause 3A.4, Table 3A.4.1-2] |  |  |  |  |

Table 11.6.2.4.5-4: Relative accuracy requirements for the reported values for NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x – FFS | RSRQ_x – FFS |
| Highest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x + FFS | RSRQ_x + FFS |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x – FFS | RSRQ_x – FFS |
| Highest reported value (Cell 3) | RSRQ_x – FFS | RSRQ_x + FFS | RSRQ_x + FFS |
| RSRQ_x is the reported value of Cell 2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

# 12 to 13	FFS

# 14	NR standalone tests for Satellite access

## 14.0	General

Editor's Note:

- ephemerisInfo is still FFS

The Satellite Access tests defined in this clause apply provided that UE indicates nonTerrestrialNetwork and is accessing a cell served by a Satellite Access Node (SAN). The requirements apply provided that serving and all neighbour satellites on the same layer are of same satellite type (LEO or GEO).

### 14.0.1	Principle of testing GSO and NGSO scenarios

The test cases related to satellite access are defined for both GSO and NGSO. The testing principle for these test cases is as follows:

-	A UE capable of GSO only is required to pass the test cases with GSO.

-	A UE capable of NGSO only is required to pass the test cases with NGSO.

-	A UE capable of both GSO and NGSO is required to pass the test cases with NGSO only.

Support of GSO and NGSO scenario is indicated via ntn-ScenarioSupport-r17.

### 14.0.2	Principle of testing different RRM requirements

Satellite Access RRM test cases are defined for all applicable RRM requirements. The testing principle for these test cases is as follows:

-	A UE capable of NTN only is required to pass all the test cases defined in clause 14.

-	A UE capable of both TN and NTN is required to pass the test cases for NTN specific requirements in Table 14.0.2-1.

Table 14.0.2-1: Test cases for NTN specific requirements

| Clause | Test case slogan |
| --- | --- |
| 14.1.2 | SA FR1 Cell reselection for UE configured with [capability for enhanced requirements] for NR satellite access |
| 14.1.3 | SA FR1 Time-based cell reselection for NR satellite access |
| 14.1.4 | SA FR1 Location-based cell reselection for NR satellite access |
| 14.1.7 | SA FR1-FR1 Cell reselection for UE configured with [capability for enhanced requirements] for NR satellite access |
| 14.1.8 | SA FR1-FR1 Time-based Cell reselection for NR satellite access |
| 14.1.9 | SA FR1-FR1 Location-based Cell reselection for NR satellite access |
| 14.2.1.3 | SA FR1 SAN time-based conditional Handover for NR satellite access |
| 14.2.1.4 | SA FR1-FR1 SAN time-based conditional Handover for NR satellite access |
| 14.2.1.5 | SA FR1 SAN distance-based conditional Handover for NR satellite access |
| 14.2.1.6 | SA FR1-FR1 SAN distance-based conditional Handover for NR satellite access |
| 14.3.1.1 | SA FR1 NR UE Transmit Timing Test for NR satellite access |
| 14.5.1.1 | SA FR1 event triggered reporting tests without gap under non-DRX for NR satellite access |
| 14.5.1.2 | SA FR1 event triggered reporting tests without gap under DRX for NR satellite access |
| 14.5.1.3 | SA FR1 event triggered reporting tests without gap under non-DRX with SSB index reading for NR satellite access |
| 14.5.1.4 | SA FR1 event triggered reporting tests with single measurement gap under non-DRX for satellite access for NR satellite access |
| 14.5.1.5 | SA FR1 event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for NR satellite access |
| 14.5.1.6 | SA FR1 event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for NR satellite access |
| 14.5.2.1 | SA FR1-FR1 Event triggered reporting test without gap under non-DRX for NR satellite access |
| 14.5.2.2 | SA FR1-FR1 Event triggered reporting tests without gap under DRX for NR satellite access |
| 14.6.3.1 | SA FR1 SA measurement accuracy for NR satellite access |
| 14.6.3.2 | SA FR1-FR1 SA measurement accuracy for NR satellite access |
| 14.6.4.1 | SA FR1 SSB based L1-RSRP measurement for NR satellite access |
| 14.6.4.2 | SA FR1 CSI-RS based L1-RSRP measurement on resource set with repetition off for NR satellite access |

### 14.0.3	Principle of testing different ephemeris formats

Satellite access RRM test cases are defined such that satellite ephemeris information is sent to UE in each test case, according to Tables 14.0.3-1 and 14.0.3-2.

Table 14.0.3-1: Test cases configuring EphemerisInfo as PositionVelocity

| Functional Area | Test Case |
| --- | --- |
| RRC_IDLE state mobility | 14.1.3 |
|  | 14.1.4 |
|  | 14.1.5 |
|  | 14.1.6 |
|  | 14.1.9 |
|  | 14.1.10 |
| Handover | 14.2.1.3 |
|  | 14.2.1.4 |
|  | 14.2.1.5 |
|  | 14.2.1.6 |
| Timing | 14.3.1 |
| Radio Link Monitoring | 14.4.1.1 |
|  | 14.4.1.2 |
|  | 14.4.1.5 |
|  | 14.4.1.6 |
| BFD and LR procedures | 14.4.2.1 |
|  | 14.4.2.3 |
|  | 14.4.2.5 |
| Active BWP switch | 14.4.3.1 |
| UE specific CBW change | 14.4.4.1 |
| PL-RS switching delay | 14.4.5.1 |
| Intra-frequency measurements | 14.5.1.1 |
|  | 14.5.1.3 |
|  | 14.5.1.5 |
| Inter-frequency measurements | 14.5.2.1 |
|  | 14.5.2.3 |
|  | 14.5.2.7 |
| L1-RSRP measurements | 14.5.3.1 |
|  | 14.5.3.3 |
| SS-RSRP accuracy | 14.6.1.1 |
| SS-RSRQ accuracy | 14.6.2.1 |
| SS-SINR accuracy | 14.6.3.1 |
| L1-RSRP accuracy | 14.6.4.1 |

Table 14.0.3-2: Test cases configuring EphemerisInfo as Orbital

| Functional Area | Test Case |
| --- | --- |
| RRC_IDLE state mobility | 14.1.1 |
|  | 14.1.2 |
|  | 14.1.5 |
|  | 14.1.6 |
| Handover | 14.2.1.1 |
|  | 14.2.1.2 |
| RRC Connection Mobility Control | 14.2.2.1 |
|  | 14.2.2.2 |
|  | 14.2.2.3 |
| Timing | 14.3.2 |
| Radio Link Monitoring | 14.4.1.3 |
|  | 14.4.1.4 |
|  | 14.4.1.7 |
|  | 14.4.1.8 |
| BFD and LR procedures | 14.4.2.2 |
|  | 14.4.2.4 |
|  | 14.4.2.6 |
| Active BWP switch | 14.4.3.2 |
| Intra-frequency measurements | 14.5.1.2 |
|  | 14.5.1.4 |
|  | 14.5.1.6 |
| Inter-frequency measurements | 14.5.2.2 |
|  | 14.5.2.4 |
|  | 14.5.2.6 |
|  | 14.5.2.8 |
| L1-RSRP measurements | 14.5.3.2 |
|  | 14.5.3.4 |
| SS-RSRP accuracy | 14.6.1.2 |
| SS-RSRQ accuracy | 14.6.2.2 |
| SS-SINR accuracy | 14.6.3.2 |
| L1-RSRP accuracy | 14.6.4.2 |

### 14.0.4	General setup for SIB19

The general parameters for SIB19 setup is specified in Table 14.0.4-1.

Table 14.0.4-1: SIB19 configuration for NTN test cases

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO: 900s |
| cellSpecificKoffset | slot | LEO: 8 GEO: 256 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  | FFS |
| ta-Report |  | Not configured |

## 14.1	RRC_IDLE state mobility

FFS

## 14.2	RRC_CONNECTED state mobility

FFS

## 14.3	Timing for Satellite Access

FFS

## 14.4	Signalling characteristics

FFS

## 14.5	Measurement procedure

### 14.5.1	Intra-frequency Measurements for SAN

#### 14.5.1.0	Minimum conformance requirements

##### 14.5.1.0.1	Minimum conformance requitements for intra-frequency measurements without measurement gaps

14.5.1.0.1.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 14.5.1.0.1.1-1

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 14.5.1.0.1.1-2

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 14.5.1.0.1.2-1

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and

if SMTCs do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

if SMTCs partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

-	NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap is the number of SMTCs that partially overlap with each other.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps, or according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

if the high layer in TS 38.331 [13] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where

-	If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP max is the maximum MGRP across all configured per-UE measurement gap. Otherwise, MGRP max is the MGRP of configured measurement gap.

-	Starting from the beginning of any SMTC occasion:

-	Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and

-	Navailable_SAN is the number of SMTC occasions within the window W that don’t collide with any non-dropped MG occasion within or outside the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in 38.133 [6] section 9.1C.8.3. The collision rule between SMTC occasion and measurement gap occasion is defined in 38.133 [6] section 9.1C.9.1

Kp = [1] when Navailable_SAN = 0 and measurement gap sharing in 38.133 [6] clause 9.1.2.1a shall apply.

Kp = 1 when intra-frequency SMTC is fully non overlapping with measurement gaps.

For calculation of Kp, if the high layer signalling (TS 38.331 [13]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

Klayer1_measurement: it is scaling factor for sharing between L3 and L1 measurement, and Klayer1_measurement =1, if GEO satellites are measured on the carrier, or if LEO satellites are measured on the carrier and UE supports parallelMeasurementWithoutRestriction, otherwise

Klayer1_measurement =1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement =1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If the higher layer signalling in TS38.331 [13] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

Table 14.5.1.0.1.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5 x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 14.5.1.0.1.1-2: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The requirements in clause 14.5.1.0.1.1 and 14.5.1.0.1.2 are not applicable when the overall overhead ratio due to scheduling restriction caused by all configured SMTCs (i.e. scheduling restriction overhead of all SMTCs in one SMTC periodicity), is larger than 75%.

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.1.

14.5.1.0.1.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 14.5.1.0.1.2-1.

If the higher layer signalling in TS38.331 [13] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra.

Table 14.5.1.0.1.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.2.

14.5.1.0.1.3	Scheduling availability of UE during intra-frequency measurements

When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged (in TS 38.331 [13]), if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [8] are included.

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.3.

14.5.1.0.1.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement:

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured (in TS 38.331 [13]), the SMTC periodicity follows smtc2; Otherwise, the SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [13], the SMTC periodicity follows smtc2; Otherwise, the SMTC periodicity follows smtc1.

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.3.1.

14.5.1.0.1.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO

For UE which do not support TBD the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured (in TS 38.331 [13]), the SMTC periodicity follows smtc2; Otherwise, the SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [13], the SMTC periodicity follows smtc2; Otherwise, the SMTC periodicity follows smtc1.

-	If the following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots

-	The UE is expected to receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and/or the corresponding PDSCH, on SSB symbols to be measured.

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.3.2.

##### 14.5.1.0.2	Minimum conformance requitements for intra-frequency measurements with measurement gaps

14.5.1.0.2.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 14.5.1.0.2.1-1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 14.5.1.0.2.1-2.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 14.5.1.0.2.2-1.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. When the UE is configured with concurrent measurement gaps and the two measurement gaps are fully overlapping with MGRP=160ms, Kgap = 2. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps.

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and

if SMTCs within a measurement gap do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

if SMTCs within a measurement gap partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

-	NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap is the number of SMTCs that partially overlap with each other.

If the higher layer signaling in TS 38.331 [13] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

Table 14.5.1.0.2.1-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x 5) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

Table 14.5.1.0.2.1-2: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 3 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 3) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 3 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.6.1 and 9.2C.6.2.

14.5.1.0.2.2	Intra-frequency measurement period

The measurement period for FR1 intrafrequency measurements with gaps is as shown in table 14.5.1.0.2.2-1.

Table 14.5.1.0.2.2-1: Measurement period for intra-frequency measurements with gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5) x Kmulti_SMTC x Kgap x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.6.3.

#### 14.5.1.1	NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

14.5.1.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 14.5.1.0.1.1 and 14.5.1.0.1.2.

14.5.1.1.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.1.

14.5.1.1.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table 14.5.1.1.4.1-1 and 14.5.1.1.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.

UE is configured with 2 non-overlapping SMTCs for the intra-frequency measurement. The SMTC periodicity is 40ms, and SMTC1 is associated with Cell 1 with offset 0, and SMTC2 is associated with Cell 2 with offset 20ms.

14.5.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.1.4.1-1.

Table 14.5.1.1.4.1-1: Supported test configurations for NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.1-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.1-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.1.4.1-2.

Table 14.5.1.1.4.1-2: Initial conditions for NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.1.4.1-3.

2.	Message contents are defined in clause 14.5.1.1.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.1.4.1-3: General test parameters for NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |
| NTN reference configuration |  | 1 | TBD | For GSO |
|  |  | 2 | TBD | For NGSO |
| SMTC1 configuration |  | 1 | TBD | Period: 40ms, offset: 0 |
| SMTC2 configuration |  | 2 | TBD | Period: 40ms, offset: 20ms |
| A3-Offset | dB | 1, 2 | -4.5 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 2 | 20 ms | Asynchronous cells. The timing of Cell 2 is 20ms later than the timing of Cell 1. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

14.5.1.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.1.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, and SMTC configuration according to Table 14.5.1.1.4.1-3.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.1.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by event A3. If the overall delay measured from the beginning of time period T2 is less than [800] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.1.1.4.3	Message contents

FFS

14.5.1.1.5	Test requirements

Tables 14.5.1.1.4.1-3 and 14.5.1.1.5-1 define the primary level settings including test tolerances for NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access.

Table 14.5.1.1.5-1: Cell specific test parameters for NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | TBD |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.1.2	NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

14.5.1.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 14.5.1.0.1.1 and 14.5.1.0.1.2.

14.5.1.2.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.2.

14.5.1.2.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table 14.5.1.2.4.1-1 and 14.5.1.2.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The UE shall be provided with the valid information about the SAN serving the each cell in the test before the start of the test.

UE is configured with 1 SMTC for the intra-frequency measurement. Both Cell 1 and Cell 2 are associated with the configured SMTC.

14.5.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.2.4.1-1.

Table 14.5.1.2.4.1-1: Supported test configurations for NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.2-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.2-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.2.4.1-2.

Table 14.5.1.2.4.1-2: Initial conditions for NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.2.4.1-3.

2.	Message contents are defined in clause 14.5.1.2.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.2.4.1-3: General test parameters for NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| NTN reference configuration |  | 1 | TBD |  | For GSO |
|  |  | 2 | TBD |  | For NGSO |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |  |
| SMTC configuration |  | 1, 2 | SMTC.2 |  |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX. 7 |  |
| Time offset between serving and neighbour cells |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 5 | 10 |  |

14.5.1.2.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.2.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, and SMTC configuration according to Table 14.5.1.2.4.1-3.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.2.5-1. T2 starts.

6. 	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delay measured from the beginning of time period T2 is less than [TBD] ms for Test 1, or [TBD] ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

11. Repeat step 1-10 for each sub-test in Table 14.5.1.2.4.1-3 as appropriate.

14.5.1.2.4.3	Message contents

FFS

14.5.1.2.5	Test requirements

Tables 14.5.1.2.4.1-3 and 14.5.1.2.5-1 define the primary level settings including test tolerances for NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access.

Table 14.5.1.2.5-1: Cell specific test parameters for NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than TBD ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than TBD ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.1.3	NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

14.5.1.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clauses 14.5.1.0.1.1 and 14.5.1.0.1.2.

14.5.1.3.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.3.

14.5.1.3.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in Table 14.5.1.3.4.1-1 and 14.5.1.3.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.

UE is configured with 2 overlapping SMTC for the intra-frequency measurement. The SMTC periodicity is 40ms, and SMTC1 is associated with Cell 1 with offset 0, and SMTC2 is associated with Cell 2 with offset 3ms.

14.5.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.3.4.1-1.

Table 14.5.1.3.4.1-1: Supported test configurations for NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.3-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.3-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.3.4.1-2.

Table 14.5.1.3.4.1-2: Initial conditions for NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.3.4.1-3.

2.	Message contents are defined in clause 14.5.1.3.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.3.4.1-3: General test parameters for NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| NTN reference configuration |  | 1 | TBD | For GSO |
|  |  | 2 | TBD | For NGSO |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  |
| SMTC1 configuration |  | 1, 2 | TBD |  |
| SMTC2 configuration |  | 1, 2 | TBD |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 2 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

14.5.1.3.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.3.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, and SMTC configuration according to Table 14.5.1.3.4.1-3.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.3.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by event A3. If the overall delay measured from the beginning of time period T2 is less than [TBD] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.1.3.4.3	Message contents

FFS

14.5.1.3.5	Test requirements

Tables 14.5.1.3.4.1-3 and 14.5.1.3.5-1 define the primary level settings including test tolerances for NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access.

Table 14.5.1.3.5-1: Cell specific test parameters for NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | TBD |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2 | DLBWP.0,1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than TBD ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.1.4	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets or are TBD

- Whether GNSS configuration is needed is FFS

14.5.1.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA intra-frequency NR cell search requirements in clause 14.5.1.0.2.

14.5.1.4.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.4.

14.5.1.4.4	Test description

Two cells are deployed in the test, which are NR FR1 PCell (Cell 1) and a NR FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Tables 14.5.1.4.4.1-1, 14.5.1.4.4.1-3 and 14.5.1.4.5-1 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.4.1. The UE shall be provided with the valid information about the SAN serving each cell before the test.]

14.5.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.4.4.1-1.

Table 14.5.1.4.4.1-1: Supported test configurations for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.4-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.4-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.4.4.1-2.

Table 14.5.1.4.4.1-2: Initial conditions for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.4.4.1-3.

2.	Message contents are defined in clause 14.5.1.4.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.4.4.1-3: General test parameters for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |
| NTN reference configuration |  | 1 | TBD | For GSO |
|  |  | 2 | TBD | For NGSO |
| Measurement gap type |  | 1, 2 | Per-UE gaps |  |
| Gap Pattern ID |  | 1, 2 | 0 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap repetition periodicity | ms | 1, 2 | 40 |  |
| Measurement gap length | ms | 1, 2 | 6 |  |
| Measurement gap offset | ms | 1, 2 | 39 |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2 |  | OFF |
| Time offset between serving and neighbour cells |  | 1,2 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

14.5.1.4.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

UE is configured with 1 SMTC for the intra-frequency measurement. Both Cell 1 and Cell 2 are associated with the configured SMTC.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.4.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, gap configuration, and SMTC configuration according to Tables 14.5.1.4.4.1-3 and 14.5.1.4.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.4.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by event A3. If the overall delay measured from the beginning of time period T2 is less than [802] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10.	Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.1.4.4.3	Message contents

FFS

14.5.1.4.5	Test requirements

Tables 14.5.1.4.4.1-3 and 14.5.1.4.5-1 define the primary level settings including test tolerances for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access.

Table 14.5.1.4.5-1: Cell specific test parameters for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | 1, 2 | SMTC.2 |  | SMTC.2 |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.1.2 FDD resource #0 |  | N/A |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.2 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.1.5	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets or TBD

- Whether GNSS configuration is needed is FFS

14.5.1.5.1	Test purpose

The purpose of this test is to verify that the multiple gaps capable UE makes correct reporting of an event. This test will partly verify the SA intra-frequency NR cell search requirements in clause 14.5.1.0.2.

14.5.1.5.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and 2 parallel measurement gaps (parallelMeasurementGap-r17, TS 38.306).

14.5.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.5.

14.5.1.5.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Tables 14.5.1.5.4.1-1, 14.5.1.5.4.1-3 and 14.5.1.5.5-1 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The UE is configured with 2 concurrent and fully non-overlapping (FNO) measurement gaps for the intra-frequency measurement. Serving Cell 1 is expected to be measured within MeasGapId #0 and Neighbour Cell 2 is expected to be measured within MeasGapId #1.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.4.1. The UE shall be provided with the valid information about the SAN serving each cell before the test.]

14.5.1.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.5.4.1-1.

Table 14.5.1.5.4.1-1: Supported test configurations for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.5-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.5-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.5.4.1-2.

Table 14.5.1.5.4.1-2: Initial conditions for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.5.4.1-3.

2.	Message contents are defined in clause 14.5.1.5.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.5.4.1-3: General test parameters for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |  |
| NTN reference configuration |  | 1 | TBD |  | For GSO |
|  |  | 2 | TBD |  | For NGSO |
| Measurement gap type |  | 1, 2 | Per-UE gap |  |  |
| Gap Pattern ID |  | 1 | 1 |  | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap repetition periodicity | ms | 1, 2 | 40 |  |  |
| Measurement gap length | ms | 1, 2 | 6 |  |  |
| Measurement gap offset | ms | 1, 2 | 19 for MeasGapId #0 4 for MeasGapId #1 |  |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX. 7 |  |
| Time offset between serving and neighbour cells |  | 1, 2 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 5 | 10 |  |

14.5.1.5.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.5.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, two parallel gap configurations, and SMTC configuration according to Tables 14.5.1.5.4.1-3 and 14.5.1.5.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.5.5-1. T2 starts.

6.	[UE shall transmit a MeasurementReport message triggered by event A3]. If the overall delay measured from the beginning of time period T2 is less than [922] ms for Test 1 or [6402] ms for Test 2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

11. Repeat step 1-10 for each sub-test in Table 14.5.1.5.4.1-3 as appropriate.

14.5.1.5.4.3	Message contents

FFS

14.5.1.5.5	Test requirements

Tables 14.5.1.5.4.1-3 and 14.5.1.5.5-1 define the primary level settings including test tolerances for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access.

Table 14.5.1.5.5-1: Cell specific test parameters for NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | SSB.7 FR1 |  |
| SMTC configuration |  | 1, 2 | SMTC.2 |  | SMTC.Y |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.1.2 FDD resource #0 |  | N/A |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.2 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.1.6	NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets or TBD

- Whether GNSS configuration is needed is FFS

14.5.1.6.1	Test purpose

The purpose of this test is to verify that the multiple gaps capable UE makes correct reporting of an event. This test will partly verify the SA intra-frequency NR cell search requirements in clause 14.5.1.0.2.

14.5.1.6.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and 2 parallel measurement gaps (parallelMeasurementGap-r17, TS 38.306).

14.5.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.6.

14.5.1.6.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Tables 14.5.1.6.4.1-1, 14.5.1.6.4.1-3 and 14.5.1.6.5-1 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The UE is configured with 2 PPO concurrent measurement gaps for the intra-frequency measurement. Serving Cell 1 is expected to be measured within MeasGapId #0 and Neighbour Cell 2 is expected to be measured within MeasGapId #1. And the priority for MeasGapId #1 is higher than the priority for MeasGapId #0.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.4.1. The UE shall be provided with the valid information about the SAN serving each cell before the test.]

14.5.1.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.6.4.1-1.

Table 14.5.1.6.4.1-1: Supported test configurations for NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.6-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.6-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.6.4.1-2.

Table 14.5.1.6.4.1-2: Initial conditions for NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.6.4.1-3.

2.	Message contents are defined in clause 14.5.1.6.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.6.4.1-3: General test parameters for NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |
| NTN reference configuration |  | 1 | TBD | For GSO |
|  |  | 2 | TBD | For NGSO |
| Measurement gap type |  | 1, 2 | Per-UE gap |  |
| Gap Pattern ID |  | 1, 2 | 0 for MeasGapId #0 1 for MeasGapId #1 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap repetition periodicity | ms | 1, 2 | 40ms for MeasGapId #0 80ms for MeasGapId #1 |  |
| Measurement gap length | ms | 1, 2 | 6 |  |
| Measurement gap offset | ms | 1, 2 | 39 for MeasGapId #0 4 for MeasGapId #1 |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 2 | 5 ms | Asynchronous cells. The timing of Cell 2 is 5ms later than the timing of serving Cell 1. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

14.5.1.6.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

The UE is required to report the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.6.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, two parallel gap configurations, and SMTC configuration according to Tables 14.5.1.6.4.1-3 and 14.5.1.6.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.6.5-1. T2 starts.

6.	[UE shall transmit a MeasurementReport message triggered by event A3]. If the overall delay measured from the beginning of time period T2 is less than [1242] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10.	Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.1.6.4.3	Message contents

FFS

14.5.1.6.5	Test requirements

Tables 14.5.1.6.4.1-3 and 14.5.1.6.5-1 define the primary level settings including test tolerances for NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access.

Table 14.5.1.6.5-1: Cell specific test parameters for NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | 1, 2 | SMTC.2 |  | SMTC.Y |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.1.2 FDD resource #0 |  | N/A |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.2 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1240 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 14.5.2	Inter-frequency Measurements for SAN

#### 14.5.2.0	Minimum conformance requirements

14.5.2.0.1	Minimum conformance requirements for inter-frequency measurements with measurement gaps

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 14.5.2.0.1-1.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 14.5.2.0.1-2.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 14.5.2.0.1-3.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in 38.133 [6] clause 9.1D.5.2 for measurement conducted within measurement gaps.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. When the UE is configured with concurrent measurement gaps and the two measurement gaps are fully overlapping with MGRP=160ms, Kgap = 2. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in 38.133 [6] section 9.1.8.3.

Kgap is only applicable for UE supporting parallelMeasurementGap-r17. When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0, or if one SMTC overlaps more than one MGs associated to the frequency layer.

K_satellite: it is a statellite specific scaling factor.

- If SMTCs within a measurement gap do not overlap with each other, and if LEO satellite(s) is/are required to be measured within SMTC

  - K_satellite = 1, if GSO satellites are measured on the carrier

  - K_satellite=Num of LEO satellites to be measured in the SMTCnumber of LEO satellites UE is capable to measure in one SMTC, if LEO satellites are measured on the carrier.

- If SMTCs within a measurement gap partially overlap with each other, and if LEO and/or GEO satellite(s) is/are required to be measured within overlapped SMTCs

  - K_satellite=number of overlapped SMTCs, if only GEO satellites are measured on the carrier

  - K_satellite=iNum of LEO satellites to be measured in the SMTC inumber of LEO satellites UE is capable to measure in one SMTC, if only LEO satellites are measured on the carrier.

Table 14.5.2.0.1-1: Time period for PSS/SSS detection (Frequency range FR1)

| Condition NOTE1 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Ceil(8 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(8 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

Table 14.5.2.0.1-2: Time period for time index detection (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, Ceil(3 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(3 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in 38.133 [6] clauses 10.1C.4, 10.1C.5, 10.1C.9, 10.1C.10, 10.1C.14 and 10.1C.15, respectively, as shown in table 14.5.2.0.1-3.

Table 14.5.2.0.1-3: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition NOTE1 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(8 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(8 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in 38.133 [6] clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.3C.4 and 9.3C.5.

14.5.2.0.2	Minimum conformance requirements for inter-frequency measurements with measurement gaps

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index. It is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1, the following conditions are met:

-	SFN and frame boundary across serving cell and inter-frequency neighbor cells is aligned, and

-	the timing of SSBs across serving cell and inter-frequency neighbor cells are aligned

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 14.5.2.0.2-1.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 14.5.2.0.2-2.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 14.5.2.0.2-3.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in 38.133 [6] clause 9.1D.5.1 for measurement conducted outside measurement gaps, i.e. when inter-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps or according to CSSFwithin_gap,i in 38.133 [6] clause 9.1D.5.2 for measurement conducted within measurement gaps, i.e. when inter-frequency SMTC is fully overlapping with measurement gaps.

Kp is the scaling factor for a SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where

-	If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP max is the maximum MGRP across all configured per-UE measurement gap. Otherwise, MGRP max is the MGRP of configured measurement gap.

-	Starting from the beginning of any SMTC occasion:

-	Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and

-	Navailable_SAN is the number of SMTC occasions within the window W that don’t collide with any non-dropped MG occasion within or outside the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in 38.133 [6] section 9.1C.8.3. The collision rule between SMTC occasion and measurement gap occasion is defined in 38.133 [6] section 9.1C.9.1

Kp = [1] when Navailable_SAN = 0 and measurement gap sharing in 38.133 [6] clause 9.1.2.1a shall apply.

Kp = 1 when inter-frequency SMTC is fully non overlapping with measurement gaps.

Ksatellite: it is a satellite specific scaling factor.

- If SMTCs do not overlap with each other, and if LEO and/or GEO satellite(s) is/are required to be measured within SMTC

  - Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier

  - Ksatellite=Num of LEO satellites to be measured in the SMTCnumber of LEO satellites UE is capable to measure in one SMTC, if LEO satellite(s) is/are measured on the carrier.

- If SMTCs partially overlap with each other, and if LEO and/or GEO satellite(s) is/are required to be measured within overlapped SMTCs

  - Ksatellite=number of overlapped SMTCs, if only GEO satellite(s) is/are measured on the carrier

  - Ksatellite=iNum of LEO satellites to be measured in the SMTC inumber of LEO satellites UE is capable to measure in one SMTC, if only LEO satellite(s) is/are measured on the carrier.

Table 14.5.2.0.2-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

Table 14.5.2.0.2-2: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in 38.133 [6] clauses 10.1C.4, 10.1C.5, 10.1C.9, 10.1C.10, 10.1C.14 and 10.1C.15, respectively, as shown in table 14.5.2.0.2-3, if UE supports inter-frequency measurement without measurement gaps:

Table 14.5.2.0.2-3: Measurement period for inter-frequency measurements without gaps ((FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter K_satellite |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter K_satellite |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFinter K_satellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.3C.7.1 and 9.3C.7.2.

#### 14.5.2.1	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptions to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

- Whether GNSS configuration is needed is FFS

14.5.2.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 14.5.2.0.1.

14.5.2.1.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.2.1.

14.5.2.1.4	Test description

Two cells are deployed in the test, which are NR FR1 PCell (Cell 1) on NR RF channel 1 and NR FR1 neighbour cell (Cell 2) on NR RF channel 2. The test parameters for PCell and neighbour cell are given in Tables 14.5.2.1.4.1-1, 14.5.2.1.4.1-3 and 14.5.2.1.5-1 below. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. Measurement gap pattern configuration # 0 is provided to the UE.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.]

14.5.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.2.1.4.1-1.

Table 14.5.2.1.4.1-1: Supported test configurations for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.2.1-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.2.1-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.2.1.4.1-2.

Table 14.5.2.1.4.1-2: Initial conditions for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.2.1.4.1-3.

2.	Message contents are defined in clause 14.5.2.1.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.2.1.4.1-3: General test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NR RF Channel Number |  | Config 1,2 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |
| Hysteresis | dB | Config 1,2 | 0 |  |
| CP length |  | Config 1,2 | Normal |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |
| Filter coefficient |  | Config 1,2 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | Config 1,2 | 5 |  |
| T2 | s | Config 1,2 | 1 |  |

14.5.2.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.2.1.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an inter-frequency measurement object, event A3, gap configuration, and SMTC configuration according to Tables 14.5.2.1.4.1-3 and 14.5.2.1.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.2.1.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by event A3. If the overall delay measured from the beginning of time period T2 is less than [922] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.2.1.4.3	Message contents

FFS

14.5.2.1.5	Test requirements

Tables 14.5.2.1.4.1-3 and 14.5.2.1.5-1 define the primary level settings including test tolerances for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access.

Table 14.5.2.1.5-1: Cell specific test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  |
| Satellite information |  |  | Config 1 | SSC.1 |  | NSC.1 |  |
|  |  |  | Config 2 | SSC.2 |  | NSC.2 |  |
| Duplex mode |  |  | Config 1,2 | FDD |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
| BWP configuration | Initial DL BWP | Config 1 | Config 1,2 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.1 FDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 FDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 FDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,2 | CCR.1.1 FDD |  |  |  |
| SSB parameters |  |  | Config 1,2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.2 |  | SMTC.5 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -98+TT |  | -98+TT |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT | -Infinity | -91+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT | -70.05+TT | -62.26+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

The UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.2.2	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptions to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

- Whether GNSS configuration is needed is FFS

14.5.2.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 14.5.2.0.1.

14.5.2.2.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and long DRX cycle.

14.5.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.2.2.

14.5.2.2.4	Test description

Two cells are deployed in the test, which are NR FR1 PCell (Cell 1) on NR RF channel 1 and NR FR1 neighbour cell (Cell 2) on NR RF channel 2. The test parameters for PCell and neighbour cell are given in Tables 14.5.2.2.4.1-1, 14.5.2.2.4.1-3, 14.5.2.2.5-1, 14.5.2.2.5-2 and 14.5.2.2.5-3 below. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. Measurement gap pattern configuration # 0 is provided to the UE.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.]

14.5.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.2.2.4.1-1.

Table 14.5.2.2.4.1-1: Supported test configurations for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.2.2-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.2.2-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.2.2.4.1-2.

Table 14.5.2.2.4.1-2: Initial conditions for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.2.2.4.1-3.

2.	Message contents are defined in clause 14.5.2.2.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbour cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.2.2.4.1-3: General test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 |  | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 |  |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |
| CP length |  | Config 1,2 | Normal |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2 | DRX.1 | DRX. 7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | Config 1,2 | 5 |  |  |
| T2 | s | Config 1,2 | [1.1] | [11] |  |

14.5.2.2.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

The UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.2.2.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an inter-frequency measurement object, event A3, gap configuration, DRX configuration, and SMTC configuration according to Tables 14.5.2.2.4.1-3 and 14.5.2.2.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.2.2.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by event A3. If the overall delay measured from the beginning of time period T2 is less than [1082] ms for Test 1 or less than [10242] ms for Test 2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10.	Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

11. Repeat step 1-10 for each sub-test in Table 14.5.2.2.4.1-3 as appropriate.

14.5.2.2.4.3	Message contents

FFS

14.5.2.2.5	Test requirements

Tables 14.5.2.2.4.1-3, 14.5.2.2.5-1, 14.5.2.2.5-2 and 14.5.2.2.5-3 define the primary level settings including test tolerances for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access.

Table 14.5.2.2.5-1: Cell specific test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  |
| Satellite information |  |  | Config 1 | SSC.1 |  | NSC.1 |  |
|  |  |  | Config 2 | SSC.2 |  | NSC.2 |  |
| Duplex mode |  |  | Config 1,2 | FDD |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1,2 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.1 FDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 FDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 FDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,2 | CCR.1.1 FDD |  |  |  |
| SSB parameters |  |  | Config 1,2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.2 |  | SMTC.5 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -98+TT |  | -98+TT |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT | -Infinity | -91+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT | -70.05+TT | -62.2+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table 14.5.2.2.5-2: DRX-Configuration for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [13] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | ms640 |  |
| shortDRX | disable | disable |  |

Table 14.5.2.2.5-3: TimeAlignmentTimer configuration for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [13] |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [1080] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [10240] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.2.3	NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

- Whether GNSS configuration is needed is FFS

14.5.2.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 14.5.2.0.1.

14.5.2.3.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.2.3.

14.5.2.3.4	Test description

Two cells are deployed in the test, which are NR FR1 PCell (Cell 1) on NR RF channel 1 and NR FR1 neighbour cell (Cell 2) on NR RF channel 2. The test parameters for PCell and neighbour cell are given in Tables 14.5.2.3.4.1-1, 14.5.2.3.4.1-3 and 14.5.2.3.5-1 below. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. Measurement gap pattern configuration # 0 is provided to the UE.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.]

14.5.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.2.3.4.1-1.

Table 14.5.2.3.4.1-1: Supported test configurations for NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.2.3-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.2.3-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.2.3.4.1-2.

Table 14.5.2.3.4.1-2: Initial conditions for NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.2.3.4.1-3.

2.	Message contents are defined in clause 14.5.2.3.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.2.3.4.1-3: General test parameters for NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Parameter | Unit | Test configuration | Value | Comment |
| NR RF Channel Number |  | Config 1,2 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |
| Hysteresis | dB | Config 1,2 | 0 |  |
| CP length |  | Config 1,2 | Normal |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |
| Filter coefficient |  | Config 1,2 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | Config 1,2 | 5 |  |
| T2 | s | Config 1,2 | [1.1] |  |

14.5.2.3.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

The UE is required to report the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.2.3.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an inter-frequency measurement object, event A3, gap configuration, and SMTC configuration according to Tables 14.5.2.3.4.1-3 and 14.5.2.3.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.2.3.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by event A3. If the overall delay measured from the beginning of time period T2 is less than [1042] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.2.3.4.3	Message contents

FFS

14.5.2.3.5	Test requirements

Tables 14.5.2.3.4.1-3 and 14.5.2.3.5-1 define the primary level settings including test tolerances for NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access.

Table 14.5.2.3.5-1: Cell specific test parameters for NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,2 | FDD |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
| BWP configuration | Initial DL BWP | Config 1 | Config 1,2 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.1 FDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 FDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 FDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,2 | CCR.1.1 FDD |  |  |  |
| SSB parameters |  |  | Config 1,2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.2 |  | SMTC.5 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -98+TT |  | -98+TT |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT | -Infinity | -91+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT | -70.05+TT | -62.2+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [1040] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

The UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.2.4	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptions to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

- Whether GNSS configuration is needed is FFS

14.5.2.4.1	Test purpose

The purpose of this test is to verify that the multiple gaps capable UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 14.5.2.0.1.

14.5.2.4.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and 2 parallel measurement gaps (parallelMeasurementGap-r17, TS 38.306).

14.5.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.2.4.

14.5.2.4.4	Test description

Three cells are deployed in the test, which are NR FR1 PCell (Cell 1) on NR RF channel 1, NR FR1 neighbour cell (Cell 2) on NR RF channel 2, and NR FR1 neighbour cell (Cell 3) also on NR RF channel 2. The test parameters for PCell and neighbour cell are given in Tables 14.5.2.4.4.1-1, 14.5.2.4.4.1-3 and 14.5.2.4.5-1 below. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. Two parallel non-overlapping measurement gap pattern configurations, MeasGapId #0 and MeasGapId #1, are provided to the UE.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.]

14.5.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.2.4.4.1-1.

Table 14.5.2.4.4.1-1: Supported test configurations for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.2.4-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.2.4-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.2.4.4.1-2.

Table 14.5.2.4.4.1-2: Initial conditions for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.2.4.4.1-3.

2.	Message contents are defined in clause 14.5.2.4.4.3.

3.	Cell 1, Cell 2 and Cell 3 are NR cells (PCell and two neighbor cells, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1, Cell 2 and Cell 3 are satellite access cells].

Table 14.5.2.4.4.1-3: General test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR Cell2 and NR Cell 3 | NR Cell 2 and NR Cell 3 are on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 for MeasGapId #0 0 for MeasGapId #1 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 for MeasGapId #0 19 for MeasGapId #1 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |
| Hysteresis | dB | Config 1,2 | 0 |  |
| CP length |  | Config 1,2 | Normal |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |
| Filter coefficient |  | Config 1,2 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF | DRX is not used |
| Time offset between serving and neighbour cell 2,3 |  | Config 1,2 | 3ms | Asynchronous cells. The timing of Cell 2 and Cell 3 is 3ms later than the timing of Cell 1. |
| T1 | s | Config 1,2 | 5 |  |
| T2 | s | Config 1,2 | [1] |  |

14.5.2.4.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.2.4.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an inter-frequency measurement object, event A3, two parallel gap configurations, and two SMTC configurations, respectively, according to Tables 14.5.2.4.4.1-3 and 14.5.2.4.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.2.4.5-1. T2 starts.

6.	[UE shall transmit a MeasurementReport message triggered by event A3]. If the overall delay measured from the beginning of time period T2 is less than [922] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.2.4.4.3	Message contents

FFS

14.5.2.4.5	Test requirements

Tables 14.5.2.4.4.1-3 and 14.5.2.4.5-1 define the primary level settings including test tolerances for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access.

Table 14.5.2.4.5-1: Cell specific test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  | 2 |  |  |
| Satellite information |  |  | Config 1 | SSC.1 |  | NSC.1 |  |  |  |  |
|  |  |  | Config 2 | SSC.2 |  | NSC.2 |  |  |  |  |
| Duplex mode |  |  | Config 1,2 | FDD |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1,2 | DLBWP.0.1 |  | NA |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.1 FDD |  | NA |  | NA |  |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 FDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 FDD |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,2 | CCR.1.1 FDD |  |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2 | SSB.1 FR1 |  | SSB.5 FR1 |  | SSB.1 FR1 |  |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.2 |  | SMTC.Y |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2 | 0 |  | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT | -Infinity | -91+TT | -Infinity |  | -91+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity |  | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity |  | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT | -70.05+TT | -62.26+TT | -70.05+TT |  | -62.26+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [920] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.2.5	Void

#### 14.5.2.6	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptions to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

- Whether GNSS configuration is needed is FFS

- Test requirement is TBD in 38.133

14.5.2.6.1	Test purpose

The purpose of this test is to verify that the multiple gaps capable UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 14.5.2.0.1.

14.5.2.6.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and 2 parallel measurement gaps (parallelMeasurementGap-r17, TS 38.306).

14.5.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.2.6.

14.5.2.6.4	Test description

Three cells are deployed in the test, which are NR FR1 PCell (Cell 1) on NR RF channel 1, NR FR1 neighbour cell (Cell 2) on NR RF channel 2, and NR FR1 neighbour cell (Cell 3) also on NR RF channel 2. The test parameters for PCell and neighbour cell are given in Tables 14.5.2.6.4.1-1, 14.5.2.6.4.1-3 and 14.5.2.6.5-1 below. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. Two parallel partially overlapping measurement gap pattern configurations, MeasGapId #0 and MeasGapId #1, are provided to the UE.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.]

14.5.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.2.6.4.1-1.

Table 14.5.2.6.4.1-1: Supported test configurations for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.2.6-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.2.6-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.2.6.4.1-2.

Table 14.5.2.6.4.1-2: Initial conditions for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.2.6.4.1-3.

2.	Message contents are defined in clause 14.5.2.6.4.3.

3.	Cell 1, Cell 2 and Cell 3 are NR cells (PCell and two neighbor cells, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1, Cell 2 and Cell 3 are satellite access cells].

Table 14.5.2.6.4.1-3: General test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR Cell2 and NR Cell 3 | NR Cell 2 and NR Cell 3 are on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 for MeasGapId #0 1 for MeasGapId #1 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 79 for MeasGapId #0 4 for MeasGapId #1 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |
| Hysteresis | dB | Config 1,2 | 0 |  |
| CP length |  | Config 1,2 | Normal |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |
| Filter coefficient |  | Config 1,2 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF | DRX is not used |
| Time offset between serving and neighbour cell 1 |  | Config 1,2 | 3s | Cell 1 and Cell 2 are synchronous. |
| Time offset between serving and neighbour cell 2 |  | Config 1,2 | 5ms | Cell 1 and Cell 3 are asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1. |
| T1 | s | Config 1,2 | 5 |  |
| T2 | s | Config 1,2 | 1.5 |  |

14.5.2.6.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.2.6.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an inter-frequency measurement object, event A3, two parallel gap configurations, and two SMTC configurations, respectively, according to Tables 14.5.2.6.4.1-3 and 14.5.2.6.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.2.6.5-1. T2 starts.

6.	[UE shall transmit a MeasurementReport message triggered by event A3]. If the overall delay measured from the beginning of time period T2 is less than [TBD] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.2.6.4.3	Message contents

FFS

14.5.2.6.5	Test requirements

Tables 14.5.2.6.4.1-3 and 14.5.2.6.5-1 define the primary level settings including test tolerances for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access.

Table 14.5.2.6.5-1: Cell specific test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  | 2 |  |  |
| Satellite information |  |  | Config 1 | SSC.1 |  | NSC.1 |  |  |  |  |
|  |  |  | Config 2 | SSC.2 |  | NSC.2 |  |  |  |  |
| Duplex mode |  |  | Config 1,2 | FDD |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1,2 | DLBWP.0.1 |  | NA |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.1 FDD |  | NA |  | NA |  |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 FDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 FDD |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,2 | CCR.1.1 FDD |  |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.2 |  | SMTC.2 |  | SMTC.Y |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2 | 0 |  | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT | -Infinity | -91+TT | -Infinity |  | -91+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity |  | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity |  | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT | -70.05+TT | -62.26+TT | -70.05+TT |  | -62.26+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# 15	FFS

# 16	NR standalone tests with all NR cells in FR1 for RedCap

## 16.0	General

This clause contains test scenarios for NR standalone for Reduced Capability UE. All NR cells are in Frequency Range 1.

## 16.1	RRC_IDLE state mobility

### 16.1.1	NR cell re-selection

#### 16.1.1.0	Minimum conformance requirements

##### 16.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS 38.304 [30] within Tdetect,NR_Intra_RedCap when that Treselection= 0 . An intra frequency cell is considered to be detectable according to the conditions defined in TS 38.133 [6] Annex B.x.y for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra_RedCap for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra_RedCap/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS 38.304 [30] within Tevaluate,NR_Intra_RedCap when Treselection = 0 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 for 2 Rx RedCap.

-	the cell is at least 4dB better ranked in FR1 for 1 Rx RedCap.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS 38.304 [30] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3dB better ranked in FR1 if the current serving cell is among them for 2 Rx RedCap.

-	the cell is at least 4dB better ranked in FR1 if the current serving cell is among them for 1 Rx RedCap.

The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signalled value of absThreshSS-BlocksConsolidation [2] + 1 dB.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non-zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS 38.304 [30], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For 1 Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap are specified in Table 16.1.1.0.1-1. For 2 Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap are same as Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra specified in TS 38.133 [6] in Table 4.2.2.3-1.

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap are specified in Table 16.1.1.0.1-2 for FR1, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap when multiple PTWs are used.

Table 16.1.1.0.1-1: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra_RedCap [s] (number of DRX cycles) | Tmeasure,NR_Intra_RedCap [s] (number of DRX cycles) | Tevaluate,NR_Intra_RedCap [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for RedCap UE of all FR2 power class. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra_RedCap is expected. |  |  |  |  |  |

Table 16.1.1.0.1-2: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥[1.28] ([1]) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x M2 (1 x M2) | 0.64 x M2 (2 x M2) |
|  | 0.64 | ≥[1.28] ([1]) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥[2.56] ([2]) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥[5.12] ([4]) |  | 2.56 (1) | 5.12 (2) |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 5: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

##### 16.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS 38.304 [30] within Kcarrier_RedCap * Tdetect,NR_Inter_RedCap  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met

For 2 Rx RedCap by a margin of at least

5 dB in FR1 for reselections based on ranking or

6 dB in FR1 for SS-RSRP reselections based on absolute priorities or

4 dB in FR1 for SS-RSRQ reselections based on absolute priorities

For 1 Rx RedCap by a margin of at least

[5 dB] in FR1 or for reselections based on ranking or

[6 dB] in FR1 for SS-RSRP reselections based on absolute priorities or

[4 dB] in FR1 for SS-RSRQ reselections based on absolute priorities.

The parameter Kcarrier_RedCap is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.x.y for a corresponding Band. When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter_RedCap. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier_RedCap * Tmeasure,NR_Inter_RedCap for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within Kcarrier * Tevaluate,NR_Inter_RedCap when Treselection = 0 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

-	when rangeToBestCell is not configured:

-	the cell is at least 5dB better ranked in FR1 for 2 Rx RedCap.

-	the cell is at least [5dB] better ranked in FR1 for 1 Rx RedCap.

-	when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value defined in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 if the current serving cell is among them, or 6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities for 2 Rx RedCap or 4dB in FR1 or 4dB in FR2 for SS-RSRQ reselections based on absolute priorities for 2 Rx RedCap.

-	the cell is at least [5dB] better ranked in FR1 if the current serving cell is among them, or [6dB] in FR1 for SS-RSRP reselections based on absolute priorities or [4dB] in FR1 for SS-RSRQ reselections based on absolute priorities for 1 Rx RedCap.

The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signalled value of absThreshSS-BlocksConsolidation [2] + 1 dB.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non-zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2B.2.4-1 or Table 4.2.2.4-1 for 1 Rx RedCap and 2 Rx RedCap respectively, under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion defined in TS38.304 [1].

For UE not configured with eDRX_IDLE cycle, Tdetect,NR_Inter_RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in Table 4.2B.2.4.1-1.

For 1 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in Table 4.2B.2.4-2 for FR1.  For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in Table 4.2B.2.4-2 and Table 4.2B.2.4-3 for FR1 and FR2 respectively. The requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap when multiple PTWs are used.

Table 4.2B.2.4-1: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for RedCap UE of all FR2 power class. |  |  |  |  |  |

Table 4.2B.2.4-2: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |

Table 4.2B.2.4-3: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |  | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 3 | 58.88 x N1 (23 x N1) |  | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) |  | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - | 3 | 235.52 x N1 (23 x N1) |  | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) |  | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| NOTE 1: Applies for RedCap UE of all power class. NOTE 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 4: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 5: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

##### 16.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion

This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement criteria:

-	Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in TS 38.304 [30],

-	Relaxed measurement criterion for a stationary UE not at cell edge defined in clause 5.2.4.9.4 in 38.304 [30],

-	Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in 38.304 [30] respectively.

The 1 Rx RedCap UE for the evaluation of one or more relaxed measurement criteria defined in clause 5.2.4.9 38.304 [30] applies:

- s-SearchDeltaP-r16 as the signalled value of s-SearchDeltaP-r16 [2] - 1 dB.

- s-SearchDeltaP-Stationary-r17 as the signalled value of s-SearchDeltaP-Stationary-r17 [2] - 1 dB.

- s-SearchThresholdP-r16 as the signalled value of s-SearchThresholdP-r16 [2] + 1 dB.

- s-SearchThresholdQ-r16 as the signalled value of s-SearchThresholdQ-r16 [2] + 1 dB.

- s-SearchThresholdP2-r17 as the signalled value of s-SearchThresholdP2-r17 [2] + 1 dB.

- s-SearchThresholdQ2-r17 as the signalled value of s-SearchThresholdQ2-r17 [2] + 1 dB.

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion

The requirements defined in clause 4.2B.2.3 apply for this clause except that:

-	Tdetect,NR_Intra_RedCap_Relax as specified in Table 4.2B.2.9.2-1 and Table 4.2B.2.9.2-2 for 1 Rx RedCap  and 2 Rx RedCap respectively.

-	Tmeasure,NR_Intra_RedCap_Relax as specified in Table 4.2B.2.9.2-1 and Table 4.2B.2.9.2-2 for 1 Rx RedCap  and 2 Rx RedCap respectively.

-	Tevaluate,NR_Intra_RedCap_Relax as specified in Table 4.2B.2.9.2-1 and Table 4.2B.2.9.2-2 for 1 Rx RedCap  and 2 Rx RedCap respectively.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 4.2B.2.9.2-3 and Table 4.2B.2.9.2-4 are applicable for eDRX cycle up to 10.24 s in FR1 and FR2 respectively.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, then the requirements in Table 4.2B.2.9.2-5 and Table 4.2B.2.9.2-6 respectively apply provided eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

Table 4.2B.2.9.2-1: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UEs fulfilling stationary criterion for 1 Rx RedCap UE

| DRX cycle length [s] | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x M2 x K3 (36 x M2 x K3) | 1.28 x M2 x K3 (4 x M2 x K3) | 5.12 x M2 x K3 (16 x M2 x K3) |
| 0.64 | 17.92 x K3 (28 x K3) | 1.28 x K3 (2 x K3) | 5.12 x K3 (8 x K3) |
| 1.28 | 32 x K3 (25 x K3) | 1.28x K3 (1 x K3) | 6.4 x K3 (5 x K3) |
| 2.56 | 58.88 x K3 (23 x K3) | 2.56 x K3 (1 x K3) | 7.68 x K3 (3 x K3) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.9.2-2: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UEs fulfilling stationary criterion for 2 Rx RedCap UE

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K3 (36 x N1 x M2 x K3) | 1.28 x N1 x M2 x K3 (4 x N1 x M2 x K3) | 5.12 x N1 x M2 x K3 (16 x N1 x M2 x K3) |
| 0.64 |  | 5 | 17.92 x N1 x K3 (28 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) | 5.12 x N1 x K3 (8 x N1 x K3) |
| 1.28 |  | 4 | 32 x N1 x K3 (25 x N1 x K3) | 1.28 x N1 x K3 (1 x N1 x K3) | 6.4 x N1 x K3 (5 x N1 x K3) |
| 2.56 |  | 3 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x N1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 3: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.9.2-3: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1) for eDRX_IDLE cycle up to 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x K3 (23 x K3) | 2.56 x K3 (1 x K3) | 7.68 x K3 (3 x K3) |
| 5.12 | 117.76 x K3 (23 x K3) | 5.12 x K3 (1 x K3) | 10.24 x K3 (2 x K3) |
| 10.24 | 235.52 x K3 (23 x K3) | 10.24 x K3 (1 x K3) | 20.48 x K3 (2 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.9.2-4: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle up to 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| 5.12 | 117.76 x N1 x K3 (23 x N1 x K3) | 5.12 x N1 x K3 (1 x N1 x K3) | 10.24 x N1 x K3 (2 x N1 x K3) |
| 10.24 | 235.52 x N1 x K3 (23 x N1 x K3) | 10.24 x N1 x K3 (1 x N1 x K3) | 20.48 x N1 x K3 (2 x N1 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.9.2-5: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥[6.4] ([5]) | eDRX_cycle_length×23PTW/DRX_cycle_lengthx K3 (23 x K3) | 0.32 x M2 x K3 (1 x M2 x K3) | 0.64 x M2 x K3 (2 x M2 x K3) |
|  | 0.64 | ≥[12.8] ([10]) |  | 0.64 x K3 (1 x K3) | 1.28 x K3 (2 x K3) |
|  | 1.28 | ≥[15.36] ([12]) |  | 1.28 x K3 (1 x K3) | 2.56 x K3 (2 x K3) |
|  | 2.56 | ≥[30.72] ([24]) |  | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 4: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra_RedCap is expected. Note 5: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.9.2-6: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |  | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥30.72 (24) | 8 | K3 x eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1 x K3) |  | 0.32 x N1 x K3 (1 x N1 x K3) | 0.64 x N1 x K3 (2 x N1 x K3) |
|  | 0.64 | ≥38.4 (30) | 5 |  |  | 0.64 x N1 x K3 (1 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) |
|  | 1.28 | ≥61.44 (48) | 4 |  |  | 1.28 x N1 x K3 (1 x N1 x K3) | 2.56 x N1 x K3 (2 x N1 x K3) |
|  | 2.56 | ≥92.16 (72) | 3 |  |  | 2.56 x N1 x K3 (1 x N1 x K3) | 5.12 x N1 x K3 (2 x N1 x K3) |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 5: The measurement shall not be performed across PTW’s. In this case the measurement is performed in the next available PTW. Note 6: The evaluation shall not be performed across PTW’s. In this case the evaluation is performed in the next available PTW. Note 7: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |  |  |

##### 16.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion

This clause contains the requirements for measurements on inter-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement criteria:

-	Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in 38.304 [30],

-	Relaxed measurement criterion for a stationary UE not at cell edge defined in clause 5.2.4.9.4 in 38.304 [30],

-	Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in 38.304 [30] respectively.

The 1 Rx RedCap UE for the evaluation of one or more relaxed measurement criteria defined in clause 5.2.4.9 38.304 [30] applies:

- s-SearchDeltaP-r16 as the signalled value of s-SearchDeltaP-r16 [2] - 1 dB.

- s-SearchDeltaP-Stationary-r17 as the signalled value of s-SearchDeltaP-Stationary-r17 [2] - 1 dB.

- s-SearchThresholdP-r16 as the signalled value of s-SearchThresholdP-r16 [2] + 1 dB.

- s-SearchThresholdQ-r16 as the signalled value of s-SearchThresholdQ-r16 [2] + 1 dB.

- s-SearchThresholdP2-r17 as the signalled value of s-SearchThresholdP2-r17 [2] + 1 dB.

- s-SearchThresholdQ2-r17 as the signalled value of s-SearchThresholdQ2-r17 [2] + 1 dB.

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion, and

The requirements defined in TS 38.133 [6] in clause 4.2B.2.4 apply for this clause except that:

-	Tdetect,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tmeasure,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tevaluate,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 240.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 4.2B.2.10.2-3 and Table 4.2B.2.10.2-4 are applicable for eDRX cycle up to 10.24 s in FR1 and FR2 respectively.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, then the requirements in Table 4.2B.2.10.2-5 and Table 4.2B.2.10.2-6 respectively apply provided that eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

Table 4.2B.2.10.2-1: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for 1 Rx RedCap UE

| DRX cycle length [s] | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x 1.5 x K4(36 x 1.5 x K4) | 1.28 x 1.5 x K4 (4 x 1.5 x K4) | 5.12 x 1.5 x K4 (16 x 1.5 x K4) |
| 0.64 | 17.92 x K4 (28 x K4) | 1.28x K4 (2 x K4) | 5.12 x K4 (8 x K4) |
| 1.28 | 32 x K4 (25 x K4) | 1.28x K4 (1 x K4) | 6.4 x K4 (5 x K4) |
| 2.56 | 58.88 x K4 (23 x K4) | 2.56 x K4 (1 x K4) | 7.68 x K4 (3 x K4) |
| Note 1: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.10.2-2: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for 2 Rx RedCap UE

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K4 (36 x N1 x 1.5 x K4) | 1.28 x N1 x 1.5 x K4 (4 x N1 x 1.5 x K4) | 5.12 x N1 x 1.5 x K4 (16 x N1 x 1.5 x K4) |
| 0.64 |  | 5 | 17.92x N1 x K4 (28 x N1 x K4) | 1.28 x N1 x K4 (2 x N1 x K4) | 5.12 x N1 x K4 (8 x N1 x K4) |
| 1.28 |  | 4 | 32 x N1 x K4 (25 x N1 x K4) | 1.28 x N1 x K4 (1 x N1 x K4) | 6.4 x N1 x K4 (5 x N1 x K4) |
| 2.56 |  | 3 | 58.88 x N1 x K4 (23 x N1 x K4) | 2.56 x N1 x K4 (1 x N1 x K4) | 7.68 x N1 x K4 (3 x N1 x K4) |
| Note 1: Applies for RedCap UE of all supporting power class. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.10.2-3: Tdetect,E-UTRAN _RedCap_Relax, Tmeasure,NR_,E-UTRAN _RedCap_Relax and Tevaluate,NR_,E-UTRAN _RedCap_Relax for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | Tdetect,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x K3 (23 x K3) | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| 5.12 | 117.76 x K3 (23 x K3) | 5.12 x K3 (1 x K3) | 10.24 x K3 (2 x K3) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.10.2-4: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle up to 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| 5.12 | 117.76 x N1 x K3 (23 x N1 x K3) | 5.12 x N1 x K3 (1 x N1 x K3) | 10.24 x N1 x K3 (2 x N1 x K3) |
| 10.24 | 235.52 x N1 x K3 (23 x N1 x K3) | 10.24 x N1 x K3 (1 x N1 x K3) | 20.48 x N1 x K3 (2 x N1 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

able 4.2B.2.10.2-5: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_ Inter _RedCap_Relax and Tevaluate,NR_ Inter _RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1) for eDRX_IDLE cycle larger than 10.24 s

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥[6.4] ([5]) | eDRX_cycle_length×23PTW/DRX_cycle_lengthx K3 (23 x K3) | 0.32 x M2 x K3 (1 x M2 x K3) | 0.64 x M2 x K3 (2 x M2 x K3) |
|  | 0.64 | ≥[12.8] ([10]) |  | 0.64 x K3 (1 x K3) | 1.28 x K3 (2 x K3) |
|  | 1.28 | ≥[15.36] ([12]) |  | 1.28 x K3 (1 x K3) | 2.56 x K3 (2 x K3) |
|  | 2.56 | ≥[30.72] ([4]) |  | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 4: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.10.2-6: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle larger than 10.24 s

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥30.72 (24) | 8 | K3 x eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1 x K3) | 0.32 x N1 x K3 (1 x N1 x K3) | 0.64 x N1 x K3 (2 x N1 x K3) |
|  | 0.64 | ≥38.4 (30) | 5 |  | 0.64 x N1 x K3 (1 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) |
|  | 1.28 | ≥61.44 (48) | 4 |  | 1.28 x N1 x K3 (1 x N1 x K3) | 2.56 x N1 x K3 (2 x N1 x K3) |
|  | 2.56 | ≥92.16 (72) | 3 |  | 2.56 x N1 x K3 (1 x N1 x K3) | 5.12 x N1 x K3 (2 x N1 x K3) |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 5: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |  |

#### 16.1.1.1	NR SA FR1 Cell reselection for 1 Rx UE

16.1.1.1.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection for 1Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.3.

16.1.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 and forward.

16.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.1.

16.1.1.1.4	Test description

16.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.1.4.1-1.

Table 16.1.1.1.4.1-1: Supported test configurations for NR SA FR1 Cell reselection to FR1 intra-frequency NR case for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.1.1.1.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.1.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.1.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.1.1.1.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.1.4.1-2.

Table 16.1.1.1.4.1-2: Initial conditions for NR SA FR1 Cell reselection to FR1 intra-frequency NR case for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.1.4.1-3.

2.	Message contents are defined in clause 16.1.1.1.4.3.

3.	There is 1 NR carrier, and 2 cells specified in the test. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 16.1.1.1.4.1-3: General test parameters for intra frequency NR cell re-selection test case for 1 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3, 4 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1 |  |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3, 4 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3, 4 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.1.1.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1. Set Cell 2 physical cell identity = initial cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 16.1.1.1.5-1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.1.1.1.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 34 seconds from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7a.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.

7a	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 16.1.1.1.5-1. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 8 seconds from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, continue with step 11a. Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 12.

11a.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 in Cell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

16.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions |  |

z

Table 16.1.1.1.4.3-2: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -65 | -130 is actual value in dBm (-65 * 2 dBm) | 16.1.1.1-1, 16.1.1.1-2 |
|  | -64 | -128 is actual value in dBm (-64 * 2dBm) | 16.1.1.1-3 |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.1.4.3-3: SIB2 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -65 | Actual value is -65*2 = -130dBm | 16.1.1.1-1, 16.1.1.1-2 |
|  | -64 | Actual value is -64*2 = -128dBm | 16.1.1.1-3 |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 |  | (16.1.1.1-1 OR 16.1.1.1-4) AND Cell 1 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.6 |  | (16.1.1.1-1 OR 16.1.1.1-4) AND Cell 2 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.1-2, 16.1.1.1-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.1-1 |
|  | true |  | 16.1.1.1-2, 16.1.1.1-3 |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.1.4.3-4: SIB3: (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |
| IntraFreqNeighCellInfo ::= SEQUENCE{ |  |  |  |
| q-OffsetCell | dB-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.1.1.1.5	Test requirement

Table 16.1.1.1.5-1 defines the primary level settings including test tolerances for intra frequency NR cell re-selection test case in AWGN for 1 Rx UE.

Table 16.1.1.1.5-1: NR cell specific test parameters for intra frequency NR cell re-selection test case in AWGN for 1 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1, 4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1, 4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1, 4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -130 |  |  | -130 |  |  |
|  |  | 3 | -127 |  |  | -127 |  |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4 | 0 |  |  | 0 |  |  |
| Qoffsets, nNote 4 | dB | 1, 2, 3, 4 | -2 |  |  | -2 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1, , 4 | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1, 4 | 16 | 13 | 16 | -infinity | 16 | 13 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 2 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 3 | -79 | -82 | -79 | -infinity | -79 | -82 |
| Io | dBm/9.36 MHz | 1, 4 | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -52.21 | -52.21 |  |  |  |
|  | dBm/18.36 MHz | 3 | -51.02 | -49.30 | -49.30 |  |  |  |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3, 4 | 60 |  |  | 60 |  |  |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Value modified in Test Tolerance analysis in TS 38.903. |  |  |  |  |  |  |  |  |

During T2 the cell re-selection delay to a newly detectable cell shall be less than 34 s.

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

During T3 the cell re-selection delay to an already detected cell shall be less than 8 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on cell 1.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,NR_Intra_RedCap + TSI-NR, and to an already detected cell can be expressed as: Tevaluate,NR_Intra_RedCap + TSI-NR,

Where:

Tdetect,NR_Intra_RedCap	See Table 16.1.1.0.1-1 in clause 16.1.1.0.1

Tevaluate,NR_Intra_RedCap	See Table 16.1.1.0.1-1 in clause 16.1.1.0.1

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 33.28 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.68 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

#### 16.1.1.2	NR SA FR1 Cell reselection for 2 Rx UE

16.1.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection for 2Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.3.

16.1.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward.

16.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.2.

16.1.1.2.4	Test description

16.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.2.4.1-1.

Table 16.1.1.2.4.1-1: Supported test configurations for NR SA FR1 Cell reselection to FR1 intra-frequency NR case for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.1.1.2.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.2.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.2.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.1.1.2.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.2.4.1-2.

Table 16.1.1.2.4.1-2: Initial conditions for NR SA FR1 Cell reselection to FR1 intra-frequency NR case for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.2.4.1-3.

2.	Message contents are defined in clause 16.1.1.2.4.3.

3.	There is 1 NR carrier, and 2 cells specified in the test. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 16.1.1.2.4.1-3: General test parameters for intra frequency NR cell re-selection test case for 2 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3, 4 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1 |  |
| Time offset between cells |  |  | 1, , 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3, 4 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3, 4 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.1.2.4.2	Test procedure

Same test procedure as in 16.1.1.1.4.2, except Table 16.1.1.1.5-1 will be replaced with Table 16.1.1.2.5-1.

16.1.1.2.4.3	Message contents

Same message contents as in 16.1.1.1.4.3.

16.1.1.2.5	Test requirement

Table 16.1.1.2.5-1 defines the primary level settings including test tolerances for intra frequency NR cell re-selection test case in AWGN for 2 Rx UE.

Table 16.1.1.2.5-1: NR cell specific test parameters for intra frequency NR cell re-selection test case in AWGN for 2 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1, 4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1, 4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1, 4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -130 |  |  | -130 |  |  |
|  |  | 3 | -127 |  |  | -127 |  |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1, 4 | 16 | -3.55 | 3.24 | -infinity | 3.24 | -3.55 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1, 4 | 16 | 13 | 16.45 | -infinity | 16.45 | 13 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -82 | -85 | -81.55 | -infinity | -81.55 | -85 |
|  |  | 2 | -82 | -85 | -81.55 | -infinity | -81.55 | -85 |
|  |  | 3 | -79 | -82 | -78.54 | -infinity | -78.54 | -82 |
| Io | dBm/9.36 MHz | 1, 4 | -53.94 | -51.91 | -51.91 | Same as parameters specified in Cell 1 columns- |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -51.91 | -51.91 |  |  |  |
|  | dBm/18.36 MHz | 3 | -51.01 | -48.99 | -48.99 |  |  |  |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3, 4 | 60 |  |  | 60 |  |  |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

During T2 the cell re-selection delay to a newly detectable cell shall be less than 34 s.

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

During T3 the cell re-selection delay to an already detected cell shall be less than 8 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on cell 1.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra		See Table 16.1.1.0.1-1 in clause 16.1.1.0.1

Tevaluate, NR_ intra	See Table 16.1.1.0.1-1 in clause 16.1.1.0.1

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 33.28 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.68 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

#### 16.1.1.3	NR SA FR1-FR1 Cell reselection for 1 Rx UE

16.1.1.3.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection for 1 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.4.

16.1.1.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 and forward.

16.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.3.

16.1.1.3.4	Test description

16.1.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.3.4.1-1.

Table 16.1.1.3.4.1-1: Supported test configurations for FR1-FR1 Cell reselection for 1 Rx UE

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.1.3.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.3.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.3.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.1.1.3.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.3.4.1-2.

Table 16.1.1.3.4.1-2: Initial conditions for FR1-FR1 Cell reselection for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.1.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.3.4.1-3.

2.	Message contents are defined in clause 16.1.1.3.4.3.

3.	There are 2 NR carriers, and 2 cells specified in the test. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

Table 16.1.1.3.4.1-3: General test parameters for FR1-FR1 Cell reselection for 1 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2, 3, 4 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2, 3, 4 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3, 4 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.1.3.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

2.	Set the parameters according to T1 in Table 16.1.1.3.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, Cell 1.

4.	If the UE responds on lower priority cell, Cell 1 during time duration T1 within 8 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”.

5.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5a.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 11.

5a.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 16.1.1.3.5-1. T2 starts. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE.

7.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 16.1.1.3.5-1. T3 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, Cell 2.

9.	If the UE responds on higher priority cell, Cell 2 during time duration T3 within 68 seconds from the beginning of time period T3, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”.

10.	If the UE has re-selected Cell 2 within T3, after the re-selection or when T3 expires, continue with step 10a.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 11.

10a.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

11.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 on Cell 2.

12.	Repeat step 2-11 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

16.1.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.1.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 16.1.1.2-1/4 and Cell 1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for configuration 16.1.1.2-1/4 and Cell 2 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.1 and synchronous cells for configuration 16.1.1.2-2 Table H.2.2-2 with Condition SSB.1 RedCap FR1, SMTC.1 and synchronous cells for configuration 16.1.1.2-3 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions |  |

16.1.1.3.5	Test requirement

Table 16.1.1.3.5-1 defines the primary level settings including test tolerances for FR1-FR1 Cell reselection for 1 Rx UE

Table 16.1.1.3.5-1: NR cell specific test parameters for FR1-FR1 Cell reselection for 1 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1, 4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1, 4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1, 4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1, 4 | 15.6 | 15.6 | 15.6 | -3.7 | -infinity | 13.6 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 | -98 | -98 | -102 | -98 | -98 |
|  |  | 2 | -98 | -98 | -98 | -102 | -98 | -98 |
|  |  | 3 | -95 | -95 | -95 | -99 | -95 | -95 |
| Note2 | dBm/15 kHz | 1, 4 | -98 | -98 | -98 | -102 | -98 | -98 |
|  |  | 2 | -98 | -98 | -98 | -102 | -98 | -98 |
|  |  | 3 | -98 | -98 | -98 | -102 | -99 | -98 |
|  | dB | 1, 4 | 15.6 | 15.6 | 15.6 | -3.7 | -infinity | 13.6 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -82.4 | -82.4 | -82.4 | -105.7 | -infinity | -84.4 |
|  |  | 2 | -82.4 | -82.4 | -82.4 | -105.7 | -infinity | -84.4 |
|  |  | 3 | -79.4 | -79.4 | -79.4 | -102.7 | -infinity | -81.4 |
| Io | dBm/9.36 MHz | 1, 4 | -54.35 | -54.35 | -54.35 | -72.51 | -70.05 | -56.26 |
|  | dBm/9.36 MHz | 2 | -54.35 | -54.35 | -54.35 | -72.51 | -70.05 | -56.26 |
|  | dBm/18.36 MHz | 3 | -51.43 | -51.43 | -51.43 | -72.59 | -70.13 | -56.35 |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3, 4 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

During T1, The cell re-selection delay to a lower priority cell shall be less than 8 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

During T3, the cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 2.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate,NR_Inter_RedCap + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate,NR_Inter_RedCap + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2B.2.7 in 38.133

Tevaluate,NR_Inter_RedCap	See Table 4.2B.2.4-1 in clause 16.1.1.0.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

#### 16.1.1.4	NR SA FR1-FR1 Cell reselection for 2 Rx UE

16.1.1.4.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.4.

16.1.1.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward.

16.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.4.

16.1.1.4.4	Test description

16.1.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.4.4.1-1.

Table 16.1.1.4.4.1-1: Supported test configurations for NR SA FR1-FR1 Cell reselection for 2 Rx UE

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.1.4.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.4.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.4.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.1.1.4.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.4.4.1-2.

Table 16.1.1.4.4.1-2: Initial conditions for FR1-FR1 Cell reselection for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.1.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.4.4.1-3.

2.	Message contents are defined in clause 16.1.1.4.4.3.

3.	There are 2 NR carriers, and 2 cells specified in the test. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

Table 16.1.1.4.4.1-3: General test parameters for FR1-FR1 Cell reselection for 2 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2, 3, 4 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2, 3, 4 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.1.4.4.2	Test procedure

Same test procedure as in 16.1.1.3.4.2 except Table 16.1.1.3.5-1 will be replaced with Table 16.1.1.4.5-1.

16.1.1.4.4.3	Message contents

Same message contents as in 16.1.1.3.4.3.

16.1.1.4.5	Test requirement

Table 16.1.1.4.5-1 defines the primary level settings including test tolerances for FR1-FR1 Cell reselection for 2 Rx UE

Table 16.1.1.4.5-1: NR cell specific test parameters for FR1-FR1 Cell reselection for 2 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1, 4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1, 4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1, 4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1, 4 | 15.6 | 15.6 | 15.6 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  | -100 | -98 | -98 |
|  |  | 2 | -98 |  |  | -100 | -98 | -98 |
|  |  | 3 | -95 |  |  | -97 | -95 | -95 |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  | -100 | -98 | -98 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1, 4 | 15.60 | 15.60 | 15.60 | -3.60 | -infinity | 13.60 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -82.40 | -82.40 | -82.40 | -103.60 | -infinity | -84.40 |
|  |  | 2 | -82.40 | -82.40 | -82.40 | -103.60 | -infinity | -84.40 |
|  |  | 3 | -79.40 | -79.40 | -79.40 | -100.60 | -infinity | -81.40 |
| Io | dBm/9.36 MHz | 1, 4 | -54.33 | -54.33 | -54.33 | -70.48 | -70.05 | -56.26 |
|  | dBm/9.36 MHz | 2 | -54.33 | -54.33 | -54.33 | -70.48 | -70.05 | -56.26 |
|  | dBm/18.36 MHz | 3 | -51.40 | -51.40 | -51.40 | -70.56 | -70.13 | -56.35 |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3, 4 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

During T1, The cell re-selection delay to a lower priority cell shall be less than 8 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

During T3, the cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 2.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate,NR_Inter_RedCap + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate,NR_Inter_RedCap + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2B.2.7 in 38.133

Tevaluate,NR_Inter_RedCap	See Table 4.2B.2.4-1 in clause 16.1.1.0.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

#### 16.1.1.5	NR SA FR1 	Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

16.1.1.5.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection requirements for UE fulfilling stationary relaxed measurement criterion. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.9.2.

16.1.1.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 and forward supporting Rel.17 stationary idle mode relaxed measurement criterion.

16.1.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.5.

16.1.1.5.4	Test description

16.1.1.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.5.4.1-1.

Table 16.1.1.5.4.1-1: Supported test configurations for NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.1.1.5.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.5.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.5.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.1.1.5.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.5.4.1-2.

Table 16.1.1.5.4.1-2: Initial conditions for NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.1.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.5.4.1-3.

2.	Message contents are defined in clause 16.1.1.5.4.3.

3.	There is 1 NR carrier, and 2 cells specified in the test. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 16.1.1.5.4.1-3: General test parameters for NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE reselects to cell 1 during T2 period |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1 |  |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | 32 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4 | 32 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.1.5.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 2 successive time periods, with time duration of T1, and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility.

Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1. stationaryMobilityEvaluation-r17 is provided in SIB2 of Cell 1 and Cell 2. Set Cell 2 physical cell identity = initial cell 2 physical cell identity.

1.	Set the parameters according to T2 in Table 16.1.1.5.5-1. Then wait 54s to ensure that Cell 2 has been detected by the UE.

2.	Set the parameters according to T1 in Table 16.1.1.5.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 2.

4.	If the UE responds on Cell 2 within 32 seconds from the beginning of time period T1, then count a success for the event “Re-select Cell 2”. Otherwise count a fail for the event “Re-select Cell 2”.

5.	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.

6.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 16.1.1.53.5-1. T2 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 1.

9.	If the UE responds on Cell 1 within 32 seconds from the beginning of time period T2, then count a success for the event “Re-select Cell 1”. Otherwise count a fail for the event “Re-select Cell 1”.

10.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, continue with step 11. Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 12.

11.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1. Skip to step 14.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 in Cell 1.

13.	Set the parameters according to T2 in Table 16.1.1.5.5-1. Then wait 54s to ensure that Cell 2 has been detected by the UE.

14.	Repeat step 2-14 until a test verdict has been achieved. Each of the events “Re-select Cell 2” and “Re-select Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict. If both events pass, the test passes. If one event fails, the test fails.

16.1.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.1.5.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions |  |

Table 16.1.1.5.4.3-2: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -69 | -137 is actual value in dBm (-69 * 2+1 dBm) | Config 16.1.1.5.1-3 |
| } |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| downlinkConfigCommon SEQUENCE { |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |
| defaultPagingCycle | rf64 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.5.4.3-3: SIB2 (Cell 1)

| Derivation Path: Table H.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| smtc | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 |  | 16.1.1.5-1, 16.1.1.5-4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.5-2, 16.1.1.5-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.5-1, 16.1.1.5-4 |
|  | true |  | 16.1.1.5-2, 16.1.1.5-3 |
| } |  |  |  |
| relaxedMeasurement-r16 | Not present |  |  |
| cellEquivalentSize-r17 | Not present |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB6 |  | 16.1.1.5.5-1 |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.5.4.3-4: SIB2 (Cell 2)

| Derivation Path: Table H.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| smtc | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.6 |  | 16.1.1.5-1, 16.1.1.5-4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.5-2, 6.1.1.3-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.5-1 |
|  | true |  | 16.1.1.5-2, 6.1.1.3-3 |
| } |  |  |  |
| relaxedMeasurement-r16 | Not present |  |  |
| cellEquivalentSize-r17 | Not present |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB6 |  | 16.1.1.5.5-1 |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.5.4.3-5: SIB3

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |
| IntraFreqNeighCellInfo ::= SEQUENCE{ |  |  |  |
| physCellId | Set according to the neighbour cell PCI |  |  |
| q-OffsetCell | dB1 | 2 is actual value in dB (0 * 2 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.1.1.5.5	Test requirement

Table 16.1.1.5.5-1 defines the primary level settings including test tolerances for FR1-FR1 Cell reselection for 2 Rx UE

Table 16.1.1.5.5-1: NR cell specific test parameters for NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.2.1-1 |  | OP.1 defined in A.2.1-1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -140 |  | -140 |  |
|  |  | 3 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2, 3, 4 | 0 |  | 0 |  |
| Qoffsets, n Note4 | dB | 1, 2, 3, 4 | -2 |  | -2 |  |
| SSearchDeltaP-Stationary Note4 | dB | 1, 2, 3, 4 | 6 |  | 6 |  |
| TSearchDeltaP-Stationary | s | 1, 2, 3, 4 | 5 |  | 5 |  |
| Cell_selection_and_ reselection_quality_ measurement |  | 1, 2, 3, 4 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1, 2, 3, 4 | -2.57 | 2.24 | 2.24 | -2.57 |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note2 | dBm/15 kHz | 1, 2, 3, 4 | -98 |  |  |  |
|  | dB | 1, 2, 3, 4 | 13 | 15.45 | 15.45 | 13 |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -85 | -82.55 | -82.55 | -85 |
|  |  | 2 | -85 | -82.55 | -82.55 | -85 |
|  |  | 3 | -82 | -79.55 | -79.55 | -82 |
| Io | dBm/9.36 MHz | 1, 4 | -52.56 | -52.24 | specified in Cell 1 columns- |  |
|  | dBm/9.36 MHz | 2 | -52.56 | -52.56 |  |  |
|  | dBm/18.36 MHz | 3 | -49.64 | -49.64 |  |  |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3, 4 | 60 |  | 60 |  |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Value modified in Test Tolerance analysis in TS 38.903. |  |  |  |  |  |  |

During T1, the cell re-selection delay to an already detected cell shall be less than 32 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

During T2, the cell re-selection delay to an already detected cell shall be less than 32 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The cell re-selection delay to an already detected cell can be expressed as: Tevaluate,NR_Intra_RedCap_Relax + TSI-NR,

Where:

Tevaluate,NR_Intra_RedCap_Relax	See Table 4.2B.2.9.2-1 in clause 16.1.1.0.3 for reselection to Cell 2 during T1 with UE fulfilling stationary criterion, 30.72 s.

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 32 s for the cell re-selection delay to an already detected cell for UE fulfilling stationary criterion in the test case.

#### 16.1.1.6	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

16.1.1.6.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection requirements for UE fulfilling stationary relaxed measurement criterion. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.9.2.

16.1.1.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward supporting Rel.17 stationary idle mode relaxed measurement criterion.

16.1.1.6.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.6.

16.1.1.6.4	Test description

16.1.1.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.6.4.1-1.

Table 16.1.1.6.4.1-1: Supported test configurations for NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.1.1.6.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.6.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.6.1-3 | 30 kHz SSB SCS, 20MHz bandwidth, TDD duplex mode |
| 16.1.1.6.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.6.4.1-2.

Table 16.1.1.6.4.1-2: Initial conditions for NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.1.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.6.4.1-3.

2.	Message contents are defined in clause 16.1.1.6.4.3.

3.	There is 1 NR carrier, and 2 cells specified in the test. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 16.1.1.6.4.1-3: General test parameters for NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE reselects to cell 1 during T2 period |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1 |  |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | 32 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4 | 32 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.1.6.4.2	Test procedure

Same test procedure as in 16.1.1.5.4.2.

16.1.1.6.4.3	Message contents

Same message content as in 16.1.1.5.4.3.

16.1.1.6.5	Test requirement

Table 16.1.1.6.5-1 defines the primary level settings including test tolerances for FR1-FR1 Cell reselection for 2 Rx UE

Table 16.1.1.6.5-1: NR cell specific test parameters for NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.2.1-1 |  | OP.1 defined in A.2.1-1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -140 |  | -140 |  |
|  |  | 3 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2, 3, 4 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2, 3, 4 | 0 |  | 0 |  |
| SSearchDeltaP-Stationary Note4 | dB | 1, 2, 3, 4 | 6 |  | 6 |  |
| TSearchDeltaP-Stationary | s | 1, 2, 3, 4 | 5 |  | 5 |  |
| Cell_selection_and_ reselection_quality_ measurement |  | 1, 2, 3, 4 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1, 2, 3, 4 | -3.56 | 3.22 | 3.22 | -3.56 |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note2 | dBm/15 kHz | 1, 2, 3, 4 | -98 |  |  |  |
|  | dB | 1, 2, 3, 4 | 12.55 | 16 | 16 | 12.55 |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -85.45 | -82 | -82 | -85.45 |
|  |  | 2 | -85.45 | -82 | -82 | -85.45 |
|  |  | 3 | -82.45 | -79 | -79 | -82.45 |
| Io | dBm/9.36 MHz | 1, 4 | -52.44 | -52.44 | specified in Cell 1 columns- |  |
|  | dBm/9.36 MHz | 2 | -52.44 | -52.44 |  |  |
|  | dBm/18.36 MHz | 3 | -49.43 | -49. 43 |  |  |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3, 4 | 60 |  | 60 |  |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSearchDeltaP includes Test Tolerance applied to fulfil the low mobility criterion. |  |  |  |  |  |  |

During T1, the cell re-selection delay to an already detected cell shall be less than 32 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

During T2, the cell re-selection delay to an already detected cell shall be less than 32 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The cell re-selection delay to an already detected cell can be expressed as: Tevaluate,NR_Intra_RedCap_Relax + TSI-NR,

Where:

Tevaluate,NR_Intra_RedCap_Relax	See Table 4.2B.2.9.2-2 in clause 16.1.1.0.3 for reselection to Cell 2 during T1 with UE fulfilling stationary criterion, 30.72 s.

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 32 s for the cell re-selection delay to an already detected cell for UE fulfilling stationary criterion in the test case.

#### 16.1.1.7	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

16.1.1.7.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection requirements for UE fulfilling stationary relaxed measurement criterion. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.10.2.

16.1.1.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 and forward supporting Rel.17 stationary idle mode relaxed measurement criterion.

16.1.1.7.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.7.

16.1.1.7.4	Test description

16.1.1.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.7.4.1-1.

Table 16.1.1.7.4.1-1: Supported test configurations for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.1.7.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.7.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.7.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.1.1.7.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.7.4.1-2.

Table 16.1.1.7.4.1-2: Initial conditions for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.1.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.7.4.1-3.

2.	Message contents are defined in clause 16.1.1.7.4.3.

3.	There is 1 NR carrier, and 2 cells specified in the test. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 16.1.1.7.4.1-3: General test parameters for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE camps on cell 2 in the initial phase, it fulfils stationary relaxation measurements criterion, and during T1 period the UE reselects to cell 1 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | 32 | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4 | 32 | T2 is defined so that cell re-selection reaction time is taken into account. |

16.1.1.7.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR Cell 1 and Cell 2 on different frequencies. The test consists of 2 successive time periods, with time duration of T1, and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 1.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2. SIB2 of Cell 1 and Cell 2 are configured as in 16.1.1.7.4.3.

2.	Set the parameters according to T1 in Table 16.1.1.7.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 1.

4.	If the UE responds on Cell 1 within 32 seconds from the beginning of time period T1, then count a success for the event “Re-select Cell 1”. Otherwise count a fail for the event “Re-select Cell 1”.

5.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 12.

6.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 16.1.1.7.5-1. T2 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 2.

9.	If the UE responds on Cell 2 within 32 seconds from the beginning of time period T2, then count a success for the event “Re-select Cell 2”. Otherwise count a fail for the event “Re-select Cell 2”.

10.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 11. Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.

11.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2. skip to 13

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 in Cell 2.

13.	Repeat step 2-12 until a test verdict has been achieved. Each of the events “Re-select Cell 2” and “Re-select Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict. If both events pass, the test passes. If one event fails, the test fails.

16.1.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.1.7.4.3-1: SIB1 (Cell 1 and Cell 2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| downlinkConfigCommon SEQUENCE { |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |
| defaultPagingCycle | rf64 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.7.4.3-2: SIB2 (Cell 1)

| Derivation Path: Table H.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| threshServingLowP | 22 | Actual value = 22*2 = 44dB |  |
| } |  |  |  |
| relaxedMeasurement-r16 | Not present |  |  |
| cellEquivalentSize-r17 | Not present |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB3 |  |  |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.7.4.3-3: SIB2 (Cell 2)

| Derivation Path: Table H.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| threshServingLowP | 23 | Actual value = 23*2 = 46dB | Cell 2 |
| } |  |  |  |
| relaxedMeasurement-r16 | Not present |  |  |
| cellEquivalentSize-r17 | Not present |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB15 |  | Cell 2 |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.7.4.3-4: SIB4 (Cell 1)

| Derivation Path: Table H.2.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | ARFCN-ValueNR for Cell 2 |  |  |
| smtc | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.7-2, 16.1.1.7-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | true |  | 16.1.1.7-2, 16.1.1.7-3 |
| threshX-HighP | 24 | Actual value = 24*2 = 48dB |  |
| threshX-LowP | 24 | Actual value = 25*2 = 50dB |  |
| cellReselectionPriority | 5 | Same as the priority in SIB 2 of Cell 2. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.7.4.3-5: SIB4 (Cell 2)

| Derivation Path: Table H.2.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | ARFCN-ValueNR for Cell 1 SSB |  | Cell 2 |
| smtc | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.6 |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.7-2, 16.1.1.7-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | true |  | 16.1.1.7-2, 16.1.1.7-3 |
| threshX-HighP | 22 | Actual value = 22*2 = 44dB | Cell 2 |
| threshX-LowP | 25 | Actual value = 25*2 = 50dB | Cell 2 |
| cellReselectionPriority | 4 | Same as the priority in SIB 2 of Cell 1. | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.1.1.7.5	Test requirement

Table 16.1.1.7.5-1 defines the primary level settings including test tolerances for FR1-FR1 Cell reselection for 2 Rx UE

Table 16.1.1.7.5-1: NR cell specific test parameters for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1, 4 | N/A |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1, 4 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1, 4 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1, 4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.2.1-1 |  | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -140 |  | -140 |  |  |
|  |  | 3 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1, 4 | 14 | 14 | -3.7 | 9.75 |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1, 4 | 14 | 14 | -3.7 |  | 9.75 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -84 | -84 | -101.7 |  | -88.25 |
|  |  | 2 | -84 | -84 | -101.7 |  | -88.25 |
|  |  | 3 | -81 | -81 | -98.7 |  | -85.25 |
| Io | dBm/9.36 MHz | 1, 4 | -55.88 | -55.88 | -68.51 |  | -59.86 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -68.51 |  | -59.86 |
|  | dBm/18.36 MHz | 3 | -52.96 | -52.96 | 65.58 |  | -56.93 |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 |  | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4 | Not sent |  | Not sent |  |  |
| Threshx, highP Note 4 | dB | 1, 2, 3, 4 | 48 |  | 44 |  |  |
| Threshserving, lowP Note 4 | dB | 1, 2, 3, 4 | 44 |  | 46 |  |  |
| Threshx, lowP Note 4 | dB | 1, 2, 3, 4 | 48 |  | 50 |  |  |
| SSearchDeltaP-Stationary Note 4 | dB | 1, 2, 3 | 3 |  | 15 |  |  |
| TSearchDeltaP-Stationary | s | 1, 2, 3 | 5 |  | 5 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Value modified in Test Tolerance analysis in TS 38.903. |  |  |  |  |  |  |  |

During T1, the cell re-selection delay to a lower priority cell for UE fulfilling stationary relaxed measurements shall be less than 32 s.

The cell reselection delay to an already detected lower priority cell for UE fulfilling stationary relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

During T2, the cell re-selection delay to an already detected higher priority cell for UE fulfilling stationary relaxed measurements shall be less than 32 s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling stationary relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The cell re-selection delay to a known lower priority cell can be expressed as: Tevaluate,NR_Inter_RedCap_Relax + TSI-NR,

Where:

Tevaluate,NR_Inter_RedCap_Relax	See Table 4.2B.2.10.2-1 in clause 16.1.1.0.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 32 s for the cell re-selection delay to an already detected lower priority cell and 32 s for the cell re-selection delay to an already detected higher priority cell, for UE fulfilling stationary relaxed measurements in the test case.

#### 16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

16.1.1.8.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection requirements for UE fulfilling stationary relaxed measurement criterion. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.10.2.

16.1.1.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward supporting Rel.17 stationary idle mode relaxed measurement criterion.

16.1.1.8.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.8.

16.1.1.8.4	Test description

16.1.1.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.8.4.1-1.

Table 16.1.1.8.4.1-1: Supported test configurations for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.1.8.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.8.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.8.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.1.1.8.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.8.4.1-2.

Table 16.1.1.8.4.1-2: Initial conditions for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.1.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.8.4.1-3.

2.	Message contents are defined in clause 16.1.1.8.4.3.

3.	There are 2 NR carriers, and 2 cells specified in the test. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 16.1.1.8.4.1-3: General test parameters for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE camps on cell 2 in the initial phase, it fulfils stationary relaxation measurements criterion, and during T1 period the UE reselects to cell 1 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | 32 | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4 | 32 | T2 is defined so that cell re-selection reaction time is taken into account. |

16.1.1.8.4.2	Test procedure

Same test procedure as in 16.1.1.7.4.2.

16.1.1.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.1.8.4.3-1: SIB1 (Cell 1 and Cell 2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| downlinkConfigCommon SEQUENCE { |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |
| defaultPagingCycle | rf64 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.8.4.3-2: SIB2 (Cell 1)

| Derivation Path: Table H.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| threshServingLowP | 22 | Actual value = 22*2 = 44dB |  |
| } |  |  |  |
| cellEquivalentSize-r17 | Not present |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB3 |  |  |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.8.4.3-3: SIB2 (Cell 2)

| Derivation Path: Table H.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| threshServingLowP | 23 | Actual value = 23*2 = 46dB | Cell 2 |
| } |  |  |  |
| cellEquivalentSize-r17 | Not present |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB15 |  | Cell 2 |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.8.4.3-4: SIB4 (Cell 1)

| Derivation Path: Table H.2.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | ARFCN-ValueNR for Cell 2 |  |  |
| smtc | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 |  | 16.1.1.8-1, 16.1.1.8-4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.8-2, 16.1.1.8-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.8-1, 16.1.1.8-4 |
|  | true |  | 16.1.1.8-2, 16.1.1.8-3 |
| threshX-HighP | 24 | Actual value = 24*2 = 48dB |  |
| threshX-LowP | 24 | Actual value = 24*2 = 48dB |  |
| cellReselectionPriority | 5 | Same as the priority in SIB 2 of Cell 2. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.8.4.3-5: SIB4 (Cell 2)

| Derivation Path: Table H.2.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | ARFCN-ValueNR for Cell 1 SSB |  | Cell 2 |
| smtc | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.6 |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.7-2, 16.1.1.7-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | true |  | 16.1.1.7-2, 16.1.1.7-3 |
| threshX-HighP | 22 | Actual value = 22*2 = 44dB | Cell 2 |
| threshX-LowP | 25 | Actual value = 25*2 = 50dB | Cell 2 |
| cellReselectionPriority | 4 | Same as the priority in SIB 2 of Cell 1. | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.1.1.8.5	Test requirement

Table 16.1.1.8.5-1 defines the primary level settings including test tolerances for FR1-FR1 Cell reselection for 2 Rx UE

Table 16.1.1.8.5-1: NR cell specific test parameters for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1, 4 | N/A |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1, 4 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1, 4 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1, 4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.2.1-1 |  | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -140 |  | -140 |  |  |
|  |  | 3 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1, 4 | 14 | 14 | -3.7 | 9.75 |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1, 4 | 14 | 14 | -3.70 |  | 9.75 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -84 | -84 | -101.70 |  | -88.25 |
|  |  | 2 | -84 | -84 | -101.70 |  | -88.25 |
|  |  | 3 | -81 | -81 | -98.7 |  | -85.25 |
| Io | dBm/9.36 MHz | 1, 4 | -55.96 | -55.96 | -68.60 |  | -59.95 |
|  | dBm/9.36 MHz | 2 | -55.96 | -55.96 | -68.60 |  | -59.95 |
|  | dBm/18.36 MHz | 3 | -52.95 | -52.95 | -65.58 |  | -56.93 |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 |  | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4 | Not sent |  | Not sent |  |  |
| Threshx, highP Note 4 | dB | 1, 2, 3, 4 | 48 |  | 44 |  |  |
| Threshserving, lowP Note 4 | dB | 1, 2, 3, 4 | 44 |  | 46 |  |  |
| Threshx, lowP Note 4 | dB | 1, 2, 3, 4 | 48 |  | 50 |  |  |
| SSearchDeltaP-Stationary Note 4 | dB | 1, 2, 3 | 3 |  | 15 |  |  |
| TSearchDeltaP-Stationary | s | 1, 2, 3 | 5 |  | 5 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Thresholds include Test Tolerance applied to fulfil the cell re-selection criterion. |  |  |  |  |  |  |  |

During T1, the cell re-selection delay to a lower priority cell for UE fulfilling stationary relaxed measurements shall be less than 32 s.

The cell reselection delay to an already detected lower priority cell for UE fulfilling stationary relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

During T2, the cell re-selection delay to an already detected higher priority cell for UE fulfilling stationary relaxed measurements shall be less than 32 s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling stationary relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The cell re-selection delay to a known lower priority cell can be expressed as: Tevaluate,NR_Inter_RedCap_Relax + TSI-NR,

Where:

Tevaluate,NR_Inter_RedCap_Relax	See Table 4.2B.2.10.2-2 in clause 16.1.1.0.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 32 s for the cell re-selection delay to an already detected lower priority cell and 32 s for the cell re-selection delay to an already detected higher priority cell, for UE fulfilling stationary relaxed measurements in the test case.

### 16.1.2	NR – E-UTRA cell re-selection

#### 16.1.2.0	Minimum conformance requirements

##### 16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in TS 38.133 [6] in clause 4.2B.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this clause apply for inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

The parameter NEUTRA_carrier_RedCap is the total number of configured E-UTRA carriers in the neighbour frequency list. The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN_RedCap/2.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 [30] within (NEUTRA_carrier_RedCap) * Tdetect,EUTRAN_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least [6dB] for RSRP reselections based on absolute priorities or [4dB] for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

Cells which have been detected shall be measured at least every (NEUTRA_carrier_RedCap) * Tmeasure,EUTRAN_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN_RedCap. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT E-UTRAN carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [30] within (NEUTRA_carrier_RedCap) * Tevaluate,EUTRAN_RedCap when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least [6dB] for RSRP reselections based on absolute priorities or [4dB] for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

If Treselection timer has a non-zero value and the inter-RAT E-UTRA cell is satisfied with the reselection criteria which are defined in TS 38.304 [1], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap are specified in Table 16.1.2.0.1-1.

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap are specified in Table 16.1.2.0.1-2, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap when multiple PTWs are used.

Table 16.1.2.0.1-1: Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap, and Tevaluate,EUTRAN_RedCap for 1 Rx RedCap

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Table 16.1.2.0.1-2: Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap, and Tevaluate,EUTRAN_RedCap for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_RedCap [s] (number of DRX or eDRX cycles Note 3) | Tmeasure,EUTRAN_RedCap [s] (number of DRX or eDRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap [s] (number of DRX or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | N/A | N/A | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | N/A | N/A | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | N/A | N/A | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | (23) | 0.32 (1) | 0.64 (2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN_RedCap*DRX_cycle1.28*1.28. NOTE 5: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

##### 16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx

Same Minimum conformance requirements as in clause 16.1.2.0.1.

##### 16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion, and

The requirements defined in TS 38.133 [6] clause 4.2B.2.5 apply for this clause except that:

-	Tdetect,EUTRAN_RedCap Relax as specified in Table 16.1.2.0.3-1 and Table 16.1.2.0.3-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tmeasure,EUTRAN RedCap Relax as specified in Table 16.1.2.0.3-1 and Table 16.1.2.0.3-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tevaluate,EUTRAN RedCap Relax as specified in Table 16.1.2.0.3-1 and Table 16.1.2.0.3-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 16.1.2.0.3-3 are applicable for eDRX cycle < 10.24 s.

If the UE is configured with eDRX_IDLE cycle ≥ 10.24 s, then the requirements in Table 16.1.2.0.3-4 apply provided that filtering of a measurement is done within a single PTW and provided that the eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

Table 16.1.2.0.3-1: Tdetect,EUTRAN_RedCap_Relax, Tmeasure,EUTRAN_RedCap_Relax, and Tevaluate,EUTRAN_RedCap_Relax for 1 Rx RedCap

| DRX cycle length [s] | Tdetect,EUTRAN_Relax [s] (number of DRX cycles) | Tmeasure,EUTRAN_Relax [s] (number of DRX cycles) | Tevaluate,EUTRAN_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K5 (36 x K5) | 1.28 x K5 (4 x K5) | 5.12 x K5 (16 x K5) |
| 0.64 | 17.92 x K5 (28 x K5) | 1.28 x K5 (2 x K5) | 5.12 x K5 (8 x K5) |
| 1.28 | 32 x K5 (25 x K5) | 1.28 x K5 (1 x K5) | 6.4 x K5 (5 x K5) |
| 2.56 | 58.88 x K5 (23 x K5) | 2.56 x K5 (1 x K5) | 7.68 x K5 (3 x K5) |
| Note 1: K5 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 16.1.2.0.3-2: Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax, and Tevaluate,EUTRAN_Relax for 2 Rx RedCap

| DRX cycle length [s] | Tdetect,EUTRAN_Relax [s] (number of DRX cycles) | Tmeasure,EUTRAN_Relax [s] (number of DRX cycles) | Tevaluate,EUTRAN_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K5 (36 x K5) | 1.28 x K5 (4 x K5) | 5.12 x K5 (16 x K5) |
| 0.64 | 17.92 x K5 (28 x K5) | 1.28 x K5 (2 x K5) | 5.12 x K5 (8 x K5) |
| 1.28 | 32 x K5 (25 x K5) | 1.28 x K5 (1 x K5) | 6.4 x K5 (5 x K5) |
| 2.56 | 58.88 x K5 (23 x K5) | 2.56 x K5 (1 x K5) | 7.68 x K5 (3 x K5) |

Table 16.1.2.0.3-3: Tdetect,E-UTRAN _RedCap_Relax, Tmeasure,NR_,E-UTRAN _RedCap_Relax and Tevaluate,NR_,E-UTRAN _RedCap_Relax for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | Tdetect,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 5.12 | 117.76 x K3 (23 x K3) | 5.12 x K3 (1 x K3) | 10.24 x K3 (2 x K3) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 16.1.2.0.3-4: Tdetect,E-UTRAN _RedCap_Relax, Tmeasure,NR_,E-UTRAN _RedCap_Relax and Tevaluate,NR_,E-UTRAN _RedCap_Relax for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) | Tmeasure,EUTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 10.24 ≤ eDRX_IDLE cycle length ≤ 2621.444 | 0.32 | ≥1.28 (1) | K3 x (23 x K3) | 0.32 x K3 (1 x K3) | 0.64 x K3 (2 x K3) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 x K3 (1 x K3) | 1.28 x K3 (2 x K3) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 x K3 (1 x K3) | 2.56 x K3 (2 x K3) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |

#### 16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

16.1.2.1.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection to higher priority E-UTRA cell, for 1 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.5.

16.1.2.1.2	Test applicability

This test applies to all types of NR RedCap UEwith 1Rx from release 17 and forward.

16.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.1.

16.1.2.1.4	Test description

16.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.2.1.4.1-1.

Table 16.1.2.1.4.1-1: Supported test configurations for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.2.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.1-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.1-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.1-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.2.1.4.1-2.

Table 16.1.2.1.4.1-2: Initial conditions for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.2.1.4.1-3.

2.	Message contents are defined in clause 16.1.2.1.4.3.

3.	There are two cells in the test on two carriers. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1. E-UTRAN cell 2 is of higher priority than cell 1.

Table 16.1.2.1.4.1-3: General test parameters for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE camps on cell 1 in the initial phase and during T2 period the UE reselects to cell 2. |
| T2 end | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | 2 during T2. |
| T3 end | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | 1 during T3 for iteration of the tests. |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 7 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6, 8 | 4 |  |
| E-UTRAN PRACH |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in |
| configuration index |  |  | 4, 5, 6 | 4 | TS 36.211 [23] |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | >7 | During T1, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.2.1.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an E-UTRA neighbour cell (Cell 2). The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively.

Before T1 the UE is camped on to cell 1. During T1, cell 2 shall be powered off. At the start of T2 the UE is expected to detect cell 2, send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2. At the start of T3 cell 2 becomes weaker than cell 1, and the UE reselects to Cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

2.	Set the parameters according to T1 in Table 16.1.2.1.5-1 and 16.1.2.1.5-2. T1 starts.

3.	During T1, Cell 2 shall be powered off and set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008 for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.1.2.1.5-1 and 16.1.2.1.5-2. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a higher priority Cell 2.

6.	If the UE responds on Cell 2 during time duration T2 within 68 seconds from the beginning of time period T2, then count a success for the event “re-select higher priority E-UTRA Cell 2”, continue with step 7. Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, count a fail for the event “re-select higher priority E-UTRA Cell 2”, and the TE shall switch off and, on the UE, and skip to step 12.

7.	The SS shall send an RRCConnectionRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 16.1.2.1.5-1 and 16.1.2.1.5-2. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to a lower priority Cell 1.

10.	If the UE has re-selected Cell 1 within T3, then count a success for the event “re-select lower priority NR Cell 1” and continue with step 11. Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, count a fail for the event “re-select lower priority NR Cell 1” and the TE shall switch off and, on the UE and skip to step 12.

11. The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5.

13.	Repeat step 2-12 until a test verdict has been achieved. Each of the events “re-select higher priority E-UTRA Cell 2” and “re-select lower priority NR Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

16.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with Condition SMTC 1 and higher priority for configuration 16.1.2.1-2, 16.1.2.1-3, 16.1.2.1-5 and 16.1.2.1-6 Table H.2.3-2 with Condition SMTC 2 and higher priority for configuration 16.1.2.1-1, 16.1.2.1-4, 16.1.2.1-7 and 16.1.2.1-8 Table H.2.3-3 with Condition higher priority |
| Default RRC messages and information elements contents exceptions |  |

Table 16.1.2.1.4.3-2: SystemInformationBlockType3

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 22 | Actual value = 44 dB |  |
| cellReselectionPriority | 5 |  | higher priority E-UTRA |
| } |  |  |  |
| } |  |  |  |

Table 16.1.2.1.4.3-3: SystemInformationBlockType24

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority-r15[n] | 4 |  | higher priority E-UTRA |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| } |  |  |  |

16.1.2.1.5	Test requirement

Table 16.1.2.1.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for 1 Rx UE.

Table 16.1.2.1.5-1: NR cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

| Parameter | Unit | Test configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4, 7, 8 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH parameters |  | 1, 4, 7, 8 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 4, 7, 8 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| Dedicated CORESET parameters |  | 1, 4, 7, 8 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| SSB parameters |  | 1, 4, 7, 8 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.1 RedCap FR1 |  |  |
| NR SMTC parameters |  | 1, 4, 7, 8 | SMTC.2 |  |  |
|  |  | 2, 5 | SMTC.1 |  |  |
|  |  | 3, 6 | SMTC.1 |  |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6, 7, 8 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
|  | dBm/SCS | 1, 4, 7, 8 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6, 7, 8 | -98 |  |  |
| SS-RSRP | dBm/SCS | 1, 4, 7, 8 | -84 | -84 | -82.40- |
|  |  | 2, 5 | -84 | -84 | -82.40- |
|  |  | 3, 6 | -81 | -81 | -79.40- |
|  | dB | 1, 4, 7, 8 | 14 | 14 | 15.60 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4, 7, 8 | 14 | 14 | 15.60 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Io | dBm/9.36 MHz | 1, 4, 7, 8 | -55.88 | -55.88 | -54.33 |
|  | dBm/9.36 MHz | 2, 5 | -55.88 | -55.88 | -54.33 |
|  | dBm/18.36 MHz | 3, 6 | -52.95 | -52.95 | -51.40 |
| Treselection | S | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |

Table 16.1.2.1.5-2: E-UTRA Cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

| Parameter | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| QrxlevminNote 3 | dBm | -139 |  |  |
| Noc |  | -98 | -98 | -100 |
| RSRP | dBm/15 KHz | -infinity | -84.4 | -103.6 |
|  | dB | -infinity | 13.6 | -3.6 |
|  | dB | -infinity | 13.6 | -3.6 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | Not sent |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP (Note 2) | dB | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, Low which is included in E-UTRA system information, and is a threshold for the NR target cell Note 3: Value modified in Test Tolerance analysis in TS 38.903. |  |  |  |  |

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX

16.1.2.2.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection to higher priority E-UTRA cell, for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.5.

16.1.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward.

16.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.2.

16.1.2.2.4	Test description

16.1.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.2.2.4.1-1.

Table 16.1.2.2.4.1-1: Supported test configurations for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 RX

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.2.1.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.2-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.2-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.2-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.2-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.2.2.4.1-2.

Table 16.1.2.2.4.1-2: Initial conditions for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.2.2.4.1-3.

2.	Message contents are defined in clause 16.1.2.2.4.3.

3.	There are two cells in the test on two carriers. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 16.1.2.2.4.1-3: General test parameters for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 RX

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE camps on cell 1 in the initial phase and during T2 period the UE reselects to cell 2. |
| T2 end | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | 2 during T2. |
| T3 end | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | 1 during T3 for iteration of the tests. |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 7 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6, 8 | 4 |  |
| E-UTRAN PRACH |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in |
| configuration index |  |  | 4, 5, 6 | 4 | TS 36.211 [23] |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | >7 | During T1, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.2.2.4.2	Test procedure

Same test procedure as in 16.1.2.1.4.2.

16.1.2.2.4.3	Message contents

Same message contents  as in 16.1.2.1.4.3.

16.1.2.2.5	Test requirement

Table 16.1.2.2.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for 2 Rx UE.

Table 16.1.2.2.5-1: NR cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 RX

| Parameter | Unit | Test configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4, 7, 8 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH parameters |  | 1, 4, 7, 8 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 4, 7, 8 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| Dedicated CORESET parameters |  | 1, 4, 7, 8 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| SSB parameters |  | 1, 4, 7, 8 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.1 RedCap FR1 |  |  |
| NR SMTC parameters |  | 1, 4, 7, 8 | SMTC.2 |  |  |
|  |  | 2, 5 | SMTC.1 |  |  |
|  |  | 3, 6 | SMTC.1 |  |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6, 7, 8 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
|  | dBm/SCS | 1, 4, 7, 8 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6, 7, 8 | -98 |  |  |
| SS-RSRP | dBm/SCS | 1, 4, 7, 8 | -84 | -82.40 | -82.40 |
|  |  | 2, 5 | -84 | -82.40 | -82.40 |
|  |  | 3, 6 | -81 | -79.40 | -79.40 |
|  | dB | 1, 4, 7, 8 | 14 | 15.60 | 15.60 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4, 7, 8 | 14 | 15.60 | 15.60 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Io | dBm/9.36 MHz | 1, 4, 7, 8 | -55.88 | -54.33 | -54.33 |
|  | dBm/9.36 MHz | 2, 5 | -55.88 | -54.33 | -54.33 |
|  | dBm/18.36 MHz | 3, 6 | -52.95 | -51.40 | -51.40 |
| Treselection | S | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |

Table 16.1.2.2.5-2: E-UTRA Cell specific test parameters for for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 RX

| Parameter | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -98 | -98 | -100 |
| RSRP | dBm/15 KHz | -infinity | -84.4 | -103.6 |
|  | dB | -infinity | 13.6 | -3.6 |
|  | dB | -infinity | 13.6 | -3.6 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | Not sent |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP (Note 2) | dB | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, Low which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |  |

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX

16.1.2.3.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection to lower priority E-UTRA cell, for 1 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.5.

16.1.2.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 and forward.

16.1.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.3.

16.1.2.3.4	Test description

16.1.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.2.3.4.1-1.

Table 16.1.2.3.4.1-1: Supported test configurations for NR SA FR1 – E-UTRA Cell reselection to lower priority E-UTRA for 1RX

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.2.1.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.3-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.3-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.3-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.3-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.2.3.4.1-2.

Table 16.1.2.3.4.1-2: Initial conditions for NR SA FR1 – E-UTRA Cell reselection to lower priority E-UTRA for 1RX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.2.3.4.1-3.

2.	Message contents are defined in clause 16.1.2.3.4.3.

3.	There are two cells in the test on two carriers. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 16.1.2.3.4.1-3: General test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE camps on cell 1 in the initial phase. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 7 | 534 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6, 8 |  |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.2.3.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an E-UTRA neighbour cell (Cell 2). The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 on Cell 1.

2.	Set the parameters according to T2 in Table 16.1.2.3.5-1 and 16.1.2.3.5-2. SS waits at least for 32sec(Tdetect,EUTRAN)  to ensure the UE can detect E-UTRA neighbour cell (Cell2).

3.	Set the parameters according to T1 in Table 16.1.2.3.5-1 and 16.1.2.3.5-2. T1 starts.

4.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

5.	If the UE responds on Cell 2 during time duration T1 within 8 seconds from the beginning of time period T1 then count a success for the event “re-select lower priority E-UTRA Cell 2” and continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, count a failure for the event “re-select lower priority E-UTRA Cell 2”, the TE shall switch off and on the UE and skip to step 11.

6	The SS shall send an RRCConnectionRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 16.1.2.3.5-1 and 16.1.2.3.5-2. T2 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

9.	If the UE has re-selected Cell 1 within T2, then count a success for the event “re-select higher priority NR Cell 1” and continue with step 10. Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, count a fail for the event “re-select higher priority NR Cell 1” and the TE shall switch off and, on the UE and skip to step 11.

10.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

11.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5.

12.	Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.1.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.2.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-3 with Condition lower priority |
| Default RRC messages and information elements contents exceptions |  |

Table 16.1.2.3.4.3-2: SIB2

| Derivation Path: Table H.2.3-2 with condition lower priority |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present | UE applies the default value of infinity |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.2.2-2, 16.1.2.2-3, 16.1.2.2-5, 16.1.2.2-6 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 |  | 16.1.2.2-1, 16.1.2.2-4, 16.1.2.2-7, 16.1.2.2-8, |
| } |  |  |  |
| } |  |  |  |

16.1.2.3.4.3-3: SystemInformationBlockType3

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 22 | Actual value = 44 dB |  |
| cellReselectionPriority | 4 |  | lower priority E-UTRA |
| } |  |  |  |
| } |  |  |  |

16.1.2.3.4.3-4: SystemInformationBlockType24

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority-r15[n] | 5 |  | lower priority E-UTRA |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| } |  |  |  |

16.1.2.3.5	Test requirement

Table 16.1.2.3.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for 1 Rx UE.

Table 16.1.2.3.5-1: NR cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4, 7, 8 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4, 7, 8 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4, 7, 8 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4, 7, 8 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4, 7, 8 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1, 4, 7, 8 | SMTC.2 |  |
|  |  | 2, 5 | SMTC.1 |  |
|  |  | 3, 6 | SMTC.1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6, 7, 8 | SSB |  |
| QrxlevminNote3 | dBm/SCS | 1, 2, 4, 5 | -139 |  |
|  |  | 3, 6 | -136 |  |
|  | dBm/SCS | 1, 4, 7, 8 | -100 |  |
|  |  | 2, 5 | -100 |  |
|  |  | 3, 6 | -97 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6, 7, 8 | -100 |  |
| SS-RSRP | dBm/SCS | 1, 4, 7, 8 | -103.6 | -84.4 |
|  |  | 2, 5 | -103.6 | -84.4 |
|  |  | 3, 6 | -100.6 | -81.4 |
|  | dB | 1, 4, 7, 8 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4, 7, 8 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4, 7, 8 | -70.48 | -56.26 |
|  | dBm/9.36 MHz | 2, 5 | -70.48 | -56.26 |
|  | dBm/18.36 MHz | 3, 6 | -65.12 | -53.15 |
| Treselection | S | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | Not sent |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell Note 3: Value modified in Test Tolerance analysis in TS 38.903. |  |  |  |  |

Table 16.1.2.3.5-2: E-UTRA Cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -82.4 | -84 |
|  | dB | 15.6 | 14 |
|  | dB | 15.6 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP (Note 2) | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX

16.1.2.4.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection to lower priority E-UTRA cell, for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.5.

16.1.2.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward.

16.1.2.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.4.

16.1.2.4.4	Test description

16.1.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.2.4.4.1-1.

Table 16.1.2.4.4.1-1: Supported test configurations for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.2.1.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.4-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.4-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.4-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.4-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.4-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.4-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.2.4.4.1-2.

Table 16.1.2.4.4.1-2: Initial conditions for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.2.4.4.1-3.

2.	Message contents are defined in clause 16.1.2.4.4.3.

3.	There are two cells in the test on two carriers. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 16.1.2.4.4.1-3: General test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE camps on cell 1 in the initial phase. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 7 | 534 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6, 8 |  |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.2.4.4.2	Test procedure

Same test procedure as in 16.1.2.3.4.2.

16.1.2.4.4.3	Message contents

Same message contents  as in 16.1.2.3.4.3.

16.1.2.4.5	Test requirement

Table 16.1.2.4.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for 2 Rx UE.

Table 16.1.2.4.5-1: NR cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4, 7, 8 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4, 7, 8 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4, 7, 8 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4, 7, 8 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4, 7, 8 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1, 4, 7, 8 | SMTC.2 |  |
|  |  | 2, 5 | SMTC.1 |  |
|  |  | 3, 6 | SMTC.1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6, 7, 8 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -100 |  |
|  |  | 2, 5 | -100 |  |
|  |  | 3, 6 | -97 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6, 7, 8 | -100 |  |
| SS-RSRP | dBm/SCS | 1, 4, 7, 8 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4, 7, 8 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4, 7, 8 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4, 7, 8 | -70.84 | -56.26 |
|  | dBm/9.36 MHz | 2, 5 | -70.84 | -56.26 |
|  | dBm/18.36 MHz | 3, 6 | -65.12 | -55.15 |
| Treselection | S | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | Not sent |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table 16.1.2.4.5-2: E-UTRA cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -82.4 | -84 |
|  | dB | 15.6 | 14 |
|  | dB | 15.6 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP (Note 2) | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 16.1.2.5	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

16.1.2.5.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.11.2.

16.1.2.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 and forward supporting Rel.17 stationary idle mode relaxed measurement criterion.

16.1.2.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.5.

16.1.2.5.4	Test description

16.1.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.2.5.4.1-1.

Table 16.1.2.5.4.1-1: Supported test configurations for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.2.1.5-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.5-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.5-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.5-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.5-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.5-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.5-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.5-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.2.5.4.1-2.

Table 16.1.2.5.4.1-2: Initial conditions for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.2.5.4.1-3.

2.	Message contents are defined in clause 16.1.2.5.4.3.

3.	There are two cells in the test on two carriers. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 16.1.2.5.4.1-3: General test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE camps on cell 1 in the initial phase, it fulfils stationary criterion, and during T1 period the UE reselects to cell 2 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE shall perform reselection to cell 1 with higher priority during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 0.64 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 7, 8 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 24 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 24 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.2.5.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an E-UTRA neighbour cell (Cell 2). The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

The UE is configured with the relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in TS 38.304 [30]. So, Cell 1 configures the UE as follows:

-	stationaryMobilityEvaluation [13] criterion is configured according to the parameters listed TS 38.133 [6] in Table A.16.1.1.8.2-3;

-	cellEdgeEvaluationWhileStationary [13] criterion is not configured;

-	combineRelaxedMeasCondition2 [13] is not configured;

The cell reselection delay to a lower priority E-UTRAN cell with UE fulfilling stationary relaxed measurement criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCConnectionRequest message to perform a Tracking Area Update procedure on cell 2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 16.1.2.5.5-1 and 16.1.2.5.5-2. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

4.	If the UE responds on Cell 2 during time duration T1 within 32 seconds from the beginning of time period T1 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

5.	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, continue with step 6.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 11.

6	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 16.1.2.5.5-1 and 16.1.2.5.5-2. T2 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

9.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, continue with step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 11.

10	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE, and then skip to step 12.

11.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5.

12.	Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.1.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.2.5.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with Condition SMTC 1 and lower priority for configuration 16.1.2.5-2, 16.1.2.5-3, 16.1.2.5-5 and 16.1.2.5-6 Table H.2.3-2 with Condition SMTC 2 and lower priority for configuration 16.1.2.5-1, 16.1.2.5-4, 16.1.2.5-7 and 16.1.2.5-8 Table H.2.3-3 with Condition lower priority |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

16.1.2.5.4.3-2: SIB2

| Derivation Path: TS 38.508-1 [14] Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB3 |  |  |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

16.1.2.5.4.3-3: SystemInformationBlockType3

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| relaxedMeasurement-r16 SEQUENCE { |  |  |  |
| threshServingLow | 22 | Actual value = 44 dB |  |
| cellReselectionPriority | 4 |  | lower priority E-UTRA |
| } |  |  |  |
| } |  |  |  |

16.1.2.5.4.3-4: SystemInformationBlockType24

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority-r15[n] | 5 |  | lower priority E-UTRA |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| } |  |  |  |

16.1.2.5.5	Test requirement

Table 16.1.2.5.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE.

Table 16.1.2.5.5-1: NR cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4,7, 8 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4, 8 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4, 8 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4, 8 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4, 8 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1, 4, 8 | SMTC.2 |  |
|  |  | 2, 5 | SMTC.1 |  |
|  |  | 3, 6 | SMTC.1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6, 7, 8 | SSB |  |
| QrxlevminNote3 | dBm/SCS | 1, 2, 4, 5 | -139 |  |
|  |  | 3, 6 | -136 |  |
|  | dBm/SCS | 1, 4, 8 | -100 |  |
|  |  | 2, 5 | -100 |  |
|  |  | 3, 6 | -97 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6, 7, 8 | -100 |  |
| SS-RSRP | dBm/SCS | 1, 4, 8 | -103.6 | -84.4 |
|  |  | 2, 5 | -103.6 | -84.4 |
|  |  | 3, 6 | -100.6 | -81.4 |
|  | dB | 1, 4, 8 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4, 8 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4, 8 | -70.84 | -56.26 |
|  | dBm/9.36 MHz | 2, 5 | -70.84 | -56.26 |
|  | dBm/18.36 MHz | 3, 6 | -65.12 | -55.15 |
| TreselectionP | S | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |
| Snonintrasearch | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |
| SSearchDeltaP-Stationary | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 3 |  |
| TSearchDeltaP-Stationary | s | 1, 2, 3, 4, 5, 6, 7, 8 | 5 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, low which is included in NR system information, and is a threshold for the E-UTRA target cell Note 3: Value modified in Test Tolerance analysis in TS 38.903. |  |  |  |  |

Table 16.1.2.5.5-2: E-UTRA cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -82.4 | -84 |
|  | dB | 15.6 | 14 |
|  | dB | 15.6 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | Not sent |  |
| Threshx, high (Note 2) | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

The cell reselection delay to a lower priority E-UTRAN cell with UE fulfilling stationary relaxed measurement criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCConnectionRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 32 s.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 16.1.2.6	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

16.1.2.6.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.11.2.

16.1.2.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward supporting Rel.17 stationary idle mode relaxed measurement criterion.

16.1.2.6.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.6.

16.1.2.6.4	Test description

16.1.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.2.6.4.1-1.

Table 16.1.2.6.4.1-1: Supported test configurations for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.2.1.6-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.6-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.6-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.6-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.6-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.6-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.6-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.6-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.2.6.4.1-2.

Table 16.1.2.6.4.1-2: Initial conditions for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.2.6.4.1-3.

2.	Message contents are defined in clause 16.1.2.6.4.3.

3.	There are two cells in the test on two carriers. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 16.1.2.6.4.1-3: General test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE camps on cell 1 in the initial phase, it fulfils stationary criterion, and during T1 period the UE reselects to cell 2 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE shall perform reselection to cell 1 with higher priority during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 0.64 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 7 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6, 8 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 24 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 24 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.2.6.4.2	Test procedure

Same test procedure as in 16.1.2.5.4.2.

16.1.2.6.4.3	Message contents

Same message contents as in 16.1.2.5.4.3.

16.1.2.6.5	Test requirement

Table 16.1.2.6.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE.

Table 16.1.2.6.5-1: NR cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4, 7, 8 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4, 7, 8 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4, 7, 8 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4, 7, 8 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4, 7, 8 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1, 4, 7, 8 | SMTC.2 |  |
|  |  | 2, 5 | SMTC.1 |  |
|  |  | 3, 6 | SMTC.1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6, 7, 8 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4, 7, 8 | -100 |  |
|  |  | 2, 5 | -100 |  |
|  |  | 3, 6 | -97 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6, 7, 8 | -100 |  |
| SS-RSRP | dBm/SCS | 1, 4, 7, 8 | -103.6 | -84.4 |
|  |  | 2, 5 | -103.6 | -84.4 |
|  |  | 3, 6 | -100.6 | -81-4 |
|  | dB | 1, 4, 7, 8 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4, 7, 8 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4, 7, 8 | -70.84 | -56.26 |
|  | dBm/9.36 MHz | 2, 5 | -70.84 | -56.26 |
|  | dBm/18.36 MHz | 3, 6 | -65.12 | -53.15 |
| TreselectionP | S | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |
| Snonintrasearch | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |
| SSearchDeltaP-Stationary | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 3 |  |
| TSearchDeltaP-Stationary | s | 1, 2, 3, 4, 5, 6, 7, 8 | 5 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, low which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table 16.1.2.6.5-2: E-UTRA cell specific test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -82.4 | -84 |
|  | dB | 15.6 | 14 |
|  | dB | 15.6 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | Not sent |  |
| Threshx, high (Note 2) | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

The cell reselection delay to a lower priority E-UTRAN cell with UE fulfilling stationary relaxed measurement criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCConnectionRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 32 s.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

## 16.2	RRC_INACTIVE state mobility for RedCap

### 16.2.1	Configured Grant based Small Data Transmissions for RedCap

#### 16.2.1.0	Minimum conformance requirements

##### 16.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation using cg-SDT-RSRP-ChangeThreshold for transmitting in uplink using CG-SDT as specified in TS 38.331 [13].

The requirements in this clause are applicable for the UE performing small data tranmissions using configured resources as TS 38.331 [13].

The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards (i.e. using the timing derived using the latest available NTA value as specified in subclause 7.1.2) the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the small data transmission. The UE determines the small data transmission occasion according to the received CG-SDT configuration as specified in TS 38.331 [13].

When cg-SDT-RSRP-ChangeThreshold [13] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2 in [12], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in TS 38.133 [6] subclause 7.1 provided that

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in [12].

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 16.2.1.0.1-1 are met for FR1.

Table 16.2.1.0.1-1 Valid measurement for FR1

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, for the initial CG-SDT transmission, the UE shall not transmit in an CG-SDT occasion that occurs more than 640 ms after T2.

Where:

-	T1 is the time when

-	RRCRelease with CG-SDT configuration (TS 38.331 [2]) is received, or

-	the latest TA is received if TA is received while in RRC_INACITVE state.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in TS 38.321 [12] clause 5.27.2 for transmission using CG-SDT.

-	T2’ is the time when the UE has completed RSRP2.

-	TDRX is the DRX cycle length in ms.

-	M1 the scaling factor as defined in TS 38.133 [6] clause 4.2.2.2.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

For UE which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to SS-RSRP/RSRQ measurement

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH on all symbols within SMTC window duration.

For UEs which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to measurement of serving cell.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/ on SSB symbols of the serving cell.

The UE is not required to meet the following measurement requirements during subsequent SDT transmissions:

-	Measurements of inter-frequency NR cells in TS 38.133 [6] clause 5.1.2.4

-	Measurements of inter-RAT E-UTRAN cells in TS 38.133 [6] clause 5.1.2.5

-	Idle Mode CA/DC Measurements in TS 38.133 [6] clause 5.4

The UE is allowed to delay the reception of PRS resources on the positioning frequency layer until the SDT session is completed if the measurement using PRS resource overlaps with the SDT resources.

The normative reference for this requirement is TS 38.133 [6] clauses 5.2B.

#### 16.2.1.1	NR SA FR1 CG-SDT for 1Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	Test parameters are still in brackets;

-	Test procedure needs further updates;

-	TT analysis is still missing.

16.2.1.1.1	Test purpose

To verify that the UE properly perform TA validation for CG-SDT transmission as specified in TS 38.133 [6] clause 5.2B.2.1.

16.2.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards and supporting CG-SDT.

16.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.2.1.1.

16.2.1.1.4	Test description

Same as the test description given in clause 6.2.1.4.

16.2.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.2.1.4.1 with following exceptions:

-	Table 6.2.1.4.1-1 is replaced by Table 16.2.1.1.4.1-1;

-	Table 6.2.1.4.1-2 is replaced by Table 16.2.1.1.4.1-2;

-	Table 6.2.1.4.1-3 is replaced by Table 16.2.1.1.4.1-3;

Table 16.2.1.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.2.1.1-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR FDD |
| 16.2.1.1-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR TDD |
| 16.2.1.1-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, NR TDD |
| 16.2.1.1-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR HD-FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.2.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.2.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2,3 |  | TDD |  |
|  | Config 4 |  | HD-FDD |  |
| TDD Configuration | Config 1,4 |  | N/A |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.3 FDD |  |
|  | Config 2 |  | CCR.1.3 TDD |  |
|  | Config 3 |  | CCR.2.2 TDD |  |
| SSB configuration | Config 1,2,4 |  | SSB.3 FR1 |  |
|  | Config 3 |  | SSB. 1 RedCap FR1 |  |
| OCNG Patterns |  |  | OP.1 |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  |  | SMTC.1 |  |
| DRX configuration |  |  | DRX.7 |  |
| T_delay_modeB |  | s | [4] |  |
| T1 |  | s | 0.4 |  |
| T2 |  | s | 1.28 | 2 x W1 |
| T3 |  | s | 2.72 | T_timer_modeB - W1 -W2 |
| T4 |  | s | 2.24 | w1+w2+w3+100 |
| T5 |  | s | 4.22 | T3+W2+W3 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | 8 |  |
| cg-SDT-RSRP-ThresholdSSB |  | dBm | -110 |  |
| cg-SDT-TimeAlignmentTime |  |  | Infinity |  |
| CG-SDT resource period |  | ms | 320 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |
| Propagation condition |  |  | AWGN |  |

16.2.1.1.4.2	Test procedure

Same as the test procedure given in clause 6.2.1.4.2.

16.2.1.1.4.3	Message contents

Same as the message contents given in 6.2.1.4.3 with following exceptions:

-	Table 6.2.1.4.3-2 is replaced by Table 16.2.1.1.4.3-1.

Table 16.2.1.1.4.3-1: SDT-MAC-PHY-CG-Config-r17

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDT-MAC-PHY-CG-Config-r17::= SEQUENCE { |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { | 1 entry |  |  |
| logicalChannelIdentity-r17 | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 | true |  |  |
| allowedCG-List-r17 | Not present |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup | BWP-UplinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  |  |  |
| setup | BWP-DownlinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 46 | -110dBm |  |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.2.1.1.5	Test requirements

Same as the test requirements given in clause 6.2.1.5 with following exceptions:

-	Table 6.2.1.5-1 is replaced by Table 16.2.1.1.5-1.

Table 16.2.1.1.5-1: Cell-specific test parameters (NR Cell 1)

| Parameter | Config | Unit | SSB#0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Note1 |  | dBm/15kHz | -98 |  |  |  |  |
| Note1 | 1,2,4 | dBm/SSB SCS | -98 |  |  |  |  |
|  | 3 |  | -95 |  |  |  |  |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
| SS RSRP Note2 | 1,2,4 | dBm/SSB SCS | -98 | -98 | -98 | -98 | -98 |
|  | 3 |  | -95 | -95 | -95 | -95 | -95 |
| Io Note2 | 1,2,4 | dBm/ 9.36 MHz | -67.04 | -67.04 | -67.04 | -67.04 | -67.04 |
|  | 3 | dBm/ 18.36 MHz | -67.12 | -67.12 | -67.12 | -67.12 | -67.12 |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### 16.2.1.2	NR SA FR1 CG-SDT for 2Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	Test parameters are still in brackets;

-	Test procedure needs further updates;

-	TT analysis is still missing.

16.2.1.2.1	Test purpose

To verify that the UE properly perform TA validation for CG-SDT transmission as specified in TS 38.133 [6] clause 5.2B.2.1.

16.2.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting CG-SDT.

16.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.2.1.2.

16.2.1.2.4	Test description

Same as the test description given in clause 6.2.1.4.

16.2.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.2.1.4.1 with following exceptions:

-	Table 6.2.1.4.1-1 is replaced by Table 16.2.1.2.4.1-1;

-	Table 6.2.1.4.1-2 is replaced by Table 16.2.1.2.4.1-2;

-	Table 6.2.1.4.1-3 is replaced by Table 16.2.1.2.4.1-3;

Table 16.2.1.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.2.1.2-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR FDD |
| 16.2.1.2-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR TDD |
| 16.2.1.2-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, NR TDD |
| 16.2.1.2-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR HD-FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.2.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.2.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.2.1.2.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2,3 |  | TDD |  |
|  | Config 4 |  | HD-FDD |  |
| TDD Configuration | Config 1,4 |  | N/A |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.3 FDD |  |
|  | Config 2 |  | CCR.1.3 TDD |  |
|  | Config 3 |  | CCR.2.2 TDD |  |
| SSB configuration | Config 1,2,4 |  | SSB.3 FR1 |  |
|  | Config 3 |  | SSB. 1 RedCap FR1 |  |
| OCNG Patterns |  |  | OP.1 |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  |  | SMTC.1 |  |
| DRX configuration |  |  | DRX.7 |  |
| T_delay_modeB |  | s | [4] |  |
| T1 |  | s | 0.4 |  |
| T2 |  | s | 1.28 | 2 x W1 |
| T3 |  | s | 2.72 | T_timer_modeB - W1 -W2 |
| T4 |  | s | 2.24 | w1+w2+w3+100 |
| T5 |  | s | 4.22 | T3+W2+W3 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | 8 |  |
| cg-SDT-RSRP-ThresholdSSB |  | dBm | -110 |  |
| cg-SDT-TimeAlignmentTime |  |  | Infinity |  |
| CG-SDT resource period |  | ms | 320 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |
| Propagation condition |  |  | AWGN |  |

16.2.1.2.4.2	Test procedure

Same as the test procedure given in clause 6.2.1.4.2.

16.2.1.2.4.3	Message contents

Same as the message contents given in 6.2.1.4.3 with following exceptions:

-	Table 6.2.1.4.3-2 is replaced by Table 16.2.1.2.4.3-1.

Table 16.2.1.2.4.3-1: SDT-MAC-PHY-CG-Config-r17

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDT-MAC-PHY-CG-Config-r17::= SEQUENCE { |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { | 1 entry |  |  |
| logicalChannelIdentity-r17 | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 | true |  |  |
| allowedCG-List-r17 | Not present |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup | BWP-UplinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  |  |  |
| setup | BWP-DownlinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 46 | -110dBm |  |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.2.1.2.5	Test requirements

Same as the test requirements given in clause 6.2.1.5 with following exceptions:

-	Table 6.2.1.5-1 is replaced by Table 16.2.1.2.5-1.

Table 16.2.1.2.5-1: Cell-specific test parameters (NR Cell 1)

| Parameter | Config | Unit | SSB#0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Note1 |  | dBm/15kHz | -98 |  |  |  |  |
| Note1 | 1,2,4 | dBm/SSB SCS | -98 |  |  |  |  |
|  | 3 |  | -95 |  |  |  |  |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
| SS RSRP Note2 | 1,2,4 | dBm/SSB SCS | -98 | -98 | -98 | -98 | -98 |
|  | 3 |  | -95 | -95 | -95 | -95 | -95 |
| Io Note2 | 1,2,4 | dBm/ 9.36 MHz | -67.04 | -67.04 | -67.04 | -67.04 | -67.04 |
|  | 3 | dBm/ 18.36 MHz | -67.12 | -67.12 | -67.12 | -67.12 | -67.12 |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

## 16.3	RRC_CONNECTED state mobility for RedCap

### 16.3.1	Handover

#### 16.3.1.0	Minimum conformance requirements

##### 16.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover

The requirements in clause 6.3.1.0.1 shall apply when RedCap UE is capable of 2 Rx.

When UE is only required to support 1 Rx antenna, the requirements for category 1bis UE defined in 36.133 [23] clause 5.1.2 shall apply.

The normative reference for this requirement is TS 38.133 [6] clause 6.1D.2.1.

##### 16.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover

The purpose of NR handover is to change the NR PCell to another NR cell for RedCap UE. The requirements in this clause are applicable to SA NR.

The requirements in this clause apply for the following handover scenarios:

-	Handover to a target cell’s initial BWP associated with CD-SSB;

-	Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP).

The requirements in clause 6.3.1.0.2 shall apply when RedCap UE is capable of 2 Rx.

When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.1.1.2 shall apply except that:

-	Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 2*Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 5* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise, Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing as NCD-SSB indicated by nonCellDefiningSSB-r17 if the first active DL BWP included in handover command is configured with nonCellDefiningSSB-r17, otherwise, as CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL in handover command.

-	If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

For RedCap UE with HD-FDD, the handover requirements are met provided that

-	SSB is available at the UE once every SMTC period during Tsearch

-	One SSB is available during T∆

-	One SSB is available during TIU.

The normative reference for this requirement is TS 38.133 [6] clause 6.1D.1.2.

#### 16.3.1.1	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE

16.3.1.1.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements for 1 Rx RedCap UE specified in 38.133 [6] clause 6.1D.1.2.

16.3.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.1.

16.3.1.1.4	Test description

16.3.1.1.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.1.1.4.1 with the following exceptions:

-	Table 6.3.1.1.4.1-1 is replaced by Table 16.3.1.1.4.1-1.

-	Table 6.3.1.1.4.1-2 is replaced by Table 16.3.1.1.4.1-2.

-	Table 6.3.1.1.4.1-3 is replaced by Table 16.3.1.1.4.1-3.

Table 16.3.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.1-1 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.1.1-2 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.1.1-3 | Source cell:NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.1.1-4 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.3.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | -1.5 | A3 offset is updated via TT analysis |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

16.3.1.1.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.1.1.4.2.

16.3.1.1.4.3	Message contents

Same as the message contents specified in sub-clause 6.3.1.1.4.3 with following exceptions:

-	Table H.3.1-4 is used with A3-offset = -1.5 dB.

16.3.1.1.5	Test requirement

Same as the test requirements given in sub-clause 6.3.1.1.5 with following exceptions:

-	Table 6.3.1.1.5-1 is replaced by Table 16.3.1.1.5-1.

-	Tinterrupt in test requirements is specified in 38.133 [6] clause 6.1D.2.2.

Table 16.3.1.1.5-1: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  | 1 |  |  | 1 |  |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| BWP BW | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  | SSB.1 RedCap FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR1 |  |  |  |  |  |
| Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |  |  |
| Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |  |  |
| Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |  |  |
| Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2,4 | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |  |
|  |  | dB | 8 | -2.55 | -2.55 | -∞ | 1.51 | 1.51 |
|  |  | dB | 8 | 8 | 8 | -∞ | 10.15 | 10.15 |
| SSB_RP | Config 1,2,4 | dBm/SCS | -90 | -90 | -90 | -∞ | -87.85 | -87.85 |
|  | Config 3 | dBm/SCS | -87 | -87 | -87 | -∞ | -84.85 | -84.85 |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -61.41 | -57.58 | -57.58 | -61.41 | -57.58 | -57.58 |
|  | Config 3 | dBm/ 18.36MHz | -58.49 | -54.65 | -54.65 | -58.49 | -54.65 | -54.65 |
| Propagation condition |  | - | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  |  | 1x1 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### 16.3.1.2	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE

16.3.1.2.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements for 2 Rx RedCap UE as specified in 38.133 [6] clause 6.1D.1.2

16.3.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.2.

16.3.1.2.4	Test description

16.3.1.2.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.1.1.4.1 with the following exceptions:

-	Table 6.3.1.1.4.1-1 is replaced by Table 16.3.1.2.4.1-1.

-	Table 6.3.1.1.4.1-2 is replaced by Table 16.3.1.2.4.1-2.

-	Table 6.3.1.1.4.1-3 is replaced by Table 16.3.1.2.4.1-3

Table 16.3.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.2-1 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.1.2-2 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.1.2-3 | Source cell:NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.1.2-4 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.3.1.2.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | -1 |  |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

16.3.1.2.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.1.1.4.2.

16.3.1.2.4.3	Message contents

Same as the message contents specified in 6.3.1.1.4.3 with following exceptions:

-	Table 7.3.1-3 in TS 38.508-1 [14] is used with condition SMTC.1.

16.3.1.2.5	Test requirement

Table 16.3.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 16.3.1.2.5-1: Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| BWP BW |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference |  | Config 1,4 |  | SR.1.1 FDD |  |  |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |
| CORESET Reference |  | Config 1,4 |  | CR.1.1 FDD |  |  |  |  |  |
| Channel |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |  | SSB.1 RedCap FR1 |  |  |
| PDSCH/PDCCH subcarrier |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
| spacing |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
| spacing |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | PRACH.1 FR1 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/ 15kHz | -98 |  |  |  |  |  |
| Note2 |  | Config 1,2,4 | dBm/ SCS | -98 |  |  |  |  |  |
|  |  | Config 3 |  | -95 |  |  |  |  |  |
|  |  |  | dB | 8 | -2.41 | -2.41 | -∞ | 1.36 | 1.36 |
|  |  |  | dB | 8 | 8 | 8 | -∞ | 10 | 10 |
| SSB_RP |  | Config 1,2,4 | dBm/ SCS | -90 | -90 | -90 | -∞ | -88 | -88 |
|  |  | Config 3 | dBm/ SCS | -87 | -87 | -87 | -∞ | -85 | -85 |
| IoNote3 |  | Config 1,2,4 | dBm/ 9.36MHz | -61.41 | -57.67 | -57.67 | -61.41 | -57.67 | -57.67 |
|  |  | Config 3 | dBm/ 18.36MHz | -58.49 | -54.74 | -54.74 | -58.49 | -54.74 | -54.74 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

-	RRC procedure delay = 10 ms, is the RRC processing delay specified in TS 38.331 [2] clause 12.

-	Tinterrupt = 62 ms, is the interruption time specified in TS 38.133 [6] clause 6.1D.1.2.

This gives a total of 72 ms.

#### 16.3.1.3	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE

16.3.1.3.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements for 1Rx RedCap as specified in TS 38.133 [6] clause 6.1D.1.2.

16.3.1.3.2	Test applicability

This test applies to all types of NR RedCap UEs with 1 Rx from Release 17 onwards.

16.3.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.3.

16.3.1.3.4	Test description

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively.

Before the test starts,

-	UE is connected to Cell 1 with active DL BWP and active UL BWP

-	UE is configured with nonCellDefiningSSB-r17 under BWP-DownlinkDedicated which serves as the reference SSB for the serving cell. The nonCellDefiningSSB-r17 corresponds to the NCD-SSB within the active DL BWP.

At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives an RRC handover command from the network indicating the UE to handover to cell 2 with firstActiveDownlinkBWP-Id configured to DL BWP and firstActiveUplinkBWP-Id configured to UL BWP. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

16.3.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.3.1.3.4.1-1.

Table 16.3.1.3.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.1.3-1 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.1.3-2 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.1.3-3 | Source cell:NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.1.3-4 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.3.1.3.4.1-2

Table 16.3.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.3.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 16.3.1.3.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 16.3.1.3.4.1-3 below.

Table 16.3.1.3.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

16.3.1.3.4.2	Test procedure

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell and RedCap-specific dedicated BWP of Cell 1 is the activated BWP. The SS transmits NCD-SSB on Cell 1 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

2.	Set the parameters according to T1 in Table 16.3.1.3.5-1.

3. 	Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4.	SS shall transmit an RRCReconfiguration message implying handover to the RedCap-specific dedicated BWP of Cell 2.

5.	At the instant when the last TTI containing the RRCReconfiguration message in step 4) is sent to the UE, the SS shall switch the power settings from T1 to T2 as specified in Table 16.3.1.3.5-1. T2 starts. The SS also transmits NCD-SSB on Cell 2 with an absolute frequency of fNCDSSB calculated above.

6.	If the UE transmits a PRACH preamble to Cell 2 satisfying the delay requirement specified in clause 16.3.1.3.5 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, the SS shall

-	transmit an RRCReconfiguration message to cause the UE handover back to Cell 1. if the handover fails, switch off and on the UE

or

-	switch off and on the UE.

8.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell and RedCap-specific dedicated BWP of Cell 1 is the activated BWP.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.3.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.3.1.3.4.3-1: RRCReconfiguration (Step 1, 4 and 8, test procedure)

| Derivation Path: Table H.3.2-2 with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 16.3.1.3.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.3.4.3-2: CellGroupConfig (Table 16.3.1.3.4.3-1)

| Derivation Path: Table H.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| smtc | SMTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap |  |  |
| } |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 16.3.1.3.4.3-3 |  |
| } |  |  |  |

Table 16.3.1.3.4.3-3: ServingCellConfig (Table 16.3.1.3.4.3-2)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.3.1.3.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.3.1.3.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.3.4.3-4: BWP-Downlink (Table 16.3.1.3.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.1 RedCap corresponding to NCD-SSB PRB index |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.3.4.3-5: BWP-Uplink (Table 16.3.1.3.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.1 RedCap. The starting PRB index for dedicated UL BWP is the same as the corresponding RedCap-specific dedicated DL BWP |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.3.1.3.5	Test requirements

Table 16.3.1.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 16.3.1.3.5-1: Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf. 2.1 |  |  |  |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | Config 1,2,4 | MHz | 10: NRB,c = 25 |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 25 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC configuration for CD-SSB |  |  |  | SMTC.1 |  |  |  |
| SMTC configuration for NCD-SSB |  |  |  | SMTC.2 RedCap |  |  |  |
| CD-SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |
| NCD-SSB Configuration |  | Config 1,2,4 |  | SSB.6 RedCap FR1 Note 1 |  |  |  |
|  |  | Config 3 |  | SSB.7 RedCap FR1 Note 1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PRACH configuration |  |  |  | PRACH.1 FR1 |  |  |  |
| BWP configuration | Initial DL BWP |  |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 RedCap Note 2 |  | DLBWP.1.1 RedCap Note 2 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 RedCap Note 3 |  | ULBWP.1.1 RedCap Note 3 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 4 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 4 |  |  |  |  |  |  |  |
| Note5 |  |  | dBm/15kHz | -98 |  |  |  |
| Note5 |  | Config 1,2,4 | dBm/SCS | -98 |  |  |  |
|  |  | Config 3 |  | -95 |  |  |  |
|  |  |  | dB | 8 | -0.64 | -Infinity | -0.64 |
|  |  |  | dB | 8 | 8 | -Infinity | 8 |
| SSB_RP |  | Config 1,2,4 | dBm/SCS | -90 | -90 | -Infinity | -90 |
|  |  | Config 3 | dBm/SCS | -87 | -87 | -Infinity | -87 |
| IoNote6 |  | Config 1,2,4 | dBm/9.36MHz | -61.41 | -58.71 | -61.41 | -58.71 |
|  |  | Config 3 | dBm/18.36MHz | -58.48 | -55.78 | -58.48 | -55.78 |
| Propagation condition |  |  |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  |  | 1x1 |  | 1x1 |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP and CD-SSB is configured completely outside the dedicated RedCap DL BWP. Note 2: The starting PRB index for dedicated DL BWP corresponding to NCD-SSB PRB index; Note 3: The starting PRB index for dedicated UL BWP is the same as the dedicated DL BWP corresponding to NCD-SSB PRB index; Note 4: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 6: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The handover delay is measured from the beginning of time period T2 to the time that the UE transmit the first PRACH to Cell 2. The handover delay shall be no larger than: RRC procedure delay + Tinterrupt, where:

-	RRC procedure delay = 10 ms, is the RRC processing delay specified in TS 38.331 [13] clause 12,

-	Tinterrupt = 282 ms, is the interruption time specified in TS 38.133 [6] clause 6.1D.1.2.

This gives a total of 292 ms.

#### 16.3.1.4	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE

16.3.1.4.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements for 2Rx RedCap UE as specified in TS 38.133 [6] clause 6.1D.1.2.

16.3.1.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.3.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.4.

16.3.1.4.4	Test description

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively.

Before the test starts,

-	UE is connected to Cell 1 with active DL BWP and active UL BWP

-	UE is configured with nonCellDefiningSSB-r17 under BWP-DownlinkDedicated which serves as the reference SSB for the serving cell. The nonCellDefiningSSB-r17 corresponds to the NCD-SSB within the active DL BWP.

At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives an RRC handover command from the network indicating the UE to handover to cell 2 with firstActiveDownlinkBWP-Id configured to DL BWP and firstActiveUplinkBWP-Id configured to UL BWP. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

16.3.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.3.1.4.4.1-1.

Table 16.3.1.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.4-1 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.1.4-2 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.1.4-3 | Source cell:NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.1.4-4 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.3.1.4.4.1-2.

Table 16.3.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 16.3.1.4.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 16.3.1.4.4.1-3 below.

Table 16.3.1.4.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

16.3.1.4.4.2	Test procedure

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell and RedCap-specific dedicated BWP of Cell 1 is the activated BWP. The SS transmits NCD-SSB on Cell 1 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

2.	Set the parameters according to T1 in Table 16.3.1.4.5-1.

3. 	Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4.	SS shall transmit an RRCReconfiguration message implying handover to the RedCap-specific dedicated BWP of Cell 2.

5.	At the instant when the last TTI containing the RRCReconfiguration message in step 4) is sent to the UE, the SS shall switch the power settings from T1 to T2 as specified in Table 16.3.1.4.5-1. T2 starts.  The SS also transmits NCD-SSB on Cell 2 with an absolute frequency of fNCDSSB calculated above.

6.	If the UE transmits a PRACH preamble to Cell 2 satisfying the delay requirement specified in clause 16.3.1.4.5 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, the SS shall

-	transmit an RRCReconfiguration message to cause the UE handover back to Cell 1. if the handover fails, switch off and on the UE

or

-	switch off and on the UE.

8.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell and RedCap-specific dedicated BWP of Cell 1 is the activated BWP.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.3.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.3.1.4.4.3-1: RRCReconfiguration (Step 1, 4 and 8, test procedure)

| Derivation Path: Table H.3.2-2 with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 16.3.1.4.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.4.4.3-2: CellGroupConfig (Table 16.3.1.4.4.3-1)

| Derivation Path: Table H.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| smtc | SMTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap |  |  |
| } |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 16.3.1.4.4.3-3 |  |
| } |  |  |  |

Table 16.3.1.4.4.3-3: ServingCellConfig (Table 16.3.1.4.4.3-2)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.3.1.4.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.3.1.4.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.4.4.3-4: BWP-Downlink (Table 16.3.1.4.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.1 RedCap corresponding to NCD-SSB PRB index |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.4.4.3-5: BWP-Uplink (Table 16.3.1.4.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.1 RedCap. The starting PRB index for dedicated UL BWP is the same as the corresponding RedCap-specific dedicated DL BWP |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.3.1.4.5	Test requirement

Table 16.3.1.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 16.3.1.4.5-1: Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf. 2.1 |  |  |  |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | Config 1,2,4 | MHz | 10: NRB,c = 25 |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 25 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |
| TRS configuration |  | Config 1, 4 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC configuration for CD-SSB |  |  |  | SMTC.1 |  |  |  |
| SMTC configuration for NCD-SSB |  |  |  | SMTC.2 RedCap |  |  |  |
| CD-SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |
| NCD-SSB Configuration |  | Config 1,2,4 |  | SSB.6 RedCap FR1 Note 1 |  |  |  |
|  |  | Config 3 |  | SSB.7 RedCap FR1 Note 1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PRACH configuration |  |  |  | PRACH.1 FR1 |  |  |  |
| BWP configuration | Initial DL BWP |  |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 RedCap Note 2 |  | DLBWP.1.1 RedCap Note 2 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 RedCap Note 3 |  | ULBWP.1.1 RedCap Note 3 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 4 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 4 |  |  |  |  |  |  |  |
| Note5 |  |  | dBm/15kHz | -98 |  |  |  |
| Note5 |  | Config 1,2,4 | dBm/SCS | -98 |  |  |  |
|  |  | Config 3 |  | -95 |  |  |  |
|  |  |  | dB | 8 | -0.64 | -Infinity | -0.64 |
|  |  |  | dB | 8 | 8 | -Infinity | 8 |
| SSB_RP |  | Config 1,2,4 | dBm/SCS | -90 | -90 | -Infinity | -90 |
|  |  | Config 3 | dBm/SCS | -87 | -87 | -Infinity | -87 |
| IoNote6 |  | Config 1,2,4 | dBm/9.36MHz | -61.41 | -58.71 | -61.41 | -58.71 |
|  |  | Config 3 | dBm/18.36MHz | -58.41 | -55.71 | -58.41 | -55.71 |
| Propagation condition |  |  |  | AWGN |  | AWGN |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP and CD-SSB is configured completely outside the dedicated RedCap DL BWP. Note 2: The starting PRB index for dedicated DL BWP corresponding to NCD-SSB PRB index; Note 3: The starting PRB index for dedicated UL BWP is the same as the dedicated DL BWP corresponding to NCD-SSB PRB index. Note 4: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 6: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The handover delay is measured from the beginning of time period T2 to the time that the UE transmit the first PRACH to Cell 2. The handover delay shall be no larger than: RRC procedure delay + Tinterrupt, where:

-	RRC procedure delay = 10 ms, is the RRC processing delay specified in TS 38.331 [13] clause 12,

-	Tinterrupt = 202 ms, is the interruption time specified in TS 38.133 [6] clause 6.1D.1.2.

This gives a total of 212 ms.

#### 16.3.1.5	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE

16.3.1.5.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 inter frequency handover requirements for 1 Rx RedCap UE as specified in TS 38.133 [6] clause 6.1D.1.2.

16.3.1.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.3.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.5.

16.3.1.5.4	Test description

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives an RRC handover command from the network indicating the UE to handover to Cell 2 with firstActiveDownlinkBWP-Id configured to DL BWP and firstActiveUplinkBWP-Id configured to UL BWP, where cell 2’s DL BWP is the Redcap specific BWP associated with NCD-SSB, then UE handover from cell 1’s CD-SSB to cell 2’s specific Redcap BWP associated with NCD-SSB. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

16.3.1.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.3.1.5.4.1-1.

Table 16.3.1.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.5-1 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.1.5-2 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.1.5-3 | Source cell:NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.1.5-4 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.3.1.5.4.1-2.

Table 16.3.1.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 16.3.1.5.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 16.3.1.5.4.1-3 and 16.3.1.5.5-1 below.

Table 16.3.1.5.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

16.3.1.5.4.2	Test procedure

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 16.3.1.5.5-1.

3. 	Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4.	SS shall transmit an RRCReconfiguration message implying handover to the RedCap-specific dedicated BWP of Cell 2.

5.	At the instant when the last TTI containing the RRCReconfiguration message in step 4) is sent to the UE, the SS shall switch the power settings from T1 to T2 as specified in Table 16.3.1.5.5-1. T2 starts. The SS transmits NCD-SSB on Cell 2 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;

-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

6.	If the UE transmits a PRACH preamble to Cell 2 satisfying the delay requirement specified in clause 16.3.1.5.5 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, the SS shall

-	transmit an RRCReconfiguration message to cause the UE handover back to Cell 1. if the handover fails, switch off and on the UE

or

-	switch off and on the UE.

8.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.3.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.3.1.5.4.3-1: RRCReconfiguration (Step 4, test procedure)

| Derivation Path: Table H.3.2-2 with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 16.3.1.5.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.5.4.3-2: CellGroupConfig (Table 16.3.1.5.4.3-1)

| Derivation Path: Table H.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| smtc | SMTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap |  |  |
| } |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 16.3.1.5.4.3-3 |  |
| } |  |  |  |

Table 16.3.1.5.4.3-3: ServingCellConfig (Table 16.3.1.5.4.3-2)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.3.1.5.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.3.1.5.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.5.4.3-4: BWP-Downlink (Table 16.3.1.5.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.1 RedCap corresponding to NCD-SSB PRB index |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.5.4.3-5: BWP-Uplink (Table 16.3.1.5.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.1 RedCap The starting PRB index for dedicated UL BWP is the same as the corresponding RedCap-specific dedicated DL BWP |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.3.1.5.5	Test requirement

Table 16.3.1.5.5-1 defines the primary level settings including test tolerances for all tests.

Table 16.3.1.5.5-1: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| BWP BW | Config 1,2,4 | MHz | 10: NRB,c = 25 |  |  |  |
|  | Config 3 |  | 20: NRB,c = 25 |  |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR. 2.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC configuration for CD-SSB |  |  | SMTC.1 |  |  |  |
| SMTC configuration for NCD-SSB |  |  | SMTC.2 RedCap |  |  |  |
| CD-SSB Configuration | Config 1,2,4 |  | SSB.1 FR1 |  | N/A |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  | N/A |  |
| NCD-SSB Configuration | Config 1,2,4 |  | N/A |  | SSB.6 RedCap FR1 Note 4 |  |
|  | Config 3 |  | N/A |  | SSB.7 RedCap FR1 Note 4 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR1 |  |  |  |
| Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |
| Dedicated DL BWP |  |  | DLBWP.1.1 RedCap Note 5 |  | DLBWP.1.1 RedCap Note 6 |  |
| Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |
| Dedicated UL BWP |  |  | DLBWP.1.1 RedCap Note 7 |  | ULBWP.1.1 RedCap Note 8 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/ 15kHz | -98 |  | -98 |  |
| Note2 | Config 1,2,4 | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  | -95 |  | -95 |  |
|  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2,4 | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 | dBm/ 18.36MHz | -61.67 | -61.67 | -67.13 | -60.93 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: NCD-SSB is configured within dedicated RedCap DL BWP. Note 5: The starting PRB index for dedicated DL BWP corresponding to CD-SSB PRB index; Note 6: The starting PRB index for dedicated DL BWP corresponding to NCD-SSB PRB index; Note 7: The starting PRB index for dedicated UL BWP is the same as the dedicated DL BWP corresponding to CD-SSB PRB index; Note 8: The starting PRB index for dedicated UL BWP is the same as the dedicated DL BWP corresponding to NCD-SSB PRB index; |  |  |  |  |  |  |

The handover delay is measured from the beginning of time period T2 to the time that the UE transmit the first PRACH to Cell 2. The handover delay shall be no larger than: RRC procedure delay + Tinterrupt, where:

-	RRC procedure delay = 10 ms, is the RRC processing delay specified in TS 38.331 [13] clause 12,

-	Tinterrupt = 522 ms, is the interruption time specified in TS 38.133 [6] clause 6.1D.1.2.

This gives a total of 532 ms.

#### 16.3.1.6	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE

16.3.1.6.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 inter frequency handover requirements for 2 Rx RedCap UE as specified in TS 38.133 [6] clause 6.1D.1.2.

16.3.1.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.3.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.6.

16.3.1.6.4	Test description

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives an RRC handover command from the network indicating the UE to handover to Cell 2 with firstActiveDownlinkBWP-Id configured to DL BWP and firstActiveUplinkBWP-Id configured to UL BWP, where cell 2’s DL BWP is the Redcap specific BWP associated with NCD-SSB, then UE handover from cell 1’s CD-SSB to cell 2’s specific Redcap BWP associated with NCD-SSB. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

16.3.1.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.3.1.6.4.1-1.

Table 16.3.1.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.6-1 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.1.6-2 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.1.6-3 | Source cell:NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.1.6-4 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.3.1.6.4.1-2.

Table 16.3.1.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 16.3.1.6.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 16.3.1.6.4.1-3 and 16.3.1.6.5-1 below.

Table 16.3.1.6.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

16.3.1.6.4.2	Test procedure

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 16.3.1.6.5-1.

3. 	Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4.	SS shall transmit an RRCReconfiguration message implying handover to the RedCap-specific dedicated BWP of Cell 2.

5.	At the instant when the last TTI containing the RRCReconfiguration message in step 4) is sent to the UE, the SS shall switch the power settings from T1 to T2 as specified in Table 16.3.1.6.5-1. T2 starts. . The SS transmits NCD-SSB on Cell 2 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;

-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

6.	If the UE transmits a PRACH preamble to Cell 2 satisfying the delay requirement specified in clause 16.3.1.6.5 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, the SS shall

-	transmit an RRCReconfiguration message to cause the UE handover back to Cell 1. if the handover fails, switch off and on the UE

or

-	switch off and on the UE.

8.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.3.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.3.1.6.4.3-1: RRCReconfiguration (Step 4, test procedure)

| Derivation Path: Table H.3.2-2 with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 16.3.1.6.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.6.4.3-2: CellGroupConfig (Table 16.3.1.6.4.3-1)

| Derivation Path: Table H.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| smtc | SMTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap |  |  |
| } |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 16.3.1.6.4.3-3 |  |
| } |  |  |  |

Table 16.3.1.6.4.3-3: ServingCellConfig (Table 16.3.1.6.4.3-2)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.3.1.6.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.3.1.6.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.6.4.3-4: BWP-Downlink (Table 16.3.1.6.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.1 RedCap corresponding to NCD-SSB PRB index |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.3.1.6.4.3-5: BWP-Uplink (Table 16.3.1.6.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.1 RedCap. The starting PRB index for dedicated UL BWP is the same as the corresponding RedCap-specific dedicated DL BWP |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.3.1.6.5	Test requirement

Table 16.3.1.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 16.3.1.6.5-1: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| BWP BW | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR. 2.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC configuration for CD-SSB |  |  | SMTC.1 |  |  |  |
| SMTC configuration for NCD-SSB |  |  | SMTC.2 RedCap |  |  |  |
| CD-SSB Configuration | Config 1,2,4 |  | SSB.1 FR1 |  | N/A |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  | N/A |  |
| NCD-SSB Configuration | Config 1,2,4 |  | N/A |  | SSB.6 RedCap FR1 Note 4 |  |
|  | Config 3 |  | N/A |  | SSB.7 RedCap FR1 Note 4 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR1 |  |  |  |
| Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |
| Dedicated DL BWP |  |  | DLBWP.1.1 RedCap Note 5 |  | DLBWP.1.1 RedCap Note 6 |  |
| Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |
| Dedicated UL BWP |  |  | DLBWP.1.1 RedCap Note 7 |  | ULBWP.1.1 RedCap Note 8 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/ 15kHz | -98 |  | -98 |  |
| Note2 | Config 1,2,4 | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  | -95 |  | -95 |  |
|  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2,4 | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 | dBm/ 18.36MHz | -61.67 | -61.67 | -67.13 | -60.93 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: NCD-SSB is configured within dedicated RedCap DL BWP. Note 5: The starting PRB index for dedicated DL BWP corresponding to CD-SSB PRB index; Note 6: The starting PRB index for dedicated DL BWP corresponding to NCD-SSB PRB index; Note 7: The starting PRB index for dedicated UL BWP is the same as the dedicated DL BWP corresponding to CD-SSB PRB index; Note 8: The starting PRB index for dedicated UL BWP is the same as the dedicated DL BWP corresponding to NCD-SSB PRB index; |  |  |  |  |  |  |

The handover delay is measured from the beginning of time period T2 to the time that the UE transmit the first PRACH to Cell 2. The handover delay shall be no larger than: RRC procedure delay + Tinterrupt, where:

-	RRC procedure delay = 10 ms, is the RRC processing delay specified in TS 38.331 [13] clause 12,

-	Tinterrupt = 362 ms, is the interruption time specified in TS 38.133 [6] clause 6.1D.1.2.

This gives a total of 372 ms.

#### 16.3.1.7	NR SA FR1 - E-UTRA handover for 1Rx UE

16.3.1.7.1	Test purpose

To verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall also verify the NR to E-UTRAN handover requirements as specified in 38.133 [6] clause 6.1D.2.1.

16.3.1.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.3.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.7.

16.3.1.7.4	Test description

16.3.1.7.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.1.4.4.1 with the following exceptions:

-	Table 6.3.1.4.4.1-1 is replaced by Table 16.3.1.7.4.1-1.

-	Table 6.3.1.4.4.1-2 is replaced by Table 16.3.1.7.4.1-2.

-	Table 6.3.1.4.4.1-3 is replaced by Table 16.3.1.7.4.1-3

Table 16.3.1.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.7-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.3.1.7-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.1.7-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.1.7-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.3.1.7-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.1.7-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.1.7-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD mode, LTE FDD |
| 16.3.1.7-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.1.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.3.1.7.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | As specified in Table 16.3.1.7.5-1 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2EUTRAN |  | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in 38.133 [6] Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

16.3.1.7.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.1.4.4.2.

16.3.1.7.4.3	Message contents

Same as the message contents specified in 6.3.1.4.4.3 with the following exceptions:

-	Table 7.3.1-3 in TS 38.508-1 [14] is used with condition SMTC.1 RedCap.

-	Table H.3.1-4A is replaced by Table 16.3.1.7.4.3-1.

Table 16.3.1.7.4.3-1: ReportConfigInterRAT- DEFAULT

| Derivation Path: Table H.3.1-4A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | 63 | Actual value = 63 – 156 = -93dBm | 16.3.1.7-1 16.3.1.7-2 16.3.1.7-4 16.3.1.7-5 16.3.1.7-7 16.3.1.7-8 |
|  | 66 | Actual value = 66 – 156 = -90dBm | 16.3.1.7-3 16.3.1.7-6 |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | 42 | Actual value = 42 – 140 = -98dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.3.1.7.5	Test requirement

Same as the test requirements given in sub-clause 6.3.1.4.5 with following exceptions:

-	Table 6.3.1.4.4.1-3 is replaced by Table 16.3.1.7.4.1-3.

-	Table 6.3.1.4.5-1 is replaced by Table 16.3.1.7.5-1.

-	Table 6.3.1.4.5-2 is replaced by Table 16.3.1.7.5-2.

-	Tinterrupt in test requirements is specified in 38.133 [6] clause 6.1D.2.1.

Table 16.3.1.7.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |  |
| Duplex mode |  |  | 1,4 | FDD |  |  |
|  |  |  | 2,3,5,6 | TDD |  |  |
|  |  |  | 7,8 | HD-FDD |  |  |
| TDD Configuration |  |  | 2,5 | TDDConf.1.1 |  |  |
|  |  |  | 3,6 | TDDConf.1.2 |  |  |
| BWchannel |  | MHz | 1,4,7,8 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2,5 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3,6 | 20: NRB,c = 51 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1,4,7,8 | SR.1.1 FDD |  |  |
|  |  |  | 2,5 | SR.1.1 TDD |  |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1,4,7,8 | CR.1.1 FDD |  |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1,4,7,8 | TRS.1.1 FDD |  |  |
|  |  |  | 2,5 | TRS.1.1 TDD |  |  |
|  |  |  | 3,6 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |  |
| BWP | Initial DL BWP |  |  | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 RedCap |  |  |
| SSB configuration |  |  | 1,2,4,5,7,8 | SSB.1 FR1 |  |  |
|  |  |  | 3,6 | SSB.1 RedCap FR1 |  |  |
| b2-Threshold1Note 4 |  | dBm | 1,2,4,5,7,8 | -93 |  |  |
|  |  |  | 3,6 | -90 |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | 1,2,3,4,5,6,7,8 | 0 |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -100 | -104 | -100 |
| NocNote2 |  | dBm/SCS | 1,2,4,5,7,8 | -100 | -104 | -100 |
|  |  |  | 3,6 | -97 | -101 | -97 |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | 12 | 0 | 0 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | 12 | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -88 | -104 | -100 |
|  |  |  | 3,6 | -85 | -101 | -97 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,7,8 | -59.78 | -73.04 | -69.04 |
|  |  | dBm/18.36 MHz | 3,6 | -56.86 | -70.11 | -66.11 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Including test tolerance given in Annex F. |  |  |  |  |  |  |

Table 16.3.1.7.5-2: Cell specific test parameters (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |  |
|  |  | 4,5,6,8 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH |  | 1,2,3,7 | 4 |  |  |
| ConfigurationNote2 |  | 4,5,6,8 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement |  | 1,2,3,7 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
| ChannelNote3 |  | 4,5,6,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference |  | 1,2,3,7 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
| Measurement ChannelNote3 |  | 4,5,6,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1,2,3,7 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 4,5,6,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 10.55 | 8 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 10.55 | 8 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -87.45 | -90 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -87.45 | -90 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -67.21 +10log(NRB,c/100) | -56.29 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [24]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |  |

#### 16.3.1.8	NR SA FR1 - E-UTRA handover for 2Rx UE

Editor's Note:

- TT analysis needs to be updated

16.3.1.8.1	Test purpose

To verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall also verify the NR to E-UTRAN handover requirements as specified in 38.133 [6] clause 6.1D.2.1.

16.3.1.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.3.1.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.8.

16.3.1.8.4	Test description

16.3.1.8.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.1.4.4.1 with the following exceptions:

-	Table 6.3.1.4.4.1-1 is replaced by Table 16.3.1.8.4.1-1.

-	Table 6.3.1.4.4.1-2 is replaced by Table 16.3.1.8.4.1-2.

-	Table 6.3.1.4.4.1-3 is replaced by Table 16.3.1.8.4.1-3

Table 16.3.1.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.8-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.3.1.8-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.1.8-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.1.8-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.3.1.8-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.1.8-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.1.8-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD mode, LTE FDD |
| 16.3.1.8-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.1.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.3.1.8.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | As specified in Table 16.3.1.8.5-2 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2EUTRAN |  | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in 38.133 [6] Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

16.3.1.8.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.1.4.4.2.

16.3.1.8.4.3	Message contents

Same as the message contents specified in 6.3.1.4.4.3 with the following exceptions:

-	Table 7.3.1-3 in TS 38.508-1 [14] is used with condition SMTC.1 RedCap.

16.3.1.8.5	Test requirement

Same as the test requirements given in sub-clause 6.3.1.4.5 with following exceptions:

-	Table 6.3.1.4.4.1-3 is replaced by Table 16.3.1.8.4.1-3.

-	Table 6.3.1.4.5-1 is replaced by Table 16.3.1.8.5-1.

-	Table 6.3.1.4.5-2 is replaced by Table 16.3.1.8.5-2.

-	Tinterrupt in test requirements is specified in 38.133 [6] clause 6.1D.2.1.

Table 16.3.1.8.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |  |
| Duplex mode |  |  | 1,4 | FDD |  |  |
|  |  |  | 2,3,5,6 | TDD |  |  |
|  |  |  | 7,8 | HD-FDD |  |  |
| TDD Configuration |  |  | 2,5 | TDDConf.1.1 |  |  |
|  |  |  | 3,6 | TDDConf.1.2 |  |  |
| BWchannel |  | MHz | 1,4,7,8 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2,5 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3,6 | 20: NRB,c = 51 (TDD) |  |  |
| PDSCH reference measurement |  |  | 1,4,7,8 | SR.1.1 FDD |  |  |
| channel |  |  | 2,5 | SR.1.1 TDD |  |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1,4,7,8 | CR.1.1 FDD |  |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1,4,7,8 | TRS.1.1 FDD |  |  |
|  |  |  | 2,5 | TRS.1.1 TDD |  |  |
|  |  |  | 3,6 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |  |
| BWP | Initial DL BWP |  |  | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 RedCap |  |  |
| SSB configuration |  |  | 1,2,4,5,7,8 | SSB.1 FR1 |  |  |
|  |  |  | 3,6 | SSB.1 RedCap FR1 |  |  |
| b2-Threshold1 |  | dBm | 1,2,4,5,7,8 | -96 |  |  |
|  |  |  | 3,6 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | 1,2,3,4,5,6,7,8 | 0 |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -100+TT | -104+TT | -100+TT |
| NocNote2 |  | dBm/SCS | 1,2,4,5,7,8 | -100+TT | -104+TT | -100+TT |
|  |  |  | 3,6 | -97+TT | -101+TT | -97+TT |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | 12+TT | 0+TT | 0+TT |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | 12+TT | 0+TT | 0+TT |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -88+TT | -104+TT | -100+TT |
|  |  |  | 3,6 | -85+TT | -101+TT | -97+TT |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,7,8 | -59.78+TT | -73.04+TT | -69.04 |
|  |  | dBm/18.36 MHz | 3,6 | -56.86+TT | -70.11+TT | -66.11 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1,2,3,4,5,6,7.8 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 16.3.1.8.5-2: Cell specific test parameters (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |  |
|  |  | 4,5,6,8 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH |  | 1,2,3,7 | 4 |  |  |
| ConfigurationNote2 |  | 4,5,6,8 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement |  | 1,2,3,7 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
| ChannelNote3 |  | 4,5,6,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference |  | 1,2,3,7 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
| Measurement ChannelNote3 |  | 4,5,6,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1,2,3,7 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 4,5,6,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98+TT |  |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 8+TT | 8+TT |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 8+TT | 8+TT |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -90+TT | -90+TT |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -90+TT | -90+TT |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -67.21 +10log(NRB,c/100) | -58.57+TT +10log(NRB,c/100) | -58.57+TT +10log(NRB,c/100) |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [24]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 4: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |  |

#### 16.3.1.9	NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE

16.3.1.9.1	Test purpose

To verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall also verify the NR to E-UTRAN handover requirements for the case when the target E-UTRAN cell is unknown as specified in 38.133 [6] clause 6.1D.2.1.

16.3.1.9.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.3.1.9.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.9.

16.3.1.9.4	Test description

16.3.1.9.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.1.5.4.1 with the following exceptions:

-	Table 6.3.1.5.4.1-1 is replaced by Table 16.3.1.9.4.1-1.

-	Table 6.3.1.5.4.1-2 is replaced by Table 16.3.1.9.4.1-2.

-	Table 6.3.1.5.4.1-3 is replaced by Table 16.3.1.9.4.1-3

Table 16.3.1.9.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.9-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.3.1.9-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.1.9-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.1.9-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.3.1.9-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.1.9-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.1.9-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD mode, LTE FDD |
| 16.3.1.9-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.1.9.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.9.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.3.1.9.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

16.3.1.9.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.1.5.4.2.

16.3.1.9.4.3	Message contents

Same as the message contents specified in 6.3.1.5.4.3.

16.3.1.9.5	Test requirement

Same as the test requirements given in sub-clause 6.3.1.5.5 with following exceptions:

-	Table 6.3.1.5.4.1-3 is replaced by Table 16.3.1.9.4.1-3.

-	Table 6.3.1.5.5-1 is replaced by Table 16.3.1.9.5-1.

-	Table 6.3.1.5.5-2 is replaced by Table 16.3.1.9.5-2.

-	Tinterrupt in test requirements is specified in 38.133 [6] clause 6.1D.2.1.

Table 16.3.1.9.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |
| Duplex mode |  |  | 1,4 | FDD |  |
|  |  |  | 2,3,5,6 | TDD |  |
|  |  |  | 7,8 | HD-FDD |  |
| TDD Configuration |  |  | 2,5 | TDDConf.1.1 |  |
|  |  |  | 3,6 | TDDConf.1.2 |  |
| BWchannel |  | MHz | 1,4,7,8 | 10: NRB,c = 52 (FDD) |  |
|  |  |  | 2,5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  | 3,6 | 20: NRB,c = 51 (TDD) |  |
| PDSCH reference measurement channel |  |  | 1,4,7,8 | SR.1.1 FDD |  |
|  |  |  | 2,5 | SR.1.1 TDD |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  | 1,4,7,8 | CR.1.1 FDD |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |
| TRS configuration |  |  | 1,4,7,8 | TRS.1.1 FDD |  |
|  |  |  | 2,5 | TRS.1.1 TDD |  |
|  |  |  | 3,6 | TRS.1.2 TDD |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |
| BWP | Initial DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 RedCap |  |
| SSB configuration |  |  | 1,2,4,5,7,8 | SSB.1 FR1 |  |
|  |  |  | 3,6 | SSB.1 RedCap FR1 |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | 1,2,3,4,5,6,7,8 | 0 |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -98 |  |
| NocNote2 |  | dBm/SCS | 1,2,4,5,7,8 | -98 |  |
|  |  |  | 3,6 | -95 |  |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | 0 | 0 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -98 | -98 |
|  |  |  | 3,6 | -95 | -95 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,7,8 | -67.04 | -67.04 |
|  |  | dBm/38.16 MHz | 3,6 | -64.12 | -64.12 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 16.3.1.9.5-2: Cell specific test parameters (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |
|  |  | 4,5,6,8 | TDD |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1,2,3,7 | 4 |  |
|  |  | 4,5,6,8 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,7 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,7 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1,2,3,7 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -91 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -91 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -70.22 | -62.43 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [24]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

#### 16.3.1.10	NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE

16.3.1.10.1	Test purpose

To verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall also verify the NR to E-UTRAN handover requirements for the case when the target E-UTRAN cell is unknown as specified in 38.133 [6] clause 6.1D.2.1.

16.3.1.10.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.3.1.10.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.10.

16.3.1.10.4	Test description

16.3.1.10.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.1.5.4.1 with the following exceptions:

-	Table 6.3.1.5.4.1-1 is replaced by Table 16.3.1.10.4.1-1.

-	Table 6.3.1.5.4.1-2 is replaced by Table 16.3.1.10.4.1-2.

-	Table 6.3.1.5.4.1-3 is replaced by Table 16.3.1.10.4.1-3

Table 16.3.1.10.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.10-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.3.1.10-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.1.10-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.1.10-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.3.1.10-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.1.10-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.1.10-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD mode, LTE FDD |
| 16.3.1.10-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.1.10.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.10.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.3.1.10.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

16.3.1.10.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.1.5.4.2.

16.3.1.10.4.3	Message contents

Same as the message contents specified in 6.3.1.5.4.3.

16.3.1.10.5	Test requirement

Same as the test requirements given in sub-clause 6.3.1.5.5 with following exceptions:

-	Table 6.3.1.5.4.1-3 is replaced by Table 16.3.1.10.4.1-3.

-	Table 6.3.1.5.5-1 is replaced by Table 16.3.1.10.5-1.

-	Table 6.3.1.5.5-2 is replaced by Table 16.3.1.10.5-2.

-	Tinterrupt in test requirements is specified in 38.133 [6] clause 6.1D.2.1.

Table 16.3.1.10.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |
| Duplex mode |  |  | 1,4 | FDD |  |
|  |  |  | 2,3,5,6 | TDD |  |
|  |  |  | 7,8 | HD-FDD |  |
| TDD Configuration |  |  | 2,5 | TDDConf.1.1 |  |
|  |  |  | 3,6 | TDDConf.1.2 |  |
| BWchannel |  | MHz | 1,4,7,8 | 10: NRB,c = 52 (FDD) |  |
|  |  |  | 2,5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  | 3,6 | 20: NRB,c = 51 (TDD) |  |
| PDSCH reference measurement |  |  | 1,4,7,8 | SR.1.1 FDD |  |
| channel |  |  | 2,5 | SR.1.1 TDD |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  | 1,4,7,8 | CR.1.1 FDD |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |
| TRS configuration |  |  | 1,4,7,8 | TRS.1.1 FDD |  |
|  |  |  | 2,5 | TRS.1.1 TDD |  |
|  |  |  | 3,6 | TRS.1.2 TDD |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |
| BWP | Initial DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 RedCap |  |
| SSB configuration |  |  | 1,2,4,5,7,8 | SSB.1 FR1 |  |
|  |  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH-DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH-DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH-DMRS to SSS |  |  | 1,2,3,4,5,6,7,8 | 0 |  |
| EPRE ratio of PDCCH to PDCCH-DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH-DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -98 |  |
| NocNote2 |  | dBm/SCS | 1,2,4,5,7,8 | -98 |  |
|  |  |  | 3,6 | -95 |  |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | 0 | 0 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -98 | -98 |
|  |  |  | 3,6 | -95 | -95 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,7,8 | -67.04 | -67.04 |
|  |  | dBm/18.36 MHz | 3,6 | -64.12 | -64.12 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 16.3.1.10.5-2: Cell specific test parameters (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |
|  |  | 4,5,6,8 | TDD |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1,2,3,7 | 4 |  |
|  |  | 4,5,6,8 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,7 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,7 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1,2,3,7 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -91 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -91 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -70.22 | -62.43 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [24]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

### 16.3.2	RRC connection mobility control for RedCap

#### 16.3.2.1	RRC re-establishment for RedCap

##### 16.3.2.1.0	Minimum conformance requirements

16.3.2.1.0.1	Minimum conformance requirements for FR1 RRC re-establishment

The requirements in clause 6.3.2.1.0.1 shall apply when RedCap UE is capable of 2 Rx.

When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.3.2.1.0.1 shall apply except that:

-	Tidentify_intra_NR as specified in Table 16.3.2.1.0.1-1.

-	Tidentify_inter_NR, i as specified in Table 16.3.2.1.0.1-2.

Table 16.3.2.1.0.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC) | MAX (800 ms, [11] x TSMTC) |
| < -8 | FR1 | N/A | 800Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Table 16.3.2.1.0.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 7 x TSMTC, i) | MAX (800 ms, [14] x TSMTC, i) |
| < -8 | FR1 | N/A | 800Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.1B.

##### 16.3.2.1.1	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE

16.3.2.1.1.1	Test purpose

To verify that the NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.1B.

16.3.2.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.1.1.

16.3.2.1.1.4	Test description

16.3.2.1.1.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.1.1.4.1 with following exceptions:

-	Table 6.3.2.1.1.4.1-1 is replaced by Table 16.3.2.1.1.4.1-1.

-	Table 6.3.2.1.1.4.1-2 is replaced by Table 16.3.2.1.1.4.1-2.

-	Table 6.3.2.1.1.4.1-3 is replaced by Table 16.3.2.1.1.4.1-3

Table 16.3.2.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.1.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.1.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.3.2.1.1.4.1-3: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1,2,3,4 | Cell1 |  |
|  | Neighbour cells |  | 1,2,3,4 | Cell2 |  |
| Final condition | Active cell |  | 1,2,3,4 | Cell2 |  |
| RF Channel Number |  |  | 1,2,3,4 | 1 |  |
| Time offset between cells |  |  | 1,4 | 3 ms | Asynchronous cells |
|  |  |  | 2,3 | 3 s | Synchronous cells |
| N310 |  | - | 1,2,3,4 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1,2,3,4 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1,2,3,4 | 0 | Radio link failure timer; |
| T311 |  | ms | 1,2,3,4 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1,2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1,2,4 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1,4 | SMTC.2 |  |
|  |  |  | 2,3 | SMTC.1 |  |
| DRX cycle length |  | s | 1,2,3,4 | OFF |  |
| PRACH configuration |  |  | 1,2,3,4 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1,2,3,4 | 5 |  |
| T2 |  | ms | 1,2,3,4 | 440 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1B in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1B.5 in TS 38.133[6]) |
| T3 |  | s | 1,2,3,4 | 2 |  |

16.3.2.1.1.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.1.1.4.2.

16.3.2.1.1.4.3	Message contents

Same as the message contents given in 6.3.2.1.1.4.3 with the following exceptions:

-	Table 7.3.1-3 in 38.508-1 [14] is used with condition SMTC.2 for Test Configuration 16.3.2.1.1-4.

16.3.2.1.1.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.1.1.5 with following exceptions:

-	Table 6.3.2.1.1.5-1 is replaced by Table 16.3.2.1.1.5-1.

Table 16.3.2.1.1.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1,4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC |  | 1,4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC |  | 1,4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1,2,3,4 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial DL BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.1 | N/A | N/A | N/A | N/A | DLBWP.1.1 |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 | N/A | N/A | N/A | N/A | ULBWP.1.1 |
| RLM-RS |  | 1,2,3,4 | SSB |  |  | SSB |  |  |
|  | dB | 1,2,3,4 | 1.54 | -infinity | -infinity | -3.79 | 4 | 4 |
| Note2 | dBm/SCS | 1,2,4 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  |  |  |  |  |
|  | dB | 1,2,3,4 | 7 | -infinity | -infinity | 4 | 4 | 4 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4 | -91 | -infinity | -infinity | -94 | -94 | -94 |
|  |  | 3 | -88 | -infinity | -infinity | -91 | -91 | -91 |
| Io | dBm/9.36 MHz | 1,2,4 | -60.74 | -64.59 | -64.59 | -60.74 | -64.59 | -64.59 |
|  | dBm/18.36 MHz | 3 | -57.81 | -61.66 | -61.66 | -57.81 | -61.66 | -61.66 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### 16.3.2.1.2	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE

16.3.2.1.2.1	Test purpose

To verify that the NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.1B.

16.3.2.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.3.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.1.2.

16.3.2.1.2.4	Test description

16.3.2.1.2.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.1.1.4.1 with following exceptions:

-	Table 6.3.2.1.1.4.1-1 is replaced by Table 16.3.2.1.2.4.1-1.

-	Table 6.3.2.1.1.4.1-2 is replaced by Table 16.3.2.1.2.4.1-2.

-	Table 6.3.2.1.1.4.1-3 is replaced by Table 16.3.2.1.2.4.1-3

Table 16.3.2.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.1.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.1.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.3.2.1.2.4.1-3: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1,2,3,4 | Cell1 |  |
|  | Neighbour cells |  | 1,2,3,4 | Cell2 |  |
| Final condition | Active cell |  | 1,2,3,4 | Cell2 |  |
| RF Channel Number |  |  | 1,2,3,4 | 1 |  |
| Time offset between cells |  |  | 1,4 | 3 ms | Asynchronous cells |
|  |  |  | 2,3 | 3 s | Synchronous cells |
| N310 |  | - | 1,2,3,4 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1,2,3,4 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1,2,3,4 | 0 | Radio link failure timer; |
| T311 |  | ms | 1,2,3,4 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1,2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1,2,4 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1,4 | SMTC.2 |  |
|  |  |  | 2,3 | SMTC.1 |  |
| DRX cycle length |  | s | 1,2,3,4 | OFF |  |
| PRACH configuration |  |  | 1,2,3,4 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1,2,3,4 | 5 |  |
| T2 |  | ms | 1,2,3,4 | 240 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1B in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1B.5 in TS 38.133[6]) |
| T3 |  | s | 1,2,3,4 | 2 |  |

16.3.2.1.2.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.1.1.4.2.

16.3.2.1.2.4.3	Message contents

Same as the message contents given in 6.3.2.1.1.4.3 with the following exceptions:

-	Table 7.3.1-3 in 38.508-1 [14] is used with condition SMTC.2 for Test Configuration 16.3.2.1.2-4.

16.3.2.1.2.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.1.1.5 with following exceptions:

-	Table 6.3.2.1.1.5-1 is replaced by Table 16.3.2.1.2.5-1.

Table 16.3.2.1.2.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1,4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC |  | 1,4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC |  | 1,4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1,2,3,4 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial DL BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.1 | N/A | N/A | N/A | N/A | DLBWP.1.1 |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 | N/A | N/A | N/A | N/A | ULBWP.1.1 |
| RLM-RS |  | 1,2,3,4 | SSB |  |  | SSB |  |  |
|  | dB | 1,2,3,4 | 1.54 | -infinity | -infinity | -3.79 | 4 | 4 |
| Note2 | dBm/SCS | 1,2,4 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  |  |  |  |  |
|  | dB | 1,2,3,4 | 7 | -infinity | -infinity | 4 | 4 | 4 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4 | -91 | -infinity | -infinity | -94 | -94 | -94 |
|  |  | 3 | -88 | -infinity | -infinity | -91 | -91 | -91 |
| Io | dBm/9.36 MHz | 1,2,4 | -60.74 | -64.59 | -64.59 | -60.74 | -64.59 | -64.59 |
|  | dBm/18.36 MHz | 3 | -57.81 | -61.66 | -61.66 | -57.81 | -61.66 | -61.66 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### 16.3.2.1.3	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE

16.3.2.1.3.1	Test purpose

To verify that the NR inter-frequency RRC re-establishment delay in FR1 without known target cell is within the specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.1B.

16.3.2.1.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.1.3.

16.3.2.1.3.4	Test description

16.3.2.1.3.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.1.2.4.1 with following exceptions:

-	Table 6.3.2.1.2.4.1-1 is replaced by Table 16.3.2.1.3.4.1-1.

-	Table 6.3.2.1.2.4.1-2 is replaced by Table 16.3.2.1.3.4.1-2.

-	Table 6.3.2.1.1.4.1-3 is replaced by Table 16.3.2.1.3.4.1-3

Table 16.3.2.1.3.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.3.2.1.3-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.1.3-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.3-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.3-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table 16.3.2.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.3.2.1.3.4.1-3: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1,2,3,4 | Cell1 |  |
|  | Neighbour cells |  | 1,2,3,4 | Cell2 |  |
| Final condition | Active cell |  | 1,2,3,4 | Cell2 |  |
| RF Channel Number |  |  | 1,2,3,4 | 1,2 |  |
| Time offset between cells |  |  | 1,4 | 3 ms | Asynchronous cells |
|  |  |  | 2,3 | 3 s | Synchronous cells |
| N310 |  | - | 1,2,3,4 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1,2,3,4 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1,2,3,4 | 0 | Radio link failure timer; |
| T311 |  | ms | 1,2,3,4 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1,2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1,2,4 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1,4 | SMTC.2 |  |
|  |  |  | 2,3 | SMTC.1 |  |
| DRX cycle length |  | s | 1,2,3,4 | OFF |  |
| PRACH configuration |  |  | 1,2,3,4 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1,2,3,4 | 5 |  |
| T2 |  | ms | 1,2,3,4 | 440 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1B in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1B.5 in TS 38.133[6]) |
| T3 |  | s | 1,2,3,4 | 5 |  |

16.3.2.1.3.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.1.2.4.2.

16.3.2.1.3.4.3	Message contents

Same as the message contents given in 6.3.2.1.2.4.3 with the following exceptions:

-	Table 7.3.1-3 in 38.508-1 [14] is used with condition SMTC.2 for Test Configuration 16.3.2.1.3-4.

16.3.2.1.3.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.1.2.5 with following exceptions:

-	Table 6.3.2.1.2.5-1 is replaced by Table 16.3.2.1.3.5-1.

Table 16.3.2.1.3.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1,2,3,4 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1,4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC |  | 1,4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC |  | 1,4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1,2,3,4 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial DL BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.1 | N/A | N/A | N/A | N/A | DLBWP.1.1 |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 | N/A | N/A | N/A | N/A | ULBWP.1.1 |
| RLM-RS |  | 1,2,3,4 | SSB |  |  | SSB |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
| Note2 | dBm/SCS | 1,2,4 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  |  |  |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4 | -94 | -infinity | -infinity | -infinity | -infinity | -91 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -88 |
| Io | dBm/9.36 MHz | 1,2,4 | -64.59 | -70.05 | -70.05 | -70.05 | -70.05 | -62.26 |
|  | dBm/18.36 MHz | 3 | -61.66 | -67.12 | -67.12 | -67.12 | -67.12 | -59.33 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### 16.3.2.1.4	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE

16.3.2.1.4.1	Test purpose

To verify that the NR inter-frequency RRC re-establishment delay in FR1 without known target cell is within the specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.1B.

16.3.2.1.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.3.2.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.1.4.

16.3.2.1.4.4	Test description

16.3.2.1.4.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.1.2.4.1 with following exceptions:

-	Table 6.3.2.1.2.4.1-1 is replaced by Table 16.3.2.1.4.4.1-1.

-	Table 6.3.2.1.2.4.1-2 is replaced by Table 16.3.2.1.4.4.1-2.

-	Table 6.3.2.1.2.4.1-3 is replaced by Table 16.3.2.1.4.4.1-3

Table 16.3.2.1.4.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.3.2.1.4-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.1.4-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.4-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.4-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table 16.3.2.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.3.2.1.4.4.1-3: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1,2,3,4 | Cell1 |  |
|  | Neighbour cells |  | 1,2,3,4 | Cell2 |  |
| Final condition | Active cell |  | 1,2,3,4 | Cell2 |  |
| RF Channel Number |  |  | 1,2,3,4 | 1,2 |  |
| Time offset between cells |  |  | 1,4 | 3 ms | Asynchronous cells |
|  |  |  | 2,3 | 3 s | Synchronous cells |
| N310 |  | - | 1,2,3,4 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1,2,3,4 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1,2,3,4 | 0 | Radio link failure timer; |
| T311 |  | ms | 1,2,3,4 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1,2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1,2,4 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1,4 | SMTC.2 |  |
|  |  |  | 2,3 | SMTC.1 |  |
| DRX cycle length |  | s | 1,2,3,4 | OFF |  |
| PRACH configuration |  |  | 1,2,3,4 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1,2,3,4 | 5 |  |
| T2 |  | ms | 1,2,3,4 | 240 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1B in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1B.5 in TS 38.133[6]) |
| T3 |  | s | 1,2,3,4 | 5 |  |

16.3.2.1.4.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.1.2.4.2.

16.3.2.1.4.4.3	Message contents

Same as the message contents given in 6.3.2.1.2.4.3 with the following exceptions:

-	Table 7.3.1-3 in 38.508-1 [14] is used with condition SMTC.2 for Test Configuration 16.3.2.1.4-4.

16.3.2.1.4.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.1.2.5 with following exceptions:

-	Table 6.3.2.1.2.5-1 is replaced by Table 16.3.2.1.4.5-1.

Table 16.3.2.1.4.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1,2,3,4 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1,4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC |  | 1,4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC |  | 1,4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1,2,3,4 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial DL BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.1 | N/A | N/A | N/A | N/A | DLBWP.1.1 |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 | N/A | N/A | N/A | N/A | ULBWP.1.1 |
| RLM-RS |  | 1,2,3,4 | SSB |  |  | SSB |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
| Note2 | dBm/SCS | 1,2,4 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  |  |  |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4 | -94 | -infinity | -infinity | -infinity | -infinity | -91 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -88 |
| Io | dBm/9.36 MHz | 1,2,4 | -64.59 | -70.05 | -70.05 | -70.05 | -70.05 | -62.26 |
|  | dBm/18.36 MHz | 3 | -61.66 | -67.12 | -67.12 | -67.12 | -67.12 | -59.33 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### 16.3.2.1.5	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing

16.3.2.1.5.1	Test purpose

To verify that the NR intra-frequency RRC re-establishment delay in FR1 without serving cell timing is within the specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.1B.

16.3.2.1.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.1.5.

16.3.2.1.5.4	Test description

16.3.2.1.5.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.1.3.4.1 with following exceptions:

-	Table 6.3.2.1.3.4.1-1 is replaced by Table 16.3.2.1.5.4.1-1.

-	Table 6.3.2.1.3.4.1-2 is replaced by Table 16.3.2.1.5.4.1-2.

-	Table 6.3.2.1.1.4.1-3 is replaced by Table 16.3.2.1.5.4.1-3

Table 16.3.2.1.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.1.5-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.1.5-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.5-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.5-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.1.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.3.2.1.5.4.1-3: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1,2,3,4 | Cell1 |  |
|  | Neighbour cells |  | 1,2,3,4 | Cell2 |  |
| Final condition | Active cell |  | 1,2,3,4 | Cell2 |  |
| RF Channel Number |  |  | 1,2,3,4 | 1 |  |
| Time offset between cells |  |  | 1,4 | 3 ms | Asynchronous cells |
|  |  |  | 2,3 | 3 s | Synchronous cells |
| N310 |  | - | 1,2,3,4 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1,2,3,4 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1,2,3,4 | 6000 | Radio link failure timer; |
| T311 |  | ms | 1,2,3,4 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1,2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1,2,4 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1,4 | SMTC.2 |  |
|  |  |  | 2,3 | SMTC.1 |  |
| DRX cycle length |  | s | 1,2,3,4 | OFF |  |
| PRACH configuration |  |  | 1,2,3,4 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1,2,3,4 | 5 |  |
| T2 |  | s | 1,2,3,4 | 6.24 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1B in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1B.5 in TS 38.133[6]) |
| T3 |  | s | 1,2,3,4 | 3 |  |

16.3.2.1.5.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.1.3.4.2.

16.3.2.1.5.4.3	Message contents

Same as the message contents given in 6.3.2.1.3.4.3 with the following exceptions:

-	Table 7.3.1-3 in 38.508-1 [14] is used with condition SMTC.2 for Test Configuration 16.3.2.1.5-4.

16.3.2.1.5.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.1.3.5 with following exceptions:

-	Table 6.3.2.1.3.5-1 is replaced by Table 16.3.2.1.5.5-1.

Table 16.3.2.1.5.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1,4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC |  | 1,4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC |  | 1,4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1,2,3,4 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial DL BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1,2,3,4 | SSB |  |  | SSB |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
| Note2 | dBm/SCS | 1,2,4 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  |  |  |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -91 |
| Io | dBm/9.36 MHz | 1,2,4 | -64.59 | -70.05 | -70.05 | -70.05 | -70.05 | -64.59 |
|  | dBm/18.36 MHz | 3 | -61.68 | -67.12 | -67.12 | -67.12 | -67.12 | -61.68 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### 16.3.2.1.6	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing

16.3.2.1.6.1	Test purpose

To verify that the NR intra-frequency RRC re-establishment delay in FR1 without serving cell timing is within the specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.1B.

16.3.2.1.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.3.2.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.1.6.

16.3.2.1.6.4	Test description

16.3.2.1.6.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.1.3.4.1 with following exceptions:

-	Table 6.3.2.1.3.4.1-1 is replaced by Table 16.3.2.1.6.4.1-1.

-	Table 6.3.2.1.3.4.1-2 is replaced by Table 16.3.2.1.6.4.1-2.

-	Table 6.3.2.1.3.4.1-3 is replaced by Table 16.3.2.1.6.4.1-3

Table 16.3.2.1.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.1.6-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.1.6-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.6-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.6-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.1.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.3.2.1.6.4.1-3: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1,2,3,4 | Cell1 |  |
|  | Neighbour cells |  | 1,2,3,4 | Cell2 |  |
| Final condition | Active cell |  | 1,2,3,4 | Cell2 |  |
| RF Channel Number |  |  | 1,2,3,4 | 1 |  |
| Time offset between cells |  |  | 1,4 | 3 ms | Asynchronous cells |
|  |  |  | 2,3 | 3 s | Synchronous cells |
| N310 |  | - | 1,2,3,4 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1,2,3,4 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1,2,3,4 | 6000 | Radio link failure timer; |
| T311 |  | ms | 1,2,3,4 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1,2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1,2,4 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1,4 | SMTC.2 |  |
|  |  |  | 2,3 | SMTC.1 |  |
| DRX cycle length |  | s | 1,2,3,4 | OFF |  |
| PRACH configuration |  |  | 1,2,3,4 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1,2,3,4 | 5 |  |
| T2 |  | s | 1,2,3,4 | 6.24 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1B in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1B.5 in TS 38.133[6]) |
| T3 |  | s | 1,2,3,4 | 3 |  |

16.3.2.1.6.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.1.3.4.2.

16.3.2.1.6.4.3	Message contents

Same as the message contents given in 6.3.2.1.3.4.3 with the following exceptions:

-	Table 7.3.1-3 in 38.508-1 [14] is used with condition SMTC.2 for Test Configuration 16.3.2.1.6-4.

16.3.2.1.6.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.1.3.5 with following exceptions:

-	Table 6.3.2.1.3.5-1 is replaced by Table 16.3.2.1.6.5-1.

Table 16.3.2.1.6.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1,4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC |  | 1,4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC |  | 1,4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1,2,3,4 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial DL BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1,2,3,4 | SSB |  |  | SSB |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
| Note2 | dBm/SCS | 1,2,4 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  |  |  |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -91 |
| Io | dBm/9.36 MHz | 1,2,4 | -64.59 | -70.05 | -70.05 | -70.05 | -70.05 | -64.59 |
|  | dBm/18.36 MHz | 3 | -61.66 | -67.12 | -67.12 | -67.12 | -67.12 | -61.66 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### 16.3.2.2	Random access for RedCap

##### 16.3.2.2.0	Minimum conformance requirements

16.3.2.2.0.1	Minimum conformance requirements for Contention based random access

The 4-step and 2-step random access requirements described in clause 6.3.2.2.0.1 are applicable for TDD and FDD RedCap UEs.

The 4-step and 2-step RA requirements for contention based random access defined in clause 6.3.2.2.0.1 apply to HD-FDD UE with the following conditions:

-	The RedCap UE operating in HD-FDD mode is not expected to perform PRACH transmission on a PRACH resource of a cell if UE has not received at least one SSB associated with that PRACH resource during the last Tp period in the cell, where Tp=160 ms.

-	The RedCap UE operating in HD-FDD mode shall meet the PRACH requirements when performing PRACH transmission on a PRACH resource of a cell provided that the UE has received at least one SSB associated with that PRACH resource during the last Tp period before the PRACH transmission, where Tp=160 ms.

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2B.

16.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access

The 4-step and 2-step non-contention based random access requirements described in clause 6.3.2.2.0.1 are applicable for TDD and FDD RedCap UEs.

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.1B.

##### 16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE

16.3.2.2.1.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

16.3.2.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.2.1.

16.3.2.2.1.4	Test description

16.3.2.2.1.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.2.1.4.1 with following exceptions:

-	Table 6.3.2.2.1.4.1-1 is replaced by Table 16.3.2.2.1.4.1-1.

-	Table 6.3.2.2.1.4.1-2 is replaced by Table 16.3.2.2.1.4.1-2.

Table 16.3.2.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.2.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.3.2.2.1.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.2.1.4.2 with following exceptions:

-	Step 4 (Test 1) is replaced by following steps:

4.	Test 1: Correct behaviour when transmitting Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB+1dB.

16.3.2.2.1.4.3	Message contents

Same as the message contents given in 6.3.2.2.1.4.3.

16.3.2.2.1.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.2.1.5 with following exceptions:

-	Table 6.3.2.2.1.5-1 is replaced by Table 16.3.2.2.1.5-1.

-	The check on the power of the first preamble in Test 2, Test 3, Test 5 and Test 7 is replaced as follows:

-	The power of the first preamble shall be X dBm within the accuracy specified in Table 16.3.2.2.1.5-2, where

-	X = -23.55 for test configuration 16.3.2.2.1-1, 16.3.2.2.1-3 and 16.3.2.2.1-4,

-	X = -26.55 for test configuration 16.3.2.2.1-2.

Table 16.3.2.2.1.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 | As defined in A.3 and A.3A, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3 and A.3A |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3 and A.3A |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  | Config 1,4 |  | SR.1.1 FDD | As defined in A.1.1 |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 4.55 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB+1dB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | 4.55 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -93.45 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB+1dB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1,2,4 | dBm | -64.19/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3 |  | -64.27/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | PRACH.1 FR1 | As defined in A.7.1 |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Table 16.3.2.2.1.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 20.1 dB |

##### 16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE

16.3.2.2.2.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will also verify the requirements in 38.133 [6] Clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model

16.3.2.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.3.2.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.2.2.

16.3.2.2.2.4	Test description

16.3.2.2.2.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.2.1.4.1 with following exceptions:

-	Table 6.3.2.2.1.4.1-1 is replaced by Table 16.3.2.2.2.4.1-1.

-	Table 6.3.2.2.1.4.1-2 is replaced by Table 16.3.2.2.2.4.1-2.

Table 16.3.2.2.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.2.2.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 16.3.2.2.2.4.1-2: Initial conditions for Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.3.2.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.1.4.2.

16.3.2.2.2.4.3	Message contents

Same as the message contents given in clause 6.3.2.2.1.4.3.

16.3.2.2.2.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.1.4.2 with following exceptions:

-	Table 6.3.2.2.1.5-1 is replaced by Table 16.3.2.2.2.5-1.

Table 16.3.2.2.2.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 | As defined in A.3 and A.3A, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.10 and A.3.10B |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.10 and A.3.10B |
| Duplex Mode for Cell 1 |  | Config 1,4 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1. |
| PDSCH parameters Note 4 |  | Config 1,4 |  | SR.1.1 FDD | As defined in A.1.1. |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 | dBm/15kHz | -95 |  |
|  |  | Config 3 |  | -98 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 | dBm/15kHz | -115 |  |
|  |  | Config 3 |  | -118 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1,2,4 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3 |  | -65.38/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | PRACH.1 FR1 | As defined in A.7.1. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

##### 16.3.2.2.3	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE

16.3.2.2.3.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

16.3.2.2.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.2.3.

16.3.2.2.3.4	Test description

16.3.2.2.3.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.2.2.4.1 with following exceptions:

-	Table 6.3.2.2.2.4.1-1 is replaced by Table 16.3.2.2.3.4.1-1.

-	Table 6.3.2.2.2.4.1-2 is replaced by Table 16.3.2.2.3.4.1-2.

Table 16.3.2.2.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.2.3-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.3-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.3-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.3-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.2.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.3.2.2.3.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.2.2.4.2

16.3.2.2.3.4.3	Message contents

Same as the message contents given in 6.3.2.2.2.4.3.

16.3.2.2.3.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.2.2.5 with following exceptions:

-	Table 6.3.2.2.2.5-1 is replaced by Table 16.3.2.2.3.5-1.

-	The check on the power of the first preamble in Test 3 and Test 4 is replaced as follows:

-	The power of the first preamble shall be X dBm within the accuracy specified in Table 16.3.2.2.3.5-2, where

-	X = -23.55 for test configuration 16.3.2.2.3-1, 16.3.2.2.3-3 and 16.3.2.2.3-4,

-	X = -26.55 for test configuration 16.3.2.2.3-2.

Table 16.3.2.2.3.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 | SSB.1 FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3 |  | SSB.1 RedCap FR1 | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | 2 | Different from the definition in A.3.10B |
| SS/PBCH block index |  |  |  | 0,1 | 0,1 | Different from the definition in A.3.10B |
| CSI-RS Configuration |  | Config 1,2,4 |  | N/A | CSI-RS.1.1 FDD | As defined in A.1.4 |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD | FDD |  |
|  |  | Config 2,3 |  | TDD | TDD |  |
|  |  | Config 4 |  | HD-FDD | HD-FDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | OP.1 | As defined in A.2.1. |
| PDSCH parameters Note 4 |  | Config 1,4 |  | SR.1.1 FDD | SR.1.1 FDD | As defined in A.3.1.1 |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB | 0 | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 4.55 | 4.55 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB+1dB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3 |  | -101 | -101 |  |
|  |  |  | dB | 4.55 | 4.55 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -93.45 | -93.45 |  |
| SSB with index 1 |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB+1dB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3 |  | -101 | -101 |  |
|  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1,2,4 | dBm | -64.19/9.36MHz | -64.19/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3 |  | -64.27/18.36 MHz | -64.27/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | PRACH.2 FR1 | PRACH.3 FR1 | As defined in A.3.8B.2. |
| Propagation Condition |  |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

Table 16.3.2.2.3.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 20.1 dB |

##### 16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE

16.3.2.2.4.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will also verify the requirements in 38.133 [6] Clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model

16.3.2.2.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards. Additionally Test 2 is applicable to UE that supports CSI-RS based Random Access Preamble which requires UE to support csi-RSRP-AndRSRQ-MeasWithSSB or csi-RSRP-AndRSRQ-MeasWithoutSSB.

16.3.2.2.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.2.4.

16.3.2.2.4.4	Test description

16.3.2.2.4.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.2.2.4.1 with following exceptions:

-	Table 6.3.2.2.2.4.1-1 is replaced by Table 16.3.2.2.4.4.1-1.

-	Table 6.3.2.2.2.4.1-2 is replaced by Table 16.3.2.2.4.4.1-2.

Table 16.3.2.2.4.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.2.2.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.4-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.4-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 16.3.2.2.4.4.1-2: Initial conditions for non-contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.3.2.2.4.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.2.4.2.

16.3.2.2.4.4.3	Message contents

Same as the message contents given in clause 6.3.2.2.2.4.3.

16.3.2.2.4.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.2.4.2 with following exceptions:

-	Table 6.3.2.2.2.5-1 is replaced by Table 16.3.2.2.4.5-1.

Table 16.3.2.2.4.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 | SSB.1 FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3 |  | SSB.1 RedCap FR1 | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | 2 | Different from the definition in A.3.10B |
| SS/PBCH block index |  |  |  | 0,1 | 0,1 | Different from the definition in A.3.10B |
| CSI-RS Configuration |  | Config 1,2,4 |  | N/A | CSI-RS.1.1 FDD | As defined in A.1.4 |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD | FDD |  |
|  |  | Config 2,3 |  | TDD | TDD |  |
|  |  | Config 4 |  | HD-FDD | HD-FDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | OP.1 | As defined in A.2.1. |
| PDSCH parameters Note 4 |  | Config 1,4 |  | SR.1.1 FDD | SR.1.1 FDD | As defined in A.3.1.1 |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB | 0 | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 3 | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3 |  | -101 | -101 |  |
|  |  |  | dB | 3 | 3 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 | dBm/15kHz | -95 | -95 |  |
|  |  | Config 3 |  | -98 | -98 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 | -95 |  |
| SSB with index 1 |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3 |  | -101 | -101 |  |
|  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 | dBm/15kHz | -115 | -115 |  |
|  |  | Config 3 |  | -118 | -118 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1,2,4 | dBm | -65.3/9.36MHz | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3 |  | -65.38/18.36 MHz | -65.38/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | PRACH.2 FR1 | PRACH.3 FR1 | As defined in A.3.8B.2. |
| Propagation Condition |  |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

##### 16.3.2.2.5	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE

16.3.2.2.5.1	Test purpose

To verify that the behaviour of the 2-step RA type random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

16.3.2.2.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx and supporting 2-step RACH from Release 17 onwards.

16.3.2.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.2.5.

16.3.2.2.5.4	Test description

16.3.2.2.5.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.2.3.4.1 with following exceptions:

-	Table 6.3.2.2.3.4.1-1 is replaced by Table 16.3.2.2.5.4.1-1.

-	Table 6.3.2.2.3.4.1-2 is replaced by Table 16.3.2.2.5.4.1-2.

Table 16.3.2.2.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.2.5-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.5-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.5-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.5-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.2.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.3.2.2.5.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.2.3.4.2.

16.3.2.2.5.4.3	Message contents

Same as the message contents given in 6.3.2.2.3.4.3 with following exceptions:

Table 16.3.2.2.5.4.3-1: BWP-UplinkCommon

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| MsgA-ConfigCommonL-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA |  |  |
| msgA-PUSCH-Config-r16 | MsgA-PUSCH-Config |  |  |
| } |  |  |  |

16.3.2.2.5.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.2.3.5 with following exceptions:

-	Table 6.3.2.2.3.5-1 is replaced by Table 16.3.2.2.5.5-1.

-	The check on the power of the first preamble in Test 1, Test 2, Test 3 and Test 7 is replaced as follows:

-	The power of the first preamble shall be X dBm within the accuracy specified in Table 16.3.2.2.5.5-2, where

-	X = -23.55 for test configuration 16.3.2.2.5-1, 16.3.2.2.5-3 and 16.3.2.2.5-4,

-	X = -26.55 for test configuration 16.3.2.2.5-2.

Table 16.3.2.2.5.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3 |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  | Config 1,4 |  | SR.1.1 FDD | As defined in A.1.1 |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 4.55 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | 4.55 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -93.45 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1,2,4 | dBm | -64.19/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3 |  | -64.27/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  | MsgA.1 FR1 | As defined in A.7A.1 |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Table 16.3.2.2.5.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 20.1 dB |

##### 16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE

16.3.2.2.6.1	Test purpose

To verify that the behaviour of the 2-step RA type random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will also verify the requirements in 38.133 [6] Clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model

16.3.2.2.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx and supporting 2-step RACH from Release 17 onwards.

16.3.2.2.6.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.2.6.

16.3.2.2.6.4	Test description

16.3.2.2.6.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.2.3.4.1 with following exceptions:

-	Table 6.3.2.2.3.4.1-1 is replaced by Table 16.3.2.2.6.4.1-1.

-	Table 6.3.2.2.3.4.1-2 is replaced by Table 16.3.2.2.6.4.1-2.

Table 16.3.2.2.6.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.2.2.6-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.6-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.6-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.6-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 16.3.2.2.6.4.1-2: Initial conditions for Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.3.2.2.6.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.3.4.2.

16.3.2.2.6.4.3	Message contents

Same as the message contents given in clause 6.3.2.2.3.4.3 with following exceptions:

Table 16.3.2.2.6.4.3-1: BWP-UplinkCommon

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| MsgA-ConfigCommonL-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA |  |  |
| msgA-PUSCH-Config-r16 | MsgA-PUSCH-Config |  |  |
| } |  |  |  |

16.3.2.2.6.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.3.4.2 with following exceptions:

-	Table 6.3.2.2.3.5-1 is replaced by Table 16.3.2.2.6.5-1.

Table 16.3.2.2.6.5-1: General test parameters

| Parameter |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  |  | Config 1,2,4 |  | SSB.1 FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  |  | Config 3 |  | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  |  | 2 | Different from the definition in A.3 |
| SS/PBCH block index |  |  |  |  | 0,1 | Different from the definition in A.3 |
| Duplex Mode for Cell 1 |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2,3 |  | TDD |  |
|  |  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  |  | Config 1,4 |  | SR.1.1 FDD | As defined in A.1.1 |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| SSB with index 0 |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  |  | Config 3 |  | -101 |  |
|  |  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 |  | Config 1,2,4 | dBm/15kHz | -95 |  |
|  |  |  | Config 3 |  | -98 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  |  | Config 3 |  | -101 |  |
|  |  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 |  | dBm/15kHz | -115 |  |
|  |  | Config 3 |  |  | -118 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  | Config 1,2,4 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  |  | Config 3 |  | -65.38/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  |  | MsgA.1 FR1 | As defined in A.7A.1 |
| msgA-RSRP-ThresholdSSB |  |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

##### 16.3.2.2.7	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE

16.3.2.2.7.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

16.3.2.2.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx and 2-step RACH from Release 17 onwards.

16.3.2.2.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.2.7.

16.3.2.2.7.4	Test description

16.3.2.2.7.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.2.4.4.1 with following exceptions:

-	Table 6.3.2.2.4.4.1-1 is replaced by Table 16.3.2.2.7.4.1-1.

-	Table 6.3.2.2.4.4.1-2 is replaced by Table 16.3.2.2.7.4.1-2.

-	The check on the power of the first preamble in Test 1, Test 2 and Test 3 is replaced as follows:

-	The power of the first preamble shall be -23.55 dBm within the accuracy specified in Table 16.3.2.2.7.5-2.

Table 16.3.2.2.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.2.7-1 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.7-2 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.2.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.3.2.2.7.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.2.4.4.2.

16.3.2.2.7.4.3	Message contents

Same as the message contents given in 6.3.2.2.4.4.3

with following exceptions:

Table 16.3.2.2.7.4.3-1: BWP-UplinkCommon

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| MsgA-ConfigCommonL-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA |  |  |
| msgA-PUSCH-Config-r16 | MsgA-PUSCH-Config |  |  |
| } |  |  |  |

16.3.2.2.7.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.2.4.5 with following exceptions:

-	Table 6.3.2.2.4.5-1 is replaced by Table 16.3.2.2.7.5-1.

Table 16.3.2.2.7.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB.1 RedCap FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB.1 FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3 |
| Duplex Mode for Cell 1 |  | Config 1 |  | TDD |  |
|  |  | Config 2 |  | HD-FDD |  |
| TDD Configuration |  | Config 1 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  | TRS. 1.2 TDD |  |
|  |  | Config 2 |  | TRS. 1.1 FDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.2.1 TDD | As defined in A.1.1 |
|  |  | Config 2 |  | SR.1.1 FDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.2.1 TDD |  |
|  |  | Config 2 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.2.1 TDD |  |
|  |  | Config 2 |  | CCR.1.1 FDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB | 0 |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 4.55 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  | Config 2 |  | -98 |  |
|  |  |  | dB | 4.55 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | --93.45 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  | Config 2 |  | -98 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | --94.19/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | --64.27/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  | MsgA.2 FR1 | As defined in A.7A.1 |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Table 16.3.2.2.7.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 20.1 dB |

##### 16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE

16.3.2.2.8.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in 38.133 [6] Clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

16.3.2.2.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx and 2-step RACH from Release 17 onwards.

16.3.2.2.8.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.2.8.

16.3.2.2.8.4	Test description

16.3.2.2.8.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.2.4.4.1 with following exceptions:

-	Table 6.3.2.2.4.4.1-1 is replaced by Table 16.3.2.2.8.4.1-1.

-	Table 6.3.2.2.4.4.1-2 is replaced by Table 16.3.2.2.8.4.1-2.

Table 16.3.2.2.8.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.3.2.2.8-1 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.8-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 16.3.2.2.8.4.1-2: Initial conditions for Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.3.2.2.8.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.4.4.2.

16.3.2.2.8.4.3	Message contents

Same as the message contents given in clause 6.3.2.2.4.4.3 with following exceptions:

Table 16.3.2.2.8.4.3-1: BWP-UplinkCommon

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| MsgA-ConfigCommonL-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA |  |  |
| msgA-PUSCH-Config-r16 | MsgA-PUSCH-Config |  |  |
| } |  |  |  |

16.3.2.2.8.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.4.4.2 with following exceptions:

-	Table 6.3.2.2.4.5-1 is replaced by Table 16.3.2.2.8.5-1.

Table 16.3.2.2.8.5-1: General test parameters

| Parameter |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  |  | Config 1 |  | SSB.1 RedCap FR1 | As defined in A.3, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  |  | Config 2 |  | SSB.1 FR1 |  |
| Number of SSBs per SS-burst |  |  |  |  | 2 | Different from the definition in A.3 |
| SS/PBCH block index |  |  |  |  | 0,1 | Different from the definition in A.3 |
| Duplex Mode for Cell 1 |  |  | Config 1 |  | TDD |  |
|  |  |  | Config 2 |  | HD-FDD |  |
| TDD Configuration |  |  | Config 2 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  |  | Config 1 |  | TRS.1.2 TDD |  |
|  |  |  | Config 2 |  | TRS.1.1 FDD |  |
| OCNG Pattern Note 1 |  |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  |  | Config 1 |  | SR.2.1 TDD | As defined in A.1.1 |
|  |  |  | Config 2 |  | SR.1.1 FDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.2.1 TDD |  |
|  |  |  | Config 2 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.2.1 TDD |  |
|  |  |  | Config 2 |  | CCR.1.1 FDD |  |
| NR RF Channel Number |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| SSB with index 0 |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  | Config 2 |  | -98 |  |
|  |  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 | Config 1,2,4 |  | dBm/15kHz | -95 |  |
|  |  | Config 3 |  |  | -98 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  | Config 2 |  | -98 |  |
|  |  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | Config 1,2,4 | dBm/15kHz | -115 |  |
|  |  |  | Config 3 |  | -118 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  |  | Config 2 |  | -65.38/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  |  | MsgA.2 FR1 | As defined in A.7A.1 |
| msgA-RSRP-ThresholdSSB |  |  |  |  | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

#### 16.3.2.3	RRC connection release with redirection for RedCap

##### 16.3.2.3.0	Minimum conformance requirements

16.3.2.3.0.1	Minimum conformance requirements for FR1 RRC connection release with redirection

The requirements in clause 6.3.2.3.0.1 shall apply when RedCap UE is capable of 2 Rx.

When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.3.2.3.0.1 shall apply except that:

-	Tidentify-NR as specified in Table 16.3.2.3.0.1-1.

Table 16.3.2.3.0.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, [12] x Trs) |
| Note: If the UE has been provided with higher layer signalling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. |  |

The HD-FDD UE shall meet the RRC connection release with redirection requirements provided that SSB is available at the UE once every SMTC period during Tsearch.

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.3A.2.1.

16.3.2.3.0.2	Minimum conformance requirements for FR1 – E-UTRAN RRC connection release with redirection

The requirements in clause 6.3.2.3.0.2 shall apply.

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.3A.2.2.

##### 16.3.2.3.1	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE

16.3.2.3.1.1	Test purpose

To verify RRC connection release with redirection from NR to NR requirements specified in 38.133 [6] clause 6.2.3A.2.1.

16.3.2.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.2.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.3.1.

16.3.2.3.1.4	Test description

16.3.2.3.1.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.3.1.4.1 with following exceptions:

-	Table 6.3.2.3.1.4.1-1 is replaced by Table 16.3.2.3.1.4.1-1.

-	Table 6.3.2.3.1.4.1-2 is replaced by Table 16.3.2.3.1.4.1-2.

-	Table 6.3.2.3.1.4.1-3 is replaced by Table 16.3.2.3.1.4.1-3.

Table 16.3.2.3.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.3.1-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.3.1-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.3.1-3 | Source cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.3.1-4 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
|  | Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.2.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.3.2.3.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

16.3.2.3.1.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.3.1.4.2.

16.3.2.3.1.4.3	Message contents

Same as the message contents given in 6.3.2.3.1.4.3 with following exceptions.

-	Table 6.3.2.3.1.4.3-1 is replaced by Table 16.3.2.3.1.4.3-1.

Table 16.3.2.3.1.4.3-1: RRCRelease (Test procedure, Step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| nr SEQUENCE { |  |  |  |
| carrierFreq | ARFCN-ValueNR of Cell 2 |  |  |
| ssbSubcarrierSpacing | kHz15 |  | Config 1,2,4 |
|  | kHz30 |  | Config 3 |
| smtc SEQUENCE { |  |  |  |
| duration | sf1 |  | SMTC.1 |
|  | sf5 |  | SMTC.2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.3.2.3.1.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.3.1.5 with following exceptions:

-	Table 6.3.2.3.1.5-1 is replaced by Table 16.3.2.3.1.5-1.

Table 16.3.2.3.1.5-1: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2, 3 |  | TDD |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |
| SSB Configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |
| CSI-RS for tracking | Config 1,4 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| BWP BW | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference | Config 1 |  | SR.1.1 FDD |  |  |  |
| measurement channel | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
|  | Config 4 |  | SR.1.1 FDD |  |  |  |
| CORESET Reference | Config 1 |  | CR.1.1 FDD |  |  |  |
| Channel | Config 2 |  | CR.1.1 FDD |  |  |  |
|  | Config 3 |  | CR.2.1 FDD |  |  |  |
|  | Config 4 |  | CR.1.1 FDD |  |  |  |
| OCNG Patterns |  |  | OG.1 |  |  |  |
| SMTC configuration | Config 1,2,4 |  | SMTC.1 FR1 |  |  |  |
|  | Config 3 |  | SMTC.2 FR1 |  |  |  |
| PDSCH/PDCCH | Config 1,2,4 | kHz | 15 kHz |  |  |  |
| subcarrier spacing | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH | Config 1,2,4 | kHz | 15 kHz |  |  |  |
| subcarrier spacing | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2,4 | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  | -95 |  |  |  |
|  |  | dB | 4 | 4 | -infinity | 4 |
|  |  | dB | 4 | 4 | -infinity | 4 |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -64.59 |
|  | Config 3 | dBm/ 18.36MHz | -61.66 | -61.66 | -67.12 | -61.66 |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### 16.3.2.3.2	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE

16.3.2.3.2.1	Test purpose

To verify RRC connection release with redirection from NR to NR requirements specified in 38.133 [6] clause 6.2.3A.2.1.

16.3.2.3.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.3.2.3.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.3.2.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.16.3.2.3.2.

16.3.2.3.2.4	Test description

16.3.2.3.2.4.1	Initial conditions

Same as the initial conditions given in 6.3.2.3.1.4.1 with following exceptions:

-	Table 6.3.2.3.1.4.1-1 is replaced by Table 16.3.2.3.2.4.1-1.

-	Table 6.3.2.3.1.4.1-2 is replaced by Table 16.3.2.3.2.4.1-2.

-	Table 6.3.2.3.1.4.1-3 is replaced by Table 16.3.2.3.1.4.1-3.

Table 16.3.2.3.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.3.2-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.3.2-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.3.2-3 | Source cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.3.2-4 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
|  | Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.2.3.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.3.2.3.2.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

16.3.2.3.2.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.3.1.4.2.

16.3.2.3.2.4.3	Message contents

Same as the message contents given in 6.3.2.3.1.4.3 with following exceptions.

-	Table 6.3.2.3.1.4.2-1 is replaced by Table 16.3.2.3.2.4.3-1.

Table 16.3.2.3.2.4.3-1: RRCRelease (Test procedure, Step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| nr SEQUENCE { |  |  |  |
| carrierFreq | ARFCN-ValueNR of Cell 2 |  |  |
| ssbSubcarrierSpacing | kHz15 |  | Config 1,2,4 |
|  | kHz30 |  | Config 3 |
| smtc SEQUENCE { |  |  |  |
| duration | sf1 |  | SMTC.1 |
|  | sf5 |  | SMTC.2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.3.2.3.2.5	Test requirement

Same as the test requirements given in clause 6.3.2.3.1.5 with following exceptions:

-	Table 6.3.2.3.1.5-1 is replaced by Table 16.3.2.3.2.5-1.

Table 16.3.2.3.2.5-1: Cell specific test parameter

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2, 3 |  | TDD |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |
| SSB Configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |
| CSI-RS for tracking | Config 1,4 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| BWP BW | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference | Config 1,4 |  | SR.1.1 FDD |  |  |  |
| measurement channel | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference | Config 1,4 |  | CR.1.1 FDD |  |  |  |
| Channel | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC configuration | Config 1,2,4 |  | SMTC.1 FR1 |  |  |  |
|  | Config 3 |  | SMTC.2 FR1 |  |  |  |
| PDSCH/PDCCH | Config 1,2,4 | kHz | 15 kHz |  |  |  |
| subcarrier spacing | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH | Config 1,2,4 | kHz | 15 kHz |  |  |  |
| subcarrier spacing | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2,4 | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  | -95 |  |  |  |
|  |  | dB | 4 | 4 | -infinity | 4 |
|  |  | dB | 4 | 4 | -infinity | 4 |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -64.59 |
|  | Config 3 | dBm/ 18.36MHz | -61.66 | -61.66 | -67.12 | -61.66 |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### 16.3.2.3.3	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 1 Rx UE

16.3.2.3.3.1	Test purpose

To verify RRC connection release with redirection from NR to E-UTRAN requirements specified in 38.133 [4] clause 6.2.3A.2.2.

16.3.2.3.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.2.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.3.3.

16.3.2.3.3.4	Test description

16.3.2.3.3.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.3.2.4.1 with following exceptions:

-	Table 6.3.2.3.2.4.1-1 is replaced by Table 16.3.2.3.3.4.1-1.

-	Table 6.3.2.3.2.4.1-2 is replaced by Table 16.3.2.3.3.4.1-2.

-	Table 6.3.2.3.2.4.1-3 is replaced by Table 16.3.2.3.3.4.1-3.

Table 16.3.2.3.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.3.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.3.2.3.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.2.3.3-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.2.3.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.3.2.3.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.2.3.3-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.2.3.3-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 16.3.2.3.3-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.2.3.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.3.2.3.3.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

16.3.2.3.3.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.3.2.4.2.

16.3.2.3.3.4.3	Message contents

Same as the message contents given in 6.3.2.3.2.4.3.

16.3.2.3.3.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.3.2.5 with following exceptions:

-	Table 6.3.2.3.2.5-1 is replaced by Table 16.3.2.3.3.5-1.

-	Table 6.3.2.3.2.5-2 is replaced by Table 16.3.2.3.3.5-2.

Table 16.3.2.3.3.5-1: Cell specific test parameters for NR Cell (Cell 1)

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1,4 |  | FDD |  |
|  | Config 2,3,5,6 |  | TDD |  |
|  | Config 7,8 |  | HD-FDD |  |
| SSB Configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |  |
|  | Config 3,6 |  | SSB.1 RedCap FR1 |  |
| CSI-RS for tracking | Config 1,4,7,8 |  | TRS.1.1 FDD |  |
|  | Config 2,5 |  | TRS.1.1 TDD |  |
|  | Config 3,6 |  | TRS.1.2 TDD |  |
| TDD configuration | Config 1,4,7,8 |  | Not Applicable |  |
|  | Config 2,5 |  | TDDConf.1.1 |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 20: NRB,c = 51 |  |
| BWP BW | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 20: NRB,c = 51 |  |
| DRX Cycle |  | ms | Not Applicable |  |
| PDSCH Reference | Config 1,4,7,8 |  | SR.1.1 FDD |  |
| measurement channel | Config 2,5 |  | SR.1.1 TDD |  |
|  | Config 3,6 |  | SR.2.1 TDD |  |
| CORESET Reference | Config 1,4,7,8 |  | CR.1.1 FDD |  |
| Channel | Config 2,5 |  | CR.1.1 TDD |  |
|  | Config 3,6 |  | CR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1,2,4,5,7,8 |  | SMTC.1 FR1 |  |
|  | Config 3,6 |  | SMTC.2 FR1 |  |
| PDSCH/PDCCH | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
| subcarrier spacing | Config 3,6 |  | 30 kHz |  |
| PUCCH/PUSCH | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
| subcarrier spacing | Config 3,6 |  | 30 kHz |  |
| PRACH configuration |  |  | PRACH.1 FR1 |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4,5,7,8 | dBm/SCS | -98 |  |
|  | Config 3,6 |  | -95 |  |
|  |  | dB | 4 | 4 |
|  |  | dB | 4 | 4 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/ 9.36MHz | -64.59 | -64.59 |
|  | Config 3,6 | dBm/ 18.36MHz | -61.66 | -61.66 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 16.3.2.3.3.5-2: Cell specific test parameters for E-UTRAN Cell (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |
|  |  | 4,5,6,8 | TDD |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1,2,3,7 | 4 |  |
|  |  | 4,5,6,8 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,7 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement |  | 1,2,3,7 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
| ChannelNote3 |  | 4,5,6,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1,2,3,7 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 4 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 4 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -94 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -94 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -70.22 | -64.76 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [24]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

##### 16.3.2.3.4	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE

16.3.2.3.4.1	Test purpose

To verify RRC connection release with redirection from NR to E-UTRAN requirements specified in 38.133 [4] clause 6.2.3A.2.2.

16.3.2.3.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.3.2.3.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.2.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.3.4.

16.3.2.3.4.4	Test description

16.3.2.3.4.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.3.2.4.1 with following exceptions:

-	Table 6.3.2.3.2.4.1-1 is replaced by Table 16.3.2.3.4.4.1-1.

-	Table 6.3.2.3.2.4.1-2 is replaced by Table 16.3.2.3.4.4.1-2.

-	Table 6.3.2.3.2.4.1-3 is replaced by Table 16.3.2.3.4.4.1-3.

Table 16.3.2.3.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.3.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.3.2.3.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.2.3.4-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.2.3.4-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.3.2.3.4-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.2.3.4-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.2.3.4-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 16.3.2.3.4-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.2.3.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.3.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.3.2.3.4.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

16.3.2.3.4.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.3.2.4.2.

16.3.2.3.4.4.3	Message contents

Same as the message contents given in 6.3.2.3.2.4.3.

16.3.2.3.4.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.3.2.5 with following exceptions:

-	Table 6.3.2.3.2.5-1 is replaced by Table 16.3.2.3.4.5-1.

-	Table 6.3.2.3.2.5-2 is replaced by Table 16.3.2.3.4.5-2.

Table 16.3.2.3.4.5-1: Cell specific test parameters for NR Cell (Cell 1)

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1,4 |  | FDD |  |
|  | Config 2,3,5,6 |  | TDD |  |
|  | Config 7,8 |  | HD-FDD |  |
| SSB Configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |  |
|  | Config 3,6 |  | SSB.1 RedCap FR1 |  |
| CSI-RS for tracking | Config 1,4,7,8 |  | TRS.1.1 FDD |  |
|  | Config 2,5 |  | TRS.1.1 TDD |  |
|  | Config 3,6 |  | TRS.1.2 TDD |  |
| TDD configuration | Config 1,4,7,8 |  | Not Applicable |  |
|  | Config 2,5 |  | TDDConf.1.1 |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 20: NRB,c = 51 |  |
| BWP BW | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 20: NRB,c = 51 |  |
| DRX Cycle |  | ms | Not Applicable |  |
| PDSCH Reference | Config 1,4,7,8 |  | SR.1.1 FDD |  |
| measurement channel | Config 2,5 |  | SR.1.1 TDD |  |
|  | Config 3,6 |  | SR.2.1 TDD |  |
| CORESET Reference | Config 1,4,7,8 |  | CR.1.1 FDD |  |
| Channel | Config 2,5 |  | CR.1.1 TDD |  |
|  | Config 3,6 |  | CR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1,2,4,5,7,8 |  | SMTC.1 FR1 |  |
|  | Config 3,6 |  | SMTC.2 FR1 |  |
| PDSCH/PDCCH | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
| subcarrier spacing | Config 3,6 |  | 30 kHz |  |
| PUCCH/PUSCH | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
| subcarrier spacing | Config 3,6 |  | 30 kHz |  |
| PRACH configuration |  |  | PRACH.1 FR1 |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4,5,7,8 | dBm/SCS | -98 |  |
|  | Config 3,6 |  | -95 |  |
|  |  | dB | 4 | 4 |
|  |  | dB | 4 | 4 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/ 9.36MHz | -64.59 | -64.59 |
|  | Config 3,6 | dBm/ 18.36MHz | -61.66 | -61.66 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 16.3.2.3.4.5-2: Cell specific test parameters for E-UTRAN Cell (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |
|  |  | 4,5,6,8 | TDD |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1,2,3,7 | 4 |  |
|  |  | 4,5,6,8 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,7 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement |  | 1,2,3,7 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
| ChannelNote3 |  | 4,5,6,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1,2,3,7 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 4 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 4 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -94 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -94 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -70.22 | -64.76 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [24]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

## 16.4	Timing for RedCap

### 16.4.1	UE transmit timing for RedCap

#### 16.4.1.0	Minimum conformance requirements

##### 16.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 16.4.1.0.1-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE during the last 160 ms on the condition that:

the SSB is within the UE’s active BWP, or

the SSB is not within the UE’s active BWP, and the measurement gap is configured

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at UE antenna. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in 38.133 [6] clause 7.3A was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table 16.4.1.0.1-2.

Table 16.4.1.0.1-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 16.4.1.0.1-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 0 (Note 1) |
| FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. Note 2: Void |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in 38.133 [6] clause 7.3A is applied.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 16.4.1.0.1-3.

Table 16.4.1.0.1-3: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The normative reference for this requirement is TS.38.133 [6] clause 7.1A.

#### 16.4.1.1	NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE

16.4.1.1.1	Test purpose

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in 38.133 [6] clause 7.1A.2.

16.4.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.4.1.0.1.

The normative reference for this requirement is TS.38.133 [6] clause A.16.4.1.1

16.4.1.1.4	Test Description

16.4.1.1.4.1	Initial Conditions

Same as the initial conditions given in clause 6.4.1.1.4.1 with following exceptions:

-	Table 6.4.1.1.4.1-1 is replaced by Table 16.4.1.1.4.1-1.

-	Table 6.4.1.1.4.1-2 is replaced by Table 16.4.1.1.4.1-2

Table 16.4.1.1.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 16.4.1.1-1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 16.4.1.1-2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 16.4.1.1-3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz |
| 16.4.1.1-4 | NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.4.1.1.4.1-2: Initial conditions for NR SA FR1 transmit timing accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.4.1.1.4.2	Test procedure

Same as the test procedure given in clause 6.4.1.1.4.2.

16.4.1.1.4.3	Message contents

Same as the message contents given in clause 6.4.1.1.4.3.

16.4.1.1.5	Test Requirements

Same as the test Requirements given in clause 6.4.1.1.5 with following exceptions:

-	Table 6.4.1.1.5-1 is replaced by Table 16.4.1.1.5-1.

Table 16.4.1.1.5-1: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 | Test2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | Freq1 | Freq1 |
| TDD configuration |  | 1,4 | Not Applicable |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz | 1,2,4 | 10: NRB,c = 52 |  |
|  |  | 3 | 20: NRB,c = 51 |  |
| Initial BWP Configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  | 1,2,3,4 | DLBWP.1.1 ULBWP.1.1 |  |
| DRX Cycle | ms | 1,2,3,4 | N/A | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1,4 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1,4 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |
| Channel |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC Configuration |  | 1,2,4 | SMTC.1 |  |
|  |  | 3 | SMTC.2 |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 | -98 |
| Note2 | dBm/SCS | 1,2,4 | -98 | -98 |
|  |  | 3 | -95 | -95 |
|  |  | 1,2,3,4 | 3.3 | 3.3 |
|  |  | 1,2,3,4 | 3.3 Note 7 | 3.3 Note 7 |
| SS-RSRPNote3 | dBm/SCS | 1,2,4 | -94.7 | -94.7 |
|  |  | 3 | -91.7 | -91.7 |
| IoNote3 | dBm/9.36MHz | 1,2,4 | -65.08 | -65.08 |
|  | dBm/18.36MHz | 3 | -62.16 | -62.16 |
| Propagation condition |  | 1,2,3,4 | AWGN |  |
| SRS Config |  | 1,2,4 | SRSConf.1Note6 | SRSConf.3Note6 |
|  |  | 3 | SRSConf.1Note6 | SRSConf.2Note6 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRX related parameters are given in Table A.5-1 Note 6: SRS configs are given in Table 16.4.1.1.5-2 Note 7: With the test tolerance given in Annex F. |  |  |  |  |

Table 16.4.1.1.5-2: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 12 for test configuration 1,2, 3 | 12 | 12 |  |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 5 | sl320, 3 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

#### 16.4.1.2	NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE

16.4.1.2.1	Test purpose

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in 38.133 [6] clause 7.1A.2.

16.4.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.4.1.0.1.

The normative reference for this requirement is TS.38.133 [6] clause A.16.4.1.2

16.4.1.2.4	Test Description

16.4.1.2.4.1	Initial Conditions

Same as the initial conditions given in clause 6.4.1.1.4.1 with following exceptions:

-	Table 6.4.1.1.4.1-1 is replaced by Table 16.4.1.2.4.1-1.

-	Table 6.4.1.1.4.1-2 is replaced by Table 16.4.1.2.4.1-2

Table 16.4.1.2.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 16.4.1.2-1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 16.4.1.2-2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 16.4.1.2-3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz |
| 16.4.1.2-4 | NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.4.1.2.4.1-2: Initial conditions for NR SA FR1 transmit timing accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without LTE link |  |  |

16.4.1.2.4.2	Test procedure

Same as the test procedure given in clause 6.4.1.1.4.2.

16.4.1.2.4.3	Message contents

Same as the message contents given in clause 6.4.1.1.4.3.

16.4.1.2.5	Test Requirements

Same as the test Requirements given in clause 6.4.1.1.5 with following exceptions:

-	Table 6.4.1.1.5-1 is replaced by Table 16.4.1.2.5-1.

Table 16.4.1.2.5-1: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 | Test2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | Freq1 | Freq1 |
| TDD configuration |  | 1,4 | Not Applicable |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz | 1,2,4 | 10: NRB,c = 52 |  |
|  |  | 3 | 20: NRB,c = 51 |  |
| Initial BWP Configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  | 1,2,3,4 | DLBWP.1.1 ULBWP.1.1 |  |
| DRX Cycle | ms | 1,2,3,4 | N/A | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1,4 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1,4 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |
| Channel |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC Configuration |  | 1,2,4 | SMTC.1 |  |
|  |  | 3 | SMTC.2 |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 | -98 |
| Note2 | dBm/SCS | 1,2,4 | -98 | -98 |
|  |  | 3 | -95 | -95 |
|  |  | 1,2,3,4 | 3.3 | 3.3 |
|  |  | 1,2,3,4 | 3.3 Note 7 | 3.3 Note 7 |
| SS-RSRPNote3 | dBm/SCS | 1,2,4 | -94.7 | -94.7 |
|  |  | 3 | -91.7 | -91.7 |
| IoNote3 | dBm/9.36MHz | 1,2,4 | -65.08 | -65.08 |
|  | dBm/18.36MHz | 3 | -62.16 | -62.16 |
| Propagation condition |  | 1,2,3,4 | AWGN |  |
| SRS Config |  | 1,2,4 | SRSConf.1Note6 | SRSConf.3Note6 |
|  |  | 3 | SRSConf.1Note6 | SRSConf.2Note6 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRX related parameters are given in Table A.5-1 Note 6: SRS configs are given in Table 16.4.1.1.5-2 Note 7: With the test tolerance given in Annex F. |  |  |  |  |

### 16.4.2

### 16.4.3	Timing advance for RedCap

#### 16.4.3.0	Minimum conformance requirement

The timing advance is initiated from gNB to UE configured with only PCell, with MAC message that implies the adjustment of the timing advance, as defined in TS 38.321 [12] clause 5.2.

UE shall adjust the timing of its uplink transmission at time slot n+k+1 for a timing advance command received in time slot n, and the value of k is defined in TS 38.213 [8] clause 4.2. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 16.4.3.0-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS 38.213 [8].

Table 16.4.3.0-1: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 |
| --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc |

#### 16.4.3.1	NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE

16.4.3.1.1	Test purpose

To verify UE timing advance adjustment delay and accuracy requirement defined in TS 38.133 [6] clause 7.3A.

16.4.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.4.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.4.3.0.

The normative reference for this requirement is TS 38.133 [6] clause A.16.4.3.1.

16.4.3.1.4	Test description

16.4.3.1.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.4.3.1.4.1 with following exceptions:

-	Table 6.4.3.1.4.1-1 is replaced by Table 16.4.3.1.4.1-1.

-	Table 6.4.3.1.4.1-2 is replaced by Table 16.4.3.1.4.1-2.

-	Table 6.4.3.1.4.1-3 is replaced by Table 16.4.3.1.4.1-3.

Table 16.4.3.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.4.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.4.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.4.3.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.4.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.4.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.4.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without LTE link |  |  |

Table 16.4.3.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.8.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.8.1-2 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.8.2-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.8.2-2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For SCS = 15kHz : NTA_new = NTA_old + 8192*Tc For SCS = 30kHz : NTA_new = NTA_old + 4096*Tc (based on equation in TS 38.213 [8] clause 4.2) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

16.4.3.1.4.2	Test Procedure

Same as the test procedure given in sub-clause 6.4.3.1.4.2.

16.4.3.1.4.3	Message Contents

Same as the message contents specified in sub-clause 6.4.3.1.4.3.

16.4.3.1.5	Test Requirement

Same as the test requirements given in sub-clause 6.4.3.1.5 with following exceptions:

-	Table 6.4.3.1.5-1 is replaced by Table 16.4.3.1.5-1.

-	Table 6.4.3.1.5-2 is replaced by Table 16.4.3.1.5-2.

-	Table 6.4.3.1.5-3 is replaced by Table 16.4.3.1.5-3.

Table 16.4.3.1.5-1: Cell specific test parameters

| Parameter |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2,3 |  | TDD |  |
|  | Config 4 |  | HD-FDD |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |
| BWP BW | Config 1,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |
| DRX Cycle |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1,2,4 |  | SMTC.1 FR1 |  |
|  | Config 3 |  | SMTC.2 FR1 |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 kHz |  |
|  | Config 3 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4 | kHz | 15 kHz |  |
|  | Config 3 |  | 30 kHz |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4 | dBm/SCS | -98 |  |
|  | Config 3 |  | -95 |  |
|  |  | dB | 3 |  |
|  |  | dB | 3 |  |
| IoNote3 | Config 1,2,4 | dBm/9.36MHz | -67.57 |  |
|  | Config 3 | dBm/18.36MHz | -65.76 |  |
| Propagation condition |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 16.4.3.1.5-2: Sounding Reference Symbol Configuration

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS |  | 12 | Frequency hopping is disabled |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | Config 1,2,4 | sl5=2 | Once every 5 slots |
|  | Config 3 | sl5=4 |  |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols |  | n1 |  |
| repetitionFactor |  | n1 |  |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331. |  |  |  |

Table 16.4.3.1.5-3: UE Timing Advance adjustment accuracy

| Sub Carrier Spacing, SCS kHz | 15 | 30 |
| --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±344 Tc | ±344 Tc |

#### 16.4.3.2	NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE

16.4.3.2.1	Test purpose

To verify UE timing advance adjustment delay and accuracy requirement defined in TS 38.133 [6] clause 7.3A.

16.4.3.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.4.3.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.4.3.0.

The normative reference for this requirement is TS 38.133 [6] clause A.16.4.3.2.

16.4.3.2.4	Test description

16.4.3.2.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.4.3.1.4.1 with following exceptions:

-	Table 6.4.3.1.4.1-1 is replaced by Table 16.4.3.2.4.1-1.

-	Table 6.4.3.1.4.1-2 is replaced by Table 16.4.3.2.4.1-2.

-	Table 6.4.3.1.4.1-3 is replaced by Table 16.4.3.2.4.1-3.

Table 16.4.3.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.4.3.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.4.3.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.4.3.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.4.3.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.4.3.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.4.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.1 |  |
| Exceptions to connection diagram | Without LTE link |  |  |

Table 16.4.3.2.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.8.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.8.1-2 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.8.2-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.8.2-2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For SCS = 15kHz : NTA_new = NTA_old + 8192*Tc For SCS = 30kHz : NTA_new = NTA_old + 4096*Tc (based on equation in TS38.213 section 4.2) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

16.4.3.2.4.2	Test Procedure

Same as the test procedure given in sub-clause 6.4.3.1.4.2.

16.4.3.2.4.3	Message Contents

Same as the message contents specified in sub-clause 6.4.3.1.4.3.

16.4.3.2.5	Test Requirement

Same as the test requirements given in sub-clause 6.4.3.1.5 with following exceptions:

-	Table 6.4.3.1.5-1 is replaced by Table 16.4.3.2.5-1.

-	Table 6.4.3.1.5-2 is replaced by Table 16.4.3.2.5-2.

-	Table 6.4.3.1.5-3 is replaced by Table 16.4.3.2.5-3.

Table 16.4.3.2.5-1: Cell specific test parameters

| Parameter |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2,3 |  | TDD |  |
|  | Config 4 |  | HD-FDD |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |
| BWP BW | Config 1,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |
| DRX Cycle |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1,2,4 |  | SMTC.1 FR1 |  |
|  | Config 3 |  | SMTC.2 FR1 |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 kHz |  |
|  | Config 3 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4 | kHz | 15 kHz |  |
|  | Config 3 |  | 30 kHz |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4 | dBm/SCS | -98 |  |
|  | Config 3 |  | -95 |  |
|  |  | dB | 3 |  |
|  |  | dB | 3 |  |
| IoNote3 | Config 1,2,4 | dBm/9.36MHz | -67.57 |  |
|  | Config 3 | dBm/18.36MHz | -65.76 |  |
| Propagation condition |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 16.4.3.2.5-2: Sounding Reference Symbol Configuration

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS |  | 12 | Frequency hopping is disabled |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | Config 1,2,4 | sl5=2 | Once every 5 slots |
|  | Config 3 | sl5=4 |  |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols |  | n1 |  |
| repetitionFactor |  | n1 |  |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331. |  |  |  |

Table 16.4.3.2.5-3: UE Timing Advance adjustment accuracy

| Sub Carrier Spacing, SCS kHz | 15 | 30 |
| --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±344 Tc | ±344 Tc |

## 16.5	Signalling characteristics for RedCap

### 16.5.1	Radio link monitoring for RedCap

#### 16.5.1.0	Minimum requirements

##### 16.5.1.0.1	General

The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell as specified in TS 38.213 [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP. The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified.

On each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell.

The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate (BLERout) as defined in Table 16.5.1.0.1-1. For SSB based radio link monitoring, Qout_SSB,RedCap is derived based on the hypothetical PDCCH transmission parameters listed in Table 16.5.1.0.1-2. For CSI-RS based radio link monitoring, Qout_CSI-RS,RedCap is derived based on the hypothetical PDCCH transmission parameters listed in Table 16.5.1.0.1-4.

The threshold Qin is defined as the level at which the downlink radio link quality can be significantly more reliably received than at Qout and shall correspond to the in-sync block error rate (BLERin) as defined in Table 16.5.1.0.1-1. For SSB based radio link monitoring, Qin_SSB,RedCap is derived based on the hypothetical PDCCH transmission parameters listed in Table 16.5.1.0.1-3. For CSI-RS based radio link monitoring, Qin_CSI-RS,RedCap is derived based on the hypothetical PDCCH transmission parameters listed in Table 16.5.1.0.1-5.

The out-of-sync block error rate (BLERout) and in-sync block error rate (BLERin) are determined from the network configuration via parameter rlmInSyncOutOfSyncThreshold signalled by higher layers. When UE is not configured with RLM-IS-OOS-thresholdConfig from the network, UE determines out-of-sync and in-sync block error rates from Configuration #0 in Table 16.5.1.0.1-1 as default. All requirements here are applicable for BLER Configuration #0 in Table 16.5.1.0.1-1.

Table 16.5.1.0.1-1: Out-of-sync and in-sync block error rates

| Configuration | BLERout | BLERin |
| --- | --- | --- |
| 0 | 10% | 2% |

UE shall be able to monitor up to NRLM RLM-RS resources of the same or different types in each corresponding carrier frequency range, depending on a maximum number  of SSBs per half frame according to TS 38.213 [3], where NRLM is specified in Table 16.5.1.0.1-6 according TS 38.213 [3], and meet the requirements as specified in 38.133 [6] clause 8.1B. UE is not required to meet the requirements in 38.133 [6] clause 8.1B if RLM-RS is not configured and no TCI state for PDCCH is activated.

Table 16.5.1.0.1-2: PDCCH transmission parameters for out-of-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16; for RedCap UE with 1Rx branch. 8; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 48; for RedCap UE with 1Rx branch. 24; for RedCap UE with 2Rx branches. |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note: SCS = 60KHz is not applicable for FR1. |  |

Table 16.5.1.0.1-3: PDCCH transmission parameters for in-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8; for RedCap UE with 1Rx branch. 4; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 48; for RedCap UE with 1Rx branch. 24; for RedCap UE with 2Rx branches. |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note: SCS = 60KHz is not applicable for FR1. |  |

Table 16.5.1.0.1-4: PDCCH transmission parameters for out-of-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16; for RedCap UE with 1Rx branch. 8; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 4dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note: SCS = 60KHz is not applicable for FR1. |  |

Table 16.5.1.0.1-5: PDCCH transmission parameters for in-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8; for RedCap UE with 1Rx branch. 4; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note: SCS = 60KHz is not applicable for FR1. |  |

Table 16.5.1.0.1-6: Maximum number of RLM-RS resources XRLM-RS

| Carrier frequency range of PCell | Lmax | Maximum number of RLM-RS resources, NRLM |
| --- | --- | --- |
| FR1, ≤ 3 GHzNote | 4 | 2 |
| FR1, > 3 GHzNote | 8 | 4 |
| FR2 | 64 | 8 |
| NOTE: For unpaired spectrum operation with Case C - 30 kHz SCS, 3GHz is replaced by 1.88GHz, as specified in clause 4.1 in TS 38.213 [3]. |  |  |

When the UE transitions between RLM CD-SSB resource and RLM NCD-SSB resource during one evaluation period, the UE is allowed to restart the ongoing evaluation using the second SSB type after BWP switching. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each RLM-RS resource.

The requirements in 38.133 [6] clause 8.1.4 shall also apply except the scenarios described above.

The requirements in 38.133 [6] clause 8.1.5 shall apply.

When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, Layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers. A Layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [13].

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin,RedCap, layer 1 of the UE shall send an in-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [2].

The out-of-sync and in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 in TS 38.213 [8]. Two successive indications from layer 1 shall be separated by at least TIndication_interval,RedCap.

When DRX is not used for RedCap UEs, TIndication_interval,RedCap is max(10ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 38.133 [6] 8.1B.2 if the RLM-RS resource is SSB,  or TCSI-RS specified in 38.133 [6] clause 8.1B.3 if the RLM-RS resource is CSI-RS.

In case DRX is used for RedCap UEs, TIndication_interval,RedCap is max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms. Upon start of T310 timer as specified in TS 38.331 [2], the UE shall monitor the configured RLM-RS resources for recovery using the evaluation period and layer 1 indication interval corresponding to the no DRX mode until the expiry or stop of T310 timer.

When DRX is not used for HD-FDD RedCap UEs, TIndication_interval,RedCap is max(10ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in 38.133 [6] clause 8.1B.2 if the RLM-RS resource is SSB, or TCSI-RS specified in 38.133 [6] clause 8.1B.3 if the RLM-RS resource is CSI-RS, under the following condition

-	For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period.

In case DRX is used for HD-FDD RedCap UEs, TIndication_interval,RedCap is max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms, under the following condition

-	For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period.

The requirements in 38.133 [6] clause 8.1.7 shall apply

The normative reference for this requirement is TS 38.133 [6] clauses 8.1B.1, 8.1B.2, 8.1B.3, 8.1B.4, 8.1B.5, 8.1B.6 and 8.1B.7.

##### 16.5.1.0.2	Minimum conformance requirements for SSB-based RLM

The requirements in this clause apply for each SSB based RLM-RS resource configured for PCell, provided that the SSB configured for RLM is actually transmitted within UE active DL BWP during the entire evaluation period specified in this clause.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB,RedCap [ms] period becomes worse than the threshold Qout_SSB,RedCap within TEvaluate_out_SSB,RedCap [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,RedCap [ms] period becomes better than the threshold Qin_SSB,RedCap within TEvaluate_in_SSB,RedCap [ms] evaluation period.

TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap are defined in Table 16.5.1.0.2-1 and Table 16.5.1.0.2-2 for FR1 for UE with 2Rx RedCap and 1Rx RedCap, respectively.

Table 16.5.1.0.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 16.5.1.0.2-2: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(20 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320ms | Max(400, Ceil(30 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(20 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

For FR1,

-	P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, and these measurement gaps are overlapping with some but not all occasions of the SSB; and

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

If the high layer in TS 38.331 [13] signalling of smtc2 is present, TSMTCperiod follows smtc2; otherwise TSMTCperiod follows smtc1.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

Longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

Longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

For FR1, when the SSB for RLM is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for RLM without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for RLM without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

The normative reference for this requirement is TS 38.133 [6] clauses 8.1B.2.

##### 16.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM

The requirements in this clause apply for each CSI-RS based RLM-RS resource configured for PCell, provided that the CSI-RS configured for RLM is actually transmitted within UE active DL BWP during the entire evaluation period specified in this clause. UE is not expected to perform radio link monitoring measurements on the CSI-RS configured as RLM-RS if the CSI-RS is not in the active TCI state of any CORESET configured in the UE active BWP.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS,RedCap ms period becomes worse than the threshold Qout_CSI-RS,RedCap within TEvaluate_out_CSI-RS,RedCap ms evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS,RedCap ms period becomes better than the threshold Qin_CSI-RS,RedCap within TEvaluate_in_CSI-RS,RedCap ms evaluation period.

TEvaluate_out_CSI-RS,RedCap and TEvaluate_in_CSI-RS,RedCap are defined in Table 16.5.1.0.3-1 and Table 16.5.1.0.3-2 for FR1 for RedCap 2Rx RedCap and 1Rx RedCap, respectively.

Table 16.5.1.0.3-1: Evaluation period TEvaluate_out_CSI-RS,RedCap and TEvaluate_in_CSI-RS,RedCap for FR1 for 2Rx RedCap UE

| Configuration | TEvaluate_out_CSI-RS,RedCap (ms) | TEvaluate_in_CSI-RS,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout,RedCap×P)×TCSI-RS) | Max(100, Ceil(Min,RedCap×P) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout,RedCap ×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min,RedCap×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout,RedCap ×P) × TDRX | Ceil(Min,RedCap ×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Table 16.5.1.0.3-2: Evaluation period TEvaluate_out_CSI-RS,RedCap and TEvaluate_in_CSI-RS,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_CSI-RS,RedCap (ms) | TEvaluate_in_CSI-RS,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(Mout,RedCap×P)×TCSI-RS) | Max(100, Ceil(Min,RedCap×P) × TCSI-RS) |
| DRX ≤ 320ms | Max(400, Ceil(1.5×Mout,RedCap ×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min,RedCap×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout,RedCap ×P) × TDRX | Ceil(Min,RedCap ×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

For FR1,

-	P=11-TCSI-RSMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, and these measurement gaps are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

Longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

Longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

For FR1, when the CSI-RS for RLM is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for RLM in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD, or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for RLM, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for RLM, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS for RLM measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for RLM and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

For FR1, when the CSI-RS for RLM is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for RLM without any restriction.

The normative reference for this requirement is TS 38.133 [6] clauses 8.1B.3.

#### 16.5.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE

16.5.1.1.1	Test purpose

To verify that the 1Rx RedCap UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell for 1Rx RedCap UE. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.2.

16.5.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards.

16.5.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.1.

16.5.1.1.4	Test description

Same as the test description given in clause 6.5.1.1.4.

16.5.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.1.4.1 with following exceptions:

-	Table 6.5.1.1.4.1-1 is replaced by Table 16.5.1.1.4.1-1.

-	Table 6.5.1.1.4.1-2 is replaced by Table 16.5.1.1.4.1-2.

-	Table 6.5.1.1.4.1-4 is replaced by Table 16.5.1.1.4.1-3.

Table 16.5.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.1-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.1-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.1-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.1-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.1.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1,2,3,4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.96 |
| T3 |  |  | s | 1 |
| D1 |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.1.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.1.4.2.

16.5.1.1.4.3	Message Contents

Same as the message contents given in clause 6.5.1.1.4.3.

16.5.1.1.5	Test Requirement

Same as the test requirements given in clause 6.5.1.1.5 with following exceptions:

-	Table 6.5.1.1.5-1 is replaced by Table 16.5.1.1.5-1.

Table 16.5.1.1.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1,2,4 | dB | 1.8 | -6.2 | -15.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 |
|  | Config 1,2,4 | dBm/15kHz | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1,2,4 | dBm/SCS | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.1.4-1. |  |  |  |  |  |

#### 16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE

16.5.1.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell for 2Rx RedCap UE. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.2.

16.5.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards.

16.5.1.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.2.

16.5.1.2.4	Test description

Same as the test description given in clause 6.5.1.1.4.

16.5.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.1.4.1 with following exceptions:

-	Table 6.5.1.1.4.1-1 is replaced by Table 16.5.1.2.4.1-1.

-	Table 6.5.1.1.4.1-2 is replaced by Table 16.5.1.2.4.1-2.

-	Table 6.5.1.1.4.1-4 is replaced by Table 16.5.1.2.4.1-3.

Table 16.5.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.2-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.2-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.2-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.2-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.2.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1,2,3,4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.48 |
| T3 |  |  | s | 0.48 |
| D1 |  |  | s | 0.44 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.2.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.1.4.2.

16.5.1.2.4.3	Message Contents

Same as the message contents given in clause 6.5.1.1.4.3.

16.5.1.2.5	Test Requirement

Same as the test requirements given in clause 6.5.1.1.5 with following exceptions:

-	Table 6.5.1.1.5-1 is replaced by Table 16.5.1.2.5-1.

Table 16.5.1.2.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1,2,4 | dB | 1.8Note 5 | -6.2Note 5 | -15.8Note 5 |
|  | Config 3 |  | 1.8Note 5 | -6.2Note 5 | -15.8Note 5 |
|  | Config 1,2,4 | dBm/15kHz | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1,2,4 | dBm/SCS | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.1.4-1. Note 5: Including the test tolerance given in Annex F. |  |  |  |  |  |

#### 16.5.1.3	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE

16.5.1.3.1	Test purpose

To verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.

16.5.1.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards.

16.5.1.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.3.

16.5.1.3.4	Test description

Same as the test description given in clause 6.5.1.2.4.

16.5.1.3.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.2.4.1 with following exceptions:

-	Table 6.5.1.2.4.1-1 is replaced by Table 16.5.1.3.4.1-1.

-	Table 6.5.1.2.4.1-2 is replaced by Table 16.5.1.3.4.1-2.

-	Table 6.5.1.2.4.1-4 is replaced by Table 16.5.1.3.4.1-3.

Table 16.5.1.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.3-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.3-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.3-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.3-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.3.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.3.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference |  | Config 1,4 |  | CR.1.1 FDD |
| Channel |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference |  | Config 1,4 |  | CCR.1.1 FDD |
| Channel |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1,2,3,4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier |  | Config 1,2,4 |  | 15 kHz |
| spacing |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| In sync transmission | DCI format |  |  | 1-0 |
| parameters | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 1000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for | Config 1,4 |  |  | CSI-RS.1.1 FDD |
| CSI reporting | Config 2 |  |  | CSI-RS.1.1 TDD |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking | Config 1,4 |  |  | TRS.1.1 FDD |
|  | Config 2 |  |  | TRS.1.1 TDD |
|  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.24 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.3.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.2.4.2.

16.5.1.3.4.3	Message Contents

Same as the message contents given in clause 6.5.1.2.4.3.

16.5.1.3.5	Test Requirement

Same as the test requirements given in clause 6.5.1.2.5 with following exceptions:

-	Table 6.5.1.2.5-1 is replaced by Table 16.5.1.3.5-1.

Table 16.5.1.3.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1,2,4 | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 1,2,4 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1,2,4 | dBm/SCS | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure 6.5.1.2.1-1. |  |  |  |  |  |  |  |

#### 16.5.1.4	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE

16.5.1.4.1	Test purpose

To verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.

16.5.1.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards.

16.5.1.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.4.

16.5.1.4.4	Test description

Same as the test description given in clause 6.5.1.2.4.

16.5.1.4.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.2.4.1 with following exceptions:

-	Table 6.5.1.2.4.1-1 is replaced by Table 16.5.1.4.4.1-1.

-	Table 6.5.1.2.4.1-2 is replaced by Table 16.5.1.4.4.1-2.

-	Table 6.5.1.2.4.1-4 is replaced by Table 16.5.1.4.4.1-3.

Table 16.5.1.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.4-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.4-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.4-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.4-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.4.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.4.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference |  | Config 1,4 |  | CR.1.1 FDD |
| Channel |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference |  | Config 1,4 |  | CCR.1.1 FDD |
| Channel |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1,2,3,4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier |  | Config 1,2,4 |  | 15 kHz |
| spacing |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| In sync transmission | DCI format |  |  | 1-0 |
| parameters | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 1000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for | Config 1,4 |  |  | CSI-RS.1.1 FDD |
| CSI reporting | Config 2 |  |  | CSI-RS.1.1 TDD |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking | Config 1,4 |  |  | TRS.1.1 FDD |
|  | Config 2 |  |  | TRS.1.1 TDD |
|  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.24 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.4.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.2.4.2.

16.5.1.4.4.3	Message Contents

Same as the message contents given in clause 6.5.1.2.4.3.

16.5.1.4.5	Test Requirement

Same as the test requirements given in clause 6.5.1.2.5 with following exceptions:

-	Table 6.5.1.2.5-1 is replaced by Table 16.5.1.4.5-1.

Table 16.5.1.4.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1,2,4 | dB | 1.8Note 6 | -6.2 Note 6 | -15.8 Note 6 | -5.3 Note 6 | 1.8 Note 6 |
|  | Config 3 |  | 1.8 Note 6 | -6.2 Note 6 | -15.8 Note 6 | -5.3 Note 6 | 1.8 Note 6 |
|  | Config 1,2,4 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1,2,4 | dBm/SCS | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure 6.5.1.2.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |  |

#### 16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

16.5.1.5.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used for 1Rx RedCap UE. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.

16.5.1.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards.

16.5.1.5.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.5.

16.5.1.5.4	Test description

Same as the test description given in clause 6.5.1.3.4.

16.5.1.5.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.3.4.1 with following exceptions:

-	Table 6.5.1.3.4.1-1 is replaced by Table 16.5.1.5.4.1-1.

-	Table 6.5.1.3.4.1-2 is replaced by Table 16.5.1.5.4.1-2.

-	Table 6.5.1.3.4.1-4 is replaced by Table 16.5.1.5.4.1-3.

Table 16.5.1.5.4.1-1: Supported test configurations for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.1.5-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.5-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.5-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.5-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.5.4.1-2: Initial conditions for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.5.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.5.4.1-3: General test parameters for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
|  |  | Config 4 |  | 10: NRB,c = 52 |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
|  |  | Config 4 |  | Not Applicable |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1, 4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1, 4 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1, 2, 4 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2, 4 |  | Table A.7.1, PRACH.1 FR1 |
|  |  | Config 3 |  | Table A.7.1, PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.68 |
| T3 |  |  | s | 0.68 |
| D1 |  |  | s | 0.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.5.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.3.4.2.

16.5.1.5.4.3	Message Contents

Same as the message contents given in clause 6.5.1.3.4.3.

16.5.1.5.5	Test Requirement

Same as the test requirements given in clause 6.5.1.3.5 with following exceptions:

-	Table 6.5.1.3.5-1 is replaced by Table 16.5.1.5.5-1.

Table 16.5.1.5.5-1: Cell specific test parameters (Cell 1) for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1, 4 | dB | 1.8 | -6.2 | -15.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 |
|  | Config 1, 4 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1, 4 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.3.4-1. |  |  |  |  |  |

#### 16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

16.5.1.6.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used for 2Rx RedCap UE. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.

16.5.1.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards.

16.5.1.6.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.6.

16.5.1.6.4	Test description

Same as the test description given in clause 6.5.1.3.4.

16.5.1.6.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.3.4.1 with following exceptions:

-	Table 6.5.1.3.4.1-1 is replaced by Table 16.5.1.6.4.1-1.

-	Table 6.5.1.3.4.1-2 is replaced by Table 16.5.1.6.4.1-2.

-	Table 6.5.1.3.4.1-4 is replaced by Table 16.5.1.6.4.1-3.

Table 16.5.1.6.4.1-1: Supported test configurations for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.1.6-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.6-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.6-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.6-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.6.4.1-2: Initial conditions for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.6.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.6.4.1-3: General test parameters for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
|  |  | Config 4 |  | 10: NRB,c = 52 |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
|  |  | Config 4 |  | Not Applicable |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1, 4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1, 4 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1, 2, 4 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2, 4 |  | Table A.7.1, PRACH.1 FR1 |
|  |  | Config 3 |  | Table A.7.1, PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.68 |
| T3 |  |  | s | 0.68 |
| D1 |  |  | s | 0.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.6.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.3.4.2.

16.5.1.6.4.3	Message Contents

Same as the message contents given in clause 6.5.1.3.4.3.

16.5.1.6.5	Test Requirement

Same as the test requirements given in clause 6.5.1.3.5 with following exceptions:

-	Table 6.5.1.3.5-1 is replaced by Table 16.5.1.6.5-1.

Table 16.5.1.6.5-1: Cell specific test parameters (Cell 1) for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1, 4 | dB | 1.8 | -6.2 | -15.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 |
|  | Config 1, 4 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1, 4 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS Res. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.3.4-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |

#### 16.5.1.7	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

16.5.1.7.1	Test purpose

To verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.

16.5.1.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards.

16.5.1.7.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.7.

16.5.1.7.4	Test description

Same as the test description given in clause 6.5.1.4.4.

16.5.1.7.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.4.4.1 with following exceptions:

-	Table 6.5.1.4.4.1-1 is replaced by Table 16.5.1.7.4.1-1.

-	Table 6.5.1.4.4.1-2 is replaced by Table 16.5.1.7.4.1-2.

-	Table 6.5.1.4.4.1-4 is replaced by Table 16.5.1.7.4.1-3.

Table 16.5.1.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.7-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.7-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.7-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.7-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.7.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.7.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference |  | Config 1,4 |  | CR.1.1 FDD |
| Channel |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference |  | Config 1,4 |  | CCR.1.1 FDD |
| Channel |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1,2,3,4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier |  | Config 1,2,4 |  | 15 kHz |
| spacing |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| In sync transmission | DCI format |  |  | 1-0 |
| parameters | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 1000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for | Config 1,4 |  |  | CSI-RS.1.1 FDD |
| CSI reporting | Config 2 |  |  | CSI-RS.1.1 TDD |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking | Config 1,4 |  |  | TRS.1.1 FDD |
|  | Config 2 |  |  | TRS.1.1 TDD |
|  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.64 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.7.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.4.4.2.

16.5.1.7.4.3	Message Contents

Same as the message contents given in clause 6.5.1.4.4.3.

16.5.1.7.5	Test Requirement

Same as the test requirements given in clause 6.5.1.4.5 with following exceptions:

-	Table 6.5.1.4.5-1 is replaced by Table 16.5.1.7.5-1.

Table 16.5.1.7.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1,2,4 | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 1,2,4 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1,2,4 | dBm/SCS | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure 6.5.1.4.1-1. |  |  |  |  |  |  |  |

#### 16.5.1.8	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

16.5.1.8.1	Test purpose

To verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.

16.5.1.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards.

16.5.1.8.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.8.

16.5.1.8.4	Test description

Same as the test description given in clause 6.5.1.4.4.

16.5.1.8.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.4.4.1 with following exceptions:

-	Table 6.5.1.4.4.1-1 is replaced by Table 16.5.1.8.4.1-1.

-	Table 6.5.1.4.4.1-2 is replaced by Table 16.5.1.8.4.1-2.

-	Table 6.5.1.4.4.1-4 is replaced by Table 16.5.1.8.4.1-3.

Table 16.5.1.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.8-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.8-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.8-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.8-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.8.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.8.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference |  | Config 1,4 |  | CR.1.1 FDD |
| Channel |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference |  | Config 1,4 |  | CCR.1.1 FDD |
| Channel |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1,2,3,4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier |  | Config 1,2,4 |  | 15 kHz |
| spacing |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| In sync transmission | DCI format |  |  | 1-0 |
| parameters | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 2000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for | Config 1,4 |  |  | CSI-RS.1.1 FDD |
| CSI reporting | Config 2 |  |  | CSI-RS.1.1 TDD |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking | Config 1,4 |  |  | TRS.1.1 FDD |
|  | Config 2 |  |  | TRS.1.1 TDD |
|  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.64 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.8.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.4.4.2.

16.5.1.8.4.3	Message Contents

Same as the message contents given in clause 6.5.1.4.4.3.

16.5.1.8.5	Test Requirement

Same as the test requirements given in clause 6.5.1.4.5 with following exceptions:

-	Table 6.5.1.4.5-1 is replaced by Table 16.5.1.8.5-1.

Table 16.5.1.8.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1,2,4 | dB | 1.8Note 6 | -6.2 Note 6 | -15.8 Note 6 | -5.3 Note 6 | 1.8 Note 6 |
|  | Config 3 |  | 1.8Note 6 | -6.2 Note 6 | -15.8 Note 6 | -5.3 Note 6 | 1.8 Note 6 |
|  | Config 1,2,4 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1,2,4 | dBm/SCS | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure 6.5.1.4.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |  |

#### 16.5.1.9	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE

16.5.1.9.1	Test purpose

To verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used for 1 Rx RedCap UE. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in 38.133 [6] clause 8.1B.3.

16.5.1.9.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting CSI-RS based RLM.

16.5.1.9.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.9.

16.5.1.9.4	Test description

Same as the test description given in clause 6.5.1.5.4.

16.5.1.9.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.5.4.1 with following exceptions:

-	Table 6.5.1.5.4.1-1 is replaced by Table 16.5.1.9.4.1-1.

-	Table 6.5.1.5.4.1-2 is replaced by Table 16.5.1.9.4.1-2.

-	Table 6.5.1.5.4.1-3 is replaced by Table 16.5.1.9.4.1-3.

Table 16.5.1.9.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.9-1 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.1.9-2 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.1.9-3 | 30 kHz SSB SCS, 30 kHz data SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.1.9-4 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.9.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.9.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.9.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM |  | Config 1,4 |  | Resource #4 in TRS.1.1 FDD |
|  |  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  |  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.8 |
| T3 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.9.4.2	Test procedure

Same as the test procedure given in clause 6.5.1.5.4.2.

16.5.1.9.4.3	Message contents

Same as the message contents given in clause 6.5.1.5.4.3.

16.5.1.9.5	Test requirement

Same as the test requirements given in clause 6.5.1.5.5 with following exceptions:

-	Table 6.5.1.5.5-1 is replaced by Table 16.5.1.9.5-1.

Table 16.5.1.9.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |
| SNR on RLM-RS | dB | 1.8 | -6.2 | -15.8 |
| Noc | dBm/15KHz | -98 |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.1.5.4-1. |  |  |  |  |

#### 16.5.1.10	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE

16.5.1.10.1	Test purpose

To verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used for 2 Rx RedCap UE. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in 38.133 [6] clause 8.1B.3.

16.5.1.10.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting CSI-RS based RLM.

16.5.1.10.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.10.

16.5.1.10.4	Test description

Same as the test description given in clause 6.5.1.5.4.

16.5.1.10.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.5.4.1 with following exceptions:

-	Table 6.5.1.5.4.1-1 is replaced by Table 16.5.1.10.4.1-1.

-	Table 6.5.1.5.4.1-2 is replaced by Table 16.5.1.10.4.1-2.

-	Table 6.5.1.5.4.1-3 is replaced by Table 16.5.1.10.4.1-3.

Table 16.5.1.10.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.10-1 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.1.10-2 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.1.10-3 | 30 kHz SSB SCS, 30 kHz data SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.1.10-4 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.10.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.10.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.10.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM |  | Config 1,4 |  | Resource #4 in TRS.1.1 FDD |
|  |  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  |  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| SSB index assigned as BFD-RS |  |  |  | 0 |
| TCI configuration for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.48 |
| T3 |  |  | s | 0.48 |
| D1 |  |  | s | 0.44 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.10.4.2	Test procedure

Same as the test procedure given in clause 6.5.1.5.4.2.

16.5.1.10.4.3	Message contents

Same as the message contents given in clause 6.5.1.5.4.3.

16.5.1.10.5	Test requirement

Same as the test requirements given in clause 6.5.1.5.5 with following exceptions:

-	Table 6.5.1.5.5-1 is replaced by Table 16.5.1.10.5-1.

Table 16.5.1.10.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |
| SNR on RLM-RS | dB | 1.8 | -6.2 | -15.8 |
| Noc | dBm/15KHz | -98 |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.1.5.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |

#### 16.5.1.11	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE

16.5.1.11.1	Test purpose

To verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in 38.133 [6] clause 8.1B.3.

16.5.1.11.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting CSI-RS based RLM.

16.5.1.11.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.11.

16.5.1.11.4	Test description

Same as the test description given in clause 6.5.1.6.4.

16.5.1.11.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.6.4.1 with following exceptions:

-	Table 6.5.1.6.4.1-1 is replaced by Table 16.5.1.11.4.1-1.

-	Table 6.5.1.6.4.1-2 is replaced by Table 16.5.1.11.4.1-2.

-	Table 6.5.1.6.4.1-3 is replaced by Table 16.5.1.11.4.1-3.

Table 16.5.1.11.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.11-1 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.1.11-2 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.1.11-3 | 30 kHz SSB SCS, 30 kHz data SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.1.11-4 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.11.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.11.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.11.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM |  | Config 1,4 |  | Resource #4 in TRS.1.1 FDD |
|  |  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  |  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N.A |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 2000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.88 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 0.88 |
| T6 |  |  | s | 0.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.11.4.2	Test procedure

Same as the test procedure given in clause 6.5.1.6.4.2.

16.5.1.11.4.3	Message contents

Same as the message contents given in clause 6.5.1.6.4.3.

16.5.1.11.5	Test requirement

Same as the test requirements given in clause 6.5.1.6.5 with following exceptions:

-	Table 6.5.1.6.5-1 is replaced by Table 16.5.1.11.5-1.

Table 16.5.1.11.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
| SNR on RLM-RS | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | dBm/15kHz | -98 |  |  |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 6.5.1.6.4-1. |  |  |  |  |  |  |

#### 16.5.1.12	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE

16.5.1.12.1	Test purpose

To verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in 38.133 [6] clause 8.1B.3.

16.5.1.12.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting CSI-RS based RLM.

16.5.1.12.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.12.

16.5.1.12.4	Test description

Same as the test description given in clause 6.5.1.6.4.

16.5.1.12.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.6.4.1 with following exceptions:

-	Table 6.5.1.6.4.1-1 is replaced by Table 16.5.1.12.4.1-1.

-	Table 6.5.1.6.4.1-2 is replaced by Table 16.5.1.12.4.1-2.

-	Table 6.5.1.6.4.1-3 is replaced by Table 16.5.1.12.4.1-3.

Table 16.5.1.12.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.12-1 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.1.12-2 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.1.12-3 | 30 kHz SSB SCS, 30 kHz data SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.1.12-4 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.12.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.12.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.12.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM |  | Config 1,4 |  | Resource #4 in TRS.1.1 FDD |
|  |  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  |  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| SSB index assigned as BFD-RS |  |  |  | 0 |
| TCI configuration for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N.A |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 1000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.44 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 0.88 |
| T6 |  |  | s | 0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.12.4.2	Test procedure

Same as the test procedure given in clause 6.5.1.6.4.2.

16.5.1.12.4.3	Message contents

Same as the message contents given in clause 6.5.1.6.4.3.

16.5.1.12.5	Test requirement

Same as the test requirements given in clause 6.5.1.6.5 with following exceptions:

-	Table 6.5.1.6.5-1 is replaced by Table 16.5.1.12.5-1.

Table 16.5.1.12.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
| SNR on RLM-RS | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | dBm/15kHz | -98 |  |  |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 6.5.1.6.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |  |

#### 16.5.1.13	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE

16.5.1.13.1	Test purpose

To verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used for 1 Rx RedCap UE. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in 38.133 [6] clause 8.1B.3.

16.5.1.13.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting CSI-RS based RLM and long DRX cycle.

16.5.1.13.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.13.

16.5.1.13.4	Test description

Same as the test description given in clause 6.5.1.7.4.

16.5.1.13.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.7.4.1 with following exceptions:

-	Table 6.5.1.7.4.1-1 is replaced by Table 16.5.1.13.4.1-1.

-	Table 6.5.1.7.4.1-2 is replaced by Table 16.5.1.13.4.1-2.

-	Table 6.5.1.7.4.1-3 is replaced by Table 16.5.1.13.4.1-3.

Table 16.5.1.13.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.13-1 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.1.13-2 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.1.13-3 | 30 kHz SSB SCS, 30 kHz data SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.1.13-4 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.13.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.13.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.13.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM |  | Config 1,4 |  | Resource #4 in TRS.1.1 FDD |
|  |  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  |  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 2.24 |
| T3 |  |  | s | 2.28 |
| D1 |  |  | s | 2.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.13.4.2	Test procedure

Same as the test procedure given in clause 6.5.1.7.4.2.

16.5.1.13.4.3	Message contents

Same as the message contents given in clause 6.5.1.7.4.3.

16.5.1.13.5	Test requirement

Same as the test requirements given in clause 6.5.1.7.5 with following exceptions:

-	Table 6.5.1.7.5-1 is replaced by Table 16.5.1.13.5-1.

Table 16.5.1.13.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |
| SNR on RLM-RS | dB | 1.8 | -6.2 | -15.8 |
| Noc | dBm/15KHz | -98 |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.1.7.4-1. |  |  |  |  |

#### 16.5.1.14	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE

16.5.1.14.1	Test purpose

To verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used for 2 Rx RedCap UE. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in 38.133 [6] clause 8.1B.3.

16.5.1.14.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting CSI-RS based RLM and long DRX cycle.

16.5.1.14.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.14.

16.5.1.14.4	Test description

Same as the test description given in clause 6.5.1.7.4.

16.5.1.14.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.7.4.1 with following exceptions:

-	Table 6.5.1.7.4.1-1 is replaced by Table 16.5.1.14.4.1-1.

-	Table 6.5.1.7.4.1-2 is replaced by Table 16.5.1.14.4.1-2.

-	Table 6.5.1.7.4.1-3 is replaced by Table 16.5.1.14.4.1-3.

Table 16.5.1.14.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.14-1 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.1.14-2 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.1.14-3 | 30 kHz SSB SCS, 30 kHz data SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.1.14-4 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.14.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.14.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.14.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM |  | Config 1,4 |  | Resource #4 in TRS.1.1 FDD |
|  |  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  |  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| SSB index assigned as BFD-RS |  |  |  | 0 |
| TCI configuration for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 1.28 |
| T3 |  |  | s | 1.28 |
| D1 |  |  | s | 1.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.14.4.2	Test procedure

Same as the test procedure given in clause 6.5.1.7.4.2.

16.5.1.14.4.3	Message contents

Same as the message contents given in clause 6.5.1.7.4.3.

16.5.1.14.5	Test requirement

Same as the test requirements given in clause 6.5.1.7.5 with following exceptions:

-	Table 6.5.1.7.5-1 is replaced by Table 16.5.1.14.5-1.

Table 16.5.1.14.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |
| SNR on RLM-RS Config 1,2,3,4 | dB | 1.8 | -6.2 | -15.8 |
| Noc | dBm/15KHz | -98 |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.1.7.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |

#### 16.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE

16.5.1.15.1	Test purpose

To verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in 38.133 [6] clause 8.1B.3.

16.5.1.15.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting CSI-RS based RLM and long DRX cycle.

16.5.1.15.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.15.

16.5.1.15.4	Test description

Same as the test description given in clause 6.5.1.8.4.

16.5.1.15.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.8.4.1 with following exceptions:

-	Table 6.5.1.8.4.1-1 is replaced by Table 16.5.1.15.4.1-1.

-	Table 6.5.1.8.4.1-2 is replaced by Table 16.5.1.15.4.1-2.

-	Table 6.5.1.8.4.1-3 is replaced by Table 16.5.1.15.4.1-3.

Table 16.5.1.15.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.15-1 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.1.15-2 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.1.15-3 | 30 kHz SSB SCS, 30 kHz data SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.1.15-4 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.15.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.15.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.15.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM |  | Config 1,4 |  | Resource #4 in TRS.1.1 FDD |
|  |  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  |  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 4000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 2.28 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 1.2 |
| T6 |  |  | s | 1 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.15.4.2	Test procedure

Same as the test procedure given in clause 6.5.1.8.4.2.

16.5.1.15.4.3	Message contents

Same as the message contents given in clause 6.5.1.8.4.3.

16.5.1.15.5	Test requirement

Same as the test requirements given in clause 6.5.1.8.5 with following exceptions:

-	Table 6.5.1.8.5-1 is replaced by Table 16.5.1.15.5-1.

Table 16.5.1.15.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
| SNR on RLM-RS | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | dBm/15kHz | -98 |  |  |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 6.5.1.8.4-1. |  |  |  |  |  |  |

#### 16.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE

16.5.1.16.1	Test purpose

To verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in 38.133 [6] clause 8.1B.3.

16.5.1.16.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting CSI-RS based RLM and long DRX cycle.

16.5.1.16.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.16.

16.5.1.16.4	Test description

Same as the test description given in clause 6.5.1.8.4.

16.5.1.16.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.8.4.1 with following exceptions:

-	Table 6.5.1.8.4.1-1 is replaced by Table 16.5.1.16.4.1-1.

-	Table 6.5.1.8.4.1-2 is replaced by Table 16.5.1.16.4.1-2.

-	Table 6.5.1.8.4.1-3 is replaced by Table 16.5.1.16.4.1-3.

Table 16.5.1.16.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.16-1 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.1.16-2 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.1.16-3 | 30 kHz SSB SCS, 30 kHz data SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.1.16-4 | 15 kHz SSB SCS, 15 kHz data SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.16.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.16.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.16.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM |  | Config 1,4 |  | Resource #4 in TRS.1.1 FDD |
|  |  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  |  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| SSB index assigned as BFD-RS |  |  |  | 0 |
| TCI configuration for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 2000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 1.24 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 1.88 |
| T6 |  |  | s | 1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.16.4.2	Test procedure

Same as the test procedure given in clause 6.5.1.8.4.2.

16.5.1.16.4.3	Message contents

Same as the message contents given in clause 6.5.1.8.4.3.

16.5.1.16.5	Test requirement

Same as the test requirements given in clause 6.5.1.8.5 with following exceptions:

-	Table 6.5.1.8.5-1 is replaced by Table 16.5.1.16.5-1.

Table 16.5.1.16.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
| SNR on RLM-RS | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | dBm/15kHz | -98 |  |  |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 6.5.1.8.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |  |

### 16.5.2	Beam Failure Detection and Link recovery procedures for RedCap

#### 16.5.2.0	Minimum requirements

##### 16.5.2.0.1	General

The Redcap UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set  as specified in TS 38.213 [3] in order to detect beam failure on PCell in SA. The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified.

The RS resource configurations in the set  on PCell can be periodic CSI-RS resources and/or SSBs. UE is not required to perform beam failure detection outside the active DL BWP. UE is not required to meet the requirements in 38.133 [6] clause 8.5B.2 and 8.5B.3 if UE does not have set .

On each RS resource configuration in the set , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR_Redcap for the purpose of accessing downlink radio link quality of the serving cell beams.

The threshold Qout_LR_Redcap is defined as the level at which the downlink radio level link of a given resource configuration on set  cannot be reliably received and shall correspond to the BLERout = 10% block error rate of a hypothetical PDCCH transmission. For SSB based beam failure detection, Qout_LR_SSB is derived based on the hypothetical PDCCH transmission parameters listed in Table 16.5.2.0.1-1. For CSI-RS based beam failure detection, Qout_LR_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in Table 16.5.2.0.1-2.

Table 16.5.2.0.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16 for 1 Rx UE; 8 for 2 Rx UE |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 48 for 1 Rx UE; 24 for 2 Rx UE |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note 1： Note: SCS=60kHz is not applicable for FR1 |  |

Table 16.5.2.0.1-2: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16 for 1 Rx UE; 8 for 2 Rx UE |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0dB |
| Bandwidth (PRBs) | 48 for 1 Rx UE; 24 for 2 Rx UE |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note 1： Note: SCS=60kHz is not applicable for FR1 |  |

Upon request the UE shall deliver configuration indexes from the set as specified in TS 38.213 [3] to higher layers,  and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR_RedCap , which is indicated by higher layer parameter rsrp-ThresholdSSB. The UE applies the Qin_LR_RedCap  threshold to the L1-RSRP measurement obtained from an SSB. The UE applies the Qin_LR_RedCap  threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by higher layer parameter powerControlOffsetSS. The RS resource configurations in the set  can be periodic CSI-RS resources or SSBs or both SSB and CSI-RS resources. UE is not required to perform candidate beam detection outside the active DL BWP.

When the radio link quality on all the RS resources in set  is worse than Qout_LR_Redcap, layer 1 of the UE shall send a beam failure instance indication to the higher layers

The beam failure instance evaluation for the RS resources in set  shall be performed as specified in TS 38.213 [3] clause 6. Two successive indications from layer 1 shall be separated by at least TIndication_interval_BFD_Redcap.

When DRX is not used, TIndication_interval_BFD_Redcap is max(2ms, TSSB-RS,M) ) or max(2ms, TCSI-RS,M), where TSSB-RS,M and TCSI-RS,M is the shortest periodicity of all RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB  in the set  or CSI-RS resource in the set .

When DRX is used, for SSB based link quality measurement,

-	TIndication_interval_BFD_Redcap = Max(1.5 × DRX_cycle_length, 1.5 × TSSB-RS,M), if DRX_cycle_length ≤ 320ms,

-	TIndication_interval_BFD_Redcap = DRX_cycle_length, if DRX_cycle_length > 320ms.

When DRX is used, for CSI-RS based link quality measurement,

-	TIndication_interval_BFD_Redcap = Max(1.5 × DRX_cycle_length, 1.5 × TCSI-RS,M), if DRX_cycle_length ≤ 320ms,

-	TIndication_interval_BFD_Redcap = DRX_cycle_length, if DRX_cycle_length > 320ms.

For HD-FDD UE, the above conditions and requirements TIndication_interval_BFD_Redcap apply given that for each BFD-RS configuration, at least one BFD-RS sample must fall with DL occasion within an indication period.

There are no scheduling restrictions due to beam failure detection performed on SSB and CSI-RS configured for BFD with the same SCS as PDSCH or PDCCH in FR1.

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to beam failure detection when SSB is configured as BFD. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to beam failure detection when SSB is configured as BFD.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for beam failure detection.

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as link recovery detection resource with the same SCS as PDSCH or PDCCH in FR1.

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as link recovery detection resource. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured as link recovery detection resource.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, TRS, CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for L1-RSRP.

When the UE transitions between BFD CD-SSB resource and BFD NCD-SSB resource during one evaluation period, the UE is allowed to restart the ongoing evaluation using the second SSB type after BWP switching. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each BFD-RS resource.

The requirements in 38.133 [6] clause 8.5.10 shall also apply except the scenarios described above.

The normative reference for this requirement is TS 38.133 [6] clauses 8.5B.1, 8.5B.2, 8.5B.3, 8.5B.4, 8.5B.6, 8.5B.8 and 8.5B.9.

##### 16.5.2.0.2	Minimum conformance requirements for SSB-based BFD and CBD

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP during the entire evaluation period specified in Table 16.5.2.0.2-1.

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in Table 16.5.2.0.2-2.

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB_Redcap ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB_Redcap ms period.

The value of TEvaluate_BFD_SSB_Redcap is defined in Table 16.5.2.0.2-1 for FR1.

For FR1,

-	P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB.

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

where,

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer evaluation period would be expected if the combination of BFD-RS resource, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For an FR1 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 16.5.2.0.2-1: Evaluation period TEvaluate_BFD_SSB_Redcap for FR1

| Configuration | TEvaluate_BFD_SSB_Redcap (ms) for Redcap UE with 2 Rx | TEvaluate_BFD_SSB_Redcap (ms) for 1 Rx Redcap |
| --- | --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) | [ Max(50, Ceil(10 P) TSSB)] |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) | [ Max(50, Ceil(15 P) Max(TDRX,TSSB)) ] |
| DRX cycle > 320ms | Ceil(5 P) TDRX | [ Ceil(10 P) TDRX] |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |  |

The UE is required to be capable of measuring SSB for BFD without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following scenarios.

For FR1, when the SSB for BFD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for BFD measurement and CSI-RS. Longer measurement period for SSB based BFD measurement is expected, and no requirements are defined.

The UE shall monitor the configured SSB resources using the evaluation period in Table 16.5.2.0.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_SSB_Redcap is defined in Table 16.5.2.0.2-2 for FR1.

where,

For FR1,

-	P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB,

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

where,

If the high layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For an FR1 serving cell, longer CBD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of PCBD used in Table 16.5.2.0.2-2 are defined as

For each SSB resource in the set  configured for PCell

-	PCBD = 1.

Table 16.5.2.0.2-2: Evaluation period TEvaluate_CBD_SSB_Redcap for FR1

| Configuration | TEvaluate_CBD_SSB_Redcap (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 P PCBD) TSSB) |
| DRX cycle > 320ms | Ceil(3 P PCBD) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

The normative reference for this requirement is TS 38.133 [6] clauses 8.5B.2 and 8.5B.5.

##### 16.5.2.0.3	Minimum conformance requirements for CSI-RS based RLM

The requirements in this clause apply for each CSI-RS resource in the set  of resource configurations for a serving cell, provided that the CSI-RS resource(s) in set for beam failure detection are actually transmitted within the UE active DL BWP during the entire evaluation period specified in Table 16.5.2.0.3-1.

The requirements in this clause apply for each CSI-RS resource in the set  configured for a serving cell, provided that the CSI-RS resources configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in Table 16.5.2.0.3-2.

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS_Redcap ms period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS_Redcap ms period.

The value of TEvaluate_BFD_CSI-RS_Redcap is defined in Table 16.5.2.0.3-1 for FR1.

For FR1,

-	P=11-TCSI-RSMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS.

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

where,

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note:	The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the BFD-RS resource, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For an FR1 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MBFD used in Table 16.5.2.0.3-1 are defined as

-	MBFD = 10, if the CSI-RS resource(s) in set  used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PBFD used in Table 16.5.2.0.3-1 are defined as

For each CSI-RS resource in the set  configured for PCell

-	PBFD = 1.

Table 16.5.2.0.3-1: Evaluation period TEvaluate_BFD_CSI-RS_Redcap for FR1

| Configuration | TEvaluate_BFD_CSI-RS_Redcap (ms) for Redcap UE with 2 Rx | TEvaluate_BFD_CSI-RS_Redcap (ms) for 1 Rx Redcap |
| --- | --- | --- |
| no DRX | Max(50, Ceil(MBFD P PBFD) TCSI-RS) | Max(50, Ceil(2MBFD P PBFD) TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) | Max(50, Ceil(21.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | Ceil(MBFD P PBFD) TDRX | Ceil(2MBFD P PBFD) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |  |

The UE is required to be capable of measuring CSI-RS for BFD without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following scenarios.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS_Redcap [ms] period becomes better than the threshold Qin_LR_RedCap  within TEvaluate_CBD_CSI-RS_Redcap [ms] period provided CSI-RS Ês/Iot is according to 38.133 [6] Annex Table B.2.4.2 for a corresponding band.

The UE shall monitor the configured CSI-RS resources using the evaluation period in Table 16.5.2.0.3-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_CSI-RS_Redcap is defined in Table 16.5.2.0.3-2 for FR1.

For FR1,

-	P=11-TCSI-RSMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

where,

If the high layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Note:	The overlap between CSI-RS for CBD and SMTC means that CSI-RS for CBD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM, BFD, BM-RS, or other CBD-RS, according to the measurement restrictions defined in clause 8.5.6.3.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For an FR1 serving cell, longer CBD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MCBD used in Table 16.5.2.0.3-2 are defined as

-	MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PCBD used in Table 16.5.2.0.3-2 are defined as

For each CSI-RS resource in the set  configured for PCell

-	PCBD = 1.

Table 16.5.2.0.3-2: Evaluation period TEvaluate_CBD_CSI-RS_Redcap for FR1

| Configuration | TEvaluateC_CBD_CSI-RS_Redcap (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P PCBD) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P PCBD) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

The normative reference for this requirement is TS 38.133 [6] clauses 8.5B.3 and 8.5B.6.

#### 16.5.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE

16.5.2.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in 38.133 [6] clause 8.5B.

16.5.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from release 17 onwards and supporting link recovery.

16.5.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.1.

16.5.2.1.4	Test description

Same as the test description given in clause 6.5.5.1.4.

16.5.2.1.4.1	Initial conditions

Same as the initial conditions 6.5.5.1.4.1 with following exceptions:

-	Table 6.5.5.1.4.1-1 is replaced by Table 16.5.2.1.4.1-1.

-	Table 6.5.5.1.4.1-2 is replaced by Table 16.5.2.1.4.1-2.

-	Table 6.5.5.1.4.1-3 is replaced by Table 16.5.2.1.4.1-3.

Table 16.5.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.2.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.2.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.2.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.2.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.1.4.1-3: General test parameters

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| SSB Configuration |  | Config 1,4 |  | SSB.3 FR1 |  |
|  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
| SMTC Configuration |  | Config 1,2,4 |  | SMTC.1 RedCap |  |
|  |  | Config 3 |  | SMTC.1 RedCap |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | 15 KHz |  |
|  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.2 FR1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  | 1 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |  |
| Beam failure detection | DCI format |  |  | 1-0 |  |
| transmission parameters | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 16 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | OFF |  |
| Gap pattern ID |  |  |  | gp0 |  |
| gapOffset |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1,2,4 | dBm/SCS | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  | -95 |  |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI |  | Config 1,4 |  | CSI-RS.1.1 FDD |  |
| reporting |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0,1 |  |
| T310 Timer |  | ms |  | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.37 |  |
| T3 |  |  | s | 0.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.17 |  |
| D1 |  |  | s | 0.13 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

16.5.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.5.5.1.4.2.

16.5.2.1.4.3	Message contents

Same as the message contents given in clause 6.5.5.1.4.3 with following exceptions:

-	Table 7.3.1-3 in TS 38.508-1 [14] is used with condition SMTC.1 RedCap.

16.5.2.1.5	Test requirement

Same as the test requirements given in clause 6.5.5.1.5 with following exceptions:

-	Table 6.5.5.1.5-1 is replaced by Table 16.5.2.1.5-1.

Table 16.5.2.1.5-1: NR Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1,2,4 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1,2,4 | dB | -13.25 | -13.25 | 13.25 | 13.25 | 13.25 |
|  | Config 3 |  | -13.25 | -13.25 | 13.25 | 13.25 | 13.25 |
| SSB_RP of set q1 | Config 1,2,4 | dBm/SCS | -111.25 | -111.25 | -84.75 | -84.75 | -84.75 |
|  | Config 3 |  | -108.25 | -108.25 | -81.75 | -81.75 | -81.75 |
|  | Config 1,2,3,4 | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.1.4-1. |  |  |  |  |  |  |  |

#### 16.5.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE

16.5.2.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in 38.133 [6] clause 8.5B.

16.5.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from release 17 onwards and supporting link recovery.

16.5.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.2.

16.5.2.2.4	Test description

Same as the test description given in clause 6.5.5.1.4.

16.5.2.2.4.1	Initial conditions

Same as the initial conditions 6.5.5.1.4.1 with following exceptions:

-	Table 6.5.5.1.4.1-1 is replaced by Table 16.5.2.2.4.1-1.

-	Table 6.5.5.1.4.1-2 is replaced by Table 16.5.2.2.4.1-2.

-	Table 6.5.5.1.4.1-3 is replaced by Table 16.5.2.2.4.1-3.

Table 16.5.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.2.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.2.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.2.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.2.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.2.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.5.2.2.4.1-3: General test parameters

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| SSB Configuration |  | Config 1,4 |  | SSB.3 FR1 |  |
|  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
| SMTC Configuration |  | Config 1,2,4 |  | SMTC.1 RedCap |  |
|  |  | Config 3 |  | SMTC.1 RedCap |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | 15 KHz |  |
|  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.2 FR1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  | 1 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection | DCI format |  |  | 1-0 |  |
| transmission parameters | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | OFF |  |
| Gap pattern ID |  |  |  | gp0 |  |
| gapOffset |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1,2,4 | dBm/SCS | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  | -95 |  |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI |  | Config 1,4 |  | CSI-RS.1.1 FDD |  |
| reporting |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0,1 |  |
| T310 Timer |  | ms |  | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.37 |  |
| T3 |  |  | s | 0.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.17 |  |
| D1 |  |  | s | 0.13 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

16.5.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.5.5.1.4.2.

16.5.2.2.4.3	Message contents

Same as the message contents given in clause 6.5.5.1.4.3 with following exceptions:

-	Table 7.3.1-3 in TS 38.508-1 [14] is used with condition SMTC.1 RedCap.

16.5.2.2.5	Test requirement

Same as the test requirements given in clause 6.5.5.1.5 with following exceptions:

-	Table 6.5.5.1.5-1 is replaced by Table 16.5.2.2.5-1.

Table 16.5.2.2.5-1: NR Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1,2,4 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1,2,4 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1,2,4 | dBm/ | -108.2 | -108.2 | -88.2 | -88.2 | -88.2 |
|  | Config 3 | SCS | -105.2 | -105.2 | -85.2 | -85.2 | -85.2 |
|  | Config 1,2,3,4 | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.1.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |  |  |

#### 16.5.2.3	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

16.5.2.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5B.2.

16.5.2.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards supporting link recovery and long DRX cycle.

16.5.2.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.3.

16.5.2.3.4	Test description

Same as the test description given in clause 6.5.5.2.4.

16.5.2.3.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.5.2.4.1 with following exceptions:

-	Table 6.5.5.2.4.1-1 is replaced by Table 16.5.2.3.4.1-1.

-	Table 6.5.5.2.4.1-2 is replaced by Table 16.5.2.3.4.1-2.

-	Table 6.5.5.2.4.1-3 is replaced by Table 16.5.2.3.4.1-3.

Table 16.5.2.3.4.1-1: Supported test configurations for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.2.3-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 16.5.2.3-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 16.5.2.3-3 | TDD duplex mode, 30 kHz SSB SCS, 20 MHz bandwidth |
| 16.5.2.3-4 | HD-FDD duplex mode, 15kHz SSB SCS, 10 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.2.3.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1and and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.3.4.1-3: General test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2, 3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |
|  |  | Config 4 |  | 10: NRB,c = 52 |  |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
|  |  | Config 4 |  | Not Applicable |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
|  |  | Config 4 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
|  |  | Config 4 |  | CCR.1.1 FDD |  |
| SSB Configuration |  | Config 1 |  | SSB.3 FR1 |  |
|  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
|  |  | Config 4 |  | SSB.3 FR1 |  |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |  |
|  |  | Config 3 |  | SMTC.1 |  |
|  |  | Config 4 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 KHz |  |
|  |  | Config 3 |  | 30 KHz |  |
|  |  | Config 4 |  | 15 KHz |  |
| PRACH Configuration |  | Config 1, 2 |  | PRACH.1 | Table A.7-1 |
|  |  | Config 3 |  | PRACH.1 | Table A.7-1 |
|  |  | Config 4 |  | PRACH.1 | Table A.7-1 |
| SSB Index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  | 1 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 16 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 | A.5 |
| Gap pattern ID |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | Absent | When the field is absent, the UE applies the value 0. (38.133 [6] Table 8.1.1-1). |
| Rsrp-ThresholdSSB | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  | -95 |  |
|  | Config 4 |  |  | -98 |  |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
|  | Config 4 |  |  | CSI-RS.1.1 FDD |  |
| CSI-RS for tracking | Config 1 |  |  | TRS.1.1 FDD |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |
|  | Config 4 |  |  | TRS.1.1 FDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0, 1 |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 5.17 |  |
| T3 |  |  | s | [5.84] |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

16.5.2.3.4.2	Test Procedure

Same as the test procedure given in clause 6.5.5.2.4.2.

16.5.2.3.4.3	Message Contents

Same as the message contents given in clause 6.5.5.2.4.3.

16.5.2.3.5	Test Requirement

Same as the test requirements given in clause 6.5.5.2.5 with following exceptions:

-	Table 6.5.5.2.5-1 is replaced by Table 16.5.2.3.5-1.

Table 16.5.2.3.5-1: Cell specific test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5.8 | -2.2 | -2.2 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -2.2 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -2.2 | -12.8 | -12.8 |
|  | Config 4 |  | 5.8 | -2.2 | -2.2 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1 | dB | -13.25 | -13.25 | -13.25 | -13.25 | -13.25 |
|  | Config 2 |  | -13.25 | -13.25 | -13.25 | -13.25 | -13.25 |
|  | Config 3 |  | -13.25 | -13.25 | -13.25 | -13.25 | -13.25 |
|  | Config 4 |  | -13.25 | -13.25 | -13.25 | -13.25 | -13.25 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -111.25 | -111.25 | -84.75 | -84.75 | -84.75 |
|  | Config 2 |  | -111.25 | -111.25 | -84.75 | -84.75 | -84.75 |
|  | Config 3 |  | -108.25 | -108.25 | -81.75 | -81.75 | -81.75 |
|  | Config 4 |  | -111.25 | -111.25 | -84.75 | -84.75 | -84.75 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 4 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting is assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS Res. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.1.4-1. |  |  |  |  |  |  |  |

#### 16.5.2.4	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

16.5.2.4.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5B.2.

16.5.2.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards supporting link recovery and long DRX cycle.

16.5.2.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.4.

16.5.2.4.4	Test description

Same as the test description given in clause 6.5.5.2.4.

16.5.2.4.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.5.2.4.1 with following exceptions:

-	Table 6.5.5.2.4.1-1 is replaced by Table 16.5.2.4.4.1-1.

-	Table 6.5.5.2.4.1-2 is replaced by Table 16.5.2.4.4.1-2.

-	Table 6.5.5.2.4.1-3 is replaced by Table 16.5.2.4.4.1-3.

Table 16.5.2.4.4.1-1: Supported test configurations for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.2.4-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 16.5.2.4-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 16.5.2.4-3 | TDD duplex mode, 30 kHz SSB SCS, 20 MHz bandwidth |
| 16.5.2.4-4 | HD-FDD duplex mode, 15kHz SSB SCS, 10 MHz bandwidth |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.2.4.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.4.4.1-3: General test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2, 3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| Bwchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |
|  |  | Config 4 |  | 10: NRB,c = 52 |  |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
|  |  | Config 4 |  | Not Applicable |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
|  |  | Config 4 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
|  |  | Config 4 |  | CCR.1.1 FDD |  |
| SSB Configuration |  | Config 1 |  | SSB.3 FR1 |  |
|  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
|  |  | Config 4 |  | SSB.3 FR1 |  |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |  |
|  |  | Config 3 |  | SMTC.1 |  |
|  |  | Config 4 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 KHz |  |
|  |  | Config 3 |  | 30 KHz |  |
|  |  | Config 4 |  | 15 KHz |  |
| PRACH Configuration |  | Config 1, 2 |  | PRACH.1 | Table A.7-1 |
|  |  | Config 3 |  | PRACH.1 | Table A.7-1 |
|  |  | Config 4 |  | PRACH.1 | Table A.7-1 |
| SSB Index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  | 1 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 | A.5 |
| Gap pattern ID |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | Absent | When the field is absent, the UE applies the value 0. (38.133 [6] Table 8.1.1-1). |
| Rsrp-ThresholdSSB | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  | -95 |  |
|  | Config 4 |  |  | -98 |  |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
|  | Config 4 |  |  | CSI-RS.1.1 FDD |  |
| CSI-RS for tracking | Config 1 |  |  | TRS.1.1 FDD |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |
|  | Config 4 |  |  | TRS.1.1 FDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0, 1 |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 5.17 |  |
| T3 |  |  | s | 3.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

16.5.2.4.4.2	Test Procedure

Same as the test procedure given in clause 6.5.5.2.4.2.

16.5.2.4.4.3	Message Contents

Same as the message contents given in clause 6.5.5.2.4.3.

16.5.2.4.5	Test Requirement

Same as the test requirements given in clause 6.5.5.2.5 with following exceptions:

-	Table 6.5.5.2.5-1 is replaced by Table 16.5.2.4.5-1.

Table 16.5.2.4.5-1: Cell specific test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 4 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 4 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 4 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 4 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting is assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS Res. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.2.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |  |  |

#### 16.5.2.5	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE

16.5.2.5.1	Test purpose

To verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in 38.133 [6] clause 8.5B.3.

16.5.2.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting CSI-RS based RLM and link recovery.

16.5.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.5.

16.5.2.5.4	Test description

Same as the test description given in clause 6.5.5.3.4.

16.5.2.5.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.5.3.4.1 with following exceptions:

-	Table 6.5.5.3.4.1-1 is replaced by Table 16.5.2.5.4.1-1.

-	Table 6.5.5.3.4.1-2 is replaced by Table 16.5.2.5.4.1-2.

-	Table 6.5.5.3.4.1-3 is replaced by Table 16.5.2.5.4.1-3.

Table 16.5.2.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.2.5-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.2.5-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.2.5-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.2.5-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.2.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.5.4.1-3: General test parameters

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| SSB Configuration |  | Config 1,2,4 |  | SSB.3 FR1 |  |
|  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 KHz |  |
|  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  |  | PRACH.4 FR1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 16 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | OFF |  |
| Gap pattern ID |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1,2,4 | dBm/SCS | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 | dBm/SCS | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], section 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], section 5.17 |
| CSI-RS configuration for q0 and q1 |  | Config 1,4 |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS |  | Config 1,4 |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.18 |  |
| T3 |  |  | s | 0.27 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.08 |  |
| D1 |  |  | s | 0.04 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

16.5.2.5.4.2	Test procedure

Same as the test procedure given in clause 6.5.5.3.4.2.

16.5.2.5.4.3	Message contents

Same as the message contents given in clause 6.5.5.3.4.3.

16.5.2.5.5	Test requirement

Same as the test requirements given in clause 6.5.5.3.5 with following exceptions:

-	Table 6.5.5.3.5-1 is replaced by Table 16.5.2.5.5-1.

Table 16.5.2.5.5-1: Cell specific test parameters

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 |  | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_CSI-RS of set q1 |  | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| CSI-RS_RP of set q1 | Config 1,2,4 | dBm/ SCS | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  |  | dBm/ 15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.3.4-1. |  |  |  |  |  |  |  |

#### 16.5.2.6	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE

16.5.2.6.1	Test purpose

To verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in 38.133 [6] clause 8.5B.3.

16.5.2.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting CSI-RS based RLM and link recovery.

16.5.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.6.

16.5.2.6.4	Test description

Same as the test description given in clause 6.5.5.3.4.

16.5.2.6.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.5.3.4.1 with following exceptions:

-	Table 6.5.5.3.4.1-1 is replaced by Table 16.5.2.6.4.1-1.

-	Table 6.5.5.3.4.1-2 is replaced by Table 16.5.2.6.4.1-2.

-	Table 6.5.5.3.4.1-3 is replaced by Table 16.5.2.6.4.1-3.

Table 16.5.2.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.2.6-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.2.6-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.2.6-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.2.6-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.2.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 with n=1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.6.4.1-3: General test parameters

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| SSB Configuration |  | Config 1,2,4 |  | SSB.3 FR1 |  |
|  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 KHz |  |
|  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  |  | PRACH.4 FR1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | OFF |  |
| Gap pattern ID |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1,2,4 | dBm/SCS | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 | dBm/SCS | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], section 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], section 5.17 |
| CSI-RS configuration for q0 and q1 |  | Config 1,4 |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS |  | Config 1,4 |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.18 |  |
| T3 |  |  | s | 0.14 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.08 |  |
| D1 |  |  | s | 0.04 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

16.5.2.6.4.2	Test procedure

Same as the test procedure given in clause 6.5.5.3.4.2.

16.5.2.6.4.3	Message contents

Same as the message contents given in clause 6.5.5.3.4.3.

16.5.2.6.5	Test requirement

Same as the test requirements given in clause 6.5.5.3.5 with following exceptions:

-	Table 6.5.5.3.5-1 is replaced by Table 16.5.2.6.5-1.

Table 16.5.2.6.5-1: Cell specific test parameters

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 |  | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_CSI-RS of set q1 |  | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| CSI-RS_RP of set q1 | Config 1,2,4 | dBm/ SCS | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  |  | dBm/ 15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.3.4-1. Note 8: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |  |  |

#### 16.5.2.7	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE

16.5.2.7.1	Test purpose

To verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in 38.133 [6] clause 8.5B.3.

16.5.2.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting CSI-RS based RLM, long DRX cycle and link recovery.

16.5.2.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.7.

16.5.2.7.4	Test description

Same as the test description given in clause 6.5.5.4.4.

16.5.2.7.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.5.4.4.1 with following exceptions:

-	Table 6.5.5.4.4.1-1 is replaced by Table 16.5.2.7.4.1-1.

-	Table 6.5.5.4.4.1-2 is replaced by Table 16.5.2.7.4.1-2.

-	Table 6.5.5.4.4.1-3 is replaced by Table 16.5.2.7.4.1-3.

Table 16.5.2.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.2.7-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.2.7-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.2.7-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.2.7-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.2.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.7.4.1-3: General test parameters

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| SSB Configuration |  | Config 1,2,4 |  | SSB.3 FR1 |  |
|  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 KHz |  |
|  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  |  | PRACH.4 FR1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 16 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 |  |
| Gap pattern ID |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1,2,4 | dBm/SCS | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 | dBm/SCS | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], section 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], section 5.17 |
| CSI-RS configuration for q0 and q1 |  | Config 1,4 |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS |  | Config 1,4 |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 8.37 |  |
| T3 |  |  | s | 12.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

16.5.2.7.4.2	Test procedure

Same as the test procedure given in clause 6.5.5.4.4.2.

16.5.2.7.4.3	Message contents

Same as the message contents given in clause 6.5.5.4.4.3.

16.5.2.7.5	Test requirement

Same as the test requirements given in clause 6.5.5.4.5 with following exceptions:

-	Table 6.5.5.4.5-1 is replaced by Table 16.5.2.7.5-1.

Table 16.5.2.7.5-1: Cell specific test parameters

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 |  | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_CSI-RS of set q1 |  | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| CSI-RS_RP of set q1 | Config 1,2,4 | dBm/ SCS | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  |  | dBm/ 15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.4.4-1. |  |  |  |  |  |  |  |

#### 16.5.2.8	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE

16.5.2.8.1	Test purpose

To verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in 38.133 [6] clause 8.5B.3.

16.5.2.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting CSI-RS based RLM, long DRX cycle and link recovery.

16.5.2.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.8.

16.5.2.8.4	Test description

Same as the test description given in clause 6.5.5.4.4.

16.5.2.8.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.5.4.4.1 with following exceptions:

-	Table 6.5.5.4.4.1-1 is replaced by Table 16.5.2.8.4.1-1.

-	Table 6.5.5.4.4.1-2 is replaced by Table 16.5.2.8.4.1-2.

-	Table 6.5.5.4.4.1-3 is replaced by Table 16.5.2.8.4.1-3.

Table 16.5.2.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.2.8-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.2.8-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.2.8-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.2.8-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.2.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.8.4.1-3: General test parameters

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| SSB Configuration |  | Config 1,2,4 |  | SSB.3 FR1 |  |
|  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 KHz |  |
|  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  |  | PRACH.4 FR1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 |  |
| Gap pattern ID |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1,2,4 | dBm/SCS | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 | dBm/SCS | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], section 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], section 5.17 |
| CSI-RS configuration for q0 and q1 |  | Config 1,4 |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS |  | Config 1,4 |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 8.37 |  |
| T3 |  |  | s | 6.44 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

16.5.2.8.4.2	Test procedure

Same as the test procedure given in clause 6.5.5.4.4.2.

16.5.2.8.4.3	Message contents

Same as the message contents given in clause 6.5.5.4.4.3.

16.5.2.8.5	Test requirement

Same as the test requirements given in clause 6.5.5.4.5 with following exceptions:

-	Table 6.5.5.4.5-1 is replaced by Table 16.5.2.8.5-1.

Table 16.5.2.8.5-1: Cell specific test parameters

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 |  | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_CSI-RS of set q1 |  | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| CSI-RS_RP of set q1 | Config 1,2,4 | dBm/ SCS | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  |  | dBm/ 15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.4.4-1. Note 8: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |  |  |

### 16.5.3	Active BWP switch delay for RedCap

#### 16.5.3.1	DCI-based and time-based active BWP switch for RedCap

##### 16.5.3.1.0	Minimum conformance requirements

16.5.3.1.0.1	Minimum conformance requirements for DCI-based and time-based active BWP switch for RedCap

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n. Where,

-	Y=0, if the serving cell where UE receives DCI for BWP switch request is same as the serving cell on which BWP switch occurs.

-	Y equals to the length of 1 slot, if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. In this scenario, TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change.

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n except DCI triggering BWP switch on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this clause when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where slot n is the first slot of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [13] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay after bwp-InactivityTimer [13] expires on the cell where timer-based BWP switch occurs.

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 16.5.3.1.0.1-1.

Table 16.5.3.1.0.1-1: BWP switch delay

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Provided the UE does not have the required TCI-state information to receive PDCCH and PDSCH in the new BWP, the UE shall use old TCI-states before the BWP switch until a new MAC CE updating the required TCI-state information for PDCCH and PDSCH is received after the BWP switch.

If UE has the information on the required TCI-state information to receive PDCCH and PDSCH in the new BWP,

-	UE shall be able to receive PDCCH and PDSCH with old TCI-states before the delay as specified in 38.133 [6] Clause 8.10 in the new BWP.

-	UE shall be able to receive PDCCH and PDSCH with new TCI-states after the delay as specified in 38.133 [6] Clause 8.10 in the new BWP.

If the BWP switch is triggered within or outside DRX active time, and one of the two BWPs in a BWP switching is a dormant BWP [TS 38.321, 12], UE shall be able to complete active BWP switching within the time duration of

-	TdormantBWPswitchDelay =TBWPswitchDelay+ X, provided that the dormancy indication is received in any of the first 3 OFDM symbols of a slot in the serving cell where DCI for dormancy indication is received, or

-	TdormantBWPswitchDelay =TBWPswitchDelay + X + Z, provided that the dormancy indication is received after the first 3 OFDM symbols of a slot in the serving cell where DCI for dormancy indication is received, where

-	TBWPswitchDelay is defined in Table 16.5.3.1.0.1-1 corresponding to the smallest value among the SCS of the serving cell where UE receives dormancy indication and the SCSs of the dormant BWP and the active BWP immediately before or after switching the BWP of the serving cell where BWP switching occurs.

-	X equals to the length of 1 slot corresponding to the smallest value among the SCS of the serving cell where UE receives dormancy indication and the SCSs of the dormant BWP and the active BWP immediately before or after switching the BWP of the serving cell where BWP switching occurs.

-	Z equals to the length of 1 slot corresponding to the SCS of the serving cell where UE receives dormancy indication.

For DCI-based BWP switch, if the new BWP is a dormant BWP, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive CSI-RS (for DL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL slot occurs right after a time duration of TdormantBWPswitchDelay which starts from the beginning of DL slot n.

The normative reference for this requirement is TS 38.133 [6] clauses 8.6A.2

##### 16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

16.5.3.1.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for 1Rx RedCap defined in TS 38.133 [6] clause 8.6A.

16.5.3.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards supporting BWP adaptation of at least 2 BWPs, DCI and timer-based active BWP switching delay Type1 or Type2.

16.5.3.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.3.0.1 and 16.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.3.1.1.

16.5.3.1.1.4	Test description

Same as the test description given in clause 6.5.6.1.2.4.

16.5.3.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.6.1.2.4.1 with following exceptions:

-	Table 6.5.6.1.2.4.1-1 is replaced by Table 16.5.3.1.1.4.1-1.

-	Table 6.5.6.1.2.4.1-2 is replaced by Table 16.5.3.1.1.4.1-2.

-	Table 6.5.6.1.2.4.1-3 is replaced by Table 16.5.3.1.1.4.1-3.

Table 16.5.3.1.1.4.1-1: Supported test configurations for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.3.1.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.3.1.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.3.1.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.3.1.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.3.1.1.4.1-2: Initial conditions for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.3.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.3.1.1.4.1-3: General test parameters for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | 200 |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

16.5.3.1.1.4.2	Test Procedure

Same as the test procedure given in clause 6.5.6.1.2.4.2.

16.5.3.1.1.4.3	Message Contents

Same as the message contents given in clause 6.5.6.1.2.4.3.

16.5.3.1.1.5	Test Requirement

Tables 16.5.3.1.1.4.1-3 and 16.5.3.1.1.5-1 define the primary level settings including test tolerances.

Table 16.5.3.1.1.5-1: Cell specific test parameters for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1,4 |  | FDD |
|  |  | Config 2,3 |  | TDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
|  |  | Config 4 |  | 10 MHz: NRB,c = 52 |
| Active BWP ID |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  | Config 1,2,3,4 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 RedCap Note 4 |
| Initial UL BWP Configuration |  | Config 1,2,3,4 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  | Config 1,4 |  | N/A |
|  |  | Config 2,3 |  | ULBWP.1.1 RedCap Note 4 |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  | CCR.1.2 FDD |
|  |  | Config 2 |  | CCR.1.2 TDD |
|  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x1 Low |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 | Config 1,2,4 |  | dBm/SCS | -104 |
|  | Config 3 |  |  | -101 |
| NocNote 2 |  |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2,4 |  | dBm/SCS | -87 |
|  | Config 3 |  |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4 | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.1 RedCap is linked with ULBWP.1.1 RedCap defined in clause 12 of TS 38.213 [8]. |  |  |  |  |

During T1, the UE shall start to send the ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 16.5.3.1.0.1-1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### 16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

16.5.3.1.2.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for 2Rx RedCap defined in TS 38.133 [6] clause 8.6A.

16.5.3.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards supporting BWP adaptation of at least 2 BWPs, DCI and timer-based active BWP switching delay Type1 or Type2.

16.5.3.1.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.3.1.2.

16.5.3.1.2.4	Test description

Same as the test description given in clause 6.5.6.1.2.4.

16.5.3.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.6.1.2.4.1 with following exceptions:

-	Table 6.5.6.1.2.4.1-1 is replaced by Table 16.5.3.1.2.4.1-1.

-	Table 6.5.6.1.2.4.1-2 is replaced by Table 16.5.3.1.2.4.1-2.

-	Table 6.5.6.1.2.4.1-3 is replaced by Table 16.5.3.1.2.4.1-3.

Table 16.5.3.1.2.4.1-1: Supported test configurations for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.3.1.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.3.1.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.3.1.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.3.1.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.3.1.2.4.1-2: Initial conditions for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.3.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.3.1.2.4.1-3: General test parameters for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | 200 |  |
| PDCCH and PDSCH maximum number of HARQ transmission |  | 1 |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

16.5.3.1.2.4.2	Test Procedure

Same as the test procedure given in clause 6.5.6.1.2.4.2.

16.5.3.1.2.4.3	Message Contents

Same as the message contents given in clause 6.5.6.1.2.4.3 with following exceptions:

-	Table 6.5.6.1.2.4.3-3 is replaced by Table 16.5.3.1.2.4.3-1.

-	Table 6.5.6.1.2.4.3-4 is replaced by Table 16.5.3.1.2.4.3-2.

Table 16.5.3.1.2.4.3-1: BWP-Downlink (Table 6.5.6.1.2.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 RedCap |  | BWP2 |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon | Table 6.5.6.1.2.4.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.5.3.1.2.4.3-2: BWP-Uplink (Table 6.5.6.1.2.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.1 | BWP-1 | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.1 RedCap | BWP-2 | BWP2 |
| } |  |  |  |
| } |  |  |  |

16.5.3.1.2.5	Test Requirement

Tables 16.5.3.1.2.4.1-3 and 16.5.3.1.2.5-1 define the primary level settings including test tolerances.

Table 16.5.3.1.2.5-1: Cell specific test parameters for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1,4 |  | FDD |
|  |  | Config 2,3 |  | TDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
|  |  | Config 4 |  | 10 MHz: NRB,c = 52 |
| Active BWP ID |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  | Config 1,2,3,4 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 RedCap Note 4 |
| Initial UL BWP Configuration |  | Config 1,2,3,4 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  | Config 1,4 |  | N/A |
|  |  | Config 2,3 |  | ULBWP.1.1 RedCap Note 4 |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  | CCR.1.2 FDD |
|  |  | Config 2 |  | CCR.1.2 TDD |
|  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 | Config 1,2,4 |  | dBm/SCS | -104 |
|  | Config 3 |  |  | -101 |
| NocNote 2 |  |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2,4 |  | dBm/SCS | -87 |
|  | Config 3 |  |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4 | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.1 RedCap is linked with ULBWP.1.1 RedCap defined in clause 12 of TS 38.213 [8]. |  |  |  |  |

During T1, the UE shall start to send the ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 16.5.3.1.0.1-1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### 16.5.3.2	RRC-based active BWP switch for RedCap

##### 16.5.3.2.0	Minimum conformance requirements

16.5.3.2.0.1	Minimum conformance requirements for RRC-based active BWP switch for RedCap

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where

DL slot n is the last slot overlapping with the PDSCH containing the RRC command, and

NR Slot length is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch if the BWP switch involves changing of SCS.

TRRCprocessingDelay is the length of the RRC procedure delay in ms as defined in clause 11.2 in TS 36.331 [29] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the length of the RRC procedure delay in ms as defined in clause 12 in TS 38.331 [13], and

TBWPswitchDelayRRC=6ms is the time used by the UE to perform BWP switch.

The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC on the cell where RRC-based BWP switch occurs. When  THARQ> TRRCprocessingDelay a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [8].

The normative reference for this requirement is TS 38.133 [6] clauses 8.6A.3.

##### 16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

16.5.3.2.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in TS 38.133 [6] clause 8.6A.

16.5.3.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards supporting BWP adaptation of at least 2 BWPs.

16.5.3.2.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.3.0.1 and 16.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.3.2.1.

16.5.3.2.1.4	Test description

Same as the test description given in clause 6.5.6.2.1.4.

16.5.3.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.6.1.2.4.1 with following exceptions:

-	Table 6.5.6.2.1.4.1-1 is replaced by Table 16.5.3.2.1.4.1-1.

-	Table 6.5.6.2.1.4.1-2 is replaced by Table 16.5.3.2.1.4.1-2.

-	Table 6.5.6.2.1.4.1-3 is replaced by Table 16.5.3.2.1.4.1-3.

Table 16.5.3.2.1.4.1-1: Supported test configurations for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.3.2.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.3.2.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.3.2.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.3.2.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.3.2.1.4.1-2: Initial conditions for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.3.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.3.2.1.4.1-3: General test parameters for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

16.5.3.2.1.4.2	Test Procedure

Same as the test procedure given in clause 6.5.6.2.1.4.2.

16.5.3.2.1.4.3	Message Contents

Same as the message contents given in clause 6.5.6.2.1.4.3.

16.5.3.2.1.5	Test Requirement

Tables 16.5.3.2.1.4.1-3 and 16.5.3.2.1.5-1 define the primary level settings including test tolerances.

Table 16.5.3.2.1.5-1: Cell specific test parameters for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1,4 |  | FDD |
|  |  | Config 2,3 |  | TDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,4 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
| Active BWP ID |  |  |  | 1 |
| Initial DL BWP Configuration |  | Config 1,2, 3,4 |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  | Config 1,2, 3,4 |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration | Config 1, 2, 3,4 |  | DLBWP.1.1 RedCap |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3,4 |  | ULBWP.1.1 RedCap |
| Final Condition | Active DL BWP-1 Configuration | Config 1, 2, 3,4 |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3,4 |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  | CCR.1.2 FDD |
|  |  | Config 2 |  | CCR.1.2 TDD |
|  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  | 1x1 Low |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1,2 | dBm/SCS | -104 |
|  |  | Config 3 |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 | dBm/SCS | -87 |
|  |  | Config 3 |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.1 RedCap is linked with ULBWP.1.1 RedCap defined in clause 12 of TS 38.213 [8]. |  |  |  |  |

During T1, the UE shall be ready for the reception of uplink grant for the Cell from the first DL slot that occurs right after the beginning of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

##### 16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

16.5.3.2.2.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in TS 38.133 [6] clause 8.6A.

16.5.3.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards supporting BWP adaptation of at least 2 BWPs.

16.5.3.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.3.2.2.

16.5.3.2.2.4	Test description

Same as the test description given in clause 6.5.6.2.1.4.

16.5.3.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.6.2.1.4.1 with following exceptions:

-	Table 6.5.6.2.1.4.1-1 is replaced by Table 16.5.3.2.2.4.1-1.

-	Table 6.5.6.2.1.4.1-2 is replaced by Table 16.5.3.2.2.4.1-2.

-	Table 6.5.6.2.1.4.1-3 is replaced by Table 16.5.3.2.2.4.1-3.

Table 16.5.3.2.2.4.1-1: Supported test configurations for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.3.2.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.3.2.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.3.2.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.3.2.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.3.2.2.4.1-2: Initial conditions for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.3.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.3.2.2.4.1-3: General test parameters for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| PDCCH and PDSCH maximum number of HARQ transmission |  | 1 |  |
| T1 | s | 0.2 |  |

16.5.3.2.2.4.2	Test Procedure

Same as the test procedure given in clause 6.5.6.2.1.4.2.

16.5.3.2.2.4.3	Message Contents

Same as the message contents given in clause 6.5.6.2.1.4.3 with following exceptions:

-	Table 6.5.6.2.1.4.3-1F is replaced by Table 16.5.3.2.2.4.3-1.

-	Table 6.5.6.2.1.4.3-1G is replaced by Table 16.5.3.2.2.4.3-2.

Table 16.5.3.2.2.4.3-1: BWP-Downlink (Table 6.5.6.2.1.4.3-1E)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 RedCap |  | Step 3 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | Step 5 |
| } |  |  |  |
| } |  |  |  |

Table 16.5.3.2.2.4.3-2: BWP-Uplink (Table 6.5.6.2.1.4.3-1E)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.1 RedCap |  | Step 3 |
|  | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.1 |  | Step 5 |
| } |  |  |  |
| } |  |  |  |

16.5.3.2.2.5	Test Requirement

Tables 16.5.3.2.2.4.1-3 and 16.5.3.2.2.5-1 define the primary level settings including test tolerances.

Table 16.5.3.2.2.5-1: Cell specific test parameters for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1,4 |  | FDD |
|  |  | Config 2,3 |  | TDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,4 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
| Active BWP ID |  |  |  | 1 |
| Initial DL BWP Configuration |  | Config 1,2, 3,4 |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  | Config 1,2, 3,4 |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration | Config 1, 2, 3,4 |  | DLBWP.1.1 RedCap |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3,4 |  | ULBWP.1.1 RedCap |
| Final Condition | Active DL BWP-1 Configuration | Config 1, 2, 3,4 |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3,4 |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  | CCR.1.2 FDD |
|  |  | Config 2 |  | CCR.1.2 TDD |
|  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1,2,4 | dBm/SCS | -104 |
|  |  | Config 3 |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2,4 | dBm/SCS | -87 |
|  |  | Config 3 |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4 | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.2 RedCap is linked with ULBWP.1.1 RedCap defined in clause 12 of TS 38.213 [8]. |  |  |  |  |

During T1, the UE shall be ready for the reception of uplink grant for the Cell from the first DL slot that occurs right after the beginning of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

### 16.5.4	UE specific CBW change for RedCap

#### 16.5.4.0	Minimum conformance requirements

##### 16.5.4.0.1	Minimum conformance requirements for UE specific CBW change

The requirements in this clause apply for a RedCap UE receives reconfiguration of offsetToCarrier or carrierBandwidth to change channel bandwidth.

After the UE receives RRC reconfiguration involving offsetToCarrier or carrierBandwidth change on the old CBW, UE shall be able to receive PDSCH/PDCCH on the DL BWP with BWP ID firstActiveDownlinkBWP-Id or transmit PUSCH on the UL BWP with BWP ID firstActiveUplinkBWP-Id  of the new CBW right after a time duration of TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where

-	DL slot n is the last slot overlapping with the PDSCH containing the RRC command, and

-	TRRCprocessingDelay is the length of the RRC procedure delay in millisecond as defined in TS 36.331 [29] clause 11.2 is the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the length of the RRC procedure delay in millisecond as defined in TS 38.331 [13] clause 12, and

-	TCBWchangeDelayRRC=6 ms is the time used by the UE to perform CBW change.

The UE is not required to transmit UL signals or receive DL signals during the above defined time duration TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length on the cell where UE-specific CBW change occurs. When THARQ> TRRCprocessingDelay a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [8].

The normative reference for this requirement is TS 38.133 [6] clause 8.13A.1 and 8.13A.2.

#### 16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE

16.5.4.1.1	Test purpose

To verify the UE specific CBW change delay requirement defined in 38.133 [6] clause 8.13A.

16.5.4.1.2	Test applicability

This test applies to all types of NR RedCap UEs with 1 Rx from Release 17 onwards.

16.5.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.4.1.

16.5.4.1.4	Test description

Same as the test description given in clause 6.5.8.1.

16.5.4.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.8.1 with following exceptions:

-	Table 6.5.8.1.4.1-1 is replaced by Table 16.5.4.1.4.1-1.

-	Table 6.5.8.1.4.1-2 is replaced by Table 16.5.4.1.4.1-2.

-	Table 6.5.8.1.4.1-3 is replaced by Table 16.5.4.1.4.1-3.

Table 16.5.4.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.4.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.4.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.4.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.4.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.5.4.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

16.5.4.1.4.2	Test procedure

Same as the test procedure given in clause 6.5.8.1.4.2.

16.5.4.1.4.3	Message contents

Same as the message contents given in clause 6.5.8.1.4.2.

16.5.4.1.5	Test requirement

Same as the test requirements given in clause 6.5.8.1.5 with followings exceptions:

-	Table 6.5.8.1.5-1 is replaced by Table 16.5.4.1.5-1.

Table 16.5.4.1.5-1: NR Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,2,4 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
| Active DL BWP ID |  |  |  | 0 |
| Initial DL BWP Configuration (BWP-1) |  |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration (BWP-1) |  |  |  | ULBWP.0.2 |
| Initial Condition | Active DL CBW Configuration (CBW-1) |  |  | DLCBW.1.1 |
|  | Active UL CBW Configuration (CBW-1) |  |  | ULCBW.1.1 |
| Final Condition | Active DL CBW Configuration (CBW-2) |  |  | DLCBW.1.2 |
|  | Active UL CBW Configuration (CBW-2) |  |  | ULCBW.1.2 |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 RedCap FR1 |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  | 1x1 |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1,2,4 | dBm/SCS | -104 |
|  |  | Config 3 |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2,4 | dBm/SCS | -87 |
|  |  | Config 3 |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4 | dBm/9.36MHz | -58.96 |
|  |  | Config 3 | dBm/18.36MHz | -56.04 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.1 is linked with ULBWP.0.1; DLBWP.1.1 is linked with ULBWP.1.1; as defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

#### 16.5.4.2	NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE

16.5.4.2.1	Test purpose

To verify the UE specific CBW change delay requirement defined in 38.133 [6] clause 8.13A.

16.5.4.2.2	Test applicability

This test applies to all types of NR RedCap UEs with 2 Rx from Release 17 onwards.

16.5.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.4.2.

16.5.4.2.4	Test description

Same as the test description given in clause 6.5.8.1.

16.5.4.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.8.1 with following exceptions:

-	Table 6.5.8.1.4.1-1 is replaced by Table 16.5.4.2.4.1-1.

-	Table 6.5.8.1.4.1-2 is replaced by Table 16.5.4.2.4.1-2.

-	Table 6.5.8.1.4.1-3 is replaced by Table 16.5.4.2.4.1-3.

Table 16.5.4.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.4.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.4.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.4.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.4.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.4.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.4.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.5.4.2.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

16.5.4.2.4.2	Test procedure

Same as the test procedure given in clause 6.5.8.1.4.2.

16.5.4.2.4.3	Message contents

Same as the message contents given in clause 6.5.8.1.4.2.

16.5.4.2.5	Test requirement

Same as the test requirements given in clause 6.5.8.1.5 with followings exceptions:

-	Table 6.5.8.1.5-1 is replaced by Table 16.5.4.2.5-1.

Table 16.5.4.2.5-1: NR Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,2,4 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
| Active DL BWP ID |  |  |  | 0 |
| Initial DL BWP Configuration (BWP-1) |  |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration (BWP-1) |  |  |  | ULBWP.0.2 |
| Initial Condition | Active DL CBW Configuration (CBW-1) |  |  | DLCBW.1.1 |
|  | Active UL CBW Configuration (CBW-1) |  |  | ULCBW.1.1 |
| Final Condition | Active DL CBW Configuration (CBW-2) |  |  | DLCBW.1.2 |
|  | Active UL CBW Configuration (CBW-2) |  |  | ULCBW.1.2 |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 RedCap FR1 |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  | 1x2 low |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1,2,4 | dBm/SCS | -104 |
|  |  | Config 3 |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2,4 | dBm/SCS | -87 |
|  |  | Config 3 |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4 | dBm/9.36MHz | -58.96 |
|  |  | Config 3 | dBm/18.36MHz | -56.04 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.1 is linked with ULBWP.0.1; DLBWP.1.1 is linked with ULBWP.1.1; as defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

## 16.6	Measurement procedures for RedCap

### 16.6.1	Intra-frequency measurements for RedCap

#### 16.6.1.0	Minimum conformance requirements

##### 16.6.1.0.1	General

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig.

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE can perform intra-frequency SSB based measurements without measurement gaps if

-	the SSB is completely contained in the active BWP of the UE, or

-	the active downlink BWP is initial BWP [3].

For intra-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in 38.133 [6] clause 9.2B.5.3.

SSB based measurements are configured along with one or two measurement timing configuration(s) (SMTC(s)) which provides periodicity, duration and offset information on a window of up to 5ms where the measurements are to be performed. For intra-frequency connected mode measurements, up to two measurement window periodicities may be configured. A single measurement window offset and measurement duration are configured per intra-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. Switching time is 0.5ms for frequency range FR1.

The requirements in this clause apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	For 2Rx RedCap:

-	SS-RSRP related side conditions given in 38.133 [6] clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in 38.133 [6] clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in 38.133 [6] clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

-	For 1Rx RedCap:

-	SS-RSRP related side conditions given in clauses [x.y.z] and [x.y.z] for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses [x.y.z] and [x.y.z] for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses [x.y.z] and [x.y.z] for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex [x.y.z] for a corresponding Band.

For 2Rx RedCap: The requirements in 38.133 [6] clause 9.2.4.3 shall apply.

For 1Rx RedCap: Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in 38.133 [6] clauses 10.1.2.1 (RSRP for FR1), 10.1.7.1 (RSRQ for FR1), 10.1.12.1 (RS-SINR for FR1).

The UE shall not send any event triggered measurement reports as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index_RedCap or T identify intra without index_RedCap defined in clause 16.6.1.0.2 or clause 16.6.1.0.3. When L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index_RedCap or Tidentify_intra_with_index_RedCap as defined in clause 16.6.1.0.2 or clause 16.6.1.0.3. If a cell which has been detectable at least for the time period Tidentify intra without index_RedCap or Tidentify intra with index_RedCap defined in clause 16.6.1.0.2 or clause 16.6.1.0.3 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra_RedCap provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2B.1 and 9.2B.4.3.

##### 16.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index_RedCap if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_RedCap. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_RedCap. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD.

Tidentify_intra_without_index_RedCap = (TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap) ms

Tidentify_intra_with_index_RedCap = (TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TSSB_time_index_intra_RedCap) ms

Where:

TPSS/SSS_sync_intra_RedCap: it is the time period used in PSS/SSS detection given in Table 16.6.1.0.2-1 for 2RX RedCap and Table 16.6.1.0.2-2 for 1Rx RedCap.

TSSB_time_index_intra_RedCap: it is the time period used to acquire the index of the SSB being measured given in Table 16.6.1.0.2-3 for 2RX RedCap and Table 16.6.1.0.2-4 for 1RX RedCap.

T SSB_measurement_period_intra_RedCap: equal to a measurement period of SSB based measurement given in Table 16.6.1.0.2-5 for 2RX RedCap and Table 16.6.1.0.2-6 for 1RX RedCap.

CSSFintra_RedCap: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap_RedCap,i in 38.133 [6] clause 9.1A.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps.

When intra-frequency SMTC is fully non overlapping with measurement gaps, Kp=1.

Table 16.6.1.0.2-1: Time period for PSS/SSS detection, (Frequency range FR1) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max( 600ms, ceil(5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.1.0.2-2: Time period for PSS/SSS detection, (Frequency range FR1) for 1Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max( 600ms, ceil( 7 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max( 600ms, ceil(7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(7 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.1.0.2-3: Time period for time index detection (FR1) for 2Rx RedCap UE

| DRX cycle | TSSB_time_index_intra_RedCap |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(120ms, ceil (3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.1.0.2-4: Time period for time index detection (FR1) for 1Rx RedCap UE

| DRX cycle | TSSB_time_index_intra_RedCap |
| --- | --- |
| No DRX | max(160ms, ceil( [6] x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(160ms, ceil ([6] x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | Ceil([6] x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.1.0.2-5: Measurement period for intra-frequency measurements without gaps (FR1) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.1.0.2-6: Measurement period for intra-frequency measurements without gaps (FR1) for 1Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max([200]ms, ceil( [5] x Kp) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max([200]ms, ceil(1.5x [5] x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil( [5] x Kp ) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a HD-FDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a HD-FDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2B.5.

##### 16.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index_RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_RedCap. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_RedCap. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD.

Tidentify_intra_without_index_RedCap = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap  ms

Tidentify_intra_with_index_RedCap = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TSSB_time_index_intra_RedCap ms

Where:

TPSS/SSS_sync_intra_RedCap: it is the time period used in PSS/SSS detection given in Table 16.6.1.0.3-1 for 2RX RedCap or Table 16.6.1.0.3-2 for 1RX RedCap.

TSSB_time_index_intra_RedCap: it is the time period used to acquire the index of the SSB being measured given in Table 16.6.1.0.3-3 for 2RX RedCap or Table 16.6.1.0.3-4 for 1RX RedCap.

T SSB_measurement_period_intra_RedCap: equal to a measurement period of SSB based measurement given in Table 16.6.1.0.3-5 for 2RX RedCap or Table 16.6.1.0.3-6 for 1RX RedCap.

CSSFintra_RedCap: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_RedCap,i in 38.133 [6] clause 9.1A.5.2 for measurement conducted within measurement gaps.

Table 16.6.1.0.3-1: Time period for PSS/SSS detection (FR1) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max(600ms, 5 x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, 5 x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 16.6.1.0.3-2: Time period for PSS/SSS detection (FR1) for 1Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max(600 ms, 7 x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600 ms, 7 x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 16.6.1.0.3-3: Time period for time index detection (Frequency range FR1) for 2Rx RedCap UE

| DRX cycle | TSSB_time_index_intra_RedCap |
| --- | --- |
| No DRX | max(120ms, 3 x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(120ms, 3 x max(MGRP, SMTC period,DRX cycle) x CSSFintra_RedCap) |
| DRX cycle>320ms | 3 x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 16.6.1.0.3-4: Time period for time index detection (Frequency range FR1) for 1Rx RedCap UE

| DRX cycle | TSSB_time_index_intra_RedCap |
| --- | --- |
| No DRX | max(160ms, [6] x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(160ms, [6] x max(MGRP, SMTC period,DRX cycle) x CSSFintra_RedCap) |
| DRX cycle>320ms | [6] x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 16.6.1.0.3-5: Measurement period for intra-frequency measurements with gaps (FR1) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max(200ms, 5 x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 16.6.1.0.3-6: Measurement period for intra-frequency measurements with gaps (FR1) for 1Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max([200] ms, [5] x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max([200] ms, ceil(1.5x [5]) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | [5] x max(MGRP, DRX cycle) x CSSFintra_RedCap |

The normative reference for this requirement is TS 38.133 [6] clauses 9.2B.6.

#### 16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE

16.6.1.1.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2B.5.1 and 9.2B.5.2.

16.6.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.1.

16.6.1.1.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.1.1.4.1-3 and Table 16.6.1.1.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the PCell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.1.1.4.1-1.

Table 16.6.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.1.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.1.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.1.1.4.1-2.

Table 16.6.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 16.6.1.1.4.1-3 below.

2. Message contents are defined in clause 16.6.1.1.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 16.6.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| RF Channel Number |  |  | 1: Cell 1 and Cell 2 |  |
| CD-SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |
| NCD-SSB configuration | Config 1,2,4 |  | SSB.6 RedCap FR1 Note 1 |  |
|  | Config 3 |  | SSB.7 RedCap FR1 Note 1 |  |
| SMTC configuration for NCD-SSB | Config 1,4 |  | SMTC.4 RedCap |  |
|  | Config 2,3 |  | SMTC.2 RedCap |  |
| A3-Offset |  | dB | -4.5 |  |
| CP length |  |  | Normal |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | N/A | OFF |
| Time offset between serving and neighbour cells | Config 1,4 | ms | 3 | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  | Config 2,3 | s | 3 | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP. |  |  |  |  |

16.6.1.1.4.2	Test procedure

The test consists of two successive time periods with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1 and RedCap-specific dedicated BWP is the activated BWP. The SS transmits NCD-SSB on Cell 1 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

2. Set the parameters according to T1 in Table 16.6.1.1.5-1.

3. Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4. SS shall transmit an RRCReconfiguration message to configure Event A3 triggered reporting.

5. The UE shall transmit an RRCReconfigurationComplete message. T1 starts upon the SS receives the RRCReconfigurationComplete message.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.6.1.1.5-1. T2 starts. The SS also transmits NCD-SSB on Cell 2 with an absolute frequency of fNCDSSB calculated above.

7. The SS increases the number of successful tests by one if the UE transmits a MeasurementReport message triggered by event A3 satisfying the overall delay requirement specified in clause 16.6.1.1.5, otherwise the SS increases the number of failure tests by one.

8. After the SS receive the MeasurementReport message in step 7) or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

or

-	switch the UE off.

9. Depending on the choice in step 8, the SS shall,

-	transmit in NR Cell 1 a Paging message including matched ng-5G-S-TMSI for the UE if an RRCRelease has been sent in step 8. If the paging fails, switches off and on the UE,

or

-	switch on the UE if the UE is switched off in step 8.

10. Repeat step 1-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.6.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM |

Table 16.6.1.1.4.3-2: MeasObjectNR (step 4, test procedure)

| Derivation Path: Table H.3.1-3 with condition INTRA-FREQ MO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of NCD-SSB |  |  |
| smtc1 | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap | SMTC.2 RedCap | Config 1,4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.4 RedCap | SMTC.4 RedCap | Config 2,3 |
| } |  |  |  |

Table 16.6.1.1.4.3-3: ServingCellConfig (step 1, test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.6.1.1.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.6.1.1.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-4: BWP-Downlink (Table 16.6.1.1.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| servingCellMO-r17 | 1 | MeasObjectId associated to NCD-SSB |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-5: BWP-Uplink (Table 16.6.1.1.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.6.1.1.5	Test requirement

Table 16.6.1.1.4.1-3 and Table 16.6.1.1.5-1 define the primary level settings including test tolerances.

Table 16.6.1.1.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under non-DRX

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration | Config 1,4 |  | N/A |  | N/A |  |
|  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration | Config 1,4 |  | SR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration | Config 1,4 |  | CR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration | Config 1,4 |  | CCR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  | OP.1 |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |  | N/A |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| Initial DL BWP configuration |  |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| Active DL BWP configuration |  |  | DLBWP.1.3 RedCap |  | DLBWP.1.3 RedCap |  |
| Active UL BWP configuration |  |  | ULBWP.1.3 RedCap |  | ULBWP.1.3 RedCap |  |
| RLM-RS |  |  | SSB |  | SSB |  |
| Noc Note 2 | Config 1,2,4 | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  | -95 |  |  |  |
| Noc Note 2 |  | dBm/15 KHz | -98 |  |  |  |
| Ȇs/Iot |  | dB | 4 | -1.46 | -Infinity | -1.46 |
| Ȇs/Noc |  | dB | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | Config 1,2,4 | dBm/SCS | -94 | -94 | -Infinity | -94 |
|  | Config 3 |  | -91 | -91 | -Infinity | -91 |
| Io | Config 1,2,4 | dBm/9.36 MHz | -64.60 | -62.25 | -64.60 | -62.25 |
|  | Config 3 | dBm/18.36 MHz | -61.68 | -59.34 | -61.68 | -59.34 |
| Propagation Condition |  |  | AWGN |  |  |  |
| Antenna Configuration |  |  | 1x1 |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delay is measured from the beginning of time period T2 to the time that the SS receives the first MeasurementReport message triggered by event A3. It is allowed to be up to 2xTTIDCCH higher than the measurement reporting delay specified in 38.133[6] clause 9.2B.4 due to the TTI insertion uncertainty of the measurement report in DCCH.

Then the overall delay shall be no larger than: Tidentify_intra_without_index + 2xTTIDCCH, where

-	Tidentify_intra_without_index = 1000 ms, is the time for intra-frequency cell identification specified in 38.133 [6] clause 9.2B.5.1,

-	TTIDCCH = 1 ms.

Which gives a total of 1002 ms.

The UE is not required to read the neighbour cell SSB index in this test.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 16.6.1.2	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE

16.6.1.2.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2B.5.1 and 9.2B.5.2.

16.6.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.2.

16.6.1.2.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.1.2.4.1-3 and Table 16.6.1.2.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the PCell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.1.2.4.1-1.

Table 16.6.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.1.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.1.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.1.2.4.1-2.

Table 16.6.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 16.6.1.2.4.1-3 below.

2. Message contents are defined in clause 16.6.1.2.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 16.6.1.2.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| RF Channel Number |  |  | 1: Cell 1 and Cell 2 |  |
| CD-SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |
| NCD-SSB configuration | Config 1,2,4 |  | SSB.6 RedCap FR1 Note 1 |  |
|  | Config 3 |  | SSB.7 RedCap FR1 Note 1 |  |
| SMTC configuration for NCD-SSB | Config 1,4 |  | SMTC.4 RedCap |  |
|  | Config 2,3 |  | SMTC.2 RedCap |  |
| A3-Offset |  | dB | -4.5 |  |
| CP length |  |  | Normal |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | N/A | OFF |
| Time offset between serving and neighbour cells | Config 1,4 | ms | 3 | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  | Config 2,3 | s | 3 | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP. |  |  |  |  |

16.6.1.2.4.2	Test procedure

The test consists of two successive time periods with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1 and RedCap-specific dedicated BWP is the activated BWP. The SS transmits NCD-SSB on Cell 1 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

2. Set the parameters according to T1 in Table 16.6.1.2.5-1.

3. Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4. SS shall transmit an RRCReconfiguration message to configure Event A3 triggered reporting.

5. The UE shall transmit an RRCReconfigurationComplete message. T1 starts upon the SS receives the RRCReconfigurationComplete message.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.6.1.2.5-1. T2 starts. The SS also transmits NCD-SSB on Cell 2 with an absolute frequency of fNCDSSB calculated above.

7. The SS increases the number of successful tests by one if the UE transmits a MeasurementReport message triggered by event A3 satisfying the overall delay requirement specified in clause 16.6.1.2.5, otherwise the SS increases the number of failure tests by one.

8. After the SS receive the MeasurementReport message in step 7) or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

or

-	switch the UE off.

9. Depending on the choice in step 8, the SS shall,

-	transmit in NR Cell 1 a Paging message including matched ng-5G-S-TMSI for the UE if an RRCRelease has been sent in step 8. If the paging fails, switches off and on the UE,

or

-	switch on the UE if the UE is switched off in step 8.

10. Repeat step 1-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.6.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM |

Table 16.6.1.2.4.3-2: MeasObjectNR (step 4, test procedure)

| Derivation Path: Table H.3.1-3 with condition INTRA-FREQ MO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of NCD-SSB |  |  |
| smtc1 | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap | SMTC.2 RedCap | Config 2,3 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.4 RedCap | SMTC.4 RedCap | Config 1,4 |
| } |  |  |  |

Table 16.6.1.2.4.3-3: ServingCellConfig (step 1, test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.6.1.2.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.6.1.2.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 16.6.1.2.4.3-4: BWP-Downlink (Table 16.6.1.2.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| servingCellMO-r17 | 1 | MeasObjectId associated with NCD-SSB |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.2.4.3-5: BWP-Uplink (Table 16.6.1.2.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.6.1.2.5	Test requirement

Table 16.6.1.2.4.1-3 and Table 16.6.1.2.5-1 define the primary level settings including test tolerances.

Table 16.6.1.2.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under non-DRX

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration | Config 1,4 |  | N/A |  | N/A |  |
|  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration | Config 1,4 |  | SR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration | Config 1,4 |  | CR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration | Config 1,4 |  | CCR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  | OP.1 |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |  | N/A |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| Initial DL BWP configuration |  |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| Active DL BWP configuration |  |  | DLBWP.1.3 RedCap |  | DLBWP.1.3 RedCap |  |
| Active UL BWP configuration |  |  | ULBWP.1.3 RedCap |  | ULBWP.1.3 RedCap |  |
| RLM-RS |  |  | SSB |  | SSB |  |
| Noc Note 2 | Config 1,2,4 | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  | -95 |  |  |  |
| Noc Note 2 |  | dBm/15 KHz | -98 |  |  |  |
| Ȇs/Iot |  | dB | 4 | -1.46 | -Infinity | -1.46 |
| Ȇs/Noc |  | dB | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | Config 1,2,4 | dBm/SCS | -94 | -94 | -Infinity | -94 |
|  | Config 3 |  | -91 | -91 | -Infinity | -91 |
| Io | Config 1,2,4 | dBm/9.36 MHz | -64.60 | -62.25 | -64.60 | -62.25 |
|  | Config 3 | dBm/18.36 MHz | -61.68 | -59.34 | -61.68 | -59.34 |
| Propagation Condition |  |  | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delay is measured from the beginning of time period T2 to the time that the SS receives the first MeasurementReport message triggered by event A3. It is allowed to be up to 2xTTIDCCH higher than the measurement reporting delay specified in 38.133[6] clause 9.2B.4 due to the TTI insertion uncertainty of the measurement report in DCCH.

Then the overall delay shall be shall be no larger than: Tidentify_intra_without_index + 2xTTIDCCH, where

-	Tidentify_intra_without_index = 1000 ms, is the time for intra-frequency cell identification specified in 38.133 [6] clause 9.2B.5.1,

-	TTIDCCH = 1 ms.

Which gives a total of 1002 ms.

The UE is not required to read the neighbour cell SSB index in this test.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 16.6.1.3	NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE

16.6.1.3.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2B.5.1 and 9.2B.5.2.

16.6.1.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 onwards and supporting long DRX cycle.

16.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.3.

16.6.1.3.4	Test description

16.6.1.3.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.2.4.1 with following exceptions:

-	Table 6.6.1.2.4.1-1 is replaced by Table 16.6.1.3.4.1-1.

-	Table 6.6.1.2.4.1-2 is replaced by Table 16.6.1.3.4.1-2.

-	Table 6.6.1.2.4.1-3 is replaced by Table 16.6.1.3.4.1-3.

Table 16.6.1.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.3-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 16.6.1.3-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 16.6.1.3-3 | 30 kHz SSB SCS, 20MHz bandwidth, TDD duplex mode |
| 16.6.1.3-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.6.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.1.3.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1,2,3,4 | Cell 1 |  |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |  |
|  |  | 2,3 | SMTC.1 |  |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |  |
| CP length |  | 1,2,3,4 | Normal |  |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2,3 | 3 us |  | Synchronous cells |
| T1 | s | 1,2,3,4 | 5 |  |  |
| T2 | s | 1,2,3,4 | 5 | 10 |  |

16.6.1.3.4.2	Test procedure

Same as the test procedure given in 6.6.1.2.4.2 with following exceptions:

-	Step 6 in 6.6.1.2.4.2 is replaced by following step.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 7682 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

16.6.1.3.4.3	Message contents

Same as the message contents given in 6.6.1.2.4.3.

16.6.1.3.5	Test requirement

Same as the test requirement given in 6.6.1.2.5 with following exceptions:

-	Table 6.6.1.2.5-1 is replaced by Table 16.6.1.3.5-1.

-	The overall delay measured is expressed as:

Overall delays measured = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TTI insertion uncertainty

-	TPSS/SSS_sync_intra_RedCap = 600 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 4480 ms for Test 2;

-	T SSB_measurement_period_intra_RedCap= 320 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2

-	TTI insertion uncertainty = 2 ms

As a result, the overall delay measured for Test 1 and Test 2 shall be less than a total of 922 ms and 7682ms respectively.

Table 16.6.1.3.5-1: NR Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/ 9.36 MHz | 1,2,4 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/ 18.36 MHz | 3 | -61.68 | -59.34 |  |  |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### 16.6.1.4	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE

16.6.1.4.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2B.5.1 and 9.2B.5.2.

16.6.1.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 onwards and supporting long DRX cycle.

16.6.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.4.

16.6.1.4.4	Test description

16.6.1.4.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.2.4.1 with following exceptions:

-	Table 6.6.1.2.4.1-1 is replaced by Table 16.6.1.4.4.1-1.

-	Table 6.6.1.2.4.1-2 is replaced by Table 16.6.1.4.4.1-2.

-	Table 6.6.1.2.4.1-3 is replaced by Table 16.6.1.4.4.1-3.

Table 16.6.1.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.4-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 16.6.1.4-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 16.6.1.4-3 | 30 kHz SSB SCS, 20MHz bandwidth, TDD duplex mode |
| 16.6.1.4-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.6.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.6.1.4.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1,2,3,4 | Cell 1 |  |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |  |
|  |  | 2,3 | SMTC.1 |  |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |  |
| CP length |  | 1,2,3,4 | Normal |  |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2,3 | 3 us |  | Synchronous cells |
| T1 | s | 1,2,3,4 | 5 |  |  |
| T2 | s | 1,2,3,4 | 5 | 10 |  |

16.6.1.4.4.2	Test procedure

Same as the test procedure given in 6.6.1.2.4.2.

16.6.1.4.4.3	Message contents

Same as the message contents given in 6.6.1.2.4.3.

16.6.1.4.5	Test requirement

Same as the test requirement given in 6.6.1.2.5 with following exceptions:

-	Table 6.6.1.2.5-1 is replaced by Table 16.6.1.4.5-1.

-	The overall delay measured is expressed as:

Overall delays measured = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TTI insertion uncertainty

-	TPSS/SSS_sync_intra_RedCap = 600 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2;

-	T SSB_measurement_period_intra_RedCap= 320 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2

-	TTI insertion uncertainty = 2 ms

As a result, the overall delay measured for Test 1 and Test 2 shall be less than a total of 922 ms and 6402 ms respectively.

Table 16.6.1.4.5-1: NR Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/ 9.36 MHz | 1,2,4 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/ 18.36 MHz | 3 | -61.68 | -59.34 |  |  |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### 16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE

16.6.1.5.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clause 9.2B.6.1 and 9.2B.6.2.

16.6.1.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting CSI-RS based RLM and BWP operation without bandwidth restriction.

16.6.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.5.

16.6.1.5.4	Test description

16.6.1.5.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.3.4.1 with following exceptions:

-	Table 6.6.1.3.4.1-1 is replaced by Table 16.6.1.5.4.1-1.

-	Table 6.6.1.3.4.1-2 is replaced by Table 16.6.1.5.4.1-2.

-	Table 6.6.1.3.4.1-3 is replaced by Table 16.6.1.5.4.1-3.

Table 16.6.1.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.5-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.5-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.1.5-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.1.5-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.6.1.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.1.5.4.1-3: General test parameters

| Parameter | Unit | Test config. | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2,3,4 | Cell 1 |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1,2,3,4 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1,2,3,4 | 40 |  |
| Measurement gap length | ms | 1,2,3,4 | 6 |  |
| Measurement gap offset | ms | 1,2,3,4 | 39 |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |
|  |  | 2,3 | SMTC.1 |  |
| CSI-RS parameters |  | 1,4 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |
| CP length |  | 1,2,3,4 | Normal |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |
| Filter coefficient |  | 1,2,3,4 | 0 | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | N/A | OFF |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2,3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

16.6.1.5.4.2	Test procedure

Same as the test procedure given in clause 6.6.1.3.4.2.

16.6.1.5.4.3	Message contents

Same as the message contents given in clause 6.6.1.3.4.3 with the following exception:

-	For test configuration 16.6.1.5-1 and 16.6.1.5-4,

-	Table H.3.1-3 is used with condition INTRA-FREQ MO.

-	Table 7.3.1-3 specified in 38.508-1 [14] is used with condition SMTC.2.

-	For test configuration 16.6.1.5-2 and 16.6.1.5-3,

-	Table H.3.1-3 is used with condition INTRA-FREQ MO and Synchronous cells.

-	Table 7.3.1-3 specified in 38.508-1 [14] is used with condition SMTC.1.

16.6.1.5.5	Test requirement

Same as the test requirements given in clause 6.6.1.3.5 with following exceptions:

-	Table 6.6.1.3.5-1 is replaced by Table 16.6.1.5.5-1.

Table 16.6.1.5.5-1: Cell specific test parameters

| Parameter | Unit | Test config. | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  |
|  |  | 3 | CCR.2.3 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.2 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.2 RedCap |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,2,4 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/18.36 MHz | 3 | -61.68 | -59.34 | -61.68 | -59.34 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### 16.6.1.6	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE

16.6.1.6.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clause 9.2B.6.1 and 9.2B.6.2.

16.6.1.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting CSI-RS based RLM and BWP operation without bandwidth restriction.

16.6.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.6.

16.6.1.6.4	Test description

16.6.1.6.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.3.4.1 with following exceptions:

-	Table 6.6.1.3.4.1-1 is replaced by Table 16.6.1.6.4.1-1.

-	Table 6.6.1.3.4.1-2 is replaced by Table 16.6.1.6.4.1-2.

-	Table 6.6.1.3.4.1-3 is replaced by Table 16.6.1.6.4.1-3.

Table 16.6.1.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.6-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.6-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.1.6-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.1.6-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.6.1.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.1.6.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2,3,4 | Cell 1 |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1,2,3,4 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1,2,3,4 | 40 |  |
| Measurement gap length | ms | 1,2,3,4 | 6 |  |
| Measurement gap offset | ms | 1,2,3,4 | 39 |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |
|  |  | 2,3 | SMTC.1 |  |
| CSI-RS parameters |  | 1,4 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |
| CP length |  | 1,2,3,4 | Normal |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |
| Filter coefficient |  | 1,2,3,4 | 0 | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | N/A | OFF |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2,3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

16.6.1.6.4.2	Test procedure

Same as the test procedure given in clause 6.6.1.3.4.2.

16.6.1.6.4.3	Message contents

Same as the message contents given in clause 6.6.1.3.4.3 with the following exception:

-	For test configuration 16.6.1.6-1 and 16.6.1.6-4,

-	Table H.3.1-3 is used with condition INTRA-FREQ MO.

-	Table 7.3.1-3 specified in 38.508-1 [14] is used with condition SMTC.2.

-	For test configuration 16.6.1.6-2 and 16.6.1.6-3,

-	Table H.3.1-3 is used with condition INTRA-FREQ MO and Synchronous cells.

-	Table 7.3.1-3 specified in 38.508-1 [14] is used with condition SMTC.1.

16.6.1.6.5	Test requirement

Same as the test requirements given in clause 6.6.1.3.5 with following exceptions:

-	Table 6.6.1.3.5-1 is replaced by Table 16.6.1.6.5-1.

Table 16.6.1.6.5-1: Cell specific test parameters

| Parameter | Unit | Test config. | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  |
|  |  | 3 | CCR.2.3 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.2 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.2 RedCap |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,2,4 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/18.36 MHz | 3 | -61.68 | -59.34 | -61.68 | -59.34 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### 16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE

16.6.1.7.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2B.6.1 and 9.2B.6.2.

16.6.1.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 onwards and supporting CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle.

16.6.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.7.

16.6.1.7.4	Test description

16.6.1.7.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.4.4.1 with following exceptions:

-	Table 6.6.1.4.4.1-1 is replaced by Table 16.6.1.7.4.1-1.

-	Table 6.6.1.4.4.1-2 is replaced by Table 16.6.1.7.4.1-2.

-	Table 6.6.1.4.4.1-3 is replaced by Table 16.6.1.7.4.1-3.

Table 16.6.1.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.7-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 16.6.1.7-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 16.6.1.7-3 | 30 kHz SSB SCS, 20MHz bandwidth, TDD duplex mode |
| 16.6.1.7-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.6.1.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.1.7.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1,2,3,4 | Cell 1 |  |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1,2,3,4, | Per-UE gaps |  |  |
| Measurement gap repetition periodicity | ms | 1,2,3,4 | 40 |  |  |
| Measurement gap length | ms | 1,2,3,4 | 6 |  |  |
| Measurement gap offset | ms | 1,2,3,4 | 39 |  |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |  |
|  |  | 2,3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1, 4 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |  |
| CP length |  | 1,2,3,4 | Normal |  |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2,3 | 3 us |  | Synchronous cells |
| T1 | s | 1,2,3,4 | 5 |  |  |
| T2 | s | 1,2,3,4 | 5 | 10 |  |

16.6.1.7.4.2	Test procedure

Same as the test procedure given in 6.6.1.4.4.2 with following exceptions:

-	Step 6 in 6.6.1.4.4.2 is replaced by following step.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 7682 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

16.6.1.7.4.3	Message contents

Same as the message contents given in 6.6.1.4.4.3.

16.6.1.7.5	Test requirement

Same as the test requirement given in 6.6.1.4.5 with following exceptions:

-	Table 6.6.1.4.5-1 is replaced by Table 16.6.1.7.5-1.

-	The overall delay measured is expressed as:

Overall delays measured = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TTI insertion uncertainty

-	TPSS/SSS_sync_intra_RedCap = 600 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 4480 ms for Test 2;

-	T SSB_measurement_period_intra_RedCap= 320 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2

-	TTI insertion uncertainty = 2 ms

As a result, the overall delay measured for Test 1 and Test 2 shall be less than a total of 922 ms and 7682ms respectively.

Table 16.6.1.7.5-1: NR Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.3 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.1 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 RedCap |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/ 9.36 MHz | 1,2,4 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/ 18.36 MHz | 3 | -61.68 | -59.34 |  |  |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### 16.6.1.8	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE

16.6.1.8.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2B.6.1 and 9.2B.6.2.

16.6.1.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 onwards and supporting CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle.

16.6.1.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.8.

16.6.1.8.4	Test description

16.6.1.8.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.4.4.1 with following exceptions:

-	Table 6.6.1.4.4.1-1 is replaced by Table 16.6.1.8.4.1-1.

-	Table 6.6.1.4.4.1-2 is replaced by Table 16.6.1.8.4.1-2.

-	Table 6.6.1.4.4.1-3 is replaced by Table 16.6.1.8.4.1-3.

Table 16.6.1.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.8-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 16.6.1.8-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 16.6.1.8-3 | 30 kHz SSB SCS, 20MHz bandwidth, TDD duplex mode |
| 16.6.1.8-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.6.1.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.6.1.8.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1,2,3,4 | Cell 1 |  |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1,2,3,4, | Per-UE gaps |  |  |
| Measurement gap repetition periodicity | ms | 1,2,3,4 | 40 |  |  |
| Measurement gap length | ms | 1,2,3,4 | 6 |  |  |
| Measurement gap offset | ms | 1,2,3,4 | 39 |  |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |  |
|  |  | 2,3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1, 4 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |  |
| CP length |  | 1,2,3,4 | Normal |  |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2,3 | 3 us |  | Synchronous cells |
| T1 | s | 1,2,3,4 | 5 |  |  |
| T2 | s | 1,2,3,4 | 5 | 10 |  |

16.6.1.8.4.2	Test procedure

Same as the test procedure given in 6.6.1.4.4.2.

16.6.1.8.4.3	Message contents

Same as the message contents given in 6.6.1.4.4.3.

16.6.1.8.5	Test requirement

Same as the test requirement given in 6.6.1.4.5 with following exceptions:

-	Table 6.6.1.4.5-1 is replaced by Table 16.6.1.8.5-1.

-	The overall delay measured is expressed as:

Overall delays measured = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TTI insertion uncertainty

-	TPSS/SSS_sync_intra_RedCap = 600 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2;

-	T SSB_measurement_period_intra_RedCap= 320 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2

-	TTI insertion uncertainty = 2 ms

As a result, the overall delay measured for Test 1 and Test 2 shall be less than a total of 922 ms and 6402 ms respectively.

Table 16.6.1.8.5-1: NR Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  |
|  |  | 3 | CCR.2.3 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.2 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.2 RedCap |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/ 9.36 MHz | 1,2,4 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/ 18.36 MHz | 3 | -61.68 | -59.34 |  |  |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### 16.6.1.9	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE

16.6.1.9.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2B.5.1 and 9.2B.5.2.

16.6.1.9.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.1.9.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.9.

16.6.1.9.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for FDD PCell and neighbour cell are given in Table 16.6.1.9.4.1-3 and Table 16.6.1.9.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the PCell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.1.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.1.9.4.1-1.

Table 16.6.1.9.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.9-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.9-2 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.1.9.4.1-2.

Table 16.6.1.9.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.9.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 16.6.1.9.4.1-3 below.

2. Message contents are defined in clause 16.6.1.9.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 16.6.1.9.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1: Cell 1 and Cell 2 |  |
| CD-SSB configuration |  | SSB.1 FR1 |  |
| NCD-SSB configuration |  | SSB.6 RedCap FR1 Note 1 |  |
| SMTC configuration for CD-SSB |  | SMTC.2 |  |
| SMTC configuration for NCD-SSB |  | SMTC.4 RedCap |  |
| A3-Offset | dB | -4.5 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | N/A | OFF |
| Time offset between serving and neighbour cells | ms | 3 | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP. |  |  |  |

16.6.1.9.4.2	Test procedure

The test consists of two successive time periods with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1 and RedCap-specific dedicated BWP is the activated BWP. The SS transmits NCD-SSB on Cell 1 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

2. Set the parameters according to T1 in Table 16.6.1.9.5-1.

3. Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4. SS shall transmit an RRCReconfiguration message to configure Event A3 triggered reporting.

5. The UE shall transmit an RRCReconfigurationComplete message. T1 starts upon the SS receives the RRCReconfigurationComplete message.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.6.1.9.5-1. T2 starts. The SS also transmits NCD-SSB on Cell 2 with an absolute frequency of fNCDSSB calculated above.

7. The SS increases the number of successful tests by one if the UE transmits a MeasurementReport message triggered by event A3 satisfying the overall delay requirement specified in clause 16.6.1.9.5, otherwise the SS increases the number of failure tests by one.

8. After the SS receive the MeasurementReport message in step 7) or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

or

-	switch the UE off.

9. Depending on the choice in step 8, the SS shall,

-	transmit in NR Cell 1 a Paging message including matched ng-5G-S-TMSI for the UE if an RRCRelease has been sent in step 8. If the paging fails, switches off and on the UE,

or

-	switch on the UE if the UE is switched off in step 8.

10. Repeat step 1-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.6.1.9.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.6.1.9.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with condition SSB Index and A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ and SSB Index Table H.3.1-8 with Condition SSB RLM |

Table 16.6.1.9.4.3-2: MeasObjectNR (step 4, test procedure)

| Derivation Path: Table H.3.1-3 with condition INTRA-FREQ MO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of NCD-SSB |  |  |
| smtc1 | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.4 RedCap | SMTC.4 RedCap |  |
| } |  |  |  |

Table 16.6.1.9.4.3-3: ServingCellConfig (step 1, test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.6.1.9.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.6.1.9.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 16.6.1.9.4.3-4: BWP-Downlink (Table 16.6.1.9.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| servingCellMO-r17 | 1 | MeasObjectID associated with NCD-SSB |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.9.4.3-5: BWP-Uplink (Table 16.6.1.9.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.6.1.9.5	Test requirement

Table 16.6.1.9.4.1-3 and Table 16.6.1.9.5-1 define the primary level settings including test tolerances.

Table 16.6.1.9.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | N/A |  | N/A |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | OP.1 |  | OP.1 |  |
| TRS Configuration |  | TRS.1.1 FDD |  | N/A |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| Active DL BWP configuration |  | DLBWP.1.3 RedCap Note 4 |  | DLBWP.1.3 RedCap Note 4 |  |
| Active UL BWP configuration |  | ULBWP.1.3 RedCap Note 5 |  | ULBWP.1.3 RedCap Note 5 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | -98 |  |  |  |
| Noc Note 2 | dBm/15 KHz | -98 |  |  |  |
| Ȇs/Iot | dB | 4 | -1.46 | -Infinity | -1.46 |
| Ȇs/Noc | dB | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | -64.60 | -62.25 | -64.60 | -62.25 |
| Propagation Condition |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dedicated DL BWP2 corresponding to NCD-SSB PRB index. Note 5: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |

The overall delay is measured from the beginning of time period T2 to the time that the SS receives the first MeasurementReport message triggered by event A3. It is allowed to be up to 2xTTIDCCH higher than the measurement reporting delay specified in 38.133[6] clause 9.2B.4 due to the TTI insertion uncertainty of the measurement report in DCCH.

Then the overall delay shall be shall be no larger than: Tidentify_intra_with_index + 2xTTIDCCH, where

-	Tidentify_intra_with_index = 1480 ms, is the time for intra-frequency cell identification specified in 38.133 [6] clause 9.2B.5.1,

-	TTIDCCH = 1 ms.

Which gives a total of 1482 ms.

The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 16.6.1.10	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE

16.6.1.10.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2B.5.1 and 9.2B.5.2.

16.6.1.10.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.6.1.10.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.10.

16.6.1.10.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for FDD PCell and neighbour cell are given in Table 16.6.1.10.4.1-3 and Table 16.6.1.10.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the PCell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.1.10.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.1.10.4.1-1.

Table 16.6.1.10.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.10-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.10-2 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.1.10.4.1-2.

Table 16.6.1.10.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.10.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 16.6.1.10.4.1-3 below.

2. Message contents are defined in clause 16.6.1.10.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 16.6.1.10.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1: Cell 1 and Cell 2 |  |
| CD-SSB configuration |  | SSB.1 FR1 |  |
| NCD-SSB configuration |  | SSB.6 RedCap FR1 Note 1 |  |
| SMTC configuration for CD-SSB |  | SMTC.2 RedCap |  |
| SMTC configuration for NCD-SSB |  | SMTC.4 RedCap |  |
| A3-Offset | dB | -4.5 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | N/A | OFF |
| Time offset between serving and neighbour cells | ms | 3 | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP. |  |  |  |

16.6.1.10.4.2	Test procedure

The test consists of two successive time periods with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1 and RedCap-specific dedicated BWP is the activated BWP. The SS transmits NCD-SSB on Cell 1 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

2. Set the parameters according to T1 in Table 16.6.1.10.5-1.

3. Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4. SS shall transmit an RRCReconfiguration message to configure Event A3 triggered reporting.

5. The UE shall transmit an RRCReconfigurationComplete message. T1 starts upon the SS receives the RRCReconfigurationComplete message.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.6.1.10.5-1. T2 starts. The SS also transmits NCD-SSB on Cell 2 with an absolute frequency of fNCDSSB calculated above.

7. The SS increases the number of successful tests by one if the UE transmits a MeasurementReport message triggered by event A3 satisfying the overall delay requirement specified in clause 16.6.1.10.5, otherwise the SS increases the number of failure tests by one.

8. After the SS receive the MeasurementReport message in step 7) or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

or

-	switch the UE off.

9. Depending on the choice in step 8, the SS shall,

-	transmit in NR Cell 1 a Paging message including matched ng-5G-S-TMSI for the UE if an RRCRelease has been sent in step 8. If the paging fails, switches off and on the UE,

or

-	switch on the UE if the UE is switched off in step 8.

10. Repeat step 1-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.6.1.10.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.6.1.10.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with condition SSB Index and A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ and SSB Index Table H.3.1-8 with Condition SSB RLM |

Table 16.6.1.10.4.3-2: MeasObjectNR (step 4, test procedure)

| Derivation Path: Table H.3.1-3 with condition INTRA-FREQ MO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of NCD-SSB |  |  |
| smtc1 | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.4 RedCap | SMTC.4 RedCap |  |
| } |  |  |  |

Table 16.6.1.10.4.3-3: ServingCellConfig (step 1, test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.6.1.10.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.6.1.10.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 16.6.1.10.4.3-4: BWP-Downlink (Table 16.6.1.10.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| servingCellMO-r17 | 1 | MeasObjectId associated with NCD-SSB |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.10.4.3-5: BWP-Uplink (Table 16.6.1.10.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.6.1.10.5	Test requirement

Table 16.6.1.10.4.1-3 and Table 16.6.1.10.5-1 define the primary level settings including test tolerances.

Table 16.6.1.10.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | N/A |  | N/A |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | OP.1 |  | OP.1 |  |
| TRS Configuration |  | TRS.1.1 FDD |  | N/A |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| Active DL BWP configuration |  | DLBWP.1.3 RedCap Note 4 |  | DLBWP.1.3 RedCap Note 4 |  |
| Active UL BWP configuration |  | ULBWP.1.3 RedCap Note 5 |  | ULBWP.1.3 RedCap Note 5 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | -98 |  |  |  |
| Noc Note 2 | dBm/15 KHz | -98 |  |  |  |
| Ȇs/Iot | dB | 4 | -1.46 | -Infinity | -1.46 |
| Ȇs/Noc | dB | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | -64.60 | -62.25 | -64.60 | -62.25 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dedicated DL BWP2 corresponding to NCD-SSB PRB index; Note 5: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |

The overall delay is measured from the beginning of time period T2 to the time that the SS receives the first MeasurementReport message triggered by event A3. It is allowed to be up to 2xTTIDCCH higher than the measurement reporting delay specified in 38.133[6] clause 9.2B.4 due to the TTI insertion uncertainty of the measurement report in DCCH.

Then the overall delay shall be shall be no larger than: Tidentify_intra_with_index + 2xTTIDCCH, where

-	Tidentify_intra_with_index = 1240 ms, is the time for intra-frequency cell identification specified in 38.133 [6] clause 9.2B.5.1,

-	TTIDCCH = 1 ms.

Which gives a total of 1242 ms.

The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 16.6.1.11	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE

16.6.1.11.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2B.6.1 and 9.2B.6.2.

16.6.1.11.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 onwards and supporting CSI-RS-based RLM and BWP operation without bandwidth restriction.

16.6.1.11.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.11.

16.6.1.11.4	Test description

16.6.1.11.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.6.4.1 with following exceptions:

-	Table 6.6.1.6.4.1-1 is replaced by Table 16.6.1.11.4.1-1.

-	Table 6.6.1.6.4.1-2 is replaced by Table 16.6.1.11.4.1-2.

-	Table 6.6.1.6.4.1-3 is replaced by Table 16.6.1.11.4.1-3.

Table 16.6.1.11.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.11-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.11-2 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.6.1.11.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.11.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.1.11.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2 | Cell 1 |  |
| Neighbour cell |  | 1,2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1,2 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1,2 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1,2 | 40 |  |
| Measurement gap length | ms | 1,2 | 6 |  |
| Measurement gap offset | ms | 1,2 | 39 |  |
| CD-SSB configuration |  | 1,2 | SSB.1 FR1 |  |
| SMTC configuration for CD-SSB |  | 1,2 | SMTC.2 |  |
| A3-Offset | dB | 1,2 | -4.5 |  |
| CP length |  | 1,2 | Normal |  |
| Hysteresis | dB | 1,2 | 0 |  |
| Time To Trigger | s | 1,2 | 0 |  |
| Filter coefficient |  | 1,2 | 0 | L3 filtering is not used |
| DRX |  | 1,2 | N/A | OFF |
| Time offset between serving and neighbour cells |  | 1,2 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
| T1 | s | 1,2 | 5 |  |
| T2 | s | 1,2 | 5 |  |

16.6.1.11.4.2	Test procedure

Same as the test procedure given in 6.6.1.6.4.2 with following exceptions:

-	Step 6 in 6.6.1.6.4.2 is replaced by following step.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1042 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

16.6.1.11.4.3	Message contents

Same as the message contents given in 6.6.1.6.4.3.

16.6.1.11.5	Test requirement

Same as the test requirement given in 6.6.1.6.5 with following exceptions:

-	Table 6.6.1.6.5-1 is replaced by Table 16.6.1.11.5-1.

-	The overall delay measured is expressed as:

Overall delays measured = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TSSB_time_index_intra_RedCap + TTI insertion uncertainty

-	TPSS/SSS_sync_intra_RedCap = 600 ms

-	T SSB_measurement_period_intra_RedCap= 200 ms

-	TSSB_time_index_intra_RedCap = 240 ms

-	TTI insertion uncertainty = 2 ms

As a result, the overall delay measured shall be less than a total of 1042 ms.

Table 16.6.1.11.5-1: NR Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,2 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1,2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1,2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1,2 | CCR.1.2 FDD |  | N/A |  |
| OCNG Patterns |  | 1,2 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,2 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP2 configuration |  | 1,2 | DLBWP.1.2 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP2 configuration |  | 1,2 | ULBWP.1.2 RedCap |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2 | -98 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2 | -98 |  |  |  |
|  | dB | 1,2 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2 | -94 | -94 | -Infinity | -94 |
| Io | dBm/ 9.36 MHz | 1,2 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
| Propagation Condition |  | 1,2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### 16.6.1.12	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2Rx UE

16.6.1.12.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2B.6.1 and 9.2B.6.2.

16.6.1.12.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 onwards and supporting CSI-RS-based RLM and BWP operation without bandwidth restriction.

16.6.1.12.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.12.

16.6.1.12.4	Test description

16.6.1.12.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.6.4.1 with following exceptions:

-	Table 6.6.1.6.4.1-1 is replaced by Table 16.6.1.12.4.1-1.

-	Table 6.6.1.6.4.1-2 is replaced by Table 16.6.1.12.4.1-2.

-	Table 6.6.1.6.4.1-3 is replaced by Table 16.6.1.12.4.1-3.

Table 16.6.1.12.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.12-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.12-2 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.6.1.12.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.12.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.6.1.12.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2 | Cell 1 |  |
| Neighbour cell |  | 1,2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1,2 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1,2 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1,2 | 40 |  |
| Measurement gap length | ms | 1,2 | 6 |  |
| Measurement gap offset | ms | 1,2 | 39 |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |
| SMTC configuration |  | 1,2 | SMTC.2 |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.1.2 FDD resource #0 |  |
| A3-Offset | dB | 1,2 | -4.5 |  |
| CP length |  | 1,2 | Normal |  |
| Hysteresis | dB | 1,2 | 0 |  |
| Time To Trigger | s | 1,2 | 0 |  |
| Filter coefficient |  | 1,2 | 0 | L3 filtering is not used |
| DRX |  | 1,2 | N/A | OFF |
| Time offset between serving and neighbour cells |  | 1,2 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
| T1 | s | 1,2 | 5 |  |
| T2 | s | 1,2 | 5 |  |

16.6.1.12.4.2	Test procedure

Same as the test procedure given in 6.6.1.6.4.2.

16.6.1.12.4.3	Message contents

Same as the message contents given in 6.6.1.6.4.3.

Table 16.6.1.12.4.3-1: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 RedCap configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 RedCap configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

16.6.1.12.5	Test requirement

Same as the test requirement given in 6.6.1.6.5 with following exceptions:

-	Table 6.6.1.6.5-1 is replaced by Table 16.6.1.12.5-1.

-	The overall delay measured is expressed as:

Overall delays measured = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TSSB_time_index_intra_RedCap + TTI insertion uncertainty

-	TPSS/SSS_sync_intra_RedCap = 600 ms

-	T SSB_measurement_period_intra_RedCap= 200 ms

-	TSSB_time_index_intra_RedCap = 120 ms

-	TTI insertion uncertainty = 2 ms

As a result, the overall delay measured shall be less than a total of 922 ms.

Table 16.6.1.12.5-1: NR Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,2 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1,2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1,2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1,2 | CCR.1.2 FDD |  | N/A |  |
| OCNG Patterns |  | 1,2 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,2 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2 | DLBWP.1.2 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2 | ULBWP.1.2 RedCap |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2 | -98 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2 | -98 |  |  |  |
|  | dB | 1,2 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2 | -94 | -94 | -Infinity | -94 |
| Io | dBm/ 9.36 MHz | 1,2 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
| Propagation Condition |  | 1,2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

### 16.6.2	Inter-frequency measurements for RedCap

#### 16.6.2.0	Minimum conformance requirements

##### 16.6.2.0.1	Minimum conformance requirements for inter-frequency event-triggered measurement with measurement gaps

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index_RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index_RedCap. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index_RedCap.

Tidentify_inter_without_index_RedCap = (TPSS/SSS_sync_inter_RedCap + T SSB_measurement_period_inter_RedCap) ms

Tidentify_inter_with_index_RedCap = (TPSS/SSS_sync_inter_RedCap + T SSB_measurement_period_inter_RedCap + TSSB_time_index_inter_RedCap) ms

Where:

TPSS/SSS_sync_inter_RedCap: it is the time period used in PSS/SSS detection given in Table 16.6.2.0.1-1 and Table 16.6.2.0.1-2.

TSSB_time_index_inter_RedCap: it is the time period used to acquire the index of the SSB being measured given in Table 16.6.2.0.1-3 and Table 16.6.2.0.1-4.

TSSB_measurement_period_inter_RedCap: equal to a measurement period of SSB based measurement given in Table 16.6.2.0.1-5 and Table 16.6.2.0.1-6.

Editor’s note: which power class to be used for RedCap depends on the RF session outcome.

CSSFinter_RedCap: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_RedCap,i in clause 9.1A.5.2 for measurement conducted within measurement gaps.

Table 16.6.2.0.1-1: Time period for PSS/SSS detection (Frequency range FR1) for 2Rx RedCap UE

| Condition NOTE1 | TPSS/SSS_sync_inter_RedCap |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Table 16.6.2.0.1-2: Time period for PSS/SSS detection (Frequency range FR1) for 1Rx RedCap UE

| Condition NOTE1 | TPSS/SSS_sync_inter_RedCap |
| --- | --- |
| No DRX | Max(600ms, 10 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(10 *1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 10 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Table 16.6.2.0.1-3: Time period for time index detection (Frequency range FR1) for 2Rx RedCap UE

| Condition NOTE1 | TSSB_time_index_inter_RedCap |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 3 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Table 16.6.2.0.1-4: Time period for time index detection (Frequency range FR1) for 1Rx RedCap UE

| Condition NOTE1 | TSSB_time_index_inter_RedCap |
| --- | --- |
| No DRX | Max(160ms, 6 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(160ms, Ceil(6 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

When measurement gaps are provided for inter frequency measurements, the RedCap UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 38.133 [6] clauses 10.1A.4, 10.1A.5, 10.1A.8, 10.1A.9, 10.1A.12 and 10.1A.13, respectively,  as shown in table 16.6.2.0.1-5 for 2Rx RedCap and in table 16.6.2.0.1-6 for 1Rx RedCap, respectively.

Table 16.6.2.0.1-5: Measurement period for inter-frequency measurements with gaps (Frequency FR1) for 2Rx RedCap UE

| Condition NOTE1 | T SSB_measurement_period_inter_RedCap |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Table 16.6.2.0.1-6: Measurement period for inter-frequency measurements with gaps (Frequency FR1) for 1Rx RedCap UE

| Condition NOTE1 | T SSB_measurement_period_inter_RedCap |
| --- | --- |
| No DRX | Max(400ms, 8 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(400ms, ceil(1.5 8) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

For 2 Rx RedCap UE: Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.4.1, 10.1.5.1, 10.1.9.1, 10.1.10.1, 10.1.14.1 and 10.1.15.1, respectively.

For 1 Rx RedCap UE: Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses10.1A.4.1, 10.1A.8.1 and 10.1A.12.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 × TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be within Tidentify_inter_without_index_ RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index_ RedCap. Both Tidentify_inter_without_index_ RedCap and Tidentify_inter_with_index_ RedCap are defined in TS 38.133 [6] in clause 9.3B.4. When L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index_ RedCap or Tidentify_intra_with_index_ RedCap as defined in TS 38.133 [6] in clause 9.2B.5.1 or clause 9.2B.6.2. If a cell which has been detectable at least for the time period Tidentify intra without index_ RedCap or Tidentify intra with index_ RedCap defined in TS 38.133 [6] in clause 9.2B.5.1 or clause 9.2B.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra_ RedCap provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

##### 16.6.2.0.2	Minimum conformance requirements for inter-frequency event-triggered measurements without measurement gaps

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index_RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index_RedCap. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index_RedCap. It is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2, the following conditions are met:

-	SFN and frame boundary across serving cell and inter-frequency neighbor cells is aligned, and

-	the timing of SSBs across serving cell and inter-frequency neighbor cells are aligned

Tidentify_inter_without_index_RedCap = (TPSS/SSS_sync_inter_RedCap + T SSB_measurement_period_inter_RedCap) ms

Tidentify_inter_with_index_RedCap = (TPSS/SSS_sync_inter_RedCap + T SSB_measurement_period_inter_RedCap + TSSB_time_index_inter_RedCap) ms

Where:

TPSS/SSS_sync_inter_RedCap: it is the time period used in PSS/SSS detection given in table 16.6.2.0.2-1, table 16.6.2.0.2-2.

TSSB_time_index_inter_RedCap: it is the time period used to acquire the index of the SSB being measured given in table 16.6.2.0.2-3 and table 16.6.2.0.2-4.

T SSB_measurement_period_inter_RedCap: equal to a measurement period of SSB based measurement given in table 16.6.2.0.2-5, table 16.6.2.0.2-6.

CSSFinter_RedCap: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap_RedCap,i in clause 9.1A.5.1 for measurement conducted outside measurement gaps, i.e. when interfrequency SMTC is fully non overlapping or partially overlapping with measurement gaps or according to CSSFwithin_gap_RedCap,i in clause 9.1A.5.2 for measurement conducted within measurement gaps, i.e. when interfrequency SMTC is fully overlapping with measurement gaps.

Mpss/sss_sync_inter_RedCap: For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync_inter_RedCap = 40 samples. For a UE supporting FR2 power class 2, Mpss/sss_sync_inter_RedCap = 24 samples. For a UE supporting FR2 power class 3, Mpss/sss_sync_inter_RedCap = 24 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync_RedCap = 24 samples. For a UE supporting FR2 power class 7, Mpss/sss_sync_inter_RedCap = 24 samples.

MSSB_index_inter_RedCap: For a UE supporting power class 1 or 5, MSSB_index_inter_RedCap = 40 samples. For a vehicle mounted UE supporting power class 2, Mpss/sss_sync_inter_RedCap = 24 samples. For a UE supporting power class 3, MSSB_index_inter_RedCap = 24 samples. For a UE supporting power class 4, Mmeas_period_inter_RedCap = 24 samples. For a UE supporting power class 7, MSSB_index_inter_RedCap = 24 samples.

Mmeas_period_inter_RedCap: For a UE supporting FR2 power class 1 or 5, Mmeas_period_inter_RedCap =40 samples. For a vehicle mounted UE supporting FR2 power class 2, Mpss/sss_sync_inter_RedCap =24 samples. For a UE supporting FR2 power class 3, Mmeas_period_inter_RedCap =24 samples. For a UE supporting FR2 power class 4, Mmeas_period_inter_RedCap = 24 samples. For a UE supporting FR2 power class 7, Mmeas_period_inter_RedCap =24 samples.

When interfrequency SMTC is fully non overlapping with measurement gaps or interfrequency SMTC is fully overlapping with MGs, Kp=1.

When interfrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP.

For FR2,

Klayer1_measurement =1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by inter-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by inter-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

Table 16.6.2.0.2-1: Time period for PSS/SSS detection (FR1) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_inter_RedCap |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter_RedCap |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter_RedCap |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFinter_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.2.0.2-2: Time period for PSS/SSS detection (FR1) for 1Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_inter_RedCap |
| --- | --- |
| No DRX | max( 600ms, ceil( 7 x Kp) x SMTC period )Note 1 x CSSFinter_RedCap |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 7 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter_RedCap |
| DRX cycle>320ms | ceil(7 x Kp) x DRX cycle x CSSFinter_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.2.0.2-3: Time period for time index detection (FR1) for 2Rx RedCap UE

| DRX cycle | TSSB_time_index_inter_RedCap |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFinter_RedCap |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter_RedCap |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFinter_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.2.0.2-4: Time period for time index detection (FR1) for 1Rx RedCap UE

| DRX cycle | TSSB_time_index_inter_RedCap |
| --- | --- |
| No DRX | max(160ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter_RedCap |
| DRX cycle≤ 320ms | max(160ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter_RedCap |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFinter_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The RedCap UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clauses 10.1A.4, 10.1A.5, 10.1A.8, 10.1A.9, 10.1A.12 and 10.1A.13, respectively, as shown in table 16.6.2.0.2-5, for 2Rx RedCap and table 16.6.2.0.2-5 for 1Rx RedCap, if UE supports inter-frequency measurement without measurement gaps:

Table 16.6.2.0.2-5: Measurement period for inter-frequency measurements without gaps ((FR1) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter_RedCap |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter_RedCap |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFinter_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 16.6.2.0.2-6: Measurement period for inter-frequency measurements without gaps ((FR1) for 1Rx RedCap UE

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter_RedCap |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter_RedCap |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFinter_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

#### 16.6.2.1	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE

16.6.2.1.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.1.

16.6.2.1.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.1.4.1-3 and Table 16.6.2.1.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.1.4.1-1.

Table 16.6.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.1.4.1-2.

Table 16.6.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 16.6.2.1.4.1-3 below.

2. Message contents are defined in clause 16.6.2.1.4.3.

3. There  two cells on two carriers specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 16.6.2.1.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 |  | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |  |
| CP length |  | Config 1,2,3,4 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | DRX.1 | DRX. 7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |  |
| T2 | s | Config 1,2,3,4 | 1.1 | 12 |  |

Table 16.6.2.1.4.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table 16.6.2.1.4.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

16.6.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.2.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.2.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test 1, 11520 ms for Test 2, 1520 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.1.4.3	Message contents

Same as the message contents given in clause 6.6.2.2.4.3.

16.6.2.1.5	Test requirement

Table 16.6.2.1.4.1-3 and Table 16.6.2.1.5-1 define the primary level settings including test tolerances.

Table 16.6.2.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3,4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3,4 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  | Config 1, 2, 3,4 | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  | Config 1, 2, 3,4 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR.2.1 TDD |  | NA |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1SSB.1 FR1 |  |
|  |  |  | Config 2 | SSB.1 RedCap FR1 |  | SSB.1 35 RedCap FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.1 3 RedCap FR1 |  |
| SMTC configuration defined in A.4A |  |  | Config 1,4 | SMTC.21 RedCap |  | SMTC.51 RedCap |  |
|  |  |  | Config 2, 3 | SMTC.1 RedCap |  | SMTC.41 RedCap |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -61.66 | -61.66 | -67.11 | -59.32 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 11520 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.2	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE

16.6.2.2.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.2.

16.6.2.2.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.2.4.1-3 and Table 16.6.2.2.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.2.4.1-1.

Table 16.6.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.2.4.1-2.

Table 16.6.2.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 16.6.2.2.4.1-3 below.

2. Message contents are defined in clause 16.6.2.2.4.3.

3. There  two cells on two carriers specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 16.6.2.2.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 |  |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 |  |  | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |  |  |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |  |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |  |  |
| CP length |  | Config 1,2,3,4 | Normal |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |  |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | DRX.1 |  | DRX. 7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |  |  |
| T2 | s | Config 1,2,3,4 | 1.1 | 11 |  |  |

Table 16.6.2.2.4.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table 16.6.2.2.4.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

16.6.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.2.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.2.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test 1, 10240 ms for Test 2, 1080 ms for Test 3 and 10240 ms for Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.2.4.3	Message contents

Same as the message contents given in clause 6.6.2.2.4.3.

16.6.2.2.5	Test requirement

Table 16.6.2.2.4.1-3 and Table 16.6.2.2.5-1 define the primary level settings including test tolerances.

Table 16.6.2.2.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3,4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3,4 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR.2.1 TDD |  | NA |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4A |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.3	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE

16.6.2.3.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.3.

16.6.2.3.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.3.4.1-3 and Table 16.6.2.3.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.3.4.1-1.

Table 16.6.2.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.3-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.3-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.3-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.3-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.3.4.1-2.

Table 16.6.2.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.3.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Parameter | Unit | Test configuration | Value |  | Comment |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 |  | As specified in clause 38.133 [6] 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |  |
| SMTC-SSB parameters |  | Config 1,2,4 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.1 RedCap FR1 |  | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |  |
| CP length |  | Config 1,2,3,4 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |  |
| T2 | s | Config 1,2,3,4 | 1.1 |  |  |

16.6.2.3.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.2.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.2.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 920 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.3.4.3	Message contents

Same as the message contents given in clause 6.6.2.1.4.3.

16.6.2.3.5	Test requirement

Table 16.6.2.3.4.1-3 and Table 16.6.2.3.5-1 define the primary level settings including test tolerances.

Table 16.6.2.3.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3, 4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1,2,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4A |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/18.72 MHz | Config 3 | -61.68 | -61.68 | -67.13 | -59.34 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

The UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE

16.6.2.4.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.6.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.4.

16.6.2.4.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.4.4.1-3 and Table 16.6.2.4.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.4.4.1-1.

Table 16.6.2.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.4-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.4-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.4-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.4-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.4.4.1-2.

Table 16.6.2.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.4.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |
| SMTC-SSB parameters |  | Config 1,2,4 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.1 RedCap FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |
| CP length |  | Config 1,2,3,4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |
| T2 | s | Config 1,2,3,4 | 1 |  |

16.6.2.4.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.2.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.2.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 920 ms Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.4.4.3	Message contents

Same as the message contents given in clause 6.6.2.1.4.3.

16.6.2.4.5	Test requirement

Table 16.6.2.4.4.1-3 and Table 16.6.2.4.5-1 define the primary level settings including test tolerances.

Table 16.6.2.4.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3, 4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR2.1 TDD |  | NA |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1,2,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4A |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/18.72 MHz | Config 3 | -61.68 | -61.68 | -67.13 | -59.34 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSBRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

The UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE

16.6.2.5.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.5.

16.6.2.5.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.5.4.1-3 and Table 16.6.2.5.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.5.4.1-1.

Table 16.6.2.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.5-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.5-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.5-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.5-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.5.4.1-2.

Table 16.6.2.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.5.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |
| SMTC-SSB parameters |  | Config 1,2,4 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.1 RedCap FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |
| CP length |  | Config 1,2,3,4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |
| T2 | s | Config 1,2,3,4 | 1 |  |

16.6.2.5.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.2.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.2.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 920 mss then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.5.4.3	Message contents

Same as the message contents given in clause 6.6.2.1.4.3.

16.6.2.5.5	Test requirement

Table 16.6.2.5.4.1-3 and Table 16.6.2.5.5-1 define the primary level settings including test tolerances.

Table 16.6.2.5.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3, 4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1,2,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4A |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/18.72 MHz | Config 3 | -61.68 | -61.68 | -67.13 | -59.34 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | Config 1,2,3,4 | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE

16.6.2.6.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.6.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.6.

16.6.2.6.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.6.4.1-3 and Table 16.6.2.6.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.6.4.1-1.

Table 16.6.2.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.6-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.6-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.6-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.6-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.6.4.1-2.

Table 16.6.2.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.6.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |
| SMTC-SSB parameters |  | Config 1,2,4 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.1 RedCap FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |
| CP length |  | Config 1,2,3,4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |
| T2 | s | Config 1,2,3,4 | 1 |  |

16.6.2.6.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.2.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.2.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 920 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.6.4.3	Message contents

Same as the message contents given in clause 6.6.2.1.4.3.

16.6.2.6.5	Test requirement

Table 16.6.2.6.4.1-3 and Table 16.6.2.6.5-1 define the primary level settings including test tolerances.

Table 16.6.2.6.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3, 4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1,2,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4A |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/18. 36 MHz | Config 3 | -61.68 | -61.68 | -67.13 | -59.34 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | Config 1,2,3,4 | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE

16.6.2.7.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.2.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.7.

16.6.2.7.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.7.4.1-3 and Table 16.6.2.7.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.7.4.1-1.

Table 16.6.2.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.7-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.7-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.7-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.7-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.7.4.1-2.

Table 16.6.2.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.7.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 |  | As specified in 38.133 [6] clause 9.1A.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |  |
| CP length |  | Config 1,2,3,4 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | DRX.1 | DRX. 7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |  |
| T2 | s | Config 1,2,3,4 | 1.5 | 16 |  |

16.6.2.7.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.6.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.6.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1440 ms for Test 1, 15360 ms for Test 2, , then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

16.6.2.7.4.3	Message contents

Same as the message contents given in clause 6.6.2.6.4.3.

16.6.2.7.5	Test requirement

Table 16.6.2.7.4.1-3 and Table 16.6.2.7.5-1 define the primary level settings including test tolerances.

Table 16.6.2.7.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3,4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4 |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| Ês/Iot |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| Ês/Noc |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/18.36MHz | Config 3 | -61.68 | -61.68 | -67.13 | -59.34 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | Config 1,2,3,4 | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 15360 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test 1 and 2 the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE

16.6.2.8.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.6.2.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.8.

16.6.2.8.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.8.4.1-3 and Table 16.6.2.8.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.8.4.1-1.

Table 16.6.2.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.8-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.8-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.8-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.8-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.8.4.1-2.

Table 16.6.2.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.8.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 |  | As specified in 38.133 [6] clause 9.1A.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |  |
| CP length |  | Config 1,2,3,4 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | DRX.1 | DRX. 7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |  |
| T2 | s | Config 1,2,3,4 | 1.3 | 13.5 |  |

16.6.2.8.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.6.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.6.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1260ms for Test 1, 12160ms for Test 2, , then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.8.4.3	Message contents

Same as the message contents given in clause 6.6.2.6.4.3.

16.6.2.8.5	Test requirement

Table 16.6.2.8.4.1-3 and Table 16.6.2.8.5-1 define the primary level settings including test tolerances.

Table 16.6.2.8.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3,4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4 |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| Ês/Iot |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| Ês/Noc |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/18.36MHz | Config 3 | -61.68 | -61.68 | -67.13 | -59.34 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | Config 1,2,3,4 | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1260 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test 1 and 2 the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.9	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE

16.6.2.9.1	Test purpose

To verify that the UE makes correct reporting of an event when mandatory gap pattern with 3ms MGL is configured. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.9.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.2.9.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.9.

16.6.2.9.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.9.4.1-3 and Table 16.6.2.9.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.9.4.1-1.

Table 16.6.2.9.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.9-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.9-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.9-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.9-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.9.4.1-2.

Table 16.6.2.9.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.9.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.9.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |  |
| NR RF Channel Number |  | Config 1,2,3, 4 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3, 4 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3, 4 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3, 4 | 3 |  | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3, 4 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1 |  |

16.6.2.9.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.9.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.9.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1440 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.9.4.3	Message contents

Same as the message contents given in clause 6.6.2.9.4.3.

16.6.2.9.5	Test requirement

Table 16.6.2.9.4.1-3 and Table 16.6.2.9.5-1 define the primary level settings including test tolerances.

Table 16.6.2.9.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3,4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4 |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| Ês/Iot |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| Ês/Noc |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/18.36MHz | Config 3 | -61.66 | -61.66 | -67.11 | -59.32 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | Config 1,2,3,4 | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

The UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.10	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE

16.6.2.10.1	Test purpose

To verify that the UE makes correct reporting of an event when mandatory gap pattern with 3ms MGL is configured.

16.6.2.10.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.6.2.10.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.10.

16.6.2.10.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.10.4.1-3 and Table 16.6.2.10.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the  neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.10.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.10.4.1-1.

Table 16.6.2.10.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.10-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.10-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.10-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.10-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.10.4.1-2.

Table 16.6.2.10.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.10.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.10.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NR RF Channel Number |  | Config 1,2,3, 4 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3, 4 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3, 4 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3, 4 | 3 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3, 4 | 9 |  |
| SMTC-SSB parameters |  | Config 1, 4 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.1 RedCap FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1.5 |  |

16.6.2.10.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.9.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.9.4.2 is replaced by following step.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1280 ms for Test 1 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.10.4.3	Message contents

Same as the message contents given in clause 6.6.2.9.4.3.

16.6.2.10.5	Test requirement

Table 16.6.2.10.4.1-3 and Table 16.6.2.10.5-1 define the primary level settings including test tolerances.

Table 16.6.2.10.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3,4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.4 |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| Ês/Iot |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| Ês/Noc |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/18.36MHz | Config 3 | -61.66 | -61.66 | -67.11 | -59.32 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | Config 1,2,3,4 | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

The UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.11	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE

16.6.2.11.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency NR cell search without measurement gap requirements in 38.133 [6] clause 9.3B.7.

16.6.2.11.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx which supports interFrequencyMeas-Nogap-r16 from Release 17 onwards.

16.6.2.11.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.11.

16.6.2.11.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.11.4.1-3 and Table 16.6.2.11.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.11.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.11.4.1-1.

Table 16.6.2.11.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.11-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.11-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.11-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.11-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.11.4.1-2.

Table 16.6.2.11.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.11.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.11.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3, 4 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3, 4 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3, 4 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3, 4 | -6 |  |
| Hysteresis | dB | Config 1,2,3, 4 | 0 |  |
| CP length |  | Config 1,2,3, 4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3, 4 | 0 |  |
| Filter coefficient |  | Config 1,2,3, 4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3, 4 | DRX.1 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1, 4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3, 4 | 5 |  |
| T2 | s | Config 1,2,3, 4 | 1.5 |  |

Table 16.6.2.11.4.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

16.6.2.11.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.10.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.10.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1320 ms for Test Configuration 1 and 4 and 900 ms for Test Configuration 2 and 3 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.11.4.3	Message contents

Same as the message contents given in clause 6.6.2.10.4.3.

16.6.2.11.5	Test requirement

Table 16.6.2.11.4.1-3 and Table 16.6.2.11.5-1 define the primary level settings including test tolerances.

Table 16.6.2.11.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,4 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3, 4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3, 4 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1,4 | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,4 | SMTC.2 |  | SMTC.2 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2, 4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -61.66 | -61.66 | -67.11 | -59.32 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test config 1 and 4, UE is required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1320 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%..

In test config 2 and 3, UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 900 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 16.6.2.12	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE

16.6.2.12.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency NR cell search without measurement gap requirements in 38.133 [6] clause 9.3B.7.

16.6.2.12.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx which supports interFrequencyMeas-Nogap-r16 from Release 17 onwards.

16.6.2.12.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.12.

16.6.2.12.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.12.4.1-3 and Table 16.6.2.12.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the  neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.12.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.12.4.1-1.

Table 16.6.2.12.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.12-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.12-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.12-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.12-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.12.4.1-2.

Table 16.6.2.12.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.12.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.12.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3, 4 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3, 4 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3, 4 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3, 4 | -6 |  |
| Hysteresis | dB | Config 1,2,3, 4 | 0 |  |
| CP length |  | Config 1,2,3, 4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3, 4 | 0 |  |
| Filter coefficient |  | Config 1,2,3, 4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3, 4 | DRX.1 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1, 4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3, 4 | 5 |  |
| T2 | s | Config 1,2,3, 4 | 1.5 |  |

Table 16.6.2.12.4.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

16.6.2.12.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.10.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.10.4.2 is replaced by following step.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test Configuration 1 and 4 and 900 ms for Test Configuration 2 and 3then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.12.4.3	Message contents

Same as the message contents given in clause 6.6.2.10.4.3.

16.6.2.12.5	Test requirement

Table 16.6.2.12.4.1-3 and Table 16.6.2.12.5-1 define the primary level settings including test tolerances.

Table 16.6.2.12.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,4 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3, 4 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3, 4 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3,4 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1,4 | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  | SSB.3 RedCap FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,4 | SMTC.2 |  | SMTC.2 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2, 4 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/18.36MHz | Config 3 | -61.66 | -61.66 | -67.11 | -59.32 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test config 1 and 4, UE is required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

In test config 2 and 3, UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 900 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 16.6.3	Inter-RAT measurements for RedCap

#### 16.6.3.0	Minimum conformance requirements

TBD

#### 16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE

#### 16.6.3.2	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE

#### 16.6.3.3	NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE

#### 16.6.3.4	NR - E-UTRA event-triggered reporting in DRX in FR1 for 2 Rx UE

### 16.6.4	L1-RSRP measurement for beam reporting for RedCap

#### 16.6.4.0	Minimum conformance requirements

##### 16.6.4.0.1	General

When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for PCell, on the resources configured for L1-RSRP measurements within the active BWP.

The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP.

The UE shall send L1-RSRP reports only for report configurations configured for the active BWP.

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause TBD for FR1, for 1 Rx RedCap, and 10.1.19 for FR1 and 10.1.20 for FR2 for 2 Rx RedCap, if nrofReportedRS is configured to one. If nrofReportedRS is configured to be larger than one, or if groupBasedBeamReporting is enabled, the UE shall use differential L1-RSRP based reporting as defined in clause TBD for FR1, for 1 Rx RedCap, and 10.1.19 for FR1 and 10.1.20 for FR2 for 2 Rx RedCap. The differential L1-RSRP is quantized to a 4-bit value with 2dB step size. The mapping between the reported L1-RSRP value and the measured quantity is described in TBD for 1 Rx RedCap, and 10.1.6 for 2 Rx RedCap.

For 1 Rx RedCap, reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clause TBD for FR1.

For 2 Rx RedCap, reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall only send periodic L1-RSRP measurement reports for an active BWP.

The UE shall transmit the periodic L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

For 1 Rx RedCap, reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clause TBD.

For 2 Rx Redcap, reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall only send aperiodic L1-RSRP measurement reports, if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

For FD-FDD and TDD RedCap UEs, there are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 16.6.4.0.2, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 16.6.4.0.3.

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	For FD-FDD and TDD RedCap UEs, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

-	For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 16.6.4.0.2, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 16.6.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause 9.5B.1, 9.5B.3 and 9.5B.6.

16.6.4.0.2	Minimum conformance requirements for SSB based L1-RSRP measurement for beam reporting

The requirements in this clause apply, provided:

-	The SSB resources configured for L1-RSRP measurements are measurable.

An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

For 1 Rx RedCap:

-	L1-RSRP related side conditions given in 38.133 [6] clauses TBD for FR1, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to 38.133 [6] Annex TBD for a corresponding band.

For 2 Rx RedCap:

-	L1-RSRP related side conditions given in 38.133 [6] clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to 38.133 [6] Annex B.2.4.1 for a corresponding band.

The UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap.

The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 16.6.4.0.2-1 for FR1 for 2 Rx RedCap, and Table 16.6.4.0.2-2 for FR1 for 1 Rx RedCap, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise.

For FR1,

-	P=11-TSSBMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

Where:

-	TSSB = ssb-periodicityServingCell

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For an FR1 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 16.6.4.0.2-1: Measurement period TL1-RSRP_Measurement_Period_SSB_RedCap (FR1) for 2 Rx RedCap

| Configuration | TL1-RSRP_Measurement_Period_SSB_RedCap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 16.6.4.0.2-2: Measurement period TL1-RSRP_Measurement_Period_SSB_RedCap (FR1) for 1 Rx RedCap

| Configuration | TL1-RSRP_Measurement_Period_SSB_RedCap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

For FR1, when the SSB for L1-RSRP measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

The normative reference for this requirement is TS 38.133 [6] clauses 9.5B.2, 9.5B.4.1 and 9.5B.5.1.

16.6.4.0.3	Minimum conformance requirements for CSI-RS based L1-RSRP measurement for beam reporting

The requirements in this clause apply, provided:

-	The CSI-RS resources configured for L1-RSRP measurements are measurable.

A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met:

For 1 Rx RedCap:

-	L1-RSRP related side conditions given in 38.133 [6] clauses TBD for FR1, respectively, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to 38.133 [6] Annex TBD for a corresponding band.

For 2 Rx RedCap:

-	L1-RSRP related side conditions given in 38.133 [6] clauses 10.1.19.2 and 10.1.20.2 for FR1 and FR2, respectively, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to 38.133 [6] Annex B.2.4.2 for a corresponding band.

Requirements are defined for periodic, semi-persistent and aperiodic resources.

The UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS_RedCap.

The value of TL1-RSRP_Measurement_Period_CSI-RS_RedCap is defined in Table 16.6.4.0.3-1 for FR1 for 2 Rx RedCap, and in Table 16.6.4.0.3-2 for 1 Rx RedCap, where

-	For periodic CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	For aperiodic CSI-RS resources M=1

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in 38.133 [6] clause 10.1.19.2 and 10.1.20.2 if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured with QCL-TypeD for all resources in the resource set.

For FR1,

-	P=11-TCSI-RSMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

Where:

TCSI-RS = the periodicity of CSI-RS configured for L1-RSRP measurement

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For an FR1 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 16.6.4.0.3-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS_RedCap (FR1) for 2 Rx RedCap

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS_RedCap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Table 16.6.4.0.3-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS_RedCap (FR1) for 1 Rx RedCap

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS_RedCap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

For FR1, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

The normative reference for this requirement is TS 38.133 [6] clauses 9.5B.2, 9.5B.4.2 and 9.5B.5.2.

#### 16.6.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE

16.6.4.1.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in 38.133 [6] clause 9.5B.4.1.

16.6.4.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 onwards.

16.6.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.1.

16.6.4.1.4	Test description

16.6.4.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.1.4.1 with following exceptions:

-	Table 6.6.4.1.4.1-1 is replaced by Table 16.6.4.1.4.1-1.

-	Table 6.6.4.1.4.1-2 is replaced by Table 16.6.4.1.4.1-3.

-	Table 6.6.4.1.4.1-3 is replaced by Table 16.6.4.1.4.1-2.

Table 16.6.4.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.6.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.6.4.1.4.1-2: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.1.4.1-3: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2,3 |  | TDD |
|  | 4 |  | HD-FDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |
|  | 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |
|  | 3 |  | SSB.2 RedCap FR1 |
| OCNG Patterns | 1~4 |  | OP.1 |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~4 |  | SMTC.1 RedCap |
| TRS Configuration | 1,4 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~4 |  | Off |
| reportConfigType | 1~4 |  | periodic |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |
| Number of reported RS | 1~4 |  | 2 |
| L1-RSRP reporting period | 1~4 | slot | 80 |
| T1 | 1~4 | s | 5 |
| T2 | 1~4 | s | 1 |
| EPRE ratio of PSS to SSS | 1~4 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~4 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.1.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.1.4.2 with following exceptions:

-	Step 5 in test procedure given in clause 6.6.4.1.4.2 is replaced by following step.

5.	The UE shall start sending valid L1-RSRP reports. The SS shall check following requirements:

- R1:	the UE shall start to transmit valid reports no later than 720 ms for configuration 1, 2, 4 and no later than 680 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement for SSB#1 (Table 16.6.4.1.5-2 for test configurations 1, 2, 4 and Table 16.6.4.1.5-3 for test configurations 3) and the relative L1-RSRP requirement for SSB#0 in Table 16.6.4.1.5-4. If the first valid report is received earlier than the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the UE shall transmit reports every 80 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

- R3:	The L1-RSRP value of SSB#1 reported by the UE is compared to the expected L1-RSRP value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 16.6.4.1.5-2 for test configurations 1, 2, 4 and 5 and in Table 16.6.4.1.5-3 for test configurations 3 and 6 or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4:	The DIFF RSRP value of SSB#0 reported by the UE is compared to the expected DIFF RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 16.6.4.1.5-4 or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

16.6.4.1.4.3	Message contents

Same as the message contents given in clause 6.6.4.1.4.3.

16.6.4.1.5	Test requirement

Same as the test requirements given in clause 6.6.4.1.5 with following exceptions:

-	Table 6.6.4.1.5-1 is replaced by Table 16.6.4.1.5-1.

-	Table 6.6.4.1.5-2 is replaced by Table 16.6.4.1.5-2.

-	Table 6.6.4.1.5-3 is replaced by Table 16.6.4.1.5-3.

-	Table 6.6.4.1.5-4 is replaced by Table 16.6.4.1.5-4.

Table 16.6.4.1.5-1: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~4 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2,4 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~4 | dB | 0 | 0 | -Infinity | 6.5 |
| SSB RSRP Note3 | 1,2.4 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -88.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -85.15 |
| Io Note3 | 1,2.4 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -59.32 |
|  | 3 | dBm/18.36 MHz | -60.77 | -60.77 | -63.79 | -56.39 |
|  | 1~4 | dB | 0 | 0 | -Infinity | 6.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 16.6.4.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (SSB#1) | 55 |
| Highest reported value (SSB#1) | 81 |

Table 16.6.4.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (SSB#1) | 58 |
| Highest reported value (SSB#1) | 84 |

Table 16.6.4.1.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T2 |
| --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | 0 |
| Highest DIFF RSRP reported (SSB#0) | 6 |

#### 16.6.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE

16.6.4.2.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in 38.133 [6] clause 9.5B.4.1.

16.6.4.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 onwards.

16.6.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.2.

16.6.4.2.4	Test description

16.6.4.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.1.4.1 with following exceptions:

-	Table 6.6.4.1.4.1-1 is replaced by Table 16.6.4.2.4.1-1.

-	Table 6.6.4.1.4.1-2 is replaced by Table 16.6.4.2.4.1-3.

-	Table 6.6.4.1.4.1-3 is replaced by Table 16.6.4.2.4.1-2.

Table 16.6.4.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.6.4.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.6.4.2.4.1-2: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.2.4.1-3: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2,3 |  | TDD |
|  | 4 |  | HD-FDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |
|  | 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |
|  | 3 |  | SSB.2 RedCap FR1 |
| OCNG Patterns | 1~4 |  | OP.1 |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~4 |  | SMTC.1 RedCap |
| TRS Configuration | 1,4 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~4 |  | Off |
| reportConfigType | 1~4 |  | periodic |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |
| Number of reported RS | 1~4 |  | 2 |
| L1-RSRP reporting period | 1~4 | slot | 80 |
| T1 | 1~4 | s | 5 |
| T2 | 1~4 | s | 1 |
| EPRE ratio of PSS to SSS | 1~4 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~4 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.2.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.1.4.2.

16.6.4.2.4.3	Message contents

Same as the message contents given in clause 6.6.4.1.4.3.

16.6.4.2.5	Test requirement

Same as the test requirements given in clause 6.6.4.1.5 with following exceptions:

-	Table 6.6.4.1.5-1 is replaced by Table 16.6.4.2.5-1.

Table 16.6.4.2.5-1: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~4 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2,4 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~4 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1,2.4 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.15 |
| Io Note3 | 1,2.4 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/18.36 MHz | -60.77 | -60.77 | -63.79 | -58.67 |
|  | 1~4 | dB | 0 | 0 | -Infinity | 3.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void |  |  |  |  |  |  |

#### 16.6.4.3	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE

16.6.4.3.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in 38.133 [6] clause 9.5B.4.1.

16.6.4.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 onwards and supporting long DRX cycle.

16.6.4.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.3.

16.6.4.3.4	Test description

16.6.4.3.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.2.4.1 with following exceptions:

-	Table 6.6.4.2.4.1-1 is replaced by Table 16.6.4.3.4.1-1.

-	Table 6.6.4.2.4.1-2 is replaced by Table 16.6.4.3.4.1-3.

-	Table 6.6.4.2.4.1-3 is replaced by Table 16.6.4.3.4.1-2.

Table 16.6.4.3.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.6.4.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.3-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.6.4.3.4.1-2: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.3.4.1-3: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2,3 |  | TDD |
|  | 4 |  | HD-FDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |
|  | 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |
|  | 3 |  | SSB.2 RedCap FR1 |
| OCNG Patterns | 1~4 |  | OP.1 |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~4 |  | SMTC.1 RedCap |
| TRS Configuration | 1,4 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~4 |  | DRX.3 |
| reportConfigType | 1~4 |  | periodic |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |
| Number of reported RS | 1~4 |  | 2 |
| L1-RSRP reporting period | 1~4 | slot | 80 |
| T1 | 1~4 | s | 5 |
| T2 | 1~4 | s | 1 |
| EPRE ratio of PSS to SSS | 1~4 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~4 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.3.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.2.4.2 with following exceptions:

-	Step 5 in test procedure given in clause 6.6.4.2.4.2 is replaced by following step.

5.	The UE shall start sending valid L1-RSRP reports. The SS shall check following requirements:

- R1:	the UE shall start to transmit valid reports no later than 720 ms for configuration 1, 2, 4 and no later than 680 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement for SSB#1 (Table 16.6.4.3.5-2 for test configurations 1, 2, 4 and Table 16.6.4.3.5-3 for test configurations 3) and the relative L1-RSRP requirement for SSB#0 in Table 16.6.4.3.5-4. If the first valid report is received earlier than the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the UE shall transmit reports every 80 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

- R3:	The L1-RSRP value of SSB#1 reported by the UE is compared to the expected L1-RSRP value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 16.6.4.3.5-2 for test configurations 1, 2, 4 and 5 and in Table 16.6.4.3.5-3 for test configurations 3 and 6 or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4:	The DIFF RSRP value of SSB#0 reported by the UE is compared to the expected DIFF RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 16.6.4.3.5-4 or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

16.6.4.3.4.3	Message contents

Same as the message contents given in clause 6.6.4.2.4.3.

16.6.4.3.5	Test requirement

Same as the test requirements given in clause 6.6.4.2.5 with following exceptions:

-	Table 6.6.4.2.5-1 is replaced by Table 16.6.4.3.5-1.

-	Table 6.6.4.2.5-2 is replaced by Table 16.6.4.3.5-2.

-	Table 6.6.4.2.5-3 is replaced by Table 16.6.4.3.5-3.

-	Table 6.6.4.2.5-4 is replaced by Table 16.6.4.3.5-4.

Table 16.6.4.3.5-1: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~4 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2,4 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~4 | dB | 0 | 0 | -Infinity | 6.5 |
| SSB RSRP Note3 | 1,2.4 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -88.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -85.15 |
| Io Note3 | 1,2.4 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -59.32 |
|  | 3 | dBm/18.36 MHz | -60.77 | -60.77 | -63.79 | -56.39 |
|  | 1~4 | dB | 0 | 0 | -Infinity | 6.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 16.6.4.3.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (SSB#1) | 55 |
| Highest reported value (SSB#1) | 81 |

Table 16.6.4.3.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (SSB#1) | 58 |
| Highest reported value (SSB#1) | 84 |

Table 16.6.4.3.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T2 |
| --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | 0 |
| Highest DIFF RSRP reported (SSB#0) | 6 |

#### 16.6.4.4	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE

16.6.4.4.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in 38.133 [6] clause 9.5B.4.1.

16.6.4.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 onwards and supporting long DRX cycle.

16.6.4.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.4.

16.6.4.4.4	Test description

16.6.4.4.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.2.4.1 with following exceptions:

-	Table 6.6.4.2.4.1-1 is replaced by Table 16.6.4.4.4.1-1.

-	Table 6.6.4.2.4.1-2 is replaced by Table 16.6.4.4.4.1-3.

-	Table 6.6.4.2.4.1-3 is replaced by Table 16.6.4.4.4.1-2.

Table 16.6.4.4.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.6.4.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.4-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.4-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.6.4.4.4.1-2: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.4.4.1-3: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2,3 |  | TDD |
|  | 4 |  | HD-FDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |
|  | 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |
|  | 3 |  | SSB.2 RedCap FR1 |
| OCNG Patterns | 1~4 |  | OP.1 |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~4 |  | SMTC.1 RedCap |
| TRS Configuration | 1,4 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~4 |  | DRX.3 |
| reportConfigType | 1~4 |  | periodic |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |
| Number of reported RS | 1~4 |  | 2 |
| L1-RSRP reporting period | 1~4 | slot | 80 |
| T1 | 1~4 | s | 5 |
| T2 | 1~4 | s | 1 |
| EPRE ratio of PSS to SSS | 1~4 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~4 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.4.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.2.4.2.

16.6.4.4.4.3	Message contents

Same as the message contents given in clause 6.6.4.2.4.3.

16.6.4.4.5	Test requirement

Same as the test requirements given in clause 6.6.4.2.5 with following exceptions:

-	Table 6.6.4.2.5-1 is replaced by Table 16.6.4.4.5-1.

Table 16.6.4.4.5-1: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~4 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2,4 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~4 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1,2.4 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.15 |
| Io Note3 | 1,2.4 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/18.36 MHz | -60.77 | -60.77 | -63.79 | -58.67 |
|  | 1~4 | dB | 0 | 0 | -Infinity | 3.5Note 4 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### 16.6.4.5	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE

16.6.4.5.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.2.

16.6.4.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.4.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.5.

16.6.4.5.4	Test description

16.6.4.5.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.3.4.1 with following exceptions:

-	Table 6.6.4.3.4.1-1 is replaced by Table 16.6.4.5.4.1-1.

-	Table 6.6.4.3.4.1-2 is replaced by Table 16.6.4.5.4.1-2.

-	Table 6.6.4.3.4.1-3 is replaced by Table 16.6.4.5.4.1-3.

Table 16.6.4.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.4.5-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.5-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.5-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.5-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.6.4.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.4.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.5.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN |  |  | freq1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1,4 |  | N/A |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2,4 |  | SSB.3 FR1 |
|  | Config 3 |  | SSB.2 Redcap FR1 |
| CSI-RS configuration | Config 1,4 |  | CSI-RS 1.3 FDD |
|  | Config 2 |  | CSI-RS 1.3 TDD |
|  | Config 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns |  |  | OP.1 |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  |  | SMTC.1 RedCap |
| DRX configuration |  |  | Off |
| reportConfigType |  |  | aperiodic |
| reportQuantity |  |  | cri-RSRP |
| Number of reported RS |  |  | 2 |
| qcl-Info |  |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList |  | slots | 8 |
| T1 |  | s | 5 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.5.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.3.4.2.

16.6.4.5.4.3	Message contents

Same as the message contents given in clause 6.6.4.3.4.3.

16.6.4.5.5	Test requirement

Same as the test requirements given in clause 6.6.4.3.5 with following exceptions:

-	Table 6.6.4.3.5-1 is replaced by Table 16.6.4.5.5-1.

Table 16.6.4.5.5-1: Cell specific test parameters

| Parameter |  | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 |  | dBm/15kHz | -94.65 |  |
| Note1 | Config 1,2,4 | dBm/SCS | -94.65 |  |
|  | Config 3 |  | -91.65 |  |
|  |  | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | Config 1,2,4 | dBm/SSB SCS | -94.65 | -91.15 |
|  | Config 3 |  | -91.65 | -88.15 |
| Io Note2 | Config 1,2,4 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | Config 3 | dBm/18.36 MHz | -60.77 | -58.67 |
|  |  | dB | 0 | 3.5 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### 16.6.4.6	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE

16.6.4.6.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.2.

16.6.4.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.6.4.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.6.

16.6.4.6.4	Test description

16.6.4.6.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.3.4.1 with following exceptions:

-	Table 6.6.4.3.4.1-1 is replaced by Table 16.6.4.6.4.1-1.

-	Table 6.6.4.3.4.1-2 is replaced by Table 16.6.4.6.4.1-2.

-	Table 6.6.4.3.4.1-3 is replaced by Table 16.6.4.6.4.1-3.

Table 16.6.4.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.4.6-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.6-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.6-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.6-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.6.4.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.4.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.6.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN |  |  | freq1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1,4 |  | N/A |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2,4 |  | SSB.3 FR1 |
|  | Config 3 |  | SSB.2 Redcap FR1 |
| CSI-RS configuration | Config 1,4 |  | CSI-RS 1.3 FDD |
|  | Config 2 |  | CSI-RS 1.3 TDD |
|  | Config 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns |  |  | OP.1 |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  |  | SMTC.1 RedCap |
| DRX configuration |  |  | Off |
| reportConfigType |  |  | aperiodic |
| reportQuantity |  |  | cri-RSRP |
| Number of reported RS |  |  | 2 |
| qcl-Info |  |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList |  | slots | 8 |
| T1 |  | s | 5 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.6.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.3.4.2.

16.6.4.6.4.3	Message contents

Same as the message contents given in clause 6.6.4.3.4.3.

16.6.4.6.5	Test requirement

Same as the test requirements given in clause 6.6.4.3.5 with following exceptions:

-	Table 6.6.4.3.5-1 is replaced by Table 16.6.4.6.5-1.

Table 16.6.4.6.5-1: Cell specific test parameters

| Parameter |  | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 |  | dBm/15kHz | -94.65 |  |
| Note1 | Config 1,2,4 | dBm/SCS | -94.65 |  |
|  | Config 3 |  | -91.65 |  |
|  |  | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | Config 1,2,4 | dBm/SSB SCS | -94.65 | -91.15 |
|  | Config 3 |  | -91.65 | -88.14 |
| Io Note2 | Config 1,2,4 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | Config 3 | dBm/18.36 MHz | -60.77 | -58.67 |
|  |  | dB | 0 | 3.5 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### 16.6.4.7	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE

16.6.4.7.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.2.

16.6.4.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting long DRX cycle.

16.6.4.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.7.

16.6.4.7.4	Test description

16.6.4.7.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.4.4.1 with following exceptions:

-	Table 6.6.4.4.4.1-1 is replaced by Table 16.6.4.7.4.1-1.

-	Table 6.6.4.4.4.1-2 is replaced by Table 16.6.4.7.4.1-2.

-	Table 6.6.4.4.4.1-3 is replaced by Table 16.6.4.7.4.1-3.

Table 16.6.4.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.4.7-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.7-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.7-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.7-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.6.4.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.4.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.7.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN |  |  | freq1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1,4 |  | N/A |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2,4 |  | SSB.3 FR1 |
|  | Config 3 |  | SSB.2 Redcap FR1 |
| CSI-RS configuration | Config 1,4 |  | CSI-RS 1.3 FDD |
|  | Config 2 |  | CSI-RS 1.3 TDD |
|  | Config 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns |  |  | OP.1 |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  |  | SMTC.1 RedCap |
| DRX configuration |  |  | DRX.3 |
| reportConfigType |  |  | aperiodic |
| reportQuantity |  |  | cri-RSRP |
| Number of reported RS |  |  | 2 |
| qcl-Info |  |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList |  | slots | 8 |
| T1 |  | s | 5 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.7.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.4.4.2.

16.6.4.7.4.3	Message contents

Same as the message contents given in clause 6.6.4.4.4.3.

16.6.4.7.5	Test requirement

Same as the test requirements given in clause 6.6.4.4.5 with following exceptions:

-	Table 6.6.4.4.5-1 is replaced by Table 16.6.4.7.5-1.

Table 16.6.4.7.5-1: Cell specific test parameters

| Parameter |  | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 |  | dBm/15kHz | -94.65 |  |
| Note1 | Config 1,2,4 | dBm/SCS | -94.65 |  |
|  | Config 3 |  | -91.65 |  |
|  |  | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | Config 1,2,4 | dBm/SSB SCS | -94.65 | -91.15 |
|  | Config 3 |  | -91.65 | -88.15 |
| Io Note2 | Config 1,2,4 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | Config 3 | dBm/18.36 MHz | -60.77 | -58.67 |
|  |  | dB | 0 | 3.5 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### 16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE

16.6.4.8.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.2.

16.6.4.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting long DRX cycle.

16.6.4.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.8.

16.6.4.8.4	Test description

16.6.4.8.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.4.4.1 with following exceptions:

-	Table 6.6.4.4.4.1-1 is replaced by Table 16.6.4.8.4.1-1.

-	Table 6.6.4.4.4.1-2 is replaced by Table 16.6.4.8.4.1-2.

-	Table 6.6.4.4.4.1-3 is replaced by Table 16.6.4.8.4.1-3.

Table 16.6.4.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.4.8-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.8-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.8-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.8-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.6.4.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.4.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.8.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN |  |  | freq1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1,4 |  | N/A |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2,4 |  | SSB.3 FR1 |
|  | Config 3 |  | SSB.2 Redcap FR1 |
| CSI-RS configuration | Config 1,4 |  | CSI-RS 1.3 FDD |
|  | Config 2 |  | CSI-RS 1.3 TDD |
|  | Config 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns |  |  | OP.1 |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  |  | SMTC.1 RedCap |
| DRX configuration |  |  | DRX.3 |
| reportConfigType |  |  | aperiodic |
| reportQuantity |  |  | cri-RSRP |
| Number of reported RS |  |  | 2 |
| qcl-Info |  |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList |  | slots | 8 |
| T1 |  | s | 5 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.8.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.4.4.2.

16.6.4.8.4.3	Message contents

Same as the message contents given in clause 6.6.4.4.4.3.

16.6.4.8.5	Test requirement

Same as the test requirements given in clause 6.6.4.4.5 with following exceptions:

-	Table 6.6.4.4.5-1 is replaced by Table 16.6.4.8.5-1.

Table 16.6.4.8.5-1: Cell specific test parameters

| Parameter |  | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 |  | dBm/15kHz | -94.65 |  |
| Note1 | Config 1,2,4 | dBm/SCS | -94.65 |  |
|  | Config 3 |  | -91.65 |  |
|  |  | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | Config 1,2,4 | dBm/SSB SCS | -94.65 | -91.15 |
|  | Config 3 |  | -91.65 | -88.14 |
| Io Note2 | Config 1,2,4 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | Config 3 | dBm/18.36 MHz | -60.77 | -58.67 |
|  |  | dB | 0 | 3.5 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

### 16.6.5	NR measurements with autonomous gaps

#### 16.6.5.0	Minimum conformance requirements

##### 16.6.5.0.1	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE

The requirements in this clause are specified for CGI identification of an NR target cell and are applicable for a UE in RRC_CONNECTED state and configured with SA operation mode.

The UE shall identify and report the CGI of a known NR target cell when requested by the network for the purpose of reportCGI. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI. The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [13]. Note that a UE is not required to use autonomous gap if useAutonomousGaps is set to false. If autonomous gaps are used for measurement with the purpose of reportCGI, regardless of whether DRX is used or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI_redcap = (TMIB_redcap + T SIB1_redcap) ms

Where:

TMIB_redcap is the time period used to acquire MIB message. TMIB_redcap = 6 * TSMTC ms for target cell carrier frequency on FR1.

TSIB1_redcap is the time period used to acquire SIB1 message. TSIB1_redcap = 12 * TRMSI-scheduling ms.

Where TSMTC is the SMTC periodicity configured for the target cell measurement, and TRMSI-scheduling is

-	the periodicity with which the SIB1 is actually transmitted by the NR target cell when SSB and RMSI CORESET multiplexing pattern is 1

-	the maximum between the periodicity with which the SIB1 is actually transmitted by the NR target cell and TSMTC when SSB and RMSI CORESET multiplexing pattern is 2 or 3.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI_redcap is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [13] is used.

Within the time Tidentify_CGI_redcap, over which the UE identifies the CGI of an NR cell, the UE shall fulfil interruption requirements specified in,

-	Clause [8.2.2.2.14] of TS 38.133 [6] if the UE is configured with SA operation mode.

In the requirement a cell is known if,

-	During the last 5 seconds before the reception of the report CGI command:

-	The UE has sent a valid L3-RSRP measurement report with SSB index for the target cell and

-	During MIB decoding at least reported SSBs remains detectable according to the cell identification conditions specified in clauses 9.2B or 9.3B of TS 38.133 [6], and

-	During SIB1 decoding the SSB used for MIB decoding remains detectable according to the cell identification conditions specified in clauses 9.2B or 9.3B of TS 38.133 [6], and

-	During MIB decoding, the SSB for MIB decoding remains detectable with SNR ≥-3dB

-	During SIB1 decoding, the PDSCH for SIB1 decoding remains detectable with SNR ≥-3dB

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_redcap plus RRC procedure delay defined in clause 12 in TS 38.331 [13].

When a RedCap UE is identifying CGI of an NR cell with autonomous gaps, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on PCell for the following slots:

-	with up to K1 interruptions with interrupted slots up to X1 specified in Table 16.6.5.0.1-1 for each interruption during MIB decoding time period TMIB_redcap (ms).

-	with up to L1 interruptions with interrupted slots up to Y1 specified in Table 16.6.5.0.1-1 during SIB1 decoding time period TSIB1_redcap (ms) for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to L2 interruptions with interrupted slots up to interruption length Y2 specified in Table 16.6.5.0.1-1 during SIB1 decoding time period TSIB1_redcap (ms) for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Where:

-	K1 = 6 for the target cell carrier frequency on FR1, and

-	L1 = TSIB1_redcap /20 and

-	L2 = TSIB1_redcap /TSMTC, where TSMTC is the periodicity of the SMTC occasion configured for the target cell carrier.

Table 16.6.5.0.1-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |

The normative reference for this requirement is TS 38.133 [6] clause 9.11A.

##### 16.6.5.0.2	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE

Same as clause 16.6.5.0.1 with the following exception:

TSIB1_redcap = 6 * TRMSI-scheduling ms

The normative reference for this requirement is TS 38.133 [6] clause 9.11A.

##### 16.6.5.0.3	Minimum conformance requirements for CGI identification of an E-UTRA cell with autonomous gaps

The requirements in this clause apply when the 1Rx RedCap UE or 2Rx RedCap UE is configured with standalone NR. The UE shall identify and report the CGI when requested by an NR PCell for the purpose ‘reportCGI’. The UE may make autonomous gaps in downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3.1 in TS 38.331 [13].

For 2Rx RedCap UE: If autonomous gaps are used for measurement with the purpose of ‘reportCGI’, the UE shall be able to identify a new CGI of E-UTRA cell withinTidentify_CGI_RedCap, E-UTRAN= 150 ms.

For 1Rx RedCap UE: If autonomous gaps are used for measurement with the purpose of ‘reportCGI’, the UE shall be able to identify a new CGI of E-UTRA cell within Tidentify_CGI_RedCap, E-UTRAN= 190 ms.

This is the maximum allowed time for the UE to identify a new CGI of an E-UTRA cell, provided that the E-UTRA cell has been already identified by the UE.

A cell shall be considered identifiable following conditions are fulfilled:

-	RSRP related side conditions given in Clause 9.1 in TS 36.133 [23] are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex B.2.2 in TS 36.133 [23] for a corresponding Band

The MIB of an E-UTRA cell whose CGI is identified shall be considered decodable by the UE provided the PBCH demodulation requirements are met according to TS 36.101 [27].

The requirement for identifying a new CGI of an E-UTRA cell within Tidentify_CGI_RedCap, E-UTRAN is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [13] is used.

The E-UTRA CGI reporting delay is defined as the time between a command that will trigger an E-UTRA CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_RedCap, E-UTRAN plus RRC procedure delay defined in clause 12 in TS 38.331 [13], and an additional 30ms margin.

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA TDD cell with autonomous gaps, within time period Tidentify_CGI_RedCap, E-UTRA specified in this clause, the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 16.6.5.0.3-1 on PCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 16.6.5.0.3-1: Minimum number of ACK/NACKs transmitted by the UE during Tidentify_CGI_RedCap, E-UTRAN

| Minimum number of transmitted ACK/NACKs | SCS |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| 84 | FDD | 15 kHz |
| 193 | FDD | 30 kHz |
| 28 | TDD Note 1 | 15 kHz |
| 81 | TDD Note 1 | 30 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [2]. |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.4A.4.

#### 16.6.5.1	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	Test Procedure and Message Contents are incomplete.

-	TT analysis is missing.

16.6.5.1.1	Test Purpose

The purpose of this test is to verify that the UE makes correct reporting of intra-frequency CGI identification of an NR neighbour cell in FR1 with autonomous gaps. This test shall partly verify the measurement requirements in clause 9.11A of TS 38.133 [6].

16.6.5.1.2	Test Applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 onwards and supporting acquisition of CGI from neighbour NR cell using autonomous gap.

16.6.5.1.3	Minimum Conformance Requirements

The minimum conformance requirements are specified in clause 16.6.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.5.1.

16.6.5.1.4	Test Description

16.6.5.1.4.1	Initial conditions

This test can be run in the configuration defined in Table 16.6.5.1.4.1-1.

Table 16.6.5.1.4.1-1: Supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 16.6.5.1-1 | 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.5.1-2 | 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.5.1-3 | 3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.5.1-4 | 4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.6.5.1.4.1-2.

Table 16.6.5.1.4.1-2: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration from Table 16.6.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 16.6.5.1.4.1-3.

2.	Message contents are defined in clause 16.6.5.1.4.3.

3.	There are two cells: Cell 1 and Cell 2 specified in the test. Cell 1 is the FR1 PCell and Cell 2 is an FR1 neighbour cell on the same frequency as the PCell. The Cell 1 and Cell 2 are configured according to the settings in Annex C.1.1 and C.1.2.

Table 16.6.5.1.4.1-3: General test parameters for SA intra-frequency CGI identification of NR neighbour cell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3, 4 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3, 4 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3, 4 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3, 4 | -4.5 |  |
| CP length |  | 1, 2, 3, 4 | Normal |  |
| Hysteresis | dB | 1, 2, 3, 4 | 0 |  |
| Time To Trigger | s | 1, 2, 3, 4 | 0 |  |
| Filter coefficient |  | 1, 2, 3, 4 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 4 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 ms | Synchronous cells |
|  |  | 3 | 3 ms | Synchronous cells |
| T1 | s | 1, 2, 3, 4 | 5 |  |
| T2 | s | 1, 2, 3, 4 | 5 |  |
| T3 | s | 1, 2, 3, 4 | 5 |  |

16.6.5.1.4.2	Test procedure

The test consists of two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is an FR1 neighbour cell on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.5.1.4.1-3 and Table 16.6.5.1.5-1, respectively. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 16.6.5.1.5-1. T1 starts.

3. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4. The SS shall switch the power setting from T1 to T2 as specified in Table 16.6.5.1.5-1. T2 starts.

5. SS shall transmit an RRCReconfiguration message to configure the measurement object for the frequency of the serving cell and to indicate the UE that event-triggered reporting with Event A3 is used.

6. The UE shall transmit RRCReconfigurationComplete message.

7. UE shall transmit a MeasurementReport message triggered by Event A3.

8.  After the SS receives the MeasurementReport message in step 6, the SS shall transmit RRCReconfiguration message triggering CGI identification and configuring reportCGI and useAutonomousGaps IEs set to TRUE.

9. T3 starts when the last TTI containing the above RRC message implying CGI identification is sent to the UE.

10. The UE shall transmit RRCReconfigurationComplete message.

11. The SS verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell during T3 until a measurement report with CGI is sent. If the UE sends a measurement report containing the CGI of Cell 2 within 372 ms from the start of time period T3, then the number of successful tests is increased by one otherwise the number of failure tests is increased by one.12. Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.6.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.6.5.1.4.3-1: Common Exception messages for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with condition SSB Index and A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with condition INTRA-FREQ and SSB Index Table H.3.1-8 with condition SSB RLM Table 4.6.3-142 of TS 38.508-1 [14] with condition CGI and AutoGaps |

16.6.5.1.5	Test Requirements

Tables 16.6.5.1.4.1-3 and 16.6.5.1.5-1 define the general parameters and cell specific parameters for this test.

Table 16.6.5.1.5-1: NR Cell specific test parameters for SA intra-frequency CGI identification of NR neighbour cell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3, 4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1, 4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 4 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 4 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1, 4 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/18.36 MHz | 3 | -61.67 | -59.32 | -61.67 | -59.32 |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send a measurement report containing the CGI of cell 2 within 372 ms from the start of time period T3.

Test requirement = RRC Procedure delay + Tidentify_CGI_redcap + reporting delay = 10 + 360 + 2ms from the start of T3= 372 ms, allow 375ms.

The UE shall be scheduled continuously throughout the test. From the start of T3 until 375 ms, the interruption on Pcell shall not be more than the values specified for SA in TS 38.133 [6] clause 9.11A.

The rate of correct events observed during repeated tests shall be at least 90%.

#### 16.6.5.2	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE

16.6.5.2.1	Test Purpose

The purpose of this test is to verify that the UE makes correct reporting of intra-frequency CGI identification of an NR neighbour cell in FR1 with autonomous gaps. This test shall partly verify the measurement requirements in clause 9.11A of TS 38.133 [6].

16.6.5.2.2	Test Applicability

This test applies to all types of NR RedCap UE with 2 Rx from release 17 onwards and supporting acquisition of CGI from neighbour NR cell using autonomous gap.

16.6.5.2.3	Minimum Conformance Requirements

The minimum conformance requirements are specified in clause 16.6.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.5.2.

16.6.5.2.4	Test Description

16.6.5.2.4.1	Initial conditions

This test can be run in the configuration defined in Table 16.6.5.2.4.1-1.

Table 16.6.5.2.4.1-1: Supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 16.6.5.2-1 | 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.5.2-2 | 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.5.2-3 | 3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.5.2-4 | 4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.6.5.2.4.1-2.

Table 16.6.5.2.4.1-2: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration from Table 16.6.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 16.6.5.2.4.1-3.

2.	Message contents are defined in clause 16.6.5.2.4.3.

3.	There are two cells: Cell 1 and Cell 2 specified in the test. Cell 1 is the FR1 PCell and Cell 2 is an FR1 neighbour cell on the same frequency as the PCell. The Cell 1 and Cell 2 are configured according to the settings in Annex C.1.1 and C.1.2.

Table 16.6.5.2.4.1-3: General test parameters for SA intra-frequency CGI identification of NR neighbour cell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3, 4 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3, 4 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3, 4 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3, 4 | -4.5 |  |
| CP length |  | 1, 2, 3, 4 | Normal |  |
| Hysteresis | dB | 1, 2, 3, 4 | 0 |  |
| Time To Trigger | s | 1, 2, 3, 4 | 0 |  |
| Filter coefficient |  | 1, 2, 3, 4 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 4 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 ms | Synchronous cells |
|  |  | 3 | 3 ms | Synchronous cells |
| T1 | s | 1, 2, 3, 4 | 5 |  |
| T2 | s | 1, 2, 3, 4 | 5 |  |
| T3 | s | 1, 2, 3, 4 | 5 |  |

16.6.5.2.4.2	Test procedure

Same as in clause 16.6.5.1.4.2.

16.6.5.2.4.3	Message contents

Same as in clause 16.6.5.1.4.3.

16.6.5.2.5	Test Requirements

Tables 16.6.5.2.4.1-3 and 16.6.5.2.5-1 define the general parameters and cell specific parameters for this test.

Table 16.6.5.2.5-1: NR Cell specific test parameters for SA intra-frequency CGI identification of NR neighbour cell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3, 4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1, 4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 4 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1, 4 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1, 4 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1, 4 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 4 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1, 4 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/18.36 MHz | 3 | -61.67 | -59.32 | -61.67 | -59.32 |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send a measurement report containing the CGI of cell 2 within 255 ms from the start of time period T3.

Test requirement = RRC Procedure delay + Tidentify_CGI + reporting delay = 10 + 240 + 2ms from the start of T3 = 252 ms, allow 255 ms.

The UE shall be scheduled continuously throughout the test. From the start of T3 until 255 ms, the interruption on PCell shall not be more than the values specified for SA in TS 38.133 [6] clause 9.11A.

The rate of correct events observed during repeated tests shall be at least 90%.

#### 16.6.5.3	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.6.5.3.1	Test purpose

To verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in TS 38.133 clause 9.4A.4.

16.6.5.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards and supporting acquisition of CGI from neighbour E-UTRA cell using autonomous gap.

16.6.5.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.5.3.

16.6.5.3.4	Test description

The test scenario comprises of one NR carrier and an E-UTRA carrier and two cells. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively:

-	At the start of time duration T1, the UE does not have any timing information of Cell 2.

-	Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B2. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to setup.

-	The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

16.6.5.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.5.3.4.1-1.

Table 16.6.5.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.5.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.6.5.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.6.5.3-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.6.5.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.6.5.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.6.5.3-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.6.5.3-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 16.6.5.3-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.6.5.3.4.1-2

Table 16.6.5.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 16.6.5.3.4.3.

2.	The power levels and settings for NR PCell (Cell 1) and E-UTRA neighbour cell (Cell 2) are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 16.3.1.3.4.1-3 below.

Table 16.6.5.3.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.6.1.1 |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | setup | As specified in clause 5.5.3.1 in TS 38.331[13]. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

16.6.5.3.4.2	Test procedure

Throughout the test, the UE shall be scheduled to perform DL reception on every slot suitable for DL.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 16.6.5.3.5-1 and Table 16.6.5.3.5-2. T1 starts.

3.	The SS transmits an RRCReconfiguration message to configure event B2 triggered measurement reporting on E-UTRA Cell 2.

4.	The UE transmits an RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.6.5.3.5-1 and 16.6.5.3.5-2, T2 starts.

6.	UE transmits a MeasurementReport message triggered by Event B2.

7.	After receiving MeasurementReport message in step 6, the SS transmits an RRCReconfiguration message to configure an autonomous gap based CGI measurement reporting on E-UTRA Cell 2.

8.	The start of T3 is the instant when the last TTI containing the RRCReconfiguration message in step 7 is sent to the UE, at that instant the SS shall switch the power setting from T2 to T3 as specified in Table 16.6.5.3.5-1 and 16.6.5.3.5-2.

9.	The UE transmits an RRCReconfigurationComplete message.

10. If the UE

a)	transmits a MeasurementReport message containing CGI of E-UTRA Cell 2 less than 235 ms from the beginning of time period T3, and

b)	transmits ACK/NACKs no less than the minimum number of ACK/NACKs given in Table 16.6.5.3.5-5 from the start of T3 until 235 ms,

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

11.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

12.	Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.6.5.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.6.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 |

Table 16.6.1.1.4.3-2: MeasConfig (Step 3 & 8, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-RAT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-B2 | Table 16.6.1.1.4.3-3 | Step 3 |
|  | ReportConfigInterRAT-CGI | Table 16.6.1.1.4.3-5 | Step 8 |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-DEFAULT specified in Table H.3.1-6 with condition gapUE and Pattern #0 |  | Step 3 |
| measGapConfig SEQUENCE { |  |  | Step 8 |
| gapUE CHOICE { |  |  |  |
| release | null | MG needs to be released to apply interruption requirements |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-3: ReportConfigInterRAT-B2 (Table 16.6.1.1.4.3-2)

| Derivation Path: Table H.3.1-4A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | 58 | Actual threshold = 58-156 = -98dBm/SCS | SCS15 |
|  | 61 | Actual threshold = 61-156 = -95dBm/SCS | SCS30 |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | 31 | Actual threshold = 31-140 = -109dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-4: MeasResults (Step 6, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measResultServingMOList | Not checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA { | 1 entry |  |  |
| MeasResultEUTRA[1] SEQUENCE { |  | Entry 1 |  |
| eutra-PhysCellId | EUTRA-PhysCellId of Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrp | (0..97) |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-5: ReportConfigInterRAT-CGI (Table 16.6.1.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-141 with condition CGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| reportCGI SEQUENCE { |  |  |  |
| cellForWhichToReportCGI | EUTRA-PhysCellId of Cell 2 |  |  |
| useAutonomousGaps-r16 | setup |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-6: MeasResults (Step 10, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measResultServingMOList | Not checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA { | 1 entry |  |  |
| MeasResultEUTRA[1] SEQUENCE { |  | Entry 1 |  |
| eutra-PhysCellId | EUTRA-PhysCellId of Cell 2 |  |  |
| measResult | Not checked |  |  |
| cgi-Info | Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.6.5.3.5	Test requirements

Table 16.6.5.3.4.1-3, Table 16.6.5.3.5-1 and Table 16.6.5.3.5-2 define the primary level settings including test tolerances for the test.

Table 16.6.5.3.5-3: NR cell specific test parameters (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |  |
| Duplex mode |  |  | 1,4 | FDD |  |  |
|  |  |  | 2,3,5,6 | TDD |  |  |
|  |  |  | 7,8 | HD-FDD |  |  |
| TDD Configuration |  |  | 2,5 | TDDConf.1.1 |  |  |
|  |  |  | 3,6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1,2,4,5,7,8 | 10: NRB,c = 52 |  |  |
|  |  |  | 3,6 | 20: NRB,c = 51 |  |  |
| PDSCH reference measurement channel |  |  | 1,4,7,8 | SR.1.1 FDD |  |  |
|  |  |  | 2,5 | SR.1.1 TDD |  |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1,4,7,8 | CR.1.1 FDD |  |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |  |
| BWP configurations | Initial DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  | 1,2,3,4,5,6,7,8 | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  | 1,2,3,4,5,6,7,8 | ULBWP.1.1 |  |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 |  |  |
| SSB configuration |  |  | 1,2,4,5,7,8 | SSB.1 FR1 |  |  |
|  |  |  | 3,6 | SSB.1 RedCap FR1 |  |  |
| b2-Threshold1 |  | dBm/SCS | 1,2,4,5,7,8 | -98 |  |  |
|  |  |  | 3,6 | -95 |  |  |
| b2-Threshold2EUTRA |  | dBm/15kHz | 1,2,3,4,5,6,7,8 | -109 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -106 |  |  |
| NocNote2 |  | dBm/SCS | 1,2,4,5,7,8 | -106 |  |  |
|  |  |  | 3,6 | -103 |  |  |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | 18 | -2 | -2 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | 18 | -2 | -2 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -88 | -108 | -108 |
|  |  |  | 3,6 | -85 | -105 | -105 |
| SSB_RPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -88 | -108 | -108 |
|  |  |  | 3,6 | -85 | -105 | -105 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,7,8 | -59.98 | -75.92 | -75.92 |
|  |  | dBm/18.36 MHz | 3,6 | -57.05 | -72.99 | -72.99 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration |  |  | 1,2,3,4,5,6,7,8 | 1x1 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 16.6.5.3.5-4: E-UTRA cell specific test parameters (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |  |
|  |  | 4,5,6,8 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 10 MHz: NRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1,2,3,7 | 10 MHz: R.3 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1,2,3,7 | 10 MHz: R.6 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1,2,3,7 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -106 |  |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7 | 7 |
| Ês/IotNote5 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7 | 7 |
| RSRPNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -99 | -99 |
| SCH_RPNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -99 | -99 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration |  | 1,2,3,4,5,6,7,8 | 1x1 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall transmit a measurement report containing the cell global identifier of Cell 2 within overall delay from the start of T3, where the overall delay is calculated as follows,

Overall delay = TRRC Processing+Tidentify_CGI_RedCap, E-UTRAN + additional margin + TTI insertion uncertainty

-	TRRC Processing = 10 ms, is the RRC procedure delay specified in 38.331 [13] clause 12;

-	Tidentify_CGI_RedCap, E-UTRAN = 190 ms, is the maximum allowed time for the UE to identify a new CGI of an E-UTRA cell;

-	TTI insertion uncertainty = 2 x TTIDCCH = 2ms, is the delay uncertainty when inserting the measurement report to the TTI of the uplink DCCH;

-	additional margin = 30 ms, is specified in 38.133 [6] clause 9.4A.4.2

Which gives overall delay = 10 + 190 + 30 + 2 = 232 ms, allow 235 ms.

From the start of T3 until 235 ms. The number of ACK/NACK being transmitted by the UE shall be no less than the minimum number of ACK/NACK given in Table 16.6.5.3.5-5.

The rate of correct events observed during repeated tests shall be at least 90%.

Table 16.6.5.3.5-5: Minimum number of ACK/NACKs transmitted by the UE during the overall delay

| Test configuration | Minimum number of transmitted ACK/NACKsNote 1 |
| --- | --- |
| 1,4,7,8 | 129 |
| 2,5 | 55 |
| 3,6 | 135 |
| NOTE 1: The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of cell 2, where X is defined in 38.133 [6] Table 9.4A.4.3-1. Secondly, given that continuous DL data allocation, additional 45, 27 and 54 ACK/NACK shall be sent for FDD/HD-FDD 15 kHz, TDD 15 kHz and TDD 30 kHz, respectively, from the start of T3 until 235 ms excludes 190 ms for identifying the cell global identifier of cell 2. |  |

#### 16.6.5.4	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.6.5.4.1	Test purpose

To verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in TS 38.133 clause 9.4A.4.

16.6.5.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting acquisition of CGI from neighbour E-UTRA cell using autonomous gap.

16.6.5.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.5.4.

16.6.5.4.4	Test description

Same as the test description given in clause 16.6.5.3.4

16.6.5.4.4.1	Initial conditions

Same as the initial conditions given in clause 16.6.5.3.4 with following exceptions:

-	Table 16.6.5.3.4.1-1 is replaced by Table 16.6.5.4.4.1-1;

-	Table 16.6.5.3.4.1-2 is replaced by Table 16.6.5.4.4.1-2;

-	Table 16.6.5.3.4.1-3 is replaced by Table 16.6.5.4.4.1-3.

Table 16.6.5.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.5.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.6.5.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.6.5.4-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.6.5.4-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.6.5.4-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.6.5.4-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.6.5.4-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 16.6.5.4-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.6.5.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.5.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.6.5.4.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.6.1.1 |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | setup | As specified in clause 5.5.3.1 in TS 38.331[13]. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

16.6.5.4.4.2	Test procedure

Same as the test procedure given in clause 16.6.5.3.4.2 with following exceptions:

-	Step 10 in clause 16.6.5.3.4.2 is replaced by following step:

10.	If the UE

a)	transmits a MeasurementReport message containing CGI of E-UTRA Cell 2 less than 195 ms from the beginning of time period T3, and

b)	transmits ACK/NACKs no less than the minimum number of ACK/NACKs given in Table 16.6.5.4.5-5 from the start of T3 until 195 ms,

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

16.6.5.4.4.3	Message contents

Same as the message contents given in clause 16.6.5.3.4.3.

16.6.5.4.5	Test requirements

Table 16.6.5.4.4.1-3, Table 16.6.5.4.5-1 and Table 16.6.5.4.5-2 define the primary level settings including test tolerances for the test.

Table 16.6.5.4.5-3: NR cell specific test parameters (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |  |
| Duplex mode |  |  | 1,4 | FDD |  |  |
|  |  |  | 2,3,5,6 | TDD |  |  |
|  |  |  | 7,8 | HD-FDD |  |  |
| TDD Configuration |  |  | 2,5 | TDDConf.1.1 |  |  |
|  |  |  | 3,6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1,2,4,5,7,8 | 10: NRB,c = 52 |  |  |
|  |  |  | 3,6 | 20: NRB,c = 51 |  |  |
| PDSCH reference measurement channel |  |  | 1,4,7,8 | SR.1.1 FDD |  |  |
|  |  |  | 2,5 | SR.1.1 TDD |  |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1,4,7,8 | CR.1.1 FDD |  |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |  |
| BWP configurations | Initial DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  | 1,2,3,4,5,6,7,8 | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  | 1,2,3,4,5,6,7,8 | ULBWP.1.1 |  |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 |  |  |
| SSB configuration |  |  | 1,2,4,5,7,8 | SSB.1 FR1 |  |  |
|  |  |  | 3,6 | SSB.1 RedCap FR1 |  |  |
| b2-Threshold1 |  | dBm/SCS | 1,2,4,5,7,8 | -98 |  |  |
|  |  |  | 3,6 | -95 |  |  |
| b2-Threshold2EUTRA |  | dBm/15kHz | 1,2,3,4,5,6,7,8 | -109 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -106 |  |  |
| NocNote2 |  | dBm/SCS | 1,2,4,5,7,8 | -106 |  |  |
|  |  |  | 3,6 | -103 |  |  |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | 18 | -2 | -2 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | 18 | -2 | -2 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -88 | -108 | -108 |
|  |  |  | 3,6 | -85 | -105 | -105 |
| SSB_RPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -88 | -108 | -108 |
|  |  |  | 3,6 | -85 | -105 | -105 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,7,8 | -59.98 | -75.92 | -75.92 |
|  |  | dBm/18.36 MHz | 3,6 | -57.05 | -72.99 | -72.99 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 16.6.5.4.5-4: E-UTRA cell specific test parameters (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |  |
|  |  | 4,5,6,8 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 10 MHz: NRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1,2,3,7 | 10 MHz: R.3 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1,2,3,7 | 10 MHz: R.6 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1,2,3,7 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -106 |  |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7 | 7 |
| Ês/IotNote5 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7 | 7 |
| RSRPNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -99 | -99 |
| SCH_RPNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -99 | -99 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall transmit a measurement report containing the cell global identifier of Cell 2 within overall delay from the start of T3, where the overall delay is calculated as follows,

Overall delay = TRRC Processing+Tidentify_CGI_RedCap, E-UTRAN + additional margin + TTI insertion uncertainty

-	TRRC Processing = 10 ms, is the RRC procedure delay specified in 38.331 [13] clause 12;

-	Tidentify_CGI_RedCap, E-UTRAN = 150 ms, is the maximum allowed time for the UE to identify a new CGI of an E-UTRA cell;

-	TTI insertion uncertainty = 2 x TTIDCCH = 2ms, is the delay uncertainty when inserting the measurement report to the TTI of the uplink DCCH;

-	additional margin = 30 ms, is specified in 38.133 [6] clause 9.4A.4.2

Which gives overall delay = 10 + 150 + 30 + 2 = 192 ms, allow 195 ms.

From the start of T3 until 195 ms. The number of ACK/NACK being transmitted by the UE shall be no less than the minimum number of ACK/NACK given in Table 16.6.5.4.5-5.

The rate of correct events observed during repeated tests shall be at least 90%.

Table 16.6.5.4.5-5: Minimum number of ACK/NACKs transmitted by the UE during the overall delay

| Test configuration | Minimum number of transmitted ACK/NACKsNote 1 |
| --- | --- |
| 1,4,7,8 | 129 |
| 2,5 | 55 |
| 3,6 | 135 |
| NOTE 1: The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of cell 2, where X is defined in 38.133 [6] Table 9.4A.4.3-1. Secondly, given that continuous DL data allocation, additional 45, 27 and 54 ACK/NACK shall be sent for FDD/HD-FDD 15 kHz, TDD 15 kHz and TDD 30 kHz, respectively, from the start of T3 until 195 ms excludes 150 ms for identifying the cell global identifier of cell 2. |  |

## 16.7	Measurement performance requirements for RedCap

### 16.7.1	SS-RSRP for RedCap

#### 16.7.1.0	Minimum conformance requirements

##### 16.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements

The accuracy requirements in clause 6.7.1.0.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 16.7.1.0.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 16.7.1.0.1-1: SS-RSRP Intra frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 5.5 | 10 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 9 | 12 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.2.1 and 10.1.6.

##### 16.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements

The accuracy requirements in clause 6.7.1.0.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 16.7.1.0.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 16.7.1.0.2-1: SS-RSRP Intra frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.2.1 and 10.1.6.

##### 16.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements

The accuracy requirements in clause 6.7.1.0.3 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 16.7.1.0.3-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 16.7.1.0.3-1: SS-RSRP Inter frequency Absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 5.5 | 10 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 9 | 12 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.4.1.1 and 10.1.6.

##### 16.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements

The accuracy requirements in clause 6.7.1.0.4 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 16.7.1.0.4-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 16.7.1.0.4-1: SS-RSRP Inter frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 5.5 | 7 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.4.1.2 and 10.1.6.

#### 16.7.1.1	Intra-frequency measurements for RedCap

##### 16.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.

-	Message Contents is incomplete

-	Applicability is missing in 38.522

16.7.1.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.1.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.1.

16.7.1.1.1.4	Test description

16.7.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.1.1.4.1-1.

Table 16.7.1.1.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.1.1-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.1.1.4.1-2.

Table 16.7.1.1.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.1.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.1.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.1.4.2 with following exceptions:

-	Table 6.7.1.1.1.5-1 is replaced by Table 16.7.1.1.1.5-1.

-	Table 6.7.1.1.1.5-2 is replaced by Table 16.7.1.1.1.5-2.

16.7.1.1.1.4.3	Message contents

TBD

16.7.1.1.1.5	Test requirement

Table 16.7.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 16.7.1.1.1.5-2 for test configurations 1, 2 and 4 and the corresponding absolute accuracy requirements in Table 16.7.1.1.1.5-3 for test configuration 3.

Table 16.7.1.1.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration |  | Config 1, 4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| BWP BW |  | Config 1, 4 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1, 4 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1, 4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1, 4 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB.1 RedCap FR1 | SSB.1 RedCap FR1 |
| Time offset with Cell 1 |  | Config 1, 4 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1, 4 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 | Depending on band group | dBm/15KhZ | -106 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -94 |  | -114 + ΔBG_offset |  |
| Note2 | Config 1,2,4 |  | dBm/SCS | -106 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -91 |  | -111 + ΔBG_offset |  |
|  |  |  | dB | 2.46+TT | -5.97+TT | 2.46+TT | -5.97+TT | -0.01+TT | -4.76+TT |
|  |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4 | Depending on band group | dBm/SCS | -100+ TT | -105+ TT | -82+ TT | -87+ TT | -111.00 + ΔBG_offset+ TT | -114.00 + ΔBG_offset+ TT |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 | Not applicableNote 5 | -85+ TT | -90+ TT | -108.00 + ΔBG_offset+ TT | -111.00 + ΔBG_offset+ TT |
| IoNote3 | Config 1,2,4 | Depending on band group | dBm/ 9.36MHz | -70.09 |  | -52.09 |  | -80.03 + ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | Not applicableNote 5- |  | -55.18 |  | -77.12 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x1 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 16.7.1.1.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | FFS | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | FFS | FFS | NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | NR_FDD_FR1_B | FFS |
|  |  |  | NR_TDD_FR1_C | FFS |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | FFS |
|  |  |  | NR_FDD_FR1_G | FFS |
|  |  |  | NR_FDD_FR1_H | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | FFS | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | FFS | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 16.7.1.1.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | N/A | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | N/A | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 16.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.

-	Message Contents is incomplete

-	Applicability is missing in 38.522

16.7.1.1.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.1.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.1.

16.7.1.1.2.4	Test description

16.7.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.1.2.4.1-1.

Table 16.7.1.1.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.1.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.1.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.1.2-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.1.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.1.2.4.1-2.

Table 16.7.1.1.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.1.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.1.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.2.4.2 with following exceptions:

-	Table 6.7.1.1.2.5-1 is replaced by Table 16.7.1.1.2.5-1.

-	Table 6.7.1.1.2.5-2 is replaced by Table 16.7.1.1.2.5-2.

16.7.1.1.2.4.3	Message contents

Message contents are same as in clause 16.7.1.1.1.4.3.

16.7.1.1.2.5	Test requirement

Table 16.7.1.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.1.2.5-1 shall meet the corresponding relative accuracy requirements in Table 16.7.1.1.2.5-2.

Table 16.7.1.1.2.5-1: Same as Table 16.7.1.1.1.5-1 with the following exceptions:

TBD

Table 16.7.1.1.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

TBD

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 16.7.1.2.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.

-	Message Contents is incomplete

-	Applicability is missing in 38.522

16.7.1.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.1.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.2.

16.7.1.2.1.4	Test description

16.7.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.2.1.4.1-1.

Table 16.7.1.2.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.1-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.2.1.4.1-2.

Table 16.7.1.2.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.2.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.1.4.2 with following exceptions:

-	Table 6.7.1.1.1.5-1 is replaced by Table 16.7.1.2.1.5-1.

-	Table 6.7.1.1.1.5-2 is replaced by Table 16.7.1.2.1.5-2.

16.7.1.2.1.4.3	Message contents

TBD

16.7.1.2.1.5	Test requirement

Table 16.7.1.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 16.7.1.2.1.5-2 for test configurations 1, 2 and 4 and the corresponding absolute accuracy requirements in Table 16.7.1.2.1.5-3 for test configuration 3.

Table 16.7.1.2.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| BWP BW |  | Config 1,4 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1,4 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB.1 RedCap FR1 | SSB.1 RedCap FR1 |
| Time offset with Cell 1 |  | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 | Depending on band group | dBm/15KhZ | -106 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -94 |  | -114 + ΔBG_offset |  |
| Note2 | Config 1,2,4 |  | dBm/SCS | -106 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -91 |  | -111 + ΔBG_offset |  |
|  |  |  | dB | 2.46+ TT | -5.97+ TT | 2.46+ TT | -5.97+ TT | -0.01+ TT | -4.76+ TT |
|  |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4 | Depending on band group | dBm/SCS | -100+ TT | -105+ TT | -82+ TT | -87+ TT | -111.00 + ΔBG_offset+ TT | -114.00 + ΔBG_offset+ TT |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 | Not applicableNote 5 | -85+ TT | -90+ TT | -108.00 + ΔBG_offset+ TT | -111.00 + ΔBG_offset+ TT |
| IoNote3 | Config 1,2,4 | Depending on band group | dBm/ 9.36MHz | -70.09 |  | -52.09 |  | -80.03 + ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | Not applicableNote 5- |  | -55.18 |  | -77.12 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 16.7.1.2.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | FFS | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | FFS | FFS | NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | NR_FDD_FR1_B | FFS |
|  |  |  | NR_TDD_FR1_C | FFS |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | FFS |
|  |  |  | NR_FDD_FR1_G | FFS |
|  |  |  | NR_FDD_FR1_H | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | FFS | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | FFS | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 16.7.1.2.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | N/A | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | N/A | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  |  | Bands NR_FDD_FR1_B | FFS |
|  |  |  | Bands NR_TDD_FR1_C | FFS |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  |  | Bands NR_FDD_FR1_G | FFS |
|  |  |  | Bands NR_FDD_FR1_H | FFS |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 16.7.1.2.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.

-	Message Contents is incomplete

-	Applicability is missing in 38.522

16.7.1.2.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.1.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.2.

16.7.1.2.2.4	Test description

16.7.1.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.2.2.4.1-1.

Table 16.7.1.2.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.2.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.2.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.2-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.2.2.4.1-2.

Table 16.7.1.2.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.2.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.2.4.2 with following exceptions:

-	Table 6.7.1.1.2.5-1 is replaced by Table 16.7.1.2.2.5-1.

-	Table 6.7.1.1.2.5-2 is replaced by Table 16.7.1.2.2.5-2.

16.7.1.2.2.4.3	Message contents

Message contents are same as in clause 16.7.1.2.1.4.3.

16.7.1.2.2.5	Test requirement

Table 16.7.1.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.2.2.5-1 shall meet the corresponding relative accuracy requirements in Table 16.7.1.2.2.5-2.

Table 16.7.1.2.2.5-1: Same as Table 16.7.1.2.1.5-1 with the following exceptions:

TBD

Table 16.7.1.2.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

TBD

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 16.7.2	SS-RSRQ

#### 16.7.2.0	Minimum conformance requirements

##### 16.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 4.7.2.0.1 shall apply.

When RedCap UE is only required to support 1Rx, the absolute accuracy requirements in Table 16.7.2.0.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to TS 38.133 [6] Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 16.7.2.0.1-1: SS-RSRQ Intra frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.5 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 4.5 | 5 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in 38.133 [6] clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.6.1.1.

##### 16.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 4.7.2.0.2 shall apply.

When RedCap UE is only required to support 1Rx, the absolute accuracy requirements in Table 16.7.2.0.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to TS 38.133 [6] Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 16.7.2.0.2-1: SS-RSRQ Inter frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.5 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 4.5 | 5 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.8.1.1.

##### 16.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 4.7.2.0.3 shall apply.

When RedCap UE is only required to support 1Rx, the relative accuracy requirements in Table 16.7.2.0.3-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to TS 38.133 [6] Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 16.7.2.0.3-1: SS-RSRQ Inter frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 5 | 5 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.8.1.2.

#### 16.7.2.1

#### 16.7.2.2

#### 16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

##### 16.7.2.3.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.2.3.1.1	Test purpose

To verify that the inter-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.8.1.1

16.7.2.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.7.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.2.3.

16.7.2.3.1.4	Test description

16.7.2.3.1.4.1	Initial conditions

Same as the initial conditions given in 6.7.2.2.1 with followings exceptions:

-	Table 6.7.2.2.1.4.1-1 is replaced by Table 16.7.2.3.1.4.1-1;

-	Table 6.7.2.2.1.4.1-2 is replaced by Table 16.7.2.3.1.4.1-2;

Table 16.7.2.3.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.2.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.2.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.2.3.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.2.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.2.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.2.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.7.2.3.1.4.2	Test procedure

Same as the test procedure in clause 6.7.2.1.4.2 with following exceptions:

-	Table 6.7.2.2.1.5-1 is replaced by Table 16.7.2.3.1.5-1;

-	Table 6.7.2.2.1.5-2 is replaced by Table 16.7.2.3.1.5-2.

16.7.2.3.1.4.3	Message contents

Same as the message contents in clause 6.7.2.2.1.4.3 with following exceptions:

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.2.3.1-3.

16.7.2.3.1.5	Test requirement

Same as the test requirements in clause 6.7.2.2.1.5 with followings exceptions:

-	Table 6.7.2.2.1.5-1 is replaced by Table 16.7.2.3.1.5-1;

-	Table 6.7.2.2.1.5-2 is replaced by Table 16.7.2.3.1.5-2;

Table 16.7.2.3.1.5-1: NR cell specific test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| Gap pattern ID |  |  | 0 |  |  |  |  |  |
| BWP BW | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH RMC | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | Config 1,4 |  | CR.1.1 FDD | - | R.1.1 FDD | - | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |  |
| Time offset with Cell 1 | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |  |  |
|  | Config 3 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 | dBm/ 15kHz | -80.18 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -86.27 |  | -113 |  | -116 + ΔBG_offset |  |
| Note2 | Config 1,2,4 | dBm/ SCS | -80.18 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -83.27 |  | -110 |  | -113 + ΔBG_offset |  |
|  |  | dB | -1.75 |  | -1.75 |  | 3 | -1.75 |
|  |  | dB | -1.75 |  | -1.75 |  | 3 | -1.75 |
| SS-RSRPNote3 | Config 1,2,4 | dBm/ SCS | -81.93 |  | -107.75 |  | -113+ ΔBG_offset | -117.7+ ΔBG_offset |
|  | Config 3 |  | -85.02 |  | -111.75 |  | -110+ ΔBG_offset | -114.7+ ΔBG_offset |
| SS-RSRQNote3 |  | dB | -14.77 |  | -14.77 |  | -12.56 | -14.76 |
| IoNote3 | Config 1,2,4 | dBm/ Ch BW | -50 |  | -75.83 |  | -83.28+ ΔBG_offset | -85.83+ ΔBG_offset |
|  | Config 3 |  | -53.17 |  | -79.90 |  | -80.36+ ΔBG_offset | -82.90+ ΔBG_offset |
| Propagation condition |  |  | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | 1x1 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

Table 16.7.2.3.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_57+TT | SS-RSRQ_57+TT | SS-RSRQ_57+TT |
| Highest reported value (Cell 2) | SS-RSRQ_57+TT | SS-RSRQ_57+TT | SS-RSRQ_57+TT |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_57+TT | SS-RSRQ_57+TT | SS-RSRQ_57+TT |
| Highest reported value (Cell 2) | SS-RSRQ_57+TT | SS-RSRQ_57+TT | SS-RSRQ_57+TT |

##### 16.7.2.3.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.2.3.2.1	Test purpose

To verify that the inter-frequency SS-RSRQ relative measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.8.1.2.

16.7.2.3.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.7.2.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.2.3.

16.7.2.3.2.4	Test description

16.7.2.3.2.4.1	Initial conditions

Same as the initial conditions given in 6.7.2.2.2 with followings exceptions:

-	Table 6.7.2.2.2.4.1-1 is replaced by Table 16.7.2.3.2.4.1-1;

-	Table 6.7.2.2.2.4.1-2 is replaced by Table 16.7.2.3.2.4.1-2;

Table 16.7.2.3.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.2.3.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.2.3.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.2.3.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.2.3.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.2.3.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.2.3.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.7.2.3.2.4.2	Test procedure

Same as the test procedure in clause 6.7.2.2.2.4.2 with following exceptions:

-	Table 6.7.2.2.2.5-1 is replaced by Table 16.7.2.3_1.5-1;

-	Table 6.7.2.2.2.5-2 is replaced by Table 16.7.2.3.2.5-2.

16.7.2.3.2.4.3	Message contents

Same as the message contents in clause 6.7.2.2.2.4.3 with following exceptions:

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.2.3.2-3.

16.7.2.3.2.5	Test requirement

Same as the test requirements in clause 6.7.2.2.2.5 with followings exceptions:

-	Table 6.7.2.2.2.5-1 is replaced by Table 16.7.2.3_1.5-1;

-	Table 6.7.2.2.2.5-2 is replaced by Table 16.7.2.3_1.5-2.

Table 16.7.2.3.2.5-1: SS-RSRQ Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x +TT | SS-RSRQ_x +TT | SS-RSRQ_x +TT |
| Highest reported value (Cell 2) | SS-RSRQ_x +TT | SS-RSRQ_x +TT | SS-RSRQ_x +TT |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x +TT | SS-RSRQ_x +TT | SS-RSRQ_x +TT |
| Highest reported value (Cell 2) | SS-RSRQ_x +TT | SS-RSRQ_x +TT | SS-RSRQ_x +TT |
| NOTE: SS-RSRQ_x is the reported value of Cell 1. |  |  |  |

#### 16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

##### 16.7.2.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.2.4.1.1	Test purpose

To verify that the inter-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.8.1.1

16.7.2.4.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.7.2.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.2.4.

16.7.2.4.1.4	Test description

16.7.2.4.1.4.1	Initial conditions

Same as the initial conditions given in 6.7.2.2.1 with followings exceptions:

-	Table 6.7.2.2.1.4.1-1 is replaced by Table 16.7.2.4.1.4.1-1;

-	Table 6.7.2.2.1.4.1-2 is replaced by Table 16.7.2.4.1.4.1-2;

Table 16.7.2.4.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.2.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.2.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.2.4.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.2.4.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.2.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.2.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

16.7.2.4.1.4.2	Test procedure

Same as the test procedure in clause 6.7.2.2.1.4.2 with followings exceptions:

-	Table 6.7.2.2.1.5-1 is replaced by Table 16.7.2.4.1.5-1;

-	Table 6.7.2.2.1.5-2 is replaced by Table 16.7.2.4.1.5-2.

16.7.2.4.1.4.3	Message contents

Same as the message contents in clause 6.7.2.2.1.4.3 with following exceptions:

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.2.4.1-3.

16.7.2.4.1.5	Test requirement

Same as the test requirements in clause 6.7.2.2.1.5 with followings exceptions:

-	Table 6.7.2.2.1.5-1 is replaced by Table 16.7.2.4.1.5-1;

-	Table 6.7.2.2.1.5-2 is replaced by Table 16.7.2.4.1.5-2;

Table 16.7.2.4.1.5-1: NR cell specific test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| Gap pattern ID |  |  | 0 |  |  |  |  |  |
| BWP BW | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH RMC | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | Config 1,4 |  | CR.1.1 FDD | - | R.1.1 FDD | - | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |  |
| Time offset with Cell 1 | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |  |  |
|  | Config 3 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 | dBm/ 15kHz | -80.18 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -86.27 |  | -113 |  | -116 + ΔBG_offset |  |
| Note2 | Config 1,2,4 | dBm/ SCS | -80.18 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -83.27 |  | -110 |  | -113 + ΔBG_offset |  |
|  |  | dB | -1.75 |  | -1.75 |  | 3 | -1.75 |
|  |  | dB | -1.75 |  | -1.75 |  | 3 | -1.75 |
| SS-RSRPNote3 | Config 1,2,4 | dBm/ SCS | -81.93 |  | -107.75 |  | -113+ ΔBG_offset | -117.7+ ΔBG_offset |
|  | Config 3 |  | -85.02 |  | -111.75 |  | -110+ ΔBG_offset | -114.7+ ΔBG_offset |
| SS-RSRQNote3 |  | dB | -14.77 |  | -14.77 |  | -12.56 | -14.76 |
| IoNote3 | Config 1,2,4 | dBm/ Ch BW | -50 |  | -75.83 |  | -83.28+ ΔBG_offset | -85.83+ ΔBG_offset |
|  | Config 3 |  | -53.17 |  | -79.90 |  | -80.36+ ΔBG_offset | -82.90+ ΔBG_offset |
| Propagation condition |  |  | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

Table 16.7.2.4.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_57+TT | SS-RSRQ_57+TT | SS-RSRQ_57+TT |
| Highest reported value (Cell 2) | SS-RSRQ_57+TT | SS-RSRQ_57+TT | SS-RSRQ_57+TT |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_57+TT | SS-RSRQ_57+TT | SS-RSRQ_57+TT |
| Highest reported value (Cell 2) | SS-RSRQ_57+TT | SS-RSRQ_57+TT | SS-RSRQ_57+TT |

##### 16.7.2.4.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.2.4.2.1	Test purpose

To verify that the inter-frequency SS-RSRQ relative measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.8.1.2.

16.7.2.4.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.7.2.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.2.4.

16.7.2.4.2.4	Test description

16.7.2.4.2.4.1	Initial conditions

Same as the initial conditions given in 6.7.2.2.2 with followings exceptions:

-	Table 6.7.2.2.2.4.1-1 is replaced by Table 16.7.2.4.2.4.1-1;

-	Table 6.7.2.2.2.4.1-2 is replaced by Table 16.7.2.4.2.4.1-2;

Table 16.7.2.4.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.2.4.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.2.4.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.2.4.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.2.4.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.2.4.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.2.4.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

16.7.2.4.2.4.2	Test procedure

Same as the test procedure in clause 6.7.2.2.2.4.2 with followings exceptions:

-	Table 6.7.2.2.2.5-1 is replaced by Table 16.7.2.3_1.5-1;

-	Table 6.7.2.2.2.5-2 is replaced by Table 16.7.2.4.2.5-1.

16.7.2.4.2.4.3	Message contents

Same as the message contents in clause 6.7.2.2.2.4.3 with following exceptions:

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.2.4_1-3.

16.7.2.4.2.5	Test requirement

Same as the test requirements in clause 6.7.2.2.2.5 with followings exceptions:

-	Table 6.7.2.2.2.5-1 is replaced by Table 16.7.2.3_1.5-1;

-	Table 6.7.2.2.2.5-2 is replaced by Table 16.7.2.4.2.5-1;

Table 16.7.2.4.2.5-1: SS-RSRQ Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x +TT | SS-RSRQ_x +TT | SS-RSRQ_x +TT |
| Highest reported value (Cell 2) | SS-RSRQ_x +TT | SS-RSRQ_x +TT | SS-RSRQ_x +TT |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x +TT | SS-RSRQ_x +TT | SS-RSRQ_x +TT |
| Highest reported value (Cell 2) | SS-RSRQ_x +TT | SS-RSRQ_x +TT | SS-RSRQ_x +TT |
| NOTE: SS-RSRQ_x is the reported value of Cell 1. |  |  |  |

### 16.7.3	SS-SINR

#### 16.7.3.0	Minimum conformance requirements

##### 16.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 4.7.3.0.1 shall apply.

When RedCap UE is only required to support 1Rx, the absolute accuracy requirements in Table 16.7.3.0.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to TS 38.133 [6] Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 16.7.3.0.1-1: SS-SINR Intra frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.0 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB under non-HST scenarios. NOTE 4: NR operating band groups in FR1 are as defined in TS 38.133 [6] clause 3.5.2. NOTE 5: The requirements apply for SSB Ês/Iot ≤5 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.10.1.1.

##### 16.7.3.0.2	Inter-frequency SS-SINR absolute measurement accuracy requirements

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 4.7.3.0.2 shall apply.

When RedCap UE is only required to support 1Rx, the absolute accuracy requirements in Table 16.7.3.0.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to TS 38.133 [6] Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 16.7.3.0.2-1: SS-SINR Inter frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.0 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 4.5 | 5 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups in FR1 are as defined in TS 38.133 [6] clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.12.1.1.

##### 16.7.3.0.3	Inter-frequency SS-SINR relative measurement accuracy requirements

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 4.7.3.0.3 shall apply.

When RedCap UE is only required to support 1Rx, the relative accuracy requirements in Table 16.7.3.0.3-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to TS 38.133 [6] Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 16.7.3.0.3-1: SS-SINR Inter frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.5 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 5 | 5 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ [25] dB. NOTE 5: NR operating band groups in FR1 are as defined in TS 38.133 [6] clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.12.1.2.

#### 16.7.3.1	NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.3.1.1	Test purpose

To verify that the intra-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.10.1.1

16.7.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards and supporting SS-SINR measurement.

16.7.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.3.1.

16.7.3.1.4	Test description

16.7.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.3.1.4.1 with followings exceptions:

-	Table 6.7.3.1.4.1-1 is replaced by Table 16.7.3.1.4.1-1;

-	Table 6.7.3.1.4.1-2 is replaced by Table 16.7.3.1.4.1-2;

Table 16.7.3.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.3.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.7.3.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.3.1.4.2 with following exceptions:

-	Table 6.7.3.1.5-1 is replaced by Table 16.7.3.1.5-1;

-	Table 6.7.3.1.5-2 is replaced by Table 16.7.3.1.5-2.

16.7.3.1.4.3	Message contents

Same as the message contents given in clause 6.7.3.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.3.1-1 applies to test configuration 16.7.3.1-4.

16.7.3.1.5	Test requirement

Same as the test requirements given in clause 6.7.3.1.5 with followings exceptions:

-	Table 6.7.3.1.5-1 is replaced by Table 16.7.3.1.5-1;

-	Table 6.7.3.1.5-2 is replaced by Table 16.7.3.1.5-2;

Table 16.7.3.1.5-1: NR cell specific test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 |  | freq1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  | ms | Not Applicable |  |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH RMC | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET RMC | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  | Not Applicable |  |  |  |
| SMTC configuration | Config 1,4 |  | SMTC.2 |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |
| Time offset with Cell 1 | Config 1,4 | ms | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -93 |  | -116 + ΔBG_offset |  |
| Note2 | Config 1,2,4 | dBm/SCS | -93 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -90 |  | -113 + ΔBG_offset |  |
|  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  | dB | 4.54 | 2.66 | -4 | -4 |
| SS-RSRPNote3 | Config 1,2,4 | dBm/SCS | -88.46 | -90.34 | -120 + ΔBG_offset | -120 + ΔBG_offset |
|  | Config 3 |  | -85.46 | -87.34 | -117 + ΔBG_offset | -117 + ΔBG_offset |
| SS-SINR Note3 |  | dB | 0 | -3.19 | -5.46 | -5.46 |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -57.5 |  | -85.51 + ΔBG_offset |  |
|  | Config 3 | dBm/ 18.36MHz | -54.57 |  | -82.57 + ΔBG_offset |  |
| Propagation condition |  | - | AWGN |  |  |  |
| Antenna configuration |  | - | 1x1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

Table 16.7.3.1.5-2: SS-SINR Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SINR_40+TT | SINR_36+TT |
| Highest reported value (Cell 2) | SINR_40+TT | SINR_36+TT |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SINR_40+TT | SINR_36+TT |
| Highest reported value (Cell 2) | SINR_40+TT | SINR_36+TT |

#### 16.7.3.2	NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.3.2.1	Test purpose

To verify that the intra-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.10.1.1

16.7.3.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting SS-SINR measurement.

16.7.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.3.2.

16.7.3.2.4	Test description

16.7.3.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.3.1.4.1 with followings exceptions:

-	Table 6.7.3.1.4.1-1 is replaced by Table 16.7.3.2.4.1-1;

-	Table 6.7.3.1.4.1-2 is replaced by Table 16.7.3.2.4.1-2;

Table 16.7.3.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.3.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.3.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.3.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.3.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.3.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.3.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

16.7.3.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.3.1.4.2 with following exceptions:

-	Table 6.7.3.1.5-1 is replaced by Table 16.7.3.2.5-1;

-	Table 6.7.3.1.5-2 is replaced by Table 16.7.3.2.5-2.

16.7.3.2.4.3	Message contents

Same as the message contents given in clause 6.7.3.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.3.1-1 applies to test configuration 16.7.3.2-4.

16.7.3.2.5	Test requirement

Same as the test requirements given in clause 6.7.3.1.5 with followings exceptions:

-	Table 6.7.3.1.5-1 is replaced by Table 16.7.3.2.5-1;

-	Table 6.7.3.1.5-2 is replaced by Table 16.7.3.2.5-2;

Table 16.7.3.2.5-1: NR cell specific test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 |  | freq1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  | ms | Not Applicable |  |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH RMC | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET RMC | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  | Not Applicable |  |  |  |
| SMTC configuration | Config 1,4 |  | SMTC.2 |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |
| Time offset with Cell 1 | Config 1,4 | ms | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -93 |  | -116 + ΔBG_offset |  |
| Note2 | Config 1,2,4 | dBm/SCS | -93 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -90 |  | -113 + ΔBG_offset |  |
|  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  | dB | 4.54 | 2.66 | -4 | -4 |
| SS-RSRPNote3 | Config 1,2,4 | dBm/SCS | -88.46 | -90.34 | -120 + ΔBG_offset | -120 + ΔBG_offset |
|  | Config 3 |  | -85.46 | -87.34 | -117 + ΔBG_offset | -117 + ΔBG_offset |
| SS-SINR Note3 |  | dB | 0 | -3.19 | -5.46 | -5.46 |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -57.5 |  | -85.51 + ΔBG_offset |  |
|  | Config 3 | dBm/ 18.36MHz | -54.57 |  | -82.57 + ΔBG_offset |  |
| Propagation condition |  | - | AWGN |  |  |  |
| Antenna configuration |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

Table 16.7.3.2.5-2: SS-SINR Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SINR_40+TT | SINR_36+TT |
| Highest reported value (Cell 2) | SINR_40+TT | SINR_36+TT |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SINR_40+TT | SINR_36+TT |
| Highest reported value (Cell 2) | SINR_40+TT | SINR_36+TT |

#### 16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

##### 16.7.3.3.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.3.3.1.1	Test purpose

To verify that the inter-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.12.1.1

16.7.3.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards and supporting SS-SINR measurement.

16.7.3.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.3.3.

16.7.3.3.1.4	Test description

16.7.3.3.1.4.1	Initial conditions

Same as the initial conditions given in 6.7.3.2.1.4.1 with followings exceptions:

-	Table 6.7.3.2.1.4.1-1 is replaced by Table 16.7.3.3.1.4.1-1;

-	Table 6.7.3.2.1.4.1-2 is replaced by Table 16.7.3.3.1.4.1-2;

Table 16.7.3.3.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.3.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.3.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.3.3.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.3.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.3.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.3.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.7.3.3.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.3.2.1.4.2 with following exceptions:

-	Table 6.7.3.2.1.5-1 is replaced by Table 16.7.3.3.1.5-1;

-	Table 6.7.3.2.1.5-2 is replaced by Table 16.7.3.3.1.5-2;

16.7.3.3.1.4.3	Message contents

Same as the message contents given in clause 6.7.3.2.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.3.2.1-1 applies to test configuration 16.7.3.3.1-4.

16.7.3.3.1.5	Test requirement

Same as the test requirements in clause 6.7.3.2.1.5 with followings exceptions:

-	Table 6.7.3.2.1.5-1 is replaced by Table 16.7.3.3.1.5-1;

-	Table 6.7.3.2.1.5-2 is replaced by Table 16.7.3.3.1.5-2;

Table 16.7.3.3.1.5-1: NR cell specific test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle configuration |  | ms | Not Applicable |  |  |  |  |  |
| Gap pattern ID |  |  | 0 | - | 0 | - | 0 | - |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH RMC | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  | Not Applicable |  |  |  |  |  |
| Time offset with Cell 1 | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |  |  |
|  | Config 3 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/ 15kHz | -88 |  | -108.5 |  | -119.5 + ΔBG_offset |  |
| Note2 | Config 1,2,4 | dBm/ SCS | -88 |  | -108.5 |  | -119.5 + ΔBG_offset |  |
|  | Config 3 |  | -85 |  | -105.5 |  | -116.5 + ΔBG_offset |  |
|  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
|  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| SS-RSRP Note3 | Config 1,2,4 | dBm/ SCS | -89.75 |  | -88.5 |  | -123.5 + ΔBG_offset |  |
|  | Config 3 |  | -86.75 |  | -85.5 |  | -120.5 + ΔBG_offset |  |
| SS-SINRNote3 |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -57.83 |  | -60.5 |  | -90.09 + ΔBG_offset |  |
|  | Config 3 | dBm/ 18.36MHz | -54.87 |  | -57.57 |  | -87.97 + ΔBG_offset |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  | - | 1x1 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

Table 16.7.3.3.1.5-2: SS-SINR Inter frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SINR_43+TT | SINR_86+TT | SINR_38+TT |
| Highest reported value (Cell 2) | SINR_43+TT | SINR_86+TT | SINR_38+TT |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SINR_43+TT | SINR_86+TT | SINR_38+TT |
| Highest reported value (Cell 2) | SINR_43+TT | SINR_86+TT | SINR_38+TT |

##### 16.7.3.3.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.3.3.2.1	Test purpose

To verify that the inter-frequency SS-SINR relative measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.12.1.2.

16.7.3.3.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards and supporting SS-SINR measurement.

16.7.3.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.3.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.3.3.

16.7.3.3.2.4	Test description

16.7.3.3.2.4.1	Initial conditions

Same as the initial conditions given in 6.7.3.2.2 with followings exceptions:

-	Table 6.7.3.2.2.4.1-1 is replaced by Table 16.7.3.3.2.4.1-1;

-	Table 6.7.3.2.2.4.1-2 is replaced by Table 16.7.3.3.2.4.1-2;

Table 16.7.3.3.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.3.3.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.3.3.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.3.3.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.3.3.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.3.3.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.3.3.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.7.3.3.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.3.2.2.4.2 with following exceptions:

-	Table 6.7.3.2.2.5-1 is replaced by Table 16.7.3.3_1.5-1;

-	Table 6.7.3.2.2.5-2 is replaced by Table 16.7.3.3.2.5-1;

16.7.3.3.2.4.3	Message contents

Same as the message contents given in clause 6.7.3.2.2.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.3.2.2-1 applies to test configuration 16.7.3.3.2-4.

16.7.3.3.2.5	Test requirement

Same as the test requirements given in clause 6.7.3.2.2.5 with followings exceptions:

-	Table 6.7.3.2.2.5-1 is replaced by Table 16.7.3.3_1.5-1;

-	Table 6.7.3.2.2.5-2 is replaced by Table 16.7.3.3.2.5-1;

Table 16.7.3.3.2.5-1: SS-SINR Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x +TT | SS-SINR_x +TT | SS-SINR_x +TT |
| Highest reported value (Cell 2) | SS-SINR_x +TT | SS-SINR_x +TT | SS-SINR_x +TT |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x +TT | SS-SINR_x +TT | SS-SINR_x +TT |
| Highest reported value (Cell 2) | SS-SINR_x +TT | SS-SINR_x +TT | SS-SINR_x +TT |
| NOTE: SS-SINR_x is the reported value of Cell 1. |  |  |  |

#### 16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

##### 16.7.3.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.3.4.1.1	Test purpose

To verify that the inter-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.12.1.1

16.7.3.4.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting SS-SINR measurement.

16.7.3.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.3.4.

16.7.3.4.1.4	Test description

16.7.3.4.1.4.1	Initial conditions

Same as the initial conditions given in 6.7.3.2.1 with followings exceptions:

-	Table 6.7.3.2.1.4.1-1 is replaced by Table 16.7.3.4.1.4.1-1;

-	Table 6.7.3.2.1.4.1-2 is replaced by Table 16.7.3.4.1.4.1-2;

Table 16.7.3.4.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.3.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.3.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.3.4.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.3.4.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.3.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.3.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

16.7.3.4.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.3.2.1.4.2 with following exceptions:

-	Table 6.7.3.2.1.5-1 is replaced by Table 16.7.3.4.1.5-1;

-	Table 6.7.3.2.1.5-2 is replaced by Table 16.7.3.4.1.5-2;

16.7.3.4.1.4.3	Message contents

Same as the message contents given in clause 6.7.3.2.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.3.2.1-1 applies to test configuration 16.7.3.4.1-4.

16.7.3.4.1.5	Test requirement

Same as the test requirements in clause 6.7.3.2.1.5 with followings exceptions:

-	Table 6.7.3.2.1.5-1 is replaced by Table 16.7.3.4.1.5-1;

-	Table 6.7.3.2.1.5-2 is replaced by Table 16.7.3.4.1.5-2;

Table 16.7.3.4.1.5-1: NR cell specific test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle configuration |  | ms | Not Applicable |  |  |  |  |  |
| Gap pattern ID |  |  | 0 | - | 0 | - | 0 | - |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH RMC | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  | Not Applicable |  |  |  |  |  |
| Time offset with Cell 1 | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |  |  |
|  | Config 3 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/ 15kHz | -88 |  | -108.5 |  | -119.5 + ΔBG_offset |  |
| Note2 | Config 1,2,4 | dBm/ SCS | -88 |  | -108.5 |  | -119.5 + ΔBG_offset |  |
|  | Config 3 |  | -85 |  | -105.5 |  | -116.5 + ΔBG_offset |  |
|  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
|  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| SS-RSRP Note3 | Config 1,2,4 | dBm/ SCS | -89.75 |  | -88.5 |  | -123.5 + ΔBG_offset |  |
|  | Config 3 |  | -86.75 |  | -85.5 |  | -120.5 + ΔBG_offset |  |
| SS-SINRNote3 |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -57.83 |  | -60.5 |  | -90.09 + ΔBG_offset |  |
|  | Config 3 | dBm/ 18.36MHz | -54.87 |  | -57.57 |  | -87.97 + ΔBG_offset |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

Table 16.7.3.4.1.5-2: SS-SINR Inter frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SINR_43+TT | SINR_86+TT | SINR_38+TT |
| Highest reported value (Cell 2) | SINR_43+TT | SINR_86+TT | SINR_38+TT |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SINR_43+TT | SINR_86+TT | SINR_38+TT |
| Highest reported value (Cell 2) | SINR_43+TT | SINR_86+TT | SINR_38+TT |

##### 16.7.3.4.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

Editor’s Notes: This test case is incomplete in following aspects:

  - TT analysis is still missing.

16.7.3.4.2.1	Test purpose

To verify that the inter-frequency SS-SINR relative measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.12.1.2.

16.7.3.4.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting SS-SINR measurement.

16.7.3.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.3.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.2.3.

16.7.3.4.2.4	Test description

16.7.3.4.2.4.1	Initial conditions

Same as the initial conditions given in 6.7.3.2.2 with followings exceptions:

-	Table 6.7.3.2.2.4.1-1 is replaced by Table 16.7.3.4.2.4.1-1;

-	Table 6.7.3.2.2.4.1-2 is replaced by Table 16.7.3.4.2.4.1-2;

Table 16.7.3.4.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.3.4.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.3.4.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.3.4.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.3.4.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.3.4.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.3.4.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

16.7.3.4.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.3.2.2.4.2 with following exceptions:

-	Table 6.7.3.2.2.5-1 is replaced by Table 16.7.3.4_1.5-1;

-	Table 6.7.3.2.2.5-2 is replaced by Table 16.7.3.4.2.5-1.

16.7.3.4.2.4.3	Message contents

Same as the message contents given in clause 6.7.3.2.2.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.3.2.2-1 applies to test configuration 16.7.3.4.2-4.

16.7.3.4.2.5	Test requirement

Same as the test requirements in clause 6.7.3.2.2.5 with followings exceptions:

-	Table 6.7.3.2.2.5-1 is replaced by Table 16.7.3.4_1.5-1;

-	Table 6.7.3.2.2.5-2 is replaced by Table 16.7.3.4.2.5-1;

Table 16.7.3.4.2.5-1: SS-SINR Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x +TT | SS-SINR_x +TT | SS-SINR_x +TT |
| Highest reported value (Cell 2) | SS-SINR_x +TT | SS-SINR_x +TT | SS-SINR_x +TT |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x +TT | SS-SINR_x +TT | SS-SINR_x +TT |
| Highest reported value (Cell 2) | SS-SINR_x +TT | SS-SINR_x +TT | SS-SINR_x +TT |
| NOTE: SS-SINR_x is the reported value of Cell 1. |  |  |  |

### 16.7.4	L1-RSRP measurement for beam reporting

#### 16.7.4.0	Minimum conformance requirements

##### 16.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 4.7.4.0.1 shall apply.

When RedCap UE is only required to support 1Rx, the absolute accuracy requirements in Table 16.7.4.0.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to TS 38.133 [6] Annex B.4.1 for a corresponding Band for each relevant SSB.

Table 16.7.4.0.1-1: SSB based L1-RSRP absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±8.0 | ±12.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| ±11.5 | ±14.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.14.1.1.

##### 16.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 4.7.4.0.2 shall apply.

When RedCap UE is only required to support 1Rx, the relative accuracy requirements in Table 16.7.4.0.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to TS 38.133 [6] Annex B.4.1 for a corresponding Band for each relevant SSB.

Table 16.7.4.0.2-1: SSB based L1-RSRP relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±6 | ±7 | -3 | NR_FDD_FR1_D NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1A.14.1.2.

##### 16.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements

The minimum requirements given in clause 4.7.3.0.3 shall apply.

##### 16.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements

The minimum requirements given in clause 4.7.3.0.4 shall apply.

#### 16.7.4.1	SSB based L1-RSRP measurements for 1 Rx UE

##### 16.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 1 Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.1.1.1	Test purpose

To verify that the SSB based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.4.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.1.

16.7.4.1.1.4	Test description

16.7.4.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.1.1.4.1 with following exceptions:

-	Table 6.7.4.1.1.4.1-1 is replaced by Table 16.7.4.1.1.4.1-1;

-	Table 6.7.4.1.1.4.1-2 is replaced by Table 16.7.4.1.1.4.1-2;

Table 16.7.4.1.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.1.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.1.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.1.1.4.2.

16.7.4.1.1.4.3	Message contents

Same as the message contents given in clause 6.7.4.1.1.4.3.

16.7.4.1.1.5	Test requirement

Same as the thest requirements given in clause 6.7.4.1.1.5 with following exceptions:

-	Table 6.7.4.1.1.5-1 is replaced by Table 16.7.4.1.1.5-1;

-	Table 6.7.4.1.1.5-2 is replaced by Table 16.7.4.1.1.5-2;

Table 16.7.4.1.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2,3 |  | TDD |  |
|  | 4 |  | HD-FDD |  |
| TDD Configuration | 1,4 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | 3 |  | 20: NRB,c = 51 |  |
| PDSCH RMC | 1,4 |  | SR.1.1 FDD |  |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | 1,4 |  | CR.1.1 FDD |  |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | 1,4 |  | CCR.1.1 FDD |  |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |  |
|  | 3 |  | SSB.2 RedCap FR1 |  |
| OCNG Patterns | 1~4 |  | OP.1 |  |
| TRS configuration | 1,4 |  | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~4 |  | SMTC.1 |  |
| reportConfigType | 1~4 |  | periodic |  |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |  |
| Number of reported RS | 1~4 |  | 2 |  |
| L1-RSRP reporting period | 1~4 |  | slot80 |  |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | 1~4 | dBm/ 15kHz | -94.65 | -117+ ΔBG_offset |
| Note2 | 1,2,4 | dBm/ SCS | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -91.65 | -114+ ΔBG_offset |
|  | 1~4 | dB | 10 | -3 |
| SS-RSRP Note3 | 1,2,4 | dBm/ SCS | -84.65 | -120+ ΔBG_offset |
|  | 3 |  | -81.65 | -117 + ΔBG_offset |
| Io Note3 | 1,2,4 | dBm/ 9.36 MHz | -56.28 | -87.28 + ΔBG_offset |
|  | 3 | dBm/ 18.36 MHz | -53.36 | -84.36 + ΔBG_offset |
|  | 1~4 | dB | 10 | -3 |
| Propagation condition | 1~4 |  | AWGN |  |
| Antenna configuration | 1~4 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |

Table 16.7.4.1.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Highest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Highest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 16.7.4.1.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Highest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Highest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

##### 16.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement for 1 Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.1.2.1	Test purpose

To verify that the SSB based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.4.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.1.

16.7.4.1.2.4	Test description

16.7.4.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.1.1.4.1 with following exceptions:

-	Table 6.7.4.1.2.4.1-1 is replaced by Table 16.7.4.1.2.4.1-1;

-	Table 6.7.4.1.2.4.1-2 is replaced by Table 16.7.4.1.2.4.1-2;

Table 16.7.4.1.2.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.1.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.1.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.1.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.1.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.1.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.1.2.4.2.

16.7.4.1.2.4.3	Message contents

Same as the message contents given in clause 6.7.4.1.2.4.3.

16.7.4.1.2.5	Test requirement

Same as the thest requirements given in clause 6.7.4.1.2.5 with following exceptions:

-	Table 6.7.4.1.2.5-1 is replaced by Table 16.7.4.1.2.5-1;

-	Table 6.7.4.1.2.5-2 is replaced by Table 16.7.4.1.2.5-2;

Table 16.7.4.1.2.5-1: Same as Table 16.7.4.1_1.5-1

Table 16.7.4.1.2.5-2: L1-RSRP relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |
| Extreme Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |

#### 16.7.4.2	SSB based L1-RSRP measurements for 2 Rx UE

##### 16.7.4.2.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 2 Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.2.1.1	Test purpose

To verify that the SSB based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.4.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.2.

16.7.4.2.1.4	Test description

16.7.4.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.1.1.4.1 with following exceptions:

-	Table 6.7.4.1.1.4.1-1 is replaced by Table 16.7.4.2.1.4.1-1;

-	Table 6.7.4.1.1.4.1-2 is replaced by Table 16.7.4.2.1.4.1-2;

Table 16.7.4.2.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.2.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.1.1.4.2.

16.7.4.2.1.4.3	Message contents

Same as the message contents given in clause 6.7.4.1.1.4.3.

16.7.4.2.1.5	Test requirement

Same as the thest requirements given in clause 6.7.4.1.1.5 with following exceptions:

-	Table 6.7.4.1.1.5-1 is replaced by Table 16.7.4.2.1.5-1;

-	Table 6.7.4.1.1.5-2 is replaced by Table 16.7.4.2.1.5-2;

Table 16.7.4.2.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2,3 |  | TDD |  |
|  | 4 |  | HD-FDD |  |
| TDD Configuration | 1,4 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | 3 |  | 20: NRB,c = 51 |  |
| PDSCH RMC | 1,4 |  | SR.1.1 FDD |  |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | 1,4 |  | CR.1.1 FDD |  |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | 1,4 |  | CCR.1.1 FDD |  |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |  |
|  | 3 |  | SSB.2 RedCap FR1 |  |
| OCNG Patterns | 1~4 |  | OP.1 |  |
| TRS configuration | 1,4 |  | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~4 |  | SMTC.1 |  |
| reportConfigType | 1~4 |  | periodic |  |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |  |
| Number of reported RS | 1~4 |  | 2 |  |
| L1-RSRP reporting period | 1~4 |  | slot80 |  |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | 1~4 | dBm/ 15kHz | -94.65 | -117+ ΔBG_offset |
| Note2 | 1,2,4 | dBm/ SCS | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -91.65 | -114+ ΔBG_offset |
|  | 1~4 | dB | 10 | -3 |
| SS-RSRP Note3 | 1,2,4 | dBm/ SCS | -84.65 | -120+ ΔBG_offset |
|  | 3 |  | -81.65 | -117 + ΔBG_offset |
| Io Note3 | 1,2,4 | dBm/ 9.36 MHz | -56.28 | -87.28 + ΔBG_offset |
|  | 3 | dBm/ 18.36 MHz | -53.36 | -84.36 + ΔBG_offset |
|  | 1~4 | dB | 10 | -3 |
| Propagation condition | 1~4 |  | AWGN |  |
| Antenna configuration and Correlation matrix | 1~4 |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |

Table 16.7.4.2.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Highest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Highest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 16.7.4.2.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Highest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Highest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

##### 16.7.4.2.2	NR SA FR1 SSB based L1-RSRP relative measurement for 2 Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.2.2.1	Test purpose

To verify that the SSB based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.4.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.4.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.2.

16.7.4.2.2.4	Test description

16.7.4.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.1.2.4.1 with following exceptions:

-	Table 6.7.4.1.2.4.1-1 is replaced by Table 16.7.4.2.2.4.1-1;

-	Table 6.7.4.1.2.4.1-2 is replaced by Table 16.7.4.2.2.4.1-2;

Table 16.7.4.2.2.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.2.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.2.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.2.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.2.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.2.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.1.2.4.2.

16.7.4.2.2.4.3	Message contents

Same as the message contents given in clause 6.7.4.1.2.4.3.

16.7.4.2.2.5	Test requirement

Same as the thest requirements given in clause 6.7.4.1.2.5 with following exceptions:

-	Table 6.7.4.1.2.5-1 is replaced by Table 16.7.4.2.2.5-1;

-	Table 6.7.4.1.2.5-2 is replaced by Table 16.7.4.2.2.5-2;

Table 16.7.4.2.2.5-1: Same as Table 16.7.4.2_1.5-1

Table 16.7.4.2.2.5-2: L1-RSRP relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |
| Extreme Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |

#### 16.7.4.3	CSI-RS based L1-RSRP measurements for 1 Rx UE

##### 16.7.4.3.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 1 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.3.1.1	Test purpose

To verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.4.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.4.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.3.

16.7.4.3.1.4	Test description

16.7.4.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.1.4.1 with following exceptions:

-	Table 6.7.4.2.1.4.1-1 is replaced by Table 16.7.4.3.1.4.1-1;

-	Table 6.7.4.2.1.4.1-2 is replaced by Table 16.7.4.3.1.4.1-2;

Table 16.7.4.3.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.3.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.3.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.2.1.4.2.

16.7.4.3.1.4.3	Message contents

Same as the message contents given in clause 6.7.4.2.1.4.3.

16.7.4.3.1.5	Test requirement

Same as the thest requirements given in clause 6.7.4.2.1.5 with following exceptions:

-	Table 6.7.4.2.1.5-1 is replaced by Table 16.7.4.3.1.5-1;

-	Table 6.7.4.2.1.5-2 is replaced by Table 16.7.4.3.1.5-2;

Table 16.7.4.3.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2,3 |  | TDD |  |
|  | 4 |  | HD-FDD |  |
| TDD Configuration | 1,4 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | 3 |  | 20: NRB,c = 51 |  |
| PDSCH RMC | 1,4 |  | SR.1.1 FDD |  |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | 1,4 |  | CR.1.1 FDD |  |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | 1,4 |  | CCR.1.1 FDD |  |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |  |
|  | 3 |  | SSB.2 RedCap FR1 |  |
| OCNG Patterns | 1~4 |  | OP.1 |  |
| TRS configuration | 1,4 |  | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~4 |  | SMTC.1 |  |
| CSI-RS configuration | 1,4 |  | CSI-RS 1.2 FDD |  |
|  | 2 |  | CSI-RS 1.2 TDD |  |
|  | 3 |  | CSI-RS 2.2 TDD |  |
| reportConfigType | 1~4 |  | periodic |  |
| reportQuantity | 1~4 |  | cri-RSRP |  |
| Number of reported RS | 1~4 |  | 2 |  |
| L1-RSRP reporting period | 1~4 |  | slot80 |  |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | 1~4 | dBm/ 15kHz | -94.65 | -117+ ΔBG_offset |
| Note2 | 1,2,4 | dBm/ SCS | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -91.65 | -114+ ΔBG_offset |
|  | 1~4 | dB | 10 | -3 |
| CSI-RSRP Note3 | 1,2,4 | dBm/ SCS | -84.65 | -120+ ΔBG_offset |
|  | 3 |  | -81.65 | -117 + ΔBG_offset |
| Io Note3 | 1,2,4 | dBm/ 9.36 MHz | -56.28 | -87.28 + ΔBG_offset |
|  | 3 | dBm/ 18.36 MHz | -53.36 | -84.36 + ΔBG_offset |
|  | 1~4 | dB | 10 | -3 |
| Propagation condition | 1~4 |  | AWGN |  |
| Antenna configuration | 1~4 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |

Table 16.7.4.3.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Highest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Highest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 16.7.4.3.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Highest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Highest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

##### 16.7.4.3.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 1 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.3.2.1	Test purpose

To verify that the CSI-RS based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.4.3.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.4.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.3.

16.7.4.3.2.4	Test description

16.7.4.3.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.2.4.1 with following exceptions:

-	Table 6.7.4.2.2.4.1-1 is replaced by Table 16.7.4.3.2.4.1-1;

-	Table 6.7.4.2.2.4.1-2 is replaced by Table 16.7.4.3.2.4.1-2;

Table 16.7.4.3.2.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.3.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.3.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.3.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.3.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.3.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.3.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.2.2.4.2.

16.7.4.3.2.4.3	Message contents

Same as the message contents given in clause 6.7.4.2.2.4.3.

16.7.4.3.2.5	Test requirement

Same as the thest requirements given in clause 6.7.4.2.2.5 with following exceptions:

-	Table 6.7.4.2.2.5-1 is replaced by Table 16.7.4.3.2.5-1;

-	Table 6.7.4.2.2.5-2 is replaced by Table 16.7.4.3.2.5-2;

Table 16.7.4.3.2.5-1: Same as Table 16.7.4.3_1.5-1

Table 16.7.4.3.2.5-2: L1-RSRP relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |
| Extreme Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |

#### 16.7.4.4	CSI-RS based L1-RSRP measurements for 2 Rx UE

##### 16.7.4.4.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 2 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.4.1.1	Test purpose

To verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.4.4.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.4.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.4.

16.7.4.4.1.4	Test description

16.7.4.4.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.1.4.1 with following exceptions:

-	Table 6.7.4.2.1.4.1-1 is replaced by Table 16.7.4.4.1.4.1-1;

-	Table 6.7.4.2.1.4.1-2 is replaced by Table 16.7.4.4.1.4.1-2;

Table 16.7.4.4.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.4.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.4.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.4.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.2.1.4.2.

16.7.4.4.1.4.3	Message contents

Same as the message contents given in clause 6.7.4.2.1.4.3.

16.7.4.4.1.5	Test requirement

Same as the thest requirements given in clause 6.7.4.2.1.5 with following exceptions:

-	Table 6.7.4.2.1.5-1 is replaced by Table 16.7.4.4.1.5-1;

-	Table 6.7.4.2.1.5-2 is replaced by Table 16.7.4.4.1.5-2;

Table 16.7.4.4.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2,3 |  | TDD |  |
|  | 4 |  | HD-FDD |  |
| TDD Configuration | 1,4 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | 3 |  | 20: NRB,c = 51 |  |
| PDSCH RMC | 1,4 |  | SR.1.1 FDD |  |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | 1,4 |  | CR.1.1 FDD |  |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | 1,4 |  | CCR.1.1 FDD |  |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |  |
|  | 3 |  | SSB.2 RedCap FR1 |  |
| OCNG Patterns | 1~4 |  | OP.1 |  |
| TRS configuration | 1,4 |  | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~4 |  | SMTC.1 |  |
| CSI-RS configuration | 1,4 |  | CSI-RS 1.2 FDD |  |
|  | 2 |  | CSI-RS 1.2 TDD |  |
|  | 3 |  | CSI-RS 2.2 TDD |  |
| reportConfigType | 1~4 |  | periodic |  |
| reportQuantity | 1~4 |  | cri-RSRP |  |
| Number of reported RS | 1~4 |  | 2 |  |
| L1-RSRP reporting period | 1~4 |  | slot80 |  |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | 1~4 | dBm/ 15kHz | -94.65 | -117+ ΔBG_offset |
| Note2 | 1,2,4 | dBm/ SCS | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -91.65 | -114+ ΔBG_offset |
|  | 1~4 | dB | 10 | -3 |
| CSI-RSRP Note3 | 1,2,4 | dBm/ SCS | -84.65 | -120+ ΔBG_offset |
|  | 3 |  | -81.65 | -117 + ΔBG_offset |
| Io Note3 | 1,2,4 | dBm/ 9.36 MHz | -56.28 | -87.28 + ΔBG_offset |
|  | 3 | dBm/ 18.36 MHz | -53.36 | -84.36 + ΔBG_offset |
|  | 1~4 | dB | 10 | -3 |
| Propagation condition | 1~4 |  | AWGN |  |
| Antenna configuration and Correlation matrix | 1~4 |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |

Table 16.7.4.4.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Highest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Highest reported value (Cell 1) | RSRP_72+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_36+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_36+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_34+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_33+TT |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 16.7.4.4.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Highest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Highest reported value (Cell 1) | RSRP_75+TT | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_39+TT |
|  |  | Bands NR_FDD_FR1_B | RSRP_39+TT |
|  |  | Bands NR_TDD_FR1_C | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_40+TT |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_G | RSRP_41+TT |
|  |  | Bands NR_FDD_FR1_H | RSRP_42+TT |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

##### 16.7.4.4.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.4.2.1	Test purpose

To verify that the CSI-RS based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.4.4.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.4.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.4.

16.7.4.4.2.4	Test description

16.7.4.4.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.2.4.1 with following exceptions:

-	Table 6.7.4.2.2.4.1-1 is replaced by Table 16.7.4.4.2.4.1-1;

-	Table 6.7.4.2.2.4.1-2 is replaced by Table 16.7.4.4.2.4.1-2;

Table 16.7.4.4.2.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.4.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.4.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.4.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.4.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.4.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.4.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.4.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.2.2.4.2.

16.7.4.4.2.4.3	Message contents

Same as the message contents given in clause 6.7.4.2.2.4.3.

16.7.4.4.2.5	Test requirement

Same as the thest requirements given in clause 6.7.4.2.2.5 with following exceptions:

-	Table 6.7.4.2.2.5-1 is replaced by Table 16.7.4.4.2.5-1;

-	Table 6.7.4.2.2.5-2 is replaced by Table 16.7.4.4.2.5-2;

Table 16.7.4.4.2.5-1: Same as Table 16.7.4.4.2_1.5-1

Table 16.7.4.4.2.5-2: L1-RSRP relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |
| Extreme Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |

### 16.7.5	E-UTRAN RSRP

#### 16.7.5.0	Minimum conformance requirements

##### 16.7.5.0.1	E-UTRAN RSRP absolute accuracy

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 6.7.5.0.1 shall apply.

When RedCap UE is only required to support 1Rx, the requirements for absolute accuracy of RSRP in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 16.7.5.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in TS 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to TS 36.133 [23] Annex B.3.3 for a corresponding Band.

Table 16.7.5.0.1-1: E-UTRAN RSRP absolute accuracy for 1Rx RedCap UE

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 5.5 | 10 | -6 | FDD_A, TDD_A | -121 | N/A | -70 |
|  |  |  | FDD_B1 | -120.5 | N/A | -70 |
|  |  |  | FDD_C, TDD_C | -120 | N/A | -70 |
|  |  |  | FDD_D | -119.5 | N/A | -70 |
|  |  |  | FDD_E, TDD_E | -119 | N/A | -70 |
|  |  |  | FDD_F | -118.5 | N/A | -70 |
|  |  |  | FDD_G | -118 | N/A | -70 |
|  |  |  | FDD_H | -117.5 | N/A | -70 |
|  |  |  | FDD_N | -114.5 | N/A | -70 |
| 9 | 12 | -6 | FDD_A, TDD_A, FDD_B1, FDD_C, TDD_C, FDD_D, FDD_E, TDD_E, FDD_F, FDD_G, FDD_H, FDD_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in TS 36.133 [23] Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in TS 36.133 [23] Section 3.5. |  |  |  |  |  |  |

The reporting range and mapping specified for RSRP measurements in clause 9.1.4 of TS 36.133 [23] shall apply.

The normative reference for this requirement is TS 38.133 [6] clause 10.2A.2 and TS 36.133 clause 9.1.3.3.

#### 16.7.5.1	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE

Editor’s Note: this test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.5.1.1	Test purpose

To verify that the E-UTRAN RSRP measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 10.2A.2 for SA inter-RAT E-UTRAN RSRP measurements.

16.7.5.1.2	Test applicability

This test applies to all types of NR RedCap UEs with 1Rx from Release 17 onwards and supporting E-UTRA.

16.7.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.5.1.

16.7.5.1.4	Test description

16.7.5.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.5.1.4.1 with following exceptions:

-	Table 6.7.5.1.4.1-1 is replaced by Table 16.7.5.1.4.1-1;

-	Table 6.7.5.1.4.1-2 is replaced by Table 16.7.5.1.4.1-2;

Table 16.7.5.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.7.5.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 16.7.5.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.5.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.5.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 16.7.5.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.5.1-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.5.1-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN FDD |
| 16.7.5.1-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.5.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.5.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.5.1.4.2.

16.7.5.1.4.3	Message contents

Same as the message contents given in 6.7.5.1.4.3.

16.7.5.1.5	Test requirement

Same as the test requirements given in clause 6.7.5.1.5 with following exceptions:

-	Table 6.7.5.1.5-1 is replaced by Table 16.7.5.1.5-1;

-	Table 6.7.5.1.5-2 is replaced by Table 16.7.5.1.5-2;

-	Table 6.7.5.1.5-3 is replaced by Table 16.7.5.1.5-3;

Table 16.7.5.1.5-1: NR Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1,4 |  | FDD |
|  | Config 2,3,5,6 |  | TDD |
|  | Config 7,8 |  | HD-FDD |
| TDD Configuration | Config 1,4,7,8 |  | N/A |
|  | Config 2,5 |  | TDDConf.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |
|  | Config 3,6 |  | 20: NRB,c = 51 |
| Gap pattern Id |  |  | 0 |
| PDSCH RMC | Config 1,4,7,8 |  | SR.1.1 FDD |
|  | Config 2,5 |  | SR.1.1 TDD |
|  | Config 3,6 |  | SR.2.1 TDD |
| RMSI CORSET RMC | Config 1,4,7,8 |  | CR.1.1 FDD |
|  | Config 2,5 |  | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD |
| Dedicated CORSET RMC | Config 1, 4,7,8 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1,4,7,8 |  | TRS.1.1 FDD |
|  | Config 2,5 |  | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD |
| Initial DL BWP |  |  | DLBWP.0.1 |
| Dedicated DL BWP |  |  | DLBWP.1.1 |
| Initial UL BWP |  |  | ULBWP.0.1 |
| Dedicated UL BWP |  |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.1 RedCap FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1,2,4,5,7,8 | dBm/SCS | -104 |
|  | Config 3,6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| SSB_RPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/18.36 MHz | -56.03 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration |  |  | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 16.7.5.1.5-2: E-UTRAN Cell specific test parameters (Cell 2)

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 2 |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |
|  | Config 4,5,6,8 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1,2,3,7 |  | N/A |  |
|  | Config 4,5,6,8 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1,2,3,7 |  | N/A |  |
|  | Config 4,5,6,8 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 |  | dBm/15kHz | -91.65 | -117 + ΔBG_offset |
| Ês/Noc |  | dB | 10 | -4 |
| Ês/IotNote5 |  | dB | 10 | -4 |
| RSRPNote5 |  | dBm/15kHz | -81.65 | -121 + ΔBG_offset |
| SCH_RPNote5 |  | dBm/15kHz | -81.65 | -121 + ΔBG_offset |
| IoNote5 |  | dBm/Ch BW | -53.45 + 10log(NRB,c/50) | -87.76 + 10log(NRB,c/50) + ΔBG_offset |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

Table 16.7.5.1.5-3: E-UTRA RSRP absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |
|  |  | FDD_B | 19+TT |
|  |  | TDD_C | 19+TT |
|  |  | FDD_D, TDD_D | 19+TT |
|  |  | FDD_E, TDD_E | 19+TT |
|  |  | FDD_G | 19+TT |
|  |  | FDD_H | 19+TT |
| Highest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |
|  |  | FDD_B | 19+TT |
|  |  | TDD_C | 19+TT |
|  |  | FDD_D, TDD_D | 19+TT |
|  |  | FDD_E, TDD_E | 19+TT |
|  |  | FDD_G | 19+TT |
|  |  | FDD_H | 19+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |
|  |  | FDD_B | 19+TT |
|  |  | TDD_C | 19+TT |
|  |  | FDD_D, TDD_D | 19+TT |
|  |  | FDD_E, TDD_E | 19+TT |
|  |  | FDD_G | 19+TT |
|  |  | FDD_H | 19+TT |
| Highest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |
|  |  | FDD_B | 19+TT |
|  |  | TDD_C | 19+TT |
|  |  | FDD_D, TDD_D | 19+TT |
|  |  | FDD_E, TDD_E | 19+TT |
|  |  | FDD_G | 19+TT |
|  |  | FDD_H | 19+TT |
| Note 1: E-UTRA operating band groups are as defined in TS 36.521-3 [26] clause 3.5.1 |  |  |  |

#### 16.7.5.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE

Editor’s Note: this test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.5.2.1	Test purpose

To verify that the E-UTRAN RSRP measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 10.2A.2 for SA inter-RAT E-UTRAN RSRP measurements.

16.7.5.2.2	Test applicability

This test applies to all types of NR RedCap UEs with 2Rx from Release 17 onwards and supporting E-UTRA.

16.7.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.5.2.

16.7.5.2.4	Test description

16.7.5.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.5.1.4.1 with following exceptions:

-	Table 6.7.5.1.4.1-1 is replaced by Table 16.7.5.2.4.1-1;

-	Table 6.7.5.1.4.1-2 is replaced by Table 16.7.5.2.4.1-2;

Table 16.7.5.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.7.5.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 16.7.5.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.5.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.5.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 16.7.5.2-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.5.2-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.5.2-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN FDD |
| 16.7.5.2-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.5.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.5.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.5.1.4.2.

16.7.5.2.4.3	Message contents

Same as the message contents given in 6.7.5.1.4.3.

16.7.5.2.5	Test requirement

Same as the test requirements given in clause 6.7.5.1.5 with following exceptions:

-	Table 6.7.5.1.5-1 is replaced by Table 16.7.5.2.5-1;

-	Table 6.7.5.1.5-2 is replaced by Table 16.7.5.2.5-2;

-	Table 6.7.5.1.5-3 is replaced by Table 16.7.5.2.5-3;

Table 16.7.5.2.5-1: NR Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1,4 |  | FDD |
|  | Config 2,3,5,6 |  | TDD |
|  | Config 7,8 |  | HD-FDD |
| TDD Configuration | Config 1,4,7,8 |  | N/A |
|  | Config 2,5 |  | TDDConf.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |
|  | Config 3,6 |  | 20: NRB,c = 51 |
| Gap pattern Id |  |  | 0 |
| PDSCH RMC | Config 1,4,7,8 |  | SR.1.1 FDD |
|  | Config 2,5 |  | SR.1.1 TDD |
|  | Config 3,6 |  | SR.2.1 TDD |
| RMSI CORSET RMC | Config 1,4,7,8 |  | CR.1.1 FDD |
|  | Config 2,5 |  | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD |
| Dedicated CORSET RMC | Config 1,4,7,8 |  | CCR.1.1 FDD |
|  | Config 2,5 |  | CCR.1.1 TDD |
|  | Config 3,6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1,4,7,8 |  | TRS.1.1 FDD |
|  | Config 2,5 |  | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD |
| Initial DL BWP |  |  | DLBWP.0.1 |
| Dedicated DL BWP |  |  | DLBWP.1.1 |
| Initial UL BWP |  |  | ULBWP.0.1 |
| Dedicated UL BWP |  |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.1 RedCap FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1,2,4,5,7,8 | dBm/SCS | -104 |
|  | Config 3,6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| SSB_RPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/18.36 MHz | -56.03 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 Low |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 16.7.5.2.5-2: E-UTRAN Cell specific test parameters (Cell 2)

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 2 |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |
|  | Config 4,5,6,8 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1,2,3,7 |  | N/A |  |
|  | Config 4,5,6,8 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1,2,3,7 |  | N/A |  |
|  | Config 4,5,6,8 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 |  | dBm/15kHz | -91.65 | -117 + ΔBG_offset |
| Ês/Noc |  | dB | 10 | -4 |
| Ês/IotNote5 |  | dB | 10 | -4 |
| RSRPNote5 |  | dBm/15kHz | -81.65 | -121 + ΔBG_offset |
| SCH_RPNote5 |  | dBm/15kHz | -81.65 | -121 + ΔBG_offset |
| IoNote5 |  | dBm/Ch BW | -53.45 + 10log(NRB,c/50) | -87.76 + 10log(NRB,c/50) + ΔBG_offset |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

Table 16.7.5.2.5-3: E-UTRA RSRP absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |
|  |  | FDD_B | 19+TT |
|  |  | TDD_C | 19+TT |
|  |  | FDD_D, TDD_D | 19+TT |
|  |  | FDD_E, TDD_E | 19+TT |
|  |  | FDD_G | 19+TT |
|  |  | FDD_H | 19+TT |
| Highest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |
|  |  | FDD_B | 19+TT |
|  |  | TDD_C | 19+TT |
|  |  | FDD_D, TDD_D | 19+TT |
|  |  | FDD_E, TDD_E | 19+TT |
|  |  | FDD_G | 19+TT |
|  |  | FDD_H | 19+TT |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |
|  |  | FDD_B | 19+TT |
|  |  | TDD_C | 19+TT |
|  |  | FDD_D, TDD_D | 19+TT |
|  |  | FDD_E, TDD_E | 19+TT |
|  |  | FDD_G | 19+TT |
|  |  | FDD_H | 19+TT |
| Highest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |
|  |  | FDD_B | 19+TT |
|  |  | TDD_C | 19+TT |
|  |  | FDD_D, TDD_D | 19+TT |
|  |  | FDD_E, TDD_E | 19+TT |
|  |  | FDD_G | 19+TT |
|  |  | FDD_H | 19+TT |
| Note 1: E-UTRA operating band groups are as defined in TS 36.521-3 [26] clause 3.5.1 |  |  |  |

### 16.7.6	E-UTRAN RSRQ

#### 16.7.6.0	Minimum conformance requirements

##### 16.7.6.0.1	E-UTRAN RSRQ absolute accuracy

When RedCap UE is capable of 2Rx, the minimum requirements given in clause 6.7.6.0.1 shall apply.

When RedCap UE is only required to support 1Rx, the requirements for absolute accuracy of RSRQ in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 16.7.6.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in TS 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to TS 36.133 [23] Annex B.3.1 for a corresponding Band.

Table 16.7.6.0.1-1: E-UTRAN RSRQ absolute accuracy for 1Rx RedCap UE

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 3.5 | 5 | -3 dB | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 4.5 | 5 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in TS 36.133 [23] Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in TS 36.133 [23] Section 3.5. |  |  |  |  |  |

The reporting range and mapping specified for RSRP measurements in clause 9.1.4 of TS 36.133 [23] shall apply.

The normative reference for this requirement is TS 38.133 [6] clauses 10.2A.3 and TS 36.133 [23] clause 9.1.5.5.

#### 16.7.6.1	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE

Editor’s Note: this test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.6.1.1	Test purpose

To verify that the E-UTRAN RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 10.2A.2 for SA inter-RAT E-UTRAN RSRP measurements.

16.7.6.1.2	Test applicability

This test applies to all types of NR RedCap UEs with 1Rx from Release 17 onwards and supporting E-UTRA.

16.7.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.6.1.

16.7.6.1.4	Test description

16.7.6.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.6.1.4.1 with following exceptions:

-	Table 6.7.6.1.4.1-1 is replaced by Table 16.7.6.1.4.1-1;

-	Table 6.7.6.1.4.1-2 is replaced by Table 16.7.6.1.4.1-2;

Table 16.7.6.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.7.6.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 16.7.6.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.6.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.6.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 16.7.6.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.6.1-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.6.1-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN FDD |
| 16.7.6.1-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.6.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.6.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.6.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.6.1.4.2.

16.7.6.1.4.3	Message contents

Same as the message contents given in 6.7.6.1.4.3.

16.7.6.1.5	Test requirement

Same as the test requirements given in clause 6.7.6.1.5 with following exceptions:

-	Table 6.7.6.1.5-1 is replaced by Table 16.7.6.1.5-1;

-	Table 6.7.6.1.5-2 is replaced by Table 16.7.6.1.5-2;

-	Table 6.7.6.1.5-3 is replaced by Table 16.7.6.1.5-3;

Table 16.7.6.1.5-1: NR Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1,4 |  | FDD |
|  | Config 2,3,5,6 |  | TDD |
|  | Config 7,8 |  | HD-FDD |
| TDD Configuration | Config 1,4,7,8 |  | N/A |
|  | Config 2,5 |  | TDDConf.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |
|  | Config 3,6 |  | 20: NRB,c = 51 |
| Gap pattern Id |  |  | 0 |
| PDSCH RMC | Config 1,4,7,8 |  | SR.1.1 FDD |
|  | Config 2,5 |  | SR.1.1 TDD |
|  | Config 3,6 |  | SR.2.1 TDD |
| RMSI CORSET RMC | Config 1,4,7,8 |  | CR.1.1 FDD |
|  | Config 2,5 |  | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD |
| Dedicated CORSET RMC | Config 1, 4,7,8 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1,4,7,8 |  | TRS.1.1 FDD |
|  | Config 2,5 |  | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD |
| Initial DL BWP |  |  | DLBWP.0.1 |
| Dedicated DL BWP |  |  | DLBWP.1.1 |
| Initial UL BWP |  |  | ULBWP.0.1 |
| Dedicated UL BWP |  |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.1 RedCap FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1,2,4,5,7,8 | dBm/SCS | -104 |
|  | Config 3,6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| SSB_RPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/18.36 MHz | -56.03 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration |  |  | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 16.7.6.1.5-2: E-UTRAN Cell specific test parameters (Cell 2)

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 2 |  |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |  |
|  | Config 4,5,6,8 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1,2,3,7 |  | N/A |  |  |
|  | Config 4,5,6,8 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1,2,3,7 |  | N/A |  |  |
|  | Config 4,5,6,8 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 |  | dBm/15kHz | -83 | -104.70 | -119.5 + ΔBG_offset |
| Ês/Noc |  | dB | -1.75 | -4 | -4 |
| Ês/IotNote5 |  | dB | -1.75 | -4 | -4 |
| RSRPNote5 |  | dBm/15kHz | -84.75 | -108.70 | -123.5 + ΔBG_offset |
| RSRQNote5 |  | dB | -14.76 | -16.25 | -16.25 |
| IoNote5 |  | dBm/Ch BW | -53 + 10log(NRB,c /50) | -75.46 + 10log(NRB,c /50) | -90.26 + 10log(NRB,c /50) + ΔBG_offset |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration |  |  | 1x1 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table 16.7.6.1.5-3: E-UTRA RSRQ absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 10+TT | 6+TT | 6+TT |
| Highest reported value (Cell 2) | 10+TT | 6+TT | 6+TT |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest reported value (Cell 2) | 10+TT | 6+TT | 6+TT |
| Highest reported value (Cell 2) | 10+TT | 6+TT | 6+TT |

#### 16.7.6.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE

Editor’s Note: this test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.6.2.1	Test purpose

To verify that the E-UTRAN RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 10.2A.2 for SA inter-RAT E-UTRAN RSRP measurements.

16.7.6.2.2	Test applicability

This test applies to all types of NR RedCap UEs with 2Rx from Release 17 onwards and supporting E-UTRA.

16.7.6.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.6.2.

16.7.6.2.4	Test description

16.7.6.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.6.1.4.1 with following exceptions:

-	Table 6.7.6.1.4.1-1 is replaced by Table 16.7.6.2.4.1-1;

-	Table 6.7.6.1.4.1-2 is replaced by Table 16.7.6.2.4.1-2;

Table 16.7.6.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.7.6.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 16.7.6.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.6.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.6.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 16.7.6.2-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.6.2-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.6.2-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN FDD |
| 16.7.6.2-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.6.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.6.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.6.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.6.1.4.2.

16.7.6.2.4.3	Message contents

Same as the message contents given in 6.7.6.1.4.3.

16.7.6.2.5	Test requirement

Same as the test requirements given in clause 6.7.6.1.5 with following exceptions:

-	Table 6.7.6.1.5-1 is replaced by Table 16.7.6.2.5-1;

-	Table 6.7.6.1.5-2 is replaced by Table 16.7.6.2.5-2;

-	Table 6.7.6.1.5-3 is replaced by Table 16.7.6.2.5-3;

Table 16.7.6.2.5-1: NR Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1,4 |  | FDD |
|  | Config 2,3,5,6 |  | TDD |
|  | Config 7,8 |  | HD-FDD |
| TDD Configuration | Config 1,4,7,8 |  | N/A |
|  | Config 2,5 |  | TDDConf.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |
|  | Config 3,6 |  | 20: NRB,c = 51 |
| Gap pattern Id |  |  | 0 |
| PDSCH RMC | Config 1,4,7,8 |  | SR.1.1 FDD |
|  | Config 2,5 |  | SR.1.1 TDD |
|  | Config 3,6 |  | SR.2.1 TDD |
| RMSI CORSET RMC | Config 1,4,7,8 |  | CR.1.1 FDD |
|  | Config 2,5 |  | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD |
| Dedicated CORSET RMC | Config 1, 4,7,8 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1,4,7,8 |  | TRS.1.1 FDD |
|  | Config 2,5 |  | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD |
| Initial DL BWP |  |  | DLBWP.0.1 |
| Dedicated DL BWP |  |  | DLBWP.1.1 |
| Initial UL BWP |  |  | ULBWP.0.1 |
| Dedicated UL BWP |  |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.1 RedCap FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1,2,4,5,7,8 | dBm/SCS | -104 |
|  | Config 3,6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| SSB_RPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/18.36 MHz | -56.03 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 Low |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 16.7.6.2.5-2: E-UTRAN Cell specific test parameters (Cell 2)

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 2 |  |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |  |
|  | Config 4,5,6,8 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1,2,3,7 |  | N/A |  |  |
|  | Config 4,5,6,8 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1,2,3,7 |  | N/A |  |  |
|  | Config 4,5,6,8 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 |  | dBm/15kHz | -83 | -104.70 | -119.5 + ΔBG_offset |
| Ês/Noc |  | dB | -1.75 | -4 | -4 |
| Ês/IotNote5 |  | dB | -1.75 | -4 | -4 |
| RSRPNote5 |  | dBm/15kHz | -84.75 | -108.70 | -123.5 + ΔBG_offset |
| RSRQNote5 |  | dB | -14.76 | -16.25 | -16.25 |
| IoNote5 |  | dBm/Ch BW | -53 + 10log(NRB,c /50) | -75.46 + 10log(NRB,c /50) | -90.26 + 10log(NRB,c /50) + ΔBG_offset |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table 16.7.6.2.5-3: E-UTRA RSRQ absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 10+TT | 6+TT | 6+TT |
| Highest reported value (Cell 2) | 10+TT | 6+TT | 6+TT |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest reported value (Cell 2) | 10+TT | 6+TT | 6+TT |
| Highest reported value (Cell 2) | 10+TT | 6+TT | 6+TT |

# 17	NR standalone tests with at least one NR cell in FR2 for RedCap

## 17.1	RRC_IDLE state mobility

### 17.1.1	NR cell re-selection

#### 17.1.1.0	Minimum conformance requirements

##### 17.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS 38.304 [30] within Tdetect,NR_Intra_RedCap when that Treselection= 0 . An intra frequency cell is considered to be detectable according to the conditions defined in TS 38.133 [6] Annex B.1.4 for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra_RedCap for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra_RedCap/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS 38.304 [30] within Tevaluate,NR_Intra_RedCap when Treselection = 0 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 4.5dB better ranked in FR2 for 2 Rx RedCap.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS 38.304 [30] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 4.5dB better ranked in FR2 if the current serving cell is among them for 2 Rx RedCap.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non-zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS 38.304 [30], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For 2Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap are specified in Table 17.1.1.0.1-1.

For 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap are specified in Table 17.1.1.0.1-2 for FR2, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap when multiple PTWs are used.

Table 17.1.1.0.1-1: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra_RedCap [s] (number of DRX cycles) | Tmeasure,NR_Intra_RedCap [s] (number of DRX cycles) | Tevaluate,NR_Intra_RedCap [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for RedCap UE of all FR2 power class. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra_RedCap is expected. |  |  |  |  |  |

Table 17.1.1.0.1-2: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 4) | Tmeasure,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 4) | Tevaluate,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| NOTE 1: Applies for RedCap UE of all power class. NOTE 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 4: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 5: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. NOTE 6: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

##### 17.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in TS 38.133 [6] in clause 4.2.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS 38.304 [30] within Kcarrier_RedCap * Tdetect,NR_Inter_RedCap  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met

For 2 Rx RedCap by a margin of at least

5 dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or

7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or

4 dB in FR2 for SS-RSRQ reselections based on absolute priorities

The parameter Kcarrier_RedCap is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in TS 38.133 [6] in Annex B.1.5 for a corresponding Band. When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter_RedCap. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier_RedCap * Tmeasure,NR_Inter_RedCap for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [30] within Kcarrier * Tevaluate,NR_Inter_RedCap when Treselection = 0 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

-	when rangeToBestCell is not configured:

-	the cell is at least 6.5dB better ranked in FR2 for 2 Rx RedCap.

-	when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value defined in TS38.304 [30] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 6.5dB better ranked in FR2 if the current serving cell is among them, or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities for 2 Rx RedCap or 4dB in FR2 for SS-RSRQ reselections based on absolute priorities for 2 Rx RedCap.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non-zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in TS 38.133 [6] in Table 17.1.1.0.2-1 for 2Rx RedCap under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion defined in TS 38.304 [30].

For Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,NR_Inter_RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in Table 17.1.1.0.2.1-1.

For 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in Table 17.1.1.0.2-2 for FR2. The requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap when multiple PTWs are used.

Table 17.1.1.0.2-1: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for RedCap UE of all FR2 power class. |  |  |  |  |  |

Table 17.1.1.0.2-2: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |  | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 3 | 58.88 x N1 (23 x N1) |  | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) |  | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - | 3 | 235.52 x N1 (23 x N1) |  | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) |  | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| NOTE 1: Applies for RedCap UE of all power class. NOTE 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 4: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 5: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. NOTE 6: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

##### 17.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion

The requirements defined in clause 17.1.1.0.3 apply for this clause except that:

-	Tdetect,NR_Intra_RedCap_Relax as specified in Table 17.1.1.0.3-1 for 2 Rx RedCap.

-	Tmeasure,NR_Intra_RedCap_Relax as specified in Table 17.1.1.0.3-1 for 2 Rx RedCap.

-	Tevaluate,NR_Intra_RedCap_Relax as specified in Table 17.1.1.0.3-1 for 2 Rx RedCap.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 17.1.1.0.3-2 are applicable for eDRX cycle up to 10.24 s in FR2.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR2, then the requirements in Table 17.1.1.0.3-3 apply provided eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

Table 17.1.1.0.3-1: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UEs fulfilling stationary criterion for 2 Rx RedCap UE

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K3 (36 x N1 x M2 x K3) | 1.28 x N1 x M2 x K3 (4 x N1 x M2 x K3) | 5.12 x N1 x M2 x K3 (16 x N1 x M2 x K3) |
| 0.64 |  | 5 | 17.92 x N1 x K3 (28 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) | 5.12 x N1 x K3 (8 x N1 x K3) |
| 1.28 |  | 4 | 32 x N1 x K3 (25 x N1 x K3) | 1.28 x N1 x K3 (1 x N1 x K3) | 6.4 x N1 x K3 (5 x N1 x K3) |
| 2.56 |  | 3 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x N1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 3: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 17.1.1.0.3-2: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle up to 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of eDRX IDLE cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x N1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| 5.12 | 117.76 x N1 x K3 (23 x N1 x K3) | 5.12 x N1 x K3 (1 x N1 x K3) | 10.24 x N1 x K3 (2 x N1 x K3) |
| 10.24 | 235.52 x N1 x K3 (23 x N1 x K3) | 10.24 x N1 x K3 (1 x N1 x K3) | 20.48 x N1 x K3 (2 x N1 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 17.1.1.0.3-3: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |  | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥30.72 (24) | 8 | K3 x eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1 x K3) |  | 0.32 x N1 x K3 (1 x N1 x K3) | 0.64 x N1 x K3 (2 x N1 x K3) |
|  | 0.64 | ≥38.4 (30) | 5 |  |  | 0.64 x N1 x K3 (1 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) |
|  | 1.28 | 40.96 (32) | 4 |  |  | 1.28 x N1 x K3 (1 x N1 x K3) | 2.56 x N1 x K3 (2 x N1 x K3) |
|  | 2.56 | 40.96 (32) | 3 |  |  | 20.48 | 40.96 |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap_Relax*DRX_cycle1.28*1.28. Note 5: The measurement shall not be performed across PTW’s. In this case the measurement is performed in the next available PTW. Note 6: The evaluation shall not be performed across PTW’s. In this case the evaluation is performed in the next available PTW. Note 7: For DRX cycle length is 0.32s, 0.64s, and 2.56s, K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. For DRX cycle length is 1.28s, K3 = 4 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |  |  |

##### 17.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion, and

The requirements defined in 17.1.1.0.4 apply for this clause except that:

-	Tdetect,NR_Inter_RedCap_Relax as specified in Table 17.1.1.0.4-1 for 2 Rx RedCap.

-	Tmeasure,NR_Inter_RedCap_Relax as specified in Table 17.1.1.0.4-1 for 2 Rx RedCap.

-	Tevaluate,NR_Inter_RedCap_Relax as specified in Table 17.1.1.0.4-1 and for 2 Rx RedCap.

-	When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in TS 38.133 [6] in clause 4.2.2.7 and, K2 = 240.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 17.1.1.0.4-2 are applicable for eDRX cycle up to 10.24s in FR2.

If the UE is configured with eDRX_IDLE cycle greater than 10.24s FR2, then the requirements in Table 17.1.1.0.4-3 apply provided that eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

Table 17.1.1.0.4-1: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for 2 Rx RedCap UE

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K4 (36 x N1 x 1.5 x K4) | 1.28 x N1 x 1.5 x K4 (4 x N1 x 1.5 x K4) | 5.12 x N1 x 1.5 x K4 (16 x N1 x 1.5 x K4) |
| 0.64 |  | 5 | 17.92x N1 x K4 (28 x N1 x K4) | 1.28 x N1 x K4 (2 x N1 x K4) | 5.12 x N1 x K4 (8 x N1 x K4) |
| 1.28 |  | 4 | 32 x N1 x K4 (25 x N1 x K4) | 1.28 x N1 x K4 (1 x N1 x K4) | 6.4 x N1 x K4 (5 x N1 x K4) |
| 2.56 |  | 3 | 58.88 x N1 x K4 (23 x N1 x K4) | 2.56 x N1 x K4 (1 x N1 x K4) | 7.68 x N1 x K4 (3 x N1 x K4) |
| Note 1: Applies for RedCap UE of all supporting power class. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 17.1.1.0.4-2: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle up to 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x N1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| 5.12 | 117.76 x N1 x K3 (23 x N1 x K3) | 5.12 x N1 x K3 (1 x N1 x K3) | 10.24 x N1 x K3 (2 x N1 x K3) |
| 10.24 | 235.52 x N1 x K3 (23 x N1 x K3) | 10.24 x N1 x K3 (1 x N1 x K3) | 20.48 x N1 x K3 (2 x N1 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 17.1.1.0.4-3: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle larger than 10.24 s

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |  | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥30.72 (24) | 8 | K3 x eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1 x K3) |  | 0.32 x N1 x K3 (1 x N1 x K3) | 0.64 x N1 x K3 (2 x N1 x K3) |
|  | 0.64 | ≥38.4 (30) | 5 |  |  | 0.64 x N1 x K3 (1 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) |
|  | 1.28 | 40.96 (32) | 4 |  |  | 1.28 x N1 x K3 (1 x N1 x K3) | 2.56 x N1 x K3 (2 x N1 x K3) |
|  | 2.56 | 40.96 (32) | 3 |  |  | 20.48 | 40.96 |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 5: For DRX cycle length is 0.32s, 0.64s and 2.56s, K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. For DRX cycle length is 1.28s, K3 = 4 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |  |  |

#### 17.1.1.1	NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx

Editor’s Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz.

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

17.1.1.1.1	Test purpose

The purpose of this test is to verify the requirement for the NR SA intra-frequency cell reselection in FR2 for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.3..

17.1.1.1.2	Test applicability

This test applies to all types of NR RedCap UE release 17 and forward.

17.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 17.1.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.1.1.1.

17.1.1.1.4	Test description

17.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 17.1.1.1.4.1-1.

Table 17.1.1.1.4.1-1: Supported test configurations for NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx

| Configuration | Description |
| --- | --- |
| 17.1.1.1-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 17.1.1.1-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 17.1.1.1.4.1-2.

Table 17.1.1.1.4.1-2: Initial conditions for NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 17.1.1.1.4.1-3.

2.	Message contents are defined in clause 17.1.1.1.4.3.

3.	The test scenario comprises of 1 NR carrier and 2 cells. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 17.1.1.1.4.1-3: General test parameters for NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cell |  | 1, 2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, the intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 135 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2 | 35 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

17.1.1.1.4.2	Test procedure

Two cells are deployed in the test, which are one FR2 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1. Set Cell 2 physical cell identity = initial Cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 17.1.1.1.5-1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 17.1.1.1.5-1.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 130 seconds from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a failure for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 8.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 14.

8.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

9.	The SS shall switch the power setting from T2 to T3 as specified in Table 17.1.1.1.5-1.

10.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

11.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 27 seconds from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

12.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, continues with step 12.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 14.

13.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

14.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

15.	Repeat step 2-14 until a test verdict has been achieved.
Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

17.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 17.1.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.1.4.3-1A: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -69 | Actual value is -60*2 = -138dBm | 7.1.1.1-1 |
|  | -68 | Actual value is -68*2+1 = -135dBm | 7.1.1.1-2 |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3-1B: SIB2 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-1 with condition SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -60 | Actual value is -60*2 = -120dBm | 7.1.1.1-1 |
|  | -58 | Actual value is -58*2 = -116dBm | 7.1.1.1-2 |
| s-IntraSearchP | 25 | Actual value is 25*2 = 50dB |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

17.1.1.1.5	Test requirement

Table 17.1.1.1.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for 2 Rx UE.

Table 17.1.1.1.5-1: Cell specific test parameters for NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
|  |  | 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
|  |  | 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
|  |  | 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  |  | OP.4 |  |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  | 1, 2 | 66 |  |  | 66 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -120Note 6 |  |  | -120 Note 6 |  |  |
|  |  | 2 | -117 Note 6 |  |  | -117 Note 6 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.9.1 |  |  | Setup 1 defined in A.9.1 |  |  |
| Es/Iot BB Note 5 | dB | 1 | 7.56 | -3.34 | 1.61 | -infinity | 1.61 | -3.34 |
|  |  | 2 |  |  |  |  |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | -2.9 | 2.05 | -infinity | 2.05 | -2.9 |
|  |  | 2 |  |  |  |  |  |  |
| SSB_RP Note3 | dBm/SCS | 1 | -85 | -95.9 | -90.9 | -infinity | -90.9 | -95.9 |
|  |  | 2 | -82 | -92.9 | -87.9 | -infinity | -87.9 | -92.9 |
| Io on SSB symbols | dBm/95.04 MHz | 1 | -60.53 | -67.37 | -65.01 | -69.17 | -65.01 | -67.37 |
| of each cell |  | 2 | -57.52 | -64.39 | -62.33 | -66.16 | -62.33 | -64.39 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 130 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than 27 s.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 17.1.1.2	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx

Editor’s Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz.

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

17.1.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the NR SA intra-frequency cell reselection in FR2 for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.4.

17.1.1.2.2	Test applicability

This test applies to all types of NR RedCap UE release 17 and forward.

17.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 17.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.1.1.2.

17.1.1.2.4	Test description

17.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 17.1.1.2.4.1-1.

Table 17.1.1.2.4.1-1: Supported test configurations for NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 17.1.1.2-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 17.1.1.2-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 17.1.1.2.4.1-2.

Table 17.1.1.2.4.1-2: Initial conditions for NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 17.1.1.2.4.1-3.

2.	Message contents are defined in clause 17.1.1.2.4.3.

3.	The test scenario comprises of 2 cells on 2 different NR carriers. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 17.1.1.2.4.1-3: General test parameters for NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 35 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 95 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

17.1.1.2.4.2	Test procedure

Two cells on two RF channels are deployed in the test, which are one FR2 NR PCell (Cell 1) on RF channel 1 and an NR neighbour cell (Cell 2) on RF channel 2. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1. Set Cell 2 physical cell identity = initial Cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 17.1.1.2.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, Cell 1.

4.	If the UE responds on lower priority cell, Cell 1 during time duration T1 within 27 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”.

5.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6a.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 13.

6.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 17.1.1.2.5-1. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE.

8.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 17.1.1.2.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, Cell 2.

10.	If the UE responds on higher priority cell, Cell 2 during time duration T3 within 87 seconds from the beginning of time period T3, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”.

11.	If the UE has re-selected Cell 2 within T3, after the re-selection or when T3 expires, continues with step 12.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and continue with step 13.

12.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

13.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

14.	Repeat step 2-13 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

17.1.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 17.1.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 for NR Cell 1 |
| Default RRC messages and information elements contents exceptions |  |

Table 17.1.1.2.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 17.1.1.2.4.3-3: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | Actual value is -62*2 = -140dBm | 17.1.1.2-1 |
|  | -68 | Actual value is -62*2 = -136dBm | 17.1.1.2-2 |
| } |  |  |  |
| } |  |  |  |

Table 17.1.1.2.4.3-4: SIB2(Cell2)

| Derivation Path: Table H.2.2-1 with condition NR Cell 2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | Actual value is 25*2 = 50dBm |  |
| threshServingLowP | 22 | Actual value is 22*2 = 44dBm |  |
| } |  |  |  |
| } |  |  |  |

Table 17.1.1.2.4.3-5: SIB4 (Cell 1)

| Derivation Path: Table H.2.2-2 with condition NR Cell 1, SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 Entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | Entry 1 |  |
| q-RxLevMin | -70 | Actual value = -70*2 = -140dBm | 17.1.1.2-1 |
|  | -68 | Actual value = -68*2 = -136dBm | 17.1.1.2-2 |
| threshX-HighP | 24 | Actual value is 24*2 = 48dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.1.1.2.4.3-6: SIB4 (Cell 2)

| Derivation Path: Table H.2.2-2 with condition NR Cell 2, SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 Entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | Entry 1 |  |
| q-RxLevMin | -70 | Actual value = -70*2 = -140dBm | 17.1.1.2-1 |
|  | -68 | Actual value = -68*2 = -136dBm | 17.1.1.2-2 |
| threshX-LowP | 25 | Actual value =25*2 = 50 dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

17.1.1.2.5	Test requirement

Table 17.1.1.2.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for 2 Rx UE.

Table 17.1.1.2.5-1: Cell specific test parameters for NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -124 Note 6 |  |  |
|  |  | 2 | -137 |  |  | -121 Note 6 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.9.1 |  |  | Setup 1 defined in A.9.1 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  |  | Rough |  |  |
| Es/Iot BB Note 5 | dB | 1 | 10.06 | 10.06 | 7.56 | -3.34 | -infinity | 8.06 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  | -93 |  |  |
|  |  | 2 | -90 |  |  | -90 |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  | -102 |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 10.5 | 10.5 | 8 | 2.9 | -infinity | 8.5 |
|  |  | 2 |  |  |  |  |  |  |
| SSB_RP Note3 | dBm/SCS | 1 | -82.5 | -82.5 | -85 | 95.9 | -infinity | 84.5 |
|  |  | 2 | -79.5 | -79.5 | -82 | -92.9 | -infinity | -91.5 |
| Io | dBm/95.04 MHz | 1, 2 | -53.11 | -53.11 | -55.34 | -62.18 | -63.98 | -54.91 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2 | 50 |  |  | 62 Note 6 |  |  |
| Threshx, highP | dB | 1, 2 | 32 Note 6 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2 | 44 |  |  | 62 Note 6 |  |  |
| Threshx, lowP | dB | 1, 2 | 50 |  |  | 18 Note 6 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |  |  |

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 87 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to a lower priority cell shall be less than 27 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 17.1.1.3	NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

Editor’s Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz.

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

17.1.1.3.1	Test purpose

The purpose of this test is to verify the requirement for the NR SA intra-frequency cell reselection in FR2 for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.9.2..

17.1.1.3.2	Test applicability

This test applies to all types of NR RedCap UE release 17 and forward supporting Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE.

17.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 17.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.17.1.1.3.

17.1.1.3.4	Test description

17.1.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 17.1.1.3.4.1-1.

Table 17.1.1.3.4.1-1: Supported test configurations for NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 17.1.1.3-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 17.1.1.3-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 17.1.1.3.4.1-2.

Table 17.1.1.3.4.1-2: Initial conditions for NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 17.1.1.3.4.1-3.

2.	Message contents are defined in clause 17.1.1.3.4.3.

3.	The test scenario comprises of 1 NR carrier and 2 cells. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. During T1 and T2, only criteria stationaryMobilityEvaluation is configured and fulfilled. UE has not registered with network for the tracking area containing cell2.

Table 17.1.1.3.4.1-3: General test parameters for NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 | The UE reselects to cell 1 during T2 period |
|  | Neighbour cells |  | 1,2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 100 |  |
| T2 |  | s | 1, 2 | 100 |  |

17.1.1.3.4.2	Test procedure

Two cells are deployed in the test, which are one FR2 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 2 successive time periods, with time duration of T1, T2 respectively. Both cell 1 and cell2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas.

The UE is configured only low mobility criterion defined in clause 5.2.4.9.1 in TS 38.304 [30]. So, Cell 1 configures the UE as follows:

-	stationaryMobilityEvaluation-r17 [2] is configured according to the parameters listed in Table 17.1.1.3.5-1;

-	cellEdgeEvaluationWhileStationary-r17 [2] is not configured;

-	combineRelaxedMeasCondition2-r17 [2] is not configured

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 17.1.1.3.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 2.

4.	If the UE responds on Cell 2 during time duration T1 within 155 seconds from the beginning of time period T1, then count a success for cell reselection. Otherwise count a fail for cell reselection.

5.	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 11.

6	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 17.1.1.3.5-1. T2 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

9.	If the UE responds to Cell 1 during time duration T2 within 155 seconds from the beginning of time period T2, then count a success for cell reselection. Otherwise count a fail for cell reselection.

10. If T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and continue with step 11.

11.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

12.	Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

17.1.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 17.1.1.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions |  |

Table 17.1.1.3.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 17.1.1.3.4.3-2A: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | Actual value is -70*2 = -140dBm | 17.1.1.3-1 |
|  | -68 | Actual value is -68*2 = -136dBm | 17.1.1.3-2 |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.4.3-3: SIB2

| Derivation Path: Table H.2.1-1 with condition SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | Actual value is -70*2 = -140dBm | 17.1.1.3-1 |
|  | -68 | Actual value is -68*2 = -136dBm | 17.1.1.3-2 |
| s-IntraSearchP | 25 | Actual value is 25*2 = 50dB |  |
| } |  |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB6 |  |  |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

17.1.1.3.5	Test requirement

Table 17.1.1.3.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE.

Table 17.1.1.3.5-1: Cell specific test parameters for NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| PDSCH RMC |  | 1 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| configuration |  | 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  | OP.4 |  |  |
| Initial DL BWP configuration |  | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| Initial UL BWP configuration |  | 1, 2 | 66 |  | 66 |  |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -120 Note 6 |  | -120 Note 6 |  |  |
|  |  | 2 | -117 Note 6 |  | -117 Note 6 |  |  |
| SSearchDeltaP-Stationary | dB | 1, 2 | 6 |  | 6 |  |  |
| TSearchDeltaP-Stationary | s | 1,2 | 5 |  | 5 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.9.1 |  | Setup 1 defined in A.9.1 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |  |
| Es/Iot BB Note 5 | dB | 1 | -3.34 | 1.61 | 1.61 | -3.64 |  |
|  |  | 2 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  | dB | 1 | -2.9 | 2.05 | 2.05 Note 6 | -2.9 |  |
|  |  | 2 |  |  |  |  |  |
| SSB_RP Note3 | dBm/SCS | 1 | -95.9 | -90.9 | -90.9 | -95.9 |  |
|  |  | 2 | -92.9 | -87.9 | -87.9 | -92.9 |  |
| Io on SSB symbols of each cell | dBm/95.04 MHz | 1 | -67.37 | -65.01 | -65.01 | -67.37 |  |
|  |  | 2 | -64.36 | -62.00 | -62.00 | -64.36 |  |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |  |
| SintrasearchP | dB | 1, 2 | 50 |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |  |

The cell reselection delay to an already detected cell for UE fulfilling stationary relaxed criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected cell shall be less than 155 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detectable cell can be expressed as: Tevaluate,NR_Intra_RedCap_Relax + TSI-NR,

Where:

Tevaluate,NR_Intra_RedCap_Relax	See Table 4.2B.2.9.2-2 in clause 4.2B.2.9.2,

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 154.88 s, allow 155 s for the cell re-selection delay to an already detected cell for UE fulfilling stationary criterion in the test case.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 17.1.1.4	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE

Editor’s Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz.

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

17.1.1.4.1	Test purpose

The purpose of this test is to verify the requirement for the NR SA intra-frequency cell reselection in FR2 for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.10.2.

17.1.1.4.2	Test applicability

This test applies to all types of NR RedCap UE release 17 and forward supporting Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE.

17.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 17.1.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.17.1.1.4.

17.1.1.4.4	Test description

17.1.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 17.1.1.4.4.1-1.

Table 17.1.1.4.4.1-1: Supported test configurations for NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 17.1.1.4-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 17.1.1.4-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 17.1.1.4.4.1-2.

Table 17.1.1.4.4.1-2: Initial conditions for NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 17.1.1.4.4.1-3.

2.	Message contents are defined in clause 17.1.1.4.4.3.

3.	The test scenario comprises of 2 cells (Cell 1 and Cell 2) on 2 different NR carriers respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2. Cell 2 is of higher priority than Cell 1. The UE is configured with stationaryMobilityEvaluation criterion.

Table 17.1.1.4.4.1-3: General test parameters for NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell2 and fulfils stationary (stationaryMobilityEvaluation [2]) criterion. |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 final condition | Active cell |  | 1, 2 | Cell1 | The UE reselects to low priority cell1 during T1 |
|  | Neighbour cell |  | 1, 2 | Cell2 |  |
| T2 final condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to high priority cell2 during T2 |
|  | Neighbour cell |  |  | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | TBD |  |
|  |  |  | 2 | TBD |  |
| SMTC configuration |  |  | 1, 2 | TBD |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 85 | T1 needs to be long enough to allow cell re-selection to already known cell1 |
| T2 |  | s | 1, 2 | 85 | T2 needs to be long enough to allow cell re-selection to already known cell2 |

17.1.1.4.4.2	Test procedure

The test consists of one active cell and one neighbour cell on two different NR RF channels. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of two successive time periods, with time duration of T1, T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

The UE is configured only low mobility criterion defined in clause 5.2.4.9.1 in TS 38.304 [30]. So, Cell 1 configures the UE as follows:

-	stationaryMobilityEvaluation-r17 [2] is configured according to the parameters listed in Table 17.1.1.4.5-1;

-	cellEdgeEvaluationWhileStationary-r17 [2] is not configured;

-	combineRelaxedMeasCondition2-r17 [2] is not configured

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 17.1.1.5.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 1.

4.	If the UE responds on lower priority Cell 1 during time duration T1 within 155 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”

5.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 7. Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 11.

6.	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 17.1.1.5.5-1. T2 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 2.

9.	If the UE responds to higher priority Cell 2 during time duration T2 within 155 seconds from the beginning of time period T2, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”

10.	If T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and continue with step 11.

11.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

12.	Repeat step 2-11 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

17.1.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 17.1.1.5.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |

Table 17.1.1.5.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 17.1.1.5.4.3-2A: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | Actual value is -62*2 = -140dBm | 17.1.1.5-1 |
|  | -68 | Actual value is -62*2 = -136dBm | 17.1.1.5-2 |
| } |  |  |  |
| } |  |  |  |

Table 17.1.1.5.4.3-3: SIB2 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | Actual value is 25*2 = 50 | Cell 2 |
| threshServingLowP | 22 | Actual value is 22*2 = 44 |  |
| cellReselectionPriority | 4 |  | Cell 1 |
|  | 5 |  | Cell 2 |
| } |  |  |  |
| relaxedMeasurement-r17 SEQUENCE { |  |  |  |
| stationaryMobilityEvaluation-r17 SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB6 |  |  |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluationWhileStationary-r17 | Not present |  |  |
| combineRelaxedMeasCondition2-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.1.1.5.4.3-4: SIB4 (Cell 1)

| Derivation Path: Table H.2.2-2 with condition NR Cell 1, SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 Entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | Entry 1 |  |
| q-RxLevMin | -70 | Actual value = -70*2 = -140dBm | 17.1.1.2-1 |
|  | -68 | Actual value = -68*2 = -136dBm | 17.1.1.2-2 |
| threshX-HighP | 24 | Actual value is 24*2 = 48dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.1.1.5.4.3-5: SIB4 (Cell 2)

| Derivation Path: Table H.2.2-2 with condition NR Cell 2, SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 Entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | Entry 1 |  |
| q-RxLevMin | -70 | Actual value = -70*2 = -140dBm | 17.1.1.2-1 |
|  | -68 | Actual value = -68*2 = -136dBm | 17.1.1.2-2 |
| threshX-LowP | 25 | Actual value = 25*2 = 50dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

17.1.1.4.5	Test requirement

Table 17.1.1.4.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE.

Table 17.1.1.4.5-1: Cell specific test parameters for NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -124 Note6 |  |
|  |  | 2 | -137 |  | -121 Note6 |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |
| Cell_selection_and_reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.9.1 |  | Setup 1 defined in A.9.1 |  |
| Beam assumptionNote 4 |  | 1, 2 | Rough |  | Rough |  |
| Es/Iot BB Note 5 | dB | 1, 2 | 10.06 | 7.56 | -3.34 | -8.06 |
| Note2 | dBm/SCS | 1 | -93 |  | -93 |  |
|  |  | 2 | -90 |  | -90 |  |
| Note2 | dBm/15 kHz | 1, 2 | -102 |  | -102 |  |
|  | dB | 1, 2 | 10.5 | 8 | -2.9 | 8.5 |
| SSB_RP Note3 | dBm/SCS | 1 | -82.5 | -85 | -95.9 | -84.5 |
|  |  | 2 | -79.5 | -82 | -92.9 | -81.5 |
| Io | dBm/95.04 MHz | 1 | -53.14 | -55.37 | -62.18 | -54.91 |
|  |  | 2 | -58.10 | -60.189 | -62.18 | -54.91 |
| TreselectionNR | s | 1, 2 | 0 |  | 0 |  |
| SnonintrasearchP | dB | 1, 2 | 50 |  | Not sent |  |
| SSearchDeltaP-Stationary | dB | 1, 2 | 12 Note6 |  | 12 Note6 |  |
| TSearchDeltaP-Stationary | s | 1, 2 | 5 |  | 5 |  |
| Threshx, highP | dB | 1, 2 | 32 Note6 |  | 48 |  |
| Threshserving, lowP | dB | 1, 2 | 44 |  | 62 Note6 |  |
| Threshx, lowP | dB | 1, 2 | 50 |  | 18 Note6 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |

The cell reselection delay to an already detected low priority cell (Cell 1) for UE fulfilling stationary criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected low priority cell, Cell 1, shall be less than 155 s.

The cell reselection delay to an already detected high priority cell (Cell 2) for UE fulfilling stationary criterion is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected high priority cell, Cell 2, shall be less than 155 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE 1:	The cell re-selection delay to an already detected low priority cell can be expressed as: Tevaluate,NR_Inter_RedCap_Relax + TSI-NR

NOTE 2:	The cell re-selection delay to an already detected higher priority cell can be expressed as: Tevaluate,NR_Inter_RedCap_Relax + TSI-NR

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

## 17.2	RRC_INACTIVE state mobility for RedCap

### 17.2.1	Configured Grant based Small Data Transmissions for RedCap

#### 17.2.1.0	Minimum conformance requirements

##### 17.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation using cg-SDT-RSRP-ChangeThreshold for transmitting in uplink using CG-SDT as specified in TS 38.331 [13].

The requirements in this clause are applicable for the UE performing small data tranmissions using configured resources as TS 38.331 [13].

The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards (i.e. using the timing derived using the latest available NTA value as specified in subclause 7.1.2) the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the small data transmission. The UE determines the small data transmission occasion according to the received CG-SDT configuration as specified in TS 38.331 [13].

When cg-SDT-RSRP-ChangeThreshold [13] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2 in [12], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in TS 38.133 [6] subclause 7.1 provided that

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in [12].

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 17.2.1.0.1-1 are met for FR2-1 regardless of whether the UE is configured with eDRX cycle by serving gNB or core network (AMF).

Table 17.2.1.0.1-1 Valid measurement for FR2-1

| Measurement | FR2-1 |
| --- | --- |
| RSRP1 | (T1 – max{480ms, 8*SMTC periodicity}) ≤ T1’ ≤ (T1 + max{480ms, 8*SMTC periodicity}) |
| RSRP2 | (T2 – max{480ms, 8*SMTC periodicity}) ≤ T2’ ≤ T2 |

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, for the initial CG-SDT transmission, the UE shall not transmit in an CG-SDT occasion that occurs more than 640 ms after T2.

Where:

-	T1 is the time when

-	RRCRelease with CG-SDT configuration (TS 38.331 [2]) is received, or

-	the latest TA is received if TA is received while in RRC_INACITVE state.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in TS 38.321 [12] clause 5.27.2 for transmission using CG-SDT.

-	T2’ is the time when the UE has completed RSRP2.

-	TDRX is the DRX cycle length in ms.

-	M1 the scaling factor as defined in TS 38.133 [6] clause 4.2.2.2.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

For UE which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to SS-RSRP/RSRQ measurement

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH on all symbols within SMTC window duration.

For UEs which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to measurement of serving cell.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/ on SSB symbols of the serving cell.

The UE is not required to meet the following measurement requirements during subsequent SDT transmissions:

-	Measurements of inter-frequency NR cells in TS 38.133 [6] clause 5.1.2.4

-	Measurements of inter-RAT E-UTRAN cells in TS 38.133 [6] clause 5.1.2.5

-	Idle Mode CA/DC Measurements in TS 38.133 [6] clause 5.4

The UE is allowed to delay the reception of PRS resources on the positioning frequency layer until the SDT session is completed if the measurement using PRS resource overlaps with the SDT resources.

The normative reference for this requirement is TS 38.133 [6] clauses 5.2B.

#### 17.2.1.1	NR SA FR2 TA validation for CG-SDT for 2 Rx

Editor’s Note: This test case is incomplete in following aspects:

-	Test procedure needs further updates;

-	TT analysis is still missing.

17.2.1.1.1	Test purpose

To verify that the UE properly perform TA validation for CG-SDT transmission as specified in TS 38.133 [6] clause 5.2B.2.1.

17.2.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting CG-SDT.

17.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.2.1.1.

17.2.1.1.4	Test description

17.2.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.2.1.1.4.1 with following exceptions:

-	Table 7.2.1.1.4.1-1 is replaced by Table 17.2.1.1.4.1-1;

-	Table 7.2.1.1.4.1-2 is replaced by Table 17.2.1.1.4.1-2;

-	Table 7.2.1.1.4.1-3 is replaced by Table 17.2.1.1.4.1-3;

Table 17.2.1.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.2.1.1-1 | NR TDD, SSB SCS 120 kHz, data SCS 120KHz, BW 100 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.2.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.2.1.1.4.1-3: General test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| Active PCell |  | Cell 1 |
| RF Channel Number |  | 1 |
| Duplex mode |  | TDD |
| BWchannel | MHz | 100: NRB,c = 66 |
| DL initial BWP configuration |  | DLBWP.0.1 |
| UL initial BWP configuration |  | ULBWP.0.1 |
| TDD Configuration |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |
| PRACH Configuration |  | PRACH.4 FR2 |
| OCNG parameters |  | OP.5 |
| CP length |  | Normal |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |
| DRX | s | 0.64 |
| cg-SDT-RSRP-ThresholdSSB | dBm | [-110] |
| cg-SDT-RSRP-ChangeThreshold | dB | 8 |
| cg-SDT-TimeAlignmentTime |  | infinity |
| CG-SDT resource period | ms | 320 |
| T_delay_modeB | s | 4 |
| T1 | s | [0.8] |
| T2 | s | [0.96] |
| T3 | s | [3.04] |
| T4 | s | [2.12] |
| T5 | s | [4.58] |

17.2.1.1.4.2	Test procedure

Same as the test procedure given in clause 7.2.1.1.4.2.

17.2.1.1.4.3	Message contents

Same as the message contents given in 7.2.1.1.4.3.

17.2.1.1.5	Test requirements

Same as the test requirements given in clause 7.2.1.1.5 with following exceptions:

-	Table 7.2.1.1.5-1 is replaced by Table 17.2.1.1.5-1.

Table 17.2.1.1.5-1: Cell-specific test parameters (NR Cell 1)

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 4 |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
|  | dBm/15kHz | [-98]+TT |  |  |  |  |
|  | dBm/SCS | [-89] +TT |  |  |  |  |
|  | dB | [0] +TT | [10.35] +TT | [0] +TT | [15.25] +TT | [6] +TT |
|  | dB | [0] +TT | [10.35] +TT | [0] +TT | [15.25] +TT | [6] +TT |
| SS-RSRP | dBm/SCS | [-89] +TT | [-78.65] +TT | [-89] +TT | [-73.75] +TT | [-83] +TT |
| Io | dBm/95.04 MHz | [-57] +TT | [-49.27] +TT | [-57] +TT | [-44.63] +TT | [-53.04] +TT |
| Propagation condition |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

## 17.3	RRC_CONNECTED state mobility for RedCap

### 17.3.1	Handover for RedCap

#### 17.3.1.0	Minimum conformance requirements

##### 17.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover

The purpose of NR handover is to change the NR PCell to another NR cell for RedCap UE. The requirements in this clause are applicable to SA NR.

Handover for a RedCap UE is defined as intra-frequency handover if the center frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the center frequency and SCS of the reference SSB of the target cell, where:

The reference SSB of the serving cell is the SSB in the active DL BWP of serving cell

The reference SSB of the target cell is the SSB in the first active DL BWP of the target cell upon reconfiguration.

The requirements in this clause apply for the following handover scenarios:

Handover to a target cell’s initial DL BWP associated with CD-SSB;

Handover to a target cell’s Redcap specific DL BWP associated with NCD-SSB.

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR2 cell to NR FR2 cell.

When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [13] plus the interruption time.

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is the time required to search the target cell which depends on whether the target cell is already known when the handover command is received by the UE.

-	If the target cell is a known intra-frequency cell, Tsearch = 0ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 8*Trs ms.

-	If the target cell is a known inter-frequency cell, then

if the measured SSB is the target SSB for HO of the target cell, Tsearch = 0 ms;

if the measured SSB and the target SSB for HO belong to the same NR target cell and fulfil the following conditions, Tsearch = Trs ms:

    - CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in RedCap specific DL BWP is the target SSB for HO

    - NCD-SSB in RedCap specific DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for HO

    - Both measured SSB and the target SSB for HO are NCD-SSBs within different RedCap specific DL BWPs

Otherwise, the target cell is an unknown inter-frequency cell. In this case, if the target cell Es/Iot≥-2 dB, then Tsearch = 24* Trs ms.

Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cell.

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the Table 8.1-1 of TS 38.213 [8].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise,

-	Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing as NCD-SSB indicated by nonCellDefiningSSB-r17 if the first active DL BWP included in handover command is configured with nonCellDefiningSSB-r17, otherwise, as CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL in handover command.

-	If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

In FR2, the target cell is known if it has been meeting the following conditions:

-	At least one of the SSBs measured belongs to the same NR target cell,

-	During the last 5 seconds before the reception of the handover command:

-	the UE has sent a valid measurement report for the target cell and

-	at least one of the SSBs measured which belongs to the same NR target cell remains detectable according to the cell identification requirements as described in TS 38.133 [6] clause 9.2B for intra-frequency measurements and TS 38.133 [6] clause 9.3B for inter-frequency measurements,

-	The reference SSB of the NR target cell also remains detectable during the handover delay according to the cell identification requirements are described in TS 38.133 [6] clause 9.2B for intra-frequency handover and TS 38.133 [6] clause 9.3B for inter-frequency handover.

Otherwise, it is unknown.

The normative reference for this requirement is TS 38.133 [6] clauses 6.1D.1.1 and 6.1D.1.3.

#### 17.3.1.1	NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

17.3.1.1.1	Test purpose

To verify the requirement for the NR FR2 intra frequency handover requirements specified in TS 38.133 [6] clause 6.1D.1.3.

17.3.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting FR2.

17.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.1.1.

17.3.1.1.4	Test description

17.3.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.1.2.4.1 with following exceptions:

-	Table 7.3.1.2.4.1-1 is replaced by Table 17.3.1.1.4.1-1;

-	Table 7.3.1.2.4.1-2 is replaced by Table 17.3.1.1.4.1-2;

-	Table 7.3.1.2.4.1-3 is replaced by Table 17.3.1.1.4.1-3;

Table 17.3.1.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.3.1.1-1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.3.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.3.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Offset |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

17.3.1.1.4.2	Test procedure

Same as the test procedure given in clause 7.3.1.2.4.2.

17.3.1.1.4.3	Message contents

Same as the message contents given in 7.3.1.2.4.3.

17.3.1.1.5	Test requirements

Same as the test requirements given in clause 7.3.1.2.5 with following exceptions:

-	Table 7.3.1.2.5-1 is replaced by Table 17.3.1.1.5-1.

Table 17.3.1.1.5-1: Cell-specific test parameters (NR Cell 1)

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  | Rough |  | Rough |  |
| AoA setup |  |  | Setup 1 as defined in A.9 |  |  |  |
| NR RF Channel Number |  |  | 1 |  |  |  |
| Duplex mode |  |  | TDD |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  |  | 66 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |  |  |  |
| Control Channel RMC |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  | SSB.3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 120 |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR2 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7+TT |  |  |  |
| Note2 |  | dBm/SCS | -95.7+TT |  |  |  |
|  |  | dB | 6+TT | -1.8+TT | -Infinity | 0+TT |
|  |  | dB | 6+TT | 6+TT | -Infinity | 7+TT |
| IoNote3 |  | dBm/ BW | -59.7+TT | -56.7+TT | -59.7+TT | -56.7+TT |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

#### 17.3.1.2	NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

17.3.1.2.1	Test purpose

To verify the requirement for the NR FR2 NCD-SSB to NR FR2 NCD-SSB inter frequency handover requirements specified in TS 38.133 [6] clause 6.1D.1.3

17.3.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards and supporting FR2.

17.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.1.2.

17.3.1.2.4	Test description

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively.

-	At the start of time duration T1, the UE does not have any timing information of Cell 2.

-	Starting T2, Cell 2 becomes detectable and the UE receives an RRC handover command from the network indicating the UE to handover to Cell 2 with firstActiveDownlinkBWP-Id configured to DL BWP and firstActiveUplinkBWP-Id configured to UL BWP, where Cell 2’s DL BWP is the Redcap specific BWP associated with NCD-SSB, then UE handover from cell 1’s NCD-SSB to cell 2’s specific Redcap BWP associated with NCD-SSB. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

17.3.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 17.3.1.2.4.1-1.

Table 17.3.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.3.1.2-1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 17.3.1.2.4.1-2.

Table 17.3.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 17.3.1.2.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 17.3.1.2.4.1-3 and 17.3.1.2.5-1 below.

Table 17.3.1.2.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Offset |  | dB | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

17.3.1.2.4.2	Test procedure

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell and its RedCap-specific dedicated BWP is the activated BWP. The SS transmits NCD-SSB on Cell 1 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of Cell 1 CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;

-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

2.	Set the parameters according to T1 in Table 17.3.1.2.5-1.

3. 	Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4.	SS shall transmit an RRCReconfiguration message implying handover to the RedCap-specific dedicated BWP of Cell 2.

5.	At the instant when the last TTI containing the RRCReconfiguration message in step 4) is sent to the UE, the SS shall switch the power settings from T1 to T2 as specified in Table 17.3.1.2.5-1. T2 starts. The SS transmits NCD-SSB on Cell 2 with an absolute frequency (in ARFCN value) of

fNCDSSB=fpointA+NSCRBSCSSSBΔFGlobal⋅fCDSSB-fpointA⋅ΔFGlobalNSCRBSCSSSB+NSCSSBSCSSSBΔFGlobal

where,

-	fCDSSB is the absolute frequency of Cell 2 CD-SSB (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	fpointA is the absolute frequency of Point A (in ARFCN value) specified in 38.508-1 [14] clause 7.2.3.1;

-	ΔFGlobal is the granularity of the global frequency raster (in kHz) specified in 38.101-1 [2] clause 5.4.2.1;

-	SCSSSB is the subcarrier spacing of SSB (in kHz);

-	NSCRB=12, is the number of subcarriers in an RB;

-	NSCSSB=240, is the number of subcarriers in an SSB.

6.	If the UE transmits a PRACH preamble to Cell 2 satisfying the delay requirement specified in clause 17.3.1.2.5 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, the SS shall

-	transmit an RRCReconfiguration message to cause the UE handover back to Cell 1. if the handover fails, switch off and on the UE

or

-	switch off and on the UE.

8.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell and its RedCap-specific dedicated BWP is the activated BWP. The SS transmits NCD-SSB on Cell 1 with an absolute frequency given in step 1.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

17.3.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 17.3.1.2.4.3-1: RRCReconfiguration (Step 1, 4 and 8, test procedure)

| Derivation Path: Table H.3.2-2 with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 17.3.1.2.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.3.1.2.4.3-2: CellGroupConfig (Table 17.3.1.2.4.3-1)

| Derivation Path: Table H.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| smtc | SMTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap |  |  |
| } |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 17.3.1.2.4.3-3 |  |
| } |  |  |  |

Table 17.3.1.2.4.3-3: ServingCellConfig (Table 17.3.1.2.4.3-2)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 17.3.1.2.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 17.3.1.2.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.3.1.2.4.3-4: BWP-Downlink (Table 17.3.1.2.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.3 RedCap corresponding to NCD-SSB PRB index |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.3.1.2.4.3-5: BWP-Uplink (Table 17.3.1.2.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.3 RedCap. The starting PRB index for dedicated UL BWP is the same as the corresponding RedCap-specific dedicated DL BWP |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

17.3.1.2.5	Test requirement

Table 17.3.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 17.3.1.2.5-1: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  | Rough |  | Rough |  |
| AoA setup |  |  | Setup 1 as defined in A.9.1 |  |  |  |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode |  |  | TDD |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  |  | 66 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH RMC |  |  | SR.3.1 TDD |  |  |  |
| RMSI CORESET RMC |  |  | CR.3.1 TDD |  |  |  |
| Control Channel RMC |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP. 1 |  |  |  |
| SMTC Configuration for CD-SSB |  |  | SMTC.1 |  |  |  |
| SMTC Configuration for NCD-SSB |  |  | SMTC.2 RedCap |  |  |  |
| CD-SSB Configuration |  |  | SSB.3 FR2 |  |  |  |
| NCD-SSB Configuration |  |  | SSB.3 RedCap FR2 Note 7 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 120 |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR2 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |  |  |
| BWP configuraiton | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 RedCap Note 8 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 RedCap Note 9 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7+TT |  | -104.7+TT |  |
| Note2 |  | dBm/SCS | -95.7+TT |  | -95.7+TT |  |
|  |  | dB | 5+TT | 5+TT | -Infinity | 5+TT |
|  |  | dB | 5+TT | 5+TT | -Infinity | 5+TT |
| IoNote3 |  | dBm/Ch BW | -60.5+TT | -60.5+TT | -66.7+TT | -60.5+TT |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: NCD-SSB is configured within dedicated RedCap DL BWP and CD-SSB is configured completely outside the dedicated RedCap DL BWP Note 8: The starting PRB index for dedicated DL BWP corresponding to NCD-SSB PRB index Note 9: The starting PRB index for dedicated UL BWP is the same as the dedicated DL BWP corresponding to NCD-SSB PRB index |  |  |  |  |  |  |

The handover delay is measured from the beginning of time period T2 to the time that the UE transmit the first PRACH to Cell 2, which is calculated as follows:

Handover delay = RRC procedure delay + Tinterrupt

-	RRC procedure delay = 10 ms, is the RRC processing delay specified in TS 38.331 [13] clause 12,

-	Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin, is the interruption time specified in TS 38.133 [6] clause 6.1D.1.2.

-	Tsearch = 1920 ms, is the time required to search the unknown target cell;

-	TIU = 20 ms, is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

-	Tprocessing = 20 ms, is time for UE processing;

-	T∆ = 80 ms, is time for fine time tracking and acquiring full timing information of the target cell;

-	Tmargin = 2 ms, is time for SSB post-processing.

This gives a total of 2052 ms.

### 17.3.2	RRC connection mobility control for RedCap

#### 17.3.2.1	RRC re-establishment for RedCap

##### 17.3.2.1.0	Minimum conformance requirements

The requirements in clause 7.3.2.1.0 shall apply.

The normative reference for this requirement is TS 38.133 [6] clause 6.2.1B.

##### 17.3.2.1.1	NR SA FR2 Intra-frequency RRC Re-establishment in FR2

Editor’s Note: This test case has been completed for the following configurations:

-	Test frequency f ≤ 40.8 GHz

-	UE PC3

The test is incomplete for UE power classes other than PC3 (e.g. PC7)

The test is incomplete for test frequencies > 40.8 GHz

17.3.2.1.1.1	Test purpose

To verify hat the NR intra-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits. These tests will verify the requirements in 38.133 [6] clause 6.2.1B

17.3.2.1.1.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards.

17.3.2.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.2.1.1

17.3.2.1.1.4	Test description

17.3.2.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.2.1.1.4.1 with following exceptions:

-	Table 7.3.2.1.1.4.1-2 is replaced by Table 17.3.2.1.1.4.1-1.

Table 17.3.2.1.1.4.1-1: General test parameters

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 RedCap FR2 |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | PRACH.1 FR2 | As specified in Annex A.7 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133) |
| T3 |  | s | 1 | 5 |  |

17.3.2.1.1.4.2	Test procedure

Same as the test procedure given in clause 7.3.2.1.1.4.2.

17.3.2.1.1.4.3	Message contents

Same as the message contents given in clause 7.3.2.1.1.4.3.

17.3.2.1.1.5	Test requirement

Same as the test requirements given in clause 7.3.2.1.1.5 with following exceptions:

-	Table 7.3.2.1.1.5-1 is replaced by Table 17.3.2.1.1.5-1.

Table 17.3.2.1.1.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  | 1 | 24 |  |  | 24 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 |  |  | Setup 1 |  |  |
|  | dB | 1 | -0.12 | -infinity | -infinity | -3.46 | 2 | 2 |
| Note2 | dBm/ SCS | 1 | -104.7 |  |  |  |  |  |
| Note2 | dBm/ 15 kHz | 1 | -95.7 |  |  |  |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | 2 | 2 | 2 |
| SS-RSRP Note3 | dBm/ SCS | 1 | -91.7 | -infinity | -infinity | -93.7 | -93.7 | -93.7 |
| Io | dBm/ 95.04 MHz | 1 | -64.00 | -66.95 | -66.95 | -64.00 | -66.95 | -66.95 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

##### 17.3.2.1.2	NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2

Editor’s Note: This test case has been completed for the following configurations:

-	Test frequency f ≤ 40.8 GHz

-	UE PC3

The test is incomplete for UE power classes other than PC3 (e.g. PC7)

The test is incomplete for test frequencies > 40.8 GHz

17.3.2.1.2.1	Test purpose

To verify that the NR inter-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits. These tests will verify the requirements in 38.133 [6] clause 6.2.1B

17.3.2.1.2.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards.

17.3.2.1.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.2.1.2

17.3.2.1.2.4	Test description

17.3.2.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.2.1.2.4.1 with following exceptions:

-	Table 7.3.2.1.2.4.1-2 is replaced by Table 17.3.2.1.2.4.1-1.

Table 17.3.2.1.2.4.1-1: General test parameters

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1,2 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 RedCap FR2 |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | PRACH.1 FR2 | As specified in Annex A.7 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133) |
| T3 |  | s | 1 | 6 |  |

17.3.2.1.2.4.2	Test procedure

Same as the test procedure given in clause 7.3.2.1.2.4.2.

17.3.2.1.2.4.3	Message contents

Same as the message contents given in clause 7.3.2.1.2.4.3.

17.3.2.1.2.5	Test requirement

Same as the test requirements given in clause 7.3.2.1.2.5 with following exceptions:

-	Table 7.3.2.1.2.5-1 is replaced by Table 17.3.2.1.2.5-1.

Table 17.3.2.1.2.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| AoA setup |  | 1 | Setup 3 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  | 1 | 24 |  |  | 24 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| Note2 | dBm/15 kHz | 1 | -92.3 Note 6 |  |  | -92.3 Note 6 |  |  |
| Note2 | dBm/SCS | 1 | -83.3 |  |  | -83.3 |  |  |
|  | dB | 1 | 0 | -infinity | -infinity | -infinity | -infinity | 0 |
| BB Note 5 | dB | 1 | -1.05 | -infinity | -infinity | -infinity | -infinity | -1.05 |
| SSB_RP Note3 | dBm/SCS | 1 | -83.3 | -infinity | -infinity | -infinity | -infinity | -83.3 |
| Io | dBm/95.04 MHz | 1 | -55.66 | -58.68 | -58.68 | -58.68 | -58.68 | -55.66 |
| Propagation Condition |  | 1 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that a constant total transmitted power is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |  |  |

##### 17.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.

-	Message content is TBD

17.3.2.1.3.1	Test purpose

To verify that the NR intra-frequency RRC re-establishment delay in FR2 without serving cell timing is within the specified limits. These tests will verify the requirements in clause 6.2.1B.

17.3.2.1.3.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards.

17.3.2.1.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.2.1.3

17.3.2.1.3.4	Test description

17.3.2.1.3.4.1	Initial conditions

Same as the initial condition given in clause 7.3.2.1.3.4.1 with following exception:

-	Table 7.3.2.1.3.4.1-1 is replaced by Table 17.3.2.1.3.4.1-1

-	Table 7.3.2.1.3.4.1-2 is replaced by Table 17.3.2.1.3.4.1-2

-	Table 7.3.2.1.3.4.1-3 is replaced by Table 17.3.2.1.3.4.1-3

Table 17.3.2.1.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.3.2.1.3-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table 17.3.2.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5.1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2 |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.2.1.3.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 17.3.2.1.3.4.1-3: General test parameters

| Parameter |  | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 RedCap FR2 |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | PRACH.1 FR2 | Table A.3.8.3.1-1 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 10.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1B in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1B.5 in TS 38.133 [6] |
| T3 |  | s | 1 | 5 |  |

17.3.2.1.3.4.2	Test procedure

Same as the test procedure given in clause 7.3.2.1.3.4.2.

17.3.2.1.3.4.3	Message contents

TBD

17.3.2.1.3.5	Test requirement

Same as the test requirements given in clause 7.3.2.1.3.5 with following exceptions:

-	Table 7.3.2.1.3.5-1 is replaced by Table 17.3.2.1.3.5-1.

Table 17.3.2.1.3.5-1: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 |  |  | Setup 1 |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| Note2 | dBm/15 kHz | 1 | -104.7 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -95.7 |  |  |  |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| SS-RSRP Note3 | dBm/SCS | 1 | -90.7 | -infinity | -infinity | -infinity | -infinity | -90.7 |
| Io | dBm/95.04 MHz | 1 | -60.52 | -66.71 | -60.52 | -60.52 | -66.71 | -60.52 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

#### 17.3.2.2	Random access for RedCap

##### 17.3.2.2.0	Minimum conformance requirements

###### 17.3.2.2.0.1	Minimum conformance requirements for Contention based random access

The minimum conformance requirements given in clause 7.3.2.2.0.1 are applicable for TDD RedCap UEs.

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2B.2.

###### 17.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access

The minimum conformance requirements given in clause 7.3.2.2.0.2 are applicable for TDD RedCap UEs.

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2B.2.

##### 17.3.2.2.1	NR SA FR2 4-step RA type contention based random access test in FR2 for NR Standalone

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	TT analysis is missing.

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

17.3.2.2.1.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

17.3.2.2.1.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards.

17.3.2.2.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.2.2.1.

17.3.2.2.1.4	Test description

17.3.2.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.2.2.1.4.1 with following exceptions:

-	Table 7.3.2.2.1.4.1-1 is replaced by Table 17.3.2.2.1.4.1-1.

-	Table 7.3.2.2.1.4.1-2 is replaced by Table 17.3.2.2.1.4.1-2.

Table 17.3.2.2.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.3.2.2.1-1 | NR PCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 17.3.2.2.1.4.1-2: Test Environment

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram |  |  |  |

17.3.2.2.1.4.2	Test procedure

Same as the test procedure given in clause 7.3.2.2.1.4.2.

17.3.2.2.1.4.3	Message contents

Same as the message contents given in clause 7.3.2.2.1.4.3.

17.3.2.2.1.5	Test requirement

Same as the test requirements given in clause 7.3.2.2.1.5 with following exception:

-	Table 7.3.2.2.1.5-1 is replaced by Table 17.3.2.2.1.5-1.

-	Table 7.3.2.2.1.5-2 is replaced by Table 17.3.2.2.1.5-2.

Table 17.3.2.2.1.5-1: General test parameters

| Parameter | Unit | Test-1 | Comments |
| --- | --- | --- | --- |
| SSB Configuration |  | SSB.1 FR2 | As defined in A.3.2 |
| CSI-RS for tracking |  | TRS.2.1 TDD |  |
| Duplex Mode |  | TDD |  |
| TDD Configuration |  | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 24 |  |
| OCNG Pattern Note 1 |  | OP.1 | As defined in A.2.1 |
| PDSCH Reference Channel Note 2 |  | SR.3.1 TDD | As defined in A.1.1 |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | As defined in A.1.2 |
| NR RF Channel Number |  | 1 |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS | dB |  |  |
| ss-PBCH-BlockPower | dBm/SCS | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 of TS 38.101-2 [3] |
| PRACH Configuration |  | PRACH.1 FR2 | As defined in A.7.2, with exceptions as defined below |
| rsrp-ThresholdSSB | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower | dBm | -100 | As defined in TS 38.331 [13] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |

Table 17.3.2.2.1.5-2: OTA-related test parameters

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### 17.3.2.2.2	NR SA FR2 4-step RA type non-contention based random access test in FR2 for NR Standalone

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	TT analysis is missing

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

17.3.2.2.2.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

17.3.2.2.2.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards. Additionally Test 2 is applicable to NR RedCap UEs that supports CSI-RS based Random Access Preamble.

17.3.2.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.2.2.2.

17.3.2.2.2.4	Test description

17.3.2.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.2.2.2.4.1 with following exceptions:

-	Table 7.3.2.2.2.4.1-1 is replaced by Table 17.3.2.2.2.4.1-1.

-	Table 7.3.2.2.2.4.1-2 is replaced by Table 17.3.2.2.2.4.1-2.

Table 17.3.2.2.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.3.2.2.2-1 | NR PCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 17.3.2.2.2.4.1-2: Test Environment

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram |  |  |  |

17.3.2.2.2.4.2	Test procedure

Same as the test procedure given in clause 7.3.2.2.2.4.2.

17.3.2.2.2.4.3	Message contents

Same as the message contents given in clause 7.3.2.2.2.4.3.

17.3.2.2.2.5	Test requirement

Same as the test requirements given in clause 7.3.2.2.2.5 with following exception:

-	Table 7.3.2.2.2.5-1 is replaced by Table 17.3.2.2.2.5-1.

-	Table 7.3.2.2.2.5-2 is replaced by Table 17.3.2.2.2.5-2.

Table 17.3.2.2.2.5-1: General test parameters

| Parameter | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration |  | SSB.1 FR2 | SSB.1 FR2 | As defined in A.3.2 |
| CSI-RS for tracking |  | TRS.2.1 TDD | TRS.2.1 TDD |  |
| CSI-RS Configuration |  | N/A | CSI-RS.3.1 TDD | As defined in A.1.4 |
| Duplex Mode |  | TDD | TDD |  |
| TDD Configuration |  | TDDConf.3.1 | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 24 | 24 |  |
| OCNG Pattern Note 1 |  | OP.3 | OP.3 | As defined in A.2.1. |
| PDSCH Reference Channel Note 2 |  | SR.3.1 TDD | SR.3.1 TDD | As defined in A.1.1. |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | CR.3.1 TDD | As defined in A.1.2 |
| NR RF Channel Number |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS | dB |  |  |  |
| ss-PBCH-BlockPower | dBm/ SCS | +20 +ΔUL | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () | dBm | maximum value configurable for certain power class | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [3] |
| PRACH Configuration |  | PRACH.2 FR2 | PRACH.3 FR2 | As defined in A.7.2, with exceptions as defined below. |
| rsrp-ThresholdSSB | dBm | RSRP_69 +ΔDL | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower | dBm | -100 | -100 | As defined in TS 38.331 [13] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table 17.3.2.2.2.5-2: OTA-related test parameters

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  | Rough | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 | -80.6 |  |
|  | Es/IotBB | dB | 21.09 | 21.09 |  |
|  | Io | dBm/ 95.04 MHz | -56.01 | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 | -95.0 |  |
|  | Es/IotBB | dB | 6.69 | 6.69 |  |
|  | Io | dBm/ 95.04 MHz | -70.41 | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### 17.3.2.2.3	NR SA FR2 2-step RA type contention based random access test in FR2 for NR Standalone

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	TT analysis is missing

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

17.3.2.2.3.1	Test purpose

To verify that the behaviour of the 2-step RA type random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

17.3.2.2.3.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards supporting 2-step RACH.

17.3.2.2.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.2.2.3.

17.3.2.2.3.4	Test description

17.3.2.2.3.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.2.2.3.4.1 with following exceptions:

-	Table 7.3.2.2.3.4.1-1 is replaced by Table 17.3.2.2.3.4.1-1.

-	Table 7.3.2.2.3.4.1-2 is replaced by Table 17.3.2.2.3.4.1-2.

Table 17.3.2.2.3.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.3.2.2.3-1 | NR PCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 17.3.2.2.3.4.1-2: Test Environment

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.2.2.3.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram |  |  |  |

17.3.2.2.3.4.2	Test procedure

Same as the test procedure given in clause 7.3.2.2.3.4.2.

17.3.2.2.3.4.3	Message contents

Same as the message contents given in clause 7.3.2.2.3.4.3.

17.3.2.2.3.5	Test requirement

Same as the test requirements given in clause 7.3.2.2.3.5 with following exception:

-	Table 7.3.2.2.3.5-1 is replaced by Table 17.3.2.2.3.5-1.

-	Table 7.3.2.2.3.5-2 is replaced by Table 17.3.2.2.3.5-2.

Table 17.3.2.2.3.5-1: General test parameters

| Parameter | Unit | Test-1 | Comments |
| --- | --- | --- | --- |
| SSB Configuration |  | SSB.1 RedCap FR2 | As defined in A.3.10 |
| Duplex Mode for Cell 1 |  | TDD |  |
| TDD Configuration |  | TDDConf.3.1 | As defined in A.3.1.4 |
| BWchannel | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH Reference Channel Note 2 |  | SR.3.1 TDD | As defined in A.3.1.1. |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | As defined in A.3.1.2 |
| NR RF Channel Number |  | 1 |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS | dB |  |  |
| ss-PBCH-BlockPower | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [2]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [19] |
| MsgA Configuration |  | FR2 MsgA configuration 1 | As defined in A.3.20.3, with exceptions as defined below |
| msgA-RSRP-ThresholdSSB | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower | dBm | -100 | As defined in TS 38.331 [2] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PMsgA0 -1), where PMsgA0 is the measured first MsgA PRACH power with -80.6dBm/SCS applied, msgA-PreambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send MsgA. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |

Table 17.3.2.2.3.5-2: OTA-related test parameters

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 2b | As defined in A.9.2 |
| Assumption for UE beamsNote 2 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### 17.3.2.2.4	NR SA FR2 2-step RA type non-contention based random access test in FR2 for NR Standalone

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	TT analysis is missing

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

17.3.2.2.4.1	Test purpose

To verify that the behaviour of the 2-step RA type random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

17.3.2.2.4.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards and supporting 2-step RACH.

17.3.2.2.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.3.2.2.4.

17.3.2.2.4.4	Test description

17.3.2.2.4.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.2.2.4.4.1 with following exceptions:

-	Table 7.3.2.2.4.4.1-1 is replaced by Table 17.3.2.2.4.4.1-1.

-	Table 7.3.2.2.4.4.1-2 is replaced by Table 17.3.2.2.4.4.1-2.

Table 17.3.2.2.4.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.3.2.2.4-1 | NR PCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 17.3.2.2.4.4.1-2: Test Environment

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.2.2.4.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram |  |  |  |

17.3.2.2.4.4.2	Test procedure

Same as the test procedure given in clause 7.3.2.2.4.4.2.

17.3.2.2.4.4.3	Message contents

Same as the message contents given in clause 7.3.2.2.4.4.3.

17.3.2.2.4.5	Test requirement

Same as the test requirements given in clause 7.3.2.2.4.5 with following exception:

-	Table 7.3.2.2.4.5-1 is replaced by Table 17.3.2.2.4.5-1.

-	Table 7.3.2.2.4.5-2 is replaced by Table 17.3.2.2.4.5-2.

Table 17.3.2.2.4.5-1: General test parameters

| Parameter | Unit | Test-1 | Comments |
| --- | --- | --- | --- |
| SSB Configuration |  | SSB.1 RedCap FR2 | As defined in A.3.10 |
| Duplex Mode for Cell 1 |  | TDD |  |
| TDD Configuration |  | TDDConf.3.1 | As defined in A.3.1.4 |
| BWchannel | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  | OP.3 | As defined in A.3.2.1. |
| PDSCH Reference Channel Note 2 |  | SR.3.1 TDD | As defined in A.3.1.1. |
| NR RF Channel Number |  | 1 |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS | dB |  |  |
| ss-PBCH-BlockPower | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [2]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [19] |
| MsgA Configuration |  | FR2 MsgA configuration 1 | As defined in A.3.20.3, with exceptions as defined below |
| msgA-RSRP-ThresholdSSB | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower | dBm | -100 | As defined in TS 38.331 [2] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PMsgA0 -1), where PMsgA0 is the measured first MsgA PRACH power with -80.6dBm/SCS applied, msgA-PreambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send MsgA. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |

Table 17.3.2.2.4.5-2: OTA-related test parameters

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 2b | As defined in A.9.2 |
| Assumption for UE beamsNote 2 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

#### 17.3.2.3	RRC Connection Release with Redirection for RedCap

##### 17.3.2.3.0	Minimum conformance requirements

###### 17.3.2.3.0.1	Minimum conformance requirements for FR2-FR2 RRC connection release with redirection

The minimum requirements given in clause 7.3.2.3.0.1 shall apply.

##### 17.3.2.3.1	NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

17.3.2.3.1.1	Test purpose

To verify RRC connection release with redirection from NR to NR requirements specified in clause 6.2.3A.2.1.

17.3.2.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards and supporting FR2.

17.3.2.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.3.2.3.0.1.

The normative reference for this requirement is TS 38.133 [6] A.17.3.2.3.1.

17.3.2.3.1.4	Test description

17.3.2.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.3.2.3.1.4.1 with following exceptions:

-	Table 7.3.2.3.1.4.1-1 is replaced by Table 17.3.2.3.1.4.1-1;

-	Table 7.3.2.3.1.4.1-2 is replaced by Table 17.3.2.3.1.4.1-2;

-	Table 7.3.2.3.1.4.1-3 is replaced by Table 17.3.2.3.1.4.1-3;

Table 17.3.2.3.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.3.2.3.1-1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table 17.3.2.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.15 and TS 38.508-1 [14] clause 4.3.1 and 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.3.2.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.3.2.3.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 3.2 |  |

17.3.2.3.1.4.2	Test procedure

Same as the test procedure given in clause 7.3.2.3.1.4.2.

17.3.2.3.1.4.3	Message contents

Same as the message contents given in clause 7.3.2.3.1.4.3.

17.3.2.3.1.5	Test requirement

Same as the test requirements given in 7.3.2.3.1.5 with followings exception:

-	Table 7.3.2.3.1.5-1 is replaced by Table 17.3.2.3.1.5-1.

Table 17.3.2.3.1.5-1: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  | Rough |  | Rough |  |
| AoA setup |  |  | Setup 1 as defined in A.9 |  |  |  |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode |  |  | TDD |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  |  | 66 |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH RMC |  |  | SR.3.1 TDD |  |  |  |
| RMSI CORESET RMC |  |  | CR.3.1 TDD |  |  |  |
| Control Channel RMC |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC configuration |  |  | SMTC.1 FR2 |  |  |  |
| SSB Configuration |  |  | SSB.3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 120 |  |  |  |
| PRACH configuration |  |  | PRACH.1 FR2 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS | -95.7 |  | -95.7 |  |
|  |  | dB | 5 | 5 | -Infinity | 5 |
|  |  | dB | 5 | 5 | -Infinity | 5 |
| IoNote3 |  | dBm/BW | -60.5 | -60.5 | -66.7 | -60.5 |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

## 17.4	Timing for RedCap

### 17.4.1	UE transmit timing for RedCap

#### 17.4.1.0	Minimum Conformance Requirements

##### 17.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy

The UE shall have capability to follow the frame timing change of the reference cell in connected state or when transmiting PUSCH on CG resources for SDT in RRC_Inactive. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 17.4.1.0.1-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

-	when it is the transmission for PUSCH on CG resources for SDT in RRC_Inactive.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms on the condition that:

-	the SSB is within the UE’s active BWP, or

-	the SSB is not within the UE’s active BWP, and the measurement gap is configured.

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at UE antenna. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in TS 38.133 [6] clause 7.3A was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table Table 17.4.1.0.1-2.

Table 17.4.1.0.1-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 2 | 240 | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 17.4.1.0.1-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR2 | 13792 |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in TS 38.133 [6] clause 7.3A is applied.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 17.4.1.0.1-3.

Table 17.4.1.0.1-3: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 2 | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The normative reference for this requirement is TS.38.133 [6] clause 7.1A.1 and 7.1A.2.

#### 17.4.1.1	NR SA FR2 NR UE Transmit Timing Test for FR2

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is still missing

17.4.1.1.1	Test purpose

To verify that the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 7.1A.2.

17.4.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onward and supporting FR2.

17.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.4.1.0.1.

The normative reference for this requirement is TS.38.133 [6] clause A.17.4.1.1

17.4.1.1.4	Test Description

17.4.1.1.4.1	Initial Conditions

Same as the initial conditions given in clause 7.4.1.1.4.1 with following exceptions:

-	Table 7.4.1.1.4.1-1 is replaced by Table 17.4.1.1.4.1-1;

-	Table 7.4.1.1.4.1-2 is replaced by Table 17.4.1.1.4.1-2;

Table 17.4.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.4.1.1-1 | NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

Table 17.4.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.4.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

17.4.1.1.4.2	Test procedure

Same as the test procedure given in 7.4.1.1.4.2.

17.4.1.1.4.3	Message contents

Same as the message contents given in 7.4.1.1.4.3.

17.4.1.1.5	Test Requirements

Same as the test requirements given in 7.4.1.1.5 with following exceptions:

-	Table 7.4.1.1.5-1 is replaced by Table 17.4.1.1.5-1;

-	Table 7.4.1.1.5-2 is replaced by Table 17.4.1.1.5-2;

-	Table 7.4.1.1.5-3 is replaced by Table 17.4.1.1.5-3;

-	Table 7.4.1.1.5-4 is replaced by Table 17.4.1.1.5-4;

-	Table 7.4.1.1.5-5 is replaced by Table 17.4.1.1.5-5;

Table 17.4.1.1.5-1: Cell specific test parameters

| Parameter | Unit | Test1 | Test2 |
| --- | --- | --- | --- |
| SSB ARFCN |  | Freq1 | Freq1 |
| Duplex Mode |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 66 |  |
| Initial BWP Configuration |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  | DLBWP.1.1 ULBWP.1.1 |  |
| TRS Configuration |  | TRS.2.1 TDD |  |
| PDSCH/PDCCH TCI state |  | TCI.State.2 |  |
| DRx Cycle | ms | N/A | DRX.8Note2 |
| PDSCH Reference measurement channel |  | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel |  | CR.3.2 TDD |  |
| Dedicated CORESET Reference Channel |  | CCR.3.7 TDD |  |
| OCNG Patterns |  | OP.1 |  |
| SSB Configuration |  | SSB.1 FR2 |  |
| SMTC Configuration |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition |  | AWGN |  |
| SRS Config |  | SRSConf.1Note3 | SRSConf.2Note3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: DRX related parameters are given in Table A.5-1 Note 3: SRS configs are given in Table A.17.4.1.1.5-2 |  |  |  |

Table 17.4.1.1.5-2: SRS Configurations

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1,0 | sl2560,4 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

Table 17.4.1.1.5-3: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined in A.9.1 |  |
| Assumption for UE beamsNote 5 |  | Fine |  |
| Note1 | dBm/15kHzNote3 | -112+TT |  |
| Note1 | dBm/SCS | -100+TT |  |
|  | dB | 4+TT |  |
| SS-RSRPNote2 | dBm/SCS Note3 | -96+TT |  |
|  | dB | 4+TT |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5+TT |  |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone NOTE 4: As observed with 0dBi gain antenna at the centre of the quiet zone NOTE 5: Information about types of UE beam is given in TS 38.133 [6] section B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table 17.4.1.1.5-4: Te Timing Error Limit

| Frequency Range | SCS of SSB signals ( kHz) | SCS of uplink signals ( kHz) | Te |
| --- | --- | --- | --- |
| 2 | 240 | 120 | 3.75*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 17.4.1.1.5-5: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (KHz) | The Maximum timing change in one adjustment Tq | The Minimum aggregate adjustment rate Tp | The Maximum aggregate adjustment rate Tq |
| --- | --- | --- | --- | --- |
| 2 | 120 | 3.125*64*Tc | -1.225*64*Tc | +3.725*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

### 17.4.2	UE timer accuracy for RedCap

### 17.4.3	Timing advance for RedCap

#### 17.4.3.0	Minimum conformance requirements

##### 17.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy

The minimum requirements given in clause 7.4.3.0.1 shall apply.

17.4.3.1	NR SA FR2 timing advance adjustment accuracy

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

17.4.3.1.1	Test purpose

To verify UE timing advance adjustment delay and accuracy requirement defined in TS 38.133 clause 7.3A.

17.4.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from release 17 onward and supporting FR2.

17.4.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.4.3.0.1.

The normative reference for this requirement is TS.38.133 [6] clause A.17.4.3.1.

17.4.3.1.4	Test description

17.4.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.4.3.1.4.1 with following exceptions:

-	Table 7.4.3.1.4.1-1 is replaced by Table 17.4.3.1.4.1-1;

-	Table 7.4.3.1.4.1-2 is replaced by Table 17.4.3.1.4.1-2;

-	Table 7.4.3.1.4.1-3 is replaced by Table 17.4.3.1.4.1-3.

Table 17.4.3.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.4.3.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table 17.4.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table 1E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.4.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.4.3.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.8.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.8.1-2 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.8.2-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.8.2-2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For 120 kHz SCS NTA_new = NTA_old + 1024*Tc (based on equation in TS 38.213 [3] clause 4.2) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

17.4.3.1.4.2	Test Procedure

Same as the test procedure given in clause 7.4.3.1.4.2.

17.4.3.1.4.3	Message Contents

Same as the message contents given in clause 7.4.3.1.4.3 with following exceptions:

-	Table 7.3.1-3a is used with associated condition SSB.1 FR2.

17.4.3.1.5	Test Requirement

Same as the test requirement given in 7.4.3.1.5 with following exceptions:

-	Table 7.4.3.1.5-1 is replaced by Table 17.4.3.1.5-1;

-	Table 7.4.3.1.5-1a is replaced by Table 17.4.3.1.5-2;

-	Table 7.4.3.1.5-2 is replaced by Table 17.4.3.1.5-3;

-	Table 7.4.3.1.5-3 is replaced by Table 17.4.3.1.5-4.

Table 17.4.3.1.5-1: Cell specific test parameters

| Parameter | Unit | Test1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Duplex mode |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |
| BWP BW | MHz | 100: NRB,c = 66 |  |
| DRx Cycle | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  |
| CORESET Reference Channel |  | CR.3.1 TDD |  |
| OCNG Patterns |  | OP.1 |  |
| TRS configuration |  | TRS.2.1 TDD |  |
| PDSCH/PDCCH TCI state |  | TCI.State.2 |  |
| SMTC configuration |  | SMTC.1 FR2 |  |
| SSB Configuration |  | SSB.1 FR2 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing | kHz | 120 |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone |  |  |  |

Table 17.4.3.1.5-2: OTA specific test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Angle of arrival configuration |  | Setup 1 as defined in A.9 |  |
| Assumption for UE beamsNote 6 |  | Fine |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -103 |  |
|  | dB | 4 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -99 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone NOTE 5: As observed with 0dBi gain antenna at the centre of the quiet zone NOTE 6: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation |  |  |  |

Table 17.4.3.1.5-3: Sounding Reference Symbol Configuration

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 16 | Frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=4 | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | Codebook | Codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| NOTE: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

Table 17.4.3.1.5-4: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 120 |
| --- | --- |
| UE Timing Advance adjustment accuracy | ±72 Tc |

## 17.5	Signalling characteristics for RedCap

### 17.5.1	Radio link monitoring for RedCap

#### 17.5.1.0	Minimum requirements

##### 17.5.1.0.1	General

The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell as specified in TS 38.213 [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP. The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified.

On each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell.

The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate (BLERout) as defined in Table 17.5.1.0.1-1. For SSB based radio link monitoring, Qout_SSB,RedCap is derived based on the hypothetical PDCCH transmission parameters listed in Table 17.5.1.0.1-2. For CSI-RS based radio link monitoring, Qout_CSI-RS,RedCap is derived based on the hypothetical PDCCH transmission parameters listed in Table 17.5.1.0.1-4.

The threshold Qin is defined as the level at which the downlink radio link quality can be significantly more reliably received than at Qout and shall correspond to the in-sync block error rate (BLERin) as defined in Table 17.5.1.0.1-1. For SSB based radio link monitoring, Qin_SSB,RedCap is derived based on the hypothetical PDCCH transmission parameters listed in Table 17.5.1.0.1-3. For CSI-RS based radio link monitoring, Qin_CSI-RS,RedCap is derived based on the hypothetical PDCCH transmission parameters listed in Table 17.5.1.0.1-5.

The out-of-sync block error rate (BLERout) and in-sync block error rate (BLERin) are determined from the network configuration via parameter rlmInSyncOutOfSyncThreshold signalled by higher layers. When UE is not configured with RLM-IS-OOS-thresholdConfig from the network, UE determines out-of-sync and in-sync block error rates from Configuration #0 in Table 17.5.1.0.1-1 as default. All requirements here are applicable for BLER Configuration #0 in Table 17.5.1.0.1-1.

Table 17.5.1.0.1-1: Out-of-sync and in-sync block error rates

| Configuration | BLERout | BLERin |
| --- | --- | --- |
| 0 | 10% | 2% |

UE shall be able to monitor up to NRLM RLM-RS resources of the same or different types in each corresponding carrier frequency range, depending on a maximum number  of SSBs per half frame according to TS 38.213 [3], where NRLM is specified in Table 17.5.1.0.1-6 according TS 38.213 [3], and meet the requirements as specified in 38.133 [6] clause 8.1B. UE is not required to meet the requirements in 38.133 [6] clause 8.1B if RLM-RS is not configured and no TCI state for PDCCH is activated.

Table 17.5.1.0.1-2: PDCCH transmission parameters for out-of-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16; for RedCap UE with 1Rx branch. 8; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 48; for RedCap UE with 1Rx branch. 24; for RedCap UE with 2Rx branches. |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note: SCS = 60KHz is not applicable for FR1. |  |

Table 17.5.1.0.1-3: PDCCH transmission parameters for in-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8; for RedCap UE with 1Rx branch. 4; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 48; for RedCap UE with 1Rx branch. 24; for RedCap UE with 2Rx branches. |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note: SCS = 60KHz is not applicable for FR1. |  |

Table 17.5.1.0.1-4: PDCCH transmission parameters for out-of-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16; for RedCap UE with 1Rx branch. 8; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 4dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note: SCS = 60KHz is not applicable for FR1. |  |

Table 17.5.1.0.1-5: PDCCH transmission parameters for in-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8; for RedCap UE with 1Rx branch. 4; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| Note: SCS = 60KHz is not applicable for FR1. |  |

Table 17.5.1.0.1-6: Maximum number of RLM-RS resources XRLM-RS

| Carrier frequency range of PCell | Lmax | Maximum number of RLM-RS resources, NRLM |
| --- | --- | --- |
| FR1, ≤ 3 GHzNote | 4 | 2 |
| FR1, > 3 GHzNote | 8 | 4 |
| FR2 | 64 | 8 |
| NOTE: For unpaired spectrum operation with Case C - 30 kHz SCS, 3GHz is replaced by 1.88GHz, as specified in clause 4.1 in TS 38.213 [3]. |  |  |

When the UE transitions between RLM CD-SSB resource and RLM NCD-SSB resource during one evaluation period, the UE is allowed to restart the ongoing evaluation using the second SSB type after BWP switching. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each RLM-RS resource.

The requirements in 38.133 [6] clause 8.1.4 shall also apply except the scenarios described above.

The requirements in 38.133 [6] clause 8.1.5 shall apply.

When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, Layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers. A Layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [13].

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin,RedCap, layer 1 of the UE shall send an in-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [2].

The out-of-sync and in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 in TS 38.213 [8]. Two successive indications from layer 1 shall be separated by at least TIndication_interval,RedCap.

When DRX is not used for RedCap UEs, TIndication_interval,RedCap is max(10ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 38.133 [6] 8.1B.2 if the RLM-RS resource is SSB,  or TCSI-RS specified in 38.133 [6] clause 8.1B.3 if the RLM-RS resource is CSI-RS.

In case DRX is used for RedCap UEs, TIndication_interval,RedCap is max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms. Upon start of T310 timer as specified in TS 38.331 [2], the UE shall monitor the configured RLM-RS resources for recovery using the evaluation period and layer 1 indication interval corresponding to the no DRX mode until the expiry or stop of T310 timer.

The following scheduling restriction applies due to radio link monitoring on an FR2 serving PCell.

-	If the RLM-RS is CSI-RS which is type-D QCLed with active TCI state for PDCCH or PDSCH, and the CSI-RS is not in a CSI-RS resource set with repetition ON,

-	There are no scheduling restrictions due to radio link monitoring based on the CSI-RS.

-	Otherwise

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring.

For FR2, if following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB for RLM and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for RLM; and

For the SSB for RLM and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for RLM.

The normative reference for this requirement is TS 38.133 [6] clauses 8.1B.1, 8.1B.2, 8.1B.3, 8.1B.4, 8.1B.5, 8.1B.6 and 8.1B.7.

##### 17.5.1.0.2	Minimum conformance requirements for SSB-based RLM

The requirements in this clause apply for each SSB based RLM-RS resource configured for PCell, provided that the SSB configured for RLM is actually transmitted within UE active DL BWP during the entire evaluation period specified in this clause.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB,RedCap [ms] period becomes worse than the threshold Qout_SSB,RedCap within TEvaluate_out_SSB,RedCap [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,RedCap [ms] period becomes better than the threshold Qin_SSB,RedCap within TEvaluate_in_SSB,RedCap [ms] evaluation period.

TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap are defined in Table 17.5.1.0.2-1 and for FR2 with scaling factor N=8 for 2Rx RedCap.

Table 17.5.1.0.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR2 for 2Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P N) TSSB) | Max(100, Ceil(5 P N) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 P N) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 P N) TDRX | Ceil(5 P N) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

For FR2,

-	P=11-TSSBTSMTCperiod, when RLM-RS resource is not overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when the RLM-RS resource is not overlapped with measurement gap and RLM-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P=11-TSSBMGRP - TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P=Psharing factor1-TSSBMGRP, when the RLM-RS is partially overlapped with measurement gap and the RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5 × TSMTCperiod

-	P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P=Psharing factor1-TSSBMRGP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 1, if the RLM-RS resource outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

where,

If the high layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

If the high layer in TS 38.331 [13] signalling of smtc2 is present, TSMTCperiod follows smtc2; otherwise TSMTCperiod follows smtc1.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

Longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

Longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

For FR2, when the SSB for RLM measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

The normative reference for this requirement is TS 38.133 [6] clauses 8.1B.2.

##### 17.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM

The requirements in this clause apply for each CSI-RS based RLM-RS resource configured for PCell, provided that the CSI-RS configured for RLM is actually transmitted within UE active DL BWP during the entire evaluation period specified in this clause. UE is not expected to perform radio link monitoring measurements on the CSI-RS configured as RLM-RS if the CSI-RS is not in the active TCI state of any CORESET configured in the UE active BWP.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS,RedCap ms period becomes worse than the threshold Qout_CSI-RS,RedCap within TEvaluate_out_CSI-RS,RedCap ms evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS,RedCap ms period becomes better than the threshold Qin_CSI-RS,RedCap within TEvaluate_in_CSI-RS,RedCap ms evaluation period.

TEvaluate_out_CSI-RS,RedCap and TEvaluate_in_CSI-RS,RedCap are defined in Table 17.5.1.0.3-1 for FR2 with scaling factor N=1 for 2 Rx RedCap.

Table 17.5.1.0.3-1: Evaluation period TEvaluate_out_CSI-RS,RedCap and TEvaluate_in_CSI-RS,RedCap for FR2 for 2Rx RedCap UE

| Configuration | TEvaluate_out_CSI-RS,RedCap (ms) | TEvaluate_in_CSI-RS,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout,RedCap×P×N)×TCSI-RS) | Max(100, Ceil(Min,RedCap×P×N) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout,RedCap×P×N)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min,RedCap×P×N)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout,RedCap×P×N) × TDRX | Ceil(Min,RedCap×P×N) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

For FR2,

-	P=1, when the RLM-RS resource is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSMGRP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is not overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the RLM-RS resource is not overlapped with measurement gap and RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSMGRP - TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSMGRP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P=Psharing factor1-TCSI-RSMGRP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 1, if the RLM-RS resource outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

where,

If the high layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

If the high layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Note:	The overlap between CSI-RS for RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.

The values of Mout,RedCap and Min,RedCap used in Table 17.5.1.0.3-1 for UE with 2Rx are defined as:

-	Mout,RedCap = 20 and Min,RedCap = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density  set to 3 and over the bandwidth ≥ 24 PRBs.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

Longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

Longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The UE is required to be capable of measuring CSI-RS for RLM without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following clauses.

For FR2, when the CSI-RS for RLM is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for RLM in the PRBs that overlap with an SSB.

For FR2, when the CSI-RS for RLM is in the same OFDM symbol as SSB for RLM, BFD, or L1-RSRP measurement, or in the same symbol as SSB for CBD when beam failure is detected, UE is required to measure one of but not both CSI-RS for RLM and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

For FR2, when the CSI-RS for RLM measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	In the following cases, UE is required to measure one of but not both CSI-RS for RLM and the other CSI-RS. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

-	The CSI-RS for RLM or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for RLM without any restriction.

The normative reference for this requirement is TS 38.133 [6] clauses 8.1B.3.

#### 17.5.1.1	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

Editor's Note: This test case has been completed for the following configurations:

-	Test frequency f ≤ 40.8 GHz

-	UE PC3

-	Normal conditions

The test is incomplete for UE power classes other than PC3 (e.g. PC7)

The test is incomplete for test frequencies > 40.8 GHz

The test is incomplete for extreme conditions

17.5.1.1.1	Test purpose

To verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR2 radio link monitoring requirements in 38.133[6] clause 8.1B.

17.5.1.1.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards.

17.5.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.1.1.

17.5.1.1.4	Test description

Same as the test description given in clause 7.5.1.1.4.

17.5.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.1.1.4.1 with following exceptions:

-	Table 7.5.1.1.4.1-1 is replaced by Table 17.5.1.1.4.1-1.

-	Table 7.5.1.1.4.1-2 is replaced by Table 17.5.1.1.4.1-2.

-	Table 7.5.1.1.4.1-3 is replaced by Table 17.5.1.1.4.1-3.

Table 17.5.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.1.1-1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table 17.5.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.1.1.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 |  | 100: NRB,c = 66 |
| Data RBs allocated |  | Config 1 |  | 24 |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.3.4 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration |  | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |
| PRACH Configuration |  | Config 1 |  | PRACH.4 FR2 |
| SSB index assigned as RLM RS |  | Config 1 |  | 0,1 |
| OCNG parameters |  |  |  | OP.5 |
| CP length |  |  |  | Normal |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI reporting |  | Config 1 |  | CSI-RS.3.1 TDD |
| reportConfigType |  |  |  | periodic |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  |  | slot | 40 |
| CSI reporting offset |  |  | slot | 4 |
| TCI states for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| CSI-RS for tracking |  | Config 1 |  | TRS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 9.68 |
| T3 |  |  | s | 9.68 |
| D1 |  |  | s | 9.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

17.5.1.1.4.2	Test Procedure

Same as the test procedure given in clause 7.5.1.1.4.2.

17.5.1.1.4.3	Message Contents

Same as the message contents given in clause 7.5.1.1.4.3.

Message contents are according to TS 38.508-1 [14] clause 7.3.

17.5.1.1.5	Test Requirement

Same as the test requirements given in clause 7.5.1.1.5 with following exceptions:

-	Table 7.5.1.1.5-1 is replaced by Table 17.5.1.1.5-1.

Table 17.5.1.1.5-1: Cell specific test parameters

| Parameter |  | Unit | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  |  | Setup 3 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| Assumption for UE beams Note 5 |  |  | Rough |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  | Not sent |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| ssb-Index 0 SNR | Config 1 | dB | 4.1Note 6,7 | -3.9Note 6,7 | -15 |  |  |  |
| ssb-Index 1 SNR | Config 1 |  | Not sent |  |  | 4.1Note 6,7 | -15 | -15 |
|  | Config 1 | dBm/ 15kHz | -94.8 Note 7 |  |  | -94.8 Note 7 |  |  |
| Time multiplexing of the downlink transmissions from each AoA |  |  | Defined in Figure 7.5.1.1.4-2 |  |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  | TDL-A 30ns 75Hz |  |  |
| Note 1: OCNG shall be used such a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 6: This value allows up to 1dB degradation from applied SNR to UE baseband Note 7: Including test tolerance given in Annex F |  |  |  |  |  |  |  |  |

#### 17.5.1.2	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

Editor's Note: This test case has been completed for the following configurations:

-	Test frequency f ≤ 40.8 GHz

-	UE PC3

-	Normal conditions

The test is incomplete for UE power classes other than PC3 (e.g. PC7)

The test is incomplete for test frequencies > 40.8 GHz

The test is incomplete for extreme conditions

17.5.1.2.1	Test purpose

To verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR2 radio link monitoring requirements in 38.133[6] clause 8.1B.

17.5.1.2.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards.

17.5.1.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.1.2.

17.5.1.2.4	Test description

Same as the test description given in clause 7.5.1.2.4.

17.5.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.1.2.4.1 with following exceptions:

-	Table 7.5.1.2.4.1-1 is replaced by Table 17.5.1.2.4.1-1.

-	Table 7.5.1.2.4.1-2 is replaced by Table 17.5.1.2.4.1-2.

-	Table 7.5.1.2.4.1-3 is replaced by Table 17.5.1.2.4.1-3.

Table 17.5.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.1.2-1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table 17.5.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.1.2.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 |  | 100: NRB,c = 66 |
| Data RBs allocated |  | Config 1 |  | 24 |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.3.1 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration |  | Config 1 |  | SMTC.3 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |
| PRACH Configuration |  | Config 1 |  | PRACH.4 FR2 |
| SSB index assigned as RLM RS |  | Config 1 |  | 0,1 |
| OCNG parameters |  |  |  | OP.5 |
| CP length |  |  |  | Normal |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 4000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI reporting |  | Config 1 |  | CSI-RS.3.1 TDD |
| reportConfigType |  |  |  | periodic |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  |  | slot | 40 |
| CSI reporting offset |  |  | slot | 4 |
| TCI states for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| CSI-RS for tracking |  | Config 1 |  | TRS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 1.88 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 3.84 |
| D1 |  |  | s | 3.8 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

17.5.1.2.4.2	Test Procedure

Same as the test procedure given in clause 7.5.1.2.4.2.

17.5.1.2.4.3	Message Contents

Same as the message contents given in clause 7.5.1.2.4.3.

Message contents are according to TS 38.508-1 [14] clause 7.3.

17.5.1.2.5	Test Requirement

Same as the test requirements given in clause 7.5.1.2.5 with following exceptions:

-	Table 7.5.1.2.5-1 is replaced by Table 17.5.1.2.5-1.

Table 17.5.1.2.5-1: Cell specific test parameters

| Parameter |  | Unit | Test 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 3 defined in A.9 |  |  |  |  |  |  |  |  |  |
|  |  |  | AoA1 |  |  |  |  | AoA2 |  |  |  |  |
| Assumption for UE beams Note 5 |  |  | Rough |  |  |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB | 0 |  |  |  |  | Not sent |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| ssb-Index 0 SNR | Config 1 | dB | 4.1 Note 6,7 | -3.9 Note 6,7 | -15 | -15 | -15 | Not sent |  |  |  |  |
| ssb-Index 1 SNR | Config 1 |  | Not sent |  |  |  |  | 4.1 Note 6,7 | -15 | -15 | -4.5 | 4.1 Note 6,7 |
|  | Config 1 | dBm/ 15kHz | -94.8 Note 7 |  |  |  |  | -94.8 Note 7 |  |  |  |  |
| Time multiplexing of the downlink transmissions from each AoA |  |  | Defined in Figure 7.5.1.2.1-2 |  |  |  |  |  |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 6: This value allows up to 1dB degradation from applied SNR to UE baseband Note 7: Including test tolerance given in Annex F. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 17.5.1.4	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode

Editor's Note: This test case has been completed for the following configurations:

-	Test frequency f ≤ 40.8 GHz

-	UE PC3

-	Normal conditions

The test is incomplete for UE power classes other than PC3 (e.g. PC7)

The test is incomplete for test frequencies > 40.8 GHz

The test is incomplete for extreme conditions

17.5.1.4.1	Test purpose

To verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR2 radio link monitoring requirements in 38.133[6] clause 8.1B.

17.5.1.4.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards and supporting long DRX cycle.

17.5.1.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.1.4.

17.5.1.4.4	Test description

Same as the test description given in clause 7.5.1.4.4.

17.5.1.4.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.1.4.4.1 with following exceptions:

-	Table 7.5.1.4.4.1-1 is replaced by Table 17.5.1.4.4.1-1.

-	Table 7.5.1.4.4.1-2 is replaced by Table 17.5.1.4.4.1-2.

-	Table 7.5.1.4.4.1-3 is replaced by Table 17.5.1.4.4.1-3.

Table 17.5.1.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.1.4-1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table 17.5.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.1.4.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.1.4.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 |  | 100: NRB,c = 66 |
| Data RBs allocated |  | Config 1 |  | 66 |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.3.1 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration |  | Config 1 |  | SMTC.3 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |
| PRACH Configuration |  | Config 1 |  | PRACH.4 FR2 |
| SSB index assigned as RLM RS |  | Config 1 |  | 0,1 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.11 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 4000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI reporting |  | Config 1 |  | CSI-RS.3.1 TDD |
| reportConfigType |  |  |  | periodic |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  |  | slot | 40 |
| CSI reporting offset |  |  | slot | 4 |
| TCI states for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| CSI-RS for tracking |  | Config 1 |  | TRS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 2.8 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 3.88 |
| D1 |  |  | s | 3.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

17.5.1.4.4.2	Test Procedure

Same as the test procedure given in clause 7.5.1.4.4.2.

17.5.1.4.4.3	Message Contents

Same as the message contents given in clause 7.5.1.4.4.3.

Message contents are according to TS 38.508-1 [14] clause 7.3.

17.5.1.4.5	Test Requirement

Same as the test requirements given in clause 7.5.1.4.5 with following exceptions:

-	Table 7.5.1.4.5-1 is replaced by Table 17.5.1.4.5-1.

Table 17.5.1.4.5-1: Cell specific test parameters

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 |  |  |  |  |
| Assumption for UE beams Note 5 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| ssb-Index 0 SNR | Config 1 | dB | 3.3 Note 6,7 | -4.7 Note 6,7 | -15.4 Note 7 | -4.9 Note 7 | 3.3 Note 6,7 |
| ssb-Index 1 SNR | Config 1 |  | 3.3 Note 6,7 | -15.4 Note 7 | -15.4 Note 7 | -15.4 Note 7 | -15.4 Note 7 |
|  | Config 1 | dBm/15KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG.3 Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 6: This value allows up to 1dB degradation from applied SNR to UE baseband. Note 7: Including the test tolerance given in Annex F. |  |  |  |  |  |  |  |

#### 17.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

Editor's Note: This test case is incomplete due to the following:

-	TT analysis is still missing

-	Procedure to ensure signal balancing in 2AoA setups is still FFS and is tracked via AP#97.25 at RAN5

-	Signal levels and propagation conditions need further evaluation at RAN4/5

17.5.1.5.1	Test purpose

To verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR2 PCell CSI-RS Out-of-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1B.3.

17.5.1.5.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting FR2 and CSI-RS based RLM.

17.5.1.5.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.1.5.

17.5.1.5.4	Test description

Same as the test description given in clause 7.5.1.5.4.

17.5.1.5.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.1.5.4.1 with following exceptions:

-	Table 7.5.1.5.4.1-1 is replaced by Table 17.5.1.5.4.1-1,

-	Table 7.5.1.5.4.1-2 is replaced by Table 17.5.1.5.4.1-2,

-	Table 7.5.1.5.4.1-3 is replaced by Table 17.5.1.5.4.1-3

Table 17.5.1.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.1.5-1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table 17.5.1.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.15-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.1.5.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.1.5.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode |  |  | TDD |
| BWchannel |  |  | 100: NRB,c = 66 |
| Data RBs allocated |  |  | 24 |
| BWoccupied |  |  | 24 |
| TDD Configuration |  |  | TDDConf.3.1 |
| DL initial BWP configuration |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | DLBWP.1.4 |
| UL initial BWP configuration |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | ULBWP.1.4 |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  |  | CCR.3.4 TDD CCR.3.6 TDD |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |
| CSI-RS for RLM |  |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.5 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting |  |  | CSI-RS.3.1 TDD |
| reportConfigType |  |  | periodic |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  | slot | 40 |
| CSI reporting offset |  | slot | 4 |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.35 |
| T3 |  | s | 0.35 |
| D1 |  | s | 0.31 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

17.5.1.5.4.2	Test Procedure

Same as the test procedure given in clause 7.5.1.5.4.2.

17.5.1.5.4.3	Message Contents

Same as the message contents given in clause 7.5.1.5.4.3.

17.5.1.5.5	Test Requirement

Same as the test requirements given in clause 7.5.1.5.5 with following exceptions:

-	Table 7.5.1.5.5-1 is replaced by Table 17.5.1.5.5-1,

-	Table 7.5.1.5.5-2 is replaced by Table 17.5.1.5.5-2.

Table 17.5.1.5.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  | Setup 3 defined in A.9.3 |  |  |  |  |  |
|  |  | AoA1 |  |  | AoA2 |  |  |
| Assumption for UE beams Note 10 |  | Rough |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  | Not sent |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |  |
| SNR on RLM-RS1 | dB | 2+TT Note 11 | -6+TT Note 11 | -15+TT | Not sent |  |  |
| SNR on RLM-RS2 | dB | Not sent |  |  | 2+TT Note 11 | -15+TT | -15+TT |
|  | dBm/ 15kHz | -92.1+TT |  |  |  |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.1.5.4.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 10: Information about types of UE beam is given in 38.133 [6] section B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |

#### 17.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode

Editor's Note: This test case is incomplete due to the following:

-	TT analysis is still missing

-	Procedure to ensure signal balancing in 2AoA setups is still FFS and is tracked via AP#97.25 at RAN5

-	Signal levels and propagation conditions need further evaluation at RAN4/5

17.5.1.6.1	Test purpose

To verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR2 PCell CSI-RS In-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1B.3.

17.5.1.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting FR2 and CSI-RS based RLM.

17.5.1.6.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.5.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.1.6.

17.5.1.6.4	Test description

Same as the test description given in clause 7.5.1.6.4.

17.5.1.6.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.1.6.4.1 with following exceptions:

-	Table 7.5.1.6.4.1-1 is replaced by Table 17.5.1.6.4.1-1,

-	Table 7.5.1.6.4.1-2 is replaced by Table 17.5.1.6.4.1-2,

-	Table 7.5.1.6.4.1-3 is replaced by Table 17.5.1.6.4.1-3.

Table 17.5.1.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.1.6-1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table 17.5.1.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.15-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.1.6.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.1.6.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode |  |  | TDD |
| BWchannel |  |  | 100: NRB,c = 66 |
| Data RBs allocated |  |  | 24 |
| BWoccupied |  |  | 24 |
| TDD Configuration |  |  | TDDConf.3.1 |
| DL initial BWP configuration |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | DLBWP.1.4 |
| UL initial BWP configuration |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | ULBWP.1.4 |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  |  | CCR.3.1 TDD CCR.3.3 TDD |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | 120 KHz |
| CSI-RS for RLM |  |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.5 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 1000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
| reportConfigType |  |  | periodic |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  | slot | 40 |
| CSI reporting offset |  | slot | 4 |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 0.24 |
| T4 |  | s | 0.2 |
| T5 |  | s | 0.88 |
| D1 |  | s | 0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

17.5.1.6.4.2	Test Procedure

Same as the test procedure given in clause 7.5.1.6.4.2.

17.5.1.6.4.3	Message Contents

Same as the message contents given in clause 7.5.1.6.4.3.

17.5.1.6.5	Test Requirement

Same as the test requirements given in clause 17.5.1.6.5 with following exceptions:

-	Table 7.5.1.6.5-1 is replaced by Table 17.5.1.6.5-1.

Table 17.5.1.6.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | Setup 3 defined in A.9.3 |  |  |  |  |  |  |  |  |  |
|  |  | AoA1 |  |  |  |  | AoA2 |  |  |  |  |
| Assumption for UE beams Note 10 |  | Rough |  |  |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |  |  | Not sent |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |  |  |  |  |  |
| SNR on RLM-RS1 | dB | 2 + TT Note 11 | -6 + TT Note 11 | -15 + TT | -15 + TT | -15 + TT |  |  |  |  |  |
| SNR on RLM-RS2 | dB | Not sent |  |  |  |  | 2 + TT Note 11 | -15 + TT | -15 + TT | -4.5 + TT | 2 + TT Note 11 |
|  | dBm/ 15KHz | -92.1+TT |  |  |  |  |  |  |  |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting is assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 17.5.1.6.4.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 10: Information about types of UE beam is given in 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |  |  |  |  |

#### 17.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode

Editor's Note: This test case is incomplete due to the following:

-	TT analysis is still missing

17.5.1.7.1	Test purpose

To verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR2 PCell CSI-RS Out-of-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1B.3.

17.5.1.7.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting FR2, CSI-RS based RLM and long DRX cycle.

17.5.1.7.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.1.7.

17.5.1.7.4	Test description

Same as the test description given in clause 7.5.1.7.4.

17.5.1.7.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.1.7.4.1 with following exceptions:

-	Table 7.5.1.7.4.1-1 is replaced by Table 17.5.1.7.4.1-1,

-	Table 7.5.1.7.4.1-2 is replaced by Table 17.5.1.7.4.1-2,

-	Table 7.5.1.7.4.1-3 is replaced by Table 17.5.1.7.4.1-3

Table 17.5.1.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.1.7-1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table 17.5.1.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.15-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.1.7.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.1.7.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode |  |  | TDD |
| BWchannel |  |  | 100: NRB,c = 66 |
| Data RBs allocated |  |  | 24 |
| BWoccupied |  |  | 24 |
| TDD Configuration |  |  | TDDConf.3.1 |
| DL initial BWP configuration |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  |  | CCR.3.4 TDD CCR.3.6 TDD |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |
| CSI-RS for RLM |  |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.5 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting |  |  | CSI-RS.3.1 TDD |
| reportConfigType |  |  | periodic |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  | slot | 40 |
| CSI reporting offset |  | slot | 4 |
| T1 |  | s | 0.2 |
| T2 |  | s | 1.28 |
| T3 |  | s | 1.28 |
| D1 |  | s | 1.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

17.5.1.7.4.2	Test Procedure

Same as the test procedure given in clause 7.5.1.7.4.2.

17.5.1.7.4.3	Message Contents

Same as the message contents given in clause 7.5.1.7.4.3.

17.5.1.7.5	Test Requirement

Same as the test requirements given in clause 7.5.1.7.5 with following exceptions:

-	Table 7.5.1.7.5-1 is replaced by Table 17.5.1.7.5-1,

-	Table 7.5.1.7.5-2 is replaced by Table 17.5.1.7.5-2.

Table 17.5.1.7.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| AoA setup | dB | Setup 1 defined in A.3.15 |  |  |
| Assumption for UE beams Note 10 |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |
| SNR on RLM-RS1 | dB | 2Note 11+TT | -6Note 11+TT | -15+TT |
| SNR on RLM-RS2 | dB | 2Note 11+TT | -14+TT | -15+TT |
|  | dBm/15KHz | -104.7+TT |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.1.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 10: Information about types of UE beam is given in TS 38.133 [6] B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |

#### 17.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode

Editor's Note: This test case is incomplete due to the following:

-	TT analysis is still missing

17.5.1.8.1	Test purpose

To verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR2 PCell CSI-RS In-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1B.3.

17.5.1.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting FR2, CSI-RS based RLM and long DRX cycle.

17.5.1.8.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.5.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.1.8.

17.5.1.8.4	Test description

Same as the test description given in clause 7.5.1.8.4.

17.5.1.8.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.1.8.4.1 with following exceptions:

-	Table 7.5.1.8.4.1-1 is replaced by Table 17.5.1.8.4.1-1,

-	Table 7.5.1.8.4.1-2 is replaced by Table 17.5.1.8.4.1-2,

-	Table 7.5.1.8.4.1-3 is replaced by Table 17.5.1.8.4.1-3.

Table 17.5.1.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.1.8-1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table 17.5.1.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.15-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.1.8.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.1.8.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode |  |  | TDD |
| TDD Configuration |  |  | TDDConf.3.1 |
| DL initial BWP configuration |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  |  | CCR.3.1 TDD CCR.3.3 TDD |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |
| CSI-RS for RLM |  |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
| reportConfigType |  |  | periodic |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  | slot | 40 |
| CSI reporting offset |  | slot | 4 |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 1.64 |
| T4 |  | s | 0.2 |
| T5 |  | s | 1.88 |
| D1 |  | s | 1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

17.5.1.8.4.2	Test Procedure

Same as the test procedure given in clause 7.5.1.8.4.2.

17.5.1.8.4.3	Message Contents

Same as the message contents given in clause 7.5.1.8.4.3.

17.5.1.8.5	Test Requirement

Same as the test requirements given in clause 17.5.1.8.5 with following exceptions:

-	Table 7.5.1.8.5-1 is replaced by Table 17.5.1.8.5-1.

Table 17.5.1.8.5-1: Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup | dB | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 10 |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
| SNR on RLM-RS1 | dB | 2Note 11 | -6Note 11 | -15 | -4.5 | 2Note 11 |
| SNR on RLM-RS2 | dB | 2Note 11 | -14 | -15 | -15 | -14 |
|  | dBm/ 15KHz | -104.7 |  |  |  |  |
| Propagation condition |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 7.5.1.8.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band.. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |

#### 17.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	MU and TT analysis is still missing.

-	Message content is TBD

17.5.1.9.1	Test purpose

To verify that the NR UE correctly follows the RLM scheduling restrictions requirements defined in 38.133 [6] clause 8.1B.7. This test verifies that the UE correctly receive the PDCCH scheduled on the symbols right before the RLM SSB symbols without overlap so that it sends ACK/NACK correctly.

17.5.1.9.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards supporting pdcch-MonitoringAnyOccasions or pdcch-MonitoringAnyOccasionsWithSpanGap.

17.5.1.9.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 17.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.1.9.

17.5.1.9.4	Test description

Same as the test description given in clause 7.5.1.9.4.

17.5.1.9.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.1.9.4.1 with following exceptions:

-	Table 7.5.1.9.4.1-1 is replaced by Table 17.5.1.9.4.1-1.

-	Table 7.5.1.9.4.1-2 is replaced by Table 17.5.1.9.4.1-2.

-	Table 7.5.1.9.4.1-3 is replaced by Table 17.5.1.9.4.1-3.

Table 17.5.1.9.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.1.9-1 | 120 kHz SSB SCS, 120 kHz RMC SCS, 100 MHz bandwidth, TDD duplex mode |

Table 17.5.1.9.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.1.9.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.1.9.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1 |  |
| SSB configuration |  | SSB.1 FR2 |  |
| SMTC configuration |  | SMTC.1 |  |
| DRX cycle length | s | OFF |  |
| T1 | s | 5 | During T1 the UE is required to correctly transmit ACK/NACK |

17.5.1.9.4.2	Test Procedure

Same as the test procedure given in clause 7.5.1.9.4.2.

17.5.1.9.4.3	Message Contents

Same as the message contents given in clause 7.5.1.9.4.3.

17.5.1.9.5	Test Requirement

Same as the test requirements given in clause 7.5.1.9.5 with following exceptions:

-	Table 7.5.1.9.5-1 is replaced by Table 17.5.1.9.5-1.

Table 17.5.1.9.5-1: Cell specific test parameters

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
| AoA setup |  | Setup 3 |  |
|  |  | AoA1 | AoA2 |
| Assumption for UE beams Note 1 |  | Rough | Rough |
| TDD configuration |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 24 |  |
| PDSCH Reference measurement channel |  | SR.3.2 TDD | Not sent |
| RMSI CORESET RMC configuration |  | CR.3.1 TDD | Not sent |
| Dedicated CORESET RMC configuration |  | CCR.3.2 TDD | Not sent |
| TRS configuration |  | TRS.2.1 TDD | TRS.2.2 TDD |
| PDCCH/PDSCH TCI state |  | TCI.State.2 | N/A |
| OCNG Pattern |  | OP.5 | Not sent |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |
| RLM-RS |  | SSB with index 0 | SSB with index 1 |
|  | dBm/15kHz | -92.1 | -92.1 |
| Note2 | dBm/SCS | -83.1 | -83.1 |
|  | dB | 2 | 2 |
| BB Note 4 | dB | 1 | 1 |
| SSB_RP Note3 | dBm/SCS | -81.1 | -81.1 |
| Io | dBm/95.04 MHz | -54.35 | -54.35 |
| Time multiplexing of the downlink transmissions from each AoA |  | Defined in Figure A.7.5.1.9.1-1 |  |
| Propagation Condition |  | AWGN | AWGN |
| Note 1: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |

### 17.5.2	Beam Failure Detection and Link recovery procedures for RedCap

#### 17.5.2.0	Minimum conformance requirements

##### 17.5.2.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures

The minimum requirements given in clause 5.5.5.0.1 applies.

The normative reference for this requirement is TS 38.133 [6] clause 8.5B.1, 8.5B.2, 8.5B.4, 8.5B.5 and 8.5B.9

##### 17.5.2.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures

The minimum requirements given in clause 5.5.5.0.2 applies.

The normative reference for this requirement is TS 38.133 [6] clause 8.5B.1, 8.5B.3, 8.5B.4, 8.5B.6, 8.5B.9

##### 17.5.2.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection

The minimum requirements given in clause 5.5.5.0.3 applies.

The normative reference for this requirement is TS 38.133 [6] clause 8.5B.1, 8.5B.2, 8.5B.4, 8.5B.5, 8.5B.7, 8.5B.8 and 8.5B.9

#### 17.5.2.1 – 17.5.2.2

#### 17.5.2.3	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode

Editor's Note:

This test case is complete for the following configurations:

-	UE PC3

-	Test frequencies f ≤ 40.8 GHz;

This test case is incomplete for UE power classes other than PC3;

This test case is incomplete for test frequencies f > 40.8 GHz.17.5.2.3.1	Test purpose

To verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5B.

17.5.2.3.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards supporting CSI-RS-based RLM and link recovery.

17.5.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.2.3.

17.5.2.3.4	Test description

Same as the test description given in clause 7.5.5.3.4.

17.5.2.3.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.5.3.4.1 with following exceptions:

-	Table 7.5.5.3.4.1-1 is replaced by Table 17.5.2.3.4.1-1.

-	Table 7.5.5.3.4.1-2 is replaced by Table 17.5.2.3.4.1-2.

-	Table 7.5.5.3.4.1-3 is replaced by Table 17.5.2.3.4.1-3.

Table 17.5.2.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.2.3-1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table 17.5.2.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.2.3.4.1-1. |  |  |
| Propagation conditions | Multi-path fading |  | As specified in Annex C.2.3. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.2.3.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode |  |  | TDD |  |
| TDD Configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |
| DL initial BWP configuration |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |  |
| PDSCH Reference Channel |  |  | SR.3.2 TDD |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | CCR.3.1 TDD |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.0 |  |
| CSI-RS for tracking |  |  | TRS.2.1 TDD |  |
| SSB Configuration |  |  | SSB.1 FR2 |  |
| SMTC Configuration |  |  | SMTC.3 |  |
| PRACH Configuration |  |  | PRACH.4 FR2 | A.3.8.3.4 |
| DRX configuration |  |  | OFF |  |
| CSI-RS configuration for BFD/CBD/RLM |  |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS index assigned as BFD RS (q0) |  |  | 0 |  |
| CSI-RS index assigned as CBD RS (q1) |  |  | 1 |  |
| CSI-RS index assigned as RLM RS |  |  | 0,1 |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| Gap pattern ID |  |  | N/A |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | Value 0 is applied. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | dBm/ SCS | -109 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  |  | pbfd4 | see TS 38.321 [7], clause 5.17 |
| CSI-RS configuration for CSI reporting |  |  | CSI-RS.3.1 TDD | A.3.14.2 |
| reportConfigType |  |  | periodic |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | slot | 40 |  |
| CSI reporting offset |  | slot | 4 |  |
| T310 |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  | s | 1.17 |  |
| T3 |  | s | 0.9 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

17.5.2.3.4.2	Test procedure

Same as the test procedure given in clause 7.5.5.3.4.2.

17.5.2.3.4.3	Message contents

Same as the message contents given in clause 7.5.5.3.4.3.

17.5.2.3.5	Test requirement

Same as the test requirements given in clause 7.5.5.3.5 with following exceptions:

-	Table 7.5.5.3.5-1 is replaced by Table 17.5.2.3.5-1.

Table 17.5.2.3.5-1: NR Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 9 |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | dB | 13.7Note | 5.7Note 10 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | dB | 0.2 | 0.2 | 20 | 20 | 20 |
| CSI-RS_RP of set q1 | dBm/SCS | -104.5 | -104.5 | -84.7 | -84.7 | -84.7 |
|  | dBm/ 120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.3.4-1. Note 8: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 9: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 10: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |

#### 17.5.2.4	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode

Editor's Note:

This test case is complete for the following configurations:

-	UE PC3

-	Test frequencies f ≤ 40.8 GHz;

This test case is incomplete for UE power classes other than PC3;

This test case is incomplete for test frequencies f > 40.8 GHz.17.5.2.4.1	Test purpose

To verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5B.

17.5.2.4.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards supporting CSI-RS-based RLM, long DRX cycle and link recovery.

17.5.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.2.4.

17.5.2.4.4	Test description

Same as the test description given in clause 7.5.5.4.4.

17.5.2.4.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.5.4.4.1 with following exceptions:

-	Table 7.5.5.4.4.1-1 is replaced by Table 17.5.2.4.4.1-1.

-	Table 7.5.5.4.4.1-2 is replaced by Table 17.5.2.4.4.1-2.

-	Table 7.5.5.4.4.1-3 is replaced by Table 17.5.2.4.4.1-3.

Table 17.5.2.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.2.4-1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table 17.5.2.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.2.4.4.1-1. |  |  |
| Propagation conditions | Multi-path fading |  | As specified in Annex C.2.3. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.2.4.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode |  |  | TDD |  |
| TDD Configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |
| DL initial BWP configuration |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |  |
| PDSCH Reference Channel |  |  | SR.3.2 TDD |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | CCR.3.1 TDD |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.0 |  |
| CSI-RS for tracking |  |  | TRS.2.1 TDD |  |
| SSB Configuration |  |  | SSB.1 FR2 |  |
| SMTC Configuration |  |  | SMTC.3 |  |
| PRACH Configuration |  |  | PRACH.4 FR2 | A.3.8.3.4 |
| DRX configuration |  |  | DRX.3 |  |
| CSI-RS configuration for BFD/CBD/RLM |  |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS index assigned as BFD RS (q0) |  |  | 0 |  |
| CSI-RS index assigned as CBD RS (q1) |  |  | 1 |  |
| CSI-RS index assigned as RLM RS |  |  | 0,1 |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| Gap pattern ID |  |  | N/A |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | Value 0 is applied. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | dBm/ SCS | -109 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  |  | pbfd4 | see TS 38.321 [7], clause 5.17 |
| CSI-RS configuration for CSI reporting |  |  | CSI-RS.3.1 TDD | A.3.14.2 |
| reportConfigType |  |  | periodic |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | slot | 40 |  |
| CSI reporting offset |  | slot | 4 |  |
| T310 |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  | s | 5.43 |  |
| T3 |  | s | 5.16 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

17.5.2.4.4.2	Test procedure

Same as the test procedure given in clause 7.5.5.4.4.2.

17.5.2.4.4.3	Message contents

Same as the message contents given in clause 7.5.5.4.4.3.

17.5.2.4.5	Test requirement

Same as the test requirements given in clause 7.5.5.4.5 with following exceptions:

-	Table 7.5.5.4.5-1 is replaced by Table 17.5.2.4.5-1.

Table 17.5.2.4.5-1: NR Cell specific test parameters

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 9 |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | dB | 13.7Note 10 | 5.7Note 10 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | dB | 0.2 | 0.2 | 20 | 20 | 20 |
| CSI-RS_RP of set q1 | dBm/SCS | -104.5 | -104.5 | -84.75 | -84.7 | -84.7 |
|  | dBm/ 120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.4.4-1. Note 8: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 9: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 10: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |

#### 17.5.2.5	NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

Editor's Note:

This test case is complete for the following configurations:

-	UE PC3

-	Test frequencies f ≤ 40.8 GHz;

This test case is incomplete for UE power classes other than PC3;

This test case is incomplete for test frequencies f > 40.8 GHz.

17.5.2.5.1	Test purpose

To test scheduling availability restrictions when the UE is performing beam failure detection or when the UE is performing L1-RSRP measurement for candidate beam detection, when no DRX is used. This test will verify the scheduling availability restriction requirements in 38.133 [6] clause 8.5B.7 and 8.5B.8.

17.5.2.5.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards supporting link recovery.

17.5.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.5.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.2.5.

17.5.2.5.4	Test description

Same as the test description given in clause 7.5.5.5.4.

17.5.2.5.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.2.5.4.1 with following exceptions:

-	Table 7.5.5.5.4.1-1 is replaced by Table 17.5.2.5.4.1-1.

-	Table 7.5.5.5.4.1-2 is replaced by Table 17.5.2.5.4.1-2.

-	Table 7.5.5.5.4.1-3 is replaced by Table 17.5.2.5.4.1-3.

Table 17.5.2.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.5.2.5-1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 17.5.2.5-2 | TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |

Table 17.5.2.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.2.5.4.1-1. |  |  |
| Propagation conditions | Multi-path fading |  | As specified in Annex C.2.3. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.2.5.4.1-3: General test parameters

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD Configuration |  |  |  | TDDConf.3.1 | Table A.1.5-3 |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |
| DL initial BWP configuration |  |  |  | DLBWP.0.1 | Table A.8.1-1 |
| DL dedicated BWP configuration |  |  |  | DLBWP.1.1 | Table A.8.1-2 |
| UL initial BWP configuration |  |  |  | ULBWP.0.1 | Table A.8.2-1 |
| UL dedicated BWP configuration |  |  |  | ULBWP.1.1 | Table A.8.2-2 |
| PDSCH Reference Channel |  | Config 1 |  | SR.3.2 TDD | Table A.1.1.2-3 |
|  |  | Config 2 |  | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD | Table A.1.2.2-3 |
|  |  | Config 2 |  | CR.3.2 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.3.1 TDD | Table A.1.3.2-3 |
|  |  | Config 2 |  | CCR.3.7 TDD |  |
| OCNG parameters |  |  |  | OP.1 | Table A.2.1-1 |
| CP length |  |  |  | Normal |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.0 | Table A.10.2-1 |
| CSI-RS for tracking |  |  |  | TRS.2.1 TDD |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 | Table A.3.2-1 |
|  |  | Config 2 |  | SSB.2 FR2 |  |
| SMTC Configuration |  |  |  | SMTC.1 | Table A.4-1 |
| PRACH Configuration |  |  |  | PRACH.2 FR2 | Table A.7.2-1 |
| DRX configuration |  |  |  | OFF |  |
| SSB index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  |  | 1 |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| Gap pattern ID |  |  |  | N/A |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | Value 0 is applied. (TS 38.133 [6] Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1 | dBm/SCS | -109 | Threshold used for Qin_LR_SSB |
|  |  | Config 2 |  | -106 |  |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting |  |  |  | CSI-RS.3.1 TDD | Table A.1.4.2-3 |
| reportConfigType |  |  |  | periodic |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  |  | slot | 40 |  |
| CSI reporting offset |  |  | slot | 4 |  |
| T310 |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  |  | s | 2.6 |  |
| T3 |  |  | s | 1.64 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.01 |  |
| D1 |  |  | s | 0.97 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

17.5.2.5.4.2	Test procedure

Same as the test procedure given in clause 7.5.5.5.4.2.

17.5.2.5.4.3	Message contents

Same as the message contents given in clause 7.5.5.5.4.3.

17.5.2.5.5	Test requirement

Same as the test requirements given in clause 7.5.5.5.5 with following exceptions:

-	Table 7.5.5.5.5-1 is replaced by Table 17.5.2.5.5-1.

Table 17.5.2.5.5-1: NR Cell specific test parameters

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 9 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 |  | dB | 13.7 Note 10 | 5.7Note 10 | -12 | -12 | -12 |
| SNR_SSB of set q1 |  | dB | 0.2 | 0.2 | 20 | 20 | 20 |
| SSB_RP of set q1 | Config 1 | dBm/ SCS | -104.5 | -104.5 | -84.7 | -84.7 | -84.7 |
|  | Config 2 |  | -101.5 | -101.5 | -81.7 | -81.7 | -81.7 |
|  |  | dBm/ 120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.5.4-1. Note 8: The SNR values are specified for testing a UE which supports 2RX on at least one band. Note 9: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 10: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |

### 17.5.3

### 17.5.4	Active TCI state switch delay for RedCap

#### 17.5.4.0	Minimum conformance requirements

##### 17.5.4.0.1	Known conditions for TCI state

The minimum requirements given in clause 7.5.4.0.0 shall apply.

##### 17.5.4.0.2	Minimum conformance requirements for MAC-CE based active TCI state switch

The minimum requirements given in clause 7.5.4.0.1 shall apply.

##### 17.5.4.0.3	Minimum conformance requirements for RRC based active TCI state switch

The minimum requirements given in clause 7.5.4.0.2 shall apply.

#### 17.5.4.1	MAC-CE based active TCI state switch for RedCap

17.5.4.1.1	NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state

17.5.4.1.1.1	Test purpose

To verify the active TCI state switch delay requirement defined in TS 38.133 [6] clause 8.10B.3.

17.5.4.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting SA FR2, more than one TCI state configuration, and more than one simultaneously trackable TRS resource set.

17.5.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.5.4.0.1 and 17.5.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.4.1.1.

17.5.4.1.1.4	Test description

17.5.4.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.8.1.1.4.1 with following exceptions:

-	Table 7.5.8.1.1.4.1-1 is replaced by Table 17.5.4.1.1.4.1-1;

-	Table 7.5.8.1.1.4.1-2 is replaced by Table 17.5.4.1.1.4.1-2;

-	Table 7.5.8.1.1.4.1-3 is replaced by Table 17.5.4.1.1.4.1-3;

Table 17.5.4.1.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.5.4.1.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table 17.5.4.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15 and TS 38.508-1 [14] clause 4.3.1 and 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.4.1.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |

17.5.4.1.1.4.2	Test procedure

Same as the test procedure given in clause 7.5.8.1.1.4.2.

17.5.4.1.1.4.3	Message contents

Same as the message contents given in clause 7.5.8.1.1.4.3.

17.5.4.1.1.5	Test requirement

Same as the test requirements given in clause 7.5.8.1.1.5 with following exceptions:

-	Table 7.5.8.1.1.5-1 is replaced by Table 17.5.4.1.1.5-1;

-	Table 7.5.8.1.1.5-2 is replaced by Table 17.5.4.1.1.5-2;

Table 17.5.4.1.1.5-1: NR Cell specific test parameters

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 |
| Data RBs allocated |  | 24 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State 0 |  | TCI.State.0 |
| TCI State 1 |  | TCI.State.1 |
| TRS Configuration |  | TRS.2.1 TDD TRS.2.2 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table 17.5.4.1.1.5-2: OTA related test parameters

| Parameter | Unit | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |
|  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup 3 according to clause A.9 |  |  |  |
|  |  | AoA1 |  | AoA2 |  |
| Assumption for UE beams Note 4 |  | Rough |  |  |  |
| Ês | dBm/SCS | -80.9+TT | -80.9+TT | -Infinity | -80.9+TT |
| SSB_RP Note 1 | dBm/ SCS | -80.9+TT | -80.9+TT | -Infinity | -80.9+TT |
| BB Note 5 | dB | 8.3+TT | 8.3+TT | -Infinity | 8.3+TT |
| IoNote1 | dBm/ 95.04 MHz Note2 | -55.71+TT | -55.71+TT | - Infinity | -55.71+TT |
| Note 1: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 3: As observed with 0dBi gain antenna at the centre of the quiet zone. Note 4: Information about types of UE beam is given in 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |

#### 17.5.4.2	RRC based active TCI state switch for RedCap

##### 17.5.4.2.1	NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state

17.5.4.2.1.1	Test purpose

To verify the active TCI state switch delay requirement defined in TS 38.133 [6] clause 8.10B.5.

17.5.4.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting SA FR2, more than one TCI state configuration, and more than one simultaneously trackable TRS resource set.

17.5.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.5.4.0.1 and 17.5.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.17.5.4.2.1.

17.5.4.2.1.4	Test description

17.5.4.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.5.8.2.1.4.1 with following exceptions:

-	Table 7.5.8.2.1.4.1-1 is replaced by Table 17.5.4.2.1.4.1-1;

-	Table 7.5.8.2.1.4.1-2 is replaced by Table 17.5.4.2.1.4.1-2;

-	Table 7.5.8.2.1.4.1-3 is replaced by Table 17.5.4.2.1.4.1-3;

Table 17.5.4.2.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 17.5.4.2.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table 17.5.4.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15 and TS 38.508-1 [14] clause 4.3.1 and 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.5.4.2.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |
| T2 | s | 2 |  |

17.5.4.2.1.4.2	Test procedure

Same as the test procedure given in clause 7.5.8.2.1.4.2.

17.5.4.2.1.4.3	Message contents

Same as the message contents given in clause 7.5.8.2.1.4.3.

17.5.4.2.1.5	Test requirement

Same as the test requirements given in clause 7.5.8.2.1.5 with following exceptions:

-	Table 7.5.8.2.1.5-1 is replaced by Table 17.5.4.2.1.5-1;

-	Table 7.5.8.2.1.5-2 is replaced by Table 17.5.4.2.1.5-2;

Table 17.5.4.2.1.5-1: NR Cell specific test parameters

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 |
| Data RBs allocated |  | 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State 0 |  | TCI.State.0 |
| TCI State 1 |  | TCI.State.1 |
| reportConfigType |  | ssb-Index-RSRP |
| reportConfigType |  | periodic |
| Number of reported RS |  | 2 |
| L1-RSRP reporting period | slot | 640 |
| timeRestrictionForChannelMeasurements |  | configured |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table 17.5.4.2.1.5-2: OTA related test parameters

| Parameter | Unit | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |
|  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup 3 according to clause A.9 |  |  |  |
|  |  | AoA1 |  | AoA2 |  |
| Assumption for UE beams Note 4 |  | Rough |  |  |  |
| Ês | dBm/SCS | -80.9+TT | -80.9+TT | -Infinity | -80.9+TT |
| SSB_RP Note 1 | dBm/ SCS | -80.9+TT | -80.9+TT | -Infinity | -80.9+TT |
| BB Note 5 | dB | 8.3+TT | 8.3+TT | -Infinity | 8.3+TT |
| IoNote1 | dBm/ 95.04 MHz Note2 | -55.71+TT | -55.71+TT | - Infinity | -55.71+TT |
| Note 1: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 3: As observed with 0dBi gain antenna at the centre of the quiet zone. Note 4: Information about types of UE beam is given in 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |

## 17.6	Measurement procedures for RedCap

### 17.6.1	Intra-frequency measurements for RedCap

#### 17.6.1.0	Minimum conformance requirements

##### 17.6.1.0.1	General

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig.

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE can perform intra-frequency SSB based measurements without measurement gaps if

-	the SSB is completely contained in the active BWP of the UE, or

-	the active downlink BWP is initial BWP [3].

For intra-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in 38.133 [6] clause 9.2B.5.3.

SSB based measurements are configured along with one or two measurement timing configuration(s) (SMTC(s)) which provides periodicity, duration and offset information on a window of up to 5ms where the measurements are to be performed. For intra-frequency connected mode measurements, up to two measurement window periodicities may be configured. A single measurement window offset and measurement duration are configured per intra-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. Switching time is 0.5ms for frequency range FR1.

The requirements in this clause apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	For 2Rx RedCap:

-	SS-RSRP related side conditions given in 38.133 [6] clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in 38.133 [6] clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in 38.133 [6] clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

For 2Rx RedCap: The requirements in 38.133 [6] clause 9.2.4.3 shall apply.

The UE shall not send any event triggered measurement reports as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra without index_RedCap defined in clause 17.6.1.0.2 or clause 17.6.1.0.3. When L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index_RedCap as defined in clause 17.6.1.0.2 or clause 17.6.1.0.3. If a cell which has been detectable at least for the time period Tidentify intra without index_RedCap defined in clause 17.6.1.0.2 or clause 17.6.1.0.3 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra_RedCap provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2B.1 and 9.2B.4.3.

##### 17.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index_RedCap if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_RedCap. It is assumed that deriveSSB-IndexFromCell is always enabled for FR2.

Tidentify_intra_without_index_RedCap = (TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap) ms

Where:

TPSS/SSS_sync_intra_RedCap: it is the time period used in PSS/SSS detection given in Table 17.6.1.0.2-1.

T SSB_measurement_period_intra_RedCap: equal to a measurement period of SSB based measurement given in Table 17.6.1.0.2-2.

CSSFintra_RedCap: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap_RedCap,i in 38.133[6] clause 9.1A.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps,  or according to CSSFwithin_gap_RedCap,i in 38.133[6] clause 9.1A.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

For 2Rx RedCap:

Mpss/sss_sync_w/o_gaps_RedCap : For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync_w/o_gaps_RedCap =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps_RedCap =24.  For a UE supporting FR2 power class 3, Mpss/sss_sync_w/o_gaps_RedCap =24. For a UE supporting FR2 power class 4, Mpss/sss_sync_w/o_gaps_RedCap =24, For a UE supporting FR2 power class 7, Mpss/sss_sync_w/o_gaps_RedCap = 24.

Mmeas_period_w/o_gaps_RedCap : For a UE supporting power class 1 or 5, Mmeas_period_w/o_gaps_RedCap =40. For a UE supporting FR2 power class 2, Mmeas_period_w/o_gaps_RedCap =24. For a UE supporting power class 3, Mmeas_period_w/o_gaps_RedCap =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps_RedCap =24, For a UE supporting power class 7, Mmeas_period_w/o_gaps_RedCap = 24.

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

If the higher layer signalling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index_RedCap

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If the higher layer signalling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra_RedCap.

For FR2, a longer measurement period is allowed, if aperiodic CSI-RS resource is measured for L1-RSRP measurement on any FR2 serving frequency in the same band, and the CSI-RS resource is outside measurement gap and overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols. If SSB-ToMeasure or SS-RSSI-Measurement is configured, the SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same band which can be merged and the RSSI symbols are indicated by SS-RSSI-Measurement.

Table 17.6.1.0.2-1: Time period for PSS/SSS detection, (Frequency range FR2) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps_RedCap x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_RedCap x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps_RedCap x Kp x Klayer1_measurement) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 17.6.1.0.2-2: Measurement period for intra-frequency measurements without gaps (FR2) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps_RedCap x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_RedCap x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps_RedCap xKp x Klayer1_measurement ) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 17.6.1.0.2-6: Measurement period for intra-frequency measurements without gaps (FR1) for 1Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max([200]ms, ceil( [5] x Kp) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max([200]ms, ceil(1.5x [5] x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil( [5] x Kp ) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The following scheduling restriction applies due to SS-RSRP or SS-SINR measurement on an FR2 intra-frequency cell

The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration (The signalling deriveSSB_IndexFromCell is always enabled for FR2). If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

The following scheduling restriction applies to SS-RSRQ measurement on an FR2 intra-frequency cell

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration (The signalling deriveSSB_IndexFromCellc is always enabled for FR2). If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

If following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots.

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, the UE is expected to receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, the UE is expected to receive PDSCH that corresponds to the PDCCH that the UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2B.5.

##### 17.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index_RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_RedCap. It is assumed that deriveSSB-IndexFromCell is always enabled for FR2.

Tidentify_intra_without_index_RedCap = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap  ms

Where:

TPSS/SSS_sync_intra_RedCap: it is the time period used in PSS/SSS detection given in Table 17.6.1.0.3-1.

T SSB_measurement_period_intra_RedCap: equal to a measurement period of SSB based measurement given in Table 17.6.1.0.3-2.

CSSFintra_RedCap: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_RedCap,i in 38.133 [6] clause 9.1A.5.2 for measurement conducted within measurement gaps.

For 2Rx RedCap:

Mpss/sss_sync_with_gaps_RedCap : For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync with_gaps_RedCap =40. For a UE supporting FR2 power class 2, Mpss/sss_sync with_gaps_RedCap =24.  For a UE supporting FR2 power class 3, Mpss/sss_sync with_gaps_RedCap =24. For a UE supporting power class 4, Mpss/sss_sync with_gaps_RedCap =24, For a UE supporting FR2 power class 7, Mpss/sss_sync with_gaps_RedCap =24.

Mmeas_period_ with_gaps_RedCap: For a UE supporting power class 1 or 5, Mmeas_period_ with_gaps_RedCap =40. For a UE supporting power class 2, Mmeas_period_ with_gaps_RedCap =24. For a UE supporting power class 3, Mmeas_period_ with_gaps_RedCap =24. For a UE supporting power class 4, Mmeas_period with_gaps_RedCap =24, For a UE supporting power class 7, Mmeas_period_ with_gaps_RedCap =24.

If the higher layer signalling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index_RedCap or Tidentify_intra_with_index_RedCap.

Table 17.6.1.0.3-1: Time period for PSS/SSS detection (FR2) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max(600ms, Mpss/sss_sync_with_gaps_RedCap x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5x Mpss/sss_sync_with_gaps_RedCap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra_RedCap_RedCap |
| DRX cycle>320ms | Mpss/sss_sync_with_gaps_RedCap x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 17.6.1.0.3-2: Measurement period for intra-frequency measurements with gaps (FR2) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max(400ms, Mmeas_period with_gaps_RedCap x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x Mmeas_period with_gaps_RedCap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra_RedCap |
| DRX cycle>320ms | Mmeas_period with_gaps_RedCap x max(MGRP, DRX cycle) x CSSFintra_RedCap |

The normative reference for this requirement is TS 38.133 [6] clauses 9.2B.6.

#### 17.6.1.1	NR SA FR2 Event triggered reporting test without gap under non-DRX

Editor's Note: This test case has been completed for the following configurations:

Test frequency f ≤ 40.8 GHz

UE PC3

This test case is incomplete for UE power classes other than PC3 (e.g. PC7)

This test case is incomplete for test frequencies > 40.8 GHz

17.6.1.1.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements for RedCap in 38.133 [6] clause 9.2B.5.1 and 9.2B.5.2

17.6.1.1.2	Test applicability

This test applies to all types of NR RedCap UE from release 17 onwards.

17.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.1.0.1 and 17.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.1.1.

17.6.1.1.4	Test description

17.6.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.6.1.1.4.1 with following exceptions:

-	Table 7.6.1.1.4.1-1 is replaced by Table 17.6.1.1.4.1-1.

-	Table 7.6.1.1.4.1-2 is replaced by Table 17.6.1.1.4.1-2.

Table 17.6.1.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 17.6.1.1-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 17.6.1.1-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 17.6.1.1.4.1-2.

Table 17.6.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.6.1.1.4.1-3: General test parameters

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -11 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| TimeToTrigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

17.6.1.1.4.2	Test procedure

Same as the test procedure given in clause 7.6.1.1.4.2 with following exceptions:

-	Step 6 in test procedure in clause 7.6.1.1.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is

-	2402 ms for UE supporting power class 1 or 5

-	1442 ms for UE supporting power class 2, 3, 4 or 7,

17.6.1.1.4.3	Message contents

Same as the message contents given in clause 7.6.1.1.4.3.

17.6.1.1.5	Test requirement

Same as the test requirements given in clause 7.6.1.1.5 with following exceptions:

-	Table 7.6.1.1.5-1 is replaced by Table 17.6.1.1.5-1.

-	Table 7.6.1.1.5-2 is replaced by Table 17.6.1.1.5-2.

-	X is

-	2.4s for a UE supporting power class 1 or 5,

-	1.44s for a UE supporting power class 2, 3, 4 or 7.

Table 17.6.1.1.5-1: NR Cell specific test parameters

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1,2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1 | 24 |  | 24 |  |
|  |  | 2 | 48 |  | 48 |  |
| Initial BWP configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.2 TDD |  | N/A |  |
|  |  | 2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
|  |  | 2 | CR.3.2 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
|  |  | 2 | CCR.3.7 TDD |  | N/A |  |
| TRS configuration |  | 1,2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1,2 | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 120 |  | 120 |  |
| OCNG Patterns |  | 1,2 | OP.5 |  | N/A |  |
| cellIndividualOffset | dB | 1,2 | N/A |  | 16 |  |
| SSB |  | 1 | SSB.1 FR2 |  | SSB.7 FR2 |  |
|  |  | 2 | SSB.2 FR2 |  | SSB.8 FR2 |  |
| Propagation Condition |  | 1,2 | AWGN |  | AWGN |  |

Table 17.6.1.1.5-2: NR OTA Cell specific test parameters

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.9.3 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
| Es | dBm/SCS | 1 | -89 | -89 | -Infinity | -89 |
|  |  | 2 | -86 | -86 | -Infinity | -86 |
| BB Note 5 | dB | 1, 2 | -0.12 | -0.12 | -Infinity | -0.12 |
| SSB_RP | dBm/SCS | 1 | -89 | -89 | -Infinity | -89 |
|  |  | 2 | -86 | -86 | -Infinity | -86 |
|  | dBm/95.04MHz | 1 | -64.41 | -64.41 | -Infinity | -64.41 |
|  |  | 2 | -61.41 | -61.41 | -Infinity | -61.41 |
| Time multiplexing of the downlink transmissions from each AoA |  | 1, 2 | Defined in Figure 7.6.1.1.5-1 |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Void Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

#### 17.6.1.2	NR SA FR2 Event triggered reporting test without gap under DRX

Editor's Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

17.6.1.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in 38.133 [6] clause 9.2B.5.1 and 9.2B.5.2.

17.6.1.2.2	Test applicability

This test applies to all types of NR RedCap UEs from release 17 supporting 5GS NR SA FR2 and long DRX cycle.

17.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.1.0.1 and 17.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.1.2.

17.6.1.2.4	Test description

17.6.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 7.6.1.2.4.1 with following exceptions:

-	Table 7.6.1.2.4.1-1 is replaced by Table 17.6.1.2.4.1-1.

-	Table 7.6.1.2.4.1-2 is replaced by Table 17.6.1.2.4.1-2.

Table 17.6.1.2.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.1.2-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.2-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 17.6.1.2.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

17.6.1.2.4.2	Test procedure

Same as the test procedure given in clause 7.6.1.2.4.2 with following exceptions:

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms for sub-test 1 or less than Y ms for sub-test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one. Where X is

-	7202 for UE supporting power class 1 or 5,

-	4322 for UE supporting power class 2, 3, 4 or 7.

and Y is

-	51202 for UE supporting power class 1 or 5,

-	30722 for UE supporting power class 2, 3, 4 or 7.

17.6.1.2.4.3	Message contents

Same as the message contents given in clause 7.6.1.2.4.3.

17.6.1.2.5	Test requirement

Same as the test requirements given in clause 7.6.1.2.5 with following exceptions:

-	For sub-test 1, X is

-	X = 7202 for UE supporting power class 1 or 5;

-	X = 4322 for UE supporting power class 2, 3, 4 or 7;

-	For sub-test 2, X is

-	X = 51202 for UE supporting power class 1 or 5;

-	X = 30722 for UE supporting power class 2, 3, 4 or 7;

#### 17.6.1.3	NR SA FR2 Event triggered reporting test with per-UE gaps under non-DRX

Editor's Note:

This test case is complete for the following configurations:

-	UE PC3

-	Test frequencies f ≤ 40.8 GHz;

This test case is incomplete for UE power classes other than PC3;

This test case is incomplete for test frequencies f > 40.8 GHz.

17.6.1.3.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under non-DRX within intra-frequency cell search with gaps requirements. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 [6] clause 9.2B.6.1 and 9.2B.6.2.

17.6.1.3.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards supporting CSI-RS based RLM and BWP operation without bandwidth restriction.

17.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.1.0.1 and 17.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.1.3.

17.6.1.3.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 17.6.1.3.4.1-3, Table 17.6.1.3.5-1 and Table 17.6.1.3.5-2, respectively. In the measurement configuration a measurement object is configured for the frequency of the PCell and indicates to the UE that event-triggered reporting with Event A3 is used.

17.6.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 17.6.1.3.4.1-1.

Table 17.6.1.3.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 17.6.1.3-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 17.6.1.3-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 17.6.1.3.4.1-2.

Table 17.6.1.3.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting with gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 17.6.1.3.4.1-3.

2. Message contents are defined in clause 17.6.1.3.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 17.6.1.3.4.1-3: General test parameters for NR SA FR2 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | PCell (Cell 1) |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 40 |  |
| Measurement gap length | ms | 6 |  |
| Measurement gap offset | ms | 4 |  |
| SMTC configuration for CD-SSB |  | SMTC.1 RedCap |  |
| SMTC configuration for NCD-SSB |  | SMTC.2 RedCap |  |
| CSI-RS parameters |  | CSI-RS.3.2 TDD |  |
| A3-Offset | dB | -11 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time-To-Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between Cell 1 and Cell 2 | s | 3 | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

17.6.1.3.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 17.6.1.3.5-1.

3. Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 14 + 2 for current iteration of the test procedure loop

4. SS shall transmit an RRCReconfiguration message to configure RedCap-specific dedicated BWP and Event A3 triggered reporting.

5. The UE shall transmit an RRCReconfigurationComplete message. T1 starts upon the SS receives the RRCReconfigurationComplete message.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 17.6.1.3.5-1. T2 starts.

7. The SS increases the number of successful tests by one if the UE transmits a MeasurementReport message triggered by event A3 satisfying the overall delay requirement specified in clause 17.6.1.3.5, otherwise the SS increases the number of failure tests by one.

8. After the SS receive the MeasurementReport message in step 7) or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

or

-	switch the UE off.

9. Depending on the choice in step 8, the SS shall,

-	transmit in NR Cell 1 a Paging message including matched ng-5G-S-TMSI for the UE if an RRCRelease has been sent in step 8. If the paging fails, switches off and on the UE,

or

-	switch on the UE if the UE is switched off in step 8.

10. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

17.6.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 17.6.1.3.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition GAP NEEDED Table H.3.1-4 with A3-offset = -11dB Table H.3.1-6 with Condition gapUE, Pattern #0 and gapOffset is set to 4 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition CSI-RS RLM |

Table 17.6.1.3.4.3-2: MeasObjectNR (Step 4, test procedure)

| Derivation Path: Table H.3.1-3 with condition INTRA-FREQ MO, Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of NCD-SSB |  |  |
| smtc1 | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap |  |  |
| cellsToAddModList SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod { | 1 entry |  |  |
| CellsToAddMod[1] SEQUENCE { |  | entry 1 |  |
| physCellId | PhysCellId of Cell 2 |  |  |
| cellIndividualOffset SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB16 |  |  |
| rsrqOffsetSSB | Not present |  |  |
| sinrOffsetSSB | Not present |  |  |
| rsrpOffsetCSI-RS | Not present |  |  |
| rsrqOffsetCSI-RS | Not present |  |  |
| sinrOffsetCSI-RS | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.6.1.3.4.3-3: ServingCellConfig (step 4, test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 17.6.1.3.4.3-4 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 17.6.1.3.4.3-5 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 17.6.1.3.4.3-4: BWP-Downlink (Table 17.6.1.3.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to DLBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms40 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| servingCellMO-r17 | MeasId |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.6.1.3.4.3-5: BWP-Uplink (Table 17.6.1.3.4.3-3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV corresponds to ULBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

17.6.1.3.5	Test requirement

Table 17.6.1.3.4.1-3 and Table 17.6.1.3.5-1 define the primary level settings including test tolerances.

Table 17.6.1.3.5-1: NR Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated | Config 1 |  | 24 |  | 24 |  |
|  | Config 2 |  | 48 |  | 48 |  |
| Initial BWP configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated DL BWP configuration |  |  | DLBWP.1.3 RedCap Note 1 |  | DLBWP.1.3 RedCap Note 1 |  |
| Dedicated UL BWP configuration |  |  | ULBWP.1.3 RedCap Note 2 |  | ULBWP.1.3 RedCap Note 2 |  |
| RLM-RS |  |  | CSI-RS |  | SSB |  |
| PDSCH RMC configuration | Config 1 |  | SR.3.2 TDD |  | N/A |  |
|  | Config 2 |  | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.3.1 TDD |  | CR.3.1 TDD |  |
|  | Config 2 |  | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
|  | Config 2 |  | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| TRS configuration |  |  | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  |  | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  | 120 |  |
| OCNG Patterns |  |  | OP.1 |  | OP.1 |  |
| cellIndividualOffset |  | dB | N/A |  | 16 |  |
| CD-SSB | Config 1 |  | SSB.2 RedCap FR2 |  | SSB.2 RedCap FR2 |  |
|  | Config 2 |  | SSB.4 RedCap FR2 |  | SSB.4 RedCap FR2 |  |
| NCD-SSB | Config 1 |  | SSB.3 RedCap FR2 |  | SSB.3 RedCap FR2 |  |
|  | Config 2 |  | SSB.5 RedCap FR2 |  | SSB.5 RedCap FR2 |  |
| Propagation Condition |  |  | AWGN |  |  |  |
| Note 1: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dedicated DL BWP2 corresponding to NCD-SSB PRB index; Note 2: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |  |

Table 17.6.1.3.5-2: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps without DRX

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Setup 3 defined in A.9 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 4 |  |  | Rough |  | Rough |  |
| Es | Config 1 | dBm/ SCS | -89 | -89 | -Infinity | -89 |
|  | Config 2 |  | -86 | -86 | -Infinity | -86 |
| BB Note 5 |  | dB | -0.12 | -0.12 | -Infinity | -0.12 |
| SSB_RP | Config 1 | dBm/ SCS | -89 | -89 | -Infinity | -89 |
|  | Config 2 |  | -86 | -86 | -Infinity | 86 |
|  | Config 1 | dBm/ 95.04MHz | -64.41 | -64.41 | -Infinity | -64.41 |
|  | Config 2 |  | -61.41 | -61.41 | -Infinity | -61.41 |
| Time multiplexing of the downlink transmissions from each AoA |  |  | Defined in Figure 17.6.1.3.5-1 |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

The overall delay is measured from the beginning of time period T2 to the time that the SS receives the first MeasurementReport message triggered by event A3. It is allowed to be up to 2xTTIDCCH higher than the measurement reporting delay specified in 38.133[6] clause 9.2B.4 due to the TTI insertion uncertainty of the measurement report in DCCH.

Then the overall delay shall be shall be no larger than: Tidentify_intra_without_index + 2xTTIDCCH, where

-	Tidentify_intra_without_index = 3200 ms for UE supporting power class 1 or 5 and 1920 ms for UE supporting power class 2, 3, 4 or 7, is the time for intra-frequency cell identification specified in 38.133 [6] clause 9.2B.5.1,

-	TTIDCCH = 1 ms.

Which gives a total of 3202 ms for UE supporting power class 1 or 5 and 1922 ms for UE supporting power class 2, 3, 4 or 7.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

Figure 17.6.1.3.5-1: Time multiplexed downlink transmissions (Config 1 example)

#### 17.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX

Editor's Note:

This test case is complete for the following configurations:

-	UE PC3

-	Test frequencies f ≤ 40.8 GHz;

This test case is incomplete for UE power classes other than PC3;

- This test case is incomplete for test frequencies f > 40.8 GHz.

17.6.1.4.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 [6] clause 9.2B.6.1 and 9.2B.6.2

17.6.1.4.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards supporting CSI-RS based RLM, BWP operation without bandwidth restriction and long DRX cycle.

17.6.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.1.0.1 and 17.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.1.4.

17.6.1.4.4	Test description

17.6.1.4.4.1	Initial conditions

Same as the initial conditions given in clause 7.6.1.4.4.1 with following exceptions:

-	Table 7.6.1.4.4.1-1 is replaced by Table 17.6.1.4.4.1-1.

-	Table 7.6.1.4.4.1-2 is replaced by Table 17.6.1.4.4.1-2.

-	Table 7.6.1.4.4.1-3 is replaced by Table 17.6.1.4.4.1-3.

Table 17.6.1.4.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 17.6.1.4-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 17.6.1.4-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 17.6.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.6.1.4.4.1-3: General test parameters

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Active cell |  | PCell (Cell 1) |  |  |
| Neighbour cell |  | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | Per-UE gaps |  |  |
| Measurement gap repetition periodicity | ms | 40 |  |  |
| Measurement gap length | ms | 6 |  |  |
| Measurement gap offset | ms | 39 |  |  |
| SMTC configuration |  | SMTC.1 |  |  |
| CSI-RS parameters |  | CSI-RS.3.2 TDD |  |  |
| A3-Offset | dB | -7 |  |  |
| CP length |  | Normal |  |  |
| Hysteresis | dB | 0 |  |  |
| Time To Trigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | DRX.1 | DRX.7 |  |
| Time offset between Cell 1 and Cell 2 |  | 3 s |  | Synchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 10 | 52 |  |

17.6.1.4.4.2	Test procedure

Same as the test procedure given in clause 7.6.1.4.4.2 with following exceptions:

- Step 6 in clause 7.6.1.4.4.2 is replaced by:

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms for Test 1 or less than Y ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X and Y is

-	X = 7202, Y = 51202 for UE supporting power class 1 or 5,

-	X = 4322, Y = 30722 for UE supporting power class 2, 3, 4 or 7,

17.6.1.4.4.3	Message contents

Same as the message contents given in clause 7.6.1.4.4.3.

17.6.1.4.5	Test requirement

Same as the test requirements given in clause 7.6.1.4.5 with following exceptions:

-	Table 7.6.1.4.5-1 is replaced by Table 17.6.1.4.5-1.

-	Table 7.6.1.4.5-2 is replaced by Table 17.6.1.4.5-2.

-	In Test 1 the overall delay measured shall be less than a total of X ms, where

-	X = 7202 for UE supporting power class 1 or 5,

-	X = 4322 for UE supporting power class 2, 3, 4 or 7.

-	In Test 2 the overall delay measured shall be less than a total of X ms, where

-	X = 51202 for UE supporting power class 1 or 5,

-	X = 30722 for UE supporting power class 2, 3, 4 or 7.

Table 17.6.1.4.5-1: NR Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | 66 |  | 66 |  |
| Initial BWP configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  |  | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  |  | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  |  | CSI-RS |  | SSB |  |
| PDSCH RMC configuration | Config 1 |  | SR.3.2 TDD |  | N/A |  |
|  | Config 2 |  | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.3.1 TDD |  | N/A |  |
|  | Config 2 |  | CR.3.2 TDD |  |  |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.3.1 TDD |  | N/A |  |
|  | Config 2 |  | CCR.3.7 TDD |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  | 120 |  |
| OCNG Patterns |  |  | OP.1 |  | OP.1 |  |
| SSB | Config 1 |  | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  | Config 2 |  | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  |  | AWGN |  |  |  |

Table 17.6.1.4.5-2: NR OTA Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Setup 1 defined in A.9.1 |  |  |  |
| Beam AssumptionNote 4 |  |  | Rough |  |  |  |
| BB Note 5 |  | dB | 3.51 | -1.60 | -Infinity | -1.60 |
| Note 2 |  | dBm/ 15 KHz | -101.5 |  |  |  |
| Note 2 | Config 1 | dBm/SCS | -92.5 |  |  |  |
|  | Config 2 |  | -89.5 |  |  |  |
| SSB_RPSS-RSRP | Config 1 | dBm/SCS | -88.50 | -88.50 | -Infinity | -88.50 |
|  | Config 2 |  | -85.50 | -88.50 | -Infinity | -88.50 |
|  |  | dB | 4 | 4 | -Infinity | 4 |
|  |  | dBm/ 95.04MHz | -58.03 | -55.68 | -58.03 | -55.68 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

### 17.6.2

### 17.6.3	L1-RSRP measurement for beam reporting for RedCap

#### 17.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting

##### 17.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting

The minimum requirements given in 5.6.3.0.1 applies.

The normative reference for this requirement is TS 38.133 [6] clause 9.5B.1, 9.5B.2, 9.5B.3.1, 9.5B.4.1 and 9.5B.5.1.

##### 17.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting

The minimum requirements given in clause 5.6.3.0.2 applies.

The normative reference for this requirement is TS 38.133 [6] clauses 9.5B.1, 9.5B.2, 9.5B.3.3, 9.5B.4.2 and 9.5B.5.1.

#### 17.6.3.1	NR SA FR2 SSB based L1-RSRP measurement when DRX is not used

Editor’s Notes:

This test case has been completed for the following configurations:

-	Test frequencies f ≤ 40.8 GHz

-	UE PC3

-	This test case is incomplete for UE power classes other than PC3

-	This test case is incomplete for Test frequencies f > 40.8 GHz

17.6.3.1.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.1.

17.6.3.1.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards.

17.6.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.3.1.

17.6.3.1.4	Test description

17.6.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.6.3.1.4.1 with following exceptions:

-	Table 7.6.3.1.4.1-1 is replaced by Table 17.6.3.1.4.1-1.

-	Table 7.6.3.1.4.1-3 is replaced by Table 17.6.3.1.4.1-2.

Table 17.6.3.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 17.6.3.1.4.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 17.6.3.1.4.1-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.6.3.1.4.1-2: Test environment

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.5.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

17.6.3.1.4.2	Test procedure

Same as the test procedure given in clause 7.6.3.1.4.2.

17.6.3.1.4.3	Message contents

Same as the test procedure given in clause 7.6.3.1.4.3.

17.6.3.1.5	Test requirement

Same as the test requirements given in clause 7.6.3.1.5.

#### 17.6.3.2	NR SA FR2 SSB based L1-RSRP measurement when DRX is used

Editor’s Notes:

This test case has been completed for the following configurations:

-	Test frequencies f ≤ 40.8 GHz

-	UE PC3

This test case is incomplete for UE power classes other than PC3

This test case is incomplete for Test frequencies f > 40.8 GHz

17.6.3.2.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.1.

17.6.3.2.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards supporting long DRX cycle.

17.6.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.3.2.

17.6.3.2.4	Test description

17.6.3.2.4.1	Initial conditions

Same as the initial conditions given in 7.6.3.2.4.1 with following exceptions:

-	Table 7.6.3.2.4.1-1 is replaced by Table 17.6.3.2.4.1-1.

-	Table 7.6.3.2.4.1-3 is replaced by Table 17.6.3.2.4.1-2.

-	Table 7.6.3.2.4.1-2 is replaced by Table 17.6.3.2.4.1-3.

Table 17.6.3.2.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 17.6.3.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 17.6.3.2-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.6.3.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.6.3.2.4.1-3: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1,2 |  | freq1 |
| Duplex mode | 1,2 |  | TDD |
| TDD Configuration | 1,2 |  | TDDConf.3.1 |
| BWchannel | 1,2 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1,2 |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD |
|  | 2 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| OCNG Patterns | 1,2 |  | OP.1 |
| Initial BWP Configuration | 1,2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1,2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1,2 |  | SMTC.1 |
| TRS Configuration | 1,2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1,2 |  | TCI.State.2 |
| DRX configuration | 1,2 |  | DRX.3 |
| reportConfigType | 1,2 |  | periodic |
| reportQuantity | 1,2 |  | ssb-Index-RSRP |
| Number of reported RS | 1,2 |  | 2 |
| L1-RSRP reporting period | 1,2 | slot | 320 |
| T1 | 1,2 | s | 5 |
| T2 | 1,2 | s | 3 |
| EPRE ratio of PSS to SSS | 1,2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1,2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

17.6.3.2.4.2	Test procedure

Same as the test procedure given in clause 7.6.3.2.4.2.

17.6.3.2.4.3	Message contents

Same as the message content given in 7.6.3.2.4.3.

17.6.3.2.5	Test requirement

Same as the test requirements given in clause 7.6.3.2.5 with following exceptions:

-	Table 7.6.3.2.5-1 is replaced by Table 17.6.3.2.5-1.

Table 17.6.3.2.5-1: Cell specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.9 |  |  |  |
| Assumption for UE beamsNote 4 | 1,2 |  | Rough |  |  |  |
| Note2 | 1,2 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 2 |  | -93 |  |  |  |
|  | 1,2 | dB | 0 | 0 | -Infinity | 9 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 2 |  | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1,2 | dB | 0 | 0 | -Infinity | 9 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

#### 17.6.3.3	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is not used

Editor’s Notes:

This test case has been completed for the following configurations:

-	Test frequencies f ≤ 40.8 GHz

-	UE PC3

This test case is incomplete for UE power classes other than PC3

This test case is incomplete for Test frequencies f > 40.8 GHz

17.6.3.3.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.2.

17.6.3.3.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards.

17.6.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.3.3.

17.6.3.3.4	Test description

17.6.3.3.4.1	Initial conditions

Same as the initial conditions given in 7.6.3.3.4.1 with following exceptions:

-	Table 7.6.3.3.4.1-1 is replaced by Table 17.6.3.3.4.1-1.

-	Table 7.6.3.3.4.1-3 is replaced by Table 17.6.3.3.4.1-2.

-	Table 7.6.3.3.4.1-2 is replaced by Table 17.6.3.3.4.1-3.

Table 17.6.3.3.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 17.6.3.3-1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.6.3.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.6.3.3.4.1-3: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 8 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

17.6.3.3.4.2	Test procedure

Same as the test procedure given in clause 7.6.3.3.4.2.

17.6.3.3.4.3	Message contents

Same as the message content given in 7.6.3.3.4.3.

17.6.3.3.5	Test requirement

Same as the test requirements given in clause 7.6.3.3.5 with following exceptions:

-	Table 7.6.3.3.5-1 is replaced by Table 17.6.3.3.5-1.

Table 17.6.3.3.5-1: Cell specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.9 |  |
| Beam AssumptionNote 3 | 1 |  | Rough | Rough |
| Note1 | 1 | dBm/15kHz | -105 |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |
|  | 1 | dB | 0 | 9 |
| CSI-RS RSRP Note 2 | 1 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note 2 | 1 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1 | dB | 0 | 9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in 38.133 [6] B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

#### 17.6.3.4	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used

Editor’s Notes:

This test case has been completed for the following configurations:

-	Test frequencies f ≤ 40.8 GHz

-	UE PC3

This test case is incomplete for UE power classes other than PC3

This test case is incomplete for Test frequencies f > 40.8 GHz

17.6.3.4.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.2.

17.6.3.4.2	Test applicability

This test applies to all types of NR RedCap UEs from Release 17 onwards supporting long DRX cycle.

17.6.3.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.3.4.

17.6.3.4.4	Test description

17.6.3.4.4.1	Initial conditions

Same as the initial conditions given in 7.6.3.4.4.1 with following exceptions:

-	Table 7.6.3.4.4.1-1 is replaced by Table 17.6.3.4.4.1-1.

-	Table 7.6.3.4.4.1-3 is replaced by Table 17.6.3.4.4.1-2.

-	Table 7.6.3.4.4.1-2 is replaced by Table 17.6.3.4.4.1-3.

Table 17.6.3.4.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 17.6.3.4-1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.6.3.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.3.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 17.6.3.4.4.1-3: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | DRX.3 |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 8 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

17.6.3.4.4.2	Test procedure

Same as the test procedure given in clause 7.6.3.4.4.2.

17.6.3.4.4.3	Message contents

Same as the message content given in 7.6.3.4.4.3.

17.6.3.4.5	Test requirement

Same as the test requirements given in clause 7.6.3.4.5 with following exceptions:

-	Table 7.6.3.4.5-1 is replaced by Table 17.6.3.4.5-1.

Table 17.6.3.4.5-1: Cell specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.9 |  |
| Beam AssumptionNote 3 | 1 |  | Rough | Rough |
| Note1 | 1 | dBm/15kHz | -105 |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |
|  | 1 | dB | 0 | 9 |
| CSI-RS RSRP Note 2 | 1 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note 2 | 1 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1 | dB | 0 | 9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in 38.133 [6] B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

### 17.6.4	NR Measurements with autonomous gaps for RedCap

#### 17.6.4.0	Minimum conformance requirements

##### 17.6.4.0.1	Minimum conformance requirements for CGI identification of an NR cell with autonomous gaps

The UE shall identify and report the CGI of a known NR target cell when requested by the network for the purpose of reportCGI. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [13]. Note that a UE is not required to use autonomous gap if useAutonomousGaps is set to false. If autonomous gaps are used for measurement with the purpose of reportCGI, regardless of whether DRX is used or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI_redcap = (TMIB_redcap + T SIB1_redcap) ms

Where:

TMIB_redcap is the time period used to acquire MIB message. TMIB_redcap = 25 * TSMTC ms for target cell carrier frequency on FR2.

TSIB1_redcap is the time period used to acquire SIB1 message. TSIB1_redcap = 6 * TRMSI-scheduling ms.

Where TSMTC is the SMTC periodicity configured for the target cell measurement, and TRMSI-scheduling is

-	the periodicity with which the SIB1 is actually transmitted by the NR target cell when SSB and RMSI CORESET multiplexing pattern is 1

-	the maximum between the periodicity with which the SIB1 is actually transmitted by the NR target cell and TSMTC when SSB and RMSI CORESET multiplexing pattern is 2 or 3.The requirement for identifying the CGI of an NR cell within Tidentify_CGI_redcap is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [13] is used.

In the requirement a cell is known if,

-	During the last 5 seconds for FR1 or 3 seconds for FR2 before the reception of the report CGI command:

-	The UE has sent a valid L3-RSRP measurement report with SSB index for the target cell and

-	During MIB decoding at least reported SSBs remains detectable according to the cell identification conditions specified in clauses 9.2B or 9.3B of TS 38.133 [6], and

-	During SIB1 decoding the SSB used for MIB decoding remains detectable according to the cell identification conditions specified in clauses 9.2B or 9.3B of TS 38.133 [6], and

-	During MIB decoding, the SSB for MIB decoding remains detectable with SNR ≥-3dB

-	During SIB1 decoding, the PDSCH for SIB1 decoding remains detectable with SNR ≥-3dB

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_redcap plus RRC procedure delay defined in clause 12 in TS 38.331 [13], and additional 20ms margin if target cell is on FR2.

When a RedCap UE is identifying CGI of an NR cell with autonomous gaps, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on PCell for the following slots:

-	with up to K1 interruptions with interrupted slots up to X1 specified in Table 17.6.4.0.1-1 for each interruption during MIB decoding time period TMIB_redcap (ms).

-	with up to L1 interruptions with interrupted slots up to Y1 specified in Table 17.6.4.0.1-1 during SIB1 decoding time period TSIB1_redcap (ms) for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to L2 interruptions with interrupted slots up to interruption length Y2 specified in Table 17.6.4.0.1-1 during SIB1 decoding time period TSIB1_redcap (ms) for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Where:

-	K1 = 25 for the target cell carrier frequency on FR2, and

-	L1 = TSIB1_redcap /20 and

-	L2 = TSIB1_redcap /TSMTC, where TSMTC is the periodicity of the SMTC occasion configured for the target cell carrier.

Table 17.6.4.0.1-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 3 | 0.125 | 48 | 49 | 37 |

The normative reference for this requirement is TS 38.133 [6] clauses 9.11A.2, 9.11A.3 and 9.11A.4.

#### 17.6.4.1	NR SA FR2 Interfrequency CGI reporting in autonomous gaps test (PCell in FR2)

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

17.6.4.1.1	Test purpose

To verify that the UE makes correct reporting of an CGI. This test will partly verify the SA inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.11A.

17.6.4.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards supporting FR2 and acquisition of CGI from neighbour NR cell using autonomous gap.

17.6.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.4.1.

17.6.4.1.4	Test description

The test scenario comprises of two NR carriers and two NR cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

The test consists of three successive time periods, with time durations of T1 and T2 respectively:

-	Measurement gap patterns are configured. During T1 the UE shall report event A3 for cell 2. Within 3 seconds of the event report, the test equipment shall add a measurement reporting configuration using ReportConfigNR which contains a ReportCGI IE with cellForWhichToReportCGI set to the physical Cell ID of cell 2 and including the optional IE useAutonomousGaps-r16.

-	Time period T2 begins 10ms after the test equipment has transmitted the RRC reconfiguration message containing the ReportCGI IE.

17.6.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 17.6.4.1.4.1-1.

Table 17.6.4.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.6.4.1-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 17.6.4.1.4.1-2

Table 17.6.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 17.6.4.1.4.3.

2.	The power levels and settings for NR PCell (Cell 1) and NR neighbour cell (Cell 2) are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 16.3.1.3.4.1-3 below.

Table 17.6.4.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | NR Cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | 39 |  |
| SMTC-SSB parameters |  | SSB.3 FR2 | As specified in clause A.3.10.2 |
| SI-RNTI scheduling rate | ms | 40 | S-RNTI scheduled on 4 occasions per 160ms transmission period |
| A3-Offset | dB | -30 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | 3s | Synchronous cells. |
| T1 | s | <10 | UE expected to report event A3 for cell 2 within 5,2s (PC1)or 3.5s (other PC) of the start of T1. Test equipment shall configure CGI reporting within 3s after receiving the event A3 report. T2 begins 10ms after test equipment has transmitted the RRC reconfiguration to configure CGI reporting. |
| T2 | s | 1 |  |

17.6.4.1.4.2	Test procedure

Throughout the test, the UE shall be scheduled to perform DL reception on every slot suitable for DL.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 17.6.4.1.5-1 and Table 17.6.4.1.5-2. T1 starts.

3.	The SS transmits an RRCReconfiguration message to configure event A3 triggered measurement reporting on NR Cell 2.

4.	The UE transmits an RRCReconfigurationComplete message.

5.	UE transmits a MeasurementReport message triggered by Event A3.

6.	Within 3 seconds after receiving MeasurementReport message in step 6, the SS transmits an RRCReconfiguration message to configure an autonomous gap based CGI measurement reporting on NR Cell 2.

7.	The start of T2 is the instant 10ms after the last TTI containing the RRCReconfiguration message in step 7 is sent to the UE, at that instant the SS shall switch the power setting from T1 to T2 as specified in Table 17.6.4.1.5-1 and 17.6.4.1.5-2.

8.	If the UE

a)	transmits a MeasurementReport message containing CGI of NR Cell 2 less than 645 ms from the beginning of time period T2, and

b)	has missed no more than 1240 ACK/NACKs from the start of T2 until 645 ms,

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

9.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

10.	Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

17.6.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 17.6.4.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-7 with condition INTER-FREQ for test procedure step 6 |

Table 17.6.4.1.4.3-2: MeasConfig (Step 3 & 8, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT(-15) specified in Table H.3.1-4 | A3-Offset is set to -30 Acutal threshold = -30*0.5 = -15dB | Step 3 |
|  | ReportConfigNR-CGI | Table 17.6.4.1.4.3-3 | Step 8 |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-DEFAULT specified in Table H.3.1-6 with condition gapUE and Pattern #13 |  | Step 3 |
| measGapConfig SEQUENCE { |  |  | Step 8 |
| gapUE CHOICE { |  |  |  |
| release | null | MG needs to be released to avoid interfering with the missed ACK/NACK count |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.6.4.1.4.3-3: ReportConfigNR-CGI (Table 17.6.4.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition CGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| reportCGI SEQUENCE { |  |  |  |
| cellForWhichToReportCGI | PhysCellId of NR Cell 2 |  |  |
| useAutonomousGaps-r16 | setup |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.6.4.1.4.3-4: MeasResults (Step 10, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measResultServingMOList | Not checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR [1] SEQUENCE { |  | Entry 1 |  |
| physCellId | PhysCellId of NR Cell 2 |  |  |
| measResult | Not checked |  |  |
| cgi-Info | Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

17.6.4.1.5	Test requirements

Table 17.6.4.1.4.1-3 and Table 17.6.4.1.5-1 define the primary level settings including test tolerances for the test.

Table 17.6.4.1.5-1: NR Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Setup 3 as specified in clause A.3.15 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  | Rough |  |  |  |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| Duplex mode |  |  | TDD |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| OCNG Patterns |  |  | OP.1 |  | Not sent |  |
| PDSCH RMC |  |  | SR.3.1 TDD |  | - |  |
| CORESET RMC |  |  | CR.3.1 TDD |  | - |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  | N/A |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 | -99.03+TT |  | -99.03+TT |  |
| Note2 |  | dBm/SCS Note4 | -90+TT |  | -90+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | -87+TT |  | -93+TT |  |
|  |  | dB | 3+TT |  | -3+TT |  |
|  |  | dB | 3+TT |  | -3+TT |  |
| IoNote3 |  | dBm/95.04 MHzNote5 | -56.25+TT |  | -59.25+TT |  |
| Propagation Condition |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 [6] B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The UE shall transmit a measurement report containing the cell global identifier of Cell 2 within overall delay from the start of T2, where the overall delay is calculated as follows,

Overall delay = Tidentify_CGI_RedCap + additional margin for FR2 + TTI insertion uncertainty

-	Tidentify_CGI_RedCap = TMIB_redcap + TSIB1_redcap

-	TMIB_redcap = 500 ms, is the time period used to acquire MIB message;

-	TSIB1_redcap = 120 ms, is the time period used to acquire SIB1 message;

-	TTI insertion uncertainty = 2 x TTIDCCH = 2ms, is the delay uncertainty when inserting the measurement report to the TTI of the uplink DCCH;

-	additional margin for FR2 = 20 ms, is specified in 38.133 [6] clause 9.11A.3

Which gives overall delay = 500 + 120 + 20 + 2 = 642 ms, allow 645 ms.

From the start of T2 until 645 ms. The number of missed ACK/NACK shall be no more than the maximum number of missed ACK/NACK given in Table 17.6.4.1.5-2.

The rate of correct events observed during repeated tests shall be at least 90%.

Table 17.6.4.1.5-2: Maximum number of missed ACK/NACKs during the overall delay

| Test config. | Maximum number of missed ACK/NACKs |
| --- | --- |
| 1 | 1240 Note 1 |
| NOTE 1: The overall missed ACK/NACK number is caused by two parts. Firstly, up to 25 interruptions, each has a length of up to 48 slots, are allowed during TMIB_redcap as specified in 38.133 [6] Table 9.11A.4-1. Secondly, up to 6 interruptions, each has a length of up to 49 slots are allowed during TSIB1_redcap as specified in 38.133 [6] Table 9.11A.4-1 assuming SSB/CORESET0 multiplxing pattern #0. In worst case, they will cause up to ceil(48/5)*4*25 + ceil(49/5)*4*6 = 1240 missed ACK/NACKs (assuming TDDConf.3.1 is used). |  |

# 18	E-UTRA - NR inter-RAT with E-UTRA serving cell

## 18.1	RRC_IDLE state mobility

### 18.1.1	Inter-rat NR cell re-selection

#### 18.1.1.0	Minimum conformance requirements

TBD

#### 18.1.1.1	E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing.

-	Test procedure is TBD

-	Test applicability is missing

-	TT analysis is missing.

-	Cell mapping is missing

18.1.1.1.1	Test purpose

The purpose of this test is to verify the requirement for the E-UTRA - NR SA FR1 inter-RAT cell reselection for RedCap UEs. The requirements are specified in TS 38.133 [6] in clause TBD.

18.1.1.1.2	Test applicability

This test applies to all types of NR RedCap UE release 17 and forward.

18.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 18.1.1.0.x.

The normative reference for this requirement is TS 38.133 [6] clause A.18.1.1.1.

18.1.1.1.4	Test description

18.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 18.1.1.1.4.1-1.

Table 18.1.1.1.4.1-1: Supported test configurations for E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1

| Configuration | Description |
| --- | --- |
| 18.1.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.1.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.1.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 18.1.1.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.1.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.1.1.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 18.1.1.1-7 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 18.1.1.1-8 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 18.1.1.1.4.1-2.

Table 18.1.1.1.4.1-2: Initial conditions for E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 18.1.1.1.4.1-3.

2.	Message contents are defined in clause 18.1.1.1.4.3.

3.	The test scenario comprises of 1 E-UTRA cell and 1 NR cell. E-UTRA cell 1 is already identified by the UE prior to the start of the test. Cell 2 is of higher priority than cell 1.

Table 18.1.1.1.4.1-3: General test parameters for E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE camps on cell 2 in the initial phase |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 |  |
| T1 end condition | Active cell |  |  | Cell1 | During T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  |  | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 4, 7, 8 | 3 ms | Asynchronous cells |
|  |  |  | 2, 5 | 3 s | Synchronous cells |
|  |  |  | 3, 6 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

18.1.1.1.4.2	Test procedure

TBD

18.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

TBD

18.1.1.1.5	Test requirement

Table 18.1.1.1.5-1 defines the primary level settings including test tolerances for NR – E-UTRA cell re-selection test case in AWGN for 2 Rx UE.

Table 18.1.1.1.5-1: NR cell 2 specific test parameters for E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4, 7, 8 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1, 4, 7, 8 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1, 4, 7, 8 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel |  | 1, 4, 7, 8 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 |  |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | SMTC.1 |  |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.1 RedCap FR1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6, 7, 8 | SS-RSRP |  |  |
|  | dB | 1, 4, 7, 8 | -4 | -infinity | 12 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Note2 | dBm/SCS | 1, 4, 7, 8 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
| Note2 | dBm/15 kHz | 1, 4, 7, 8 | -98 |  |  |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4, 7, 8 | -4 | -infinity | 12 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4, 7, 8 | -102 | -infinity | -86 |
|  |  | 2, 5 | -102 | -infinity | -86 |
|  |  | 3, 6 | -99 | -infinity | -83 |
| Io | dBm/9.36 MHz | 1, 4, 7, 8 | -68.60 | -70.05 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -70.05 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -63.95 | -51.69 |
| Treselection | s | 1, 2, 3, 4, 5, 6, 7, 8 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 18.1.1.1.5-2: E-UTRA cell 1 specific test parameters for E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -84 |
|  | dB | 14 | 14 | 14 |
|  | dB | 14 | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | 50 |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

The cell reselection delay to a higher priority NR cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR + TSI-NR,

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

## 18.2	RRC_CONNECTED state mobility for RedCap

### 18.2.1	Handover for RedCap

#### 18.2.1.0	Minimum conformance requirements

Then minimum requirements given in clause 16.3.1.0.2 apply.

The normative reference for this requirement is TS 36.133 [23] clause 5.3.4B.

#### 18.2.1.1	E-UTRA - NR FR1 handover  for 2Rx UE

18.2.1.1.1	Test purpose

To verify the E-UTRAN to NR FR1 handover requirements for 2RX RedCap as specified in TS 36.133 [23] clause 5.3.4B.

18.2.1.1.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards and capable of NR RedCap with 2Rx.

18.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 18.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.18.2.1.1.

18.2.1.1.4	Test description

Same as the test description given in clause 8.3.1.1.4.

18.2.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 8.3.1.1.4.1 with following exceptions:

-	Table 8.3.1.1.4.1-1 is replaced by Table 18.2.1.1.4.1-1.

-	Table 8.3.1.1.4.1-2 is replaced by Table 18.2.1.1.4.1-2.

-	Table 8.3.1.1.4.1-3 is replaced by Table 18.2.1.1.4.1-3.

Table 18.2.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.2.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.2.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.2.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.2.1.1-4 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 18.2.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.2.1.1-6 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.2.1.1-7 | LTE TDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.2.1.1-8 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 18.2.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.2.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | E-UTRAN cell |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | -83 | Absolute E-UTRAN RSRP threshold for event B2 |
| b2-Threshold2NR |  | dBm | As specified in Table 18.2.1.1.5-2 | Absolute NR SS-RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts [23] |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

18.2.1.1.4.2	Test procedure

Same as the test procedure given in clause 8.3.1.1.4.2.

18.2.1.1.4.3	Message contents

Same as the message contents given in clause 8.3.1.1.4.3 with following exceptions.

-	Table 7.3.1-3 in TS 38.508-1 [14] is used with condition SMTC.1 RedCap.

18.2.1.1.5	Test requirement

Same as the test requirements given in clause 8.3.1.1.5 with following exceptions:

-	Table 8.3.1.1.5-1 is replaced by Table 18.2.1.1.5-1.

-	Table 8.3.1.1.5-2 is replaced by Table 18.2.1.1.5-2.

Table 18.2.1.1.5-1: Cell specific test parameters for E-UTRA PCell (Cell 1)

| Parameter | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |  |
| Duplex mode |  | 1,2,3,4 | FDD |  |  |
|  |  | 5,6,7,8 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 5,6,7,8 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 5,6,7,8 | 1 |  |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1,2,3,4 | 4 |  |  |
|  |  | 5,6,7,8 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,4 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 5,6,7,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,4 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 5,6,7,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1,2,3,4 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 5,6,7,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | 7 | 5.45 Note 8 | 5.45 Note 8 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | 7 | 5.45 | 5.45 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -91 | -92.55 | -92.55 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -91 | -92.55 | -92.55 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -62.43 | -63.68 | -63.68 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. NOTE 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. NOTE 8: Including test tolerance given in Annex F. |  |  |  |  |  |

Table 18.2.1.1.5-2: NR neighbour Cell specific test parameters for NR neighbour (Cell 2)

| Parameter |  | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |  |
| Duplex mode |  |  | 1,5 | FDD |  |  |
|  |  |  | 2,3,6,7 | TDD |  |  |
|  |  |  | 4.8 | HD-FDD |  |  |
| TDD Configuration |  |  | 2 6 | TDDConf.1.1 |  |  |
|  |  |  | 3,7 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1,4,5,8 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2,6 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3,7 | 20: NRB,c = 51 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1,4,5,8 | SR.1.1 FDD |  |  |
|  |  |  | 2,6 | SR.1.1 TDD |  |  |
|  |  |  | 3,7 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1,4,5,8 | CR.1.1 FDD |  |  |
|  |  |  | 2,6 | CR.1.1 TDD |  |  |
|  |  |  | 3,7 | CR.2.1 TDD |  |  |
| PRACH configuration |  |  | 1,2,3,4,5,6,7,8 | PRACH.1 FR1 |  |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 RedCap |  |  |
| SSB configuration |  |  | 1,2,4,5,6,8 | SSB.1 RedCap FR1 |  |  |
|  |  |  | 3,7 | SSB.2 RedCap FR1 |  |  |
| b2-Threshold2NR |  | dBm | 1,2,4,5,6,8 | -106 |  |  |
|  |  |  | 3,7 | -103 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -98 |  |  |
| NocNote2 |  | dBm/SCS | 1,2,4,5,6,8 | -98 |  |  |
|  |  |  | 3,7 | -95 |  |  |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | -infinity | 1.55 Note 4 | 1.55 Note 4 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | -infinity | 1.55 | 1.55 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,6,8 | -infinity | -96.45 | -96.45 |
|  |  |  | 3,7 | -infinity | -93.44 | -93.44 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,6,8 | -70.05 | -66.19 | -66.19 |
|  |  | dBm/18.36 MHz | 3,7 | -67.13 | -63.27 | -63.27 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Including test tolerance given in Annex F. |  |  |  |  |  |  |

### 18.2.2	RRC connection release with redirection for RedCap

#### 18.2.2.0	Minimum conformance requirements

##### 18.2.2.0.1	Redirection from E-UTRA to FR1 RedCap UE

The RedCap UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR_RedCap.

The time delay (Tconnection_release_redirect_NR_RedCap) is the time between the end of the last TTI containing the RRC command, “RRCConnectionRelease” (TS 36.331 [2]) on the E-UTRAN PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR_RedCap) shall be less than:

Tconnection_release_redirect_NR_RedCap = TRRC_procedure_delay + Tidentify-NR_Redcap + TSI-NR_RedCap + TRACH_RedCap

The target NR cell shall be considered detectable for 1 Rx RedCap when for each relevant SSB:

-	SSB_RP and SSB Ês/Iot according to Annex B.2.X of TS 38.133 [6] for a corresponding NR Band.

The target NR cell shall be considered detectable for 2 Rx RedCap when for each relevant SSB:

-	SSB_RP and SSB Ês/Iot according to Annex B.2.X of TS 38.133 [6] for a corresponding NR Band.

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCConnectionRelease” as defined in clause 6.2.2 of TS 36.331 [2]. It shall be less than 110 ms.

Tidentify-NR_RedCap: It is the time to identify the target NR cell and depends on the frequency range (FR) of the target NR cell. It is defined in Table 18.2.2.0-1 for 1 Rx RedCap for FR1 and in Table 18.2.2.0-2 for 2 Rx RedCap. Tidentify-NR_RedCap = TPSS/SSS-sync_RedCap + Tmeas_RedCap, where TPSS/SSS-sync_RedCap is the cell search time and Tmeas_RedCap is the measurement time due to cell selection criteria evaluation. The RedCap UE operating in NR HD-FDD shall meet the requirements in this section provided that the SSB is available at the UE at least once every SMTC period during Tidentify-NR_RedCap.

TSI-NR_RedCap: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released.

TRACH_RedCap: It is the delay caused due to the random access procedure when sending random access to the target NR cell. TRACH_RedCap can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [39]. The RedCap UE operating in NR HD-FDD mode shall perform the PRACH transmission on the PRACH resource of the target NR cell provided that the UE has received at least one SSB associated with the PRACH resource during the last [TBD] period in the target NR cell.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

-	the requirement in this section is applied with Trs = 20 ms assuming the SSB transmission periodicity is not larger than 20 ms,

-	there is no requirement if the SSB transmission periodicity is larger than 20 ms.

Table 18.2.2.0-1: Time to identify target NR cell for RRC connection release with redirection to NR for 1 Rx RedCap

| Frequency range (FR) of target NR cell | Tidentify-NR_RedCap |
| --- | --- |
| FR1 | MAX (680 ms, 12 x Trs) |

Table 18.2.2.0-2: Time to identify target NR cell for RRC connection release with redirection to NR for 2 Rx RedCap

| Frequency range (FR) of target NR cell | Tidentify-NR_RedCap |
| --- | --- |
| FR1 | MAX (680 ms, 11 x Trs) |
| FR2 | MAX (880 ms, 88 x Trs) |

The normative reference for this requirement is TS 36.133 [23] clause 6.3.2.6.

#### 18.2.2.1	Redirection from E-UTRA to NR SA FR1 for redcap UE

18.2.2.1.1	Test purpose

To verify RRC connection release with redirection from E-UTRA to NR requirements for 2Rx RedCap specified in 36.133 [23] clause 6.3.2.6.

18.2.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

18.2.2.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 18.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.18.2.2.1.

18.2.2.1.4	Test description

18.2.2.1.4.1	Initial conditions

This test can be run in the configurations defined in Table 18.2.2.1.4.1-1

Table 18.2.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.2.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 18.2.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 18.2.2.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 18.2.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 18.2.2.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 18.2.2.1-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 18.2.2.1-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 18.2.2.1-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 18.2.2.1.4.1-2.

Table 18.2.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameters settings are given in Table 18.2.2.1.4.1-3 below.

2. Message contents are defined in clause 18.2.2.1.4.3.

3. There are two cells specified in the test. Cell 1 is the E-UTRA PCell and Cell 2 is the NR neighbour cell. Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1, Cell 2 is configured according to Annex C.1.1 and C.1.2,

Table 18.2.2.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | E-UTRA Cell |
|  | Neighbouring cell |  | Cell 2 | NR Cell |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

18.2.2.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCConnectionRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to T1 in Table 18.2.2.1.5-1 for Cell 1 and 18.2.2.1.5-2 for Cell 2. T1 starts.

3.	SS shall transmit an RRCConnectionRelease with redirectedCarrierInfo during period T1.

4.	The SS shall start T2 timer when the last TTI containing the RRCConnectionRelease message is sent to UE.

5.	When T2 starts, the SS shall switch the power setting from T1 to T2 as specified in Table 18.2.2.1.5-1 for Cell 1 and 18.2.2.1.5-2 for Cell 2.

6.	The UE shall transmit an RRCSetupRequest message.

7.	If the UE transmits the PRACH to Cell 2 less than 2205 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T2 expires, the UE shall be switched off. Then ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3. Cell 1 is the active cell and Cell 2 shall be powered OFF.

9.	The SS shall set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14+2) for next iteration of the test procedure loop.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

18.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 18.2.2.1.4.3-1: RRCConnectionRelease (Test procedure, Step 3)

| Derivation Path: 36.508 [25] Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| nr-r15 SEQUENCE { |  |  |  |
| carrierFreq-r15 | ARFCN-ValueNR-r15 of Cell 2 |  |  |
| subcarrierSpacingSSB-r15 | khz15 |  | Config 1,2,4,5,7,8 |
|  | Khz30 |  | Config 3,6 |
| smtc-r15 SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf40-r15 | 0 |  |  |
| } |  |  |  |
| ssb-Duration-r15 | sf1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

18.2.2.1.5	Test requirement

Table 18.2.2.1.4.3-1 and Table 18.2.2.1.4.3-2 define the primary level settings for redirection from E-UTRA to NR RedCap test case.

Table 18.2.2.1.5-1: E-UTRA Cell specific test parameters (Cell 1)

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |
|  |  | 4,5,6,8 | TDD |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1,2,3,7 | 4 |  |
|  |  | 4,5,6,8 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,7 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement |  | 1,2,3,7 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
| ChannelNote3 |  | 4,5,6,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1,2,3,7 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA |  |  |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | 4 | 4 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | 4 | 4 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -94 | -94 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -94 | -94 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -64.76 | -64.76 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table 18.2.2.1.5-2: NR Cell specific test parameters (Cell 2)

| Parameter |  |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |
|  |  | Config 2,3,5,6 |  | TDD |  |
|  |  | Config 7,8 |  | HD-FDD |  |
| SSB Configuration |  | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |  |
|  |  | Config 3,6 |  | SSB.1 RedCap FR1 |  |
| TDD configuration |  | Config 1,4,7,8 |  | Not Applicable |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 3,6 |  | 20: NRB,c = 51 |  |
| BWP BW |  | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 3,6 |  | 20: NRB,c = 51 |  |
| DRX Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference |  | Config 1,4,7,8 |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2,5 |  | SR.1.1 TDD |  |
|  |  | Config 3,6 |  | SR.2.1 TDD |  |
| CORESET Reference |  | Config 1,4,7,8 |  | CR.1.1 FDD |  |
| Channel |  | Config 2,5 |  | CR.1.1 TDD |  |
|  |  | Config 3,6 |  | CR.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC configuration |  | Config 1,2,4,5,7,8 |  | SMTC.1 RedCap FR1 |  |
|  |  | Config 3,6 |  | SMTC.1 RedCap FR1 |  |
| PDSCH/PDCCH |  | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
| subcarrier spacing |  | Config 3,6 |  | 30 kHz |  |
| PUCCH/PUSCH |  | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |
| subcarrier spacing |  | Config 3,6 |  | 30 kHz |  |
| PRACH configuration |  |  |  | PRACH.1 FR1 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4,5,7,8 |  | dBm/SCS | -98 |  |
|  | Config 3,6 |  |  | -95 |  |
|  |  |  | dB | -infinity | 4 |
|  |  |  | dB | -infinity | 4 |
| IoNote3 | Config 1,2,4,5,7,8 |  | dBm/ 9.36MHz | -70.05 | -64.59 |
|  | Config 3,6 |  | dBm/ 18.36MHz | -67.12 | -61.67 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR_RedCap = TRRC_procedure_delay + Tidentify-NR_Redcap + TSI-NR_RedCap + TRACH_RedCap

where:

TRRC_procedure_delay = 110 ms and is specified in clause 12 in TS 38.331 [13].

Tidentify-NR = 680 ms in the test.

TSI-NR_RedCap = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331[13] for the target NR cell.

TRACH_RedCap = 170 ms in the test.

This gives a total of 2240 ms.

## 18.3	Measurement procedure for RedCap

### 18.3.1	E-UTRA - NR Measurements for RedCap

#### 18.3.1.0	Minimum conformance requirements

##### 18.3.1.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement

The UE shall be able to identify new inter-RAT NR cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-RAT cells if carrier frequency information is provided by the PCell, even if no explicit neighbour list with physical layer cell identities is provided.

Requirements in this clause are applicable to both E-UTRA FDD and E-UTRA TDD serving cell.

When measurement gaps are scheduled, the RedCap UE shall be able to identify a new detectable cell within Tidentify_irat_without_index_RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise, UE shall be able to identify a new detectable inter-RAT cell within Tidentify_irat_with_index_RedCap. The UE shall be able to identify a new detectable inter-RAT SS block of an already detected cell within Tidentify_irat_without_index_RedCap.

Tidentify_irat_without_index_RedCap = (TPSS/SSS_sync_irat_RedCap + T SSB_measurement_period_irat_RedCap) ms

Tidentify_irat_with_index_RedCap = (TPSS/SSS_sync_irat_RedCap + T SSB_measurement_period_irat_RedCap + TSSB_time_index_irat_RedCap) ms

Where:

TPSS/SSS_sync_irat_RedCap: it is the time period used in PSS/SSS detection given in Table 18.3.1.0.1-1 and Table 18.3.1.0.1-3 for 2 Rx RedCap, Table 18.3.1.0.1-2 for 1 Rx RedCap.

TSSB_time_index_irat_RedCap: it is the time period used to acquire the index of the SSB being measured given in Table 18.3.1.0.1-4 and Table 18.3.1.0.1-6 for 2 Rx RedCap, Table 18.3.1.0.1-5 for 1 Rx RedCap.

TSSB_measurement_period_irat_RedCap: equal to a measurement period of SSB based measurement given in Table 18.3.1.0.1-7 and Table 18.3.1.0.1-9 for 2 Rx RedCap, Table 18.3.1.0.1-8 for 1 Rx RedCap.

Mpss/sss_sync_irat_RedCap: For a 2 Rx RedCap supporting FR2 power class 7, Mpss/sss_sync_irat_RedCap = 40 samples.

MSSB_index_irat_RedCap: For a 2 Rx RedCap supporting FR2 power class 7, MSSB_index_irat_RedCap = 40 samples.

Mmeas_period_irat_RedCap: For a 2 Rx RedCap supporting FR2 power class 7, Mmeas_period_irat = 40 samples.

Nfreq_RedCap is defined in 38.133 [6] clause 8.1.2.1.1d.

Table 18.3.1.0.1-1: Time period for PSS/SSS detection for 2 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TPSS/SSS_sync_irat_RedCap |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(81.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 8 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 18.3.1.0.1-2: Time period for PSS/SSS detection for 1 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TPSS/SSS_sync_irat_RedCap |
| --- | --- |
| No DRX | Max(600ms, 7 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(7 *1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 7 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 18.3.1.0.1-3: Time period for PSS/SSS detection for 2 Rx RedCap (Frequency range FR2)

| Condition NOTE1 | TPSS/SSS_sync_irat_RedCap |
| --- | --- |
| No DRX | Max(600ms, Mpss/sss_sync_irat_RedCap Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, (1.5 Mpss/sss_sync_irat_RedCap) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | Mpss/sss_sync_irat_RedCap DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 18.3.1.0.1-4: Time period for time index detection for 2 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_irat_RedCap |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 3 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 18.3.1.0.1-5: Time period for time index detection for 1 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_irat_RedCap |
| --- | --- |
| No DRX | Max(160ms, 6 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(160ms, Ceil(6 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 6 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 18.3.1.0.1-6: Time period for time index detection for 2 Rx RedCap (Frequency range FR2)

| Condition NOTE1 | TSSB_time_index_irat_RedCap |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_irat_RedCap Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 MSSB_index_irat_RedCap) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | MSSB_index_irat_RedCap DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

In the requirements, an NR cell is considered detectable when:

-	NR SS-RSRP related conditions in the accuracy requirements in Section 9.11.x are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50],

-	NR SS-RSRQ related conditions in the accuracy requirements in Section 9.11.y are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50],

-	NR SS-SINR related conditions in the accuracy requirements in Section 9.11.z are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50].

The RedCap UE operating in NR HD-FDD shall meet the current PSS/SSS detection requirements provided that at least 5 SMTC windows are available at the UE during the PSS/SSS detection time.

The RedCap UE operating in NR HD-FDD shall meet the current time index detection requirements provided that at least 3 SMTC windows are available at the UE during the time index detection time.

When measurement gaps are scheduled for NR measurements the UE physical layer shall be capable of reporting NR SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clause 9.11, with measurement period as shown in table 8.20.2.1-5 and 8.20.2.1-6 for 2Rx UE, table 8.20.2.1-5a for 1Rx UE:

Table 8.20.2.1-5: Measurement period for inter-RAT measurements for 2 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TSSB_measurement_period_irat_RedCap |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 8 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-5a: Measurement period for inter-RAT measurements for 1 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TSSB_measurement_period_irat_RedCap |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 8 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-6: Measurement period for inter-RAT measurements for 2 Rx RedCap (Frequency range FR2)

| Condition NOTE1 | TSSB_measurement_period_irat_RedCap |
| --- | --- |
| No DRX | Max(400ms, Mmeas_period_irat_RedCap Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(400ms, (1.5 Mmeas_period_irat_RedCap) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | Mmeas_period_irat_RedCap DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

The RedCap UE operating in NR HD-FDD shall meet the current measurement requirements provided that at least 5 SMTC windows are available at the UE during the measurement period.

Reported measurements in event triggered measurement reports shall meet the requirements in clause 9.

The UE shall not send any event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The measurement reporting delay is defined as the time between any event that will trigger a measurement report until the UE starts to transmit the measurement report over the Uu interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is twice the TTI of the uplink DCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_irat_without_index_RedCap or Tidentify_irat_with_index_RedCap for the minimum requirements.

If an NR cell which has been detectable at least for the time period Tidentify_irat_without_index_RedCap. or Tidentify_irat_with_index_RedCap defined in clause 8.20.2.1 for the minimum requirements and then triggers the measurement report as per TS 36.331 [2], the event triggered measurement reporting delay shall be less than TSSB_measurement_period_irat_RedCap defined in clause 8.20.2.1 provided the timing to that cell has not changed more than 3200 Tc while measurement gap has not been available and the L3 filter has not been used.

#### 18.3.1.1	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

18.3.1.1.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

18.3.1.1.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards and capable of NR RedCap with 2Rx. Test 1 applies to UEs not supporting per-FR gap and test 2 applies only to UEs supporting per-FR gap and gap pattern #4.

18.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 18.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.18.3.1.1.

18.3.1.1.4	Test description

Same as the test description given in clause 8.4.2.1.4.

18.3.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 8.4.2.1.4.1 with following exceptions:

-	Table 8.4.2.1.4.1-1 is replaced by Table 18.3.1.1.4.1-1.

-	Table 8.4.2.1.4.1-2 is replaced by Table 18.3.1.1.4.1-2.

-	Table 8.4.2.1.4.1-3 is replaced by Table 18.3.1.1.4.1-3.

Table 18.3.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.3.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.3.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.3.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.3.1.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.3.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.3.1.1-6 | LTE TDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.3.1.1-7 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 18.3.1.1-8 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table 18.3.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.16-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.3.1.1.4.1-1. |  |  |
| Propagation conditions | Multi-path fading |  | As specified in clause C.2.3 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.3.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  |  | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  |  | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  |  | E-UTRA cell 1 |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  |  | 0 | 4 | As specified in Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  |  | 39 | 19 | As specified in TS 36.331 [29]. |
| Hysteresis |  | dB | 0 |  |  |
| CP length |  |  | Normal |  |  |
| TimeToTrigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells | Config 1,4,7,8 |  | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | Config 2,3,5,6 |  | 3s |  | Synchronous cells. |
| T1 |  | s | 5 |  |  |
| T2 |  | s | 1 |  |  |

18.3.1.1.4.2	Test procedure

Same as the test procedure given in clause 8.4.2.1.4.2

18.3.1.1.4.3	Message contents

Same as the message contents given in clause 8.4.2.1.4.3 with following exceptions:

-	The specific message contents exceptions for test configuration 8.4.2.1-1 and 8.4.2.1-4 also apply to test configuration 8.4.2.1-7 and 8.4.2.1-8.

-	Table 8.4.2.1.4.3-2 is replaced by table 18.3.1.1.4.3-1.

Table 18.3.1.1.4.3-1: SchedulingRequest-Config-DEFAULT

| Derivation Path: TS 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 18.3.1.1-1/2/3/7 |
|  | 22 |  | Config 18.3.1.1-4/5/6/8 |
| } |  |  |  |
| } |  |  |  |

18.3.1.1.5	Test requirement

Same as the test requirements given in clause 8.4.2.1.5 with following exceptions:

-	Table 8.4.2.1.5-1 is replaced by Table 18.3.1.1.5-1.

-	Table 8.4.2.1.5-2 is replaced by Table 18.3.1.1.5-2.

Table 18.3.1.1.5-1: E-UTRAN Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  |  | 1 |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |
|  | Config 4,5,6,8 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 4,5,6,8 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 4,5,6,8 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 |  | dBm | -77 |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 |  | dBm/15 kHz | -104 |  |
| Ês/Noc |  | dB | 17 | 17 |
| Ês/IotNote5 |  | dB | 17 | 17 |
| RSRPNote5 |  | dBm/15 kHz | -87 | -87 |
| SCH_RPNote5 |  | dBm/15 kHz | -87 | -87 |
| IoNote5 |  | dBm/ 9 MHz | -56.12 | -56.12 |
| Propagation Condition Note6 |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 18.3.1.1.5-2: NR cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1,4 |  | FDD |  |
|  | Config 2,3,5,6 |  | TDD |  |
|  | Config 7,8 |  | HD-FDD |  |
| TDD configuration | Config 2,5 |  | TDDConf.1.1 |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 20: NRB,c = 51 |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1,4,7,8 |  | SMTC.2 |  |
|  | Config 2,3,5,6 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5,7,8 | kHz | 15 |  |
|  | Config 3,6 |  | 30 |  |
| b2-Threshold2NR | Config 1,2,4,5,7,8 | dBm/SCS | -101 |  |
|  | Config 3,6 |  | -98 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4,5,7,8 | dBm/SCS | -98 |  |
|  | Config 3,6 |  | -95 |  |
| SS-RSRP Note 3 | Config 1,2,4,5,7,8 | dBm/SCS | -Infinity | -91 |
|  | Config 3,6 |  | -Infinity | -88 |
|  |  | dB | -Infinity | 7 |
|  |  | dB | -Infinity | 7 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36MHz | -70.05 | -62.26 |
|  | Config 3,6 | dBm/18.36MHz | -67.12 | -59.33 |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

#### 18.3.1.2	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used

18.3.1.2.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

18.3.1.2.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards supporting NR RedCap with 2Rx and long DRX cycle. Test 1 and test 2 apply to UEs not supporting per-FR gap. Test 3 and test 4 apply only to UEs supporting per-FR gap and gap pattern #4.

18.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 18.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.18.3.1.2.

18.3.1.2.4	Test description

Same as the test description given in clause 8.4.2.2.4.

18.3.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 8.4.2.2.4.1 with following exceptions:

-	Table 8.4.2.2.4.1-1 is replaced by Table 18.3.1.2.4.1-1.

-	Table 8.4.2.2.4.1-2 is replaced by Table 18.3.1.2.4.1-2.

-	Table 8.4.2.2.4.1-3 is replaced by Table 18.3.1.2.4.1-3.

Table 18.3.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.3.1.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.3.1.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.3.1.2-3 | LTE FDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.3.1.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.3.1.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.3.1.2-6 | LTE TDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.3.1.2-7 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 18.3.1.2-8 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table 18.3.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.16-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.3.1.2.4.1-1. |  |  |
| Propagation conditions | Multi-path fading |  | As specified in clause C.2.3 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.3.1.2.4.1-3: General test parameters

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Numbers |  |  | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  |  | 1 |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  |  | E-UTRA Cell 1 |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | NR Cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  |  | 0 |  | 4 |  | As specified in TS 36.133 [23] Table 8.1.2.1-1. |
| Measurement gap offset |  |  | 39 |  | 19 |  | As specified in TS 36.331 [29]. |
| Hysteresis |  | dB | 0 |  |  |  |  |
| CP length |  |  | Normal |  |  |  |  |
| TimeToTrigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | DRX.9 | DRX.12 | DRX.9 | DRX.12 | DRX is not used |
| Time offset between serving and neighbour cells | Config 1,4,7,8 |  | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | Config 2,3,5,6 |  | 3s |  |  |  | Synchronous cells. |
| T1 |  | s | 5 |  |  |  |  |
| T2 |  | s | 2 | 11 | 2 | 11 |  |

18.3.1.2.4.2	Test procedure

Same as the test procedure given in clause 8.4.2.2.4.2

18.3.1.2.4.3	Message contents

Same as the message contents given in clause 8.4.2.2.4.3 with following exceptions:

-	The specific message contents exceptions for test configuration 8.4.2.2-1 and 8.4.2.2-4 also apply to test configuration 8.4.2.2-7 and 8.4.2.2-8.

-	Table 8.4.2.2.4.3-2 is replaced by table 18.3.1.2.4.3-1.

Table 18.3.1.2.4.3-1: SchedulingRequest-Config-DEFAULT

| Derivation Path: TS 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 18.3.1.2-1/2/3/7 |
|  | 22 |  | Config 18.3.1.2-4/5/6/8 |
| } |  |  |  |
| } |  |  |  |

18.3.1.2.5	Test requirement

Same as the test requirements given in clause 8.4.2.2.5 with following exceptions:

-	Table 8.4.2.2.5-1 is replaced by Table 18.3.1.2.5-1.

-	Table 8.4.2.2.5-2 is replaced by Table 18.3.1.2.5-2.

Table 18.3.1.2.5-1: E-UTRAN Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  |  | 1 |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |
|  | Config 4,5,6,8 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 4,5,6,8 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 4,5,6,8 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 |  | dBm | -77 |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 |  | dBm/15 kHz | -104 |  |
| Ês/Noc |  | dB | 17 | 17 |
| Ês/IotNote5 |  | dB | 17 | 17 |
| RSRPNote5 |  | dBm/15 kHz | -87 | -87 |
| SCH_RPNote5 |  | dBm/15 kHz | -87 | -87 |
| IoNote5 |  | dBm/ 9 MHz | -56.12 | -56.12 |
| Propagation Condition Note6 |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 18.3.1.2.5-2: NR cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1,4 |  | FDD |  |
|  | Config 2,3,5,6 |  | TDD |  |
|  | Config 7,8 |  | HD-FDD |  |
| TDD configuration | Config 2,5 |  | TDDConf.1.1 |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 20: NRB,c = 51 |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1,4,7,8 |  | SMTC.2 |  |
|  | Config 2,3,5,6 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5,7,8 | kHz | 15 |  |
|  | Config 3,6 |  | 30 |  |
| b2-Threshold2NR | Config 1,2,4,5,7,8 | dBm/SCS | -101 |  |
|  | Config 3,6 |  | -98 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4,5,7,8 | dBm/SCS | -98 |  |
|  | Config 3,6 |  | -95 |  |
| SS-RSRP Note 3 | Config 1,2,4,5,7,8 | dBm/SCS | -Infinity | -91 |
|  | Config 3,6 |  | -Infinity | -88 |
|  |  | dB | -Infinity | 7 |
|  |  | dB | -Infinity | 7 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36MHz | -70.05 | -62.26 |
|  | Config 3,6 | dBm/18.36MHz | -67.12 | -59.33 |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

#### 18.3.1.3	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

18.3.1.3.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

18.3.1.3.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards and capable of NR RedCap with 2Rx. Test 1 applies to UEs not supporting per-FR gap and test 2 applies only to UEs supporting per-FR gap and gap pattern #4.

18.3.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 18.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.18.3.1.3.

18.3.1.3.4	Test description

Same as the test description given in clause 8.4.2.3.4.

18.3.1.3.4.1	Initial conditions

Same as the initial conditions given in clause 8.4.2.3.4.1 with following exceptions:

-	Table 8.4.2.3.4.1-1 is replaced by Table 18.3.1.3.4.1-1.

-	Table 8.4.2.3.4.1-2 is replaced by Table 18.3.1.3.4.1-2.

-	Table 8.4.2.3.4.1-3 is replaced by Table 18.3.1.3.4.1-3.

Table 18.3.1.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.3.1.3-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.3.1.3-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.3.1.3-3 | LTE FDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.3.1.3-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.3.1.3-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.3.1.3-6 | LTE TDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.3.1.3-7 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 18.3.1.3-8 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table 18.3.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.16-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.3.1.3.4.1-1. |  |  |
| Propagation conditions | Multi-path fading |  | As specified in clause C.2.3 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.3.1.3.4.1-3: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  |  | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  |  | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  |  | E-UTRA cell 1 |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  |  | 0 | 4 | As specified in Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  |  | 39 | 19 | As specified in TS 36.331 [29]. |
| Hysteresis |  | dB | 0 |  |  |
| CP length |  |  | Normal |  |  |
| TimeToTrigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells | Config 1,4,7,8 |  | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | Config 2,3,5,6 |  | 3s |  | Synchronous cells. |
| T1 |  | s | 5 |  |  |
| T2 |  | s | 2 | 1 |  |

18.3.1.3.4.2	Test procedure

Same as the test procedure given in clause 8.4.2.3.4.2.

18.3.1.3.4.3	Message contents

Same as the message contents given in clause 8.4.2.3.4.3 with following exceptions:

-	The specific message contents exceptions for test configuration 8.4.2.3-1 and 8.4.2.3-4 also apply to test configuration 8.4.2.3-7 and 8.4.2.3-8.

-	Table 8.4.2.3.4.3-2 is replaced by table 18.3.1.3.4.3-1.

Table 18.3.1.3.4.3-1: SchedulingRequest-Config-DEFAULT

| Derivation Path: TS 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 18.3.1.3-1/2/3/7 |
|  | 22 |  | Config 18.3.1.3-4/5/6/8 |
| } |  |  |  |
| } |  |  |  |

18.3.1.3.5	Test requirement

Same as the test requirements given in clause 8.4.2.3.5 with following exceptions:

-	Table 8.4.2.3.5-1 is replaced by Table 18.3.1.3.5-1.

-	Table 8.4.2.3.5-2 is replaced by Table 18.3.1.3.5-2.

Table 18.3.1.3.5-1: E-UTRAN Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  |  | 1 |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |
|  | Config 4,5,6,8 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 4,5,6,8 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 4,5,6,8 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 |  | dBm | -77 |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 |  | dBm/15 kHz | -104 |  |
| Ês/Noc |  | dB | 17 | 17 |
| Ês/IotNote5 |  | dB | 17 | 17 |
| RSRPNote5 |  | dBm/15 kHz | -87 | -87 |
| SCH_RPNote5 |  | dBm/15 kHz | -87 | -87 |
| IoNote5 |  | dBm/ 9 MHz | -56.12 | -56.12 |
| Propagation Condition Note6 |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 18.3.1.3.5-2: NR cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1,4 |  | FDD |  |
|  | Config 2,3,5,6 |  | TDD |  |
|  | Config 7,8 |  | HD-FDD |  |
| TDD configuration | Config 2,5 |  | TDDConf.1.1 |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 20: NRB,c = 51 |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1,4,7,8 |  | SMTC.2 |  |
|  | Config 2,3,5,6 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5,7,8 | kHz | 15 |  |
|  | Config 3,6 |  | 30 |  |
| b2-Threshold2NR | Config 1,2,4,5,7,8 | dBm/SCS | -101 |  |
|  | Config 3,6 |  | -98 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4,5,7,8 | dBm/SCS | -98 |  |
|  | Config 3,6 |  | -95 |  |
| SS-RSRP Note 3 | Config 1,2,4,5,7,8 | dBm/SCS | -Infinity | -91 |
|  | Config 3,6 |  | -Infinity | -91 |
|  |  | dB | -Infinity | 7 |
|  |  | dB | -Infinity | 7 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36MHz | -70.05 | -62.26 |
|  | Config 3,6 | dBm/18.36MHz | -67.12 | -59.33 |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

#### 18.3.1.4	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used

18.3.1.4.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

18.3.1.4.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards supporting NR RedCap with 2Rx and long DRX cycle. Test 1 and test 2 apply to UEs not supporting per-FR gap. Test 3 and test 4 apply only to UEs supporting per-FR gap and gap pattern #4.

18.3.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 18.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.18.3.1.4.

18.3.1.4.4	Test description

Same as the test description given in clause 8.4.2.4.4.

18.3.1.4.4.1	Initial conditions

Same as the initial conditions given in clause 8.4.2.4.4.1 with following exceptions:

-	Table 8.4.2.4.4.1-1 is replaced by Table 18.3.1.4.4.1-1.

-	Table 8.4.2.4.4.1-2 is replaced by Table 18.3.1.4.4.1-2.

-	Table 8.4.2.4.4.1-3 is replaced by Table 18.3.1.4.4.1-3.

Table 18.3.1.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.3.1.4-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.3.1.4-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.3.1.4-3 | LTE FDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.3.1.4-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.3.1.4-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.3.1.4-6 | LTE TDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.3.1.4-7 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 18.3.1.4-8 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table 18.3.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.16-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.3.1.4.4.1-1. |  |  |
| Propagation conditions | Multi-path fading |  | As specified in clause C.2.3 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.3.1.4.4.1-3: General test parameters

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Numbers |  |  | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  |  | 1 |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  |  | E-UTRA Cell 1 |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | NR Cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  |  | 0 |  | 4 |  | As specified in TS 36.133 [23] Table 8.1.2.1-1. |
| Measurement gap offset |  |  | 39 |  | 19 |  | As specified in TS 36.331 [29]. |
| Hysteresis |  | dB | 0 |  |  |  |  |
| CP length |  |  | Normal |  |  |  |  |
| TimeToTrigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | DRX.9 | DRX.12 | DRX.9 | DRX.12 | DRX is not used |
| Time offset between serving and neighbour cells | Config 1,4,7,8 |  | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | Config 2,3,5,6 |  | 3s |  |  |  | Synchronous cells. |
| T1 |  | s | 5 |  |  |  |  |
| T2 |  | s | 2 | 13 | 2 | 13 |  |

18.3.1.4.4.2	Test procedure

Same as the test procedure given in clause 8.4.2.4.4.2.

18.3.1.4.4.3	Message contents

Same as the message contents given in clause 8.4.2.4.4.3 with following exceptions:

-	The specific message contents exceptions for test configuration 8.4.2.4-1 and 8.4.2.4-4 also apply to test configuration 8.4.2.4-7 and 8.4.2.4-8.

-	Table 8.4.2.4.4.3-2 is replaced by table 18.3.1.4.4.3-1.

Table 18.3.1.4.4.3-1: SchedulingRequest-Config-DEFAULT

| Derivation Path: TS 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 18.3.1.4-1/2/3/7 |
|  | 22 |  | Config 18.3.1.4-4/5/6/8 |
| } |  |  |  |
| } |  |  |  |

18.3.1.4.5	Test requirement

Same as the test requirements given in clause 8.4.2.4.5 with following exceptions:

-	Table 8.4.2.4.5-1 is replaced by Table 18.3.1.4.5-1.

-	Table 8.4.2.4.5-2 is replaced by Table 18.3.1.4.5-2.

Table 18.3.1.4.5-1: E-UTRAN Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  |  | 1 |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |
|  | Config 4,5,6,8 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 4,5,6,8 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 4,5,6,8 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 |  | dBm | -77 |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 |  | dBm/15 kHz | -104 |  |
| Ês/Noc |  | dB | 17 | 17 |
| Ês/IotNote5 |  | dB | 17 | 17 |
| RSRPNote5 |  | dBm/15 kHz | -87 | -87 |
| SCH_RPNote5 |  | dBm/15 kHz | -87 | -87 |
| IoNote5 |  | dBm/ 9 MHz | -56.12 | -56.12 |
| Propagation Condition Note6 |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 18.3.1.4.5-2: NR cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1,4 |  | FDD |  |
|  | Config 2,3,5,6 |  | TDD |  |
|  | Config 7,8 |  | HD-FDD |  |
| TDD configuration | Config 2,5 |  | TDDConf.1.1 |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 20: NRB,c = 51 |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1,4,7,8 |  | SMTC.2 |  |
|  | Config 2,3,5,6 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5,7,8 | kHz | 15 |  |
|  | Config 3,6 |  | 30 |  |
| b2-Threshold2NR | Config 1,2,4,5,7,8 | dBm/SCS | -101 |  |
|  | Config 3,6 |  | -98 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2,4,5,7,8 | dBm/SCS | -98 |  |
|  | Config 3,6 |  | -95 |  |
| SS-RSRP Note 3 | Config 1,2,4,5,7,8 | dBm/SCS | -Infinity | -91 |
|  | Config 3,6 |  | -Infinity | -88 |
|  |  | dB | -Infinity | 7 |
|  |  | dB | -Infinity | 7 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36MHz | -70.05 | -62.26 |
|  | Config 3,6 | dBm/18.36MHz | -67.12 | -59.33 |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

#### 18.3.1.5	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is not used

Editor’s Notes: This test case is incomplete:

-	E-UTRA – NR FR2 testability issue is not cleared.

-	The TT analysis is complete for test frequencies f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	This test case is incomplete for UE power classes other than PC3

-	This test case is incomplete for Test frequencies f > 40.8 GHz18.3.1.5.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

18.3.1.5.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards and capable of NR RedCap with 2Rx. Test 1 applies to UEs not supporting per-FR gap and test 2 applies only to UEs supporting per-FR gap.

18.3.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 18.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.18.3.1.5.

18.3.1.5.4	Test description

18.3.1.5.4.1	Initial conditions

Same as the initial conditions given in clause 8.4.2.5.4.1 with following exceptions:

-	Table 8.4.2.5.4.1-1 is replaced by Table 18.3.1.5.4.1-1.

-	Table 8.4.2.5.4.1-2 is replaced by Table 18.3.1.5.4.1-2.

-	Table 8.4.2.5.4.1-3 is replaced by Table 18.3.1.5.4.1-3.

Table 18.3.1.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.3.1.5-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 18.3.1.5-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table 18.3.1.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.16-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.3.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.3.1.5.4.1-3: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  |  | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  |  | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  |  | E-UTRA Cell 1 |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | NR Cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  |  | 0 | N/A | As specified in Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  |  | 39 | N/A | As specified in TS 36.331 [29]. |
| Hysteresis |  | dB | 0 |  |  |
| CP length |  |  | Normal |  |  |
| TimeToTrigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells | Config 1 |  | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | Config 2 |  | 3s |  | Synchronous cells. |
| T1 |  | s | 10 |  |  |
| T2 |  | s | 6 | 3 |  |

18.3.1.5.4.2	Test procedure

Same as the test procedure given in clause 8.4.2.5.4.2.

18.3.1.5.4.3	Message contents

Same as the message contents given in clause 8.4.2.5.4.3.

18.3.1.5.5	Test requirement

Same as the test requirements given in clause 8.4.2.5.5 with following exceptions:

-	Table 8.4.2.5.5-1 is replaced by Table 18.3.1.5.5-1.

Table 18.3.1.5.5-1: NR cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  |  | Setup 2a |  |
| Assumption for UE beamsNote 3 |  |  | Rough |  |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode |  |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 24 |  |
| OCNG patterns |  |  | OP.3 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | -118 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Ês |  | dBm/SCS | -Infinity | -80.6 |
| SSB_RP Note 2 |  | dBm/SCS | -Infinity | -80.6 |
| Ês/IotBB Note 4 |  | dB | -Infinity | 8.3 |
| IoNote3 |  | dBm/95.04MHz | -Infinity | -56.0 |
| Propagation Condition |  |  | AWGN |  |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |

#### 18.3.1.6	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is used

Editor’s Notes: This test case is incomplete:

-	E-UTRA – NR FR2 testability issue is not cleared.

-	The TT analysis is complete for test frequencies f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	This test case is incomplete for UE power classes other than PC3

-	This test case is incomplete for Test frequencies f > 40.8 GHz

18.3.1.6.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

18.3.1.6.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards and capable of NR RedCap with 2Rx. Test 1 and test 2 apply to UEs not supporting per-FR gap. Test 3 and test 4 apply only to UEs supporting per-FR gap.

18.3.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 18.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.18.3.1.6.

18.3.1.6.4	Test description

18.3.1.6.4.1	Initial conditions

Same as the initial conditions given in clause 8.4.2.6.4.1 with following exceptions:

-	Table 8.4.2.6.4.1-1 is replaced by Table 18.3.1.6.4.1-1.

-	Table 8.4.2.6.4.1-2 is replaced by Table 18.3.1.6.4.1-2.

-	Table 8.4.2.6.4.1-3 is replaced by Table 18.3.1.6.4.1-3.

Table 18.3.1.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.3.1.6-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 18.3.1.6-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table 18.3.1.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.16-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.3.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.3.1.6.4.1-3: General test parameters

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Numbers |  |  | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  |  | 1 |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  |  | E-UTRA Cell 1 |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | NR Cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  |  | 0 |  | N/A |  | As specified in Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  |  | 39 |  | N/A |  | As specified in TS 36.331 [29]. |
| Hysteresis |  | dB | 0 |  |  |  |  |
| CP length |  |  | Normal |  |  |  |  |
| TimeToTrigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | DRX.9 | DRX.12 | DRX.9 | DRX.12 |  |
| Time offset between serving and neighbour cells | Config 1 |  | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | Config 2 |  | 3s |  |  |  | Synchronous cells. |
| T1 |  | s | 10 |  |  |  |  |
| T2 |  | s | 6 | 83 | 6 | 83 |  |

18.3.1.6.4.2	Test procedure

Same as the test procedure given in clause 8.4.2.6.4.2.

18.3.1.6.4.3	Message contents

Same as the message contents given in clause 8.4.2.6.4.3.

18.3.1.6.5	Test requirement

Same as the test requirements given in clause 8.4.2.6.5 with following exceptions:

-	Table 8.4.2.6.5-1 is replaced by Table 18.3.1.6.5-1.

Table 18.3.1.6.5-1: NR cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  |  | Setup 1 |  |
| Assumption for UE beamsNote 3 |  |  | Rough |  |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode |  |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |
| OCNG patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | -106 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 |  |
| Note2 |  | dBm/SCS | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS | -Infinity | -87.7 |
| Note 4 |  | dB | -Infinity | 8 |
|  |  | dB | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | -66.7 | -58.0 |
| Propagation Condition |  |  | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

#### 18.3.1.7	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is not used

Editor’s Notes: This test case is incomplete:

-	E-UTRA – NR FR2 testability issue is not cleared.

-	The TT analysis is complete for test frequencies f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	This test case is incomplete for UE power classes other than PC3

-	This test case is incomplete for Test frequencies f > 40.8 GHz

18.3.1.7.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

18.3.1.7.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards and capable of NR RedCap with 2Rx. Test 1 applies to UEs not supporting per-FR gap and test 2 applies only to UEs supporting per-FR gap.

18.3.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 18.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.18.3.1.7.

18.3.1.7.4	Test description

18.3.1.7.4.1	Initial conditions

Same as the initial conditions given in clause 8.4.2.7.4.1 with following exceptions:

-	Table 8.4.2.7.4.1-1 is replaced by Table 18.3.1.7.4.1-1.

-	Table 8.4.2.7.4.1-2 is replaced by Table 18.3.1.7.4.1-2.

-	Table 8.4.2.7.4.1-3 is replaced by Table 18.3.1.7.4.1-3.

Table 18.3.1.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.3.1.7-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 18.3.1.7-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table 18.3.1.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.16-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.3.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.3.1.7.4.1-3: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  |  | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  |  | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  |  | E-UTRA Cell 1 |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | NR Cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  |  | 0 | N/A | As specified in Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  |  | 39 | N/A | As specified in TS 36.331 [29]. |
| Hysteresis |  | dB | 0 |  |  |
| CP length |  |  | Normal |  |  |
| TimeToTrigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells | Config 1 |  | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | Config 2 |  | 3s |  | Synchronous cells. |
| T1 |  | s | 5 |  |  |
| T2 |  | s | 5 | 3 |  |

18.3.1.7.4.2	Test procedure

Same as the test procedure given in clause 8.4.2.7.4.2.

18.3.1.7.4.3	Message contents

Same as the message contents given in clause 8.4.2.7.4.3.

18.3.1.7.5	Test requirement

Same as the test requirements given in clause 8.4.2.7.5 with following exceptions:

-	Table 8.4.2.7.5-1 is replaced by Table 18.3.1.7.5-1.

Table 18.3.1.7.5-1: NR cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  |  | Setup 1 |  |
| Assumption for UE beamsNote 3 |  |  | Rough |  |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode |  |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |
| OCNG patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | -106 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  |  | -104.7 |  |
| Note2 |  |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS | -Infinity | -87.7 |
|  |  |  | -Infinity | 8 |
|  |  | dB | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | -66.7 | -58.0 |
| Propagation Condition |  |  | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

#### 18.3.1.8	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is used

Editor’s Notes: This test case is incomplete:

-	E-UTRA – NR FR2 testability issue is not cleared.

-	The TT analysis is complete for test frequencies f ≤ 40.8 GHz

-	The TT analysis is complete for UE PC3

-	This test case is incomplete for UE power classes other than PC3

-	This test case is incomplete for Test frequencies f > 40.8 GHz

18.3.1.8.1	Test purpose

To verify that the RedCap UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause TS 36.133 [23] clause 8.20.2 for E-UTRAN FDD-NR measurements and TS 36.133 [23] clause 8.20.3 for E-UTRAN TDD-NR measurements.

18.3.1.8.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards and capable of NR RedCap with 2Rx. Test 1 and test 2 apply to UEs not supporting per-FR gap. Test 3 and test 4 apply only to UEs supporting per-FR gap.

18.3.1.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 18.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.18.3.1.8.

18.3.1.8.4	Test description

18.3.1.8.4.1	Initial conditions

Same as the initial conditions given in clause 8.4.2.8.4.1 with following exceptions:

-	Table 8.4.2.8.4.1-1 is replaced by Table 18.3.1.8.4.1-1.

-	Table 8.4.2.8.4.1-2 is replaced by Table 18.3.1.8.4.1-2.

-	Table 8.4.2.8.4.1-3 is replaced by Table 18.3.1.8.4.1-3.

Table 18.3.1.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.3.1.8-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 18.3.1.8-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table 18.3.1.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.16-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.3.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.3.1.8.4.1-3: General test parameters

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Numbers |  |  | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  |  | 1 |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  |  | E-UTRA Cell 1 |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | NR Cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  |  | 0 |  | N/A |  | As specified in Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  |  | 39 |  | N/A |  | As specified in TS 36.331 [29]. |
| Hysteresis |  | dB | 0 |  |  |  |  |
| CP length |  |  | Normal |  |  |  |  |
| TimeToTrigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | DRX.9 | DRX.12 | DRX.9 | DRX.12 |  |
| Time offset between serving and neighbour cells | Config 1 |  | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | Config 2 |  | 3s |  |  |  | Synchronous cells. |
| T1 |  | s | 5 |  |  |  |  |
| T2 |  | s | 7 | 70 | 7 | 70 |  |

18.3.1.8.4.2	Test procedure

Same as the test procedure given in clause 8.4.2.8.4.2.

18.3.1.8.4.3	Message contents

Same as the message contents given in clause 8.4.2.8.4.3.

18.3.1.8.5	Test requirement

Same as the test requirements given in clause 8.4.2.8.5 with following exceptions:

-	Table 8.4.2.8.5-1 is replaced by Table 18.3.1.8.5-1.

Table 18.3.1.8.5-1: NR cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  |  | Setup 1 |  |
| Assumption for UE beamsNote 3 |  |  | Rough |  |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode |  |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |
| OCNG patterns |  |  | OP.1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | -106 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 |  |
| Note2 |  | dBm/SCS | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS | -Infinity | -87.7 |
| Note 4 |  | dB | -Infinity | 8 |
|  |  | dB | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | -66.7 | -58.0 |
| Propagation Condition |  |  | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |