3GPP TS 23.542 V0.2.0 (2023-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Application layer support for Personal IoT Network;

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Intpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Overview	9

4.1	General	9

5	Architectural requirements	9

5.1	General	9

5.2	Architectural requirements	9

5.2.1	General requirements	9

5.2.1.1	General	9

5.2.1.2	Requirements	9

5.2.2	PIN Management	10

5.2.2.1	General	10

5.2.2.2	Requirements	10

5.2.3	PIN enable 5GS communication	10

5.2.3.1	General	10

5.2.3.2	Requirements	10

5.2.4	Service Switch	10

5.2.4.1	General	10

5.2.4.2	Requirements	10

5.2.5	Application server discovery	11

5.2.5.1	General	11

5.2.5.2	Requirements	11

5.2.6	Service continuity	11

5.2.6.1	General	11

5.2.6.2	Requirements	11

5.2.7	Security	11

5.2.7.1	General	11

5.2.7.2	Requirements	11

5.2.7	Subscription service	11

5.2.7.1	General	11

5.2.7.2	Requirements	11

5.2.8	PIN application KPIs	11

5.2.8.1	General	11

5.2.8.2	Requirements	11

6	Application layer architecture	12

6.1	General	12

6.2	Architecture	12

6.2.1	General Architecture	12

6.2.2	Architecture of user accessing services provided by PIN Element from outside the PIN	12

6.3	Functional entities	13

6.3.1	General	13

6.3.2	PIN client	13

6.3.3	PIN Management Client	14

6.3.4	PIN Gateway Client	14

6.3.5	PIN server	15

6.3.6	Application Client	15

6.4	Reference Points	15

6.4.1	General	15

6.4.2	PIN-1	15

6.4.3	PIN-2	15

6.4.4	PIN-3	15

6.4.5	PIN-4	16

6.4.6	PIN-5	16

6.4.7	PIN-6	16

6.4.8	PIN-7	16

6.4.9	PIN-8	16

6.4.10	PIN-9	16

6.4.11	PIN-10	16

6.4.12	PIN-11	16

6.4.13	PIN-12	17

6.5	Cardinality rules	17

6.5.1	Application Client (AC)	17

6.5.2	PEMC	17

6.5.3	PEGC	17

6.5.4	PIN server	17

6.5.5	PIN client	17

7	Identities and commonly used values	17

7.1	General	17

7.2	Identities	17

7.2.1	General	17

7.2.2	PIN ID	17

7.2.3	PIN server ID	17

7.2.4	PIN client ID	18

7.2.5	Application Client ID (ACID)	18

7.2.6	UE ID	18

7.2.7	UE Location	18

8	Procedures and information flows	18

8.1	General	18

8.2	Common Information Elements	18

8.2.1	General	18

8.2.2	PIN Profile	18

8.2.2.1	PIN Profile in a PIN	18

8.2.2.2	Dynamic profile information of a PIN	19

8.2.2.3	PIN client profile	21

8.2.3	PIN Application KPIs	21

8.3	PIN server discovery	21

8.3.1	General	21

8.3.2	Procedure	22

8.3.2.1	Static PIN server discovery	22

8.3.2.2	Procedures of PIN server discovery via PEGC	22

8.3.3	Information flows	23

8.3.3.1	General	23

8.3.3.2	PIN server discovery request	23

8.3.3.3	PIN server discovery response	23

8.4	Registration	23

8.4.1	General	23

8.4.2	Registration Procedure	24

8.4.2.1	General	24

8.4.2.2	Procedure	24

8.4.2.2.1	PINE registration directly to PIN server	24

8.4.2.2.2	PINE registration indirectly to PIN server	25

8.4.2.2.3	PINE registration via PEGC before join	26

8.4.2.2.4	PINE registration during the PIN join via the PEGC	26

8.4.2.3	Information flow/elements	27

8.4.2.3.1	General	27

8.4.2.3.2	PINE Registration request	27

8.4.2.3.3	PINE Registration response	28

8.5	PIN Management	28

8.5.1	General	28

8.5.2	PIN Create	28

8.5.2.1	General	28

8.5.2.2	Procedure	28

8.5.2.3	Information flows	30

8.5.3	PIN delete	32

8.5.3.1	General	32

8.5.3.2	Procedure	32

8.5.3.2.1	PIN delete procedure decided by PIN server	32

8.5.3.2.2	PEMC decided PIN deletion	33

8.5.3.3	Information flows	34

8.5.3.3.1	General	34

8.5.3.3.2	PIN delete request	34

8.5.3.3.3	PIN delete response	35

8.5.3.3.4	PIN delete notification request	35

8.5.3.3.5	PIN delete notification response	35

8.5.4	Multiple PEMCs/PEGCs	35

8.5.4.1	General	35

8.5.4.2	Procedure	35

8.5.4.3	Information flows	37

8.5.4.3.1	General	37

8.5.4.3.2	PIN management PINE Join into PIN request enhancements.	37

8.5.4.3.3	PIN management PINE Join into PIN response enhancements.	37

8.5.4.3.4	PEGC_notification	37

8.5.5	PIN Profile Recovery	38

8.5.5.1	General	38

8.5.5.2	Procedure	38

8.5.5.3	Information flows	38

8.5.6	Credential Provision	38

8.5.6.1	General	38

8.5.6.2	Procedure	38

8.5.6.3	Information flows	38

8.5.7	PIN discovery	38

8.5.7.1	General	38

8.5.7.2	Procedure	38

8.5.7.3	Information flows	38

8.5.8	PINE management	38

8.5.8.1	General	38

8.5.8.2	Procedure	39

8.5.8.2.1	PIN client requests to join into a PIN	39

8.5.8.2.2	Procedure of PIN elements decides to leave the PIN	40

8.5.8.2.3	Procedure of PEMC removes the PIN elements from a PIN	41

8.5.8.3	Information flows	42

8.5.9	PIN join	42

8.5.9.1	General	42

8.5.9.2	Procedure	42

8.5.9.2.1 PINE join via PEGC	42

8.5.10	PIN modification	43

8.5.10.1	General	43

8.5.10.2	Procedures	43

8.5.10.2.1	General	43

8.5.10.2.2	PIN modification after local PEMC failure	43

8.5.10.2.3	PIN modification with PEGC role change	44

8.6	PIN enable 5GS communication	47

8.6.1	General	47

8.6.2	Procedure	47

8.6.2.1	AF trigger QoS establishment	47

8.6.2.2	Procedures of PIN communication via 5GS triggered by PEGC	47

8.6.3	Information flows	47

8.7	Service Switch	47

8.7.1	General	47

8.7.2	Procedure	47

8.7.2.1	Service switch in a PIN with PIN server support	47

8.7.2.1.1	General	47

8.7.2.1.1	PIN Service Switch procedure	47

8.7.2.1.2	PIN Service Switch Configure procedure	48

8.7.2.2	Service switch in a PIN without PIN server support	50

8.7.3	Information flows	50

8.7.4	APIs	50

8.8	Application server discovery in PIN	50

8.8.1	General	50

8.8.2	Procedure	50

8.8.3	Information flows	50

8.9	Service Continuity	50

8.9.1	General	50

8.9.2	Procedure	51

8.9.2.1	Service continuity in PEGC relocation scenario	51

8.9.2.1.1	General	51

8.9.2.1.2	PIN Management PEGC Service Continuity procedure	51

8.9.2.1.3	PIN Management PEGC Configuration procedure	52

8.9.2.1.4	PIN Configuration Update procedure	52

8.9.2.2	Service continuity in changing access to 5GS	53

8.9.3	Information flows	53

8.9.4	APIs	53

Annex A: Change history	54

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

Personal IoT Networks (PIN) is based on the greatly increasing number of consumers IoT devices. Users create Personal IoT Networks out of all these Personal IoT devices mainly in their homes or around their body. This technical specification provides application layer architecture and related procedures for enabling PIN applications over 3GPP networks.

1	Scope

The present document specifies the application layer architecture, procedures and information flows necessary for enabling PIN applications over 3GPP networks. It includes architectural requirements for enabling PIN applications, application layer architecture fulfilling the architecture requirements and procedures to enable the deployment of PIN applications. Also, includes the procedures to enable 5GS communication, service switch, service continuity and server discovery internal PIN.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 22.101: "Service Principles".

[4] 	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[5] 	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Control Information: A set of information that assists the authorized PINE in a PIN to access 5GS network via PEGC, for example, the username or password.

PIN enabler: Refers to the overall functionality provided by the entities such as PIN Client, PIN Gateway Client, PIN Management Client, and PIN server in support of applications as per the architecture defined in solution 7.2.

PIN management: Refers to the set of operations related to creation, modification, maintenance and removal of PIN.

PIN Profile: A set of data and information about the PIN and PIN elements resides in the PIN. Service Switch: A mechanism to switch the service traffic flow between Application server and PINE to application server and other PINE.

NOTE:	3GPP TS 22.101 [5] clause 26a lists information that can be included in a PINE profile.

For the purposes of the present document, the following terms given in 3GPP TS 22.261 [2] apply:

Personal IoT Network

PIN direct connection

PIN Element

PIN Element with Gateway Capability

PIN Element with Management Capability

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

PIN	Personal IoT Network

PINAPP	Personal IoT Network Application

PEMC	PIN Element with Management Capability

PEGC	PIN Element with Gateway Capability

PINE	PIN Element

4	Overview

4.1	General

5	Architectural requirements

5.1	General

5.2	Architectural requirements

5.2.1	General requirements

5.2.1.1	General

This clause specifies general requirements for the architecture.

5.2.1.2	Requirements

[AR-5.2.1.2-a]	The application layer architecture shall support deployment of personal IoT network.

[AR-5.2.1.2-b]	The application layer architecture shall support different deployment models in conjunction with an operator's 3GPP network.

[AR-5.2.1.2-c]	The application layer architecture shall be compatible with the 3GPP network system.

5.2.2	PIN Management

5.2.2.1	General

This clause specifies PIN management requirements for the architecture.

5.2.2.2	Requirements

[AR-5.2.2.2-a]	The application layer architecture shall provide mechanisms to create PIN for UE or PIN elements.

[AR-5.2.2.2-b]	The application layer architecture shall provide mechanisms to delete PIN, either triggered by PINEs or by PIN server.

[AR-5.2.2.2-c]	The application layer architecture shall support the mechanisms of PIN modification procdure, for example, PEMC/PEGC relocation.

[AR-5.2.2.2-d]	The application layer architecture shall support the deployment and mechanism of multiple PEMCs/PEGCs.

[AR-5.2.2.2-e]	The application layer architecture shall support mechanisms to obtain PIN server endpoint address.

[AR-5.2.2.2-f]	The application layer architecture shall support the mechanisms to perform PIN discovery, and enable the PINEs to join/leave the PIN.

[AR-5.2.2.2-g]	The application layer architecture shall support the mechanisms of PINE registration to PIN server.

[AR-5.2.2.2-h]	The application layer architecture shall support mechanisms to maintain, configure, update the PIN profile/PIN client profile.

5.2.3	PIN enable 5GS communication

5.2.3.1	General

This clause specifies PIN communication requirements for the architecture.

5.2.3.2	Requirements

[AR-5.2.3.2-a]	The application layer architecture shall provide mechanisms to configure routing information in PEGC to enable the PINE to access the network provided by PEGC.

[AR-5.2.3.2-b]	The application layer architecture shall provide mechanisms to support the PIN and the PINEs in PIN to consume the 5GS communication.

[AR-5.2.3.2-c]	The application layer architecture shall provide mechanisms to support the PEMC/PEGC to request the 5GS resource for PIN.

5.2.4	Service Switch

5.2.4.1	General

This clause specifies service switch requirements for the architecture.

5.2.4.2	Requirements

[AR-5.2.4.2-a]	The application layer architecture shall provide mechanisms to support the service switching in a PIN between different PINE for achieving better service experience.

5.2.5	Application server discovery

5.2.5.1	General

This clause specifies application server discovery requirements for the architecture.

5.2.5.2	Requirements

[AR-5.2.5.2-a]	The application layer architecture shall provide mechanisms to support the application server discovery for PIN.

5.2.6	Service continuity

5.2.6.1	General

This clause specifies service continuity requirements for the architecture.

5.2.6.2	Requirements

[AR-5.2.6.2-a]	The application layer architecture shall provide mechanisms to support the PEGC relocation procedure to enable service continuity.

[AR-5.2.6.2-b]	The application layer architecture shall provide mechanisms to change the communication from via PEGC to via 5GS, and enable the service continuity.

5.2.7	Security

5.2.7.1	General

5.2.7.2	Requirements

5.2.7	Subscription service

5.2.7.1	General

5.2.7.2	Requirements

5.2.8	PIN application KPIs

5.2.8.1	General

5.2.8.2	Requirements

6	Application layer architecture

6.1	General

6.2	Architecture

6.2.1	General Architecture

Figure 6.2.1-1 illustrates the reference point representation of the architecture for PINAPP.

Figure 6.2.1-1: PINAPP architecture

The PIN elements contains PIN client and/or application clients. The PIN Element with gateway capability (PEGC) performs the role of an entity supporting gateway capability for PIN. The PIN Element with management capability (PEMC) performs the role of an entity supporting management capability for PIN.

NOTE 1:	It is possible that an application client on PIN elements can communicate with application server directly via 5GS or indirectly via PEGC.

NOTE 2:	It is possible that an application client can communicate with other application client in the same PIN directly or via PEGC.

NOTE 3:	It is possible that an application client can communicate with other application client in another PIN via PEGC.

NOTE 4:  There is no restrict that only one PEMC/PEGC deployed in PIN. It is possible that in a PIN, multiple PEMCs/PEGCs are deployed.

Editor's note: Whether PIN-5 is in the scope of PINAPP is FFS.

6.2.2	Architecture of user accessing services provided by PIN Element from outside the PIN

The Figure 6.2.2-1 shows the application architecture to enable authorized user to access services provided by PIN element behind the PEGC. For simplicity, not all functional elements of Figure 6.2.2-1 are shown in below Figure 6.2.2-1.

Figure 6.2.2-1: PINAPP architecture of User accessing services provided by PIN Element from outside the PIN

The interactions between PEGC and PIN client of the authorized user are supported over interface PIN-11. The interactions between PEMC and PIN client of the authorized user are supported over interface PIN-12. The authorized user uses PIN-12 to configure the policies in a PIN.

NOTE1:	The authorized user is allowed to manage a PIN due to authorized user has PIN-12 interface to communicate with PEMC.

6.3	Functional entities

6.3.1	General

6.3.2	PIN client

The PIN enabler layer entity residing in the PIN elements that provides the client side functionalities required for application clients in order to consume the services offered by the PIN or to offer services for other PIN elements to consume.

It provides the following functionalities:

-	Registration of the available service and capabilities;

-	Service discovery of other PIN elements and application server;

-	Communication with PIN clients of other PIN elements;

-	Selection of relay for direct communication;

-	Maintaining the PIN profile;

-	Perform to join/leave a PIN;

-	Support to discover the available PIN; and

-	Support to receive the information to access the 5G core network via PEGC.

-	Support service switch internal PIN.

-	Support PIN service continuity of PEGC relocation or changing to 5GS communication.

6.3.3	PIN Management Client

A PIN Element with Management Capability is a PIN Element that provides a means for an authorised administrator to configure and manage a PIN.

It provides following functionalities:

-	For a network operator or authorized user to configure the policies of the PIN;

-	Provide life span information of the PIN to the authorized user or the PIN elements;

-	Maintain the list of PIN elements who joined the PIN. Maintaining available PIN services;

-	Maintain the PIN profile for each PIN and PINE in PIN;

-	To configure and manage a PIN, including:

-	authorizing the PIN elements requesting to join the PIN;

-	authorizing the PEGC and configure the parameters in PEGC to support PINE communication (via 5GS or direct communication);

-	configuring PIN elements to enable discovery of services offered by other PIN Elements;

-	add PIN elements to the PIN;

-	configure PIN elements to enable direct communications;

-	configure PIN elements to communicate with each other when gateway device is unavailable.

-	support the PIN server endpoint address delivery to PIN elements;

-	support the credentials delivery to PIN elements;

NOTE:	When gateway device is unavailable, the configurations are required to enable direct communication.

-	Support service switch internal PIN.

-	Support PIN service continuity of PEGC relocation or changing to 5GS communication.

6.3.4	PIN Gateway Client

A PIN Element with Gateway Capability is a PIN Element PIN Element that has the ability to provide connectivity to a PINE for data and signalling exchange with PEMC, other PIN Elements or the DN. It may act as both the Layer-3 type relay which transparently forwards the traffic for PINE, and the application layer relay which terminates the PIN-2 reference points, processes the PIN management messages from/to the PINE, and performs authorization check.

It provides the following functionalities when acting as application layer relay:

-	Maintain the PIN profile for each PIN and PINE in PIN;

-	Maintain the access control information for each PIN and each PINE in PIN;

-	Support to trigger the PDU session modification towards 5GS to request the resource for PIN;

-	Enable the 5GS communication or direct communication;

-	Support to PIN server address delivery;

-	Support to deliver the credentials to PINE;

-	Support PIN discovery function

-	Support service switch internal PIN.

-	Support PIN service continuity of PEGC relocation or changing to 5GS communication.

It provides the following functionalities when acting as Layer-3 type relay:

-	Receives the packet from the PINE and forwards the traffic to network transparently

-    Receives the packet from the PINE and forwards the traffic transparently or another PINE transparently

-	Receives the packet from the Network and fowards the packet towards to the PINE transparently

6.3.5	PIN server

A PIN server is deployed at network that provides server side functionalities required for managing the PIN.

It provides the following functionalities:

-	Provisioning of configuration information to the PIN elements;

-	Maintain the PIN profile for each PIN and PINE in PIN;

-	PIN Management (Creation, modification and deletion) of PIN;

-	Determine the access control information of PEGC/PINE in PIN;

-	Authorization of the PINE to be added/removed into/from the PIN

-	Support PIN discovery and application server discovery

-	Support service switch internal PIN.

-	Support PIN service continuity of PEGC relocation or changing to 5GS communication.

6.3.6	Application Client

An application client is the application resident in the PIN elements.

6.4	Reference Points

6.4.1	General

Editor's note: Some of the reference points should be coordinated with the related PIN procedure, and will be supplied the details of interface function in the following meeting.

6.4.2	PIN-1

The interactions related to enabling PINAPP, between the Application client and the PIN client.

6.4.3	PIN-2

This reference point exists between PIN client and PEGC which connects PIN client of UE to PEGC. The PIN client uses this interface to communicate with other PIN clients within PIN or to access 3GPP network.

6.4.4	PIN-3

This reference point exists between the PIN client and PEMC and following functionalities are supported over this reference point:

-	Authorizing PIN clients to access PIN;

-	Discovery of services offered byother PIN elements;

-	Discovery and selection of relay PIN elements;

-	Notifying the PIN information modification details (e.g. PEMC change, PEGC change, PIN capabilities change).

6.4.5	PIN-4

This reference point exists between the PEGC and PEMC and following functionalities are supported over this reference point:

-	Authorizing PEGC for PIN access;

-	Notification of PIN elements joining or leaving the PIN by PEMC to PEGC;

-	Delivery of PIN dynamic profile information by PEMC to PEGC whenever it changes;

6.4.6	PIN-5

This reference point exists between the one PIN client and another PIN client and it supports direct connection over 3GPP or non-3GPP RAT. It also connects to PIN client of a PIN element to the PIN client of another PIN element acting as a relay.

Editor's note: It is FFS that whether the PIN-5 should be standardized in PINAPP.

6.4.7	PIN-6

This reference point exists between the PEMC and PIN server and supports the following functionalities:

-	Authorization of PEMC;

-	Notifying PIN server whenever a PIN element joins or leaves the PIN, whenever a PIN client updates its capabilities;

-	Notifying PIN server of PEGC replacement;

-	Delivery of PIN dynamic profile information;

6.4.8	PIN-7

This reference point exists between the PEGC and PIN server for the interactions related to enabling PINAPP

6.4.9	PIN-8

This reference point exists between the PIN server and 3GPP core network for the interactions related to enabling PINAPP

6.4.10	PIN-9

This reference point exists between the application server and PIN server for the interactions related to enabling PINAPP.

6.4.11	PIN-10

This reference point exists between the PIN client in PIN element and PIN server for the interactions related to enabling PINAPP.

6.4.12	PIN-11

This reference point exists between the PEGC and PIN client from outside the PIN to access the services provided by PIN elements within the PIN.

PIN-11 utilizes Uu reference point as described in 3GPP TS 23.501[4].

6.4.13	PIN-12

This reference point exists between the PEMC and PIN client for configuring and managing the PIN from outside the PIN.

PIN-12 utilizes Uu reference point as described in 3GPP TS 23.501[5].

6.5	Cardinality rules

6.5.1	Application Client (AC)

The following cardinality rules apply for application client:

a)	one or more application clients per PIN client.

6.5.2	PEMC

The following cardinality rules apply for PEMC:

a)	one or more PEMCs per PIN.

b)	Only one PEMC in certain PIN is assigned as primary role and other PEMCs if any are assigned with secondary role.

c)	One PEMC can act as PEMC for multiple PINs.

6.5.3	PEGC

The following cardinality rules apply for PEGC:

a)	one or more PEGCs per PIN.

a)	For a certain PINE in PIN, one PEGC acts as default PEGC and other PEGCs act as backup.

c)	One PEGC can act as PEGC for multiple PINs.

6.5.4	PIN server

The following cardinality rules apply for PIN server:

a)	one PIN server per PIN.

b)	Multiple PINs per PIN server.

6.5.5	PIN client

The following cardinality rules apply for PIN client:

a)	one or more PIN clients per PIN.

b)	one PIN client per PIN element.

7	Identities and commonly used values

7.1	General

The following clauses list identities and commonly used values that are used in this technical specification.

7.2	Identities

7.2.1	General

7.2.2	PIN ID

The PIN ID is a unique value in PLMN that identifies the PIN.

7.2.3	PIN server ID

The PIN server ID is a unique value in PLMN domain that identifies the PIN server.

7.2.4	PIN client ID

The PIN client ID is a globally unique value that identifies the PIN client.

The PIN client ID is used for general identities for PINE, PEMC and PEGC.

For PEMC and PEGC, the PIN client ID may also be PEMC ID and PEGC ID.

For the PINE, the PIN client ID may also be PINE ID.

7.2.5	Application Client ID (ACID)

The ACID identifies the client side of a particular application, for e.g. SA6Video viewer, SA6MsgClient etc. For example, all SA6MsgClient clients will share the same ACID.

In case that the UE is running mobile OS, the ACID is a pair of OSId and OSAppId.

7.2.6	UE ID

The UE ID uniquely identifies a particular UE within a PLMN domain. Following identities are examples that can be used:

a)	GPSI, as defined in 3GPP TS 23.501 [4].

NOTE:	To protect privacy of the user, MSISDN can be used as GPSI only after obtaining user's consent.

7.2.7	UE Location

8	Procedures and information flows

8.1	General

The PINE communicates with the PEMC via direct way (i.e., via WiFi, BlueTooth), or indirect way relayed by a PEGC. PINE determines to use direct way or indirect way based on the capabilites of access types supported by the PINE, PEMC and PEGC.

The Application layer must be able to influence the RAT selection for establishing connectivity to the PIN.

NOTE: How the Application layer is able to influence the RAT selection is up to implementation.

8.2	Common Information Elements

8.2.1	General

This clause provides descriptions for Information Elements which are commonly used in several procedures.

8.2.2	PIN Profile

8.2.2.1	PIN Profile in a PIN

PIN profile includes information about the static data needed for configuration for the Personal IoT networks.

Table 8.2.2.1-1: PIN Profile

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC |
| --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y |
| PIN Description | Human-readable description of the PIN, for example, the company name, location or the type of service. | Y | Y | Y |
| Duration | Specifies the time period of how long the PIN can be active | Y | Y | Y |
| Maximum number of PIN elements | Maximum number of PIN elements allowed to join the PIN | Y | Y | N |
| PIN service | List of service that a PIN can provide, including the PINE service or the service that can provided by application client on PINE: > PIN service Provider Identifier > PIN service type > PIN service Feature | Y | Y | N |
| PEMC list | The list of identifiers of the PIN elements which can be allowed to take the role as PEMC (e.g.: PIN client ID, UE GPSI etc.,) and also it contains whether the role is primary or secondary > PIN Element ID > Role | Y | Y | Y |
| PEGC ID list | The list of identifiers of the PIN elements which can be allowed to take the role as PEGC (e.g.: PIN client ID, UE GPSI etc.,) > PIN Element ID | Y | Y | Y |
| PIN Server ID | The identifier of the PIN server that serves the PIN | N | Y | Y |
| PIN server Endpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PIN server. | N | Y | Y |
| PIN Elements List | List of PIN elements which can be allowed to join the PIN > PIN element ID | Y | Y | Y |

Editor's Note: Other IEs for the purpose of enabling multiple PEMC/PEGC scenarios, e.g. procedure for role change, are FFS.

8.2.2.2	Dynamic profile information of a PIN

Dynamic profile information of a PIN contains the PIN dynamic data needed for management of the Personal IoT networks.

Table 8.2.2.2-1 describes the list of parameters that are classified as dynamic profile information and which are maintained at the PIN server, PEMC and PEGC. PIN dynamic profile information maintained at these entities are updated based on the events occurring in the PIN. Below are some of the events (not exhaustive):

-	PINE joins or leaves the PIN;

-	Role of PEMC or PEGC changes;

-	When the services offered by the PIN changes;

-	When a PINE updates the services it offers;

-	When a PINE joins or leaves the PIN;

Table 8.2.2.2-1: Dynamic profile information of a PIN

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC |
| --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y |
| PIN Description | Human-readable description of the PIN, for example, the company name, location or the type of service. | Y | Y | Y |
| Services offered | List of services currently offered by the PIN | Y | Y | N |
| PEMC list | The list of identifiers of the PIN elements which are currently serving as PEMC (e.g.: PIN client ID, UE GPSI etc.,) and whether the PEMC is authorized as primary PEMC or secondary PEMC. | Y | Y | Y |
|  | > PIN Element ID | Y | Y | Y |
|  | > PEMC role | Y | Y | Y |
|  | > Endpoint information of each PEMC (e.g. URI, FQDN, IP address) used to communicate with the PEMC. | Y | Y | Y |
|  | > Duration or time period this PIN element allowed to being PEMC |  |  |  |
| PEGC list | The list of identifiers of the PIN elements which are currently serving as PEGC (e.g.: PIN client ID, UE GPSI etc.,) | Y | Y | Y |
|  | > PIN Element ID | Y | Y | Y |
|  | > Endpoint information of each PEGC (e.g. URI, FQDN, IP address) used to communicate with the PEGC. | Y | Y | Y |
|  | > Duration or time period this PIN element allowed to being PEGC | Y | Y | Y |
|  | > KPIs supported by this PEGC (e.g., maximum number of assigned PIN elements) | Y | Y | Y |
|  | > List of PIN elements being served by PEGC and their connectivity information |  |  |  |
| PIN Server Details | The details of the PIN server that serves the PIN |  |  |  |
|  | > Identifier of the PIN server | N | Y | Y |
|  | > Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PIN server. | N | Y | Y |
| PIN Elements List | List of PIN elements currently registered/joined the PIN and their details |  |  |  |
|  | > PIN Element ID | Y | Y | Y |
|  | > Services offered by the the PIN element | Y | Y | N |
|  | > Reachability information of the PIN element | Y | Y | Y |
|  | > List of application clients for this PIN element: |  |  |  |
|  | >> Minimum KPIs required by each application client to operate effectively within the PIN (e.g., PIN bandwidth, PIN request rate, PIN response time) | Y | Y | Y |
|  | >> Operational schedules of each application client (e.g., time windows) | Y | Y | Y |
|  | > Identifier of the default PEGC authorized to service this PIN element. The PIN element will use this PEGC as the primary PEGC to relay PIN communications. Location and/or schedule information for the default PEGC may also be included such that the default PEGC may be selected by the PIN element based on its current location and proximity to the default PEGC and/or the availability schedule of the default PEGC. | Y | Y | Y |
|  | > Identifiers of backup PEGCs authorized to service this PIN element. The list is in prioritized order (the first PEGC listed will serve as the first backup PEGC). If the default PEGC is not available, the PIN element will use this prioritized list of PEGCs to relay PIN communications. Location and/or schedule information for each of the backup PEGCs may also be included such that a backup PEGC may be selected by the PIN element based on its current location and proximity to a backup PEGC and/or the availability schedule of the PEGC. | Y | Y | Y |

Editor's Note: Other PIN Profile IEs for the purpose of enabling multi PEMC/PEGC scenarios, e.g. procedure for role change, are FFS.

8.2.2.3	PIN client profile

Table 8.2.2.3-1 describes the list of information elements that needs to be shared by the PIN element when requesting to join the PIN.

Table 8.2.2.3-1: PIN client profile

| Parameter Name | Status | Parameter Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN to which the PIN element wants to join |
| UE identifier | O | PIN Element or UE identifier |
| PIN client ID | M | The unique identity of the PIN client within PIN |
| Name of the device | O | Human-readable name of the device (i.e. door sensor, watch, smart TV, etc) along with manufacturer details |
| Application List | O | List of application identities |
| > Application Identity | O | Identity of the application |
| > Application schedule | O | Operational schedules of each application (e.g., time windows) which the |
| > Application KPIs | O | Minimum KPIs required by each application to operate effectively within the PIN (e.g., PIN bandwidth, PIN request rate, PIN response time) |
| capabilities | M | capabilities of the PIN client like whether it provides relay capability for other PIN elements, whether it can be assigned with the role of PEMC or PEGC etc. |
| Visibility | M | Determines whether this PIN element is discoverable by other PIN elements within PIN, discoverable by other UEs outside the PIN etc., |
| access type | M | Access type supported for the communication |
| Layer-2 details | O | Layer-2 address of the PIN element |
| Required services | O | Identifies the list of services the PINE wants to consume |
| Supported services | O | Identifies the list of services the PINE is providing and allowed to be accessed |

Editor's Note: Other IEs for the purpose of enabling multiple PEMC/PEGC scenarios, e.g. procedure for role change, are FFS.

8.2.3	PIN Application KPIs

8.3	PIN server discovery

8.3.1	General

Some of the PIN management procedures needs the PIN server to help. So, the PINE should receive the PIN server endpoint address first and then trigger the other following procedure, for example, the PIN creation procedure.

Due to the PIN server in a PLMN can be multiple, so it is important to PIN elements to discover the appropriate PIN server to connect.

8.3.2	Procedure

8.3.2.1	Static PIN server discovery

The aim of PIN server discovery procedure is to receive one or more endpoint information (e.g. URI(s), FQDN(s), IP address(es)) of PIN server. And the PEMC, PEGC, PIN elements are all able to receive the PIN server endpoint information.

The PIN server can be discovered by the following method:

-	pre-configured in the PIN elements or PIN clients;

-	configured by the user;

-	derived from HPLMN identifier for non-roaming scenario or from VPLMN identifier for roaming scenario.

8.3.2.2	Procedures of PIN server discovery via PEGC

Due to for some of the PIN elements can have the application interaction towards the PEGC, for example, via WiFi or Bluetooth pairing, so the PEGC can provide the PIN server end point information to PIN elements.

For some of the PEGC, it has the open access capability to accept the application layer connection from the PIN elements.

Pre-conditions:

1.	The PIN elements or PIN client has application layer connection with PEGC;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PEGC supports the open access and can reroute the request from PINE to PEMC behind the PEGC;

Figure 8.3.2.2-1: Procedures of PIN server discovery via PEGC

0.	The PINE has already had the application layer connection towards PEGC. For example, the PIN elements can communicate with PEGC via WiFi or Bluetooth.

The PINE can also have the open access to PEGC that with no user name or password. For this situation, the PINE can’t consume the communication service that provided by PEGC, but can have communication with the PEMC behind the PEGC.

1.	The PINE sends PIN server discovery request to PEGC. The requests include the GPSI, MAC address, if has, UE location.

2.	(Optional) The PEGC can directly deliver the PIN server discovery response to PINE, including PIN server end point information to PIN elements. The end point information of PIN server includes URI(s), FQDN(s), IP address(es)) of PIN server.

3.	If the PINE has open access to PEGC, that the PEGC should route the PIN server discovery request to PEMC that behind the PEGC.

4-5.	The PEMC delivers the PIN server discovery response to PEGC and the PEGC routes the response to PINE, including PIN server end point information to PIN elements. The end point information of PIN server includes URI(s), FQDN(s), IP address(es)) of PIN server.

8.3.3	Information flows

8.3.3.1	General

The following information flows are specified for PIN creation:

-	PIN server discovery request and response;

8.3.3.2	PIN server discovery request

Table 8.3.3.2-1 describes information elements in the PIN discovery request from the PINE to the PEGC/PEMC.

Table 8.3.3.2-1: PIN server discovery request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) or PINE. |
| MAC address | O | MAC address of the device. |
| UE location | O | The location information of the UE. The UE location is described in clause x.x.x. |

8.3.3.3	PIN server discovery response

Table 8.3.3.3-1: PIN server discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the PIN creation request was successful. |
| > PIN server endpoint information | M | Includes URI(s), FQDN(s), IP address(es)) of PIN server. |
| Failure response | O | Indicates that the PIN creation request failed. |
| > Cause | M | Provides the cause for PIN creation request failure. |

8.4	Registration

8.4.1	General

The PINE (including PEMC and PEGC) should have a registration procedure in PIN server, before consuming the certain PIN service.

The registration procedure includes the direct registration and indirect registration.

-	Direct registration:

-	For the PEMC, the PEMC sends PINE Registration Request (MAC address, vendor name, device description, PEMC Address) to the PIN server. After successful registration in PIN server, the PIN server allocates the PIN client ID to this PEMC and the PEMC receives the role of PEMC.

-	The PINE can also directly register to the PIN server via the PEGC, if the PEMC/PIN server has already provided the PIN server address to the PINE. And if the PINE is accessing the PEGC without any registration and authorization, the PEGC may reject the message from the PINE and request the PINE to perform the registration.

-	Indirect registration:

-	For the PINE and PEGC, the PEMC substitutes the PINE/PEGC to register on PIN server with the device metadata from PINE/PEGC (MAC address, vendor name, device description, PINE/PEGC Address). After successful registration in PIN server, the PIN server allocates the PIN client ID to PINE/PEGC.

At the network side, a PIN server should be deployed. During the PINE registration procedure, the PIN server is responsible for the authorization of the request of the role of PEMC from PINE. The PIN server has this verification procedure with 5GC, which the procedure is defined in SA2 specification.

8.4.2	Registration Procedure

8.4.2.1	General

The PINE may register to the PIN server via either the PEMC or the PEGC depends on the communication range, supported RATs.

8.4.2.2	Procedure

8.4.2.2.1	PINE registration directly to PIN server

Figure 8.4.2.2.1-1 illustrates PIN creation procedure based on request/response model.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE Identifier is available;

3.	The PINE has been authorized to communicate with the PIN server;

Figure 8.4.2.2.2-1: PINE registration directly to PIN server

1.	The PINE (including PINE/PEMC/PEGC) sends a PINE Registration Request to the PIN server. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PINE Address.

If the PEMC trigger the request, this registration request carries the PIN ID of the PIN for which it is intending to register as PEMC and the PIN element may indicate whether it is to be assigned with primary or secondary PEMC role.

2.	The PIN server interacts with the 5GS to check whether the UE identified by the GPSI has subscribed to be a PEMC. Also, the PIN server can check the whether the UE identified by the GPSI has subscribed to be a PEMC itself. If subscribed, it checks the PIN profile whether the requesting PIN element can be assigned with primary or secondary role and authorize the PIN element accordingly. In case if the PIN has already been created, the requesting PIN element is assigned with the secondary PEMC role irrespective of the role requested by the PIN element.

Editor's note: whether and how to 5GS check the UE identified by the GPSI has subscribed to be a PEMC is in SA2 scope.

3.	The PIN server responds to the PINE with PINE Registration Response with allocated PIN client ID in successful response.

If the registration procedure fails, the PIN server should give the failure response to indicates that indicates the cause of registration request failure.

8.4.2.2.2	PINE registration indirectly to PIN server

The following procedure defines the PEMC can substitute the PINE/PEGC to register into PIN server.

Pre-conditions:

1.	The PEMC has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PEMC has been authorized to communicate with the PIN server;

4.	The PINE/PEGC has already received the IP address of PEMC.

Figure 8.4.2.2.2-1: PINE registration indirectly to PIN server

1.	The PINE/PEGC sends PINE Registration Request to the PIN server. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PINE Address.

2.	The PEMC substitutes the PINE/PEGC to send the request to PIN server.

3.	The PIN server responds to the PEMC with PINE Registration Response with allocated PIN client ID in successful response.

4.	The PEMC sends the responds received during step 3 to the PINE/PEGC.

8.4.2.2.3	PINE registration via PEGC before join

The PINE registration towards the PIN server via the PEGC based on request/response model is depicted in figure 8.4.2.2.3-1.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The PINE already establishes the connection with PEGC;

Figure 8.4.2.2.3-1: PINE registration to PIN server via PEGC before join

1.	The PINE sends PIN registration request to the PIN server. The PIN registration request is routed to the PEGC. The PINE device identity, device credentials and other necessary formation which is also needed for PIN service authorization are included.

2.	The PEGC identifies the received message is the PIN registration request which is allowed to be forwarded regardless the PINE is authorized or not.

3.	The PEGC forwards the PIN registration request towards the PIN server.

4.	The PIN server performs service authorization and verifies the PINE device with the information in the registration request. If authorized, the PIN server allocates the PIN client ID, authorization and security credentials for the PINE.

5.	The PINE server returns the PIN registration response to the PEGC.

6.	The PEGC forwards the PIN registration response to the PINE.

8.4.2.2.4	PINE registration during the PIN join via the PEGC

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The PINE already establishes the connection with PEGC;

3.	The PINE may get the PIN information from the PEMC, PEGC via PIN announcement after connecting to PEMC or PEGC.

Figure 8.4.2.2.4-1 illustrates the PINE registration during the PIN join via the PEGC procedure based on request/response model.

Figure 8.4.2.2.4-1: PINE registration to PIN server via PEGC

1.	The PINE sends PIN join/discovery request to the PEGC. The PINE device identity is included.

-	For the PIN join request, the PIN ID is included.

-	For the PIN discovery request, the discovery criteria are included.

2.	The PEGC identifies the received message is the PIN join/discovery request, and the PINE is not registered and authorized due to no PIN client ID and credentials in the message.

3.	The PEGC returns the PIN join/discovery reject message to the PINE. The registration and authorization indication are also included to instruct the PINE to perform registration.

4.	The PINE if not registered, initiates the PIN registration towards the PIN server via the PEGC as described in clause 8.4.2.2.1.

5.	After the registration, the PINE will use the PIN client ID and credentials to initiate the PIN join/discovery again.

8.4.2.3	Information flow/elements

8.4.2.3.1	General

The following information flows are specified for PIN creation:

-	PINE Registration request and response;

8.4.2.3.2	PINE Registration request

Table 8.4.2.3.2-1 describes information elements in the PINE Registration request from the PINE (including PEMC, PEGC, PINEs) to the PIN server.

Table 8.5.2.3.2-1: PINE Registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) of PINE/PEMC/PEGC |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| MAC address | O | MAC address of the requested PINEs. |
| Vendor name | O | The vendor name of the PINE |
| Device description | O | Give the description of the device. |
| PINE Address | O | The IP address of PINE, if available. |

8.4.2.3.3	PINE Registration response

Table 8.4.2.3.3-1: PINE Registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the PINE Registration request was successful. |
| > PIN client ID | M | Identifier of the newly assigned PIN client ID to PINEs. |
| > Role of PEMC | O | The UE identified by the GPSI has subscribed to be a PEMC. |
| Failure response | O | Indicates that the PIN creation request failed. |
| > Cause | M | Provides the cause for PIN creation request failure. |

8.5	PIN Management

8.5.1	General

8.5.2	PIN Create

8.5.2.1	General

After the UE or PINE acquires the role of PEMC and receives the address of PIN server, the UE or PINE can trigger a creation of PIN towards PIN server.

Below are the possible scenarios when the PEMC request for the creation of PIN:

-	No PIN elements or PEGC have established connection with PEMC;

-	One or more PIN elements including PEGCs, PEMCs have established connection with PEMC via non-3GPP access. In this case the PEMC can trigger creation of PIN with these PIN elements in group.

After the creation of PIN is accepted by network, the PIN server responds to PEMC containing the details of the PIN including the PIN ID, the PEGC information, access control information configured in PEGC etc.

At the network side, a PIN server should be deployed. The PIN server is responsible for the authorization of the Creation request of PIN, and arranges the PEGC information about access control to PIN.

8.5.2.2	Procedure

##### 8.5.2.2.1	PIN creation procedure

Figure 8.5.2.2.1-1 illustrates PIN creation procedure based on request/response model.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE Identifier or PIN client Identifier is available;

3.	The UE or PINE has already been registered in PIN server;

4.	The UE or PINE has been authorized to communicate with the PIN server;

5.	PINE 1 is assigned the role of PEMC by PIN server and PINE-2 is the PEGC of the PIN;

Figure 8.5.2.2.1-1: PIN creation procedure

1.	The PEMC sends a PIN creation request to the PIN server in order to create a PIN. The PIN creation request includes the security credentials of the UE or PINE-1 received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID, UE location and PIN client profile(s) information.

The PEMC can request to create a PIN including the details of other PIN elements (list of PINEs) that has already established connection with it. The details of the PIN elements could be for example, UE identifier such as GPSI, PIN client ID, UE location and PIN client profile(s) information.

If there are no other PIN elements in the request, the PEMC requests to create a PIN including itself.

In order to save the procedure of several PEMCs to be involved into the certain PIN as individual PEMC, the PEMC can have the additional PEMC GPSIs/PIN client ID in the PIN create request, to indicate additional PEMCs that are allowed to manage the PIN. This procedure doesn’t have conflict with that other PEMC requests to join the certain PIN and becomes PEMC separately.

NOTE 1:	For a certain PIN, only one PEMC at a time can be assigned with primary role and other PEMCs if any are assigned with secondary role.

2.	Upon receiving the request, the PIN server performs an authorization check to verify whether the PEMC (PINE 1) has authorization to perform the PIN creation operation.

Editor's Note:	Whether and how the 5GC involved in the PIN creation depends on the feedback from SA2.

3.	The PIN server sends a successful PIN creation response to PEMC, which includes a newly assigned PIN ID to indicate the PIN. It also includes the list of PIN elements and their identifier which are authorized and made as member of the newly created PIN if the PIN creation request contains the list of PIN elements to be included in the PIN.

If the PIN creation request fails, the PIN server should give the failure response indicating the cause of PIN creation request failure.

If there are no other PIN elements in the PIN creation request and the PIN creation is successful, the PIN server indicates the PEMC to be the PEGC. The PEMC who has already had the role of PEMC can also has the role of PEGC.

If the one or more PEGCs are indicated in the PIN creation response, the PIN server indicates the PIN client ID/GPSI of one PINE to be the PEGC. Also, the assigned IP address or port number is delivered in the PIN creation response to PEMC. And, the PIN Server also sends the PEGC information about access control in the response, including:

-	Access control information includes: user name, account, SSID, BSSID. All the information is used by PIN elements in PIN to access 5G or access other application outside of PIN;

The PIN server or PEMC can decide the access control information in certain PEGC.

-	If the access control information decided by PIN server, the PIN server sends the access control information to PEMC via PIN creation response. And the PEMC delivers the access control information to PEGC via PIN creation notification request.

-	If the access control information decided by PEMC, the PEMC delivers the access control information to PEGC via PIN creation notification request.

4a-4c.	[Optional] If the PIN creation response contains the list of PIN elements, the PEMC generates the PIN creation notification request to individual PINEs based on the list received in step 3. This notification request includes the PIN ID of the newly created PIN and also contains an indication that the PIN element is made the member of the newly created PIN.

5a-5c. [Optional] The individual PIN elements sends the PIN creation notification response to acknowledge the receipt of the notification. The PIN elements receiving the PIN creation notification request with joined indication shall not join the PIN by issuing the PIN join request since they are already made as the member of the PIN.

After the procedure above, the PINE-1 (PEMC) creates a PIN with PEGC (PINE-2) and other accepted PIN elements in PIN.

8.5.2.3	Information flows

##### 8.5.2.3.1	General

The following information flows are specified for PIN creation:

-	PIN creation request and response;

-	PIN creation notification request and response;

Editor's note: A review and update of procedural text and IEs is necessary to ensure that multiple PEMC/PEGCs are supported.

##### 8.5.2.3.2	PIN creation request

Table 8.5.2.3.2-1 describes information elements in the PIN creation request from the PINE/PEGC to the PIN server.

Table 8.5.2.3.2-1: PIN creation request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) or the PIN client ID of PINE/PEMC |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client profile(s) | O | Profiles of PIN clients. The PIN client profiles are further described in Table 8.2.2.3. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.2.7. |
| Lists of PINEs | O | The PINEs that has already communicated with PEMC directly, and intend to add these PINE into PIN. |
| Additional PEMCs | O | Indicate additional PEMCs that are allowed to manage the PIN. |

##### 8.5.2.3.3	PIN creation response

Table 8.5.2.3.3-1: PIN creation response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the PIN creation request was successful. |
| > PIN ID | M | Identifier of the newly created PIN. |
| > Expiration time | M | Indicates the expiration time of the PIN. |
| > Lists of PINEs | O | List of PIN elements and their identifier which are authorized and made as member of the newly created PIN if the PIN creation request contains the list of PIN elements to be included in the PIN. |
| > Identifier of PEGCs | O | Indicates the PINE identifier authorized to be the PEGCs of this PIN. |
| >> PEGC address | O | Assigned IP address or port number of PEGC |
| > PEGC information | O | Includes the PEGC information for example, |
| >> Access control information | O | Includes: user name, account, SSID, BSSID. All the information is used by PIN elements in PIN to access 5G or access other application outside of PIN |
| Failure response | O | Indicates that the PIN creation request failed. |
| > Cause | M | Provides the cause for PIN creation request failure. |

##### 8.5.2.3.4	PIN creation notification request

Table 8.5.2.3.4-1: PIN creation notification request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the newly created PIN. |
| Indication | O | Indicates the PIN element is made the member of the newly created PIN. |
| > Identifier of PEGCs | O | Indicates the PINE identifier that to be the PEGCs of this PIN. |
| >> PEGC address | O | Assigned IP address or port number of PEGC |
| > PEGC information | O | Includes the PEGC information for example, |
| >> Access control information | O | Includes: user name, account, SSID, BSSID. All the information is used by PIN elements in PIN to access 5G or access other application outside of PIN |

##### 8.5.2.3.5	PIN creation notification response

Table 8.5.2.3.5-1: PIN creation notification response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the PIN creation notification request was successful. |
| Failure response | O | Indicates that the PIN creation notification request failed. |
| > Cause | O | Indicates the cause of PIN creation notification request failure. |

8.5.3	PIN delete

8.5.3.1	General

The PIN which is in use can be deleted based on the decision by PEMC or PIN server as described below:

-	Decided by PEMC. The PEMC of a PIN decides to delete the PIN and sends request to PIN server. The PIN server accepts the request and deletes the PIN.

-	Decided by PIN server. If the PIN is configured to exist for a particular duration and if it continues to exist post the duration the PIN server can decide to delete the PIN and release the resources associated with the PIN.

Once the PIN is deleted, the PIN elements in PIN shall not be able to utilize the services by the PIN or 5GS anymore and cannot access the application server. The network resources allocated for this PIN will be released.

Since the configuration related to the duration of the PIN is available with PEMC and when the duration of the PIN expires, the PEMC can directly delete the PIN locally and without having to be authorized by the PIN server. After the PIN is deleted by PEMC, the PEMC can update the status of PIN to the PIN server.

8.5.3.2	Procedure

8.5.3.2.1	PIN delete procedure decided by PIN server

Figure 8.5.3.2.1-1 illustrates PIN delete procedure triggered by PEMC based on request/response model.

Pre-conditions:

1.	PIN is successfully created and in use;

2.	PEMC of the PIN decides to delete the PIN which could be based on the request from the authorized user or for any other reason which are implementation specific.

Figure 8.5.3.2.2-1: PIN delete procedure triggered by PEMC

1a.	The PEMC sends a PIN delete request to the PIN server to request to delete the PIN. The PIN delete request includes the security credentials of the PIN client received during PIN client authorization procedure and PIN ID. The PIN ID identifies the PIN to be deleted.

1b.	An event occurs at the PIN server that satisfies the trigger conditions for deleting the PIN. This event could the PIN continues to exist post the expiry of duration associated with it or the PIN server decides to not provide any PIN service in this PIN.

NOTE:	The security credentials delivery and the authorization procedure happen at application layer.

2.	Upon receiving the request, the PIN server validates the PIN delete request and verifies the security credentials.

3.	Upon successful authorization, the PIN server sends a successful PIN delete response to PEMC.

After the PIN is deleted which is indicated by PIN ID, the access control information in PEGC is also disabled and the PIN elements in this PIN cannot access to 5GS via PEGC anymore.

4.	When the PIN is deleted, the PEMC sends the PIN delete notification request to the PEGC containing the PIN ID of the deleted PIN.

5.   PEMC sends the PIN delete notification request to the PIN elements containing the PIN ID of the deleted PIN.

6.	The PEGC sends the PIN delete notification response to acknowledge the receipt of the notification and  disables the 5GS connection permission and access control information for the PIN elements in this PIN.

7.   The PIN elements in this PIN sends the PIN delete notification response to acknowledge the receipt of the notification. The PIN elements in this PIN can delete the information about this PIN, for example, the PIN profile.

8.5.3.2.2	PEMC decided PIN deletion

The PEMC can directly trigger the PIN delete procedure, for example, when the the duration assicated with the PIN expires, without having to be authorized from PIN server. After the PIN is deleted successfully, the PEMC updates the PIN status to PIN server.

Figure 8.5.3.2.2-1 illustrates PIN delete locally procedure triggered by PEMC based on request/response model.

Pre-conditions:

1.	PIN is successfully created and in use;

2.	PEMC of the PIN decides to delete the PIN.

Figure 8.5.3.2.2-1: PIN delete locally by PEMC

0.	The duration associated with the PIN expires and PEMC decides to delete the PIN.

1.	The PEMC deletes the PIN which indicated by PIN ID locally. The PEGC information and the information related to this PIN such as PIN profile, PIN dynamic profileare also deleted.

2-5.	The same procedures as step 4-7 defined in Figure 8.5.2.2.1-1.

6.   The PEMC sends the PIN delete notification request to the PIN server that the PIN is deleted locally and this notification request contains the PIN ID of the deleted PIN.

7.	The PIN server sends the PIN delete notification response to acknowledge the receipt of the notification.

8.5.3.3	Information flows

8.5.3.3.1	General

The following information flows are specified for PIN creation:

-	PIN delete request and response;

-	PIN delete notification request and response;

8.5.3.3.2	PIN delete request

Table 8.5.3.3.2-1 describes information elements in the PIN delete request from the PEMC to the PIN server.

Table 8.5.3.3.2-1: PIN delete request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the deleted PIN. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |

8.5.3.3.3	PIN delete response

Table 8.5.2.3.3-1: PIN delete response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the PIN delete request was successful. |
| Failure response | O | Indicates that the PIN delete request failed. |
| > Cause | M | Provides the cause for PIN delete request failure. |

8.5.3.3.4	PIN delete notification request

Table 8.5.3.3.4-1: PIN delete notification request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the deleted PIN. |

8.5.3.3.5	PIN delete notification response

Table 8.5.3.3.5-1: PIN delete notification response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the PIN delete notification request was successful. |
| Failure response | O | Indicates that the PIN delete notification request failed. |
| > Cause | O | Indicates the cause of PIN delete notification request failure. |

8.5.4	Multiple PEMCs/PEGCs

8.5.4.1	General

This clause describes the procedures for the configuration and use of multiple PEMCs and PEGCs in a PIN.

For a PIN having a large number of PIN elements, covering a large area, and/or requiring extra reliability, multiple PIN Elements may be assigned the role of PEMC and/or PEGC.

8.5.4.2	Procedure

##### 8.5.4.2.1	PIN configuration with default and backup PEGCs

Figure 8.5.4.2.1-1 illustrates a procedure for configuring a new PIN Element, when requesting to join, with a default PEGC and one or more backup PEGCs.

Pre-conditions:

1.	The PEMC is pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server.

2.	The PEMC is authorized to access the PIN server.

3.	The PEMC has registered to the PIN server.

4.	The PEMC has requested and has been authorized by the PIN server to create a PIN.

5.	The required KPIs of the PIN have been configured.

6.	PINE-1, PINE-2, PEGC-A and PEGC-B have registered to the PEMC and PIN server.

7. 	PINE-1, PINE-2, PEGC-A and PEGC-B have subscribed with the PEMC for the PIN status as specified in clause 8.5.X.2.1.

Figure 8.5.4.2.2-1: Multiple PEGC configuration for a PIN

1.	PEGCs (e.g., PEGC-A and PEGC-B) shall join the PIN by sending PIN management PINE join into PIN requests to the PEMC. The PIN Client Profile information (supported PIN roles, supported PEGC KPIs, Supported PEGC schedule, UE location) shall be included in the requests. The PEMC shall use this information to determine whether multiple PEGCs are needed to meet the required KPIs of the PIN and which PIN elements (e.g., PEGC-A and PEGC-B) are able to serve as PEGCs. The PEMC shall configure multiple PEGCs (e.g., PEGC-A and PEGC-B) with a role of PEGC if needed. The PEMC shall then send a PIN profile update to the PIN server informing the PIN server that of the multiple PEGCs.

2.	When PINEs (e.g., PINE-1) send a PIN management PINE join into PIN request to the PEMC, PIN Client Profile information (application client KPIs, application client schedule, UE location) shall be included in the request. The PEMC shall use this information and the PEGC information the PEMC receives in step 1 to determine the optimal default and backup PEGCs for the PINE. For example, PEMC assigns PEGC-A as the default PEGC for PINE-1 and PEGC-B as the backup PEGC for PINE-1.

3.	The PEMC shall send a default PEGC (e.g., PEGC-A) PIN status notification to update its local PIN Profile information to reflect that a PINE (e.g., PINE-1) is authorized to relay PIN communications via the PEGC.

4.	The PEMC shall send any backup PEGCs (e.g., PEGC-B) PIN status notification to update their local PIN Profile information to reflect that a PINE (e.g., PINE-1) is authorized to relay PIN communications via the PEGC.

5.	The PEMC shall return a PIN management PINE join into PIN response to PINE-1. The response shall include updated PIN client profile information informing the PINE that it shall use a specified PEGC (e.g., PEGC-A) as its default PEGC and a specified PEGC (e.g., PEGC-B) as its backup PEGC.

6.	The PEMC shall send a PIN profile update to the PIN server informing the PIN server of a PEGC (e.g., PEGC-A) serving as the default PEGC for a PINE (e.g., PINE-1) and any PEGCs (e.g., PEGC-B) serving as backup PEGCs for a PINE.

7.	Steps 2 to 6 may be repeated for any additional PINEs joining the PIN. For example, based on PIN client profile information provided by PINE-2, the PEMC configures PEGC-B to serve as the default PEGC for PINE-2 and PEGC-A to serve as the backup PEGC for PINE-2.

##### 8.5.4.2.2	PIN management with multiple PEMCs

Figure 8.5.4.2.2-1 illustrates a procedure for managing the PIN with multiple PEMCs. PIN elements when registering with the PIN indicates that they have the capability to act as PEMC and are assigned with primary or secondary PEMC role as described in the subclause 8.4.2.2.1.

Pre-conditions:

1.	A PIN owner or admin creates a PIN and configured multiple PEMCs.

2.	PEMC-S is assigned with the role of secondary PEMC and PEMC-P is assigned with the role of primary PEMC.

3.	PEMC-S has direct connection or PIN direct connection with the PEMC-P.

Figure 8.5.4.2.2-1: PIN management with multiple PEMCs

1. 	PEMC-S receives a request from PIN owner or PIN admin to perform any of the PIN management operations. These operations could be PIN element removal, PIN element addition, PIN deletion, PIN configuration update etc.

2. 	PEMC-S prepares the corresponding request including the required details for the requested operation and sends it to PEMC-P. These requests shall carry the PIN client ID of the PEMC-S mandatorily.

3. 	PEMC-P on receiving the request checks whether the PEMC-S is authorized as secondary PEMC in-order to perform the operation.

4. 	If the authorization succeeds, PEMC-P proceeds with the requested operation.

5. 	PEMC-P sends the response to PEMC-S containing the status or result of the requested operation.

NOTE:	Only the operations that are required to be performed by the PIN owner or PIN admin can be performed through secondary PEMC and all other operations like PIN discovery, PIN service discovery, authorizing PIN elements to join PIN etc., cannot be handled by the secondary PEMC.

8.5.4.3	Information flows

8.5.4.3.1	General

8.5.5	PIN Profile Recovery

8.5.5.1	General

8.5.5.2	Procedure

8.5.5.3	Information flows

8.5.6	Credential Provision

8.5.6.1	General

8.5.6.2	Procedure

8.5.6.3	Information flows

8.5.7	PIN discovery

8.5.7.1	General

Before the PINE trigger the PINE join into the PIN, the PINE should discover the available PIN.

For a certain PIN element, the PIN should be discovered and the PIN element can decide whether to join in the PIN. There are two situations that the PIN elements can discover the PIN as following:

-	If the PIN elements can have an application layer communication with the PEMC which manages a PIN, the PIN elements can receive the PIN ID, PIN description and the PIN service that a PIN can provide, and decides whether to join the PIN;

-	The PEGC can be set as open access and the PIN element can communicate with PIN server to receive the PIN ID, PIN description and the PIN service that a PIN can provide from PIN server via the PEGC.

8.5.7.2	Procedure

8.5.7.2.1	Procedures of PIN discovery based on PEMC

Pre-conditions:

1.	The UE Identifier or PIN client Identifier is available;

2.	The PIN client has been authorized to communicate with the PEMC;

3.	The UE or PIN client has already received the role of PEMC from PIN server;

4.	The PIN element has already had an application layer connection with a PEMC which manages a PIN.

If the PIN element has already had a communication connection with the PEMC, which managements of a PIN, the PIN element can receive the PIN ID, PIN description information and IP address of PEMC and decides whether to join the PIN.

Figure 8.5.7.2.1-1: Procedures of PIN discovery based on PEMC

1. 	The PIN client sends the PIN discovery request to PEMC. The PIN discovery request includes the security credentials of the UE or PIN client and may include the UE identifier such as GPSI if available, PIN client ID, UE location, Security credentials, the service that PINE wants to consume and PIN client profile(s) information

2.	The PEMC performs an authorization check to verify whether PINE is allowed to perform the operation.

3.	The PEMC sends the PIN discovery response to PIN element including the configuration information of the PIN(s), which are offering the services requested by the PINE in the PIN discovery request. The configuration information includes PIN ID, PIN description (Human-readable description of the PIN, for example, the company name, location or the type of service), services that each PIN can provide and the PEMC IP address.

The PIN client receives the configuration information and decides whether to join in the PIN.

8.5.7.2.2	Procedures of PIN discovery with assistance of PIN server via PEGC

Due to for some of the PIN elements can have the application interaction towards the PEGC, for example, via WiFi or Bluetooth pairing, and the PIN element can have the communication with PIN server to receive the lists of PIN ID and corresponding PIN description information. And the PIN elements can decide whether to join in the PIN.

Figure 8.5.7.2.2-1 illustrates PIN server discovery via PEGC based on request/response model.

Pre-conditions:

1.	The PIN elements or PIN client has application layer connection with PEGC;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PEGC has been authorized to communicate with the PIN server;

4.	The PIN element has already had an application layer connection with a PEGC, and the PEGC can route the PIN discovery request to PIN server.

5.	The PINE has already received the PIN server IP address.

Figure 8.5.7.2.2-1: PIN discovery based on PIN server

1.	The PIN client sends the PIN discovery request to via PEGC PIN server. The PEGC can route the request to PIN server. The PIN discovery request includes the security credentials of the UE or PIN client and may include the UE identifier such as GPSI, PIN client ID, UE location, Security credentials, the service that PINE wants to consume and PIN client profile(s) information.

The PIN client can have the filter information in the PIN discovery request for example, the interesting area, the interesting type of PIN and etc.

2.	Upon receiving the request, the PIN server performs an authorization check to verify whether the PIN client has authorization to perform the operation.

3.	The PIN server sends a successful response to PIN client via PEGC, which includes the configuration information to PIN elements. The configuration includes PIN ID, PIN description (Human-readable description of the PIN, for example, the company name, location or the type of service), services that each PIN can provide and the corresponding PEMC IP address.

8.5.7.3	Information flows

8.5.7.3.1	General

The following information flows are specified for PIN discovery:

-	PIN discovery request and response;

8.5.7.3.2	PIN discovery request

Table 8.5.7.3.2-1 describes information elements in the PIN discovery request from the PINE to the PEMC/PIN server.

Table 8.5.3.3.2-1: PIN discovery request

| Information element | Status | Description |
| --- | --- | --- |
| PIN client ID | M | The identifier of the PIN client |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client profile(s) | O | Profiles of PIN clients. The PIN client profiles are further described in Table 8.2.2.3. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.2.7. |
| Request PIN service | O | Indicate the service that PINE wants to consume in the PIN |

8.5.7.3.3	PIN discovery response

Table 8.5.7.3.3-1 describes information elements in the PIN discovery response from the PEMC/PIN server to PINE.

Table 8.5.3.3.3-1: PIN discovery response

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the the PIN. |
| PIN Description | O | Human-readable description of the PIN, for example, the company name, location or the type of service. |
| PIN service | O | List of service that a PIN can provide, including the PINE service or the service that can provided by application client on PINE: > PIN service Provider Identifier > PIN service type > PIN service Feature |
| PEMC information | O | Including the address or identifier of PEMC |

8.5.8	PINE management

8.5.8.1	General

After the PIN is created by the PEMC, the other PIN elements can be added into the PIN. For the PIN elements that have already added into the PIN, they can be removed from a certain PIN by the PEMC.

When a PIN element is added into a PIN, the PEMC should configure with the PIN elements with the necessary permission, for example, to be able to access to 5GS via the PEGC.

Before the PIN element to be added into a PIN, the PEMC should discover the PIN element first, or the PIN element receives the lists of PIN Profile first (for example, the list of PIN ID, PIN description information).

When a PIN element is added into a PIN, the PINE can indicate the service it can provide. The service includes both the service that PIN client in PINE can provide and the service that application client on PINE can provide.

8.5.8.2	Procedure

8.5.8.2.1	PIN client requests to join into a PIN

Figure 8.5.8.2.1-1 illustrates procedure of PIN client requests to join into a PIN, based on request/response model.

Pre-conditions:

1.	The UE (PIN client) has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PEMC;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC;

4.	The PIN client has already received the list of PIN ID, corresponding PEMC IP address;

Figure 8.5.8.2.2-1: PIN client requests to join into a PIN

1.	he PIN element already has an application layer connection with a PEMC which manages the PIN.

NOTE:	If the PIN client communicates with PEMC via WiFi or Bluetooth provided by PEMC, the PEMC can identify the PIN elements by the account number and password that PEMC distributed and the PIN client ID, UE GPSI if available.

And the PEMC has already send the PIN profile to PIN elements. The configuration includes PIN ID, PIN description (Human-readable description of the PIN, for example, the company name, location or the type of service) and the PEMC IP address.

2.	The PIN client sends the PIN Management PINE join into PIN request to PEMC to join the PIN. The request includes the security credentials of the PIN client received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID, UE location, PIN ID and PIN client profile(s) information as shown in Table xxx.

The request also includes service that PINE can provide. In the request, both the service that PIN client in PINE can provide and the service that application client on PINE can provide.

Editor's note: The authorization procedure between PIN client and PEMC should be captured in SA3 scope.

3.	Upon receiving the request, the PEMC performs an authorization check to verify whether the PIN client has authorization to join the PIN.

4.	The PEMC sends a successful PIN Management PINE join into PIN response to PIN client. Also, the access control information for the PIN client is also included, for example, user name, account, SSID, BSSID. All the information is used by PIN elements in PIN to access 5G or access the network provided by PEGC. The PEMC also provides lifetime of the PIN, identity, address of PEGC and may also provide unique PIN client ID to identify the PIN element within a PIN.

5.	The PEMC sends PIN status notification Request to the PIN server containing the details of the new PIN client that joined the PIN, including PIN client ID, GPSI and etc.

6.	The PEMC sends PIN status notification Request to the PEGC and other PIN elements containing the details of the new PIN client that joined the PIN. And the PEGC decides to enables the PINE to access 5GS.

7-9.	The PEMC/PEGC/PINE updates PIN profile with the details of the new PIN client that joined the PIN and the service that the PINE can provide.

10-11.		The PIN server/PEGC sends PIN status notification Response to the PEMC to acknowledge the receipt of the notification.

8.5.8.2.2	Procedure of PIN elements decides to leave the PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC;

Figure 8.5.8.2.2-1: PIN client decides to leave a PIN

1.	The PIN element already has an application layer connection with a PEMC which manages the PIN.

2.	The PINE decides to leave a PIN, and sends the PIN Management PINE leave from PIN request to PEMC to leave the PIN. The request includes the security credentials of the UE or PIN client received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID, UE location and PIN ID.

Editor's note: The authorization procedure between PIN client and PEMC should be captured in SA3 scope.

3.	The PEMC authorizes the request, and decides to remove a PIN client from a PIN which indicated by PIN client ID or UE GPSI.

4.	The PEMC sends the PIN Management PINE leave from PIN response to PINE to notify that the PIN client may not be included in the PIN anymore.

5-6.	The PEMC sends the PIN status notification Request to the PEGC/PIN server containing the details of the PIN client that requested to leave the PIN. The details of the PINE include PIN client ID, GPSI and etc.

7-9.	The PEMC/PEGC/PIN server updates the PIN profile and remove the details of the PIN client that requested to leave the PIN. The PEGC disables the access control information for this PINE.

10-11.		The PIN server/PEGC sends PIN status notification Response to the PEMC to acknowledge the receipt of the notification.

8.5.8.2.3	Procedure of PEMC removes the PIN elements from a PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC;

Figure 8.5.8.2.3-1: Remove a PIN element from a PIN by PEMC

1.	The PIN element already has an application layer connection with a PEMC which manages the PIN.

2.	The PEMC decides to remove the PINE (identified by GPSI, PIN client ID).

3.	The PEMC sends the PIN status notification Request to PINE, to notify that the PINE has been removed from the PIN.

4.	The PINE sends PIN status notification Response to the PEMC to acknowledge the receipt of the notification.

5-6.	The PEMC sends the PIN status notification Request to the PEGC/PIN server containing the details of the PIN client that removed from the PIN. The details of the PINE include PIN client ID, GPSI and etc.

7-9.	The PEMC/PEGC/PIN server updates the PIN profile and remove the details of the PIN client that requested to leave the PIN. The PEGC disables the access control information for this PINE.

10-11.		The PIN server/PEGC sends PIN status notification Response to the PEMC to acknowledge the receipt of the notification.

8.5.8.2.4	PINE join into PIN via PEGC

The PINE joins the PIN via the PEGC is depicted in Figure 8.5.8.2.4-1.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PEMC;

2.	The PINE already establishes the connection with PEGC;

3.	The PIN information to join is available at the PINE via e.g., PIN discovery procedure.

Figure 8.5.8.2.4-1: PINE join into PIN via PEGC

1.	The PINE sends PIN Management PINE join into PIN request to the PEGC. The PIN Management PINE join into PIN request contains the PIN ID which identifies the PIN to join, PINE client ID and credentials if available, PEMC identify/PIN server address.

2.	The PEGC identifies the received message is the PIN Management PINE join into PIN request and perform the authorization. If authorized, the PEGC determines to forward the PIN Management PINE join into PIN request to the PEMC or the PIN server.

3.	The PEGC forwards the PIN Management PINE join into PIN request to the PEMC in step 3a, based on the PEMC identity in step 1 or by resolving the PIN ID. The PEMC authorized the PINE to join the PIN, and returns the PIN Management PINE join into PIN response to the PEGC in step 3b.

4.	(Optional) The PEGC forwards the PIN Management PINE join into PIN request to the PIN server in step 4a based on the PIN server address in step 1 or by resolving the PIN ID. The PIN server forwards the PIN Management PINE join into PIN request to the PEMC in step 4b, and the PEMC authorized the PINE to join the PIN, and returns the PIN Management PINE join into PIN response to the PIN server in step 4c. Further the PIN server return the PIN server to the PEGC in step 4d.

5.	After the join, the PEMC update the PIN and may notify other entities (e.g., existing joined members, PIN server). The PEMC triggers the PIN status update request to PEGC/PINE as indicated in step 5-6 of procedure Figure 8.5.8.2.2-1. And the PEMC may receive the PIN status update notification response from PEGC/PINE as indicated in step 10-11 of procedure Figure 8.5.8.2.2-1.

6.	The PEGC return the PIN Management PINE join into PIN response to the PINE.

8.5.8.2.5	PINE leave via PEGC

The PINE leaves the PIN via the PEGC is depicted in figure 8.5.8.2.5-1.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PEMC or PIN server;

2.	The PINE already establishes the connection with PEGC;

3.	The PEGC acts as the application layer relay as defined in 6.3.4

Figure 8.5.8.2.5-1: PINE leave via the PEGC

1.	The PINE determines to leave the PIN and sends PIN leave request to the PEGC. The PIN leave request contains the PIN ID which identifies the PIN to leave, PINE client ID and credentials if available, PEMC identify/PIN server address[optional].

2.	The PEGC identifies the received message is the PIN leave request and perform the authorization. If authorized, the PEGC determines to forward the PIN leave request to the PEMC or the PIN server.

3.	If PEGC determine to forward the PIN leave request to the PEMC in step 2, PEGC forwards the PIN leave request to the PEMC based on the PEMC identity in step 1 or by resolving the PIN ID.

The PEMC authorized the PINE to leave the PIN, and returns the PIN leave response to the PEGC.

4.	If PEGC determine to forward the PIN leave request to the PIN Sever in step2, the PEGC forwards the PIN leave request to the PIN server based on the PIN server address in step 1 or by resolving the PIN ID. The PIN server forwards the PIN leave request to the PEMC, and the PEMC authorized the PINE to leave the PIN, and returns the PIN leave response to the PIN server. Further the PIN server return the PIN leave response to the PEGC.

5.	Further, the PEMC updates the PIN and may notify other entities (e.g., existing joined members, PIN server).

6.	The PEGC returns the PIN leave response to the PINE.

8.5.8.3	Information flows

8.5.8.3.1	General

The following information flows are specified for PIN creation:

-	PIN Management PINE join into PIN request and response;

-	PIN Management PINE leave from PIN request and response;

8.5.8.3.2	PIN Management PINE join into PIN request

Table 8.5.8.3.2-1 describes information elements in the PIN Management PINE join into PIN request from the PINE to PEMC, from PINE to PEGC, from PEGC to PEMC, from PIN server to PEMC.

Table 8.5.8.3.2-1: PIN Management PINE join into PIN request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the PIN that wants to join in. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client ID | M | The PIN client ID of PINE |
| PIN client profile(s) | O | Profiles of PIN clients. The PIN client profiles are further described in Table 8.2.2.3. |
| PIN server endpoint information | O | Includes URI(s), FQDN(s), IP address(es)) of PIN server. |

8.5.8.3.3	PIN Management PINE join into PIN response

Table 8.5.8.3.3-1 describes information elements in the PIN Management PINE join into PIN response from the PEMC to PINE, from PEGC to PINE, from PEMC to PEGC, from PEMC to PIN server.

Table 8.5.8.3.3-1: PIN Management PINE join into PIN response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the PIN Management PINE join into PIN request was successful. |
| > Updated PIN client profile | O | PIN client profile information updated by the PEMC (e.g., default and backup PEGCs assigned to PINE). |
| > Identifier of PEGCs | O | Indicates the PINE identifier authorized to be the PEGCs of this PIN. |
| >> PEGC address | O | Assigned IP address or port number of PEGC |
| > PEGC information | O | Includes the PEGC information for example, |
| >> Access control information | O | Includes: user name, account, SSID, BSSID. All the information is used by PIN elements in PIN to access 5G or access other application outside of PIN |
| Failure response | O | Indicates that the PIN Management PINE join into PIN request failed. |
| > Cause | M | Provides the cause for PIN Management PINE join into PIN request failure. |

8.5.8.3.4	PIN Management PINE leave from PIN request

Table 8.5.8.3.4-1 describes information elements in the PIN Management PINE leave from PIN request from the PINE to PEMC.

Table 8.5.8.3.4-1: PIN Management PINE leave from PIN request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the PIN that wants to leave from. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client ID | M | The PIN client ID of PINE |

8.5.8.3.5	PIN Management PINE leave from PIN response

Table 8.5.8.3.5-1 describes information elements in the PIN Management PINE leave from PIN response from the PEMC to PINE.

Table 8.5.8.3.5-1: PIN Management PINE leave from PIN response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the PIN Management PINE leave from PIN request was successful. |
| Failure response | O | Indicates that the PIN Management PINE leave from PIN request failed. |
| > Cause | M | Provides the cause for PIN Management PINE leave from PIN request failure. |

8.5.9	PIN status subscription

8.5.9.1	General

The PIN status update is used by the PINE, PEGC and PIN server to be notified of PIN status information by the PEMC.

PIN status information includes the following information:

-	PINE add/remove into/from the PIN;

-	PEGC/PEMC relocation in the PIN;

-	PIN profile update;

8.5.9.2	Procedure

8.5.9.2.1	PIN status subscribe

Figure 8.5.9.2.1-1 illustrates the PIN status subscribe procedure.

Pre-conditions:

1.	The PINE/PEGC/PIN server has already received the address of PEMC;

2.	The PINE/PEGC/PIN server has been authorized to communicate with PEMC;

Figure 8.5.9.2.1-1: PIN status subscribe

1.	The PINE/PEGC/PIN server sends the PIN status subscribe request to the PEMC. The PIN status subscribe request includes the PIN client ID along with the security credentials, Event ID.

2.	Upon receiving the request from the PINE/PEGC/PIN server, the PEMC checks if the PINE/PEGC/PIN server is authorized to subscribe for information of the requested PIN status information. The authorization check may apply to an individual PIN. If the request is authorized, the PEMC creates and stores the subscription for PIN.

3.	If the processing of the request is successful, the PEMC sends a PIN status subscribe response to the PINE/PEGC/PIN server, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the PEMC shall send an PIN status information subscription update request prior to the expiration time. If a new PIN status subscribe request is not received prior to the expiration time, the PEMC shall treat the PINE/PEGC/PIN server as implicitly unsubscribed.

8.5.9.2.2	PIN status notify

Figure 8.5.9.2.2-1 illustrates the PIN status notify procedure.

Pre-conditions:

1.	The PINE/PEGC/PIN has subscribed with the PEMC for the PIN status as specified in clause 8.5.X.2.1.

Figure 8.5.9.2.2: PIN status notify procedure

1.	When an event occurs at the PEMC that satisfies trigger conditions for notifying (e.g. to provide updated PIN status, for example when a PINE joins into the PIN) a PINE/PEGC/PIN server.

The status information of PIN is updated including the following cases:

-	PIN modification: The PIN modification includes the PEMC/PEGC relocation and the PINE joins into PIN or leaves the PIN;

-	PIN profile updates: The PIN profiles is referred to the section 8.2.2, and the updated PIN profile may be coordinated to other PINEs;

2.	The PEMC sends an PIN status notify to the PINE/PEGC/PIN server with the updated PIN status information.

The PIN status information notification includes the following parameters:

-	PIN modification: If the PEMC decides the PEMC/PEGC relocation, the PEMC includes the parameters about the newly assigned PEMCs and PEGCs in the PIN status update request, for example, the PEMC/PEGC ID or address, the PEGC information in Table 8.5.2.3.3-1 about access control information and etc;

-	PINE joins or leave the PIN: If the PEMC decides the PINE to join or leave the PIN, the PEMC includes the parameters about the newly added/removed PINE in the PIN status update request, for example, the PIN client ID or address, the PIN client profile and etc;

-	PIN profile update: If the PIN profile is updated, the PEMC includes the parameters about updated PIN profile in the PIN status update request;

3.	Upon receiving the notification, the PINE, PEGC or PIN server updates the PIN profile according to the information in the PIN status notify.

Editor’s Note: PIN status update and PIN status delete procedures are FFS.

8.5.9.3	Information flows

8.5.9.3.1	General

The following information flows are specified for PIN status:

-	PIN status information subscription

-	PIN status notification;

8.5.9.3.2	PIN status subscribe request

Table 8.5.9.3.2: PIN status subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| PIN client ID | M | Unique identifier of the PINE/PEGC/PIN server. |
| Subscribed Events | M | Identifies PIN status event types for which the subscriber is notified, more than one PIN status event type can be provided. PIN status event types - PIN modification - PINE management (PINE join/leave) - PIN profile update |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN ID | M | The identifier of PIN. |
| Notification Target Address | O | The Notification target address (e.g. URL) where the notifications destined for the PINE/PEGC/PIN server should be sent to. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

8.5.9.3.3	PIN status subscribe response

Table 8.5.9.3.3: PIN status subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

8.5.9.3.4	PIN status notify

Table 8.5.9.3.4-1 describes information elements in the PIN status notify from the PEMC to PEMC/PEGC/PINE/PIN server.

Table 8.5.9.3.4-1: PIN status notify

| Information element | Status | Description |
| --- | --- | --- |
| PIN Status event type | M | Identifies PIN status event types contained in the notification; more than one PIN status event type can be provided. PIN status event types - PIN modification - PINE management (PINE join/leave) - PIN profile update |
| PINE management | O | Indicates the PINE join/leave the PIN, this IE is present if the PIN status event type is PINE management. |
| > PINE join into the PIN | O | Indicates the PINEs join into the PIN |
| >> PIN client ID | M | The identifier of the joined PINE |
| >> PIN client profile(s) | O | Profiles of PIN clients that joins the PIN. The PIN client profiles are further described in Table 8.2.2.3. |
| > PINE leaves the PIN | O | Indicates the PINEs leaves from the PIN |
| >> PIN client ID | M | The identifier of the leaved PINE |
| PIN modification | O | Indicates the modification of PIN, this IE is present if the PIN status event type is PIN modification. |
| > Updated PIN profile | M | Updated PIN profile information that includes identifiers of PIN clients authorized to use as PEGC or PEMC. |
| >> PEGC information | O | Includes the PEGC information defined in Table 8.5.2.3.3-1. |
| >> PEMC information | O | Includes the PEMC information. |
| >>> PEMC address | O | Assigned IP/MAC address or port number of PEMC |
| >>> Identifier of PEMCs | O | Indicates the PINE identifier authorized to be the PEMCs of this PIN. |
| PIN profiles update | O | Indicates a PIN profile update that indicated in section 8.2.2, this IE is present if the PIN status event type is PIN profile update. |

8.5.10	PIN modification

8.5.10.1	General

This clause describes PIN modification functionality.

8.5.10.2	Procedures

8.5.10.2.1	General

8.5.10.2.2	PIN modification after local PEMC failure

Figure 8.5.10.2.2-1 describes the PIN modification procedure to perform a PEMC role change due to the failure of the PEMC. An authorised administrator is the owner of the PIN and accesses PIN configuration using an application on a UE, which is one of the PEMC for the PIN. The authorised administrator can manage the PIN locally or through the 5G network. This procedure describes a PEMC (e.g. an authorized administrator on a UE) managing the PIN remotely via the 5G network.

The procedure may used e.g. when a PEMC is available on a UE for PIN management by an authorised administrator. When there is a local PEMC failure, the authorized administrator can be enabled to manage the PIN remotely, via the 5G network, using the following steps.

Pre-conditions:

1.	The PIN server has authorised the creation of the PIN.

2.	The authorised administrator is the owner of the PIN and has created the PIN.

3.	The authorised administrator configures PEMC2 to provide PIN management for the PIN. PEMC2 becomes the active PEMC and UE/PEMC1 is the inactive PEMC.

4.	PEMC2, PEGC, PINE-1, PINE-2, and the UE/PEMC1 are members of the PIN. PINE-1 has PEMC capability.

5.  The authorised administrator leaves the local area of the PIN (e.g. in a home) and is able to access the PIN remotely through the 5G network. As a result, the authorised administrator is able to manage the PIN through the 5G network

Figure 8.5.10.2.2-1: PIN Modification after local PEMC failure

1.	 UE/PEMC1 receives a notification from PEGC that there is a communication failure with PEMC2.

2.	An authorised administrator on UE/PEMC1 sends a PIN_configuration_request to the PIN server through the 5G network. The request includes the security credentials of the authorised administrator, the UE ID, the PIN ID, PIN member ID, authorization type indicating the role change, and a request that PINE-1 be assigned the new PEMC.

3.	The PIN server processes the modification request and checks if the authorised administrator is allowed to modify the PIN. The PIN server verifies PINE-1 has the capability to serve as a PEMC using information in the PIN profile.

4. 	If the authorised administrator is allowed to perform PIN modification, the PIN server sends a PIN_management_request to PINE-1 to assign PINE-1 as the new PEMC and provides PIN profile and dynamic profile information to PINE-1.

Editor's Note: Usage of PIN profile information and PIN dynamic profile information in the information flows, and the procedures where each of these IEs are applicable are FFS.

5.	PINE-1 returns an accept response in the PIN_management_response to the PIN server.

6.	The PIN server notifies the other members of the PIN that PINE-1 will be the new PEMC for the PIN and updates the PIN profile and dynamic profile information.

7.	The PIN server sends a PIN_configuration_response to the PIN_configuration_request with updated information for the PIN profile and the dynamic profile information to the UE/PEMC1.

8.	PIN communications resume with PINE-1 serving as the new PEMC.

8.5.10.2.3	PIN modification with PEGC role change

Figure 8.5.10.2.3-1 describes a PIN modification procedure to perform a PEGC role change due to the unavailability of the PEGC. This procedure describes a PEMC detecting the unavailability of a PEGC (e.g. PEGC leaves the local service area of the PIN) and performing a PIN modification with the PIN Server to assign a new PEGC. As part of PIN management, a PEMC subscribes to receive periodic notifications from PEGCs to ensure PIN routing is available for members of the PIN at all times. If a PEMC does not receive the periodic notifications from the PEGC, then the PEMC needs to assign a new PEGC or request the PIN server to assign the new PEGC.

Pre-conditions:

1.	The PIN server has authorized the PEMC to create PINs.

2.	The PEMC creates a local PIN with members: PEMC, UE serving as PEGC, PINE1, and PINE2.

3.	The PEMC maintains a PIN profile with information on the capabilities of each PIN member.

4.	PINE2 is a PIN member that also has gateway capability.

Figure 8.5.10.2.3-1: PIN Modification due to PEGC unavailability

1.	PEMC subscribes to receive periodic notifications from PEGC to monitor the availability of PIN routing service provided by PEGC.

NOTE:	The subscription/ notification is for further development.

Editor's Note: How the PEMC detects the unavailability of PEGC is FFS.

2.	PEGC leaves the local coverage area of the PIN, e.g., leaves the home, and is not available to route PIN communications.

3.	PEMC does not receive a notification from PEGC at the configured interval and determines that PEGC is no longer providing PIN routing capability.

4.	PEMC sends a PIN_configuration_request to the PIN server to select a new PEGC. The request includes the PIN ID, the PEMC ID, the PEGC ID, authorization type indicating the role change, the ID of a PIN member that can serve as the new PEGC (e.g., PINE2), and a timestamp.

5.	The PIN server considers which member of the PIN can serve as the new PEGC, including the PIN member the PEMC provided, and selects a PIN member to serve as the new PEGC. The PIN server sends a PIN_management_request to PINE-2 with PIN profile and dynamic profile information. The dynamic profile information includes PIN traffic routing rules that PINE2 would need to make routing decisions.

6.	PINE-2 sends a PIN_management_response accepting to serve as the new PEGC.

7.	The PIN server sends a PIN_configuration_response with PIN profile and dynamic profile information to the PEMC with the status of the request, the ID of the new PEGC, and PIN traffic routing rules. The PIN server response triggers the PEMC to notify other PIN members of the PEGC role change.

8.	PEMC notifies the other members of the PIN that PINE2 will serve as the new PEGC. The PEMC includes PIN profile and dynamic profile information that includes traffic routing rules applicable to each member.

8.5.10.2.4	PEMC replacement triggered internally within the PIN

A PIN element could have been authorized to act as PEMC for a certain duration after which it is either removed from the PIN or de-authorized to act as PEMC. Another PIN element in the PIN takes over the role of PEMC.

When the duration of its role as PEMC is expiring or for some other reasons (implementation specific) the current PEMC requests another PINE to take the role of PEMC. Once the role assignment succeeds, the PIN server and other PIN elements including PEGC are notified of this role change.

Figure 8.5.2.10.2-4-1 illustrates PEMC replacement procedure triggered internally within the PIN by current PEMC.

Pre-conditions:

1.	PEMC-2 PIN element has already indicated that it can act as PEMC during the registration process.

2.	Dynamic profile information about the PIN is available at the current PEMC and PIN server

Figure 8.5.2.10.2.4-1: PEMC replacement triggered internally within the PIN

1.	PEMC-1, PEMC-2, PEGC, PINE-1 and PINE-2 are part of same PIN. PEMC-1 is currently the PEMC of the PIN.

2.	PEMC-1 decides to relinquish its PEMC role and handover to another PIN element. It may decide if it detects that its UE power is draining or its role as PEMC is nearing expiry.

3.	The PEMC-1 looks into the PIN dynamic profile information to identify the new PINE which can take up the role of PEMC (here PEMC-2 PIN element) and requests PEMC-2 to take the role of PEMC by sending PIN PEMC role takeover request which includes the PIN identifier, PIN element identifier of PEMC-2.

4.	If the PEMC-2 PINE decides to take up the role of PEMC it sends the PIN PEMC role takeover success response to the PEMC-1.

5.	The PEMC-1 updates the PIN dynamic profile information with this role change details and delivers the PIN dynamic profile information to the PEMC-2.

6.	The PEMC-1 signals all the PINEs in the PIN including the PEGC and PIN server about the change in the PIN element acting as PEMC and its reachability information by sending PIN status notification request. On receiving this notification PIN server and PEGC updates the PIN dynamic profile information with the details of PEMC-2.

8.5.10.2.5	PEGC replacement triggered by PEMC

In some scenarios, like hardware failure, crash or power drain, the current PEGC may not be in a position to indicate to the PIN server or request the PIN server to assign the role of PEGC to another PIN element. In these cases, the PEMC on detecting the unavailability of the PIN element acting as PEGC, need to assign the PEGC role to another PINE and deliver the PIN dynamic information to the new PEGC.

Figure 8.5.2.10.2-5-1 illustrates PEGC replacement procedure triggered by PEMC on detecting current PEGC is unavailable or PEGC role change is required.

Pre-conditions:

1.	PEGC-2 PIN element has already indicated that it can act as PEGC during the registration process.

2.	Dynamic information about the PIN is available at the PEMC.

3.	PIN has only one PIN element configured with the role of PEGC.

Figure 8.5.2.10.2.5-1: PEGC replacement triggered by PEMC

1.	PEMC, PEGC-1, PEGC-2, PINE-1 and PINE-2 are part of same PIN. PEGC-1 is currently the PEGC of the PIN.

2.	The PEMC identifies that PEGC-1 is down or crash or its duration to act as PEGC is ending. How PEMC identifies that the current PINE acting as PEGC is crashed or down is not in the scope of this specification.

3.	The PEMC looks into the PIN profile and PIN dynamic information to identify the new PINE which can take up the role of PEGC (here PEGC-2 PIN element) and requests PEGC-2 to take the role of PEGC by sending PIN PEGC role takeover request and this request includes the PIN Element ID of PEGC-2, PIN identifier and time duration to act as PEGC etc.

4.	If the PEGC-2 PINE decides to take up the role of PEGC it sends the success PIN PEGC role takeover response to the PEMC.

5.	The PEMC notifies the PIN server that PEGC-2 is the new PEGC of the PIN and it is releasing PEGC-1 from its role as PEGC by sending PIN status notification request containing the required details.

6.	The PEMC and PIN server updates the PIN dynamic information with the relevant details of PEGC-2.

7.	The PEMC delivers the PIN dynamic information to the PEGC-2.

8.	The PEMC sends PIN status notification request to notify the relevant PINEs in the PIN about the change in the PIN element acting as PEGC and its reachability information.

#### 8.5.10.3	Information flows.

##### 8.5.10.3.1	General

##### 8.5.10.3.2	PIN_configuration_request

Table 8.5.10.3.2-1 shows the informational elements of the PIN configuration request sent by a PIN Element to the PIN server to obtain authorization for the modification of a PIN.

Table 8.5.10.3.2-1: PIN configuration request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN |
| Requester PINE ID | M | The identifier of the PIN Element making the request |
| Authorization type | M | Request for the authorization to modify the configuration of a PIN: PEMC role change, PEGC role change |
| PINE ID of predecessor in the role | M | PINE ID of the element serving in the indicated role and needs to be changed e.g. due to being unavailable. |
| PINE ID of proposed successor in the role | O | PINE ID of the element proposed to assume the role |

##### 8.5.10.3.3	PIN_configuration_response

Table 8.5.10.3.3-1 shows the informational elements of the PIN configuration response provided by the PIN server to authorize the modification of a PIN.

Table 8.5.10.3.3-1: PIN configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Response | M | The response (authorize or not authorize) from the PIN Server. |
| PIN profile information | M | The IEs from the PIN profile information that the PIN Server has updated for modifying the PIN |
| Dynamic PIN profile information | M | The IEs from the dynamic PIN profile information that the PIN Server has authorized for modifying the PIN |

##### 8.5.10.3.4	PIN_management_request

Table 8.5.10.3.4-1 shows the informational elements of the PIN management request sent to PIN elements to make changes to the configuration of the PIN.

Table 8.5.10.3.4-1: PIN management request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN |
| Requestor ID | M | The identifier of the PIN Server or PIN Element making the request |
| Modification type | M | Request for the modification of the PIN: PEMC assignment, PEGC assignment |
| Dynamic PIN profile information | M | IEs from the dynamic PIN profile information that the PINE needs to operate in the new role |

##### 8.5.10.3.5	PIN_management_response

Table 8.5.3.3.5-1 shows the informational elements of the PIN management response received from the PIN element to the PIN modification request.

Table 8.5.10.3.5-1: PIN management response

| Information element | Status | Description |
| --- | --- | --- |
| Response | M | The response (accept or deny) from the PIN Element to the PIN management request. |

8.6	PIN enable 5GS communication

8.6.1	General

For a certain PINE, it has the subscription whether the PINE has the permission to use 5GS to communicate.

The PIN client within a PIN can communicate with other devices, services and applications within the same PIN. Furthermore, PIN client can connect the 5G Network via a PEGC. Also, as a feature, some of the PIN client in PIN has the permissions that they can communicate with other UE or application outside of PIN with the help of 5GC.

-	There are two methods to enable the PIN with 5GS communication:

-	Establish QoS for PINE with AF support.

-	PEGC triggers PDU session establishment/modification for PINE.

For the AF related procedure, the AF trigger the QoS create/modification procedure with parameters of Packet filters, DN specific ID, to request the 5GS to arrange resource for PIN.

For PEGC related procedure, the PEGC sends PIN Communication Request to the PEMC. The PEMC sends Create/Update/Remove Communication Request (PIN ID, Packet filters, requested QoS) to the PEGC. The PEGC configures the local rule accordingly, or according to the Packet filters, the PEGC may initiate PDU Session Modification with the Packet filters and requested QoS towards 5G system in order to make 5GC configure the N4 rules for UPF(s).

8.6.2	Procedure

8.6.2.1	AF trigger QoS establishment

For the AF related procedure, the AF trigger the QoS create/modification procedure as defined in Section 4.15.6.6 of TS 23.502[x] with parameters of Packet filters (that related to the PINEs needs communication), DN specific ID, to request the 5GS to arrange resource for PIN.

8.6.2.2	Procedures of PIN communication via 5GS triggered by PEGC

Pre-conditions:

1.	The PINE is able to connected into PEGC via password and username;

2.	The UE Identifier or PIN client Identifier is available for PINE;

3.	The PIN client has been authorized to communicate with PEMC;

4.	The PINE has the subscription that it can communicate via 5GS;

5.	The PINE has already received the IP address of other PINE.

6.	The communication between PEMC and PEGC is available.

Figure 8.6.2.2-1: Procedures of PIN communication via 5GS triggered by PEGC

The PINE may have the traffic to send via 5GS communication.

1a.	The PINE sends PIN Communication Create/Update/Remove Request (PIN ID, MAC address/IP address, Traffic descriptors, Packet filters, requested QoS) to the PEGC.

1b.	The PEMC sends PIN Communication Create/Update/Remove Request (PIN ID, MAC address/IP address of PINE, Traffic descriptors, Packet filters, requested QoS) to the PEGC.

If the PEGC has already got the information to trigger the PDU session, the interaction between PEMC/PINE and PEGC will be saved. If the PEGC doesn’t have the related information (PIN ID, Packet filters, requested QoS), the procedure needs the PEMC to send related information to PEGC.

After the PEGC receives the PIN Communication Create/Update/Remove Request, the PEGC configures the local rule accordingly.

2.	[Optional] According to the Packet filters, the PEGC may initiate PDU Session Modification with the Packet filters and requested QoS towards 5G system in order to make 5GC configure the N4 rules for UPF(s).

NOTE: How to the PEGC interacts with 5GS to initiate PDU Session Modification is refered to section 4.3.3.2 of TS 23.502[x].

3a-3b.	The PEGC sends PIN Communication Create/Update/Remove Response to the PEMC or PINE.

8.6.3	Information flows

8.7	Service Switch

8.7.1	General

PIN service switch procedures enable a PIN Element participating in a PIN to transfer application session(s) to a different PIN Element participating in the same PIN. For example, a first PIN Element (e.g. a UE) can transfer a video streaming session to a second PIN Element (e.g. a television). PIN service switch can be triggered by a PIN Element when needed, for example when the first PIN Element joins a PIN and determines that an application flow can be switched to a second PIN Element present in that PIN.

Two scenarios are specified for PIN service switch:

-	PIN service switch with PIN server support

-	PIN service switch without PIN Server support (e.g. using only internal PIN communication).

8.7.2	Procedure

8.7.2.1	Service switch in a PIN with PIN server support

8.7.2.1.1	General

Following procedures are supported for service switch in a PIN with PIN server support:

-	PIN Service Switch procedure;

-	PIN Service Switch Configure procedure;

8.7.2.1.2	PIN Service Switch procedure

Pre-conditions:

1.	The PIN Client established an application session with an Application Server;

2.	The PIN Client joined a PIN.

Figure 8.7.2.1.1-1: PIN Service Switch procedure

1.	The PIN Client sends a PIN Service Switch request to the PIN Server. The PIN Service Switch request includes the requestor identifier [PIN Client ID], security credential, a PIN identifier [PIN ID], an Application Client identifier [ACID], an Application Server identifier, an application session identifier, can include an IP 4 tuple that describes the traffic of the application session and can include a target PIN Client [PIN Client ID] if a target is known.

2.	Upon receiving the request from the PIN Client, the PIN Server checks if the PIN Client is authorized to request service switching for the given Application Server and validates the request. If the request is authorized and valid, the PIN server can determine a target PIN Client if the target PIN Client was not included in the request. The PIN Server sends a PIN Configure Service Switch request to the PIN Management Client and the Application Server as in clause  8.7.2.1.2. The PIN Server can use the PIN ID to identify the PIN Management Client instance and the Application Server identifier, or IP 4 tuple, to identify the Application Server instance.

3.	If the processing of the request was successful, the PIN Server sends a PIN Service Switch response to the PIN Client that indicates the Service Swich request was successfully processed and can include the target PIN Client information if it was decided by the PIN Server. Otherwise, the PIN Server sends a PIN Service Switch response to the PIN Client indicating that the request processing failed and can include appropriate reasons.

Upon reciving the PIN Service Switch response, the PIN Client validates if the request was succesful and can indicate the result to the Application Client; the PIN Client can transfer the application context to the target PIN Client. If the PIN Service Switch response indicated a failure, the PIN Client can attempt perform service switch again for the same or a different target PIN Client considering the failure reason.

8.7.2.1.3	PIN Service Switch Configure procedure

Pre-conditions:

1.	The PIN Client successfully requested a service switch with the PIN Server

Figure 8.7.2.1.2-1: PIN Configure Service Switch procedure

1.	The PIN Server sends the PIN Configuration Service Switch Configure request to the PIN Management Client or Application Server. The PIN Configuration Service Switch Configure request includes the requestor identifier, security credential, Application Client identifier, Application Server identifier, target PIN Client, application session identifier and can include IP 4 tuple that descibes the traffic of the application session.

Editor's note: Whether subscription/notification mechanisms are used for informing the AS over PIN-9 reference point is FFS.

NOTE 1:	The PIN Server first performs PIN Configure Service Switch procedure with the PIN Management Client, then if successful, with the Application Server.

2.	Upon receiving the request from the PIN Server, the PIN Management Client or the Application Server checks if the PIN Server is authorized to request service switch configuration and validates the request.

If the request was recevied at the Application Server, if the request is authorized and valid, the Application Server uses the information provided in the request to switch the application session to the target PIN client and proceeds to step 6 of the procedure.

If the request was recevied at the the PIN Management Client, if the request is authorized and valid, the PIN Management Client prepares for sending PIN Configure Service Switch request to the PIN Gateway Client or the target PIN Client. The PIN Management Client can use the PIN ID to identify the PIN Gateway Client instance and the target PIN Client identifier to identify the target PIN Client.

3.	The PIN Management Client sends the PIN Management Service Switch Configure request to the PIN Gateway Client or the target PIN Client including the information defined in step 1.

NOTE 2:	The PIN Management Client first performs PIN Configure Service Switch procedure with the PIN Gateway Client, then if successful, with the target PIN Client.

4.	Upon receiving the request from the PIN Management Client, the PIN Gateway Client or the target PIN Client checks if the PIN Managment Client is authorized to request service switch configuration and validates the request. If the PIN Management Client is authorized and the request is valid, the PIN Gateway Client or the target PIN Client use the information provided in the request to switch the application session to the target PIN Client.

NOTE 3:	The target Application Client needs to receive the application context prior to re-establishing a switched application session with the Application Server.

NOTE 4:	How the target Application Client recovers the switched application session is out of scope of this specification.

5.	If the processing of the request was successful, the PIN Gateway Client or the target PIN Client send a PIN management Service Switch Configure response to the PIN Management Client indicating the processing was successful. Otherwise, the PIN Gateway Client or target PIN Client send a PIN Management Service Switch Configure response to the PIN Management Client indicating that processing the request failed and can include appropriate reasons.

6.	If the processing of the request was successful, the PIN Management Client or Application Server send a PIN Configuration Service Switch Configure response to the PIN Server that indicates the request was successfully processed. Otherwise, the PIN Management Client or Application Server send a PIN Configuration Service Switch Configure response to the PIN Server indicating the processing the request failed and can include appropriate reasons.

8.7.2.2	Service switch in a PIN without PIN server support

Editor’s Note: The procedure definition for Service Switch in a PIN without PIN server support is FFS

Figure 8.7.2.2-1: PIN Service Switch internal PIN

In Figure 8.7.2.2-1, it describes the service switch scenario supported internal PIN.

The PINE A has the application communication with application server. And when the PINE A decides to select other alternative PINE B to apply the traffic flow, the PINE A firstly should discover a PIN and join in. And then, in the PIN, there should exist the PINE that can be hosted with the same service type as PINE A, for example, the video flow, music flow or game flow.

The PINE A can send the request to PEMC to determine the PINE B to host the application traffic. After the determination, the PEMC sends the endpoint information to PINE A and the PINE A can offload the traffic either directly to PINE B or via PEGC.

Pre-conditions:

1.	The UE Identifier or PIN client Identifier of PINE A or PINE B is available;

2.	The PIN client in PINE A or PINE B has been authorized to communicate with the PEMC;

3.	The PIN client in PINE B can provide the same PIN service as PINE A’s traffic flow.

Figure 8.7.2.2-2: Service switch procedure internal PIN

0.	The PINE A has application layer communication with application server. And the PINE A decides to do the service switch to other PINEs. And the PINE A has already been in a PIN.

1.	The PINE A trigger the PIN service discovery request towards PEMC. This request carries the list of services the PINE A wants to consume.

2.	Upon receiving the request, the PEMC performs an authorization check to verify whether the PINE A has authorization to perform the operation.

3.	The PEMC provides the list of PINE endpoint(s), application client endpoint(s) information that are offering the requested services to PINE A in PIN service discovery response. The PIN service can be represented by service type that PINE provides or the application client on PINE. If the request fails, the PEMC should give the failure response to indicates the cause of request failure.

4.	The PINE A determines the PINE B for service switch as follow:

The PINE A maintains the service towards AS and the service corresponds to a service type, for example, music or streaming media. The PINE A determines the potential PINE B for service switch according to whether this PINE has the capability to carry the service or whether this PINE has deployed the same application client as PINE A.

5.	(Use PINE B as example) The PINE switches the service traffic to PINE B: The PINE A switches the traffic flow directly to PINE B via direct communication or via PEGC.

8.7.3	Information flows

Editor's note: Information flows definition for Service Switch is FFS

#### 8.7.3.1 	General

The following information flows are specified for Service Switch:

-	PIN service switch request and response

-	PIN configuration service switch configure request and response

-	PIN management service switch configure request and response

#### 8.7.3.2 	PIN service switch request

Table 8.7.3.2-1 describes information elements for the PIN service switch request that is sent from the PIN client to the PIN server.

Table 8.7.3.2-1: PIN service switch request

| Information element | Status | Description |
| --- | --- | --- |
| PIN client identifier | M | Requestor identifier. |
| Security credentials | M | Security credentials of the PIN client. |
| PIN identifier | M | Identifier of the PIN. |
| Application client identifier | M | Identifier of the application client. |
| Application server identifier | M |  |
| Application session identifier | M |  |
| Application session descriptor | O | Descriptor of application traffic flows (e.g., IP 4 tuple) |
| Target PIN client identifier (NOTE) | O | Target PIN client identifier. |
| NOTE: Only if target PIN client is known. |  |  |

#### 8.7.3.3 	PIN service switch response

Table 8.7.3.3-1 describes information elements for the PIN service switch response.

Table 8.7.3.3-1: PIN service switch response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE 1) | O | Indicates that the request was successful. |
| > Target PIN client identifier (NOTE 2) | O | Target PIN client identifier. |
| Failure response (NOTE 1) | O | Indicates that the request failed. |
| > Cause | O | Indicates the cause of the request failure. |
| NOTE 1: Only one of the IE must be included in the response. NOTE 2: Only if target PIN client is not provided in the PIN service switch request. |  |  |

#### 8.7.3.4 	PIN configuration service switch configure request

Table 8.7.3.4-1 describes information elements for the PIN configuration service switch configure request that is sent from the PIN server to the PIN management client and the application server.

Table 8.7.3.4-1: PIN configuration service switch configure request

| Information element | Status | Description |
| --- | --- | --- |
| PIN server identifier | M | Requestor identifier. |
| Security credentials | M | Security credentials of the PIN server. |
| PIN identifier | M | Identifier of the PIN. |
| Application client identifier | M | Identifier of the application client. |
| Application server identifier (NOTE 1) | O | Identifier of the application server. |
| Application session identifier | M | Identifier of the application session. |
| Application session descriptor | O | Descriptor of application traffic flows (e.g., IP 4 tuple) |
| Target PIN client identifier | M | Target PIN client identifier. |
| NOTE 1: The IE is present if the request is sent to the PIN management client. NOTE 2: The IE is present if the request is sent to the application server. |  |  |

#### 8.7.3.5 	PIN configuration service switch configure response

Table 8.7.3.5-1 describes information elements for the PIN configuration service switch configure response.

Table 8.7.3.5-1: PIN configuration service switch configure response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request failed. |
| > Cause | O | Indicates the cause of the request failure. |
| NOTE: Only one of the IE must be included in the response. |  |  |

#### 8.7.3.6 	PIN management service switch configure request

Table 8.7.3.6-1 describes information elements for the PIN management service switch configure request that is sent from the PIN management client to the PIN gateway client and the target PIN client.

Table 8.7.3.6-1: PIN management service switch configure request

| Information element | Status | Description |
| --- | --- | --- |
| PIN management client identifier | M | Requestor identifier. |
| Security credentials | M | Security credentials of the PIN management client. |
| PIN identifier | M | Identifier of the PIN. |
| Application client identifier | M | Identifier of the application client. |
| Application server identifier | M | Identifier of the application server. |
| PIN gateway client identifier (NOTE 1) | O | Identifier of the PIN gateway client. |
| Application session identifier | M | Identifier of the application session. |
| Application session descriptor | O | Descriptor of application traffic flows (e.g., IP 4 tuple) |
| Target PIN client identifier (NOTE 2) | O | Target PIN client identifier. |
| NOTE 1: The IE is present if the request is sent to the target PIN client. NOTE 2: The IE is present if the request is sent to the PIN gateway client. |  |  |

#### 8.7.3.7 	PIN management service switch configure response

Table 8.7.3.7-1 describes information elements for the PIN management service switch configure response.

Table 8.7.3.7-1: PIN management service switch configure response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request failed. |
| > Cause | O | Indicates the cause of the request failure. |
| NOTE: Only one of the IE must be included in the response. |  |  |

8.7.4	APIs

Editor's note: API definition for Service Switch in a PIN without PIN server support is FFS

#### 8.7.4.1 	General

Table 8.7.4.1-1 illustrates the API for service switch.

Table 8.7.4.1-1: ServiceSwitch API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ppsvr_ServiceSwitch | Request | Request/Response | PIN Client |
| Ppemc_ConfigurationServiceSwitchConfigure | Request | Request/Response | PIN Server |
| Ppegc_ManagementServiceSwitchConfigure | Request | Request/Response | PEMC |
| Ppine_ManagementServiceSwitchConfigure | Request | Request/Response | PEMC |

#### 8.7.4.2	 Ppsvr_ServiceSwitch API

##### 8.7.4.2.1	General

This clause describes the  Ppsvr_ServiceSwitch API and its operations.

##### 8.7.4.2.2	 Ppsvr_ServiceSwitch_Request operation

API operation name:  Ppsvr_ServiceSwitch_Request

Description: The consumer requests a service switch operation from the PIN Server.

Inputs: See clause 8.7.3.2.

Outputs: See clause 8.7.3.3.

See clause 8.7.2.1.2 for details of usage of this operation.

#### 8.7.4.3	 Ppemc_ConfigurationServiceSwitchConfigure API

##### 8.7.4.3.1	General

This clause describes the Ppemc_ConfigurationServiceSwitchConfigure API and its operations.

##### 8.7.4.3.2	 Ppemc_ConfigurationServiceSwitchConfigure_Request operation

API operation name:  Ppemc_ConfigurationServiceSwitchConfigure_Request

Description: The consumer requests a service switch configuration operation from the PIN management client.

Inputs: See clause 8.7.3.3.

Outputs: See clause 8.7.3.4.

See clause 8.7.2.1.3 for details of usage of this operation.

#### 8.7.4.4	 Ppegc_ManagementServiceSwitchConfigure API

##### 8.7.4.4.1	General

This clause describes the  Ppegc_ManagementServiceSwitchConfigure API and its operations.

##### 8.7.4.4.2	 Ppegc_ManagementServiceSwitchConfigure_Request operation

API operation name:  Ppegc_ManagementServiceSwitchConfigure_Request

Description: The consumer requests a service switch configuration operation from the PIN gateway client.

Inputs: See clause 8.7.3.3.

Outputs: See clause 8.7.3.4.

See clause 8.7.2.1.3 for details of usage of this operation.

#### 8.7.4.5	 Ppine_ManagementServiceSwitchConfigure API

##### 8.7.4.5.1	General

This clause describes the Ppine_ManagementServiceSwitchConfigure API and its operations.

##### 8.7.4.5.2	 Ppine_ManagementServiceSwitchConfigure_Request operation

API operation name: Ppine_ManagementServiceSwitchConfigure_Request

Description: The consumer requests a service switch configuration operation from the PIN client.

Inputs: See clause 8.7.3.3.

Outputs: See clause 8.7.3.4.

See clause 8.7.2.1.3 for details of usage of this operation.

8.8	Application server discovery in PIN

8.8.1	General

8.8.2	Procedure

8.8.3	Information flows

8.9	Service Continuity

8.9.1	General

PIN service continuity procedures enable an Application Client participating in a PIN to maintain service when PIN Elements enter or leave the PIN. The following examples illustrate scenarios where PIN service continuity is needed.

In a first scenario, a first PIN Element (e.g. a UE) receives data from a second PIN Element (e.g. a home sensor), both PIN Elements are participating in the same PIN. Service contnuity can be triggered when the UE leaves home in order to re-configure the connectivity between both PIN Elements.

In a second scenario, a PIN Element is communicating with an Application Server via a PIN Gateway Client.Service continuity can be triggered when the PIN Gateway Client becomes unreachable in order to find a replacement PIN Gateway Client and re-configure connectivity between the PIN Element, the replacement PIN Gateway and the Application Server.

Two scenarios are specified for PIN Service Continuity:

- 	PIN service continuity in PIN Gateway Client relocation;

- 	PIN service continuity in changing access to 5GS.

8.9.2	Procedure

8.9.2.1	Service continuity in PEGC relocation scenario

8.9.2.1.1	General

Following procedures are supported for service continuity in a PEGC relocation scenario:

-	PIN Management PEGC Service Continuity procedure;

- 	PIN Management PEGC Configuration procedure;

-	PIN Configuration Update procedure;

-	PIN Management PEGC Discovery procedure.

8.9.2.1.2	PIN Management PEGC Service Continuity procedure

Pre-conditions:

1.	PIN Element has an application session that goes through a PIN Gateway Client; and

2.	PIN Gateway Client needs to be replaced (e.g., PEGC will become unavailable or PEGC is unreachable).

Figure 8.9.2.1.2-1: PIN PEGC Service Continuity procedure

1.	The PIN Gateway Client or the PIN Client sends a PIN Management PEGC Service Continuity request to the PIN Management Client for a specific service. The PIN Management PEGC Service Continuity request includes security credentials, PIN Client identifier(s), PIN Gateway identifier, and Service identifier.

2	Upon receiving the request, the PIN Management Client validates the request and checks if the request is authorized for the given PIN Client(s), PIN Gateway Client and service. If the request is authorized and valid, the PIN Management Client determines if a target PIN Gateway Client meets the requirements for handling the application traffic for the provided service.

If the PIN Management Client identifes a target PIN Gateway Client that meets the requirements, the PIN Management Client sends PIN Management PEGC Configuration request to the target PIN Gateway Client to configure the target PIN Gateway for handling service traffic from the PIN Client(s) as in clause 8.9.2.1.3. If the service is provided by an Application Server, the PIN Management Client send a PIN Configuration Update request to the PIN Server if needed to inform of the service continuity changes in the PIN as in clause 8.9.2.1.4.

3.	If the procesing of the request was successful, the PIN Management Client sends a PIN Management PEGC Service Continuity response to indicate that the request processing was successfull and includes target PIN Gateway Client information. Otherwise, the PIN Management Client sends a PIN PEGC Service Continuity response indicating that the request processing failed and can include appropriate reasons.

Upon receiving the PIN Management PEGC Service Continuity response, if the response was received by the PIN Client, the PIN Client validates if the request was successful and can inform the application client of the target PIN Gateway Client provided in the response to establish connectivity. If the response was received by the PIN Gateway Client, the PIN Gateway Client notifies each PIN Client included in the PIN Management PEGC Service Continuity request about the target PIN Gateway Client change as in clause 8.9.2.1.5.

8.9.2.1.3	PIN Management PEGC Configuration procedure

Pre-conditions:

1.	The PIN Management Client has received a PIN Management PEGC Service Continuity request; and

2.	The PIN Management client has identified a target PIN Gateway Client.

Figure 8.9.2.1.3-1: PIN Management PEGC Configuration procedure

1.	The PIN Management Client sends a PIN Management PEGC Configuration request to the target PIN Gateway Client. The PIN Management PEGC Configuration request includes security credentials, PIN Client identifier(s), PIN Gateway identifier, and Service identifier.

2.	Upon receiving the request, the PIN Gateway Client validates the request and checks if the PIN Management Client is authorized to request PEGC Configuration. If the request is authorized and the request is valid, the PIN Gateway Client creates a new configuration for the PIN Client(s) and service that are indicated in the request.

3.	If the processing of the request was successful, the target PIN Gateway Client sends a PIN Management PEGC Configuration response to the PIN Management Client to indicate that the request was successful and can include connectivity information to be used by the PIN Client(s). Otherwise, the target PIN Gateway sends a PIN Management PEGC Configuration response indicating that the request failed and can include appropriate reasons.

Upon receiving the PIN Management PEGC Configuration response, the PIN Management Client uses the information provided in the response to inform the PIN Client(s) or the PIN Gateway Client from which the service continuity request originated as in clause 8.9.2.1.2.

8.9.2.1.4	PIN Service Continuity Update procedure

The PIN Service Continuity Update procedure allows the PEMC to inform the PIN Server about a service continuity request received at the PEMC for a PINE, and it allows the PIN Server to inform the related AS of the same if such functionality is supported by the AS.

The PIN Server may provide, in the response, policy information to the PEMC about service continuity for the concerned PINE.

Pre-conditions:

1.	PINE has an application session that goes through a PEGC; and

2.	PEGC needs to be replaced (e.g., PEGC will become unavailable or PEGC is unreachable).

Figure 8.9.2.1.4-1: PIN Configuration Update procedure

1.	The PEMC sends a PIN Configuration Service Continuity Update request to the PIN Server. The PIN Configuration Service Continuity Update request includes security credentials, service continuity information, PINE identifier(s), PIN Gateway identifier, and Service identifier.

2.	Upon receiving the request, the PIN Server validates the request and checks if the PEMC is authorized to request service continuity update. If the request is authorized and valid, if the PIN Server has not authorized the PINE for service continuity, the PIN Server validates if the PINE is authorized and determines policy information about service continuity for the PINE. If the PIN Server has already authorized the PINE for service continuity, the PIN Server updates the configuration for the PINE and service indicated in the request.

3.	If the PIN Server needs to update the AS with service continuity information, the PIN Server sends a PIN Service Continuity Update request to the AS. The PIN Service Continuity Update request includes security credentials, service continuity information, PINE identifier(s), PEGC identifier, service identifier and may include an application session identifier and terminating endpoints on the PINE and PEGC.

Editor's note: Whether subscription/notification mechanisms are used for informing the AS over PIN-9 reference point is FFS.

4. 	Upon receiving the request, the AS validates the request and checks if the PIN Server is authorized to request service continuity update. If the request is authorized and valid, if the AS updates the service continuity configuration related to the PINE and service indicated in the request.

5.	If the processing of the request was successful, the AS sends a PIN Service Continuity Update response to indicate that the request processing was successful. Otherwise, the AC sends a PIN Service Continuity Update response indicating that the request processing failed and can include appropriate reasons.

6.	If the processing of the PIN Configuration Service Continuity Update request was successful, the PIN Server sends a PIN Configuration Service Continuity Update response to indicate that the request processing was successful and may include policy information about service continuity for the PINE. Otherwise, the PIN Server sends a PIN Configuration Service Continuity Update response indicating that the request processing failed and can include appropriate reasons.

Upon receiving the PIN Configuration Service Continuity Update response, the PEMC uses the information provided in the response to inform the PINE(s) or the PEGC from which the service continuity request originated as in clause 8.9.2.1.2.

8.9.2.1.5	PIN Management PEGC Discovery procedure

PEMC performs PIN Management PEGC discovery procedure to discover which PEGC are available at the PINE when the PEMC does not have that information; for example, the PEMC may ignore if a PEGC is available at the PINE or PEGC information may not have been provided to the PEMC in the service provisioning request.

Pre-conditions:

1.	The PEMC has been triggered for service continuity;

2.	The target PEGC is not known by the PEMC.

Figure 8.9.2.1.5-1: PIN Management PEGC Discovery procedure

1.	The PEMC sends a PIN Management PEGC Discovery request to the PINE involved in the service continuity procedure. The PIN Management PEGC Discovery request includes security credentials.

2.	Upon receiving the request, the PINE validates the request and checks if the request is authorized. If the request is authorized and valid, the PINE creates a list of PEGC that are available at the PINE.

Editor’s Note: It is FFS how the PIN Client determines available PEGC and if it is in scope of TS 23.542.

3.	If the procesing of the request was successful, the PINE sends a PIN Management PEGC Discovery response to indicate that the request processing is successfull and includes the list of PEGC information available at the PINE. Otherwise, the PINE sends a PIN Management PEGC Discovery response indicating that the request processing failed and can include appropriate reasons.

Upon receiving the response, the PEMC selects the target PEGC based on the list of available PEGC received and the PIN policy.

8.9.2.2	Service continuity in changing access to 5GS

This procedure solves the situation, that PINE A has application layer communication with PINE B via PEGC. But when the communication via PEGC is not viable, for example, the PINB moves out of the coverage of PEGC 1, only newly establish the 5GS communication to connect PINE B is the potential way.

Figure 8.9.2.2-1 illustrates the service continuity to change application layer communication to communication via PEGC + 5GS based on request/response model.

Pre-conditions:

1.	The PEMC in a PIN has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The PIN has already been created and a PIN ID is distributed by PIN server;

3.	The PINE A has the subscription that can communicate with PINE B via 5GS;

Figure 8.9.2.2-1: Change application layer communication to communication via 5GS

1.	The PINE A has application layer communication with PINE B via PEGC 1.

2.	The PEGC 1 receives the related parameter (PIN ID, MAC address/IP address, Traffic descriptors, Packet filters, requested QoS) as indicated in section 8.6.2.2 from PINE 1 or PEMC.

3.	According to the Packet filters, the PEGC may initiate PDU Session Modification as indicated in section 4.3.3.2 of TS 23.502[x] with the Packet filters and requested QoS towards 5G system in order to make 5GC configure the N4 rules for UPF(s), referred to section 8.6.2.2.

4.	The PINE A communicates with PINE 2 via 5GS.

8.9.3	Information flows

Editor's note: Information flows definition for Service Continuity is FFS

8.9.4	APIs

Editor's note: API definition for Service Continuity is FFS

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-01 | SA6#52bis-e |  |  |  |  | TS skeleton (version 0.0.0) approved in S6-230388. | 0.0.0 |
| 2023-01 | SA6#52bis-e |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-230081, S6-230082, S6-230083, S6-230118, S6-230121, S6-230124, S6-230126, S6-230184, S6-230185, S6-230300, S6-230369, S6-230370, S6-230381, S6-230384, S6-230386, S6-230387, S6-230388, S6-230487, S6-230488, S6-230489, S6-230490, S6-230491, S6-230492, | 0.1.0 |
| 2023-02 | SA6#53 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-230554, S6-230577, S6-230648, S6-230896, S6-230900, S6-230901, S6-230902, S6-230904, S6-230905, S6-230906, S6-230940, S6-230941, S6-230942, S6-230948, S6-230949, S6-230950, S6-230952, S6-231017, S6-231057, S6-231083 | 0.2.0 |