| 3GPP TS 24.542 V0.2.0 (2023-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Notification management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Abbreviations	7

4	General description	7

5	Functional Entities	7

5.1	SEAL notification management client (SNM-C)	7

5.2	SEAL notification management server (SNM-S)	7

6	Notification management procedures	8

6.1	General	8

6.2.	On-network procedures	8

6.2.1	General	8

6.2.1.1	Authenticated identity in HTTP request	8

6.2.1.2	Boot up procedure	8

6.2.2	Notification channel creation procedure	8

6.2.2.1	SNM client procedures	8

6.2.2.2	SNM server procedures	8

6.3	Off-network procedures	9

Annex A (normative): Parameters for different operations	9

A.1	Creating notification channel	9

A.1.1	General	9

A.1.2	Client side parameters	10

A.1.3	Server side parameters	10

Annex B (informative): Change history	10

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol aspects for the notification management capability of SEAL to support vertical applications (e.g. V2X) over the 3GPP system.

The present document is applicable to the User Equipment (UE) supporting the notification management client functionality as described in 3GPP TS 23.434 [2], to the application server supporting the notification management server functionality as described in 3GPP TS 23.434 [2] and to the application server supporting the vertical application server (VAL server) functionality as defined in specific vertical application service (VAL service) specification.

NOTE:	The specification of the VAL server for a specific VAL service is out of scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[4]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[5]	IETF RFC 7159: "The JavaScript Object Notation (JSON) Data Interchange Format".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

SEAL notification management client: An entity that provides the client side functionalities corresponding to the SEAL notification management service.

SEAL notification management server: An entity that provides the server side functionalities corresponding to the SEAL notification management service.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.434 [2] apply:

SEAL client

SEAL server

SEAL service

VAL notification

VAL server

VAL service

VAL user

Vertical

Vertical application

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SEAL	Service Enabler Architecture Layer for verticals

SNM-C	SEAL Notification Management Client

SNM-S	SEAL Notification Management Server

VAL	Vertical Application Layer

# 4	General description

Notification management is a SEAL service that provides the notification management related capabilities to one or more vertical applications. The present document enables a SEAL notification management Client (SNM-C) and VAL server to manage notifications through the SEAL notification management Server (SNM-S).

# 5	Functional Entities

## 5.1	SEAL notification management client (SNM-C)

The SNM-C is a functional entity that acts as the application client for notification management.

To be compliant with the HTTP procedures in the present document, the SNM-C:

a)	shall support the procedures of creating, opening and deleting notification channel as per clause 6.2; and

b)	shall support the procedure to receive notification message from the SNM-S as per clause 6.2.

## 5.2	SEAL notification management server (SNM-S)

The SNM-S functional entity provides notification management support within the vertical application layer.

To be compliant with the HTTP procedures in the present document, the SNM-S:

a)	shall support the procedures of creating, opening and deleting notification channel as per clause 6.2;

b)	shall support the procedure to receive notification message from the VAL server as per clause 6.2; and

c)	shall support the procedure to send notification message to the SNM-C as per clause 6.2.

# 6	Notification management procedures

## 6.1	General

## 6.2.	On-network procedures

### 6.2.1	General

#### 6.2.1.1	Authenticated identity in HTTP request

Upon receiving an HTTP request, the SNM-S shall authenticate the identity of the sender of the HTTP request as specified in 3GPP TS 24.547 [3], and if authentication is successful, the SNM-S shall use the identity of the sender of the HTTP request as an authenticated identity.

#### 6.2.1.2	Boot up procedure

Upon device boot up, the NM-C in the UE shall create the notification channel with the notification management server as specified in clause 6.2.2.1.

### 6.2.2	Notification channel creation procedure

#### 6.2.2.1	SNM client procedures

Upon receiving a request from VAL service to receive notifications via the notification channel; the SNM-C may create a notification channel by sending an HTTP POST request to the SNM-S. In the HTTP POST request the SNM-C:

- shall set the Request-URI to the URI of the SNM-S;

- shall include the Host header with public user identity of SNM-S;

- shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [4];

- shall include a Content-Type header field set to "application/vnd.3gpp.seal-create-notification-channel-request";

- shall generate the create notification channel request message as specified in clause A.1.2:

1)	shall set the requestor identity to the notification management client identity;

2)	shall set the channel type to PULL or PUSH based on the VAL Application requesting the use of notification channel;

3)	may set the PUSH channel details; and

4)	shall set the validity duration of the notification channel; and

f)	include the parameters specified in clause A.1.2 serialized into a JavaScript Object Notation (JSON) structure as specified in IETF RFC 7159 [5].

Editor's note: Changes related to HTTP features like HTTP long polling and HTTP multiplexing shall be defined as part of FFS.

#### 6.2.2.2	SNM server procedures

Upon reception of an HTTP POST request from SNM-C where the Request-URI of the HTTP POST request contains the URI of the SNM-S, the SNM-S:

a)	shall determine the requestor identity of the received HTTP POST request as specified in clause 6.2.1.1, and:

1)	if the identity of the sender of the received HTTP POST request is not authorized user, shall respond with an HTTP 403 (Forbidden) response to the HTTP POST request and skip rest of the steps;

b)	shall process the create notification channel request and if the channel type is:

1)	PUSH, the PUSH channel details message shall be processed to get the subscription identity(s) and its associated VAL Server information(s) for which the NM-C prefers to receive notification via the notification channel. When the Val server shares the notification message to NM-S, NM-S shall forward the same to NM-C;

2)	PULL, the NM-S shall wait for the NM-C to pull the notification messages;

Editor's note: Procedures for NM-C initiated PULL is FFS.

Editor's note: In case of multiple notification accumulated at the NM-S from same VAL Server towards the NM-C. How the NM-S decides to share all the notifications of latest notifications is FFS.

- shall process the validity duration share by the NM-C.

NOTE:	The NM-S shall store the of the authorized user and information shared as part of create notification channel request for future references

Upon successful creation of notification channel; the SGM-S:

- shall create a notification channel response message with below attributes as specified in clause A.1.3;

1)	shall generate unique channel identifier;

2)	shall generate the callback URL, which shall be used by VAL clients in UE for sharing it to VAL Server as part of their respective services;

3)	may generate the validity duration of the notification channel;

4)	may generate a notification URL that shall be used by NM-C to pull the notifications from NM-S in case of PULL channel type;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-create-notification-channel-response"; and

c)	shall send an HTTP 200 (OK) response including message generated above.

## 6.3	Off-network procedures

Editor’s note: This clause will describe the off-network procedures based on 3GPP TS 23.434 [2].

Annex A (normative): Parameters for different operations

# A.1	Creating notification channel

## A.1.1	General

The information in this annex provides a normative description of the parameters which will be sent by SNM-C while creating notification channel request and the parameters which will be sent by SNM-S as a response to request for creating notification channel.

## A.1.2	Client side parameters

The SNM-C shall convey the following parameters while sending request for create notification channel request.

Table X.1.2-1: Client side parameters for create notification channel request

| Parameter | Description |
| --- | --- |
| Requestor identity | REQUIRED. Represents the identity of the notification management client |
| Channel type | REQUIRED. Represents PULL or PUSH method to be used for delivering the notification messages - 0x01: PUSH TYPE - 0x02: PULL TYPE |
| PUSH channel details | OPTIONAL. Represents details of the type of PUSH delivery and its associated data |
| Expiry Time | REQUIRED. Represents the duration the notification channel shall be active (i.e. channel lifetime) as requested by the notification management client |

## A.1.3	Server side parameters

The SNM-S shall convey the following parameters while sending response to the create notification channel request.

Table X.1.3-1: Server side parameters for create notification channel response

| Parameter | Description |
| --- | --- |
| Notification URL | OPTIONAL. Represents the URL that shall be used by NM-C to pull notification if the channel type is PULL |
| Callback URL | REQUIRED. Represents the URL, which shall be notified to VAL client by NM-C. Further this URL shall be shared by VAL client to VAL server while subscribing for a VAL services |
| Channel Identifier | REQUIRED. Represents the identifier of the newly created notification channel |
| Expiry Time | OPTIONAL. Represents the duration the notification channel shall be active (i.e. channel lifetime) as requested by the notification management client |

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-02 | CT1#140 | C1-230655 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2023-03 | CT1#140 |  |  |  |  | Implementing the following p-CR agreed in CT1: C1-230538, C1-230544, C1-230572, C1-230873 | 0.1.0 |
| 2023-04 | CT1#141-e |  |  |  |  | Implementing the following p-CR agreed in CT1: C1-232796, C1-232797, C1-232798 | 0.2.0 |