| 3GPP TS 28.535 V0.1.0 (2019-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management Services for Communication Service Assurance; Requirements (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4 	Concepts and background	7

4.1	Lifecycle of a communication service	7

4.2	Management control loops	7

5 	Business level use cases and requirements	8

5.1	Use cases	8

5.1.1	Communication service assurance	8

5.1.2	Requirements	8

6 	Specification level use cases and requirements	8

6.1	Use cases	8

6.1.1	<Name of use case>	8

6.2	Requirements	8

Annex <X> (informative): Change history	9

.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document describes closed loop assurance solution enabling a service provider or an operator to continuously deliver the requested level of communication service quality to the customer and is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.535: Management Services for Communication Service Assurance; Requirements

TS 28.536: Management Services for Communication Service Assurance; Stage 2 and stage 3

The solution described builds upon the management services specifications as identified below:

TS 28.530: Management and orchestration; Concepts, use cases and requirements

TS 28.533: Management and orchestration; Architecture framework.

TS 28.532: Management and orchestration; Management services.

TS 28.540: Management and orchestration; 5G Network Resource Model (NRM); Stage 1

TS 28.541: Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

TS 28.531: Management and orchestration; Provisioning.

TS 28.545: Management and orchestration; Fault Supervision (FS)

TS 28.550: Management and orchestration; Performance assurance

TS 28.552: Management and orchestration; 5G performance measurements

TS 28.554: Management and orchestration; 5G End to end Key Performance Indicators (KPI).

# 1	Scope

The present document describes, concepts and background, and specifies use cases and requirements for closed loop communication service assurance solution that adjusts and optimizes the services provided by NG-RAN and 5GC.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for next generation new services and markets"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CS	Communication Service

CSI	Communication Service Instance

SLA	Service Level agreement

SLS	Service Level Specification

# 4 	Concepts and background

Editor’s Note: the following concepts are amongst others to be described in this clause, lifecycle of a communication service, service level specification, management control loop, interface/management services,

## 4.1	Lifecycle of a communication service

Communication service assurance applies to different phases in the life of a communication service instance (CSI), a CSI goes through the following lifecycle phases; preparation, commissioning, operation and decommissioning.

- Preparation phase:

Providing a CSI starts with preparation, which includes communication service design, pre-planning, feasibility check, i.e., checking the attainable communication service quality from both resource and service aspects, negotiation of the communication service attributes, preparing communication service and network requirements derived from SLA.

- CSI commissioning phase:

Once the CSI is prepared, it can be established by converting the communication service requirement for the CSI to network requirements (interaction and use of NF resources including RAN, CN) and creation of the CSI. When the CSI is created, it is deployed on the network resources and ready to be used by the communication service consumers (subscribers, UEs). Before allowing the maximum agreed number of communication service consumers to use the CSI a communication service assurance control loop is deployed to allow the network to converge to a state where the communication service assurance is stable and within the boundaries of the SLS. The assurance control loop learns the communication service behaviour during an initial deployment or trail phase.

- CSI operation phase:

After the commissioning phase, the CSI is activated for use of all communication service consumers (subscribers, UEs) that are allowed to use the communication service. The initial deployment or trail phase for the training of the communication service assurance algorithms has entered the operation phase. An activated CSI allows run-time operation of the communication service, e.g., quality of experience assurance, quality of service assurance. The optimization of CSI utilization may continue during the operation phase of the CSI.

- CSI decommissioning phase:

When the CSI is no longer needed, after being de-activated, the lifecycle of the CSI ends with CSI termination.

Figure 4.1.1 Lifecycle of a communication service instance

## 4.2	Management control loops

Editor’s Note: this clause describes the concept of management control loops for communication service assurance and describes the relations with other management and managed entities.

# 5 	Business level use cases and requirements

## 5.1	Use cases

### 5.1.1	Communication service assurance

The CSP wants to meet the CSC expectations on automation as well as internal goals on CAPEX and OPEX efficiency.

The CSP has access to capabilities, procedures and tools that can address both CAPEX and OPEX in the provisioning and management of communication services to their customers (CSC). The CSC expects the CSP to offer a variety of communication services including business critical communication services that allow the CSC (e.g. Enterprise) to run their applications in a predictable manner [2]. Hence automation of the on-boarding of the CSC application, which will use communication services provided by the CSP, on a 5GS, is a requirement to meet the following needs:

- reduce the complexity for a CSC application to be on-boarded on a 5GS.

- improve the network performance over time, based on predicting communication service behaviour

- reduce the cost ownership through automation

During the operation of the communication service the CSP provides assurance of service quality expectation and CSP meets the CSC expectations on automation as well as internal goals on CAPEX and OPEX efficiency.

### 5.1.2	Requirements

REQ-CSA_CSA-FUN-01 The CSP shall be able to provide a statement of CS service requirements to a 5GS and receive capability information about these from the 5GS.REQ-CSA_CSA-FUN-03 The 5GS shall have the capabilities to monitor, and report to CSP the fulfilment of committed CS requirements and actions taken to adjust for deviations.

REQ-CSA_CSA-FUN-03 The 5GS shall have the capabilities to monitor and report to CSP actions taken to adjust deviations on committed CS requirements.

REQ-CSA_CSA-FUN-04 The 5GS shall have the capability to provide in-operation assurance of service quality expectation.

REQ-CSA_CSA-FUN-04 The 5GS shall have the capability to ensure the service quality requirements during the service operations.

Editor’s Note: The specification level requirements on OAM are for FFS.

# 6 	Specification level use cases and requirements

## 6.1	Use cases

Editor’s Note: use cases are documented as flat list clauses with one use case per clause until a structure appears, then the use cases may be grouped. Each use case is documented using the use case template

### 6.1.1	<Name of use case>

## 6.2	Requirements

Editor’s Note: requirements are documented as a flat list until a structure appears then the requirements may be grouped.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | SA5#126 |  |  |  |  | Skeleton | 0.0.0 |
| 2019-10 | SA5#127 | S5-196611 |  |  |  | pCR 28.535 Add scope | 0.1.0 |
| 2019-10 | SA5#127 | S5-196612 |  |  |  | pCR 28.535 Add introduction | 0.1.0 |
| 2019-10 | SA5#127 | S5-196483 |  |  |  | pCR 28.535 Introduce table of content | 0.1.0 |
| 2019-10 | SA5#127 | S5-196615 |  |  |  | pCR 28.535 Add lifecycle of a communication service | 0.1.0 |
| 2019-10 | SA5#127 | S5-196478 |  |  |  | pCR 28.535 Add management control loops | 0.1.0 |