| 3GPP TS 38.133 V18.3.0 (2023-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	82

1	Scope	84

2	References	84

3	Definitions, symbols and abbreviations	85

3.1	Definitions	85

3.2	Symbols	86

3.3	Abbreviations	87

3.4	Test tolerances	90

3.5	Frequency bands grouping	90

3.5.1	Introduction	90

3.5.2	NR operating bands in FR1	90

3.5.2A	NR operating bands for satellite access in FR1	91

3.5.3	NR operating bands in FR2	92

3.6	Applicability of requirements in this specification version	92

3.6.1	RRC connected state requirements in DRX	93

3.6.2	Number of serving carriers	93

3.6.2.1	Number of serving carriers for SA	93

3.6.2.2	Number of serving carriers for EN-DC	93

3.6.2.3	Number of serving carriers for NE-DC	93

3.6.2.4	Number of serving carriers for NR-DC	94

3.6.3	Applicability for intra-band FR2	94

3.6.4	Applicability for FR2 UE power classes	94

3.6.5	Applicability for SDL bands	94

3.6.6	Applicability of requirements for NGEN-DC operation	94

3.6.7	Applicability of QCL	94

3.6.9	Applicability of requirements for scheduling availability	95

3.6.10	Applicability of requirements for measurement restrictions	95

3.6.11	Applicability of requirements for Redcap UEs	95

3.6.11.1	RRC connected state requirements in DRX	95

3.6.11.2	Applicability for FR2 Redcap UE power classes	95

3.6.11.3	Applicability of QCL	95

3.6.12	Applicability of requirements for Satellite Access	96

3.6.13	Applicability of requirements for FR2	96

3.6.14	Applicability of requirements for FR2 Power Class 6	96

3.6.15	Applicability of requirements for per-FR gap	96

3.6.16	Applicability of requirements for ATG	96

4	SA: RRC_IDLE state mobility	96

4.1	Cell Selection	96

4.2	Cell Re-selection	97

4.2.1	Introduction	97

4.2.2	Requirements	97

4.2.2.1	UE measurement capability	97

4.2.2.2	Measurement and evaluation of serving cell	97

4.2.2.3	Measurements of intra-frequency NR cells	100

4.2.2.4	Measurements of inter-frequency NR cells	103

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	108

4.2.2.6	Maximum interruption in paging reception	111

4.2.2.7	General requirements	112

4.2.2.8	Minimum requirement at transitions	112

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	112

4.2.2.9.1	Introduction	112

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	112

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	115

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	117

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	118

4.2.2.10.1	Introduction	118

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	118

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	121

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	124

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	124

4.2.2.11.1	Introduction	124

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	125

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	126

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	128

4.2A	Cell Re-selection when subject to CCA	129

4.2A.1	Introduction	129

4.2A.2	Requirements	129

4.2A.2.1	UE measurement capability	129

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	129

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	130

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	132

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	133

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	134

4.2A.2.7	General requirements	134

4.2B	Cell Re-selection for RedCap	134

4.2B.1	Introduction	134

4.2B.2	Requirements	134

4.2B.2.1	UE measurement capability for RedCap	134

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	134

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	135

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	135

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	137

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	139

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	142

4.2B.2.6	Maximum interruption in paging reception for RedCap	144

4.2B.2.7	General requirements for RedCap	144

4.2B.2.8	Minimum requirement at transitions	145

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	145

4.2B.2.9.1	Introduction	145

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	146

4.2B.2.9.3	Measurements for a UE fulfilling stationary and not-at-cell edge criteria	149

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criteria	149

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	150

4.2B.2.9.5	Measurements for a UE fulfilling low mobility criterion and stationary and not-at-cell edge criteria	150

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary and not-at-cell edge criteria	150

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary and notatcell edge criteria	150

4.2B.2.9.8	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary criterion	151

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	151

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	154

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	156

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	156

4.2B.2.10.1	Introduction	156

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	157

4.2B.2.10.3	Measurements for a UE fulfilling stationary not at cell edge criterion	159

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	160

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	160

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	161

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	161

4.2B.2.10.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	161

4.2B.2.10.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	161

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	162

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	164

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	167

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	167

4.2B.2.11.1	Introduction	167

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	168

4.2B.2.11.3	Measurements for a UE fulfilling stationary not at cell edge criterion	169

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	170

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	170

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	171

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	171

4.2B.2.11.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	171

4.2B.2.11.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	171

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	172

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	173

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	174

4.2C	Cell Re-selection for NR UE for Satellite Access	175

4.2C.1	Introduction	175

4.2C.2	Requirements	175

4.2C.2.1	UE measurement capability	175

4.2C.2.2	Measurement and evaluation of serving cell	175

4.2C.2.3	Measurements of intra-frequency NR cells	176

4.2C.2.4	Measurements of inter-frequency NR cells	178

4.2C.2.5	Maximum interruption in paging reception	182

4.2C.2.6	Minimum requirement at transitions	182

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	182

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	182

4.2C.2.9	General requirements	182

4.2D	Cell Re-selection for ATG	183

4.2D.1	Introduction	183

4.2D.2	Requirements	183

4.2D.2.1	UE measurement capability	183

4.2D.2.2	Measurement and evaluation of serving cell	183

4.2D.2.3	Measurements of intra-frequency NR cells	184

4.2D.2.4	Measurements of inter-frequency NR cells	185

4.2D.2.5	Maximum interruption in paging reception	187

4.2D.2.6	General requirements	187

4.3	Minimization of Drive Tests (MDT)	187

4.3.1	Introduction	187

4.3.2	Measurement Requirements	188

4.3.3	Requirements for Relative Time Stamp Accuracy	188

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	188

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	189

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	189

4.3C.1	Introduction	189

4.3C.2	Measurement Requirements	189

4.3C.3	Requirements for Relative Time Stamp Accuracy	190

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	190

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	190

4.4	Idle Mode CA/DC Measurements	191

4.4.1	Introduction	191

4.4.2	Measurement Requirements	191

4.4.2.1	Detected cell requirement during state transition and Idle mode	191

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	191

4.4.2.3	Measurements on serving cell	193

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	193

5	SA: RRC_INACTIVE state mobility	194

5.1	Cell Re-selection	194

5.1.1	Introduction	194

5.1.2	Requirements	194

5.1.2.1	UE measurement capability	194

5.1.2.2	Measurement and evaluation of serving cell	194

5.1.2.3	Measurements of intra-frequency NR cells	194

5.1.2.4	Measurements of inter-frequency NR cells	194

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	194

5.1.2.6	Maximum interruption in paging reception	194

5.1.2.7	General requirements	195

5.1A	Cell Re-selection with CCA	195

5.1A.1	Introduction	195

5.1A.2	Requirements	195

5.1A.2.1	UE measurement capability	195

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	195

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	195

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	195

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	195

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	195

5.1A.2.7	General requirements	196

5.1B	Cell Re-selection for RedCap	196

5.1B.1	Introduction	196

5.1B.2	Requirements	196

5.1B.2.1	UE measurement capability	196

5.1B.2.2	Measurement and evaluation of serving cell	196

5.1B.2.3	Measurements of intra-frequency NR cells	197

5.1B.2.4	Measurements of inter-frequency NR cells	198

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	199

5.1B.2.6	Maximum interruption in paging reception	199

5.1B.2.7	General requirements	199

5.1B.2.8	Minimum requirement at transitions	199

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	199

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	200

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	200

5.1C	Cell Re-selection	200

5.1C.1	Introduction	200

5.1C.2	Requirements	200

5.1C.2.1	UE measurement capability	200

5.1C.2.2	Measurement and evaluation of serving cell	200

5.1C.2.3	Measurements of intra-frequency NR cells	200

5.1C.2.4	Measurements of inter-frequency NR cells	200

5.1C.2.5	Maximum interruption in paging reception	200

5.1C.2.6	General requirements	200

5.1D	Cell Re-selection for ATG	201

5.1D.1	Introduction	201

5.1D.2	Requirements	201

5.1D.2.1	UE measurement capability	201

5.1D.2.2	Measurement and evaluation of serving cell	201

5.1D.2.3	Measurements of intra-frequency NR cells	201

5.1D.2.4	Measurements of inter-frequency NR cells	201

5.1D.2.5	Maximum interruption in paging reception	201

5.1D.2.6	General requirements	201

5.2	Void	201

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	201

5.2B.1	Introduction	201

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	202

5.2B.2.1	Void5.2B.3 TA validation requirements for RedCap	202

5.2B.2.2	Void	203

5.2B.4	Scheduling restriction	203

5.2B.5	Applicability conditions for CG-SDT for RedCap	203

5.3	Minimization of Drive Tests (MDT)	203

5.3.1	Introduction	203

5.3.2	Measurement Requirements	204

5.3.3	Requirements for Relative Time Stamp Accuracy	204

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	204

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	204

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	204

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	204

5.3C.1	Introduction	204

5.3C.2	Measurement Requirements	205

5.3C.3	Requirements for Relative Time Stamp Accuracy	205

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	205

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	205

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	205

5.4	Idle Mode CA/DC Measurements	205

5.4.1	Introduction	205

5.4.2	Measurement Requirements	205

5.4.2.1	Detected cell requirement during state transition and Idle mode	205

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	206

5.4.2.3	Measurements on serving cell	206

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	206

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	206

5.5.1	Introduction	206

5.5.2	Requirements on UE synchronization for small data transmissions	206

5.5.3	TA validation requirements	206

5.5.4	Scheduling restriction	207

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	207

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	207

5.5.4.3	Scheduling availability of UE performing measurements on FR2	208

5.5.5	Applicability conditions for SDT	208

5.5D	Configured Grant based Small Data Transmissions (CG-SDT) for ATG	208

5.6	NR measurements for positioning	208

5.6.1	Introduction	208

5.6.2	RSTD measurements	209

5.6.2.1	Introduction	209

5.6.2.2	Requirements Applicability	209

5.6.2.3	Measurement Capability	209

5.6.2.5	Measurements Period Requirements	210

5.6.3	PRS-RSRP measurements	213

5.6.3.1	Introduction	213

5.6.3.2	Requirements applicability	213

5.6.3.3	Measurement Capability	213

5.6.3.4	Measurement Reporting Requirements	213

5.6.3.5	Measurement Period Requirements	213

5.6.4	UE Rx-Tx time difference measurements	216

5.6.4.1	Introduction	216

5.6.4.2	Requirements Applicability	216

5.6.4.3	Measurement Capability	216

5.6.4.4	Measurement Reporting Requirements	216

5.6.4.5	Measurement Period Requirements	217

5.6.5	PRS-RSRPP measurements	219

5.6.5.1	Introduction	219

5.6.5.2	Requirements Applicability	219

5.6.5.3	Measurement Capability	219

5.6.5.4	Measurement Reporting Requirements	220

5.6.5.5	Measurement Period Requirements	220

5.7	Random access based Small Data Transmissions (RA-SDT)	220

5.7.1	Introduction	220

5.7.2	Requirements for small data transmissions based on 2-step RA	220

5.7.3	Requirements for small data transmissions based on 4-step RA	220

5.7.4	Applicability conditions for SDT	220

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	221

5.7B.1	Introduction	221

5.7B.2	Requirements for small data transmissions based on 2-step RA	221

5.7B.3	Requirements for small data transmissions based on 4-step RA	221

5.7B.4	Applicability conditions for RA-SDT for RedCap	221

5.7D	Random access based Small Data Transmissions (RA-SDT) for ATG	221

6	RRC_CONNECTED state mobility	222

6.1	Handover	222

6.1.1	NR Handover	222

6.1.1.1	Introduction	222

6.1.1.2	NR FR1 - NR FR1 Handover	222

6.1.1.2.1	Handover delay	222

6.1.1.2.2	Interruption time	222

6.1.1.3	NR FR2- NR FR1 Handover	223

6.1.1.3.1	Handover delay	223

6.1.1.3.2	Interruption time	223

6.1.1.4	NR FR2- NR FR2 Handover	224

6.1.1.4.1	Handover delay	224

6.1.1.4.2	Interruption time	224

6.1.1.5	NR FR1- NR FR2 Handover	225

6.1.1.5.1	Handover delay	225

6.1.1.5.2	Interruption time	225

6.1.2	NR Handover to other RATs	226

6.1.2.1	NR – E-UTRAN Handover	226

6.1.2.1.1	Introduction	226

6.1.2.1.2	Handover delay	226

6.1.2.1.3	Interruption time	226

6.1.2.2	NR – UTRAN Handover	226

6.1.2.2.1	Introduction	226

6.1.2.2.2	Handover delay	227

6.1.2.2.3	Interruption time	227

6.1.3	NR DAPS Handover	227

6.1.3.1	Introduction	227

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	228

6.1.3.2.1	DAPS handover delay	228

6.1.3.2.2	Interruption time	229

6.1.3.3	NR FR2- NR FR1 DAPS Handover	230

6.1.3.3.1	DAPS handover delay	231

6.1.3.3.2	Interruption time	231

6.1.3.4	NR FR1- NR FR2 DAPS Handover	232

6.1.3.4.1	DAPS handover delay	232

6.1.3.4.2	Interruption time	232

6.1.4	NR Conditional Handover	233

6.1.4.1	Introduction	233

6.1.4.2	NR FR1 – NR FR1 conditional handover	233

6.1.4.3	NR FR2 – NR FR1 conditional handover	234

6.1.4.4	NR FR2 – NR FR2 conditional handover	234

6.1.4.4.1	Handover delay	234

6.1.4.4.2	Measurement time	235

6.1.4.4.3	Preparation time	235

6.1.4.4.4	Interruption time	235

6.1.4.5	NR FR1 – NR FR2 conditional handover	236

6.1.5	NR Handover with PSCell	236

6.1.5.1	Introduction	236

6.1.5.2	Handover with PSCell from NR SA to EN-DC	236

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	236

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	237

6.1.5.3	HO with PSCell from NE-DC to NE-DC	237

6.1.5.3.1	Handover delay	237

6.1.5.3.2	HO with PSCell - PCell Interruption time	238

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	238

6.1.5.4	HO with PSCell from NR-DC to NR-DC	238

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	239

6.1.5.5.1	Introduction	239

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	240

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	240

6.1A	Void	241

6.1A.1	Void	241

6.1A.1.1	Void	241

6.1A.1.2	Void	241

6.1A.1.2.1	Void	241

6.1A.1.2.2	Void	241

6.1B	Handover to target cell using CCA	241

6.1B.1	NR Handover	241

6.1B.1.1	Introduction	241

6.1B.1.2	NR FR1 - NR FR1 Handover	242

6.1B.1.2.1	Handover delay	242

6.1B.1.2.2	Interruption time	242

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	243

6.1B.1.3.1	Handover delay	243

6.1B.1.3.2	Interruption time	243

6.1B.1.4	NR FR1- NR FR2-2 Handover	244

6.1B.1.4.1	Handover delay	244

6.1B.1.4.2	Interruption time	244

6.1C	Handover for SAN	245

6.1C.1	NR SAN Handover	245

6.1C.1.1	Introduction	245

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	245

6.1C.1.2.1	Handover delay	245

6.1C.1.2.2	Interruption time	245

6.1C.2	NR SAN Conditional Handover	246

6.1C.2.1	Introduction	246

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	246

6.1C.2.2.1	Handover delay	246

6.1C.2.2.2	Measurement time	247

6.1C.2.2.3	Preparation time	248

6.1C.2.2.4	Interruption time	248

6.1D	Handover for RedCap	248

6.1D.1	NR Handover	248

6.1D.1.1	Introduction	248

6.1D.1.2	NR FR1 - NR FR1 Handover	249

6.1D.1.2.1	Handover delay	249

6.1D.1.2.2	Interruption time	249

6.1D.1.3	NR FR2- NR FR2 Handover	250

6.1D.1.3.1	Handover delay	251

6.1D.1.3.2	Interruption time	251

6.1D.2	NR Handover to other RATs	252

6.1D.2.1	NR – E-UTRAN Handover	252

6.1E	Handover for ATG	252

6.1E.1	NR Handover	252

6.1E.1.1	Introduction	252

6.1E.1.2	NR FR1 - NR FR1 Handover	252

6.1E.1.2.1	Handover delay	252

6.1E.1.2.2	Interruption time	253

6.1E.2	NR Conditional Handover	253

6.1E.2.1	Introduction	253

6.1E.2.2	NR FR1 – NR FR1 conditional handover	253

6.1E.2.2.1	Handover delay	253

6.1E.2.2.2	Measurement time	254

6.1E.2.2.3	Preparation time	255

6.1E.2.2.4	Interruption time	255

6.2	RRC Connection Mobility Control	255

6.2.1	SA: RRC Re-establishment	255

6.2.1.1	Introduction	255

6.2.1.2	Requirements	256

6.2.1.2.1	UE Re-establishment delay requirement	256

6.2.1A	RRC Re-establishment with CCA	257

6.2.1A.1	Introduction	257

6.2.1A.2	Requirements	258

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	258

6.2.1B	SA: RRC Re-establishment for RedCap	260

6.2.1B.1	Introduction	260

6.2.1B.2	Requirements	260

6.2.2	Random access	260

6.2.2.1	Introduction	260

6.2.2.2	Requirements for 4-step RA type	261

6.2.2.2.1	Contention based random access	261

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	261

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	261

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	261

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	261

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	261

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	262

6.2.2.2.2	Non-Contention based random access	262

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	262

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	262

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	263

6.2.2.2.3	UE behaviour when configured with supplementary UL	263

6.2.2.3	Requirements for 2-step RA type	263

6.2.2.3.1	Contention based random access	263

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	263

6.2.2.3.1.2	Correct behaviour when receiving MsgB	264

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	264

6.2.2.3.2	Non-Contention based random access	264

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	264

6.2.2.3.2.2	Correct behaviour when receiving MsgB	264

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	264

6.2.2.3.3	UE behaviour when configured with supplementary UL	265

6.2.2A	Random access when CCA is used on target frequency	265

6.2.2A.1	Introduction	265

6.2.2A.2	Requirements for 4-step RA type	265

6.2.2A.2.1	Contention based random access	265

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	265

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	266

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	266

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	266

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	266

6.2.2A.2.2	Non-Contention based random access	266

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	266

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	267

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	267

6.2.2A.3	Requirements for 2-step RA type	267

6.2.2A.3.1	Contention based random access	268

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	268

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	268

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	269

6.2.2A.3.2	Non-Contention based random access	269

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	269

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	269

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	269

6.2.2B	Random access for RedCap	270

6.2.2B.1	Introduction	270

6.2.2B.2	Requirements	270

6.2.3	SA: RRC Connection Release with Redirection	270

6.2.3.1	Introduction	270

6.2.3.2	Requirements	271

6.2.3.2.1	RRC connection release with redirection to NR	271

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	271

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	272

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	273

6.2.3A.1	Introduction	273

6.2.3A.2	Requirements	273

6.2.3A.2.1	RRC connection release with redirection to NR	273

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	274

6.2C	RRC Connection Mobility Control for Satellite Access	274

6.2C.1	SA: RRC Re-establishment for Satellite Access	274

6.2C.1.1	Introduction	274

6.2C.1.2	Requirements	274

6.2C.1.2.1	UE Re-establishment delay requirement	274

6.2C.2	Random access for satellite access	275

6.2C.2.1	Introduction	275

6.2C.2.2	Requirements for 4-step RA type	276

6.2C.2.2.1	Contention based random access	276

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	276

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	276

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	276

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	276

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	276

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	277

6.2C.2.2.2	Non-Contention based random access	277

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	277

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	277

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	278

6.2C.2.3	Requirements for 2-step RA type	278

6.2C.2.3.1	Contention based random access	278

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	278

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	278

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	279

6.2C.2.3.2	Non-Contention based random access	279

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	279

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	279

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	279

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	279

6.2C.3.1	Introduction	279

6.2C.3.2	Requirements	280

6.2C.3.2.1	RRC connection release with redirection to NR	280

6.2D	RRC Connection Mobility Control for ATG	280

6.2D.1	SA: RRC Re-establishment	280

6.2D.1.1	Introduction	280

6.2D.1.2	Requirements	281

6.2D.1.2.1	UE Re-establishment delay requirement	281

6.2D.2	Random access	282

6.2D.2.1	Introduction	282

6.2D.2.2	Requirements for 4-step RA type	282

6.2D.2.2.1	Contention based random access	283

6.2D.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	283

6.2D.2.2.1.2	Correct behaviour when receiving Random Access Response	283

6.2D.2.2.1.3	Correct behaviour when not receiving Random Access Response	283

6.2D.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	283

6.2D.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	283

6.2D.2.2.1.6	Correct behaviour when contention Resolution timer expires	283

6.2D.2.2.2	Non-Contention based random access	283

6.2D.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	283

6.2D.2.2.2.2	Correct behaviour when receiving Random Access Response	284

6.2D.2.2.2.3	Correct behaviour when not receiving Random Access Response	284

6.2D.2.3	Requirements for 2-step RA type	284

6.2D.2.3.1	Contention based random access	285

6.2D.2.3.1.1	Correct behaviour when transmitting MsgA	285

6.2D.2.3.1.2	Correct behaviour when receiving MsgB	285

6.2D.2.3.1.3	Correct behaviour when not receiving MsgB	285

6.2D.2.3.2	Non-Contention based random access	285

6.2D.2.3.2.1	Correct behaviour when transmitting MsgA	285

6.2D.2.3.2.2	Correct behaviour when receiving MsgB	286

6.2D.2.3.2.3	Correct behaviour when not receiving MsgB	286

6.2D.3	SA: RRC Connection Release with Redirection	286

6.2D.3.1	Introduction	286

6.2D.3.2	Requirements	286

6.2D.3.2.1	RRC connection release with redirection to NR	286

7	Timing	287

7.1	UE transmit timing	287

7.1.1	Introduction	287

7.1.2	Requirements	287

7.1.2.1	Gradual timing adjustment	289

7.1.2.2	Void	290

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	290

7.1A	UE transmit timing for RedCap	291

7.1A.1	Introduction	291

7.1A.2	Requirements	291

7.1A.2.1	Gradual timing adjustment	292

7.1C	UE transmit timing for Satellite Access	292

7.1C.1	Introduction	292

7.1C.2	Requirements	293

7.1C.2.1	Gradual timing adjustment	293

7.1D	UE transmit timing for ATG	294

7.1D.1	Introduction	294

7.1D.2	Requirements	294

7.1D.2.1	Gradual timing adjustment	295

7.2	UE timer accuracy	295

7.2.1	Introduction	295

7.2.2	Requirements	295

7.2A	UE timer accuracy for RedCap	296

7.2A.1	Introduction	296

7.2A.2	Requirements	296

7.2C	UE timer accuracy for satellite access	296

7.2C.1	Introduction	296

7.2C.2	Requirements	296

7.2D	UE timer accuracy for ATG	297

7.2D.1	Introduction	297

7.2D.2	Requirements	297

7.3	Timing advance	297

7.3.1	Introduction	297

7.3.2	Requirements	297

7.3.2.1	Timing Advance adjustment delay	297

7.3.2.2	Timing Advance adjustment accuracy	297

7.3A	Timing Advance for RedCap	298

7.3A.1	Introduction	298

7.3A.2	Requirements	298

7.3A.2.1	Timing Advance adjustment delay	298

7.3A.2.2	Timing Advance adjustment accuracy	298

7.3C	Timing advance for satellite access	298

7.3C.1	Introduction	298

7.3C.2	Requirements	298

7.3C.2.1	Timing Advance adjustment delay	298

7.3C.2.2	Timing Advance adjustment accuracy	298

7.3D	Timing advance for ATG	299

7.3D.1	Introduction	299

7.3D.2	Requirements	299

7.3D.2.1	Timing Advance adjustment delay	299

7.3D.2.2	Timing Advance adjustment accuracy	299

7.4	Cell phase synchronization accuracy	300

7.4.1	Definition	300

7.4.2	Minimum requirements	300

7.5	Maximum Transmission Timing Difference	300

7.5.1	Introduction	300

7.5.2	Minimum Requirements for inter-band EN-DC	300

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	301

7.5.3	Minimum Requirements for intra-band EN-DC	301

7.5.4	Minimum Requirements for NR Carrier Aggregation	301

7.5.5	Minimum Requirements for inter-band NE-DC	302

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	302

7.5.6	Minimum Requirements for inter-band NR DC	303

7.6	Maximum Receive Timing Difference	303

7.6.1	Introduction	303

7.6.2	Minimum Requirements for inter-band EN-DC	304

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	304

7.6.3	Minimum Requirements for intra-band EN-DC	305

7.6.4	Minimum Requirements for NR Carrier Aggregation	305

7.6.5	Minimum Requirements for inter-band NE-DC	306

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	306

7.6.6	Minimum Requirements for inter-band NR DC	307

7.6.7	Minimum Requirements for PC6 UE in FR2	308

7.7	deriveSSB-IndexFromCell tolerance	308

7.7.1	Minimum requirements	308

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	309

7.7A.1	Minimum requirements	309

7.7D	DeriveSSB-IndexFromCell tolerance for ATG	309

7.7D.1	Minimum requirements	309

7.8	Void	309

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	309

7.9.1	Minimum requirements	309

7.9D	DeriveSSB-IndexFromCellInter-r17 tolerance for ATG	309

7.9D.1	Minimum requirements	309

8	Signalling characteristics	310

8.1	Radio Link Monitoring	310

8.1.1	Introduction	310

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	311

8.1.2	Requirements for SSB based radio link monitoring	311

8.1.2.1	Introduction	312

8.1.2.2	Minimum requirement	313

8.1.2.3	Measurement restrictions for SSB based RLM	317

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	317

8.1.3	Requirements for CSI-RS based radio link monitoring	318

8.1.3.1	Introduction	318

8.1.3.2	Minimum requirement	319

8.1.3.3	Measurement restrictions for CSI-RS based RLM	322

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	323

8.1.4	Minimum requirement at transitions	324

8.1.5	Minimum requirement for UE turning off the transmitter	324

8.1.6	Minimum requirement for L1 indication	324

8.1.7	Scheduling availability of UE during radio link monitoring	325

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	325

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	325

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	325

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	326

8.1.8	Minimum requirement under IDC Interference	326

8.1A	Radio Link Monitoring with CCA on Target Frequency	326

8.1A.1	Introduction	326

8.1A.2	Requirements for SSB Based Radio Link Monitoring	327

8.1A.2.1	Introduction	327

8.1A.2.2	Minimum Requirement	328

8.1A.3	Minimum requirement at transitions	331

8.1A.4	Minimum requirement for UE turning off the transmitter	332

8.1A.5	Minimum requirement for L1 indication	332

8.1A.6	Scheduling availability of UE during radio link monitoring	332

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	332

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	333

8.1B	Radio Link Monitoring for RedCap	333

8.1B.1	Introduction	333

8.1B.2	Requirements for SSB based radio link monitoring	334

8.1B.2.1	Introduction	334

8.1B.2.2	Minimum requirement	335

8.1B.2.3	Measurement restrictions for SSB based RLM	337

8.1B.3	Requirements for CSI-RS based radio link monitoring	338

8.1B.3.1	Introduction	338

8.1B.3.2	Minimum requirement	338

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	341

8.1B.4	Minimum requirement at transitions	342

8.1B.5	Minimum requirement for UE turning off the transmitter	342

8.1B.6	Minimum requirement for L1 indication	342

8.1B.7	Scheduling availability of UE during radio link monitoring	342

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	343

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	343

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	343

8.1C	Radio Link Monitoring for Satellite Access	343

8.1C.1	Introduction	343

8.1C.2	Requirements for SSB based radio link monitoring	344

8.1C.2.1	Introduction	344

8.1C.2.2	Minimum requirement	345

8.1C.2.3	Measurement restrictions for SSB based RLM	346

8.1C.3	Requirements for CSI-RS based radio link monitoring	346

8.1C.3.1	Introduction	346

8.1C.3.2	Minimum requirement	347

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	348

8.1C.4	Minimum requirement at transitions	349

8.1C.5	Minimum requirement for UE turning off the transmitter	349

8.1C.6	Minimum requirement for L1 indication	349

8.1C.7	Scheduling availability of UE during radio link monitoring	349

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	350

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	350

8.1D	Radio Link Monitoring for ATG	350

8.1D.1	Introduction	350

8.1D.2	Requirements for SSB based radio link monitoring	351

8.1D.2.1	Introduction	351

8.1D.2.2	Minimum requirement	352

8.1D.2.3	Measurement restrictions for SSB based RLM	352

8.1D.3	Requirements for CSI-RS based radio link monitoring	353

8.1D.3.1	Introduction	353

8.1D.3.2	Minimum requirement	353

8.1D.3.3	Measurement restrictions for CSI-RS based RLM	354

8.1D.4	Minimum requirement at transitions	355

8.1D.5	Minimum requirement for UE turning off the transmitter	355

8.1D.6	Minimum requirement for L1 indication	355

8.1D.7	Scheduling availability of UE during radio link monitoring	355

8.1D.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	356

8.1D.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	356

8.2	Interruption	356

8.2.1	EN-DC Interruption	356

8.2.1.1	Introduction	356

8.2.1.2	Requirements	357

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	357

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	357

8.2.1.2.3	Interruptions at SCell addition/release	357

8.2.1.2.4	Interruptions at SCell activation/deactivation	359

8.2.1.2.5	Interruptions during measurements on SCC	361

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	361

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	361

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	361

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	362

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	362

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	362

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	364

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	364

8.2.1.2.8.2	Interruptions during direct SCell activation	364

8.2.1.2.9	Interruptions at SCell hibernation	364

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	364

8.2.1.2.11	Interruptions due to UE-specific CBW change	365

8.2.1.2.12	Interruptions at NR SRS carrier based switching	365

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	366

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	367

8.2.1.2.15	Interruptions due to SCell dormancy	368

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	368

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	368

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	368

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	368

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	369

8.2.1.2.18	Interruptions at NR SRS antenna port switching	369

8.2.1.2.19	Interruptions at fast SCell activation	371

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	371

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	372

8.2.2.1	Introduction	372

8.2.2.2	Requirements	372

8.2.2.2.1	Interruptions at SCell addition/release	372

8.2.2.2.2	Interruptions at SCell activation/deactivation	374

8.2.2.2.3	Interruptions during measurements on deactivated SCC	375

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	376

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	376

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	378

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	379

8.2.2.2.8	Interruptions due to UE-specific CBW change	379

8.2.2.2.9	Interruptions at NR SRS carrier based switching	379

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	381

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	381

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	382

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	382

8.2.2.2.10D	DL Interruptions at UE switching across three or four uplink bands	383

8.2.2.2.11	Interruptions at direct SCell activation	383

8.2.2.2.12	Interruptions due to SCell dormancy	384

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	384

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	384

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	384

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	384

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	384

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	385

8.2.2.2.16	Interruptions at NR SRS antenna port switching	385

8.2.2.2.17	Interruptions at fast SCell activation	386

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	387

8.2.3	NE-DC Interruptions	387

8.2.3.1	Introduction	387

8.2.3.2	Requirements	388

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	388

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	388

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	388

8.2.3.2.4	Interruptions at SCell activation/deactivation	390

8.2.3.2.5	Interruptions during measurements on SCC	391

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	391

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	391

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	392

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	392

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	392

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	393

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	393

8.2.3.2.9	Interruptions at SCell hibernation	393

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	394

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	394

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	395

8.2.3.2.13	Interruptions due to SCell dormancy	396

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	397

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	397

8.2.3.2.17	Interruptions at fast SCell activation	399

8.2.3.2.18	Interruptions due to UE-specific CBW change	399

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	400

8.2.4	NR-DC: Interruptions	400

8.2.4.1	Introduction	400

8.2.4.2	Requirements	401

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	401

8.2.4.2.2	Interruptions at SCell activation/deactivation	402

8.2.4.2.3	Interruptions during measurements on SCC	403

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	403

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	404

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	404

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	404

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	405

8.2.4.2.9	Interruptions at NR SRS carrier based switching	405

8.2.4.2.10	Interruptions at direct SCell activation	406

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	407

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	407

8.2.4.2.13	 Interruptions due to SCell dormancy	408

8.2.4.2.14	Interruptions at NR SRS antenna port switching	408

8.2.4.2.15	Interruptions at fast SCell activation	409

8.2.4.2.16	Interruptions at SCG activation/deactivation	410

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	410

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	411

8.2.4.2.19	Interruptions due to UE-specific CBW change	411

8.2.4.2A	Void	411

8.2.4.2A.1	Void	411

8.2.4.2A.2	Void	411

8.2.4.2A.3	Void	411

8.3	SCell Activation and Deactivation Delay	411

8.3.1	Introduction	411

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	411

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	416

8.3.4	Direct SCell Activation at SCell addition	416

8.3.5	Direct SCell Activation at Handover	418

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	420

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	424

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	424

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	425

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	427

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	429

8.3.13.1	Introduction	429

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	431

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	431

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	432

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	435

8.3A.1	Introduction	435

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	435

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	439

8.4	UE UL carrier RRC reconfiguration delay	440

8.4.1	Introduction	440

8.4.2	UE UL carrier configuration delay requirement	440

8.4.3	UE UL carrier deconfiguration delay requirement	440

8.5	Link Recovery Procedures	440

8.5.1	Introduction	440

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	441

8.5.2	Requirements for SSB based beam failure detection	442

8.5.2.1	Introduction	442

8.5.2.2	Minimum requirement	443

8.5.2.3	Measurement restriction for SSB based beam failure detection	446

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	447

8.5.3	Requirements for CSI-RS based beam failure detection	448

8.5.3.1	Introduction	448

8.5.3.2	Minimum requirement	448

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	452

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	453

8.5.4	Minimum requirement for L1 indication	453

8.5.5	Requirements for SSB based candidate beam detection	454

8.5.5.1	Introduction	454

8.5.5.2	Minimum requirement	454

8.5.5.3	Measurement restriction for SSB based candidate beam detection	457

8.5.6	Requirements for CSI-RS based candidate beam detection	458

8.5.6.1	Introduction	458

8.5.6.2	Minimum requirement	458

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	461

8.5.7	Scheduling availability of UE during beam failure detection	461

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	462

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	462

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	462

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	463

8.5.8	Scheduling availability of UE during candidate beam detection	463

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	463

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	463

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	463

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	464

8.5.9	Requirements for Beam Failure Recovery in SCell	464

8.5.9.1	Introduction	464

8.5.9.2	Requirement	464

8.5.10	Minimum requirement at transitions for beam failure detection	465

8.5.11	Minimum requirement under IDC Interference	465

8.5A	Link Recovery Procedures when CCA is used on target frequency	465

8.5A.1	Introduction	465

8.5A.2	Requirements for SSB based beam failure detection	466

8.5A.2.1	Introduction	466

8.5A.2.2	Minimum requirement	466

8.5A.2.3	Measurement restriction for SSB based beam failure detection	468

8.5A.3	Void	469

8.5A.4	Minimum requirement for L1 indication	469

8.5A.5	Requirements for SSB based candidate beam detection	469

8.5A.5.1	Introduction	469

8.5A.5.2	Minimum requirement	469

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	472

8.5A.6	Void	472

8.5A.7	Scheduling availability of UE during beam failure detection	472

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	472

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	473

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	473

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	473

8.5A.8	Scheduling availability of UE during candidate beam detection	473

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	473

8.5.8A.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	473

8.5B	Link Recovery Procedures for Redcap	473

8.5B.1	Introduction	473

8.5B.2	Requirements for SSB based beam failure detection for Redcap	474

8.5B.2.1	Introduction	474

8.5B.2.2	Minimum requirement	474

8.5B.2.3	Measurement restriction for SSB based beam failure detection	476

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	476

8.5B.3.1	Introduction	476

8.5B.3.2	Minimum requirement	477

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	479

8.5B.4	Minimum requirement for L1 indication for Redcap	480

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	480

8.5B.5.1	Introduction	480

8.5B.5.2	Minimum requirement	480

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	482

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	482

8.5B.6.1	Introduction	482

8.5B.6.2	Minimum requirement	483

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	485

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	485

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	485

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	485

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	485

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	486

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	486

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	486

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	486

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	487

8.5C	Link Recovery Procedures for Satellite Access	487

8.5C.1	Introduction	487

8.5C.2	Requirements for SSB based beam failure detection	488

8.5C.2.1	Introduction	488

8.5C.2.2	Minimum requirement	488

8.5C.2.3	Measurement restriction for SSB based beam failure detection	489

8.5C.3	Requirements for CSI-RS based beam failure detection	489

8.5C.3.1	Introduction	489

8.5C.3.2	Minimum requirement	490

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	491

8.5C.4	Minimum requirement for L1 indication	491

8.5C.5	Requirements for SSB based candidate beam detection	492

8.5C.5.1	Introduction	492

8.5C.5.2	Minimum requirement	492

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	493

8.5C.6	Requirements for CSI-RS based candidate beam detection	493

8.5C.6.1	Introduction	493

8.5C.6.2	Minimum requirement	493

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	494

8.5C.7	Scheduling availability of UE during beam failure detection	495

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	495

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	495

8.5C.8	Scheduling availability of UE during candidate beam detection	495

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	495

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	495

8.5C.9	Minimum requirement at transitions for beam failure detection	496

8.5D	Link Recovery Procedures for ATG	496

8.5D.1	Introduction	496

8.5D.2	Requirements for SSB based beam failure detection	497

8.5D.2.1	Introduction	497

8.5D.2.2	Minimum requirement	497

8.5D.2.3	Measurement restriction for SSB based beam failure detection	498

8.5D.3	Requirements for CSI-RS based beam failure detection	498

8.5D.3.1	Introduction	498

8.5D.3.2	Minimum requirement	499

8.5D.3.3	Measurement restrictions for CSI-RS beam failure detection	499

8.5D.4	Minimum requirement for L1 indication	500

8.5D.5	Requirements for SSB based candidate beam detection	500

8.5D.5.1	Introduction	500

8.5D.5.2	Minimum requirement	500

8.5D.5.3	Measurement restriction for SSB based candidate beam detection	501

8.5D.6	Requirements for CSI-RS based candidate beam detection	501

8.5D.6.1	Introduction	501

8.5D.6.2	Minimum requirement	501

8.5D.6.3	Measurement restriction for CSI-RS based candidate beam detection	502

8.5D.7	Scheduling availability of UE during beam failure detection	503

8.5D.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	503

8.5D.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	503

8.5D.8	Scheduling availability of UE during candidate beam detection	503

8.5D.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	503

8.5D.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	503

8.5D.9	Minimum requirement at transitions for beam failure detection	504

8.6	Active BWP switch delay	504

8.6.1	Introduction	504

8.6.2	DCI and timer based BWP switch delay on a single CC	504

8.6.2A	DCI based BWP switch delay on multiple CCs	506

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	506

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	507

8.6.2B	Timer based BWP switch delay on multiple CCs	507

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	508

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	508

8.6.3	RRC based BWP switch delay on a single CC	509

8.6.3A	RRC based BWP switch delay on multiple CCs	509

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	509

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	510

8.6.4	BWP switch delay on Consistent UL CCA recovery	510

8.6A	Active BWP switch delay for RedCap	511

8.6A.1	Introduction	511

8.6A.2	DCI and timer based BWP switch delay on a single CC	511

8.6A.3	RRC based BWP switch delay on a single CC	512

8.6C	Active BWP switch delay for satellite access	513

8.6C.1	Introduction	513

8.6C.2	DCI and timer based BWP switch delay on a single CC	513

8.6C.3	RRC based BWP switch delay on a single CC	515

8.6D	Active BWP switch delay for ATG	515

8.6D.1	Introduction	515

8.6D.2	DCI and timer based BWP switch delay	515

8.6D.3	RRC based BWP switch delay on a single CC	516

8.7	Void	517

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	517

8.8.1	Introduction	517

8.8.2	E-UTRAN PSCell Addition Delay Requirement	517

8.8.3	E-UTRAN PSCell Release Delay Requirement	517

8.9	NR-DC: PSCell Addition and Release Delay	518

8.9.1	Introduction	518

8.9.2	PSCell Addition Delay Requirement	518

8.9.3	PSCell Release Delay Requirement	519

8.9A	Conditional PSCell Addition Delay	519

8.9A.1	Introduction	519

8.9A.2	Conditional PSCell Addition Delay Requirement	519

8.9A.2.1	Measurement time	520

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	520

8.9B.1	Introduction	520

8.9B.2	PSCell Addition Delay Requirement	520

8.9B.3	PSCell Release Delay Requirement	521

8.10	Active TCI state switching delay	521

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	522

8.10.4	DCI based TCI state switch delay	523

8.10.5	RRC based TCI state switch delay	523

8.10.6	Active TCI state list update delay	524

8.10A	Active TCI state switching delay with CCA	524

8.10A.1	Introduction	524

8.10A.2	Known conditions for TCI state	524

8.10A.3	MAC-CE based TCI state switch delay	524

8.10A.4	DCI based TCI state switch delay	525

8.10A.5	RRC based TCI state switch delay	526

8.10A.6	Active TCI state list update delay	527

8.10B	Active TCI state switching delay for RedCap	527

8.10B.1	Introduction	527

8.10B.2	Known conditions for TCI state	527

8.10B.3	MAC-CE based TCI state switch delay	527

8.10B.4	DCI based TCI state switch delay	528

8.10B.5	RRC based TCI state switch delay	528

8.10B.6	Active TCI state list update delay	529

8.10C	Active TCI state switching delay for satellite access	529

8.10C.1	Introduction	529

8.10C.2	MAC-CE based TCI state switch delay	529

8.10C.4	DCI based TCI state switch delay	529

8.10C.5	RRC based TCI state switch delay	530

8.10C.6	Active TCI state list update delay	530

8.10D	Active TCI state switching delay for ATG	530

8.10D.2	Known conditions for TCI state	530

8.10D.6	Active TCI state list update delay	531

8.11	PSCell Change	531

8.11A	PSCell Change in Carriers with CCA	532

8.11B	Conditional PSCell Change	532

8.11B.1	Introduction	532

8.11B.2	Conditoinal PSCell Change delay	532

8.11B.2.1	Measurement time	533

8.11D	Conditional PSCell Change in Carriers with CCA	533

8.11D.1	Introduction	533

8.11D.2	Conditional PSCell Change delay	533

8.11D.2.1	Measurement time	534

8.12	Uplink spatial relation switch delay	535

8.12.1	Introduction	535

8.12.2	Known conditions for spatial relation when associated with DL-RS	535

8.12.3	MAC-CE based spatial relation switch delay	535

8.12.4	DCI based spatial relation switch delay	536

8.12.5	RRC based spatial relation switch delay	536

8.12A	Uplink spatial relation switch delay for RedCap	537

8.12A.1	Introduction	537

8.12A.2	Known conditions for spatial relation when associated with DL-RS	537

8.12A.3	MAC-CE based spatial relation switch delay	537

8.12A.4	DCI based spatial relation switch delay	538

8.12A.5	RRC based spatial relation switch delay	538

8.12C	Uplink spatial relation switch delay for satellite access	539

8.12C.1	Void	539

8.12C.2	Void	539

8.12C.3	Void	539

8.12C.4	Void	539

8.12C.5	Void	539

8.13	UE-specific CBW change	539

8.13.1	Introduction	539

8.13.2	UE-specific CBW change delay	539

8.13A	UE-specific CBW change for RedCap	540

8.13A.1	Introduction	540

8.13A.2	UE-specific CBW change delay	540

8.13C	UE-specific CBW change for satellite access	540

8.13C.1	Introduction	540

8.13C.2	UE-specific CBW change delay	540

8.13D	UE-specific CBW change for ATG	541

8.13D.1	Introduction	541

8.13D.2	UE-specific CBW change delay	541

8.14	Pathloss reference signal switching delay	541

8.14.1	Introduction	541

8.14.2	Known conditions for pathloss reference signal	541

8.14.3	MAC-CE based pathloss reference signal switch delay	542

8.14C	Pathloss reference signal switching delay for satellite access	542

8.14C.1	Introduction	543

8.14C.2	Known conditions for pathloss reference signal	543

8.14C.3	MAC-CE based pathloss reference signal switch delay	543

8.14D	Pathloss reference signal switching delay for ATG	544

8.14D.1	Introduction	544

8.14D.2	Known conditions for pathloss reference signal	544

8.14D.3	MAC-CE based pathloss reference signal switch delay	544

8.15	Active downlink TCI state switching delay for unified TCI	545

8.15.1	Introduction	545

8.15.4	DCI based downlink TCI state switch delay	546

8.15.5	Active Downlink TCI state list update delay	547

8.15D	Active downlink TCI state switching delay for unified TCI for ATG	548

8.15D.1	Introduction	548

8.15D.2	Known conditions for downlink TCI state	548

8.15D.4	DCI based downlink TCI state switch delay	548

8.15D.5	Active Downlink TCI state list update delay	549

8.16	Active uplink TCI state switching delay for unified TCI	549

8.16.1	Introduction	549

8.16.4	DCI based uplink TCI state switch delay	551

8.16.5	Active Uplink TCI state list update delay	552

8.16D	Active uplink TCI state switching delay for unified TCI for ATG	553

8.16D.1	Introduction	553

8.16D.2	Known conditions for uplink TCI state	553

8.16D.4	DCI based uplink TCI state switch delay	554

8.16D.5	Active Uplink TCI state list update delay	555

8.17	SCG Activation and Deactivation Delay	555

8.17.1	Introduction	555

8.17.2	SCG Activation Delay Requirement	555

8.17.3	SCG Deactivation Delay Requirement	556

8.18	TRP specific Link Recovery Procedures	557

8.18.1	Introduction	557

8.18.2	Requirements for TRP specific SSB based beam failure detection	557

8.18.2.1	Introduction	557

8.18.2.2	Minimum requirement	558

8.18.2.3	Measurement restriction for SSB based beam failure detection	560

8.18.3	Requirements for CSI-RS based beam failure detection	560

8.18.3.1	Introduction	560

8.18.3.2	Minimum requirement	561

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	563

8.18.4	Minimum requirement for L1 indication	564

8.18.5	Requirements for SSB based candidate beam detection	564

8.18.5.1	Introduction	564

8.18.5.2	Minimum requirement	564

8.18.5.3	Measurement restriction for SSB based candidate beam detection	566

8.18.6	Requirements for CSI-RS based candidate beam detection	567

8.18.6.1	Introduction	567

8.18.6.2	Minimum requirement	567

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	569

8.18.7	Requirements for TRP specific Beam Failure Recovery	570

8.18.7.1	Introduction	570

8.18.7.2	Requirement	570

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	570

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	570

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	571

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	571

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	572

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	572

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	572

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	572

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	572

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	573

8.19	Pre-configured measurement gap activation/deactivation delay	573

8.19.1	Introduction	573

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	573

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	573

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	573

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	574

8.19D	Pre-configured measurement gap activation/deactivation delay for ATG	574

8.19D.1	Introduction	574

8.19D.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	574

8.19D.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	574

8.19D.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	574

9	Measurement Procedure	576

9.1	General measurement requirement	576

9.1.1	Introduction	576

9.1.2	Measurement gap	576

9.1.2.1	EN-DC: Measurement Gap Sharing	588

9.1.2.1a	SA: Measurement Gap Sharing	589

9.1.2.1b	NE-DC: Measurement Gap Sharing	590

9.1.2.1c	NR-DC: Measurement Gap Sharing	591

9.1.3	UE Measurement capability	592

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	592

9.1.3.1a	SA: Monitoring of multiple layers using gaps	592

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	593

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	593

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	594

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	595

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	595

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	596

9.1A.3.2	Void	597

9.1.3A	UE Measurement capability under operation mode with CCA	597

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	597

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	597

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	597

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	598

9.1.3C	UE Measurement capability under operation mode with satellite access	598

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	598

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	599

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	599

9.1.4.1	Introduction	599

9.1.4.2	Requirements	599

9.1.5	Carrier-specific scaling factor	603

9.1.5.1	Monitoring of multiple layers outside gaps	604

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	606

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	607

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	608

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	608

9.1.5.2	Monitoring of multiple layers within gaps	609

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	611

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	613

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	615

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	617

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	618

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	619

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	619

9.1.5.3	Monitoring of multiple layers within NCSG	619

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	620

9.1.6	Minimum requirement at transitions	620

9.1.7	Pre-configured measurement gap	621

9.1.7.1	Introduction	621

9.1.7.2	Requirements applicability	621

9.1.7.3	Requirements	621

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	622

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	622

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	623

9.1.8	Concurrent measurement gaps	623

9.1.8.1	Introduction	623

9.1.8.2	Requirements	623

9.1.8.3	Collision between concurrent measurement gaps	625

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	625

9.1.9	Network controlled small gap	625

9.1.9.1	Introduction	625

9.1.9.2	Requirements applicability	626

9.1.10	MUSIM gaps	628

9.1.11	UL gap for Tx power management	629

9.1A	General measurement requirement for RedCap	630

9.1A.1	Introduction	630

9.1A.2	Measurement gap	630

9.1A.2.1	SA: Measurement Gap Sharing	634

9.1A.3	UE Measurement capability	635

9.1A.3.1	SA: Monitoring of multiple layers using gaps	635

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	635

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	635

9.1A.4.1	Introduction	635

9.1A.4.2	Requirements	636

9.1A.5	Carrier-specific scaling factor	636

9.1A.5.1	Monitoring of multiple layers outside gaps	636

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	637

9.1A.5.2	Monitoring of multiple layers within gaps	637

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	637

9.1A.6	Minimum requirement at transitions	638

9.1C	General measurement requirement for SAN	639

9.1C.1	Introduction	639

9.1C.2	Measurement gap	639

9.1C.8	Concurrent measurement gaps for SAN	641

9.1C.8.1	Introduction	641

9.1C.8.2	Requirements	641

9.1C.8.3	Collision between concurrent measurement gaps	641

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	642

9.1C.9	Collision between SMTC and measurement gap for SAN	642

9.1C.9.1	Introduction	642

9.1C.9.2	Collision between SMTCs and measurement gap	642

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	643

9.1D	General measurement requirement for ATG	643

9.1D.1	Introduction	643

9.1D.2	Measurement gap	643

9.1D.2.1a	SA: Measurement Gap Sharing	646

9.1D.3	UE Measurement capability	647

9.1D.3.1	SA: Monitoring of multiple layers using gaps	647

9.1D.3.2	SA: Maximum allowed layers for multiple monitoring	647

9.1D.4	Void	647

9.1D.5	Carrier-specific scaling factor	647

9.1D.5.1	Monitoring of multiple layers outside gaps	647

9.1D.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	648

9.1D.5.2	Monitoring of multiple layers within gaps	648

9.1D.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	649

9.1D.6	Void	649

9.1D.7	Pre-configured measurement gap	649

9.1D.7.1	Introduction	649

9.1D.7.2	Requirements applicability	649

9.1D.7.3	Requirements	650

9.1D.7.3.1	Requirements for autonomous activation/deactivation mechanism	650

9.1D.7.3.2	Requirements for network-controlled activation/deactivation mechanism	650

9.1D.7.3.3	Requirements for reception/transmission during activation/deactivation	651

9.2	NR intra-frequency measurements	651

9.2.1	Introduction	651

9.2.2	Requirements applicability	652

9.2.3	Number of cells and number of SSB	653

9.2.3.1	Requirements for FR1	653

9.2.3.2	Requirements for FR2	653

9.2.4	Measurement Reporting Requirements	653

9.2.4.1	Periodic Reporting	653

9.2.4.2	Event-triggered Periodic Reporting	653

9.2.4.3	Event Triggered Reporting	653

9.2.5	Intrafrequency measurements without measurement gaps	654

9.2.5.1	Intrafrequency cell identification	654

9.2.5.2	Measurement period	659

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	662

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	662

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	663

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	663

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	665

9.2.5.4	SFTD Measurements between PCell and PSCell	665

9.2.5.4.1	Introduction	665

9.2.5.4.2	SFTD Measurement delay	665

9.2.5.4.3	SFTD Measurement Reporting Delay	666

9.2.6	Intra-frequency measurements with measurement gaps	666

9.2.6.1	Void	666

9.2.6.2	Intra-frequency cell identification	666

9.2.6.3	Intrafrequency Measurement Period	669

9.2.7	Intra-frequency measurements with NCSG	671

9.2.7.1	Intra-frequency cell identification	671

9.2.7.2	Measurement period	673

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	674

9.2A	NR intra-frequency measurements with CCA	674

9.2A.1	Introduction	674

9.2A.2	Requirements applicability	675

9.2A.3	Number of cells and number of SSB	675

9.2A.3.1	Requirements for FR1	675

9.2A.3.2	Requirements for FR2-2	676

9.2A.4	Measurement Reporting Requirements	676

9.2A.5	Intra-frequency measurements without measurement gaps	677

9.2A.5.2	Measurement period	681

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	683

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	684

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	684

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	684

9.2A.6	Intra-frequency measurements with measurement gaps	684

9.2A.6.1	Intra-frequency cell identification	684

9.2A.6.2	Intra-frequency Measurement Period	687

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	688

9.2A.7.1	Intra-frequency RSSI measurements	688

9.2A.7.2	Intra-frequency Channel occupancy measurements	690

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	691

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	691

9.2B	NR intra-frequency measurements for RedCap	692

9.2B.1	Introduction	692

9.2B.2	Requirements applicability	692

9.2B.3	Number of cells and number of SSB	693

9.2B.3.1	Requirements for FR1	693

9.2B.3.2	Requirements for FR2	693

9.2B.4	Measurement Reporting Requirements	693

9.2B.4.1	Periodic Reporting	693

9.2B.4.2	Event-triggered Periodic Reporting	693

9.2B.4.3	Event Triggered Reporting	693

9.2B.5	Intra-frequency measurements without measurement gaps	694

9.2B.5.1	Intra-frequency cell identification	694

9.2B.5.2	Measurement period	696

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	697

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	697

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	697

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	698

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	698

9.2B.6	Intra-frequency measurements with measurement gaps	699

9.2B.6.1	Intra-frequency cell identification	699

9.2B.6.2	Intra-frequency Measurement Period	700

9.2C	NR intra-frequency measurements for SAN	701

9.2C.1	Introduction	701

9.2C.2	Requirements applicability	702

9.2C.3	Number of cells and number of SSB	702

9.2C.3.1	Requirements for FR1	702

9.2C.4	Measurement Reporting Requirements	702

9.2C.4.1	Periodic Reporting	702

9.2C.4.2	Event-triggered Periodic Reporting	703

9.2C.4.3	Event Triggered Reporting	703

9.2C.5	Intra frequency measurements without measurement gaps	703

9.2C.5.1	Intra frequency cell identification	703

9.2C.5.2	Measurement period	705

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	706

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	706

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	706

9.2C.6	Intra-frequency measurements with measurement gaps	707

9.2C.6.1	Intra-frequency cell identification	707

9.2C.6.3	Intrafrequency Measurement Period	708

9.2D	NR intra-frequency measurements for ATG	708

9.2D.1	Introduction	708

9.2D.2	Requirements applicability	709

9.2D.3	Number of cells and number of SSB	709

9.2D.3.1	Requirements for FR1	709

9.2D.4	Measurement Reporting Requirements	709

9.2D.4.1	Periodic Reporting	709

9.2D.4.2	Event-triggered Periodic Reporting	709

9.2D.4.3	Event Triggered Reporting	709

9.2D.5	Intra-frequency measurements without measurement gaps	710

9.2D.5.1	Intra-frequency cell identification	710

9.2D.5.2	Measurement period	711

9.2D.5.3	Scheduling availability of UE during intra-frequency measurements	712

9.2D.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	712

9.2D.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	713

9.2D.6	Intra-frequency measurements with measurement gaps	713

9.2D.6.1	Void	713

9.2D.6.2	Intra-frequency cell identification	713

9.2D.6.3	Intra-frequency Measurement Period	714

9.3	NR inter-frequency measurements	715

9.3.1	Introduction	715

9.3.2	Requirements applicability	716

9.3.2.1	Void	716

9.3.2.2	Void	716

9.3.3	Number of cells and number of SSB	716

9.3.3.1	Requirements for FR1	716

9.3.3.2	Requirements for FR2	716

9.3.4	Inter-frequency measurement with measurement gaps	717

9.3.4.1	Void	721

9.3.4.2	Void	721

9.3.5	Inter-frequency measurements	721

9.3.5.1	Void	723

9.3.5.2	Void	723

9.3.5.3	Void	723

9.3.6	Inter-frequency measurements reporting requirements	723

9.3.6.1	Periodic Reporting	723

9.3.6.2	Event-triggered Periodic Reporting	723

9.3.6.3	Event-triggered Reporting	723

9.3.7	Void	724

9.3.8	Inter-frequency SFTD measurement requirements	724

9.3.8.1	Introduction	724

9.3.8.2	SFTD Measurement delay	724

9.3.8.3	SFTD Measurement reporting delay	725

9.3.9	Inter frequency measurements without measurement gaps	725

9.3.9.1	Inter frequency Cell identification	725

9.3.9.2	Measurement period	728

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	730

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	730

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	730

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	731

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	731

9.3.10	Inter-frequency measurement with NCSG	731

9.3.10.1	Inter-frequency cell identification	731

9.3.10.2	Measurement period	733

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	733

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	734

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	734

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	735

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	737

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	737

9.3A.1	Introduction	737

9.3A.2	Requirements applicability	738

9.3A.3	Number of cells and number of SSB	738

9.3A.3.1	Requirements for FR1	738

9.3A.3.2	Requirements for FR2-2	738

9.3A.4	Inter-frequency cell identification	738

9.3A.5	Inter-frequency measurements	741

9.3A.6	NR Inter-frequency measurements reporting requirements	742

9.3A.6.1	Periodic Reporting	742

9.3A.6.2	Event-triggered Periodic Reporting	742

9.3A.6.3	Event-triggered Reporting	742

9.3A.8	Inter-frequency RSSI measurements	743

9.3A.9	Inter-frequency channel occupancy measurements	744

9.3B	NR inter-frequency measurements for RedCap	744

9.3B.1	Introduction	744

9.3B.2	Requirements applicability	745

9.3B.3	Number of cells and number of SSB	745

9.3B.3.1	Requirements for FR1	745

9.3B.3.2	Requirements for FR2	745

9.3B.4	Inter-frequency measurement with measurement gaps	746

9.3B.5	Inter-frequency measurements	747

9.3B.6	Inter-frequency measurements reporting requirements	748

9.3B.6.1	Periodic Reporting	748

9.3B.6.2	Event-triggered Periodic Reporting	748

9.3B.6.3	Event-triggered Reporting	748

9.3B.7	Inter frequency measurements without measurement gaps	749

9.3B.7.1	Inter frequency Cell identification	749

9.3B.7.2	Measurement period	751

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	752

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	752

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	752

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	753

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	753

9.3C	NR inter-frequency measurements for SAN	753

9.3C.1	Introduction	753

9.3C.2	Requirements applicability	754

9.3C.3	Number of cells and number of SSB	754

9.3C.3.1	Requirements for FR1	754

9.3C.4	Inter-frequency measurement with measurement gaps	754

9.3C.5	Inter-frequency measurements	756

9.3C.6	Inter-frequency measurements reporting requirements	756

9.3C.6.1	Periodic Reporting	756

9.3C.6.2	Event-triggered Periodic Reporting	756

9.3C.6.3	Event-triggered Reporting	756

9.3C.7	Inter frequency measurements without measurement gaps	757

9.3C.7.1	Inter frequency Cell identification	757

9.3C.7.2	Measurement period	759

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	759

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	759

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	759

9.3D	NR inter-frequency measurements for ATG	760

9.3D.1	Introduction	760

9.3D.2	Requirements applicability	760

9.3D.3	Number of cells and number of SSB	761

9.3D.3.1	Requirements for FR1	761

9.3D.4	Inter-frequency measurement with measurement gaps	761

9.3D.5	Inter-frequency measurements	761

9.3D.6	Inter-frequency measurements reporting requirements	761

9.3D.7	Void	761

9.3D.8	Void	761

9.3D.9	Inter frequency measurements without measurement gaps	761

9.3D.9.1	Inter frequency Cell identification	761

9.3D.9.2	Measurement period	763

9.3D.9.3	Scheduling availability of UE during inter-frequency measurements	763

9.3D.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	764

9.3D.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	764

9.4	Inter-RAT measurements	765

9.4.1	Introduction	765

9.4.2	NR − E-UTRAN FDD measurements	767

9.4.2.1	Introduction	767

9.4.2.2	Requirements when no DRX is used	767

9.4.2.3	Requirements when DRX is used	768

9.4.2.4	Measurement reporting requirements	770

9.4.2.4.1	Periodic Reporting	770

9.4.2.4.2	Event-Triggered Periodic Reporting	770

9.4.2.4.3	Event-Triggered Reporting	771

9.4.3	NR − E-UTRAN TDD measurements	771

9.4.3.1	Introduction	771

9.4.3.2	Requirements when no DRX is used	771

9.4.3.3	Requirements when DRX is used	773

9.4.3.4	Measurement reporting requirements	774

9.4.3.4.1	Periodic Reporting	774

9.4.3.4.2	Event-Triggered Periodic Reporting	775

9.4.3.4.3	Event-Triggered Reporting	775

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	775

9.4.4	Inter-RAT RSTD measurements	775

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	775

9.4.4.1.1	Introduction	775

9.4.4.1.2	Requirements	776

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	779

9.4.4.2.1	Introduction	779

9.4.4.2.2	Requirements	780

9.4.5	Inter-RAT E-CID measurements	783

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	783

9.4.5.1.1	Introduction	783

9.4.5.1.2	Requirements	783

9.4.5.1.3	Measurement Reporting Delay	783

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	783

9.4.5.2.1	Introduction	783

9.4.5.2.2	Requirements	783

9.4.5.2.3	Measurement Reporting Delay	784

9.4.6	NR − UTRAN FDD measurements	784

9.4.6.1	Introduction	784

9.4.6.2	Requirements when no DRX is used	784

9.4.6.3	Requirements when DRX is used	785

9.4.7	NR – E-UTRAN measurements with autonomous gaps	787

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	787

9.4.7.2	CGI reporting delay	787

9.4A	Inter-RAT measurements for RedCap	788

9.4A.1	Introduction	788

9.4A.2	NR − E-UTRAN FDD measurements	789

9.4A.2.1	Introduction	789

9.4A.2.2	Requirements when no DRX is used	789

9.4A.2.3	Requirements when DRX is used	790

9.4A.2.4	Measurement reporting requirements	791

9.4A.2.4.1	Periodic Reporting	791

9.4A.2.4.2	Event-Triggered Periodic Reporting	791

9.4A.2.4.3	Event-Triggered Reporting	791

9.4A.3	NR − E-UTRAN TDD measurements	792

9.4A.3.1	Introduction	792

9.4A.3.2	Requirements when no DRX is used	792

9.4A.3.3	Requirements when DRX is used	794

9.4A.3.4	Measurement reporting requirements	795

9.4A.3.4.1	Periodic Reporting	795

9.4A.3.4.2	Event-Triggered Periodic Reporting	795

9.4A.3.4.3	Event-Triggered Reporting	795

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	796

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	796

9.4A.4.2	CGI reporting delay	796

9.4A.4.3	CGI reporting scheduling restriction	796

9.5	L1-RSRP measurements for Reporting	797

9.5.1	Introduction	797

9.5.2	Requirements applicability	797

9.5.3	Measurement Reporting Requirements	798

9.5.3.1	Periodic Reporting	798

9.5.3.2	Semi-Persistent Reporting	798

9.5.3.3	Aperiodic Reporting	798

9.5.4	L1-RSRP measurement requirements	799

9.5.4.1	SSB based L1-RSRP Reporting	799

9.5.4.2	CSI-RS based L1-RSRP Reporting	802

9.5.4A	Void	805

9.5.4A.1	Void	805

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	805

9.5.5.1	Measurement restriction for SSB based L1-RSRP	805

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	806

9.5.6	Scheduling availability of UE during L1-RSRP measurement	807

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	807

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	807

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	807

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	809

9.5A	L1-RSRP measurements for Reporting under CCA	809

9.5A.1	Introduction	809

9.5A.2	Requirements applicability	809

9.5A.3	Measurement Reporting Requirements	810

9.5A.3.1	Periodic Reporting	810

9.5A.3.2	Semi-Persistent Reporting	810

9.5A.3.3	Aperiodic Reporting	810

9.5A.4	L1-RSRP measurement requirements	811

9.5A.4.1	SSB based L1-RSRP Reporting	811

9.5A.5	Measurement restriction for L1-RSRP measurement	813

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	813

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	814

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	814

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	814

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	814

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	814

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	815

9.5B	L1-RSRP measurements for Reporting for RedCap	815

9.5B.1	Introduction	815

9.5B.2	Requirements applicability	815

9.5B.3	Measurement Reporting Requirements	816

9.5B.3.1	Periodic Reporting	816

9.5B.3.2	Semi-Persistent Reporting	816

9.5B.3.3	Aperiodic Reporting	817

9.5B.4	L1-RSRP measurement requirements	817

9.5B.4.1	SSB based L1-RSRP Reporting	817

9.5B.4.2	CSI-RS based L1-RSRP Reporting	819

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	822

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	822

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	822

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	822

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	823

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	823

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	823

9.5C	L1-RSRP measurements for Reporting for satellite access	824

9.5C.1	Introduction	824

9.5C.3	Measurement Reporting Requirements	824

9.5C.3.1	Periodic Reporting	825

9.5C.3.2	Semi-Persistent Reporting	825

9.5C.3.3	Aperiodic Reporting	825

9.5C.4	L1-RSRP measurement requirements	825

9.5C.4.1	SSB based L1-RSRP Reporting	825

9.5C.5	Measurement restriction for L1-RSRP measurement	827

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	827

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	827

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	828

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	828

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	828

9.5D	L1-RSRP measurements for Reporting for ATG	828

9.5D.1	Introduction	828

9.5D.2	Requirements applicability	828

9.5D.3	Measurement Reporting Requirements	829

9.5D.3.1	Periodic Reporting	829

9.5D.3.2	Semi-Persistent Reporting	829

9.5D.3.3	Aperiodic Reporting	829

9.5D.4	L1-RSRP measurement requirements	830

9.5D.4.1	SSB based L1-RSRP Reporting	830

9.5D.4.2	CSI-RS based L1-RSRP Reporting	831

9.6	NE-DC: Measurements	833

9.6.1	Introduction	833

9.6.2	SFTD Measurements	833

9.6.2.1	Introduction	833

9.6.2.2	SFTD Measurement requirements	833

9.7	Cross Link Interference measurements	834

9.7.1	Introduction	834

9.7.2	SRS-RSRP measurements	834

9.7.2.1	Introduction	834

9.7.2.2	Requirements applicability	834

9.7.2.3	Measurement Reporting Requirements	834

9.7.2.3.1	Periodic Reporting	834

9.7.2.3.2	Event-triggered Periodic Reporting	835

9.7.2.3.3	Event Triggered Reporting	835

9.7.2.4	Measurement capability	835

9.7.2.5	SRS-RSRP measurement period	835

9.7.3	CLI-RSSI measurements	835

9.7.3.1	Introduction	835

9.7.3.2	Requirements applicability	835

9.7.3.3	Measurement Reporting Requirements	836

9.7.3.3.1	Periodic Reporting	836

9.7.3.3.2	Event-triggered Periodic Reporting	836

9.7.3.3.3	Event Triggered Reporting	836

9.7.3.4	Measurement capability	836

9.7.3.5	CLI-RSSI measurement period	836

9.7.4	Scheduling availability of UE during CLI measurements	837

9.7.4.1	Scheduling availability of UE performing measurement on FR1	837

9.7.4.2	Scheduling availability of UE performing measurement on FR2	837

9.8	L1-SINR measurements for Reporting	838

9.8.1	Introduction	838

9.8.2	Requirements applicability	838

9.8.3	Measurement Reporting Requirements	839

9.8.3.1	Periodic Reporting	839

9.8.3.2	Semi-Persistent Reporting	839

9.8.4	L1-SINR measurement requirements	840

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	840

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	844

9.8.5	Measurement restriction for L1-SINR measurement	846

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	846

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	847

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	848

9.8.6	Scheduling availability of UE during L1-SINR measurement	848

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	848

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	848

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	849

9.8D	L1-SINR measurements for Reporting for ATG	850

9.8D.1	Introduction	850

9.8D.2	Requirements applicability	850

9.8D.3	Measurement Reporting Requirements	851

9.8D.3.1	Periodic Reporting	851

9.8D.3.2	Semi-Persistent Reporting	851

9.8D.3.3	Aperiodic Reporting	851

9.8D.4	L1-SINR measurement requirements	851

9.8D.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	851

9.8D.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	853

9.8D.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	854

9.9	NR measurements for positioning	855

9.9.1	Introduction	855

9.9.1.1	General Aspects of Gap-based Measurement	855

9.9.1.2	General Aspects of Gapless Measurement	856

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	857

9.9.2	RSTD measurements	858

9.9.2.1	Introduction	858

9.9.2.2	Requirements Applicability	858

9.9.2.3	Measurement Capability	858

9.9.2.4	Measurement Reporting Requirements	858

9.9.2.4.1	Void	858

9.9.2.4.2	Void	858

9.9.2.4.3	Void	858

9.9.2.5	Measurements Period Requirements	858

9.9.2.6	Void	861

9.9.2.7	Measurements Period Requirements without Measurement Gaps	861

9.9.2.8	Void	864

9.9.2.9	Measurements Period Requirements with both MG and PPW	864

9.9.3	PRS-RSRP measurements	865

9.9.3.1	Introduction	865

9.9.3.2	Requirements applicability	865

9.9.3.3	Measurement Capability	865

9.9.3.4	Measurement Reporting Requirements	865

9.9.3.5	Measurement Period Requirements	865

9.9.3.6	Measurement Period Requirements without Measurement Gaps	868

9.9.3.7	Void	870

9.9.3.8	Measurements Period Requirements with both MG and PPW	870

9.9.4	UE Rx-Tx time difference measurements	871

9.9.4.1 Introduction	871

9.9.4.2 Requirements Applicability	871

9.9.4.3 Measurement Capability	871

9.9.4.4 Measurement Reporting Requirements	871

9.9.4.5	Measurement Period Requirements	872

9.9.4.6	Measurement Period Requirements without Measurement Gaps	875

9.9.4.7	Void	878

9.9.4.8	Measurements Period Requirements with both MG and PPW	879

9.9.5	E-CID measurements	879

9.9.5.1	Introduction	879

9.9.5.2	Measurement Requirements	879

9.9.5.2.1	Intra-frequency Measurement Requirements	879

9.9.5.2.2	Inter-frequency Measurement Requirements	880

9.9.5.2.3	Measurement Reporting Delay	880

9.9.6	PRS-RSRPP measurements	880

9.9.6.1	Introduction	880

9.9.6.2	Requirements applicability	880

9.9.6.3	Measurement capability	881

9.9.6.4	Measurement reporting requirements	881

9.9.6.5	Measurement period requirements	881

9.9.6.6	Measurement Period Requirements without Measurement Gaps	881

9.9.6.7	Void	881

9.9.6.8	Measurements Period Requirements with both MG and PPW	881

9.10	CSI-RS based L3 measurements	881

9.10.1	Introduction	881

9.10.2	CSI-RS based intra-frequency measurements	882

9.10.2.1	Introduction	882

9.10.2.2	Requirements applicability	882

9.10.2.3	Number of cells and number of CSI-RS	883

9.10.2.3.1	Requirements for FR1	883

9.10.2.3.2	Requirements for FR2	883

9.10.2.4	Measurement Reporting Requirements	884

9.10.2.4.1	Periodic Reporting	884

9.10.2.4.2	Event-triggered Periodic Reporting	884

9.10.2.4.3	Event Triggered Reporting	884

9.10.2.5	Intra-frequency measurements without measurement gaps	884

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	886

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	886

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	886

9.10.3	CSI-RS based Inter-frequency measurements	887

9.10.3.1	Introduction	887

9.10.3.2	Requirements applicability	887

9.10.3.3	Number of cells and number of CSI-RS resources	888

9.10.3.3.1	Requirements for FR1	888

9.10.3.3.2	Requirements for FR2	888

9.10.3.4	Measurements reporting requirements	888

9.10.3.4.1	Periodic Reporting	888

9.10.3.4.2	Event-triggered Periodic Reporting	888

9.10.3.4.3	Event-triggered Reporting	888

9.10.3.5	Inter frequency measurements with measurement gaps	889

9.10D	CSI-RS based L3 measurements for ATG	890

9.10D.1	Introduction	890

9.10D.2	CSI-RS based intra-frequency measurements	890

9.10D.2.1	Introduction	890

9.10D.2.2	Requirements applicability	891

9.10D.2.3	Number of cells and number of CSI-RS	892

9.10D.2.3.1	Requirements for FR1	892

9.10D.2.4	Measurement Reporting Requirements	892

9.10D.2.4.1	Periodic Reporting	892

9.10D.2.4.2	Event-triggered Periodic Reporting	892

9.10D.2.4.3	Event Triggered Reporting	892

9.10D.2.5	Intra-frequency measurements without measurement gaps	893

9.10D.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	894

9.10D.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	894

9.10D.3	CSI-RS based Inter-frequency measurements	894

9.10D.3.1	Introduction	894

9.10D.3.2	Requirements applicability	894

9.10D.3.3	Number of cells and number of CSI-RS resources	895

9.10D.3.3.1	Requirements for FR1	895

9.10D.3.4	Measurements reporting requirements	895

9.10D.3.4.1	Periodic Reporting	895

9.10D.3.4.2	Event-triggered Periodic Reporting	895

9.10D.3.4.3	Event-triggered Reporting	896

9.10D.3.5	Inter frequency measurements with measurement gaps	896

9.11	NR measurements with autonomous gaps	897

9.11.1	Introduction	897

9.11.2	CGI identification of an NR cell with autonomous gaps	897

9.11.3	CGI reporting delay	898

9.11A	NR measurements with autonomous gaps for RedCap	898

9.11A.1	Introduction	898

9.11A.2	CGI identification of an NR cell with autonomous gaps	899

9.11A.3	CGI reporting delay	900

9.11A.4	CGI reporting scheduling restriction	900

9.11D	NR measurements with autonomous gaps for ATG	900

9.12	Measurement for Propagation Delay Compensation	900

9.12.1	Introduction	900

9.12.2	Requirements Applicability	901

9.12.3	Measurement Capability	901

9.12.4	Measurement period requirements	901

9.12.4.1	PRS Measurement Period	901

9.12.4.2	TRS Measurement Period	902

9.12.5	Measurement Reporting Requirements	903

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	903

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	903

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	904

9.13.1	Introduction	904

9.13.2	Requirements Applicability	904

9.13.3	Measurement Reporting Requirements	905

9.13.3.1	Periodic Reporting	905

9.13.3.2	Semi-Persistent Reporting	905

9.13.3.3	Aperiodic Reporting	905

9.13.4	L1-RSRP measurement requirements	905

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	905

9.13.5	Measurement restriction for L1-RSRP measurement	908

9.13.5.1	Measurement restriction for SSB based L1-RSRP	908

9.13.6	Scheduling availability of UE during L1-RSRP measurement	909

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	909

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	909

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	909

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	910

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	910

10	Measurement Performance requirements	911

10.1	NR measurements	911

10.1.1	Introduction	911

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	911

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	911

10.1.2.1.1	Absolute SS-RSRP Accuracy	911

10.1.2.1.2	Relative SS-RSRP Accuracy	912

10.1.2.2	Void	913

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	913

10.1.2.3.1	Absolute CSI-RSRP Accuracy	913

10.1.2.3.2	Relative CSI-RSRP Accuracy	914

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	915

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	915

10.1.2B.1.1	Absolute SS-RSRP Accuracy	915

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	916

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	916

10.1.2C.1.1	Absolute SS-RSRP Accuracy	916

10.1.2C.1.2	Relative SS-RSRP Accuracy	917

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	917

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	917

10.1.3.1.1	Absolute SS-RSRP Accuracy	917

10.1.3.1.2	Relative SS-RSRP Accuracy	918

10.1.3.2	Void	919

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	919

10.1.3.3.1	Absolute CSI-RSRP Accuracy	919

10.1.3.3.2	Relative CSI-RSRP Accuracy	920

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	921

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	921

10.1.3B.1.1	Absolute SS-RSRP Accuracy	921

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	922

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	922

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	922

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	923

10.1.4.2	Void	924

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	924

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	924

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	925

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	926

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	926

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	926

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	927

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	927

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	927

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	928

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	928

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	928

10.1.5.1.1	Absolute SS-RSRP Accuracy	928

10.1.5.1.2	Relative SS-RSRP Accuracy	929

10.1.5.2	Void	930

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	930

10.1.5.3.1	Absolute CSI-RSRP Accuracy	930

10.1.5.3.2	Relative CSI-RSRP Accuracy	931

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	932

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	932

10.1.5B.1.1	Absolute SS-RSRP Accuracy	932

10.1.6	RSRP Measurement Report Mapping	933

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	935

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	935

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	935

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	936

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	936

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	937

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	937

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	937

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	938

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	938

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	938

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	939

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	939

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	939

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	940

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	940

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	940

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	940

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	941

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	941

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	941

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	941

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	942

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	943

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	943

10.1.9.2.2	Relative CSI-RSRQ Accuracy	944

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	945

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	945

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	945

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	946

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	946

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	946

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	947

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	948

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	948

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	948

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	948

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	949

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	949

10.1.10.2.2	Relative CSI-RSRQ Accuracy	950

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	951

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	951

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	951

10.1.11	RSRQ report mapping	952

10.1.12	Intra-frequency SINR accuracy requirements for FR1	952

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	952

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	952

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	953

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	953

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	954

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	954

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	954

10.1.13	Intra-frequency SINR accuracy requirements for FR2	955

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	955

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	955

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	956

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	956

10.1.14	Inter-frequency SINR accuracy requirements for FR1	956

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	956

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	956

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	957

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	958

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	958

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	959

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	960

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	960

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	960

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	961

10.1.15	Inter-frequency SINR accuracy requirements for FR2	962

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	962

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	962

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	962

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	963

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	963

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	964

10.1.16	SINR report mapping	965

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	965

10.1.17	Power Headroom	966

10.1.17.1	Power Headroom Report	966

10.1.17.1.1	Power Headroom Report Mapping	966

10.1.18	PCMAX,c,f	967

10.1.18.1	Report Mapping	967

10.1.19	L1-RSRP accuracy requirements for FR1	967

10.1.19.1	SSB based L1-RSRP accuracy requirements	967

10.1.19.1.1	Absolute Accuracy	967

10.1.19.1.2	Relative Accuracy	968

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	969

10.1.19.2.1	Absolute Accuracy	969

10.1.19.2.2	Relative Accuracy	970

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	971

10.1.19C.1	SSB based L1-RSRP accuracy requirements	971

10.1.19C.1.1	Absolute Accuracy	971

10.1.19C.1.2	Relative Accuracy	972

10.1.20	L1-RSRP accuracy requirements for FR2	972

10.1.20.1	SSB based L1-RSRP accuracy requirements	972

10.1.20.1.1	Absolute Accuracy	972

10.1.20.1.2	Relative Accuracy	973

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	974

10.1.20.2.1	Absolute Accuracy	974

10.1.20.2.2	Relative Accuracy	975

10.1.21	SFTD accuracy requirements	976

10.1.21.1	SFTD acuracy requirements for NE-DC	976

10.1.21.2	SFTD acuracy requirements for NR-DC	978

10.1.21.3	Inter frequency SFTD acuracy requirements	979

10.1.22	CLI measurement accuracy requirements	980

10.1.22.1	SRS-RSRP	980

10.1.22.1.1	SRS-RSRP Accuracy	980

10.1.22.1.2	SRS-RSRP report mapping	982

10.1.22.2	CLI-RSSI	982

10.1.22.2.1	CLI-RSSI Accuracy	982

10.1.22.2.2	CLI-RSSI report mapping	983

10.1.23	RSTD Measurements	984

10.1.23.1	Introduction	984

10.1.23.2	Measurement Accuracy Requirements	984

10.1.23.3	Report mapping	994

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	994

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	996

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	998

10.1.24	PRS-RSRP Measurements	1000

10.1.24.1	Introduction	1000

10.1.24.2	Measurement Accuracy Requirements	1000

10.1.24.2.1	Absolute PRS RSRP accuracy	1000

10.1.24.2.2	Relative PRS RSRP accuracy	1004

10.1.24.3	Report mapping	1008

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	1008

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	1009

10.1.25	UE Rx-Tx Time Difference Measurements	1011

10.1.25.1	Introduction	1011

10.1.25.2	Measurement Accuracy Requirements	1011

10.1.25.3	Report mapping	1028

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1028

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	1030

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1031

10.1.26	FR2 P-MPR report	1033

10.1.26.1	Report mapping	1033

10.1.27	L1-SINR accuracy requirements for FR1	1033

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1033

10.1.27.1.1	Absolute Accuracy	1033

10.1.27.1.2	Relative Accuracy	1034

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1035

10.1.27.2.1	Absolute Accuracy	1035

10.1.27.2.2	Relative Accuracy	1036

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1038

10.1.27.3.1	Absolute Accuracy	1038

10.1.27.3.2	Relative Accuracy	1040

10.1.28	L1-SINR accuracy requirements for FR2	1041

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1048

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1048

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1048

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1048

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1048

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	1048

10.1.30.1.2	Relative Accuracy of SS-RSRQ	1049

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1050

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1050

10.1.31.1.1	Absolute SS-SINR Accuracy	1050

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1050

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1050

10.1.32.1.1	Aboslute Accuracy of SS-SINR	1050

10.1.32.1.2	Relative Accuracy of SS-SINR	1051

10.1.33	L1-RSRP accuracy requirements under CCA	1052

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1052

10.1.33.1.1	Absolute Accuracy	1052

10.1.33.1.2	Relative Accuracy	1052

10.1.34	RSSI measurements under CCA	1053

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1053

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1053

10.1.34.3	RSSI measurement report mapping	1053

10.1.35	Channel occupancy measurements under CCA	1054

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1054

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1054

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1054

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1054

10.1.36.1.1	Absolute SS-RSRP Accuracy	1054

10.1.36.1.2	Relative SS-RSRP Accuracy	1055

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1056

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1056

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1056

10.1.37.1.2	Relative Accuracy of SS-RSRP	1056

10.1.38	PRS-RSRPP Measurements	1057

10.1.38.1	Introduction	1057

10.1.38.2	Measurement Accuracy Requirements	1057

10.1.38.2.1	Absolute PRS RSRPP accuracy	1057

10.1.38.3	Report mapping	1061

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1061

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1062

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1063

10.1.39.1	 Introduction	1063

10.1.39.2	 Measurement Accuracy Requirements for PRS	1063

10.1.39.3	 Measurement Accuracy Requirements for TRS	1067

10.1.40	Measurement Accuracy Requirements for Propagation Delay Compensation	1070

10.1.40.1	Measurement Accuracy Requirements for PRS	1070

10.1.40.2	Measurement Accuracy Requirements for TRS	1073

10.1A	NR measurements for RedCap	1076

10.1A.1	Introduction	1076

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1076

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1076

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1076

10.1A.2.1.2	Relative SS-RSRP Accuracy	1077

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1078

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1078

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1078

10.1A.3.1.2	Relative SS-RSRP Accuracy	1078

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1078

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1078

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1078

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1079

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1080

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1080

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1080

10.1A.5.1.2	Relative SS-RSRP Accuracy	1080

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1080

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1080

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1080

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1081

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1081

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1081

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1081

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1081

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1081

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1082

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1083

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1083

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1083

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1083

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1083

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1083

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1083

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1084

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1084

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1084

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1084

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1084

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1084

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1085

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1086

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1086

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1086

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1086

10.1A.14	 L1-RSRP accuracy requirements for FR1	1086

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1086

10.1A.14.1.1	Absolute Accuracy	1086

10.1A.14.1.2	Relative Accuracy	1087

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1088

10.1A.14.2.1	Absolute Accuracy	1088

10.1A.14.2.2	Relative Accuracy	1089

10.1A.15	 L1-RSRP accuracy requirements for FR2	1090

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1090

10.1A.15.1.1	Absolute Accuracy	1090

10.1A.15.1.2	Relative Accuracy	1090

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1090

10.1A.15.2.1	Absolute Accuracy	1090

10.1A.15.2.2	Relative Accuracy	1090

10.2	E-UTRAN measurements	1091

10.2.1	Introduction	1091

10.2.2	E-UTRAN RSRP measurements	1091

10.2.3	E-UTRAN RSRQ measurements	1091

10.2.4	E-UTRAN RSTD measurements	1091

10.2.5	E-UTRAN RS-SINR measurements	1092

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1092

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1092

10.2A	E-UTRAN measurements for RedCap	1093

10.2A.1	Introduction	1093

10.2A.2	E-UTRAN RSRP measurements	1093

10.2A.3	E-UTRAN RSRQ measurements	1093

10.2A.4	E-UTRAN RS-SINR measurements	1094

10.3	UTRAN FDD Measurements	1094

10.3.1	UTRAN FDD CPICH RSCP	1094

10.3.2	UTRAN FDD CPICH Ec/No	1095

10.4	V2X measurements	1095

10.4.1	Introduction	1095

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1095

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1095

10.4.2.2	PSBCH-RSRP Relative Accuracy	1096

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1097

10.4.3.1	Absolute SL-RSSI Accuracy	1097

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1097

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1097

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1098

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1098

11	Void	1099

12	V2X Requirements	1100

12.1	Introduction	1100

12.2	UE Transmit Timing	1100

12.2.1	Introduction	1100

12.2.2	GNSS as synchronization reference source	1100

12.2.3	NR Cell as synchronization reference source	1101

12.2.4	E-URTAN Cell as synchronization reference source	1101

12.2.5	SyncRef UE as synchronization reference source	1101

12.3	Initiation/Cease of SLSS Transmissions	1102

12.3.1	Introduction	1102

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1102

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1103

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1104

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1104

12.4	Selection / Reselection of V2X Synchronization Reference Source	1105

12.5	L1 SL-RSRP measurements	1106

12.5.1	Introduction	1106

12.5.2	SL-RSRP measurements	1106

12.6	Congestion Control measurements	1107

12.7	Interruption	1107

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1107

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1108

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1109

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1110

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1110

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1111

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1111

12.7.8	Interruptions at NR sidelink discovery configuration	1111

12.8	Reliability of GNSS signal	1111

12.9	Scheduling availability	1112

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1112

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1112

12.10	Selection / Reselection of relay UE	1113

12.10.1	Introduction	1113

12.10.2	Selection / Reselection of relay UE	1113

13	Measurement Performance Requirements for NR gNB	1113

13.1	UL-RTOA	1113

13.1.1	Report mapping	1113

13.1.1A	Additional Path Report Mapping for UL-RTOA	1115

13.2	gNB Rx-Tx time difference	1117

13.2.1	Report mapping	1117

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	1119

13.2.2	Measurement Accuracy Requirements	1121

13.2.2.1	Introduction	1121

13.3	UL SRS RSRP measurement	1122

13.3.1	Report mapping	1122

13.3.2	Measurement accuracy requirements	1123

13.3.2.1	Introduction	1123

13.3.2.2	Requirements	1123

13.4	AoA/ZoA	1124

13.4.1	Report mapping	1124

13.5	Timing advance (TADV)	1125

13.5.1	Report mapping	1125

13.6	UL SRS RSRPP measurement	1126

13.6.1	Report mapping	1126

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1128

13.7.1	Report mapping	1128

13.7.2	Measurement Accuracy Requirements	1128

13.7.2.1	Introduction	1128

13.7.2.2	Requirements	1128

Annex A (normative): Test Cases	1130

A.1	Purpose of annex	1130

A.2	Requirement classification for statistical testing	1130

A.2.1	Types of requirements in TS 38.133	1130

A.2.1.1	Time and delay requirements on UE higher layer actions	1130

A.2.1.2	Measurements of power levels, relative powers and time	1130

A.2.1.3	Implementation requirements	1131

A.2.1.4	Physical layer timing requirements	1131

A.2.1.5	Requirements under CCA	1131

A.3	RRM test configurations	1132

A.3.1	Reference measurement channels	1132

A.3.1.1	PDSCH	1132

A.3.1.1.1	FDD	1132

A.3.1.1.2	TDD	1133

A.3.1.2	CORESET for RMSI scheduling	1136

A.3.1.2.1	FDD	1136

A.3.1.2.2	TDD	1137

A.3.1.3	CORESET for RMC scheduling	1140

A.3.1.3.1	FDD	1140

A.3.1.3.2	TDD	1141

A.3.1.4	TDD UL/DL configuration	1144

A.3.1A	Reference measurement channels under CCA	1146

A.3.1A.1	PDSCH	1146

A.3.1A.1.1	TDD	1146

A.3.1A.2	CORESET for RMSI scheduling	1147

A.3.1A.2.1	TDD	1147

A.3.1A.3	CORESET for RMC scheduling	1148

A.3.1A.3.1	TDD	1148

A.3.1A.4	TDD UL/DL configuration	1149

A.3.1A.5	RMC burst transmission model	1149

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1149

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1150

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1150

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1151

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1151

A.3.2.2	Void	1152

A.3.3	Reference DRX configurations	1152

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1152

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1153

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1153

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1153

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1154

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1154

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1154

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1155

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1155

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1155

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1156

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1156

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1156

A.3.4	Test Cases with Different Channel Bandwidths	1157

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1157

A.3.4.1.1	Introduction	1157

A.3.4.1.2	Principle of testing	1157

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1157

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1157

A.3.5.1.1	Introduction	1157

A.3.5.1.2	Principle of Testing	1157

A.3.6	Antenna configurations	1157

A.3.6.1	Antenna configurations for FR1	1157

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1157

A.3.6.1.1.1	Introduction	1157

A.3.6.1.1.2	Principle of testing	1158

A.3.6.2	Antenna configurations for FR2	1161

A.3.6A	Antenna configurations with unlicensed bands	1161

A.3.6A.1	Antenna configurations for FR1	1161

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1161

A.3.6A.1.1.1	Introduction	1161

A.3.6A.1.1.2	Principle of testing	1161

A.3.7	EN-DC test setup	1163

A.3.7.1	Introduction	1163

A.3.7.2	E-UTRAN Serving Cell Parameters	1163

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1163

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1164

A.3.7A	NR FR1-FR2 test setup	1165

A.3.7B	EN-DC test setup with unlicensed bands	1166

A.3.7B.1	Introduction	1166

A.3.7B.2	E-UTRAN Serving Cell Parameters	1166

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1166

A.3.7C	LTE-FR1/FR2 test setup	1167

A.3.7D	NE-DC test setup	1167

A.3.7D.1	Introduction	1167

A.3.7D.2	E-UTRAN Serving Cell Parameters	1167

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1167

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1167

A.3.8	PRACH configurations	1167

A.3.8.1	Introduction	1167

A.3.8.2	PRACH configurations in FR1	1168

A.3.8.2.1	FR1 PRACH configuration 1	1168

A.3.8.2.2	FR1 PRACH configuration 2	1168

A.3.8.2.3	FR1 PRACH configuration 3	1169

A.3.8.2.4	FR1 PRACH configuration 4	1170

A.3.8.3	PRACH configurations in FR2	1171

A.3.8.3.1	FR2 PRACH configuration 1	1171

A.3.8.3.2	FR2 PRACH configuration 2	1172

A.3.8.3.3	FR2 PRACH configuration 3	1173

A.3.8.3.4	FR2 PRACH configuration 4	1174

A.3.8A	PRACH configurations under CCA	1175

A.3.8A.1	Introduction	1175

A.3.8A.2	PRACH configurations in FR1	1175

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1175

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1176

A.3.9	BWP configurations	1177

A.3.9.1	Introduction	1177

A.3.9.2	Downlink BWP configurations	1178

A.3.9.2.1	Initial BWP	1178

A.3.9.2.2	Dedicated BWP	1178

A.3.9.3	Uplink BWP configurations	1179

A.3.9.3.1	Initial BWP	1179

A.3.9.3.2	Dedicated BWP	1179

A.3.9A	BWP configurations for RedCap	1179

A.3.9A.1	Introduction	1179

A.3.9A.2	Downlink BWP configurations	1180

A.3.9A.2.1	Dedicated BWP	1180

A.3.9A.3	Uplink BWP configurations	1180

A.3.9A.3.2	Dedicated BWP	1180

A.3.10	SSB Configurations	1181

A.3.10.1	SSB Configurations for FR1	1181

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1181

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1183

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1183

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1184

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1184

A.3.10.2	SSB Configurations for FR2	1185

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1185

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1185

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1186

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1186

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1187

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1187

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1188

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1188

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1189

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1189

A.3.10A	SSB Configurations under CCA	1194

A.3.10A.1	SSB Configurations under CCA for FR1	1194

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1194

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1194

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1195

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1195

A.3.10B	SSB Configurations for RedCap	1196

A.3.10B.1	SSB Configurations for FR1	1196

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1196

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1196

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1197

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1197

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1198

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1198

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1199

A.3.10B.2	SSB Configurations for FR2	1199

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1199

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1200

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1200

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1201

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1201

A.3.11	SMTC Configurations	1202

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1202

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1202

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1202

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1202

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1203

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1203

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1203

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1203

A.3.11A	SMTC Configurations for RedCap	1204

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1204

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1204

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1204

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1204

A.3.12	Test Cases with Different CC Configurations	1204

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1204

A.3.12.1.1	Introduction	1204

A.3.12.1.2	Principle of testing	1205

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1205

A.3.12.2.1	Introduction	1205

A.3.12.2.2	Principle of testing	1205

A.3.13	Test Cases in SA and EN-DC Operations	1205

A.3.13.1	Introduction	1205

A.3.13.2	Principle of Testing	1205

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1206

A.3.13A.1	Introduction	1206

A.3.13A.2	Principle of Testing in EN-DC	1206

A.3.13A.3	Principle of Testing in SA	1206

A.3.13A.4	Principle of Testing in E-UTRA	1207

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1207

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1207

A.3.13B.1.1	Introduction	1207

A.3.13B.1.2	Principle of Testing	1207

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1207

A.3.13B.2.1	Introduction	1207

A.3.13B.2.2	Principle of Testing	1207

A.3.14	CSI-RS configurations	1208

A.3.14.1	FDD	1208

A.3.14.2	TDD	1210

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1216

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1216

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1216

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1216

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1217

A.3.15.3	Setup 3: 2 AoAs	1217

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1217

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1217

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1217

A.3.16	TCI State Configuration	1218

A.3.16.1	Introduction	1218

A.3.16.2	TCI states	1218

A.3.16A	Unified TCI State Configuration	1218

A.3.16A.1	Introduction	1218

A.3.16A.2	DLorJoint TCI states	1219

A.3.16A.3	UL TCI states	1219

A.3.17	Configurations of CSI-RS for tracking	1220

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1220

A.3.17.1.1	FDD	1220

A.3.17.1.2	TDD	1222

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1224

A.3.17.2.1	TDD	1224

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1225

A.3.18.1	Introduction	1225

A.3.18.2	PRACH Power Measurement	1225

A.3.19	Test applicability for DAPS handover	1225

A.3.19.1	Introduction	1225

A.3.19.2	Principle of testing	1225

A.3.20	MsgA configurations	1226

A.3.20.1	Introduction	1226

A.3.20.2	MsgA configurations in FR1	1226

A.3.20.2.1	FR1 MsgA configuration 1	1226

A.3.20.2.2	FR1 MsgA configuration 2	1227

A.3.20.3	MsgA configurations in FR2	1229

A.3.20.3.1	FR2 MsgA configuration 1	1229

A.3.20.3.2	FR2 MsgA configuration 2	1230

A.3.20A	MsgA configurations under CCA	1232

A.3.20A.1	Introduction	1232

A.3.20A.2	MsgA configurations in FR1	1232

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1232

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1233

A.3.21	V2X sidelink communication	1236

A.3.21.1	Introduction	1236

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1236

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1240

A.3.21.4	Reference SL-DRX configurations	1241

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1241

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1241

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1241

A.3.22	CSI-IM configurations	1241

A.3.22.1	FDD	1241

A.3.22.2	TDD	1242

A.3.23	Spatial Relation Configuration	1243

A.3.23.1	Introduction	1243

A.3.23.2	Spatial Relation	1244

A.3.24	SRS configuration	1245

A.3.25	Channel bandwidth (CBW) configurations	1247

A.3.25.1	DL UE specific CBW	1247

A.3.25.2	UL UE specific CBW	1248

A.3.26	CCA model	1248

A.3.26.1	Introduction	1248

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1248

A.3.26.2.1	DL CCA model	1248

A.3.26.2.2	UL CCA model	1250

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1250

A.3.26.3.1	DL CCA model	1250

A.3.26.3.2	UL CCA model	1251

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1251

A.3.27.1	Introduction	1251

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1251

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1252

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1252

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1252

A.3.28	Discovery Burst Transmission Window configuration under CCA	1252

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1252

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1252

A.3.29.1	Introduction	1252

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1252

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1253

A.3.30	CSI-RS configurations for RRM	1254

A.3.30.1	FDD	1254

A.3.30.2	TDD	1255

A.3.31	PRS Configurations	1257

A.3.31.1.	PRS Configurations for FR1	1257

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1257

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1258

A.3.31.2.	PRS Configurations for FR2	1258

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1258

A.3.32	NR sidelink discovery	1258

A.3.32.1	Introduction	1258

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1258

A.3.32.3	Principle of Testing	1259

A.3.33	PRS Processing Window (PPW) configurations	1259

A.3.34	Testing principles for test cases related to PRS measurements	1259

A.3.34.1	Introduction	1259

A.3.34.2	Test cases in RRC_INACTIVE state	1260

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1260

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1260

A.3.35	Testing principle for RedCap UE	1261

A.3.35.1	Introduction	1261

A.3.35.2	Principle of testing for FR1	1261

A.3.35.3	Principle of testing for FR2	1261

A.3.36	Testing related to Satellite access	1261

A.3.36.1	Introduction	1261

A.3.36.1	Introduction	1261

A.3.36.2	Principle of testing GSO and NGSO scenarios	1261

A.3.36.2	Principle of testing different RRM requirements	1262

A.3.36.3	Principle of testing different ephemeris formats	1262

A.3.36.4	General setup for SIB19	1264

A.3.36.5	Satellite specific parameters configuration	1265

A.3.36.5.1	Satellite specific configuration for serving cell	1265

A.3.36.5.2	Satellite specific configuration for neighbour cell	1265

A.4	EN-DC tests with all NR cells in FR1	1267

A.4.1	Void	1267

A.4.2	Void	1267

A.4.3	RRC_CONNECTED state mobility	1267

A.4.3.1	Void	1267

A.4.3.2	RRC Connection Mobility Control	1267

A.4.3.2.1	Void	1267

A.4.3.2.2	Random Access	1267

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1267

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1271

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1275

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1279

A.4.3.2.3	Void	1283

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1283

A.4.3.3.1	Test Purpose and Environment	1283

A.4.3.3.2	Test Requirements	1289

A.4.4	Timing	1290

A.4.4.1	UE transmit timing	1290

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1290

A.4.4.1.1.1	Test Purpose and environment	1290

A.4.4.1.1.2	Test requirements	1294

A.4.4.2	UE timer accuracy	1294

A.4.4.3	Timing advance	1294

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1294

A.4.4.3.1.1	Test Purpose and Environment	1294

A.4.4.3.1.2	Test Parameters	1294

A.4.4.3.1.3	Test Requirements	1298

A.4.5	Signaling characteristics	1298

A.4.5.1	Radio link Monitoring	1298

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1299

A.4.5.1.1.1	Test Purpose and Environment	1299

A.4.5.1.1.2	Test Requirements	1303

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1303

A.4.5.1.2.1	Test Purpose and Environment	1303

A.4.5.1.2.2	Test Requirements	1309

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1309

A.4.5.1.3.1	Test Purpose and Environment	1309

A.4.5.1.3.2	Test Requirements	1314

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1314

A.4.5.1.4.1	Test Purpose and Environment	1314

A.4.5.1.4.2	Test Requirements	1320

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1320

A.4.5.1.5.1	Test Purpose and Environment	1320

A.4.5.1.5.2	Test Requirements	1325

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1325

A.4.5.1.6.1	Test Purpose and Environment	1325

A.4.5.1.6.2	Test Requirements	1331

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1331

A.4.5.1.7.1	Test Purpose and Environment	1331

A.4.5.1.7.2	Test Requirements	1336

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1337

A.4.5.1.8.1	Test Purpose and Environment	1337

A.4.5.1.8.2	Test Requirements	1342

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1342

A.4.5.1.9.1	Test Purpose and Environment	1342

A.4.5.2	Interruption	1348

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1348

A.4.5.2.1.1	Test Purpose and Environment	1348

A.4.5.2.1.2	Test Requirements	1352

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1352

A.4.5.2.2.1	Test Purpose and Environment	1352

A.4.5.2.2.2	Test Requirements	1356

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1356

A.4.5.2.3.1	Test Purpose and Environment	1356

A.4.5.2.3.2	Test Requirements	1361

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1362

A.4.5.2.4.1	Test Purpose and Environment	1362

A.4.5.2.4.2	Test Requirements	1366

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1367

A.4.5.2.5.1	Test Purpose and Environment	1367

A.4.5.2.5.2	Test Requirements	1371

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1371

A.4.5.2.6.1	Test Purpose and Environment	1371

A.4.5.2.6.2	Test Requirements	1375

A.4.5.2.7	Void	1375

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1375

A.4.5.2.8.1	Test Purpose and Environment	1375

A.4.5.2.8.2	Test Requirements	1378

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1379

A.4.5.2.9.1	Test Purpose and Environment	1379

A.4.5.2.9.2	Test Requirements	1383

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1384

A.4.5.2.10.1	Test Purpose and Environment	1384

A.4.5.2.10.2	Test Requirements	1387

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1387

A.4.5.2.11.1	Test Purpose and Environment	1387

A.4.5.2.11.2	Test Requirements	1392

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1393

A.4.5.2.12.1	Test Purpose and Environment	1393

A.4.5.3	SCell Activation and Deactivation Delay	1401

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1401

A.4.5.3.1.1	Test Purpose and Environment	1401

A.4.5.3.1.2	Test Requirements	1407

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1408

A.4.5.3.2.1	Test Purpose and Environment	1408

A.4.5.3.2.2	Test Requirements	1408

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1408

A.4.5.3.3.1	Test Purpose and Environment	1408

A.4.5.3.3.2	Test Requirements	1409

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1410

A.4.5.3.4.1	Test Purpose and Environment	1410

A.4.5.3.3.2	Test Requirements	1412

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1412

A.4.5.3.5.1	Test Purpose and Environment	1412

A.4.5.3.5.2	Test Requirements	1420

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1421

A.4.5.3.6.1	Test Purpose and Environment	1421

A.4.5.3.6.2	Test Requirements	1425

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1426

A.4.5.3.7.1	Test Purpose and Environment	1426

A.4.5.3.7.2	Test Requirements	1426

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1427

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1427

A.4.5.4.1.1 Test Purpose and Environment	1427

A.4.5.4.1.2	Test Requirements	1437

A.4.5.5	Beam Failure Detection and Link recovery procedures	1437

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1437

A.4.5.5.1.1	Test Purpose and Environment	1437

A.4.5.5.1.2	Test Requirements	1443

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1444

A.4.5.5.2.1	Test Purpose and Environment	1444

A.4.5.5.2.2	Test Requirements	1449

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1450

A.4.5.5.3.1	Test Purpose and Environment	1450

A.4.5.5.3.2	Test Requirements	1455

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1456

A.4.5.5.4.1	Test Purpose and Environment	1456

A.4.5.5.4.2	Test Requirements	1461

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1462

A.4.5.5.5.1	Test Purpose and Environment	1462

A.4.5.5.5.2	Test Requirements	1467

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1468

A.4.5.5.6.1	Test Purpose and Environment	1468

A.4.5.5.6.2	Test Requirements	1474

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1475

A.4.5.5.7.1	Test Purpose and Environment	1475

A.4.5.5.7.2	Test Requirements	1480

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1481

A.4.5.5.8.1	Test Purpose and Environment	1481

A.4.5.5.8.2	Test Requirements	1487

A.4.5.6	Active BWP switch	1488

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1488

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1488

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1493

A.4.5.6.2	RRC-based Active BWP Switch	1499

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1503

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1503

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1509

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1509

A.4.5.6.4.1.1	Test Purpose and Environment	1509

A.4.5.6.4.1.2	Test Requirements	1516

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1516

A.4.5.6.4.2.1	Test Purpose and Environment	1516

A.4.5.6.4.2.2	Test Requirements	1526

A.4.5.6.5	SCell dormancy switch	1527

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1527

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1532

A.4.5.6.5.2.1	Test Purpose and Environment	1532

A.4.5.6.5.2.2	Test Requirements	1538

A.4.5.7	PSCell addition and release delay	1538

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1538

A.4.5.7.1.1	Test purpose and environment	1538

A.4.5.7.1.2	Test Requirements	1543

A.4.5.8	DL Interruptions at switching between two uplink carriers	1544

A.4.5.8.1	Test Purpose and Environment	1544

A.4.5.8.2	Test Requirements	1547

A.4.5.9	UE specific CBW change	1548

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1548

A.4.5.9.1.1	Test Purpose and Environment	1548

A.4.5.9.1.2	Test Requirements	1552

A.4.5.10	PSCell activation and deactivation delay	1552

A.4.5.10.1	PSCell activation and deactivation delay	1552

A.4.5.10.1.1	Test purpose and environment	1552

A.4.5.10.1.2	Test Requirements	1556

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1557

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1557

A.4.5.11.1.1	Test purpose and environment	1557

A.4.5.11.1.2	Test Parameters	1557

A.4.5.11.1.3	Test Requirements	1561

A.4.6	Measurement procedure	1561

A.4.6.1	Intra-frequency Measurements	1561

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1561

A.4.6.1.1.1	Test purpose and Environment	1561

A.4.6.1.1.2	Test parameters	1561

A.4.6.1.1.3	Test Requirements	1565

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1565

A.4.6.1.2.1	Test purpose and Environment	1565

A.4.6.1.2.2	Test parameters	1565

A.4.6.1.2.2	Test Requirements	1569

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1569

A.4.6.1.3.1	Test purpose and Environment	1569

A.4.6.1.3.2	Test parameters	1569

A.4.6.1.3.3	Test Requirements	1573

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1573

A.4.6.1.4.1	Test purpose and Environment	1573

A.4.6.1.4.2	Test parameters	1573

A.4.6.1.4.3	Test Requirements	1577

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1577

A.4.6.1.5.1	Test purpose and Environment	1577

A.4.6.1.5.2	Test parameters	1577

A.4.6.1.5.3	Test Requirements	1579

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1580

A.4.6.1.6.1	Test purpose and Environment	1580

A.4.6.1.6.2	Test parameters	1580

A.4.6.1.6.3	Test Requirements	1582

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1583

A.4.6.1.7.1	Test purpose and Environment	1583

A.4.6.1.7.2	Test parameters	1583

A.4.6.1.7.3	Test Requirements	1587

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1587

A.4.6.1.8.1	Test Purpose and Environment	1587

A.4.6.1.8.2	Test Requirements	1592

A.4.6.2	Inter-frequency Measurements	1593

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1593

A.4.6.2.1.1	Test Purpose and Environment	1593

A.4.6.2.1.2	Test Requirements	1597

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1597

A.4.6.2.2.1	Test Purpose and Environment	1597

A.4.6.2.2.2	Test Requirements	1601

A.4.6.2.3	Void	1601

A.4.6.2.4	Void	1601

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1601

A.4.6.2.5.1	Test Purpose and Environment	1601

A.4.6.2.5.2	Test Requirements	1606

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1606

A.4.6.2.6.1	Test Purpose and Environment	1606

A.4.6.2.6.2	Test Requirements	1611

A.4.6.2.7	Void	1611

A.4.6.2.8	Void	1611

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1612

A.4.6.2.9.1	Test Purpose and Environment	1612

A.4.6.2.9.2	Test Requirements	1615

A.4.6.3	Void	1616

A.4.6.4	L1-RSRP measurement for beam reporting	1616

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1616

A.4.6.4.1.1	Test Purpose and Environment	1616

A.4.6.4.1.2	Test parameters	1616

A.4.6.4.1.3	Test Requirements	1619

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1619

A.4.6.4.2.1	Test Purpose and Environment	1619

A.4.6.4.2.2	Test parameters	1620

A.4.6.4.2.3	Test Requirements	1623

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1623

A.4.6.4.3.1	Test Purpose and Environment	1623

A.4.6.4.3.2	Test parameters	1624

A.4.6.4.3.3	Test Requirements	1627

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1627

A.4.6.4.4.1	Test Purpose and Environment	1627

A.4.6.4.4.2	Test parameters	1628

A.4.6.4.4.3	Test Requirements	1630

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1630

A.4.6.4.5.1	Test Purpose and Environment	1630

A.4.6.4.5.2	Test parameters	1631

A.4.6.4.5.3	Test Requirements	1634

A.4.6.5	CLI measurements	1634

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1634

A.4.6.5.1.1	Test Purpose and Environment	1634

A.4.6.5.1.2	Test Parameters	1635

A.4.6.5.1.3	Test Requirements	1638

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1638

A.4.6.5.2.1	Test Purpose and Environment	1638

A.4.6.5.2.2	Test Parameters	1639

A.4.6.5.2.3	Test Requirements	1640

A.4.6.6.1.2	Test Requirements	1645

A.4.6.7	L1-SINR measurement for beam reporting	1645

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1648

A.4.6.7.2.1	Test Purpose and Environment	1648

A.4.6.7.2.2	Test parameters	1648

A.4.6.7.2.3	Test Requirements	1651

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1651

A.4.6.7.3.1	Test Purpose and Environment	1651

A.4.6.7.3.2	Test parameters	1651

A.4.6.7.3.3	Test Requirements	1653

A.4.6.8	CSI-RS based intra-frequency Measurement	1654

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1654

A.4.6.8.1.1	Test purpose and Environment	1654

A.4.6.8.1.2	Test Requirements	1658

A.4.6.9	CSI-RS based inter-frequency Measurement	1658

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1658

A.4.6.9.1.1	Test Purpose and Environment	1658

A.4.6.9.1.2	Test Requirements	1662

A.4.7	Measurement Performance requirements	1663

A.4.7.1	SS-RSRP	1663

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1663

A.4.7.1.1.1	Test Purpose and Environment	1663

A.4.7.1.1.2	Test parameters	1663

A.4.7.1.1.3	Test Requirements	1667

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1667

A.4.7.1.2.1	Test Purpose and Environment	1667

A.4.7.1.2.2	Test parameters	1668

A.4.7.1.2.3	Test Requirements	1672

A.4.7.1.3	Void	1672

A.4.7.2	SS-RSRQ	1672

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1672

A.4.7.2.1.1	Test Purpose and Environment	1672

A.4.7.2.1.2	Test Parameters	1673

A.4.7.2.1.3	Test Requirements	1678

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1678

A.4.7.2.2.1	Test Purpose and Environment	1678

A.4.7.2.2.2	Test Parameters	1678

A.4.7.2.2.3	Test Requirements	1683

A.4.7.3	SS-SINR	1683

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1683

A.4.7.3.1.1	Test Purpose and Environment	1683

A.4.7.3.1.2	Test Parameters	1683

A.4.7.3.1.3	Test Requirements	1687

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1687

A.4.7.3.2.1	Test Purpose and Environment	1687

A.4.7.3.2.2	Test Parameters	1687

A.4.7.3.2.3	Test Requirements	1692

A.4.7.4	L1-RSRP measurement for beam reporting	1692

A.4.7.4.1	SSB based L1-RSRP measurement	1692

A.4.7.4.1.1	Test Purpose and Environment	1692

A.4.7.4.1.2	Test parameters	1693

A.4.7.4.1.3	Test Requirements	1696

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1696

A.4.7.4.2.1	Test Purpose and Environment	1696

A.4.7.4.2.2	Test parameters	1697

A.4.7.4.2.3	Test Requirements	1700

A.4.7.5	SFTD accuracy	1700

A.4.7.5.1	SFTD accuracy	1700

A.4.7.5.1.1	Test Purpose and Environment	1700

A.4.7.5.1.2	Test Parameters	1700

A.4.7.5.1.3	Test Requirements	1705

A.4.7.5.2	Void	1705

A.4.7.5.3	Void	1705

A.4.7.6	CLI measurements	1705

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1705

A.4.7.6.1.1	Test Purpose and Environment	1705

A.4.7.6.1.2	Test parameters	1706

A.4.7.6.1.3	Test Requirements	1711

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1711

A.4.7.6.2.1	Test Purpose and Environment	1711

A.4.7.6.2.2	Test parameters	1712

A.4.7.6.2.3	Test Requirements	1715

A.4.7.7	L1-SINR measurement for beam reporting	1715

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1719

A.4.7.7.2.1	Test Purpose and Environment	1719

A.4.7.7.2.2	Test parameters	1719

A.4.7.7.2.3	Test Requirements	1723

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1723

A.4.7.7.3.1	Test Purpose and Environment	1723

A.4.7.7.3.2	Test parameters	1723

A.4.7.7.3.3	Test Requirements	1726

A.4.7.8	CSI-RSRP	1726

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1726

A.4.7.8.1.1	Test Purpose and Environment	1726

A.4.7.8.1.2	Test parameters	1727

A.4.7.8.1.3	Test Requirements	1732

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1732

A.4.7.8.2.1	Test Purpose and Environment	1732

A.4.7.8.2.2	Test parameters	1732

A.4.7.8.2.3	Test Requirements	1737

A.4.7.9	CSI-RSRQ	1737

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1737

A.4.7.9.1.1	Test Purpose and Environment	1737

A.4.7.9.1.2	Test Parameters	1737

A.4.7.9.1.3	Test Requirements	1742

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1742

A.4.7.9.2.1	Test Purpose and Environment	1742

A.4.7.9.2.2	Test Parameters	1742

A.4.7.9.2.3	Test Requirements	1747

A.4.7.10	CSI-SINR	1747

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1747

A.4.7.10.1.1	Test Purpose and Environment	1747

A.4.7.10.1.2	Test Parameters	1748

A.4.7.10.1.3	Test Requirements	1752

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1752

A.4.7.10.2.1	Test Purpose and Environment	1752

A.4.7.10.2.2	Test Parameters	1753

A.4.7.10.2.3	Test Requirements	1757

A.4.8	Void	1757

A.4A	NE-DC test with all NR cells in FR1	1758

A.4A.1	Signaling characteristics	1758

A.4A.1.1	E-UTRAN PSCell addition	1758

A.4A.1.1.1	Test purpose and environment	1758

A.4A.1.1.2	Test Requirements	1762

A.4A.1.2	Active BWP switch	1763

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1763

A.4A.1.2.1.1	Test Purpose and Environment	1763

A.4A.1.2.1.2	Test Requirements	1767

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1768

A.4A.1.3.1	Test purpose and environment	1768

A.4A.1.3.2	Test Requirements	1772

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1773

A.4A.1.4.1	Test Purpose and Environment	1773

A.4A.1.4.2	Test Parameters	1773

A.4A.1.4.3	Test Requirements	1778

A.4A.1.4.3.1	Test Requirements for NR HO	1778

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1778

A.4A.2	Measurement performance	1779

A.4A.2.1	SFTD accuracy	1779

A.4A.2.1.1	SFTD accuracy	1779

A.4A.2.1.1.1	Test Purpose	1779

A.4A.2.1.1.2	Test Environment	1779

A.4A.2.1.1.3	Test Requirements	1782

A.5	EN-DC tests with one or more NR cells in FR2	1783

A.5.1	Void	1783

A.5.2	Void	1783

A.5.3	RRC_CONNECTED state mobility	1783

A.5.3.1	Void	1783

A.5.3.2	RRC Connection Mobility Control	1783

A.5.3.2.1	Void	1783

A.5.3.2.2	Random Access	1783

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1783

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1786

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1792

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1795

A.5.3.2.3	Void	1799

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1799

A.5.3.3.1	Test purpose and environment	1799

A.5.3.3.2	Test Requirements	1803

A.5.5.3.3	PUCCH SCell Activation and deactivation of known SCell in FR2	1804

A.5.5.3.3.1	Test Purpose and Environment	1804

A.5.5.3.3.2	Test Requirements	1807

A.5.5.3.4	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1808

A.5.5.3.4.1	Test Purpose and Environment	1808

A.5.5.3.4.2	Test Requirements	1811

A.5.5.3.5	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1812

A.5.5.3.5.1	Test Purpose and Environment	1812

A.5.5.3.5.2	Test Requirements	1815

A.5.5.3.6	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1816

A.5.5.3.6.1	Test Purpose and Environment	1816

A.5.5.3.6.2	Test Requirements	1819

A.5.4	Timing	1820

A.5.4.1	UE transmit timing	1820

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1820

A.5.4.1.1.1	Test Purpose and environment	1820

A.5.4.1.1.2	Test requirements	1823

A.5.4.2	UE timer accuracy	1824

A.5.4.3	Timing advance	1824

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1824

A.5.4.3.1.1 Test Purpose and Environment	1824

A.5.4.3.1.2 Test Parameters	1824

A.5.4.3.1.3	Test Requirements	1828

A.5.5	Signaling characteristics	1828

A.5.5.1	Radio link Monitoring	1828

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1828

A.5.5.1.1.1	Test Purpose and Environment	1828

A.5.5.1.1.2	Test Requirements	1832

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1833

A.5.5.1.2.1	Test Purpose and Environment	1833

A.5.5.1.2.2	Test Requirements	1836

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1837

A.5.5.1.3.1	Test Purpose and Environment	1837

A.5.5.1.3.2	Test Requirements	1841

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1841

A.5.5.1.4.1	Test Purpose and Environment	1841

A.5.5.1.4.2	Test Requirements	1845

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1845

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1849

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1853

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1858

A.5.5.1.8.2	Test Requirements	1862

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1863

A.5.5.1.9.1	Test Purpose and Environment	1863

A.5.5.1.9.2	Test Requirements	1865

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1865

A.5.5.1.10.1	Test Purpose and Environment	1865

A.5.5.1.10.2	Test Requirements	1868

A.5.5.2	Interruption	1868

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1868

A.5.5.2.1.1	Test Purpose and Environment	1868

A.5.5.2.1.2	Test Requirements	1871

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1871

A.5.5.2.2.1	Test Purpose and Environment	1871

A.5.5.2.2.2	Test Requirements	1874

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1874

A.5.5.2.3.1	Test Purpose and Environment	1874

A.5.5.2.3.2	Test Requirements	1878

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1878

A.5.5.2.4.1	Test Purpose and Environment	1878

A.5.5.2.4.2	Test Requirements	1881

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1882

A.5.5.2.5.1	Test Purpose and Environment	1882

A.5.5.2.5.2	Test Requirements	1885

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1885

A.5.5.2.6.1	Test Purpose and Environment	1885

A.5.5.2.6.2	Test Requirements	1888

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1888

A.5.5.2.7.1	Test Purpose and Environment	1888

A.5.5.2.7.2	Test Requirements	1892

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1892

A.5.5.2.8.1 Test Purpose and Environment	1892

A.5.5.2.8.3	Test Requirements	1894

A.5.5.3	SCell Activation and Deactivation Delay	1894

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1894

A.5.5.3.1.1	Test Purpose and Environment	1894

A.5.5.3.1.2	Test Requirements	1896

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1896

A.5.5.3.2.1	Test Purpose and Environment	1896

A.5.5.3.2.2	Test Requirements	1900

A.5.5.3.3	Void	1900

A.5.5.3.4	Void	1900

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1900

A.5.5.3.5.1	Test Purpose and Environment	1900

A.5.5.3.5.2	Test Requirements	1903

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1904

A.5.5.3.6.1	Test Purpose and Environment	1904

A.5.5.3.6.2	Test Requirements	1908

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1909

A.5.5.3.7.1	Test Purpose and Environment	1909

A.5.5.3.7.2	Test Requirements	1912

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1912

A.5.5.3.8.1	Test Purpose and Environment	1912

A.5.5.3.8.2	Test Requirements	1916

A.5.5.4	Void	1917

A.5.5.5	Beam Failure Detection and Link recovery procedures	1917

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1917

A.5.5.5.1.1	Test Purpose and Environment	1917

A.5.5.5.1.2	Test Requirements	1921

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1922

A.5.5.5.2.1	Test Purpose and Environment	1922

A.5.5.5.2.2	Test Requirements	1926

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1927

A.5.5.5.3.1	Test Purpose and Environment	1927

A.5.5.5.3.2	Test Requirements	1931

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1932

A.5.5.5.4.1	Test Purpose and Environment	1932

A.5.5.5.4.2	Test Requirements	1936

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1937

A.5.5.5.5.1	Test Purpose and Environment	1937

A.5.5.5.5.2	Test Requirements	1941

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1942

A.5.5.5.6.1	Test Purpose and Environment	1942

A.5.5.5.6.2	Test Requirements	1946

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1947

A.5.5.5.7.1	Test Purpose and Environment	1947

A.5.5.5.7.2	Test Requirements	1951

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1952

A.5.5.5.8.1	Test Purpose and Environment	1952

A.5.5.5.8.2	Test Requirements	1957

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1957

A.5.5.5.9.1	Test Purpose and Environment	1957

A.5.5.5.9.2	Test Requirements	1961

A.5.5.6	Active BWP switch	1961

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1961

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1961

A.5.5.6.1.1.1	Test Purpose and Environment	1961

A.5.5.6.1.1.2	Test Requirements	1965

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1965

A.5.5.6.2	RRC-based Active BWP Switch	1970

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1970

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1974

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1974

A.5.5.6.4	SCell dormancy switch	1978

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1978

A.5.5.6.4.1.1	Test Purpose and Environment	1978

A.5.5.6.4.1.2	Test Requirements	1982

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1982

A.5.5.6.4.2.1	Test Purpose and Environment	1982

A.5.5.6.4.2.2	Test Requirements	1989

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1989

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1989

A.5.5.7	PSCell addition and release delay	1992

A.5.5.7.1	Addition and Release Delay of NR PSCell	1992

A.5.5.7.1.1	Test purpose and environment	1992

A.5.5.7.1.2	Test Requirements	1995

A.5.5.8	Active TCI state switch delay	1996

A.5.5.8.1	MAC-CE based active TCI state switch	1996

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1996

A.5.5.8.1.1.1	Test Purpose and Environment	1996

A.5.5.8.1.1.2	Test Requirements	1999

A.5.5.8.2	RRC based active TCI state switch	1999

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1999

A.5.5.8.2.1.1	Test Purpose and Environment	1999

A.5.5.8.2.1.2	Test Requirements	2003

A.5.5.9	Uplink spatial relation switch delay	2003

A.5.5.9.1	MAC-CE based uplink spatial relation switch	2003

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	2003

A.5.5.9.1.1.1	Test Purpose and Environment	2003

A.5.5.9.1.1.2	Test Requirements	2006

A.5.5.9.2	RRC based spatial relation switch	2006

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	2006

A.5.5.9.2.1.1	Test Purpose and Environment	2006

A.5.5.9.2.1.2	Test Requirements	2009

A.5.5.10	UE specific CBW change	2009

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	2009

A.5.5.10.1.1	Test Purpose and Environment	2009

A.5.5.10.1.2	Test Requirements	2012

A.5.5.11	Unified TCI state switch delay	2013

A.5.5.11.1	MAC-CE based active joint TCI state switch	2013

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	2013

A.5.5.11.1.1.1	Test Purpose and Environment	2013

A.5.5.11.1.1.2	Test parameters	2013

A.5.5.11.1.1.3	Test Requirements	2016

A.5.5.11.2	MAC-CE based active uplink TCI state switch	2016

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	2016

A.5.5.11.2.1.1	Test Purpose and Environment	2016

A.5.5.11.2.1.2	Test parameters	2017

A.5.5.11.2.1.3	Test Requirements	2019

A.5.5.11.3	MAC-CE based active downlink TCI state switch	2019

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	2019

A.5.5.11.3.1.1	Test Purpose and Environment	2019

A.5.5.11.3.1.2	Test Parameters	2020

A.5.5.11.3.1.3	Test Requirements	2024

A.5.5.12	PSCell activation and deactivation delay	2024

A.5.5.12.1	PSCell activation and deactivation delay	2024

A.5.5.12.1.1	Test purpose and environment	2024

A.5.5.12.1.2	Test Requirements	2027

A.5.5.13	Conditional PSCell addition and release delay	2028

A.5.5.13.1	Addition and Release Delay of NR PSCell	2028

A.5.5.13.1.1	Test purpose and environment	2028

A.5.5.13.1.2	Test Requirements	2031

A.5.6	Measurement procedure	2032

A.5.6.1	Intra-frequency Measurements	2032

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	2032

A.5.6.1.1.1	Test purpose and Environment	2032

A.5.6.1.1.2	Test Requirements	2035

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	2035

A.5.6.1.2.1	Test purpose and Environment	2035

A.5.6.1.2.2	Test Requirements	2037

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	2038

A.5.6.1.3.1	Test purpose and Environment	2038

A.5.6.1.3.2	Test Requirements	2042

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	2042

A.5.6.1.4.1	Test purpose and Environment	2042

A.5.6.1.4.2	Test Requirements	2045

A.5.6.2	Inter-frequency Measurements	2046

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2046

A.5.6.2.1.1	Test Purpose and Environment	2046

A.5.6.2.1.2	Test Requirements	2049

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2049

A.5.6.2.2.1	Test Purpose and Environment	2049

A.5.6.2.2.2	Test Requirements	2053

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2053

A.5.6.2.3.1	Test Purpose and Environment	2053

A.5.6.2.3.2	Test Requirements	2057

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2057

A.5.6.2.4.1	Test Purpose and Environment	2057

A.5.6.2.4.2	Test Requirements	2061

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2061

A.5.6.2.5.1	Test Purpose and Environment	2061

A.5.6.2.5.2	Test Requirements	2066

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2066

A.5.6.2.6.1	Test Purpose and Environment	2066

A.5.6.2.6.2	Test Requirements	2070

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2070

A.5.6.2.7.1	Test Purpose and Environment	2070

A.5.6.2.7.2	Test Requirements	2075

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2075

A.5.6.2.8.1	Test Purpose and Environment	2075

A.5.6.2.8.2	Test Requirements	2080

A.5.6.3	L1-RSRP measurement for beam reporting	2081

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2081

A.5.6.3.1.1	Test Purpose and Environment	2081

A.5.6.3.1.2	Test parameters	2081

A.5.6.3.1.3	Test Requirements	2083

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2083

A.5.6.3.2.1	Test Purpose and Environment	2083

A.5.6.3.2.2	Test parameters	2084

A.5.6.3.2.3	Test Requirements	2086

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2086

A.5.6.3.3.1	Test Purpose and Environment	2086

A.5.6.3.3.2	Test parameters	2087

A.5.6.3.3.3	Test Requirements	2089

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2089

A.5.6.3.4.1	Test Purpose and Environment	2089

A.5.6.3.4.2	Test parameters	2090

A.5.6.3.4.3	Test Requirements	2092

A.5.6.4	CLI measurements	2093

A.5.6.4.1	SRS-RSRP measurement with DRX	2093

A.5.6.4.1.1	Test Purpose and Environment	2093

A.5.6.4.1.2	Test Parameters	2093

A.5.6.4.1.3	Test Requirements	2095

A.5.6.4.2	CLI-RSSI measurement with DRX	2095

A.5.6.4.2.1	Test Purpose and Environment	2095

A.5.6.4.2.2	Test Parameters	2096

A.5.6.4.2.3	Test Requirements	2098

A.5.6.5	Measurements with autonomous gaps	2098

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2098

A.5.6.5.1.1	Test Purpose and Environment	2098

A.5.6.5.1.2	Test Requirements	2101

A.5.6.6	L1-SINR measurement for beam reporting	2102

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2104

A.5.6.6.2.1	Test Purpose and Environment	2104

A.5.6.6.2.2	Test parameters	2105

A.5.6.6.2.3	Test Requirements	2108

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2108

A.5.6.6.3.1	Test Purpose and Environment	2108

A.5.6.6.3.2	Test parameters	2109

A.5.6.6.3.3	Test Requirements	2111

A.5.6.7	CSI-RS based Intra-frequency Measurements	2111

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2111

A.5.6.7.1.1	Test purpose and Environment	2111

A.5.6.7.1.2	Test Requirements	2114

A.5.6.8	CSI-RS based Inter-frequency Measurements	2114

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2114

A.5.6.8.1.1	Test Purpose and Environment	2114

A.5.6.8.1.2	Test Requirements	2119

A.5.7	Measurement Performance requirements	2119

A.5.7.1	SS-RSRP	2119

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2119

A.5.7.1.1.1	Test Purpose and Environment	2119

A.5.7.1.1.2	Test parameters	2120

A.5.7.1.1.3	Test Requirements	2122

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2123

A.5.7.1.2.1	Test Purpose and Environment	2123

A.5.7.1.2.2	Test parameters	2123

A.5.7.1.2.3	Test Requirements	2127

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2128

A.5.7.1.3.1	Test Purpose and Environment	2128

A.5.7.1.3.2	Test parameters	2128

A.5.7.1.3.3	Test Requirements	2131

A.5.7.2	SS-RSRQ	2132

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2132

A.5.7.2.1.1	Test Purpose and Environment	2132

A.5.7.2.1.2	Test Parameters	2132

A.5.7.2.1.3	Test Requirements	2135

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2135

A.5.7.2.2.1	Test Purpose and Environment	2135

A.5.7.2.2.2	Test Parameters	2135

A.5.7.2.2.3	Test Requirements	2137

A.5.7.3	SS-SINR	2138

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2138

A.5.7.3.1.1	Test Purpose and Environment	2138

A.5.7.3.1.2	Test Parameters	2138

A.5.7.3.1.3	Test Requirements	2141

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2141

A.5.7.3.2.1	Test Purpose and Environment	2141

A.5.7.3.2.2	Test Parameters	2141

A.5.7.3.2.3	Test Requirements	2143

A.5.7.4	L1-RSRP measurement for beam reporting	2143

A.5.7.4.1	SSB based L1-RSRP measurement	2143

A.5.7.4.1.1	Test Purpose and Environment	2143

A.5.7.4.1.2	Test parameters	2144

A.5.7.4.1.3	Test Requirements	2146

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2147

A.5.7.4.2.1	Test Purpose and Environment	2147

A.5.7.4.2.2	Test parameters	2147

A.5.7.4.2.3	Test Requirements	2149

A.5.7.5	CLI measurements	2150

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2150

A.5.7.5.1.1	Test Purpose and Environment	2150

A.5.7.5.1.2	Test parameters	2150

A.5.7.5.1.3	Test Requirements	2153

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2154

A.5.7.5.2.1	Test Purpose and Environment	2154

A.5.7.5.2.2	Test parameters	2154

A.5.7.5.2.3	Test Requirements	2156

A.5.7.6	L1-SINR measurement for beam reporting	2157

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2160

A.5.7.6.2.1	Test Purpose and Environment	2160

A.5.7.6.2.2	Test parameters	2160

A.5.7.6.2.3	Test Requirements	2162

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2163

A.5.7.6.3.1	Test Purpose and Environment	2163

A.5.7.6.3.2	Test parameters	2163

A.5.7.6.3.3	Test Requirements	2165

A.5.7.7	CSI-RSRP	2166

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2166

A.5.7.7.1.1	Test Purpose and Environment	2166

A.5.7.7.1.2	Test parameters	2166

A.5.7.7.1.3	Test Requirements	2168

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2169

A.5.7.7.2.1	Test Purpose and Environment	2169

A.5.7.7.2.2	Test parameters	2169

A.5.7.7.2.3	Test Requirements	2173

A.5.7.8	CSI-RSRQ	2174

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2174

A.5.7.8.1.1	Test Purpose and Environment	2174

A.5.7.8.1.2	Test Parameters	2174

A.5.7.8.1.3	Test Requirements	2176

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2176

A.5.7.8.2.1	Test Purpose and Environment	2176

A.5.7.8.2.2	Test Parameters	2176

A.5.7.8.2.3	Test Requirements	2178

A.5.7.9	CSI-SINR	2178

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2178

A.5.7.9.1.1	Test Purpose and Environment	2178

A.5.7.9.1.2	Test Parameters	2178

A.5.7.9.1.3	Test Requirements	2181

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2181

A.5.7.9.2.1	Test Purpose and Environment	2181

A.5.7.9.2.2	Test Parameters	2181

A.5.7.9.2.3	Test Requirements	2184

A.5.8	Void	2184

A.6	NR standalone tests with all NR cells in FR1	2187

A.6.1	SA: RRC_IDLE state mobility	2187

A.6.1.1	Cell re-selection to NR	2187

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2187

A.6.1.1.1.1	Test Purpose and Environment	2187

A.6.1.1.1.2	Test Parameters	2187

A.6.1.1.1.3	Test Requirements	2191

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2191

A.6.1.1.2.1	Test Purpose and Environment	2191

A.6.1.1.2.2	Test Parameters	2191

A.6.1.1.2.3	Test Requirements	2195

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2195

A.6.1.1.3.1	Test Purpose and Environment	2195

A.6.1.1.3.2	Test Parameters	2195

A.6.1.1.3.3	Test Requirements	2200

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2200

A.6.1.1.4.1	Test Purpose and Environment	2200

A.6.1.1.4.2	Test Parameters	2200

A.6.1.1.4.3	Test Requirements	2203

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2203

A.6.1.1.5.1	Test Purpose and Environment	2203

A.6.1.1.5.2	Test Parameters	2203

A.6.1.1.5.3	Test Requirements	2208

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2208

A.6.1.1.6.1	Test Purpose and Environment	2208

A.6.1.1.6.2	Test Parameters	2209

A.6.1.1.6.3	Test Requirements	2213

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2214

A.6.1.1.7.1	Test Purpose and Environment	2214

A.6.1.1.7.2	Test Parameters	2214

A.6.1.1.7.3	Test Requirements	2217

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2217

A.6.1.1.8.1	Test Purpose and Environment	2217

A.6.1.1.8.2	Test Parameters	2217

A.6.1.1.8.3	Test Requirements	2221

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2221

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2221

A.6.1.2.1.1	Test Purpose and Environment	2221

A.6.1.2.1.2	Test Parameters	2221

A.6.1.2.1.3	Test Requirements	2224

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2225

A.6.1.2.2.1	Test Purpose and Environment	2225

A.6.1.2.2.2	Test Parameters	2225

A.6.1.2.2.3	Test Requirements	2228

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2229

A.6.1.2.3.1	Test Purpose and Environment	2229

A.6.1.2.3.2	Test Parameters	2229

A.6.1.2.3.3	Test Requirements	2232

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2233

A.6.1.2.4.1	Test Purpose and Environment	2233

A.6.1.2.4.2	Test Parameters	2233

A.6.1.2.4.3	Test Requirements	2236

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2237

A.6.1.2.5.1	Test Purpose and Environment	2237

A.6.1.2.5.2	Test Parameters	2237

A.6.1.2.5.3	Test Requirements	2241

A.6.1.1.7	Void	2242

A.6.2	SA: RRC_INACTIVE state mobility	2242

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2242

A.6.2.1.1	Test purpose and Environment	2242

A.6.2.1.2	Test Parameters	2243

A.6.2.1.3	Test requirements	2245

A.6.3	RRC_CONNECTED state mobility	2245

A.6.3.1	Handover	2245

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2245

A.6.3.1.1.1	Test Purpose and Environment	2245

A.6.3.1.1.2	Test Parameters	2245

A.6.3.1.1.3 Test Requirements	2249

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2249

A.6.3.1.2.1	Test Purpose and Environment	2249

A.6.3.1.2.2	Test Parameters	2249

A.6.3.1.2.3	Test Requirements	2253

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2253

A.6.3.1.3.1	Test Purpose and Environment	2253

A.6.3.1.3.2	Test Parameters	2253

A.6.3.1.3.3	Test Requirements	2257

A.6.3.1.4	SA NR - E-UTRAN handover	2257

A.6.3.1.4.1	Test Purpose and Environment	2257

A.6.3.1.4.2	Test Requirements	2263

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2263

A.6.3.1.5.1	Test Purpose and Environment	2263

A.6.3.1.5.2	Test Requirements	2269

A.6.3.1.6	 SA NR - UTRAN FDD handover	2269

A.6.3.1.6.1	Test Purpose and Environment	2269

A.6.3.1.6.2	Test Requirements	2273

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2273

A.6.3.1.7.1	Test Purpose and Environment	2273

A.6.3.1.7.2	Test Parameters	2273

A.6.3.1.7.3	Test Requirements	2277

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2278

A.6.3.1.8.1	Test Purpose and Environment	2278

A.6.3.1.8.2	Test Parameters	2278

A.6.3.1.8.3	Test Requirements	2282

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2283

A.6.3.1.9.1	Test Purpose and Environment	2283

A.6.3.1.9.2	Test Parameters	2283

A.6.3.1.9.3	Test Requirements	2287

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2287

A.6.3.1.10.1	Test Purpose and Environment	2287

A.6.3.1.10.2	Test Parameters	2287

A.6.3.1.10.3	Test Requirements	2290

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2290

A.6.3.1.11.1	Test Purpose and Environment	2290

A.6.3.1.11.2	Test Parameters	2290

A.6.3.1.11.3 Test Requirements	2297

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2297

A.6.3.1.12.1	Test Purpose and Environment	2297

A.6.3.1.12.2	Test Parameters	2297

A.6.3.1.12.3 Test Requirements	2305

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2305

A.6.3.1.13.1	Test Purpose and Environment	2305

A.6.3.1.13.2	Test Requirements	2314

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2314

A.6.3.1.14.1	Test Purpose and Environment	2314

A.6.3.1.14.2	Test Requirements	2323

A.6.3.2	RRC Connection Mobility Control	2324

A.6.3.2.1	SA: RRC Re-establishment	2324

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2324

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2328

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2332

A.6.3.2.2	Random Access	2335

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2335

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2339

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2344

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2349

A.6.3.2.3	SA: RRC Connection Release with Redirection	2353

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2353

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2357

A.6.3.3 Conditional handover	2364

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2364

A.6.3.3.1.1	Test Purpose and Environment	2364

A.6.3.3.1.2	Test Parameters	2364

A.6.3.3.1.3 Test Requirements	2368

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2368

A.6.3.3.2.1	Test Purpose and Environment	2368

A.6.3.3.2.2	Test Parameters	2368

A.6.3.3.2.3 Test Requirements	2372

A.6.4	Timing	2372

A.6.4.1	UE transmit timing	2372

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2372

A.6.4.1.1.1	Test Purpose and environment	2372

A.6.4.1.1.2	Test requirements	2376

A.6.4.2	UE timer accuracy	2376

A.6.4.3	Timing advance	2376

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2376

A.6.4.3.1.1	Test Purpose and Environment	2376

A.6.4.3.1.2	Test Parameters	2376

A.6.4.3.1.3	Test Requirements	2380

A.6.5	Signalling characteristics	2380

A.6.5.1	Radio link Monitoring	2380

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2381

A.6.5.1.1.1	Test Purpose and Environment	2381

A.6.5.1.1.2	Test Requirements	2386

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2386

A.6.5.1.2.1	Test Purpose and Environment	2386

A.6.5.1.2.2	Test Requirements	2392

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2392

A.6.5.1.3.1	Test Purpose and Environment	2392

A.6.5.1.3.2	Test Requirements	2398

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2398

A.6.5.1.4.1	Test Purpose and Environment	2398

A.6.5.1.4.2	Test Requirements	2404

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2404

A.6.5.1.5.1	Test Purpose and Environment	2404

A.6.5.1.5.2	Test Requirements	2410

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2410

A.6.5.1.6.1	Test Purpose and Environment	2410

A.6.5.1.6.2	Test Requirements	2415

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2415

A.6.5.1.7.1	Test Purpose and Environment	2415

A.6.5.1.7.2	Test Requirements	2420

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2420

A.6.5.1.8.1	Test Purpose and Environment	2420

A.6.5.1.8.2	Test Requirements	2425

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2425

A.6.5.1.9.1	Test Purpose and Environment	2425

A.6.5.1.9.2	Test Requirements	2431

A.6.5.2	Interruption	2431

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2431

A.6.5.2.1.2	Test Requirements	2435

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2436

A.6.5.2.2.1	Test Purpose and Environment	2436

A.6.5.2.2.2	Test Parameters	2436

A.6.5.2.2.3	Test Requirements	2438

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2439

A.6.5.2.3.1	Test Purpose and Environment	2439

A.6.5.2.3.2	Test Parameters	2439

A.6.5.2.3.3	Test Requirements	2441

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2442

A.6.5.2.4.1	Test Purpose and Environment	2442

A.6.5.2.4.2	Test Parameters	2442

A.6.5.2.4.3	Test Requirements	2444

A.6.5.3	SCell Activation and Deactivation Delay	2445

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2445

A.6.5.3.1.1	Test Purpose and Environment	2445

A.6.5.3.1.2	Test Requirements	2450

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2451

A.6.5.3.2.1	Test Purpose and Environment	2451

A.6.5.3.2.2	Test Requirements	2451

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2451

A.6.5.3.3.1	Test Purpose and Environment	2451

A.6.5.3.3.2	Test Requirements	2452

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2452

A.6.5.3.4.1	Test Purpose and Environment	2452

A.6.5.3.4.2	Test Requirements	2460

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2460

A.6.5.3.5.1	Test Purpose and Environment	2460

A.6.5.3.5.2	Test Requirements	2466

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2467

A.6.5.3.6.1	Test Purpose and Environment	2467

A.6.5.3.6.2	Test Requirements	2471

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2471

A.6.5.3.7.1	Test Purpose and Environment	2471

A.6.5.3.7.2	Test Requirements	2475

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2476

A.6.5.3.8.1	Test Purpose and Environment	2476

A.6.5.3.8.2	Test Requirements	2480

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2481

A.6.5.3.9.1	Test Purpose and Environment	2481

A.6.5.3.9.2	Test Requirements	2485

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2486

A.6.5.3.10.1	Test Purpose and Environment	2486

A.6.5.3.10.2	Test Requirements	2489

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2489

A.6.5.3.11.1	Test Purpose and Environment	2489

A.6.5.3.11.2	Test Requirements	2490

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2490

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2490

A.6.5.4.1.1	Test Purpose and Environment	2490

A.6.5.4.1.2	Test Requirements	2500

A.6.5.4.2	Void	2501

A.6.5.5	Beam Failure Detection and Link recovery procedures	2501

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2501

A.6.5.5.1.1	Test Purpose and Environment	2501

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2507

A.6.5.5.2.1	Test Purpose and Environment	2507

A.6.5.5.2.2	Test Requirements	2513

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2514

A.6.5.5.3.1	Test Purpose and Environment	2514

A.6.5.5.3.2	Test Requirements	2519

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2520

A.6.5.5.4.1	Test Purpose and Environment	2520

A.6.5.5.4.2	Test Requirements	2525

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2526

A.6.5.5.5.1	Test Purpose and Environment	2526

A.6.5.5.5.2	Test Requirements	2530

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2530

A.6.5.5.6.1	Test Purpose and Environment	2530

A.6.5.5.6.2	Test Requirements	2535

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2535

A.6.5.5.7.1	Test Purpose and Environment	2535

A.6.5.5.7.2	Test Requirements	2541

A.6.5.6	Active BWP switch	2542

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2542

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2542

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2548

A.6.5.6.2	RRC-based Active BWP Switch	2553

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2553

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2557

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2557

A.6.5.6.4	SCell dormancy switch	2564

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2564

A.6.5.6.4.1.1	Test Purpose and Environment	2564

A.6.5.6.4.1.2	Test Requirements	2570

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2570

A.6.5.6.4.2.1	 Test Purpose and Environment	2570

A.6.5.6.4.2.2	Test Requirements	2577

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2577

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2577

A.6.5.7	DL interruptions at switching between two uplink carriers	2582

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2582

A.6.5.7.1.1	Test Purpose and Environment	2582

A.6.5.7.1.2	Test Requirements	2586

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2586

A.6.5.7.2.1	Test Purpose and Environment	2586

A.6.5.7.2.2	Test Requirements	2590

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2590

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2590

A.6.5.7A.1.1	Test Purpose and Environment	2590

A.6.5.7A.1.2	Test Requirements	2594

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2594

A.6.5.7A.2.1	Test Purpose and Environment	2594

A.6.5.7A.2.2	Test Requirements	2598

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2598

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2598

A.6.5.7B.1.1	Test Purpose and Environment	2598

A.6.5.7B.1.2	Test Requirements	2602

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2602

A.6.5.7B.2.1	Test Purpose and Environment	2602

A.6.5.7B.2.2	Test Requirements	2606

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2606

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2606

A.6.5.7C.1.1	Test Purpose and Environment	2606

A.6.5.7C.1.2	Test Requirements	2610

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2611

A.6.5.7C.2.1	Test Purpose and Environment	2611

A.6.5.7C.2.2	Test Requirements	2615

A.6.5.8	UE specific CBW change	2616

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2616

A.6.5.8.1.1	Test Purpose and Environment	2616

A.6.5.8.1.2	Test Requirements	2620

A.6.5.9	Pathloss reference signal switching delay	2620

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2620

A.6.5.9.1.1	Test Purpose and Environment	2620

A.6.5.9.1.2	Test Requirements	2623

A.6.6	Measurement procedure	2624

A.6.6.1	Intra-frequency Measurements	2624

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2624

A.6.6.1.1.1	Test purpose and Environment	2624

A.6.6.1.1.2	Test parameters	2624

A.6.6.1.1.3	Test Requirements	2628

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2628

A.6.6.1.2.1	Test purpose and Environment	2628

A.6.6.1.2.2	Test parameters	2628

A.6.6.1.2.3	Test Requirements	2632

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2632

A.6.6.1.3.1	Test purpose and Environment	2632

A.6.6.1.3.2	Test parameters	2632

A.6.6.1.3.3	Test Requirements	2636

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2636

A.6.6.1.4.1	Test purpose and Environment	2636

A.6.6.1.4.2	Test parameters	2636

A.6.6.1.4.3	Test Requirements	2640

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2640

A.6.6.1.5.1	Test purpose and Environment	2640

A.6.6.1.5.2	Test parameters	2640

A.6.6.1.5.3	Test Requirements	2642

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2643

A.6.6.1.6.1	Test purpose and Environment	2643

A.6.6.1.6.2	Test parameters	2643

A.6.6.1.6.3	Test Requirements	2644

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2645

A.6.6.1.7.1	Test purpose and Environment	2645

A.6.6.1.7.2	Test parameters	2645

A.6.6.1.7.3	Test Requirements	2649

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2649

A.6.6.1.8.1	Test purpose and Environment	2649

A.6.6.1.8.2	Test parameters	2649

A.6.6.1.8.3	Test Requirements	2653

A.6.6.2	Inter-frequency Measurements	2653

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2653

A.6.6.2.1.1	Test Purpose and Environment	2653

A.6.6.2.1.2	Test Requirements	2657

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2657

A.6.6.2.2.1	Test Purpose and Environment	2657

A.6.6.2.2.2	Test Requirements	2661

A.6.6.2.3	Void	2662

A.6.6.2.4	Void	2662

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2662

A.6.6.2.5.1	Test Purpose and Environment	2662

A.6.6.2.5.2	Test Requirements	2666

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2666

A.6.6.2.6.1	Test Purpose and Environment	2666

A.6.6.2.6.2	Test Requirements	2670

A.6.6.2.7	Void	2670

A.6.6.2.8	Void	2670

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2670

A.6.6.2.9.1	Test Purpose and Environment	2670

A.6.6.2.9.2	Test Requirements	2674

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2674

A.6.6.2.10.1	Test Purpose and Environment	2674

A.6.6.2.10.2	Test Requirements	2678

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2682

A.6.6.2.12.1	Test Purpose and Environment	2682

A.6.6.2.12.2	Test Requirements	2686

A.6.6.3	Inter-RAT Measurements	2686

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2686

A.6.6.3.1.1	Test Purpose and Environment	2686

A.6.6.3.1.2	Test Requirements	2692

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2692

A.6.6.3.2.1	Test Purpose and Environment	2692

A.6.6.3.2.2	Test Requirements	2699

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2699

A.6.6.3.3.1	Test Purpose and Environment	2699

A.6.6.3.3.2	Test Requirements	2704

A.6.6.4	L1-RSRP measurement for beam reporting	2704

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2704

A.6.6.4.1.1	Test Purpose and Environment	2704

A.6.6.4.1.2	Test parameters	2705

A.6.6.4.1.3	Test Requirements	2708

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2708

A.6.6.4.2.1	Test Purpose and Environment	2708

A.6.6.4.2.2	Test parameters	2709

A.6.6.4.2.3	Test Requirements	2712

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2712

A.6.6.4.3.1	Test Purpose and Environment	2712

A.6.6.4.3.2	Test parameters	2713

A.6.6.4.3.3	Test Requirements	2716

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2716

A.6.6.4.4.1	Test Purpose and Environment	2716

A.6.6.4.4.2	Test parameters	2717

A.6.6.4.4.3	Test Requirements	2720

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2720

A.6.6.4.5.1	Test Purpose and Environment	2720

A.6.6.4.5.2	Test parameters	2721

A.6.6.4.5.3	Test Requirements	2724

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2724

A.6.6.4.6.1	Test Purpose and Environment	2724

A.6.6.4.6.2	Test parameters	2725

A.6.6.4.6.3	Test Requirements	2728

A.6.6.5	Inter-RAT UTRAN FDD measurements	2729

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2729

A.6.6.5.1.1	Test Purpose and Environment	2729

A.6.6.5.1.2	Test Requirements	2732

A.6.6.6	CLI measurements	2732

A.6.6.6.1	SRS-RSRP measurement with DRX	2732

A.6.6.6.1.1	Test Purpose and Environment	2732

A.6.6.6.1.2	Test Parameters	2733

A.6.6.6.1.3	Test Requirements	2736

A.6.6.6.2	CLI-RSSI measurement with DRX	2736

A.6.6.6.2.1	Test Purpose and Environment	2736

A.6.6.6.2.2	Test Parameters	2737

A.6.6.6.2.3	Test Requirements	2739

A.6.6.7	NR measurements with autonomous gaps	2739

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2739

A.6.6.7.1.1	Test Purpose and Environment	2739

A.6.6.7.1.2	Test Parameters	2739

A.6.6.7.1.3	Test Requirements	2742

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2742

A.6.6.7.2.1	Test Purpose and Environment	2742

A.6.6.7.2.2	Test Requirements	2745

A.6.6.8	L1-SINR measurement for beam reporting	2746

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2748

A.6.6.8.2.1	Test Purpose and Environment	2748

A.6.6.8.2.2	Test parameters	2748

A.6.6.8.2.3	Test Requirements	2752

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2752

A.6.6.8.3.1	Test Purpose and Environment	2752

A.6.6.8.3.2	Test parameters	2753

A.6.6.8.3.3	Test Requirements	2755

A.6.6.9	Idle Mode CA/DC Measurements	2755

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2755

A.6.6.9.1.1	Test Purpose and Environment	2755

A.6.6.9.1.2	Test Requirements	2762

A.6.6.10	CSI-RS based intra-frequency Measurements	2762

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2762

A.6.6.10.1.1	Test purpose and Environment	2762

A.6.6.10.1.2	Test Requirements	2765

A.6.6.11	CSI-RS based inter-frequency Measurements	2765

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2765

A.6.6.11.1.1	Test Purpose and Environment	2765

A.6.6.11.1.2	Test Requirements	2769

A.6.6.12	RSTD measurements	2769

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2769

A.6.6.12.1.1	Test Purpose and Environment	2769

A.6.6.12.1.2	Test Requirements	2776

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2777

A.6.6.12.2.1	Test Purpose and Environment	2777

A.6.6.12.2.2	Test Requirements	2783

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2783

A.6.6.12.3.1	Test Purpose and Environment	2783

A.6.6.12.3.2	Test Requirements	2789

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2789

A.6.6.12.4.1	Test Purpose and Environment	2789

A.6.6.12.4.2	Test Requirements	2793

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2794

A.6.6.12.5.1	Test Purpose and Environment	2794

A.6.6.12.5.2	Test Requirements	2798

A.6.6.13 PRS-RSRP measurements	2798

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2798

A.6.6.13.1.1	Test purpose and Environment	2798

A.6.6.13.1.2	Test Requirements	2802

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2802

A.6.6.13.2.1	Test purpose and Environment	2802

A.6.6.13.2.2	Test Requirements	2806

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2806

A.6.6.13.3.1	Test purpose and Environment	2806

A.6.6.13.3.2	Test Requirements	2810

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2810

A.6.6.13.4.1	Test purpose and Environment	2810

A.6.6.14	UE Rx-Tx time difference measurements	2814

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2814

A.6.6.14.1.1	Test purpose and environment	2814

A.6.6.14.1.2	Test requirements	2818

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2818

A.6.6.14.2.1	Test purpose and environment	2818

A.6.6.14.2.2	Test requirements	2822

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2822

A.6.6.14.3.1	Test purpose and environment	2822

A.6.6.14.3.2	Test requirements	2826

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2826

A.6.6.14.4.1	Test purpose and environment	2826

A.6.6.14.4.2	Test requirements	2830

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2830

A.6.6.14.4.1	Test purpose and environment	2830

A.6.6.14.4.2	Test requirements	2834

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2834

A.6.6.15.1	Test Purpose and Environment	2834

A.6.6.15.2	Test Requirements	2841

A.6.6.16	PRS-RSRPP measurements	2842

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2842

A.6.6.16.1.1	Test purpose and Environment	2842

A.6.6.16.1.2	Test Requirements	2844

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2844

A.6.6.16.2.1	Test purpose and Environment	2844

A.6.6.16.2.2	Test Requirements	2847

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2847

A.6.6.16.3.1	Test purpose and Environment	2847

A.6.6.16.3.2	Test Requirements	2851

A.6.6.17	SA event triggered reporting tests with Pre-MG	2851

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2851

A.6.6.17.1.1	Test purpose and Environment	2851

A.6.6.17.1.2	Test parameters	2851

A.6.6.17.1.3	Test Requirements	2855

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2855

A.6.6.17.2.1	Test purpose and Environment	2855

A.6.6.17.2.2	Test parameters	2855

A.6.6.17.2.3	Test Requirements	2860

A.6.6.17.3	Void	2860

A.6.6.17.3.1	Void	2860

A.6.6.17.3.2	Void	2860

A.6.6.17.3.3	Void	2860

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2860

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2860

A.6.6.18.1.1	Test Purpose and Environment	2860

A.6.6.18.1.2	Test Requirements	2864

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2864

A.6.6.18.2.1	Test Purpose and Environment	2864

A.6.6.18.2.2	Test Requirements	2868

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2868

A.6.6.18.3.1	Test Purpose and Environment	2868

A.6.6.18.3.2	Test Requirements	2875

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2875

A.6.6.18.4.1	Test Purpose and Environment	2875

A.6.6.18.4.2	Test Requirements	2879

A.6.6.19	SA event triggered reporting tests with NCSG	2880

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2880

A.6.6.19.1.1	Test purpose and Environment	2880

A.6.6.19.1.2	Test parameters	2880

A.6.6.19.1.3	Test Requirements	2884

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2884

A.6.6.19.2.1	Test Purpose and Environment	2884

A.6.6.19.2.2	Test parameters	2884

A.6.6.19.2.3	Test Requirements	2888

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2888

A.6.6.19.3.1	Test Purpose and Environment	2888

A.6.6.19.3.2	Test parameters	2889

A.6.6.19.3.3	Test Requirements	2894

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2894

A.6.6.19.4.1	Test purpose and Environment	2894

A.6.6.19.4.2	Test parameters	2894

A.6.6.19.4.3	Test Requirements	2897

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2897

A.6.6.20.1	Test purpose and environment	2897

A.6.6.20.2	Test requirements	2901

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2901

A.6.6.21.1	Test purpose and environment	2901

A.6.6.21.2	Test requirements	2903

A.6.7	Measurement Performance requirements	2904

A.6.7.1	SS-RSRP	2904

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2904

A.6.7.1.1.1	Test Purpose and Environment	2904

A.6.7.1.1.2	Test parameters	2904

A.6.7.1.1.3	Test Requirements	2909

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2909

A.6.7.1.2.1	Test Purpose and Environment	2909

A.6.7.1.2.2	Test parameters	2909

A.6.7.1.2.3	Test Requirements	2913

A.6.7.1.3	Void	2913

A.6.7.2	SS-RSRQ	2913

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2913

A.6.7.2.1.1	Test Purpose and Environment	2913

A.6.7.2.1.2	Test Parameters	2914

A.6.7.2.1.3	Test Requirements	2919

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2919

A.6.7.2.2.1	Test Purpose and Environment	2919

A.6.7.2.2.2	Test Parameters	2919

A.6.7.2.2.3	Test Requirements	2924

A.6.7.3	SS-SINR	2924

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2924

A.6.7.3.1.1	Test Purpose and Environment	2924

A.6.7.3.1.2	Test Parameters	2925

A.6.7.3.1.3	Test Requirements	2929

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2930

A.6.7.3.2.1	Test Purpose and Environment	2930

A.6.7.3.2.2	Test Parameters	2930

A.6.7.3.2.3	Test Requirements	2934

A.6.7.4	L1-RSRP measurement for beam reporting	2935

A.6.7.4.1	SSB based L1-RSRP measurement	2935

A.6.7.4.1.1	Test Purpose and Environment	2935

A.6.7.4.1.2	Test parameters	2935

A.6.7.4.1.3	Test Requirements	2939

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2939

A.6.7.4.2.1	Test Purpose and Environment	2939

A.6.7.4.2.2	Test parameters	2940

A.6.7.4.2.3	Test Requirements	2944

A.6.7.5	E-UTRAN RSRP	2944

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2944

A.6.7.5.1.1	Test Purpose and Environment	2944

A.6.7.5.1.2	Test parameters	2945

A.6.7.5.1.3	Test Requirements	2951

A.6.7.6	E-UTRAN RSRQ	2952

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2952

A.6.7.6.1.1	Test Purpose and Environment	2952

A.6.7.6.1.2	Test parameters	2952

A.6.7.6.1.3	Test Requirements	2957

A.6.7.7	E-UTRAN RS-SINR	2958

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2958

A.6.7.7.1.1	Test Purpose and Environment	2958

A.6.7.7.1.2	Test parameters	2958

A.6.7.7.1.3	Test Requirements	2964

A.6.7.8	CLI measurements	2965

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2965

A.6.7.8.1.1	Test Purpose and Environment	2965

A.6.7.8.1.2	Test parameters	2965

A.6.7.8.1.3	Test Requirements	2971

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2971

A.6.7.8.2.1	Test Purpose and Environment	2971

A.6.7.8.2.2	Test parameters	2972

A.6.7.8.2.3	Test Requirements	2975

A.6.7.9	L1-SINR measurement for beam reporting	2976

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2980

A.6.7.9.2.1	Test Purpose and Environment	2980

A.6.7.9.2.2	Test parameters	2981

A.6.7.9.2.3	Test Requirements	2986

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2986

A.6.7.9.3.1	Test Purpose and Environment	2986

A.6.7.9.3.2	Test parameters	2986

A.6.7.9.3.3	Test Requirements	2991

A.6.7.10	CSI-RSRP	2991

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2991

A.6.7.9.10.1	Test Purpose and Environment	2991

A.6.7.9.10.2	Test parameters	2992

A.6.7.10.1.3	Test Requirements	2997

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2997

A.6.7.9.10.1	Test Purpose and Environment	2997

A.6.7.10.2.2	Test parameters	2997

A.6.7.10.2.3	Test Requirements	3002

A.6.7.11	CSI-RSRQ	3002

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3002

A.6.7.11.1.1	Test Purpose and Environment	3002

A.6.7.11.1.2	Test Parameters	3002

A.6.7.11.1.3	Test Requirements	3007

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3007

A.6.7.11.2.1	Test Purpose and Environment	3007

A.6.7.11.2.2	Test Parameters	3007

A.6.7.11.2.3	Test Requirements	3013

A.6.7.12	CSI-SINR	3014

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3014

A.6.7.12.1.1	Test Purpose and Environment	3014

A.6.7.12.1.2	Test Parameters	3014

A.6.7.12.1.3	Test Requirements	3018

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3019

A.6.7.12.2.1	Test Purpose and Environment	3019

A.6.7.12.2.2	Test Parameters	3019

A.6.7.12.2.3	Test Requirements	3025

A.6.7.13	RSTD measurements	3025

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	3025

A.6.7.13.1.1	Test purpose and Environment	3025

A.6.7.13.1.2	Test Requirements	3028

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	3028

A.6.7.13.2.1	Test purpose and Environment	3028

A.6.7.13.2.2	Test Requirements	3032

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3032

A.6.7.13.3.1	Test purpose and Environment	3032

A.6.7.13.3.2	Test Requirements	3034

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	3034

A.6.7.14	PRS-RSRP measurements	3038

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	3038

A.6.7.14.1.1	Test Purpose and Environment	3038

A.6.7.14.1.2	Test parameters	3039

A.6.7.14.1.3	Test Requirements	3043

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	3043

A.6.7.14.2.1	Test Purpose and Environment	3043

A.6.7.14.2.2	Test parameters	3043

A.6.7.14.2.3	Test Requirements	3046

A.6.7.14.3	Void	3047

A.6.7.14.3.1	Void	3047

A.6.7.14.3.2	Void	3047

A.6.7.14.3.3	Void	3047

A.6.7.15	UE Rx-Tx time difference measurements	3047

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	3047

A.6.7.15.1.1	Test purpose and environment	3047

A.6.7.15.1.2	Test parameters	3047

A.6.7.15.1.3	Test requirements	3050

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3051

A.6.7.15.2.1	Test purpose and environment	3051

A.6.7.15.2.2	Test parameters	3051

A.6.7.15.2.3	Test requirements	3054

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3054

A.6.7.15.3.1	Test purpose and environment	3054

A.6.7.15.3.2	Test parameters	3055

A.6.7.15.3.3	Test requirements	3058

A.6.7.16	PRS-RSRPP measurements	3058

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3058

A.6.7.16.1.1	Test Purpose and Environment	3058

A.6.7.16.1.2	Test parameters	3059

A.6.7.16.1.3	Test Requirements	3064

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3064

A.6.7.16.2.1	Test Purpose and Environment	3064

A.6.7.16.2.2	Test parameters	3064

A.6.8	Measurement procedure in RRC_INACTIVE	3068

A.6.8.1	RSTD measurements	3068

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3068

A.6.8.1.1.1	Test Purpose and Environment	3068

A.6.8.1.1.2	Test Requirements	3071

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3071

A.6.8.1.2.1	Test Purpose and Environment	3071

A.6.8.1.2.2	Test Requirements	3076

A.6.8.2	PRS-RSRP measurements	3076

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3076

A.6.8.2.1.1	Test purpose and Environment	3076

A.6.8.2.1.2	Test Requirements	3078

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3079

A.6.8.2.2.1	Test purpose and Environment	3079

A.6.8.2.2.2	Test Requirements	3082

A.6.8.3	UE Rx-Tx time difference measurements	3082

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3082

A.6.8.3.1.1	Test purpose and environment	3082

A.6.8.3.1.2	Test requirements	3086

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3086

A.6.8.3.2.1	Test purpose and environment	3086

A.6.8.3.2.2	Test requirements	3089

A.6.8.4	PRS-RSRPP measurements	3089

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3089

A.6.8.4.1.1	Test purpose and Environment	3089

A.6.8.4.1.2	Test Requirements	3092

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3092

A.6.8.4.2.1	Test purpose and Environment	3092

A.6.8.4.2.2	Test Requirements	3094

A.6.9	Measurement performance requirements in RRC_INACTIVE	3094

A.6.9.1	RSTD measurements	3094

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3094

A.6.9.1.1.1	Test purpose and Environment	3094

A.6.9.1.1.2	Test Requirements	3096

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3096

A.6.9.1.2.1	Test purpose and Environment	3096

A.6.9.1.2.2	Test Requirements	3098

A.6.9.2	PRS-RSRP measurements	3098

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3098

A.6.9.2.1.1	Test Purpose and Environment	3098

A.6.9.2.1.2	Test parameters	3099

A.6.9.2.1.3	Test Requirements	3104

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3104

A.6.9.2.2.1	Test Purpose and Environment	3104

A.6.9.2.2.2	Test parameters	3104

A.6.9.2.2.3	Test Requirements	3107

A.6.9.3	UE Rx-Tx time difference measurements	3108

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3108

A.6.9.3.1.1.1	Test purpose and environment	3108

A.6.9.3.1.1.2 Test parameters	3108

A.6.9.3.1.1.3	Test requirements	3111

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3111

A.6.9.3.2.1	Test purpose and environment	3111

A.6.9.3.2.2	Test parameters	3111

A.6.9.3.2.3	Test requirements	3114

A.6.9.4	PRS-RSRPP measurements	3114

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3114

A.6.9.4.1.1	Test Purpose and Environment	3114

A.6.9.4.1.2	Test parameters	3114

A.6.9.4.1.3	Test Requirements	3119

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3119

A.6.9.4.2.1	Test Purpose and Environment	3119

A.6.9.4.2.2	Test parameters	3119

A.6.9.4.2.3	Test Requirements	3122

A.7	NR standalone tests with one or more NR cells in FR2	3125

A.7.1	SA: RRC_IDLE state mobility	3125

A.7.1.1	Cell re-selection to NR	3125

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3125

A.7.1.1.1.1	Test Purpose and Environment	3125

A.7.1.1.1.2	Test Parameters	3125

A.7.1.1.1.3	Test Requirements	3128

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3128

A.7.1.1.2.1	Test Purpose and Environment	3128

A.7.1.1.2.2	Test Parameters	3129

A.7.1.1.2.3	Test Requirements	3131

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3131

A.7.1.1.3.1	Test Purpose and Environment	3131

A.7.1.1.3.2	Test Parameters	3131

A.7.1.1.3.3	Test Requirements	3134

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3134

A.7.1.1.4.1	Test Purpose and Environment	3134

A.7.1.1.4.2	Test Parameters	3134

A.7.1.1.4.3	Test Requirements	3137

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3137

A.7.1.1.5.1	Test Purpose and Environment	3137

A.7.1.1.5.2	Test Parameters	3137

A.7.1.1.5.3	Test Requirements	3140

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3140

A.7.1.1.6.1	Test Purpose and Environment	3140

A.7.1.1.6.2	Test Parameters	3140

A.7.1.1.6.3	Test Requirements	3143

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3143

A.7.1.1.7.1	Test Purpose and Environment	3143

A.7.1.1.7.2	Test Parameters	3143

A.7.1.1.7.3	Test Requirements	3147

A.7.2	SA: RRC_INACTIVE state mobility	3147

A.7.2.1	Small Data Transmission	3147

A.7.2.1.1	TA validation for CG-SDT in FR2	3147

A.7.2.1.1.1	Test Purpose and Environment	3147

A.7.2.1.1.2	Test Requirements	3151

A.7.3	RRC_CONNECTED state mobility	3151

A.7.3.1	Handover	3151

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3151

A.7.3.1.1.1	Test Purpose and Environment	3151

A.7.3.1.1.2	Test Parameters	3151

A.7.3.1.1.3	Test Requirements	3155

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3155

A.7.3.1.2.1	Test Purpose and Environment	3155

A.7.3.1.2.2	Test Parameters	3155

A.7.3.1.2.3	Test Requirements	3158

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3158

A.7.3.1.3.1	Test Purpose and Environment	3158

A.7.3.1.3.2	Test Parameters	3158

A.7.3.1.3.3	Test Requirements	3159

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3160

A.7.3.1.4.1	Test Purpose and Environment	3160

A.7.3.1.4.2	Test Parameters	3160

A.7.3.1.4.3 Test Requirements	3167

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3167

A.7.3.1.5.1	Test Purpose and Environment	3167

A.7.3.1.5.2	Test Parameters	3167

A.7.3.1.5.3 Test Requirements	3174

A.7.3.1.6	Handover with PSCell from SA to EN-DC; unknown FR2 target cell	3174

A.7.3.1.6.1	Test Purpose and Environment	3174

A.7.3.1.6.2	Test Parameters	3174

A.7.3.1.6.3	Test Requirements	3179

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3179

A.7.3.1.7.1	Test purpose and environment	3179

A.7.3.1.7.2	Test Requirements	3185

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3185

A.7.3.1.8.1	Test Purpose and Environment	3185

A.7.3.1.8.2	Test Requirements	3191

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3191

A.7.3.1.9.1	Test Purpose and Environment	3191

A.7.3.1.9.2	Test Parameters	3191

A.7.3.1.9.3	Test Requirements	3194

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3194

A.7.3.1.10.1	Test Purpose and Environment	3194

A.7.3.1.10.2	Test Parameters	3194

A.7.3.1.10.3	Test Requirements	3198

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3198

A.7.3.1.11.1	Test Purpose and Environment	3198

A.7.3.1.11.2	Test Parameters	3198

A.7.3.1.11.3	Test Requirements	3202

A.7.3.2	RRC Connection Mobility Control	3202

A.7.3.2.1	SA: RRC Re-establishment	3202

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3202

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3205

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3208

A.7.3.2.1.3.1	Test Purpose and Environment	3208

A.7.3.2.1.3.2	Test Requirements	3210

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3211

A.7.3.2.1.4.1	Test Purpose and Environment	3211

A.7.3.2.1.4.2	Test Requirements	3213

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3214

A.7.3.2.1.5.1	Test Purpose and Environment	3214

A.7.3.2.1.5.2	Test Requirements	3217

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3217

A.7.3.2.1.6.1	Test Purpose and Environment	3217

A.7.3.2.1.6.2	Test Requirements	3219

A.7.3.2.2	Random Access	3220

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3220

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3224

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3228

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3231

A.7.3.2.3	SA: RRC Connection Release with Redirection	3234

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3234

A.7.3.3 Conditional Handover	3238

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3238

A.7.3.3.1.1	Test Purpose and Environment	3238

A.7.3.3.1.2	Test Parameters	3238

A.7.3.3.1.2.3	Test Requirements	3241

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3241

A.7.3.3.2.1	Test Purpose and Environment	3241

A.7.3.3.2.2	Test Parameters	3241

A.7.3.3.2.3 Test Requirements	3243

A.7.4	Timing	3243

A.7.4.1	UE transmit timing	3243

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3243

A.7.4.1.1.1	Test Purpose and environment	3243

A.7.4.1.1.2	Test requirements	3246

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3247

A.7.4.1.2.1	Test Purpose and environment	3247

A.7.4.1.2.2	Test requirements	3251

A.7.4.2	UE timer accuracy	3252

A.7.4.3	Timing advance	3252

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3252

A.7.4.3.1.1	Test Purpose and Environment	3252

A.7.4.3.1.2	Test Parameters	3252

A.7.4.3.1.3 Test Requirements	3255

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3256

A.7.4.3.2.1	Test Purpose and Environment	3256

A.7.4.3.2.2	Test Parameters	3256

A.7.4.3.2.3	Test Requirements	3260

A.7.5	Signaling characteristics	3260

A.7.5.1	Radio link Monitoring	3260

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3260

A.7.5.1.1.1	Test Purpose and Environment	3260

A.7.5.1.1.2	Test Requirements	3263

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3264

A.7.5.1.2.1	Test Purpose and Environment	3264

A.7.5.1.2.2	Test Requirements	3268

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3269

A.7.5.1.3.1	Test Purpose and Environment	3269

A.7.5.1.3.2	Test Requirements	3273

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3273

A.7.5.1.4.1	Test Purpose and Environment	3273

A.7.5.1.4.2	Test Requirements	3278

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3278

A.7.5.1.5.1	Test Purpose and Environment	3278

A.7.5.1.5.2	Test Requirements	3283

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3283

A.7.5.1.6.1	Test Purpose and Environment	3283

A.7.5.1.6.2	Test Requirements	3287

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3287

A.7.5.1.7.1	Test Purpose and Environment	3287

A.7.5.1.7.2	Test Requirements	3291

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3291

A.7.5.1.8.1	Test Purpose and Environment	3291

A.7.5.1.8.2	Test Requirements	3296

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3296

A.7.5.1.9.1	Test Purpose and Environment	3296

A.7.5.1.9.2	Test Requirements	3299

A.7.5.2	Interruption	3299

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3299

A.7.5.2.1.1	Test Purpose and Environment	3299

A.7.5.2.1.2	Test Requirements	3302

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3303

A.7.5.2.2.1	Test Purpose and Environment	3303

A.7.5.2.2.2	Test Parameters	3303

A.7.5.2.2.3	Test Requirements	3305

A.7.5.3	SCell Activation and Deactivation Delay	3305

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3305

A.7.5.3.1.1	Test Purpose and Environment	3305

A.7.5.3.1.2	Test Requirements	3307

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3307

A.7.5.3.2.1	Test Purpose and Environment	3307

A.7.5.3.2.2	Test Requirements	3311

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3312

A.7.5.3.3.1	Test Purpose and Environment	3312

A.7.5.3.3.2	Test Requirements	3315

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3316

A.7.5.3.4.1	Test Purpose and Environment	3316

A.7.5.3.4.2	Test Requirements	3319

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3320

A.7.5.3.5.1	Test Purpose and Environment	3320

A.7.5.3.5.2	Test Requirements	3323

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3324

A.7.5.3.6.1	Test Purpose and Environment	3324

A.7.5.3.6.2	Test Requirements	3328

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3329

A.7.5.3.7.1	Test Purpose and Environment	3329

A.7.5.3.7.2	Test Requirements	3333

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3334

A.7.5.3.8.1	Test Purpose and Environment	3334

A.7.5.3.8.2	Test Requirements	3338

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3339

A.7.5.3.9.1	Test Purpose and Environment	3339

A.7.5.3.9.2	Test Requirements	3342

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3343

A.7.5.3.10.1	Test Purpose and Environment	3343

A.7.5.3.10.2	Test Requirements	3347

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3348

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3348

A.7.5.3.11.1.1	Test Purpose and Environment	3348

A.7.5.3.11.1.2	Test Requirements	3352

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3353

A.7.5.3.11.2.1	Test Purpose and Environment	3353

A.7.5.3.11.2.2	Test Requirements	3357

A.7.5.3.12	Void	3358

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3358

A.7.5.3.13.1	Test Purpose and Environment	3358

A.7.5.3.13.2	Test Requirements	3360

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3361

A.7.5.3.14.1	Test Purpose and Environment	3361

A.7.5.3.14.2	Test Requirements	3364

A.7.5.4	Void	3365

A.7.5.5	Beam Failure Detection and Link recovery procedures	3365

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3365

A.7.5.5.1.1	Test Purpose and Environment	3365

A.7.5.5.1.2	Test Requirements	3368

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3369

A.7.5.5.2.1	Test Purpose and Environment	3369

A.7.5.5.2.2	Test Requirements	3372

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3373

A.7.5.5.3.1	Test Purpose and Environment	3373

A.7.5.5.3.2	Test Requirements	3377

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3378

A.7.5.5.4.1	Test Purpose and Environment	3378

A.7.5.5.4.2	Test Requirements	3382

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3383

A.7.5.5.5.1	Test Purpose and Environment	3383

A.7.5.5.5.2	Test Requirements	3386

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3387

A.7.5.5.6.1	Test Purpose and Environment	3387

A.7.5.5.6.2	Test Requirements	3391

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3391

A.7.5.5.7.1	Test Purpose and Environment	3391

A.7.5.5.7.2	Test Requirements	3395

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3396

A.7.5.5.8.1	Test Purpose and Environment	3396

A.7.5.5.8.2	Test Requirements	3400

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3400

A.7.5.5.9.1	Test Purpose and Environment	3400

A.7.5.5.9.2	Test Requirements	3406

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3406

A.7.5.5.10.1	Test Purpose and Environment	3406

A.7.5.5.10.2	Test Requirements	3412

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3412

A.7.5.5.11.1	Test Purpose and Environment	3412

A.7.5.5.11.2	Test Requirements	3417

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3417

A.7.5.5.12.1	Test Purpose and Environment	3417

A.7.5.5.12.2	Test Requirements	3422

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3422

A.7.5.5.13.1	Test Purpose and Environment	3422

A.7.5.5.13.2	Test Requirements	3426

A.7.5.6	Active BWP switch	3426

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3426

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3426

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3431

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3436

A.7.5.6.1.3.1	Test Purpose and Environment	3436

A.7.5.6.1.3.2	Test Requirements	3439

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3439

A.7.5.6.1.4.1	Test Purpose and Environment	3439

A.7.5.6.1.4.2	Test Requirements	3444

A.7.5.6.2	RRC-based Active BWP Switch	3445

A.7.5.6.2.1.1	Test Purpose and Environment	3445

A.7.5.6.2.1.2	Test Requirements	3448

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3449

A.7.5.6.2.2.1	Test Purpose and Environment	3449

A.7.5.6.2.2.2	Test Requirements	3453

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3454

A.7.5.6.3.1.1	Test Purpose and Environment	3454

A.7.5.6.3.1.2	Test Requirements	3457

A.7.5.6.4	SCell dormancy switch	3457

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3457

A.7.5.6.4.1.1	Test Purpose and Environment	3457

A.7.5.6.4.1.2	Test Requirements	3462

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3462

A.7.5.6.4.2.1	 Test Purpose and Environment	3462

A.7.5.6.4.2.2	 Test Requirements	3467

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3467

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3467

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3470

A.7.5.6.5.2.1	Test Purpose and Environment	3470

A.7.5.6.5.2.2	Test Requirements	3474

A.7.5.7	PSCell addition and release delay	3475

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3475

A.7.5.7.1.1	Test Purpose and Environment	3475

A.7.5.7.1.2	Test Requirements	3478

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3478

A.7.5.7.2.1	Test Purpose and Environment	3478

A.7.5.7.2.2	Test Requirements	3481

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3481

A.7.5.7.3.1	Test Purpose and Environment	3481

A.7.5.7.3.2	Test Requirements	3484

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3484

A.7.5.7.4.1	Test Purpose and Environment	3484

A.7.5.7.4.2	Test Requirements	3487

A.7.5.8	Active TCI state switch delay	3487

A.7.5.8.1	MAC-CE based active TCI state switch	3487

A.7.5.8.2	RRC based active TCI state switch	3491

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3495

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3495

A.7.5.8.3.1.1	Test Purpose and Environment	3495

A.7.5.8.3.1.2	Test Requirements	3499

A.7.5.9	Uplink spatial relation switch delay	3500

A.7.5.9.1.1.1	Test Purpose and Environment	3500

A.7.5.9.1.1.2	Test Requirements	3503

A.7.5.9.2	RRC based spatial relation switch	3503

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3503

A.7.5.9.2.1.1	Test Purpose and Environment	3503

A.7.5.9.2.1.2	Test Requirements	3506

A.7.5.10	UE specific CBW change	3506

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3506

A.7.5.10.1.1	Test Purpose and Environment	3506

A.7.5.10.1.2	Test Requirements	3509

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3510

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3510

A.7.5.11.1.1	Test Purpose and Environment	3510

A.7.5.11.1.2	Test Requirements	3513

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3513

A.7.5.12.1	Addition and Release Delay of PSCell	3513

A.7.5.12.1.1	Test purpose and environment	3513

A.7.5.12.1.2	Test Parameters	3513

A.7.5.12.1.3	Test Requirements	3516

A.7.5.13	 Unified TCI state switching delay	3516

A.7.5.13.1	MAC-CE based active joint TCI state switching	3516

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3516

A.7.5.13.1.1.1	Test Purpose and Environment	3516

A.7.5.13.1.1.2	Test parameters	3517

A.7.5.13.1.1.3	Test Requirements	3519

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3519

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3519

A.7.5.13.2.1.1	Test Purpose and Environment	3519

A.7.5.13.2.1.2	Test parameters	3520

A.7.5.13.2.1.3	Test Requirements	3522

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3522

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3522

A.7.5.13.3.1.1	Test Purpose and Environment	3522

A.7.5.13.3.1.2	Test Parameters	3523

A.7.5.13.3.1.3	Test Requirements	3526

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3526

A.7.5.14.1	Test Purpose and Environment	3526

A.7.5.14.2	Test Requirements	3529

A.7.5.15	Void	3530

A.7.6	Measurement procedure	3530

A.7.6.1	Intra-frequency Measurements	3530

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3530

A.7.6.1.1.1	Test purpose and Environment	3530

A.7.6.1.1.2	Test Requirements	3533

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3533

A.7.6.1.2.1	Test purpose and Environment	3533

A.7.6.1.2.2	Test Requirements	3536

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3537

A.7.6.1.3.1	Test purpose and Environment	3537

A.7.6.1.3.2	Test Requirements	3540

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3540

A.7.6.1.4.1	Test purpose and Environment	3540

A.7.6.1.4.2	Test Requirements	3543

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasCA-Scell-r17	3544

A.7.6.1.5.1	Test purpose and Environment	3544

A.7.6.1.5.2	Test Requirements	3547

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3547

A.7.6.1.6.1	Test purpose and Environment	3547

A.7.6.1.6.2	Test Requirements	3550

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3551

A.7.6.1.7.1	Test purpose and Environment	3551

A.7.6.1.7.2	Test Requirements	3553

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3554

A.7.6.1.8.1	Test purpose and Environment	3554

A.7.6.1.8.2	Test Requirements	3557

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3558

A.7.6.1.9.1	Test purpose and Environment	3558

A.7.6.1.9.2	Test Requirements	3560

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3561

A.7.6.1.10.1	Test purpose and Environment	3561

A.7.6.1.10.2	Test Requirements	3564

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3565

A.7.6.1.11.1	Test purpose and Environment	3565

A.7.6.1.11.2	Test Requirements	3567

A.7.6.2	Inter-frequency Measurements	3568

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3568

A.7.6.2.1.1	Test Purpose and Environment	3568

A.7.6.2.1.2	Test Requirements	3571

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3571

A.7.6.2.2.1	Test Purpose and Environment	3571

A.7.6.2.2.2	Test Requirements	3575

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3575

A.7.6.2.3.1	Test Purpose and Environment	3575

A.7.6.2.3.2	Test Requirements	3579

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3579

A.7.6.2.4.1	Test Purpose and Environment	3579

A.7.6.2.4.2	Test Requirements	3583

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3583

A.7.6.2.5.1	Test Purpose and Environment	3583

A.7.6.2.5.2	Test Requirements	3587

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3588

A.7.6.2.6.1	Test Purpose and Environment	3588

A.7.6.2.6.2	Test Requirements	3592

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3593

A.7.6.2.7.1	Test Purpose and Environment	3593

A.7.6.2.7.2	Test Requirements	3597

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3598

A.7.6.2.8.1	Test Purpose and Environment	3598

A.7.6.2.8.2	Test Requirements	3602

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3603

A.7.6.2.9.1	Test Purpose and Environment	3603

A.7.6.2.9.2	Test Requirements	3607

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3607

A.7.6.2.10.1	Test Purpose and Environment	3607

A.7.6.2.10.2	Test Requirements	3609

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3609

A.7.6.2.11.1	Test Purpose and Environment	3609

A.7.6.2.11.2	Test Requirements	3612

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3612

A.7.6.2.12.1	Test Purpose and Environment	3612

A.7.6.2.12.2	Test Requirements	3616

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3617

A.7.6.2.13.1	Test Purpose and Environment	3617

A.7.6.2.13.2	Test Requirements	3621

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3622

A.7.6.2.14.1	Test Purpose and Environment	3622

A.7.6.2.14.2	Test Requirements	3626

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3627

A.7.6.2.15.1	Test Purpose and Environment	3627

A.7.6.2.15.2	Test Requirements	3631

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3632

A.7.6.2.16.1	Test Purpose and Environment	3632

A.7.6.2.16.2	Test Requirements	3638

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3638

A.7.6.2.17.1	Test Purpose and Environment	3638

A.7.6.2.17.2	Test Requirements	3645

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3646

A.7.6.2.18.1	Test Purpose and Environment	3646

A.7.6.2.18.2	Test Requirements	3652

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3652

A.7.6.2.19.1	Test Purpose and Environment	3652

A.7.6.2.19.2	Test Requirements	3660

A.7.6.3	L1-RSRP measurement for beam reporting	3661

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3661

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3661

A.7.6.3.1.1	Test Purpose and Environment	3661

A.7.6.3.1.2	Test parameters	3661

A.7.6.3.1.3	Test Requirements	3663

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3663

A.7.6.3.2.1	Test Purpose and Environment	3663

A.7.6.3.2.2	Test parameters	3664

A.7.6.3.2.3	Test Requirements	3666

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3666

A.7.6.3.3.1	Test Purpose and Environment	3666

A.7.6.3.3.2	Test parameters	3667

A.7.6.3.3.3	Test Requirements	3669

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3670

A.7.6.3.4.1	Test Purpose and Environment	3670

A.7.6.3.4.2	Test parameters	3670

A.7.6.3.3.3	Test Requirements	3672

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3673

A.7.6.3.5.1	Test Purpose and Environment	3673

A.7.6.3.5.2	Test parameters	3673

A.7.6.3.5.3	Test Requirements	3675

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3675

A.7.6.3.6.1	Test Purpose and Environment	3675

A.7.6.3.6.2	Test parameters	3676

A.7.6.3.6.3	Test Requirements	3679

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3679

A.7.6.3.7.1	Test Purpose and Environment	3679

A.7.6.3.7.2	Test parameters	3680

A.7.6.3.7.3	Test Requirements	3683

A.7.6.4	CLI measurements	3684

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3684

A.7.6.4.1.1	Test Purpose and Environment	3684

A.7.6.4.1.2	Test Parameters	3684

A.7.6.4.1.3	Test Requirements	3686

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3686

A.7.6.4.2.1	Test Purpose and Environment	3686

A.7.6.4.2.2	Test Parameters	3687

A.7.6.4.2.3	Test Requirements	3688

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3689

A.7.6.5.1.1	Test Purpose and Environment	3689

A.7.6.5.1.2	Test Requirements	3692

A.7.6.6	L1-SINR measurement for beam reporting	3692

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3694

A.7.6.6.2.1	Test Purpose and Environment	3694

A.7.6.6.2.2	Test parameters	3695

A.7.6.6.2.3	Test Requirements	3697

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3697

A.7.6.6.3.1	Test Purpose and Environment	3697

A.7.6.6.3.2	Test parameters	3698

A.7.6.6.3.3	Test Requirements	3700

A.7.6.7	CSI-RS based intra-frequency Measurements	3700

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3700

A.7.6.7.1.1	Test purpose and Environment	3700

A.7.6.7.1.2	Test Requirements	3703

A.7.6.8	CSI-RS based inter-frequency Measurements	3703

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3703

A.7.6.8.1.1	Test Purpose and Environment	3703

A.7.6.8.1.2	Test Requirements	3706

A.7.6.9	RSTD measurements	3706

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3706

A.7.6.9.1.1	Test Purpose and Environment	3706

A.7.6.9.1.2	Test Requirements	3714

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3714

A.7.6.9.2.1	Test Purpose and Environment	3714

A.7.6.9.2.2	Test Requirements	3721

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3721

A.7.6.9.3.1	Test Purpose and Environment	3721

A.7.6.9.3.2	Test Requirements	3727

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3728

A.7.6.9.4.1	Test Purpose and Environment	3728

A.7.6.9.4.2	Test Requirements	3731

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3731

A.7.6.9.5.1	Test Purpose and Environment	3731

A.7.6.9.5.2	Test Requirements	3735

A.7.6.10 PRS-RSRP measurements	3735

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3735

A.7.6.10.1.1	Test Purpose and Environment	3735

A.7.6.10.1.2	Test Requirements	3739

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3739

A.7.6.10.2.1	Test Purpose and Environment	3739

A.7.6.10.2.2	Test Requirements	3743

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3743

A.7.6.10.3.1	Test Purpose and Environment	3743

A.7.6.10.3.2	Test Requirements	3747

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3747

A.7.6.10.4.1	Test Purpose and Environment	3747

A.7.6.10.4.2	Test Requirements	3751

A.7.6.11	UE Rx-Tx time difference measurements	3751

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3751

A.7.6.11.1.1	Test purpose and environment	3751

A.7.6.11.1.2	Test requirements	3755

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3755

A.7.6.11.2.1	Test purpose and environment	3755

A.7.6.11.2.2	Test requirements	3759

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3759

A.7.6.11.3.1	Test purpose and environment	3759

A.7.6.11.3.2	Test requirements	3761

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3762

A.7.6.11.4.1	Test purpose and environment	3762

A.7.6.11.4.2	Test requirements	3764

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3765

A.7.6.11.5.1	Test purpose and environment	3765

A.7.6.11.5.2	Test requirements	3767

A.7.6.12	PRS-RSRPP measurements	3768

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3768

A.7.6.12.1.1	Test Purpose and Environment	3768

A.7.6.12.1.2	Test Requirements	3770

A.7.6.12.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3771

A.7.6.12.2.1	Test Purpose and Environment	3771

A.7.6.12.2.2	Test Requirements	3773

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3773

A.7.6.12.3.1	Test Purpose and Environment	3773

A.7.6.12.3.2	Test Requirements	3777

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3777

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3777

A.7.6.13.1.1	Test purpose and environment	3777

A.7.6.13.1.2	Test requirements	3779

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3780

A.7.6.13.2.1	Test purpose and environment	3780

A.7.6.13.2.2	Test requirements	3782

A.7.6.14	SA event triggered reporting tests with Pre-MG	3783

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3783

A.7.6.14.1.1	Test purpose and Environment	3783

A.7.6.14.1.2	Test parameters	3783

A.7.6.14.1.3	Test Requirements	3785

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3786

A.7.6.14.2.1	Test purpose and Environment	3786

A.7.6.14.2.2	Test parameters	3786

A.7.6.14.2.3	Test Requirements	3788

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3789

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3789

A.7.6.15.1.1	Test Purpose and Environment	3789

A.7.6.15.1.2	Test Requirements	3792

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3792

A.7.6.15.2.1	Test Purpose and Environment	3792

A.7.6.15.2.2	Test Requirements	3795

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overalpping scenario for SSB-based measurements and PRS-based measurement	3795

A.7.6.15.3.1	Test Purpose and Environment	3795

A.7.6.15.3.2	Test Requirements	3799

A.7.6.16	SA event triggered reporting tests with NCSG	3799

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3799

A.7.6.16.1.1	Test purpose and Environment	3799

A.7.6.16.1.2	Test Requirements	3803

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3803

A.7.6.16.2.1	Test Purpose and Environment	3803

A.7.6.16.2.2	Test Requirements	3806

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3806

A.7.6.16.3.1	Test Purpose and Environment	3806

A.7.6.16.3.2	Test Requirements	3809

A.7.7	Measurement Performance requirements	3809

A.7.7.1	SS-RSRP	3810

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3810

A.7.7.1.1.1	Test Purpose and Environment	3810

A.7.7.1.1.2	Test parameters	3810

A.7.7.1.1.3	Test Requirements	3814

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3814

A.7.7.1.2.1	Test Purpose and Environment	3814

A.7.7.1.2.2	Test parameters	3815

A.7.7.1.2.3	Test Requirements	3819

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3820

A.7.7.1.3.1	Test Purpose and Environment	3820

A.7.7.1.3.2	Test parameters	3820

A.7.7.1.3.3	Test Requirements	3824

A.7.7.2	SS-RSRQ	3824

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3824

A.7.7.2.1.1	Test Purpose and Environment	3824

A.7.7.2.1.2	Test Parameters	3824

A.7.7.2.1.3	Test Requirements	3826

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3826

A.7.7.2.2.1	Test Purpose and Environment	3826

A.7.7.2.2.2	Test Parameters	3826

A.7.7.2.2.3	Test Requirements	3828

A.7.7.3	SS-SINR	3828

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3828

A.7.7.3.1.1	Test Purpose and Environment	3828

A.7.7.3.1.2	Test Parameters	3828

A.7.7.3.1.3	Test Requirements	3830

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3830

A.7.7.3.2.1	Test Purpose and Environment	3830

A.7.7.3.2.2	Test Parameters	3830

A.7.7.3.2.3	Test Requirements	3832

A.7.7.4	L1-RSRP measurement for beam reporting	3832

A.7.7.4.1	SSB based L1-RSRP measurement	3832

A.7.7.4.1.1	Test Purpose and Environment	3832

A.7.7.4.1.2	Test parameters	3833

A.7.7.4.1.3	Test Requirements	3835

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3836

A.7.7.4.2.1	Test Purpose and Environment	3836

A.7.7.4.2.2	Test parameters	3836

A.7.7.4.2.3	Test Requirements	3838

A.7.7.5	CLI measurements	3839

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3839

A.7.7.5.1.1	Test Purpose and Environment	3839

A.7.7.5.1.2	Test parameters	3839

A.7.7.5.1.3	Test Requirements	3842

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3843

A.7.7.5.2.1	Test Purpose and Environment	3843

A.7.7.5.2.2	Test parameters	3843

A.7.7.5.2.3	Test Requirements	3845

A.7.7.6	L1-SINR measurement for beam reporting	3846

A.7.7.6.1.1	Test Purpose and Environment	3846

A.7.7.6.1.2	Test parameters	3846

A.7.7.6.1.3	Test Requirements	3848

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3849

A.7.7.6.2.1	Test Purpose and Environment	3849

A.7.7.6.2.2	Test parameters	3849

A.7.7.6.2.3	Test Requirements	3851

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3852

A.7.7.6.3.1	Test Purpose and Environment	3852

A.7.7.6.3.2	Test parameters	3852

A.7.7.6.3.3	Test Requirements	3854

A.7.7.7	CSI-RSRP	3855

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3855

A.7.7.7.1.1	Test Purpose and Environment	3855

A.7.7.7.1.2	Test parameters	3855

A.7.7.7.1.3	Test Requirements	3858

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3859

A.7.7.7.2.1	Test Purpose and Environment	3859

A.7.7.7.2.2	Test parameters	3859

A.7.7.7.2.3	Test Requirements	3862

A.7.7.8	CSI-RSRQ	3863

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3863

A.7.7.8.1.1	Test Purpose and Environment	3863

A.7.7.8.1.2	Test Parameters	3863

A.7.7.8.1.3	Test Requirements	3865

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3865

A.7.7.8.2.1	Test Purpose and Environment	3865

A.7.7.8.2.2	Test Parameters	3865

A.7.7.8.2.3	Test Requirements	3867

A.7.7.9	CSI-SINR	3867

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3867

A.7.7.9.1.1	Test Purpose and Environment	3867

A.7.7.9.1.2	Test Parameters	3867

A.7.7.9.1.3	Test Requirements	3869

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3869

A.7.7.9.2.1	Test Purpose and Environment	3869

A.7.7.9.2.2	Test Parameters	3870

A.7.7.9.2.3	Test Requirements	3872

A.7.7.10	RSTD measurements	3872

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3872

A.7.7.10.1.1	Test purpose and Environment	3872

A.7.7.10.1.2	Test Requirements	3875

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3875

A.7.7.10.2.1	Test purpose and Environment	3875

A.7.7.10.2.2	Test Requirements	3879

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3880

A.7.7.10.3.1	Test purpose and Environment	3880

A.7.7.10.3.2	Test Requirements	3881

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3882

A.7.7.10.4.1	Test purpose and Environment	3882

A.7.7.10.4.2	Test Requirements	3884

A.7.7.11	PRS-RSRP measurements	3884

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3884

A.7.7.11.1.1	Test Purpose and Environment	3884

A.7.7.11.1.2	Test parameters	3884

A.7.7.11.1.3	Test Requirements	3888

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3889

A.7.7.11.2.1	Test Purpose and Environment	3889

A.7.7.11.2.2	Test parameters	3889

A.7.7.11.2.3	Test Requirements	3891

A.7.7.12	UE Rx-Tx time difference measurements	3892

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3892

A.7.7.12.1.1	Test purpose and environment	3892

A.7.7.12.1.2	Test parameters	3892

A.7.7.12.1.3	Test requirements	3895

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3895

A.7.7.12.2.1	Test purpose and environment	3895

A.7.7.12.2.2	Test parameters	3896

A.7.7.12.2.3	Test requirements	3899

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3899

A.7.7.12.3.1	Test purpose and environment	3899

A.7.7.12.3.2	Test parameters	3899

A.7.7.12.3.3	Test requirements	3902

A.7.7.13	PRS-RSRPP measurements	3902

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3902

A.7.7.13.1.1	Test Purpose and Environment	3902

A.7.7.13.1.2	Test parameters	3902

A.7.7.13.1.3	Test Requirements	3905

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3906

A.7.7.13.2.1	Test Purpose and Environment	3906

A.7.7.13.2.2	Test parameters	3906

A.7.7.13.2.3	Test Requirements	3908

A.7.8	Measurement procedure in RRC_INACTIVE	3909

A.7.8.1	RSTD measurements	3909

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3909

A.7.8.1.1.1	Test Purpose and Environment	3909

A.7.8.1.1.2	Test Requirements	3912

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3912

A.7.8.1.2.1	Test Purpose and Environment	3912

A.7.8.1.2.2	Test Requirements	3918

A.7.8.2	PRS-RSRP measurements	3919

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3919

A.7.8.2.1.1	Test Purpose and Environment	3919

A.7.8.2.1.2	Test Requirements	3923

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3923

A.7.8.2.2.1	Test purpose and Environment	3923

A.7.8.2.2.2	Test Requirements	3927

A.7.8.3	UE Rx-Tx time difference measurements	3927

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3927

A.7.8.3.1.1	Test purpose and environment	3927

A.7.8.3.1.2	Test requirements	3931

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3931

A.7.8.3.2.1	Test purpose and environment	3931

A.7.8.3.2.2	Test requirements	3933

A.7.8.4	PRS-RSRPP measurements	3934

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3934

A.7.8.4.1.1	Test Purpose and Environment	3934

A.7.8.4.1.2	Test Requirements	3936

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3936

A.7.8.4.2.1	Test Purpose and Environment	3936

A.7.8.4.2.2	Test Requirements	3939

A.7.9	Measurement performance requirements in RRC_INACTIVE	3939

A.7.9.1	RSTD measurements	3939

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3939

A.7.9.1.1.1	Test purpose and Environment	3939

A.7.9.1.1.2	Test Requirements	3941

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3941

A.7.9.1.2.1	Test purpose and Environment	3941

A.7.9.1.2.2	Test Requirements	3943

A.7.9.2	PRS-RSRP measurements	3943

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3943

A.7.9.2.1.1	Test Purpose and Environment	3943

A.7.9.2.1.2	Test parameters	3943

A.7.9.2.1.3	Test Requirements	3945

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3946

A.7.9.2.2.1	Test Purpose and Environment	3946

A.7.9.2.2.2	Test parameters	3946

A.7.9.2.2.3	Test Requirements	3948

A.7.9.3	UE Rx-Tx time difference measurements	3949

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3949

A.7.9.3.1.1	Test purpose and environment	3949

A.7.9.3.1.2	Test parameters	3949

A.7.9.3.1.3	Test requirements	3951

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3951

A.7.9.3.2.1	Test purpose and environment	3951

A.7.9.3.2.2	Test parameters	3951

A.7.9.3.2.3	Test requirements	3954

A.7.9.4	PRS-RSRPP measurements	3954

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3954

A.7.9.4.1.1	Test Purpose and Environment	3954

A.7.9.4.1.2	Test parameters	3954

A.7.9.4.1.3	Test Requirements	3956

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3957

A.7.9.4.2.1	Test Purpose and Environment	3957

A.7.9.4.2.2	Test parameters	3957

A.7.9.4.2.3	Test Requirements	3959

A.8	E-UTRA standalone tests for NR RRM	3961

A.8.1	Void	3961

A.8.2	RRC_IDLE state mobility	3961

A.8.2.1	Inter-RAT NR Cell re-selection	3961

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3961

A.8.2.1.1.1	Test Purpose and Environment	3961

A.8.2.1.1.2	Test Requirements	3966

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3967

A.8.2.1.2.1	Test Purpose and Environment	3967

A.8.2.1.2.2	Test Requirements	3971

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3972

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	3972

A.8.2.2.1.1	Test Purpose and Environment	3972

A.8.2.2.1.2	Test Requirements	3976

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	3976

A.8.2.2.2.1	Test Purpose and Environment	3976

A.8.2.2.2.2	Test Requirements	3979

A.8.3	RRC_CONNECTED state mobility	3980

A.8.3.1	Handover	3980

A.8.3.1.1	E-UTRAN - NR handover in FR1	3980

A.8.3.1.1.1	Test Purpose and Environment	3980

A.8.3.1.1.2	Test Requirements	3985

A.8.4	Measurement procedure	3985

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3985

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3985

A.8.4.1.1.1	Test Purpose and Environment	3985

A.8.4.1.1.2	Test Requirements	3987

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3988

A.8.4.1.2.1	Test Purpose and Environment	3988

A.8.4.1.2.2	Test Requirements	3989

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3989

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3989

A.8.4.2.1.1	Test Purpose and Environment	3989

A.8.4.2.1.2	Test Requirements	3994

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3994

A.8.4.2.2.1	Test Purpose and Environment	3994

A.8.4.2.2.2	Test Requirements	3998

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3998

A.8.4.2.3.1	Test Purpose and Environment	3998

A.8.4.2.3.2	Test Requirements	4003

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4003

A.8.4.2.4.1	Test Purpose and Environment	4003

A.8.4.2.4.2	Test Requirements	4007

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4007

A.8.4.2.5.1	Test Purpose and Environment	4007

A.8.4.2.5.2	Test Requirements	4009

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4010

A.8.4.2.6.1	Test Purpose and Environment	4010

A.8.4.2.6.2	Test Requirements	4012

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4013

A.8.4.2.7.1	Test Purpose and Environment	4013

A.8.4.2.7.2	Test Requirements	4015

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4016

A.8.4.2.8.1	Test Purpose and Environment	4016

A.8.4.2.8.2	Test Requirements	4018

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	4019

A.8.4.2.9.1	Test Purpose and Environment	4019

A.8.4.2.9.2	Test Requirements	4023

A.8.5	Measurement performance	4023

A.8.5.1	SFTD accuracy	4023

A.8.5.1.1	SFTD accuracy	4023

A.8.5.1.1.1	Test Purpose	4023

A.8.5.1.1.2	Test Environment	4024

A.8.5.1.1.3	Test Requirements	4030

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	4030

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4030

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4035

A.8.5.2.1.2.1	Test Purpose and Environment	4035

A.8.5.2.1.2.2	Test Parameters	4035

A.8.5.2.1.2.3	Test Requirements	4037

A.8.5.2.2	SS-RSRQ	4037

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4037

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4042

A.8.5.2.2.2.1	Test Purpose and Environment	4042

A.8.5.2.2.2.2	Test Parameters	4042

A.8.5.2.2.2.3	Test Requirements	4044

A.8.5.2.3	SS-SINR	4044

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4044

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4048

A.8.5.2.3.2.1	Test Purpose and Environment	4048

A.8.5.2.3.2.2	Test Parameters	4048

A.8.5.2.3.2.3	Test Requirements	4050

A.9	V2X Tests	4051

A.9.1	V2X Tests in FR1	4051

A.9.1.1	Test for V2X UE Transmit Timing	4051

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	4051

A.9.1.1.1.1	Test Purpose and Environment	4051

A.9.1.1.1.2	Test requirements	4051

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	4051

A.9.1.1.2.1	Test Purpose and Environment	4051

A.9.1.1.2.2	Test requirements	4052

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4052

A.9.1.1.3.1	Test Purpose and Environment	4052

A.9.1.1.3.2	Test requirements	4055

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4055

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4055

A.9.1.2.1.1	Test Purpose and Environment	4055

A.9.1.2.1.2	Test Requirements	4058

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4058

A.9.1.2.2.1	Test Purpose and Environment	4058

A.9.1.2.2.2	Test Requirements	4060

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4060

A.9.1.2.3.1	Test Purpose and Environment	4060

A.9.1.2.3.2	Test Requirements	4062

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4062

A.9.1.3.1	 Test for GNSS configured as the highest priority	4062

A.9.1.3.1.1	Test Purpose and Environment	4062

A.9.1.3.1.2	Test Requirements	4064

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4065

A.9.1.3.2.1	Test Purpose and Environment	4065

A.9.1.3.2.2	Test Requirements	4067

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4068

A.9.1.3.3.1	Test Purpose and Environment	4068

A.9.1.3.3.2	Test Requirements	4070

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4071

A.9.1.3.4.1	Test Purpose and Environment	4071

A.9.1.3.4.2	Test Requirements	4073

A.9.1.4	Test for L1 SL-RSRP Measurement	4074

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4074

A.9.1.4.1.1	Test Purpose and Environment	4074

A.9.1.4.1.2	Test Requirements	4077

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4077

A.9.1.4.2.1	Test Purpose and Environment	4077

A.9.1.4.2.2	Test Requirements	4081

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4081

A.9.1.4.3.1	Test Purpose and Environment	4081

A.9.1.4.3.2	Test Requirements	4088

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4088

A.9.1.4.4.1	Test Purpose and Environment	4088

A.9.1.4.4.2	Test Requirements	4092

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4092

A.9.1.4.5.1	Test Purpose and Environment	4092

A.9.1.4.5.2	Test Requirements	4095

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4095

A.9.1.4.6.1	Test Purpose and Environment	4095

A.9.1.4.6.2	Test Requirements	4099

A.9.1.5	Test for Congestion Control Measurement	4099

A.9.1.5.1	Test Purpose and Environment	4099

A.9.1.5.2	Test Requirements	4105

A.9.1.6	Test for Interruption	4105

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4105

A.9.1.6.1.1	Test Purpose and Environment	4105

A.9.1.6.1.2	Test Requirements	4108

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4108

A.9.1.6.2.1	Test Purpose and Environment	4108

A.9.1.6.2.2	Test Requirements	4112

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4112

A.9.1.6.3.1	Test Purpose and Environment	4112

A.9.1.6.3.2	Test Requirements	4115

A.9.1.7	Selection / Reselection of relay UE	4115

A.9.1.7.1	Test Purpose and Environment	4115

A.9.1.7.2	Test Requirements	4121

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4122

A.10.1	RRC_CONNECTED state mobility	4122

A.10.1.1	RRC connection mobility control	4122

A.10.1.1.1	Random Access	4122

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4122

A.10.1.1.1.1.1	Test Purpose and Environment	4122

A.10.1.1.1.1.2	Test Requirements	4125

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4125

A.10.1.1.1.1.2.2	Random Access Response Reception	4125

A.10.1.1.1.1.2.3	No Random Access Response Reception	4126

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4126

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4126

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4127

A.10.1.1.1.2.1	Test Purpose and Environment	4127

A.10.1.1.1.2.2	Test Requirements	4130

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4130

A.10.1.1.1.2.2.2	Random Access Response Reception	4131

A.10.1.1.1.2.2.3	No Random Access Response Reception	4131

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4131

A.10.1.1.1.3.1	Test Purpose and Environment	4131

A.10.1.1.1.3.2	Test Requirements	4135

A.10.1.1.1.3.2.1	MsgA Transmission	4135

A.10.1.1.1.3.2.2	MsgB Reception	4135

A.10.1.1.1.3.2.3	No MsgB Reception	4136

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4136

A.10.1.1.1.4.1	Test Purpose and Environment	4136

A.10.1.1.1.4.2	Test Requirements	4140

A.10.1.1.1.4.2.1	MsgA Transmission	4140

A.10.1.1.1.4.2.2	MsgB Reception	4141

A.10.1.1.1.4.2.3	No MsgB Reception	4141

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4141

A.10.1.2.1	Test Purpose and Environment	4141

A.10.1.2.2	Test Requirements	4147

A.10.2	Timing	4148

A.10.2.1	UE transmit timing	4148

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4148

A.10.2.1.1.1	Test Purpose and environment	4148

A.10.2.1.1.2	Test requirements	4151

A.10.2.2	UE timing advance	4152

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4152

A.10.2.2.1.1	Test Purpose and Environment	4152

A.10.2.2.1.2	Test Parameters	4152

A.10.2.2.1.3	Test Requirements	4156

A.10.3	Signalling characteristics	4156

A.10.3.1	Radio link monitoring	4156

A.10.3.1.1	Introduction	4156

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4157

A.10.3.1.2.1	Test purpose and environment	4157

A.10.3.1.2.2	Test requirements	4162

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4162

A.10.3.1.3.1	Test purpose and environment	4162

A.10.3.1.3.2	Test requirements	4168

A.10.3.1.4	Void	4168

A.10.3.1.4.1	Void	4168

A.10.3.1.4.2	Void	4168

A.10.3.1.5	Void	4168

A.10.3.1.5.1	Void	4168

A.10.3.1.5.2	Void	4168

A.10.3.2	Interruption	4168

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	4168

A.10.3.2.1.1	Test Purpose and Environment	4168

A.10.3.2.1.2	Test Requirements	4173

A.10.3.3	SCell activation and deactivation delay	4173

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4173

A.10.3.3.1.1	Test Purpose and Environment	4173

A.10.3.3.1.2	Test Requirements	4178

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4178

A.10.3.3.2.1	Test Purpose and Environment	4178

A.10.3.3.2.2	Test Requirements	4179

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4179

A.10.3.3.3.1	Test Purpose and Environment	4179

A.10.3.3.3.2	Test Requirements	4180

A.10.3.4	Beam failure detection and link recovery procedures	4180

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4180

A.10.3.4.1.1	Test Purpose and Environment	4180

A.10.3.4.1.2	Test Requirements	4185

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4185

A.10.3.4.2.1	Test Purpose and Environment	4185

A.10.3.4.2.2	Test Requirements	4191

A.10.3.5	Active BWP switching	4191

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4191

A.10.3.5.1.2	Test Requirements	4196

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4197

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4197

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4200

A.10.3.5.3	RRC-based Active BWP Switch	4204

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4204

A.10.3.6	PSCell addition and release delay	4208

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4208

A.10.3.6.1.1	Test purpose and environment	4208

A.10.3.6.1.2	Test Requirements	4213

A.10.3.7	Void	4214

A.10.4	Measurement procedure	4214

A.10.4.1	Intra-frequency measurements	4214

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4214

A.10.4.1.1.1	Test purpose and environment	4214

A.10.4.1.1.2	Test parameters	4214

A.10.4.1.1.3	Test Requirements	4214

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	4215

A.10.4.1.2.1	Test purpose and environment	4215

A.10.4.1.2.2	Test parameters	4215

A.10.4.1.2.3	Test Requirements	4215

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	4215

A.10.4.1.3.1	Test purpose and environment	4215

A.10.4.1.3.2	Test parameters	4215

A.10.4.1.3.3	Test Requirements	4215

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4216

A.10.4.1.4.1	Test purpose and environment	4216

A.10.4.1.4.2	Test parameters	4216

A.10.4.1.4.3	Test Requirements	4216

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4216

A.10.4.1.5.1	Test purpose and environment	4216

A.10.4.1.5.2	Test parameters	4216

A.10.4.1.5.3	Test Requirements	4216

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4217

A.10.4.1.6.1	Test purpose and environment	4217

A.10.4.1.6.2	Test parameters	4217

A.10.4.1.6.3	Test Requirements	4217

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4217

A.10.4.1.7.1	Test purpose and environment	4217

A.10.4.1.7.2	Test parameters	4217

A.10.4.1.7.3	Test Requirements	4217

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4218

A.10.4.1.8.1	Test purpose and environment	4218

A.10.4.1.8.2	Test parameters	4218

A.10.4.1.8.3	Test Requirements	4218

A.10.4.1.9	RSSI measurement reporting on PSCC	4218

A.10.4.1.9.1	Test purpose and environment	4218

A.10.4.1.9.2	Test parameters	4218

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	4219

A.10.4.1.10.1	Test purpose and environment	4219

A.10.4.1.10.2	Test parameters	4219

A.10.4.1.11	RSSI measurement reporting on SCC	4219

A.10.4.1.11.1	Test purpose and environment	4219

A.10.4.1.11.2	Test parameters	4219

A.10.4.1.12	Channel occupancy measurement reporting on SCC	4220

A.10.4.1.12.1	Test purpose and environment	4220

A.10.4.1.12.2	Test parameters	4220

A.10.4.2	Inter-frequency measurements	4220

A.10.4.2.1	RSSI measurement reporting	4220

A.10.4.2.1.1	Test purpose and environment	4220

A.10.4.2.1.2	Test parameters	4220

A.10.4.2.2	Channel occupancy measurement reporting	4221

A.10.4.2.2.1	Test purpose and environment	4221

A.10.4.2.2.2	Test parameters	4221

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4221

A.10.4.2.3.1	Test Purpose and Environment	4221

A.10.4.2.3.2	Test Requirements	4226

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4227

A.10.4.2.4.1	Test Purpose and Environment	4227

A.10.4.2.4.2	Test Requirements	4231

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4232

A.10.4.2.5.1	Test Purpose and Environment	4232

A.10.4.2.5.2	Test Requirements	4236

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4237

A.10.4.2.6.1	Test Purpose and Environment	4237

A.10.4.2.6.2	Test Requirements	4241

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4242

A.10.4.2.7.1	Test Purpose and Environment	4242

A.10.4.2.7.2	Test Requirements	4248

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4248

A.10.4.2.8.1	Test Purpose and Environment	4248

A.10.4.2.8.2	Test Requirements	4254

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4255

A.10.4.2.9.1	Test Purpose and Environment	4255

A.10.4.2.9.2	Test Requirements	4260

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4260

A.10.4.2.10.1	Test Purpose and Environment	4260

A.10.4.2.10.2	Test Requirements	4266

A.10.4.3	L1-RSRP measurements for beam reporting	4267

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4267

A.10.4.3.1.1	Test Purpose and Environment	4267

A.10.4.3.1.2	Test parameters	4267

A.10.4.3.1.3	Test Requirements	4269

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4270

A.10.4.3.2.1	Test Purpose and Environment	4270

A.10.4.3.2.2	Test parameters	4270

A.10.4.3.2.3	Test Requirements	4272

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4273

A.10.4.3.3.1	Test Purpose and Environment	4273

A.10.4.3.3.2	Test parameters	4273

A.10.4.3.3.3	Test Requirements	4276

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4277

A.10.4.3.4.1	Test Purpose and Environment	4277

A.10.4.3.4.2	Test parameters	4277

A.10.4.3.4.3	Test Requirements	4280

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4281

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4281

A.10.4.4.1.1	Test Purpose and Environment	4281

A.10.4.4.1.2	Test Requirements	4285

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4286

A.10.4.4.2.1	Test Purpose and Environment	4286

A.10.4.4.2.2	Test Requirements	4291

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4292

A.10.4.4.3.1	Test Purpose and Environment	4292

A.10.4.4.3.2	Test Requirements	4296

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4297

A.10.4.4.4.1	Test Purpose and Environment	4297

A.10.4.4.4.2	Test Requirements	4302

A.10.5	Measurement performance	4303

A.10.5.1	SS-RSRP	4303

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4303

A.10.5.1.1.1	Test Purpose and Environment	4303

A.10.5.1.1.2	Test parameters	4303

A.10.5.1.1.3	Test Requirements	4306

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4306

A.10.5.1.2.1	Test Purpose and Environment	4306

A.10.5.1.2.2	Test parameters	4306

A.10.5.1.2.3	Test Requirements	4309

A.10.5.2	SS-RSRQ	4309

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4309

A.10.5.2.1.1	Test Purpose and Environment	4309

A.10.5.2.1.2	Test Parameters	4309

A.10.5.2.1.3	Test Requirements	4312

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4312

A.10.5.2.2.1	Test Purpose and Environment	4312

A.10.5.2.2.2	Test Parameters	4312

A.10.5.2.2.3	Test Requirements	4315

A.10.5.3	SS-SINR	4315

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4315

A.10.5.3.1.1	Test Purpose and Environment	4315

A.10.5.3.1.2	Test Parameters	4315

A.10.5.3.1.3	Test Requirements	4318

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4318

A.10.5.3.2.1	Test Purpose and Environment	4318

A.10.5.3.2.2	Test Parameters	4318

A.10.5.3.2.3	Test Requirements	4321

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4321

A.10.5.3.3.1	Test Purpose and Environment	4321

A.10.5.3.3.2	Test Parameters	4321

A.10.5.3.3.3	Test Requirements	4324

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4324

A.10.5.4.1	SSB based L1-RSRP measurement	4324

A.10.5.4.1.1	Test Purpose and Environment	4324

A.10.5.4.1.2	Test parameters	4325

A.10.5.4.1.3	Test Requirements	4327

A.10.5.5	RSSI	4327

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4327

A.10.5.5.1.1	Test Purpose and Environment	4327

A.10.5.5.1.2	Test parameters	4327

A.10.5.5.1.3	Test Requirements	4330

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4330

A.10.5.5.2.1	Test Purpose and Environment	4330

A.10.5.5.2.2	Test parameters	4330

A.10.5.5.2.3	Test Requirements	4333

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4333

A.10.5.5.3.1	Test Purpose and Environment	4333

A.10.5.5.3.2	Test parameters	4333

A.10.5.5.3.3	Test Requirements	4336

A.10.5.6	Channel occupancy	4336

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4336

A.10.5.6.1.1	Test Purpose and Environment	4336

A.10.5.6.1.2	Test parameters	4336

A.10.5.6.1.3	Test Requirements	4340

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4340

A.10.5.6.2.1	Test Purpose and Environment	4340

A.10.5.6.2.2	Test parameters	4340

A.10.5.6.2.3	Test Requirements	4343

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4343

A.10.5.6.3.1	Test Purpose and Environment	4343

A.10.5.6.3.2	Test parameters	4343

A.10.5.6.3.3	Test Requirements	4347

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4350

A.11.1	RRC_IDLE state mobility	4350

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4350

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4350

A.11.1.1.1.1	Test Purpose and Environment	4350

A.11.1.1.1.2	Test Parameters	4350

A.11.1.1.1.3	Test Requirements	4353

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4353

A.11.1.1.2.1	Test Purpose and Environment	4353

A.11.1.1.2.2	Test Parameters	4353

A.11.1.1.2.3	Test Requirements	4357

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4357

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4357

A.11.1.2.1.1	Test Purpose and Environment	4357

A.11.1.2.1.2	Test Parameters	4358

A.11.1.2.1.3	Test Requirements	4364

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4365

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4365

A.11.1.3.1.1	Test Purpose and Environment	4365

A.11.1.3.1.2	Test Parameters	4365

A.11.1.3.1.3	Test Requirements	4371

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4372

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4372

A.11.1.4.1.1	Test Purpose and Environment	4372

A.11.1.4.1.2	Test Parameters	4372

A.11.1.4.1.3	Test Requirements	4375

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4376

A.11.1.4.2.1	Test Purpose and Environment	4376

A.11.1.4.2.2	Test Requirements	4379

A.11.2	RRC_CONNECTED state mobility	4380

A.11.2.1	Handover	4380

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4380

A.11.2.1.1.1	Test Purpose and Environment	4380

A.11.2.1.1.2	Test Parameters	4380

A.11.2.1.1.3 Test Requirements	4383

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4383

A.11.2.1.2.1	Test Purpose and Environment	4383

A.11.2.1.2.2	Test Parameters	4383

A.11.2.1.2.3	Test Requirements	4386

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4386

A.11.2.1.3.1	Test Purpose and Environment	4386

A.11.2.1.3.2	Test Parameters	4386

A.11.2.1.3.3 Test Requirements	4389

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4389

A.11.2.1.4.1	Test Purpose and Environment	4389

A.11.2.1.4.2	Test Parameters	4389

A.11.2.1.4.3 Test Requirements	4393

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4394

A.11.2.1.5.1	Test Purpose and Environment	4394

A.11.2.1.5.2	Test Parameters	4394

A.11.2.1.5.3 Test Requirements	4397

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4398

A.11.2.1.6.1	Test Purpose and Environment	4398

A.11.2.1.6.2	Test Parameters	4398

A.11.2.1.6.3 Test Requirements	4401

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4402

A.11.2.1.7.1	Test Purpose and Environment	4402

A.11.2.1.7.2	Test Requirements	4408

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4408

A.11.2.1.8.1	Test Purpose and Environment	4408

A.11.2.1.8.2	Test Requirements	4413

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4413

A.11.2.1.9.1	Test Purpose and Environment	4413

A.11.2.1.9.2	Test Requirements	4422

A.11.2.2	RRC connection mobility control	4423

A.11.2.2.1	RRC re-establishment	4423

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4423

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4426

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4433

A.11.2.2.2	Random Access	4438

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4438

A.11.2.2.2.1.1	Test Purpose and Environment	4438

A.11.2.2.2.1.2	Test Requirements	4441

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4441

A.11.2.2.2.1.2.2	Random Access Response Reception	4441

A.11.2.2.2.1.2.3	No Random Access Response Reception	4442

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4442

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4442

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4443

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4443

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4443

A.11.2.2.2.2.1	Test Purpose and Environment	4443

A.11.2.2.2.2.2	Test Requirements	4446

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4446

A.11.2.2.2.2.2.2	Random Access Response Reception	4447

A.11.2.2.2.2.2.3	No Random Access Response Reception	4447

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4447

A.11.2.2.2.3.1	Test Purpose and Environment	4447

A.11.2.2.2.3.2	Test Requirements	4451

A.11.2.2.2.3.2.1	MsgA Transmission	4451

A.11.2.2.2.3.2.2	MsgB Reception	4451

A.11.2.2.2.3.2.3	No MsgB Reception	4452

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4452

A.11.2.2.2.4.1	Test Purpose and Environment	4452

A.11.2.2.2.4.2	Test Requirements	4456

A.11.2.2.2.4.2.1	MsgA Transmission	4456

A.11.2.2.2.4.2.2	MsgB Reception	4457

A.11.2.2.2.4.2.3	No MsgB Reception	4457

A.11.2.2.3	RRC connection release with redirection	4458

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4458

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4462

A.11.3	Timing	4467

A.11.3.1	UE transmit timing	4467

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4467

A.11.3.1.1.1	Test Purpose and environment	4467

A.11.3.1.1.2	Test requirements	4470

A.11.3.2	UE timing advance	4471

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4471

A.11.3.2.1.1	Test Purpose and Environment	4471

A.11.3.2.1.2	Test Parameters	4471

A.11.3.2.1.3	Test Requirements	4475

A.11.4	Signalling characteristics	4475

A.11.4.1	Radio link monitoring	4475

A.11.4.1.1	Introduction	4475

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4476

A.11.4.1.2.1	Test purpose and environment	4476

A.11.4.1.2.2	Test requirements	4480

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4480

A.11.4.1.3.1	Test purpose and environment	4480

A.11.4.1.3.2	Test requirements	4486

A.11.4.1.4	Void	4486

A.11.4.1.4.1	Void	4486

A.11.4.1.4.2	Void	4486

A.11.4.1.5	Void	4486

A.11.4.1.5.1	Void	4486

A.11.4.1.5.2	Void	4486

A.11.4.2	Interruption	4487

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4487

A.11.4.2.1.1 Test Purpose and Environment	4487

A.11.4.2.1.2	Test Requirements	4491

A.11.4.3	SCell activation and deactivation delay	4491

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4491

A.11.4.3.1.1	Test Purpose and Environment	4491

A.11.4.3.1.2	Test Requirements	4496

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4496

A.11.4.3.2.1	Test Purpose and Environment	4496

A.11.4.3.2.2	Test Requirements	4497

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4497

A.11.4.3.3.1	Test Purpose and Environment	4497

A.11.4.3.3.2	Test Requirements	4498

A.11.4.4	Beam failure detection and link recovery procedures	4498

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4498

A.11.4.4.1.1	Test Purpose and Environment	4498

A.11.4.4.1.2	Test Requirements	4503

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4503

A.11.4.4.2.1	Test Purpose and Environment	4503

A.11.4.4.2.2	Test Requirements	4509

A.11.4.5	Active BWP switching	4509

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4509

A.11.4.5.1.1	Test Purpose and Environment	4509

A.11.4.5.1.2	Test Requirements	4513

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4514

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4514

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4517

A.11.4.5.3	RRC-based Active BWP Switch	4520

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4520

A.11.4.6	Void	4523

A.11.5	Measurement procedure	4523

A.11.5.1	Intra-frequency measurements	4523

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4523

A.11.5.1.1.1	Test purpose and environment	4523

A.11.5.1.1.2	Test parameters	4523

A.11.5.1.1.3	Test Requirements	4527

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4527

A.11.5.1.2.1	Test purpose and environment	4527

A.11.5.1.2.2	Test parameters	4527

A.11.5.1.2.3	Test Requirements	4531

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4531

A.11.5.1.3.1	Test purpose and environment	4531

A.11.5.1.3.2	Test parameters	4531

A.11.5.1.3.3	Test Requirements	4531

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4532

A.11.5.1.4.1	Test purpose and environment	4532

A.11.5.1.4.2	Test parameters	4532

A.11.5.1.4.3	Test Requirements	4532

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4532

A.11.5.1.5.1	Test purpose and environment	4532

A.11.5.1.5.2	Test parameters	4532

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4533

A.11.5.1.6.1	Test purpose and environment	4533

A.11.5.1.6.2	Test parameters	4533

A.11.5.1.6.3	Test Requirements	4533

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4533

A.11.5.1.7.1	Test purpose and environment	4533

A.11.5.1.7.2	Test parameters	4533

A.11.5.1.7.3	Test Requirements	4533

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4534

A.11.5.1.8.1	Test purpose and environment	4534

A.11.5.1.8.2	Test parameters	4534

A.11.5.1.8.3	Test Requirements	4534

A.11.5.1.9	RSSI measurement reporting on PCC	4534

A.11.5.1.9.1	Test purpose and environment	4534

A.11.5.1.9.2	Test parameters	4534

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4535

A.11.5.1.10.1	Test purpose and environment	4535

A.11.5.1.10.2	Test parameters	4535

A.11.5.1.11	RSSI measurement reporting on SCC	4535

A.11.5.1.11.1	Test purpose and environment	4535

A.11.5.1.11.2	Test parameters	4535

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4536

A.11.5.1.12.1	Test purpose and environment	4536

A.11.5.1.12.2	Test parameters	4536

A.11.5.2	Inter-frequency measurements	4536

A.11.5.2.1	RSSI measurement reporting	4536

A.11.5.2.1.1	Test purpose and environment	4536

A.11.5.2.1.2	Test parameters	4536

A.11.5.2.2	Channel occupancy measurement reporting	4537

A.11.5.2.2.1	Test purpose and environment	4537

A.11.5.2.2.2	Test parameters	4537

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4537

A.11.5.2.3.1	Test Purpose and Environment	4537

A.11.5.2.3.2	Test Requirements	4541

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4542

A.11.5.2.4.1	Test Purpose and Environment	4542

A.11.5.2.4.2	Test Requirements	4547

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4547

A.11.5.2.5.1	Test Purpose and Environment	4547

A.11.5.2.5.2	Test Requirements	4551

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4552

A.11.5.2.6.1	Test Purpose and Environment	4552

A.11.5.2.6.2	Test Requirements	4557

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4557

A.11.5.2.7.1	Test Purpose and Environment	4557

A.11.5.2.7.2	Test Requirements	4561

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4562

A.11.5.2.8.1	Test Purpose and Environment	4562

A.11.5.2.8.2	Test Requirements	4567

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4567

A.11.5.2.9.1	Test Purpose and Environment	4567

A.11.5.2.9.2	Test Requirements	4572

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4573

A.11.5.2.10.1	Test Purpose and Environment	4573

A.11.5.2.10.2	Test Requirements	4578

A.11.5.3	Inter-RAT E-UTRAN measurements	4579

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4579

A.11.5.3.1.1	Test Purpose and Environment	4579

A.11.5.3.1.2	Test Requirements	4583

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4583

A.11.5.3.2.1	Test Purpose and Environment	4583

A.11.5.3.2.2	Test Requirements	4588

A.11.5.4	L1-RSRP measurements for beam reporting	4588

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4588

A.11.5.4.1.1	Test Purpose and Environment	4588

A.11.5.4.1.2	Test parameters	4588

A.11.5.4.1.3	Test Requirements	4590

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4591

A.11.5.4.2.1	Test Purpose and Environment	4591

A.11.5.4.2.2	Test parameters	4591

A.11.5.4.2.3	Test Requirements	4593

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4594

A.11.5.4.3.1	Test Purpose and Environment	4594

A.11.5.4.3.2	Test parameters	4594

A.11.5.4.3.3	Test Requirements	4597

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4598

A.11.5.4.4.1	Test Purpose and Environment	4598

A.11.5.4.4.2	Test parameters	4598

A.11.5.4.4.3	Test Requirements	4601

A.11.6	Measurement performance	4602

A.11.6.1	SS-RSRP	4602

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4602

A.11.6.1.1.1	Test Purpose and Environment	4602

A.11.6.1.1.2	Test parameters	4602

A.11.6.1.1.3	Test Requirements	4604

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4604

A.11.6.1.2.1	Test Purpose and Environment	4604

A.11.6.1.2.2	Test parameters	4604

A.11.6.1.2.3	Test Requirements	4606

A.11.6.2	SS-RSRQ	4606

A.11.6.2.1	Intra-frequency measurement accuracy	4606

A.11.6.2.1.1	Test Purpose and Environment	4606

A.11.6.2.1.2	Test Parameters	4606

A.11.6.2.1.3	Test Requirements	4609

A.11.6.2.2	Inter-frequency measurement accuracy	4609

A.11.6.2.2.1	Test Purpose and Environment	4609

A.11.6.2.2.2	Test Parameters	4609

A.11.6.2.2.3	Test Requirements	4612

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4612

A.11.6.2.3.1	Test Purpose and Environment	4612

A.11.6.2.3.2	Test Parameters	4612

A.11.6.2.3.3	Test Requirements	4615

A.11.6.2.4	Inter-frequency measurement accuracy	4615

A.11.6.2.4.1	Test Purpose and Environment	4615

A.11.6.2.4.2	Test Parameters	4615

A.11.6.2.4.3	Test Requirements	4623

A.11.6.3	SS-SINR	4624

A.11.6.3.1	Intra-frequency measurement accuracy	4624

A.11.6.3.1.1	Test Purpose and Environment	4624

A.11.6.3.1.2	Test Parameters	4624

A.11.6.3.1.3	Test Requirements	4627

A.11.6.3.2	Inter-frequency measurement accuracy	4627

A.11.6.3.2.1	Test Purpose and Environment	4627

A.11.6.3.2.2	Test Parameters	4627

A.11.6.3.2.3	Test Requirements	4630

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4630

A.11.6.3.3.1	Test Purpose and Environment	4630

A.11.6.3.3.2	Test Parameters	4630

A.11.6.3.3.3	Test Requirements	4633

A.11.6.3.4	Inter-frequency measurement accuracy	4633

A.11.6.3.4.1	Test Purpose and Environment	4633

A.11.6.3.4.2	Test Parameters	4633

A.11.6.3.4.3	Test Requirements	4641

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4641

A.11.6.4.1	SSB based L1-RSRP measurement	4641

A.11.6.4.1.1	Test Purpose and Environment	4641

A.11.6.4.1.2	Test parameters	4642

A.11.6.4.1.3	Test Requirements	4645

A.11.6.5	RSSI	4645

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4645

A.11.6.5.1.1	Test Purpose and Environment	4645

A.11.6.5.1.2	Test parameters	4645

A.11.6.5.1.3	Test Requirements	4648

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4648

A.11.6.5.2.1	Test Purpose and Environment	4648

A.11.6.5.2.2	Test parameters	4648

A.11.6.5.2.3	Test Requirements	4651

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4651

A.11.6.5.3.1	Test Purpose and Environment	4651

A.11.6.5.3.2	Test parameters	4651

A.11.6.5.3.3	Test Requirements	4654

A.11.6.6	Channel occupancy	4654

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4654

A.11.6.6.1.1	Test Purpose and Environment	4654

A.11.6.6.1.2	Test parameters	4654

A.11.6.6.1.3	Test Requirements	4657

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4657

A.11.6.6.2.1	Test Purpose and Environment	4657

A.11.6.6.2.2	Test parameters	4657

A.11.6.6.2.3	Test Requirements	4660

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4660

A.11.6.6.3.1	Test Purpose and Environment	4660

A.11.6.6.3.2	Test parameters	4660

A.11.6.6.3.3	Test Requirements	4663

A.11.6.7	E-UTRAN RSRP	4663

A.11.6.8	E-UTRAN RSRQ	4663

A.11.6.9	E-UTRAN SINR	4663

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4663

A.12.1	RRC_IDLE state mobility	4663

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4663

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4663

A.12.1.1.1.1	Test Purpose and Environment	4663

A.12.1.1.1.2	Test Requirements	4668

A.12.2	RRC_CONNECTED state mobility	4669

A.12.2.1	Handover	4669

A.12.2.1.1	E-UTRAN - NR with CCA handover	4669

A.12.2.1.1.1	Test Purpose and Environment	4669

A.12.2.1.1.2	Test Requirements	4674

A.12.3	Signalling characteristics	4675

A.12.3.1	Interruptions	4675

A.12.4	Measurement procedure	4675

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4675

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4675

A.12.4.1.1.1	Test Purpose and Environment	4675

A.12.4.1.1.2	Test Requirements	4679

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4679

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4679

A.12.4.2.1.1	Test Purpose and Environment	4679

A.12.4.2.1.2	Test Requirements	4683

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4684

A.12.4.2.2.1	Test Purpose and Environment	4684

A.12.4.2.2.2	Test Requirements	4688

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4689

A.12.4.2.3.1	Test Purpose and Environment	4689

A.12.4.2.3.2	Test Requirements	4693

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4694

A.12.4.2.4.1	Test Purpose and Environment	4694

A.12.4.2.4.2	Test Requirements	4698

A.12.4.2.5	RSSI measurement reporting	4699

A.12.4.2.5.1	Test purpose and environment	4699

A.12.4.2.5.2	Test parameters	4699

A.12.4.2.5.3	Test Requirements	4702

A.12.4.2.6	Channel occupancy measurement reporting	4702

A.12.4.2.6.1	Test purpose and environment	4702

A.12.4.2.6.2	Test parameters	4702

A.12.4.2.6.3	Test Requirements	4706

A.12.5	Measurement performance	4706

A.12.5.1	E-UTRANNR SFTD	4706

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4706

A.12.5.1.1.1	Test Purpose	4706

A.12.5.1.1.2	Test Environment	4706

A.12.5.1.1.3	Test Requirements	4712

A.12.5.2	E-UTRANNR SS-RSRP	4712

A.12.5.3	E-UTRANNR SS-RSRQ	4712

A.12.5.4	E-UTRANNR SS-SINR	4712

A.12.5.5	E-UTRANNR RSSI	4712

A.12.5.6	E-UTRANNR channel occupancy	4712

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4713

A.13.1	Timing	4713

A.13.1.1	UE transmit timing	4713

A.13.1.2	Timing advance	4713

A.13.2	Signalling characteristics	4713

A.13.2.1	Interruption	4713

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4713

A.13.2.1.1.1	Test Purpose and Environment	4713

A.13.2.1.1.2	Test Requirements	4717

A.13.2.2	SCell activation and deactivation delay	4717

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4717

A.13.2.2.1.1	Test Purpose and Environment	4717

A.13.2.2.1.2	Test Requirements	4722

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4722

A.13.2.2.2.1	Test Purpose and Environment	4722

A.13.2.2.2.2	Test Requirements	4723

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4723

A.13.2.2.3.1	Test Purpose and Environment	4723

A.13.2.2.3.2	Test Requirements	4724

A.13.2.3	Void	4724

A.13.3	Measurement procedure	4724

A.13.3.1	Intra-frequency measurements	4724

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4724

A.13.3.1.1.1	Test purpose and environment	4724

A.13.3.1.1.2	Test parameters	4724

A.13.3.1.1.3	Test Requirements	4729

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4729

A.13.3.1.2.1	Test purpose and environment	4729

A.13.3.1.2.2	Test parameters	4729

A.13.3.1.2.3	Test Requirements	4734

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4734

A.13.3.1.3.1	Test purpose and environment	4734

A.13.3.1.3.2	Test parameters	4734

A.13.3.1.3.3	Test Requirements	4739

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4739

A.13.3.1.4.1	Test purpose and environment	4739

A.13.3.1.4.2	Test parameters	4739

A.13.3.1.4.3	Test Requirements	4744

A.13.3.1.5	RSSI measurement reporting on SCC	4744

A.13.3.1.5.1	Test purpose and environment	4744

A.13.3.1.5.2	Test parameters	4744

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4745

A.13.3.1.6.1	Test purpose and environment	4745

A.13.3.1.6.2	Test parameters	4746

A.13.3.2	Inter-frequency measurements	4746

A.13.3.2.1	RSSI measurement reporting	4746

A.13.3.2.1.1	Test purpose and environment	4746

A.13.3.2.1.2	Test parameters	4746

A.13.3.2.2	Channel occupancy measurement reporting	4747

A.13.3.2.2.1	Test purpose and environment	4747

A.13.3.2.2.2	Test parameters	4747

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4747

A.13.3.2.3.1	Test Purpose and Environment	4747

A.13.3.2.3.2	Test Requirements	4752

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4752

A.13.3.2.4.1	Test Purpose and Environment	4752

A.13.3.2.4.2	Test Requirements	4757

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4758

A.13.3.2.5.1	Test Purpose and Environment	4758

A.13.3.2.5.2	Test Requirements	4763

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4763

A.13.3.2.6.1	Test Purpose and Environment	4763

A.13.3.2.6.2	Test Requirements	4769

A.13.3.3	L1-RSRP measurements for beam reporting	4770

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4770

A.13.3.3.1.1	Test Purpose and Environment	4770

A.13.3.3.1.2	Test parameters	4770

A.13.3.3.1.3	Test Requirements	4774

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4774

A.13.3.3.2.1	Test Purpose and Environment	4774

A.13.3.3.2.2	Test parameters	4774

A.13.3.3.2.3	Test Requirements	4778

A.13.4	Measurement performance	4778

A.13.4.1	SS-RSRP	4778

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4778

A.13.4.1.1.1	Test Purpose and Environment	4778

A.13.4.1.1.2	Test parameters	4778

A.13.4.1.1.3	Test Requirements	4780

A.13.4.2	SS-RSRQ	4780

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4780

A.13.4.2.1.1	Test Purpose and Environment	4780

A.13.4.2.1.2	Test Parameters	4780

A.13.4.2.1.3	Test Requirements	4787

A.13.4.3	SS-SINR	4788

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4788

A.13.4.3.1.1	Test Purpose and Environment	4788

A.13.4.3.1.2	Test Parameters	4788

A.13.4.3.1.3	Test Requirements	4795

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4795

A.13.4.4.1	SSB based L1-RSRP measurement	4795

A.13.4.4.1.1	Test Purpose and Environment	4795

A.13.4.4.1.2	Test parameters	4795

A.13.4.4.1.3	Test Requirements	4799

A.13.4.5	RSSI	4799

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4799

A.13.4.5.1.1	Test Purpose and Environment	4799

A.13.4.5.1.2	Test parameters	4799

A.13.4.5.1.3	Test Requirements	4802

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4802

A.13.4.5.2.1	Test Purpose and Environment	4802

A.13.4.5.2.2	Test parameters	4802

A.13.4.5.2.3	Test Requirements	4806

A.13.4.6	Channel occupancy	4806

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4806

A.13.4.6.1.1	Test Purpose and Environment	4806

A.13.4.6.1.2	Test parameters	4806

A.13.4.6.1.3	Test Requirements	4810

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4810

A.13.4.6.2.1	Test Purpose and Environment	4810

A.13.4.6.2.2	Test parameters	4810

A.13.4.6.2.3	Test Requirements	4814

A.14	NR standalone tests for Satellite access	4814

A.14.1	RRC_IDLE state mobility	4814

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4814

A.14.1.1.1	Test Purpose and Environment	4814

A.14.1.1.2	Test Parameters	4814

A.14.1.1.3	Test Requirements	4816

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4817

A.14.1.2.1	Test Purpose and Environment	4817

A.14.1.2.2	Test Parameters	4817

A.14.1.2.3	Test Requirements	4821

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4822

A.14.1.3.1	Test Purpose and Environment	4822

A.14.1.3.2	Test Parameters	4822

A.14.1.3.3	Test Requirements	4823

A.14.1.4	Location-based measurement initiation to FR1 intra-frequency NR cell reselection	4824

A.14.1.4.1	Test Purpose and Environment	4824

A.14.1.4.2	Test Parameters	4824

A.14.1.4.3	Test Requirements	4826

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4827

A.14.1.5.1	Test Purpose and Environment	4827

A.14.1.5.2	Test Parameters	4827

A.14.1.5.3	Test Requirements	4829

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4830

A.14.1.6.1	Test Purpose and Environment	4830

A.14.1.6.2	Test Parameters	4830

A.14.1.6.3	Test Requirements	4832

A.14.1.7	Time-based measurement initiation to FR1 inter-frequency cell reselection	4833

A.14.1.7.1	Test Purpose and Environment	4833

A.14.1.7.2	Test Parameters	4833

A.14.1.7.3	Test Requirements	4834

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4835

A.14.1.8.1	Test Purpose and Environment	4835

A.14.1.8.2	Test Parameters	4835

A.14.1.8.3	Test Requirements	4837

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4838

A.14.1.9.1	Test Purpose and Environment	4838

A.14.1.9.2	Test Parameters	4838

A.14.1.9.3	Test Requirements	4841

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4841

A.14.1.10.1	Test Purpose and Environment	4841

A.14.1.10.2	Test Parameters	4841

A.14.1.10.3	Test Requirements	4843

A.14.2	RRC_CONNECTED state mobility	4844

A.14.2.1	Handover	4844

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4844

A.14.2.1.1.1	Test Purpose and Environment	4844

A.14.2.1.1.2	Test Parameters	4844

A.14.2.1.1.3	Test Requirements	4846

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4847

A.14.2.1.2.1	Test Purpose and Environment	4847

A.14.2.1.2.2	Test Parameters	4847

A.14.2.1.2.3	Test Requirements	4849

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4849

A.14.2.1.3.1	Test Purpose and Environment	4849

A.14.2.1.3.2	Test Parameters	4849

A.14.2.1.3.3	Test Requirements	4852

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4852

A.14.2.1.4.1	Test Purpose and Environment	4852

A.14.2.1.4.2	Test Parameters	4852

A.14.2.1.4.3	Test Requirements	4854

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4854

A.14.2.1.5.1	Test Purpose and Environment	4854

A.14.2.1.5.2	Test Parameters	4855

A.14.2.1.5.3	Test Requirements	4858

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4858

A.14.2.1.6.1	Test Purpose and Environment	4858

A.14.2.1.6.2	Test Parameters	4858

A.14.2.1.6.3	Test Requirements	4862

A.14.2.2	RRC Connection Mobility Control	4862

A.14.2.2.1	SA: RRC Re-establishment for SAN	4862

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4862

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4865

A.14.2.2.2	Random Access	4868

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4868

A.14.2.2.2.1.1	Test Purpose and Environment	4868

A.14.2.2.2.1.2	Test Requirements	4871

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4872

A.14.2.2.2.2.1	Test Purpose and Environment	4872

A.14.2.2.2.2.2	Test Requirements	4876

A.14.2.2.3	RRC Connection Release with Redirection	4877

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4877

A.14.2.2.3.1.1	Test Purpose and Environment	4877

A.14.2.2.3.1.2	Test Parameters	4877

A.14.2.2.3.1.3	Test Requirements	4880

A.14.3	Timing for Satellite Access	4880

A.14.3.1	UE transmit timing for Satellite Access	4880

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4880

A.14.3.1.1.1	Test Purpose and environment	4880

A.14.3.1.1.2	Test requirements	4882

A.14.3.2	Timing advance for satellite access	4883

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4883

A.14.3.2.1.1	Test Purpose and Environment	4883

A.14.3.2.1.2	Test Parameters	4883

A.14.3.2.1.3	Test Requirements	4886

A.14.4	Signalling characteristics	4886

A.14.4.1	Radio link Monitoring	4886

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4886

A.14.4.1.1.1	Test Purpose and Environment	4886

A.14.4.1.1.2	Test Requirements	4891

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4891

A.14.4.1.2.1	Test Purpose and Environment	4891

A.14.4.1.2.2	Test Requirements	4897

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4897

A.14.4.1.3.1	Test Purpose and Environment	4897

A.14.4.1.3.2	Test Requirements	4901

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4901

A.14.4.1.4.1	Test Purpose and Environment	4901

A.14.4.1.4.2	Test Requirements	4906

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4906

A.14.4.1.5.1	Test Purpose and Environment	4906

A.14.4.1.5.2	Test Requirements	4911

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4911

A.14.4.1.6.1	Test Purpose and Environment	4911

A.14.4.1.6.2	Test Requirements	4916

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4916

A.14.4.1.7.1	Test Purpose and Environment	4916

A.14.4.1.7.2	Test Requirements	4921

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4921

A.14.4.1.8.1	Test Purpose and Environment	4921

A.14.4.1.8.2	Test Requirements	4926

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4926

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4926

A.14.4.2.1.1	Test Purpose and Environment	4926

A.14.4.2.1.2	Test Requirements	4932

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4932

A.14.4.2.2.1	Test Purpose and Environment	4932

A.14.4.2.2.2	Test Requirements	4937

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4937

A.14.4.2.3.1	Test Purpose and Environment	4937

A.14.4.2.3.2	Test Requirements	4943

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4943

A.14.4.2.4.1	Test Purpose and Environment	4943

A.14.4.2.4.2	Test Requirements	4948

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	4948

A.14.4.2.5.1	Test Purpose and Environment	4948

A.14.4.2.5.2	Test Requirements	4953

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	4953

A.14.4.2.6.1	Test Purpose and Environment	4953

A.14.4.2.6.2	Test Requirements	4958

A.14.4.3	Active BWP switch for satellite access	4958

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4958

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4958

A.14.4.3.1.1.1	Test Purpose and Environment	4958

A.14.4.3.1.1.2	Test Requirements	4963

A.14.4.3.2	RRC-based Active BWP Switch	4963

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4963

A.14.4.3.2.1.1	Test Purpose and Environment	4963

A.14.4.3.2.1.2	Test Requirements	4967

A.14.4.4	UE specific CBW change for satellite access	4967

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4967

A.14.4.4.1.1	Test Purpose and Environment	4967

A.14.4.4.1.2	Test Requirements	4971

A.14.4.5	Pathloss reference signal switching delay	4971

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4971

A.14.4.5.1.1	Test Purpose and Environment	4971

A.14.4.5.1.2	Test Requirements	4973

A.14.5	Measurement procedure	4974

A.14.5.1	Intra-frequency Measurements	4974

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4974

A.14.5.1.1.1	Test purpose and Environment	4974

A.14.5.1.1.2	Test parameters	4974

A.14.5.1.1.3	Test Requirements	4976

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4977

A.14.5.1.2.1	Test purpose and Environment	4977

A.14.5.1.2.2	Test parameters	4977

A.14.5.1.2.3	Test Requirements	4979

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4980

A.14.5.1.3.1	Test purpose and Environment	4980

A.14.5.1.3.2	Test parameters	4980

A.14.5.1.3.3	Test Requirements	4982

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4983

A.14.5.1.4.1	Test purpose and Environment	4983

A.14.5.1.4.2	Test parameters	4983

A.14.5.1.4.3	Test Requirements	4985

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4986

A.14.5.1.5.1	Test purpose and Environment	4986

A.14.5.1.5.2	Test parameters	4986

A.15.5.1.5.3	Test Requirements	4988

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4989

A.14.5.1.6.1	Test purpose and Environment	4989

A.14.5.1.6.2	Test parameters	4989

A.14.5.1.6.3	Test Requirements	4991

A.14.5.2	Inter-frequency Measurements	4992

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4992

A.14.5.2.1.1	Test Purpose and Environment	4992

A.14.5.2.1.2	Test Requirements	4996

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4996

A.14.5.2.2.1	Test Purpose and Environment	4996

A.14.5.2.2.2	Test Requirements	5000

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	5000

A.14.5.2.3.1	Test Purpose and Environment	5000

A.14.5.2.3.2	Test Requirements	5004

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	5004

A.14.5.2.4.1	Test Purpose and Environment	5004

A.14.5.2.4.2	Test Requirements	5008

A.14.5.2.5	void	5008

A.14.5.2.5.1	void	5008

A.14.5.2.5.2	void	5008

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	5008

A.14.5.2.6.1	Test Purpose and Environment	5008

A.14.5.2.6.2	Test Requirements	5012

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	5012

A.14.5.2.7.1	Test purpose and Environment	5012

A.14.5.2.7.2	Test parameters	5012

A.14.5.2.7.3	Test Requirements	5014

A.14.5.2.8	Event triggered reporting tests without gap under DRX	5015

A.14.5.2.8.1	Test purpose and Environment	5015

A.14.5.2.8.2	Test parameters	5015

A.14.5.2.8.3	Test Requirements	5017

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	5018

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	5018

A.14.5.3.1.1	Test Purpose and Environment	5018

A.14.5.3.1.2	Test parameters	5018

A.14.5.3.1.3	Test Requirements	5021

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	5021

A.14.5.3.2.1	Test Purpose and Environment	5021

A.14.5.3.2.2	Test parameters	5021

A.14.5.3.2.3	Test Requirements	5025

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	5025

A.14.5.3.3.1	Test Purpose and Environment	5025

A.14.5.3.3.2	Test parameters	5025

A.14.5.3.3.3	Test Requirements	5029

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	5029

A.14.5.3.4.1	Test Purpose and Environment	5029

A.14.5.3.4.2	Test parameters	5030

A.14.5.3.4.3	Test Requirements	5033

A.14.6	Measurement Performance requirements	5033

A.14.6.1	 SS-RSRP for SAN	5033

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5033

A.14.6.1.1.1	Test Purpose and Environment	5033

A.14.6.1.1.2	Test parameters	5033

A.14.6.1.1.3	Test Requirements	5037

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5037

A.14.6.1.2.1	Test Purpose and Environment	5037

A.14.6.1.2.2	Test parameters	5037

A.14.6.1.2.3	Test Requirements	5040

A.14.6.2	SS-RSRQ	5040

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5040

A.14.6.2.1.1	Test Purpose and Environment	5040

A.14.6.2.1.2	Test Parameters	5040

A.14.6.2.1.3	Test Requirements	5043

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5043

A.14.6.2.2.1	Test Purpose and Environment	5043

A.14.6.2.2.2	Test Parameters	5043

A.14.6.2.2.3	Test Requirements	5046

A.14.6.3	SS-SINR	5046

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5046

A.14.6.3.1.1	Test Purpose and Environment	5046

A.14.6.3.1.2	Test Parameters	5046

A.14.6.3.1.3	Test Requirements	5050

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5050

A.14.6.3.2.1	Test Purpose and Environment	5050

A.14.6.3.2.2	Test Parameters	5050

A.14.6.3.2.3	Test Requirements	5054

A.14.6.4	L1-RSRP measurement for beam reporting	5054

A.14.6.4.1	SSB based L1-RSRP measurement	5054

A.14.6.4.1.1	Test Purpose and Environment	5054

A.14.6.4.1.2	Test parameters	5054

A.14.6.4.1.3	Test Requirements	5057

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5058

A.14.6.4.2.1	Test Purpose and Environment	5058

A.14.6.4.2.2	Test parameters	5058

A.14.6.4.2.3	Test Requirements	5061

A.15	NR standalone tests with one or more NR cells in FR2-2	5062

A.15.1	SA: RRC_IDLE state mobility	5062

A.15.1.1	Cell re-selection to NR	5062

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	5062

A.15.1.1.1.1	Test Purpose and Environment	5062

A.15.1.1.1.2	Test Parameters	5062

A.15.1.1.1.3	Test Requirements	5066

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5066

A.15.1.2.1	Test Purpose and Environment	5066

A.15.1.2.2	Test Parameters	5066

A.15.1.2.3	Test Requirements	5070

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5070

A.15.1.3.1	Test Purpose and Environment	5070

A.15.1.3.2	Test Parameters	5070

A.15.1.3.3	Test Requirements	5073

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5073

A.15.1.4.1	Test Purpose and Environment	5073

A.15.1.4.2	Test Parameters	5073

A.15.1.4.3	Test Requirements	5076

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5076

A.15.1.5.1	Test Purpose and Environment	5076

A.15.1.5.2	Test Parameters	5076

A.15.1.5.3	Test Requirements	5080

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5080

A.15.1.6.1	Test Purpose and Environment	5080

A.15.1.6.2	Test Parameters	5080

A.15.1.6.3	Test Requirements	5083

A.15.2	Signaling characteristics	5084

A.15.2.1	SCell Activation and Deactivation Delay	5084

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5084

A.15.2.1.1.1	Test Purpose and Environment	5084

A.15.2.1.1.2	Test Requirements	5086

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5086

A.15.2.1.2.1	Test Purpose and Environment	5086

A.15.2.1.2.2	Test Requirements	5091

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5092

A.15.2.1.3.1	Test Purpose and Environment	5092

A.15.2.1.3.2	Test Requirements	5095

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5096

A.15.2.1.4.1	Test Purpose and Environment	5096

A.15.2.1.4.2	Test Requirements	5099

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5100

A.15.2.1.5.1	Test Purpose and Environment	5100

A.15.2.1.5.2	Test Requirements	5104

A.15.3	RRC_CONNECTED state mobility	5105

A.15.3.1	Handover	5105

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5105

A.15.3.1.1.1	Test Purpose and Environment	5105

A.15.3.1.1.2	Test Parameters	5105

A.15.3.1.1.3	Test Requirements	5109

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5109

A.15.3.1.2.1	Test Purpose and Environment	5109

A.15.3.1.2.2	Test Parameters	5109

A.15.3.1.2.3	Test Requirements	5113

A.15.4	Measurement procedure	5114

A.15.4.1	Intra-frequency Measurements	5114

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5114

A.15.4.1.1.1	Test purpose and Environment	5114

A.15.4.1.1.2	Test Requirements	5116

A.15.4.2	Inter-frequency Measurements	5117

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5117

A.15.4.2.1.1	Test Purpose and Environment	5117

A.15.4.2.1.2	Test Requirements	5120

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5121

A.16.1	SA: RRC_IDLE state mobility for RedCap	5121

A.16.1.1	Cell re-selection to NR	5121

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5121

A.16.1.1.1.1	Test Purpose and Environment	5121

A.16.1.1.1.2	Test Parameters	5121

A.16.1.1.1.3	Test Requirements	5125

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5125

A.16.1.1.2.1	Test Purpose and Environment	5125

A.16.1.1.2.2	Test Parameters	5125

A.16.1.1.2.3	Test Requirements	5129

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5129

A.16.1.1.3.1	Test Purpose and Environment	5129

A.16.1.1.3.2	Test Parameters	5129

A.16.1.1.3.3	Test Requirements	5134

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5134

A.16.1.1.4.1	Test Purpose and Environment	5134

A.16.1.1.4.2	Test Parameters	5134

A.16.1.1.4.3	Test Requirements	5139

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5139

A.16.1.1.5.1	Test Purpose and Environment	5139

A.16.1.1.5.2	Test Parameters	5139

A.16.1.1.5.3	Test Requirements	5144

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5144

A.16.1.1.6.1	Test Purpose and Environment	5144

A.16.1.1.6.2	Test Parameters	5144

A.16.1.1.6.3	Test Requirements	5149

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5149

A.16.1.1.7.1	Test Purpose and Environment	5149

A.16.1.1.7.2	Test Parameters	5149

A.16.1.1.7.3	Test Requirements	5154

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5154

A.16.1.1.8.1	Test Purpose and Environment	5154

A.16.1.1.8.2	Test Parameters	5155

A.16.1.1.8.3	Test Requirements	5159

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5160

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5160

A.16.1.2.1.1	Test Purpose and Environment	5160

A.16.1.2.1.2	Test Parameters	5160

A.16.1.2.1.3	Test Requirements	5163

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5164

A.16.1.2.2.1	Test Purpose and Environment	5164

A.16.1.2.2.2	Test Parameters	5164

A.16.1.2.2.3	Test Requirements	5167

A.16.1.2.3.1	Test Purpose and Environment	5168

A. 16.1.2.3.2	Test Parameters	5168

A.16.1.2.3.3	Test Requirements	5171

A.16.1.2.4.1	Test Purpose and Environment	5172

A.16.1.2.4.2	Test Parameters	5172

A.16.1.3.1.3	Test Requirements	5175

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5176

A.16.1.2.5.1	Test Purpose and Environment	5176

A.16.1.2.5.2	Test Parameters	5176

A.16.1.2.5.3	Test Requirements	5179

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5180

A.16.1.2.6.1	Test Purpose and Environment	5180

A.16.1.2.6.2	Test Parameters	5180

A.16.1.2.6.3	Test Requirements	5183

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5184

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5184

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5184

A.16.2.1.1.1	Test purpose and Environment	5184

A.16.2.1.1.2	Test Parameters	5185

A.16.2.1.1.3	Test requirements	5188

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5188

A.16.2.1.2.1	Test purpose and Environment	5188

A.16.2.1.2.2	Test Parameters	5189

A.16.2.1.2.3	Test requirements	5192

A.16.3	RRC_CONNECTED state mobility for RedCap	5192

A.16.3.1	Handover	5192

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5192

A.16.3.1.1.1	Test Purpose and Environment	5192

A.16.3.1.1.2	Test Parameters	5192

A.16.3.1.1.3	Test Requirements	5196

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5196

A.16.3.1.2.1	Test Purpose and Environment	5196

A.16.3.1.2.2	Test Parameters	5196

A.16.3.1.2.3	Test Requirements	5200

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5200

A.16.3.1.3.1	Test Purpose and Environment	5200

A.16.3.1.3.2	Test Parameters	5200

A.16.3.1.3.3	Test Requirements	5204

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5204

A.16.3.1.4.1	Test Purpose and Environment	5204

A.16.3.1.4.2	Test Parameters	5204

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5208

A.16.3.1.5.1	Test Purpose and Environment	5208

A.16.3.1.5.2	Test Parameters	5208

A.16.3.1.5.3	Test Requirements	5212

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5212

A.16.3.1.6.1	Test Purpose and Environment	5212

A.16.3.1.6.2	Test Parameters	5212

A.16.3.1.6.3	Test Requirements	5216

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5216

A.16.3.1.7.1	Test Purpose and Environment	5216

A.16.3.1.7.2	Test Requirements	5222

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5222

A.16.3.1.8.1	Test Purpose and Environment	5222

A.16.3.1.8.2	Test Requirements	5228

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5228

A.16.3.1.9.1	Test Purpose and Environment	5228

A.16.3.1.9.2	Test Requirements	5235

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5235

A.16.3.1.10.1	Test Purpose and Environment	5235

A.16.3.1.10.2	Test Requirements	5241

A.16.3.2	RRC Connection Mobility Control	5241

A.16.3.2.1	SA: RRC Re-establishment	5241

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5241

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5245

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5249

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5254

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5259

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5263

A.16.3.2.2	Random Access	5267

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5267

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5273

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5278

A.16.3.2.2.4	4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	5283

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5288

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5292

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5296

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5300

A.16.3.2.3	SA: RRC Connection Release with Redirection	5304

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5304

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5308

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5312

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5319

A.16.4	Timing for RedCap	5326

A.16.4.1	UE transmit timing	5326

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5326

A.16.4.1.1.1	Test Purpose and environment	5326

A.16.4.1.1.2	Test requirements	5329

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5329

A.16.4.1.2.1	Test Purpose and environment	5329

A.16.4.1.2.2	Test requirements	5331

A.16.4.2	UE timer accuracy	5332

A.16.4.3	Timing advance	5332

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5332

A.16.4.3.1.1	Test Purpose and Environment	5332

A.16.4.3.1.2	Test Parameters	5332

A.16.4.3.1.3	Test Requirements	5336

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5336

A.16.4.3.2.1	Test Purpose and Environment	5336

A.16.4.3.2.2	Test Parameters	5337

A.16.4.3.2.3	Test Requirements	5340

A.16.5	Signalling characteristics for RedCap	5341

A.16.5.1	Radio link Monitoring	5341

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5341

A.16.5.1.1.1	Test Purpose and Environment	5341

A.16.5.1.1.2	Test Requirements	5346

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5346

A.16.5.1.2.1	Test Purpose and Environment	5346

A.16.5.1.2.2	Test Requirements	5352

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5352

A.16.5.1.3.1	Test Purpose and Environment	5352

A.16.5.1.3.2	Test Requirements	5358

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5358

A.16.5.1.4.1	Test Purpose and Environment	5358

A.16.5.1.4.2	Test Requirements	5364

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5364

A.16.5.1.5.1	Test Purpose and Environment	5364

A.16.5.1.5.2	Test Requirements	5370

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5370

A.16.5.1.6.1	Test Purpose and Environment	5370

A.16.5.1.6.2	Test Requirements	5375

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5375

A.16.5.1.7.1	Test Purpose and Environment	5375

A.16.5.1.7.2	Test Requirements	5380

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5380

A.16.5.1.8.1	Test Purpose and Environment	5380

A.16.5.1.8.2	Test Requirements	5386

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5386

A.16.5.1.9.1	Test Purpose and Environment	5386

A.16.5.1.9.2	Test Requirements	5392

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5392

A.16.5.1.10.1	Test Purpose and Environment	5392

A.16.5.1.10.2	Test Requirements	5398

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5398

A.16.5.1.11.1	Test Purpose and Environment	5398

A.16.5.1.11.2	Test Requirements	5404

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5404

A.16.5.1.12.1	Test Purpose and Environment	5404

A.16.5.1.12.2	Test Requirements	5409

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5409

A.16.5.1.13.1	Test Purpose and Environment	5409

A.16.5.1.13.2	Test Requirements	5415

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5415

A.16.5.1.14.1	Test Purpose and Environment	5415

A.16.5.1.14.2	Test Requirements	5421

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5421

A.16.5.1.15.1	Test Purpose and Environment	5421

A.16.5.1.15.2	Test Requirements	5427

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5427

A.16.5.1.16.1	Test Purpose and Environment	5427

A.16.5.1.16.2	Test Requirements	5433

A.16.5.2	Beam Failure Detection and Link recovery procedures	5433

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5433

A.16.5.2.1.1	Test Purpose and Environment	5433

A.16.5.2.1.2	Test Requirements	5439

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5439

A.16.5.2.2.1	Test Purpose and Environment	5439

A.16.5.2.2.2	Test Requirements	5445

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5445

A.16.5.2.3.1	Test Purpose and Environment	5445

A.16.5.2.3.2	Test Requirements	5452

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5452

A.16.5.2.4.1	Test Purpose and Environment	5452

A.16.5.2.4.2	Test Requirements	5458

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5458

A.16.5.2.5.1	Test Purpose and Environment	5458

A.16.5.2.5.2	Test Requirements	5464

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5464

A.16.5.2.6.1	Test Purpose and Environment	5464

A.16.5.2.6.2	Test Requirements	5470

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5470

A.16.5.2.7.1	Test Purpose and Environment	5470

A.16.5.2.7.2	Test Requirements	5476

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5476

A.16.5.2.8.1	Test Purpose and Environment	5476

A.16.5.2.8.2	Test Requirements	5482

A.16.5.3	Active BWP switch	5482

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5482

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5482

A.16.5.3.1.1.1	Test Purpose and Environment	5482

A.16.5.3.1.1.2	Test Requirements	5487

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5487

A.16.5.3.1.1.2	Test Purpose and Environment	5487

A.16.5.3.1.2.2	Test Requirements	5492

A.16.5.3.2	RRC-based Active BWP Switch	5492

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5492

A.16.5.3.2.1.1	Test Purpose and Environment	5492

A.16.5.3.2.1.2	Test Requirements	5496

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5496

A.16.5.3.2.2.1	Test Purpose and Environment	5496

A.16.5.3.2.2.2	Test Requirements	5500

A.16.5.4	UE specific CBW change	5500

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5500

A.16.5.4.1.1	Test Purpose and Environment	5500

A.16.5.4.1.2	Test Requirements	5504

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5504

A.16.5.4.2.1	Test Purpose and Environment	5504

A.16.5.4.2.2	Test Requirements	5508

A.16.6	Measurement procedure for RedCap	5508

A.16.6.1	Intra-frequency Measurements	5508

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5508

A.16.6.1.1.1	Test purpose and Environment	5508

A.16.6.1.1.2	Test parameters	5508

A.16.6.1.1.3	Test Requirements	5512

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5512

A.16.6.1.2.1	Test purpose and Environment	5512

A.16.6.1.2.2	Test parameters	5512

A.16.6.1.2.3	Test Requirements	5516

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5516

A.16.6.1.3.1	Test purpose and Environment	5516

A.16.6.1.3.2	Test parameters	5516

A.16.6.1.3.3	Test Requirements	5520

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5520

A.16.6.1.4.1	Test purpose and Environment	5520

A.16.6.1.4.2	Test parameters	5520

A.16.6.1.4.3	Test Requirements	5524

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5524

A.16.6.1.5.1	Test purpose and Environment	5524

A.16.6.1.5.2	Test parameters	5524

A.16.6.1.5.3	Test Requirements	5528

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5528

A.16.6.1.6.1	Test purpose and Environment	5528

A.16.6.1.6.2	Test parameters	5528

A.16.6.1.6.3	Test Requirements	5532

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5532

A.16.6.1.7.1	Test purpose and Environment	5532

A.16.6.1.7.2	Test parameters	5532

A.16.6.1.7.3	Test Requirements	5536

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5536

A.16.6.1.8.1	Test purpose and Environment	5536

A.16.6.1.8.2	Test parameters	5536

A.16.6.1.8.3	Test Requirements	5540

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5540

A.16.6.1.9.1	Test purpose and Environment	5540

A.16.6.1.9.2	Test parameters	5540

A.16.6.1.9.3	Test Requirements	5542

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5543

A.16.6.1.10.1	Test purpose and Environment	5543

A.16.6.1.10.2	Test parameters	5543

A.16.6.1.10.3	Test Requirements	5544

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5545

A.16.6.1.11.1	Test purpose and Environment	5545

A.16.6.1.11.2	Test parameters	5545

A.16.6.1.11.3	Test Requirements	5547

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5548

A.16.6.1.12.1	Test purpose and Environment	5548

A.16.6.1.12.2	Test parameters	5548

A.16.6.1.12.3	Test Requirements	5550

A.16.6.2	Inter-frequency Measurements	5551

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5551

A.16.6.2.1.1	Test Purpose and Environment	5551

A.16.6.2.1.2	Test Requirements	5556

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5556

A.16.6.2.2.1	Test Purpose and Environment	5556

A.16.6.2.2.2	Test Requirements	5561

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5561

A.16.6.2.3.1	Test Purpose and Environment	5561

A.16.6.2.3.2	Test Requirements	5564

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5565

A.16.6.2.4.1	Test Purpose and Environment	5565

A.16.6.2.4.2	Test Requirements	5568

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5569

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5569

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5569

A.16.6.2.7.1	Test Purpose and Environment	5569

A.16.6.2.7.2	Test Requirements	5573

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5574

A.16.6.2.8.1	Test Purpose and Environment	5574

A.16.6.2.8.2	Test Requirements	5578

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5579

A.16.6.2.9.1	Test Purpose and Environment	5579

A.16.6.2.9.2	Test Requirements	5581

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5582

A.16.6.2.10.1	Test Purpose and Environment	5582

A.16.6.2.10.2	Test Requirements	5584

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5585

A.16.6.2.11.1	Test Purpose and Environment	5585

A.16.6.2.11.2	Test Requirements	5588

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5588

A.16.6.2.12.1	Test Purpose and Environment	5588

A.16.6.2.12.2	Test Requirements	5592

A.16.6.3	Inter-RAT Measurements	5593

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5593

A.16.6.3.1.1	Test purpose and Environment	5593

A.16.6.3.1.2	Test Requirements	5598

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5598

A.16.6.3.2.1	Test purpose and Environment	5598

A.16.6.3.2.2	Test Requirements	5603

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5603

A.16.6.3.3.1	Test purpose and Environment	5603

A.16.6.3.3.2	Test Requirements	5608

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5608

A.16.6.3.4.1	Test purpose and Environment	5608

A.16.6.3.4.2	Test Requirements	5613

A.16.6.4	L1-RSRP measurement for beam reporting	5613

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5613

A.16.6.4.1.1	Test Purpose and Environment	5613

A.16.6.4.1.2	Test parameters	5613

A.16.6.4.1.3	Test Requirements	5616

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5616

A.16.6.4.2.1	Test Purpose and Environment	5616

A.16.6.4.2.2	Test parameters	5617

A.16.6.4.2.3	Test Requirements	5620

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5620

A.16.6.4.3.1	Test Purpose and Environment	5620

A.16.6.4.3.2	Test parameters	5621

A.16.6.4.3.3	Test Requirements	5624

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5624

A.16.6.4.4.1	Test Purpose and Environment	5624

A.16.6.4.4.2	Test parameters	5625

A.16.6.4.4.3	Test Requirements	5628

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5628

A.16.6.4.5.1	Test Purpose and Environment	5628

A.16.6.4.5.2	Test parameters	5629

A.16.6.4.5.3	Test Requirements	5632

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5632

A.16.6.4.6.1	Test Purpose and Environment	5632

A.16.6.4.6.2	Test parameters	5633

A.16.6.4.6.3	Test Requirements	5636

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5636

A.16.6.4.7.1	Test Purpose and Environment	5636

A.16.6.4.7.2	Test parameters	5637

A.16.6.4.7.3	Test Requirements	5640

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5640

A.16.6.4.8.1	Test Purpose and Environment	5640

A.16.6.4.8.2	Test parameters	5641

A.16.6.4.8.3	Test Requirements	5644

A.16.6.7	NR measurements with autonomous gaps	5644

A.16.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5644

A.16.6.7.1.1	Test Purpose and Environment	5644

A.16.6.7.1.2	Test Parameters	5644

A.16.6.7.1.3	Test Requirements	5647

A.16.6.7.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5647

A.16.6.7.2.1	Test Purpose and Environment	5647

A.16.6.7.2.2	Test Parameters	5647

A.16.6.7.2.3	Test Requirements	5650

A.16.6.7.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5650

A.16.6.7.3.1	Test Purpose and Environment	5650

A.16.6.7.3.2	Test Requirements	5657

A.16.6.7.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5657

A.16.6.7.4.1	Test Purpose and Environment	5657

A.16.6.7.4.2	Test Requirements	5664

A.16.7	Measurement Performance requirements for RedCap	5664

A.16.7.1	SS-RSRP	5664

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5664

A.16.7.1.1.1	Test Purpose and Environment	5664

A.16.7.1.1.2	Test parameters	5664

A.16.7.1.1.3	Test Requirements	5669

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5669

A.16.7.1.2.1	Test Purpose and Environment	5669

A.16.7.1.2.2	Test parameters	5669

A.16.7.1.2.3	Test Requirements	5674

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5674

A.16.7.1.3.1	Test Purpose and Environment	5674

A.16.7.1.3.2	Test parameters	5674

A.16.7.1.3.3	Test Requirements	5678

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5678

A.16.7.1.4.1	Test Purpose and Environment	5678

A.16.7.1.4.2	Test parameters	5679

A.16.7.1.4.3	Test Requirements	5683

A.16.7.2	SS-RSRQ	5683

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5683

A.16.7.2.1.1	Test Purpose and Environment	5683

A.16.7.2.1.2	Test Parameters	5684

A.16.7.2.1.3	Test Requirements	5689

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5689

A.16.7.2.2.1	Test Purpose and Environment	5689

A.16.7.2.2.2	Test Parameters	5689

A.16.7.2.2.3	Test Requirements	5694

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5694

A.16.7.2.3.1	Test Purpose and Environment	5694

A.16.7.2.3.2	Test parameters	5694

A.16.7.2.3.3	Test Requirements	5699

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5699

A.16.7.2.4.1	Test Purpose and Environment	5699

A.16.7.2.4.2	Test parameters	5699

A.16.7.2.4.3	Test Requirements	5705

A.16.7.3	SS-SINR	5705

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5705

A.16.7.3.1.1	Test Purpose and Environment	5705

A.16.7.3.1.2	Test parameters	5706

A.16.7.3.1.3	Test Requirements	5710

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5711

A.16.7.3.2.1	Test Purpose and Environment	5711

A.16.7.3.2.2	Test parameters	5711

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5715

A.16.7.3.3.1	Test Purpose and Environment	5715

A.16.7.3.3.2	Test parameters	5715

A.16.7.3.3.3	Test Requirements	5720

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5721

A.16.7.3.4.1	Test Purpose and Environment	5721

A.16.7.3.4.2	Test parameters	5721

A.16.7.3.4.3	Test Requirements	5725

A.16.7.4	L1-RSRP measurement for beam reporting	5726

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5726

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5726

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5726

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5726

A.16.7.5	E-UTRAN RSRP	5726

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5726

A.16.7.5.1.1	Test Purpose and Environment	5726

A.16.7.5.1.2	Test parameters	5726

A.16.7.5.1.3	Test Requirements	5732

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5733

A.16.7.5.2.1	Test Purpose and Environment	5733

A.16.7.5.2.2	Test parameters	5733

A.16.7.5.2.3	Test Requirements	5739

A.16.7.6	E-UTRAN RSRQ	5740

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5740

A.16.7.6.1.1	Test Purpose and Environment	5740

A.16.7.6.1.2	Test parameters	5740

A.16.7.6.1.3	Test Requirements	5746

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5747

A.16.7.6.2.1	Test Purpose and Environment	5747

A.16.7.6.2.2	Test parameters	5747

A.16.7.6.2.3	Test Requirements	5753

A.16.7.7	E-UTRAN RS-SINR	5754

A.16.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5754

A.16.7.7.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5754

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5755

A.17.1	SA: RRC_IDLE state mobility for RedCap	5755

A.17.1.1	Cell re-selection to NR	5755

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5755

A.17.1.1.1.1	Test Purpose and Environment	5755

A.17.1.1.1.2	Test Parameters	5755

A.17.1.1.1.3	Test Requirements	5759

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5759

A.17.1.1.2.1	Test Purpose and Environment	5759

A.17.1.1.2.2	Test Parameters	5759

A.17.1.1.2.3	Test Requirements	5763

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5763

A.17.1.1.3.1	Test Purpose and Environment	5763

A.17.1.1.3.2	Test Parameters	5763

A.17.1.1.3.3	Test Requirements	5766

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5766

A.17.1.1.4.1	Test Purpose and Environment	5766

A.17.1.1.4.2	Test Parameters	5766

A.17.1.1.4.3	Test Requirements	5769

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5769

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5769

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5769

A.17.2.1.1.1	Test Purpose and Environment	5769

A.17.2.1.1.2	Test Requirements	5773

A.17.3	RRC_CONNECTED state mobility for RedCap	5773

A.17.3.1	Handover for RedCap	5773

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5773

A.17.3.1.1.1	Test Purpose and Environment	5773

A.17.3.1.1.2	Test Parameters	5773

A.17.3.1.1.3	Test Requirements	5776

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5776

A.17.3.1.2.1	Test Purpose and Environment	5776

A.17.3.1.2.2	Test Parameters	5776

A.17.3.1.2.3 Test Requirements	5778

A.17.3.2	RRC Connection Mobility Control for RedCap	5778

A.17.3.2.1	SA: RRC Re-establishment	5778

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5778

A.17.3.2.1.1.1	Test Purpose and Environment	5778

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5781

A.17.3.2.1.2.1	Test Purpose and Environment	5781

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5784

A.17.3.2.1.3.1	Test Purpose and Environment	5784

A.17.3.2.1.3.2	Test Requirements	5786

A.17.3.2.2	Random Access	5787

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5787

A.17.3.2.2.1.1	Test Purpose and Environment	5787

A.17.3.2.2.1.2	Test Requirements	5789

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5791

A.17.3.2.2.2.1	Test Purpose and Environment	5791

A.17.3.2.2.2.2	Test Requirements	5793

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5795

A.17.3.2.2.3.1	Test Purpose and Environment	5795

A.17.3.2.2.3.2	Test Requirements	5797

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5798

A.17.3.2.2.4.1	Test Purpose and Environment	5798

A.17.3.2.2.4.2	Test Requirements	5800

A.17.3.2.3	SA: RRC Connection Release with Redirection	5801

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5801

A.17.3.2.3.1.1	Test Purpose and Environment	5801

A.17.3.2.3.1.2	Test Parameters	5801

A.17.3.2.3.1.3	Test Requirements	5805

A.17.4	Timing	5805

A.17.4.1	UE transmit timing	5805

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5805

A.17.4.1.1.1	Test Purpose and environment	5805

A.17.4.1.1.2	Test requirements	5808

A.17.4.2	UE timer accuracy	5809

A.17.4.3	Timing advance	5809

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5809

A.17.4.3.1.1	Test Purpose and Environment	5809

A.17.4.3.1.2	Test Parameters	5809

A.17.4.3.1.3 Test Requirements	5812

A.17.4.2	UE timer accuracy	5812

A.17.4.3	Timing advance	5812

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5812

A.17.5	Signaling characteristics for RedCap	5812

A.17.5.1	Radio link Monitoring for RedCap	5812

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5812

A.17.5.1.1.1	Test Purpose and Environment	5812

A.17.5.1.1.2	Test Requirements	5815

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5816

A.17.5.1.2.1	Test Purpose and Environment	5816

A.17.5.1.2.2	Test Requirements	5820

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5821

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5821

A.17.5.1.4.1	Test Purpose and Environment	5821

A.17.5.1.4.2	Test Requirements	5825

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5825

A.17.5.1.5.1	Test Purpose and Environment	5825

A.17.5.1.5.2	Test Requirements	5830

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5830

A.17.5.1.6.1	Test Purpose and Environment	5830

A.17.5.1.6.2	Test Requirements	5834

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5834

A.17.5.1.7.1	Test Purpose and Environment	5834

A.17.5.1.7.2	Test Requirements	5838

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5838

A.17.5.1.8.1	Test Purpose and Environment	5838

A.17.5.1.8.2	Test Requirements	5843

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5843

A.17.5.1.9.1	Test Purpose and Environment	5843

A.17.5.1.9.2	Test Requirements	5846

A.17.5.2	Beam Failure Detection and Link recovery procedures	5846

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5846

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5846

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5846

A.17.5.2.3.1	Test Purpose and Environment	5846

A.17.5.2.3.2	Test Requirements	5850

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5850

A.17.5.2.4.1	Test Purpose and Environment	5850

A.17.5.2.4.2	Test Requirements	5854

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5854

A.17.5.2.5.1	Test Purpose and Environment	5854

A.17.5.2.5.2	Test Requirements	5858

A.17.5.3	Active BWP switch for RedCap	5858

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5858

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5858

A.17.5.3.1.1.1	Test Purpose and Environment	5858

A.17.5.3.1.1.2	Test Requirements	5861

A.17.5.3.2	RRC-based Active BWP Switch	5861

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5861

A.17.5.3.2.1.1	Test Purpose and Environment	5861

A.17.5.3.2.1.2	Test Requirements	5864

A.17.5.4	Active TCI state switch delay	5865

A.17.5.4.1	MAC-CE based active TCI state switch	5865

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5865

A.17.5.4.1.1.1	Test Purpose and Environment	5865

A.17.5.4.1.1.2	Test Requirements	5868

A.17.5.4.2	RRC based active TCI state switch	5868

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5868

A.17.5.4.2.1.1	Test Purpose and Environment	5868

A.17.5.4.2.1.2	Test Requirements	5872

A.17.5.5	Uplink spatial relation switch delay	5872

A.17.5.5.1	MAC-CE based Spatial Relation switch	5872

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5872

A.17.5.5.1.1.1	Test Purpose and Environment	5872

A.17.5.5.1.1.2	Test Requirements	5875

A.17.5.5.2	RRC based spatial relation switch	5875

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5875

A.17.5.5.2.1.2	Test Requirements	5878

A.17.5.6	UE specific CBW change	5878

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5878

A.17.5.6.1.1	Test Purpose and Environment	5878

A.17.5.6.1.2	Test Requirements	5881

A.17.6	Measurement procedure for RedCap	5882

A.17.6.1	Intra-frequency Measurements	5882

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5882

A.17.6.1.1.1	Test purpose and Environment	5882

A.17.6.1.1.2	Test Requirements	5885

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5885

A.17.6.1.2.1	Test purpose and Environment	5885

A.7.6.1.2.2	Test Requirements	5885

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5886

A.17.6.1.3.1	Test purpose and Environment	5886

A.17.6.1.3.2	Test Requirements	5890

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5890

A.17.6.1.4.1	Test purpose and Environment	5890

A.17.6.1.4.2	Test Requirements	5893

A.17.6.2	Inter-frequency Measurements	5894

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5894

A.17.6.2.1.1	Test Purpose and Environment	5894

A.17.6.2.1.2	Test Requirements	5897

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5897

A.17.6.2.2.1	Test Purpose and Environment	5897

A.17.6.2.2.2	Test Requirements	5901

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5901

A.17.6.2.3.1	Test Purpose and Environment	5901

A.17.6.2.3.2	Test Requirements	5905

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5905

A.17.6.2.4.1	Test Purpose and Environment	5905

A.17.6.2.4.2	Test Requirements	5909

A.17.6.3	L1-RSRP measurement for beam reporting	5909

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5909

A.17.6.3.1.1	Test Purpose and Environment	5909

A.17.6.3.1.2	Test parameters	5910

A.17.6.3.1.3	Test Requirements	5910

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5910

A.17.6.3.2.1	Test Purpose and Environment	5910

A.17.6.3.2.2	Test parameters	5910

A.17.6.3.2.3	Test Requirements	5912

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5912

A.17.6.3.3.1	Test Purpose and Environment	5912

A.17.6.3.3.2	Test parameters	5913

A.17.6.3.3.3	Test Requirements	5915

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5916

A.17.6.3.4.1	Test Purpose and Environment	5916

A.17.6.3.4.2	Test parameters	5916

A.7.6.3.3.3	Test Requirements	5918

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5919

A.17.6.4.1.1	Test Purpose and Environment	5919

A.17.6.4.1.2	Test Requirements	5922

A.17.7	Measurement Performance requirements	5922

A.17.7.1	SS-RSRP	5922

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5922

A.17.7.1.1.1	Test Purpose and Environment	5922

A.17.7.1.1.2	Test parameters	5922

A.17.7.1.1.3	Test Requirements	5926

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5927

A.17.7.1.2.1	Test Purpose and Environment	5927

A.17.7.1.2.2	Test parameters	5927

A.17.7.1.2.3	Test Requirements	5931

A.17.7.2	SS-RSRQ	5932

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5932

A.17.7.2.1.1	Test Purpose and Environment	5932

A.17.7.2.1.2	Test Parameters	5932

A.17.7.2.1.3	Test Requirements	5934

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5934

A.17.7.2.2.1	Test Purpose and Environment	5934

A.17.7.2.2.2	Test parameters	5934

A.17.7.2.2.3	Test Requirements	5936

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5936

A.17.7.3	L1-RSRP measurement for beam reporting	5936

A.17.7.3.1	SSB based L1-RSRP measurement	5936

A.17.7.3.1.1	Test Purpose and Environment	5936

A.17.7.3.1.2	Test parameters	5937

A.17.7.3.1.3	Test Requirements	5937

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5937

A.17.7.3.2.1	Test Purpose and Environment	5937

A.17.7.3.2.2	Test parameters	5937

A.17.7.3.2.3	Test Requirements	5938

A.17.7.4	SS-SINR	5938

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5938

A.17.7.4.1.1	Test Purpose and Environment	5938

A.17.7.4.1.2	Test parameters	5938

A.17.7.4.1.3	Test Requirements	5940

A.18	E-UTRA standalone tests for NR RRM for RedCap	5940

A.18.1	RRC_IDLE state mobility	5940

A.18.1.1	Inter-RAT NR Cell re-selection	5940

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5940

A.18.1.1.1.1	Test Purpose and Environment	5940

A.18.2.1.1.2	Test Requirements	5945

A.18.2	RRC_CONNECTED state mobility	5945

A.18.2.1	Handover	5945

A.18.2.1.1	E-UTRAN - NR handover in FR1	5945

A.18.2.1.1.1	Test Purpose and Environment	5945

A.18.2.1.1.2	Test Requirements	5950

A.18.2.2	RRC connection release with redirection	5950

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5950

A.18.2.2.1.1	Test Purpose and Environment	5950

A.18.2.2.1.2	Test Parameters	5950

A.18.2.2.1.3	Test Requirements	5957

A.18.3	Measurement procedure	5957

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5957

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5957

A.18.3.1.1.1	Test Purpose and Environment	5957

A.18.3.1.1.2	Test Requirements	5963

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5963

A.18.3.1.2.1	Test Purpose and Environment	5963

A.18.3.1.2.2	Test Requirements	5969

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5969

A.18.3.1.3.1	Test Purpose and Environment	5969

A.18.3.1.3.2	Test Requirements	5975

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5975

A.18.3.1.4.1	Test Purpose and Environment	5975

A.18.3.1.4.2	Test Requirements	5981

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5981

A.18.3.1.5.1	Test Purpose and Environment	5981

A.18.3.1.5.2	Test Requirements	5983

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5984

A.18.3.1.6.1	Test Purpose and Environment	5984

A.18.3.1.6.2	Test Requirements	5986

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5987

A.18.3.1.7.1	Test Purpose and Environment	5987

A.18.3.1.7.2	Test Requirements	5989

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5990

A.18.3.1.8.1	Test Purpose and Environment	5990

A.18.3.1.8.2	Test Requirements	5992

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5994

B.1	Conditions for NR RRC_IDLE state mobility	5994

B.1.1	Introduction	5994

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5994

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5995

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5995

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5997

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5997

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5998

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5998

B.2.1	Introduction	5998

B.2.1.1	General	5998

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5998

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5998

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5998

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5999

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	6000

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	6000

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	6000

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	6001

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	6001

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	6001

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	6001

B.2.2	Conditions for NR intra-frequency measurements	6002

B.2.3	Conditions for NR inter-frequency measurements	6003

B.2.4	Conditions for NR L1-RSRP reporting	6004

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6004

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6005

B.2.5	Conditions for RRC connection release with redirection to NR	6007

B.2.6	Void	6008

B.2.6.1	Void	6008

B.2.6.2	Void	6008

B.2.7	Conditions for SRS-RSRP measurements	6008

B.2.8	Conditions for NR L1-SINR reporting	6009

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	6009

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	6010

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	6010

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	6010

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	6012

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	6012

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	6013

B.2.9	Conditions for NR intra-frequency measurements under CCA	6014

B.2.10	Conditions for NR inter-frequency measurements under CCA	6014

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6014

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6014

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	6015

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	6016

B.2.14	Conditions for NR PRS-based measurements	6018

B.2.15	Conditions for NR intra-frequency measurements	6020

B.2.16	Conditions for NR inter-frequency measurements	6021

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6023

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6024

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6024

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6024

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6024

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6025

B.3	RRM Requirements Exceptions	6025

B.3.1	Introduction	6025

B.3.2	Receiver sensitivity relaxation for CA	6025

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6025

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6025

B.3.2.2.1	Inter-band carrier aggregation	6025

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6025

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6026

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6026

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6026

B.3.2.4.1	Intra-band contiguous carrier aggregation	6026

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6026

B.3.3	Receiver sensitivity relaxation for DC	6026

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6026

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6026

B.3.4	Receiver sensitivity relaxation for SUL	6026

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6026

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6027

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6027

B.4	Conditions for V2X	6027

B.4.1	Test parameters for GNSS signals	6027

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6027

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6028

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6028

B.5	High level test procedure for SAN RRM tests	6028

Annex C (informative): Change history	6029