| 3GPP TR 38.848 V0.2.0 (2023-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Deployment scenarios, use cases, services	7

4.1	Use cases/services	7

4.1.1	Representative use cases	7

4.2	Deployment scenarios and connectivity topologies	10

4.2.1	Connectivity topologies	10

4.2.1.1	Topology 1: BS ↔ Ambient IoT device	10

4.2.1.2	Topology 2: BS ↔ intermediate node ↔ Ambient IoT device	11

4.2.1.3	Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS	11

4.2.1.4	Topology 4: UE ↔ Ambient IoT device	12

4.2.2	Deployment scenarios	12

4.2.2.1	Deployment scenario 1: Device indoors, basestation indoors	12

4.2.2.2	Deployment scenario 2: Device indoors, basestation outdoors	13

4.2.2.3	Deployment scenario 3: Device indoors, UE-based reader	13

4.2.2.4	Deployment scenario 4: Device outdoors, basestation outdoors	13

4.2.2.5	Deployment scenario 5: Device outdoors, UE-based reader	14

4.3	Device categorization	14

5	RAN design targets	15

5.1	Device power consumption	15

5.2 	Device complexity	15

5.3	Coverage	15

5.4	User experienced data rate	15

5.5	Maximum message size (or maximum ‘TB’ size)	15

5.6	Latency	15

5.7	Positioning accuracy	15

5.8	Connection/device density	15

5.9 	Moving speed of device	15

6	 Comparison and assessment	16

7	Conclusions and recommendations	16

Annex A (informative): Change history	17

Annex B: Energy sources for energy harvesting	17

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Editor's note: Partial text from the justification in the SID is omitted since it touches requirements, etc. which should be reflected in the main body of the TR as outcomes of the SI.

In recent years, IoT has attracted much attention in the wireless communication world. More 'things' are expected to be interconnected for improving productivity efficiency and increasing comforts of life. Further reduction of size, complexity, and power consumption of IoT devices can enable the deployment of tens or even hundreds of billions of IoT devices for various applications and provide added value across the entire value chain. It is impossible to power all the IoT devices by battery that needs to be replaced or recharged manually, which leads to high maintenance cost, serious environmental issues, and even safety hazards for some use cases, for example, wireless sensors in electrical power, and petroleum industries.

Most of the existing wireless communication devices are powered by batteries that need to be replaced or recharged manually. The automation and digitization of various industries opens numerous new markets requiring new IoT technologies of supporting batteryless devices with no energy storage capability or devices with energy storage that do not need to be replaced or recharged manually.

An example type of application is asset identification, which presently has to resort mainly to barcodes and RFID in most industries. The main advantage of these two technologies is the ultra-low complexity and small form factor of the tags. However, the limited reading range of a few meters usually requires handheld scanning which leads to labor intensive and time-consuming operations, or RFID portals/gates which leads to costly deployments. Moreover, the lack of interference management scheme results in severe interference between RFID readers and capacity problems, especially in case of dense deployment. It is hard to support a large-scale network with seamless coverage for RFID.

In contrast, this study investigates the feasibility of a new IoT technology to open new markets within 3GPP systems, whose number of connections and/or device density can be orders of magnitude higher than existing 3GPP IoT technologies, and which can provide complexity and power consumption orders-of-magnitude lower than existing 3GPP LPWA technologies such as NB-IoT and LTE-MTC.

# 1	Scope

The present document reports on the feasibility of meeting the design targets for relevant use cases of a new 3GPP IoT technology, on the basis of suitable deployment scenarios in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for very-low end IoT applications. It intends to provide a clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology.

In terms of energy storage, the study considers the following device characteristics:

- Pure batteryless devices with no energy storage capability at all, and completely dependent on the availability of an external source of energy.

- Devices with limited energy storage capability that do not need to be replaced or recharged manually.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.840: "Study on Ambient power-enabled Internet of Things".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DO	Device-originated

DT	Device-terminated

DO-A	Device-originated – autonomous

DO-DTT	Device-originated – device-terminated triggered

IoT	Internet of Things

LPWA	Low-power, wide-area

LTE-MTC	Long Term Evolution – Machine Type Communication

NB-IoT	Narrowband IoT

RFID	Radio-frequency identification

# 4	Deployment scenarios, use cases, services

### 4.1	Use cases/services

### 4.1.1	Representative use cases

Two sets or levels of grouping were defined. The first, Grouping A, is on the basis of the deployment environment(s) described for a use case in [2, TR 22.840], and the second, Grouping B, is on the basis of functionality/application described in [2, TR 22.840].

Grouping A:

- Indoor

- Outdoor

- Indoor/outdoor

Grouping B:

- Inventory

- Sensors

- Positioning

- Command

These two groupings are then used to form representative use cases (rUCs) as follows, which are used in Clause 4.2 – Deployment scenarios and connectivity topologies.

- rUC1: Indoor inventory

- rUC2: Indoor sensors

- rUC3: Indoor positioning

- rUC4: Indoor command

- rUC5: Outdoor inventory

- rUC6: Outdoor sensors

- rUC7: Outdoor positioning

- rUC8: Outdoor command

This resulted in the following mapping from SA1 use cases and traffic scenarios onto RAN rUCs:

Table 4.1.1-1: Mapping between RAN representative use cases and SA1 use cases

| rUC | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction |
| rUC2: Indoor sensor | 5.6 Smart homes 5.13 Base station machine room environmental supervision 5.15 Smart laundry 5.20 Smart agriculture 5.23 Smart pig farm 6.2 Cow stable |
| rUC3: Indoor positioning | 5.8 Finding Remote Lost Item 5.9 Location service 5.10 Ranging in a home 5.12 Personal belongings finding 5.14 Positioning in shopping centre 5.21 Museum Guide |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |
| rUC5: Outdoor inventory | 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution |
| rUC6: Outdoor sensor | 5.3 Smart grids 5.19 Forest Fire Monitoring 5.22 Dairy farming 5.24 Smart manhole cover safety monitoring 5.25 Smart bridge health monitoring |
| rUC7: Outdoor positioning | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding |
| rUC8: Outdoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.30 Controller in smart agriculture |

## 4.2	Deployment scenarios and connectivity topologies

Deployment scenarios for Ambient IoT have been studied on the basis of a list of characteristics, and the representative use case(s) applicable to a scenario. The possible descriptions of the characteristics are as follows:

Table 4.2-1: Characteristics of deployment scenarios

| Characteristic | Possible description entries |
| --- | --- |
| Environment (of the device) | Indoor Outdoor Indoor or outdoor |
| Basestation characteristic (if any) | Macro-cell-based deployment Micro-cell-based deployment Pico-cell-based deployment None |
| Connectivity topology | See section 4.2.1 |
| Spectrum | Licensed FDD Licensed TDD Unlicensed |
| Coexistence with existing 3GPP technologies | Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type. Deployed on new sites without an assumption of an existing 3GPP deployment. |
| Traffic assumption | Device-terminated (DT) Device-originated (DO) DO traffic includes DO autonomous (DO-A), and DO device-terminated triggered (DO-DTT) |
| Device characteristic | See Section 4.3: Device A Device B Device C |

The study has considered Ambient IoT deployments in-band to NR, in guard-band of NR, and in standalone band from NR.

### 4.2.1	Connectivity topologies

The following connectivity topologies for Ambient IoT networks and devices are defined for the purposes of the study. In all these topologies, the Ambient IoT device may be provided with a carrier wave from other node(s) either inside or outside the topology. The links in each topology may be bidirectional or unidirectional.

BS, UE, assisting node, or intermediate node could be multiple BSs or UEs, respectively. The mixture of indoor and outdoor placement of such nodes is regarded as a network implementation choice. Account would need to be taken of potential impact on device or node complexity. In the connectivity topologies, this does not imply the existence of multi-hop assisting or intermediate nodes.

#### 4.2.1.1	Topology 1: BS ↔ Ambient IoT device

Figure 4.2.1.1-1: Topology 1

In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a basestation. The communication between the basestation and the ambient IoT device includes Ambient IoT data and/or signalling. This topology includes the possibility that the BS transmitting to the Ambient IoT device is a different from the BS receiving from the Ambient IoT device.

#### 4.2.1.2	Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

Figure 4.2.1.2-1: Topology 2

In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and basestation. In this topology, the intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT. The intermediate node transfers the information between BS and the Ambient IoT device.

#### 4.2.1.3	Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS

Figure 4.2.1.3-1: Topology 3 with downlink assistance

Figure 4.2.1.3-2: Topology 3 with uplink assistance

In Topology 3, the Ambient IoT device transmits data/signalling to a basestation, and receives data/signalling from the assisting node; or the Ambient IoT device receives data/signalling from a basestation and transmits data/signalling to the assisting node. In this topology, the assisting node can be a relay, IAB, UE, repeater, etc. which is capable of ambient IoT.

#### 4.2.1.4	Topology 4: UE ↔ Ambient IoT device

Figure 4.2.1.4-1: Topology 4

In Topology 4, the Ambient IoT device communicates bidirectionally with a UE. The communication between UE and the ambient IoT device includes Ambient IoT data and/or signalling.

### 4.2.2	Deployment scenarios

#### 4.2.2.1	Deployment scenario 1: Device indoors, basestation indoors

With Ambient IoT device indoors and basestation indoors, this deployment scenario is characterized according to Table 4.2.2.1-1.

Table 4.2.2.1-1: Characteristics of deployment scenario 1

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico-cell |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.2	Deployment scenario 2: Device indoors, basestation outdoors

With Ambient IoT device indoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.2-1.

Table 4.2.2.2-1: Characteristics of deployment scenario 2

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1), (2), (3) Note: The location of intermediate or assisting node (if any) is indoor or outdoor |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device C may support Topology (1), (2), (3), Device A may support Topology (2), Device B may support Topology (2), (3) |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.3	Deployment scenario 3: Device indoors, UE-based reader

With Ambient IoT device indoors and UE-based reader, this deployment scenario is characterized according to Table 4.2.2.3-1.

Table 4.2.2.3-1: Characteristics of deployment scenario 3

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | NA |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.4	Deployment scenario 4: Device outdoors, basestation outdoors

With Ambient IoT device outdoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.4-1.

Table 4.2.2.4-1: Characteristics of deployment scenario 4

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, licensed TDD, or unlicensed. |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device C may support Topology (1), (2), (3), Device A may support Topology (2),Device B may support Topology (2), (3) |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.5	Deployment scenario 5: Device outdoors, UE-based reader

With Ambient IoT device outdoors and UE-based reader, this deployment scenario is characterized according to Table 4.2.2.5-1.

Table 4.2.2.5-1: Characteristics of deployment scenario 5

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | NA |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

## 4.3	Device categorization

Ambient IoT devices are characterized in the study according to their energy storage capacity, and capability of generating RF signals for their transmissions.

The study considers that a device has one of the following energy storage capacities:

- Storage capacity 1: No storage at all.

- Storage capacity 2: Up to E1 Joules

- Storage capacity 3: Up to E2 Joules

Editor's note: Values of E1 and E2 are FFS, and it is possible that E1 = E2, in which case storage 2 and 3 could be replaced by a single description such as "limited energy storage".

Relying on these storage capacities, the study considers the following set of Ambient IoT devices:

- Device A: No energy storage, no independent signal generation/amplification, i.e. backscattering transmission.

- Device B: Has energy storage, no independent signal generation, i.e. backscattering transmission. Use of stored energy can include amplification for reflected signals.

- Device C: Has energy storage, has independent signal generation, i.e., active RF components for transmission.

Editor's note: FFS whether to include device function; whether to include a target maximum power consumption for each device; whether/how to describe what stored energy is used for; if combinations of these devices are considered.

# 5	RAN design targets

## 5.1	Device power consumption

For Device A, the power consumption target during transmitting/receiving is [≤ 1 μW] or [≤ 10 μW],

For Device B, the target during transmitting/receiving is such that:

-	Device A power consumption ≪ Device B power consumption < Device C power consumption; or

-	Device A power consumption ≤ Device B power consumption < Device C power consumption.

The device power consumption for Device C is ≤ 1 mW to ≤ 10 mW.

## 5.2 	Device complexity

Editor’s note: In RAN#99, the following is a working assumption.

For Device A, the complexity target is to be comparable to UHF RFID ISO18000-6C (EPC C1G2).

For Device B, the target is such that:

-	Device A complexity < Device B complexity < Device C complexity.

For Device C, the complexity target is to be orders-of-magnitude lower than NB-IoT.

## 5.3	Coverage

## 5.4	User experienced data rate

The user experienced data rate target is, at least for the uplink, maximum not less than 5 kbps, and minimum not less than 0.1 kbps.

## 5.5	Maximum message size (or maximum ‘TB’ size)

## 5.6	Latency

## 5.7	Positioning accuracy

## 5.8	Connection/device density

## 5.9 	Moving speed of device

Editor’s note: It is FFS on absolute speed, relative speed, or both.

# 6	 Comparison and assessment

## 6.x	Feasibility assessment

## 6.y	Required functionalities

The assumptions on required functionality have been studied on the basis of supporting certain RAN design targets as well as other requirements. At least the following potential functionalities are identified in different sets, respectively, according to the purpose of each functionality assumed to be mainly used for.

Required functionality set #1: for supporting RAN design target

| Design target | Functionality |
| --- | --- |
| Device power and complexity |  |
| Coverage |  |
| User experienced data rate |  |
| Maximum message size |  |
| Latency |  |
| Positioning accuracy |  |
| Connection/Device density |  |
| Moving speed of device |  |

Required functionality set #2: for supporting other requirements

| Requirement | Functionality |
| --- | --- |
| Device management |  |
| Security* |  |
| Mobility |  |
| Interference management and coexistence |  |
| CN connectivity |  |
| Forward compatibility |  |

*Note: This does not necessarily mean security has RAN impact, further study is needed.

# 7	Conclusions and recommendations

# Annex A: Energy sources for energy harvesting

Companies have reported the following energy sources for energy harvesting in literature: RF, solar/light, piezoelectric (kinetic/vibration), electromagnetic, electrostatic, heat/thermal, thermoelectric, magnetic, wind/water, acoustic, etc.

Editor's note: The location of this text in the TR is TBD.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-12 | RAN#98e | RP-223073 |  |  |  | Skeleton for Study on Ambient IoT (Internet of Things) in RAN | 0.0.1 |
| 2022-12 | RAN#98e | RP-223526 |  |  |  | Skeleton for Study on Ambient IoT (Internet of Things) in RAN | 0.0.2 |
| 2023-03 | RAN#99 | RP-230419 |  |  |  | TR 38.848 v0.1.0 Study on Ambient IoT (Internet of Things) | 0.1.0 |
| 2023-06 | RAN#100 | RP-231208 |  |  |  | TR 38.848 v0.2.0 Study on Ambient IoT (Internet of Things) | 0.2.0 |