| 3GPP TR 26.854 V0.3.0 (2024-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects Study on Haptics in 5G Media Services (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Background	11

4.1	Introduction	11

4.2 	Definition of Haptics Media	12

4.3 	Haptics media pipeline	12

4.3.1 	Haptics media creation	13

4.3.2 	Haptic media rendering	14

4.4 	Example of Haptics media devices	14

5	Use cases and requirements	14

5.1 	Introduction	14

5.2 	Haptics enhanced media distribution	14

5.3	Haptic-enhanced Communication	16

5.4	Immersive Entertainment	17

5.5	Immersive multi-modal XR and metaverse	18

5.X	Use Case Template	20

6	Haptics signals, media formats and device types	21

6.1	Introduction	21

6.2	Haptics signals	21

6.2.1	Introduction	21

6.2.2	 Time-sampled signals	21

6.2.3	Synthetic signals	22

6.2.4	 Comparison and applications	22

6.3	Haptic media formats	23

6.3.1 	Introduction	23

6.3.2	 PCM format	23

6.3.4	 Proprietary parametric formats	23

6.3.5	Standardized and Interchangeable parametric formats	23

6.4	Haptics device types	24

6.4.1	Introduction	24

6.4.2	Haptics device type 1: Basic sensory feedback	24

6.4.3	Haptics device type 2: Sensorial texture feedback or Spatial sensory feedback	24

6.4.4	Haptics device type 3: Full-body and complex motion feedback	25

6.4.5	Haptics device type 4: Interactive and spatialised feedback	25

7	Candidate technologies	25

7.1	Codecs	25

7.1.1	MPEG Haptics Coding	25

7.1.1.1	Overview	25

7.1.1.2	MPEG Haptics Codec Architecture	26

7.1.1.3 Integration in MPEG-SD	26

7.1.2	IEEE	26

7.1.2.1 Overview	26

7.2	Storage format	27

7.3	Transport protocols	27

Haptics media delivery over DASH	27

7.3.2	Haptics top-level media type and subtypes	27

7.3.3	Haptics media RTP payload	27

7.3.4	OpenXR APIs	28

8	3GPP services	28

8.1 	 Haptics media integration in the Generalized Media Delivery architecture	28

8.1.1	overview	28

8.1.2	Network functions and UE entities	28

8.1.3. 	Haptics media integration in XR Split rendering architecture	29

8.1.3.1 Overview	29

8.2.2	 Network functions and UE entities	30

8.2	 Haptics media integration in IMS architecture.	30

8.2.1 Overview.	30

8.2.2 Network functions and UE entities	31

8.3	 Haptics media integration in 3GPP services	31

8.3.1	5G Media Streaming services	31

8.3.2	Broadcast services	32

8.3.3	Real time media communication	33

8.3.3.1 WebRTC services	33

8.3.3.2 IMS based RTC services	33

8.4 Identified gaps.	34

9 Typical haptics media traffic characteristics	34

9.1 Typical traffic characteristics summarized per use cases and formats.	34

10 QoS	35

10.1 Introduction	35

10.2 A-synchronicity between haptics and other media	35

11	Conclusions and proposed next steps	36

Annex <A> (informative): Example of a JSON HJIF file 	37

Annex <C> (informative): Bibliography	53

Annex <X> (informative): Change history	55

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This clause is mandatory; do not alter the text in any way other than to choose between "Specification" and "Report".

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

This clause shall start on a new page.

The present document …

Editor’s note: SID objectives

-	Identify relevant use cases and requirements already defined in TR 22.847, TR 22.856 related to the integration of haptics into media streaming and communication services, and refine them as necessary.

-	Collect and document market-relevant device types and technologies (for example existing APIs)  providing support for haptic playback and/or capture.

-	Collect and describe the candidate input and representation formats for haptic experience, relevant to the above use cases and device types.

-	Define relevant QoE metrics for suitable haptics experiences

-	Identify 3GPP streaming and communication services that may benefit from the haptics media type and identify the gaps and requirements need to integrate haptics experiences.

-	Identify candidate technologies that address the gaps including digital representation and compression formats, transport formats and potential QoS requirements in context with the services.

-	Provide potential needs and benefits for normative work

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR22.847: “Study on supporting tactile and multi-modality communication services”

[3]	3GPP TR26.813: “Study of Avatars in Real-Time Communication Services”

[4]	3GPP TR22.856: “Study on Localized Mobile Metaverse Services”

[5]	Haptic editor for full-body immersive experiences, In Proceedings of the 24th ACM Symposium on Virtual Reality Software and Technology 2018

[6]	Khronos: “OpenXR 1.1.40 Specification (with all registered extensions)”

[7] 	ISO/IEC 23090-31 Information technology — Coded representation of immersive media Part 31: Haptics coding

[8]	bHaptics Designer: https://docs.bhaptics.com/designer/ retrieved Sept 15th 2024.

[9]	Meta Haptics Studio : https://www.meta.com/experiences/meta-haptics-studio/6759764157450104/ retrieved Sept 15th 2024.

[10]	Interhaptics Haptic Composer: https://www.interhaptics.com/tech/haptic-composer retrieved Sept 15th 2024.

[11]		Meta Haptics SDK for Unity: https://developers.meta.com/horizon/documentation/unity/unity-haptics-sdk retrieved Sept 15th 2024.

[12]	Meta Haptics SDK for Unreal: https://developers.meta.com/horizon/documentation/unreal/unreal-haptics-sdk, retrieved Sept 15th 2024.

[13]		bHaptics SDK: https://docs.bhaptics.com/sdk/ , retrieved Sept 15th 2024.

[14]	Apple Core Haptics SDK: https://developer.apple.com/documentation/corehaptics, retrieved Sept 15th 2024.

[15] 		Interhaptics Platform: https://www.interhaptics.com/platform-partner/ , retrieved Sept 15th 2024.

[16]	Qualcomm  Snapdragon G3X gen2: https://docs.qualcomm.com/bundle/publicresource/87-69021-1_REV_A_Snapdragon_G3x_Gen_2_Gaming_Platform_Product_Brief.pdf, retrieved Sept 15th 2024.

[17]	SMPTE ST 2100-1:2017 “Definition and Representation of Haptic-Tactile Essence for Broadcast Production Applications”.

[18]	Apple Haptic and Audio Pattern JSON format for Haptics https://developer.apple.com/documentation/corehaptics/representing_haptic_patterns_in_ahap_files, retrieved Sept. 2024.

[19]	https://www.immersion.com/technology/ retrieved Sept. 15th. 2024.

[20]		I. M. Vogels, “Detection of temporal delays in visual-haptic interfaces,” Human Factors, vol. 46, no. 1, pp. 118–134, 2004

[21] 				https://dl.acm.org/doi/pdf/10.1145/3625468.3647626 ; retrieved nov.11 2024.

[22]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[23]				3GPP TS 26.264: “IMS-based AR Real-Time Communication”

[24]				3GPP TS 26.565: "Split Rendering Media Service Enabler".

[24]				3GPP TS 23.501: “System architecture for the 5G System (5GS)”

[25] 	ISO/IEC 23090-32, “Information technology — Coded representation of immersive media — Part 32: Carriage of Haptics Data”

[26] 		ISO/IEC 23090-37, “Information technology — Coded representation of immersive media — Part 37: Conformance and Reference Software for Carriage of Haptics Data”

[27]	https://www.iana.org/assignments/media-types/media-types.xhtml#haptics retrieved Oct 24th.2024

[28]		https://datatracker.ietf.org/doc/draft-ietf-mediaman-haptics/, retrieved Oct 24th, 2024.

[29]		https://datatracker.ietf.org/doc/html/draft-ietf-avtcore-rtp-haptics, retrieved Oct 24th, 2024.

[30] 	1918.1.1-2024, “IEEE Standard for Haptic Codecs for the Tactile Internet”

[31]		ISO/IEC 23090-14, “Information technology — Coded representation of immersive media Part 14: Scene description”

[32]	3GPP TS 26.119: “Media Capabilities for Augmented Reality”

[33]	3GPP TS 26.501: “5G Media Streaming (5GMS); General description and architecture”

[34]	3GPP TS 26.506: “5G Real-time Media Communication Architecture”

[35]	3GPP TS 26.113: “Real-Time Media Communication; Protocols and APIs”

[HAPT]	xxx

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

AI	Artificial Intelligence

AHAP	Apple Haptic and Audio Pattern

AVH	Audio Video Haptics

ERM	Eccentric Rotating Mass

HJIF	Haptic JSON Interchanged Format

HMD	Head Mounted Display

JSON	JavaScript Object Notation

LRA	Linear Resonant Actuators

MPEG	Motion Picture Expert Group

PCM	Pulse-Code Modulation

SMPTE	Society of Motion Picture Television Experts

RIFF	Resource Interchange File Format

WAV	Waveform Audio File format

XML	Extensible Markup Language

XR	Extended Reality

# 4	Background

## 4.1	Introduction

For the purpose of this document, two different haptic control-loops systems are defined: closed-loop-haptics and open-loop-haptics.

Closed-loop-haptics systems refer to architectures where the user received feedback is based on (local or remote) information (sensory information or simulated information) received from the system following user interactions. Closed-loop-haptics systems are illustrated in figure 4-1.

The haptic feedback is an integral part of the interaction mechanism, it provides necessary information to the user to adjust his input to the system. User inputs are continuously monitored by the system in order to constantly adjust the haptic effects sent to the haptic device. Such closed-loop-haptics system is typically (but not exclusively) used for human–machine interactions, i.e. a human controls a remote machine and the machine responds according to the input control with measures, allowing the human to adapt his behaviour. Rapid response time is therefore mandatory along with ultra reliability which may require new network architectures. Some examples of closed-loop haptic systems are surgical simulators and teleoperation robots.

Closed-loop-haptic systems are out of scope for this feasibility study.

Figure 4-1: closed-loop: remote involvement (top), and local involvement only (bottom).

Open-loop haptics systems refer to architectures where the user received haptic feedback is provided by the system (local or remote) without using sensory or simulated information, i.e., they operate based on pre-defined settings and do not adjust based on the user's interactions. They include unidirectional or bi-directional haptics media transmission with similar requirements/constraints on QoE as those applying to audio and video streaming or real-time communication in the current 3GPP architecture, taking into account human touch perceptual thresholds. Some examples of open-loop haptic systems are vibration feedback in gaming controllers and tactile feedback in wearable devices like smartwatches. The different haptic modalities (including both tactile and kinesthetic) relevant in open loop haptic systems are part of the study. Open-loop haptics systems are illustrated in the figure 4-2 below.

Figure 4-2: open-loop: unidirectional (top ), and bidirectional (bottom).

Note: When a local open-loop system is in place, the Network and Remote system elements in figure 4-2 are not used.

## 4.2 	Definition of Haptics Media

Haptics relates to the sense of touch. As a media, Haptics represents information describing physical feedback rendered for a specific user device and body location. This information is defined as a new media type and is described by a time-based signal (a haptic effect) or a spatial signal (a physical property of an object). The rendering is triggered by a timing information or an interaction. Different Haptic modalities are considered, targeting different human mechanoreceptors (tactile, kinesthetic, proprioception) and thermoreceptors. [7]

## 4.3 	Haptics media pipeline

Haptics can be used as a media type at the same level as audio and video. Figure 4.3-1 depicts an end-to-end streaming pipeline associating audio, video and haptics in the most complete scenario of interactive 3D scenes. A simpler version of the pipeline consists in 2D AVH (Audio-Video-Haptics media) with only 2D assets and no interaction.

Figure 4.3-1 Audio-visual Haptic end-to-end streaming pipeline.

This figure also illustrates the different formats and some existing APIs supporting haptics media.

Figure 4.3-2 depicts an end-to-end real-time communication pipeline associating audio, video and haptics media. In a real-time communication use case, the haptic media is managed similarly to audio and video. The input is captured from input sensors, a camera for the video, a microphone for audio and dedicated haptic sensors (motion, pressure…) for haptics media. Alternatively, the signal can be loaded from the storage, typically a library of encoded effects can be stored and retrieved on the fly and sent to the receiver. The signals are then encoded and distributed using existing coded formats and protocols.

On the receiver side, the various media are decoded and rendered with the appropriate devices, loudspeaker, screen or haptics devices (e.g. vibrotactile).

Figure 4.3-2- Audio-visual Haptic real-time communication pipeline.

Figure 4.3-2 also illustrates the different formats and some existing APIs supporting haptics media.

### 4.3.1 	Haptics media creation

For the creation of haptics effects and their association with A/V content, several tools have been developed and can be used. In Figure 4.3.1-1 an example of such tool, the HFX studio [5] is illustrated. It shows how to create timeline for haptics, with several channels and different haptic effects. It also shows how to design effects for particular body parts on the user.

The main principle of authoring tools is to create effects for the user and the targeted experience, and let the application manage this experience regarding the available rendering devices. For instance, in this picture a haptic effect is generated on the user torso assuming several actuators. If the receiving application does not support a haptic suit as a rendering device but may just be run on a smartphone with a single actuator, the application will select only the first vibrotactile signal from the distributed file or stream. On the opposite more complex receivers might use the full file or stream with more complex setups.

Figure 4.3.1-1- Haptic studio editing tool.

The HFX studio supports the HJIF interchanged format (or mezzanine format) defined in clause 6 of [7] for the creation, editing and distribution of haptic effects.

Other commercial tools exist, and they are generally dedicated to their associated proprietary format or platform:

bHaptics Designer [8]

Meta Haptics Studio [9]

Haptic Composer - Design, Test, & Play Haptics [10]

### 4.3.2 	Haptic media rendering

Several sdks have been provided to integrate haptics into standalone applications or for developing dedicated applications such as Unity and Unreal. Often provided with their proprietary format such as the meta sdk for unity [11] or unreal [12], bHaptics sdk [13] or Apple Core Haptics sdk [14].

The Interhaptics platform [15] is providing both authoring tools and Software Development Kit (SDK) for software providers and OEMs manufacturers.

APIs with the devices are often based on OpenXR [6]. However the current haptic support in OpenXR is limited, but some work is ongoing to extend capabilities.

## 4.4 	Example of Haptics media devices

A large number of haptic devices exist. The simpler devices are the smartphone and wearables integrating vibrotactile devices, usually one. Chipsets support mono or stereo haptics, for instance the Snapdragon G3X gen2 [16]. When several wearables are supported by an application some form of spatialization is then possible.

# 5	Use cases and requirements

The main text of the document should start here, after the above clauses have been added.

The following styles and editing techniques are aimed to help in the formatting of the document using the 3GPP Template: 3GPP_70.dot, available from the 3GPP FTP site (ftp://ftp.3gpp.org/Information).

## 5.1 	Introduction

The following use cases have been extracted from 3GPP TR22.847 [2], TR26.813 [3] and TR22.856 [4]. Only the use cases and requirements where Haptics can be used are considered and detailed. Use cases with similar haptics considerations have been grouped into one.

NOTE: The representative use-cases described in this section aimed at identifying the use, requirements and gaps when considering the support of haptics media signals in the 5GMS streaming and Communication architecture. Some of the functions of the described use-cases may not be fully studied.

Ed Notes: to be reviewed when all use cases are finalised.

Editor’s note 2: We may keep a summary of the scenarios in this section and move the tables with the details of all scenarios in an Annex.

## 5.2 	Haptics enhanced media distribution

Table 5.2-1: use case 3GPP media

Use Case Name:

The Haptics-enhanced media distribution is the basic scenario extending the delivery of audio-visual media with additional channel(s) for haptics. Note: this use-case is not listed in TR 22.847[2], TR 22.856[4] and TR 26.813[3]

Description:

A service provider receives from various content providers, media contents such as movies, live sport feeds, 2D video or audio content (audio books, music,) which are enhanced with haptics. The service provider distributes the content to various devices that can render and augment the Audio-Visual experience with haptics data. The haptic effects are encapsulated in additional channel(s) and delivered synchronously with the AV content to add physical feedback in the form of haptic effects. This haptic enhanced experience is created by the content provider or creator to maximize the user engagement. The user is enticed to this new type of media and consumes more content from his service provider. The UE does the rendering and content adaptation based on the UE haptic device(s) capabilities. In some cases, adaptation and selection into the available haptics data is necessary (either at the UE or by the service provider) Haptic modalities included here: motion, thermal and vibrotactile. Haptic characteristics: 1-way, passive, single-user, low data-rates per channel but several channels.

Categorization

Type: distributed Audio Visual and haptics content. Delivery: broadcast, http streaming Device: smartphones, wearables, seats, suits.

Preconditions

- End-user devices that support the haptic modalities - haptic effects associated to the media content to be delivered

Requirements

Bit-rates = 8-16 kbit/s per channel Message size = TBD Number of Channels = low to high, typically 1 to 32 with non-continuous packets or continuous signal (depending on the format) Delay = TBD Format: both signal (time samples) and parametric Editor’s note: to be refined when additional data is provided.

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering - Availability of sensors to capture live information (accelerometer, pressure, temperature…) - Editing tools to create Haptic effects - Integration into Media applications - Integration into hardware devices Connected devices in various form factors exist that are capable of decoding and rendering haptics (smartphones, headphones, suits, cushions…).

Interoperability considerations

Haptic data should be able to be transmitted and rendered with various applications, environments and devices. Support for an haptics media type is considered with one to several data channels. RTP, ISOBMFF and DASH protocols support for haptics media. Adaptation to the rendering capabilities should be supported. User adaptation could be provided.

Potential Standardization Status and Needs

- Haptic data representation and coding (section 6 and 7) - Haptic data carriage, transport and streaming (section 7) - Integration in 5G architecture and services. (Section 8)

## 5.3	Haptic-enhanced Communication

Table 5.3-1: use case Haptics-enhanced Communication

Use Case Name:

Haptic-enhanced Communication Including: TR 22.856 clause 5.9 “Synchronized predictive avatars” TR 22.856 clause 5.11 “IMS-based 3D Avatar Communication” TR 26.813 UC1 “Avatar Communication”

Description:

This use case addresses communication between people, directly or through an avatar representation. In the simple case, haptics is a new channel added to the traditional communication means, such as i) on text messaging services to add vibrations to emojis, ii) on top of an audio call to enhance the context or just for adding effects, iii) in addition to A/V in a video call to increase the emotional impact. In a more complex scenario, this use case is about one-to-one and multi-party communication (which may be an IMS multimedia telephony call using AR/MR/VR) with spatial audio and haptics rendering, where avatars, audio and haptics of each participant in avatar call are transmitted and spatially rendered in the direction of their geolocation. An avatar call is similar to a video call in that both are visual, interactive, provide live feedback to participants regarding their emotions, attentiveness and other social information. An avatar may interact with the environment, another avatar or object, and, through direct or indirect communication, relay haptics feedback to one or more avatars in the shared space. Similarly, avatar interaction could include throwing a ball towards another participant or using a stick to interact with environmental objects or through another object interact with thermal sinks (cold / hot sources) within the environment or scene through another object. Each participant is equipped with display devices (phones, AR glasses, etc.) with external or built-in headphones and haptics renderer. Note 1: This can include help to people with disabilities. Note 2: Emotional haptic is relevant in this use case. Haptic modalities included here: mostly vibrotactile. Haptic characteristics: Includes 2-way communication (talks, social media, phone calls…) and 1-way (alerts messages, information). Multi-users, low to high data-rates per channel, limited number of channels.

Categorization

Type: 2D/3D audio-video sessions Delivery: Conversational, Split Device: headphones, smartphones, smartwatches

Preconditions

- Video and audio communication devices with associated haptic actuators for physical feedback. - Library of created effects to be sent or/and - Accessories to capture information and generate haptic effects.

Requirements

Bit-rates = 8-16 kbit/s per channel Message size = TBD Number of Channels = limited, between 1 and 4 with sparse packets. Delay = lower than 1 audio frame (< 10 ms) Format: parametric Editor’s note: to be refined when additional data is provided.

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering. Haptic effects library can be used. - Integration into communication applications - Integration into hardware devices Connected devices in various form factors exist that are capable of decoding and rendering haptics (smartwatches, mobile phones, rings…).

Interoperability considerations

Haptic data should be distributed and rendered with various applications, environments and devices. Adaptation to the rendering capabilities should be supported.

Potential Standardization Status and Needs

- Haptics data representation and coding (section 6 and 7) - Haptics support in Scene description and avatar representation - Haptics data carriage, transport and streaming (section 7) - Integration in 5G architecture and services. (Section 8)

## 5.4	Immersive Entertainment

Table 5.4-1: Use-case - Immersive Entertainment

Use Case Name:

Immersive entertainment (live events, sport, gaming, movies, music) Including: TR 22.847 5.6 “Live Event Selective Immersion” TR 22.856 5.6 “Mobile Metaverse for Immersive Gaming and Live Shows” TR 22.856 5.22 “Mobile Metaverse Live Concert”

Description:

Use cases from SA2 include complex scenario for the mobile live metaverse entertainment. This use case built upon the SA2 use cases, and considers simpler experiences covering traditional 2D and future mobile immersive applications The considered media including here are: immersive movies, immersive TV series, social networks, gaming... The later could be multi-party or single user. In this scenario users are playing content (local or streamed) with video (2D or 3D immersive), audio (potentially spatialized) and haptics. The haptic signal is synchronized with the content to add physical feedback in the form of some haptic effects and is also generated from triggers and actions in a gaming environment. The user is using at least a smartphone with vibrotactile actuators, or more advanced devices such as gaming controllers, XR glasses and potentially multi wearable devices to spatialize haptic effects. The gaming use case is the most challenging, as low delay interactive experiences are required. Haptic modalities included here: thermal, kinesthetic and vibrotactile, but acceleration can be used for some events, especially gaming. Haptic characteristics: 2-way, interactive, spatialized, multi-users, potentially significant data-rates per channel and several channels.

Categorization

Type: VR, AR, mobile Delivery: broadcast, streaming, real time interactive communication Device: HMD, headphone, cushion, chair, suits, gaming controller, mobile phone

Preconditions

- Capture sensors for feedback from the live events - End-user devices with haptics actuators, displays and headphones. - Interactive content creation

Requirements

Bit-rates = 8-16 kbit/s for good quality (32-64 kbps for very high quality) per channel Message size = TBD Number of Channels = low to high, typically 1 to 32 with non-continuous packets or continuous signal (depending on the format). Live applications could be continuous if direct sensors information is recorded and distributed. [Delay = lower than 1 video frame (< 25 ms), except for gaming (< 10ms)] Format: both signal (time samples) and parametric. Editor’s note: to be refined when additional data are provided.

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering - Editing tools to create Haptic effects - Integration into Media applications - Integration into hardware devices - Availability of sensors to capture live information (accelerometer, pressure, temperature…) Connected devices in various form factors exist that are capable of decoding and rendering haptics (cushion, controllers, suits…). On site production units can ingest sensors signals and integrate synthesizers to generate haptic effects directly in the production vans for live haptics.

Interoperability considerations

Haptic data should be delivered and rendered with various applications, environments and devices. Adaptation to the rendering capabilities should be supported. User adaptation could be provided.

Potential Standardization Status and Needs

- Haptics data representation and coding: (Section 6 and 7) - Haptics data carriage, transport and streaming (Section 7) - Integration in 5G architecture and services. (Section 8)

## 5.5	Immersive multi-modal XR and metaverse

Table 5.5-1: use case Immersive multi-modal XR and metaverse

Use Case Name:

Immersive multi-modal XR and metaverse (VR, AR, multi-users gaming) From: TR 22.847 5.1 “Immersive multi-modal Virtual Reality (VR) application” TR 26.813 UC3 “Multi-user Gaming” TR 22.856 5.7 “AR Enabled Immersive Experience” TR 22.856 5.12 “Virtual humans in metaverse”

Description:

Immersive multi-modal VR application describes the case of a human interacting with virtual entities in a remote environment such that the perception of interaction with a real physical world is achieved. Users are supposed to perceive multiple senses (vision, sound, touch) for full immersion in the virtual environment. Virtual humans (or digital representations of humans, also referred to as 'avatars' in this use case) are simulations of human beings on computers. There is a wide range of applications using avatars, such as games, film and TV productions, financial industry (smart adviser), telecommunications (avatars), etc. In the coming era, the technology of virtual humans is one of foundations of mobile metaverse service. A virtual human can be a digital representation of a natural person in a mobile metaverse service, which is driven by the natural person, or a virtual human also can be a digital representation of a digital assistant driven by AI model. Mobile metaverse services offer an important opportunity for socialization and entertainment, where user experience of the virtual world and the real world combine. This use case focuses on the scenario of a natural person's digital embodiment in a metaverse as a location agnostic service experience. A virtual human is customized according to a user's personal characteristics and shape preferences. Users wear motion capture devices, vibrating backpacks, haptic gloves, VR glasses to drive the virtual human in a meta-universe space for semi-open exploration. The devices mentioned above are 5G UEs, which need to collaborate with each other to complete the actions of user and get real-time feedback. Note: This SA2 scenario is the most complex considered in this study. While a metaverse platform might require new architectures, this scenario is considered here for the purpose of evaluating the coding and representation formats for haptics and their capability for extensions in current architecture to support future complex scenario. Haptic modalities included here: force, motion, thermal and vibrotactile. Haptic characteristics: Bi-directional, interactive, spatialized, multi-users, low latency, high data-rates.

Categorization

Type: VR /AR/XR Delivery: streaming, Split, conversational Device: HMD, glove, suit, AR/VR controller, motion platform

Preconditions

- Capture sensors for feedback from the metaverse scene and other users actions - End-user devices with haptic actuators, sensors, force, HMDs and headphones. - Interactive content creation - Bi-directional streaming - VR/AR/metaverse and supporting architecture.

Requirements

Bit-rates = 8-16 kbit/s per channel Message size = TBD Number of Channels = high, typically 6 to 32 with non-continuous packets. [Delay = TBD] Format: mostly parametric (esp. virtual scenes) Editor’s note; to be refined when additional data is provided.

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering - Editing tools to create Haptic effects - Integration into CR/Metaverse environment is considered, but still in development. - Integration into hardware devices Connected devices in various form factors exist that are capable of decoding and rendering haptics (HMD, gloves, controllers, suits…).

Interoperability considerations

Haptic data should be able to be transmitted and rendered with various applications, environments and devices. Adaptation to the rendering capabilities should be supported. User adaptation could be provided.

Potential Standardization Status and Needs

- Haptics data representation and coding (section 6 and 7) - [Haptic data integration into scenes and with avatar representations ] - Haptics data carriage, transport and streaming (section 7) - Integration in 3GPP architecture and services. (Section 8)

## 5.X	Use Case Template

Use Case Name:

xxx

Description:

xxx

Categorization

Type: Delivery: Device: Other:

Preconditions

xxx

Requirements

xxx

Feasibility

xxx

Interoperability considerations

xxx

Potential Standardization Status and Needs

xxx

# 6	Haptics signals, media formats and device types

## 6.1	Introduction

Haptics refers to the sense of touch, and encompasses the generation, manipulation, and perception of tactile sensations, forces, and motions. In the media domain it relates to the capture and rendering of physical information about objects and environments due to interactions with the user. The capture is generally performed with sensors and the rendering with hardware actuators allowing to render different modalities.

In general, three types of modalities are considered, tactile (vibration, temperature, pression), kinesthetic (force) and proprioception (motion, acceleration). Those modalities are rendered in the human body through different mechanoreceptors. The density and properties of those receptors are different depending on the modality and the body parts. In addition, each individual has different sensibility, which can be managed through users’ profiles and adaptation.

Finally, the rendering is performed with specific hardware actuators embedded into user devices. The different Haptic modalities are rendered with different actuators. Typically, tactile sensations can be rendered with vibrotactile (e.g. linear resonant actuators (LRA), eccentric rotating mass (ERM), temperature, wind (fan), force  devices.

Haptics signals correspond to the raw haptics data at the output of the sensors or authoring tools, described in section 6.2

Haptics media formats are the formats representing the haptics signals once processed for use by a production framework, as described in section 6.3. For example, the raw synthetic signal described in 6.2.3 can be transformed in any of the haptics media parametric representation formats described in 6.3.4

## 6.2	Haptics signals

### 6.2.1	Introduction

Haptic signals are essential in capturing and transmitting tactile sensations in a variety of applications, ranging from simple vibrations in mobile devices to complex force feedback in virtual reality systems. Two types of signals exist, time-sampled signals typically captured by sensors, and synthetic signals of haptic effects, typically created from authoring tools.

### 6.2.2		Time-sampled signals

In Figure 6.2-1, two examples of time-sampled signals are provided. The left side shows the samples of 3D angular orientations and linear accelerations of a horse captured with an accelerometer positioned on the horseman. The right side illustrates the haptic signal of a texture (here a carpet) captured from the sound of a pen moving on the surface of the carpet in one direction.

Those haptics time sampled signals are generally used for signals captured from sensors.

Figure 6.2-1 - Example of time sampled signals (left: Horse riding 3D angular orientations and linear accelerations captured with an accelerometer, right: texture captured from sound on one direction).

Generally, the sampling frequency is set between 2kHz and 16kHz (8kHz in Figure 6.2-1) with the haptic perceptual bandwidth being around [5Hz-1kHz], with some variations depending on the haptic modality, rendering device and human haptic receptor. 5Hz and below is particularly significant for force feedback. This leads to a raw data-rate average of 128 kbps per channel.

### 6.2.3	Synthetic signals

Synthetic signals of haptic effects are mostly generated with design tools and thus correspond to synthetic data. In Figure 6.2-2, two examples of synthetic signals are provided, on the left a heartbeat synthetic signal (thus the regularity of the signal), and on the right a synthetic rain effect (quasi random design) with 4 channels corresponding to 4 different target locations on the user.

Figure 6.2-2 - Example of synthetic rendered haptic signal (left: heart beat synthetic signal, right: synthetic rain effect with 4 channels for 4 different locations on the user).

The synthetic aspect of those signals compared to time-sampled signals captured from sensor is obvious. In general, a synthetic signal is used to synthesize a short duration effect and is used only at some events or at dedicated point in time during a gaming or movie experience, by opposition to time-sampled signals that represent a continuous signal sampled at a constant frequency

Synthetic signals need a synthesizer to be converted to a timed signal compatible with the actuators used. The raw bit-rate dependents on the number of effects and their distribution in time.

### 6.2.4		Comparison and applications

The choice between time-sampled signals and synthetic signals depends on the application and the desired level of control. Time-sampled signals are more suitable for applications requiring high fidelity and real-time capture of physical interactions, such as in medical simulations, playback of recorded signals (motion) or fine-grained tactile feedback systems. Synthetic signals, on the other hand, are ideal for applications where flexibility and ease of use are paramount, such as in mobile notifications, communication, movies or gaming, where the haptic feedback needs to be easily adjustable and consistent across a variety of devices. Synthetic signals are also well suited for XR applications, where haptics effect may be attached to events or objects.

## 6.3	Haptic media formats

## 6.3.1 	Introduction

In this clause, haptics media formats are the formats representing the raw data (or haptics signals described in 6.2) and used at the production side (e.g. for encoding, packaging and delivery). Most common media formats for haptics are proprietary. However standardised interchangeable media formats (or mezzanine formats) have now been defined.

## 6.3.2		PCM format

PCM or Pulse Code Modulation is a digital format that represents analog signals in digital format as a series of samples at a regular sampling frequency. It has been defined for audio but can be used to store haptic signals from sensors.

Various file formats support PCM such as the audio WAV format. ISO/IEC MPEG Haptic [7] is using WAV as one of the input file format for time-sampled haptic signals. This type of file format uses different bit depth and data types to store the signal usually interpreted on a [-1 ; 1] range. For this reason, additional headers and metadata is required to specify the actual value ranges, gain or other characteristics of the signal essential for rendering. There are currently no standard defining this type of metadata and therefore the interpretation of the signal is often application dependent.

This PCM format is suitable for storing or distributing sensors data and its implementability has been demonstrated for audio signals. The drawback of the PCM format is its overhead as it is sampled continuously at constant frequency in time as opposed to parametric formats that are much sparser.

## 6.3.4		Proprietary parametric formats

Descriptive haptic is represented through parametric primitives. It can be described by an amplitude, a frequency, a duration, and a waveform shape such as sinusoid, or square. Two types of parametric effects are usually defined: transient and continuous. A transient effect is a short pulse with an amplitude, a position in time or space, and a reference frequency. A continuous effect is represented by a frequency and an amplitude information modulated on the timeline.

Existing proprietary descriptive formats use JSON or XML to describe haptic effects. Two of those proprietary formats are commercially deployed: AHAP from Apple [18] and IVS [19] from immersion corp.

AHAP has been mainly developed for vibrotactile signals and implementation into iOS Core Haptic framework.

IVS was targeting media applications (movies, gaming) and communication (smartphone) using mainly vibrotactile signals described in XML format.  A binary version is available for networks transmission.

HAPT is a vendor-specific haptic coding format based on the RIFF format. It is designed to enable efficient coding of a device-specific haptic effects. [HAPT]

Editor’s note: need to find proper reference for HAPT

These formats may further be encoded and compressed using haptic codecs. The ISO/IEC MPEG Haptic [7] codec supports both the AHAP and IVS formats, while the IEEE codec [30] only supports PCM input signals.

## 6.3.5	Standardized and Interchangeable parametric formats

SMPTE defined the Haptic-Tactile Essence for Broadcast Production Applications [17]. However, it was mainly dedicated to position, orientation, velocity and acceleration.

MPEG HJIF: MPEG defined in section 6 of specification [7] a standardized and interchanged parametric JSON based format (or mezzanine format) for describing haptics effects called HJIF. An example of an HJIF file is provided in Annex A.

This format is compatible with the proprietary AHAP format. Transcoding from and to the other is possible. Similarly, the IVS format can be transcoded from XML to JSON, for most of its functionalities, since similar parametric patterns/functions are used. Transcoding of the primitives is lossless.

In addition, MPEG HJIF can integrate a binary encoded version of a PCM signal.

The main advantage of MPEG HJIF is its flexibility to support various haptic modalities, various parametric formats and its flexibility for heterogeneous end-user devices and experiences. Thanks to its JSON formatting, it is also easy to edit and modify the file.

MPEG HJIF can further be encoded and compressed using MPEG haptic codecs, in a binary format for networks transmission.

## 6.4	Haptics device types

### 6.4.1	Introduction

Haptics devices may be categorized into types defining their primary capability. Each category reflects the device's intended application and the type of haptic experience it provides. Distinction is done between input (sensors) and playback (actuators or rendering devices) functionalities of devices. These two functions can be collocated or be on different devices.

Editor’s note: example of devices should be provided to illustrate the haptic functionalities.

### 6.4.2	Haptics device type 1: Basic sensory feedback

Devices in this category are designed to provide passive, non-spatialised sensory feedback, in applications such as simple mobile game and entertainment, alerting, or communication.

They include actuators integrated into smartphone, smartwatch, wearables, finger UI, headphone, HMD. They usually have a low number of actuators and usually vibrotactile actuators.

In this category it is anticipated that the haptic signals are synthetic haptic effects created by a designer.

In addition, some simple sensors to capture sensory information mostly related to the environment, such as temperature, pressure, sound, humidity can be available into smartphones and wearables. Generally, it consists of a single sensor with time sampled signals recorded or sent.

### 6.4.3	Haptics device type 2: Sensorial texture feedback or Spatial sensory feedback

Devices in this category are designed to deliver highly detailed and precise haptics (e.g., tactile, kinesthetic, proprioception) sensations. These devices simulate the feel of textures, pressure, and other subtle interactions, enabling users to experience intricate touch-based feedback. They are commonly used in applications where detailed touch interaction is crucial, such as virtual reality try-on, objects manipulation, training/education..

Fine-tuned sensory feedback devices include Haptic gloves Haptic touchpads, touch-sensitive surfaces and screens. Haptic modalities here are not restricted to vibrotactile, kinesthetic and mid-air feedback is also considered.

For the capture, sensors in this category capture sensory information related to objects surfaces physical properties. It consists of more complex systems combining tracking and measurement to get the localization and physics. The tracking measures location and speed of the sensors, the measurement unit captures pressure, force feedback and motion to infer smoothness, roughness, relief and other physical properties of a 2D surface.

### 6.4.4	Haptics device type 3: Full-body and complex motion feedback

Devices in this category provide immersive, whole-body sensations, but mostly as passive feedback to the user. They are ideal for enhancing immersion in virtual environments or simulation scenarios by delivering comprehensive, multi-sensory (e.g., vibrotactile, thermal, pneumatic, electrotactile) feedback.

These rendering devices use a network of actuators to simulate a wide range of physical experiences, such as vibrations, impacts, and movements across the body.. Haptics suits and furniture such as racing game seats, motion platforms and simulators are example of Full-Body and complex feedback devices.  Multiple modalities are also provided ranging from vibrations, wind, water spray, heat or motion.

For the capture, sensors in this category capture information related to position, speed and forces. Typical GPS units and accelerometers can be used (such as those integrated into smartphones), as well as pressure sensors. The information is related to position in 3D space with global tensor fields and targeted force vectors.

### 6.4.5	Haptics device type 4: Interactive and spatialised feedback

Devices in this category are focused on providing targeted, spatialised interactive haptics (e.g., tactile, kinaesthetic, proprioception, thermal) feedback. These devices simulate specific actions or interactions, such as button presses or in-game effects, through vibrations or adaptive triggers. This category includes haptic controllers such as game console, phones, HMD or VR controllers offering localized feedback through vibration motors and adaptive triggers to simulate various in-game sensations and interactions.  Usually bi-directional interaction is considered.

Sensors for capturing haptics signals in this category are mostly related to pressure sensors to get a position and force related to an action. Here typically input devices such as phone, HMD or VR controllers are used for feedback but also their sensors are used to capture interactions, motion and forces.

# 7	Candidate technologies

Editor’s note:  existing standards/work done in other SDOs.

## 7.1	Codecs

### 7.1.1	MPEG Haptics Coding

#### 7.1.1.1	Overview

JTC 1/SC 29/WG 7 (MPEG-3DGH) has completed the development of the MPEG Haptics Coding standard at the April 2023 meeting and is pending publication as ISO/IEC 23090-31 [7].

In addition to a codec and bitstream format, this MPEG Haptic coding standard supports multiple haptic modalities and specifies a human readable JSON coded representation format (HJIF) for descriptive and quantized media formats. Haptic primitive from AHAP and IVS parametric signals can be losslessly converted to and from HJIF. The HJIF data model is a flexible hierarchical structure that can describe one or more channels, allowing for mono, stereo or multi-channel haptics media experiences, targeting one or more actuators or devices. Application areas especially targeted for the use of MPEG Haptics coding include media streaming and broadcast, immersive applications and XR services, real time communications, as well as tactile communication, the latter being out of scope of this study.

During the standardization process, MPEG also worked on a reference and conformance software [ISO/IEC 23090-33] that was publicly released.

[An in-depth evaluation of the performances of this reference codec was conducted with both objective metrics (PSNR) and subjective tests (MUSHRA) for different target bitrates and different input test streams. The encoding of parametric input content is usually around 1-4 kbps with lossless coding. The results on PCM input data show that signals encoded with the 2 kbps target bitrate present perceptible distortions with an average PSNR of 24db. Signals encoded with a 8 kbps target bitrates show some distortions, but not annoying, with an average PSNR of 39.17db. Finally signals encoded with a 16kpbs bitrate have no perceptible distortion with an average PSNR of 45.62db]

#### 7.1.1.2	MPEG Haptics Codec Architecture

Figure 7.1.1.2-1 represents the MPEG haptics codec architecture. Media formats supported by the MPEG Haptics representation and coding includes AHAP, HJIF, IVS parametric media formats, as well as the WAV time sampled media format.

The synthesizer is not defined by the standard and is illustrated in this figure to highlight how the MPEG Haptics codec can be integrated with a renderer.

Fig 7.1.1.2-1 – MPEG Haptics codec architecture.

#### 7.1.1.3 Integration in MPEG-SD

MPEG Scene Description [31] is used in IBACS [23] and in [SR-MSE] via MeCAR [32]. To address use-cases in section 5.4 and 5.5 and these 3GPP services, Haptics media may be integrated with a scene description.

MPEG Scene Description provides extensions for the support of haptics media.

Editor’s Note: further details are TBD.

### 7.1.2	IEEE

#### 7.1.2.1 Overview

The IEEE Standard for Haptic Codecs for the Tactile Internet was developed in 2016 and approved in 2024 [30]. This standard specifies 3 different codecs (no-delay kinesthetic codec, delay-robust kinesthetic codec, and a tactile codec) supporting only a time-sampled media format (PCM) and no parametric media formats.

## 7.2	Storage format

A specification for the storage and delivery signaling for haptics media as defined in [7] is currently under development by MPEG and has reached FDIS stage. ISO/IEC 23090-32 [25] defines how a haptics MIHS bitstream can be encapsulated in ISOBMFF media containers. The specification supports single and multi-track encapsulation, where different channels or bands of the haptics media can be stored in separate tracks to enable selective access.

MPEG is also working on a reference software implementation for the storage and delivery aspects defined in ISO/IEC 23090-32 as part of a new specification (ISO/IEC 23090-37 [26]) that has recently been initiated and is expected to be completed by the end of 2025.

## 7.3	Transport protocols

### Haptics media delivery over DASH

ISO/IEC 23090-32 also specifies how haptics media is signaled in an MPEG-DASH manifest (MPD) for adaptive media delivery and defines descriptors to signal information pertaining to the haptics experience to allow a streaming client to select the parts of the haptics media to stream based on playback timeline, network conditions, and/or user interaction.

### 7.3.2	Haptics top-level media type and subtypes

The 'haptics' media type has been documented and registered by IANA as a top-level media type, along with 'audio', 'video', 'application', and others. [27].

Under this top-level media type, the following haptics subtypes are currently registered:

- ivs, haptics/ivs

- hjif, haptics/hjif

- hmpg, haptics/hmpg

The justification for the 'haptics' top-level media type is found in [28]

### 7.3.3	Haptics media RTP payload

An RTP payload format for haptics media is under development in IETF [29] and has reached the working group last call. The draft describes how the haptic data, in MIHS units defined in [7] can be transmitted using the RTP protocol. This RTP payload format enables the transport of the “hmpg” media subtype defined in [28].

Some of the characteristics of the payload formats include:

- The 4 types of MIHS units are indicated in the payload header: initialization, time dependent (temporal), time independent (spatial), and silent units.

- A MIHS unit can be marked as independent or dependent. An independent unit resets the previous haptics effect and correspond to a “sync” MIHS unit as defined in [7].

- 3 payload structures are defined and can be used within a same stream: a single unit payload structure for a single MIHS unit per packet, a fragmented unit payload structure for MIHS units which are too large to be transmitted in a single packet, and an aggregation packet payload structure to transport multiple MIHS units in a single RTP packet.

- The aggregation packet payload structure can be used to transport multiple MISH units that correspond to the same timestamp, single-time aggregation packet (STAP), or to transport multiple MISH units that correspond to different timestamps, multi-time aggregation packet (MTAP).

- In case of congestion control, the draft recommends prioritizing initialization units, to treat silent units as less important, and to use the MIHS unit layer information present in the RTP payload header to prioritize packets.

- The draft also describes SDP considerations, to define additional optional parameters that can be used in the SDP exchange.

### 7.3.4	OpenXR APIs

Details are TBD.

# 8	3GPP services

## 8.1 	 Haptics media integration in the Generalized Media Delivery architecture

### 8.1.1	overview

The Haptics media functionalities in the generalized 5G Media Delivery architecture defined in [33], and [34] are shown in figure 8.1.1-1. Existing interfaces and functions are reused to support Haptic media type and Haptic media transport in 5G services (e.g. 5GMS, RTC) to address the use-cases of section 5.

The architecture is not modified, the haptics media functions are added in the below figures for illustrative purposes.

Figure 8.1.1.-1: Haptics media in the Generalized Media Delivery architecture

### 8.1.2	Network functions and UE entities

As Haptics media can be integrated in 3GPP services as a companion media type to Video and Audio, the media related definitions described in [34] and [33] are extended to describe the haptics media related functions highlighted in the figure 8.1.1-1 as follow:

-	Media AF: An Application Function as defined in clause 6.2.10 of [24] dedicated to Media Delivery, including haptics media delivery.

-	Media AS: An Application Server dedicated to Media Delivery including haptics media delivery. The depicted Haptics media engine support access, coding and delivery function, of user plane haptic media data

-	Media Client: A UE internal function dedicated to Media Delivery comprising:

-	Media Session Handler: An entity on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media session. The media session includes delivery of Haptics media.

-	Media Access Function: An entity on the UE that communicates with the Media AS in order to access and deliver media content, including haptics media. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation, including those related to Haptic media data.

-	Media-aware Application: An application entity on the UE that makes use of 3GPP-defined APIs to invoke the Media Session Handler and/or the Media Access Function in order to support Media Delivery, including haptics media delivery.

### 8.1.3. 	Haptics media integration in XR Split rendering architecture

#### 8.1.3.1 Overview

As expressed in [24], typical use cases for split rendering includes immersive gaming and immersive communication. These use cases, haptic enabled, are also described in section 5 of this document.

Figure 8.1.3.1-1 is based on figure 6.1.4-1 of [24] and illustrates the haptics media functions as part of the SRS, providing support for haptic media coding and delivery, similarly to audio and video media.

Figure 8.1.3.1-1: Haptic media functions in the User Plane Architecture for Split management architecture

On the UE, Figure 8.1.3.1-2, based on Figure 5.1.2-1[24] illustrates the haptics media entities in the XR baseline client.

Figure 8.1.3.1-2 Haptic media entities in the XR Baseline Client architecture

### 8.2.2	 Network functions and UE entities

The haptics media related network functions in the XR end to end split rendering architecture are similar to the one described in section 8.1.2

The haptics media entities on the XR baseline Client consist of:

-   the haptic media codec, handling and decompressing a compressed haptics media bitstream is illustrated in the MAF function of the SRC along with AV codecs.

-   the haptic renderer, handling the rendering of haptics effects using the targeted actuators is illustrated in the light presentation engine function of the SRC.

## 8.2	 Haptics media integration in IMS architecture.

### 8.2.1 Overview.

To support the use-cases in section 5 for IMS based services (MTSI, and IBACS), the existing IMS architecture is not modified. Existing functions and API are reused to enable haptic enhanced IMS based real time communication.

Figure 8.2.1-1, illustrates the haptic media functionalities in the MTSI client defined in figure 4.2 of [22] which is similar to figure 4.2.1 of the AR-MTSI client of [23]

Figure 8.2.1-1: Functional components of an MTSI client with haptic media support.

### 8.2.2 Network functions and UE entities

For MTSI, [22] the network functions such as the MRFP and MGW may handle haptics media similarly to other media for supplemental services.

Referring to Figure 4.3.1 of [23], Generalized IMS DC Architecture to support AR communication, the AR AS, the IMS AD and the MF/MRD handles haptics media similarly to video or speech media.

## 8.3	 Haptics media integration in 3GPP services

### 8.3.1	5G Media Streaming services

The support of Haptic media streaming over 5G would enable the use-cases defined in section 5, particularly those for on demand and live streaming.

The 5GMS architecture [33] defines in table 4.0.1-1 the list of main features of 5GMS, highlighting whether they are specified for the uplink and/or downlink. These features are equally applicable to Haptics media as illustrated in Table 8.3.1-1.

Editor’s note: Priority of the work in the study on integration in 5G streaming services is on the downlink path, rather than on the uplink content publishing and uplink remote control.

Table 8.3.1-1applicability of the 5GMS features to haptics enhanced media streaming services.

| Feature | Procedures applicable for haptics media |  |
| --- | --- | --- |
|  | Downlink media streaming | Uplink media streaming |
| Content hosting | Yes | Not applicable |
| Content publishing | Not applicable | Yes |
| Content preparation | Not defined | Not defined |
| Network assistance | Yes | Yes |
| Dynamic policies | Yes | Yes |
| Remote control | Not applicable | Yes |
| Consumption reporting | Yes | Not applicable |
| QoE metrics reporting | Yes | Not applicable |
| Edge processing | Yes |  |
| eMBMS delivery | Yes | Not applicable |
| Data collection, reporting and exposure | Yes | Yes |
| Service URL handling | Yes | Yes |

In addition to the generalized architecture described in section 8.2 of this document, the 5GMS specification [33] details the UE functions of the 5GMSd client for downlink, in section 4.2.2 and uplink in section 4.2.3.

For handling haptic media, no new UE subfunctions are needed. The following highlights some requirements needed within some of the subfunctions described in [33] to support haptic media:

-	Media Access Client: Accesses media content, such as DASH-formatted media segments, for immediate or delayed consumption. To access haptic media content, the haptic media content needs to be provided in suitable formatted media segments (e.g. Dash segments)

- 	Media encoder and decoder: Decodes or compress the media, such as audio, video or haptics media.

- 	Media Capturing: Devices such as video cameras or microphones that transform an analogue media signal into digital media data. For haptic media, capturing devices, sensors are described in section 6.4 of this document

- 	Metrics Collection and Reporting and Consumption Collection and Reporting: Information specific to haptic media, if any are FFS.

### 8.3.2	Broadcast services

This section is FFS

### 8.3.3	Real time media communication

#### 8.3.3.1 WebRTC services

In addition to the generalized architecture described in section 8.2 of this document, the GA4RTAR specification in section 4.2 [34] details the UE functions of the RTC endpoint. For handling haptic media, no new UE subfunctions are needed. The RTC Access Function is extended to support accessing, coding, and transmitting haptic media.

The protocol stack for a basic RTC endpoint is specified in section 13.1 of [35].

Figure 8.3.3.1-1 illustrates the integration of haptics media support in this protocol stack.

Figure 8.3.3.1-1 Haptics media support in the protocol stack for a basic RTC endpoint

Haptics media is represented at the same level as the other Video and Audio media. While there is no change to the structure of the protocol stacks, RTP payload for Haptics media would be handled similarly to those necessary for Audio and Video.

#### 8.3.3.2 IMS based RTC services

IMS based RTC services include IBACS [23] and MTSI [22], in which Haptics media can be integrated to satisfy the use case of section 5. Figure 4.3 of [22] specifies the user plane protocol stack for a basic MTSI client. It also applies to IBACS.

Figurr 8.3.32-1 illustrates the integration of haptics media support in this protocol stack.

Figure 8.3.3.2-1 Haptics media support in the user plane protocol stack for a basic MTSI endpoint

Haptics media is represented at the same level as the other media types. While there is no change to the structure of the protocol stacks, Payload formats and RTP payload for Haptics media would be handled similarly to those necessary for Audio and Video.

## 8.4 Identified gaps.

This section is TBD and will summarize the gaps identified in previous sections.

## 9 Typical haptics media traffic characteristics

### 9.1 Typical traffic characteristics summarized per use cases and formats.

In general, haptics media PCM coded bitstreams require substantially more bandwidth than parametric coded bitstreams, this is due to the capability of having silent units in a parametric bitstream.

When coding repetitive haptics media effect, a key difference between a parametric coded bitstream and a pcm coded bitstream on the traffic characteristic is the following:

- 	In a parametric coded bitstreams, identical or similar effects can be coded and sent a single time and then referenced, rather than coding multiple time the same or similar haptics media effect

- 	In a pcm coded bitstream, similarly to audio and video, coding performance improves for these repeated consecutive effects but provide a much higher overhead compared to the parametric coded bitstream.

The following table summarise the expected characteristics and average bitrates per channel applicable for the use cases of section 5. In this table the density represents the quantity of haptics effects over the duration of the sequence, it is not related to the intensity of the haptics effect. Three parameters impact the bandwidth requirements: the number of channels, the media format and the density.

Table 9.1-1: summary of typical haptics media traffic characteristic.

| Use case | Nb of channels | Media Format | Density | Average bitrate per channel |
| --- | --- | --- | --- | --- |
| Haptic enhanced media distribution (5.2) | 1 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 1 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density. 32-64 kbps for very high quality at medium and high density. |
| Haptic enhanced communication (5.3) | 1 to 4 | Parametric | Light Medium | 0.25 to 0.75 kbps 0.5 to 2 kbps |
| Immersive Entertainment (5.4) | 1 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 1 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density 32-64 kbps for very high quality at medium and high density |
| Immersive multi-modal XR and metaverse (5.5) | 6 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 6 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density 32-64 kbps for very high quality at medium and high density* |

* Note1: TR 22.847, mentioned a bitrate of up to 200kpbs, which may be related to use cases for close-loop tactile internet, robotics etc.

Note 2: values in the table may be re-evaluated if more tests are done.

# 10 QoS

## 10.1 Introduction

TBD

## 10.2 A-synchronicity between haptics and other media

In Haptics enhanced media services, the quality of experience may be impacted by the a-synchronicity of the haptics media with either audio, or visual information. This a-synchronicity may be due to various QoS factors which are FFS.

Studies demonstrate (such as [21] and [20]) that haptics effects perceived prior visual cues is less detrimental to the experience than the reverse and that a-synchronicity is more tolerable in passive than active user involvement.

The suitable threshold for a-synchronicity [20], [21], [2], [4] is listed in table Y.2-1

Table 10.2-1: Tolerable a-synchronicity thresholds per use-cases

| Use case | Media | Tolerable a-synchronicity threshold (note 1) |  |
| --- | --- | --- | --- |
| Haptic enhanced media distribution (5.2) | audio-haptics | audio delay: TBD | haptic delay: TBD |
|  | visual-haptics | visual delay: TBD | Haptic delay: TBD |
| Haptic enhanced communication (5.3) | audio-haptics | audio delay: TBD | haptic delay: TBD |
|  | Visual-haptics | Visual delay: TBD | Haptic delay: TBD |
| Immersive games (5.4 and 5.5) [2] | audio-haptics | audio threshold: 50 ms | haptic threshold: 25 ms |
|  | visual-haptics | visual delay: 15 ms | Haptic delay: 50 ms |
| Immersive entertainment (5.4) [4] | audio-haptics | audio delay: 25 ms | haptic delay: 12 ms |
|  | visual-haptics | visual delay: 20 ms | Haptic delay: 30ms |
| Immersive multimodal XR and metaverse (5.5) | audio-haptics | audio delay: TBD | haptic delay: TBD |
|  | visual-haptics | Visual delay: TBD | Haptic delay: TBD |
|  | NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

Editor’s Note: Further work needed to complete and refine these values.

# 11	Conclusions and proposed next steps

xxxx

Annex <A> (informative):
Example of a JSON HJIF file

{

"version":"2023",

"profile":"Main",

"level":1,

"date":"Mon Sep 16 15:17:39 2024\n",

"description":"Ringtone effect",

"timescale":1000,

"avatars":[

],

"perceptions":[

{

"id":0,

"avatar_id":0,

"description":"Vibration effect",

"perception_modality":"Vibrotactile",

"unit_exponent":-3,

"perception_unit_exponent":0,

"reference_devices":[

],

"effect_library":[

],

"channels":[

{

"id":0,

"description":"Left Hand Palm",

"gain":1.0,

"mixing_coefficient":0.9999983310699463,

"body_part_mask":131072,

"bands":[

{

"band_type":"VectorialWave",

"lower_frequency_limit":0,

"upper_frequency_limit":1000,

"effects":[

{

"effect_type":"Basis",

"position":74,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":195,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":542,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":20,

"amplitude_modulation":0.5921568870544434

},

{

"relative_position":77,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":844,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":118,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":1004,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

},

{

"relative_position":91,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":1320,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":118,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":1651,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":20,

"amplitude_modulation":0.5921568870544434

},

{

"relative_position":77,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":1937,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

},

{

"relative_position":91,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":2242,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":0,

"amplitude_modulation":0.46666669845581055

},

{

"relative_position":45,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":2308,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.27843141555786133,

"frequency_modulation":200

},

{

"relative_position":68,

"amplitude_modulation":0.27843141555786133,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":2548,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":195,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":3016,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":20,

"amplitude_modulation":0.5921568870544434

},

{

"relative_position":77,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":3318,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":118,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":3478,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

},

{

"relative_position":91,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":3794,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":118,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":4099,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":20,

"amplitude_modulation":0.5921568870544434

},

{

"relative_position":77,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":4411,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

},

{

"relative_position":91,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":4716,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":0,

"amplitude_modulation":0.46666669845581055

},

{

"relative_position":45,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":4782,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.27843141555786133,

"frequency_modulation":200

},

{

"relative_position":68,

"amplitude_modulation":0.27843141555786133,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":5023,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":108,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":5487,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":57,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":5640,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.34117650985717773,

"frequency_modulation":200

},

{

"relative_position":86,

"amplitude_modulation":0.34117650985717773,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":5796,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":118,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":5949,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

},

{

"relative_position":91,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":6265,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":118,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":6570,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":20,

"amplitude_modulation":0.5921568870544434

},

{

"relative_position":77,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":6882,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

},

{

"relative_position":91,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":7187,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":0,

"amplitude_modulation":0.46666669845581055

},

{

"relative_position":45,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":7253,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.27843141555786133,

"frequency_modulation":200

},

{

"relative_position":68,

"amplitude_modulation":0.27843141555786133,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":7493,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":107,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":7961,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":20,

"amplitude_modulation":0.5921568870544434

},

{

"relative_position":77,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":8271,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":118,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":8423,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

},

{

"relative_position":91,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":8739,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

},

{

"relative_position":118,

"amplitude_modulation":0.12156867980957031,

"frequency_modulation":142

}

]

},

{

"effect_type":"Basis",

"position":9044,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":20,

"amplitude_modulation":0.5921568870544434

},

{

"relative_position":77,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":9356,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

},

{

"relative_position":91,

"amplitude_modulation":0.4431372880935669,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":9661,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.5921568870544434,

"frequency_modulation":200

},

{

"relative_position":0,

"amplitude_modulation":0.46666669845581055

},

{

"relative_position":45,

"amplitude_modulation":-0.003921568393707275,

"frequency_modulation":200

}

]

},

{

"effect_type":"Basis",

"position":9727,

"phase":0.0,

"base_signal":"Sine",

"keyframes":[

{

"relative_position":0,

"amplitude_modulation":0.27843141555786133,

"frequency_modulation":200

},

{

"relative_position":68,

"amplitude_modulation":0.27843141555786133,

"frequency_modulation":200

}

]

}

]

}

]

}

]

}

],

"syncs":[

{

"timestamp":0,

"timescale":1000

}

]

}

Annex <C> (informative):
Bibliography

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Bibliography is optional. If it exists, it shall follow the last technical annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

<Publication>: "<Title>".

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | S4-129e |  |  |  |  | Proposed skeleton TR | 0.0.1 |
| 2024-08 | S4-129e | S4-241506 S4-241771 S4-241770 S4-241761 S4-241769 |  |  |  | Includes: Use Cases for Immersive entertainment (S4-241506), Haptics-enhanced Communication (S4-241771) Immersive multi-modal XR and metaverse (S4-241770); Device types (S4-241761); input formats (S4-241769) Editor’s clean-up | 0.1.0 |
| 2024-09 | RTC adhoc call sept 25th | S4aR240067 S4aR240048 S4aR240049 S4aR240050 S4aR240052 |  |  |  | Includes: new use-case for haptics enhanced media distribution (S4aR240067); revision to use-cases (S4aR240048, S4aR240049, S4aR240050); haptics media definition and pipeline (S4aR240052) Editor’s clean-up | 0.1.1 |
| 2024-10 | RTC adhoc call Oct 23rd | S4aR240077 |  |  |  | Includes: S4aR240077 Editor’s clean-up | 0.2.0 |
| 2024-11 | S4#131 | S4-241852 S4-241854 S4-242099 S4-242100 S4-241856 S4-241857 S4-241858 S4-241859 |  |  |  | Includes: communication pipeline (S4-241852), control loops (S4-241854), update device types (S4-242099),define haptics signals and media formats (S4-242100), haptics media traffic characteristics (S4-241856), integration in 3GPP architecture (S4-241857), integration in 3GPP services (S4-241858) candidate technologies (S4-241859), Update to haptic communication use case (S4-241859) Editor’s clean-up. | 0.3.0 |