| 3GPP TS 31.121 V18.1.0 (2024-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; UICC-terminal interface; Universal Subscriber Identity Module (USIM) application test specification (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	35

Introduction	36

1	Scope	35

2	References	35

3	Definitions and abbreviations	38

3.1	Definitions	38

3.2	Symbols	39

3.3	Abbreviations	39

3.4	Coding Conventions	41

3.5	Generic procedures for E-UTRAN/UTRAN/GERAN/IMS/NB-IoT/5G-NR	42

3.6	Applicability	43

3.6.1	Applicability of the present document	43

3.6.2	Applicability to terminal equipment	43

3.6.3	Applicability of the individual tests	43

3.7	Table of optional features	44

3.8	Applicability table	48

4	Default values for UICC	65

4.1	Definition of default values for USIM-Terminal interface testing (Default UICC)	65

4.1.1	Values of the EF's (Default UICC)	65

4.1.1.1	EFIMSI (IMSI)	65

4.1.1.3	EFLOCI (Location Information)	66

4.1.1.4	EFKeys (Ciphering and Integrity Keys)	66

4.1.1.5	EFKeysPS (Ciphering and Integrity Keys for Packet Switched domain)	66

4.1.1.6	EFACC (Access Control Class)	66

4.1.1.7	EFFPLMN (Forbidden PLMNs)	66

4.1.1.8	EFUST (USIM Service Table)	66

4.1.1.9	EFEST (Enable Service Table)	67

4.1.1.10	EFADN (Abbreviated Dialling Number)	67

4.1.1.11	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	67

4.1.1.12	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	68

4.1.1.13	Void	69

4.1.1.14	PIN	69

4.1.1.15	PIN2	69

4.1.1.16	Unblock PIN	69

4.1.1.17	Unblock PIN2	69

4.1.1.18	Other Values of the USIM	69

4.1.1.19	EFPSLOCI (Packet Switch Location Information)	69

4.1.1.20	Universal PIN	70

4.1.1.21	Unblock Universal PIN	70

4A	Default values for NG-SS and UE configurations	70

4A.1	Default NG-SS and UE configuration for different usage settings	70

4A.2	Default NG-SS configuration for 3GPP access with IMS service	70

4A.2.1	Default NG-SS configuration for IMS Registration support	70

4A.2.2	Default NG-SS configuration for PS voice support	70

4A.3	Default NG-SS procedure for 5G Registration on 3GPP access with IMS service	71

4A.4	Default UE configuration for IMS service	71

4.2	Definition of FDN UICC	71

4.2.1	Values of the EF's (FDN UICC)	71

4.2.1.3	EFFDN (Fixed Dialling Numbers)	71

4.2.1.4	EFECC (Emergency Call Codes)	72

4.2.1.5	Other Values of the USIM	72

4.3	Void	73

4.4	Definition of E-UTRAN/EPC UICC	73

4.4.1	EFUST (USIM Service Table)	73

4.4.2	EFEPSLOCI (EPS Information)	73

4.4.3	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	73

4.4.4	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	74

4.4.5	EFACSGL (Allowed CSG Lists)	74

4.4.6	EFCSGT (CSG Type)	75

4.4.7	EFHNBN (Home (e)NodeB Name)	77

4.4.8	EFEPSNSC (EPS NAS Security Context)	79

4.5	Definition of E-UTRAN/EPC ISIM-UICC	79

4.5.1	Applications on the E-UTRAN/EPC ISIM-UICC	79

4.5.2	Default USIM values on E-UTRAN/EPC ISIM-UICC	79

4.5.3	Default ISIM values on E-UTRAN/EPC ISIM-UICC	79

4.5.3.1	EFAD (Administrative Data)	79

4.5.3.3	EFIMPI (IMS private user identity)	80

4.5.3.4	EFDOMAIN (Home Network Domain Name)	80

4.5.3.5	EFIMPU (IMS public user identity)	80

4.5.3.6	EFP-CSCF (P-CSCF ADDRESS)	81

4.5.3.7	EFSMS (Short Message Service)	81

4.5.3.8	EFSMSR (Short message status reports)	82

4.5.3.9	EFSMSP (Short message service parameters)	82

4.5.3.10	EFSMSS (SMS Status)	82

4.5.4	Default values at DF_TELECOM	82

4.5.4.1	EFPSISMSC (Public Service Identity of the SM-SC)	82

4.6	Definition of ACSGL/OCSGL E-UTRAN/EPC UICC	83

4.6.1	EFUST (USIM Service Table)	83

4.6.2	EFAD (Administrative Data)	83

4.6.3	EFOCSGL (Operator CSG Lists)	83

4.6.4	EFOCSGT (Operator CSG Type)	84

4.6.5	EFOHNBN (Operator Home (e)NodeB Name)	86

4.7	Definition of Non Access Stratum Configuration UICC	87

4.7.1	Values of the EFs	87

4.7.1.2	EFNASCONFIG (Non Access Stratum Configuration)	87

4.8	Definition of Non Access Stratum Configuration of E-UTRAN/EPC UICC	88

4.8.1	EFUST (USIM Service Table)	88

4.8.2	EFEPSLOCI (EPS Information)	88

4.8.3	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	88

4.8.4	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	89

4.8.5	EFACSGL (Allowed CSG Lists)	89

4.8.6	EFCSGT (CSG Type)	90

4.8.7	EFHNBN (Home (e)NodeB Name)	92

4.8.8	EFEPSNSC (EPS NAS Security Context)	93

4.8.9	EFNASCONFIG (Non Access Stratum Configuration)	93

4.9	Definition of 5G-NR UICC	94

4.9.0	Introduction	94

4.9.2	EFIMSI (IMSI)	94

4.9.3	EF5GS3GPPLOCI (5GS 3GPP location information)	95

4.9.4	EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)	95

4.9.5	EFRouting_Indicator (Routing Indicator EF)	96

4.9.6	EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)	96

4.9.7	EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)	96

4.9.8	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	97

4.9.9	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	98

4.9.10	Void	98

4.10	Definition of 5G-NR UICC – non-IMSI SUPI Type	98

4.10.1	EFUST (USIM Service Table)	98

4.10.2	EFIMSI (IMSI)	99

4.10.3	EFSUPI_NAI (SUPI as Network Access Identifier)	99

4.10.4	EF5GS3GPPLOCI (5GS 3GPP location information)	99

4.10.5	EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)	99

4.10.6	EFRouting_Indicator (Routing Indicator EF)	100

4.10.7	EFAD (Administrative Data)	100

4.11	5G-NR UICC – support of Rel-16 features	101

4.11.1	Introduction	101

4.11.2	EFUST (USIM Service Table)	101

4.11.3	EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)	101

4.11.4	EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)	102

4.11.5	EF5GAUTHKEYS (5G Authentication Keys)	103

4.12	Definition of CAG 5G-NR UICC	103

4.12.1	EFUST (USIM Service Table)	103

4.12.2	EFCAG (Pre-configured CAG information list)	103

4.13	Definition of 5G-NR ISIM-UICC	104

4.13.1	Applications on the 5G-NR ISIM-UICC	104

4.13.2	Default ISIM values on 5G-NR ISIM-UICC	104

4.14	Default WLAN AP and UE configuration for non-3GPP access	104

4.14.1	Default NG-SS WLAN AP configuration	104

4.14.1.1	WLAN AP frequency	104

4.14.1.2	WLAN AP default parameters	104

4.14.2	Default WLAN configuration for the UE	104

4.14.2.1	N3IWF Identifier configuration	104

4.14.2.2	Non-3GPP access node selection information	105

4.15	Definition of 5G-NR UICC supporting Rel-17 features	105

4.15.0	Introduction	105

4.15.2	EFMCHPPLMN (Multiplier Coefficient for Higher Priority PLMN search)	105

4.15.3	EFHPLMNwACT (HPLMN selector with Access Technology)	105

4.15.4	EFNASCONFIG (Non Access Stratum Configuration)	106

4.15.5	EFSOR-CMCI (Steering Of Roaming - Connected Mode Control Information)	106

4.16	Definition of 5G-NR SSIM-UICC	107

4.16.0	Introduction	107

4.16.1	Applications on the 5G-NR SSIM-UICC	107

4.16.2	Default SSIM values on 5G-NR SSIM-UICC	107

4.16.2.1	EFEAPID (EAP Identifier)	107

4.16.2.2	EFNSSAI (S-NSSAI List)	108

4.17	Definition of 5G-NR UICC supporting Rel-18 features	108

4.17.0	Introduction	108

4.17.2	EFOCST ("Operator controlled signal threshold per access technology")	108

5	Subscription related tests	108

5.1	IMSI / TMSI handling	108

5.1.1	UE identification by short IMSI	108

5.1.1.1	Definition and applicability	108

5.1.1.2	Conformance requirement	108

5.1.1.3	Test purpose	108

5.1.1.4	Method of test	108

5.1.1.4.1	Initial conditions	108

5.1.1.4.2	Procedure	108

5.1.1.5	Acceptance criteria	109

5.1.2	UE identification by short IMSI using a 2 digit MNC	109

5.1.2.1	Definition and applicability	109

5.1.2.2	Conformance requirement	109

5.1.2.3	Test purpose	109

5.1.2.4	Method of test	109

5.1.2.4.1	Initial conditions	109

5.1.2.4.2	Procedure	110

5.1.2.5	Acceptance criteria	110

5.1.3	UE identification by "short" TMSI	111

5.1.3.1	Definition and applicability	111

5.1.3.2	Conformance requirement	111

5.1.3.3	Test purpose	111

5.1.3.4	Method of test	111

5.1.3.4.1	Initial conditions	111

5.1.3.4.2	Procedure	112

5.1.3.5	Acceptance criteria	112

5.1.4	UE identification by "long" TMSI	112

5.1.4.1	Definition and applicability	112

5.1.4.2	Conformance requirement	112

5.1.4.3	Test purpose	112

5.1.4.4	Method of test	113

5.1.4.4.1	Initial conditions	113

5.1.4.4.2	Procedure	113

5.1.4.5	Acceptance criteria	113

5.1.5	UE identification by long IMSI, TMSI updating and key set identifier assignment	114

5.1.5.1	Definition and applicability	114

5.1.5.2	Conformance requirement	114

5.1.5.3	Test purpose	114

5.1.5.4	Method of test	115

5.1.5.4.1	Initial conditions	115

5.1.5.4.2	Procedure	115

5.1.5.5	Acceptance criteria	116

5.1.6	UE identification by short IMSI when accessing E-UTRAN/EPC	117

5.1.6.1	Definition and applicability	117

5.1.6.2	Conformance requirement	117

5.1.6.3	Test purpose	117

5.1.6.4	Method of test	117

5.1.6.4.1	Initial conditions	117

5.1.6.4.2	Procedure	117

5.1.6.5	Acceptance criteria	118

5.1.7	UE identification by short IMSI using a 2 digit MNC when accessing E-UTRAN/EPC	118

5.1.7.1	Definition and applicability	118

5.1.7.2	Conformance requirement	118

5.1.7.3	Test purpose	118

5.1.7.4	Method of test	118

5.1.7.4.1	Initial conditions	118

5.1.7.4.2	Procedure	119

5.1.7.5	Acceptance criteria	119

5.1.8	UE identification after changed IMSI with service "EMM Information" not available	119

5.1.8.1	Definition and applicability	119

5.1.8.2	Conformance requirement	120

5.1.8.3	Test purpose	120

5.1.8.4	Method of test	120

5.1.8.4.1	Initial conditions	120

5.1.8.4.2	Procedure	120

5.1.8.5	Acceptance criteria	121

5.1.9	UE identification by GUTI when using USIM with service "EMM Information" not available	121

5.1.9.2	Conformance requirement	121

5.1.9.3	Test purpose	121

5.1.9.4	Method of test	121

5.1.9.4.1	Initial conditions	121

5.1.9.4.2	Procedure	122

5.1.9.5	Acceptance criteria	122

5.1.10	UE identification by GUTI when using USIM with service "EMM Information" available	122

5.1.10.1	Definition and applicability	122

5.1.10.2	Conformance requirement	122

5.1.10.3	Test purpose	123

5.1.10.4	Method of test	123

5.1.10.4.1	Initial conditions	123

5.1.10.4.2	Procedure	123

5.1.10.5	Acceptance criteria	123

5.2	Access Control handling	124

5.2.1	Access Control information handling	124

5.2.1.1	Definition and applicability	124

5.2.1.2	Conformance requirement	124

5.2.1.3	Test purpose	125

5.2.1.4	Method of test	125

5.2.1.4.1	Initial conditions	125

5.2.1.4.2	Coding details	125

5.2.1.4.3	Procedure	126

5.2.1.5	Acceptance criteria	126

5.2.2	Access Control information handling for E-UTRAN/EPC	138

5.2.2.1	Definition and applicability	138

5.2.2.2	Conformance requirement	138

5.2.2.3	Test purpose	138

5.2.2.4	Method of test	139

5.2.2.4.1	Initial conditions	139

5.2.2.4.2	Coding details	139

5.2.2.4.3	Procedure	139

5.2.2.5	Acceptance criteria	139

5.2.3	Access Control information handling for NB-IoT	150

5.2.3.1	Definition and applicability	150

5.2.3.2	Conformance requirement	150

5.2.3.3	Test purpose	150

5.2.3.4	Method of test	151

5.2.3.4.1	Initial conditions	151

5.2.3.4.2	Coding details	151

5.2.3.4.3	Procedure	151

5.2.3.5	Acceptance criteria	151

5.3	Handling subscription identifier privacy for 5G	163

5.3.1	SUCI calculation by ME using null scheme	163

5.3.1.1	Definition and applicability	163

5.3.1.2	Conformance requirement	163

5.3.1.3	Test purpose	163

5.3.1.4	Method of test	164

5.3.1.4.1	Initial conditions	164

5.3.1.4.2	Procedure	165

5.3.1.5	Acceptance criteria	165

5.3.2	SUCI calculation by ME using Profile B	166

5.3.2.1	Definition and applicability	166

5.3.2.2	Conformance requirement	166

5.3.2.3	Test purpose	166

5.3.2.4	Method of test	166

5.3.2.4.1	Initial conditions	166

5.3.2.4.2	Procedure	167

5.3.2.5	Acceptance criteria	167

5.3.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	167

5.3.3.1	Definition and applicability	167

5.3.3.2	Conformance requirement	167

5.3.3.3	Test purpose	167

5.3.3.4	Method of test	168

5.3.3.4.1	Initial conditions	168

5.3.3.4.2	Procedure	169

5.3.3.5	Acceptance criteria	169

5.3.4	UE identification by SUCI in response to IDENTITY REQUEST message	169

5.3.4.1	Definition and applicability	169

5.3.4.2	Conformance requirement	170

5.3.4.3	Test purpose	170

5.3.4.4	Method of test	170

5.3.4.4.1	Initial conditions	170

5.3.4.4.2	Procedure	171

5.3.4.5	Acceptance criteria	171

5.3.5	UE identification by SUCI in response to IDENTITY REQUEST message with T3519 timer expiry	172

5.3.5.1	Definition and applicability	172

5.3.5.2	Conformance requirement	172

5.3.5.3	Test purpose	172

5.3.5.4	Method of test	173

5.3.5.4.1	Initial conditions	173

5.3.5.4.2	Procedure	173

5.3.5.5	Acceptance criteria	174

5.3.6	UE identification by SUCI in response to IDENTITY REQUEST message and AUTHENTICATION REJECT	174

5.3.6.1	Definition and applicability	174

5.3.6.2	Conformance requirement	174

5.3.6.3	Test purpose	175

5.3.6.4	Method of test	175

5.3.6.4.1	Initial conditions	175

5.3.6.4.2	Procedure	176

5.3.6.5	Acceptance criteria	176

5.3.7	SUCI calculation by the ME using null scheme – missing parameters for subscription identifier privacy support by the USIM	176

5.3.7.1	Definition and applicability	176

5.3.7.2	Conformance requirement	176

5.3.7.3	Test purpose	177

5.3.7.4	Method of test	177

5.3.7.4.1	Initial conditions	177

5.3.7.4.2	Procedure	177

5.3.7.5	Acceptance criteria	177

5.3.8	UE identification by 5G-GUTI – Last Registered TAI stored on USIM	178

5.3.8.1	Definition and applicability	178

5.3.8.2	Conformance requirement	178

5.3.8.3	Test purpose	178

5.3.8.4	Method of test	178

5.3.8.4.2	Procedure	179

5.3.8.5	Acceptance criteria	179

5.3.9	UE identification by 5G-GUTI – Last Registered TAI stored by ME	179

5.3.9.1	Definition and applicability	179

5.3.9.3	Test purpose	180

5.3.9.4	Method of test	180

5.3.9.4.1	Initial conditions	180

5.3.9.4.2	Procedure	180

5.3.9.5	Acceptance criteria	181

5.3.10	UE identification after SUPI is changed	181

5.3.10.1	Definition and applicability	181

5.3.10.2	Conformance requirement	181

5.3.10.3	Test purpose	181

5.3.10.4	Method of test	182

5.3.10.4.1	Initial conditions	182

5.3.10.4.2	Procedure	182

5.3.10.5	Acceptance criteria	182

5.3.11	SUCI calculation by ME using Profile A	182

5.3.11.1	Definition and applicability	182

5.3.11.2	Conformance requirement	182

5.3.11.3	Test purpose	183

5.3.11.4	Method of test	183

5.3.11.4.1	Initial conditions	183

5.3.11.4.2	Procedure	184

5.3.11.5	Acceptance criteria	184

5.3.12	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A	184

5.3.12.1	Definition and applicability	184

5.3.12.2	Conformance requirement	184

5.3.12.3	Test purpose	185

5.3.12.4	Method of test	185

5.3.12.4.1	Initial conditions	185

5.3.12.4.2	Procedure	186

5.3.12.5	Acceptance criteria	186

5.3.13	SUCI calculation by ME using null scheme– no Protection Scheme Identifier provisioned in the USIM	187

5.3.13.1	Definition and applicability	187

5.3.13.2	Conformance requirement	187

5.3.13.3	Test purpose	187

5.3.13.4	Method of test	187

5.3.13.4.1	Initial conditions	187

5.3.13.4.2	Procedure	187

5.3.13.5	Acceptance criteria	188

5.3.14	SUCI calculation by ME using null scheme – no Home Network Public Key for supported protection scheme provisioned in the USIM	188

5.3.14.1	Definition and applicability	188

5.3.14.2	Conformance requirement	188

5.3.14.3	Test purpose	188

5.3.14.4	Method of test	189

5.3.14.4.1	Initial conditions	189

5.3.14.4.2	Procedure	189

5.3.14.5	Acceptance criteria	189

5.3.15	SUCI calculation by ME using null scheme with the E-UTRAN/EPC UICC	189

5.3.15.1	Definition and applicability	189

5.3.15.2	Conformance requirement	190

5.3.15.3	Test purpose	190

5.3.15.4	Method of test	190

5.3.15.4.1	Initial conditions	190

5.3.15.4.2	Procedure	190

5.3.15.5	Acceptance criteria	190

5.3.16	SUCI calculation by ME using the lower priority protection scheme when the higher priority protection scheme is not supported by the ME	191

5.3.16.1	Definition and applicability	191

5.3.16.2	Conformance requirement	191

5.3.16.3	Test purpose	191

5.3.16.4	Method of test	192

5.3.16.4.1	Initial conditions	192

5.3.16.4.2	Procedure	193

5.3.16.5	Acceptance criteria	193

5.3.17	SUCI calculation by ME using Profile B with compressed Home Network Public Key	193

5.3.17.1	Definition and applicability	193

5.3.17.2	Conformance requirement	193

5.3.17.3	Test purpose	194

5.3.17.4	Method of test	194

5.3.17.4.1	Initial conditions	194

5.3.17.4.2	Procedure	195

5.3.17.5	Acceptance criteria	195

5.4	Unified Access Control information handling for 5G-NR	196

5.4.1	Unified Access Control – Access identity 0, no access identities indicated by USIM	196

5.4.1.1	Definition and applicability	196

5.4.1.2	Conformance requirement	196

5.4.1.3	Test purpose	196

5.4.1.4	Method of test	197

5.4.1.4.1	Initial conditions	197

5.4.1.4.2	Procedure	198

5.4.1.4.3	Acceptance criteria	198

5.4.1A	Unified Access Control – Access identity 0, no access identities indicated by USIM, Access Category 8	198

5.4.1A.1	Definition and applicability	198

5.4.1A.2	Conformance requirement	199

5.4.1A.3	Test purpose	199

5.4.1A.4	Method of test	199

5.4.1A.4.1	Initial conditions	199

5.4.1.4.2	Procedure	201

5.4.1A.4.3	Acceptance criteria	201

5.4.2	Unified Access Control – Access Identity 1 – MPS indicated by USIM	202

5.4.2.1	Definition and applicability	202

5.4.2.2	Conformance requirement	202

5.4.2.3	Test purpose	202

5.4.2.4	Method of test	203

5.4.2.4.1	Initial conditions	203

5.4.2.4.2	Procedure	203

5.4.2.4.3	Acceptance criteria	204

5.4.2A	Unified Access Control – Access Identity 1 – MPS indicated by USIM, Access Category 8	205

5.4.2A.1	Definition and applicability	205

5.4.2A.2	Conformance requirement	205

5.4.2A.3	Test purpose	206

5.4.2A.4	Method of test	206

5.4.2A.4.1	Initial conditions	206

5.4.2A.4.2	Procedure	207

5.4.2A.4.3	Acceptance criteria	207

5.4.3	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI not changed	208

5.4.3.1	Definition and applicability	208

5.4.3.2	Conformance requirement	208

5.4.3.3	Test purpose	209

5.4.3.4	Method of test	209

5.4.3.4.1	Initial conditions	209

5.4.3.4.2	Procedure	211

5.4.3.4.3	Acceptance criteria	211

5.4.4	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI is changed	212

5.4.4.1	Definition and applicability	212

5.4.4.2	Conformance requirement	212

5.4.4.3	Test purpose	213

5.4.4.4	Method of test	213

5.4.4.4.1	Initial conditions	213

5.4.4.4.2	Procedure	214

5.4.4.4.3	Acceptance criteria	215

5.4.5	Unified Access Control – Access Identity 2 – MCS indicated by USIM	215

5.4.5.1	Definition and applicability	215

5.4.5.2	Conformance requirement	216

5.4.5.3	Test purpose	216

5.4.5.4	Method of test	216

5.4.5.4.1	Initial conditions	216

5.4.5.4.2	Procedure	217

5.4.5.4.3	Acceptance criteria	217

5.4.5A	Unified Access Control – Access Identity 2 – MCS indicated by USIM	218

5.4.5A.1	Definition and applicability	218

5.4.5A.2	Conformance requirement	219

5.4.5A.3	Test purpose	219

5.4.5A.4	Method of test	220

5.4.5A.4.1	Initial conditions	220

5.4.5A.4.2	Procedure	221

5.4.5A.4.3	Acceptance criteria	221

5.4.6	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is not changed	222

5.4.6.1	Definition and applicability	222

5.4.6.2	Conformance requirement	222

5.4.6.3	Test purpose	223

5.4.6.4	Method of test	223

5.4.6.4.1	Initial conditions	223

5.4.6.4.2	Procedure	224

5.4.6.4.3	Acceptance criteria	225

5.4.7	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is changed	225

5.4.7.1	Definition and applicability	225

5.4.7.2	Conformance requirement	226

5.4.7.3	Test purpose	227

5.4.7.4	Method of test	227

5.4.7.4.1	Initial conditions	227

5.4.7.4.2	Procedure	228

5.4.7.4.3	Acceptance criteria	229

5.4.8	Unified Access Control – Access Identities 11 and 15 indicated by USIM	229

5.4.8.1	Definition and applicability	229

5.4.8.2	Conformance requirement	230

5.4.8.3	Test purpose	230

5.4.8.4	Method of test	230

5.4.8.4.1	Initial conditions	230

5.4.8.4.2	Procedure	231

5.4.8.4.3	Acceptance criteria	231

5.4.8A	Unified Access Control – Access Identities 11 and 15 indicated by USIM, Access Category 8	232

5.4.8A.1	Definition and applicability	232

5.4.8A.2	Conformance requirement	233

5.4.8A.3	Test purpose	233

5.4.8A.4	Method of test	233

5.4.8A.4.1	Initial conditions	233

5.4.8A.4.2	Procedure	234

5.4.8.4.3	Acceptance criteria	235

5.4.9	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM	236

5.4.9.1	Definition and applicability	236

5.4.9.2	Conformance requirement	236

5.4.9.3	Test purpose	236

5.4.9.4	Method of test	237

5.4.9.4.1	Initial conditions	237

5.4.9.4.2	Procedure	237

5.4.9.4.3	Acceptance criteria	238

5.4.9A	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM, Access Category 8	239

5.4.9A.1	Definition and applicability	239

5.4.9A.2	Conformance requirement	239

5.4.9A.3	Test purpose	240

5.4.9A.4	Method of test	240

5.4.9A.4.1	Initial conditions	240

5.4.9A.4.2	Procedure	241

5.4.9A.4.3	Acceptance criteria	242

5.4.10	Unified Access Control – Operator-Defined Access Category	242

5.4.10.1	Definition and applicability	242

5.4.10.2	Conformance requirement	243

5.4.10.3	Test purpose	244

5.4.10.4	Method of test	244

5.4.10.4.1	Initial conditions	244

5.4.10.4.2	Procedure	246

5.4.10.5	Acceptance criteria	246

5.4.11	Unified Access Control – Operator-Defined Access Categories, no change in SUPI	247

5.4.11.1	Definition and applicability	247

5.4.11.2	Conformance requirement	248

5.4.11.3	Test purpose	249

5.4.11.4	Method of test	249

5.4.11.4.1	Initial conditions	249

5.4.11.4.2	Procedure	251

5.4.11.4.3	Acceptance criteria	252

5.4.12	Unified Access Control – Operator-Defined Access Categories, SUPI change	252

5.4.12.1	Definition and applicability	252

5.4.12.2	Conformance requirement	253

5.4.12.3	Test purpose	254

5.4.12.4	Method of test	254

5.4.12.4.1	Initial conditions	254

5.4.12.4.2	Procedure	256

5.4.12.4.3	Acceptance criteria	257

5.5	Handling of operator controlled features	258

5.5.1	Display of registered 5G PLMN name from USIM	258

5.5.1.1	Definition and applicability	258

5.5.1.2	Conformance requirement	258

5.5.1.3	Test purpose	258

5.5.1.4	Method of tests	258

5.5.1.4.1	Initial conditions	258

5.5.1.4.2	Procedure	260

5.5.1.4.3	Acceptance criteria	261

5.5.2	Display of registered 5G PLMN name from ME	261

5.5.2.1	Definition and applicability	261

5.5.2.2	Conformance requirement	261

5.5.2.3	Test purpose	261

5.5.2.4	Method of tests	261

5.5.2.4.1	Initial conditions	261

5.5.2.4.2	Procedure	263

5.5.2.4.3	Acceptance criteria	264

5.6	Handling subscription identifier privacy for 5G - SUPI type in NAI format	264

5.6.1	SUCI calculation by ME using null scheme	264

5.6.1.1	Definition and applicability	264

5.6.1.2	Conformance requirement	264

5.6.1.3	Test purpose	265

5.6.1.4	Method of test	265

5.6.1.4.1	Initial conditions	265

5.6.1.4.2	Procedure	266

5.6.1.5	Acceptance criteria	266

5.6.2	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A	267

5.6.2.1	Definition and applicability	267

5.6.2.2	Conformance requirement	267

5.6.2.3	Test purpose	267

5.6.2.4	Method of test	268

5.6.2.4.1	Initial conditions	268

5.6.2.4.2	Procedure	269

5.6.2.5	Acceptance criteria	269

5.6.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	269

5.6.3.1	Definition and applicability	269

5.6.3.2	Conformance requirement	270

5.6.3.3	Test purpose	270

5.6.3.4	Method of test	270

5.6.3.4.1	Initial conditions	270

5.6.3.4.2	Procedure	272

5.6.3.5	Acceptance criteria	272

5.6.4	UE identification after SUPI is changed	272

5.6.4.1	Definition and applicability	272

5.6.4.2	Conformance requirement	273

5.6.4.3	Test purpose	273

5.6.4.4	Method of test	273

5.6.4.4.1	Initial conditions	273

5.6.4.4.2	Procedure	274

5.6.4.5	Acceptance criteria	274

5.6.5	UE identification by SUCI during initial registration – SUCI calculation by ME using profile A	274

5.6.5.1	Definition and applicability	274

5.6.5.2	Conformance requirement	275

5.6.5.3	Test purpose	275

5.6.5.4	Method of test	275

5.6.5.4.1	Initial conditions	275

5.6.5.4.2	Procedure	276

5.6.5.5	Acceptance criteria	277

5.6.6	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	277

5.6.6.1	Definition and applicability	277

5.6.6.2	Conformance requirement	277

5.6.6.3	Test purpose	278

5.6.6.4	Method of test	278

5.6.6.4.1	Initial conditions	278

5.6.6.4.2	Procedure	279

5.6.6.5	Acceptance criteria	279

6	Security related Tests	280

6.1	PIN handling	280

6.1.1	Entry of PIN	280

6.1.1.1	Definition and applicability	280

6.1.1.2	Conformance requirement	280

6.1.1.3	Test purpose	280

6.1.1.4	Method of test	280

6.1.1.4.1	Initial conditions	280

6.1.1.4.2	Procedure	280

6.1.1.5	Acceptance criteria	280

6.1.2	Change of PIN	281

6.1.2.1	Definition and applicability	281

6.1.2.2	Conformance requirement	281

6.1.2.3	Test purpose	281

6.1.2.4	Method of test	281

6.1.2.4.1	Initial conditions	281

6.1.2.4.2	Procedure	281

6.1.2.5	Acceptance criteria	281

6.1.3	Unblock PIN	282

6.1.3.1	Definition and applicability	282

6.1.3.2	Conformance requirement	282

6.1.3.3	Test purpose	282

6.1.3.4	Method of test	282

6.1.3.4.1	Initial conditions	282

6.1.3.4.2	Procedure	282

6.1.3.5	Acceptance criteria	283

6.1.4	Entry of PIN2	283

6.1.4.1	Definition and applicability	283

6.1.4.2	Conformance requirement	283

6.1.4.3	Test purpose	283

6.1.4.4	Method of test	283

6.1.4.4.1	Initial conditions	283

6.1.4.4.2	Procedure	284

6.1.4.5	Acceptance criteria	284

6.1.5	Change of PIN2	284

6.1.5.1	Definition and applicability	284

6.1.5.2	Conformance requirement	284

6.1.5.3	Test purpose	284

6.1.5.4	Method of test	284

6.1.5.4.1	Initial conditions	284

6.1.5.4.2	Procedure	284

6.1.5.5	Acceptance criteria	285

6.1.6	Unblock PIN2	285

6.1.6.1	Definition and applicability	285

6.1.6.2	Conformance requirement	285

6.1.6.3	Test purpose	285

6.1.6.4	Method of test	285

6.1.6.4.1	Initial conditions	285

6.1.6.4.2	Procedure	285

6.1.6.5	Acceptance criterias	286

6.1.7	Replacement of PIN	287

6.1.7.1	Definition and applicability	287

6.1.7.2	Conformance requirement	287

6.1.7.3	Test purpose	287

6.1.7.4	Method of test	287

6.1.7.4.1	Initial conditions	287

6.1.7.4.2	Procedure	287

6.1.7.5	Acceptance criteria	287

6.1.8	Change of Universal PIN	288

6.1.8.1	Definition and applicability	288

6.1.8.2	Conformance requirement	288

6.1.8.3	Test purpose	288

6.1.8.4	Method of test	288

6.1.8.4.1	Initial conditions	288

6.1.8.4.2	Procedure	288

6.1.8.5	Acceptance criteria	289

6.1.9	Unblock Universal PIN	289

6.1.9.1	Definition and applicability	289

6.1.9.2	Conformance requirement	289

6.1.9.3	Test purpose	289

6.1.9.4	Method of test	289

6.1.9.4.1	Initial conditions	289

6.1.9.4.2	Procedure	289

6.1.9.5	Acceptance criteria	290

6.1.10	Entry of PIN on multi-verification capable UICCs	290

6.1.10.1	Definition and applicability	290

6.1.10.2	Conformance requirement	290

6.1.10.4	Method of test	290

6.1.10.4.1	Initial conditions	290

6.1.10.4.2	Procedure	291

6.1.10.5	Acceptance criteria	291

6.1.11	Change of PIN on multi-verification capable UICCs	291

6.1.11.1	Definition and applicability	291

6.1.11.2	Conformance requirement	291

6.1.11.3	Test purpose	292

6.1.11.4	Method of test	292

6.1.11.4.1	Initial conditions	292

6.1.11.4.2	Procedure	292

6.1.11.5	Acceptance criteria	293

6.1.12	Unblock PIN on multi-verification capable UICCs	293

6.1.12.1	Definition and applicability	293

6.1.12.2	Conformance requirement	293

6.1.12.3	Test purpose	293

6.1.12.4	Method of test	293

6.1.12.4.1	Initial conditions	293

6.1.12.4.2	Procedure	294

6.1.12.5	Acceptance criteria	295

6.1.13	Entry of PIN2 on multi-verification capable UICCs	295

6.1.13.1	Definition and applicability	295

6.1.13.2	Conformance requirement	295

6.1.13.3	Test purpose	295

6.1.13.4	Method of test	295

6.1.13.4.1	Initial conditions	295

6.1.13.4.2	Procedure	296

6.1.13.5	Acceptance criteria	296

6.1.14	Change of PIN2 on multi-verification capable UICCs	297

6.1.14.1	Definition and applicability	297

6.1.14.2	Conformance requirement	297

6.1.14.3	Test purpose	297

6.1.14.4	Method of test	297

6.1.14.4.1	Initial conditions	297

6.1.14.4.2	Procedure	298

6.1.14.5	Acceptance criteria	298

6.1.15	Unblock PIN2 on multi-verification capable UICCs	298

6.1.15.1	Definition and applicability	298

6.1.15.2	Conformance requirement	298

6.1.15.3	Test purpose	299

6.1.15.4	Method of test	299

6.1.15.4.1	Initial conditions	299

6.1.15.4.2	Procedure	300

6.1.15.5	Acceptance criterias	300

6.1.16	Replacement of PIN with key reference "07"	301

6.1.16.1	Definition and applicability	301

6.1.16.2	Conformance requirement	301

6.1.16.3	Test purpose	301

6.1.16.4	Method of test	301

6.1.16.4.1	Initial conditions	301

6.1.16.4.2	Procedure	302

6.1.16.5	Acceptance criteria	302

6.2	Fixed Dialling Numbers (FDN) handling	303

6.2.1	Terminal and USIM with FDN enabled, EFADN readable and updateable	303

6.2.1.1	Definition and applicability	303

6.2.1.2	Conformance requirement	303

6.2.1.3	Test purpose	303

6.2.1.4	Method of test	304

6.2.1.4.1	Initial conditions	304

6.2.1.4.2	Procedure	304

6.2.1.5	Acceptance criteria	304

6.2.2	Terminal and USIM with FDN disabled	305

6.2.2.1	Definition and applicability	305

6.2.2.2	Conformance requirement	305

6.2.2.3	Test purpose	305

6.2.2.4	Method of test	305

6.2.2.4.1	Initial conditions	305

6.2.2.4.2	Procedure	306

6.2.2.5	Acceptance criteria	306

6.2.3	Enabling, disabling and updating of FDN	306

6.2.3.1	Definition and applicability	306

6.2.3.2	Conformance requirement	306

6.2.3.3	Test purpose	306

6.2.3.4	Method of test	307

6.2.3.4.1	Initial conditions	307

6.2.3.4.2	Procedure	307

6.2.3.5	Acceptance criteria	307

6.2.4	Terminal and USIM with FDN enabled, EFADN readable and updateable (Rel-4 and onwards)	307

6.2.4.1	Definition and applicability	307

6.2.4.2	Conformance requirement	307

6.2.4.3	Test purpose	308

6.2.4.4	Method of test	308

6.2.4.4.1	Initial conditions	308

6.2.4.4.2	Procedure	308

6.2.4.5	Acceptance criteria	309

6.3	Void	309

6.4	Advice of charge (AoC) handling	309

6.4.1	AoC not supported by USIM	309

6.4.1.1	Definition and applicability	309

6.4.1.2	Conformance requirement	309

6.4.1.3	Test purpose	310

6.4.1.4	Method of test	310

6.4.1.4.1	Initial conditions	310

6.4.1.4.2	Procedure	310

6.4.1.5	Acceptance criteria	311

6.4.2	Maximum frequency of ACM updating	311

6.4.2.1	Definition and applicability	311

6.4.2.2	Conformance requirement	311

6.4.2.3	Test purpose	311

6.4.2.4	Method of test	311

6.4.2.4.1	Initial conditions	311

6.4.2.4.2	Procedure	312

6.4.2.5	Acceptance criteria	314

6.4.3	Call terminated when ACM greater than ACMmax	314

6.4.3.1	Definition and applicability	314

6.4.3.2	Conformance requirement	314

6.4.3.3	Test purpose	314

6.4.3.4	Method of test	315

6.4.3.4.1	Initial conditions	315

6.4.3.4.2	Procedure	315

6.4.3.5	Acceptance criteria	317

6.4.4	Response codes of increase command of ACM	317

6.4.4.1	Definition and applicability	317

6.4.4.2	Conformance requirement	318

6.4.4.3	Test purpose	318

6.4.4.4	Method of test	318

6.4.4.4.1	Initial conditions	318

6.4.4.4.2	Procedure	319

6.4.4.5	Acceptance criteria	321

7	PLMN related tests	322

7.1	FPLMN handling	322

7.1.1	Adding FPLMN to the Forbidden PLMN list	322

7.1.1.1	Definition and applicability	322

7.1.1.2	Conformance requirement	322

7.1.1.3	Test purpose	324

7.1.1.4	Method of test	324

7.1.1.4.1	Initial conditions	324

7.1.1.4.2	Procedure	325

7.1.1.5	Acceptance criteria	328

7.1.2	UE updating forbidden PLMNs	330

7.1.2.1	Definition and applicability	330

7.1.2.2	Conformance requirement	331

7.1.2.3	Test purpose	331

7.1.2.4	Method of test	331

7.1.2.4.1	Initial conditions	331

7.1.2.4.2	Procedure	332

7.1.2.5	Acceptance criteria	333

7.1.3	UE deleting forbidden PLMNs	333

7.1.3.1	Definition and applicability	333

7.1.3.2	Conformance requirement	334

7.1.3.3	Test purpose	334

7.1.3.4	Method of test	335

7.1.3.4.1	Initial conditions	335

7.1.3.4.2	Procedure	335

7.1.3.5	Acceptance criteria	336

7.1.4	Adding FPLMN to the forbidden PLMN list when accessing E-UTRAN	337

7.1.4.1	Definition and applicability	337

7.1.4.2	Conformance requirement	338

7.1.4.3	Test purpose	338

7.1.4.4	Method of test	338

7.1.4.4.1	Initial conditions	338

7.1.4.4.2	Procedure	339

7.1.4.5	Acceptance criteria	340

7.1.5	UE updating forbidden PLMNs when accessing E-UTRAN	340

7.1.5.1	Definition and applicability	340

7.1.5.2	Conformance requirement	340

7.1.5.3	Test purpose	341

7.1.5.4	Method of test	341

7.1.5.4.1	Initial conditions	341

7.1.5.4.2	Procedure	341

7.1.5.5	Acceptance criteria	341

7.1.6	UE deleting forbidden PLMNs when accessing E-UTRAN	342

7.1.6.1	Definition and applicability	342

7.1.6.2	Conformance requirement	342

7.1.6.3	Test purpose	342

7.1.6.4	Method of test	343

7.1.6.4.1	Initial conditions	343

7.1.6.4.2	Procedure	343

7.1.6.5	Acceptance criteria	343

7.1.7	Updating the Forbidden PLMN list  after receiving non-integrity protected reject message – UTRAN	344

7.1.7.1	Definition and applicability	344

7.1.7.2	Conformance requirement	344

7.1.7.3	Test purpose	345

7.1.7.4	Method of test	345

7.1.7.4.1	Initial conditions	345

7.1.7.4.2	Procedure	345

7.1.7.5	Acceptance criteria	346

7.1.8	Updating the Forbidden PLMN list after receiving non-integrity protected reject message – E-UTRAN	346

7.1.8.1	Definition and applicability	346

7.1.8.2	Conformance requirement	346

7.1.8.3	Test purpose	347

7.1.8.4	Method of test	347

7.1.8.4.1	Initial conditions	347

7.1.8.4.2	Procedure	347

7.1.8.5	Acceptance criteria	348

7.1.9	Adding FPLMN to the forbidden PLMN list when accessing satellite NG-RAN	348

7.1.9.1	Definition and applicability	348

7.1.9.2	Conformance requirement	349

7.1.9.3	Test purpose	349

7.1.9.4	Method of test	349

7.1.9.4.1	Initial conditions	349

7.1.9.4.2	Procedure	350

7.1.9.5	Acceptance criteria	350

7.1.10	UE updating forbidden PLMNs when accessing satellite NG-RAN	351

7.1.10.1	Definition and applicability	351

7.1.10.2	Conformance requirement	351

7.1.10.3	Test purpose	352

7.1.10.4	Method of test	352

7.1.10.4.1	Initial conditions	352

7.1.10.4.2	Procedure	352

7.1.10.5	Acceptance criteria	352

7.1.11	UE deleting forbidden PLMNs when accessing satellite NG-RAN in manual mode	353

7.1.11.1	Definition and applicability	353

7.1.11.2	Conformance requirement	353

7.1.11.3	Test purpose	353

7.1.11.4	Method of test	354

7.1.11.4.1	Initial conditions	354

7.1.11.4.2	Procedure	354

7.1.11.5	Acceptance criteria	354

7.1.12	Adding FPLMN to the forbidden PLMN list when accessing Satellite E-UTRAN in NB-S1 mode	355

7.1.12.1	Definition and applicability	355

7.1.12.2	Conformance requirement	355

7.1.12.3	Test purpose	356

7.1.12.4	Method of test	356

7.1.12.4.1	Initial conditions	356

7.1.12.4.2	Procedure	356

7.1.12.5	Acceptance criteria	357

7.1.13	UE updating forbidden PLMNs when accessing Satellite E-UTRAN in NB-S1 mode	358

7.1.13.1	Definition and applicability	358

7.1.13.2	Conformance requirement	358

7.1.13.3	Test purpose	358

7.1.13.4	Method of test	358

7.1.13.4.1	Initial conditions	358

7.1.13.4.2	Procedure	359

7.1.13.5	Acceptance criteria	359

7.1.14	UE deleting forbidden PLMNs when accessing Satellite E-UTRAN in NB-S1 mode in manual mode	360

7.1.14.1	Definition and applicability	360

7.1.14.2	Conformance requirement	360

7.1.14.3	Test purpose	360

7.1.14.4	Method of test	360

7.1.14.4.1	Initial conditions	360

7.1.14.4.2	Procedure	361

7.1.14.5	Acceptance criteria	361

7.2	User controlled PLMN selector handling	362

7.2.1	UE updating the User controlled PLMN selector list	362

7.2.1.1	Definition and applicability	362

7.2.1.2	Conformance requirement	362

7.2.1.3	Test purpose	362

7.2.1.4	Method of test	362

7.2.1.4.1	Initial conditions	362

7.2.1.4.2	Procedure	362

7.2.1.5	Acceptance criteria	362

7.2.2	UE recognising the priority order of the User controlled PLMN selector list with the same access technology.	363

7.2.2.1	Definition and applicability	363

7.2.2.2	Conformance requirement	363

7.2.2.3	Test purpose	364

7.2.2.4	Method of test	364

7.2.2.4.1	Initial conditions	364

7.2.2.4.2	Procedure	365

7.2.2.5	Acceptance criteria	366

7.2.3	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference.	367

7.2.3.1	Definition and applicability	367

7.2.3.2	Conformance requirement	367

7.2.3.3	Test purpose	367

7.2.3.4	Method of test	367

7.2.3.4.1	Initial conditions	367

7.2.3.5	Acceptance criteria	368

7.2.4	Void	369

7.2.5	UE updating the User controlled PLMN selector list for E-UTRAN	369

7.2.5.1	Definition and applicability	369

7.2.5.2	Conformance requirement	369

7.2.5.3	Test purpose	369

7.2.5.4	Method of test	369

7.2.5.4.1	Initial conditions	369

7.2.5.4.2	Procedure	369

7.2.5.5	Acceptance criteria	369

7.2.6	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference- UTRAN/E-UTRAN	370

7.2.6.1	Definition and applicability	370

7.2.6.2	Conformance requirement	370

7.2.6.3	Test purpose	371

7.2.6.4	Method of test	371

7.2.6.4.1	Initial conditions	371

7.2.6.4.2	Procedure	371

7.2.6.5	Acceptance criteria	371

7.2.7	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference- GSM/E-UTRAN	372

7.2.7.1	Definition and applicability	372

7.2.7.2	Conformance requirement	372

7.2.7.3	Test purpose	372

7.2.7.4	Method of test	372

7.2.7.4.1	Initial conditions	372

7.2.7.4.2	Procedure	373

7.2.7.5	Acceptance criteria	373

7.2.8	UE recognising the priority order of the User controlled PLMN selector list with the same access technology – E-UTRAN in NB-S1 mode	374

7.2.8.1	Definition and applicability	374

7.2.8.2	Conformance requirement	374

7.2.8.3	Test purpose	374

7.2.8.4	Method of test	374

7.2.8.4.1	Initial conditions	374

7.2.8.2	Procedure	375

7.2.8.5	Acceptance criteria	375

7.2.9	UE recognising the priority order of the User controlled PLMN selector list using the ACT preference – E-UTRAN in WB-S1/E-UTRAN in NB-S1	376

7.2.9.1	Definition and applicability	376

7.2.9.2	Conformance requirement	376

7.2.9.3	Test purpose	376

7.2.9.4	Method of test	376

7.2.9.4.1	Initial conditions	376

7.2.9.4.2	Procedure	377

7.2.9.5	Acceptance criteria	377

7.2.10	UE updating the User controlled PLMN selector list for satellite-NG-RAN	378

7.2.10.1	Definition and applicability	378

7.2.10.2	Conformance requirement	378

7.2.10.3	Test purpose	378

7.2.10.4	Method of test	378

7.2.10.4.1	Initial conditions	378

7.2.10.4.2	Procedure	378

7.2.10.5	Acceptance criteria	378

7.2.11	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference GSM/satellite NG-RAN	379

7.2.11.1	Definition and applicability	379

7.2.11.2	Conformance requirement	379

7.2.11.3	Test purpose	379

7.2.11.4	Method of test	380

7.2.11.4.1	Initial conditions	380

7.2.11.4.2	Procedure	380

7.2.11.5	Acceptance criteria	380

7.2.12	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference UTRAN/satellite NG-RAN	381

7.2.12.1	Definition and applicability	381

7.2.12.2	Conformance requirement	381

7.2.12.3	Test purpose	381

7.2.12.4	Method of test	381

7.2.12.4.1	Initial conditions	381

7.2.12.4.2	Procedure	382

7.2.12.5	Acceptance criteria	382

7.2.13	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference E-UTRAN/satellite NG-RAN	383

7.2.13.1	Definition and applicability	383

7.2.13.2	Conformance requirement	383

7.2.13.3	Test purpose	383

7.2.13.4	Method of test	383

7.2.13.4.1	Initial conditions	383

7.2.13.4.2	Procedure	383

7.2.13.5	Acceptance criteria	384

7.2.14	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference NG-RAN/satellite NG-RAN	384

7.2.14.1	Definition and applicability	384

7.2.14.2	Conformance requirement	384

7.2.14.3	Test purpose	385

7.2.14.4	Method of test	385

7.2.14.4.1	Initial conditions	385

7.2.14.4.2	Procedure	385

7.2.14.5	Acceptance criteria	385

7.2.15	UE updating the User controlled PLMN selector list for Satellite-E-UTRAN in NB-S1 mode	386

7.2.15.1	Definition and applicability	386

7.2.15.2	Conformance requirement	386

7.2.15.3	Test purpose	386

7.2.15.4	Method of test	386

7.2.15.4.1	Initial conditions	386

7.2.15.4.2	Procedure	387

7.2.15.5	Acceptance criteria	387

7.2.16	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference GSM/Satellite E-UTRAN in NB-S1 mode	388

7.2.16.1	Definition and applicability	388

7.2.16.2	Conformance requirement	388

7.2.16.3	Test purpose	388

7.2.16.4	Method of test	388

7.2.16.4.1	Initial conditions	388

7.2.16.4.2	Procedure	389

7.2.16.5	Acceptance criteria	390

7.2.17	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference UTRAN/Satellite E-UTRAN in NB-S1 mode	390

7.2.17.1	Definition and applicability	390

7.2.17.2	Conformance requirement	390

7.2.17.3	Test purpose	390

7.2.17.4	Method of test	390

7.2.17.4.1	Initial conditions	390

7.2.17.4.2	Procedure	391

7.2.17.5	Acceptance criteria	392

7.2.18	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference E-UTRAN/Satellite E-UTRAN in NB-S1 mode	392

7.2.18.1	Definition and applicability	392

7.2.18.2	Conformance requirement	392

7.2.18.3	Test purpose	392

7.2.18.4	Method of test	393

7.2.18.4.1	Initial conditions	393

7.2.18.4.2	Procedure	394

7.2.18.5	Acceptance criteria	394

7.2.19	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference E-UTRAN in NB-S1 mode/Satellite E-UTRAN in NB-S1 mode	394

7.2.19.1	Definition and applicability	394

7.2.19.2	Conformance requirement	394

7.2.19.3	Test purpose	395

7.2.19.4	Method of test	395

7.2.19.4.1	Initial conditions	395

7.2.19.4.2	Procedure	396

7.2.19.5	Acceptance criteria	396

7.2.20	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference NG-RAN/Satellite E-UTRAN in NB-S1 mode	397

7.2.20.1	Definition and applicability	397

7.2.20.2	Conformance requirement	397

7.2.20.3	Test purpose	397

7.2.20.4	Method of test	397

7.2.20.4.1	Initial conditions	397

7.2.20.4.2	Procedure	398

7.2.20.5	Acceptance criteria	398

7.3	Operator controlled PLMN selector handling	399

7.3.1	UE recognising the priority order of the Operator controlled PLMN selector list.	399

7.3.1.1	Definition and applicability	399

7.3.1.2	Conformance requirement	399

7.3.1.3	Test purpose	399

7.3.1.4	Method of test	399

7.3.1.4.1	Initial conditions	399

7.3.1.4.2	Procedure	401

7.3.1.5	Acceptance criteria	402

7.3.2	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list.	403

7.3.2.1	Definition and applicability	403

7.3.2.2	Conformance requirement	403

7.3.2.3	Test purpose	403

7.3.2.4	Method of test	403

7.3.2.4.1	Initial conditions	403

7.3.2.4.2	Procedure	405

7.3.2.5	Acceptance criteria	406

7.3.3	UE recognising the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN	407

7.3.3.1	Definition and applicability	407

7.3.3.2	Conformance requirement	407

7.3.3.3	Test purpose	407

7.3.3.4	Method of test	407

7.3.3.4.1	Initial conditions	407

7.3.3.4.2	Procedure	408

7.3.3.5	Acceptance criteria	409

7.3.4	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN	409

7.3.4.1	Definition and applicability	409

7.3.4.2	Conformance requirement	409

7.3.4.3	Test purpose	410

7.3.4.4	Method of test	410

7.3.4.4.1	Initial conditions	410

7.3.4.4.2	Procedure	410

7.3.4.5	Acceptance criteria	411

7.3.10	UE recognising the priority order of the Operator controlled PLMN selector list when accessing NG-RAN	423

7.3.10.1	Definition and applicability	423

7.3.10.2	Conformance requirement	423

7.3.10.3	Test purpose	423

7.3.10.4	Method of test	423

7.3.10.4.1	Initial conditions	423

7.3.10.4.2	Procedure	424

7.3.10.5	Acceptance criteria	424

7.3.11	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – NG-RAN	425

7.3.11.1	Definition and applicability	425

7.3.11.2	Conformance requirement	425

7.3.11.3	Test purpose	425

7.3.11.4	Method of test	425

7.3.11.4.1	Initial conditions	425

7.3.11.4.2	Procedure	426

7.3.11.5	Acceptance criteria	426

7.3.12	UE recognising the priority order of the Operator controlled PLMN selector list when accessing satellite NG-RAN	427

7.3.12.1	Definition and applicability	427

7.3.12.2	Conformance requirement	427

7.3.12.3	Test purpose	427

7.3.12.4	Method of test	427

7.3.12.4.1	Initial conditions	427

7.3.12.4.2	Procedure	428

7.3.12.5	Acceptance criteria	428

7.3.13	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – satellite NG-RAN	429

7.3.13.1	Definition and applicability	429

7.3.13.2	Conformance requirement	429

7.3.13.3	Test purpose	429

7.3.13.4	Method of test	429

7.3.13.4.1	Initial conditions	429

7.3.13.4.2	Procedure	429

7.3.13.5	Acceptance criteria	430

7.3.14	UE recognising the priority order of the Operator controlled PLMN selector list when accessing Satellite E-UTRAN in NB-S1 mode	430

7.3.14.1	Definition and applicability	430

7.3.14.2	Conformance requirement	431

7.3.14.3	Test purpose	431

7.3.14.4	Method of test	431

7.3.14.4.1	Initial conditions	431

7.3.14.4.2	Procedure	432

7.3.14.5	Acceptance criteria	432

7.3.15	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – Satellite E-UTRAN in NB-S1 mode	433

7.3.15.1	Definition and applicability	433

7.3.15.2	Conformance requirement	433

7.3.15.3	Test purpose	433

7.3.15.4	Method of test	433

7.3.15.4.1	Initial conditions	433

7.3.15.4.2	Procedure	435

7.4	Higher priority PLMN search handling	436

7.4.1	UE recognising the search period of the Higher priority PLMN	436

7.4.1.1	Definition and applicability	436

7.4.1.2	Conformance requirement	436

7.4.1.3	Test purpose	436

7.4.1.4	Method of test	436

7.4.1.4.1	Initial conditions	436

7.4.1.4.2	Procedure	437

7.4.1.5	Acceptance criteria	439

7.4.2	GSM/UmTS dual mode UEs recognising the search period of the Higher priority PLMN	440

7.4.2.1	Definition and applicability	440

7.4.2.2	Conformance requirement	440

7.4.2.3	Test purpose	441

7.4.2.4	Method of test	441

7.4.2.4.1	Initial conditions	441

7.4.2.4.2	Procedure	442

7.4.2.5	Acceptance criteria	443

7.4.3	UE recognising the search period of the Higher priority PLMN – E-UTRAN	444

7.4.3.1	Definition and applicability	444

7.4.3.2	Conformance requirement	444

7.4.3.3	Test purpose	444

7.4.3.4	Method of test	444

7.4.3.4.1	Initial conditions	444

7.4.3.4.2	Procedure	445

7.4.3.5	Acceptance criteria	446

7.4.4	E-UTRAn/EPC capable UEs recognising the search period of the Higher priority PLMN – GSM/E-UTRAN	446

7.4.4.1	Definition and applicability	446

7.4.4.2	Conformance requirement	446

7.4.4.3	Test purpose	446

7.4.4.4	Method of test	446

7.4.4.4.1	Initial conditions	446

7.4.4.4.2	Procedure	448

7.4.4.5	Acceptance criteria	448

7.4.5	E-UTRAn/EPC capable UEs recognising the search period of the Higher priority PLMN – UTRAN/E-UTRAN	449

7.4.5.1	Definition and applicability	449

7.4.5.2	Conformance requirement	449

7.4.5.3	Test purpose	449

7.4.5.4	Method of test	449

7.4.5.4.1	Initial conditions	449

7.4.5.4.2	Procedure	450

7.4.5.5	Acceptance criteria	451

7.4.6	UE recognising Multiplier Coefficient for Higher Priority PLMN search - satellite-NG-RAN	451

7.4.6.1	Definition and applicability	451

7.4.6.2	Conformance requirement	451

7.4.6.3	Test purpose	452

7.4.6.4	Method of test	452

7.4.6.4.1	Initial conditions	452

7.4.6.4.2	Procedure	453

7.4.6.5	Acceptance criteria	453

7.4.7	UE recognising the search period of the Higher priority PLMN – NG-RAN	454

7.4.7.1	Definition and applicability	454

7.4.7.2	Conformance requirement	454

7.4.7.3	Test purpose	454

7.4.7.4	Method of test	454

7.4.7.4.1	Initial conditions	454

7.4.7.4.2	Procedure	456

7.4.7.5	Acceptance criteria	456

7.5	Void	457

8	Subscription independent tests	457

8.1	Phone book procedures	457

8.1.1	Recognition of a previously changed phonebook	457

8.1.1.1	Definition and applicability	457

8.1.1.2	Conformance requirement	457

8.1.1.3	Test purpose	457

8.1.1.4	Method of test	457

8.1.1.4.1	Initial conditions	457

8.1.1.4.2	Procedure	458

8.1.1.5	Acceptance criteria	458

8.1.2	Update of the Phonebook Synchronisation Counter (PSC)	459

8.1.2.1	Definition and applicability	459

8.1.2.2	Conformance requirement	459

8.1.2.3	Test purpose	459

8.1.2.4	Method of test	459

8.1.2.4.1	Initial conditions	459

8.1.2.4.2	Procedure	460

8.1.2.5	Acceptance criteria	460

8.1.3	Phonebook content handling	461

8.1.3.1	Handling of BCD number/ SSC content extension	461

8.1.3.1.1	Definition and applicability	461

8.1.3.1.2	Conformance requirement	461

8.1.3.1.3	Test purpose	461

8.1.3.1.4	Method of test	461

8.1.3.1.5	Acceptance criteria	464

8.1.4	Phonebook selection	465

8.1.4.1	Definition and applicability	465

8.1.4.2	Conformance requirement	465

8.1.4.3	Test purpose	465

8.1.4.4	Method of test	465

8.1.4.4.1	Initial conditions	465

8.1.4.4.2	Procedure	468

8.1.4.5	Acceptance criteria	468

8.1.5	Local Phonebook handling	469

8.1.5.1	Definition and applicability	469

8.1.5.2	Conformance requirement	469

8.1.5.3	Test purpose	469

8.1.5.4	Method of test	469

8.1.5.4.1	Initial conditions	469

8.1.5.4.2	Procedure	470

8.1.5.5	Acceptance criteria	470

8.2	Short message handling report	471

8.2.1	Correct storage of a SM on the USIM	471

8.2.1.1	Definition and applicability	471

8.2.1.2	Conformance requirement	471

8.2.1.3	Test purpose	471

8.2.1.4	Method of test	471

8.2.1.4.1	Initial conditions	471

8.2.1.4.2	Procedure	472

8.2.1.5	Acceptance criteria	472

8.2.2	Correct reading of a SM on the USIM	472

8.2.2.1	Definition and applicability	472

8.2.2.2	Conformance requirement	472

8.2.2.3	Test purpose	473

8.2.2.4	Method of test	473

8.2.2.4.1	Initial conditions	473

8.2.2.4.2	Procedure	474

8.2.2.5	Acceptance criteria	474

8.2.3	SM memory capacity exceeded handling	474

8.2.3.1	Definition and applicability	474

8.2.3.2	Conformance requirement	474

8.2.3.3	Test purpose	475

8.2.3.4	Method of test	475

8.2.3.4.1	Initial conditions	475

8.2.3.4.2	Procedure	477

8.2.3.5	Acceptance criteria	477

8.2.4	Correct storage of an SM on the UICC	479

8.2.4.1	Definition and applicability	479

8.2.4.2	Conformance requirement	479

8.2.4.3	Test purpose	479

8.2.4.4	Method of test	479

8.2.4.4.1	Initial conditions	479

8.2.4.4.2	Procedure	481

8.2.4.5	Acceptance criteria	482

8.2.5	Correct reading of a SM on the USIM if USIM and ISIM are present	483

8.2.5.1	Definition and applicability	483

8.2.5.2	Conformance requirement	483

8.2.5.3	Test purpose	483

8.2.5.4	Method of test	484

8.2.5.4.1	Initial conditions	484

8.2.5.4.2	Procedure	485

8.2.5.5	Acceptance criteria	485

8.2.6	Correct reading of a SM on the ISIM if USIM and ISIM are present	485

8.2.6.1	Definition and applicability	485

8.2.6.2	Conformance requirement	485

8.2.6.3	Test purpose	486

8.2.6.4	Method of test	486

8.2.6.4.1	Initial conditions	486

8.2.6.4.2	Procedure	487

8.2.6.5	Acceptance criteria	487

8.2.7	Correct storage of an SM on the UICC	487

8.2.7.1	Definition and applicability	487

8.2.7.2	Conformance requirement	488

8.2.7.3	Test purpose	488

8.2.7.4	Method of test	488

8.2.7.4.1	Initial conditions	488

8.2.7.4.2	Procedure	488

8.2.7.5	Acceptance criteria	488

8.3	MMS related tests	488

8.3.1	UE recognising the priority order of MMS Issuer Connectivity Parameters	488

8.3.1.1	Definition and applicability	488

8.3.1.2	Conformance requirement	489

8.3.1.3	Test purpose	489

8.3.1.4	Method of test	489

8.3.1.4.1	Initial conditions	489

8.3.1.4.2	Procedure	494

8.3.1.5	Acceptance criteria	494

8.3.2	UE recognising the priority order of MMS User Connectivity Parameters	494

8.3.2.1	Definition and applicability	494

8.3.2.2	Conformance requirement	494

8.3.2.3	Test purpose	495

8.3.2.4	Method of test	495

8.3.2.4.1	Initial conditions	495

8.3.2.4.2	Procedure	500

8.3.2.5	Acceptance criteria	500

8.3.3	UE recognising the priority order of MMS Issuer Connectivity Parameters over the MMS User Connectivity Parameters	500

8.3.3.1	Definition and applicability	500

8.3.3.2	Conformance requirement	501

8.3.3.3	Test purpose	501

8.3.3.4	Method of test	501

8.3.3.4.1	Initial conditions	501

8.3.3.4.2	Procedure	507

8.3.3.5	Acceptance criteria	507

8.3.4	Usage of MMS notification	507

8.3.4.1	Definition and applicability	507

8.3.4.2	Conformance requirement	507

8.3.4.3	Test purpose	508

8.3.4.4	Method of test	508

8.3.4.4.1	Initial conditions	508

8.3.4.4.2	Procedure	512

8.3.4.5	Acceptance criteria	512

8.4	UICC presence detection	513

8.4.1	Definition and applicability	513

8.4.2	Conformance requirement	513

8.4.3	Test purpose	513

8.4.4	Method of test	514

8.4.4.1	Initial conditions	514

8.4.4.2	Procedure	514

8.4.5	Acceptance criteria	514

8.5	UICC presence detection when connected to E-UTRAN/EPC	514

8.5.1	Definition and applicability	514

8.5.2	Conformance requirement	514

8.5.3	Test purpose	515

8.5.4	Method of test	515

8.5.4.1	Initial conditions	515

8.5.4.2	Procedure	515

8.5.5	Acceptance criteria	515

9	USIM service handling	488

9.1	Access Point Name Control List handling	488

9.1.1	Access Point Name Control List handling for terminals supporting ACL	488

9.1.1.1	Definition and applicability	488

9.1.1.2	Conformance requirement	488

9.1.1.3	Test purpose	488

9.1.1.4	Method of test	488

9.1.1.4.1	Initial conditions	488

9.1.1.4.2	Procedure	489

9.1.1.5	Acceptance criteria	489

9.1.2	Network provided APN handling for terminals supporting ACL	490

9.1.2.1	Definition and applicability	490

9.1.2.2	Conformance requirement	490

9.1.2.3	Test purpose	490

9.1.2.4	Method of test	490

9.1.2.4.1	Initial conditions	490

9.1.2.4.2	Procedure	491

9.1.2.5	Acceptance criteria	491

9.1.3	Access Point Name Control List handling for terminals not supporting ACL	492

9.1.3.1	Definition and applicability	492

9.1.3.2	Conformance requirement	492

9.1.3.3	Test purpose	492

9.1.3.4	Method of test	492

9.1.3.4.1	Initial conditions	492

9.1.3.4.2	Procedure	493

9.1.3.5	Acceptance criteria	493

9.1.4	Access Point Name Control List handling for terminals supporting ACL connected to E-UTRAN/EPC	493

9.1.4.1	Definition and applicability	493

9.1.4.2	Conformance requirement	493

9.1.4.3	Test purpose	493

9.1.4.4	Method of test	493

9.1.4.4.1	Initial conditions	493

9.1.4.4.2	Procedure	494

9.1.4.5	Acceptance criteria	495

9.1.5	Void9.1.6 Void	495

9.2	Service Dialling Numbers handling	495

9.2.1	Definition and applicability	495

9.2.2	Conformance requirement	495

9.2.3	Test purpose	495

9.2.4	Method of test	496

9.2.4.1	Initial conditions	496

9.2.4.2	Procedure	498

9.2.5	Acceptance criteria	498

10	CSG list handling	499

10.1	CSG list handling for E-UTRA	499

10.1.1	Automatic CSG selection in E-UTRA with CSG list on USIM, success	499

10.1.1.1	Definition and applicability	499

10.1.1.2	Conformance requirement	499

10.1.1.3	Test purpose	499

10.1.1.4	Method of test	499

10.1.1.4.1	Initial conditions	499

10.1.1.4.2	Procedure	500

10.1.1.5	Acceptance criteria	500

10.1.2	Automatic CSG selection in E-UTRA with CSG list on USIM, removal of CSG ID from the USIM	501

10.1.2.1	Definition and applicability	501

10.1.2.2	Conformance requirement	501

10.1.2.3	Test purpose	501

10.1.2.4	Method of test	501

10.1.2.4.1	Initial conditions	501

10.1.2.4.2	Procedure	502

10.1.2.5	Acceptance criteria	502

10.1.3	Manual CSG selection in E-UTRA with CSG list on USIM, success	503

10.1.3.1	Definition and applicability	503

10.1.3.2	Conformance requirement	503

10.1.3.3	Test purpose	503

10.1.3.4	Method of test	503

10.1.3.4.1	Initial conditions	503

10.1.3.4.2	Procedure	504

10.1.3.5	Acceptance criteria	504

10.1.4	Manual CSG selection in E-UTRA with CSG list on USIM, rejected	506

10.1.4.1	Definition and applicability	506

10.1.4.2	Conformance requirement	506

10.1.4.3	Test purpose	506

10.1.4.4	Method of test	506

10.1.4.4.1	Initial conditions	506

10.1.4.4.2	Procedure	507

10.1.4.5	Acceptance criteria	507

10.1.5	CSG selection in E-UTRA with no CSG list on USIM, no IMSI change	508

10.1.5.1	Definition and applicability	508

10.1.5.2	Conformance requirement	508

10.1.5.3	Test purpose	508

10.1.5.4	Method of test	509

10.1.5.4.1	Initial conditions	509

10.1.5.4.2	Procedure	509

10.1.5.5	Acceptance criteria	510

10.1.6	CSG selection in E-UTRA with no CSG list on USIM, with IMSI change	511

10.1.6.1	Definition and applicability	511

10.1.6.2	Conformance requirement	511

10.1.6.3	Test purpose	511

10.1.6.4	Method of test	511

10.1.6.4.1	Initial conditions	511

10.1.6.4.2	Procedure	512

10.1.6.5	Acceptance criteria	513

10.1.7	Manual CSG selection without display restrictions in E-UTRA with ACSG list and OCSG list on USIM	513

10.1.7.1	Definition and applicability	513

10.1.7.2	Conformance requirement	513

10.1.7.3	Test purpose	514

10.1.7.4	Method of test	514

10.1.7.4.1	Initial conditions	514

10.1.7.4.2	Procedure	514

10.1.7.5	Acceptance criteria	515

10.1.8	Manual CSG selection with display restrictions in E-UTRA with ACSG list and OCSG list on USIM	516

10.1.8.1	Definition and applicability	516

10.1.8.2	Conformance requirement	516

10.1.8.3	Test purpose	517

10.1.8.4	Method of test	517

10.1.8.4.1	Initial conditions	517

10.1.8.4.2	Procedure	518

10.1.8.5	Acceptance criteria	518

10.2	CSG list handling for UTRA	520

10.2.1	Manual CSG selection without display restrictions in UTRA with ACSG list and OCSG list on USIM	520

10.2.1.1	Definition and applicability	520

10.2.1.2	Conformance requirement	520

10.2.1.3	Test purpose	520

10.2.1.4	Method of test	520

10.2.1.4.1	Initial conditions	520

10.2.1.4.2	Procedure	521

10.2.1.5	Acceptance criteria	521

10.2.2	Manual CSG selection with display restrictions in UTRA with ACSG list and OCSG list on USIM	523

10.2.2.1	Definition and applicability	523

10.2.2.2	Conformance requirement	523

10.2.2.3	Test purpose	523

10.2.2.4	Method of test	523

10.2.2.4.1	Initial conditions	523

10.2.2.4.2	Procedure	524

10.2.2.5	Acceptance criteria	525

10.2.3	Manual CSG selection in UTRA with CSG list on USIM, success	526

10.2.3.1	Definition and applicability	526

10.2.3.2	Conformance requirement	526

10.2.3.3	Test purpose	527

10.2.3.4	Method of test	527

10.2.3.4.1	Initial conditions	527

10.2.3.4.2	Procedure	527

10.2.3.5	Acceptance criteria	528

11	NAS security context parameter handling	529

11.1	NAS security context parameter handling when service "EMM Information" is available	529

11.1.1	Definition and applicability	529

11.1.2	Conformance requirement	529

11.1.3	Test purpose	530

11.1.4	Method of test	530

11.1.4.1	Initial conditions	530

11.1.4.2	Procedure	530

11.1.5	Acceptance criteria	531

11.2	NAS security context parameter handling when service "EMM Information" is not available, no IMSI change	531

11.2.1	Definition and applicability	531

11.2.2	Conformance requirement	531

11.2.3	Test purpose	532

11.2.4	Method of test	532

11.2.4.1	Initial conditions	532

11.2.4.2	Procedure	532

11.2.5	Acceptance criteria	533

11.3	NAS security context parameter handling when service "EMM Information" is not available, IMSI changed	533

11.3.1	Definition and applicability	533

11.3.2	Conformance requirement	533

11.3.3	Test purpose	534

11.3.4	Method of test	534

11.3.4.1	Initial conditions	534

11.3.4.2	Procedure	534

11.3.5	Acceptance criteria	535

11.4	EPS NAS Security Context Storage	535

11.4.1	Definition and applicability	535

11.4.2	Conformance requirement	535

11.4.3	Test purpose	535

11.4.4	Method of test	536

11.4.4.1	Initial conditions	536

11.4.4.2	Procedure	536

11.4.5	Acceptance criteria	537

12	Non Access Stratum (NAS) Configuration parameter handling	537

12.1	EFNASCONFIG – NAS signaling priority handling	537

12.1.1	Definition and applicability	537

12.1.2	Void	537

12.1.3	Void	537

12.1.4	Void	538

12.1.5	Void	538

12.2	EFNASCONFIG – NMO I Network Mode of Operation I handling	538

12.2.1	Definition and applicability	538

12.2.2	Void	538

12.2.3	Void	538

12.2.4	Void	538

12.2.5	Void	538

12.3	EFNASCONFIG – Attach with IMSI handling	538

12.3.1	Definition and applicability	538

12.3.2	Void	538

12.3.3	Void	538

12.3.4	Void	538

12.3.5	Void	538

12.4	EFNASCONFIG – Verifying Minimum Periodic Search Timer	538

12.4.1	Definition and applicability	538

12.4.2	Void	539

12.4.3	Void	539

12.4.4	Void	539

12.4.5	Void	539

12.5	EFNASCONFIG – Extended access barring handling	539

12.5.1	Definition and applicability	539

12.5.2	Void	539

12.5.3	Void	539

12.5.4	Void	539

12.5.5	Void	539

12.6	EFNASCONFIG – Verifying Timer T3245 Behaviour	539

12.6.1	Definition and applicability	539

12.6.2	Conformance requirement	540

12.6.3	Test purpose	540

12.6.4	Method of test	540

12.6.4.1	Initial conditions	540

12.6.4.2	Procedure	541

12.6.5	Acceptance criteria	541

12.7	EFNASCONFIG – Override NAS signalling low priority	542

12.7.1	Definition and applicability	542

12.7.2	Conformance requirement	542

12.7.3	Test purpose	542

12.7.4	Method of test	542

12.7.4.1	Initial conditions	542

12.7.4.2	Procedure	543

12.7.5	Acceptance criteria	543

12.8	EFNASCONFIG – Override Extended access barring	543

12.8.1	Definition and applicability	543

12.8.2	Conformance requirement	543

12.8.3	Test purpose	544

12.8.4	Method of test	544

12.8.4.1	Initial conditions	544

12.8.4.2	Procedure	544

12.8.5	Acceptance criteria	545

12.9	EFNASCONFIG – Fast First Higher Priority PLMN Search	545

12.9.1	Definition and applicability	545

12.9.2	Conformance requirement	545

12.9.3	Test purpose	545

12.9.4	Method of test	545

12.9.4.1	Initial conditions	545

12.9.4.2	Procedure	546

12.9.5	Acceptance criteria	546

12.10	EFNASCONFIG – E-UTRA Disabling Allowed for EMM cause #15	547

12.10.1	Definition and applicability	547

12.10.2	Conformance requirement	547

12.10.3	Test purpose	547

12.10.4	Method of test	547

12.10.4.1	Initial conditions	547

12.10.4.2	Procedure	548

12.10.5	Acceptance criteria	548

12.11	EFNASCONFIG – SM_RetryWaitTime	549

12.11.1	Definition and applicability	549

12.11.2	Conformance requirement	549

12.11.3	Test purpose	549

12.11.4	Method of test	549

12.11.4.1	Initial conditions	549

12.11.4.2	Procedure	550

12.11.5	Acceptance criteria	550

12.12	EFNASCONFIG – SM_RetryAtRATChange	550

12.12.1	Definition and applicability	550

12.12.2	Conformance requirement	550

12.12.3	Test purpose	550

12.12.4	Method of test	551

12.12.5	Acceptance criteria	551

12.13	EFNASCONFIG – No E-UTRA Disabling In 5GS	551

12.13.1	Definition and applicability	551

12.13.2	Conformance requirement	551

12.13.3	Test purpose	551

12.13.4	Method of test	551

12.13.4.1	Initial conditions	551

12.13.4.2	Procedure	552

12.13.5	Acceptance criteria	552

13	UICC interface during PSM	553

13.1	UICC interface in PSM handling for E-UTRAN – No UICC deactivation in PSM	553

13.1.1	Definition and applicability	553

13.1.2	Conformance requirement	553

13.1.3	Test purpose	553

13.1.4	Method of test	553

13.1.4.1	Initial conditions	553

13.1.4.2	Procedure	554

13.1.5	Acceptance criteria	554

13.2	UICC interface in PSM handling for E-UTRAN – PSM not accepted by E-USS/NB-SS	554

13.2.1	Definition and applicability	554

13.2.2	Conformance requirement	554

13.2.3	Test purpose	555

13.2.4	Method of test	555

13.2.4.1	Initial conditions	555

13.2.4.2	Procedure	555

13.2.5	Acceptance criteria	555

13.3	UICC interface in PSM handling for E-UTRAN – UICC deactivation in PSM	556

13.3.1	Definition and applicability	556

13.3.2	Conformance requirement	556

13.3.3	Test purpose	556

13.3.4	Method of test	556

13.3.4.1	Initial conditions	556

13.3.4.2	Procedure	557

13.3.5	Acceptance criteria	557

13.4	UICC interface in PSM for E-UTRAN – SUSPEND UICC	557

13.4.1	Definition and applicability	557

13.4.2	Conformance requirement	557

13.4.4	Method of test	558

13.4.4.1	Initial conditions	558

13.4.4.2	Procedure	558

13.4.5	Acceptance criteria	559

14	UICC interface during eDRX	559

14.1	UICC interface during eDRX for E-UTRAN – eDRX is not supported by the UICC	559

14.1.1	Definition and applicability	559

14.1.2	Conformance requirement	559

14.1.3	Test purpose	559

14.1.4	Method of test	559

14.1.4.1	Initial conditions	559

14.1.4.2	Procedure	560

14.1.5	Acceptance criteria	560

14.2	UICC interface during eDRX for E-UTRAN – eDRX is not accepted by E-USS/NB-SS	561

14.2.1	Definition and applicability	561

14.2.2	Conformance requirement	561

14.2.3	Test purpose	561

14.2.4	Method of test	561

14.2.4.1	Initial conditions	561

14.2.5	Acceptance criteria	562

14.3	UICC interface during eDRX for E-UTRAN – UICC deactivation during eDRX	562

14.3.1	Definition and applicability	562

14.3.2	Conformance requirement	562

14.3.3	Test purpose	563

14.3.4	Method of test	563

14.3.4.1	Initial conditions	563

14.3.4.2	Procedure	564

14.3.5	Acceptance criteria	564

14.4	UICC interface during eDRX for E-UTRAN– SUSPEND UICC	564

14.4.1	Definition and applicability	564

14.4.2	Conformance requirement	564

14.4.3	Test purpose	565

14.4.4	Method of test	565

14.4.4.1	Initial conditions	565

14.4.4.2	Procedure	566

14.4.5	Acceptance criteria	566

15	Authentication procedure and NAS security context handling for 5G	566

15.1	Authentication procedure for EAP-AKA'	566

15.1.1	Authentication procedure for EAP-AKA' - Authentication is successful	566

15.1.1.1	Definition and applicability	566

15.1.1.2	Conformance requirement	567

15.1.1.3	Test purpose	567

15.1.1.4	Method of test	567

15.1.1.4.1	Initial conditions	567

15.1.1.4.2	Procedure	568

15.1.1.5	Acceptance criteria	568

15.1A.1	Authentication procedure for EAP-AKA' - Authentication is Successful	569

15.1A.1.1	Definition and applicability	569

15.1A.1.2	Conformance requirement	570

15.1A.1.3	Test purpose	570

15.1A.1.4	Method of test	570

15.1A.1.4.1	Initial conditions	570

15.1A.1.4.2	Procedure	570

15.1A.1.5	Acceptance criteria	571

15.1.2	Authentication procedure for EAP-AKA' – Authentication is successful - GSM UICC	572

15.1.2.1	Definition and applicability	572

15.1.2.2	Conformance requirement	572

15.1.2.3	Test purpose	573

15.1.2.4	Method of test	573

15.1.2.4.1	Initial conditions	573

15.1.2.4.2	Procedure	573

15.1.2.5	Acceptance criteria	574

15.1.3	Authentication procedure for EAP-AKA' – AUTN fails on the USIM	574

15.1.3.1	Definition and applicability	574

15.1.3.2	Conformance requirement	574

15.1.3.3	Test purpose	575

15.1.3.4	Method of test	575

15.1.3.4.1	Initial conditions	575

15.1.3.4.2	Procedure	575

15.1.3.5	Acceptance criteria	576

15.1.4	Authentication procedure for EAP-AKA' - after SUPI is changed	576

15.1.4.1	Definition and applicability	576

15.1.4.2	Conformance requirement	576

15.1.4.3	Test purpose	576

15.1.4.4	Method of test	576

15.1.4.4.2	Procedure	577

15.2	Authentication procedure for 5G AKA	578

15.2.1.4	Method of test	579

15.2.1.4.1	Initial conditions	579

15.2A.1.4	Method of test	581

15.2A.1.4.1	Initial conditions	581

15.2A.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC	583

15.2A.2.1	Definition and applicability	583

15.2A.2.2	Conformance requirement	583

15.2A.2.3	Test purpose	584

15.2A.2.4	Method of test	584

15.2A.2.4.1	Initial conditions	584

15.2A.2.4.2	Procedure	584

15.2A.2.5	Acceptance criteria	585

15.2.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC	585

15.2.2.1	Definition and applicability	585

15.2.2.2	Conformance requirement	585

15.2.2.3	Test purpose	586

15.2.2.4	Method of test	586

15.2.2.4.1	Initial conditions	586

15.2.2.4.2	Procedure	586

15.2.2.5	Acceptance criteria	587

15.2.3	Authentication procedure 5G AKA – AUTN fails on the USIM	587

15.2.3.1	Definition and applicability	587

15.2.3.2	Conformance requirement	587

15.2.3.3	Test purpose	588

15.2.3.4	Method of test	588

15.2.3.4.1	Initial conditions	588

15.2.3.4.2	Procedure	588

15.2.3.5	Acceptance criteria	589

15.2.4	Authentication procedure for 5G AKA - after SUPI is changed	589

15.2.4.1	Definition and applicability	589

15.2.4.2	Conformance requirement	589

15.2.4.3	Test purpose	589

15.2.4.4	Method of test	590

15.2.4.4.1	Initial conditions	590

15.2.4.4.2	Procedure	590

15.2.4.5	Acceptance criteria	591

15.2.5.2	Conformance requirement	591

15.2.5.4	Method of test	592

15.2.5.4.1	Initial conditions	592

15.2.5.4.2	Procedure	593

15.2.5.5	Acceptance criteria	594

15.3	Authentication procedure for 5G AKA over non-3GPP access via N3IWF	597

15.3.1.4	Method of test	598

15.3.1.4.1	Initial conditions	598

15.3.1.4.2	Procedure	598

15.4	5G AKA Authentication procedure for Multiple registration over 3GPP access and non-3GPP access	601

15.4.1.4	Method of test	602

15.4.1.4.1	Initial conditions	602

15.4.1.4.2	Procedure	602

15.4.2.4	Method of test	608

15.4.2.4.1	Initial conditions	608

15.4.2.4.2	Procedure	608

16	UE Route Selection Policy (URSP) procedure	612

16.1	Pre-configured URSP rules	612

16.1.1	Support for URSP by USIM	612

16.1.1.1	Definition and applicability	612

16.1.1.2	Conformance requirement	612

16.1.1.3	Test purpose	613

16.1.1.4	Method of test	613

16.1.1.4.1	Initial conditions	613

16.1.1.4.2	Procedure	615

16.1.1.5	Acceptance criteria	615

16.1.2	Support for URSP by ME	615

16.1.2.1	Definition and applicability	615

16.1.2.2	Conformance requirement	615

16.1.2.3	Test purpose	615

16.1.2.4	Method of test	615

16.1.2.4.1	Initial conditions	615

16.1.2.4.2	Procedure	616

16.1.2.5	Acceptance criteria	616

16.1.3	Support of Signalled URSP	617

16.1.3.1	Definition and applicability	617

16.1.3.2	Conformance requirement	617

16.1.3.3	Test purpose	617

16.1.3.4	Method of test	617

16.1.3.4.1	Initial conditions	617

16.1.3.4.2	Procedure	619

16.1.3.5	Acceptance criteria	619

17	CAG list handling	620

17.1	CAG list handling for 5G	620

17.1.1	Automatic CAG selection with preconfigured CAG list on USIM	620

17.1.1.1	Definition and applicability	620

17.1.1.2	Conformance requirement	620

17.1.1.3	Test purpose	620

17.1.1.4	Method of tests	620

17.1.1.4.1	Initial conditions	620

17.1.1.4.2	Procedure	621

17.1.1.4.3	Acceptance criteria	621

18	NSSAA procedure with SSIM	621

18.1	SSIM management procedures	621

18.1.1	SSIM identification, S-NSSAI list request	621

18.1.1.1	Definition and applicability	621

18.1.1.2	Conformance requirement	621

18.1.1.3	Test purpose	622

18.1.1.4	Method of tests	622

18.1.1.4.1	Initial conditions	622

18.1.1.4.2	Procedure	623

18.1.1.4.2	Acceptance criteria	624

18.2	SSIM commands	624

18.2.1	AUTHENTICATE command, Identity EAP-Request/EAP-Response messages	624

18.2.1.1	Definition and applicability	624

18.2.1.2	Conformance requirement	624

18.2.1.3	Test purpose	625

18.2.1.4	Method of tests	625

18.2.1.4.1	Initial conditions	625

18.2.1.4.2	Procedure	625

18.2.1.4.3	Acceptance criteria	625

18.2.2	AUTHENTICATE command, other EAP-Request/EAP-Response messages	625

18.2.2.1	Definition and applicability	625

18.2.2.2	Conformance requirement	626

18.2.2.3	Test purpose	626

18.2.2.4	Method of tests	626

18.2.2.4.1	Initial conditions	626

18.2.2.4.2	Procedure	626

18.2.2.4.3	Acceptance criteria	626

18.2.3	AUTHENTICATE command, EAP-Success/EAP-Failure messages	627

18.2.3.1	Definition and applicability	627

18.2.3.2	Conformance requirement	627

18.2.3.3	Test purpose	627

18.2.3.4	Method of tests	627

18.2.3.4.1	Initial conditions	627

18.2.3.4.2	Procedure	627

18.2.3.4.3	Acceptance criteria	627

19	SOR-CMCI	628

19.1	SOR-CMCI PDU session	628

19.1.1	SOR-CMCI PDU session 'DNN'	628

19.1.1.1	Definition and applicability	628

19.1.1.2	Conformance requirement	628

19.1.1.3	Test purpose	628

19.1.1.4	Method of tests	628

19.1.1.4.1	Initial conditions	628

19.1.1.4.2	Procedure	629

19.1.1.4.3	Acceptance criteria	629

19.1.2	SOR-CMCI PDU session 'S-NSSAI SST and SD'	630

19.1.2.1	Definition and applicability	630

19.1.2.2	Conformance requirement	630

19.1.2.3	Test purpose	630

19.1.2.4	Method of tests	630

19.1.2.4.1	Initial conditions	630

19.1.2.4.2	Procedure	631

19.1.2.4.3	Acceptance criteria	631

19.1.3	SOR-CMCI PDU session with multiple rules 'DNN' and 'S-NSSAI SST and SD'	631

19.1.3.1	Definition and applicability	631

19.1.3.2	Conformance requirement	632

19.1.3.3	Test purpose	632

19.1.3.4	Method of tests	632

19.1.3.4.1	Initial conditions	632

19.1.3.4.2	Procedure	633

19.1.3.4.3	Acceptance criteria	633

20	SENSE	633

20.1	Usage of Operator Controlled Signal threshold per ACT (operator enabled SENSE usage) on E-UTRAN	633

20.1.1	Definition and applicability	633

20.1.2	Conformance requirement	634

20.1.3	Test purpose	634

20.1.4	Method of test	634

20.1.4.1	Initial conditions	634

20.1.4.2	Procedure	635

20.1.5	Acceptance criteria	636

20.2	Usage of Operator Controlled Signal threshold per ACT (operator disabled SENSE usage) on E-UTRAN	636

20.2.1	Definition and applicability	636

20.2.2	Conformance requirement	637

20.2.3	Test purpose	637

20.2.4	Method of test	637

20.2.4.1	Initial conditions	637

20.2.4.2	Procedure	637

20.2.5	Acceptance criteria	638

Annex A (informative):	Structure for uac-BarringInfo and Operator-defined access category	629

Annex B (informative):	5G-NR generic procedure for 5G Registration on 3GPP access with IMS service	634

Annex C (informative):	Change history	635

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document defines the application test specification.

The aim of the present document is to ensure interoperability between an UICC and a terminal independently of the respective manufacturer, card issuer or operator. The present document does not define any aspects related to the administrative management phase of the UICC. Any internal technical realisation of either the UICC or the Terminal is only specified where these are reflected over the interface.

Application specific details for applications residing on an UICC are specified in the respective application specific documents. The logical and physical Characteristics of the UICC Terminal interface is specified in document ETSI TS 102 221 [5]. The Universal Subscriber Identity Module (USIM)-application for 3G telecommunication networks is specified in document TS 31.102 [4].