3GPP TSG-CT Meeting #106	CP-243315

# Madrid, Spain; 9th – 10th December 2024										(was CP-243290)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.514 | CR | 0050 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on SL-MT-LR for periodic, triggered Location Events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.20.4 of 3GPP TS 23.273, the procedures of SL-MT-LR for periodic, triggered Location Events are defined. However, - in clause 5.2.2.4 of TS 24.571, UE initiated Event Reporting Procedure does not support to be applied to procedures defined in clause 6.20.4 of 3GPP TS 23.273. So it is proposed to update on clause 5.2.2.4 of TS 24.571, the UE initiated Event Reporting Procedure, to apply the UE-LMF interaction to support procedures defined in clause 6.20.4 of 3GPP TS 23.273. So the related reference description in TS 24.514 needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the reference description in clause 7.3 for the UE initiated Event Reporting Procedure update to apply the UE-LMF interaction to support procedures defined in clause 6.20.4 of 3GPP TS 23.273. Backward compatibility analysis: The CR is backward compatible since it is to add clarification to the reference description the UE initiated Event Reporting Procedure to support procedures defined in clause 7.3 of 3GPP TS 23.273 in order to align the stage 2 requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures of SL-MT-LR for periodic, triggered Location Events defined in clause 6.20.4 of 3GPP TS 23.273 is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: - place the new item into the above part - remove "Updated" in the descrption to avoid misunderstanding of the procedure name Rev#2: - 1st letter of words in the procedure name changed to lower case Rev#3: - Change the WIC on the coversheet to “TEI19, Ranging_SL” Rev#4: - Change source to TSG from “C1” to “Xiaomi” |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.3	Ranging and sidelink positioning communication on LCS aspect

The UE or the network initiates the ranging and sidelink positioning communication utilizing the location services signalling messages defined in 3GPP TS 23.273 [11] to obtain the location information including one or more of the following:

a)	absolute location of the UE;

b)	absolute velocity of the UE;

c)	range and direction between a pair of UEs (see clause 5.10 of 3GPP TS 23.032 [16]);

d)	relative location between a pair of UEs; (see clause 5.11 and 5.12 of 3GPP TS 23.032 [16]); and

e)	relative velocity between a pair of UEs (see clause 8.4a of 3GPP TS 23.032 [16]).

In order to obtain the absolute location the absolute velocity, or both of the target UE, the following procedures defined in 3GPP TS 23.273 [11] are applied:

a)	 SL-MO-LR procedure;

b)	MO-LR using sidelink positioning;

c)	SL-MT-LR procedure;

d)	SL-MT-LR for periodic and triggered Location events procedure; and

e)	MT-LR using sidelink positioning.

NOTE:	In order to estimate the location of the UE, the network can decide to utilize the ranging and sidelink positioning during the MO-LR procedure (i.e., MO-LR using sidelink positioning) and the MT-LR procedure (i.e., MT-LR using sidelink positioning).

In order to obtain one or more among the relative location, the range and direction, and the relative velocity between a pair of UEs, the following procedures defined in 3GPP TS 23.273 [11] are applied:

a)	SL-MO-LR procedure;

b)	SL-MT-LR procedure;

c)	SL-MT-LR for periodic and triggered Location events procedure.

The following interactions between UE and LMF for ranging and sidelink positioning are supported:

  - Sidelink positioning information transport procedure as defined in clause 5.2.1.6 of 3GPP TS 24.571 [22];

  - Network initiated RSPP supplementary information transport procedure as defined in clause 5.2.1.7 of 3GPP TS 24.571 [22];

  - Sidelink mobile terminating location request procedure as defined in clause 5.2.1.8 of 3GPP TS 24.571 [22];

  - Sidelink mobile originated location request procedure as defined in clause 5.2.2.9 of 3GPP TS 24.571 [22];

  - UE initiated sidelink positioning information transport procedure as defined in clause 5.2.2.10 of 3GPP TS 24.571 [22];

  - UE initiated RSPP supplementary information transport procedure as defined in clause 5.2.2.11 of 3GPP TS 24.571 [22]; and

  - UE initiated event reporting procedure as defined in clause 5.2.2.4 of 3GPP TS 24.571 [22].

* * * End of Changes * * * *