3GPP TSG-CT Meeting #107	CP-250212

Incheon, Korea (Republic Of), 12th Mar 2025 - 13th Mar 2025	revision of C3-250527

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0367 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of SS_ADAE_AIMLMemberCapabilityAnalytics API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per stage-2 TS 23.436 the requirement is to define Application Layer AI/ML Member Capability Analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution proposes to define the SS_ADAE_AIMLMemberCapabilityAnalytics API for the application layer AI/ML member capability analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not fulfilment of stage-2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.28(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a new OpenAPI file for the following API: TS29549_SS_ADAE_AIMLMemberCapabilityAnalytics.yaml. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2 for CT#107 corrections to enable successful OpenAPI parsing: API version fixed as "1.0.0-alpha.1". Fixed reference to $ref: '#/components/schemas/AimlMemberIds' as $ref: '#/components/schemas/AimlMemberId'. Removed "AnalyticsOutput" data type definition, as it is FFS in main body. In AimlMemberCapNoitif, changed "analyticsOutput" as "string". Removed the unused data type "AimlmcaList". |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

# A.28	SS_ADAE_AIMLMemberCapabilityAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_AIMLMemberCapabilityAnalytics

description: |

API for ADAE AIML member capability analytics analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.549 v19.2.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-aimlmca/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/application-aimlmca:

post:

description: Creates a new individual AIML member capability analytics event subscription.

operationId: CreateIndAimlmcaEventSubsc

tags:

- AIML member capability analytics event subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

callbacks:

paNotification:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: AIML member capability analytics event subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/application-aimlmca/{appAimlmcaId}:

get:

description: Retrieves an individual AIML member capability analytics event subscription.

operationId: ReadIndAimlmcaEventSubsc

tags:

- Individual AIML member capability analytics event subscription (Document)

parameters:

- name: appAimlmcaId

in: path

description: Identifier of an individual AIML member capability analytics event subscription.

required: true

schema:

type: string

responses:

'200':

description: >

The individual AIML member capability analytics subscription matching the appAimlmcaId is retrieved.

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual AIML member capability analytics event subscription.

operationId: DeleteIndAimlmcaEventSubsc

tags:

- Individual AIML member capability analytics event subscription (Document)

parameters:

- name: appAimlmcaId

in: path

description: Identifier of an individual AIML member capability analytics event subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual AIML member capability analytics subscription matching the appAimlmcaId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AppAimlMemCapSub:

description: Represents an individual AIML member capability analytics event subscription resource.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

analyticsId:

$ref: '#/components/schemas/AppAimlMemCapAnalytics'

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

ueIds:

type: array

items:

type: string

minItems: 1

description: >

One or more VAL UE IDs whose analytics or data collection are subject to subcription.

aimlMemberIds:

type: array

items:

$ref: '#/components/schemas/AimlMemberId'

minItems: 1

description: >

One or more AIML members whose analytics or data collection are subject to subcription.

confLevel:

type: integer

minimum: 0

maximum: 100

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- analyticsType

AppAimlMemCapNotif:

description: Represents notification of the AIML member capability analytics event.

type: object

properties:

analyticsOutput:

type: string

analyticsId:

$ref: '#/components/schemas/AppAimlMemCapAnalytics'

ueIds:

type: array

items:

type: string

minItems: 1

description: List of VAL UEs identifiers.

aimlMemberIds:

type: array

items:

$ref: '#/components/schemas/AimlMemberId'

minItems: 1

description: List of AIML members identifiers.

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- analyticsOutput

- analyticsId

# SIMPLE DATA TYPES

#

AimlMemberId:

description: Represents unique identifier of a AIMLE Member.

type: string

#

# ENUMERATIONS

#

AppAimlMemCapAnalytics:

anyOf:

- type: string

enum:

- APP_MEMBER_CAPABILITY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents Connection Status Events.

Possible values are:

- APP_MEMBER_CAPABILITY: Indicates that the identifier of the analytics consumer.

# * * * * End of Change * * * *