3GPP TSG CT Meeting #107	CP-250224

# Incheon , KR, 12th – 13th March, 2025															was C3-250263

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.257 | CR | 0064 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates and corrections to the UAE_FlightRouteSupport API |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, InterDigital |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, InterDigital |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  | 2025-02-28 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | The following issues have been identified in the definition of the UAE_FlightRouteSupport API: - In clause 6.7.6.2.3, the list of waypoints constituting the flight path is unnecessarily restricted to a minimum of 2 array elements and it is not clarified that it is the list of waypoints in between the start and end waypoints provided in the related request and it does not include them. - In clause 6.7.6.2.4, the start point/time and end point/time can be merged to be conveyed via a single attribute encoded using the Waypoint data structure. - Various terminilogy alignments need to be addressed. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues are not addressed. |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.1, 5.8.2.2.2, 6.7.4.2.2, 6.7.6.1, 6.7.6.2.3, 6.7.6.2.4, A.8 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces a backwards compatible new feature to the OpenAPI descriptions of the following APIs: - TS29257_UAE_FlightRouteSupport.yaml |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 (to CT#107): - Update the names of the "startPoint" and "endpoint" attributes of the FlightChars data type in the OpenAPI description to align with the related changes in the main body of the TS, clause 6.7.6.2.4. |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

### 5.8.1	Service Description

The UAE_FlightRouteSupport service exposed by the UAE Server enables a service consumer to:

-	request flight routes management.

* * * * Next changes * * * *

##### 5.8.2.2.2	Flight Route Request

Figure 5.8.2.2.2-1 depicts a scenario where a service consumer sends a request to the UAE Server to request the flight route plan (see also clause 7.10 of 3GPP°TS°23.255°[6]).

Figure 5.8.2.2.2-1: Procedure for Flight Route Request

1.	In order to request the flight route plan, the service consumer shall send an HTTP POST request (custom operation: "FlightRouteRequest") to the UAE Server, with the request body including the FlightRouteReq data structure.

If the UAE Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI of the resource located in an alternative UAE Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

2a.	Upon success, the UAE Server shall respond with an HTTP "200 OK" status code with the response body containing the requested flight route plan and the related information within the FlightRouteResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.7.7.

* * * * Next changes * * * *

##### 6.7.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 6.7.4.2.2-1 and 6.7.4.2.2-2.

Table 6.7.4.2.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlightRouteReq | M | 1 | Contains the parameters to request the flight route plan. |

Table 6.7.4.2.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlightRouteResp |  |  | 200 OK | Successful case. The flight route request is successfully received and processed, and the requested flight route plan and the related information are returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2] |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.7.4.2.2-3: Headers supported by the 307 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative UAE Server. |

Table 6.7.4.2.2-4: Headers supported by the 308 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative UAE Server. |

* * * * Next changes * * * *

#### 6.7.6.1	General

This clause specifies the application data model supported by the API.

Table 6.7.6.1-1 specifies the data types defined for the UAE_FlightRouteSupport API.

Table 6.7.6.1-1: UAE_FlightRouteSupport API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| FlightChars | 6.7.6.2.4 | Represents the flight characteristics. |  |
| FlightRouteReq | 6.7.6.2.2 | Represents a flight route request. |  |
| FlightRouteResp | 6.7.6.2.3 | Represents a flight route response. |  |

Table 6.7.6.1-2 specifies data types re-used by the UAE_FlightRouteSupport API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the UAE_FlightRouteSupport API.

Table 6.7.6.1-2: UAE_FlightRouteSupport API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| C2LinkQualityThrlds | Clause 6.1.6.2.11 | Represents the C2 link quality thresholds. |  |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| GeographicArea | 3GPP TS 29.572 [8] | Represents a geographical area. |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| UavId | Clause 6.1.6.2.7 | Represents the identifier of a UAV (e.g., UAV, UAV-C). |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |
| Waypoint | Clause 6.6.6.2.8 | Represents a waypoint along a flight path. |  |

* * * * Next changes * * * *

##### 6.7.6.2.3	Type: FlightRouteResp

Table 6.7.6.2.3-1: Definition of type FlightRouteResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| frWaypointsList | array(Waypoint) | C | 1..N | Contains the flight route encoded in the form of a list of waypoint(s) in between the start waypoint and end waypoint of the flight provided within the corresponding flight route request. (NOTE) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.7.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.7.6.2.4	Type: FlightChars

Table 6.7.6.2.4-1: Definition of type FlightChars

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flightStart | Waypoint | M | 1 | Contains the start point (e.g., a geographical area in the form of a point and an altitude value) of the flight together with the start time. |  |
| flightEnd | Waypoint | M | 1 | Contains the end point (e.g., a geographical area in the form of a point and an altitude value) of the flight together with the end time. |  |
| reqMinQos | C2LinkQualityThrlds | M | 1 | Contains the required minimum QoS to support the flight's mission. |  |
| servAvail | integer | O | 0..1 | Contains the required service availability expressed as a percentage. Minimum: 0, Maximum: 100. |  |
| shortestRouteInd | boolean | O | 0..1 | Indicates whether the shortest route is needed for the flight. - "true" indicates that the shortest route is needed for the flight. - "false" indicates that the shortest route is not needed for the flight. - The default value is "false" if this attribute is omitted. |  |

* * * * Next changes * * * *

# A.8	UAE_FlightRouteSupport API

openapi: 3.0.0

info:

title: UAE Server Flight Route Support Service

version: 1.0.0-alpha.1

description: |

UAE Server Flight Route Support Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.257 V19.1.0; Application layer support for Uncrewed Aerial System (UAS);

UAS Application Enabler (UAE) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.257/

servers:

- url: '{apiRoot}/uae-frs/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/fr-request:

post:

summary: Request the flight route plan.

operationId: FlightRouteRequest

tags:

- FlightRouteRequest

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/FlightRouteReq'

responses:

'200':

description: >

OK. The flight route request is successfully received and processed, and the requested

flight route plan and the related information are returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/FlightRouteResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

FlightRouteReq:

description: >

Represents a flight route request.

type: object

properties:

requestorId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

uavId:

$ref: 'TS29257_UAE_C2OperationModeManagement.yaml#/components/schemas/UavId'

flightChars:

$ref: '#/components/schemas/FlightChars'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- requestorId

- uavId

- flightChars

FlightRouteResp:

description: >

Represents a flight route response.

type: object

properties:

frWaypointsList:

type: array

items:

$ref: 'TS29257_UAE_FlightPathMonitoring.yaml#/components/schemas/Waypoint'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

anyOf:

- required: [frWaypointsList]

FlightChars:

description: Represents the flight characteristics.

type: object

properties:

flightStart:

$ref: 'TS29257_UAE_FlightPathMonitoring.yaml#/components/schemas/Waypoint'

flightEnd:

$ref: 'TS29257_UAE_FlightPathMonitoring.yaml#/components/schemas/Waypoint'

reqMinQos:

$ref: 'TS29257_UAE_C2OperationModeManagement.yaml#/components/schemas/C2LinkQualityThrlds'

servAvail:

type: integer

minimum: 0

maximum: 100

shortestRouteInd:

type: boolean

default: false

description: >

Indicates whether the shortest route is needed for the flight.

true indicates that the shortest route is needed for the flight.

false indicates that the shortest route is not needed for the flight.

The default value is false if this attribute is omitted.

required:

- flightStart

- flightEnd

- reqMinQos

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

* * * * End of changes * * * *