3GPP TSG-CT WG4 Meeting #117	C4-233683

Göteborg, Sweden, 21st– 25th August 2023	was C4-233201

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.581 | CR | 0037 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the object acquisition identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, 5MBS |  |  |  |  |  | Date: |  |  | 2023-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 26.502 has clarified the object distribution method includes four modes: 1. OBJECT_SINGLE, When the pull-based object acquisition method is provisioned, the MBS Distribution Session parameters (see table 4.5.6-2) shall cite a set of one or more object URLs as Object acquisition identifiers. When the push-based object acquisition method is provisioned, the set of Object acquisition identifiers shall be empty. 2. OBJECT_COLLECTION, The MBS Distribution Session parameters (see table 4.5.6-2) shall cite a single object manifest URL in Object acquisition identifiers. 3. OBJECT_CAROUSEL The MBS Distribution Session parameters (see table 4.5.6-2) shall cite a single object manifest URL in Object acquisition identifiers. 4. OBJECT_STREAMING, For both object acquisition methods, the MBS Distribution Session parameters (see table 4.5.6-2) shall cite a single Application Service Entry Point URL of each type in Object acquisition identifiers Therefore, the attribute objAcquisitionIdsPull can be either an array of Uris, or a single Uri. However, TS 29.581 defines the objAcquisitionIdsPull as an array that at least with an element and the NOTE 1 also limited the usage of this array only can have a single element, which are not inline with SA4 requirements. Further discussion can be found in the DP C4-233199. Also the description of the meaning of Uri has been further clarified in TS 26.502, the description in TS 29.581 shall be updated. OBJECT_COLLECTION and OBJECT_CAROUSEL can be used for PUSH ingest method, therefore the description in Table 6.1.6.3.4 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove NOTE 1 in table 6.1.6.2.5 Correct the description of the attributes objAcquisitionIdsPull and objAcquisitionIdsPush. Correct the Enumeration ObjDistributionOperatingMode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA4 Stage 2 requirements thus possible incorrect implementation and leads interoperability problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.5, 6.1.6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has no impact on OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: remove extra description of objAcquisitionIdsPull and objAcquisitionIdsPush, and just leave the reference to 3GPP TS 26.502 [17]. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** First Change ***

##### 6.1.6.2.5	Type: ObjDistributionData

Table 6.1.6.2.5-1: Definition of type ObjDistributionData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| objDistributionOperatingMode | ObjDistributionOperatingMode | M | 1 | Operating Mode for the Object distribution method |  |
| objAcquisitionMethod | ObjAcquisitionMethod | M | 1 | Indicates whether the objects(s) are to be pushed into the MBSTF by the MBS Application Provider or whether they are to be pulled from the MBS Application Provider by the MBSTF |  |
| objAcquisitionIdsPull | array(Uri) | C | 1..N | Represents the URL(s) (expressed as path(es) relative to the object ingest base URL provided in the "objIngestBaseUrl" attribute) pointing to the root object(s) to be pulled by the MBSTF and then distributed during this MBS Distribution Session. This attribute shall be provided, when available. See clause 6.1 of 3GPP TS 26.502 [17] (NOTE 1, NOTE 2, NOTE 5) |  |
| objAcquisitionIdPush | Uri | C | 0..1 | Represents the URL (expressed as a path relative to the object ingest base URL provided in the "objIngestBaseUrl" attribute) pointing to the root object(s) to be pushed to the MBSTF and then distributed during this MBS Distribution Session. This attribute shall be provided, when available. See clause 6.1 of 3GPP TS 26.502 [17] (NOTE 2, NOTE 5) |  |
| objIngestBaseUrl | Uri | C | 0..1 | Represents the object ingest base URL. It contains a URL prefix that is replaced by the object distribution base URL by the MBSTF to derive the object distribution URI prior to the distribution of ingested objects. Shall be present if the object Distribution base URL (within the "objDistributionBaseUrl") is present. (NOTE 3, NOTE 4) |  |
| objDistributionBaseUrl | Uri | O | 0..1 | Represents the object distribution base URL. It contains a URL prefix with which the MBSTF replaces the object ingest base URL to derive the object distribution URL prior to the distribution of ingested objects. (NOTE 4) |  |
| NOTE 1: void. NOTE 2: Either the "objAcquisitionIdsPull" attribute or the "objAcquisitionIdsPush" attribute shall be provided, when available. NOTE 3: When the "objAcquisitionMethod" attribute is set to "PULL", this attribute may be provided by the MBSF during the creation and/or update/modification of the MBS Distribution Session (if received from the AF). When the "objAcquisitionMethod" attribute is set to "PUSH", this attribute may be provided by the MBSTF in the response to the creation and/or update/modification request of the MBS Distribution Session. NOTE 4: When the "objDistributionBaseUrl" attribute is omitted, nothing is replaced/removed from the object ingest URL when deriving the object distribution URL. NOTE 5: For group message delivery, the NEF transforms the group message payload into a file, so as an object to be ingested and distributed by the MBSTF using PULL or PUSH Method as specified in 3GPP TS 23.247 [15]. |  |  |  |  |  |

# *** Next Change ***

##### 6.1.6.3.4	Enumeration: ObjDistributionOperatingMode

The enumeration ObjDistributionOperatingMode defines the mode of data ingestion for Object distribution method. It shall comply with the values defined in table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: Enumeration ObjDistributionOperatingMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SINGLE" | Each object ingested by the MBSTF is distributed once |  |
| "COLLECTION" | A set of objects described by a manifest is ingested by the MBSTF and distributed once |  |
| "CAROUSEL" | A set of one or more objects described by a manifest is ingested by the MBSTF and distributed according to a repetition pattern specified in the manifest |  |
| "STREAMING" | A sequence of objects is ingested by the MBSTF and streamed in real time |  |

*** End of Changes ***