3GPP TSG-CT Meeting #101	CP-232069

# Bangalore, India, 11th – 12th September 2023                                    (Revision of C4-233855)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.531 | CR | 0163 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on Subscription, Unsubscription and Notification of NSSF for Network Slice and Network Slice Instance replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the new agreement in S2-2307539 new subscription events have been added for the NSSF with respect to Network Slice Replacement and Network Slice Instance Replacement. Accordingly, new input requirements have been added for the Subscription and Notification of these new events. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates the NSSF subscription, unsubscription and notification events to incorporate the new requirements with respect to Network Slice Replacement and Network Slice Instance Replacement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-compliant with stage-2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 5.3.2.3.1, 5.3.2.3.2, 5.3.2.4.1, 5.3.2.5.1, 6.2.6.1, 6.2.6.2.8, 6.2.6.2.9, 6.2.6.2.10, 6.2.6.2.x, 6.2.6.2.y, 6.2.6.3.3, 6.2.8, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new backward compatible features to the OpenAPI definition of the Nssf_NSSAIAvailability API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 3: corrects the following errors found during the implementation of the CT4 agreed CR: - "item" has been corrected to "items" for snssaiToSubscribe in SnssaiReplacementSubscribeInfo in the OpenAPI. - "item" has been corrected to "items" for nsiToSubscribe and for snssaiToSubscribe in NsiUnavailabilitySubscribeInfo in the OpenAPI. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.3.1	Service Description

The Nnssf_NSSAIAvailability service is used by the NF service consumer (e.g AMF) to update the S-NSSAI(s) the AMF supports on a per TA basis on the NSSF, subscribe and unsubscribe the notification of any changes to the NSSAI availability information on a per TA basis, of the S-NSSAIs available per TA (unrestricted) and the restricted S-NSSAI(s) per PLMN in that TA in the serving PLMN of the UE.

It also enables the NF service consumer (e.g. AMF) to update the NSAG(s) associated with the S-NSSAI(s) supported by the AMF on a per TA basis.

It also enables the NF service consumer (e.g., AMF, V-NSSF) to receive updates for Network Slice Replacement and Network Slice Instance Replacement.

* * * Next Change * * * *

##### 5.3.2.3.1	Creation of a subscription

The Subscribe Operation is used by a NF Service Consumer (e.g. AMF, V-NSSF) to subscribe to a notification of Network Slice Replacement, Network Slice Instance Replacement and/or of any changes in status of the NSSAI availability information (e.g. S-NSSAIs available per TA and the restricted S-NSSAI(s) per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF.

Figure 5.3.2.3.1-1 Create a subscription

1.	The NF Service Consumer shall send a POST request to create a subscription resource in the NSSF. The payload body of the POST request shall contain a representation of the individual event subscription resource to be created in the NssfEventSubscriptionCreateData.

The request shall indicate the type(s) of events for which the subscription is created, i.e., Network Slice Replacement, Network Slice Instance Replacement and/or of any changes in status of the NSSAI availability information.

The request may contain an expiry time, suggested by the NF Service Consumer as a hint, representing the time upto during which the subscription is desired to be kept active and describes the maximum duration after which the subscribed event shall stop generating report.

The request may also indicate a specific AMF Set to restrict the subscriptions to notifications applicable to the AMF Set (i.e. notifications related to S-NSSAIs supported by the AMF Set). If the AMF Set is provided and the NSSF support the "SATAS" feature (see clause 6.2.8), the NF Service Consumer may also indicate that the subscription is for all TAI(s) associated with the AMF Set.

If the service operation is invoked for subscription to Network Slice Replacement notification, then the request shall contain:

- for VPLMN S-NSSAI: the list of S-NSSAIs in the VPLMN served by the NF Service Consumer that may be replaced with, the NF type of the NF Service Consumer (e.g., AMF) and the NF instance ID of the requester NF.

- for HPLMN S-NSSAI: the list of S-NSSAIs in the HPLMN that the S-NSSAI may be replaced with, the NF type of the NF Service Consumer (e.g., AMF, V-NSSF), the NF instance ID of the requester NF and the HPLMN ID.

If the service operation is invoked for subscription to Network Slice Instance Replacement notification, then the request shall contain:

-	the list of S-NSSAIs and/or the list of NSI IDs that may become congested or no longer available.

2.	On success, "201 Created" shall be returned, and the payload body of the POST response shall contain the representation describing the status of the created subscription in NssfEventSubscriptionCreatedData.

For a subscription to any changes in status of the NSSAI availability information,  the NssfEventSubscriptionCreatedData may contain the AuthorizedNssaiAvailabilityData information, if available.

If there is no supported S-NSSAIs authorized by the NSSF for the TA, the NSSF shall not return the AuthorizedNssaiAvailabilityData for the corresponding TA in the response.

The Location header shall contain the location (URI) of the created subscription resource.

The response, based on operator policy and taking into account the expiry time included in the request, may contain the expiry time, as determined by the NSSF, after which the subscription becomes invalid. Once the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the NSSF. The NSSF shall not provide the same expiry time for many subscriptions in order to avoid all of them expiring and recreating the subscription at the same time. If the expiry time is not included in the response, the NF Service Consumer shall consider the subscription to be valid without an expiry time.

On failure or redirection, the NSSF shall return one of the HTTP status code together with the response body listed in Table 6.2.3.3.3.1-2.

* * * Next Change * * * *

##### 5.3.2.3.2	Modification of a subscription

The Subscribe Operation may be used by a NF Service Consumer (e.g. AMF, V-NSSF) towards an NSSF supporting the SUMOD feature, when it needs to modify an existing subscription previously created by itself.

The NF Service Consumer shall modify the subscription by using HTTP method PATCH with the URI of the individual subscription resource to be modified.

Figure 5.3.2.3.2-1 Modify a subscription

1.	The NF Service Consumer (e.g. AMF, V-NSSF) shall send a PATCH request to the resource URI identifying the individual subscription resource. The payload of the body shall contain the PatchDocument which contains one or more PatchItem instructions for updating the subscription data.

The NF Service Consumer shall not change the event IE included in the NssfEventSubscriptionCreateData by invoking the PATCH request message.

For a subscription to any changes in status of the NSSAI availability information, the taiList IE may only be set to an empty array in PATCH request if the NF service consumer and NSSF support the ONSSAI feature.

2.	On success, "200 OK" shall be returned, the payload body of the PATCH response shall contain the representation describing the updated subscription in NssfEventSubscriptionCreatedData.

For a subscription to any changes in status of the NSSAI availability information, if there is no supported S-NSSAIs authorized by the NSSF for the TA, the NSSF shall not return the AuthorizedNssaiAvailabilityData for the corresponding TA in the response.

On failure or redirection, the NSSF shall return one of the HTTP status code together with the response body listed in Table 6.2.3.4.3.2-2.

* * * Next Change * * * *

##### 5.3.2.4.1	General

The Unsubscribe Operation is used by a NF Service Consumer (e.g. AMF, V-NSSF) to unsubscribe to a notification of any previously subscribed changes to the NSSAI availability information, Network Slice Replacement and/or Network Slice Instance Replacement.

Figure 5.3.2.4.1-1 Unsubscribe a subscription

1.	The NF Service Consumer shall send a DELETE request to delete an existing subscription resource in the NSSF.

2.	If the request is accepted, the NSSF shall respond with the status code 204 indicating the resource identified by subscription ID is successfully deleted.

On failure or redirection, the NSSF shall return one of the HTTP status code together with the response body listed in Table 6.2.3.4.3.1-2.

* * * Next Change * * * *

##### 5.3.2.5.1	General

The Notify Service operation shall be used by the NSSF to update the NF Service Consumer (e.g. AMF) with any change in status, on a per TA basis, of the S-NSSAIs available per TA (unrestricted) and the S-NSSAIs restricted per PLMN in that TA in the serving PLMN of the UE. The service operation is also used to notify the NF Service Consumer (e.g., AMF, V-NSSF) of Network Slice Replacement and/or Network Slice Instance Replacement.

Figure 5.3.2.5.1-1: Update the AMF with any S-NSSAIs restricted per TA

1.	The NSSF shall send a POST request to the resource representing the NSSF availability resource in the NF service consumer (e.g. AMF, V-NSSF). The payload body of the POST request shall contain the one representations of the individual NssfEventNotification resource.

For a subscription to any changes in status of the NSSAI availability information:

-	If there is no supported S-NSSAIs authorized by the NSSF for the TA, the NSSF shall not return the AuthorizedNssaiAvailabilityData for the corresponding TA in the notification.

-	If there is no supported S-NSSAIs authorized by the NSSF for all TAs and the NF Service Consumer has indicated support of "EANAN" feature, the NSSF shall set authorizedNssaiAvailabilityData attribute to an empty array.

For a subscription to Network Slice Instance Replacement:

-	If the NSSF supports the NSIUN feature (see clause 6.2.8) and if the Network Slice instance becomes no longer available, the NSSF shall notify the NF Service Consumer (e.g., AMF, V-NSSF)  having subscribed to this event for the related S-NSSAI and/or NSI ID  that the NSI is no longer available. In that case, the POST request from the NSSF shall contain the list of S-NSSAIs and the associated NSI IDs for which the status is changed (e.g., which become congested or no longer available). The request may also contain congestion mitigation information.

For a subscription to Netwok Slice Replacement:

-	If the NSSF supports the NSRP feature (see clause 6.2.8) and if the NSSF detects that an S-NSSAI becomes unavailable (e.g., based on OAM or NWDAF analytics output), the NSSF shall send  Network Slice Replacement notification for the S-NSSAI to the NF service consumer if the NF service consumer has subscribed to this event for the related S-NSSAI. The notification shall include an alternative S-NSSAI which can be used by the NF service consumer to replace the unavailable S-NSSAI. In case of roaming, the notification shall include:

-	for VPLMN S-NSSAI replacement: the alternative VPLMN S-NSSAI for the S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	for HPLMN S-NSSAI replacement: the alternative HPLMN S-NSSAI for the S-NSSAI and the corresponding mapping to the HPLMN S-NSSAI to be replaced and the HPLMN ID.

-	The NSSF shall notify the NF service consumer when the S-NSSAI becomes available again. The notification shall contain:

-	an indication to stop Network Slice replacement for new UEs; or

-	an indication to terminate Network Slice Replacement for all the UEs and move back the PDU sessions from the alternative S-NSSAI to the S-NSSAI.

If the notification is triggered by the AMF that updates the supported S-NSSAIs per TA by using the Update operation, the NSSF shall not send the notification to the same AMF.

2.	On success, "204 No Content" shall be returned and the payload body of the POST response shall be empty.

On failure or redirection, the NF service consumer shall return one of the HTTP status code together with the response body listed in Table 6.2.5.2.3.1-2.

* * * Next Change * * * *

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nnssf_NSSAIAvailability service based interface protocol.

Table 6.2.6.1-1: Nnssf_NSSAIAvailability specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| NssaiAvailabilityInfo | 6.2.6.2.2 | This contains the Nssai availability information requested by the AMF. |
| SupportedNssaiAvailabilityData | 6.2.6.2.3 | This contains the Nssai availability data information per TA supported by the AMF. |
| AuthorizedNssaiAvailabilityData | 6.2.6.2.4 | This contains the Nssai availability data information per TA authorized by the NSSF |
| RestrictedSnssai | 6.2.6.2.5 | This contains the restricted SNssai information per PLMN. |
| AuthorizedNssaiAvailabilityInfo | 6.2.6.2.6 | This contains the Nssai availability data information authorized by the NSSF |
| PatchDocument | 6.2.6.2.7 | This contains the JSON Patch instructions for updating the Nssai availability data information at the NSSF. |
| NssfEventSubscriptionCreateData | 6.2.6.2.8 | This contains the information for event subscription. |
| NssfEventSubscriptionCreatedData | 6.2.6.2.9 | This contains the information for created event subscription. |
| NssfEventNotification | 6.2.6.2.10 | This contains the notification for created event subscription. |
| SnssaiReplacementSubscribeInfo | 6.2.6.2.x | This contains the input requirements for Network Slice Replacement event subscription. |
| NsiUnavailabilitySubscribeInfo | 6.2.6.2.y | This contains the input requirements for Network Slice Instance Replacement event subscription. |
| NssfEventType | 6.2.6.3.3 | This contains the event for the subscription. |

Table 6.2.6.1-2 specifies data types re-used by the Nnssf service based interface protocol from other specifications.

Table 6.2.6.1-2: Nnssf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [7] | Used to negotiate the applicability of the optional features defined in table 6.2.8-1. |
| Snssai | 3GPP TS 29.571 [7] |  |
| PatchItem | 3GPP TS 29.571 [7] | Identifies the JSON Patch instructions |
| DateTime | 3GPP TS 29.571 [7] |  |
| RedirectResponse | 3GPP TS 29.571 [7] |  |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| ExtSnssai | 3GPP TS 29.571 [7] |  |
| NsagId | 3GPP TS 29.571 [7] | Network Slice AS Group ID |
| SnssaiReplaceInfo | 3GPP TS 29.571 [7] | Alternative S-NSSAI information |
| PlmnId | 3GPP TS 29.571 [7] |  |
| TaiRange | 3GPP TS 29.510 [13] |  |
| NFType | 3GPP TS 29.510 [13] |  |
| NsagInfo | 3GPP TS 29.531 | See clause 6.1.6.2.14 |
| NsiId | 3GPP TS 29.531 | See clause 6.1.6.3.2 |

* * * Next Change * * * *

##### 6.2.6.2.8	Type: NssfEventSubscriptionCreateData

Table 6.2.6.2.8-1: Definition of type NssfEventSubscriptionCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nfNssaiAvailabilityUri | Uri | M | 1 | Identifies the recipient of notifications sent by the NF service consumer (e.g. AMF, V-NSSF) for this subscription |  |
| taiList | array(Tai) | C | 0..N | When present, this IE shall identify the TAIs supported by the NF service consumer (e.g. AMF). This IE shall be present if the NF Service Consumer subscribes to the SNSSAI_STATUS_CHANGE_REPORT event. This IE may be absent if the NSSF supports the NSRP feature or the NSIUN feature and the NF service consumer does not subscribe to the SNSSAI_STATUS_CHANGE_REPORT event. (NOTE) |  |
| event | NssfEventType | M | 1 | Describes the event to be subscribed for this subscription. |  |
| additionalEvents | array(NssfEventType) | C | 1..N | This IE shall be present if the NSSF supports the NSRP feature or the NSIUN feature and the NF Service Consumer wishes to subscribe to more than one event type. When present, this IE shall indicate the additional event(s) requested to be subscribed. |  |
| expiry | DateTime | O | 0..1 | This IE may be included by the NF service consumer. When present, this IE shall represent the suggested time after which the subscription becomes invalid. |  |
| amfSetId | string | O | 0..1 | This IE may be included to identify a specific AMF Set for which this subscription applies. When present, this IE shall be constructed from PLMN-ID (i.e. three decimal digits MCC and two or three decimal digits MNC), AMF Region Id (8 bit), and AMF Set Id (10 bit). Pattern: '^[0-9]{3}-[0-9]{2-3}-[A-Fa-f0-9]{2}-[0-3][A-Fa-f0-9]{2}$' |  |
| taiRangeList | array(TaiRange) | O | 1..N | Identifies a list of TAI ranges supported by the NF service consumer (e.g. AMF). The NF service consumer shall only include this IE when it knows that the NSSF supports the "ONSSAI" feature. (NOTE) | ONSSAI |
| amfId | NfInstanceId | O | 0..1 | This IE may be included to indicate the instance identity of the network function creating the subscription. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.2.8 is supported. |  |
| allAmfSetTaiInd | boolean | O | 0..1 | This IE may be present when amfSetId is present. When present, this IE shall indicate whether the subscription is for all TAIs of the AMF set: - ture: all TAIs of the AMF Set is subscribed. - false (default): indicated TAIs (in the taiList IE and/or taiRangeList IE) are subscribed. | SATAS |
| nsrpSubscribeInfo | SnssaiReplacementSubscribeInfo | C | 0..1 | This IE shall be present when the NF Service Consumer subscribes to the Network Slice Replacement event. |  |
| nsiunSubscribeInfo | NsiUnavailabilitySubscribeInfo | C | 0..1 | This IE shall be present when the NF Service Consumer subscribes to the Network Slice Instance Replacement event. |  |
| NOTE: The taiList attribute shall only be set to an empty array if the NSSF supports the "ONSSAI" feature and taiRangeList IE is present, or if the NSSF supports the "SATAS" feature and the allAmfSetTaiInd IE is present with value true. A NF service consumer (e.g. AMF) may provide both taiRangeList and taiList attributes, to carry individual TAI(s) in the taiList attribute and ranges of TAIs in the taiRangeList attribute. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.2.6.2.9	Type: NssfEventSubscriptionCreatedData

Table 6.2.6.2.9-1: Definition of type NssfEventSubscriptionCreatedData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Identifies the subscription Id for the created subscription. |
| expiry | DateTime | C | 0..1 | This IE shall be included, if, based on operator policy and taking into account the expiry time included in the request, the NSSF needs to include an expiry time. When present, it represents the time after which the subscribed event shall stop generating report and the subscription becomes invalid. Upon reaching this expiry time the NF service consumer shall delete the representation of the subscription it may have. |
| authorizedNssaiAvailabilityData | array(AuthorizedNssaiAvailabilityData) | O | 1..N | This IE may be included if the NF Service Consumer requested to subscribe to changes in the status of NSSAI availability information and if the authorized NSSAI availability (i.e. S-NSSAIs available per TA (unrestricted) and any S-NSSAIs restricted per PLMN in that TA in the serving PLMN of the UE) is available. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.2.8 is supported. |

* * * Next Change * * * *

##### 6.2.6.2.10	Type: NssfEventNotification

Table 6.2.6.2.10-1: Definition of type NssfEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Indicates which subscription generated event notificaiton. This parameter is generated by NSSF and returned in "Location" header in HTTP responses. This can be useful if a NF use a common call-back URI for multiple subscriptions. |  |
| authorizedNssaiAvailabilityData | array(AuthorizedNssaiAvailabilityData) | C | 0..N | This IE shall be present for a notification of changes in the status of the NSSAI availability information. When present, this IE shall contain the authorized NSSAI availability information for all TAs the AMF subscribed to. Each element shall contain the current status of the list of S-NSSAI available in a TA and the list of S-NSSAI restricted per PLMN in that TA. The NF Service Consumer shall replace any authorizedNssaiAvailabilityData received earlier by the new authorizedNssaiAvailabilityData received in the notification. When no supported S-NSSAIs authorized by the NSSF for all TAs, this IE shall contain an empty array indicating Authorized NSSAI Availability information is empty. When received this IE with empty array, the NF Service Consumer shall remove any locally stored authorizedNssaiAvailabilityData previously received from NSSF. (NOTE) |  |
| altNssai | array(SnssaiReplaceInfo) | C | 1..N | The IE shall be present for a notification of Network Slice Replacement. When present, this IE shall indicate the impacted S-NSSAIs if one or more of S-NSSAIs availability status changed from available to not available and vice versa, and the current status for each reported S-NSSAI. This IE may contain the alternative S-NSSAI per impacted S-NSSAI for the S-NSSAIs that are reported as being not available. See clause 5.15.19 in 3GPP TS 23.501 [2]. | NSRP |
| unavailableNsiList | array(NsiId) | C | 1..N | This IE shall be present for a of Network Slice Instance Replacement. When present, this IE shall indicate the NSI IDs for which the status has changed (e.g., that are congested or no longer available). | NSIUN |
| NOTE: NSSF shall only send notificaiton with empty array to NF Service Consumer previously indicated support of "EANAN" feature, when there is no supported S-NSSAIs authorized by the NSSF for all TAs. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.2.6.2.x	Type: SnssaiReplacementSubscribeInfo

Table 6.2.6.2.x-1: Definition of type SnssaiReplacementSubscribeInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| snssaiToSubscribe | array(Snssai) | M | 0..N | This IE shall indicate the S-NSSAIs for which notification is requested during Network Slice Replacement. In the case of roaming it shall indicate: - the VPLMN S-NSSAIs for which notification is requested in case of Network Slice Replacement; or - the HPLMN S-NSSAIs for which notifcation is requested in case of Network Slice Replacement. (NOTE) |
| nfType | NFType | M | 1 | This IE shall contain the NF type of the NF service consumer. |
| nfId | NfInstanceId | M | 1 | This IE shall contain the NF identifier of the NF service consumer. |
| plmnId | PlmnId | C | 0..1 | This IE shall be present in roaming scenarios, if the indicated S-NSSAI is an HPLMN S-NSSAI. It may be present otherwise. When present, it shall indicate the PLMN ID of the S-NSSAI. |
| NOTE: The snnsaiToSubscribe attribute shall be set to an empty array for a Network Slice Replacement subscription applying to all S-NSSAIs. |  |  |  |  |

##### 6.2.6.2.y	Type: NsiUnavailabilitySubscribeInfo

Table 6.2.6.2.y-1: Definition of type NsiUnavailabilitySubscribeInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nsiToSubscribe | array(NsiId) | O | 0..N | When present, this IE shall indicate the identifier of the Network Slice Instance(s) for which notications are requested in case the status of the NSI changes (e.g., becomes congested or no longer available). (NOTE X) (NOTE Y) |
| snssaiToSubscribe | array(Snssai) | O | 0..N | When present, this IE shall indicate the identifier of the S-NSSAI related to the NSI for which notications shall be invoked in case the NSI becomes congested or no longer available.(NOTE W) (NOTE Y) |
| NOTE X: The nsiToSubscribe attribute shall be set to an empty array for a Network Slice Instance Replacement subscription applying to all NSIs. NOTE W: The snnsaiToSubscribe attribute shall be set to an empty array for a Network Slice Instance Replacement subscription applying to all S-NSSAIs. NOTE Y: At least one of the nsiToSubscribe IE or snssaiToSubscribe IE shall be present. |  |  |  |  |

* * * Next Change * * * *

##### 6.2.6.3.3	Enumeration: NssfEventType

Table 6.2.6.3.3-1: Enumeration NssfEventType

| Enumeration value | Description |
| --- | --- |
| "SNSSAI_STATUS_CHANGE_REPORT" | A NF subscribes to this event to receive the status change about the current S-NSSAI available (i.e unrestricted) per TA and the status change about the list of restricted S-NSSAI per TA and per PLMN in the serving PLMN of the UE. |
| "SNSSAI_REPLACEMENT_REPORT" | A NF subscribes to this event to receive a replacement S-NSSAI for each impacted S-NSSAI. See clause 5.15.19 of 3GPP TS 23.501 [2]. |
| "NSI_UNAVAILAIBILITY_REPORT" | A NF subscribes to this event to receive the list of unavailable NSIs (e.g., due to overload). See clause 5.15.20 of 3GPP TS 23.501 [2]. |

* * * Next Change * * * *

### 6.2.8	Feature negotiation

The feature negotiation mechanism specified in clause 6.6 of 3GPP TS 29.500 [4] shall be used to negotiate the optional features applicable between the NSSF and the NF Service Consumer, for the Nnssf_NSSAIAvailability service, if any.

The NF Service Consumer shall indicate the optional features it supports for the Nnssf_NSSAIAvailability service, if any, by including the supportedFeatures attribute in the HTTP PUT request when requesting the NSSF to update the NSSAI Availability information.

The NSSF shall determine the supported features for the updated NSSAI Availability information resource as specified in clause 6.6 of 3GPP TS 29.500 [4] and shall indicate the supported features by including the supportedFeatures attribute in the authorized NSSAI availability information it returns in the HTTP response.

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [7].

The following features are defined for the Nnssf_NSSAIAvailability service.

Table 6.2.8-1: Features of supportedFeatures attribute used by Nnssf_NSSAIAvailability service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | ONSSAI | O | Optimized NSSAI Availability Data encoding When this feature is supported: - NSSAI Availability data may be signalled per list or range(s) of TAIs (see clauses 6.2.6.2.3 and 6.2.6.2.4); and - RestrictedSnssai may encode a list of Home PLMN IDs or may be applicable to all of the Home PLMN IDs (see clause 6.2.6.2.5). - NSSF event subscription may encode a list of TAI ranges (see clause 6.2.6.2.8). |
| 2 | SUMOD | O | Subscription Modification in Subscribe Service Operation When this feature is supported, the subscription of NSSAI availability information is supported to be modified (see clause 5.3.2.3.2). |
| 3 | EANAN | O | Empty Authorized NSSAI Availability Notification A NSSF supporting this feature shall send a notification to NF consumer (as subscriber) with empty array of Authorized NSSAI Availability Data, when no supported NSSAI Authorized by the NSSF for all TAs after latest update and the NF consumer indicated support of this feature. A NF Consumer support this feature shall accept empty array of Authorized NSSAI Availability Data in a notification from NSSF and delete locally stored Authorized NSSAI Availability Data previously received. |
| 4 | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. AMF) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the Nnssf_NSSAIAvailability service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| 5 | SATAS | O | Subscribe ALL TAIs for AMF Set A NSSF supporting this feature shall support the NF Consumer to subscribe to all TAI(s) for an AMF set. |
| 6 | NSIUN | O | Network Slice Instance Unavailability Notification An NF Service Consumer (e.g., AMF, V-NSSF) and NSSF supporting this feature shall support notifications from the NSSF to the NF Service Consumer about the unavailability of Network Slice Instances, as specified in clause 5.15.5.3 of 3GPP TS 23.501 [2]. |
| 7 | NSRP | O | Network Slice Replacement An NF service consumer (e.g., AMF, V-NSSF) that supports this feature shall support network slice replacement as specified in clause 5.15.19 of 3GPP TS 23.501 [2]. |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

* * * Next Change * * * *

# A.3	Nnssf_NSSAIAvailability API

openapi: 3.0.0

info:

version: '1.3.0-alpha.3'

title: 'NSSF NSSAI Availability'

description: |

NSSF NSSAI Availability Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

NssfEventSubscriptionCreateData:

description: This contains the information for event subscription

type: object

required:

- nfNssaiAvailabilityUri

- event

properties:

nfNssaiAvailabilityUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

event:

$ref: '#/components/schemas/ NssfEventType'

additionalEvents:

type: array

items:

$ref: '#/components/schemas/NssfEventType'

minItems: 1

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

amfSetId:

type: string

pattern: '^[0-9]{3}-[0-9]{2-3}-[A-Fa-f0-9]{2}-[0-3][A-Fa-f0-9]{2}$'

taiRangeList:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TaiRange'

minItems: 1

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

allAmfSetTaiInd:

type: boolean

default: false

nsrpSubscribeInfo:

$ref: '#/components/schemas/SnssaiReplacementSubscribeInfo'

nsiunSubscribeInfo:

$ref: '#/components/schemas/NsiUnavailabilitySubscribeInfo'

[…]

NssfEventNotification:

description: This contains the notification for created event subscription

type: object

required:

- subscriptionId

properties:

subscriptionId:

type: string

authorizedNssaiAvailabilityData:

type: array

items:

$ref: '#/components/schemas/AuthorizedNssaiAvailabilityData'

altNssai:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SnssaiReplaceInfo'

minItems: 1

description: >

Indicate the impacted S-NSSAIs, the current status for each reported S-NSSAI, and

if available the alternative S-NSSAI per impacted S-NSSAI for the S-NSSAIs that are

reported as being not available.

unavailableNsiList:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/NsiId'

minItems: 1

SnssaiReplacementSubscribeInfo:

description: Present if the NF service consumer subscribes to events related to Network Slice Replacement.

type: object

properties:

snssaiToSubscribe:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

nfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

required:

- snssaiToSubscribe

- nfType

- nfId

NsiUnavailabilitySubscribeInfo:

description: Present if the NF service consumer subscribes to events related to Network Slice Instance Replacement.

type: object

properties:

nsiToSubscribe:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/NsiId'

snssaiToSubscribe:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

NssfEventType:

description: This contains the event for the subscription

anyOf:

- type: string

enum:

- SNSSAI_STATUS_CHANGE_REPORT

- SNSSAI_REPLACEMENT_REPORT

- NSI_UNAVAILABILITY_REPORT

- type: string

PatchDocument:

description: This contains the JSON Patch instructions for updating the Nssai availability data information at the NSSF

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

* * * End of Changes * * * *