3GPP TSG CT WG3 Meeting #129	C3-233617

# Goteborg, Sweden, August 21 – 25 2023										(revision of C3-233433)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.122 | CR | 0747 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN for Application-Specific Expected UE Behaviour parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 4.15.6.3f Application-Specific Expected UE Behaviour parameters of TS 23.502 v18.2.0 clearly defined Expected PDU session Inactivity Time, Application ID or Packet Filters, no other parameters has been defined, hence the FFS EN for the content of AppExpUeBehaviour data type can be removed. The cardinality of appExpUeBehvs attribute in CpParameterSet data type needs to be changed from 1..N to 0..N to align with the existing parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the FFS EN for the content of AppExpUeBehaviour data type. Change the cardinality of appExpUeBehvs attribute in CpParameterSet data type from 1..N to 0..N. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting stage 2 requirement of AF request for QoS for target UE not identified by a UE address. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.2.2.2, 5.10.2.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Changes ***

##### 5.10.2.2.2	Type: CpParameterSet

This data type represents an offered communication pattern parameter set sent from the SCC/AS to the SCEF.

Table 5.10.2.2.2-1: Definition of type CpParameterSet

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- |
| setId | string | 1 | SCS/AS-chosen correlator provided by the SCS/AS in the request to create a resource fo CP parameter set(s). (NOTE 4) |  |
| self | Link | 0..1 | Link to the resource "Individual CP set Provisioning". This parameter shall be supplied by the SCEF in HTTP responses. |  |
| validityTime | DateTime | 0..1 | Identifies when the CP parameter set expires and shall be deleted. If absent, it indicates that there is no expiration time for this CP parameter set. |  |
| periodicCommunicationIndicator | CommunicationIndicator | 0..1 | Identifies whether UE communicates periodically or on demand. |  |
| communicationDurationTime | DurationSec | 0..1 | Identifies duration time of periodic communication |  |
| periodicTime | DurationSec | 0..1 | Identifies interval time of periodic communication |  |
| scheduledCommunicationTime | ScheduledCommunicationTime | 0..1 | Identifies time zone and day of the week when the UE is available for communication. |  |
| scheduledCommunicationType | ScheduledCommunicationType | 0..1 | Indicates the Scheduled Communication Type. May only be present if the "scheduledCommunicationTime" attribute is provided. | ScheduledCommType_5G |
| stationaryIndication | StationaryIndication | 0..1 | Identifies whether the UE is stationary or mobile. |  |
| batteryInds | array(BatteryIndication) | 0..N | Indicates the power consumption type(s) of the UE. (NOTE 3) |  |
| trafficProfile | TrafficProfile | 0..1 | Identifies the type of data transmission. |  |
| expectedUmts | array(UmtLocationArea5G) | 0..N | Identifies the UE's expected geographical movement. The attribute is only applicable in 5G. (NOTE 2) | ExpectedUMT_5G ExpectedUmtTime_5G |
| expectedUmtDays | DayOfWeek | 0..6 | Identifies the day of the week. If absent, it indicates every day of the week. | ExpectedUmtTime_5G |
| expectedUmtDaysAdd | array(DayOfWeek) | 0..5 | Identifies the additional day(s) of the week. (NOTE 5) | ExpectedUmtTime_Add |
| appExpUeBehvs | array(AppExpUeBehaviour) | 0..N | Contains the Application Specific Expected UE Behaviour parameters. | AppExpUeBehaviour |
| NOTE 1: Properties marked with a feature as defined in clause 5.10.4 are applicable as described in clause 5.2.7. If no features are indicated, the related property applies for all the features. NOTE 2: The first instance of the attribute represents the start of the location, and the last one represents the stop of the location. NOTE 3: If "BATTERY_RECHARGE" is provided, "BATTERY_NO_RECHARGE" shall not be provided simultaneously, vice versa; If "BATTERY_REPLACE" is provided, "BATTERY_NO_REPLACE" shall not be provided simultaneously, vice versa; If "NO_BATTERY" is provided, any value indicating UE powered with battery shall not be provided simultaneously, vice versa. NOTE 4: A setId can only belong to one "Individual CP Provisioning Subscription" resource. NOTE 5: "expectedUmtDaysAdd" attribute may only be provided to indicate the additional day(s) if the "expectedUmtDays" attribute is also provided. |  |  |  |  |

# *** 2nd Changes ***

##### 5.10.2.2.6	Type: AppExpUeBehaviour

Table 5.10.2.2.6-1: Definition of type AppExpUeBehaviour

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appId | string | C | 0..1 | Indicates the Application Identifier. (NOTE) |  |
| expPduSesInacTm | TimeWindow | O | 0..1 | Identifies the expected PDU Session Inactivity time during which the UE will not have traffic related to the application. |  |
| flowDescriptions | array(string) | C | 0..N | Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic. The content of the string has the same encoding as the IPFilterRule AVP value as defined in IETF RFC 6733 [46]. (NOTE) |  |
| NOTE: One of the "appId" attribute or "flowDescriptions" attribute shall be included. |  |  |  |  |  |

# *** End of Changes ***