# 3GPP TSG-CT Meeting #101	CP-232240

# Bangalore, India, 11th – 12th September 2023								(revision of C3-233679)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.525 | CR | 0274 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of the Provisioning of VPLMN specific URSP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LGE, Ericsson, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | LGE, Ericsson, Intel |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-08-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.503 specifies: " When the PCF determines to provision VPLMN specific URSP rules to the UE the PCF provides the URSP rules to be applied in VPLMN in specific Policy Section(s) identified by corresponding PSIs. Such Policy Section(s) do not include any non-VPLMN specific URSP rules. The PCF also provides to the UE the tuple (PLMN ID, list of PSIs of the Policy Sections containing URSP rules to be applied at specific VPLMN). In roaming scenarios, the H-PCF provides this information via V-PCF." To fulfill the requirements above, TS 24.501 specified (agreed in C1-233881) a new IE within the MANAGE UE POLICY COMMAND, the VPS URSP configuration IE, that contains zero or more tuples that consist of a tuple Id, VPLMN identification (network description, in terms e.g. of PLMN ID or MCC, MNC info), and the list of UPSCs (of HPLMN) with UE Policy Sections of URSP rules applicable only to the VPLMN and its equivalent PLMN(s). With this information, the EN below can be solved: Editor's Note: The stage 3 details about the provisioning of VPLMN-Specific URSP rules to the UE within the "MANAGE UE POLICY COMMAND" will be aligned with CT1 agreement, when ready. TS 23.503 also specifies: " VPLMN specific URSP Rules: A VPLMN specific URSP Rule is applicable when the UE is registered in the VPLMN and its equivalent PLMNs. A VPLMN specific URSP Rule may be used to route traffic to the VPLMN, if the V-PCF provides a local DNN to the AMF, using the PCRT on SMF selection management. A VPLMN specific URSP Rule may be used to route traffic to the Home PLMN if the HPLMN operator wants to provide a different list of RSDs for the same Traffic Descriptor for a VPLMN and for the HPLMN. When provided, the Time and Location criteria in the RSD(s) contain values that are based on agreements with the VPLMN or service parameters that were received from the VPLMN." And: " In the case of a roaming UE, the V-PCF may provide guidance on VPLMN specific URSP determination to the H-PCF as defined in clause 4.15.6.10 of TS 23.502 [3]. The H-PCF is required to generate VPLMN specific URSP rule(s) and provide the URSP rules to the UE. This can be triggered by the UE's registration in the VPLMN or it can happen before UE roams into the VPLMN. (…) If the UE does not indicate support for VPLMN specific URSP rules, the H-PCF may trigger an update of the UE's URSP Rules, which may be based on the guidance from the VPLMN, upon receiving a notification that the UE has registered in the VPLMN." and: " (…) - checks whether the requested application guidance are included in HPLMN subscription by checking the LBO information (i.e. DNN and/or S-NSSAI allowed for LBO Session in VPLMN, which is part of SMF Selection Data) if the V-PCF received the LBO information from the AMF. If LBO information is not available at V-PCF in VPLMN, the V-PCF may send the PCRT for reporting the LBO information. " TS 23.502 specifies: "1. UE to (R)AN: AN message ((..)[UE Policy Container (the list of PSIs, indication of UE support for ANDSP, the operating system identifier, Indication of URSP Provisioning Support in EPS, UE capability of reporting URSP rule enforcement to network, UE capability of supporting VPLMN-specific URSP rules)] (..)). NOTE 1: The UE Policy Container and its usage is defined in TS 23.503 [20]." CR 4271 to TS 23.502 completes the flows for the provisioning of VPLMN specific URSP rules in roaming and non-roaming scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update of the Definitions section to include the VPLMN specific URSP definition - Update of the abbreviations to include the VPS term - Editor's Note in clause 4.2.2.2.1.1 is removed. A reference to TS 24.501 D.6.8 is added, to refer to the encoding of VPLMN-specific URSP rules within the VPS URSP Configuration IE. - UE Policy Association establishment and modification procedures are updated to cover the report from the V-PCF to the H-PCF of V-AF guidance on VPLMN-specific URSP rules. - URSP are updated to cover the provisioning of VPLMN-specific URSP rules, as specified by stage 2. - LBO_INFO_CH PCRT is defined to enable the V-PCF requests LBO information changes to the AMF. - VPLMNSpecificURSP rule feature is defined to cover the new impacts in the UE Policy Control service. - Data model is updated accordingly: - to report from the V-PCF to the H-PCF the V-AF guidance on URSP rule, a new attribute, vpsUePolGuidance is defined, that includes, per AF the provided service parameters. - the VPS_UE_POLICY policy control request trigger is defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of VPLMN-specific URSP rules is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 4.2.2.1, 4.2.2.2.1.1, 4.2.2.2.3, 4.2.2.2.3.1, 4.2.2.2.3.2, 4.2.3.1, 4.2.3.2, 4.2.3.3, 5.6.1, 5.6.2.2, 5.6.2.3, 5.6.2.4, 5.6.2.5, 5.6.2.8, 5.6.2.9, 5.6.2.10, 5.6.3.3, 5.8, A.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.502 CR 4271 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR impacts the OpenAPI file with a backwards compatible feature. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C3-233679 agreed in CT3#129 is revised to correct in the OpenAPI file the UePolicyParameters data type. The urspGuidance attribute contained two mistakes in the reference to the corresponding data type: - TS_29522 needs to be corrected to TS29522 - LboRoamingInformation data type needs to be corrected to UrspRuleRequest (see table 5.6.2.9). In addition, the description field needs to remove the "LBO" term. |  |  |  |  |  |  |  |  |

# * * * * Start of Changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[6]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[7]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[8]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[9]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[10]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[11]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[12]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[13]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[14]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[15]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[16]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[17]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Data, Application Data and Structured Data for Exposure; Stage 3".

[18]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[19]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[20]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[21]	IETF RFC 7807: "Problem Details for HTTP APIs".

[22]	3GPP TR 21.900: "Technical Specification Group working methods".

[23]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[24]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[25]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[26]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository service for Subscription Data; Stage 3".

[27]	3GPP TS 29.504:"5G System; Unified Data Repository Services; Stage 3".

[28]	3GPP TS 24.554: "Proximity based services (ProSe) in 5G system (5GS) protocol aspects; Stage 3".

[29]	3GPP TS 24.555: "Proximity based services (ProSe) in 5G system (5GS); User Equipment (UE) policies; Stage 3".

[30]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[31]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[32]	3GPP TS 24.577: "Aircraft-to-Everything (A2X) services in 5G System (5GS) protocol aspects; Stage 3".

[33]	3GPP TS 24.588: "Aircraft-to-Everything (A2X) services in 5G System (5GS); UE policies".

[34]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3"

[35]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[36]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[37]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[38]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[39]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

# * * * * Next change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.503 [4], subclause 3.1 apply:

VPLMN specific URSP rules

# * * * * Next change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G-RG	5G Residential Gateway

5G-VN	5G Virtual Network

A2X	Aircraft-to-Everything

A2XP	Aircraft-to-Everything Policy

AMF	Access and Mobility Management Function

ANDSP	Access Network Discovery and Selection Policy

API	Application Programming Interface

DNN	Data Network Name

EPS	Evolved Packet Core System

FN-RG	Fixed Network Residential Gateway

FN-BRG	Fixed Network Broadband Residential Gateway

FN-CRG	Fixed Network Cable Residential Gateway

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coaxial

HTTP	Hypertext Transfer Protocol

H-PCF	Home Policy Control Function

JSON	JavaScript Object Notation

N3AN	Non-3GPP access network

N3IWF	Non-3GPP InterWorking Function

NID	Network Identifier

NF	Network Function

NRF	Network Repository Function

NSWO	Non-Seamless WLAN Offload

OS	Operating System

OSId	Operating System Identity

PCF	Policy Control Function

PDU	Packet Data Unit

PEI	Permanent Equipment Identifier

PIN	Personal IoT Network

PRA	Presence Reporting Area

ProSeP	5G ProSe Policy

PTI	Procedure Transaction Identity.

RSN	Redundancy Sequence Number

SNPN	Stand-alone Non-Public Network

SUPI	Subscription Permanent Identifier

TNGF	Trusted Non-3GPP Gateway Function

UDR	Unified Data Repository

UPSC	UE policy section code

UPSI	UE policy section identifier

URSP	UE Route Selection Policy

V2X	Vehicle-to-Everything

V2XP	Vehicle-to-Everything Policy

V-PCF	Visited Policy Control Function

VPS	VPLMN Specific

W-5GAN	Wireline 5G Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# * * * * Next change * * * *

#### 4.2.2.1	General

The procedure in the present clause is applicable when the NF service consumer creates a UE policy association in the following cases:

-	UE performs initial registration to the network, as defined in clause 5.5.1.2.2 of 3GPP TS 24.501 [15];

-	UE performs a mobility registration, if the UE operating in single-registration mode performs inter-system change from S1 mode to N1 mode, as defined in clause 5.5.1.3.2 of 3GPP TS 24.501 [15], and there is no existing UE Policy Association between AMF and PCF for this UE; and

-	the AMF is relocated (between the different AMF sets) and the new AMF selects a new PCF. The procedure for the case where the AMF is relocated and the new AMF selects the old PCF is defined in clause 4.2.3.1.

To support the delivery of URSP in EPC, the procedure in the present clause is also applicable when:

-	When the UE triggers a BEARER RESOURCE MODIFICATION REQUEST message with a UE policy container IE after the UE performs ePCO capability negotiation during PDN connection establishment (during the Initial Attach with default PDN connection establishment or the first PDN connection establishment) procedure as defined in 3GPP TS 24.301 [33], and both, the UE and the network support URSP provisioning in EPS PCO; and

-	5GS to EPS handover or 5GS to EPS Idle Mode mobility (both referred as 5GS to EPS mobility in the present document) as defined in 3GPP TS 24.501 [15].

The creation of a UE policy association only applies for normally registered UEs, i.e. it does not apply for emergency-registered UEs.

Figure 4.2.2.1-1 illustrates the procedure used for the creation of a policy association.

Figure 4.2.2.1-1: Creation of a UE policy association

NOTE 1:	For the roaming scenario, the PCF represents the V-PCF, if the NF service consumer is an AMF, and the PCF represents the H-PCF, if the NF service consumer is a V-PCF.

When a UE registers to the network and a UE context is being established, if the AMF obtains from the UE a UE policy delivery protocol message as defined in Annex D of 3GPP TS 24.501 [15] and/or the authorized PC5 capability for 5G ProSe, and/or the authorized PC5 capability for V2X communications and/or A2X communications, the AMF shall establish a UE policy association with the (V-)PCF, in case there is no existing UE policy association for the UE; otherwise, the AMF may establish a UE Policy Association with the (V-)PCF based on AMF local configuration.

NOTE 2:	In the roaming scenario, the visited AMF's local configuration can indicate whether UE Policy delivery is needed based on the roaming agreement with the home PLMN of the UE.

During UE Initial Attach with default PDN connection or the etablishment of the first PDN connection in EPS, if the UE and the SMF+PGW support URSP provisioning in EPS PCO, and the "EpsUrsp" feature is supported between the SMF+PGW-C and the PCF for the PDU session, the PCF for a PDU session associated with the SMF+PGW-C serving the PDN connection obtains from the UE a UE policy container in a Npcf_SMPolicyControl_Update procedure triggered by a bearer resource modification procedure as described in 3GPP TS 29.512 [31]. Then, if the "EpsUrsp" feature described in clause 5.8 is supported, the PCF for a PDU session shall establish a UE policy association with the (V-)PCF for the UE for the delivery of URSP only.

During 5GS to EPS mobility with N26, and if the "EpsUrsp" feature described in clause 5.8 is supported, the PCF for the PDU session determines whether 5GS to EPS mobility applies based on the received RAT and/or Access-Type change event as described in 3GPP TS 29.512 [31]. Then, the PCF for a PDU session shall determine whether the UE supports URSP provisioning in EPS by checking the UE Policy Set information in UDR as specified in 3GPP TS 29.519 [17], and if supported, shall establish a UE policy association with the PCF for the UE that is handling the UE policy association with the source AMF.

NOTE 3:	The PCF for the PDU session discovers the address of the PCF for the UE handling the UE policy association with the source AMF by querying the BSF as described in 3GPP TS 29.521 [22].

Editor's Note: It is FFS how 5GS to EPS mobility with N26 is supported in roaming scenarios.

To establish a UE policy association with the PCF, the NF service consumer (e.g. AMF) shall send an HTTP POST request with "{apiRoot}/npcf-ue-policy-control/v1/policies" as Resource URI and the PolicyAssociationRequest data structure as request body, which shall include:

-	the Notification URI encoded as "notificationUri" attribute;

-	the SUPI encoded as "supi" attribute; and

-	the features supported by the NF service consumer encoded as "suppFeat" attribute,

shall also include, when available:

-	the GPSI encoded as "gpsi" attribute;

-	the Access type encoded as "accessType" attribute;

-	the Permanent Equipment Identifier (PEI) encoded as "pei" attribute;

-	the User Location Information encoded as "userLoc" attribute;

-	the UE Time Zone encoded as "timeZone" attribute;

-	the identifier of the serving network (the PLMN Identifier or the SNPN Identifier), encoded as "servingPlmn" attribute;

NOTE 4:	The SNPN Identifier consists of the PLMN Identifier and the NID.

-	the RAT type encoded as "ratType" attribute;

-	the received UE policy delivery protocol message defined in Annex D of 3GPP TS 24.501 [15] encoded as "uePolReq" attribute;

-	for the roaming scenario, if the NF service consumer is an AMF, the H-PCF ID encoded as "hPcfId" attribute;

-	the Internal Group Identifier(s) encoded as "groupIds" attribute;

-	the PC5 capability for V2X encoded as "pc5Capab" attribute if the "V2X" feature defined in clause 5.8 is supported;

-	the 5G ProSe capability within the "proSeCapab" attribute, if the "ProSe" feature defined in clause 5.8 is supported;

-	if the NF service consumer is an AMF, the GUAMI encoded as "guami" attribute;

-	if the NF service consumer is an AMF, the serving AMF Id encoded as "servingNfId" attribute;

-	if the NF service consumer is an AMF, the "SliceAwareANDSP" feature is supported, and the AMF has determined that the UE has selected a non-3gpp access node (i.e. TNGF or N3IWF) that does not match its subscribed S-NSSAI(s) (or Configured NSSAI, in the roaming case), the wrongly selected non-3gpp access node encoded as "n3gNodeReSel" attribute, and, in the roaming case, also the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute;

-	if the NF service consumer is an AMF, the Satellite Backhaul Category encoded as "satBackhaulCategory" attribute, if the "EnSatBackhaulCategoryChg" feature defined in clause 5.8 is supported

-	for the roaming scenario, if the NF service consumer is an AMF and the "NssaiChange" feature is supported, the configured NSSAI for the serving PLMN encoded as "confSnssais" attribute and optionally the mapped each S-NSSAI value of home network corresponding to the configured S-NSSAI values in the serving PLMN encoded as "mappedHomeSnssai" attribute within the "confSnssais" attribute;

-	the PC5 capability for A2X encoded as "pc5CapA2x" attribute if the "A2X" feature defined in clause 5.8 is supported;

-	for the roaming scenario, if the NF service consumer is a V-PCF and the "VPLMNSpecificURSP" feature is supported, the VPLMN AF guidance on VPLMN-specific URSP rules related information, if applicable, within the "vpsUrspGuidance" attribute; and

-	for the roaming scenario, if the NF service consumer is an AMF, and the "VPLMNSpecificURSP" feature is supported, LBO information within the "lboRoamingInfo" attribute.

and may include:

-	if the NF service consumer is an AMF, the name of a service produced by the AMF that expects to receive information via the Npcf_UEPolicyControl_UpdateNotify service operation encoded as "serviceName" attribute;

-	if the NF service consumer is an AMF, the alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

-	if the NF service consumer is an AMF, the alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

-	if the NF service consumer is an AMF, the alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

-	if the NF service consumer is the PCF for the PDU session, and the "EpsUrsp" feature defined in clause 5.8 is supported, the indication that the trigger for the UE Policy Association Establishment is the 5GS to EPS mobility scenario encoded as the "5gsToEpsMob" attribute.

Upon the reception of the HTTP POST request,

-	the (V-)(H-)PCF shall assign a UE policy association ID;

-	for the roaming scenario and based on operator policy, the V-PCF (as the NF service consumer) should send to the H-PCF a request for the Creation of a UE policy association as described in the present clause;

-	the (V-)(H-)PCF shall determine the applicable UE policy as detailed in clause 4.2.2.2. For the V-PCF, any policy received from the H-PCF in the reply to the possible request for the Creation of a policy association should be taken into consideration;

-	if the (V-)PCF determines that UE policy needs to be provisioned, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall subscribe to the AMF to notifications on N1 messages for UE Policy Delivery Results using the Namf_Communication_N1N2MessageSubscribe service operation;

(ii)	the (V-)PCF shall send the determined UE policy (e.g. ANDSP, URSP, V2XP, A2XP, ProSeP) using Namf_Communication_N1N2MessageTransfer service operation(s); and

(iii) the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the AMF and/or subsequent UE policy requests (e.g. for V2XP and/or A2XP and/or ProSeP) within the Namf_Communication_N1MessageNotify service operation. For the V-PCF, if the received UE Policy Delivery results relate to UE policy sections provided by the H-PCF, the V-PCF shall use the Npcf_UEPolicyControl_Update Service Operation defined in clause 4.2.3 to send those UE Policy Delivery results to the H-PCF;

-	if the UE indicates the support of V2X communications over PC5 reference point and the "V2X" feature is supported, the (H-)PCF shall determine the applicable V2XP, as detailed in clause 4.2.2.2.1.2, and V2X N2 PC5 policy, as detailed in clause 4.2.2.3 and based on the operator's policy;

-	if the UE indicates the support of 5G ProSe and the "ProSe" feature is supported, the (H-)PCF shall determine the applicable ProSeP, as detailed in clause 4.2.2.2.1.3, and 5G ProSe N2 PC5 policy, as detailed in clause 4.2.2.4 and based on the operator's policy;

-	if the PCF determines that N2 PC5 policy (e.g. for V2X communications, for 5G ProSe) needs to be provisioned, including the case of the V-PCF when receiving the N2 PC5 policy from the H-PCF, the PCF shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the N2 PC5 policy according to clause 4.2.2.3 and/or clause 4.2.2.4;

-	if the UE indicates support for URSP provisionng in EPS, the "EpsUrsp" feature is supported, and the (V-)PCF determines that UE policy needs to be provisioned via a PCF for a PDU session, the (V-)PCF shall provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall send a UE policy container with the determined URSP using Npcf_UEPolicyControl_Create response service operation(s); and

(ii) the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the PCF for a PDU session. The PCF for a PDU session shall use the Npcf_UEPolicyControl_Update service operation defined in clause 4.2.3 to send those UE Policy Delivery results to the (V-)PCF;

-	if the UE indicates the support of A2X communications over PC5 reference point and the "A2X" feature is supported, the (H-)PCF shall determine the applicable A2XP, as detailed in clause 4.2.2.2.1.4, and V2X N2 PC5 policy, as detailed in clause 4.2.2.5 and based on the operator's policy;

for the successful case, the (V-)(H-)PCF shall send a HTTP "201 Created" response with the URI for the created resource in the "Location" header field.

NOTE 5:	The assigned policy association ID is part of the URI for the created resource and is thus associated with the SUPI.

and the PolicyAssociation data type as response body, including:

-	mandatorily, the negotiated supported features encoded as "suppFeat" attribute;

-	optionally, the information provided by the NF service consumer when requesting the creation of this policy association encoded as "request" attribute;

-	optionally, for the H-PCF as service producer communicating with the V-PCF, UE policy (see clause 4.2.2.2) encoded as "uePolicy" attribute;

-	optionally, for the H-PCF as service producer communicating with the V-PCF, N2 PC5 policy (see clause 4.2.2.3 and/or clause 4.2.2.4) encoded as "n2Pc5Pol" attribute (for V2X communications) and/or "n2Pc5PolA2x" attribute (for A2X communications) and/or "n2Pc5ProSePol" attribute (for 5G ProSe);

Editor's Note:	It is FFS if both V2X and A2X subscription is available at same time for the UE.

-	optionally, if the UE indicates support for URSP provisionng in EPS and the "EpsUrsp" feature is supported, for the (V-)PCF as service producer communicating with the PCF for a PDU session, UE policy (see clause 4.2.2.2) encoded as "uePolicy" attribute;

-	optionally, for the H-PCF as service producer communicating with the V-PCF, and when the feature "UECapabilityIndication" is supported, if the H-PCF did not receive from the UE information about ANDSP support and the information is available and reliable in the UDR (see clause 4.2.2.2.1.1), the ANDSP support indication retrieved from UDR encoded as "andspInd" attribute;

-	optionally, one or several of the following Policy Control Request Trigger(s) encoded as "triggers" attribute (see clause 4.2.3.2):

a)	Location change (tracking area);

b)	Change of UE presence in PRA;

c)	Change of PLMN, if the "PlmnChange" feature is supported;

d)	Change of UE connectivity state, if the "ConnectivityStateChange" feature is supported; and

e)	LBO information change, applicable to roaming scenarios, if the "VPLMNSpecificURSP" feature is supported and the NF service consumer is an AMF; and

-	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided, the presence reporting areas for which reporting is required encoded as "pras" attribute;

-	if the Policy Control Request Trigger "LBO information change" is provided, optionally, the DNNs(s) and S-NSSAI(s) for which LBO information is required encoded as "pduSessions" attribute;

e)	Change of Configured NSSAI, if the "NssaiChange" feature is supported;

NOTE 6:		If the PCF uses a Presence Reporting Area identifier referring to a Set of Core Network predefined Presence Reporting Areas as defined in 3GPP TS 23.501 [2], the PCF includes the identifier of this Presence Reporting Area set within the "praId" attribute.

-	if the SliceAwareANDSP feature is supported, the PCF received the "n3gNodeReSel" attribute and the PCF has successfully delivered the updated ANDSP/WLANSP to the UE with the slice information for the corresponding non-3gpp node, the notification of this successful delivery by providing the "andspDelInd" attribute with the value "true".

-	if errors occur when processing the HTTP POST request, the (V-)(H-)PCF shall apply error handling procedures as specified in clause 5.7 and according to the following provisions:

-	if the user information received within the "supi" attribute is unknown, the (V-)(H-)PCF shall reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "USER_UNKNOWN"; and

-	if the (V-)(H-)PCF is, due to incomplete, erroneous or missing information in the request, not able to provision a UE policy decision, the (V-)(H-)PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_REQUEST_PARAMETERS".

If the (V-)PCF received a GUAMI, the (V-)PCF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [14], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF (service) set.

# * * * * Next change * * * *

###### 4.2.2.2.1.1	Provisioning of the UE Access Network discovery and selection policies and UE Route Selection Policy

During Initial Registration and 5GS Registration during UE mobility from EPS to 5GS, and when:

a)	t the UE has one or more stored UE policy sections corresponding to the serving PLMN/SNPN or HPLMN; or

b)	the UE does not have any stored UE policy section corresponding to the serving PLMN/SNPN or HPLMN and the UE needs to send a UE policy container to the network;

then the UE includes the "UE STATE INDICATION" message as defined in clause D.5.4.1 of 3GPP TS 24.501 [15], whichis transferred transparently by the AMF within the "uePolReq" attribute during the creation of a policy association, as described in clause 4.2.2.1.

The (H-)PCF, or the PCF of the SNPN for the UEs subscribed to the SNPN, may store in the UDR, as specified in 3GPP TS 29.519 [17]:

a)	UPSCs and related UE policy sections of the own PLMN or SNPN it provided to a UE;

b)	the PEI received from the NF service consumer (e.g. AMF), if available;

c)	the OSId(s) received from the UE within the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available;

d)	the indication of UE's support for ANDSP included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available;

e)	if the "EpsUrsp" feature defined in 3GPP TS 29.519 [17] is supported, the indication of UE's support for URSP provisioning in EPS included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available; and

f)	if the "VPLMNSpecificURSP" feature defined 3GPP TS 29.519 [17] is supported, the indication of UE's support for VPLM-Specific URSP included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available.

The PCF shall retrieve from UDR the information previously stored in UDR, if not locally available, for URSP/ANDSP rule determination as specified in 3GPP TS 29.519 [17].

The V-PCF may retrieve UPSCs and related UE policy sections applicable for all UEs from a HPLMN from the V-UDR, using the HPLMN ID as key as specified in 3GPP TS 29.519 [17]. The PCF of the serving SNPN has locally configured the UPSCs and related UE policy sections applicable for all UEs other than the UEs subscribed to the SNPN.

When receiving the "UE STATE INDICATION" message, the (V-)(H-)PCF or the PCF of the serving SNPN, shall determine, based on the UPSIs indicated in that message, if available, the ANDSP support indication and the OSId(s) indicated in that message, if available, the UE Policy Sections and UPSCs stored in the UDR, if available, the policy subscription data, if available, application data, if available, inputs received from the NF service consumer,and local policy, as specified in clauses 4.2.2.2.2 and 4.2.2.2.3, whether any new UE policy section(s) need to be installed and whether any existing UE policy section(s) need to be updated or deleted.

When the received "UE STATE INDICATION" message indicated that the UE supports VPLMN-specific URSP rules as specified in Annex D of 3GPP TS 24.501 [15], the (H-)PCF may determine URSP rules specific per VPLMN as specified in clauses 4.2.2.2.3.2. In this case, the (H-)PCF shall provide to the UE within the "MANAGE UE POLICY COMMAND" the UE policy sections containing the VPLMN-specific URSP rules within the VPS URSP configuration IE as specified in subclause D.6.8 of 3GPP TS 24.501 [15].

NOTE:	The VPS URSP configuration IE includes zero or more tuples, each tuple containing a tuple Id, VPLMN ID(s) and a list of UPSC(s) (of HPLMN's UE policy sections) with UE policies with URSP rules applicable to the VPLMN(s) and its equivalent PLMN(s).

# * * * * Next change * * * *

##### 4.2.2.2.3	UE Route Selection Policy (URSP)

###### 4.2.2.2.3.1	General

The UE Route Selection Policy is used by the UE to determine how to route outgoing traffic.

The UE Route Selection Policy shall consist of one or several URSP rules. The PCF determines whether URSP rule(s) have to be provisioned based on input parameters received from the NF service consumer, the received list of UPSIs from the UE, if available, the UE Policy Sections stored in the UDR, if available, other received UE parameters, if available, the policy subscription and application data retrieved from UDR, if available, analytics information received from NWDAF, if available, and local policies.

URSP rules are encoded as defined in 3GPP TS 24.526 [16].

UE Route Selection Policy may only be provided by a H-PCF or the PCF of the SNPN, but shall not be provided by a V-PCF. However, UE Route Selection Policy determined and provided by the H-PCF may be retrieved by a V-PCF from the H-PCF and forwarded to a UE.

The (H-)PCF shall use the UE policy subscription data stored in UDR as specified in 3GPP TS 29.519 [17] to ensure the values included in the Route Selection Descriptor of the generated URSP rules are always supported by subscription.

For the received list of internal group Ids, the (H-)PCF retrieves the corresponding 5G VN group configuration data stored from the UDR as specified in 3GPP TS 29.504[27] and 3GPP TS 29.505 [26], if available. For each available 5G VN group, the (H-)PCF may use the retrieved 5G VN group configuration values to encode the values for the Route Selection Descriptor and the values for the Traffic Descriptor of the generated URSP rules.

If the "EnhancedBackgroundDataTransfer" feature is supported, the (H-)PCF may retrieve the Background Data Transfer Reference ID(s) by retrieving the UE's Application Data from the UDR as defined in clause 6.2.9 of 3GPP TS 29.519 [17]. In this case, the PCF shall retrieve the transfer policy corresponding to the Background Data Transfer Reference ID(s) as defined in clause 5.2.8 of 3GPP TS 29.519 [17] and then may create the URSP rules including the Route Selection Validation Criteria for the UE as defined in clause 6.6.2.1 of 3GPP TS 23.503 [4]. If the (H-)PCF provisions the URSP rules including the Route Selection Validation Criteria for the UE, it shall use the associated S-NSSAI and DNN to store in the UDR the Background Data Transfer Reference ID(s) in the UE's session management policy data as specified in 3GPP TS 29.519 [17].

If the (H-)PCF retrieves the BDT policy and corresponding related information (e.g. network area information, the volume of data to be transferred per UE, etc.) within the BdtData data type, and the "bdtpStatus" attribute within the BdtData data type is set to value "INVALID", the (H-)PCF shall not provision the URSP rules based on the invalid BDT policy. When the BDT policy re-negotiation is completed the PCF may:

-	if the new BDT Policy is determined, create or update the applicable URSP rules based on the new BDT policy; or

-	if the invalid BDT policy is removed, remove applicable URSP rules.

If the "AfGuideURSP" feature is supported by the Nudr_DataRepository service, the (H-)PCF may receive Service specific parameter information that contains data for AF guidance information on the URSP determination as defined in clause 6.4.2.15 of 3GPP TS 29.519 [17]. In this case, the (H-)PCF may also use this AF guidance information as input to determine the URSP that will be provisioned to the UE. If the received AF guidance information is not consistent with the UE subscription data, or the local operator policy does not allow the specific S-NSSAI and DNN provided by the AF guidance information, the corresponding AF guidance information shall not be used to determine the URSP rules. The PCF may also determine not to use AF guidance based on the analytics info received from the NWDAF.

When the (H-)PCF decides to provide URSP rules based on the AF guidance information, it shall derive the information as follows:

-	Application traffic descriptor within the "trafficDesc" attribute is used to set the Traffic Descriptor of URSP rule (defined in Figure 5.2.2 of 3GPP TS 24.526 [16]).

-	Each route selection parameter set within the "routeSelParamSets" attribute of the UrspRuleRequest data type is used to determine a Route selection descriptor (defined in Figure 5.2.2 of 3GPP TS 24.526 [16]) as follows:

-	DNN (within the "dnn" attribute of the RouteSelectionParameterSet data type) and S-NSSAI (within the "snssai" attribute of the RouteSelectionParameterSet data type) from the route selection parameter set are used to set the Route selection descriptor contents (defined in Figure 5.2.4 of 3GPP TS 24.526 [16]);

-	Route selection precedence (within the "precedence" attribute of the RouteSelectionParameterSet data type) is used to set the Precedence value of route selection descriptor (defined in Figure 5.2.4 of 3GPP TS 24.526 [16]); and

-	the spatial validity condition (within the "spatialValidityTais" attribute of the RouteSelectionParameterSet data type) is used to set the Location criteria of the route selection descriptor (defined in Figure 5.2.5 of 3GPP TS 24.526 [16]).

-	The precedence for the generated URSP rule is determined by the (H-)PCF. The (H-)PCF may use the "relatPrecedence" attribute within the "UrspRuleRequest" data type to derive the relative precedence of the URSP rule for a request coming from the same AF.

URSP rules based on AF guidance should not be set as the URSP rules with the "match all" application traffic descriptor.

The (H-)PCF may obtain the information about the UE's OS from the UE as described in the Annex D of 3GPP TS 24.501 [15] or it may derive the information about the UE's OS from the PEI provided by the NF service consumer (e.g. AMF).

If the (H-)PCF is required to provide UE policies to the UE that includes application descriptors then:

a)	If the (H-)PCF has been provided with one UE's OS Id by the UE, the (H-)PCF shall use either the traffic descriptor "OS App Id type" or the traffic descriptor "OS Id + OS App Id type" as defined in 3GPP TS 24.526 [16].

NOTE 1:	The (H-)PCF uses the traffic descriptor "OS Id + OS App Id type" when the (H-)PCF does not take the received UE's OS Id into account.

b)	If the (H-)PCF has been provided with more than one UE's OS Id by the UE,

-	the (H-)PCF shall use the traffic descriptor "OS Id + OS App Id type" for the UE's OS Id provided by the UE as defined in 3GPP TS 24.526 [16]; and

-	the (H-)PCF shall not use the traffic descriptor "OS App Id type" as defined in 3GPP TS 24.526 [16].

c)	If the (H-)PCF has not been provided with the UE's OS Id by the UE,

-	the (H-)PCF shall use the traffic descriptor "OS Id + OS App Id type" as defined in 3GPP TS 24.526 [16]; and

-	the (H-)PCF shall not use the traffic descriptor "OS App Id type" as defined in 3GPP TS 24.526 [16].

d)	If the (H-)PCF has been provided with the UE's OS Id by the UE and the (H-)PCF has derived the UE's OS Id from the PEI and if there is an inconsistency between the OS Id provided by the UE and the OS Id derived from the PEI, the (H-)PCF shall use the OS Id provided by the UE for providing UE policies to the UE that include application descriptors.

URSP rules may be used to support end to end redundant user plane paths by establishing two redundant PDU sessions. PCF configuration based on e.g. deployment, terminal implementation or policies per group of UE(s) may be used by the PCF to determine whether the URSP Rules shall include PDU Session Pair ID and RSN to indicate that they refer to redundant PDU sessions or whether the UE will determine these values instead.

If the (H-)PCF has been provided with Personal IoT Network identifier (PIN ID), the (H-)PCF shall define PIN ID within the traffic descriptor of the URSP Rule attribute as defined in 3GPP TS 24.526 [16] for UE to choose an appropriate PIN to establish the PDU session.

NOTE 2:	The PCF can provide two distinct URSP rules to support end to end redundant user plane paths using Dual Connectivity for the duplicated traffic of an application. Duplicated traffic from the UE application is differentiated by two distinct traffic descriptors (different DNNs, and for IP traffic, different IP descriptors or non-IP descriptors), each one defined in a different URSP rule, so that the two redundant PDU sessions are matched to the specific Route Selection Descriptors of distinct URSP rules. These Route Selection Descriptors of distinct URSP rules may include corresponding RSNs and PDU Session Pair IDs as defined in 3GPP TS 24.526 [16]. The Route Selection Descriptors share the same PDU Session Pair ID, if included, to denote the two traffic are redundant with each other.

NOTE 3:	For backward compatibility, PCF can provide a Route Selection Descriptor with PDU Session Pair ID and RSN and a Route Selection Descriptor without PDU Session Pair ID and RSN in the URSP rule. In this case, the Route Selection Descriptor with PDU Session Pair ID and RSN has a lower precedence value (i.e. higher prioritised) than the one without PDU Session Pair ID. It allows that if a non-supporting UE receives the Route Selection Descriptor containing PDU Session Pair ID, it ignores this Route Selection Descriptor.

NOTE 4:	PIN ID and other traffic descriptor components are mutually exclusive, i.e., if PIN ID is included in a URSP rule, then no other traffic descriptor components are supported in the same URSP rule.

The PCF may make policy control decisions based on awareness of URSP rule enforcement for an application by using the following mechanisms:

-	Policy control decisions based on awareness of URSP rule enforcement with UE assistance: If the PCF for a UE and the PCF for a PDU session are different, then the PCF for a UE may obtain UE reporting of URSP rule enforcement from the PCF for a PDU session as defined in 3GPP TS 29.514 [37]. Based on the received URSP rule enforcement information, the PCF may adjust the URSP rules e.g. when the PCF determines that the UE does not have up-to-date URSP rules.

-	Policy control decisions based on awareness of URSP rule enforcement without UE assistance: The PCF may subscribe to statistics for traffic monitoring of known traffic according to provisioned URSP rule(s) at the NWDAF as defined in 3GPP TS 29.520 [38]. If the PCF is notified with traffic which is not expected according to a URSP rule, the PCF may adjust the URSP rules when unexpected application traffic is detected.

Editor’s note: Other details of policy control decision for awareness of URSP rule enforcement is FFS.

Editor’s note: The details that the PCF receives the report of URSP rule enforcement info from NWDAF is FFS.

# * * * * Next change * * * *

###### 4.2.2.2.3.2	Provisioning of VPLMN-specific URSP Rules

When the UE supports VPLMN-specific URSP rules, the H-PCF may provision VPLMN specific URSP rules to the UE for the purpose to route traffic to the VPLMN as described in clause 4.2.2.2.1.1. The H-PCF provides VPLMN specific URSP rules that contains HPLMN values.

NOTE 1:	 For network slice information, the VPLMN-specific URSP rule contains HPLMN NSSAI values. For DNN information, the VPLMN-specific URSP rule contains DNN values according to the subscribed DNNs for which LBO roaming is allowed, as specified in 3GPP TS 29.519 [17].

The (H-)PCF may use AF guidance on URSP determination as input for VPLMN-specific URSP rule determination as specified in clause 4.2.2.2.3.1. The (H-)PCF retrieves from the UDR at the HPLMN the AF guidance for the VPLMN-specific URSP rules for a UE, group of UEs or any UE as specified in 3GPP TS 29.519 [17].

In case of roaming and if the feature "VPLMNSpecificURSP" is supported, the H-PCF may receive from the V-PCF the AF-guidance on VPLMN specific URSP rules within the "vpsUrspGuidance" attribute as specified in clauses 4.2.2.1, and 4.2.3.1. The V-PCF receives from the UDR at the VPLMN the V-AF guidance for the VPLMN specific URSP rules for all roaming UEs of a HPLMN as specified in 3GPP TS 29.519 [17]. The V-PCF determines based on LBO information received from the AMF whether the received V-AF-guidance on VPLMN specific URSP rules may apply for this UE, and if it is so, the V-PCF forwards the related information to the H-PCF within the "vpsUePolGuidance" attribute as specified in clause 4.2.2.1.

For a UE for which AF guidance on VPLMN specific URSP rules is forwarded to the H-PCF within the "vpsUePolGuidance" attribute, the V-PCF maps the S-NSSAI of the VPLMN (indicated by the AF and retrieved from the UDR, if available) into the S-NSSAI of the HPLMN. The V-PCF uses the Configured NSSAI for the Serving PLMN and mapping of each S-NSSAI of the Configured NSSAI to corresponding HPLMN S-NSSAI values provided by the AMF within the "confSnssais" attribute as specified in clauses 4.2.2.1, and 4.2.3.1. The V-PCF shall subscribe to the "CONF_NSSAI_CH" policy control request trigger. Then, for each URSP rule included within the "urspGuidance" attribute, the V-PCF sends the mapped application guidance on URSP determination including the HPLMN S-NSSAI values to the H-PCF within the "snssai" attribute included within the corresponding "routeSelParamSets" entry.

The H-PCF generates new or updated VPLMN-specific URSP rules using the received application guidance on the URSP rule determination, where the VPLMN ID(s) included in the (H-)AF and/or V-PCF request is used to indicate to the UE that this URSP rule applies when the UE is registered in the VPLMN ID. The H-PCF provides URSP rules for the received AF-guidance parameter values that are within the subscribed values defined in the UE Policy Data Set, as specified in 3GPP TS 29.519 [17]. The VPLMN ID(s) received in the (H-)(V-)AF request, as specified in 3GPP TS 29.522[39], and/or received in the V-PCF request, and provided by the H-PCF within the VPLMN-specific URSP rule, as specified in 3GPP TS 24.501 [15], may contain one or more specific values for the MCC and MNC and/or may indicate any MNC for a MCC. The H-PCF, based on operator policies, may set the precedence in the URSP Rules to ensure that the UE checks the VPLMN ID(s) that contain one or more specific values for the MCC and MNC. The H-PCF should also set the precedence in the URSP rules to ensure that the UE checks any VPLMN-specific URSP rule related to the serving PLMN before any non-VPLMN specific URSP rules.

If the UE does not indicate the support for VPLMN specific URSP rules, the H-PCF generates new or updated URSP rules using the VPLMN ID related information retrieved from the UDR and/or received from the V-PCF.

NOTE 2:	To avoid the UE stores obsolete information about VPLMN-specific URSP rules, the H-PCF could delete those determined based on V-AF guidance and once the UE has left the VPLMN.

# * * * * Next change * * * *

#### 4.2.3.1	General

The procedure in the present clause is applicable when the NF service consumer modifies an existing UE policy association (including the case where the AMF is relocated and the new AMF selects to maintain the policy association with the old PCF and to update the Notification URI).

Figure 4.2.3.1-1 illustrates the update of a policy association.

Figure 4.2.3.1-1: Update of a UE policy association

NOTE 1:	For the roaming case, the PCF represents the V-PCF if the NF service consumer is an AMF and the PCF represents the H-PCF if the NF service consumer is a V-PCF.

The AMF, as NF service consumer, invokes this procedure when a subscribed policy control request trigger (see clause 4.2.3.2) occurs. When a policy control request trigger that requires the subscription as defined in table 5.6.3.3-1 (e.g. LOC_CH trigger) occurs, the NF service consumer (AMF) shall only invoke this procedure if the PCF has explicitly subscribed to that event trigger. When a policy control request trigger that does not require the subscription as defined in table 5.6.3.3-1 (e.g. GROUP_ID_LIST_CHG trigger) occurs, the NF service consumer (AMF) shall always invoke the procedure.

NOTE 2:	The AMF uses the Namf_Communication_N1MessageNotify service operation specified in 3GPP TS 29.518 [14] to send to the V-PCF a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15] or a "UE POLICY PROVISIONING REQUEST" message as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24].

If an AMF as NF service consumer knows by implementation specific means that the UE context has been transferred to an AMF with another GUAMI within the AMF set, it may also invoke this procedure to update the Notification URI.

NOTE 3:	Either the old or the new AMF can invoke this procedure.

During the AMF relocation, if the new AMF received the resource URI of the individual UE Policy from the old AMF and selects the old PCF, the new AMF shall also invoke this procedure to update the Notification URI. The new AMF may also update the alternate or backup IP addresses. If the feature "FeatureRenegotation" is supported, the new AMF may perform feature renegotiation, as described in clause 4.2.3.4.

The V-PCF, as NF service consumer, invokes this procedure when a policy control request trigger (see clause 4.2.3.2) occurs. When a policy control request trigger that does not require the subscription as defined in table 5.6.3.3-1 (e.g. UE_POLICY trigger) occurs, the V-PCF shall always invoke the procedure. When a policy control request trigger that requires the subscription as defined in table 5.6.3.3-1 (e.g. LOC_CH trigger) occurs, the V-PCF shall only invoke this procedure if the H-PCF has subscribed to that event trigger.

To request policies (e.g. policy control request trigger(s) is/are met) from the PCF, to update the Notification URI, to renegotiate features, to update the trace control configuration or to request the termination of trace, the NF Service Consumer shall request the update of the associated UE Policy Association by providing the relevant parameters about the UE context in an HTTP POST request with "{apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}/update" as Resource URI and the PolicyAssociationUpdateRequest data structure as request body that shall include:

-	at least one of the following:

1.	a new Notification URI encoded in the "notificationUri" attribute;

2.	observed Policy Control Request Trigger(s) (see clause 4.2.3.2) encoded as "triggers" attribute;

3.	if a UE location change occurred, the UE location encoded as "userLoc" attribute;

4.	if a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message of the UE policy delivery protocol defined in Annex D of 3GPP TS 24.501 [15] has been received by the V-PCF as NF service consumer, and at least parts of the contents relate to UPSIs of the HPLMN, the parts of that message that relate to UPSIs of the HPLMN encoded as "uePolDelResult" attribute;

5.	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided, the current presence status of the UE for the presence reporting areas for which reporting was requested, if not previously provided, or the presence reporting areas for which reporting was requested and the status has changed encoded as "praStatuses" attribute;

NOTE 4:		If the PCF included the identifer of a Core Network predefined Presence Reporting Area Set within the "praId" attribute during the subscription to changes of UE presence in PRA, the AMF only provides the presence reporting area information corresponding to the concerned individual Presence Reporting Area Identifier(s) within the Set. The "praId" attribute within each returned "PresenceInfo" data type hence includes the identifier of the concerned individual Presence Reporting Area.

6.	if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

7.	if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

8.		if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

9.  for AMF relocation scenarios, the GUAMI encoded as "guami" attribute;

NOTE 5:	An alternate NF service consumer than the one that requested the generation of the subscription resource can send the request. For instance, an AMF as service consumer can change.

10.	if the NF service consumer is an AMF, for AMF relocation scenarios, the new serving AMF Id encoded in the "servingNfId" attribute;

11.	if a UE PLMN change occurred and the "PlmnChange" feature defined in clause 5.8 is supported, the PLMN Identifier or the SNPN Identifier of the new serving network encoded as "plmnId" attribute;

NOTE 6:	The SNPN Identifier consists of the PLMN Identifier and the NID.

NOTE 7:	When the UE moves between PLMNs, the trigger reports changes of equivalent PLMNs.

NOTE 8:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

12. if a "UE POLICY PROVISIONING REQUEST" message defined in clause 7.2.1.1 of 3GPP TS 24.587 [24] has been received by the V-PCF as NF service consumer and respectively the "V2X" feature , and/or the "A2X" feature and/or the "ProSe" feature defined in clause 5.8 is/are supported, the message encoded as "uePolReq" attribute;

13.	if a UE Internal Group Identifier(s) change occurred and the "GroupIdListChange" feature defined in clause 5.8 is supported, the Internal Group Identifier(s) of the served UE encoded as "groupIds" attribute; and/or

14.	if a change of PC5 capablity for 5G ProSe occurred and the "ProSe" feature defined in clause 5.8 is supported, the PC5 capability for 5G ProSe encoded as "proSeCapab" attribute.

15.	if a change of the connectivity state of the UE occurred and the "ConnectivityStateChange" feature defined in clause 5.8 is supported, the connectivity state of the served UE encoded as "connectState" attribute; and/or

16.	when a response with HTTP status code 4xx or 5xx as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14] or a N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14] is received by the V-PCF after provisioning the UE policy by invoking the Namf_Communication_N1N2MessageTransfer service operation to the AMF, this UE policy transfer failure notification encoded as "uePolTransFailNotif" attribute;

17.		if the NF service consumer is an AMF, the "SliceAwareANDSP" feature is supported, and the "NON_3GPP_NODE_RESELECTION" trigger is reported within the "triggers" attribute, the wrongly selected non-3gpp access node encoded as "n3gNodeReSel" attribute, and, in the roaming case, also the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute;

18.		if satellite backhaul category change occurred and the "EnSatBackhaulCategoryChg" feature defined in clause 5.8 is supported, the satellite backhaul category or non-satellite backhaul encoded as "satBackhaulCategory" attribute.

19.	for the roaming scenario, if the NF service consumer is an AMF, Configured NSSAI change occurred and the "NssaiChange" feature is supported, the configured NSSAI for the serving PLMN encoded as "confSnssais" attribute and optionally the mapped each S-NSSAI value of home network corresponding to the configured S-NSSAI values in the serving PLMN encoded as "mappedHomeSnssai" attribute within the "confSnssais" attribute;

20.	for the roaming scenario, if the NF service consumer is a V-PCF the "VPLMNSpecificURSP" feature is supported, the VPLMN AF guidance on VPLMN-specific URSP rules related information within the "vpsUePolGuidance" attribute;

21.	for the roaming scenario, if the NF service consumer is an AMF, the "VPLMNSpecificURSP" feature is supported and the "LBO_INFO_CH" policy control request trigger is met, the LBO roaming information within the "lboRoamingInfo" attribute.

Upon the reception of the HTTP POST request:

-	if the PCF is a V-PCF and the V-PCF has an established policy association with the H-PCF, the V-PCF shall determine based on the contents of a potentially received "uePolDelResult" attribute to be sent to the H-PCF (see above) and requested event triggers of the H-PCF whether to send as the NF service consumer towards the H-PCF a request for the update of the policy association as described in the present clause;

-	the (V-)(H-)PCF shall determine the applicable UE policy based on the contents of the received HTTP POST request, the UE Policy Sections stored in UDR, local policy and, for the H-PCF, taking into consideration the information received within the UE policy delivery protocol encoded in the "uePolReq" attribute, if available, and for the V-PCF, taking into consideration any policy received from the H-PCF encoded in the "uePolicy" attribute in the reply to the possible request for the update of the associated policy association. When the "ProSe" feature is supported, the H-PCF shall determine the applicable ProSeP based on the received PC5 capability for 5G ProSe. When the UE disables a 5G ProSe capability the PCF may stop updating the corresponding ProSeP, and when the UE enables a 5G ProSe capability the PCF may update the corresponding ProSeP;

-	if the UE indicated the support of A2X communications over PC5 reference point, "A2X" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable A2XP and A2X N2 PC5 policy as detailed in clauses 4.2.2.2.1.4 and 4.2.2.5, based on the operator's policy;

-	if the UE indicates the support of 5G ProSe communications over PC5 reference point, the "ProSe" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message with the requested 5G ProSe policies was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable ProSeP and 5G ProSe N2 PC5 policy, as detailed in clauses 4.2.2.2.1.3 and 4.2.2.4, based on the operator's policy;

-	if the UE indicated the support of V2X communications over PC5 reference point, "V2X" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable V2XP and V2X N2 PC5 policy as detailed in clauses 4.2.2.2.1.2 and 4.2.2.3, based on the operator's policy;

-	for the succesfull case, the (V-)(H-)PCF shall send a HTTP "200 OK" response with the PolicyUpdate data type as response body with the possibly updated of UE policy (for the H-PCF), and/or ProSe N2 PC5 policy (for the H-PCF) as specified in clause 4.2.2.4, N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe (for the H-PCF) as specified in clause 4.2.2.3 and/or Policy Control Request Trigger(s) encoded as described in clause 4.2.3.3;

-	if the (V-)PCF determines that UE policy needs to be updated, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall send the determined UE policy using Namf_Communication_N1N2MessageTransfer service operation(s); and

(ii)	the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the AMF within the Namf_Communication_N1MessageNotify service operation, and for the V-PCF, if the received UE Policy Delivery results relate to UE policy sections provided by the H-PCF, the V-PCF shall use the Npcf_UEPolicyControl_Update Service Operation to send those UE Policy Delivery results to the H-PCF; and

NOTE 9:	A PolicyUpdate data structure with only mandatory attribute(s) is included in the "200 OK" response when the PCF decides not to update the policies.

-	if the PCF determines that the V2XP and N2 PC5 policy (e.g. for V2X communications, for 5G ProSe) for V2X communications need to be updated, and for the V-PCF when receiving the updated V2XP and N2 PC5 policy for V2X communications from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the V2XP to the UE and the V2X N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.2 and 4.2.2.3;

-	if the PCF determines that the A2XP (e.g. for A2X communications) for A2X communications need to be updated, and for the V-PCF when receiving the updated A2XP and N2 PC5 policy for A2X communications from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the A2XP to the UE and the A2X N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.4 and 4.2.2.5;

-	if the PCF determines that ProSeP and 5G ProSe N2 PC5 policy needs to be updated, and for the V-PCF when receiving the updated ProSeP and 5G ProSe N2 PC5 policy from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the ProSeP to the UE and 5G ProSe N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.3 and 4.2.2.4;

-	if the SliceAwareANDSP feature is supported, the PCF received the "NON_3GPP_NODE_RESELECTION" trigger, and the PCF has successfully delivered the updated ANDSP/WLANSP to the UE with the slice information for the corresponding non-3gpp node, the notification of this successful delivery by providing the "andspDelInd" attribute with the value "true".

-	if errors occur when processing the HTTP POST request, the (V-)(H-)PCF shall:

-	send an HTTP error response as specified in clause 5.7; or

-	if the feature "ES3XX" is supported, and the (V-)(H-)PCF determines the received HTTP POST request needs to be redirected, send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5];

according to the following provisions:

-	if the (V-)(H-)PCF is, due to incomplete, erroneous or missing information in the request not able to provision a UE policy decision, the PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_REQUEST_PARAMETERS".

If the PCF received a new GUAMI, the PCF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [14], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF set.

# * * * * Next change * * * *

#### 4.2.3.2	Policy Control Request Triggers

The following Policy Control Request Triggers are defined (see clause 6.1.2.5 of 3GPP TS 23.503 [4]):

-	"LOC_CH", i.e. location change (tracking area): the tracking area of the UE has changed;

-	"PRA_CH", i.e. change of UE presence in PRA: the UE is entering/leaving a Presence Reporting Area. This includes reporting the initial status at the time the request for this reporting is initiated;

-	"UE_POLICY", i.e. a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15] has been received by the V-PCF and is being forwarded to the H-PCF, or has been received by a PCF for a PDU session (in case for URSP provisioning in EPS) and is being forwarded to the (V-)PCF, or a "UE POLICY PROVISIONING REQUEST" message, as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24] has been received by the V-PCF and is being forwarded to the H-PCF;

-	"PLMN_CH", i.e. PLMN change: the serving network (PLMN or SNPN) of the UE has changed;

NOTE 1:	The "PLMN_CH" trigger only applies if the "PlmnChange" feature is supported.

NOTE 2:	When the UE is moving between PLMNs, the trigger reports changes of equivalent PLMNs.

NOTE 3:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

-	"CON_STATE_CH", i.e. connectivity state change: the connectivity state of the UE has changed;

NOTE 4:	The "CON_STATE_CH" trigger only applies if the "ConnectivityStateChange" feature is supported.

-	"GROUP_ID_LIST_CHG", i.e. UE Internal Group Identifier(s) change: the UDM provided list of group Ids has changed; and

NOTE 5:	The "GROUP_ID_LIST_CHG" trigger only applies if the "GroupIdListChange" feature is supported. This Policy Control Request Trigger does not require an explicit subscription from the PCF.

-	"UE_CAP_CH", i.e. UE Capabilities change: the UE provided 5G ProSe capabilities have changed.

NOTE 6:	The "UE_CAP_CH" trigger only applies if the "ProSe" feature is supported. This Policy Control Request Trigger does not require a subscription.

-	"SAT_CATEGORY_CHG", i.e. Satellite Backhaul Category change: the AMF has detected a change between different satellite category, or non-satellite backhaul.

NOTE 7:	The "SAT_CATEGORY_CHG" trigger only applies if the "EnSatBackhaulCategoryChg" feature is supported.

-	"NON_3GPP_NODE_RESELECTION", i.e. wrong TNGF or N3IWF: the UE has connected to a wrong non-3GPP access node that does not match its subscribed (or configured, in the roaming case) S-NSSAI(s).

NOTE 8:	The "NON_3GPP_NODE_RESELECTION" trigger only applies if the "SliceAwareANDSP" feature is supported. This Policy Control Request Trigger does not require expilict subscription by the PCF.-	"CONF_NSSAI_CH", i.e. Configured NSSAI: the configured NSSAI has changed.NOTE 9:	The "CONF_NSSAI_CH" trigger only applies if the "NssaiChange" feature is supported.

-	"LBO_INFO_CH", i.e. LBO information change: The AMF reports LBO roaming allowed or not allowed for the requested DNN(s) and S-NSSAI(s).

NOTE 10:	The "LBO_INFO_CH" trigger only applies to the AMF and when the "VPLMNSpecificURSP" feature is supported.

# * * * * Next change * * * *

#### 4.2.3.3	Encoding of updated policy

Updated policies shall be encoded within the PolicyUpdate data type that may include:

-	only when the updated policy is supplied by the H-PCF in the roaming scenario, UE policy (see clause 4.2.2.2) encoded as "uePolicy" attribute, and N2 PC5 policy for V2X communications (see clause 4.2.2.3) encoded as "n2Pc5Pol" attribute and/or the N2 PC5 policy for A2X communications (see clause 4.2.2.5) encoded as "n2Pc5PolA2x" attribute and/or the N2 PC5 policy for 5G ProSe (see clause 4.2.2.4) encoded as "n2Pc5ProSePo" attribute;

Editor's Note:	It is FFS if both V2X and A2X subscription is available at same time for the UE.

-	when the updated policy is supplied via PCF of a PDU session by the (V-)PCF in case of URSP provisioning in EPS, UE policy (see clause 4.2.2.2) encoded as "uePolicy" attribute;

-	updated Policy Control Request Trigger(s) (see clause 4.2.3.2) encoded as "triggers" attribute, i.e.:

1)	either a new complete list of applicable Policy Control Request Trigger(s) including one or several of the following:

a)	Location change (tracking area); or

b)	Change of UE presence in PRA;

c)	Change of PLMN, if the "PlmnChange" feature is supported;

d)	Change of UE connectivity state, if the "ConnectivityStateChange" feature is supported;

e)	Change of configured NSSAI, if the "NssaiChange" feature is supported; or

f)	LBO information change, applicable to roaming scenarios, if the "VPLMNSpecificURSP" feature is supported and the NF service consumer is an AMF;

2)	a "NULL" value to request the removal of all previously installed Policy Control Request Trigger(s); and

-	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided or if that trigger was already set but the requested presence reporting areas need to be changed, the presence reporting areas for which reporting is required encoded as "pras" attribute encoded as follows:

a)	A new entry shall be added by supplying a new identifier as key and the corresponding PresenceInfo data type instance with complete contents as value as an entry within the map.

b)	An existing entry shall be modified by supplying the existing identifier as key and the PresenceInfo data type instance with complete contents as value as an entry within the map.

c)	An existing entry shall be deleted by supplying the existing identifier as key and "NULL" as value as an entry within the map.

d)	For an unmodified entry, no entry needs to be provided within the map.

-	if the Policy Control Request Trigger "LBO information change" is provided or if that trigger was already set but the requested LBO information needs to be changed, the requested LBO information encoded in the "pduSessions" attribute, a list of DNN and S-NSSAI combinations previously provided is updated by providing either a new complete list within the"pduSessions" attribute or by setting it to "NULL". If the "pduSessions" attribute is not provided or the previously provided "pduSessions" attribute is deleted, the LBO information change policy control request trigger applies to any S-NSSAI and DNN combination.

# * * * * Next change * * * *

### 5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Npcf_UEPolicyControl service based interface protocol.

Table 5.6.1-1: Npcf_UEPolicyControl specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LboRoamingInformation |  | 5.6.2.10 |  | LBO roaming information for a DNN and S-NSSAI |  | VPLMNSpecificURSP |  |
| Non3gppAccess |  | 5.6.3.7 |  | Represents a Non-3gpp access node. |  | SliceAwareANDSP |  |
| Pc5Capability |  | 5.6.3.5 |  | Indicates the specific PC5 RAT(s) which the UE supports for V2X communications and/or A2X communications over PC5 reference point. |  | V2X, A2X |  |
| ProSeCapability |  | 5.6.3.6 |  | Indicates the 5G ProSe capabilities. |  | ProSe |  |
| PolicyAssociation |  | 5.6.2.2 |  | Description of a policy association that is returned by the PCF when a policy Association is created, updated, or read. |  |  |  |
| PolicyAssociationReleaseCause |  | 5.6.3.4 |  | The cause why the PCF requests the termination of the policy association. |  |  |  |
| PolicyAssociationRequest |  | 5.6.2.3 |  | Information that NF service consumer provides when requesting the creation of a policy association. |  |  |  |
| PolicyAssociationUpdateRequest |  | 5.6.2.4 |  | Information that NF service consumer provides when requesting the update of a policy association. |  |  |  |
| PolicyUpdate |  | 5.6.2.5 |  | Updated policies that the PCF provides in a notification or in the reply to an Update Request. |  |  |  |
| RequestTrigger |  | 5.6.3.3 |  | Enumeration of possible Request Triggers. |  |  |  |
| TerminationNotification |  | 5.6.2.6 |  | Request to terminate a policy Association that the PCF provides in a notification. |  |  |  |
| UeRequestedValueRep |  | 5.6.2.8 |  | Contains the current applicable values corresponding to the policy control request triggers. |  | ImmediateReport |  |
| UePolicy |  | 5.6.3.2 |  | UE Policies |  |  |  |
| UePolicyDeliveryResult |  | 5.6.3.2 |  | UE Policy delivery Result |  |  |  |
| UePolicyParameters |  | 5.6.2.9 |  | Contains the service parameters used to guide the VPLMN-specific URSP rule determination. |  | VPLMNSpecificURSP |  |
| UePolicyRequest |  | 5.6.3.2 |  | Request for UE Policies |  |  |  |
| UePolicyTransferFailureNotification |  | 5.6.2.7 |  | Information that the UE policy is failure to be transferred to the UE because the UE is not reachable. |  |  |  |

Table 5.6.1-2 specifies data types re-used by the Npcf_UEPolicyControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_UEPolicyControl service based interface.

Table 5.6.1-2: Npcf_UEPolicyControl re-used Data Types

| Data type |  | Reference | Comments | Applicability |
| --- | --- | --- | --- | --- |
| AccessType |  | 3GPP TS 29.571 [11] |  |  |
| Bytes |  | 3GPP TS 29.571 [11] | String with format "byte". |  |
| ConfiguredSnssai | 3GPP TS 29.531 [34] | Contains the configured S-NSSAI and optionally mapped home S-NSSA. | SliceAwareANDSP, NssaiChange |
| CmState |  | 3GPP TS 29.518 [14] | Connectivity state of UE | ConnectivityStateChange |
| Fqdn |  | 3GPP TS 29.571 [11] | FQDN |  |
| Gpsi |  | 3GPP TS 29.571 [11] | Generic Public Subscription Identifier |  |
| GroupId |  | 3GPP TS 29.571 [11] |  |  |
| Guami |  | 3GPP TS 29.571 [11] | Globally Unique AMF Identifier |  |
| Ipv4Addr |  | 3GPP TS 29.571 [11] |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [11] |  |  |
| N1N2MessageTransferCause |  | 3GPP TS 29.518 [14] |  |  |
| N2InfoContent |  | 3GPP TS 29.518 [14] | Represents a transparent N2 information content to be relayed by AMF. | V2X, A2X,ProSe |
| NfInstanceId |  | 3GPP TS 29.571 [11] |  |  |
| PduSessionInfo |  | 3GPP TS 29.571 [11] | Contains a DNN and SNSSAI combination | VPLMNSpecificURSP |
| Pei |  | 3GPP TS 29.571 [11] | Permanent Equipment Identifier |  |
| PlmnId |  | 3GPP TS 29.571 [11] |  |  |
| PlmnIdNid |  | 3GPP TS 29.571 [11] | Identifies the network: PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). |  |
| PresenceInfo |  | 3GPP TS 29.571 [11] | Presence reporting area information |  |
| ProblemDetails |  | 3GPP TS 29.571 [11] |  |  |
| RatType |  | 3GPP TS 29.571 [11] |  |  |
| RedirectResponse |  | 3GPP TS 29.571 [11] | Contains redirection related information. | ES3XX |
| ServiceName |  | 3GPP TS 29.510 [13] | Name of the service instance. |  |
| SatelliteBackhaulCategory |  | 3GPP TS 29.571 [11] | Indicates the satellite backhaul category or non-satellite backhaul. | EnSatBackhaulCategoryChg |
| Snssai |  | 3GPP TS 29.571 [11] | Represents an S-NSSAI | SliceAwareANDSP |
| Supi |  | 3GPP TS 29.571 [11] | Subscription Permanent Identifier |  |
| SupportedFeatures |  | 3GPP TS 29.571 [11] | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |
| TimeZone |  | 3GPP TS 29.571 [11] |  |  |
| Uinteger |  | 3GPP TS 29.571 [11] |  |  |
| Uri |  | 3GPP TS 29.571 [11] |  |  |
| UrspRuleRequest |  | 3GPP TS 29.522 [39] | URSP rule guidance information | VPLMNSpecificURSP |
| UserLocation |  | 3GPP TS 29.571 [11] |  |  |

# * * * * Next change * * * *

#### 5.6.2.2	Type PolicyAssociation

Table 5.6.2.2-1: Definition of type PolicyAssociation

| Attribute name | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| request | PolicyAssociationRequest |  | O |  | 0..1 |  | The information provided by the NF service consumer when requesting the creation of a policy association |  |  |  |
| uePolicy | UePolicy |  | O |  | 0..1 |  | The UE policy as determined by the H-PCF (for the H-PCF as NF service producer). |  |  |  |
| n2Pc5Pol | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for V2X communications as determined by the H-PCF. |  | V2X |  |
| n2Pc5PolA2x |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for A2X communications as determined by the H-PCF. |  | A2X |
| n2Pc5ProSePol | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for 5G ProSe as determined by the PCF. |  | ProSe |  |
| triggers | array(RequestTrigger) |  | O |  | 1..N |  | Request Triggers to which the PCF subscribes. Only the values "LOC_CH", "PRA_CH", "PLMN_CH", "LBO_INFO_CH" , and "CON_STATE_CH" are permitted. |  | (NOTE) |  |
| pras | map(PresenceInfo) |  | C |  | 1..N |  | If the Request Trigger "PRA_CH" is provided, the presence reporting area(s) for which reporting is requested shall be provided. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The "presenceState" and the "additionalPraId" attributes within the PresenceInfo data type shall not be supplied. The "praId" attribute within the PresenceInfo data type shall include the identifier of either a presence reporting area or a presence reporting area set. |  |  |  |
| andspDelInd |  | boolean |  | O |  | 0..1 |  | Indication that the updated ANDSP/WLANSP has been successfully delivered to the UE. "true" means that it has been successfully delivered. The default value is "false". |  | SliceAwareANDSP |
| andspInd |  | boolean |  | O |  | 0..1 |  | Indication of UE support of ANDSP. True: The UE supports ANDSP; False: The UE does not support ANDSP. |  | UECapabilityIndication |
| pduSessions |  | array(PduSessionInfo) |  | O |  | 1..N |  | Contains the DNNs and S-NSSAIs for which LBO information is being requested. It may be provided when the "LBO_INFO_CH" request trigger is provided. |  | VPLMNSpecificURSP |
| suppFeat | SupportedFeatures |  | M |  | 1 |  | Indicates the negotiated supported features. |  |  |  |
| NOTE: The "PLMN_CH", "LBO_INFO_CH" and "CON_STATE_CH" values in the "triggers" attribute apply under feature control as described in clause 4.2.3.2. |  |  |  |  |  |  |  |  |  |  |

# * * * * Next change * * * *

#### 5.6.2.3	Type PolicyAssociationRequest

Table 5.6.2.3-1: Definition of type PolicyAssociationRequest

| Attribute name | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| notificationUri | Uri |  | M |  | 1 |  | Identifies the recipient of Notifications sent by the PCF. |  |  |  |
| altNotifIpv4Addrs | array(Ipv4Addr) |  | O |  | 1..N |  | Alternate or backup IPv4 Addess(es) where to send Notifications. |  |  |  |
| altNotifIpv6Addrs | array(Ipv6Addr) |  | O |  | 1..N |  | Alternate or backup IPv6 Addess(es) where to send Notifications. |  |  |  |
| altNotifFqdns | array(Fqdn) |  | O |  | 1..N |  | Alternate or backup FQDN(s) where to send Notifications. |  |  |  |
| supi | Supi |  | M |  | 1 |  | Subscription Permanent Identifier. |  |  |  |
| gpsi | Gpsi |  | C |  | 0..1 |  | Generic Public Subscription Identifier. Shall be provided when available. |  |  |  |
| accessType | AccessType |  | C |  | 0..1 |  | The Access Type where the served UE is camping. Shall be provided when available. |  |  |  |
| pei | Pei |  | C |  | 0..1 |  | The Permanent Equipment Identifier of the served UE. Shall be provided when available. |  |  |  |
| userLoc | UserLocation |  | C |  | 0..1 |  | The location of the served UE. Shall be provided when available. |  |  |  |
| timeZone | TimeZone |  | C |  | 0..1 |  | The time zone where the served UE is camping. Shall be provided when available. |  |  |  |
| servingPlmn | PlmnIdNid |  | C |  | 0..1 |  | The serving network (a PLMN or an SNPN) where the served UE is camping. For the SNPN the NID together with the PLMN ID identifies the SNPN. Shall be provided when available. |  |  |  |
| ratType | RatType |  | C |  | 0..1 |  | The RAT Type where the served UE is camping. Shall be provided when available. |  |  |  |
| groupIds | array(GroupId) |  | C |  | 1..N |  | Internal Group Identifier(s) of the served UE. Shall be provided when available. |  |  |  |
| hPcfId | NfInstanceId |  | C |  | 0..1 |  | H-PCF Identifier. Shall be provided when available. |  |  |  |
| uePolReq | UePolicyRequest |  | C |  | 0..1 |  | A request for UE Policies. Shall be provided when the AMF receives an "UE STATE INDICATION" message, as defined in Annex D.5.4 of 3GPP TS 24.501 [15]. |  |  |  |
| guami | Guami |  | C |  | 0..1 |  | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as NF service consumer. |  |  |  |
| serviceName | ServiceName |  | O |  | 0..1 |  | If the NF service consumer is an AMF, it should provide the name of a service produced by the AMF that makes use of information received within the Npcf_UEPolicyControl_UpdateNotify service operation. |  |  |  |
| servingNfId | NfInstanceId |  | C |  | 0..1 |  | If the NF service consumer is an AMF, it shall contain the identifier of the serving AMF. |  |  |  |
| pc5Capab | Pc5Capability |  | C |  | 0..1 |  | Indicates the PC5 Capability for V2X communications supported by the UE. It shall be provided when available at the NF service consumer. |  | V2X |  |
| pc5CapA2x |  | Pc5Capability |  | C |  | 0..1 |  | Indicates the PC5 Capability for A2X communications supported by the UE. It shall be provided when available at the NF service consumer. |  | A2X |  |  |
| proSeCapab | array(ProSeCapability) |  | C |  | 1..N |  | Indicates whether the UE is capable of one or more of the the following 5G ProSe Capabilities: 5G ProSe Direct Discovery, 5G ProSe Direct Communication, Layer-2 and/or Layer 3 5G ProSe UE-to-Network Relay and Layer-2 and/or Layer 3 5G ProSe Remote UE, and when the "ProSe_Ph2" feature is supported, Layer-2 and/or Layer-3 5G ProSe UE-to-UE Relay and Layer-2 and/or Layer-3 5G ProSe End UE. . It shall be provided when available at the NF service consumer. |  | ProSe |  |
| confSnssais | array(ConfiguredSnssai) |  | C |  | 1..N |  | The Configured NSSAI for the serving PLMN, and optionally the mapped S-NSSAI value of home network corresponding to the configured S-NSSAI in the serving PLMN. . It shall be provided in the roaming case when available at the NF service consumer. It shall be provided in the roaming case when available at the NF service consumer and the "NON_3GPP_NODE_RESELECTION" trigger is reported within the "triggers" attribute. It shall be provided for trigger "CONF_NSSAI_CH". (NOTE) |  | SliceAwareANDSP, NssaiChange |  |
| n3gNodeReSel |  | Non3gppAccess |  | O |  | 0..1 |  | A wrongly selected non-3gpp access node. It shall be provided in the roaming case when available at the NF service consumer, i.e. when the UE has selected a non-3gpp access node that does not match its subscribed or configured NSSAI. |  | SliceAwareANDSP |  |
| satBackhaulCategory | SatelliteBackhaulCategory |  | C |  | 0..1 |  | Indicates the type of the satellite when the AMF is aware that the UE is accessing over a gNB using satellite backhaul. It shall be provided by an AMF as NF service consumer based on configuration. |  | EnSatBackhaulCategoryChg |  |
| 5gsToEpsMob |  | boolean |  | O |  | 0..1 |  | When it is set to true, it indicates the UE Policy Association creation is triggered by a 5GS to EPS mobility scenario. Default value is false. |  | EpsUrsp |  |
| vpsUePolGuidance |  | map(UePolicyParameters) |  | O |  | 1..N |  | Contains the service parameter used to guide the VPLMN-specific URSP. The key of the map represents the AF request to guide the VPLMN-specific URSP rules. This attribute only applies in roaming and when the V-PCF is the NF service consumer. |  | VPLMNSpecificURSP |  |
| lboRoamInfo |  | array(LboRoamingInformation) |  | O |  | 1..N |  | Contains LBO roaming information for DNN and S-NSSAI combination(s). This attribute only applies in roaming and when the AMF is the NF service consumer. |  | VPLMNSpecificURSP |  |
| suppFeat | SupportedFeatures |  | M |  | 1 |  | Indicates the features supported by the service consumer. |  |  |  |
| NOTE: The "mappedHomeSnssai" attribute within the ConfiguredSnssai data type may only be provided if the "NssaiChange" feature is supported. |  |  |  |  |  |  |  |  |  |  |  |

# * * * * Next change * * * *

#### 5.6.2.4	Type PolicyAssociationUpdateRequest

Table 5.6.2.4-1: Definition of type PolicyAssociationUpdateRequest

| Attribute name |  | Data type |  |  | P |  |  | Cardinality |  |  | Description |  |  | Applicability |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| notificationUri |  | Uri |  |  | O |  |  | 0..1 |  |  | Identifies the recipient of Notifications sent by the PCF. |  |  |  |  |  |
| altNotifIpv4Addrs |  | array(Ipv4Addr) |  |  | O |  |  | 1..N |  |  | Alternate or backup IPv4 Address(es) where to send Notifications. |  |  |  |  |  |
| altNotifIpv6Addrs |  | array(Ipv6Addr) |  |  | O |  |  | 1..N |  |  | Alternate or backup IPv6 Address(es) where to send Notifications. |  |  |  |  |  |
| altNotifFqdns |  | array(Fqdn) |  |  | O |  |  | 1..N |  |  | Alternate or backup FQDN(s) where to send Notifications. |  |  |  |  |  |
| triggers |  | array(RequestTrigger) |  |  | C |  |  | 1..N |  |  | Request Triggers that the NF service consumer observes. |  |  |  |  |  |
| praStatuses |  | map(PresenceInfo) |  |  | C |  |  | 1..N |  |  | If the Trigger "PRA_CH" is reported, the UE presence status for tracking area for which changes of the UE presence occurred shall be provided. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The "presenceState" attribute within the PresenceInfo data type shall be supplied. The "additionalPraId" attribute within the PresenceInfo data type shall not be supplied. The "praId" attribute within the PresenceInfo data type shall include the identifier of an individual presence reporting area. |  |  |  |  |  |
| userLoc |  | UserLocation |  |  | C |  |  | 0..1 |  |  | The location of the served UE shall be provided for trigger "LOC_CH". |  |  |  |  |  |
| uePolDelResult |  | UePolicyDeliveryResult |  |  | C |  |  | 0..1 |  |  | UE Policy Delivery Result. Shall be provided together with trigger "UE_POLICY" when a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15], has been received by the V-PCF and is being forwarded to the H-PCF. |  |  |  |  |  |
| uePolTransFailNotif |  | UePolicyTransferFailureNotification |  |  | C |  |  | 0..1 |  |  | The UE policy transfer failure notification. Shall be the provided together with trigger "UE_POLICY" when a response with HTTP status code 4xx or 5xx as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14] or a N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14] is received after the V-PCF provisioned the UE policy by invoking the Namf_Communication_N1N2MessageTransfer service operation to the AMF and is notifying the H-PCF. |  |  |  |  |  |
| uePolReq |  | UePolicyRequest |  |  | C |  |  | 0..1 |  |  | A request for UE Policies. Shall be provided together with trigger "UE_POLICY" when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message, as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24], if the "V2X" feature is supported, and/or when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message for 5G ProSe, as defined in clause 10.4.1 of 3GPP TS 24.554 [28], if the "ProSe" feature is supported and/or when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message for A2X, as defined 3GPP TS 24.577 [32], if the "A2X" feature is supported. |  |  | V2X, A2X, ProSe |  |  |
| guami |  | Guami |  |  | C |  |  | 0..1 |  |  | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as NF service consumer during the AMF relocation. |  |  |  |  |  |
| servingNfId |  | NfInstanceId |  |  | C |  |  | 0..1 |  |  | It shall contain the identifier of the new AMF during the AMF relocation. |  |  |  |  |  |
| plmnId |  | PlmnIdNid |  |  | C |  |  | 0..1 |  |  | The serving network identity (a PLMN or an SNPN) of the served UE shall be provided for trigger "PLMN_CH". |  |  | PlmnChange |  |  |
| connectState |  | CmState |  |  | C |  |  | 0..1 |  |  | The connectivity state of the served UE shall be provided for trigger "CON_STATE_CH". |  |  | ConnectivityStateChange |  |  |
| groupIds |  | array(GroupId) |  |  | C |  |  | 1..N |  |  | Internal Group Identifier(s) of the served UE. Shall be provided for trigger "GROUP_ID_LIST_CHG". |  |  | GroupIdListChange |  |  |
| proSeCapab |  | array(ProSeCapability) |  |  | O |  |  | 1..N |  |  | Indicates whether the UE is capable of one or more of the the following 5G ProSe Capabilities: 5G ProSe Direct Discovery, 5G ProSe Direct Communication, Layer-2 and/or Layer 3 5G ProSe UE-to-Network Relay and Layer-2 and/or Layer 3 5G ProSe Remote UE, and when the "ProSe_Ph2" feature is supported, Layer-2 and/or Layer-3 5G ProSe UE-to-UE Relay and Layer-2 and/or Layer-3 5G ProSe End UE. |  |  | ProSe |  |  |
| confSnssais |  | array(ConfiguredSnssai) |  |  | C |  |  | 1..N |  |  | The Configured NSSAI for the serving PLMN, and optionally the mapped S-NSSAI value of home network corresponding to the configured S-NSSAI in the serving PLMN.It shall be provided for trigger "CONF_NSSAI_CH". (NOTE) |  |  | SliceAwareANDSP, NssaiChange |  |  |
| n3gNodeReSel |  |  | Non3gppAccess |  |  | O |  |  | 0..1 |  |  | A wrongly selected non-3gpp access node. It shall be provided in the roaming case when available at the NF service consumer and the "NON_3GPP_NODE_RESELECTION" trigger is reported within the "triggers" attribute. |  |  | SliceAwareANDSP |  |  |
| satBackhaulCategory |  | SatelliteBackhaulCategory |  |  | C |  |  | 0..1 |  |  | Indicates the type of the satellite when the AMF is aware that the UE is accessing over a gNB using satellite backhaul. It shall be provided for trigger "SAT_CATEGORY_CHG". |  |  | EnSatBackhaulCategoryChg |  |  |
| vpsUePolGuidance |  | map(UePolicyParameters) |  |  | O |  |  | 1..N |  |  | Contains the service parameter used to guide the VPLMN-specific URSP rule determination. The key of the map represents the AF request to guide VPLMN-specific URS rules. This attribute only applies in roaming and when the V-PCF is the NF service consumer. |  |  | VPLMNSpecificURSP |  |  |
| lboRoamInfo |  | array(LboRoamingInformation) |  |  | O |  |  | 1..N |  |  | Contains LBO roaming information for a DNN and S-NSSAI combination(s). This attribute only applies in roaming and when the AMF is the NF service consumer. |  |  | VPLMNSpecificURSP |  |  |
| suppFeat |  | SupportedFeatures |  |  | C |  |  | 0..1 |  |  | Indicates the features supported by the NF service consumer. It shall be included by the target AMF in inter-AMF mobility scenarios for trigger "FEAT_RENEG". |  |  |  |  |  |
| NOTE: The "mappedHomeSnssai" attribute within the ConfiguredSnssai data type may only be provided if the "NssaiChange" feature is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor's Note: The reference to CT1 specification for A2X related UE messages to be updated.

# * * * * Next change * * * *

#### 5.6.2.5	Type PolicyUpdate

Table 5.6.2.5-1: Definition of type PolicyUpdate

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| resourceUri |  | Uri |  | M |  | 1 |  | The resource URI of the individual UE policy association related to the notification. (NOTE 2) |  |  |  |
| uePolicy |  | UePolicy |  | O |  | 0..1 |  | The UE policy as determined by the H-PCF. |  |  |  |
| n2Pc5Pol |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for V2X communications as determined by the H-PCF. |  | V2X |  |
| n2Pc5PolA2x |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for A2X communications as determined by the H-PCF. |  | A2X |  |
| n2Pc5ProSePol |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for 5G ProSe as determined by the PCF. |  | ProSe |  |
| triggers |  | array(RequestTrigger) |  | O |  | 1..N |  | Request Triggers that the PCF subscribes. Only values "LOC_CH", "PRA_CH", "PLMN_CH", "LBO_INFO_CH" and "CON_STATE_CH" are permitted. |  | (NOTE 1) |  |
| pras |  | map(PresenceInfo) |  | C |  | 1..N |  | If the Trigger "PRA_CH" is provided or if that trigger was already set but the requested presence reporting areas need to be changed, the presence reporting area(s) for which reporting is requested shall be provided. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The "presenceState" attribute within the PresenceInfo data type shall not be supplied. The "praId" attribute within the PresenceInfo data type shall include the identifier of either a presence reporting area or a presence reporting area set. |  |  |  |
| andspDelInd |  | boolean |  | O |  | 0..1 |  | Indication that the updated ANDSP/WLANSP has been successfully delivered to the UE. "true" means that it has been successfully delivered. The default value is "false". |  | SliceAwareANDSP |  |
| pduSessions |  | array(PduSessionInfo) |  | O |  | 1..N |  | Contains the list of the DNN and SNSSAI pairs for which LBO information is being requested. It may be provided when the "LBO_INFO_CH" request trigger is provided. |  | VPLMNSpecificURSP |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Indicates the negotiated supported features. It shall be included in the HTTP POST response when the NF service consumer provided the supported features in the HTTP POST request. |  | FeatureRenegotiation |  |
| NOTE 1: The "PLMN_CH", "LBO_INFO_CH" and "CON_STATE_CH" values in the "triggers" attribute apply under feature control as described in clause 4.2.3.2. NOTE 2: When the PolicyUpdate data type is used in a policy update notify service operation, either the complete resource URI included in the "resourceUri" attribute or the "apiSpecificResourceUriPart" component (see clause 5.1) of the resource URI included in the "resourceUri" attribute may be used by the NF service consumer (e.g. AMF) for the identification of the Individual UE Policy Association resource related to the notification. |  |  |  |  |  |  |  |  |  |  |  |

# * * * * Next change * * * *

#### 5.6.2.8	Type UeRequestedValueRep

Table 5.6.2.8-1: Definition of type UeRequestedValueRep

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| userLoc | UserLocation | O | 0..1 | The location of the served UE is camping. |  |
| praStatuses | map(PresenceInfo) | O | 1..N | The UE presence statuses for tracking areas. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. |  |
| plmnId | PlmnIdNid | C | 0..1 | The serving network identity (a PLMN or an SNPN) of the served UE shall be provided for trigger "PLMN_CH". | PlmnChange |
| connectState | CmState | O | 0..1 | The connectivity state of the served UE. | ConnectivityStateChange |
| lboRoamInfo | array(LboRoamingInformation) | C | 1..N | Contains a list of LBO roaming information for a DNN and S-NSSAI combination. It shall be provided for trigger "LBO_INFO_CH". | VPLMNSpecificURSP |

# * * * * Next change * * * *

#### 5.6.2.9	Type UePolicyParameters

Table 5.6.2.9-1: Definition of type UePolicyParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| urspGuidance | array(UrspRuleRequest) | O | 1..N | Contains the service parameter used to guide the URSP. |  |

# * * * * Next change * * * *

#### 5.6.2.10	Type LboRoamingInformation

Table 5.6.2.10-1: Definition of type LboRoamingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lboRoamAllowed | boolean | O | 0..1 | Indicates whether local breakout for the DNN and S-NSSAI is allowed when roaming. true: allowed false: not allowed. If the attribute is absent it means not allowed. |  |
| dnn | Dnn | M | 1 | Data Network Name with Network Identifier only. |  |
| snssai | Snssai | M | 1 | S-NSSAI. |  |

# * * * * Next change * * * *

#### 5.6.3.3	Enumeration: RequestTrigger

The enumeration RequestTrigger represents the possible Policy Control Request Triggers.. It shall comply with the provisions defined in table 5.6.3.3-1.

Table 5.6.3.3-1: Enumeration RequestTrigger

| Enumeration value | Description |  |  |  | Applicability |
| --- | --- | --- | --- | --- | --- |
| LOC_CH | Location change (tracking area): the tracking area of the UE has changed. (NOTE) |  |  |  |  |
| PRA_CH | Change of UE presence in PRA: the AMF reports the current presence status of the UE in a Presence Reporting Area, and notifies that the UE enters/leaves the Presence Reporting Area. (NOTE) |  |  |  |  |
| UE_POLICY | A "MANAGE UE POLICY COMPLETE" message, a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15] has been received by the V-PCF and is being forwarded to the H-PCF, or has been received by a PCF for a PDU session and is being forwarded to the (V-)PCF (and then from the V-PCF to the H-PCF) when the "EpsUrsp" feature is supported. A Namf_Communication_N1N2MessageTransfer failure response as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14], an N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14], a UE Policy transfer failure is notifying to the H-PCF, or a UE Policy transfer failure is notifying to the (V-)PCF when the "EpsUrsp" feature is supported. When the "ProSe" feature is supported it indicates that a "UE POLICY PROVISIONING REQUEST" message, as defined in clause 10.4 of 3GPP TS 24.554 [28] has been received by the V-PCF and is being forwarded to the H-PCF. When the "V2X" feature is supported it indicates that a "UE POLICY PROVISIONING REQUEST" message, as defined in clause 7.2 of 3GPP TS 24.587 [24] has been received by the V-PCF and is being forwarded to the H-PCF. When the "A2X" feature is supported it indicates that a "UE POLICY PROVISIONING REQUEST" message, as defined in 3GPP TS 24.577 [32] has been received by the V-PCF and is being forwarded to the H-PCF. This event does not require a subscription and is only applicable for the V-PCF as NF service consumer and the H-PCF as NF service producer or a PCF for a PDU session as NF service consumer and the (V-)PCF as NF service producer when the “EpsUrsp” feature is supported. |  |  |  |  |
| PLMN_CH | PLMN change: the serving network (a PLMN or an SNPN) of UE has changed. (NOTE) |  |  |  | PlmnChange |
| CON_STATE_CH | Connectivity state change: the connectivity state of UE has changed. (NOTE) |  |  |  | ConnectivityStateChange |
| GROUP_ID_LIST_CHG | UE Internal Group Identifier(s) has changed: the AMF reports that UDM provided list of group Ids has changed. This policy control request trigger does not require a subscription. |  |  |  | GroupIdListChange |
| UE_CAP_CH |  |  | UE Capabilities change: the UE provided 5G ProSe capabilities have changed. This policy control request trigger does not require subscription. |  | ProSe |  |  |
| SAT_CATEGORY_CHG |  |  |  |  | Satellite Backhaul Category change: the AMF has detected a change between different satellite backhaul category, or non-satellite backhaul. | EnSatBackhaulCategoryChg |  |
| CONF_NSSAI_CH |  |  |  |  | Configured NSSAI change: the configured NSSAI has changed. | NssaiChange |  |
| NON_3GPP_NODE_RESELECTION |  |  |  |  | Wrong TNGF or N3IWF: the UE has connected to a wrong non-3GPP access node that does not match its subscribed S-NSSAI(s). This policy control request trigger does not require a subscription. | SliceAwareANDSP |  |
| FEAT_RENEG |  |  |  |  | The target AMF determines feature re-negotiation is required. This policy control request trigger does not require subscription. | FeatureRenegotiation |  |
| LBO_INFO_CH |  |  |  |  | LBO information change. The AMF reports LBO roaming allowed or not allowed for the requested DNN(s) and S-NSSAI(s). This policy control request trigger only applies in roaming scenarios when the NF service consumer is the AMF. (NOTE) | VPLMNSpecificURSP |  |
| NOTE: The report of this trigger includes reporting the current value at the time the trigger is provisioned during the update or update notification of the policy association. |  |  |  |  |  |

Editor's Note: The reference to CT1 specification for A2X related UE messages to be updated.

# * * * * Next change * * * *

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Npcf_UEPolicyControl API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number |  |  | Feature Name | Description |
| --- | --- | --- | --- | --- |
| 1 |  |  | PendingTransaction | This feature indicates support for the race condition handling as defined in 3GPP TS 29.513 [7]. |
| 2 |  |  | PlmnChange | This feature indicates support for the change of PLMN trigger handling. |
| 3 |  |  | ConnectivityStateChange | This feature indicates support for the UE connectivity state change trigger handling. |
| 4 | V2X | This feature indicates support for the UE policy provisioning and N2 information provisioning for V2X communications. |  |
| 5 | GroupIdListChange | This feature indicates the support for the notification of changes in the list of internal group identifiers. |  |
| 6 | ImmediateReport | This feature indicates the support of the current applicable values report corresponding to the policy control request triggers for policy update notification. |  |
| 7 | ErrorResponse | This feature indicates support for "404 Not Found" error response code for policy update notification between AMF and (V-)PCF. |  |
| 8 | ES3XX | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [5]. |  |
| 9 | ProSe | This feature indicates support of UE policy and N2 information provisioning for 5G ProSe. |  |
| 10 |  | FeatureRenegotiation | This feature indicates the support of feature renegotiation during the update of a policy association triggered by UE mobility with AMF change. |
| 11 |  | SliceAwareANDSP | This feature indicates the support of ANDSP/WLANSP policies that consider the slices supported by the UE. |
| 12 |  | EpsUrsp | This feature indicates support of URSP provisioning in EPS and is only applicable in the case of of 5GC and EPC interworking. |
| 13 |  | EnSatBackhaulCategoryChg | This feature indicates the support of notification of a change between different satellite backhaul categories, or dynamic satellite backhaul categories, or between satellite backhaul and non-satellite backhaul. |
| 14 |  | UECapabilityIndication | This feature indicates the support of the provisioning by the H-PCF to the V-PCF of the UE Capability for UE Policy, when the UE Capability is not received from the UE and the information is available and reliable in the UDR. |
| 15 |  | A2X | This feature indicates support of A2X communications. |
| 16 |  | NssaiChange | This feature indicates support for the change of Configured NSSAI trigger handling. |
| 17 |  | ProSe_Ph2 | This feature indicates the support of UE policy and N2 information provisioning for 5G ProSe UE-to-UE Relay function. This feature requires that the ProSe feature is also supported. |
| 18 | VPLMNSpecificURSP | This feature indicates the support of AF guidance on VPLMN-specific URSP rules. It requires the support of NssaiChange feature. |

# * * * * Next change * * * *

# A.2	Npcf_UEPolicyControl API

openapi: 3.0.0

info:

version: 1.3.0-alpha.3

title: Npcf_UEPolicyControl

description: |

UE Policy Control Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.525 V18.2.0; 5G System; UE Policy Control Service.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.525/'

servers:

- url: '{apiRoot}/npcf-ue-policy-control/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- npcf-ue-policy-control

paths:

/policies:

post:

operationId: CreateIndividualUEPolicyAssociation

summary: Create individual UE policy association.

tags:

- UE Policy Associations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociationRequest'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociation'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

policyUpdateNotification:

'{$request.body#/notificationUri}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyUpdate'

responses:

'200':

description: >

OK. The current applicable values corresponding to the policy control request

trigger is reported

content:

application/json:

schema:

$ref: '#/components/schemas/UeRequestedValueRep'

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

policyAssocitionTerminationRequestNotification:

'{$request.body#/notificationUri}/terminate':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TerminationNotification'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policies/{polAssoId}:

get:

operationId: ReadIndividualUEPolicyAssociation

summary: Read individual UE policy association.

tags:

- Individual UE Policy Association (Document)

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

operationId: DeleteIndividualUEPolicyAssociation

summary: Delete individual UE policy association.

tags:

- Individual UE Policy Association (Document)

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policies/{polAssoId}/update:

post:

operationId: ReportObservedEventTriggersForIndividualUEPolicyAssociation

summary: >

Report observed event triggers and possibly obtain updated policies for an individual UE

policy association.

tags:

- Individual UE Policy Association (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociationUpdateRequest'

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'200':

description: OK. Updated policies are returned

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyUpdate'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npcf-ue-policy-control: Access to the Npcf_UEPolicyControl API

schemas:

PolicyAssociation:

description: >

Contains the description of a policy association that is returned by the PCF when a policy

Association is created, updated, or read.

type: object

properties:

request:

$ref: '#/components/schemas/PolicyAssociationRequest'

uePolicy:

$ref: '#/components/schemas/UePolicy'

n2Pc5Pol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5PolA2x:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5ProSePol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

description: >

Request Triggers that the PCF subscribes.

pras:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: >

Contains the presence reporting area(s) for which reporting was requested.

The praId attribute within the PresenceInfo data type is the key of the map.

andspDelInd:

type: boolean

description: >

Indication that the updated ANDSP/WLANSP has been successfully delivered to the UE.

andspInd:

description: >

Indication of UE support of ANDSP. When set to true, it indicates the UE supports ANDSP,

when set to false it indicates the UE does not support ANDSP.

type: boolean

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pduSessions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

description: Combination of DNN and S-NSSAIs for which LBO information is requested.

required:

- suppFeat

PolicyAssociationRequest:

description: >

Represents information that the NF service consumer provides when requesting the creation of

a policy association.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altNotifIpv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

description: Alternate or backup IPv4 Address(es) where to send Notifications.

altNotifIpv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

description: Alternate or backup IPv6 Address(es) where to send Notifications.

altNotifFqdns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

minItems: 1

description: Alternate or backup FQDN(s) where to send Notifications.

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

servingPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

groupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

hPcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

uePolReq:

$ref: '#/components/schemas/UePolicyRequest'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pc5Capab:

$ref: '#/components/schemas/Pc5Capability'

pc5CapA2x:

$ref: '#/components/schemas/Pc5Capability'

proSeCapab:

type: array

items:

$ref: '#/components/schemas/ProSeCapability'

minItems: 1

confSnssais:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/ConfiguredSnssai'

minItems: 1

n3gNodeReSel:

$ref: '#/components/schemas/Non3gppAccess'

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

5gsToEpsMob:

type: boolean

description: >

It indicates the UE Policy Association is triggered by a 5GS to EPS mobility

scenario.

vpsUePolGuidance:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicyParameters'

minProperties: 1

description: >

Contains the service parameter used to guide the VPLMN-specific URSP.

The key of the map represents the AF request to guide VPLMN-specific URS rules.

This attribute only applies in roaming and when the V-PCF is the NF service consumer.

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

This attribute only applies in roaming and when the AMF is the NF service consumer.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notificationUri

- suppFeat

- supi

PolicyAssociationUpdateRequest:

description: >

Represents Information that the NF service consumer provides when requesting the update of

a policy association.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altNotifIpv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

description: Alternate or backup IPv4 Address(es) where to send Notifications.

altNotifIpv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

description: Alternate or backup IPv6 Address(es) where to send Notifications.

altNotifFqdns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

minItems: 1

description: Alternate or backup FQDN(s) where to send Notifications.

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

description: Request Triggers that the NF service consumer observes.

praStatuses:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

description: >

Contains the UE presence status for tracking area for which changes of the UE presence

occurred. The praId attribute within the PresenceInfo data type is the key of the map.

minProperties: 1

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uePolDelResult:

$ref: '#/components/schemas/UePolicyDeliveryResult'

uePolTransFailNotif:

$ref: '#/components/schemas/UePolicyTransferFailureNotification'

uePolReq:

$ref: '#/components/schemas/UePolicyRequest'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

connectState:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CmState'

groupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

proSeCapab:

type: array

items:

$ref: '#/components/schemas/ProSeCapability'

minItems: 1

confSnssais:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/ConfiguredSnssai'

minItems: 1

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

vpsUePolGuidance:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicyParameters'

minProperties: 1

description: >

Contains the service parameter used to guide the VPLMN-specific URSP.

The key of the map represents the AF request to guide VPLMN-specific URS rules.

This attribute only applies in roaming and when the V-PCF is the NF service consumer.

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

This attribute only applies in roaming and when the AMF is the NF service consumer.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

PolicyUpdate:

description: >

Represents updated policies that the PCF provides in a notification or in the reply to an

Update Request.

type: object

properties:

resourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

uePolicy:

$ref: '#/components/schemas/UePolicy'

n2Pc5Pol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5PolA2x:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5ProSePol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

nullable: true

description: >

Request Triggers that the PCF subscribes.

pras:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

description: >

Contains the presence reporting area(s) for which reporting was requested.

The praId attribute within the PresenceInfo data type is the key of the map.

minProperties: 1

nullable: true

andspDelInd:

type: boolean

description: >

Indication that the updated ANDSP/WLANSP has been successfully delivered to the UE.

pduSessions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

description: >

Combination of DNN and S-NSSAIs for which LBO information is requested.

nullable: true

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- resourceUri

TerminationNotification:

description: >

Represents a request to terminate a policy association that the PCF provides in a

notification.

type: object

properties:

resourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

cause:

$ref: '#/components/schemas/PolicyAssociationReleaseCause'

required:

- resourceUri

- cause

UePolicyTransferFailureNotification:

description: >

Represents information on the failure of a UE policy transfer to the UE because the UE is not

reachable.

type: object

properties:

cause:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N1N2MessageTransferCause'

ptis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

required:

- cause

- ptis

UeRequestedValueRep:

description: >

Contains the current applicable values corresponding to the policy control request triggers.

type: object

properties:

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

praStatuses:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: >

Contains the UE presence statuses for tracking areas. The praId attribute within the

PresenceInfo data type is the key of the map.

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

connectState:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CmState'

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

UePolicyParameters:

description: >

Contains the service parameters used to guide the VPLMN-specific URSP rule determination.

type: object

properties:

urspGuidance:

type: array

items:

$ref: 'TS29522_ServiceParameter.yaml#/components/schemas/UrspRuleRequest'

minItems: 1

description: Contains the service parameter used to guide the URSP.

LboRoamingInformation:

description: >

Contains LBO roaming information for a DNN and S-NSSAI.

type: object

properties:

lboRoamAllowed:

type: boolean

description: >

Indicates whether LBO for the DNN and S-NSSAI is allowed when roaming.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- dnn

- snssai

UePolicy:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

UePolicyDeliveryResult:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

UePolicyRequest:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

RequestTrigger:

anyOf:

- type: string

enum:

- LOC_CH

- PRA_CH

- UE_POLICY

- PLMN_CH

- CON_STATE_CH

- GROUP_ID_LIST_CHG

- UE_CAP_CH

- SAT_CATEGORY_CHG

- NON_3GPP_NODE_RESELECTION

- CONF_NSSAI_CH

- LBO_INFO_CH

- FEAT_RENEG

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the possible request triggers.

Possible values are:

- LOC_CH: Location change (tracking area). The tracking area of the UE has changed.

- PRA_CH: Change of UE presence in PRA. The AMF reports the current presence status of the UE

in a Presence Reporting Area, and notifies that the UE enters/leaves the Presence Reporting

Area.

- UE_POLICY: A MANAGE UE POLICY COMPLETE message or a MANAGE UE POLICY COMMAND REJECT

message, as defined in Annex D.5 of 3GPP TS 24.501 or a "UE POLICY PROVISIONING REQUEST"

message, as defined in clause 7.2.1.1 of 3GPP TS 24.587, has been received by the AMF

and is being forwarded.

- PLMN_CH: PLMN change. the serving PLMN of UE has changed.

- CON_STATE_CH: Connectivity state change: the connectivity state of UE has changed.

- GROUP_ID_LIST_CHG: UE Internal Group Identifier(s) has changed. This policy control request

trigger does not require a subscription.

- UE_CAP_CH: UE Capabilities change: the UE provided 5G ProSe capabilities have changed.

This policy control request trigger does not require subscription.

- SAT_CATEGORY_CHG: Indicates that the AMF has detected a change between different satellite

category, or non-satellite backhaul.

- NON_3GPP_NODE_RESELECTION: The UE has connected to a wrong non-3GPP access node that

does not match its subscribed S-NSSAI(s). This policy control request trigger does not

require a subscription.

- CONF_NSSAI_CH: Configured NSSAI change. Indicates that the configured NSSAI has changed.

- LBO_INFO_CH: LBO information change. The AMF reports LBO roaming allowed or not allowed

for the requested DNN(s) and S-NSSAI(s). This policy control request trigger only applies

in roaming scenarios when the NF service consumer is the AMF.

- FEAT_RENEG: The NF service consumer notifies that the target AMF is requesting feature

re-negotiation.

PolicyAssociationReleaseCause:

anyOf:

- type: string

enum:

- UNSPECIFIED

- UE_SUBSCRIPTION

- INSUFFICIENT_RES

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the cause why the PCF requests the policy association termination.

Possible values are:

- UNSPECIFIED: This value is used for unspecified reasons.

- UE_SUBSCRIPTION: This value is used to indicate that the policy association needs to be

terminated because the subscription of UE has changed (e.g. was removed).

- INSUFFICIENT_RES: This value is used to indicate that the server is overloaded and needs

to abort the policy association.

Pc5Capability:

anyOf:

- type: string

enum:

- LTE_PC5

- NR_PC5

- LTE_NR_PC5

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the specific PC5 RAT(s) which the UE supports for V2X or A2X communications over

PC5 reference point.

Possible values are:

- LTE_PC5: This value is used to indicate that UE supports PC5 LTE RAT for V2X

communications or A2X communications over the PC5 reference point

over the PC5 reference point.

- NR_PC5: This value is used to indicate that UE supports PC5 NR RAT for V2X communications

or A2X communications over the PC5 reference point.

- LTE_NR_PC5: This value is used to indicate that UE supports both PC5 LTE and NR RAT for

V2X communications or A2X communications over the PC5 reference point.

ProSeCapability:

anyOf:

- type: string

enum:

- PROSE_DD

- PROSE_DC

- PROSE_L2_U2N_RELAY

- PROSE_L3_U2N_RELAY

- PROSE_L2_REMOTE_UE

- PROSE_L3_REMOTE_UE

- PROSE_L2_U2U_RELAY

- PROSE_L3_U2U_RELAY

- PROSE_L2_END_UE

- PROSE_L3_END_UE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

the content defined in the present version of this API.

description: |

Represents the 5G ProSe capabilities.

Possible values are:

- PROSE_DD: This value is used to indicate that 5G ProSe Direct Discovery is supported

by the UE.

- PROSE_DC: This value is used to indicate that 5G ProSe Direct Communication is supported

by the UE.

- PROSE_L2_U2N_RELAY: This value is used to indicate that Layer-2 5G ProSe UE-to-Network

Relay is supported by the UE.

- PROSE_L3_U2N_RELAY: This value is used to indicate that Layer-3 5G ProSe UE-to-Network

Relay is supported by the UE.

- PROSE_L2_REMOTE_UE: This value is used to indicate that Layer-2 5G ProSe Remote UE is

supported by the UE.

- PROSE_L3_REMOTE_UE: This value is used to indicate that Layer-3 5G ProSe Remote UE is

supported by the UE.

- PROSE_L2_U2U_RELAY: This value is used to indicate that Layer-2 5G ProSe UE-to-UE

Relay is supported by the UE.

- PROSE_L3_U2U_RELAY: This value is used to indicate that Layer-3 5G ProSe UE-to-UE

Relay is supported by the UE.

- PROSE_L2_END_UE: This value is used to indicate that Layer-2 5G ProSe End UE is

supported by the UE.

- PROSE_L3_END_UE: This value is used to indicate that Layer-3 5G ProSe End UE is

supported by the UE.

Non3gppAccess:

anyOf:

- type: string

enum:

- N3IWF

- TNGF

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents a non-3gpp access node.

Possible values are:

- N3IWF: Non-3gpp Interworking Function.

- TNGF: Trusted Non-3gpp Gateway Function.

# * * * * End of change * * * *