3GPP TSG-CT Meeting #101	CP-232255

Bangalore, 11th – 12th September 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.520 | CR | 0751 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Nnwdaf_MLModelTraining Service API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TS 23.288 clauses 6.2C.2, 6.2F.1, 6.2F.2 and 7.10 have been updated based on the stage 2 approved CRs #0773 (S2-2307882) and #0755 (S2-2307884). New parameters are added to the Nnwdaf_MLModelTraining service. This CR proposes to update the Nnwdaf_MLModelTraining Service API to align with the changes with in stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Removed the EN "Whether need the "roundInd" attribute is FFS and will align with stage 2 requirements." in clauses 4.6.2.2.2, 5.5.6.2.2, 5.5.6.2.3, and 5.5.6.2.8. - Added descriptions of "delayEventNotif" and "statusReport" attributes to the descriptions of service operations in clause 4.6.2.4.2. - Removed the EN "The details of available data requirement is FFS and will align with stage 2 requirements." In clause 5.5.6.2.5. - Defined new data types "DataAvReq", "DelayEventNotif", "StatusReportInfo", "TrainDataInfo", and "DelayCause". - Updated the Table 5.5.6.1-1 in clause 5.5.6.1. - Added application error "ML_MODEL_TRAINING_REQS_NOT_MET" to the Table 5.5.7.3-1 in clause 5.5.7.3. - Updated the A.6 for the Nnwdaf_MLModelTraining Service API accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2. No support of new parameters for the Nnwdaf_MLModelTraining service and may cause implementation errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.2.2, 4.6.2.4.2, 5.5.6.1, 5.5.6.2.2, 5.5.6.2.3, 5.5.6.2.5, 5.5.6.2.8, 5.5.6.2.10(new), 5.5.6.2.11(new), 5.5.6.2.12(new), 5.5.6.2.13(new), 5.5.6.3.5(new), 5.5.7.3, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the Nnwdaf_MLModelTraining API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The changes in rev 2 to CT#101 include: - Change “TS29574_CommonData” to “TS29574_Ndccf_DataManagement” for the inpEvents attribute of the DataAvReq data type in A.6 according to the rapporter’s comments. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

##### 4.6.2.2.2	Subscription for event notifications

Figure 4.6.2.2.2-1 shows a scenario where the NF service consumer sends a request to the NWDAF to subscribe for event notification(s) (as shown in 3GPP TS 23.288 [17]).

Figure 4.6.2.2.2-1: NF service consumer subscribes to notifications

The NF service consumer shall invoke the Nnwdaf_MLModelTraining_Subscribe service operation to subscribe to event notification(s). The NF service consumer shall send an HTTP POST request with "{apiRoot}/nnwdaf-mlmodeltraining/<apiVersion>/subscriptions" as Resource URI representing the "NWDAF ML Model Training Subscriptions", as shown in figure 4.6.2.2.2-1, step 1, to create a subscription for an "Individual NWDAF ML Model Training Subscription" according to the information in message body.

The NwdafMLModelTrainSubsc data structure provided in the request body shall include:

-	an URI where to receive the requested notifications as the "notifUri" attribute;

-	a description of the subscribed events as the "mLEventSubscs" attribute that, for each event, the MLEventSubscription data type shall include:

1)	an event identifier as the "mLEvent" attribute;

2)	event filter information as the "mLEventFilter" attribute;

-	the interoperability information as the "modelInterInfo" attribute; and

-	a notification correlation identifier assigned by the NF service consumer for the requested notifications as "notifCorreId" attribute;

and may include:

1)	an identification of UE information for which data for ML model training is requested as the "tgtRepUe" attribute;

2)	the ML model information as the "mLModelInfos" attribute;

3)	the ML model training information as the "mLModelTrainInfos" attribute;

4)	identification of the ML procesure for training the ML model as the "mLCorreId" attribute;

5)	an indication of preparation request for ML model training as the "mLPreFlag" attribute;

6)	an indication of request using the local training data as the testing dataset to calculate the Model Accuracy of the global ML model provided by the consumer as the "mLAccChkFlg" attribute;

7)	the ML model training reporting information as the "mLTrainRepInfo" attribute;

8)	the round number of the training in a multi-round training process as the "roundInd" attribute;

9)	the use case context of the ML model as the "uCaseCont" attribute; and

10)	the reporting requirement information of the subscription as the "eventReq" attribute.

Upon the reception of an HTTP POST request with: "{apiRoot}/nnwdaf-mlmodeltraining/<apiVersion>/subscriptions" as Resource URI and NwdafMLModelTrainSubsc data structure as request body, the NWDAF shall create a new subscription and store the subscription.

If the NWDAF created an "Individual NWDAF ML Model Training Subscription" resource, the NWDAF shall respond with "201 Created" with the message body containing a representation of the created subscription, as shown in figure 4.6.2.2.2-1, step 2. The NWDAF shall include a Location HTTP header field. The Location header field shall contain the URI of the created subscription i.e. "{apiRoot}/nnwdaf-mlmodeltraining/<apiVersion>/subscriptions/{subscriptionId}".

If the immediate reporting indication in the "immRep" attribute within the "eventReq" attribute sets to true during the event subscription, the NWDAF shall include the reports of the subscribed events, if available, as the "mLModelInfos" attribute in the HTTP POST response.

If not all the requested events in the subscription are accepted, then the NWDAF may include the "failEventReports" attribute indicating the event(s) for which the subscription failed and the associated reason(s).

If there is no associated ML model training available for all the listed "mLEvent" attribute, the NWDAF which contains MTLF shall send a "500 Internal Server Error" status code to the NF service consumer. Also, the corresponding failure reason via a "problemDetails" attribute with the "cause" attribute set to "UNAVAILABLE_ML_MODEL_TRAINING_FOR_ALLEVENTS".

If there is no ML model training satisfying the requirements listed in "mLModelTrainInfos" attribute or the ML model cannot be downloaded successfully, the NWDAF which contains MTLF shall send a "403 Forbidden" status code to the NF service consumer, and it may send together with the corresponding failure reason via a "problemDetails" attribute with the "cause" attribute set to "ML_MODEL_TRAINING_REQS_NOT_MET".

If other errors occur when processing the HTTP POST request, the NWDAF shall send an HTTP error response as specified in clause 5.5.7.

# *** 2nd Change ***

##### 4.6.2.4.2	Notification about subscribed event

Figure 4.6.2.4.2-1 shows a scenario where the NWDAF sends a request to the NF Service Consumer to notify for event notifications (see also 3GPP TS 23.288 [17]).

Figure 4.6.2.4.2-1: NWDAF notifies the subscribed event

The NWDAF shall invoke the Nnwdaf_MLModelTraining_Notify service operation to notify the subscribed event. The NWDAF shall send an HTTP POST request with "{notifUri}" received in the Nnwdaf_MLModelTraining_Subscribe service operation as Resource URI, as shown in figure 4.6.2.4.2-1, step 1. The NwdafMLModelTrainNotif data structure provided in the request body that shall include:

-	a notification correlation identifier as "notifCorreId" attribute;

-	at least one of the notification detailed information:

-	description of the notified event as "mLModelInfos" attribute;

-	a delay event notification for training the ML model as "delayEventNotif" attribute when the service is for Federated Learning;

-	an indication that the subscription is requested to be terminated, i.e. no further notifications related to this subscription will be provided, as "termTrainReq";

and may include:

-	an identification of the Machine Learning procedure for training the ML model as "mLCorreId" attribute when the service is for Federated Learning;

-	an identification of the round number of the training in a multi-round training process as "roundInd" attribute;

-	the status report for the ML model training as "statusReport" attribute when the service is for Federated Learning; and/or

-	an indication of the use case context of the ML model as "uCaseCont".

Upon the reception of an HTTP POST request, if the NF service consumer successfully processed and accepted the received HTTP POST request, the NF Service Consumer shall store the notification and respond with HTTP "204 No Content" status code.

If the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [6].

If errors occur when processing the HTTP POST request, the NWDAF shall send an HTTP error response as specified in clause 5.5.7.

# *** 3rd Change ***

##### 5.5.5.2.2	Operation Definition

Callback URI: {notifUri}

The operation shall support the callback URI variables defined in table 5.5.5.2.2-1, the request data structures specified in table 5.5.5.2.2-2 and the response data structure and response codes specified in table 5.5.5.2.2-3.

Table 5.5.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | The Notification Uri as assigned within the Individual NWDAF ML Model Training Subscription and described within the NwdafMLModelTrainSubsc type (see table 5.5.6.2.2-1). |

Table 5.5.5.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(NwdafMLModelTrainNotif) | M | 1..N | Provides Information about observed events. |

Table 5.5.5.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during the event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during the event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] also apply. NOTE 2: Failure cases are described in subclause 5.5.7.3. |  |  |  |  |  |

Table 5.5.5.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

Table 5.5.5.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

# *** 4th Change ***

#### 5.5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.5.6.1-1 specifies the data types defined for the Nnwdaf_MLModelTraining service-based interface protocol.

Table 5.5.6.1-1: Nnwdaf_MLModelTraining specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DataAvReq | 5.5.6.2.10 | Represents the requirement on available data for the ML model training. |  |
| DelayEventNotif | 5.5.6.2.11 | Indicates that the NWDAF containing MTLF is not able to complete the training of ML model within the maximum response time, the cause code, and the expected time complete the training. |  |
| FailureCodeTrain | 5.5.6.3.3 | Identifies the failure reason. |  |
| FailureEventInfoForMLModelTrain | 5.5.6.2.7 | Represents the failure event information for a ML Model Training subscription. |  |
| NwdafMLModelTrainSubsc | 5.5.6.2.2 | Represents a ML Model Training subscription. |  |
| NwdafMLModelTrainSubscPatch | 5.5.6.2.3 | Represents parameters to request the modification of a ML Model Training subscription. |  |
| MLModelInfo | 5.5.6.2.4 | Represents the ML Model information. |  |
| MLModelTrainInfo | 5.5.6.2.5 | Represents the ML Model training information, include requirement on data availability and time availability, training filter information. |  |
| NwdafMLModelTrainNotif | 5.5.6.2.8 | Represents notification of a ML Model Training subscription. |  |
| MLTrainReportInfo | 5.5.6.2.6 | Indicates the training reporting information. |  |
| StatusReportInfo | 5.5.6.2.12 | Indicates status information generated by the NWDAF containing MTLF during ML model training. |  |
| TermMLModelTrainInfo | 5.5.6.2.9 | Indicates that the subscription is requested to be terminated, contain the reasons. |  |
| TermTrainCause | 5.5.6.3.4 | Represents the reasons that the ML Model Training to be terminated. |  |
| TrainDataInfo | 5.5.6.2.13 | Represents the training input data information. |  |

Table 5.5.6.1-2 specifies data types re-used by the Nnwdaf_MLModelTraining service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnwdaf_MLModelTraining service-based interface.

Table 5.5.6.1-2: Nnwdaf_MLModelTraining re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Accuracy | 5.1.6.3.5 | Represents the accuracy level of the ML model. |  |
| DatasetStatisticalProperty | 5.1.6.3.15 | Dataset statistical properties of the data used for ML model training. |  |
| DccfEvent | 3GPP TS 29.574 [26] | Identifies the input data event. |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies the DNN. |  |
| DurationSec | 3GPP TS 29.571 [8] | Represents the duration time in second(s). |  |
| Float | 3GPP TS 29.571 [8] |  |  |
| MLEventSubscription | 5.4.6.2.3 | Represents an Individual NWDAF Event Subscription resource. |  |
| MLEventNotif | 5.4.6.2.6 | Represents notifications about Individual Events. |  |
| NwdafEvent | 5.1.6.3.4 | Describes the NWDAF Events. |  |
| RedirectResponse | 3GPP TS 29.571 [8] |  |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Represents the requirements of reporting the subscription. |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI (Single Network Slice Selection Assistance Information). |  |
| SupportedFeatures | 3GPP TS 29.571 [8] |  |  |
| TargetUeInformation | 5.1.6.2.8 | Identifies the target UE information. |  |
| TimeWindow | 3GPP TS 29.122 [19] |  |  |
| Uri | 3GPP TS 29.571 [8] |  |  |

# *** 5th Change ***

##### 5.5.6.2.2	Type NwdafMLModelTrainSubsc

Table 5.5.6.2.2-1: Definition of type NwdafMLModelTrainSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventReq | ReportingInformation | O | 0..1 | Reporting requirement information of the subscription. If omitted, the default values within the ReportingInformation data type apply. |  |
| failEventReports | array(FailureEventInfoForMLModelTrain) | O | 1..N | Supplied by the NWDAF containing MTLF when available, shall contain the event(s) that the subscription is not successful including the failure reason(s). |  |
| mLCorreId | string | C | 0..1 | Identifies the Machine Learning procedure for training the ML model. It shall be present when the service is for Federated Learning. |  |
| mLEventSubscs | array(MLEventSubscription) | M | 1..N | Each element identifies the subscription for each event. |  |
| mLModelInfos | array(MLModelInfo) | O | 1..N | Each element represents the ML Model information for each event. |  |
| mLModelTrainInfos | array(MLModelTrainInfo) | O | 1..N | Each element represents the ML Model training information for each event, include requirement on data availability and time availability. |  |
| mLPreFlag | boolean | C | 0..1 | Indicates whether the subscription is for preparation of ML Model training. Set to "true" if it is for ML training preparation, otherwise set to "false". Default value is "false" if omitted. It shall be present when the service is for preparation of Federated Learning. |  |
| mLAccChkFlg | boolean | O | 0..1 | Indicates whether request using the local training data as the testing dataset to calculate the Model Accuracy of the global ML model provided by the consumer. Set to "true" if it is requested, otherwise set to "false". Default value is "false" if omitted. |  |
| mLTrainRepInfo | MLTrainReportInfo | O | 0..1 | Indicates the training reporting information. This attribute can be provided when the "notifMethod" attribute within the ReportingInformation structure is set to "ON_EVENT_DETECTION" in the "eventReq" attribute. |  |
| modelInterInfo | string | M | 1 | Represents the ML Model Interoperability Information. This is vendor-specific information and is agreed between vendors, if necessary for sharing purposes. The format of value is out of 3GPP. |  |
| notifCorreId | string | M | 1 | The value of Notification Correlation ID in the corresponding notification. |  |
| notifUri | Uri | M | 1 | URI at which the NF service consumer requests to receive notifications. |  |
| roundInd | Uinteger | O | 0..1 | Indicates the round number of the training in a multi-round training process. |  |
| suppFeats | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 5.5.8. It shall be supplied by NF service consumer in the POST requests that request the creation of an NWDAF ML Model Training Subscriptions resource and shall be supplied by the NWDAF in the reply of corresponding request. |  |
| tgtRepUe | TargetUeInformation | O | 0..1 | Indicates the UE(s) information for which data for ML model training is requested. |  |
| uCaseCont | string | O | 0..1 | Indicates the use case context of the ML model. The value and format of this parameter are not standardized. |  |

# *** 6th Change ***

##### 5.5.6.2.3	Type NwdafMLModelTrainSubscPatch

Table 5.5.6.2.3-1: Definition of type NwdafMLModelTrainSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventReq | ReportingInformation | O | 0..1 | Reporting requirement information of the subscription. If omitted, the default values within the ReportingInformation data type apply. |  |
| failEventReports | array(FailureEventInfoForMLModelTrain) | O | 1..N | Supplied by the NWDAF containing MTLF when available, shall contain the event(s) that the subscription is not successful including the failure reason(s). |  |
| mLModelInfo | MLModelInfo | O | 0..1 | Represents the ML Model information. |  |
| mLModelTrainInfo | MLModelTrainInfo | O | 0..1 | Represents the ML Model training information, include requirement on data availability and time availability, training filter information. |  |
| mLPreFlag | boolean | O | 0..1 | Indicates whether the subscription is for preparation of ML Model training. Set to "true" if it is for ML training preparation, otherwise set to "false". |  |
| mLAccChkFlg | boolean | O | 0..1 | Indicates whether request using the local training data as the testing dataset to calculate the Model Accuracy of the global ML model provided by the consumer. Set to "true" if it is requested, otherwise set to "false". |  |
| mLTrainRepInfo | MLTrainReportInfo | O | 0..1 | Indicates the training reporting information. This attribute can be provided when the "notifMethod" attribute within the ReportingInformation structure is set to "ON_EVENT_DETECTION" in the "eventReq" attribute. |  |
| modelInterInfo | string | O | 1 | Represents the ML Model Interoperability Information. This is vendor-specific information and is agreed between vendors, if necessary for sharing purposes. The format of value is out of 3GPP. |  |
| notifUri | Uri | O | 1 | URI at which the NF service consumer requests to receive notifications. |  |
| roundInd | Uinteger | O | 0..1 | Indicates the round number of the training in a multi-round training process. |  |
| tgtRepUe | TargetUeInformation | O | 0..1 | Indicates the UE(s) information for which data for ML model training is requested. |  |
| uCaseCont | string | O | 0..1 | Indicates the use case context of the ML model. The value and format of this parameter are not standardized. |  |

# *** 7th Change ***

##### 5.5.6.2.5	Type MLModelTrainInfo

Table 5.5.6.2.5-1: Definition of type MLModelTrainInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataAvReq | DataAvReq | C | 0..1 | Represents the requirement on available data for the ML model training. Shall be provided when the "mlPreFlag" attribute in data type "NwdafMLModelTrainSubsc" is set to "true". |  |
| timeAvReq | string | C | 0..1 | Represents the requirement on available time for the ML model training. Shall be provided when the "mlPreFlag" attribute in data type "NwdafMLModelTrainSubsc" is set to "true". |  |

# *** 8th Change ***

##### 5.5.6.2.8	Type NwdafMLModelTrainNotif

Table 5.5.6.2.8-1: Definition of type NwdafMLModelTrainNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| delayEventNotif | DelayEventNotif | C | 0..1 | Indicates that the NWDAF containing MTLF is not able to complete the training of ML model within the maximum response time, the cause code, and the expected time complete the training. May be provided for Federated Learning training. (NOTE) |  |
| mLCorreId | string | C | 0..1 | Identifies the Machine Learning procedure for training the ML model. It shall be present when the service is for Federated Learning. |  |
| mLModelInfos | array(MLModelInfo) | C | 1..N | Each element represents the ML Model information for each event. (NOTE) |  |
| notifCorreId | string | M | 1 | Notification correlation ID used to identify the subscription to which the notification relates. It shall be set to the same value as the "notifCorreId" attribute of NwdafMLModelTrainSubsc data type. |  |
| roundInd | Uinteger | O | 0..1 | Indicates the round number of the training in a multi-round training process. |  |
| statusReport | StatusReportInfo | O | 0..1 | Indicates status information generated by the NWDAF containing MTLF during ML model training. May be provided for Federated Learning training. |  |
| termTrainReq | TermMLModelTrainInfo | C | 0..1 | Indicates that the subscription is requested to be terminated, i.e. no further notifications related to this subscription will be provided. Supplied by the NWDAF containing MTLF when available, shall contain the reasons that the subscription is requested to be terminated. (NOTE) |  |
| uCaseCont | string | O | 0..1 | Indicates the use case context of the ML model. The value and format of this parameter are not standardized. |  |
| NOTE: At least one of the "delayEventNotif", "mLModelInfos" or "termTrainReq" attribute shall be provided. The "delayEventNotif" attribute and "mLModelInfos" attribute are mutually exclusive. The "delayEventNotif" attribute and "termTrainReq" attribute are mutually exclusive. |  |  |  |  |  |

# *** 9th Change (new) ***

##### 5.5.6.2.10	Type DataAvReq

Table 5.5.6.2.10-1: Definition of type DataAvReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataStatProps | array(DatasetStatisticalProperty) | O | 1..N | List of dataset statistical properties of the data to be used for ML model training. |  |
| inpEvents | array(DccfEvent) | M | 1..N | Event ID list of the data to be collected for ML model training. |  |
| minNumSamples | Uinteger | O | 0..1 | Minimum number of samples that be taken to train an ML model. |  |
| timeWindows | array(TimeWindow) | O | 1..N | The time periods of the data samples. |  |

# *** 10th Change (new) ***

##### 5.5.6.2.11	Type DelayEventNotif

Table 5.5.6.2.11-1: Definition of type DelayEventNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| delayEventInd | boolean | M | 1 | Indicates the NWDAF containing MTLF is not able to complete the training of the ML model within the maximum response time provided by the "maxResTime" attribute in the "MLTrainReportInfo" data type. Set to "true" if not able to complete the ML model training on time, otherwise set to "false". |  |
| delayCause | DelayCause | O | 0..1 | Represents the reasons for the ML Model training delay. |  |
| expCompTime | DurationSec | O | 0..1 | Indicates the expected remaining training time. |  |

# *** 11th Change (new) ***

##### 5.5.6.2.12	Type StatusReportInfo

Table 5.5.6.2.12-1: Definition of type StatusReportInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelAcc | Uinteger | O | 0..1 | Indicates the accuracy of the ML model in percent. Minimum = 0. Maximum = 100. |  |
| trainInDataInfo | TrainDataInfo | O | 0..1 | Represents the training input data information. |  |

# *** 12th Change (new) ***

##### 5.5.6.2.13	Type TrainDataInfo

Table 5.5.6.2.13-1: Definition of type TrainDataInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| areaDataSet | string | O | 0..1 | Indicates the areas covered by the data set. |  |
| maxValues | array(string) | O | 1..N | Each element represents the maximum value of one dimension of data. |  |
| minValues | array(string) | O | 1..N | Each element represents the minimum value of one dimension of data. |  |
| samplRatio | Uinteger | O | 0..1 | Sampling ratio, indicates the percentage of the available data that are taken to train an ML model. Minimum = 0. Maximum = 100. |  |

Editior’s Note:	The data type for the "areaDataSet" attribute is FFS and will align with stage 2 requirement.

Editior’s Note:	Whether need the "maxValues" and "minValues" attributes and their data type are FFS and will align with stage 2 requirements.

# *** 13th Change (new) ***

##### 5.5.6.3.5	Enumeration: DelayCause

Table 5.5.6.3.5-1: Enumeration DelayCause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ML_MODEL_TRAIN_FAILURE | Indicates the ML model training is failed. |  |
| NEED_MORE_TIME | Indicates that more time is needed for the ML model training. |  |
| OTHERS | Indicates other cause. |  |

# *** 14th Change ***

#### 5.5.7.3	Application Errors

The application errors defined for the Nnwdaf_MLModelTraining API are listed in table 5.5.7.3-1.

Table 5.5.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| UNAVAILABLE_ML_MODEL_TRAINING_FOR_ALLEVENTS | 500 Internal Server Error | Indicates the requested all events ML model training is unavailable. |
| ML_MODEL_TRAINING_REQS_NOT_MET | 403 Forbidden | Indicates the ML model training requirements are not met. The "ProblemDetails" data structure shall contain the attribute(s) that do not meet the ML model training requirements within the "invalidParams" attribute. |
| NOTE: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |

# *** 15th Change ***

# A.6	Nnwdaf_MLModelTraining API

openapi: 3.0.0

info:

title: Nnwdaf_MLModelTraining

version: 1.0.0-alpha.1

description: |

Nnwdaf_MLModelTraining API Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.520 V18.2.0; 5G System; Network Data Analytics Services.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.520/

servers:

- url: '{apiRoot}/nnwdaf-mlmodeltraining/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnwdaf-mlmodeltraining

paths:

/subscriptions:

post:

summary: Create a new Individual NWDAF ML Model Training Subscription resource.

operationId: CreateNWDAFMLModelTrainingSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NwdafMLModelTrainSubsc'

responses:

'201':

description: Create a new Individual NWDAF ML Model Training Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/NwdafMLModelTrainSubsc'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nnwdaf-mlmodeltraining/v1/subscriptions/{subscriptionId}.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/NwdafMLModelTrainNotif'

minItems: 1

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

summary: update an existing Individual NWDAF ML Model Training Subscription

operationId: UpdateNWDAFMLModelTrainingSubcription

tags:

- Individual NWDAF ML Model Training Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NwdafMLModelTrainSubsc'

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Nnwdaf_MLModelTraining Service.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual NWDAF ML Model Training Subscription resource was modified successfully

and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/NwdafMLModelTrainSubsc'

'204':

description: >

The Individual NWDAF ML Model Training Subscription resource was modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: partial update an existing Individual NWDAF ML Model Training Subscription

operationId: PartialUpdateNWDAFMLModelTrainingSubcription

tags:

- Individual NWDAF ML Model Training Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/NwdafMLModelTrainSubscPatch'

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Nnwdaf_MLModelTraining Service.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual NWDAF ML Model Training Subscription resource was partial modified

successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/NwdafMLModelTrainSubsc'

'204':

description: >

The Individual NWDAF ML Model Training Subscription resource was partial modified

successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual NWDAF ML Model Training Subscription.

operationId: DeleteNWDAFMLModelTrainingSubcription

tags:

- Individual NWDAF ML Model Training Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Nnwdaf_MLModelTraining Service.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual NWDAF ML Model Training Subscription matching the

subscriptionId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnwdaf-mlmodeltraining: Access to the Nnwdaf_MLModelTraining API

schemas:

NwdafMLModelTrainSubsc:

description: Represents a ML Model Training subscription..

type: object

properties:

mLEventSubscs:

type: array

items:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/MLEventSubscription'

minItems: 1

description: Subscribed events

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

failEventReports:

type: array

items:

$ref: '#/components/schemas/FailureEventInfoForMLModelTrain'

minItems: 1

description: >

Supplied by the NWDAF containing MTLF when available, shall contain the event(s) that

the subscription is not successful including the failure reason(s).

mLCorreId:

type: string

description: String identifying the subscription is for a Federated Learning procedure.

mLModelInfos:

type: array

items:

$ref: '#/components/schemas/MLModelInfo'

minItems: 1

description: Represents the ML Model information.

mLModelTrainInfos:

type: array

items:

$ref: '#/components/schemas/MLModelTrainInfo'

minItems: 1

description: Represents the ML Model training information.

mLPreFlag:

type: boolean

description: >

Indicates whether the subscription is for preparation of ML Model training. Set to

"true" if it is for ML training preparation, otherwise set to "false".

mLAccChkFlg:

type: boolean

description: >

Indicates whether request using the local training data as the testing dataset to

calculate the Model Accuracy of the global ML model provided by the consumer. Set to

"true" if it is requested, otherwise set to "false".

mLTrainRepInfo:

$ref: '#/components/schemas/MLTrainReportInfo'

modelInterInfo:

type: string

description: >

String identifying the ML Model Interoperability Information. This is vendor-specific

information and is agreed between vendors, if necessary for sharing purposes.

notifCorreId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding notification.

roundInd:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

tgtRepUe:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/TargetUeInformation'

uCaseCont:

type: string

description: >

String identifying the use case context of the ML model. The value and format of this

parameter are not standardized.

required:

- mLEventSubscs

- modelInterInfo

- notifUri

- notifCorreId

NwdafMLModelTrainSubscPatch:

description: >

Represents parameters to request the modification of a ML Model Training subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

failEventReports:

type: array

items:

$ref: '#/components/schemas/FailureEventInfoForMLModelTrain'

minItems: 1

description: >

Supplied by the NWDAF containing MTLF when available, shall contain the event(s) that

the subscription is not successful including the failure reason(s).

mLModelInfo:

$ref: '#/components/schemas/MLModelInfo'

mLModelTrainInfo:

$ref: '#/components/schemas/MLModelTrainInfo'

mLPreFlag:

type: boolean

description: >

Indicates whether the subscription is for preparation of ML Model training. Set to

"true" if it is for ML training preparation, otherwise set to "false".

mLAccChkFlg:

type: boolean

description: >

Indicates whether request using the local training data as the testing dataset to

Calculate the Model Accuracy of the global ML model provided by the consumer. Set to

"true" if it is requested, otherwise set to "false".

mLTrainRepInfo:

$ref: '#/components/schemas/MLTrainReportInfo'

modelInterInfo:

type: string

description: >

String identifying the ML Model Interoperability Information. This is vendor-

specific information and is agreed between vendors, if necessary for sharing

purposes.

roundInd:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

tgtRepUe:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/TargetUeInformation'

uCaseCont:

type: string

description: >

String identifying the use case context of the ML model. The value and format of this

parameter are not standardized.

NwdafMLModelTrainNotif:

description: Represents notifications on events that occurred.

type: object

properties:

delayEventNotif:

$ref: '#/components/schemas/DelayEventNotif'

mLCorreId:

type: string

description: String identifying the subscription is for a Federated Learning procedure.

mLModelInfos:

type: array

items:

$ref: '#/components/schemas/MLModelInfo'

minItems: 1

description: Represents the ML Model information.

notifCorreId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding notification.

roundInd:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

statusReport:

$ref: '#/components/schemas/StatusReportInfo'

termTrainReq:

$ref: '#/components/schemas/TermMLModelTrainInfo'

uCaseCont:

type: string

description: >

String identifying the context of use of ML model. The value and format of this

parameter are not standardized.

required:

- notifCorreId

oneOf:

- required: [delayEventNotif]

- required: [mLModelInfos]

- required: [termTrainReq]

- required: [mLModelInfos, termTrainReq]

MLModelInfo:

description: Represents the ML Model information.

type: object

properties:

accMLModel:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/Accuracy'

eventNotif:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/MLEventNotif'

modelUniqueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

MLModelTrainInfo:

description: >

Represents the ML Model training information, include requirement on data availability and

time availability, training filter information.

type: object

properties:

dataAvReq:

$ref: '#/components/schemas/DataAvReq'

timeAvReq:

type: string

description: >

String representing the requirement on available time for the ML model training.

MLTrainReportInfo:

description: Represents the ML Model training reporting information.

type: object

properties:

maxResTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

FailureEventInfoForMLModelTrain:

description: Represents the failure event information for a ML Model Training subscription.

type: object

properties:

mLTrainEvent:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

failureCodeTrain:

$ref: '#/components/schemas/FailureCodeTrain'

required:

- mLTrainEvent

- failureCodeTrain

TermMLModelTrainInfo:

description: Indicating that the subscription is requested to be terminated.

type: object

properties:

terminationTrainReq:

type: boolean

description: >

Indication that the subscription is requested to be terminated if set to "true", i.e. no

further notifications related to this subscription will be provided. Otherwise set to

"false". The default value is "true".

termTrainCause:

$ref: '#/components/schemas/TermTrainCause'

required:

- terminationTrainReq

DataAvReq:

description: Represents the requirement on available data for the ML model training.

type: object

properties:

dataStatProps:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/DatasetStatisticalProperty'

minItems: 1

inpEvents:

type: array

items:

$ref: 'TS29574_Ndccf_DataManagement.yaml#/components/schemas/DccfEvent'

minItems: 1

minNumSamples:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeWindows:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

required:

- inpEvents

DelayEventNotif:

description: >

Indicating that the NWDAF containing MTLF is not able to complete the training of ML model

within the maximum response time, the cause code, and the expected time complete the

training.

type: object

properties:

delayEventInd:

type: boolean

description: >

Indicates that the NWDAF containing MTLF is not able to complete the training of ML

model within the maximum response time. Set to "true" if not able to complete the ML

model training on time, otherwise set to "false".

delayCause:

$ref: '#/components/schemas/DelayCause'

expCompTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- delayEventInd

StatusReportInfo:

description: >

Indicating status information generated by the NWDAF containing MTLF during ML model

training.

type: object

properties:

mlModelAcc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

trainInDataInfo:

$ref: '#/components/schemas/TrainDataInfo'

TrainDataInfo:

description: Represents the training input data information.

type: object

properties:

areaDataSet:

type: string

description: Indicates the areas covered by the data set.

maxValues:

type: array

items:

type: string

minItems: 1

description: Represents the maximum value of one dimension of data.

minValues:

type: array

items:

type: string

minItems: 1

description: Represents the minimum value of one dimension of data.

samplRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

#

# ENUMERATIONS DATA TYPES

#

FailureCodeTrain:

anyOf:

- type: string

enum:

- UNAVAILABLE_ML_MODEL_TRAIN

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the failure reason.

Possible values are:

- UNAVAILABLE_ML_MODEL_TRAIN: The ML model training is unavailable.

TermTrainCause:

anyOf:

- type: string

enum:

- NWDAF_OVERLOAD

- NOT_AVAILABLE_ML_TRAIN

- OTHERS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the reasons that ML Model Training to be terminated.

Possible values are:

- NWDAF_OVERLOAD: The NWDAF is overloaded for the ML model training.

- NOT_AVAILABLE_ML_TRAIN: The ML model training process is not available.

- OTHERS: Other cause.

DelayCause:

anyOf:

- type: string

enum:

- ML_MODEL_TRAIN_FAILURE

- NEED_MORE_TIME

- OTHERS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the reasons for ML Model training delay.

Possible values are:

- ML_MODEL_TRAIN_FAILURE: The ML model training is failure.

- NEED_MORE_TIME: The ML model training needs more time.

- OTHERS: Other cause.

*** End of Changes ***