3GPP TSG-CT Meeting #101	CP-232265

# Bangalore, India, 11th - 12th September 2023				was CP-232174 was C3-233651 was C3-233045

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0966 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completing the definition of the GroupParametersProvisioning API |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  | 2023-09-04 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  | The OpenAPI description of the new Nnef_GroupParametersProvisioning API needs to be defined and the corresponding Editor's Note resolved/removed. In addition, the following issues have been identified in the definition of the GroupParametersProvisioning API: - The LADN Service Area definition in stage 2 in clause 5.20b.1 of TS 23.501 indicates that it can take the format of a list of TAI(s). This possibilty should hence be reflected in stage 3. - It should be clarified to which attributes the table NOTE in clause 5.33.5.2.4 should apply. Also, the cardinality should be changed to "1..N". - The cardinality and presence conditions are missing in the definition of the response body of the PATCH and PUT methods based operations. - "v1" needs to be replaced by the "<apiVersion>" placeholder in some clauses to align with the other clauses of this API definition. - The cardinality of the response body of the GET based operation on the collection resource should be changed to "0..N" to allow for the case where no child Group Parameters Provisioning resource is available at the NEF. - The "5QiPriorityLevel" data type needs to be replaced by the "5QiPriorityLevelRm" to allow for the 5QI Priority Level to be removed by the AF. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Define the OpenAPI description of the new Nnef_GroupParametersProvisioning API. - Correct the above listed issues. - Apply additional editorial enhancements/corrections. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - Stage 2 requirements for DNN and S-NSSAI specific Group Parameters provisioning are not fully supported in stage 3 as the OpenAPI description for the new API is missing. - Incorrect definition of the new GroupParametersProvisioning API. |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.37, 4.4.37.1, 4.4.37.2 (new clause), 5.33.2.1, 5.33.2.2.1, 5.33.2.2.2, 5.33.2.2.3.1, 5.33.2.3.1, 5.33.2.3.2, 5.33.2.3.3.2, 5.33.2.3.3.3, 5.33.5.1, 5.33.5.2.4, 5.33.5.2.5, 5.33.5.2.6, A.31 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces a new OpenAPI description for the new Nnef_GroupParametersProvisioning API defined in this specification. |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2 (to CT#101): - Correct the wrong definition/indentation of the “$ref” property in the definition of the "tais" attribute, two spaces need to be added. Rev 3 (to CT#101): - Correct the tdoc number in the coversheet. |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

### 4.4.37	Procedures for Group Parameters Provisioning

* * * * Next changes * * * *

#### 4.4.37.1	General

The procedures described in the clauses below are used by an AF to interact with the 5GC for Group Parameters Provisioning, in order to carry out the following procedures:

-	DNN and S-NSSAI Group parameters provisioning procedures (see clause 4.15.6.3e of 3GPP TS 23.502 [2]).

#### 4.4.37.2	Procedures for DNN and S-NSSAI Group Parameters Provisioning

This procedure is used by an AF to request the creation/update/deletion of a DNN and S-NSSAI Group parameters provisioning.

In order to request the creation of a DNN and S-NSSAI Group Parameters Provisioning:

-	an AF shall trigger the Nnef_GroupParametersProvisioning API by sending an HTTP POST request to the NEF targeting the "Group Parameters Provisionings" collection resource, with the request body including the GrpPpData data structure that shall contain:

-	within the "afId" attribute, the identifier of the AF that is sending the request;

-	within the "dnnSnssaiGrpData" attribute, the DNN and S-NSSAI specific Group parameters data that are to be provisioned; and

-	within the "suppFeat" attribute, the features supported by the AF, if applicable (i.e., feature negociation needs to take place);

-	the NEF shall then check whether the AF is authorized to perform this operation or not;

-	if the AF is authorized:

-	if the LADN Service Area is provided by the AF within the "dnnSnssaiGrpData" attribute (via the "ladnServArea" attribute) and in the form of a list of geographic area(s) or a list of civic address(es), the NEF shall translate this information into a list of TAI(s); and

-	the NEF shall then trigger the Nudm_ParameterProvision service API of the UDM to request the provisioning of the received DNN and S-NSSAI Group parameters provisioning data as specified in 3GPP TS 29.503 [17];

and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with an HTTP "200 OK" status code including a Location header field containing the URI of the created resource, and the response body including a representation of the created "Individual Group Parameters Provisioning" resource within the GrpPpData data structure.

In order to request the update of an existing "Individual Group Parameters Provisioning" resource:

-	an AF shall trigger the Nnef_GroupParametersProvisioning API by sending to the NEF either:

-	an HTTP PUT request targeting the concerned "Individual Group Parameters Provisioning" resource with the request body including the GrpPpData data structure; or

-	an HTTP PATCH request targeting the concerned "Individual Group Parameters Provisioning" resource with the request body including the GrpPpDataPatch data structure;

-	after authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API to request the provisioning of the received updated DNN and S-NSSAI Group parameters provisioning data; and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Group Parameters Provisioning" resource within the GrpPpData data structure; or

-	an HTTP "204 No Content" status code.

In order to request the deletion of an existing "Individual Parameters Provisioning" resource:

-	an AF shall trigger the Nnef_GroupParametersProvisioning API by sending an HTTP DELETE request targeting the concerned "Individual Group Parameters Provisioning" resource to the NEF; and

-	upon success, the NEF shall respond to the AF with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error code from the UDM, the NEF shall take proper error handling actions, as specified in clause 5.33.7, and respond to the AF with an appropriate error status code.

Editor's Note:	Whether the existing parameters provisioning service APIs are impacted by the DNN and S-NSSAI Group parameters provisioning is FFS.

* * * * Next changes * * * *

#### 5.33.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.33.2.1-1 and the resources and HTTP methods used for the GroupParametersProvisioning API.

Figure 5.33.2.1-1: Resource URI structure of the GroupParametersProvisioning API

Table 5.33.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.33.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Group Parameters Provisionings | /pp | GET | Retrieve all the active Group Parameters Provisionings managed by the NEF. |
|  |  | POST | Request the creation of a new Group Parameters Provisioning at the NEF. |
| Individual Group Parameters Provisioning | /pp/{ppId} | GET | Retrieve an existng "Individual Group Parameters Provisioning" managed by the NEF. |
|  |  | PUT | Update an existng "Individual Group Parameters Provisioning" managed by the NEF. |
|  |  | PATCH | Modify an existng "Individual Group Parameters Provisioning" managed by the NEF. |
|  |  | DELETE | Delete an existng "Individual Group Parameters Provisioning" managed by the NEF. |

* * * * Next changes * * * *

##### 5.33.2.2.1	Introduction

This resource represents the collection of Group Parameters Provisionings managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

* * * * Next changes * * * *

##### 5.33.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-grp-pp/<apiVersion>/pp

This resource shall support the resource URI variables defined in table 5.33.2.2.2-1.

Table 5.33.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.30.1. |

* * * * Next changes * * * *

###### 5.33.2.2.3.1	GET

This method enables an AF to request to retrieve all the "Group Parameters Provisionings" resources managed by the NEF.

This method shall support the URI query parameters specified in table 5.33.2.2.3.1-1.

Table 5.33.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.2.3.1-2 and the response data structures and response codes specified in table 5.33.2.2.3.1-3.

Table 5.33.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.33.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(GrpPpData) | M | 0..N | 200 OK | Successful case. All the "Individual Group Parameters Provisioning" resources managed by the NEF are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |  |

Table 5.33.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.33.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

* * * * Next changes * * * *

##### 5.33.2.3.1	Introduction

This resource represents an "Individual Group Parameters Provisioning" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

* * * * Next changes * * * *

##### 5.33.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-grp-pp/<apiVersion>/pp/{ppId}

This resource shall support the resource URI variables defined in table 5.33.2.3.2-1.

Table 5.33.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.33.1. |
| ppId | string | Represents the identifier of the "Individual Group Parameters Provisioning" resource. |

* * * * Next changes * * * *

###### 5.33.2.3.3.2	PUT

This method enables an AF to request the update of an existing "Individual Group Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.33.2.3.3.2-1.

Table 5.33.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.3.3.2-2 and the response data structures and response codes specified in table 5.33.2.3.3.2-3.

Table 5.33.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GrpPpData | M | 1 | Represents the updated "Individual Group Parameters Provisioning" resource representation. |

Table 5.33.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| GrpPpData | M | 1 | 200 OK | Successful response. The "Individual Group Parameters Provisioning" resource is successfully updated and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual Group Parameters Provisioning" resource is successfully updated and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.33.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.33.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

* * * * Next changes * * * *

###### 5.33.2.3.3.3	PATCH

This method enables an AF to request the modification of an existing "Individual Group Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.33.2.3.3.3-1.

Table 5.33.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.3.3.3-2 and the response data structures and response codes specified in table 5.33.2.3.3.3-3.

Table 5.33.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GrpPpDataPatch | M | 1 | Represents the requested modifications to the "Individual Group Parameters Provisioning" resource. |

Table 5.33.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| GrpPpData | M | 1 | 200 OK | Successful response. The "Individual Group Parameters Provisioning" resource is successfully modified and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual Group Parameters Provisioning" resource is successfully modified and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.33.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.33.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

* * * * Next changes * * * *

#### 5.33.5.1	General

This clause specifies the application data model supported by the GroupParametersProvisioning API. Table 5.33.5.1-1 specifies the data types defined for the GroupParametersProvisioning API.

Table 5.33.5.1-1: GroupParametersProvisioning API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AfReqDefaultQoS | 5.33.5.2.5 | Represents the AF requested default QoS. |  |
| DnnSnssaiGrpData | 5.33.5.2.4 | Represents DNN and S-NSSAI specific Group Parameters data. |  |
| GrpPpData | 5.33.5.2.2 | Represents Group Parameters Provisioning data. |  |
| GrpPpDataPatch | 5.33.5.2.3 | Represents the requested modification to an existing Group Parameters Provisioning data. |  |
| LadnServArea | 5.33.5.2.6 | Represents an LADN Service Area. |  |

Table 5.33.5.1-2 specifies data types re-used by the GroupParametersProvisioning API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the GroupParametersProvisioning API.

Table 5.33.5.1-2: GroupParametersProvisioning API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| 5Qi | 3GPP TS 29.571 [8] | Represents a 5G QoS Identifier. |  |
| 5QiPriorityLevelRm | 3GPP TS 29.571 [8] | Represents the 5QI Priority Level. |  |
| Arp | 3GPP TS 29.571 [8] | Represents an ARP. |  |
| BitRate | 3GPP TS 29.571 [8] | Represents a bit rate. |  |
| BitRateRm | 3GPP TS 29.571 [8] | This data type is defined in the same way as the "BitRate" data type, but with the OpenAPI "nullable: true" property. |  |
| CivicAddress | 3GPP TS 29.572 [34] | Represents a civic address. |  |
| ExternalGroupId | 3GPP TS 29.122 [4] | Represents the External Group Identifier for a user group. |  |
| GeographicArea | 3GPP TS 29.572 [34] | Represents a geographical area. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Tai | 3GPP TS 29.571 [8] | Represents a Tracking Area Identifier. |  |

* * * * Next changes * * * *

##### 5.33.5.2.4	Type: DnnSnssaiGrpData

Table 5.33.5.2.4-1: Definition of type DnnSnssaiGrpData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extGroupId | ExternalGroupId | M | 1 | Represents the external group identifier of the targeted group. |  |
| dnn | Dnn | M | 1 | Represents the concerned DNN. |  |
| snssai | Snssai | M | 1 | Represents the concerned S-NSSAI. |  |
| defQos | AfReqDefaultQoS | C | 0..1 | Represents the AF-requested default QoS parameters to be applied to each UE within the targeted group. (NOTE) |  |
| ladnServArea | LadnServArea | C | 0..1 | Represents the AF-requested LADN Service Area to be applied to each UE within the targeted group. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

* * * * Next changes * * * *

##### 5.33.5.2.5	Type: AfReqDefaultQoS

Table 5.33.5.2.5-1: Definition of type AfReqDefaultQoS

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| 5qi | 5Qi | M | 1 | Represents the AF requested default 5G QoS Identifier. |  |
| arp | Arp | M | 1 | Represents the AF requested default Allocation and Retention Priority. |  |
| priorityLevel | 5QiPriorityLevelRm | O | 0..1 | Represents the AF requested 5QI Priority Level. |  |

* * * * Next changes * * * *

##### 5.33.5.2.6	Type: LadnServArea

Table 5.33.5.2.6-1: Definition of type LadnServArea

| Attribute name | Data type | P | Cardinality | Description |  | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| geographicAreas | array(GeographicArea) | C | 1..N | Represents the AF requested list of geographic area(s). (NOTE) |  |
| civicAddresses | array(CivicAddress) | C | 1..N | Represents the AF requested list of civic addresse(s). (NOTE) |  |
| tais | array(Tai) | C | 1..N | Represents the AF requested list of Tracking Area Identifier(s). (NOTE) |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

* * * * Next changes * * * *

# A.31	GroupParametersProvisioning API

openapi: 3.0.0

info:

title: 3gpp-grp-pp

version: 1.0.0-alpha.1

description: |

API for Group Parameters Provisioning.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V18.1.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

servers:

- url: '{apiRoot}/3gpp-grp-pp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials: []

paths:

/pp:

get:

summary: Request to retrieve all the active Group Parameters Provisioning resources at the NEF.

operationId: GetGrpParamsProvisionings

tags:

- Group Parameters Provisionings (Collection)

responses:

'200':

description: >

OK. All the active Group Parameters Provisioning resources managed by the NEF are

returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/GrpPpData'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new Group Parameters Provisioning.

tags:

- Group Parameters Provisioning (Collection)

operationId: CreateGrpParamsProvisioning

requestBody:

description: >

Representation of the new Group Parameters Provisioning to be created at the NEF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/GrpPpData'

responses:

'201':

description: >

Created. Successful creation of a new Individual Group Parameters Provisioning resource.

content:

application/json:

schema:

$ref: '#/components/schemas/GrpPpData'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/3gpp-grp-pp/v1/pp/{ppId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/pp/{ppId}:

parameters:

- name: ppId

in: path

description: >

Represents the identifier of the Individual Group Parameters Provisioning resource.

required: true

schema:

type: string

get:

summary: Request to retrieve an existing Individual Group Parameters Provisioning resource.

operationId: GetIndGrpParamsProvisioning

tags:

- Individual Group Parameters Provisioning (Document)

responses:

'200':

description: >

OK. Successful retrieval of the requested Individual Group Parameters Provisioning.

resource.

content:

application/json:

schema:

$ref: '#/components/schemas/GrpPpData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Group Parameters Provisioning resource.

tags:

- Individual Group Parameters Provisioning (Document)

operationId: UpdateIndGrpParamsProvisioning

requestBody:

description: >

Represents the updated Individual Group Parameters Provisioning resource representation.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/GrpPpData'

responses:

'200':

description: >

OK. The Individual Group Parameters Provisioning resource is successfully updated and a

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/GrpPpData'

'204':

description: >

No Content. The Individual Group Parameters Provisioning resource is successfully

Updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Group Parameters Provisioning resource.

tags:

- Individual Group Parameters Provisioning (Document)

operationId: ModifyIndGrpParamsProvisioning

requestBody:

description: >

Contains the parameters to request the modification of the Individual Group Parameters

Provisioning resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/GrpPpDataPatch'

responses:

'200':

description: >

OK. The Individual Group Parameters Provisioning resource is successfully modified and a

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/GrpPpData'

'204':

description: >

No Content. The Individual Group Parameters Provisioning resource is successfully

Modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Group Parameters Provisioning resource.

tags:

- Individual Group Parameters Provisioning (Document)

operationId: DeleteIndGrpParamsProvisioning

responses:

'204':

description: >

No Content. The Individual Group Parameters Provisioning resource is successfully

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

GrpPpData:

description: Represents the Group Parameters Provisioning data.

type: object

properties:

afId:

type: string

dnnSnssaiGrpData:

$ref: '#/components/schemas/DnnSnssaiGrpData'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

GrpPpDataPatch:

description: >

Represents the requested modification to the existing Group Parameters Provisioning data

instance.

type: object

properties:

dnnSnssaiGrpData:

$ref: '#/components/schemas/DnnSnssaiGrpData'

DnnSnssaiGrpData:

description: Represents DNN and S-NSSAI specific Group Parameters data.

type: object

properties:

extGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

defQos:

$ref: '#/components/schemas/AfReqDefaultQoS'

ladnServArea:

$ref: '#/components/schemas/LadnServArea'

required:

- extGroupId

- dnn

- snssai

anyOf:

- required: [ defQos ]

- required: [ ladnServArea ]

AfReqDefaultQoS:

description: Represents the AF requested default QoS.

type: object

properties:

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

arp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

priorityLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5QiPriorityLevelRm'

required:

- 5qi

- arp

LadnServArea:

description: Represents an LADN Service Area.

type: object

properties:

geographicAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

civicAddresses:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

minItems: 1

tais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

oneOf:

- required: [ geographicAreas ]

- required: [ civicAddresses ]

- required: [ tais ]

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

* * * * End of changes * * * *