3GPP TSG-CT WG4 Meeting #119	C4-235548

# Chicago, US; 13th– 17th November 2023													   was C4-235090

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.536 | CR | 0103 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE already registered indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The presence condition of UE already registered indication is not accurate, and further clarification is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Further clairify the presence condition of UE already registered indication in clause 5.2.2.2.3 and clause 6.1.6.2.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The presence condition of UE already registered indication is not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.3, 6.1.6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR#4649 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not introduce any change to the OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: Typos fixed. Add dependency to stage 2 CR in the cover page. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.2.2.3	NSAC for controlling the number of UEs in hierarchical NSACF architecture

When hierarchical NSACF architecture is deployed in the network, the NF Service Consumer (e.g. AMF, combined SMF+PGW-C) shall invoke the NumOfUEsUpdate service operation to request the NSACF to perform network slice admission control procedure based on UE admission quota or UE admission threshold to control the number of UEs. In this procedure, the NSACF shall delegate the request to the primary NSACF for further processing in specific cases.

Figure 5.2.2.2.3-1: NSAC procedure for controlling the number of UEs in hierarchical NSACF architecture

1.	Same as step 1 of Figure 5.2.2.2.2-1, with the following modifications.

The POST request may contain:

-	the UE already registered indication, if the UE has been registered with the associated S-NSSAI in another NSAC service area before.

NOTE:	During inter-AMF mobility or EPS to 5GS mobility, the target AMF determines the UE already registered indication based on the allowed NSSAI information received from the source AMF or the combined SMF+PGW-C, as specified in clause 5.15.11.1.2 of 3GPP TS 23.501 [2].

2a.	For each S-NSSAI included in UeACRequestData, the NSACF shall perform the actions to control the number of UEs.

-	if the update flag is set to "INCREASE" or "DECREASE", the NSACF shall behave as described in clause 4.2.11.2a of 3GPP TS 23.502 [3].

2b and 2c.	Same as step 2b and 2c of Figure 5.2.2.2.2-1.

3.	If the NSAC processing involves the primary NSACF, the NF Service Consumer (e.g. NSACF) shall send a POST request to the resource representing the network slice admission control related to the number of UEs (i.e. …/slices/ues) in the primary NSACF.

The payload body of the POST request shall contain the input data structure (i.e. UeACRequestData) for network slice admission control, which shall contain the information received in step 1. The UE already registered indication may be included if it is received in step 1 and the UE admission type is quota-based.

4a.	For each S-NSSAI included in UeACRequestData, the primary NSACF shall check the global maximum number of UEs and determines whether to accept or reject the request to update the local maximum number of UEs  and/or the UE admission threshold configurations of NSACF.

-	if the update flag is set to "INCREASE" or "DECREASE", the primary NSACF shall behave as described in clause 4.2.11.2a of 3GPP TS 23.502 [3].If in above primary NSACF handling not all S-NSSAIs are successful, "200 OK" shall be returned, with necessary response data indicating the failed S-NSSAI and the failure reason, e.g. "EXCEED_MAX_UE_NUM".

If in above primay NSACF handling all S-NSSAIs are successful, "204 No Content" shall be returned which could represent the maximum number of UEs for the S-NSSAI not reached.

4b.	On failure, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned.

A ProblemDetails IE shall be included in the payload body of POST response, with the "cause" attribute of ProblemDetails set to application error codes specified in table 6.1.3.2.3.1-1.

4c.	On redirection, "307 Temporary Redirect" or "308 Permanent Redirect" shall be returned. A RedirectResponse IE shall be included in the payload body of POST response.

5a.	If the response message includes the updated local maximum number of UEs and/or the updated UE admission threshold, the NSACF shall replaces the local maximum number of UEs and/or the local UE admission threshold with the received updated values respectively. The NSACF shall execute the same action as step 2a of Figure 5.2.2.2.2-1 based on the updated values;

If the response does not include the updated local maximum number of UEs, the NSACF shall returns the response to NF service consumer based on the received NSAC response from Primary NSACF.

5b and 5c.	Same as step 2b and 2c of Figure 5.2.2.2.2-1.

* * * Next Change * * * *

6.1.6.2.5	Type: AcuOperationItem

Table 6.1.6.2.5-1: Definition of type AcuOperationItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| updateFlag | AcuFlag | M | 1 | Indicates the operation (i.e. increase or decrease) to the impacted S-NSSAI. |  |
| snssai | Snssai | M | 1 | Indicates the S-NSSAI for the increase or decrease operation. It shall contain S-NSSAI in serving PLMN or the mapped S-NSSAI in home PLMN. |  |
| plmnId | PlmnId | C | 0..1 | Indicates the PLMN ID associated to the S-NSSAI for increase or decrease operation. It shall be present in the NSAC procedure for the HR or LBO roaming case, or if the NSACF serves multiple PLMNs. |  |
| ueRegInd | boolean | C | 0..1 | This IE shall be present and set to true if the UE has been registered with the associated S-NSSAI in another NSAC service area before, in hierarchical NSAC architecture. Presence of this IE with false value shall be prohibited. | HNSAC |
| servingPlmnId | PlmnId | C | 0..1 | It shall be present in the NSAC procedure for the HR or LBO roaming case, and if the mapped S-NSSAI in home PLMN is included in snssai attribute. When present, it shall contain the serving PLMN ID of the UE. |  |
| nsacMode | NsacAdmissionMode | C | 0..1 | This IE shall be present for inbound roamer, indicating VPLMN NSAC admission mode or VPLMN with HPLMN assistance NSAC admission mode. |  |

* * * End of Changes * * * *