3GPP TSG-CT WG4 Meeting #119	C4-235497

# Chicago, US; 13th– 17th November 2023										was C4-235298

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.559 | CR | 0024 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Npkmf_Discovery_AnnounceAuthorize service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As defined in TS 33.503, Table 7.2.1-1: 5G ProSe Services provided by 5G PKMF Service; Service Operations; Operation Semantics; Example Consumer(s). Npkmf_Discovery; AnnounceAuthorize; Request/Response; 5G PKMF. MonitorKey; Request/Response; 5G PKMF. DiscoveryKey; Request/Response; 5G PKMF. Npkmf_PKMFKeyRequest; ProseKey; Request/Response; 5G PKMF. Npkmf_ResolveRemoteUserId; Npkmf_ResolveRemoteUserId_Get; Request/Response; SMF, 5G PKMF. There is a new Service of 5G PKMF. It proposes to define the general part of Npkmf_Discovery. |
| Summary of change: |  | It proposes to define the general part of Npkmf_Discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with Stage2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution does not introduce any changes to the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Based on C4-234580, the following update needed: - Several Reference NO. update for 3GPP TS 29.500; - Table 5.1-2: A.3->A.z; - Table 6.y.6.1-1: lower case->upper case for Data types; - Update DiscSecMaterials: removing ProseRestrictedCode(and related reference to 29.555) and adding keys as ProseRestrictedCode is not included in DiscSecMaterials but only the keys are included. - Removing the optional output of "duik" in MonitorKeyRespData and DiscoveryKeyRespData, because Match Report is skipped for U2N. - Replace the AnnounceAuthReqData by AnnounceAuthData and add AnnounceAuthData for 201 Created in the Figure 5.x.2.2.1-1: Announce Authorize. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5	Services offered by the 5G PKMF

5.1	Introduction

The table 5.1-1 shows the PKMF Services and PKMF Service Operations:

Table 5.1-1: List of 5G PKMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Npkmf_PKMFKeyRequest | ProseKey | Request/Response | PKMF |
| Npkmf_ResolveRemoteUserId | Retrieve | Request/Response | SMF, 5G PKMF |
| Npkmf_Discovery | AnnounceAuthorize | Request/Response | 5G PKMF |
|  | MonitorKey | Request/Response | 5G PKMF |
|  | DiscoveryKey | Request/Response | 5G PKMF |

Table 5.1-2 summarizes the corresponding APIs defined for this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Npkmf_PKMFKeyRequest | 6.1 | PKMF Key Request Service | TS29559_Npkmf_PKMFKeyRequest.yaml | npkmf-keyrequest | A.2 |
| Npkmf_ResolveRemoteUserId | 6.2 | PKMF Resolve Remote User ID Service | TS29559_Npkmf_UserId.yaml | npkmf-userid | A.3 |
| Npkmf_Discovery | 6.y | PKMF Discovery Service | TS29559_Npkmf_Discovery.yaml | npkmf-disc | A.z |

* * * Next Change * * * *

5.x	Npkmf_ Discovery Service

5.x.1	Service Description

This service enables an NF (i.e. another 5G PKMF in another PLMN) to request authorization information. The following are the key functionalities of this NF service.

-	Provide the authorization from the 5G PKMF for announcing in the PLMN

-	Provide the discovery key from the 5G PKMF for monitoring in the PLMN

-	Provide the discovery key from the 5G PKMF for a discoverer UE in the PLMN to operate Model B restricted discovery

5.x.2	Service Operations

5.x.2.1	Introduction

The Npkmf_Discovery service supports following service operations:

-	AnnounceAuthorize

-	MonitorKey

-	DiscoveryKey

5.x.2.2	AnnounceAuthorize

5.x.2.2.1	General

The AnnounceAuthorize service operation is invoked by a NF Service Consumer, i.e. another 5G PKMF in another PLMN, towards the 5G PKMF to retrieve the authorization from the 5G PKMF for announcing in the PLMN.

The NF Service Consumer (e.g., 5G PKMF) shall request the 5G PKMF to get authorization as shown in Figure 5.x.2.2.1-1

Figure 5.x.2.2.1-1: Announce Authorize

1.	The NF service consumer (e.g., 5G PKMF) sends a HTTP PUT request to the resource representing the announce-authorize custom operation. The request body shall contain the RSC.

2a.	If the context indicated by the userInfoId doesn't exist, the 5G PKMF shall create the new resource, and upon success of creation of the resource, "201 created" shall be returned.

2b.	If the context indicated by the userInfoId already exists, the 5G PKMF shall replace the stored data using the received data, and upon success of the update of the resource, "204 No Content" shall be returned.

2c.	On failure or redirection, one of the HTTP status code listed in Table 6.y.3.2.3.1-3 may be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.y.3.2.3.1-3.

* * * Next Change * * * *

6.y	Npkmf_Discovery Service API

6.y.1	Introduction

The Npkmf_Discovery shall use the Npkmf_Discovery API.

The API URI of the Npkmf_Discovery API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "npkmf-discovery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.y.3.

6.y.2	Usage of HTTP

6.y.2.1	General

HTTP/2, IETF RFC 9113 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [5].

The OpenAPI [7] specification of HTTP messages and content bodies for the Npkmf_Discovery API is contained in Annex A.

6.y.2.2	HTTP standard headers

6.y.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [5] for the usage of HTTP standard headers.

6.y.2.2.2	Content type

JSON, IETF RFC 8259 [9], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [5]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [10].

6.y.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [5] shall be applicable, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [5] may be supported.

6.y.3	Resources

6.y.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.y.3.1-1 describes the resource URI structure of the Npkmf_Discovery API.

Figure 6.y.3.1-1: Resource URI structure of the Npkmf_Discovery API

Table 6.y.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.y.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AnnounceAuthorize | /{ueId}/announce-authorize/{userInfoId} | PUT | Obtain the authorization from the 5G PKMF for announcing in the PLMN |
| MonitorKey | /{ueId}/monitor-key/{userInfoId} | PUT | Obtain the discovery key from the 5G PKMF for monitoring in the PLMN |
| DiscoveryKey | /{ueId}/discovery-key/{userInfoId} | PUT | Obtain the discovery key from the 5G PKMF for a discoverer UE in the PLMN to operate Model B restricted discovery |

6.y.3.2	Resource: AnnounceAuthorize

6.y.3.2.1	Description

6.y.3.2.2	Resource Definition

Resource URI: {apiRoot}/npkmf-disc/<apiVersion>/{ueId}/announce-authorize/{userInfoId}

This resource shall support the resource URI variables defined in table 6.y.3.2.2-1.

Table 6.y.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.y.1 |
| ueId | VarUeId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [16] |
| userInfoId | UserInfoId | Represents User Info Id. |

6.y.3.2.3	Resource Standard Methods

6.y.3.2.3.1	PUT

This method shall support the URI query parameters specified in table 6.y.3.2.3.1-1.

Table 6.y.3.2.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.y.3.2.3.1-2 and the response data structures and response codes specified in table 6.y.3.2.3.1-3.

Table 6.y.3.2.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnnounceAuthData | M | 1 | Contains the Announce Authorization Data for the indicated UE and indicated user info id. |

Table 6.y.3.2.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnnounceAuthData | M | 1 | 201 Created | Upon success of creation of the resource, a response body shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| n/a |  |  | 204 No Content | Upon success of the update of the resource, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - PROSE_SERVICE_UNAUTHORIZED See table 6.y.7.3-1 for the description of these errors. |
| NOTE 1: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [5]. |  |  |  |  |

Table 6.y.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npkmf-disc/<apiVersion>/{ueId}/announce-authorize/{userInfoId} |

Table 6.y.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same 5G PKMF or 5G PKMF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target 5G PKMF (service) instance ID towards which the request is redirected |

Table 6.y.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same 5G PKMF or 5G PKMF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target 5G PKMF (service) instance ID towards which the request is redirected |

6.y.3.3	Resource: MonitorKey

6.y.3.3.1	Description

This resource represents the Monitor Key.

6.y.3.3.2	Resource Definition

Resource URI: {apiRoot}/npkmf-disc/<apiVersion>/{ueId}/monitor-key/{userInfoId}

This resource shall support the resource URI variables defined in table 6.y.3.3.2-1.

Table 6.y.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| ueId | VarUeId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [16] |
| userInfoId | UserInfoId | Represents User Info Id. |

6.y.3.3.3	Resource Standard Methods

6.y.3.3.3.1	PUT

This method shall support the URI query parameters specified in table 6.y.3.3.3.1-1.

Table 6.y.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.y.3.3.3.1-2 and the response data structures and response codes specified in table 6.y.3.3.3.1-3.

Table 6.y.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitorKeyReqData | M | 1 | Contains the Monitor Key Data for the indicated UE and indicated user info id. |

Table 6.y.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MonitorKeyRespData | M | 1 | 201 Created | Upon success of creation of the resource, a response body containing a representation of the discovery key data to monitor for the UE shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| n/a |  |  | 204 No Content | Upon success of the update of the resource, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - PROSE_SERVICE_UNAUTHORIZED See table 6.y.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - APPLICATION_NOT_FOUND See table 6.y.7.3-1 for the description of these errors. |
| NOTE 1: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [5]. |  |  |  |  |

Table 6.y.3.3.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npkmf-disc/<apiVersion>/{ueId}/monitor-key/{userInfoId} |

Table 6.y.3.3.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same 5G PKMF or 5G PKMF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target 5G PKMF (service) instance ID towards which the request is redirected |

Table 6.y.3.3.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same 5G PKMF or 5G PKMF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target 5G PKMF (service) instance ID towards which the request is redirected |

6.y.3.4	Resource: DiscoveryKey

6.y.3.4.1	Description

This resource represents the Discovery Key.

6.y.3.4.2	Resource Definition

Resource URI: {apiRoot}/npkmf-disc/<apiVersion>/{ueId}/discovery-key/{userInfoId}

This resource shall support the resource URI variables defined in table 6.y.3.4.2-1.

Table 6.y.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| ueId | VarUeId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [16] |
| userInfoId | UserInfoId | Represents User Info Id. |

6.y.3.4.3	Resource Standard Methods

6.y.3.4.3.1	PUT

This method shall support the URI query parameters specified in table 6.y.3.4.3.1-1.

Table 6.y.3.4.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.y.3.4.3.1-2 and the response data structures and response codes specified in table 6.y.3.4.3.1-3.

Table 6.y.3.4.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DiscoveryKeyReqData | M | 1 | Contains the Discovery Key Data for the indicated UE and indicated user info id. |

Table 6.y.3.4.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DiscoveryKeyRespData | M | 1 | 201 Created | Upon success of creation of the resource, a response body containing a representation of the discovery key data for the discoverer UE in the PLMN to operate Model B restricted discovery shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| n/a |  |  | 204 No Content | Upon success of the update of the resource, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - PROSE_SERVICE_UNAUTHORIZED See table 6.y.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - APPLICATION_NOT_FOUND See table 6.y.7.3-1 for the description of these errors. |
| NOTE 1: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [5]. |  |  |  |  |

Table 6.y.3.4.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npkmf-disc/<apiVersion>/{ueId}/discovery-key/{userInfoId} |

Table 6.y.3.4.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same 5G PKMF or 5G PKMF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target 5G PKMF (service) instance ID towards which the request is redirected |

Table 6.y.3.4.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same 5G PKMF or 5G PKMF (service) set. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target 5G PKMF (service) instance ID towards which the request is redirected |

6.y.4	Custom Operations without associated resources

There is no custom operation without associated resources supported in Npkmf_Discovery Service.

6.y.5	Notifications

There is no notification defined for Npkmf_Discovery service.

6.y.6	Data Model

6.y.6.1	General

This clause specifies the application data model supported by the API.

Table 6.y.6.1-1 specifies the data types defined for the Npkmf_Discovery service based interface protocol.

Table 6.y.6.1-1: Npkmf_Discovery specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AnnounceAuthData | 6.y.6.2.2 | Represents Data used to request the authorization to announce for a UE |  |
| MonitorKeyReqData | 6.y.6.2.3 | Represents Data used to request the discovery key data to monitor for a UE |  |
| MonitorKeyRespData | 6.y.6.2.4 | Represents the obtained Monitor discovery key data for a UE |  |
| DiscoveryKeyReqData | 6.y.6.2.5 | Represents Data used to request the discovery key data for a discoverer UE |  |
| DiscoveryKeyRespData | 6.y.6.2.6 | Represents the obtained the discovery key data for a discoverer UE. |  |
| DiscSecMaterials | 6.y.6.2.7 | Represents the discovery security materials |  |
| UeSecurityCapability | 6.y.6.3 | PC5 UE security capability |  |
| ChosenPc5CipheringAlgorithm | 6.y.6.3 | The chosen PC5 ciphering algorithm |  |
| Duik | 6.y.6.3 | Discovery User Integrity Key |  |
| Duck | 6.y.6.3 | Discovery User Confidentility Key |  |
| Dusk | 6.y.6.3 | Discovery User Scrambling Key |  |
| UserInfoId | 6.y.6.3 | User Info ID |  |

Table 6.y.6.1-2 specifies data types re-used by the Npkmf_Discovery service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npkmf_Discovery service based interface.

Table 6.y.6.1-2: Npkmf_Discovery re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| VarUeId | 3GPP TS 29.571 [15] | String represents the SUPI or GPSI. |  |
| RelayServiceCode | 3GPP TS 29.571 [15] | Relay Service Code |  |

6.y.6.2	Structured data types

6.y.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.y.6.2.2	Type: AnnounceAuthData

Table 6.y.6.2.2-1: Definition of type AnnounceAuthData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| relayServCode | RelayServiceCode | M | 1 | This IE shall indicate the Relay Service Code. |  |

6.y.6.2.3	Type: MonitorKeyReqData

Table 6.y.6.2.3-1: Definition of type MonitorKeyReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| relayServCode | RelayServiceCode | M | 1 | This IE shall indicate the Relay Service Code. |  |
| ueSecurityCapability | UeSecurityCapability | M | 1 | This IE shall indicate the PC5 UE security capability |  |

6.y.6.2.4	Type: MonitorKeyRespData

Table 6.y.6.2.4-1: Definition of type MonitorKeyRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| chosenPc5CipheringAlgorithm | ChosenPc5CipheringAlgorithm | M | 1 | This IE shall indicate the chosen PC5 ciphering algorithm |  |
| discSecMaterials | DiscSecMaterials | M | 1 | This IE shall indicate the discovery security materials |  |

6.y.6.2.5	Type: DiscoveryKeyReqData

Table 6.y.6.2.5-1: Definition of type DiscoveryKeyReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| relayServCode | RelayServiceCode | M | 1 | This IE shall indicate the Relay Service Code. |  |
| ueSecurityCapability | UeSecurityCapability | M | 1 | This IE shall indicate the PC5 UE security capability |  |

6.y.6.2.6	Type: DiscoveryKeyRespData

Table 6.y.6.2.6-1: Definition of type DiscoveryKeyRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| chosenPc5CipheringAlgorithm | ChosenPc5CipheringAlgorithm | M | 1 | This IE shall indicate the chosen PC5 ciphering algorithm |  |
| discSecMaterials | DiscSecMaterials | M | 1 | This IE shall indicate the discovery security materials |  |

6.y.6.2.7	Type: DiscSecMaterials

Table 6.y.6.2.7-1: Definition of type DiscSecMaterials

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| duik | Duik | O | 0..1 | Discovery User Integrity Key |  |
| duck | Duck | O | 0..1 | Discovery User Confidentility Key |  |
| dusk | Dusk | O | 0..1 | Discovery User Scrambling Key |  |

6.y.6.3	Simple data types and enumerations

6.y.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.y.6.3.2	Simple data types

The simple data types defined in table 6.y.6.3.2-1 shall be supported.

Table 6.y.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| UserInfoId | string | User Info ID is a string of hexadecimal characters, encoding the value of the user info ID parameter which is a 48-bit long bit string. |  |
| UeSecurityCapability | Bytes | String with format "byte" as defined in OpenAPI Specification [7], i.e. base64-encoded characters, encoding the "UE security capability" IE as specified in clause 11.3.11 of 3GPP TS 24.554 [16] (starting from octet 1). |  |
| ChosenPc5CipheringAlgorithm | integer | This IE shall indicate the chosen PC5 ciphering algorithm as specified in clause 11.4.2.51 of 3GPP TS 24.554 [16] |  |
| Duik | Bytes | String with format "byte" as defined in OpenAPI Specification [7], i.e. base64-encoded characters, encoding the "DUIK" IE as specified in clause 11.6.2.13 of 3GPP TS 24.554 [16]. |  |
| Duck | Bytes | String with format "byte" as defined in OpenAPI Specification [7], i.e. base64-encoded characters, encoding the "DUCK" IE as specified in clause 11.6.2.14 of 3GPP TS 24.554 [16]. |  |
| Dusk | Bytes | String with format "byte" as defined in OpenAPI Specification [7], i.e. base64-encoded characters, encoding the "DUSK" IE as specified in clause 11.6.2.12 of 3GPP TS 24.554 [16]. |  |

6.y.6.4	Data types describing alternative data types or combinations of data types

There is no data type describing alternative data types or combinations of data types in Npkmf_Discovery Service.

6.y.6.5	Binary data

There is no binary data type in Npkmf_Discovery Service.

6.y.7	Error Handling

6.y.7.1	General

For the Npkmf_Discovery API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [6]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [5] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [5].

In addition, the requirements in the following clauses are applicable for the Npkmf_Discovery API.

6.y.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [5].

6.y.7.3	Application Errors

The application errors defined for the Npkmf_Discovery service are listed in Table 6.y.7.3-1.

Table 6.y.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| PROSE_SERVICE_UNAUTHORIZED | 403 Forbidden | It is used when the requested ProSe service is not authorized for this UE Identity. |
| APPLICATION_NOT_FOUND | 404 Not Found | It is used when the requested ProSe Application doesn't exist |

6.y.8	Feature negotiation

The optional features in table 6.y.8-1 are defined for the Npkmf_Discovery API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 6.y.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| N/A |  |  |

6.y.9	Security

As indicated in 3GPP TS 33.501 [11] and 3GPP TS 29.500 [5], the access to the Npkmf_Discovery API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [12]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [13]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Npkmf_Discovery API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [13], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Npkmf_Discovery service.

The Npkmf_Discovery API defines the following scopes "npkmf-keyrequest" for OAuth2 authorization as specified in 3GPP TS 33.501 [8]:

Table 6.y.9-1: OAuth2 scopes defined in Npanf_ProseKey API

| Scope | Description |
| --- | --- |
| "npkmf-disc" | Access to the Npkmf_Discovery API |
| "npkmf-disc:announce-authorize:modify" | Access to modify the authorization to announce for a UE in the PLMN |
| "npkmf-disc:monitor-key:modify" | Access to modify the authorization for monitoring for an UE in the PLMN |
| "npkmf-disc:discovery-authorize:modify" | Access to modify the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery |

6.y.10	HTTP redirection

An HTTP request may be redirected to a different 5G PKMF service instance, within the same 5G PKMF or a different 5G PKMF of an 5G PKMF set, e.g. when an 5G PKMF service instance is part of an 5G PKMF (service) set or when using indirect communications (see 3GPP TS 29.500 [5]).

An SCP that reselects a different 5G PKMF producer instance will return the NF Instance ID of the new 5G PKMF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [5].

If an 5G PKMF within an 5G PKMF set redirects a service request to a different 5G PKMF of the set using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new 5G PKMF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [5].

* * * Next Change * * * *

A.z	Npkmf_Discovery API

openapi: 3.0.0

info:

title: Npkmf_Discovery API

version: '1.0.0'

description: |

Npkmf_Discovery Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.559 V17.5.0; 5G System; 5G ProSe Key Management Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.559/

servers:

- url: '{apiRoot}/npkmf-discovery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- npkmf-discovery

paths:

/{ueId}/announce-authorize/{userInfoId}:

put:

summary: Obtain the authorization from the 5G PKMF for announcing in the PLMN

operationId: ObtainAnnounceAuth

tags:

- Obtain the authorization from the 5G PKMF for announcing in the PLMN

security:

- {}

- oAuth2ClientCredentials:

- npkmf-disc

- oAuth2ClientCredentials:

- npkmf-disc

- npkmf-disc:announce-authorize:modify

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: userInfoId

in: path

description: User Info Id

required: true

schema:

$ref: '#/components/schemas/UserInfoId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AnnounceAuthData'

required: true

responses:

'201':

description: Successful creation of the resource

content:

application/json:

schema:

$ref: '#/components/schemas/AnnounceAuthData'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/npkmf-disc>/<apiVersion>/{ueId}/announce-authorize/{userInfoId}

required: true

schema:

type: string

'204':

description: Successful update of the resource.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/monitor-key/{userInfoId}:

put:

summary: Obtain the discovery key from the 5G PKMF for monitoring in the PLMN

operationId: ObtainMonitorKey

tags:

- Obtain the discovery key from the 5G PKMF for monitoring in the PLMN

security:

- {}

- oAuth2ClientCredentials:

- npkmf-disc

- oAuth2ClientCredentials:

- npkmf-disc

- npkmf-disc:monitor-key:modify

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: userInfoId

in: path

description: User Info Id

required: true

schema:

$ref: '#/components/schemas/UserInfoId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MonitorKeyReqData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/MonitorKeyRespData'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/npkmf-disc>/<apiVersion>/{ueId}/monitor-key/{userInfoId}

required: true

schema:

type: string

'204':

description: Successful update of the resource.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/discovery-key/{userInfoId}:

put:

summary: Obtain the discovery key from the 5G PKMF for a discoverer UE

operationId: ObtainDiscKey

tags:

- Obtain the discovery key for a discoverer UE

security:

- {}

- oAuth2ClientCredentials:

- npkmf-disc

- oAuth2ClientCredentials:

- npkmf-disc

- npkmf-disc:discovery-authorize:modify

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: userInfoId

in: path

description: User Info Id

required: true

schema:

$ref: '#/components/schemas/UserInfoId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveryKeyReqData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveryKeyRespData'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/npkmf-disc>/<apiVersion>/{ueId}/discovery-key/{userInfoId}

required: true

schema:

type: string

'204':

description: Successful update of the resource.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npkmf-disc: Access to the Npkmf_Discovery API

npkmf-disc:announce-authorize:modify: >

Access to modify the authorization to announce for a UE in the PLMN

npkmf-disc:monitor-key:modify: >

Access to modify the authorization for monitoring for an UE in the PLMN

npkmf-disc:discovery-key:modify: >

Access to modify the authorization from the 5G DDNMF for a discoverer UE

in the PLMN to operate Model B restricted discovery

schemas:

# STRUCTURED TYPES:

AnnounceAuthData:

type: object

description: Represents Data used to request the authorization to announce for a UE

required:

- relayServCode

properties:

relayServCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RelayServiceCode'

MonitorKeyReqData:

type: object

description: Data used to request the discovery key to monitor for a UE

required:

- relayServCode

- ueSecurityCapability

properties:

relayServCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RelayServiceCode'

ueSecurityCapability:

$ref: '#/components/schemas/UeSecurityCapability'

MonitorKeyRespData:

type: object

description: Represents the obtained Monitor Discovery Key Data for a UE

required:

- chosenPc5CipheringAlgorithm

- discSecMaterials

properties:

chosenPc5CipheringAlgorithm:

$ref: '#/components/schemas/ChosenPc5CipheringAlgorithm'

discSecMaterials:

$ref: '#/components/schemas/DiscSecMaterials'

DiscoveryKeyReqData:

type: object

description: Data used to request the discovery key to monitor for a discoverer UE

required:

- relayServCode

- ueSecurityCapability

properties:

relayServCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RelayServiceCode'

ueSecurityCapability:

$ref: '#/components/schemas/UeSecurityCapability'

DiscoveryKeyRespData:

type: object

description: Represents the obtained Monitor Discovery Key Data for a discoverer UE

required:

- chosenPc5CipheringAlgorithm

- discSecMaterials

properties:

chosenPc5CipheringAlgorithm:

$ref: '#/components/schemas/ChosenPc5CipheringAlgorithm'

discSecMaterials:

$ref: '#/components/schemas/DiscSecMaterials'

DiscSecMaterials:

type: object

description: Represents the discovery security materials

properties:

duik:

$ref: '#/components/schemas/Duik'

dusk:

$ref: '#/components/schemas/Dusk'

duck:

$ref: '#/components/schemas/Duck'

# SIMPLE TYPES:

UserInfoId:

type: string

UeSecurityCapability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

ChosenPc5CipheringAlgorithm:

description: Contains the chosen PC5 ciphering algorithm.

type: integer

Duik:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

Dusk:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

Duck:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

# ENUMS:

* * * End of Changes * * * *